ACN: 2317417 (51 of 2908)

Time / Day

Date : 202512
Local Time Of Day : 0601-1200

Environment

Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2317417
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Flight from ZZZZ to ZZZ at cruising altitude FO checked the system pages and noticed the green hydraulic fluid was almost at 0 but we still had pressure normal and no ECAM. We called dispatch and maintenance and sent messages advising. Later in the flight we got the ECAM and procedure was followed and then the QRH procedure was completed. We coordinated with dispatch and maintenance the steps to follow. We advise everyone and gave a briefing to the flight attendants. We requested priority handling and continued to our destination. After landing we were informed about hydraulic leaking. We were informed that cleaning was necessary. Maintenance was called the leak was controlled and we were towed into the gate. No further assistance was needed.

Synopsis

Air carrier pilot reported identifying a hydraulic system malfunction during cruise and an ECAM later in the flight. The flight continued to the destination, landed, then was towed to the gate.

ACN: 2317387 (52 of 2908)

Time / Day

Date : 202512
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 35000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Main Gear Door
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2317387

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise at FL350 about 1.5 hrs into the flight, a Flight Attendant called to advise me there was a constant loud noise and vibration felt through the floor at row 13, predominantly on aircraft left. We checked all indications in the flight deck and there was nothing unusual noted. I requested the Flight Attendant record the sound with her phone. She airdropped me the recording, but it wasnâ€™t clear enough to decipher what the problem was. We coordinated for me to leave the flight deck and investigate the problem myself. I walked through the cabin, stopping at row 13 and could definitely hear that the air noise was louder than usual and I could feel an abnormal vibration in through the floor. After I returned to the flight deck, I contacted Maintenance Control through Dispatch via SATCOM. We discussed the issue; concluded there were no other abnormal indications and suspected it might be a loose fairing panel or seal. We concurred that continuing to ZZZZ was safe. I made a logbook writeup, discussed the plan with the FO (First Officer) and the FAâ€™s (flight attendants) and we all felt comfortable continuing. On the ground at ZZZZ we contacted Maintenance to come inspect the plane. Maintenance informed me and showed me in-person that we were missing most of our left gear door and it appeared to have broken off at some point. The aircraft was taken out of service for further inspection.

Synopsis

Air carrier Captain reported abnormal noise/vibration was reported to him during cruise. Flight continued to destination with a discovery of a missing main gear door.

ACN: 2316886 (53 of 2908)

Time / Day

Date : 202512
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 6700

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class D : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2316886
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Aircraft
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While on the visual approach to runway XX and backed up with the ILS into ZZZ, autopilot/autothrust off, the PF requested gear down prior to the ZZZZZ FAF at 6700ft. PM moved the gear handle down and the usual three red transit lights came on. However by the time the gear should have been extended we received the Left Gear not downlocked ECAM. The ECAM directed us to recycle the gear which the PM did while the PF continued the descent on GS at flaps 3. Once the recycling did not yield a gear down locked indication either we conducted a go around. Obviously with no attempt to raise the gear. Tower instructions were to climb to 9000ft and a heading of 150deg. PF requested 200kts followed by 210kts to stay above green dot with flaps retracted. After go around we entered cruise for the airport vicinity and the PF engaged the autopilot to free up capacity. The PM coordinated with ATC about requesting priority handling, type of malfunction, intentions for new approach, delay vectors and SOB/ FOB questions later with tower. PM advised flight attendants and passengers of the situation. The crew determined the safer option to land in ZZZ over returning to ZZZ1 since fuel consumption might be reduced with landing gear doors open and left gear possibly exposed. Also the weather in ZZZ1 was moderate turbulence below FL180. The ILS XX was loaded in the FMGC, and we received vectors to the visual final again. Prior to the ZZZZZ FAF we used the manual gear extension as directed by the ECAM. The gear extended with the three green indication and no further ECAM to the contrary. All checklists were completed and crew discussed touchdown and rollout. Since we knew that the procedure inhibited nose wheel steering after landing as per ECAM the PM had already advised ATC of possibly needing to be towed from the runway. Emergency vehicles were standing by. PF was to decelerate to a safe taxi speed on the runway before transferring controls to the Captain. After an uneventful landing and roll out, the Captain took flight controls and managed to taxi the airplane off the runway onto the parallel Taxiway 1 heading towards the ramp. By that time, operations in ZZZ acknowledged our request for a tow in from Taxiway 1 short of Taxiway 2. After completing all necessary flows and tow in checklists we started the APU and shut down all engines. No further issues occurred while getting towed to Gate XX with regular egress for passengers. Crew finished all applicable checklists and concluded the termination checklist after talking to operations.

Synopsis

Air carrier First Officer reported a go around due to unsafe gear indication followed by a return to the airport for a safe landing.

ACN: 2313946 (54 of 2908)

Time / Day

Date : 202512
Local Time Of Day : 1201-1800

Place

Altitude.MSL.Single Value : 30000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2313946
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Tear in #2 Engine attrition liner noted on walkaround. Discrepancy written in logbook, signed off by Maintenance Control. In flight approaching cruise at FL300, intermittent ECAM â€œENG (Engine) 2 HIGH VIBRATION. HI ENG VIB PROC..APPLY.â€ VIB N1 continuously fluctuated between 3.3 and 9.9. VIB N2 continuously fluctuated between 1.7 and 5.4. Other engine parameters were normal. Called Mx (Maintenance) Control via Dispatch on SATCOM to discuss. We were told N1 and N2 VIB sensors separate, but go through same computer. Pilots were not comfortable going over Airspace with this maintenance issue, and the torn liner was a concern of ours due to possible further tear and ingestion in flight. Coordinated with Mx Control and Dispatch to divert to ZZZ, where there was the least likely chance of landing overweight, and a spare aircraft was located. Pax (Passengers) notified. Diverted to ZZZ, discrepancy documented in logbook, and swapped aircraft as planned.

Synopsis

Air carrier pilot reported high engine vibration indication during cruise. Flight diverted and landed safely.

ACN: 2312887 (55 of 2908)

Time / Day

Date : 202512
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2312887
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Normal push off gate and taxi-out. Completed all checks up to "Before Take-off Below the Line". Issued "Line-up and Wait - Runway XX" by ATC. As we were turning onto the runway, aircraft Nose-Wheel Steering (NWS) failed and associated ECAM. Was able to complete turn utilizing differential braking and differential thrust. Notified ATC of issue and need to clear runway to troubleshoot. ATC directed to proceed down Runway XX, exit taxiway 1, hold-short Runway XYR and contact ground. We complied with instructions. Once parking brake set, completed non-normal procedure. Notified ZZZ operations and requested a tow. Then contacted Dispatch & Tech Ops via SATCOM. Tech ops directed resetting circuit breakers, which reset the NWS to operating status. Airport Ops arrived at this time with the tow-vehicle. We thanked everyone for coming out and notified them we were fixed and were going to depart. Airport Operations & tug departed. Passengers informed. We departed ZZZ with not further incident.

Synopsis

A320 pilot reported a nose wheel steering malfunction while turning onto their cleared runway resulting in a momentary loss of aircraft control. The crew reset the circuit breakers per maintenance instruction which fixed the issue and were able to continue their flight.

ACN: 2309302 (56 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2309302

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2309335

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During initial takeoff roll from runway XX at approximately 75 knots, the aircraft sounded a CAUTION aural alert and displayed the ECAM "ENG 1 COMPRESSOR VANE". The FO was the flying pilot and I was pilot monitoring, with my hands on the thrust levers per the FM. I announced "rejecting, I have the aircraft" and initiated the low speed RTO (Rejected Takeoff) procedure. The FO verified the auto brakes MAX had engaged, the spoilers were deployed, and that the Reversers were extended, then told the tower we were aborting our takeoff, and then finally issued a PA to the passengers to remain seated. I directed the FO to disarm the auto brakes and used manual braking to slow the aircraft. After determining there was no fire or control issue, once at a slow speed I elected to taxi off the runway. Once safely stopped on a taxiway, the FO and I ran the RTO QRC from the card. I spoke with the lead FA and determined there was no need to conduct an evacuation, so we chose to continue dealing with the non-normal event. I directed the FO to run the ECAM. The ECAM instruction directed us to shut down the #1 engine, which we did without any issues. Once the ECAM was resolved and the screens returned to normal, we looked up the non-normal procedures for both the RTO as well as the Engine Compressor Vane failure in the FM for any further steps, and to review notes, warnings, and cautions. Once all FM procedures were complied with, we called station ops and MX (Maintenance), to determine the best way forward. The loss of the #1 engine meant we did not have normal system pressure on the Green hydraulic system. While we could have taxied using pressure from the PTU (Power Transfer Unit), I determined the safest course of action was to shut down and allow ground operations to tow us to the gate. We coordinated with local OPS and MX for a tow team. Following the FM "Tow In" procedures, we were towed back to the gate. At the gate we met MX, debriefed, and turned the plane over to them. The passengers were turned over to the gate agent for deplaning and rebooking.

Narrative: 2

During initial take off roll from runway XX at approximately 65 knots a ECAM (ENG 1 COMPRESSOR VANE) came up. We initiated the low speed RTO (Rejected Takeoff) and taxied off the runway, after telling tower of the rejection. We were instructed to taxi on 1 taxiway short of 2. We ran the QRC, the ECAM, then ran the non-normal check list in the FM which guided us to turn the number one engine off. With the number one engine off we had no nose wheel steering. We coordinated with local OPS and MX (Maintenance) for a tow team. We were towed back to the terminal where we meet MX, debriefed them and turn the plane over to them.

Synopsis

A320 flight crew reported a rejected takeoff due to ECAM indication. Flight returned to gate.

ACN: 2309262 (57 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.AGL.Single Value : 37000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : Balloon
Crew Size.Number Of Crew : 0
Operating Under FAR Part.Other 
Flight Plan : None
Mission.Other 
Airspace.Class A : ZZZ

Person : 1

Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 8
ASRS Report Number.Accession Number : 2309262

Person : 2

Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 24
ASRS Report Number.Accession Number : 2309266
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Person : Flight Crew
Miss Distance.Horizontal : 100
Miss Distance.Vertical : 100
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While attending to another sector in the area, the controller on sector RXX called for me and told me that an aircraft almost hit a weather balloon. After getting more details, we determined that the plane estimated to have missed the white weather balloon by approximately 50-100 feet, while flying at an altitude of FL370. No evasive actions were taken by the pilot. The area and altitude where it was reported, is an area of high traffic throughout the day. No weather balloon information had been provided, and after checking emails and one of the weather balloon websites, we had not seen any information that would've let us know of this balloon. Recommendation: Due to recent events with weather balloons hitting airplanes and the drastic increase in weather balloons operating, better procedures need to be in place. To my knowledge, no further action has been taken on this particular issue besides the controller advising planes of the reported balloon. Recommendations: Transponders on balloons, a liaison on site to determine where and when balloons will enter our airspace, balloons that are more visible (not white)

Narrative: 2

While working two sectors combined, Aircraft X reported almost hitting a balloon at FL370. Pilot reported the balloon came within 50 to 100 feet of the aircraft. Aircraft was traveling at approximately 500 knots across the ground. He stated it had strings attached to it and appeared to have some kind of payload under it. He stated they barely missed it. I did not have a primary target or anything with a transponder in the area that would have identified a balloon in the area. Many pilots on the frequency became very concerned this was in the area and clogged the frequency asking questions. I did not have any other aircraft report seeing this balloon during my session. I reported this to my supervisor who brought it up to the watch manager. The watch manager at the desk, according to the supervisor, didn't appear to show any concern over it and made a statement about they can be out there. This has been the constant response to these reports and shows a great lack of concern over the safety of aircraft in our airspace in my opinion. Recommendation: Management at the higher level in the building continues to brush off these incidents. After the Local Safety Council briefed an accident over Location X with an aircraft at FL350, I hoped the attitude about this would improve. It remains the same. I fear a serious accident will have to happen before people show attention this deserves. The Local Safety Council has been working on a joint response to this which is already being met with resistance from the operations managers because it would add work to their plate. In an agency that claims to represent safety, I can't believe this is the response to a serious safety concern.

Synopsis

A Center Supervisor and Controller reported an A320 at FL370 experienced an NMAC with a weather balloon.

ACN: 2307402 (58 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic Auxiliary/Standby System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2307402
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Object
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Day 0 we were operating flight from ZZZ-ZZZ1. We just arrived on ZZZ2-ZZZ flight to turn the same tail number (Aircraft X) to ZZZ1. We noticed no malfunctions, abnormalities or ECAM messages during flight ZZZ2-ZZZ. During cockpit preparation I conducted a break check per SOP and it checked normal. After boarding, we pushed back normally to spot XX per ramp control instruction. We were utilizing â€œsmart startâ€ and did not start an engine on push back. Once the push back was complete, ground ops requested that we set the breaks. I switched the parking break to on, noticed no abnormalities or ECAMs and then requested the tug to be disconnected. Seconds later, ground ops came back on the mic and asked us if the breaks were set. We replied yes and they said the aircraft rolled slightly. We did not have any ECAM faults or failures. We asked if the parking break light was on and he replied it was on. We recycled the breaks and noticed that our triple indicator was showing all zeros on the break pressure and accumulator pressure. We requested that ground ops immediately chock the aircraft and check for any fluid leaking on the ground around the wheels. (there were no leaks). We then coordinated with ramp control of the issue and that we needed to hold location. With the APU on, Engines off and Aircraft chocked I called maintenance control for guidance. After explaining the situation and events he asked us to verify N/W (Nosewheel) Steering Disc was shown on the ECAM and asked if we had any ECAM messages. We replied no ECAM messages and he requested that we push the yellow electric pump to pressurize the system. System pressure returned and maintenance control advised us that we did not need to come back to the gate and we could continue the flight. Now with the parking break on and showing pressure we requested chalk removal and we started engines. We taxied out, tested the breaks several times and had no other breaking issues during the flight and initial taxi in. When we arrived at the gate in ZZZ1 we set the parking break, shut the engines down and completed the parking checklist. With the aircraft chalked at the gate we called maintenance and recycled the parking break to test the yellow system again. Once again we lost all accumulator pressure. A logbook entry was made for the yellow accumulatorâ€™s loss of pressure. *This report is being filed now instead of immediately after the event because new information of the aircraft bumping into the tow bar after the break was set has been recently relayed to us. At the time of the event ground ops only notified us that the â€œaircraft rolled a little.â€ It was so slight that the First Officer and I didnâ€™t feel or notice anything. When we reset the break in ZZZ1 after blocking in, maintenance told us that he believes that it was low pressure or a slow leak in the accumulator. He believed this is why we never got an ECAM fault or failure.

Synopsis

Air carrier Captain reported slight un-commanded aircraft movement due to low pressure in brake accumulator after push back was completed. Issue was resolved and flight continued.

ACN: 2306526 (59 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 1201-1800

Place

Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 2306526
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Other

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Jet Blast
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

why would the pushback driver given clearance to push if another plane that was pushed from [Gate] XX be in our path? I filed a report on this same type of situation and nobody did [EXPLETIVE] like normal cause safety is not a thing at the airport. me and other wingwalker stopped out plane cause [Gate] XX plane was in our path plus when that plane turned their jet blast would have injured us. Cause: the tower and the company do not care about safety when it comes to pushing planes. this happens a lot yet very little people do anything cause they know the behavior of the tower and the company do not care about safety and it shows every day.

Synopsis

Ground personnel reported receiving push back clearance in conflict with another aircraft in their path. Push back was stopped.

ACN: 2306467 (60 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Aerofoil Ice System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2306467
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Aircraft flew through moderate time icing between 15,000 and 13,000 with substantial accumulation observed. Wing anti-icing system failed with associated ECAM. We requested a decent to lowest altitude and were assigned 8000. Through 9500 captain noticed speed was still 280 knots. We momentarily slowed our descent and reduced to 250 knots.

Synopsis

Air carrier First Officer reported a failure of the wing anti-icing system during descent. Flight crew requested lower altitude and continued to the destination.

ACN: 2306393 (61 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2306393

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During 2 engine taxi out and APU off, we experienced a total electrical failure. Even lost the systems, all screens, FCU (Flight Control Unit) data, ACP (Audio Control Panel) panels and radio 1. It looked just like you find a plane at the gate, completely powered down with no ground power and batteries OFF. I immediately stopped the aircraft with no issues. About 5 seconds later, IDG (Integrated drive generator) 2 came back online and aircraft started to power up. At this point I noticed that the systems was staring up just as if the aircraft was initialized from a cold airplane with all the manufacturer labels and version data on the screen and then started the countdown for initialization. The only two ECAMS we had was both FWC (Flight Warning Computer) fault and GEN 1 FAULT. I turned the nose wheel steering and I could tell that NWS (Nose Wheel Steering) was working with no issues and no ECAMS related to it. I had the FO start the APU. All the numbers on the ELEC (Electrical) page were indicating. Once we were able to get some electrical power on the aircraft we were able to make appropriate calls to return to the gate. I notified FAs and made PA to passengers. Made Logbook entry. Once at the gate, had a conversation with mechanics. Called dispatcher and we conferenced with Maintenance Control to try to get some feedback since the ZZZ mechanics were puzzled about what had happened. I did get the same feedback from Maintenance Control. No one could understand how we ended up with a total electrical power failure. Aircraft was eventually swapped from Aircraft X to Aircraft Y. Report filed. Cause: NA Suggestions: The report comes up with Aircraft Y. As well as the filed report. However, the event happened on Aircraft X (A320) and we swapped into Aircraft Y (A319).

Synopsis

A320 Captain reported experiencing a total electrical power loss during taxi. The flight crew returned to the gate after the power was restored.

ACN: 2305858 (62 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 0
Experience.Flight Crew.Last 90 Days : 0
Experience.Flight Crew.Type : 0
ASRS Report Number.Accession Number : 2305858
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Ramp
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were pushed back off of Gate XX and tug disconnected us at Spot 1. I flashed our taxi light to the ground crew and received our salute from them. We configured for single engine taxi and ran the after start start checklist. FO (First Officer) called ramp for taxi and we were told to exit through Spot XX. I turned on the taxi light after clearance was received, visually cleared left and ahead while verbalizing it and also scanning to the right. Saw 2 construction pickup trucks stopped at the edge of the vehicle road waiting to make a right turn which would cross directly in front of us. FO visually and verbally cleared right and ahead. I advanced the left throttle to start taxi and FO said to wait, the trucks were now moving. I immediately brought the throttle to idle and applied brakes as I saw the 2 construction pickup trucks pass in front of us from right to left on the service road, just visible over the nose of the aircraft. I flashed the taxi light multiple times at them as they drove by.

Synopsis

A320 Captain reported a traffic conflict with ground vehicles not yielding to the aircraft when beginning to taxi on the ramp. The First Officer called out the moving trucks to the Captain, who applied the brakes as the vehicles passed in front of the aircraft.

ACN: 2304877 (63 of 2908)

Time / Day

Date : 202510

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2304877
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After the pilot monitoring set FLC/MCT (Maximum Continuous Thrust), I noticed that the airspeed indicator was not responding and called out that we had no airspeed indication on either PFD (Primary Flight Display). The Captain promptly rejected the take off and we taxied to the gate without further issue. The rejected takeoff speed was less than 50 kts according to the indicated ground speed on the ND (Navigation Display). Additionally, the auto brake did not disengage when the Captain pressed on the brake pedals, and had do be turned off manually by pressing the A/Brake button. The airplane was tail swap we were flying to ZZZ1, and had a previous write up regarding the same issue but only on the Captainâ€™s PFD.

Synopsis

A320 First Officer reported an inoperative airspeed indicator during the initial takeoff roll. The Captain rejected the takeoff.

ACN: 2304833 (64 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Gear Pins
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2304833

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2304834

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

I completed the preflight walk around prior to departure and did not observe the nose gear pin flag hanging below the nose gear doors. On takeoff, when the Captain called for gear up, I moved the gear lever to the up position and observed that the nose gear was showing in transit. A Caution ECAM then alerted us to the L/G Nosegear Not Unlocked. We attempted to cycle the gear and got the same response. We then left the gear down and requested priority handling. We received vectors around while the Captain worked through his steps, while I flew. We landed underweight and safely back in ZZZ. Once at the gate, the mechanics found that the nose gear pin was left in from an earlier tire and brake change and the flag was only about 8 inches long and was not visible below the gear doors. Cause: Longer and Brighter Gear Pin Flag and maintenance verification of all pin removals.

Narrative: 2

Shortly after selecting the gear up we were presented with the red indication on the nose gear light and the ECAM nose gear not uplocked. We then lowered the landing gear successfully. We stayed at 200 kts requested priority handling leveled off at 5000 feet. Ensured the priority handling was received and started preparing for returning to ZZZ. I assigned the FO (first Officer) pilot flying duties. I contacted Dispatch, Maintenance Control, and updated the flight attendants. I made a brief PA. We ran the non routine landing checklist and over weight landing checklist. After we finished the set up and review and the approach briefing we ran the descent checklist requested vectors for the ILS Runway XXR and returned to a successful landing that was also under max landing weight. Cause: The post flight inspection revealed the nose gear pin was in place. The flag on the pin was approximately 8 inches in length and did not hang down past the gear doors. The Mechanic and I agreed a full length flag would have prevented this from occurring. He then placed a full length flag on the pin.

Synopsis

A320 flight crew reported nose gear did not retract on take off due to the nose gear pin being left in.

ACN: 2304052 (65 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Fuel Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2304052

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were dealing with an APU not doing a great job of cooling the aircraft while on the ground in ZZZ. The cockpit temperature never got below 90 and the cabin remained at 84/82. Knowing we had a 20 minute flight back to ZZZ1, we elected to continue. Performance mandated we take off in the opposite direction in ZZZ using runway XX. The push was delayed 10 minutes for ground traffic out to runway XX. Startup and taxi went normally but the interior started heating up after engine start. Temps hovered at 98 in the cockpit and 88/84 in the cabin. We were hoping to hustle because once airborne the plane would start cooling off. Ground told us takeoff would be delayed at least 20 minutes due to arriving and departing traffic. After a 10 minute sit waiting for takeoff, we got an Master Caution Light and ECAM message stating a fuel overheat condition in the left wingtip fuel tank. The temperature was 57 C and in less than 20 seconds it climbed to and hit 72 C. We accomplished the ECAM actions shutting down the left generator and left engine. We were now obligated to return to the gate. We asked for a taxi back but ground stated we could not taxi back for at least 20 minutes. In the meantime the right wingtip fuel was overheating hitting about 58 C when we made the decision to advise ATC to gain traffic priority. We got an immediate back taxi down the runway and returned to our pad location. We accomplished the remaining ECAM actions once parked, shutting down the right generator and right engine. The temp reached 68C in the right tank at shutdown. Cabin air temps remained at the previous highs of 98/88/84 during the whole incident. I exited the cockpit and asked the passengers to quickly but orderly depart the aircraft down the front and rear airstairs. We were concerned about how high the temperatures actually went in the fuel tanks. Ground fire fighting crews rolled and monitored the fuel tank temperatures. The passengers were calm and the exit was swift and orderly. I asked my inflight crew to go with the passengers and had my first officer manage the arriving staff while I cleaned up the remaining checklist items and shut down the aircraft.

Synopsis

A320 Captain reported returning to the gate after noting high fuel temperatures during taxi.

ACN: 2303795 (66 of 2908)

Time / Day

Date : 202511

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2303795

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The flight was delayed due to maintenance swapping the number 1 and 3 ADIRUs and deferring the number 3. All indications were normal as we taxied to and lined up on runway XX. I set the thrust in the flex temp and cross checked the instruments, indications were normal. As the aircraft built speed going down the runway, I could tell that my airspeed indications were slow to come up. At the 80 kt check, my airspeed had just started to register. At about the same time, I got red flashing FD (Flight Director) and multiple ECAM cautions. I called the abort and brought the thrust levers to max reverse. I tried to disconnect the auto brakes and take over manually to make the abort less violent, but they wouldnâ€™t disconnect. We came to a stop on the runway with the auto brakes still in max. I told my FO to disconnect the brakes, he cycled the anti-skid switch and the brakes disconnected. I gave the remain in your seats PA to the flight attendants and passengers. We exited the runway and taxied back to the gate. On our first leg, ZZZ1 to ZZZ, we got an unreliable airspeed on takeoff. We followed the procedure and determined that the airspeed on the captains PFD (Primary Flight Display) was unreliable. We switched Air Data to Capt 3 on the Switching panel and that restored my airspeed. I ACARSed dispatch and let them know of the problem and that we would need maintenance in ZZZ. We continued to ZZZ without further incident. That was the reason for the maintenance in ZZZ, we canâ€™t defer ADIRU 1. Whatever the mechanics did in ZZZ, made the problem worse.

Synopsis

A320 flight crew reported unreliable airspeed indicator during takeoff roll. Flight crew rejected takeoff.

ACN: 2302908 (67 of 2908)

Time / Day

Date : 202511

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2302908
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Work Refused
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When reviewing the logbook it was discovered that the previous crew had done an air return based on unreliable speed. A special flight permit was then issued based on ADR1 being inoperative. The logbook stated that an inspection had been done by Maintenance of the TAT1 (Total Air Temperature) prob, no defects were noted, and the aircraft was signed off to ferry the aircraft and aircraft was safe for flight. Under the other restrictions section of the special flight permit stated, follow Manual pro/abnormal and emergency procedures for ADR1 fault. Based on the logbook entry of the aircraft safe for flight, I took this statement as follow the Manual procedure if ADR1 fault to occur. Based on previous crews write-up, Captain briefed me to pay close attention to airspeed disagreement on takeoff and we both came to an agreement that we will abort if a speed discrepancy is noted. On takeoff role, me the FO, was the pilot PM, said 80 kts and Captain noted the airspeed was only at 50 kts so he performed the abort. We exited the runway and called dispatch to coordinate a gate return. While waiting for ground personnel to get in place, Captain spoke with Maintenance control about the abort and he stated we needed to preform the Manual abnormal procedure and thatâ€™s what the special flight permit is for. Soon after, dispatch sent us an ACARS telling the ramp crew was ready to bring us into the gate. After arriving at the gate and talk with the captain we agreed to call Maintenance Control and verify exactly what we were to do. He confirmed to follow abnormal ADR1 procedures which after doing so, showed no airspeed, altitude, or vs on the captain side. The Captain and I both agreed that this is unsafe and therefore refused the aircraft in the interest of safety. The ferry permit guidance in the Operations Manual could be more detailed on procedures to follow. Also, this specific permit should have been more informative on what to expect and do. I feel I could have also called maintenance before departure for clarification on what is expected which would have prevented us departing in the first place. I feel this ferry permit was unsafe and should not have been issued. Ferry Permit issuance and listed procedure created an aircraft to be unsafe for departure.

Synopsis

With an inoperative air speed indicator, the FO reported the ferry permit and aircraft were refused as unsafe by the Captain.

ACN: 2301768 (68 of 2908)

Time / Day

Date : 202511
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2301768

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approaching 100 kts (approximately 95 kts), we received a Master Warning and Engine 2 Fire indication. As the reject was initiated, the fire indication disappeared. After completing the Reject maneuver, we referenced the QRC and confirmed with Tower and FAs that they saw no indications of a fire. We then reviewed the Rejected Takeoff checklist in the Flight Manual, exited the runway, and had ARFF (Airport Rescue and Firefighting) look over the aircraft. After ARFF indicted that everything looked normal, we shutdown Engine 2 to taxi back using normal single-engine procedures. After parking and confirming that the aircraft was chocked, the brakes were left off in accordance with flight manual guidance. Throughout the situation, passengers were kept informed and there was no indication of any injuries. After passengers deplaned, the Captain communicated with Dispatch/Maintenance Control/Chief Pilot and the crew debriefed the event. The crew subsequently operated the scheduled flight with a different aircraft.

Synopsis

A320 Captain reported rejecting a takeoff at 95 knots due to an engine fire warning and indication on the right engine. Crew returned to gate after CFR evaluated the aircraft.

ACN: 2300745 (69 of 2908)

Time / Day

Date : 202510

Place

Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2300745
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Being vectored for visual approach. ILS out of service. Used RNAV RNP as a reference (did not fly the approach- just used vertical guidance for reference on the descent to runway). Unfortunately we did not go direct to an approach transition and arm the approach prior to descent. After being cleared for a visual approach we reverted to flawed habit patterns and set the FAF altitude of 2000' in MCP and initiated descent. Big threat was the FAF crossing altitude was 1000' not the normal 1500'. We failed to brief and then mitigate that and subsequently got a "low altitude warning" from Tower controller. Lowest we saw on the RA approximately 2mi from ZZZZZ was 800'

Synopsis

A320 flight crew reported low altitude alert from ATC while on approach.

ACN: 2300184 (70 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : UAV: Unpiloted Aerial Vehicle
Crew Size.Number Of Crew : 1
Airspace.Class B : ZZZ
Flying In / Near / Over (UAS) : Airport / Aerodrome / Heliport
Flying In / Near / Over (UAS) : Aircraft / UAS

Person : 1

Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 7
ASRS Report Number.Accession Number : 2300184
Human Factors : Communication Breakdown
UAS Communication Breakdown.Party1 : Other
UAS Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2300480

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Airport ops approved a drone to be flown on the airport south of taxiway 1. Aircraft X was on approach to runway XXR. I observed the drone flying north of taxiway 1 over RWY XXL which was active. I advised Aircraft X of the drone. Then I observed the drone flying towards the approach path of RWY XXR I sent Aircraft X around. On Aircraft X second approach the arrival controller asked if they should hold the aircraft, I said yes so we could get the drone on the ground. The arrival controller told me to advise when its clear. After several failed attempts to reach the airport operation by the supervisor, Aircraft X called on approach to XXR. I advised Aircraft X of the drones position and told them if the drone headed towards them again Id send them around again. Luckily I was relieved by my sup at that time. Aircraft X landed safely without further conflict with the drone. We had no comms with the drone operator and couldn't get hold of airport ops. I recommend not flying drones on the airfield.

Narrative: 2

Forecast planned landing fuel was 1:05. I added 10 minutes of fuel. The flight was uneventful until contacting ZZZ tower. Enroute we checked notams and did not see any drone comments. We printed 3 ZZZ ATIS' enroute and did not note drone activity mentioned. The last ATIS my FO read from JeppFD Pro while on downwind and did not note any drone activity mentioned. When we contacted ZZZ tower, the controller said there was a drone which was in front of XXL and was moving in the direction of XXR. The last he said it was between the runways. I thought that the tower controller must have good eyes or good binoculars to pick out a drone. We had just passed the FAF. Shortly there after tower directed a go around. (I don't know what it says in the controllers handbook (if there is such a thing) but can't someone tell them to be quiet for about 30 seconds after directing a go around unless absolutely necessary?) Tower directed the go around and then immediately gave my FO a heading, altitude and frequency change. I delayed the go around for a few seconds to let my FO "catch up." The go around was uneventful. We returned to approach control. He gave us delaying vectors in hopes that the drone would move. After about 8 minutes we were vectored back for a visual. When we got on tower I had my FO ask tower who was controlling that drone. He said the drone was authorized to fly in the vicinity of the airport. It was to remain clear of the approach to RWY XXR and that he had no control over it or a direct line to who did. One comment was he couldn't believe airport authorities allowed the drone to fly without tower input (or something to that effect) It was my impression he had to call someone to have that someone call whoever was controlling the drone. Tower then told us to continue the approach. He said he may have to direct another go around. Shortly thereafter we were given clearance to land. The visual approach, landing and taxi to the gate was uneventful.

Synopsis

Tower Local Controller and air carrier Captain reported a UAS flying over the runway resulted in an airborne conflict during final approach and the Controller directing the air carrier to perform a go-around. The controller reported the UAS was given authorization to fly over airport property but should not have flown over the runway.

ACN: 2299680 (71 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 1801-2400

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Fuel Control Computer
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2299680
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Flight ABCD, we experienced a fuel anomaly with no ECAM or any abnormal indications where the fuel fluctuated about 3000 pounds. It dropped from where we expected it to be down and then cycled back and forth for probably about 40 minutes and caused the FOB between MCDUs (Multipurpose Control Display Unit) to read differently. The FO started keeping a fuel log based on fuel burned as it seemed to be the most accurate. After this amount of time, the fuel cycling stopped and all readings returned to normal, including the MCDUs, and the landing in ZZZ was uneventful. We elected to write the anomaly up and have MX (Maintenance) take a look once on the ground. MX was called as we were taxing in to meet the aircraft and boarding for the flight back to ZZZ1 commenced on schedule. After witnessing exactly what we had seen in the air (we had made a video to show MX but the problem also presented while we were on the ground in front of MX), MX tested the fuel system and found the error was presenting in a fuel computer. MX said they had only one part on station and it was brought to the aircraft and replaced swiftly. Unfortunately, once the part was in place it was not able to be tested, and could not be verified as working properly. MX chose to replace the new computer with the old bad computer and mentioned to the pilots that if it tested, it was good to take. The old computer was put back into place and it did test, however we did not feel comfortable taking the part that they determined was faulty in the first place. After determining we werenâ€™t going to take the aircraft, it was taken out of service and company was notified that we will be deplaning and searching for an alternative solution for the flight. Soon after, it was determined that we would take an aircraft that was arriving from ZZZ2 and continue the flight to ZZZ1. Once the aircraft arrived, the passengers were boarded and the flight continued to ZZZ1 uneventfully. Passengers and crew were kept up to date throughout the situation by a combination of CA and the company.

Synopsis

A320 pilot reported a fuel anomaly during cruise which returned to normal, flight landed uneventfully.

ACN: 2299482 (72 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2299482

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2299496

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After the First Officer landed on XXC in ZZZ got reverser 1 fault ECAM. Once slowed and transfer of controls occurred I took 1 high speed taxiway we got HYD G RSVR LO LVL ECAM. Aircraft was stopped on the intersection of 1 2 taxiway since nose wheel steering was lost. At this time we notified Tower and Operations of our situation and arranged for a tow to the gate. Once Tower was notified that we couldn't move, ARFF (Airport Rescue and Firefighting) came out to check the aircraft over and was cleared we were towed to the gate. Chief Pilot, Dispatch and Maintenance Control were all notified of our situation and what occurred.

Narrative: 2

After a normal approach and touchdown on Runway XXC, I deployed thrust reversers. Once deployed the ECAM Thrust Reverser 1 fault displayed. We both acknowledged that it was displayed and I continued to slow the aircraft. 1, the upcoming high-speed taxiway was approaching so we transferred the flight controls. As we took 1, many other ECAMs appeared. HYD G RSVR LO LVL G ENG 1 PUMP LO PR G SYS LO PR F/ CTL SPOILER FAULT The Captain told me that nose wheel steering was inoperative but the brakes were still functional. We continued straight ahead and stopped clear of the runway. We set the parking brake. We were given instructions to continue taxiing via 2 then 3 route. I then notified the Tower that we had a hydraulic system failure and to standby. There were some items on the ECAM which essentially directed us to turn off the PTU and turn off the Green ENG 1 Pump. We continued to run the ECAM and once it was completed the Captain and I discussed the next steps. Without nose wheel steering taxing to the gate was not an option. I contacted Tower to inform that we would need to be towed back to the gate and that we would be coordinating with operations. I believe the Captain was informing the flight attendants followed by the passengers about the situation. I believe he also sent a message to Dispatch notifying them of the situation. Operation was notified and they assembled a recovery crew to tow the aircraft back to the gate. A few port authority trucks arrived on the scene followed by a fire crew. We were notified that there was presumably hydraulic fluid leaking from the aircraft. Stated that there were no other hazards or visible damage on the aircraft. The tow crew arrived. After reviewing and configuring the aircraft for tow (gear pins installed) they hooked us up and towed us back to the gate. We ran the parking checklist. We put in an log book entry for the Hydraulic System failure. We did a debrief and then thanked passengers while they deplaned. Maintenance arrived momentarily and I gave them a quick briefing and then we didn't see them again. The Captain spoke with the Chief Pilot and Dispatch and Maintenance Control.

Synopsis

A320 flight crew reported loss of hydraulic system during landing. Crew ran the ECAM and were towed to the gate.

ACN: 2298081 (73 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2298081
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff we heard a loud bang followed by an ENG2 STALL ECAM caution. Engine 2 rolled back and recoverd itself quickly without any pilot intervention. The ECAM then message self cleared. I accomplished the appropriate parts of the Engine Stall QRH procedure. ATC was advised, priority handling was requested, and communication with the FAs (Flight Attendants) was established to advise them of the situation. The FAs were advised that we would be performing a precautionary landing back in ZZZ and to expect airport safety equipment to inspect us after landing. I made several PAs to the passengers explaining the same information. The Overweight Landing QRH procedure was accomplished. The landing was uneventful with the exception that we did hear another loud bang when selecting maximum reverse thrust. We stopped on the runway centerline and ARFF (Airport Rescue and Firefighting) personnel advised us everything appeared normal and they followed us to the gate. They also advised us a single puff of white smoke appeared from engine 2 after touchdown. We parked at gate XX and accomplished the shutdown checklist. We elected to do a CONF3 landing, which is contrary to what the Overweight Landing checklist calls for. The logic behind this is: 1) The A320 software calculated a required landing distance (CONF3) that was well under the amount of available landing distance is on Runway XX in ZZZ. 2) In the event of a go around, CONF3 would give us better go-around performance with a compromised #2 engine (Threat) especially considering there is an engine failure missed approach procedure specific to Runway XX. Cause: Although contrary to what the overweight landing checklist calls for, executing a CONF3 landing, helped mitigate the risks (reduced climb performance) associated with possible go-around with high terrain in the area.

Synopsis

Air carrier pilot reported loud bang after take-off. Flight returned to departure airport for an uneventful landing.

ACN: 2297777 (74 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 8000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Landing
Flight Phase : Final Approach
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Air Cycle Machine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2297777
Human Factors : Physiological - Other

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2299212
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At about 8,000 ft MSL on the visual approach into XXR in ZZZ, the First Officer and I started to notice a smell within the aircraft. To me is seemed like a plastic type smell, but wasnâ€™t that out of the ordinary compared to other aircraft. The Flight Attendants called, below sterile but due to concern regarding the smell. During this period the APU was off. Upon landing and taxiing clear of the runway the APU was turned on. On taxi in the Flight Attendant called again and now said the smell was that of a dirty sock and all three of them were getting headaches and feeling nauseous. I called Maintenance on the way in to the gate so that they could promptly meet us and experience the smell. By the time we were getting to the gate, the First Officer and I were both getting headaches from the fumes. Once at the gate and shut down I went to stand by the cabin door while the Flight Attendant #1 assisted disabled passengers. While walking past the forward lavatory the smell changed from what it was in the cockpit. In the cabin is smelled strongly of exhaust or dirty sock, seemed as though it was an over-serviced APU. I completed the Logbook write up, the company smoke and fume event worksheet within the company app, and then cross checked the FOM to ensure all required items were taken care of. I spoke with maintenance on the jet bridge further about the sequence of events. The First Officer and I then walked to our next gate, our symptoms continued to get worse. We had headaches and were feeling light headed. When we got to the next gate we met up with the Flight Attendants to see how they were feeling and were contemplating a medical evaluation. Both Pilots and Flight Attendants were feeling so poorly we did not feel fit to fly the next leg and opted for removal and medical evaluation. Cause: None, aircraft malfunction. Was later found by maintenance that the high voltage wires were smoking and there was a deteriorated drain line.

Narrative: 2

Below 10,000 MSL on our descent into ZZZ the #1 Flight Attendant called the flight deck to report a strong odor of burning plastics. We could generally smell the odor as she said something. At this time of the flight, the APU and APU bleed was off. We made a normal landing on XXR in ZZZ. After clearing the runway, the #1 called again and reported the odor was very strong. She reported that her and the #2 and #4 (A320) were experiencing nausea from the odor. We continued to our gate and were held for outbound traffic on the taxiway and ramp. The Captain made a logbook entry and notified ZZZ MX (Maintenance) as we were waiting. Once we parked at the gate, the Captain opened the flight deck and the odor was very strong near the forward galley. It was a much stronger odor than we experienced in the flight deck. The cabin cleaners came onboard and made a comment about how bad the aircraft smelled. ZZZ MX showed up a short time after and they also commented it was the worst, strongest odor they have smelled. I assisted the Captain as he filled out the smoke/fume event required by MX. When finished, we continued to our next gate. I felt an intense headache with mild light headiness when walking. When we arrived at our gate, the Captain and all three flight attendants complained of similar symptoms. We decided to seek medical treatment and removed ourselves from flying for the day. Cause: This event was caused by a substance ingested into the environmental system to cause an odor.

Synopsis

A320 fight crew reported a strong odor in the aircraft during approach and landing, requiring maintenance action.

ACN: 2297414 (75 of 2908)

Time / Day

Date : 202510

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 7000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class C : ZZZ

Component

Aircraft Component : Engine Oil Seals
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2297414
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2297415
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Going back about a week before this event happened, I flew to ZZZ1 and saw Aircraft X at a remote ramp. Contract Maintenance met us at the airplane that we flew in and I asked what is going on with Aircraft X? The contract Maintenance guy said that the right engine was just changed and a carbon seal let go and the engine lost 5 quarts of oil from the time it left the departure airport. About a week later, now I was flying Aircraft X. I looked at the history in the logbook to confirm my memory about the airplane & right engine. I briefed the FO (First Officer) about the history during the cockpit prep and the brief. Not only did we check the oil quantity at the gate, we also checked after starting the engine and before takeoff to see if there was any major change in oil quantity. Everything was normal until about 7000 feet when we got an advisory for low oil quantity (4.5 quarts) on the right engine. I asked the FO to tell ATC we want to level off at 8000 feet. By the time we got to 8000 feet the oil quantity was down to 2.0 quarts. I had a strong feeling that we were going to lose the engine soon, so I told the FO to tell ATC we are returning to the airport. We went through all of the procedures for returning back to the airport. We were an overweight landing so we did the overweight landing checklist. During the descent the oil quantity did rise to 6.0 quarts, but I figured it was due to the engine being at idle and not at climb power anymore. On final for the runway the oil quantity went up to about 12 quarts. By the time we parked at the gate the oil quantity went to 17 quarts. We wrote up the right engine for the oil quantity indication and the overweight landing. I then spoke to Dispatch. After we switched airplanes, our gate was right next to Aircraft X and we could see Maintenance had opened the right engine cowling and he was putting absorbent pads on the ground to soak up the fluid (I am assuming engine oil). I didn't ask for the FO to run the low engine oil advisory checklist because this engine had prior history of oil loss.

Narrative: 2

During our threats to flight brief, we had discussed a prior Maintenance write up about the right engine. It had an issue where it lost 5 quarts of oil due to a carbon seal. I had briefed to the Captain that on my walk around that I had checked that there was no signs of visual oil leaks from both engines, as this is part of what I look for when I visually check the oil chip detector. As well as below the engine on the tarmac for any fluids and there were none. From here on out operational parameters were normal. During taxi out and takeoff oil qty was normal till we were at around 7000 feet about to be cleared to our first nav fix ZZZZZ intersection. It was at this time that I saw the oil advisory flashing the qty and I watched it drop from 18 steadily down to 4 then quarts, and then 2 by around 8000 ft. Due to the prior history of the oil loss the decision to return was made and I then had to set up the FMS to land XXL and run the overweight procedure and also have the single engine straight in approach procedure briefed in case we lost the right engine. From the next gate over when we were in the flight deck I saw fluid dripping from the right engine. Maintenance were putting down what looked like white paper towels or some cloth that was laid down to dry up that fluid. As safety is my number one concern, I believe the Captain made the right decision to return to ZZZ due to prior oil leaks with the right engine on Aircraft X.

Synopsis

A320 flight crew reported low oil quantity indications on initial climb and return to departure airport.

ACN: 2295261 (76 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Relative Position.Angle.Radial : 90
Relative Position.Distance.Nautical Miles : 80
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 9999
Weather Elements / Visibility.Other 
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use.SID : ZZZZZ
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 6629
Experience.Flight Crew.Last 90 Days : 223
Experience.Flight Crew.Type : 6370
ASRS Report Number.Accession Number : 2295261

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Day 0, an abnormal situation occurred during flight ABCD. Situation: Everything was carried out without anomalies during the initial climb; ATC instructed us to climb and maintain FL270. Our final cruising level was FL370. Once level at FL270, an ECAM CAUTION "AIR PACK 1 REGUL FAULT" appeared, showing on the ECAM SD BLEED PAGE both PACKS in amber and position CLOSED. I assessed for a few seconds the condition of the PACK FLOW VALVES and verified on the OVERHEAD PANEL if there was any light on the pushbutton switch related to the current system status; I noticed that the information presented did not correspond to the associated ECAM. An OEB (Operations Engineering Bulletin) was mentioned and recalled, and it was discussed between both pilots that the ECAM entry did not correspond to our situation, so a possible application of said OEB was dismissed. As part of my analysis, I verified the status of CAB ALT and CAB V/S, which apparently was increasing at 900 ft/min and a cabin altitude of 5,450 ft. Actions: A few seconds later, ATC instructed us to continue the climb, to which we requested to maintain FL270. I decided to call the flight attendants normally to inform them that we were facing a possible depressurization and that they should remain seated. I determined that for safety we had to descend to 13,000 ft, an altitude previously identified during the threat management phase in the BRIEFING. Initially, descent to FL190 was authorized. During the descent between FL270 and FL190, there was an abrupt increase in CAB V/S and CAB ALT, reaching and exceeding 9,550 ft of CAB ALT, so the EMER DESCENT procedure was applied in accordance with the manual. Priority handling was requested with ATC. ATC instructed us to descend to 13,000 ft and gave an initial heading toward ZZZ, which I identified as meeting minimum obstacle clearance altitudes and confirmed that the AFS (Autoflight System) was complying with the ATC instruction. Once established in descent, the ECAM action lines were initiated. It is important to mention that the CAB ALT showed a positive trend and close to 14,000 ft CAB ALT, so the MASKS MAN ON pushbutton switch was pressed in accordance with manual/EMER DESCENT. Upon reaching 10,000 ft MSL, the First Officer removed his oxygen mask, followed by me. Due to weather conditions, operational benefits, emergency service support, and commercial and Dispatch advantages, the conscious decision was made to return to ZZZ. I told my purser that we were already at 10,000 ft, that everything was under control, and that we were returning to ZZZ with an estimated 15 minutes. In addition to notifying her of those elements, I asked about the passengers' status and added that I would make an announcement regarding what had happened and the next steps. This passenger announcement was made once the decision had been taken, emphasizing the following points: Everything is under control. We are returning to ZZZ. Estimated 15 minutes to landing. Please follow the instructions of our flight attendants. Already flying toward ZZZ, I took into account the crew's well-being. In particular, my First Officer was in good condition to begin the instrument approach to Runway XXL. I decided to transfer controls to prepare the FMGC and ensured that APPROACH C/L had been completed. We noticed that ZZZ Approach expedited other traffic to give us priority; however, I decided to notify them that we were no longer in a priority situation so that we could continue a normal approach and thus manage our workload. During final approach, we received a call from the cabin crew. Given the nature of the situation, I decided to authorize the First Officer to answer it, anticipating a possible medical situation and being in time to make the request to ATC. Medical assistance was requested upon confirming that the call was due to an infant who was unconscious. Result: The landing was carried out without incident. After landing, police services escorted us to gate XX, and upon opening the aircraft door, they entered to ensure everything was under control. A passenger announcement was made prioritizing the following points: Reason for the return. Situation under control at all times. Message of calm. Appreciation for patience and cooperation.

Synopsis

A320 Captain reported safely returning to departure airport after experiencing a problem with a pack that led to a loss of cabin pressure.

ACN: 2294641 (77 of 2908)

Time / Day

Date : 202510
Local Time Of Day : 0001-0600

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 2294641
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On that date we were working flight ABCD. The following statement is to the best of my recollection and after reviewing the video. The incident occurred a week ago, when we pushed out the flight and disconnected the tow bar, from the aircraft. I was on the right wing and was tasked with the actual disconnection of the tow bar. Person A was the driver and was on the headset in communication with the pilot. Person B was on the left wing and was tasked with notifying the pilot when all was clear. Person A pushed out the aircraft to the alley and came to a stop. Once he was stopped I walked over to the tow bar and began lowering the landing gear on the tow bar but leaving the wheels off the ground about an inch or so. I leave the wheels off the ground so that I don't create tension and/or possibly to cause damage to the equipment or aircraft in case of any movement. Once I lowered the wheels I looked and Person A to give me the brakes set signal. I then returned signal. I then proceed to disconnect the headset cord and the towbar. Right before I lift the handle to release the tow bar I normally look at the light and see if it is on. Meaning the brakes have been set. I am not 100 percent sure as this happened a week ago. I do remember lifting the handle and seeing the tow bar dropping to the latch that holds the tow bar in place. After the drop Person A reversed about a foot and stopped I noticed the plane start to roll forward. I motioned to Person A to move out of the way to reverse as the plane was moving towards the push out tractor and tow bar. He did reverse about 10 ft and stopped. I signaled to Person B to notify the pilot so set brakes to which he did. Seeing the plane was not stopping I signaled to Person A to move out of the way and clear the area as to not create any damage or possibly get injured. Person B was furiously trying to get the pilot's attention, but the pilot was not paying attention to the situation. Person B was even jumping up and down waving his wands. I noticed Person A was clear and out of the way and since the pilot was not responding to Person B I then carefully without getting ran over by the plane I tapped the fuselage and then a few seconds and about 20ft later the plane came to a stop. I then proceeded to disconnect the bypass pin and walk it over to Person B. Person B showed the pin and I believe saluted him. I then proceeded to calm Person B down as we were lucky that our quick thinking adverted a possible aircraft damage or worse possibility of an injury to a passenger, flight crew or coworker. I'm not sure why it happened. Person A said that he gave the command to set brakes and pilot said they were set. I do know that after the event it seemed the pilot was distracted. Maybe he accidentally released the brakes. The reason I say he was distracted was because Person B was trying to get his attention and he did not react or responded by setting the brakes right away. It wasnâ€™t til I believe I tapped on the fuselage that he reacted. As mentioned above. I quickly motioned for Person A to move out of the way. And signaled for Person B to get the pilot to set brakes. I believe after I tapped on the plane is when he realized what was going on. Once he set them. He did not call to return to gate or anything. Nor did he call ops. He taxi out and left. Suggestions: Honestly, I guess double confirmation on brakes being set from push back operator and pilot. Also while disconnecting tow bar pilots have to remain aware of ground crew and not distracted.

Synopsis

Ramp personnel reported the aircraft rolled after disconnecting the towbar. The pilots did not respond to ramp personnel attempts to signal the pilots to apply brakes until the reporter tapped on the aircraft fuselage.

ACN: 2293169 (78 of 2908)

Time / Day

Date : 202510

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Door
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2293169
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Attendant

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2293160
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We were traveling from ZZZ1 to ZZZ on flight ABCD an Airbus 320-20S. This was the fourth flight of five on a two day trip. I was purser. After we landed in ZZZ and the seatbelt sign went off, I disarmed doors 1L and 2L. I then made the announcement to disarm doors and then made the all call. FA A answered the all call and said doors 2L and 2R were disarmed and cross checked. The passengers and pilots deplaned, and then we did our cabin sweep and left the aircraft. I saw FA C taking a photo of door 2L but thought nothing of it, as I know some FAs take photos of the doors they arm and cross check. As we were walking to the next gate in ZZZ, I noticed FA D and FA A hanging back and talking in a hushed tone. We finally stopped at the restrooms right by a restaurant in ZZZ and I asked what was going on. FA A said, â€œShould we tell her?â€ Once again I asked what was going on. FA A told me that when she disarmed the door, she initially pulled up the white door handle â€œjust a little,â€ realized her mistake, then pushed it back down quickly. She then moved the RME [arming] lever up and told FA C what happened. I did not know any of this occurred until we were stopped near the restaurant on our way to the next gate in ZZZ, about 10-15 minutes after the incident. Neither FA C nor FA A told me about the incident while we were on board the aircraft. I sent a message to FA C in the app and told her we needed to return to gate XX to check on an issue. I was afraid the slide at 2L might deploy upon opening by catering or by the mechanics. She asked me to call her, so I did. I explained what FA A told me, and she instructed all three of the FAs to return to the aircraft at gate XX. I stayed on the line with her and she told me the Supervisor was coming to talk to us. Supvr came on the aircraft and went to the back to check the 2L door. Meanwhile, FA A called the union to discuss the situation while we were on board. One of the mechanics said all was ok, the slide didn't deploy and we were good to leave the aircraft. We got our bags and headed up the jet bridge. Supvr and Person A were standing at the top of the jet bridge and told us we could not leave and needed to return to the aircraft until he gave us further instructions. Supvr finally came on board and told FA C and I we could leave, but FA A had to stay on board because there would be an investigation. FA C and I left with Person A, and he helped get our schedules readjusted and ensured we had a hotel room for the night. FA C, FA A , and I deadheaded back to ZZZ2.

Narrative: 2

When I went to disarm the door I touched the door operating handle it went up a tad and immediately I pushed it down. I let the purser and maintenance know about the situation. Maintenance took a look at the door said it was okay and nothing was wrong.

Synopsis

A320 flight attendants reported during door disarming the door operating handle moved slightly, but was immediately pushed back in place. The escape slide did not deploy.

ACN: 2289386 (79 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Initial Climb
Flight Phase : Takeoff / Launch
Airspace.Class C : ZZZ

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2289386
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Ferry flight out of ZZZ with #2 PAK on MEL. Only flight crew aboard. Normal takeoff roll. On liftoff strong chemical / fine mist in the cockpit. No ECAM. Oxygen masks donned immediately. AC control was transferred and we missed the initial heading due to confusion. Coordinated with ATC for a return to ZZZ. We didnâ€™t get higher than 8000 feet. Once stabilized we shut off PAK 1 which triggered an ECAM which we then followed. RAM AIR on. Uneventful return and landing and runway XX L.

Synopsis

A320 pilot reported just after takeoff and during initial climb the flight deck began to fill with a strong chemical smell and mist.

ACN: 2288209 (80 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2288209
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

This event began with a routine landing in ZZZ followed by a taxi to the gate where we waited 5-10 minutes for the ground crew to park us for a remain overnight. I shut down both engines at the gate, did the shutdown checklist, and the FO (First Officer) and I leisurely gathered our belongings while we waited as the passengers deplaned. Eventually FO stated that they were going to do the post flight walk around inspection and left the flight deck. I had previously entered a logbook deferral sheet in the logbook that both engines required oil servicing. Up to this point everything was routine and there were no unusual indications or messages from the aircraft. A few minutes later the FO reappeared in the flight deck and stated â€œI think the #2 engine is on fire.â€ They then told me that looking at the engine from the rear it looked like there was a flickering flame inside the engine as you looked in the tailpipe. There were no fire indications or other anomalies in the flight deck. There were still about a dozen passengers on board so out of an abundance of caution I called the Tower, explained the situation, and asked for the fire trucks to be sent. I later learned that the crew chief had independently called for the fire trucks a minute or two earlier. I did not initiate either the engine fire or tailpipe fire procedures since it was unclear to me what the nature of the problem was at this point. I then went down to the ramp. As I approached the number 2 engine I observed that the ground crew was unloading baggage from the rear cargo compartment just a few feet behind the engine. I walked to the aft end of the engine with the FO and could clearly see the flickering light from a fire when I looked in the tailpipe. Moments later the fire trucks arrived. Just as the fire personnel walked over to us the fire apparently went out on its own. We observed for a few minutes and after discussing the situation with the FO, fire personnel, crew chief and AMT who was there to do the oil servicing we concluded that the immediate danger was over and any subsequent actions would be done by Maintenance. We did note that there was a clear trail of oil spots leading back from the engine to the taxiway, and a fresh puddle of oil under the engine. I called Maintenance Control to inform them of the situation and returned to the flight deck to enter the problem in the logbook and conduct the checklist. One of my challenges in this incident was in deciding how to accurately describe the problem. It was also late in the evening after a long day and I was certainly affected by fatigue. I wrote that the problem was a â€œTailpipe Fireâ€ and added some additional description. In hindsight this was probably wrong. I should have described it as a â€œFire internal to the engine viewed through the tailpipe.â€ Later at the hotel the chief pilot called and said that Maintenance Control was confused about what the exact problem was. Luckily the FO had taken pictures and video that I sent to chief pilot and they shared with Maintenance Control, which helped clarify things. I have thought about whether I should have initiated either the engine fire or tailpipe fire procedure. However, I never had any indications of an engine fire in the flight deck and almost as soon as I went down and observed it the fire went out with the fire department now on scene. Similarly I had no indications of a tailpipe fire. I subsequently researched in our manuals and have determined that there is no definition or description of â€œtailpipe fireâ€ anywhere that I can find. I donâ€™t actually know what constitutes a â€œtailpipe fireâ€ versus other types of engine fire, and when to apply which procedure after shutdown. Maintenance eventually determined that there was enough fire damage to the #2 engine that an engine change was required. Among other issues, one of the oil gaskets was found to be out of place and severely damaged. I would note that a previous crew had written up a suspected oil leak from the #2 engine earlier that day and it was signed off by Maintenance with no repairs made. Based on that earlier write-up, the oil consumption on our flight, and the visible oil leak after landing, I concluded that we were very lucky not to have had an engine fire in flight. First, Maintenance should be more through in investigating reported engine oil leaks. Second, it would be extremely beneficial to receive additional training on how to identify and distinguish post-shutdown engine and tailpipe fires.

Synopsis

A320 Captain reported engine fire detected after parking at the gate. Fire was quickly extinguished with no injuries.

ACN: 2286417 (81 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Fuel Quantity-Pressure Indication
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2286417

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fuel System center tank increasing and decreasing from 0-3000. FOB (Fuel On Board) was doing the same. Multiple ECAMsâ€¦Auto Fuel Feed and Center Tank Fuel Low pressure. Weather enroute and TS (Thunderstorm) at dest. High alternate fuel for ZZZ1. RTB (Return to Base) ZZZ. Overweight landing at 142,000.

Synopsis

A320 Captain reported multiple ECAM messages during climb for center tank fuel quantity, feed, and low pressure. They returned to the departure airport.

ACN: 2284853 (82 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2284853
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2285077
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

XA:15am departure out of ZZZ1 to ZZZ. Planned 1 hr sit and return flight to ZZZ1 on same tail. FAs mentioned they had come in from ZZZ2 on a red eye, were laying over in ZZZ. Departure, flight and arrival all uneventful. Taxiing into gate XX, ZZZ, in blocks, parking brake set, engine shutdown and immediately we hear loud hissing. We see the slide inflated out of door L1, jet bridge is still parked up away from the aircraft. FA calls and says sorry, they inadvertently deployed the slide. Notify Operations (Ops), dispatch and Chief Pilot. ELB (Electronic Log Book) entry made. Run parking checklist. Ops brings air stairs to left aft door to deplane passengers.

Narrative: 2

Upon arrival at the gate in ZZZ, my FO and I had shut the airplanes engine off and the fasten seatbelt sign. Almost immediately, I felt the airplane shake and heard what appeared to be the L1 slide inflating. I looked out my window and confirmed indeed the slide was deploying. I informed my FO what was happening. We made sure to return to our securing duties to ensure the airplane was parked. I got up and exited the flight deck to find the purser in the galley very upset at the events. I made sure no injuries were reported or evident and then made a PA at the front of the cabin for passengers to remain seated, what had happened and that steps were going to be taken to safely deplane the airplane. My FO and I stated coordination with ops and our dispatcher to get everyone on the same pages. We requested air stairs to be brought to the back of the aircraft and deplane on aircraft left. This seemed like the safest course of action to keep passengers away from GSE (Ground Support Equipment) and fueling equipment on aircraft right. The response from ops was swift and the passengers deplaned via air stairs. I called the Chief Pilot for a debrief and also guidance for the cabin crew as to which representatives they should contact. Ops took statements from the 2 aft FAs and informed us that the purser would need to visit the medical clinic for evaluation. The coordination between the crew was excellent to accommodate the passengers safely and I commend my crew for their professional response to this event.

Synopsis

A320 flight crew reported that during their arrival and while parked at gate, there was an inadvertent escape slide deployment resulting in the passengers exiting the aircraft by air stairs.

ACN: 2284059 (83 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Ceiling.Single Value : 5000

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2284059
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Flight (ZZZ-ZZZ1) pushed back from the gate with clearance from Ramp. Communicated completion of push with standard phraseology. CA set the PARK BRK handle to the â€œONâ€ position, scanned accumulator pressure in the green and PARK BRK indicated on ECAM before replying with standard phraseology to the TUG driver that the PARK BRK was â€œONâ€. The flight crew planned a SET and opted to wait for wave off to start Engine 1 due to the anticipated lengthy delay time to taxi out of the ramp area. During engine start, an inbound aircraft was entering the ramp at Taxiway 1. CA noticed the inbound aircraft and sensed own aircraft movement, initially thinking it was a parallax / relative motion illusion. PARK BRK was indicated on the ECAM but the Triple Gauge Accumulator needle was outside of the â€œgreenâ€. CA initiated the toe brakes and stopped aircraft movement. While overall movement seemed slight from the flight crewâ€™s perspective, it was enough to alert the Ramp Controller to query, and the inbound aircraft to stop. CA re-cycled the parking brake and verified accumulator pressure â€œin the greenâ€ and ECAM indicating PARK BRK. PARK BRK seemed operational and a scan of the HYD system page indicated systems normal. Crew decided to continue, but had to return to gate due to critical MOT (Minimum Off Time). PARK BRK was added to the Aircraft Logbook for further investigation by Maintenance.

Synopsis

Air carrier pilot reported after push back the aircraft rolled during engine start even though the parking brake indicated it was on.

ACN: 2284040 (84 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Weather Elements / Visibility : Windshear

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2284040
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2284045
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise at FL360 the crew received an ECAM Caution message that the number 1 engine had low oil pressure. There were no ECAM actions associated with this ECAM caution message so the message was cleared and the QRH referenced. There is a note in the QRH that states that if the ECAM message is in amber but the oil pressure indicates red that the crew may elect to keep the engine running as long as other engine parameters are in normal range. Low oil pressure was initially observed to be in the caution range (amber arc) on the engine page of the lower ECAM. After several seconds the oil pressure slowly dropped into the Warning range (red arc) and then began making large fluctuating movements sometimes going to zero PSI for several seconds. The observed conditions indicated by the oil pressure gauge is what led to the engine ultimately being shutdown. The engine was shut down and secured with reference to the QRH. At this time ATC was advised and a lower altitude requested. Cabin crew was notified. Dispatch was contacted and a recovery airfield was coordinate between the crew and dispatch. Customers were notified of our situation and told of our diversion. Aircraft was flown single engine and landed at ZZZ airfield. Aircraft was inspected by CFR (Crash Fire Rescue) after landing and then taxied to the assigned gate without further incident. After arrival at the gate the deplaning of customers was coordinated with Airport Operations and the aircraft was deplaned. The flight deck was also visited by attending contract maintenance in ZZZ where we learned that the oil pressure indications we saw were caused by a faulty and broken electrical connection. While there were times that procedurally, checklist and sop items became disjointed and even missed, ultimately items became trapped by the crew and accomplished. The QRH was stopped and restarted at least twice while coordinating and communicating with ATC and cabin crew. In hindsight this event felt urgent especially due to the intermittent low to zero oil pressure indication. My concern in the moment was that the low oil pressure would lead to a catastrophic engine failure. The crew continued to work through all of the requirements of the situation and as a result the approach checklist was not accomplished until well into the arrival phase of our flight. Aircraft control was predominantly maintained by the first officer except during engine shut down where the Captain guarded the thrust lever and engine switches.

Narrative: 2

The Captain and I were operating a flight from ZZZ1 to ZZZZ. Approximately 45 minutes into the flight we noticed an Amber E1 low Oil Pressure ECAM message. Upon opening the ENG page we observed the oil pressure fluctuating from amber to red. I immediately referenced the QRH and read the steps out loud but did not complete the requested actions because I wanted to discuss our next steps with the Captain. We agreed shutting down the engine was safest solution to prevent a catastrophic failure. I remained pilot flying while the Captain advised ATC and notified our inflight crew. Shortly after we requested a lower altitude and started the QRH procedure to shut down and secure the Engine. Once the engine was shut down the Captain notified the customers about the situation and that we would be diverting. I remained pilot flying and worked the radios while the Captain was on a SAT call with dispatch where It was decided we were to divert to ZZZ which I then requested from ATC. After a successful single engine landing, Crash Fire Rescue completed a visual inspection of the aircraft on the runway and again before entering the ramp area. Arriving at the gate we were met By ZZZ ops, Contract Maintenance, and a Crash Fire Rescue representative.

Synopsis

A320 flight crew reported they shut down an engine due to low oil pressure indication and diverted to a nearby airport.

ACN: 2282789 (85 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 0001-0600

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Cabin Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2282789

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

FO got locked out on lav door for 10 minutes. I called Dispatch and when [we] were going to get transfer with Maintenance Control, the FA called me that the FO was out back to cockpit. My plan of action was to use a deadheading CA Airbus pilot to assist me in case if I had to call ATC for emergency. In this case FO we got him out in time. No need to call emergency in that moment.

Synopsis

A320 Captain reported the First Officer was locked inside the lavatory for 10 minutes but was able to eventually get out and return to the cockpit.

ACN: 2282644 (86 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 12400

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class E : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 0
Experience.Flight Crew.Last 90 Days : 0
Experience.Flight Crew.Type : 0
ASRS Report Number.Accession Number : 2282644
Human Factors : Fatigue
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 0
Experience.Flight Crew.Last 90 Days : 0
Experience.Flight Crew.Type : 0
ASRS Report Number.Accession Number : 2282659
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During our descent/approach into ZZZ, I was pilot monitoring. We received the following clearance, "Cross ZZZZZ at or above 14 thousand cleared RNAV GPS X runway XX ZZZ". I set 14,000 into the altitude window of the FCP and pointed while my Captain (PF) verbally confirmed and pointed to the assigned altitude. Since we were cleared approach he selected approach mode. I verified app nav on the FMA (Flight Mode Annunciator) and saw the brick, sticks and the proper FMAs. Shortly after the FMA changed to "Final App". We set the missed approach altitude which coincidentally was also 14,000 feet. In my head, I expected the airplane to maintain 14,000 feet until reaching ZZZZZ then capture the path. I became distracted verifying the next frequency for tower on the approach plate and tuning it in. As I was doing this we received a new clearance to cross ZZZZZ at or above 13,000 cleared RNAV GPS X runway XX approach. This confused me as we had already been cleared for the approach at 14,000 so I started to say that to the Captain. At the same time I set 13,000 into the altitude select. At this point ATC issued a low altitude alert and stated we needed to be at 13,000 I looked up and saw we were approximately 12,800. I was surprised because my mental model was thinking that we would stay level at 14,000 until the IAF. My Captain disconnected the auto pilot and I pulled for open climb to reach the assigned 13,000 I believe the lowest altitude I saw during the recovery was approximately 12,400. ATC stated that the lowest IFR altitude for the area was 12,500 and that we needed to be at 13,000 ft. She then re-cleared us to cross ZZZZZ at or above 13,000 cleared RNAV GPS X runway XX. We continued the approach to a normal landing, although the 13,000 foot restriction made it challenging to capture Final App as the charted altitude there is 11,700. I continued to think about what happened as we continued on the approach. I realized that the correct sequence should have been to set 14,000 in the altitude and then come down to the FMC and hard code the altitude for ZZZZZ at 14,000. This would have honored the 14,000 foot restriction. I believe a combination of 3 legs and a relatively long duty day resulted in some fatigue on my part and lead to the error of thinking the automation would honor the restriction.

Narrative: 2

We flew a segment from ZZZ1 to ZZZ. The landing runway was XX. We were cleared to cross ZZZZZ (IAF) at 14,000 and cleared the RNAV (GPS) X RWY XX. We set 14,000 in the altitude selection window and started to descend to 14,000 feet from a higher altitude. Since we were cleared the approach, I armed the approach and saw APP NAV FMA (Flight Mode Annunciator), but we were not in FINAL APP mode. As we approached ZZZZZ, ATC cleared us to cross ZZZZZ at 13,000 and cleared the RNAV (GPS) X RWY XX. After the FO set the altitude, I noticed we were descending below 13,000 and we were in FINAL APP mode on the FMAs. I initially pushed the Vertical Speed/Flight Path Angle Selector to try to level off, but that wasn't reacting fast enough, so I disengaged the auto pilot and climbed back to 13,000 feet. I think I saw that we had descended to approximately 12,300 feet. We were subsequently recleared the approach from ZZZZZ at 13,000 feet and executed the approach to a landing.

Synopsis

A320 flight crew reported they descended below the assigned altitude at the FAF and received a Low Altitude Warning from ATC.

ACN: 2282144 (87 of 2908)

Time / Day

Date : 202509
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Elevator ControlSystem
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2282144
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

I refused Aircraft X due to a safety of flight issue based on the MEL ELAC (Elevator and Aileron Computer) 1 Pitch Fault. When we attempted to complete the MEL pre-departure procedure to verify the elevator flight controls were operating correctly the left elevator did not move up or down. Local maintenance arrived at the aircraft soon after and we repeated the procedure line by line again and again it was confirmed that the left elevator was not moving. The airplane was then written up and taken out of service. Approximately 30-45 min later the aircraft was returned to service as airworthy. We returned to the aircraft and spoke with the two local mechanics who signed off the airplane. They stated that after the power was reset that the procedure was re-run and was successfully completed. I then did witness this procedure being re-run again and I did witness the successful outcome. But, after again reviewing the history of the initial write-up and the 30 day history of the aircraft I felt that the issue was not with the ELAC 1 computer but with something down the line such as wiring or the actuator that controlled the movement of the elevator. To get more information I elected to speak to the duty pilot and we also spoke with a Maintenance Control supervisor about the issue. I still did not feel comfortable with their explanations for why when we first ran the test we did not have the left elevator movement so I then refused the aircraft and entered an write-up in the AML (Aircraft Maintenance Logbook). Cause: The refusal was due to the failure of the MEL procedure then the failure of maintenance to provide a solution to the problem other than a power reset. This ELAC 1 had a prior history of issues that had been written up more than once.

Synopsis

A320 Captain reported they refused an aircraft due to a recurring elevator control malfunction.

ACN: 2281791 (88 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person : 1

Location Of Person.Other 
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2281791

Person : 2

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2281799

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Other Person
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

After completing flight and home in bed, was informed by an air carrier crew member the R1 slide deployed on the aircraft. All systems functioned normally during flight. Post flight duties including parking, shutdown and securing of aircraft were performed per SOP.

Narrative: 2

After I was off duty I was told that Aircraft X (which I had just operated as flight ABCD) had an inadvertent slide deployment from the R1 door. There were no abnormalities for the duration of the flight. Landing, parking, securing, and post flight walk around were all performed per SOP.

Synopsis

Air carrier flight crew reported being informed of an inadvertent escape slide deployment after they were off duty.

ACN: 2281438 (89 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility.Visibility : 10
Light : Daylight
Ceiling.Single Value : 5500

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 114000
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 9500
ASRS Report Number.Accession Number : 2281438

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Day 0, flight, ZZZZ - ZZZ, Airbus 320. During landing roll leaving runway XXL, taxiway 1 at ZZZ, next ECAM (Electronic Centralized Aircraft Monitoring Computer) massages displayed, HYD RSVR LO LVL, HYD ENG 2 PUMP LO PR & HYD Y SYS LO PR. Associated to a YELLOW HYD SYSTEM LEAK. Due to loss of nose wheel steering as a secondary failure, we requested to ground control hold at the intersection of taxiway 2 and 3, which was the last point with directional control of the aircraft. Towing the aircraft was necessary to get to the gate. Once aircraft arrived at gate the failure was recorded on logbook and Maintenance start to working on it. Injuries or complaints wasn't report on cabin during this situation.

Synopsis

A320 Captain reported a loss of the Yellow Hydraulic System during landing roll, which resulted in a loss of nose wheel steering.

ACN: 2280740 (90 of 2908)

Time / Day

Date : 202508

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2280740
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft X ZZZ1 - ZZZ. CA was pilot flying, I was pilot monitoring. Weather enroute and around ZZZ. Initially set up for XXR but cell on final and tower informed us there was heavy precipitation on final from 1-5 miles from runway and LLWS advisories in effect. We asked for delayed vectors and ran numbers for XY which had a clearer final approach path. Normal approach and landing with medium auto brakes. After landing brake temperatures started to rise. When we got to gate after tires were chocked we released the brake and called ops for brake fans. They did not have brake fans. Captain did post flight to inspect fuse plugs and all tires looked normal. We advised outbound crew of high brake temps. On van ride to hotel outbound crew called to inform us both left fuse plugs did blow and wheels were deflated.

Synopsis

Air carrier First Officer reported above normal brake temperatures after landing then was advised the tires were deflated post flight.

ACN: 2280601 (91 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2280601

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

The ATIS at ZZZ was reporting windshear advisories and Rwy XX was the active runway. Approach control reported heavy precipitation on a 1 to 5 mile final so we opted to delay our approach while requested performance for Rwy XY. I excecuted a normal full-flaps landing to runway XY with medium auto-brakes and while taxiing in I noticed that the brake temperatures were increasing rapidly. At shutdown station operations informed us that brake fans were not available and some of the temperatures had reached over 600 degrees by the time we left the flight deck. The outbound Captain mentioned the possibility of blown fuse plugs at such high temperatures so I inspected the tyres and they appeared to be normally inflated. On the way to the hotel I received a call from said Captain stating that the tyres on the left main were deflated.

Synopsis

A320 Captain reported high brake temperatures after landing on a short high altitude runway. The outbound Captain discovered the tire fuse plugs melted causing the main tire to deflate, with no ground injuries reported.

ACN: 2279744 (92 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2279744

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Day 0 Flight ABC ZZZ to ZZZ1 was operated. During the Captains cockpit preparation flow it was verified that the number one and number two engines had 22 quarts of oil. During the logbook review the Captain and First Officer were aware that the number one engine was being monitored for high oil consumption. During the preflight inspection, the First Officer thoroughly inspected with a flashlight, the interior and exterior of both the number one and number two engines and observed no evidence of oil leaking. The flight taxied out and departed for ZZZ1. Approximately 35 minutes in route the First Officer and Captain recognized the oil quantities were substantially different. The number one engine was indicating 11 1/2 quarts of oil. We begin monitoring the oil quantity level in the number one engine and determined that the engine was losing a half a quart of oil every four minutes. The Captain made a sat phone call to Dispatch and was patched in with Maintenance Control. Both were made aware of the situation and it was determined the flight would not be able to continue to ZZZ1. Dispatch and Maintenance coordinated the decision to go to ZZZ2 as our diversion city. The Captain informed the in-flight crew of the diversion to ZZZ2 and the possibility of shutting down the number one engine. The Captain then made a PA to the customers informing them of our situation and diversion to ZZZ2. There was no code issued at that time. There were no ECAM (Electronic Centralized Aircraft Monitor) messages or abnormal indications, we only had an advisory message when the oil quantity on the number one engine reached 5 quarts. During the descent passing through 22,000 feet the Captain started the APU in preparation for a potential engine shutdown. During the arrival phase into ZZZ2, priority handling was issued by ZZZ Approach on behalf of flight ABC. ARFF (Airport Rescue and Firefighting) equipment was dispatched and awaiting the arrival of flight ABC in ZZZ2. After safely arriving in ZZZ2, the number one engine was indicating one and a half quarts of oil prior to engine shut down at the gate. Once the engine was completely shut down it then indicated 7 quarts. Maintenance met the aircraft at the gate and advised the Captain and First Officer that he observed zero oil in the sight glass.

Synopsis

A320 flight crew reported oil loss on number one engine requiring diversion.

ACN: 2279740 (93 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1201-1800

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2279740

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2279743

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Planned diversion due to loss of oil in the number 1 engine. While conducting post emergency tasks and duties, FO immediately reassigned to continue flight to ZZZ on flight ABC just over 2.5 hrs later. Flight ABC landed in ZZZ over 5 hours past scheduled arrival.

Narrative: 2

Planned diversion due to loss of oil in the number 1 engine. Continuing to deal with post emergency flight duties, the captain was reassigned to continue to operate to ZZZ on flight ABC. From block in to block out was just over 2 1/2 hours. Flight ABC landed in ZZZ just over 5 hours past scheduled arrival.

Synopsis

A320 flight crew reported engine oil loss resulted in diversion and flight delay.

ACN: 2278456 (94 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Lubrication Oil
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Valve/Oil System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2278456
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cabin crew reported a fume event after engine 2 was started during taxi for departure. The odor was described as engine oil, strong and persistent, with the aft cabin mostly affected. A few minutes earlier, ECAM ENG2 OIL FILTER CLOGGED message was triggered several times after engine 2 was started. Abnormally high oil pressure of 180 psi was noticed by the flight crew. We advised ATC of our mechanical issue. We were instructed to proceed onto Taxiway 1 and hold short of Taxiway 2. Maintenance Control was contacted. We were advised to shut down engine 2 and return to the gate for further inspection. As we prepared to taxi back to the terminal, the cabin crew notified us of the fume event. The A320/A321 guide was referenced. The cabin crew notified us that the odor was dissipating a few minutes later. After customers had deplaned, two FAs appeared visibly ill. Their symptoms included significantly irritated eyes, nose, throat and light-headedness. Their facial expression was very different than what I had seen during the cabin crew brief an hour earlier. The aircraft was taken out of service, and our flight was swapped into another aircraft within the hour. However, the two FAs continued to appear ill. I spoke directly to a Chief Pilot to express my strong concerns that they were not well to fly. The entire cabin crew was removed.

Synopsis

Air carrier pilot reported there was a fumes event after Engine #2 was started during taxi for departure and after returning to the gate, the reporter noticed the flight attendants were experiencing symptoms.

ACN: 2277218 (95 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Relative Position.Angle.Radial : 60
Relative Position.Distance.Nautical Miles : 40
Altitude.MSL.Single Value : 31000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 2277218

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Commercial
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1904
Experience.Flight Crew.Last 90 Days : 150
ASRS Report Number.Accession Number : 2277604

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Primary Problem : Ambiguous

Narrative: 1

Summary of Event Aircraft was dispatched under MEL XX-XX-XXX, which required ENG 2 BLEED and PACK 2 OFF. After leveling at cruise and in contact with ZZZ Center, the cockpit and cabin temperatures increased significantly. QRH PRO COND TOO HOT was consulted. Shortly after, ECAM ADVISORY, CAB PR CAB ALTITUDE appeared. CAB ALT was already in red and increasing, although no warning had yet been triggered. Crew Actions: -Requested immediate descent to FL200 due to imminent depressurization. ATC cleared descent and it was initiated. -During descent, ECAM WARNING CAB PR EXCESS CAB ALT was triggered. Initial EMERGENCY DESCENT PROCEDURE actions were carried out, including a priority handling request to ATC. -As part of troubleshooting, PACK 2 was selected ON (per MEL PROC note). PACK 2 came online, cabin re-pressurized, and conditions improved. Stabilization: -Descent continued to FL200 to monitor and stabilize the situation. -ATC was updated with the full priority handling report in accordance with company manuals. -Once situation under control, priority handling status was canceled with ATC. -Aircraft climbed back to FL290 and continued flight to ZZZZ uneventfully. -Dispatch was informed of the situation.

Narrative: 2

Sending cordial regards, hoping this message finds you well. The aircraft was dispatched with MEL XX-XX-XXX, ENGINE 2 AIR BLEED FAULT. According to this procedure, after engine start we are required to cut off the inoperative bleed along with its respective pack. This was carried out following the procedure step by step. Already in the cruise phase, a few minutes after leveling off, we received a report from our flight attendants that the cabin felt very warm. After analyzing the situation, we decided to consult the QRH procedure COND TOO HOT - COCKPIT AND CABIN TEMP IN FLIGHT. While doing so and reviewing the system display page, we noticed an advisory related to CAB PR CAB ALTITUDE. We analyzed the system and realized that the cabin altitude was in the red range, above the maximum permitted value. We requested descent from ZZZ Center to FL200 since cabin depressurization was imminent. Once the descent was initiated, we received the ECAM warning CAB PR EXCESS CAB ALT. The memory items were performed and priority handling was requested. During these actions, Pack 2 was selected ON in accordance with a note referenced in the MEL. At that moment Pack 2 engaged, and the system began to respond normally. We leveled off at FL200, analyzed the situation, informed ATC as well as Dispatch, cancelled the priority handling status, and decided to climb back to FL290 to continue to our destination without any further issues.

Synopsis

A320 flight crew reported during cruise the flight deck and cabin temperatures increased, and they got an ECAM for cabin pressure altitude advisory. They descended and pack 2 was selected on per MEL procedures. The cabin was pressurized and the conditions improved.

ACN: 2276095 (96 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 800

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Flight Phase : Initial Climb
Airspace.Class D : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2276095
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departing ZZZ, we experienced an ENG 2 failure on takeoff. Initially, there was an extraordinarily loud bang with a sudden uncommanded roll to the right. FO called engine failure. We had not yet quite reached acceleration altitude but were just below it at about 800-900ft MSL. We followed the appropriate engine out procedure and V1 climb profile, proceeded to request priority handling and diverted to ZZZ1. We ran the appropriate ECAM actions, QRH for ENG (2) Fail, and Overweight Landing checklist. Upon completing all applicable ECAM actions, QRH procedures, and checklists, we notified flight attendants, passengers, and company. Upon completing a normal landing (with one engine inop) and stopping on the runway, we were inspected by crash and rescue services. We were informed the aft section of ENG 2 was completely destroyed with loose bits of debris. Upon hearing this we opted to be towed back in case we were leaking any fluids and to avoid further potential damage with loose debris. Ultimately, we arrived at the gate under-towed. No passengers were injured and no evacuation was executed.

Synopsis

A319 flight crew member reported diverting to an alternate airport after experiencing the failure of #2 engine on takeoff.

ACN: 2275882 (97 of 2908)

Time / Day

Date : 202508

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2275882
Human Factors : Fatigue
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Due to poor/miscommunication, Parking Brake ECAM distraction, communicating with Maintenance Control, coordinating gate return, and fatigue; I released the parking brake and the A/C moved forward approximately two feet before I stopped it.

Synopsis

A320 Captain reported unintentional brake release and uncommanded aircraft movement while parking.

ACN: 2274299 (98 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2274299
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Based on the ATIS into ZZZ we set up for the visual approach RWYXX backed up by the RNAVXX. RWYXY/XZ was closed. After receiving an ATIS update shortly before landing, the wind conditions changed from a crosswind to a three knots tailwind. RWYXX at ZZZ has an LDA of 6007 feet. With manual braking, the landing distance required was about five thousand feet. Not a lot of room to spare. We ended up requesting RWYXA, and we were cleared to expect the RNAVXA under visual conditions. We opted to land with a headwind and on a longer RWY as a safer option. Below 500 feet we received two GPWS caution terrain alerts while on the glide path. We decided to continue with the approach because no obstacles were visible. The PF reduced the rate of descent and adjusted the flight path accordingly. The touchdown was executed within the first 2000 feet, within the required distance. After debriefing the approach and analyzing the warnings we experienced, we found a note on the RNAVXA approach plate indicating that the procedure is not authorized at night. We missed that note because we were more focus on the tailwind and landing distance required while executing a visual approach. On the company pages we saw the note about potential GPWS terrain alerts on RWYXX but we werenâ€™t expecting any alerts on RWYXA. Adding the same note for RWYXA would be helpful. Also, even though it was our fault for missing that note prohibiting the RNAVXA approach during night operations, advertising it in a second place under company pages would be great.

Synopsis

A320 pilot reported receiving terrain alerts while conducting an approach that was not authorized for night operations.

ACN: 2274068 (99 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2274068

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through approximately 12,000ft we received 2 ECAMs simultaneously, DOOR AFT CARGO and COND AFT CRG HEAT FAULT. We cleared the first ECAM, DOOR, and then cleared the second, COND. We immediately decided since we were climbing through altitudes where oxygen may be needed, to descend to 10,000 immediately, and continue to investigate the issue and run our procedures and checklists. We then reviewed our manifest and saw no live animals were onboard and reviewed our cabin pressurization. Cabin pressurization appeared normal at 10,000ft and level. We decided at that point we could continue since the checklist advised â€œno pilot action requiredâ€. As we reinitiated the climb after our ECAM procedures and checklist review, I kept looking at the Door page and Press page. I determined that I cannot guarantee the condition or position of the aft cargo door, decided we could be loosing cargo or have sustained damage. I had a discussion with my First Officer about it and we decided to divert and advise ATC. ZZZ seemed to be the best option, so we made a 180 degree turn direct ZZZ advising ATC, Dispatch, Crew, and Cabin of our intentions to divert to ZZZ. We prepared for runway XXR per ATC request, preparing the FMCs and running out descent checklist. Upon discovering the runway length to be shorter than XXR and we were overweight, we queried ATC for our need for XXR or alternative longer runway. We ran the Landing, Overweight Checklist, Landing Checklist and landed with out incident on runway XXR. The Fire Chief inspected the airplane on Taxiway 1 and said the Aft Cargo Door latch handle appeared to be out and open and there appeared to be damage to the door. We decided to taxi back to the gate and have them follow us. Once parked at the gate, I decided to deplane the airplane while Mx (Maintenance) inspections occurred and we could debrief with the cabin crew. We coordinated with local Mx, Maintenance Control, and the Chief Pilot until our duty period timed out.

Synopsis

A320 flight crew reported a malfunction with cargo door. Flight crew diverted and landed safely.

ACN: 2274060 (100 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B747 Undifferentiated or Other Model
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2274060
Analyst Callback : Completed

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

While in cruise in smooth air at FL320 with the seatbelt sign off, we suddenly encountered moderate turbulence that lasted approximately 2 seconds. The autopilot remained engaged and while sudden and abrupt, we did not gain or lose any appreciable altitude and our speed remained within +/- 5 knots of our intended speed. We quickly illuminated the seatbelt sign and made sure the aircraft was still under control. By the time we did this, the air was smooth again. We called the cabin via the interphone to make sure everybody was okay. The flight attendants informed us that they were okay, although they had been quite startled, and that they would check the cabin for any injuries now that it was safe to do so. While they did that, we queried ATC as to whether there was an aircraft's wake that we could have flown through, as we suspected it to be wake turbulence based on the duration and nature. They informed us that it was likely from a heavy 747 that had passed 1000' above our altitude. The flight attendants advised us that there were no injuries in the cabin and that everyone was okay. I made a PA to the passengers informing them of what had happened and assuring them that everything was under control and we would not be experiencing any more turbulence like that. We notified dispatch via ACARS. We continued to ZZZ and landed without incident.

Callback: 1

Reporter stated he would classify the wake encounter as "moderate" intensity.

Synopsis

A320 Captain reported encountering wake turbulence in cruise flight at FL320 when a B747 overflew them at FL330.