ACN: 2272411 (101 of 2908)

Time / Day

Date : 202508
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2272411

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2273621

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After I called â€œrotateâ€ we experienced a total loss of power in the #1 (left) engine. The First Officer maintained directional control of the aircraft while we determined what was happening. We followed our engine out procedures and completed the QRC, ECAM and QRH while returning back to ZZZ Runway XXR as we briefed on the ground prior to departure. After all checklists and cabin communications were completed we safely landed on Runway XXR. After clearing the runway Crash Fire Rescue (CFR) visually inspected the aircraft and nonted to anomalies. I then taxied to the gate.

Narrative: 2

After I called â€œrotateâ€ we experienced a total loss of power in the #1 (left) engine. The First Officer maintained directional control of the aircraft while we determined what was happening. We followed our engine out procedures and completed the QRC, ECAM and QRH while returning back to ZZZ Runway XXR as we briefed on the ground prior to departure. After all checklists and cabin communications were completed we safely landed on Runway XXR. After clearing the runway Crash Fire Rescue (CFR) visually inspected the aircraft and noted no anomalies. I then taxied to the gate. Within minutes of arriving back at the gate I received 4 phone calls from company. I did not answer. They then called the First Officer (FO) and reassigned us to operate ZZZ-ZZZ1 flight and wanted us to be at the gate in 10 minutes to fly. FO contacted the chief pilot and said we were not accepting the trip. He agreed and removed us from further duty.

Synopsis

Air carrier flight crew reported left engine loss of power during rotation resulting in the flight crew returning to the departure airport where they landed safely.

ACN: 2269834 (102 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZZ

Component : 1

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2269834
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While holding at ZZZZZ we were issued "descend to 3000 and slow to 180 kts." Upon selecting flaps we received an ECAM that we weren't able to identify because it cleared almost immediately. We then received another ECAM HYD G RSVR LO LVL. We went through our non normal and ECAM and QRH. We advised ATC and asked direct to the field. Upon landing we stopped on the runway and requested a tug since we had no Nose Wheel Steering. We then informed inflight of our failure, then I informed the customers that we had a system failure and required a tug to tow us to the gate. Dispatch reached out via ACARS aware of us advising ATC we let them know everyone was safe and we would contact them once able which we did as soon as we got to the gate. We later filled out the maintenance logbook in detail and spoke with maintenance control in ZZZ and left the plane with the local maintenance personnel.

Synopsis

A320 pilot reported Hydraulic System malfunction and loss of nose wheel steering.

ACN: 2269176 (103 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2269176
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Trainee. Pushed us back and we set brakes. I told them we couldn't make the turn and they needed to move us. Pushback driver told me to release brakes. I acknowledged and did and we started to roll. We only rolled maybe 2 feet. Push driver called for brakes and I gently stopped the aircraft. No damage nor was it a mechanical breakaway. Apparently we were not hooked up as they had disconnected us already, but then called for brakes release. Supervisor took over and we debriefed it. Communication was normal until the point of issue.

Synopsis

A320 pilot reported aircraft loss of control during gate push-back procedure.

ACN: 2268436 (104 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2268436
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On Day 0, during our preflight inspection of Aircraft X, we identified significant evidence of bird ingestion into the #1 engine from the inbound flight. We observed multiple pieces of bird remainsâ€”including guts and feathersâ€”on and around the #1 LP (Low Pressure) fan blades, inside the LP bypass area, and splatter/staining consistent with ingestion. The spinner also showed signs of bird impact. Further inspection revealed that the #1 LP stator vanes had visible feathers and bird remains on the leading edges. We contacted maintenance, who conducted a visual inspection. The mechanic stated he â€œran his hand along the blades and stator vanesâ€ and reported no damage. The discrepancy was signed off referencing the finding as â€œevidence of a possible bird ingestion.â€ We disagreed with this characterizationâ€”there was clear evidence that at least one bird of unknown size had entered the #1 engine. No confirmation was provided regarding whether the bird had passed through the engine core. Following the maintenance action, we performed an additional preflight and found that bird remains were still present. This included a partial bird wing in the bypass and additional solid remains on the #1 LP stator vane leading edges. Given the remaining bird-related FOD and the unresolved possibility of internal engine damage, we did not consider the aircraft airworthy until a borescope inspection had been performed and verified no engine damage. In the interest of safety, we refused the aircraft until such inspections could be completed.

Synopsis

Air carrier pilot reported refusing an aircraft after finding bird remains inside of engine.

ACN: 2268205 (105 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Ground
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2
Experience.Air Traffic Control.Time Certified In Pos 1 (mon) : 6
ASRS Report Number.Accession Number : 2268205
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X was issued taxi instructions to Runway XXL/1 via 2-3-4, which they read back. When I looked up a moment later, they had joined Taxiway 5 in the opposite direction of oncoming traffic. I instructed them to stop, because there was opposite direction traffic. Aircraft X complained about having to slam on their brakes even though they were the ones that joined the wrong taxiway, and the opposite direction traffic on 1 complained about having to slam on the brakes. Aircraft X continued to complain in an unprofessional tone. They were also taxiing faster than advisable. Share this with their company, because this kind of thing happens all of the time. They go the wrong way and then complain to ATC about it, even though they were in the wrong. It should be noted they we are all in a safety profession and should remain professional at all times. Additionally, share this with the city. The constantly changing flow of the taxiways due to construction, it's causing confusion for the pilots and we have wrong turns multiple times every single day.

Synopsis

A Ground Controller reported an aircraft turned the wrong direction on a taxiway placing it in conflict with another aircraft.

ACN: 2267593 (106 of 2908)

Time / Day

Date : 202507

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Work Environment Factor : Temperature - Extreme

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2267593
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

On preparation for takeoff from ZZZ runway XXL we setup to perform a flex takeoff. During setup I failed to identify that the outside air temperature was within 5 degrees of our set flex temp. OAT was 34 and flex temp was 36. During the takeoff maneuver after setting the thrust levers in the MCT (Maximum Continuous Thrust) detention per takeoff procedure I noticed asymmetric thrust requiring me to input a lot of right rudder to maintain centerline. The captain quickly took controls and performed a reject at around approximately 65 knots. I called tower and informed them we were rejecting and we exited the runway. After exit the brakes were hot. The captain coordinated with Maintenance Control and we called ops to taxi back to the gate. Later we discussed with a jumpseater who had been on board and he asked what the ECAM â€œENG THRUST LEVERS NOT SETâ€ meant and said he had seen that ecam just prior to our reject. However due to maintaining aircraft control my crosscheck was mostly outside and at the engines indications and I never saw the ecam before the reject. If I had seen it we could have set TOGA and continued the takeoff. Moving forward I will make sure to pay closer attention to OAT and follow the bulletin posted for this situation.

Synopsis

A320 First Officer reported failure to notice temperature exceeded parameters for a FLEX takeoff resulted in an ECAM alert and a rejected takeoff.

ACN: 2267580 (107 of 2908)

Time / Day

Date : 202507

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 800

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Ice/Rain Protection System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 147.25
Experience.Flight Crew.Type : 4109.97
ASRS Report Number.Accession Number : 2267580
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

During takeoff out of ZZZ climbing though 800 feet we got an ECAM to switch the Capts air data to position 3. We followed the ECAM which restored my air data. This ECAM was followed by more ECAMS letting us know that all the Capts probes had failed. We completed all the ECAMs and checklists. Out of about 10,000 I asked a Captain who was in our jumpseat to back up my FO while I talked to Dispatch and Maintenance Control. I briefed them on the maintenance issue. Dispatch agreed with me that the best course of action was to continue to ZZZ1. ZZZ1 at the time had better weather than ZZZ. I advised dispatch that I felt it would be a good idea to request priority handling into ZZZ1. There were delays due to arrival traffic and I didn't want to get put into holding while having Avionics issues. The weather was clear during cruise and the temperature was well above freezing at ZZZ1 at the time. All of the anti icing failed on the captain's probes as well. Dispatch agreed and we proceeded to ZZZ1. I elected not to brief the flight attendants at this time. If the systems degraded further we would bring them into the loop. After requesting priority handling we were giving priority handling and initially flew an ILS which turned into a visual approach to XXR. Once on the ground we canceled priority handling with ATC and proceeded to the gate.

Synopsis

A320 Captain reported an air data malfunction during the initial climb. The Captain considered the weather at the departure airport then made the decision to continue the short leg to the destination airport and landed.

ACN: 2267103 (108 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Vehicle Driver
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 2267103
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X ZZZ on [Taxiway] 1, Lektro push: instructed to push back to [Taxiway] 2. Agent performing hand signals due to ramp. Airbus 320 blocking crew from pushing back to 2, crew disconnected from aircraft (Airbus 319) adjacent to [Taxiway] 3. Crew went to return to Gate (XX). Aircraft 320 taxis around outbound plane and cuts ground crew off from returning to gate. Lektro stopped before being hit by aircraft and two agents ran back away from incoming aircraft to safety in the [Taxiway] 4 envelope. Team receiving next [Taxiway] 1 aircraft stopped plane from entering the gate and allows ground crew to return to gate safely and park. I was the pusher in the Lektro during the incident. Cause: Pilot failure to allow ground crew to safely return to gate. Crew Chief notified safety and filed an incident report. Suggestions: Pilot awareness to safety of ground crew.

Synopsis

A Lektro tug driver reported a taxiing aircraft maneuvering around another aircraft on the ramp almost collided with the tug and two ramp agents.

ACN: 2266526 (109 of 2908)

Time / Day

Date : 202507

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Taxi

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 2266526
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X arrived in ZZZ from ZZZ1 operating as flight ABCD. Earlier in the day, we deferred the APU generator under MEL XX-XX-XXX. This MEL requires that an engine be left operating until ground AC power is connected and established at the gate. When the aircraft came to a stop, one ramp employee began entering the red safety area in the vicinity of the still operating engine. My colleague immediately gained this individual's attention and informed him that the engine was still operating. The ramp crews in ZZZ have been experiencing an ongoing high employee turnover rate. Coincident with the constant staffing changes, I have noticed a trend of "lack of respect" for running aircraft engines. While not appearing malicious in nature, it appears that a significant percentage of ramp personnel simply do not appreciate the dangerous environment within which they operate on a daily basis. On multiple occasions, I have observed ramp personnel placing cones and walking in front of engines which had been shutdown less than just 2 or 3 seconds prior. Even though the engines were technically shut down in these instances, they had hardly even begun to spool down by this point. I have brought this fact to the attention of the individuals in question each time. However, it has always been a different individual on every occurrence. Again, while I don't feel the actions I've witnessed were committed with any ill intent, I do feel they exhibit a lack of awareness and a normalization of deviance. Also of note, inoperative APUs and inoperative APU generators result in a change from routine operating procedures. Engines may be running longer than anticipated, or they may need to be started out of the usual sequence. When able, at a minimum, a shift briefing or group huddle to discuss any specific issues should occur prior to the arrival and/or departure of aircraft which fall into this category. I believe this step may in fact already take place in our station. I am not privy to what specific training the ramp personnel currently receive regarding the hazards of engine ingestion and jet blast. I do feel, however, that a dedicated training module highlighting these particular threats is warranted before the next deviation claims a victim. Perhaps providing case studies of engine ingestion and jet blast events may drive the point home.

Synopsis

Technician reported a ramp employee walked into the red safety area while the engine was still on. Another Technician stopped the ramp employee. Reporter stated these types of events have become a common occurrence.

ACN: 2266100 (110 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 844.85
Experience.Flight Crew.Last 90 Days : 192.25
Experience.Flight Crew.Type : 844.85
ASRS Report Number.Accession Number : 2266100
Human Factors : Workload
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 498.20
Experience.Flight Crew.Last 90 Days : 139.35
Experience.Flight Crew.Type : 498.20
ASRS Report Number.Accession Number : 2266783

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

All procedures and indications were normal upon the main cabin door closing. After the jet bridge had pushed away, there was about a five minute break until the cargo door was shut, and there was verbal contact through the headset communication with the ground personnel. The entire time communication was clear and concise through the intercom and as the First Officer, I was listening to the intercom on a very low volume two monitor ground operations as it was busy and there were a lot of push backs off the XX to XD gates along with ground traffic such as employee busses, fuel trucks, baggage carts and catering vehicles. The Captain after communicating through the intercom with the ground personnel , he gave me the go-ahead to contact ramp to request to push back off of Gate XC. We were immediately told to hold the push for ground equipment and buses behind us from other aircraft pushing around us creating a traffic jam with the employee, buses and other ground equipment. The ground crew stood by until we were given the go-ahead to push from ramp controllers. Once ramp cleared us to push back from XC. I as the First Officer turned on the beacon and gave the Captain to go-ahead to push off of XC. The ground crew acknowledged immediately and began to push back. Everything had a normal sequence of events, including the clearance to go ahead and start engines from the ground crew in the middle of the push sequence. The Captain gave me a clear command to go ahead and start both engines as XC is a 2 engine gate. So I began to do so, starting engine 1 first. After a brief heads down moment, I reached up to start the clock for situational awareness on the engine start. This being about 5-6 seconds into the initial start sequence of the engine 1. I had noticed the push back had stopped, and looking outside realize we were inching forward very slowly. This seemed off, as the sight picture was in the middle of the push as far as distance backwards from the gate. I immediately verbalized "are we moving forward " to the Captain. We are both a little perplexed for a brief one or two seconds from the startle factor as we were looking at the Marshaller at our 12 o'clock with no expression in his face and no movement with his hands or wands. There was no loud noises or movements to indicate a breakaway from equipment failure. By this time, our engine had spooled up to about 40% and our aircraft had slowly started progressing faster forwards. Out of the corner of my eye, I saw the tug right underneath the First Officers right hand window in a position too far backwards to be attached by just instinct. I immediately pressed the brakes and held them. Telling the Captain immediately I had pressed the brake pedals and was holding them. Throughout the sequence of events, the Captain, nor I had heard any ground personnel communicate verbally that there was a breakaway or that the tow bar had been inadvertently disconnected. Or that the push was complete. And once we had rolled to far forward for this to be under control of the ground personnel, the Marshaller never indicated anything other than a X shape with his wands. The Captain and I also never heard the ground personnel say set brake or any verbal communication during the roll forward, until the Captain asked what happened and if that was them that moved us forward or if we had a breakaway. We got a response from the ground personnel stating there was a issue and that the tow bar was disconnected and there may be a small scrape on the paint but they we're going to reconnect and reassured us there wasn't anything wrong and the details were very unclear. "Just a minor scratch". The Captain and I both communicated internally stating we are both uncomfortable with what happened and we're going to be contacting Maintenance and returning to the gate. So with the APU still running I asked the Captain if it was ok to shut down the running engine and he confirmed so I began the shutdown flow. About that same time ramp had called us and asked if there was anything of the ordinary. I quickly communicated that I believe we just had a breakaway and that we were holding brakes and we were returning back to the gate to have someone from Maintenance take a look. They indicated they agreed and that what they saw on the cameras indicated a mandatory call to Maintenance as there was possible physical contact with the equipment. I then communicated with ramp had said to the Captain, and he then clearly communicated via the headset through intercom to the ground crew that we would need to return to the gate. They answered back immediately stating OK that they would take us back. Upon pulling us back into the gate, the tug driver did about five or six back-and-forth sequences to try and get the nose of the aircraft turned into the XC parking line. The Captain and I both thought that was unusual. Once at the gate we completed the parking checklist to ensure the aircraft was secured properly. Once back at the gate with the brake set I immediately alerted ramp via the radio that we would be quite a while at the gate and to have the aircraft waiting for our gate that was inbound to go to another one. After that was done, I immediately sent a message to maintenance stating, "breakaway on push back tug driver mentioned scrape on paint". As soon as that was done, I contacted Operations via radio and advised them that there was a possible breakaway situation and damage, and that I had sent a request to Maintenance. They were already aware by that time. Subsequently I called Maintenance via radio and advise them of the possible breakaway and damage to the aircraft or tug. They were aware and said to open the main cabin door, so a Maintenance Supervisor can come on board and talk to us. I immediately reached up for the forward and aft flight attendant call buttons and let them know we had returned to the gate due to possible damage to the aircraft and that we would need disarm the doors for the jet bridge to be connected and the main cabin door to be opened. They acknowledged and made the announcement to the other flight attendants to disarm doors for arrival. About 10 minutes after the jet bridge was connected, the Maintenance Supervisor came on board and had mentioned they had talked to the ground crew that pushed us back. They said that the tug had disconnected from the tow bar, and that the tow bar was still attached to the aircraft during the breakaway. And that there is a large gash on the aircraft that was going to down the aircraft for the day and to go ahead and de-plane the passengers once we got to go ahead from operations. After confirming with operations via the radio and the gate agent on the jet bridge the Captain made a PA announcement to everyone explaining that we were going to exit the aircraft with all of their baggage and that we were going to have to change aircraft. Once everybody goes off the aircraft and Maintenance confirmed they were going to take the aircraft to the hanger and we're ready for the aircraft to be shut down completely, the Captain and I did the termination checklist and called operations to then go and do a drug testing on the airport grounds.

Narrative: 2

All procedures and indications were normal upon the main cabin door closing. After the jet bridge had pushed away, there was about a five minute break until the cargo door was shut, and there was verbal contact through the headset communication with the ground personnel. The entire time communication was clear and concise through the intercom. The First Officer was monitoring as well. Ground operations were busy with pushbacks off the XX to XD gates along with ground traffic such as employee busses, fuel trucks, baggage carts and catering vehicles. Parking brakes were released with clearance from the ground crew. I instructed the First Officer to request clearance to push. We were immediately told to hold the push for ground equipment and buses behind us from other aircraft pushing around us creating a traffic jam with the employee, buses and other ground equipment. The ground crew stood by until we were given the go-ahead to push from Ramp Controllers. Once ramp cleared us to push back from XC, the First Officer (FO) turned on the beacon and gave me the go-ahead to push off of XC. The ground crew acknowledged immediately and began to push back. Everything had a normal sequence of events, including the clearance to go ahead and start engines from the ground crew in the middle of the push sequence. I cleared the FO to start both engines as XC is a 2 engine gate. We noticed the pushback had stopped, and looking outside realized we were moving forward very slowly. I assumed the ground crew were moving us to a better angle for a left turn out. The Marshaller off the front left side of the aircraft made no indication anything was abnormal. I asked the FO to confirm on his side the Marshaller was following along. The FO noticed the tug was out of position and disconnected from the aircraft. He immediately applied max braking. This seemed off, as the sight picture was in the middle of the push as far as distance backwards from the gate. There was no loud noises or movements to indicate a breakaway from equipment failure. Throughout the sequence of events, neither the FO nor myself heard any ground personnel communicate us verbally that there was a breakaway or that the tow bar had been inadvertently disconnected. Or that the push was complete. We were never instructed to set the parking brake. Once we had rolled too far forward for this to be under control of the ground personnel, the marshaller never indicated anything. We got a response from the ground personnel stating there was an issue and that the tow bar was disconnected and there may be a small scrape on the paint . He reassured us there wasn't anything wrong and the details were very unclear. "Just a minor scratch". The FO and I both communicated internally stating we are both uncomfortable with what happened and we're going to be contacting Maintenance and returning to the gate. With the APU still running the FO asked for clearance to shutdown the running engine. When the aircraft was stopped I asked the ground crew if I was cleared to set the parking brake. I was instructed to hold. The ground personnel inserted the chocks then gave clearance to set the parking brake. About that same time ramp the FO applied the brakes ramp asked if there was anything out of the ordinary with the push. The FO quickly communicated that we believed we just had a breakaway and that we were holding brakes and we were returning back to the gate to have someone from maintenance take a look. The indicated they agreed and that what they saw on the cameras indicated a Mandatory call to Maintenance as there was possible contact. The FO relayed the call from ramp and I communicated via the headset intercom to the ground crew that we would need to return to the gate. They answered back immediately stating OK that they would take us back. Upon pulling us back into the gate, the tug driver did about five or six back-and-forth sequences to try and get the nose of the aircraft turned into the XC parking line. The FO and I both thought that was unusual. Once at the gate we completed the parking checklist to ensure the aircraft was secured properly. Once back at the gate with the brake set we immediately alerted ramp via the radio that we would be delayed at the gate and to have the aircraft waiting for our gate Go to another one. After that was done, the FO sent a message to Maintenance stating, "breakaway on pushback tug driver mentioned scrape on paint". As soon as that was complete we contacted Operations via radio and advised them that there was a possible breakaway situation and damage, and notified Maintenance. They were already aware by that time. Subsequently we called Maintenance via radio and advise them of the possible breakaway and damage to the aircraft or tug. They were aware and said to open the main cabin door, so a Maintenance Supervisor can come on board and talk to. About 10 minutes after the jet bridge was connected, the Maintenance Supervisor came on board and had mentioned they had talked to the ground crew that pushed us back. They said that the tug had disconnected from the tow bar, and that the towbar was still attached to the aircraft during the breakaway. There was a large gash on the aircraft. After confirming with operations via the radio and the gate agent on the jetbridge I made a PA announcement to everyone explaining that we were going to exit the aircraft with all of their baggage and that we were going to have to change aircraft. The FO and myself were instructed to report for mandatory drug testing.

Synopsis

Air carrier flight crew reported the tow bar broke off during push back and came in to contact with the aircraft.

ACN: 2262112 (111 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2262112
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Yesterday while operating flight ZZZ â€“ ZZZ1, Aircraft X, we experienced a fumes event. When I arrived at the plane, it was hot and musty, and there was a very faint chemical odor in the cabin. Since the air wasnâ€™t hooked up and the temperature was nearing 80, I started the APU, and turned on the APU bleed. Once on, the noxious odor got worse, and the flight attendants came into the cockpit to let me know about it. I went out to the cabin and walked the length of the plane to confirm, then called Maintenance Control to come inspect. While waiting I turned off the APU bleed. Once Maintenance Control got there I troubleshooted with him, and then made a logbook entry, gathered my things and exited the plane. All things considered, I probably spent about 30 â€“ 40 minutes in the aircraft. They decided to take that aircraft out of service, but about 30 minutes later while the new plane was being towed to our gate, I felt lightheaded, as well as a moderate headache coming on. I was also starting to perspire, so thatâ€™s when I called crew services, medical advisory service, and the Duty Pilot to start the fumes event process. Medical advisory service deemed me and my First Officer unfit to operate, and at around XA:00, still feeling sick, I decided to check myself into the hospital. I was discharged at XG:00 after 6 hours of observation.

Synopsis

A320 Captain reported experiencing physiological symptoms a little while after the flight crew turned on the APU bleed that worsened a noxious odor that was previously faintly present in the cabin.

ACN: 2260573 (112 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : DC Generator
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2260573
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We landed in ZZZ and were taxing to the gate when we were advised by ops that it would be a 30-40 min wait for a gate. We coordinated with ground and parked on a taxi way. Our APU was MEL'D, and we were in a SETWA (Single-Engine Taxi Without APU) aircraft. We shut down engine 2. I told passengers that we will be waiting 30-40 min for a gate, but never stated it was ok to get out of their seats, and I did not turn the seat belt sign off. After 5 min we were informed by ops that a gate opened and we could proceed in. Within moments of receiving the info from ops, the FA called and told me people were out of their seats. I told the FA that we just got a gate, and I would make an announcement. I then made the announcement that we had a gate open for us and we would be proceeding to the gate, and to remain seated. After a few minutes of coordinating with ground and ramp, we started taxing to the gate. Within seconds of starting to taxi our only generator failed, and all the displays went dark in the flight deck including our ECAMs. I also heard engine 1 spool up. I stopped the aircraft rather quickly. I then heard screams from the cabin, which I assumed the lights turning off in the back frightened passengers. I made a "remain seated announcement and we were dealing with a mechanical issue." We had no engine gauge instrumentation so I could not tell the status of engine 1. But we had no APU to get another generator online. We told ground control and they asked if we would like a tow in, and I said yes. We consulted the QRH and found a procedure to reset gen 1, we did that and gen 1 came online. But we had no nose wheel steering. We thought the safer option would be to get towed into the gate still, and we were worried the generator would fail again if we tried to crossbleed start engine 2. My assumption was right and the generator failed again while waiting for the tow in. Also the FA called me and told me we would need medics at the gate as one of the passengers was still up when we restarted our taxi to the gate. They fell and hit their head in the fwd galley during the abrupt stop. We called ops to get medics at the gate. Once we were hooked up to a tug, I told the passengers that we would be shutting down engine 1 and we would be towed into the gate. Once hooked up to the tug, I told the ground crew that I could lose them when I shut down engine 1, and to chock-us when they get to the gate and use hand signals. I shut down engine 1, and we were towed to the gate.

Synopsis

A320 Captain reported during single-engine taxi with inoperative APU, the generator failed resulting in loss of cabin lighting and nosewheel steering. Aircraft was towed to the gate.

ACN: 2260380 (113 of 2908)

Time / Day

Date : 202507
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 35.15
Experience.Flight Crew.Type : 265.47
ASRS Report Number.Accession Number : 2260380
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Coming in from [taxiway] 1 taxiing to gate XX. My attention was on the gate and Marshallers. The FO yelled stop. I slammed on the brakes and a bus passed within 10 feet of my nose. No one was injured but the bus never slowed down. This was scary.

Synopsis

Air carrier Captain reported a bus drove in front of the aircraft during gate arrival requiring the Captain to slam on the brakes to avoid a collision.

ACN: 2259588 (114 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2259588

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise flight we received above ECAM alert and corresponding low fluid indication on HYD page for G system. I confirmed FO had controls and we proceeded with Non-Normal procedures. ECAM drove us to PTU..OFF. Then GREEN ENG 1 PUMP..OFF. We finished all Non-Normals with no negative effects on flight controls while in cruise. Just Prior to this event we had an intermittent SAT phone so we contacted Dispatch who included Maintenance control. We confirmed malfunction and were directed to continue to ZZZ and they would notify ATC and CFR in advance the details of our issues. The FO and I discussed the details of our malfunction, what we could expect, as well as possibilities if our conditions deteriorated. We were still about 90 minutes our from destination. We decided to take a bathroom break and stretch while we had some time. I took the opportunity to brief the number 2 FA while I was in the galley explaining our situation. Once I returned to the cockpit I went over the details with the 1 FA. We should land normally. But in the final phase of flight is where we find out if the gravity gear drop and lock. I told her if I perceive any problems it would be close to landing and I would announce 'Cabin Crew at Stations'. I believed this occurrence would be unlikely but possible. CFR would be standing by and we would be towed to the gate. As we got into ZZZ airspace and talking with Approach, we asked for CFR to be standing by for our arrival. They were already aware of our situation and prompted us they would go ahead and handle us a priority aircraft. We informed them of our remaining fuel in lbs and souls onboard. The arrival and approach was all normal and within FAR's. We knew configuring the aircraft would take a little more time with our malfunction so I began to slow and configure the aircraft early. I was flaps 2 and roughly 180 kts when I manually gravity dropped the gear the asked for the gear handle after completion. We indicated down, 3-green and the gear page confirmed gear position. I landed smoothly without any issues. I was able to confirm immediately we had zero turn control with rudder pedals or tiller wheel. I was able to control our direction with differential braking. While using differential braking, we had plenty of runway left and I felt in good control of the aircraft and decided to slow roll to the high speed taxiway 1 â€“ towards the end of XXR. I mentioned to the FO if I was unable to safely make the exit we would stop on the runway XXR. I was able to exit and clear the runway safely on centerline taxiway 1 and come to a complete stop. I started the APU start sequence so I could shut down both engines. I made an announcement to the passengers that we had a steering control indication on the ground and we would shut down to be towed to our gate. CFR and Maintenance approached the aircraft and made us aware we were dumping residual hydraulic fluid all over the taxiway from Engine 1. We made the decision to push the ENG 1 fire handle to close all valves to the engine. By doing this we were able to stop the flow of fluids out of engine 1. Maintenance installed gear pins on all the gears and we were towed to the gate and deplane normally.

Synopsis

A320 Captain reported experiencing loss of G hydraulic system in cruise flight. Flight continued to destination airport and landed safely.

ACN: 2256780 (115 of 2908)

Time / Day

Date : 202506

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 2256780
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While preforming a 2 Service on Aircraft X mechanics found a through Bolt missing and another one backed out. Tyre S/N XXX. Tyre was installed on Day 0 Tracking tag number for tire XXXXXXX

Synopsis

A320 technician reported while performing a 2 service, they found a landing gear wheel through bolt missing and another one backed out.

ACN: 2254942 (116 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 129.38
Experience.Flight Crew.Type : 434.13
ASRS Report Number.Accession Number : 2254942
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had a Green Hydraulic system low level ECAM at FL 300. We ran the ECAM. Then consulted the QRH. We had lost all hydraulic fluid and pressure on the green system. Dispatch was notified. I decision was made to divert to ZZZ. Dispatch agreed. The Chief Pilot was also informed of this decision. [Priority handling was requested] all appropriate checklists were run. Landed in ZZZ.

Synopsis

Air carrier Captain reported loss of hydraulic pressure during cruise which resulted in a diversion to a nearby airport.

ACN: 2254935 (117 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 87.1
Experience.Flight Crew.Type : 755.05
ASRS Report Number.Accession Number : 2254935
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Object
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Upon arrival at ZZZ, station operations informed us that due to lightning in the vicinity the ramp was closed and a PAATS (Park At Any Time System) truck would guide our parking. After parking, the PAATS driver asked us to verify brakes were not hot (they were not, but brake temperatures were still increasing post-landing) and that the parking brake was set (it was). After completing the parking checklist, thinking I had seen the chocks installed signal from the PAATS truck driver, I released the parking brake to slow the brake heating. Meanwhile, the jet bridge had been extended to the airplane, and door 1L had been disarmed and opened to deplane passengers. Shortly thereafter an alarm sounded in the jet bridge and I sensed movement outside my left side windows. I went back to door 1L to determine what was happening as the aft portion of the jet bridge was making contact with the aft portion of the open door. Based on the alarm in the jet bridge, I thought the movement I had sensed earlier was uncommanded movement of the jet bridge. But after talking to station operations personnel, we determined it was the aircraft that was moving, and that wheel chocks had not been installed.

Synopsis

Air carrier pilot reported the aircraft rolled forward at the gate resulting in the aircraft striking the jet bridge.

ACN: 2254766 (118 of 2908)

Time / Day

Date : 202502

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Rudder Control System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 2254766
Human Factors : Other / Unknown

Person : 2

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 2254028
Human Factors : Other / Unknown

Person : 3

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 2250807
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Aircraft X Rudder Control Rod between the rudder mixer and quadrant was found with the anti-rotation tab misaligned, with broken safety wire attached. Double check my work.

Narrative: 2

I was told by MX (Maintenance) to Buy-Back the installation of the rudder control rod only. This was performed on an Open Task card item. There was no open non routines for adjustments of the control rods needed. Just an install of the control rod and continue with the ops checks for the ferry flight ZZZ1. no other open items other than the installation of the control rod. I was involved with the installation only. The removal is mechanic.

Narrative: 3

Maintenance Safety has discovered an apparent discrepancy on Aircraft X related to a control rod between the rudder mixer and quadrant, in which it was found with broken safety wire and the anti-rotation tab misaligned on Day 5. A maintenance record review found this area was previously worked on under a WO (Work Order) on Day 5. I just wanted to add that the control rod had been removed and installed on Day 0. The aircraft then ferried out to ZZZ1 on Day 2 where this apparent discrepancy was found.

Synopsis

A320 technicians reported that they were notified by Maintenance Safety that the control rod between the rudder mixer and quadrant was found with broken safety wire and the anti-rotation tab misaligned.

ACN: 2253973 (119 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2253973
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2253975
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After coming to a complete stop at the gate, I did my parking flow as normal and was waiting to turn off the beacon when both the First Officer and I noticed the aircraft began to roll backwards. I immediately pressed the brakes and the aircraft rocked a bit as we stopped. I looked down and noticed that the parking brake was off and so I immediately set the parking brake. I then finished my parking flow and the First Officer completed the parking checklist. Obviously, I had not set the parking brake as the first step in my flow. I opened the door to check in the back and inflight said everything was fine. I also spoke with ground personnel. They also said everything was fine.

Narrative: 2

Upon coming to a stop at the gate, as the Captain was performing his parking flow, we noticed the plane started moving backwards. I started to push brakes and the Captain also promptly applied his brakes. We both noticed the parking brake was off. The Captain set the parking brakes and I completed the parking check list.

Synopsis

Air carrier flight crew reported they forgot to set the parking brake at the gate and the aircraft rolled backwards.

ACN: 2253952 (120 of 2908)

Time / Day

Date : 202506

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : ZZZ
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2253952

Events

Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Controller pointed out traffic and turned us to intercept localizer. Traffic continued towards us, passed under us 300â€™ below. Got an RA to monitor vertical speed. RA was complied with per Manual procedures.

Synopsis

Air Carrier pilot reported they responded to a TCAS/RA for traffic passing 300 ft. below them.

ACN: 2253570 (121 of 2908)

Time / Day

Date : 202506

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Initial Approach
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2253570
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Terrain Warning
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

During the approach into ZZZ we were given vectors onto final above the glide slope, this combined with us being empty led to having a tough time getting down to join the GS the usual way. I clicked the Autopilot off to manually push the nose over and attempt to capture. I quickly realized that we would not have made it to the GS and configured in time to be stable so I pushed to level off and told the First Officer (FO) to report going missed. He made the call and I pushed to level off at ~1200 ft. Only being at flaps 2 and still +1000 ft I intended to suspend the approach and not do a full go-around. I commanded gear up and a few moments later we received the too low gear aural alert. A few moments later we received it again and I pushed the thrust levers to TOGA to initiate the go-around in the FMS. We received another too low flaps alert soon after. I noticed the speed climbing, called for flaps 0 and then as the speed continued to climb I initiated the climb from ~900ft using the side stick as I had turned the Autopilot off. We climbed up to the altitude assigned in the go around and returned to ZZZ without any other issues. I had lost situational awareness and was commanding the airplane as if the Autopilot was on which led to an undesired aircraft state.

Synopsis

A320 flight crew reported a go-around due to unstabilized approach.

ACN: 2253367 (122 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : ILS XXR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 224.27
Experience.Flight Crew.Type : 1477.60
ASRS Report Number.Accession Number : 2253367
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Function.Flight Crew : Trainee
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 82.68
Experience.Flight Crew.Type : 0.00
ASRS Report Number.Accession Number : 2253359
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Inflight Shutdown
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft was dispatched with FADEC class 2 message. (For info only. Possibly unrelated). The flight was uneventful until about 13 minutes before top of descent, position approximately 70nm east of ZZZ. We received ECAM caution msg ENG 2 compressor vane, EGT was higher than engine 1 but still in normal margins. Only ECAM action was "avoid thrust changes". The speed mode was switched from managed to selected to avoid thrust changes. Dispatch and Maintenance Control were notified via ACARS. 13-15 minutes later ATC issued descent clearance. Aircraft started descent, maintaining the same thrust settings and accepting the increase in speed. During the descent the thrust was later gradually and manually reduced to an intermediate setting and engines continued to function normally. As the power reduction continued and slightly above idle, bangs and vibration was felt and associated ENG 2 STALL ECAM with the EGT increasing into the RED. The engine was idled , ECAM accomplished and engine was shut down per SOP. As the EGT had exceeded the normal parameter with the highest we observed to be 757, we determined that constitutes severe engine damage, therefore agent 1 was discharged per the ECAM. [Priority handling was requested] with ZZZ Center and a lower altitude was requested. Initially 16,000 was issued, but avoiding the need for level segments with the good engine at full thrust trying to maintain altitude just below the 1 engine out ceiling (141,000 lbs with anti ice on), we requested 10,000 which was granted as requested. Flight Attendants (FA) where given a briefing and a PA was also made to the passengers as the bangs and vibration as well as the power xfr were all observed in the cabin. Dispatch was informed of the [situation] and shut down via ACARS. Upon checking with ZZZ Approach, we were given vectors for Runway XX. We declined Runway XX and requested Runway XYR as it was longer. Another brief was given to the FAs. ATC wanted to put us on a visual approach, however a request for the ILS which was granted. The aircraft was configured on profile, and all stabilized approach criteria were met. Landing in TDZ on centerline utilizing MED brakes and only engine 1 reverse. Aircraft came to a complete stop on the runway and parking brake was set. Communication with Rescue and Firefighting (RFF)/Crash Fire Rescue (CFR) services established on ground frequency. An inspection by RFF/CFR services revealed no anomalies on the exterior and brake temps to be good. All ground equipment was confirmed clear of the aircraft. PAs completed keeping passengers updated of the event. Aircraft exited runway at its own power and taxied to the gate with normal passengers disembarkation via jet way. Cabin crew debriefed. Post flight pilot debriefing completed. Dispatch and chief pilot contacted and debriefed.

Narrative: 2

Pilot monitoring (PM) in left seat for Captain IOE, Engine 2 shut down. Aircraft Dispatched with CLASS 2 FADEC status message. During cruise at FL300 received ECAM caution msg ENG 2 Compressor vane at time XA:33. Engine indications were normal. The ECAM directed to avoid rapid thrust changes. Referenced checklist. ECAM action completed. Dispatch and Maintenance Control informed via ACARS. For descent, as power smoothly reduced toward idle, a loud bang and a chugging was heard and felt. At XA:50 ECAM ENG 2 STALL. The engine was idled per ECAM action. EGT continued to rise. Aircraft and radios given to right seat pilot flying (PF). Left seat PM completed ECAM actions. EGT OVERLIMIT Engine 2 was shut down per ECAM and FIRE P/B pressed and AGENT 1 discharged. [Requested priority handling]. Lower altitude requested. Flight attendants called up after generator loss led to the in flight entertainment screens turning off. A quick test brief was given about the engine failure and I told them I would call back with more info. After completing all ECAM directions, discussing a plan and referencing the checklist, another call was made to all flight attendants to further discuss the briefing, evacuation was not planned. An announcement was then made to the passengers that due to a mechanical malfunction we had to shut down the right engine, we were capable of continuing flying safely, that fire trucks would look over the aircraft upon landing and we would plan to taxi into the gate. Then sent a message to Dispatch the engine 2 failed, [priority handling] and landing at ZZZ. Runway XX was offered by ATC. Declined as we wanted the longer runway. Requested vectors for ILS XYR. Uneventful approach and landing. Stopped on the runway to be inspected by crash fire rescue equipment and all clear received. Passengers were informed that the fire department looked over the aircraft and we were safe to taxi to the gate. Taxied at own power to the gate with normal disembarkation.

Synopsis

A-320 air carrier crew reported an engine compressor stall during cruise flight prior to descent on a training flight. The crew shutdown the engine and landed at the destination.

ACN: 2253289 (123 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Relative Position.Distance.Nautical Miles : 30
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 8600
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 2700
ASRS Report Number.Accession Number : 2253289
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At flight ABCD ZZZ-ZZZ1 the take off was uneventful, Durian initial climb after takeoff we had an aircraft malfunction of avionic ventilation systems and we address the malfunction accordingly to manufacturers procedures, then at crossing FL310 we notices an abnormal cabin climb rate, we stoped the aircraft climb and requested descent, then the cabin altitude continued to climb out of limits resulting on a cabin excess altitude. Therefore we notify ATC and [requested assistance], started our emergency descent to stop it at 10,000ft MSL. We decide to go back to our departure airport (ZZZ), TO perform normal approach and landing and had the aircraft checked. The approach landing and taxi were uneventful landing at runway XX at XA:44 local time and arrived at the gate XA:49 gate XX of terminal X, all passengers, crew and airplane were ok. We didn't required any assistance from emergency responders.

Synopsis

A320 Captain reported during climb experiencing ventilation system malfunction resulting in excessive cabin pressure, rapid descent and return to the departure airport.

ACN: 2251650 (124 of 2908)

Time / Day

Date : 202506
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component : 1

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Cabin Lighting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2251650
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

When reaching FL350, received an ECAM "CAB PRESS (Cabin Pressure) SYS 2 FAULT". After performing the non normal procedures, I contacted Maintenance via SATCOM. Failure continued, and was written in the logbook. This airplane had this same issue a day or 2 before. about 30' after, still at FL350, Received an ECAM WARNING- EXCESSIVE CABIN ALTITUDE. Both pilots had immediately the O2 Masks on, I was PF, and continued to be PF, [requested priority handling] with ZZZ Center, Told them we were depressurizing, and descending to 10,000'. With the airplane and the situation under control, and about 1 minute into the descend, the ECAM Warning disappeared, and the lower ECAM was showing again the CAB PRESS page, SYS 1 working, and amber XX for SYS 2. Cabin Pressure was regained and I requested to stop the descend at FL250. We don't recall how high the cabin pressure went, but the O2 masks for the cabin were not deployed. Note, that a few seconds into the situation, the number 1 FA (Flight Attendant), Person A, called us, to inform about some anomaly with lights in the cabin. We told him to standby, and give us some time to talk to him. At FL 250, after checking everything, and considering back to a normal condition, and being about half way between ZZZ and ZZZ1, I took my O2 mask off, and requested the FO to stay with the mask on some more time., I cancelled the [priority handling] with ZZZ Center, and requested to climb to FL 330 to have enough fuel to continue to ZZZ1. I called the number 1 FA, to retrieve his info. They had 2 chimes, all cabin lights went bright, then 2 more chimes, then, all Lights flickering for a while, then stayed bright. The FA lost control of the lights on the FA panel. Flight landed in ZZZ1 safely with no more issues.

Synopsis

A320 Captain reported cabin pressurization and cabin lighting malfunctions during cruise flight. The pressurization problem resolved and the flight continued to the destination.

ACN: 2250655 (125 of 2908)

Time / Day

Date : 202506

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2250655
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2250654
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon reaching cruise, we had the following ECAMs: BRAKE SYS 1 FAULT Y RSVR LO LVL Y ENG 2 PUMP LO PR Y SYS LO PO F/CTL SPLR FAULT Approximately 201 miles away we assessed the situation at hand with the above failures. We also had a SPLR FAULT which increased our fuel consumption. There was a possible pipe rupture and it still being daylight along with ZZZ being our base and the unknown of the fuel irregularities led us back to ZZZ.

Narrative: 2

Y hydraulic system failed on climb out and induced cascading system failures. We elected to return to ZZZ. Followed company procedures.

Synopsis

A320 flight crew reported returning to departure airport after experiencing loss of Y hydraulic system.

ACN: 2248120 (126 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Rudder
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 87.58
Experience.Flight Crew.Type : 244.65
ASRS Report Number.Accession Number : 2248120

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 133.47
Experience.Flight Crew.Type : 2337.15
ASRS Report Number.Accession Number : 2248111

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of the ZZZ through 17,000 ft, the aircraft started to shake and vibrate on the yaw axis. The shaking and vibration were strong, fast, and rhythmic; we requested a level-off at FL230. Slowing the aircraft from approximately 300 knots to 280 knots showed no change to the shaking/vibration, nor did the level off. At that time, we [requested priority handling] and requested to divert to ZZZ. ATC issued a turn direct to ZZZ. Dispatch was notified of our [request for priority handling] and new destination via ACARS. The shaking/vibration slowed and smoothed out in the descending turn at 250 knots. A standard ARFF (Airport Rescue and Firefighting) response upon arrival at the airport for landing as we touched down and taxied to the gate uneventfully, requesting no assistance. Using the FM QRH airframe vibration definitions, which categorize vibration severity, we would classify it as strong vibrations.

Narrative: 2

In the climb as we passed approximately 18,000 and 300kts the aircraft started to shake laterally. It felt as if the rudder was unstable and shaking. There was no movement in the rudder pedals. We slowed down to 280kts and it continued to shake. We [requested priority handling] and returned to ZZZ. Did a straight in landing and taxied back to parking.

Synopsis

A320 flight crew reported experiencing "strong" airframe vibrations during climb which resulted in a return to the departure airport.

ACN: 2246819 (127 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : ILS XXR
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2246819
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Approximately 1 hour before arrival into ZZZ, we got a ECAM .. HYD G ENG 1 PUMP LO PR..G SYS LO PR. The first officer and I ran, per SOP, ECAM actions and QRH. We realized there would be a potential for numerous systems associated with the G HYD system that may be inoperative. No Normal braking, no auto brakes... to name a few. I contacted dispatch and maintenance control. After concurring with Maintenance Control that we had done everything possible and dispatch verified the QRH procedure and calculating landing distance required with loss of braking and single reverser. We proceeded to notify the cabin crew, ATC and customers. I had the purser come into the cockpit to brief him on the situation and told him it would be a moderate level situation, which could escalate to an urgent situation. We then continued the arrival and [requested priority handling] with ZZZ approach and were given runway XXR ILS. I then made a PA to the customers advising them that we had lost one of our hydraulic systems, but to be rest assured we have 2 other systems. I did tell them out of abundance of precaution we [requested priority] and that CFAR (Crash Fire and Rescue) will meet us upon landing. I advised them them to follow all flight attendant instructions and to give them their undivided attention. We turned on the Hyd engine and PTU (Power Transfer Unit) on to put the gear down with the remaining fluid,we did this on base leg, in case it failed to come down, we could do a manual extension. Gear came down and locked. We turned the pumps back off and turned them on just before touchdown to use it for braking and steering. After landing XXR we were able to taxi off the runway, where we started the APU and shut down the engines and had the aircraft towed the remainder of the way to gate XX.

Synopsis

A320 Captain reported a loss of a hydraulic system during cruise. The flight continued to the destination and landed but required a tow from the taxiway to the gate.

ACN: 2245151 (128 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Door
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 69
Experience.Flight Crew.Last 90 Days : 136.53
Experience.Flight Crew.Type : 696.55
ASRS Report Number.Accession Number : 2245151
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2246004
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.No Specific Anomaly Occurred : Unwanted Situation
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Operating Flight ABC ZZZ-ZZZ1, we were placed on a gate hold for connecting passengers. A few minutes later, the gate agent came to the cockpit and informed us that everyone is onboard, closed the door and removed the jet-bridge approximately 10 minutes prior the gate hold time. Via the app we received a message that there were 3 passengers still in the gate area. We believed the message was from another gate agent informing the app that the 3 passengers were part of the gate hold so we contacted operations to confirm. While coordinating with operations, the gate agent appeared in the jet-bridge and began moving it back to the aircraft. We immediately checked the status of the doors and determined the doors were stilled armed. We contacted the lead flight attendant to inform them the jet-bridge is being connected to the aircraft and that the doors need to be disarmed. We returned to the discussion with operation and moments later were informed that the main cabin door had been opened while still armed and the emergency slide had partially deployed. We immediately assessed if anyone was injured and determined there were no injuries. We then contacted maintenance to inform them of the situation and they sent a team of technicians to secure the slide from further deploying and fully opened the door. Maintenance told us that the aircraft needed to be swapped and we contacted dispatch and the Chief Pilot to inform them of the situation. Once we were assured that our crew was safe and passengers were being accommodated, we were able to determine that the message in the app actually came from a flight attendant that was not assigned to the flight.

Narrative: 2

We were operating Flight ABC ZZZ-ZZZ1. Just prior to pushback after the jet bridge was already removed from the airplane, we were informed of a customer service issue from a message in the app, that would require the jet bridge to be reattached to resolve. While I was coordinating with operations on the radio, we noticed the jet bridge was in motion towards the airplane. The captain quickly called the flight attendants and advised them that we would be reopening the door and to disarm all doors. Moments later, we were advised by a flight attendant that door 1L was not disarmed when it was opened, and the emergency slide had partially deployed. We immediately assessed the situation for injuries, and determined there were none. As the captain was dealing with the situation in the cabin, I called maintenance on the radio to alert them of the situation. Maintenance advised the airplane would be grounded and that they were working to find us a new airplane. A manager came to talk with us, and we informed her that we needed to deplane. As she initiated that process, we called the Chief Pilot and alerted them of the situation as well. Once we were assured that our crew and passengers were safe and being accommodated, we noticed the message in the app that initiated the customer service issue was from a flight attendant sitting in the terminal who was not assigned to the flight and had no operational duties with the flight.

Synopsis

A320 flight crew reported that a gate agent opened the armed aircraft door and partially deployed the escape slide. Maintenance was called and aircraft had to be swapped.

ACN: 2245072 (129 of 2908)

Time / Day

Date : 202505

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Door
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2389.80
Experience.Flight Crew.Last 90 Days : 207.85
Experience.Flight Crew.Type : 2389.80
ASRS Report Number.Accession Number : 2245072
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X to ZZZ [Airport] following a gate return for maintenance and an aircraft swap. When the First Officer and I arrived at the new gate, boarding had already began. For this second time doing the same flight things were flowing like normal and not too rushed. The gate agent asked if she could close the door, I asked all crew if they were ready and confirmed with the gate agent there were no passengers standing by. We closed the door and shortly after the First Officer and I began the pre flight checklist with all doors armed and I also heard the Flight Attendants begin their safety briefing. Then without warning we simultaneously received a call from the Flight Attendant and an annunciation that the main boarding door was open. Our purser said that she waived at the gate agent through the port hole when she saw her but she simply went straight to the door and opened it with no notification or knock. The door was armed. When we asked why she opened the door she said she wanted to get a few more passengers on board....no crew had been notified of this. Fortunately the Airbus slide won't deploy in this situation but I believe this is still a safety concern and should not be done in this way. The gate manager came down and told me the gate agent was new and didn't know she couldn't just open the door....to which I wondered why she was allowed down to the gate alone in the first place....not to mention she was probably overwhelmed as I noticed she was also working our flight alone. I did debrief [with] both the manager and the gate agent about my safety concerns for them and the aircraft, and that they should follow normal arrival procedures at a minimum, but really needed to also make certain the crew is aware of and ok with the boarding door being opened before just simply doing it. For the employee involved I'm in support of some grace here, but strongly recommend extra training.

Synopsis

A320 Captain reported that a gate agent opened an armed aircraft door without notification to allow additional passengers to board. Escape slide did not deploy.

ACN: 2243506 (130 of 2908)

Time / Day

Date : 202505

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Windshear

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2243506

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2243507

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

During a TOGA takeoff on runway XXR at ZZZ airport, we received an ECAM warning for Engine 2 High Vibration accompanied by noticeable shuddering. The TOGA takeoff was planned due to reported low-level wind shear (LLWS) at the time of departure. The ECAM alert appeared at approximately 80 knots, and we rejected the takeoff at an estimated 90 knots. We brought the aircraft to a complete stop on the runway, assessed the situation, and made a PA announcement to the passengers. The aircraft then exited the runway under its own power. We coordinated with Dispatch and Maintenance Control for a return to the gate.

Narrative: 2

Taking off XXR in ZZZ winds were gusty out of the NW with LLWS reported in the area, we elected for a TOGA takeoff. At around 80 KTS we received HIGH ENG 2 VIB ECAM and a brief shuddering, CA aborted the takeoff at approx 80-90 kts. The aircraft came to a stop abruptly but safely, at a stop we requested no assistance with tower and taxied off the runway under our own power after assessing the situation. We contacted dispatch and they advised us to return to gate in which we did with no further incident.

Synopsis

A-320 air carrier crew reported an engine malfunction during the takeoff roll. The Captain rejected the takeoff and returned to the gate.

ACN: 2243504 (131 of 2908)

Time / Day

Date : 202505

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2243504
Human Factors : Time Pressure

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2243505
Human Factors : Time Pressure

Events

Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Around 120knot a small bird flew in front of us from right to left and apparently got ingested into the #1 engine. We heard an extremely loud bang so I believe it caused a compressor stall. After the abort a pax (Passenger) told our FA (Flight Attendant) that they saw fire blow out of the engine after the sound. There was also a burning smell. We were then towed in back to our gate. Nothing really to be done. The procedures worked great.

Narrative: 2

Around 120 Kts a small bird flew in front of the aircraft and apparently got ingested into the #1 Engine. We heard a loud bang so it could have caused a compressor stall. After the abort a pax (Passenger) told a FA (Flight Attendant) that they saw fire coming out of the engine right after the sound. There was also a burning smell. We were towed back to the gate. Procedures worked accordingly

Synopsis

Air carrier flight crew reported a bird strike/engine ingestion during takeoff roll. The flight aborted safely and returned to the gate.

ACN: 2243465 (132 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2243465
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were cleared to push [gate] XX. While pushing back it appeared as though we were being turned at a highly unusual angle. Our tug driver turned us more and accelerated on a path directly into the gate XY aircraft. At this time people started running out from nearby gates screaming and waving to stop. Wing walkers saying to stop. We were now on path to hit the aircraft and had several people standing right in front of our number two engine that was running trying to wave at the tug. They realized this and moved away form the engine. Both the Captain and I started yelling stop stop stop stop while all this was going on. We came to a stop maybe five feet from the other aircraft. The tug driver still didn't understand why we told him to stop. After being stopped and everyone calmed down. We asked if the tug was driver was okay and if he understood. He did not. Operations, Ramp, and supervisors, and Dispatch were all notified of the incident. Ramp pulled the tapes and was astonished by this event and how close this was to two planes colliding and one or more people endangered of being sucked into an engine. Cause: The tug driver is/was behaving dangerously and recklessly

Synopsis

A320 First Officer reported a near collision with another aircraft during pushback while connected to the tug. The pilot crew alerted the tug operator, along with the ramp agents to stop the aircraft and avoid the collision.

ACN: 2239653 (133 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2239653
Human Factors : Time Pressure
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During takeoff and Thrust at Flex setting speeding up to 50 knots and non flying FO pilot calling airspeed alive we received an ECAM for Engine 1 COMPRESSOR VANE. I slightly adjusted Thrust levers to see if message would disappear since no engine indications of EPR, EGT, N2 or any sounds were abnormal that we could tell. Message persisted and I decided to reject the takeoff at about 50 knots on the runway. I stopped the aircraft on the runway while FO called the Tower that we were rejecting the takeoff. I applied the Parking Brake and announced CABIN CREW AT STATIONS on the PA. NO QRC actions were required and FO proceeded with ECAM Actions that resulted in shutting down Engine Number 1 on the ground. After ECAM was completed, I announced on the PA to crew members to STAY SEATED and also quickly explain to the customers what happened and that we were taxing back to gate for Maintenance and change of aircraft. We referenced the QRH and then contacted Maintenance control through ACARS and also informed the Station that we were coming back to the gate. Maintenance logbook was written up with the discrepancy. In addition, a commuting Captain in the back in [seat] XX heard some abnormal noises from engine 1 during the takeoff.

Synopsis

A320 Captain reported receiving an ECAM for Engine 1 Compressor Vane during takeoff roll. Flight crew rejected takeoff and returned to the gate.

ACN: 2239570 (134 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2239570
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We received a HYD Y RSVR LO LVL ECAM. We ran the ECAM and QRH procedures. We advised ATC. I attempted to contact dispatch, but was unsuccessful. We decided to divert to ZZZ. I notified dispatch thru ACARS and received an amended flight plan. After landing we stopped on the runway for ARFF (Airport Rescue and Firefighting) to inspect the aircraft. After receiving an all clear, we proceeded to the gate. Cause: Company communication.

Synopsis

A320 pilot reported a hydraulic system ECAM during cruise. Flight diverted and landed.

ACN: 2239479 (135 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2181.57
Experience.Flight Crew.Last 90 Days : 136.53
Experience.Flight Crew.Type : 2181.57
ASRS Report Number.Accession Number : 2239479
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 144.75
Experience.Flight Crew.Type : 276.37
ASRS Report Number.Accession Number : 2239489
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overrode Automation
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through approximately 5000 ft with speed managed and auto thrust on, airspeed continued to increase past 250 kts to 320 kts. Speed did not respond to 250 selected on FCU (Flight Control Unit) either. Initially, I suspected an unreliable airspeed, accomplished memory items, and the unreliable airspeed checklist. Conditions were IMC and with ZZZ departure. We inquired our speed with them. They said 360 kts. I was cautious to manually slow speed with auto thrust off because I didn't trust the airspeed indication. CA, FO, and standby airspeed indicator all had same indication so I now wasn't for sure the problem was unreliable airspeed. There were no ECAMS. I turned AP1 back on to test the autopilot. The autopilot did not respond to selected altitude or pitch commands. So I disconnected AP1 and flew raw data the rest of the way. I did not attempt AP2 at that point. [ATC was advised] and we were vectored for visual Rwy XX in ZZZ. I requested my FO to brief the FA,s and make a PA to the passengers since I had my hands full flying the airplane on raw data. We elected to land overweight by 4000 lbs at 146,000. We stopped on the runway while emergency vehicles confirmed there were no smoking brakes or anything else appearing abnormal from overweight landing. I asked my FO to announce â€œremain seated, remain seatedâ€ on PA to prevent any inadvertent evacuation. Once stopped and communication established with emergency vehicles, I made a PA to passengers that they would see fire trucks as a precaution and that I expected to be taxiing to gate shortly. Afterward we continued to the gate uneventful. A total of 5 writeups were submitted to maintenance to include overweight landing, landing lights extended at 320 kts, and no ECAMS associated with emergency. A brief with local maintenance, ZZZ flight ops, and Chief Pilot were subsequently accomplished.

Narrative: 2

Departed ZZZ runway XYL on the ZZZZZ [SID], initially in heading mode then direct ZZZZZ. I was PM for this leg. We engaged NAV and made the left turn east, climbing to 4,000 assigned. We were then cleared to climb via SID and I selected 16,000 in the FCU (Flight Control Unit). CA asked for managed speed at ~2,600 ft. I noticed we were above the SID altitudes at this point inadvertently (5,000') and then CA noticed the airspeed was running away well above 300 knots at this point with the auto thrust engaged. CA disconnected autopilot (AP), auto thrust (AT) and flight directors at this point and we executed the memory items for unreliable airspeed. The AT was not commanding the correct speed and we accelerated to 320 knots indicated airspeed at 5,000 feet. I called ATC and told them we were correcting and they noticed we were doing around 360 knots ground speed at this point. We decided it was not unreliable airspeed given all 3 airspeeds matched, but the AP and AT appeared to not be functioning properly. We [advised ATC] at this point, since there was no ECAM and we couldn't trust the automation to keep the plane safe and level. We received vectors from ATC back for runway XX, CA was flying all raw data at this point. He tried to re-engage the AP and AT but it was still not responding appropriately. CA instructed me to do a brief with the flight attendants as well as making the PA to the passengers informing them of what was going on, since CA was deeply concentrating on hand flying the airplane in IMC. Eventually we were cleared for the visual approach to XX, and landed successfully and announced â€œRemain seated, remain seatedâ€ to mitigate an inadvertent evacuation. We did land at 146,000 pounds, roughly 4,000 pounds overweight. The ARFF (Airport Rescue and Firefighting) vehicles did an aircraft inspection on the runway. We taxied back to the gate thereafter.

Synopsis

A320 flight crew reported during climb the aircraft did not respond to the airspeed selected on the FCU. They disconnected the autopilot and returned to the departure airport.

ACN: 2239472 (136 of 2908)

Time / Day

Date : 202505

Place

Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Emergency Exit
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 119.57
Experience.Flight Crew.Type : 1503.00
ASRS Report Number.Accession Number : 2239472
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through FL240 we got an ECAM for Left after over wing exit open. We stopped the climb at FL 270. We check the cabin pressurization page and it indicated we had stable cabin pressure. I gave the FO the radios and sent a message to dispatch to call me. A few minutes later I was on a radio patch with dispatch and Maintenance Control. During the conversation we referenced the QRH and it wanted us to limit altitude to 10,000 feet. I informed dispatch we would divert to ZZZ. The FO requested a descent to 10,000 feet while this was happening. We never [requested priority handling] but I believe our dispatcher did based upon how ATC was handling us. The divert to ZZZ was uneventful and the fire trucks followed us to the hangar.

Synopsis

A320 pilot reported an ECAM for left aft over wing exit open during cruise climb. Flight diverted and landed safely.

ACN: 2239133 (137 of 2908)

Time / Day

Date : 202505

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2239133

Events

Anomaly.Ground Excursion : Taxiway
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Airport
Primary Problem : Airport

Narrative: 1

As we taxied northbound on taxiway 1 at ZZZ there was light rain falling and the taxiway appeared damp. We slowed to turn onto [taxiway] 2 to hold short of runway XXL, prior to turning the aircraft speed was approx 8-10kts. As I initiated the turn with the tiller the nosewheel began to slip and the aircraft continued to track forward towards the edge of the taxiway. I continued braking and believing the aircraft may exit the taxiway I deployed the TRs momentarily. The nosewheel regained traction and I brought the aircraft to a stop on the taxiway centerline short of the hold short line for XXL. As the aircraft was skidding there was a significant chatter from the nosewheel and multiple ecams popped up. As a result of this we contacted Maintenance Control through dispatch and decided to return to the gate. As we were taxiing back we informed tower of what occurred and requested they have airport ops inspect the area where the skid occurred as it didnâ€™t seem our speed or the damp conditions were significant enough to cause that kind of loss of traction. Airport ops reported back that the area had a build up of liquid de icing agent which the damp taxiway conditions must have obscured. The only other possible contributing factor was a somewhat light aircraft gross weight. Both the FO and myself recall the aircraft speed being around 10 knots or less. So I must believe the main factor was the de icing agent mixed with water and light aircraft weight. Subsequent to this occurrence the ZZZ airport authority closed the taxiway to clean it indicating they also believed this may have been the culprit.

Synopsis

A320 Captain reported the aircraft began skidding during taxi when it encountered de-icing fluid on the taxiway. Flight returned to gate.

ACN: 2238602 (138 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2238602
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

The CA made a detailed briefing on previous near mishaps into ZZZ due to the terrain risk on the RNAV for XX and the expectation that he wanted vectors outside of the IAF. He had me request vectors outside of ZZZZZ so we would be outside of that it IAF. The controller gave us the last vector that took us just inside ZZZZZ and the CA accepted. The direction was to intercept the final approach course and contact the tower. We were at conf 2 and 180 knots on the turn and at 3000 msl. The ca selected direct to the next fix on the nav page. He didnâ€™t realize that he selected ZZZZZ1. He said confirm and then pressed insert before I could review the direct to selection I was reading back the approach clearance and was confirming the tower frequency on the approach plate by looking on my iPad, so I was looking to my right. The ca set 2500 feet and V/S down and then engaged APP and said FINAL APP NAV. Flaps 2, 180 knots. The aircraft started a steep descent (to make the next fix it saw which was 1280 feet and not 2500). I was stating final app nav a few times to cue then ca and he didnâ€™t respond so started too say do you want me to set the missed approach alt and then the red warning light and audible went off and I looked to the ECAM and said the flaps over speed warning. The jet was essentially diving for the 1280 altitude and exceeded the 200 knot flap speed. I crossed compared our altitude to the approach plate and started that we are too low. I looked outside and confirmed we were too low. The Ca agreed and disconnected the AP and manually leveled us off. A few seconds later the tower issued us a low altitude alert. The ca stated that this is not right and unstable so letâ€™s just go around. I agreed. We didnâ€™t do a Togo go around as we were vfr and could see we were safe from terrain. We climbed back up to 3000 ft and vectored back around for a visual approach. I reloaded the approach and selected the direct to when we were on base and confirmed it with the ca. we both ensured the correct direct to selections were made on the mcdu on the second approach. Cause: We got seperated operationally due to responding to atc and making AP selections at the same time without proper verification.

Synopsis

Air carrier First Officer reported a CFTT event during final approach. The flight crew executed a missed approach and returned for a second approach.

ACN: 2238271 (139 of 2908)

Time / Day

Date : 202505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Unknown
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2238271
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After normal pushback and taxi, we observed no abnormalities prior to take-off. We accomplished all checklists and we taxied into position for take-off from Runway XX at ZZZ. Weather conditions at the time were winds 120/19 and VMC. I transferred the controls to the First Officer, and after given clearance to take-off, the First Officer advanced thrust. I observed normal engine and airspeed indications throughout the take-off roll. Passing 80 kts. all indications continued normal and the First Officer maintained directional control. Take-off data calculated V1 at 135 kts. Somewhere between 95-115 kts., the aircraft pulled uncommanded to the left. The aircraft quickly departed the centerline of the runway at a rate that made the takeoff unsafe to continue. Deviation from centerline was approximately 20 feet left. There were no abnormal engine indications or ECAMs. I (Captain) called â€˜Rejectâ€™ and took control of aircraft. I executed Rejected Take-off procedures, brought the aircraft to a safe stop, and advised the cabin to â€˜Remain Seated, Remain Seatedâ€™. After taxiing off the runway and setting the Parking Brake, I made appropriate calls to the cabin crew, a PA to the customers advising of our situation, and debriefed with the First Officer quickly on what happened and what he experienced from his perspective as Pilot-Flying at the time of the event. During this time we observed brake temperatures between 350-500 deg and Brake Fan was turned on. We then coordinated with ZZZ Operations to facilitate a gate return and to have Maintenance Control meet us. I documented the uncommanded left pulling and pertinent information into the Maintenance Logbook. I made an additional PA to the customers explaining of the abnormal experience and that Company was working on getting a new aircraft to continue our flight to ZZZ1.

Synopsis

A320 pilot reported uncommanded pull to the left during the takeoff roll. Flight crew rejected the takeoff.

ACN: 2237230 (140 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct

Component

Aircraft Component : Cabin Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2237230
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Electronic Device
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

While in flight from ZZZ1 to ZZZ, with approximately 15 minutes prior to landing, a passengerâ€™s self contained cell phone charger had a thermal runaway and caught fire. The inflight crew notified the flight crew of the situation, and the Captain instructed the inflight crew to extinguish the fire. The Captain requested priority handling with ATC and the flight was cleared to proceed directly to ZZZ. The inflight crew successfully extinguished the fire within seconds by containing the device in the garbage bin of the forward lavatory and then submerging the device with water. The device remained submerged with water in the garbage bin of the forward lavatory for the remainder of the flight. The inflight crew advised the Captain that the fire had been extinguished but the cabin had filled with smoke as a result of the fire. The flight crew performed the Smoke Removal procedure per the QRH. The smoke was only present in the cabin and it did not enter the cockpit at any point. The flight crew was successful in removing the smoke from the cabin. The flight crew received multiple LAV SMOKE ECAM warnings as a result of the smoldering device being placed in the forward lavatory. The Captain issued a briefing to the inflight crew and made a PA to the passengers. Dispatch was notified by ACARS of the situation. The Captain and inflight crew remained in constant communication throughout the remainder of the flight regarding the condition of the device. The flight landed on runway XX in ZZZ, taxied to gate XX and was meet by Crash Fire Rescue personnel who removed the garbage bin containing the submerged device from the aircraft. The passengers deplaned normally through the jet bridge. There were no reported injuries to any passengers or crew as a result of the fire. There was some charring on the seat frame in row 4 ABC and melted plastic on the aisle carpet where the fire took place. The aircraft maintenance log book updated with the appropriate maintenance discrepancies. ZZZ Technician Operations addressed the damage to the aircraft cabin and returned the aircraft into service.

Synopsis

A320 First Officer reported a cabin smoke event from a passenger cell phone charger prior to landing. The cabin crew contained the malfunctioning device in water, the plane landed safely with no injuries, some charring to a passenger seat frame, and cabin furnishings.

ACN: 2235023 (141 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Direct
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2235023
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through 12k, master caution and ecam indicating loss of green system hydraulic pressure. Hydraulic system page indicated complete loss of green sys fluid. Completed ecam actions and qrh. Advised dispatch, inflight and customers that we would return to ZZZ. Requested priority handling and advised inflight. Manual gear extension and over weight landing with no issues. Emergency equipment followed us to gate.

Synopsis

A320 pilot reported a hydraulic main system failure during climb resulting in a return to the departure airport.

ACN: 2234247 (142 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 2234247
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working flight ABCD ZZZ on [Gate] X as crew chief, before the loading process I check if there any DG plan for this flight ABCD to ZZZ, but I did not see any listed for this flight. During the loading process I scan two boxes containing Dry ice with the label un3373 and the class 9 division label on it in the scanner I press the important package option for hazmat. I notified the captain about the DG is not planned for this flight and he told me that he was going to check on it. I double checked with him, and he said that he is still checking on it. I was waiting for the captain to see if everything is good to go or I had to remove the boxes. All this conversation happened to the gate before he was authorized by the tower to push the aircraft. While I was waiting to find out from the captain if I was needed to remove the boxes from the aircraft prior to be push from the gate I thought that the captain received the ok from dispatch. aircraft did not take off from ZZZ1 it return to another gate to remove danger good from aircraft In futures Dry Ice DG they should eliminate UN 3373 because that create confusion if DG notification is required or not.

Synopsis

Air carrier Maintenance personnel reported undeclared dry ice being loaded. Dry ice was removed prior to departure.

ACN: 2234061 (143 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 50

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2234061

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After an uneventful flight, we were landing with a gusting to 15 knot crosswind on XXC. When I put in my de-crab crosswind controls in the flare around 50 feet, my rudder pedals broke/unlocked and I lost all lateral control authority. This lack of control resulted in a slightly firmer than normal touchdown and I quickly transferred aircraft control to the Captain who was able to uneventfully and without issue bring the aircraft to a stop and to the gate. Upon inspection, I found that the FOâ€™s rudder pedals moved forward/aft with no resistance and it appeared some aspect of the spring adjuster had broken or become disconnected. We gave the aircraft to maintenance and wrote up the issue. A following write up included that the FOâ€™s brake reservoir had become completely depleted. The aircraft experienced a serious control malfunction with the FOâ€™s controls, which in a different phase of flight or with slower pilot action to react and transfer aircraft control could have had potentially disastrous results. I wanted to highlight it to safety via this report in case this becomes a trend, because of the severity of the malfunction. Should this occur on both sets of rudder pedals simultaneously, complete loss of control on the ground would result.

Synopsis

A320-200 pilot reported during the landing flare around 50 feet they lost all lateral control authority and upon touchdown they transferred aircraft control to the captain who was able to stop the aircraft and taxi to the gate.

ACN: 2233787 (144 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2233787
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

ZZZ1-ZZZ flight ABC into ZZZ. Had an incident while performing post flight walkaround where I was almost hit by luggage from above coming out of FWD cargo bin. As I completed my walk around and was about to head back up the jetway stairs, the Ramp Agent who was in the cargo bin approached me to apologize. He stated â€œthe reason the bags were coming off the aircraft like that is because there is an oil spill or leak in the cargo bin causing the bin floor to be extremely â€œslipperyâ€ I asked him to confirm it was oil, to which he replied â€œYesâ€. I went up the stairs to discuss the issue with the F/O. We agreed it needed to be written up and the baggage loading needed to stop for the time being. I informed Supervisor to inform Ramp Agent to not load bags (based on the fact that they had just got done unloading I assumed they had not started loading yet). Flightcrew further discussed if this was in fact a hazmat situation and determined that unknown oil very well may be. I seemed to remember a need to notify the airport authority of possible hazmat situation from our training program. I had Supervisor connect me with the local airport authority and they informed me they would send the fire dept over to inspect. There was a rather unproductive conversation with Maintenance Control on site as to whether or not to consider it may be hazmat and also whether or not the Airport authority had to be called. It was moot at this point as I had already written it up in the logbook and notified the Airport. Fire Dept cleared the aircraft, bag loading resumed, and logbook signed off. Flight departed, safely, and without concern there was a dangerous, unknown â€œoilâ€ substance onboard. As per FOM, Dispatch was also notified of the possibility of hazmat on the aircraft. They informed the On Duty Supervisor. They subsequently called me for more details, which I gave him. He recommended a report in addition the other report.

Synopsis

Air carrier Captain reported an oil leak or spill in the cargo bin during preflight. Ground personnel along with the Fire Department cleared the spill and the flight departed.

ACN: 2233747 (145 of 2908)

Time / Day

Date : 202504

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ground : ZZZ
Make Model Name : PA-28 Cherokee/Archer/Dakota/Pillan/Warrior
Crew Size.Number Of Crew : 1

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2233747
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2233750
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While working a flight ZZZ-ZZZ1, at about, XA25 on Day 0, we were stopped short of taxiway 1 on taxiway 2, First Officer informed me there was a Piper Cherokee aircraft abeam the FO side window. My line of sight was blocked. I had to lean over towards the FO side window to take a look. I saw Aircraft Y abeam the FO side window. Not being able to see Aircraft Y from to CAâ€™s seat means they were very close. If First Officer had not said anything to me I would not have known Aircraft Y was abeam us. If I were to have released the parking brake to move up to the Taxiway 1 hold short line, without the First Officerâ€™s keen observation, we could have potentially struck that aircraft with the right hand engine. Aircraft Aircraft Y was not there when we taxied westbound on taxiway 2 from taxiway 3. There were only three aircraft in sight when making the left turn onto Taxiway 2. A Cessna 172 on the ramp in the non-movement area south of Taxiway 2 near Taxiway 1, and two aircraft holding short of runway XXL on Taxiway 1. Those two aircraft were: a Piper Seminole and a Piper Cherokee (not Aircraft Y). The only possible way Aircraft Y could have gotten their position was to taxi under our wing. I queried ZZZ Ground Control if Aircraft Y had taxied under our wing. The pilot of Aircraft Y broadcasted his admission that he that had taxied under our wing to get to that position. Disregarding safety, the pilot of Aircraft Y endangered the well-being of the Crew, Passengers, and another aircraft. The First Officer had mentioned to me that they were holding their cell phones up possibly taking pictures and laughing (again that how close the were) raising an eyebrow to potentially suspicious activity.

Narrative: 2

While holding short of Taxiway 1 intersection for takeoff on runway XXL, behind 2 aircraft, we noticed another aircraft very close, too close to our right side near the engine, it just appeared and as I looked in awe, I noticed 3 occupants, laughing, waving, cameras out taking video/pictures of us in the flight deck. This was extremely suspicious and quite possibly a test or rehearsal of sorts. Upon immediately notifying my captain, we quarried ground control, ground control called the aircraft Aircraft Y, questioning why they cut in front of us, the aircraft admitted they maneuvered under our wing to get ahead us. In turn, they were told to contact the tower via phone, after they return, ultimately they were told to taxi back to the ramp, which cleared the way for us to take off in sequence. This was a first for me, it raises concern given todayâ€™s climate regarding the â€œsee something, say somethingâ€ philosophy and the current state of world affairs. I instantly thought that it wouldnâ€™t be too far fetched to believe this could possibly be a test of reaction or even a rehearsal for something worse in the future. Does the FBO have security searching the students and staff before entering the flight line? We have to go through a security checkpoint to access our aircraft. I am concerned that this is something more than a simple mistake on the FBO folks. Please let me know if further elaboration is required.

Synopsis

Air Carrier Flight Crew reported while taxing out for departure and waiting at an intersection when they noticed an aircraft that taxied too close to their aircraft resulting in them not being able to move to avoid a collision with the other aircraft. ATC moved the other aircraft and they departed the airport without further incident.

ACN: 2233587 (146 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1170.23
Experience.Flight Crew.Last 90 Days : 146.43
Experience.Flight Crew.Type : 654.17
ASRS Report Number.Accession Number : 2233587

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise flight at FL360, we received an ECAM for HYD ENG (Engine) 2 PUMP LO PR (Pressure). We followed the ECAM which instructed us to turn off the PTU (Power Transfer Unit) (fault light was illuminated) and ENG 2 PUMP OFF (fault light illuminated). The HYD page showed a cautionary line and PTU line, the fluid level was at the top of the cautionary box. The CA designated our duties; I was to continue PF and ATC and the CA to contact Dispatch and contact the flight attendants and passengers. We initially thought of ZZZ1 as our divert since it was just to our south. Dispatch suggested ZZZ due to ZZZ1 having bad weather. We agreed to advise ATC and divert to ZZZ. The CA informed the flight attendants and made a PA to the passengers. We then continued on the non normal checklist to determine the required landing distance. We were interrupted with another call from Dispatch. The CA called and the Chief Pilot was with him and a guy from the company. There was static on the frequency and the CA was having a hard time understanding them. I turned on and tried to listen. I heard pieces of their conversation. I was thinking why is this being done now when we already have advised ATC, made a decision to go to ZZZ and we need to finish the briefing, verify the landing distance chart etc. I received direct to ZZZZZ on the ZZZZZ1 arrival. We briefed the approach and decided to configure early and get the landing gear down early to see if there might be any more INOP systems that came up. Normal visual approach, touchdown and roll out. We had two reversers, brakes and steering. ATC asked if we needed any additional assistance, to which I replied no. ARFF (Airport Rescue and Firefighting) trucks followed us to the gate. After shutdown and parking checklist, we noticed the landing gear doors were down. MX (Maintenance) came up and notified us HYD fluid was continuing to leak out, the landing gear doors were open. After passengers deplaned, MX continued to troubleshoot. He notified us that there was no fluid left, the ENG pump was cavitated and the electric pump was split in half. The CA continued to debrief the flight attendants, me and the passengers at the gate area. The CA did good with multiple communications and keeping us all up to date. I felt I supported him fully and we had a very successful and safe landing.

Synopsis

A320 flight crew reported failure of hydraulic pump in cruise.

ACN: 2232648 (147 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2232648

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the landing flare, the First Officer's rudder pedals became unlocked, resulting in them sliding forward. First Officer quickly transferred the controls to me and I completed the landing. The transition did result in a firm landing. The pedals remained unlocked. Cause: Aircraft malfunction.

Synopsis

A320 Captain reported the First Officer's rudder pedals became unlocked during landing flare. The controls were immediately transferred to the Captain, yet the incident still resulted in a firm landing.

ACN: 2232557 (148 of 2908)

Time / Day

Date : 202504

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1475.13
Experience.Flight Crew.Last 90 Days : 137.97
Experience.Flight Crew.Type : 1475.13
ASRS Report Number.Accession Number : 2232557

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb, the aircraft developed a moderate vibration. The vibration reduced and stopped as airspeed was reduced to a cost index of zero. The use of or the lack of an autopilot had no bearing on the vibration. Changing the pitch of the aircraft did not have any bearing on the vibration. The vibration was stopped by reducing indicated airspeed.

Synopsis

A320 flight crew reported engine vibration during climb. Flight crew returned to departure airport.

ACN: 2230450 (149 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0001-0600

Place

Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2230450
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While cruising at FL360, the First Officer and I received an ECAM alert for HYD YELLOW RESERVOIR LOW LEVEL. FO was the pilot flying, and I instructed him to continue flying while I referenced the QRC and QRH. I accomplished the procedures outlined in QRH XX-XX, then contacted Dispatch via Satcom to discuss the situation. Maintenance Control was brought into the conversation. Together, we assessed the status of the aircraft and discussed a plan of action to ensure a safe arrival. Maintenance recommended continuing to ZZZ, which we agreed to. I then contacted ATC and [requested priority handling], providing the nature of the problem, fuel remaining, and requested Runway XXR for landing. I briefed the flight attendants on the situation and instructed them to prepare. A PA was made to the passengers to inform them of the situation. The FO and I reviewed the checklists again, verified all necessary procedures were completed, and continued the flight to ZZZ. Weather was cavok. Upon landing on Runway XXR, we exited the runway via Taxiway 1 and stopped at the intersection of 1 and 2 to allow Airport Fire Rescue (AFR) to inspect the aircraft, with a focus on identifying any signs of hydraulic fluid leaks. AFR found no visible leaks, and we were cleared to continue taxi to the gate with AFR following. After arrival at the gate, we had the nosewheel chocked, shut down both engines, and then set the parking brake, in accordance with QRH procedures. No injuries or aircraft damage were noted, but a hydraulic leak was discovered during post-flight inspection by maintenance.

Synopsis

Air carrier Captain reported a Hydraulic System malfunction during cruise. Flight crew continued to the destination airport and landed safely.

ACN: 2230432 (150 of 2908)

Time / Day

Date : 202504

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2230432

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise at FL370, AIR PACK 1+2 OVHT and (or just) AIR PACK 1+2 FAULT ECAM message. PACK 1 was on deferral under MEL 21-XX-XXX. Additionally Configuration Deviation List (CDL) XX-XX was utilized for both pack outlet flaps. Pack outlet temperature was high but barely in amber and only intermediately amber. Passengers and Flight Attendants were seated while we coordinated with ATC for a descent. Once Cabin Confirmed secured and while maintaining view of Outlet temperature, we began descent and initiated the ECAM generated depressurization. We donned oxygen masks and performed descent in accordance with the descent profile respecting weather conditions. At some point ATC [provided priority handling to us] on our behalf as we relayed our system issue. We were cleared to 10,000 feet and then to 7000 feet. Cabin Altitude reached 12,000 feet as we descended. Cabin oxygen masks did not deploy. We successfully diverted to ZZZ, [terminating the priority handling services] once level at 7000 feet. We were able to turn on PACK 2 once the overheat condition ceased around 4000 feet. Cabin temperature had reached 82 degrees. This plane had just the right factors for the Fault. We were mostly full with two deferred Outlet flaps and a deferred PACK. Maybe we should revisit the CDL and reconsider the suitability for the deferrals we were operating under. Also, we need to consider the cabin air filtration routine maintenance. We might benefit from more aggressive filtration replacements and inspections.

Synopsis

Air carrier Captain reported an Air Pack Fault message during cruise and diverted to nearby airport.