ACN: 2230249 (151 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 73.7
Experience.Flight Crew.Type : 668.32
ASRS Report Number.Accession Number : 2230249

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 209.25
Experience.Flight Crew.Type : 3041.15
ASRS Report Number.Accession Number : 2230216

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

First Officer landed at ZZZZ runway XXL at XA00. He used full reverse and then braking. A normal aircraft transition occurred and I exited the runway on taxiway 1 holding short of runway XXR. No indications of any braking problems. FO completed after landing flows, we discussed keeping #2 engine running until we crossed parallel runway. After several departures/landings on XXR we were cleared to cross XXR behind a 727 and check in with Ground. Ground cleared us to gate XX via northbound on taxiway 2. The FO shut down #2 engine and I taxied north on 2 to abeam the gate. I made the left turn towards the gate with no braking issues noted. There seems to be a little dip in the apron and as the main gear crossed over it I noted our speed slowed to 1 knot of ground speed on the PFD (Primary Flight Display). I added a little power to keep aircraft moving, then back to idle. As I approached the stopping point I glanced at PFD and saw 2 knots, as the Ground Handler crossed the wands for us to stop I applied additional pressure to the brakes and the pedals seem to go limp and all the way forward with no additional braking. I told the FO to â€œcheck your brakesâ€ he quickly responded they were not working. I set the parking brake and the aircraft surprisingly came to a very smooth stop. Both sets of brake pedals were non responsive and were physically all the way forward. Ground crews placed chocks and gave the signal. It took a bit of time for them to get communications connected and clarify what happened. During the 5-8 seconds of the event happening we probably rolled forward of the planned stopping spot by ~12-15 feet. It then took about an additional 5 plus minutes to get a tractor hooked up and the aircraft pushed back to the correct position for the jet bridge and start deplaning. As soon as jet bridge was hooked up, Maintenance personnel were right onboard as were we writing up the Maintenance Request. He had photos of the leak that appears to have started about half way down the lead in line. We checked the hydraulic page and the green system quantity was very low.

Narrative: 2

After landing on XXL and taxiing across XXR to our gate XX, as we were marshalled in to stop, the Captain said â€No brakesâ€. I immediately jumped on mine and pushed them all the way to the floor. We probably rolled another 8 feet before we could get it stopped with the parking brake. Aircraft stopped smoother than expected with the parking brake. No one was injured thankfully! Shortly after the ramp reached out to us so we could get pushed back to our original line - this was uneventful. After the jet bridge was pulled up, a Maintenance guy came into the flight deck and told us that our right main gear had leaked hydraulic fluid everywhere. This clearly explained why our brakes gave out. Aircraft was taken out of service to fix.

Synopsis

A320 flight crew reported they had no brakes on taxi in to the gate, and were able to bring the aircraft to a stop using the parking brake. It was apparent after inspection that the green hydraulic system had lost its fluid.

ACN: 2229132 (152 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 126.58
Experience.Flight Crew.Type : 1451.92
ASRS Report Number.Accession Number : 2229132
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During push back from Gate XX at ZZZ the push crew brought the aircraft to a stop, but no command to set brakes was given. After about 10 seconds with no aircraft movement I sensed that the ground crew failed to have me set the brakes. I then queried them twice, could hear them talking in the background, but they didn't respond. I sensed a forward rolling motion and stepped on the brakes. As I did, the ground crew gave the, â€œset brakesâ€ command in an excited tone. I believe we rolled forward less than a foot. I queried the ground crew if any impact with equipment or personnel occurred. They said that no such event occurred. I debriefed the ground crew and they apologized.

Synopsis

An air carrier Captain reported that after pushing back, there was no communication to set brakes and the aircraft began to roll.

ACN: 2229031 (153 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2229031

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Operating flight ABCD from ZZZ to ZZZ1 with XX pax and X crew. On the runway (XXR) the radar was showing a full display of yellow and green precipitation on our departure flight path. We had a tilt of approximately 6 to 7 degrees up. There were no reports of severe turbulence from ATC or other aircraft, however, all previous departures were to the northeast. Once airborne and into the clouds we encountered light and moderate turbulence. I continued to hand fly the aircraft and the FO managed the FCU (Flight Control Unit) as we were given a turn to 150. The FO adjusted the radar appropriately as we climbed. The radar began to show small spots of red and magenta in a field of mostly yellow and some green. I hand flew the aircraft to avoid any red or magenta while in the turn to heading 150. Passing 4000 feet the turbulence increased to all moderate chop and turbulence. I noticed the airspeed increasing and adjusted pitch in an attempt of maintaining 250kts. The airspeed was approaching 260kts with vertical speed indicating up to 6000 fpm and an amber indicator. I said that we must be in an updraft to the FO. Shortly after, we encountered a mix of severe and moderate turbulence. I began to have difficulty focusing my eyes on the PFD (Primary Flight Display) and ND (Navigation Display) due to the turbulence so I engaged the autopilot and continued to monitor the PFD and ND as best I could. We briefly encountered turbulence that pushed us down into our seats followed by a feeling of lightness. The severe turbulence encountered was between 5000 and 9000 feet. I delayed the 10000 foot notification until we were out of the precipitation, approximately 12600 feet. Afterwords I called the cabin crew to ask if everyone was ok and advise that we were out of the precipitation and area of greatest turbulence. They advised me that all crew and pax (passengers) were unharmed but scared for a bit. After the encounter I notified our dispatcher and Maintenance Controller of the severe encounter. The reply from dispatch was â€œMaintenance Controller WANTS TO KNOW IF IT WAS SEVERE OR BAD MOD AND WHAT MAKES YOU THINK ITS SEVERE DUE TO THE LARGE INSPECTION REQUIRED. FROM RAâ€. A logbook entry was made for the severe turbulence encounter

Synopsis

An air carrier pilot reported temporary loss of control due to severe turbulence encounter.

ACN: 2228999 (154 of 2908)

Time / Day

Date : 202503

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Lubrication Oil
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Turbine Engine
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2228999
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arrival at Gate XX we were informed by our gate agent that our airplane was waiting on maintenance. When we boarded the airplane the PCA (Aircraft Preconditioned Air Unit) was connected but not running so I called Operations for ramp to come over and turn it on. The APU was running providing power, but not air. Already knowing the tail, I radioed local maintenance to see if we could run the bleed air and asked them if they had an ETA the logbook would be ready. I was told that they were waiting to complete the logbook, but there was no ETA when that would be. Not too long after that another mechanic arrived at the plane and I asked him what the issue was and were told that the previous week they had found metal chips in the engine oil and after a subsequent inspection that night they had recovered additional metal fragments from the aft sump, but the inspection procedure they had been operating under did not give any guidance about what to do in the event they found metal in the subsequent inspection. That was what they were waiting for from engineering. The mechanic also showed us a picture of what I believe he said was the first set of fragment they had recovered the previous week. The fragments were laid out on a grid to show their relative size. He said they believe they were from a bearing in the #1 engine based on the studies that had been done on them. After being informed that engineering had approved an additional inspection schedule I formally initiated the procedure I was questioning the safety of the aircraft. I had previously initiating the procedure with my original dispatcher, but at the request of her supervisor we all decided to wait for the engineerâ€™s decision before pulling that trigger. I spoke the Chief Pilot where we had a professional discussion about the situation but ultimately decided to wait until I had the logbook so I could review the history of the airplane as best I could. One question I did get from the Chief Pilot was â€œwould my decision to refuse the plane change if I hadnâ€™t been informed by the local mechanic about the actual condition of the engine.â€ I found this question to be disturbing for obvious reasons. Once the logbook arrived I review the last few pages to find that the plane had been operating on a deferral for the #1 EMCD (Electronic Master Chip Detector) and that deferral had been cleared. It looked as if that was the deferral that was used when the metal fragments were originally discovered. I phoned Maintenance Control to verify and to ask if, since the airplane had required inspections before each takeoff under the original MEL, would continue to be the case. After reviewing his information he said it would not. Moving forward under the new guidance from manual they werenâ€™t required to inspect it again for and additional 50-70 flight hours. This didnâ€™t appear to add-up since there were 2 known occasions in a matter of 1 week where metal fragments were found in the engine oil . The timeline looked like the #1 EMCD tripped and the corrective action was to defer the #1 EMCD. At some point shortly after metal fragments were recovered and sent in for analyses. Over the course of a few days recurring inspections of the #1 EMCD were conducted and signed-off on. Then on night of Day 0 additional metal fragments were found in the aft sump and the corrective action was to close out the #1 EMCD deferral and put in place a different inspection protocol that didnâ€™t have anyone look again for another 50-70 flight hours. This was not adding up since on multiple occasions metal fragments had been found. I again refused the aircraft as the safety of the aircraft was in doubt. As we concluded the conference call between myself, dispatch, and the Chief Pilot one of them told the other to just replace the crew as we ended the call. My observation of the events of this report is that the integrity of the schedule was the number one priority - to the complete exclusion of safety. As the pilot in command, my assessment of the aircraft maintenance history, MEL application, and discussions with maintenance technicians regarding the presence of metal shavings indicated by the chip detector was completely disregarded. I was unable to legally or ethically sign the dispatch release guaranteeing the safe operation of flight - and dispatchâ€™s answer was to find someone else who would. The fact that I was actually asked â€œif you didnâ€™t know what you know about the engine history, would that change your decisionâ€ is preposterous. In my XX years as an airline pilot, I have never refused an aircraft before. I do not take it lightly. The complete disregard for the checks and balances of safety is extraordinarily troubling.

Synopsis

A320 Captain reported refusing an aircraft because metal fragments were found in the engine oil.

ACN: 2228390 (155 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14743.92
Experience.Flight Crew.Last 90 Days : 173.65
Experience.Flight Crew.Type : 12625.32
ASRS Report Number.Accession Number : 2228390
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While deviating around weather at FL370 approximately 20nm south of ZZZZZ, we received an ECAM alerting us to low yellow system hydraulic fluid quantity, followed by a HYD Y ENG 2 PUMP LO PR ECAM. We checked the Hydraulic System Display and noted that our Yellow hydraulic fluid quantity was nearly depleted. We followed all ECAM procedures, and then I contacted Dispatch to notify them or our situation. After considering our landing weight, the weather at ZZZ, and calculating our landing distance as directed by the ECAM, the Dispatcher and I determined that it was prudent to continue to our destination (ZZZ). We agreed that I would [request priority handling] once we were handed off to ZZZ Approach Control to allow us sufficient flexibility to vary our speeds and configuration as necessary to ensure a stable approach. We landed uneventfully and taxied to the gate.

Synopsis

A320 flight crew reported a malfunction with the hydraulic system while in cruise. Flight crew landed at destination uneventfully.

ACN: 2228102 (156 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 68.18
Experience.Flight Crew.Type : 68.18
ASRS Report Number.Accession Number : 2228102
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

After arriving at the gate and performing the parking checklist, the FO and I noticed that the slide indicator at door 2L was amber. Just as we observed this, we received a call from the flight attendant stating that the door handle had been accidentally opened before the door was disarmed. The flight attendant was holding the handle about halfway to prevent the slide from deploying. I instructed the flight attendant to continue holding the door handle while we contacted station operations, local maintenance, and Maintenance Control. The FO also coordinated with ground personnel to ensure the area remained clear in case of an unintended slide deployment. Once the aircraft was deplaned, maintenance arrived to assess the situation. They determined that the nitrogen tank had been punctured, but the slide had not deployed. Maintenance personnel took over holding the door handle to deplete the nitrogen tank, allowing them to safely open the door without triggering the slide. After resolving the issue, maintenance informed us that replacement parts would be needed. The situation was managed without further incident. We ensured that a maintenance write-up was completed. Additionally, we coordinated with the Chief Pilot to establish communication with the cabin crew manager and Maintenance Control.

Synopsis

A320 Captain reported an armed aircraft door was opened after arrival at gate. Maintenance was called and determined the nitrogen tank was punctured but escape slide was not deployed.

ACN: 2227855 (157 of 2908)

Time / Day

Date : 202504
Local Time Of Day : 0601-1200

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Syst Pressure/Temp Indication
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2227855
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Reaching cruise altitude the ECAM message and procedure HYD G RESERVOIR OVHT (Overhead) displayed on the ECAM. Accomplished ECAM procedure which resulted in G hydraulic system depressurized. The procedure allowed for an attempt to repressurize system G for approach if the reservoir OVHT was no longer occurring. G system pressure and quantity were normal prior to the overheat. All other systems were normal. Had a conference call with Dispatch and Maintenance Control in which it was decided to land short of ZZZ1 at ZZZ. We were above max landing weight and the time enroute to ZZZ would have us land lighter and below max landing weight. If we were unable to recover G system pressure for approach it would require a manual gear extension, no nose wheel steering, one reverser and no normal braking. Under these circumstances I wanted a long runway into the wind with the ability to get towed after landing. ZZZ was a good selection to meet our needs and keep the safety level as high as possible. During approach the reservoir temperature was back to normal and we were able to pressurize the G hydraulic system. The overheat did not reoccur during the approach, landing and taxi to the gate. Cause: Diverted due to loss of use of G hydraulic pressure as a result of ECAM procedure for reservoir overheat.

Synopsis

A320 pilot reported a hydraulic system overheat malfunction after climbing to cruise altitude. The flight diverted to an alternate airport and the hydraulic system returned to normal temperature and the aircraft landed safely.

ACN: 2226253 (158 of 2908)

Time / Day

Date : 202503

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2226253
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon passing through FL300 the aircraft generated an ECAM â€œCAB PR SAFETY VALVE OPEN.â€ The Captain immediately leveled off the aircraft and had me notify ATC of the need to level off for a pressurization issue. Upon working through the ECAM it was discovered that the differential pressure was not over 8 psi and only if above 8 psi ECAM actions were to be followed. The ECAM was cleared even though the safety valve was clearly open on the pressurization page. The Captain requested a descent and we were given FL280. We continued working on the issue and turned to the manual, which suggested resolutions only if differential pressure was above 8 psi. The safety valve at this point had closed. We considered the ECAM actions to be complete and decided to slowly climb back up at 500 fpm. It wasnâ€™t soon after we initiated that second climb that the same ECAM was once again triggered. The Captain this time requested a descent to FL260 and we would eventually make it down to FL200 before a request to return to ZZZ was initiated. Cabin pressure during this time seemed stable and was not climbing. However, the aircraft had an obvious issue with maintaining delta pressurization levels, which was resulting in the safety outflow valve being opened. Upon returning, the aircraft managed the lowering of delta pressure absolute cabin pressure perfectly fine and everything went smoothly. No emergency was declared for this event. It should be noted: This aircraft had been written up every day for the last 7 days for the same issue but it was returned to service each and every time! This is beyond the need for a safety flight with this aircraft, especially when dealing with cabin pressure related events. This has also occurred with other Airbus in our fleet, leading me to question whether an airworthiness directive should be forthcoming?

Synopsis

A320 First Officer reported there was a malfunction with the aircraftâ€™s pressurization control system and the flight crew safely performed an air turnback. The reporter also noted that this had been a reoccurring issue with this aircraft, and was also present in other Airbus aircraft.

ACN: 2225962 (159 of 2908)

Time / Day

Date : 202503

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : FBO
Make Model Name : PA-44 Seminole/Turbo Seminole
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Mission : Training
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2225962
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After contacting ZZZ Tower while on taxiway 1 moving towards taxiway 2 we were instructed to taxi via 2 and to hold short of Runway XXR. While holding short of XXR on 2, a Piper Seminole called ready for departure in the hold box. Tower advised the aircraft to hold short of 2 in the hold box. Despite this instruction the Seminole proceeded to taxi out of the hold box toward our left wing and engine, crossing over the hold short line of the hold box. I made eye contact with the pilots and gave them hand signals to stop taking towards us to prevent them from taking in the path of our wing tip. Have ZZZ Tower, or airport operations remind flight schools of the meaning of taxiway markings and the importance of following ATC instructions to prevent collisions.

Synopsis

A320 Captain reported having a ground conflict with another aircraft that taxied towards their left wing without clearance from ATC.

ACN: 2225783 (160 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 163
Experience.Flight Crew.Type : 222.80
ASRS Report Number.Accession Number : 2225783
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We departed Runway XX headed to ZZZ1. During the climb I heard air rushing in the cockpit, in a pulsing fashion. We noticed the Cabin Press page was showing slightly abnormal numbers for that phase of flight. We agreed to get to cruise altitude and see if it stabilized, but it did not. The cabin altitude was at 7,250 feet at FL300 which was around 2,000 feet higher than normal. The cabin rate was fluctuating from +500 FPM to -500 FPM within a couple seconds and continuously. We could feel the bumping in our ears too, and noticed the outflow valve was constantly modulating open and closed trying to pressurize the aircraft appropriately, but could not. The left pack pressure was erratic and fluctuating between 8-40 psi within a couple seconds and constantly. Captain was discussing the issue with Maintenance Control and Dispatch via ACARS. Since there was no ECAM at any point, nor any guidance from Maintenance Control for a resolution, we as a crew elected to air return to ZZZ out of significant concern for safety. After we made the 180 degree turn, I noticed both engines were surging simultaneously with the pressurization fluctuations. I recommended we request priority handling, and the Captain agreed. The Captain briefed the Purser in accordance with SOP. ZZZ Center descended us slowly and we were cleared direct to the airport. During the descent we noticed the cabin rate was now +/- 1,000 FPM and could really be felt in the ears and sinuses. Because we had no guidance during this event, we elected to not change any pack or air configuration of the aircraft to not further any issues. We landed at ZZZ without further incident.

Synopsis

A320 First Officer reported they experienced cabin pressurization fluctuations and noticed both engines surging simultaneously during cruise. Flight crew elected to return to the departure airport and landed safely.

ACN: 2225635 (161 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 11000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2225635
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Level 1 fumes event from Pack 1. Descent through 11,000 ft smelled a burnt plastic smell. Did isolation procedure for 3-5mins and the smell went away. Turned Pack 1 on to see if the smell came back and it was still there. Complied with the isolation procedure and left the pack off and xbleed shut for the rest of the flight. The FAâ€™s did not smell it in the back just on flight deck. FO also confirmed he smelled it also.

Synopsis

A320 Captain reported a Level 1 fumes event described as "burnt plastic" during descent. After troubleshooting the left pack the pack was off for the rest of the flight.

ACN: 2225628 (162 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb
Route In Use : Direct
Airspace.Class D : ZZZ

Component : 1

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Main Gear Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2225628
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon gear retraction on climb out from ZZZ, we received the ECAM L/G Doors Not Closed. We complied with the ECAM and QRH procedures and could not rectify the issue. We advised ATC and then contacted Dispatch and Maintenance Control to review the issue and options. We informed the Flight Attendants of the situation. We all agreed to divert to ZZZ1. We complied with the Divert and Overweight landing procedures in the manual and QRH. On final approach to ZZZ1 Runway XXR with gear and flaps down, we received another ECAM HYD G RSVR (Reservoir) LO LVL. We complied with those ECAM procedures and proceeded to land without any further incident. We exited the runway and stopped. After landing we had Crash, Fire, and Rescue visually look at the left main gear door and hydraulic fluid for any evidence of fire or damage. No further issues noted at that time. We requested a tug to tow us to our gate.

Synopsis

A320 pilot reported that a main gear door would not close after takeoff and soon the crew received a hydraulic failure from one system.

ACN: 2224797 (163 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Initial Approach
Flight Phase : Descent
Route In Use : Direct
Airspace.Class C : ZZZ

Component

Aircraft Component : FCU (Flight Control Unit)
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1608.52
Experience.Flight Crew.Last 90 Days : 128.75
Experience.Flight Crew.Type : 1608.52
ASRS Report Number.Accession Number : 2224797
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending into ZZZ we were deviating around some weather and was IMC. We got an ECAM ALTERNATE LAW MESSAGE. We notified ATC and requested priority handling due to flight control failure. We ran the ECAM and referred to the non normal section of the FM (Flight Manual). We decided to continue to ZZZ and landed without incident and taxied to the gate normally.

Synopsis

A320 Captain reported an ECAM ALTERNATE LAW MESSAGE indicating a flight control malfunction.

ACN: 2224770 (164 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : RNAV RNP XX
Flight Phase : Final Approach
Route In Use.STAR : ZZZZZ1
Airspace.Class D : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 161.57
Experience.Flight Crew.Type : 161.57
ASRS Report Number.Accession Number : 2224770
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Neither the Captain or myself have ever been into ZZZ. We were unfamiliar with the approach and airport. We referenced the company pages as well as the guide to familiarize ourselves. While on the ZZZZZ1 RNAV we were cleared to ZZZZZ 12000ft, speed assigned 230kts. We briefed and executed speed breaks 1/2 crossing ZZZZZ. We were cleaned the RNAV RNP Z rwy XX. We armed the approach and set 6500ft initially. FMA's where SPEED, FINAL APP. Brick and blue stick. Aircraft momentarily leveled at 10,600 between ZZZZZ and ZZZZZ1. Once passing ZZZZZ1 we still have 1/2 speed brakes, the aircraft began the decent for approach and accelerated to 260kts indicated. Unable to slow to gear speed, Auto pilot was disengaged by pilot flying and speed brakes full was engaged. Decent rate was decreased and speed was bled off. Reaching 230kts indicated we extended the gear. Aircraft slowed in this configuration to flaps 1, 2 speed and we configured on schedule. We crossed ZZZZZ2 6500ft, autopilot off, gear extended, flaps 2 and speed 190kts. We descended below 6500ft and received a â€œsink Rateâ€ alert in the flight deck. We descended to 6200ft. We received a call from ZZZ tower to check altitude. We arrested decent manually and climbed back to 6500ft. Autopilot was re- engaged. Manage speed selected and flaps 3 then full was selected on schedule. We were stabilized on approach profile at 1200ft AGL between ZZZZZ3 and ZZZZZ4 at 5700ft indicated. The remainder of the approach was uneventful and we landed successfully.

Synopsis

Air carrier First Officer reported receiving an altitude alert from ATC during approach. Flight crew returned to safe altitude and continued to landing.

ACN: 2224295 (165 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 39000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2224295
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise flt at FL390 we we got a notification that one of our hydraulic system had lost the hydraulic fluid after running the non-normal flight attendants were notified of the problems we had, also that I did not expect it to change unless we had additional systems degrade. They were given the time to landing, and were notified about the fact that we were gonna have to be towed in. We were entering ZZZ airspace from ZZZZ and [requested priority handling] based on the fact that we knew that we were down to alternate braking and also no ability to steer the aircraft off the runway after landing. Air Traffic Control was told of all the issues we were gonna have, but also that we expected it to be a normal landing with no forced deplaning however we were gonna be unable to get off the runway without tow. We were then given a vector to ZZZ and requested to land on Runway XX left, which were the longest runway available in ZZZ. Just outside the initial approach fix we started emergency extension of the gear as suggested and before that we notified ATC that we were gonna slow down. Once the gear was down successfully extended we tracked inbound at 180 knots until touchdown. After a normal landing and normal braking distance with one reverser available we came to complete stop and set the parking brake. We then started to shut down the engines and notify the Tower that we were ready to be towed in. I then notified the passengers that we had lost our ability to steer on the ground in route and weâ€™re waiting for tow vehicles to come and bring us into the gate and requested that they remain seated and keep them seatbelts on. I also notified the passengers that when you stop on the runway as a precaution, emergency vehicles would come out and take a look at the plane to make sure that nothing that we were unaware of was wrong with our plane.Once the emergency vehicles were done with their inspection the move team moved up and hooked up to the aircraft and brought us to gate XX. No further incident of any kind on the way in. During this incident, no emergency authority was taken by me, and even though we lost one braking system, we still had two more options, including reversers, so I expected a normal landing and, did not expect any emergency deplaning of the aircraft unless we had further system degradation. For safety reasons, I elected not to try to move the aircraft off the runway using the brakes since that would've hampered the accessibility from emergency vehicles that may or may not want to check the aircraft. Other than the hydraulic leak, no additional damage to aircraft passengers or crew was recorded by me.

Synopsis

Air carrier pilot reported a hydraulic system failure during cruise.

ACN: 2224113 (166 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 78.27
Experience.Flight Crew.Type : 78.27
ASRS Report Number.Accession Number : 2224113
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While on final for visual XXL into ZZZ the Capt was PF and commanded gear down, I selected gear down and the gear failed to extend. We received an ECAM of L/G GEAR NOT DOWNLOCKED. We advised ATC and were vectored off the approach to address the issue. After completing the ECAM, which led us to manually extend the gear, we [requested priority handling] and the Capt briefed the flight attendants. We advised ATC that we would need to stop on the runway and would need a tug to tow us to the gate due to not having nose wheel steering. After an uneventful landing we were towed to the gate.

Synopsis

A320 First Officer reported they selected the gear down and the landing gear failed to extend on final approach . They manually extended the landing gear and landed.

ACN: 2221773 (167 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 140.55
Experience.Flight Crew.Type : 3177.90
ASRS Report Number.Accession Number : 2221773
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise portion of STAR, an ECAM message appeared Nav Fm/gpc pos disagree. Within seconds ATC queried if we were direct ZZZZZ, 20 deg right of course. Captain NAV showed direct, FO, showed right of course. Ran ECAM checklist, but ECAM went away. About 5-10 mins later, ATC once again asked if we're direct ZZZZZ, he was showing us right of course. From that point on we were given headings to fly. After arrival, MX (Maintenance) indicated there was a problem with the FMGC and that it needed to be replaced.

Synopsis

A320 Captain reported course deviations due to FMGC failure resulted in critical equipment failure requiring ATC guidance.

ACN: 2221714 (168 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 20000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb

Component

Aircraft Component : Airframe
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2221714
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb out from ZZZZ, we started experiencing airframe vibrations at approximately 280 knots at FL200. We slowed down to 260 knots, approximately Mach 0.75, it started to improve. We attempted to increase speed and the vibration would come back immediately. When we called the inflight, they informed us they felt a very strong vibration in the back, specifically near row XX and beyond. Once at FL340 we still felt some vibration at .75. We initiated a satcom call with Dispatch and Maintenance. We attempted to gently increase the speed and it immediately got much worse. At the mean time we got informed by inflight that customers at row XX were getting very uncomfortable and some were starting to cry. Since the vibration wasnâ€™t getting any better and after agreement between us (CA and FO) Dispatch and Maintenance we decided that the best course of action was to divert to ZZZZ1. We informed the inflight and customers of our intentions, and diverted to ZZZZ1, performed the diversion and overweight landing checklist. Landed approximately 5000 pounds overweight and less than 100 FPM descent at touchdown. After arrival made multiple logbook entries, and fill a report.

Synopsis

A320 pilot reported airframe vibration resulted in a divert with expeditious handling to a safe landing.

ACN: 2220584 (169 of 2908)

Time / Day

Date : 202503

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 230.98
Experience.Flight Crew.Type : 722.58
ASRS Report Number.Accession Number : 2220584
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the climb out from ZZZ we experienced multiple ECAM messages. The Captain (PF) continued flying as I (PM) worked on the ECAM. As soon as I cleared the first ECAM message, all the other ECAMs went away and everything was back to normal. A few seconds later more ECAM messages popped up and which included a CB (Circuit Breaker) in the avionics bay, a fuel feed, (and 2 other fuel related ECAMs) which cleared in about 5-10 seconds before I could go through them. About 30 seconds later, we got other ECAM failures SDAC2 (System Data Acquisition Concentrator) fault and another avionics CB related ECAM. We worked on the ECAMs and at this point knew there might be a bigger problem with the aircraft than what we are seeing. The Captain and I were not concerned about the aircraft status and we decided to contact Maintenance Control and explain what we were experiencing. At this time the Captain handed the aircraft and got in touch with Dispatch and Maintenance Control and collectively deemed it necessary to return to ZZZ. We briefed ourselves on the plan to return information ATC, FAs (Flight Attendants) and passengers of the situation. ATC vectored us back to ZZZ and we returned to ZZZ. We arrived at the gate and were met with multiple maintenance personnel. Dispatch and crew scheduling called us and check on us then later released us from duty.

Synopsis

Air carrier First Officer reported on climb out receiving multiple ECAM messages resulting in the flight crew returning to the departure airport.

ACN: 2219588 (170 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2219588
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After blocking in on Aircraft X (operating as a part 91 maintenance ferry) in ZZZ gate XX, I inadvertently partially deployed the door slide on the L-1 door by mistakenly pulling up on the door handle briefly along with the disarming handle. I immediately stopped and made sure the disarming lever was fully opened and stopped the door from opening further. Although the slide did not deploy, the door was not fully disarmed while partially open. I told the gate agent and he notified maintenance as well as the ramp supervisors. There were no passengers, and maintenance removed the slide from the door without the slide deploying. Cause: I was unintentionally rushing myself doing something that I rarely do... I should have slowed myself down and been more careful in disarming the door, and the next time I will have the FO with me to confirm which lever I actually am using.

Synopsis

A320 Captain reported an inadvertent partial escape slide deployment due to a door not being fully disarmed.

ACN: 2218503 (171 of 2908)

Time / Day

Date : 202503
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class E : ZZZ

Person

Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 8
Experience.Air Traffic Control.Time Certified In Pos 1 (mon) : 0
ASRS Report Number.Accession Number : 2218503
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X was inbound from the west given vectors to final for the ILS at ZZZ. The pilots kept asking for lower. Controller had them at an appropriate altitude for the area they were in. As they were vectored on to final the aircraft started descending below the assigned altitude and eventually went down to 010 MSL about 8-9 mile final before the approach clearance was cancelled. Pilot insisted on continuing but upon further examination aircraft was also left of course and vectored to the east for another attempt. Readback all seemed clear, there was a possible language barrier as well with some accent on the pilots part. This all came less than 2 hours after receiving simulator training on Low altitude alert phraseology for both Controllers. Aircraft X was eventually re-sequenced without further issue into ZZZ. Pilot also asked for the weather information which was not available due to the constant issue of not having the weather populate correctly on the system, ZZZ is also not an adapted weather site in the system. ZZZ1 weather was issued to the pilots. Recommendation: No changes. the refresher training I do believe helped although the phraseology not issued properly. made the Controller aware of the possibility of the aircraft descending below a given altitude. Honestly, perfect timing of the simulator training!

Synopsis

TRACON Controller reported an aircraft descended below their assigned altitude resulting in the aircraft flying towards terrain. ATC canceled the approach clearance, climbed and vectored the aircraft back to the final approach course and they landed safely at their destination.

ACN: 2217022 (172 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Descent
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2217022
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight ABCD enroute to ZZZ an EICAS in cyan for ELECTRIC FAULT. We ran ECL (Electronic Checklist) for ELECTRIC FAULT and another EICAS for ENGINE VIBRATION in yellow. We ran ECL for ENGINE VIBRATION and at complete ENGINE VIBRATION EICAS reappeared 3 times at cruise and once in descent. Contact Maintenance but could not resolve issue. Divert to ZZZ1, no other issue. Maintenance menu on ground showed an over vibration on Engine 1.

Synopsis

Air carrier pilot reported electric fault warning and engine vibration during cruise and descent. Flight crew diverted to nearby airport.

ACN: 2216488 (173 of 2908)

Time / Day

Date : 202502

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 117.15
Experience.Flight Crew.Type : 416.22
ASRS Report Number.Accession Number : 2216488
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Pre-flight

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During preflight, we received neither a "no Dangerous Goods (DG) planned" message nor a preliminary DG report from ACARS. Per Operations, we did not push back early for our leg to ZZZ (for slot time compliance), but after the Before Push Checklist was complete and the ground crew confirmed "ready for brake release and pushback" we released brakes five minutes prior to push time. At that point we received an ACARS message with the final DG summary, which we acknowledged via DG acknowledgement code, before we pushed back, but after brake release. Also, during this time, I called Dispatch to advise them of the irregularity, and the Dispatcher advised that he could not see any DG plans for the flight where he normally would have seen them. No cargo doors were opened at all during this period. In flight, I resent a DG acknowledgement code to reconfirm the DG onboard, and it matched what was printed just before pushback.

Synopsis

Air carrier Captain reported there was no confirmation whether there was DG on board during preflight and the flight crew received the final DG summary after brake release but before pushback.

ACN: 2214911 (174 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.CTAF : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1847.87
Experience.Flight Crew.Last 90 Days : 161.72
Experience.Flight Crew.Type : 1847.87
ASRS Report Number.Accession Number : 2214911
Human Factors : Confusion

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 795.62
Experience.Flight Crew.Last 90 Days : 175.08
Experience.Flight Crew.Type : 795.62
ASRS Report Number.Accession Number : 2214928

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were holding short of Runway XX for departure in ZZZ (uncontrolled airport). ZZZ Center landed 2 airplanes in front of us and allowed 2 smaller aircraft to depart, one Piper Cherokee (IFR) and one Cessna Citation 2 (VFR). Because the Cherokee was slower (and was not following ATC instructions initially, to my recollection), Center asked if we would accept a left visual terrain clearance departure turn after departure, heading of 030 (it was completely clear and I had experience in and out of ZZZ airport). Center informed us we have a void if not off time of 2 minutes and did not make any other indication of traffic in the area. After we both agreed to take visual responsibility for terrain and weather per FOM/FM (Flight Manual) (which there was no weather), we departed after making announcements on UNICOM. There were no other aircraft that made any advisories. After takeoff and in the turn to 030, I announced on UNICOM that we were turning left and departing to the northeast. We contacted ZZZ Center in the climbing turn and checked on successfully. We noticed converging traffic at our 2-3 o clock on our TCAS as we climbed through 8900 feet to 9000 feet. ATC instructed us to level at 9200 (initial clearance was 15000), which was the same altitude as the other converging traffic (9200). We were confused about the instruction and began to level. We immediately received an RA to climb, in which the FO executed without hesitation. We then received a level off instruction RA. Upon clear of conflict, ATC allowed us to resume our climb uneventfully.

Narrative: 2

While holding short Runway XX in ZZZ, ZZZ Center asked us if we can fly a heading of 030 and climb and maintain 15,000 ft for departure while maintaining visual obstacle clearance since the usual Runway XX Obstacle Departure Procedure provided in our Jeppesen FD (Flight Data) chart page has traffic that is in our way in the departure corridor. Captain and I agreed that we can do that since we were in daytime VMC. We were then cleared for that departure procedure with a clearance void time of 3 minutes and to contact ZZZ Center once airborne. We then switched to the UNICOM/CTAF frequency and announced our intentions. Once airborne and turning left to a heading of 030 and climbing to 15,000 feet, we switched back to ZZZ Center and checked in with them. As we were climbing just prior to 9,000 feet at a heading of 030, we had a Traffic Advisory at about our 2 or 3 o'clock position from TCAS and from ATC simultaneously. We were then told to level off from ATC soon afterwards at about 9,200 feet. Now TCAS is showing the other traffic with a relative altitude of "00" in relation to our altitude. A few seconds later, we received a RA to "climb, climb". As the pilot flying, I immediately pushed the thrust levers to TOGA and manually pitched the aircraft upwards to comply with the vertical speed needed to avoid the midair collision. I decided to have TOGA thrust despite that our FM (Flight Manual) in the "Maneuvers" chapter in the Resolution Advisory procedure not specifying to do that for our current phase of flight and aircraft configuration since we were at a high airport elevation of 6,606 feet MSL with rising mountainous terrain in front of us. As we climbed past 10,000 feet, TCAS advised us to "level off". I then lowered the pitch of the aircraft to reduce the climb and reduced the power setting of the thrust levers to not overspeed. ATC then gave us a higher altitude and I resumed normal climb after CA informed them we had a TCAS RA. I was hand flying throughout this entire event since takeoff.

Synopsis

A320 flight crew reported TA and RA during departure climb from a non-towered airport.

ACN: 2214363 (175 of 2908)

Time / Day

Date : 202502

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Landing Gear Indicating System
Problem : Malfunctioning

Person

Location Of Person : Company
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2214363
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We arrived at the airport to find out that Aircraft X had 9 MELâ€™s and one was LGCIU (Landing Gear Control Interface Unit) 2 which is very important part on the aircraft. This was causing more issues because it only had 7 when it left ZZZ1 and it triggered 2 more on the prior flight getting to ZZZ. To make matters worse this is an aircraft that has in the past have major mechanical issues including a depressurization not to long ago. Me and my FO even though we knew that everything had been MEL correctly did not feel safe flying that aircraft. Not only did these MELâ€™s had multiple procedures that we normally don't do, but if anything else failed we could possibly go into alternate law. The flight was already delayed because of the multiple failures it had on the prior leg. My FO and I seriously felt scared to fly that aircraft. We offered to wait for another aircraft to arrive and perform the flight which we did. That aircraft needs some heavy maintenance. It has been brought to my attention that it had already been sent to retirement because of its age and multiple issues, and was brought back. I donâ€™t know if thatâ€™s true, but does seem like it definitely has consistent maintenance issues.

Synopsis

A320 Captain reported they refused to take aircraft due to the aircraft having several maintenance issues.

ACN: 2214335 (176 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 4000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : ILS XX
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : ZZZ
Make Model Name : Cessna Aircraft Undifferentiated or Other Model
Operating Under FAR Part : Part 91

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2214335
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Miss Distance.Vertical : 400
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

This report concerns profound failures by ATC leading to a near-midair collision. On arrival to ZZZ, after being handed off to Approach we descended to 4000 feet on a southerly heading. The approach in use was ILS Runway XX. The Approach Controller was busy working quite a few aircraft. We received a call from the Approach Controller for â€œtraffic ahead, a Cessna 400 feet above you.â€ She provided no other information. We replied that we were â€œlooking for the traffic.â€ At no time during this approach did we ever report any traffic in sight. At this point we had the traffic directly ahead of us on TCAS at about 10 miles, also indicating +400 feet. We looked diligently for the aircraft and finally saw it (a single-engine prop like a 152/172) as we approached 5nm separation, now slightly to the right of our course. The Cessna was apparently flying westbound, parallel to and in the opposite direction of the ILS Runway XX final approach course that was just a few miles to the south. We were discussing our options and wondering what approach was planning when the controller (a different voice) gave us a turn to the west. We made that turn and began to pull ahead of the Cessna. Eventually it disappeared from the TCAS display as it fell behind. During this time the Approach Controller was busy giving vectors to other aircraft and taking new hand-offs from Center. He never mentioned the Cessna traffic. The Approach Controller then gave us a turn back to the south towards the final approach course. As we rolled out of the turn we got an immediate RA â€œDESCEND DESCENDâ€. There was no traffic advisory prior to the RA. As pilot flying I complied with the RA until the TCAS system stated clear of conflict, then leveled off. We never saw the Cessna. Once we were stabilized we called Approach and told them that we had responded to an RA. The controller acknowledged with an â€œOKâ€ then went back to vectoring other aircraft. We had to assertively inquire what altitude he wanted us at now and what heading we should take since we were about to cross the final approach course. He turned us toward the airport and we completed an uneventful approach and landing. He never mentioned the Cessna. After parking at the gate I called the Approach Control facility and spoke with a supervisor. He seemed unaware of the situation. After giving him a brief summary of what happened the only thing he asked is whether we had called the traffic in sight at any time. Again, we had not. This event has many disturbing similarities to the recent mid-air collision in DCA. Approach Control was tolerating a VFR general aviation aircraft flying very close to a very busy instrument approach corridor. It was not clear to us whether approach was even communicating with the Cessna. The Approach Controllers made a single call to us about traffic, seemingly unconcerned about an aircraft directly ahead of us and only 400 feet above. After the first call there was never any communication from ATC about the Cessna, even when they turned us directly into its path. This situation reeks of complacency by the local Approach Controllers, who may be accustomed to seeing VFR aircraft traveling close to the final approach path to the airport. Furthermore, it was extremely lucky that the Cessna was reporting its altitude (obviously not always the case for small civilian aircraft). With that information our TCAS was able to give us the â€œcorrectâ€ instruction to descend. Without that is possible that we might have climbed instead directly into the conflicting traffic. Again, ATC was of no assistance. Finally, it seems likely that there was some kind of conflict alert that was ignored by the Approach Controllers, since both aircraft were reporting position and altitude. Again, controller complacency and misplaced priorities seem likely factors. In hindsight I should have been more proactive about asking ATC exactly where the Cessna was before I executed the final turn to the south. I mistakenly relied on Approach Control to provide separation between two aircraft that were both reporting an accurate position and altitude. Cause: Controller complacency and perhaps normalization of deviance. The FAA should institute much clearer directives for Approach Controllers to further separate IFR and VFR traffic in the vicinity of major airports, as well as to prioritize immediate responses to any alerts about conflicting traffic.

Synopsis

A320-200 pilot reported during initial approach they had a near mid-air collision with a VFR aircraft that was flying opposite direction in the approach corridor resulting in a TCAS RA.

ACN: 2213672 (177 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2213672
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Just after takeoff from ZZZ an ECAM for DOOR FWD Cargo appeared. We followed the ECAM and QRH procedure. Pressurization was normal. We were still in the terminal area of ZZZ and I decided the safest action was to land and have the door inspected. I notified dispatch, the flight attendants and passengers we would be returning. Upon landing firetrucks met the aircraft and inspected the door. The trucks were not requested by the crew. We returned to the gate. The door light was put on MEL and the door was inspected and verified closed and locked. We then continued our flight to ZZZ1.

Synopsis

A320 air carrier pilot reported a cargo door ECAM malfunction after takeoff. The flight returned to the departure field where the cargo door was put on MEL by maintenance.

ACN: 2212998 (178 of 2908)

Time / Day

Date : 202502

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Landing

Person

Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 2212998

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Extremely hard landing by FO. Pax were emotionally shaken. I was FA2 and my neck and shoulders and back are very sore. MX inspected A/C and we took a delay while inspection occurred. More training for pilot.

Synopsis

Flight Attendant reported aircraft made a hard landing resulting in injury and maintenance inspection.

ACN: 2212368 (179 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Snow
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 5874.82
Experience.Flight Crew.Last 90 Days : 17.9
Experience.Flight Crew.Type : 4439.42
ASRS Report Number.Accession Number : 2212368
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Excursion : Runway
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

ZZZ1 - ZZZ night landing in ZZZ during cold weather operations. I was reassigned to fly the ZZZ1 - ZZZ leg after my original flight to ZZZ2 was delayed several hours. I arrived at the aircraft approximately 37 minutes prior to push back and started downloading the paperwork. The First Officer (FO) had already loaded the FMC. Enroute, received report from Dispatch that ZZZ Runway Condition Code (RCC) was 5-5-3 for Runway 19. We confirmed by asking for airport conditions using ACARS. Based on our landing weight of 138.6 and and using the ZZZ 10-7D landing distance charts (no tailwind) we determined we needed a RCC of no less than 4 (this produced a landing distance around 5820 feet plus 220 feet for QNH correction, unless we burned fuel down to a landing weight less than 137.7 with no tailwind and QNH correction). Approximately 15 minutes later, Dispatch informed us that ZZZ had worked on runway and RCC was 5-5-4. We asked to be kept up to date on RCC if it changed and we would need to burn 900 lb of fuel if the RCC went back to 3. We also asked about using ZZZ3 (instead of ZZZ4) as an alternate if we needed to lower our landing weight more. In the end, we kept ZZZ4 as an alternate since we would be able to burn fuel down to an acceptable landing weight and still land in ZZZ4 with 4500 - 5000 lb of fuel with a BINGO of 8000 lb of fuel. We elected to descend early to burn more fuel and lower landing weight to around 138.3. We performed the ILS Z Runway XX. Broke out of overcast between ZZZZZ and ZZZZZ1 with moderate snow around 2500 feet AGL. FO asked for another braking action report and Tower said it was still 5-5-4. Autopilot and auto thrust off, on airspeed and glide slope with aircraft getting slightly high around 1100 feet, then back on glide slope by 900 feet. At 200 feet, I shifted the aim point to 500 feet from the approach end. At this point the FO asked Tower to lower the runway light intensity because it was hard to make out details on the snow covered runway with the bright lights. I noticed the runway's center 2000 feet was partially cleared of snow, but the approach and departure ends of the runway (~2000 feet on each end) were snow covered. Never noticed the light intensity decreasing. Landed approximately 1300 feet down runway (hard to tell exactly with snow covered runway) and close to VLS. Medium autobrakes were selected as per the 10-7 page, but I did not feel a deceleration or see the decel light illuminate so I immediately applied manual brakes to start the deceleration. Brakes seemed to work normally entering the center 2000 feet of the runway, but with 3000 feet remaining, the FO and I both noticed the brake's effectiveness was not what we needed to stop the aircraft on the runway. At this point we both applied full manually braking and the anti-skid was cycling. I said we are going into the overrun about 500 feet from end of runway. I continued using full reverse thrust and maximum manual braking. The aircraft entered the overrun around 15 - 20 knots until we stopped approximately 200 feet onto the paved overrun. At this point I moved the thrust levers to forward idle. We informed the Tower that we had stopped in the overrun and we were instructed to turn around and taxi to the gate. After informing Tower of the RCC being less than 4; we said the RCC was 1 or 2, the Tower noted we were fast on the runway (we were, due to the poor braking conditions). We performed a 360 degree turn in the overrun and taxied uneventfully to the gate. I performed walk-around after engine shutdown and no aircraft damage was observed.

Synopsis

A320 Captain reported during landing at night on a snow covered runway the aircraft did not stop until entering the overrun.

ACN: 2212317 (180 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2212317

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Flight was uneventful from preflight thru landing. Due to a short landing roll and traffic on adjacent taxiways, I slowed the aircraft to make a Taxiway 1 turnoff. It became evident that we could make a XXL turnoff as well, so I slowed to taxi speed for that and as I started to ask the First Officer (FO) to get clearance for XXL turnoff off (ATIS only gave permission to use XXR) Tower gave us the clearance without asking as I think he saw what we were thinking and knew it would be better for ground traffic as well. It is important to note, I had never made that turnoff before, but due to the frigid temp, headwind, and light landing weight it was a smooth and controlled braking effort down to taxi speed. Taxi to gate XX was normal and with appropriate timing we shutdown Eng #2 and performed a Single Engine Taxi Without APU (SETWA) as all taxiways were clear. We stopped at the gate per the safe dock system. We hooked up power and shut down the #1 Eng and performed our parking flows. Once I saw "chock on" I checked the brakes and saw them increasing in temperature, I then took the parking brake off and looked outside for a few seconds to confirm we weren't moving. When the FO finished her items, she did ask me if I wanted to set the parking brake and I told her "the chocks are in and I will set it and take care of the fans when the temps stop climbing". Apparently, around this point, either slightly before or after, we started moving backwards slowly, but I had no relative motion we were moving because airport operations (AO) was moving the jet bridge towards us at the time. We then felt a thump and I assumed it was the jetbridge hitting us. From my window, the jetbridge was at an angle and while the near corner was 5 ft away the other corner was angled out of my view and towards the aircraft so I thought it might have hit us. After a quick discussion with the FO, we knew something had happened. I set the Parking Brake and notified operations that AO may have hit the jetbridge or something to that effect. The AO crewmember looked frazzled, so I called operations for Maintenance and I think I told them to contact the safety department as well. Ground operations (GO) came on the headset and told us the aircraft had rolled back "like 1 to 2 feet" I said "werenâ€™t the chocks in" and he said they were pushed out of the way by the nose gear and that we also rolled over a chock or something to that effect. He confirmed the jet bridge did not hit us. After some time, Maintenance and GO looked over the aircraft, saw no damage, and then they hooked up a tug and towed us forward. The tow forward seemed more than just a few feet. The pax were kept well informed throughout the ordeal and they and the flight attendants were oblivious to any movement of the aircraft after our initial block in. Even despite the additional time of the Maintenance check and arrival the tow forward, the aircraft blocked in 6 minutes early. After block in, we were met by a ZZZ Assistant Chief Pilot (ACP) who saw the video and saidâ€”-we rolled closer to 4 feet, that only the nose was chocked at the time, the chock was pushed away by the aircraft, and that the thump we felt was us rolling over one of the chocks the GO threw in front of it before the aircraft came to a stop. We also discussed my thoughts for releasing the parking brake for cooling. My rationale was simple, we did a quick stop in a short distance which would heat up our brakes for our quick turn. That being said, this ended up being poor situational awareness on my part. The same reasons we were able to stop so quickly on the runway would be the very same reasons our brakes wouldnâ€™t get very hot. Taking into accountâ€”light landing weight, frigid temperatures, a good wind, and an early arrival there was little chance our brakes would ever reach the 350deg threshold of the manual or affect our next departure. A post flight walk around revealed no damage to the aircraft. However, the following issues were noted; a noticeable dip in the ramp and a 3 feet by 2 feet wet/deiced/sandy spot right where the nose gear should stop with safe dock. A log book entry was made, cleared, and we departed on time for our next leg.

Synopsis

A320 Captain reported while attempting to park at the gate they didn't set the parking brake to allow brake cooling resulting in the aircraft rolling backwards. The Captain was able to apply the brakes stopping the backwards roll.

ACN: 2210367 (181 of 2908)

Time / Day

Date : 202502

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 112.2
Experience.Flight Crew.Type : 677.72
ASRS Report Number.Accession Number : 2210367
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Ground Personnel
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During the pushback phase of flight while waiting to disconnect the tow bar we experienced slight uncommanded aircraft movement. The initial pushback from the gate was uneventful and we received clearance to start engines. The Captain asked for both engines to be started and I began my flow to start the number 1 engine. Once it was time in the pushback sequence to set parking brake, the command from the push crew was not received. This caused confusion and the captain asked if it was set. We felt the aircraft move forward. As soon the sensation was felt, he swiftly applied the parking brake. The airplane moved no more than 1 foot. The ground informed us that a new ground crew personnel had moved too close to the starting engine danger area. He said that he removed the tow bar before asking for the brakes to be set. He apologized and we continued with the #2 engine start when it was safe to do so.

Synopsis

A320 First Officer reported failure of ground crew communication on pushback resulted in uncommanded aircraft movement and personnel to close to engine safety area.

ACN: 2210203 (182 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Government
Make Model Name : Helicopter
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Mission.Other 
Flight Phase.Other 
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2210203

Events

Anomaly.Conflict : NMAC
Detector.Automation : Air Traffic Control
Detector.Automation : Aircraft TA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were vectored for the RNAV Z XXL into ZZZ. Somewhere between ZZZZZ and ZZZZZ1 ATC informed us of law enforcement helicopter traffic to our south and below our altitude and were told they would stay clear of us. During our descent on profile we received a TCAS Traffic Advisory for the helicopter. On TCAS they showed 300 feet below our altitude. I reduced our descent rate to insure we donâ€™t get any closer. At no point did myself or the FO see the helicopter traffic as they were below us. Shortly after adjusting our descent rate the TA subsided and I was able to continue on the descent for a normal and landing. At no point during the approach did I feel unsafe however it was uncomfortable not having the helicopter traffic in sight with a 300 foot separation. Especially since [we] were descending on profile for landing. Once on the ground I called ZZZ tower to discuss the event. The controller I spoke to was very professional and showed a willingness to help. The controller took down the flight information and mine and said he would look at the flight track and voice transmission. A couple of hours later the controller contacted me. He explained that the helicopter traffic was law enforcement and was in pursuit along the freeway. He mentioned that this occurrence is not standard however does not happen often. The controller added that the helicopter had us in sight and later turned south during the pursuit. I completely understand that this may not happen very often, but believe it should not happen at all especially in light of the events in DCA. Law enforcement pursuits have the potential of being anything but controlled flights and the aircraft flying the pursuit may have to maneuver erratically. It is my belief that in instances like the one I described, where helicopter or aircraft traffic is maneuvering at low altitudes near the approach end or departure end of a busy airports more care should be taken.

Synopsis

A320 pilot reported while on final approach being advised of helicopter traffic resulting in the pilot receiving an TA and reducing their descent rate to avoid the traffic.

ACN: 2209353 (183 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class D : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 223.82
Experience.Flight Crew.Last 90 Days : 152.97
Experience.Flight Crew.Type : 223.82
ASRS Report Number.Accession Number : 2209353
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 6989.48
Experience.Flight Crew.Last 90 Days : 168.47
Experience.Flight Crew.Type : 6989.48
ASRS Report Number.Accession Number : 2210068
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Cleared for visual app to rwy XX at about 13000 ft, set 6700 ft for the FAF, started a slow descent and turned downwind. At about 8000 ft, a terrain warning was triggered. The PF successfully executed a terrain avoidance maneuver.

Narrative: 2

Approximately 17 miles east of ZZZ we were cleared the visual to runway XX. I turned to a heading of 275 and started my descent. We got a terrain warning followed quickly by a pull up too low terrain. I executed the terrain escape maneuver and began a climb. The warning went away and we proceeded to the airport and landed uneventfully.

Synopsis

A320 flight crew reported terrain warning during approach.

ACN: 2208936 (184 of 2908)

Time / Day

Date : 202501

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Cowling
Aircraft Reference : X
Problem : Failed

Person

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2208936
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Other Person
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Flt crew reported during flt to check the #1 eng inlet accustic mesh liner. This log was created after aircraft departed ZZZ and was in the air. On arrival the #1 engine was inspected and found that the inlet wire mesh/screen linner was out of limits by more than twice the limit. this issue should have been addressed before departing ZZZ. The flight crew was aware of the discrepancy and intentionally chose not to report it while on the ground at ZZZ and then made a write up while in the air!!! It is obvious that the flight crew violated all the safety protocols of the company and the FAA. The flight crew departed ZZZ with known discrepancy and without reporting it. And also just for your info. the Company had scheduled 2 departures from ZZZ-ZZZ1 Day 0. The first flt was cancelled due to maintenance issues. this makes you wonder why on the log was created after taking off from ZZZ, so to prevent second flt from canceling? If the flight crew were to report this discrepancy while on the ground at ZZZ, this acft would have been OOS (Out of Service) causing further delay, disruption or cancellation. This is not the only example of pilots reporting known discrepancies while in the air!! some other examples are tires with chord showing, brakes flush, lights inop, etc, etc. Pilots should report the discrepancies when found and not decide when and how to report based on flight routing. why wait until in the air to report a discrepancy that was discovered while on the ground at a station - regardless of it is (out station) or at (mx station). All the flight crew members, and flight attendants included, should be trained and instructed to report known discrepancies right away and not to wait regardless of where the acft is. Safety is top priority at this company and should not be taken for granted. Always follow the SOP, FAA rules and company safety protocols for reporting known discrepancies. Why kick the problem to the next station. Do what's right to address the issue at the present time. What if this mesh liner came apart on take off or in flight and was ingested into engine??.. what if?? -- for example, see the several logs at "ZZZ2" on Day 1 for details. It is the same scenario of inlet cowl acoustic lining coming apart and was ingested into the engine causing significant damage.

Synopsis

Maintenance technician reported a flight crew did not report engine cowling damage prior to departure, and instead reported the damage during the flight.

ACN: 2208614 (185 of 2908)

Time / Day

Date : 202502
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 59.22
Experience.Flight Crew.Type : 389.25
ASRS Report Number.Accession Number : 2208614
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Security
Detector.Person : Passenger
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

As passengers were boarding, one husband asked his wife if she got the battery from the luggage. She said, no, and the passenger came forward to a flight attendant and admitted there was a lithium battery in his luggage in the cargo area. The flight attendant asked the gate personnel if they could retrieve the battery from the bag. Gate personnel indicated it was not their policy, a new policy, to do that. The gate agent said the passengers bag would have to be removed and sent back to baggage claim, forcing the passengers to miss their flight. My first officer, asked me if I minded if he could go retrieve the bag so the passenger could retrieve his battery. I looked in the FOM, and found nothing prohibiting this. My first officer went to the cargo bay, with several ground crew, watching, to include the head security agent. He retrieved the bag, brought it to the gateway, and the passenger was allowed to remove his battery in full view of my first officer, the purser and the gate agent. Shortly after, the head of security who had witnessed the first officer, remove the bag, but did not stop him, came to the cabin door and said a security incident had been committed, and all the passengers had to deplane for a security inspection. I called dispatch and the Chief Pilot, and told them of the situation. A security inspection was accomplished, and the flight departed one hour late.

Synopsis

Air carrier Captain reported a Lithium battery was retrieved from cargo during preflight in order to avoid a Hazmat violation. A security inspection was completed and the flight departed late.

ACN: 2208283 (186 of 2908)

Time / Day

Date : 202501

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2208283
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Day 0, while operating ZZZ-ZZZ1, during the climb phase of flight, we received an ECAM for SMOKE AFT CARGO SMOKE. I was PM during this leg and started ECAM actions when directed. After turning as CAB FANS per the procedure, the initial ECAM disappeared, leading us to believe it was a spurious alert. A short moment later, I noticed what I felt was the presence of a smoke odor in the cockpit. I alerted the Captain to the smell and he agreed that it did not smell right. At this time, I called to the FWD flight attendant and asked them if everything was normal in the back. The flight attendant advised that people were starting to stand up and they were trying to figure out why. I asked if there was smoke in the cabin, and then she noticed it and advised there was smoke coming from the approximate area of rows XX-XY. Almost simultaneously, we got the AFT CARGO SMOKE ECAM again, we followed through with the entire ECAM, discharging the aft cargo agent into the bay. The Captain and I decided to [request priority handling] at this time and start heading back to ZZZ. I advised ATC and requested immediate turn back to the airport. I advised the flight attendants what was happening. At this time we received the HYD Y RSVR LO LVL ECAM, and performed ECAM actions. Once we had started our turn and descent back towards the airport, I advised the passengers that we would be returning out of an â€œabundance of cautionâ€ back to ZZZ. ATC gave us routing directly to the ILS for RWY ZZ, the Captain advised dispatch with ACARS and we continued to the airport. I continued to communicate with the FAâ€™s and requested they secure the cabin immediately for landing. The flight attendants advised they were moving passengers away from the rows to get them away from the smoke. The flight attendants advised me when the cabin was secured prior to landing. I updated the passengers during our descent, appreciably 10 minutes out, advising we would be landing in 10 minutes and to expect a normal landing. We landed without further incident and were followed back to pad #2 by Airport Rescue and Firefighting (ARFF).

Synopsis

A320 First Officer reported receiving an ECAM for smoke in the aft cargo compartment. Extinguishing agent was discharged and flight returned to departure airport.

ACN: 2208095 (187 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb
Airspace.Class C : ZZZ

Component : 1

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Failed

Component : 3

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2208095
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2208101
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Weather
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

WX conditions: low ceilings in icing Conditions, IMC until short final on return. Terrain in vicinity of airport. Runway condition:wet 5/5/5. Channelized Attention Shortly after takeoff, after gear retraction was commanded, ECAM alerted on L/G (Landing Gear) Right Shock Absorber followed by L/G Door and A/THR (Autothrust) Off. L/G ECAMs required speed less that 220kts. A/FLT ECAM also alerted requiring CA to hand fly. At this time CA became fixated on airspeed which resulted in a very steep decent rate greater than 3500 fpm below 1800â€™ MSL in vicinity of terrain while still well below Minimum Safe Altitude. Both a sink rate and donâ€™t sink call were observed from GPWS. Airspeed accelerated due to the dive, which led to a L/G overspeed. To prevent further overspeed, CA reduced throttles to near idle, but failed to recognize dive. FO called â€œclimb, climb, climbâ€ and directed at least 15 degree nose up to CA. During the attempted climb, FO noticed throttles were still near idle and called â€œTO/GAâ€ while pushing throttles to TO/GA from base of throttles. CA responded to â€œTO/GAâ€ as well, placing throttles to TO/GA and resuming climb to 4000â€™, a Minimum Safe Altitude we remembered from our approach brief the previous night. Upon reaching 4000â€™, CA attempted to re-engage A/FLT and A/THR however neither seemed to operate properly and CA resumed hand flying. FO ran L/G ECAM procedures and recycled gear to down position. Upon noting down and locked, FO suggested that we leave them down and return to ZZZ. CA approved of that plan and FO coordinated with ATC and [advised ATC]. FO passed required information to ATC, who then vectored us to ILSXX. FO gave a very short ILSXX brief to cover highlights, then CA commanded for Descent Checklist while FO ensured FMS was reprogrammed for the return. As we were turning toward final, CA requested a landing assessment, FO responded with â€œrunway is 5/5/5 braking action is good, so standard assessment should apply. I selected MED auto brakes for you so I donâ€™t have to think about itâ€. In hindsight, FO could have double checked and noticed the slightly overweight condition, but after the near-CFIT event, getting on the ground was the driving factor. After completing configuration for landing, FO EMER CANC the L/G ECAM messages in order to display the LDG ECAM so we could complete the checklist. Upon landing, CA stopped on Runway XX while FO coordinated with ATC and emergency vehicles to inspect aircraft. After initial inspection was complete, CA cleared Runway XX while emergency vehicles followed us to gate. After shutdown, due to the GPWS event, CVR and RCDR (Recorder) circuit breakers were pulled at the request of safety. Cause: L/G malfunction, A/THR & A/FLT malfunction, Channelized attention, Spatial Disorientation. None of the malfunctions had obvious early indicators of possible failure to flight crew. Hand flying and crosscheck skills need to remain as emphasis items practiced inflight and in training simulators. Additionally, reminding PMs of â€˜keywordsâ€™ that can be used as commands to trigger PF muscle memory to breakthrough channelized attention. If FO had not commanded â€œTO/GAâ€, this event could have ended very differently.

Narrative: 2

On climb out from ZZZ we received a landing gear shock absorber fail message, followed by a landing gear door open message. We recycled the gear per ECAM guidance and lowered it, indicating three green lights for safe gear. We left the gear down, [advised ATC], and got vectored back to the airport. Somewhere in there, I was hand flying, having trouble controlling speed below the 220 kts mandated by the ECAM. Thrust levers were still in TOGA, Iâ€™d missed that cue somewhere. Set the thrust levers to climb and engaged auto thrust, which had become disconnected. Auto thrust failed to engage. With this sorted, auto pilot was selected but would not engage. Subsequent attempts only spawned further ECAM warnings about auto flight not working and were abandoned. The FO, (the hero in this story), noticed we were descending, TOGA thrust was applied and escape maneuver initiated. The GPWS was activated at some point. Landing in ZZZ was uneventful but overweight. The casual factor was my micro concentration on speed and little elseâ€”credit to FO for pulling me out of my stupor. As mitigation, I would greatly appreciate a sim session to practice hand-flying.

Synopsis

Air carrier flight crew reported during initial climb Landing Gear, Auto Thrust and Auto Pilot ECAMs resulted in a return to the airport. While hand flying, the Captain temporarily lost control of the aircraft resulting in a Ground Proximity Warning.

ACN: 2207964 (188 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.Tower : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Cruise
Airspace.Class C : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2207964
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after take off out of ZZZ, we experienced a HYD G RSVR LO LEVEL ECAM. I complied with and accomplished all of the ECAM items as well as the QRH procedures up to but not including the manual extension of the landing gear. I then contacted dispatch via SATCOM and included Maintenance Control and the chief pilot on duty to advise them of the situation. After a brief discussion and input from all involved, I decided the best course of action was to continue to the destination. I then briefed the number one flight attendant regarding the issue and gave her a briefing. As we got closer to ZZZ1, I briefed the passengers regarding the issue and told them they would see emergency equipment on the runway and that I would be stopping on the runway so that we could be towed to the gate due to not having nose wheel steering. Once all those items were complete, I notified ATC and advised them of the situation. I [requested priority handling] and briefed them on requiring a longer downwind so that I would have time to manually extend the landing gear. Once on downwind for ILS XXR, I accomplished the manual extension procedure in the QRH. Once procedure was accomplished and completed, an uneventful landing to XXR occurred. We were met by CFR (crash fire rescue) and Maintenance with a tow crew. The aircraft had an inoperable APU and the tow crew required both engines shut down for the tow back to gate XX. I briefed the inflight crew and the passengers and proceeded to get towed into the gate. The specifics of the events are as follows. Right after selecting gear up, I received a momentary master caution chime along with seeing a HYD G message. The chime and ECAM disappeared. After selecting flaps zero, I received the same momentary master caution which also disappeared immediately. With no active ECAM, I transferred controls to the FO and looked at the synoptic hydraulic page. I saw amber hydraulic quantity along with 3050 PSI in the Green system. Even though it looked like there was no green hydraulic quality, there was no ECAM present. Shortly after passing 3000 feet, the ECAM for HYD G LOW LEVEL latched on to the system. I performed all required items on the ECAM and QRH and contacted dispatch and asked him to include Maintenance Control and duty pilot. I reviewed the additives for failure and crossed checked them with dispatch and duty pilot. We needed to add 1.2 to the max manual breaks distance which kept us well below the available runway for XXR. As a point of discussion with the duty pilot, he recommended I use the failure code associated with the HYDG failure in the performance section of the manual. I ran those numbers with the failure code entered into the perf weight and balance page for landing numbers. The numbers came back identical to the normal numbers. I did the math and applied the 1.2 additive which confirmed the required distance. The next day I discussed this code with my check airman, he advised me the codes are not yet active and that's why the landing distance numbers came back the same. Once I was abeam the airport, I proceeded to finish the QRH procedure for manually extending the gear. The gear manually came down as advertised however, it only took two turns of the lever vice three. I used two hands and could not get past the detent to get the third full turn. After arriving at the gate, the appropriate log book entries were made.

Synopsis

A320 pilot reported a hydraulic system failure during initial climb. Flight crew continued to destination, performed a manual gear extension, and landed safely.

ACN: 2207913 (189 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 13000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2207913
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure, passing through about 13,000â€™, we received ECAMs for G system low pressure, G Reservoir low level, and G Engine 1 pump low pressure. These were verified by info on the HYD page and fault on Eng 1 pump. The FO maintained flying duties and took control of radio calls, while I completed the ECAM and follow-up QRH checklists. We then received an ECAM for PTU fault and completed that checklist. After talking to dispatch and maintenance control via VHF radio, we decided to return to ZZZ. We delayed the return flight by 10 to 20 minutes to burn enough fuel to get below our max landing weight. On about a ten mile final we lowered the gear handle but did not get a down and locked indication, so we performed a manual gear extension. On final, approach control vectored in another jet to land behind us even though we had already told them we were stopping on the runway. After we told them again, that jet was assigned a different runway. Due to lack of nosewheel steering, we stopped straight ahead on runway XXC. ARFF (airport rescue and firefighting) checked us out and didnâ€™t notice anything out of the ordinary. We shut down engines as maintenance and a tug crew arrived. Maintenance noticed hydraulic fluid dripping near the right main landing gear. Maintenance pinned all 3 gear and the tug crew towed us in. Cause: Hydraulic line failure/maintenance practices? ATC internal communications between controllersâ€”(forgot we told them about stopping on runway)

Synopsis

A320 flight crew reported receiving multiple hydraulic system fault messages during departure. Flight crew returned to the airport and stopped on runway due to loss of nosewheel steering.

ACN: 2206965 (190 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 30000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2206965
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing thru, or above Flight Level 300 in climb to 340, before entering ZZZZ airspace. We experienced a #1 engine stall, with engine pressure ratio (EPR) rollback, and an ECAM message â€œ#1Engine Stallâ€. The FADEC did an auto relight and the engine recovered after about 2 seconds without pilot action. Both the First Officer (FO) and I perceived a yaw to the left, I noted the igniters were on, and honestly, I was laser focused on EPR and did not observe the rest of the engine parameters. I transferred controls to the First Officer and called Dispatch/Maintenance Control via satellite communications. While talking with Dispatch/Maintenance the #1 engine experienced another engine stall. I told the First Officer to start coordinating with ATC, to get us turned around, and told him under no circumstances do we cross the oceanic boundary. We all agreed to do a return to ZZZ. Dispatch/Operations/Maintenance were advised of our status, and Maintenance Control advised to do an overweight landing. Cabin crew was briefed, I advised cabin crew we were doing an air return, but, it was not an urgent situation, as both engines were still running. Customers were informed of our return, and a return to ZZZ clearance was coordinated with ATC. No emergency was declared. We ran the divert QRH and the OVERWEIGHT LANDING QRH We complied with all items and procedures in the overweight landing checklist. We experienced 4 or 5 engine stalls in total. We landed, significantly overweight, without delay, as that was the safest course of action. A logbook writeup was done for engine stall inflight A logbook writeup was done for overweight landing This aircraft had experienced #1 engine stalls on a previous flight. The aircraft was out of service in ZZZ1 for, I believe, a couple days while the engine was worked on. It was ferried to ZZZ2 in the morning, and we flew in from ZZZ2 to ZZZ. So we experienced the engine anomaly on the second revenue leg after the engine work was done for the previous engine stall at altitude.

Synopsis

A320 Captain reported returning to departure airport after experiencing multiple stalls in #1 engine.

ACN: 2205842 (191 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2205842

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the takeoff roll, shortly after the "Airspeed Alive" callout, the crew received an "ENG 1 COMPRESSOR VANE" ECAM caution message. A low speed Rejected Takeoff (RTO) was initiated at approximately 52 kts. The crew taxied clear of the runway, made PA to customers and completed non-normal checklists. After coordinating with dispatch, maintenance control and Operations, the crew taxied back to the gate.

Synopsis

A320 flight crew reported on takeoff roll receiving a ENG 1 COMPRESSOR VANE ECAM message resulting in a rejected takeoff and return to the gate.

ACN: 2204884 (192 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2204884
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb out between FL270- FL280 received ECAM - NAV LS TUNING DISAGREE. Captain was pilot flying (PF) and continued to fly and called for ECAM actions. First Officer (FO) ( pilot monitoring (PM) ) acknowledged ECAM and applied QRH procedures for this non-normal and the fault was cleared. After a few minutes we received the same ECAM - NAV LS TUNING DISAGREE followed by Autopilot, Auto throttles and Flight Director (FD) 1&2 disconnected. Also MAP DISPLAY NOT AVAILABLE and GPS PRIMARY LOST message on Navigation display ( ND ). I ( Captain ) continued to fly and called for ECAM actions. FO ran ECAM and proceeded to QRH. We advised ATC and requested radar vectors and headings to avoid weather North of our flight path. When QRH was completed we made the decision to return to ZZZ and [requested priority handling]. Control was then transferred from Captain to FO. We assessed our non normal. I ( Captain ) contacted Dispatch and Maintenance Control via SATCOM. Maintenance asked if we performed QRH for dual FMGC failure. I advised we had but I would do it again. After reviewing QRH with no success we made the decision to use Raw Data as primary navigation aid and made ATC aware of the aircraft malfunctions. I then briefed the flight attendants. PA was made for the passengers about our intention to return to ZZZ due to aircraft maintenance issues. We calculated approach speed using QRH and flight manual and completed our checklist. Requested radar vectors from ATC for a visual approach and the landing was uneventful. When clear of the runway advised flight attendants to remain seated and taxied to the gate. Passengers deplaned and made logbook entries. END.

Synopsis

A320 Captain reported failure of multiple navigation systems during climb resulted in the flight crew using raw data. Flight crew returned to the departure airport.

ACN: 2202724 (193 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight
Ceiling.Single Value : 6000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use : Visual Approach
Route In Use.STAR : ZZZZZ
Airspace.Class D : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2202724
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2586.6
Experience.Flight Crew.Last 90 Days : 71.1
Experience.Flight Crew.Type : 234.5
ASRS Report Number.Accession Number : 2202489
Human Factors : Fatigue
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

While briefing for arrival the first officer as pilot flying, briefed a visual approach to runway XX backed up by the ils. The decision of the visual approach was supported by the reported weather at ZZZ. On arrival into ZZZ we were told by approach control to expect the Ils XX, and I read that back. A short time later the first officer stated he had the field in sight, and we did in fact both have the field in sight, but I didnâ€™t communicate this to atc at that time. After being cleared for the approach we were approaching ZZZZZ while following the ils approach the first officer, assuming we were cleared for the visual approach, stated he was going to cut the corner and then turned to a heading toward ZZZZZ1 and selected open descent to 7000. This was helping to stay further away from the restricted area just beyond ZZZZZ. There was some discussion between the first officer and I at this time of whether we were cleared for the visual or the ils, and I couldnâ€™t remember which one we had actually been cleared for, but the first officer was under the understanding we were cleared for the visual, so we continued on the current path. At the point where we reached 7000 atc contacted us and informed us we were under minimum vectoring altitude and instructed us to climb to 8000 and gave us a slightly different heading which we complied with. Atc then cleared us for the ils approach runway XX in which I read back and we complied. At no point during the course of course of these events did we have any gpws warnings or any other aircraft warnings. Some of the factors which contributed to this event were an unclear communication between the first officer and I, unclear communication between atc and myself, and the new arrival procedures which had just been implemented about a week prior. The crew communication being unclear was partially due to the busy environment and trying to accomplish all of our duties at the same time. I should have communicated the field in sight when we had it, and also clarify whether we were cleared for the visual approach or ils with atc even though we were both very busy with him managing flying the aircraft, and me managing the monitoring of everything. We spent a good amount of time prior to entering approach environment in briefing the new procedures, and were clear on how they should be flown and set up for, but were still unsure of how we were going to be controlled by atc through them.

Narrative: 2

I was Pilot Flying on approach into ZZZ utilizing ILS XX which I briefed as a visual, backed up with the ILS XX approach. I take full ownership of my actions and accountability as the Pilot Flying that performed these actions. To help characterize the events that occurred, this is my perspective to the best of my recollection. After descending on the ZZZZZ arrival with assigned altitudes, my understanding was that we were then cleared for a visual approach to ZZZ runway XX, as I had talked with the Pilot Monitoring about the airport being in sight a few minutes prior, and ceilings supported a visual approach. In addition to this understanding that we were cleared for the visual, and knowing that the new ZZZZZ arrival brings us very close to the Restricted Area and it's associated military activity and terrain, I looked to my right to verify that the airport and surrounding terrain was still in sight, then asked the Pilot Monitoring "are you ok with a right turn - I believe we were cleared for the visual?" There was a little hesitation/confusion but he seemed to agree so I selected a heading towards a final approach as well as a safe altitude to head that direction. (utilizing an Open Descent to 7000' MSL, heading approximately toward ZZZZZ1). Increasing the safety of flight was my motivation for doing this, thinking that it gave us more distance from the Restricted Area and its associated military activity, the terrain within it and turbulence created by the terrain in North wind, as well as the small but increasingly utilized ZZZ1 airport. We then briefly discussed the question of being cleared for the visual but were interrupted by other duties associated with flying the airplane (callouts for configuring if I remember correctly). During that time, ATC notified us that we were below the minimum vectoring altitude, and assigned us a new altitude and heading which I immediately complied with. The rest of the approach was flown uneventfully. At no time were any terrain alert features of the aircraft activated or triggered, nor was safety of flight affected from my perspective. Causal Factors & Lessons Learned: 1) Miscommunication - I should have been more clear in my communication at several points - A) When reporting that I had the field in sight to the Pilot Monitoring B) When listening for our approach clearance C)When asking if we had been cleared for the visual prior to changing course. I also should have requested & waited for confirmation that we were cleared for the visual before changing course, as I have done many times before. 2)New Arrival - I believe the ZZZZZ has been published for about two weeks. I flew it only once before, and remember being uncomfortably close to the restricted areaâ€¦ 3)Proximity to the Restricted Area & ZZZ1 airport - Although I know and respect that when under an IFR flight plan, we should be protected from Restricted Area incursions by ATC as well as NOTAM review, it is always possible to be given a visual at any point near there, at which point I prefer to have more distance from the restricted area and ZZZ1 airport, as well as from the terrain and rotor turbulence caused by it with winds out of the north. If ZZZZZ1 were not located so close to it, I wouldn't have felt the need to vary course away from it. Better NOTAM review on my part, as well as having a note in the ZZZ ATIS regarding the status of R-XXXX as does ZZZ2 ATIS could also help with local situational awareness for inbound aircraft. 4)Approach confusion due to winds - I briefed the assigned ZZZZZ arrival and associated weather with contradictory North winds, which could dictate that we change to the RNAV Visual XY if winds exceeded 10 knots, which would require a different arrival as well since there is no Via for it from the ZZZZZ1. Crossing ZZZZZ1 with the assigned altitude of 9000' instead of the 11,000 on the plate, it momentarily seemed like we might have been getting a rerouting. This did not end up happening, but some of the mental questioning that I did during that time may have distracted me from hearing which approach we ended up being cleared for. This would have been easily avoided with more clear communication on my part with the Pilot Monitoring as well as ATC. Additionally, better arrival and approach options for both runways would help simplify the briefing and flying into ZZZ2. (I do however appreciate the time and effort that it took to get the procedures that we currently have.) 5) Fatigue - My Reserve duty period began at XA:30, and I was called at approximately XB:00 for the XD:00 showtime during the primary Window of Circadian Low. We were then delayed due to maintenance. After this, and at the end of the last flight of the day, The approach then occurred near the afternoon trough in circadian rhythm after nearly 12 hours of duty time. I felt alert and competent throughout the flight, mitigating any effects with hydration and light caffeine intake, but I believe the effects of this timing on the body is well known and worth mentioning. It could also be partly mitigated with Crew Support notifying pilots closer to the minimum XX minute required callout instead of 2-4 hours in advance as happened here and previously, where they wake you up 30-120 minutes earlier than necessary. In conclusion, I sincerely appreciate the opportunity to share my perspective and factors that led to this, and have learned how important seemingly small details in communication are in such a high stakes environment. I would like to reiterate that I take full responsibility and ownership of my actions and I hope to carry these lessons forward in my flying career. I'm happy to help clarify or answer any questions regarding this, I'm eager to learn and improve, and hope to hear any feedback, suggestions or improvements I can make in my flying.

Synopsis

Air carrier pilots reported they were advised by ATC that their aircraft was below the minimum vectoring altitude.

ACN: 2199237 (194 of 2908)

Time / Day

Date : 202501
Local Time Of Day : 1201-1800

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Galley
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 2199237

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Galley
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 2199243

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

CA called the crew to inform that we would divert to ZZZ due to Hydraulic fluids leaking. He then made an announcement to inform the passengers. Once in ZZZ we had a plane swap to a 321 which required another FA. FA3 was ZZZ1 based. The flight crew continued the trip to ZZZ2 and the crew had an overnight in ZZZ1.

Narrative: 2

Flight 350 ZZZ-ZZZ1 diverted to ZZZ2 due to hydraulic issue. Normal landing and deplaning.

Synopsis

A320 crew members reported aircraft diverted due to hydraulic leak.

ACN: 2198658 (195 of 2908)

Time / Day

Date : 202412

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1800

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class D : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Personal
Make Model Name : Stearman
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Plan : None
Mission : Personal
Flight Phase : Final Approach
Airspace.Class D : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2426.23
Experience.Flight Crew.Last 90 Days : 91.93
Experience.Flight Crew.Type : 2426.23
ASRS Report Number.Accession Number : 2198658
Human Factors : Workload
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1522.33
Experience.Flight Crew.Last 90 Days : 105.27
Experience.Flight Crew.Type : 1522.33
ASRS Report Number.Accession Number : 2200324
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft RA
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On approach into ZZZ for runway XXL ATC advised us a Steraman was on a dogleg to final for runway XXR. Myself nor the FO were able to ever identify the traffic visually. As the Stearman turned final for XXR we got a TCAS RA to CLIMB. Performed TCAS escape maneuver and then rolled right into a standard go-around. ATC instructed us to fly heading 040 and maintain 4,000 ft. It was very busy in the traffic pattern and ATC seemed to forget about us. We were headed straight for terrain and I immediately told ATC we needed a vector away from the terrain. We had issues getting the FMC setup for the subsequent approach. ATC then told us to cross ZZZZZ at 5,300 and cleared us for the RNAV XXL Y approach. The AP was on at this point but started to descend below the 5,300 feet. Airspeed also began to decay. I disconnected AP and AT and began manually flying the aircraft to a safe approach and landing.

Narrative: 2

While on the RNAV Y XXL to ZZZ, ATC notified us that a Stearman was on a dogleg to XXR. We did not have the aircraft in sight and never called the aircraft in sight and we were on an instrument approach (in VMC). The RNAV Y XXL approach is offset to the right and at about the moment of the incident, our approach path was crossing the extended centerline for XXR. We were fully configured to land and inside 5 miles when we noticed a TCAS hit a few hundred feet below us, but it was inconsistent and disappeared and reappeared. At about 4 miles from the runway we received the TCAS RA and executed a go-around. After the go-around, I announced TCAS RA to tower. We were switched to approach control who gave us a turn to the east at 4,000 ft and queried the go-around. It seemed like after that point, ATC forgot about us as we headed right for a mountain and CA asked for a turn when we became uncomfortable with the situation. We then executed a visual approach backed up with the RNAV Y XXL approach and landed without further issue.

Synopsis

Air carrier flight crew reported an airborne conflict during final approach with an intruder aircraft. After complying with a TCAS RA the flight crew obtained a new clearance to a safe landing.

ACN: 2198606 (196 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 1801-2400

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2198606
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flt ABCD ZZZ-ZZZ1 around XA:50Z, 90 nm S of Location X, flight attendant reported burnt rubber odor/fume smell in forward galley near coffee maker XXX. Approach descent around 19,000 feet, FA reported the burnt rubber smell/odor/fume is from the forward galley air vent. Followed manual. 800â€™ called FA per manual, FA said still burnt rubber smell from air vent. Sent ACARS message to maintenance. After landing logbook write up, cabin odor sheet and referenced FOM X.XX (on ground). FA 1+2 both reported light head ache.

Synopsis

A320 Captain reported a Flight Attendant reported "burnt rubber fumes" during approach descent fumes.

ACN: 2197577 (197 of 2908)

Time / Day

Date : 202412

Place

Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 166.8
Experience.Flight Crew.Type : 1142.8
ASRS Report Number.Accession Number : 2197577

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 153.3
Experience.Flight Crew.Type : 601.5
ASRS Report Number.Accession Number : 2197595

Events

Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

While cruising at 35000 feet just prior to fix ZZZ we noticed a red dot on our weather software. We took action and made announcements and seat belt sign on and asked the Fas to be seated as well. Approx 3-5 minutes later we started getting turbulence moderate which quickly changed to severe We asked atc for lower and were unable 330 due to traffic and were told 90 seconds before we could descend . We were proactive and had already slowed to Mach .75 . We lost about 200 feet and gained 30-50 knots on airspeed indicator and then gained over 200 feet within seconds . After about 45 seconds I asked atc how much longer to descend and that we were in severe turbulence . Was told 30 more seconds due to traffic in area. The VSI was pitching upwards to 1200-1700 feet per minute however we only gained around 200feet and but airspeed then lost 30-40 knots. Autopilot was on and did a great job of correcting. Seat belt sign was on prior to encounter and after about 30-45 seconds we were given permission to descend to fl330. We were out of severe at approx 34400 feet and once below fl340 it was fairly smooth down to fl330 . We then had Fas check in and no injuries and no passengers were up during the event . I then notified dispatch we had descended to Fl330 as we did not have as we did not have a lot of reserve fuel . I then began messaging dispatch all the relevent info . Wrote up a Mx report and the rest of flight was uneventful.

Narrative: 2

While cruising along at FL350 just prior to reaching ZZZ, I was pilot flying and noticed on our weather software a red dot up ahead at FL360 and lots of orange dots scattered around about 15 minutes in front of us. We had been having off and on smooth to light chop so the seatbelt sign was off at the current time. I turned the seatbelt sign on and the PA was made. We asked ATC about the rides ahead, and were told light to occasional moderate but 330 was a better ride but not smooth. I made the PA "flight attendants take your jump seats in 2 minutes", and started my timer. I then selected speed to Mach .76. The FO asked ATC to descent to FL330, we were told it would be a few minutes due to traffic below us. About 12 minutes after I started my timer we started getting constant moderate turbulence, that then progressed to severe turbulence. The FO asked ATC again about descending to FL330, stating we were in severe turbulence. ATC replied about 30 more seconds due to traffic below us. On the PFD we were showing airspeed fluctuations between a gain and loss of 30-40 knots, fortunately we had a large buffer between VMO with the airspeed at .76 turbulence penetration speed, so we never over-sped the aircraft. We also had an altitude gain and loss of 200 feet, while the vsi was showing rapid and abrupt changes between 1200 ft/min climbs and descents. I believe we were in the severe turbulence for about 2-3 minutes before ATC was able to get us FL330. The FO set the altitude and we immediately descended to FL330. Passing thru about FL345 it started smoothing out to light chop again. I asked the FO when he could get a message typed to send it to dispatch to let him know about the severe turbulence. We leveled at FL330, ATC asked about the ride, injuries and aircraft damage. I called the FAs and asked if everyone was ok in the back. They said everyone was fine as everyone was seated and I told them to stay in their seats and I would do the check in PA when appropriate. We reported to ATC and dispatch. We had a Jumpseater, I asked them if they noticed anything that I missed as jumpseaters usually see the bigger picture when they aren't focused on flying. They agreed with what we had discussed about the altitude and airspeed deviations and large trend vector changes and the time frame of the turbulence. After discussing more with dispatch and looking over the FOM info, we input a maintenance entry for the severe turbulence. I informed the purser that the turbulence we encountered was considered severe and she might get some phone calls or need to fill out some paperwork as well, so to check her manual. The rest of the flight continued uneventfully. Once we landed in ZZZ, after parking I called the Chief Pilot per FOM guidance.

Synopsis

Air carrier flight crew reported encountering severe turbulence resulting in loss of aircraft control. The flight crew regained control of the aircraft and proceeded to their destination.

ACN: 2196505 (198 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 25000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2196505
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Climbing through approximately 25,000 ft., we received a HYD B LO LVL CAUTION, followed by HYD B LO PR just two minutes later. As we performed the appropriate ECAMs and checklists, ZZZ Center was trying to hand us off to ZZZ OCEANIC. At this point, we could see that we'd lost all hydraulic fluid from the BLUE system. In the interest of safety we decided not continue northbound into oceanic airspace. Instead, we decided to turn towards ZZZ. Once we were established southeast bound, we were able to report the problem to Dispatch via SATCOM, but the signal was weak and the call continually dropped. Also ACARS was inoperative due to the ATSU having been deferred. Once communication was reestablished with Dispatch and Maintenance Control, they recommended that we continue to ZZZ1 on the original flight plan. We felt that this was no longer an option as our Estimated Fuel On Board wouldâ€™ve been 5,200 lbs at our ETA. While we were holding north of ZZZ we agreed with Dispatch to change our destination to ZZZ2 as the aircraft was then in a safe condition and the aircraft weight would be less than upon arrival in ZZZ2. We continued to ZZZ2, and the remainder of the flight was uneventful.

Synopsis

A320 pilot reported loss of hydraulic pressure and diversion to nearby suitable airport.

ACN: 2196284 (199 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 564.77
Experience.Flight Crew.Last 90 Days : 96.38
Experience.Flight Crew.Type : 96.38
ASRS Report Number.Accession Number : 2196284
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2026.07
Experience.Flight Crew.Last 90 Days : 169.38
Experience.Flight Crew.Type : 2026.07
ASRS Report Number.Accession Number : 2196304
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Needed to return to gate after pushback due to brake pressure issue. At gate, tug team asked for brakes to be set, Captain responded with "brakes set, pressure not normal you need to chock the plane before you release". Bluetooth headsets cut out and they did not hear the entire exchange. Airplane began to roll back with calls to "set brakes set brakes set brakes". Tow team was able to chock aircraft to stop movement with 5-6 feet. Aircraft stopped before we could activate yellow pump to return brake pressure.

Narrative: 2

Return to the gate after brake pressure issue. Tug crew said to set parking brake. I sent parking brake and relayed to them that the parking brake was set and the pressure was not good. Evidently the Bluetooth headset cut out and he did not hear the last part. The aircraft rolled back approximately 6 feet. The chock was put in by maintenance to stop the aircraft. Brakes did not work. The chock stopped the aircraft before the FO or I could activate the yellow pump to get brake pressure back. No injuries and no damage to the aircraft or any facilities were noted. No contact with any other thing other than the chock.

Synopsis

A320 flight crew reported their aircraft had a brake malfunction and rolled uncontrolled at the gate due to ground crew not chocking the wheels prior to releasing the tug.

ACN: 2194837 (200 of 2908)

Time / Day

Date : 202412

Place

Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach

Component : 1

Aircraft Component : Flap/Slat Indication
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1187.58
Experience.Flight Crew.Last 90 Days : 83.87
Experience.Flight Crew.Type : 1187.58
ASRS Report Number.Accession Number : 2194837
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1263.32
Experience.Flight Crew.Last 90 Days : 122.03
Experience.Flight Crew.Type : 1263.32
ASRS Report Number.Accession Number : 2194838
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Two hours into our flight our flap indicator became visible on the EWD (Engine Warning Display) and displayed â€œxxâ€ under the F. We were also getting a master warning light that kept illuminating without an ECAM. No audio was observed either. We never felt any pitch or speed changes and assumed flaps were still at zero. We called maintenance and they agreed that it was an indication issue. Everybody agreed that continuing to ZZZ was the best option. Upon arrival in ZZZ we slowed and deployed the slats via flap 1 selection. The slats moved to .5 and moved slowly. We decide to take a vector northbound to configure to flaps 3. We ended up with .5 slats and 2.5 flaps with the flap selector at three. We still had no ECAM and had to improvise a procedure assuming that what was indicating was true. We referenced the flight control flaps locked procedure which yielded a flaps three landing and +25 knots to our approach speed. We [requested priority handling] due to the higher approach speed and took the longest runway (XXR) into ZZZ. The landing was uneventful and we taxied to the gate without incident. While dealing with this we switched runways from XXL to the offset approach for XXR. I loaded the approach and extended a course from ZZZZZ. ZZZ approach told us to maintain 8000 until established and cleared approach. I armed the final approach mode and the plane descended before the 8000 foot restriction at ZZZZZ2 because ZZZZZ2 wasn't loaded. ATC noticed our altitude and reissued a clearance to maintain 7000 until established. If I would have done anything differently, I would have stuck with our original plan to land on XXL. We had plenty of runway to safely land in our un-standard configuration and our workload was high with improvising a procedure for our condition.

Narrative: 2

Approximately half way through our flight from ZZZ1 to ZZZ at FL 340 we noticed that the slat/flap indicator was amber and flashing. Underneath the flap symbol indicated 0 and underneath the F symbol had two xx 's. We did not receive an ECAM message. The master warning light was flashing intermittently. The captain talked to Maintenance Control over the radio. Maintenance Control suggested that we continue to ZZZ and suspected a faulty indicator. The captain and I briefed a couple of possible scenarios prior to arrival into ZZZ. We planned to configure the aircraft early to allow time to address any threats. We positioned the flaps to 1 and the slats slowly came down but only to around .5 ( still no ECAM message of any sort). We asked for vectors to the north for some additional time. We slowed to 180 and selected flaps 2. Then slowed to 170 and selected flaps 3. At this point the flaps only went to around 2.5. We [requested priority handling] and asked to go direct to the airport. We decided to stay at flaps 3 and followed the abnormal procedure. We were cautious of our speed and per the performance charts flew at VREF + 25. ATC asked if we would like XXR vs XXL (which we were originally set up for). We entered the RNAV Y into the box. ATC cleared us for the approach and told us to maintain 8,000 until ZZZZZ. We transferred the controls back to the captain. ATC called us asking what altitude we were at and we noticed that we were at 7,400 feet. ATC then said to maintain 7,000 until ZZZZZ1. The landing was very smooth, we didn't require any additional support and taxied to the gate.

Synopsis

A320 flight crew reported a malfunction with the flap control and indication system inflight.