ACN: 2194473 (201 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2194473
Human Factors : Time Pressure

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We were cruising in the clouds at 37000' along a route just north of the ZZZZZ intersection when we encountered moderate turbulence. The radar was on and we were not painting any weather. My Weather App was also showing no convective weather along our route. The turbulence lasted for approximately 30 seconds. I immediately asked the flight attendants to be seated over the PA. The airplane quickly lost energy in the turbulence and then pitched over aggressively to regain speed. We lost approximately 400-500' before the first officer (who was the flying pilot) disengaged the auto pilot and very carefully arrested our sink rate. When clear of the turbulence, we immediately climbed back to 37000' and reported our turbulence encounter and altitude loss to ARINC (Aeronautical Radio, Incorporated). They acknowledged our message. We called the cabin and checked on the crew and passengers and everyone was fine.

Synopsis

A320 Captain reported encountering moderate turbulence resulted in loss of airspeed and altitude with momentary loss of aircraft control.

ACN: 2193112 (202 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class A : ZZZ
Airspace.Class C : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2193112
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were operating flight XXXX on Aircraft X from ZZZ1 to ZZZ2. We got an ECAM for ENG 1 STALL, despite the fact that all systems appeared to be operating normally. We ran the QRH procedure, which called for disconnecting the autothrust and retarding THR 1 to IDLE, which we did, with slightly erratic engine 1 indications. I [requested priority handling] with ATC and requested immediate descent, which we received. After activating the Eng Anti Ice (as per the directions for the current engine option in the QRH), we gradually began applying power to Eng 1. The ENG 1 STALL ECAM appeared again, and the Engine Pressure Ratio (EPR) and Exhaust Gas Temperature (EGT) gauges gave mildly erratic readings. Flight Attendant (FA) 3 reported that she heard unusual noises from that side of the aircraft, but FA 2 did not hear, see, or feel anything unusual. We returned THR 1 to idle and continued with the QRH, which dictated a shutdown and to land ASAP. Coordinating with Dispatch via satphone that we were overhead ZZZ in VMC, they advised us to divert to ZZZ, which we did. I communicated with the Inflight crew the nature of the situation and that we would be landing in ZZZ in 20 minutes. I then made an announcement to the passengers that we had a significant mechanical problem, and were making a precautionary landing in ZZZ. The First Officer (FO) and I set up for and briefed the approach to ZZZ, and ran the appropriate checklists. ATC vectored us for descent and the approach, which concluded in a safe landing. We stopped on the runway so that Crash Fire Rescue (CFR) could inspect the aircraft. After making a "remain seated" announcement, I further explained to the passengers that the firefighters would check the aircraft, after which we would park at gate XX to examine the options for getting them to their destinations. CFR reported no damage or leaks visible from their inspection, and we taxied to the gate and parked the aircraft. The passengers disembarked without injury or incident.

Synopsis

A320 pilot reported an engine stall warning and other erratic readings so they diverted to a suitable airport.

ACN: 2190965 (203 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Component : 1

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 163.5
Experience.Flight Crew.Type : 1750.55
ASRS Report Number.Accession Number : 2190965
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 67.87
ASRS Report Number.Accession Number : 2191012
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On arrival into ZZZ, gear did not extend when the handle was put down. Performed soft go around and received vectors. Attempted to recycle gear handle per ECAM with no luck. Decided to return to ZZZ1 due to longer runway. Gravity extended gear on final to RWY XX successfully. On landing discovered nose wheel steering was inoperative stopped straight on runway and waited for tow team

Narrative: 2

On arrival into ZZZ, gear did not extend when the handle was put down. Captain performed soft go around and we received vectors while investigating. Attempted to recycle gear handle per ECAM with no luck. Decided to return to ZZZ1 due to longer runway. Gravity extended gear on final to RWY XX successfully. On landing discovered nose wheel steering was inoperative stopped straight on runway and waited for tow team.

Synopsis

A320 flight crew reported landing gear would not extend on final approach. Flight crew returned to their departure airport and was able to manually extend the landing gear to land safely.

ACN: 2189941 (204 of 2908)

Time / Day

Date : 202412
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2189941
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

The First Officer was pilot flying on a normal landing in ZZZZ on runway XX. There was about a 3 knot tailwind and the far end of the runway was wet. The landing was normal with medium autobrake set as per the landing data. All was normal until we transferred control. The first officer said it felt like the breaking action was reduced (or words to that effect) just as I took over control. We had proceeded to the last third of the runway where it was considerably wetter than the touchdown and rollout area. When I applied the brakes, I could feel the aircraft was not decelerating. I took a quick look at the triple brake pressure indicator in case the brakes had failed, but the indicator showed good brake pressure so I realize we were hydroplaning. I put the aircraft thrust back into full reverse to prevent a runway overrun. We did stop just in time to make our turn into the taxiway at the end of runway XX without blocking the traffic that was already there waiting to back taxi. I had to leave reverse in place until nearly at a stop to ensure a safe stop. I did not see any limitations exceeded but felt I should submit this report just in case. Following our landing, ATC turned the airport around.

Synopsis

Air carrier Captain reported hydroplaning on the runway during landing and using thrust reversers to a complete stop to prevent a wet runway overrun.

ACN: 2188742 (205 of 2908)

Time / Day

Date : 202411
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2188742

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

One hour into flight at FL350 after just entering ZZZ airspace past ZZZZZ intersection we received a HYD Y RSV LO LVL ECAM. I followed the company procedure and transferred control to the FO and then ran the ECAM actions and completed the QRH procedure. We confirmed loss of yellow system fluid via systems page. I then called dispatch via SATCOM and had MX (Maintenance) control and Chief Pilot on duty patched in. After conferring the decision was made to divert to ZZZ1 airport for recovery. Via HF (no CPDLC on this aircraft) we coordinated the diversion with ZZZ center and were given direct ZZZ1 airport. We consulted the manual to establish a degradation factor for landing performance and then applied that the landing data and consulted against the baseline numbers from the manual. I consulted with and briefed the Purser and made a PA to the customers which the Purser translated into another language. We then ran the Overweight Landing procedure as a precaution but landed under the max landing weight at ZZZ1. ZZZ1 ATC gave us our choice of runways so l asked for XXL. The approach and landing were normal with the loss of a few systems from the failure. We parked the aircraft as per the QRH procedure with no parking brake available. CFR (Crash Fire Rescue) followed us to the gate. Maintenance met the aircraft and we deplaned normally.

Synopsis

Air carrier Captain reported low hydraulic pressure indications and divert to a nearby airport.

ACN: 2186222 (206 of 2908)

Time / Day

Date : 202411
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 83.78
Experience.Flight Crew.Type : 388.15
ASRS Report Number.Accession Number : 2186222
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When landing gear did not extend we had no accompanying ECAM. We recycled the landing gear handle one time and the gear still did not extend. We accomplished a go around with Tower then switched off to Approach Control requesting vectors and a climb to 5000'. We received the L/G (Landing Gear) not down locked after the go around. After completing after takeoff checklist and climb to 5000', the FO read and completed the ECAM. The last step was for L/G gravity extension. We prepared by reading the QRH and made preparations for a tow in to the gate. We performed the L/G gravity extension on long final and made an uneventful landing to Runway XX at ZZZ. We were able to exit the runway at Intersection 1, clear the runway and set the parking break while waiting for a company tow to the gate. We contacted Dispatch and sent in a Maintenance Request code to Maintenance. After the company tow arrived we referenced the tow in checklist from the QRH and got towed to the gate.

Synopsis

A320 Captain reported the landing gear failed to extend during final approach. The Captain executed a go around and completed a safe landing.

ACN: 2186187 (207 of 2908)

Time / Day

Date : 202411
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Lubrication Oil
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 3698.2
Experience.Flight Crew.Last 90 Days : 167.6
Experience.Flight Crew.Type : 3698.2
ASRS Report Number.Accession Number : 2186187
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Human Factors : Time Pressure

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2259.6
Experience.Flight Crew.Last 90 Days : 181.2
Experience.Flight Crew.Type : 2259.6
ASRS Report Number.Accession Number : 2186236
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While operating Aircraft X ZZZ â€“ ZZZ1 we had just departed runway XXR and were climbing out on the ZZZZZ Sid and had been cleared direct ZZZZZ1 with a climb to 16000 ft. Passing 5000 I witnessed the right engine oil qty plummet to 2 qts then bounce between1 - 3 qts. The white title on the engine system page started flashing and the quantity periodically showed amber xx's but hovered around 1 qt. We coordinated with ATC to halt the climb at 6000 ft and to put us in a box pattern so we didn't get too far from the airport in the event engine 2 failed. We coordinated with dispatch on a precautionary return to ZZZ as opposed to continuing 3 hrs to ZZZ1 with the continued indications on Engine 2 and the uncertainty of the oil status. [Priority handling was requested], ATC was given the necessary information and we prepared the aircraft to return to ZZZ for a landing on runway XYL. The FAs were given all the appropriate briefing information and then passengers were briefed accordingly. As we had no ECAM procedure to follow and there is no QRH procedure for the situation and the engine oil temp was not showing an overheat and had pressure plus still generating thrust we elected to leave it running until instructed to shut it down - engine 2. Additionally as we were slightly over the max landing weight we performed the "Landing Overweight" QRH checklist. Dispatch provided us with the necessary landing data and we proceeded to land uneventfully on runway XYL the taxied to gate XX without any further incident. Upon arriving at the gate we conferred with Maintenance and entered the appropriate electronic logbook report. Finally we debriefed with a company representative performing the human factors checklist then debriefed as a crew.

Narrative: 2

On climbout, the captain noticed our oil quantity indicator for the right engine was dropping and beginning to flash as we passed through 5000 MSL on our climb up to 160000 MSL. He immediately requested a level off at 6000 MSL to ATC and a vector to keep us nearby the airport. The oil quantity gauge continued to fluctuate between 0-11 quarts of oil with the remaining indicators showing normal ranges. Uncertain of whether the indicator was accurate or not, we decided the safest course of action was to return to the field in the event the engine failed. I continued to fly the aircraft while the captain coordinated with dispatch and our flight attendants. Once all completing our descent and overweight landing checklists, and ensuring all steps had been completed, we returned to the field and conducted a normal visual approach to XYL for an overweight landing. After completing the landing, and ensuring brake temperatures were within normal parameters, we taxied to the gate without further issues. We then went through our parking procedures and debriefed on the situation with each other.

Synopsis

A320 flight crew reported a drop in the right engine oil quantity shortly after takeoff, resulting in a return to the departure airport.

ACN: 2184010 (208 of 2908)

Time / Day

Date : 202411
Local Time Of Day : 1201-1800

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2184010
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise with the First Officer in the rest room and an FAA inspector on the jump seat, we started getting a high number of ECAMS and CAUTION messages associated with ENGINE 1. The First Officer returned. The QRH led us to put ENGINE 1 at idle thrust. All ECAMS, CAUTION messages and the QRH were taken care of per the SOP. We [requested priority handling] and diverted to ZZZ. ATC, Dispatch, Flight Attendants and the passengers were all advised of the situation. I briefed the Flight Attendant. The cockpit was extremely busy and time was very limited. I started the APU. I set up for the ILS XXR at ZZZ, briefed the approach. Received the ATIS from ATC. Completed the approach and landing checklists. Approximately on a 3-mile final we got an EGT over limit on ENG 1. As the engine was already at idle and I didnâ€™t want a possible engine fire which could have turned the situation into a urgent situation with a possible evacuation and with seconds to landing we shut down ENGINE 1. We landed as normal, I asked CFR (Crash Fire Rescue) to inspect our ENGINE 1 for smoke and fire, which they did, I also asked them to follow us to the gate. The passengers deplaned as normal and I did a maintenance write up in the log book. I called Dispatch, operations and the Chief Pilot on duty and also spoke with 3 of Company's mechanics who happened to be in ZZZ. I saw the maintenance post flight report. There were 10 Warning/Maintenance status messages and 13 Failure messages.

Synopsis

A320 flight crew reported engine failure inflight. Flight crew diverted and landed without incident.

ACN: 2183323 (209 of 2908)

Time / Day

Date : 202411

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Angle of Attack Vane
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 2183323
Human Factors : Training / Qualification
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Ambiguous

Narrative: 1

Field service from ZZZ to ZZZ1 for R/R cap Aoa (Angle of Attack) sensor. Performed ops check pass on phone with Maintenance Control. Fault went away. Once aircraft landed and engines shut down in ZZZ2 fault reappeared. Part replaced in ZZZ2 due to wrong part installed.

Synopsis

A320 technician reported an incorrect angle of attack sensor was installed on an aircraft. The sensor was replaced after the aircraft had been flown.

ACN: 2181836 (210 of 2908)

Time / Day

Date : 202411
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class E : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 14.57
Experience.Flight Crew.Type : 162.55
ASRS Report Number.Accession Number : 2181836
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Aircraft

Narrative: 1

While on radar vectors base turn to final for runway XXL at ZZZ, descending through approximately 4,000 feet MSL 23°C on the ground, VMC, all other conditions normal, I heard a small pop then the Captain said "wow my window just cracked." At first we thought it was the outer pane of his escape window. He had the yellow sunshield extended, we could both see spider webbing glass. We were so close to the airfield with no additional traffic in the vicinity that we decided not to say anything to ATC rather than [request assistance]. Our rationale was that the best thing to do was get the plane out of the air as fast as possible and both focus on those procedures rather than inducing additional distraction or delay. On final, the Captain noted that little pieces of glass were falling off. It was then we realized the inner pane had broken. I mentally prepared myself to take the plane in the event the conditioning worsened, and the Captain was unable to continue flying. He made a smooth landing and we landed uneventfully. Once at the gate, we told the CSR who reported it to maintenance, then we contacted Maintenance Control and wrote it up.

Synopsis

A320 First Officer reporter the inner pane of the Captainâ€™s side window cracked during approach. Flight crew continued approach to a safe landing.

ACN: 2181486 (211 of 2908)

Time / Day

Date : 202411

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 10700

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : ZZZZZX

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 181.3
Experience.Flight Crew.Type : 496.40
ASRS Report Number.Accession Number : 2181486
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 198.58
Experience.Flight Crew.Type : 4025.47
ASRS Report Number.Accession Number : 2181471
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Other Person
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

When on the ZZZZZX arrival into ZZZZ we were told to go direct ZZZZZ1 from the arrival. At the same time and just prior to this, we were also told to descend and maintain 16,000 then 15,000 and finally 10,000. After receiving a phone call from operations, I realized that when we're on our way to 10,000 feet we came within 1192 feet of terrain.

Narrative: 2

While flying the ZZZZZX RNAV arrival into ZZZZ at 12,000 ft. ATC cleared us to descend and maintain 10,000 ft. The crew set the altitude and descended to 10,000 ft. We were not yet to ZZZZZ1 intersection.

Synopsis

Air carrier flight crew reported descending on arrival to an ATC-issued altitude that resulted in them passing close to terrain.

ACN: 2180578 (212 of 2908)

Time / Day

Date : 202411

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1547.72
Experience.Flight Crew.Last 90 Days : 121.38
Experience.Flight Crew.Type : 1547.72
ASRS Report Number.Accession Number : 2180578

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At approx. 10 mins from top of descent aircraft generated green system low pressure ECAM. We completed the ECAM 'to do' items and reviewed the non-normal checklist. Contacted dispatch and requested a phone patch with maintenance control, after concluding call we [requested priority handling] with ATC and advised we would need Runway XX at ZZZ with maintenance standing by. At this point I briefed the lead flight attendant using the brief and followed that with an announcement to the passengers explaining the situation and why there would be emergency personnel standing by. Due to the nature of the failure advised ATC we would need to stop on the runway and be towed to the gate. We were given vectors to an approx. 10 mi final, extended the landing gear using the emergency gear extension checklist and completed a normal approach and landing.

Synopsis

A320 Captain reported a loss of green hydraulic system prior to top of descent. Crew performed checklists and landed normally.

ACN: 2180041 (213 of 2908)

Time / Day

Date : 202410
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class C : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 573.50
Experience.Flight Crew.Last 90 Days : 120.57
Experience.Flight Crew.Type : 573.50
ASRS Report Number.Accession Number : 2180041
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12294.4
Experience.Flight Crew.Last 90 Days : 109.2
Experience.Flight Crew.Type : 6644.7
ASRS Report Number.Accession Number : 2180049
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Primary Problem : Human Factors

Narrative: 1

During pre-departure setup, it was noted on the ATIS that the airport was departing Runway XX. Runway XY was loaded into the FMCG as the departure runway by the FO. Takeoff data was sent for Runway XX and loaded into the FMGC. The oversight was not discovered during FMGC verification or the Before Takeoff Checklist and therefore went uncorrected. After an uneventful taxi to Runway XX, the aircraft was issued a takeoff clearance, departed Runway XX, and instructed to contact ZZZ departure. During this time, the flight director commanded a left turn which was followed by the pilot-flying. Referencing the SID chart for cleared procedure, it was confirmed that the correct procedure was a right turn to a heading of 210 after takeoff. Before correction could be initiated, ZZZ departure immediately announced a "low altitude alert" and directed an "immediate turn to a 180 heading" with further climbs and turns northbound. ATC instruction was complied with and the crew was later handed off to ZZZ center without further incident. The crew was also directed to contact ZZZ approach on a provided phone number and warned about a "possible pilot deviation". After reaching cruise altitude, the event was debriefed. It was at this time that the crew realized the cause for the incorrect flight director guidance was likely the improper runway loaded into the FMGC. This was not confirmed, however, as the FMGC had already removed that information from the flight plan.

Narrative: 2

It was the FO's leg, so I asked him to build the FMGC. Active departure Runway XX, FO programmed Runway XY (which was our landing runway a few minutes earlier). I did not detect the error in my FMGC review (I checked the departure heading of 210°, but missed the incorrect runway). Also, in the Before Takeoff Checklist, both the FO and I missed the runway programming error. Note, the FO requested and programed the correct runway data for Runway XX. After takeoff, the FO made a left turn, in accordance with flight director guidance. I was confused about the left turn guidance did not say anything to fix the error. Almost immediately ZZZ [TRACON] commanded an immediate turn to heading 180°, then subsequent right turns to north.

Synopsis

Air carrier flight crew reported receiving a low altitude alert issued by ATC during departure.

ACN: 2179332 (214 of 2908)

Time / Day

Date : 202410

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2179332
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Attendant
Detector.Person : Gate Agent / CSR
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

At the conclusion of the flight to ZZZ airport, the aircraft was marshalled into and parked at the gate without issue in accordance with SOP procedures. The aircraft was subsequently shut down and both PF and PM performed their "parking" flows. The PF then called for the parking checklist after viewing the "chocks in" signal from the marshaller. I then read the checklist and the CA responded with the appropriate remarks. After the checklist was completed, aircraft secured at the gate, and flight deck door opened, one of the forward FA's was heard shouting "stop, stop, stop." After looking back into the cabin thinking it was likely a significant passenger disturbance, attention was then brought back into the cockpit where the jetbridge was seen in an abnormal position as viewed from the CA side DV window (Direct Vision window). With the gate agent at the control panel of the jetbridge just prior to it stopping movement, it was assumed the jet bridge had malfunctioned and began to retract uncommanded. Shortly after this transpired, one forward flight attendant came into the entryway of the flight deck and said "they just broke the door, now we're stuck." The CA then went into the forward galley and observed the situation while I remained in the flight deck. Throughout this event, there was never any attempt by the marshaller or the wing walkers to signal the flight crew of an issue with the aircraft or jetbridge such as uncommanded movement. After the event, the ground crew was observed all standing forward of the aircraft giving no signs of an issue. Upon consulting with the FA's, it was learned that approximately 4 passengers had already exited the aircraft prior to jetbridge/aircraft movement. Both forward FA's stated they felt the attaching of the jet bridge and subsequent opening of the MCD (Main Cabin Door) seemed abnormally fast and that the gate agent who opened the door seemed rushed. The lead flight attendant stated that they had not even completed their arrival procedures prior to the gate agent knocking on the MCD. I worked to confirm that no passengers or FAs were injured during the event. I also confirmed with the gate agent that none of the ramp personnel including themself were not injured and that none of the passengers who had disembarked the aircraft had fallen in the jetbridge during its possible movement. After completing the parking checklist, we had no internal or external indication of uncommanded aircraft movement. While discussing at a later time, the agent responsible for marshalling the aircraft stated that they never observed the aircraft rolling backward after giving the chocks in signal which was why they never signaled for the flight crew's attention. The gate agent who was operating the jet bridge had stated that the Marshaller was giving their wing walkers the "chocks in" signal and not the flight crew and that they themselves had observed this while the jet bridge was attached. The Gate Agent then stated they would review footage with the airport to confirm and that the flight crew was forbidden from viewing the footage. They stated that they also shouted "stop stop stop" from within the jetbridge which is confirmed by a forward flight attendant while they ran to the control panel in an effort to retract the jetbridge. Both wing walkers stated that they were actively placing the chocks when they noticed the aircraft moving and were not able to immediately chock the aircraft. They did not state if they had stopped the aircraft utilizing chocks or if the aircraft came to rest on its own. After informing Dispatch and Maintenance Control, a plan was made to deplane utilizing airstairs via door R1. This was done without issue. Airport operations, ARFF (Airport Rescue and Firefighting), and other personnel all greeted the aircraft but their assistance was not needed. We did suggest that they may need to carry a wheel chair passenger down the airstairs but the passenger ultimately decided to walk off the aircraft. All passengers and bags were offloaded without further incident and the station was advised to not attempt to move the jetbridge until maintenance arrived. After reviewing the parking procedures in the SOP and the parking procedures in the Manual, and Bulletin, I believe there could be further emphasis placed on the verification of a secured aircraft prior to the attaching of the jetbridge and subsequent opening of the MCD (Main Cabin Door). The SOP states that the wingwalker is supposed to give the jet bridge driver a "thumps up" allowing them to approach the aircraft with the jetbridge. It does not state whether this is before or after chocking the aircraft. On the line, it is also very rare to witness a wingwalker giving this signal. There also could be clarification on the signal used by the Marshaller to allow the wingwalkers to approach the MLG for chocking and the signal used to notify the flight deck that the chocks are in. Often time, complacency causes the signals to be nearly identical. Our procedures call for the release of the parking brake in the event that the brakes are over 350 degrees with the brake fans on, a frequent occurrence, especially in a fully loaded A320. Once we receive the chocks in signal, the brakes are to be released to prevent damage of brake components. Due to this procedure, it is prudent that the ground crew verify that the aircraft is secured utilizing chocks prior to giving the signal that they are placed.

Synopsis

Air carrier First Officer reported, after receiving the chocks in signal, they released brakes and the aircraft rolled backwards uncontrolled.

ACN: 2177635 (215 of 2908)

Time / Day

Date : 202410

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Microphone
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2177635
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had vh1 transmitting ecam flash twice on arrival into ZZZ but cleared itself immediately. There was a stuck Mike all the way to landing. On the ground, no stuck Mike at all and so unsure if it was us. Maint checked and found ops check good. We taxi to rwy to take off, no issues. Told to line up and wait on rwyXXL. We do. Then we are told cancel takeoff clearance, stuck Mike. We never heard (received) a take off clearance. Then we are told to take off and we do with no issues. We had issues on the climb out with stuck Mike, with departure finally telling us that it was us, first time this was confirmed. We still had no idea what was causing it and searched manuals all the way to ZZZ1, but no further ecam this leg. On the downwind for ZZZ1, we thought the culprit was the captains radio trigger switch, which appeared to be rattling. On the ground, no issue taxing in. We wrote up the Mike issue again, this time directing maintenance to the switch. After an hour, they determined the switch was faulty and rapidly failing and disabled it. We had no further issues after this. This issue took 2 legs and 2 separate maint people to determine where and what the inconsistent problem was.

Synopsis

A320 flight crew reported having a stuck mic which prevented them from receiving a takeoff clearance.

ACN: 2176205 (216 of 2908)

Time / Day

Date : 202410

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.AGL.Single Value : 28000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Pneumatic System - Indicating and Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 6.12
Experience.Flight Crew.Type : 6.12
ASRS Report Number.Accession Number : 2176205
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 159.05
Experience.Flight Crew.Type : 2743.03
ASRS Report Number.Accession Number : 2176227
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Climbing through FL280 ECAM Air Eng Bleed 1, began procedure, then Eng 2 bleed ECAM followed immediately with APU bleed leak. Leveled around FL290 [advised ATC] and return to ZZZ and began descent. Landing uneventful in ZZZ.

Narrative: 2

While climbing through approximately FL280, we received an ECAM for AIR ENG 1 BLEED LEAK. While responding to the ECAM, it was noticed that we also had a AIR ENG 2 BLEED LEAK and APU BLEED LEAK. [We advised ATC] and clearance back to ZZZ was requested. The ECAM was actioned and an uneventful return and landing at ZZZ was accomplished.

Synopsis

A320 air carrier flight crew reported receiving Air Engine Bleed ECAM alerts during high altitude climb. The crew returned to the departure airport and landed.

ACN: 2176163 (217 of 2908)

Time / Day

Date : 202410

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2176163
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On our climb out, we got a report to look for traffic at our 3 O'Clock position. My First Officer advised me that he couldn't see the traffic do to his window having several scratches which obstructed his view for locating our traffic. Local Maintenance told me that if it was within limits he was going to sign it off. This is a safety issue due to the inability to locate our traffic because you cannot effectively see out the window because of the severity of the scratches.

Synopsis

A320 Captain reported a maintenance issue with a cockpit window obscuring visibility and restricting the ability to observe air traffic during climb. The window was written up for maintenance action.

ACN: 2176022 (218 of 2908)

Time / Day

Date : 202410
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Door Warning System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 213.05
Experience.Flight Crew.Type : 913.90
ASRS Report Number.Accession Number : 2176022
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We experienced several MX (Maintenance) issues during climb out and then ultimately at cruise enroute from ZZZ to ZZZ1. First issue was a flight control issue we noticed during our multiple level offs during our climb up to cruise. We noted that the plane was flying with a roughly 3 degree turn of roll to the right while in level flight. We tried to rectify the issue by cycling the autopilot and hand flying but the issue remained. The CA (Captain) started a discussion with Maintenance Control and Dispatch to write up the issue. While we worked problem 1 problem 2 emerged. The number 2 brake temp sensor started to behave erratically and was giving us an intermittent hot brakes ECAM and was showing brake temps up to 900C, before the sensor displayed XX and stopped providing input. We discussed our options and contacted Dispatch and Maintenance Control. After working on problem 2 we got another ECAM for Door R Aft EMER Exit. This ECAM has the crew monitor cabin pressure and if necessary perform an emergency descent down to 10,000 ft. After discussing this third problem with Maintenance Control and Dispatch the CA decided to change our destination to ZZZ2 in order to have MX look at the plane. I contacted ATC and notified them of our intentions to change our destination, and that we were not [requesting priority handling]. We chose a lower altitude and then had an uneventful return to ZZZ after Dispatch asked us to fly there instead of ZZZ2. We landed at ZZZ and handed the plane back off to MX.

Synopsis

A320 First Officer reported a flight control issue during cruise followed by two unrelated equipment issues. Flight crew returned to departure airport and aircraft was sent to maintenance.

ACN: 2176003 (219 of 2908)

Time / Day

Date : 202410
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : PCT.TRACON
State Reference : VA
Altitude.MSL.Single Value : 11000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : ANTHM4
Airspace.Class B : BWI

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning
Problem : Design

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 3927.77
Experience.Flight Crew.Last 90 Days : 127.63
Experience.Flight Crew.Type : 3927.77
ASRS Report Number.Accession Number : 2176003
Human Factors : Confusion
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overrode Automation
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Software and Automation

Narrative: 1

While flying the ANTHM4 arrival into BWI, we were descending through 11,800 approximately 5 miles from the JABRR fix, when I pulled the airspeed knob and selected 250 KIAS. The aircraft slightly pitched up and slowed its descent to facilitate the slowdown. When it reached 250 KIAS, we were descending through approximately 11,300' MSL, two miles prior to the fix. The nose then lowered and the aircraft settled into a 1500 FPM descent with no sign of leveling at the hard-coded altitude restriction of 11,000. I intervened and leveled the airplane manually at 11,000. Then, once we had passed the fix, I re-entered NAV mode, with no further issues on the arrival. It is important to note that I have flown this arrival three times in the past two weeks, in three different nose numbers, and I have replicated this behavior EACH TIME, exactly as described here.

Synopsis

A320 Captain reported the aircraft was about to descend below the JABBR crossing restriction on the ANTHM4 arrival to BWI until they intervened and stopped the descent. Captain indicated they had encountered this issue three separate times in different A320 aircraft.

ACN: 2175592 (220 of 2908)

Time / Day

Date : 202410
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 129.5
Experience.Flight Crew.Type : 2580.77
ASRS Report Number.Accession Number : 2175592
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We started our takeoff roll on runway XX. All systems were normal at start of roll. At around 60-75 knots we got an ECAM with a single chime. I quickly evaluated, it was a compressor vane fault on the number 2 engine. I rejected the takeoff. We exited the runway, did an Rejected Takeoff (RTO) QRC followed by the ECAM procedure (negative hot brakes). The ECAM procedure had us shut down the number 2 engine which completed the ECAM. We made a log entry, notified Maintenance and ops. We returned to the gate and debriefed. Aircraft was removed from service.

Synopsis

A320 Captain reported rejected takeoff due to compressor vane fault ECAM message. Flight crew shut down affected engine and aircraft was taken out of service.

ACN: 2171199 (221 of 2908)

Time / Day

Date : 202410
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 157.77
Experience.Flight Crew.Type : 1841.05
ASRS Report Number.Accession Number : 2171199
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 201.8
Experience.Flight Crew.Type : 656.18
ASRS Report Number.Accession Number : 2171214
Human Factors : Confusion
Human Factors : Distraction
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected.Other 
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Ready for pushback, communication is established via headset between Captain and pushback crew. Captain asks tug driver if ready for brake release after before push checklist is complete. Tug driver confirms ready for brake release. Captain releases brakes and directs standby for push. First officer sends for silent push. As ramp is clearing us to push, aircraft starts rolling backwards before Captain relays "brakes released, cleared to push" to ground crew. Tug driver announces "break away". Captain quickly applies brakes and stops uncontrolled roll backwards. Captain asks ground crew if everyone was ok. Tug driver says standby and was silent for approximately 1 minute. Crew cancels pushback clearance. There was no tow bar or shear pin breakage. The aircraft just started rolling backwards. Captain again asks for communication with tug driver. Tug driver responds he thought aircraft was captured but it was not. Captain asks if ready to reset parking brake. Tug driver confirms to set parking brake. Captain asked for clarification of pushback with tow bar or tow barless tug. Tug driver responded he was using the towbarless tug. Captain requested to communicate again when ready for pushback and brake release. Tug driver responded ready for brake release and pushback. Clearance to pushback was again obtained. Captain responds "brakes released, cleared to push". Tug driver repeats back clearance. However, flightdeck crew felt aircraft being slightly lifted just before pushback with brakes released. Earlier this year, flight ops issued guidance that when using the towbarless tug that parking brake remained set until capture. Once capture was complete, parking brake release would subsequently be coordinated. I'm not confident this procedure was followed by pushback crew. Captain asked for clarification on ramp towbarless procedure to tug driver, but his answer simply was he confirmed brake release and was commencing push. Pushback did occur without further incident. There was no incursion or damage due to breakaway.

Narrative: 2

Communication was established per company manual with Captain and ground crew. Ground crew stated they were ready for brake release and push back. Captain stated parking brake released, standby for push. I requested a silent push and received clearance to push from the ramp. As I reached up to turn on the beacon I heard the ground crew announce "BREAKAWAY BREAKAWAY" (as it is acceptable for the FO to monitor the flight interphone per company manual. The Captain, then quickly depressed the brake pedals and stopped the aircraft. It is unknown if the headset failed or not but there was no other communication from flight deck to ground between the point where the Captain said parking brake released standby for push, and when the ground crew said breakaway. The Captain then re-established communication with ground crew and the tug driver said, "I thought I had captured the aircraft, but I did not." I would approximate that the aircraft moved only around 5 to 15 feet straight backwards on the push line. Once again the Captain asked if the ground crew was ready for parking brake release, to which the tug driver replied, "ready for brake release and pushback." The captain said the parking brake released, standby. After the breakaway incident, I canceled our push clearance with the ramp. I then called the ramp back for push and received clearance to push. The captain then told the ground crew, parking brake released cleared to push but at this point it felt like the aircraft was just now being captured. This was a towbarless tug and the aircraft needed to be captured before releasing the parking brake. The captain tried clarifying this to the tug driver but at no resolve. No incident or accident occurred.

Synopsis

Air carrier flight crew reported the aircraft rolled backwards uncontrolled during push back due to the tug becoming disconnected after the crew released the brakes.

ACN: 2169595 (222 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2169595
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Shortly after the application of Flex Takeoff power, received a master caution and ECAM for ENG 1 compressor vane fault. Rejected takeoff in the low speed regime, advised ATC, cleared the runway and made â€œremain seatedâ€œ PA. We completed the ECAM which led to shutting down the engine, and completed the QRH. Notified Dispatch and Maintenance Control. Returned to the gate. Entered the discrepancy in the logbook. The line mechanics were aware because it flagged on their computer at the time of occurrence. Subsequently both FADEC (Full Authority Digital Electronic Control) channels registered a fault and the aircraft was removed from service pending maintenance and a high power engine run.

Synopsis

An Airbus A320 pilot reported an Engine 1 Compression Vane Fault as take off power was applied, resulting in a rejected takeoff and return to the gate.

ACN: 2169089 (223 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 621.87
Experience.Flight Crew.Last 90 Days : 182.6
Experience.Flight Crew.Type : 621.87
ASRS Report Number.Accession Number : 2169089
Human Factors : Workload
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

CA (Captain) was PF (Pilot Flying), FO (First Officer) PM (Pilot Monitoring). In cruise at FL370 we received a HYD G LOW LVL ECAM. The CA directed the FO to action the ECAM items. We completed the steps which lead to securing the G HYD system engine pump and PTU. After ECAM actions were completed the CA directed the FO to fly the aircraft and he created a phone patch with Dispatch and [maintenance] via comms. [Maintenance] verified the G sys was lost but offered no other input or suggestions. Our reception with dispatch was poor and spotty. It was hard to make out much if anything. We knew we were most likely going to divert. CA led an open discussion on options with FO in put. He consulted with dispatch via ACARS. Our closest preferred diversion airport with a longer runway was ZZZ and they were between CAT 1 and CAT 2 minimums. We were halfway to our destination ZZZZ however, the terrain, possible wet runway/decreased braking action and elevation made it a less than preferred choice. CA and FO opted to return to ZZZ1 since we had the gas, the weather was VFR and they had two 11,000 ft runways. Under CA direction, the FO [requested priority handling] with ZZZZ Center and received direct to ZZZ1. Our dispatcher working through ACARS attempted to modified the release with new landing data which caused the aircraft to have an init disagree that made our ACARS intermittent. We were able to receive a message with landing length of 4600ft. After briefing the FAs, the CA resumed PF duties. Both CA and FO reviewed the manual gear extension and tow in procedures. Coordinated tow in with company via center. The CA briefed the approach and supplementary procedures. We landed safely and were towed in. During tow in, CA coordinated with ops regarding procedures for two deportees bound for ZZZ3 onboard. With procedures in position we were given approval to open door and deplane. We debriefed in the flight deck and CA debriefed FAs after flight.

Synopsis

A320 First Officer reported Hydraulic Failure and return to departure airport.

ACN: 2168629 (224 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class C : ZZZ

Component : 1

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2168629
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While approaching ZZZ on the ILS to XXL, we received a , HYD G RSVR LOW LVL message , when moving the flap lever to flaps two. Upon receiving this message, we were on final and continuing the approach. Any immediate threat was not obvious so we continued while following the ecam actions. After lowering the gear handle it was obvious the gear did not come down both audibly and by the position indicator. We performed a go-around and advised ATC of our situation and [requested priority handling]. We were given vectors as we ran through the QRH for the above message. When this was complete, we briefed the Flight attendants, set up for the approach and ran all appropriate checklist. We put the gear down using the manual gear extension per QRH. We then commenced the approach, landed and taxied to the gate without further incident. Upon arriving at the gate, we were met by maintenance where we filled out the appropriate write up in the logbook. Assessing the situation after, I feel that the [request of priority handling] was the proper course of action as we did not know what the outcome of a manual gear extension would be at the time. I do feel, however, that I allowed our situation to be rushed and wish I wouldâ€™ve taken a few more moments to brief the flight attendants more thoroughly about what exactly to expect on landing and what was happening. Before commencing approach, we neglected to arm the approach mode, which caused us a bit more work as I disconnected the auto pilot and auto thrust upon noticing this for landing. We wanted to get the gear down as soon as practical, as we have never done a manual gear extension live, and wanted to know early as possible that we would have a good down and locked indication. Trying to limit the amount of flying time with the gear down, as well as the loud noise we were dealing with, played a major factor in feeling pressure to expedite this second approach.

Synopsis

A320 pilot reported landing gear failed to extend during approach. Flight crew performed manual gear extension and landed safely.

ACN: 2167861 (225 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.MSL.Single Value : 3800

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : F11
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.STAR : FATHE3
Airspace.Class B : MCO

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : F11
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.STAR : FATHE3
Airspace.Class B : MCO

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 34000
Experience.Flight Crew.Last 90 Days : 280
Experience.Flight Crew.Type : 19000
ASRS Report Number.Accession Number : 2167861
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Entered wake turbulence from proceeding aircraft. Wind was light and variable at altitude. Started at approximately 3800 feet., 3 miles behind, and continued until courses diverged. Slowed aircraft to give more separation and turned 0.5 right of course to get out. Still encountered wake in flight levels, distance between planes was 10 miles with 2500 feet difference in altitude. Effect was shaking and slight rocking of aircraft.

Synopsis

A320 Captain reported encountering wake turbulence from a preceding A320 departing MCO.

ACN: 2167015 (226 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 10000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : PHX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : PHX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : PHX
Aircraft Operator : Air Carrier
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Airspace.Class B : PHX

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2167015
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I descended below the minimum altitude restriction at ESDEE intersection on arrival due to inadvertently selecting FLC rather than VNAV after multiple speed assignment changes given by ATC, and a wake turbulence encounter. The Captain and I noticed the error at the same time and I corrected before the next waypoint without a query from ATC. I believe I became distracted after encountering wake turbulence and being instructed by ATC to maintain 280 kt. with only a few thousand feet until having to slow to 250 kt. at 10,000 ft. and then maintain that speed until ESDEE. I leveled the airplane momentarily and must have then selected FLC after leaving the wake event, I donâ€™t actually remember doing so, but it is most likely when I did it. I must have become momentarily too fixated on slowing at ESDEE and our separation with the aircraft in front of us and neglected to watch the path.

Synopsis

A320 First Officer reported descending below an altitude restriction on descent into PHX, citing distractions from a wake turbulence encounter and multiple ATC clearances as contributing.

ACN: 2165544 (227 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2165544

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2166065

Person : 3

Location Of Person.Aircraft : X
Location In Aircraft : Galley
Cabin Activity : Safety Related Duties
Cabin Activity : Deplaning
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 2165667

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During taxi in abeam arrival gate HYD Green RSVR low level ecam. Isolated G hydraulics per ECAM. Verified QRH for follow ups. Ramp Tower reported possible puddle under left main. Began turn into gate. Upon slowing, braking erratic. Stopped aircraft. Ramp Tower reported substantial smoke on #1 main. Stopped and shut down engines. Asked Ramp to monitor and report if any fire was present while requesting Crash Fire Rescue (CFR). Informed Flight Attendants (FAs) of mechanical failure, and instructed them to remain at their stations and to standby for further instructions and that an evacuation while unlikely was possible due to smoke on the exterior of aircraft. Meanwhile ground personnel connected headset and verified no fire present. Ramp personnel towed the aircraft to the gate, and remained in comms with over interphone until CFR arrival. CFR monitored during normal deplaning. Maintenance notified, logbook entry complete. Cause: Mechanical failure.

Narrative: 2

While on the ramp, taxiing to the gate, we stopped the aircraft waiting on the gate area to clear and ramp agents to get into position. We received an ECAM caution "HYD G RSVR LO LVL." I called out the ECAM caution and preceded to complete the ECAM procedure. The Captain checked the QRH for follow ups while I contacted the Ramp Tower and advised them we had a mechanical issue and asked if they could see any fluid leaking or pooling under the aircraft. The Controller said he could see something reflective near our #1 main gear but said it could be a metal plate. I advised him we may have a hydraulic leak. The Captain and I reviewed the INOP systems again noting that the only item was GREEN HYD. I was surprised that there were no secondary failures or other inoperative systems. We decided to attempt to taxi to the gate under power (2 engine taxi) and that we would stop the airplane immediately if we encountered anything unusual with handling or braking. We turned into the gate and the Captain started applying brakes but said something wasn't right. I could feel the brakes skipping a little. I called out for him to stop the aircraft. The Captain set the parking brake and motioned for the ramp crew to tow us in. Almost immediately, the Ramp Tower advised us of heavy smoke from our #1 main gear. I acknowledged and requested that they roll the trucks. The ramp crew connected the tug and interphone and advised us of smoke and fluid on the left gear. We told them to immediately notify us if there was any fire. They pulled us the remaining 20 feet to the gate, advised us it stopped smoking, and we started to deplane. I remained at my seat communicating as necessary to the tug driver, Ramp Tower, and Airport Rescue and Firefighting (ARFF) until the emergency was over. Of note I want to emphasize that we received no additional ECAM messages, there were no secondary failures or additional inop systems shown on the ECAM. In particular NORM BRK and N/W STRG. Cause: Mechanical failure.

Narrative: 3

Aircraft X ZZZ1- ZZZ landed with hydraulic failure and break fluid leaking onto the landing gear, causing the tires to smoke. When we came to a complete stop, the Captain issued a PA stating "THIS IS THE CAPTAIN, REMAIN SEATED." He then informed us that there is smoke coming from aircraft left, and that evacuation was highly likely. Flight Attendant (FA 2) and I (FA4) began our review and discussed proper protected positions. We assessed conditions outside our exits and readied ourselves to command an evacuation. After some time, the seatbelt sign turned off and passengers deplaned safely under ordinary circumstances. This form is being submitted alongside a Company incident report to ensure all parties are informed of the critical safety incident.

Synopsis

A320 crew reported during taxi a hydraulic system failure resulting with the aircraft being towed to the gate.

ACN: 2164850 (228 of 2908)

Time / Day

Date : 202409

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 481.22
Experience.Flight Crew.Last 90 Days : 184.33
Experience.Flight Crew.Type : 481.22
ASRS Report Number.Accession Number : 2164850
Human Factors : Troubleshooting

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
When Detected : Aircraft In Service At Gate
When Detected : Routine Inspection
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Upon receiving SVC INFO SUMMARY via ACARS, I noticed there was a lithium ion wheelchair listed. I consulted the FOM to verify that a report was not required. There was no Dangerous Goods (DG) listed upon ACARS initialization. I did however, after being airborne realize that we never received a mobility device tag from customer service.

Synopsis

Air carrier Captain reported after departure that the required mobility wheelchair tag was not provided to the Captain.

ACN: 2162294 (229 of 2908)

Time / Day

Date : 202409
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2162294
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

This event really started prior to our flight to ZZZ. Uneventful landing in ZZZZ, brakes heated up as they always do in ZZZZ. Turned brake fans on prior to gate arrival. Aircraft boarded for on time departure. After 45 minutes of brake fans on; noted brake temps as 45 40 215 40. Called maintenance to check #3 brake and brake fan. Maintenance said brake fan working, temp is under the 300 limit and signed off as Ops Check OK. After take off received auto brake ECAM which instructs crew to calculate landing distance and use manual baking. No big deal. The FO made a stabilized approach and normal landing on XXL. Around 90kts he applied braking as to make the last high speed turn off. The aircraft abruptly veered 30 degrees to the left, I took the aircraft and was able to stop the aircraft before runway excursion occurred. Slowly taxied to gate a wrote up right side brakes not as effective as left side. On taxi in noted bake temps now indicating 365 295 80 560 Looking back, I should have refused the aircraft in ZZZZ even though it received a maintenance sign off. The mechanics were correct that the brake temp of 215 was legal, but it was obvious that there was something wrong with the temps indicating 45 40 215 40. The airplane was boarded and we were at departure time and it even seemed that the mechanics were under pressure to sign the aircraft off and get it pushed off the gate.

Synopsis

A320 pilot reported temporary loss of control during landing due to number three brake malfunction.

ACN: 2160372 (230 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 141.98
Experience.Flight Crew.Last 90 Days : 141.98
Experience.Flight Crew.Type : 141.98
ASRS Report Number.Accession Number : 2160372

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Upon starting to taxi and turn out from gate a ground vehicle drove directly in front of us without looking or stopping. While this is an all too common occurrence in ZZZ, this was a particularly close call. This problem seems to be getting worse not better.

Synopsis

Air carrier flight crew reported near collision with ground vehicle during taxi out from gate.

ACN: 2159564 (231 of 2908)

Time / Day

Date : 202408

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 87.52
Experience.Flight Crew.Type : 958.67
ASRS Report Number.Accession Number : 2159564

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During Engine acceleration from 50% to FLEX, ENG 1 lagged noticeably behind ENG 2. Once "stabilized" at FLEX, ENG 1 was not producing as much power as ENG 2 and they also sounded very noticeably out of sync. I elected to reject the takeoff instead of trying to investigate. The PF (F/O) also noticed that he was having to use significant rudder to maintain runway centerline.

Synopsis

Air carrier flight crew flying A320 aircraft, reported engine malfunction during takeoff roll. Flight crew rejected takeoff.

ACN: 2158858 (232 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 20000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 21500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 450
ASRS Report Number.Accession Number : 2158858
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb ENG1 EPR MODE FAULT ECAM illuminated on upper ECAM screen. Secondary ECAM AUTOFLT A/THR OFF illuminated with an amber caution. The captain assigned me as the PF, while he assumed the PM duties responding to the ECAM and â€to-do itemsâ€. I continued to climb to FL360, while the captain executed the ECAM procedures. These actions were unsuccessful in resetting the ECAM messages. Upon completing the ECAM procedures, the captain continued with the associated NON-NORMAL procedures in the purser. Running this NON-NORMAL checklist, it became clear and it was confirmed that the number 1 engine was not being monitored by the FADEC anymore. The big threat was making sure we kept the engine within its normal parameters. The checklist guided us to the appropriate max manual N1 settings for the different phases of the flight. We discussed briefly to turn around, but the captain first would contact the company and coordinate the possible return to gate. As the airplane leveled off at FL 360, I made sure the airspeed stabilized well below the â€œred lineâ€ (max speed) and well above the â€œred lineâ€ (min speed) and verified the N1 settings with the tables in the purser. Once the return to ZZZ decision was made, the captain informed ATC we were to return to ZZZ with â€œengine indication problemsâ€ and did not declare an emergency. Initially we were given headings followed by direct ZZZZZ for the ZZZZZ1 arrival. The captain reprogrammed â€œthe boxâ€ with ZZZ as the destination and loaded the arrival. The captain returned to completing the Diversion Guide and made sure we were not overweight for landing, etc. We were given a descent. The aircraft had lost the Auto Thrust capability, but the Auto Pilot was still engaged, so I adjusted the thrust manually for a reasonable descend rate. As the aircraft was under positive control and it was responding as expected, I loaded the expected runway and approach procedures, verified the altitudes, and prepared for the arrival and approach phases of the flight, while the captain now informed the flight attendants and the passengers of the newly developed scenario. Once the captain felt that all parties were informed of the new situation, we came â€œtogether again as a crewâ€ on the flight deck . This is where he informed me of all he had done and I brought him up to speed of my status and the status of the aircraft, altitude and direct to fix, the â€œbox having been loaded and verified by me for the arrival and approach into ZZZ. Once caught up, he resumed the PF duties and I became the PM. We ran the arrival briefing and descent checklist. We discussed the big threat in case of a â€œGo-Aroundâ€, and other potential threats. An uneventful approach and landing followed. Once on the ground, we received a gate and the good news that another aircraft was already assigned to take the passengers to ZZZZ after all. The passengers were informed of the new gate and plane to make sure their expectations were met and would still make it to ZZZZ. After all checklists were completed and the passengers were deplaned, the CA called the chief pilot and discussed the stressful situation we had been in. It was decided, after answering and completing questions that the CA would not continue to ZZZZ, but I would.

Synopsis

A320 First Officer reported engine indicating anomalies and loss of auto thrust capability during climb. Flight crew diverted and landed safely.

ACN: 2157996 (233 of 2908)

Time / Day

Date : 202408

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 146.53
Experience.Flight Crew.Type : 1071.50
ASRS Report Number.Accession Number : 2157996
Human Factors : Situational Awareness

Events

Anomaly.Ground Event / Encounter : Vehicle
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This was day five for me. I had just finished a 4 day trip. I had a good nights sleep prior to this trip. During pushback the airplane made contact with the tug upon disconnect. During pushback, we were cleared to start engines. I told the FO to start the number one engine. It was started before the disconnect from the tow. When the ground crew requested me to set brakes for disconnect the FO inquired if he should start number two because of the anticipated short taxi with no ground traffic. I recall stating â€œparking brake set, pressure normalâ€ and then telling the FO to start number 2. We felt the tug release us and roll backward and then we rolled forward at which time the pushback crew stated over the interphone â€œCaptâ€. I simultaneously hit the brakes but it was too late and contacted the tug. When I looked the parking brake was not set. The ground crew stated we had contacted the tug and we called maintenance.

Synopsis

A320 Captain reported contact with the tug after disconnect when he apparently failed to set the brakes.

ACN: 2156373 (234 of 2908)

Time / Day

Date : 202408

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Technician : 1
ASRS Report Number.Accession Number : 2156373
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Mechanic with approximately one year was working the terminal and had an extra plane dropped in around XA:15. There were not enough people here to work in pairs. There was a hydraulic leak coming from the RH wing outboard and middle canoes. Mechanic was put on the plane to find the cause of the leak. To find the leak they needed to run hydraulics and lower slats and flaps. They were alone 90% of the task and were told to find the cause of the leak before the plane could be sent to the hangar. They spoke to the leads and were told that they could not send the plane there without finding the leak, The mechanic ended up having to stick their head in the lowered slats with hydraulics running and no second person watching the control in the flight deck. At one point Mechanic had a box guy out there to run spoilers so we could check for active leaks there. He was only able to stay for about 20 minutes. After feeling very uneasy and unsafe Mechanic decided it was best for them to make an educated guess about where the leak was coming from, so they did not have to put their head in flight controls with hydraulics running. The supervisor then asked them to go back out there and get better pictures of the leaking component and make sure everything would be clean for the hangar. Mechanic told him I was not going to put my head back up there and that he needs to send the plane down to the hangar. Mechanic wrote the item up as an educated guess because they did not feel safe working alone and putting their self in a dangerous situation. Mechanics are often pressured to do things at the gates, which can result in injury. Leads lets the supervisors dictate maintenance and will not stand up for us or our safety. Supervisors need to use proper channels of communication. If there are not enough people to work a job for safety reasons. The job can wait. AND never should any Mechanic be sticking any body parts in flight controls with hydraulics running. LOCK OUT TAG OUT!

Synopsis

Terminal mechanic reported while working alone troubleshooting a flight control hydraulic leak, was told to find leak with hydraulics pressurized without flight control locks. Mechanic decided to write up an item as an educated guess because they did not feel safe working alone and in a dangerous situation.

ACN: 2155682 (235 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class C : ZZZ

Component

Aircraft Component : Service/Access Door
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2155682
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2155688
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Cleared to T/O on Rwy XX from ZZZ and while waiting for engine spool up, FA1 called the CA to note an exit light flicker on and off. FO pulled power and cancelled T/O with ZZZ Tower. CA confirmed the light was off with FA1 and obtained T/O clearance from Tower. T/O was uneventful until FA1 called again during cleanup following Climb power setting. FA1 advised CA that the 1R door pressurization light was blinking red. FO continued flying and assumed radio duties while CA continued to obtain info from FA1. CA decided to return to ZZZ due to door issue. CA called dispatch and Maintenance Control to discuss options and whether to conduct an overweight landing or burn fuel. After completing a landing assessment and finding the current conditions prohibited a landing option at ZZZ, CA decided to divert to ZZZ1 to avoid weather in the area. Due to runway closures at ZZZ1, CA determined that ZZZ2 was the closest suitable diversion airport. After [requesting priority handling] and taking vectors from ATC to burn down to a suitable landing weight, the FO landed at ZZZ2 without incident. Cause: Future situations like this event that arise while on the runway will result in exiting the runway and waiting on a taxiway to gather more data to confirm that no other issues are present before taking the runway again to depart.

Narrative: 2

Upon taking the runway for takeoff, FA1 called the flightdeck and said exit lights came on. We stopped the aircraft on the runway and informed the tower that we needed to delay. There were no other associated lights in the flightdeck or in the cabin, and after talking with Person A, and the FO, I continued the takeoff. Reaching 2500 feet MSL, the FA1 informed me that the R1 door red light was flashing at times. It would flash and at periods stop flashing. We leveled the aircraft and informed ATC that we might have a pressurization issue and requested to stay close to ZZZ airport. The FO flew and I began communicating with the FA's and passengers. Afterwards, I called dispatch and maintenance to discuss options. The aircraft did not have a pressurization issue, but decided to divert to ZZZ2 and declared a pan pan. The aircraft continued pressurizing for the entire flight. I informed the crew and passengers that we have an issue with one of our cabin doors, but the aircraft was safe. Also informed the passengers that we are not able to continue to ZZZ3 with the condition of the cabin door and we would be headed to ZZZ2 and that I would keep them informed along the way. We climbed to 6000 feet MSL enroute to ZZZ2. We held for an additional 20-25 minutes to burn fuel in order to land below max landing weight and decided to descend to 4000 feet MSL to burn more fuel as we were holding outside of ZZZ2. The FO continued flying and executed the ILS YY at ZZZ2. Upon landing. I taxied to the gate and shutdown with the safety vehicles standing by. Cause: I will not takeoff with any exit or cabin lights that illuminate. It could be associated with an electrical or pressurization issue. And depart the runway and make the decision on the taxiway to allow more time for decision making

Synopsis

Air carrier flight crew flying A320 aircraft reported cabin door malfunction inflight. Diverted and landed uneventfully.

ACN: 2155366 (236 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2155366

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During our approach into ZZZ ILS XXR and after the landing gear extension, we received a Green Hydraulic Reservoir Low Level ECAM alert. We accomplished the immediate ECAM action items and reviewed the affected aircraft system components. I made the decision to land the aircraft given the weather conditions (VMC/dry runway) and the runway length for ZZZ XXR (11,000+ feet), our fuel status at the end of a mid-flight reroute (6,000 lbs), and most importantly, the lack of aircraft performance due to the high density altitude at ZZZ and the inability to retract the landing gear (associated with the hydraulic failure) in conjunction with the high terrain surrounding the ZZZ airport. [Assistance was requested] with the ZZZ Tower and Crash Fire Rescue (CFR) responded to our aircraft. The landing was accomplished without further incident. The aircraft was stopped on the runway and our inflight crew members were instructed to remain seated while we coordinated with ATC/CFR and monitored the affected aircraft systems. The passengers were informed of the situation and the aircraft was towed off the runway and to the gate (due to the loss of steering control associated with the hydraulic failure). An ACARS message was sent to Dispatch informing them of the situation.

Synopsis

A320 Captain reported landing safely following an ECAM alert for low fluid quantity in the Green hydraulic system.

ACN: 2154304 (237 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Gear Down Lock
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2154304
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While on final approach the FO called for the gear down and I (PM) selected the landing gear down. At about 1,500 feet the nose gear and right main gear were down. However, we got an indication that the left gear door was open. That was followed by the left gear showing unlocked on both the ECAM and landing gear panel. At this time, we advised the tower we would like to execute a missed approach. We had several seconds to discuss what we would do and then we went around. We elected to keep the landing gear down. On the go-round, tower, advised us that the left gear did look down. Also while on the go-around, the left gear door closed and the left gear went from unlocked to down and locked and green. Who are you contacted ZZZ departure and they vectored us for a close and left downwind. ZZZ1 Tower was able to visually inspect our gear and didn't say it. It looked down. We continued on and landed visually in ZZZ1 with no further incident. However, after landing, received a caution for landing gear system disagree. I wrote this all up in the logbook and maintenance met the aircraft at the gate.

Synopsis

A320 flight crew on final approach reported main gear not down and locked.

ACN: 2151280 (238 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility : Hail
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2151280
Human Factors : Communication Breakdown
Human Factors : Fatigue
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

My day started at my home, waking up at XA30 to work an ZZZ-ZZZ1 flight followed by a ZZZ2 turn. First two legs were uneventful. After we held for a few minutes, we diverted to ZZZ3 on the ZZZ2-ZZZ1 leg due to weather conditions at ZZZ1 during our landing time. Given that our 12 hour duty period was ending and the refuel and redispatch was quick, the CA and I agreed to a short FDP (Flight Duty Period) extension. During our ZZZ3-ZZZ1 preflight discussion and briefing, we noted the line of convective weather which stretched from the northeast to at least northern Location A and it was moving northeast. Our original route of flight was ZZZ [VOR] ZZZZZ. The WSI (Weather App) and other apps showed a gap in the weather on this route. Once we were airborne, we used the radar, discussion with ATC as well as the aforementioned apps and determined that the gap in the weather on the ZZZZZ arrival was not sufficient and the cells were in excess of our ability to climb over them. We discussed our options and using the information from WSI, apps and ATC, we decided to try for a gap that was approximately 50 miles southwest of our position, near ZZZ4. If it was not passable, then return to ZZZ3. This gap appeared to be larger than the first gap based on what the radar was showing as well as the apps. As we approached the gap, I noticed another aircraft on TCAS 1000feet above us proceeding into the gap we were trying for. I briefed the captain that I intended to turn the aircraft towards the gap and evaluate the radar return and if it changed for the worse from what we were currently seeing, that I would turn around to avoid any severe weather. We also both mentioned that there was no need to push it and that ZZZ3 was a suitable alternate with no significant weather near it. The CA notified the FAs (Flight Attendants) that moderate turbulence procedures were in effect and he made a PA to the passengers to expect turbulence and remain seated with seatbelts on. As I turned the aircraft towards the gap, I slowed the aircraft for turbulence and checked the radar for significant changes in the return. Almost immediately we encountered lightning, turbulence and hail. Both front windshields were cracked. The autopilot remained on throughout the turbulence. We used the non-normal methodology and strategized to work towards a course of action. We noted that neither windshield had cracks on the inside; they were both smooth using the fingernail test. Given this, the QRH noted that there were no limitations for this condition. When we cleared the weather and the situation was stable, the CA [advised ATC] and requested direct to ZZZ1. We descended to 11,000 feet out of an abundance of caution. Additionally, we decided that the CA would take the landing since his windscreen had better visibility/less cracks and we set up for an Autoland approach in case the CA windscreen cracked more. The captain spoke to dispatch and the FAs. I monitored the engine parameters to ensure no rise or fluctuations in vibes or EGTs as I continued to fly. The CA executed an Autoland in VFR conditions, we taxied clear of the runway and had a lengthy tarmac delay (over X hour and XXmins). To my knowledge, no injuries were reported. Cause: Severe weather and potential loss of situation awareness of the true extent of the weather. While the CA and I used all available information at hand to make the decision to try for the gap in the weather near ZZZ4, we didnâ€™t have the specific information regarding the severity of the weather on the exact route of flight. The lesson learned in this situation is to divert to the stated VFR option instead of trying to pick our way through a line of weather.

Synopsis

A-320 First Officer reported a cracked windshield on both cockpit window outer panes during cruise flight in an area of lightening, turbulence, and hail. The crew performed an autoland at the destination and landed safely.

ACN: 2150454 (239 of 2908)

Time / Day

Date : 202408
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : ILS XXC
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2150454
Human Factors : Troubleshooting
Human Factors : Fatigue
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

My day started in ZZZ1 with a scheduled flight to ZZZ. We operated ZZZ1 to ZZZ to ZZZ2 to ZZZ3 (diversion). When we diverted to ZZZ3 due to ZZZ weather, our duty period was not going to allow us to refuel and depart to ZZZ. The FO and I agreed to a FDP (Flight Duty Period) extension. We knew there was a significant line of convective weather along the route to ZZZ. Once airborne, we used onboard radar in conjunction with WSI (Weather App), FDPro, and ATC to evaluate the weather confronting us. Initially we considered a gap a little north of our route. As we neared the target gap, it became apparent that we could not safely pass though. The gap had 70,000â€™ cells on either side, both producing hail, one of which was moving towards the gap. Time for a new plan. Further discussion and data evaluation lead us about 50 miles southwest towards a gap that was larger than the first one. It looked like a passable route based on information we gathered. For a 2nd time, I called the flight attendants and advised them â€œModerate turbulence procedures in effect until I call backâ€, I also made a PA to the passengers that turbulence is expected and they must absolutely remain seated. I expected turbulence to be present, but brief, between cells. The line of storms was wide, but not particularly deep. As we neared the second possible gap, we reiterated to one another that returning to ZZZ3 was a viable option if we were unable to safely pass through. Using all our sources of information, we decided to fly into the gap between storms and continue to ZZZ. Airspeed was decreased and we made a right turn towards the west-northwest to enter the gap. Almost immediately, we encountered turbulence, hail, and lightning. Both front windshields sustained damage. The autopilot did not self disconnect. Non-Normal methodology and the company strategy were utilized per SOP. The inner portion of both windshields were confirmed smooth. The QRH procedure indicated no limitations for this condition. Once we were clear of the weather and the non normal situation stabilized, I [advised ATC] and requested direct to ZZZ. As a precaution, we descended to 11,000. I informed Dispatch of our situation via the app. I briefed flight attendants on what happened and what to expect with our precautionary landing. The CA windshield sustained minimal damage so it was decided that I would assume PF duties prior to approach. After crew discussion, we setup and briefed an autoland approach to Runway XXC. The autoland approach would serve as an added barrier in case the CA windshield degraded and became obstructed during approach, landing, or rollout. Weather was VFR in ZZZ. The autoland approach to XXC was completed. We waited a while for a gate once on the ground. I used the Tarmac Delay app to track this delay and comply with necessary actions. Parked at the gate, logbook entries completed. Report completed. Presently, no injuries reported. Cause: It was a long day. Fatigue was starting to become a factor. I suspect we traveled further south than desired while in the right turn to enter the gap. I considered intervening by turning off the AP (Autopilot) and increasing bank, but I felt the added risk of potentially entering the storm at 67 degrees of bank with AP off outweighed the benefit.

Synopsis

A320 Captain reported a windshield malfunction during cruise after flying near an area of turbulence, hail, and lightning. Both front windshields sustained damage and the flight landed safely.

ACN: 2145897 (240 of 2908)

Time / Day

Date : 202407
Local Time Of Day : 1201-1800

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Rudder Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2145897
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise on Aircraft X at 35,000ft, Mach .78 and smooth ride conditions we encountered an aggressive yawing moment to the left estimating 4-5 degrees. The first officer and I would describe the abnormality as if you were to abruptly step on the left rudder pedal and create a quick, hard yaw to the left. There were no ECAM notifications and we checked that there were no abnormalities to any of the system pages. The only â€œnormalâ€ indication we noticed was whenever the aircraft yawed to the left, the rudder on the flight control page respectively moved with it. The yawing moment started out with a frequency of every 10-15 seconds and our concern grew as the frequency became closer together. The first officer took over the flight controls and radios and I called dispatch to notify them of the abnormality. They instructed us to check both autopilots and manual flight and unfortunately the condition continued. After disconnecting with dispatch, the frequency continued and we decided the safest option was to divert to the closest airport. I spoke with flight attendants asking if they felt the disturbance and they said they felt it but wasnâ€™t that aggressive in the cabin. ZZZ was roughly 50-60NM off our nose so we started the diversion by notifying ATC, going through the diversion checklist, notifying dispatch via ACARS, setting up for the approach, notifying the customers and briefing the flight attendants if condition escalated. Due to it not being a flight control malfunction we did not [request priority handling] but notified ATC that it was an abnormality and we wanted CFR (crash fire rescue) as a precaution. We landed with no incident and taxied safely to the gate in ZZZ.

Synopsis

A320 pilot reported the aircraft began to produce a yawing moment during cruise which continued to increase in frequency. Flight diverted and landed safely.

ACN: 2145662 (241 of 2908)

Time / Day

Date : 202407
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1642.58
Experience.Flight Crew.Last 90 Days : 154.53
Experience.Flight Crew.Type : 1642.58
ASRS Report Number.Accession Number : 2145662
Human Factors : Distraction
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Departed from Gate XX and was given taxi instructions to exit Taxiway 1, left on Taxiway 2 for [Runway] XXR. While exiting at 1, a vehicle on the service road accelerated to cross in front of our moving aircraft, the FO commanded I stop the aircraft and I did to avoid a possible collision. After turning left on 2, the FO went heads down as directed to send a late weight message and to contact Ops as we had not received our final weights. The Ground Controller queried if we would be ready at the end and I responded we did not have final weights. He then gave taxi instructions of turn left on Taxiway 3 and hold short Taxiway 4. I made the left on 3 and then turned right, however, 4 required a left turn so I turned in the wrong direction. I immediately recognized my error, came to a stop and queried ground. He instructed us to hold position on Taxiway 5 and call when ready.

Synopsis

Air carrier Captain reported a critical ground conflict with a service vehicle while exiting the ramp.

ACN: 2144446 (242 of 2908)

Time / Day

Date : 202407
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Component : 1

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2144446
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

The aircraft with the same cockpit and cabin crew was scheduled for ZZZ2 to ZZZ1 and then as ZZZ1 to ZZZ. During [the initial] flight an ECAM for landing gear LGCIU (Landing Gear Control Interface Unit) 1 fault had to be managed by the crew in accordance with QRH procedures. Upon landing in ZZZ1 that fault was addressed by Maintenance Control and cleared in the logbook. Note: the same LGCIU had been previously written up days prior and cleared with a reset. This time a new part was installed though. On the second flight all indications were normal until the beginning of the approach into ZZZ. After landing gear extension we received multiple ECAM messages for landing gear doors open, gear unsafe with a â€˜unlockâ€™ indication. Crew performed a go around to troubleshoot the issue. An additional ECAM for no nose wheel steering was received after the go around. ZZZ Tower was advised of the reason for the discontinued approach and go around and informed of the potential for no nose wheel steering after landing which would necessitate a tow vehicle and temporary closure of the (only) Runway in ZZZ. Flight crew was given the brief items. Cockpit crew worked through QRH procedures and was able to clear the no nose-wheel issue. It was determined that after the second gear extension the only ECAM left was a L/G (Landing Gear) System disagree which points to a discrepancy between the two LGCIUs but had no further consequences on gear extension or steering for a safe landing. On short final all gear indications returned to correct indications and the aircraft landed normally and taxied to the gate. After landing ECAM messages returned and were debriefed with Maintenance Control.

Synopsis

A320 Pilot reported a Landing Gear Control Interface Unit fault with landing gear disagree and no nose wheel steering ECAMs after Maintenance cleared repair. Pilot reported a safe landing after working through QRH procedures.

ACN: 2142101 (243 of 2908)

Time / Day

Date : 202407

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 167.12
Experience.Flight Crew.Type : 210.68
ASRS Report Number.Accession Number : 2142101
Human Factors : Workload
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were flying the ZZZZZ arrival to ZZZ XXR. We were asked to provide our best forward speed on arrival. We slowed to 250 kts below 10K and then slowed to 210 kts at ZZZZZ1. We were cleared for the approach XXR passing ZZZZZ2. Approach asked us to maintain 170 kts until a 5-mile final. Prior to the turn on to the LOC the CA turned off the autopilot and stated he was going to NAV to the final approach course. After arriving on final the CA asked me to arm the LOC and LOC was captured. At that point the CA turned off the auto thrust. At approximately 2000 agl the CA called for flaps 3 APPR arm and managed speed. I did not understand the â€œmanaged speedâ€ and asked him to repeat what he said. I selected flaps 3 and armed the approach. We had the speed brake out and were descending to capture the glide path. We flew below the glide path and still had the speed brake extended. The CA asked again for managed speed, and I selected managed speed. At that point we received a â€œSpeed, speedâ€ warning and a â€œGlide path, glidepathâ€ warning. The CA executed a go around at approximately 1000 AGL. We did not execute the go-around. I was coordinating with Tower for our climb out instructions and was asked why we executed a go around. We were told to fly 190 heading and maintain 3000 msl. The CA asked me to reprogram the approach. While reprogramming the approach XXR the CA asked for the after TO checklist. I read the after TO checklist and said the gear was up when it was down. We cleaned up the flaps on schedule as to not overspeed anything. We then continued for an uneventful approach and landing on XXR ZZZ.

Synopsis

A320 First Officer reported that during approach the aircraft descended below glide path and a speed warning annunciated. Flight crew conducted a go-around and returned to landing.

ACN: 2142087 (244 of 2908)

Time / Day

Date : 202407

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Unknown
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 2246.65
Experience.Flight Crew.Last 90 Days : 114.52
Experience.Flight Crew.Type : 2246.65
ASRS Report Number.Accession Number : 2142087

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Arrived at aircraft, FO light circuit breaker popped. Reviewed MEL and found no pulled or collared circuit breakers expected. Notified maintenance. They request ELB (Electronic Log Book) entry and sent tech out. Maintenance reset cb (Circuit Breaker) and cleared malfunction. Approximately 10 minutes later, FO detected smoke and announced. I confirmed. Notified flight attendants to extract all passengers immediately. Called Ramp requested fire support. Notified maintenance and ops. All passengers deplaned. Debriefed crew and called the chief pilot.

Synopsis

A320 Captain reported detecting smoke in the aircraft after a maintenance procedure at the gate and deplaned the passengers immediately via the jet bridge.

ACN: 2141305 (245 of 2908)

Time / Day

Date : 202407
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2141305
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were cleared to land Runway XX by Tower and the approach was uneventful up until this point. At approximately 2.5 mile final, a Company 737 who was holding short of the runway was cleared for takeoff on Runway XX. The flight crew believed that this was a possible go around situation immediately if the 737 was slow to take the runway. I should note here that our flight and the Company flight were the only two airplanes moving on and around the airport- it was not busy at all. We continued our approach, flying a flaps full configuration with a relatively slow approach speed, while keeping a close eye on the Company 737. As we approached closer to the runway, we believed that we had been backed into a corner by the Towerâ€™s takeoff clearance to the 737 and a go-around would have put us climbing directly over the top of the departing airplane while performing the go around procedure. The Captain chose to continue while I monitored the exact progress of the 737 on their takeoff roll. As we were in the flare, the company aircraft rotated several thousand feet down the runway in front of us, and we touched down approximately 1-2 seconds after their wheels left the runway. The landing, rollout, and runway exit were uneventful. After clearing the runway, I asked Tower â€œDo you think that was close?â€ To which the Tower controller responded, â€œthatâ€™s what we aim forâ€ or something close to that quote. Both the Captain and I have never experienced traffic separation that close, even at the most congested airports we fly to. We were both caught off-guard by the controllers response and as if nothing really occurred in their mind. In hindsight, we probably should have conducted a go-around earlier when we truly realized we were not going to have adequate separation. This could have allowed us to deconflict the two airplanes by turning and climbing earlier than getting into a situation where a go-around would have put us dangerously close and potentially over top of the departing 737. I believe the major causal factor in this situation was Tower clearing the 737 for takeoff while we were on short final to Runway XX. Had they been in a â€œline up and waitâ€ position, then cleared for takeoff, we believe that we would have had comfortable separation from them. However, this was the closest I have ever seen two airplanes almost being on the runway at the same time, and a go-around in a late stage phase of landing would have created a dangerous situation and complete loss of separation between the two aircraft. If the Tower Controller â€œaimsâ€ for that close of spacing between arriving and departing traffic, then I believe that their safety mindset and ATC aircraft separation requirements need to be revisited for safety reasons.

Synopsis

Air carrier First Officer reported they touched down only seconds after a departing aircraft lifted off the runway. Reporter stated that ATC cleared the departure for takeoff with inadequate separation which could have put the two aircraft in unsafe proximity to each other in the event of a go-around.

ACN: 2139090 (246 of 2908)

Time / Day

Date : 202407
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1525.07
Experience.Flight Crew.Last 90 Days : 140.8
Experience.Flight Crew.Type : 1525.07
ASRS Report Number.Accession Number : 2139090
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Primary Problem : Airport

Narrative: 1

We were cleared into XX while taxiing NB on the lane. We rounded the corner and were turning into gate XX lead in line when an approaching truck did not yield. We had to slam on the brakes while turning onto the line. The driver was heads down and texting. Had we not stopped, an accident would have occurred... after the truck was clear of our aircraft, we continued down the line to park safely. The truck's speed never changed.

Synopsis

Air carrier Captain reported taxiing into the gate when a truck failed to yield resulted in the Captain braking hard to avoid a collision. The Captain reported the driver was heads down texting.

ACN: 2139069 (247 of 2908)

Time / Day

Date : 202407
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Engine Vibration Indication
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2139069
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Aircraft

Narrative: 1

Immediately after TO from [RWY] XXR at ZZZ we felt and heard a vibration from our number one engine. We asked to level off to troubleshoot. All engine parameters were normal and when the power was reduced the vibration was not noticeable. While we were trying to ascertain the problem ATC informed us that several aircraft on the ground reported some sort of debris came off the back of our aircraft. We contacted Maintenance Control via Dispatch. We all agreed to return to ZZZ. Engines still indicated normal. I briefed inflight and gave them briefing information due to the uncertainty of our problem. We [requested priority handling] and complied with the overweight landing QRH. We landed normally on [RWY] XYL. Once clear of the runway we had the fire trucks visually check the aircraft and they observed nothing unusual. We then taxied back to gate XX and were towed in. After shutdown I made 2 logbook write up's. One for the initial vibration and reports of debris falling from the aircraft and the other for an overweight landing noting the touchdown weight and speed. Maintenance came to the flight deck and reported we ingested a bird in our number one engine damaging the fan blades and a portion of the rear engine casing which was the debris that was reported. Airport ops found the engine debris after a runway sweep of [RWY] XXR.

Synopsis

A320 pilot reported engine vibration on takeoff resulted in return to departure airport. Post-flight inspection found engine damage due to bird ingestion.

ACN: 2137751 (248 of 2908)

Time / Day

Date : 202406

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component : 1

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2137751
Human Factors : Workload
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2138040
Human Factors : Workload
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Person : 3

Location Of Person.Aircraft : X
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2137753
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Smelled almost like diarrhea. I thought someone just had bad gas throughout the whole flight. Was pretty consistent off and on throughout flight. Asked FA 4 but she could barely smell it and said maybe it was a Pax feet. But the Pax went back to their seat and smell was still present. Finally brought it to front FAâ€™s attention and they came back and said they believed it was stinky sock odor. They smelled the air vents in the galley and it was coming out of here. They had been stuck up front due to electrical going out in whole cabin. So they hadnâ€™t been back to the back the whole flight. I felt it was strong enough for so long that I traded spots with one of them so they could check it out in the back. They both confirmed they believed it was stinky sock odor. I have never experienced it so I did not know that it could smell like diarrhea. Maybe communicate that diarrhea smell could be it too. I was thinking it was just stinky socks smell. If electrical had been working, front crew would have confirmed it sooner and we may not have been exposed as long. If company would have had us go back due to the electrical issue we wouldnâ€™t have been exposed as long either.

Narrative: 2

The flight crew that brought in our Aircraft, Aircraft ABC, said they lost power during flight coming back to ZZZ. Maintenance looked at the aircraft and signed it off saying it was ok to fly. They didnâ€™t take too much time looking at the aircraft or doing checks. Even though there were other aircraft there, since it was around XA:30pm, and planes were back for the evening, they still made us take this aircraft that had lost power. So we were about XX minutes into our flight heading all the way to ZZZ1 from ZZZ when we lost power. The cabin lights went out, and it was dark in the cabin, only the reading lights worked. We lost all communication via the interphone between the pilots and F/Aâ€™s and the PA address system did not work to speak to the passengers. The smoke detectors did not work. The Lavatory call lights didnâ€™t work. There was a loud screeching noise over row XX which was very bothersome to the passengers so we had to move them from that row. Apparently it was the brake fan which may have been broken. We had to pound on the flight deck door to communicate with the pilots and scream through the door to communicate. Our Captain, contacted dispatch. Dispatch told the pilots they were to continue on. Captain informed Dispatch that we were very uncomfortable and we felt unsafe. The Dispatcher said he would get to back to us, that he would speak with the â€œhigher-upsâ€ and see what they say. Meanwhile we could have landed in ZZZ2 that we passed right by or some other bases along the way. As time continued on and the Dispatcher was not expeditious in getting back to us, we were about X hours away from ZZZ1 when we were told that the decision was that we needed to continue on. They had no idea what the underlying problem was of why we lost this power. This was a careless, reckless, negligent and extremely dangerous decision made by someone/people who are not even on the aircraft with us and had no problem endangering all of our lives. Our pleas for help fell on deaf ears. It is a very good thing it was a â€œRed-Eyeâ€ flight. People had no idea what was going on as most everyone was asleep. Had it been another time of the day, there would have been chaos and panic. Upon descent, F/A 2, mentioned she had smelled a foul odor in the back of the aircraft. She said she had smelled it the entire flight long, but she had thought someone was passing gas. I went back and smelled the odor, which was the â€œdirty sockâ€ smell. F/A 3, also went back and confirmed it was the dirty sock smell, which had been present in the back galley the entire flight long. It is imperative that we, as flight attendants, have a voice that is heard and taken seriously with action when we are saying things are not safe, especially with such serious conditions. These are the types of situations that have ended very badly in the past, with no CRM and a nonchalant attitude with those in authority and in the decision making process. This was a [priority handling] and in such serious situations the flight crew should not have to ask for permission from someone who is not on the plane with their own life at stake, whether they can land or not. That decision should lie solely on the crew as it is only the crew who has a true understanding of what the situation is onboard the aircraft and it is their very own lives on the line. The lack of knowledge or care by the maintenance staff has been alarming lately, whether in base or contract maintenance at the outstations. In recent months things have not been fixed, but the problem re-occurs inflight as though nothing was fixed, but just signed off as to hurriedly get the plane out. It is my plea and hope that these matters and warnings will be addressed and resolved speedily and thoroughly before something catastrophic happens.

Narrative: 3

We showed up for a red eye flight to ZZZ1. They delayed boarding due to mechanical. And then all of a sudden they said we could come on board. The mechanic was starting to leave and the captain said hey hey wait up. He said how is everything already fixed? And the mechanic said everythingâ€˜s fine. The captain asked if he pressurize the cabin and mechanic said no he didnâ€™t need to. They had a discussion about the mechanical problems. The mechanic said that they couldnâ€™t duplicate anything, and everything seemed fine. They had mentioned there was a loud noise. The previous crew mentioned that the power went out during the flight, disrupting all communication with the flight deck, and throughout the cabin, and all the lights had gone out. The mechanic said they couldnâ€™t duplicate it and everything seemed fine. We boarded the flight and took off, about XX minutes out all the lights powered down, communication went out, and just the reading lights all turned on. The reading lights were stuck on. We had no galley power, no smoke detector, no flight attendant call lights or bathroom call lights. They all stopped working. The only thing that appeared to work were the reading lights. And after a few minutes the reading lights were able to be turned off manually. So everyone did. Our lead pulled out a cart and knocked on the door and was able to communicate with the flight deck. They called dispatch and dispatch from what I understand had to look into it. It just felt like everything was drawn on and on, and dispatch took a long time to get us answers. We also noticed the squealing after a passenger had mentioned it. We notified the flight deck and they communicated it with dispatch. We were really disappointed that they told us to continue on. We did not feel safe for our passengers, how would they communicate if anything was wrong? Also we couldnâ€™t see in the cabin because it was pitch black. Luckily the lavs still flushed, but if anything was to go wrong in the lavatory, we would not know. And if the smoke detectors werenâ€™t able to ding, we wouldnâ€™t know if there was a fire. Also, throughout the flight I smelt pretty consistently, off and on the smell of diarrhea in the aft galley and rows. I thought it was somebody having gas. I finally asked my coworker if it was her which she said no. She smelt it faintly, but said maybe it was a ladies feet. But the lady moved back to her seat and we still smelled it. I finally went up to the front and asked them to switch me spots so they could go smell because it was just pretty strong throughout the flight and I have never had stinky sock odor before. The lead went back and she came up and confirmed that it smelled like stinky sock odor. We also had FA 3 go back as well, and she also confirmed that it was faint in the air vent in the galley and the lavs. Because the communication was out, we had to pull a cart out in the front and leave it out blocking the front the whole flight and FA 3 had to stay up in the front in case the flight deck needed to open the door for communication. So they were stuck up front the whole flight. If that was not the case, they probably wouldâ€™ve come back sooner and noticed the stinky socks smell, and we wouldnâ€™t had been exposed for as long. I never knew that the stinky stock smell could smell like diarrhea and I had never had an experience with it. It was the perfect storm unfortunately and just a little frustrated that we did not turn back. I think communicating that the stinky stock smell could smell like diarrhea wouldâ€™ve given me more insight and maybe I wouldâ€™ve spoken up sooner knowing that that is what it couldâ€™ve been. Also if we wouldâ€™ve turned back when all communication was lost being exposed to the stinky socks, smell as long as we did could have been avoided. Also, we as a crew felt unsafe, and that should have been taken into consideration as well. I think the time out procedure should come back. When we landed, I wanted to open the back door for fresh air, but again because the communication was not possible I could not contact the flight deck to open the back door.

Synopsis

Air carrier Flight Attendant crew reported the simultaneous failure of the cabin electrical power and a fume event during cruise. After notifying the flight deck crew and communicating with Dispatch the flight was directed to continue to destination.

ACN: 2137615 (249 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2137615
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2137638
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were cleared to push from the gate in ZZZ, I confirmed the brake was released, and the push commenced. During push back we were cleared to start engines, the FO started engine #1. The tug driver informed me the push was complete and asked me to "set brakes". I set the brake, and confirmed it on the upper ECAM display and verified pressure on the triple indicator, I informed him "brake is set, clear to disconnect interphone". The tug was disconnected and began to drive away, I was looking for a salute. The FO mentioned needing a performance date as it had started raining and a runway change. That's when I noticed that we were moving. Initially my thought was that we were getting tugged further, but that couldn't beâ€¦. I looked down, the brake was set, upper ECAM had a green PARK BRK illuminated, the triple indicator was at ZERO. Simultaneously the FO pushed the yellow electric pump, I released the brake, stepped on the toe brakes and re set the brake. All pressure on the triple indicator returned to normal. After the startle of that was over, I looked out the window and saw the ground crew off to the right of the nose holding the steering disconnect pin up and giving me a salute. If I had to guess, I would say we only moved a few feet. At this point I wanted to be sure everyone was clear of the aircraft. I called ramp and asked them to check the camera and be sure we were clear of any ground personal, they said we were. I then called maintenance and explained what happened, I was told if all indications were normal, we were to continue and monitor it. We taxied and departed for ZZZ1. The parking brake was set, and confirmed by switch position, upper ECAM display, and pressure on the triple indicator. At some point after setting the brake, all brake pressure was lost, the brake was NEVER released.

Narrative: 2

During pushback from the gate we were cleared to start engines. I was in the process of starting engine no1 and we realized the airport has changed runway for departure. After pushback completed, I used ACARS for getting performance for Runway XXL wet conditions. I heard brake set and normal communication happening between ground crew and Capt. While I was busy with performance data for Runway XXL departure I heard Captain saying Park brake set and upper ECAM memo showing parking brakes set. At this time, we both realized aircraft movement. When I looked up at the triple gauge brake indication, I saw all pressures zero. I immediately started yellow electric pump and saw accumulator charging and showing correct pressure again. In the meanwhile, Captain had applied brakes and thereafter we rechecked the parking brakes and ensured we were at complete stop. Captain after that spoke to ramp controller to reconfirm that all ground personnel were clear of our aircraft and ramp did say that it appears all clear to him. Captain also spoke to maintenance and since we were showing all correct pressures with no ECAM we were good to proceed. It probably was a failure of accumulator pressures wherein pressures dropped to zero after brake set. For future reference I will look at the pressure gauges of triple indicator whenever parking brakes are set or released irrespective of whatever else I am doing as part of my flows or cockpit workload.

Synopsis

A320 flight crew reported a parking brake pressure failure during pushback resulted in an uncommanded aircraft movement and a loss of control.

ACN: 2137281 (250 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : Mixed
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Instructor
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 8800
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 8600
ASRS Report Number.Accession Number : 2137281

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During flight from ZZZ to ZZZ1 an ecam alert ENG 2 CLOG was presented during Cruise, without observing any additional alerts, we as crew decided to check any information in the manual , then we started preparing an escape route in secondary flight plan, 5 minutes after the initial ECAM a loud bang was heard and and ENG 2 STALL ECAM appeared, immediately Flight Attendant crew called to cockpit to advise an explosion was heard and felt, ENG 2 STALL procedure was done, and Purser informed to prepare cabin, ATC was notified of nature of our situation and we diverted no ZZZ2 being the closest airport and suitable for the failure. We [requested priority handling], APU was started to avoid any excessive electric and pneumatic workload of ENG 1, ENG 2 was tried to be used with caution, resulting in another explosion and high n2 vibrations and abnormal parameters, after landing in Runway XX ZZZ2 we decided to shut down eng 2 and continued to a gate waiting for instructions. Visual inspection after landing revealed smoke/burnt traces or marking in the under side of the right wing, and on the eng 2 outlet

Synopsis

A320 Captain reported an ENG 2 CLOG ECAM followed by a loud bang and ENG 2 STALL ECAM. Crew diverted and shut down engine after landing.