ACN: 2135816 (251 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 800

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2135816
Human Factors : Situational Awareness
Human Factors : Time Pressure

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2136230
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were descending via the ZZZZZ.ZZZ and then provided vectors to join the visual for Runway XX. Vectors included a descent to 3000' and towards ZZZZZ1. As we approached ZZZZZ1 from the east at ~ 3000 and 250 KIAS, we were cleared the visual XX. Due to the non-ideal direction to ZZZZZ1, pilot monitoring (PM) suggested direct ZZZZZ2, and I concurred. Autopilot started the turn to ZZZZZ2 and I input a decent to 2600 with 1500fpm (as we were still high), but due to airspeed the Autopilot overshot and had us on the east side of the river. I decided to correct manually so I disconnected the Autopilot and hand flew the remainder of the approach. I failed to request to have the Flight Director (FD) turned off and fixated outside to fly the runway. As a result, we descended to ~1,000', prior to ZZZZZ3. Being off the anticipated glide path with FD on the Autothrottles added power resulting in increasing airspeed. I elected to disengage Autothrottles at that time. As we were low, a terrian cation annuciation was activated. A couple seconds later, the PM directed a go around (GA) as we descended through 800'. At that time we were configured flaps 20 and gear down. The GA was non-standard as it was initiated at 800', as such I incorrected did not apply standard GA procedures. Instead, I pushed the throttles up (not to TOGA) and started a climb. PM reported the GA and we were immediately given a heading and altitude to fly. With positive rate I directed Gear UP and Flaps 1 passing F speed and directed the PM to bug 200 KIAS. At that time I engaged the Autothrottles. We then received vectors for the visual XX which was flown without event. Cause: Over-reliance on Automation, leading to decay in hand flying skills and scan. I try to handfly with Autothrottles off at least on approach per trip. They are always non-challenging ILS approaches. Last leg and 2 members of crew (1 P and 1 FA) had tight connections. This was discussed in the Arrival Brief. Suggestions: The chain of errors started on the arrival. As pilot flying (PF), I was flying faster than standard in order to give the crewmembers with tight commutes a better chance to make it. Approaching ZZZZZ1, we were still at ~250 KIAS, and as soon as we were cleared the visual, I should have immediately slowed to below flap extension speeds. I also should not have descended at 1500fpm approaching ZZZZZ2, but should have stayed level, slowed and configured prior to descending. On the GA, as we were below 1000' I should have applied SOP

Narrative: 2

We were set up for a left-turn to ZZZZZ1 on the visual XX at ZZZ. We were number 1 for the approach, and the pilot flying had his speed at 250kts. We turned inside of ZZZZZ1 to ZZZZZ4 and the PF tried to slow the aircraft, configure flaps, and fly the visual approach procedure. As the PM, I was heads-down monitoring our flap speeds as I knew we were fast and it was going to be a challenge to configure. Approaching the waypoint ZZZZZ2, the Airbus did not execute the required right turn to the next point, I think it did not hack the turn because we were too fast. We might have had flaps 1 out at this point. The PF kicked off the Autopilot and intervened. He started flying the approach visually, but the approach mode was no longer engaged, so the PF did not have any vertical guidance to go with his visual lateral guidance. I looked up and noticed we were low. I am not sure how low, but based on my experience, probably 200-300 feet low. as the PM and Captain, I told the PF we were low and to climb or level off. We continued to descend with flaps in setting 2 now. The PF called for flaps 3 at 800', I directed a go-around at this point as we were not stable on the approach. We executed a normal go-around and returned for an uneventful 2nd approach. There were several causal factors: 1. Perceived pressure by the PF to land quickly. We were both commuters and had talked about that we had a tight timeline to make our commutes. However, as the Captain, I never explicitly said, "let's keep your speed up" or anything like that. In my own review of the event, it was clear to me the PF felt pressure to be "fast" when it was not required or requested. 2. Energy management was poor. The PF had too much speed as we started our turn towards ZZZZZ1. It was salvageable if we configured early to bleed the speed off, however, as a crew we did not, and the excess energy led to time compression and eventually an unstable approach. 3. Flight Mode Awareness. The PF kicked off the Autopilot, but in the course of the events, also lost the approach mode, which provides vertical as well as lateral guidance. Without the vertical guidance, we carried too-high of a rate-of-descent and ultimately were low on the approach. At one point, the PM had to tell the PM to level off or climb based on visual cues that we were too low. 4. Get-there-itis. The PF continued to press the approach, calling for Flaps 3 below 1000' (unstable) and most-likely cause was trying to hack the approach because if we did a go-around, he might have been concerned we would miss a commute. There was never any direction to "make the commute" but in retrospect, the PF seemed to internalize perceived pressure and this led to a decision to continue a bad approach in order to not do a go-around. Suggestions: A clear discussion about expectations during the arrival brief that there is no pressure at all to get the aircraft on deck "quickly". Second, as the Captain and PM, I could have directed the PF to control his energy better on downwind, to get to Flaps 1 prior to the turn towards the approach. Thirdly, if you are unstable, and we were low which is unstable, then execute the go-around then.

Synopsis

Air carrier flight crew reported unstable approach and CFIT alert. They performed a go around and landed uneventfully.

ACN: 2134753 (252 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Maintenance Items Involved : Repair

Component : 1

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pneumatic Control Valves
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 2100
Experience.Flight Crew.Last 90 Days : 65
Experience.Flight Crew.Type : 193
ASRS Report Number.Accession Number : 2134753
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

The flight started as a normal, routine flight. We reviewed the MELs, most were just placards, but one: 36-11-XX-X specified a unique procedure regarding the high pressure valve. We briefed the OEB61 Issue 1.0: CAB PR EXCESS CAB ALT and determined that if an excess cab alt alert appeared that we "may unduly disregard the CAB PR EXCESS CAB ALT alert, or delay the start of the emergency descent when recard. The captain performed pilot flying duties while I performed Pilot monitoring duties. We followed the procedure for the MEL's procedure on taxi and takeoff, and climbed out normally. We reached cruise at FL340, shortly after cruising for a few minutes, a CAB PR SYS 2 FAULT message appeared on the ECAM. We evaluated the message, compared it with the OEB (Operations Engineering Bulletin), determined the message differed from the OEB. Within about 20 seconds of this message, the Flight attendants called: the captain took the message in which the flight attendants indicated that their emergency light for pressurization was going off, aural warning for depressurization was going off, and it was communicated that they might be having a depressurization issue in the cabin. Following this information, we immediately put our oxygen masks on; the captain indicated for me to take the flight controls, descend, and request that ATC give us a descent and a turn back towards ZZZ. We came to the conclusion that based off the CAB PR SYS 2 FAULT, the aural and visual warnings in the cabin, and the desire for the safest outcome, we agreed to descend to 10,000 feet to assure the safety of our crew and passengers. We were overflying our OA1 point and decided to [advise ATC] to remove any altitude or heading restrictions and allow us to return back to ZZZ rather than diverting towards ZZZ1. We agreed that given the stress of the situation and the possible safety risk of our passengers that reconvening in a holding pattern at 10,000 feet would be the safest decision to safely aviate the aircraft, determine our best course of navigation, and later inform ATC of our plan. Once we reached 10,000 feet, ECAM messages cleared and our pressurization gauges indicated normal operation. We held for about an hour to burn off enough fuel to get the aircraft below the max landing weight. We briefed over our plan, updated our route, and checked in on the well-being of the each other, our flight attendants, and the passenger on board. We notified dispatch, ATC, and made sure all of the appropriate sources were properly notified. Once below our max landing weight of 145,505 lbs, we notified ATC that we would like to head back to ZZZ. They rerouted us to allow us to safely navigate back to the airport at 10,000 feet, and joined the ILS via a few fixes and radar vectors. We landed safely and taxied back to the gate as if it were a normal operation. We debriefed the situation and determined that heading back to the airport was still the safest decision and were ultimately happy with the outcome of our decision-making.

Synopsis

A320 First Officer reported a pressurization alert resulted in a diversion to a safe landing. First Officer also indicated an operations engineering bulletin confused flight crew about the validity of ECAM alert.

ACN: 2134261 (253 of 2908)

Time / Day

Date : 202406

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2134261
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Object
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After pushback and engine 1 start, tug driver informed me that push was complete and asked me to set the parking brake. I set the brake, visually confirmed that the pressure increased on the brake pressure gauge, and the green ecam memo â€œpark brakeâ€ appeared on the upper ecam display. Shortly after the first officer said â€œengine one stableâ€ the tug driver said â€œWhoa, whatâ€™s going on up there, you guys are rolling!â€ Simultaneously we felt and heard a clunk from under the nose. I quickly stepped on the toe brakes but at this point we werenâ€™t moving anymore. I asked if everyone was ok and the tug driver responded that everyone was. I kept my feet on the toe brakes and kept them depressed, while trying to assess the situation. I noted to the first officer that the park brake handle was set, the park brake memo was still displayed, but now the brake pressure gauge and accumulator were now reading zero. I recycled the park brake handle, and after doing so the accumulator pressure immediately jumped into the green range and the brake pressure now read 2000psi for both the left and right wheel brakes. I tried to communicate with the tug driver again and now all I could hear were clicks over the interphone. Shortly after he appeared and gave the signal that his headset was no longer working. After some discussion with the first officer and with our park brake being unreliable at this point, I concluded that the safest thing to do was have the ground crew clear the tug and tow bar out of the area and pull back into the gate under our own power. I coordinated with our dispatcher and the fo coordinated our gate return with ground ops. I pulled back into our gate very slowly and kept a close eye on our brake pressure. After returning to the gate I wrote up the park brake failure and nose wheel striking the tow bar. After getting home and having the benefit of hindsight, I feel like I probably should have shut down the engines and had the ground crew reconnect the tug and tow bar and pull us back into the gate. I could tell that the tug driver was rattled by this incident of our aircraft rolling into the tow bar, and now the situation was complicated by the fact that his headset was no longer working. I just wanted them out of the way, but I realize now that shutting down the engines and being tugged back to the gate would have been the more conservative approach.

Synopsis

A320 Captain reported the aircraft rolled after pushback and before the ground crew had cleared the area, even though the brakes were set.

ACN: 2134026 (254 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Temperature - Extreme

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2134026
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

XA:31 brake release flight ABCE. Last communication from tug driver was â€œclear to start enginesâ€. Shortly after we started hearing the sound of the tow bar being disconnected. First officer stated that we were starting to move forward slightly. I hovered my feet on the brakes as a precaution. Parking brake set when it was clear that they had disconnected and we were moving slightly. At no time was there a â€œpush complete set parking brakeâ€ communication received from the tug operator. The only reason we knew we were done with the push was the tug was pulling away from the aircraft and the pin was being shown to us. No further incident occurred and there was no known physical contact with objects or personnel on the ground. I called a safety timeout and requested communication with tug driver be established from operations. Driver admitted he didnâ€™t do what he was supposed to and apologized. Being concerned that the tug operator didnâ€™t understand the required communication procedures and to hopefully prevent a repeat event in future we requested to speak with a supervisor to let them know what had happened on the spot while it was still fresh. Proceeded to taxi without incident. Temperatures were extreme for ZZZ and ramp heat index was exceeding 100° F. Knowing disorientation from heat exhaustion can occur perhaps this was a contributing factor to the incident. I believe that the tug operator understood the seriousness of the situation and it will not happen with them again in the future.

Synopsis

Air carrier pilot reported the aircraft rolled uncommanded during pushback after the tug driver disconnected but failed to advise the flight crew to set brakes.

ACN: 2133867 (255 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 142.1
Experience.Flight Crew.Type : 1671.67
ASRS Report Number.Accession Number : 2133867
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Dispatch
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During preflight we had received a Planned Dangerous Goods (DG) Summary, so prior to pushback we expected to receive a Final DG Summary. The push crew contacted us via intercom advising they were ready for brake release and pushback. I told him we could not push back until we received and acknowledged the Final DG Summary and asked him if it had been sent. He said he would ask his Ramp Lead. After a couple of minutes, I asked again and he said they were working on it. I asked if the DG had been loaded and was told yes, it was loaded. At that point we sent a report DG message via ACARS and a â€œFinal DG Summaryâ€ printed out. Not realizing this was not in the currently accepted format we sent the DG acknowledgement code message acknowledging receipt of the Final DG Summary, requested pushback, and advised the push crew when we were cleared to push. After pushback and engine start, we were advised by Ramp Control to return to the gate. We were told Load Planning was not satisfied with the way the DG had been loaded and documented so they opened the forward cargo pit and removed it. After reviewing the latest pilot bulletin related to DG, we realized the format of the final summary message we received was incorrect.

Synopsis

A320 Captain reported having to return to the gate as the dangerous goods were loaded and documented in an unaccepted format.

ACN: 2133424 (256 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 113.98
Experience.Flight Crew.Type : 357.47
ASRS Report Number.Accession Number : 2133424
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Ground Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Prior to pushback we received the DG (Dangerous Goods) notification and final DG summary. The FO and I discussed the final DG acknowledgement code message and both thought he sent it at my request. While this was going on, we went through coordinating three reroutes through ZZZZ airspace with ZZZZ ATC and Dispatch while having minimum fuel to accomplish the flight. On taxi out, we revisited the DG goods issue and looked in the ACARS to find the final message but ACARS was having message menu issues. I had him resend prior to takeoff to be sure it had been acknowledged.

Synopsis

Air Carrier Captain reported ACARS had issues with messages during taxi and the flight crew was unsure if the DG acknowledgement code was sent over, and resent it just prior to takeoff as a result.

ACN: 2133215 (257 of 2908)

Time / Day

Date : 202406

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Temperature - Extreme

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 2133215
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 2133217
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
When Detected : Pre-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

We were on Aircraft X. We unexpectedly diverted to ZZZ due to there being an airspace stop in ZZZ1. Aircraft X APU is not working. Making the aircraft really hot. After pulling up to the gate the Captain demanded to the gate agent to let the pax off it was too hot for them to be on the plane while we wait for the airspace to open back up in ZZZ1. It was expected to be a 2.5 hour stop. We ended up boarding approximately 40 mins after letting everyone off. During boarding because the lack of air on the ground the Captain gave us permission to open a couple of aircraft doors to try and get circulation. It wasnâ€™t helping. While waiting on final paperwork and sitting on a fully loaded plane we handed out tri-fold paper towels that we had soaked in ice water to help keep people cool. At this point it was well over 95 degrees on the plane. We get confirmation to close up and get ready to depart. After closing up the doors and going back through demo they were trying to air start the aircraft. After 2 aborted attempts at starting the aircraft we were told by Captain we have to open back up and to disarm our doors. This is when the passengers came from mid cabin having a hard time breathing. Shortness of breath and could hardly breath at all. She mentioned she was about to pass out. While this is happening I exchanged a thumbs up to the gate agent while grabbing oxygen for the sick passenger. As soon as that door opened I immediately asked for her to get paramedics here asap we have a medical. We had to get this lady off the plane where she could get air and breath. With assistance from myself and an RN we helped her move off the aircraft onto the jet bridge next to the boarding door where the nurse was assessing her vitals. At the point we have oxygen on her, ice pack as well as cold towel behind her neck. Paramedics came and took her for further examination. We immediately deplaned the rest of the aircraft as it was not safe to have anyone on board in the heat and no air flow. Passengers began getting vulgar and verbally aggressive at flight crew at this point. Once everyone was off the aircraft the Captain came and spoke with us and said that while everyone was on board the plane was over 100 degrees. Aircraft X has had apu issues for a while now and it needs fixed. Not deferred. For the safety of passengers and crew. We should not have boarded with how hot it was. But Dispatch told gate agents to board.

Narrative: 2

After diverting to ZZZ instead of the scheduled landing in ZZZ1, passengers were declined due to inoperative with inop PACs and no working ground air at the gate. Once aircraft was refueled and air start acquired, passengers were then reboarded into a reading cabin temp of 90 degrees which increased to over 100 once everyone was seated and ready to push back. Aitstart was attempted twice with no success. After the second time, passenger jumped up, panicking they could not breath due it being was too hot and very stuffy and started to hyperventilate. Passenger was immediately then administer oxygen while lead flight attendant also called for disarming of doors due to failed air start. Pax stumbled to door to try and exit. Passenger was held off. Oxygen was administered ,when the door was open, passenger was to assisted immediately onto the jet bridge. Ice pack made and provided to back of neck to help cool pax. another passenger jumped up to assist stated she was an ER nurse took blood pressure. Medical professionals escorted the passenger wheelchair to the area of her further assessment.

Synopsis

A320 flight attendants reported excessive cabin temperatures at the gate due to inoperative APU and no air cooling. Several passengers experienced physiological effects and the aircraft was deplaned.

ACN: 2132971 (258 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2132971
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise we had an ECAM message pop up for Hydraulic low level in the Yellow reservoir. We checked the synopsis page and it was zero. We quickly performed the QRC, and completed the ECAM actions and QRH. We were over waters airspace so we looked at suitable airports if we needed to land. I decided to contact dispatch and Mx (Maintenance) control over Sat phone to formulate a final plan and have them contact the appropriate ATC to [request priority handling]. ZZZZ airport seemed like a more suitable place for us to land based off the aircraftâ€™s condition and the facilities available to us there. I then notified the inflight crew of the situation and used condition yellow. After that I notified our pax (passengers) we had an indication one of the aircraft systems was out and we would be diverting. We had a normal approach and landing but stopped on the runway to get towed off because we had inop nose wheel steering. We had no further problems.

Synopsis

A320 pilot reported receiving a message for low level of hydraulic fluid. Flight diverted and after landing was towed off runway.

ACN: 2132964 (259 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FCU (Flight Control Unit)
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2132964
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We departed ZZZ [airport] at XA:38 and landed ZZZ1 [airport] XC:35 While cruising at FL340 about 15 minutes from the top of decent the aircraft banked into an un commanded bank of 10-15 degrees. The autopilot was engaged and it returned the aircraft back to level flight. When the wings leveled a ECAM alert F/CTL AIL SERVO FAULT was generated. We ran the procedure and the QRH mentioned (NOTE: LAF (Load Alleviation Function) degraded if installed) no other actions were required. I called dispatch on the sat phone and had them patch in maintenance. We discussed the issue and we all felt confident the best course of action was to continue to ZZZ1 where maintenance would meet the aircraft. Just prior to the top of decent we received another ECAM alert this time it was AUTO FLIGHT/FAC1. Again we ran the procedure. With the other flight control issue we decided the safest course of action was to get the aircraft on the ground without any delay. I [requested priority handling] and requested direct to the airport, we discussed the possibility of more flight control issues and formulated a plan. Aircraft configuration was normal and landing was uneventful. CFR (Crash Fire Rescue) was on station and maintenance met us at the gate.

Synopsis

Airbus pilot reported a flight control malfunction at cruise resulting in an uncommanded bank. The flight crew received another flight control EICAS message prior to descent and landed safely at the destination airport.

ACN: 2132718 (260 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 6000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class C : ZZZ

Person

Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 12
ASRS Report Number.Accession Number : 2132718
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X is inbound to ZZZ and requests RWY XX visual approach while we are landing RWY XY. I coordinate with the RWY XY controller for sequence to the intersecting runway and then call ZZZ tower advising them that Aircraft X will be on a visual approach to RWY XX. After coordination is accomplished, I start vectoring and descending Aircraft X to RWY XX. After I point out the location of the airport the pilot asks me if I am setting them up for the RNAV visual approach which was not their initial request. I tell them no and then they tell me that's the approach they need. At this point they are well past any fix I could give them to set them up appropriately, so I have to bring them back around for another approach. This is where my problems start. I tell the pilot by mistake to turn left heading 250 when the more direct route and intended direction of the turn was a right turn heading 250. This unintentional left turn points Aircraft X at a 7,000-foot MVA while he is at 6,000 feet and as soon as I realized what's happening, I issued a low altitude alert and climbed him to 7,000 feet. I initiated the climb prior to him entering the MVA but he was too close to the MVA area, and I lost MVA separation within seconds of my clearance. This was a verbal error by myself which I will just have to be more careful of in the future.

Synopsis

A TRACON Controller reported they turned an aircraft left when they meant to say right and the aircraft flew below the Minimum Vectoring Altitude.

ACN: 2131659 (261 of 2908)

Time / Day

Date : 202406

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2131659
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb, around 20 minutes after take-off, we had HYD Y RSVR LO LVL ECAM. We performed the ECAM actions. As expected, the ECAM switched from LO LVL to LO PR. Since there was no applicable QRH, we went through the manual procedure (HYD Y RSVR LO LVL, since that was the main/root issue). The manual pointed out the possibility of losing yellow accumulator pressure, so we started a timer (as per the manual procedure) to check the accumulator pressure later. It turned out that the accumulator was holding pressure normally. The manual procedure also stated there was a possibility of fuel consumption increased, if the spoiler's panel lost (2 and 4) were not completely retracted. On the F/CTLs synoptic page, the depiction indicated that the failed panels were in the completely retracted position and there was no "FUEL CONSUMPT INCRSD" in the ECAM Status, but we wanted to be sure. We called the Flight Attendant (who was already aware of the issue since she called us to report service was done) and asked if they could verify if the spoilers were flush. Because they were not assertive in their answer, we decided that I should go back to the cabin and verify it. I did it and we ruled out the fuel consumption increase/fuel penalty factor. We kept track of the fuel consumption fix by fix and it was consistent with the flight release. We ran landing performance numbers on our EFB and the results were compatible with landing in ZZZ [airport]. We then decided to continue the flight to destination and communicated the issue and the decision to Dispatch, asking them to make arrangements for our tow to the gate. The captain made a PA to the passengers letting them know we would have to be towed to the gate after landing. Meanwhile, we started to get repetitive ECAM alerts (HYD Y RSVR LO LVL), so we decided to cancel it. Later on the flight, we decided to call dispatch and ask for ARFF (Airport Rescue and Firefighting) services to be standing by, just as a precaution for the (remote) possibility of fire from dripping hydraulic fluid onto the hot brakes. We descended to ZZZ and landed safely, stopping on the runway. We got towed into the gate uneventfully.

Synopsis

A320 Captain reported receiving a hydraulic fluid low level message during climb. Flight returned to departure airport and aircraft was towed to the gate.

ACN: 2129596 (262 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap/Slat Control System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1107.98
Experience.Flight Crew.Last 90 Days : 108.9
Experience.Flight Crew.Type : 1107.98
ASRS Report Number.Accession Number : 2129596

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 145.52
Experience.Flight Crew.Last 90 Days : 145.52
Experience.Flight Crew.Type : 145.52
ASRS Report Number.Accession Number : 2130088

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

This was a Maintenance ferry flight. This occurred at about XA:00am. We flew ZZZ-ZZZ1 and this was the return flight. While tracking localizer inbound to Runway XXC into ZZZ, I called for flaps 2. At that time we received a master caution, with associated single chime and ECAM. Flight Control (F/CTL) Flaps Locked, Alignment Fault. We executed a successful go-around. We [requested priority]. I transferred controls and radios to the First Officer (FO). I coordinated with Dispatch and Maintenance Control. The FO flew delay vectors, so we could run the checklist. We completed a safe landing and followed SOP and QRH procedures.

Narrative: 2

Operated XXXX to ZZZ1 then flew another Airbus from a 2 month heavy check back to ZZZ. Was a late flight back arriving at XA:30am. Flight was completely normal up to Captain (CA) asking for flaps 2. We flew the ILS XXC and at flaps 2 we received a master caution and the associated ECAM for a flight control fault/flaps locked. We then told tower told that we would be going around and [requested priority]. After we switched to departure we asked for vectors to run the ECAM and Flight Manual (FM). The CA transferred aircraft to me as he ran ECAM and talked to Dispatch etc. After briefing what to expect such as the high pitch attitude and that we would be flaps 3 etc. we told approach we would like to come back to shoot the approach back in. We had Crash Fire Rescue (CFR) standing by in case of hot brakes. We landed safely cleared runway and brake temperatures stayed normal.

Synopsis

A320 flight crew reported during final approach they selected flaps 2 and received a master caution ECAM for a flight control fault /flap locked message.

ACN: 2128733 (263 of 2908)

Time / Day

Date : 202406
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb

Component

Aircraft Component : Engine Indications
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2128733
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.No Specific Anomaly Occurred : Unwanted Situation
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight with service to ZZZ [airport], was scheduled to depart from ZZZZ [airport] at XA:43 local time. CA and FO arrived to the airport before report time (XZ:58) and the incoming flight was scheduled to arrive late. When incoming plane finally arrived, deplaning was completed and we had the chance to get to the plane, we were informed by the incoming flight crew that multiple ECAM failures were shown after landing and they suspected a lightning strike during the final approach so they were putting the note in the logbook and that can be checked out by local maintenance. Local Maintenance took control of the plane and performed multiple resets but they had no luck solving the issue. The plane was momentarily put on "out of service" status and we were told that the flight was going to be cancelled. 10 minutes after, the CA received a call indicating that the plane was going to be repaired and we needed to stay at the airport. Around XF:30, we were called to get back to the plane and passengers were sent to board right after. Local operations and Operations Control were pushing our inflight crew to board even without CA "ok to board" instruction given the circumstances. Maintenance was still on board working on tests to make sure everything was functional and the CA made an announcement to passengers stating that we were not taking a plane without making sure that everything was safe and working properly. The plane was finally repaired and after 6 hours total delay, it ended up with 2 MELs showing a SDAC 2 and FWC 2 deferred and the plane was authorized only for CAT I ops. CA and FO went over each MEL in detail and determined that everything was in compliance with the MELs. We started to prepared the cockpit for takeoff and everything was normal during taxi and takeoff roll. We were cleared to take off from RWY XX and follow the noise abatement procedures for the airport. FO was the PF and CA the PM. Then, during initial climb and about 1500 ft both pilots noticed multiple ECAM failures, caution warning and we noticed we lost EPR (Engine Pressure Ratio) 1 indication and ECAM instruction to shut ENG1 off but after checking temperature, N2 we noticed the engine was working properly. All the indications and alerts were coming in and out all the time and it was impossible to determine the root cause of the issue at that precise moment. PF leveled off at 3000 ft. and we decided we needed to get back immediately. FO remained as PF and PM requested an immediate return to the field due to multiple failure indications. We [advised ATC] and received instructions to make a right pattern to get back to RWY XX. During this time PF noticed ENG1 thrust was going up and down and decided to disconnect the auto thrust. That stabilized our speed control. During downwind, PM called inflight and gave them instructions for a code yellow preparation, made a PA to passengers advising we were returning to ZZZZ and advised dispatch. PM set up the approach for landing and PF focused on keeping the plane flying despite the multiple distractions from the ECAM indications. When we felt we were ready for the approach, we requested a right turn direct to ZZZZZ (IAF) for the RNP RWY XX. We were cleared for the approach and then cleared to land and PF hand flown the plane down to the ground uneventfully. Normal taxi to the gate was performed after touchdown. After that, local authorities and personnel approached to the plane and CA took care of the situation.

Synopsis

A320 pilot reported being assigned to an aircraft that had been previously returned to service after a lightning strike. During initial climb, engine instrument indications became erratic and the flight return to the departure airport.

ACN: 2126786 (264 of 2908)

Time / Day

Date : 202406

Place

Altitude.MSL.Single Value : 32000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Cruise

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2126786
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Physiological - Other
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2126787
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Physiological - Other
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight XXXX (ferry flight to ZZZ for pick up) on board 4 Flight Attendantâ€™s (fa) 2 charter reps and Company Mechanic About 3/4 or less FA 3 and I Noticed chip bags that were in a box on expanding and popping Like popcorn . Then we heard one chime and suddenly the lights came on the plane then ALL the oxygen mask came down. I tried to call the flight deck with no answer. We had a Mechanic on board and he was in the front lav when o2 mask fell. With not getting in touch flight deck I was concerned that they were in trouble. I tried to call again and no answer. The Mechanic was trying to open door on keypad then we finally got a call. We already put our masks on by the Captain said put mask on he mentioned 14 and Red pressure. Then quickly hung up. It was easy to figure we needed the masks due to difficult breathing. Chimes kept going off randomly. There was a terrible smell like plastic electrical smell. We had no clue if we were needing to prepare for landing or not. We had zero communication from the flight deck! We had no clue if we were landing else where...nothing! We had no direction and we stayed belted in our seats. Until landing. The FO finally came on and said mask off prepare for landingâ€¦he was very had to hear him. Once landed The Captain came out and said there was no emergency. But we never were told anything while in the air. Communication was as very lacking and we had zero direction. Then he was worried about me opening the door due to pressureâ€¦no warning lights showing on the door. I open the door. We need fresh air. Three times the Captain told us he grabbed the wrong the mask that made him unable to talk to us. But he was not taking it off to communicate with us. So if it wasnâ€™t an emergency then why did he not want to remove it? Captain said they were pushing wrong buttons...pushing WRONG buttons! Captain said did not declare emergency because it worked itself out when we go to 10,000 feet! And who ever made the announcement we could take our mask off never relayed we were safe and no emergency landing...we knew nothing until we actually landed which was a hard landing to add to everything. FA3 said he had hard time breathing, FA 4 said she was getting leg cramps and Fa2 ears were poppingâ€¦prior to mask falling. I for one was having difficulty putting my seat belt on and FA3 put it on meâ€¦I guess I was so busy trying to call the flight deck I did not "think" about myself. As a personal note, I was thinking â€œis this it? I was trying to be brave and upbeatâ€¦but zero communication as to are we safe or something...the not knowing was stressfulâ€¦not only me but the others also. We told the Captain we are going back to ZZZ1 and he said do what you have to do. I think he assumed he was going to resume the trip. Communication was needed and not sugar coat and say it was not an emergency when Masks are falling, breathing issues and rapid descending Cause: Loss of cabin pressure rapid decent.

Narrative: 2

At approximately XA30 we were at cruising altitude of approximately 32,000 feet when we felt the plane slowly start to descend. Then we starting hearing the potato chips start popping that were in a box on seat XX. The more bags of chips starting popping more frequently and we felt our descent becoming more steep and faster. Lead and myself kept asking each other why are all the chip bags popping, which was very unusual to us. As this is happening we started getting random single chimes, which isn't normal for us or what we have been trained to listen for. We received probably 5 random single chimes. On the last single chime all the lights in the aircraft went to bright and the oxygen masks deployed. When this happened lead and myself looked at each other and said what is happening. I told lead to call the flight deck. Lead initiated a call to the flight deck but it went unanswered. When they didn't answer we decided we needed to put on our oxygen masks for safety. As we are doing this we can still feel the plane in a fast and steep decent. During this process before we got our masks on my chest felt very heavy as if someone was sitting on my chest. Once we had our masks on the Captain called. Upon answering the phone the pilot stated over the phone put on your masks red pressure and hung up the phone on us. We made sure we were seated and buckled in our jump seats and we had our oxygen masks on. A few minutes later the pilot came over the pa and stated we could take off our masks. Looking outside it seemed we were approximately at 10,000 feet. Once we landed we could smell a burning smell in the plane like electrical. We pulled to a stop disarmed our doors to prepare to open the doors. The Captain came out of the flight deck and stated we were never in an emergency but that they couldnâ€™t answer our phone call at the time as they had their masks on and if they would have taken them off it would have been a serious issue. I questioned the Captain if it wasn't an emergency and we werenâ€™t loosing pressure why were we advised to put our oxygen masks on and why did he have his masks on. He stated we never lost pressure we weren't in an emergency. I do have a screenshot of the comments from flight XXXX stating lost cabin pressure in FLT masks dropped. I will attach the screenshot to the report. If we weren't in an emergency why did we descend so quick? Why did our oxygen masks drop and why were we told to put them on? I do know we have an emergency button that when pressed it causes all the light to go bright for an emergency evacuation. The pilots had to push this button in the flight deck as we didn't in the cabin and it caused our oxygen masks to drop. The pilots failed to give us the 6 chimes signaling an emergency so we could prepare. I personally feel the was a breakdown in communication from the flight deck to the cabin crew causing undue stress on us mentally and physically. Cause: Loss of cabin pressure rapid decent.

Synopsis

A320 Flight Attendants reported a loss of cabin pressurization during cruise that was exacerbated by poor flight deck-cabin communications. The flight completed a safe landing at destination.

ACN: 2125888 (265 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 28500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Direct
Airspace.Class B : ZZZ

Component

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2435.83
Experience.Flight Crew.Last 90 Days : 146.53
Experience.Flight Crew.Type : 2003.40
ASRS Report Number.Accession Number : 2125888
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Time Pressure

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 129.82
ASRS Report Number.Accession Number : 2125908
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Climbing through approximately 28,500 ft. MSL on our initial climb to an assigned cruising altitude of FL320, the aircraft abruptly yawed left and then recentered itself. Autopilot two was engaged and remained engaged. Initial thoughts weâ€™re an engine one failure, however, all engine indications were normal. Approximately, 10-sec after the abrupt yaw left an 'Integrated Drive Generator (IDG) 1 OIL LOW PRâ€™ ECAM appeared on our lower ECAM screen. The Captain, also the pilot monitoring (PM), ran the ECAM steps and then verbally reviewed the Notes, Warnings and Cautions. As the pilot flying, I requested a level off with ATC, stopping our climb at FL300 and started the APU. The Captain and I briefly discussed the abnormal yaw we felt prior to the IDG ECAM and felt that was an abnormal indication, possibly indicative of some greater issues with the aircraft and we should consider diverting the aircraft for further inspection. To back up this judgement and our thoughts, the Captain contacted Dispatch and was patched though with Maintenance Control for a short discussion. Since we were due east on our flight plan route and would be overflying ZZZ, diverting immediately was the best course of action. Since the aircraft was stabilized and we had the APU providing additional electrical power, we elected not request priority handling. The Captain then notified the flight attendants (FAs) of our situation, whom also felt the abrupt yaw and initially lost galley power, agreed with the decision and began preparing the cabin for landing. The Captain then made a PA announcement as to the generator situation and the course of action we would be taking to divert to ZZZ in an abundance of caution. We then coordinated with ZZZ Center for a destination change due to loss of an engine generator. We reviewed the overweight landing procedures and I made a soft landing on XXR at a GW of 149k lbs at a descent rate of less than 100ft/sec. We then taxied to the gate and post flight met Maintenance to discuss the situation and ensure they had all maintenance requests and notes needed for their corrective action.

Narrative: 2

Climbing through approximately FL270 the aircraft yawed abruptly left then straightened out. (I was unaware an Integrated Drive Generator (IDG) failure could do that) My first reaction was that an engine had failed. Engines looked normal. Within a few seconds, ECAM ELEC IDG 1OIL LO PR appeared. I handed the First Officer (FO) the radios and aircraft and we accomplished the ECAM and disconnected the IDG. We started the APU for redundancy and the APU picked up AC bus one from generator two. I conferred with Dispatch and Maintenance Control and we made the decision to divert to ZZZ. FO and I discussed [requesting priority handling] and did not because redundant electrical power was available. We remained ready to do so if needed. ATC was very helpful and the arrival was uneventful and expeditious. We had to land over weight by approximately 6000 lbs. Dispatch was helpful to send landing data. The landing and taxi to gate were also uneventful. The IDG failure and overweight landing were written up. We also spoke to Maintenance about details. It can take a long time to get ahold of Dispatch and even longer to then get Maintenance Control on. 150 miles out is probably too close to initiate. They couldnâ€™t really help much and it was a lot of work for the FO to fly, talk to ATC and program while I was talking. It would have been safer for me to be with the FO from top of descent on down. The â€˜stuck micâ€™ ECAM is not helpful in this case either.

Synopsis

Air carrier pilot crew reported the aircraft began to yaw after an integrated drive generator failed on the left engine.

ACN: 2125295 (266 of 2908)

Time / Day

Date : 202405

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 132.1
Experience.Flight Crew.Type : 690.57
ASRS Report Number.Accession Number : 2125295

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying. We had just begun our descent with all briefings and checklists completed. Between 30 and 29,000 feet I felt and heard a small jolt. I thought the Captain had dropped a water bottle on the floor, but I also felt almost imperceptible yaw. We immediately looked at the ECAM and saw that engine 2 had automatically reduced itself to idle, with all other parameters normal at that instant. However, this was quickly accompanied by ENG 2 Stall ECAM. The Captain immediately assigned me to continue flying duties while he completed the ECAM. Because the EGT was climbing rapidly and out of limits, we followed ECAM procedures to shut down engine 2. [Requested priority handling] and we were cleared directly to the airport. The Captain checked FM non-normals for ENG 2 Stall and reviewed notes warnings and cautions. He then notified Dispatch, completed a briefing with the flight attendants, and made a passenger PA. After a discussion with the Captain, we decided that he would be pilot flying for the landing. We were set up originally to land on runway XL, however they offered us landing on XX to expedite our arrival. We accepted this, and I verified on the landing distance non-normals table that this would be acceptable with the wind component. We were cleared directly to a fix on the ILS, which we briefed. However after arming the approach, it became clear to me that the localizer was not reliable. I loaded the RNAV XX in the box to give the Captain backup guidance, although we were visual with the runway at this time. We were cleared for a visual approach. I queried ATC if the localizer was active for XX, and they turned it on. After receiving positive identification, I reloaded the ILS and used this to backup the visual approach. After landing, we exited the runway and asked airport rescue and firefighting (ARFF) to inspect the number 2 engine for anomalies. After receiving word that nothing was out of the ordinary, the Captain made a PA to the passengers, and we taxied to the gate.

Synopsis

A320 First Officer reported landing safely after the in flight shutdown of the number 2 engine.

ACN: 2122034 (267 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Fire/Overheat Warning
Aircraft Reference : X

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Trainee
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2122034
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Prior our descent to ZZZ we had a master caution indication and an ECAM message for Smoke: avionics smoke. Then we performed the ECAM actions, [advised ATC] and we both decided to divert to ZZZ1. We had no indication of smoke or fumes inside the cockpit nor the cabin. As soon as we turned to ZZZ1 the ECAM message disappeared and since we were close to initiating the descent to ZZZ we decided to cancel the [priority handling] and continue to ZZZ. Later, during the descent to ZZZ and approximately 10 minutes after the indication went out, we had the ECAM avionics smoke message again. [Requested priority handling] one more time, running the ECAM and flying fast to get the aircraft on the ground as soon as possible. After an assessment we decided to continue to ZZZ. The Captain informed the company, flight attendants (FAs) and the passengers of the situation. Once on ground a firefighter team checked the aircraft for any smoke indication, the passengers were well aware of what was happening. After the inspection we continued to the assigned parking spot, deplaning the aircraft normally. Ground personnel also checked the avionics compartment. This was the last flight as part of my IOE.

Synopsis

A320 pilot reported an avionics smoke ECAM prior to descent extinguished but returned later as flight continued to destination.

ACN: 2121988 (268 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2121988
Human Factors : Situational Awareness

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2121554
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After push/pull from the gate, we started ENG #1. We set brakes, and told the tug driver â€œBrakes setâ€¦cleared to disconnect inter phone.â€ Unbeknownst to us, immediately after ENG #1 completed its start cycle, the parking brakes failed. We started to very slowly drift forward. For a moment I thought it was an optical illusion. I looked at the parking brake and it was still set. The tug driver screamed â€œSTOP, THE PLANE IS MOVINGâ€. I forcefully applied the toe brakes and we came to an unusually smooth slow stop. Right at that moment my First Officer (FO) pointed to the accumulator pressure gauge and it was readingâ€0â€! We assumed that the smooth stop might have been that the accumulator pressure was already near zero when I applied the toe brakes. I kept pressure on the brakes until a Super Tug could be hooked up to our aircraft and tug us to a gate. We spoke to line maintenance and Maintenance Control while waiting to be tugged back in and went through steps with Maintenance control to restore braking. Recycling the nose wheel switch had no effect on accumulator pressure but recycling the parking brake did. Unfortunately, the pressure fell back to zero fairly quickly. Maintenance control mentioned some sort of Technical Standard Order (TSO) issue they were working regarding brakes but I wasnâ€™t clear as to exactly how that applied to our situation. We returned to the gate and Maintenance control took the aircraft out of service Cause: In our post event debrief I told my FO I was surprised that we didnâ€™t hit the tug and how fortunate that was! He told me that the tug appeared to be clear to aircraft right when we started drifting and that she apparently was still attached to the aircraft by the com line. Iâ€™m pretty certain thatâ€™s not in keeping with SOPâ€™s but in this case, it was a fortunate mistake for us, as she was the one who alerted us to the fact that we were drifting. As for us, I obviously couldnâ€™t help but wonder if I checked the triple indicator carefully enough before engine start or if complacency had been a factor. After realizing that the inbound crew parked the aircraft without incident, the FO and I checking it during our preflight flows and the troubleshooting steps with Maintenance control, it was unlikely that we missed it and it was just a failure without any warning!

Narrative: 2

During pushback, the Captain set the brakes once the push was complete. We had started one engine, as the tug crew pulled away, the plane started to roll. We heard the ground crew still connected on headset yell out and scream, stop stop stop. The Captain proceeded to apply manual brake pressure. I immediately looked at the brake pressure accumulator which showed 0. No damage to the aircraft occurred. We proceeded working with Maintenance control, line maintenance, ops and ramp to get the aircraft tugged back to the gate. Cause: Mechanical issue

Synopsis

A320 flight crew reported the aircraft rolled unexpectedly after ground personnel disconnected the tow bar because the parking brake failed to engage.

ACN: 2121419 (269 of 2908)

Time / Day

Date : 202405

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLA

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A319
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLA

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 156.48
Experience.Flight Crew.Type : 508.58
ASRS Report Number.Accession Number : 2121419
Analyst Callback : Completed

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

While at cruise at FL360 and as PM, aircraft started to rumble then entered at slow left bank for a few seconds, then a sudden left bank reaching about 25 degrees of bank. At this point the autopilot was disconnected and CA made corrective action to the right, while FO turned on fasten seatbelt sign and shadowed controls and monitored path. The plane then quickly banked to the right also reaching about 25 degrees of bank. "Upset" was vocalized by FO and then CA, CA brought the plane to wings level while vocalizing "push roll thrust stabilize". Plane was then stable, autopilot re-engaged and ATC was notified of event. Left of course deviation and 2,000 feet descent was requested and granted. ATC notified us we were about 10 miles in trail of a company A319 same altitude, additionally it was almost a direct headwind aloft. After vector and altitude change the flight resumed uneventfully to destination.

Callback: 1

Reporter stated they were surprised to encounter wake at FL360.

Synopsis

A320 First Officer reported an inflight upset at FL360 from wake turbulence encounter with the preceding A319.

ACN: 2120752 (270 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Relative Position.Distance.Nautical Miles : 7
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : Helicopter
Flight Phase : Cruise
Airspace.Class E : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 5100
Experience.Flight Crew.Last 90 Days : 201
Experience.Flight Crew.Type : 4900
ASRS Report Number.Accession Number : 2120752

Events

Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Primary Problem : Airspace Structure

Narrative: 1

Aircraft Registration: Aircraft X (A230) Cleared by ZZZ Approach to the RNAV Z Runway XXL approach. Conflict registered descending from 3000ft to 2500ft from ZZZZZ to ZZZZZ1. PF: Captain During the approach of Aircraft X from ZZZZ to ZZZ the pilots of Aircraft X were instructed to fly heading 360 and fly direct to ZZZZZ2. Just before reaching ZZZZZ2, ZZZ Approach instructed to maintain 3000ft until ZZZZZ2 and cleared us for the approach RNAV Z Runway XXL, speed at pilot discretion. After ZZZZZ2 and maintaining 3000ft as per the approach, the pilots began decelerating during the level segment from ZZZZZ2 to ZZZZZ. ZZZ Approach now instructed to change to ZZZ Tower over ZZZZZ1. Close to ZZZZZ we noticed a helicopter [Aircraft Y] northeast of ZZZZZ 400ft below as marked by TCAS. We immediately received a traffic advisory, however as soon as starting the descent as per the RNAV Z Runway XXL approach from ZZZZZ to ZZZZZ1 (descent from 3000ft to 2500 ft at ZZZZZ1), we soon realized the helicopter traffic was closely approaching our trajectory from of the RNAV Z XXL approach. Near to 2900 ft on the segment from ZZZZZ to ZZZZZ1, we received a TCAS resolution requesting an immediate level off. We responded to the TCAS resolution by leveling off, a second command again requested to continue level flight and a final and third instruction requested to initiate a climb. A couple of second later of responding to this final climb instruction and upon crossing 3100ft, the TCAS system called a "Clear of Conflict". Indeed the helicopter was now behind our trajectory and no further conflict was registered. We immediately notified ZZZ Approach of the TCAS resolution, at which they asked if we where still able to continue the approach. Not being too high of the recommended flight profile, we responded we where still able to continue the approach. ZZZ Approach again cleared us for the approach and instructed us to change to ZZZ Tower at XXX.X. The pilots where able to quickly configure the aircraft for landing and thus recovering the flight profile. ZZZ Tower cleared Aircraft X for landing at Runway XXL. Flying an stabilized approach at 1000ft, the pilots continued the final approach to Runway XXL and completed a successful landing with no further incidents.

Synopsis

A320 Captain reported a NMAC with a helicopter during approach resulting in a TCAS advisory. Flight responded to TCAS advisory to clear conflict and continue approach.

ACN: 2118998 (271 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2118998
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Runway
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After Captain (CA) landed Runway XX at ZZZ he began exiting Taxiway 1. After established on 1, steering issues began and the airplane was steering left and right abruptly. CA used counter steering and counter braking to keep the aircraft on the centerline. Due to the short distance of clearing Runway XX and Holding Short XY, CA was able to get the aircraft under control inside of the hold short line of XY. Ground advised us to stop, which we complied with. Then told us to continue to parking via Taxiway 2.

Synopsis

A320 pilot reported a steering malfunction after landing rollout while taxiing on a taxiway. The pilot regained control of the aircraft, then continued to parking.

ACN: 2118575 (272 of 2908)

Time / Day

Date : 202405

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2118575

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Software and Automation
Primary Problem : Aircraft

Narrative: 1

Plane had a spoiler maintenance action night prior. Yellow hydraulic quantity was extremely high noticed in preflight. Maintenance removed hydraulic fluid assuming over service. Flight was outbound XXX. Outbound flight XXXX. We noticed hydraulic fluid was now low but acceptable. Called Maintenance Control and Dispatch to confirm. All gave approval to continue with flight. On takeoff roll at approximately 40 knots received an ECAM HYD Y RSVR LO LVL. We accomplished a low-speed abort. And returned to gate. The system is indicating quantity high and low due to air being leaked inside the accumulator. Accumulator inop.

Synopsis

A320 Captain reported a hydraulic malfunction which resulted in a low speed rejected takeoff due to one hydraulic system indicating low. The aircraft returned to the gate.

ACN: 2117370 (273 of 2908)

Time / Day

Date : 202405

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 36.23
Experience.Flight Crew.Type : 737.43
ASRS Report Number.Accession Number : 2117370
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Shortly after releasing parking brake for pushback, the Captain and I realized we never received the Final Dangerous Goods Summary. Upon recognition we re-set the parking brake and sent a dangerous goods acknowledgment code which sent the report to us. The aircraft never moved during this process and we acknowledged with dangerous goods acknowledgement code upon receipt.

Synopsis

Air carrier First Officer reported the parking brake was released for pushback but the flight crew then realized the Final Dangerous Goods Summary was never received. The brakes were set again so the flight crew could send the dangerous goods acknowledgement code.

ACN: 2117321 (274 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 1201-1800

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 5023.05
Experience.Flight Crew.Last 90 Days : 167.32
Experience.Flight Crew.Type : 5023.05
ASRS Report Number.Accession Number : 2117321
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Ground Controllers appeared to be in training, many instructions had to amended by second controller for our taxi out. We received clearance to taxi onto and back taxi on RWY XXL. There were many blocked communications during this 10 minute period. As we approached Taxiway 1 a 737 pulled out right in front of us. I immediately used full brakes to stop and avoid contact with 737. First Officer queried Ground once we exited RWY XXL and Ground said they did give us a clearance to give way to aircraft but neither of us heard or read back any such clearance. Controller said someone else answered for us. I believe tapes should be reviewed in this incident as I believe a controller error was made. Both of us were actively heads-up and listening while we were taxiing.

Synopsis

Air Carrier Captain reported another air carrier taxied in front of them requiring the Captain to use full brakes to avoid a collision.

ACN: 2116950 (275 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 31000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2116950

Person : 2

Location Of Person.Aircraft : X
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2116954

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While climbing through 31000 feet with one pack inop, we were alerted to a high cabin altitude problem. We briefly leveled off between 310 and 320 but the cabin kept climbing so we started a descent. At that time we got a Master Warning due to the cabin climbing above 9700 feet so we expedited the descent and shortly thereafter the cabin began to come down. We continued down to 10000 feet to help burn off fuel to get below landing weight as we returned to ZZZ. Cause: Aircraft mechanical issues. Suggestion: During single pack operations, check the CAB PR SD page more frequently, especially during the climb segment to ensure that the cabin is pressurizing normally.

Narrative: 2

Captain was pilot flying and the First Officer pilot monitoring. Departure from ZZZ was uneventful, apart from we were operating with a single air conditioning pack. During initial climb it was verified that the cabin was pressurizing according to SOP (Standard Operating Procedure). Cleared to climb to FL350 by ZZZ control. Passing roughly 32,000ft the ECAM (Electronic Centralized Aircraft Monitor) advisory appeared showing the cabin pressure rising above 8800ft. Captain elected to stop climb at current altitude. First officer informed ATC of our action. Shortly after it was evident that the cabin altitude was still rising despite the outflow valves showing closed and air conditioning pack 2 appearing to show no fault indications. Captain elected to begin a precautionary descent and clearance to FL250 requested from ATC by first officer. Moments later the cabin altitude breached 9600ft and triggered the CAB PR Excess Cab Alt Master Warning. First officer donned oxygen mask while Captain issued an urgent situation call and began a descending turn to ZZZ. By the time the First Officer had reestablished cockpit communications with the Captain, the cabin pressure had stopped climbing and began to stabilize. Descent clearance to 10,000ft requested. Since the aircraft pressurization had begun to stabilize below 10,000ft cabin altitude, First Officer removed oxygen mask to improve cockpit communication. Captain transferred control of the aircraft to the First Officer while they managed communication with the cabin. Captain began a call with Dispatch and Maintenance to come up with a plan of action. Aircraft was significantly overweight for landing and since we were not in a [priority handling] situation it was deemed that it would not be prudent to land overweight. ATC cleared us for a holding pattern at ZZZZZ at 10,000ft while we burned the extra fuel before we reached max landing weight. Once below max landing weight Captain took control of the aircraft and flew an uneventful approach and landing to [Runway] XXL at ZZZ. Cause: We arrived at the aircraft with an Engineer already on board from a maintenance issue reported by the previous crew. The problem was an overheat on air conditioning pack 1 and the associated pack valve was also not responding correctly to the overhead switch position. A decision was made to manually secure the number one pack valve closed and depart under the associated MEL XX-XXa and XX-Xb. It would appear that the remaining pack was unable to properly pressurize the aircraft despite not showing any fault indications during flight. Whilst holding at 10,000ft, as we burned fuel before returning to land at ZZZ, the cabin crew frequently called to complain of areas within the cabin that were too hot. The pack flow selector had been at MAX throughout the flight and the temperature selections at minimum through the flight so the flight crew were unable to do any more to bring the temperature down. This deficiency was reported to maintenance crews on arrival back to ZZZ. Suggestions: Engineering were satisfied to dispatch under single pack operation, I do not think it would have been possible to identify there was an issue with the pressurization system without a more thorough maintenance inspection. Since we were under a little bit of time pressure with our flight duty limit approaching, this was not appropriate or required as per the MEL. The flight was limited to FL370 in accordance with single pack operation, a more restrictive max flight level may have helped.

Synopsis

A320 flight crew reported returning safely to departure airport after experiencing high cabin altitude related to single pack operation.

ACN: 2116002 (276 of 2908)

Time / Day

Date : 202405
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 104.8
Experience.Flight Crew.Type : 155.60
ASRS Report Number.Accession Number : 2116002
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While at cruise (FL360), we received a yellow system hydraulic pump low pressure ECAM. SD (System Display) page also showed low hydraulic quantity, cursor was at the bottom showing almost no hydraulic fluid remaining. We went through the ECAM steps and secured the PTU (Power Transfer Unit) and yellow HYD pump. We then [requested priority handling] and diverted to ZZZ. We looked through the flight manual for additional information. We made an uneventful landing at ZZZ and stopped on the runway until we were towed to the gate. Dispatch made a comment about continuing on to ZZZ1 with this failure. We elected to divert.

Synopsis

A320 First Officer reported receiving a hydraulic pump low pressure ECAM message during cruise. Flight diverted to the nearest suitable airport.

ACN: 2113825 (277 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 4679.37
Experience.Flight Crew.Last 90 Days : 164.13
Experience.Flight Crew.Type : 3839.68
ASRS Report Number.Accession Number : 2113825

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

During pushback the strap holding the nose gear to the lifting tug came off releasing the aircraft. Maintenance was called to visually inspect the nose wheel and no damage was found.

Synopsis

A320 Captain reported the aircraft was released from the tug inadvertently during pushback when the nose gear strap came off. No damage was found.

ACN: 2112114 (278 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : Airbus Industrie Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 130.37
Experience.Flight Crew.Type : 8299.40
ASRS Report Number.Accession Number : 2112114
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After push back from Gate XX and engine start, Ground Metering released us to Ground Control. Received clearance to â€œtaxi Runway XXL/ left on [Taxiway] 1, right on 2, left on 3...give way to air carrier from the leftâ€. After air carrier passed by, started to move forward. Nose was 8+ feet over non-movement line (into Taxiway 1). When seeing Aircraft Y northbound on [Taxiway] 1 approaching and passing 4 at fast pace, I stopped, flashed lights multiple times. FO attempted to contact Ground about conflict but frequency congestion prevented transmission. Aircraft Y (on separate frequency) never slowed--no nosegear strut compression whatsoever. Had I not stopped, Iâ€™m not sure Aircraft Y would have been able to prevent collision given speed at which they passed. After they passed, finally able to contact Ground Control who replied with a comment about not being able to transmit. No further discussion or comment.

Synopsis

A320 First Officer reported during gate pushback, another aircraft taxied by at a high rate of speed requiring the A320 crew to stop push to avoid a collision.

ACN: 2112056 (279 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Nav In Use.Localizer/Glideslope/ILS : RNAV GPS Y XXR
Flight Phase : Initial Approach
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class C : ZZZ

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 156.75
Experience.Flight Crew.Type : 2208.07
ASRS Report Number.Accession Number : 2112056
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 171.87
Experience.Flight Crew.Type : 940.15
ASRS Report Number.Accession Number : 2112069
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Air Traffic Control
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Overrode Automation
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On arrival to ZZZ being vectored for the RNAV (GPS) Y XXR everything was progressing well. When given a decent the PF said Open Descent (OP DES) wouldnâ€™t engage and was using V/S. In retrospect this was the first distraction. When cleared for the approach, the approach was armed and FINAL APP Flight Mode Annunciator (FMA) was cross checked. The aircraft flew through the final approach course which was the second distraction. The PF disconnected the autopilot simultaneous to ZZZ giving us a heading to reintercept. The PF over banked and our altitude began to increase. PM called "Check Path" and path stabilized. ZZZ canceled our approach clearance, told us to level at 4,000 ft and gave us a vector for another approach. Altitude was decreasing to 3,800 which was called out by ATC as correction was being applied. Airspeed was slowing as Auto Thrust was previously disconnected with Autopilot by PF. PIC assumed PF duties. Aircraft was stabilized. We were given a decent and heading. PIC was unable to duplicate problem engaging OP DES. Once stabilized and FINAL APP was engaged and verified to be working correctly, PIC resumed PM duties followed by a normal landing.

Narrative: 2

During final short vector to intercept final approach course the auto pilot did not capture and proceeded inbound or descend . However, we did have final app on the Primary Flight Display (PFD) Pilot intervened to correct and the Approach Controller directed a cancel of approach clearance And told to maintain an altitude of 4000 feet. I checked my altitude at 3800 and corrected. Returned to the same approach and completed. Safe landing.

Synopsis

A320 pilots reported the autopilot failed to capture the RNAV inbound course and contributed to the unstable approach.

ACN: 2109728 (280 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 22000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 129.48
Experience.Flight Crew.Type : 734.65
ASRS Report Number.Accession Number : 2109728
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb FL220 ENG 1 stalled with audible bang. Engine self-recovered and Non-Normal checklists performed and ATC advised. Engine indications were normal so we continued climb. At FL330 level-off engine stalled again with bang, self recovered, and normal engine indications. Options were discussed to continue or divert. Contacted Dispatch and Maintenance Control and we agreed to continue to destination. Moments later ENG 1 Compressor Vane ECAM occurred. Non-Normal checks performed which advised to avoid rapid thrust changes so we opted to operate the engines using manual thrust for the remainder of the flight and landed uneventfully.

Synopsis

A320 Captain reported the #1 engine stalled during cruise climb. The engine recovered but stalled again in cruise and a Compressor Vane ECAM occurred. Flight crew continue to destination using manual thrust.

ACN: 2109695 (281 of 2908)

Time / Day

Date : 202404

Place

Altitude.MSL.Single Value : 22000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 1177.73
Experience.Flight Crew.Last 90 Days : 86.8
Experience.Flight Crew.Type : 1177.73
ASRS Report Number.Accession Number : 2109695
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During climb FL220 ENG 1 stalled with audible bang. Engine self-recovered. Non-normal checklist performed and ATC advised. Engine indications were normal so we continued climb. At FL330 level-off engine stalled again with bang, self-recovered, and normal engine indications. Options were discussed to continue or divert. Contacted Dispatch and Maintenance Control and we agreed to continue to destination. Moments later ENG 1 Compressor Vane ECAM occurred. Non-normal checks performed which advised to avoid rapid thrust changes so we opted to operate engines using manual thrust for remainder of the flight and landed uneventfully.

Synopsis

A320 Pilot reported experiencing multiple engine compressor stalls, during climb and cruise flight, resulting in a decision to operate the engine at reduced thrust to destination.

ACN: 2109012 (282 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : GPS & Other Satellite Navigation
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 100.35
Experience.Flight Crew.Type : 2299.93
ASRS Report Number.Accession Number : 2109012
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : Aircraft

Narrative: 1

At FL350, we got a "SET OFFSIDE RNG (Range)" message on the Captain's ND (Navigation Display). We ran the QRC for this item and it cleared. At the same time we lost GPS signal. After GPS signal was regained, we discovered that we were slightly left of course. I spoke to the controller about our issue. He didn't seem at all concerned. He gave us direct ZZZ [VOR]. He then explained that there had been some GPS jamming in his sector. There was no mention of GPS issues in the logbook or the NOTAMs.

Synopsis

A320 Captain reported momentarily losing GPS signal and after regaining it, the flight crew discovered the aircraft was slightly left of course. ATC advised that there had been GPS jamming occurring in the area.

ACN: 2108354 (283 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2108354
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2108355
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We had no APU so had to start engine at the gate. Proceeded to push back and was informed by ramp crew to set brakes. I set brakes. After about a minute the ramp crew informed me that they were having trouble disconnecting the tow bar and requested I release brakes. I released brakes and the plane moved forward, backward there was a pause and the plane began to move forward again. I hear in a frantic voice "set brakes, set brakes" I set the brakes and ask what happened and they responded that they tried to disconnect tow bar and the airplane rolled up onto the tow bar. Maintenance came out and I verified with them that the brake light on nose gear was working. Maintenance eventually lowered the tow bar and removed it. They found a new tow bar and towed us back to the gate. On initial contact with tug driver, another person had to step in and talk on the interphone. I feel that there was training involved in the pushback and that lack SOP's and inexperience played a factor in the incident. Suggestions: Make sure that ramp crew follows SOP's, takes there time and know that they can verify brakes are set by the light on nose gear.

Narrative: 2

Preflight duties for Aircraft X were normal. After briefing the supplemental procedures, we started the left engine at the gate using an external air start unit (APU MEL). The push procedure was normal till after the push was completed and the parking brake was set as directed by the ground crew. Flight crew was notified that the ground crew was unable to disconnect the tow bar, and the Captain was asked to release parking brakes. I forget the wording the ground crew used, but my recollection is that they needed to adjust the tow truck position to be able to disconnect the tow bar. After the parking brake was released, the ground crew moved the aircraft forward and aft a few inches several times. The flight crew did not receive a command to set the parking brake before a grinding noise was heard on the flight deck. Right after the noise was heard, the ground crew repeated "set parking brake" several times in a frantic voice. The Captain immediately set the parking brake. Flight crew was notified the aircraft had rolled on top of the tow bar and that the tow bar was stuck under the nose gear. Ramp Control and Maintenance were notified of our status. Flight attendants and passengers were briefed several times as information was passed to flight crew from Ramp and Maintenance personnel through the ground intercom. After approximately an hour, the tow bar was dislodged, and the aircraft was successfully towed back to the originating gate where passengers were deplaned. The Captain and First Officer were directed to submit to alcohol and drug tests, which we did. The parking brake light on the nose gear was verified operational during my walk around inspection before the flight. My impression was that the ground crew pushing the aircraft was inexperienced. I base that on the way their communication with the flight deck was conducted. Not directing the parking brake to be set and not verifying the light on the nose gear being illuminated before disconnecting the tow bar were contributing to the event. Suggestions: Having two persons from the ground crew independently verifying the parking brake being set before disconnecting the tow bar would mitigate the risk of this event happening again. It was dark on the ramp and I'm unsure whether ground crew had proper illumination while disconnecting the tow bar. If not, that may have helped mitigate risk as well.

Synopsis

A320 flight crew reported loss of aircraft control during pushback procedure.

ACN: 2106818 (284 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Altimeter
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2106818
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On flight XXXX from ZZZ-ZZZZ I inserted the wrong altimeter setting and got low on a visual approach that was backed up by RNP to Runway XX at ZZZZ. This happened due to a combination of errors that we as a crew allowed due to the ease of flying in VFR conditions. When I set up for the approach, we were too far out to receive ATIS from ZZZZ. I used the weather from FD Pro X app and can only guess that I had old weather due to losing the internet connection earlier in the flight. My First Officer (FO) was not able to get weather due to no internet and I input my weather. We planned to get ATIS when closer to the airport, but on arrival we were given multiple slow downs on the decent and were talking about making the crossing restrictions. When on the approach and visual with the airport we started our final descent to landing and I realized that we were low. There is a large drop off just prior to landing on Runway XX, so I delayed leveling off in case it was a visual illusion. I eventually clicked off the Autopilot and called for flight directors off. At the same time we received a warning of "too low". It went away as soon as we crossed the ridge line to the depression prior to the runway. We landed uneventfully.

Synopsis

Air carrier pilot reported descending too low during visual approach and receiving an aircraft terrain warning as a result of entering the wrong altimeter setting. Flight crew continued approach and landed.

ACN: 2106640 (285 of 2908)

Time / Day

Date : 202404

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1607.40
Experience.Flight Crew.Last 90 Days : 182.7
Experience.Flight Crew.Type : 1313.35
ASRS Report Number.Accession Number : 2106640
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 125.95
Experience.Flight Crew.Type : 209.70
ASRS Report Number.Accession Number : 2106634
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approaching the top of descent, we received a HYD Y RSVR LO LVL, the ECAM was announced, the FO remained Pilot Flying and assumed the radio. I completed the ECAM, briefed the status page, referred to the FM (Flight Manual) for any additional notes, and looked up the non-normal landing distance chart. I tried to confer with Dispatch and Maintenance, but the frequency the Dispatcher gave me did not work. We [requested priority handling) with Dispatch and ATC. I gave the flight attendants the briefing, and told them not to prep for evacuation, but to review all items in case another failure presented itself. I made an announcement to the passengers quickly explaining the situation and offering reassurances. I then briefed the FO on the lack of reverser, and spoiler panels we wouldn't have and discussed how we would configure early to account for the slower flap speed. I briefed the approach, and then switched to the Pilot Flying while the FO took over the Pilot Monitoring duties. We rolled the fire truck for a precautionary measure. I flew the approach, adjusted my aim point to account for the longer landing roll. The landing and rollout were uneventful. We notified ATC after landing that the trucks were not needed. We taxied to the hard stand and accomplished the parking checklist.

Narrative: 2

Near the time we were going to begin the initial descent into ZZZZ we received an ECAM for HYD Y RSVR LO LVL. I announced the ECAM, and the CA told me to remain Pilot Flying while she initiated the actions required by the ECAM. Once completed, the CA briefed me on the systems that were affected by this ECAM as well as updated landing data, and that she would assume Pilot Flying responsibilities for the approach and landing. During that time the CA [requested priority handling]. The controls were transferred, and I assumed Pilot Monitoring responsibilities. The approach flown by the CA was uneventful as well as the landing. We did not require emergency service assistance, and were able to taxi to a pad adjacent the terminal.

Synopsis

A320 flight crew reported receiving a HYD Y RSVR LO LVL message during cruise. Flight crew continued to destination airport and landed uneventfully.

ACN: 2105403 (286 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5400

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 59.32
Experience.Flight Crew.Type : 3253.18
ASRS Report Number.Accession Number : 2105403
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cleared for Takeoff [runway] XXL ZZZ [airport], winds were from the West at 9 kts. Captain was PF. First Officer was on first leg of Operating Experience (OE). The Jumpseater was on his observation for new Line Check Pilot (LCP) training. I advanced the throttles to 50 percent N1, engines spooled up evenly and were stable. I advanced the throttles to the FLEX position. Standard callouts were made along with SOP Flight Mode Annunciator (FMA) callouts. No irregularities were noted. Estimated around 90kts there was an aggressive uncommand yaw to the left that did not decrease. The aircraft was heading towards the left edge of the runway. I elected to reject the takeoff. When I felt the yaw to the left, I immediately looked at my engine instruments, all were normal and synced. It felt like an engine failure. The Auto brakes did engage. once I had the aircraft under control I dis-engaged the auto brakes and made a right turn onto taxiway 1 and then another right turn heading North on taxiway 2 where we ran our checklists and made the appropriate calls to operations, Dispatch and maintenance. All of the appropriate PA announcements were made. After talking with Maintenance Control a decision was made to go back to the gate. Once we were off the active runway all 3 pilots discussed the event. None of us saw any type of irregularity with the engines nor did we feel any type of wind gust. We had Dead Heading pilots in the passenger cabin. I asked them if they felt the yaw and they both said â€Yesâ€ thinking it was an engine failure. After the Rejected Takeoff (RTO) ZZZ tower told us they had not seen any type of a wind gust. they reported the winds to us at 290/9kts

Synopsis

A320 Captain reported the aircraft veered uncontrollably left during takeoff possibly due to engine failure resulting in a rejected takeoff.

ACN: 2102924 (287 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2102924
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2102915

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Upon landing in ZZZZ the runway I applied max reverse and was using medium auto brakes (as briefed). The runway was very very wet and slippery and ungrooved. I applied manual brakes and took the thrust levers out of max reverse. I started to brake and slow however the aircraft was pulling left. I told the FO I was sliding, as I was letting off the brakes to try and stop the slide and go to the end of the runway to turn off he said â€œI have control.â€ He slowed the aircraft and pulled it right slightly. As I felt the aircraft slowing, I said I have it and taxied off the runway. There were no violations that occurred.

Narrative: 2

Upon landing on RWY XX in ZZZZ on a wet runway with 8 kts 90 degrees right crosswind, the aircraft started aquaplaning. To the left, the Captain said â€œI am sliding â€œseveral times. The aircraft continued to slide toward the edge. When I saw that the aircraft did not change its trajectory I said â€œI have controlâ€ pushed almost full rudder to stop the slide and the aircraft was stabilized parallel to center line, the Captain said â€œI got itâ€ I relinquished controls and we taxied to the gate. There was no runway excursion and no damage to the aircraft.

Synopsis

A320 flight crew reported a loss of control when landing on a wet ungrooved runway with a slight right crosswind using maximum reverse and medium autobrakes. The flight crew resumed control of plane after the First Officer stabilized the track parallel to centerline and exited the runway normally.

ACN: 2102217 (288 of 2908)

Time / Day

Date : 202404
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class D : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 111.93
Experience.Flight Crew.Type : 765.80
ASRS Report Number.Accession Number : 2102217
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overrode Automation
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Primary Problem : Ambiguous

Narrative: 1

VFR night into ZZZ. ATC cleared us for the visual approach to XXL. We had the RNAV RNP Z Rwy XXL loaded in the FMGC. After activating the approach, the airplane entered into FINAL APP and was descending on the flight path to 2000 ft, the altitude at the final approach fix ZZZZZ. While on the path, with autopilot on, we noticed the aircraft was descending to be at 2000 ft, about 5 miles prior to the final approach point. As we were discussing this, we noticed that there was terrain to the right of the airplane and discussed how we were well above the terrain but the airplane should not have descended as low as it did, as early as it did. I disconnected autopilot and directed the Pilot Monitoring to select 3000 ft. Right at that moment we got a call from ATC asking us if we were aware of our altitude being lower than normal and we responded yes. We climbed up, and finished the arrival without incident.

Synopsis

Air carrier Captain reported the autopilot descended the aircraft prematurely before the final approach fix resulting in an ATC altitude alert. Captain disconnected autopilot, climbed to safe altitude and continued approach.

ACN: 2101020 (289 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2101020

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We rejected the takeoff at 60 kts for an ENG 1 Master Caution ENG 1 low oil pressure ECAM. We advised Tower and the passengers and cleared the runway. Then called Dispatch and [Maintenance] and reported the situation and return to the gate without any further incident. The weight of the aircraft was a 148.6 and the brake temps remained in the green the whole time. Entered it into the logbook. The aircraft was written up twice in the previous four days for the same ECAM message.

Synopsis

A320 Captain reported rejecting the takeoff after receiving a Master Caution for ENG 1 Low Oil Pressure, noting that the aircraft had been written up for this anomaly several times in the last few days.

ACN: 2097813 (290 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2097813
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While pushing back from ZZZ, the ground operations employees had trouble disconnecting the tow bar. We could feel them struggling with it from within the flight deck. I noticed the tug driver give us a release the brakes hand signal to create play or possible movement to disconnect the tow bar or to move us forward. The aircraft moved approximately 2 feet forward (we thought being moved by the tug), then I saw the tug driver and second ground ops employee give us the set brake hand signal. At that time we saw the tow bar disconnected and pulled back from the tug, followed by a thumbs up from the tug driver and a wave off from the second grounds operations employee. All three vacated the area and we proceeded with our flows. We did not see anything that was unsafe, just a difficult situation for the ground operations employees. The flight proceeded safely with no further issues.

Synopsis

A320 pilot reported uncommanded aircraft movement during push back. The pilot noticed the aircraft moving after disconnecting from the tug and set the brakes with no damage or injury to ground personnel.

ACN: 2097688 (291 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 0001-0600

Place

Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Autopilot
Aircraft Reference : X

Component : 2

Aircraft Component : PFD
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Component : 5

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3056.08
Experience.Flight Crew.Last 90 Days : 179.77
Experience.Flight Crew.Type : 3056.08
ASRS Report Number.Accession Number : 2097688
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

1. Mid way thru flight autopilot 1 kicked off spontaneously. Could not reengage. Engaged autopilot 2. 2. Lost captains map. Went to FOâ€™s map by matching distance with his. 3. Approximately 40 miles from ZZZ saw that green hydraulic system reservoir was half empty. 4. Requested expedited handling to ZZZ. ATC controller [requested priority assistance] for us. 5. Got a Cabin pressure failure ecam. 6. About 8 miles out from landing, constant ecam alerts stating that our throttles were out of position while in CT (they weren't) along with a decay in airspeed. 7. Disconnected the auto throttle and hand flew the throttles to landing but had to adjust throttle position constantly to maintain flyable airspeed to landing with constant ecam â€œdingsâ€ going off.

Synopsis

A320 Captain reported the Autopilot 1 disengaged during cruise and multiple other aircraft systems malfunctioned, including hydraulic system, during remainder of flight. Flight crew continued to destination and landed.

ACN: 2097657 (292 of 2908)

Time / Day

Date : 202403

Place

Altitude.MSL.Single Value : 20000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent

Component : 1

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Failed

Component : 3

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Failed

Component : 4

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 253.90
Experience.Flight Crew.Last 90 Days : 157.27
Experience.Flight Crew.Type : 253.90
ASRS Report Number.Accession Number : 2097657

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Initially we [requested priority assistance] for the green system hydraulic indication dropping. Then various failures started like Autopilot One failure at about FL200. On final approach Cabin Pressure amber ECAM appeared with the outflow valve closing, descending through 4,000 or 3,000. Then through about 1,500 ft, the "thrust levers not set" message continually kept flashing with the aural warning ringing every time. The Captain kept trying to set Support per the message but it would just spool the engines up without setting the automatic function. Even though he disconnected the auto-thrust the message would briefly appear and disappear any way all the way to touch down. After landing the emergency personnel verified the aircraft was clear of abnormalities and we taxied to the gate normally. The after landing flow was accomplished nearing the gate so the slats not fully retracting after shut could have been because both engines were already shut down, but the message appeared in amber.

Synopsis

Air carrier flight crew flying an A320 aircraft reported hydraulic system fail and a multitude of other system failures inflight. Flight crew landed uneventfully.

ACN: 2096957 (293 of 2908)

Time / Day

Date : 202403

Place

Altitude.MSL.Single Value : 20000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent

Component : 1

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 157.27
Experience.Flight Crew.Type : 253.90
ASRS Report Number.Accession Number : 2096957

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Initially we [requested priority handling] for the green system hydraulic indication dropping. Then various failures started like Autopilot One failure at about FL200. On final approach Cabin Pressure amber ECAM appeared with the outflow valve closing, descending through 4000 or 3000. Then through about 1500 ft, the â€œthrust levers not setâ€ message continually kept flashing with the aural warning ringing every time. The Captain kept trying to set Maintenance Control per the message but it would just spool the engines up without setting the automatic function. Even though he disconnected the auto-thrust the message would briefly appear and disappear any way all the way to touch down. After landing the emergency personnel verified the aircraft was clear of abnormalities and we taxied to the gate normally. The after landing flow was accomplished nearing the gate so the slats not fully retracting after shut could have been because both engines were already shut down, but the message appeared in amber.

Synopsis

Air carrier First Officer reported multiple system failures in flight. Flight crew landed uneventfully.

ACN: 2096540 (294 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZKC.ARTCC
State Reference : KS
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZKC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZKC
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Plan : IFR
Airspace.Class A : ZKC

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 109.58
Experience.Flight Crew.Type : 559.28
ASRS Report Number.Accession Number : 2096540
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Flying at FL350 over Kansas we experienced wake turbulence and a moderate 2-3 second jolt in previously smooth air. As a result, Flight Attendant spilled hot coffee on her hand while serving in the back of the plane. I asked If she was OK to continue on to PVR and she was. There were no other reported injuries. [She] declined any medical assistance upon arrival into PVR. She said she would fill out a report for inflight as well as my own. Notified Dispatch after the event and confirmed with Flight Attendants, no medical attention was needed in flight or upon arrival into PVR.

Synopsis

Air Carrier A320 Captain reported a Flight Attendant was slightly injured when they experienced wake turbulence in cruise flight at FL350.

ACN: 2094588 (295 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 160.08
Experience.Flight Crew.Type : 1410.52
ASRS Report Number.Accession Number : 2094588
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

A single engine taxi was being completed after landing and taxiing to gate X in ZZZ.. A marshaller and two wing walkers were confirmed prior to proceeding on the lead in line. I noted the fact the jetway at this gate is fixed and a precise stop is required for successful parking and attaching the jetway. I was moving very slowly and was given the stop signal from the marshaller. At this time, I set the parking brake and verbally and by pointing confirmed the parking brake message was displayed, the triple gauge showed brake pressure on both brakes and accumulator pressure in the green. I then directed the First Officer to shut down the number one engine. As the aircraft power switched to APU generator, the parking brake seemed to release and the aircraft moved forward approximately five feet. I immediately sensed the motion and applied manual toe brakes to stop the aircraft. The marshaller also saw the movement and got in to a position to get my attention that the aircraft was moving. After confirming we were chocked, I released the toe brakes and cycled the parking brake handle. A tug was then attached, to push the aircraft back to the proper position. A maintenance write up was entered and local maintenance notified. We completed a verbal brief with the maintenance technician.

Synopsis

A320 Captain reported an inadvertent release of the aircraft parking brake after arriving at the gate, resulting in unwanted aircraft movement.

ACN: 2094036 (296 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 96.35
Experience.Flight Crew.Last 90 Days : 96.35
Experience.Flight Crew.Type : 96.35
ASRS Report Number.Accession Number : 2094036
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X. ZZZ-ZZZ1. Approximately 1.5hrs into flight at cruise when HYD Y RSVR LO LVL ECAM appeared. I remained Pilot Flying while the Captain remained Pilot Monitoring and ran the ECAM, talked to dispatch, briefed flight attendants and talked to the passengers. [Requested priority]. We began looking for alternate airports and decided on ZZZ2. Began descent into ZZZ2 after running appropriate checklists and talking with dispatch. Did the ZZZ3 arrival to the RNP XX approach. Overweight landing and landed smoothly. Taxied to the hard stand without any other incident.

Synopsis

A320 First Officer reported a hydraulic system failure and a successful diversion.

ACN: 2093888 (297 of 2908)

Time / Day

Date : 202403
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 8000

Aircraft

Reference : X
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class D : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2093888

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

FO flying. Descending through 8000 ft on ZZZZZ2 arrival into ZZZ. Experienced a very slight bang and very faint smell in cockpit which dissipated quickly. ECAM indicated Engine #1 Stall for 2 seconds then ECAM went back to Normal and could not be recalled. Engine gauges were also normal at idle. Flight Attendant called saying they experienced the same things. Captain let them know we were working on it and would get back to them. Captain did Procedures while FO flying to follow. Leveling off at 6000 ft #1 engine started to increase thrust as temperature increased with ECAM indicating Engine #1 EGT Over limit with directions to shut down â€“ Engine #1. It took but 4 seconds to complete with no further smells or sounds. Engine #1 indications back to below normal and OFF. Completed ECAM- Followed by appropriate QRHs. [Requested priority assistance] â€“ Contacted Flight Attendants code XXXXXX. â€œExpect remain seated.â€ Contacted ZZZ operations- Lastly, Captain spoke to passengers and let them know â€œWhat happened followed with expect a normal landing and please follow cabin crew instructions.â€ Captain did normal landing on Runway XX. Stopped aircraft on runway. â€œCabin crew remain seatedâ€. Captain spoke to Crash Fire Rescue to look at #1 engine briefly to look for any abnormalities. Taxied to gate. MEL was applied to Maintenance Logbook.

Synopsis

A320 pilot reported a bang and faint smell during descent followed by an engine stall ECAM. Crew followed ECAM procedures, shut down the engine and landed single engine.

ACN: 2091249 (298 of 2908)

Time / Day

Date : 202402
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : A320
Flight Phase : Parked

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 175.97
Experience.Flight Crew.Type : 4615.43
ASRS Report Number.Accession Number : 2091249
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Last 90 Days : 175.97
Experience.Flight Crew.Type : 4615.43
ASRS Report Number.Accession Number : 2092582

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I met the First Officer, whom had arrived first and begun cockpit preparations, at the aircraft and was made aware that we were without ACARS. Knowing that maintenance records indicated it was not an aircraft issue, I recalled with the First Officer that it was a long standing problem associated with the particular gate, at ZZZ. He explained to me that he had already reached out to our dispatcher and made arrangements takeoff performance details to be delivered to both of us via company email. We utilized IPAD for ATIS, [verified] our current Maintenance Release, and acquired our departure clearance via VHF, during pre-departure set-up. What we didnâ€™t do was to call operations prior to pushback to determine if we were planned to carry any DG (Dangerous Goods), assuming the known ACARS issues associated with the gate would trigger proper DG paper fallback procedures. With no paper Final DG Summary delivered to us, a gate agent final check and delivery of paper reports, and a ready call from our push crew, we initiated pushback unaware that we were carrying any dangerous goods. It wasnâ€™t until after engine starts and initial taxi that ACARS messages began arriving and those did not include notification of planned DG, nor any Final DG Summary. What we did receive was a message that read as follows: â€œNotification Form Warning ZZZ ZZZZ SENT: XA:47:13Z Pilot in command must acknowledge receipt of final notification form Message using ACARS dangerous goods acknowledgement code prior to aircraft movementâ€. I recognized the message was associated with DG and had the First Officer contact operations for assistance. Upon initial contact with ZZZ Operations we were told that we would need to return to the gate in order to rearrange things in our aft loading pit, and only after a couple more queries did we first become aware that we were carrying DG. We now understood that we were carrying dry-ice and that it had been determined it was not packed properly in the aft pit. Consequently, we returned to the gate and shut down both engines. I had the flight attendants keep the doors armed as the jet-bridge was back and no gate agent was visible. Our aft cargo pit was opened for a short time and I questioned operations about proper DG notification plans. Operations indicated they would look into it and that there were known ACARS issues at the gate. Our aft cargo pit door was closed and we heard from the push crew indicating they were ready to push. I asked to speak with the Ramp Lead to determine plans for DG notification and explained to him that we had no DG notifications of any sort and asked if there were plans to deliver paper copy to the cockpit. We were told to expect it via ACARS and that our loading issues had been resolved. At this point I now noticed a gate agent was positioning the jet-bridge next to our aircraft and I attempted to wave her off while asking the First Officer to call Ops again to determine what was going on. Operations explained that the gate agent was not fully aware of plans and that there were no plans to deliver anything to the flight deck via the jet-bridge. We completed Pre-Flight and Push checklists again prior to our second pushback of the evening with the expectation of receiving a DG summary report once clear of the Gate. After pushback and engine starts we then elected to taxi clear of the ramp area while still awaiting our Final Weight Manifest and the final DG summary. This hadnâ€™t happened approaching the runway so we made arrangements to delay in the adjacent holding pad area. After approximately 10 minutes utilizing ACARS and additional calls to operations in an effort to receive required data and documents, I then elected to call Dispatch via personal cell-phone. The Dispatcher got the Load-Planner on the connection and she informed us that the required DG Summary had been sent to the aircraft 3 separate times. When I told her that we had not got it, she said she would send it again. We talked briefly about having it sent to personal email when it finally arrived at the aircraft. With everything we needed after what seemed a great deal of extra effort, we departed one hour after initial push back. The remainder of the flight was mostly normal, other than conflicting OFF, ON and IN time data that never seemed to sync across the multitude of wifi applications.

Narrative: 2

In addition to ACARS/DG (Dangerous Goods) complications I reported on via report approximately 48 hours ago, I received the following email from our dispatcher yesterday. While his concerns and experiences add additional issues to this flight, I suspect all of it is somehow related to ACARS connectivity at and around ZZZ Gate and the way the system buffers data in an electronic cache. ZZZ ZZZZ irregular takeoff weight. I the dispatcher for flight ZZZ ZZZZ. I just wanted to let you know that I have filed a report for takeoff weight issues that occurred during the generation of the second release. At the time of the second release it appears as though it reduced our max allowable takeoff weight from 169700 to 165800ish. Release number 2 was issued about 20-30 min after takeoff but the maintenance computer system did not recognize that you guys were in the air till I had the desk update your FOB and off time (XB41z FOB 35.7). The actual takeoff weight 165519, so based off the initial release there was around 4000 lbs buffer. Both the planned zfw and fuel on board had decreased by the time of takeoff. In terms of legality, we were legal off of release 1 since takeoff occurred before release 2 was generated. Due to the issue with acars, numbers were initially sent through email and it appeared there were no issues there. I believe a Takeoff gross of 169.7 was requested. The only thing that I can think of that would have affected the max takeoff weight would be a change in the atmospheric conditions at the time of release 2. Thatâ€™s assuming that the runway for takeoff was the same as the runway that was planned, or some kind of maintenance computer system anomaly occurred since the maintenance computer system still thought the aircraft was on the ground. Load planning called after release 2 was sent and informed me that we were 300 lbs overweight. I told them that you were already in the air for about 20-30 minutes. We spoke for a while, and it appeared as though everything was fine. I called back a few times to verify that everything was recorded correctly because at the time of release number 2, your off time and FOB had not been recorded yet. I was assured that everything was ok, but later I went back to check some of the other releases, and I saw the change in allowable takeoff weight and what they were alluding to earlier on release number 2. I had a dispatch chief look into it for me and he agreed that the allowable takeoff weight should not have changed that much. I also completed another takeoff calculation while I was talking to the chief dispatcher and there appeared to be no issues at that time. Normally once an aircraft takes off and a flight is recalculated, weights are locked but, in this situation, release 2 was generated with a different allowable takeoff weight and a position report but no off time or FOB. From the people that I talked to, it appears that it was most likely an error due to the fact the flight was enroute and both unimatic and the maintenance computer system thought the aircraft was still on the ground. I decided to just submit a report because there are just too many unknowns as to what actually went wrong with the generation of release 2 and only some assumptions of what could have caused the adjustment in take off weight. The only thing that was changed in the maintenance computer system was an assumed off time and a standard recalc.

Synopsis

Air carrier Captain reported a gate return to correct improper DG gate departure procedures. After correcting multiple issues the flight departed safely.

ACN: 2090153 (299 of 2908)

Time / Day

Date : 202402

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 12.53
Experience.Flight Crew.Type : 228.92
ASRS Report Number.Accession Number : 2090153
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We had completed the before push checklist. I was doing a threat forward brief with the push back crew regarding push back. They asked for brake release for capture. After the capture I stated we would not push back until we received the final Dangerous Good (DG) and sent the aknowledgment code. It was then that I realized we were not supposed to release the parking brake. In the future I will make myself a better visual cue to not release the brake before receiving final DG.

Synopsis

Air carrier Captain reported the inadvertent brake released prior to verifying the DG Final Summary and submitting the flight crew acknowledgement message.

ACN: 2089095 (300 of 2908)

Time / Day

Date : 202402
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Descent

Component

Aircraft Component : Unknown
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2089095
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

A burning plastic odor was detected on the flight deck by the FO and myself as we were crossing ZZZZZ at FL 240 on the ZZZZZ arrival into ZZZ. No ECAM smoke or fire related messages were displayed. The cabin crew immediately called to notify us that customers and themselves were detecting the same burning plastic odor near row XX. Itâ€™s associated overhead bin was very hot to the touch by two different F/As, but no smoke was visible. Customers were being relocated to other seats, away from the affected row. I [requested priority handling] with ZZZ center and diverted to ZZZ1. The cabin crew was briefed, and provided them an ETE of 10 minutes before landing. The FO and I briefed that he was still the PF; to coordinate with ATC as I would be focused on the cabin; that we will transfer controls prior to turning final; and that we will stop the aircraft on the runway to assess the cabin situation before continuing. The odor intensified. I began reaching for the crew O2 mask and contemplated running the smoke / fume removal QRH. But it quickly stabilized and began to decrease afterwards. The odor was not causing significant irritation to the eyes, nose or throat. We therefore prioritized getting on the ground as soon as possible. The cabin crew reported that the row XX overhead bin was still hot, but the burning plastic odor had decreased. However, they received conflicting reports from customers who claimed that they felt heat coming from underneath. I checked the cargo smoke panel and verified that the smoke lights were not illuminated. ATC cleared us for the visual XXR. The touchdown was normal and under max landing weight. The burnt plastic odor continued to persist during the decent and landing. After touchdown, I applied max manual braking and set the parking brake to ON after coming to a complete stop. The cabin crew reported that the hot overhead bin had cooled off, but the burnt plastic smell persisted. An evacuation was not warranted; therefore we proceeded to taxi to the gate. Nearing the gate, the odor on the flight deck had mostly dissipated. One of the first persons to board the aircraft was a security inspector. She detected the residual burnt plastic odor, starting on the flight deck to row XX. She did not detect the odor beyond row XX. She performed a quick search of the bags in the overhead bin of row XX, but found no indications of fire or heat damage to neither bags or bin. The burnt plastic odor discrepancy was documented into the aircraftâ€™s maintenance log. The odor was described to assist tech ops in troubleshooting. The FO and I declared ourselves fatigued. During the de-brief with the cabin crew, it appeared that the burning plastic odor decreased after the flight attendant had de-selected the IFE (In-flight entertainment)/ PED (Personal Electronic Device) system.

Synopsis

Air carrier Captain reported a burning plastic odor during descent with no aircraft indications. The flight crew diverted where a safe landing was completed.