ACN: 2044984 (351 of 2908)

Time / Day

Date : 202310

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Unknown
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2044984
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

At cruising altitude about 30 minutes in the flight. Captain asked what we are burning. Lead Flight Attendant responded we were not cooking anything. There was an electrical burning smell in the cockpit and forward cabin. They both searched for the source of the smell. When they couldnâ€™t find it FA3 (Flight Attendant) searched. FA3 couldnâ€™t locate it. I searched and couldnâ€™t locate it. When I stepped back in the forward galley I heard a whining sound and then heard something shut down. Then the Captain said take your seats I am declaring a [priority] landing. We prepared for landing. We did land safely without out incident. We did not evacuate. We pulled to the gate safely. Unfortunately the airport was not equipped for our aircraft. So we deplaned by baggage belt. That was scary as well. Once off the plane and inside the airport and adrenaline was gone. I was not okay mentally to continue on with the rest of my trip. I was pulled from my trip and put in a hotel to go home the next day. The crew was informed that we had to take the plane back. Because maintenance MEL the issue. I was not okay with that. But when I called to ask crew service if I could get a car home. Instead of the plane because I didnâ€™t feel comfortable until it was completely fix. They said I can deviate from what is on my schedule but. I would have to find my on way home and follow the proper company procedure. After speaking to the Captain about my concerns. He stated that he would not take this plane it he was not comfortable. I was not okay with that. But if I didnâ€™t want to be stuck in ZZZ, another night I had no other choice but to ride on the plane I was not comfortable with. We did make it back without incident the next day. When we as crew go through an [unexpected issue] we should not be expected to continue on. We should be pulled from the trip. And make sure we are okay physically and mentally. We should not be in fear of losing our jobs and or being reprimanded for not wanting to continue on from a [priority] event.

Synopsis

Air carrier Flight Attendant reported an electrical burning odor in the cockpit and forward cabin that was corroborated by the Captain. The Captain completed a diversion and landing.

ACN: 2042723 (352 of 2908)

Time / Day

Date : 202310

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2042723
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X Prior to pushback we briefed that I would be the PF (Pilot Flying). Prior to departing from ZZZ on XXR, before crossing the hold short line, the Captain set the thrust levers and began taxing onto the Runway. While still in motion and with the power still set (somewhere above 1.05 Engine Pressure Ratio) the Captain stated, â€œyou have the thrust levers.â€ Given that we were still in motion and in the midst of a Captain initiated tiller turn, I did not move the thrust levers, as I had not been given control of the aircraft. During this turn, the aircraft continued to turn to the left well over centerline and I felt the rudder pedals deflect full right as the Captain was trying to regain/establish centerline control. The plane momentarily straightened then immediately began turning hard right. As we again reached the center line (now heading towards the right side of the Runway) I felt the rudder pedals deflect to the left, shortly after the plane started turning left. As we passed the center line again, moving to the left side of the Runway the Captain set the thrust to idle and brought the aircraft to a stop. The Captain then made a PA asking the passengers and flights attendants to remain seated. Tower asked us to vacate the Runway and upon doing so, we requested an inspection of the landing gear by station ops. Seeing no signs of smoke, fire, damage or deflation, we were able to taxi to the Gate. Tower mentioned that a tire â€œappearedâ€ flat but inspection determined that it was not. Brake temps were monitored and remained within normal limits. Upon arrival to the Gate, Maintenance was called. At no time during the events of this report was I given nor did I accept control of the aircraft. Photos attached show the condition of the tires after the event.

Synopsis

Air carrier flight crew had miscommunication regarding aircraft control during taxi for takeoff.

ACN: 2042580 (353 of 2908)

Time / Day

Date : 202310
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 83
Experience.Flight Crew.Type : 83
ASRS Report Number.Accession Number : 2042580
Human Factors : Workload
Human Factors : Time Pressure

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 139
ASRS Report Number.Accession Number : 2042906
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Dispatch
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Prior to pushback, we received the final Dangerous Goods summary. The FO (First Officer) provided me the printout, I told him â€thanks, I got itâ€ and simultaneously the push crew asked for clarification on our pushback time.... During the push, we received the notice that the confirmation message was not sent. On completion of the push, the parking brake was set and the normal disconnect procedures were completed. I then sent the confirmation message, told the FO that what had happened, we completed the after start checklist and continued the flight.

Narrative: 2

Prior to pushback, we received the final dangerous good summary. I provided the CA (Captain) the DG (Dangerous Goods) acknowledgement print out. Simultaneously the push crew were advising us they were ready for pushback. During the push, we received a message notifying us the confirmation message was not received. After the push, the captain sent the confirmation message. We then continued the flight normally.

Synopsis

A320 flight crew reported gate pushback was initiated prior to sending the company Dangerous Goods acknowledgment form to Dispatch. The acknowledgement form was sent once pushback was completed.

ACN: 2042200 (354 of 2908)

Time / Day

Date : 202310

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2660
Experience.Flight Crew.Last 90 Days : 184
Experience.Flight Crew.Type : 2660
ASRS Report Number.Accession Number : 2042200
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Dispatch
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Flight from ZZZ to ZZZ1. There was some difficulty achieving ACARS initialization, yet eventually we were able to uplink the flight plan and receive Takeoff data, etc. After Takeoff we received the reminder message to confirm the receipt of Dangerous Good (DG.) I called ZZZ OPS on the radio and they confirmed with the Ramper's that it had been loaded. We never received nor were told on the ground that DG had been loaded. I made a logbook entry that ACARS was intermittent on aircraft.

Synopsis

Air carrier Captain reported receiving Dispatch airborne message requesting the flight crew DG acknowledgement that DG cargo was loaded during pre-flight. Captain replied that no Ramp or ACARS information of DG cargo being loaded during pre-flight.

ACN: 2041377 (355 of 2908)

Time / Day

Date : 202310
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2041377
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arrival into gate, shutdown/parking checklist/deplaning normal. After a few minutes, Maintenance came on the flight deck off-handedly mentioning that I had a tailpipe fire and Fire Department (FD) had been notified and was on the way. Passengers were almost 2/3 deplaned at that point, I looked out the window of the jet bridge to see Maintenance personnel standing directly behind the tail cone of eng 1. I instructed the FO (First Officer) to have his Tailpipe Fire QRH ready. As personnel were directly behind the engine, I did not have the QRH procedure run. Maintenance came up to the aircraft again, taking the fire extinguisher from the FWD FA (Flight Attendant) station and heading downstairs. I followed them down and looked in the tail cone as they were getting the extinguisher ready, there was a small, but steady drip of oil that was on fire as it fell from the tail cone into the rear bypass area. Maintenance sprayed the extinguisher in the tail cone, the fire extinguished immediately, and I returned to the flight deck. FD arrived, took my information, asked if there was any sign of fire on the flight deck and whether there were any indications out of the ordinary, to which there was not anything unusual noticed by either flight deck crew. The maintenance crew should have a fire extinguisher handy in each vehicle for occasions like these. The extra time wasted going down to the engine and coming back up for the aircraftâ€™s extinguisher wasnâ€™t critical this time, but could have averted disaster had anything more dire happened. I did not elect to begin an evacuation because most of the passengers were already off and I think it may have caused more problems than solutions in regards to safety. Also adding to the decision was the lax, off-handed nature of the Maintenance personnel, to include other Maintenance personnel standing directly behind the source of the fire.

Synopsis

A320 Captain reported the aircraft experienced a tail pipe fire after parking at the gate.

ACN: 2040479 (356 of 2908)

Time / Day

Date : 202309
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2040479
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2040486
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 3

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
ASRS Report Number.Accession Number : 2040485
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 4

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2040480
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Person 2, Person 4 and I [Person 1] were assigned to Aircraft X. Landing gears doors were already in the down position. The Lead [Person 3] assigned us with removing the accumulator that was, causing a hydraulic leak. The Lead and I pulled breakers, depressurize the system, removing the head pressure. We removed the bottle placed on the table, replace the valve and packing reinstalled the bottle pressurize the system and the Lead pushed in all the breakers. then we refilled the green hydraulic reservoir. The accumulator still leaked, we received a new accumulator we replaced it, Person 2 and the Lead proceeded to go upstairs to the flight deck. The Lead was in an hurry to get the job done before night shift came on. The Lead stowed the manual gear drop and proceeded to turn on the system approx XE00 hours. At that time I was next to the green hydraulic system reservoir I went around to where the ladder that was inside the wheel well to look if the accumulator was leaking at that time before even knew it, the landing gear and ladder was lifting up and pushing me towards the landing gear wheel. My legs were caught in between the ladder. Person 4 next to me, screamed to get out of the way I wasn't quick enough. He tried radio over the radio to stop for me. I was able to get one foot out and get out of the way from the wheel, and then the ladder went up into the wheel well with the landing gear closing behind it. Suggestion: Slowing down.

Narrative: 2

There were three AMTs that were farmed out from the terminal to the hangar, we arrived at XA30 and were given a task to finish 2 service on another aircraft on the west ramp at spot X. At around XC00 we were then called out over the radio by our Lead to help find the problem as to why we had an air return on Aircraft X, the original write up was that the pilots reported losing all HYD pressure in the green system after takeoff causing him to manually drop the landing gear. The landing gear pins were already installed and we were told by Lead to standby, as he was going to pressurize HYDs in order to locate where the leak was coming from. We found the leak and were then instructed to begin removing the accumulator while the Lead went to locate and order our parts, we began to pull circuit breakers and depressurizing the green system in order to take the accumulator off. This was accomplished, aside from receiving parts which then arrived around XE30, which was just the joiner and the packing, after installing and leak checking, the leak was still present and bottle needed to be changed along with the joiner and packing, which we then were met by Person A, he assisted in finding the accumulator. after install, around XG00 Person 3 and I went to the cockpit to pressurize HYDS after getting the all clear From both Person A and Lead, Person 3 then stowed the manual gear down handle and pressurized HYDs, the gear doors then came up pinching Person 1 between the ladder and the MLG inboard tire, thankfully he was able to get out in time and only resulted in minor bruises and scrapes. Suggestion: The crew working the job should be the ones touching and carrying out the tests and work. We needed to be more safe when it came to locking out the gear doors, but the configuration of the plane wouldn't have been changed if the original crew was working the problem. No matter the time crunch.

Narrative: 3

Aircraft X had lost green sys hydraulic quantity after taking off flight crew accomplished a manual gear extension after an overweight uneventful landing aircraft was recovered by the ramp taken to a gate unloaded and eventually towed to the hanger MLG doors remained down and the manual extension handle still deployed sufficient hydraulic quantity remained so hydraulic power was applied to confirm source of leak I had assigned three technicians to replace the green sys hydraulic accumulator and ordered parts needed to fix the leak then my attention was on three other aircraft that were out of service including one that I had to take out for a run. I was the only qualified mechanic on shift to do so several hours later I stopped by to see how work was progressing on Aircraft X they had replaced a fitting and o ring on the accumulator serviced and re-pressurized the hydraulic system and it was still leaking we located the new accumulator and I assisted with its replacement then I went to the cockpit accidentally hit my knee on the manual gear extension handle so i cranked it to normal stowed position and with clearance applied hyd power and the gear doors came up and pinched a ladder that had been used for access causing damage to the door and to the ceiling of the MLG. Suggestion: Any time the MLG doors are down the collars / locks need to be installed and the manual maintenance access handles need to be pulled and safeties pins installed regardless of the condition or position of the doors are in when the aircraft is brought into the hanger or for that matter wherever the aircraft is because there had been three prior applications of hydraulic power prior to this event without incident and my distractions dedicated to other out of service aircraft and no prior experience on the aircraft type by the three new technicians i had assigned to this job all contributed to this very serious incident.

Narrative: 4

Person 1 and I were instructed by our Lead [Person 3] to come and look for hydraulic leak while he goes up in the cockpit to turn the pumps on. We found the leak coming from the green system accumulator and asked him to shut off the hydraulic pumps. He comes down and instructed us to depressurize the green system, and remove the on nut that is holding the accumulator in place. The MLG gear doors were already opened and there was other mechanics working on a different task (overweight landing). So the accumulator was removed after depressurization, a new o ring and fitting were installed and the whole thing was put back together. We did a leak check and it was still leaking. Lead ordered a new accumulator bottle and instructed us to replace it. After doing so, he goes up in the cockpit this time and he stowed the manual gear drop handle then he turned the hydraulic pumps on which made the gear doors go up crushing the ladder. Suggestion: I think the lead was in a rush and did not provide us with enough information for the job.

Synopsis

Maintenance technicians working on an A320 aircraft reported inadvertent gear retraction causing minor injury to another technician.

ACN: 2040310 (357 of 2908)

Time / Day

Date : 202310
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 114.8
Experience.Flight Crew.Type : 287.92
ASRS Report Number.Accession Number : 2040310
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We had just received clearance to push back into the alley from gate in ZZZ. I released the brakes, and informed the tug driver we were cleared to push. He read back the confirmation. We had not moved 5 ft. when we felt a sudden jolt and the aircraft stopped moving. I queried the tug driver about what happened, and he informed us that a ground vehicle had continued driving past the wing walkers behind the aircraft and that he had to slam on the brakes to avoid a collision. Due to less than ideal audio caused by an intermittent headset, I asked the tug driver twice to confirm that the aircraft did not impact any vehicle or equipment, and he confirmed twice. However, he stated that the towbar pin appeared to have been sheared due to the force of the abrupt stop. I set the aircraft parking brake and informed him of such. We then coordinated with hub maintenance for an inspection of the nose gear (which was signed off with no damage noted) and the ground crew retrieved a new towbar pin. The rest of the push and flight were uneventful.

Synopsis

Air carrier Captain reported a critical ground conflict during push back when a ground vehicle drove behind the wing walkers. The push back tug driver had to brake suddenly to avoid a collision and in the process the towbar pin was sheared.

ACN: 2039100 (358 of 2908)

Time / Day

Date : 202309
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2039100
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2039110
Human Factors : Physiological - Other
Human Factors : Time Pressure
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arriving at Aircraft X in ZZZ, the FO (First Officer) found the APU running and the APU bleed off. After ascertaining that the ground air was not connected, FO turned on the APU bleed to cool the aircraft. Immediately, the FO smelled dirty sock and announced that. The CA (Captain), who was entering the cockpit behind the FO, remarked that she immediately smelled it, also. The CA then walked back through the aircraft and determined that the smell permeated the whole plane. The inflight crew had not yet arrived at the plane, it was just the pilots at this point. We turned the bleed off, called Maintenance Control, and the CA wrote up a fume event in the aircraft logbook and began work on the required fume report. We probably did not get out of the airplane as quickly as we should have. When the tech arrived, he turned the bleed back on to perform his checks, and we exited the aircraft to reduce our exposure. The tech came up the jet bridge to chat with us and verified that in his opinion it was a valid fume event and the aircraft would be taken out of service. CA experienced some headache symptoms, FO had a bit of irritated sinus/throat feelings, but both of us felt well enough to continue when Operations swapped airplanes about an hour later. We proceeded to destination uneventfully.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

Air carrier flight crew reported a fume event described as "Dirty socks" during preflight. After Maintenance performed checks, the aircraft was removed from service.

ACN: 2038676 (359 of 2908)

Time / Day

Date : 202309

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2038676
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff the landing gear handle would not move to the up position. Then we got L/G shock absorber fault ECAM. I continued to fly while FO started ECAM actions. Moments later we also got autoflight, autopilot inop, and autothrottle inop ECAMs. Also Brakes, nosewheel steering inop ECAMs. FO cleared ECAMs and ran checklists and alerted tower that we would be [priority handling] aircraft returning to ZZZ. I continued to hand fly airplane and modified profile to set up for return. Elected to remain at flaps one 190 knots. I alert flight attendants to air return and notified the passengers. FO sent a message to dispatch regarding same. We told ATC we wanted long runway and equipment standing by due to loss of normal braking and nosewheel steering. Uneventful landing. We were able to stop with use of reverse mostly then alternate braking below 80 knots. Emergency responders checked the aircraft and then company tug towed us to gate.

Synopsis

A320 Captain reported multiple malfunctions after takeoff and returned to departure airport where the aircraft was towed to the gate due to loss of nosewheel steering.

ACN: 2037429 (360 of 2908)

Time / Day

Date : 202309
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 4748.62
Experience.Flight Crew.Last 90 Days : 184.43
Experience.Flight Crew.Type : 4748.62
ASRS Report Number.Accession Number : 2037429
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Asked by approach control if we had the field in sight for a visual approach. Neither myself or the F/O (First Officer) did. Controller cleared us to the final approach fix for the RNAV XX. We were at 3000 feet FAF was 2600 feet understood that we were cleared the approach -not the visual, so descended to FAF altitude. Controller came back with a low altitude alert. Climbed back to 3000 feet. Recleared for the approachâ€¦ started to descend for a segment of the approach. Controller once again asked about us descending to FAF altitude. Said to stay at 2700 feet got the field in sight and notified the approach controller. He handed us off to the tower and gave us a phone number to contact with Center.

Synopsis

Air carrier pilot reported ATC low altitude alert while maneuvering for approach.

ACN: 2033110 (361 of 2908)

Time / Day

Date : 202309

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 4649.87
Experience.Flight Crew.Last 90 Days : 152.93
Experience.Flight Crew.Type : 3706.35
ASRS Report Number.Accession Number : 2033110

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight started with a normal preflight, pushback, taxi out, and take-off. I was the Flying Pilot of this flight. Climbing through FL210 with autopilot and autothrust on, we experienced a ENG 2 Compressor Vane ECAM. I monitored the aircraft and took ATC communications while the First Officer accomplished the ECAM. I leveled off at FL230 while notifying ATC that we were have an engine issue. We did not [request priority handling] at that point in time. Shortly after this ECAM while the First Officer was going through the ENG2 Compressor Vane ECAM, we received an ENG 2 FAIL ECAM. I subsequently [requested priority handling] with ZZZ Center, and told them we needed to return to ZZZ. They responded quickly and gave us a turn back to ZZZ while we maintained FL230. The First Officer and I worked through the ECAMS and checklists associated with the above. As we were completing our checklists, the First Officer called the flight attendants to suspend the cabin service and advise them we were going back to ZZZ. I worked with ATC to give them information they asked for, souls on board, fuel quantity, etc as we progressed back to land. I briefed the flight attendants, told them that no prep of the cabin was needed, and that we should be on the ground in about 20 minutes. I made a PA announcement to the passengers and told them what had happened, what to expect, and that we were returning to ZZZ. ZZZ Approach set us up for a visual landing with ILS backup on [Runway] XX. A successful single engine approach and landing was accomplished. Fire/Rescue equipment followed us upon landing and inspected the aircraft after we cleared the runway. (We were approximately 6000 to 7000 lbs over max landing weight and the temperature was in the mid 90â€™s so I requested they inspect our wheels as the brakes were quite warm.) We were subsequently cleared to taxi to gate with firetrucks in tow. ZZZ Operations gave us our gate assignment and we taxied to complete the flight. After the aircraft was secured at the gate and parking checklist was completed, the First Officer and I took a couple minutes to debrief ourselves before opening the cockpit door. I then went into the cabin to make an announcement to our passengers about what to expect. I was able to coordinate with the gate agents and operations and we then deplaned, the passengers to re-accommodate them. I then debriefed the flight attendants to check on them. After a short time, Company Maintenance arrived at the aircraft. Of note was one of the mechanics that came onboard and made a comment that we really didnâ€™t have any engine problem or damage and that â€œWeâ€™ve seen this kind of thing before.â€ I thought that to be kind of an odd statement with the ECAM screen still showing ENG 2 FAIL. I canâ€™t speak for the First Officer, but I felt like he just blew off what had just happened with our aircraft. Enough said about that.

Synopsis

A320 pilot reported engine failure ECAM message and returned to departure airport.

ACN: 2032428 (362 of 2908)

Time / Day

Date : 202309
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb
Airspace.Class A : ZZZ
Airspace.Class B : ZZZ
Airspace.Class E : ZZZ

Component

Aircraft Component : Elevator ControlSystem
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2032428
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying and departed ZZZ Runway XXL. As I began climbing out, I noticed the aircraft required forward pressure on the side stick. Once I leveled off, and was level on the flight directors, the aircraft still required forward pressure to maintain level flight. The aircraft would constantly pitch up. We were given a higher altitude and I climbed to it. All the while having to push slightly forward on the side stick in the climb. I engaged the autopilot and it maintained the climb and then cruise. Once our workload was reduced I contacted dispatch and maintenance on the satellite phone. I told them that when the autopilot was off, the aircraft required forward pressure on the side stick. Both Flight Directors were indicating the same. Also, when the FO flew, he noticed and felt the same problem. Maintenance had me disconnect the autopilot while in level flight and I noticed the airplane again pitch up. We all agreed the safest course of action was a return to ZZZ. Maintenance also noted that as we burned fuel and the CG changed, the aircraft could be harder to fly manually. We notified ATC that we would be returning to ZZZ. We also [requested priority handling] as we had some sort of issue with pitch trim in manual flight. I used the [priority handling] call button and gave the flight attendants a heads up. I told them to expect a normal approach and landing. I let them know they would also see emergency vehicles on our landing roll out. We had a long final for XXL at ZZZ and configured early. As I turned the autopilot off, the aircraft still wanted to pitch up. We landed uneventfully back and ZZZ and taxied to our gate. I entered the issue in the logbook and also spoke with the two mechanics that met our aircraft at the gate.

Synopsis

A320 Captain reported experiencing constant uncommanded pitch up movement while on manual flight during climbout requiring forward pressure. The flight crew contacted Dispatch and Maintenance and decided to return to the departure airport.

ACN: 2030619 (363 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 199
Experience.Flight Crew.Type : 2999
ASRS Report Number.Accession Number : 2030619
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Due to a known problem with ACARS communication at gate in ZZZ, the initialization of the ACARS did not occur. We did not receive information about any dangerous goods on the flight. Not until we pushed backed out of the cone of confusion, that the data link was connected. We received two planned dangerous goods summary reports but never the final report indicating the package was loaded, I damaged and employee verifying such We did not send the DA (Dangerous Good Acknowledgment) report. The dangerous good was dry ice.

Synopsis

Air carrier Captain reported receiving ACARS DG message for two planned DG items during pushback but did not received the final DG summary indicating the DG cargo was not loaded. The flight crew did not send the acknowledgement code and departed safely.

ACN: 2029935 (364 of 2908)

Time / Day

Date : 202308

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2029935
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

During parking at Gate X at ZZZ the captain neglected to set the parking brake. The aircraft rolled backward, away from the gate approximately XX ft. The captain noticed this and applied the parking brake to stop the movement. I was completing my parking flow when I felt the aircraft move slightly. This motion was the result of the parking brake application. I did not notice the initial backward motion as it was slight and linear and I was focused on shutting down fuel pumps. The aircraft came to a stop. We checked with the flight attendants and ground crew. No injuries or disturbances of any kind were reported. We prepared the aircraft to be towed back to the parking line. We parked and completed the parking checklist. There was heavy rain at ZZZ. The visibility was low and the wipers were in use when we were parking. I believe shutting off the wipers interrupted the captainâ€™s parking flow and led to the omission. Anyone couldâ€™ve done it. Preventative measures are adherence to company procedures.

Synopsis

Air carrier flight crew reported aircraft movement while parked at the gate due to parking brake not set.

ACN: 2029740 (365 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2029740
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During push from gate X, everything was routine until tug driver said he wanted to disconnect the tug and reconnect it with the tug facing the same direction as the plane. Driver said he wanted to pull the aircraft forward a little to complete the push. Driver asked me to set the brake and so I did. I then watched the tug disconnect turn around and back toward the plane with its rear now facing the nose to be reconnected. However, as the driver was now on the opposite end of the nose we were no longer in comms with the driver. After a minute or so a new voice came on the headset and excitedly asked me to "release the brake!" so I released the brake, but when I did the plane lurched a little indicating that they must have been attempting to pull the aircraft with the brake set. After a min or so I was monitoring something in the cockpit awaiting the "set brake" command when FO (First Officer) states "I think we have been disconnected" . I looked up to see plane creeping toward tug which was still in proximity to the nose. I stepped on brake and saved plane from contacting tug. I saw one ground crew reprimanding another although it was not the tug driver who was reprimanding. I felt this push crew was completely disorganized and without a designated leader. We never got a "set brake" command and I was never able to give permission to disconnect. Although they did show me the pin as they walked away. This push crew was doing their own procedures unlike I have ever seen after decades at my company. If it had not been for FO noticing that plane was creeping and alerting me we may have contacted the tug.

Synopsis

A320 Captain reported the aircraft rolled forward after brake release when push crew failed to communicate and follow procedures for disconnect. Captain applied brakes to avoid hitting the tug.

ACN: 2029668 (366 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 15000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Lubrication Oil
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2029668
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2029688
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ENG 1 OIL LO PR ECAM during climb out at approximately 15,000 feet. We [advised ATC] and followed the ECAM which directed us to shut down the engine. We accomplished that and completed QRH follow up and related SOPâ€™s. We landed without further incident and taxied to the gate after ARFF (Airport Rescue and Fire Fighting) inspection. According to the maintenance person who met us at the aircraft, it appeared to be a mechanical issue. Following the prescribed maintenance programs should minimize occurrences of this type of event.

Narrative: 2

On departure while climbing through 15,000 ft, we received a triple chime warning for ENG 1 OIL LO PR. We completed the ECAM shut down engine 1. We [advised ATC] and requested vectors to ZZZ. We completed the QRH and diverted to ZZZ, landing on [runway] XXC without incident. After landing, maintenance discovered oil on the side of the wing and the oil quantity engine 1 completely depleted. Cause: Abnormal Procedures/Engine Failure Suggestion: ATC gave us an arrival to ZZZ which added to our task saturation during the emergency.

Synopsis

A320 flight crew reported a loss of engine oil during climb out that resulted in engine shut down and divert.

ACN: 2029079 (367 of 2908)

Time / Day

Date : 202308

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2029079
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On our descent into ZZZ we received ECAM ENG 1 Compressor Vane repeatedly. The subsequent fault was written up and it was noted that in the MEL cross reference list stated â€œNo Dispatchâ€. Maintenance came out and was instructed by Maintenance Control to perform a reset and then run the engines. The engines were both run for 2 minutes with no faults and the discrepancy was cleared. The plane was boarded and during our crew brief, the first officer and I collaborated about the fault in the event the fault came back during the takeoff. Since ZZZ is a short runway, I lined the aircraft up and held the brakes for a static T.O. FLEX thrust was applied and the T.O. roll began. The best estimate was that the fault occurred between 80-100 knots. The same fault came up as in the previous flight except line item 1 was different. The first officer read off â€œEngine 1 Thrust Lever Idleâ€, and at that time the Takeoff was Aborted. After we returned to the gate we were able to talk to the FAâ€™s (Flight Attendant) and they brought up that after we started moving they noticed a popping sound coming from the left side of the aircraft. I had the FO (First Officer) go check the #1 engine for any physical damage but he reported that that he did not see any. When serious faults that deal with major components occur, it might be prudent to take a more strict approach than just doing a soft reset and sending the aircraft on its way.

Synopsis

Air carrier Captain reported a safe rejected takeoff executed due to an ECAM ENG 1 Compressor Vane light illuminated during takeoff roll. The Captain stated this anomaly has been a recurring issue.

ACN: 2028951 (368 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 5590.8
Experience.Flight Crew.Last 90 Days : 42.62
Experience.Flight Crew.Type : 5520.22
ASRS Report Number.Accession Number : 2028951
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Just prior to top of descent into ZZZ, we received an ECAM for loss of the green hydraulic fluid. ECAM directed us to turn off the PTU (Power Transfer Unit) and the green system engine driven pump. The system page showed the green system quantity at zero. We then discussed the landing implications without the green system. A phone patch with Dispatch and Maintenance confirmed the best decision was to land at ZZZ1 due to the longer runway. With no green system, we had to manually lower the gear, would not have left thrust reverser, no nose wheel steering and be on alternate braking. We [requested priority handling] with ATC and coordinated that we would be stopping on the runway. After gravity extending the gear, the approach and landing were uneventful on XXR in ZZZ1. Maintenance and fire/rescue met us on the runway. We shut down and were towed to the gate.

Synopsis

An A320 First Officer reported a loss of hydraulic fluid and diversion to an airport with a longer runway.

ACN: 2028774 (369 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2028774
Human Factors : Troubleshooting
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During final descent in ZZZ flight attendants advised crew of a portable battery fire in the cabin. Flight attendants called back moments later and advised that the situation was resolved. The plane landed uneventfully. Upon arrival at the gate local maintenance was called and a report was submitted.

Synopsis

Air carrier pilot reported flight attendants advised there was a portable battery fire in the cabin during descent. Moments later the flight attendants reported the issue had been resolved and the flight landed safely at destination.

ACN: 2026903 (370 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2026903
Human Factors : Time Pressure
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climb out from ZZZ passing 12,000 ft. the aircraft experienced a pressurization event which included an abrupt pulsating increase of air flow into the cockpit and cabin. The air flow was accompanied by a loud rush of air and debris flying about the cockpit. We immediately stopped the climb and requested a descent to 10,000 feet. The cabin vertical speed was fluctuating with the pulsing of the airflow the hot air valve was noted in the closed position as per MEL. A call to the cabin confirmed the same conditions occurring in the cabin. The Flight Attendants were advised of our return to ZZZ for a normal landing. With several symptoms of a depressurization event we quickly got clearance to return to ZZZ for an uneventful landing and return to the gate. All normal checklists were completed. Landing data was received. The overweight landing checklist was referenced and complied with. Dispatch was advised via ACARS

Synopsis

Air carrier flight crew reported the loss of aircraft pressurization during climb. The QRH procedures were completed and received clearance to departure airport where a safe overweight landing was completed.

ACN: 2025749 (371 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2025749
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Prior to departure the airplane was dispatched with one pack on MEL. One of the requirements of that MEL is operational flight spoilers. SEC 1 failure with unsuccessful reset caused the failure of flight spoilers 3 and 4 on each wing. Accomplished ECAM and QRH procedure with no success of regaining SEC 1. SAT call to Maintenance Control, resets unsuccessful. Patched in with Dispatch and determined ZZZ as the best airport for diversion. I determined it necessary to [request priority handling], descend to 10000 feet and divert, due to the loss of some flight spoilers and the aircraft's capability to comply with the requirement of reaching 10000 feet in the required time in the event of the loss of the second pack or any other failure or incident that would require a descent.

Synopsis

A320 pilot reported failure of one spoiler computer during cruise. The aircraft was dispatched with another discrepancy on MEL, in which this partial spoiler failure created an operational conflict with the current MEL, to perform a descent, if needed, in a timely manner. The aircraft diverted safely to an alternate airport.

ACN: 2025233 (372 of 2908)

Time / Day

Date : 202307

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Component

Aircraft Component : Flight Dynamics Navigation and Safety
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11469.48
Experience.Flight Crew.Last 90 Days : 167.38
Experience.Flight Crew.Type : 11469.48
ASRS Report Number.Accession Number : 2025233

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cleared ILS XXC 210 kts given 170 FO (First Officer) flying pulled the nose up and full speed brakes airplane had a significant buffet. There were no warnings, not even close to VLS (Minimum Selectable Speed). Buffet felt like prestall buffet. Told FO to lower nose and retract speed brakes, buffet stopped. Iâ€™ve been in touch with Person A because this is not normal. There should be some warning, speed, alpha floor, VLS increase but there was nothing. Airplane should not ever get into a prestall condition in normal law. FYI Iâ€™m not 100% sure this is the flight because itâ€™s been a while.

Synopsis

A320 Captain reported possible control law malfunction during approach.

ACN: 2023714 (373 of 2908)

Time / Day

Date : 202308
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Component

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2023714
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On short final at approximately 1500 ft. MSL generator 1 failed. First Officer started APU. Approximately 10 seconds after generator 1 failure, generator 2 failed. Ram Air Turbine extended and APU generator came on line. [Requested priority] with ATC. Landed ZZZ uneventfully. Fire crews inspected aircraft and found no obvious issues.

Synopsis

Air carrier Captain reported both generators failed during final approach.The flight crew activated the backup electrical power and executed a safe landing.

ACN: 2022066 (374 of 2908)

Time / Day

Date : 202307

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Design

Person

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2022066

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

Report subject â€“ Left C-Duct Thrust Reverser Harness. Reason for report. To correct and update IPC (Illustrated Parts Catalog) parts lists, assembly instructions, and associated illustrations. Prevent wrong part number installation and possible malfunction of associated wiring and aircraft thrust reverser system. Engineering request to revise the associated work order has been made and corrections have been agreed to and process to revise has been initiated. Primary issue is for the backshell on Connector 905P. The harness installation requires a 90-degree backshell for Connector 905P per IPC. The 90-degree backshell provides less bend radius for the wire versus a straight backshell which would make it susceptible to higher stress and possible malfunction of the wire. Associated parts lists and illustrations show a straight backshell but should reflect a 90-degree backshell on all IPCs and illustrations. Connector 905P connects to the left thrust reverser stow and lock proximity sensor. This is a critical indication for the flight crew. Suggested resolution â€“ work with Airbus, Company A, Company B to correct and update IPC parts lists, assembly instructions, and associated illustrations. Report subject â€“ Right C-Duct Thrust Reverser Harness. Reason for report â€“ to correct and update IPC (Illustrated Parts Catalog) parts lists, assembly instructions, and associated illustrations. Prevent wrong part number installation and possible malfunction of associated wiring and aircraft thrust reverser system. Engineering request to revise the associated work order has been made and corrections have been agreed to and process to revise has been initiated. Primary issue is for the backshell on Connector 904P. The harness installation requires a 45-degree backshell for Connector 904P per IPC. The 45-degree backshell provides less bend radius for the wire versus a straight or 90-degree backshell, which would make it susceptible to higher stress and possible malfunction of the wire. Associated parts lists and illustrations show a straight or 90-degree but should reflect a 45-degree backshell on all IPCs and illustrations. Connector 904P connects to the right thrust reverser stow and lock proximity sensor and is a critical indication for the flight crew. Suggested resolution â€“ work with Airbus, Company A, Company B to correct and update IPC parts lists, assembly instructions, and associated illustrations.

Synopsis

A320 Maintenance Technician reported that the incorrect part was listed in the Illustrated Parts Catalog (IPC) and needs to be corrected and updated. The installation of the incorrect part as instructed in the current IPC could lead to possible malfunction of the aircraftâ€™s thrust reverser system.

ACN: 2021892 (375 of 2908)

Time / Day

Date : 202307

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1839
Experience.Flight Crew.Last 90 Days : 144
Experience.Flight Crew.Type : 1839
ASRS Report Number.Accession Number : 2021892

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

I advanced the thrust levers to 1.05 EPR (Engine Pressure Ratio). Engines stable. I then advanced the thrust levers. Engines started accelerating properly. CL (Climb Detent) and later Engine 1 advanced normally. Engine 2 EPR hung about 2/3 low. Aircraft yawed, EPR 2 not advancing. I believe I was out of the keep out zone. I rejected the takeoff at 40/50 knots. I briefly considered Max thrust. However with #2 not accelerating at all I was concerned about #1 continuing to advance to Max and #2 not accelerating. We did get a quick ding and ECAM flash. It might have been "ENG THR levers not set". Regardless, it was unsafe to continue and advance to MAX and the RTO (Rejected Take Off) was underway. After review, maintenance presented a company bulletin. We decided it would be safe to attempt another takeoff but in MAX. On next take off, #2 did not advance as quickly. But it was smooth, stabilized at Max and I decided to continue. In hindsight, even though I feel it was a safe maneuver, I wish I would have accomplished another RTO. Of course I have more information now. The jet was grounded in ZZZ1 and after an engine run it looks like the P2T2 probe was an issue.

Synopsis

A320 flight crew reported engine acceleration anomaly during takeoff roll resulting in a rejected takeoff.

ACN: 2021714 (376 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Descent
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Component : 2

Aircraft Component : Pressurization Outflow Valve
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2021714

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X Inbound to ZZZ at 24,000 ft. straight and level we started having a pressurization issue. We recognized the pressurization issue due to our ears popping and feeling pressure. We looked at the pressurization screen and seen the cabin altitude at negative 4,400 ft. and the VSI moving erratically up a down reaching negative 3,900 ft. at one point. This was followed by a master caution CAM CAB PR SYS 2 FAULT. The outflow valve showed completely closed, and the safety valve showed completely open. We followed the ECAM to manually operate the outflow valve by using the manual vertical speed. This however, did not work. We did an urgent decent from 24,000 feet to 8,000 ft. and [requested priority handling] straight to ZZZ. We continued to troubleshoot the problem with no success. Upon landing on Runway XXR in ZZZ, making a turn off, our ears were still popping and feeling pressure. The cabin altitude still showed negative 4,400 ft. and a differential pressure psi of 2.5. After turning both packs off this still did not fix the problem. I asked the First Officer (FO) if he could open his cockpit window to depressurize the aircraft. The First Officer was unable to open his window. I was finally able to open my window and depressurize the airplane. We continued to taxi to the gate. No injuries reported just inner ear discomfort from Crew and Passengers. We filled out the logbook and gave a full brief of our issue to maintenance and left the aircraft.

Synopsis

A320 Captain reported pressurization problems and being unable to rectify the issue with manual mode as per ECAM. After landing crew was unable to depressurize the aircraft on the ground until a cockpit window was opened after air conditioning packs were selected to off.

ACN: 2021140 (377 of 2908)

Time / Day

Date : 202307

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked

Person : 1

Location Of Person : Repair Facility
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2021140
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Location Of Person : Repair Facility
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2021141
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 3

Location Of Person : Repair Facility
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 2021142
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Fuel Issue
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Supervisor assigned job to a late part that arrived to aircraft. All three mechanics were unfamiliar with job but were aware of the danger of fuel spilling. Arrived at aircraft, pulled breakers before turning on power, and created logbook write-up shorty after. Turned on power, noticed it had 3000 fuel, and asked older mechanics about defueling but after looking into the AMM (Aircraft Maintenance Manual). There was nothing that had details about transferring fuel beforehand, so continued. Proceeded to use the tool referenced in the manual to unseat the valve and started turning. There was a little fuel dripping from the seal as we turned it counterclockwise but not nearly enough for the manual that showed maximum of one bucket, so we kept turning until it started to have a lot of resistance and then proceeded to open the drain plug because then manual said to after the turning stopped. The drain plug showed no signs of fuel so we kept turning because the manual said only in steps beforehand to check the fuel flowing from the valve only after taking the assembly off. As we were turning the valve the keeper lock fell off from the body and dropped into the bucket and then the valve gave out, dropping the cover into the bucket and with a lot of resistance. We managed to lock the valve back in partially. I then had one of the mechanics go tell the Lead that we needed help and we tried to get the valve back in but it was locked in place. To add to this all three mechanics were reading the manual independently and wasnâ€™t following one single person reading off the manual. The reference to change the fuel pump had very vague points and at one point told the Mechanic after taking the fuel pump off to gauge the amount that would possible to work with. The manual should reference fuel transferring beforehand to prevent larger spills along with a maximum amount of fuel total before using a fuel truck to drain the fuel if there is too much to transfer all in one wing. The manual highly highlighted the danger of cutting safety wire in red but had other warning orange.

Narrative: 2

When we were working on Aircraft X, my coworkers were changing left wing tank inboard boost pump. I was helping them after I had done my jobs on the aircraft. We followed AMM (Aircraft Maintenance Manual) steps to remove, but the boost pump is stuck and hard to remove. While I was reviewing the AMM to find a solution related to this problem, other technicians removed the pump but fuel was coming out from the bank. They tried to put the pump back to the tank to stop the fuel, but that did not work. So I helped them put the bump back, also did not work. Because we could not stop and fuel leak, I went to grab fuel leak kit to clean the fuel. At the same time we notified the supervisors.

Narrative: 3

I was following fuel pump AMM (Aircraft Maintenance Manual) task removal. Once the pump was removed fuel came out after following step in accordance with AMM. Fuel was out on ramp. Then, we tried to reinstall pump but could not because retaining pin and spring assembly failed. I ask did we have to defuel but the AMM didnâ€™t call for a defuel.

Synopsis

A320 technicians reported the Aircraft Maintenance Manual (AMM) did not clearly provide defuel instructions for a fuel pump removal procedure. After removing the pump, fuel began to leak, spilling all over on the ramp.

ACN: 2020522 (378 of 2908)

Time / Day

Date : 202307

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2020522
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2020824
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On my way to the airport, I got a call from another ZZZ1 CA (Captain). He was being tail swapped out of an aircraft that had a fume event. Neither of us knew that I was about to be swapped into that very aircraft. Upon checking in, I noticed a gate change and tail swap. When I got to the aircraft, there was a lot of confusion. Incorrect flight release, unknown status of security search, FAâ€™s (Flight Attendant) had just been tail swapped. Because of this, I didn't thoroughly review the logbook for some time. Upon review, I noticed that the aircraft had JUST been written up for a fume event, with a perfunctory, "APU OPS check ok," as the corrective action, and I saw that the entry was made by this other ZZZ1 CA. At the same time, the Lead FA asked if I was aware that the aircraft had had a fume event and the prior crew tail swapped. I called for MX (Maintenance) and made a call to the previous CA. Maintenance indicated that they had, "run the APU full hot and full cold," and that it was, "fine." I spoke with the previous CA and he described the nature of the fume event and that it hadn't actually occurred to him, the aircraft still smelled like one. I spoke with a Chief Pilot. The Chief Pilot, MX, my FO (First Officer), and I all concluded that without any actual present indications of a problem, it was safe to make the flight with extra vigilance for a potential fume event. We flew to ZZZ uneventfully. Upon arrival in ZZZ, the FO and I went to get some food. Upon returning to the aircraft, I was hit with a wall of noxious, acrid fumes that were immediately irritating and obviously the classic presentation of a fume event. The APU was shut down and no external air was connected. I exited the aircraft and advised the FA contingent to do the same. We contacted Maintenance Control, Contract Maintenance came out and began troubleshooting. Maintenance Control asked me to perform an engine run and I declined. There was already a presence of noxious fumes of unknown origin and I was disinclined to continue to expose myself, my FO, or anyone else to them. Contract Maintenance said that he perceived the fumes on PACK 1 while running the APU, so the decision was made to defer the APU. I was generally OK with that, but was immediately confronted with even worse, persistent fumes after APU shutdown. I consulted with Maintenance Control and alerted them that the problem was not at all solved by deferring the APU. The FO, Contract Maintenance, Maintenance Control, a Chief Pilot, and I all consulted and agreed that based on the information we had, contamination in PACK 1 was the next logical culprit. We discussed the logic of that decision, contingencies for any further problems, and evaluated it as reasonable and safe. We all eventually timed out and were rescheduled.

Narrative: 2

I was on Aircraft X and experienced a fume event. I got on Aircraft X in ZZZ1. The Captain noticed that the aircraft had just been written up for a fume event, but that it had been cleared by Maintenance without any deferral as they were not able to duplicate it. He called Maintenance and asked why nothing had been deferred, and they told him the previous crew said that only ground air was hooked up when the fumes occurred. Maintenance disconnected ground air and had run the APU and there were no fumes they could detect. The Captain also called a Chief [Pilot] to ask how that could happen if just on ground air and to get a second opinion on accepting the aircraft. Ultimately he accepted the aircraft. We operated flight from ZZZ1 to ZZZ in that aircraft without incident. After landing, we turned on the APU and let it run its required three minutes before utilizing the bleed air. We utilized bleed air while parked at the gate while the passengers were deplaning. The ground air was not hooked up to the aircraft because the ground crew believed it to be inoperative. After the parking checklist and after all passengers had deplaned, the Captain shut down the APU. The Captain and I left the airplane to find lunch in the airport. We had a fairly tight turn around, and the Captain went back to the airplane to prepare it while I waited for the fast food. I went down the jet bridge a short time later and found the Captain at the bottom of the jet bridge along with all the Flight Attendants. He said he smelled fumes on the aircraft and told me not to board. He called Maintenance Control for a fume event. Maintenance Control asked if he would do an engine run to isolate the PACK, but the Captain did not want to do that and wanted to wait for Maintenance. Once Maintenance showed up, they ran the APU, but could not isolate a smell. The Maintenance personnel then asked if we wanted to board and check for ourselves. The Captain declined. I felt like we were at an impasse because Maintenance personnel couldn't reproduce the smell and the Captain didn't seem to want to get on the aircraft to do a "sniff test." I did stick my head in the aircraft cabin and I couldn't smell anything either, which I reported to the Captain. Maintenance then said they would keep trying for a little while longer to see if they could reproduce the smell. I think the Captain felt like he was in an awkward position because he felt like he was going to potentially have to board an aircraft with passengers and he was afraid we would then get a fume event. He had the idea to call Medical to see if it was advisable to board the aircraft with the potential for fumes. He did speak to Medical and reported that he was exposed. I told Medical that I had not been exposed at that time. While he was on the phone, Maintenance personnel came back out of the aircraft and said he did get a slight fume smell. The mechanic talked to Maintenance Control and they decided to defer the APU. The Captain agreed to that plan. While Maintenance was shutting down and deferring the APU, we all (the flight crew) walked up to the gate area. I had to talk to crew scheduling about getting an extension to my flight duty period, and talk to the gate agent about organizing an air start cart, while the Captain updated the passengers. All that took about 10 to 15 minutes. We thought it would be safe for the flight crew to get back on the aircraft and resume our duties after APU deferral, so we headed back down the jet bridge. Maintenance was already off the aircraft. I was the first one on the aircraft and I immediately noticed a very strong acrid smell right as I entered. [I was in the Army and had to do the CS gas chamber (tear gas) for training. The best way I can describe the smell is that it was almost the exact same smell as tear gas]. I did a quick assessment that it was strongest in the cockpit and cabin rows 1 through 4. I relayed that info to the Captain, who was just about to board. He also came on and confirmed that. We grabbed the Maintenance can and it showed the APU deferral had already been signed off. It must have been after APU shutdown again that the fumes returned. We again left the aircraft; there was no way to board the aircraft in that condition. Ramp crew got the external air working and hooked it up. The flight attendants then opened all the aircraft doors, and the Captain opened the cockpit windows. We left the aircraft, getting back on every so often to see if the smell had gone away, but it was intermittently persistent. The Captain got back on the phone with Maintenance Control to discuss further options such as deferring PACK 1. My FDP (Flight Duty Period) expired before Maintenance personnel could return to our aircraft.

Synopsis

Air Carrier flight crew reported a recurring fume event during pre-flight. As Maintenance was formulating a plan to defer the APU and/or Pack 1, the flight crew timed out resulting in the flight being rescheduled.

ACN: 2019880 (379 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Descent
Flight Phase : Final Approach

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2019880
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Passenger
Detector.Person : Flight Attendant
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon landing in ZZZ and taxiing clear of the runway, my lead FA (Flight Attendant) called and notified me that they were experiencing a fume event in the cabin. Multiple passengers and all of the flight attendants were feeling the effects of it. Flight Attendant B felt ill enough that she vomited. I immediately shut off PACK 2 and we initiated the Procedure for removal of odor/fumes. The Lead Flight Attendant reported that she had experienced symptoms as early as 30 minutes prior to landing. As this was during descent and arrival, there were multiple power changes throughout the time period. After blocking in at the gate, I shut down engines and kept all bleeds and PACKS turned off. We notified Maintenance Control, completed the safety form, notified the Chief Pilot, and assisted the cabin crew in obtaining medical assistance. I worked with the Maintenance crew to explain what had happened. After several cycles of the APU Bleed, I got back onboard to help Maintenance identify whether the odor was cleared out. It was not and I helped them identify the specific smell that indicates a TCP (tricresyl phosphate) generated fume event. At that point, I left the aircraft and helped coordinate for the cabin crew to receive medical attention. Neither myself nor my First Officer felt any ill effects during this event.

Synopsis

Air carrier A320 Captain reported being notified at destination arrival of a fume event in the cabin. Maintenance was requested and medical attention provided for the cabin crew.

ACN: 2019750 (380 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Flight Phase : Landing

Component : 1

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Component : 2

Aircraft Component : Thrust Reverser Control
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2019750
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon landing in ZZZ, I the flying pilot selected idle reverse. We got a green Rev on engine 1 and a yellow Rev on engine 2. After taxing off the runway I went back into idle and received an engine 2 reverser fault and engine 2 Reverser Unlocked ECAM message. We asked to taxi somewhere to run some checklists. The ECAM called to shutdown Engine 2 which we followed. After all checklists were complete we started to taxi to the gate. Shortly after I started to taxi I lost steering and all screens went black. I stopped the aircraft on the taxiway. It appeared we lost generator 1. Our APU was on MEL, Engine 2 was now inop and was left with battery power. I could hear engine 1 running but it seemed to be higher than idle based on sound but nothing could be confirmed with no engine indications. I decided to shut down engine 1 as a precautionary shutdown not knowing its current conditions or any parameters on the engine. We contacted ATC and advised we will need to be towed in. We contacted Ops to coordinate the tow in. We arrived just short of the gate with battery power when I turned the batteries off as they died. APU, engine 2 reverser, gen 1 fault had previous write ups. Maintenance write ups were done.

Synopsis

A320 pilot reported receiving fault messages for both thrust reversers after deployment on landing. During taxi in, steering and all electrical power was lost except the battery which also failed as the aircraft was being towed to the gate.

ACN: 2019080 (381 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 17000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component : 1

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Electrical Distribution Relay
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2019080
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Troubleshooting
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Climbing through about 17000ft - FL190, we heard multiple clicking/banging sounds coming from the main BUS circuit board (Relays). At the same time, we quickly saw multiple amber Xs and screens flicker, but was too quick to see which ones. Shortly after we saw it again and it was clear that multiple relays seem to be malfunctioning, but more this time. I was able to see over the next couple occurrences that the TAT (Total Air Temperature) and SAT (Static Air Temperature) indicators, CA (Captain) outlet, FMGS1, ND1 (Navigation Display 1), VHF1 on FO (First Officer) side also went out temporarily. I called back to ask the FAs (Flight Attendants) if they noticed anything, and they hadn't yet, but on the third occurrence, they reported the WIFI and the lights were flickering. About the 3rd or 4th occurrence (they happened quickly in a fairly short span of time) that my FMGS1 was now out as well as the Fuel Used numbers were reset and Fuel used for engine 2 indicators also ambered out. At this point I had asked the lead FA to do a sweep of the cabin to see if they see or smell smoke or hear anything suspicious. I had remembered that a pattern of electrical anomalies can be a possible sign of electrical fire, so I had begun to plan for that. Shortly after the lead FA said that there was a strange buzzing sound over row XX (where the Wi-Fi is) and a strange smell of possible smoke. At this point while still on the line with the FA, I gave him a briefing, and as I had seen ZZZ as the closest airport, fortunately with maintenance as well, telling him that is where we are going, [advising ATC] (while coordinating with the FO who was the flying pilot). At this point, I had decided to prepare for a possible electrical fire, and we rapidly descended from our cruise altitude of FL370. We were about 10 min away and did our duties preparing for the landing, calling the fire trucks as a precautionary method. As electrical fire is a dire situation, I was not going to hesitate, as dispatch and Maintenance Control was not fully aware of what was going on, although I did briefly send a few ACARS messages explaining it. They had suggested ZZZ1, but it was about 30 min away. I called for ZZZ while dispatch/Maintenance Control coordinated well to prepare for the landing in ZZZ. We landed safely with a full stop on the runway, completing all SOPs. Fire trucks pulled the infrared sensors and noted no heat anomalies, and we were cleared to the gate. On arrival Maintenance Control stated they received a message from our aircraft that the ENG1 IDG (Integrated Drive Generator) had failed. We had not seen that message, and while it may have popped up briefly, I did hit the recall button multiple times showing normal during the events on the flight. We had deplaned immediately on gate arrival; fire fighters did a full infrared/temperature screening twice of the interior and exterior and found no heat anomalies. It was concluded that the ENG1 IDG had or was failing, so we did an engine run to confirm and deactivate the IDG.

Synopsis

Airbus A320 air carrier pilot reported electrical problems while climbing to cruise altitude. The crew coordinated with the cabin crew to troubleshoot the problem, diverted, and landed safely.

ACN: 2018728 (382 of 2908)

Time / Day

Date : 202307

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Nav In Use.Localizer/Glideslope/ILS : ILS
Flight Phase : Climb
Route In Use : Direct
Airspace.Class B : ZZZ

Component

Aircraft Component : Lubrication Oil
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2018728
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload
Human Factors : Time Pressure

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2018729
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon climbout through 18,000 ft. we received an ECAM for LS tuning disagree. When following the Flight Crew Operations Manual (FCOM) procedure for the disagreement through FL210 we got an Adivsory indication flashing on the E/WD. The ENG page was up on the SD and QTY was flashing. The oil quantity was quickly decreasing from 3 qts. Within a few seconds while looking up the ECAM Advisory for oil quantity, I started noticing the oil pressure had decreased from 44 psi then started fluctuating from 30 psi to 25 psi and then started quickly dropping. The oil quantity indications had dropped and amber xx's replaced the indication. Within a few seconds the oil pressure dropped below red line and the Master Warning with repetitive chime came on. ECAM ENG 2 Low Oil PR appeared. We then proceeded to follow the ECAM actions. The ECAM directed us to shut down ENG 2. We then went through each secondary failure listed with the engine shutdown. After the engine was secured and preliminary ECAM actions were completed we stopped ECAM, leveled off at our assigned altitude of FL230 and I contacted ATC [requesting priority handling] due to having to shut down ENG 2. I requested to divert to ZZZ due to the facilities available and the longer runways. While talking to ATC the First Officer (FO) talked to the lead Flight Attendant (FA). ATC directed us to fly heading 150 and descend to 13,000 ft. We then ran the approach checklist and set the plane up for approach into ZZZ. The FO ran the landing performance for single engine Runway XXC. I requested Runway XXC due to the length of the runway. While I was flying and talking to ATC the FO was going through the FCOM procedure for ENG Low Oil PR ECAM. I made an announcement to the passengers that we had to shutdown the engine due to a mechanical issue and were diverting to ZZZ. I asked the passengers to remain seated and we would update them once we landed in ZZZ. I requested vectors for the RNAV XXC. While descending, the FO continued going through the overweight landing checklist and then the single engine straight in approach checklist. After the checklist were completed, we requested vectors to the approach. We landed normally with no issues and then taxied off the runway under our own power. We performed the after landing checklist and turned on the brake fans as instructed by the overweight landing checklist. I stopped on the taxiway just clear of the runway and emergency personnel drove around the aircraft and checked the #2 engine. The emergency personnel reported back that the #2 engine was leaking fluid. We were then instructed by ground to taxi into the ramp area and they wanted us to go to Gate XXX.

Narrative: 2

Shortly after passing FL180 we received NAV LS Disagree ECAM. After that was resolved Advisory was flashing and our engine oil quantity indicator was flashing. We began loosing quantity and pressure then the ECAM Eng 2 Oil Low Pressure displayed along with the master warning and received repetitive chimes. We followed the ECAM instructions which instructed us to shutdown the engine. After running the ECAM we followed the Flight Crew Operations Manual (FCOM) procedures for ENG 1(2) OIL LO PR followed by ENG 1(2) SHUT DOWN, QRH One Engine inoperative straight in approach and QRH Overweight landing. We [requested priority handling] early on and committed to landing at ZZZ for the longer runway due to being single engine and overweight.

Synopsis

A320 flight crew reported the R Engine on was shutdown inflight because of a loss of oil quantity and pressure. The pilots successfully diverted to a nearby airport.

ACN: 2018413 (383 of 2908)

Time / Day

Date : 202307

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2018413
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2020500
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon taxiing to the runway, everything was standard and as expected. It was my First Officerâ€™s leg and the ZZZ Tower cleared us for take-off. After we taxied onto the runway and completed the below the line section of the Before Take-off Checklist, I asked my First Officer if he wanted it on the roll. He confirmed that he did and the take-off roll was initiated. I called thrust set and upon reaching 80 kts, I called 80 kts. My First Officer was silent. After about a second or two (at about 90 kts), I quickly glanced at him and realized he looked confused. I then looked at his airspeed indicator and noticed it said 40 kts. I immediately initiated the abort at that exact same moment my First Officer said, "My airspeed doesn't look right." The spoilers deployed and MAX auto-brakes engaged. The aircraft was stopped and I told the passengers on the PA to remain seated. We cleared the runway and stopped, and I had my First Officer roll the trucks just in case the hot brakes would've ignited. The ARFF (Airport Rescue and Fire Fighting) vehicle told us that the brakes were hot, but there was no smoke or fire present. I called Dispatch and Maintenance Control to advise them of what happened. Maintenance agreed that we needed to go back to the gate. My First Officer coordinated for a gate with Operations and the ARFF vehicles chose to follow us to Gate X. The aircraft was shutdown, the Parking Checklist was accomplished and a maintenance logbook entry was made. There is nothing that I can think of that I would've done differently.

Narrative: 2

Normal SOP until take-off power was applied. I applied 50 percent first, released the breaks, and positioned the thrust levers at the climb detent. At the 80 kts call out from the PM (Pilot Monitoring); I looked at my airspeed and hesitated. My airspeed was at 40 kts with a positive trend vector. Due to the hesitation, the PM looked at my PFD to verify. I said, "this doesn't look right." At that point the Captain (PM) took control of the aircraft and completed the rejected take-off procedure. I estimate that the procedure was started at about 90 kts. Once the airplane came to a stop, I informed the Tower that we had aborted the take-off roll and that we can taxi off the runway. We did request to have the airplane/ breaks inspected by ARFF (Airport Rescue and Fire Fighting). Our highest break temperature was at 405 [degrees] C. After a few minutes, we returned to the gate. Maintenance performed several tests and concluded that the ADR 2 was faulty. We swapped airplanes and continued to ZZZ with about a 4.5 hour delay.

Synopsis

A320 flight crew reported a disagreement between airspeed indicators during take-off and rejected take-off at approximately 90 kts. Crew returned to gate for maintenance after airport responders inspected brakes.

ACN: 2018372 (384 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y

Component : 1

Aircraft Component : Cockpit Door
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Exhaust Pipe
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2018372
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Found cluster of 9 damaged rivets consisting of 1 missing rivet, 8 popped rivet heads on #2 engine exhaust on Aircraft X. Documented discrepancy and called Maintenance in ZZZ. Mechanics (were) showed to Aircraft X. I walked down with them and pointed out the location. They took photos and took Aircraft X out of service. We swapped planes. Next day I looked in our company's app to read the corrective action and it stated "could not locate unsecured fastener". I also noticed this aircraft was returned to service and was flying. I immediately contacted the appropriate safety department who are actively working this issue while I am writing this time sensitive report. I have filed multiple maintenance reports in the past. I believe the maintenance department as a whole routinely ignores, does not properly inspect, falsifies documents, and attempts to intimidate pilots into not properly documenting discrepancies. I believe a culture change needs to happen starting at the top from this department. Culture change in maintenance department to prevent pilot intimidation. Second report regarding Maintenance on same flight on same plane. Aircraft X. After filing report for damaged rivets on #2 engine being improperly documented and addressed, I read our company's app for other discrepancies. I found a second discrepancy that was assigned a NEF (Non-Essential for Flight) improperly. They assigned a NEF for "cockpit trim" (which includes paint, placards, markings. Glare-shield padding or covering) to a broken cockpit door panel. This is obviously wrong this also solidifies my point that these are not accidental human errors. These are intentional falsifications of the logbook. Lack of oversight of maintenance department. Intentional disregard for safety. Intentional noncompliance of SOP's. I have filed multiple maintenance reports in the past. I believe the maintenance department as a whole routinely ignores, does not properly inspect, falsifies documents, and attempts to intimidate pilots into not properly documenting discrepancies. I believe a culture change needs to happen starting at the top from this department. Per my previous report, a complete safety / culture overhaul of maintenance department is necessary. After filing reports in regards to this unairworthy aircraft's discrepancies being improperly documented, inspected, and ultimately repaired, I noticed this aircraft was flying via our company's app. I immediately took these steps: 1. Contacted the safety department. XA:00 2. Filed report. XB:00. 3. Called Duty Pilot (left message and text message). XC:00 4. Notified Dispatch via phone. XD:00. 5. Notified maintenance desk via phone. XE:00 . AIRCRAFT STILL DEPARTED AT XF:00. Again, this is an obvious disregard of safety. Why do all groups involved allow maintenance department to continue these unsafe operations? I have highlighted multiple times on other reports what I believe should be a cultural change from maintenance department so solve these obvious safety concerns.

Synopsis

Air carrier pilot reported multiple aircraft maintenance discrepancies that were not properly addressed by the maintenance department.

ACN: 2017316 (385 of 2908)

Time / Day

Date : 202307

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class C : ZZZ

Component

Aircraft Component : Cabin Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Instructor
ASRS Report Number.Accession Number : 2017316
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After departure the lead FA (Flight Attendant) called up and said they had a very strong electrical smoke smell and the walls of the center aft lav was extremely hot to the touch. I asked if there was any smoke or fire. They said there wasn't but the smell was very strong and not dissipating at all. At this point I made the decision to request priority handling and divert to ZZZ. We turned around and started down and ran all of our checklist including the overweight landing checklist. The I reached out again to the FAs and said I needed to know if any smoke started appearing at all. They said they would. They were given a priority code and we prepped for landing. Unfortunately with all that was going on I failed to notice they turned us in about 5 miles closer then I expected. Due to this we ended up being much to high for the landing. Since we didn't have an actual fire at this point I felt it was safer to go around instead of forcing a bad landing. We went around and then came back for a smooth overweight landing. After landing we stopped on the high speed and I had the fire trucks position behind us and look for any signs. I had the aft FA go back into the lav to check again. The strong electrical smell and the heat continued with no reduction in intensity. I told the Airport Rescue and Firefighting to stay close and follow us in as I felt it was safer at this point to deplane normally. I told them if we got any indication of smoke we would immediately stop and evacuate. We got to the gate and again I felt it safer to remove all passengers thru the Jetway and then have the firemen inspect the lav. Upon inspection they found the water heater to be at fault and basically running away as hot as it would go. It was so hot that the hot water coming out was 130 degrees and even the cold water was 108 degrees. For reference the other lav showed 80 hot water. The end result was a safe landing with no evacuation needed. I did make the mistake of not realizing we were as close as we were so ultimately the only thing I would have done different would have been to verify how far out we were being vectored. In the future I will attempt to look at that more closely. However a very strong mitigating factor was the possibility of this turning into a fire. Ultimately it did not luckily.

Synopsis

Air carrier Captain reported cabin odor and walls in the aft lavatory were extremely hot, as reported by the aft Flight Attendant. After a divert, a missed approach due to arriving too high on final and then an overweight landing, the aircraft arrived safely. The Fire Marshal and maintenance determined the aft water heater was running uncontrolled full power and overheating.

ACN: 2017206 (386 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 0001-0600

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 80.2
Experience.Flight Crew.Type : 255.27
ASRS Report Number.Accession Number : 2017206

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While I was briefing the flight attendants prior to board I felt a tug and pull, vibrations, from the nose. Ground ramp crew pulled the aircraft and disconnected tug abruptly without notifying crew. Maintenance was called to inspect the nose. Write-up was placed in book. Maintenance checked the nose gear door and movement. No damage noted.

Synopsis

A320 Captain reported Ground Personnel disconnect the tug from aircraft without coordination with crew resulting in aircraft movement and vibration.

ACN: 2016918 (387 of 2908)

Time / Day

Date : 202307
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Hydraulic Fluid
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 5770
Experience.Flight Crew.Last 90 Days : 117
Experience.Flight Crew.Type : 5229
ASRS Report Number.Accession Number : 2016918

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

First the aircraft number on this report is incorrect, it was XXX. On takeoff roll with TOGA power we got an ECAM and performed a low speed abort. We exited the runway. We both recalled it was a hydraulic ECAM but werenâ€™t sure exactly what it said. It would not recall so we pulled up the hydraulic page and saw that we had lost yellow system quantity. We called ops and maintenance and taxied back to the gate without incident.

Synopsis

A320 pilot reported a low speed rejected takeoff after the loss of the yellow system hydraulic quantity.

ACN: 2014851 (388 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2014851
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During preflight, we received via ACARS an Expect NOTOC (Notice to Crew) message for dangerous goods. We never did receive the NOTOC. We queried the tug driver, and he confirmed that no DG (Dangerous Goods) was loaded on the aircraft. We then asked the tug driver to confirm that, in accordance with FOM, the Crew Chief cancelled the NOTOC. We received verbal confirmation that no DG was loaded on the aircraft, but we did not receive confirmation that the NOTOC had been cancelled in accordance with the FOM, nor did we receive the NOTOC cancellation message via ACARS in accordance with FOM. I contacted station operations, but they were unable to assist in producing the NOTOC cancellation message. I contacted my dispatcher. My dispatcher was not helpful. In fact, my dispatcher informed me that in all of his years as a dispatcher, he had never had a Captain insist on receiving a NOTOC cancellation message, as required by my FOM. Central Load Planning then sent a message via ACARS informing us that no Dangerous Goods were loaded on the flight, but the message did not contain the verbiage found in FOM "cancellation." Ultimately, I made contact with the ZZZ Director of Operations. The ZZZ Director of Operations authorized me to depart, and he sent both email and text message confirmation of his authorization. He also had a message sent to us via ACARS, though the text of that message did not comport with the text found in FOM. In all, I refused to depart until receiving the NOTOC Cancellation message required by the FOM. We incurred a delay of approximately 1:40 since nobody was able to send us via ACARS the required NOTOC Cancellation message. Other departments, including Ground Support, Station Operations, Dispatch, and Load Planning are either not aware of my FOM requirement to receive a NOTOC Cancellation message or they do not know how to cancel a NOTOC and generate the NOTOC Cancellation message. Other departments should be trained on the requirement to cancel a NOTOC and the process for generating the required NOTOC Cancellation message.

Synopsis

A320 Captain reported "no NOTOC" cancellation message received prior to pushback. After an extensive delay a message stating "no DG cargo on board" was provided to the crew, enabling the flight to depart.

ACN: 2013584 (389 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Airspace.Class B : LAX

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2013584
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were on the IRNMN2 arrival for LAX prior to BAYST. We encountered moderate turbulence, then the aircraft banked 45 degrees to the left. The autopilot stayed engaged. I was preparing to disconnect the autopilot, but it began to roll us level, so I kept it engaged. We reported the wake turbulence to ATC. We checked with Inflight to ensure the safety of our crew and customers. We landed safely. I informed Dispatch and consulted with the Duty Chief about the encounter. A logbook write-up was made describing the encounter.

Synopsis

An air carrier Captain reported encountering wake turbulence on approach into LAX.

ACN: 2012936 (390 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 6300

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Crew Size.Number Of Crew : 1
Flight Plan : None
Flight Phase : Climb

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2012936
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Automation : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On our flight from ZZZ to ZZZ1, we were descending on the ZZZZZ arrival and had just crossed ZZZZZ1 at 7000 ft. We continued to descend through 6900 ft. when we received a traffic call from ATC. The Controller indicated that the traffic was at 10 oâ€™clock at around 5500 ft. and climbing. They told us to level off immediately. They spoke quickly so I was not sure what they said. I asked for confirmation that they wanted us to level off. They confirmed and we then received a TA. The First Officer (FO) Pilot flying (PF) leveled off and announced "TCAS I have control". We immediately received an RA with the RA order "Monitor Vertical Speed". We followed the procedure for an RA, both announcing "Autopilot off, flight directors off." The FO disengaged the autopilot and I turned off the flight directors. My Navigation Display (ND) displayed a red traffic symbol at -3 at almost the same location as our Own Ship symbol. I looked out my window and saw the traffic, a high wing prop aircraft, passing below us at about 300 ft. below our altitude. At this point we were at approximately 6300 ft. After a few seconds we received a "Clear of Traffic" call from the TCAS system. I told the Controller that we had received an RA. They told us that the traffic was not a factor and cleared us directly to ZZZZZ2 to cross at 4100 ft. and were cleared for the ILS XX approach. The rest of the flight proceeded normally.

Synopsis

A320 Captain reported an NMAC event during initial approach. The flight crew complied with the TCAS RA until clear of conflict and proceeded normally.

ACN: 2011850 (391 of 2908)

Time / Day

Date : 202306

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Initial Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2011850
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Previous crew reported ENG2 N2 high vibration ECAM. Mechanics checked aircraft and noted no discrepancy and added to tracking. On climbout, we received ENG2 N2 high vibration ECAM and applied the HI ENG VIB procedure. Maintenance Control was advised and we continued after the ECAM went away. Just prior to descent into ZZZZ, we received ENG2 OIL CHIP DETECTED ECAM. On arrival, I called Maintenance Control, advised of both, put the current ECAM in the logbook, and was told the high vibration was already tracked and being taken care of by Maintenance. I spoke to the mechanic in ZZZZ, he confirmed this, and entered in the logbook per Maintenance Control. The next flight leg, the #2 engine failed.

Synopsis

Air Carrier A320 pilot reported an engine failed in flight and was preceded by the flight crew receiving high engine vibration and oil chip detector messages on prior legs.

ACN: 2011730 (392 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class C : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 118
Experience.Flight Crew.Type : 528
ASRS Report Number.Accession Number : 2011730
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

With the Captain flying and I performing PM duties, Aircraft Y departed Runway XXR out of ZZZ. Shortly after take-off, the Captain said "Gear Up" and I went to retract the landing gear. Upon moving the gear handle, we received an ECAM of HYD G Eng 1 Pump Lo Pr and the landing gear and doors did not retract. The Captain put on the autopilot and we cleaned up the flaps and maintained an airspeed of 205 KIAS. We started to run the ECAM which had us turn off the green pump. Then I asked ATC to level off at 4000 ft. MSL and a heading while we ran a checklist. I turned the green pump back on to attempt to retract the landing gear, but after the system indicated no pressure, I turned the pump back off and left the gear down. We also checked the hydraulic page and noted that we had lost all fluid in the green system. At that time the Captain handed me the aircraft and radios while they worked in coordination with Dispatch and Maintenance Control. After conferring with Dispatch, I [requested priority handling] with ATC and notified them of our intention to return to ZZZ while the Captain continued to troubleshoot with Maintenance Control. I ran the HYD G Eng 1 Pump Lo Pr FM procedure, ran the landing distance tables, and built the box to an ILS to Runway YY, which was ZZZâ€™s longest runway that supported the winds (250 @ 6 gusting 21). ATC began to give us vectors while I maintained an airspeed of 205 KIAS. Shortly thereafter the Captain got off the phone, informed me the flight controls would be slow to move, we had lost nose wheel steering, and thrust reversers on the left side, and would have to stop straight ahead on the runway. They talked to the passengers and Flight Attendants (FAs) to inform them of the situation, then asked me to run the overweight landing checklist, which I did. I also conferred with them on the L/G gravity extension checklist, which we did per the HYD G Eng 1 Lo Pr checklist. After that they reviewed the mission computer work I had done, looked at the approach, and briefed the procedures. I handed the aircraft back to them and I informed ATC we were ready for the approach and they set us up for a 25 mile straight in for the ILS to Runway YY. I ran the descent checklist, we configured the aircraft early due to slow flight control movement, and I ran the landing checklist. The Captain did a great, smooth landing on the runway and they brought us to a stop with the reversers in reverse idle. The Captain applied the brakes at 105 kts. and brought us to a stop abeam taxiway X. I asked Tower to have Crash Fire Rescue (CFR) check on the brakes to ensure they didnâ€™t overheat. I ran my normal after landing flow, we started the APU, the Captain talked to the passengers, and then we coordinated a tug with operations. We shut down the engines with the parking brake set and the Captain called the Chief Pilot to advise them of our situation and plan. The tug came and got us about 20 min later and pulled us back to the gate with no further issues. Upon arriving at the gate, the gate agent spoke to the passengers and deplaned the aircraft while we talked with maintenance.

Synopsis

A320 First Officer reported that the landing gear and doors did not retract after takeoff and the aircraft lost all fluid in the green hydraulic system. The flight crew left the gear extended and returned to the departure airport where after landing they were towed to the gate.

ACN: 2011542 (393 of 2908)

Time / Day

Date : 202306

Place

Altitude.MSL.Single Value : 9000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Climb

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2011542

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Pickup aircraft from heavy maintenance for a reposition flight. After take-off CAB PR SYS 1 + 2 fault ECAM. Manual control also would not close the outflow valve. We got a master warning CAB PR EXESS CAB ALT ECAM for about 5 to 10 seconds. The highest we went on the plane was 9000 ft. but cabin altitude at one point showed around 10500 ft. We informed ATC of an issue with our pressurization system and requested altitudes below 10000. We also saw ECAM CAB PR LO DIFF PR. Manual control would not work to close the outflow valve but it did work to open it during the approach (as per ECAM status and Flight Crew Operations Manual instructions). We performed ECAM actions, referenced QRH and Operations Manual, and communicated with ATC and Dispatch. Please follow up with what the problem is that would cause a loss of three redundant systems.

Synopsis

A320 Captain reported a cabin pressurization problem during take-off while on a repositioning flight. The Captain stated they flew to their destination below 10000 ft. with ATC assistance.

ACN: 2010124 (394 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2010124

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2010129

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X, myself and First Officer (FO) were operating Aircraft X ZZZ-ZZZZ on Day 0, our second flight that day on Aircraft X. It was my leg and I was pilot flying. We pushed back from gate in ZZZ at XA57, started both engines, complied with the procedures for MEL XX-XX-XX-X High Pressure Valve (HPV) (an open MEL on that aircraft), performed our checklist and taxied out to Runway XXL as instructed. At approximately XB08 we were first cleared to line up and wait, which we did and up to that point were without incident and/or any abnormalities. About 1 minute later we were then cleared for takeoff. We performed our normal task along with the required operational procedure in that MEL's "BEFORE TAKE OFF UNTIL DECENT" section just before applying take off thrust. I announced "takeoff" and began to push the thrust levers forward to the initial 50% N1 target. It was while doing so that I sensed that there was some sort of operational abnormality with one of the engines. I heard the sound of a muffled "thud" to start with followed by the sound of an engine decreasing in rotation verses increasing as it should be. Simultaneously, and as I was verifying this on the engine indications, we were presented with an aural warning and the ENG 1 FAIL ECAM notification and associated actions. At that point we were roughly at a speed of just around 10 kts. I immediately rejected the takeoff, brought the aircraft to a stop, put the parking brake on and called for "FLIGHT ATTENDANT STATIONS" over the PA. FO notified the Tower that we were aborting the takeoff. I then called for "ECAM actions" and we proceeded through those. After preforming and clearing the ENG FAIL ECAM actions there was also an ENG 1 STALL ECAM alert and actions to follow. We performed and cleared those as well. Once all ECAM action were completed I made the triple "REMAIN SEATED" call over the PA and then called to speak directly with the lead Flight Attendant (FA). I advised him that we had encountered an engine failure on the number 1 engine during the takeoff roll, that no other threats were perceived from our position, and that we would be taxiing back to the gate. He was in agreement and made it known that all was clear in the cabin to do so. After that call I requested FO to send an ACARS message to our Dispatcher advising him that we had encountered an engine failure and that we needed to return to the gate. As he was doing that I called Tower ATC to advise them of the same and requested taxi. ATC cleared us to taxi down XXL, to exit left on taxiway 1, to hold short of taxiway 2 and that he would assist us in getting an open gate to return to. After clearing the runway I made a PA announcement to the passengers that we were returning to the gate due to a mechanical issue. We were then cleared into the ramp by ATC. During the taxi we performed the After Landing flows, checklist, and received an ACARS response from our Dispatcher acknowledging our gate return. FO also called station operations over the radio informing them of our return and for gate coordination. We started the APU, parked the aircraft, shut down down ENG 2, and preformed the Parking Checklist. Once parked at the gate and after opening the cabin and cockpit door we were met by maintenance personnel. I briefly spoke with them, our lead FA, and the gate agent to ensure all was stable before calling our Dispatcher to discuss the incident, which I then proceeded to do. At the end of that conversation I asked to be transferred to Maintenance Control where I gave them the run down as well and discussed the logbook entries. I then called the Chief Pilot and explained the situation, and got the authorization to deplane the passengers. I coordinated that with station personnel and then asked Lead FA to make the deplaning announcement. At some point I was contacted by the Dispatch Coordinator on duty to get an understanding of the situation and to determine our fitness for duty. I told him I would discuss that with the rest of the crew and call him back with a decision. FO and I debriefed the situation amongst ourselves and I followed that with a debriefing with the flight attendants. It was unanimously determined that in the interest of safety it would not be the best course of action to continue on duty and to preform any possible subsequent flight(s). I reported that back to the Dispatch Coordinator, crew scheduling and Chief Pilot and we were all cleared to duty off.

Narrative: 2

Captain and I were working Aircraft X from ZZZ-ZZZZ. We pushed off of gate in ZZZ and planned for a two-engine taxi out to XXL/Taxiway 1 so we started both engines and completed the associated "MEL XX-XX-XX-X High Pressure Valve (HPV)" procedure after engine start that Aircraft X had. We continued our taxi out to Runway XXL/Taxiway 1 as briefed and were number 1 for takeoff. Upon reaching the runway we had elapsed over 6 minutes on engine warmup and received a line up and wait clearance. I verified the runway heading and shortly after we were cleared for takeoff. Captain completed the MEL items and called for takeoff. Upon bringing the thrust up to roughly 50% N1, Captain began to notice an issue with the number 1 engine and began verbalizing it. As he was doing so, we received an ENG 1 Fail ECAM followed by an ENG 1 Stall ECAM in which he retarted the thrust levers to idle and called for a rejected takeoff. We were not even at 10 kts. when this happened. Captain stopped the aircraft and called "flight attendant's stations" while I notified ZZZ Tower we were rejecting on Runway XXL. Once back together he called for ECAM actions in which we worked through the ENG 1 Fail ECAM procedure determining no damage was suspected. ZZZ Tower asked if we needed any assistance to which I replied "standby" as we were working through the ECAM. We followed up with the ENG 1 Stall ECAM and Status page, after which Captain made the remain seated announcement then talked with the flight attendants. Captain then told ATC we had an Engine 1 failure and that we would be returning to the gate. We received taxi instructions to continue down Runway XXL, and taxi to the ramp. I coordinated a new gate with operations and sent a message to Dispatch informing them of our return to gate status. On the way back to gate we ran an after-landing checklist and continued into parking where Maintenance took over the aircraft.

Synopsis

A320 crew reported a muffled thud and indications of a decelerating engine followed by ENG 1 FAIL and ENG 1 STALL ECAM's as power was applied for takeoff. Crew rejected the takeoff and returned to the gate after accomplishing ECAM items.

ACN: 2008302 (395 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2008302
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While en-route from ZZZ Airport to ZZZ1 Airport north and east of ZZZ VOR at 34, 000 ft. an ECAM presented. Hyd y low pressure. The reservoir was empty according to the systems page. We applied the procedure and referenced the COM. The status page indicated that nose wheel steering would be unavailable. I then called Maintenance and Dispatch via ARINC. After conferring with and using all available information I requested priority handling. On approach the status page procedure had me select yellow hyd pump on. Although it was indicating 0 pressure in-flight once re-selected it produced 2950 PSI. We landed the aircraft on Runway XXR and were able to exit the runway and taxi to the gate. At this time the ECAM and status procedures were not presented. Emergency vehicles followed us to the gate where we parked and secured the aircraft. During post-flight a large puddle of what appeared to be hydraulic fluid was dripping and pooling under the #2 engine. Maintenance personnel boarded and started troubleshooting. I called the Chief Pilot on duty to inform the company of the actions that I had taken.

Synopsis

A320 pilot reported a hydraulic system failure during cruise. After consulting with maintenance and dispatch the flight continued to the destination where the post flight inspection confirmed a hydraulic fluid leak.

ACN: 2008232 (396 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : APU
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Nacelle Fire/Overheat Warning
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 2008232
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Evacuated
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Upon arrival into ZZZ at Gate XX with the APU on MEL, we shut down engine 2 and then waited for the ground power to be connected. After ground power was connected and engine 1 shutdown, we were doing our shutdown checklist when a ramp worker knocked on the cockpit door and informed us that we had a tailpipe fire in engine 1. We immediately performed Engine Tailpipe Fire QRC. We had no indications in the cockpit that there was a tailpipe fire. Since the APU was on MEL we were not able to dry crank the engine. Without knowing if the fire was still there or not I made the decision to call ARFF (Airport Rescue and Firefighting). When they arrived, they shot water up the tailpipe. The passengers were halfway through disembarking and felt that an evacuation was not necessary as everyone was getting off in a timely matter. I looked out at the engine and did not see any flames before or after the ARFF had put water on the engine. Having the APU on MEL, it prevented us from dry cranking the engine which most likely would have prevented us from calling ARFF and having them to put water on the engine. The APU was on MEL for 7 days and if this was fixed it would have us able to complete the Engine Fire Tailpipe QRC.

Synopsis

A320 pilot reported after shutting the engines down at the gate, a ramp worker informed the flight crew that engine 1 had a tailpipe fire. Airport responders put the fire out and passengers deplaned normally.

ACN: 2008133 (397 of 2908)

Time / Day

Date : 202306

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2008133

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure from ZZZ we accomplished a rejected takeoff and returned to the gate for maintenance. After Maintenance cleared the aircraft we departed ZZZ approximately 2 hours late. On the departure we experienced a high engine vibration event and subsequently [requested priority handling] and returned to ZZZ with one engine at idle thrust. Then, we taxied to the gate and deplaned the passengers.

Synopsis

A320 First Officer reported performing a rejected takeoff and returning to the gate for maintenance. After being cleared by Maintenance and taking off a second time, the flight crew experienced a high engine vibration. The flight crew then returned to the airport with one engine on idle thrust.

ACN: 2007800 (398 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2007800
Human Factors : Troubleshooting
Human Factors : Physiological - Other
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Before take off FA#2 called me about a odor in the back cabin and galley, so I went back to confirm what he smelled and I smelled the odor too as I approached row 21 and FA#4 did as well. After take off FA#2 told me that the odor was still there so I went back there again to confirm what they could smell. I told him to go ahead and call the captain and let him know. I went back up to the front galley. FA#2 came to front galley and told me that he let the captain know and that he was feeling light headed and a little dizzy so I told him he may need to go on oxygen and call the captain back and let him know what he was experiencing. The captain then asked if he needed to call the POC (Physician on Call). The POC was contacted and he was told to get on oxygen. Captain called and asked about smell, so I did a cabin check and called him back and let him know the sweet odor was still present. Captain told me that he was going to try to flush the cabin air and see if that helps. Afterwards he called and I did a cabin check and sweet odor was still back there. After the cabin service that FA#4 and I did her symptoms heightened and she went on oxygen as well. I updated the captain on the odor and both fa's and told him passengers were asking concerning questions. He let us know that we would be diverting and that the oxygen mask would be deployed at 14,000 ft when we started descending and he made a PA to the passengers. FA# 2 and FA #4 said they had a combination of symptoms of feeling foggy, lightheaded, dizzy, sick to stomach, eyes burning, metallic taste, dry mouth, and a little slow articulating. Captain made an announcement that oxygen mask were being deployed when we reached 14,000 ft and FA# 2 made an announcement instructing on how to use them and I went through the cabin to check on the passengers and to make sure they were ok and knew how to properly use them if they needed oxygen. After landing we were met by two managers and the police, emt, fire dept. A announcement was made to the passengers that if they needed any medical attention and want to be checked out it was available for them. The emt's checked our vitals. I was told mine checked out fine. I only had a metallic taste in my mouth which I have had [previously]. We were then bused to terminal. Once we deplaned 2 supervisors met the flight and was given an brochure and a letter to attend a mandatory meeting tomorrow that I have been withheld from service for a investigation into a potential performance matter. This event was handled really well by the crew. We all communicated well and worked as a team. We all took care of each other and our passengers. We all put safety first. I was also contacted by several inflight managers to see how we were doing and received instruction and support about what we needed to do.

Synopsis

Air carrier Flight Attendant reported a persistent dirty socks odor in the passenger cabin during cruise which was reported to the Captain. After troubleshooting the flight diverted to a suitable airport where airport personnel met and provided medical support.

ACN: 2007436 (399 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2007436
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 2007781

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I operated flight ZZZ [to] ZZZ1 with an electric wheelchair in the cargo bin that wasn't documented on the aircraft load sheet. I saw the wheelchair get loaded on the aircraft and didn't think to look at the aircraft load sheet to ensure that it was documented until after we departed ZZZ.

Narrative: 2

Operated listed flight with electric wheelchair that did not seem to be included on aircraft load sheet. Wheelchair was loaded last minute and did not double check to ensure it was included in the load sheet until after we had departed.

Synopsis

A320 Captain reported a wheelchair was loaded in the cargo compartment, but failed to verify that the wheelchair was included in the aircraft loading document prior to gate departure.

ACN: 2007434 (400 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 5100

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors
Airspace.Class E : ZZZ

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Compressor
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 2007434

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
ASRS Report Number.Accession Number : 2007437

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Captain was pilot flying, and as we were climbing above approximately 5100 ft. indicated altitude and in a left turn to the North, a loud noise was heard in combination with a jolt and yaw to the left. An ENG 1 stall ECAM warning subsequently appeared but quickly disappeared. The next ECAM to appear was the ENG 1 FAIL warning. During this period I observed a vapor briefly enter the cockpit via the vents above the windows - it quickly dissipated and was not observed again. Captain called for ECAM actions after establishing positive control, and I ran through the ECAM procedure. There were no applicable OEBs (Operational Engineering Bulletins). We determined that damage had occurred, and that fuel was not leaking. We reviewed the affected systems pages, and after ECAM actions were complete we verified that all checklists were completed and that there were no resets or relights possible prior to reviewing the status page. We also determined that we would be overweight for landing. We considered reviewing the comm procedures but determined that that activity was not a priority. I resumed comms at this point briefly before taking control while the Captain set up for, and briefed, the approach. They then took back control and comms so that I could run the landing distance assessment. After I completed that, I resumed pilot monitoring duties for the remainder of the flight. Captain executed the RNAV XXL approach to land smoothly and uneventfully; the landing was very soft. We stopped on the runway and waited for Airport Rescue and Firefighting (ARFF) to approach the airplane and conduct a visual inspection. ARFF communicated with us directly on Tower frequency and told us there were no indications of fire or leaking fluids. We taxied to the ramp and gate with ARFF on the trail as a precaution.

Narrative: 2

The flight originated in ZZZ with destination to ZZZ1 and additionally was a training event OE new student. We experienced a ground delay due to smoke in ZZZ1 and ATC informed us of an ETD of XA:41. Maintenance was in the process of removing MELs 22-XX-XX-X and 24-XX-XX-XX. We waited for the log book to arrive. Once the log book arrived we started our push and taxi single engine to Runway XXR. Once we confirmed with ATC there were no more delays to ZZZ1 we started the number 2 engine. I was the pilot flying. We take off right at wheel up time and in our climb to our assigned altitude 7000 ft. While climbing through 5100 ft. we heard a very loud boom on the left side of the aircraft and a yaw to the left. ECAM warning was ENG 1 stall followed by ENG fail. I ask for ECAM actions. We communicated the issue to ATC. We established positive control. The First Officer ran the associated ECAM procedures. We confirmed the engine was damaged, reviewed status pages, OEBs (Operational Engineering Bulletins), mention the overweight landing and comm procedures. Set up for the approach Runway XYR but ATC offered us Runway RNAV XYL. I set up for that approach transfer controls to the First Officer and briefed the approach. I talked to our Flight Attendants and made an announcement to the passengers. I took control of the aircraft, got vectors and cleared the approach. We verified the Airport Rescue and Firefighting (ARFF) and equipment was standing by on the runway. Once we landed I stopped the aircraft and called Flight Attendants to stations and verify with ARFF that the aircraft didn't have any indication of smoke or fluids of any type. I made the PA remain seated announcement three times and we were clear to taxi off the runway. We taxied to gate and no injuries or damages were experienced besides the engine failure. I added two log book entries, one for engine fail and one for overweight landing. Considering the First Officer was only their fourth trip on OE their performance was outstanding. The cabin crew was also outstanding at communicating and maintaining passengers safe and calm.

Synopsis

A320 flight crew reported an engine failure and a return to the departure airport