ACN: 2006695 (401 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Nose Gear
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 2551
Experience.Flight Crew.Last 90 Days : 212
Experience.Flight Crew.Type : 2551
ASRS Report Number.Accession Number : 2006695
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 279
Experience.Flight Crew.Last 90 Days : 146
Experience.Flight Crew.Type : 279
ASRS Report Number.Accession Number : 2006737
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We were holding short of Runway X and given take-off clearance with a turn to a left heading of 320 degrees. The throttles felt like less tension and I remarked that as I brought the power up to stabilize at 50% N1, then advanced the throttles to take-off thrust and felt like there was a pull to the right side and corrected with left rudder application and felt as if the nose was skipping and not straight, so elected to reject and notified Tower followed by QRC for RTO (Rejected Take-off). Cleared Runway and contacted hub maintenance and Maintenance Control to discuss and wrote up the aircraft and returned to Gate X in ZZZ. Discussion with local maintenance as to what we all experienced on take-off roll.

Narrative: 2

We were cleared for takeoff on Runway X at ZZZ. The Captain advanced the thrust and called check thrust. As the engines spooled up, the aircraft began to yaw to the right. I noticed the Captain reject the take-off and I told Tower we were stopping on the runway, notified the passengers, and preformed the Rejected Take-off Checklist.

Synopsis

A320 flight crew reported nose wheel steering problems on take off resulted in a rejected the take off and return to the gate for maintenance action.

ACN: 2006539 (402 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2006539
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

First leg, Pilot Flying [was] on new hire OE approach was well flown, but didn't flare soon enough on landing. Hard landing with report generated.

Synopsis

A320 Captain reported a new hire pilot failed to flare soon enough resulting in a hard landing.

ACN: 2006007 (403 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13050
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 4106
ASRS Report Number.Accession Number : 2006007
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 395
Experience.Flight Crew.Last 90 Days : 158
Experience.Flight Crew.Type : 395
ASRS Report Number.Accession Number : 2006020
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Landing conducted on XXL in ZZZ. On landing, ECAM ENG 1 THR LEVER DISAGREE, only engine #2 went into reverse thrust with slight pull to the right. Once slowed, we exited the runway on taxiway 1, when ECAM HYD G RSVR LO LVL illuminated. Stopped aircraft with alternate brakes at intersection of taxiway 1 and taxiway 2. Note, during transfer of braking, aircraft braking was sluggish and brake pedals had limited response. When the brakes finally engaged, anti skid protection activated. Since there was no steering, aircraft was directed towards median between taxiways. Aircraft stopped on the midline of the taxiway heading. ZZZ Ground indicated we had a large pool of fluid forming under the fuselage of the aircraft. ECAMs were completed and APU started. Flight Attendant briefings were completed. PA announcement made to the passengers. ZZZ Crash Fire Rescue (CFR) responded to ensure the safety of aircraft. Hazardous material spill cleanup responded to aircraft. Because of loss steering and normal brakes, aircraft was towed to gate. Passengers disembarked via jet way. ZZZ airport safety, CFR and HAZMAT debriefed crew. After initial briefing to Purser who relayed info to aft Flight Attendants, aft Flight Attendants indicated they smelled hydraulic fluid. I had to directly call them to discern what they really meant, understand current cabin status. Confusion could have led to unnecessary evacuation. Location of aircraft near ZZZ XXL and high speed taxiway during continued airport operations created difficulty for CFR, and HAZMAT response crews.

Narrative: 2

We were cleared to land on Runway XXL in ZZZ. Approach and touchdown were normal. Upon rollout we had a left reverser unlocked ECAM message. We continued to decelerate and exit the runway at taxiway. While exiting the aircraft braked hard and the Captain (CA) said I do not have any more brakes or steering and they set the parking brake. I informed Ground Control that we had a steering issue and would need to remain in place while we figured out what was wrong. Ground told us to proceed as requested. We ran the ECAM which led us to shut off the hydraulic pump and the left engine. Other aircraft told us we had fluid dripping from the underside of the center's fuselage and a large puddle under the aircraft. We then coordinated with Company operations and Ground Control to get a tug to tow us into gate. Airport operations and the Crash Fire Rescue (CFR) escorted us to the gate. The tow in was uneventful. After passengers deplaned Company Maintenance came aboard. We briefed Maintenance and then took over the airplane. We continued on the remainder of our pairing without further incident.

Synopsis

A320 flight crew reported ECAM HYD G RSVR LO LVL illuminated after landing with ECAM ENG 1 THR LEVER DISAGREE and no reverser deployed on the number one engine. The flight crew moved off the active runway and requested a tow to the gate.

ACN: 2005591 (404 of 2908)

Time / Day

Date : 202306
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : EWR.Airport
State Reference : NY

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : EWR

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : B777 Undifferentiated or Other Model
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : EWR

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2005591
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Newark Approach Control gave us vectors behind a 777. Separation was less than 6 miles. I executed a go around after encountering wake turbulence at 100 feet. We were on ILS 04R. Winds were 200 degrees 5 knots. After the go-around, we were asked too many times the reason for the go-around. It added unnecessary communication after we explained the reason during the initial contact after going-around. I needed to instruct the Controller to inquire on the ground.

Synopsis

A320 Captain reported executing a go-around at EWR after encountering wake turbulence on approach 6 miles in trail of a B777.

ACN: 2003550 (405 of 2908)

Time / Day

Date : 202305
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2003550
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2004489
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As we were climbing through 33,000 ft., the First Officer (FO) noticed that the cabin altitude was increasing at a rapid rate and it was starting to get excessively high. Immediately after noticing this, we got a Master Warning followed by the Cabin Pressure Excessive Cabin Altitude ECAM. We immediately did our memory action items for [Priority] Descent and ran the QRC and started an [Priority] Descent. We notified ATC, and squawked 7700. The flight attendants were notified via the [priority] call button to immediately get on oxygen due to our cabin depressurization, and given the briefing. Once we were stabilized in the descent, I finished running the ECAM, and the QRH. We did an air return back to ZZZ and ran the overweight landing checklist in the QRH. The passenger oxygen masks deployed in the cabin. The flight attendants were smelling the smoke from the oxygen generators, but it sounded like it was more centrally located in the rear of the cabin toward the tail. We decided for precautionary measures to get CFR to inspect the tail section for any potential hot spots. All clear was given, and we then returned back to the gate with no further incidences. This discrepancy was entered into the aircraft logbook. We had no known passenger injuries. Three flight attendants, #2, #3, and #4, were complaining about being dizzy afterwards.

Narrative: 2

I was the pilot monitoring. As we were climbing through FL 330, I noticed that all indications were "green," but the cabin altitude was excessively high--9,200 ft.--and was rapidly increasing. Immediately after I asked the Captain about this reading, there was a Master Warning followed by the "Cabin Pressure: Excessive Cabin Altitude" ECAM message. We immediately did our memory action items for [Priority] Descent. The Captain started an [Priority] Descent and told me to notify ATC that we need an [Priority] Descent to 10,000 MSL, which I did. He changed the transponder code to 7700. ATC initially cleared us down to FL 300, and very soon after to 10,000 MSL. The flight attendants were notified to "get on oxygen" due to our cabin depressurization, and they were briefed. Once we were stabilized in the descent, the Captain gave me control of the airplane and then continued running the ECAM actions and the QRH. The passenger oxygen masks were deployed. Once we leveled at 10,000 MSL, we advised ATC that we wanted to return to ZZZ, and were given the clearance "Right turn, direct ZZZ." The Captain advised the passengers it was now safe to remove their masks. He also sent an ACARS message to Dispatch informing them of our pressurization and an air return back to ZZZ, and he ran the Overweight Landing checklist from the QRH. The two flight attendants in the rear of the aircraft informed us that they smelled the odor from the oxygen generators. The Captain told me to go fast in order to expedite our return. We decided on precautionary measures, so we advised ATC that we would be stopping on the runway and would need Crash Fire Rescue (CFR) inspect the aircraft for any potential hot spots. We also requested the longer Runway--XXR. After the Captain loaded the approach, he took control of the airplane. I made a brief announcement to the passengers that we would be stopping on the runway for a precautionary inspection. After landing and stopping on the runway, CFR inspected the airplane. "All clear" was given to us, and we then taxied to the gate. After the cabin door was opened, the CFR leader came onto the flight deck to check if there were any injuries or medical issues. We had no known passenger injuries. Three flight attendants, #2, #3, and #4, were complaining about being dizzy afterwards. The original ECAM message and Overweight Landing were entered into the aircraft logbook.

Synopsis

A320 flight crew reported a loss of pressurization system during climb. Flight crew executed QRH procedures and requested priority handling to return to departure airport where a safe landing completed.

ACN: 2002959 (406 of 2908)

Time / Day

Date : 202305

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2002959
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X Gate XX in ZZZ. The aircraft doors were closed and the jet bridge had been pulled away. We are on a 13 min delay due to a passenger removal and now we were ready to pushback. ATC cleared our push and I informed the ground crew. We initiated the push when in just a few seconds later I hear "brake" through the headset. It seemed like it was coming from Ramp Control but their frequency became congested. I looked around outside and noticed the gate agents standing at the edge of the jet bridge making gestures to stop. We also saw a ground crew making signals to set the parking brake on. I tried to call the ground personnel but there were no answer. I looked at the Engine and Warning Display (E/WD) and it still showed the NW STRG DISC memo. Then I looked down and saw that the tug was stopped. It was not moving and then as the airplane kept slowly moving backwards I was able to see the front of the tug and noticed that the tow bar was not attached to it. Immediately I realized that the tow bar had been disconnected from the tug for some reason and that the aircraft was moving backwards without any control from the ground crew. We were just a few feet away from our originally parking position but knew I couldn't delay on my reaction because not so far behind the XX gate was a wall barrier and our aircraft was an A321. I also couldn't stop violently because the flight attendants were up doing their presentation so I immediately started stepping easily on the brake pedals until the aircraft got to a complete stopped. I would estimate the aircraft probably rolled back less than 10 ft. and we still had a good clearance from the back wall. I kept both of my feet on the brakes and waited for the ground crew to reconnect their headset. As soon as he reconnected the headset he confirmed that the bar had been disconnected from the tug. I asked if anyone got hurt and if there were any sort of damage to the NW STRG connection, the nose wheel itself or in anywhere else. He confirmed no one got injured and there were no damage anywhere whatsoever. I asked if I could set the parking brake and he confirmed yes. Parking brake was set, tow bar was reconnected to the tug, we informed Ramp Control and continued with our pushback with no further events.

Synopsis

Air carrier Captain reported an unannounced tow bar disconnect during pushback resulting in an aircraft breakaway. Captain realized the situation and stopped the aircraft movement enabling the tow crew to reconnect headset complete a safe pushback.

ACN: 2002608 (407 of 2908)

Time / Day

Date : 202305

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Other / Unknown
ASRS Report Number.Accession Number : 2002608
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

A few years ago, [the company] decided to remove the nesting systems out of the A320, this caused a few changes in how the ramp can load the airplane. 1. It changed the location of the pit nets in 4/5/6. 2. With the reduction of size in pit 4 the weight is still at 5348 lbs. No way one can fit that much in pit 4 without damaging the pit floor. 3. With the reduction in pit 6 the new max weight is 882lbs. ZZZ had a flight a few weeks ago where the lead loaded the flight with 650 lbs. of hazmat and pitted in pit six. When they went to surround the Dangerous Goods (DG) with 15 bags the scanner would not allow them to add anymore weight to the pit once it approached 882 lbs. The lead was not able to understand why, since there wasnâ€™t any type of communication from the company on this change. I would also like to suggest that the scanner [and load planning software] should not allow anyone from submitting a load of DG in pit 6 if the weight of the bags will put it over the pit limits. The DG was removed off of the flight and the flight departed. The [job manuals] have not been updated. I was not the lead, but it appears not too many people inside the station understood what happened, I tried to explain it to the Lead and management and they had a hard time understanding what happened.

Synopsis

Air carrier ground personnel reported the DG cargo pit size could not comply with the 15 passenger bags requirement for proper configuration. DG cargo was removed prior to departure.

ACN: 2002420 (408 of 2908)

Time / Day

Date : 202305

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 2002420
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Attendant
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On Flight XXXX ZZZ â€“ ZZZ1 passenger in [row] XX brought on a Hover Board. The gate agent verify with the Manager whether the Hover Board could be stowed in the plane? She said they said "yes". Even though she knew that wasn't correct. Flight Attendant 4 (FA) came to the back and told me they allowed a Hover Board on board and she didn't think it was allowed. So we looked it up and under Carry On Baggage, Manual, it states Hover Boards (do not accept) X- Carry- On or X- Checked. So I went to the front and told [the purser]. Asked where did I find it and questioned weather my IPAD was updated. Then I looked at the Captain (CA) and told him if they don't remove the Hover Board I'm not flying. So CA got up and he asked gate agent to get passenger. I showed [the purser] how to find the Hover Board under carry-on, on her personal iPad. CA & First Officer (FO) show passenger the iPad that Hover Boards were excluded from flying on the aircraft. CA told me to get the Hover Board. The husband brought Hover Board forward. The passenger, stated to me that they had called our customer service before they brought the Hover Board on vacation and they stated it was OK, plus the Hover Board had flown on [another airline] already from ZZZ1 â€“ ZZZ. I told her that the previous crew had missed it and it was in our manual stating that it could not fly and I didn't feel safe with the Hover board on board our aircraft, especially since it was written in our guide lines. I know I should not write my opinion, but [the purser] did not think that Carry-On was part of our manual, because the information was not located in the manual. The second guessing between us was probably the most difficult confronting the passenger. Another suggestion is if we can get an explanation to why it can not be on the plane?, what can happen?, what can we explain to the passengers.

Synopsis

Air carrier Flight Attendant reported confusion regarding the transport of a Hover Board. The crew determined that Hover Boards are not allowed onboard.

ACN: 2001537 (409 of 2908)

Time / Day

Date : 202305
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors

Component : 1

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 2001537
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying (PF) on a flight ZZZZ-ZZZ. Approximately 35-40 minutes after takeoff, and shortly after reaching top of climb at FL380 and still speaking with ZZZZ Control, we were both alerted to the Master Warning and Master Caution alerts going off. This was approximately XA10. We discovered that both the Autopilot and the auto thrust had kicked off, and we both lost flight directors and the First Officerâ€™s (FO's) Navigation Display (ND) was showing in red MAP NOT AVAILABLE. I took over hand flying the airplane and adjusted the thrust levers to match the necessary Engine Pressure Ratios (EPRs) to maintain altitude and speed. Knowing we were in RVSM airspace I directed the First Officer to request a descent to FL280 while we tried to sort out the multiple issues presented. As we leveled off at FL280 it became immediately apparent two things. One, our fuel burn was significantly higher as we were originally filed for FL380 and two, there was a large line of thunderstorms approaching us as we neared ZZZZ1 airspace. I scanned my ND to consider a divert to ZZZZ1, our nearest station not behind us, and saw that there were multiple cells painting in the general area. Returning to ZZZZ was not an option, as we were battered by fairly large cells on the inbound trip on the ZZZ XX STAR. I concluded that the safest option was to climb back into RVSM airspace and maintain heading and altitude by hand while we attempted to troubleshoot the systems and bring the automation back online. It was roughly XA00 local time; the TCAS showed no airplanes in the vicinity and the radios were dead quiet. There was no conflicting traffic to contend with. I briefly discussed with my First Officer whether or not to [request priority handling]. In the end I felt it was probably going to cause more trouble than it solved, as the airplane was under control, we were safely above any weather and we were for all intents and purposes continuing as if nothing had happened. At some point over Area the systems came back online by themselves; we found that we could engage the Autopilot and the auto thrust and both flight directors came back online. We left ZZZZ1 airspace under full automation, although ZZZZ1 Control did query us as we were nearing ZZZZZ about our altitude, as he was showing us some 200 ft. low. Both my First Officer's and my Primary Flight Display (PFD) were reading FL340, and the standby indicator was showing a little high, about FL341. Shortly after entering ZZZ airspace we lost all automation again. We were on the ZZZZZ1 arrival and, as is customary on that arrival, given direct to ZZZZZ2. I told ZZZ that we had lost automation and all we could accept was headings. The phrase "we're basically a big Piper Cub" was uttered, meaning we were simply stick and rudder at the time. The controller gave us a heading of 024 degrees which was an intercept vector for XXR and gave us a descent to 7,000 ft. During the descent the automation once again availed itself, which we activated. By this time we had lost automation multiple times, each time eventually getting it back. We intercepted the glide slope uneventfully for XXR and I manually disconnected the Autopilot and auto thrust, not wanting to be surprised yet once again by a finicky system. The landing was uneventful and after a short wait we taxied up to the gate and parked the airplane. Maintenance was notified and the logbook entry was made.

Synopsis

A320 Captain reported loss of automation including autopilot and auto throttles in cruise. The flight crew continued to destination airport and landed.

ACN: 2000456 (410 of 2908)

Time / Day

Date : 202305
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 72
Experience.Flight Crew.Type : 515
ASRS Report Number.Accession Number : 2000456

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We arrived at the aircraft about 35 minutes before departure due to security and customs. During our preflight we noted current MEL items and an inbound write-up. The aircraft had two concerning MELs, the Fuel Auto Feed Center tank pumps were inoperative and the ACARS Printer was inoperative. We discussed the Center Tank Fuel pumps and were not concerned as we were not going to have center tank fuel. The ACARS Printer was a bigger threat as we would have to transcribe all our ACARS information. We would also have to receive the Maintenance Release in paper form. The inbound write-up was for the #2 Pack Flow Control always in LO. There was a Mechanic on board looking at something in the cabin. I asked him if he was aware of and dealing with the inbound write-up. he indicated he would check, but I never heard back from him. As we were finalizing our preflight, I called Dispatch to see if they could find out about our pack write-up. They said they would ask Maintenance Control and call back. They called back a few minutes later with Maintenance Control on the line. Maintenance Control advised that they would just defer the #2 pack if that was okay with Dispatch. Maintenance Control proceeded to go through an MEL procedure. I assumed he was just telling me about it and did not realize initially that they wanted me to complete the deferral process. I asked them if we have Maintenance here and why can they not do this? He blew me off and said we can just walk through this process. I had him hold for a minute while I discussed the concern of losing one pack with my FO (First Officer). He was okay and I knew we could talk about it more if needed. So I found the MEL card, 21-XX-X for PACK 2, followed Maintenance Control's instructions, and we deferred the pack. I spoke with Dispatch again and discussed the new release and the need to have a paper Maintenance Release sent to the aircraft. I finally had a free moment to bring ZZZ Operations into the loop about the status of the maintenance issues and to inform the FAs (Flight Attendant) of why the delay and what impact the single pack may have on cabin comfort. My FO and I then had a detailed discussion about the implications of all these MEL items and the pack in particular. In my mind, I had always reserved the final right to turn down this plane if we felt uncomfortable. We talked about the weather in ZZZ1 and how the pack inoperative may affect us. We felt that there was no present threat with the pack inoperative and the forecasted low ceilings in ZZZ1. We noted that all our alternates, including closer airports than the planned ZZZ2, were good alternates. We also discussed that we had lots of alternates all down the coast in the event we had any pack related complications. We decided the fuel MEL would not be a labor intensive and the ACARS Printer was more labor intensive, but not a concerning threat. We came to the conclusion that we would go once we got the Maintenance Release. We started up and normal taxi and takeoff out of ZZZ. Normal climb and level off at cruise FL290, for turbulence above. As we passed over ZZZ3 and were continuing to the south, we got an ECAM, HYD Y ENG PUMP PR LO. It was my leg, I said I have the aircraft and the radios and the FO completed the ECAM. We noted the low level in the yellow system and the inoperative PTU (Power Transfer Unit). We assessed our options, continue to ZZZ4 or turn back for ZZZ3, ZZZ1 was not a consideration. ZZZ4 was about one hour and twenty minutes away with comparable weather to ZZZ3. We transferred the controls and comms to FO. I called Dispatch to bring them into the loop and see what their recommendations were. I discussed the issue and they recommended ZZZ4. I told them to hold and discussed it with my FO. We discussed the potential for additional fluid loss. We could tell we had lost fluid, but did not know from where. As silly as it may sound, I was not in the mood to lose a second HYD system to a mystery leak. Considering time in route to ZZZ4 and proximity to ZZZ3, we chose to go back into ZZZ3. I informed Dispatch that we considered ZZZ4, but were being more cautious and chose ZZZ3. I informed the flight attendants and then informed the passengers. We then [requested priority handling] with ZZZ Center and proceeded back to ZZZ3. Checklists and landing performance, V-speeds and conditions considered, we chose vectors for a straight-in ILS to a visual approach to Runway XXR. We discussed the approach considering the flaps and the after-landing contingencies. We assessed the tools we had and did not have, normal brakes, nose wheel steering, only one thrust reverser, slow flap deployment, less effective spoilers. The landing was normal â€“ for me! â€“ and uneventful. We declined any assistance on the runway and told them we would be able to exit and taxi to a gate. We declined any further need of the CFR (Crash Fire Rescue) equipment as all was normal for the taxi. We taxied to the gate, parked, and shut down as normal. We had some complication at the gate with deplaning the passengers. The ZZZ3 Station Operations was there to meet us, but they were unsure whether to deplane or what the plan was. Then once they had a plan, they had a computer issue with the gate reader to accurately deplane the passengers in the system. With a few calls to Chief Pilot [and] their help from the Company Communications System, we were able to get the passengers off the aircraft and taken care of. We and the flight attendants debriefed and we deplaned. FO and I discussed continuing for the day and determined that we were safer not continuing. We worked with the crew desk to reschedule.

Synopsis

A320 Captain reported receiving an ECAM, HYD Y ENG PUMP PR LO during cruise flight. Due to the possibility of additional fluid loss from the unknown leak, the flight crew opted to land at an alternate airport.

ACN: 2000314 (411 of 2908)

Time / Day

Date : 202305

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Company Operations Manual
Aircraft Reference : X
Problem : Improperly Operated

Component : 3

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Inspector
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 2000314
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Day 0, maintenance management robbed #1 engine LH reverser translating sleeve from a serviceable #1 engine LH thrust reverser assembly Company part number (P/N) XX-XXXX-X-XXXX, this thrust reverser was made serviceable over a month prior on Repair Order# XXXXXXX. Robbed #1 engine LH thrust reverser sleeve Company P/N XX-XXXX-X-XXXY, was installed on Aircraft X without maintenance management following General Maintenance Manual (GMM) XX-XX-XX process, that is to fill out robbed unit tag and to make a serviceable tag after verifying the serviceable condition of the removed part. Also per GMM, the quality department Inspector is required to complete computer transactions to update removal and installation record of rotable part. Stores receiving storekeeper was notified to make serviceable tag for robbed #1 engine LH thrust reverser translating sleeve after Technician installed on aircraft (storekeeper is not responsible for the serviceability of any robbed part, that is the responsibility of the Technician removing the robbed part). Storekeeper also completed the removal transaction in the computer system for the robbed #1 engine LH reverser sleeve. Maintenance management, stores never followed the GMM XX-XX-XX process and quality department failure to support the removal and installation process (computer transactions, off log and on log of thrust reverse sleeve). GMM XX-XX-XX Item-XA Pg.1 through Pg.10 basically states the Technician removing or installing a robbed part or component from an aircraft is responsible for compliance as outline in this GMM, also GMM Item XA (13) states robbing a rotable component from another rotable component such as engine, APU or landing gear, requires computer transactions. Signee assigned to the component installation will be responsible to complete all required entries (This was not accomplished by quality department). Making leadership responsible for not adhering to GMM procedures, too many departments never read the GMM on robbed components, putting pressure on workers just to get the aircraft back in service regardless of the situation, even when they have no knowledge of what the process is. For example the storekeepers who have never got involved in robbed parts before and are now told it is now their job. The Company needs to stop taking short cuts and get serious with keeping their standards at a high level.

Synopsis

Inspector reported procedural errors caused a thrust reverser sleeve to be installed without correct certifications and tracking information after being removed serviceable (robbed) for installation on another aircraft.

ACN: 1999496 (412 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1999496
Human Factors : Human-Machine Interface
Human Factors : Fatigue
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Event / Encounter : Weather / Turbulence
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Rejected Takeoff
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were in ZZZ to ZZZ1 taxing out it was raining and overcast 1,700 ft. and rain. After three reroutes we then was cleared for position and hold. We aligned on Runway X be ready for immediate departure traffic 2 miles out. Aligned, I held the brakes powered up the engines to approximately 40% N1 FF approximately 1400 pounds, feeling the aircraft rumble/ shake engines stabilized are ready to go. Once cleared for Takeoff I then stood up engines throttles to the 12 o'clock position stable released brakes with aircraft moving then smoothly advanced to FLEX detent. Once in detent with engines powered up I could not keep aircraft on center line and was veering quickly to the right. Giving the engines No 2 sec more to catch up, veering uncontrollably to the right off center line. I rejected the takeoff, full reverse manual brakes and placing my hand on the tiller to align back to center line. This exacerbated the loss of nose control, nose wheel skidding on the wet runway left then right then to a slight left. I then once again hit the brakes to a quick total stop. All then cooled down on runway with reversers stowed. We then taxied off on the second left taxiway. This event on the runway was approximately 850 ft. total airspeed was not even alive for the First Officer (FO) "call" We both examined all engine gauges all was normal and sat for a moment to review what just went on called the flight staff on all call. They were all OK and passengers too then several passengers wanted to get off jet as we were think if it would it be prudent to go to gate for maintenance review. This was then with no hesitation of us to go back to the gate and deplane jet and allow maintenance to figure out what was the cause of engine issue. The use of the tiller exacerbated the loss of control of aircraft steering below VMC ground on a wet runway. The use of the brakes to a stop is when we truly regained total control of the Jet. Once we returned to gate is when we then realized the event and how my adrenaline was running. The station manager even noticed how our demeanor was and asked how we felt. I was reluctant to say then it hit me I need to wind the clock and slow down I then called myself off of the flight, fatigued not safe. I as Captain felt not safe to continue flying this day. Take the event in to adjust there was no hull loss nor any soul lost and the next day I flew as Captain ZZZ to ZZZ2 with no further issues.

Synopsis

A320 Captain reported a loss of directional control resulted in a rejected takeoff. The flight crew elected to return to the gate for maintenance action and to deplane several passengers who did not want to continue on the flight.

ACN: 1999021 (413 of 2908)

Time / Day

Date : 202305
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5021
Experience.Flight Crew.Last 90 Days : 166
Experience.Flight Crew.Type : 5021
ASRS Report Number.Accession Number : 1999021
Human Factors : Workload
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3920
Experience.Flight Crew.Last 90 Days : 194
Experience.Flight Crew.Type : 1426
ASRS Report Number.Accession Number : 1997547
Human Factors : Workload
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
Detector.Person : Flight Crew
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This started as a normal pushback with standard calls. When established on Taxiway 1, at the point after forward motion had stopped, the Captain, having felt he should have heard the standard request to set the brakes, made several attempts to contact the push crew on the intercom. While he was making his third or fourth attempt to establish communications, I felt backwards motion and looked out to see that we were moving and the tug was not. I told the Captain we were not attached and moving. Simultaneously, the ground crew made a frantic transmission to set the brakes. We were already moving when we received this call. This was the first time we heard from them (within the disconnect sequence) and they did not communicate further. Captain set the parking brake and the aircraft stopped. The parking brake engaging caused a very minor jolt, it wasnâ€™t any more than a normal brake application. I would guess we moved about 2 [to] 3 ft. The push crew did not re-establish communications. I remember seeing the ramper hold up the bypass pin, but the Captain was never given the standard calls. He tried to raise them several times after we stopped the aircraft, but they did not reply. The crew walked away, we did not have the opportunity to discuss the event. The Captain did call operations to tell them what had occurred.

Narrative: 2

This event was the result of a total and complete disregard of standard pushback procedures from the tug driver. Our aircraft was cleared to push off Gate X, tail west on Taxiway 1 and to disconnect abeam Gate Y. When the push was complete and aircraft was abeam Gate Y, with no further movement from the tug, the Captain made multiple calls to the driver asking if he was ready for the parking brake. At about this same time, the F/O (First Officer) looked over the nose of the aircraft to see if he could signal anyone on the ground that the Captain was attempting to contact the tug driver. Just as the F/O recognized the aircraft appeared to be rolling backwards, the Tug Driver finally and frantically told the Captain to set the parking brake. The parking brake was set with a noticeable jolt as the aircraft came to a stop. Captain responded, "Parking brake set pressure normal," and was about to inquire if there are any issues with the tug or aircraft. Immediately afterwards, Captain received the "aircraft safe to taxi" signal. The push crew arbitrarily DISCONNECTED the tug from the aircraft, and removed the NW BYPASS pin before any confirmation from the Captain the parking brake was SET. To recap; push crew never told Captain to "set parking brake" until aircraft was rolling backward, push crew never verbalized "Aircraft disconnected, by-pass pin removed." Push crew went off headset without Captain instructing push crew to "disconnect headset." More concerning, the push crew display an obvious lack of concern about the sequence of events because both the tug and rest of the push crew abandoned the aircraft as quickly as possible.

Synopsis

A320 flight crew reported the pushback ground crew failed to follow the company's pushback procedures which resulted in the uncontrolled aircraft to rollback. The Captain applied the aircraft brakes to regain control of the aircraft and prevent any damage.

ACN: 1996561 (414 of 2908)

Time / Day

Date : 202305

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1996561
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 390
Experience.Flight Crew.Last 90 Days : 171
Experience.Flight Crew.Type : 390
ASRS Report Number.Accession Number : 1996832
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Dispatch
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This morning, we pushed back and taxied out to Runway 30 when we were told to return to the gate due to our dangerous goods (dry ice) being improperly secured. Upon returning to the gate, the rampers and Dulles operations decided to remove all DG from the aircraft since it was causing them confusion on which box had DG in it. It was later verbally confirmed that all DG was removed from the aircraft from both our ground crew and [airport] operations. We never sent for another DG verification upon our second pushback since we were told all DG was removed from the aircraft. After takeoff, we noticed that we had received a DG receipt stating that we still had dry ice on the aircraft after being told it was all removed. We contacted our Dispatcher who also stated we had dry ice on. When landing, the Captain went out to verify with the rampers in ZZZ1 to confirm if we had DG onboard which they said we did, and lithium batteries which we were unaware of. We tried to message [airport] operations to clarify the confusion in flight through ACARS, but received no reply.

Narrative: 2

Flight XXXX had dangerous goods on board. We received the final DG sent msg and departed the gate. We then did not receive the weights and pulled into the run up block by Runway 30 ZZZ. While waiting, ground control advised operations requested us to return to the gate. Operations was contacted and advised us to return to gate because their was an issue with the DG being properly secured. We returned to the gate and both forward and aft cargo bins were opened. The forward was closed and work continued in the aft cargo bin. We were at the gate for an extended time. During this time I was keeping the passengers advised and also communicating with operations via the radio and when connected to headsets the ramp personnel. The cargo bins were closed and we were waiting again for final. In communication with operations we were told the additional delay was for ramp to confirmed what they had done with load planning. Then the doors were opened again. We queried operations again and were told that they did not have enough bags to cover the DG. That now the extended time at gate was due to removal of the bags to remove the freight, then reload the bags. I was told that they were removing all the freight because they could not identify the box that needed to be covered. I specifically asked confirmation operations and from the ground push crew that all freight was removed I was told yes, all freight removed. The subsequent push and taxi out was completed by the crew as if we did not have any dangerous goods on board. Our final weights were delayed again. When they did arrive they had reduced close to the amount of DG we thought was removed. The message also did not include a dangerous goods message at the bottom. With the arrival of weights, our attention was directed to final preparations for takeoff and completing the before takeoff procedures. It was not until climb out that the additional final dangerous goods message had also printed with the final weight message on the ground. This event happened because the crew was under the belief that all dangerous goods were removed from the flight and this printout was not necessary.

Synopsis

Air carrier flight crew reported the transport of undocumented Hazmat cargo. At destination unloading, it was discovered that in addition to the unloaded Dry ice, a parcel of lithium batteries was also discovered.

ACN: 1996325 (415 of 2908)

Time / Day

Date : 202305
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb
Route In Use : Vectors

Component

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1996325

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departed from XXR on ZZZZZ departure and on climbout received ENG THRUST REVERSE FAULT ECAM. Ran the ECAM procedure and consulted QRH. I remained PF (Pilot Flying) and the CA (Captain) contacted dispatch where we were advised to return to field. We ran the overweight landing checklist and other appropriate checklists and landed uneventfully on XXC. Emergency vehicles conducted an inspection after we taxied off the Runway and escorted us to the Gate. Aircraft had been written up twice in April for the same issue. Further maintenance investigation into repeat issues.

Synopsis

A320 First Officer reported receiving an ENG THRUST REVERSE FAULT ECAM on climb out. The flight crew performed a return to the departure airport.

ACN: 1994749 (416 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1994749
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Procedure
Primary Problem : Airport

Narrative: 1

After taxiing into gate at ZZZ after operating flight from ZZZ1 to ZZZ we experienced an uncommanded aircraft movement while the airplane was chocked after block in. The event occurred after releasing the parking brake in order to cool the aircraft's brakes in accordance with an ECAM message. After the parking brake was set to on after block in and verifying that the aircraft was chocked, I released the parking brake which resulted in the aircraft slowly moving backward. After realizing that the aircraft was in fact moving rearward and not just rocking into a stopped position against the chocks, I immediately applied the brakes and set the parking brake to on to stop the aircraft. After communicating with the ground crew once the aircraft was stopped, I was informed that the nose wheel of the aircraft had crushed the chocks and rolled on top of them. Fortunately, my prompt response was able to stop the aircraft before the nosewheel continued beyond the crushed chocks which could've resulted in the aircraft impacting ground personnel, the jetbridge, ground equipment or another aircraft. After further investigation, I was notified that incorrect chocks were used to chock the aircraft and that they were not rated for airplane use. A further safety investigation found that the chocks used on the ramp that day were actually meant for ground service equipment and were inadvertently mixed in with other airplane rated chocks which allowed incorrect chocks to be used on our aircraft. After making sure that the aircraft was no longer a threat to having further uncommanded movement, we verified that no person was injured and that no damage had occurred during the event.

Synopsis

A320 Captain reported nose wheel's uncommanded roll over improper chocks resulted in ground loss of control.

ACN: 1993743 (417 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component : 1

Aircraft Component : FMS/FMC
Manufacturer : FMGC1
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : FMS/FMC
Manufacturer : FMGC2
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1993743
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During climbout we experienced a Dual FMGC Failure. After multiple attempts to reset the system in guidance with Maintenance we're unsuccessful we returned to ZZZ after consulting with Dispatch. Before the Systems Failed, we had a CAT 3 Dual ECAM Message. While we were looking into the reason for this both FMGCs failed which resulted in the Autopilot Disconnecting. We were able to use the backup NAV and use VOR navigation but due to lack of Autopilot we couldn't remain in RVSM Airspace. When I queried the Dispatcher about fuel burn to ZZZ1 he stated that we would land with 7.3. We found this odd because our original release had us landing with 7.0 and a cruise of FL350. I didn't feel we could realistically make ZZZ1 at FL270 and we decided that a return to ZZZ was best.

Synopsis

B737-800 pilot reported a Dual FMGC Failure during cruise. The flight crew ran the QRH and checklists, then contacted Dispatch. Due to doubts as to the fuel on board being sufficient to reach the destination airport, the flight crew performed an air turn back to departure airport.

ACN: 1993578 (418 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors

Component : 1

Aircraft Component : Nose Gear
Manufacturer : LGICU2
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Squat Switch
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 779.65
Experience.Flight Crew.Last 90 Days : 101.82
Experience.Flight Crew.Type : 1205
ASRS Report Number.Accession Number : 1993578

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After a normal departure and gear retraction we noticed a significant noise/banging sound coming from the Nose Gear Area. It was intermittent yet continued for several minutes. Initially we thought it may have been a Blown Nose Tire and asked ATC to check for debris on the runway. Eventually we received a Landing Gear Control Interface Unit 2 Master Caution. The Captain assigned me as Pilot Flying (PF) and he accomplished the appropriate checklist, spoke with M/C and Dispatch as well as communicated with the Flight Attendants and ZZZ OPS. We requested priority handling and elected to return to the ZZZ. I programmed the FMS and coordinated with ATC while he completed his duties. Lastly we ran the overweight landing checklist together while waiting for landing data from Dispatch. I was the PF for the remainder of the flight. The landing was smooth and uneventful.

Synopsis

A320 First Officer reported after a normal departure and gear retraction, noticing significant Noise/Banging Sound coming from the Nose Gear Area. Eventually, the flight crew received a Landing Gear Control Interface Unit 2 Master Caution and decided to return to the departure airport.

ACN: 1993174 (419 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 700

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : RNAV
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 85
Experience.Flight Crew.Type : 694
ASRS Report Number.Accession Number : 1993174
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Commercial
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 34.87
Experience.Flight Crew.Type : 34.87
ASRS Report Number.Accession Number : 1993207
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

ZZZ to ZZZ1. Descent brief noted the terrain in vicinity of the airport and points near final for the RNAV (GPS) XX. Winds were 280 at 13G20. First Officer (FO) was flying the approach. Vectors were to a left downwind, base and final. Configuration and checklists were as briefed. Approach mode was armed and the managed/managed V/DEV "brick" was captured. The FO's approach was stabilized and he clicked off the autopilot and auto thrust at approximately 1,500 feet AGL. The gusty conditions made it a challenging approach, and at approximately 900 feet MSL I felt a slight sink due to the gusty winds. I glanced inside, noted the V/DEV was slightly above the horizon and then looked outside to see our PAPI indicators transitioning from 2 red/two white to three red. I verbally instructed the FO to level off and regain the two red and two white PAPIs. As he was accomplishing this, we heard/perceived a "terrain ahead" Enhanced Ground Proximity Warning System (EGPWS) caution call out. My previous verbal instruction had him accomplishing the first step from the FM of adjusting the flight path. As I analyzed the situation for a possible go around or escape maneuver, it quickly resolved, we regained two red/two white PAPI indications and landed. We debriefed the flight, specifically talking about the terrain notification verbiage and I briefed approach techniques as the FO is inexperienced. I received a call from the FOQA gatekeeper and we debriefed the flight. He had additional information and his data showed us receiving a "too low terrain" warning message but no "pull up" command. The FO and I both do not recall hearing a "too low terrain" nor "pull up" command, which would have triggered us to accomplish the terrain escape maneuver.

Narrative: 2

On the RNAV XX approach into ZZZ, at approximately a 2-4 mile final, a GPWS "Too Low Terrain" alert was annunciated. Auto pilot was off and as best as I can recall, the autothrust was off by that point. Upon hearing the GPWS, I shallowed out the descent and established 2 red, 2 white on the PAPI to continue the approach to land (the approach plate notes VGSI and RNAV glidepath not coincident). It was nighttime VMC with gusty winds. Captain and I debriefed that the vertical deviation brick on the approach was centered with minor deviations throughout the approach past the final approach fix. No excessive descent rate was indicated. No indication on the navigation display or Primary Flight Display indicated that terrain closure rate was excessive. We discussed that the approach plate indicates an elevation of 510 feet along the final approach course approximately 1-3 miles from the runway and a likely cause of the GPWS annunciation. We also debriefed that had the GPWS annunciated "Terrain, Pull Up" an immediate escape maneuver would have been required. Given the fact it was nighttime, and a terrain alert was annunciated, I should have performed a go-around regardless of vertical deviation scale indications. At the time of submitting this report, Captain and I have both debriefed with an FOQA gate keeper discussing the events of the approach and agreed a safety report was warranted. Additionally, we both agree that in hindsight, a safety report was warranted immediately following the flight.

Synopsis

A320 flight crew reported receiving an EGPWS terrain warning while on an RNAV approach in windy conditions. The First Officer, per the Captain, was inexperienced and the aircraft briefly went below the glide-path. A normal landing was accomplished.

ACN: 1992971 (420 of 2908)

Time / Day

Date : 202304

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 426.27
Experience.Flight Crew.Last 90 Days : 139.15
Experience.Flight Crew.Type : 426.27
ASRS Report Number.Accession Number : 1992971
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Incomplete information regarding DG. What is required to acknowledge receipt. What documentation needs to be completed prior to dispatch. Who needs to be notified.

Synopsis

Air carrier Captain reported incomplete DG documentation was provided to flight crew during pre-flight.

ACN: 1991480 (421 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Relative Position.Angle.Radial : 305
Relative Position.Distance.Nautical Miles : 27
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class E : ZZZ

Component

Aircraft Component : INS / IRS / IRU
Manufacturer : ADR3
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22148
Experience.Flight Crew.Last 90 Days : 46
Experience.Flight Crew.Type : 3946
ASRS Report Number.Accession Number : 1991480
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We operated an Airbus A320, on a part 91 ferry flight from ZZZ with an intended destination of ZZZ1. The flight was dispatched with an inop ADR 3 (Air Data Reference) compliant with the aircraft's MEL, provided that IR1, IR 2, ADR 1, and ADR 2 are operative. They were. Shortly after take-off, we began receiving a stream of amber ECAM messages. Messages included VENT Blower Fault, NAV ADR 3 Fault, AUTO FLT AP OFF, and other navigation-related faults. After complying with ECAM procedures, and troubleshooting, we determined that we could not recover the autopilot. As the autopilot is required for RVSM operations, and as we planned to climb to FL390, we realized that we would have insufficient fuel reserves at ZZZ1 flying at FL270 with the higher fuel burn. We elected to return to our point of origin, ZZZ1, for repairs. While descending into the ZZZ2 airspace, we were aware of significant VFR traffic at ZZZ3 and other VFR aircraft operating in the local area. When we requested vectors and a descent for landing at ZZZ2, ZZZ Approach Control advised us that our dispatcher requested we divert to ZZZ. In what appeared to us to be an encouraging tone of voice, the Controller asked if we were requesting priority handling. We affirmed, provided fuel and souls on board, and stated that we would not need Crash Fire Rescue (CFR) equipment standing by and that our problem was multiple indications of failures of secondary systems, and that our return to ZZZ was driven by the fuel problem that resulted from our technical inaccessibility of RVSM airspace. The landing at ZZZ was otherwise normal and uneventful. In debriefing this flight, we concluded that the multiple issues we encountered seemed to stem from the inoperative ADR 3.

Synopsis

A320 First Officer reported encountering navigation system failures after departure that would preclude RVSM operations and there was insufficient fuel to complete the flight. The flight crew diverted to another airport and made a normal landing.

ACN: 1991320 (422 of 2908)

Time / Day

Date : 202304

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1991320
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Ground Personnel

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Lead working Aircraft X loaded DG on the aircraft however when he locked the flight, he held off the shipment. DG flew undeclared to Captain.

Synopsis

Lead Ramp personnel reported Dangerous Goods parcel flew undeclared without Captain notification.

ACN: 1990713 (423 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 440
Experience.Flight Crew.Last 90 Days : 192
Experience.Flight Crew.Type : 440
ASRS Report Number.Accession Number : 1990713
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 732
Experience.Flight Crew.Last 90 Days : 149
Experience.Flight Crew.Type : 732
ASRS Report Number.Accession Number : 1990749
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Dispatch
Detector.Person : Flight Crew
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Crew did not receive final dangerous goods report prior to brake release and pushback. Realized error and waited for report prior to taxi.

Narrative: 2

I pushed back off the gate in ZZZ without receiving the Final DG (Dangerous Goods) notification. It was completely inadvertent and once I realized my error, we did not taxi away from the gate until I had the proper form and sent in the acceptance code via ACARS. I read the most recent pilot bulletin and for some reason, I was thinking that I needed the Final Report before departure as in before takeoff, not before departing the gate. It was just a brain hiccup on the last leg of the last day of our 3 day trip as we have always needed the Final DG report before pushback.

Synopsis

A320 flight crew reported receiving the final dangerous goods report during pushback. The flight crew final dangerous goods acknowledgement response was submitted prior to taxi.

ACN: 1989935 (424 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1600

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 770
Experience.Flight Crew.Last 90 Days : 142
Experience.Flight Crew.Type : 770
ASRS Report Number.Accession Number : 1989935

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X was on a vectored / modified turn from downwind to base, just outside of ZZZZZ FAF for a visual approach to ZZZ Runway XX, speed 190 Knots, flaps1, when ECAM HYD B RSVR LO LVL was triggered. ECAM procedure was performed, and priority handling was requested, the approach was continued to an uneventful landing.

Synopsis

A320 Captain reported receiving an ECAM HYD B RSVR LO LVL Message on final approach. ECAM procedures were performed and the flight crew continued the approach and landing to destination.

ACN: 1989795 (425 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 1801-2400

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Fog
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1989795
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Scheduled flight was ZZZ-ZZZ1 leaving at XA05L. Flight departed early, made it as far North as abeam Location A, then elected to air return to ZZZ due to a loss of all or nearly all hydraulic fluid. Decision to air return was largely based on the extremely low visibility in and around our ZZZ2 metro destination and the loss of a CAT3 dual ILS capability. The decision was made in collaboration with Dispatch and ops via SATCOM. We returned to ZZZ at XB30L, did a tail swap from Aircraft X to Aircraft Y, then departed ZZZ again at XC40L. We were 10 minutes from landing in ZZZ1 at XD00L when ZZZ1 closed their Tower operations per their normal schedule. ZZZ1 was reporting VV001 and 1/4 mile visibility with 2200-3000 RVR. Since the Tower had closed the ILS XX CAT2 into ZZZ1 it became illegal and, since we didn't have the 200-1/2 minimums (specifically the ceiling minimums) for an ILS XX CAT1, we diverted to ZZZ3. Although ZZZ4 was our listed alternate, the pre decision made in coordination with Dispatch as we approached ZZZ1 was to first try to divert to ZZZ3 (if feasible) should ZZZ1 weather be illegal.

Synopsis

A320 pilot reported the loss of the hydraulic system after departure and return to departure airport. The flight crew was assigned a different aircraft and departed, but had to divert when destination airport weather deteriorated below minimums.

ACN: 1989063 (426 of 2908)

Time / Day

Date : 202304

Place

Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 13146
Experience.Flight Crew.Last 90 Days : 193
Experience.Flight Crew.Type : 9522
ASRS Report Number.Accession Number : 1989063

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 9958
Experience.Flight Crew.Last 90 Days : 96
Experience.Flight Crew.Type : 7863
ASRS Report Number.Accession Number : 1989068

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While in cruise we received an ECAM indicating green system hydraulic lo pressure. Upon further investigation found green system hydraulic low level. Systems display indicated loss of green hydraulic system quantity. Followed and completed all checklists. [Requested priority handling]. Requested landing on Runway XXR at ZZZ due to gusty winds (170 @17g27)and loss of nose wheel steering. Uneventful landing was accomplished. Stopped on runway and towed to hangar.

Narrative: 2

While in cruise we got a HYD Green Engine 1 Pump Lo Pressure, HYD G RSVR Low Lvl and HYD Green SYS Pressure. We performed the ECAMS, contacted Dispatch/Maintenance Control and [requested priority handling]. We requested Runway XXR at ZZZ due to high winds and loss of nose wheel steering. We landed uneventfully and was towed off the runway and to the hangar.

Synopsis

A320 flight crew reported loss of green hydraulic system quantity in cruise. Flight crew continued to destination airport and landed, and aircraft was towed to hangar.

ACN: 1988977 (427 of 2908)

Time / Day

Date : 202304
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component : 1

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1988977

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1988979

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb out to FL 380 we had an ADV (Advisory) message come up and the ENG page displayed. The ENG2 oil quantity was at 1.0 quarts. We immediately contacted Maintenance and went to the procedure. The procedure in the ADV checklist says to monitor other engine parameters and proceed. Our message back from Maintenance said likewise. We sent them the oil pressure and temperature readings. At the time the pressure was 150 PSI and the temperature was 90 degrees C. The #1 engine was also at 90 degrees C, but the pressure was 170 PSI. So the #2 was slightly lower. We started getting checklists bookmarked for low oil pressure and eventual engine shutdown if it came to that. We notified the very senior lead Flight Attendant (FA) of the issues. They were a godsend to have on this day. The pressure dropped incrementally as we progressed. We notified Maintenance when it hit (I believe) 135 PSI. Then again when it got to just a hair above the AMBER bar. At this time we got a message from our Dispatcher, to return to ZZZ. We started our turn for ZZZ and I called lead FA. As we were making the turn we got the RED ECAM of low oil pressure. The ECAM procedure has you pull the thrust lever to idle and shut off fuel to the engine. We did that, and we got a call from ZZZ Center asking if we were [requesting priority handling]. We [requested priority handling] and asked for lower at the same time as we were at FL380. They gave us FL360 and I told them we needed lower immediately as our Multipurpose Control Display Unit (MCDU) was showing FL254 as I recollect for highest. They gave us FL230 and since we had the fuel we went to FL230. At this point we finished the procedures for engine shutdown and traded controls and I became Pilot Flying and the First Officer (FO) became Pilot Monitoring. We did balance the fuel as we descended some. We had been cleared to the FAF on ILS XXL but when handed over to ZZZ1 Approach they switched us to RNAV YY. The winds favored that runway and it is over 11000 ft. long. We changed our approach. We re-briefed the approach and flew the RNAV to YY. I spoke to the lead FA multiple times and also made a PA to the passengers about what was going on. We landed without incident and came to a stop on the runway to evaluate before turning off onto taxiway. Fire trucks were located and could see that we had no issues. We taxied to gate.

Narrative: 2

During climb (approximately at FL360) we got a flashing "adv" [alert] with an engine page indicating oil quantity 1.0 quart. We contacted Maintenance and advised them about the situation. We were informed that if all of the other indications were normal (they were) we were good to continue. We continued to monitor the engine page. Oil quantity was showing 0 quart and the pressure was slowly decreasing. At one point the PSI indication changed to amber and we got amber ENG 2 OIL LO PR on ECAM. We referenced the procedures. Contacted Dispatch and was advised by our Dispatcher to turn around and go back to ZZZ. As we were turning back, we got RED ENG 2 OIL LO PR, followed ECAM actions which instructed us to follow eng 1(2) shutdown procedure. Since we were at FL380, we had to request lower altitude (initially we were given FL360 then FL230). After the engine shutdown, we [requested priority handling]. Per Dispatch's recommendation, we elected to divert to ZZZ1. During this time, we switched our roles and I continued as PM for the rest of the flight. The remainder of the flight was uneventful. We landed on Runway YY, stopped on the runway for a short period of time to verify that everything was fine and taxied to our gate.

Synopsis

A320 flight crew reported loss of oil quantity and oil pressure for number two engine in cruise. Flight crew shut down the engine and completed a diversion and landing.

ACN: 1988592 (428 of 2908)

Time / Day

Date : 202304

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1988592
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After takeoff from Runway XXL in ZZZ, ECAM for green low pressure occurred. Could hear power transfer unit operating. G low pressure turned into a green low level. Yellow electric pump overheated with power transfer unit continually running. Ran ECAM and turned off power transfer unit and yellow electric pump for overheat. Overheat was resolved and we regained yellow system. We got extended vectors for the air return. While on vectors we completed the ECAMs, comms, and got landing performance numbers. Performed a flap 3 landing under gross weight on runway XXL in ZZZ. Stopped on runway and was inspected by airport rescue and firefighting vehicles. Taxi back to the gate under aircraft power followed by an airport rescue and firefighting vehicle. Attempted APU start twice. Both times resulted in APU auto shutdown. Upon arriving at the gate, passengers were deplaned.

Synopsis

A320 First Officer reported multiple failures of the hydraulic system shortly after takeoff necessitating a return to the departure airport.

ACN: 1987274 (429 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Pitot/Static Ice System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : FCC (Flight Control Computer)
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Data Processing
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 23905
Experience.Flight Crew.Last 90 Days : 91
Experience.Flight Crew.Type : 12966
ASRS Report Number.Accession Number : 1987274
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 3506
Experience.Flight Crew.Last 90 Days : 94
Experience.Flight Crew.Type : 3506
ASRS Report Number.Accession Number : 1987293
Human Factors : Confusion
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

While in cruise at FL360 in the clear, we got an ANTI ICE STBY PITOT ECAM. While First Officer (FO) Person A continued to fly the aircraft, I took care of the ECAM. No action items were displayed and I referenced non normals page for further guidance. The FM (Flight Manual) only indicated that we had to monitor the air data information if standby instruments were used. Our maintenance history showed that this was a repeat write up and I proceeded to send a maintenance request code through ACARS. The ECAM was cleared and the screens were normal. We initiated our descent into ZZZ with Autopilot2 engaged west of the ZZZ1 VOR when we received another ECAM, this time for a L/G Shock Absorber Fault. Before we could even address that, we received the Autopilot Disconnect Warning Tone, an Auto Thrust Lock ECAM, and both Flight Directors (FDs) failed. We regained manual control of the throttles and FO continued to fly with the flight path vector selected and leveled off at FL220. We attempted to reengage the Autopilot, Auto Throttles and Flight Directors to no avail. All Flight mode Annunciators (FMAs) on both of our Primary Flight Displays (PFDs) were blank except for the barometric minimums altitude for the RNAV XXL approach to ZZZ we had previously selected. Our Navigation Display (ND) displays still showed a course line, the Multipurpose Control Display Unit (MCDUs) still had all of the navigation information displayed and our PFDs still showed accurate attitude, altitude, heading and airspeed indications as cross referenced with our standby instruments. GPWS terrain information was not available. FO continued to fly the aircraft while I performed the FMGC Dual Failure procedure. Since the FMGCs did not recover we attempted the short FMGC reset with no effect. An ARINC three-way call was set up with Dispatch and Maintenance Control to discuss our plan of action since our approach capability was limited to raw data ILS and visual approaches. The only ILS available at ZZZ was for Runway XYR and the winds were out of the southeast at 16 kts. so that was out of the question. The weather in ZZZ2 was also questionable with low ceilings and strong cross winds. ZZZ3 was VFR so we agreed to divert there. We advised ZZZ Center. They were advised of our situation and radar vectors were provided to ZZZ3. I briefed our Purser with the aircraft situation, our diversion plans, how long we had before landing and our after landing plans. I then made an announcement to the Passengers making them aware of our situation. Dispatch soon provided all landing data for Runways XZR and XAL at ZZZ3 through ACARS. Half way to ZZZ3 we regained both of our FMGCs, FDs, Autopilot and Auto Thrust. All systems were re-engaged while we actively monitored for possible failures. Vectors were provided for the visual approach to Runway XZR at ZZZ3, the Auto Throttles and Autopilot were disconnected and the approach and landing were flown uneventfully. Taxi to gate number XX was also uneventful and after parking, a full verbal debrief was conducted with our waiting Mechanics.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported experiencing multiples system failures and ECAM warnings during cruise. After running through the appropriate checklists the crew diverted for a safe landing.

ACN: 1986770 (430 of 2908)

Time / Day

Date : 202303

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class E : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1986770

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Normal takeoff out of ZZZ. And cleared left turn on course after T/O. I, the First Officer (FO) was the Pilot Flying and was still hand flying. A left turn was made and we continued our climb. As I leveled the turn I was cross referencing the instruments and visuals when I looked and saw a flock of geese 12 o'clock. I yelled out birds. The geese where unavoidable and we impacted within seconds of me calling them out. Damage to Engine #1 was apparent as the vibrations it was putting out where extremely high. And you could feel it throughout the airframe. The Captain selected the Fuel Control Unit (FCU) ALT of 4,000 feet and a decision to return to ZZZ was made as we didn't know the extend of airframe damage that had occurred. I started a decent and left turn for a downwind back to Runway XX. The Captain alerted the Flight Attendants of the situation and advised ATC. The Captain ran the overweight landing checklist. The secondary flight plan was then activated and the ILSXX was already programmed by me on the preflight so I set the airplane up for a base and intercepted the localizer coming back to ZZZ Runway XX. A visual approach was flown and a smooth landing was done for the overweight procedure. We then taxied to the gate and deplaned as normal and resumed normal ops. Upon deplaning we inspected engine #1 to find serious blade and engine damage.

Synopsis

A320 First Officer reported a bird strike shortly after takeoff. Geese were observed striking the aircraft and the number one engine resulting in heavy vibrations and severe damage to the engine. The crew returned to the departure airport.

ACN: 1985949 (431 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Make Model Name : Medium Transport, Low Wing, 2 Turbojet Eng
Crew Size.Number Of Crew : 2
Flight Phase : Takeoff / Launch

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1985949
Human Factors : Situational Awareness
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1985952
Human Factors : Situational Awareness
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Runway
Detector.Automation : Air Traffic Control
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X, landed [Runway] XXL, exited Taxiway 1, possibly [Taxiway] 2, clear of runway, Tower instructed us to hold short of [Runway] XXR at [Taxiway] 3. As I was making a slow turn at [Taxiway] 3 to hold short of [Runway] XXR, Tower cleared us to cross [Runway] XXR at [Taxiway] 3. FO (First Officer) cleared right, I cleared left visually and verbalized, turned appropriate lights on, and intentionally did not disarm speed brakes to prevent FO from being distracted with after-landing flow. As we crossed the hold bars and I gained a better perspective to our right, of potential traffic on [Runway] XXR, Tower told us to hold position and I nearly simultaneously saw an aircraft approaching, potentially taking off, on [Runway] XXR. We of course immediately stopped. I quickly thought about reversing, but we had one reverser inoperative, and although across the hold bars, we were clear of the actual runway surface. Also, the departing aircraft was a narrow body, so geometrically a collision shouldn't occur. After the departing aircraft became airborne, Tower cleared us to cross again, and we confirmed they had initially cleared us to cross, which they confirmed.

Narrative: 2

Aircraft X. Upon landing on Runway XXL, we exited the runway at Taxiway 1 and were instructed by ATC to hold short [Runway] XXR at [Taxiway] 3. We did. Shortly after, ATC cleared us to cross [Runway] XXR at [Taxiway] 3. Both the Captain and I looked to make sure the runway was clear. On my side, I saw the lights of an aircraft at the end of the runway that appeared to be waiting â€“ line up and wait. We started moving forward slowly at that time. Then I saw the aircraft appear to be moving, and at the same time, the controller told us to stop. By this time, we were past the hold-short line, but still short of the runway edge line. Too close for comfort, however. The departing aircraft lifted off prior to our intersection. Shortly after, ATC cleared us a second time to cross. I clarified with them that they cleared us to cross previously and they confirmed "yes." After that, we taxied to the gate uneventfully.

Synopsis

A320 flight crew reported a critical ground conflict while taxiing to the gate. ATC cleared the flight crew to cross but there was another aircraft that was taking off on the runway prior to the intersection.

ACN: 1984908 (432 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class E : ZZZ

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1984908

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff during climb out of ZZZ, received ECAM â€“ BRAKES AUTO BRAKES FAULT, BRAKE RELEASED. Non normal procedures accomplished. Discovered required landing distance with this failure was greater than landing distance available in ZZZ1. Commenced a phone patch with AIRINC to speak with Dispatch and Maintenance. Dispatch concurred that the landing distance available in ZZZ1 did not meet requirements. Decision was made to divert to ZZZ2. Briefing given to In-flight. Customers were advised. Diversion checklist accomplished. Requested longest runway in ZZZ2 and cleared Visual XXL. Aircraft landed and braked uneventfully. Flight terminated in ZZZ2 and Customers were given transportation to ZZZ1. Maintenance write up was completed and spoke with Maintenance.

Synopsis

A320 Pilot reported receiving an ECAM for BRAKES AUTO BRAKES FAULT, BRAKE RELEASED after takeoff during climb out. The flight crew consulted with dispatch and maintenance and diverted to make a precautionary landing.

ACN: 1983541 (433 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1983541

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X. While cruising at 39,000 ft., we received a Master Caution with a Y ENG 2 PUMP LO PR ECAM Message. We accomplished the ECAM actions, and analyzed the Hydraulic and Status pages. The Yellow System Fluid level was well below normal but there was still a small amount in the reservoir. We reviewed the ECAM procedure and discussed a plan of action. There were no significant in flight issues, but the affected systems on the status page eliminated Nose Wheel Steering, Reverser 2 and Ground Spoilers 2 and 4. We discussed alternate airports and decided that ZZZ was the most suitable given our proximity and the combination of wind straight down the runway, runway length, Airport Rescue and Firefighting (ARFF) services and available Maintenance. In the meantime we received associated ECAM Messages for Y RSVR LO LVL and Y SYS LO PR. We reviewed the ECAM actions, which required no additional steps beyond those we had already taken and reviewed the COMM procedures. The Hydraulic Fluid Level continued to decrease throughout the flight but did not change our analysis of the situation. After [requesting priority handling] with ZZZ Center we contacted Maintenance and Dispatch via ACARS, who relayed the situation to Base Operations at ZZZ. The Captain made the appropriate announcement to the Passengers. From there we proceeded to ZZZ, requesting longer vectors to account for slow flap deployment. Once safely on the ground we came to a stop on the runway and had ARFF inspect the aircraft for damage and had a Company Mechanic specifically inspect the Nose Gear. We continued to update the Passengers. The Captain was able to safely taxi the aircraft off the runway and to the ramp where we were met by Maintenance and deplaned the Passengers.

Synopsis

First Officer reported receiving a Master Caution with a Y ENG 2 PUMP LO PR ECAM Message in cruise. The flight crew elected to divert.

ACN: 1983475 (434 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class E : ZZZ

Component : 1

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : PFD
Manufacturer : CAPT PFD
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : PFD
Manufacturer : CAPT ND
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1983475
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1983476
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Just prior to turning downwind at 9000 ft. on the ZZZZZ arrival for XXR, the Captain had their Primary Flight Display (PFD) and Multi-function Flight Display (MFD) flicker off and back on. They mentioned this to me and after I looked over and began to smell a burning smell that quickly intensified. I also noticed what appeared to be some haze potentially from smoke. The Captain retained control of the aircraft and we immediately donned our oxygen masks and I began working the QRC. We then had a frequency change and I checked in [to advise ATC]. The Controller tried to change our runway to XXL but we elected to land on XXR which we had previously briefed and to avoid adding to our workload. We landed without further incident and the fumes seemed to clear. The Airport Rescue and Firefighting (ARFF) evaluated our aircraft for hot spots which didn't exist and we taxied to the gate where they made further checks. After the event, Maintenance found a burned wire and also replaced the Captain PFD.

Narrative: 2

On descent into ZZZ on Aircraft X, and passing through approximately 9000 ft., I had on the Captain's side Primary Flight Display (PFD) (while I was flying) go completely black. Then the PFD appeared on the side of the NAV display and the NAV display appeared on the side of the PFD. Soon after that, the First Officer (FO) and I smelled a very strong electrical burning smell. We both donned our oxygen masks. We [advised ATC] and ran the appropriate QRC checklist for smoke & fumes. There was a haze in the flight deck near the Captain's side of the aircraft. We rolled the Airport Rescue and Firefighting (ARFF). The Flight Attendants (FAs) were notified. ATC asked us for the total souls on board and fuel remaining. I landed uneventfully on Runway XXR (the runway we were set up for and previously given). We cleared the runway and the ARFF checked the aircraft for hot spots. No hot spots were found. I took off my oxygen mask to check the flight deck fumes and it seemed that the fumes had dissipated. I made an announcement to the passengers and we taxied to the gate. The fire trucks followed us to the gate. Once we arrived at the gate, both the FO and I opened our windows and ran the shutdown checklist. The Lead Fire Fighter came on the flight deck and checked the flight deck temperature with a hand held device and said it appeared that the temperatures were normal in the flight deck. The Fire Fighter gave the FO and I a "fist bump" and said great job. It was brought to my attention that maintenance took the aircraft out of service and later found "broken wires" and replaced these wires. Better inspections could be helpful.

Synopsis

A320 flight crew reported a loss of the Captain's PFD and ND on final approach with an electrical smell and smoke noticeable in the cockpit. The flight crew donned O2 masks and continued to destination airport where a maintenance inspection discovered wiring issues.

ACN: 1981612 (435 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16800
Experience.Flight Crew.Last 90 Days : 135
Experience.Flight Crew.Type : 1557
ASRS Report Number.Accession Number : 1981612
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Upon arrival at gate, we used the auto park system and stopped. The system told me OK on the screen. The ground crew stated I had gone 1 foot too far and needed to push us back. After waiting 10 mins we started the push back. The first attempt failed as we were pushed too far. The second attempt pulling forward also failed. On the third attempt on pushback the ground crew started yelling "Set Brakes! Captain Set Brakes!". I told my First Officer (FO) there is no way I went this far. Come to find out after I hit the brakes that the aircraft had rolled off the tug and was rolling backwards. I looked to my rear and a catering truck was directly behind me. The term "breakaway breakaway breakaway" was never used.[The] terminal has all new employees and many errors have occurred since opening. This one was big.

Synopsis

A320 flight crew reported the aircraft rolled backwards after breaking away from the tug while the aircraft was under tow at the gate.

ACN: 1981241 (436 of 2908)

Time / Day

Date : 202212

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Deplaning
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1981241
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

When we landed back in ZZZ from ZZZ1, the LFA (Lead Flight Attendant) told us that we had a huge risk on our flight that we didn't know about until we have deplaned. They said they saw a personal oxygen machine that you plug into the wall at home being brought up for our passenger who was deplaning in ZZZ and LFA asked what is that and where did it come from. Apparently it was checked in and was with the baggage under the plane from my understanding, the way the LFA explained it to us all. I never got to see it because I was in the back of the plane and by the time I made it to the front, all passengers had deplaned. From my understanding, personal oxygen is not allowed to be brought on the plane and kept on person or be checked in. This is a very serious matter and needs to be looked into before this passenger tries to travel back with it on the next flight or if someone else brings it and employees don't follow the safety protocols/rules. It's a disaster waiting to happen in the air. Train ticket counter agents, gate agents, and bag loaders to not accept personal oxygen systems to be loaded under the plane with the luggage to prevent aircraft fire/explosion.

Synopsis

A320 Flight Attendant reported being notified after arriving at destination that a passenger packed a prohibited personal oxygen machine in checked baggage.

ACN: 1981190 (437 of 2908)

Time / Day

Date : 202301

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1981190
Human Factors : Troubleshooting

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Mobility device battery in cabin. At the completion of our boarding process, as we received our Baggage Loading Record, I was notified by the Lead Flight Attendant that a mobility device battery had been brought to the cabin by a passenger as a carry-on. I went to the jetway to check with the Gate Agent in order to get the details. The passenger had used a partially foldable mobility device. I asked the Ramp Worker who had delivered the Baggage Loading Record about the device and they confirmed that it had been loaded without a battery and that the device's exposed battery was removed before bringing the device downstairs. The Gate Agent put me on the phone with the Gate Supervisor, and they confirmed that for certain mobility devices, they remove the battery and have the passenger board with the battery as a carry-on. They said that it was something that was not unusual. The battery was in a protective molded housing that was obviously designed for the particular device with unexposed terminals. During our phone conversation, I referenced the manual and Companyâ€™s Dangerous Goods Table. There are three table entries for batteries in general and another five entries for mobility aids. It appeared that the device we had loaded fit the description of a Collapsible Mobility Aid, which allowed for the battery to be removed and carried in the cabin. I briefed the Lead Flight Attendant about my conversation with the Gate Agent and we departed. There are numerous mobility device options for the traveling public. As flight crew, we are regularly notified by Ramp personnel about mobility devices that are loaded in the cargo bins and by their inclusion on the Baggage Loading Record. When batteries are removed from certain mobility devices and passengers board with these batteries as carry-on items, there should probably be a notification process beyond just our awareness that we've got a mobility device in a cargo bin. If the battery has been removed from such a device and treated as a carry-on, the flight crew and cabin crew should be notified.

Synopsis

A320 Captain reported a mobility device battery was loaded in the passenger cabin as carry-on baggage. The Captain verified the battery met the air carrier's safety regulations for cabin transport before departure.

ACN: 1981166 (438 of 2908)

Time / Day

Date : 202301

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class D : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1981166

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

GREEN HYD SYSTEM FAILURE. After takeoff during gear retraction, Green Hydraulic System Failed. Performed ECAM Actions, notified Dispatch and ZZZ OPS. Confirmed 3 Green, notified Flight Attendants that this was an immediate situation. Went into holding, After all checklist and inflight duties along with reducing landing weight, (max landing weight NOT exceeded) returned for landing at ZZZ with no issues. Normal taxi to the gate. After landing and post flight , verified Hydraulic Leak, right side wheel well with fluid trailing aft. Just very busy working through everything which takes time. Everything worked well with preparing for a air return.

Synopsis

Captain reported a hydraulic system failure after takeoff during gear retraction. The flight crew performed an air turn back and precautionary landing at the departure airport.

ACN: 1980661 (439 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors
Airspace.Class E : ZZZ

Component : 1

Aircraft Component : Powerplant Lubrication System
Manufacturer : #2
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Turbine Engine
Manufacturer : #2
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Turbine Engine
Manufacturer : #1
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Compressor
Manufacturer : #1
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1980661

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft X departed the gate on time. We started both engines and taxied to Runway XXR. All check list and engine warmup time's completed. Takeoff roll was normal. After the gear was retracted we notice number two engine oil level was low and started blinking. Shortly after we noticed the white advisory message advising us of the oil. We discussed contacting maintenance once we passed 10,000 ft. And see what they had to say. Climbing out of 4,000 ft. We received violent bangs followed by massive yawing of the aircraft indicative of an engine failure. I immediately leveled the aircraft to get more airflow into the engine. At this point the number one engine had at least 3-5 massive compressor stalls followed by a rollback. My first officer (FO), who did an amazing job, advised ATC and started the ECAM procedures. We had numerous ECAM messages going off and of course flight attendants calling us nonstop due to the flames coming out of number one engine, rightfully so. I immediately started the APU and turned back towards ZZZ. The number two engine was still low on oil so my FO and I discussed the contingency for that situation as well. Once we leveled off, I started a slow decent with reduced power on number one. The compressor stalls stopped but the engine was running hot. Once we had every checklist completed and all the proper company stations called. We decided to make an overweight landing. On final we saw all the Crash Fire Rescue (CFR) was waiting for us. After completing all normal checklists, Fight Attendant briefing, passenger announcements and abnormal checklists, we felt it was time to land. We had plenty of fuel and a lot of time to make sure things were done correctly. Our aircraft weight was around 149,900 pounds when we touched down. We stopped the aircraft in the runway and called flight attended stations. CFR looked us over and noticed our brakes were pretty hot, but did not notice any engine damage or fire on the affected side. CFR told us to shut down both engines on the runway. After inspection I called over the PA to remain seated. We started number two and taxied to the gate with CFR in tow. No further actions were taken.

Synopsis

A320 Captain reported compressor stalls, engine rollback, and failure of the number one engine while climbing through 4,000 ft. just after departure. The flight crew requested vectors back to the field and accomplished a safe landing.

ACN: 1980339 (440 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1980339

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X when taking off from ZZZ on XXR (ZZZZZ), we had a ENG 1 STALL around 3,000 ft. Originally in the climb, we were monitoring ENG 2 because the oil level started flashing and we received an advisory. Then once we began to level and accelerate, ENG 1 went into a compressor stall and starting surging with multiple loud bangs. The FAs (Flight Attendants) called reporting flames coming from ENG 1. We immediately [requested priority handling] and returned to ZZZ on Runway XXL. We landed overweight at roughly 149,000lbs without any further event.

Synopsis

First Officer reported an engine stall during initial climb. Flight crew performed an air turn back and landed at departure airport.

ACN: 1980158 (441 of 2908)

Time / Day

Date : 202303

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 125
Experience.Flight Crew.Last 90 Days : 82
Experience.Flight Crew.Type : 125
ASRS Report Number.Accession Number : 1980158

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the takeoff roll, received ECAM ENG 1 COMPRESSOR VANE. Takeoff was rejected at approximately 80 kts. The Captain performed the reject, I notified ATC, and we slowed to a safe taxi speed. The maximum brake temperature noted was about 250 degrees C. Maintenance was notified and the aircraft returned to the Gate and deplaned passengers uneventfully.

Synopsis

A320 flight crew reported a rejected takeoff after receiving ENG 1 COMPRESSOR VANE ECAM.

ACN: 1979647 (442 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1979647
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1979652
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On our approach to the ILS for Runway XX, Approach Control had us on a vector to intercept the course that also left us higher than normal. We were cleared to intercept the course and once established cleared the ILS for XX. We were hustling down to 3,000 ft. and trying to slow and configure and realized we hadn't armed the approach (GS) function and did just as the aircraft was leveling and the GS was below us thinking it would arm. We configured and had a large ground speed mini factor with turbulence that made it difficult to get fully configured. Once we were we completed the before landing checklist and then realized the AP had not captured the GS and we were below glide slope and descending. So we arrested the decent and then executed a go around. On the go we got an altitude and heading from tower and during the clean up stage got a master warning for a Vfe exceeded as the flaps were in retraction from flaps one to zero even though the barber pole had already jump above our current airspeed as we selected them up before our airspeed could enter the barber pole territory. Cleaned up and completed a successful approach to ILS XX with moderate turbulence and a ground speed mini that was close to thirty knots over Vapp. Called Maintenance Control once at the gate and told them the situation with the flaps and they said not to write it up since our airspeed never showed us in the actual barber pole. [Event caused by] getting a slam dunk approach in IMC with turbulence and wind shear made it difficult to get get established and stable. With those factors and the AP not capturing the GS and not recognizing that right away while being focused on getting slowed and configured more than we should've we comprised the approach. We fixated on one or two issues and forgot our scan for a few moments that lead to the go around. ATC not giving the usual VFR slam dunk approach with IMC conditions with active weather. More Flight Mode Annunciator (FMA) awareness in that task saturated situation and awareness of the approach vs where the plane is and what it's doing by both the Pilot Monitoring (PM) and Pilot Flying (PF).

Narrative: 2

Cleared by ATC to join the XX localizer at ZZZ during a "slam dunk" decent clearance. While descending on XX LOC ATC clears our flight for the approach during which time we were above the vertical profile. As Pilot Flying (PF) I delayed arming approach mode while trying to descend to join the vertical profile. At GS intercept altitude the approach mode was armed but did not capture. The aircraft began a decent through the GS and I did not realize the autopilot had not captured the GS. Once the First Officer (FO) and I realized the aircraft had not captured the GS and we were still descending below GS path (VMC) a go-around was executed. Tower at ZZZ gave us radar vectors for a 2nd approach that was completed successfully to an uneventful landing. [The event was caused by a] short night evening before event followed by long multi-leg (4) work day sequence construction.

Synopsis

Air carrier flight crew reported receiving a "slam dunk" approach clearance that resulted in a go around. Tower provided vectors for a second approach which landed safely.

ACN: 1979585 (443 of 2908)

Time / Day

Date : 202303

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Work Environment Factor : Glare
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 17278
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4341
ASRS Report Number.Accession Number : 1979585

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Airport

Narrative: 1

When parked at gate in ZZZ, once the jet bridge is pulled back the lights for the new baggage facility are just blinding. They aren't aimed directly at the flight deck, but may as well be. They are blinding. This is true for the lights on all sides of the facility. Also, after pushback to spot, there is temporary lighting for the construction with the same issue. That lighting is on the north side of the construction zone, and aimed south. In actuality, it's aimed right at the flight deck because of the broad spread and we are pointed almost right at it. I had to use the sunshade to safely taxi away, and that really isn't enough. Night vision is ruined for quite some time.

Synopsis

A320 Captain reported lights from a baggage facility and airport construction obstruct the flight crew's vision at night.

ACN: 1979141 (444 of 2908)

Time / Day

Date : 202303
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Microphone
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1979141
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Check Pilot
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1979142
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Electronic Device
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

As we were getting ready for the approach and land in ZZZ, FL270, the cabin crew called us via interphone, notifying us about a [situation] in the mid-cabin. [The] Captain advised me about the event. The Captain immediately told me and communicated with ATC that we [were requesting priority handling] to ZZZ1 Airport. We both donned our masks and executed the [appropriate] descent procedure. As we were descending, ATC gave us a new instruction. Level off at FL250. We set that on the FCU. We had a traffic advisory, and ATC notified us about it. We dialed the VS on the FCU less than 1500 feet per minute to successfully avoid an RA. After crossing the traffic, we selected a lower altitude on the FCU (10000); as we were descending, both cockpit crews were having issues transmitting to ATC with our masks on. So I removed my mask and used the hand mike to communicate and reaffirm the [situation]. We requested the weather and runway in use at ZZZ1. The weather was provided for Runway XX. As we loaded the MCDU with all the details, we ran the [appropriate] checklist procedure. We loaded the flight [company software] with the landing details and proceeded with the approach checklist. On the final approach to Runway XX, we discussed the need to evacuate and confirmed that no actions were required. We did this twice. We review out loud the next course of action. Once on the ground and the aircraft stopped, I announced via the PA to the flight attendants at stations. After discussing with [the] Captain the state of the aircraft, the Captain communicated with the passengers the next course of action. The Captain informed the passengers we would taxi the airplane to the gate assigned. I announce three times passengers remain seated. We spoke with the fire chief and proceeded to taxi the aircraft. We Completed all checklists. Once at the gate and the parking checklist completed, the Captain gave the order to disembark the passengers as the residual smell was still present in the cabin. Two fire extinguishers bottles were used. One PBE (protective breathing equipment) and some passengers helped to combat the fire. A retired firefighter, deadheading crew members and Flight Attendant all went above and beyond.

Narrative: 2

I was conducting OE (Operational Evaluation) on a Captain upgrade and occupying the right seat, While in level flight at FL 270, I was the PM (pilot Monitoring), First Officer PF (Pilot Flying). Also onboard we had a Check FA (Flight Attendant) and two FAs receiving OE in the cabin, 4 jumpseaters onboard seated in the cabin not too far from the source of the fire. Approximately XA32 we received a call from the cabin, advising us of the [situation] with smoke coming out of an overhead bin mid cabin. She hung up, at the same time we got a frequency change from ATC that went unacknowledged, [FA] called back and confirmed an uncontrolled [situation] mid cabin from an overhead bin. As a cockpit crew we donned 02 masks and I attempted to respond to ATC, but the mask mic was transmitting poorly. After two attempts from myself and one from [PF], I took my mask back off and confirmed with ATC we had a [situation] and were diverting to ZZZ1. I instructed PF to keep his mask on during the descent while I worked the radios. We operated in this fashion until below 10,000 feet. Also during this time, Over the PA I briefed passengers on the diversion. FA called me at some point around 10,000 and said the [situation] was out and the cabin was prepared for landing. We completed the [appropriate], checklists for approach and landing, [PF] completed landing data for ZZZ1 on [Company system], and he confirmed again with FAs prior to landing that the [situation was contained]. We briefed ourselves and the FAs that we would stop and speak to the fire chief after landing, and that we did not anticipate evacuation. After landing, [PF] called FAs to stations, I spoke to the [ground personnel] and confirmed with them the [situation was contained] and established a plan for taxi to a gate. [PF], asked FAs to remain seated and we advised passengers that we were taxing to a gate to deplane. Once at the gate, I coordinated the deplaning as quickly as possible specifically due to the strong odor remaining in the cabin. Two of passengers were treated by paramedics on the aircraft and once all passengers had deplaned I made a gate announcement asking anyone who felt the need for medical care to please come forward and see paramedics. Several more passengers were cared for gate side, eventually the FAs were also taken to the hospital for evaluation. I was advised by jumpseater, and Lead FA that they along with a nearby passenger assisted in [controlling] the [electronic device] that was inside of a backpack. Passenger burns his hands while pulling the wet [electronic device] from the backpack. The device was placed in water per procedure and the fire source was isolated. Two halon extinguishers were used. A PBE was opened, but not used as the fire was already extinguished.

Synopsis

Air carrier flight crew reported a passenger's electronic device experienced a thermal runaway during cruise. The flight crew requested and was provided priority handling to a diversion airport where the flight landed safely.

ACN: 1978245 (445 of 2908)

Time / Day

Date : 202302

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 1415
Experience.Flight Crew.Last 90 Days : 172
Experience.Flight Crew.Type : 1415
ASRS Report Number.Accession Number : 1978245
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Dangerous goods final summary Regarding Pilot Bulletin Final dangerous goods acknowledgement. There are 2 concerns I have with the acknowledgement. We are supposed to send MISC code. After it is sent, there is no indication the acknowledgement was received. If it's something I have to acknowledge as a Captain, I feel the ACARS should send a message back letting me know the confirmation was not lost in cyberspace. The final dangerous goods I received doesnâ€™t really show that itâ€™s the "Final". There is an example picture in the Bulletin that states "Final Dangerous goods summary" and states "Final dangerous goods". The old method had "final DG" at the top. Making it clear it was the final and not the planned.

Synopsis

Air carrier Captain reported the new flight crew procedure of acknowledgment of the receipt of the final dangerous goods documentation is unnecessary.

ACN: 1977642 (446 of 2908)

Time / Day

Date : 202302
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : Balloon
Crew Size.Number Of Crew : 0
Flight Plan : None
Flight Phase : Cruise

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 645
Experience.Flight Crew.Last 90 Days : 164
Experience.Flight Crew.Type : 645
ASRS Report Number.Accession Number : 1977642

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 259
Experience.Flight Crew.Last 90 Days : 197
Experience.Flight Crew.Type : 259
ASRS Report Number.Accession Number : 1978246

Events

Anomaly.Conflict : NMAC
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Object
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

At FL340, between fix ZZZZZ and ZZZZZ1 westbound, I saw a large white balloon with a golf cart sized package tethered about 100 feet below. The balloon went directly over the right side of the aircraft above at about 500 feet away. Winds were northwesterly at 25 kits at the time.

Narrative: 2

While operating Flight XXXX around XA:40 time at FL 360 between ZZZ and ZZZZZ1 intersections (approximately 20 miles East of ZZZZZ1 ) on Victor Airway, we observed a tethered white balloon about a 1000 ft. or less from our altitude. It had a gray object attached to it. The balloon was traveling North East in the direction of the wind. We advised ATC, Dispatch, and Chief Pilot.

Synopsis

Air carrier flight crew reported an NMAC event during cruise with a large balloon about 100 feet below and about 500 feet horizontally. The flight crew reported the balloon and continued the flight without issue.

ACN: 1975024 (447 of 2908)

Time / Day

Date : 202302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY
Altitude.MSL.Single Value : 1700

Environment

Flight Conditions : VMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class B : JFK

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : A350
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : JFK

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 26000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 16000
ASRS Report Number.Accession Number : 1975024
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Vectored approach maintain 2,000 ft. Cleared ILS Runway 22L. Maintain 180 kt. to ZALPO. On a late Tower handoff we were told we're â€œcleared to land Runway 22L.â€ Then, â€œYou're following a heavy A350.â€ At 1,700 ft. Tower asked if we saw the aircraft we're following? Dealing with trying to slow as much as possible and feeling the effects of the top of the heavy jet wake turbulence, the aircraft was blending into ground lights. I said I see a light ahead. The controller said not good enough. I said I clearly see the airport and runways. Controller said he needed to know if I saw the aircraft approaching 22 Right. I stated, â€œI don't see an aircraft for 22 Right BUT I DO HAVE THE AIRCRAFT AHEAD FOR 22 LEFT.â€ Nothing further was said by ATC. TCAS showed the aircraft about 5 miles ahead and very short final. We slowed to minimum speed and continued without incident. After landing we were asked to call JFK Tower on a phone line. I spoke at length with the Supervisor and cleared up the questions together. We were â€œcleared for the ILS,â€ not for the Visual. The first time we knew we were following a heavy jet was first checking with JFK Tower. We were â€œcleared to landâ€ by JFK Tower. That clearance didn't change.

Synopsis

A320 Captain reported experiencing wake turbulence and having a communication breakdown with ATC while on final approach to the runway, in regards to whether the traffic landing on the other, parallel runway was in sight.

ACN: 1974639 (448 of 2908)

Time / Day

Date : 202302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1974639
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was scheduled to operate Aircraft X ZZZ â€“ ZZZ1 on Day 0. The aircraft was approximately 2 hours late inbound from ZZZ2. The delay was caused by a maintenance issue. While taxiing out in ZZZ2 the crew encountered an ECAM for "F/CTL SPLR FAULT.â€ They returned to the gate and Maintenance Control performed a reset. They then departed for ZZZ. While enroute, the same fault reappeared. They wrote up the spoiler fault on arrival to ZZZ and Maintenance Control came out to the aircraft. They performed a reset and signed off the logbook. We pushed back from the gate. During the first engine start, the ECAM reappeared: F/CTL SPLR FAULT. After completing the non-normal procedures for this ECAM, we consulted the Computer Reset Table and followed the procedure to reset the spoiler fault. This reset was unsuccessful. The reset procedure specifically prohibits more than one reset attempt â€“ so, after advising the flight attendants and customers, we returned to the gate. The same two maintenance control personnel returned to the aircraft. They advised me that Maintenance Control directed them to apply MEL 27-XX-XXX Spoiler Surfaces. This MEL allowed us to depart with one pair of symmetrical surfaces inoperative. They deactivated the inboard spoilers on each wing. The two technicians handed me the logbook and departed the aircraft. Several minutes later they returned and pulled and collared the circuit breaker for SEC 3. When I asked about this procedure they confirmed it was part of the task list to apply MEL 27-XX-XXX. They finished their work and we pushed back from the gate. After starting both engines I reviewed the MEL again. It advises that I should see the ECAM â€œSpoiler Fault.â€ We did not have that ECAM displayed. I also examined the flight control system page and noticed that four of our spoilers were inoperative â€“ not two as prescribed in the MEL. I also performed a flight control check and verified that four of our spoilers were not deploying. I could not reconcile the MEL guidance with what I was seeing on the flight deck. I then called Maintenance Control but could not get through. I called my Dispatcher and she was able to connect me to Maintenance Control. I described what I was observing and Maintenance Control told me â€œthat doesn't sound right.â€ He advised us to return to gate. I advised the crew and customers and we returned to the gate again. We were approaching the duty time limit so we deplaned the customers. The same two technicians returned to the aircraft. I was in the jet bridge talking with my crew when, after about 5 minutes, the technicians informed the gate agents that they were done and the aircraft was â€œgood to go.â€ They performed an incomplete Securing Checklist, shut off the APU with customers still onboard, and departed without telling me anything they had done. The flight attendants and most of the customers were not comfortable returning to the same aircraft. I asked the gate agents to call the 2 technicians back out to the aircraft so they could explain to me what they had done to make the aircraft airworthy again. They explained to me that Maintenance Control had sent them the wrong task list and all they had to do was reset the circuit breaker for SEC 3. I confirmed with them that the flight control computer had been deactivated in error. After the second gate return, myself and First Officer exceeded our duty limit for the day and were removed from the flight. After explaining to my crew and most of the customers at the gate what had happened, we were able to re-board them and depart with two new pilots.

Synopsis

A320 Captain reported encountering an ECAM spoiler fault during taxi and returned to the gate to resolve the problem. Then, Maintenance Control used the incorrect MEL procedures the first time around and the flight crew was required to return to the gate again. After returning to the aircraft a second time, Maintenance Control was able to properly release the aircraft but by that time, the flight crew ran out of duty time and had to be replaced with another crew.

ACN: 1974630 (449 of 2908)

Time / Day

Date : 202302
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29400

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1974630
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Upon arrival at the aircraft 65 minutes before departure time, we noticed that Pack 1 had just been placed on MEL due to recurring Pack 1 Regulator faults. While allowed per MEL, I do not like to fly over water on one pack, especially at night, and even more so when it's the first flight on a single pack. So I contacted Operations Control via Dispatch to ask for a different aircraft. Operations Control said none were available. I also emailed the Duty Chief Pilot for assistance but they never got back to me before departure time. I considered refusing the aircraft, but the FO (First Officer) and I chose not to. We planned to monitor the pressurization system more closely, knowing it was degraded. We boarded and departed on time. Initial climb-out was uneventful. Passing 10,000 ft., we took a close look at the pressurization parameters and all looked normal. We continued to climb to FL290. We were busy in the climb conducting the numerous procedures required for Class II operations. After a few minutes at FL290 we were cleared to climb to FL330. As we initiated the climb, the Lower ECAM switched to the PRESS page and displayed the Cabin Altitude Advisory, flashing at 8,800 ft. and climbing. I pushed the V/S knob to level off at approximately FL294. It appeared that with Pack 1 OFF per MEL, our one remaining pack was "weak" and not able to fully pressurize the aircraft. We noted the outflow valve was closed. No ECAM cautions or warnings were present. Since the cabin altitude continued to climb even while level at FL294, I directed the FO to request an immediate descent to 10,000 ft. ZZZ Center cleared us right away to descend and maintain 10,000 ft. I selected 10,000 ft. in the FCU (Fuel Control Unit) and began an open descent. At this point the cabin altitude indicated above 9,550 ft. and turned red, so I proactively announced â€œemergency descentâ€ even though the cabin altitude was still below 10,000 ft. The FO read the QRC checklist and I began a descent with autopilot off and spoilers fully extended. At the same time I stated out loud, "We need to go back to ZZZ." The FO advised ATC of our descent and the desire to return to ZZZ. We were additionally cleared to turn right direct ZZZ. Once the descent was established, we began the right turn towards ZZZ. I noticed no traffic at all within 20 NM of the aircraft during the descent and turn back via TCAS on ND (Navigation Display), and out the windows. I selected Terrain on ND to ensure we were over water as expected. It was a pitch black night with no horizon or visual cues. As we descended, the cabin altitude continued to climb above 9,550 ft. but at a slower rate. Then approximately halfway through the descent, the Master Warning sounded as the cabin altitude reached 10,000 ft. Shortly thereafter the cabin began to descend as the aircraft got lower. I recall the cabin was only 100 â€“ 200 ft. above 10,000 ft. for approximately 20 seconds. We did not don oxygen masks when the cabin altitude reached 10,000 ft. due to the high workload of hand-flying the descent on instruments and the transient nature of the excessive cabin altitude. We felt the priority was on maintaining safe control of the aircraft. A320 FCOM (Flight Crew Operations Manual) 1 paragraph 5.1.2 states, â€œthe pilot flying will be tasked with verifying the non-normal condition and confirming proceduresâ€¦ (but) it should not distract the pilot flying from his primary duty of safely flying the aircraft.â€ However upon reflection, if the situation was progressing to the point where I thought declaring â€œemergency descentâ€ was warranted, putting on our oxygen masks before beginning the descent would have been the better course of action. Upon leveling at 10,000 ft. we slowed to 250 kt., re-engaged the autopilot, verified we were navigating direct to ZZZ as cleared, and assessed the situation. The cabin altitude was below 10,000 ft. and cabin vertical speed was at -250 FPM, descending. In fact as we transited back at 10,000 ft., the cabin continued to descend normally and reached field elevation before we landed. We heard no unusual sounds indicating a leak of any kind. I next called the cabin to advise them of the situation and to check in on the cabin condition. They said everything was okay and the oxygen masks did not deploy. I told the F1 we were returning to ZZZ, landing in about 25 minutes. I said the landing would be normal but CFR (Crash Fire Rescue) may meet the aircraft. Next, we advised Dispatch via ACARS of our descent and plan to return to ZZZ. We initially set up the FMS for a return to ZZZ and ran the Overweight Landing Checklist. Planned landing weight was 154,000 lb. I then asked Dispatch if they preferred we go to ZZZ or ZZZ1, in case one was better than the other for our situation. Dispatch told us Operations Control preferred we go to ZZZ1 to facilitate an easier tail swap. We coordinated with ZZZ Center to divert to ZZZ1 and we reprogrammed the FMS again for ZZZ1. I continued to fly the aircraft and talk with ATC while the FO reprogrammed the FMS both times. ATC [provided priority handling] for us due to the descent, diversion, and planned overweight landing, and we concurred. I advised the flight attendants of our new plan to go to ZZZ1 and asked them to make a PA since we were in a descent below 10,000 ft. at this point and our workload was very high. The rest of the descent, approach and landing, ILS XXR ZZZ, were normal. Medium brakes were used per [the] software and max reverse per the Overweight Landing Checklist. Touchdown was in the touchdown zone at 200 FPM or less. ZZZ emergency equipment was dispatched to a location near the runway but they never approached the aircraft. We exited [Runway] XXR at [Taxiway] XX and taxied to [Taxiway] XY Gate XX normally. Brake fan use was delayed until brake temperatures reached 400°C on the left main gear, approximately 4 minutes after landing. Winds were 320 at 16 kt. for landing on [Runway] XXR. Brake temperatures never went much above 400°C and then began to cool. At the gate we wrote up the loss of pressurization and overweight landing. We debriefed Maintenance Control crew members face-to-face. We never ran the "Diversion" checklist since both ZZZ and ZZZ1 are primary and well-known cities â€“ we prioritized other necessary tasks. I provided the flight attendants all the information required for an event but I believe I may have failed to speak the words "event" directly. The flight attendants confirmed with me after the flight that it was clear to them that it would be an event/normal landing.

Synopsis

A320 Captain reported the cabin was not pressurizing properly and the flight crew opted to divert. The aircraft had a pack that was on MEL, leaving only one pack available for use on the flight.

ACN: 1974622 (450 of 2908)

Time / Day

Date : 202302
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors

Component

Aircraft Component : Cargo Compartment Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1974622
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

ZZZZ RETURN. Operating Aircraft X, from ZZZZ to ZZZ. Climbing through FL360, we received a Forward and Aft Cargo Smoke Detection Fault. We reviewed the QRC and QRH. We completed QRH XX but it was only for the Forward or Aft and not Both Cargo Compartments, and I wasn't sure but I also thought I smelled something burning. I called, Dispatch and Maintenance (via SATCOM) for some guidance. Maintenance lead me to XY, a different QRH checklist which directed us to land ASAP, and Dispatch told us to return to ZZZZ. In the heat of the moment I was wrapped around my return to ZZZZ that the remaining checklist was not completed, I believe that Maintenance misunderstood my situation and that was not the right checklist to begin with. We notified ATC and [requested priority handling] immediately. The crew was given a briefing code which later changed to lower code just prior to the final approach. After landing safely in ZZZZ, Crash Fire Rescue (CFR) conducted an inspection of the aircraft prior to taxiing and additional inspections of the cargo compartments and interior of the aircraft after parking, and no issues were found. Forward and Aft Cargo Compartment Smoke Detection fault climbing through FL360 was put into the aircraft logbook.

Synopsis

A320 Captain reported Forward and Aft Cargo Compartment Smoke Detection Faults climbing through FL360. The flight crew ran the QRH and check lists. Priority handling was requested and the flight crew performed an air turn back and precautionary landing at departure airport.