ACN: 1974307 (451 of 2908)

Time / Day

Date : 202302

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Powerplant Fuel Control Unit
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1974307
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On Day 1 hangar work load 7 aircraft and five mechanics and myself where assigned Aircraft X. At Approx. XA30 am Person A (lead) asked us to assist in closure of number 2 eng reverser/fan cowls and run #2 engine for ops check of bleed system for that motor. With all five mechanics getting aircraft ready for engine run. Everyone assisted with closure. No one noticed the red streamer hanging for the the part being pinned. Latches were closed and secured by Person B. Person A and I ran #2 eng performed operational checks. No ECAM warning or messages noted. aircraft was pulled back into hangar after engine runs due to lack of parking.

Synopsis

Technicians reported a rig pin attached to a red streamer was left installed in the #2 engine when the engine cowls and thrust reversers were closed and a post maintenance engine run was accomplished.

ACN: 1974302 (452 of 2908)

Time / Day

Date : 202302

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : FCDC #1
Aircraft Reference : X
Problem : Malfunctioning
Problem : Improperly Operated

Person : 1

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1974302
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other

Person : 2

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1989510
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other

Person : 3

Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1989517
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

A FCDC (Flight Control Data Concentrator) was robbed from Aircraft Y for out of service Aircraft X at the gate, which had a non-deferrable unserviceable FCDC in Position #1. We attempted to determine configuration through the company parts hotline who were confused. Then, ZZZ Engineering stepped in to determine effectivity as the IPC (Illustrated Parts Catalog) was extremely complex and convoluted for this part. I, Supervisor, was provided IPC references, and an Airbus FCDC interchangeability logic table, then provided clear pathway to prove effectivity for a Pre-Mod XXXXX A319 aircraft. We were assured it was effective via these references and guiding us through them, Avionics and I. The avionics technicians were fine with that and installed the deemed effective FCDC into aircraft and it tested fine with no faults. I signed off the log page and the aircraft was released and flew away to ZZZ1. Engineering came back and said they had dug through induction paperwork and found that Aircraft X, that we just signed off on, was actually a Post-Mod XXXXX aircraft, and the new logic table showed the part to be not effective and we would now have an inspection situation on our hands. I immediately contacted Maintenance Control and let them know the aircraft was flying with a non-effective part and they took it from there. I signed off and cleared the log page as a Supervisor with the wrong IPC and logic data provided to me by Engineering and wanted it on the record. The technicians were notified as they did the installation and parts rob. While the part functioned correctly, it was only for a pre-mod aircraft, not a post-mod aircraft like Aircraft X. I feel the extremely complex and convoluted IPC confused the technicians, myself, and ZZZ Engineering, and would need to be simplified for components like these. Aircraft will be required to replace inspection with correct, effective part as researched by Engineering, and shipped to assigned station. Engineering is following up with Maintenance Control to figure out best course of action.

Narrative: 2

Aircraft X came in with a Flight Control Data Concentrator (FCDC) # 1 Fail Message. I looked at the Illustrated Parts Catalog (IPC) and could not verify that the part number that was going to be robbed was the correct part number. Called the Company parts support line number, and after a half hour without finding anything out with the part number, I turned it over to my Supervisor. They were on the phone for 1 hour with parts and they came to agreement that it was the correct part number and told us to install it. We installed the part and tested per the Maintenance Manual (MM). All tests passed.

Narrative: 3

We were told we were going to rob the Flight Control Data Concentrator (FCDC) from an aircraft at the hangar which had the wrong part number. (Not the correct part). We were told to take that part back to its original aircraft and rob the other FCDC which was also the wrong part. We spent 30 minutes on the phone with support, which was no help. The Supervisor spent 1 hour on phone with Parts and they verified the part's effectiveness to Illustrated Parts Catalog (IPC) item per guidance. We were told to install that part. We then installed the part reference Aircraft Maintenance Manual (AMM). All Tests Passed.

Synopsis

A320 Maintenance Supervisor and two technicians reported signing off a log page and learning that the incorrect and ineffective part was used after the aircraft was released.

ACN: 1971383 (453 of 2908)

Time / Day

Date : 202302
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Component : 2

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X
Problem : Improperly Operated

Component : 3

Aircraft Component : Minimum Equipment List (MEL)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1971383
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X. We arrived at the aircraft for our departure from ZZZ when we began working through the multiple MELs. MEL 21-XX-XX-X was discussed and we could not locate the logbook entry required before each flight. Maintenance was notified and Contract Maintenance performed the test and logbook entry. We then had a Blower Fault ECAM which was prohibited per the MEL. Contract Maintenance came back to the airplane and the test was performed again clearing the Blower Fault. We had failed to notice that per MEL 21-XX-XX-X there was a note stating MEL 21-XX-XY must also be applied. This MEL was not on our paperwork. We arrived in ZZZZ and had coordinated with Maintenance Control to have Contract Maintenance there to perform the test. We received the Blower Fault with the test and Maintenance began troubleshooting the issue. Circuit Breaker AE02 was pulled and collared and the Mechanic stated this was not appropriate for MEL 21-XX-XX-X. The Mechanic worked with Maintenance and cleared the Blower Fault, pushed the Circuit Breaker in, and made a logbook entry per the MEL. While we were flying back to ZZZ we began researching the history of this further. There had been multiple transfers from MEL 21-XX-XX-Y to MEL 21-XX-XX-Z and finally the night before our flight to MEL 21-XX-XX-X. We believed this is where the Circuit Breaker issue was probably missed. As we dug into this we also noticed the note to apply MEL 21-XX-XY (which was nowhere to be found on the paperwork or the logbook). We notified Dispatch and Maintenance of needing to speak to a Maintenance Supervisor and the Chief Pilot on Duty upon our arrival. The Captain made a phone call to the on call Chief Pilot and Maintenance to bring the issues to their attention. Contract Maintenance met us on the aircraft where we handed the aircraft over to them.

Synopsis

A320 First Officer reported numerous delays during the day was caused by MEL non compliance and confusion over procedures. The aircraft was turned over to Maintenance for action.

ACN: 1969775 (454 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors

Component

Aircraft Component : FADEC / TCC
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1969775
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Some background information â€“ we showed up at the aircraft at XA:45 local time. The thrust reverser on the #2 engine was under MEL. During the engine #2 start we had an ignition fault that aborted the start. Contacted Dispatch and Maintenance before attempting a second start. The second start was successful. Then we had a #1 FD situation that also required us to contact Maintenance. Once that resolved we had to amend our release due to our fuel being below our planned minimum takeoff fuel. At that point of the flight, my First Officer and I were ready for anything to happen next. Due to the wet runway, we were going to do a TOGA takeoff with ZZZZ as a diversion field in case of problems. Around 800 ft. we got an ECAM indicating an exceedance engine #1 EGT of 650 [degrees] with a fuel flow 800 lb. over what engine #2 was showing. The thrust was decreased for the climb, bringing the EGT just below the max limit. Within 30 seconds, the EGT and the fuel flow started increasing again past the max limit. Being out of the overcast weather and still in the valley, we asked for a heading and leveled off at 8000 ft. and slowed to keep the EGT below the limit. Running the QRH and watching the way the EGT was moving, we realized that we had lost our ability to climb without running the risk of having to shut down the engine. Contacted Dispatch, explaining what was happening. We all agreed to make a precautionary landing in ZZZZ. Due to our reduced power settings we were able to keep EGT below the limit and therefore, stay out of the QRH. While flying towards ZZZZ we received a message from Dispatch asking us if we had declared an emergency. At this time I had not declared yet. ZZZZ Tower started acting as if we had declared, asking us for SOB and fuel â€“ they even conducted a runway inspection after we landed! I got busy reviewing our fuel and weight situation and did not get a chance to readdress the need to declare. Still being below 10000 ft. at night with possibly an engine we may have to shut down, I decided that an overweight landing was safer than holding to burn fuel. We ran the QRH for an overweight landing and briefed the inflight crew members, as well as the passengers. Shortly after touchdown we had a #1 Engine Compressor Vane Fault message indicating a possible FADEC (Full Authority Digital Electronic Control) failure. Taxied to the gate at ZZZZ using power from #2 engine as per QRH. Aircraft was put out of service upon arrival.

Synopsis

A320 Captain reported that on climb-out, there was a temperature exceedance in the engine #1 EGT with a fuel flow of 800 lb. over what engine #2 was showing. Upon landing, a message showed a possible FADEC error.

ACN: 1969673 (455 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Landing
Flight Phase : Cruise

Component : 1

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : INS / IRS / IRU
Manufacturer : #3
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : INS / IRS / IRU
Manufacturer : #2
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1969673
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1969687
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

While in cruise flight from ZZZ1 to ZZZ, FMGC 1 had a partial failure. The system would not show information pertaining to the time or predictive details of the flight. The Captain was the pilot monitoring in this case, so the Captain gave me both the flight controls and the radios. The Captain proceeded to do both the short and long resets of the FMGC 1. The reset of FMGC 1 was unsuccessful. The FMGC seemed to reset itself several times in flight and almost come back to functioning as normal, but eventually stayed in the partially failed condition. The Captain then wrote up the FMGC and sent a message to Maintenance about its condition. While reviewing the procedure we revived an IR (Inertial Reference) 3 position error message on the FMGC 1. Then we determined that the IR position was drifting a bit but was within acceptable limits. Shortly afterward we received a similar error about IR 2. We made the same determination about this system that it was within limits. We talked about possible issues with navigation and how we would handle it if they arose. We continued to watch the systems and brief the approach into ZZZ for [Runway] XXR. We determined that we would only use AP2 (Autopilot) on the approach and that we would continue to watch the systems for any other errors. No other errors seemed to appear on the descent and the first half of the approach. As we reached approximately 500 ft. on the approach, I disconnected the AP and started to hand-fly the aircraft. At this time the Captain noted that the speed bug had vanished. I continued to fly and noticed that the speed was beginning to drop off. I then noticed that the GS was beginning to show that we were low. I pitched the nose up to correct but the speed continued to drop. At that moment the GS warning came on, the Captain said to kick off the AT (Autothrottles), and I disconnected the AT and added power. After the descent was stabilized, I reestablished visually on the PAPI and landed the aircraft without further incident. Upon touchdown and rollout, the TERRAIN TERRAIN PULL UP EGPWS went off. Unsure what had caused this at the moment, I attributed it to the issue with the FMGC and we continued with the landing. After we landed, we were confused as to what happened, but decided to investigate it when we got to the gate. After parking at the gate and shutting down the aircraft, we went back over the events of the flight in an effort to determine what had happened. We then re-read the FMGC failure procedure and noted that the last page had a note that the AT would not work once the AP was turned off. We took a moment to think about how we could have missed this, noting that several other messages had popped up during the procedure, and the system had come back and failed several times during the flight. We made sure the FMGC was noted in the logbook and informed Maintenance before departing the plane. The fact that there had been several issues that popped up during the flight led to checklists and procedures to become interrupted. In the future I will be sure to get out my QRH and look at the procedure also, if the situation allows for it. The procedure was a very long one. It is very easy to become distracted by multiple situations accumulating. If I am ever in a similar situation, while in cruise flight where the task saturation is lowered, I will also get out my QRH and follow along with what is happening.

Narrative: 2

While in cruise flight from ZZZ1 to ZZZ, FMGC 1 had a partial failure. The system would not show information pertaining to the time or predictive details of the flight. I was the pilot monitoring in this case, so I assigned the FO (First Officer) to work both the flight controls and the radios. I reviewed the â€œSingle Latch with AP (Autopilot) and FD Still Available in Managed Modes on Both Sidesâ€ QRH XX â€“ XX but in the process the FMGC 1 came back online, although temporary. It then failed again and I proceeded to do both the short and long resets of the FMGC 1. The reset of FMGC 1 was unsuccessful. The FMGC seemed to reset itself several times in flight, and almost come back to functioning as normal, but eventually stayed in the partially failed condition. I then wrote up the FMGC and sent a message to Maintenance about its condition. While reviewing the procedure we received an IR (Inertial Reference) 3 position error message on the FMGC 1. We determined that the IR position was drifting but was within acceptable limits. Shortly after, we received a similar error about IR 2. We made the same determination about this system that it was within limits. We backed up all navigation conventionally using manually tuned VORs and advised ATC that if we were off course, to advise us. We talked about possible issues with navigation and how we would handle it if they arose. We continued to watch the systems and brief the approach into ZZZ for [Runway] XXR. We determined that we would only use AP2 on the approach and that we would continue to watch the systems for any other errors. No other errors seemed to appear on the descent and the first half of the approach. As we reached approximately 500 ft. on the approach, the FO disconnected the AP and started to hand-fly the aircraft. At this time I noted that my speed bug had vanished. The airspeed was decreasing and the FO was getting low on the glideslope. The AT (Autothrottles) were not being responsive so I asked the FO to disconnect the autothrust and add power. The descent was stabilized and the FO landed normally and on speed without AT. After we landed, we were confused as to what happened, but decided to investigate it when we got to the gate. After parking at the gate and shutting down the aircraft, we went back over the events of the flight in an effort to determine what had happened. We then re-read the FMGC failure procedure and noted that the last page had a note that the AT would not work once the AP was turned off. We took a moment to think about how we could have missed this, noting that several other messages had popped up during the procedure, and the system had come back and failed several times during the flight. We made sure the FMGC was noted in the logbook and informed Maintenance before departing the plane. FMGC failures do not occur often and [I] have never been trained of this particular event in the simulator. When the FMGC acted normally, albeit briefly, I suspect I stopped reviewing the entire procedure.

Synopsis

A320 flight crew reported a FMGC 1 malfunction in flight that also affected the IR. The flight crew followed the FMGC failure procedure, which led to the autothrottles being unavailable during the landing.

ACN: 1968006 (456 of 2908)

Time / Day

Date : 202211

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger

Component

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : ELA

Person

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Inspector
Function.Maintenance : Quality Assurance / Audit
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1968006
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Receiving Inspection on Elevator/Aileron (ELAC Computer) *AD2018-0007R1 2793-01014*, Repaired by Vendor on order #XXXXXXX. Rejected, placed in Quarantine discrepancy area due to Vendor XXXX-X has incorrect part# as per ECRA XXXX-XXXXX Rev. X compliance, ref. Labeling instructions Page X and Page YY of ECRA (Also see page 11). Unit arrived with Labeling attached, but MFG P/N was not crossed out (as required per ECRA) from previous Labeling. Sent out Company P/N per App transaction and OAL Original order sheet showing MFG P/N 3945129100U1, received MFG P/N 3945129100 per block-X same MFG S/N for both (Means it is the same Unit). When you research MFG P/N 3945129100 in App transaction it says do not buy, Suffix after the part number indicates Software was downloaded specific to Company. The Vendor had no information stating the L101 download software was completed, but the tear down sheet did. It is an AD note; regardless I want 100% compliance without any doubt. Unit stayed in the Discrepancy area for approximately 5 weeks without any resolution until it became Hot AOG, email correspondence sent through company emails to Vendor without any resolution and then to another Inspector bypassing the resolution process trying to find another Inspector to make it Serviceable telling the Inspector to make Serviceable by email requesting Unit to be sent to Shop. During this time Order XXXXXXX was changed reflecting the MFG P/N removing Company P/N !! (Obviously to push the part through). This is ongoing problem at ZZZ removing parts out of quarantine without resolution, Avionics Shop Analyst, Business Services is constantly requesting units to bent sent to the shop before resolution is answered; Completed. Eventually after the part was again dumped off to me in my persistence, but part was not finalized through (Inspection) and was eventually completed by Avionics Shop. This ELAC Unit has chronic history of failure and failed again in service according [to] App history transaction and is back at the Vendor again.

Synopsis

Inspector reported non-compliance with procedures concerning quarantine of an ELAC following vendor repair. The unit was released from quarantine, subsequently installed and then failed, resulting in returning it to the vendor.

ACN: 1967936 (457 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors

Component : 1

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1967936
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After a normal landing on XL we proceeded to exit the runway via high speed on the taxiway. As I used the tiller to turn the aircraft, the aircraft respond as expected followed quickly by no response at all. I increased the tiller input but there was no change in the aircraft direction. The aircraft was traveling towards the grassy area between the runway and taxiway and I was able to use differential braking to continue the aircraft onto the taxiway center line and exit the runway. We were able to clear the runway and brought the aircraft to a stop. We advised ATC that we were experiencing a nose wheel steering issue and were unable to continue our taxi. We then executed our non-normal procedures and completed the ECAM actions and the QRH. After making a PA to the passengers advising them of our situation, I called Dispatch to be connected to Maintenance. While holding on the phone with Maintenance, First Officer contacted ZZZ Ops and advised them of our situation. I told Maintenance what had occurred and the associated ECAM message that was displayed. Maintenance instructed us to recycle the nose wheel steering switch and when completed, our nose wheel steering was restored. The ECAM message was no longer displayed and the status page was clear. Our call with Maintenance was ended and we advised ATC that we were ready to taxi. I started our taxi slowly ensuring that the nose wheel was operating as expected and I noticed no abnormal operation. We taxied with caution and without any further issue to the gate. After blocking in and completing the parking checklist, I called Dispatch to be contacted to Maintenance. I spoke with the Controller and asked if this was something that we needed to write up since the problem was no longer present. They told me that there was no need since the message was no longer displayed. We ended our call and deplaned the aircraft. As we were walking off the aircraft, the outbound Crew was at the gate and I discussed what had occurred to the Captain taking out the aircraft. Upon reaching the hotel, I went into the Flight Crew Operations Manual (FCOM) and referenced the computer reset table. Within the section, the message we experienced is listed with a table of reset procedures. Upon referencing it, I believe that we should have made a logbook entry. I also did not realize that there is a max speed of 10 kts. when the reset has been completed. Maintenance did not advise us of any requirements for the reset, and while we did taxi with caution, I'm not certain that we momentarily exceed 10 kts. while taxing back.

Synopsis

A320 Captain reported failure to properly document a Nose Wheel Steering System failure on landing at destination airport. Following Maintenance Control directed reset procedure, Nose wheel Steering was restored and the aircraft continued to taxi to the gate. A required log book entry was not accomplished.

ACN: 1967559 (458 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9300
Experience.Flight Crew.Last 90 Days : 350
Experience.Flight Crew.Type : 3100
ASRS Report Number.Accession Number : 1967559

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When we were in cruise flying a ferry flight from ZZZ1 to ZZZ, we got an ECAM. Y HYD 2 ENG PUMP LO PRS / LO LVL. Then the RSV LO LVL. We started to do the â€œECAM actions.â€ Then, ECAM actions [were] complete. We contacted ATC Approach frequency. We explained the situation because it was a possibility that we couldn't use the NWS, nose wheel steering, after landing. The ECAM instructions were, â€œOn the approach turn on the Y ELEC pump.â€ We did and we were able to taxi out of Runway XXL. Then we proceeded to the FBO Ramp without any problem. The controller who was attending on ZZZ Approach was very precise on the instructions, very clear and helpful.

Synopsis

A320 Captain reported receiving â€œY HYD 2 ENG PUMP LO PRS/ LO LVLâ€ and â€œRSV LO LVLâ€ on the ECAM in cruise and followed the appropriate ECAM actions.

ACN: 1967176 (459 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0601-1200

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Other / Unknown
ASRS Report Number.Accession Number : 1967176
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Time Pressure
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Ground Personnel
Detector.Person : Other Person
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were loading Aircraft X to ZZZ. There was a shipment that had missed a previous flight. Since it was a medical shipment I felt it was important to load. [This shipment] does not need DG paperwork unless marked properly. There were 8 boxes and I did not notice a Type 9 corrosive symbol. I went inside to check the DG report for my flight Aircraft X and there were none, so I proceeded to load the [shipment]. I also double-checked the packages a second time to make sure. It was pouring down rain, and I just felt it missed the previous flight. All 8 boxes were 3 lb. each. I was informed by my manager the DG had 35 lb. of dry ice. Once again, every box weighted 3 lb. each. I was sent a message to meet with Person A and to bring a Shop Steward, which was Person B. The next day, I was asked by Person A, the manager, to get a Shop Steward because I had loaded a DG without proper paperwork. I stayed with my story that [the shipment] does not need paperwork, to no avail. There was no picture of the Dangerous Good provided, just my word against his. The event occurred because I felt the medical shipment might save someone's life. If I had known one box was labeled as a DG I would have done the paperwork and given it to the Captain. We tried to explain that [the shipment] does not need a DG paperwork but he was unaware of this. He also said all 8 boxes were DG and 35 lb. of dry ice was on board. The boxes once again weighed 3 lb. each. I also explained how I got back in the front bin and checked twice for a DG label, none seen. The report says I failed to look in the computer for DG paperwork, not true, I went inside and looked there were no DG reports for Aircraft X ZZZ. I did check. [That shipment] needs no paperwork. We asked if there was a photo, there was none. [I suggest] to fully inspect all boxes with a DG Label 9 Corrosive, which is dry ice.

Synopsis

A320 ramp personnel reported a communication breakdown between Ramp and Load Control regarding shipping a category of DG cargo that contained dry ice without proper documentation.

ACN: 1965349 (460 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Nose Gear Tire
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Improperly Operated

Component : 3

Aircraft Component : Company Operations Manual
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1965349
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 25
Experience.Flight Crew.Type : 2670
ASRS Report Number.Accession Number : 1965357
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Tow bar issues. The aircraft was pushed back from the gate and the parking brake was set. The Ground Personnel was clearly having issues disconnecting the tow bar from the aircraft. Visually from the flight deck it looked like the tow bar/tug was at a very sharp angle for normal tow bar removal. During engine start the Ground Crew request the Captain release the parking brake which he declined to do. She said she needed to pull the aircraft forward to release the tow bar. The Captain responded he would release the parking brake if she reinserted the Nose Wheel Steering Bypass Pin which she did, subsequently the Captain released the brakes. The aircraft broke free from the tow bar and began to roll forward. The Tug Operator yelled, "SET BRAKES! SET BRAKES!" Simultaneously the Captain set the brakes and I quickly applied the toe brakes. The aircraft had rolled forward into the tow-bar putting a small but significant scratch on the sidewall and a small gouge into the Nose Wheel Tire. Maintenance was on site quickly and there were no injuries. We successfully taxied back to the gate where the Nose Wheel Tires were replaced.

Narrative: 2

Push back Deviation. At conclusion of push back, the Ground Ramp Person was having issues removing the tow bar from the aircraft. The Ground Ramp Person requested that the parking brake be released to remove the tow bar. I stated this could not be done. The Ground Ramp Person then requested to pull the aircraft forward in an attempt to better align the tow bar to the aircraft for removal. To do this, I said that the Nose Wheel Steering would have to be disconnected before moving the aircraft forward. The Nose Wheel Steering was then disconnected and the ground ramp person requested that the Parking Brake be released. As I released the parking brake, the aircraft began to move forward. The Ground Ramp Person then yelled "set brakes, set brakes" over intercom. The Brakes were set and the Ground Ramp Person stated that the Tow Bar was now not connected to the aircraft and under the Nose Gear Tire. Maintenance Personnel arrived and relayed that a Nose Gear Tire was now damaged. I taxied the aircraft back to the gate where a maintenance inspection of the Nose Gear was performed and a Nose Tire was replaced. At the gate, I expressed my push back communication and procedures concerns to a Ground Ramp Person Supervisor. I was unable to have a debrief with the actual Ground Ramp Person involved in the incident. This incident highlights the danger of push back operations and the importance of clear and concise communication as well as 100% SOP compliance.

Synopsis

Flight crew reported a Nose Wheel Tire was damaged during pushback when the aircraft rolled into the tow bar. Maintenance replaced both Nose Wheel Tires.

ACN: 1965342 (461 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 604
Experience.Flight Crew.Last 90 Days : 131
Experience.Flight Crew.Type : 604
ASRS Report Number.Accession Number : 1965342
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 136
Experience.Flight Crew.Last 90 Days : 86
Experience.Flight Crew.Type : 136
ASRS Report Number.Accession Number : 1965355
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Other Automation
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Incorrect altitude on approach. We were on the ZZZZZ into ZZZ, on the last leg of a four day trip. Upon checking in with Approach Control, we were told to expect vectors to the ILS XXR with an approximately nine mile final. A lot of things happened in quick succession between the end of our downwind and when the controller broke us off the approach. I don't think my memory of the order of events is exactly right, but it's close, and it's to the best of my recollection. Somewhere prior to ZZZZZ on downwind we were told to maintain present speed (240) and given a descent to 3000 ft. I selected speed 240 and pulled for Open Descent to 3000 ft. Not long after that while still in a descent, passing maybe 5000 ft., we were given speed 210 and told to hustle down to 2000 ft. I clicked off the autopilot to get the full benefit of flight spoilers and we continued descent while slowing. Somewhere below about 3500 ft. approach thanked us for hurrying down and slowed us to 170 kts. I believe there was also a frequency change in here, but I'm not sure. I asked for and received flaps 2 on speed. We were given a base turn, and nearing 2000 ft. the Controller asked us to confirm we were leveling at 2400 ft. I was passing 2200 ft. still hand flying and leveled off while the FO (First Officer) said we were going to 2000 ft. as assigned. I'm now ignoring the flight director pitch command. The Controller said something like, "Low altitude no wait you look like you're good just stay at 2200" and gave us another turn to final and cleared us for the approach. I start the turn and am now ignoring both pitch and bank commands on the flight director and unsure why the FO hasn't changed the FCU. Unfortunately, the altitude horn was going off because I had leveled at 2200 ft. and the altitude window was still set to 2000 ft. I saw that the FO was confused and frustrated at the source of the horn, but I couldn't turn on the autopilot because I didn't want it to continue down to 2000 ft. and I was in a turn, so I said something like, "It's an altitude horn, cancel it." and pointed at the FCU. Not only was I too loaded up with hand flying to explain how to kill the horn and request FCU input, but upon debriefing, I learned that the FO was only partially able to hear what ATC or I was even saying over the horn. The FO canceled the horn, but by that point, I had not rolled out of my turn and was turning away from the final approach course by about 30 degrees AND had descended to about 2080 ft. while distracted, and ATC had asked like eight questions in rapid succession that the FO was too confused to answer. ATC exclaimed we were turning away from the course and said, "Low altitude alert, I'm gonna break you off" and gave us a climb to 3000 ft. and a turn. The FO input it into the FCU, and I got the autopilot on ASAP. We cleaned up from flaps 2 to flaps 1, made sure the approach was still loaded in the box, received another downwind-to-base turn and clearance for the ILS XXR, and landed normally.

Narrative: 2

Incorrect altitude on approach. We were on the ZZZZZ into ZZZ, on the last leg of a four day trip. Upon checking in with Approach Control, we were told to expect vectors to the ILS XXR with an approximately nine mile final. A lot of things happened in quick succession between the end of our downwind and when the controller broke us off the approach. I don't think my memory of the order of events is exactly right, but it's close, and it's to the best of my recollection. Somewhere prior to ZZZZZ on downwind we were told to maintain present speed (240) and given a descent to 3000 ft. I selected speed 240 and pulled for Open Descent to 3000 ft. Not long after that while still in a descent, passing maybe 5000 ft., we were given speed 210 and told to hustle down to 2000 ft. I clicked off the autopilot to get the full benefit of flight spoilers and we continued descent while slowing. Somewhere below about 3500 ft. approach thanked us for hurrying down and slowed us to 170 kts. I believe there was also a frequency change in here, but I'm not sure. I asked for and received flaps 2 on speed. We were given a base turn, and nearing 2000 ft. the Controller asked us to confirm we were leveling at 2400 ft. I was passing 2200 ft. still hand flying and leveled off while the FO (First Officer) said we were going to 2000 ft. as assigned. I'm now ignoring the flight director pitch command. The Controller said something like, "Low altitude no wait you look like you're good just stay at 2200" and gave us another turn to final and cleared us for the approach. I start the turn and am now ignoring both pitch and bank commands on the flight director and unsure why the FO hasn't changed the FCU. Unfortunately, the altitude horn was going off because I had leveled at 2200 ft. and the altitude window was still set to 2000 ft. I saw that the FO was confused and frustrated at the source of the horn, but I couldn't turn on the autopilot because I didn't want it to continue down to 2000 ft. and I was in a turn, so I said something like, "It's an altitude horn, cancel it." and pointed at the FCU. Not only was I too loaded up with hand flying to explain how to kill the horn and request FCU input, but upon debriefing, I learned that the FO was only partially able to hear what ATC or I was even saying over the horn. The FO canceled the horn, but by that point, I had not rolled out of my turn and was turning away from the final approach course by about 30 degrees AND had descended to about 2080 ft. while distracted, and ATC had asked like eight questions in rapid succession that the FO was too confused to answer. ATC exclaimed we were turning away from the course and said, "Low altitude alert, I'm gonna break you off" and gave us a climb to 3000 ft. and a turn. The FO input it into the FCU, and I got the autopilot on ASAP. We cleaned up from flaps 2 to flaps 1, made sure the approach was still loaded in the box, received another downwind-to-base turn and clearance for the ILS XXR, and landed normally.

Synopsis

A320 flight crew reported ATC descended them too low and then issued a Terrain Alert. While distracted by the Low Altitude Alert the Captain flew through the final approach course.

ACN: 1965055 (462 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1700

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1965055
Human Factors : Communication Breakdown
Human Factors : Distraction
Human Factors : Time Pressure
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On approach to ZZZ airport while receiving radar vectors we reported the field in sight. Approach Control cleared us direct ZZZZZ, and cleared for the visual approach RWYXXR. The first officer set 1,700 ft. on the FCU and called "1,700" and I responded "1,700". This was the first mistake and the root cause of this event. I should have caught the error and corrected it. But I didn't and I assume full responsibility for it. The First Officer started the descent to 1,700 ft. and we configured the aircraft to CONFIG 2/Gear down. The controlled called us with an altitude alert stating that the Minimum Vectoring Altitude in the sector was 2,500 ft. and we were at 1,700 ft. At that time we realized that we were too low and the First Officer instead of calling for a Go Around tried to pull the nose up. The autopilot disconnected and the auto thrust system went into Thrust Lock mode with the associated ECAM warnings. At that time I called for the Go Around. A Go Around was performed and we were vectored back for a full ILS approach to Runway XXR. The landing was done uneventfully this time. The First Officer remained the Pilot Flying and completed the landing.

Synopsis

A320 Captain reported a Too Low Warning was issued by ATC while on approach. A missed approach was accomplished and another approach was completed.

ACN: 1964863 (463 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 179
Experience.Flight Crew.Type : 501
ASRS Report Number.Accession Number : 1964863
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

#2 Generator failure. At cruise several minutes before the ETP the aircraft chimed with and ECAM and the right side of the aircraft went dark. There were a number of amber ECAM Messages that did not appear to be related. The #2 generator dropped off line. EPR Mode failed and we placed the engines in N1 mode as directed by the ECAM. At some point the FAs (Flight Attendants) called to say they noticed a loss of electricity and I told them to please standby as we were working the issue. We tried to reset the #2 generator it dropped off 2-3 seconds after re engagement. The Captain started the APU and it did not automatically bring its generator online. We attempted one more generator reset on #2 and on the second time It failed 2-3 seconds after re engagement the APU generator came online and began to power the right side of the aircraft. Due to the EPR failure we matched the throttle N1 settings and decided that we had maintained aircraft control. Since the FO (First Officer) side had the ACARS written up I offered to fly the aircraft so the Captain could begin coordinating with Dispatch so we transferred aircraft control. An ECAM message appeared me told us to turn off FAC 2 but I do not recall what ECAM that was. Other ECAM messages that I can recall were displayed include: AUTO FLT YAW DAMPER 2 AUTO FLT RUD TRIM 2 FAULT NAV LS TUNING DISAGREE with check BARO REF Once we had two generators online and we were confident the aircraft was in a flyable condition the Captain discussed with Maintenance and Dispatch we decided to land at ZZZ due to greater support there. We flew vectors to the ILS XXC to and uneventful landing. ARFF vehicles met us when we cleared the runway and allowed us to proceed to the gate. We were later dispatched on another aircraft to ZZZ1 2 hours later.

Synopsis

First Officer reported loss of #2 Generator in cruise. The flight crew was able to bring the generators back on line and completed a diversion and landing.

ACN: 1964857 (464 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Interphone System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Minimum Equipment List (MEL)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2189
Experience.Flight Crew.Last 90 Days : 169
Experience.Flight Crew.Type : 2189
ASRS Report Number.Accession Number : 1964857
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : MEL

Narrative: 1

This report covers two different flights. XXXX on the Day 0 from ZZZ1 to ZZZ2, and XXX on the Day 1, from ZZZ to ZZZ1. Aircraft X was involved on both flights. I was walking on to Aircraft Y to fly ZZZ1 to ZZZ2 (Day 0) when I was told there was an aircraft swap and new aircraft was Aircraft X. It was scheduled to fly to ZZZ3 and that Captain had refused it. The refusal was based on MEL XX-XX-X-X for FA (Flight Attendant) handset inoperative. After walking on to Aircraft X, I discovered the alternative comm procedures involved rear aircraft comm only. After discussing this with the Lead FA, I refused the aircraft and asked for another handset, or to swap the front and rear handset. Maintenance decided to do neither and gave us a 3rd aircraft. During my discussion with dispatch I was told Flight Did Not Operate had been entered in the logbook. Two days later I was in ZZZ flying XXXX to ZZZ1. I pulled the paperwork in the hotel prior to getting on the van and noticed same nose number, Aircraft X, same deferral. After another conversation w/ Dispatch and Maintenance, I refused the aircraft as it was the only way to get rear and forward handsets swapped. It took Contract Maintenance 4 minutes to swap rear and forward handsets. However, it took Contract Maintenance 3 hours and 35 minutes to get to the aircraft after my initial refusal, as no one answered the phone until XA50 in ZZZ. During the delay I noticed that the original I had requested from Dispatch 2 days prior was not in the aircraft logbook. I then attempted to enter the code in the aircraft and could not enter it. Has something changed in Flight Did Not Operate procedures? If the field was populated it generated one thing, if field left blank another. Until now I have not been in an aircraft and inserted Flight Did Not Operate, as I have always been in ops and assumed Dispatch did what they said they did as in this case. Several issues come to mind. How does one enter Flight Did Not Operate? This is not taught anywhere or demonstrated in the syllabus. 2. Maintenance in the 2 phone conversations I had clearly did not agree with my refusal, and said as much. However, the briefing card I carry on my lanyard and which I use prior to and after every flight has the word COMMUNICATION in the center for a reason - and it is something that the training department and LCA emphasize repeatedly. I submit that the communication between the Captain and the forward FA is extremely important in some non-normal situations, medical situations and threat level issues. While I do understand the Maintenance viewpoint (and that of ZZZ) emphasized to me that the aircraft will still fly without a handset, I believe that in order to do the job correctly the Captain and the Purser need to be able to talk real time. Additionally, the Purser in question in ZZZ had 5 weeks of flying on the line. Therefore I would also submit that the Forward FA handset should not be deferred. If you need an NTSB report to emphasize the importance of Pilot and FA communication, then I would rec that the NTSB report from OAL high speed abort, engine failure be given to the Maintenance Controllers and ZZZ Operations to more clearly reveal some of the threats we brief and prepare for, and why communication is important.

Synopsis

Captain reported refusing the same aircraft twice over safety concerns regarding an MEL for the Forward Flight Attendant's Handset Inoperative.

ACN: 1963854 (465 of 2908)

Time / Day

Date : 202301
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Climb
Route In Use : Vectors

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1963854
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X. Departed out of lax and got Engine 2 N2 high vibration. Ran ECAM actions and adjusted thrust lever and vibration went away. Continued to get spurious high vibration ECAMs multiple times throughout flight. About an hour into flight we got Eng 2 Oil Chip Detected ECAM which procedure only required Crew awareness. About 30 minutes after that while in descent around FL 250, we noticed a vibration and something didn't feel correct immediately followed by an Engine 2 Stall. Followed ECAM and procedures and tried to initiate a relight which was unsuccessful. We shut down Engine 2 and requested priority handling. Closest suitable airport was our destination of ZZZ and also runway was considerable length for landing distance, requested direct to the airport and ATC vectored us for final approach. We let Dispatch, Flight Attendants and Passengers know. Landed safely and stopped in taxiway for trucks to check out Number 2 Engine. No damage or fire reported. Taxied to the gate normally.

Synopsis

A320 First Officer reported Engine #2 N2 vibration after takeoff. Later in the flight, flight crew received Engine #2 Oil Chip Detected ECAM Message and noticed a vibration. The flight crew performed an inflight shut down and landed at destination airport.

ACN: 1963465 (466 of 2908)

Time / Day

Date : 202301

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
Function.Ground Personnel : Vehicle Driver
ASRS Report Number.Accession Number : 1963465
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : ATC

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Aircraft
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was the Crew Chief on Gate XX working Aircraft X to ZZZ. After an uneventful upload my Wing Walkers (Person A and Person B) were in place and I contacted the Flight Deck for push clearance. The Pilot received clearance and relayed the instruction "Clear to push, pull to Spot XX." Aircraft X was on an A320 but after countless pushes of 321s out of Gate XX, when I commenced my push pull I pointed the tail of the aircraft at Gate XY (pushing a A320 tail south out of Gate XX is possible but a tight push that can end with the APU ft. away from carts at T Point). Gate XZ was given clearance to push Aircraft Y at the same time. At this point my A320 and Person C's 737 were on a collision course. Thankfully the Wing Walkers all did their job and Gate XZ halted their push while I pulled my aircraft out of the way. During my push from Gate XX and Person C's push from Gate XZ we were both in each other's blind spots. The nose of each aircraft blocked the view of the other. Once the nose of my aircraft cleared I saw the 737 and I realized what was happening. Person C was on top of it and immediately stopped his push while I cut my push tighter to clear. When Gate XX was retrofitted for A321s [Maintenance Crew members] were told by Tower and Management that Gate XZ would not be allowed to push while Gate XX was commencing a push/pull. This was because Gate XX would have to point the tail of the 321 at Gate XY (or at least wide of line XY) to avoid T-Point. A tail south push of A320s is possible but not ideal as it is very tight quarters. After the near miss I called the Tower and asked to speak to Ramp Control. That is when Person D informed me that it was SOP for Gate XX and Gate XZ to push simultaneously. She said I should have pushed my aircraft completely tail south before towing to Spot XX. I asked her "If I was pushing a 321 would you have given Gate XZ clearance to push?" The answer was no. I believe it is dangerous to have different push expectations out of the same gate with different aircraft while the push back verbiage remains the exact same. Do not allow Gate XZ and Gate XX to push at the same time.

Synopsis

Ramp Crew Chief reported communications issues and failure of ATC to coordinate simultaneous push backs on adjacent gates led to a sudden stop to prevent a collision between aircraft. The Ramp Crew Chiefs pulled aircraft back and re started the push backs with closer coordination and the aircraft departed.

ACN: 1962283 (467 of 2908)

Time / Day

Date : 202301

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 98.75
Experience.Flight Crew.Type : 1610.40
ASRS Report Number.Accession Number : 1962283
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Airport

Narrative: 1

Uncommanded Aircraft Movement Arrived in ZZZ from ZZZ1 for fuel and catering. Followed the Self Park system in to gate XX, received the "STOP" sign followed by the "OK" sign. We accomplished the Shutdown Checklist per SOP and noticed the Jetway Driver not willing to move the jetway. This being, for a lack of a better term, an irregular operation I was thinking the Jetway Driver wasn't sure if he should proceed to put the jetway up to the aircraft to open the main cabin door. I gestured for him to do just that as we needed to be fueled and his hand signals back were confusing, but clearly there was some sort of conflict with the ramp. A few moments later the ground crew plugged in a headset to advise us that we were just a little short of the stop line so they needed to tow us the last little bit in order to line up with the jetway. I explained that we had passengers standing in the aisle, etc. and that I would get back to him when we were ready. The passengers were quickly back in their seats and I informed the ground crew that we were ready. The Ramper said they were ready and asked me to release the Brakes, I released the Brakes and verbalized that they were released. I felt a sensation of the aircraft rolling backward momentarily but couldn't tell if it was the jetway moving, or the the aircraft moving. The Ramper came back on the headset and quickly asked me to set the Brakes. Sensing urgency in his voice, I immediately set the brakes and asked what was going on. He explained that the aircraft rolled backwards because he didn't have it secured. No damage or injury was incurred during the Uncommanded Aircraft Movement. Upon debriefing with both the Ramper over the headset and the Jetway Driver in person I learned that they've been having issues with the Self Park system at gates XX and XY with aircraft ending up short of the stop line even though the system shows "OK". Combine this with the fact that station ops will not allow the Jetway Driver to move the jetway until the aircraft is exactly on the stop line creates a degradation in safety with needing to coordinate to move the aircraft, literally inches, in order to position the jetway. I feel that there is a safer way to do this by either re-calibrating the Self Park system, allowing a little variance in jetway movement, or at the very least revert to a Marshaller until such time as the Self Park system is corrected.

Synopsis

A320 Captain reported difficulties during gate arrival using a â€œself-parkâ€ system. The aircraft stopped short of the gate and rolled several feet before the tug driver secured the aircraft to be towed the remaining distance.

ACN: 1962178 (468 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors

Component : 1

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1962178
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After departing ZZZZ we received a Skin Valve Fault on ECAM. FO on his third flight of IOE continued to fly the airplane and communicate with ATC. We completed the after takeoff checklist followed by the ECAM and QRH. ATC was notified we needed to stop our climb at FL100 to troubleshoot a pressurization issue. The ECAM actions did not rectify the pressurization problem. Dispatch and Maintenance were contacted via SAT COMM. Maintenance verified we had completed the ECAM and asked us to cycle both the inlet and extract override push buttons. The ECAM and QRH actions did not solve the issue of the aircraft not pressurizing. Between myself Name and Dispatch a decision was made to return to ZZZZ. We asked to hold at the IAF and informed ATC that we would be returning to ZZZZ. At this point the Flight Attenmdents (FA) and passengers were informed that we would not be able to go to ZZZ and would be returning to ZZZZ. In the holding pattern I set up for the approach to Runway XX and completed the overweight landing checklist. After briefing the approach we completed the RNP uneventfully. After landing and turning off of the Runway we received a OFV not fully open ECAM. One thing I could have done better in this event was verify the OFV was fully open. In the ECAM actions one of the steps was to select MAN pressurization and hold the VS to the full up. I did this however I was of the impression that full up meant to equalize the current pressure altitude to the cabin altitude. Upon selecting full up the cabin immediately went to our current altitude. I thought I had held full up long enough. The cabin altitude in the airplane during the decent was equal to the outside altitude. After landing and the OFV not fully open ECAM I selected MAN VS up for a few seconds and the ECAM went away. We were at sea level cabin altitude upon landing but the OFV wasn't 100% open. We taxied to the Gate and wrote up both the overweight landing and the skin valve fault.

Synopsis

A320 Captain reported a Skin Valve Fault on ECAM after departure. The flight crew ran the QRH and checklists. The aircraft would not pressurize correctly so The flight crew requested vectors and performed an air turn back to the departure airport.

ACN: 1961534 (469 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 23000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : PFD
Manufacturer : F/O
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 3191
Experience.Flight Crew.Last 90 Days : 194
Experience.Flight Crew.Type : 3191
ASRS Report Number.Accession Number : 1961534
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Engine EGT over temp, recovered in idle, SE ILS return to field. On departure out of ZZZ at around 23,000 feet, we got a Brakes Hot ECAM. The temp on the right main gear inboard brake showed 400 degrees. The First Officer was the Pilot Flying. I coordinated with ATC to level off and we started slowing down so that we could drop the gear per the ECAM. As we were slowing down, the First Officer told me to take the airplane as she had just lost both of her screens. She selected the Autopilot to AP1 as I verbally told her I was now Pilot Flying. There was no initial ECAM associated with her display losses. Her screens came back but she informed me that her speed tape was missing and information was incomplete and not reliable. The ride was deteriorating rapidly at 23,000 feet with moderate turbulence and a fire. Our radios had a lot of static and communications with ATC was difficult, so we asked for a change in altitude. The Brake Temperature went down for a few seconds to 200 degrees making that ECAM go away but then it came back just as quickly. As I was still above 240 knots trying to slow down and maybe try a different altitude we got an ENG 1 EGT OVER LIMIT ECAM. We noticed the EGT high (at least 650 degrees) with the red tick showing. Our to do step said: THRUST LEVER (affected) - BELOW LIMIT. Retarding the number one thrust lever to idle brought the EGT back in the green. We requested priority handling and asked for immediate vectors back to ZZZ. The Brakes Hot ECAM meanwhile went away so we no longer needed to think about dropping the gear. But now we started getting a NAV ADR 2 FAULT ECAM. The ECAM would stay on a few seconds but then go away before we could accomplish any steps. This kept happening repeatedly. At one point it did stay on long enough that we were able to do the first to do step, AIR DATA SWTG(SWITCHING) - F/O 3. This seemed to help stabilize her screens, however the ECAM kept coming up but then would go away on its own before further steps could be accomplished. Eventually it stopped popping up. We did an uneventful SE ILS to just above minimums on Runway XXL in ZZZ. The fire trucks gave us the all clear and we taxied to the gate. To get three completely unrelated problems within a few minutes of each other while airborne is something that I have never personally experienced until now.

Synopsis

A320 flight crew experienced three unrelated systems failures after takeoff and elected to return to the departure airport.

ACN: 1961035 (470 of 2908)

Time / Day

Date : 202212

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 21600

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Pitot/Static Ice System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1961035
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Departed ZZZ at dusk, I was Pilot Monitoring (PM), climbing through stratus clouds or haze with light chop, TAT was above 10 Degrees C and E A/I was off through maybe FL180 and we noticed the wind noise in the cockpit was much more than typical. We thought this might just be a loud airplane. When we were at about FL216 I noticed our climb rate was zero and sometimes negative. There were tailwinds of approximately 50 kts. The Captain, also Pilot Flying (PF), selected speed from our current 320 kts. to 280 kts.. The aircraft initiated a climb but I don't recall climbing more than about 600 FPM and we didn't climb for long. Maybe a few hundred feet. I suggested to use vertical speed. The airspeed slowed as much as 260 kts. and we were climbing at a selected vertical speed up 1,200-1,500 FPM. I originally thought that we were having an auto flight problem, perhaps. I checked all the system pages and found them to be normal. The Engine Pressure Ratio was 1.34 I recall and thrust indication matched thrust lever angle cyan circles. At some point before this when the TAT reached 10 Degrees C, I had turned on the engine A/I. Both PFDs and the standby indicator matched airspeed indications. Finally, I had the idea to turn off the dome light and turn on the ice light and wing lights. I saw a large piece of clear ice on the ice indicator and it extended horizontally approximately 1 inch per side. The wings were somewhat hard to be sure but looked to have a good amount of clear but lumpy ice on them. I turned on the wing A/I after mentioning to the Captain. What I think happened was that the pitot tubes (all three) must have been frozen over and were no longer indicating accurate airspeed. Maybe the engine inlet sensors also as the Engine Pressure Ratio was lower than I would have expected and it increased some minutes after exiting the icing condition. I believe our airspeed was far higher than indicated as I've never heard wind noise that loud in the cockpit before. The plane seemed to not have enough power to maintain an altitude of FL 290 and had full power going Mach .78. After a few minutes the aircraft seemed to fly better and although Engine Pressure Ratio was now limited to I believe 1.44, the engines did not need to be at max thrust. This is a hard situation to fully understand because I'm not sure if the icing and its weight and drag were a larger factor or that I believe all three airspeed indicators might be iced over.

Synopsis

A320 First Officer reported excessive wind noise on climbout. Investigation determined that they were flying in icing conditions and were experiencing erratic Air Data Information due to all three Pitot Tubes frozen. The flight crew performed Anti Icing procedures and continued to destination airport.

ACN: 1959840 (471 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Environment

Weather Elements / Visibility : Haze / Smoke

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Make Model Name : Skyhawk 172/Cutlass 172
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1959840
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Enroute from ZZZ to ZZZ1 while on final, ZZZ1 Tower mentioned that there was traffic on down wind to Runway XXL. They gave the traffic a caution that A320 was on final and caution for wake turbulence. Shortly after that we received a TA and had traffic on downwind. It was hazy and we lost the traffic until 400 ft when my peripheral vision off my left had a Cessna 172 had turned final and was under our left wing within 100 ft. We are now parallel to each other. My First Officer (FO) had lost contact when the aircraft was on down wind and I was watching that I was stable since it is a short Runway. So, I lost contact of the traffic anticipating that they would be behind us rather than turn in next to us. My first thought was to go around however, if I did, this may have caused a catastrophic upset of the 172. It was a short time of 30 to 45 seconds when the 172 descended below our glide path. I was in such disbelief that traffic would turn into us rather than follow behind which would favor due to the winds. My big concern is that our passengers could have [found] this and put it out on social media. In the near future, I will not have tower let traffic turn into our position rather enter from behind.

Synopsis

A320 pilot reported NMAC on final approach.

ACN: 1959671 (472 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 162
Experience.Flight Crew.Type : 3134
ASRS Report Number.Accession Number : 1959671
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After de-icing we received our de-ice information. We were told, as is normal, "de-icing complete. All our equipment is clear. Contact Ground for taxi." We then finished our checklist and called Ground for taxi. We were given "taxi to Runway XXL". I looked over my left shoulder and confirmed that all the trucks were clear and said "clear on the left". The First Officer (F/O) looked over his right shoulder and did the same and said "clear right..." I then turned on the taxi light and released the parking brake. The F/O then said "no, wait, there's a truck that just drove in front of the wing." I reapplied the parking brake as the truck then drove under our nose and exited the pad to my left.

Synopsis

A320 Captain reported a ground conflict with a vehicle during taxi out for takeoff.

ACN: 1959238 (473 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1959238
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff, received ECAM â€œBRAKES AUTO BRK FAULT, BRAKE RELEASED.â€ With no blue action items and no follow-ups in the QRH, I referred to the ECAM supplemental non-normal manual which indicated with brakes released, brake on one gear, or one brake on each gear may be released. After conferring with Dispatch and Maintenance Control, we made the determination to return back to ZZZ where we would be able to land on a long runway in the event that directional control may be an issue due to a brake release. ARFF (Airport Rescue and Firefighting (ARFF) presence was requested due to the potential of directional control issues.

Synopsis

A320 pilot flying reported being alerted to an ECAM message for an automatic brake fault. The flight crew then decided to perform an air turnback.

ACN: 1958748 (474 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : ELAC 1
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : ELAC 2
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1958748
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

In cruise flight at FL340 we got an ECAM, ELAC 2 Pitch Fault. After performing ECAM actions and referencing the communications (COM), it was a crew awareness item. Approximately 30 minutes later at the top of descent on the ZZZZZ arrival we received an ECAM of ELAC 1 Pitch Fault, autopilot off, Alternate law with all Protection lost. After performing ECAM and referencing the COM we decided that we were stuck in Alternate Law and the COM referenced us to perform the Alternate Law procedure. After going over that we decided that continuing on the arrival and landing in ZZZ was our best option. It was also a perfect weather day in ZZZ. We landed with Flaps 3 and also Vref plus 10 per the procedure. We requested priority handling knowing we would eventually land. Briefed the Flight Attendants that we expected a normal landing but they would see the crash fire rescue (CFR) trucks and if anything changed we would let them know. Also made a PA explaining what happened and also that they would see the CFR trucks and this was a precautionary situation but we expected a normal landing. We then proceeded to ZZZ and made a successful, normal landing in direct law. Continued to the gate with no further incidence after talking to CFR and releasing them.

Synopsis

A320 Captain reported the loss of both ELACs in cruise. The flight crew requested priority handling and continued to the destination airport in Alternate Law. A precautionary landing was made.

ACN: 1958002 (475 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1958002
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departing Runway XXL. Initial takeoff normal. Airspeed alive call made. During takeoff roll we had an ECAM pop up. I believe this happened before 80 knots. I am unsure if I made the 80 knots power set call. Captain advanced thrust levers to TOGA. No continuing call made, but I understood the intention to go at TOGA. Engine 2 Engine Pressure Ratio (EPR) mode fault along with Engine 2 system fault at the bottom of the ECAM page. To be clear this is what the ECAM read after the Rejected Takeoff (RTO) as we had a chance to read it after stopping. Everything else seemed normal with the aircraft at this point. At this point an RTO thought popped into my mind. Being go minded this seemed like a reasonable response as the aircraft did not feel as if it was unable to fly. As we accelerated we heard a Bang. The Captain called reject and at this point the takeoff was rejected. This was during high speed regime. Unsure as to actual speed. No call made as I was distracted by the bang and ECAM. I advised tower that we were rejecting the takeoff and we taxied clear of the runway to a stop. Reading through the ECAM, we shut down Engine 2 and started the APU. A call to remain seated was made. I talked to the in-flight crew to see if they smelled or saw anything of concern. One crew member mentioned the smell of burnt rubber which I figured was somewhat normal as we had indeed heated up the brakes. After coordinating with company and ATC we proceeded back to the gate. We also, at this time made a PA to the cabin advising that we were returning to the gate. The aircraft taxied without any tactile indication that the tire was flat. Didn't find out about that until gate return. At the gate the Captain addressed the customers and told them as much as he knew about our next steps.

Synopsis

A320 First Officer reported a high speed rejected takeoff due to an engine failure. The engine was shut down and the aircraft returned to the gate for maintenance.

ACN: 1957995 (476 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1957995
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were at the gate waiting on a Maintenance Technician to sign off the logbook of a discrepancy. I started up the APU and when I turned on the APU Bleed, a very strong foul odor filled the flight deck and cabin. The Maintenance Technician concurred with the odor so we proceeded to the cabin to verify the odor. The odor was extremely strong throughout the cabin. The flight attendants were very nervous and concerned with the smell. I went back to the flight deck and turned off the APU Bleed and the odor started to slowly dissipate. The Maintenance Technician called Maintenance Control after I wrote up the odor. Maintenance Control decided to take the aircraft out of service based on what the Maintenance Technician told them and they quickly were able to get us a new aircraft. None of the crew were feeling any ill affects from the odor and were comfortable continuing in the new aircraft. No customers were onboard.

Synopsis

A320 Captain reported a very strong, foul odor filled the entire aircraft shortly after APU startup during preflight. Maintenance removed the aircraft from service.

ACN: 1957203 (477 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Descent
Flight Phase : Initial Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1957203
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At approximately XA:41 and descending through 4,000 ft. on the ILS XXL approach into ZZZ, the #1 FA (Flight Attendant) called the flight deck. We did not answer the call due to workload and weather during the approach. After landing and taxiing off the runway, I contacted the #1 FA to inquire about the call. She communicated that they had experienced an odor event during the approach that started in the aft cabin and quickly enveloped the entire cabin. She described the odor as a "dirty sock smell." She indicated that the odor seemed to dissipate upon landing. We continued the taxi and parked at the gate. After the checklists were complete, I exited the flight deck and communicated further with the inflight crew. The description of the intensity varied from strong to mild depending upon the inflight crewmembers' past experience. I made my way up the aisle during the deplaning process, but could not detect any lingering odor. The FO (First Officer) did the same after the post-flight walkaround and could not detect an odor in the cabin. I called Maintenance Control and told them that we had experienced a fume/odor event during the approach. I wrote up the event in the maintenance logbook and filled out the form. We secured the aircraft since it was not due to go back out for several hours. Contract Maintenance showed up before we left the aircraft and I briefed them on the event. The entire crew regrouped and discussed the event in the jet bridge. All crewmembers communicated that they were feeling fine and had zero lingering effects from the event.

Synopsis

A320 Captain reported a fume event that was identified by the flight attendants as a "dirty socks" odor during descent. After arriving at the gate, the flight crew contacted Maintenance to inspect the aircraft.

ACN: 1956915 (478 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1956915
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Fatigue
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

I was the PM (Pilot Monitoring) 40 miles from ZZZZ at 20000 ft. Asked ATC for lower. Cleared to descend to 4,000 ft. and cleared for the VOR DME 1 Runway XX approach. We were at 16,000 ft. and approximately 10 miles from the VOR when I suggested gear down. Gear selected down at approximately 14,000 ft. and 240 kts. Managed descent to 1,600 ft. and armed the approach. Prior to making procedure turn inbound at 5,000 ft., I asked Approach for present heading for two miles. "Cleared as requested". FO (First Officer) selected heading but 1,600 ft. remained in FCU with V/S defaulted at 2,700 ft. per min. Speed was selected to 200 kts. I suggested turning left direct ZZZZ when it was obvious we were no longer high on the approach. I called out low altitude at approximately 2,200 ft. and asked PF (Pilot Flying) to climb back up to 2,500 ft. We received a "TERRAIN AHEAD" warning when we were close to 1,600 ft. Day Visual conditions and cleared for the visual at this point. Could see terrain at all times. Climbed back above 2,000 ft. and commenced approach. Stable and configured by 1,000 ft. We were kept very high by ATC. Bad descent planning and bad descent technique. First flight for the CA (Captain) for several months due to check pilot duties in the simulator. FO tired from early wake up call, and still new to Company. Ask ATC for lower sooner. Prepare and brief for slam dunk. Communicate better between pilots. Use better techniques for descending fast.

Synopsis

An Air Carrier Captain reported while descending on an approach they received a Ground Proximity Warning and climbed back to the appropriate altitude.

ACN: 1955938 (479 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Fog

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Component : 1

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 7600
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 1950
ASRS Report Number.Accession Number : 1955938
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On initial climb out from ZZZ received a Vent Blower Fault ECAM. Leveled off at 10,000 feet AGL and Completed QRH procedure. Asked ATC for a hold at ZZZZZ to trouble shot. Realized we couldn't control cabin pressurization in manual mode so then requested priority handling. Returned to ZZZ for maintenance.

Synopsis

First Officer reported receiving a Vent Blower Fault ECAM on initial climb out. An air turn back and precautionary landing was made at the departure airport.

ACN: 1955601 (480 of 2908)

Time / Day

Date : 202212
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1955601

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon gear extension for landing at ZZZ, we received an ECAM for BRAKE AUTO BRK FAULT as well as BRAKES RELEASED. A go around was initiated followed by a hold so that we could run all of our necessary checklists and troubleshoot the issue. A phone patch with me was initiated to get their input as to whether or not we would have normal braking with the BRAKES RELEASED message. After we were comfortable with all of the information that we had, we requested priority handling as a precaution and requested landing on Runway XXL as it was the longest runway available. Code yellow was issued to the inflight. Landing was uneventful with normal braking. Crash Fire Rescue was requested to follow the aircraft just in case we encountered any other issues.

Synopsis

A320 pilot reported upon gear extension for landing, receiving an ECAM for BRAKE AUTO BRK FAULT as well as BRAKES RELEASED. A go-around was performed and the checklists and QRH were run. The flight crew requested priority handling and made a precautionary landing at destination airport.

ACN: 1954902 (481 of 2908)

Time / Day

Date : 202211

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 1
Flight Phase : Taxi

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1954902
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Distraction

Events

Anomaly.Ground Event / Encounter : Vehicle
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Equipment / Tooling

Narrative: 1

It was a rainy day we had been waiting for a while for the plane to come in. The plane was parked by the driver and I was crew chief and chocked the plane. The driver brought over the towbar to connect it. He inserted the bypass pin and I lined up the towbar so he can connect it. I moved on to the pushback. I turned it on everything seemed to work fine as I approached the towbar. I was seated and driving the pushback on first gear. As I came closer I came to stop so that the driver could remove the pin in front of the pushback. All of a sudden I felt the pushback start rolling. Everything happened so fast and I could not stop the tractor. I try pressing on the brake but it was too late. Everything happened so fast. I realized the event occurred because of the pushback rolling and my not being unable to stop it. From my perspective, it was an equipment failure. I wholeheartedly believe that my foot was on the brakes while trying to remove the pin. I even remember trying to press down on the brake to stop it but it just kept rolling. I also believe giving the I was position seated I would've felt my leg move because it was bent while pressing the brakes. Additionally, if it had so happened to press the accelerator instead of the brake the pushback would have kicked up in speed, which it did not, and caused a bigger impact. Immediately after the incident, I called the manager on duty to report it. I stayed in position with my foot on the brake because I did not want to move in case it could cause more damage. I believe that we should be more trained in the inspection of all equipment. That it should be something that is done on regular basis. We can prevent these incidents by getting ahead of them. I also think the brake and gas pedal could benefit from having a safety grip or material in order to make equipment safer. In my history of working at the airport, we've seen many belt loaders take off under the plane, and employees have to jump off to prevent impact. I think an extra line of defense would help in cases such as these where our equipment fails to work the way it should.

Synopsis

Ramp Agent reported a tug brake failure during pushback allowing the aircraft to roll backwards uncontrolled.

ACN: 1954653 (482 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1954653
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Odor/fume event due [to] de-icing procedure performed in ZZZ. Strong odor from de-icing fluid caused all 4 inflight crewmembers to be treated by medical personnel due to the effects of the odors. EMS was called initially and then a follow-up call to Company was made by the inflight crewmembers.

Synopsis

A320 Captain reported the Flight Attendants experienced physiological symptoms from the de-icing procedure at departure airport. The affected Crew Members were treated by medical personnel.

ACN: 1954651 (483 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Taxi

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1954651
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Immediately after landing, flight attendants called and reported a dirty sock odor in the cabin. We did not have it in the flight deck. Complied with the odor checklist.

Synopsis

A320 Captain reported that the Flight Attendants notified the flight crew of a "dirty socks" odor upon landing. The Flight Crew complied with the odor elimination checklist.

ACN: 1953654 (484 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1953654
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Detector.Person : Passenger
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after takeoff, the inflight crew notified me of an "intense fume event" that was happening in the back half of the cabin. I was told that passengers were very much complaining and trying to move to the forward half of the plane during the climb-out, approximately 4,000 ft. The inflight crew had to relocate as well. We immediately ran all necessary procedures. I asked my FO (First Officer) to continue flying and to take the radios so that I could begin troubleshooting. The odor isolation procedure proved to be unsuccessful. After each step, I checked back with the inflight crew to see if conditions improved. They did not. The QRH procedures were also unsuccessful, up to the point where we were forced to turn BOTH packs off. At this point, it was obvious that we would not be able to climb above 10,000 ft. due to our unpressurized state. Continuing to ZZZZ was no longer viable. At this point, I assumed the radios and asked ATC for a return to ZZZ. Inflight crewmembers stated that the fumes had subsided at this point, but they were experiencing symptoms such as dizziness, burning eyes, chest pain, and labored breathing. Each inflight crewmember seemed to have different symptoms. I asked if they wanted medical attention, to which they declined. As we got closer to ZZZ, I told the First Officer that I would rather request help and not need it rather than the other way around. While the First Officer was managing the aircraft and radios, I notified Dispatch, ran the divert QRH, the Over-Weight Landing QRH, helped reprogram the FMS, etc. A normal approach and soft landing was made. All overweight considerations were made and the emergency vehicles followed us to the gate. No assistance was needed. Upon arrival, a normal deplaning was conducted. EMS checked out the entire crew.

Synopsis

A320 Captain reported being notified by the Flight Attendants of an "intense fume event" in the cabin. The Flight Crew returned to the departure airport.

ACN: 1953384 (485 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 95
Experience.Flight Crew.Type : 282
ASRS Report Number.Accession Number : 1953384
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise on Aircraft X, from ZZZ to ZZZZ, an ECAM HYD SYS B LO PR annunciated. Captain was the Pilot Monitoring for the first leg and a transfer of controls to me. He then ran the ECAM items and completed the applicable QRH items. At this time we were well into ZZZZ's airspace and approaching the ETP. Captain contacted Dispatch and they conferenced in Maintenance Control for advise and a second opinion. Maintenance did not have any additional suggestions. After consulting the Team, which included the Flight Deck jump seater, a Company First Officer, who happened to have Airbus experience, the decision was made to continue on to the destination as it could be done safely. We elected to [request priority handling] with ZZZZ Center and have emergency equipment standing by in the interest of safety. We landing in ZZZZ safely without and other incident.

Synopsis

A320 First Officer reported loss of Blue Hydraulic System in cruise. The flight crew continued to destination airport and landed.

ACN: 1953382 (486 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 4535
Experience.Flight Crew.Last 90 Days : 124
Experience.Flight Crew.Type : 3696
ASRS Report Number.Accession Number : 1953382

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Pressure on hydraulic page was steady at 3,100 PSI. No QRH procedure correlated. After conference call with Dispatch and Maintenance Control crew elected to return to ZZZ. [Requested priority handling]. Landed Runway XXR. Taxied to gate.

Synopsis

Captain reported hydraulic pressure malfunction and returned to departure airport.

ACN: 1952917 (487 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Initial Approach

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1952917

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending into ZZZ we got a call from the flight attendants. I was told the aft galley smelt like dirty sock smell. All 4 of the flight attendants agreed that it did. I inquired if anyone was experiencing any symptoms or irritants as a result and they all reported feeling normal and well. I wasn't told about the odor until below 10,000 ft. but I was later told it started at top of descent. The odor was persistent in nature during the entire descent. Then, it disappeared upon landing and shutdown. At the gate, Maintenance ran the APU bleed with the packs on at the gate with the engines shut down. Nobody on the plane noticed the dirty sock odor anymore. Because the airplane had no prior history and the odor had disappeared, Maintenance returned the aircraft to service with the APU still operable. We flew the plane to ZZZ1 and did not notice the odor on the subsequent flight.

Synopsis

A320 Captain reported receiving a call from the Flight Attendants who reported a "dirty socks" odor in the passenger cabin during the top of the descent. After arriving at the gate, Maintenance tested the APU and the odor was no longer there.

ACN: 1952674 (488 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : FADEC / TCC
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1952674

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Primary Problem : Aircraft

Narrative: 1

Aircraft X. During Cruse flight preparing for the descent I gave control to Captain to do Approach Briefing. Doing that at FL370 Master Caution came on and ENG 2 FADEC SYS FAULT and ENG 2 SYSTEM FAULT ECAM messages as well. Captain called for ECAM actions and they were performed. With ENG 2 in IDLE aircraft started to slow down. A Descend to FL240 was requested to ATC and they denied it due to traffic. Captain requested to get off from Airway to perform drift down as aircraft was slowing down. FL340 was given but we needed to decent more. Then Captain requested priority handling and explained ATC what was going on in order to get to lower altitude and we were cleared direct to ZZZ. Around 5 minutes later ECAM messages disappeared and engine parameters were checked. We decided to use engine 2 back as Eng 2 parameter were continuously checked. ATC was notified that we had that engine back. COM 2 was checked for more actions to do. I took aircraft control and we briefed for plan of action in case we had more problem with Eng 2. I landed aircraft without any problems. During taxi ENG 2 TURB COOL FAULT ECAM message appeared. Eng 2 was shutdown after 3 minutes and message went away. Maintenance was notified and next flying crew were briefed about what happened.

Synopsis

A320 First Officer reported a dual system FADEC loss in cruise. The flight crew requested priority handling and continued to destination airport at a lower altitude.

ACN: 1952568 (489 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 20300
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 10356
ASRS Report Number.Accession Number : 1952568

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 10562
Experience.Flight Crew.Last 90 Days : 174
Experience.Flight Crew.Type : 3689
ASRS Report Number.Accession Number : 1952588

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Diverted
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Aircraft

Narrative: 1

Just prior to rotation a pelican flew in front of the plane on the right side. Immediately after rotation we felt it hit the plane and #2 engine started to vibrate. We continued the climb. Told ATC what happened, [requested priority], and prepared for a return to ZZZ. We leveled off at 3000 ft. All engine indications on #2 were normal except for the vibration. It was at 7.0. We kept the engine running. We did the overweight checklist, got landing performance data and discussed that if we had to do a go around, it would be the engine failure procedure profile, just in case. We briefed the flight attendants and passengers and made an uneventful landing on Runway XXR. After fire rescue personnel determined that engine 2 was not smoking, we taxied back to the gate.

Narrative: 2

Ingested a large pelican on the number 2 engine on takeoff. Requested priority handling and landed on XXR. Engine vibration but did not shut down. Landed safely.

Synopsis

A320 Air Carrier Flight Crew reported bird strike during takeoff.

ACN: 1952564 (490 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1952564

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight from ZZZ started out pretty normal, earlier in cruise flight we had an ELAC2 computer reset. The event happen around XA59z in cruise. The ECAM reported a (HYD G RSVR LO LVL / HYD G ENG 1 PUMP LO PR / HYD G SYS LO PR). We ran the ECAM action and the COM2, and recognized that we would have to lower the landing gear via a gravity extension. At this point I knew we had commuter that was headed to ZZZ1. Since I was doing IOE with the First Officer, I thought it would be better to have more input on the situation especially from an Instructor. I notified the Flight Attendants, the Company and [requested priority handling] and continued to ZZZ1. I requested a longer then normal approach to final for the flight control and the gravity gear extension. Landed the airplane on Runway XXR from the ILS, came to a complete stop on the Runway ran the emergency procedures with fire and rescue. I taxied to the Gate with no incident after land and park checklist completed. I reserve the right to amend this report as additional information becomes available to me.

Synopsis

Captain reported loss of the Green Hydraulic System in cruise. The flight crew continued to destination airport and landed.

ACN: 1952304 (491 of 2908)

Time / Day

Date : 202211

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1952304
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On [Date] I was preparing to fly an Airbus A320 from ZZZ to ZZZ1. During preflight we potentially had a fume event. The estimated duration of the event was 15 minutes. During the event the air was being supplied by the APU Bleed. I have not experienced this in the same aircraft type before. There was no evidence of smoke or fire in either the flight deck or cabin. The fumes smelled musty like dirty socks and were mild in intensity. They were mostly experienced in the flight deck. During the time of the event, our air supply source was the APU. We opened the windows and shut off the APU Bleed Air because of the event. This event impacted our operation by causing [the company] to delay the flight to find a suitable aircraft. I went to the ER to have arterial blood gas (ABG) & cholinesterase tests performed after the exposure.

Synopsis

A320 First Officer reported a fume event during preflight. Maintenance removed aircraft from service and a replacement aircraft was requested.

ACN: 1952300 (492 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1952300
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During takeoff, right after rotation, #2 Eng EGT OVERLIMIT ECAM when I noted it, was approximately 627 degrees. After conversation with Maintenance personnel they recorded 635 degrees. At thrust reduction altitude, power was reduced from TO/GA to climb power and few moments later, EGT came back to approximately 575 degrees and stayed in this range throughout climb. After contacting Dispatch and Maintenance Control with my concerns â€“ #2 Eng EGT running 60 â€“ 80 degrees higher/fuel burn 400 lb. more than number one engine, and a slight smoke odor â€“ they concurred it was prudent and out of interest of safety to return to ZZZZ. Appropriate logbook entries were annotated and verbally explained to Maintenance personnel upon arrival.

Synopsis

A320 Captain reported receiving an "EGT OVERLIMIT ECAM" alert for the number 2 engine during initial climb. After contacting Maintenance, the flight crew performed an air turnback and submitted logbook entries for event.

ACN: 1951964 (493 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DCA.Airport
State Reference : DC
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DCA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : DCA
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1951964
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.No Specific Anomaly Occurred : Unwanted Situation

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Airport

Narrative: 1

Runway 19 hold short line has been moved. Taxiway Juliet center line markings have been moved 75 feet west. There is a NOTAM for non-standard markings. This change has changed the Tower controller's options for aircraft movements at the hold short line. This is an accident waiting to happen! We were number 1 in the hold bay. Tower instructed us to pull up to the hold short line of Runway 19. Tower changed the plan and cleared a B737-800 to position and hold. This aircraft was holding short on the taxiway Juliet line. As the 737 started to roll it quickly became apparent that the left wingtip of 737 family aircraft hangs over the north-south hold short line by 15 to 20 feet. Same for A320 family aircraft. Luckily, the 737 pilot realized his left wing tip to my nose clearance was not sufficient and he moved off the taxiway Juliet line to allow enough clearance for him to taxi on the runway. Terrible engineering and measurements! Never should have moved the Juliet taxi line! Tower controllers had standard operating procedures and overnight they were made obsolete. Was any training conducted for Tower Controllers with new SOPs? Repaint the hold short lines back the way they were. If not, at least repaint the lines such that there is proper wing-tip clearance by making proper measurements!

Synopsis

Air Carrier pilot reported an air carrier taxiing onto the runway had to take evasive action to avoid the reporter's aircraft which was at the hold short line. B737 and A320 aircraft taxiing on the new Taxiway J centerline markings at DCA do not remain clear of the north-south hold short line.

ACN: 1951415 (494 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1951415
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was flying an A320 from ZZZZ to ZZZ. I have not experienced something like this in this aircraft type. Power did not change throughout the event as the APU was the bleed source. It did rain for a few minutes while we taxied in to the gate. After deplaning an odor of dirty socks was noticed in Cabin. Crew dispersed throughout aircraft to determine source. No physical item was found so we thought it must be air supply. Captain and I performed fume event checklist. Once we turned off APU bleed the odor stopped. Cabin was also clear of smell. We tested the bleed again and odor immediately returned. We called Operations and requested local Maintenance to come to aircraft and delayed boarding. Local Maintenance agreed it smelt like an APU issue but said it was okay. We then wanted Maintenance Control involvement but had many dropped calls throughout process. Maintenance Control wanted us to start engines at the gate and test engine bleed through packs to see if the odor returned. We preformed test with Chief Pilot approval. I was not and have never been trained on performing maintenance tests. I have not had training on certifying air cabin quality for human use yet I was asked to do so. The odor was minimal to none during the test. I asked the Flight Attendants to leave the aircraft. I asked that the Spanish fluent FA stay in case we needed translation operating 2 engines at gate. Maintenance Control and local Maintenance placed the APU under MEL. We then had the issue of the cabin overheating so we requested ground air. The crew agreed it was safe to continue and no one felt sick or disoriented. This took some time to arrive. We began to cool the cabin as quickly as possible. We then boarded and pushed back, there was a faint lingering odor that was intermittent. No one else smelt it so I did not think anything of it as we were operating the aircraft at an international airport. It stopped about 10 min into engines supplying bleed. Flight was completed after lengthy delay. We did not don the masks as we were on ground and had cabin door open. There was also concern if we went on mask then the limited communication capability we had would be lost.

Synopsis

A320 Captain reported a "dirty socks" odor during arrival taxi to gate. Maintenance MEL'ed the APU and returned the aircraft to service.

ACN: 1951403 (495 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1951403
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

All aircraft systems were operating normally, APU used for engine start. No de-icing of aircraft, engine anti-ice selected on for takeoff and engine bleeds on. During climb to cruise at 10,000 ft., Inflight called and notified that the crew and passengers were smelling "dirty sock" odor in the cabin. We also began to smell the odor in the cockpit. Cockpit / Cabin Odor Recognition and Isolation, was referenced and flight deck oxygen masks were deployed. Inflight was called and advised to utilize oxygen as well if necessary. The odor was reported as increasing and intensifying in the cabin. Per the guidelines, the intensity was determined to be [low] initially but increased. A message was sent to Operations advising of the situation and requesting diversion options. Operations indicated ZZZ as the preferred divert destination and we concurred. ATC was notified and we received clearance to ZZZ. Dispatch sent planning information. A passenger PA was made explaining the cabin air issue and that we would be returning to land at ZZZ. ATC provided expedited handling of our arrival and thus no emergency was declared. Inflight was advised of information. An approach, landing and gate arrival were completed without incident. PACs 1 and 2 were deselected when on the ground. None of the passengers or crew requested or showed indication of needing immediate medical assistance. Maintenance met the aircraft, a logbook entry was made and a cabin odor report was completed.

Synopsis

A320 Captain reported a fumes event during initial climb. After conferring with Dispatch the flight diverted to a suitable alternate.

ACN: 1951327 (496 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1951327
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent at 12,000 ft., I smelled a bad smell and commented to the Captain about it. He also smelled it. We then noticed that it smelled like dirty socks. The Captain directed me to send a message to Dispatch so they could plan for Maintenance due to a fume event while he checked with the Flight Attendants. The A Flight Attendant also smelled the odor. We continued the descent and the smell grew stronger. I stated that my head was hurting and I felt nauseated. The Captain also was noticing weird symptoms so he called for us to don the oxygen masks and run the procedure for smoke/fire/fumes. While I was accomplishing the checklist, the Captain flew the airplane and handled radio communications and [requested priority handling to destination]. We flew the RNAV approach due to a NOTAM for the ILS and IMC. Upon landing, we taxied to the gate and shutdown the engines and opened the windows before removing the oxygen masks. We got the passengers off quickly. The A Flight Attendant admitted that she threw up and had a sore throat. I also felt unwell. While walking to the hotel van, all 3 Flight Attendants said they felt odd and dizzy with headaches. I also had the same symptoms. We all discussed going to the ER but the Pilots and Flight Attendants were scheduled for 2 different hotels. I exchanged numbers with them and told them to call if they continued to worsen or needed medical care. After getting to the hotel, I continued to feel dizzy, have a headache, and felt like my coordination and concentration was taking more energy than usual. I went to the nearest ER within 1 hour of the event. The Flight Attendant text me and said she vomited 2 times and felt very sick but that she and the other flight attendants were not going to the ER because they "can't afford the sick points, and our union said we'd get penalized and have to fight to remove the points." I received blood work, fluids, and O2 for 4.5 hours at the ER. I removed myself from the trip and still felt unwell for 48 hours. The A Flight Attendant worked the next day but sought medical attention after her trip ended due to lingering effects. Find and fix the issues causing fume events and don't allow employees to be penalized or fear reprisal for seeking medical assistance/ being pressured to fly unwell.

Synopsis

A320 First Officer reported a dirty socks odor during descend. The Flight crew requested priority handling to destination where a safe landing was completed. Crew experienced physiological symptoms.

ACN: 1950531 (497 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component : 1

Aircraft Component : AC Generator/Alternator
Manufacturer : IDG 2
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1950531
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After departure from ZZZ, we experienced an uncommanded transition to thrust lock with the auto thrust. Although at approximately 7000 ft. MSL, managed speed displayed 313 kts. Inflight crew members would later advise that they lost power at the galley. A few moments later, the ECAM indicated ELEC IDG 2 LOW OIL PRESS. After accomplishing the ECAM actions and review of applicable QRH checklists I turned on the APU. We coordinated for holding with ZZZ and then contacted Dispatch and Maintenance Control. Maintenance Control advised that they had an MEL for the Integrated Drive Generator (IDG), however, I felt that flying with one generator across open water in a non-radar environment to ZZZZ was not a good idea. Also, with the APU as a second source of electrical power, I would have increased fuel consumption concerns. Lastly, I was concerned that other damage or oil from the IDG leaking could later present other problems later in the flight. I felt it was safer to have someone look it over on the ground. A [request for priority handling] was made and we performed an overweight landing at ZZZ. The Overweight Landing QRH was reviewed prior to commencing our approach. The landing was smooth with approximately 200 FPM at touchdown. Crash Fire Rescue inspected the engine upon landing and observed no evidence of leaks or damage. We returned to the gate. At the gate, I filled out the IDG discrepancy while several people were waiting to speak with me. Crew services called my First Officer regarding our duty day. I did not enter into the aircraft logbook that we had performed an overweight landing. I also could have entered more specific information regarding the malfunction that we had just experienced. After arriving at the gate, I had numerous parties engage offer help and support as well as asking operational questions. There was an immense amount of pressure from customers, gate agents and supervisors trying to mitigate the next operational obstacle. I became distracted from the logbook while engaging with people who had numerous questions. Next time following an unusual event such as this I will keep the cockpit door closed and review the next steps that I need to accomplish before making myself available to other parties.

Synopsis

A320 pilot reported auto throttle problems and ELEC IDG 2 LOW OIL PRESS indication during departure. The flight crew returned to departure airport and landed.

ACN: 1950253 (498 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component : 1

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1950253

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft X. I arrived at the gate about an hour before departure. I completed normal pre-flight duties noting no abnormalities. We departed on time and reached our cruise of FL380 with no issues. While in cruise, we received a "HYD G RSVR LO LVL" ECAM (electronic centralized aircraft monitor). The Captain continued to fly while I completed the ECAM actions. We turned the Green Pump and PTO (power takeoff) off in accordance with the ECAM. Looking in the COM (communications), it became evident that we would need to manually extend the landing gear. We used to ACARS system to advise Dispatch and Maintenance Control of the situation. They recommended that we return to ZZZ. While returning, I remembered seeing an entry in the logbook of service to the hydraulic system the night before. I looked at the entry again and it showed that the green hydraulic accumulator had been replaced. A longer than normal final approach was requested just to give us time to manually extend the gear and because the slats/flaps would be slow. We landed uneventfully without requesting priority handling and taxied to the gate. While setting up a replacement airplane for the flight, maintenance personal advised they believed that the mechanic the night before had cross threaded the fittings on the accumulator.

Synopsis

A320 First Officer reported a HYD G RSVR LO LVL ECAM at cruise. The flight crew returned to departure airport and landed.

ACN: 1949410 (499 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 4551
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4412
ASRS Report Number.Accession Number : 1949410

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 14415
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 14415
ASRS Report Number.Accession Number : 1949427

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While flying in cruise flight at FL360, aircraft ECAM presented with a Yellow Hydraulic System Low Reservoir Level. Accomplished the ECAM, verified that the Yellow reservoir was indicating low level on the system page, and reviewed the Status page. Reviewed landing distance and choice of runway. Went through options with the FO (First Officer) and we agreed to continue on to ZZZ after contacting Dispatch. [Requested priority handling] with Center and proceeded to a point on final for Runway XXC. FO completed the uneventful landing, FO was very experienced and I was comfortable with him landing aircraft. [Priority] personnel checked the aircraft on taxiway and then we proceeded to the gate with trucks following. While at cruise, I decided to bring the Purser up to brief her on the situation and brief the situation. We did not prep the cabin, but just had them review steps for evacuation so they were fresh in memory. I had plenty of time to bring her to the cockpit for the brief. I had an immediate situation with a young crew before and they came off the plane shaking, so if there is time I now try to look them in eye and let them see I am calm. I think this worked well with our situation.

Narrative: 2

Cruise at FL360 got ECAM Yellow Hydraulic System Low Reservoir Level. Followed the ECAM procedures and calculated landing distance. Contacted Dispatch and [requested priority handling] with ATC. Landed uneventfully on Runway XX center at ZZZ.

Synopsis

Captain reported loss of Yellow Hydraulic System during cruise. The flight crew continued to destination airport and landed.

ACN: 1949087 (500 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1949087
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1949091
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I was a Deadheading Pilot from ZZZ to ZZZ1. I arrived at the plane preparing to sit where I was assigned. When I boarded, the A Flight Attendant told me that they had held boarding to clear the air from the airplane. She said that she could barely breathe when she first stepped on the plane and that the plane had had the dirty sock smell. But the cabin doors had been open for quite some time and the ground air-conditioning cart had been attached for so long by the time I had gotten there that I didn't smell anything. The A Flight Attendant then proceeded to say that it was her impression that this smell had happened on the flight in on the ground after the APU was started on the taxi in. I was unclear as to whether or not the A Flight Attendant had learned the history information from the previous crew, or the Mechanic who was on the Plane at the time in ZZZ. At this point there were no operating pilots on the airplane. I then took the initiative to call Maintenance Control in ZZZ2 and advise them of the situation; I advised them that I was not the operating Captain, nor current on the A320. I advised them that the A Flight Attendant told me that she had noticed the dirty sock smell, however I noticed nothing as I boarded well after the ground air cart was hooked up to the plane with the APU off. Maintenance Control took no action as a result of my call. When the operating pilots finally were about to board the plane, I met with them in the jetway to discuss the matter privately. I advised them that the A Flight Attendant said this plane had had the dirty sock smell and contaminated air. I also told them I didn't smell anything as the ground air cart had been running for long time. The Captain seemed to understand. I went to my assigned seat of XX. The APU bleed was turned on and the door was closed. I immediately noticed the dirty sock smell. I didn't say or do anything about this observation. I briefly contemplated the "what if's" of me, the Deadheading Pilot, attempting to stop the operation or demanding to get off the plane. I didn't choose this option, partly because I know that I have been exposed to this type of contaminated air at least 5 times before. I don't believe that I have been exposed to this more often than my fellow crew members at this airline, or throughout the industry; I do believe that my nose and mind have become more highly attuned to knowing what I am smelling due to the correct diagnosing in the past, and following up with Maintenance Control in each of those instances. Given the ubiquitous nature of how often these issues, and most people exposed haven't been significantly affected, I decided to not inject myself again. I assumed that they would smell it too. The A FA (Flight Attendant) seemed to be highly vigilant about this matter; I assumed she would speak up and say she smelt the dirty sock smell. After we blocked in to ZZZ1 she told me she did tell the Captain that she reported the dirty sock smell to the Captain, and that the Captain replied to her that he was sure it was okay, or something to that effect. I am unclear as to whether that reported interaction between the Captain and the A FA happened in ZZZ or ZZZ1. The smell cleared out some time after the engines were started on climb out. The smell returned on the ground in ZZZ1 nearing gate block in when, I assume the APU bleed was re-engaged. I had the Captain's phone number, so I sent him some text messages so that he would hopefully see then after he blocked in, in case I wouldn't see him after block in. The text messages said: "This plane definitely had oil in the bleed duct", "I've had it five times", and "If you feel any symptoms over the coming days contact [Person A]". I then got up to leave and saw the Captain as he was making his way to leave. I told him I was pretty sure this plane had contaminated bleed. He told me he disagreed and he thought nothing was wrong. He then left. I was highly incredulous to be dismissed so easily on an issue that I knew to be such a threat to health and safety. This was when I had the aforementioned conversation with the A FA regarding whether or not she advised the Captain on the dirty sock smell. I then stepped into the flight deck to see if they made any write-up. There was nothing in the logbook. I waited to talk to a Mechanic who was doing the post-flight walk around. I advised the Technician that I am highly aware what a contaminated bleed smells like and this plane had oil contaminating the bleed. He assured me he would inspect it. As I was leaving the plane from talking with the Technician, I received the following text from the Captain: "I disagree, we had no issues this whole flight. Go home and get some rest." I then responded "I've had it five times. I know what it smells like." I walked to my hotel in ZZZ1. To say I was angry at the Captain is a profound understatement. To have been dismissed by him, when I had been through these issues so often, knowing friends and colleagues who are disabled due to these events, ensured I was in a state of vigilance on this issue. I called Maintenance Control when I got to the hotel before I went to bed. I inquired as to whether or not the Mechanic found anything. They showed no record of anything being inspected regarding air quality. The plane was now enroute to ZZZ3. I sent a text to the new operating Captain of what I smelled. No response (I'm not even sure I had the right phone number). I asked that Maintenance Controller if they could inspect the plane for APU oil contaminating the bleed. I called back about four hours later as the plane was on it's RON (Routine Overnight) in ZZZ3 and spoke to a different Maintenance Controller asking if anyone would inspect the plane for this issue. The reported no one had and made only abstract hints that they may have someone look into it. The plane was to fly again at XG:00 AM. I had a XA:30 wake up in ZZZ1 for my XC:30 AM departure. I called Maintenance Control a third time, speaking to a third different Maintenance Control. I once again inquired if this issue had been looked into or documented. Again nothing. I did get the flight number of the departing flight so I could look up the Captain. I sent the next operating Captain an email about what I smelled yesterday and my confidence that the bleed was contaminated. I called the Duty Officer before I departed to ZZZ4. I encouraged the Duty Officer to get Maintenance to inspect the plane; I told him I will gladly "eat crow" if I am wrong. He told me that while it wasn't within his scope to tell Maintenance what to do; he would contact the next operating Captain as well, encouraging his vigilance around a possible dirty sock smell. Thankfully, this is where the safety issue finally got trapped. The Captain of the plane supposed to depart ZZZ3 reported to me that he turned on the APU bleed, it smelled like the dirty sock smell, and he wrote it up. I learned by following up with Maintenance Control later that the plane was pulled off line for a couple days and they did in fact find oil in the APU bleed duct, and had to decontaminate the system. I am writing this narrative a week after the incident. It took a week after this incident to gain a better perspective and reframe the significant anger I had at my fellow Captain. He isn't the problem. His dismissiveness of me and his A FA are only symptoms of a much bigger cultural problems about this issue. We do not have a culture that is committed to stopping operating with a questionable air quality issue. If I told the Captain there was ice all over the wing; he would 100% ensure that he did not operate the plane with ice on it. While this issue should be every bit as binary as departing with ice on our wings; if one hasn't ever correctly correlated this smell with Maintenance positively finding oil in the bleed duct; one would be reasonably reticent to ground an airplane over this. I will further add that the intensity of the smell varies, and "dirty socks", while very close in character to the smell, doesn't quite nail it. The smell is unique. And the smell can vary depending on whether the oil is burning. It is also a smell that almost any crew member who has flown for decades has smelled before on otherwise unremarkable flights. It will not raise psychological alarm bells unless it happens to be an outlier case (severe contamination), or the crew has had experience correctly identifying this in the past. I will say that the first time I wrote up a plane for this I felt very nervous that I would be wrong, and would be judged as being overly vigilant. I am sure that this plays a factor into why more people will not write it up. No one wants to be "the guy" who writes up a plane that just smells only a little bit funny. Our brains are primed to come up with any other plausible reason why we might be smelling this smell, e.g., humid musty air outside the airport. My concern as to the process and procedures here are that I, a fully qualified pilot, who told each person that I have correctly identified this issue five times before, expressed my concern to Maintenance Control on the ground in ZZZ. A fellow pilot (along with the A FA expressing the same concern). A Mechanic. A second Maintenance Controller. A third Maintenance Controller. A fourth Maintenance Controller. A Duty Officer who didn't feel empowered to invoke the safety pause to inspect it at his command, but did at least feel empowered to advise the incoming Captain. The plane operated from ZZZ to ZZZ1 with air contamination (and who knows how many legs before hand). The plane then operated from ZZZ1-ZZZ3 with air contamination after a Pilot and Mechanic were advised of the issue. The error chain may have stopped in ZZZ3 without the Duty Officer and myself contacting the next operating Captain. It is unknowable to me whether or not he would have written it up had we not "primed the pump" for him. My recommendation is we must change the culture around the reporting of smells and the empowerment of our crews to encourage proper responses. Furthermore, given the difficulty of correctly identifying the smell, crews who have been operating contaminated planes should be notified after the fact so that they may have the opportunity to reflect on whether or not they smelled anything that their noses may be trained so that they may be more vigilant in the future. Last but not least. We must have detection. Not simple CO detectors, but detectors that indicate whether we have the neurotoxic chemicals present in the cabin. Detectors that must advise the crew in these situations, not merely advising Maintenance on an inspection. We can do better. Let us not let fear of liability stop up from appropriate action on this issue.

Narrative: 2

On Flight XXX ZZZ-ZZZ1 I was approached before the flight by a Deadheading Crewmember and he said the airplane "smells like dirty socks". My FO (First Officer) and I proceeded onto the aircraft and did not notice any unusual smell. While I performed the walk-around I did notice the musty, salty smell in ZZZ from the ocean air. Inside the aircraft we had that same musty smell because the door was open. We proceeded to ZZZ1 and had no issues, no smell, no complaints from passengers. This is a record of flight XXX from the operating crew.

Synopsis

Air Carrier Deadheading Captain reported a communications breakdown between DH Captain and Operating Captain regarding APU bleed duct contamination. The APU bleed duct contamination was identified and corrected by Maintenance a week after the flight.