ACN: 1949038 (501 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Ceiling.Single Value : 1500

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1949038
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Flying the ZZZZZ arrival in ZZZ to the ILS XXR, the Pilot Flying (PF) and Pilot Monitoring (PM) oversped flaps on a go around after an unstable approach. Weather was approximately 1500 ft. ceiling with strong tailwind at altitude during beginning of the approach, which switched to a headwind at approximately 2500 ft. Initially ATC told to expect XY, then toward the end of the STAR switched to XXR. ATC kept us at/or above 10000 ft. a little longer than expected and 210 kts., faster than desired. After being cleared for the approach with a high crossing restriction of 10000 ft., we had to attempt to configure early and attempt to capture the glide slope from above. The tailwind kept us high and fast. It became obvious we would not be stable and began talking about a go-around. Around that time, the autopilot began to capture the glide slope from above and the aircraft began to aggressively dive. We received a momentary GPWS "sink rate" warning. We immediately executed a go around. Due to the weather factors and startle effect, climbing away from the terrain became the top priority and airspeed quickly increased before the flaps could be retracted before they were oversped by about 5-10 kts. We stabilized the aircraft and successfully performed another ILS approach to XXR and landed. Initially the First Officer (F/O) was PF for the approach, but during the start of the go-around the Captain (CA) stated "my aircraft" and took control of the plane until the aircraft was stabilized after the go-around. The aircraft was then turned back over to the F/O for the subsequent approach and landing. Ceilings at 1500 ft. High altitude airport. Inexperience with this airport. F/O with 50 hours. CA did not point out recommended techniques quickly enough on approach. Excessive tailwind during initial phase of approach. ATC high crossing restriction and required speed of 210 kts. until attempting to capture glideslope. Inadequate go-around procedures by F/O required CA to take control of the aircraft. GPWS alert startle factor caused F/O to delay PM duties during go-around. ATC at high altitude airports should not impose excessive height or speed restrictions. Company recommended configuration, altitudes, and speeds at high altitude airport.

Synopsis

A320 flight crew reported an unstabilized approach, go-around procedure, and a subsequent flap overspeed.

ACN: 1948980 (502 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Snow
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Person

Location Of Person : Company
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 748
Experience.Flight Crew.Last 90 Days : 176
Experience.Flight Crew.Type : 748
ASRS Report Number.Accession Number : 1948980
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

The weather in ZZZ1 on Day 7 was showing light snow all day with visibility forecast to go down to 1.5 sm and winds shifting to 360/08 starting at XA00/Day 8. Our arrival for Aircraft X/Day 7 was scheduled for XC50 on Day 8. I called Dispatch at XV37 (3 hours and 40 minutes before departure) to give them a heads up that the forecast was going to be a problem due to the fact that the alert effective on Day 7 for ZZZ1 (indicating that a new 5G cell tower in ZZZ1 was not currently covered by an Alternate Means of Compliance (AMOC) and the RNAV (RNP) Z Runway X was prohibited.) combined with the low visibility would not permit a landing to the north because of the increased visibility requirements of the available approaches (>3sm). The Dispatcher had not looked at the situation since the flight was still almost 4 hours out. We discussed the issue with the wind and visibility, and the possibility of landing to the south with an 8 knot tailwind. I told the Dispatcher that I would not be comfortable landing in ZZZ1 at night with low visibility, snow, a strong tailwind and reduced braking action. The Dispatcher said that he would discuss the issue with the Dispatch manager. I did not hear anything from Dispatch or see a flight plan so I drove into work. I also noticed that my original First Officer (FO) had been removed from the trip and a reserve FO assigned to do the ZZZ-ZZZ1-ZZZ turn. When I got to the airport parking lot, I noticed that there was a new flight plan and reviewed it. The Dispatch release showed that they were planning on landing ZZZ1 Runway X using the RNAV (RNP) Z XX. I called the Dispatcher to inform him that the release was illegal due to the alert prohibiting use of the RNAV (RNP) Z Runway XX. The Dispatcher seemed unaware of the alert and only seemed concerned with whether both the Captain and FO were qualified to do RNP approaches. Once I alerted him again to the alert, he agreed that the Dispatch was not legal. He said that he would speak with the duty manager to discuss options and call me back. The Dispatcher eventually called back to say that the flight would probably be cancelled, but the duty manager wanted to know if I would be agreeable to dispatching with enough fuel to fly to ZZZ1, see if the weather would support shooting the RNAV (GPS) XX Runway X once we arrived, and if not, diverting back to ZZZ1. I told the Dispatcher that I would be okay with that plan if they could build a legal dispatch with as much fuel as possible to give us divert options. The Dispatcher said that he would work on it and get back to me. Eventually the Dispatcher called me to let me know that they could not build a legal Dispatch and that the flight was canceled. After the flight was canceled, I found out that my original FO had been removed from the ZZZ-ZZZ1-ZZZ turn and reassigned because he was not RNAV (RNP) qualified. So it appears that the Dispatcher misinterpreted my original call (3 hours and 40 minutes before departure) as a concern about my FO's qualification to do the RNAV (RNP) Z Runway XX, as opposed to my actual concern that the RNAV (RNP) Z Runway XX was prohibited due to an alert.

Synopsis

Air carrier Captain reported a discussion with Dispatch regarding an approach procedure made illegal due to 5G and a misunderstanding with flight crew qualifications.

ACN: 1948854 (503 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1948854
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Boarding the aircraft, the First Officer (FO) immediately mentioned a musty/oily smell. I noticed it also. I would call it a 1 out of 5 in intensity. 5 being the strongest. I started the APU and when the FO returned from the pre-flight inspection he noticed the smell was stronger as did I. At this time, the Flight Attendant No. 1 (F1) came into the cockpit and voiced her concerns over "the smell" too. By this time, the smell was intensifying into a 3 out of 5. We immediately called Maintenance and turned OFF the APU bleed. The smell did not dissipate after turning off the APU bleed while we were present. Maintenance arrived at the aircraft within minutes and confirmed the smell. I immediately informed the Gate Agent and the F1 to de-board the passengers. The F1 relayed to us a few passengers mentioned "the smell". These events happened quickly and in unison. The crew and passengers de-boarded and Maintenance took command of the aircraft.

Synopsis

A320 Captain reported a "musty/oily" odor during preflight. The odor became stronger which prompted the Captain, in agreement with Maintenance, to secure the APU and deplane passengers.

ACN: 1947893 (504 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1947893

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout of ZZZ First Officer asked me if I smelled an odor. I said that it reminded me of left over de-ice fluid. To me it was a very week smell and dissipated shortly into the climb. I did not notice it after that. Inflight never mentioned anything to me either. After landing first officer ran into the next Captain and updated him on status of the MELed APU and told him to keep an eye out on climb out for the possible smell. Previous Captain that brought the airplane to me briefed me about the APU being MELed but never mentioned any odors.

Synopsis

A320 Captain reported a weak "de-ice odor" on departure which dissipated shortly into the climb with no further issues noticed.

ACN: 1947890 (505 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 13500

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Climb

Component : 1

Aircraft Component : Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1947890
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When I arrived at [aircraft] to operate flight, I was greeted with the information that the aircraft had experienced dirty sock odor on climbout and descent. It was brought to my attention by a member of the in-flight crew. I proceeded to write-up the occurrence in the logbook and hand it over to Maintenance. Maintenance ultimately performed a dual engine isolation test and cleared the aircraft for return to service. No odor issues were detected until we were on descent into ZZZ. At approximately 13,500 feet, the cockpit filled with the dirty sock smell and slowly dissipated. Then again at approximately 2,000 feet while on the RNAV XXL into ZZZ the cockpit again filled with another blast of foul air (same smell) and slowly dissipated. This aircraft had been written up for an odor event on date 1 (I believe) and the APU placed on MEL and had now encountered two back-to-back events. I wrote up the aircraft upon arrival in ZZZ and ZZZ Maintenance Control was notified.

Synopsis

A320 Captain reported a recurring "dirty socks" odor during descent. In addition to submitting a logbook discrepancy, the Captain enumerated the recent aircraft fume events to Maintenance.

ACN: 1947881 (506 of 2908)

Time / Day

Date : 202211
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Landing
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1947881

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During landing, we were notified by Inflight that a strong cheese odor was smelled throughout the cabin. We landed and were told that the smell persisted. We did the checklist and parked at the gate. A MX (maintenance) logbook entry was made. I informed Dispatch by phone call. No physical effects were reported.

Synopsis

A320 Captain reported a persistent "strong cheese odor" during approach. Upon arrival a Maintenance logbook discrepancy was submitted.

ACN: 1947055 (507 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1947055
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fumes event in the cabin reported after blocking in by inflight. Started mid cabin and eventually in the rear.

Synopsis

A320 Captain reported being notified by a Flight Attendant of a fumes event in the cabin after gate arrival.

ACN: 1946936 (508 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class C : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 144
Experience.Flight Crew.Type : 202
ASRS Report Number.Accession Number : 1946936

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were in our initial descent into ZZZ when we received an ECAM for a GRN HYD ENG 1 LO PR. We had fault lights illuminated on the GEN ENG 1 and PTU (Power Transfer Unit) switch. We ran the ECAM which directed us to switch both the GRN ENG 1 and PTU off. The status page listed all the inoperative systems. It also stated that we should attempt to restore GRN ENG 1 to extend the gear and if unable a gravity extension would be required. We then referenced the FM non-normals for review, reviewed the gravity extension and computed the landing distance required. The Captain [advised] Center and requested direct ZZZ. We had XXX [passengers and crew] and around 1:45 minutes of [fuel]. Next the Captain made contact with my Dispatcher through ARINC. He explained the situation and his intentions to land at ZZZ1. We set the aircraft up for a 20 nm final, turned the GRN ENG 1 pump back on and selected gear down. The gear extension was slower than normal but we did receive a 3 GREEN and locked indication. The GRN ENG 1 was switched back off. We touched down in the first 1000 ft. of XXL and came to a stop with 1500 ft. remaining. We ran an after landing flow, then referenced the FM Supplementary section for Tow-in procedures. After shutting down the engines the Fire department informed us we were leaking hydraulic fluid out of the nose wheel area. End of incident, the Captain notified all required authorities.

Synopsis

A320 Air carrier First Officer aircraft reported Hydraulic System Malfunction on descent.

ACN: 1946224 (509 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 26000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : VFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1946224
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X Immediately after takeoff I noticed a faint smell of dirty socks which only lasted a few seconds. The First Officer (FO) did not notice the smell. When we began our initial descent into our arrival airport, I believe we leveled off around 26,000 ft., the First Officer asked me if I smelled something. Coincidentally, I noticed the smell at the exact same time he said something. He either smelled it first, or I did not realize I smelled something until he brought it to my attention. We were then given another descent and the odor dissipated before we had time to apply the appropriate procedures. In the descent, we had time to open our manuals to the fume removal procedure as we anticipated possibly another instance on level off. We leveled off around 11,000 ft., but we did not encounter any more odors for the remainder of the flight. Upon arrival at the gate, the First Officer felt dizzy, lightheaded, and possibly had a few other symptoms. The cabin crew did not encounter any indication of an odor in the cabin. I filled out the paper air quality form maintenance personnel requested and I also filled out the digital online form, I notified Dispatch, and I also notified the Chief Pilot on duty per our safety reporting requirements. The online form would not allow me access to the description section. I tried to login to the form twice however, each time I was unable to fill out the description section. I submitted it with the information that it allowed me to input. The First Officer went on to seek an evaluation due to his symptoms.

Synopsis

Air Carrier Captain reported an intermittent fume event during initial climb and again during destination descent which eventually dissipated and did not return. At gate arrival the Captain submitted company reports and First Officer was treated for physiological symptoms.

ACN: 1946067 (510 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Cowling
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1946067

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While on approach to land in ZZZ, we heard a noise that we thought might have been a bird strike. We didn't see any bird but thought it was a possibility. After arrival at the gate, the First Officer performed a post flight walk around inspection and didn't see any indication of a bird strike but noticed that a panel on the inside of the number two engine pylon was missing. We immediately notified ATC and called Maintenance Control. I wrote a logbook entry for both the missing panel and a possible bird strike. I assume that the noise we heard was the panel popping off.

Synopsis

A320 pilot reported missing engine pylon panel following post flight inspection.

ACN: 1946062 (511 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb

Component : 1

Aircraft Component : Rudder Trim System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autoflight Yaw Damper
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1946062
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Report late due to IT issues. "Repeat Writeup". Day XA00Z. On climb out passing FL100 at 280 knots starting getting vibrations in cockpit which continued up to FL320 and 300 knots. The vibration events ranged from mild to medium throughout the climb. Initially we chalked it up to rough air but the N1 (.7) on the ECAM had us thinking otherwise. We received 3 calls from the cabin saying the aircraft was swaying back and forth (left/right) and was getting very uncomfortable. Later we were told by inflight that it was from row 10 and towards rear of plane. I made a call to Dispatch/Maintenance explaining the situation and we decided it would be best to RTB which was uneventful.

Synopsis

Pilot reported an air turn back due to excessive airframe vibration after take off.

ACN: 1945095 (512 of 2908)

Time / Day

Date : 202210

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Route In Use : Vectors
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1945095
Human Factors : Communication Breakdown
Human Factors : Confusion
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

There are multiple issues that occurred on the flight all impacting safety. Rather than file multiple reports include them all in one. As a background the flight crew arrived after a minimum rest delayed overnight that resulted in a XA:45 AM hotel arrival after a stressful duty day 10 minutes shy of max legal duty limit. both flight crew members or fit to fly, however I'm going to state that fit to fly and well rested are two entirely different things. As a side note this was a crew base with an opportunity to re crew. The first item of consideration is that printed with the release was a page noting that we are not authorized to fly the Runway XX RNAV visual in ZZZ. This is in contradiction to the email dated Date 0, which authorizes us to shoot the approach. It takes time, effort, and concentration to coordinate and come up with a true legitimate answer in regards to our flight operation authorizations which seem to change frequently and that are not consolidated in one spot but spread over many many documents. A side note on this point is that the ACARS also generated a warning as we were descending towards the high workload airport in the mountains which told us that again we are not authorized to use the RNAV V XX. The weather was marginal with a wet runway and wind 9 kts. Essentially right down Runway XX. Clearly XX was favored. The second safety concern involves an MEL item. Out of diligence I always trace back in the logbook to see what trends the aircraft might have had or things that may have previously been problematic. This aircraft had been written up for a leaking hydraulic line On the number 2 brake. It had previously been deferred which involves an additional anti skid deferral and an aircraft landing capability downgrade. Prior to us excepting the aircraft it had been repaired signed off and returned to service with a clean slate and no deferrals. It had additionally been written up since that time for a discrepancy between the number one and number two brake temperatures. That was also signed off and Operations checked good. After landing in ZZZ during taxi the number one brake temp Reached around 330 degrees, number two was 30 degrees and three and four were fairly equal at around 270 degrees. That discrepancy requires a write up per AOM volume two. When Maintenance arrived It was discovered that the number 2 brake Had never been hydraulically reactivated. The Forward line was unplugged taped off and safety wired together essentially adjacent. we flew an airplane with a deactivated brake with no knowledge of that or consideration for the performance into a mountainous airport on a wet runway. Maintenance in ZZZ reconnected the line and we continued on our trip. Clearly this is the item that could've been caught on preflight however I will say that the manner in which this was wired together would make it very difficult to catch, along with the fact that we had multiple other issues which required maintenance prior to departing such as loose fasteners etc. The workload generated with these combined distractions and after a very short night. There's no way other to say it then it is an incredible amount of distraction and attention diverted from the task at hand.

Synopsis

Captain reported the #2 main brake was never reactivated after corrective action and removal of an MEL. This was discovered after landing at destination airport.

ACN: 1944768 (513 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Takeoff / Launch

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1944768
Human Factors : Physiological - Other
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

From takeoff to 8/10,000 ft. I was in my jumpseat. I fumbled with shaking, tingling hands to undo my seat belt as a wave of nausea [and] dizziness hit me. I struggled to stand and get myself to the front of the plane. The FA (Flight Attendant) A, who has an emergency medical background, assessed that my pulse was racing. My fingers were cold and tingly, as were my lips. I was sweating and still dizzy, nauseous, and starting a headache. The exact same symptoms the 2 other times I have had an air quality event as the FA B on the Airbus. Both other times I went to the hospital. A 3rd event did not send me to the hospital. After maintenance was performed, all 3 events were deemed to have been caused by a faulty APU. I have experienced and heard of too many air quality problems on the Airbus. What is going on?

Synopsis

A320 Flight Attendant reported an air quality event from takeoff through 10,000 ft. which caused multiple physiological symptoms. Flight Attendant has experienced the same symptoms previously on other Airbus aircraft.

ACN: 1944161 (514 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component : 1

Aircraft Component : Main Gear Door
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nose Gear Door
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1944161
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1944158

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On approach, just after gear extension, received a "LDG GEAR" ECAM, showing the gear was down, but the gear doors were stuck in the open position. There was also a "STEERING" warning. I broke off the approach and continued over the airport, asking ATC to visually confirm if the doors were open. They so confirmed. We requested delaying vectors while we worked through the ECAM and QRH checklists and attempted to contact Dispatch and/or Maintenance to consult with them. Using all available methods, we were unable to make contact (until later, on the descent into ZZZ). After completing the checklists - and taking note of the tremendous fuel burn associated with flying with the gear doors open - we reviewed our options. We did not think it advisable to land in our present configuration, nor to attempt landing at ZZZ1, when ZZZ was close by. ZZZ has much longer runways, better CFR (Crash Fire Rescue) and more available medical facilities, should they be required. I elected to [advise ATC] and requested a diversion to ZZZ. Upon that decision being made, we raised the gear, climbed to 10,000 ft. and proceeded to ZZZ. Although the gear retracted normally, the "STEERING" warning remained, which indicated to me that the problem, whatever it was, was not entirely resolved. I briefed the passengers via the PA, and told the FAs (Flight Attendants) to prepare for a precautionary landing in ZZZ. On the approach into ZZZ, we re-extended the gear and as hoped, it extended normally. The "STEERING" ECAM also disappeared after gear extension. ECAM "Recall" then indicated "Normal". We had planned to fly by the ZZZ Tower for visual inspection, but with the normal indications, we asked Tower for a visual check while we were still on approach and Tower said that all appeared normal to them. We elected to proceed with the landing and were cleared for the option. Landed normally. Nosewheel steering worked normally, so we cleared the runway and had CFR look the aircraft over to make sure there was no visible damage. There was not, so we taxied to the gate. ZZZ Maintenance met the aircraft and electronically confirmed the previous gear malfunction. The aircraft was taken out of service by ZZZ Maintenance. Mechanical and/or electronic failure, possibly transient in nature I would like to know why the "Crew Phone" did not work on this aircraft; why we were unable to contact Dispatch or Maintenance Control.

Narrative: 2

ECAM landing gear down door unlock and steering inop. [Requested priority handling with ATC]. Diverted to ZZZ.

Synopsis

A320 flight crew reported diverting to an alternate airport after experiencing a landing gear door malfunction.

ACN: 1944012 (515 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.AGL.Single Value : 900

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1944012
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On approach to Runway XXR we got a vector from the arrival onto final. Slam dunk and inside the FAF. Aircraft was not fully configured or stabilized. We were cleared for the visual, first officer rolled in 2000 because he was past the FAF and wanted to capture the glide slope profile. He had a descent rate of 2000 fpm, we then over shot the centerline and Tower alerted us to a low altitude warning. We were visual and had a clear sight of the runway but at this point we're below the glide path. Upon correcting and on extended centerline we had not completed the landing checklist. We were at 900 ft. AGL and I called the go around. We got the vectors as expected and returned back around for a stabilized approach and landing.

Synopsis

A320 Captain reported unstabilized approach and go around.

ACN: 1943727 (516 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1943727
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X. Upon landing at ZZZ the number 2 brake temp indicated 30 degrees while brake 1 was well over 300. This required a write up. It was subsequently discovered that the brake line for brake 2 had been left disconnected from a deferral that had been cleared. Maintenance fixed the original issue (replaced rear brake line on brake 2) but never reconnected the front line which I assume was disconnected per the deferral. I did not notice it on the pre flight walk around but it was wired in place which may have made it appear connected. In summation, we went down to ZZZZ (rainy, mountains, ungrooved runway) with 3 brakes.

Synopsis

First Officer reported brake temperature issues when returning from a foreign airport. It was discovered that the #2 brake had been deactivated and never was reactivated as required.

ACN: 1941593 (517 of 2908)

Time / Day

Date : 202210

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 8000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class E : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1941593
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft X. ZZZ-ZZZ1. During approach to XXL at 8,000 ft., cleared for the approach with the APPR psb armed (right around ZZZZZ), we received an ECAM for Engine 2 EGT over limit. Looking at the EGT, we had both recognized that it was in the red over 800*C. Captain commanded ECAM actions to which I had began running. It had us reduce engine 2 thrust to idle to reduce EGT which caused no change and required us to shutdown engine 2. The ECAM elected us to select engine 2 master off. Captain increased engine 1 thrust lever to MCT. Systems pages showed, to which we went through each to see what systems were effected, followed by reading the status page. Eng anti ice went on and the APU was started. After ECAM actions were complete a request for priority handling was made to ATC and a hold was requested over ZZZZZ1 at our current altitude of 8,000 ft. by Captain as I went into COM 2 to run the procedures for ENG 2 EGT over limit and ENG 2 shutdown. Getting set up in the hold at ZZZZZ1 at some point our altitude became uncaptured putting us in v/s mode. We had been -250 originally and +250 during the first turn before getting the airplane to capture 8000 ft. on the outbound leg. While I was running the COM Captain called the lead Flight Attendant to give a brief, sent an ACARS message to Dispatch, contacted ZZZ1 ops, took ATC commands and requested emergency vehicles to stand by for landing. The COM instructed for the cross bleed switch to be in the open position if performance permitted. I should have kept this in auto due to high terrain for the possibility of a go-around. After the COM was complete I had taken over ATC communication and requested our approach for the ILS XXL. As we started the approach I told the passengers we would have them on the ground in approximately 5 minutes and to remain seated with seat belts fastened. Upon landing, "spoilers" "reverse green" and "decel" were called. "70 kts." was called. Upon coming to a stop simultaneously I informed ATC we were stopping on the Runway while Captain set the parking brake and called for "flight attendant stations". The firetrucks did an inspection to which nothing abnormal was found. Captain announced "remain seated remain seated remain seated". I then asked ATC for taxi instructions back to the Gate. Emergency vehicles followed us to the Gate. Once parked and the jet bridge pulled up we were met by a Firefighter and Maintenance to which a logbook entry was made and phone calls made to both dispatch and a Chief Pilot.

Synopsis

A320 First Officer reported receiving an ECAM for Engine 2 EGT over limit. A go-around was performed and the flight crew ran the QRH and check lists. Priority handling was requested, the affected engine was shut down and a precautionary landing was performed at destination airport.

ACN: 1941554 (518 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use : Vectors

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1941554
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the takeoff roll I noticed it was difficult to keep the aircraft on center line. It was yawing to the left. I could feel resistance on the peddles and I thought it may have been the Captain's feet. I found out later it was not him. Right after bringing the gear up on takeoff, we got an ECAM caution message about the nose wheel steering. I can't remember the exact message. I called 'my aircraft' and focused on flying and radios while the Captain ran the checklists. We determined that the nose wheel steering was INOP. But we didn't know the position of the gear, whether straight or at a 90 degree angle. We wouldn't know until we lowered the gear. Either way, we made the decision to request priority handling and make a full stop landing in ZZZ. We requested ARFF be waiting for us. The Captain made the appropriate calls to Dispatch and Maintenance. He alerted the Flight Attendants and made the PAs to the passengers. I requested priority handling with ATC and gave them the requested information. It was a very short flight so things were moving fast. We decided to get the gear out early to see if there were any additional ECAM messages. Thankfully, there were not. We determined the nose gear was straight and that we would make a normal landing to a full stop on XXL in ZZZ. We touched down and came to a stop without incident. Crash, Fire, and Rescue was standing by. They inspected the aircraft for damage and found none. We handed them the gear pins and a push crew towed us to the Gate.

Synopsis

A320 First Officer reported on the takeoff roll they noticed it was difficult to keep the aircraft on center line. The flight crew elected to continue to destination airport, requested priority handling and made a precautionary landing.

ACN: 1941323 (519 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1941323
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While sitting in the airplane at the gate we noticed an odd odor. After we had finished boarding all the passengers we closed all the doors. The ramp was closed due to rain and lightning. We sat for over 20 minutes at the gate and the odor got stronger to the point it was unbearable. Flight Attendant (FA) in the back called the flight deck to say that the whole airplane smelled really strong with a certain odor. I called maintenance and operations to inform them. Maintenance advised me to deplane, due an odor report and put it in the logbook. Gate agent took several minutes to bring the jet bridge back to the airplane. I called operations again to get an ETA on the gate agent. Few minutes later he showed up and we deplaned.

Synopsis

A320 Captain reported a strong odor during a ground stop due to lighting. After conferring with maintenance they decided to deplane and submit a logbook entry.

ACN: 1940699 (520 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Seatbelt
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1940699
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.General : Physical Injury / Incapacitation
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

About 15-20 [minutes] prior to push back sitting in the flight deck there was a loud "crack" sound which sounded like a cart had fallen over. Soon after we smelled spent gunpowder up front. I went to check and observed Person A (B Flight Attendant (FA)) assisting guests in row X. He looked at me and said that the passengers had been injured in that row as the airbags in those seats had inadvertently gone off. I instructed the First Officer to 1. Call Operations and send EMS immediately and 2. Contact Maintenance to send a person/team to see what could be done in this situation. I made an announcement regarding the situation and that we were aware of the connection status of a number of our guests and would be looking at ways to mitigate delays for them. As the injured passengers were being attended to by EMS a decision was made to remove the aircraft from service. The entire FA team should be commended for their quick and cool response. Person A (B) immediately rushed to assist the guests while the A and C asked everyone to remain out of the aisles. Person B was perfectly clear as to what he needed..."Three injuries...EMS...NOW!" That's exactly what works and the team should be commended for their quick and calm response. Maintenance made the right call to remove the aircraft from service as it could have been an installation issue or manufacturer problem, etc. We won't know until the seats are inspected in detail.

Callback: 1

Reporter stated he is unsure of the cause of the airbag detonation. He stated all three airbags in the row detonated, and with enough force that the trim surrounding the window at that aisle was damaged. Passenger injuries were limited to minor skin irritation and all passengers continued on the flight on another aircraft. The Maintenance Technician working the problem theorized that perhaps the g-meter beneath the seat was triggered by the passenger in the row behind the incident possibly shoving a carry-on bag beneath the seat in front of him with excessive force.

Synopsis

A320 Captain reported air bag style seat belts misfired on the ground, injuring three passengers. The aircraft was removed from service for evaluation and repair.

ACN: 1940302 (521 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors

Component : 1

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System Pump
Manufacturer : EDP1
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1940302
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of ZZZ at approximately 13-14,000 ft., we experienced a HYD G ENG 1 PUMP LO PR ECAM. Name ran the QRC then we divided our duties. As I started to run the LO PR ECAM it disappeared. I went to the HYD page and saw the G HYD fluid was low. Less than a minute later we received the HYD G SYS LO PR. I ran the ECAM, accomplished the QRH and contacted Dispatch and Maintenance. I discussed the issue with Maintenance and from their end he could see the the HYD G SYS LO PR. We agreed all the procedures were accomplished and discussed whether to continue or return to ZZZ. Maintenance stated that nowhere in the ECAM does it say land ASAP and it would be safe to continue on. Dispatch agreed the weather in ZZZ1 would be fine at arrival and I also agreed it was safe to continue. Name and I talked about what we need to do as we got closer to our destination. We ACARS'd dispatch that we would need to be tugged off the Runway for lack of NWS. The decision was also made that we would [request priority handling] with ZZZ Center so ZZZ1 can prepare for our arrival. We discussed when we would extend the gear and talked about the other Inop Sys associated with the G Hyd. I called the Flight Attendant 1 to the cockpit to explain the issue and what to expect. I gave her the alert and she was to tell the other inflight of the issue and to contact me with any questions. We requested priority handling with ZZZ Center. In the descent we informed the customers that we would be stopping on the Runway to remain seated till we were at the Gate. The ZZZZZ arrival was uneventful and we were given Runway XXL. We requested a long final to manually extend our gear and finish our remaining checklists. The landing was uneventful We stopped on the Runway and crash fire rescue met us at the plane. We were talking to our tug in minutes, engines were shut down and we were tugged to the Gate. Maintenance meet us at the plane and they were given a debrief.

Synopsis

A320 pilot reported av HYD G ENG 1 PUMP LO PR ECAM in flight. This was followed by a HYD G SYS LO PR. The flight crew elected to continue the flight and make a precautionary landing at destination airport.

ACN: 1940178 (522 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Takeoff / Launch
Route In Use : Vectors

Component : 1

Aircraft Component : FADEC / TCC
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Indicating and Warning - Lighting Systems
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Powerplant Fuel Valve
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 1369
ASRS Report Number.Accession Number : 1940178
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 4905
Experience.Flight Crew.Last 90 Days : 181
Experience.Flight Crew.Type : 841
ASRS Report Number.Accession Number : 1940195
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Aircraft X, ZZZ-ZZZ1 holding short of [Runway] XXR, we received a COND TRIM AIR SYS FAULT ECAM as a repeat write up. While the CA (Captain) was discussing with Maintenance, the fault cleared and we deemed it was safe to continue and were then soon cleared for takeoff. On the takeoff roll around 80 kts., we received an ECAM stating the takeoff thrust disagreed. Both engines were indicating normal so I elected to go into TOGA and continue the takeoff roll. After a normal rotation, we received numerous ECAMs including ENG 1 SHUTDOWN, ENG 1 FUEL VALVE FAULT, FUEL VALVE NOT CLOSED, ENG 1 FUEL VALVE OPEN. All engine indications were still normal except the engine 1 fuel valve was indicating amber but open on the systems page. Upon climb out, we [requested priority handling] with ZZZ and elected to return to the field. We then handled the ECAMS while the CA notified Dispatch and the FO (First Officer) briefed the FAs (Flight Attendants) and passengers. We set up for a visual approach to XXC and landed uneventfully and under max weight. After clearing the runway, local AARF inspected the aircraft and noted no leaks or damages. We then taxied to the gate and shutdown. Upon shutdown we noted FADEC 1 and ENG 1 EIU class two maintenance messages were displayed.

Narrative: 2

Aircraft X, ZZZ-ZZZ1, was scheduled for a XB06Z (XA06 local) departure. Out time was XA59Z and the aircraft had an uneventful startup and taxi to Runway XXR. CA (Captain) was PF (Pilot Flying) and FO (First Officer) was PM (Pilot Monitoring). At the hold short, the aircraft incurred a COND TRIM AIR SYS FAULT ECAM which was a repeat gripe from the previous two flights. Tower was notified to delay as Maintenance was called. The ECAM cleared while the CA was on the phone with Maintenance, however, and the aircraft was determined to be safe for flight. Tower issued clearance for takeoff, Runway XXR at XX. During the takeoff roll at approximately 80 KIAS, there was an ECAM indicating that the engine takeoff thrust disagreed. Since all other engine indications were normal, PF set TOGA and continued the takeoff. The rotation was normal but during the climb out, the aircraft displayed ENG 1 FUEL VALVE FAULT and FUEL VALVE NOT CLOSED as well as ENG 1 SHUT DOWN and FUEL ENG 1 LP VALVE OPEN. All engine indications however including EPR, fuel flow, N1 and N2 RPM, and EGT were normal. The auto thrust system was not operating so the CA took over manually. He also noted that the SRS indications were still indicating steep pitch up and elected to use selected speed. The crew continued the climb out, cleaned up the aircraft on schedule, and at a safe altitude and airspeed, handled the ECAMs. The number one engine fuel pressure valve on the Fuel Page indicated amber but in the open position and the PFD displayed "Set Green Dot". All other indications, other than the ECAMs and the auto thrust, were normal. The crew elected to [request priority handling] and return to ZZZ. The CA notified Dispatch while the FO notified the FAs and the passengers. The CA set up and briefed the approach to XXC. The aircraft was below max landing weight. The FO continued to watch the fuel system page and engine performance. Fuel stayed balanced and both engine indications remained normal. The CA landed the aircraft and during rollout, the Engine 1 reverser was inoperative. After clearing the runway, AARF conducted a quick visual inspection and reported no leaks or damage. The aircraft was taxied back under its own power and shutdown uneventfully. Upon shutdown, FADEC 1 and ENG 1 EIU class 2 maintenance messages were displayed.

Synopsis

A320 reported multiple engine system malfunctions after takeoff. The flight crew requested priority handling and performed an air turn back and precautionary landing at departure airport.

ACN: 1940032 (523 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1940032
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X. Green hydraulic pump failed at 32,000 ft. while descending for ZZZ. [Requested priority handling], requested fire trucks, landed safely and taxied to gate.

Synopsis

A320 Captain reported loss of the Green Hydraulic System Pump on descent. The flight crew continued and landed at destination airport.

ACN: 1939919 (524 of 2908)

Time / Day

Date : 202210
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Electrical Distribution
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 9200
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 740
ASRS Report Number.Accession Number : 1939919
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We pushed back and during engine start at 25% N2 the aircraft went completely dark and we were left with only one transmitter. This is not the correct electrical configuration that should result from an APU failure. All screens were black and we were left with just one transmitter/receiver. I called maintenance crew and they said to do a gate return. While waiting for the tug we ran through the manual and confirmed that the aircraft was not in fact reacting correctly to the shut down. We were unable to restore any electrical power. We returned to the gate and maintenance told us that the APU had some kind of failure. I insisted that even in the event of an APU failure, the aircraft should not go completely dark, and they just shrugged at me. I am certain the aircraft was very broken and the priority bus system was not working properly. So I refused the aircraft. The aircraft was later dispatched with an APU sensor repair, however, the priority electrical system was not inspected (to my knowledge), which was the reason I refused the aircraft. I was additionally told that no codes were stored due to the sudden power loss, so nothing that I was saying could be confirmed. I am not satisfied with how maintenance handled the pilot report and I am disappointed that this event was listed as a refusal instead of a maintenance issue.

Synopsis

A320 Captain reported refusing the aircraft after electrical system malfunctions were not addressed properly by Maintenance during a return to gate event.

ACN: 1937195 (525 of 2908)

Time / Day

Date : 202209

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 17306
Experience.Flight Crew.Last 90 Days : 192
Experience.Flight Crew.Type : 13903
ASRS Report Number.Accession Number : 1937195
Human Factors : Time Pressure
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff roll on [Runway] XXL experienced an engine 1 compressor stall at 40-50 knots. Rejected at approximately 50 knots came to a stop on the runway, notified Tower and ran the Reject QRC. Exited at [Taxiway] F1 and coordinated for a return to gate with Ground Control. Once at the gate debriefed maintenance and made a logbook entry.

Synopsis

Air carrier Captain reported a rejected takeoff due to a compressor stall on takeoff roll.

ACN: 1936906 (526 of 2908)

Time / Day

Date : 202209

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Pitot-Static System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Qualification.Other 
ASRS Report Number.Accession Number : 1936906
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After push back off of Gate XX, The flight crew on Aircraft X called the hangar controller radio stating they had an Anti Ice Standby Pitot AOA message. I reviewed the history of the aircraft and proceeded to have the crew do a circuit breaker reset for the Standby AOA. The fault cleared and the crew was informed to send an ACARS code when able, to log the reset into the history. At this time the crew did not mention they had made a log for this and would need a new Maintenance Release. When I previously had checked, this aircraft had a valid Maintenance Release. Maintenance later called to notify me that the aircraft departed with an open log and did not have a valid Maintenance Release due to this. The flight crew did send the code after take-off however (ELB log XXXXXXX) at XA:47 local, they took off at XA:14. During the time I was working with the flight crew, we were on the tail end of a busy bank of flights and also several arrivals calling inbound logs. My time was not uninterrupted as I was multi-tasking with other tasks simultaneously. I have never had an aircraft leave without a valid Maintenance Release before this event and believe there was a failure on my part to re-check the ELB and also on the part of the flight crew to not communicate the log was open and they needed a new Maintenance Release. Working solo in this position puts far too much pressure on one individual and creates situations where you are distracted in many directions (phone calls, radio calls, operations center chat updates, vendors, people in the operations center approaching you when you are dealing with aircraft on the radio) all can lead to a situation like this where you lose full concentration and then move onto the next task without the time to properly go over the current event and make sure every step of our process has been followed. Always having two or more hangar controllers working per shift so one is able to work without being distracted when working technical issues on aircraft off the gate. The number of distractions can be extremely high and takes away our ability to fully concentrate on one task at a time. Double checking the ELB prior to an aircraft departing after pushback.

Synopsis

Maintenance Controller reported confusion and interruptions caused a procedure to be left incomplete. This resulted in an aircraft departing with an open log page. The flight crew sent the required response after take off.

ACN: 1936664 (527 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 148
Experience.Flight Crew.Type : 3217
ASRS Report Number.Accession Number : 1936664

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

About 2 hours into the flight, in cruise at FL 350, we got the HYD Y RSVR LO LVL ECAM. Ran the ECAM. Contacted Dispatch via radio, who then patched in Maintence. Brake accumulator had bled to zero, and Y hydraulic level indicated zero, so I felt the safest course was to have ARFF standing by at the rollout end of the landing Runway. Dispatch [requested priority handling] and arranged for ARFF in ZZZ, stating the nature of the malfunction and that we anticipated a normal landing and rollout. Dispatch also notified station operations that we would keep ENG 1 running upon parking at the Gate until the aircraft was chocked. Maintence contacted Station Operations to coordinate maintenance. I briefed the Flight Attendants, electing not to do a cabin prep. We planned the long Runway, XX. Weather in ZZZ was CAVU. Approach, landing, and taxi in were normal. We advised ATC, upon clearing the Runway, that no assistance was needed.

Synopsis

Captain reported a HYD Y RSVR LO LVL ECAM in cruise. The flight crew continued to destination and landed normally.

ACN: 1936569 (528 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Test Flight / Demonstration
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Unknown
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1936569
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Our flight was a maintenance test flight being performed following the completion of a maintenance check. Shortly after leveling at FL310 and while accelerating to .78M we all noticed a burning smell followed immediately by observance of smoke emanating from right cockpit sidewall behind First Officerâ€™s seat. We all immediately donned O2 mask and I (as pilot flying (PF)) initiated left turn back to ZZZ. [We requested priority handling] and then FO and third pilot (Flight Test Engineer) began working the problem with the Smoke, Fumes, or AVIONICS SMOKE checklist. The problem seemed to resolve shortly after the first 6 steps of the checklist were performed. Smoke was no longer visible and odor was not detectable. [Priority] status was maintained in order to keep priority handling with ATC and RTB continued at best forward speed. Shortly after descending through 10,000 ft. the situation reoccurred this time with heavier smoke emanating from cockpit sidewalls on both sides. At this point we were being vectored onto downwind for XXR ZZZ. We requested immediate vectors to XXR and were offered Runway XX which we accepted as it was off our nose and less distance. Some S turns were performed on short final to lose altitude and airspeed followed by a full stop landing on Runway XX. By then smoke had once again dissipated. We opened cockpit windows and ran first 6 steps of Evacuation checklist stopping before Fire switches (Engines & APU) ...Push as engines had shut down normally and APU was never started. Fire Crew and Tower both reported no evidence of fire so we remained on Aircraft with windows open until Tug and Airstairs were brought to aircraft. We did leave batteries on for the purpose of radio communication with Tower. No additional info. The cause is unknown at the moment. Our recommendation for ourselves as well as other aircrews in the industry is to further train communications with O2 mask on. Things to emphasize would include volume levels of cockpit speakers, knowing in advance whether or not you want to remove cockpit headset and glasses, proper use of intercom transmit switches, etc. The amount of background noise generated by breathing O2 is highly distracting and could be significantly mitigated by more training and comfort with the use of the O2 mask. Having just come out of the Covid pandemic I suspect that emphasis on this type of training has probably waned industry wide.

Synopsis

Air Carrier Captain on a post maintenance test flight reported smoke and fumes filling the flight deck from the sidewall. The pilots established priority handling with ATC and returned to the origin airport. The Captain indicated the events were challenging while wearing the oxygen mask.

ACN: 1935735 (529 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 8000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1935735
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On our scheduled flight from ZZZ to ZZZ1 we had an air return due to a cargo door 'open' indication. We ran our checklists and had no door open indications prior to pushback. The ramp gave their speech before brake release confirming doors and latches were closed and secure. We then had a normal and uneventful departure. At approximately 8000 feet in the climb we received a 'cargo door open' caution EICAS (engine indicating and crew alerting system). Leveling off at 14,000 feet the FO (first officer) continued to fly the aircraft while I ran the ECL (emergency checklist). We both decided that the pressurization of the aircraft seemed to be operating normally but we could hear a rhythmic noise at the approximate location of the cargo door. I confirmed with Inflight that they were not moving around the galley carts which might have caused that noise. After conferring with dispatch I opted to do an air return back to ZZZ. The landing and taxi to the gate was normal. After arriving at the gate the ramp supervisor confirmed that the forward cargo door handle was ajar. Maintenance put the cargo door indicator on MEL and after refueling we made our way to ZZZ1 with no subsequent indications.

Synopsis

Air Carrier Captain reported a "cargo door open" EICAS illuminated during initial climb. After troubleshooting and conferring with Dispatch the flight executed a safe air return where Maintenance put the cargo door indicator on MEL and released the aircraft.

ACN: 1935734 (530 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1935734
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While boarding, HYD B LO AIR PRESS ECAM displayed. Maintenance said it was over serviced. They drained hydraulic fluid and cleared the fault. During takeoff, just prior to 80 knots, ECAM displayed HYD B RSVR LO LVL. We rejected the takeoff around 120 knots, cleared the runway and taxied to gate without further incident.

Synopsis

Pilot reported a HYD B LO AIR PRESS ECAM displayed during boarding. Maintenance drained hydraulic fluid and cleared the fault. During takeoff, just prior to 80 knots, ECAM displayed HYD B RSVR LO LVL. The Captain rejected the takeoff around 120 knots, cleared the runway and taxied to gate without further incident.

ACN: 1935688 (531 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class E : ZZZ

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Compressor
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1935688

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1935689
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We took off Runway XX out of ZZZ. I was the Pilot Flying (PF). Climbing out of 7,000 ft. we experienced a loud bang, accompanied by severe shaking and vibrations. After a second bang, I looked at the engine indications and saw the right engine was surging severely. I suspected a compressor stall. I immediately shallowed our climb, pulled the thrust lever to idle and the compressor stall went away with normal engine indications at idle. The Captain (CA) initiated a request for priority handling with ZZZ Approach, and we leveled off at 8,000 ft. We ran the ECAM procedure and pulled out the QRH for Engine Stall. We ran it and tried several times advancing the thrust lever, and the compressor stall returned every time. We left the thrust lever at idle, ran the single-engine approach procedures and told ATC we were diverting to ZZZ2. I continued being the PF and flew the ILS to [Runway] YYR. The landing was uneventful, and we exited the runway. As we began exiting the runway the engine auto-shutdown and displayed an Engine Stall ECAM. I transferred control to the Captain, ran the ECAM procedure, and we came to a stop. Airport Rescue & Firefighting checked over the aircraft and the right engine and saw no abnormal indications. We taxied to Gate XXX and disembarked the passengers as normal. No factors played a role in this from the flight crew. All indications and procedures were normal up to this point. No suggestions, this was a mechanical issue.

Narrative: 2

Departed ZZZ on Runway XX, Captain was Pilot Monitoring (PM), First Officer (FO) was Pilot Flying (PF). Climbing through 7,000 ft. we heard [a] loud banging accompanied with shaking. Shortly after Flight Attendant (FA) reported flames from #2 engine. Requested priority handling with ATC and leveled off at 8,000 ft. Followed ECAM procedure. Normal engine parameters at idle. Made several attempts to raise above idle but engine stall returned. Started APU. Sent message via ACARS to dispatch intent to divert to ZZZ1. Notified ATC of intent to divert to ZZZ1 also. Weather was CAVU. We went through single engine procedures. First Officer continued being PF and we flew the ILS [Runway] YYR ZZZ1. We cleared the runway. As we came to a stop engine #2 failed on its own. Airport Rescue & Firefighting met us as requested. After an inspection by fire personnel we continued to Gate XXX.

Synopsis

A320 flight crew reported compressor stall on takeoff with the right engine. The flight crew requested priority handling and diverted to make a precautionary landing.

ACN: 1935339 (532 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1935339
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Enroute in cruise, we suddenly noticed a burning odor that clearly was not normal. We donned our oxygen masks and I coordinated with ATC to divert to ZZZ. After the FO (First Officer) performed several steps of the smoke checklist, assisted by the jumpseater, the smoke smell disappeared. We continued for a safe landing in ZZZ, had the aircraft evaluated after clearing the runway by CFR (Crash Fire Rescue) personnel who then followed us as we taxied to the gate, and deplaned the passengers. The CFR stayed to evaluate the aircraft after deplaning and ensure no active fire remained. [The probable] cause [was an] unknown malfunction in the avionics ventilation system.

Synopsis

A320 Captain reported a burning odor during cruise. Flight crew coordinated with ATC to divert to a suitable alternate where a safe landing was executed.

ACN: 1933809 (533 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1933809
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Last night, on Aircraft X approaching ZZZ from ZZZ1, we had a F\CTL SLATS FAULT. Then proceeded to cancel the approach and troubleshoot. After 4 unsuccessful attempts to regain control with the Flaps selector as per ECAM, we performed the checklist and gather pertinent information from the performance chart, notified the cabin crew and passengers. Also notified Operation/Dispatch via local company frequency. We attempted to recycle the flaps selector a 5th time and this time we regain control of the slats. We continue an advised ATC the situation was resolved, no request for priority handling was made and we returned for normal landing.

Synopsis

A320 Captain reported a F\CTL SLATS FAULT was observed on approach. The crew conducted a go around and ran the checklists and QRH. After the flap handle was cycled for the fifth time, the slats resumed function and a normal approach to landing was made.

ACN: 1933506 (534 of 2908)

Time / Day

Date : 202209

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1933506
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While cruising at FL380, I noticed a hairline crack on Captain's windshield forming from center right curving across to upper the left corner, no ECAMs alerted or sparks were seen. I called for the procedure for cracked windshield. I could feel the crack was on the inside pane with my fingernail. We donned our oxygen masks and started a descent down to reduce the differential pressure per the procedures. The Seat belt sign was on. The Flight attendants were informed of the issue and instructed to strap into their jump seats. I [requested priority handling], informed ATC of the problem and was cleared to descend. Dispatch was advised via ACARS and a prudent decision was made to divert to ZZZ. I made a passenger announcement, briefly explaining the issue and our decision to divert to ZZZ for a Normal landing. We landed uneventfully. Dispatch and company were notified and a maintenance discrepancy was logged.

Synopsis

A320 Captain reported a cracked windshield during cruise resulted in a diversion.

ACN: 1933479 (535 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Pneumatic Ducting
Aircraft Reference : X

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1933479
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Air quality issue on descent. APU deferred due to several previous air quality writeups. Descending through approx FL250 into ZZZ, very strong "sweet oily" smell in cockpit. First Officer used oxygen momentarily due to lung irritation. Smell dissipated somewhat below 10000 feet, but still there. Contacted Flight Attendant's to ask if they were OK, and they said they didn't smell anything abnormal. We wrote up the aircraft upon arrival at the Gate, contacted the Duty Officer, and filled out the air quality event form. This aircraft had multiple air quality and air bleed issues in previous days. I think that it would be better policy to prioritize APU deferrals and air bleed issues rather than to defer them.

Synopsis

A320 Captain reported a very strong sweet oily odor during descent which dissipated somewhat at lower altitudes. Upon gate arrival a logbook discrepancy and an air quality form was submitted.

ACN: 1933196 (536 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 35000

Aircraft

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : None
Airspace.Class A : ZLC
Airspace.Class E : ZLC

Component

Aircraft Component : Traffic Collision Avoidance System (TCAS)
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Facility : ZLC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 12
ASRS Report Number.Accession Number : 1933196
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Software and Automation
Primary Problem : Aircraft

Narrative: 1

I am at ZLC and have received 3 reports in the past 3 days for A320 responding to TCAS RA's when there was no traffic in close proximity. Here are the details from those reports: Flight X was on present heading climbing to FL350 for traffic Flight Y descending to FL300. Flight X reported a TCAS RA that they were descending for, but didn't have an altitude on the traffic. Traffic was issued Flight X on Flight Y and informed them that they were going to pass eight miles apart. Flight Y asked if there was any other traffic for them and we reported that there were no other traffic that was a factor. Closest proximity to Flight Y was 9.33 miles and 1,800 feet. Day 2 XA40Z Flight Z (A320) Flight Z checked on with ZLC Sector 40 and reported that they were responding to a TCAS RA. Controller asked if they knew what it was for, and Flight Z responded that it was for traffic below them and they didn't know if it was a false one or not, but they were starting their descent back to FL350. R40 responded that they were showing nothing on their scope. Day 0 XX06Z Flight A (A320) At XW:06:45 Flight A advised R44 that they received a TCAS-RA while climbing from FL290 to FL330. No traffic in the vicinity. Closest traffic was at FL240 at 9 miles and FL380 at 12 miles. Flight A did state that they didn't get a traffic alert and it went straight to a TCAS-RA. R44 advised Flight A of the known traffic. Flight A advised that they were able to return on course and altitude with no interruptions. Recommendation - Check and see if there was an update with A320's in the past few days.

Synopsis

ZLC Center Controller reported issues with A320 aircraft that are receiving erroneous TCAS RA's when there is no other traffic around the aircraft.

ACN: 1932957 (537 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class E : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1932957

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1933507

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

After takeoff, at around 2,500 feet, a large bird struck the aircraft near the lower front edge of the FO's front window. A large smear of bird remains covered the window after the event. Upon inspection we observed what looked like a crack in the window. I then informed ATC of the event and asked for delay vectors while we worked some procedures for the event and would soon be requesting vectors for a return to ZZZ. I advised dispatch through ACARS of the event and that we would be returning. We then went through the Overweight Landing procedure as we were around 157,000 pounds. I then called the lead FA and informed here of the situation and then made a PA to the passengers of the situation and that we would be returning to ZZZ. We were given vectors to [Runway] XXR in ZZZ and made an uneventful landing. The max brake temperature reached was 460 degrees. Upon reaching the gate maintenance took the aircraft out of service to inspect the window and do the Overweight landing checks. The aircraft was deplaned and all passengers were directed to a replacement aircraft. The flight was resumed to ZZZ1 without further incident.

Narrative: 2

Approximately 5 minutes after takeoff and roughly 2,500 ft AGL we struck a bird that hit just below the windscreen on the FO's position. There were a large amount of remains splattered on the windscreen. I observed what I believed to be a crack on the lower right portion of the outer window. The Captain determined that it would be best to return to station and I fully supported his decision. We contacted ATC and asked for vectors off the SID to evaluate the situation and take the appropriate action. Dispatch was notified, the lead FA was notified and the passengers informed of the situation and our decision to return. We then went through the overweight landing procedure and set up for a landing back at ZZZ. ATC vectored us for a landing on [Runway] XXR back in ZZZ and we landed without incident. Upon arrival back at the gate we were informed that the aircraft required an inspection for the bird strike and overweight landing and would be out of service for at least 2 hours. Our pax deplaned and we were given an other plane for the flight to ZZZ1. We resumed our flight to ZZZ1 without incident.

Synopsis

A320 Flight Crew reported a bird strike during takeoff resulting in a crack on the First Officer's lower front edge window. The flight crew informed ATC and Dispatch of the situation and decided to return to the departure airport.

ACN: 1932726 (538 of 2908)

Time / Day

Date : 202209
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Cruise
Airspace.Class A : ZZZ
Airspace.Class E : ZZZ

Component

Aircraft Component : Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1932726
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1932730
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the Pilot Flying (PF) and my First officer (FO) Pilot Monitoring (PM). We were about to start our ZZZZZ arrival over ZZZ1. During our cruise systems check, we discovered the oil quantity indication on the ENG2 appeared to be lower than usual. ENG1,16 qt. ENG2, 6.5 qt. I decided to contact Dispatch for a Maintenance Control consultation. This communication with Dispatch was through SATCOM with a few failed attempts at least twice since we got the failure messages on the FMGC and the call dropping or a weak voice from the Dispatcher. I also tried through ZZZ1 Dispatch VHF XXX.X. When I did this, I accidentally tune the frequency on VHF 1 out from ZZZ Center, but flipped the ZZZ frequency back to it. When we finally got hold Maintenance Control through SATCOM, at that time the oil quantity was at 5.5 on ENG2. I explained our situation and concern with Maintenance Control and a possible engine leak and the trend of oil quantity, recommending to divert to ZZZ1 for a precautionary landing. At that present moment we had an ECAM advisory caution for ENG2 oil qty, 5 qt. Since Maintenance Control concurred with our plan for diverting to ZZZ1 we informed to ATC and continued our descent to divert. Another consideration to this decision was the marginal weather at ZZZ2, with light rain and low ceilings with a front coming into the area. The weather at ZZZ1 was VFR. The inflight and customers were briefed for the divert due to oil engine indication. We were vectored around for our sequence into ZZZ1, RNAV XXL. At this time, a priority was not declared yet. As we continued our descent at low power setting, the oil quantity ECAM advisory for ENG2 was still present and decreasing, 4.5qt. At that time, [priority handling was requested] and the inflight were briefed for a code yellow. We started the APU for systems redundancy. Once we were approximately 15 miles final and cleared for the RNAV XXL approach, our oil quantity for ENG2 was as low as 2qt at leveled off and temperature rising, I believe it was 138 C and increasing, if I remember correctly. It was evident, this was an oil engine leak and not an indication issue. Concerned about an engine damage and having an ECAM warning during short final a busy phase of flight, we performed a precautionary engine 2 shutdown, thrust lever 2 idle and engine master 2 off followed by completing the ECAM actions procedures. We continued the approach and the landing was uneventful. Once cleared of the runway, we briefly stopped at the taxiway for fire and rescue staff inspect the engine 2, if there was any visible damage. They discovered no damage but an apparent minor oil leak from the bottom of the engine. It took only a few minutes to asses this situation before we taxied to the gate. Upon arrival, we met with tech ops and gave them details of our situation. They commented that it was good we salvaged the engine, the oil leak was quite visible and evident once the cowlings were open. Finalizing my narrative, in spite of the circumstances, a rapid decreasing oil quantity on ENG2, [First Officer] and I did our best on bringing the airplane safely on the ground at ZZZ1 where the weather was favorable. It was an overwhelming situation trying to get hold of Dispatch or maintenance at the beginning due to the SATCOM connection issue and the time constraint. But at the end everything was accomplished safe and professional. However, in retrospect. I was really concentrated on getting the aircraft on the ground quickly once [priority handling was requested]. I would have emphasize on a potential precautionary engine shutdown during the approach briefing. It seemed that even though that my FO and I had a continuous dialogue and talking out aloud as we were descending to ZZZ1 we saw ourselves making a last minute decision for a precautionary engine shutdown. Perhaps doing the engine shutdown earlier would have gotten more time to review the contingencies for the flight. In-spite of how comfortable I feel flying the A320 this was a definitely a learning experience for me to manage my time accordingly.

Narrative: 2

During our flight from ZZZ3 to ZZZ2, somewhere close to top of descent, the Captain who was the Pilot Flying (PF) brought to my attention the oil quantity level for our number 2 engine. The oil quantity was significantly lower than engine number 1 by about 10 quarts and this was causing some concern. The Captain elected to contact maintenance and Dispatch with his concerns and loop them into the situation and determine an appropriate course of action. After conferring with Maintenance Control, we elected to divert to ZZZ1 which was close by instead of continuing on to our destination as the oil quantity kept decreasing significantly faster than a normal burn rate, as well as the advisory message on the ECAM showing 5 quarts of oil in engine number 2. While on the patch with Maintenance Control and during reduced workload I consulted the Quick Reference Handbook (QRH) to see if there was any specific guidance for low oil quantity but I was unable to find any. The weather in ZZZ2 also aided in our decision being marginal and ZZZ1 having better weather. ATC was informed of our decision and intention to divert to as well as the reason and we were given a further decent as well as answering the typical questions, "were we [requesting priority handling] at this time?, reason for the divert decision, if we needed any assistance?". At this time we elected not to [request priority handling], but did request one shortly after in order to get priority handling and get the aircraft on the ground as quickly and safely as we could. The customers and in-flight were also briefed. While being vectored into ZZZ1 we noticed the oil quantity was still decreasing reaching as low as 2 quarts. While thinking aloud and in the interest of safety and not wanting the oil quantity to completely reach 0 quarts and to prevent any damage to the engine or have it malfunction during this critical phase of flight, we decided an engine shutdown would be the best course of action last minute and the associated ECAM actions were accomplished. This was done as not to add any more complexity to the situation and we continued in for a single engine landing which was accomplished by the Captain uneventfully and flawlessly. Upon exiting the runway CFR which was standing by inspected the number two engine and did let us know that there was some oil they could see on the cowling of the engine. We then elected to taxi to the gate under our own power where we were told maintenance would be waiting for us. After a quick inspection by the maintenance personnel, they did confirm that in fact there was an oil leak and even showed us where it was coming from as well as stating we did the right thing by shutting the engine down and preventing any damage to the engine.

Synopsis

A320 flight crew reported due an engine fuel leak they shut an engine down and diverted to the nearest suitable airport.

ACN: 1928109 (539 of 2908)

Time / Day

Date : 202208

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Powerplant Fuel Distribution
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person : 1

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1928109
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1928111
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 3

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1928119
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 4

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1928118
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 5

Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1928120
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Fuel Issue
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Turn over to install VSV actuator, Installed actuator per AMM XX-XX-XX. Performed idle leak check no leaks noted at that time. When aircraft was on the blocks the incident was noted.

Narrative: 2

Performed removal of VSV actuator on engine #2 as per AMM XX-XX-XX. Gave verbal turnover to on coming shift.

Narrative: 3

Performed removal of VSV actuator on engine #2 as per AMM XX-XX-XX. Gave verbal turnover to on coming shift.

Narrative: 4

Received verbal turnover from midnight shift to install VSV (Variable Stator Vane) Actuator on # 2 engine. Proceeded to assist in installation of actuator as per AMM XX-XX-XX. During installation, I was tasked as the gopher, getting tooling, parts, going back and forth several times. Hangar Technicians performed engine idle run. After engine run, opened engine cowls, we performed visual leak check. No apparent leaks were noted at the time. Closed cowlings and aircraft proceeded to go out for high power assurance test. This is when [engine fire] incident occurred. Upon return to hangar, opened engine cowls, we inspected engine and found loose LP return line B nut at t-connector. Suggestion - going forward. more detailed physical and verbal turnover on aircraft instead of just a verbal turnover

Narrative: 5

Received verbal turnover from midnight shift to install VSV actuator on #2 engine. Assisted Technician to install as per AMM XX-XX-XX. Other Technician performed idle run for leak check , after run opened engine cowl and performed leak check. No leak noted at the time. Closed cowling and aircraft went for high power assurance test. This is when incident occurred. Aircraft returned to hanger. We opened engine cowl, inspected engine and found loose B-nut on return line at tee connector. Suggestion - For safety purposes need more elaborate physical as well as verbal turnover.

Synopsis

B Nut on a fuel line in the VSV actuator fuel system was not tightened, resulting in an engine fire during engine high power checks.

ACN: 1928069 (540 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1928069
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Arrived at aircraft at Gate in ZZZ. Ground air was connected and on. Aircraft was hot. Ground Ops disconnected ground air. Started APU and APU bleed was selected on. Went to mid cabin where crew was gathered to give them the briefing. Began to smell dirty socks odor in cabin. Odor was becoming stronger within a few minutes. Mid cabin to aft had stronger odor. No passengers were onboard. Followed cockpit/odor recognition and isolation procedure for aircraft on the ground. Called maintenance. Odor disappeared after APU bleed selected off. Made logbook entry. Maintenance took aircraft out of service.

Synopsis

Air Carrier Captain reported a fume event at gate described as "dirty socks". Captain completed ground fume procedures and requested Maintenance who removed from service.

ACN: 1927071 (541 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1927071
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Dirty socks smell throughout aircraft during taxi in ZZZZ coinciding with selection of APU bleed air. Flight Attendants noticed it right away and described it as intensifying. I didn't find out about their experience till opening the cockpit door and asking them. I asked because at shutdown I noticed the smell in the cockpit. I selected APU bleed off and the smell began to subside. In the aft cabin the smell was described as stronger on the right side than the left. This is a repeat malfunction for this aircraft. Two other instances were in the logbooks I had available, one on Date, another on Date 1. Two flight attendants reported headaches that subsided after thirty plus minutes. Flight Crew Operations Manual 1 odor procedure was followed. FOM was referenced. Logbook and odor reporting form completed. Maintenance Control requested a dual engine run which contract Maintenance could not do so the First Officer and I did a ten minute run with both packs on while using oxygen, the Maintenance Technician did not use oxygen. We both noticed the smell coming from our forward vents after engine shutdown when we took off the masks. Two Flight Attendants entered the aircraft and immediately noticed the smell, one got a sore throat. They left and the other two entered and immediately noticed the smell. A second engine run was requested and accomplished, this time for 15 minutes with the temperature selectors in full hot. We wore oxygen masks, the Maintenance Technician did not. This time the smell was not encountered after the engine run. APU was placed on MEL.

Synopsis

A320 Captain reported a dirty socks odor during arrival taxi. After conferring with Dispatch and Maintenance, two engine engine runs were completed and APU was placed on MEL.

ACN: 1926214 (542 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1926214
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Oil odor at gate during boarding APU BLEED ON. Previous APU oil leak and fume event three days prior. Aircraft removed from service, no crew affects.

Synopsis

Air Carrier Captain reported an oil odor during boarding with "APU Bleeds On". Maintenance removed the aircraft from service due to repeat APU malfunctions.

ACN: 1926211 (543 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Final Approach
Route In Use : Vectors

Component : 1

Aircraft Component : Main Gear Tire
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
ASRS Report Number.Accession Number : 1926211

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Visual approach to Runway XXR planned at flaps 3 due turbulence, wind shear advisories and 23 knot crosswind component gusting up to 27. Conflicting traffic resulted in clearance for visual approach XXL. PF maneuvered to visual XXL intercepting vertical profile using PAPI while PNF selected XXL visual guidance from FMS and auto brakes medium per [performance numbers]. Stabilized approach criteria was met and a normal landing performed in the touchdown zone, right (upwind) of centerline. Aircraft proceeded to gate normally, brake fans selected on during taxi in and brakes released after shutdown and chocked to aid brake cooling. During deplaning, tires 1 and 2 deflated liked due to fuse plugs and brake temperature. Outside air temperature 37C.

Synopsis

A320 Pilot reported discovering the number 1 and 2 main wheel tires had deflated due to melted fuse plugs after landing

ACN: 1925365 (544 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft : 1

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1925365

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Event / Encounter : Aircraft
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft X Event occurred while operating on Flight XXX (ZZZ-ZZZ1) upon pushback from Gate XX at ZZZ Airport. Upon completion of the before start checklist, we initially received clearance from Ramp Control to the middle of the alley, which was then revised by Ramp Control to continue to the top of the alley to Spot XX. While under tow, and while being towed backwards down the alley, our left Sharklet contacted the right elevator of Aircraft Y parked at Gate XY. Aircraft Y advised Ramp Control that their aircraft shuddered with possible contact with our aircraft. We immediately called the tug driver to stop the aircraft to analyze the situation. Upon Aircraft Y Captain's inspection, it was noted that Aircraft Y was contacted with visible damage to the outer most portion of the right elevator. I then contacted the Chief Pilot for further guidance. We then decided, with Chief Pilot's concurrence, to return to the gate to have Maintenance perform a more detailed inspection of the damage. Upon inspection of our aircraft, it was discovered that the fourth static wick from the top of the sharklet was missing with surface damage to the rear edge of the sharklet and minor scratches to the outer surface. Once blocked in, I contacted both the Dispatcher and Operations to notify them of the gate return. Our aircraft was configured for push, with APU operating and both engines off. Event occurred approximately at XA45 local, XA45 UTC, on Date.

Synopsis

A320 Captain reported push crew failure resulted in aircraft sharklet making contact with another A320 elevator resulted in damage and tow back to gate.

ACN: 1925136 (545 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1925136
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arriving at the plane for flight a persistent mild odor was observed in the cockpit area. The odor could be described as oil/dry musty. The odor was verified by other crew members and Tech Operations. A logbook entry was made, maintenance form filled out, and Tech Operations debriefed. No medical attention was required for crew or customers. The aircraft had a previous history of odor events. For one event, the APU was deferred and then had oil related parts serviced. The other could not be duplicated. Additionally, one pack and one bleed (high stage) were already differed, which would have led to limited ability to trouble shoot or isolate in flight. The odor was able to be observed by nearly everyone that went into the flight deck.

Synopsis

Air Carrier Captain reported a persistent "oil/dry musty" odor on the flight deck during gate preflight. A Maintenance logbook entry was submitted due to repeated fume issues.

ACN: 1924234 (546 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1924234
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Dirty sock odor in flight deck during descent. Shut down Pack 1 and seemed to dissipate. Odor returned when Pack 1 turned on during taxi. No odor in cabin. Aircraft had previous odor history and APU oil leak with repair.

Synopsis

Air carrier Captain reported a dirty socks odor during descent which dissipated when Pack 1 was selected off. Flight continued to a safe landing.

ACN: 1924127 (547 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZDV

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZDV

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 49
Experience.Flight Crew.Type : 612
ASRS Report Number.Accession Number : 1924127
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Close to BDROC waypoint which is west of MTJ VOR, [we] encountered moderate wake turbulence off another aircraft flying above us. Prior to this encounter the ride was smooth. We were flying westbound at FL340. Denver Center advised of an aircraft flying at FL350 at our 1200 position. Both the FO (First Officer) and I saw the aircraft and commented that it looked lower than 1000 ft. separation. TCAS readout was 900 ft. above our altitude for the approaching aircraft. All three altimeters read 34,000 in our aircraft. The aircraft approached us head on to the point where we could not see the livery to determine what airline it was or what type of aircraft. Seeing winglets it was probably a B757 or A321NEO. We both noted the winds at FL340 were 239 at 28 kts. which was less than 10 degrees off our heading to the left. A few minutes later we encountered moderate wake turbulence and after about 15 seconds the aircraft began to roll left at a good rate. Throughout the turbulence until the left roll, the autopilot was on and maintaining altitude. When the aircraft started to roll left, I clicked off the autopilot and rolled the aircraft to the right. After approximately another minute we flew out of the wake turbulence. There were no deviations in altitude or heading. During this wake turbulence encounter ATC was calling for a frequency change. After flying the airplane, we responded, reported the wake turbulence encounter and switched frequencies. We both forgot to ask what type of aircraft flew over us. After another frequency change, another sector asked if the aircraft had any damage or if there were any injuries. We looked at all the systems pages on the ECAM and couldnâ€™t determine any damage. There were no passenger or Flight Attendant injuries. Thankfully, all the passengers just happened to be seated. The Flight Attendants took their jump seats when they felt the turbulence even before we could make a PA or call them. This isn't the first time I have encountered wake turbulence from another aircraft passing within 1000 ft. of our aircraft. This is the most severe encounter and the most roll I have encountered. We both did not say "Upset". But we were both guarding the controls and ready to take control if required. Thinking back on it, I think we didn't say the word upset because the aircraft wasn't in an upset state but I wonder if I would have said it if the situation got worse. We definitely had positive control of the aircraft and both pilots were monitoring the situation. I wonder if the FAA has done an evaluation on the new aircraft with large winglets coming into the market for wake turbulence probability. Especially with light winds aloft.

Synopsis

A320 Captain reported encountering wake turbulence in cruise at FL340 from an opposite direction air carrier aircraft equipped with winglets at FL350.

ACN: 1923821 (548 of 2908)

Time / Day

Date : 202208

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component : 1

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : PMC, Performance/Thrust Management Computer
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Maintenance : Nondestructive Testing (NDT)
Experience.Flight Crew.Total : 140
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 140
ASRS Report Number.Accession Number : 1923821
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Flight Cancelled / Delayed
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was looking ahead to flight XXXXX on Aircraft X and noticed the aircraft that I had been assigned had numerous writeups. Of the write-ups, three stuck out as a concern for me. 1. Autothrust System 2. GPWS for glideslope deviation and 3. ILS 1. I contacted Maintenance Control as well as my Dispatcher to see if we could get some of these items fixed and/or swap airplanes. Maintenance Control said that they weren't planning on fixing any of the write ups between when I called (XA00 CST) and when I was supposed to depart (XE00 CST). I am a new Captain, flying with a new First Officer and we are supposed to land around midnight. I was concerned with the maintenance status of the aircraft, flying with a new First Officer as a new Captain, as well as the time of night. I decided in an abundance of caution to refuse the aircraft due to those issues.

Synopsis

A320 Captain reported being uncomfortable with accepting an aircraft that had numerous write ups and instead refused the aircraft out of an abundance of caution.

ACN: 1922808 (549 of 2908)

Time / Day

Date : 202208
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MSY.Airport
State Reference : LA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : MSY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 3
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Airspace.Class B : MSY

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1922808
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1922817
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 3

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1922809
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Ramp
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

New Captain on OE with Check Pilot operating as First Officer (hereafter referred to as the FO) and myself on the jumpseat conducting FAA New Captain check. The Company pages note not to start engines until pushback complete in MSY and from experience I am aware that they will push you tail north into the alley to the east while remaining on the ramp side of the taxiway line or will push you tail north on the west side of the terminal by the holding pad so as to not block the taxiways. Received an initial clearance from Ground to push tail east into the alley. As pushback commenced Company called for push in the alley and Ground amended the clearance to a tail west. The Tug Driver began a rapid push as the FO read back the clearance. By the time the FO acknowledged the clearance, advised the CA (Captain) and he was able to have the Ground Crew stop the push we were 90 degrees to the gate line with our tail west. At this point the crew asked the Tug Driver if he could pull us forward to the east alley and swing the tail in. The driver was becoming agitated at this time and advised that he couldn't due to Company A baggage carts blocking the ramp east of the gate. The Captain asked the FO to see if ground was okay with disconnecting where we were. As the FO began to work on that the CA advised the Ground Crew that we're 'good this way.' It appeared that his intent was to convey that disconnecting here would be okay but the Tug Driver interpreted it to mean that he was good to continue to push to the west as a few seconds later the aircraft began to move again, the tail began to swing south toward the taxiway to continue westerly around the terminal. Looking out the window there was a Company 737 coming around the taxiway to the west. I pointed out to the CA that this was an uncommanded push and it took two or three tries of him saying "Stop" on the interphone to stop the pushback. At this point we were tail south facing the terminal possibly protruding on the taxiway. The Tug Driver was very frustrated at this point and directed the Captain to set brakes. The Captain did so and replied brakes set, cleared to disconnect interphone. Meanwhile the FO was on the radio trying to advise ground we were disconnecting in place so that the Company 737 would be aware. While this was going on the tug was disconnected but still on headset when the Tug Driver yells "set brakes, set brakes." I looked inside from monitoring the Company aircraft and see we are slowly rolling forward. The CA and FO are both standing on the brakes, PARK BRK is on the ECAM and the CA reaches over to turn of the Anti-Skid/NW Steering switch (Part of the loss of all braking procedure - not appropriate here) and then rapidly turns it back on. I quickly reached up and turn on the ELEC HYD Pump on the overhead and the aircraft jolts to a stop. Once the push crew was clear the crew started both engines and discussed the event. The brakes were carefully checked and no anomalies had presented themselves throughout the event. With no faults present and all systems normal the flight continued to ZZZ. Standard phraseology. "Stop Pushback" is the term in the AOM (Aircraft Operations Manual). We discussed the importance of using this in communication with Ground Crew rather than 'stop'. Being in positive command of the push as the Captain. The Tug Driver was assertive in advocating for the way he felt the push needed to go. Ground was clearing the crew differently. Emphasized with the Captain that the aircraft should not move anywhere until and when he was satisfied and ready. It is always a challenge on the jumpseat to strike the appropriate balance between not interfering with the operation and watching out for safety of flight items. The interactions during the initial push were discomforting but not unsafe. It was when the aircraft began the second push without the Captain's approval that it appeared necessary to speak up. When the aircraft began rolling it was apparent that it was necessary to intervene. We spent a good portion of the debrief discussing all the appropriate barriers the crew could utilize fully during push operations to mitigate the distractions and threats. Standard Phraseology/CA more assertive with push crew. Had CA set brake after the first stop, (While directing Ground Crew to remain connected), it would have created a barrier to the ensuing events and confusion.

Narrative: 2

Second leg of turn for Captain upgrade OE. Upgrade CA (Captain) left seat, I was Check Pilot right seat, [FAA Check Pilot] in jump seat. Gate at MSY for return flight to ZZZ. All checklists complete, called for push from ground. Ground initially cleared us to push tail west onto Taxiway G for Runway 20 departure. As push was beginning, ground changed clearance to tail east into alley between B and C terminals. Captain advised Guideman of change, but Guideman/Driver began to push us tail west. CA began conversation with Guideman that they were pushing wrong direction, and directed him to stop a couple of times. He finally stopped push, and was told by CA that he needed to take us to other alley per ground instructions. What I could tell he refused due to some baggage carts [in the vicinity]. The decision was made to end push there. I was on ground frequency, trying to get a word in to advise of intentions, and looked to right to see if a taxi out from there was feasible. During that time, the Guideman disconnected. While I was looking to right, I heard loud voices from CA and Guideman. I looked forward and in, and saw we were rolling forward. I saw CA pressing on brake pedals, I simultaneously stepped on my pedals and glanced down at Park Brk lever, saw it was on, as well as Park Brk annunciation on E/WD panel. There were no abnormal or fault messages. Did not have time to look at Triple Indicator, as then Check Pilot jumpseater reached up and turned on Y electric hydraulic pump. Aircraft came to stop. No damage or injuries, thankfully, as Push Crew were already moving clear with tug and tow bar. We started engines, checked all systems, did a brake check, all systems normal, we decided to continue to ZZZ. No repeat occurrence. Debriefed in ZZZ, possible future prevention would entail making sure at least one engine is started before any disconnect. As a Check Pilot, I will also direct more of my attention to CA during push. Poor communication between CA and Guideman and Guideman not listening to CA instructions. Start at least one engine prior to disconnect. Monitor pushback better as Check Pilot in right seat.

Narrative: 3

During push back MSY Ground Control changed push instructions. I relayed the new instructions to the Push Crew. The Push Crew responded that they were unable to comply with the new instructions due to ground equipment in the way. I instructed the Push Crew to standby so that we could request new instructions from Ground Control. Ground told us that we were okay where we were but before I could relay that information to the crew, they started pushing the aircraft on their own. I told them several times to stop but got no response. After pushing us West approximately 30 ft., the crew contacted me and requested brakes set. No clearance for engine start had been given. I set the brakes, confirmed the green Parking Brake memo on the EWD and checked the triple indicator for correct pressure and confirmed "brakes set" and "cleared to disconnect interphone" with the Push Crew. While the crew was disconnected the equipment the aircraft started slowly rolling towards the crew and the terminal with the parking brake set. I tried to apply the "loss of normal braking procedure" to no avail. The Observer then turned on the yellow electric pump and we came to a stop with pedal braking. After confirming with the Push Crew that no damage or injuries had occurred, we started engines and confirmed that the brakes were now working properly. No faults or ECAMs were observed. The Push Crew was upset when ground control changed instructions. Communication with the Push Crew was then difficult. MSY prohibits engine start during push back. Better ground to flight deck communication. Requesting an engine start with the Push Crew after the push back is complete but prior to disconnecting the tug from the aircraft

Synopsis

A320 flight crew reported non standard phraseology with ground personnel resulted in aircraft uncommanded movement due to brake failure, and the aircraft pushed wrong direction.

ACN: 1922004 (550 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1922004

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X. FL 370, prior to top of descent, ENG page appeared on lower display. Indication was 1.5 OIL ENG 1. Pilot flying (PF) reduced thrust engine 1. Crew discussed possible sensor issue. After a short while oil pressure on engine one began to drop. Prepared to perform Electronic Centralized Aircraft Monitor (ECAM) actions and engine shut down. Following the ECAM actions, performed additional checklist and com. Crew requested priority handling, advised normal landing in ZZZZ1. Approach and landing uneventful. Emergency vehicles met aircraft just off runway. Aircraft was towed to the gate by ZZZZ1 personnel.

Synopsis

First Officer reported #1 Engine loss of oil quantity followed by loss of oil pressure. The flight crew performed an in flight shut down of the engine and continued to the destination airport.