ACN: 1922002 (551 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1922002

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X. In cruise FL370 ENG page came up. Crew noticed that the oil quantity was rapidly decreasing for the #1 engine. First suspected the problem to be a sensor error. After time, the oil pressure on #1 engine began to decrease. Crew brought the thrust lever to idle. Oil pressure decreased more. Crew attempted to ascertain it this was also sensor related and brought thrust lever to climb, but there was no noticeable increase in oil pressure. Therefore brought thrust to idle again. After some time, an ECAM appeared which directed engine master off. Crew performed ECAM actions and then ECAM for engine shutdown as well as com procedures. Crew began a slow decent, performed a single engine ILS and landed uneventfully. There were several airplanes behind us which lacked holding fuel, so crew taxied clear of runway, stopped, called FA stations, and had fire/rescue look for immediate threats. None found, FAs directed to remain seated, #2 engine was shut down and aircraft towed to the gate and deplaned normally. M/C (Maintenance Crew) was contacted by telephone and discussions regarding aircraft status and engine parameters during the event were discussed.

Synopsis

A320 Captain reported a loss of #1 Engine oil quantity and pressure. The flight crew performed and in flight shut down of the #1 Engine and continued to the destination airport.

ACN: 1921234 (552 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1921234
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arrival to the aircraft there was an obvious very strong "dirty sock" odor in cockpit and cabin. APU Bleed was on with both Packs. Logbook showed a history of APU problems and odor events. I immediately went outside to check the APU section of the tail and fresh oil was easily visible on the APU service doors and tail cone. The Inflight crew had been on a DH inbound on the same aircraft and stated that they had smelled a strong odor on the flight. The FO (First Officer) confirmed the "dirty sock" odor as well. I turned off the APU bleed, shut the APU down, informed the Gate Agent and called for Maintenance. Airbus Odor Reporting Form was completed.

Synopsis

A320 Captain reported a strong dirty socks odor upon arriving at the aircraft. Maintenance was requested and odor form was submitted.

ACN: 1920194 (553 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 15000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1920194
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft had been written up for dirty sock smell by a previous crew at the gate while on APU bleed the day prior. Maintenance performed a decontamination. Next entry in logbook was from Tech Ops that dirty sock smell returned after check and the APU was placed on MEL. Taxi takeoff and cruise were normal. We were given a late descent from FL350 down to 11,000 ft. We did an open idle descent. We noticed a dry musty dirty sock smell as we descended through 15,000. Smell intensity was minimal at first but did increase as we leveled out at 1,700 ft. From first indication until landing was 10 minutes. At no time was the odor overpowering and did not affect our abilities. After landing smell went away. Smell was only in the flight deck not in cabin. Contacted Maintenance Control, filled out odor summary at the gate. Passengers were not affected and Flight Attendants did not notice smell.

Synopsis

A320 pilot reported a "dirty socks" odor during destination descent. Flight continued to a safe landing where Maintenance was requested and a fumes report was submitted.

ACN: 1920181 (554 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 9000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1920181
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight departed ZZZZ, during climb passing 9,000 ft. Number one FA (Flight Attendant) reported a burning rubber smell in the forward galley. FO (First Officer) (Pilot Flying) continued climb. Captain (Pilot Monitoring) reviewed system pages on ECAM (Electronic Centralized Aircraft Monitor). No abnormalities noted. Captain initiated the Smoke/Fumes/Avionics Smoke QRH procedure. Continuing through the checklist, once the Commercial Pushbutton was pressed, inflight reported smell dissipating. The procedure was completed per SOP. The smell had been localized to the galley and first 3 rows of the cabin. The flight leveled off at 25,000. Captain placed a SELCAL to Dispatch and Maintenance Control. The decision was made to return to ZZZZ. Inflight and passengers were informed of the decision to return. Overweight checklist was completed. ATC was informed of our decision to return, no emergency declared. The aircraft returned to land without further incident. There were no reports of illness or discomfort from the passengers or crew. Logbook entries completed.

Synopsis

A320 Captain reported Flight Attendant A notified them of a "burning rubber smell in the forward galley area". After completing the QRH Fumes procedures and conferring with Dispatch the flight returned to departure airport for a safe landing.

ACN: 1919787 (555 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1919787
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After a quick taxi into Gate X from Runway XXL, we set the parking brake as instructed by the ground crew. Both of us confirmed the brake pressure. My attention was then on the timer to wait for the 3 minute cool down. We then received an ECAM for parking brake failure and brakes hot. I looked up and the Aircraft was rolling forward. I applied normal braking and the Aircraft stopped. No one was injured and no equipment was damaged. The Aircraft rolled approximately 2-3 ft. past the stop line. I did not notice what the triple indicator was reading when we received the ECAMs. I was intent on getting the Aircraft stopped and secure. The indicator did read normal before we got the ECAM and after I cycled the parking brake. The period in between that time frame I cannot say what the indicator was reading.

Synopsis

Captain reported ECAM Messages for Parking Brake Failure and Brakes Hot while parked at the gate with the parking brake set. The aircraft began to roll forward and the Captain applied normal brakes to stop the aircraft.

ACN: 1919623 (556 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : VFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 549
Experience.Flight Crew.Last 90 Days : 177
Experience.Flight Crew.Type : 549
ASRS Report Number.Accession Number : 1919623

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Avoided major collision with a company truck. After pushback and release at gate X at ZZZ, my First Officer (FO) requested a right turn out for taxi clearance due to single engine operations. Ramp Tower cleared us to give way to a B757 turning left from [gate] Y and to turn right, maneuver wide around a B777 pushed back at [gate] Z and proceed on Taxiway 1. I cleared left and forward and my FO cleared right as I released brakes and started a slow right hand turn. I asked my FO to query the Ramp Controller about our clearance because we were expecting Runway XX. The Ramp Controller then cleared us to turn right to Taxiway 2 and Spot 1. As I was making my turn, I noticed movement from my left periphery and looked to see a company semi-trailer proceeding east on the access road at about 30 mph. My nose was already clear of the access road, but my left wingtip was about to swing over the access road and into the path of the oncoming truck. I quickly applied brakes to stop the aircraft. The company truck continued on and came to a stop short of [gate Z] and the B777. My FO alerted Ramp Tower as we taxied to Taxiway 2 that we had almost collided with the truck and they responded that they would look into it. We started the remaining engine, ran the checklist and continued taxiing on Taxiway 3 to Runway XX. I called the Flight Attendants to ensure that everyone was OK and made a quick announcement to the passengers explaining what had happened.

Synopsis

A320 Captain reported making an aggressive stop to avoid a collision with a truck approaching at high speed on a surface road.

ACN: 1919603 (557 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : APU
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2199
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2199
ASRS Report Number.Accession Number : 1919603
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft Refusal for APU Bleed and Inadequate jetway Air. This report concerns an aircraft refusal (Unable to Fly- UTO) for no APU bleed on Aircraft X and being unable to cool the cabin to acceptable levels for the PAX on the ground. The real issue is the inadequate jet way air all over our system. There was also an issue with ACARS not accepting my UTO write-up. We started off with this aircraft on the first flight of a two day trip and were scheduled to keep it for 4 legs. During preflight, we got an AIR APU BLEED LEAK ECAM. The APU bleed air was put on MEL and we proceeded to ZZZ1. The issue quickly became the hot weather at all our destinations (ZZZ1, ZZZ2, ZZZ3, etc.) and the inability of the jet way or portable air carts to keep the cabin cool enough. We struggled with this in ZZZ1 and back in ZZZ2, but managed to keep the cabin temp no higher than 80 before the PAX were boarded. Once the engine was started at the gate, we could immediately cool the cabin more effectively with the packs. However, ZZZ3 proved more problematic. The aircraft was supposed to be fixed overnight there and it didn't happen. We were also at a minimum layover of 9 hours behind the door due to WX in ZZZ2 the prior evening. When we got to the aircraft in ZZZ3 the next morning (gate X) for our 1½ hour late departure, it was 86 degrees and humid in both cabins with 80 degree air coming in from the jet way hose. The outside air temp was about 90. We notified ZZZ3 Ops that this was too hot to load our PAX (several were elderly, in wheel chairs, etc., and the flight was almost full and we've all seen how the temps go up another 5-10 degrees once the plane is loaded without proper cooling). Ops agreed and they held off boarding and tried to adjust the jet way air. These are all city-owned jet ways (as with most places) and ops has little control other than turning them on and off. In fact, they mostly have no controls now for adjusting the temps and it's all supposed to be automatic, as long as the temp probe is in the cabin (it was). But evidently we can put men on the moon 50 years ago but can't solve the jet way temperature mystery for the 50 years since. It seems to be an ongoing engineering mountain and even with relatively new jet ways in ZZZ3, they just can't produce cold air for some unfathomable reason. After two different portable air carts were brought, we never got less than 80 degree duct temps or lowered the cabin temps from 86. I made the suggestion that we start the number one engine with the jet way attached, cool the cabin, and load the PAX with it running. We coordinated with ZZZ3 Ops, Dispatch, and the Operations and all agreed it was doable so long as everyone was briefed on the ramp and the PAX were spread out during the loading process so they wouldn't have to listen to the engine for too long and could just walk directly onboard. This worked and after 10-12 minutes, we notified ops they could begin boarding. As we finished our flight planning, I noticed they had kept us with this airplane for our upcoming ZZZ2-ZZZ1-ZZZ2 turn, which would make it 6 straight legs with this issue. That was a change as the original schedule had us changing planes once back in ZZZ2. Having already done the ZZZ1 turn the day before, I realized this plane needed to be fixed (and good thing as ZZZ1 was 42C when we got there later, or 108 degrees). After reviewing the aircraft refusal process in the FOM and an information sheet, I notified Dispatch en route to ZZZ2 that we were refusing the airplane, to give ZZZ2 a heads-up and time to find another aircraft as required. He acknowledged and said he'd start the process. I also asked if we needed to bring Maintenance or others in to discuss (per the guidance) and he said not required because he notified them already and they were in the process of coordinating its removal back in ZZZ2. I sent the code (I initially forgot the "XXX", but cancelled that entry and put in the right code). I also put in the comments field the following: â€œZZZ2 ZZZ1 APU BLEED INOP..HIGH SFC TEMPS W/ INADEQUATE COOLING AIR FOR PAX.â€ per protocols from FOM. But I got back an ACARS message saying that log number was "not active for this aircraft". I double checked the Maintenance Release (MRD) and our maintenance papers and it was indeed the right and still active log number and in fact the same one I put on the sticker next to the APU BLEED switch (that Maintenance read to me) the day prior. But the system said NOPE basically, and wouldn't let me send the UTO after a couple tries. I asked Dispatch about it and he said I could disregard the ACARS entry since "it is entered in the system so MX/OPS has the info." This seemed a little strange and I sent a follow-on message about the fact it said the log wasn't active when it was, and also the fact the UTO needs to be in the "Unable to Operate" section at the top of the Log History Report for 14 days/20 items, to notify pilots on subsequent flights (also per Company protocols). I never received a response. At this point, we were approaching the top of descent and further communication efforts on my part ceased, to focus on the arrival. We landed in ZZZ2 without incident. However, while pre flighting in our new aircraft for the next leg to ZZZ1, our Rep in the Ramp Tower called to ask about the UTO. He said it hadn't shown I'd coordinated with MX for the refusal, which is part of the protocols, and suggested I could still contact them. I told him that was strange since I had asked the Dispatch when I refused the aircraft en route if we needed to bring them in and he said he'd already done the coordination and they were working on it, as mentioned above. He thought that should qualify as contact and not to worry about it. But clearly the ball was dropped somewhere.

Synopsis

A320 Pilot reported the refusal of an aircraft due to the APU not working correctly resulting in increased cabin temperatures over multiple legs.

ACN: 1919195 (558 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use : Vectors

Component

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 582
Experience.Flight Crew.Last 90 Days : 194
Experience.Flight Crew.Type : 582
ASRS Report Number.Accession Number : 1919195

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

GEN FAULT ECAM on take off roll at 95 kts. Rejected take per Company policy and training. Ran appropriate checklist. Taxied back to a Gate and reported it to Maintenance both generator and hot brakes.

Synopsis

Captain reported a rejected take off at 95 kts. due to a GEN FAULT ECAM illuminated. The flight Crew returned to the Gate for maintenance action.

ACN: 1919074 (559 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1919074
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At gate with APU running and APU bleeds powering the packs, a significant rain shower approached the airport and it started to rain very hard. Shortly after the rain started an odor became present in the flight deck of a dirty sock smell. I called the cabin to ask the Flight Attendants if they noticed an odor and they reported that they did smell the dirty sock smell and that it was more present from the middle of the cabin forward. I ran the Odor Isolation Guide in the flight operations manual, with the APU bleed off the odor dissipated quickly. I then contacted Maintenance Control to inform them of the issue. The decision was made to deplane while Maintenance could run their procedures to address the odor.

Synopsis

A320 Captain reported a "dirty socks" odor while parked at gate. The flight crew completed appropriate procedures and requested maintenance who had everyone deplane for troubleshooting.

ACN: 1918564 (560 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1918564
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
When Detected : Taxi
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X, Day 0 was my 4th day. The previous day, I flew a 5 leg day with several delays with a short layover. Upon arriving in ZZZ from ZZZ1, we had to wait in the holding area until the gate became available. The First Officer (FO) and I verified the ramp was clear and we had 3 Marshallers. I followed the Lead Marsaller to the gate. Once he gave me the "Stop" signal, I set the parking brake and we began the Parking Checklist Procedure. During this procedure, I believed I saw the "Chocks Inserted" signal, so I released the parking brake. Since the Jet Bridge was moving towards the aircraft, I didn't realize the aircraft began moving. The Marshaller promptly repeated the "Stop" signal and I quickly pressed the brakes and reset the parking brake. The aircraft rolled forward around 10 feet. There was no injury or any damage to the aircraft. I failed to confirm the "Chock Insert" signal and after releasing brakes, monitor for aircraft movement for a few seconds.

Synopsis

Air Carrier Captain reported aircraft rolled forward during gate parking when parking brake was released. The Marshaller immediately repeated the "Stop" signal preventing any injury or damage.

ACN: 1918252 (561 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : ELAC2
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1918252

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Low speed rejected T/O (takeoff) at 70 knots due to F/CTL (control) ELAC 2 PITCH FAULT.

Synopsis

An A320 First Officer reported low speed rejected T/O at 70 knots due to F/CTL ELAC 2 PITCH FAULT. The flight crew returned to the gate for aircraft repairs.

ACN: 1918245 (562 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1918245
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon entering the [aircraft] from the jetbridge, I noticed a foul odor in the forward galley and cockpit. APU Bleed Air and Pack 1 were turned off. The smell dissipated. There were no previous logbook write-ups for similar issues. While performing the walkaround I noticed the PC Air was hooked up, and we continued to use that for cabin climate control. The FO (First Officer) and I discussed the odor issue and company protocol. We elected to minimize use of the APU and continued our [preparation]. We experienced a strong foul odor during takeoff and climb. We referenced the FOM and FCOM (Flight Crew Operations Manual) 2 odor isolation guidance. I selected PACK 1 off and closed the crossbleed valve as per the procedure. As time went on, I continued to smell the odor, though not as strong as the takeoff and initial climb. Following a restroom break, I returned to the flight deck and the smell was very apparent. The FO (First Officer) and I discussed our condition, options and symptoms, and [queried] the cabin crew. We decided it was safe to continue to our destination. The aircraft was written up for cabin odors and ZZZ Tech Ops met the aircraft on arrival.

Synopsis

A320 Captain reported a fumes event during takeoff. Flight crew followed appropriate procedures which resulted in the odor to dissipate enough to continue to destination where Maintenance was requested.

ACN: 1918237 (563 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0001-0600

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1918237
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The aircraft we were flying had experienced an odor event on the previous flight. The Auxiliary Power Unit was deferred and the aircraft went back into service for our flight. The flight was uneventful however on the descent. I'm unsure of what altitude it started. We smelled the "Dirty Sock Smell". The cockpit jump seat was occupied and all of us in the cockpit smelled it. We shut pack one off and shut the cross bleed and the smell dissipated. We landed without any further incident. The inflight crew did not smell anything.

Synopsis

A320 Captain reported a "dirty socks" odor during descent. Flight crew completed appropriate procedures and landed without any further issues.

ACN: 1916732 (564 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Descent
Crew Size Flight Attendant.Number Of Crew : 3

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
Experience.Flight Attendant.Total : 25
Experience.Flight Attendant.Airline Total : 25
Experience.Flight Attendant.Number Of Acft Qualified On : 2
Experience.Flight Attendant.Type : 90
ASRS Report Number.Accession Number : 1916732

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon boarding I noticed a rancid/locker room smell that continued throughout the cabin. I asked the other Flight Attendants and they said they did not smell anything. When we were descending the odor got worse and the #2 Flight Attendant said she smelled what seemed to her to be burning plastic smell. The smell did not change for me only got worse upon descent.

Synopsis

Air Carrier Flight Attendant reported a rancid/locker room odor initially during boarding that was also reported by another Flight Attendant during descent when the odor intensified.

ACN: 1916282 (565 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1916282
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Showed up to work with no illness or headache. At engine starts I smelled chemical fumes (magic marker/dry erase pen). After engine starts I shut down the APU and these fumes disappeared so I figured they were transient. In climb out through a marine layer of clouds, I smelled it again and got a call from the aft galley Flight Attendants saying they smelled something from their overhead air gaspers. One described it as a burnt sugar smell and the other as a strong window washing chemical smell. We looked at the FOM and determined it was a Level 1 transient odor. On Taxi in at ZZZ1 after starting the APU and bringing on the bleed air, I smelled the same chemical smell. Got a call from Flight Attendant right away from the aft galley. Turned off the APU and taxied to the gate. Called Maintenance and wrote up Level 1 fume/odor event in logbook. (Later realized it was more of a Level 2 since multiple crew sensed it and different times on the flight). Had the crew leave the plane and get fresh air in the terminal while Maintenance checked it out. Myself and another Flight Attendant from the aft had strong headaches. Spoke to the Duty Chief Pilot Name about the event. Took ibuprofen for headache and flight attendant took acetaminophen for hers. Returned to the aircraft after about an hour and did not smell that fume/odor. Maintenance signed off the APU and cleared the aircraft to fly and we returned to ZZZ on Aircraft X. Had a headache the rest of the day and when I got home it was more of a migraine. Went to bed extremely early to sleep it off. Had a headache the following couple days plus fatigue and flu-like symptoms. Called Medlink to talk about my symptoms and get their recommendations on what to look out for.

Synopsis

Air Carrier Captain reported an intermittent fume event during a two segment out and back flight. Flight crew submitted an APU logbook discrepancy after first segment which Maintenance signed off. Several crew members reported experiencing lingering physiological symptoms.

ACN: 1916138 (566 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 8500

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors

Component : 1

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Pressurization Outflow Valve
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 192
Experience.Flight Crew.Type : 816
ASRS Report Number.Accession Number : 1916138
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Late takeoff out of ZZZ1 due to Engine #2 HP Valve Inoperative. We followed MEL procedures for low power settings in descent. Descent into ZZZ at FL245, we started getting the altimeter flashing amber (wanting local altimeter setting). Both Captain and I checked the FMC, and programming was correct. We continued descent and set local altimeter at FL180. We were a little task saturated with getting ready for approach, moderate turbulence flying through buildups, unable to deviate due to traffic, TCAS TA , and it was getting a little hard to hear in the cabin. At approximately 8 ft. 500 MSL, we get an ECAM (CAB PR LO DIFF PR...A/C V/S REDUCE). The aircraft was only doing approximately 1,300 FPM descent. Looking at the pressurization page, we see the outflow valve amber and fully open. FMC was correct with destination and data in approach PERF page. Since we were below 10,000 ft. MSL and about to land, we continued to the field and landed uneventfully. No one in the back knew of the issue as masks did not drop. We were scheduled to take this aircraft from ZZZ to ZZZ2. Maintenance performed checks, and all their tests passed and was going to put the aircraft in service. The Captain was not comfortable taking the aircraft as maintenance could not find a reason as to why the outflow valve fully opened. Captain refused the aircraft and I concur with his rationalization. The #2 HP Valve Inoperative would not have caused the issue, and given that we had a flashing amber altimeter on the PFD at FL 245, it did not feel right taking the aircraft with a plane load of passengers not knowing the cause. Subsequently, we flew the aircraft to ZZZ3 at 10,000 ft. MSL and prior to taking off we had a PACK 2 failure which we had to have deferred.

Synopsis

A320 First Officer reported aircraft cabin altitude began to descend ahead of depressurization schedule on initial descent. The flight crew continued on programmed descent and and a normal landing at destination airport. The flight crew made a log book entry and maintenance ran their checks and signed the aircraft off. The Captain refused the aircraft after disagreeing with the maintenance actions.

ACN: 1915939 (567 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Failed

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1915939
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1916303
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X, I the First Officer (FO) was the Pilot Flying (PF) and the Captain was the Pilot Monitoring (PM). Our approaching to ZZZ Runway XX was normal. Vapp was approximately 135 kias and the aircraft was configured flaps FULL. At 1,000 ft. AGL the aircraft was stable and cleared to land and the Captain made that call referencing such. The aircraft touched down within the touchdown zone and the landing was smooth. Upon touchdown the Captain called 'Spoilers' and 'Reverse Green'. The aircraft landed with Auto brakes LO and I transitioned to manual brakes at approximately 100 kias. At approximately 90 kias I felt the aircraft jerk and yaw. My assumption was that a wheel had momentarily locked up. This jerk was just for a second and then the aircraft continued a normal deceleration on the centerline. The Captain called '70 Knots' and then assumed control of the aircraft. The Captain taxied off the runway at SG4 and all indications were normal with no unusual steering input or power required. After performing the after landing flow the Captain began to monitor the WHEEL page and noticed that brake #2 was increasing in temp out of line with the other brakes. Upon runway exit brake #1 was approx 210, brake #3 approx 150, and brake #4 approx 150. Brake #2 was 300 and climbing. Turning north on Taxiway SC approaching Runway XYL the #2 brake was climbing steadily to 400. The Captain told the FO that he suspected that we were dragging brake #2 and to contact OPS and request that maintenance meet the aircraft. ATC had us hold short of Runway XYL at SC and the cleared us across that closed runway and instructed us to hold short of Runway XYR at U. The Captain stopped short of XYR at U and set the parking brake confirming the triple indication and the ECAM and then turned the taxi light off. Almost immediately ATC stated 'Aircraft X your left main appears to be on fire. The Captain hit the PA button and stated 'Flight Attendant Stations'. ATC stated that ARFF was en-route and then I asked if we needed to remain on the current frequency or to switch to a different. We were told to stay on our current frequency. The LEAD Flight Attendant (FA) called and asked if we needed to evacuate. The Captain told her not at this time, ARFF was responding, and that we would let her know. We got an ECAM regarding HYD pressure low but our attention was taken back to the ARFF response and the aircraft configuration. ATC reported Airport Rescue and Firefighting (ARFF) approaching and we could feel the impact of the water hitting the aircraft. We went back to the ECAM and noticed that all of the YELLOW HYD fluid was gone, our left side brakes were going, and our nose wheel steering was lost. We started the APU and shutdown both engines never turning on the yellow electric pump. ARFF reported that the fire was out but still smoking. The Captain hit the PA button and stated 'Flight Attendants, remain seated, remain seated, remain seated'. The Captain told ARFF that we would need to be towed in and confirmed that they would escort us to the gate. The Captain made a PA announcement informing the passengers of the brake issue, the subsequent fire, and that ARFF had extinguished the flames and that we would be towed to the gate with the escort. The Captain further asked them to be patient as we coordinated a tow to the gate. The FA called asking if passengers could use the LAV. The Captain allowed them but reiterated once a tug showed up they would need to be seated. While waiting for the tow The Captain sent a message via ACARS to Dispatch and Maintenance informing them of our situation. We coordinated with Airline Operations, ZZZ Operations, and ATC for the tow. Once connected we confirmed with ATC that the aircraft movement was now under control of ZZZ Operations and that they would coordinate clearance across XYR and to the gate. At the gate The Captain set the brake, turned off the beacon, and confirmed that we were chocked before releasing the parking brake and turning off the seat belt sign. At the parking check list we noticed the FAULT lights on the PTU and the Y ELEC PUMP. The Captain turned them both off in accordance with the ECAM and finished the checklist. The passengers deplaned via normal procedures. The Captain then contacted the Chief Pilot of Duty. The Captain was instructed to make phone calls to Airline Operation and ZZZ Tower to provide additional information.

Narrative: 2

[Narrative contained no additional information.]

Synopsis

A320 First Officer reported after landing they had a brake overheat and a fire in the brake area of the number 2 brake.

ACN: 1915392 (568 of 2908)

Time / Day

Date : 202207

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 18000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1915392
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent around FL180, the lead FA (Flight Attendant) called us to report a dirty socks smell in the aft cabin. She said multiple FAs and passengers smelled it, and passengers were concerned. We put on our oxygen masks, turned on Engine Anti-Ice, and shut off Pack 2 per COM. I [requested priority handling], got cleared direct to the FAF, and requested paramedics meet us at the gate in case anyone was feeling unwell. The FO (First Officer) called the lead FA back, and she said the smell stopped, so we took off our oxygen masks. There was never any smell in the cockpit. The FO coordinated with Operations to ensure we had an open gate, and advised Dispatch via ACARS. Everything was in place to allow us to park immediately and deplane normally. I requested the ground crew not connect the air conditioning. Paramedics and Airport Rescue and Fire Fighting boarded the aircraft, but no one requested medical assistance. After all the passengers were off, the whole crew debriefed with Airport Rescue and Fire Fighting and local Maintenance. At this time, the FAs said there was a passenger who took their shoes off. They said it was possible this may have been the source of the smell, but they weren't sure. They said the smell was pervasive in the cabin, but did not seem to be coming from the air vents. I wrote up a discrepancy in the logbook and filled out a Cabin Odor Reporting Sheet. I called Maintenance Control, Dispatch, and the Chief Pilot to report the event.

Synopsis

Air carrier Captain reported being notified by the lead Flight Attendant of a fume event during descent. Flight crew continued to the destination airport where a safe landing was executed.

ACN: 1915220 (569 of 2908)

Time / Day

Date : 202207

Place

Locale Reference.ATC Facility : BOS.Tower
State Reference : MA

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : BOS

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A330
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : BOS

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 5500
Experience.Flight Crew.Last 90 Days : 205
Experience.Flight Crew.Type : 600
ASRS Report Number.Accession Number : 1915220
Analyst Callback : Attempted

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 172
Experience.Flight Crew.Type : 345
ASRS Report Number.Accession Number : 1915209

Events

Anomaly.ATC Issue : All Types
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Anomaly.Ground Event / Encounter : Weather / Turbulence
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

This report is to document the circumstances around two go-arounds on Runway 27 at BOS. I was the Captain and pilot flying on the flight [to] BOS. Flight was uneventful until short final when the actual winds and the programmed winds began to differ greatly, causing an increased mini groundspeed (Airbus gust correction) of 150 kts. vs programmed approach speed of 139. This increased ground speed, light weight, gusty winds, and delayed power reduction because of speed required, caused a ballooned flare which floated the aircraft out of the touchdown zone. Additionally, Runway 27 is only 7000 ft., making the touchdown zone even more critical; therefore, I executed a go-around. There was a helicopter crossing departure end of 27, forcing an aggressive climb to 3000 ft. We were vectored back around for a second attempt at ILS 27; however, we were vectored 6 miles in trail of an A330 and given specific speeds that maintained that spacing on final. Due to winds of 340/13G19 and the 7000 ft. runway, the wake of the preceding A330 caused the aircraft to balloon and float out of a safe landing position again, forcing a second go-around from landing position. The go-around was uneventful; however, due to approaching reserve fuel levels, we elected to declare minimum fuel and requested vectors to Runway 33, to be more aligned with the prevailing winds and on the longer runway. Approach and landing were uneventful and we arrived at the gate with exactly 3,900 lbs. fuel, our reserve for that flight.

Narrative: 2

Two go arounds performed landing on Runway 27 BOS before a successful approach was conducted to 33L. Both go arounds were initiated below 50 ft., the first of which ground contact was made during the go around. I believe the primary cause for both go arounds to be an increased reference speed due to inaccurate wind reporting at BOS. Loaded winds were 340/13g19. Airplane mini gs pushed ref speed up to 150 kts. causing excess float. This in combination with wake turbulence from a heavy on second approach prevented us from making a safe landing during first 2 events. Min fuel was declared and during 3rd approach ATC gave us updated winds of (from memory) 340/27g35. Successful landing was made.

Synopsis

A320 flight crew reported executing two go-arounds at BOS in gusty wind conditions. The second go-around also involved a wake turbulence encounter.

ACN: 1915210 (570 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 174
Experience.Flight Crew.Last 90 Days : 101
Experience.Flight Crew.Type : 174
ASRS Report Number.Accession Number : 1915210
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climb out, received "CAB PR SYS1 FAULT" Electronic Centralized Aircraft Monitor (ECAM). SYS 2 worked normally. Referenced the FM and continued. At FL340, received "CAB PR EXCESS ALT" ECAM with associated master warning. We flew the airplane, silenced the alarm and confirmed the [problem]. The Captain directed me to continue to fly the aircraft and we donned our masks and established comms. He got clearance to descend from ATC and I initiated our descent. The ECAM cleared and reappeared at least once during the descent. During the descent, the Captain let the flight attendants (FAs) know what was going on and they mentioned that the emergency lights came on but that the masks did not drop. We leveled at 10,000 ft. and elected to return back to ZZZ1 since the FAs reported that there were no injuries on board. I coordinated the return with ATC while the Captain informed the passengers of what had happened and communicated with Dispatch. On final at ZZZ1, we received a "CAB PR LO DIFF" ECAM that cleared on its own after a few seconds. The Captain elected to land the aircraft since we had [priority] and we were overweight by about 5k lbs. We landed without incident and returned under our own power to the gate.

Synopsis

A320 First Officer reported an EICAS message warned of a malfunction with the #1 pressurization system during climb out. Later another warning regarding the pressurization system appeared. The pilots descended to a lower altitude and returned to the departure airport.

ACN: 1915175 (571 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1915175

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At gate with APU running the cockpit had a moderate odor of chemicals/musty. This aircraft had had a significant odor history in the past month. Dispatch notified and aircraft taken out of service (OOS).

Synopsis

Air Carrier Captain reported a fume event described as a moderate "chemicals/musty" odor during gate preflight. The aircraft, which had a history of fume events, was removed from service by Maintenance.

ACN: 1914982 (572 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Person

Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2
ASRS Report Number.Accession Number : 1914982
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was working Local West and had the turn releases. Local Center owned the straight out releases. I cleared Aircraft X for takeoff from Runway XXR and told to turn right heading 295. Everything was read back correctly. I lined up traffic afterwards. Once Aircraft X was at departure end, he [requested priority handling] and informed me that he lost an engine and that he was going straight out. I quickly issued traffic and a caution wake turbulence with a heavy that was two miles south of the field that was straight out off of the center runway. Aircraft X did not climb and when offered any type of turn, he told me to standby. I couldn't get much from the pilot as he was busy and would tell me to standby. Aircraft X continued flying straight out at a low altitude. We stopped all departures on all runways and waited to see what the pilots intentions were. Aircraft X finally informed me he wanted to climb to [9000 ft] and wanted a wide left turn to gain altitude and to then turn right to enter a right downwind. He also informed me he would then like vectors for the approach. I asked him if he was able to maintain his own terrain and obstruction clearance. The pilot said he could and started his climb to enter a right downwind. Once in the downwind, the pilot was willing to take a frequency change to final so they could give him vectors. Once I handed him off to final, there was a small [fire] on the southwest area close to Runway XXR. I sent fire command over to investigate while telling them the emergency would now be landing the center runway. I also had a line of departures at the runway and traffic that checked in late on final. I offered the arrival the option to move to the center runway or I would have to send them around due to the fire. The arrival was able to switch runways and land with no issues. Shortly after, airport ops closed Runway XXR due to the fire. I got approval from airport ops to back taxi down the runway to Taxiway 1 and taxi the line of departures to the center runway. Aircraft X landed the center runway and after inspection was told they were fine to taxi back to the gate. I should have issued more MVA instructions or maybe offered an opposite direction arrival. The winds were 130/11 and I do not think he would have taken it and I do not recall what was on final but being he was at a low altitude, that could have been an option. It was difficult to offer much at the time due to trying to coordinate with the Sup, TRACON, Local Center, fire command, airport ops, arrivals, departures waiting in line, etc. and being told to standby from Aircraft X.

Synopsis

Tower Controller reported an aircraft on departure had an engine failure and returned for a landing. The situation was further complicated by the need to coordinate with numerous people and the pilots repeated replies to stand by.

ACN: 1914662 (573 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1914662
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After starting descent into ZZZ we had initially a gen 1 failure and then a dual gen failure followed by a flood of failures that I believe were associated with the initial failures- we tried resetting the generators and gen 2 came back online. We then had an Integrated drive generator 1 oil temp high and we disconnected Integrated drive generator 1. The airplane was in alternate law with no autopilot or auto thrust. At this point we believed trying to land ASAP was the best decision. We requested Runway XX at ZZZ. We kept the Flight Attendants updated but let them know that a brace call was unnecessary and that it should be a normal landing. We landed and taxied in without incident.

Synopsis

A320 Captain reported a dual generator failure during descent and a subsequent diversion for landing.

ACN: 1914381 (574 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1914381

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Passenger
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While preparing for departure some passengers an inflight smelled an odor. We shut off the APU bleed and called maintenance they had us run an engine and determined the aircraft required additional maintenance.

Synopsis

Air Carrier Captain reported an odor during pre-flight. Maintenance determined additional troubleshooting was required.

ACN: 1914363 (575 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1914363

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Arriving on aircraft that just flew in from previous flight we noticed a musty dirty smell. We were assessing the situation with inflight as we were boarding. We decided to call Maintenance to investigate the smell. Maintenance arrived on the aircraft and we were told to deplane. The aircraft was taken out of service.

Synopsis

A320 Captain reported a "musty dirty smell" during preflight. Maintenance was requested which resulted in the aircraft being taken out of service.

ACN: 1914361 (576 of 2908)

Time / Day

Date : 202207
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1914361
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After starting both engines, #2 from start cart then crossbreed start #1, and completing the appropriate checklists, we started taxiing and both immediately commented that the odor issue that the Plane had the previous day was still there. We continued to taxi a little more and realized it was getting worse, we were both experiencing burning eyes and a closing feeling in the backs of our throats. I then stopped and requested a return to the Gate. As soon as the Maintenance Personnel entered the aircraft they also smelled and confirmed the fumes smell onboard.

Synopsis

Air carrier Captain reported a fume event during departure taxi. Flilght crew elected to return to departure Gate for maintenance.

ACN: 1914069 (577 of 2908)

Time / Day

Date : 202206

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Cabin Lighting : High
Passengers On Board.Number : 0
Crew Size Flight Attendant.Number Of Crew : 3

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Experience.Flight Attendant.Total : 38
Experience.Flight Attendant.Airline Total : 38
Experience.Flight Attendant.Number Of Acft Qualified On : 4
Experience.Flight Attendant.Type : 80
ASRS Report Number.Accession Number : 1914069
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The Captain noticed the odor first, then called maintenance. It was a faint burning oil smell. Some crew experienced burning eyes, and I had a headache. We all deplaned the aircraft and stayed on the Jetbridge so maintenance could do their checks. The aircraft was eventually taken out of service.

Synopsis

Air carrier Flight Attendant reported a fume event during preflight described as burning oil odor. Captain requested Maintenance resulting in the aircraft being taken out of service.

ACN: 1913753 (578 of 2908)

Time / Day

Date : 202207

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1913753
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On Date we were flying an A320 from ZZZ to ZZZ1. During cruise we had a fume event. The estimated duration of the event was 2 min. During the event the power levels did not change. I have not experienced this in this same aircraft type before. There was no evidence of smoke or fire in the cockpit or cabin. The fumes smelled like wood or caramel burning and were moderate to strong. During the time of the event our air supply was on the engines. We used oxygen mask because of the event. We were able to isolate the smell and land in ZZZ1.

Synopsis

Air Carrier Captain reported a temporary fume event during cruise. The flight crew was able to isolate the burning odor which enabled the flight to continue to destination and land safely.

ACN: 1913581 (579 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1913581
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had a distinct smell of oil with the APU bleed on.

Synopsis

Air Carrier flight crew reported an oil odor with the APU bleed on.

ACN: 1913572 (580 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1913572
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1913574
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise the number one [Flight Attendant] called and reported a strong chemical smell in the forward galley and rows 1-4 in the cabin. The odor was causing eye and throat irritation, along with headaches in her and most of the passengers in the first four rows. The FO (First Officer) and myself started the Odor recognition and isolation check list. As we ran the checklist I received feedback from the number 1 and determined that the procedure was not helping. The [Flight Attendant] advised me the irritation was getting worse and the passengers were concerned. After informing ATC of our situation we were cleared to 10000 and continued running the Smoke/Fumes Removal procedure (QRH). After several minutes into this procedure inflight advised the situation was not improving and that they could no longer remain in the forward galley do to the odor. I was in contact with Dispatch and Maintenance and was advised by Maintenance that we had done all we could do. Dispatch advised that we had enough fuel to continue to ZZZ but I felt the situation was worsening and the safest action would be to divert to ZZZ1. I advised ATC of the divert and [requested priority handling] along with EMS at the gate. We were vectored to ZZZ1 and landed without an further incident. The strong odor persisted until the forward door was opened. All passengers and crew members declined medical attention and the plane was unloaded.

Narrative: 2

[Narrative contained no additional information.]

Synopsis

Air carrier flight crew reported a flight attendant notification of a strong chemical odor in the forward cabin. Due to no improvements in the air quality after troubleshooting, the flight requested special handling to a diversion airport where a safe landing was completed.

ACN: 1912636 (581 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component : 1

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1912636
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

This report is being submitted only because it is a recommended step by ALPA: no errors in policies or procedures were made by the crew. Airplane was a Routine Overnight (RON) from the previous flight. Upon arrival at report time, the cabin was being cooled by ground air and was comfortable. We started the APU shortly before departure time. Climb and cruise were normal. Passing 11,000 ft. on the descent, the FO (First Officer) and I both noticed a strong (4 out of 5) "dirty sock" odor coming from the cockpit vents. I turned off PACK1 and the odor began to abate. I called the cabin to ask the Inflight Crew if they were experiencing the same in the cabin. They said they were not. Maintenance was requested during taxi in, PACK1 remained off. Upon inspection of the tail section, I saw fresh oil coming from the APU service door seams and drains. According to the logbook, the APU had been serviced after a "LOW OIL" message three days earlier. The Contract Maintenance Technician opened the APU service doors and observed oil throughout the compartment, including on the breather port. When he opened the APU oil service point, he estimated that more than a quart of oil spilled out and the reservoir gauge still showed over-filled. Maintenance Control had the Technician run both packs for 15 minutes at a high temp setting to decontaminate the PACKs with the intention to put the APU on MEL and have us fly our return flight with passengers. Inflight were encouraged to leave the aircraft for this procedure. At the beginning of this procedure, the Maintenance Technician also smelled the odor and agreed that the system was contaminated. The FO and I decided that we would do a brief engine run of the #2 engine at the gate before we boarded crew and customers in case contamination of the environmental system was still a problem. PACK1 was turned on first and did not seem to still be contaminated. When PACK2 was turned on with PACK1 off, we both smelled a strong "dirty sock" odor again. I turned PACK2 off after about 10 seconds and we advised the Maintenance Technician. Operations determined that the aircraft would be repositioned to a Maintenance station. We ferried the aircraft unpressurized to ZZZ1.

Synopsis

Air Carrier pilot reported a fume event during descent. After collaborating with maintenance and further troubleshooting it was decided to ferry the aircraft to a company maintenance base.

ACN: 1912620 (582 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1912620
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Magic marker smell in flight deck through final descent. High Pressure valves open. Both pilots smelled it. Odor isolation procedure used but couldn't isolate it to one pack. Both packs secured after landing. Aircraft taken out of service. Flight Attendants did not smell it. No announcements made.

Synopsis

Air Carrier pilot reported a fume event during final descent. Aircraft landed safely and taxied to the gate where removed the aircraft was removed from service.

ACN: 1911504 (583 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1911504
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1911507
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft X. I was the Pilot Monitoring (PM) on this leg. Uneventful descent into ZZZ. We were vectored for a visual approach XXR and at some point cleared for the visual. We backed this up with the ILS XXR. Just prior to G/S intercept the gear was selected down. Shortly thereafter we got a Master Warning and ECAM L/G not down locked. This was confirmed by the red nose gear light and both LGCIUs red on the wheel page. At this point I ordered go-around so we could deal with this at a higher altitude. Got a vector and 4,000 ft. once we were stabilized, I ran the ECAM. We selected flaps 2 and 180 kts. We recycled the gear and still had an unsafe nose gear indication. At this time we requested priority handling and requested Airport Rescue and Firefighting (ARFF) to be standing by. Flight Attendant (FA) A was briefed after the go-around and after the ECAM procedure. Passengers were also informed after the go around and later again what our situation was. The COMMs were run by me and also the landing with abnormal gear. At this point we switched Pilot Flying (PF) PM duties. We needed to burn fuel and we decided to begin the approach with about 5000 lbs. We stayed at flaps 2 and 180 kts and gear down to burn the fuel down. During this we elected to do a Tower fly by at 1,000 ft. AGL on Runway XYR. At Delta, a crew waiting for take-off reported that the gear looked normal. We requested vectors for Runway XYL. Runway XYR is the longer runway but it has major construction at the roll out end on the right side. We felt it could potential be danger for the passengers if an evacuation was needed. Also ARFF might be hindered. The approach to runway XYL was flown by me and I elected to use manual thrust to better control the touch down. VLS was 122 kts. Touch down was smooth and as the nose started to come down we shut engines off. We had started the APU in the air. We roll out to the last 1,500 ft. and stopped close to the end truck. The confirmed no damage, told the passengers to remain seated at this time. Maintenance came and pinned the gear and towed us to the gate. After the passengers were gone we debriefed with the FAs.

Narrative: 2

Aircraft X. I was the Pilot Flying on A/C X ZZZ1-ZZZ. Name was the Pilot Monitoring. On the visual approach to XXR at ZZZ, after selecting gear down, we got L/G NOT DOWN/LOCKED warning and associated ECAM for the Nose Gear. We executed a go-around and received delay vectors. We executed ECAM action and COM action, and after the gravity extension was executed the warning persisted and both LGCIUs indicated red and the landing gear position indicator also indicated red. At this point, Pilot Flying (PF) and Pilot Monitoring (PM) rolls were exchanged accordingly throughout the remainder of the flight to properly balance workload and pilot tasks. We requested priority handling with ATC and requested Air Rescue and Firefighting (ARFF) upon arrival. Proper communication was made to the Flight Attendants (FAs) and passengers in regards to the situation, what was going on, and what to expect. Positive contact was made with Dispatch, as well as ZZZ station ops who relayed information to ZZZ maintenance personnel. An attempt was made to reach maintenance via AIRINC, however this was unsuccessful most likely due to low altitude. We were tankering fuel, so we took delay vectors for some time in order to land at roughly 5,000 lbs FOB. We conducted a low approach on Runway XYR at approximately 1500 ft. MSL. We did this to allow personnel in the Tower then verify if the gear appeared down. After the low approach, Tower Controllers and another air crew holding short of XYR relayed that the nose gear appeared to be in normal configuration. Although XYR is the longest runway, we elected to land on XYL. XYR has significant construction within the vicinity of the departure end of the runway as well as traffic awaiting departure on Taxiway Y at the time of arrival. XYL provided the least external danger to the landing, as well as sufficient runway. Prior to landing we started the APU. Name was the PF and executed the landing on XYL in compliance to the COM procedure for Landing with Abnormal Landing Gear (nose gear). Just prior to the nose gear touching down, both engine masters were selected off. We gently came to a stop with approximately 1,500 ft. of runway remaining. ARFF inspected the aircraft and deemed it safe with nothing out of the ordinary. We did not push Engine/APU fire push buttons and we did not discharge Engine/APU agents. Crew and passengers were ordered to remain seated. Maintenance personnel arrived at the aircraft and secured landing gear pins in the nose and main gear. We were towed to the gate and deplaned using normal procedure. Crew debriefed together after all passengers were off the aircraft.

Synopsis

A320 flight crew reported landing gear failed to report extended on gear selected down. The flight crew requested priority handling and performed a go-around. After running the check list and QRH procedures, they performed a fly by to ascertain the landing gear condition. The flight crew made a precautionary landing and were towed to the gate with landing gear pins installed.

ACN: 1911489 (584 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1911489
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

After push back with one engine operating, ground crew asked me for parking brake, I set the parking brake ON, and asked him to remove, disconnect, and give me signals. Shortly after, the airplane started to move forward, and as I applied the manual brakes, the [pushback crew], requested me to apply the brakes. I asked him if everything was good, and he replied yes. I confirmed that the parking brake was on, and at that moment, there was no pressure on the indicator. I communicated to the [pushback crew] that we lost the parking brake, and might have to go back to the gate. I recycled the parking brake lever several times, but did not work. [Pushback crew] told me the tow bar was already disconnected and he was only plugged with the headset. I asked him to disconnect and give me signals in order to coordinate accordingly. Then I called Maintenance ZZZ, and asked for any advice. They told me to call ZZZ1, which is normal procedure. At that point, I recycled the A/Skid & N/W. STRG switch, and the parking brake was recovered. I checked all indications and parameters, and after several cycles, the operation of the parking brake was normal. We continued with Aircraft X uneventful to ZZZ1.

Synopsis

A320 pilot reported the parking brake failed during pushback but was recovered after consultation with maintenance.

ACN: 1911475 (585 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1911475
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While taxiing out for departure in a very heavy rain storm, inflight crew called the flight deck to advise they were experiencing a fumes event. It was explained to us only 2 of the 4 crew members smelled the fumes and that it was not extremely terrible. After referring to the Flight Crew Operations Manual procedure for odor isolation, we successfully stopped the fumes from continuing to enter the cabin any further. With the APU bleed off and the rain outside stopping, the flight was able to depart normally with no one complaining about odors. A few hours later on this same flight while descending through clouds for arrival, inflight crew advised they were experiencing another fumes event. This time the event was a little more pronounced with both pilots getting the odor in the flight deck as well. Once we were clear of the clouds and all outside moisture, the fumes/odor dissipated. Landing was successful with [a] logbook entry of level 1 fumes event.

Synopsis

Air Carrier flight crew member reported a fume event during departure taxi which dissipated and resumed during destination descent temporally. A safe landing was completed and a logbook entry was submitted.

ACN: 1911181 (586 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1911181
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During our taxi to the gate in ZZZ, we received an AIR PACK 1 REG FAULT ECAM. We followed ECAM followed by the X procedure. Shortly thereafter, we turned the APU BLEED on when we entered the ramp area for our tow into the gate. We immediately smelled some sort of odor neither of us have ever experienced before. Having had fume events in the past, and all of them having the same smell, that of a dirty sock or gym locker room, this was a different smell. I immediately turned off the APU BLEED and at the same time, the lead FA called the flight deck and said she smelled an odor as well. The FO and I also opened up our cockpit window. There was no more evidence of the odor and we were towed into the gate. The FO called Operations to have them send maintenance out to meet us at the gate. We described everything and I asked if I should fill out the fumes/odor report, they said it wasn't necessary. I don't know why I questioned it, I should have filled it out. I am now deeming this situation as a possible fume event. Our configuration was both engines OFF, APU BLEED ON. The odor smelled like a burning chemical. In my past experience with fume events, my typical symptoms were sore throat, itchy eyes and the following day I feel as if I had brain fog. I didn't experience any of these immediate symptoms, which is why I initially disregarded it as a fume event. The next morning, the FO and I both had headaches. Later that afternoon, I started to feel like I had brain fog and not being able to think clear and a sore/scratchy throat. I received a text from one of the flight attends the same afternoon saying she didn't feel well all day and her text also said some of the passengers in the front of the cabin were coughing. That was the first time I heard about the passengers. When we arrived at the airplane in ZZZ the following day, I spoke with the mechanics from the previous evening. I asked what happened with the aircraft and they told me they "could not duplicate the smell." I asked Maintenance Control if that aircraft had any follow up write ups regarding the same ECAM or any fume events since, and there were none. I looked back at the whole situation and decided that it is best to write a fumes report as well as an safety report.

Synopsis

A320 flight crew reported a fumes event during arrival taxi which was different from any other fumes the flight crew had experienced previously. Flight crew and a Flight Attendant reported "brain fog," headaches, and sore throat symptoms. Maintenance could not duplicate the issue.

ACN: 1911017 (587 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class C : ZZZ

Person

Location Of Person.Aircraft : X
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 8
ASRS Report Number.Accession Number : 1911017
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X had previously been taken off of the ZZZZZ descend via STAR due to traffic. I left them on the lateral path of the STAR, but assigned altitudes. They were being vectored for an ILS approach to Runway XXL I issued Aircraft X a slight heading of 180 as a lead in to the base turn and a descent to 10000 ft. Several miles later I issued a heading of 260 for the base turn. I then noticed their ground speed was faster than I expected, so I issued the turn to final with an approach clearance. I let them fly approximately another 2 miles and noticed they were not turning or turning very slow which was going to take them across the localizer and into a higher MVA of 10900 ft. I then instructed Aircraft X to "tighten up the turn" and issued a low altitude alert with a climb to 10900 ft. I observed them start a climb then I gave "vector across the localizer" with a new heading of 020 and maintain 11000 ft. until established. Aircraft X landed without further incident The ZZZZZ arrival that Aircraft X was on has a bottom altitude of 10000 ft. but it also has a speed restriction of 210 knots. I didn't think twice about issuing 10000 ft. as Aircraft X neared the base turn because that is the bottom altitude of the STAR. I had done the same turn to final multiple times earlier in the shift without incident. Because I had issued the altitude as a descend and maintain the pilot did not fly the speed assigned on the arrival and I noticed too late. I was about 2.5 miles away from the localizer when I issued the [approach clearance] so I felt like it was plenty of time for them to join the localizer, but it would have been better for Aircraft X to be at a slower speed.

Synopsis

TRACON Controller reported they misjudged the rate of turn when vectoring an aircraft to the final approach course resulting in the aircraft crossing the final approach course and flying below the Minimum Vectoring Altitude.

ACN: 1910644 (588 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Taxi

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1910644

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On flight XX from ZZZ to ZZZ1 on Date there was a mild odor event. On taking our first break inflight informed they smelled an odor at the end of boarding. The smell had dissipated and was not a factor during the flight. The forward two inflight had smelled the odor, while the others had not. No customers reported the odor. When we first boarded the plane it smelled a little stuffy, but I asked the inflight if they had noticed anything on the inbound and they said no. We thought the stuffy smell was due to the heavy rain in the ZZZ area. Upon reaching ZZZ1 we started the APU to prepare for a single engine taxi. When we switched to the APU bleed a musty dirty sock smell was observed in the cockpit. The APU bleed was deselected and tech ops notified. A logbook entry was made and an Maintenance form filled out.

Synopsis

A320 Captain reported an intermittent fumes event initially during boarding which dissipated and returned during arrival taxi. Captain requested Maintenance and submitted a logbook entry as well as a completed company fume report.

ACN: 1910635 (589 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1910635
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon boarding the aircraft, ground air was being used with the APU shut down. As it was 90 degrees in the Cabin and the ground air was putting out 100 degree air, per the COND System page, I started the APU and went outside to conduct the preflight and request the ground air to be disconnected. PACK 2 was already on MEL and selected off. The First Officer turned the APU Bleed on once the ground air was turned off. During the preflight, I noticed oil on the underside of the tail, forward of and around the APU inlet flap. Upon entering the cockpit a few minutes later, I was immediately aware of a strong "dirty sock" odor. The First Officer said it smelled "musty." I immediately turned the APU Bleed off and called Operations to ask Maintenance to come to the aircraft. The Maintenance Technician briefly turned on the APU Bleed and then quickly turned it back off after also detecting the "dirty sock" odor. There were no inflight crew or customers on-board during this time because the inflight crew was still deplaning at a different Gate. Upon further review of the logbook, I read that the APU had had a shutdown event and oil contamination in the same area around the inlet flap a few flights earlier. Operations decided on a tail swap for our flight.

Synopsis

A320 flight crew reported a fumes event during pre-flight described as a dirty sock odor. Captain requested Maintenance which resulted in an aircraft swap.

ACN: 1910517 (590 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 13135
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 7619
ASRS Report Number.Accession Number : 1910517
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 15107
Experience.Flight Crew.Last 90 Days : 184
Experience.Flight Crew.Type : 7221
ASRS Report Number.Accession Number : 1910525
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As we started our descent into ZZZ near ZZZZZ intersection at FL320 we had an ECAM (Electronic Centralized Aircraft Monitor) HYD G RESR LO LVL. I flew while PM (Pilot Monitoring) ran the ECAM checklist. HYD Quantity was zero. Contacted Dispatch and informed them of the situation and requested runway data for Runway XXC and to have fire equipment and tug ready for our arrival. We determined that [requesting priority handling] with ATC would be helpful and the best course of action due to loss of steering and some of the brake functions along with 1 thrust reverser, as well as possibility of fire from all the fluid loss. I called FA (Flight Attendant) with a briefing of situation and informed her would be a normal landing with some precautions. I informed passengers of same. We requested a 20 mile final so we had extra time for slower flaps and manual gear extension. We slowed to 210 kts. and configured aircraft around 4,000 feet. Normal landing and slowed to safe taxi speed and made [Taxiway] 1 turnoff with differential braking and rudder. Stopped aircraft, set brakes and communicated with fire rescue. No issues, tug was at our aircraft in a few minutes to pull us back to gate after completing tow in checklist after starting APU and shutting down engines.

Narrative: 2

On descent into ZZZ approximately near ZZZZZ at FL320 we received an ECAM HYD G RSVR LO LVL. Captain continued flying while I ran the ECAM (Electronic Centralized Aircraft Monitor). HYD System Quantity showed near zero. After ECAM was complete I went into the FM (Flight Manual) to complete checklist. Captain called Dispatch and had them run landing distance for Runway XXC. [Priority handling was requested] due to manual gear extension/Hyd fluid gone (unsure where it leaked, possible fire) and no nose wheel steering along w no normal brakes. Captain briefed FAs (Flight Attendants) and briefed passengers. We requested 20 mile final due slower configuration and manual gear extension. We slowed to 210 knots and configured with gear at 4,000 feet. Normal landing and we were at a slow taxi speed near [Taxiway] 1 and the Captain was able to clear runway with braking. Stopped, coordinated with fire/rescue. No issues, tug was right there. We had started the APU and shut down engines and ran tow-in checklist. Plane towed to gate without incident. ZZZ did a great job coordinating with tug team/fire rescue etc. Ended up 12 mins early.

Synopsis

A320 flight crew reported loss of Green Hydraulic System fluid in flight. The flight Crew requested priority handling and made a precautionary landing at destination airport.

ACN: 1909557 (591 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1909557

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1909857

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When the throttles were moved forward for takeoff I got no response from the engines, so I continued to move the throttles slightly more foward still no response, I was getting no spool up at all. Then I felt a very slight buffeting and closed the throttles, and then got a momentary ECAM, that cleared itself as rapidly as it appeared, so we were unable to determine what it was. The takeoff was rejected and we taxied clear of the runway on to Taxiway 1 facing west. The Captain ran the engines up slightly and they responded normally, so we decided to try another takeoff. Moments later we got an ECAM high fuel temperature on the left outboard fuel tank (62)c which required us to turnoff #1 generator and shutdown #1 engine. Taxing back to the gate the Captain remarked that the #2 engine was very slow to respond to throttle input. The Captain wrote up both the rejected takeoff due to sluggish engine response and the fuel temp issue. The aircraft was taken out of service. Cause - Possibly the OAT which was 38c.

Narrative: 2

First officer takeoff RWY XL. We were cleared for takeoff, First Officer advanced the thrust levers to spool up and stabilize the engines before setting Flex thrust. Both engines were very slow spooling up and when they finally spooled up and stabilized we got a master caution. It came and disappeared immediately and unfortunately neither of us saw what it was and there was no record in the current leg ECAM report. We rejected the takeoff at a speed less than 10 knots and cleared the runway on the first available taxiway. Once we were stopped on the taxiway, I called maintenance and let them know what happened. While sitting on the taxiway we got a master warning for hot fuel, followed the ECAM and shut the number one engine down and returned to the gate. If the engines would have spooled up and stabilized faster, I believe there wouldn't have been an issue. Only suggestion is the flight crews need to be aware if the engines don't spool up in a timely fashion, it needs to be written up in the AML.

Synopsis

A320 flight crew reported executing two consecutive rejected takeoffs due to engine response issues during power up. The aircraft was taken out of service.

ACN: 1909542 (592 of 2908)

Time / Day

Date : 202206

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1909542

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight attendant called the flight deck to report a (misty, dirty sock) odor in the whole cabin. A few minutes later we could also smell it in the flight deck. This was late in the flight about 45 min until landing. I called Maintenance wrote it up in the log book and filled out a odor [report]. No diversion was needed. No medical attention was needed. No other factors played a roll in this event. Not sure where or what caused the smell.

Synopsis

Air carrier flight crew reported being notified of a "dirty socks" odor in the passenger cabin. Flilght crew called Maintenance and submitted an Aircraft Maintenance Logbook discrepancy as well as fumes reports.

ACN: 1909234 (593 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1909234
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Prior to boarding the aircraft out going FA's (Flight Attendants) advised us of a lithium battery having been dropped at row 23 seat A. They then left without further information. I immediately called Maintenance to help with finding the battery. Additionally, the cleaners were just finishing and I asked if they had found the battery and to dump the contents of the vacuum to see if it was in it. They said they cleaned row 23 but then quickly left without checking the contents of the vacuum. I began searching one row behind and in front so as to not hit the battery by stepping on it. ZZZ Maintenance arrived and began the search as well. At some point a woman not in uniform arrived. I had noticed the floor was filthy as well as the side wall. I made reference that this was making the search difficult. During the search this unknown woman began pulling seat cushions off seats in row 23. I had already discussed with ZZZ Maintenance and was told pulling the cushions wasn't necessary. Yet here was this person yanking seat cushions off. I asked if she was a Mechanic or a Maintenance Supervisor and received a very terse response that she was a manager. So no she was not and was in the row kicking around the debris and possibly the battery. I asked her to please stop and allow Maintenance to do the search. Along with giving me some time to figure out what to do. Her terse response along with telling me I have no authority and she does was enough for me to remove myself from the situation. I made a call to Operations to have her removed then I returned to my duties. Maintenance had done a quick search then left, there was an open log book entry. Maintenance informed my FO (First Officer) that he didn't find it and if we didn't feel safe then don't take it. At this point having spoken to our Dispatcher we once again asked for the cabin to be cleaned and more eyes from Maintenance to see if it could be located. Before anyone arrived the battery was found on the jet bridge. In bound crew left without providing any additional information. It was apparent they had not informed their Captain. Cabin cleaning needs to do a better job, the floor was filthy and the side walls as well. Having non qualified personnel getting involved just made the situation worse. Her behavior alone with her insistence that I had no authority and she did only delayed what we were doing. Maintenance seemed unconcerned, apparently it's not common knowledge that a ruptured lithium battery can ignite. FA's need to understand that informing the Captain and getting good information to us is critical. Non qualified personnel need to stay in their area not get in the way of flight deck crews responsibly. An extra set of eyes is one thing but this person's behavior and actions only hindered our effort. Cabin cleaning needs to be done, not just run the vacuum up the isle then run off. Better understanding of what a lithium battery can do.

Synopsis

Air carrier Captain reported being notified by out going Flight Attendant that a Lithium Ion battery was dropped in a row and not recovered. After extensive search the battery was recovered and flight departed safely.

ACN: 1908730 (594 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1908730

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after takeoff in ZZZ we had an AUTO BRAKE FAULT / BRAKE RELEASED ECAM MSG. All QRH procedures were accomplished. Due to our original destination (ZZZ1) having a short/wet runway and performance penalties applied we needed a new destination. We talked to dispatch/maintenance over the SATCOM and ACARS. We coordinated with dispatch to make ZZZ2 our new destination where we landed without further issues. The weather in the northeast closed the ZZZ center airspace and we were rerouted due to our destination change. The flight time and fuel burned increased significantly and dispatch had to used exemption XXXXX to void the requirement of and alternate with ZZZ2 as the new destination.

Synopsis

A320 Pilot reported AUTO BRAKE FAULT / BRAKE RELEASED ECAM MSG was illuminated on climb. The flight crew diverted to make a precautionary landing.

ACN: 1908727 (595 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 9000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Parked
Flight Phase : Taxi
Flight Phase : Final Approach

Component

Aircraft Component : Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1908727
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : In-flight
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent through 9000 ft., a slight oily smell was noted in the cockpit by both pilots. The APU was not in use at the time. After we blocked in and the cabin door was opened, a noticeable oily smell was noticed throughout the cabin. The smell persisted. About 1 hour later, the smell was still apparent in the cabin and cockpit and was noted by both pilots and operations. A logbook entry was made, the odor form was completed, the odor survey was completed, and a report was submitted. Aircraft was removed from service.

Synopsis

Air Carrier flight crew reported an "oily smell" odor during descent until gate arrival. An AML discrepancy was submitted resulting in aircraft being removed from service.

ACN: 1908722 (596 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1908722
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Departed ZZZ for ZZZZ1. On initial climb out as we leveled at 5,000 ft. we received an ECAM ENG2 Compressor Vane. As we accomplished the Memory Items, QRC and ECAM Actions the ECAM went away. I established that the First Officer was the Pilot Flying and I coordinated via SATCOM with our dispatcher and Maintenance Control. After conferencing with a Supervisor Maintenance Control made the decision to have us return to ZZZ. The return to ZZZ was uneventful and no request for priority handling was made as all systems were operational and no ECAM was present. The aircraft weight at landing was 145,200 lbs. with a max landing weight variant 145,500 lbs. We did discuss and review the Overweight Landing checklist as a precaution. The touch down was light with normal breaking and reverse thrust. As we exited the runway the same ECAM did appear again. A logbook entry was made and the aircraft was turned over to ZZZ Maintenance Control.

Synopsis

A320 Pilot reported Compressor Vane ECAM message on climb out. The flight crew performed an air turn back and precautionary landing at departure airport.

ACN: 1908719 (597 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1908719
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We arrived at the aircraft at XA20L for the first flight of the day for the aircraft. The #2 pack was operating off the APU bleed. A moderate dirty sock smell was noticed immediately upon boarding, with the odor increasingly strong farther aft. A review of the logbook revealed several "odor" events going back at least two weeks. The latest one was two days prior, after which an attempted pack burn caused pack #1 to overheat. Pack #1 was then deferred. I refused the airplane due to ECS (Environmental Control System) contamination. I contacted Dispatch to get the ball rolling on a tail swap and he forwarded my call to the Maintenance Controller. The Maintenance Controller informed me that the airplane was airworthy and to call my Chief Pilot and then hung up. A few minutes later, the duty Chief Pilot contacted me and we discussed options. Since I hadn't yet written up the new event, the airplane was technically airworthy, so I wrote up the fume event, filled out the odor report form, and awaited further developments. About 1 1/2 hrs later, we were informed that the aircraft was ready, so we returned. The corrective action performed was to defer the APU. Maintenance did perform an engine run to operate the remaining pack from the engine bleeds to check the pack for odors. This was the extent of their investigation. There was no ECS decontamination. Consulting further with the duty Chief Pilot presented me with 3 options: accept the airplane, refuse the airplane, or verify the maintenance work by performing my own engine run. I found out later, but was never once explained to me at the time, that the procedure for a confirmed ECS contamination event *requires* a decontamination and a thorough check to make sure further contamination is halted. Instead, I was to take the word of the Maintenance staff that all was correct since they were following FAA approved procedures, and the corrective action actually performed was appropriate to "get the flight out". I was deeply uncomfortable with the situation, seemingly the only person involved (other than the rest of my crew) to be so. Certainly not a single layer of operational support (other than Dispatch, who was powerless) - including Line Maintenance, Maintenance Control, or the duty Chief Pilot - conveyed any sense that what was occurring was in any way cause for concern or further inquiry. I was told repeatedly that I would have final say, but I felt completely unsupported that my professional judgment was in any way welcome or worth heeding. Calling a safety timeout is one of the most important decisions I need to make as Captain, and in this case, it took four hours and an hour or more of virtual gaslighting to finally be able to make it stick. I felt as though I was the only person out of line, and who was I to question the professional judgment of everyone else? It was the closest I have felt to pilot pushing in more than XX years with the airline.

Synopsis

Air Carrier Captain reported a fumes event upon arrival for preflight. An AML discrepancy was submitted which resulted in a communications breakdown with Maintenance Control and an extensive delay before departing.

ACN: 1908717 (598 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1908717
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent in to ZZZ around 10,000 ft. the FO (First Officer) and I smelled what we thought was a "chemical" smell. Very shortly after that the cabin crew called up and reported the odor/fumes. We were below 10,000 ft. busy talking to Approach Control. We elected to continue what was almost an immediate landing thereby skipping any troubleshooting.

Synopsis

A320 Captain reported a fumes event during approach that was also detected by the cabin crew. Flight crew landed safely without troubleshooting do to the proximity to the airport.

ACN: 1908715 (599 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1908715

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departing ZZZ on climbout ECAM for yellow reservoir overheat. Assigned flying FO (First Officer). Complied with SOP ECAM actions and QRH. Briefing information shared with inflight team. Contacted Dispatch/ Operations. Spoke with Duty Pilot. While speaking with Duty Pilot, light fault light extinguished Once on Approach informed of situation. Configured ship early (lights remained out) reinstated as per QRH and landed normally and taxi to gate. Safety equipment was standing by. Achieved gate normally. Deplaned normally. No questions from crew or customers. Logbook completed. Verbal brief Assistant Chief Pilot.

Synopsis

A320 flight crew reported a Hydraulic Reservoir Overheat on climbout.

ACN: 1908651 (600 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1908651
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Ground Personnel
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After push back, the Ramp Crew advised me to set the brakes. I thought l had, however I noticed one of the ramp crew waving their wands. I then noticed that the airplane was slowly moving and immediately applied the brakes. No one was hurt and no equipment was damaged. This event has really shook me up. I was sure I had set the brakes, but the parking brake worked correctly for the rest of the flight, so obviously I did not. Contributing to this event was a dark ramp and that I had the dome light on in the cockpit. Obviously, attention to detail on my part. I will also have the dome light off during push back.

Synopsis

A320 flight crew reported the aircraft rolled forwards uncontrolled after the flight crew failed to set the brakes after push back.