ACN: 1908631 (601 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1908631

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1908642

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After engines stabilized, flex thrust was set, aircraft turned abruptly right. Full left rudder and nose wheel steering did not help. Rejected the takeoff at approximately 50 kts. After stopping and assessing we taxied off the runway and then to the gate. Aircraft taxied normally. Engine's acted fine. Completely surprised by this event and don't know the reason or cause.

Narrative: 2

Cleared for takeoff from before the hold short line for [Runway] XXL. Power was added to taxi out onto runway and line up. When pushing power up after engine stabilization the aircraft turned sharply right and we aborted. Not sure what happened from 50% to flex power. They looked like they were coming up ok as I heard the clicks of the power levers. The aircraft was already going right before we reached flex power. At that point I heard reject as Pilot Flying was outside noticing the turn before I could. I looked at the speed, I remember seeing a 4. Definitely was below 80 kts. No spoilers/auto brake activated. Stopped and assessed. Decided safe to taxi off. Returned to gate for Maintenance to check it out. Possibly a sticky brake. I don't think it was a engine stabilization issue. PF (Pilot Flying) had power up to taxi onto the runway so they were spooled. There was a time constraint of making our wheels up time but we were right at it so I didn't feel rushed. Maybe less rolling takeoffs would prevent the large abrupt turns when something goes wrong that would lead to a rejected take off.

Synopsis

A320 flight crew reported the aircraft veered right after applying takeoff power resulting in an aborted take off and a return to the gate.

ACN: 1908090 (602 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Flight Phase : Cruise
Route In Use : Vectors

Component : 1

Aircraft Component : Powerplant Fuel Control Unit
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Engine Air
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : FADEC / TCC
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 165
Experience.Flight Crew.Type : 11706
ASRS Report Number.Accession Number : 1908090
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 209
Experience.Flight Crew.Type : 929
ASRS Report Number.Accession Number : 1908093
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Reaching cruise altitude, we observed a status message, STS (Status), flashing on and off approximately every one or two seconds. Status page displayed â€œONE PACK ONLY IF WAI (Wing Anti-Ice) ONâ€ and kept going on and off with the STS for duration of flight. I wrote it up and then noticed a CFDS (Centralized Fault Display System) â€œBMC2 (Bleed Management Computer 2) OR SOLENOID 10HAW CKTâ€ and I sent info to Maintenance Control aircraft desk for informational purposes. A short time later ECAM â€œENG 2 ONE TLA (Thrust Lever Angle) FAULTâ€ displayed. This cleared on its own after less than a minute. I still made a Maintenance Request entry as there was a log history for this and then this ECAM would come back on and off several times for the duration of the flight. There were no other indications. On approach to ZZZZ with the autothrottles on, ECAM â€œENG 2 THR LEVER DISAGREEâ€ displayed. Slats and flaps were extended. ECAM directed the thrust lever affected side to idle. Status page said no thrust reversers that side. Engine thrust for engine 2 would not increase above idle. First Officer was the pilot flying and I was comfortable with his experience on the airplane and had him continue to fly as I [requested priority handling] with ZZZZ Tower. I informed them both engines were still running but we had an engine that lost thrust and was at idle power only. I sent an ACARS to Dispatch informing them of the [priority handling] and that we were on approach. Landing and taxi-in were without incident. Required calls and reports were made in compliance with the Maintenance Facility.

Narrative: 2

Reaching cruise altitude after takeoff from ZZZ we observed a status message, STS (Status), flashing on and off approximately every one or two seconds. Status page displayed â€œONE PACK ONLY IF WAI (Wing Anti-Ice) ONâ€ and kept going on and off with the STS for the duration of the flight. The Captain wrote it up and then noticed a CFDS (Centralized Fault Display System) â€œBMC2 (Bleed Management Computer 2) OR SOLENOID 10HAW CKTâ€ and he sent that information to Maintenance Control aircraft desk for informational purposes. A short time later ECAM â€œENG 2 ONE TLA FAULTâ€ displayed. This cleared on its own after less than one minute. A Maintenance Request entry was made and there was a history of this ECAM. The ECAM appeared several more times for the duration of the flight. There were no other indications. On approach to ZZZZ with the autothrottles on, ECAM â€œENG 2 THR LEVER DISAGREEâ€ displayed. Slats and flaps were already extended. ECAM directed the thrust lever affected side to idle. Status page said no thrust reverser that side. Engine thrust for engine two would not increase above idle. I was the pilot flying and was comfortable with the situation and continued to fly the approach. The Captain [requested priority handling] with ZZZZ Tower since we were already on their frequency. He informed the Tower both engines were running but the number two engine was stuck at idle thrust. An ACARS message was sent to Dispatch informing them of the [priority handling] and that we were on final approach. Landing and taxi-in was without incident. Required calls and reports were made by the Captain in accordance with Maintenance Facility.

Synopsis

A320 flight crew reported loss of engine thrust and engine control for engine #2. Flight crew requested priority handling and made a precautionary landing at destination airport.

ACN: 1907728 (603 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : ELAC1
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : FCC (Flight Control Computer)
Manufacturer : ELAC2
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1907728
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X I was operating ZZZ-ZZZ1 in Aircraft X. We departed Runway XXL on the ZZZZZ departure with a climb via SID clearance to 17,000 ft. Push back and taxi out were normal, takeoff roll was normal. Just after liftoff as the gear were retracting I noticed the amber "X"s appear on the PFD as the aircraft reverted to alternate law. There were no immediate ECAMs. We cleaned up the aircraft and accelerated to 250 kts. and asked to level off at 5,000 ft. to fly present heading to trouble shoot. The First Officer began running the ECAM actions while I communicated with ATC and the Flight Attendants. There was only an ECAM for alternate law, there were no other ECAMs at this time and there were no fault lights on the overhead panel. When we got to the status page is showed ELAC 1+2 as INOP systems. We made the decision to return to ZZZ as it wasn't clear why the ELACs had both failed. We also [requested priority handling] to get back to ZZZ as well as in case we had any further flight control issues as we weren't sure why both ELACs had failed at the same time. The weather was VFR conditions. We set up for a visual approach backed up with ILS to Runway XXL as that was our return runway in the secondary flight plan as well as the longest runway. We sent an ACARS message to Dispatch advising them of our situation and communicated with the Flight Attendants and Passengers. Landing in direct law was uneventful and after landing CFR checked out the aircraft and we taxied back to the gate uneventfully. After we had arrived at the gate and ran the parking checklist the Flight Attendants in the back of the aircraft reported that they had heard a banging or thumping sound in the back of the aircraft while we were flying.

Synopsis

A320 Captain reported loss of both ELACs after take off. The flight crew performed an air turn back and precautionary landing at departure airport.

ACN: 1907623 (604 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1907623
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Leveling off at 32000 ft., we noticed cabin altitude on lower ECAM pulsating ADV message and indicating 9000 ft. Cabin altitude. Cabin vertical speed was continuing to climb at 100ft./minute while in level flight. Requested and received clearance to descend to 30000 ft. As power came off to descend, Cabin vertical speed increased to 500 ft./minute. Added thrust in descent to abate the increased cabin vertical speed, which then reduced to 100ft./minute. Cabin reached 10000 ft. while leveling off at 30000 ft. triggering ECAM Warning. Crew donned O2 masks, established communication and began priority descent to 10000 ft. Initially received clearance to 18000 ft. from ATC but prior to reaching, [requested priority handling] and received further clearance to 10000ft. Requested vectors and clearance back to ZZZ1 at 10000 ft. Finished QRH procedure, conferred with dispatch via SAT1 and informed Flight Attendants/ Customers of our pressurization issue and plan to return to ZZZ1. Issued Flight Attendants [instructions and threat status]. Prepared for overweight landing at 156000 lbs. on Runway XXL with Crash Fire Rescue (CFR) waiting in case of hot brakes. Touched down at less than 200ft./minute, applied max thrust reversers and minimal brakes. Exited XXL at Taxiway X, where CFR was positioned. Brake temp never climbed above 150 degrees. Taxied back to gate and handed aircraft off to Maintenance for repair. Customers were understanding as they deplaned. Assigned new tail, departed approximately 90 minutes later from [new] gate.

Synopsis

A320 Pilot reported loss of cabin pressure and a return to their departure airport where they made an overweight landing.

ACN: 1907620 (605 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1801-2400

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Cabin Activity : Service
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1907620
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

25,000 dry musty odor/fume smell cockpit. ZZZ-ZZZ1 around XA22Z dry musty odor/fume smell in cockpit during descent level off 25,000 ft. Both pilots experienced the same event. Followed procedures, logbook write up, cabin odor sheet and referenced FOM. FAs (Flight Attendant) reported no odor/fume in cabin. Both pilots agreed no need for medlink.

Synopsis

A320 Flight Attendant reported a fumes event during descent which was localized to the cockpit. Flight crew completed the appropriate fumes procedures and submitted company fumes forms as well as a logbook discrepancy write up upon gate arrival.

ACN: 1906740 (606 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1906740
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On arrival at the aircraft at the gate, my FO (First Officer) and I both noticed a pervasive crayon, or dirty sock smell in the forward cabin and cockpit. The APU was running the packs. I queried the inflight crew who also noticed the smell in the forward cabin. The inbound crew mentioned that they had also noticed the same odor on descent on the inbound flight. I made a logbook entry, notified Dispatch, Maintenance Control and contacted MedLink who advised that I seek evaluation at a local clinic. Boarding had not yet begun.

Synopsis

A320 Captain reported a pervasive dirty socks odor throughout the aircraft prior to passenger boarding. Captain submitted a logbook entry and notified Maintenance and Dispatch.

ACN: 1906730 (607 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors
Airspace.Class E : ZZZ

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1906730

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were level at 7,000 ft. after departing from ZZZ when we received a #2 ENG Compressor Vane fault. We asked ATC for vectors while we conferred with Dispatch and [maintenance].The QRH was referenced. Maintenance suggested and Dispatch concurred that we return to ZZZ. We informed the crew and customers and ATC. We [requested priority handling] and landed under max landing weight and taxied to the gate under our own power. A logbook entry was made.

Synopsis

Pilot reported a Compressor Vane ECAM message on climb out. The flight crew requested conducted an air turn back and precautionary landing at departure airport.

ACN: 1906728 (608 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component : 1

Aircraft Component : Fuel Crossfeed
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Fuel Trim Pump
Aircraft Reference : X
Problem : Improperly Operated

Component : 3

Aircraft Component : Minimum Equipment List (MEL)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1906728
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Ground Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We began the day with a XA20 report in ZZZ1 to operate Aircraft X to ZZZ. Other than a delay due to customers, there were no other issues. A few months ago, I had an MEL regarding the BSCU (Braking and Steering Control Unit) unit that at first glance said we could not be dispatched to an airport with a 5G NOTAM. Another pilot and I looked deeper, figured out the MEL would allow us to go, although it wasn't clearly written. Of course, we would not get dispatched an airplane that an MEL would not allow to go to that destination. This was my mindset when we looked over this MEL as well. The plane arrived at our departure time. I never got the AREQ I requested which I like to study before a flight. Once the plane arrived, we were rushed. After reading the MEL XX-XX-XXX, the FO (First Officer) and I discussed where both center fuel pumps were deferred. We were confused, because we knew we needed the center tank to get to ZZZ2 and since we had 37.1 in total fuel, we were missing something. The front of the Release had the MEL listed as well as 37.1 lbs of fuel. Again, my mindset was we would not get dispatched with an improper MEL. After all, no one would dispatch a plane that was capable. I figured I was missing something, and the FO and I discussed it further. We both agreed that the scavenger pumps must kick on once the engines are started and that it was the electric pumps that must be deferred. We confused the system of the 321 to the 320 which doesn't have scavenger pumps for the center tanks. We again discussed also that the plane would not have been dispatched to ZZZ2 if the MEL did not allow it. Once on board, we noticed that the mode selector for the center fuel tank was left in manual by the previous crew. We looked, but nowhere in the MEL does it say to select it to manual. The FO looked up in FCOM that the FLSCU (Fuel Level Sensing Control Unit) controlled the valves for fuel transfer to the wing tanks, by placing the MODE SEL to MAN it would force open the FLSCU valves allowing fuel transfer. This part of the FCOM does not specify between the 320 and the 321. This enforced our thoughts that the scavenger pumps would transfer once the engines were running. So, since the MEL said the fuel was unusable, we thought only while using the electric pumps, but not when the scavenger pumps were operating. Which we now know, the 320 doesn't have any. Maintenance was on board for another issue, and we asked for their thoughts. We had the Fuel page up showing the 10.0 lbs of fuel in the center tank. Maintenance read the MEL and looked at the Fuel page. We discussed the FCOM and FLSCU, and Maintenance said we could leave it in auto. Now here is what I believe to be the misconnect. Thinking about the MEL I asked, "If we leave it in auto, would it transfer fuel in the air?" Maintenance said "Yes." I truly believe, in hindsight, that during our conversation, I was asking about the fuel pumps working regarding the current situation, and I think Maintenance was not connecting it to the MEL but was responding about the system in general. Because our conversation involved more than just the MEL, a miscommunication occurred. However, this is the point where I felt confident to continue as I had done my due diligence and gotten clarification from maintenance: with the erroneous release, the unspecific model in the FCOM, and the discussion with Maintenance led us to mistakenly think we had the fuel as planned. Then we noticed in the air that the fuel was not transferring. We immediately got in contact with Dispatch starting with a satellite call along with Maintenance. Maintenance now informed us there was no way for the fuel to come out of the center tanks. Of course, this was the only thing we were discussing. The Dispatcher and I discussed our option, which included a stop in ZZZ3. At the time, we had an EFOB of 15.3 minus the 10.0 in the center tank which gave us an actual EFOB of 5.3. This is typical when landing in ZZZ4. We discussed it and decided to continue to monitor and make a decision as we went along. We discussed it every hour with the Dispatcher and the fuel stayed steady at an EFOB of 15.1-15.3. We decided abeam ZZZ5 would be our final go-no go decision. When we reached abeam ZZZ5 we were still showing 15.0 and the weather was fine at ZZZ2. After a discussion with Dispatch, we felt safe continuing. As we started the arrival, ZZZ2 Center shut down the arrivals due to a line of weather further east of the field along the arrival. We asked for an immediate turn back to ZZZ3. Once in the turn, the EFOB said 14.2 at ZZZ3 and ZZZ6, but 15.2 at ZZZ5. We elected to go to the nearest suitable airport. Once on the ground, we were quickly serviced, had the new dispatch, fuel and ZZZ2 had lifted the ground stop. We were wrapping up when the Maintenance desk called to say we were illegal due to [duty policy]. With the additional leg, it would have put us over 9 hours. We taxied to a gate and deplaned, where they were accommodated on another flight.

Synopsis

Captain reported a misapplied MEL and erroneous Dispatch release resulted in a fuel issue that required a diversion to refuel and continue to destination airport.

ACN: 1906718 (609 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Oceanic
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1906718

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the pilot not flying (PNF) and FO Name was the pilot flying (PF). After an uneventful departure and initial cruise at FL350, ATC routed us direct to ZZZZZ intersection, where we remained in radar contact and VHF coverage. I completed the cruise checks on the system status pages and noted no abnormalities. Approximately 1 hour after takeoff and 100 NM prior to ZZZZZ we received an AIR (L) WING LEAK ECAM. FO Name remained the PF and I ran the ECAM and checklists. I noted that on the system status bleed page the engine bleed valve (L) had closed and the crossed bleed valve was shut. After finishing the ECAM and running the QRH, I used the SAT phone to contact both Dispatch and Operations to discuss the situation. We agreed that the most prudent option was an air-return to ZZZ as we lost the left pack as well as the use of anti-ice (and cloud avoidance couldn't be guaranteed, especially given the weather enroute due to the tropical depression). In the discussion of our circumstance with ATC, FO Name stated that ATC "initiated the [priority handling' on their end" and after I was brought in the loop, I elected to remain in an priority status and not downgrade it as I wanted to ensure that we had the ability to remain clear of clouds. Once I finished speaking to the company, I notified the flight attendants of our intentions to return to ZZZ, with normal cabin procedures. We noted that we were dispatched with tankering fuel, and therefore would need to burn approximately 800 lbs. to get below max landing weight. I asked for several early step-downs (as there were no clouds between us and ZZZ) as well as running the APU (but ensuring not to open the bleed valve, especially due to the wing leak). We made an uneventful approach and landing on XXL at ZZZ (below max landing weight) and parked next to the spare aircraft that we would then use to fly back to ZZZZ.

Synopsis

A320 Captain reported pressurization pack failed which resulted in a decision to return to the departure airport.

ACN: 1906299 (610 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Flight Director
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1906299
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.No Specific Anomaly Occurred : Unwanted Situation
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

First flight of a 3-leg day on a planned 6-leg 3-day sequence. Conducted thorough briefings with all Flight Attendants and the First Officer. Must have glanced right over the Flight Director lights on preflight. During the Before Takeoff Checklist "Flight Instruments and Bugs" Challenge, I could have sworn I saw a pink over blue bugs on the dim basic screens amidst the bright direct sunlight, and now realize I had dropped the 1 Flight Director 2 from my habitual cross check for that item, even though that isn't in the AOM. Cleared for takeoff, lined up on centerline and advanced the throttles to the second detent for a rolling flex takeoff. Did not see "FLEX" on the FMA, and selected Takeoff Go Around to see if that would help. After seeing that did not result in "Takeoff Go Around" either, I called "Reject, My Aircraft" at what I thought was just shy of 80 KIAS. Max auto-braking commenced, which I attempted unsuccessfully to override with manual braking for a couple seconds. As the aircraft came to a rocking stop, I pressed the PA button and stated " This is the Captain, remain seated, remain seated, remain seated" and the First Officer advised ATC. We taxied off the runway on X, and followed Ground's instructions to turn left on Y and hold short of XY. I made another PA stating we had an unsafe cockpit indication after applying takeoff thrust, and would need to turn to the gate for maintenance. The First Officer was first to realize the Flight Directors were off as we taxied clear of the runway, and quickly concluded my preflight oversight led to our situation. I called Dispatch connect to the Maintenance Center, who informed us a maintenance inspection would be required for any reject at or above 80 KIAS. We returned to the same departure gate where I coordinated with the #1 and Gate Agent before making the FOM DOT Extended Delay (opportunity to deplane) PA, and sent an ACARS message to Dispatch with the PA time XA:13. 1. Missed Flight Director lights on preflight. 2. Distracted by limited time to prepare for an impromptu vacation, devised after losing scheduled vacation to COVID illness. 3. Had been trying to drop this trip for over a week, but with Chiefs only authorized to must-have Employee Off days for family emergencies, I ended up preparing as much as I could for a 7 day vacation day prior to the sequence, knowing I had a list of things to accomplish in the eight hours between the conclusion of my sequence and departing with my family for the airport. That wouldn't have been necessary if our system allowed the same credit as actual pay for SA days; a systemic problem that creates an 18-day work month. Another solution commonly available to most corporate employees in the U.S. would be converting vacation lost to COVID to sick days, and permitting the employee to convert later work days to the vacation lost. Vacation best maintains employee mental health and job satisfaction when it is dedicated to quality family time, not to quarantining at home with active COVID. 1. Consider adding a 1 Flight Director 2 check to the Takeoff Data and Speeds item of the Before Takeoff Checklist. 2. Make sick time credit equal to pay. 3. Allow pilots who can document sick during vacation to convert days to sick and gain back vacation time balance 4. Permit must-have Employee Off days on a case by case basis without overly constraining the evaluation criteria.

Synopsis

Air carrier Captain reported the Flight Directors were not turned on which resulted in no airspeed bugs displaying during the takeoff roll. After the takeoff roll started the takeoff was rejected by the Captain and the flight returned to the gate.

ACN: 1905790 (611 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1905790
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Flight XXXX was delayed over an hour thirty, due to a late arriving group of passengers. We were trying to coordinate with gate, ramp and operations to get the flight underway. Upon close out we were told it would be a hand signal push because they could not find a headset. We ran the before start checklist, and in the moment of coordinating the push with hand signals we missed the permission to push message. We immediately recognized our error, but the push was already underway. We continued the push to a stop set the brake and received the permission and T.O. data.

Synopsis

A320 pilot reported pushing back from gate without clearance.

ACN: 1905788 (612 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1905788
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Turned on APU bleed after APU start. Noticed a bad odor about 30 seconds after bleed application (wet socks). Odor was weak at first but grew stronger. F/O (First Officer) noticed it as well and we started to comment on it. At same time all flight attendants started to inquire about smell. Two where located in back. Aircraft had no passengers at this point. I immediately turned APU bleed off. Smell began to dissipate after about 3 minutes. Also of note, it was raining very hard and there was a ground ops guy standing on jet-bridge. He entered cockpit and asked what that smell was as well. We called Maintenance and exited the aircraft to limit exposure.

Synopsis

Air carrier Captain reported a fume event during preflight described as "wet socks" prompting the Captain to turn off APU Bleeds and request maintenance. Flight crew exited the aircraft.

ACN: 1905777 (613 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1905777
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Arrived at aircraft and found ground air not supplying cool air sufficiently. Started APU, disconnected external air, and selected APU bleed air for cooling. Flight Attendants boarded airplane approximately 5 minutes later and advised me of "dirty socks" odor in mid/aft cabin. I called maintenance, wrote the item of concern in the maintenance logbook, and filled out odor event form. Maintenance arrived and told me they were most likely going to defer the APU. Aircraft was briefly taken out of service. Maintenance deferred APU, Aircraft X was then re-dispatched. No odors reported or observed.

Synopsis

Air carrier Captain reported a fume event during preflight described as wet socks. Logbook discrepancy and Odor form submitted to maintenance who MEL'ed the APU and re-dispatched flight which departed safely.

ACN: 1905651 (614 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 349
Experience.Flight Crew.Last 90 Days : 146
Experience.Flight Crew.Type : 349
ASRS Report Number.Accession Number : 1905651
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Observed a hydraulic leak on the green system during our periodic cruise check. The level was significantly lower than when we had first leveled off at cruise. We monitored it and reached out to MX (Maintenance) and Dispatch. It kept getting lower until it was right above the low level warning. Decided to divert to ZZZ. During divert descent we noticed it went in the low level range. Upon arrival MX said a screw was loose causing the leak.

Synopsis

A321 First Officer reported loss of fluid from Green Hydraulic System during cruise. At top of descent, the fluid level went into the low range. The flight crew continued to monitor and made the decision to divert and make a precautionary landing.

ACN: 1905036 (615 of 2908)

Time / Day

Date : 202206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 100

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 3
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class C : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 134
Experience.Flight Crew.Type : 301
ASRS Report Number.Accession Number : 1905036

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During takeoff phase upon gear retraction, green system hydraulic failed. The aircraft functioned as designed and isolated the failed system allowing the aircraft to continue the flight from ZZZ to ZZZ1. There was an Airbus jump seater who is a qualified instructor at the Company who worked with the crew using outstanding CRM principles and ran the checklists and helped back up the coordination with the flight attendants, passengers, ATC, and Dispatch. After discussing the need to burn fuel and landing distance penalties, the final decision to continue to ZZZ1 and we recovered the aircraft uneventfully. We had to stop straight ahead on the runway and get a tug back to the gate. Bottom line, training, good threat and error management, along with outstanding CRM and aircraft operating as designed when a system fails allowed a safe and uneventful day for all.

Synopsis

A320 First Officer reported green system hydraulic failure was mitigated due to proper aircraft isolation and CRM which resulted in safe landing

ACN: 1904861 (616 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1904861
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arrival to our aircraft at the gate in ZZZ the whole crew notice a very strong dirty sock smell in the entire Cabin. The odor was so strong that inflight was unable to come in the aircraft an perform their preflight duties. Me and the first officers went in the aircraft and quickly open the cockpit windows, we communicated with ground ops to open all doors of the aircraft in order to remove the odor. I noticed the bad odor was written up in prior to arriving at ZZZ. I informed maintenance and they decided to MEL the APU. Finally the odor was removed, we boarded our customers and proceeded with our push back and engine start up with the air start procedures. After we started the first engine a few minutes past and I received I call from [the flight attendants] stating the smell was back again. I informed them to let's start the second engine and see if it were to go away. We started the second engine and I received a call again stating the smell was worst than before. At that point with communication with dispatch and maintenance we decided to return to the gate. At that point we deplaned the customers. On maintenance request we ran the engine with pack 2 on individually and the odor returned, we then ran pack 1 individually and the odor returned even worst. At that point maintenance decided this was not able to continue.

Synopsis

Air carrier Captain reported "strong dirty socks" odor during preflight. After extensive maintenance troubleshooting the flight was canceled.

ACN: 1903970 (617 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.TRACON
State Reference : FO
Altitude.MSL.Single Value : 14000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
ATC / Advisory.TRACON : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : VFR
Mission : Passenger
Flight Phase : Descent

Component : 1

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Engine Air Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1903970
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Descending through 14,000 ft., on arrival to ZZZZ, both pilots began to notice an unusual odor in the flight deck. After brief discussion, it was agreed that the odor smelled like magic marker, or sharpie marker. Descending through 11,000 ft., with the engines at idle in open descent, the odor became much stronger. No smoke was noticed in the flight deck. The flight was cleared directly to the IAF for the ZZZZ approach. The pilots briefly referenced Quick Reference Handbook S-7 (Fumes), and determined a diversion was necessary. ZZZZ was the closest, and safest, airport under the current situation, and the flight continued to its scheduled destination of ZZZZ. Time did not allow for any additional troubleshooting as the aircraft was already being configured for landing. The flight landed in ZZZZ approximately 11 minutes after the odor was first discussed. After landing, the flight deck windows were opened for fresh air ventilation, and oxygen was used intermittently by the Captain. After parking at the gate, a logbook entry was completed, an Aircraft type Cabin Odor Report was completed, and a detailed discussion commenced with ZZZZ Tech-Ops. It was discussed that the APU had been deferred as per MEL from a previous flight. A logbook history review also revealed this aircraft had a recent history of odor/fume events. The Inflight Crew members stated they didn't smell any odor in the cabin until the flight deck door was opened at the gate. After reviewing the "Cockpit / Cabin Odor Recognition and Isolation" guide in the Supplemental Procedures section of FCOM Vol. 1, the event was identified as a Level 3. It was suggested that Engine #1 Bleed, or Pack #1, was the most likely source of the odor/fumes. Both pilots contacted Med-link as directed by the Chief Pilot on duty.

Synopsis

A320 Captain reported a fumes event during descent which was localized to the flight deck area.

ACN: 1902070 (618 of 2908)

Time / Day

Date : 202205

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1902070
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Event occurred in ZZZ. During initial checks and prior to boarding after our new flight attendants got on board, the Captain noticed a thick mist entering the Cockpit and Cabin of the aircraft. At the time the APU bleed was supplying the packs and a shower of moderate to heaving rain was occurring at the Airport. I was in the Forward Lavatory at the time of the event and never witnessed the abnormality myself. The Captain said that it produced a synthetic odor similar to that of burning rubber or plastic. The Captain immediately turned off the APU bleed and ordered all ground personnel and flight attendants off of the plane. Quickly after turning off the APU bleed the mist and odor cleared out and by the time I exited the Lav I noticed nothing. Necessary maintenance/fume checks were subsequently performed and the problem never returned.

Synopsis

Air Carrier First Officer reported a thick mist accompanied by a "burning plastic" odor during Gate preflight. Captain turned off the APU bleeds and ordered all personnel off the the plane. Necessary maintenance checks were completed which eliminated the mist and odor.

ACN: 1902060 (619 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1902060
Human Factors : Troubleshooting
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

During boarding at ZZZ, we smelled a level 2, musty odor from the aircraft environmental system. The odor was detected in the cockpit, forward galley and mid cabin by both pilots, the F2, a deadheading Captain, and several customers. The aircraft configuration was APU on, APU bleed on, [and] packs on. Using the FCOM (Flight Crew Operations Manual) odor isolation, we shut down the APU, but the odor continued to linger. We deplaned the aircraft and contacted MX (Maintenance) Control. MX Control stated the aircraft had a history of APU odor events and they believed that was the source of the odor. They asked if we would do an engine run without using the APU, to see if we could isolate the odor. Under the direction of contract maintenance, we conducted an engine run with no evidence of odor from the packs. Maintenance control returned the aircraft to service. The F2 reported symptoms (head ache) and stated she smelled the odor on the previous descent. The F2 contacted MedLink, and they advised that she could operate the flight to ZZZ1 if her symptoms dissipated and MX Control returned the aircraft to service. After the engine run, the F1, F3, and F4 returned to the aircraft in preparation for boarding. When they walked on, they reported that they detected a strong odor on the jet bridge and forward galley. After much deliberation with the FAâ€™s (Flight Attendant), Dispatch and Maintenance Control, we determined that an external environmental source was the cause of the FAâ€™s new odor event. The likely source was the external high pressure air cart which was running in close proximity to the jet bridge. With the F2 medically cleared, the aircraft returned to service, and the source of the new odor resolved, we reboarded the aircraft and departed uneventfully to ZZZ1.

Synopsis

Air Carrier A320 flight crew reported a fumes event during boarding. After a maintenance engine run it was determine that the fumes were caused by an external environmental source and not the aircraft. Aircraft departed safely.

ACN: 1902058 (620 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 13200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class E : ZZZ

Component : 1

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Distribution Relay
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1902058
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Descending through approximately 13,500 ft. the autopilot disconnected, auto thrust disconnected, yaw damp 1, rudder travel limiter, and Flight Directors as well. Looking at the ECAM (Electronic Centralized Aircraft Monitor) there was also a GEN1 failure. We followed the ECAM procedures and the GEN would not reset. The APU was started. During the process of disconnecting the auto thrust we went approximately 300 ft. low but recovered right away. The approach was continued and the flight landed uneventfully. Out of an abundance of caution, priority handling was requested because of the multiple failures triggered by the GEN1 failure.

Synopsis

A320 pilot reported loss of numerous systems after a #1 Generator Failure. The flight crew started the APU after numerous attempts to reset the #1 Generator failed and continued to the destination airport.

ACN: 1902044 (621 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1902044
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Apu was already on MEL due to fumes event the prior evening. Coincidentally I was also on that same flight commuting to work on flight from ZZZ1. We started engine #2 at the Gate using air start procedures. We pushed back from Gate and started taxi out to Runway XX. On the taxi we used the crossbleed start procedures to start engine #1. During the clean up procedures after the crossbleed start the First Officer turned the #1 engine bleed back on. Almost immediately we could start to smell the dirty sock smell that gradually increased in intensity. By the time we were halfway to Runway XX inflight had called up front to let us know the fumes were bad in the Cabin. We immediately told Ground we needed to return to the Gate. We then ran the odor isolation procedure in the Flight Crew Operations Manual and decided to shut down engine #1 because we thought that engine was the source of the fumes. The First Officer opened his sliding window, and the Cockpit odor diminished quickly. Three calls were made to inflight to gauge the intensity of the fumes in the back. The fumes slowly subsided in the Cabin. We had to wait 20 minutes for a Gate but returned where the aircraft was removed from service. We swapped aircraft and continued flight.

Synopsis

Air Carrier Captain reported a "dirty socks" odor of increasing intensity during departure taxi. After receiving a flight attendant call regarding the fumes the Captain requested a Ground return where Maintenance assigned an aircraft swap.

ACN: 1902041 (622 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Pitot/Static Ice System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1902041
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1902055
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the morning of DATE, NAME and I were operating Aircraft X from ZZZ to ZZZ1. NAME was the pilot flying and I was the pilot monitoring. It was an uneventful departure and climb to cruise. At cruising altitude in the low 30's we were in VMC. We received the ECAM "ANTI ICE F/O L STAT F/O R STAT". NAME started to perform the ECAM (Electronic Centralized Aircraft Monitor) actions. Simultaneously I noticed a burning electrical smell. NAME then noticed the smell as well. We both don our oxygen mask then. NAME handed me the controls and we made the decision to divert. ZZZ2 was approximately 177 miles from our current position. While we were still able to smell an electrical burning there was no visible smoke coming into the cockpit and the intensity of the smell seemed to be constant. NAME called the cabin and informed the cabin crew that we were diverting, and we were code yellow. He also made a customer PA. I sent an ACARS off to Dispatch. I believe I told them Avionics Smoke and we were diverting to ZZZ2. I then requested priority handling with ATC. I informed ATC of our situation and we wanted to divert to ZZZ2. We were cleared direct ZZZ2. ATC took the standard information of SOB, and fuel. I informed ATC that it was going to be an overweight landing. I updated the FMS with a new destination and loaded the ILS XX at ZZZ2 since that was the approach in use per the current ATIS. During this time NAME was running the "Smoke/Fumes/Avionics Smoke" QRH (Quick Reference Handbook). Name called the cabin asked them if they smelled anything or noticed anything unusual. They reported they did not. During our descent, the burning smell did dissipate. However, we stayed on our oxygen mask. NAME then ran the overweight landing QRH and landing data performance. Dispatch sent us a new flight summary and how much overweight we were predicting to be. Once NAME finished with the overweight QRH he took control. NAME briefed the approach, and we ran the descent check list. NAME also performed his 10000 foot flow. I also took a glance at the checklist and the overweight check list to back NAME up on it. About this time, we were about to check on with ZZZ2 approach. We updated ZZZ2 approach of our current situation. We were planning a negative evacuation and we wanted to stop on the runway for inspection. We also asked for direct ZZZZZ for the ILS XX. We got cleared as requested and cleared for the approach. When we were handed off to tower, I restated what our intentions were. We had a normal landing and came to a stop just prior to "B". NAME made the "remain seated call" and Tower put us on another frequency. I thought this was a direct frequency to CFR (Crash Fire Rescue). But it was another tower frequency. It did lead to some confusion because I put this new frequency into radio 2 while listening to radio 1. I reached out to CFR to tell them where to inspect the aircraft. When the controller started talking to CFR I did not realize right away he was speaking on both frequencies. It caused me to unnecessary switch back to radio 1 while CFR was on radio 2. Once I figured that out, we had CFR inspect the front of the aircraft since we knew the issue was in the avionics bay. CFR reported an all clear. We then contacted ground and taxied clear of the runway. Once clear we preformed the after landing check list. I contacted ops for our gate assignment. We were cleared into C-1. We pulled into the gate. We deplaned the aircraft as normal. CFR came up to the cockpit to asked what happen. I explained to them what happen. Maintenance met the aircraft. He found one of the computers below my seat burned itself up and was still hot to the touch. He did not know why the circuit breaker did not pop. From this point forward the customers were inside the terminal. The difficulty of using the O2 mask and communication. Without the head set on it was difficult to hear. With the head set on, it was difficult to hear over your own breathing getting picked up in the microphone. We did solve this by going "cold" mic in between sentences. However, you need to be aware of that. We did slow down to 250 knots at 10,000 feet. Should we have kept the speed up? It was not necessary at that point but, I believe we slowed down more out of habit. While we do go over the EVAS in training. It is not second nature if we had to use it. Especially since we are shown how to use it standing up.

Narrative: 2

Enroute from ZZZ-ZZZ1 at 320 received ECAM ANTI ICE F/O STAT. We both immediately smelled a strong burning odor. We put our masks on and established communication. I passed flying duties to my FO, performed the ECAM, and initiated the QRH Smoke/Fumes/Avionics Smoke checklist. I called my Cabin Crew to determine if they could smell the odor and they could not. We coordinated with Dispatch and agreed on ZZZ2 as our divert, declared a code yellow, informing them that we would be landing in 15-20 minutes. I also made several PAs to our customers informing them of the situation and telling them exactly what we were doing and what would happen when on the ground. No visible smoke was present but the burning smell persisted so we kept the speed up in our descent and exceeded 250 knots below 10,000 feet for a brief period. We performed the overweight landing checklist and made an uneventful landing in ZZZ2. We stopped on the runway and had CRF visually inspect the aircraft prior to taxiing clear. Once on the ground the contract maintenance tech determined that a Pitot Heat Controller had malfunctioned/shorted out.

Synopsis

A320 flight crew reported smoke and fumes in the cockpit with ECAM "ANTI ICE F/O L STAT F/O R STAT" Illuminated. The flight crew requested priority handling and diverted to make a precautionary landing.

ACN: 1901942 (623 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Hydraulic Fluid
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1901942
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1902094
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After landing Runway XXL and clearing onto Taxiway Bravo, we received HYD G RSVR LO LVL message with associated cautions and failures. Captain set parking brake and we ran appropriate checklists and ECAM procedures. We notified Ground Control of the problem and contacted Operations/Maintenance for a tow into the gate. Aircraft Messages HYD G RSVR LO LVL

Narrative: 2

I slowed to a normal taxi speed and exited Runway XXL. As I exited the runway we got a master caution light and an ECAM message, "HYD G RSVR LO LVL". I was clear of the runway so I stopped the aircraft. The nose wheel steering was failing. We notified ZZZZ Ground Control and they provided assistance by starting the process of getting us a tug. We ran the appropriate checklists. And eventually were towed in to Gate XX without incident or issue. The Green hydraulic system developed a leak, I later learned it was at the main landing gear bypass valve. This caused the entire reservoir on the green system to be depleted leading to a loss of hydraulic pressure. Because the older airbus 320 aircraft lose nose wheel steering with the loss of Green System pressure, there was no way to taxi to our gate. Therefore we needed to be towed in. Really [have no suggestions] whatsoever. It was a fairly uncommon maintenance failure. Fortunately Authorities as well as Company employees in ZZZZ were quick to respond and I don't think any operations were even slowed down at the ZZZZ airport. It was a nice job by all involved.

Synopsis

A320 flight crew reported a hydraulic failure and loss of nose wheel steering after landing. The aircraft was towed to the gate where it was determined a brake valve had failed causing the loss of hydraulic fluid.

ACN: 1901700 (624 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : ELB / ELT
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Component : 2

Aircraft Component : Other Documentation
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1901700
Human Factors : Workload
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Upon doing the pre-flight, the #4 Flight Attendant (FA) informed me as #1 the ELT (Emergency Locator Transmitter) designated behind row 3 aircraft L was missing. I confirmed. She said we were on the same type 320 the day before (Aircraft Y, Date) and there was an ELT in that location. We pulled up equipment location via the tablet. There was suppose to be one via the manual an ELT. There were no brackets or anything. The plane had just finished overhaul. The seats were new and the overheads not a scratch or scuff on them. We told the pilots and they called Maintenance. At first told go, because we were not EOW (Extended Over Water) but then, Pilots said they got other conflicting information after we took off. No words. So just keep quiet.

Synopsis

Flight Attendant reported an ELT was missing from the aircraft during preflight. The flight crew reported this to Maintenance and were told to conduct the flight because the aircraft was not an extended over water aircraft.

ACN: 1901262 (625 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1901262
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Work Refused
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft had been towed from the hangar and brought to gate. APU Bleed was on MEL, so APU was running but the bleed was not selected on. Preconditioned Air was also not connected. The Captain requested that the ground crew attach the ground start cart (pneumatic) and when it was connected, he selected the packs on to cool the aircraft. Within just 30 seconds or so a very foul odor of musty socks permeated the entire aircraft. My eyes immediately began to burn. We de-selected the packs and directed the start cart to be powered down. I had just had a fume event in an A319 so my eyes are still sensitive apparently. I opened the cockpit window to ventilate and we called Maintenance out, and swapped aircraft. APU Bleed not working properly was the same problem that caused my fume event last month. APU MEL's need better oversight.

Synopsis

Air Carrier First Officer reported a fumes event during preflight when a pneumatic ground cart was connected. Upon activating the packs, a foul odor of musty socks permeated the entire aircraft which necessitated the packs and cart to be deselected. Shortly thereafter Maintenance assigned an aircraft swap.

ACN: 1901094 (626 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Engine Air Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1901094
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In flight at FL370. Lead FA (Flight Attendant) called about 1/2 way into the flight and stated FA's in the back of [the aircraft] were experiencing headaches. We tried to gather information on the situation. They stated stuffy air in the back. Aircraft started out very warm from ZZZ but at that time it showed below 70 in the back. Forward FA was not experiencing any issues. MEL for HPV complicated the issue. It had opposing guidance to the fume event procedure and we weren't sure if that was what we were experiencing. I focused on flying the Aircraft and the radios while the CA (Captain) worked on the event. We doned our O2 masks when they didn't see an improvement in the cabin. CA isolated the number 1 bleed system and after some time they said they saw improvement in the back cabin. We discontinued the use of the masks after we had the report of improved air quality in the back. The lead FA never did experience any adverse effects. We continued the flight and landed in ZZZ1 with no other issues. We talked with the Cabin Crew and Maintenance about the event in detail.

Synopsis

Air Carrier First Officer reported the aft flight attendants were experiencing symptoms due to "stuffy air" in the aft cabin. Captain initiated the QRH procedure to isolated the number 1 bleed system which improved the the air quality. The flight continued to a safe landing where crew conferred with Maintenance.

ACN: 1901055 (627 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Taxi

Component : 1

Aircraft Component : Engine Air Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1901055
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

While at gate while on APU we detected an odor but when APU bleed was closed smell when away completely. At the time (on ground at gate) environmental conditions were heavy rains and strong winds directly on the tail. Since odor was not present for boarding and push back we turned on APU bleeds when tail out of wind, Started engines and taxied out with no odor of dirty socks at the time. During taxi out with APU bleed off about 10 minutes into taxi smell returned- minor overall but persistent and throughout cockpit and cabin. We referenced the odor recognition and isolation checklist and ran through the sequencing of packs and APU bleed recommendations as per checklist. CA (Captain) and FO (First Officer) donned oxygen during procedures. Isolated odor to pack 1 when on engines - contacted Dispatch and Maintenance gate return recommended. When on APU odor is strong when pack one 1 on and minor in nature when pack 2 is on.

Synopsis

Air Carrier flight crew reported an intermittent fumes event while at gate preflight and during departure taxi. After completion of the appropriate QRH procedure and conferring with MX the flight returned to departure gate.

ACN: 1899787 (628 of 2908)

Time / Day

Date : 202205

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Takeoff / Launch
Route In Use : Vectors

Component : 1

Aircraft Component : Attitude Indicator(Gyro/Horizon/ADI)
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pitot-Static System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1899787
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X during takeoff at the 100 knots airspeed cross check, the ISIS (Integrated Standby Instrument System) indicated just under 20 kts. slow. I elected not to abort the takeoff due to the guidance in the bulletin. On climb out the airspeed difference progressively got worse, eventually reading 60+ kts. slow.

Synopsis

A320 Captain reported an airspeed split on the ISIS Display on takeoff roll. The flight crew elected to continue the flight to destination airport.

ACN: 1899785 (629 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Climb
Flight Phase : Takeoff / Launch
Flight Phase : Initial Climb
Route In Use : Direct
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1899785
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Ground Event / Encounter : FOD
Anomaly.Ground Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

I'm writing this to help identify and explain a possible misunderstanding with ATC. On take-off roll just prior to v1 the aircraft pulled hard left then immediately corrected to center line. We continued and took off uneventful. Prior to being handed off departure I quarried if there were debris on the runway. I was thinking to use all resources to rule out a possible tire failure. The Tower seemed as though we were trying to report an issue without saying it. We were not and we made that clear. Over to departure and departure asked us if we have an issue because Tower called them. We had time to talk about the situation and the Captain who was flying said he thought it was a possible large wind gust and I agreed after ruling out tire and steering issues. Satisfied we reported no issues and everything was normal. They checked again and we said all is normal. We continued the rest of the flight without issues.

Synopsis

Air Carrier First Officer reported a miscommunication with ATC involving temporary loss of directional control during take-off roll and possible FOD on the runway.

ACN: 1899783 (630 of 2908)

Time / Day

Date : 202205

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pneumatic Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1899783
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After removing external air conditioning and selecting APU bleed ON, the FO (First Officer) and I smelled a mild musty odor from the cockpit air vents. I selected Pack 1 OFF, no help. I selected the APU bleed OFF and the odor stopped. We were mostly boarded at this point. I asked the FAs (Flight Attendants) if they smelled anything, and one of the aft FAs said she smelled the musty/dirty socks odor too. I instructed the passengers to deplane and had the FO call Operations to let them know what was going on and to send Maintenance. I advised Dispatch via ACARS. I wrote it up in the logbook, filled out the air quality event reporting sheet, and told the Mechanics what happened. The Mechanics said they would do a pack burnoff. I made sure everyone was off the airplane and made sure the FO and FAs were OK. No one reported any symptoms or required medical attention. Two FAs told me they noticed unusual smells upon entering the aircraft. One aft FA said she smelled a moderate vomit/musty smell. The chaser said she smelled a mild vomit/musty smell. I called Maintenance, our Dispatcher, and the Dispatch. Maintenance returned the aircraft to service and the crew got back on board. The logbook said troubleshot per Manual, Operations check good. The APU bleed and Packs were on. I walked up and down the cabin and spoke to the entire crew. No one smelled anything anymore. I called Operations to request Maintenance come back to let me know what they did, to help me decide whether to accept aircraft. One of the previous Mechanics came back with Maintenance Supervisor, who had not been present before. Before I had a chance to say anything, Maintenance Supervisor stated Maintenance had followed the [Service Manual], and he would not provide any information whatsoever about what they did. One of the Mechanics who had performed the burn off started to speak, but [Supervisor] stopped him. [Supervisor] said I could call Maintenance Control, who could call the Mechanics, and Maintenance Control could relay the information. I explained that I wanted to know what Maintenance did for safety reasons, to help me decide whether to accept the aircraft and to know when there could be a risk of the fumes returning. Maintenance Supervisor [name] again stated he would not provide any information. I called Maintenance Control, who called the Mechanics. Maintenance told me they did a pack burnoff using the APU bleed, the engines were not started, and no more odors were noted. I spoke with the entire crew, and we decided to accept the aircraft. I briefed the FAs to let me know immediately if the musty smell returned, particularly after engine start (this is part of why I wanted a debriefing from Maintenance, now I knew the engine bleeds had not been tested). We boarded, closed up, then waited over 30 minutes for a push crew. No odors re-occurred. I called the Chief Pilot on duty after arrival.

Synopsis

Air Carrier Captain reported a fumes event during boarding. Passengers were deplaned while maintenance preformed a Technical Service Manual procedure which eliminated the fumes. The flight departed safely thereafter.

ACN: 1899705 (631 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Climb
Flight Phase : Initial Climb
Route In Use.SID : ZZZZZ
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Wing Flight Control Surface
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1899705
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Other / Unknown
Human Factors : Distraction
Human Factors : Confusion

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1899713
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Other / Unknown
Human Factors : Distraction
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Immediately after rotation we noticed a vibration that both the CA (Captain) and I thought was a typical nosewheel vibration, however in a matter of seconds it greatly worsened to a point that we realized it was something worse. The CA called for autopilot engagement. Immediately upon autopilot engagement the aircraft started tracking right of course. The Captain disengaged the autopilot and auto thrust so as to maintain directional control and airspeed as the aircraft seemed to be having difficulty with direction and speed. Tower handed us off to Departure and upon checking in with departure the vibration had dramatically worsened into what I would call more of a severe shake than a vibration so we [advised ATC] and requested a return to the field on the initial check in. Departure gave us vectors for a right hand pattern back to Runway XX. We were assigned an altitude of 5,000 ft. We made it to 5,000 ft. but we had difficulty maintaining altitude. The Captain kept control of the aircraft and had me cycle though the doors, wheels, and flight control pages to try and identify the problem. We found that the flight control page showed the left 3 outboard spoilers were deployed. The FA (Flight Attendant) called twice, first to tell us about the vibration in the back and the second time to tell us that 2 rows of O2 masks had dropped. The Captain made the decision to land flaps 3. We landed on Runway XX and the vibration continued until touchdown. Upon arrival at the gate we found that spoilers on the left had closed however there was a spoiler deployed on the right wing. There was an apparent issue with SEC 3 and it's control of the spoilers as SEC 3 was on an MEL due to the same issue on a previous flight. If there is an issue with a flight control maybe the aircraft should be out of service until the repair is made instead of the MEL.

Narrative: 2

High terrain environment. During the takeoff roll, the aircraft felt difficult (heavy) to rotate. I did not rotate forcefully. I picked the nose up and let the airplane fly off the ground. Vibration started with gear retraction and grew worse. Like nose wheel shimmy multiplied. Excessive and severe vibration was apparent and increasing in intensity. I called for the autopilot to limit distraction. The autopilot instantly rolled right. Autothrust was accelerating beyond SID speeds and the trend vector beyond 250 kts. I switched the autopilot off then switched autothrust off. I had the FO (First Officer) turn flight directors off and engage Track/FPA to help hold attitude. At this point we had been switched to Departure frequency and we checked on [advising ATC] and requesting return to the field. ATC provided vectors. Severe vibration was felt throughout the aircraft. FA's (Flight Attendants) called me shortly after takeoff about the vibration (we informed them we were aware) and then they called again on the downwind to tell me O2 masks had fallen in 2 rows (we informed them we returning for landing and would deal with that later). It was difficult to hold altitude and airspeed. Any turn resulted in a descent requiring power to recover. Any distraction pulling me away from the PFD caused a change in heading and altitude. VLS and V alpha prot were very high. I had the FO cycle through the Doors, Wheels, and F/CTL ECAM pages to find out what was causing the vibration. The F/CTL page indicated the left outboard 3 spoilers were deployed (un-commanded). The high VLS and V alpha prot plus the handling characteristics were consistent with having spoilers extended. (I checked the position of the spoiler handle - it was in the retracted position). We circled back to land on Runway XX. I stated to the FO that we would have to land flaps 3 due to the spoilers being extended and had the FO look for an airframe vibration QRH checklist. (After action review indicated that this checklist would not have helped.) The approach was difficult due to the high V alpha prot. I had to choose between slowing down or maintaining the descent profile to get flaps out. I had very little margin between Flap extension speed and VLS, and coincidentally VLS was maybe 2 kts. above V alpha prot. I considered overspeeding the flaps if necessary to slow down. It wasn't necessary, but just barely. We landed Flaps 3 intentionally; however, we did not select flaps 3. Time issue. The FO was task saturated. I used selected speed and manual thrust to landing. I made no PA's until after landing. I had my hands full and the FO was too new on the aircraft to turn over control. The FO did a good job and I was issuing very fast commands. He handled it well. SEC 3 was deferred per an MEL. We were unable to see the position of the number 1 and 2 spoilers on both wings. I read and complied with the "O" procedures of the MEL. When we arrived at the gate after returning to the field we found the Right #1 spoiler full extended. I have been told that a passenger saw the spoiler extended prior to takeoff (this is here say and I have no proof if it's true or false). When and how that spoiler was extended is unknown to me. The deferral results in amber X's being displayed in that spoiler position on the ECAM page. In addition to that, we had 3 outboard spoilers deployed (un-commanded) on the left wing. This was the second event involving vibration and poor handling characteristics including the autopilot roll that I know of-- the first was clearly not as severe. Obviously, this was a serious problem with the spoilers and the aircraft should not have been in service. Aircraft should be removed from service if there is sufficient vibration to result in a loss of control. This aircraft had diverted once already because of this. We did not have enough information available to know what to expect. If we had been aware of what had occurred before, I would most likely have refused the airplane.

Synopsis

Air Carrier Captain and First Officer reported that 3 spoilers on the left outer wing deployed during rotation and climb. The pilot crew had difficulty in maintaining aircraft control and returned to the departure airport. The pilots reported a similar event had been recently reported and MEL'ed on this aircraft.

ACN: 1899174 (632 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb
Route In Use : Vectors

Component

Aircraft Component : FAC (Flight Augmentation Computer)
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1899174
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During preflight, contract Maintenance placarded SEC 3. During startup we had an ELAC 1 fault which was cleared with ECAM. I was PF (Pilot Flying) and after takeoff, the aircraft had a heavy vibration and roll action. We continued the climb and cleaned up the aircraft. We had a FA (Flight Attendant) visually confirm that the left spoilers were deployed with the spoiler handle in the down and disarmed position. We continued to climb and the vibration mostly dissipated but was still evident. After confirmation with Maintenance Control and Dispatch on the Crew Phone, we elected to divert and land in ZZZ instead of continuing to destination ZZZ1. We burned down as much fuel as we could and set up for an approach. When we configured to flaps 2 the serious vibrations started again. We preformed a controllability check at pattern altitude and determined that the vibration was too great at flaps full so we used flaps 3 for landing. As we got closer to the Runway, the vibrations dissipated a bit and we made a normal landing overweight by 10,000 pounds into ZZZ. We had ARFF (Airport Rescue and Fire) inspect the aircraft for damage, which there was none apparent and taxied to a gate and gave the aircraft to maintenance. Our inspection of the aircraft at the gate found the right ground spoiler still extended. We are unsure if that was left over from the flight or just the landing roll. The only indications we received during the flight were the green spoiler arrows on ECAM. No other warnings or messages came up. We were and still are perplexed at how any spoilers could have been extended with the handle stowed. There could have been a secondary malfunction with the SEC 3 issue. It caught us totally by surprise. We followed the placard requirements of the SEC which were very basic. Would like to hear from Maintenance on what the cause was.

Synopsis

An A320 Pilot reported left hand spoilers deployed with the spoiler handle in the down position after take off. The flight Crew requested priority handling and diverted to make a precautionary landing.

ACN: 1899105 (633 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6483.63
Experience.Flight Crew.Last 90 Days : 171.43
Experience.Flight Crew.Type : 1953.43
ASRS Report Number.Accession Number : 1899105
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departing XXL using a properly loaded flex takeoff, engine 1 failed to spool with engine 2. Engine 1 EPR hung at 1.1 EPR as engine 2 spooled properly. The adverse yaw forced a low speed rejected takeoff twice. Maintenance ordered a gate return.

Synopsis

A320 Captain reported the engines did not synchronize on departure roll and they rejected the takeoff.

ACN: 1898124 (634 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 96
Experience.Flight Crew.Type : 4273
ASRS Report Number.Accession Number : 1898124
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise we received Green Hydraulic Reservoir Overheat ECAM. We ran and completed the ECAM, [requested priority handling], Diverted to ZZZ1, relayed to Dispatch our situation and plans, and briefed the flight attendants. We followed up with the notes and cautions in the FMS for Green Reservoir Overheat, had Dispatch compute landing data with penalties per our selected runway. The approach procedures in the notes and caution section directs the green hydraulic pump on if hydraulic reservoir overheat indication is gone. In our case, it was gone. On final we turned on the green pump and had a normal landing. We exited the runway and stopped. We then requested and were towed to the gate.

Synopsis

A320 First Officer reported a hydraulic system overheat during cruise. The flight crew diverted to a nearby airport and made a precautionary landing.

ACN: 1897783 (635 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 912
Experience.Flight Crew.Last 90 Days : 195
Experience.Flight Crew.Type : 912
ASRS Report Number.Accession Number : 1897783

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 16323
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 13769
ASRS Report Number.Accession Number : 1897805

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Incursion : Taxiway
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During Taxi-in at ZZZ after required cool down time was reached, Captain instructed First Officer to shutdown No. 2 engine. At the time of engine 2 shutdown; Aircraft was on a straight section of taxiway, Auxiliary Power Unit off, aircraft taxi speed approximately 8 kts., with brakes being applied by Captain in preparation of making a left turn onto another taxiway. When no.2 engine was shut down and its associated generator tripped off, aircraft experienced a total electrical power loss except for those systems supplied by battery power only. When power was lost, aircraft experienced a very pronounced, braking deceleration as aircraft braking transferred to alternate brakes and a loss of nose wheel steering. Captain used differential braking to both counter act aircraft drift to the right (due to single engine taxi) and stop the aircraft. Aircraft safely came to a stop about 6 ft. to the right of taxi centerline. Parking brake was set and the process of recovering electrical power, coordinating with flight attendants and passengers commenced. To restore electrical power crew attempted to first reset GEN 1. GEN1 dropped off immediately after resetting fault switch. Auxiliary Power Unit was then started and after Auxiliary Power Unit GEN was powering all busses another attempt to reset GEN 1 failed. After discussion with First Officer and satisfied that Auxiliary Power Unit GEN was powering all busses normally, it was decide to continue taxi to the gate.

Narrative: 2

During Taxi-in at ZZZ after required cool down time was reached, Captain instructed FO to shutdown No. 2 engine. At the time of engine 2 shutdown; Aircraft was on a straight section of taxiway, APU off, aircraft taxi speed approximately 8 kts, with brakes being applied by Captain in preparation of making a left turn onto another taxiway. When no.2 engine was shutdown and it's associated Generator tripped off, aircraft experienced a total electrical power loss except for those systems supplied by battery power only. When power was lost, aircraft experienced a very pronounced, braking deceleration as aircraft braking transferred to alternate brakes and a loss of nose wheel steering. Captain used differential braking to both counter act aircraft drift to the right (due to single engine taxi) and stop the aircraft. Aircraft safely came to a stop about 6 feet to the right of taxi centerline. Parking brake was set and the process of recovering electrical power, coordinating with flight attendants and passengers commenced. To restore electrical power crew attempted to first reset GEN 1. GEN1 dropped off immediately after reseting fault switch. APU was then started and after APU GEN was powering all busses another attempt to reset GEN 1 failed. After discussion with FO and satisfied that APU GEN was powering all busses normally, it was decide to continue taxi to the gate.

Synopsis

A320 pilot crew reported an electrical failure while using the single engine taxi procedure. The aircraft switched to alternate brakes and the Captain was able to safely stop the aircraft.

ACN: 1897776 (636 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1378
Experience.Flight Crew.Last 90 Days : 144
Experience.Flight Crew.Type : 1378
ASRS Report Number.Accession Number : 1897776
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1247
Experience.Flight Crew.Last 90 Days : 82
Experience.Flight Crew.Type : 1247
ASRS Report Number.Accession Number : 1897791
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

When reviewing the flight plan paperwork, I saw there was an Inbound write up for the Green Hydraulic System Reservoir Overheat. The plane came in the night before, and this write up was cleared. It was the first flight of the day for this airplane. On taxi out, all systems operated normally. We had a delay on the ground for flow into ZZZ. So we had about a 25 minute taxi. Everything was operating normally until we leveled off at FL360, about 30 minutes after takeoff. Shortly after that, we got a ding. Looked over and saw Hydraulic: G RSVR OVHT ECAM (Electronic Centralized Aircraft Monitor). Captain divided task by having me fly and do the radios, and he will deal with the ECAM. The ECAM directed us to turn off the PTU (Power Transfer Unit) and Green Hydraulic Pump. We verified each switch before selecting it off. We went over they system pages, and talked about what systems were lost. The CA then went into the QRH (Quick Reference Handbook). The QRH had a statement to restore the lost system once it was no longer overheated. Also the QRH noted that we could have a 10% higher burn due to spoiler float. We took the burn to destination then added 10% subtracted that from our FOB. We felt we would still land in ZZZ with plenty of gas. Then the CA notified dispatch, and got a frequency to talk to Maintenance Control. We all agreed that continuing to ZZZ was the best course of action. After about 6 minutes, I noticed the fault had cleared. Since the Green system has landing great extension, nose wheel steering, normal breaking, thrust reverser 1, and 2 spoilers on it, we felt trying to restore the system closer to the airport was the safest option. We were concerned if we restored the system too early, it would overheat again on approach. We advised center of our situation, and requested priority handling. With a little less than an hour left, we started briefing. We started talking about all the threats we would have if the system was not restored. We talked about gravity extension, longer landing distance, not being able to steer off the runway. We talked about if we had to go around, we would not be able to retract our gear. That meant higher fuel burn coming back to land. We decided on downwind trying to restore the green system would be the best place, because if it did not restore we would have plenty of time to notify ATC we will be needing a tug and perform the gravity gear extension procedure. We decided to Configure early if the system restored In case it would over heat again. Talked about if the system over headed again on final approach after configuring, what we would do. We had two jump seaters, and we asked them if they had any other suggestions or they agreed with our plan of action. They agreed with our plan, and provided some concerns to keep in mind. We told ATC we will need XXL for landing, because it was the longest runway. Told ATC we will need ARFF (Airport Rescue and Firefighting) standing by, and notified them of potentially needing to be towed off the runway. After we were done briefing, the CA took over flying the airplane. When abeam the field, we were able to restore the Green system. Made a normal approach and landing. We are able to taxi off the runway and to the gate on our own power. Maintenance met us at the gate, and we told them of the issues. They informed us that this has been a reoccurring issue over the past several days. Great teamwork, workload management, and communication lead this event to be a successful outcome.

Narrative: 2

Shortly after reaching cruise altitude, we received an ECAM for a Green Hydraulic System Reservoir Overheat. We referenced the overhead panel and confirmed the associated fault light illuminated. After passing Flying Pilot duties to the first officer, I initiated the steps associated ECAM. Simply stated, the Green Hydraulic System was isolated (PTU Off) and secured (Engine Pump Off). The Fault light remained illuminated, and the Overheat Indication was noted. The two remaining hydraulic systems (Blue and Yellow) sustained all hydraulic needs at cruise flight. Our next step was to complete the QRH and calculate the associated Non-Normal Performance Landing Distance. Both tasks were accomplished. The First Officer and I had a quick discussion regarding our options on this short flight. After building a game plan, I queried Dispatch to Call Me and asked them to bring Maintenance Control on the line. We discussed the Hydraulic non normal and the associated re-occurring maintenance history that had been reviewed with the Flight Plan and Maintenance Release in ZZZ1. We advised that the Fault light had extinguished (approximately 6 minutes) and the hydraulic reservoir temperature had cooled to the point the system was no longer exceeding temperature limits. Noting this condition, the First Officer, Maintenance Control and I all agreed that the QRH procedure for attempting to restore Green Hydraulic power prior to arrival would be an option. Dispatch was then engaged regarding our flight and continuing to ZZZ. While this was my preference, I included the dispatcher in the conversation and confirmed they had no objection to this course of action. At this point the call concluded and we agree to stay in communication via ACARS. An ELB entry was made to document the non-normal and support Maintenance Control's phone call to ZZZ Maintenance advising them of our inbound aircraft and situation. At this point the approach briefing was conducted. The briefing outlined our plan to attempt green system restoration in the terminal area and continue with green system pressure for a normal landing. Alternatively, we brief that if the system could not be restored, we would proceed with a manual gear extension, loss of spoilers 1/5 on landing, loss of normal breaking, nose wheel steering, #1 Thrust Reverser (etc...). The approach briefing was conducted, and we engaged our two jump seating Company pilots who were in the flight deck commuting home to ZZZ. Both offered their thoughts and suggestions to enhance our game plan. Additionally, the jump seating pilots switched seats to allow the jump seating pilot with previous Airbus experience to occupy the primary (center) jump seat. I handed the jump seater my QRH to back up the First Officer in the event additional checklists were required in the terminal area (I.E.: Manual Gear Extension). Lastly, we as a crew did a preliminary review of the manual gear extension procedure should hydraulic system restoration be unsuccessful. Our flight requested priority handling with ZZZ Center and advised them of our Hydraulic System condition, our desire to attempt restoration prior to landing, Fuel, SOB and Dangerous Goods. Additionally, we informed them we required Runway XXL for arrival and that we may or may not have nose wheel steering on arrival depending on the outcome of our system restoration attempt. An ACARS was sent to Dispatch advising of our request for priority handling. The Flight Attendants were apprised next with a test Briefing and advised of the situation and game plan. Lastly, I made an announcement to the passengers advising them of the situation and thanked them for flying with us. At this point I assumed the flying pilot duties again and the First Officer resumed pilot monitoring duties. ATC shortened our routing, removed all airspeed constraints and we initiated our descent for the ZZZ Airport. While on the downwind course at approximately 8,000 feet, we executed the QRH procedure to restore the Green System as the overheat condition was no longer present. Fortunately, the system restoration was successful and the green system was restored to 3,000 psi. The aircraft was configured normally and continued a normal landing without further hydraulic issues or overheat indications. The aircraft was met by ARFF on arrival and they proceeded to followed our aircraft to the gate. The aircraft taxied to the gate under it's own steering and power. After block in, the flight was debriefed with the First Officer and Jump seaters. After deplaning, I had a debrief conversation with our purser and thanked them for their assistance. Lastly, Company maintenance boarded the aircraft for a debrief of the hydraulic reservoir overheat and system restoration.

Synopsis

A320 flight crew reported a Green Hydraulic System Reservoir Overheat light illuminated in flight. The flight crew performed the QRH procedures and secured the Green system Pump. The flight crew requested priority handling and reactivated the Green System Pump closer to the destination airport to conduct a precautionary landing.

ACN: 1897772 (637 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 192
Experience.Flight Crew.Type : 8700
ASRS Report Number.Accession Number : 1897772
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During flight from ZZZ to ZZZ1 on Aircraft X we experienced a fume/odor event that required 4 of 5 crewmembers to be evaluated by paramedics upon arrival. We actually had 2 events, the first of which was on takeoff, and the second started during the descent to landing in ZZZ1. The first odor issue was brought to my attention after signaling that we were above ten thousand ft. The Purser called to tell me that they had an odor in the back that smelled sort of like burning plastic but they weren't really sure. I had the flight attendants seated at the time due to turbulence and weather on departure. Since we had cleared the weather and had only light turbulence at the time, I had the flight attendants get up and investigate to see if they could find the source being mindful to also carefully check to see if there were any hotspots in areas they thought the smell might be coming from. They reported back a few minutes later that there was no smoke or hotspots and that the odor seemed to be dissipating. We lowered the temperature in the cabin and set the pack flow to HI and by the time we reached cruise the smell was mostly gone and continued to go away. No crew or passengers reported feeling ill at that time, and while the odor was noticeable at first it did not seem over-powering or dangerous, so with conditions improving and no way to know what the smell was, we elected to continue towards the destination knowing that better weather and suitable alternates were ahead of us should they be needed. The odor was gone at cruise and did not return until we were in our descent for landing. After we started our descent, the odor returned when the power was brought back to idle. It was first noticed in the cockpit as the same smell from before and then a few minutes later the flight attendants said they could smell it near row 26. The more we descended, it seemed the odor increased. We were close to ZZZ1 at that point so we continued toward the airport and landed with no issues. As we taxied to the gate, I stated to the First Officer that the smell was starting to cause my throat and tongue to feel irritated and I felt like I might be getting light-headed. It was at that time that the First Officer told me that he was also starting the feel sick. By the time we finished that conversation, we were pulling into the gate. After parking, we shut down the engines and opened the cockpit windows and door after completing our checklists. I checked in with the Flight Attendants to see how they were doing. They informed me, that they also were feeling tingling in their throats as well and one said she was light-headed. I inquired if any passengers reported feeling ill and wanted to be seen by paramedics and I was told there were none. I notified Maintenance Control of the issue and told them I was writing up the plane for the odor/fume issue as we deplaned. I got all of the crew off of the airplane and I then contacted the Operations Manager, who was great and very helpful. He advised me to call Medlink which was my next call where I obtained the phone number for Employees which I provided to all the other crewmembers. I had them individually call and get evaluated on the phone by Medlink. I was advised that based on my condition that unless I did not continue to improve, I did not need to see a medical professional. All 4 of the other crewmembers were advised to be seen by paramedics which they did. I continued to monitor how my crew was doing and provided them all with my personal phone number and email, spoke again with the Operations Manager, filled out the Airbus Smoke/Fume/Odor form for maintenance, and interacted with ZZZ1 Operations. I believe that there are some insidious things that occurred during this event that pilots and flight attendants should be aware of. We have odor events all the time on our aircraft and 99.99% of the time they go away with no issues and we continue on. We become biased to believe it is no big deal, and generally the first thought isn't to get out the Smoke/Fumes/Odor Checklist. Many believe they must have burned something in the oven or the airplane was de-iced and it's a residual smell or something like that. We don't usually think about how the longer-term breathing of an odor or fume will affect us even if the smell is tolerable because we may be pre-occupied with flying the aircraft around weather or preparing to land. We are always taught to fly the airplane first so it's possible that we might not react as quickly as we would like. We sometimes don't share how we are doing with our fellow crewmembers; I wish I had known my First Officer was starting to feel sick in the air. I was flying and concentrating on landing and should have asked him how he was doing, but I did not. I have a very sensitive sense of smell, so I thought the strong odor was more me than anything; We need to remember that in the future should something like this occur again, we need to open up a conversation much sooner and ask about how each person is feeling. We as pilots need to bring the Flight Attendants more into the conversation. While I pride myself on communicating with our Flight Attendants, there is always room for improvement. While I was getting feedback from them on how things were going in the cabin, I probably could have asked more and better questions to include asking how they were feeling. I didn't know until after we deplaned that at least one flight attendant felt light-headed.

Synopsis

A320 Captain reported a "burning plastic" odor during descent which resulted in the entire flight crew experiencing physiological symptoms. After a safe landing, maintenance requested and flight crew was referred for medical evaluation.

ACN: 1897263 (638 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1897263
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Passing through 12,000 ft., the FO (First Officer) and I experienced a fume event. We both noticed a musty dirty sock smell, indicative oil contamination in the ECS. I put on my oxygen mask and ran the cabin odor isolation procedure as per the manual. The smell seemed to go away after a few minutes after the "PACK-1 OFF" step. Inflight did not smell anything in the cabin. Upon arrival, I entered the discrepancy into the Maintenance logbook, filled out the odor survey, and contacted Maintenance. After deplaning, two of the Flight Attendants agreed that the flight deck smelled very strange, but could not describe the odor. One of the two FAs (Flight Attendants) then told me they had a metallic taste in their mouth, which is exactly what I am experiencing a day later. All required reports submitted.

Synopsis

A320 Captain reported a fumes event during descent described as a "musty dirty socks" odor. Flight crew completed the cabin odor isolation procedure which eliminated the odor.

ACN: 1897257 (639 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1897257
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent passing through clouds odors like sweaty socks lasting 5 minutes was detected in the cockpit only. On gate arrival after starting APU and selecting APU bleed, odor immediately detected in cockpit only again. Odor in both cases was moderate.

Synopsis

A320 flight crew reported a "sweaty socks" odor which lasted for 5 minutes during descent and also at gate.

ACN: 1897145 (640 of 2908)

Time / Day

Date : 202205

Place

Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 20800
Experience.Flight Crew.Last 90 Days : 139
Experience.Flight Crew.Type : 11067
ASRS Report Number.Accession Number : 1897145
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 75
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 75
ASRS Report Number.Accession Number : 1897168
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise our aircraft experienced a HYD G RSVR OVHT ECAM message. Flight crew accomplished ECAM and followed up with QRG checklist. Overheat extinguished 10 minutes later. Flight Crew and Dispatch communicated during the flight and agreed the use of Priority Authority was not necessary at the time due to the expectation that full hydraulic power would be restored prior to approach and landing. Dispatcher made contact with ATC to advise of our maintenance issue. When we checked in with Approach Control, the controller assigned priority handling on behalf of the flight crew. Per the checklist, the Green Hyd Pump was selected on prior to commencing the approach, with no further overheat issues. Approach, landing and taxi into gate were uneventful under ships power. Reviewing the aircraft log history shows numerous Green Hydraulic overheats occurring to this aircraft in the last 4 days.

Narrative: 2

Approximately 1.5 hours into our flight at cruise HYD G pump overheat appeared on ECAM. Captain flew the aircraft and I as first officer ran the ECAM and then the subsequent checklist. After about 10 minutes the pump returned to normal operating temperature and the crew decided to continue flight. Within 15 minutes of landing, unprompted, approach queried flight crew about the status of the hydraulic system. Captain explained the situation in addition to expressing that a normal approach and landing was anticipated. Approach then stated that they would be assigning priority handling on crews behalf. G Pump was turned on per checklist during approach phase and the flight had an uneventful approach and landing followed my an uneventful taxi in to gate.

Synopsis

A320 flight crew reported a Green Hydraulic System Overheat message at cruise. The flight crew performed the QRH procedures and continued to the destination airport.

ACN: 1896919 (641 of 2908)

Time / Day

Date : 202205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Component : 1

Aircraft Component : Turbine Reverser
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Engine Starting System
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Landing Light
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1896919
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Anomaly.No Specific Anomaly Occurred : Unwanted Situation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Work Refused
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I refused [the] aircraft due to excessive MEL's that negatively affected flight safety being Dispatched into convective weather and rain with possible low ceilings at destination. Flight was dispatched away from a maintenance base with 6 MEL's of which when all tallied up affected multiple aircraft systems both on the ground and in flight. Notable systems were, loss of thrust reverser, loss of predictive Windshear, loss of auto start protections requiring manual start, loss of avionics bay cooling functions, loss of all nose gear lights(expected arrival time would have been dark or near dark) and loss of brake cooling among many others. Also it set the airplane up for a single point failure (failure of lcgiu1) for a complete loss of autopilot and flight director function in addition to the above. There is no good reason to send an aircraft in that condition with a full passenger load out of a heavy maintenance base into the weather conditions that were present at the time of scheduled departure. MEL's that were applied to aircraft were. XX-XX, XX-XXg, XX-XXb, XX-XXb, XX-XXa, and XX-XX. Causal factor is flight operations and maintenance failure to consider the excessive work load and backup system loss that resulted from this MEL combination being Dispatched into potentially heavy weather arriving at night. In addition the failure to consider the consequences of having another failure in flight with so many systems already degraded or inoperative. My suggestion is when these decisions are being made for the decision maker to ask themselves "would I want my family on that airplane". In the case of this aircraft, my bet their answer would be a definite no.

Synopsis

A320 Captain reported a refusal to accept an aircraft with excessive MEL's. The aircraft was scheduled to fly in an area of convective weather and arrive at dusk. In the interest safety the aircraft was refused.

ACN: 1896215 (642 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ1.TRACON
State Reference : US

Environment

Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1896215

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the arrival into ZZZ we were maneuvering around thunderstorms in the terminal area. I had contacted the flight attendants earlier about being seated for the last part of the flight due to the intensity of the weather in the area. As we neared some areas of concern on the arrival I called them to verify that everyone was seated in case we encountered turbulence. This conversation occurred somewhere around 10,000 feet, and during the conversations Name, the F1 mentioned that they had smelled a dirty sock odor during the descent. We continued the arrival and approach into ZZZ, accomplished an uneventful landing, and upon taxiing in discussed the issue further. The flight attendants stated that they had smelled a dirty sock odor from about 10,000 feet until gear down on the approach. We contacted Maintenance and made the appropriate write up and completed the Odor Event Form.

Synopsis

A320 Captain reported the Lead Flight Attendant reported a "dirty socks" odor during descent. Flight accomplished a safe landing and submitted a logbook discrepancy upon gate arrival.

ACN: 1896211 (643 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Descent
Flight Phase : Landing

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1896211
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent into ZZZ dirty sock smell at 8000 ft in cockpit noted out of Captain's air vents left side. On final approach at 2000 ft oil and fuel fumes noted in aft cabin reported by inflight - with headache reported. All systems operated normally during approach and landing phase and no APU operation during the event. APU started at gate and after engine shutdown odor/fumes disappeared. Event not reported to ATC and no priority requested. Event reported to company / logbook entry and Oder-Fumes Report filled out.

Synopsis

A320 Captain reported a fumes event described as "dirty sock" odor which began during approach descent and lasted until gate arrival.

ACN: 1896209 (644 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Climb
Flight Phase : Taxi
Flight Phase : Final Approach

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1896209
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
When Detected : In-flight
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fumes began on ground APU Bleed on, Packs on during ZZZ turn after heavy rain rolled through airport. Strong musty, oily, dirty sock smell throughout cabin. We opened cockpit windows to dissipate feeling it was the typical moisture related temporary dirty sock smell when introduced to packs. However, smell persisted to cruise altitude (APU bleed was used for takeoff but turned off with After Takeoff Checklist) when we called to aft cabin to verify with inflight if still experiencing. Inflight said odor was no longer evident, but the odor remained strong in flight deck as we decided to run the supplemental checklist to isolate using oxygen masks for 3-5 minute pack 1, pack 2 isolation periods. We encountered moisture related weather for the first hour of flight in and out of clouds just before executing the checklist as we entered clear air. We assessed the fume odor event was isolated to pack 2 after conducting checklist, odor dissipated. However, we also correlated the dissipation with the probability we exited the clouds and moisture being introduced to the packs. We therefore elected to return to both packs operation and did not encounter the odor until descent passing FL180 all the way till taxi in requiring us to ventilate the cockpit upon parking by opening the windows. Inflight also reported the odor upon descent in the cabin. We attempted to isolate on decent with packs but it made no difference, pack 1 or pack 2, the odor remaining strong.

Synopsis

A320 Captain reported an intermittent fumes event described as a "dirty socks" odor that dissipated and returned during troubleshooting and while flying in and out of clouds. Fumes remained constant from approach descent to arrival gate.

ACN: 1896206 (645 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1896206
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Odorless fume event: The following morning, after Flight ABC on Date, I woke up and experienced mild symptoms of aero toxicity, which were indicated by blurry double vision. The symptoms subsided to the point that I felt fit for duty. After completing Flight DEFA on Date 1 the symptoms increased to the point that I was directed to contact Company. The neurologist in the hospital ordered blood work and a CT scan to rule out the possibility of a stroke.

Synopsis

A320 Flight Attendant reported experiencing health issues from a reported fumes event and was directed to go to the hospital for medical evaluation.

ACN: 1896197 (646 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1896197
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After landing, flight attendants called saying they had a fume event in the back of the aircraft. Both flight attendants felt dizziness and had a headache fairly quickly after smelling it. They said it came out of the air gaspers. First Officer said he had a faint smell of it, but I did not. All stated it smelled very musty and possibly like dirty socks. They said it smelled exactly like the jar samples they had in training. No fume procedures were executed due to the fact that when I was notified, I was pulling into the Gate Area, and I immediately shut the engines down. Smell dissipated quickly after engine shutdown. APU was off at time of fume event, and flight attendants stated they started to smell it below 10000 ft. after I rang them with the no smoking seat belt chime.

Synopsis

A320 Captain reported the aft flight attendants and First Officer detected a fumes event that started during approach descent. The Flight Attendants described the odor as "dirty socks" which dissipated at gate during engine shutdown.

ACN: 1895870 (647 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component : 1

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1895870
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During the cruise segment, the ECAM alerted with HDY Y RSVR OVHT, Y ENG 2 PUMP LO PR, and Y SYS LO PR in Amber. The ECAMs directed us to turn off the PTU and the Engine 2 Yellow Hydraulic Pump. We consulted the Flight Handbook and monitored the Brake Accumulator pressure for the remainder of the flight and determined that we would have the Parking Brake on Landing. The Status Page showed a number of associated systems had failed, including Nose Wheel Steering. We decided to [request priority handling] out of an abundance of caution, and contacted Dispatch via ACARS to tell them we would need to be towed off the runway after landing. We briefed the Flight Attendants, and made a PA informing the passengers of our situation. The remainder of the flight was uneventful. The OVHT condition went out passing about FL200 on the descent and the Status page directed us to select Eng 2 Hyd Pump ON during approach. The Amber Fault light extinguished initially, but came on again soon after. We landed without incident, stopped straight ahead on the runway with a single, steady application of brakes and the Engine 1 Reverser. Once stopped, we set the parking brake with the Accumulator pressure, waited 3 minutes, and then shut down both engines. The Fire department inspected the Landing Gear and Brakes, gave us the "All Clear," and we were towed to the gate with the APU powering the aircraft systems. My only recommendation would be for the Ground Crew to double check the aircraft type for the issue. The Tug crew came out to the runway but had brought a 737 tow bar, so there was a short delay in getting towed to the gate while the correct tow bar was brought.

Synopsis

A320 Captain reported multiple ECAM alerts related to Yellow Hydraulic System. The flight crew elected to request priority handling and performed a precautionary landing at destination airport. The flight crew requested a tow to the gate.

ACN: 1895649 (648 of 2908)

Time / Day

Date : 202204

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1895649
Human Factors : Fatigue
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ZZZ - ZZZ1 I smelled a dirty moldy sock smell. I asked Flight Attendant (FA) 2 if she smelled anything, and she told me no. I became very tired to the point when they were cleaning the plane and doing security checks I fell asleep in a passenger seat. at this point, I was not aware what I was smelling and experiencing was an air cabin event. we flew back to ZZZ, on arrival at ZZZ the FA 1 smelled the dirty sock smell. and told the Captain, the Capt never told FA 2 or FA 4 that he was calling maintenance for the issue. FA 2 and FA 4 didn't smell the dirty sock smell on arrival into ZZZ but FA 4, ME, I had symptoms when arriving into ZZZ - headache, sore throat, fatigue, cough. I then walked to the service center in ZZZ and they took me off the trip to seek medical attention.

Synopsis

Air carrier Flight Attendant reported a fumes event during a turn-around sequence in which physiological symptoms on both outbound and return flights. At the completion of the second flight FA was removed from trip to seek medical attention.

ACN: 1895634 (649 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1895634
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the cruise segment, the ECAM alerted with HYD Y RSVR OVHT, Y ENG 2 PUMP LO PR, & Y SYS LO PR in Amber. The ECAMs directed us to turn off the PTU and the Engine 2 Yellow Hydraulic Pump. We consulted the Flight Handbook and monitored the Brake Accumulator pressure for the remainder of the flight and determined that we would have the Parking Brake on Landing. The Status Page showed a number of associated systems had failed, including Nose Wheel Steering. We decided to [request priority handling] out of an abundance of caution, and contacted Dispatch via ACARS to tell them we would need to be towed off the runway after landing. We briefed the Flight Attendants, and made a PA informing the passengers of our situation. The remainder of the flight was uneventful. The OVHT condition went out passing about FL200 on the descent and the Status page directed us to select Eng 2 HYD Pump ON during approach. The Amber Fault light extinguished initially, but came on again soon after. We landed without incident, stopped straight ahead on the runway with a single, steady application of brakes and the Engine 1 Reverser. Once stopped, we set the parking brake with the Accumulator pressure, waited 3 minutes, and then shut down both engines. The Fire department inspected the Landing Gear and Brakes, gave us the "All Clear," and we were towed to the gate with the APU powering the aircraft systems.

Synopsis

Captain reported multiple ECAM messages relating to Yellow Hydraulic System while at cruise. The flight crew requested priority handling and continued to the destination.

ACN: 1895344 (650 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1895344
Human Factors : Human-Machine Interface
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Overrode Automation
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Receive low altitude alert radio call from Tower. On an autopilot coupled turn to final to join ILS, aircraft turned and aggressively pitched down at the same time. I gave the autopilot a few seconds to catch up and correct itself, then I disconnected and took over manually for a visual approach. Tower called the alert after we disconnected and arrested the descent. We did not recognize any anomalies with the ILS or aircraft systems.

Synopsis

A320 Pilot reported autopilot failed to capture glideslope and aggressively pitched down resulted in ATC issuing low altitude alert.