ACN: 1895340 (651 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1895340
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Ground Event / Encounter : FOD
Anomaly.Ground Event / Encounter : Object
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The landing and taxi were routine. We had wing walkers and a taxi guide for the parking, the space was clear and we taxied in. APU was inoperative so I secured the right engine while we waited for external power to be applied. External power was applied and we powered the left engine down. Ground personnel came up after shutdown to say the top part of an orange cone had been ingested while we were idling waiting on external power. The cone was previously damaged and the top was partially torn from the base prior to our taxi in. The ground personnel had the base of the cone in hand when he walked up the stairs of the jet way. It was one of the cones that gets placed in front of the engines after shutdown. The base of the cone had none of the orange top still attached. I saw parts of the orange top of the cone in the engine when I looked at it. I wrote the engine up for a FOD inspection. If the cone hadn't been damaged, it would not have torn free from its base and been ingested when we had idle power and it was properly placed while we taxied in. It was just a bad cone.

Synopsis

Pilot reported ground handlers informed the flight crew that an orange safety cone had been ingested by the left engine during parking procedures for gate arrival.

ACN: 1895254 (652 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1895254
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During taxi to Runway XXR after both engines were running and the APU was off, the AFT Flight Attendant called and said she could smell "dirty socks". I asked her how intense the smell was on a scale of 1-10 and she said it was a 6-7. There was no visible smoke. I returned to the Gate immediately after shutting down the number 2 engine and taxiing on only the number 1 engine. I requested paramedics meet us at the Gate. I did not restart the APU and we waited for ground power to be hooked up before shutting number 1 down. The smell dissipated rapidly once we got to the Gate. One Flight Attendant was taken to the hospital with symptoms of nausea, headache, blurry vision and red eyes. Odor was only detected from over wing aft. We did not smell it in the cockpit and were both unaffected. The forward FA was unaffected. The other FA, although smelling the odor was also unaffected. This aircraft had a reported odor event the day prior as well. Dirty Sock Smell throughout the aft cabin. The company and all personnel handled the event professionally and proficiently.

Synopsis

Air carrier flight crew reported a fumes event that was described as "dirty socks" odor during departure taxi. Flight crew returned to departure gate where paramedics met the aircraft.

ACN: 1895058 (653 of 2908)

Time / Day

Date : 202204

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Wing
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1895058
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Equipment / Tooling

Narrative: 1

I was starting an afternoon overtime shift and was asked by Supervisor on duty to look into a tooling situation that resulted in a fuel leak on Aircraft X. We have 1 Airbus jack pad kit at ZZZ, kit PN XXXXXXXXXXXXX PC # XXXXXX. This kit was checked out on date for use on Aircraft X. I discovered that the kit contained 2 jack pads: 1 for each wing and 6 attach bolts, although only 2 bolts are used on each jack pad. I discovered that 3 of the bolts were approximately 2.5 inches in length and the remaining 3 were 2 inches in length. The jack pad identified for the left hand side had 2 of the 2.5 inch bolts secured in the transport holes of the adapter while the right hand side jack pad had 2 of the 2 inch bolts secured in the transport holes. In reviewing the Aircraft Maintenance manual it appears that the hardware included in the kit is not what originally came with the kit. The Aircraft Maintenance manual drawing depicts a fastener with a cable or wire strap that secures it to the pad and appears to have a special shape to the barrel of the bolt. It appears that a Technician installed the pad using the longer hardware supplied in the kit that damaged the dome nut resulting in the fuel leak.

Synopsis

Technician reported that as part of a crew preparing for jacking the Airbus aircraft, it was discovered that the bolts with the jack pad adapter kit were the incorrect length.

ACN: 1895054 (654 of 2908)

Time / Day

Date : 202204

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Wing
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1895054
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Equipment / Tooling

Narrative: 1

I was working in Hangar X and when [I] finished my job on another airplane, the lead told me to help those guys putting on the jack. The guy working there told me to [get] the jack pad that was on the table. I grabbed one of the two pads in the box and installed it to the airplane with the hardware attached to the pad in the left wing side.

Synopsis

Technician reported that as part of a crew, he was instructed to install a jacking pad adapter on the aircraft.

ACN: 1894337 (655 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1894337
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Detected dirty sock smell in cockpit on arrival into ZZZ descending through 3,000-4,000 ft. Contacted flight attendants and they reported no smell. I turned off Pack 1 and the smell dissipated.

Synopsis

Air carrier Pilot reported a fumes event during descent described as a dirty socks odor on the flight deck. The fumes dissipated after completing the appropriate QRH procedure.

ACN: 1894326 (656 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1894326
Human Factors : Troubleshooting
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Experienced a fumes event (dirty sock smell) on decent into ZZZ. Ran the Odor Recognition and Isolation Guidance checklist.

Synopsis

Air carrier Pilot reported a fumes event during descent. Flight crew completed the appropriate QRH procedures.

ACN: 1894320 (657 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1894320
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Enroute from ZZZ to ZZZZ (Aircraft X), we received an ECAM warning message for FWD and AFT Cargo Smoke. The message cleared, came back, and cleared again. We requested and were cleared direct ZZZZ which was the closest airport. Upon contact with ZZZ Center, we requested priority. I spoke with dispatch and maintenance control regarding the indication. I briefed inflight of the situation and that it was a code yellow. We landed at ZZZZ, stopped on the runway, and ARFF inspected the fuselage with infrared. We communicated to the customers to remain seated while the ARFF conducted their inspection. We were cleared to taxi to the gate and deplaned normally. ECAM messages entered into logbook.

Synopsis

A320 Pilot reported ECAM warning for forward and aft cargo smoke.

ACN: 1894318 (658 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1894318
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At XA10Z while descending through 8,000 ft. and passing through the clouds, we began to smell a faint dirty socks smell in the cockpit. The smell dissipated passing through 3,000 ft. Upon landing, the in-flight crew advised that it was a pretty strong smell mostly in the back of the cabin and they described it as a "dirty socks, fume" smell. A write up was made in the log book, local maintenance was called and the cabin odor form was completed.

Synopsis

Air carrier Pilot reported a temporary fumes event during descent that was described as dirty socks. After landing, cabin crew confirmed same odor was present in the aft cabin. Logbook discrepancy and company fumes form submitted to Maintenance.

ACN: 1894224 (659 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 31000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 50
Experience.Flight Crew.Last 90 Days : 50
Experience.Flight Crew.Type : 50
ASRS Report Number.Accession Number : 1894224
Human Factors : Troubleshooting

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 384
Experience.Flight Crew.Last 90 Days : 162
Experience.Flight Crew.Type : 384
ASRS Report Number.Accession Number : 1894240
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through FL310, a loud buzzing noise was heard in the cockpit. The Purser called shortly thereafter indicating a very loud sound coming from Door 1R. We continued the climb to our cruise altitude of FL340. In an attempt to alleviate the noise we descended to FL300 with no change. We then descended to FL280 with no change. The CA (Captain) conferred with Dispatch, Maintenance Control and the FAs (Flight Attendants) and the decision was made to return to ZZZ. We ran the diversion and recovery checklists as well as the overweight landing checklist. Due to the overweight landing we requested priority handling. The CA took the PF (Pilot Flying) role for the remainder of the flight. The diversion and landing proceeded uneventfully.

Narrative: 2

Flight departed ZZZ enroute to ZZZ1. Climbing through FL 310 to FL 340, a loud '"air-horn" like noise could be heard in the cockpit coming from the cabin. Shortly thereafter, the Purser called from the forward galley notifying us of the loud sound coming from Door 1R. Cabin pressurization parameters were normal and stable. Reaching FL340 the sound increased in intensity and we descended to FL300 and then FL 280. The sound lessened in volume at FL280 although the noise was still significant. Concurrently, we conferred with Dispatch and Maintenance Control and elected to return to ZZZ. Descending into ZZZ and passing FL260, the sound dissipated. We requested priority handling and accomplished an overweight landing on Runway XX successfully. The aircraft taxied to the gate uneventfully.

Synopsis

Flight crew reported a buzzing noise from the L1 door on climb. The crew elected to perform an air turn back and precautionary landing at departure airport.

ACN: 1893898 (660 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 100

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A350
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 16500
Experience.Flight Crew.Last 90 Days : 149
Experience.Flight Crew.Type : 3245
ASRS Report Number.Accession Number : 1893898
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

The event occurred at in the landing phase at approximately XA20 local ZZZ time. Other than the flight being operated late due to ZZZ1 congestion and delays, the flight was uneventful up until this event. I was the PF during the incident. ZZZ approach vectored us to join the Runway XX ILS final approach course, and we intercepted as planned. Subsequently cleared for the ILS XX approach, handed off to ZZZ Tower and cleared to land on Runway XX. We flew a stabilized approach without incident. Upon the 500 Ft Stable callout, I noticed in my peripheral vision a large aircraft (which turned out to be a A-350) holding in position on Runway YYR. I do not recall hearing a takeoff clearance for this aircraft, as the tower may have been in a "split operation". Winds were reported to be approximately 030/15G25. Evidently the A-350 had been cleared for takeoff, unbeknownst to me or my First Officer. At approximately 100 feet AGL, significant turbulence/jet blast was encountered with a brief period of loss of aircraft control. Full left sidestick deflection was made with the aircraft still rolling right due to the jet blast. The event subsided as quickly as it began, with aircraft control being regained and a normal touchdown occurred within the touchdown zone, slightly right of centerline. An inquiry was made after touchdown and tower stated that the A-350 had indeed been cleared for takeoff. We subsequently cleared the runway and taxied to the gate without incident. Upon reaching the gate, I made a phone call to the tower supervisor, and he was very apologetic for the incident and was going to look into the event.

Synopsis

A320 Captain reported a loss of control event during landing at approximately 100 ft. AGL when jet blast from an aircraft departing a nearby runway was encountered. Aircraft control was regained and the flight continued to landing.

ACN: 1893881 (661 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : ZZZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 169
Experience.Flight Crew.Type : 213
ASRS Report Number.Accession Number : 1893881
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Environment - Non Weather Related

Narrative: 1

We were inbound to fix ZZZZZ for an RNAV arrival to ZZZZ at night and with significant thunderstorm activity in the terminal area. ATC was clearly struggling and confused as they pulled us off the STAR and initially gave us direct to ZZZ VOR. They then gave us a heading, and contradictory and confusing holding instructions followed. At one point they simply said just stay north of ZZZ VOR--that was the extent of the holding instructions. Shortly after we were given direct to ZZZZZ and a descent to 14000, cross ZZZZZ at FL 200. We had to query ATC numerous times to confirm that they wanted us to do the ZZZZZ XX [arrival] for a Runway XXL landing vice ZZZZZ XY [arrival] for Runway YYR, which we had deduced on our own during a lack of information from ATC or ATIS. ZZZZZ XX for Runway XXL was finally confirmed. Inside ZZZZZ and at approximately 15000 altitude, ATC asked if we could proceed direct to fix ZZZZZ1. This is the standard merge point for the STAR and turns to this point are routine, however this was earlier than normal for the turn. We determined we could accept the clearance. Shortly after we were cleared to descend to 10000. On the STAR the altitude for ZZZZZ1 is 12000. At this point I made a horrible assumption-- having never landed from the east on Runway XX at ZZZZ, I assumed this was normal operations. I could clearly see the ground all the way down the valley to the airport at this point. I deselected weather radar on my ND (Navigation Display) and selected the Terrain display, oriented the chart terrain on the ND which appeared to be well to our left, and thus I continued the descent below 12000. The FO (First Officer) and I discussed the EGPWS terrain painting to our left, and that we appeared to be completely clear of terrain along our track to the runway. Shortly we started painting green EGPWS terrain in front of us, which very quickly started turning yellow. I was just verbalizing that I was no longer comfortable, but had not acted yet when we received a "Caution, Terrain" EGPWS verbal warning. The FO recalls that it was not this EGPWS verbal caution but a "Pull up Terrain" GPWS warning. I immediately disconnected the autopilot, selected TOGA thrust, and pitched toward full back stick. The EGPWS terrain that was displaying disappeared nearly immediately, the warning never repeated, and we were level at 12000 in about 20 seconds. I made a mistake here in that because we were visual and apparently cleared the threat so rapidly, I did not do the complete EGPWS Terrain procedure and never pitched to full back stick, which would have been appropriate since it was night time. The FO informed ATC that we had climbed to 12000, and they cleared us to maintain 12000 until ZZZZZ1 and cleared us for the ILS Z Runway XX. I reconnected the AP and autothrottles, re-armed the approach mode, and we conducted an ILS to Runway XXL followed by a sidestep landing on Runway XXR. Numerous lessons here for me. At the outset of the apparent struggles by ATC prior to our descent, we had correctly surmised that the airport had changed to Runway XX due to the thunderstorms interfering with Runway YYR. We thoroughly briefed the descent and approach to Runway XX in addition to Runway YY while we awaited a firm descent and runway assignment. I thought we had recovered well from all the confusion thrown at us early in the arrival, but I had clearly not and was still preoccupied with it. We briefed from the mountain flying guide that ATC may give you a clearance off the route and not ensure terrain clearance with the altitude assignment. This was in my mind as we descended below 12000 per instructions, but I did not verbalize it. I succumbed to my unfamiliarity with Runway XX, my disbelief that ATC could actually give us an unsafe altitude clearance, and it was all supported by what I initially was seeing visually outside the aircraft. In the back of my mind I knew staying at 12000 was what we should do, but I did not think clearly for those several minutes while collecting our thoughts and adjusting to a challenging evolution.

Synopsis

A320 flight crew reported receiving a "pull up" terrain warning while descending on arrival at night in a mountainous area. The flight crew executed an immediate climb and continued the arrival.

ACN: 1893321 (662 of 2908)

Time / Day

Date : 202204

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 105
Experience.Flight Crew.Last 90 Days : 105
Experience.Flight Crew.Type : 105
ASRS Report Number.Accession Number : 1893321
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On ground at ZZZ the airplane was extremely hot; cabin packs were not able to cool it down. It did not get better with engine start so I called Maintenance Control and they said everything seems normal on their end and it should cool down once airborne. During climb out we started smelling an oily dirty sock smell. I did a conference call with Dispatch and Maintenance Control and it was determined the best course of action was to divert to ZZZ1. [Priority handling was not received]. Landed in ZZZ1 and myself and the FO (First Officer) were not feeling so great. Called Maintenance Control and they pulled us off the remaining trip. Performed overweight landing in ZZZ1.

Synopsis

A320 Captain reported a dirty socks odor during climb. After conferring with Dispatch and Maintenance Control flight crew executed a diversion and a safe overweight landing. Flight crew was removed from trip due to physiological symptoms.

ACN: 1892609 (663 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1190
Experience.Flight Crew.Last 90 Days : 207
Experience.Flight Crew.Type : 1190
ASRS Report Number.Accession Number : 1892609
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Ground Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Captain and First Officer checked in with ZZZ gate agents at [the gate]. They were informed of a fuel spill cleanup in progress visible from the terminal window. Additionally, that the flight attendants (F/A) were on board. Both pilots entered the Jet Bridge and detected the odor of jet fuel. The Captain entered the aircraft and found the three F/A's gathered in the rear of the aircraft near the aft galley. He asked if they were aware of the fuel spill and directed them to open the window shades. All three thought that they smelled something but didn't pursue the thought. Captain then directed all three off the aircraft and joined the two pilots on the Jet Bridge where the Jetway door was opened for airflow. The Captain then called Dispatch inquiring if the ops center knew of the spill and operational impact. Coordination with maintenance came next to ventilate the aircraft of vapors. Gate agents came down the Jet Bridge informing the crew of station ops wishing to board passengers within 30 minutes- rebuffed by the Captain. Fire Marshal came on board to assess the condition of the cabin. The dispatch release still required sign off. And it just kept coming. At some point in this series of events the Captain realized his eyes were irritated and burning and nasal passages bothered. He then made the decision to refuse the aircraft due to the tremendous amount of vapor in the rear cabin and galley area and called dispatch. This led to a series of calls to "pilot push" him into acceptance. Captain attempted to find the refusal code to enter into computer but was unable to find it. With eye burning sensation worsening while again talking with Dispatch, Dispatcher offered to enter the write up. The crew left the Jet Bridge for their respective crew centers. Total exposure lasted approximately 60 minutes. The sense of smell acclimated to the odor and it wasn't until the sensation of eye burning made the crew aware there was a health concern to be addressed. Both pilots reported illness/injury to the crew desk and were removed from the remainder of the trip pairing.

Synopsis

Captain reported a fuel spill at the gate. With the aircraft being enveloped in fumes the reporter refused the aircraft.

ACN: 1892606 (664 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 478
Experience.Flight Crew.Last 90 Days : 68
Experience.Flight Crew.Type : 478
ASRS Report Number.Accession Number : 1892606
Human Factors : Troubleshooting
Human Factors : Physiological - Other
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Captain and FO (First Officer) arrived to Gate XX roughly XA20 local time, when speaking to agent we were informed of a fuel spill due to either aircraft issue or ramp fueler issue. Captain and FO proceeded down the jet bridge to assess the issue and whether or not this was simply a ground clean up problem or a mechanical issue on the airplane. When stepping onto the aircraft Captain noticed the Flight Attendants were on board and unaware of the condition outside even though the fuel odor within the airplane was overpowering. Captain and FO deplaned with the FAs onto the jet bridge, and after numerous calls with Dispatch/Operations Captain eventually came to the conclusion that he was Unwilling to Operate this specific aircraft to ZZZ1 as a result of the overpowering fuel fumes within the cabin. After which the Captain, FO and FAs all exited the jet bridge back into the terminal to provide an announcement to the passengers about the delay and that the timeframe for finding a replacement aircraft was unknown. By this time the Captain and FO had spent close to an hour and half inside the aircraft and jet bridge trying to coordinate and had noticed that both of their eyes were stinging as a result of the fume exposure (initially un-noticed as a result of significant distractions trying to coordinate with Dispatch). Later in operations after filing Captains safety and odor reports both Captain and FO remarked about their stinging eyes and slight headaches. It was at this point that the Captain and FO elected to declare themselves unfit to fly for that day.

Synopsis

A320 First Officer reported a fumes event during pre-flight due to either aircraft fuel issue or ramp fueler issue. After extended delay with no replacement aircraft available and experiencing physiological symptoms, the flight crew declared themselves unfit to fly.

ACN: 1892454 (665 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 6000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1892454
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending through 6000 ft., a moderate chemical odor was detected in the cockpit. Immediately referenced and accomplished Odor checklist. Odor did not dissipate after approximately two (2) minutes from initial actions. Selected ENG BLEED 1 - OFF. Odor dissipated after approximately one (1) additional minute. Odor detected for approximately three (3) total minutes. Upon arrival logbook entry [was] completed, odor form completed, Dispatch notified via SAT phone and discussion accomplished with Mechanic at aircraft. Debriefed Cabin Crew and was informed that there was no odor detected in the cabin.

Synopsis

A320 Captain reported a "moderate chemical odor" during descent. Flight crew completed the appropriated Flight Crew Operations procedures which cause the odor to dissipate. Logbook entry and company odor form were submitted at destination.

ACN: 1892449 (666 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Taxi

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1892449
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After arriving at the gate in ZZZZ, the inflight crew notified me that they smelled a strong dirty sock smell during deplaning. The APU was running, but they smelled the odor prior to the APU bleed being turned on. The odor persisted during the deboarding. I called [Maintenance Control] and local Maintenance came out. I agreed to help troubleshoot by completing 2 engine runs with guidance from the local technicians. Through isolation troubleshoot procedures we determined that the odor existed when either Eng 1 or APU bleed were supplying either or both packs. Maintenance Control and local Maintenance decided to defer both the Eng 1 and APU bleed. After landing in ZZZ on the return flight, the inflight crew notified me that they again experienced the odor during descent at approximately 18k ft.

Synopsis

A320 Captain reported being notified by the flight attendant crew during deplaning of a dirty socks odor in the passenger cabin. After maintenance action was taken, the flight was subsequently completed and the Captain was again notified of odor during descent.

ACN: 1892441 (667 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1892441
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Other / Unknown
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Routine Inspection
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After landing in ZZZZ, the aircraft started to accumulate a very nasty smell. We had just landed, turned the APU on, and proceeded to park at Spot X at ZZZZ. It was raining heavily on and off since a rain front had just passed over the field. We had only one operating Pack at the time; the number one Pack was operating as the number two was on MEL. The customers starting deplaning when the number four Inflight came to the cockpit, wearing a mask and blanket over her face stating she had to leave the aircraft because an odor in the aircraft was becoming overpowering. The cleaners then came onboard and every one of them complained about the dank, wet odor in the front of the airplane and did not want to clean the aircraft. Their supervision overruled the cleaners' opinions, and they quickly cleaned the aircraft. After the cleaners left, local AO wanted to board the aircraft however, the number four Inflight stated she would not fly with that odor on the plane. I asked all the Inflight if they agreed with the odor assessment and not boarding. They all stated that they felt very unsafe in that environment. I agreed with them, stopped the boarding process then my FO (First Officer) and I proceeded to run the odor isolation checklist to figure out the issue. After running the odor isolation checklist, we determined the odor to be emanating from the remaining number one Pack. The predominate location of the smell was between rows 4 and 5 and the smell, in my description, was like a wet rag laced with some chemical odor that was warm and burning. We shut off the Pack, opened all doors to dissipate the odors and the smell in the aircraft finally cleared out. Local maintenance came out and was directed by Company Maintenance to put the remaining Pack on MEL. My FO and I volunteered to ferry the aircraft back to ZZZ for repairs.

Synopsis

A320 Captain reported a fumes event during deplaning at destination. Flight attendants refused to fly with the persistent odor after aircraft had been cleaned. Captain and First officer agreed to ferry the aircraft to maintenance base for repairs.

ACN: 1892439 (668 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1892439
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending through 5,000 ft. over Lake X, we encountered a strong fume / odor event in the cockpit. The smell could be described as burning oil, acrid and very chemically in nature. After what must have been just a couple seconds, the First Officer (FO) asked, "do you smell that?" After which, we both donned our O2 masks and continued the approach and landing. Note, the O2 masks remained on until reaching the gate where upon engine shut down the cockpit windows were opened for ventilation. Upon removing our oxygen masks, we did not smell any of the odors as described above in the cockpit. After this, we asked the F1 if they smelled anything out of the ordinary on the approach--they did not. Upon recording the event in the aircraft logbook, I was selected for a drug test, and the FO departed for the crew room. In regards to the aircraft configuration at the time of the fume / odor event: the aircraft was in a managed descent descending through 5,000 ft. with thrust at idle, and flaps in configuration 1. The APU was placed on MEL (for fume / odor event) and both PACKS were ON. The Cockpit Odor Report was completed later that evening.

Synopsis

Air Carrier Captain reported a fumes event during approach. The odor was described as a "burning oil and acrid". Flight crew donned oxygen masks and executed a safe landing. Upon gate arrival an aircraft logbook entry was submitted to maintenance.

ACN: 1892437 (669 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1892437
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After pushback, single engine start, and disconnect a squall line hit the airport. We sat on the ramp for approximately 10 minutes talking with Dispatch to get a reroute around the weather and decided to shut down the engine to conserve fuel as the sit would be a bit lengthier than initially expected. After shutdown the FO (First Officer) noticed a musty dry, dirty sock smell. Eventually I could smell the same odor. We queried in-flight and they agreed they were picking up the scent in the forward and aft cabin. One of the inflight described that it smelled like a box of crayons. It was what the FO and I would categorize as a Level 2 moderate and persistent smell. We shut off the packs and APU bleed valve and noticed the odor slowly, but eventually dissipated. We notified ZZZ Ops that we would be doing a gate return and would require maintenance. On taxi back to the gate I noticed that my throat was irritated and the FO said his lips and tongue felt a bit tingly. As we deplaned a few passengers seated in the middle of the cabin mentioned they noticed the odor as well. After we deplaned and spoke with maintenance the FO also had a dull headache and what he could only describe as "brain fog". No other physiological symptoms were present by the inflight crew beyond the strong odor they could smell in the cabin.

Synopsis

A320 flight crew reported a fumes event after pushback during a taxi delay. The fumes were described as a "dirty socks" odor which caused the flight crew to experience physiological symptoms. The flight crew executed a ground gate return for maintenance.

ACN: 1892385 (670 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors

Component

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1892385
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1892394
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft Dispatched with MEL XX-XX-# for the Engine Driven Generator Channel on the #2 engine. The IDG (Integrated drive generator) had not been disconnected at that time and the MEL instructed to only turn it off if directed by the ECAM (Electronic centralized aircraft monitor). After pushback, Assigned EDCT (estimated departure clearance time) XA:45 local. Single engine (#1) taxi to Northwest Hold pad and engine shut down to conserve fuel using only the APU. XA:35 started both engines and accomplished after start checklist associated with MEL. Just prior to taxi we received an amber ECAM message (ELEC IDG 2 Oil Low PR) instructing us to turn off IDG 2. Capt called maintenance control to confirm they wanted the IDG disconnected. They advised us to follow the ECAM. After the IDG 2 was turned off, the ECAM disappeared and the IDG was correctly showing DISC on the ELEC page. Maintenance control advised us we were safe to proceed. We continued as planned and departed ZZZ. While climbing on the departure the same ECAM reappeared. Attempted to disconnect the IDG again following ECAM and Quick Reference Handbook procedures. Momentarily showed DISC again only to re-engage seconds later. At that point Captain assigned pilot flying duties and radios to me. He used crew phone app on his iPad to call maintenance control to trouble shoot the problem. They attempted to disengage the IDG two more times without success. Captain in concurrence with maintenance control made the decision to return to ZZZ for an overweight landing. Accomplished all associated Quick Reference Handbook procedures, notifications and land assessment. Declared priority with approach control and requested airport rescue and firefighting. Successful landing with no further issues or instances. Canceled priority once clear of the runway and taxied to the gate. IDG 2 failure to disconnect after multiple attempts. Not sure if it could've been avoided. Prior to takeoff aircraft was showing all correct indications. System failure to disconnect IDG 2 correctly in flight.

Narrative: 2

MEL XX-XX-# was applied to this aircraft. After the second engine start, we received an ECAM (Electronic centralized aircraft monitor) directing us to disconnect 2 IDG (Integrated drive generator). We contacted maintenance crew and after their concurrence we disconnected IDG 2. I questioned whether this would work given the MEL already had GEN 2 OFF, but the ELEC page confirmed the IDG was disconnected. We departed and on the climb out, the IDG came back online with a ECAM directing us to disconnect it again. I contacted maintenance crew via the Dispatcher using the crew phone app. At this point, the IDG ECAM was going on and off multiple times. I again selected the IDG off at the direction of maintenance crew (we tried this 3 times). The IDG would not stay disconnected. I again questioned whether the GEN 2 being offline had anything to do with it, but they didn't seem to think I so. Maintenance crew requested a return to CLT. With Dispatcher concurrence, we returned to CLT. We requested priority handling with CLT approach due to our overweight landing and had safety vehicles standing by. We landed smoothly 4,000 lbs overweight. We cleared the runway, cancelled our priority handling and proceeded to the gate. We swapped aircraft and proceeded to ZZZ without further incident. I'm of the opinion that an aircraft with this type of MEL should not be dispatched to non maintenance stations, and definitely not to a small island in the Caribbean. This MEL was a Category B on day 2 of the 3 day repair window and being an IDG (red guarded switch) added an additional level of stress when working through procedure. This situation could have been avoided. Just because the MEL allows Dispatch to a station doesn't make it smart.

Synopsis

A320 flight crew reported a #2 IDG failure to disconnect when selected multiple times. The flight crew elected to perform an air turn back and make a precautionary landing at departure airport.

ACN: 1890509 (671 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1890509
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon arrival at the gate, Flight Attendant (FA) notified me they detected an odor in the area of the aft jumpseats/galley during descent. One FA reported a burning sensation in her throat. The other (both seated in the aft jumpseats) reported feeling light headed and having a headache. They described it like "an aerosol or ozone that you would experience in a hair salon"

Synopsis

Air Carrier Captain reported being notified by a flight attendant at gate arrival of an odor during descent. The odor described as "an aerosol or ozone" fume was detected in the aft galley area. Flight attendants reported experiencing physiological symptoms

ACN: 1889590 (672 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1889590
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During vectors for our approach into ZZZ, we noticed a "dirty sock" smell in the Cockpit. The packs were on the engine bleeds and the APU was off. We isolated each pack, but the result was the same. We started the APU and used the APU bleed for the packs, but we did not notice a change. Our landing was uneventful. We later learned the "dirty sock" smell was encountered throughout the cabin. I entered this maintence discrepancy into the logbook and completed the odor/smoke form.

Synopsis

Air Carrier Captain reported a fume event during approach which was described as a dirty socks odor. The flight crew troubleshot the odor with no improvements noted. At gate arrival a maintenance discrepancy was submitted along with a Odor/Smoke form.

ACN: 1889579 (673 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Final Approach
Flight Phase : Taxi
Airspace.Class A : ZZZ
Airspace.Class B : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1889579
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Taxi
When Detected : Aircraft In Service At Gate
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was first made aware of a dirty sock odor after the arrival of Aircraft X, ZZZ to ZZZZ. Upon landing and learning that we had to wait for a gate, we set up for a single engine taxi and shut down the number 2 engine. When we opened the flight deck door, the Flight Attendant 1 informed me that the Flight Attendant 3 smelled the odor on descent about 10 minutes out from landing. The cabin crew reported that the smell was still there at gate arrival but had dissipated a bit after engine 2 shutdown. I made a pass through the cabin and definitely smelled the odor lightly by mid cabin and more heavy in the rear of the aircraft. Since I didn't have advance notice of the odor, I didn't initiate any procedures. The odor spread throughout the aircraft as we sought to isolate the source with the mechanics. I called Dispatch via SATCOM and was patched through to maintenance who in turn had the local technicians attempt to isolate the smell. Following an engine run at the gate, it became apparent that pack 2 was the origination of the odor. Pack 2 was put on MEL and we operated the flight back to ZZZ on Pack 1 alone. The smell in the cabin was better on the return flight but the smell lingered thoughout the cabin and the flight deck all the way back to ZZZ where a Technician notified us of an odor history and took the aircraft out of service. The Flight Attendant 3 reported a burning sensation in his eyes on descent into ZZZZ associated with the odor. The cabin crew double masked once the smell was noticed but reported headaches with the smell. In helping the local mechanics analyze the problem, I too felt stomach pains and an associated headache due to the odor. None of the crew ever reported feeling sick enough to be ambulatory.

Synopsis

Air Carrier Captain reported being notified of a "dirty socks" odor at the arrival gate . The Flight Attendant noticed the odor during approach until gate arrival. After conferring with Maintenance the number two Pack was MEL'ed for the return trip to the crew base where the aircraft was removed from service.

ACN: 1889570 (674 of 2908)

Time / Day

Date : 202204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1889570
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During boarding, our inflight notified me that they smelled a strong unpleasant odor of burning metal or burning plastic in the aft galley. When I walked back there, I could smell the same thing. I immediately turned off the APU bleed and we notified ZZZZ Maintenance. While entering the issue in the logbook, I could smell a faint odor similar to acetone or other chemical in the flight deck. The FO (First Officer) described the odor as a crayon smell. These descriptions are characteristic of fumes events as a result of oil burning in the ECS. Two inflight crew members experienced mild nausea and I experienced a mild headache which resolved about 30 minutes later. The first Mechanic to arrive told us that the plane would be taken out of service for several hours and that we could expect a tail swap. We communicated with operations to deplane all customers and did so. The next Mechanic to show appeared to be in a managerial or supervisory role and was dressed differently than the others. He very sternly questioned me as to how I knew it to be a fume event if all packs were off. I explained that I turned them off to stop the fume event from occurring. He then told me that he didn't smell anything and that it could not be a real fume event because the inbound crew had no problems last night. I explained that 5 crew members confirmed the odor along with 2 customers who complained about the odor. He then explained that he would run both packs once fully deplaned and that he would simply sign it off and return it to service if he doesn't smell anything. I professionally advised against it, and recommended that he appropriately follow the guidance in the Aircraft Maintenance Manual and Information Notice. I filled out the Maintenance Odor Form and handed it over to him. Shortly thereafter, we were given a different aircraft and flew uneventfully to ZZZ. Upon arriving in ZZZ, multiple customers thanked us for changing aircraft because the first aircraft, "was very smelly" (their words). An elderly woman and her caretaker said the smell was awful in the last row.

Synopsis

Air Carrier Captain reported a fume event during passenger boarding. After troubleshooting and conferring with flight attendants a logbook entry was submitted which ultimately resulted in an aircraft swap.

ACN: 1889421 (675 of 2908)

Time / Day

Date : 202204

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2044
ASRS Report Number.Accession Number : 1889421
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Taxiway
Anomaly.Ground Event / Encounter : Ground Equipment Issue
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

During pushback, we lost comm with the push crew. We came to a stop, but did not set brakes because we had no communication from ground personnel. We tried to signal the person with the wands, but they did not understand what we were trying to say. We noticed the airplane moving back at the same time the tug driver ran into view and gave us a crossed arms signal. We applied the brakes and the aircraft stopped. When the tug driver reestablished communication, she said a new person accidentally disconnected the airplane too early. She said nothing was contacted and there was no damage. It was just a normal disconnect.

Synopsis

A320 First Officer reported loss of communication with ground crew during pushback.

ACN: 1888661 (676 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb
Route In Use : Vectors

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1888661
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On initial climb out passing 12,000 ft. cabin pressurization began to run away. The pressurization page came up and we observed 4,000 FPM. We immediately stopped our climb at 13,000 ft. When the plane leveled off and the power came back the cabin altitude and vertical speed stabilized. We had no ECAMs or warning or cautions. We asked to descend to 12,000 ft. Cabin altitude was operating normally. We asked to climb to 15,000 ft and when the power came up and we began to climb the same happened. We immediately ask for a descent to 10,000 ft. Again the cabin altitude and the rate of climb/descent was normal. We elected to [request priority handling] asked to hold at 10,000 ft. to burn fuel so as not to have an overweight landing. The cabin altitude stayed at 900 ft. during the hold. On descent for the approach the jump seaters noticed the cabin alt go into a slight climb for a few minutes than return to normal. [We had an] uneventful landing. [The problem was] high pressure air and pressurization computers malfunctioning [due to it being an] older aircraft. Some of the aircraft are old. Mechanical malfunctions will occur although in my years with the Airline they are very rare.

Synopsis

A320 Pilot reported cabin pressure control failure on climb and performed an air turn back and landing at departure airport.

ACN: 1887603 (677 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Dusk
Ceiling.Single Value : 2500

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use.Other 
Airspace.Class C : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Other 
Experience.Flight Crew.Total : 26000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 17000
ASRS Report Number.Accession Number : 1887603
Human Factors : Troubleshooting
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fumes event during flight. Noxious and toxic fumes filled the entire aircraft cabin and flight-deck during approach and landing/taxi. All occupants onboard were exposed to these fumes for more than 18 minutes. My crew of 6 and I experienced varying degrees of significant symptoms including, sore throat, burning runny eyes, hoarse vocals, light headedness, dizziness, concentration issues and breathing irritation during and immediately following this event. I used supplemental oxygen when I realized that the fumes were getting worse. I remained on O2 through landing. During deplaning I overheard several passengers concerns of exposure to toxic chemicals and fumes during flight. I made a logbook entry clearly stating we had a Heavy Noxious Fumes Event. After a brief post flight inspection of the engines maintenance techs discovered potential mechanical disintegration of an engine resulting in large metal fragments present in the oil. They grounded the aircraft for engine replacement saying 'This plane has major problems!' The maintenance staff smelt the residual toxic fumes on board. I don't think I bought off on previous fumes events entirely. I'm a believer now! This event was dangerous and could have been much worse. We were fortunate to land safely. Even though we just had this significant fumes event, my crew and I were encouraged by the Company to reluctantly fly another scheduled leg that same evening without evaluation by a medical professional. When queried on fumes exposure processes or procedures and med evaluation, company reps told me they didn't think any existed. When I spoke with MedAire at my hotel on my symptoms I was told I should not have flown after the fumes exposure.

Synopsis

Air Carrier Captain reported "Noxious and Toxic" fumes during approach. The flight continued and executed a safe landing during which the flight crew and passengers experienced physiological symptoms. At the arrival gate the aircraft was removed from service by maintenance

ACN: 1887365 (678 of 2908)

Time / Day

Date : 202203

Place

Altitude.MSL.Single Value : 10400

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 394
Experience.Flight Crew.Last 90 Days : 49
Experience.Flight Crew.Type : 394
ASRS Report Number.Accession Number : 1887365

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Electronic Device
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While in the descent, approaching 10,000 feet, the lead flight attendant called the flight deck to report that the passenger seated in seat X had a personal electronic charging device which had overheated and was smoking. They reported that they had doused it with water and put it in their thermal containment bag. They confirmed that the situation was resolved. We continued the descent and approach. We flew the ILS to Runway XXR and landed uneventfully. The thermal containment bag was placed in an aft galley cart. After parking, I sent a Maintenance Request to Maintenance, called Operations to let them know that the thermal containment bag was given to the customer service representative, and left a message with the Chief Pilot voicemail.

Synopsis

Air Carrier Captain reported being notified by a flight attendant during descent that a passenger's electronic charging device had overheated. The flight attendant stated they had properly used the Thermal Containment bag which resolved the situation. The Captain notified Dispatch and gave the bag to the CSA at arrival gate.

ACN: 1886283 (679 of 2908)

Time / Day

Date : 202203

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1886283

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1886289

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon descending in ZZZ, I smelled the dirty sock, locker room smell and was asked by the Captain, after landing if I had smelled anything during the flight and I had. Whatever corrective measures that should or could be done to these issues!

Narrative: 2

Captain, First Officer, and Flight Attendant 1 detected fume odors during flight.

Synopsis

Air Carrier Flight Attendants reported a fume event during descent. The odor was described as dirty socks and was confirm by the Captain.

ACN: 1886134 (680 of 2908)

Time / Day

Date : 202203

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1886134
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1886640
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The First Officer (FO) and I met in the cockpit and went through the pre-flight set up. We completed our flows as per [the airline]. The First Officer went outside to do the pre-flight inspection and I finished the cockpit set up for take off. I had brought a sandwich on board and went to take a bite when I heard a loud â€˜Popâ€™, followed by a "Wooosh" sound. Seconds later there was a very acrid smell and smoke. The [Lead Flight Attendant] yelled in shortly later about the smell and smoke. I then told her we had a problem and for her and the other FAs (Flight Attendant) to grab their stuff and get out. Once learning we had customers on the jet bridge I asked the [Lead Flight Attendant] to please get the customers back up to the terminal. I was able to call ZZZ Maintenance and say we have smoke in the cockpit and I am shutting down all power to the airplane. I turned off the EXT power, BATT switches and activated the fire switch to the APU for an immediate shut down. I met with the Flight Attendants (FA) again once their Supervisors showed up to lend support. Two of them later became ill feeling from the smoke. After meeting with probably 12 Tech Operations people, we went up to the gate and I gave a PA announcement to the passengers.

Narrative: 2

After arriving at the aircraft in ZZZ, I finished my flow and went out to do the walk around. As I returned back on top of the Jet Bridge door, I saw our Lead FA (Flight Attendant) escorting few passengers back to the gate and saw Captain rushed out of the cockpit. I also quickly smelled heavy electrical burning smell and saw some light smoke in forward cabin. Captain said he has taken the appropriate steps in cockpit and our Maintenance team should be arriving soon so we stayed half way towards the bottom of the jetway. Within a minute Maintenance team of four people arrived at the aircraft and also smelled the same. Captain told me and them that he heard a hissing sound in the cockpit followed by a heavy smoke. That smell was very strong and got a little headache from it so I decided to stay away from the cockpit while Maintenance team was working to find the origination of the smoke. Within 30 minutes Captain informed me that we have been pulled from the trip and we went to the hotel.

Synopsis

Air Carrier flight crew reported a fume event during preflight preparations at departure gate. A strong acrid odor and smoke was reported prompting the Captain to secure the aircraft and deplane the entire crew. Flight crew conferred with maintenance and were subsequently removed from the flight.

ACN: 1885944 (681 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1885944
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ZZZ1 to ZZZ. Three hours into the flight we descended from FL360 to 10,000 ft. for the arrival into ZZZ. After level off we detected a slight odor in the cockpit. As we continued the descent to 6,000 ft. the odor became more intense. All of the inflight confirmed the smell and characterized it as burning alcohol/oil in the aft galley. We realized the aircraft was having another fumes event and expedited the landing. Pack one was turned off after landing since that appeared to work for previous crew who had also had a fumes event in this aircraft. Unfortunately, after exposure to the fumes one becomes numb to the odor and it is difficult to determine if any action corrected the situation. At the Gate the next crew boarded the aircraft and confirmed the fumes were still present.

Synopsis

Air Carrier flight crew reported a fume event during descent which was characterized as burning alcohol/oil in the aft galley. Flight crew continued to troubleshoot the fumes issue as they expedited to a safe landing. After passengers deplaned the new crew verified the fumes were still present.

ACN: 1885934 (682 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1885934
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1885952
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was Captain operating Aircraft X on DATE. I arrived at the airport on the shuttle at approximately XA:20am local time. I proceeded immediately to the gate and found the FO (First Officer) already on board. We exchanged greetings and confirmed what we had said the previous day that the FO would act as Pilot Flying, PF to ZZZ and I would act as PF returning to ZZZ1. The FO then briefed me that they had completed the power up and receiving check lists. I then took the release and briefed inflight crew. Next, I entered the cockpit to retrieve a safety vest in order to perform the preflight inspection walk around. During the walk around, I found a greenish, somewhat foamy and oily appearing liquid dripping from the APU bay from the space between the two doors of the APU compartment. I further inspected to determine the fluid I noticed could not be from lavatory waste, water or drain masts. It was not from any of the other possibilities and the unknown fluid had formed a puddle of approximate dimensions of a circle of 2-3 ft. in diameter. I then completed the walk around inspection, looked for a member of Maintenance team outside and found no one, then re-entered the aircraft cockpit. I requested the FO call operations and request a Maintenance member to come to the aircraft. The FO said OK and good as we also had a fault that needed Maintenance to clear. In just a short time, Maintenance Lead, came to the cockpit and I told him of both issues. He performed a reset of the fault and said he would go outside to visually inspect the fluid leak. Maintenance Lead returned and told us that the leaking fluid was only type 4 de ice fluid. I questioned that since we do not deice in ZZZ1 if he was certain. Maintenance Lead said that it was not unusual for deice fluid to take a long time to exit the aircraft, but he was certain. I had not made entry in the logbook as no corrective action was required. We finished our duties and departed ZZZ1 for ZZZ. The flight was uneventful, and we landed and taxied to the gate. The preflight walk around for Aircraft X was completed by the FO who stated as soon as they returned that there did not appear to be anything leaking from the APU area. I thanked her and we continued to perform all preflight duties according to SOP. Part of these duties include checking Hydraulic fluid quantities are in the acceptable range, they were. The flight blocked out at XI51 UTC and all duties and responsibilities were performed in accordance with all regulation and SOP. Takeoff and climb were uneventful and the aircraft was cleared to ZZZZZ intersection and to climb unrestricted to flight level 340. At approximately XJ32 UTC, we were presented with a level 2 caution message HYD G RSVR LO LVL. I first stated, "I am pilot flying." Then directed for the FO to check there was no QRC including bypass item. She did and stated nothing was found. At that point I told the FO that I have had this very same hydraulic issue in a "Classic" A320 previously. I added that I could see no reason at this point to [request priority handling] as we did not need priority handling as yet. I further stated at this time that our intent was to continue to destination. The FO said she understood, and I then turned the flying over to her with the statement, "you have control". She responded with "I have control ECAM actions". I performed the ECAM and reviewed the QRH for the fault. We then discussed how we wanted to make notifications to Inflight, ATC and the company. After good discussions between the FO and myself, we made the decision to tell Inflight via briefing the number one flight attendant face-to-face and they could then tell their crew. Secondly, we would inform the company via ACARS as they needed time to arrange a ground crew to move the aircraft from the runway. We decided that we would let ZZZ1 Approach ATC know when we were directed to speak to them and at that time, we would [request priority handling]. We brought the number one flight attendant into the cockpit to brief them, we also told the number one flight attendant that this would be a [priority landing], and they asked several great questions regarding the loss of the Green hydraulic system. When all their questions were answered, the flight crew took a restroom break and the number one flight attendant returned to the cabin. I then sent an ACARS message to Operations Control that told them of the problem and our intent to land in ZZZ1. Very soon after I answered a SATCOM call from Operations Control to review the message and see if we wanted assistance from Maintenance Control. I stated they should let us know if there was anything of importance to add, but I have personally been through this same fault previously and I was not concerned. I did discuss at great length with the FO that when I had personally been through this previously, that the hydraulic fluid had drained from a broken fitting in the number one pylon into the number one engine creating a great deal of smoke. I emphasized that my experience was to remain open throughout the conduct of the non-normal situation to expect something unplanned to occur and that we would handle whatever else came up with the same professional adherence to SOP we had with the fault we were given. I went on to tell the FO that I wanted every bit of information they might have to add to the successful outcome, and nothing was to be left unsaid. I told the FO "I want to hear the craziest thing you have ever said, which is to say, I do not want you to hesitate to say anything and I value anything you have to say". We were cleared to descend using the ZZZZZ1 XX RNAV ARRIVAL to ZZZ1 and plan landing south. The arrival and approaches were briefed. On first contact with ZZZ1 Approach the FO also then [requested priority handling]. There was brief discussion on the nature of the emergency and what services we required. The FO passed along that we would require a slightly longer than normal downwind leg to facilitate deploying the gear via emergency procedures and that we would request CFR meet the aircraft after we stopped for a visual inspection, as we had previously discussed. ATC confirmed that this was acceptable, and we performed the longer downwind. The FO deployed the landing gear via the alternate procedure, and we requested to turn in for the visual landing. At approximately 1,000 ft. from landing, the flight attendants called to say that passengers were concerned with a smell of burning hydraulic fluid. I knew there was no time to do anything with that, so I performed the landing as we had briefed to a full stop and CFR conducted a visual and infrared inspection to declare all clear. We then started the APU and when the start was complete, shut down both engines using the SOP for after landing and parking. We did have to customize both checklists as some items could not be completed until we were at the gate and even then, the emergency gear extension handle was required to remain out and not stowed. The aircraft was towed by [Ramp] Personnel to Gate XX in ZZZ1 and the passengers were deplaned. The FO called to confirm our gate assignment and operations asked if we had [requested priority handling with] inflight. I asked the FO to not answer that request as we were still not complete with the flight. All possible checklists and procedures were complied with prior to leaving the aircraft once we were at the gate. The parking checklist was completed except for the emergency gear handle and deplaning was begun. Then someone from Operations knocked on the door and asked to speak to me to ask if I declared a situation to inflight. I asked if we could speak later. I spoke to all Inflight crew to thank them for their professionalism and dedication to customer safety. I reiterated the same to the two members of Inflight management who were now on board. I then called for Maintenance to come to the aircraft for a debriefing. The reply to this request was to ask if I had declared a situation to inflight, as I knew that this was an open frequency and that it was available for anyone to be listening, I did not answer. I asked again for Tech Ops to meet me at the aircraft. At this time the FO was ready to depart, and I took a moment to sincerely thank them for being such a great professional and told them how I genuinely appreciated them for making my job so much easier. We shook hands and briefly embraced and said goodbye. NAME1 from Maintenance was on board, and I briefed him on everything and asked him if he needed more than my short logbook entry that we experienced HYD G RSVR LO LVL in flight. He replied no and I handed over the aircraft to him. When I arrived in the gate area, the gate agents said I was supposed to wait and speak to Operations on the phone. The Gate Agent dialed the phone and the Operations person asked me if I had declared a situation to inflight. I replied that I was unsure of answering any questions at that time and would be filing all required reports. I then departed the airport and went home. Thoughts: I would like to know where the hydraulic leak occurred. Crew members need to not give out operational information, or ask questions regarding operational issues of emergency or non-normal situations over open radio frequencies.

Narrative: 2

I was the First Officer operating Aircraft X. I arrived at XA00 local time and began the power up and receiving checklist. The Captain arrived shortly thereafter, checked the logbook, and performed the walk around. He then reported seeing fluid under the APU. We called Maintenance for that issue as well as a reset on the ECAM. The Maintenance Technician assured us that the fluid was deicing fluid and he cleaned it up. He performed the reset. The rest of the preflight was normal. I was pilot flying first leg, and the flight to ZZZ was uneventful. I performed the walk around in ZZZ and took extra time to specifically look for any leaks or fluids. There were none. Takeoff and climb out were uneventful. We were level at FL 340 when we received a HYD G RSVR LO LVL. The Captain was Pilot Flying and called for the QRC. I checked that, including the bypass items. I said "No QRC". He then transferred control to me and I said "I have control, ECAM actions". We then carefully completed the ECAM, QRH, and discussed our plan of action. Subsequently we spoke with the Lead Flight Attendant, gave her detailed information about what to expect, and told her it was a Code Yellow. We then sent a message to Operations Control, answered a SELCAL from them, and discussed our plan with them. They were in agreement with continuing our flight to ZZZ1. Once on ZZZ1 Approach we [requested priority handling], gave ATC specific information and requested CFR meet the aircraft to confirm we were safe to be towed to the gate. We followed all checklists to manually deploy the landing gear and stopped on the runway as Nose wheel steering was inoperative. Once at the gate we completed the logbook and debriefed the flight attendants. Once all passengers deplaned, Inflight departed, Maintenance arrived. At this time I asked if the Captain needed me further. He told me I was fine to leave, and I departed.

Synopsis

A320 flight crew reported loss of Green Hydraulic System fluid in flight. The flight crew requested priority handling and performed an alternate landing gear extension, followed by a safe landing.

ACN: 1884516 (683 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1884516

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the takeoff roll, we received a caution and ECAM message "HYD Y RES LO." We elected to continue the take off as this was the safe option. We cleaned up and completed the after takeoff checklist. We ran the ECAM and QRH procedures uneventfully. After a SATCOM patch was initiated with Dispatch and Maintenance Control and in concurrence with all concerned parties, we elected to return to ZZZ. We were heavy weight so we had to burn down gas. Based on available landing distance and landing weight considerations, we elected to burn fuel down and land at a 158,000 pounds gross weight. Subsequent approach and landing were uneventful. We requested priority handling so that Crash Fire Rescue could be present and check the brake temperatures. Landing vertical speed was no more than 200 FPS. After landing, the brake temperatures never went above 500 degrees C. We taxied on our own power to local Gate X. Subsequent walk around inspection by both pilots revealed a blown hydraulic line in the Number 2 engine.

Synopsis

A320 pilot reported a HYD Y SYS LO ECAM message on takeoff. After trouble shooting, the flight crew elected to perform an air turn back and make a precautionary landing at departure airport.

ACN: 1884507 (684 of 2908)

Time / Day

Date : 202203

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors

Component : 1

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1884507

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Green hydraulic system failure. ECAM - hyd g rsvr lo level and hyd g eng 1 pump lo pr. Actions coordinated a hold with approach control to hold over ZZZZZ to afford us the time to troubleshoot and coordinate a plan prior to our approach into ZZZ. Executed the "non-normal" associated with the above ECAMS. Based on the status of the multiple failures associated with loss of green hydraulic system we declared flight 54 a priority aircraft with approach control for planning purposes. Contacted ZZZ Maintenance on VHF to foster a cohesive plan and review of the potential steps we would anticipate prior to our approach into ZZZ. Highlighted items discussed were alternate gear extension and no steering capability which will result in being towed off of the Runway. After discussion with Maintenance discussed the plan with my First Officer to find a shared mental model on which approach and expectations/cautions in what to anticipate before and after landing. I then followed up with communicating with the flight attendants providing them the test and then communicating with our customers. Approach phase coordinated a minimum of a 10 mile final with ATC. Actively reviewed and recalled ECAM ACTIONS with First Officer. Anticipating executing the Alt gear ext procedure we recalled the status page on lower ECAM related to approach procedure, ECAM directs us to turn on no.#1 eng hydraulic pump. Upon doing so, all malfunctions and status items were cleared and found normal. Landed with no degradation and taxied on our own power to the terminal. Advised flight attendants to remain seated and all was ok.

Synopsis

A320 pilot reported loss of Green Hydraulic system fluid and a Green Hydraulics System EDP low pressure ECAM message. The flight crew elected to hold for additional time for trouble shooting, the continued the approach to make a precautionary landing at destination airport.

ACN: 1884505 (685 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 11000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1884505
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending trough approximately 11,000 ft. and transiting a thin cloud layer, a noticeable chemical odor was detected in the cockpit. Immediately selected ENG BLEED 1 - OFF and PACK - OFF. Then referenced Flight Crew Operations Manual Odor Recognition and Isolation Guidance step 9, selecting X-BLEED - SHUT and then ENG BLEED 1 - ON. Odor dissipated after approximately 1-2 minutes from initial actions. Upon arrival, momentarily selected PACK 1 - ON. Odor immediately returned. Selected PACK 1 - OFF again and accomplished Guidance step 14. Logbook entry made, Fumes Report completed and discussion with Maintenance Control personnel.

Synopsis

Air Carrier Captain reported a chemical odor in the cockpit during descent. The flight crew completed the appropriate Flight Crew Operations Manual procedure which caused the odor to dissipate. The flight continued to destination.

ACN: 1883710 (686 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Initial Approach
Flight Phase : Descent
Flight Phase : Cruise
Flight Phase : Final Approach

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1883710
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent into ZZZZ, we experienced a fume event that lasted for the duration of the flight and taxi to the gate. Maintenance came aboard the aircraft at ZZZZ and also smelled the fumes and they took the aircraft out of service.

Synopsis

Air Carrier Captain reported a fume event "that lasted for the duration of the flight...to the gate". Maintenance met the aircraft and took aircraft out of service.

ACN: 1883699 (687 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZOB.ARTCC
State Reference : OH

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Icing
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1883699
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

On date as we reported to the gate we were informed that our ZZZ-ZZZ1 A320 flight Aircraft X was delayed due to maintenance. When the Tech Operations staff were finished, appropriate MELs were applied including 32-XX-XX inoperative. We complied with the MEL's required O procedures and left the gate at XA54z with 17+1 passengers. First Officer was the PF (Pilot Flying) then as per the MEL a TOGA takeoff was performed at XB10z. After reaching cruise altitude IMC southwest of ZZZ2, we received dashboard fault annunciations and LGCIU 1 fault ECAM. Non-normal SOP's were followed, the FO continued as PF and ECAM steps were performed followed by associated QRH procedures. Multiple systems were inoperative including normal gear extension, nosewheel steering, auto pilot, auto thrust, flight directors and wing anti-ice (operative for 30 seconds after selection). We decided, based on the current winter weather at our destination ZZZ1 along with reported icing conditions, to return to ZZZ. There was a large low weather system advancing northeasterly. ZZZ1 weather was 1SM -SN and OVC004. ZZZ weather was 10SM and BKN055. We requested a diversion back to ZZZ from ATC. ACARS message was sent to Dispatch. Captain communicated with the flight attendants and passengers. During the descent, the Captain took over PF duties. The First Officer reviewed the QRH and inoperative systems. Radar vectors were requested to the current runway in use, [Runway] XXR. After we both had reviewed the inoperative systems we discussed the manual gear extension steps and whether or not to declare an emergency. About this time ZZZ contacted us and informed us CFR would be standing by and that they had chosen to [request priority handling]. Subsequently Approach asked us if we could accept a visual approach Runway XXL. We had sent for landing data to both XX runways. As our landing weight was 116,400 lbs, VMC weather, and dry runway, we informed them we could accept XXL. The landing gear was lowered in accordance with the QRH. Three down and locked indications were verified. The approach and landing were uneventful. Differential braking was used to taxi clear of the runway. The aircraft was stopped, parking brake set and we contacted operations to coordinate a tow back to the gate.

Synopsis

A320 Captain reported departing airport with LGICU 2 on MEL. Upon reaching cruise altitude, LGICU 1 ECAM is received. The flight crew elected to perform an air turn back and make a precautionary landing at departure airport.

ACN: 1883356 (688 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Minimum Equipment List (MEL)
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1883356
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : MEL

Narrative: 1

We [were] operating flight Aircraft X ZZZ-ZZZ1. After door closure but before push back we get an ACARS message to contact Maintenance Control. I called on my phone and talked with Name, he informed me that there might be a problem with our aircraft maintenance logbook and an inspection that might be due. My first call was at XA:08 local time. Name informed me to hold the flight at the gate while they [were] researching the timing of the write up on log page XXXXXXX, of Aircraft X, there was supposed to be an inspection of a fan blade made within a certain time frame. We had the jet way pulled back to the aircraft and open the main cabin door. I informed passengers of the situation and that we [were] waiting on further information from our Maintenance Control in ZZZ. We communicated our situation with the Station Operations and Dispatch. At XA:48 local time I called Maintenance Control again, at this time I was informed that the aircraft was grounded because an inspection was due. I informed our passengers of the situation and asked them to deplane. The flight had been canceled. If the aircraft maintenance logbook had a block to where the time of the write up could be logged we as pilots could help track the timing of required instructions. The current logbook is of no use to pilots. If we know when a write up was done we could refer to our MEL/CDL (Configuration Deviation List) and see when an inspection or a resolution would need to be done.

Synopsis

A320 Captain reported a flight cancellation was caused by improper maintenance tracking of a required fan blade inspection.

ACN: 1883305 (689 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class B : ZZZ

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1883305
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft X ZZZ - ZZZ1. Enroute. ECAM Message appeared. AIR - ENG BLD 2 ABNORM PR. Executed ECAM procedure and follow up. Reset unsuccessful. Coordinated with Dispatch for diversion into ZZZ2 due to weather conditions at ZZZ1. Landed ZZZ2 without incident. [Priority handling requested] by Captain. This caused some confusion amongst the changing ATC facilities. Went ahead and [requested priority handling] for brevity sake.

Synopsis

A320 First Officer reported an ENG 2 BLD ABNORM PR ECAM at cruise. The flight crew elected to divert due to weather at destination airport.

ACN: 1882960 (690 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1882960
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cabin crew smelled an odd odor in the cabin during initial climb and noticed worsening of odor as we continued climb. Crew member informed the Captain regarding smell and Captain decided to divert to ZZZ. Fire department and paramedics met the flight and assessed passengers, crew and cabin. Crew members had mild headaches and dizziness and all decided to go to the ER for further assessment and blood draw for carbon monoxide poisoning. All flights and cabin crew were discharged with notes to observe got any worsening our presence of symptoms.

Synopsis

Air Carrier Flight Attendant reported a fume event during initial climb. Captain was notified of the "odd odor" and stated that cabin crew was experiencing physiological symptoms. Flight crew executed a diversion to a suitable airport where airport support personnel met the flight.

ACN: 1882672 (691 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1882672
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1882832
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of 14,000 [ft.] we encountered an extremely strong fumes event. The musty smell was overpowering and we immediately donned our oxygen masks. The FA's then called up and said they had the same strong odor throughout the whole cabin. The copilot and myself discussed our options and [requested special handling]. We decided to divert to ZZZ as it was the closest airport to us that we felt we could accomplish our tasks and land. Fire crews were requested to make sure there was nothing burning on the outside of the aircraft as the FA's at first told us it smelled like something was burning and later clarified it was a musty smell. Once at the gate , the smell still had not dissipated. Weather was low ceilings and heavy rain. Crew communicated well and coordinated for a successful landing.

Narrative: 2

While in climb phase, approximately through 14,000 feet, we smelled a strong and foul odor. It smelled acrid and like dirty gym socks. We put our oxygen masks on immediately. The FA's at that time called up and reported smoke or burning smell, the same smell in the entire cabin. We discussed our options and the Captain wanted to divert to ZZZ. I [requested priority handling] and got vectors direct to ZZZ. Crash Fire Rescue was requested for an external inspection. Once at the gate, we deplaned all passengers and crew.

Synopsis

Air Carrier flight crew reported a fume event during climb. The musty odor was also reported by the flight attendants. Flight crew requested priority handling and executed a diversion.

ACN: 1882529 (692 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Pneumatic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1882529
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

APU inop due to previous crew having fumes event within the previous 2 hours of us receiving the aircraft. Performed external air start checklist with APU deferred. After number 2 engine start and packs on per checklist, we could smell a hint of oil smell right away. With the crossbleed in auto and engine 2 bleed on with both packs on, the crossbleed valve would not open to supply pack My First Officer moved the crossbleed valve to open to verify that the valve would open. At that point, the crossbleed valve opened and we immediately smelled [a] very strong oil smell. Eyes burning, [I] could taste it. Immediately he put the crossbleed valve back to Auto and we went on oxygen and returned to the gate.

Synopsis

Air Carrier Captain reported a fume event during single engine taxi while performing a crossbleed engine start. When the crossbleed valve was opened the flight crew detected a strong oil odor and experienced physiological symptoms.

ACN: 1882524 (693 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1882524
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After both engines [were] shut down, we turned on the APU bleed to power the packs to cool [the] plane. Shortly thereafter, flight attendant said there was what appeared to be electrical smell and smoke in [the] cabin. Smoke was visible. We immediately turned off the APU, as the event seemed to have started when APU bleed was turned off. Also had FA's open rear doors, opened Flight Deck windows to create draft and dissipate smoke. Smoke did indeed dissipate. Once all passengers were deplaned, we turned on APU Bleed again, smoke immediately reappeared and was visible coming out of Flight Deck vents. Shut down APU and contacted Maintenance.

Synopsis

Air Carrier Captain reported a fume event during engine shutdown at the arrival gate. Flight Attendant notified the flight crew of smoke in the cabin shortly after APU bleeds were activated. APU was shutdown and maintenance was contacted.

ACN: 1881860 (694 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 5000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Descent
Flight Phase : Initial Approach
Airspace.Class B : ZZZ2

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1881860
Human Factors : Troubleshooting
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Descending through 5,000 feet on approach experienced light to medium fumes. Odor also reported by all inflight with light to medium in forward galley and more severe towards aft cabin with #4 flight attendant experiencing mild burning on upper lip and mild light headedness. Event reported as lasting approximately 5 minutes then dissipating.

Synopsis

Air Carrier flight crew reported a temporary fume event during approach. The odor was reported throughout the cabin and lasted approximately 5 minutes before dissipating.

ACN: 1881857 (695 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 8000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Descent

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1881857
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ZZZ to ZZZ1 flight. Upon descending at 8k I noticed a smell. Subtle. I waited a minute after closing the vents. Odor still there. I asked First Officer if he smelled something. He said yes what is that. I told him it was odor fumes and we did isolation procedure. After pack 1. Odors disappear upon landing.. At gate I asked Flight Attendants if they smelt it and the 2 in the back did.

Synopsis

Air Carrier Captain reported a fume event during destination arrival. Flight crew completed the appropriate QRH procedure and reported the odor dissipated upon landing.

ACN: 1881854 (696 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1881854
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the arrival phase at roughly 4,000 ft. we both encountered a strong "dirty sock" odor. It was strong for a few minutes then began to fade slightly and continued to do so until we arrived at the gate. We did not apply the odor recognition and isolation guidance given the fact that workload was high, we were being vectored for the approach and would be on the ground soon. Around 3,000 ft. the inflight crew called up front to notify us that they all smelled the odor as well. After landing I did not start the APU. After arrival at the gate and during the deplaning process I overheard a few of the deplaning passengers talking about the odor as well. I met with [maintenance] and described the situation then walked out with the whole crew. Everyone seemed fine and didn't require medical attention.

Synopsis

Air Carrier Captain reported a fume event during the arrival descent. No procedures were completed because the odor began to dissipate. Captain met with Maintenance Control after gate arrival.

ACN: 1881839 (697 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 20000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1881839
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending through about 20,000 ft., my FO (First Officer) and I noticed a faint odor that was quite different than previous cabin odor events that we have encountered in the past. We were both smelling a "plastic" smell similar to what you smell when opening a box with new electronics inside. It was a clean plastic smell rather than a burning plastic smell. Because it was so different and quite faint, we both felt this was not a fume event. Upon parking at the gate, I asked the rest of the crew if they experienced anything. There was no odor in the main cabin at all. I asked them how they were feeling and they said they were fine. Because my FO and I felt fine and the event was so different, we truly were uncertain as to whether or not there was even a problem. I exited the airplane to chat with the outbound CA (Captain). She and I spent a few minutes discussing the situation. In the interest of safety, the decision was made to enter the event into the logbook.

Synopsis

A320 Captain reported a fume event during descent which was isolated to the flight deck of undetermined source.

ACN: 1881836 (698 of 2908)

Time / Day

Date : 202203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors

Component : 1

Aircraft Component : Nosewheel Steering
Aircraft Reference : X

Component : 2

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1881836
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon gear extension during arrival, we received a nose wheel steering fault ECAM (Electronic Centralized Aircraft Monitor) caution. I elected to go around in order to troubleshoot. During the go around I decided to keep the gear down initially until we had time to evaluate the situation. After leveling off at go-around altitude, we consulted the QRH and attempted to reset the nose wheel steering with the n/w steering switch. The problem did not clear, and we determined it would be safe to retract the gear. We advised ATC of the steering failure and the fact that we may not be able to clear the runway under our own power. The approach and landing were accomplished successfully, and I was able to steer the aircraft clear of the runway using differential thrust and braking. After clearing the runway, we tried another reset, which was successful. We taxied to the gate with normal steering under our own power.

Synopsis

A320 Captain reported a go-around after an ECAM message "Nose Wheel Steering Fault on gear extension. After running the QRH procedures. The flight crew elected to perform a precautionary landing at destination airport.

ACN: 1879679 (699 of 2908)

Time / Day

Date : 202202
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Fuel Storage System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Fuel Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 740
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 740
ASRS Report Number.Accession Number : 1879679
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 133
Experience.Flight Crew.Last 90 Days : 47
Experience.Flight Crew.Type : 133
ASRS Report Number.Accession Number : 1879682
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We were at cruise and received an ECAM (Electronic Centralized Aircraft Monitor) caution "ctr tank 2 fuel lo level" which is normal when the tank runs dry. We waited a few seconds and a second blue ECAM directed us to turn CTR TANK 2 off and open the XFEED, which we did. We pulled up the fuel page and noticed a 1,400 pound imbalance which is also normal. We continued watching the fuel page and noticed the imbalance rapidly getting worse. We consulted the FM to fuel imbalance and followed the procedure, which was to turn tank 1 off, open XFEED and tank 2 should balance out tank one. The fuel imbalance started increasing at a rapid rate. Tank 2 went UP in fuel while tank one was falling rapidly, now approaching 3,000 pounds lower. I called Dispatch and had him connect me with M/C (Maintenance Control). M/C did not have a solution that we had not already tried. Suspecting a fuel leak, we made the decision to divert to ZZZ. I notified the passengers and flight attendants and we diverted to ZZZ. The fuel imbalance continued to get worse and the autopilot eventually had 3.2 degrees of right rudder in with left bank to keep us flying straight and level. We landed ZZZ and requested ARFF assistance to verify we did not have a fuel leak before taxing to the gate.

Narrative: 2

During cruise flight we received an ECAM (Electronic Centralized Aircraft Monitor) from low pressure from one of the center tank pumps. The tank was at 40 pounds so we believed the message would disappear however it did not. We proceeded to run the ECAM and at the conclusion we identified a 1,400 lbs fuel imbalance which is not abnormal. However, this fuel imbalance rapidly increased approaching our structural limitations. The Captain initiated the fuel imbalance QRH procedure after consulting the FM and communicated with M/C (Maintenance Control). M/C did not have any suggestions we had not attempted. We investigated whether this could be a false reading but determined it was a true imbalance due to the aircraft slowly entering a small bank with rudder trim while in level flight. Due to a possibility of a fuel leak, the aircraft uncoordinated flight condition and the uncorrectable fuel imbalance the captain elected to divert to ZZZ. We landed uneventfully in ZZZ and had ground ops trucks inspect the aircraft for a fuel leak before continuing to our gate. No leak was found.

Synopsis

A320 flight crew reported a diversion caused by a rapidly changing fuel imbalance.

ACN: 1879136 (700 of 2908)

Time / Day

Date : 202202
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Initial Climb

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1879136
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fume event. ZZZ takeoff to ZZZZ. Passing 3,000 ft. had odor in galley and cockpit. Number 1 FA (Flight Attendant) went on oxygen. Both me and FO (First Officer) also dawned mask. Diverted to ZZZ1 with an overweight landing.

Synopsis

Air Carrier Captain reported a fume event during initial climb. Flight crew dawned oxygen mask and executed a safe diversion.