ACN: 1855220 (751 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Drinkable/Waste Water Syst
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 99.73
Experience.Flight Crew.Type : 1770.20
ASRS Report Number.Accession Number : 1855220
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

Aircraft X was refused due to open write-up of water spraying into cabin from forward galley coffee maker area during takeoff roll combined with other water system write-ups. Aircraft subsequently swapped to Aircraft Y. This is the second time I have encountered this on Airbus aircraft since Date. A report was filed by me for Aircraft X for Flight XXXX on Date for this very same issue. I have never seen this before in XX years of Airbus flying and now twice in 5 months? Water escaping into any aircraft that is fly-by-wire and which has potential to escape the designed containment systems is a serious issue as well. This issue is particularly dangerous because it occurs during takeoff roll with communication inhibited and ECAMS inhibited. The original incident that I saw in Month was also accompanied by a Vent Extract Fault ECAM which created a distracting unsafe environment as well as we were departing for a foreign airport.

Synopsis

A320 Captain reported refusing the aircraft due to a potable water leak in the forward galley that was spraying into the aircraft on take off roll.

ACN: 1855108 (752 of 2908)

Time / Day

Date : 202111

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Horizontal Stabilizer Trim
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft.Other 
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1855108
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

THSA (Trimmable Horizontal Stabilizer Assembly) Master Spline threaded bushing (G1) was found missing safety wire during unrelated THSA general visual inspection. See AMM 27-44-00-710-001-A for safety wire installation requirements and Figure 27-44-00-991-00300-A test procedure of jamming simulation tool: Sheet 1. See log page XXXXXXX. This safety wire is removed and installed during THSA jamming tests, usually scheduled with some A checks. There is an inspection of the installed safety wire on the task card.

Synopsis

A320 technician reported required safety wire missing on THSA master spline threaded bushing discovered during an unrelated general visual inspection.

ACN: 1854475 (753 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1854475
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Engine 1 compressor stall required in flight shutdown. All ECAM actions were followed and First Officer was Pilot Flying. Priority handling was requested with ZZZ Center and we were given direct to ZZZ1. Dispatch was notified and FA communication was established. After being handed off to ZZZ1 approach we were vectored for an approach to RWY XX. ALL QRH follow-up procedures were completed and passenger PA accomplished. ZZZ1 approach assigned a discreet tower frequency that was monitored by ZZZ1 fire and rescue as well. Uneventful landing and roll out followed by inspection by ZZZ1 fire equipment in the east hold bay. After receiving the all clear we taxied to the open gate. The QRH has very good procedures for FA communications but a few salient bullet points should be added for the passenger PA announcements. All company and ATC people involved were at the highest professional level which reduced the overall stress of the situation.

Synopsis

A320 Captain reported compressor stalls required the in flight shut down of the #1 engine. Priority handling was requested and the aircraft diverted to make a precautionary landing.

ACN: 1854468 (754 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 6000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1854468

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent into ZZZ1 we leveled at 6,000 feet and as the power increased we noticed a dirty sock smell that lasted for approximately 1 to 2 minutes. When the power reduced again as the descent continued the odor dissipated. The flight attendants noticed it in both the forward and aft cabin in flight. Once at the gate when the APU bleed was selected the odor returned although it was much less noticeable. Possible problem with the APU causing some sort of contamination in the packs. This has been a continuing issue that the company has been working to correct. Apparently more needs to be done to prevent these types of fume exposures to both passengers and crew.

Synopsis

A320 Captain reported a temporary fumes event of "dirty socks" smell during landing approach. Odor returned at gate arrival.

ACN: 1854433 (755 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Flight Phase : Descent
Airspace.Class E : ZZZ

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1854433
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On taxi out detected odor. On descent noticed odor again and Flight Attendant in aft made complaint about it and possible eye irritation.

Synopsis

Air carrier pilot reported a fume event during taxi out and also during descent for destination. Aft flight attendant experienced physiological symptoms.

ACN: 1854147 (756 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1854147

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1854676

Person : 3

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1854148

Person : 4

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1854471
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight crew detected foul odor before boarding. Captain was notified, we all deplaned, Maintenance was called. 9 hour delay while filters were replaced.

Narrative: 2

Prior to boarding at XA:30 on Date in ZZZ on Aircraft X a strong cabin odor throughout the cabin. 9 Hour delay upon landing in ZZZ1 at XM:30 cabin odor came back. [Flight Attendant suggested to] take the plane out of service and fix the issue. Possibly figure out why itâ€™s happening more often.

Narrative: 3

Upon turning on the packs a dirty sock smell became very strong throughout the cabin. The Captain called maintenance. The flight was delayed 9 hours. We were cleared to take the plane to ZZZ1.

Narrative: 4

First flight of the day aircraft was brought to gate with APU running by ZZZ Maintenance. During preflight APU bleed was selected on and within a few minutes FA's (Flight Attendants) 3 & 4 came forward reporting strong odor in cabin. Subsequently odor was present in cockpit as well. In a matter of minutes the entire airplane and jet bridge was fill with a strong odor of dirty socks. All 3 FA's deplaned immediately as passengers had not yet boarded. I turned APU bleed off, called Maintenance and FO (First Officer) and I also deplaned. Two maintenance techs arrived and confirmed strong odor and I made appropriate aircraft maintenance logbook entry, informed Dispatch, completed fume event form and spoke to Operations Chief. ZZZ Maintenance preformed engine run, several checklist and replaced cabin air filters and completed AML (Aircraft Maintenance Logbook) sign offs. Nine hours later Maintenance Control released aircraft with APU inoperative. We departed ZZZ with revenue passengers and everything was fine until arrival in ZZZ1. Descending through 10,000 [ft.] the odor reoccurred but not as strong as before. FA's in aft cabin used supplemental O2 and we continued approach and landing. After landing and clear of runway I had FO to turn packs and engine bleeds off and odor immediately dissipated. After gate arrival I called ZZZ1 Maintenance, made AML entry, and completed another fume event form.

Synopsis

A320 Captain and flight attendants reported a fume event prior to boarding. Flight was delayed for 9 hours while maintenance corrected the fume issue.

ACN: 1853725 (757 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1853725

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We showed at XA:10 in ZZZ. We walked onto the plane and there were fumes present. After Maintenance came onboard he said we were done with that plane. The entire crew waited for further instructions. Eventually we were told never mind we were good to go. The Mechanic had deferred the APU. After conferring with the Duty Chief on whether it was a good idea to operate a plane with repeated fume events he suggested we do it. I elected to do a run up with no customers onboard. We had to wait until another plane departed before the ground crew could assist with a start. As soon as the air was selected on we had fumes throughout the cabin. The plane had multiple fume events. I should not be asked to try and operate a plane with multiple events when the only corrective action is deferring the APU.

Synopsis

A320 Captain reported a fumes event during pre-flight.

ACN: 1853717 (758 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1853717
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During idle descent from 10,000 to 3,000, mild dirty socks smell only in the cockpit. Dissipated as power increased at level off. VFR, clear of clouds, over water. Tech Ops crew could still notice smell when they entered the cockpit at the gate.

Synopsis

A320 flight crew reported a "dirty sock" fumes event during idle power descent.

ACN: 1853713 (759 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1853713
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Arrived at aircraft to find Maintenance dealing with a write up from the inbound flight. When the APU started and bleed turned on an inappropriate odor became noticed immediately. The crew exited the aircraft and we turned off the bleed and APU, then referenced the FOM. was already talking with Maintenance Control and made the logbook write up. Maintenance needed an engine run for troubleshooting the previous write up (this run was also used to troubleshoot the odor) so we did an external air start (believing the APU was causing the odor/fumes). After the start was complete and packs turned on, the odor/fumes again emanated from the gasper vents. After shut down the First Officer and I left the aircraft and I contact Dispatch and conferenced with Maintenance Control. The odor/fumes were intense enough to only abate when the aircraft doors were opened for ventilation. We both had slight headaches for a couple of hours after our last exposure. I had a feeling of light headed, or fog for a couple hours after as well. The flight was canceled and we went to rest. As of this writing I do not have any symptoms presenting that I believe stem from this event.

Synopsis

A320 Captain reported a fumes event during pre-flight after APU bleeds were activated.

ACN: 1853549 (760 of 2908)

Time / Day

Date : 202108
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Airspace.Class C : ZZZ
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Engine Air Indications
Aircraft Reference : X
Problem : Failed
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Nondestructive Testing (NDT)
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Inspector : 10
Experience.Maintenance.Lead Technician : 5
Experience.Maintenance.Technician : 11
ASRS Report Number.Accession Number : 1853549
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

DATE #2 engine was being changed due to a P3 inlet sensor which was improperly installed. This sensor was installed to the engine fan case with cherry max rivets that eventually sheared and caused the sensor to bounce around in the fan case causing the mount holes to elongate. Engineering was contacted and they found a permanent repair which was beyond our scope at this facility but verified that solid stainless steel rivets is what mounts the sensor boss to the engine case. The damage caused by this elongation led to the engine change which was done by contractors hired by Company. While the engine installation was occurring I inspected the mounting of the new engines P3 inlet sensor boss and found it to be mounted with cherry max rivets just like the one that was taking off wing. I immediately [contacted] engineering and my boss the Station Manager NAME and informed them of my findings. I stated " I'm not trying to open Pandora's box but you might want to consider issuing a FCD (Fleet Campaign Directive) to inspect the entire fleet for the sensor boss installation". Neither NAME or the Engineer replied nor called to discuss the issue. In my conscious I believe there are more nefarious acts of Maintenance taking place at [Company] out station. Under qualified Technicians working items/jobs they should not be doing. I'm not sure if this is the doing of the Station Manager NAME or the company pushing NAME into doing the maintenance the unseasoned crew is being tasked with. Example: An engine blade lube on an Airbus A320, not one of the Technicians assigned to the job had ever done a blade lube before. Not one of them had OJT on the task and the crew was asked to fill out a one time RII authorization form to buy back the job. The flying public is in danger. As I stated I cannot attest to the Maintenance done at other Company base or out stations but at ZZZ there are Systemic problems at this base!!!!! I have been a technician, lead, inspector, hangar and line supervisor with almost XX years of experience. This is not right! Something bad is going to happen in my opinion.

Synopsis

Technician reported the P3 inlet sensor on an A320 was installed incorrectly and the damage caused required an engine change.

ACN: 1853075 (761 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1853075
Human Factors : Physiological - Other

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1853084
Human Factors : Physiological - Other

Person : 3

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1853465

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The flight was deplaned due to a fumes event but shortly after boarding the plane I began to feel nauseous with a slight headache. I had recently got the vaccine and haven't been 100% so I attributed it to that. I ran to the restroom a couple times but then I felt like someone put globs of Vaseline on my eyes. I (number 4) help in the front and main cabin, said to the number 2 that I wasn't feeling well and shortly after (maybe 2 minutes) the number 1 called stating we needed to get our belongings and get off. We were already on the plane longer than usual before boarding passengers. We had already had a delayed boarding due to maintenance with an armrest and once we began boarding stopped due again due to the maintenance worker needing to do additional work. I could start to smell more of a stench of gas or something unusual in their cabin as I was talking to a passenger in the aircraft but not much in the back and we started deplaning.

Narrative: 2

We boarded about XX passengers but had to stop because of seat armrest needed to be secured or replaced. After 25 minutes we were able to board many more passengers because the mechanic had fixed the armrest. I, the 1FA (Flight Attendant) was standing in front of the cockpit and heard another mechanic who was in the cockpit say to the Captain that we have a fume event and everyone needs to deplane. I immediately felt a headache which I never get and felt a bit physically off. I asked the Captain and he said yes we are having a fume event and he was going to make an announcement immediately, which he did for everyone to deplane. I informed the rest of my crew of the situation, made an announcement also for everyone to deplane and after all passengers were back in the terminal the Captain went through his fume event list with the entire crew on the Jetbridge. ZZZ Flight service met us at the gate area and we were removed from our trip and all of the flight attendants, 3 of us went to the health center. I know I felt much better and my thought processes more normal after we waited the five hours at the hospital. They did a blood test and concluded that the tests were normal and gave us clearance that we were good to return to work.

Narrative: 3

Maintenance came on during boarding to an armrest. Boarding was halted and Maintenance declared a fumes event. Captain advised passengers to deplane. Passengers were deplaned and then crew deplaned.

Synopsis

A319 flight attendants reported a fumes event of unknown source during passenger boarding resulting in health issues. Flight was cancelled.

ACN: 1853014 (762 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1853014

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Had another fumes event. 3rd one in 2 months. The plane had a history of fumes events. Was last written up on Date1 or Date2. It would be nice if the FAA, Airbus, and our company got together and figured a way to reduce these.

Synopsis

A320 pilot reported multiple fumes events on assigned aircraft. Reporter suggested there needs to be collaboration between the FAA, aircraft manufacturer, and air carrier to remedy frequent fume events.

ACN: 1852895 (763 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Compressor
Aircraft Reference : X
Problem : Malfunctioning

Person

Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1852895
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Inflight Shutdown
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On the climb out, shortly after the cut back, ECAM displayed ENG1 Compressor Vane message. I was the pilot flying and called "my aircraft". The Captain ran the ECAM which turns out to be crew awareness. During the climb and cruise we kept an eye on Engine 1 parameters. EPR for Engine 1 was exhibiting some variations of +/-.15. A few minutes before the beginning of descent at ZZZZZ on the Hydrr 1, the aircraft shook really hard 4 times all the while ECAM was displaying ENG 1 STALL. it all happened very fast. I called "my aircraft". Flight Attendant chimed in to find out what was going on and right about then EGT on engine 1 shot up to the red. ECAM displayed ENG 1 Fail. The Captain [requested priority handling] stating we wanted to go to ZZZ, then ran the ECAM checklist. He talked to F/As, passengers, company via ACARS. Captain finished the checklist with the follow up items. We made a plan on how we would proceed after landing. At this point we were downwind for Runway XX. We landed without further issues. We pulled into the hold bay so fire rescue could inspect the aircraft. When no damage was found after inspection, we taxied to the gate. Captain made a log entry in the AML. Mechanical. Looks there may have been issues with the Engine 1 compressor.

Synopsis

A320 First Officer reported ECAM message ENG 1 Compressor Vane followed by the aircraft shaking hard four times. Engine EGT went into the red range and ECAM message ENG 1 Fail appeared. Flight crew elected to divert to ZZZ and make a precautionary landing.

ACN: 1852849 (764 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1852849
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after the First Officer and I boarded the aircraft in ZZZZ the inflight crew said they smelled the dirty socks smell coming out of the air conditioning system. I agreed with their assessment and we coordinated contract maintenance via phone with Maintenance Control. After lengthy troubleshooting (including low power engine run) the fumes were isolated to Pack #2. There was also a bit of residual fumes using Pack #1 on APU bleed air (however no fumes were noted using Pack #1 on engine bleed air). Maintenance eventually decided to defer Pack #2 and dispatch our flight. Oil servicing of the APU system was also checked, however no issues were noted. The #2 Inflight crewmember reported to me that she had a headache that she determined was caused by the fumes. We eventually departed 5 hours late for the flight to ZZZ1.

Synopsis

A320 Captain reported a fumes event during pre-flight. After a lengthy maintenance delay, Pack #2 was deferred and the flight departure.

ACN: 1852843 (765 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1852843

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent [we noticed a] dirty socks and "burnt" smell. This was on descent into ZZZ. We contacted the [Flight Attendant] and they said they also smelled it. At this time we were getting a quick descent and approach into ZZZ, so we landed the airplane. The event was entered into the logbook.

Synopsis

Air carrier Pilot reported a fume event during descent. Flight crew continued to a safe landing and submitted a logbook entry.

ACN: 1852842 (766 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Vectors
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Lubrication Oil
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Engine Air Pneumatic Ducting
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1852842
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Gate Agent / CSR
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Cabin crew and passengers (at about row 15) noticed odor during initial descent. Cabin crew initiated their procedures concerning the event. We conducted the descent, approach, landing and taxi to the gate without incident. Shortly after shut down, the Flight Attendant informed me of the odor, at which time (with the cockpit door now open) I could now also smell the odor. We then, also initiated our cockpit procedure. This drove us to make the proper logbook entry, fill out a report, and call Maintenance Control. We determined that this was a Level 2 odor event considering how long the odor lasted and in light of the fact that the gate personnel could also smell the odor in the jetway. The contract Technician on duty mentioned in his experience, this was often caused by over-servicing of the engine oil. We checked the ENG SD and it did happen to display 23 quarts of oil in the right engine and 20 quarts in the left.

Synopsis

A320 flight attendants reported a cabin fumes event during cruise. The flight crew and Gate Agent also reported that the fumes were still noticeable after arrival at the gate.

ACN: 1852461 (767 of 2908)

Time / Day

Date : 202110

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Checklists
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1418
Experience.Flight Crew.Last 90 Days : 153
Experience.Flight Crew.Type : 1418
ASRS Report Number.Accession Number : 1852461
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While sitting at the gate with passengers boarding, I heard a loud pop and the window shattered leaving me with glass shards all over me. The aircraft had a write up from the previous flight that a checklist needed to be replaced. After calling maintenance control, two technicians arrived at the aircraft. They said the previous crew had used the checklist as a sun blocker. They had a hard time removing the checklist. They believed that is why the window failed.

Synopsis

A320 Captain reported the cockpit windshield shattered during passenger boarding. Maintenance determined, based on previous logbook discrepancy, cockpit checklist was improperly used as a sunshade on previous flight.

ACN: 1852355 (768 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class E : ZZZ

Component

Aircraft Component : Electrical Distribution
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Galley
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1852355

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was advised by the B and C FAs shortly before reaching 10,000 feet, that there was a strong electrical smell coming from row 22-24, they advised the flight deck. Once we had arrived in ZZZ, passengers in the same area, advised the C and B FAs that there was condensation coming from the ceiling in the same area 22-24. When we got on the plane in ZZZ1 the flight deck advised us that the APU was inoperative.

Synopsis

A320 Flight Attendant reported a smoke and fumes event during departure.

ACN: 1852079 (769 of 2908)

Time / Day

Date : 202111
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1852079
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After landing in ZZZZ with a faulty weather radar. We turned on the APU clearing the runway with a five minute taxi to the gate. We informed Maintenance Control airborne of our radar issue, and contract maintenance technicians were there when the boarding door was opened. After a couple minutes speaking to contract maintenance and while deplaning we began to smell a sweet odor followed by dirty socks smell, which began to intensify. We immediately shut off the APU bleed to stop contaminating the aircraft. After deplaning the Flight Attendant 3 in the back of the aircraft had the same experience. Maintenance troubled shot and found leaking oil in the seal of the APU duct. They requested us to return to the aircraft and upon reaching the jetbridge we could smell the dirty sock smell immediately at the top of the jetbridge and stronger as we got to the aircraft door. Maintenance also tried to cover the smell with air freshener. We told them this was unacceptable and the odor irritated my nose and throat at this point. We informed Maintenance Control we would not be performing an engine runup as this had led to serious incapacitation previously at [company name].

Synopsis

A320 pilot reported a "dirty socks" fumes event during passenger deplaning. Maintenance determined the fumes were caused by an APU oil leak.

ACN: 1851846 (770 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121

Component : 1

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Component : 3

Aircraft Component : Electrical Distribution
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft.Other 
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Avionics
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1851846
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Fan blower avionics vent 20HQ wiring and associated connector burnt. Connectors 358QF14 and 183QF14 were found burnt along with wiring between said connectors and blower fan control relay 8HQ. Spattered metal and soot found throughout contactor box 107VU126. This is my second aircraft to find this type of damage in the past 2-3 months. I believe this is occurring due to poor manufacture design of vent fan control system. Redesign of system.

Callback: 1

Reporter stated this involves A320 aircraft, and it's becoming a widespread issue. He stated they have now seen 5 aircraft with the same burned wiring, and it appears to him to be under-gauged wire spec from Airbus.

Synopsis

Technician reported burnt wires and evidence of arcing and fire in circuits associated with the Avionics Vent Fan Blower. Reporter further stated he has now seen a total of five aircraft with similar damage.

ACN: 1851186 (771 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1851186
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent into ZZZ, passing 4,000 feet to 2,000 feet, we experienced a dirty sock odor in the cockpit. It lasted less than 5 minutes. The sky was clear at the time and we had not entered any clouds in the descent. Pack 1 was on MEL/INOP. The APU was not in use at the time. I completed a maintenance logbook write-up and a report.

Synopsis

A320 pilot reported a temporary fumes event during approach. Flight crew completed a safe landing and submitted a logbook discrepancy.

ACN: 1851183 (772 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use : Vectors
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1851183
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Regained Aircraft Control
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Rolling takeoff, First Officer flying, at transfer of control, First Officer powered up and almost simultaneously as Captain's hands on throttle / 40 knots "Airspeed Alive" First Officer calls out "you have control, you have control", as I started to take over, [the] nose began veering right and due to low speed and long runway elected to discontinue takeoff / reject unaware of nature of the problem. Once aligned with centerline and taxi speed, "remain seated" announced by Captain as we cleared runway. First Officer declared his rudder pedals had broke free of adjustment and went to the floor giving him no rudder authority. We elected to return to gate where maintenance locked pedals into desired position in accordance with the MEL.

Synopsis

A320 Captain reported a rejected takeoff and return to gate caused by the First Officer's rudder pedal adjustment failing.

ACN: 1851173 (773 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1851173

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending into ZZZ, we experienced a dirty sock/musty smell starting around FL180. The smell dissipated as we descended, and was completely gone during approach and landing. The #1 Flight Attendant (F/A) called at 4,000 feet, and reported that they were experiencing the smell throughout the airplane. I spoke to the FA after landing and the FA did not report that any customers had complained. To date, neither myself nor First Officer have experienced any physical problems. No further input.

Synopsis

A320 Captain reported a fumes event during descent starting at FL180. Fumes dissipated gradually and completely dissipated during approach and landing.

ACN: 1851036 (774 of 2908)

Time / Day

Date : 202110

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 961.32
Experience.Flight Crew.Last 90 Days : 108.03
Experience.Flight Crew.Type : 961.32
ASRS Report Number.Accession Number : 1851036
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise altitude, just prior to T/D, we received ECAM "WHEEL N/W STRG FAULT" followed shortly after by "BRAKES SYS 2 FAULT", neither of which had an ECAM procedure. I therefore asked the First Officer to reference the QRH, both of which had a QRH procedure. There was no airborne procedure for the WHEEL N/W STRG FAULT. We did apply the QRH procedure for BRAKES SYS 2 FAULT, but as soon as BRAKES SYS 2 FAULT cleared, we got a "BRAKES SYS 1 FAULT" ECAM. I transferred aircraft control to the First Officer, and also delegated the radios to him. I made voice contact with NAME via ACARS *call me request. Dispatch got Maintenance Control and an Operations representative on the call. I discussed the nature of the failure with all parties and after discussing continuing to ZZZ or diverting to either ZZZ1, ZZZ2, or ZZZ3, decide the safest choice considering runway length, weather, and emergency resources was a diversion to ZZZ3. Dispatch gave us fuel burn to and landing data at ZZZ3. They also notified the station and arranged resources. I briefed the First Officer what transpired and decided to have him remain Pilot Flying. I notified ATC of the diversion and advised ATC. The First Officer made a normal landing. We landed uneventfully, came to a stop on center line, and the emergency vehicles approached the aircraft and have the all-clear for us to be towed off the runway to the gate.

Synopsis

A320 Captain reported Brake Sys 1 and 2, plus N/W steering fault ECAM messages in flight and elected to divert and make a precautionary landing.

ACN: 1850348 (775 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1850348

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While climbing out of approximately 20,000 feet we received an ECAM G RSVR low level alert and noticed that the fluid quantity was zero. I transferred control to the First Officer and completed the ECAM actions. After referencing the QRH, there were many items now unavailable for use on landing. Among these were gravity gear extension and no nose steering upon landing. I contacted dispatch and maintenance to let them know our situation, and they advised us that ZZZ and tech ops would be ready for us upon landing. I reiterated to dispatch that we would need a tow in from the runway upon landing and was assured they would be ready. We briefly thought about diverting, but with a full load we were overweight. We elected to continue to ZZZ. We discussed at length on the flight southbound what we needed to do upon arrival. I chose not to inform inflight or the customers of the failure until closer to ZZZ. I called the #1 up to the cockpit about an hour out and explained to her we had a code yellow. I went into detail the nature of the problem and how it might affect our landing. I explained that I didn't think there would be anything abnormal on landing other than the fire trucks and being towed to the gate. Everyone was professional and did their jobs as trained. I made a PA to the customers about 40 minutes out and explained the failure in layman's terms and what they could expect on landing. Out of an abundance of caution with a hydraulic failure, I instructed the First Officer to request priority handling with ZZZ approach. After all pertinent information was exchanged we requested a 20 mile final to be sure we could get all checklist items completed before landing. On downwind we slowed and lowered the gear successfully by the gravity extension. We then configured the aircraft for landing. ATC asked if we were ready for the approach and we flew a 15 mile final to runway 10L. We reiterated to the tower that we would not be able to clear the runway upon landing. We touched down about 1500 feet down the runway and braked to a stop abeam Taxiway XX. I made a quick PA when the aircraft stopped to ask everyone to stay seated. We completed the after landing checklist, started the APU and shut down the engines. The tug then entered the runway, hooked up to us and towed us to Gate XX. The required logbook entries were made and I discussed our failure with CFR and tech ops. Tech ops said there was a major leak on top of engine number one in a hydraulic line.

Synopsis

A320 Captain reported diverting after the loss of the green hydraulic system and made a precautionary landing.

ACN: 1850347 (776 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1850347
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During preflight the cabin cleaners and flight attendants complained of dirty sock smell in the back of the airplane. The APU was supplying bleed air to the packs with the engines shut down. I closed the APU bleed and notified maintenance. The mechanic deferred the APU. Due to the persistent odor I opened the L1, R1 and R2 cabins doors and ran the air conditioning off external high pressure to attempt to remove the odor. While the smell slowly dissipated, it was still very detectable when we were swapped into a different aircraft 30 minutes or so later.

Synopsis

Air carrier flight crew reported a fume event during preflight while parked at gate. After maintenance trouble shooting an aircraft swap was ordered.

ACN: 1850340 (777 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 6000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class E : ZZZ1

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1850340

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Dirty sock smell on descent passing 6,000 ft.

Synopsis

Air carrier flight crew reported a fume event during descent for destination airport.

ACN: 1849581 (778 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Airspace.Class B : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1849581

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After deplaning all customers, our flight attendants stated that they smelled musty fumes from the mid to aft cabin area (rows 14-21) during low altitude descent during approach and that the fumes remained until the engines were shut down at the gate. I did not smell any fumes. After obtaining pertinent information from the flight attendants, ZZZ maintenance was notified, and a discrepancy was entered into the aircraft logbook. As was previously stated, the fumes event was brought to our attention only after deplaning was completed and we did not smell fumes in the flight deck.

Synopsis

Air Carrier Captain reported being notified at arrival gate by flight attendant of a fume event. Cabin fumes began during descent and lasted until engine shutdown at gate.

ACN: 1849311 (779 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1849311
Human Factors : Training / Qualification

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Ground Personnel
When Detected : Routine Inspection
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft X, ZZZ-ZZZ1 needed to returned to the gate due to no bags around the HAZMAT in pit 6. Advised ramp lead to surround the HAZMAT with 15 bags. Informed ZZZ Operations and Dispatch.

Synopsis

Air Carrier Ground Personnel reported ordering taxing aircraft to return to gate to correct Hazmat configuration error.

ACN: 1847685 (780 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1847685

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

This is a re-submission of an entry regarding this event, submitted here for increased visibility and likelihood that some action will be taken on this flight safety risk. Sadly, given the nature of this web form, all formatting is lost. This aircraft, Airbus A320, had two return-to-gate events, over two days, due to significant flight control anomalies. My main and overriding concern is that the current Airbus AMM (Aircraft Maintenance Manual) procedures for ELAC (Elevator, Aileron Control) failure and/or Aileron Servo Faults are either incorrect, and/or are easily misunderstood/misapplied, and/or can be easily confused with another related (or actually a different causal) flight control defect. On this flight, this likely combination of failures created an extremely significant flight safety hazard (Loss of primary flight controls, ailerons; Alternate Law) just prior to takeoff. In our case, after our first return-to-gate, and after many hours of diagnosis, our over 20-year old Airbus successfully passed all ground tests for Aileron Servo faults, and an ELAC #2 failure, with a "box swap" with ELAC #1. Maintenance later directed (that evening or early the next morning) ZZZ Contract Maintenance personnel to also placard ELAC #1 "INOP" with MEL XX-XXX. The next day we accomplished the 3- or 4-page test protocols as directed by MEL XX-XXX, which were all passed nominally. Given its significance, we made a specific reference to the note regarding "After first engine start, the F/CTL ALTN LAW alert may be spuriously displayed. It normally disappears after second engine start." on MEL Page XXX of X,XXX, (Item XX-XX-XX.X). We had no spurious indications, after either engine start. Furthermore, the post-second engine start flight control check was 100% nominal. However, during taxi, and only 2- or 3-minutes prior to takeoff, and also approximately ten-minutes post-second engine start, we had a surprising Master Caution, triple-chime, with an ECAM page seemingly full of F/CTL error messages, beginning with "L Aileron Fault" and "R Aileron Fault," and "Alternate Law (Prot Lim)." Along with above ECAM indications, we also noted that both ailerons were "XX'd" out on the lower ECAM display, and all four aileron actuators had amber brackets around them. We accomplished another flight control check to visually inspect our aileron positions. We were quite surprised to see that both were immobile/locked despite repeated, full, left/right side stick movements by either pilot. (Spoiler panels did appear to extend, at least partially, so we had that going for us.) Called Maintenance to advise of our system failures; they concurred that another return-to-gate was warranted. This event had a massive impact on flight safety, potentially resulting in loss of aircraft control at, or soon after takeoff. A slightly closer gate (shorter taxi time) would have likely resulted in a high-speed RTO (Rejected Takeoff), or with our aircraft at low altitude/low energy, with inoperative ailerons. (Note that there are no crosswind limits associated with this MEL.) Another area worthy of investigation: how/why this aircraft's internal diagnostic software presented such a lengthy delay in alerting the crew to the pending failure of both ailerons. Immediate advisory to Airbus, FAA, and others, regarding this MEL which does NOT require any wait time prior to takeoff to allow all internal diagnostic testing to complete. Anecdotal information suggests that older Airbus aircraft have "old" ELAC software and that "new" ELAC software may have prevented this event from ever occurring. If so, suggest an FAA Airworthiness Directive to require new software to be installed ASAP, with a set date in the near future. Suggest significant, crosswind limits be placed on this MEL so as to preclude an aircraft hull loss due to loss of ailerons during takeoff. To reduce risk to the flying public, consider "Ferry Only" limitation for this and other Flight Control MEL items.

Synopsis

A320 Captain reported significant Flight Control System anomalies affecting flight safety and possible inflight loss of aircraft control.

ACN: 1845873 (781 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 14000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 17200
Experience.Flight Crew.Last 90 Days : 153
Experience.Flight Crew.Type : 5853
ASRS Report Number.Accession Number : 1845873

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of 14,000 ft. we heard a loud bang. While investigating what it was there was a second bang and this time I saw glass shards coming off window 3L and saw the windows inner pane had completely shattered. We leveled off at 16,000 ft. assessed the situation and decided to turn to ZZZ. Dispatch was notified and with no pressurization, controllability, or overweight landing issues no emergency was declared. Approach and landing were uneventful.

Synopsis

Captain reported cockpit window 3L shattered during climb and elected to perform an air turn back to a safe precautionary landing.

ACN: 1845366 (782 of 2908)

Time / Day

Date : 202110

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft.Other 
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1845366
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Upon aircraft returning for reoccuring high stage valve bleed message, proceeded to perform MEL procedure XX-XX-S for #1 engine. Proceeded to lock out high stage valve. Closed up #1 engine and aircraft was released and departed. Received call from Maintenance Control asking if I had inoped high stage valve. I started to investigate further, at which point I realized I had locked out the intermediate valve. I immediately contacted Maintenance Control and informed them I had locked out the wrong valve. Maintenance Control said they would be notifying ZZZ Maintenance when aircraft arrives.

Synopsis

Technician reported locking out the intermediate bleed valve instead of the high stage bleed valve while applying MEL.

ACN: 1843317 (783 of 2908)

Time / Day

Date : 202110
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 28000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 679.90
Experience.Flight Crew.Last 90 Days : 107.92
Experience.Flight Crew.Type : 679.90
ASRS Report Number.Accession Number : 1843317
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 867.48
Experience.Flight Crew.Last 90 Days : 54.4
Experience.Flight Crew.Type : 867.48
ASRS Report Number.Accession Number : 1843339
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb passing approximately FL280 the flight attendant called and informed me that a passenger believed we were trailing fuel out of our #2 engine. All engine indications appeared normal including engine fuel flow. Checking our fuel we determined we were showing an imbalance of approximately 1500 pounds with the right wing being low. We continued to monitor to determine if the fuel imbalance remained stable or if the imbalance was increasing. We received a second call from the flight attendant informing us we had a company A320 instructor pass riding. He and I spoke and he told me it appeared to be fuel trailing the #2 engine. I sent the First Officer back to the cabin for a visual inspection and told him to look forward and aft of the wing and specifically any damage he might see on the forward edge of the wing. While he performed a visual inspection, the flight attendant in the cockpit showed me cell phone video of the leak. I leveled off at FL380 and verified the fuel imbalance was increasing to about 1700lbs with the right wing on the low side. I referenced the QRH for fuel leak and sent dispatch a message that we had a fuel leak, needed to divert and would be shutting down the #2 engine. The First Officer returned and informed me of the same information and we began running the fuel leak QRH. Dispatch recommended ZZZZ and looking at our distance and time required for descent I determined it to be a proper divert location. I [requested priority] with ZZZZ Center and requested an immediate turn to ZZZZ and descent to FL 200. The First Officer and I ran the fuel leak QRH which directed an engine shutdown for a fuel leak out of the engine or pylon followed by the engine shutdown checklist. We shut the engine down in an idle power descent and I started the APU. I then transferred flying duties to the First Officer. ZZZZ Center handed us off to ZZZZ Approach. ZZZZ is an non radar airfield. They told us to report 30 miles from the airfield and asked if we could maintain VMC. Weather was scattered to broken with some vertical development but was sufficient to maintain VMC and the field was reporting 6,000 scattered. Approach gave us a descent to 7,000 and offered the [non-precision] Approach to Runway XX. I asked for the ILS for XX. Approach asked if we could fly a visual to Runway 26 and conditions were sufficient for that and we were cleared for the visual approach. We never received guidance information on our ILS though it was programmed correctly. After an uneventful landing by the First Officer we taxied to a hard stand. Foreign Country aviation authorities inspected our certificates/medical/airworthiness certificate/insurance certificate which the company instructor was very helpful working that while I worked with operations. Operations moved the passengers indoors to a gate seating area. I worked with the Chief Pilot and Maintenance Control. They informed me that the passengers would be taken on a flight from ZZZZ-ZZZZ1-ZZZ. I made several announcements and trips to the gate holding area and informed all passengers of our plan. Most of the passengers had left carry-on bags on the aircraft, but Foreign Country security wouldn't let them return to retrieve it. I coordinated with Station Operations to have aft air stairs brought up so we could get the passenger to come back and file front to back through the airplane to retrieve their items before their inbound flight arrived and coordinated with Foreign Country security. Foreign Country security eventually agreed but only allowing 4 passengers at a time.

Narrative: 2

On climbout from ZZZZ1 we were passing FL280 when the Flight Attendant (FA) called us. He said a passenger (who is a pilot) said that it looked like fuel was coming out of our #2 engine. We monitored the fuel page. The engine fuel used totals were identical, as were the wingtip tanks. The inboard wing tanks showed an imbalance of 1000 pounds. Over the remaining 5 minutes of our climbout we watched the imbalance increase to 1400 pounds. During this time we got another call from the FA. He put us on with an Airbus Instructor that was non-revving in the back. He told us it appeared that fuel was coming from the engine exhaust. The Captain took over the aircraft and I went back to look for myself. There was nothing visible on the front of the wing or the front of the engine. I went back to row XX and could clearly see a fluid coming out of the exhaust of #2. I returned to the flight deck and told the Captain. We ran the Fuel Leak checklist and it directed us to shut down the engine. After starting a descent to FL200, we did so. We secured the engine and talked to dispatch. He directed us to either ZZZZ1 or ZZZZ. ZZZZ1 was further and the weather was reported as suboptimal. We chose ZZZZ. The Captain and I turned the aircraft back to ZZZZ and finished the descent to FL200. We briefed up the approach and I took control of the aircraft. Approach control informed us there was no radar service. We used the local MSA to ensure terrain clearance until we were clear of the clouds and could see the airfield. I maneuvered the aircraft to a 6-7 mile final and we configured the aircraft for landing. There was no glideslope which we knew, but the localizer wasn't working either. We confirmed with Tower that they could see us and that we were lined up on the correct airport. During the confusion we forgot the Before Landing Checklist, but the Spoilers Not Armed message reminded us. We accomplished the checklist and were fully stable and complete by 500 ft. AGL. Landing was normal, the Captain took the aircraft around 80 knots on the rollout. We taxied clear to a hard stand near the terminal. Most of the passengers deplaned into the terminal. We coordinated with the Foreign Country equivalent of the FAA, as well as dispatch and maintenance. After the passengers got on the rescue airplane, we closed up the plane and went to the hotel.

Synopsis

A320 flight crew reported a fuel leak from the #2 engine resulted in a fuel imbalance. The flight crew performed a successful diversion and landing.

ACN: 1843189 (784 of 2908)

Time / Day

Date : 202109
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1843189
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1842932
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Operating Aircraft X, ZZZ-ZZZ1 we pushed back from Gate X in ZZZ and were pulled to the top of the alleyway near spot 1. After starting engine 1 and disconnecting the push back tug, I smelled a slight fume smell. Wasn't immediately sure if it was a concern or not so I notified the Flight Attendants and asked if they could smell anything unusual in the cabin. They did not. Then after starting engine 2 as we taxied up to spot 1, the fume smell got stronger in the flight deck. The First Officer now stated that he could smell strong odors. I checked with the Flight Attendants again, and they now said they could start to smell something. We notified ground control from spot 1 that we had to return to the gate and we shut down engine 2. Ground control had us taxi from spot 1, Taxiway 1, Taxiway 2, and hold short of Taxiway 3 awaiting a gate assignment. We notified Ramp Control that we needed to quickly return to the gate because of fume smells. We had donned O2 masks and tried a few times to get a gate assignment but Ramp Control kept asking multiple times what the issue was after we had already told them. I then [requested priority handling] with Ground Control, and they replied for us to contact Ramp Control. I also [requested priority handling] with Ramp Control and started taxiing from Taxiway 4 to Taxiway 5 spot 2 in anticipation getting into Gate 2. Ground Control then asked if we needed [priority] equipment. We acknowledged yes send the personnel to check the outside of the aircraft. We parked at Gate 2, shutdown, and deplaned the passengers as soon as possible. The Flight attendants had left the aircraft with the passengers by the time I had given my report to Dispatch, Maintenance, and the [airport authorities]. The First Officer and I both had dull headaches, burning sore throats. I notified safety and they said to call the Duty Pilot on call and they would facilitate our being examined by a medical professional. After talking to the Duty Pilot/Chief Pilot, I was told to call a Paramedic if I was concerned about my health. Both of us stopped by the Airport station and asked if they could have the Paramedics take our vitals. After this was done, I drove the First Officer to ER where they examined us both. [The cause was] Maintenance issues with older aircraft.

Narrative: 2

After pushing back from the gate, we started both engines. There was a faint smell of exhaust/fumes that went away and returned back even stronger. The Flight Attendant said that they could smell it in the cabin, however, it was not that strong. As we were holding on the taxiway awaiting to get back into the ramp, the smell would dissipate and return again. The smell was very strong. I started coughing and having a dry throat sensation. We donned our O2 masks and [advised ATC]. The smell occurred after starting an engine.

Synopsis

Air Carrier flight crew reported a fume event during departure pushback. Priority handling was utilized to return to gate.

ACN: 1842541 (785 of 2908)

Time / Day

Date : 202109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Coalescer Bag
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1842541

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On take off I noticed a smell of dirty feet. It went away after a couple minutes. I didn't say anything because I honestly thought someone in the back took their shoes off. It wasn't until we were in the van on the way to the hotel that the Captain asked if we smelled dirty socks on landing. I told him I didn't smell it on landing, but I did smell it on takeoff. He said that he wrote it up, but no one ever said anything or called to ask if we smelled anything.

Synopsis

A320 Flight Attendant reported a fume event during initial departure. "Dirty socks" odor dissipated shortly thereafter.

ACN: 1842249 (786 of 2908)

Time / Day

Date : 202109

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Oxygen System/Crew
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1842249
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft upon arrival at ZZZ for scheduled A check. Found the crew O2 bottle shut off valve safety wired backwards. Safety wire as installed, would not resist the valve from turning towards the closed position.

Synopsis

Technician reported discovering during a scheduled A check, that the crew oxygen bottle had the valve safety wired backwards and made repairs.

ACN: 1838000 (787 of 2908)

Time / Day

Date : 202109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 54
Experience.Flight Crew.Type : 1557
ASRS Report Number.Accession Number : 1838000
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was assigned to fly 2 legs from ZZZ to ZZZ1 through ZZZ2. The aircraft had a MEL item for SEC (Spoiler and Elevator Computer) 2 indication INOP F/CTL page. I called Maintenance Control and after a conversation with an Airframe Technician, called Dispatch and refused the aircraft. I had 3 main concerns that the Airframe Technician did not address. There is a note under flight crew that states there are no landing penalties with SEC 2 INOP. Yet in the FM (Flight Manual) under F/CTL GND SPLR 5 INOP for loss of Spoiler 5 for SEC 2, a landing distance procedure applies. The Airframe Technician could not explain the difference. I donâ€™t know why these contradict. The following note states Maintenance is required to verify sidestick transducers associated with ELACs (Elevator and Aileron Computer) and operative SECs are operative. This sentence seems redundant to what the flight crew are asked to do in the procedure, which appears to ask the crew to pressurize all 3 hydraulic systems and do a flight control check for each operative ELAC and SEC. I say appears because this was far from clear in my conversation with the Airframe Technician. In fact, the verbiage appears different from the FMâ€™s description of the flight control check, which states both engines and generators must be running. So why am I doing this at the gate using the blue system on the aft overhead maintenance panel? At this point I am making educated guesses. The system training we as aircrew are given does not support doing this type of deferral. Having been through 4 full-course for the bus, the switch was identified as being for Maintenance use only. If I need to be able to do this type of deferral, I need better and more thorough system knowledge of what I am doing and why I am doing it, either from the Airframe Technician or ground school. I would propose a Pepsi blind taste test - call 3 different airframe technicians and ask them to explain this deferral. My bet is you would get 3 completely different answers. I just donâ€™t think our system ground training program supports this type of deferral. For a deferral of this type there needs to be a one paragraph-big picture of what you as the pilot are trying to accomplish. Pilot-to-pilot. If the Airframe Technician canâ€™t explain it to me adequately, and neither myself or the FO (First Officer) understands what is being conveyed, then we as a crew cannot meet the FOM requirements to have a complete understanding of the maintenance status. This leads to the philosophical debate of how much information needs to be taught in ground school, but I would submit for deferrals such as this we need much more. In addition, this day we received an email from ZZZ1 Operations stating there was a large weather system moving through, with the likelihood of diverts. Taking an aircraft that required maintenance action every leg to an alternate could be problematic.

Synopsis

A320 Captain reported confusion over whether certain components of the flight control system were deferrable and ultimately refused the aircraft.

ACN: 1836231 (788 of 2908)

Time / Day

Date : 202109
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.AGL.Single Value : 1800

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : ILS XXR
Flight Phase : Initial Approach
Flight Phase : Descent

Aircraft : 2

Reference : Y
Make Model Name : Cessna Aircraft Undifferentiated or Other Model
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Flight Phase : Cruise
Route In Use : None

Person

Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Local
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 13
ASRS Report Number.Accession Number : 1836231
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Miss Distance.Horizontal : 0
Miss Distance.Vertical : 400
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was training on LCL. There was a line of thunderstorms to east with moderate to extreme precipitation. Runways XXR/XXC/XXL in use. Aircraft X was on an ILS Runway XXR approaching ZZZZZ the final approach fix when he checked on the frequency. I issued traffic to Aircraft X at his two o'clock and 2nm, same altitude moving south toward him type unknown suggest you descend. I then called ZZZ sector and asked him if he was talking to the unidentified aircraft in the Class C and he said negative. I went back to Aircraft X and updated him on the traffic at his one o'clock and 1nm southbound type unknown and Aircraft X reported him in sight. Aircraft X went underneath the unidentified aircraft by about 400 ft. and landed without incident. I asked Aircraft X on arrival what type aircraft it was and he stated some sort of Cessna. Traffic [should be] issued sooner to Aircraft X via the approach sector.

Synopsis

TRACON Controller reported an NMAC occurred between a large transport on final approach and small aircraft which was not in communication with ATC.

ACN: 1835977 (789 of 2908)

Time / Day

Date : 202109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Widebody Transport
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Aircraft : 3

Reference : Z
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Large Transport
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Normal Brake System
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 23
Experience.Flight Crew.Type : 91
ASRS Report Number.Accession Number : 1835977
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Taxiway
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

During taxi-out for Aircraft X, [the] Flight Attendant (FA) injured her left elbow by falling against a quarter-turn in the aft galley as a result of my Captain having to suddenly stop the aircraft in order to prevent a possible left wingtip collision with an aircraft on an adjacent taxiway. During taxi out of ZZZ, we were taxiing eastbound on Taxiway XX. We were instructed by ZZZ Ground Control to yield to an aircraft that was joining Taxiway XX on Taxiway XY coming from an eastbound taxi on Taxiway XZ. There was a Company Aircraft Y that was in front of us, joining Taxiway XY from Taxiway XZ at the conjunction of taxiways XX, XZ, and XY. I assumed that the Company Aircraft Y was the aircraft we were to hold for, as ZZZ Ground did not specify an aircraft type, e.g., 787 or Airbus. Aircraft Y was far enough ahead and was not a conflict, so I continued to taxi on Taxiway XX towards Runway XXR, as our instructions were to taxi to XXR using taxiways XX and XX1. As we moved towards the conjunction of taxiways XX, XZ, and XY, I noticed an, up to this point, unseen Company Aircraft Z coming on Taxiway XZ from behind and to our left at a good pace, over my left shoulder view from the cockpit. I did not realize or expect that it was the aircraft that ZZZ Ground had instructed us to hold for, so I continued taxiing on Taxiway XX. Over the next few seconds, as we approached the conjunction of Taxiway XX and XY, I noticed the Company Aircraft Z on Taxiway XZ had caught up and was parallel with our aircraft, an A320, and was continuing towards Taxiway XY in order to join Taxiway XX in front of our aircraft. During this time, I instructed my FO (First Officer) to start the number 2 engine in order to be prepared for takeoff once reaching the runway. As such, he was heads down and did not notice the taxi conflict with the other Aircraft Z. Both aircraft were nearly at Taxiway XY when ZZZ Ground noticed the conflict and informed us that the Company Aircraft Z on our left was the actual aircraft we were supposed to yield for, vs. the earlier Aircraft Y. I noticed the Company Aircraft Z had initiated a turn towards Taxiway XY and as such, I ended up using firm braking in order to stop our aircraft in order to prevent any possible left wingtip conflict/contact, and in order to comply with ZZZ Ground instructions to hold short of Taxiway XY in order to yield to the other aircraft. During the braking maneuver, [a] FA (Flight Attendant) was facing towards the aft of the aircraft and lost her footing in the aft galley. She then fell backwards, towards the front of the aircraft, striking her left elbow on a quarter turn in the aft galley cart stowage area. Her impact with the quarter turn was hard enough to cause a painful injury to her elbow. I contacted the Purser and asked if the flight attendants were OK. I was informed that [the] FA was injured, and the nature of her injury was serious enough that we needed to return to the gate and to have medical personnel meet the aircraft. We coordinated with ZZZ Operations and returned to Gate XX, our original departure gate. We had paramedics, a Flight Operations Representative, and eventually an Flight Attendant Representative meet us. The paramedics tended to [the] FA and she was taken off the aircraft in their care. During the process, they had arranged for a replacement FA to work the flight segment from ZZZ to ZZZ1. I discussed the overall situation with [the] ZZZ Flight Operations Representative. I called the Chief Pilot and left a message with brief details of the incident. My First Officer and I made the determination that we were fit to fly the segment from ZZZ to ZZZ1. After further consult with [the] Flight Operations Representative and taking care of all necessary actions, we continued with the flight.

Synopsis

A320 Captain reported a Flight Attendant falling and getting injured during an abrupt braking to avoid a wingtip collision with another aircraft during taxi-out.

ACN: 1835561 (790 of 2908)

Time / Day

Date : 202108
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 8870
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 7380
ASRS Report Number.Accession Number : 1835561

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1151
Experience.Flight Crew.Last 90 Days : 76
Experience.Flight Crew.Type : 76
ASRS Report Number.Accession Number : 1835792

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At FL390, pressurization in cabin started to increase. Started a descent and went through the checklist. Shortly afterwards the cabin went to 15,000 feet and O2 masks were deployed. Had requested priority handling with Dispatch and ATC. Alerted FAs (Flight Attendant) of the problem, what to expect. Landed in ZZZ1, intended destination. Debriefed crew, Maintenance, Dispatch, [and] union. Everyone said it was done by textbook procedures.

Narrative: 2

AT FL390 we were cleared to descend and passing through FL360 we noticed the cabin altitude increasing. We told ATC and initiated an immediate descent. Complied with ECAMs and QRH procedures. Briefed FAs (Flight Attendant) and safely landed aircraft. Debriefed with crew, Maintenance, Dispatch, passengers, and emergency medical support staff.

Synopsis

A320 flight crew reported loss of cabin pressure control and requested priority handling and proceeded to land at destination airport.

ACN: 1831916 (791 of 2908)

Time / Day

Date : 202108
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class B : ZZZ

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1640.65
Experience.Flight Crew.Last 90 Days : 122.22
Experience.Flight Crew.Type : 1640.65
ASRS Report Number.Accession Number : 1831916
Human Factors : Workload
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 399.03
Experience.Flight Crew.Last 90 Days : 52.1
Experience.Flight Crew.Type : 101.95
ASRS Report Number.Accession Number : 1831886
Human Factors : Workload
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Crew departed late due to brake accumulator issues on previous aircraft, and a potential one on this aircraft. Ground Ops were completely normal and the FO (First Officer) had no problems deploying the flaps to 3 for takeoff. Takeoff was normal and as we were turning to an assigned heading, we reached the cleanup altitude. I called for flaps 1 and noticed that the FO had some what I felt was minor difficulty bringing the flaps up to 1. This is not unusual, a hand slips, or the lever used to move the flap handle out of the detects isn't gripped correctly for easy retraction. I thought little of it. When I called for flaps UP, the raising was again delayed, and I noticed my FO reaching around with his right hand to put both hands on the flap lever to get it up. We talked about him having trouble getting them up, and I flew the aircraft only to ensure that I did not over speed the flaps. Eventually he was able to get them up which I saw on the speed tape on the PFD and I verified on the display, and flap handle position. We discussed what had happened a bit but decided to focus on safely exiting the terminal area and getting to altitude since the aircraft was now in the safe, and proper configuration. We made a plan to contact Maintenance Control at cruise and notify them of the problem and get advice about continuing the flight. After reaching altitude, the FO told me what he had encountered and explained how difficult it was to move the lever that released the flap handle from the detent to raise the flaps. A phone patch was initiated with dispatch and Maintenance Control and we discovered that our radio 2 wasn't able to receive over long distances. We ended up substituting radio 3 for radio 2 and temporarily disabling ACARS to complete the phone patch. Maintenance Control was very helpful on the phone patch and felt like he knew what the problem was. Maintenance Control was asked if he felt the aircraft should continue to ZZZZ, and he wanted it to go to a maintenance hub. As a Captain, I agreed with that thought process and asked the Dispatcher to bring in the Ops Manager so that we could all agree on a plan of action. We could have landed the aircraft at ZZZZ if there had been a flap malfunction, but the aircraft would have been stuck there quite a while. Eventually after consideration of landing lengths, the close to overweight landing weight of the aircraft, and the potential for wet runways in ZZZ1, that the best and safest place to divert the aircraft would be ZZZ1 in case the flaps would not deploy normally. Consideration was based on the worst case, flaps up scenario. Destination was changed with ATC once the plan was agreed upon by all parties and the phone patch was terminated and ACARS restored. Maintenance Control was asked via ACARS if we should leave the flaps down after landing and they asked them to retract them as much as we could. Fortunately, the landing in ZZZ1 was uneventful. The flaps deployed normally. Upon taxi-in. The flaps were retraced to position 1, where the handle binding began to be very difficult to overcome. Upon arrival at the gate, after shutdown, I tried to move the lever that releases the flap handle from the detent and found that it was impossible to move with just one hand.

Narrative: 2

We were doing a flaps 3 takeoff from [Runway] XXL. After the acceleration altitude, and above flap retraction speed, the pilot flying called for flaps to 1. I noticed, while moving the lever from 3 to 1, that the flap lever was nowhere near as smooth and free moving as it usually is, rather, it was quite difficult to move. As we accelerated through flap 1 retraction speed, it took considerable effort to get the lever out of the detent. I mentioned it to the captain who quickly made sure to raise the nose and not over speed the flaps as I used both hands to get the flap lever compressed and moved to the up position. The flaps moved as expected, with no issues, the flight control page showed normal, and the hydraulic page showed normal. We continued the climb out of ZZZ, and when reaching a safe altitude, I took the controls and radios with ATC, while the captain initiated an incredibly detailed conversation with dispatch and Maintenance Control. After exhausting every bit of information we could give, and every insight Maintenance Control had, the decision was made with dispatch and everyone in planning that they were talking to, to divert to ZZZ1 instead of potentially having a flight control issue on landing in ZZZZ, on a shorter runway, at an international station, and then leaving the plane there. We didn't [request priority] but asked approach to slow down early to get the flaps out and make sure they extended. We had planned every contingency for having to go around and then [request priority] if they didn't, and were ready to run whichever QRH abnormal procedure would have applied based on where the flaps ended up. On the approach, the flaps came down just fine and a normal landing was made. After landing, we planned, with maintenance approval, to attempt to retract the flaps but to stop if they locked again and not force anything. The lever was much less stiff from flaps full to flaps 1 than earlier, but again locked at the detent there. We left the flaps at 1 into the gate, let ramp know to tell anyone they could, and had maintenance meet us there. Sounded like they were going to replace the entire flap lever section.

Synopsis

Flight crew reported difficulty in raising the trailing edge flaps and elected to divert instead of risking a grounded aircraft at an outstation.

ACN: 1831089 (792 of 2908)

Time / Day

Date : 202108
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Thrust Reverser Control
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1831089
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

This event occurred during preflight at the gate. The aircraft arrived at ZZZ gate XX from the maintenance hanger eight minutes before boarding time. After review of the AML, I confirmed the aircraft had a couple of MEL items including a #2 engine thrust reverser inoperative. We contacted maintenance for three additional items, including excessively high wheel brake temperatures on wheels three and four.  Those temperatures were 340 degrees C and 270 degrees C, respectively. When the mechanic arrived at the aircraft, I asked him for the reason the brakes were so hot after a tow or taxi delivery from the hanger. He inspected the brakes and found them to be "very warm."  He also ran a few diagnostic tests. Unfortunately the mechanic could not give an adequate explanation for the high wheel brake temperatures, nor could he verify the proper functioning of the wheel brakes on the right main gear. I refused the aircraft on the basis of safety-of-flight because I felt the aircraft was unsafe to operate with an unknown wheel brake issue coupled with an inoperative thrust reverser. The primary causal factor for refusing the aircraft was the unknown reason for the hot brakes. This issue was not resolved to my satisfaction. The risk of operating the aircraft with both a brake issue and inoperative reverser was too high to warrant continuation of the flight. Maintenance personnel should resolve obvious aircraft issues before releasing the aircraft to the flight crew.

Synopsis

A320 Captain reported refusing the aircraft due to a deferred thrust reverser and high brake temperatures that maintenance could not explain.

ACN: 1831057 (793 of 2908)

Time / Day

Date : 202108
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5323
Experience.Flight Crew.Last 90 Days : 55
Experience.Flight Crew.Type : 1234
ASRS Report Number.Accession Number : 1831057
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When we arrived in ZZZ to start our trip we meet with the inbound crew to discuss the inbound leg and aircraft. There was no mention from them about any odors or problems with the aircraft. When we boarded the aircraft we were over come by a strong smell throughout the plane. Looking back it was strange the previous crew didn't mention this. It was similar to a strong carpet cleaner smell or an odor like when you clean an oven. There was no indication of fire or smoke much less any indications of a malfunction. We discussed the issue with the crew and found no reasons not to continue safely. As we boarded the aircraft several passengers started to complain especially in the rear cabin about the odor. We discussed what the cause of the smell could be and determined that it was not a safety of flight issue since we could not isolate any problems. All systems operated normally. We agreed to continue on the short flight to ZZZ1 hoping that at altitude the fresh air would dispute any smell. We felt that since there was smoke all around the area from west coast wild fires that could possibly be the cause. At cruise the smell seemed to disappear in the cockpit but as we got closer to our destination the FA'S informed us that it remained in the cabin. There was no smoke or any indications of a malfunction. They then stated that many passengers complained that they were starting to get headaches and feel bad. We asked if there any immediate concerns and were informed there were none. Inbound we decided to inform maintenance control that there was an odor which seemed to be coming from the gasper vents via an MRM entry so that on arrival they could trouble shoot and see if they could remove it. After landing we started the APU and received a low oil level indication. Not sure if this contributed to the problem or not as we accepted the aircraft in ZZZ with the APU operating and had no indications of any problems at that time. Arriving at the gate we noticed that the smell in the cockpit remained but the cabin was stronger. We had an hour on the ground before our next flight so felt that the problem could be resolved. After block in we meet with maintenance and he noticed the smell as well. His feeling is it smelled like burning oil. We had no indications at any time of an overheat or fire on any system. I then received a call from Name our Chief Pilot to contact Name 1 in Maintenance Control for some questions. I was asked generic questions about the entire flight to help trouble shoot the problem. As I stated everything went great expect for the strong smell. They removed the aircraft from service and we continued our trip with another aircraft. The entire crew handled the situation professionally and with merit. I applaud how we worked together to assist passengers needs and answer any questions or concerns they may have had. The flight was concluded safely and professional with our [company] principles.

Synopsis

A320 Captain reported a fumes event of varying intensity from preflight to arrival at destination where maintenance removed the aircraft from service.

ACN: 1827797 (794 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 191
Experience.Flight Crew.Type : 5741
ASRS Report Number.Accession Number : 1827797

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 108
Experience.Flight Crew.Type : 1305
ASRS Report Number.Accession Number : 1827800

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight planning, preflight, ground operations, and takeoff were accomplished per SOP and were uneventful. Captain was pilot flying. Passing 18000 STD, altimeter was selected and AP 1 was selected. At XA:17, we got a master caution light, with HYD G RSVR LO LVL, HYD G ENG 1 PUMP LO PR, and HYD G SYS LO PR displayed on the upper ECAM. After confirming no Green Hydraulic QTY, I asked the FO (First Officer) to request to stop climb and return to field. Captain continued to fly and handle radios. FO accomplished the ECAM procedures, turning off the PTU and Green Eng 1 Pump. QRH was referenced after ECAMs completed, screens normal. We requested priority handling with ATC. Advised Dispatch through ACARS that we had lost all Green Hydraulic QTY, were returning to ZZZ, and would require a tow off runway due to loss of nosewheel steering. Purser was advised of the malfunction, that we were returning to field in 20 minutes, and a cabin prep was not required. A visual approach to XXL was briefed after runway assignment and the descent checklist was accomplished. Aircraft was configured early due to slow flaps/slats and alternate gravity gear extension. Approach and landing were accomplished per SOP and were uneventful. Aircraft was stopped straight ahead on centerline. Engines shut down with timing after starting APU. Aircraft was inspected by Fire Response Team, confirming gear doors down and fluid [was] leaking from under plane behind gear. Super tug arrived and towed us clear of runway to Gate XXX.

Narrative: 2

On climbout passing approximately 21,000 feet, we got an ECAM indicating loss of the green system, LO G Press, and another green system ECAM but I cannot recall it now. We ran the ECAM action items which had us turn off the PTU and Eng 1 G pump. As soon as the ECAM action items were complete, we communicated with ATC, advising them we needed to stop our climb, and requested priority handling and requested a return to ZZZ. The CA (Captain) and I split up the flying and comm duties, he advised the FA's (Flight Attendant) of our situation, and sent a message to Dispatch. I talked to ATC and flew the aircraft for a brief time while he handled the message to Dispatch. We got the expanded checklist and discussed the effects of [a] loss of the green system in terms of manual cranking of the gear and needing to stop on the runway due to loss of steering, which the CA sent an ACARs to the station to start working on a tow for us to minimize us sitting on the runway. I also advised ATC to let ZZZ know we would be a full-stop on the runway and would need to be towed. We were given priority handling back to ZZZ and we briefed the approach and threats of a go-around since the gear could not be retracted once it was hand-cranked down. The CA flew the approach XXL and the landing and stopping at the end of the runway was uneventful. We dropped the landing gear pins to the tug team and got towed back to the gate. We debriefed once we were off the aircraft and the passengers disembarked. The CA was cool, calm, and collected and flew a textbook approach. We worked well together as a team; I can't think of much that we could have done to improve on how the situation was handled.

Synopsis

First Officer reported loss of all green hydraulic system fluid caused an Air Turn Back and precautionary landing.

ACN: 1827593 (795 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1827593
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1826700
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft RA
Detector.Automation : Air Traffic Control
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On the final phase of the Visual Approach, backed up by the ILS Runway XY to ZZZ, with the intention to capture the glide path from above, the Auto Pilot (AP) and Auto Throttles were disengaged. Prevailing conditions were Good Visibility and varying Tail Winds of up to nine (9) Knots. During the final portion of the descent to touchdown, with a visual transition to the PAPI reference already established (PAPI were showing 3 RED-1 WHITE), the Aircraft descended below the referenced visual glide path, with the tendency to fly over the Runway XY overrun area at an altitude (AGL) of less than 50 ft. The moment the Captain, having the Pilot Monitoring (PM) duties, mentioned his estimation for being low on the glide path, the aircraft was overflying a point short of the RWY by about 500 feet. He consecutively called for a Go Around, which was immediately executed by the PF (Pilot Flying). The Flight landed after a RADAR pattern on the same runway uneventfully. [The cause was] Human Factors / Pilot Flying error on visually estimating the projected Glide Slope to Runway XY, with the current Tail Wind conditions. Root cause is the initially higher angle of descent / rate of descent to capture the glide slope from above, that ended up to an over correction - reverse situation of flying finally lower than the projected/correct slope. [I suggest] earlier Aircraft configuration for the Final Approach Phase and timely request from the ATC for lower altitudes / earlier descent to the recommended altitudes that lead to a better projected sequence of flying tasks.

Narrative: 2

FO was flying pilot. Performed visual approach and FO had a hard time trying to catch the glide slope from above because he was trying to slow down and put flaps down while on OPEN DESC. FO called for "the bird", which caught me off guard because it is the first time anyone has asked for the flight path vector while trying to intercept the ILS and while disconnecting the AP. He not only disconnected AP but also eventually the auto thrust without previously briefing it. I told him to never disconnect the auto thrust again while trying to save a visual approach. Finally he was established on the ILS. Below 500 feet he said he was going to follow the PAPI. I glanced down at the VSI to make sure we were stable and when I looked backed up we had 4 red on the PAPIs and at the same time we received a GS GPWS warning. Became unstable. I loudly and sternly called "unstable, go-around." Performed go-around as RA called 50 feet. FO performed a soft go-around. I overrode his decision after he brought back TL (Thrust Lever) to CLB and I pushed TL to TOGA since we were in ZZZ (high airport elevation) with high OAT. I honestly could not tell you the altitude when the FO brought back the TL to CLB. I wanted to make sure we were safe since the go-around was performed at 50 feet from an unsafe position. At 50 feet RA, aircraft was not anywhere near the approach end of the runway. Came back for an uneventful landing. FO descended below glide slope and PAPI showed 4 red. Soft Go-around procedure should be changed or at least highlight the need to take into account airport elevation and OAT when deciding to make a soft go-around from a low altitude with an obvious performance deterioration. Soft Go-around procedure should be changed or at least highlight the need to take into account airport elevation and OAT when deciding to make a soft go-around from a low altitude with an obvious performance deterioration.

Synopsis

Flight crew reported that the First Officer made errors during final approach to landing and the Captain elected to call for a go around.

ACN: 1825783 (796 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 3004
Experience.Flight Crew.Last 90 Days : 121
Experience.Flight Crew.Type : 3004
ASRS Report Number.Accession Number : 1825783

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2843
Experience.Flight Crew.Last 90 Days : 104
Experience.Flight Crew.Type : 2843
ASRS Report Number.Accession Number : 1825788

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We commenced our takeoff normally. At around 95 kts, we got an ECAM for Y RSVR Lo Lvl and Y RSVR Lo Air Pr. The Captain initiated the abort with a max speed of approximately 100 kts. All callouts and notifications were made and we were able to taxi clear of the runway and stop. ATC notified us that we had trailed smoke so we asked him to roll the ARFF (Aircraft Rescue and Firefighting) equipment. We ran the rejected takeoff checklist and then ran the ECAMs of which there were several all relating to the hydraulic issue. ARFF indicated that we had dumped a lot of hydraulic fluid. We were towed to the gate for repairs and offloaded the passengers and bags.

Narrative: 2

Received a Master Caution light and ECAM during takeoff roll on Runway XXR at ZZZ and accomplished a rejected takeoff. The Master Caution and ECAM occurred at approximately 95 kts and Reject initiated between 95-105 kts. During the reject, several ECAMS displayed indicating loss of the Yellow Hydraulic system reservoir and Yellow system air pressure. At slow speed and with normal brakes and nosewheel steering available, I opted to take high speed Taxiway XX to safely clear the runway and bring aircraft to a full stop while the First Officer coordinated with Tower controller to roll the AARF (Aircraft Rescue and Firefighting) equipment as Tower stated they saw what appeared to be smoke trailing from the number two engine. While awaiting AARF response, we ran the Rejected Takeoff QRC and handled the ECAMS, started the APU and communicated with the passengers [and] inflight crew. After AARF arrival and initial inspection, we shutdown the number 2 engine with AARF on scene, then chocked the gear and shutdown the number 1 engine which had been used to keep normal brakes available. We coordinated with Operations and Ground Control for uneventful tow back to the gate for repair.

Synopsis

A320 Flight Crew reported a Rejected Take Off when the Yellow Hydraulic system showed low quantity and had to be towed back to the gate.

ACN: 1825156 (797 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1825156

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Dirty sock odor descending into ZZZ. [Suggest] aircraft filtered bleed air.

Synopsis

Air Carrier Captain reported a fume event of "dirty socks odor" during descent to destination.

ACN: 1823945 (798 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Improperly Operated

Person

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 6707
Experience.Flight Crew.Last 90 Days : 116
Experience.Flight Crew.Type : 5884
ASRS Report Number.Accession Number : 1823945
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Software and Automation
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Inadvertent switching off of IR1 mode selector which deactivated that associated ADIRU. ADR and IR data not available. This occurred during flight one of day two of an initial Captain upgrade. The trainee came off the B777. He has been with our company for many years. The three legs of day one was extraordinarily busy. It was immediately clear to me during those three legs he had studied hard during training and had mastered much of the material. On the first two legs of day one I had a LCA (Line Check Airman) in-training watching me conduct the line check as part of that LCA's upgrade training. I commented to the new LCA not all trainees will be so well prepared as this one. This flight was leg four of the IOE and the first flight of day two. Our transportation to the airport was 15 minutes late and there was a detour so we arrived at the aircraft about 30 minutes prior to departure. Normally, I introduce myself to all the FAs (Flight Attendant). However, because we were on IOE, [we] were somewhat late to the aircraft and on Release 6. I only introduced myself to the FM. I felt I did not have time go to the back to introduce myself to the FAs assigned to the rear of the aircraft. We had been in the seat for approximately 3 hours. The upgrade Captain requested I ask the FM to prepare the meals and that we would then take a restroom break when the meals were ready. I did so. The FM called us about 25 minutes later to state the meals were ready. The upgrade Captain offered me the opportunity to use the Lav first. I declined and said he should go first and I would then go. The door chime rang and then I heard a knock. That knock is not standard. However, it occurs occasionally with legacy Company 2 flight attendants. We put on our masks. I saw the upgrade Captain move to get out of his seat, stand up, and let the FA in while he then left the cockpit. I noticed nothing odd about his movement out of the seat and to the rear of the cockpit. I saw out of the corner of my eye the FA was standing near the rear of the cockpit just behind and to the right of the Captainâ€™s seat, a normal position for a relief FA. My concentration then focused on something else. I think it was because we were getting ready to enter a large area of convective activity. I might have been looking at WSI and comparing that to what I was seeing on the weather radar. Why my gaze was generally focused forward and right I am not sure, but it was. I planned to brief the relief FA about this weather and its possible effect on their service in a few minutes. It was at that point, maybe 30-45 seconds after the upgrade Captain had left the cockpit, that for lack of a better description, "all hell broke loose." The cockpit went from a completely benign environment to one of confusion. The startle effect was in full force. The first warning was a continuous repetitive chime and the warning lights flashing. I immediately looked at the E/WD (Engine/Warning Display) and noted about 4-5 ECAMs listed, primary secondary and independent failures in both red and amber. I silenced the CRC (Continuous Repetitive Chime) by pushing [the] master warning light. That silenced the CRC. However, there still was an aural alert going off that I could not silence, which created a fair bit of consternation on my part. I look at my PFD and saw the FMAs were wrong. I then saw the AP (Autopilot) was off and that there was nothing where I expected to see the AT (Autothrottle) condition. I noted the aircraft climbing slightly and corrected that. I knew the AP disengaged for some reason but didn't know why. I thought about engaging AP 2 but then thought that may create new problems, so I elected to hand-fly until the ECAMs were sorted out. I saw we were no longer in NAV but again, wasn't sure why. I did see that the aircraft didn't diverge much from the "green line" so lateral stick inputs weren't necessary. At this point ATC called to ask us something about recycling the transponder. We were on speaker at that time; I responded we had a problem and to standby. I then noted the airspeed increasing and pulled the TLs (Thrust Lever) back; I didn't want to add an over-speed to our problems. This silenced another aural sound. I do not recall seeing a Thrust Locked ECAM or FMA which I thought would have been the source of the alert I couldn't silence. I glanced at the Captain PFD (Primary Flight Display) and ND (Navigation Display) and saw they didn't look right. The aircraft was reasonably stabilized now but, in my mind, because the ECAMs were displayed, the situation wasn't. I then told the FA to recall the trainee Captain to the cockpit. He opened the door, looked and then closed it, and said he was in the bathroom. I told the FA I knew that and I wanted him back in the cockpit now. He then had the FM knock on the Lav door. The trainee Captain returned to the cockpit and the relief FA left. The trainee Captain was incredulous when he looked at the E/WD. I told him I was hand-flying, had the aircraft and radios, and that he was to handle the ECAMs. It was at this point I noted we were about 20 knot slow, well within flying speed but slow for planned speed. I had overcompensated for my worry of overspending the aircraft. I pushed the power back up. Airspeed had fallen out of my cross-check. The trainee Captain methodically and correctly went through the ECAMs. As I recall the first several ECAMs had no action items, one ECAM had us switch the air data switch to three. At some point during this, ATC stated they now had data on us and asked if we needed assistance. We told him we were completing a procedure and to standby. Once the status messages were read and the ECAM procedure completed, I reached up to engage AP two. It did so. I armed the AT and then moved the TLs to the climb detent to engage it. Our ATC clearance was to proceed direct to a fix so we reinserted that. I don't recall having to pull or push to capture our assigned altitude but I can't be sure. It was now maybe 4-5 minutes after the initial event where I could take a breath. I recalled the ECAM and saw the ADR 1 fault. I then looked over to the ADIRS panel and saw IR mode selector one was in the OFF position. It, of course, was on for the flight up until some point in [the] last few minutes. I asked the trainee Captain if he recalls hitting the switch with his head or back as he left his seat. He said no. I then thought perhaps the switch was defective. This aircraft came to us as a through flight. We did not have to turn them on so we had no knowledge of how that switch felt by turning it on. We again discussed it and he stated he didn't recall even touching the overhead panel while leaving the seat. Because I didn't introduce myself to the FAs at the rear of the aircraft prior to the flight, I wasn't sure which FA came to the cockpit during our break. I asked the FM to ask the FA who came to the cockpit to call us on the interphone. That FA did and I asked him if he recalled touching anything on the overhead panel; he stated he did not. What I found unusual was that during the event, the FA seemed very nonplussed by the cacophony of sounds in the cockpit. I would have expected a FA to at least ask what was going on, but maybe he was somewhat shocked by what he saw and heard during his short time in the cockpit. The trainee Captain and I discussed that I would write up the ECAM and then conference Dispatch and Technical Support Maintenance Control to provide further details of what we thought may have occurred. I wanted the switch looked at but also didn't want Maintenance to search for a problem which might not exist. I ACARSâ€™d Dispatch with the request and he set up the conference call. The communications with Technical Support Maintenance Control were reasonably good and he captured exactly what I wanted him to know and he appended it to our write-up. Since the Captainâ€™s displays were significantly degraded, we switched PF/PM roles. From that point we continually deviated for weather, were put in holding, and changed our alternate three times. We over-burned by almost 5500 lb on this flight, more than on any flight I have been on in [an] Airbus. It was an understatement to say this was a busy flight. I don't know how this happened. The final flight of the day was a termination. As the trainee ran that checklist and turned off the IRUs, he stated he couldn't believe how he accidentally managed to switch that style of switch â€“ pull and turn â€“ off by getting out of the cockpit. I am disappointed in myself for not more thoroughly reviewing the flight manual for ADR 1 fault after things had settled down. We did not have a GPWS TERR FAULT light and no ECAM direction to switch it off so it remained on. We discussed that threat in that we were now the aircraftâ€™s GPWS system and to be cognizant of the terrain. We also discussed changing the transponder to position 2 but did not do so because there was no ECAM direction to [do] so and ATC noted no anomalies in our reporting. Also, once I saw the IR mode selector switch in the off position, I should have considered reviewing the NAV IR fault non-normal even though we didn't have that ECAM. I did not do a word search in the flight manual for ADR. Had I done so, I would have seen that there may be RVSM limitations associated with its failure and should have gone to the FOM for further information. I recall many years ago, our Line Manager at the time, wrote up in his weekly update a similar event. As I recall he wrote the trainee somehow managed to hook the window shade into that same switch and turned it off. That of course didn't happen here.

Synopsis

A320 Captain reported that a component was inadvertently switched off after the other pilot left the cockpit for a lavatory break and a multitude of warnings and ECAM messages went off as a result.

ACN: 1823520 (799 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1823520

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1823534

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On final approach, we lost the green hydraulic system. We entered holding to complete the non-normal procedure and get fully configured. We also requested a longer runway and emergency equipment. The landing was made to a full stop because of loss of normal brakes and nose wheel steering. We were then towed to the gate.

Narrative: 2

We experienced a mechanical failure (HYD G SYS LO PRESS) upon our descent into ZZZ on the ZZZZZ1 Arrival beginning approximately outside ZZZ waypoint. I was Pilot Flying, the Captain followed appropriate ECAM actions and checklists following company procedures. We advised ATC based on this ECAM considering our location and situational aspects. We elected to hold over the final approach fix for GPS approach to Runway XX while we completed all of our necessary actions in the interest of safety. Upon completion of these duties, we exited the hold, completed approach and landed successfully coming to full stop on the runway. Based on the mechanical condition of the aircraft as result of systems lost from hydraulic system, we coordinated with company operations to tow us in due to loss of nose wheel steering and deplaned normally. Mechanical malfunction (HYD G low pressure) ECAM.

Synopsis

A320 flight crew reported the loss of the green hydraulic system resulted in a precautionary landing and a tow to the gate.

ACN: 1820446 (800 of 2908)

Time / Day

Date : 202107
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors

Component

Aircraft Component : Compressor
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1820446

Person : 2

Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1820276

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After a 5 hour maintenance delay to change a flat tire, we departed for ZZZ1. We executed a normal, flex, CONF1 takeoff from [Runway] XXL in VFR weather with light winds and a light aircraft. I was PF (Pilot Flying) and was hand-flying the departure. At approximately 1,000 AGL I heard several loud bangs from the right side of the aircraft. My first thought was bird strike but the CA (Captain) called out "engine failure." I called out "my aircraft" and we determined there was a problem with the #2 engine. I don't recall any ECAMs but I do recall seeing the #2 EGT was high. I was continuing to hand-fly and CA suggested we reduce thrust to idle on the #2 engine. I agreed and we reduced the thrust and the surging and vibration ceased and the engine parameters went into the green. CA suggested an immediate return to the departure field and [advised] ATC and coordinated a downwind turn and level-off. I engaged the autopilot and prepared for a return to runway XXL with an ILS approach. CA set the FMGC up for the approach and I flew the downwind and base. CA advised the FAs (Flight Attendants) to prepare for a precautionary landing and gave a short PA as well. ATC vectored us to the ILS and cleared us for a visual approach once I had the field in sight. I executed a normal landing on XXL and we taxied clear on taxiway 1 and had the fire department inspect the aircraft. CA made several PAs instructing the passengers to remain seated and that the situation was under control. I noticed the #2 EGT was increasing and approaching the upper limit and we shut the engine down as the fire department was inspecting the engine. CA coordinated with operations a return gate and once the fire department was done and deemed the aircraft safe to taxi we returned to gate. The passengers deplaned normally. CA then called Dispatch to advise them of our return. We both determined the safest course of action was to get the aircraft back on the ground as soon as possible. This was made easier with good weather, no air traffic, and being below max. landing weight. We were only in the air ten minutes and this didn't give us time to do much troubleshooting or communication with anyone except ATC and the flight attendants. I was 100% busy flying the aircraft the entire time, but due to excellent ATC and constant communication with the Captain/PM, I felt in the yellow most of the time and in the green for the final approach and landing. I only felt I was 'in the red' for the few seconds between when the engine began surging until the CA said "engine failure." That statement immediately told me what was happening and I snapped into our trained procedures. We continued to discuss the plan and agreed that a quick return to the field was best throughout the short flight. Since the engine seemed stable at idle and we had no ECAMs we decided not to complicate things by troubleshooting but rather execute a quick precautionary landing. I feel our combined experience in the Airbus as well as constant precise communication and exchange of ideas led to the successful outcome. We backed each other up, trapped errors, and executed our plan efficiently and effectively. I don't see this event being preventable, but I think our training and emphasis on SOP and communication made for a good outcome.

Narrative: 2

On takeoff, at approximately 1,000 feet AGL #2 engine compressor stalling with high EGT. We brought the thrust lever to idle and stalling stopped. We [notified] ATC and returned to field. Firefighters inspected the airplane and it appeared normal. Taxied to the gate.

Synopsis

Flight Crew reported compressor stall required an air turn back and a precautionary landing.