ACN: 1719272 (901 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway XXR
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 2219
Experience.Flight Crew.Last 90 Days : 177
Experience.Flight Crew.Type : 1295
ASRS Report Number.Accession Number : 1719272

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1678.55
Experience.Flight Crew.Last 90 Days : 183.03
Experience.Flight Crew.Type : 1678.55
ASRS Report Number.Accession Number : 1719275
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While acting as Pilot Monitoring, [on] ILS XXR, all systems were normal, and approach was stable and in the landing configuration (1,500 AGL), we received "TOO LOW GEAR" at approximately 150 feet AGL. Prior to the GPWS alert, all indications were normal with the before landing checklist completed before 1,000 AGL; both landing gear lights and wheel page on lower ECAM indicated that landing gear was down and locked and GPWS did not annunciate descending through 750 AGL.

We got the "TOO LOW GEAR" warning and executed a go-around at that point. I looked at the lower ECAM and both LGCIU diamonds indicated the landing gear was down, but the normally "green" landing gear lights were not illuminated. During the go-around, the landing gear retracted normally and all doors closed. During the climbout, we received an "LGCIU 1" ECAM. The FO flew the airplane and assumed the radios once the go-around was complete and we were in clean configuration 10,000 MSL, 210 KIAS, heading 245 degrees. I ran the ECAM which directed me to turn the GPWS SYS switch the the "OFF" POSITION.

We decided to lower the gear (With LGCIU #2) at 12 miles to give us additional time just in case we had additional issues. When I placed the landing gear down, nothing happened; the landing gear did not extend. Due to our low fuel (approximately 4,000 LBS) status we elected to manually extend the landing gear. The landing gear extended without any lights on the landing gear panel; the lower ECAM indicated 3 green diamonds for LGCIU#2, amber for LGCIU#1, landing gear doors remained extended and "Steering" was displayed in amber. 

The landing was uneventful and we slowly taxied to the gate area using differential braking/thrust we elected to be towed into the gate for the last 100 feet.

Narrative: 2

On final approach to Runway XXL at ZZZ we were configured and normal checklists complete according to normal Airbus visual approach profiles. At FAF as Pilot Flying I disengaged the Autopilot and Auto-throttles and hand flew the visual approach. At approximately 200' AGL we received the "To Low, Gear" Aural Warning. We both noticed that there was no indication, red or green for all three, on the landing gear panel. Even though the gear had initially indicated down and we could hear the gear down we felt a go around was best course of action. CA XX_11 called "Go Around". I initiated the Go around and gear was raised and seemed to work normally. We then received the LGCIU 1 Fault ECAM. At approximately 1000' AGL I reengaged the Autopilot and Auto throttles. CA XX_11 announced I would remain Pilot Flying, also work ATC communications, and he would work the ECAM and checklists. We advised ATC the reason for the go around was a maintenance item. We would need to work a checklist but could go ahead and be vectored for another approach. Checklist for the LGCIU 1 was short and we anticipated the gear would come down normally and we would use the indications on the Lower ECAM Wheel page. We also discussed the loss of Engine 1 thrust reverser and that I would use reverser 2 but only at idle thrust. Also discussed to lower the gear early in case of any further problems. Finals for runway XXL were then 25 to 30 miles in length. Considering fuel levels we planned to lower the gear on a 10 to 15 mile final. I remained Pilot Flying and CA XX_11 resumed ATC communications. When I called for Gear Down and Gear handle was lowered by CA XX_11 nothing happened. There was no change in the indications on the Lower ECAM Wheel page and no normal sound of the gear doors opening and gear lowering. I inquired if there was any mention of a gravity gear extension in the checklist. CA XX_11 said there was not. As Pilot flying I continued to prioritize my attention to flying the approach as CA XX_11 looked at the QRH checklist again, checked system pages and made sure the gear handle was in the proper position. CA XX_11 began the gravity extension and we could hear the gear doors open and verified the gear was down on the Lower ECAM Wheel Page. Touchdown was normal and smooth. Aircraft control was good even with the loss of nose wheel steering and reverser 2 was used at idle thrust. CA XX_11 took command of the aircraft prior to leaving runway center line and was able to steer the aircraft off using differential braking. CA XX_11 continued to taxi the aircraft south on taxiway F and asked if I was okay with him continuing to the ramp area. I responded I felt safe. Taxiway conditions were dry, there was not a strong crosswind, and he seemed to have no trouble controlling the aircraft using differential braking. Ramp crew was notified that we would be able to taxi to the gate area but did want to be towed into the gate.

Synopsis

A320 Crew reported that while on final approach with landing gear extended, a gear warning prompted a go-around. The crew manually extended the gear and landed.

ACN: 1718921 (902 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 28000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZDV

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZDV

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 227
Experience.Flight Crew.Type : 3018
ASRS Report Number.Accession Number : 1718921
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Initial climb out of DEN. Seat belt sign on, flight attendants up. Climbing through approximately FL280 suspect we encountered wake turbulence from aircraft 10-15 nm ahead and 4,400 feet. above us (757). Auto flight was engaged. Abrupt 25 degree roll left and increasing. Disconnected autopilot and stabilized the aircraft. Offset 2 nm right of track and reported encounter to ATC. Checked with Purser, who reported that everyone was ok. Found out an hour later just prior to top of descent that one flight attendant had fallen during the turbulence encounter and had some side and back pain.

Synopsis

A320 Captain reported that a wake turbulence encounter resulted in an injury to a flight attendant.

ACN: 1718880 (903 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZKC.ARTCC
State Reference : KS
Altitude.MSL.Single Value : 37000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZKC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZKC

Person

Reference : 1
Location Of Person.Facility : ZKC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 1.0
ASRS Report Number.Accession Number : 1718880
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

One sector did a flash through from another sector for Aircraft X at 37,000 feet coming from the north. They also had Aircraft Y coming from the west at 37,000 feet. The conflict point was 10 miles inside my airspace. I accepted the handoff for Aircraft X and he was switched to my frequency. Then when they started flashing Aircraft Y at me, I tried to call to tell them to descend Aircraft Y. They were 4 minutes from the confliction point at this time. The conflict alert started flashing and they didn't respond to several calls on the shout line. Once they were 3 minutes from the conflict and with no response or action from the sector I descended Aircraft X to 36,000 feet despite that aircraft being still inside the other sector's airspace. They also violated my airspace because I hadn't taken the handoff on Aircraft Y until he was 2 miles from my airspace. Still no call from them to coordinate.

ZDV continues to give ZKC aircraft that are on a collision course without doing anything about it. This has been brought up the chain several times with numerous instances, this being one of the worst. The controllers at ZDV clearly either aren't trained well enough or just don't care. The fact the aircraft had conflict alert going off while both were in their airspace and nothing was done nor was any communication given seem to demonstrate a lack of concern and that is a major safety hazard.

Synopsis

ZKC Center Controller reported the adjacent Center sector handed them off two aircraft on conflicting headings at the same altitude.

ACN: 1718740 (904 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BDL.Airport
State Reference : CT

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BDL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Initial Approach
Airspace.Class C : BDL

Aircraft : 2

Reference : Y
Make Model Name : UAV - Unpiloted Aerial Vehicle
Airspace.Class C : BDL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1718740

Events

Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Immediately after we broke out of the clouds on approach, we observed what looked like a drone to right of the Runway 24 at BDL. It had purple and red led lights. Pointed it out to Tower and to aircraft that followed us on approach.

Synopsis

A320 Captain reported sighting a drone while on approach to BDL.

ACN: 1717758 (905 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Work Environment Factor : Poor Lighting
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 2369
Experience.Flight Crew.Last 90 Days : 141
Experience.Flight Crew.Type : 2369
ASRS Report Number.Accession Number : 1717758
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I coordinated with the Ramp Agent for an early push off the gate, per his request, to ensure their understaffed ramp team could be ready for a subsequent arrival. During pushback, and after clearance to start engines by the Tug Operator, I advised the driver via interphone that we would be holding position on the ramp due to a controlled wheels up time. I queried to ensure there was room for the inbound aircraft to get to their gate. Shortly thereafter I felt the aircraft lurch and I instinctively applied brakes and queried the tug driver if they had a problem with the tug. The driver said no and that they were already disconnected from the aircraft. At no time after the "Cleared to start engines" call did the push crew advise in accordance with SOP to "Set brakes.", nor give the standard tow bar phraseology "Aircraft disconnected, by-pass pin removed." After ensuring no one was in harm's way, no injury or damage occurred and clearing up the confusion with the tug driver regarding the lack of required phraseology, the "Disconnect headset" command was given. As the tug driver backed away, one push crew member showed the tow bar disconnect pin and flag, and then the second team member gave the aircraft clear signal, but then turned and walked away towards their next arrival gate without waiting or noting the acknowledging light flash. The non-standard interphone and push procedures, possibly exacerbated by understaffing of this sub-contracted team, led to an inadvertent aircraft movement.

Synopsis

A320 Captain reported non standard procedures and communication caused the aircraft to roll away after towbar disconnect at push back completion.

ACN: 1717747 (906 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1418
Experience.Flight Crew.Last 90 Days : 127
Experience.Flight Crew.Type : 569
ASRS Report Number.Accession Number : 1717747
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We pushed back from ZZZ gate XXX at XA:04Z. Clearance is not required. We had a wheels up time of XA:31Z. During the push I coordinated with Ground Control where they wanted us to park the aircraft after the pushback. Ground Control stated we could keep the aircraft on the uncontrolled ramp until the time came to taxi for departure. The Captain was communicating with the pushback ground crew. The aircraft was stopped, I did not hear "set brakes" or "parking brake set, pressure normal" by the ground crew or captain when the aircraft moved forward and then suddenly stopped. The Captain asked the ground crew what just happened. The ground crew responded he thought the parking brake was set at the time the aircraft moved forward. The Captain stated the brakes were not set and that he never stated they were set and reminded the ground crew of the correct phraseology. The Captain also asked if everyone was okay on the ground. The ground crew stated everyone was alright on the ground. We finished the disconnect process and continued with the flight.

Synopsis

A320 First Officer reported a communication breakdown with the ground crew, that resulted in the aircraft rolling after the tow bar was disconnected.

ACN: 1717258 (907 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Galley
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1717258
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After pushing back from the jetway the aft galley filled with Jet fuel odor. We contacted the pilot and returned to the gate. I had pain in my sinus going up behind my left eye and pain in the middle if my forehead. Followed by a metal taste in my mouth. 

I stepped off the aircraft while passengers were deplaning for fresh air due to jet fuel exposure.

Synopsis

A320 Flight Attendant reported fuel fumes in the cabin upon pushback resulting in a return to the gate.

ACN: 1716834 (908 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1716834

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We started both engines after push back. After second engine start we were cleared to taxi. Approximately 2-3 minutes into our taxi, the number 2 and 3 FAs called the flight deck to tell us there was a strong odor of "gas". They said it seemed to be dissipating however one of the FAs felt dizzy and was getting a headache. We requested to pull over and evaluate the situation. I spoke to the FAs again and I asked them if it had gotten better and they said yes, but that they said one of them still felt dizzy. I spoke with the [Chief Pilot] and discussed my concerns continuing the flight with an FA that is complaining of a headache and being dizzy coupled with 1/8 mile visibility and a takeoff alternate. He asked if I would call the FA back and confirm he was no "fit for duty", which I did. The FA confirmed he was not fit so we returned to the gate, deplaned, and FA 2 and 3 were taken to the hospital for evaluation.

Synopsis

A320 Captain reported a ground gate return due to Flight Attendant notification of physiological issues experienced from cabin fumes. Two flight attendants were replaced and treated at local hospital.

ACN: 1716103 (909 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1716103

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While operating flight I was informed by my FO (First Officer) that he was not feeling well after our conversation about an extensive dental procedure I had a few months back. FO looked a bit pale and certainly uncomfortable and said he needed to use the restroom. After summoning the A FA (Flight Attendant) to the cockpit, I heard a commotion in the front galley as the FO was exiting the flight deck. After looking back, I saw the legs of the FO on the ground. I immediately told the A FA to close the door and have a seat. At this point my position was around 300 miles north east of ZZZ at FL360. I referenced my manual on the procedures for incapacitated crew members and determined our next course of action. After about 5 minutes of communicating with the FAs on the condition of the FO they informed me he was up, doing well, and had his color back to normal. I spoke with the FO over the inter-phone system and he explained he was feeling much better and believed he was ok to continue. I discussed with the A flight attendant about staying upfront in the cockpit while the FO got situated so she could help if anymore issues came up. After talking with the FO and FA as well as a PAX (Passenger) that was a nurse who treated him, we determined he was back to normal and able to continue the flight as planned. I informed the FO I would be the pilot flying for the remainder of the flight and we discussed how to proceed. I determined a diversion to another airport was not in the best interest of safety. I considered policies/weather/distance/safety of my PAX's and crew and determined the best course of action was to not try and make a complex mountainous terrain diversion. ZZZ was clear with light winds and all our pre approach duties were accomplished for ZZZ. The flight then continued without further incidence. Once on the ground I told the FO to call the on-call chief pilot to determine the next course of action. We then talked with our medical team to get an assessment of the medical event at cruise. The Doctor told us he viewed it as an isolated incident and the FO was ok to continue. After talking with the chief pilots, the FO was removed from the flight and the rest of the crew was reassigned.

After running through the incapacitated crew member section of the volume 2 . I would like to add a suggestion for a crew member that was incapacitated but returns to normal after treatment. A note to call the medical team if two pilots are present to discuss returning the crew member to duty or to divert.

Synopsis

A320 Captain reported First Officer became incapacitated during cruise flight.

ACN: 1716040 (910 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : FLL.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : FLL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Takeoff / Launch
Airspace.Class C : FLL

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 596
Experience.Flight Crew.Last 90 Days : 142
Experience.Flight Crew.Type : 596
ASRS Report Number.Accession Number : 1716040
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We failed to correctly set flaps for takeoff after a runway change

I was First Officer and Pilot Flying. We were delayed for over an hour for an EDICT time. When we finally left our pushback clearance was changed twice then we were given taxi instructions to 10L. We started taxiing with a hold short clearance and completed the before takeoff checklist to the line but were then given a runway change to 10R along with new taxi instructions and a second hold short clearance. I updated the FMS and I allowed myself to be distracted and forgot to inform the Captain that the new runway required us to change our flap setting. We were told to expect a XA08L departure but immediately after starting the second engine we were given an early position and hold clearance with traffic on final. We completed the before takeoff checklist without repeating the items above the line. We completed a normal takeoff and climb and I only noticed my error when we retracted the flaps. Vls came up faster than normal so I lowered the pitch to ensure we would continue to accelerate. The rest of the climb was uneventful.

My biggest error was not asking for more time when I felt rushed to be ready for a position and hold. Other errors were not resetting flap in accordance with the takeoff date for our new runway and not repeating the entire before takeoff checklist.

Synopsis

A320 First Officer reported pre-departure high workload coupled with a runway change that resulted in taking off with an improper flap setting. Once flaps were retracted, the flight proceeded uneventfully.

ACN: 1716034 (911 of 2908)

Time / Day

Date : 201912

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person : Company
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 151
Experience.Flight Crew.Type : 14000
ASRS Report Number.Accession Number : 1716034

Events

Anomaly.Aircraft Equipment Problem : Critical
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Flight planned to operate an aircraft with APU (Auxiliary Power Unit) inoperative. ZZZ WX was moderate to severe icing, wind, snow. Night flight. ZZZ1 (an airport in mountainous terrain) was forecasting wind, snow, freezing precipitation, icing. I decided that this environment was not the time to be flying around without an APU. I called Dispatch, conferenced with [maintenance], and refused the aircraft. I then contacted ZZZ [operations] and advised of the refusal, and suggested we start looking for an alternate aircraft. One was coordinated- we flight planned, loaded the pax (Passengers), and departed. Arrived ZZZ1 about 1 hour late. FO (First Officer) was included in my decision making process. I followed proper established procedure/ protocol for the refusal. I utilized company procedures to achieve a successful - and safe - resolution to the issue.

Synopsis

Flight Crew reported refusing an aircraft with an inoperative Auxiliary Power Unit due to anticipated severe weather.

ACN: 1716026 (912 of 2908)

Time / Day

Date : 202001
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3735
Experience.Flight Crew.Last 90 Days : 8
Experience.Flight Crew.Type : 923
ASRS Report Number.Accession Number : 1716026
Human Factors : Situational Awareness

Events

Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Taxiing into ZZZ. Captain had completed turn. Taxiing straight ahead he stated he had no brakes. I did not perceive his taxi speed as excessive for the conditions. Apparently the ramp had a thin layer of fresh snow on top of ice. I queried if I should set the parking brake. He agreed and shortly afterwards the airplane came to a stop. There was no damage to equipment or personnel. A ramp employee stated he threw a chock under a wheel because it appeared we were sliding.

Synopsis

A320 flight crew reported they encountered ice on the ramp and lost braking capability.

ACN: 1715514 (913 of 2908)

Time / Day

Date : 201901

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2163
Experience.Flight Crew.Last 90 Days : 159
Experience.Flight Crew.Type : 2163
ASRS Report Number.Accession Number : 1715514

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1715537

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Started the APU and brought the packs on line. Cabin attendants reported a strange smell and the appearance of smoke in the cabin. Called line maintenance to inspect the problem. By the time line maintenance arrived at the aircraft, the smoke and odor had disappeared. Continued boarding the aircraft. Flight departed uneventfully.

Narrative: 2

Upon starting of APU, Cabin Crew reported faint smoke in cabin. After checking the smoke out, it did not appear to be from a fire (no fire warnings, etc present on flight deck). We called line maintenance out for a check and by the time he arrived the smoke had cleared. The flight went on uneventful and there were zero passenger complaints upon arrival.

Synopsis

A320 flight crew reported that a faint smoke appeared in passenger cabin during preflight with the APU supplying packs. Smoke dissipated prior to Maintenance arrival.

ACN: 1715078 (914 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 37000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Fluid
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1715078
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight XXX was delayed prior to departure 1 hr as Flight Crew and Maintenance worked through 5 MELs and numerous NEFs (Nonessential Equipment and Furnishings). During cruise, FL370, (1 hr from scheduled touchdown) we received a green hydraulic low ECAM followed 2 min later by a low pressure ECAM of the green pump. ECAM was initiated by the Captain. PF (Pilot Flying) duties were transferred to the FO (First Officer) and Dispatch was contacted via ACARS. After discussing options with crew and Dispatch ZZZ was determined as the landing location. Priority handling was requested from ATC and inflight was briefed with what to expect. After all members of the team were up to speed the Captain informed the passengers what to expect. FO continued the PF duties through the descent with a thorough landing briefing. Runway XXL was requested as it was the longest available runway. Once we were released to approach control duties were reversed, Captain PF and FO was PM (Pilot Monitoring). At roughly 15 miles from touchdown Captain called for gravity gear extension from the QRH as per the the briefing. Once on the ground the aircraft was stopped on the runway while fire crash and rescue did a thorough inspection of the aircraft. It was determined that the aircraft was safe to taxi to the gate under its own power. After reaching the gate the Captain made a PA allowing for any questions from the passengers. Passengers were unloaded with no delay. After all checklist items were completed, Maintenance Control, Dispatch, and Assistant Chief on duty was contacted. Timely debrief took place with inflight team, Maintenance personnel on the ground, and Customer Service. Ground Maintenance confirmed hydraulic quantity was low on the Green system and hydraulic fluid was leaking from the Number 1 Pylon. No additional comments added.

Work load was significantly increased due to the amount of deferrals on this aircraft. I have a growing concern of the sheer amount of deferrals I've observed on Airbus aircraft in recent months.

Synopsis

A320 First Officer reported receiving two hydraulic system malfunction messages during cruise, which resulted in a diversion and successful landing.

ACN: 1715070 (915 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y

Component

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1715070

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Our aircraft had 5 MELs and numerous other NEFs in ZZZ that we briefed thoroughly before departure. One of these was a deferred Pack 1. During cruise flight we spent a good amount of time briefing the arrival giving consideration to the icing reports, FICONs (Field Conditions), and gusty crosswinds we could expect. 45 mins prior to landing we received an ECAM for HYD G RSVR LO LVL and HYD G ENG 1 PUMP LO PR. We ran the ECAM and began to consider the ramifications. As speed breaks were partially affected, we knew the MEL for 1 pack inoperative now had an altitude restriction of FL310. We requested that altitude and advised ATC we were working a hydraulic issue and would get back to them with our intentions. With the ECAM complete we began to work through and brief the accompanying procedures. We ran the QRH landing distance numbers with G HYD LO PRESS and asked Dispatch to do the same via ACARS. Dispatch gave numbers quickly considering the FICONS and suggested 22L as the longest runway. We reviewed the now required landing gear gravity extension procedure and other inoperative items used for landing such as the number 1 thrust reverser, alternate brakes with accumulator, lack of auto brake, and slats flaps slow. Next the Flight Attendants were briefed and I told them I would soon talk with the passengers. We [requested priority handling] with ATC. Once down the landing gear could not be raised again and we wanted to decrease the chances of a go around in icing conditions with the gear down. We asked for trucks to inspect the aircraft after landing before taxiing into the gate. Passengers were then informed of the issue. I told the passengers this is not an emergency and that they would not notice anything different other than emergency vehicles meeting us at the runway to give us a quick inspection. Their pilots are frequently trained for these things and other than the pilots having a slightly increased workload it would be a normal landing. On approach to 22L we extended the gear via gravity extension and flaps sooner than normal. The landing gear doors remained down and we discussed the need to ensure they would clear anything we might taxi over later. The landing was normal and soft and the FO (First Officer) verified the correct brake pressures. We stopped the aircraft and set the parking brake. As the FO coordinated with ATC for our inspection which ATC preferred on the runway, I said a quick word to the passengers. Fire crews found no issues upon inspection and we taxied to the gate. Upon shutdown passengers were invited to ask the pilots any questions they had, all were happy. Flight attendants were happy and thanked for keeping passengers so content. Maintenance control and the [Operations] were called. A Mechanic upon arrival showed us hydraulic fluid draining from the engine one pylon. No additional comments

The workload on the pilots was significantly higher throughout this flight due to the large number of deferrals even before our loss of a hydraulic system.

Synopsis

Captain reported an ECAM message in flight, HYD G RSVR LO LVL and HYD G ENG 1 PUMP LOPR, causing an inflight shut down and a precautionary landing.

ACN: 1715041 (916 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1715041

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1715772
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We pushed back in the alley and started both engines. I noticed a smell of jet exhaust, but it wasn't too bad in my mind. The smell dissipated by the time we began our taxi, maybe 2 minutes. We taxied to 10R following 2 airplanes, I got one whiff of exhaust as we moved forward. But, again, not bad or even worth mentioning to the FO [First Officer]. The FO was PF Pilot Flying] and I was the PM [Pilot Monitoring]. We climbed to FL200, cruised for a few minutes and then descended via ZZZZZ arrival. Descending through about 12,000 feet we received a call from the FA [Flight Attendant]. I answered the call. I can't remember which FA called. She said "I believe we had an air quality event". I asked if everyone was OK and to describe her symptoms and what the odor was. She said the FA's in the back seemed to feel worse with scratchy eyes, scratchy throat, and a slight headache. We were now passing through approximately 9000 feet. I asked if we needed to have anyone meet them at the gate (medical wise). She said no, as the smell was gone and they weren't getting worse. I told them we'd be on the ground and at the gate in less than 15 minutes. They weren't sure what the odor was exactly, but when I asked if it [could] have been exhaust, she said that was what it most resembled, but was not sure. We landed and the FO made sure to have Maintenance meet us at the gate. As the customers were getting off the plane, I spoke with the mechanic and told him what happened. After everyone was off the plane the whole crew met up in the forward galley and we talked about the event. I asked about when they first smelled the odor. It was at cruise when they first smelled it and felt the symptoms. I asked the FA's if they were OK and if they needed medical attention. They said no. We left the aircraft with the mechanics. I asked if he wanted me write up the odor, because I had not put anything in the logbook. He told me that he didn't need it in the book. The FO and I debriefed the event and talked about it from start to finish. He also noticed the smell of exhaust, but didn't say anything because it happens from time to time. And the smell dissipated rather quickly. I wasn't sure where to file the report, I picked crew illness because I later found out that the 3 FA's were taken off of their trip due to the Air Quality Event. We were rushing to get to our next flight. We were at gate XX and were going to XY and had about 55 minutes to do so. I should have called a timeout and read the FOM about Air Quality Events. I should have written it up in the logbook and called the [Duty Officer]. Lesson learned - don't allow the tight schedules and plane swaps influence the task at hand.

Narrative: 2

I was the "B" FA, After Door closure and demo, and compliance checks we sat in jump[seat] and noticed a strong smell, it smelled like exhaust fumes, the plane backed out and taxied to takeoff, smell continued to worsen. We were covering our nose and mouth and hoping it would dispute. Our eyes and throat were burning, and headaches began, at 10,000 ft we got up to do service. As we reached the main cabin it seemed to dissipate....but then the "C" flight attendant and I noticed we started to feel fuzzy headed and lack of clarity...we completed our short pub service, trash and compliance and took our jump seats. Upon arrival and deplaning our symptoms seem to worsen...the gate agent also noticed the same smell upon our arrival. As we began to debrief with CA and FO it became clear none of us were fit to fly. We called Supervisor and went to ER in ZZZ. Suggestion: call forward to flight deck immediately upon noticing abnormal exhaust fume levels! Request back to gate!

Synopsis

A320 Captain reported Flight Attendant notification during initial descent of "air quality event" in the cabin with Flight Attendants experiencing physiological symptoms.

ACN: 1714458 (917 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 22000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class A : ZHU

Component

Aircraft Component : Indicating and Warning - Lighting Systems
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 192
Experience.Flight Crew.Type : 1249
ASRS Report Number.Accession Number : 1714458

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Passing roughly 2,000 feet, autopilot off, FO (First Officer) hand flying, get direct CSHEL. I'm heads down in the box, when my peripheral sees an ECAM and System page on lower ECAM pop up. By the time I execute the direct and look over, [the] screens go back to normal [and there is] nothing on [the] Status page. FO has no clue, but I know some thing's up.

Passing roughly 4,000 feet, No Smoking and LDG light green indications on upper ECAM screen cycle on and off a few times, then return to normal. Passing roughly 12,000 feet, "Ding," ECAM: C/B tripped rear panel W-Z. No breakers are tripped, but ECAM stays displayed for about 3 minutes. Then goes out. Passing roughly 18,000 feet, "Ding," ECAM: HYD B RSVR LO AIR PR. On status page, HYD B quantity good, system output press good - 3,000 - but air pressure indicating low.

[I] messaged Technical Support Maintenance Control, [and] they ask about quantity and fluid pressure. I reply, "Both normal." ECAM lasts roughly 3 minutes and then clears itself. Passing roughly 22,000, LDG LT indication on upper ECAM screen cycles on and off. Finally extinguishes after two minutes.

I messaged Technical Support Maintenance Control again [and] ask what's going on. They ignore me [and] never reply. In order to document [them], I wrote up all 4, tripped C/B, HYD low air pressure, smoking light, landing light. While this is all going on, I'm thinking the jet might be on fire. Always my first thought when random, inexplicable things start happening. Two thorough cycles through the system pages reveal all normal, which puts me at ease. Once established at cruise, all indications [were] normal and we continued [the] flight with no further incident.

Synopsis

A320 Captain reported multiple ECAM messages, advisory lights, and ECAM displays switching, then dissipating during flight with no observable systems failing.

ACN: 1713167 (918 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Mixed
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 276
Experience.Flight Crew.Last 90 Days : 68
Experience.Flight Crew.Type : 276
ASRS Report Number.Accession Number : 1713167
Human Factors : Time Pressure

Person : 2

Reference : 2
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 3714
Experience.Flight Crew.Last 90 Days : 178
Experience.Flight Crew.Type : 1326
ASRS Report Number.Accession Number : 1713168
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

Initial takeoff out of ZZZ. Reviewed weather and noted that ZZZ1 was designated as takeoff alternate. Briefed both FO and Lead of the takeoff alternate. ATIS stated that RVR was 500 but continuously being updated. Tower was calling RVR of 1200 due to ground fog. FO and I briefed both his minimums for takeoff as well as reviewed the back of the Airport Diagram page. Airport Diagram page stated with adequate visual reference we needed 1600 or 1/4 mile. Taxiing out watched other aircraft depart as the ground fog was quickly burning off. As we approached number one Tower stated "RVR is 1400 v 3500". They did not provide a base RVR and I did not ask. We informed Tower we were ready when they were. Tower cleared us for takeoff and I stated to the FO that visibility was good and that I could see the departure end of the 9000 ft. [Runway]. My estimate was visibility was greater than 2 miles. We took off uneventfully and shortly thereafter atis stated visibility was 3 miles. On climb out, FO and I discussed weather and reports. Upon inquiry into the FOM, it seems that we should have either queried Tower for specific RVR or delayed takeoff until all numbers were above 1600. Also, I was lulled into a VFR mindset because in hindsight what I was physically seeing did not match the min RVR Tower stated.

Narrative: 2

During preparation for takeoff, morning fog was burning off at a rapid pace. Airport Diagram page showed take off minimums for runways in ZZZ to be 1600 RVR. ATIS was updating to RVR 500 however we clearly could see the runway all the way to the end which is over 9,000 ft. long. Airport conditions were over 2 mile visibility without restrictions. Tower confirmed [a] new report of 1400 RVR variable to 3500 RVR. We told Tower that we were ready for takeoff whenever they could clear us. We were cleared for take off shortly after. The doubt we had was lack of clarity on [the] actual RVR report even though we were VMC at this point and had no problem seeing all of the runway environment. Shortly after rotation ATIS updated again to 3 SM visibility. 3 flights prior to us departed which added to the confusion of required RVR. Conditions did not match RVR reports.

Synopsis

Flight Crew reported departing with RVR under minimums.

ACN: 1713159 (919 of 2908)

Time / Day

Date : 201912

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 659
Experience.Flight Crew.Last 90 Days : 156
Experience.Flight Crew.Type : 659
ASRS Report Number.Accession Number : 1713159
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This was the last day of a four day trip. Preflight was going as planned until we were informed about a GDP for our destination. Our Destination airport had to switch to runways that are rarely used and it was creating some issues. The Captain and I took the extra time to discuss how this would affect our flight and kept a good line of communication with the Dispatcher. One of the threats that we were trying to mitigate early was the filed STAR was not used for the runways in use at our Destination. The Captain and I located two STARS that utilize the in use runway and planned on having our current STAR changed enroute.

During the flight, I asked the Dispatcher if she noticed any flights having their STARS changed but she said that we were the first aircraft approaching our destination from the north. Shortly thereafter ATC changed our STAR and I attempted to load the new arrival into the box but was unable to since the computer did not associate the STAR with the assigned runway. After several failed attempts at programming the box, I requested ATC to assigned us a different STAR that was associated with the runway to mitigate the obvious threat at hand but they informed me that we would descend via the assigned STAR before receiving vectors for the assigned runway. While the Captain and I discussed the situation along with how to mitigate the threat, we heard two other aircraft having the same issue and requesting different STARs (one was definitely an Airbus).

I set up the box as best I could with the assigned STAR and asked for a heading to allow me time to enter the approach when switched to the final controller. At one point during the flight, I asked the dispatcher about the STAR issue in the hopes that he could talk to the ATC desk or other dispatchers to proactively help other flights. The Dispatcher replied that there is not much they can do and we always have the ability to say "unable". The Captain and I discussed what the dispatcher said and could not find a good reason to deny ATC's assignment. Safety of flight was never compromised and we were able to discuss every way to mitigate the threat we perceived. The only thing the Captain and I want to know is how to get FIF to accept a Runway/STAR combination like this without having to manually insert each fix and altitude manually.

Synopsis

First Officer reported difficulty with instructions from ATC for STAR changes on approach.

ACN: 1711699 (920 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Direct
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component

Aircraft Component : Oxygen System/Pax
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 1193
Experience.Flight Crew.Last 90 Days : 113
Experience.Flight Crew.Type : 1193
ASRS Report Number.Accession Number : 1711699
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft was ferried from ZZZ to ZZZ1 and a flight test of a mechanical issue was accomplished on that flight, by I am assuming company test pilots.

Aircraft preflight was normal and departure was normal until two ECAMs for an emergency exit door being open. QRH was run and aircraft pressurization was normal so flight was continued. Probably 10 minutes later an ECAM for an inoperative TCAS was experienced and run. ATC was notified that the TCAS was now inoperative. 

About 1 hour into the flight, we received a call from the flight attendants that a passenger noticed a "placard" on the oxygen mask cover above row 25DEF that said "Oxygen Canister Needed". With no way to verify that the canister was in place or not, and with three open seats, we elected to move the passengers from that row to ensure they had oxygen available. 

During the process of moving them, it was discovered that row 23DEF had the same placard which turned out to be tape with the same wording on it.

Now, we were in the position of potentially not having enough oxygen for the passengers in the event of a depressurization. I gave the aircraft and the radios to the FO (First Officer) and had him begin a controlled descent to 10,000 feet. I got a phone patch to dispatch and Maintenance Control. We discussed the situation and agreed that the proper and safe course of action was to stay at 10,000 feet and have the aircraft go to destination that the canisters could be verified to be, or not be in place. ZZZ2 was the agreed to destination since we doubted the ability of the aircraft to be able to get to the destination or back to ZZZ1 which was slightly closer at the time, at 10,000 feet.

After the phone patch was over, and we had turned the aircraft towards ZZZ2 the Dispatcher sent a message saying we could arrive at ZZZ3 with 4.7 on the fuel. We were already heading back north, and 4.7 with an airplane that had already had a couple of spurious ECAMs and now had us doubting the safety of the aircraft maintenance inspections, led us to reject that option. We continued towards ZZZ2.

As we were heading towards ZZZ2 it occurred to me that ZZZ1 was slightly closer and might give us the option of arriving about a comfort level of 5.0 on the fuel, and would give company maintenance the opportunity to inspect the aircraft and to give our passengers better routing options in case the canisters were not in place. The Dispatcher's initial run on system gave us 5.2 at ZZZ1, so we changed the destination to ZZZ1.

After putting ZZZ1 in as the destination and with a direct ZZZ1 routing and a two fix approach for Runway XXC in the box, the fuel at ZZZ1 was 3.8 varying to as much as 4.1. This was not what the flight plan Dispatch sent us had. We did the basic math of time remaining times fuel flow and that agreed with what the box was telling us. Neither of us on the flight deck were comfortable with putting ourselves into a potential minimum fuel situation when we had just passed a perfectly good airport less than 40 miles away. 

I made a command decision to yet again go to ZZZ2. Dispatch was notified and we landed uneventfully in ZZZ2 with 7.0 approximately in the tanks.

Upon arrival in ZZZ2, the passengers were deplaned and the suspect mask covers were dropped and found to have canisters installed despite the masking tape placards saying otherwise.

The rest of the maintenance items were taken care of in ZZZ2 and the passengers were re-boarded and we departed ZZZ2 and completed the rest of the flight uneventfully.

The Dispatcher and I spoke on the ground in ZZZ2 about the fuel discrepancy. The flight plans he sent us all had us going above 10,000 on the way to ZZZ1 and that clearly led to more fuel remaining than was really going to be there. He also said that he was struggling to get system to give a direct routing, but after the fact he was able to get a flight plan that confirmed the 4.2 or so in ZZZ1 that we and the box had come up with.

Synopsis

A320 Captain reported that a cabin placard indicated several passenger oxygen masks may have been inoperative, resulting in an expedited descent and diversion.

ACN: 1711610 (921 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : Widebody, Low Wing, 2 Turbojet Eng
Crew Size.Number Of Crew : 2
Flight Phase : Parked
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Instructor
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 15
ASRS Report Number.Accession Number : 1711610
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While providing training on outbound ground, the following event happened:

Aircraft X was given taxi instructions to Runway 28R/N5 intersection departure. The session was extremely heavy/mostly difficult for the CPCIT Career Professional Controller In Training. After giving taxi instructions, the CPCIT moved on to other tasks that needed to be handled. 

After the session, the FLM told both the CPCIT and me that Aircraft X had to "slam on the brakes" to avoid hitting another taxiing aircraft on an adjacent taxiway. When that happened, a flight attendant was injured, forcing the aircraft to return to the gate.

After watching and listening to the replay, it is obvious that both the CPCIT and I missed a traffic call that should have been made to avoid the situation.

While I cannot speak for the CPCIT, CPCIT may have thought that the aircraft established on the A taxiway would be blocked by that heavy jet, allowing the time needed to taxi Aircraft X off gate XWE. However, after viewing the replay, I do not believe this is the case. My focus was on things elsewhere on the airport when this aircraft was taxied, and I missed the instruction and traffic call.

The only thing I can recommend that would have prevented this from happening is to ensure that my scan, as well as the CPCIT's scan is complete and through, and that any coordination between outbound ground and inbound ground is accomplished.

I will continue to work on my abilities as a veteran Controller to maintain my scan, and pass those traits on to those who I train.

Synopsis

Tower Controller reported aircraft under their control had to slam on the brakes to avoid contact with another taxing aircraft, resulting in a Flight Attendant injury.

ACN: 1711524 (922 of 2908)

Time / Day

Date : 201912

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Toilet Furnishing
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Lavatory
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1711524

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Airbus Lavatory Sink Strainers O-Ring when in need of replacement requires a special tool to stretch the o-ring into place on the stopper. Ref [figure]. The tool required is XXXXXXX. This tool has been difficult to acquire. The suggestion would be to have the stopper come equipped with the o-ring already attached to the assembly so as to not impact the system with respect to delays in acquiring the tool.

Synopsis

Technician reported difficulties in the installation of an O-Ring on a lavatory sink stopper without the required special tool being available.

ACN: 1709667 (923 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1709667

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fume event. The cause of the fumes must be addressed.

Synopsis

A320 First Officer reported a fume event during initial approach.

ACN: 1709314 (924 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1709314
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I am the FA (Flight Attendant) C and I was at the aft galley checking the carts when I felt a bit nauseous. 1-2 minutes later the FA A called and told me that the Captain wants to talk to all crew members in the cockpit. I went to the cockpit and the Captain asked about the aircraft odor and if there anything else that was unusual. I didn't see the haze or smell any odor at the back, but it was warm at the aft. I stayed at the front galley with the other 2 FAs for about 10-15 minutes waiting for more information about the situation. While waiting, all of us (FAs) was coughing and had itchy throats. Then Captain told us the we are going to deplane. We all deplaned. We had paramedics check us out at the terminal. All vitals were clear. I, personally, am still feeling a little bit nauseous and still have an itchy throat.

Synopsis

A320 Flight Attendant reported fumes in the cabin, Captain ordered crew to deplane.

ACN: 1708957 (925 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1708957

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fume event on Flight ABC.

Synopsis

A320 Captain reported a fume event.

ACN: 1708497 (926 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1801-2400

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Fuselage
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1708497

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Aircraft X was just painted with new paint job and already paint is peeling off inside RVSM critical area. See pictures and log for details.

Synopsis

Technician reported paint peeling inside RVSM area and referenced several instances on other Company aircraft with the same issue.

ACN: 1707961 (927 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 33000
ASRS Report Number.Accession Number : 1707961

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1708373

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Taxiing out from ZZZ, First Officer noticed a faint odor. We discussed it and both agreed it was probably residue odor from starting the engines. After takeoff and climbing through the teens, my First Officer noticed the faint odor again. I couldn't smell anything, but thought I might be able to sense it in my throat. It was then we called the flight attendants and asked them if they smelled anything, they said they thought they smelled something like a burnt nylon rope. At no time did anyone mention "dirty socks." We asked them if they felt it was okay to continue since we couldn't witness firsthand what a few flight attendants were experiencing. They said yes and they would keep us informed of any changes. It wasn't until after landing we were told that they were using their personal breathing apparatuses. We decided at that point to complete a fumes event form and record it In the AML. I don't have any idea if anything really happened or not. We initiated the call to the flight attendants. Not the other way around. Odor detection is very subjective per each individual. Maybe some matter of fact guidelines to remove individual opinions form the process.

Narrative: 2

During startup for our flight to ZZZ1, I noticed a faint musty smell once the packs were back on after engine start. I brought this to the Captain's attention; however, he didn't smell it. Thinking I might be mistaken, I didn't press the issue further and we departed for ZZZ1. During climbout the smell went away and then returned a little stronger. This time the Captain mentioned he noticed the smell, but couldn't place it. Again the musty smell seemed to dissipate, only to return a short while later. At this point, we looked through the logbook again for history. Finding none, we decided to contact Maintenance Control via Dispatch using IPad crew phone. Maintenance Control had not had any complaints on the aircraft and the APU oil was last serviced on 11/XX/19. We didn't want to startle the flight attendants but we did ask for their feedback regarding any unusual smells in the cabin. We were told there was an unusual smell, but it was more like a burning rope smell. Since we didn't have a parallel smell response from the flight attendants, we elected to continue unless things got worse. We reviewed nearby diversion airports, finding ZZZ2 and ZZZ3 nearby, we felt safe to continue. The remainder of cruise to ZZZ1 was uneventful. During descent the smell returned stronger than before and we decided to have maintenance check the aircraft and assess the smell once we arrived at the gate. We never received a call from the flight attendants regarding a change in the smell during the descent. Subsequent to the flight, we learned a jumpseating Flight Attendant reported the smell to the flight attendants during descent, but this information wasn't passed along until after arrival. Maintenance ultimately declined to release the aircraft for continued service and we were deadheaded back to ZZZ. Had we known about the Flight Attendant difficulties regarding the odor/smell, I feel more confident we would have diverted as opposed to continuing to our destination. On the flight deck, we felt the odor was manageable given it's transient nature, but felt it should be checked out by Maintenance upon our arrival in ZZZ1.

Synopsis

A320 flight crew reported an odor initiated during takeoff and continued intermittently during the entire flight. Flight attendants reported having to use personal breathing devices to destination.

ACN: 1707874 (928 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Initial Approach
Route In Use.STAR : PHLBO 3 RNAV
Airspace.Class B : EWR

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : N90
Make Model Name : Small Aircraft
Crew Size.Number Of Crew : 1
Flight Phase : Cruise
Airspace.Class B : EWR

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1707874

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1707904

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Track / Heading : All Types
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Miss Distance.Vertical : 800
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

The event at question occurred during descent to EWR on the PHLBO 3 RNAV Arrival. During descent ATC instructed to descend and maintain 6,000 feet from 8,000 feet. Since we were relatively far out (roughly over "DYLIN"), I engaged Vertical Speed 1,000 FPM down. Crossing around 7,000 feet ATC advised us of traffic, a small aircraft leveled at 6,000 around one o'clock westbound, roughly 10 miles out. We started looking for the traffic, which I obtained around 6,300 feet around 12-11 o'clock position. The traffic at question also replied he had us insight before I had him. Upon getting ALT* roughly prior to "METRO" we got the TCAS RA DESCEND command. Upon execution of the procedure per SOP we descended around 300 feet (5,700 feet), and advised ATC. Around 1-2 miles past our course we turned back to join the arrival and ATC gave us a heading to final.

Narrative: 2

ATC gave decent to 6,000 feet on arrival. Called out a small aircraft at 6,500 about 10 miles at 12 o'clock. Saw on TCAS but not visually until closer. Baron acknowledged seeing us and would avoid. Pilot Flying acquired visual just before getting the TCAS RA and following a descend RA. Went 5,700 feet then called ATC about the event and climbed back to 6,000 feet and requested a heading.

Synopsis

A320 flight crew reported complying with a conflict RA during descent to EWR on the PHLBO 3 RNAV Arrival.

ACN: 1707343 (929 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1801-2400

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1707343
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
ASRS Report Number.Accession Number : 1708048
Human Factors : Physiological - Other

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1707374
Human Factors : Physiological - Other

Person : 4

Reference : 4
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1708038
Human Factors : Physiological - Other

Person : 5

Reference : 5
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1708046
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ZZZ-ZZZ1 Fumes Odor in Cockpit [while] Airborne. Flight crew had previously flown leg (flight) ZZZ2-ZZZ on time in same aircraft. XA40 flight pushed from ZZZ on time.

XB00 Airborne at cruise, Captain and First Officer smell light increasing volume of old sock odor and fumes smell. First Officer said it was giving him a headache. Captain said he could, "...taste it in the air." Crew completed "Elimination of Foul Odors in Flight Deck" procedure. O2 masks were on.

XB05 Captain contacted Flight Attendant A (name) to ask if she smelled anything in cabin. She said "no" so we, in step with QRH procedure, determined that fumes were coming from Pack 1, and shut off pack 1. Cabin pressurization remained stable. Per QRH procedure, after a measured 5 minutes of time, we assessed that fumes had dissipated and per QRH, "No further action required." and continued to destination ZZZ1.

XB10p Notified AS Dispatch and Maintenance via ACARS. Dispatch replied acknowledgment and stated had notified Duty Manager.

XB25p Captain contacted Flight Attendant A again via intercom to confirm no fumes in cabin. She replied that she could smell a light odor fumes only near cockpit door.
No additional fumes noticed by cockpit crew until XC40. Note: Flight attendant communication and coordination during and after flight was very excellent.

XC40 At top of descent, in managed descent mode upon thrust reduction, fumes odor returned to cockpit. Captain and First Officer re-donned masks did not [advise ATC], were given increased speed in descent down arrival. Pack 1 was off entire remainder of flight.

XD01 First Officer uneventfully landed aircraft on Runway XXR in ZZZ1. After landing, started APU; however, as APU air use increased odor/fumes, APU air was taken off.

XD07 Flight entered gate (number) and deplaned. Contacted Maintenance via radio who requested flight crew stay with aircraft as to describe event to maintenance tech. Captain experienced slight burning eyes and First Officer slight nausea.

XD20 Captain called [Duty Manager] who instructed us to remain in jetway for ZZZ1 CFR EMT to arrive for initial medical evaluation.

XD25 AS Maintenance Tech arrived (name), who stated upon entering cabin that he could smell oil fumes.

XD52 ZZZ1 CFR EMT arrive and do quick health assessment. Gives crew reference to hospital for blood chem/CO2 test.

XF08 Captain recontacted [Duty Manager] and notified of we were enroute to hospital for blood tests s were flight attendants.

XF35a Captain contacted [Crew Scheduling], spoke to (name) who we notified that we were on way to hospital. He asked for us to contact [Crew Scheduling] after getting back to assigned hotel to adjust end of duty time.

XF45 Hospital Emergency Room: paperwork, blood work and waited for results. Hospital ER Doctor said that only my First Officer's blood showed elevated, above standard levels of CO2, but under an amount which they consider dangerous. She provided paperwork that stated all crew were clear to return to work.

XH15 Left hospital for assigned crew hotel.

XH30 Arrived into hotel. Captain contacted [Crew Scheduling], as per previous instructions, notified of flight crews arrival to hotel. [Crew Scheduling] rep (name) stated that her co-worker (name) was incorrect and our duty day had in fact ended after our landing in ZZZ1 earlier and time spent in hospital emergency room due to fume event was irrelevant. Furthermore, we were expected to show on time at XA50, getting less than the required 10 hours of rest. And we hadn't even made it to our rooms yet. We replied that as we were both exhausted from the event, EMT, blood draw, and long hospital process, that even if were able to immediately fall asleep upon getting to room, we weren't probably going to be in any condition on reduced rest to do tomorrow's (now today's) next flight. (Name) said that we could call in sick. Captain and I were shocked and disappointed in this altered [Crew Scheduling] interpretation. We replied that we would try to get enough rest.

XI15 Approximately, I got to sleep.

XO05 I awoke to use restroom, immediately recognized exhaustion and inability to get enough sleep in reduced time. I called Crew Scheduling and called in "Fatigue."

Cockpit crew experienced various volumes of fumes/sock odor causing physiological impact and required crew to be screened by Airport Fire Department EMT and then go to local emergency hospital for chemical blood analysis, which showed minimal but measurable levels of CO2 crew blood.

[Crew Scheduling]'s questionable alteration and re-definition of flight crew duty day not taking into account the long and arduous safety and health procedures post flight including EMT analysis and late night into morning hospital emergency visits with blood draw AND further insistence that crew continue onto next flight of pairing with real reduced rest is dubious, unsafe and detrimental to the on time and safe flight operations at company.

A veteran of six previous airlines, I have never seen a flight operation which would have a flight crew continue flying who had recently experienced an physiological impact event such as fumes and on top of that, a reduced real rest period. I highly recommend that (name), in the interest of safety and operational integrity, changes their procedure so that any crew who has experienced any impact to their health in such an event, be automatically removed from flight status for at least a day.

Narrative: 2

Cabin Air Quality event. Dirty socks odor in cabin mostly around Row 30, also much stronger in flight deck and forward galley area. All crew members reported illness due to fumes. We all were examined by paramedics, and went to emergency room at local hospital.

Narrative: 3

[Report narrative contained no additional information.]

Narrative: 4

Cabin air quality event on flight. We noticed a dirty sock smell around Row 30, but didn't think much of it. Just thought it was a smelly airplane. After takeoff and cruising altitude, the odor shifted between slight to moderate in the aft and forward galley, but became noticeably moderate to severe in the forward galley toward the end of [the] flight. I had a headache and some dizziness. When we landed and debriefed with the pilots, we all contacted supervisors, Scheduling, etc. We were seen by EMTs who only checked vitals, and said to follow up with a doctor if symptoms got worse. We called global and got set up to go to [local hospital]. Upon examination and blood tests, they did not find anything measurable. We were all released. It appears the fumes were only enough to cause temporary issues (headache and dizziness). This aircraft had a air quality event prior according to the log book.

Narrative: 5

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported a fume event of varying intensity during entire flight. Entire flight crew received medical evaluation at local hospital for physiological symptoms.

ACN: 1707092 (930 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
ASRS Report Number.Accession Number : 1707092
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was Flight Attendant A on the Airbus. I felt completely fine the entire trip. We got to 10,000 [feet] and remained seated since we were informed it would be bumpy. My head started to hurt very badly. A couple of minutes later I smelled a strong, chemical type odor. My head continued to hurt. The pilot called back and said it was safe to get up now. I immediately went to the back and asked the flight attendants if they smelled anything. They did not. I told them my head was hurting badly. I went back up to the front and started my service. I then noticed I had a metallic taste in my mouth. The pilots came out to go to the bathroom. I explained what happened to the First Officer. He told me to tell the Captain. I told the Captain when the First Officer went out. He asked if I was feeling better now. I told him I thought so but wasn't sure. He didn't seem concerned. I was confused and told the Captain that I knew something was wrong. I went out and continued the flight, still experiencing the pain in my head. I only smelled the odor two times but it went away. Upon descent, the Captain called back and said I needed to fill out a form. He said that he had never experienced any problems before. 

We landed at ZZZ and deplaned. The Flight Attendant found the form and I started to fill it out. It was very hard to concentrate and I wish I had had someone there to advocate on my behalf. I asked if I could fill the form out in the terminal and the Captain said no, he wanted it now. I told the other flight attendants to go ahead and leave. I was extremely upset and confused. They left, I filled out the form as best as I could. I wanted to get off the plane and didn't understand why I [was] being forced to remain there when something made me sick. I walked out and asked a driver if someone could take me to the terminal and he did. I spoke with a Supervisor and filled out a report. I think it should be protocol that the Captain go out and investigate to see if he notices anything. It's difficult when there's only one Flight Attendant up front. I didn't want to alarm any passengers so I couldn't ask them.

Every situation should be handled seriously. At the time that happened, there's a lot of confusion. I had extreme difficulty trying to concentrate and control my emotions. I had never experienced anything before and wish the situation could've been handled better.

Synopsis

A320 Flight Attendant in charge reported physiological symptoms resulting from strong chemical odor during climb which lasted until after landing.

ACN: 1706238 (931 of 2908)

Time / Day

Date : 201912
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Mixed

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1706238

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1706240

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Determine source and solution for odors/fumes.

Narrative: 2

If not already done, maybe a closer monitoring of the aircrafts that show continuous symptoms on this issue.

Synopsis

A320 flight crew reported a fume event.

ACN: 1705487 (932 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 6000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Climb
Route In Use.SID : NEWARK FOUR
Airspace.Class B : EWR

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1705487

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1705506

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We were given a climb to 14,000 feet after level off at 6,000 feet where we were in constant light/moderate turbulence. Controller asked us to expedite climb to 14,000 for traffic. I open climb that gave Thrust Climb/Open Climb in the FMA. When aircraft had approximately 1,000 FPM climb, I put thrust levers in FLX/MCT (Maximum Continuous Thrust) to give the aircraft more thrust to expedite. The aircraft with the autopilot on lagged in pitching up to the flight director and airspeed jumped from 250 to 270. I opted for selected speed and put 230 knots to initiate a faster climb rate. Speed was exceeded for approximately 30 seconds.

Narrative: 2

After departing EWR 22R on the EWR4 SID, we leveled off at 6,000. We were then given an additional climb with the exception to expedite the climb. At this point the, Pilot Flying (Captain) advanced the thrust levers to MCT. The airspeed started to rise above 250 knots, however, the aircraft was very slow to become fully involved in the climb. Momentarily, the airspeed indicated approximately, 270 knots indicated. The Captain allowed the automation to make the proper corrections.

Synopsis

A320 crew reported exceeding 250 knots leaving 6,000 feet during turbulence departing EWR citing inadequate autoflight system response to pilot inputs.

ACN: 1705308 (933 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 5800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1705308
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Flying westbound from JWARD intersection, east of mountains just to the west of SNA, descending into SNA on the DSNEE4 arrival, cleared for the visual Runway 20R at SNA, FO (Flight Officer) flying pilot. Got terrain terrain, pull up, pull up at 5800 ft. MSL. Complied with warning, then normal approach and landing into SNA.

Cause: Neither pilot thought we were low. Had mountains in sight. FO's first time into SNA, first time on DSNEE4 arrival contributed to lack of awareness. Captain briefed prior that SOCAL approach will keep airplane high on profile making it difficult to descend into SNA on a normal profile. This brief probably contributed to FO wanting to descend early. Captain had mountains in sight and thought clearance was going to be OK. 

Suggestions: Don't clear airplanes for visual into SNA when airplane is still east of 5600 ft. mountains east of SNA. Also make chart say keep 7600 ft. until west of mountains.

Synopsis

A320-200 Captain reported GPWS terrain warning activation on descent into destination in mountainous environment.

ACN: 1704937 (934 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Cabin Lighting : High

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1704937
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We initially had to return to the gate before takeoff because there was a computer reset mechanical. Upon returning to the gate, while waiting for mechanics to fix the issue, a passenger told me, Flight Attendant B, that she had a headache and was feeling lightheaded and a few passengers were feeling the same way around her (Row 16). I went to Row 16 to ask people how they were feeling and to see if I smelled anything unusual. A few passengers nearby claimed to feel lightheaded and one said that he had a headache but did not know why. One of the passengers said that she felt like that because of the air vent, claimed she could smell the exhaust and fumes through the air vent. I could not smell anything and Flight Attendant C and D could not smell anything either when they walked by. I told the Captain about it and he went into the cabin to see if he could smell anything and he said he could not. After the computer reset mechanical issue was fixed, we closed the main cabin door.

Then, we got the release to push back again. Then the D Flight Attendant called the front to report the strong fume odor in the aft of the cabin. The A asked the C Flight Attendant and the B Flight Attendant if we could walk through the cabin to verify the smell and as we approached Row 17 we noticed the burning acrid smell, metallic like dirty socks. The further back that we approached the stronger the smell was. The smell in the aft galley was intolerable and the D flight Attendant started vomiting. Flight Attendant C and myself started to feel nauseous, lightheaded, dizzy, itchy throat and eyes.

The C Flight Attendant called the Captain and informed the issue and we headed back to the gate. When the C Flight Attendant went back to take his position to disarm doors, the same smell in the front galley was strong too. Passengers in the front of the aircraft started to complain about it and passenger seated in 1 C started to feel very nauseous but she declined medical attention. 

Main cabin door was opened and we deplaned passengers EMT's were waiting in the gate for any passengers that needed help. We were checked out by EMT's and transported to the hospital for a further check out.

Synopsis

A320 Flight Attendant crew reported being notified by passengers of "burning acrid dirty socks" odor during ground delay. Fumes gradually intensified resulting in health issues among cabin crew and passengers.

ACN: 1704889 (935 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 18000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class C : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1704889

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1704900

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were operating as (flight) from ZZZ1-ZZZ. While on descent into ZZZ through approximately 18,000 feet we noticed an odor once we entered visible moisture. The odor persisted while on descent but once we were out of the visible moisture, the odor lessened. Upon arrival to the gate and after engine shutdown, one of the aft Flight Attendants called to report that the odor was very strong in the aft of the aircraft. A logbook entry was made and Maintenance was called. I also spoke with the [Duty Manager] and made a report to him regarding the cabin odor. After the aircraft was deplaned, the flight attendants and I felt some symptoms and we spoke with Medlink. After conferring with Medlink, we decided to go to the hospital for evaluation and treatment. During cruise, we had a Lavatory Smoke warning which only lasted a few seconds. The flight attendants performed an inspection of all three lavatories and found no evidence of smoke or fumes in any of the lavatories.

Narrative: 2

[Report narrative added no additional information.]

Synopsis

A320 flight crew reported a fume event during arrival descent. Entire flight crew received medical evaluation and treatment at local hospital.

ACN: 1704786 (936 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Parked
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Fuselage
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft.Other 
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 454
Experience.Flight Crew.Last 90 Days : 98
Experience.Flight Crew.Type : 454
ASRS Report Number.Accession Number : 1704786
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On walk around Captain noted paint missing within the RVSM critical area on the Right Side nose fuselage. The appearance is a long horizontal line approximately 2 in. wide by 18 in. long within the RVSM (Reduced Vertical Separation Minimum) critical area which appears to have been machine blended to remove paint peeling.

A write-up defect was not easily identifiable, however Maintenance reference defect # XXXXXXX from the [Inbound Leg]. 

This write up does NOT note the location/damage as being within the RVSM critical area or meeting the inspection guidelines for RVSM inspection.

It is very probable the inspection is compliant. The write-up, to eliminate ground delays, [should] specify the damage is within the RVSM critical area and passes inspection, since aircrew generally are not aware of locations such as "between frame 11; 12 at floor level."

Synopsis

Captain reported finding paint damage in an RVSM critical area that had not been documented correctly nor repaired correctly.

ACN: 1704299 (937 of 2908)

Time / Day

Date : 201911

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1704299

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : MEL

Narrative: 1

Company MEL XXXXX-Cargo Door-Opening Electrical Control System-MEL REF # XXXXX MMEL. This deferral is used to defer the electrical system that activates the hydraulic opening/closing of cargo doors. When this deferral is used, it triggers additional [Company] MEL XXXXX-Yellow System Electric Pump-MEL REF # XXXXX (MMEL 29-XX-XX Rev 26) which mandates that the (fully functional) Y ELEC HYD pump switch be placed to OFF and remain so. I presume this was done out of a preponderance of caution for the ground personnel.

My issue is with Company's mandatory trigger of XXXXX1 when XXXXX is used. There is a WARNING on A320 Flight Manual page 3.XX.X that states that if the aircraft starts to move with the parking brake handle ON, pilots are to attempt to use brake pedals to stop movement. If that doesn't work, the pilots are to press the Y ELEC PUMP switch to ON and try the brake pedals again. If we are on an aircraft that has used MEL XXXXX, we also have MEL XXXXX1 despite the fact that the pump is fully functional. Now the captain must use emergency authority to use the Y ELEC PUMP switch to deal with the UAM (Unintended Aircraft Movement). Just a reminder, UAMs are a big safety issue with us.

The FAA MMEL 52-XX-XX for the electric cargo door system does NOT require that the yellow electric hydraulic pump be deferred as well. Company has effectively prioritized ground safety when the aircraft is static over flight safety when the aircraft is in motion. I submit that flight operations has the higher priority, and that we shouldn't be forced to use emergency authority to use a system that is fully functional and whose use is governed by a warning,"...when operating procedures, techniques, etc., could result in personal injury or loss of life, if not carefully followed."

Synopsis

A320 Captain reported concerns over a Minimum Equipment List procedure.

ACN: 1703540 (938 of 2908)

Time / Day

Date : 201911

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuselage Fairings
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 41
Experience.Flight Crew.Type : 1493
ASRS Report Number.Accession Number : 1703540
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Aircraft in ZZZ. I performed the walk around and found the attached write ups. When the Mechanic arrived he found an additional, not previously addressed, discrepancies (8 more fastener issues). I look at pilot logs and see the unprofessional comments made by the Operations Manager. This is not the first time an Operations Manager in ZZZ has made unprofessional comments that required me to file a report. The other issue I had was with a Maintenance Control Mechanic. The ZZZ Contract Mechanic asked that I listen in on his conversation with the Maintenance Control Mechanic. After Maintenance Control finished his conversation with the mechanic I asked if he could clarify something he said. The Maintenance Control Mechanic rudely said he will not talk to me and said if I have a problem with that I should contact my flight office.

What kind of operation are we running here! Please look at where this aircraft overnighted and note the mechanic wrote many write ups for missing fasteners.

I would very much like an apology from Maintenance Control.

Synopsis

A320 Captain reported numerous fasteners not installed correctly and was chided by Contract Maintenance for writing them up.

ACN: 1702837 (939 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1702837
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Make this a memory item. Too much going on trying to talk to crew, ATC, run checklists etc., while dealing with the O2 mask distraction while Flight Attendants and passengers are inhaling toxic fumes. O2 on, PACKS OFF, RAM AIR on.

Synopsis

A320 Captain reported a fume event during a busy initial approach phase.

ACN: 1702557 (940 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Work Environment Factor : Poor Lighting
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Avionics
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1702557
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I am the Avionics Lead Mechanic on midnight shift. One of the aircraft I watched over was aircraft. During the night it came in with an MEL for Number 1 left hand boost pump inoperative. Mechanics X and Y were assigned to the job. Avionics Mechanic Z was assigned to assist in troubleshooting with them. During the middle of the shift it was determined the fuel low pressure switch was faulty and replaced per MM (Maintenance Manual) 28-XX-XX. CB's were pulled and collared per MM. It was brought to Mechanic Z attention that ADIRU #3 and AOA sensor were faulty during a routine preflight check. Mechanic Z replaced both, but no help and MELed both out. I worked the day shift that morning and got a call saying that all 3 hydraulic quantity indications showed xx's, a no dispatch item. I found CB 131XP-A still collared with a streamer on it behind the observer seat. I reset it and indications came back to normal. Further investigation on my part found that the wrong CB had been collared during the low pressure switch replacement. This CB deactivated Number 3 ADIRU, Number 3AOA sensor, all 3 hydraulic quantity indications and both SDAC's, obviously a no dispatch situation. The CB that should have been pulled and collared was on correct panel, but 2 rows above. Pulling and collaring the wrong CB for Maintenance is very dangerous obviously.

Synopsis

Lead Technician reported discovering the wrong circuit breaker was pulled and collared during troubleshooting and repair of an unrelated problem caused three other systems to be degraded and improperly MELed.

ACN: 1702354 (941 of 2908)

Time / Day

Date : 201911

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 285
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 285
ASRS Report Number.Accession Number : 1702354
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Our first indication of the presence of an electric wheelchair on board was when during descent the purser asked us to give the destination station a heads-up. Neither form nor verbal notification was provided by station personnel.

Synopsis

A320 Captain reported they were not given any documentation for an electric wheelchair loaded in cargo. Flight crew became aware of the during descent to destination.

ACN: 1702247 (942 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1702247
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We were at cruise FL350. There was a line of weather to our east and south and we were IMC and started getting turbulence. It was enough that we had some power fluctuations. Within a minute I smelled the smell; hot, sour, musty odor. My eyes and throat felt dry and burned. Within a minute of that, we got a call from the cabin that they were smelling the fumes also. I told them I would call them back as soon as we addressed the issue. Because we were still in radar contact, I told First Officer (FO) to get the checklist up and I sent a message to Dispatch to let them know before we got to class II. My thought was they would turn us around or send us to ZZZ or ZZZ1, but they wanted us to continue. We ran the isolation checklist, but the smell was dissipating so we were never able to isolate it. All the while were in turbulence and deviating. Over the next 2 hours we had 10 or more fumes events where it was a burst of smell then dissipation. 4 of the events led to FO and I putting on our oxygen masks. Right before reentering ZZZ2 airspace, we got a message from Dispatch that they were considering sending us to ZZZ1. We felt that was the best and safest course of action, but then they said they wanted us to press on. Because we didn't feel incapacitated and there was no guidance to divert. We felt pressured to continue on to ZZZZ knowing that wasn't a decision made with safety as a priority. We continued to ZZZZ, our conscience bothering us, and had a couple more fumes events. The last being when we transferred the bleeds from the engines to the APU. Upon reaching ZZZZ, we contacted Medlink and relayed our symptoms. Symptoms of exposure were: headaches, burning eyes and throats, nausea, light headedness, and numbness in extremities. I was told by Medlink that my symptoms warranted medical attention and that mine weren't even the worst symptoms. Every person on my crew should call them. I was not comfortable with going to an emergency room in ZZZZ so I agreed to do subsequent phone evals with medlink and get back to anywhere in the US for evaluation. In the end, medlink released me, but recommended seeing my doctor at my earliest convenience. I was alarmed at the onset and severity of the symptoms. They did seem to ease fairly quickly after getting fresh air and more water, but never completely went away. Another concern outside of all of this was inflight leadership response. The conditions that two of them were suffering were far more severe than mine. But they were actually afraid to call medlink or get checked out because they spoke to two inflight supervisors. One of those two supervisors told our inflight: "under no circumstance are you to contact medlink." and "you are only allowed to contact medlink if you are incapacitated." In the end they contacted medlink after receiving guidance outside [company] inflight leadership that they should follow FAM (Flight Attendant Manual) guidance and "take personal health seriously." My biggest concerns are the reoccurring fumes events and the downplay by the company. Being pressured to continue the mission despite safety. Finally, feel of repercussions for taking one's own health into consideration.

Synopsis

A320 Captain reported an intermittent fume event resulting in physiological symptoms by entire crew. Reporter stated he felt pressured to continue to the destination in spite of cabin crew symptoms being more severe than flight crew. Entire crew received medical evaluation after flight.

ACN: 1702113 (943 of 2908)

Time / Day

Date : 201909
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : DCA.FSS
State Reference : VA
Relative Position.Angle.Radial : 180
Relative Position.Distance.Nautical Miles : 54
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Rain
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility.Visibility : 10
Light : Daylight
Ceiling.Single Value : 025

Aircraft

Reference : X
ATC / Advisory.FSS : DCA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Facility : DCA.FSS
Reporter Organization : Contracted Service
Function.Air Traffic Control : Flight Service
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Non Radar : 12
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 11
Experience.Air Traffic Control.Time Certified In Pos 1 (mon) : 11
ASRS Report Number.Accession Number : 1702113
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC
Analyst Callback : Completed

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Weather
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

I was working the In-flight position at the DCA AFSS hub monitoring the Miami AOR [Area of Responsibility] frequencies. There was a system outage which prevented me from having a graphical display to provide time-critical meteorological and aeronautical information to the crew of Aircraft X, during which time a multi-cell thunderstorm cluster that contained low level wind shear and severe turbulence was passing over the aircraft's destination (MIA). I alerted my supervisor and no corrective action was taken to compensate the system outage. We do not have a backup system which provides graphical information that allows us to overlay critical weather data in correlation to an aircraft's flight path. We do have AISR (Aeronautical Information System Replacement) as a system back up for flight plan data and weather information processing, however, we do not have a backup system with an accurate graphical display of the aircraft's flight path and severe weather. In addition to this my judgment became altered because we are understaffed at our facility and was under and immense amount of stress because I needed a break which was denied. Specialists are worked to the point of physical and mental exhaustion and are often denied breaks during times of increased traffic volumes which causes facility management to become agitated and angry to which they respond to the operational workforce with threats and intimidation which contributes as a significant safety mitigation factor by creating a hostile work environment. Many specialists are forced and compelled to brief pilots in AORs that they are not trained or certified to do so in by a threatening and hostile management staff. Our facility is operated under Company X and the non-adequate staffing coupled with malfunctioning equipment makes an extremely stressful and hostile work environment which is exacerbated by a vengeful and vindictive managerial team.

Callback: 1

Reporter clarified acronyms and also described working condition as poor.

Synopsis

DCA FSS Specialist reported a system outage affecting flight safety and described their working environment as stressful due to staffing issues.

ACN: 1701807 (944 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 6000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use : GPS
Nav In Use.Localizer/Glideslope/ILS : Runway 22L
Flight Phase : Initial Approach
Airspace.Class B : EWR

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : N90
Make Model Name : Cirrus Aircraft Undifferentiated
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1701807

Events

Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Miss Distance.Horizontal : 15000
Miss Distance.Vertical : 400
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Ambiguous

Narrative: 1

Received a Traffic Alert and subsequent, but very brief, Resolution Advisory with associated 'Monitor Vertical Speed' command for close proximity general aviation cirrus aircraft while being vectored by Newark Approach control for an ILS 22L approach. 

The incident occurred in the vicinity of PHLBO at approximately 6000' at approximately and in conjunction with a frequency change from initial approach frequency of 128.55 to 120.15. The TCAS "Traffic, Traffic" was quickly replaced by a red target and "Monitor Vertical Speed" command and red Avoidance Area before finally receiving "Clear of Conflict" aural alert. The autopilot was disconnected per maneuver SOP and the Pilot Flying was able to maintain outside the red avoidance area without deviating from ATC cleared heading or altitude while I changed TCAS radar scale and visually acquired the general aviation aircraft. 

I initially communicated we were maneuvering in accordance with RA to the approach controller on 120.15, but after the event, reported the event as a TA only. This was inadvertently erroneous as, while brief in time, we did in fact have an RA with the closest point of approach with the cirrus aircraft being +400' and approximately 2.5 miles. 

Contributing factors to the event were: A) Simultaneous approach operations for ILS11 and ILS 22L, B) Three approach control frequencies being utilized versus the normal two(128.55, 120.15, 120.55) and C) congested airspace.

Synopsis

A320 pilot reported erroneously notifying ATC of a TA instead of an RA encountered during an ILS approach into EWR.

ACN: 1701784 (945 of 2908)

Time / Day

Date : 201911

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 800

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class C : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 164
Experience.Flight Crew.Type : 1702
ASRS Report Number.Accession Number : 1701784
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2755
Experience.Flight Crew.Last 90 Days : 195
Experience.Flight Crew.Type : 2755
ASRS Report Number.Accession Number : 1701932
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Airspace Structure

Narrative: 1

Just after sunset, cleared visual approach to XXL in ZZZ. On the base turn, while getting set up on the approach and checking in with Tower, they mention that there will be VFR traffic in the pattern for Runway XY at a nearby airport, ZZZ1. We acknowledge but did not yet see the traffic, and were still turning in. The FO (First Officer) was flying, and set up on a good stabilized visual approach using the ILS as backup. I did see one TCAS hit to the left of course and a few miles away, initially assumed that was our traffic.

Closer in, around the FAF, Tower again mentions traffic, but I respond that we didn't see it. I don't recall my exact words, probably negative contact. At this point we look in and see a TCAS hit directly ahead and slightly below. I continue to look outside for the traffic, but don't see it due to lots of lights against the dark ground. Initially I wasn't concerned as we didn't have an RA (Resolution Advisory), or even a yellow colored traffic icon. But at this point I see we are only 400 feet away. My FO was on the ball, and before I could even say anything he leveled off and offset to the right. We were near the fix ZZZZZ at this point, about 800 AGL, and were on course and glide path when we stopped the descent. The closest we saw on the TCAS was 300 feet separation, and the TCAS traffic icon was merged with ours, no separation. We never acquired the traffic visually.

If we had not stopped the descent and offset, we would have been much closer than 300 feet. I can't say we would have collided, but it is definitely in the realm of possibilities. I can't emphasize enough how high a threat this situation was. When we landed we queried the Tower, and they just said it was traffic in the pattern, as if it happened all the time. At no point did they display any urgency as to the position of the traffic relative to us.

Furthermore, we then realized that the reason we never got an RA was that we had just descended below 900 feet, and TCAS was therefore inhibited. In real time, that had not occurred to us, as we were too busy trying to find the traffic, fly the approach, then make corrections to avoid a collision.

Our best guess is that the VFR traffic had just departed Runway XY, was somewhere around 500 AGL and either level or slightly climbing, but still flying towards ZZZ, i.e., had not yet turned crosswind. This meant it was flying directly in to the glide path for Runway XXL at ZZZ, which we happened to be on at the time. We could not see them due to ground light clutter and deck angle, they were probably flying in the same direction as us and therefore could not see us descending from behind, no RA was available due to altitude inhibit, and we were not getting any kind of avoidance instructions from Tower. We were exactly where we were supposed to be, in full communication with ATC, yet still were placed in a dangerous situation.

Narrative: 2

Cleared visual to XXL at ZZZ and went direct FAF to intercept final, AP (Autopilot) and ATR (Auto Throttles) off. On base turn to final, Tower called out VFR traffic off Runway XY from a local airport. Turned out to be ZZZ1. Captain acknowledged traffic call with negative contact. Since we were configuring at the time, we did a quick scan and didn't see traffic although TCAS showed traffic off to left of centerline with no altitude. Once approach stabilized at the GSI (Glide Slope Indicator), FAF = ZZZZZ1, Tower again calls out traffic departing Runway XY ZZZ1 and Captain replies, negative contact. TCAS shows traffic 1000 ft. or more below. Since it was dusk and deck angle was 2.5 - 3 degrees, aircraft was difficult to see amongst cultural city lighting.

As we continued approach, altitude between aircraft decreased to 500 ft. on the TCAS. I then offset slightly to the right of course and started to level off, slightly inside of ZZZZZ, hoping the Captain could gain visual contact. When I leveled off, approximately 800 ft. MSL, TCAS showed traffic at 300 feet below and climbing. TCAS symbol merged slightly off to our left and 300 feet below. No RA (Resolution Advisory) or TA (Traffic Advisory) sounded. Once clear of symbol on TCAS, I continued approach from approximately 1 dot high on GS and quickly stabilized prior to 500 feet. We landed uneventfully. On taxi-in, I asked ground about traffic we encountered on final. They seemed to feel it was not out of normal operations. After Tower cleared us the visual, they really didn't point out traffic again. Even if we had been on an instrument approach, I don't think Tower would have provided separation.

Difficult to say but if we had not leveled off and if the departing traffic continued to climb or fly extended centerline off ZZZ1, a collision could have occurred. We fell into the trap of relying on Tower for separation, thinking we would visually acquire the traffic and that the traffic would turn crosswind. Unfortunately, none of these occurred. Without reviewing the radar tapes, hard to say how close we were - departing aircraft was probably a high wing aircraft, on extended centerline, climbing and we are overtaking it, and we have a high deck angle making visual acquisition extremely difficult at dusk.

Reviewing company FM (Flight Manual) and FOM and open source documents related to ZZZ1. FM - no RAs below 900 feet. No TAs below 400 feet, so I figured that would have triggered but it didn't. FOM - visual approach provides guidance when following an aircraft to the landing runway but does not provide traffic resolution for other scenarios. ZZZ1 - Remain less than 700 feet MSL due to traffic on XXL to ZZZ. Extended centerline off ZZZ1 at 700 feet puts you in direct conflict with traffic on ZZZ XXL.

Synopsis

A320 Captain reported experiencing a near mid-air collision during approach to landing.

ACN: 1701755 (946 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1701755
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately 30 to 60 minutes after reaching cruise at 36,000ft, The A FA (Flight Attendant) contacted the flight deck to inform us that 3A PAX (passenger) reported a strange smell. The passenger described the smell as a transient burn odor. Passengers next to 3A reported not smelling anything abnormal. A FA also reported not being able to detect any abnormal odor anywhere in the cabin. I instructed the A FA to keep monitoring the situation and to report any change. About 30 to 45 minutes later, A FA reported that a passenger next to 3A now has also detected the strange odor and that 1A PAX is reporting not feeling well but did not smell anything abnormal. At that point I'm assuming 1A is probably unrelated to the odor at row 3. I instructed the FAs to turn the oven off and to keep monitoring. I also switch the pack flow control from low to norm to help clear lingering odor quicker. About 30 to 45 minutes after, we are getting reports that a couple more passengers at row 5 and 8, as well as the B FA and a non-rev FA have now noticed a strange odor. The A and C FA have not detected anything abnormal, but the A FA is reporting not being able to smell anything because she is experiencing burning sensation in her nose, eyes, and throat. The C FA reported the odor as resembling 'the smell when you open a gas oven', 'sewage', or 'dirty socks'. I asked if it smelled like sulfur and she replied 'yes!'. The odor is elusive, transient and of moderate intensity. The C FA is reporting having a light headache. We are now assuming the source might be AIR COND. I applied part of the SMOKE/FUMES/AVNCS SMOKE for AIR COND and turn Pack 1 OFF and ACARS Maintenance that we were experiencing a cabin air quality event. For the remainder of the flight no one detected any abnormal odor. PAX 1A reported feeling well approximately 30 minutes after turning pack 1 OFF. A FA filled a cabin air quality event form. I determined from the FOM Air Quality table that it was a severity level 2 event. Before the flight, Maintenance worked on engine #1 to replace an HPV (High Pressure Bleed) valve. During Post flight debrief with FA, everyone felt fine. Additionally, they reported thinking at one point that it 'smelled like a gas leak'. I made a logbook entry and contacted [Flight Ops Dispatcher] as per FOM. After sending an ACARS message to maintenance, we received no acknowledgment. I sent a message to dispatch half an hour later to asked if they had received my first message and they acknowledged they did. I think it would be nice to receive an acknowledgment.

Synopsis

A320 Captain reported Flight Attendant notification of passengers reporting abnormal odor at various times during flight. Flight Attendants also detected "dirty socks/sulfur" odor during flight. Flight crew completed Air Conditioner fume QRH procedure which caused odor to dissipate.

ACN: 1701021 (947 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
ASRS Report Number.Accession Number : 1701021
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During approach I thought I smelled an "oily" scent. No one else in the crew complained of any unusual smells so I thought it might just be my imagination. Being curious, I asked the F/O (First Officer) if he noticed anything unusual. He said he caught a whiff of something. We were on final approach and landed the aircraft. At the gate I asked our #1 F/A (Flight Attendant) if she noticed any unusual scent and she said no. I then asked the F/A working in the back of the aircraft and he said he smelled a chemical odor during descent.

Synopsis

A320 Captain reported smelling an "oily" odor in the cockpit which was also reported by flight attendants in the aft galley.

ACN: 1700731 (948 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1700731
Human Factors : Physiological - Other
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1700747
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cabin crew reported odor, fumes of a chemical nature very strong in back of plane. Pilots smelled 'dirty socks' and donned oxygen masks. We determined it strong enough to return to ZZZ. We were vectored back to an arrival and landed without incident, and did not declare an emergency.

Narrative: 2

On the climbout the flight attendants called up front and told us they could smell an odor. I was pilot flying so the Captain did all communicating with the flight attendants. We had a brief conversation and did not notice anything too bad at that time. Flight attendants called back again a couple min later and reported having headaches due to the dirty sock smell which escalated to more of a chemical smell in their opinion. We donned our oxygen masks and Captain accomplished QRH procedures as I was flying and working the radios. The Captain decided to divert back to ZZZ. We proceeded back to ZZZ while our masks were on and flew the ILS to XXL...I kept my mask on to the gate and the flight ended without any more issues.

Synopsis

A320 flight crew reported chemical fumes in cabin noticed by flight attendants. Subsequently, odor was detected in the flight deck. Flight crew executed a precautionary return to departure airport.

ACN: 1700059 (949 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 3553
Experience.Flight Crew.Last 90 Days : 108
Experience.Flight Crew.Type : 1902
ASRS Report Number.Accession Number : 1700059

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After lowering the gear on final to Runway XXR in ZZZ, we received an ECAM stating that the auto brake system was inoperative. We accomplished the ECAM procedure and consulted the QRH, which stated that the normal brakes were inoperative also. I chose to go around because of the short runway and immediately asked for a diversion to ZZZ, which was our designated alternate. Passing through 2,000 feet, a Flight Attendant notified us that a passenger was having a seizure. We notified Dispatch and coordinated with ATC and ZZZ1 Operations, requesting paramedics to meet us at the gate. Things were very hectic during the 10 minute flight, but the First Officer and I worked well together.

Synopsis

A320 Captain reported that a brake system failure and a passenger medical emergency resulted in a diversion.

ACN: 1699795 (950 of 2908)

Time / Day

Date : 201911
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Work Environment Factor : Glare
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Airspace.Class D : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1699795
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1700978
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On arrival ZZZ were cleared for a visual Runway XX, FO's (First Officer's) leg. Initially at 4,000 ft. on a high wide right downwind he set and descended to 3,000 ft. for traffic pattern altitude. We did not receive a response to our in-range until moments before so with things well in hand I now selected the taxi page on my Jeps (Jeppesen) to briefly review the anticipated taxi as there were several NOTAMed closures. Then thinking of the numerous towers in the area I decided I was premature and reselected the ILS XX plate which I fumbled some with. In the mean time I did not see my FO select 2,000 feet (which was only 500 feet AGL) and begin a premature descent. I noticed as we were 1,000 feet, RA (Radar Altimeter) [a few] miles from the turn to final and told him to climb back to 2,500 feet.

In retrospect, I should have immediate taken control of the aircraft, but he initially was responding although not as quickly as I would have liked. I thought he had inadvertently selected the altitude and that a correction at a position on a wide base and well before the turn to final was appropriate but did not know that he had lost sight of the airport and become disoriented. I did not call go-around as we were in a position that put us well outside of stabilized approach criteria and did not feel it was required under the circumstances and I felt a climb to 2,500 feet for an intercept of final at about 3 [and] 1/2 miles was more appropriate. About that time we got a "too low gear" I again told him to climb which he did although slowly. I selected gear down to silence the warning in an attempt to unload my FO and get him to again listen to me, as my FO had by this point "checked out" (his words from our debrief of the situation). He was now intermittently holding the side stick priority button and I did not want to initiate a struggle for control of the aircraft and essentially a single pilot go around so using captain's emergency authority I guided him through the approach and landing.

I do want to state that on the previous trip this month and on the other occasions I have flown with this First Officer he has always displayed fine airmanship and professionalism and this situation is completely out of character for him.

In retrospect my reluctance to call for a go-around or take control at a point where I felt it was still possible aggravated the situation. I made assumptions about why the FO had done what he had done, i.e., an error, which turned out to not be correct. He had become disoriented, overwhelmed. I would have not considered before this incident telling someone to "go around" while on downwind. In our position miles from joining final on a wide base I felt the same. In our debriefing I stated that one of my mistakes in the situation was to not have taken control early on. He indicated that he felt a "go around" call may have brought him back into the situation. I actually tend to agree. I would not have hesitated in IMC to call "go around "if we were similarly out of sorts, why the hesitation in VMC.

I want to focus on my misgiving and failings in this report, but do believe it is worth note that in our debrief of this incident we did discuss the use of the ND (Navigation Display) in helping to improve situational awareness. The ILS was briefed as a backup to the visual with the glide path intercept altitude being of note. Communicating his lack of situational awareness and disorientation. And following SOP regarding changes to altitude settings. These things in addition to those already mentioned that I feel I specifically could/should have done better.

Narrative: 2

Starting with the transition from the en-route phase with Center to the approach phase with Approach Control. An estimated 20 nautical miles southwest of the airport with continued ATC descent and course changes. An attempt is made to visually acquire the airport. The Captain states the location and direction of the airport. Crosschecking the navigational display, I state "I do not have the airport". We are handed off to Tower at 4,000 feet on a right downwind to Runway XX slowing to 210 kts and call for flaps 1. It is the first time I visually acquire the airport.

Tower Controller: "Do you have the airport?". I respond "yes", the Captain states he has the airport and informs the Tower. We are cleared for a visual approach to Runway XX abeam the numbers. I descend to 3,000 feet and slow to 190 kts and call for flaps 2. Using the ILS to Runway XX as a back up reference. My intention is to turn base about 1 mile inside of ZZZZZ (FAF) and descend to 2,500 feet, complete the landing configuration and turn a 4 mile final stabilized at 2,500 feet. I begin the right turn and start the FCU selection from 3,000 feet to 2,000 feet while cross checking my visual reference. I have again lost the airport, owing to the sunlight reflection angle combined with the flatness of terrain. While focusing outside I stopped my FCU selection and did not complete the setting to 2,500 feet. Turning my attention back inside the cockpit to crosscheck my reference. I am alerted to both warning sounds and lights of "too low gear". While attempting to ascertain what was happening I observe the Captain's hand selecting gear down, instructing me to climb and configure the aircraft for landing without my input. Disconnected the autopilot and using the course line on the navigational display and started a turn toward final. With a landing clearance, an uneventful landing and flight completion.

First consideration: 20 miles from an unfamiliar airport, difficulty locating I should have requested vectors to final. Second consideration: loss of visual on base should have gone around. Third consideration: should have overridden the Captain on landing and gone around rather than continue to land. Fourth consideration: Task focused, not speaking in declarative CRM statements. Fifth consideration: "Pride" allowing my years and hours of experience rather than the humility to admit needing some assistance. Personal observation: We as a crew tore this incident apart in 4 day debriefing for the remainder of our trip. Conclusion, I have learned and discovered and will go forward in a different manner.

Synopsis

A320 flight crew reported First Officer lost sight of the airport and descended prematurely while on visual approach.