ACN: 1663693 (1051 of 2908)

Time / Day

Date : 201907
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1663693
Human Factors : Training / Qualification
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Primary Problem : Human Factors

Narrative: 1

I sent an ASAP (Aviation Safety Action Program) report that included an issue with a ZZZ Contract Maintenance mechanic that could not tell an inlet/extract valve open or closed in the Pressurization System Display while working on an MEL. I explained clearly to all of you at the ASAP committee that this mechanic was a serious threat to our operation. Your response was simply that [Maintenance Control] will be investigating... end of story. Well... whatever was done or not done, we still have this very serious issue. First, I do not want to make excuses and I accept responsibility for failing to do my job as the last line of defense in our safety culture and process, yes something fell through the cracks, and I'm very upset that I didn't catch it. However, I did warn you that having pilots babysit this mechanic's work will one day fail and create problems. I wonder how many other pilots have had issues with him, how many even knew they had issues, and how many mistakes have been made in total. This is serious guys.

Arrived at report time/on time at the gate in ZZZ for my flight and was told by the gate agents that our airplane was still at the 'A' gates because of the international arrival and that Maintenance was working on the airplane. So about 40 minutes later, the airplane arrives and the mechanic, who taxied the airplane to our gate, remained in the airplane with the cabin doors closed; this while I, my First Officer and flight attendants waited in the hot jetway. I could see the mechanic working in the cockpit with buttons and switches while on the telephone, I suspect talking to [Maintenance Control]. The mechanic finally opens the door after 10 minutes or so. We come in, he still has some work to do he says, and that delaying boarding was not necessary. So he finishes up and leaves, new MEL in place and all signed off. At first glance it all looks good to me, but of course, I still have to check these MELs dealing with issues with the fuel transfer valves. 

The 2 original MELs in the release were XX-XX-30AB-interior trim and XX-XY-01E-ROUTER TANK RH TRNSFR VLV OPEN. The new MEL was XX-XZ-01D for transfer valve CLOSED (ARTR 1.) So I read through MEL XX-XZ-01D and it stated that the transfer valves should show 'open' and ours were 'closed.' This mechanic spent all that time working on this MEL leaving us in the WRONG transfer valve configuration! So I immediately called [Maintenance Control] and asked them about the discrepancy. They instructed me to reset a couple of CBs, but that didn't do the trick. The [Maintenance Control] mechanic kept brainstorming this and figured, "Looks like this was MEL'd wrong, it should be for while the transfer valves are closed, we'll send a mechanic out."; this ladies & gentleman was the seed that led to my confusion for the remainder of the process. They send this same mechanic that brought the airplane in and left us in the wrong transfer valve configuration, and the same one I had issues with on [a few months back]. This mechanic was lost, he could not tell how an opened or closed transfer valve looked like in the system display. I had to grab my iPad, and show him the diagrams in the MEL book, on how to interpret the display, he was clueless. He had performed all these MELs with no idea on how transfer valves positions are displayed and set them wrong, very concerning. So now that I've given him my free 1 minute lesson on transfer valves displays he proceeds to change the MEL. New ARTR 2, new MEL XX-XA-01H to replace XX-XY-01E. Somehow, somewhere in my brain (the previously mentioned planted seed) I was under the impression that we were going to be dispatched with the transfer valves closed (that we were changing the MEL in alignment with our current condition, I thought.) I felt glad 'I caught it' and that now this condition is been taken care of. The mechanic checks his work, sends a picture of the logbook page to [Maintenance Control], and all is good! Here we go! Needed to update our release time since it was closing-in to an hour delay, so we received ARTR 3 for that. We were dispatched with 6,400 over our brake release fuel, so the issue of the external tank fuel total of 3,000 did not flag as a big concern in regards to meeting our fuel requirements since ZZZ was not under any delays. This extra fuel condition is what I would expect if the valves were to remain closed as they were. This 'coincidental' extra fuel was really our 'saving grace.' Through the flight and while in cruise I checked on the MELs placarding and noticed the placarding were not displaying any 'note,' just the MEL number. So I changed the placards to conform [to] the MELs. No concerns, since it indicated a condition after the refueling process and not 'Before Flight.'

We had a mechanic meet us for these MEL's FRs on our following (return) flight to ZZZ, and he set us up in just 5 minutes. Not only were the procedures carried quickly and efficiently, but this mechanic performed the *correct* procedure in stark contrast with the mechanic in ZZZ that set us up for failure: it was at this point I realized the transfer valves were to remain open for flight. With this finding, I wanted confirmation and clarification from [Maintenance Control], so I explained to them that we ended up flying the previous leg with the transfer valves closed and also explained maintenance's confusion with transfer valve MELs in ZZZ before departure. I also mentioned that I would be submitting an ASAP. 

Although I (shamefully) overlooked the directions on this MELs, I believe these MELs could be written with more clarity. The condition of 'valves closed' (when they are normally closed at the gate) present a challenging condition first, because it is a 'normal' condition at the gate and second, because there are no ECAMs associated with it. The 'REMARKS OR EXCEPTIONS' column instructions should be titled 'TRANSFER VALVES NEED TO BE OPEN FOR FLIGHT' for these MELs followed by the current text. For placarding as well, on the 401VU the term 'Before Flight' or 'For Flight' is more effective than 'after fueling.' 'After fueling' implies a procedural step more than a final/dispatch condition. While talking to [Maintenance Control] after our first leg, the mechanic stated, "Well, we don't write the MELs, Airbus does!" So these changes would definitely take time and effort, a lot of red-tape, a big mountain to climb. Is it worth it? You decide. For the sake of safety? I think it is.

In conclusion, this has been the perfect storm, the poor MEL guidance, the confused and incompetent mechanic, a flawed verification system from [Maintenance Control] (when just pictures of logbook pages are not enough) and a Captain and First Officer that had all of this fall through the cracks after initially finding an issue with an MEL and calling [Maintenance Control], and [Maintenance Control] sending mechanics to rectify the issue, then the rectification process setting us for failure AGAIN! If we want a smooth and safe operation we really need to correct these flaws. Although this issue also falls under my responsibility, poor support is not normal and should not be welcomed/tolerated. The same with paranoia, it is not normal for flight crews to feel paranoid after every maintenance action, vigilant yes, but paranoid not.

Synopsis

Flight crew reports discrepancies on MEL Deferral paperwork and substandard work performed by Maintenance.

ACN: 1663528 (1052 of 2908)

Time / Day

Date : 201907
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : IAD.Tower
State Reference : DC
Altitude.MSL.Single Value : 300

Aircraft : 1

Reference : X
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi
Route In Use : None

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A330
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Takeoff / Launch
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : IAD.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 19
ASRS Report Number.Accession Number : 1663528
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Aircraft Y was departing. Aircraft X had landed and was instructed to taxi and HOLD SHORT of the runway and REMAIN on Tower frequency. As Aircraft X was taxiing east on the taxiway Aircraft Y was cleared for takeoff commenced its takeoff roll. As Aircraft Y is accelerating down the runway Aircraft X is approaching the runway. The Ramp Tower calls over and asks (paraphrasing): "if they can bring in that [Company] in on TWY (Taxiway) E" (referring to the Airbus on). This was due to the fact that Aircraft X had initiated contact with the Ramp, on its own, prior to crossing the runway.

I quickly told Local Control to confirm that Aircraft X was still on his frequency AND was holding short of the runway. To say that my heart leapt out of my throat would have been an understatement. Once Aircraft Y passed the taxiway intersection Aircraft X was instructed to cross the runway and contact me on Ground Control.

At that point, I calmly explained to the pilot that he is NOT to contact the Ramp prior to crossing ANY RUNWAY without being explicitly instructed by ATC. Apparently, pilots are initiating early calls to the Ramp to expedite their taxi. However, they are not aware of our Letter Of Agreement (LOA) with the Ramp Tower that states that once transfer of communications to Ramp is done, we have taken care of ALL conflicts. So, because Aircraft X was on the Ramp frequency, the Ramp Controller could have told Aircraft X to continue on into the ramp, assuming that ATC had resolved any conflicts/issues prior to transfer of communications (as per the LOA). It is very fortunate that the Ramp Controller chose to ask about Aircraft X and NOT assume it was OK for it to continue its taxi. Pilots need to be briefed on the importance of maintaining communications integrity and NOT contact the Ramp Tower early, especially when an active runway lies between them and the ramp.

Synopsis

IAD Local Controller reported an aircraft contacted the Ramp Controller for clearance into the ramp too early while still on an active taxiway.

ACN: 1662496 (1053 of 2908)

Time / Day

Date : 201907
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1662496
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1662493
Human Factors : Situational Awareness
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight Attendant smelled an odor mid-cabin during our safety demonstration on Flight XXXX. She notified the rest of the cabin crew of the smell, and I along with Flight Attendant went to inspect the area. We also opened the air vents in our galleys, which reeked of exhaust. 

We notified the pilots, and contacted maintenance personnel. Although maintenance mentioned they did not smell anything unusual, three of us confirmed an unusual odor. While taxiing out, we smelled a pungent smell of exhaust.

Narrative: 2

During the safety demo I was standing at row 21. I began to notice an odor that smelled like warm tires. I smelled it for the duration of the safety demo and afterwards went forward to have the A and B come back to see if they smelled it too. They did and we decided to notify the Captain. We were still waiting at the gate for baggage so he called maintenance. We could smell it in the aft cabin as well as coming from the air vents in both galleys. Maintenance could not smell anything. The smell was not overwhelming but it was present and would come and then dissipate, or we just got used to it and would get another whiff. After they ran their tests they said they couldn't find anything wrong. The smell seemed to have lessened and we decided to go. The B FA (Flight Attendant) felt dizzy after smelling the air vents and I had throat irritation.

Synopsis

A320 cabin crew reported indistinguishable odor in the passenger cabin prior to pushback from gate. Odor was reported to the Captain who requested maintenance action.

ACN: 1662225 (1054 of 2908)

Time / Day

Date : 201907

Place

Locale Reference.Airport : RSW.Airport
State Reference : FL

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Tower : RSW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class C : RSW

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3520
Experience.Flight Crew.Last 90 Days : 178
Experience.Flight Crew.Type : 2681
ASRS Report Number.Accession Number : 1662225
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

RNAV approach to Runway 24. Lots of thunderstorms in the area. On final approach the Tower Controller was also doing Clearance Delivery duties. It was almost nonstop radio congestion the whole way in. We are required to conduct sterile cockpit procedures in this phase of flight. I felt that it was inappropriate for the Tower Controller to be tying up the frequency giving out clearances to other aircraft.

Synopsis

A320 Captain reported that the RSW Tower Controller was transmitting on Clearance Delivery and Local frequency at the same time and thought it was inappropriate for the Controller to tie up the frequency with ATC Clearances.

ACN: 1661635 (1055 of 2908)

Time / Day

Date : 201907

Place

Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZHU

Component

Aircraft Component : Elevator
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 11640.23
Experience.Flight Crew.Last 90 Days : 174.22
Experience.Flight Crew.Type : 11640.23
ASRS Report Number.Accession Number : 1661635
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I operated Aircraft X ZZZ - ZZZ1. I was the Pilot Flying. Flight was routine until we were about 170 miles out of ZZZ1. My First Officer was having problems with his radios when answering. We finally figured out that the problem was his ACP (Audio Control Panel) panel. It was causing problems with his speaker and Mic. He could only hear out of my side. I told him to make the landing and I will do the radios. 

It was just after that time we got a F/CTL ELAC 2 PITCH FAULT ECAM (Electronic Centralized Aircraft Monitor). We got the book out and went through the reset procedure. When we reset the ELAC (Elevator Aileron Control) we received a FLT CONTROL ELAC 2 FAULT and F/ CTL L ELEV FAULT. I should also mention that on takeoff on [Runway] 28L in ZZZ when I rotated the aircraft it felt unresponsive with pitch initially. I even mentioned to [First Officer] that the CG must have been wrong, so this now got my attention. Descent was now upon us and we started down. When reading the notes in the book about no use of the speed brakes and Flaps 3 landing with a note that said we would lose normal law and revert to direct law when we dropped the gear. I decided to try to communicate with Dispatch and Maintenance. I had them call me and with the radio issues and was able to confirm what we had read in the handbook. The work load was really starting to increase at this point. I told [First Officer] I would take control of the aircraft. With the degradation of the elevator system along with being unable to use the speed brakes to meet ATC constraints on the STAR, I decided to [advise ATC]. I briefed our Purser about the situation but did not prep the cabin. ATC got us right in and we landed on Runway 9 with no issues. It should be noted that when we dropped the gear we never did go to direct law. We did have elevator flutter on the descent however.

Synopsis

Flight crew flying A320 encountered multiple system failures in flight.

ACN: 1661433 (1056 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuel Quantity-Pressure Indication
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1661433
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Dispatch
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
When Detected : Pre-flight

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I would like to take this time to address an issue with the procedures used in deferring inoperative fuel tank indications on the A320 family of aircraft. This issue affects XXX and XXY fleets. This misapplication of this MEL (Minimum Equipment List) has had a negative impact on our operations by causing numerous delays and cancellations. I would also like to thank everyone in advance for any help to resolve this issue. I will try to be as concise as possible in this correspondence, as this is a complex issue involving the MEL, fueling manual, [Maintenance Control] and Flight Dispatch. I will be using Aircraft X, since it is the most recent example and typical of the problems that we encounter. Aircraft X arrived at ZZZ on flight XYZA. The right wing tank fuel quantity was deferred as per MEL 28-XXX. This deferral was applied to this aircraft on [date] in ZZZ1. This is the second time that the right fuel quantity indication was deferred on this aircraft in the last 30 days. The reason for the deferral was a flight crew report of the right main tank quantity indicating XX. The fault was recorded on CFDS (Centralized Fault Display System) as probes 34QT2 and 30QT2 as inoperative. On the A320 series aircraft, the fuel quantity is displayed in the cockpit on a CRT (Cathode Ray Tube) screen. The wing has a separate fuel quantity indicator. A failure due to probe faults affects cockpit and wing indications simultaneously. Obviously if the fault occurs in flight there is no way of knowing that the wing gauge is also inoperative. This is an important point, since it will have an impact on how Flight Dispatch applies the MEL and can utilize the aircraft. When we (Maintenance) is required to comply with MEL 28-XXX, we are directed to fuel the aircraft as per the [company] fueling manual. When fueling as per the [company] fueling manual, we must fuel the airplane in accordance with section titled "Refueling with Inoperative Fueling Quantity Indicators". This section has three methods that can be used. Method 1, method 2 and method 3. Method 1 can only be used with an operative flight deck display and inoperative wing display. Since the cockpit display was deferred as inoperative, we cannot use method 1. That leaves us with methods 2 or method 3. Either method requires us to use the MLI (Magnetic Level Sticks) to verify the final fuel quantity. Most XXX and some XXY A320 series aircraft are only equipped with three (3) MLI indicators on the wing tanks. Aircraft X is one of those aircraft. One aircraft with three MLI indicators, the wing tank fuel can only be verified up to 1800 gallons or 12000 lbs. NOTE - a full wing tank has a capacity of 13500 lbs. This limitation is clearly spelled out in the fueling manual. With this in mind, the total fuel on board that can be verified by MLI is only 24,000 lbs. This is because: 1) Wing tank fuel quantity must remain even. 2) Center tank and ACT (Additional Center Tank) tank fuel cannot be added until the wing tanks are full. (13,500 lbs.). The trap here is that Dispatch and routing are not aware of this limitation because MEL 28-XXX does not give this restriction unless MEL 28-YYY (re-fuel panel indicator inop) is also issued. The problem now is that the MEL 28-XXX, as written, has created a situation where Maintenance cannot fuel the wing tanks to full due to MLI not being capable of reading a full wing tank, and Flight Dispatch is not aware of this restriction due to MEL 28-YYY not issued to the aircraft. I look forward to your feedback in solving this problem.

Callback: 1

Maintenance Technician explained that The Minimum Equipment List (MEL) currently does not have sufficient instructions to enable maintenance to determine definitively whether the wing panel gauges are providing correct fuel quantity. If the main tanks are fueled to any amount greater than 12,000 lbs the exact amount cannot be verified. This is because the manual stick method of verifying fuel quantity is limited to 12,000 lbs max. Therefore unless the fuel is pumped out of the main tanks to a level below 12,000 lbs and verified by the stick you must flight plan the flight based on a maximum fuel load of 24,000 lbs. This of course assumes that you have verified that both left and right main tanks have at least 12,000 lbs each for a total of 24,000 lbs. The maximum of 24,000 lbs is due to the fact the Center Tank and Additional Center Tank (ACT) cannot be fueled unless the main tanks are filled to 13,500 lbs each. Therefore the Maintenance technician is recommending that the A320 MEL be revised to include guidance for Maintenance, Flight Planning and Dispatch in this regard.

Synopsis

A320 Maintenance Technician encounters Fuel Tank Minimum Equipment List (MEL) issue.

ACN: 1661320 (1057 of 2908)

Time / Day

Date : 201907
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 15987
Experience.Flight Crew.Last 90 Days : 260
Experience.Flight Crew.Type : 6525
ASRS Report Number.Accession Number : 1661320
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Around the T/D (Top of Descent) for the ZZZZZ-3 into ZZZ, # 2 ACP (Audio Control Panel) went inop. Capt (PF) (Pilot Flying) gave me (PM) (Pilot Monitoring) the controls and said he would take the radios and I would fly the a/c (Aircraft). We sent MRM (Maintenance Reporting Manual). Right after that, we got an ELAC (Elevator Aileron Computer) # 2 fault. We followed the procedures in QRH (Quick Reference Handbook). After the process of QRH steps, we got a Left ELEV fault. We followed the steps in the QRH for that. At the same time ATC (Air Traffic Control) cleared us to descend via the ZZZZZ-3. With possible flight control issues, we agreed we should declare an emergency with ATC. The QRH referenced do not use Speed Brakes and possible Direct Law at gear down. So ATC cleared us straight to ZZZ for the RWY X ILS (instrument Landing System). The Capt took back the controls and I assumed the PM duties. We landed safely at ZZZ the and taxied to the gate.

Synopsis

Flight Crew flying A320 encountered multiple system failures prior to landing.

ACN: 1661139 (1058 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : MCO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 199
Experience.Flight Crew.Type : 2292
ASRS Report Number.Accession Number : 1661139
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Headset appeared to be inoperative with no voice, although I could hear the hot mic and fumbling with the headset. After much fumbling around downstairs, I overheard the push crew mention that he didn't want to put it too close to his mouth because he was sweating. I completely sympathize, but we have to be able to hear!

Push crew then said they were ready to push, but we would have bags loaded after pushback. FO (First Officer) saw the bags sitting on a belt loader next to the airplane, I had to insist that they be loaded there rather than push and load. The three minute conversation about this was longer than it took to load the bags.

Push crew left hot mic on for the entire push. This is apparently a new headset that is showing up a lot, with a manual mic switch. Unfortunately, I'm not going to listen to a jet engine running during the entire pushback - which means starting after the disconnect.

After brakes set, push crew had difficulty releasing the tow bar. Marshaller came up, and due to the hot mic I could hear her say "you need to tell them that they need to release the parking brake so you can push them back a little further." That is a direct quote - and clearly a falsehood to get us to release the brake so they could get the tow bar off. While I inquired as to had they changed anything on the tow bar (pin or otherwise), the pin then sheared. 

After Maintenance came out, nobody told me on the headset that the tow bar was disconnected and bypass pin removed; they just disconnected after Maintenance told me there was no damage. When I flashed the taxi light three times, marshaller gave me the clear to taxi signal and hopped in the tug. Luckily, she kept looking and saw my repeated signals. After the lead hooked up, we got confirmation that the tow bar and bypass pin were indeed removed, I gave the "disconnect headset" command, and everyone then drove away without another clear to taxi signal. After calling Ops, and 4-5 minutes, we got someone to come out and give us a signal.

Synopsis

A320 Captain reported ground crew's non-compliance to Standard Operating Procedures during pushback.

ACN: 1660394 (1059 of 2908)

Time / Day

Date : 201906

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Other / Unknown
ASRS Report Number.Accession Number : 1660394
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
When Detected : Routine Inspection
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The Ramp Lead loading flight XXXX called after the flight pushed to say he could not lock out the flight because the [load report] was telling him he had Dangerous Goods onboard, but he claimed he had no Dangerous Goods. Looking at the [load report] I asked him was item 1 onboard, he stated yes. I then told him item 1 was Dangerous Goods. Item 1 was UN3373 biologicals. After advising the Ramp Lead the aircraft would need to return to the blocks, the Lead calls back to advise he actually loaded item 3 in Pit 3, which on this particular A320 meant he loaded Dangerous Goods in the nesting system. I asked the Ramp Lead if the shipment had the white sticker with the black lettering and code UN3373 on the outside, he said yes. I then informed him again item 1 was Dangerous Goods.

Synopsis

Air carrier Load Planner reported advising Ramp Lead that aircraft must return to gate to correctly comply with required Hazmat loading configuration for air transport.

ACN: 1658450 (1060 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1658450
Human Factors : Situational Awareness
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I was the Number 2 in the back. After landing back in ZZZ I got a call from my Number 1. She called after making her welcome to ZZZ announcements asking me if I smelt a strange odor in the cabin. That it was an extremely strong rubber/plastic smell in the front of the cabin. I told her I hadn't smelt anything at the time and I was chewing mint gum so that was all I could taste/smell. I walked through the cabin and when I got about half way the smell hit me in the face. It smelled of burning plastic and it was strong. After the passengers finished deplaning the smell was in the aft of the aircraft. It was so strong myself and the Number 4 in the back had to put our scarves over our noses to help deflect the strong odor. I began feeling lightheaded and wanted to get off the aircraft. We informed the Captain and all he said was going to inform maintenance. Before we could get off the aircraft the Captain had left. We are guessing he let maintenance know but I am not a hundred percent sure. So because no one seemed concerned we assumed maybe what we smelled and how we felt was just from getting up early. In ZZZ and I took Tylenol for the migraine and sore throat I was having. I did not feel great but I figured if I ate and drank water after taking medicine my throat and head would ease off. However, it didn't, I still felt under the weather but since it wasn't made a huge deal I knew I could push through and work the final 2 legs a ZZZ1 turn. We got onto flight XXX from ZZZ - ZZZ1 my Number 1 let Captain know what had happened earlier in the day and asked each of us if we were OK to fly, the other 2 and I agreed we didn't feel great, but we wanted to be paid and not get a sick point since our Number 4 had a fume event less than a week ago and had received a sick point for that as well. As a team we agreed we were going to push through. During the flight after service our Number 4 became ill. I saw in her actions she was lethargic and feeling very nauseous. She asked me for some ginger ale and went to the restroom. She was in there for some time. You could see in her face she was not feeling well. I called the Number 1 up front and let her know what was going on with [Number 4]. Once we landed in ZZZ1, I'm guessing [Number 1] informed our Captain on what had happened and informed our Captain for the return flight what was going on. Captain called us all to the front of the aircraft for briefing. He reprimand us a bit after finding out we did not seek medical attention. We explained why we didn't and because our Number 4 was still feeling very ill he didn't feel she was fit to fly, so he called the paramedics to come and have us checked out because our health was his main priority. He said he had our backs and would do anything we needed because our health was first priority. We explained to him we wanted to work our last leg and we could push through because we did not want a sick point or have the flight cancel. I spoke up and said we will take the medical evaluation, and after they check our vitals and OK us to fly we would like to finish our last leg. So the paramedics along with firefighters met us on the aircraft and checked our oxygen levels as well as pulse, and blood pressure. The oxygen in my blood was a little low but it was good enough to pass. We all signed paperwork and they asked us if we needed to be taken to the ER. We called [name] to let her know what was going on. And the medics cleared us to fly so we flew our last leg home. I do apologize that we did not seek medical attention right away, but since it was not made a huge deal we all thought we would be fine. The last leg of our flight was delayed and crew scheduling called us to ask us why we had not boarded and we told them we were being checked out by the medical examiner.

Synopsis

A320 Flight Attendant reported a fume event in the cabin which resulted in cabin crew experiencing health issues.

ACN: 1658165 (1061 of 2908)

Time / Day

Date : 201906

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6032
Experience.Flight Crew.Last 90 Days : 216
Experience.Flight Crew.Type : 3230
ASRS Report Number.Accession Number : 1658165
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We taxied to runway more than an hour behind schedule, after our inbound aircraft arrived late. Upon reaching the end of the runway, number one for takeoff, we could not acquire our final weight and balance data from Load Planning. We pulled to the side (both engines running due to prepared for takeoff). After some 30-40 minutes and numerous calls to [ZZZ Operations], it was determined we had a Hazmat issue onboard and would need to clear the movement area to allow a ground crew to check/verify/redo what our ground crew at gate XX had obviously missed. Arrived onto "Spot X" and shut down both engines. After another 10-12 minute wait, the ground crew arrived, "adjusting" whatever was needed to make us, "legal". We opted to wait to start engines until all paperwork [was] received, final weights and final dangerous goods report. The final dangerous goods report never came.

Fuel load sketchy, we opted to return to the gate to acquire 2200 pounds more fuel (our current fuel was right on the edge of comfort for me at that point). 5 minutes and on our way...no big deal for the comfort of options upon arrival to ZZZ1. On arrival to [the] gate, the fueler arrived immediately. Unfortunately, the refuel gauges had quit working. More delay. Eventually, two maintenance technicians arrived and fueled us utilizing the gauges in the cockpit and one tech in the cockpit in communication with the other tech on the ground to get our fuel situation remedied. It worked.

During the interim, I checked our Flight Operations Manual and it clearly states that lithium batteries do not require Captain notification. But if this Hazmat situation is so dire that we need to return to the gate area to address it, shouldn't I be notified of what it is? I thought we no longer carried lithium batteries in the cargo area? Am I remembering wrong or are there issues with our Operations Manual?

Now, with our fuel onboard, all required paperwork (no dangerous goods report), we looked to close the cabin door and could not find our [gate] agent. I walked to boarding area and got her. She was out front with two customers (if I didn't do that, how much longer would we have just sat there...?). She came down, checked with my Purser, and I expected to hear the door shut. We were rerunning all of our checklists at this point. Verifying all switches were in the appropriate position (we had been struggling with poor aircraft during this entire delay and the maintenance tech had been aboard doing what he needed to do). As I left my seat to recheck what was going on, I finally heard the door close (3-4 minutes?).

Next, I turned my attention to our ground crew that I had performed a radio check with only 5-7 minutes prior. I could not raise them. I looked out the window and saw two (could not see third) of my ground crew utilizing their cellphones. Pushing the 'cockpit to ground' buzzer was ineffective so my First Officer utilized his flashlight to get their attention. Finally successful (again, had we not been proactive, how long would we have sat there awaiting other members of the [Company] team to do their jobs). Although not SOP, the verbiage from our ground crew allowed for an uneventful pushback. We taxied, took off, flew and landed uneventfully arriving with 1 hour of fuel onboard. Had we not added that extra 2200 pounds of fuel, we would have been required to [advise ATC of low] (fuel) and demand priority handling.

Not a great moment for [Company]:
1. HAZMAT issues need to [be] handled professionally and given the due that the moniker implies...hazardous materials/cargo.
2. Dangerous goods is a very ambiguous term. How about we agree that ANY HAZMAT by definition, is probably a "dangerous good" and maybe we should ensure the cockpit is notified of its presence?
3. Lithium batteries. Can we get a little guidance here? Wasn't it a lithium battery that caught on fire in the ZZZ passenger terminal? Yes, a few years back and while recharging, but... Wasn't it lithium batteries that overheated on the 787 that caused a grounding of the fleet for a time? Again...a little dated... I am certain I have been reading about lithium batteries not being carried in the cargo holds, yet they are not even considered hazardous or potentially dangerous?
4. When we have a situation where we may have some passengers deplaning while others stay onboard, is it possible to get our solo gate agent a little help? Do not need to be able to run the computer screens. Just someone, in authority, with knowledge, that can assist ON the airplane.
5. Cellphones need to be put aside while ground crews are out on the ramp unless needed for an emergency or for company business related to that flight.
6. Our customers deserve and have paid for our best efforts every time. People [haphazardly] doing their job, or not doing their job (while playing whatever on their cellphones) is not the contract we made with our customers when we took their hard earned monies. Where are our Supervisors? Each of these people work for someone.

Synopsis

A320 Captain reported communications breakdown between flight crew and Dispatch regarding Hazmat cargo issues resulting in ground return to gate. Incident highlighted the need for additional Hazmat training of ground personnel.

ACN: 1657257 (1062 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : N90.TRACON
State Reference : NY
Altitude.AGL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 168
Experience.Flight Crew.Type : 2402
ASRS Report Number.Accession Number : 1657257
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

We were being vectored for the ILS 22L approach at EWR. Weather was a broken cloud layer that went down to about 700 ft. Approach did not slow us down like they normally do to intercept the LOC so Captain asked if we could slow. ATC instructed speed 210kts or greater/descend 3000 ft. / fly heading 150 deg. While in turn further instructions issued: "Fly 190deg to intercept the localizer / 2500 ft. until established / 180 kts or greater until BUZZD / cleared ILS22L approach / contact TWR 118.3". 

When we intercepted the LOC we were above glide path and fast, but slowing. To get on glide path I made slowing the aircraft and configuring the priority which I did. To get slowed up more quickly I turned the autopilot off, but left the auto-thrust on. Configured at gear down, flaps 3, and 170 kts I initiated an aggressive descent that I maintained through 1600 ft. At that point I started to reduce the descent rate, slow and go to flaps full. As we approached approximately 1200 ft. Tower advised us they had a low altitude warning on us, however when I looked at the glide slope we were still one dot above. I also had intermittent visual contact with the ground, but not the field. About this time aircraft also made a sink rate call. At this point we were fully configured and I was continuing to slow the aircraft while reducing the descent rate. I picked up the runway going through 700 ft. and the aircraft was within stable approach criteria. I also picked up the PAPI and it showed two reds and two whites which I maintained until the threshold. We continued to an uneventful landing.

Synopsis

A320 First Officer reported receiving a low altitude alert from ATC on approach into EWR.

ACN: 1657252 (1063 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 155
Experience.Flight Crew.Type : 308
ASRS Report Number.Accession Number : 1657252
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Following a normal pushback, engine start, and after start checklist in ZZZ, we prepared for a single engine taxi out, expecting runway XXL. Taxi clearance from Ramp Control was received, with instructions to "Contact Metering." Both the First Officer and I cleared and verbally announced "Clear left" and "Clear right" prior to any aircraft movement. Following brake release, my F/O continued to visually confirm that we remained clear of any ground service vehicles as we began our left turn away from the terminal area. During our initial movement, my F/O made me aware of a possible clearance issue with a northbound catering truck (passing from behind our right wing). In order to improve our visibility, as we were already moving forward, I turned on the runway turnoff lights to supplement the already illuminated taxi light. My F/O continued to monitor clearance from the truck, and when it became clear that he was not yielding to our aircraft already under movement, and that there was a significant concern that he might hit us, my F/O announced "Stop now". Due to the urgent need to stop the aircraft, we both applied brake pressure. I was not able to see the catering truck until he passed by the front of our aircraft and continued up the terminal service road. Due to the abrupt stop of the aircraft, two of our flight attendants were thrown off balance and impacted the counters in their galley areas. We were not informed of this until after our arrival. After obtaining more information about this event, from the perspective of our flight attendants, I learned that our Purser had struck her head in the area of the coffee pots and bumped her hip on the galley counter. One of our flight attendants in the aft galley bumped her hip and wrist on the galley counter. Both FA's only indicated discomfort and some stiffness in the general impact areas, but no visible injuries were apparent. I offered to obtain medical assistance during our stopover, but both FA's declined and indicated that it would not be necessary prior to our return. I checked on the well being of the two FA's again during our return flight and again upon arrival, with no additional injuries or medical developments indicated by them.

Synopsis

A320 Captain reported several flight attendants sustained minor injuries when the aircraft was stopped abruptly to avoid a collision with a ground service vehicle as they were taxiing away from the gate at ZZZ.

ACN: 1656562 (1064 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZJX.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.Other 
Airspace.Class A : ZJX

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 8
ASRS Report Number.Accession Number : 1656562
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Function.Air Traffic Control : Handoff / Assist
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 18
ASRS Report Number.Accession Number : 1656574
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Exited Penetrated Airspace
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

We had weather forecast for the southeastern US. As the day moved on, the weather started to move eastbound and build, as forecast in my pre-duty weather briefing. Warning Areas were active. About an hour before things started getting pinched, I heard at least 3 controllers inform the Supervisor that if we did not get the Warning Area released, we were going to need another plan. The Supervisor called TMU (Traffic Management Unit) and relayed them this information and requested the Warning Area to be recalled. As time moved on, aircraft started deviating further and further east, ZZZ arrivals were having to be restricted to 24000 feet (Letter of Agreement calls for 34000 feet) in order for us to safely run departures and arrivals head on with each other. 

Still manageable at this point, although the level of safety was suspect. The morning continued to roll-on and the weather continued to move east with multiple controllers, including myself, informing our supervisor that this plan wasn't going to work in the next 20 minutes. The response he was given from TMU was "I have your request". Next thing you know, aircraft were deviating into the Warning Areas. We had at least 2 get in there with point outs and managed to keep most of them outside the airspace with some persuasive suggestion. The Warning Area Controllers began to say unable to take hand offs for our sectors. One sector had at least one aircraft get into the Warning Area on an emergency squawk. Another sector had multiple aircraft requesting deviations into the Warning Area. That sector then had to shutoff the adjacent sector from all directions and another sector shut off departures out of another airport. The East Area had to hold our aircraft as well. TMU did not seem to have a plan or reroute anyone until after our controllers shut the door. Multiple controllers holding, multiple sectors shut off with weather, multiple aircraft in the warning areas. I wish I could say this is an isolated incident. However, this is routine at this Center. We continuously push the envelope of safety just a little bit more each time. We've been here once before, until CAR (Corrective Action Report) came about. CAR helped for awhile, but we've slowly crept back to where we were before it was implemented as part of our 7110.65. 

Appendix B in our 7110.65 addresses this very topic. It was a direct result of CAR some years ago. TMU absolutely did not comply with this directive. This is not the first time, and I'm certain it will not be the last. We frequently request things from TMU, it falls by the way side, or we're told we don't need it, or we have your request. This is not safe. We are not staffed appropriately to be doing the jobs of two and three people with weather deviations and no support from our own unit tasked with protecting us. A plan needed to be implemented two hours prior, like our own Facility SOP calls for. Something needs to change and people need to be held accountable.

Narrative: 2

I was the D-side at a sector when the event occurred. Aircraft X was enroute to ZZZ and was deviating, as all aircraft were through this area. A severe line of thunderstorms was moving eastward affecting all traffic. The Warning Areas were active FL500 and below. The Warning Areas are located east of where the line of weather was heading. All aircraft northbound and southbound were going through the same hole. Aircraft X was unable to turn south or southwest bound away from the warning area. I called to point the aircraft out to the Warning Area controlling agency. They were unable to accept it because of active fighters in that block of airspace. I advised them the aircraft was coming into the airspace and to keep the fighters away from the aircraft. They asked if it was an emergency aircraft and I said no. At that point, I advised the Radar Controller to have the aircraft squawk the emergency code. 

Aircraft X continued on the deviation and exited the Warning Area airspace about 5 minutes later. None of this should have occurred. Our Supervisor at the time had advised TMU (Traffic Management Unit) that we were not going to be able to accept traffic through this area. I called TMU 45 minutes prior to advise them that the hole was closing as the weather moved east, and that we had about 15 minutes before we would have to shut traffic down. I truly don't know if TMU did anything to reroute traffic away from these sectors but the traffic kept coming. It was extremely busy and chaotic. This is not the way traffic should be managed. I've been a controller for over 30 years, working in different Centers. [This Center] is notorious for pushing the limits with the Warning Areas. When I arrived in [this] ARTCC there was a strip in the area with a statement to read to the pilots "I understand you are unable to comply with ATC commands and are using your emergency authority as Captain and [requesting priority handling]". I'm probably paraphrasing but the bottom line is that [this Center] wanted us to give the aircraft a choice between a thunderstorm or the Warning Area, possibly active fire or active fighters in the Warning Area.

That was unacceptable to me then and it still is. [This Center] has been much better over the last few years but there seems to be a regression over the last year or two. This has to stop. Those two days had major traffic, weather and woefully bad staffing. We have a severe shortage of controllers in [this Center] and can't get D-sides or trackers when needed. I went over two hours and 30 minutes on position one day and so did all my coworkers. We are dedicated to safety and hard workers in a very complex environment and this shortage needs to change immediately.

Our TMU should have routed traffic on the west side of this line well before it became an issue. This was a well-defined line of weather, not pop up thunderstorms, and the Warning Area Controllers had fighters in the blocks and wouldn't accept point outs. Absolutely none of us should have been put in this situation. It's unacceptable. It's completely unsafe.

Synopsis

ZJX Center Controllers reported the Traffic Management Unit refused to provide adequate flow control through their sectors and aircraft were forced to deviate into hot Warning Areas.

ACN: 1655732 (1065 of 2908)

Time / Day

Date : 201906

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Flight Phase : Taxi
Airspace.Class B : ZZZ

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 168
Experience.Flight Crew.Type : 1410
ASRS Report Number.Accession Number : 1655732
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

While on final I noticed that I wasn't having my feet in the normal position for landing. I went to adjust the rudder pedals and as soon as I applied pressure they moved away from me again. I was able to steer and brake with only mild difficulty. As I was taxiing in I advised my FO (First Officer) of the issue so he would make sure he was ready to apply the brakes. I wrote up the issue for maintenance.

Synopsis

A320 Captain reported having problems adjusting the rudder pedals while attempting to steer the aircraft.

ACN: 1655095 (1066 of 2908)

Time / Day

Date : 201906

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1861.30
Experience.Flight Crew.Last 90 Days : 117.83
Experience.Flight Crew.Type : 1076.35
ASRS Report Number.Accession Number : 1655095
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Other

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Crew
When Detected : Routine Inspection
When Detected : Pre-flight
Result.General : Work Refused
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

At gate, pre-departure, was given dangerous good summary showing "miscellaneous hazmat" loaded on aircraft. I was curious to know exactly what the miscellaneous hazmat was, and call Dispatch for clarification. Dispatch called Load Planning and Load Planning was unable to provide further description. Dispatch was busy working several issues with other flights so I offered to call dangerous goods hot-line at. Dangerous Goods Specialist was unable to give further description of shipment. When I ask what drill code "9L" represented, Dangerous Goods Specialist said "You'll have to look that up in your pilot manual" and was able to provide any further assistance. I called Dispatch back as well as Station Operations trying to find out what exactly the hazardous material shipment was. At some time in this process, somebody offered they thought the shipment was "airline parts" but had no further information as to what that might be, or what made this a hazardous goods shipment. Batteries, flammability, toxic fumes, etc. At some point in this process, it was decided, not by me, that the shipment would be pulled from the aircraft.

Synopsis

A320 Captain reported a communications breakdown between flight crew and Dispatch regarding the substance of Hazmat on board. Ultimately Hazmat was removed prior to departure.

ACN: 1655074 (1067 of 2908)

Time / Day

Date : 201906

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLC

Component

Aircraft Component : GPS & Other Satellite Navigation
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 126
Experience.Flight Crew.Type : 1870
ASRS Report Number.Accession Number : 1655074
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

Halfway through flight at night, our first indication of a problem was aircraft went into "uncommanded" 30-degree bank turn. Shortly thereafter, the ECAM message for "NAV GPS1 FAULT" appeared. Then "GPS PRIMARY LOST" in scratchpad. Later we got the ECAM "NAV ADS-B REPORTING 1/2 FAULT." There was a fair amount of confusion in the cockpit as the Captain ND showed right of course and correcting back to course while the First Officer ND showed left of course and diverging away from course. About the same time, ATC called and ask where we were navigating to. I reported "position doubtful" and informed ATC we were having navigation difficulties. ATC gave us a vector. At the same time, multiple aircraft on the frequency reported having similar issues. ATC noted that military was doing a large drill to our south and that may be affecting GPS capability. I contacted Dispatch via phone patch. Dispatch did some investigation and found there was a large military operation in progress and NOTAM for possible GPS interference in our area. To say that my raw data navigation skills were lacking is an understatement! I've never done it on the Airbus and can't remember having done it in 25 years or more. All told, we were without GPS navigation for over 45 minutes. Why the aircraft did not go to radio updating for position in a reliable manner is unknown. We eventually landed uneventfully with all GPS capability restored. Aircraft maintenance write-ups completed.

Synopsis

Airbus Captain reported experiencing an uncommanded heading change and navigation display disruption due to GPS signal interference.

ACN: 1655046 (1068 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 12215
Experience.Flight Crew.Last 90 Days : 129
Experience.Flight Crew.Type : 9415
ASRS Report Number.Accession Number : 1655046

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 133
Experience.Flight Crew.Type : 6152
ASRS Report Number.Accession Number : 1655057

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climb out, approximately FL250, ECAM for generator 1 fault. First Officer did ECAM and QRH. Generator 1 would not come back online. Started APU. About 3 hours into flight climbing from FL330 to FL350 out of FL336, APU did an auto shutdown. Again, followed ECAM and QRH. Contacted [Maintenance] and Dispatch. Descended to FL290. Restarted APU. Worked fine but continued as [priority] aircraft as didn't know if it would shut down again. Uneventful landing. Met by fire trucks. Received okay and under ship's power taxied to gate.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported failure of generator #1, followed by auto-shutdown of APU. Flight continued to destination after the APU was successfully restarted at a lower altitude.

ACN: 1654829 (1069 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1654829
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Distraction
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

About halfway through cruise at 38,000 feet, the Captain and I noticed some very subtle, but different feeling vibration in the flight deck. We reviewed the N1/N2 vibration indicators and they were well within the normal range. I suggested to the Captain that the vibration may be happening from something other than the engines. The Captain suggested that we review the vibration advisory in the QRH. We reviewed the QRH regarding engine vibration and noted what to do, in the event we had excessive engine vibration. The procedure was not needed under these specific conditions.

The vibration was intermittent and hard to concretely sense it was really any concern. However, we did, for our own interest, review nearest airport options in case the vibration became significant.

The vibration ended, and we didn't discuss more about it. Then about 10-15 minutes later, I smelled a burning smell towards the rudder pedals. It had the smell of something similar to when you light a match. I also felt some heat from the fumes. 

I advised the Captain that I believed we had smoke in the flight deck and we immediately donned our air masks. We then called the FAs (Flight Attendants), and they reported that they did not smell any smoke. We advised the FAs that we were keeping our masks on for now because we smelled smoke. 

The Captain and I were slightly surprised that the FAs did not smell anything, because we were still smelling smoke. So to troubleshoot, we decided to look at all of our PEDs (personal devices) to see if anything was burning. PEDs were fine. While keeping the masks on, we did each take them off briefly to confirm we still had a burning smell, which we did.

There were no indications from any of our instrumentation and advisory system screens that anything out of the ordinary was going on. But, I had no confidence that the burning smell was going to end. Rather, I was concerned that that situation was going to escalate from a burning smell to a fire. 

I shared my concerns with the Captain and that I felt we needed to divert to ZZZ (we were about 10-15 minutes from Top of Descent to [original destination]) if the smell persisted for 2 more minutes, and he agreed. The smell persisted, we advised Center that we needed to divert to ZZZ. They gave us vectors direct and to descend to 16,000 ft. We then advised the flight attendants we were diverting to ZZZ because we kept smelling a burning smell in the flight deck.

Shortly after we started our descent, the FA's advised us that they also had a noticeable burning smell from the Exit Row up to the Flight Deck. Upon receiving this information, the Captain and I concurred that we should [advise ATC], for priority into ZZZ and to have Fire/Rescue at the field. (Later we would also learn that there was a First Officer in Row 17, who also noticed a burning smell while were descending and asked the Flight Attendants if we were descending because of the burning smell.) I advised the FAs to grab their [emergency gear] at the ready, in case the situation got worse.

We [advised ATC of the situation], which triggered a plethora of ATC scripted questions to us, which workload permitting I answered. (e.g. the nature of the smoke?, where the smoke was?, were we wearing our oxygen masks?, souls on board?, fuel remaining?). 

Our priority at this point was to get the aircraft on the ground safely, but as quickly as possible. On the descent, I was very busy as Pilot Monitoring. First, we had our air masks on the whole time, which was cumbersome. Second, handling numerous comms with ATC, talking to the FAs, discussing the divert with the Captain, and loading a divert into the FMS for ZZZ. The Captain was handling the emergency descent, which took most of his attention. Center offered us any runway at ZZZ, I asked which way they were landing, they said west, so I requested XXL. The Captain was so busy with the descent that he did not change his instrument approach plates from [the original destination] to ZZZ, nor did he have time to, so I asked for him to hand me his iPad and I loaded the appropriate plates for the divert to ZZZ.

Approach checklist was handled through 18,000 feet, as is standard. Flight Attendants advised they were doing okay in the cabin. We discussed how to brief the passengers, FAs asked if we were going to say something to the passengers. But, because we were wearing our air masks, I asked the FAs to give the briefing, as I did not want to raise more panic by them hearing me sound like Darth Vader on the overhead speaker with my air mask on. The Captain advised them to say we were going to ZZZ out of a safety precaution. 

While getting ready for landing, I did not have time to get ATIS, were too busy, so I asked ATC for the weather, to enter into the FMS for performance. The weather was good, it would be a night visual. We did not have time to ACARS dispatch of the divert, that would have been too much of a time sink and a distraction. I thought about it, but did not see having any time to do it, nor did I think about asking ATC to contact company, was just too busy.

When we got lined up for a long final approach, we were still too high, so ATC vectored us out towards the East. They asked me when we wanted to turn back to the field. I advised them that it was too dark in the direction we were flying, that we could not see the airport, we were descending at 3,000 fpm, and if they would help us by calling our turn back to the final approach course. I was also getting task saturated.

ATC turned us a couple more times, then switched us to Approach Control. At this point they advised us that Rwy XXL was closed (I wish Center would have told us, since we requested XXL with them, that would have saved me time close in to the field). So, I was busy again loading the FMS for XXR. When handed off to tower, we advised that we anticipated a normal landing, and that we planned to taxi off the runway. Landing Checklist was completed. Fire and Rescue was all lined up, adjacent to the field. Landing was uneventful.

Upon landing, we taxied off. At this point there began to be confusion. First, we called Ops and they said they didn't have a gate for us; Second, Ground Control advised that the Fire Department was on a different frequency and wanted to talk about how to scan the plane for hot spots; third, we had a bunch of caution/fault lights just come on after we taxied off the runway regarding Faults with the Avionics Blower and Extract Fan, requiring an ECAM procedure to override, and then the Fault warnings repeated even after we pressed the overrides. While talking with the Fire Department, they advised we needed to pull to a gate to scan the plane. At that moment, the FA's in the back called us and advised that they now, for the first time, were smelling smoke. We told them to update us if it got worse and that we were sorting out getting a gate.

We quickly advised Ops that we needed a gate immediately, or we were going to request any gate at the airport. And if we didn't get any gate and if the smoke smell escalated, then the next consideration was to potentially have the air slides deploy to get the passengers off the plane. Ops responded to go to Gate Z. 

Upon getting to the gate, the ground crew scrambled to get us in, after parking we opened up our emergency windows in the flight deck, to get fresh air in the plane.

Fire Department came on the plane first in full gear (while the passengers remained seated), followed by Maintenance in the flight deck where they reviewed the fault lights we had for the avionics. The Captain got real busy with Dispatch, maintenance, and observing the deplaning process.

While we were super busy in the flight deck, a [company] Gate Lead popped in and interrupted us in front of maintenance to tell us that Crew Scheduling had us taking another Airbus, that was a RON (Routine Overnight), to take the passengers on to [the original destination]. The Captain politely told the Gate Lead that we did not know that and we weren't sure we were going to do that, and that we had to talk to several people before we consider that.

The Captain called the [Chief Pilot], and apparently the [Chief Pilot] said that we could take the passengers on to [the original destination], but he had our back either way if we decided not to. At this point, that would have been an FDP (Flight Duty Period) extension. I told the Captain that I was shocked that the [Chief Pilot] would even suggest such a thing, us flying another leg and take an extension, after we had just had an emergency, performed an emergency descent, a divert to ZZZ, and had our air masks on for over 30 minutes. I found it to be incredibly insensitive, and inappropriate. The inconvenience to the passengers should be handled by another flight crew, it appeared to me that Management's view of Safety was getting blurred with operational needs to accommodate the stranded passengers.

After the flight had deplaned the Captain, walked up the jetway to greet passengers to see if everyone was okay. There were no health problems. Meanwhile, the flight attendants and I discussed that we did not want to do another leg. One flight attendant in particular had already told the Gate Lead that he did not want to work another leg.

While in the jetway, discussing the situation with the flight crew, the Gate Lead came down to talk to us, but she was still on the phone, talking to someone in Dispatch/Ops and telling them that she wasn't sure if the crew was going to continue to [the original destination], because one of the flight attendants was having issues. The flight attendant told the Gate Lead that it was very insensitive of her to be calling him out in front of everybody as the problem for not continuing the flight to [the original destination]. The Gate Lead did not get what the FA was saying, rather she started to bark at all of us, that all her Managers had gone for the day, they had no real help from the company to accommodate the stranded passengers and that it was going to be a real inconvenience for them, if we didn't continue on with another flight.

The Gate Lead left for a moment, and the crew discussed how we felt about flying another leg. All three FA's said they were tired and could do the flight, only because it was short. The Captain said he was fine to continue to [the original destination], but if other's didn't want to go, he would support that decision. One Flight Attendant said because it was a short flight he could do it, but really didn't want to because earlier this year he had a fume event and got sick from it, and had to get medical attention. The Captain said, whatever you guys want to do, I'll support your decision. 

The Gate Lead came back again, she looked stressed and then got really pushy and was saying "I don't know what's the hold-up, are you going to do the flight?" At this point I said that the Gate Lead was too overbearing, that she was smothering us and becoming the problem, pushing us after we had just handled an emergency, and we still needed time to decompress. The Gate Lead would not leave, and I told her that we were not going to make a decision until she gave us some space.

After the Gate Lead left, I reflected on the day (three legs, and not wanting to do an FDP extension for a 4th leg, after an emergency). While I knew I could go to [the original destination], I was now irritated by the Gate Lead singling out the FA that didn't want to go, and then being way too pushy to us to work the next flight. So, I told the Captain, there's no way I'm doing another leg, and we need hotel rooms in ZZZ. The Captain told the Gate Lead.

While waiting for Crew Scheduling to figure out hotels, the Gate Lead approached me and said she was sorry, that she realized we have a very busy night, and she was wrong to push us.

[Company] pushes crews after an emergency to work extra flight.

Since this event, I have heard from other pilots, who have flown at [this company], and [an affiliated regional airline], that after an emergency and landing, that the company already lines them up for another flight leg (often to get displaced passengers back to where they need to go). I find this to be wholly inappropriate. If a flight crew handles an emergency, they should not fly for the rest of the day, they should never be asked to take an FDP extension, notwithstanding any inconvenience to passengers. That's not our problem, and the constant "can do" attitude we carry with us to get the job done, gets taken advantage of, at a time when we really should not be operating a flight again that day. There's way too much adrenaline that's built up after events like these.

Many other pilots I've spoken with have said, they flew an extra leg after they had an emergency, because [the company] pushed them (however loudly or subtly, to make their day easier with having to find crews to work the flight) to do the next leg. And every pilot said they wished they hadn't done it, because their head was not in the game.

Further, I don't understand why me and my Captain on this flight, after the emergency, only got about 12hrs rest (behind the door). We didn't get to our hotel rooms until about [after midnight], and had a [early afternoon] van ride to the airport for a Deadhead to [the original destination] to continue the trip.

IRREGULAR OPS

When we landed in ZZZ the gate agents were whittled down to a skeleton crew, they didn't have a gate for us, then when we were at the gate they expressed frustration that if we didn't take the next flight, they didn't have managers there and they were going to be overwhelmed with dealing with stranded passengers. #1, ZZZ is a hub for [this company], we landed [late at night]. [This company] needs to find a way to muster their staff, maybe use [the regional airline for support] as well, and call back managers who have gone home for the day to handle the emergency. No excuse. 

However, as flight crew, we were going to ask for any available gate and ground crew, and not wholly reliant on [our company], and/or if needed deplane remotely with stair trucks or deploy slides if the smoke situation got worse.

AIRPORT OPS

When we landed, after taxiing off the runway, there was a bit of confusion, between ground control, the fire department, Ops, and the aircraft on what to do, where to go. This reminds me of the "Fog of War" saying, where things are not black and white. At the end of the day the Captain is PIC and the flight crew can work together to make decisions, and in that vein, maintain Safety as the number one priority. 

What we could do better, is having re-current training scenarios and discuss in the classroom, and in safety bulletins, that just landing after an emergency, while mostly saves the day, doesn't complete the task. And there will be confusion, and imperfect decisions resulting in it, that feels like too much time passes, before you ultimately park and get passengers off the plane

CREW OXYGEN

When we pulled into the gate, we had both been using our air masks for 30 minutes. We were down to 300 PSI on oxygen. Had this emergency happened over the Pacific, we would have run out of oxygen and perhaps become asphyxiated. We would have had to descend to 10,000 ft., slow to 200 KTS, and open an escape window in the flight deck. That would be a very wild experience. That procedure should be reviewed, and trained more thoroughly. Generally speaking I don't have a good solution for this, unless perhaps there's a procedure where we could get air from maybe the supplemental tanks in the cabin?

I feel fortunate I had been off of reserve for one month, prior to this emergency.

While on Reserve for the past 18 months, I had not flown a lot on average per month, and was not staying comfortable in the plane. Had this emergency happened then, I would have been a weaker FO (First Officer) supporting the Captain.

Synopsis

A320 First Officer reported mild airframe vibrations followed by a burning odor and fumes, which resulted in a diversion.

ACN: 1654758 (1070 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : GEG.Airport
State Reference : WA
Altitude.MSL.Single Value : 5200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : GEG
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class C : GEG

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1654758
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1655289
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Day VMC, terrain in sight, visual approach to GEG Runway 26 backed up by RNAV GPS Y 26. Flying direct to NAXAF from an ATC radar vector. Approximately 7,000 feet and over higher terrain. Started a descent to NAXAF to 5,200 feet when we were past (backside) of high terrain. Received a momentary (one aural annunciation) GPWS warning for terrain. Even though we were day VMC and in sight of terrain, I initiated the GPWS escape maneuver and we notified ATC of the event. We climbed to approximately 7,000 feet. Assured we were clear of terrain, we resumed descent to visual/normal/stabilized landing.

We were descending away from high terrain. I knew the altitude from HILIE to CUGEV was 6,700 feet and planned my descent to not go below that altitude until clear of terrain. We were flying direct to NAXAF so CUGEV was removed as a reference point for descent. At some point we were low enough to briefly trigger the GPWS. We both were in terrain mode and did not receive any terrain caution before the terrain warning. Also, the terrain presentation on the 320 basic aircraft is very difficult to see in bright sunlight. We would have had better backup to our visual clues if the terrain presentation on the navigation display wasn't so weak.

Better awareness of the possibility of a GPWS warning when visual approach to Runway 26. Don't descend until past CUGEV.

Narrative: 2

On vectors to final approach course Runway 26, and in VMC with good visibility we were given altitude of 6,500 feet to clear terrain. Over the high terrain to the east of GEG, between OZOLA and CUGEV south of the final course, we were asked if we had the runway in sight and given clearance for the visual 26. The Captain set the next lower altitude on the approach of 5,200 feet and began the descent. We quickly got a "Terrain" caution and even though we had the terrain in sight and knew we were clear the Captain initiated an escape maneuver. He leveled off at 7,000 feet as I reported our terrain avoidance and climb to ATC. They reconfirmed our clearance for the visual and we continued to a normal landing with no further issues.

Delay starting descent until a bit further beyond the high terrain.

Synopsis

A320 flight crew reported receiving a TERRAIN warning on approach GEG airport.

ACN: 1653482 (1071 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Final Approach
Airspace.Class B : EWR

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Plan : None
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 168
Experience.Flight Crew.Type : 948
ASRS Report Number.Accession Number : 1653482
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During extended downwind vectors at 5,000 MSL on the arrival for RWY 22L yesterday, we experienced a TCAS RA that resulted in a close call with either an aircraft or drone. We were in ALT when we received a "TRAFFIC, TRAFFIC" TA. ATC called out traffic ahead at 12 o'clock, type and altitude unknown. We looked for the traffic. The aircraft was on autopilot and leveled off. The traffic then disappeared from the screen and we thought we were in the clear. Approximately 10-15 seconds later we received an RA "CLIMB, CLIMB". When I looked down at the ND (20 mile range) the symbology showed a RED target at -01 directly in front of the airplane and to the left side of the nose. I have received RAs before, but this one was very close. I immediately disconnected the autopilot, went TOGA and started a climbing right turn. We were configured at Flaps 1. The FO told ATC "TCAS RA". I quickly came out of TOGA and restored normal auto flight when the "CLEAR OF CONFLICT" advisory sounded. We resumed the arrival and no aircraft parameters or limits were exceeded. Shortly thereafter, we heard [air carrier] Flight XXXXX also report a TCAS RA.

Synopsis

Airbus 320 captain reported encountering a aircraft or a drone via a TCAS Resolution Advisory warning.

ACN: 1652502 (1072 of 2908)

Time / Day

Date : 201906

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121

Component

Aircraft Component : Communication Systems
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1652502

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Airbus A320 fleet has a problem that I have not observed in my years of flying the Boeing fleet. The headset microphone jacks (The US type, with separate microphone and headphone jacks, not the European style integrated ones found in some aircraft) installed on the aircraft wear out with use and stop working. Unfortunately, they stop working with the personal aviation noise canceling Bose headsets that some of us use, while still working with company supplied headsets found on the aircraft. 

Discussing this issue with company mechanics I was told that the microphone plug found on Bose headset has slightly different shape than the one found on company supplied headsets, even though they are both TSO-approved. (How is that possible I do not know). The microphone jacks on Airbus fleet wear out with use, and the little metal connectors inside them bend out of shape and no longer make contact with inserted microphone plugs. To remedy the problem mechanics can replace the offending jacks, or disassemble them to bend the metal connectors back into original positions.

The problem is - when we find out that the microphone on our Bose headset no longer works in a given aircraft and make a logbook entry to that effect, the mechanics test it with company provided headset and sign it off as 'ops tests OK'. When we tell them that the jacks, while noticeably loose, still work with company provided headsets but no longer with our Bose headsets, they shrug 'it's your problem' and refuse to do anything about it.

I have a problem with the above attitude, since the company allows us to use personal headsets, per FOM (Flight Operations Manual):

'Personal Headset. Only non-modified TSO-approved headset or ear piece and boom microphone devices are authorized for use. Operate and maintain all devices in accordance with the manufacturer's instructions.'

Furthermore, good personal headsets are of higher quality, more comfortable (I wear mine for the entire duration of every flight, while nobody does it with company supplied headsets), provide better sound fidelity (I miss fewer ATC calls and don't have to ask ATC to repeat transmission using my headset), reduce fatigue, and reduce hearing loss due to their superior noise reduction characteristics compared with company provided headsets.

The company should educate the maintenance personnel about this problem and about the importance of keeping the microphone jacks in good repair so they work properly with all TSO-approved headsets, not just the company supplied headsets.

Synopsis

A320 pilot reported company mechanics are resistant to maintenance action on headset jacks that don't work with personal headsets but do work with company provided headsets.

ACN: 1651480 (1073 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Cruise

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1651480

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure climb, we received ECAM message Engine 1 Oil Filter Clog. We complied with the ECAM instructions and monitored the system. Approximately 30 minutes later, we received ECAM Msg Eng 1 Lo Oil Press. Oil pressure was 80 psi and dropping when we performed the ECAM Action. We shut down engine one in accordance with the procedure. Oil pressure was 50 psi at the time of the shutdown. The engine shut down normally. I [advised] with Center and asked for a turn direct to [a nearby alternate]. I assigned the First Officer flying duties and proceeded to communicate with the crew, passengers, and Company. At the time of the shutdown, we [were] at FL300 and cruising at M.78. There were no indicators such as noise or vibration, but the flight attendants reported loud noises at about the same time as we received the ECAM message. The descent was uneventful and ATC was extremely professional and helpful. We initially received vectors for the ILS but I requested to go [a different] Runway XXC, which is 2,000 feet longer because we were going to have to perform an overweight landing. Approach and landing were uneventful. We touched down with a descent rate of less than 100 fpm and stopped normally. I cleared the runway and was met by ARFF which inspected the engines and brakes at my request. Once released by the ARFF Commander, we taxied to the gate. The passengers deplaned the aircraft normally. The Fire Chief inspected the engine and reported that the site gauge for the oil showed it was full. The oil quantity indicator also showed there was greater than 15 quarts. It appears the issue was a completely clogged oil filter.

Synopsis

A320 Captain reported diverting to an alternate airport after an inflight shut down of the #1 engine because of low oil pressure related to a clogged oil filter.

ACN: 1651399 (1074 of 2908)

Time / Day

Date : 201906
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4600

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : XXL
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17006.6
Experience.Flight Crew.Last 90 Days : 170.1
Experience.Flight Crew.Type : 6767.5
ASRS Report Number.Accession Number : 1651399
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were being vectored for a visual at ZZZ following a go-around. ATC instructed us to descend to 4,000 [feet]. I started a V/S descent, at about 4,600 feet, ATC advised "Stop descent, traffic alert 11 o'clock, 1 mile altitude indicates 3,700 [feet]" I clicked off the autopilot and leveled off at 4,500 feet. About the same time, TCAS issued a "traffic traffic" call. We sighted the VFR aircraft, which appeared to be close off our left, and maintained visual separation as instructed by ATC. Our recommended procedure of selecting TA on simultaneous parallel approaches (ZZZ 10-7) sets us up for not having TCAS RAs available following a go-around. We aren't trained, nor did I ever think about, going back to TA/RA after our Tower instructed go-around. It wasn't until we heard "traffic traffic" that I remembered we were still in TA. This incident followed a go-around, and we were extremely busy setting up for a new approach to a different runway. We were never closer than 500 feet from the VFR aircraft to the best of my recollection but was still surprised by the proximity of the VFR aircraft. Shortly after ATC advised us to descend to 4,000 feet. then cleared us for the visual approach [Runway] XXL. I contacted the [Chief Pilot] to report the incident shortly after we landed.

Synopsis

A320 Captain reported after an ATC requested go-around, they encountered TA while on the return visual approach.

ACN: 1648931 (1075 of 2908)

Time / Day

Date : 201905

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft.Other 
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1648931

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Dispatch
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Flight XXX departed the gate without the required 15 bags securing the Hazmat. Once they returned to the gate, Ramp called and said that the mail in pit 4 is not there due to space. They originally tried to final with the 1200 lbs of mail in pit 4 when it was not. Then I was told that the cargo was split between pit 4 and pit 6. No cargo was shown in pit 6. Everything left the gate in pit 4 but once it came back, changes were made in the [load plan]. They ordinarily could not final due to the Hazmat not being secured but if this wasn't the case, they would have finalized the flight with everything in pit 4 which would be a weight change of 1273 lbs.

Synopsis

Airline Ground Personnel reported gate return aircraft to comply with Hazmat load configuration requirements.

ACN: 1648656 (1076 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 168
ASRS Report Number.Accession Number : 1648656

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

After takeoff from ZZZ on Runway XXR on climbout around 2,000 MSL we heard a loud thud off the front of the plane and felt the plane shudder. We noticed a power reduction on the Number 1 engine and received an ECAM (Electronic Centralized Aircraft Monitor) message Eng 1 Stall. The Number 1 engine returned to full power shortly after. We also smelled a burning odor coming from the air conditioning system. We determined that we struck a bird or multiple birds on the departure climb out. We immediately [notified ATC] and returned to ZZZ airport. We return to the airport without further events. Upon arrival at the gate, on the post flight inspection we noticed damage to the Number 1 engine and left wing leading edge and engine nacelle. Also bird remains on the tail of the aircraft.

Synopsis

A320 flight crew encountered a bird strike after takeoff.

ACN: 1648287 (1077 of 2908)

Time / Day

Date : 201904

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 135
ASRS Report Number.Accession Number : 1648287
Human Factors : Distraction
Human Factors : Troubleshooting

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

This happened a few weeks ago and has been bothering me ever since. Just prior to push my lead Flight Attendant came up and said she was uncomfortable with a very large passenger seated in the exit row. He said he was "willing and able" to assist in an EVAC. However in the FA'S professional opinion this person was so large she questioned whether he was really "able" to assist and asked if he could be moved. I contacted OPS and a CS Supervisor came down and said we cannot move passenger if they say "willing and able." I next contacted the [Chief Pilot], he said he had not had this before and put me on hold to get the company opinion. After a bit he came back and said we could not move this person just for being large. We departed. On arrival I saw this passenger deplaning, in my estimation he was between 500-600 lbs. He had to come down the aisle sideways to fit between the seats, stopping to catch his breath every few seats. In my professional opinion there was no way he should have been given an exit row seat. There is legal and then there is common sense safety. For future reference where do we draw the line on someone so large it may cause issues were there to be an evacuation. This evaluation of "able" to assist should be addressed at the gate prior to boarding the aircraft.

Synopsis

A320 Captain reported concern about a passenger of size sitting in the emergency exit row who had accepted being willing and able to assist in an emergency.

ACN: 1647943 (1078 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1647943

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

One day prior, previous write up "fuel smell in left aft lavatory." 1:10 into flight, flight attendants call and notify us of a smell in the aft lavatory. Unable to specifically describe the smell. First Officer [was asked] to assess if the smell was present in the lavatory. Decision made to divert to ZZZ. QRH procedure accomplished - smell significantly reduced. 15 minutes prior to landing 2 of 3 flight attendants report having headaches. Priority handling was requested, normal approach and landing accomplished. Paramedics met aircraft, attend to flight attendants. No passenger complaints about health or any request for medical assistance. [Aircraft Maintenance Log] entry made for repeat item of a "Fuel or Hydraulic smell in aft lavatory and aft cabin."

Synopsis

A320 Captain reported Flight Attendant notification of fuel fumes predominately in the aft galley. Cabin crew reported health issues in the form of headaches.

ACN: 1646160 (1079 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 193
Experience.Flight Crew.Type : 5083
ASRS Report Number.Accession Number : 1646160
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We experienced a "HYD G RSV Low LVL" ECAM (Electronic Centralized Aircraft Monitor) during descent. We followed the ECAM SOP and secured the PTU (Power Transfer Unit) Off and the ENG 1 Green Hydraulic Pump off. The result was a loss of Green (primary) Hydraulics. After completing the ECAM and following up with the QRH for HYD G RSV Low LVL we continued to landing. The gear successfully came down and normal flaps full configuration for landing. The Captain made a smooth touchdown on the centerline and was able to maintain control on landing with Reverser No 2.He was able to slow to taxi speed and gently clear the runway at the high speed taxiway and come to a complete stop using alternate brakes. Tower handed us over to the Emergency Response Frequency and after about 5 minutes we were able to communicate with the Emergency Response Crash Recovery vehicles who met us at the taxiway. We shut down both engines after getting APU up and running and then had Super Tug tow us to our gate.

Synopsis

A320 First Officer reported a normal landing followed the loss of the Green System hydraulic fluid.

ACN: 1645545 (1080 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1645545
Human Factors : Physiological - Other
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1645599
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

When signing into the aircraft, [we] noticed a faint odor on the aircraft. None of us thought much of it. As the flight progressed, I felt I smelled the odor still. None of the crew thought it was anything but galley smells. I was feeling a little off, but felt it was from coffee on an empty stomach. Talked to the crew and one Flight Attendant said she felt the same but also drank a lot of coffee on an all-night duty period. On arrival into ZZZ, the smell became very strong. The flight attendants also chimed us reporting the odor with passengers commenting on it. We donned our O2 masks and ran the QRC. We were basically on top of the airport, so decided to just continue and not complicate our workload with priority handling unless it looked like we weren't getting in immediately. We were already turning base, so workload was already high. On the ground, we asked for expedited handling and an immediate gate assignment. None of the passengers complained of symptoms and they departed the aircraft. None of the crew noted symptoms on the ground. Called Medical as a precaution and was cleared to go home. Procedures need to be developed to prevent these odor/fumes events.

Narrative: 2

On approach into ZZZ, a very strong odor of "dirty socks" normally associated with atomized oil in the bleed air system entered the flight deck and cabin. We received a call from the flight attendants about it and subsequently donned our oxygen masks and accomplished the items on the Quick Action Card. I performed a normal landing and the aircraft was taxied to the gate without further incident. The Captain called Maintenance, made an entry into the AML and filled out report.

Synopsis

A320 flight crew reported a persistent mild odor during flight which intensified on arrival.

ACN: 1645493 (1081 of 2908)

Time / Day

Date : 201905

Place

Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase.Other

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Ground Personnel : Other / Unknown
ASRS Report Number.Accession Number : 1645493
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Other Person
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When I was going down to do door training on a A320, I noticed the safety lights were positioned wrong. The cabin pressure light was on the left side of door and the slide indicator was on the right. This needs to be fixed before there is an accident.

Synopsis

Air carrier employee observed incorrect placement of safety lights on a A320 cabin door.

ACN: 1644619 (1082 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 9450
Experience.Flight Crew.Last 90 Days : 211
Experience.Flight Crew.Type : 7237
ASRS Report Number.Accession Number : 1644619

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

This report is to document a low speed rejected takeoff. We taxied out from the gate and started both engines. During start we had an ECAM (Electronic Centralized Aircraft Monitor) that said AUTO FLT YAW DAMPER 1 FAULT. We referenced checklist and contacted [Maintenance]. We shut down both engines, reset two circuit breakers and then restarted both engines to clear the fault. [Maintenance] sent us a new [Maintenance Release] and we continued taxi. 

The weather was VMC with light winds. Tower then cleared us for takeoff. I was the pilot flying. I taxied onto the runway and lined up for takeoff. I pushed the power up to about 1.05 EPR on both engines checked the indications and pushed the power up to flex thrust. I called out "check thrust". About that time the aircraft yawed to the right and the First Officer called out "engine failure". I rejected the takeoff. We were at about 30 to 40 knots for a low speed reject (auto brakes did not engage). I went to idle and reverse for the reject. I straightened out the aircraft with rudder to bring it back to centerline and returned the thrust levers to idle. The First Officer called the Tower about the reject and told the passengers to "remain seated". I slowly taxied the aircraft down the runway to analyze the malfunction. 

The First Officer stated he saw an ENG 2 STALL ECAM. We also saw an ENG 2 OVRSPD PROT FAULT ECAM. I decided have the First Officer run the REJECTED TAKEOFF QRC. We cleared the runway, stopped, and set the parking brake. The First Officer referenced the checklist and we shut down the number 2 engine. Tower asked if we required assistance. I said that as a precaution that I would like the fire department to look over the aircraft for any possible damage or turbine/compressor blades that may have created a FOD (Foreign Object Debris) issue. The fire department said that there was no damage to the aircraft. The brake temperatures maxed out at about 150C (normal) and there were no passenger issues in the back. I decided that we did not need to declare an emergency in this case. I contacted Dispatch and [Maintenance] to discuss our situation. We decided to return to the gate normally. The gate return was uneventful.

Synopsis

A320 Captain reported rejecting the takeoff when the #2 engine failed early in the roll.

ACN: 1644585 (1083 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 139
Experience.Flight Crew.Type : 139
ASRS Report Number.Accession Number : 1644585
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After pushback and start we had an ECAM for Auto FLT Yaw Damper 1. Contacted Maintenance Control and worked through resetting and testing the system per his direction. On resolution Maintenance Control sent a new MRD and we taxied out for takeoff. Got a runway change to XXR. When cleared for takeoff, Captain advanced throttles to 1.05 EPR. Power was stable and Captain advanced thrust levers to FLEX detent. Within a couple of seconds, we lost power on the right engine and the aircraft veered sharply right. I called out Engine Fail and noticed an ENG 2 Stall ECAM. Captain announced he was rejecting and restored directional control. I notified Tower and made the "Remain seated, remain seated" PA. The engine stall ECAM disappeared by itself and instead we got an ENG 2 OVERSPD PROT FAULT ECAM. Engine 2 parameters were abnormal however even at idle and after looking at both items in QRH Captain had me shut down the No. 2 engine. Fire trucks arrived and verified no issues they could see, no debris etc. Brake temps were normal as auto brakes never engaged due to the low speed. After coordinating with FAs and OPS and sending ACARs to dispatch, we returned to the gate, deplaned and sent in relevant maintenance write-ups. We completed a crew debrief and overall felt the issue was handled safely.

Synopsis

A320 First Officer reported an engine failure at power up for takeoff.

ACN: 1643086 (1084 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZDV

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1643086
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Inside of 250 miles to DEN, we were filed for the PURRL2 STAR and DEN was landing to the north. Initially briefed the ILS to Runway 35L. Enroute to BOWEE, Denver Center advised that the southern corridor was closing and rerouted us direct to PUFFR for the KOHOE 3. ATIS said that Denver was still landing north. We then changed the approach to the RNAV/RNP Runway 35R and briefed the RNAV. During the briefing Denver once again changed the arrival to the WAHUU2 RNAV Arrival. ATIS still said Denver was landing north and we queried Denver Center and they said they expected a runway change. Based on that information we briefed the approach to Runway 16L due to the shorter taxi. Enroute to GRRUB and cleared to descend via, ATIS said that Denver was only landing Runway 16R and 17R. We changed the runway for the fourth time inside 250 miles (in this case approximately 50-75 miles from the field) and when the runway was changed, the arrival didn't sequence properly so we pulled heading and tried to go direct to KOKOO. Denver Center queried us on our heading and we told him 265 and he asked why we were on a heading. We told him the FMS was not sequenced properly due to multiple arrival and runway changes. He directed us to pickup a heading of 270 and to descent to 12000. We maintained this heading until we were passed on to Denver Approach who vectored us to the visual for 17R. It becomes difficult for all parties involved with the weather in Denver and I know that Denver Center and Approach are working hard to minimize changes once we are assigned a STAR, but four runway changes and three STAR changes are difficult even under the best of circumstances. The First Officer and I discussed what we could have done differently and we both agree that we should have been proactive in letting Denver Center know that we were having difficulties and needed a heading even though the deviation was minor. Additionally, the multiple arrival changes when vectored inside the VIA highlight the need for no transition fix on the approach.

Synopsis

A320 Captain reported a track deviation occurred on arrival into DEN following multiple changes to arrival procedure and runway assignment.

ACN: 1642823 (1085 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 0601-1200

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Cabin Lighting : Low
Number Of Seats.Number : 150
Crew Size Flight Attendant.Number Of Crew : 5

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Galley
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Experience.Flight Attendant.Total : 29.9
Experience.Flight Attendant.Airline Total : 29.9
Experience.Flight Attendant.Number Of Acft Qualified On : 2
Experience.Flight Attendant.Type : 95
ASRS Report Number.Accession Number : 1642823
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent, Flight Attendant 4 came to fwd galley and reported smelling a dirty sock odor mid cabin and aft galley. I (Flight Attendant 1) walked thru the cabin and detected a smell row 8 thru wing area and again in rear galley. I called the Captain and reported [it]. Flight Attendant 2 walked thru cabin and also smelled odor but could not describe. I asked Dead heading pilot to walk thru cabin and he confirmed [the] dirty sock smell. Called Captain again and updated. Captain stated they performed a function to pull air thru/out of the cabin. The Captain called back to ask if had improved? We were strapped in jumpseats for landing (3-4 min out) so, could not confirm. Aircraft was met by firemen and a Mechanic. Captain advised he had [advised ATC].

Synopsis

A320 Flight Attendant reported notifying the flight crew of a dirty socks odor in cabin during descent.

ACN: 1642309 (1086 of 2908)

Time / Day

Date : 201905
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Parked

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1642309

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After the doors were closed, I am just prior to engine start at the gate. The Gate Agent trying to get my attention by knocking on the rear Captain's side window. He knocked no more than three times and the window shattered. I believe he did nothing wrong as many gate agents do this when they try to get your attention obviously was a defect in the window.

Synopsis

A320 pilot reported that the Cockpit window shattered after the Gate Agent tapped on it.

ACN: 1641945 (1087 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1641945
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Work Refused
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Arrived in ZZZ, upon opening door gate agents informed us that we were losing the aircraft. Said I knew aircraft was next door. New crew was on my jet bridge and stated they had a fume event. Event was logged and crew sought no medical help. I wandered over to that aircraft, can't remember tail number and contract was on aircraft. He stated that [Maintenance Control] called and said his run crew had arrived. He explained that they were doing a partial burn out procedure, which included running engines and packs. Also #3 brake was sticking and pulling right according to crew. I never heard a word about an aircraft change nor maintenance procedure until the door opened. I inquired with Dispatch and they said they informed the gate we would be changing aircraft 45 minutes to 1 hour prior. Gate says they were told upon landing. In none of this did I ever hear from or speak with [Maintenance Control] or Dispatch. I informed the Contact Maintenance that I would not run the aircraft without proper airway and eye protection. After about 2 hours, [Customer Service] called and offered hotel rooms that had us reporting back in 1.5-2 hours. None of us accepted, we felt we would spend more time traveling. Assistant Chief Pilot called and asked me about my fume event. I explained it was not mine, and I would only run the aircraft with the proper protections. As you can see, I was never in the loop on anything.

Synopsis

A320 Captain reported communication breakdown between flight crew and Maintenance regarding a fume event which required a "partial burnout procedure."

ACN: 1640350 (1088 of 2908)

Time / Day

Date : 201904

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1640350
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
ASRS Report Number.Accession Number : 1640357
Human Factors : Situational Awareness

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1641005
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was #2, I was in front galley when #4 came up to say he smelled dirty sock smell in back galley, and around row 8-15 in cabin, #1 and myself went thru cabin, and confirmed musty odor smell. DH pilot walked thru cabin and confirmed fume odor also. #1 called Captain who performed fume procedures, [notified ATC and landed], we were met by firemen.

Narrative: 2

On descent Flight Attendant 4 came to front galley and reported a dirty sock smell mid cabin and in rear galley. I was #1 and put a paper towel over nose/mouth and walked thru cabin and confirmed a musty smell, especially higher up in both those areas. #2 also walked thru and confirmed smell. Called and notified Captain. Asked DH pilot to also do a walk-thru and he confirmed smell also. Captain notified us he did a procedure to pull air throughout the cabin. Captain called us to ask if it helped but, we were strapped in jumpseat for landing and could not confirm. Aircraft was met by 2 Firemen and a Mechanic. Captain advised he had [notified ATC]. Normal deplaning.

Narrative: 3

While picking up trash smelled dirty sock, mildew and damp smell at after lavs and around row 10 mid cabin. I told #1 FA to walk thru and she smelled same thing. A pilot walked thru cabin and smelled it also. #1 reported to Captain.

Synopsis

A320 flight attendants reported Captain notification of a dirty socks odor in passenger cabin. Flight crew executed an expedited landing.

ACN: 1640114 (1089 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 2508.17
Experience.Flight Crew.Last 90 Days : 107.25
Experience.Flight Crew.Type : 1110.47
ASRS Report Number.Accession Number : 1640114

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

After takeoff we received a message from Dispatch to return to gate so the ramp could secure dangerous goods/Hazmat that was loaded . We advised that we were already airborne and discussed with dispatch a plan. The Dispatcher at the time did some research and advised that the ramp personnel that loaded the aircraft advised that the Hazmat/dangerous goods that was loaded correctly and secured. The DG sheet we received on preflight showed to misc Hazmat may with a weight of 257 LBS. Load planning and dispatch advised after we departed that the weight was actually 500 LBS, 250 LBS over what our initial sheet showed. The load planning and final weights seems to be correct, but the DG sheet we received from ACARS did not match that number. We continued the flight normally.

Synopsis

A320 Captain reported receiving Dispatch message stating Hazmat cargo weight was erroneously under reported by 250 lbs.

ACN: 1640035 (1090 of 2908)

Time / Day

Date : 201904

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1640035
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I did not have access to electronic means of reporting, so I gave a verbal report to the hotline in the morning of [date]. Just before the descent to ZZZ, the #1 Flight Attendant reported to me a musty, dirty sock odor in the cabin and both galleys. There was no smell or any other indication in the cockpit. After landing we informed Maintenance and Operations about the logbook entry. My investigation of the galley and cabin revealed no lingering smells. I didn't receive any complaints from the passengers. I completed a report to the company and an Aircraft Smoke, Odor and Fumes report for Maintenance.

Synopsis

A320 Captain reported Flight Attendant notification of "dirty socks odor" in passenger cabin.

ACN: 1638362 (1091 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : EWR

Aircraft : 2

Reference : Y
Make Model Name : UAV - Unpiloted Aerial Vehicle
Airspace.Class B : EWR

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 614
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 614
ASRS Report Number.Accession Number : 1638362

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5746
Experience.Flight Crew.Last 90 Days : 145
Experience.Flight Crew.Type : 5022
ASRS Report Number.Accession Number : 1638357

Events

Anomaly.Conflict : NMAC
Detector.Person : Flight Crew
Miss Distance.Horizontal : 200
Miss Distance.Vertical : 0
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While on a dogleg base to final Runway 22L into EWR, my First Officer reported that we had barely missed colliding with a drone. We were 5 miles west of final at 3,000 feet when a white drone with some red striping came within a couple hundred feet with us. We believe it was at our altitude but a couple hundred feet south of our course. We reported it to ATC and continued our landing without incident.

Narrative: 2

While being vectored for the ILS Runway 21L EWR and on approach control 120.15 we had a very close near miss with a drone. The near miss was the drone passed under our right wing within 50 feet while we were descending through 3,500 feet and on a heading of 130 deg approximately 10 NM North of EWR and apron 5 NM west of TEB. The drone was about 5 feet in size with a football shape and mostly white with a small amount of red on the "nose" of the drone. It appeared the drone was moving slowly to the west away from TEB.

Synopsis

A320 flight crew reported a near mid air collision with a drone during approach to landing.

ACN: 1636685 (1092 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2395
Experience.Flight Crew.Last 90 Days : 139
Experience.Flight Crew.Type : 2104
ASRS Report Number.Accession Number : 1636685
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After turning final for the visual approach to Runway XX at around 2,000 feet at 170 KIAS and flaps at 3, I called for gear down and shortly after, we received an L/G Gear not down-locked ECAM. The Captain (PM, Pilot Monitoring) called for a go-around which I executed and we informed Tower we had a gear problem. I continued the go-around, leaving the gear extended and accelerated and called for the flaps to be retracted to 1. I maintained 170 KIAS and climbed to 3,000 turning to a southeasterly heading (130?) as directed by Tower. I reengaged the autopilot and took over ATC radios as the PM Captain began running the ECAM. Running the ECAM basically entails recycling the gear which was unsuccessful. We then proceeded to the L/G Gravity Extension checklist. This checklist basically involves turning the gear hand crank 3 turns to the mechanical stop. This was also unsuccessful. I reached down and verified the hand crank was at the stop.

At this time, we initiated a "Dispatch Call me" and the Captain began discussing options with Dispatch and [Maintenance Control]. During this, I inquired about fuel (I think we had about 6,200 LBS) and we decided we would burn down to 4,000 prior to landing. I also coordinated a flyby of the tower so they could see the gear. During that flyby, tower said the gear appeared down and straight.

I did not hear much of the conversation with Dispatch and [Maintenance Control], but we then moved to the Landing with Abnormal Gear checklist. The Captain ran this while I maneuvered first in the Tower pattern and then out to an extended final. As the checklist was finishing I mentioned we were getting close to 4,000 LBS, so we decided to take a turn to the runway. Since I had been flying we elected to have me do the landing which basically involves landing and holding off the nose until just prior to loss of elevator authority. We also discussed the need to maintain below 1,000 PSI on the brake pressure during the stop. The checklist also mentions moving passengers aft, but since we were close to full that was not an option.

I re-configured normal full flap and landed, holding off the nose and gently lowering it to the runway. The full stop was uneventful.

After landing, fire and rescue met us as well as [Company Maintenance]. Upon inspection of the nose gear, Maintenance informed us that it was not over center and they could not put the pin in to be towed to the gate. Initially we were going to put jack the nose up, but that was deemed too dangerous with passengers on board and the relatively gusty wind conditions. We then elected to put a jack under the nose to prevent collapse and deplane the passengers via 2L and air stairs and bus them to the terminal. All this coordination took well over two hours.

Synopsis

A320 First Officer reported landing with an unsafe gear indication.

ACN: 1635136 (1093 of 2908)

Time / Day

Date : 201904

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.MSL.Single Value : 3300

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : BOS

Aircraft : 2

Reference : Y
Make Model Name : UAV - Unpiloted Aerial Vehicle
Airspace.Class B : BOS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13675
Experience.Flight Crew.Last 90 Days : 207
Experience.Flight Crew.Type : 8666
ASRS Report Number.Accession Number : 1635136
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
Miss Distance.Horizontal : 5279
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Drone 3,300 feet abeam Cohas ILS 33L BOS within a mile; ATC [was] notified.

Synopsis

A320 Captain reported UAV less than 1 mile abeam the ILS.

ACN: 1634832 (1094 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Angle.Radial : 076
Relative Position.Distance.Nautical Miles : 3
Altitude.AGL.Single Value : 1300

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 30
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : ZZZZZ1
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 330
ASRS Report Number.Accession Number : 1634832

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the First Officer and was Pilot Monitoring for that leg. The Captain and I arrived at the departure gate approximately an hour before departure time. I headed down the jetbridge after the inbound flight's flight attendants came off and spoke with that flight's First Officer and Captain who indicated everything was normal with the aircraft and that we may have some light bumps going over the Rockies. 

The Captain and I performed our preflight duties, and we both noted that there were numerous cleared write-ups in the aircraft logbook on the #2 engine regarding chip detection in the oil system. I noted that both engines had approximately 14.5 qts of oil quantity. 

The preflight, engine start, taxi, and takeoff were all normal. Flex takeoff data was used and called for a flaps 2 takeoff. Reaching acceleration altitude, the PF (Pilot Flying) accelerated the aircraft, reduced the thrust levers to the climb detent, called for flaps 1, and I performed the action. Maybe 10 seconds after that, we had a master warning, fire bell, and ECAM (Electronic Centralized Aircraft Monitor) message indicating a #2 engine fire. The Captain continued to fly the aircraft and called for me to read the ECAM. When the #2 thrust lever was pulled to idle per the ECAM action, the fire indication and ECAM message went away. I looked over all displayed indications on the upper and lower DU (Display Unit) and did not notice any abnormal indications. The PF increased the #2 engine thrust lever once again, and the fire indication returned. I proceeded to read the ECAM once more, and once again the thrust lever was reduced to idle and the ECAM message went away. Our procedures state that if the ECAM message disappears, the ECAM procedure is over, so we did not do any more procedures associated with the engine fire.

At that point, the PF decided to leave the #2 engine at idle thrust since all other indications were normal and I agreed with his decision. I informed ATC that we would be returning to the airport for landing. As ATC gave us vectors to a long downwind, the Captain transferred aircraft control to me and notified the flight attendants, passengers, and Dispatch of our return. Once he got landing data and set up the FMS for the approach, I transferred control back to him and read the approach checklist and overweight landing procedure as we were about 4,000 lbs overweight.

We got vectored for a visual which was mostly uneventful. The ILS was NOTAMed out of service. For some reason the aircraft did not capture the lateral path so there was a very slight overshoot of the final approach path which was promptly corrected. 

We performed the landing checklist and made a flaps full landing per the overweight landing procedure and the PF made a good normal landing. We taxied off the runway and contacted the ARFF (Airport Rescue and Firefighting) vehicles who indicated that there were no abnormalities visible on our aircraft. The Captain called for me to shut down the #2 engine and it shut down normally. We had an ARFF vehicle follow us as we taxied to our gate, and there were no further issues. The passengers were deplaned normally via jetbridge. 

Once the parking checklist was completed I asked the flight attendants if they had noticed anything abnormal such as noises or smells, and they indicated they only noticed something was up due to the aircraft yawing when the #2 engine thrust was retarded. 

Mechanics greeted us shortly after deplaning and we went outside with them to observe the engine, which appeared to have fire damage on the outer accessory area inside the rear cowl.

In retrospect, the only thing I wish we would have done differently would be to ask the flight attendants if they could see anything abnormal with the engine while we were in flight. I think the fact that the ECAM fire indication was so brief and disappeared so quickly when thrust was reduced, I wasn't overly concerned about any massive damage and didn't consider to ask them at the time.

I think the entire crew did a great job, performed their required duties well, and the result was a successful and safe resolution of a serious situation.

Synopsis

A320 First Officer reported returning to departure airport after experiencing an engine fire warning.

ACN: 1634418 (1095 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 18000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Galley
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1634418
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had begun our descent into ZZZ. Was 5 1/2 hour flight from ZZZ1. I was the A Flight Attendant. I noticed a smell in the forward galley. It was slight when I first noticed it. I am new to flying on the Airbus and there are many new sounds, etc., that are unfamiliar to me. So in my mind I think originally it was just a unique smell to the Airbus. However, once we passed 18,000 and pilots made arrival notice to prepare cabin, the smell began to get stronger and I began to have [a] headache. At 10,000 chime I was called by pilots and asked if I could smell odor in cabin, as they could smell in flight deck, and the smell was stronger. I told them that yes I could smell odor and tried to describe. They said to get crew secured and that we would be on ground in 5 minutes. The B and C were mid cabin doing final cleanup. I walked to mid cabin and could still smell the odor, but not as strong as in forward galley. Told my B and C to get seated and informed them what pilots had stated and asked if they could smell odor, and she said yes. We all took our jump seats. As we were landing, sitting in jump seat in forward galley, I tried to see where smell [was] coming from, like air vent, etc., but I could not diagnose. Once on the ground and deplaned, crew all debriefed with pilots of the event and what we all smelled, etc. At this point I now had tighter chest and lungs and felt unstable a bit, but I did not get sick. No passengers made any comment to me regarding smell during [deplaning]. Continue to research what is causing these events to happen on the Airbus.

Synopsis

A320 Flight Attendant reported strong odor in cabin and experienced health issues.

ACN: 1634183 (1096 of 2908)

Time / Day

Date : 201904

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 499.27
Experience.Flight Crew.Last 90 Days : 116.93
Experience.Flight Crew.Type : 499.27
ASRS Report Number.Accession Number : 1634183
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Primary Problem : Aircraft

Narrative: 1

Unable to operate Aircraft X [from] ZZZ due to multiple MEL's [Minimum Equipment List] on the fuel system.

Synopsis

A320 Captain refused aircraft due to multiple MEL fuel problems.

ACN: 1633435 (1097 of 2908)

Time / Day

Date : 201904

Place

Altitude.AGL.Single Value : 600

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 152
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1633435
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

In final turn for visual Runway 29 at night at 600 ft pointing at North Bridge fully configured exactly on vertical profile, ATC issues low altitude alert followed by EGPWS caution obstacle call out. Bridge tower was in sight so we continued.

Synopsis

A320 Captain reported receiving low altitude alerts on approach and continued with obstacle in sight.

ACN: 1633420 (1098 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.AGL.Single Value : 600

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 14
Experience.Flight Crew.Type : 1064
ASRS Report Number.Accession Number : 1633420
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Rolling out on final over CHUMR in the descent to cross at 500' we simultaneously got a low altitude alert call out from tower and an aural "Caution Obstacle" from the jet. We were on the stadium visual arrival to runway 29 due to strong and gusty winds out of the west. We had the RNAV 29 loaded flying a manage-managed approach with autopilot and auto thrust OFF. We were slightly above the vertical profile and on the lateral course at the time of the incident. I immediately leveled off until the situation was reconciled and was able to recapture a normal visual glide path. The approach was continued to a normal landing.

Synopsis

A320 First Officer reported EGPWS alert during approach.

ACN: 1633253 (1099 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Galley
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1633253

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Evacuated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I noticed a very strong fishy smell when I walked into airplane. I didn't think anything of it because airplanes always seem to have a funky smell, especially near the lavatories. We boarded the airplane. Since I was the C, I was prepping the galley and watching the aft doors because catering came while we were boarding. The B said passengers were commenting on smells, but I couldn't smell anything in the aft galley area. The next thing I know there was an announcement from pilots saying we were deplaning because of the odor. We deplaned, and when I went up front I didn't notice the smell. Only once we were in the jetway for a while and reentered the plane could I smell it again. I don't have a good sense of smell, but again, it smelled like rotten fish. I need to be more in tune with smells and notice these things before boarding. I honestly just thought it was the forward lavatory when I walked into plane.

Synopsis

A320 Flight Attendant reported strong fish smell during preflight. Subsequently, Captain ordered passengers to deplane.

ACN: 1633055 (1100 of 2908)

Time / Day

Date : 201904
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class B : DEN

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 122
Experience.Flight Crew.Type : 1653
ASRS Report Number.Accession Number : 1633055
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were cleared to descend via the PURRL2 RNAV STAR into DEN and were told to expect Runway 35R. As the pilot monitoring, I set 9,000 feet in the altitude window and the pilot flying engaged the managed descent mode and we started our descent. Just prior to crossing the waypoint DANDD, the pilot flying was entering data into the secondary flight plan in the hopes that we would be approved to land on Runway 35L. He accidentally activated the secondary flight plan which dumped the PURRL2 STAR and the airplane failed to make the required turn at DANDD. Also, because the altitude window still had 9,000 feet the airplane continued to descend. The pilot flying is very new to the Airbus (still a high minutes Captain) and he struggled to identify what had just happened. He was attempting to re-load the arrival into the FMC when I noticed what was going on and asked him what happened. He replied that he didn't know what happened but I could see that the RNAV arrival was no longer loaded in the FMC and that the airplane was no longer flying the PURRL2 as we had been cleared. I took over FMC programming duties as the Captain placed the airplane in heading mode in an effort to return us to the course but ATC called us stating that we were 2 miles north of course. By this time, the PURRL2 Arrival had been successfully re-loaded into the FMC but we failed to identify that the airplane was still descending below the constraint on the arrival. Once again, ATC called to state that we were below the required altitude and they vectored us off of the arrival giving us a heading and altitude to fly. The entire deviation lasted approximately two minutes but enough time to fail to make a required turn and fail to level off at the correct altitude. The remainder of the arrival to Runway 35R was uneventful.

Synopsis

A320 First Officer reported track and altitude deviations occurred on the PURRL2 RNAV arrival into DEN when the FMC was accidentally mis-programmed.