ACN: 1631506 (1101 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1631506

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[I was] advised a flight attendant experienced fume event.

Synopsis

A320 Captain reported being notified a Flight Attendant had experienced a fumes event.

ACN: 1629905 (1102 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 6981
Experience.Flight Crew.Last 90 Days : 78
Experience.Flight Crew.Type : 5834
ASRS Report Number.Accession Number : 1629905

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 11543
Experience.Flight Crew.Last 90 Days : 209
Experience.Flight Crew.Type : 6113
ASRS Report Number.Accession Number : 1629932

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

First flight of the day checks. I was the PF (Pilot Flying). Normal brief, push, start and taxi to Runway XXR. On TO roll PM (Pilot Monitoring) noticed that the Number 1 Engine EPR was higher than Number 2 Engine EPR reading. All other engine indications were normal. On climb out and at 800 feet CL detent was selected. No change to the EPR on the Number 1 Engine. Number 2 Engine responded normal. ECAM directed to Engine Number 1 master switch OFF. Normal shutdown. PF confirmed correct master switch. [Coordinated with ATC]. Vectors for a 15 mile visual backed up with ILS Rwy XXR. TEST brief with FA's (Flight Attendants). No evac. Text to [Dispatch, advising of the single-engine return]. ARFF (Aircraft Rescue and Fire Fighting) met aircraft on runway. Nothing noted. Followed aircraft to gate.

Narrative: 2

During the takeoff roll I noted that the EPR on engine 1 was well above what it should have been and also well above what engine 2 EPR was. All other engine parameters were normal. We determined that the safer course of action was to continue. After takeoff when the PF (Pilot Flying) set the thrust levers to the climb detent, there was no change in the engine 1 EPR. In fact, the EGT and N1 indications had now started to climb. The beta target on my PFD also indicated a yaw to the right, indicating engine 1 was producing much more thrust than engine 2. We then got an engine ECAM that related to fuel. I'm not sure what the exact title of the ECAM was. At the same time the ECAM popped up the PF had started pulling the engine 1 thrust lever to idle. There was no response from the engine. The next item on the ECAM was engine 1 master off. We confirmed I was about to shut down the number 1 engine and then completed the ECAM. We [coordinated with ATC] and returned to the field and then to the gate.

Synopsis

A320 flight crew reported engine anomaly on climbout resulting in shutdown and return to departure airport.

ACN: 1628424 (1103 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1628424
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was working Number 1 position. Approximately 25 minutes from landing, I noticed a fume smell in the cabin as I approached the first class/main cabin divider bulkhead. I called the Number 2 Flight Attendant and asked her if she smelled anything. She did, but was unsure as to what it was. We landed without incident and I informed the Captain as soon as he was available. As ZZZ Maintenance, etc., were conducting their tests, they kept asking us to come onto the aircraft to see if we still smelled the smell. WHY WOULD YOU GO BACK ON THE AIRCRAFT, TAKE A BIG WHIFF OF A KNOWN TOXIN/EXPOSURE/POISON AND THINK THAT IT IS SAFE TO DO SO?! I did not go back on the plane except to obtain my bags. We were all, except the Captain, evaluated on scene in operations area by EMTs. No one went to hospital.

My symptoms [were] light headed/dizziness, unable to complete thoughts/sentences, headache localized in the middle of forehead, burning/scratchy throat, malaise, tingling of arms and legs, somewhat general cognitive dysfunction. It took GREAT amounts of energy to focus on basic known tasks. In so much that if I had to evacuate the aircraft without assistance, I feel that I would have had difficulty due to the above symptoms. Results of EMT evaluation. My blood pressure was 147/9 over 97. My normal range is around 122 over 82-85. My blood glucose at this evaluation was in range of 107. When I was evaluated [one month prior] in the airport clinic, my blood pressure was approximately 120 over (less than 97). I have followed protocol for this event in regards to triage and medical evaluations. It was notated to us that this aircraft had at least 8 prior known incidences involving cabin odors. As well, the "packs" and APU had recently been replaced.

Synopsis

A320 Flight Attendant reported fumes in the cabin. Captain requested Maintenance and paramedics on landing.

ACN: 1628242 (1104 of 2908)

Time / Day

Date : 201903

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 13552
Experience.Flight Crew.Last 90 Days : 168
Experience.Flight Crew.Type : 13552
ASRS Report Number.Accession Number : 1628242

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 1952
Experience.Flight Crew.Last 90 Days : 73
Experience.Flight Crew.Type : 1952
ASRS Report Number.Accession Number : 1628235

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Dispatch
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were told after takeoff that we had dry ice in cargo pit 2, we did not get any notification by way of a DG form, [and] it was not noted on the final weights or on the service form. We were told it was less than 4 lbs, but might have been listed as less than 1lb. We had no animals in the pit.

Narrative: 2

After takeoff we were informed via ACARS that we had dry ice in cargo pit 2. We received no prior notification on a DG form, it was not noted on the final weights or on the aircraft service form. We were told it was less than 4 lbs but might have been listed as less than 1 lb. There were no animals in cargo pit 2. There were no issues as a result of this.

Synopsis

A320 flight crew reported receiving Dispatch notification during cruise of Hazmat [Dry Ice] loaded on aircraft without providing NOTOC [Notice to Captain] prior to departure.

ACN: 1627955 (1105 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1627955

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 21000
ASRS Report Number.Accession Number : 1627934

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon descending into ZZZ, the lead FA noticed an odor and asked the cabin crew that also took note of the fume. Upon landing I quickly deplaned and was not aware. Upon returning to the plane I also sensed the strong fume mostly in the middle cabin but it migrated to the entire plane and cockpit. The lead FA was visibly ill. I developed a stomach ache was not sure of the cause. We had paramedics check us and my blood pressure was very elevated. I chose to not go to the emergency room. Now I am losing my voice and have lung irritation and coughing. I will be following up with a doctor. Our Captain did not seem affected.

Narrative: 2

Upon deplaning at ZZZ I was contacted by my lead Flight Attendant about a possible fume event. He related that it occurred during descent which prompted him to contact the flight attendants in the rear of the aircraft. They did not detect anything and he said the fumes appeared to dissipate. While at the gate the fumes/odor escalated and I subsequently contacted Maintenance and my company operations. While I did not personally observe or sense the fumes, it was confirmed by my First Officer, a station manager and numerous ground personnel. After hearing that two of my flight attendants had adverse reactions and my First Officer was not feeling 100 percent, I summoned medical personnel to attend to my crew. After a couple of hours and in coordination with IOC duty pilots and a ZZZ1 Chief Pilot, and after I was assured that my crew was feeling better we left to go to the hotel. Possibly emanating from an air conditioning pack but unsubstantiated.

Synopsis

A320 flight crew reported requesting Maintenance and paramedics after gate arrival due to strong fumes in the cabin and experiencing health issues.

ACN: 1627917 (1106 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Taxi

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
ASRS Report Number.Accession Number : 1627917
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Other
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On engine start, the #1 ENG began spilling a large amount of fuel. The aircraft spilled approximately 300 gal over a 1 mile stretch of taxiing. On throttle up for T/O (Take-Off), the leak stopped and allowed the flight to continue without interruption. When the crew got on the ground, the PIC (Pilot in Command) called and reported a 2300lb overburn. I looked at the winds aloft and saw a reported 150kt wind at FL340. With a large internal overburn and the winds aloft, and no other info regarding the previous leak, I assessed that the fuel burn, while excessively large, was still within the realm of possibility given the circumstances. I told the crew to monitor their fuel burn for the return leg, as there would be a tail wind to help. When the flight arrived in ZZZ, ARFF (Airport Rescue and Fire Fighting) was waiting for the flight. Crew was instructed to taxi to the end of the RWY and shutdown. When the aircraft eventually made it back to a gate, the [Fire] Chief reported the fuel leak, and sent the PIC the pic, who then forwarded it on to [Maintenance Control], where I was finally able to see it. At no point was the fact that an aircraft was leaking fuel in such fashion, reported to either Dispatch, or ATC to relay to the crew, even though ZZZ Ops had to have known, as ARFF knew that specific tail would be returning to ZZZ, and the approximate time they would arrive.

Ensure all ground personnel know to report to flight crew if a fuel leak is suspected on push back/engine start. If unable to notify crew, or suspected after out of communication with flight crew, relay to OPS if a fuel leak is suspected. Furthermore, ensure Dispatch or [Operations Control] is made aware of the issue in order to establish communication with the Flight Crew to better assess the issue. If unable to contact Dispatch or [Operations Control], contact Station TWR/Cab to relay message to flight crew about possible fuel leaks.

Synopsis

Maintenance Control reported an aircraft experienced a fuel leak causing fuel quantity to drop below flight plan limit.

ACN: 1626989 (1107 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 145
Experience.Flight Crew.Type : 703
ASRS Report Number.Accession Number : 1626989
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 274
Experience.Flight Crew.Last 90 Days : 76
Experience.Flight Crew.Type : 274
ASRS Report Number.Accession Number : 1627821
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Aircraft was pushed back and towed out to the end of the alley. Engine 1 was started. Parking brake was set per push crew request and per SOP the accumulator pressure was noted and found to be at appx. 2000 lbs. Push crew was requested to remain hooked to the aircraft since brake pressure was not "normal." Maintenance was notified via ACARS and over the radio. The local maintenance controller asked us to call [Maintenance Control] and verify with them that a return to the gate was required to service the brake accumulator at a minimum. [Maintenance Control] concurred and a return to gate was coordinated. Aircraft was towed back to the gate.

Eventually mechanics made it to the aircraft and the accumulator was serviced and seemed to be holding pressure. Since the brake accumulator is part of the hydraulic system, the crew checked hydraulic levels on the ECAM [ Electronic Centralized Aircraft Monitoring] 
as the mechanic was leaving. The yellow system now showed greatly over serviced and the mechanic was called back and shown the problem and was asked to correct the issue that the servicing of the accumulator seemed to have caused.

At this point all hell broke loose and the serious lack of understanding of Airbus hydraulic systems by the mechanics at [this] gate went on full display. Instead of simply de-pressurization of the yellow system and draining the over-serviced system, a process that takes normally 15 minutes or so, the mechanics attempted multiple items in what appeared to be an attempt to transfer fluid between systems, like the technique used to transfer fluid between systems on the 737. Flight controls were cycled and "dual input" calls were heard coming from the flight deck. At some point the yellow system was pressurized intermittently via some other source than the flight deck electric pump control, and pressure on the yellow system could be witnessed to rise to 400-600 lbs and a bit of a whining sound could be heard on the flight deck as this occurred. This is also something that I have never witnessed on the Airbus and would have no effect on yellow system quantity to the best of my systems knowledge and might in fact be a very unsafe thing to do.

[Maintenance Control] was consulted again as what was going on was nothing that I had ever seen done to fix a simple hydraulic over service condition. [Maintenance Control] agreed that whatever was going on made no sense and that all that needed to be done was drain the fluid from the over serviced yellow system. Draining a hydraulic system requires absolutely no flight deck access by competent mechanics other than to verify readings and to put out placards to ensure hydraulics aren't operated. I've seen it done multiple times at multiple other stations. Why did we need mechanics in both flight deck seats manipulating who knows what in a vain attempt to transfer fluid between systems?

Eventually, at some point the decision was made to give us another aircraft and as we were on the jet bridge to leave the agent opened the door and said "we're boarding." What? No, I'm not satisfied with the maintenance status of this aircraft. Don't board. I called the [Flight Operations Manager] and was told that the aircraft had been signed off and that I couldn't refuse an aircraft that had been properly signed off by licensed [Company] mechanics. Well, whoever told the [Flight Operations Manager] that the aircraft was signed off was lying, as when I went to flight deck, the gear door ECAM was still present and the brake pressure was at 0. 

I have no idea who at maintenance was pushing to sign off an un-airworthy aircraft, or who chose to lie and say that an un-airworthy aircraft was signed off, but this was clearly a majorly unsafe and illegal situation.

Furthermore, not once during the entire operation did I see a mechanic at that gate with an iPad out referencing the iPad or the Tech Order. It seems to me that there was a very cavalier attitude amongst the mechanics at that gate and that really exacerbated the unsafe and incompetent handling of the hydraulic over servicing.

After I pointed out to the [Flight Operations Manager] that the aircraft was actually not signed off or airworthy, the decision to change aircraft was again put in place. I have no idea what ever was finally done to the original aircraft, but I shudder to think of how it may have been eventually signed off, and if that sign off was correct or legal.

Narrative: 2

After a normal pre-flight and pushback, the brake accumulator gauge was not in the green upon the setting of the parking brake after completion of the pushback. The Captain noticed the problem after when confirming that the "brakes set, pressure... not normal".

He did not allow the tug/ground crew to go anywhere and we coordinated with ramp to stay where we were while evaluating the problem and possible solutions. While he was on the phone ZZZ line MX (Maintenance), the gauge dropped from out of the normal range to zero. We wrote up the problem and returned to the gate. 

The real issue started when the first mechanic boarded. He immediately suggested deferring the gauge and said it was an item in the MEL. While this is true, the problem as we saw it wasn't the gauge but the accumulator not holding pressure. It took some convincing of the mechanic that they actually had to troubleshoot the problem instead of pencil whip it. He left the flight deck an unhappy camper. 

Later, as we were waiting and trying to coordinate with ZZZ line MX, a second mechanic came up and tried to convince us of the same thing. He turned on the electric pump, which pressurized the yellow system and recharged the accumulator. He did stay on the flight for the 4-5 minutes it took for the accumulator to reach zero again. At this point it seemed he understood the problem was in the accumulator. He said he would service the accumulator. 

This apparently was accomplished. The accumulator pressure, with the electric pump off and no other hydraulic system on, the gauge read in the green. The mechanic came up we said it was good and that was that. 

We then checked the hydraulic fluids. The yellow system fluid had been over serviced so much that the indicator was literally touching the fire valve icon on the systems display. We asked maintenance to bring the yellow system back to normal levels. We saw this the night before and it took around 5 minutes to accomplish, we were expecting the same. 

After 15-20 minutes, the mechanics came up to the flight deck to check the fluid levels. Nothing had changed, except the brake accumulator valve now read zero, again. They then started turning on / off packs and hydraulic pumps, and at one point during this visit the mechanic got into the left seat and started manipulating the side stick. 

Nothing worked. The mechanic left the flight deck. 

The brake accumulator was then serviced again back into the normal range. The mechanics returned to the flight deck. I was in the forward galley at the time, allowing them room to do whatever it was they needed. Twice I heard the "DUAL INPUT" flight control aural warning, meaning they were manipulating the side sticks, but this time it was both. 

We had started the process of swapping aircraft, the Captain was on the phone with the [Flight Operations Manager] out on the jet bridge. The first mechanic who showed up stormed out the jet bridge door muttering something about "He's just got an axe to grind". 

At this point the Captain told me that it had been relayed to the [Operations Control] that the aircraft had been signed off. That was interesting in that the yellow system was still over serviced. We collected our things and headed to the new aircraft.

Throughout this entire process, one that took conservatively 60 minutes, at no point did I see a mechanic with an iPad, iPhone, or paper documentation with a procedure to service either the accumulator, the yellow system, trouble shoot the problem or test to see if it was fixed. Even at the initial flight deck visit, where the mechanic told us he was deferring the gauge, we had to look up the MEL as he didn't have the number or a way to look it up with him.

Synopsis

A320 flight crew reported maintenance technicians improperly servicing the hydraulic system.

ACN: 1626559 (1108 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1626559

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon descent into ZZZ (after "Prepare for Landing" and during sterile cockpit), myself (#4) and the #2 were in our jumpseats in the aft galley and began smelling something very strong. The #2 asked me, "do you smell that?" I replied, "Oh my god, I do! I was about to ask you the same thing." It smelled chemically and like dirty socks. We both thought it was someone's feet or our own and joked about it until we started feeling "strange." At that time, I heard a guy coughing in the cabin and I looked up thinking to myself, "dang, he must be smelling it too." When I looked up, I saw the #1 walking through the cabin to the aft galley. When she got to the galley, she asked, "do y'all smell that dirty sock smell?!" We both said, "yes! We have been talking about it!" None of us had ever smelled anything like it before and the #1 said it was making her feel like she was going to pass out. She asked if she should call the flight deck and let them know. I told her yes. She called the flight deck and they replied (according to her), "it's just south Florida." Nothing else was done about this event and we didn't feel the best after the flight. Our flight was already over 4 hours delayed and the pilots were reassigned. I would like to hope this didn't affect their decision to not follow procedures due to them being tired. We felt "strange." I felt nauseated when I got to my hotel room, the #1 and #2 both expressed dizziness to me at times. It wasn't handled by procedures we don't feel like by the flight deck.

Synopsis

A320 Flight Attendant reported strong odor in cabin and subsequent health issues during initial approach to landing.

ACN: 1624938 (1109 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1624938

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1625060

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately 15-20 minutes after level off at cruise (FL370) the "A" FA (Flight Attendant) called with reports of a light acrid odor around the L1 door. The CA (Captain) queried her for more specifics (related to identifying type of odor) and they agreed to monitor for changes. The CA and I reviewed the "odors" flight ops bulletin and discussed the potential of a developing fumes event. After a few minutes the "A" reported an increase in intensity with eye irritation. We decided that this was likely a fumes event and proceeded to run the "fumes" checklist from the QRH, selecting Pack 2 off as we had no indication or any fumes in the cockpit. We also advised Dispatch of a potential need to divert and discussed ZZZ or ZZZ1 as possible choices. Dispatch advised ZZZ as closest (which we already knew). After the required 5 minute interval the CA queried the FA and she reported the condition had worsened. We then switched pack configuration per the QRH with Pack 2 on and Pack 1 off. Just before that 5 minute interval ended the FA reported continued worsening with fumes now affecting the passengers in the first few rows of the cabin. We made the decision to divert to ZZZ. The CA advised ATC. As I was already the Pilot Flying the CA had me continue as such and gave me the radio communication duties as well so that he could coordinate better handle the growing non-flying duties. ATC gave us priority handling and the remainder of the descent and landing was uneventful. Emergency equipment was offered and I accepted as well as requesting EMTs to be waiting at the gate. After landing we decided the fumes were not at a critical level so we continued to clear the runway and proceeded to the gate. We had not received any gate confirmation and were unable to contact station ops on the radio so the CA decided to proceed to our "normal" gate and after a few minutes delay station personnel arrived. We asked all passengers to deplane and the EMTs met/treated FAs and passengers as needed. We at no time noticed any fumes on the flight deck during the entire event. Suggestions: fume event as described. We diverted to a low use station and were lucky station personnel happened to be there. Dispatch should have offered more suggestions for possible divert choices based on operational factors other than closest airport. In hindsight, ZZZ1 was probably a better choice and not any further away (time wise) as we were too high/close to descend straight in to ZZZ. Also, we were unable to contact station ops once on the ground. In my experience, this has been an ongoing system wide systemic problem and in my opinion one day our collective luck will run out.

Narrative: 2

During cruise flight at FL370 I received a call from our A Flight Attendant (FA) informing me she smelled a smoky burning smell by the L1 door and in that general vicinity of the aircraft. She had confirmed it with another FA and while they could not identify the source it was prevalent. I inquired about the possibility of fume/odor event and initially, having experienced several of these type events herself, the A FA believed it was not that kind of smell. I told her my FO (First Officer) and I would discuss it and review some procedures and get back to her in a few minutes but for her to keep me posted on events in the cabin. I looked at the Navigation Display and determined we could divert to ZZZ or ZZZ1 as they were up ahead just in case things took a turn for the worse. My FO and I then reviewed the memo on odor decision making tools and the QRH for odor/fume event. We discussed the possibility of setting up a bathroom break to check the cabin for ourselves. I also sent a message to Dispatch giving them a heads up about the possible situation. About that time our A FA called back and informed us the smell was getting worse, it was now a smell typical of fume events, and confirmed by another FA experiencing symptoms such as burning eyes, difficulty breathing, etc. I informed her we would be diverting to ZZZ and to keep me posted. We continued to run through the QRH procedure in the cockpit which we completed, [notified] ATC and diverted to ZZZ where we landed without incident. During the descent the A FA called me several times with updates that the smell continued to get worse and that passengers were getting sick in the first few rows. The smell appeared to be isolated to the first few rows of the aircraft but during the descent it began to permeate the entire aircraft. I told her to do whatever she needed to do to take care of herself and the passengers and FAs. Once we arrived at gate XX in ZZZ passengers were informed to deplane and to seek medical attention with paramedics who were waiting in the terminal if they felt it was necessary. Suggestions: while there are certainly a number of lessons I learned from this event there's one thing of importance which I want to mention here and will pass on to various company officials and it's in regards to guidance received from Operations Control. As I mentioned, when I first learned of this event possibility I sent a message to Dispatch as a heads up. I received a message back telling us ZZZ was a better diversion point than ZZZ1 as it was closer. In that sense I'd agree. But what we didn't know and had no way of knowing if there weren't going to be any [company] people available in ZZZ to meet the flight! After we had deplaned we conversed with the ZZZ station manager who informed us we were "lucky" they happened to be there. [Company] has no operations in ZZZ during that time of day and it's only by happenstance (the actual reason I can't remember) that anyone at all was there to work the flight when we arrived. In fact, we had to wait a few minutes for people to show up (we had received a message from Dispatch they were notifying ZZZ of our arrival). So in other words, we could have arrived in ZZZ with an aircraft expecting support and been left stuck out on the ramp with nowhere to go and plane load of people in need of medical attention. The ZZZ station manager informed us that ZZZ1 has a full service operation all day and would have been a better place for us to divert. Now, I'm not trying to throw our Dispatcher under the bus here and I'm not blaming him for anything at all. But it seems that ZZZ wasn't that much further and that's the kind of information the Operations Control should know and provide to us when we make our decisions. We the pilots had no clue about the station staffing situation and in a time compressed situation didn't really have time to contemplate the merits as such when our concern was getting the airplane on the ground and the people off the plane. ZZZ is a major city served by [company] and this could have been really ugly if we'd arrived with no station personnel to meet us. Anyway, just something to consider.

Synopsis

A320 flight crew reported Flight Attendant informed them of acrid odor in the forward cabin area. Flight crew executed a diversion due to fumes intensifying.

ACN: 1623838 (1110 of 2908)

Time / Day

Date : 201903
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 8000
ASRS Report Number.Accession Number : 1623838

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 23000
ASRS Report Number.Accession Number : 1623852

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected.Other 
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Captain set thrust levers to TOGA (Takeoff Go Around) for takeoff. At approximately 70 knots we received an Engine 1 compressor vane warning on the ECAM. Captain called My Aircraft and rejected the takeoff. I, Pilot Monitoring, called out 2 in reverse. Captain stopped on the runway. I notified ATC we rejected the takeoff. We shut the engine down and returned to the gate. Reject was initiated due to the ECAM Warning. Followed training and rejected the aircraft safely.

Narrative: 2

On takeoff roll with TOGA (Takeoff Go-Around Power) power set, we received an engine 1 compressor vane fault at approximately 70 kts. I verbalized "reject, my aircraft" and stopped the aircraft on the runway. I told the passengers to remain seated. ATC was informed. We taxied back to the gate. 

I made 2 AML (Aircraft Maintenance Log) entries: one was for the underlying ECAM and the other was to record the speed at the time of the reject. Maintenance was called. We switched airplanes and went to [Destination]. Cause due to Compressor Vane fault.

Synopsis

A320 flight crew reported a rejected take-off due to engine compressor vane fault message.

ACN: 1623600 (1111 of 2908)

Time / Day

Date : 201902

Place

Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Items Involved : Work Cards

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1623600
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1623602
Human Factors : Training / Qualification

Person : 3

Reference : 3
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1623603
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

I was asked to be a safety observer for a slide change on door 2L on Aircraft X. I mentioned I had never done the task before, so we went through the slide checklist and removal task card together to check if we had to install anything other than the bottle pin. Upon installing the bottle pin and attempting to show one of the technicians where and how to take the girt bar out. As soon as the handle on the door was touched, it began hissing and almost pulled one of the technicians out of the plane. The door bottle blew the door wide open and the slide deployed.

Please make a sign off for the door assist bottle pin to be installed. It is very easily overlooked if being read by an inexperienced person.

Narrative: 2

Following task card for slide change on L/H #2 door on Aircraft X. I was the technician that was arming and opening the door while my co-worker was going to disarm the girt bar. The door assist bottle started to hiss as soon as I pulled the door handle open. I did not pin the bottle but verified the pin was in.

Clear instruction on how to pin and disarm the door assist bottle for Airbus aircraft task card like the Boeing aircraft slide work instruction.

Narrative: 3

We first got all of our paper work together in order. The task card, escape slide render safe check list as well as reference material for the L2 door on Aircraft X. We had one technician read out the task card and signed as we went as I worked alongside the main technician on the job. My role was to take the shroud off to reveal the slide and then insert the safety pin for the slide. Then I waited to take the girt bar and the other technician lifted the door handle slightly and heard the air release and the slide was inadvertently deployed.

Clearer steps on paper work.

Synopsis

Maintenance personnel reported inadvertently activating the aircraft cabin door slide while working on it.

ACN: 1623556 (1112 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1623556

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1623022

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft (tail) was a RON and had been pre-deiced. The entire crew arrived early, (1 hour prior to departure) introduced ourselves, and talked about the flight. We briefed security, weather, MELs, and most importantly safety. Boarding was uneventful. The weather in ZZZ1 was a 1/4 sm and VV002. We asked Dispatch for an ALT as there wasn't one on our release. We also had to remove an additional MEL on our release that was previously cleared by Maintenance. We ended up leaving 13 minutes late waiting for a printed out 2nd release. Pushing back I asked my First Officer (FO) to start engine Number 1. He started engine Number 1 while I monitored the push and talked to the ground crew. The FO immediately brought to my attention the large plume of smoke coming from the engine during the start sequence. All engine parameters were within limits. After the start there were no unusual smells or indications. We attributed the smoke to a possible over-spray of deice fluid onto the engine (not uncommon), as well as it was the first flight of the day, and it was cold outside. We taxied out on 1 engine per suggested procedures via spot XX to runway 34R. In sequence for takeoff engine Number 2 was started and the APU was shutdown. We ran all appropriate checklist. There were no abnormal instrument indications. There were no fumes or unusual odors prior to flight. I was the Pilot Flying this particular leg. Taking off Runway XXR was uneventful until acceleration altitude. Shortly after I brought the thrust levers back from takeoff to climb detent, I noticed an unusual smell. It was very light at first. I asked my FO if he could smell anything. He initially couldn't smell anything different than normal. We were extremely busy on the departure and cleaning up the aircraft. We were handed off to Departure and were told to make a left turn which was opposite of the departure procedure. It was somewhere around this time that my First Officer told me that he could smell something as well. It was faint at first and not distinguishable. The odor grew stronger and began to irritate my throat. If I had to choose it was more chemically than the musty sock smell. I had my First Officer don his O2 mask and call the back to see if they had anything unusual to report. I continued to fly the plane and was handling the radios. I turned the AP1 on at this time to reduce the workload and focus on the fume problem. The Flight Attendants (FAs) had nothing to report. We told them to let us know if anything changed. We began to troubleshoot the problem turning the bleed fans to high. It seemed to me that the smell was dissipating as time progressed. Initially I thought it was one of those smells that come and go just as fast. A good example of this is a familiar smell flying through the thin layer going into ZZZ2. We had FA B come forward to the cockpit to give us a second opinion on whether the odor was still lingering. When she came upfront she immediately noticed the smell. I had a conversation with her I'm a bit foggy as to everything she or I said. I do remember FA B saying it hurt her throat. She left the cockpit and I donned my O2 mask. I don't recall the exact amount of time or altitude this all transpired as we were obviously busy and the aircraft was light and climbing fast. I did occasionally remove it for a few seconds to see if the smell dissipated. After FA B left, we discussed the QRH and whether or not to return to ZZZ. My throat was pretty sore, I felt light headed, and we both had O2 mask on. The safest thing to do was return to ZZZ. The First Officer made the request to ATC [and] they gave us an altitude and heading. They asked if we were requesting priority handling. My First Officer acknowledged with FOB, SOB, and the nature of our situation. I called the back and told our Flight Attendants that we [notified ATC] and were returning to the field because of the fumes in the cockpit. I was talking through the O2 mask and I believe I said there weren't going to be any PAs explaining everything to the passengers from us. The FO called the company enlightened them on our situation and got us a gate. I was pretty foggy at this point even with the O2 mask on. My throat hurt and I was coughing. My First Officer conveyed that he felt better than I did and offered to be Flying Pilot. I transferred controls to my FO to be the Flying Pilot. ATC gave us direct ZZZ and eventually cleared us to land for Runway XXR. I ran the checklist and handled the radios. The First Officer did a flawless approach and landing. We exited the runway and taxied at a steady pace to spot XX and cleared to gate XX. Arriving at the gate we had no rampers and the spot was blocked by a row of baggage carts. I had both engines running and every light on to hopefully get everyone's attention. Still having my oxygen mask on I opened my window stuck my head out and waved rapidly at the rampers at the next gate over. They saw me and quickly came over to clear the area and parked us. We pulled in and shut down. Ran the checklist and opened the cockpit door. The emergency crews were already there. We collected our bags and followed the paramedics to the gate area where they evaluated us. The three of us all went to the hospital for further evaluation. We all tested positive for high CO levels in our blood. Looking back at the event, it seemed to me a few times that the odor was dissipating or gone. I felt that the conditions were the most toxic shortly after I reduced thrust from takeoff thrust. I realize today that I was completely acclimated to the odor and what I was sensing was inaccurate. Unlike an engine indication or a warning light there is no way for a pilot to definitely determine that the problem still exist with this particular fume event. I remember looking at the QRH saying "but the odor is decreasing." I've had several "fume events" at this airline and each one unlike the first. The Flight Attendants did an excellent job. They understood the severity of the situation and passed the necessary information along quickly. We called them several times and they were very straight to the point and factual. When the cockpit door opened on the ground and we exited the aircraft not one passenger was standing. Without a doubt they had positive control on the cabin. FA B coming forward in helping us asses the cockpit fumes was crucial. She remained calm, told us the conditions in the back, and immediately went into what she smelled and how it was affecting her. I lastly want to commend my First Officer. He offered several solutions as what he thought the problem was. My FO did all that he could to keep the workload down and assist me as well as reminded me if something was missed. We did deal with the situation together. We were lucky that we had flown with each other for the last 3 days and understood one another. We kept the cockpit communication open and flowing. Finally after deciding to return to ZZZ my First Officer realized the condition I was in and offered to assume control of the aircraft. I agreed with him as that was the best decision. He has good character, makes the right decisions under pressure, and is a pleasure to fly with. He will make an excellent Line Captain. Maintenance returned the aircraft back to service after performing the cabin air troubleshoot form. They discovered deice fluid on the APU inlet. They deactivated the APU and returned to flight where the next crew had another fume event. I disagree with Maintenance's solution to the problem. We did not have the APU running when we had the problem for one. Secondly the chemical smell that I experienced wasn't the typical deice smell. Does boiling deice fluid produce CO? I'm not sure? I've had fume events before and the air filters were replaced. I see nothing in this report that the filters were checked or replaced. I've heard countless remarks from Maintenance on the line that we are using different oil in our APU and not changing our air filters regularly. What happened to the carbon filters that we were getting? What happened to Maintenance running the bleed on the ramp to high temperatures for burn off procedures like [other airline]? This is my 3rd fume event. This is too common of an occurrence and the only way to prevent this is by actually cleaning the bleed systems, changing the air filters regularly, and tracking the aircraft fume events between airlines.

Narrative: 2

Flight was our first flight of the day and the fourth and final day of our trip. It was also the first flight for the aircraft after remaining overnight in ZZZ. The inbound flight arrived in ZZZ at approximately XA00 local the same day. Upon arriving to the aircraft, I noted a "Pre-Deicing Notification" form on the center pedestal indicating Type I de-ice fluid had been applied to the wings and tail. I also took note that maintenance had likely been at the aircraft as both L/R engine oil quantities were indicating 18.5 quarts. All three hydraulic reservoirs were at or near the top of their respective scales as indicated on the system synopsis pages. I performed the outside pre-flight inspection and did not note any physical damage or abnormalities. I did notice a medium quantity of residual de-ice fluid on the lower portion of the main landing gear assemblies as well as the main tires.

We were cleared to push-back. At some point past the hold point, Captain asked for the #1 engine to be started. I started the #1 engine and because we were still being pushed back, I noticed a large plume of black smoke on the left side of the aircraft. I commented to Captain that it seemed larger than normal. I continued to monitor the Engine Warning Display during start and the engine start sequence completed with no abnormal indications. We proceeded to Runway XXR on one engine. While in sequence for takeoff, I started the #2 engine and shut down the APU with no abnormal indications. We ran the appropriate checklists and awaited takeoff clearance. No abnormal odors or fumes were detected before takeoff. 

The initial takeoff from XXR was uneventful up until approximately 1,000 ft MSL when Captain asked if I could smell an odor. Initially I could not. Due to the cold temperature in ZZZ, I had all of my vents and the eyeball air vent on my side closed. I opened the eyeball vent on my side and adjusted it to point towards me; I immediately noticed an odd odor. The odor was not of a smoke or burning nature, so I was not overly concerned initially. We were still in the initial climb phase and my attention was quickly diverted to our ATC handoff from Tower to Approach. We had also reached acceleration altitude [and] I completed the After Takeoff checklist. Upon check-in with Approach, they gave us a left turn HDG 270 which was the opposite direction of the RNAV SID we were assigned. Captain and I confirmed the heading and at some point during the left turn I noticed he was coughing and indicated his throat was hurting. He asked that I don my oxygen mask to get some fresh air for a few seconds and then remove it to see if I noticed the odor and could gauge how strong it was. I donned my mask and took several deep breathes. I then pulled the mask a few inches away [from] my face and noticed a very strong odor. I donned the mask again and ensured it was well sealed. I then chimed the forward flight attendant (I may have even chimed both forward and aft attendants - I cannot recall) and asked if she noticed any unusual smell. She indicated she had not noticed anything. During this period we were cleared to climb to a higher altitude, I believe 15,000 ft. With less than 100 passengers and 21,000 lbs of fuel, we were climbing very quickly. Captain then suggested we get a third nose up front and get their opinion. He chimed the forward attendant and asked one of them to immediately block while the other enter the Flight Deck. [The Flight Attendant] entered the flight deck and immediately noticed the odor. While Captain and [the Flight Attendant] were talking, I was focused on the aircraft navigation and climb state. I recall her saying she used to be a Ground De-ice Coordinator and was very familiar with the smell of both liquid and burning glycol. She indicated it did not smell like glycol. After a very short period of time she indicated a burning sensation in the lower portion of her throat. The Captain asked that she return to her seat. Once [the Flight Attendant] had exited the flight deck, Captain and I briefed on the current state of the aircraft (navigation and altitude assigned). I believe it was at this point he donned his oxygen mask and we agreed a return to ZZZ was the best course of action. He called the flight attendants and indicated we were returning to ZZZ. I mentioned the "Elimination of Foul Odors in Flight Deck and Cabin" QRH checklist and Captain began to run it as I advised ATC of our intention to return to ZZZ, programmed the MCDU (Multifunction Control Display Unit) for the ILS XXL, obtained current ATIS, and sent for Landing Performance Data. ATC asked us if we were [requesting priority handling] and I acknowledged with SOB (Souls Onboard) and fuel. At this point we were at approximately FL250. ATC cleared us to descend to approximately 16,000 ft and we initiated a descent. ATC then gave us a left turn and cleared us direct to ZZZ. Since I had already loaded the ILS XXL approach into the MCDU, I advised ATC we were proceeding direct to ZZZZZ, a fix outside the FAF on the ILS XXL. I still had the Company Ops frequency dialed into COM2; I called Ops and advised we would be returning and requested a gate. They must have already known we were headed back as they quickly assigned XX. I also sent an ACARS message to our Flight Dispatcher indicating we were returning to ZZZ due to a strong fume/odor. We completed the approach checklist and continued our descent. We were later assigned Runway XXR, I made the change in the MCDU and we proceeded direct ZZZZZ1 on the ILS XXR. At some point, approximately 20 miles from ZZZZZ1, I could tell Captain was feeling much worse than I was. He was coughing frequently and indicated numerous times he felt light-headed and his throat was burning. I advised that I felt okay to fly and could take over as PF (Pilot Flying) if he wanted me to. He agreed that would be best and transferred aircraft control to me. The final approach and landing on XXR was uneventful and Captain proceeded to briskly taxi to the gate as I completed the after landing checklist. As we rounded the corner past Spot XY, I called out a vehicle and four baggage carts that were directly blocking our path to the jet bridge. There were a few ground service folks standing around and they did not seem to be in much of a hurry until Captain opened his side window and leaned out, with oxygen mask still on, motioning the ground service personnel to move the vehicles out of the way. The airport fire trucks following us may have also come into view and caused the ground service personnel to expeditiously move the vehicles blocking our path. This wasted a solid two to three minutes. We arrived at the gate, shut down, and completed the parking checklist. As Captain exited the flight deck to speak with the flight attendants and meet the paramedics, I ran through the after landing and parking checklists one more time to ensure I had not missed anything. As I was waiting for external power to be plugged into the aircraft, one of our mechanics entered the flight deck and advised that he would take control of the aircraft and we could leave everything as-is. One of the airport firefighters entered the flight deck with a handheld device that I assume was to take readings of gaseous components or maybe it was an infrared camera to look for a potential fire. I never heard the results of the test. We collected our bags and followed the paramedics up to the gate area where we were evaluated before being transported to the hospital.

Post Event Notes:
Knowing de-ice fluid was sprayed on the aircraft; an odor was not unexpected nor unfamiliar. I am familiar with the smell of glycol in the bleed system and have experienced it many times in the past on various aircraft. The odor on our flight was, in my opinion, not glycol related. I have also experienced the dirty sock/locker room odor in the past and while this was somewhat similar, I would not attribute what was experienced during this event to that particular odor. It also did not seem like an odor that would come from engine exhaust and certainly not the type from an electrical fire. I cannot recall ever experiencing the smell of a phosphate ester fluid, such as hydraulic fluid, coming in contact with a very hot surface; so am unsure if it was similar. The best I can describe the odor we experienced is a pungent chemical smell; and a strong one at that. 

Of the three of us that were sickened by the noxious odor, I think I experienced the symptoms to the lesser degree. That can possibly be explained by donning my mask the soonest, my age, my overall fitness level, and of course the fact that everyone's body reacts differently.

I feel very lucky to have been flying with Captain when this event occurred. Having had flown together for the previous three days only helped. I feel that we exercised great CRM and were able to non-verbally read each other well. We both had an extra ear out for each other during the whole event and filled in or reminded one another when something was missed. Captain demonstrated exceptional leadership skills throughout the entire flight and that is to be commended. Decisions were made on a joint basis, he kept our flight attendants informed, and most importantly, he was never shy about stating his overall condition. This allowed us the ability to make the best decisions with the greatest amount of information available.

TRACON was an excellent resource and gave us whatever we requested. I imagine we were on a discrete frequency at one point because I remember the radio chatter being very minimal until being handed off to the tower; that was appreciated.

One thing that surprised me a bit was how loud the audio through the speakers was from the oxygen masks. Even after turning the intercom and radio volume knobs all the way up so I could hear Captain and ATC better, it only made the extremely loud sound of the oxygen flowing worse and was distracting. I considered moving my intercom switch to the off position to silence the sound of oxygen flowing but did not want to have to move the switch every time I wanted to talk through the intercom. We were simply too busy for that.

Recommendations:
Starting both engines, and thereby shutting down the APU, after pushback could potentially reduce the likeliness of introducing any new odors into the bleed system shortly before takeoff. Not sure if this would have changed anything we experienced. 

It was very frustrating arriving to the gate only to find ground service vehicles blocking the safety area with ground service personnel just standing around. I advised station operations in excess of ten minutes prior to our return and feel they could and should have ensured the safety area was clear and personnel were standing by to marshal us in expeditiously. We did have a jet bridge operator and that was appreciated.

The single most frustrating and disappointing part of this event occurred after I was released from the hospital and was working on getting myself home. I called Scheduling to ensure they were up to speed on the situation and confirm I was released from duty. I requested that they positive space me home, from ZZZ to ZZZ1. They said they could only deadhead me to my base. I then called my Chief Pilot, and asked for his help. He was happy to call Scheduling and make the request. He reported back to me that despite his best efforts, Scheduling would not do it out of fear that a grievance would be filed. I offered to state on a recorded phone line with scheduling that I would not file a grievance for the company sending me home. It is sad and disappointing the folks in Scheduling could not think big picture and consider the circumstances to "Do the Right Thing." I would like to suggest an MOU or side-letter be drafted between the company and [the union] to ensure crewmembers can be positive spaced home if certain circumstances are met. Being transported to a hospital after an inflight situation and subsequent diversion should qualify.

Synopsis

A320 flight crew reported strong fumes during initial climb and experienced health issues. Executed a return to departure airport.

ACN: 1620220 (1113 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Taxi

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1620220

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected.Other 
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After pushback attempted to start engine 1. No N1, no fuel flow. Started #2. Attempted to start #1 again. On third attempt no N1, no fuel flow, but we got a fire warning momentarily. We heard a bang and could see smoke. The fire warning went out instantly. We had fire department check for fire and proceeded to the gate.

Synopsis

A320 flight crew reported returning to the gate after experiencing engine start anomalies, including a momentary fire warning, during pushback.

ACN: 1619902 (1114 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : TUS.Airport
State Reference : AZ
Altitude.MSL.Single Value : 10300

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : U90
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1619902
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

GPWS warning on arrival into TUS. We were on profile flying the ZONNA1 RNAV arrival for Runway 11L. Well prepared for the arrival having briefed terrain issues as well as altitude constraints, MSA, terrain pb's (push buttons) on etc. CA (Captain) was PF (Pilot Flying). Prior to SSAND intersection and before meeting the constraint, ATC gave us a heading of 265 for an area of moderate rime icing, and a further descent to 10,000 feet. Aircraft defaulted into VSI mode and I slowed the descent rate down from 2,500 FPM to approximately 1,500 FPM. At approximately 10,300 feet. the GPWS issued a warning and associated aural "terrain, pull up." We complied with the GPWS recovery actions and the warning quickly ceased after approximately 3 warning cycles. Continued climb thru MSA of 10,700 feet. and queried ATC. ATC stated that their MVA was 9,900 feet. in our area and was a bit surprised we received the warning. We did not receive a GPWS caution. Our radio altimeter never activated. We did not receive any terrain indications on our ND's (Navigation Display).

Synopsis

A320 Captain reported receiving a GPWS terrain warning on arrival into airport.

ACN: 1619310 (1115 of 2908)

Time / Day

Date : 201902

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Weather Elements / Visibility : Turbulence
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use.STAR : ZZZZZ4
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1619310

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Narrative: 1

I was in the middle of receiving a pass-down from another sector for the last hour of my shift when I received an ACARS message at XA40Z from the event flight. It stated that they had experienced 'moderate to borderline severe' turbulence FL220-210. Their message also said that two flight attendants were injured and that they had requested EMS [Emergency Medical Services] directly with [station] OPS. The flight was on descent on the ZZZZZ4 arrival and about to enter the sterile phase of flight, so I responded that I copied their message and asked them to call me when able after arrival. 

At this point, the pass-down was discontinued until after I had dealt with the immediate issues related to this event. I called [station] OPS to verify that they had EMS on the way to meet the aircraft upon arrival. I then called the [Manager On Duty] to notify them of the situation. Next, I entered a PIREP (Pilot Report) for the conditions that the crew reported -- over [the destination airport] since I didn't know exactly where the turbulence occurred but added DURD [during descent] for clarity. (While writing this report, I am realizing I may have mistakenly entered DURC [during climb] instead of the intended DURD.)

Leading up to, and at the time of the turbulence event there were no PIREPS, TAPS (Turbulence Auto-PIREP System) reports, FPG's (Flight Planning Guidance) or SIGMET's indicating any similar type/intensity of turbulence near the flight's arrival route appearing on my ASD/WSI Fusion. In addition, within the aforementioned ACARS message from the crew, they stated that the aircraft did not submit any TAPS report in relation to their turbulence event. After following up with the crew via phone, they stated that ATC had advised them of moderate turbulence being reported ahead of them on the arrival below FL200.

During the post-flight brief with the crew, they also stated that due to the aforementioned ride report given by ATC, they instructed the flight attendants to be seated. The turbulence event started approximately 4 minutes later when the flight was somewhere between ZZZZZ and ZZZZZ1 on the ZZZZZ4 arrival. The event itself was described by the crew as a sudden jolt of severe turbulence lasting approximately 3-5 seconds, resulting in a momentary loss of control. This initial jolt was then followed by moderate turbulence. Two of the flight attendants were injured as they were attempting to return to their seats when the initial jolt happened. One of them sustained a head injury after hitting it on the wall. The other sustained an ankle injury. The seatbelt sign was on and there were no reports of passenger injuries or other damage. No emergency was declared by the crew or myself, as arrival was imminent. The flight attendants were taken to the hospital upon the flight's arrival.

Synopsis

Air carrier Dispatcher reported being advised of a turbulence encounter in cruise with injuries to the flight attendants.

ACN: 1618536 (1116 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : IAH

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1618536
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

First off, I don't think it is possible to properly brief for multiple runway assignments, especially when there are NOTAMs involved for each runway. Along with being informed late by ATC. Runway 27 was the last runway I would expect while on this arrival. The expected runway via the arrival is 26R. Me underlining the NOTAM for 27 did not help me to remember unfortunately. Switching the runway up is busy enough work in this phase of flight. I'll be more aware of this type of goat rope by ATC as a possibility after this incident. Next up. ATC giving a visual clearance with a crossing restriction over a fix that is only on the ILS 27 and RNAV approach plate. This to me is a very lousy clearance. That being said, I accepted the clearance. So I take some blame. At the same time ATC needs to refrain from giving such clearances. Last minute runway switches can be very troublesome especially when NOTAMs are involved. I'm not sure why the ATIS did not advertise Runway 27 RNAV instead of the visual. Both Runway 26R and 26L were shown on the ATIS as both ILS approaches. Had this been the case (RNAV 27 approach advertised) none of this would have happened. I know I am the last line of defense and I did catch these errors, unfortunately not as quick as I would have liked to. But it would help if the system was better set up to not set the pilots on such course.

Synopsis

A320 Captain reported missing an altitude restriction on arrival into IAH, citing a late runway change as contributing.

ACN: 1618501 (1117 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.TRACON : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : CLT

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1618501
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

I was pilot flying. At approximately 4000 feet MSL and 220 KIAS with flaps up (anticipated some turbulence), being vectored for approach and landing at CLT, we were instructed to make a 310 degree left turn and encountered heavy rain, windshear and severe turbulence. Wind increased 60 knots - 80 knots and airspeed increased to 280 knots - 300 knots. Altitude plus or minus 500 feet and bank angle exceeded 45 degrees with full, opposite side-stick input. Requested an immediate turn and a climb to exit turbulence. I was hand flying and modulating thrust in and out of TOGA (Takeoff Go-Around), MCT (Maximum Continuous Thrust) and CLB (Climb) power to maintain aircraft control. Diverted to ZZZ for fuel with an uneventful landing. Severe turbulence maintenance inspection performed. No damage found.

We were monitoring all available weather information during cruise and descent. We had some concerns about the cold front and associated weather moving into the CLT area. None of our available weather resources prepared us for the severity of what we eventually encountered. WSI (Weather Services International) and our own weather radar showed nothing more than light precipitation, and areas of turbulence were not accurately depicted on our radar. Our best source of information became PIREPs from aircraft in front of us. Simply put, its hard to avoid what we didn't know was there. Looking back on events, I would have given more weight to the disturbing reports we were hearing out in front of us and worked with my Captain to exit the CLT terminal environment sooner than we did.

Synopsis

A320 First Officer reported encountering severe weather/turbulence while being vectored for landing.

ACN: 1618363 (1118 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6207
Experience.Flight Crew.Last 90 Days : 174
Experience.Flight Crew.Type : 1021
ASRS Report Number.Accession Number : 1618363
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Upon runway entry First Officer was the flying pilot. Power was applied and noted that both engines started to spin up together. At that point the First Officer put both levers into the FLX position. As he was making the call for "Check Thrust", I noted the aircraft was pulling to the left and requiring more rudder than usual. There were no warning or cautions. I checked the thrust and saw the left engine was producing FLX thrust and the right engine was producing TOGA Power. I checked both thrust levers were in the FLX detent. They were and there was no aural warning to the contrary. I called REJECT and brought the thrust levers to idle due to the First Officer having a hard time maintaining the centerline of the runway. I was going to let the aircraft roll towards the next high speed taxiway. After a lag of approximately 2 seconds the Rejected Takeoff (RTO) MAX brakes kicked in. RTO checklist was call for and aircraft was taxied off the runway.

The brakes heated up due to the MAX braking, approximately 325 degrees Maintenance Control was notified. During this time the brakes cooled back into the normal range and we taxied back to the gate. Contract MX performed tests in conjunction with Maintenance Control all of which were normal. We were re-released back to ZZZ and chose to do a TOGA takeoff. Normal takeoff and trust reduction with no problems.

Synopsis

A320 Captain reported a rejected takeoff due to asymmetric engine thrust.

ACN: 1618357 (1119 of 2908)

Time / Day

Date : 201902

Place

Altitude.MSL.Single Value : 25000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 211
Experience.Flight Crew.Type : 2456
ASRS Report Number.Accession Number : 1618357

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As we were climbing through FL250, PRESS page popped up on the Systems Display screen. The cabin altitude showed approximately 8,500 ft and was increasing. We had been cleared to climb to FL370 by ATC but we stopped our climb at FL270. As we were getting clearance to descend to a lower altitude, we got a Master Warning, along with the CAB PR, EXCESS CAB ALT, ECAM. First Officer made PA to have flight attendants take their Jumpseats. Followed ECAM procedure. The cabin altitude exceeded 11,000 ft and I believe it got as high as 11,300. The passenger masks did not deploy, nor did we expect them to in this situation. 

As we were descending through FL200, the cabin altitude went back into the normal range. We leveled off at 17,000 ft. Contacted dispatch and [Maintenance Control.] Briefed flight attendants and made PA to passengers. Discussed diverting with both First Officer and Dispatch but decided to continue, as long as there were no further problems with the pressurization system. Due to turbulence at 17,000 ft, we climbed up to 19,000 - cabin pressurization system operated normally. Continued on to destination with no further issues.

Synopsis

A320 Captain reported experiencing a pressurization problem during climb.

ACN: 1618006 (1120 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16909
Experience.Flight Crew.Last 90 Days : 207
Experience.Flight Crew.Type : 4153
ASRS Report Number.Accession Number : 1618006
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

As we started our taxi-out from our gate approximately halfway through the turnout we lost nose wheel steering. After talking with maintenance it was decided that we were to be towed back in to the gate due to this being the 5th nose wheel event in about the last 3 days. A [maintenance release] was entered and operations was contacted. The First Officer and I both believe that operations heard from maintenance that the fault cleared with a reset and we could taxi back ourselves. The First Officer informed operations two times that we would not taxi the plane due to the maintenance event. After the 3rd time I took the radio and informed ops that we agreed with maintenance that the plane needed to be towed in, a [maintenance release] was entered and we had a broken [electronic logbook], because of the extensive history we didn't trust the reset and when the Nosewheel Steering failed the nosewheel centered and took us off the tight lead out line and towards another gate and with the tight tolerances we couldn't be sure we would remain clear of the plane at that gate and I finished with I would not move the plane under its own power. The response I got was "well it will be a while then". After about 1 hour we got a tug to tow us back. Operations way overstepped their bounds and should never pretend to know how a plane should be operated.

Synopsis

A320 Captain reported after a temporary loss of nose wheel steering company operations requested that the crew taxi back to the gate without a tow.

ACN: 1617696 (1121 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X

Component : 2

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 30
Experience.Flight Crew.Type : 296
ASRS Report Number.Accession Number : 1617696
Human Factors : Situational Awareness
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 108
Experience.Flight Crew.Type : 704
ASRS Report Number.Accession Number : 1617708
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

Pack 2 deferred was inoperative. After number 2 engine start we did a status check and noticed pack 2 inop which was expected. Below this was AP 2 inop. Expectation bias and fatigue caused failure to realize that AP 2 inop was not related to Pack 2 inop. Continued normal taxi and takeoff without AP 2 inop maintenance logbook entry. Wrote up entry during cruise. [Maintenance Control] communicated that this was an FMGC 2 failure after second engine start. Fatigue was an issue here.

Narrative: 2

[Report narrative contained no addition information.]

Synopsis

A320 flight crew reported not documenting an aircraft equipment problem in the maintenance log.

ACN: 1616530 (1122 of 2908)

Time / Day

Date : 201902
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 17510
Experience.Flight Crew.Last 90 Days : 234
Experience.Flight Crew.Type : 14099
ASRS Report Number.Accession Number : 1616530

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 166.78
Experience.Flight Crew.Type : 2847
ASRS Report Number.Accession Number : 1616533

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Multiple bird strikes perceived in both engines shortly after takeoff with immediate return to [departure airport]. 

All preflight and taxi operations completed without comment. The Captain was the Pilot Flying. Initial climb was unremarkable until at approximately reaching the thrust reduction altitude of 900 feet MSL. At that time several large white streaks were observed in the aircraft lights by both the PF (Pilot Flying) and PM (Pilot Monitoring) on both sides of the aircraft's flight path. Very rapidly thereafter multiple impact sounds were heard and the PF observed roll back indications on both engines. The PF estimated that the left engine rolled back approximately 4-5% N1 and the right engine approximately 9-10% N1. A noticeable physical reduction in thrust occurred, perceived as smooth and symmetric with no apparent yawing moment, for approximately 1 second with both engines recovering to and stabilizing at usable climb thrust. No ECAMS, besides previously unrelated deferral, were presented as a consequence of the bird strike. At no time during the event flight were any abnormal ECAMs displayed.

Aircraft cleanup and after takeoff checklist were accomplished. An immediate halt to the climb was coordinated with ATC and accomplished for a level off at 3000 feet. Coordination with ATC Departure Control [was established] for an immediate return to the most immediately available runway. With both engines stable at usable thrust and no other ECAMS being presented on the aircraft systems the flight crew proceeded with the approach briefing, descent checklist, and flight attendant and cabin PA briefings. There were no other available environmental indications of the bird strike such as unusual cockpit/cabin smells or aircraft system degradations. 

Aircraft systems were set up and confirmed for the return to departure airport. A downwind pattern at 3000 feet was flown under the control of Departure Control. ATC hand off was given to Tower once the airport environment was in sight after a turn to the northwest. Clearance for a visual approach was authorized by Tower and a descent to 2000 feet to commence the approach starting at a 5 mile final.

The aircraft navigation system was reset for the return. Approach and landing were accomplished following and within normal SOP and stabilized approach parameters. The landing runway was cleared without comment and the aircraft taxied to the assigned return gate without further incident. Emergency response crews followed the aircraft until gate arrival was completed. Local Maintenance was contacted to report the aircraft condition and the aircraft was turned over to Maintenance. The passengers were deplaned for rerouting with customer service.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported returning to departure airport after experiencing multiple bird strikes in both engines.

ACN: 1616221 (1123 of 2908)

Time / Day

Date : 201902

Place

Locale Reference.Airport : TUS.Airport
State Reference : AZ
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear

Aircraft

Reference : X
ATC / Advisory.TRACON : U90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : TUS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 171
Experience.Flight Crew.Type : 575
ASRS Report Number.Accession Number : 1616221
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

With strong and gusty winds gusting to 29 knots, we planned on Runway 21 and set up for the RNAV 21 at TUS. We were not assigned a VIA segment and therefore, did not include that in the FMGC and anticipated radar vectors to final approach course. Upon checking in with the Approach Controller, I realized it was very congested airspace and the controller advised us to expect direct RAINA and to get set up for it. The Captain attempted to insert the vias with the associated RNAV 21. The Controller gave us an initial heading and then maybe a minute later came back and asked us if we were set up for RAINA. The Captain manually input it into the direct to since he could not get it to load in the Via. I acknowledged the fix and proceeded direct to it. Immediately, we realized there was a problem and I reverted back to last assigned heading [while] the Captain re-inserted the correct spelling of RAINA in the direct to function. However, because of this delay, we ended up navigating a right 270 that took us towards terrain to reach the fix. We were [queried] by the Controller and then assigned 1000 foot climb in altitude to 11,000 feet. The rest of the approach was uneventful. We debriefed the event as a crew.

Synopsis

A320 First Officer reported a track deviation and CFTT occurred on arrival into TUS following a late clearance change.

ACN: 1615286 (1124 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0601-1200

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cockpit/Cabin Communication
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 14500
ASRS Report Number.Accession Number : 1615286
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This flight was the second group of flight segments with this nose number assigned to the Captain's ID. 

On Flight an MRM was sent in concerning the incorrect placement and safety wiring of the Emergency Evacuation Signal System - Flight Deck (EESS-FD) as described on page 6.10.XX.XX of the A-320 Flight Manual. Note 3 states this switch "is safety wired to the CAPT PURS position." This switch was observed in the incorrect position and safety wire generating the MRM input.

On the following day nose XXXX was again assigned into the sequence of the crew's ID with an MEL applied to the incorrect positioning of the EESS-FD. This MEL placards the "evacuation alarm signaling system inoperative". This applied MEL significantly overstated the correction and needlessly restricted the flight crews access to and use of fully operational EVAC signally systems. The FM states "The switch is powered and wired such that regardless of the physical switch position the CAPT PURS function is always enabled." Thus the flight deck, and purser, EVAC COMMAND systems remain fully functional at all times. This was in fact verified by the flight crew with [Airport Maintenance] observing while at the gate.

When this MEL was read 2 hours prior to scheduled departure during the flight planning process the Captain contacted Dispatch and [Maintenance] to discuss the implications of needlessly inopping a fully functional EVAC COMMAND switch. [Maintenance Control] agreed in the MEL assessment of over deferral and [Maintenance Control] would contact [Airport Maintenance] to re-address the deferral.

Upon arrival at the aircraft the Captain did not observe any change to the MRD on the aircraft or any line maintenance personal working the discrepancy. [Airport Maintenance] was contacted to ensure they were aware of the issue whereupon the crew was informed that Maintenance personnel would shortly respond to the aircraft. When the two (2) Maintenance personnel arrived a discussion concerning the FM guidance, the actual operational capabilities of the aircraft, and safety implications of overall deferral for the EVAC COMMAND system took place. At the end of the discussion the 2 Maintenance personnel stated to the effect of, "well you can just use the PA can't you? So that is good enough. We don't know of another MEL, our discussion is done and we are out of here." Where upon the Maintenance personnel departed the aircraft, resent the MRD with MEL applied and releasing the aircraft for service.

This over application of an MEL, although potentially covering the discrepancy, significantly limited some the possible normal and trained resources for the crew to use during an emergency situation. One solution could be ensuring a proper MEL is in place to address the placement of the CAPT PURS and CAPT SWITCH assuming this switch position cannot be corrected via simple placement and re-safety wiring. Or addressing the FM by revising the switch positioning to allow either CAPT PURS or CAPT ONLY since the switch is functional regardless of physical switch position.

Synopsis

A320 Captain reported an over application of an MEL and contacted Maintenance to address the deferred item.

ACN: 1614326 (1125 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 24000

Environment

Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZHU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZHU
Aircraft Operator : Military
Make Model Name : Fighting Falcon F16
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Plan : IFR
Mission : Training
Flight Phase : Cruise
Airspace.Class A : ZHU

Person

Reference : 1
Location Of Person.Facility : ZHU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1614326
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was training on 59/93 with multiple complex military operations including Crystal/Amraam and AR614. Aircraft Y flashed at me from 56. I took the handoff and Aircraft Y checked on requesting AR614. I told him to maintain block FL230-FL240 and I put in FL270 as a hard altitude and flashed him at Sector 76 because they clear participating aircraft into AR614. 76 took the handoff and I gave Aircraft Y [a] frequency. He requested UHF so I gave him UHF. 76 called on the landline and asked me to try Aircraft Y again because he did not check in. I tried Aircraft Y again and gave him a different UHF. 76 called back again requesting communications with Aircraft Y and Aircraft Y was no longer on my frequency. The AR614 block is FL250-FL270.

I took a handoff on Aircraft X from 56 going to Mexico requesting FL360 and climbed him to FL240. 76 called me and said he still has not had Aircraft Y check in on his frequency. I tried to contact Aircraft Y and did not get a reply. We tried him several times. Aircraft Y was orbiting south of AR614 at FL240. We had to give Aircraft X a 180 heading immediately and a descent to FL190. We had to contact Aircraft Y on guard frequency for him to come back up on 59 frequency.

The aircraft operating in AR614 should take VHF frequency change. Aircraft Y should have reported back [to] frequency if no response on 76 frequency. 

AR614 has been a long standing issue in 59 airspace. It is located in a major corridor and 59 does not clear aircraft into the tract. The departure procedures from SAT approach should be through Rocksprings airspace since they are the ones to own AR614. Corpus acts as a middle man in the procedures outlined for AR614 which leaves room for error because if 76 doesn't take the handoff then we are stuck with a high performance jet orbiting in the corridor and we can only climb him to FL240 which is also the bottom of the high sectors airspace stratum.

The amount of military operations that we have to deal with on a day to day basis along with heavy commercial traffic can result in unsafe situations. The amount of military operations is a burden to the controllers that have to work it since we are required to bend over backwards for them and they keep requesting more and more. There needs to be better procedures, airspace design, and a limit to military operations in order to assure the safety of the NAS (National Airspace System).

Synopsis

ZHU Center Controller reported an unsafe situation when a military aircraft went No Radio and a carrier had to be vectored away from the No Radio aircraft.

ACN: 1613679 (1126 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ONT.Airport
State Reference : CA
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : SCT

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1613679
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Captain was PF (Pilot Flying) and I was PM (Pilot Monitoring.) On the Eaglz2 RNAV arrival ATC gave us a heading off of the arrival and to descend and maintain 4,000 feet. I think this was when we were at about 8,000 feet. I activated the approach and sequenced it for the visual backed up ILS 26R. After that as we were descending I was looking out the window at the terrain and houses but did not think we were in danger but just how low we were as we crossed over that area. To me this was not unusual because once crossing the terrain you are usually high on the approach. So I didn't think it was off. I go to ONT maybe once every 9-12 months. Both the Captain and I had our terrain map up on the MFD.

At about 6,400-6,500 feet ATC gave us a terrain alert and to climb to 8,000. I noticed the Captain did not react so I did and climbed the aircraft. On the way up I responded to ATC that we were in a climb to 8,000. ATC said I must have taken another aircraft's radio [call]. I do not agree, but I am human and do make mistakes so it is possible but I still don't think I did in this case. There was no evidence of stepping on anyone on the radio. The Captain and I both agree we heard our flight number and to fly present heading followed by a descent to 4,000 feet. I read back as such. Later we briefly discussed it with ATC. He claimed it was for another aircraft on the radio. I still believe ATC did tell us 4,000 feet. Either way I realize in the future when around terrain to look at the highest terrain altitude readout on my MFD map and reference that to where it is compared to what ATC gives us to be even more aware. We never got any GPWS alerts at all.

Synopsis

A320 First Officer reported receiving a low altitude alert from ATC on descent into ONT after apparently taking a descent clearance meant for another aircraft.

ACN: 1613677 (1127 of 2908)

Time / Day

Date : 201901

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Ceiling.Single Value : 10.2

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1613677

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Fume event. APU bleed was off during event. The smell, a sweet chemical odor, was present in the aft third of the aircraft. Passengers and crew in the aft galley and the last five rows of the aircraft exhibited the following symptoms immediately coinciding with the fume event: coughing, sore throat, hoarseness, slight headaches. Pack 2 was isolated and fumes dissipated within 5 minutes. No smoke was present. FAs (Flight Attendants) reported that the passengers were feeling better and no emergency or diversion was required. In hindsight, I should have contacted Stat-MD, but did not because the cabin crew reported that everyone was fine and we were busy deviating around weather and descending through moderate turbulence. Our primary concern was to fly the aircraft safely around the weather. No one wanted medical attention upon landing. After landing, one FA continued to cough, felt dizzy, and had numbness and tingling throughout body. She requested medical attention after deplaning and was subsequently taken to the hospital and removed from the next flight. Possible fumes were momentarily noted on ground when APU bleed was selected upon arrival at gate in ZZZ. At the time, I believed them to be outside agents. The aircraft was not recently de-iced.

Synopsis

A320 Captain reported Flight Attendant and passengers in the aft cabin noticed a chemical odor causing mild head and throat irritation symptoms.

ACN: 1612990 (1128 of 2908)

Time / Day

Date : 201901

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Gate Agent / CSR
ASRS Report Number.Accession Number : 1612990

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Gate Agent / CSR
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Upon arrival, I went down the jetbridge to meet the aircraft upon arrival from ZZZ1. As the aircraft approached the gate for self park, I turned on the jetbridge power and camera and waited for the stop light indication to turn on, and then proceeded to approach the aircraft. When I was about 2 feet away from reaching the aircraft I felt a sudden stop and at that moment the ramp agent bellow yelled out.....Stop ...Stop and I did. I stopped movement and looked around and opened the door to take a look outside. At that moment I saw the jetbridge slide in contact with a van that was parked inside a red no park zone. I called OPS from inside the cab to inform of situation and then advised Captain through the window. I went down the stairs to look closely and noticed it was an [company] van that was left with running engine no one around. As I was taking pictures OPS Supervisor came and did the same. OPS and [supervisor] tried to contact someone from [company] for couple of minutes, and even called a tow truck since they could not reach [company]. A few minutes later an [company] supervisor finally showed up and moved the van forward away from the zone. At that time I received an OK from [supervisor] to proceed and meet aircraft, which I did. Everything on jetbridge and slide seemed to be fine and operational. However we also noted two other [company] vehicles parked unattended within the perimeter (inside the same red no park zone).

Synopsis

Gate Agent reported jetbridge collision with an illegally parked van as the jetbridge approached the aircraft door.

ACN: 1612690 (1129 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : KORRY FOUR
Airspace.Class A : ZDC

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1612690
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While on the KORRY 4 arrival into LGA, Washington Center instructed us to cross RIDGY at FL270. The pilot flying set FL270 on the altitude select and managed the descent. The FMS was programmed to cross RIDGY at or above FL270. We forgot to reprogram the FMS to cross RIDGY at FL270. After a couple of minutes we realized our error and we knew we will not be able to make the altitude restriction. We informed Washington Center that we will not be able to make the altitude restriction. Washington Center instructed us to descend to FL240.

Make sure to verify and check that the proper altitude restriction is programmed in the FMS.

Synopsis

A320 Captain reported failing to make a crossing restriction at RIDGY that was modified by ATC on the KORRY 4 arrival into LGA.

ACN: 1612427 (1130 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : DCA.Airport
State Reference : DC

Environment

Flight Conditions : IMC
Weather Elements / Visibility.Other

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : DCA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1612427
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We were cleared to depart DCA on the HORTO3 departure off RWY 1. This is never a comfortable operation due to the close proximity of two high profile restricted areas in the Washington DC metro area and the convoluted departure procedures used to mitigate the risk of penetrating said restricted areas. Normally I want to be the Pilot flying in these cases, but in this instance, I had flown the leg in the afternoon prior due to forecasted weather and my unfamiliarity with my First Officer. [However], I had grown to trust this pilot and was impressed with his intellect, skill and attention to detail.

During preflight briefing, we had discussed the departure procedure and our required actions three times to establish the importance of flying the HORTO3 as charted to clearly avoid the restricted areas and to fly up the Potomac River. I felt comfortable with the situation as we pushed off the gate. It turned out to be a relatively quick taxi and it's fair to say we were a bit rushed taking the runway and cleared for takeoff. However, required checklists were completed and the aircraft was ready for flight in all respects.

As briefed, we climbed away from the runway, pulled heading and turned left to 330 degrees, Autopilot 2 on and direct to ADAXE via NAV mode. The track and progress appeared normal to me.

I had looked away from the displays to locate the checklist and took a quick peak at my FMS, when I looked up again, I saw a white box on my PFD indicating that we were now in HDG mode. I did not know why were no longer in NAV and exactly when he had pulled the HDG knob, or if we had experienced a navigation failure of some kind. I was puzzled and unnerved based on our proximity to Washington DC and the crosswind that was certainly pushing us towards trouble. To say I was surprised is an understatement.

I raised the alarm with words along the lines of "why are we in heading mode, we need to stay on the departure..." 

I gathered the Pilot Flying had felt we weren't tracking correctly towards ADAXE and was going to "fix it" by putting the aircraft's lateral guidance into his hands using HDG mode. I can't recall with certainty, but I believe the HDG selected was around 005. It is difficult to discern the aircraft's track in this mode, but based on the relative motion of fixes displayed on the NAV screen, we appeared to be on a track that would take us north east of ADAXE towards metro Washington DC and near restricted areas P-56A and P-56B, which aren't displayed on our NAV screens but I knew they were close.

During everything I am describing, I was running the After Takeoff Checklist and getting a frequency change to Potomac Departure. I checked on quickly and got a friendly reply from the Controller. He then asked "Are you guys on a departure?" to which I replied, "We've dropped the departure."

I said this because I wasn't sure exactly what the nature of our navigation problem [was]. Was it a FMS problem? Was it a sensor failure or had we (the crew) made a mistake? I needed to quickly convey that we needed help to get back on course and that statement, at that moment, seemed to be the most expedient way to do so.

To my eternal gratitude, the Controller quickly gave us a left turn heading 330. We selected 330 and I also told the Controller the wind at our altitude was 221 at 41 kts. He then followed up by issuing a clearance to turn to heading to 310, then direct HORTO intersection soon after. 

Everything I've just described happened very rapidly because of the close proximity to the restricted areas, and there was very little time for analysis and crew communication to collaborate on a solution. In short, the situation snowballed fast and we were bailed out of it by an attentive controller.

Every bit of radar tracking and GPS data available to us seems to indicate we avoided both aforementioned restricted areas.

Suggestions: 

1. This particular departure from the airport is high risk in many respects. The most experienced crew member should always fly it. Despite the fact that it was the FO's turn to fly, I should have flown the departure. I've done it many times, he had only done it 3-4 times.

2. Remove the restricted areas from the metro DC area. This may seem like heresy, but these restricted areas are so close to a very busy airport used by jet aircraft at speeds that didn't exist when the city, landmarks and the airport itself were established. We have to operate outside of normal procedures to make it work even on a good day. Either change the airport's operations or remove the restrictions to the airspace.

3. Display the permanent airspace restrictions on our Nav displays. If you're not going to remove these restrictions to airspace, at least program the FMS/FMGC to show it on our displays. If I can see it on my screens, it is much more likely I'd be able to avoid it.

Synopsis

A320 Captain reported an RNAV departure course deviation resulted in the aircraft tracking in close proximity of two high profile restricted areas.

ACN: 1612426 (1131 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class D : ZZZ

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1612426

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On initial climb out from ZZZ, the flight attendants called me to inform me that the B and D (AFT) flight attendants were feeling disoriented and had headaches. As we passed through 10,000 feet, they began to smell an odor of alcohol and nail polish remover that made them feel uneasy. As they came forward to inform the A and C (FWD) flight attendants, these flight attendants confirmed to them that they had a similar feeling on the way to ZZZ, but did not share that with anyone else. I followed our QRH for fume removal and the odor did not dissipate, but remained mild while in cruise for the remainder of the flight. I had the AFT flight attendants come forward to the cockpit and administered oxygen. This had the desired effect and they both started to feel better. I had them stay in the forward galley for the remainder of the flight. As we started our descent into ZZZ1, I asked them to return to the AFT galley and [let] me know if the odor changed in intensity. Within in few minutes, they called to inform me that the smell was intensifying and that they were again becoming disoriented. At this time, the passengers, starting with those seated in the last rows, started to complain about the odor and quickly the rest of the plane began to complain. I told the flight attendants to leave the AFT galley and to take a seat in the cabin for the remainder of the flight. Hence, they were not in their safety positions on landing.

It appears there was deice fluid trapped in the airplane that may have caused the event and as the airplane pitch attitude changed, so did the trap fluid. A review of the deice personnel and their application methods may be prudent to avoid the injection of deice fluid and subsequent trapping inside the airframe.

Synopsis

A320 Captain reported flight attendants experienced headaches and disoriented feelings from noxious odor in the passenger cabin.

ACN: 1612223 (1132 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0601-1200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 1600
ASRS Report Number.Accession Number : 1612223

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

A320 procedure has PF (Pilot Flying) arm spoilers for landing. It has been my experience that this swapping who arms every leg results in far too many errors. I personally have forgotten in the area of 2-3% of the time. My FO's (First Officers) have forgotten more than 10%. The error is always trapped by other crew-member and we have the ECAM to back us up, but that's not the point. The procedure is faulty. In [many] years of flying [many aircraft makes and models], I cannot remember one single instance of Captain forgetting to arm spoilers. Captain arms pre T.O. every leg, why should this be any different? This also is important for airline fleet standardization. As pilots swap aircraft, and that's happening at an alarming rate now, the procedure between fleets should be has harmonized as we can possibly make it. On that dark and stormy night we don't need another LIT incident.

Synopsis

A320 Captain reported the need for procedural standardization as a result of failure to arm the spoilers for landing.

ACN: 1611426 (1133 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 10472
Experience.Flight Crew.Last 90 Days : 135
Experience.Flight Crew.Type : 2981
ASRS Report Number.Accession Number : 1611426

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 168
ASRS Report Number.Accession Number : 1611420

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At FL350, auto flight on, we had an ECAM Yellow Hyd Low Level. We established PF/PM (Pilot Flying/ Pilot Monitoring) roles and completed the ECAM. Notified Dispatch via ACARS. Reviewed the QRH notes, and briefed the landing. We asked Dispatch to confirm our decision to land at [scheduled destination], but never got a response. Possible ACARS lost message and we didn't have enough time to follow up. Sent a message to [Operations Control] that we had a Yellow Hyd failure. The ECAM had us shut off the Yellow Hyd System, and turn it back on on final. We evaluated our landing distance based upon the Yellow system not recovering, and verified we had a very safe margin with only one reverser and two spoilers per wing inop. I performed a brief with the flight attendants and advised them there was no reason to prepare for an evacuation. 

We turned the Yellow Engine pump back on when we dropped the gear. We had Approach Control give us an extended final to ensure our flaps would still deploy when operating at half rate. The Yellow Hyd system recovered with normal pressure and very low volume. We made an uneventful landing and exited the runway. Clearing the runway we stopped to allow for the [ramp] crew to complete a walk around safety check. When complete, we taxied to gate and waited until the ground crew verified the chocks were set before securing the number 1 engine, per the QRH notes. The Yellow Hyd pressure remained normal until shutdown, although the volume dropped to zero.

On the post flight walk around we found the entire aircraft belly wet with dripping hydraulic fluid.

Post flight I debriefed the event with my FO (First Officer) [and ground personnel], all of whom responded perfectly.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported receiving an ECAM for low fluid level in the Yellow hydraulic system.

ACN: 1609177 (1134 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None
Airspace.Class C : ZZZ

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.TOWER
Reporter Organization : Government
Function.Air Traffic Control : Local
Function.Air Traffic Control : Instructor
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 1
ASRS Report Number.Accession Number : 1609177
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.TOWER
Reporter Organization : Government
Function.Air Traffic Control : Other / Unknown
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 7
ASRS Report Number.Accession Number : 1609191
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Runway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Maintenance truck, needed five minutes on Runway XXL and was granted this access while holding short of Runway XY on Runway XXL. Meanwhile, Aircraft X began taxiing to Runway XXL while an inbound was about 10 miles out for Runway XY. The South radar controller briefly called to coordinate something, but then hung up and this got the Trainee and I in a brief discussion about what APREQ [Approval Request] they may have been considering. We were using the correct memory aid to show that Runway XXL was occupied and unavailable, however, our scan broke down and Aircraft X was cleared for takeoff while taxiing on Taxiway D towards D1, as the Runway XY lander was already crossing the Runway XY/ XXL intersection. Maintenance Truck, then reported exiting Runway XXL as he heard our clearance and saw the A320 turning from D1 to begin taking the runway. I quickly verified that Maintenance Truck had exited Runway XXL, and he confirmed this immediately and then Aircraft X continue lining up on Runway XXL and he then commenced his takeoff roll. Training on local control was in progress and I was the OJT instructor.

I could have refrained from discussing a theoretical APREQ and maybe this would helped the Trainee and I from having a degraded runway scan that is obviously much more important.

Narrative: 2

I was staffing a position while training was being conducted on LC and a Developmental worked GC/CD. The session had been fairly steady with crossing Runway ops and LUAW [Line Up And Wait] being utilized. As traffic died down, I went to the computer at the SUP position to reference ART [time and position tracker]. As I did this I heard the trainer on local make a call to verify that Maintenance Vehicle was holding short of Runway XXL. I looked back to the approach end of the runway and saw that Aircraft X was entering the runway environment. I then saw Maintenance Vehicle exiting Runway XXL at TWY A1. The vehicle had exited the runway environment prior to the aircraft starting takeoff roll. Continue vigilance while training is taking place. The busier traffic had been handled well, but as the perceived lull came, the error occurred.

Synopsis

Tower controllers reported a runway incursion with a departure and a vehicle.

ACN: 1609015 (1135 of 2908)

Time / Day

Date : 201901

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : JFUND2
Airspace.Class A : ZBW

Component : 1

Aircraft Component : Navigation Database
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 177
Experience.Flight Crew.Type : 915
ASRS Report Number.Accession Number : 1609015

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Primary Problem : Incorrect / Not Installed / Unavailable Part

Narrative: 1

The previous week I flew the same [JFUND2] arrival into BOS. The aircraft should cross SEETS intersection at or above FL210. Last week it did not level off as it should have. We had to intervene and manually stop the airplane from descending. We did not have an altitude deviation. Fast forward to today and I am now doing the exact same arrival in a different airplane. This time I am watching intently to see if the aircraft levels as it should. NOPE!!!!! Three miles before SEETS the aircraft was diving below FL210. We were correctly all set up with the airplane in NAV and DES and on glide path. Without aggressively grabbing the airplane and leveling off it would have blown through FL210 by at least 1000 feet, if not more. For the second time, I had to grab the airplane and manually stop it from descending to avoid a major altitude deviation. I believe there is a problem somewhere in the database. It correctly shows on the navigation display and matches with the chart, however the airplane descends as if unaware of an altitude restriction.

Synopsis

A320 Captain reported the aircraft failed to initiate a proper level off while on the JFUND2 arrival to BOS. Captain stated this is a reoccuring type of event.

ACN: 1607749 (1136 of 2908)

Time / Day

Date : 201901

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 3854
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 2800
ASRS Report Number.Accession Number : 1607749
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Other

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We operated Aircraft X from ZZZ to [destination.] After our initial cockpit set up my First Officer went down to do the walk around. After he came up he told me that "they are de-icing our wings and tail." 

No communication had been established between us and the deicing team. We had no opportunity to configure for deicing. De-icing had commenced without our knowledge. 

I contacted de-icing control and requested to talk to a rep. A few minutes later I was talking to the team leader of the crew de-icing our jet. He said that they are now trained to go ahead and start the deicing procedure (at the gate) as long as ground power is plugged in and gate air is connected.

Synopsis

A320 Captain reported communication breakdown between flight crew and deicing crew.

ACN: 1607744 (1137 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Weather Elements / Visibility : Fog

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 138
Experience.Flight Crew.Type : 963
ASRS Report Number.Accession Number : 1607744
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 8346
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 7656
ASRS Report Number.Accession Number : 1607947
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Departed Rwy XX at ZZZ. Tower had just opened and revised our clearance to include [a specific] Departure. We loaded that into the box and briefed the departure. The Flight Director commanded a right turn prior to the ZZZ1 VOR instead of entering the hold to climb. Before we could correct it, Approach cleared us onto the airway, and stated "it's no problem, [those] aircraft FMS' seem to always command that turn early."

Narrative: 2

We initially called Center for our clearance and were cleared as filed without a SID. We briefed that and were ready for a departure on RWY XY. After de-ice, tower was open and on call for taxi they gave us a new clearance that included [a specific] SID as filed. We plugged that into the box and looked at it as we were taxiing out in the fog (approximately 1/4 mi. vis). We did not notice the note regarding the climb in the hold to the MEA/MCA before proceeding on course. The larger threat is that the box didn't depict any type of hold at ZZZ1, just a turn prior to ZZZ1 to continue on course. On contact with Departure, they asked if we were proceeding to ZZZZZ, which we said we were, as cleared, and they mentioned that there should be a climb in hold. I then said "yes, now we see that." The controller said "Yes, [those FMS's] always do this because your box sequences to the next fix without the hold. [Others] have it depicted in the box." He said "no problem, proceed on course."

Synopsis

A320 flight crew reported deviating from the departure clearance after takeoff.

ACN: 1607740 (1138 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Traffic Collision Avoidance System (TCAS)
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 3348
Experience.Flight Crew.Last 90 Days : 167
Experience.Flight Crew.Type : 647
ASRS Report Number.Accession Number : 1607740
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 166
Experience.Flight Crew.Type : 459
ASRS Report Number.Accession Number : 1607746
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft TA
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The aircraft was in cruise at FL360 in clear conditions. I noticed a message on the ND (Navigation Display) that said "decrease range TCAS." When I decreased the range on the ND the TCAS showed a TA in yellow. The aircraft was at our altitude climbing very slowly. The target changed to a descend RA. The FO (First Officer) and I followed SOP and complied with the RA. I notified Center that we were deviating from our clearance due to an RA. We descended about 200 feet when I visually acquired the aircraft flying over us. At this point the target was 1000 feet above us according to the TCAS display. When we were clear of conflict, we climbed back to our previously assigned clearance and notified ATC. 

There were no aural warnings at all from the TCAS system. When we arrived [at our destination], we wrote up the TCAS. Maintenance repaired the system and it tested good with the "TCAS system test OK" aural message.

Narrative: 2

We were level at FL360, approximately 30-40nm SE of ZZZ VOR, on a direct course [along our route]. Aircraft Y received a clearance to climb to FL370. I am unsure of their starting altitude. The CA (Captain) first saw the TCAS icon indicating a RA (red square) on the ND (Navigation Display) and brought it to my attention. It showed Aircraft Y level at FL360, head on with our track at our altitude. Unbeknownst to us, a TCAS circuit breaker had tripped prior to this event, causing it to not generate any aural warnings for traffic. It took both of us a moment or two to process what was happening due to no aural alerts. I immediately executed the TCAS RA escape maneuver and initiated a shallow descent per the green VSI indications. We descended approximately 200 feet and Aircraft Y executed a rapid climb. We cleared each other by 1100 feet as we crossed each other. The controller implied that Aircraft Y had overly delayed their climb to FL370. I queried ATC about the clearance Aircraft Y had and received no response, as if the incident never happened. Once at the gate, we called MX (maintenance) and they noticed the circuit breaker that tripped after we left [our departure airport].

Synopsis

A320 flight crew reported a TCAS malfunction during an airborne conflict situation.

ACN: 1607408 (1139 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 8340
Experience.Flight Crew.Last 90 Days : 153
Experience.Flight Crew.Type : 7650
ASRS Report Number.Accession Number : 1607408
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On a night approach to ILS XX ZZZ, we requested and were being vectored for an ILS to Runway XX. We were just northeast of the ZZZZZ fix on the ILS on a modified left base, on vectors. We were level at 6000 feet with the autopilot on. We were at 210 knots, Flaps 1 and slowing to 190 knots. We were receiving delaying vectors due to traffic inbound for Runway XY and our procedure to only use Runway XX at night. We received a Terrain Warning and executed the proper procedure for "Terrain, Terrain". 

During the procedure (I was flying) the First Officer notified ATC that we were responding to a Terrain Warning. We then received clearance to level at 8000 feet, which we did. (We only received one warning). We then received another vector to intercept the ILS and a clearance for the approach. We executed a normal approach and landing after that to Runway XX. ATC asked us to contact them on arrival. I called ATC (ZZZ Tracon) and spoke with the Tracon Supervisor who was also on duty vectoring us. He said that they had looked this over and that we were inside the 6000 foot MVA [Minimum Vectoring Altitude] corridor and that the nearest obstacle was 11 miles away at 5500 feet.

Synopsis

A320 Captain reported receiving a Terrain warning and violating a Minimum Vectoring Altitude area.

ACN: 1607168 (1140 of 2908)

Time / Day

Date : 201901
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 30

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Landing
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Last 90 Days : 141
ASRS Report Number.Accession Number : 1607168
Analyst Callback : Attempted

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

I was the flying pilot and [the] First Officer was Pilot Monitoring, landing Runway 28L in SFO. Wind approximately 010/20 G30 kts. VFR conditions, cleared for a visual approach. At approximately 1000 FT on approach a 737 passed us on our right landing 28R. At approximately 30 - 50 FT above threshold the right wing started to come up. I responded with right aileron to counteract the roll. It took full right input to stop the rising right wing. The right wing finally started to lower and I neutralized the control and added back pressure as the aircraft was losing lift. The aircraft had a firm touchdown. I feel like the crosswind and the wake turbulence from the 737 contributed to the event. Although I was on or very close to approach speed, the aircraft was at too low an energy state to counteract the above forces. I felt minimal control authority the final 2-3 seconds of flight until touchdown.

Synopsis

A320 Captain reported a firm landing occurred following a wake turbulence encounter on short final at SFO with a B737 landing on the parallel runway.

ACN: 1606586 (1141 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZOA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZOA

Component

Aircraft Component : Tablet
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1606586

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Filed on the DYAMD 4 RNAV arrival into SFO. Tried pulling it up on Jepp FDPro and it returned [an error] message. We had 3 iPads in the flight deck and none could access this arrival. We reverted to the paper chart on the airplane and continued the flight without further incident.

Synopsis

A320 Captain reported being unable to view the filed arrival into SFO on the iPads, so reverted to paper charts.

ACN: 1606254 (1142 of 2908)

Time / Day

Date : 201812

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3443
Experience.Flight Crew.Last 90 Days : 218
Experience.Flight Crew.Type : 1878
ASRS Report Number.Accession Number : 1606254
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Taxiway
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

The day started when the push back crew asked for brake release without push clearance. This is a regular event in the Mexico cities as I am sure they are under a lot of pressure for on-time. That is not even close to how bad it got though. The Ramp Controller cleared us to push to spot 5. The Captain released the brakes and told the push crew the instructions. The man on the headset was not on the tug, but spoke enough English to understand. While pushing us back the ramp told us to stop and hold position due to an aircraft behind us. The Captain told the ground crew to stop, but they didn't listen. They kept pushing. The Captain had to open his window and yell stop to them while waving his hands until they finally stopped. It blows my mind that the man on the headset doesn't understand the word stop. After we stopped the Captain was trying to explain to the man that we were instructed to stop and then they started to push us back further again. Ramp controller told us to stop the aircraft again on radio 2. I told her we are having language issues with the push back crew and apparently they don't understand the word stop. She had to stop the other aircraft or equipment that we would have been in conflict with. I called Operations on radio 1 and said we needed someone on the headset who spoke English and understands the Captain's commands. They started to send someone out and for a third time the tug driver started to move the aircraft backwards again un-commanded. The Captain continued to yell stop the aircraft to the man on the radio and the instructions were completely ignored. 

I suggested to the Captain to set the parking brake but he feared that it would inflict damage to the nose wheel strut and continued to yell stop to the ground crew. In all my years flying airliners I have never experienced anything like this in any country. The ramp crew was going to do what they wanted with no regard for the Captain's commands. Very unsafe and we are lucky we weren't pushed back into another aircraft. If we had an onboard emergency and needed them to stop pushing the aircraft with the communication breakdown clearly this would have led to a serious issue or injury. The man on the headset continued to say ok stop, then ok we push you now. He is in no position to be on the headset. Clearly a language barrier is the issue even though he seemed to speak and understand English.

Synopsis

A320 First Officer reported a communication breakdown between the Push Crew and Flight Crew caused a potential aircraft conflict.

ACN: 1606081 (1143 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Parked

Component

Aircraft Component : Fuel Crossfeed
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1606081

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I operated Aircraft X with MEL for the left outer tank transfer valve inoperative in the open position, allowing left outer tank fuel to drain into the left inner wing tank after refueling. This [type of] MEL [is] applicable for ten calendar days.

MEL was first applied in ZZZ on [Day 1], then tested in ZZZ1 and the MEL removes on [Day 3.]

On [Day 4], in ZZZ2, the aircraft was written up after flight due to the left outer tank fuel draining into the inner tank. Again, it was reset and Ops checked good. On the same day in ZZZ2, the next flight crew wrote up the left outer tank fuel draining into the inner tank after refueling. This time, [two other related MELs] were applied, but the MEL specifies that these only apply if the outer to inner transfer tank valves are "inoperative closed", which clearly is not the case. These MELs were active through [Day 12] in ZZZ3, when the left and right [fuel indicators] were [addressed] and both MELs cleared.

After the next flight on [Day 12] from ZZZ3, the aircraft was again written up for left outer tank fuel draining to the inner tank after refueling. Again, the valves were reset and Ops checked good.

On [Day 14] in ZZZ4, the same writeup occurred after refueling. Another flight crew took the aircraft that day, and wrote up the left outer tank fuel transferring to the inner tank after refueling. [The first MEL] was again applied, and still active when I accepted the aircraft on [Day 17.]

This aircraft has been operating for 17 calendar days since the original MEL was applied on [Day 1], without effective repairs that the MEL Deferral program requires. Again and again, resets are performed to limp the aircraft along on the next flight without performing the correct repair that will fix this issue. This is against the spirit and intent of the MEL program and should not be tolerated.

Synopsis

A320 pilot reported the MEL for a malfunctioning fuel cross feed system was reset several times with repairs being continuously pushed to later dates.

ACN: 1605888 (1144 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Angle.Radial : 040
Relative Position.Distance.Nautical Miles : 1
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use : None
Airspace.Class B : ZZZ1

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1605888
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure, gear failed to retract. Diverted to ZZZ1 for landing.

Synopsis

A320 Captain reported the landing gear failed to retract after takeoff that resulted in a diversion.

ACN: 1602272 (1145 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Facility : LAX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Ground
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 8.0
ASRS Report Number.Accession Number : 1602272
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Air Traffic Control
When Detected : Taxi

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Frequency bleed-over occurred again. Aircraft X called to push from on Ground 1 frequency. I was working Ground 2 on frequency and I heard Aircraft X on my frequency which has been a problem for some years now. I sent a previous report a couple of weeks ago. I just recommend investigating the equipment and determine why it happens with certain aircraft.

Synopsis

LAX Tower Controller reported one of the Ground Control frequencies receives bleed-over from another frequency from certain aircraft.

ACN: 1602166 (1146 of 2908)

Time / Day

Date : 201812

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Items Involved : Repair

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1602166
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

A LOSA (Line Operations Safety Audit) observation was performed highlighting the ongoing issues that technicians are having with the IPC (Illustrated parts Catalog) system in EDOCS (Electronic Document Control System). An Airbus A320 required a spoiler elevator computer replacement (SEC). The technician utilized the IPC through EDOCS to conduct his research for part effectivity. He stated that it was very challenging and time consuming to decipher all the cumbersome notes that could potentially determine if the part was effective for the aircraft. Those notes included researching COA's (Change Order Authorizations), AD's (Airworthiness Directives), service bulletins, etc. to determine the pre or post status. It took the technician over an hour and a half to finally determine that the part was effective for the aircraft. After the observation, I called the parts hotline telephone number to see if he would be able to give better guidance on determining part effectivity more efficiently. I had asked him to run through the same scenario that the technician faced with the SEC. It took him over an hour to finally determine that the part was effective for the aircraft. It has been documented in the past that wrong parts have been installed on aircrafts due to the inefficiencies of the IPC system. The question has been asked by technicians working the terminal and or line environment on why parts that are not effective for the aircraft not being filtered out even after the IPC EDOCS system asks for aircraft nose numbers? When aircraft nose number is entered into the IPC, it should filter out the parts that are not effective for the aircraft.

Synopsis

Maintenance Technician had to spend an inordinate amount of time researching whether a particular part is correct for a particular aircraft tail number. Streamlining this process will decrease the chance of an incorrect part being installed.

ACN: 1601892 (1147 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1601892

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During preflight, I accepted what I believed was a low risk associated with not investigating the link between the horse manure odor coming from both external air conditioning and Pack 1 because I had never experienced or heard of this type of odor being associated with fume events. No one else I spoke to during or after any of these events had ever experienced or heard of this either. Now I know this odor is associated with fume events. In the future, I would not accept an aircraft after a similar series of linked events during preflight without further investigation and correction into the root cause. In this situation, I should have taken the time to pre-plan what to do if Pack 2 produced fume odors during various phases of flight. More generally, I will learn from this experience and incorporate it into risk assessments for accepting deferred maintenance items.

The Safety Department should publish de-identified summaries of this and other fume events to allow other crews to learn from this experience as well--particularly that this type of smell can exist.

The Safety Department should follow up with the flight attendants to ensure that they receive appropriate medical care and that no disciplinary action is taken against them for calling out sick on the line.

[The company] must continue to investigate the causes and prevention of fume events. In particular, the same aircraft had numerous fume events in the several days following this flight. Repeatedly returning this aircraft to service without correcting the root cause is unacceptable.

The mechanic attempted to intimidate and interfere with me in the performance of my duties contrary to 14 CFR 91.11 and 121.580, and he refused an opportunity to reconsider and learn from his actions. This indicates his conduct was willful. Mechanics are well-trained and experienced employees held to a high standard. The Company must take disciplinary action against him.

Flight Operations Management was not concerned that a mechanic attempted to intimidate and interfere with me, and Management did not provide my requested assistance in expediting transportation to the hospital for the flight attendants. Instead, Flight Operations Management was concerned about whether I had implied liability to passengers, why I had requested ARFF, and whether I was going to be unable to operate my next flight. The Safety Department should discuss with Flight Operations Management the need to provide requested assistance to flight crews dealing with emergencies, and the need to avoid creating undue pressures against flight crews taking the safest course of action.

The flight attendants' immediate communication allowed us to correct the problem right away and substantially reduce the severity of the fume event. The First Officer did a great job on this flight, as well as during the entire trip. He completed the abnormal/emergency procedures and communications I delegated to him quickly and correctly so I could focus on landing. Maintenance Control and the other local mechanics took the time to thoroughly discuss the event with me to assist in their troubleshooting. The Captain and First Officer who were scheduled to fly the aircraft next made themselves available to assist us during the aftermath of the event. In particular, the incoming Captain looked after the flight attendants at the gate while I was busy on the jet bridge. All of these personnel should be commended.

Synopsis

A320 Captain reported a fume event on the aircraft resulted in several flight attendants going to a hospital for medical treatment.

ACN: 1600674 (1148 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 300

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Type : 229.85
ASRS Report Number.Accession Number : 1600674

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after departing ZZZ, just prior to the first turn on the ZZZZZ SID at [around] 400 feet, we received an ECAM alert from the aircraft. After performing the SID turn and completing the After Takeoff Checklist, we coordinated with ATC to stay in the local area while we ran the ECAM. [We advised with] ZZZ ATC as we were near ZZZ1 and decided as a crew to divert. There was no abnormal pressurization indication, but as we were level at 3,000 feet, we did not expect to see a pressurization issue. We were well above max landing weight, and elected to burn gas as we had no indication (drag, controllability issues, etc) that would have deemed an immediate and overweight landing necessary. While burning gas, we coordinated with Dispatch, our Flight Attendant teammates, and ATC. We informed the passengers of the situation and our intent to land in ZZZ1. After reaching a GW of 142.2, we proceeded to land on [Runway] XXL. We had discussed making as smooth a touchdown as possible, given that should the cargo door be open, [we] were concerned of structural damage of the aircraft. A gentle touchdown occurred and taxied in without incident. Upon arrival, hub MX informed us that the Aft Cargo Door handle was in fact open. It was discovered that a significant issue with the cargo door latch mechanism was present and the aircraft was removed from service. After a plane swap, we continued to [destination] and successfully completed the flight with a delayed arrival.

Synopsis

A320 First Officer reported diverting to an alternate airport after receiving an ECAM alert related to a cargo door malfunction.

ACN: 1600114 (1149 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1600114
Human Factors : Physiological - Other
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Diverted
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

After reaching our cruise altitude we received a call from the first Flight Attendant stating they had a passenger in the back that was at some point unconscious and they had an EMT on board evaluating their condition. They then called back again and stated the EMT felt the passenger was in or approaching cardiac arrest and they would be using the AED. At this point I asked the Captain if he would like to divert to an airport that was only 80 or so miles directly ahead. He attempted to use the Crew Phone and it said the system was unavailable. We reviewed the QRH and looked for communication via ARINC. The QRH says to find the frequencies in the FOM. This is ridiculous to have the emergency contact frequencies for a possible emergency situation to be buried in another manual. With time of the essence and another call from the first Flight Attendant I told the Captain I could talk to ATC and initiate a diversion if he wanted to talk to the Flight Attendant and get any other necessary information. He agreed and we initiated a diversion. At this point we still had not been able to contact Dispatch via the crew phone app or use the ARINC process.


I called Center and told them we had a medical [situation] and would like to divert. He quickly provided direct and we began our descent. I asked him for the destination weather which he provided. It was VFR and landing to the west. The Captain was still talking to the Flight Attendant and said the EMT was performing compressions on our passenger. He asked if I had talked to Dispatch yet. I told him I was calling Operations and would have them notify Dispatch. During the descent I called Operations. She was a tremendous help and did a great job! I told her we would be diverting and we had a medical [situation]. I told her we had not talked to Dispatch yet and asked her to call them. She asked what our aircraft type was and relayed the information to Dispatch. The Captain was back from talking to the first Flight Attendant and asked if I had talked to Dispatch. I told him I spoke with Operations and they were taking care of it. He then took the aircraft while we were in our descent. We landed without incident and our passenger was alert and speaking as he was taken off the aircraft by the paramedics. 

Each co-worker did an outstanding job during our diversion. From the quick action of the flight attendants, and the very calm demeanor of our First relaying the necessary information the inflight crew did a fantastic job. The operations agents were fantastic. They all took the load off of us as pilots so we could get the aircraft safely on the ground. The failure was our electronic crew phone app and the lack of quick reference to the necessary ARINC frequencies in our QRH. The crew phone app being unavailable was a serious hindrance. Looking to the QRH and not having readily available ARINC frequencies and being told to reference an entirely different manual is dumb. We barely had the time to use the crew phone app, let alone figure out which of the manuals the frequencies may be hiding in. Very, very frustrating. We more than likely did not have the time to contact Dispatch via ARINC as the time from the first call from the first Flight Attendant to the time we landed was probably less than 15 minutes. The ARINC frequencies should be placed back into the QRH.

Synopsis

A320 First Officer reported during a medical situation their crew phone was out of service and it was difficult to find the ARINC frequencies in their manuals.

ACN: 1600052 (1150 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Final Approach
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : DEN
Make Model Name : Bombardier/Canadair Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : DEN

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 6227
ASRS Report Number.Accession Number : 1600052
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 4167
Experience.Flight Crew.Last 90 Days : 196
Experience.Flight Crew.Type : 2273
ASRS Report Number.Accession Number : 1599801
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

TCAS RA [during] DEN visual Runway 16L.

On a 220 heading to intercept, we got a traffic alert for traffic off our right and behind us. I do not recall DEN Tower telling us about traffic, but I could have missed them telling us about it. I was the Pilot Monitoring when a traffic alert went to an RA with descend command which the First Officer followed while we were looking for the traffic. The RA went to clear of conflict and back to descend command. We initiated another RA descent and again got clear of conflict. The TCAS Traffic symbol (now red) showed the conflicting traffic right on top of us with less than 300 ft vertical separation. We believed he must have been behind us and overtaking us for the parallel approach to [Runway] 16R. We couldn't see the traffic. I tried to notify the Approach Controller of the TCAS RA as we were approaching LEETS, the FAF for RWY 16L. There was so much traffic on the radio I couldn't communicate with the Approach Controller. At LEETS, I switched to Tower on my own and notified him of the TCAS RA to descend. 

We could see that the traffic was a CRJ passing us on final and going for [Runway] 16R. We landed visually without incident. We were to the left of localizer until clear of conflict the second time. The CRJ overtook us by at least 30 KTS on short final and from a higher altitude to landing before us. It appeared the CRJ did not attempt to climb during the TCAS event as we were descending with our RA. 

Once on Ramp frequency, my First Officer asked the pilot of [the other] flight if he got a TA. He responded "lots of them." He then asked if he got an RA with "Climb" command and he said he did. He then asked if he followed it and he said "I had you in sight I don't have to." My First Officer explained to the pilot he was behind us and we couldn't see him so we had to descend. He responded that he "wouldn't follow the RA command because he had us in sight."

Narrative: 2

I understand that on closely spaced parallel approaches, we can turn to TA only but I could not get a good visual on the traffic, so I followed the descend command. Also, I was left of localizer when this happened and was not positive that the traffic was not overshooting his localizer. I had the map ring on its lowest selection and it was very difficult to tell his location or track.

Synopsis

A320 flight crew reported responding to a TCAS RA on approach to DEN.