ACN: 1599772 (1151 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZSE.ARTCC
State Reference : WA
Altitude.MSL.Single Value : 34000

Aircraft

Reference : X
ATC / Advisory.Center : ZSE
Aircraft Operator : Air Carrier
Make Model Name : A320
Flight Phase : Cruise

Component

Aircraft Component : Tablet
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 22000
Experience.Flight Crew.Last 90 Days : 16
Experience.Flight Crew.Type : 16825.60
ASRS Report Number.Accession Number : 1599772

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

4 hours into 5 hour flight, [my] iPad flashed brightly, shut off, and began an endless loop cycle of attempted start-ups followed by attempted shut-downs. iPad never recovered from this malfunction. Fortunately, [I] had just finished a review of NOTAMS and arrival procedures, and had printed a paper copy of the flight plan before takeoff. [I] also took out what paper Jeppesens we still carry, incomplete and hardly sufficient, but better than the iPad we've adopted. Thank God the flight conclusion was normal and uneventful and we still carry hard copy QRHs so we at least had peace of mind that, if need be, we had a trustworthy source for emergencies and contingencies. The iPad remained worthless for the rest of the flight. Let's hope we never foolishly rely on iPads as sole source for anything. They belong as convenience items only. We should never bank on them for any safety of flight items. As I type this from the layover hotel, my trusty iPad continues it endless cycling on and off. Hopefully before long the battery will expire and with it another iPad will go to the landfill. Good riddance.

Synopsis

A320 Captain reported an enroute iPad failure and reliance upon backup printed materials to safely complete the flight.

ACN: 1599447 (1152 of 2908)

Time / Day

Date : 201812

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuel Storage System
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 6829.57
ASRS Report Number.Accession Number : 1599447
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Just prior to pushback, we received our fuel form. The fuel boarded check failed as the allowable tolerance was 75 gallons and the form indicated it was 88. The fueler just handed it to us and walked out without mentioning the out of tolerance to us. After we noticed it, we called him back to discuss it. He said we are the first to ever say anything to him about the out of tolerance issue; most pilots don't say anything about it.

We called Maintenance and at their request, made a writeup to have the tanks stuck to confirm the fuel boarded. When the first of, I believe, 5 or 6 mechanics came to the cockpit to discuss what we needed, he said he had only ever stuck the tanks once before. None of the pilots ever write this up here.

He seemed somewhat unsure of the procedure and of our request. I tried to explain the whole issue referencing the FOM as best I could. After about 45 minutes, we got a new MRD stating the procedure was accomplished and the tanks were full to capacity. That still did not confirm our fuel on board. Called Maintenance again and told them of the issue. They called back and said the number was 1474. We had no idea what that meant to us. He said that was the fuel on board. Our release fuel was 25,800 pounds. I told him that was way too low and asked what that number really meant. He said it was the fuel in the outboard tank. The mechanic only stuck the outboard tanks not all the tanks. We asked for his return and was told he went home. 

A Supervisor showed up next and wanted to know what our problem was. I again explained it and told him I need to know the total gallons on board. He seemed confused because according to him nobody ever writes that up. He said we needed to write it up again which we did. The MRD was broken again and this time the reason was "the Captain was not happy with the MRD". A third mechanic came to the cockpit to stick the tanks, explained to him and never saw him again. Waited a while and a fourth one came, explained it and never saw him again. A fifth one finally came, explained it to him and he seemed to know what he was doing. He stuck the tanks and gave us the numbers we needed. Another Supervisor came to confirm we got what we needed. In total I believe 5 or 6 mechanics were involved. 

When we did get the numbers from sticking the tanks, it disagreed with the totalizer by about 900-1000 pounds. I brought that up to the mechanic and he said there is no tolerance published between the totalizer and what is actually verified in the tanks via sticking them. We did have enough per the flight plan to accomplish the flight safely and legally at that point as verified by the sticking procedure.

In the end, the fueler didn't seem too concerned about the out of tolerance nor did he mention it to us. Several mechanics came to the cockpit, seemed very confused about the verification procedure (how to stick the tanks) and in the end about the fourth one to come finally accomplished it correctly. They all acted like this is the first time they had heard of this. I find it hard to believe that I was the first plane ever to have an out of tolerance issue and need to verify the fuel on board. Part of me thinks that either the tolerance is checked and can be explained by over/under servicing OR nobody ever checks the tolerance. This whole verification procedure took about 2.5 hours, way longer than necessary or required.

Synopsis

A320 Captain reported taking an extensive delay to determine actual fuel on board after multiple mechanics expressed ignorance of the procedure to stick the fuel tanks.

ACN: 1598949 (1153 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility.Other 
Work Environment Factor : Temperature - Extreme
Work Environment Factor : Poor Lighting
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Cowling/Nacelle Fasteners, Latches
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft.Other 
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Function.Maintenance : Lead Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Lead Technician : 2
Experience.Maintenance.Technician : 15
ASRS Report Number.Accession Number : 1598949
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

An Airbus A320 departed ZZZ in the early morning hours and encountered an event in which the fan cowls separated the #2 engine. Prior to that this, Aircraft X had an overnight maintenance visit which included routine and non-routine work; the focal point of this summary is the #2 engine. The following is a brief summary. Myself and another tech were working #2 engine, that included an oil lube unit filter check, I.D.G differential pressure indicator check, [and] also an igniter change (2 per engine).

The conditions for this night is cold, windy, poor lighting and disruptive. In the course of the overnight maintenance visit, the aircraft was positioned at the remote pad. Just prior to completing the task at hand on the #2 engine, the ground crew needed the aircraft to be repositioned to gate. In order to accomplish this, any work being done to Aircraft X [was] ceased and [we] proceeded to prep the airplane for movement which included the closing of #2 engine "C" ducts and #2 engine fan cowls; this caused a workflow disruption. Upon arriving at gate, the plan was to continue work on engine 2; however, I was immediately tasked with assisting another tech/Lead in the installation of the #2 TCAS antennae which is located on forward upper half of the fuselage. Upon completion of this task, Aircraft X is down to paperwork activity; I had no further involvement with Aircraft X and proceeded to the office. At this time, the aircraft is being readied for flight by various ground personnel. 

The incident, which occurred shortly after take-off, by where the fan cowl(s) separated engine #2 after takeoff and successfully returned to the field. The fact that the fan cowl latches and thrust reverser latches may have been not completely latched is the issue; however, this was missed by Maintenance and flight crews during final walk around.

Synopsis

Maintenance Technician reported the engine cowling departed the aircraft after takeoff, likely due to the cowling not being properly latched.

ACN: 1598620 (1154 of 2908)

Time / Day

Date : 201812
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 4000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7442
Experience.Flight Crew.Type : 5398
ASRS Report Number.Accession Number : 1598620

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1424
Experience.Flight Crew.Last 90 Days : 240
ASRS Report Number.Accession Number : 1598626

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout through 4,000 feet, the flight attendants in the aft cabin called to say they had a strong electrical odor in the back of the aircraft. In less than a minute, we smelled it in the cockpit. We donned the O2 masks and ran the Smoke/Fumes checklist. [Advised ATC] and received clearance to return to ZZZ. Shortly into the checklist, the flight attendants in the back called again to say they were still getting the strong effects of the electrical odor. We landed ZZZ, and taxied to the gate to deplane.

Narrative: 2

After takeoff into IMC conditions and at approximately 3,500 feet in a climbing left turn, the Flight Attendant broke sterile cockpit to notify us pilots that there was a strong electrical fire fume smell. I was First Officer and was pilot monitoring so I took the FA's (Flight Attendant) call. I told her I will tell the Captain. I told him but as I was in process, we both confirmed we smelled it too. I said "Smoke/Fumes" IMMEDIATE ACTION CHECKLIST" and we donned our O2 masks as I grabbed the QRH to run the checklist. The Captain called for flaps up and after takeoff checklist so I paused from QRH to clean up the aircraft from takeoff and run the after takeoff checklist. The Captain was pilot flying and took radios and [advised ATC]for electrical fumes. ATC vectored us to ZZZ. I programmed new airport in FMGC, answered message to Dispatch, got landing data etc. Then Captain briefed approach, we did descent checklist and I then became pilot flying while Captain completed the Immediate action checklist and QRH.

We did not have smoke, only strong electrical fumes. Then FAs called and said their eyes were burning. We got our vector to intercept and cleared ILS to maintain 5,000 feet until ZZZZZ maintain 170 knots to ZZZZZ1. Then FA's called again to say they were feeling really sick. Captain said we were almost there. The TCAS displayed We were behind an aircraft landing ZZZ. Weather was about 200 feet above minimums. We were IMC in rain the entire flight. We landed and taxied to the gate and deplaned.

Synopsis

A320 flight crew reported flight attendants detected fumes on climbout, returned to departure airport.

ACN: 1598335 (1155 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 22000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Powerplant Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1327
ASRS Report Number.Accession Number : 1598335

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Type : 627
ASRS Report Number.Accession Number : 1598052

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise began getting nacelle overheat indications on number one engine. Suspected hot air leak and temps ranged from 320 to 385c. No other engine indications. Decided to proceed to [destination] for landing. Upon landing shut left engine down, inspected by ground fire and maintenance crews. Taxied to gate on right engine.

Narrative: 2

Upon reaching our cruising altitude of FL220 my Captain and I observed the engine system page continuously popping up and then disappearing, and upon bringing up the engine page noticed the NAC temperature on the #1 engine fluctuating between normal and very high. The fluctuation continued with temperature indications reaching as high as 385c. Both my Captain and I agreed that the safest course of action would be to [divert to an alternate]. We decided the most suitable airport would still be [destination] since there were no other real good airports along our route of flight. We continued to [destination] whereupon landing we had personnel meet the aircraft to inspect the left engine. The engine was observed normal so we continued to the gate single engine using the #2 engine.

Synopsis

A320 flight crew reported continuing to destination after noting high nacelle temperature indication on #1 engine.

ACN: 1598136 (1156 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Commercial
Experience.Flight Crew.Total : 20000
ASRS Report Number.Accession Number : 1598136
Human Factors : Distraction
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1598139
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Taxi to runway without issue. After cleared for takeoff, I (Captain) applied flex power and aircraft began to drift toward left side of runway. Attempting to correct back to centerline while airspeed increasing using rudder only the aircraft's nosewheel began to skid and continued toward left side of runway. I then applied max brakes and max thrust reversers to avoid departing the runway. The aircraft came to a stop on the runway and was able to use nosewheel steering to taxi clear of the runway. Max speed attained was approximately 30 knots. Left wheel brakes peek temp after abort was 65 C and max temp on right brakes reached 110 C. 

At this point, not sure if both engines spooled eventually since my mindset was focused on departure turn to avoid restricted area.

Narrative: 2

At this time, we are not sure if both engines spooled evenly. Attention was momentarily on the FMAs then to outside the aircraft when the aircraft began to swerve.

Synopsis

A320 flight crew reported the aircraft nosewheel started to skid and aircraft veered towards left side of runway as they started the takeoff roll.

ACN: 1597463 (1157 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : PDX.Airport
State Reference : OR
Altitude.MSL.Single Value : 13000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : P80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class E : P80

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : P80
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1597463
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were at assigned altitude of 13,000 FT. An aircraft began a fast descent towards us. At 500 FT away from us, we got an RA to climb. We began a climbing/ fairly steep turn to miss him. It was close enough that we saw his lights even though IMC. After calling ATC on the ground, we were told that he was given an approach clearance into a satellite airport. This should not have happened.

Synopsis

A320 Captain reported an NMAC with another aircraft in the vicinity of PDX airport.

ACN: 1596965 (1158 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Distribution Relay
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 8260
ASRS Report Number.Accession Number : 1596965

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 4500
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 2175
ASRS Report Number.Accession Number : 1596979

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While climbing through FL210 approximately with the FO (First Officer) operating as the PF (Pilot Flying) and autopilot 2 engaged, the aircraft experienced an abrupt yaw to the left, and the Captain's PFD, ND, and the upper ECAM screen momentarily went blank. The aircraft quickly recovered, and the blank screens returned to normal. Both the First Officer and I scanned the overhead panel and circuit breaker panels, and found nothing amiss. We also cycled through the systems pages on the ECAM display, and all was normal. Approximately two minutes later, the same thing happened, only the yaw was more abrupt, and the Captain's screens and upper ECAM display remained blank for several seconds. We then experienced the following ECAM's: 

ENG 1 EPR MODE FAULT
NAV GPWS FAULT
AUTO FLT RUDDER TRIM 1 FAULT
BRAKES SYS 1 FAULT
ELEC GEN 1 FAULT
ELEC BCL 2 FAULT

I directed the First Officer to fly the aircraft and communicate with ATC, and to ask to level the aircraft at FL240. Given the multiple malfunctions and our proximity to ZZZ, I consulted with the First Officer and decided that a return to ZZZ was in order. While the First Officer coordinated with ATC for our return to ZZZ, I started working through the displayed ECAM procedures, which resulted in taking generator 1 off line, and placing both engines in the N1 mode. We started the APU, and I coordinated with the flight attendants, made a PA announcement to the passengers, sent dispatch a brief ACARS message telling them that we were experiencing multiple issues and were returning to ZZZ, and asked for landing performance data. We completed an uneventful overweight landing back at ZZZ, and taxied back to the gate.

Narrative: 2

During the climb out heading west passing through FL210, airspeed 315 KIAS, autopilot and auto-thrust on, the aircraft made an uncommanded yaw to the left while simultaneously the Captain's side PFD, ND, and the E/WD screens momentarily went blank. The aircraft returned to stable flight conditions almost immediately, no ECAMs or other warnings were present, and all systems appeared to be operating normally, therefore the climb was continued. Approximately two minutes later climbing through FL240 there was a second uncommanded yaw to the left that was of greater magnitude than the first, causing the aircraft to also roll to the left, while again the Captain's side PFD, ND, and the E/WD screens went blank and flickered for several seconds. At this time the first of multiple ECAMs began to appear, which included ENG 1 EPR Mode Fault, NAV GPWS Fault, Auto Flt Rud Trim 1 Fault, Elec Gen 1 Fault, BCL Fault, and Brake Sys 1 Fault, along with several others. 

As I was the PF (Pilot Flying), and after briefly confirming with the Captain, I disconnected the Autopilot and Auto-thrust systems and initiated a level off, and after another quick discussion with the Captain, he began working the ECAMs while I continued to fly the aircraft and assume control of radio communications with ATC. I coordinated with ATC for a slight descent back down to maintain FL240, while I reduced thrust to slow the aircraft to 280 KIAS. While I was descending and slowing the aircraft (with Autopilot and Auto-thrust off), the aircraft again made several large, uncommanded yawing/roll movements to the left. With the Captain's concurrence, I [notified] Center and requested an immediate left turn and a heading vector back to ZZZ while also initiating a descent. There were many what I describe as "cascading" ECAMs the Captain was working through at the time as it seemed that multiple systems were being affected simultaneously. One notable failure was the loss of GEN 1, so we elected to immediately start the APU in order to have a second generator available. The Captain continued to work through the list of ECAMs, confirming with me before shutting off or resetting any systems as directed by the ECAM while I maintained aircraft control and initiated a continuous descent and return. 

I coordinated for a visual approach to runway XX, as it was the longest runway for the winds, and I also requested emergency vehicles to be dispatched to the runway. The Captain meanwhile coordinated with the flight attendants, made an announcement to the passengers about our return, and sent an ACARS message to dispatch informing them of we had multiple system failures and requesting landing data. The Captain went through the overweight landing checklist as we were at still approximately 148,000 lbs. Approach reference speed was approximately 149 KIAS with full flaps. I maintained control of the aircraft and performed a visual approach backed up by the ILS followed by an uneventful landing using Max reverse thrust and minimal braking. However during rollout, the E/WD screen indicated that the #1 engine thrust reverser was not deployed, however I did not perceive any directional control problems while using reverse thrust, and only minimal braking was required to clear the runway. The Captain performed an uneventful taxi to the gate and once the aircraft was secured, the passengers were deplaned using normal procedures.

Synopsis

A320 flight crew reported numerous system malfunctions during climb and returned to departure airport.

ACN: 1596133 (1159 of 2908)

Time / Day

Date : 201811

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Hydraulic Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 10137
Experience.Flight Crew.Type : 9568
ASRS Report Number.Accession Number : 1596133
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Crew entered MRM [Maintenance Requirements Manual] to address a puddle of HYD fluid observed during walk-around that was in front of the left landing gear. The gear well was wet with HYD fluid also. Local techs responded and inspected the aircraft. A [release document] was issued which the pilots were somewhat confused by. The sign off was very contradictory. Technical Support Maintenance Control was called and he said most of the fluid was thought to be de-ice fluid from a de-ice of the aircraft overnight. Biggest problem with this is the aircraft was not de-iced overnight and the puddle was an individual puddle on an otherwise dry ramp. The totality of the circumstances strongly pointed to hydraulic fluid, but de-ice fluid is the easy go-to answer during winter ops. Crews really do know the difference.

The sign-off was contradictory from start to finish. It is not HYD fluid on ramp. It is de-ice fluid. However, it is leaking HYD fluid and will be fixed within two days. It is dripping but somehow the HYD fluid goes through a molecular change and falls on the ramp as de-ice fluid. Would be interested to see the MX outcome of this event.

Synopsis

Air carrier pilot conducting preflight inspection noticed hydraulic fluid leak around left main gear. Maintenance was called to inspect and provided a contradictory conclusion that the leaking fluid was de-ice fluid.

ACN: 1596131 (1160 of 2908)

Time / Day

Date : 201811

Place

Locale Reference.Airport : MMMX.Airport
State Reference : FO

Aircraft

Reference : X
ATC / Advisory.TRACON : MMMX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 5R
Flight Phase : Final Approach
Route In Use.STAR : AVSAR1C

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3789
Experience.Flight Crew.Type : 1160
ASRS Report Number.Accession Number : 1596131
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Commercial
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 869
ASRS Report Number.Accession Number : 1596140
Human Factors : Communication Breakdown
Human Factors : Confusion
Human Factors : Distraction
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Automation : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Crossing MAVEK on ILS 2 5R, [we were] asked what our speed was and I replied 160 knots. We had not been assigned a speed to MAVEK, and had been cleared for the approach, so we slowed to 160 [knots]. Controller said it was charted to maintain 200 knots to MAVEK, and THEN slow to 160 knots. We acknowledged him and complied with his speed assignment of 180 until advised. When I got the chance, I looked everywhere I could to find a charted speed of 200 it's to MAVEK, but still cannot find it.

Narrative: 2

After being vectored off the AVSAR1C arrival into Mexico City, we were eventually given direct to SLM and a descent to 12,000 [feet] at 220 knots assigned. I was the Pilot Flying and noticed a white and green light out of my peripheral vision passing 14,000 feet and confirmed a laser at 3 o'clock and about 3 miles away. I informed the Captain of the event, he concurred, and I shielded my vision from the laser. 

After reaching 12,000 feet Mexico City, Approach Control cleared us for the ILS DME 2 5R approach. I verbally verified with the Captain that we were cleared for the approach and verbalized I would slow to 160 knots, he agreed. We began to slow and I configured the aircraft with flaps 1 and Flaps 2, the intent was to get configured for the remaining portion of the arrival and avoid being high and fast. Prior to reaching MAVEK waypoint, the Approach Controller asked us our speed and we replied 160 knots. The controller informed us that was incorrect and that "published" speed was 200 knots. We informed him that he had cleared us for the Approach and no speed restrictions were assigned with the clearance. The controller persisted that 200 knots was the correct speed prior to MAVEK, he then assigned 180 knots. The controller then communicated in Spanish, I think to the traffic following us to gain separation. We accelerated to 180 knots, and began a descent to 9,700 feet; the descent was late due to the discussion with the controller. I called for the landing gear to help the descent and the controller then assigned 160 knots. After getting slowed back down by ATC, we configured the aircraft, complied with the rest of the approach, and landed without incident.

During the post flight debrief we were unable to find where the controller was referencing the 200 knot published speed restriction prior to crossing MAVEK. We were not sure it is published, but if ATC expects that speed crews operating into Mexico City need to know about it.

Synopsis

A320 flight crew reported ATC advised they had slowed below a published speed descending into MMMX airport. They were unable to locate the published speeds on the approach charts. First Officer also reported being struck with laser from the ground.

ACN: 1595361 (1161 of 2908)

Time / Day

Date : 201811

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ORD

Component

Aircraft Component : Air/Ground Communication
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 930.80
Experience.Flight Crew.Type : 930.80
ASRS Report Number.Accession Number : 1595361
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Flying ILS PRM 10C to ORD. Unlike at other airports, ORD uses the "Monitor" frequency as the Tower frequency for another runway, meaning in order to monitor, I am now actively listening to 2 separate Tower controllers conduct business. This causes an inability to effectively listen to either frequency; the tendency becomes to turn off the monitor frequency and focus on your assigned Tower frequency. This is of course against PRM procedures.

Synopsis

Air carrier First Officer reported ORD procedure dictates the second COM is used for as a monitor frequency and for normal Tower operations which causes distraction.

ACN: 1594794 (1162 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 23300

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Icing

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
ASRS Report Number.Accession Number : 1594794
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Flight to ZZZ, filed and cleared FL230. While in the cruise phase we decided to climb up to FL250 in the attempt to get above the clouds. Another aircraft reported moderate rime ice in the area so I was monitoring the icing probe and conditions. We were then cleared DIRECT to ZZZZZ and to descend via the [arrival] landing north. The Captain had briefed the arrival and stated we would start down at TOD.

On the descent profile we ended crossing ZZZZZ [which has a charted minimum altitude of FL240] at 23500-ish. ATC advised us of our deviation and we stopped our descent and discussed with ATC. We then continued our descent on the arrival.

Conditions during the event Night IMC in icing conditions. When we changed altitudes from FL230 to FL250 we were now above the crossing constraints at ZZZZZ but because we had reviewed the arrival prior climbing, the constraint at ZZZZZ wasn't displayed because we were below it. If the Airbus system hadn't deleted the constraint we would have seen it and trapped the error. I don't understand why the Airbus will remove restrictions on an arrival but not on an ILS.

Had we stayed at FL230 we would have crossed ZZZZZ below the published restriction but legal because we never went above FL240.

The Airbus FMS system is flawed and it is a widely known programming error in the system. On every flight release it even states that there is a problem in the program. The responsibility has been placed squarely on the backs of the pilots to catch all of the Airbus altitude Nav failures. As a pilot on the Airbus for [many] years I have never missed an altitude crossing restriction. ZZZ has so many altitude windows on the STAR it's very likely that most pilots will bust an altitude restriction eventually.

Upgrading to the latest FMS nav system program on the Airbus system might solve the altitude deviation problems. If this is not fixed it's the pilots who are the last line of defense.

Synopsis

A320 Captain reported overshooting a crossing restriction on arrival, citing a known Airbus FMS issue as contributing.

ACN: 1594734 (1163 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Window Ice/Rain System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 19400
Experience.Flight Crew.Last 90 Days : 205
Experience.Flight Crew.Type : 1320
ASRS Report Number.Accession Number : 1594734
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 6398
Experience.Flight Crew.Type : 408
ASRS Report Number.Accession Number : 1594727
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise, we had ECAMs for the Captain's window heat and a pack regulator fault. I completed the ECAMs. About 5 minutes later, the Captain's ND, PFD and the upper ECAM switched back and forth between being blank and having a diagonal line, finally going totally blank. The 3 screens came back to life about five minutes later and worked well for the remainder of the flight. Simultaneously, we had a multitude of various ECAMs. The aircraft also started to shake considerably. I assigned the FO (First Officer) to fly and I [notified ATC], initially asking for a diversion to [a nearby airport]. After realizing that [another nearby airport] was behind us and quite a bit closer, I changed my diversion choice. 

I worked through the ECAMs and required checklists. The FO and I worked very well together and I kept Dispatch, the flight attendants and the passengers informed as the workload decreased. One important note - my iPad froze while I was trying to access JeppPro to brief the approach. I had to shut it down and restart it. Prior to this, I've never had a problem with my iPad. 

The airframe vibration intensified on the approach. The approach and landing were normal, but we didn't have reversers. This was a surprise because it wasn't displayed as an ECAM and didn't show on the STATUS page. Later the flight attendants told us that there was a vibration in the rear of the aircraft and the tail was swaying side to side. This swaying was indicative of a dual yaw damper failure. This was never made aware to us because it also wasn't an ECAM and didn't appear on the STATUS page.

Narrative: 2

ANTI ICE L Window and Windshield ECAMs came on in flight, Captain verified his windshield and back window were very cold to touch (unlike the normal heated windows). He looked up info in the [Maintenance Log] and we checked the QRH which related info we already knew "left windshield heat failed." A few moments later another ECAM relating to the left pack appeared. Captain noted the left-most fuel pump light also flicker on and off without indication of an ECAM. When the Captain was preparing to send the [Maintenance Report] codes for each of the ECAMs, his PFD, MFD and top ECAM flickered to red lines on and off and went blank. 

I think the Captain said "Let's think about diverting, I don't like this" I completely concurred. Seconds later, multiple ECAM cautions began to appear with a "ding" and yellow caution occurring almost every second, 10-15 times (?). The Captain called ATC and asked for a vector for ZZZ1 which appeared ahead of us (closest large airport, ~150 miles) on our map displays. ATC gave us a lower [altitude] (280?) and a slight turn. Moments later the Captain looked at the closest airports page and noted ZZZ was behind us but only 80 NM away - we changed our intended diversion airport to ZZZ, got another vector and continued descent. Captain gave me the radios, while he worked through the ECAM msgs/checklists. I left the autoflight system on in the ~180 deg turn to ZZZ.

The system failures continued to chime and we felt a significant vibration/shaking on the flight deck (FAs later confirmed it was worse in the back). The failed systems that I recall at that point were those mentioned above plus Blue Hydraulic, Avionics cooling, a fuel pump, a pack. I kept taking deep breaths doing my own check for the smell of smoke, expecting it any moment with my O2 mask close by. We executed an ILS/Visual approach with normal speeds and flap settings, without the use of spoilers (the vibration/shaking worsened as we slowed). Landing/touchdown was normal, but neither thrust reversers developed any reverse thrust worked despite levers being in full reverse (there was no ECAM indication that stated they wouldn't be available). I let the aircraft slow mostly on its own until applying light braking. Ground fire support noted no external damage or fire so we taxied to gate.

Synopsis

A320 flight crew reported diverting to an alternate airport after experiencing multiple system failures in cruise flight.

ACN: 1593108 (1164 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Climb
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 19000
ASRS Report Number.Accession Number : 1593108
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out at around 20,000 ft we experienced a thump like we hit something. I thought it was on my side of the aircraft and the Captain thought it was on his side of the aircraft. We discussed what it might be, too high for a bird strike, maybe a panel came off, slight roll back on one of the engines, etc. We checked all systems throughout the flight and every system was normal. We sent Maintenance a message to inspect the aircraft when we arrived in ZZZ. For the next 2 or so hours all indications remained normal. Smooth flight, good weather, nice east flight. We then began our descent into ZZZ, power on slow descents, idle descents, everything remained normal. I was flying the aircraft and the Captain was the PM (Pilot Monitoring). CA (Captain) continued to check the systems throughout the descent. 

On downwind, we were given a clearance to descend to 4000 feet. I used open descent which is an idle descent because I thought it might be a short final approach. When the # 2 engine came to idle, the vibration started and from that time it stayed at idle. We saw a N1 vibration on the lower ECAM at one time of 9.9. CA immediately called and [advised ATC] and asked for priority. We were landing on XXL and the tower offered XXC so we took [it]. We proceeded on downwind and turned on an 8 to 10 mile final. Tower got us on the ground quick. CA asked if I was comfortable landing the aircraft and I replied yes and continued. We landed the aircraft and rolled to a stop close to where the fire trucks were. 

During the rollout, we got an ECAM message that the engine failed and tower called and said we had fire coming out the tail pipe. At the end of the roll out CA took the aircraft came to a stop and set the parking brake. He then called for the evacuation checklist so we would be ready because by that time we were already talking to Fire Rescue who had pulled up several trucks and were hosing down the right engine. We never got any kind of fire indication on the FLT deck. They told us not to evacuate. We stayed on the runway for about an hour while Fire Rescue and maintenance made the decision on what to do with the aircraft and passengers. They brought out a super tug and towed us to a gate. 

This is about the best I can do to describe what happened. During the evacuation checklist, CA did blow the fire bottle on the #2 engine as Fire Rescue was approaching but he elected not to blow a bottle on the #1 engine which was fine by me since nothing was wrong with it, Fire Rescue was approaching, and he had not given the evacuation command.

Synopsis

A320 First Officer reported problems with the #2 engine, including significant vibration indication, engine failure ECAM and visible flames from the tailpipe during landing roll.

ACN: 1591451 (1165 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 27200
ASRS Report Number.Accession Number : 1591451
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

As some of our older A320 family aircraft come of age I am encountering more delaminating or bubbling forming along the edge of cockpit windows. 20 years or so I had an A320 window internally catch fire during cruise at night between the window plies at the window heating element due to window delamination. On postflight I noticed that this particular aircraft had delamination hazing starting along the edge of the L3 window, for which an AML (Aircraft Maintenance Log) entry was made to have checked, and was found to be within limits. I have been told over the years by Maintenance that so long as the delamination is less than 3 inches wide and not within the direct field of view it is within limits. I have never run across a window even close to those limits. However, I thought my write-ups would [have at] least put the window on some sort of a monitor or watch list. I am told by Maintenance Control that is not the case. Furthermore, there is no way for a pilot to know if the window has already been written up in the past. In short, it would be helpful to have additional preflight guidance, similar to the preflight limitations we have for tires, to know what the window delamination/hazing limits are. As well as guidance on if and when an AML entry is appropriate. Perhaps an entry could also be made to the aircraft damage report once a window has been reviewed and found to be within limits to avoid repeat write-ups.

Synopsis

A320 Captain reported the flight deck crew needs window delamination limits and guidance on when to enter the damaged window into the logbook.

ACN: 1590911 (1166 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
Make Model Name : UAV - Unpiloted Aerial Vehicle
Flight Phase : Cruise

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Type : 548
ASRS Report Number.Accession Number : 1590911

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1590917

Events

Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
Miss Distance.Horizontal : 1000
Miss Distance.Vertical : 500
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

A drone passed us about 500-700 feet below us and about 1000 feet to the aircraft's right as we were doing a visual approach.

Narrative: 2

While approaching DEN from the SE, on a right base for Runway 35L, we saw what appeared to be a drone approximately 500 feet below and to our right. We were level at 11,000 feet. The drone appeared to be in the shape of an octahedron, approximately two feet by two feet in size, and was heading east. We reported it to ATC.

Synopsis

Air carrier flight crew reported a conflict with a Drone on approach to DEN.

ACN: 1590638 (1167 of 2908)

Time / Day

Date : 201811
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : C90.TRACON
State Reference : IL
Altitude.AGL.Single Value : 2500

Environment

Flight Conditions : Marginal
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ORD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1590638
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

We were cleared to fly the ILS PRM Approach. We selected the monitor frequency at the proper time. We kept hearing two different tower voices giving us instructions and it appeared one was on the correct tower frequency and the other on the monitor frequency. I queried the tower if they were giving instructions on both frequencies. I was advised they were. I told them I didn't think that was the purpose of the monitor frequency and was told they would look into it. Ironically, my First Officer said it happened to him on another flight recently. He inquired as to its legitimacy. He was told then, it would get looked into.

I don't think that's the purpose for the monitor frequency, at least not according to the FOM. The monitor controller is to monitor deviations and respond accordingly to the crew, not give speed reductions and as happened to me earlier this fall, landing clearances on the monitor frequency. My understanding is they are to monitor the No Transgression Zone (NTZ) and issue instructions if that airspace is violated.

Synopsis

Air Carrier pilot reported receiving excessive transmissions on the PRM frequency.

ACN: 1590098 (1168 of 2908)

Time / Day

Date : 201810

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class B : ORD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 390
ASRS Report Number.Accession Number : 1590098
Human Factors : Workload
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Flying ORD with ZZZ as an alternate. We were outside the final approach fix to 27R beginning the configuration process when we noticed the following [traffic] go around. The weather was at least 20 north of the field so I didn't think anything of it. We were fully configured and got a reactive wind shear warning and went around (disclaimer the Captain was convinced it was predictive WS however I don't recall the aircraft saying WS ahead). I followed the [applicable] guidance then cleaned up afterwards. ATC provided zero help, initially they asked if we needed help but a few seconds later changed their minds on heading and altitude at least 5 times (maybe more) increasing our workload significantly. At one point, we were climbing to 5,000 and they asked for a level off at 4 when we were passing through 3,700 (and obviously overshot it) and I disconnected the AP and lowered the nose. With the AP back on we decided to divert to ZZZ1, our planned ALT was on the other side of the weather and both of us didn't want to fly that direction. The CA (Captain) gave me the plane and spoke to dispatch and let them know of our intentions, get weather and NOTAMS and request landing data. We landed uneventfully on XXR with zero landing data (we [alerted] min fuel, had no time to wait around for dispatch to send it to the aircraft). We landed in ZZZ1 with 4.3 and taxied to a remote pad where we were refueled.

Synopsis

A320 First Officer reported an altitude overshoot due to ATC issuing several heading and altitude changes during heavy workload missed approach.

ACN: 1589744 (1169 of 2908)

Time / Day

Date : 201810

Place

Locale Reference.Airport : PIT.Airport
State Reference : PA
Altitude.MSL.Single Value : 3500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : PIT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : PIT

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 786
ASRS Report Number.Accession Number : 1589744
Human Factors : Distraction

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Conducting the visual approach, RNAV back up Runway 28C in PIT. ATC instructed maintain 3,500 feet until established on final. Crossed FAF about 500 feet high to comply. As I was making a positive correction to get the aircraft on vertical profile, received a "Terrain, Terrain". This occurred above 2,000 feet. Made slight correction, warning ceased and landed uneventfully.

Synopsis

A320 Captain reported being high crossing FAF then receiving "terrain, terrain" warning from aircraft.

ACN: 1588968 (1170 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 2950
ASRS Report Number.Accession Number : 1588968

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While operating at FL260 we began to observe a significant increase in engine Number 1 N1 vibration. Specifically, engine Number 1 N1 vibration was indicating approximately 3.3 and rising, while engine Number 2 N1 vibration was indicating approximately 0.2. We were in cruise flight at FL260, in icing conditions, with engine Number 1 [and] Number 2 anti-ice selected ON. The engine Number 1 N1 vibration indication continued to rise steadily to 5.0 and above and we could physically sense abnormal vibration on the left side of the aircraft. As this occurred, we initiated QRH "Eng - High Vibration". As we executed "Eng - High Vibration", we experienced a decrease in engine Number 1 N1 vibration to an approximate value of 0 when engine Number 1 thrust was reduced to, or near, idle. However, once thrust was increased to cruise, engine Number 1 N1 vibration increased and was eventually sustained at approximately 9.4. In accordance with the continuance of "Eng - High Vibration", we increased engine Number 1 thrust to 90% N1. No change in engine Number 1 N1 vibration occurred. The engine Number 1 N1 vibration value continued to be sustained at approximately 9.4. As the aforementioned occurred, we requested and received clearance to climb to FL320, in an effort to exit icing conditions. Once level at FL320, we were, in fact, clear of icing conditions.

Upon completing "Eng - High Vibration" QRH with no reduction in engine Number 1 N1 vibration at cruise thrust setting, we continued flight with engine Number 1 thrust at, or near, idle. We advised Dispatch of our [issue]. At this point, we were in range of [destination] (approximately twenty minutes from landing) and had begun the descent and arrival phase of the flight. We attempted to establish communication with [Maintenance] via Dispatch. However, as we descended, we lost radio communication with Dispatch. I provided a briefing to our Purser Flight Attendant. We continued our approach and arrival. We landed without incident, taxied to gate, and concluded the flight.

Synopsis

A320 Captain reported high vibration in Number 1 engine that led to operating the engine at or near idle for the latter part of the flight.

ACN: 1588863 (1171 of 2908)

Time / Day

Date : 201810

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Communication Systems
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1588863
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Dispatch
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The NEO aircraft are continuing to get a large number of ACARS MELs wrote up by pilots due to the static on the radios caused by ACARS messaging bleeding thru on the VH1 radio. I heard the pilots have a memo saying that they are getting a lot of feedback over the VH1 radio whenever the aircraft is reporting positions and other info thru ACARS VH3 radio. Dispatch seems to be the last to know on pilot memos. We used to be able to review pilot memos thru the company site, but this is no longer available for dispatchers. The main problem is when they put the ACARS inoperative on these aircraft it is causing an even more safety risk, not only for the dispatchers when they have to use ARINC but also ATC trying to communicate. There have been a few cases where we are not able to communicate with the aircraft. A recent issue involved an ATC Center missing a transfer of their frequency to a flight. ATC Center calls Dispatch and requests the dispatcher pass on the frequency to the aircraft. The dispatcher taking the call from ATC might not be the dispatcher that has operational control and sends the ACARS of the frequency not knowing ACARS is inoperative. ATC calls again asking the dispatcher to give a frequency to the aircraft again, this time the dispatcher that has the flight notices the other dispatchers were sending frequencies via ACARS to an aircraft that cannot receive ACARS. The dispatcher with operational control calls ARINC and they cannot get ahold of the flight either [due] to too many squawk s on the radio. The crew finally takes their own initiative and calls an airport and [they] get ahold of ATC. By this time a violation is issued to the pilot for non-communicating with ATC. [Suggestions include] pilot memos need to be distributed to dispatch and let the dispatchers decide which ones that don't pertain to dispatch on a need to know bases. Instead of the dispatchers being blindsided. Dispatch Resource Management (DRM) on operational control of another dispatcher needs some work. No dispatcher should be making decisions for the other dispatcher when they are in the room and available. Airbus needs to be if they have not been already and fix these new aircraft from getting frequency feedback thru ACARS reporting. Flight Ops memos that might involve dispatch need to be forwarded to dispatch.

Synopsis

Dispatcher reported that Airbus A320 NEO aircraft are getting a large number of ACARS MEL's and the dispatchers are not included in the loop and thus flight crews are not receiving Dispatch's ACARS messages.

ACN: 1588862 (1172 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1588862
Human Factors : Troubleshooting
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.General : Physical Injury / Incapacitation
Result.General : Maintenance Action
Result.General : Evacuated
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

Upon starting APU and turning on bleed air, we immediately noticed on flight deck a medium/strong smell of burning oil/stale smell. I asked [the] First Officer to concur which he did while opening his flight deck window. I turned off pack #1 and I started to call B Flight Attendant in back, but before I could she called me and stated that they had a very oily suffocating smell in back of aircraft. I immediately turned off APU bleed air and pack #2. I pulled up the odor removal checklist in FOM and told flight attendants to open up back doors to get air flow through cabin. Contacted Maintenance Control and Dispatch. Flight attendants complained of headache, sore throat, stinging eyes. My First Officer and I only experienced sore throat and stinging eyes. We de-boarded aircraft per [Operations Control]. Passengers were sent to another aircraft. EMS responded to check out crew. I told [the] flight attendants they certainly could go to hospital to be further evaluated-they did not. I completed the odor/fumes report. Crew was released as they had replaced all of us. These events are occurring far too frequently. The pack burnout procedures are not always being followed correctly. This is a safety issue for both crews and passengers. Once a pack is contaminated with oil, in my opinion it needs to be replaced, even though this is a huge economic constraint. You can't get much of the contaminants out of system.

Synopsis

A320 Captain reported after starting the APU and turning an air conditioning pack on, the cabin filled with fumes.

ACN: 1587711 (1173 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Horizontal Stabilizer Trim
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1587711
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1588019

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise, got a status message "Cat 3 single only" inoperative systems "Cat 3 Dual". Moments later it cleared. Moments later, got an ECAM FLT/CTL ELAC 1 PITCH FAULT. Complied with our procedures and completed the ECAM. Its status message was the same: "Cat 3 single only" - inoperative systems "Cat 3 Dual". Next A/P 2 disengaged. We engaged A/P 1. Moments later, A/P1 disengaged. We started to hand fly the aircraft. Shortly thereafter we got ECAM FLT/CTL ELAC 2 PITCH FAULT. and complied with our procedures and completed the ECAM. Now we were in alternate law and knew we were going to land in direct law. We noticed the pitch trim was giving short bursts and realized that was what kicked the A/P off. At this point, we [asked for] the longest runway at [destination]. I tried calling Dispatch and [Operations] on the Dispatch frequencies [listed in the manual] but it was too scratchy and [instead] used the ARINC frequency dispatch ACARS'ed us. Approach and landing was uneventful.

Failure of ELAC systems.

My first attempt was the new call app on our satellite equipped aircraft. It did not work. Then the dispatch frequencies in FM2. Had connection been better, communication with the Dispatcher and [Operations] would have occurred much quicker.

Narrative: 2

We also received an EGPWS terrain fault and a FMC 1 failure.

Synopsis

A320 flight crew reported loss of multiple systems apparently related to pitch trim malfunctions that resulted in reversion to alternate/direct law.

ACN: 1587601 (1174 of 2908)

Time / Day

Date : 201810

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 11190
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 7221
ASRS Report Number.Accession Number : 1587601

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1587603

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Anomaly.Deviation / Discrepancy - Procedural : Security
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Pre-flight
Result.General : Police / Security Involved

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Prior to boarding, noticed two individuals in jetway boarding early. Bearded and wearing what appeared to be biker leather vests and one of them dropped the f-bomb while laughing and walking down jetway. At this point I thought they might have just been enjoying a few drinks in the bar while waiting on our two hour delayed departure, but was not very concerned since nobody else was in the jetway. Agents were very motivated to try and turn the aircraft quickly to salvage connections. Told First Officer that biggest threat right now was the the time pressure being applied. To mitigate that we would take our time and methodically accomplish preflight items and checklist and go only when we were ready. They agents were pushing to get everyone boarded and door closed asap. As boarding was concluding (last 10 passengers boarding), the lead FA (Flight Attendant) advised that we might have a problem. Two individuals with military ID's had boarded early and appeared to be intoxicated. He was not sure if this was going to be a problem? Shortly thereafter, the other FA came up to express that the passenger in XXC who boarded early was slurring words and potentially disruptive. He was concerned that this individual might become problematic during our 3+hr flight and wondered why agents had boarded passengers in this state. I asked them if they were comfortable and they hesitated in responding so I asked to have CSR (Customer Service Representative) go back and assess condition of passengers. If he was intoxicated then I conveyed we would not be taking him. While agent was going back to check on passengers, held further discussions about pitfalls of taking passengers with First Officer and FAs. Also contacted [company] to briefly advise of situation. Agent returned to cockpit, and provided update. I conveyed that we would like passengers deplaned based on his prior actions and statements and concern expressed by FAs. At this point passengers came from XXC to front of cabin. As agents and FAs were talking with him to essentially convince him why he should deplane, he asked to speak to the Captain. I could see the situation escalating from my seat and cell phone cameras were coming out. I told FA or CSR that I'd be happy to chat with him in the jetway but not on the aircraft. He declined that invitation and refused to get off the airplane. At that point the decision to remove him from the flight was clear. The friend who was traveling with him (XXC had told FAs the other gentleman who boarded with him was his brother, but we later learned this was false. They had met in the bar approximately 4 hours prior) was able to convince him to deplane. At some point during this event the CRO (Complaint Resolution Official) showed up to deescalate the situation and was apparently met with resistance by one of the CSRs. passengers voluntarily deplaned and we pushed and departed. [Later] Dispatch asked us to come up on Commercial Frequency. Once connected we learned that passenger in XXC, after being removed from flight, allegedly made statement to CSR in boarding area that there was [an object] on board our flight in the bag he checked and that this could be considered a potential Level 3 security threat if he was considered credible (second criteria used to determine legitimacy of threat). Conferenced with [company], and Corporate security and consulted with Lead FA. After all available and reviewing threat Levels, requested recommendations and input from all parties. Determined that the threat was not credible. Switched off Wifi to prevent social media event and to contain the situation.

Narrative: 2

Removed drunk passenger off plane during boarding. During our final climb segment, a message from the company to contact them was received. Drunk passenger, that was kicked off our plane, made a threat to the gate agent that there was [an object] on the plane that he had been kicked off of. Captain immediately transferred controls and radios to me (FO) to contact [operations]. Drunk passenger had a checked bag still on flight in the cargo compartment (TSA screened). After much consideration, the inflight security coordinator (Captain) made the correct decision, in this situation, to continue on to [destination]. I think the Captain did an excellent job in this situation.

Synopsis

A320 flight crew reported an unruly/intoxicated passenger that was deplaned later claimed that there was an object in checked luggage. Threat was determined not to be credible and flight continued to destination.

ACN: 1587598 (1175 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 15000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : FRNCH THREE
Airspace.Class E : D01

Aircraft : 2

Make Model Name : Small Aircraft
Operating Under FAR Part : Part 91
Airspace.Class E : D01

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7800
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1639
ASRS Report Number.Accession Number : 1587598
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10977
Experience.Flight Crew.Last 90 Days : 213
Experience.Flight Crew.Type : 10823
ASRS Report Number.Accession Number : 1587602
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Conflict : NMAC
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Descending VIA FRNCH arrival STAR, ATC leveled us at FL190 then was stepping us down a few thousand feet at a time. In the vicinity of TOMSN intersection at or near our assigned altitude of 15,000 feet we encountered a light aircraft between 300-500 feet above us traveling in the opposite direction. I was making a radio call to Approach when I saw the aircraft and said "AIRCRAFT" before releasing the microphone. Time from visually acquiring the aircraft to determining it wasn't going to impact us to having it pass was approximately 3-5 seconds. I then reported "near midair collision" to ATC. ATC informed us of traffic at 13,000 feet which I saw below us. ATC then reported a VFR target squawking 1200 shortly after. The aircraft may have turned on their transponder after seeing an Airbus nearly hit them.

A few miles earlier I had noticed an unfilled white diamond symbol on my TCAS, however it was not in our flight path. We received no TCAS RA, Traffic Alert or any other guidance or warning.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported an NMAC with a single engine light aircraft in the vicinity of DEN.

ACN: 1587173 (1176 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 16000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2907
Experience.Flight Crew.Last 90 Days : 185
Experience.Flight Crew.Type : 519
ASRS Report Number.Accession Number : 1587173
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While operating last flight of the day we received an amended PDC clearance to climb via PITZZ4 except maintain 4,000 ft. We climbed via SID after takeoff with no issues. We leveled at 4,000 ft. When switching to Houston Departure we checked in and the instruction was climb via SID, that's it. The SID clearly states to climb to top altitude of 16,000 ft. so we did. Upon checking in with Houston Center we were inquired at what altitude we were, to which we replied that we were told to climb via SID and therefore we climbed to 16,000 ft. and not our final filed altitude of 14,000 ft. Upon the controller going off frequency and checking with previous controller, he came back and said climb and maintain 16,000 ft. Both Captain and I thought this was a big gotcha as the controllers are supposed to always give an except altitude if other than what's on the SID. We proceeded to [destination] with no further issues but definitely with a lesson to take home.

Synopsis

A320 First Officer reported altitude deviation and misinterpretation of a climb via clearance.

ACN: 1587117 (1177 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1587117
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The flight blocked out two minutes late. After starting Engine 2, an ECAM message for AUTOFLT RUD TRIM 2 FAULT appeared. ZZZ Ground Control was advised that we needed some time to address a mechanical discrepancy and they requested that we move out of their way and taxi southeast on Z holding short of C. The ECAM message was addressed by calling Dispatch, being transferred to [Maintenance Control], and deferring the fault via the MEL (two MEL's, one being a CAT downgrade). The MEL Operations Procedure was lengthy and required some extra minutes to complete.

While we were waiting [on the documentation] for the MELs plus a reduction in brake release fuel, Flight Attendant C contacted the flight deck to report a passenger in or near the exit rows had flagged her down to report an unusual smell of "burning fuel." Flight Attendant C stated she also smelled it and agreed with "burning fuel." I requested all flight attendants assess the odor. Three of the four flight attendants stated they could smell it with Flight Attendant B saying it burned her nose slightly and Flight Attendant A saying it irritated his throat somewhat. There was no rush to judgment at this time because we were still waiting for the [documents] anyway. The APU was started and the APU Bleed was switched on and off with Pack 1 and Pack 2 turned off alternately with the intent to eliminate the odor or isolate the source. Eventually more passengers reported smelling the odor too. The flight attendants reported that the smell would go away and then they would call back shortly thereafter to say the odor was back. Overall the odor was persistent and we had delayed somewhat hoping it would go away. No odor was apparent in the flight deck at this time. At this point, Dispatch was contacted, the transfer to Maintenance Control was made, and a decision to return to the gate was made. 

Upon arrival at the gate with the flight deck door opened, Flight Attendant C forward to tell us she wasn't feeling well. When I asked if she needed medical assistance she said yes because she was feeling light headed and having some difficulty in breathing. I asked the First Officer to phone STAT-MD while I advised the gate agent to request EMTs to come to the aircraft. STAT-MD told the First Officer to call back when the EMTs arrived and to provide the vital signs. Then, I inquired about the fitness of the other flight attendants and all reported they were okay. Dispatch was advised of Flight Attendant [C] illness. At no time was I aware of any passengers showing any symptoms or claiming to be affected by the odor.

Paramedics arrived at the gate took Flight Attendant C to a medical facility for further treatment. 

Because pilots are prohibited from going to the cabin to assess odors, perhaps more training for flight attendants to assess and better identify the intensity and effect of these odors could provide pilots with better info to make a "go" or "no go" decision.

Synopsis

A320 Captain reported returning to gate due to fuel smell reported by passengers and flight attendants.

ACN: 1586618 (1178 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 23000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Galley Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 1047
ASRS Report Number.Accession Number : 1586618

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1586899

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout passing FL230, the Purser called the flight deck and said that when she opened the oven door some smoke came out of it. As she was talking she said she could now see flames at the back of the oven. I gave the pilot flying and ATC communication responsibilities to the First Officer and got on the interphone with the Purser. As she provided more details, one of the aft galley flight attendants was coming forward, saw the smoke and flames, grabbed the halon extinguisher from overhead row 1 on right side (1EF) and extinguished the fire. The other aft Flight Attendant saw the fire extinguisher being taken down and came forward with her handheld device and assisted by running their fire checklist. It happened fast and I was impressed by how quickly they assumed the roles of fire fighter, dedicated communicator and assisting crew member. I heard them clearly over the interphone working together. The flight attendants notified me that the fire was out and pulled the oven circuit breaker. They said it looked like a piece of paper on the side of the oven ignited when the oven door was opened. We got out the QRC and referenced the SMOKE/FUMES and REMOVAL OF SMOKE/FUMES checklists although we had no indications in the cockpit and there was no longer fire or smoke. We discussed whether to declare an emergency, but decided it was not necessary. The fire was extinguished quickly and the smoke did not persist. I contacted Dispatch via ACARS and received a radio frequency. I told Dispatch that we had an oven fire, that it was caused by a piece of paper in the oven, that it was extinguished and that there appeared to be no damage. I had the First Officer go back to confirm this. He took pictures for me and concurred that there was no damage. It appeared to have been caused by a piece of paper taped to the outside of the oven insert that the meals come in. The paper was in contact with the heating elements on the left side of the oven. No damage was observed to the oven either internally or externally. No smoke remained, but the odor from the burned paper lingered in the cabin. We cleared it by following the CABIN AIR POOR QUALITY procedure. I was concerned that we would not have something to feed the first class passengers, as the hot breakfast items were ruined by the halon. The purser said she had plenty of cold items as well as anything from the aft galley to offer them and the passengers were happy with that, appreciative and seemed to be impressed by how the flight attendants handled the incident. I asked the flight attendants how they felt. They said that, aside from a bit of an adrenaline rush from the excitement, they were comfortable continuing on. I conferred with Dispatch and decided that, since there was no damage and we had the ability to care for the passengers, the aircraft was safe and comfortable and we could continue. I then made an announcement to the cabin. I explained that the smell in the cabin came from the burnt paper in the galley oven, that it would go away shortly, that there was no damage and we were safe to continue.

Narrative: 2

[Report narrative contained no additional information]

Synopsis

A320 flight crew reported continuing to destination after the cabin crew successfully fought a fire in the forward galley oven.

ACN: 1586101 (1179 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Hydraulic Auxiliary/Standby System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1586101
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Aircraft arrived over 5 hours late due to previous maintenance issues at previous stops. Of the 8 MELs this aircraft was currently carrying, 2 were significant: SPLR #2 SV Spoiler Surface 2 or 4 and IR2 inoperative. Both of these MELs impacted aircraft performance. When the aircraft arrived in ZZZ, the previous Captain had written up the Alternate Brake Accumulator for depleting after 2 brake applications (FOM indicates there should be at least 7 applications to a fully charged accumulator).

ZZZ maintenance arrived at the aircraft and began to troubleshoot the accumulator and was servicing the accumulator at the Yellow Hydraulic service point. After servicing the accumulator and checking the pressure at the service point, the maintenance specialist came up to the cockpit to indicated that the accumulator was in the normal range and suggested we activate the brakes (via accumulator pressure) to ops check it. Again, the accumulator depleted after 2 applications of the parking brake. I again indicated that our manuals indicate we should have at least 7 applications and this time he indicated that the [Maintenance Controller] said it only needed to be 2 applications and clearly we were getting that. 

He left the cockpit to discuss with [Maintenance Controller] and decided to re-check the accumulator at the service point and came back to indicate it was in normal limits. I reiterated that the only indications I could count on in the cockpit was the triple indicator and the system's display and had no way on knowing what the accumulator was indicating at the service point in flight. While he accepted this, he was clearly following the guidance he was getting from [Maintenance Controller] and thus, after discussing the issue with [Maintenance Controller], he came back to the cockpit and began to sign it off in the log book. I asked him if the system was fixed and he said he had followed the guidance and [Maintenance Controller] concurred, directing him to sign it off. I re-checked the alternate brakes and the accumulator depleted again after 2 applications. 

This frustrated me immensely and I told him I believed he was signing off an unsafe aircraft. He called the [Maintenance Controller] to discuss and I asked to speak with the [Maintenance Controller] controller to indicate my concerns. He listened and then dismissed my concerns saying it was not my job to evaluate the maintenance procedures and that they had complied with the procedure and the aircraft was safe to fly. I took great exception to this dismissal and indicated that it was not safe to fly and I would not accept this unsafe condition. I was frustrated with the [Maintenance Controller] and he said enough and handed me off to the supervisor. I told him the same information and that I was required to have at least 7 applications before the accumulator depleted and that this solution was not acceptable. [Maintenance Controller] continued to troubleshoot to no avail and was adamant that 2 applications were the correct answer. 

Finally, the maintenance specialist asked to take a picture of our FOM reference so that he could send it to [Maintenance Controller]. After multiple attempts to deplete and re-charge the accumulator, I finally made the decision that this was not going to be resolved. The passengers had been on the aircraft for well over an hour and had already been waiting over 5 hours at the gate for this flight. I had been keeping them appraised of the situation but reached a point where I didn't believe a solution was going to be realized so I had them deplane. 

I called the Chief Pilot's Duty Phone to apprise them of the situation and my concern that [Maintenance Controller] was pushing us to take an unsafe aircraft. I indicated that our books (FOM) said to expect at least 7 applications and that [Maintenance Controller] wouldn't accept this and was challenging it. I said that after working with [Maintenance Controller] and the maintenance specialist for nearly 2 hours, we were going nowhere and I would not take the aircraft under these circumstances. I also looked up the MEL for the triple indicator in case it was inoperative and pointed out to the maintenance specialist that I still needed an acceptable indication on the systems page, and we didn't even have that. 

Throughout this entire situation, there was no doubt in my mind that both my judgment along with our FOM procedures were being challenged and discarded by both [Maintenance Controller] as well as the [Operations Control] who was giving in to the pressure of another flight needing the gate we were at the expense of ensuring the safety of this flight, its 127 passengers, and the crew of 6. In over 35 years of flying, I have never been pushed as hard to accept an unsafe aircraft as I was in this situation. I shared with the [Operations Control] that pushing someone like they were pushing me was getting [Company] dangerously close to an aircraft accident and that we needed to work together as a team to ensure the safe operations and not to discard the inputs of the crew when they believed their operating procedures were being violated. Finally, on top of all of this, the Yellow hydraulic system was grossly over-serviced and when pointed out to the maintenance specialist, he was unsure how to read the gauge and further made me uncomfortable with the competence of those advising me this was a safe solution. 

Unfortunately, the management within [the Company] is willing to sacrifice safety to sustain and improve their financial bottom line. The pressures all organizations (crew, maintenance, [Operations Control], etc.) are under are creating an environment where a lack of trust and poor communications fosters a "make it happen at all costs" culture at the cost of a safety-first mentality. I have been in aviation for over three decades and have led organizations larger than [Company]. This is a leadership and management issue that permeates from the front office of both [second Company] and [Company]. It is only a matter of time before there is an aircraft accident if this is not addressed. Safety-First and teamwork is the only remedy to this issue and leadership needs to create this culture.

Synopsis

A320 Captain reported refusing an aircraft that was returned to service from maintenance after an issue could not be satisfactorily cleared.

ACN: 1586018 (1180 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAU

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6112
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 595
ASRS Report Number.Accession Number : 1586018
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14800
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 1731
ASRS Report Number.Accession Number : 1586029
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We were operating Aircraft X to ZZZ. We were in cruise at 21,000 in clear air, with smooth flight conditions and no restrictions to visibility. The seatbelt sign was off. With no indications or reports of turbulence at our altitude, we suddenly encountered Moderate to Severe turbulence. The aircraft oversped by 3-5 knots and the autopilot disconnected. I was the pilot flying and immediately took control of the aircraft. The turbulence action plan was immediately initiated. The Captain turned on the fasten seatbelt sign and immediately made a PA for the flight attendants to be seated immediately. We then requested a descent to 18,000 and the ride improved as we passed through 20,000. The Captain then had the flight attendants check in where we were notified of two possible passenger injuries. One was to a woman in the Lav who hit her head on the door but stated that she was fine. A male passenger reported hitting his knee on a galley cart. He requested ice and said he was fine. The Captain initiated contact with dispatch and Medlink and requested that paramedics meet the airplane upon arrival in ZZZ. The rest of the flight was uneventful. On taxi in, the lead Flight Attendant called to inform us that another male passenger was complaining of back pain. We arrived at the gate, shut down, and the paramedics were there to offer assistance to the injured passengers.

Narrative: 2

Level at 21,000, CNOTA intersection. Ride went from smooth to moderate/severe turbulence in 5 seconds or less. Used Turbulence [report] commands. When Purser checked in with me, she had reports from two passengers that had been mildly injured (one in lavatory bumped head, one standing in aisle hurt knee). Approaching top of descent into ZZZ another passenger reported a hurt back, this passenger would go to the hospital for evaluation. While the first two passengers did not request medical attention, my FO (First Officer) and I agreed that with two injuries we would go ahead and have medical assistance meet the aircraft. Filed a PIREP through ACARS, notified Dispatch who correctly suggested MedLink as backup. With nothing more than mild injuries, we continued to ZZZ. 

Further info: There was turbulence forecast 5000 feet above us, and ATC did report that we would not like the ride higher but nothing worse than light at our altitude. Our dispatcher mentioned it in the release, and filed us lower to stay out of it. I even commented when accepting the release that I had been thinking the same thing. We were completely not expecting what happened. Purser informed us later that the aft flight attendants had just gotten the cart to the front of coach to start their service. Quite frankly, we're lucky more weren't injured. My 5 seconds or less estimate above for turbulence onset is likely long with time dilation in the heat of the moment. In my 25 years I've never seen turbulence hit so hard, so fast, with nothing forecast or reported in the vicinity. There wasn't time to get the seat belt sign on - and indeed I forgot it until I heard our Purser make a PA for the passengers to fasten their seat belts. I'm usually the one that would rather call MedLink proactively. This time, I did not due to my perception that nobody was really injured. I was clearly wrong here. 

Finally - I'll admit that I've rolled my eyes a bit at the Turbulence [report]. I still think we are a little over-sensitive to light turbulence; this trains the passengers that the seat belt sign doesn't signal that it's not safe to move around. However - the Turbulence [report] was perfect for this event. Perfect. The commands were excellent, and I was glad to have the guide, the training, and the shared language for what needed to happen in the flight deck and the cabin. I'll go ahead and say it: I was wrong.

Synopsis

A320 flight crew reported unexpected severe turbulence, resulting in passenger injuries.

ACN: 1585742 (1181 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cowling
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1585742
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

During the preflight exterior inspection damage to the number 2 engine cowling was discovered/failure of the material used to construct the inside of the cowling near the tips of the fan blades/a hole developed. The damaged engine cowling was placed on deferral using a Tear Down Inspection ("TDI") as there isn't a CDL that applies. Aircraft was returned to service with the hole still in the cowling, no repair attempts were made before flights with revenue passengers on board. Subsequent flight crews operating aircraft may easily miss the damage to the number 2 cowling if the fan blades were parked in a manner that covered the hole. TDIs are not listed on dispatch release and there are no placards anywhere on the aircraft identifying the damage to alert the next crew. Please review and respond...was this the correct way to "fix" this particular mechanical discrepancy? At the very least, please list all TDIs on the dispatch release and highlight them on the cover of the aircraft logbook so that each crew is fully aware of the current condition of their aircraft. Furthermore, in hindsight I have concerns regarding the safe operation of company aircraft that are dispatched for revenue flights with holes developing in the engine cowling, cowling material eroding away without predictability - just millimeters away from spinning fan blades inside the engine cowling.

Synopsis

A320 Captain reported a hole was discovered on the inside cowling of the Number 2 engine during the exterior inspection. Aircraft placed on a work order and returned to service with hole still in cowling. Captain is concerned about this procedure.

ACN: 1585236 (1182 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1585236
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I understand that safety third is sometimes part of our occupation, however clearing us legally as opposed to safely was grossly misused in this scenario. I see these situations frequently and believe it is primarily due to a generation of ATC who have not been freely allowed access to the cockpit operations in real-time, as they were prior to the events of 9-11. Why are helicopters and airliners in the same space and altitude? This is ridiculous and I have had many TA alerts on approach and a couple of RA alerts due to this conflict. ATC often asks for things that will obviously effect the stability of an approach. I sincerely believe that there is a bit of disconnect between ATC and pilots, each not knowing just what the other is required and/or able to do. Ride alongs would be nice again, and not what is currently in place. Something more along the lines of ATC having the access to jumpseat in our cockpits on their time off. I feel that some administrators made the decision to "tie their hands" and only allow structured jumpseating and only during paid shifts because they didn't like the idea of controllers having access to run off to the beach on their time off. I would love for them to run off to the beach. I don't care where they jumpseat to or why, I care that they observe and pass along what they see to their peers. The more access, the better understanding for both sides. The ATC/pilot relationship was much more in sync when they had that freedom.

Synopsis

A320 Captain reported being issued a visual approach while ATC left them unreasonably high.

ACN: 1585174 (1183 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1585174
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We arrived on the flight deck, got situated, and before we had a chance to look at the log book, someone threw the paperwork down on the center console. I looked at the release and noted two MELS (among several):

ENG 1 Minor Fault.
ENG 2 Minor Fault

As soon as I read the release, I said to the First Officer "that's not right". You can only have one ENG minor fault on MEL. I called Maintenance Controller and explained the same to the controller and he immediately agreed. The Contract Maintenance Tech was just there (at the direction of Maintenance Controller) correcting the log book control numbers from the previously applied [placards].

The Contract Maintenance Tech was still on board and I briefly explained to him the problem. These MEL's cannot be applied on both engines and he needs to contact Maintenance Controller for direction on how to correct this error. He called Maintenance Controller, took the log book, and left the aircraft.

The Contract Maintenance Tech was gone for about 30 minutes then returned to the flight deck with the log book already filled out. I'll paraphrase the entries.

Cleared ENG 1 minor fault.
Cleared ENG 2 minor fault.

I looked at the Contract Maintenance Tech....Seriously....what did you do to clear these faults? We've been here the entire time and nobody's worked on the aircraft. Where's the "performed FADEC bite test, no faults found...? Where's the "Reset EIU computer, fault cleared, OK to continue...? Where's the ball peen hammer and the roll of duct tape...?"

Where's the maintenance action performed to clear these MEL's...??
He looks at me and says... "you really wanted me to do that...?"
YES...!!!

I called Maintenance Controller but could not get through. I called and spoke with the Dispatcher and explained the situation; he was equally appalled and said he would forward the info to Maintenance Controller.

The Contract Maintenance Tech returned with his Supervisor who appeared to be much more experienced. The Maintenance Supervisor (who was not a party to this event until after the damage was done) began a speaker-phone discussion with Maintenance Controller about how we got here. 

Do I have to say anything....It's utterly ridiculous that I even have to send this report. This event clearly defines the breakdown of the operational integrity of this airline.

FAA...Do something about it....!

Flight Crew Placards should be discontinued completely. There is no Maintenance follow up, no Maintenance validation, no Maintenance back-up, [and] no Dispatch support. Once entered into the log book, the aircraft may continue on for 10 days without any double check (outside of flight crew) as to whether the MEL was applied correctly or appropriately.

Synopsis

A320 Captain reported refusing the aircraft until the appropriate maintenance documentation were completed. The Captain also included that this is a reoccurring issue.

ACN: 1584665 (1184 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1600

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class C : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 26000
ASRS Report Number.Accession Number : 1584665
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was pilot Flying (PF). Late configuring flaps on [a] night visual approach to ZZZ RWY XXL. Hand flying with auto throttles off. Cause [was] misinterpretation of final approach fix location on Jeppesen chart and GS intercept altitude on chart. I did not see that the GS intercept altitude was only 3 miles from runway and about 1100 [feet] AGL. I assumed most GS intercept altitudes were 1500-2000 AGL and 5-6 miles from runway. I turned base and descended to 1600 [feet]. I was hand flying with auto throttles off for proficiency. I noted I did get about 75-100 low on this approach. My intent was to intercept final at 1600 [feet] and finish configuring full flaps when GS was about 1/2 dot. I did this but as it turns out I was 984 [feet] AGL when it happened so I got a printout showing late configuration. My [situational awareness] was not where it should have been. I did not really even notice I was so low until the 1000 [feet] call was made. At that point I should have gone around per SOP since I was not fully configured for landing. Contributing factors [included] unfamiliar airport. Night. Poor runway lighting coming in on left downwind. Runway lights are very dim and hard to see clearly until established on final. Too much task loading on me. I thought it would have been easier flying by hand but I felt I was definitely in the yellow on this approach because I just don't practice this enough. Lack of regular flying. Me, interpreting a cross as FAF/GS intercept altitudes as always around 1500-2000 AGL and 5-6 miles from runway. The 1600 [feet] published altitude was the trap that set me up on this. I had forgotten field elevation was about 500 [feet]. Suggestions [are] for me, in the future, I will practice no autopilot/no autothrottle approaches only in day VMC to airports I am very familiar with [and] with long stable final approaches anticipated. Night visuals especially from downwinds are a challenge. If unfamiliar with airport I will ask for vectors to intercept ILS in the future. I know it's not [going to] happen but make ZZZZZ at 2100 [feet] the GS intercept altitude point. I noticed also a lot of the ILS approaches into ZZZ have this really low GS intercept altitude. This is not good!

Synopsis

A320 Captain reported a CFTT/unstable approach due to human factors.

ACN: 1584582 (1185 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 91
Experience.Flight Crew.Type : 548
ASRS Report Number.Accession Number : 1584582

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 18200
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 3729
ASRS Report Number.Accession Number : 1584593

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As the FO (First Officer), I was the PF (Pilot Flying) on Aircraft X from ZZZ.

Uneventful and standard pre-flight. We got off the gate several minutes early, followed by an early takeoff. After a normal takeoff, we began our climb out and accelerated towards 300 KIAS.

Passing approximately 11,500 feet, I noticed a faint electrical smell and asked the CA (Captain) if he picked up the smell. We immediately looked outside for indications, scanned instruments, and checked systems pages. All appeared normal. 

Very shortly after scanning and passing approximately 13,000 feet, we experienced a loud "bang," followed by pronounced roll into the number one engine. A quick instrument scan revealed excessive EGT, followed by rapid decrease in engine performance. We vocalized what appeared to be engine failure number one. CA called ECAM, which also displayed/confirmed a failed number one engine. 

I immediately started a level off. After several verbal confirmations of a failed number one engine (we wanted to be certain we were focused on the failed engine), the CA ran the ECAM. Having determined we had a damaged engine, we confirmed no desire to relight and CA discharged agent one.

I turned back towards ZZZ and CA [notified ATC]. I then took over coms, coordinating our descent and vectors to ZZZ, while CA communicated with dispatch, FAs (Flight Attendants) and let passengers know our situation/intentions. 

Because we had just landed at ZZZ one-hour prior, we were familiar with the weather, approach and 12,000 foot runway. With some clouds in the surrounding area, we briefed our plan to use vectors RNAV XX (same approach we flew one-hour prior) as our tool to get a solid visual lock on the field, then remain VMC. Once visual, we wanted to focus on flying a stable single-engine visual approach to landing (without the added effort of flying an IFR approach in VMC conditions). 

With briefing complete, CA became PF, while I became PM (Pilot Monitoring). Continuing our descent and well prior to FAF, I confirmed emergency equipment on rollout, called field in sight, and we proceeded inbound visually.

CA flew a beautiful single engine landing. We landed at XX:50Z; we flew for 22 minutes.

I made callouts, announced "remain seated, remain seated" and coordinated for a stop and inspection. Once determined safe to taxi, we taxied to gate.

Narrative: 2

This was our 3rd flight of the day in the same aircraft. Approximately 10 minutes after takeoff, climbing through approximately 13,000 feet we experienced a loud bang, jolt, followed by aircraft roll to the left. Number 1 (left) engine experienced a compressor stall with aircraft EGT extremely high (735 deg) and then engine failure. FO (First Officer) was PF (Pilot Flying) and I accomplished ECAM procedures for Engine 1 Failure with damage. Engine 1 was shut down, Fire Button pushed, and #1 Bottle discharged IAW with ECAM procedure. [ATC was notified], vectors to return to ZZZ. I sent Dispatch an ACARS message, coordinated with FA's (Flight Attendants) (did not need cabin prep at the time), made a passenger PA announcement, and reprogrammed the FMGC for a return to ZZZ. 

I then got back in the loop with the FO and took over PF duties and had FO refer to QRH procedures. We accomplished a quick brief for the RNAV RWY XX as that was the only approach available and we had previously flown it. It was VFR conditions below 2000 feet. We were cleared direct ZZZZZ and cleared for the approach. I needed some maneuvering to continue normal descent just east of ZZZZZ, then at 2000 feet airport was in sight and informed controller we were descending to 1500 feet because I wanted to be completely stable by FAF. Controller informed us we were descending below minimum vectoring altitude and we let him know we were VFR with airport in sight. Continued on approach course for RNAV and flew in visual conditions to uneventful single engine landing. Stopped straight ahead on runway and had Fire/Rescue team inspect engine and aircraft. Taxied back to gate after confirmation of no damage to aircraft. Estimate engine failure to landing was approximately 8-10 minutes. When local maintenance inspected the engine the compressor blades were okay but they found metal fragments in the engine.

NOTE: Just prior to this event the FO said he smelled something faintly that may have smelled electrical. We checked all systems screens and everything was normal, we talked about a divert plan just in case and continued to monitor but everything was normal. Shortly after the engine failure occurred. This was reported to maintenance but they did not see a correlation.

Synopsis

A320 flight crew reported an engine failure during climb resulted in a return to the departure airport.

ACN: 1583553 (1186 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 600

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 41
ASRS Report Number.Accession Number : 1583553

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 514
ASRS Report Number.Accession Number : 1583541

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were departing with a flaps 3 takeoff and TOGA power. This was NOT a reduced power takeoff as is usually the case per the performance numbers we received on ACARS. It was a normal takeoff with no discrepancies noticed. We were assigned a left turn heading 320 with our takeoff clearance and began the turn as we were passing 400 FT AGL. When established in the turn and climbing through approximately 600 FT, we heard a very loud bang from the left engine and felt the airplane shudder in response. I immediately looked at the engine instruments on the E/WD (upper screen) and noticed the EGT dial in red with a tick at the very upper end of the instrument and also noticed the rest of the engine instruments were in amber, many of them with XX's for the readouts. I had immediately thought we hit a goose as there were many reports of geese sightings during our approach just a couple of hours prior. As we were passing through a heading appropriate for a crosswind leg of a VFR pattern, my intention was to fly a visual pattern back to the field as quickly as possible. My main concern was the possibility of hitting another goose and losing our remaining engine. I was hand-flying at the time and thought we were at a safe altitude, and instructed the Pilot Monitoring (PM) to run the ENG FAIL ECAM and secure the engine as we were headed right around for a return to [departure airport]. It became clear to both of us that ATC had cleared the area for us. They provided us an exceptional level of support and vectored us to a final. Though we had no time to contact Dispatch, we did brief the flight attendants on the event (which they had already figured out) and told them that it should be an uneventful landing. At the time of our departure, [the airport] was changing over from a sole east operation to [a different plan] with winds increasing in speed out of the south. As I was trying to change the destination in the MCDU in order to set up the approach in the box, ATC provided us the frequency and vectored us over to [the runway], providing us the new localizer frequency. I was unable to change the destination and abandoned that attempt in order to fly the LOC and G/S in for a safe landing. With a quartering tailwind, the approach was becoming increasingly unstable and as we were approaching the final gate at 500 FT AGL, I determined it was safest to go-around at about 700 FT. The procedures for the single-engine go-around were executed and ATC vectored us back to the downwind asking us the requested runway and the intercept mileage inbound. We requested a runway change to the longest runway given that we were overweight. We requested a five-mile final not wanting to go out too far. The second approach was flown meeting all the stabilized approach criteria at every gate to a smooth landing, as I wanted to minimize the vertical rate of descent at touchdown. We rolled out long, cleared the runway and stopped the aircraft on the parallel taxiway in order for the ARFF to inspect the engine from afar. They saw no visible damage. Before taxiing the aircraft further, I made a PA to the passengers advising them of the issue and that we were safely on the ground and apologized for not updating them sooner as we were very busy managing the event. We received our new gate assignment and parked the aircraft and did our parking checklist. Codes were sent for the overweight landing and the engine failure. We were met at the gate by Flight Operations Management who ensured we were OK and we proceeded down to the tarmac to inspect the engine together after the passengers deplaned.

Narrative: 2

Max takeoff [and] everything was normal climbing past 500 feet. Below 1000 feet AGL in a left turn to 320 we heard an incredibly loud bang and the aircraft started to vibrate. Captain stated we lost an engine, and we received the Engine Fail ECAM and the master caution. Tower asked us if we would like extended vectors, or immediate return. I asked the Captain and we both decided it best to get on the ground ASAP. At the time we believed we had ingested a bird and were worried for more wildlife damaging the other engine and were not sure how badly damaged the engine was. I began running the ECAM while the Captain continued the climbout and level off I completed most of the ECAM as we were getting ready to turn base, and tried programming the FMC, but began to run out of time. I advised the flight attendants of the situation, that we would be on the ground shortly and that no evacuation was anticipated. Approach vectored us in very close to the runway, and we also had a strong quartering tailwind which led to us being high on final, and the Captain made the decision to go-around. We executed the single engine go around maneuver and advised Tower. They asked us what we wanted to do and we decided for a longer final for the longer runway. At this time the ECAM was complete so I was able to reprogram the FMC completely and get the proper approach loaded up. We got vectored around for the single engine ILS. I again notified the flight attendants that we would be on the ground shortly and to not expect an evacuation. We were able to accomplish a successful approach and landing. We taxied off the runway and had personnel inspect the engine who verified they did not see any damage. We taxied into the gate uneventfully.

Synopsis

A320 flight crew reported returning to departure airport after experiencing an engine failure shortly after takeoff.

ACN: 1583128 (1187 of 2908)

Time / Day

Date : 201810
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb

Component : 1

Aircraft Component : Turbine Reverser
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Pitot/Static Ice System
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 13900
ASRS Report Number.Accession Number : 1583128

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1583133

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After selecting the landing gear up, the landing gear doors opened but the landing gear did not retract. We received multiple ECAM cautions simultaneously at this point:

ENG 1 REVERSER FAULT
L/G DOORS NOT CLOSED
L/G GEAR NOT UPLOCKED
BRAKES A/SKID N/WS FAULT
ANTI ICE F/O PITOT

We informed ATC that we wished to delay our climb and departure while we worked on the non-normal indications. We requested and were cleared to maintain 4000 feet and were cleared to hold.

We proceeded to methodically work through each ECAM item and were able to clear all cautions except for the reverser fault which remained on for the remainder of the flight. The brakes/anti-skid/steering fault was cleared by cycling the A/SKID & N/W STRG switch.

We had a pilot on the jump seat for this flight. I asked him to check for popped circuit breakers and he did not find any.

With all indications normal (with the exception of the reverser fault), we continued on to [destination].

Enroute, I contacted Dispatch and Maintenance using the iPad Crew Phone app. I asked Maintenance if there was some overall issue which could have caused the multiple ECAM cautions. Maintenance suggested a possible LGCIU (Landing Gear Control Unit Interface) issue. We discussed the possibility of returning to [departure airport] or diverting for potential maintenance follow-up, but decided to continue for passenger convenience and maintenance.

Upon gear extension for landing, we received another ECAM caution: BRAKES ALTN BRK FAULT. This ECAM caution has no ECAM action associated with it. We made a normal landing.

After landing, we received two more ECAM cautions: L/G SYS DISAGREE and BRAKES-N/WS MINOR FAULT.

We taxied to the gate normally and shut down. When the mechanic arrived at the aircraft we described what had happened. He printed a maintenance post flight report which included two maintenance status messages which we had not displayed during the flight: ENG 1 EIU and AC GEN

Additionally, multiple failure messages were included on the post flight report:
NO ADF 1 DATA
CHECK LGCIU DISC INPUTS
R L/G EXT PROX SNSR 20GA TGT POS
TACHOMETER(21GG)/BSCU(10GG)
TACHOMETER(21GG)/BSCU(10GG)
TACHOMETER(22GG)/BSCU(10GG)
TACHOMETER(20GG)/BSCU(10GG)
EIU, LGCIU(WOW) ENG1B
TACHOMETER(19GG)/BSCU(10GG)
TACHOMETER(22GG)/BSCU(10GG)
TACHOMETER(20GG)/BSCU(10GG)
TACHOMETER(19GG)/BSCU(10GG)
LGCIU(5GA1)/GCU2(1XU2)
LGCIU(5GA1) RIGHT SHOCK ABSORB SIGNAL/BSCU(10GG)
ABCU(95GG)/BRK ALTN PRESS TRANSDUCER(100GG)
ABCU(95GG) DC SPLY 401PP
CHECK LGCIU2 DISC INPUTS

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported several landing gear ECAM Cautions, both in flight and after landing.

ACN: 1582635 (1188 of 2908)

Time / Day

Date : 201810

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Nose Gear Wheel
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1582635
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Approximately 20 minutes after Aircraft X departed, the Ramp Supervisor brought a part to Maintenance Lead Tech and stated they found on ground at gate in the area around nose of aircraft. After seeing the part I called Maintenance Control. I informed him that I had changed the left nose tire and wheel assembly earlier that night and that this part could possibly be from that work. [I] asked him to stop the aircraft when it lands in ZZZ1. [I] looked up part number and gave to him in case needed. All work was performed per maintenance manual. Waiting to hear outcome of inspection in ZZZ1. If part that was found here in ZZZ is indeed missing. Bushing/support is a very loose fit inside the nose wheel axle may need to pin or some sort of way to prevent it from falling out or getting stuck inside of axle nut socket with grease.

Synopsis

Maintenance Technician reported that an A320 part was found in the area where maintenance had replaced the nose wheel.

ACN: 1582185 (1189 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : ZZZZZ
Airspace.Class E : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 26000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 15000
ASRS Report Number.Accession Number : 1582185
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1582207
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Cleared and descending via [the arrival] landing north. Approaching turn [the] aircraft was tracking lateral course normally. Pre extended final approach and noticed route dropping out of FMS. Selected a parallel heading for the downwind while rebuilding arrival. After, engaged auto flight to intercept track for the downwind. Instead of a normal intercept, auto flight commanded at least a 60 degree intercept. While flying through published track and starting a correction, ATC queried. While attempting to become established on downwind track, ATC called out an altitude alert. We advised we were having issues with the FMS and were in VMC conditions with terrain in sight below us. ATC directed a heading eventually to base. All recovery actions were accomplished in VMC. Airport visually acquired and final approach and landing accomplished.

Narrative: 2

Upon descent into ZZZ on the [arrival], we were descending VIA and had passed 12000 feet and were rounding the corner to the downwind and were issued a new altitude to descend to. I can't recall which ALT, possibly 6000 feet or 9000 feet but whatever the altitude, it was the correct one. Around the turn in the STAR is when the captain inadvertently deleted the waypoints on the downwind segment of the STAR. He was trying to add points to the approach segment I believe and then I heard, "oops" or something to that effect. We continued the turn in HDG mode towards ZZZZZ Intersection while I manually rebuilt the arrival. I entered in direct ZZZZZ1 Intersection and then rebuilt ZZZZZ Intersection behind it as the A320 navigates FROM not TO and we needed to be on the correct 164-degree course. We armed the NAV and the aircraft took an almost 90-degree heading to intercept the course. So the aircraft was east of course headed south and then took up the unrealistic intercept pointing the aircraft West. We executed a manual turn south towards ZZZZZ1 Intersection and around that time ATC called to query. We advised her of an "issue with the FMS". We had previously been cleared to descend to lower altitude and ATC told us she was getting a low altitude warning and advised us we could climb a thousand feet if we needed to. We declined. This was puzzling since we were VMC, the EGPWS terrain on the NAV DISPLAY was all green and it was clear visually to both myself and the captain that we were in no danger of terrain. She gave us a base turn heading and we executed the visual approach to Runway 34L without incident.

To be absolutely clear: while the aircraft was in a [unintended state] due to the lateral navigation error and the ATC-perceived altitude deviation it was absolutely never in any danger of hitting terrain either from visual cues or aircraft warning systems.

Synopsis

A320 flight crew reported FMC issues resulted in an unstabilized approach and low altitude alert in visual conditions.

ACN: 1581863 (1190 of 2908)

Time / Day

Date : 201809

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Oxygen System/Crew
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Type : 2118
ASRS Report Number.Accession Number : 1581863
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Event began with phone conversation to [Maintenance] from hotel prior to walking to the airport. Subject inbound writeup at XA08 to ZZZ Low Oxygen Quantity. I was told the jet was just blocking in (XA43) our departure was at XB50. I arrived at the jet at approximately XB25, after going by Flight Operations for planning.

I discovered the O2 hadn't been serviced and displayed 970 psi. I immediately called Maintenance and requested servicing as the visibility was now 1/2 mist, and our takeoff alternate was ZZZ2. Having a basic understanding of the Crew Oxygen System dispatch limits 850/550 psi and other [company] publications that quantifies the dispatch limit that states a crew has approximately 15 minutes of O2 in case of cockpit smoke, I concluded - based on takeoff weather/takeoff alternate, and enroute planning where we would be on the hose from airborne to ground rollout, the safest and most prudent and course of action would be to have the O2 serviced. 

I continued with preflight duties while monitoring the Maintenance frequency. A few minutes later, Maintenance told me they weren't going to service the O2, because they would have to deplane our passengers.

I told them that I didn't think this was the correct procedure, and I immediately called Maintenance. He said a [report] would be initiated. At that point I sent a [Maintenance Request] at XB41 (9 mins before scheduled departure and due to Maintenance's request as mentioned earlier, I had notified ZZZ Maintenance approximately 16 minutes prior to this. In the end, ZZZ Maintenance made the decision to deplane everyone. In the end, we deplaned, serviced system, reboarded and departed 55 minutes late.

47 minutes into the flight we had already lost 110 psi, noted on an info only write up.

As a side note, existing ZZZ2 weather and takeoff alternate aside, actual enroute data placed us 4nm on top of ZZZ3 at FL320. Using ZZZ3 as the closest airport would require 18 minutes leaving us with a possible 3 minute deficit and all of this in a perfect world.

Synopsis

A320 Captain reported issues between the aircraft advisory display, maintenance limits and dispatch limits for servicing the crew oxygen system.

ACN: 1581849 (1191 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Altitude.MSL.Single Value : 33000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 613
ASRS Report Number.Accession Number : 1581849

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Type : 1770
ASRS Report Number.Accession Number : 1581856

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While cruising at FL330 in VMC conditions we began to get whining noise from under flight deck floor. This noise progressively got louder. After about 10 minutes we began to feel vibrations on floor in flight deck. Cabin crew could then hear noise in forward cabin as well. I put in a request for dispatch to call me on VHF and include maintenance (MX) on the line. While dispatch was getting maintenance on line we began to smell odor of hot wires or electrical components. This became stronger pretty quickly. I began to survey alternate options. Determined best options based on ND display and MCDU were ZZZ about 70 miles east and ZZZ1 about 160 west. I decided on ZZZ and advised dispatch of deteriorating conditions when he returned to frequency with MX. He agreed that ZZZ was good diversion and stated he would advise all necessary parties and send landing data. I transferred controls to FO (First Officer), [notified ATC], and set up FMGC for approach while FO worked on descent. Briefed cabin crew of diversion with TEST items stating cabin prep was not required. 

About halfway thru descent to ZZZ the avionics cooling blower fan ceased and the CB tripped. With this, the fumes and noise stopped. 

Normal landing on runway XX and taxi to gate. Fire rescue crews remained on scene until MX Could open compartment to examine cooling fan. Passengers deplaned using jetway.

Narrative: 2

An hour and twenty minutes into our flight the Captain and I both noticed a loud grinding noise from under the floor of the cockpit, associated with a noticeable vibration. There was also a noticeable electrical smell. The aircraft had a previous write up for a tripped avionics blower circuit breaker, so we determined that this was probably the issue and decided to divert to ZZZ since it was 70 miles to our east. The Captain transferred aircraft control to me, along with the radios, while he coordinated with dispatch on VHF2. I [notified ATC] and we were given direct to ZZZ with a descent. While I coordinated with ATC the Captain briefed the flight attendants and then passengers on the diversion. It was determined that an emergency egress would not be required since the avionics blower circuit breaker tripped during the descent and the electrical smell dissipated, however we still had emergency equipment standing by to meet us upon landing. I flew the uneventful approach and landing to runway XX at ZZZ. Upon landing we taxied to the gate while coordinating with the emergency personnel to ensure that they didn't see any problems with our aircraft. Upon shutting down at the gate we met with emergency personnel and inspected the aircraft. The passengers were deplaned at the gate without incident.

A replacement avionics blower was flown in from [our hub] and we continued the flight to [our destination] without further incidents.

Synopsis

A320 flight crew reported diverting due to smoke and fumes from an avionics cooling blower fan malfunction.

ACN: 1580864 (1192 of 2908)

Time / Day

Date : 201809

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 10054
ASRS Report Number.Accession Number : 1580864
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Logbook Entry

Narrative: 1

While flight planning crew noticed a sign off about the water service door being cycled by the crew. At 5'10" this reporter could not reach said door and with just under 4% of the population is reported over 6'3", (guesstimate of size required to reach) one must assume the pilots could not reach the door without some assistance. This is forbidden under FOM (Flight Operations Manual) guidance Pilot Accomplished Tasks. Just want to ensure [Operations Management] is not requesting actions of pilots that are forbidden.

Synopsis

A320 Captain reported the aircraft logbook appeared to indicate a previous flight crew had operated the water service door which is prohibited by the Flight Operations Manual.

ACN: 1580666 (1193 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air/Ground Communication
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1580666
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Coming into ZZZ we received constant radio #1 interference from ACARS burst. On reaching cruise we decided to turn off the ACARS per a new procedure. We put SELCAL into radio #2 and took ACARS temporarily off line. About 75 miles west of ZZZZZ we received SELCAL from Dispatch telling us Center was looking for us. We tried to contact them to no avail. We then re-contacted Dispatch through SELCAL and they told us to contact Center, which we did. We were NORDO for we were told about 50 minutes. We were flying our exact flight plan the entire time. Center requested I call them and I did on landing. We did not have a deviation, but they were worried about the prolonged period of time of no radio contact. I explained to him the problem we are dealing with on new aircraft and that we miss radio communications at alarming rate due to ACARS interference on radio #1. I feel this definitely contributed to problem, along with new procedure which does not have us monitor 121.5 constantly. This would have given us a much quicker alert that we had no active Center frequency. It takes some time for ATC to contact Dispatch to contact SELCAL and patch us through to Dispatch. 

We need to immediately change our policy with NEO aircraft until a permanent fix (i.e. relocation to radio #1 antennae) to require crews to not use radio #1 other than to monitor 121.5. The number 2 radio should be used for all ATC communications as it does not get interference from ACARS burst transmissions. On our first flight we caught descend to 7000 ft, but had to get them to clarify to slow to 210 as the ACARS burst covered up entire portion of clearance. This is becoming a serious safety issue, as crews are missing clearances. Sometimes ACARS burst are minutes apart. Sometimes they come almost constantly. We need to fix this problem.

Callback: 1

Reporter stated their Company and Airbus are aware of this problem. The Reporter described "work around fixes" the Company has implemented to mitigate the effects of the interference.

Synopsis

A320 NEO Captain reported ACARS burst transmissions interfered with the radios, causing the crew to miss a frequency change.

ACN: 1580313 (1194 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SJC.Airport
State Reference : CA
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class C : SJC

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : NCT
Make Model Name : Bonanza 33
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Airspace.Class C : SJC

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1286
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1286
ASRS Report Number.Accession Number : 1580313
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Conflict : NMAC
Detector.Person : Flight Crew
Miss Distance.Horizontal : 500
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On the Razzr4 arrival and cleared for the ILS 30L, as we were nearly turning onto final a Bonanza aircraft passed about 200-300 feet below and about 500 feet laterally under our left wing. There was no TCAS warning and I asked NorCal approach about the Bonanza, he replied that there was no transponder information but that there was only primary painted information.

Synopsis

A320 Captain reported a near midair collision encounter on final approach.

ACN: 1580070 (1195 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : TX

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb

Component

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1580070

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1580071

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The aircraft had a history of oil consumption. The inbound flight was serviced and the Technician said it took the maximum oil allowed. He said it was at the limit on usage per flight but still within limits. Thirteen minutes into the flight the oil level dropped 8 qts from 18.5 at takeoff to 12.5 and a few minutes later down to 12 qts. At the time I was PM (Pilot Monitoring) the Captain then transferred me to the PF (Pilot Flying) and ATC duties. The Captain contacted Dispatch and [Maintenance]. He came back from the conversation and said they agreed we should divert. After determining we would be overweight for landing, the Captain [advised ATC]. We landed at a descent rate of 200 FPM or less. The Captain and I accomplished all company procedures and checklists. I didn't think it was necessary to file but due to [advising ATC] the Captain convinced me we should.

Narrative: 2

In talking to the Captain from the inbound flight we found that the aircraft had been using a significant amount of oil in the left engine. The Mechanic reported that he had put 7.5 pints of oil in the left engine it was indicating 18.5 qts. He told us that it was right at the limit on usage and we were legal to take the aircraft.

At takeoff we were still at 18.5 qts. 13 minutes into the flight we were indicating 12.5 qts and at 25 minutes we were indicating 12.0 qts. I called Dispatch and we talked to [Maintenance] and I decided that we should divert.

The landing was overweight at 149,000 lbs., and the sink rate at touchdown was 200 fpm. I [advised ATC] due to the overweight landing. Upon gate arrival, I contacted Dispatch and made an entry in the [logbook] for the mechanical discrepancy, and one for the overweight landing.

Synopsis

A320 flight crew reported high fuel consumption after takeoff resulting in a diversion.

ACN: 1580017 (1196 of 2908)

Time / Day

Date : 201809

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Other / Unknown
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1580017
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
Detector.Person : Ground Personnel
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I took over this flight after sign-on. As the flight was closing out, I took a closer look. I noted that there was HAZMAT planned in [Bin] 2, along with 15 bags. Looking at what was actually scanned, I noted there were only 10 bags scanned in [Bin] 2. The remainder of the bags, and an [express cargo] item were scanned in [Bin] 3. The [express cargo] item isn't actually HAZMAT, but a restricted item. This item carries the SLS code of "RDS." This item carries the same loading requirement of HAZMAT. It must be secured by 15 bags, and can't be loaded in the nesting system of A320s and 757s.

As I was looking at the ramp scanning, I got a call from the Ramp Lead. He advised me that the [express cargo] item was scanned and in [Bin] 3. He seemed confused that this was actually a restricted item (HAZMAT loading rules even though not HAZMAT). I advised him that this item couldn't go in [Bin] 3 as that's a nesting system. I advised him that the flight would have to come back to the gate and either remove the [express cargo] or move it to [Bin] 2 along with 15 bags. He said the flight was already out. I told him that I'd contact the crew by ACARS and he said he'd call [Station] Operations. I advised my Shift Manager what was going on, sent the crew an ACARS message, and advised [Station] Operations. As I was doing this, my Lead was advising the Flight Dispatcher what was going on. The flight returned to the gate area. The [express cargo] and some bags were moved from [Bin] 3 to have 15 bags and the [express cargo] in [Bin] 2. The Ramp Lead finalized the flight thusly, and I sent the final weights to the pilots.

After takeoff, the Flight Dispatcher called. He said the Captain noted that the "HAZMAT Item" wasn't removed from the aircraft, and that bags were moved around when they returned to the gate. I explained the situation with the RDS item to the Dispatcher, and advised moving the item and the bags solved the problem of having a "restricted" item in the nesting system. The Dispatcher said he would pass the information on to the pilots.

Synopsis

Aircraft Load Agent reported aircraft returned to gate to correct Restricted Cargo load error (HAZMAT loading requirements applied).

ACN: 1579484 (1197 of 2908)

Time / Day

Date : 201809

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Oxygen System/Crew
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
ASRS Report Number.Accession Number : 1579484
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

If the jet experienced cockpit smoke and we had to wear the mask we would potentially consume all of the oxygen, best case, 5 minutes before landing. The [manual] states we have 15 minutes of O2 onboard if we dispatch at [minimum] dispatch limits. The [manual] doesn't distinguish the number of crew members, so we could assume 2 @550 psi and 4 @850 psi. Had we dispatched at the [minimum] limit, in a smoke scenario on that route, the outcome would have been unfavorable. So, is [it] our position, given a specific route, [that it is acceptable to] dispatch at the dispatch limits knowing that the quantity will be insufficient to safely land the jet within 15 minutes?

Synopsis

A320 Captain reported that aircraft are being dispatched with less than an adequate amount of crew oxygen.

ACN: 1578777 (1198 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 28000
ASRS Report Number.Accession Number : 1578777
Human Factors : Time Pressure
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 1579376
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Engine 1 compressor stalled 3 or 4 times about 1000 feet. Shut engine down and returned for overweight landing.

Narrative: 2

After takeoff we heard 3 or 4 large bangs. We realized we were losing number one. I immediately [advised ATC]. We leveled off at 3000 feet. We requested vectors back to land. We ran some procedures and secured number one. We did a downwind, base and final and landed XXL at ZZZ.

Synopsis

A320 flight crew reported #1 engine failure on takeoff.

ACN: 1577466 (1199 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8700
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4616
ASRS Report Number.Accession Number : 1577466
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13164
Experience.Flight Crew.Last 90 Days : 169
Experience.Flight Crew.Type : 1463
ASRS Report Number.Accession Number : 1577482
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected.Other 
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were on first flight of four day trip. Flight from ZZZ [in] night IFR conditions. We were climbing on RNAV Departure. Captain was flying; I was Pilot Monitoring. Autopilot was on approaching 12,000 feet climbing to assigned altitude of 16,000 [feet]. I heard a thump then felt the aircraft yawing. Engine display showed engine 1 (left) had failed. Captain turned off autopilot. We [requested priority] with ZZZ Departure. Captain said he would fly and I was to work the ECAM for the engine failure. Once aircraft was stable, Captain turned the autopilot back on. I followed the steps of the ECAM as the Captain coordinated with ATC and then talked to the flight attendants; he then made a PA [public announcement] to the passengers. Once the ECAM complete, I referred to the QRH ENG1 Fail Checklist to ensure all steps were accomplished. The APU was started. We discussed attempting a relight as there was no indications of engine damage. We did [it] unsuccessfully. During relight attempt, we saw No Fuel Flow and ENG Stator Fail ECAMs. I then started working the ENG Shutdown Checklist per the QRH. I discontinued that checklist to load the return to ZZZ and approach to Runway XXR; the captain briefed the approach. We discussed the overweight landing procedures as our gross weight was approximately 150,000 [lbs].

The Descent and Landing Checklist were accomplished as the aircraft was configured normally for a visual approach to Runway XXR. Approach and landing were normal; we rolled out and taxied clear of the runway. We made a PA announcement for everyone to remain seated. We communicated with the fire rescue vehicles and they could see no damage so we continued to taxi to the gate. The passengers deplaned normally through the jet bridge.

Narrative: 2

[Report Narrative contains no additional information.]

Synopsis

A320 flight crew reported an engine failure during climb that resulted in a return to the field.

ACN: 1577235 (1200 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : OAK.Airport
State Reference : CA
Altitude.MSL.Single Value : 7000

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : OAKES2
Airspace.Class B : OAK

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Commercial
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1577235

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

The OAKES2 arrival MONOH transition for 30 presents a trap for flight crew members. If we reference 10-7A, it is noted that aircraft operating at OAK may be below the Class B shelf. The 10-1B provides a depiction of the airspace, but is useless with no identifying fixes, DME, radial, etc. Upon review of the high altitude chart and cross referencing fixes, I noted that the OAKES2 STAR will take you below Class B airspace if flown at the lower altitude limits. FFIST HAS A STAR window of 7,000 - 9,000 [feet]. While the Class B shelf starts at 8,000 [feet] and PRFCT above 5,000 [feet], Class B shelf begins at 8000 [feet]. There is no reference on the STAR of exiting the airspace and no mandatory speed markings below the 280 KTS mandated on the chart. (Standard transition at 250 [knots] at 10,000 [feet].) In order to properly identify the Class B airspace, we had to overlay our arrival on the high altitude charts. It is not standard practice to reference hi altitude charts on the arrival. This creates an obvious trap for our crew members.

Synopsis

A320 First Officer reported that the OAKES2 STAR will take you below class B airspace if flown at the lower altitude limits.