ACN: 1577181 (1201 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nose Gear
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 14337
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 10588
ASRS Report Number.Accession Number : 1577181

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During pushback from gate in ZZZ, the tug suddenly stopped, causing [a] small bump to the airplane. Fortunately, no one was injured; however, the reason for the stop was due to a ZZZ Airport Operations vehicle crossing dangerously close behind us during the pushback. Because of the quick stop, the nose gear pin sheared and Maintenance was called to do an inspection of the nose gear. No damage was found by Maintenance.

Synopsis

A320 Captain reported the tow bar pin sheared during pushback due to the tug suddenly stopping.

ACN: 1576713 (1202 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Landing

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1576713

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 4500
ASRS Report Number.Accession Number : 1576714

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Weather
Primary Problem : Environment - Non Weather Related

Narrative: 1

[We were] cleared for the RNAV [arrival] with 35 knot tailwind. All speed and altitude restrictions were made. Initial Arrival ATIS had [destination airport] was landing ILS XYR with Weather 9 Vis 1,000 overcast / Winds approximately 100 at 11. Briefed and Confirmed the Arrival and the ILS Runway XYR with all parameters. Updated ATIS, I noticed the Vis dropped to 8 miles. 

Approach advised us of a tall Ship and told us to expect the RNAV (GPS) Runway XYR. The First Officer reloaded the approach and we confirmed / validated the approach. I however missed the notation on the size / height of the Vessel restriction 144 feet.

We were cleared to descend to 4,000 feet outside ZZZZZ Intersection and cleared the approach. I armed the approach and received correct Final Approach armed indications (actually checked the perf page for confirmation as I normally do thru 10,000 feet) prior to ZZZZZ Intersection.

Began landing configuration after passing ZZZZZ Intersection inbound. However the aircraft did not capture and begin descent at ZZZZZ Intersection so I used Speed / Vertical speed to make the remaining restrictions. 

I checked ground position on approach visually prior to passing the [a building] at approximately 1,000 feet With visibility less than I expected I was able to pick up Runway XYR about the same instant I observed the Large Vessel exiting the Port. 

When we reported runway XYR in sight. Tower cleared us for a Visual Approach. Some of the Cruise Ship was still in our direct path to the runway. I leveled off approximately 700 feet, called for autopilot off / flight directors off and maneuvered left of centerline then back to runway heading and a safe landing profile. First Officer noted airspeed deviations and sink rate on short final that were immediately corrected. 

We landed on speed and approximately 1200 feet down the runway on centerline. Rollout was normal. 
This event occurred due to IMC / Cruise Ship exiting the Harbor.

Personally I believe air traffic in IMC conditions should be held for 3-5 minutes to allow an occasional Cruise Ship to Exit.

Narrative: 2

Inside final approach the aircraft started to accelerate instead of slow down as expected. I called "Airspeed Increasing - watch your airspeed" - three times. Captain disengaged Autopilot and Auto thrust and corrected the airspeed which exceeded Flaps Full Airspeed by approximately 2 knots. We returned to a stabilized approach and then broke out of the weather and was cleared for a visual approach with a Tall Ship in the way. The Captain veered slightly to the left and remained slightly high of Glidepath to ensure separation. Once we were clear The Captain corrected back to glideslope and momentarily exceeded 1000ft descent. I called out 1300 descent rate. The Captain returned to an 800 descent rate and landed on speed about 1200 feet down the runway. 

High workload caused by change in Approach Type and managing the airplane during the approach. Airplane started to accelerate unexpectedly and took the Captain time to override automation which time delay caused Flap overspeed. Unstable approach was caused by unexpected Tall Ship directly on flight path once we broke out of the weather, forcing descent profile correction. The correction back to glideslope caused the momentary steep descent on final.

The Tall ship was in the harbor for a long time, but really only effected us. Tower should be able to delay approaches during the short window the ship is directly crossing the flight path. If it had been VMC then no issues.

Synopsis

B767 flight crew reported heading and glide path deviation due to a cruise ship in the flight path for landing.

ACN: 1576699 (1203 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1576699
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC Overrode Automation
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During preflight and taxi we had been assigned three different runways for takeoff. Initially planned for takeoff on Runway 18C, ramp controller then assigned 36C on initial contact and after taxi instructions were received we were changed to depart 36R. Our original paperwork both electronic and hard copy had only the south departure runways for ZZZ. We had to pull new takeoff data for the last two runway assignments and the last runway change occurred after our "before takeoff" checklist was completed. We reprogrammed the performance numbers and SID correctly on the first runway for 36C change. When we received the 36R assignment new takeoff performance data was requested via ACARS and received. The new takeoff performance speeds were entered but the runway change to 36R was not. This resulted in our Flight Director commanding a left turn after takeoff instead of a right turn. The pilots discovered our error slightly before the Tower told us to turn right to 025 degrees and we were in the process of communicating the error in the cockpit when Tower made their call.

Ramp Controllers should not be assigning runways on taxi out because they are not trained to the same standards as ATC specialists and make too many mistakes as happened in this case. We missed the opportunity to catch the error on taxi and didn't notice the error until we were into the turn airborne. Three runways assignments from pushback to takeoff helped cause our flight deck to become task saturated and we made an error that wasn't noticed until airborne and in a turn. We also had a no delay and very quick taxi to the takeoff runway and the error was not captured.

I need to slow down and accomplish the before takeoff checklist after each runway change, including route verification which would have trapped this error. SID design in ZZZ is different than SIDs in many other busy airports due to flying headings written in print on a departure procedure. Whereas, other high traffic facilities SID procedure will assign the desired/required runway heading upon issuance of takeoff clearance or it will provide a specified fly to point. This specific procedure coupled with multiple runway assignments and subsequent requests for takeoff performance did not aid in mitigating the error. RNAV departures from the runway directly to a fix are much easier to visually verify in the cockpit. I am sure there is a good reason outside of my scope to keep the current procedures in ZZZ in place instead of designing specific point RNAV departures. If there is a more important reason for leaving the departures intact at ZZZ as they are today, I would then suggest the Tower controllers add "fly heading xxx" upon takeoff clearance.

Synopsis

B767 First Officer reported a heading deviation after takeoff due to three runway changes during taxi and the failure to input the latest change into the FMC.

ACN: 1576574 (1204 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : MIA.TRACON
State Reference : FL
Altitude.MSL.Single Value : 3700

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : MIA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class C : FLL

Aircraft : 2

Reference : Y
Aircraft Operator : Personal
Make Model Name : Amateur/Home Built/Experimental
Operating Under FAR Part : Part 91
Flight Plan : VFR
Mission : Personal
Flight Phase : Descent
Airspace.Class E : HWO

Person

Reference : 1
Location Of Person.Facility : MIA.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2.9
ASRS Report Number.Accession Number : 1576574
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

FLL Final was working Aircraft X inbound to FLL RWY 10L during a heavy arrival bank using parallel runways. I was sitting on [Sector] 1Z and observed a VFR, fast moving, 066 [MSL] descending from northwest of FXE, southeast bound rapidly into HWO. I tagged the VFR watch and back-splatted it to both the FLL Feeder and Final. The Final Controller had to evasively maneuver Aircraft X into a tighter turn to final with traffic on the parallel runway he was staggering on, just to miss this VFR. I observed the VFR proceed into HWO, and the Tower told me it was Aircraft Y who was an experimental.

FLL airport is the 19th busiest airport in the USA, and one of the top 3 fastest growing, averaging 8% growth each of the last 5 years. FLL has a basic Class C airspace surrounding it that is beyond out of date and unable to aid in the safety of its aircraft on the finals. Planes inbound to each of their parallel runways are not offered any sort of protection until within 5 miles of the field, which allows several dozen VFRs each day to climb, descend, and transition across each final, without ATC advisories creating a very dangerous, unsafe, and hazardous situation in the skies above. The FLL Class C is inadequate, and out of date, and needs a major airspace change around it, whether a bigger Charlie, or a full blown Class B before it's too late.

Synopsis

Miami TRACON Controller reported an airborne conflict with a general aviation aircraft and an air carrier inbound to Fort Lauderdale.

ACN: 1576568 (1205 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : MIA.TRACON
State Reference : FL
Altitude.MSL.Single Value : 6200

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : MIA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : MIA

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : MIA
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Plan : VFR
Flight Phase : Climb
Airspace.Class C : FLL

Person

Reference : 1
Location Of Person.Facility : MIA.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2.9
ASRS Report Number.Accession Number : 1576568
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X was inbound from our western arrival gate. A VFR was transitioning around the Bravo, descending fairly rapidly through Aircraft X's altitude. Approach saw the VFR and stopped Aircraft X above the descending aircraft, who ended up proceeding and descending inbound to BCT. This created a high, unstable, and unsafe approach for Aircraft X. This should not have to be the normal procedure to evasive maneuver aircraft inbound to the 19th busiest airport, on parallel finals, [and] around VFRs who are allowed to fly all through both eastern approaches.

FLL airport is the 19th busiest airport in the USA, and one of the top 3 fastest growing, averaging 8% growth each of the last 5 years. FLL has a basic Class C airspace surrounding it that is beyond out of date and unable to aid in the safety of its aircraft on the finals. Planes inbound to each of their parallel runways are not offered any sort of protection until within 5 miles of the field, which allows several dozen VFRs each day to climb, descend, and transition across each final, without ATC advisories creating a very dangerous, unsafe, and hazardous situation in the skies above. The FLL Class C is inadequate, out of date, and needs a major airspace change around it, whether a bigger Charlie, or a full blown Class B before it's too late.

Synopsis

Miami TRACON Controller reported an airborne conflict with a VFR aircraft and an air carrier inbound to Fort Lauderdale.

ACN: 1576354 (1206 of 2908)

Time / Day

Date : 201809

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 297
ASRS Report Number.Accession Number : 1576354
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During pushback normal pushback procedures were in use. After the pushback was complete I was asked to set the brakes. I could tell that the ground crew was having some difficulty disconnecting the tow bar. They asked me if I could release the brakes to allow them to disconnect the tow bar. I replied that I could NOT release the brakes to assist them in disconnecting the tow bar. The only thing that I would agree to was releasing the brakes if the tug and tow bar was still connected so they could push or pull the aircraft to straighten the nose wheel out. When the nose wheel was straight we would reset the brakes and then use normal procedures to disconnect the tug and tow bar. They told me that they agreed to do that. The ground crew assured me that I was connected to the tug and told me to release the brakes. I released the brakes and the ground crew disconnected the tow bar without permission. When they told me that the tow bar was disconnected I set the brakes. I told the ground crew that this was a direct violation of our SOP and a serious safety issue. I suspect the ground crew lied to me to get me to release the brakes. [The] ground crew didn't seem interested in following our SOP.

Synopsis

A320 Captain reported that the push-back crew did not follow the SOP and disconnected the tow bar while the brakes were released.

ACN: 1574897 (1207 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1574897
Human Factors : Physiological - Other

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1574418

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Maintenance must use the utmost care before returning an aircraft to service following a fume event. Because the ability to smell the odor is usually lost after a few minutes of exposure to it, it may be difficult for them to make a valid assessment about whether the fumes are still present. We should ensure that mechanics are aware of this issue. Wider availability of the particle sniffer machine would help mechanics make objective assessments about whether an aircraft is fit to return to service. 

When I first met the FO (First Officer), he told me that when he was on the aircraft earlier, he did notice some residual fume smell. I did consider this and obtained the entire crew's concurrence with the plan to briefly board the aircraft and make an assessment. However, if I have a similar situation again, I will ask more active questions to determine whether it is safe to get on the aircraft even briefly. I don't think I gave anyone the impression of pressuring them to get on the aircraft to make the assessment, but if I have a future similar event, I will actively clarify that I am not creating any such pressure. 

[The company] must ensure that maintenance personnel are provided with appropriate personal protective/breathing equipment while they work on aircraft which have had a fume event. We hear a lot about medical issues affecting crew members--what about the mechanics? I didn't really think about this until I started preparing this report. If I have a future fume event, I will specifically make sure the mechanics feel comfortable working on the aircraft. 

[The company] must continue to investigate the causes of fume events and take action to prevent them. The root cause of the APU oil leak should be determined. [The company] must follow up with the FO to ensure that he is fit to return to duty and provide him with any assistance that may be necessary.

Narrative: 2

Improved evaluation of residual fumes in cabin before clearing aircraft for flight.

Synopsis

Airbus flight crew reported smelling residual fumes from an APU oil leak repair upon entering the aircraft.

ACN: 1574364 (1208 of 2908)

Time / Day

Date : 201809
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZJX.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 35250

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Cloudy

Aircraft

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZJX

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 5632
ASRS Report Number.Accession Number : 1574364
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 22000
ASRS Report Number.Accession Number : 1574867
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Cruising at FL360 to FLL our aircraft was abeam ORM (Ormond Beach VORTAC) when we encountered moderate turbulence associated with a downdraft. Aircraft was in IMC conditions with no convective activity near aircraft on radar display. I was the Pilot Flying, Captain was the Pilot Monitoring. After moderate turbulence commenced the aircraft began descending at a rate up to -1500 FPM and airspeed began to rapidly increase. Captain notified ATC and reported our inability to maintain altitude. The autopilot failed to keep up with the turbulence and attitude, while descending through 35500 I communicated the indications and disconnected automation to regain control of the aircraft. I smoothly corrected the attitude and airspeed as we exited the turbulence and downdraft. Automation was reengaged as we climbed back to FL360 from roughly 35250. No airframe limits were exceeded. Captain reported return to FL360 with ATC and encountering moderate turbulence with the associated loss of altitude. Syllabus events involving reaction to turbulence and change in attitudes/airspeeds during recurrent training prepared me to recognize and react quickly to this event.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

Flight crew reported encountering severe turbulence and a downdraft causing excursion from assigned altitude.

ACN: 1573006 (1209 of 2908)

Time / Day

Date : 201808

Place

Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Type : 245
ASRS Report Number.Accession Number : 1573006
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Climbing out of EWR on Runway 22R via the EWR2 departure we were cleared to 6,000 feet. Subsequently, we were advised of VFR traffic at 12 o'clock position [at] 6,500 feet. We had reported the aircraft in sight. As we were leveling off in the climb, we received a TCAS RA. I was still flying without the autopilot, however, neither of us remembered to de-select the Flight Directors. I noticed that the VFR traffic was only indicating 400 feet above us so I nudged the aircraft down another 80 feet to approximately 5,920 feet before the traffic passed directly overhead. A few moments later and after receiving the clear of conflict alert, we were cleared to climb to a higher altitude.

Synopsis

A320 First Officer reported a near mid-air collision on climb out from EWR when VFR traffic passed above them with less than 500 feet vertical clearance.

ACN: 1573000 (1210 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7330
Experience.Flight Crew.Last 90 Days : 2
Experience.Flight Crew.Type : 5019
ASRS Report Number.Accession Number : 1573000
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Pre-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During my cockpit checks, I found the Green Hydraulics to be above the maximum quantity limit. I immediately informed my Line Check Airman, (LCA). [The] LCA agreed the Green Hydraulic system was over serviced. I immediately entered a write-up into ACARS, while simultaneously the LCA called Maintenance on the radio as required by the Flight Operations Manual (FOM). 

After an extended period of time, a mechanic came into my cockpit with an iPad. I greeted him and gave him my name, he did not say his name. He looked at the Hydraulic Quantity page and then looked at his iPad and stated the hydraulics have not been serviced since [last month] and the system cannot make hydraulic fluid. I told him that I still cannot take it like it is. The mechanic continued to give resistance to do anything and turned to my LCA to see if he could convince him the system was not making hydraulic fluid. 

I then proceeded to [look] in my A320 Flight Manual. My LCA was digesting all the mechanic was trying to say. Then, the mechanic asked me if I could do an engine run while he looked at the Green Hydraulic system below. I told him that is not a normal procedure to assess hydraulic quantity, besides, I have a plane full of people. What are you going to do if the system needs servicing, then what? I have never done an engine run for this issue in [many] years on the Airbus.

The mechanic then starred at his iPad and said he has seen this before and once the engine starts, the system will be in the normal quantity range. About then, [another] mechanic came on board and took one look at the hydraulic quantity page and said "yep, that's over-serviced." The [first] mechanic began to argue with him. [The second mechanic] then spoke assertively and told the [first] mechanic that [the] iPad is wrong about the last Green Hydraulic service date and it was probably over-serviced on night shift and no one documented the servicing. 

[The second] mechanic told me he would need to get some tools and rags and would be back. I asked him if he wanted me to remove pneumatic pressure so he could service the Green System. He said "no, I will service it the way it is."

After an extended period of time, I saw the right main gear door come down. A while later, he comes into my cockpit and said "yes, I'm going to need you to remove the pneumatic pressure." I complied.

The Green Hydraulic System was serviced and we departed. The long process of draining an over-serviced hydraulic system is problematic and unfortunately all too typical. However, the very few times I have seen it done properly it is done in under nine minutes and usually closer to six minutes. 

I had an identical maintenance write up [before] and the Maintenance response caused almost a five-hour delay. The mechanics did not remove pneumatic pressure and the drain valve stuck open and drained the entire Green System.

Synopsis

A320 Captain reported an overfilled hydraulics system, leading to some confusion with Maintenance before being serviced correctly.

ACN: 1572707 (1211 of 2908)

Time / Day

Date : 201808

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Rudder Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 9994
ASRS Report Number.Accession Number : 1572707
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Taxi

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

[Company reporting] system is not displaying our flight for selection. It never does with a return to gate. Tech should be fixed to allow for easier reporting.

Rudder test on taxi out failed. [Maintenance Control] said it had happened a week prior. Return to blocks and new plane issued.

Manual selection for [company reporting system] failed also.

Synopsis

A320 Captain reported a malfunction causing a return to the gate, then had problems reporting it on their EFB.

ACN: 1572597 (1212 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : MIA.TRACON
State Reference : FL
Altitude.MSL.Single Value : 5000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : MIA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class C : FLL

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Plan : VFR
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Facility : MIA.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1572597
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Working the FLL Final, Aircraft X was at 5,000 feet tracking inbound the ILS for [Runway] 10R. A VFR target was observed coming off of the OPF/HWO area VFR climbing to the northwest, through both FLL finals. Aircraft X was issued the traffic and could not descend on the approach because the VFR was climbing unrestricted and uncontrolled. Aircraft X was then very high and unstable on the approach, creating an unsafe situation.

FLL airport is the 19th busiest airport in the USA, and one of the top 3 fastest growing, averaging 8% growth each of the last 5 years. FLL has a basic Class C airspace surrounding it that is beyond out of date and unable to aid in the safety of its aircraft on the finals. Planes inbound to each of their parallel runways are not offered any sort of protection until within 5 miles of the field. [This] allows several dozen VFRs each day to climb, descend, and transition across each final, without ATC advisories creating a very dangerous, unsafe, and hazardous situation in the skies above. The FLL Class C is inadequate, and out of date, and needs a major airspace change around it, whether a bigger Charlie, or a full blown Class B before it's too late.

Synopsis

MIA TRACON Controller reported an airborne conflict with an IFR arrival to FLL and a VFR aircraft.

ACN: 1572103 (1213 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 150

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Commercial
Experience.Flight Crew.Total : 16070
ASRS Report Number.Accession Number : 1572103

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Commercial
Experience.Flight Crew.Total : 17300
ASRS Report Number.Accession Number : 1572105

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff [from] RWY XXL at ZZZ, about 150 feet before gear up selection, number two engine lost all the thrust. [We] completed [the] Engine Out After Takeoff Procedure. After calling no "Quick action and ECAM exception," Captain transferred aircraft controls to me. Completed QRH and overweight landing calculations; [we] did not need to shut down engine because no damage indicated. I landed safely to ZZZ RWY XY.

Narrative: 2

Lost thrust in Number 2 engine on takeoff from ZZZ. At approximately 150 feet AGL, loss of thrust on Number 2 Engine. [We] completed QRH for our particular situation and returned to ZZZ for a single engine landing. No other incidents or events occurred after.

Synopsis

A320 flight crew reported losing thrust at 150 feet after takeoff prompting an immediate return to land.

ACN: 1571172 (1214 of 2908)

Time / Day

Date : 201808

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Component : 1

Aircraft Component : Rudder Trim System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : INS / IRS / IRU
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Weather Radar
Aircraft Reference : X
Problem : Malfunctioning

Component : 4

Aircraft Component : Traffic Collision Avoidance System (TCAS)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1571172
Human Factors : Troubleshooting
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Maintenance
When Detected : In-flight
When Detected : Pre-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

The inbound [Maintenance Release] condition was 8 pages long prior to NAV IR 2 FAULT ECAM [Electronic Centralized Aircraft Monitor] and subsequent deferral. A few items of note are described below to show the added workload.

Weather Radar System deferral includes the loss of the Predictive Windshear System. This deferral required additional thorough review of adverse weather and supplementary procedures during preflight planning due to the Flight Planning Restrictions and flight crew procedures of this deferral. 

Rudder Trim 2 Fault Deferral resulted in the downgrade of the Autoland capabilities to CAT 3 Single. It required an extensive test procedure of the remaining Rudder Trim System prior to each departure and required coordination with Ramp Service Agents.

Nav IR (Inertial Reference) 2 Fault Deferral per the [Maintenance Release] NOTE states: "THE HEADING MUST BE ENTERED THROUGH THE CDU KEYBOARD FOR THE IR 1 AND HAS TO BE FREQUENTLY RESET." This deferral also required the WiFi to be deferred and maintaining the transponder in XPDR mode for all phases of flight eliminating TCAS operation completely.

All procedures and flight crew responsibilities per the [Maintenance Release] were followed and completed as directed. However, once airborne, [the] Captain, [the] Pilot Monitoring, attempted to reset the IR heading and found the instructions lacking. First, the [Maintenance] instructions state IR 1 should have the heading reset, but IR 1 was in NAV mode per the FM (Flight Manual) IR heading is entered when an IR is set to ATT (Attitude) mode (FM 6.XXX.XX.X & X). So we attempted to set the heading for IR 2 after finding the FM procedure NAV - IR Alignment in ATT Mode (FM 2.XXX.XX), using the direction for a keyboard with and H on the 5 keyboard. This procedure resulted in the following message on the ADIRS display "- - - - ATT MODE," but the heading wouldn't reset. Unfortunately, after making several attempts to follow what instruction we could find to confirm our systems knowledge the resets continued unsuccessfully. The Captain contacted [Maintenance Control] via ACARS, but that proved undesirable due to the complexity of the procedure. He tried via [Commercial Radio], but there was a Medical Emergency Med Link priority, so ATC contacted us with another frequency to reach company. At this point, he made contact with Dispatch, but [Maintenance Control] wasn't able to join this frequency so dispatch relayed our issue. [Maintenance Control] made several attempt to send instruction via ACARS, but they didn't provide any further detail. The Captain continued making attempts to "frequently reset" the heading without success. At some point the flight attendants called because the passengers were concerned about connections and unhappy due to the delay. This was a byproduct of the WiFi MEL preventing use of the [company] APP, so we printed connecting gates via ACARS. Due to the relatively short flight we had begun descent into [the destination airport] and started planning for the parallel visual approach without TCAS to help in identifying the other aircraft in close proximity.

The Rudder Trim 2 Fault and NAV IR 2 Fault were very intermittent which resulted in use of the Emergency Cancel function of the ECAM control panel to eliminate these nuisance repetitive ECAMs and Master Caution Alerts.

While safety of flight was never in question, there was workload saturation and less than normal situational awareness as the Pilot Monitoring handled the additional responsibilities.

Synopsis

A320 First Officer reported that the aircraft was dispatched with numerous MELs which caused extra workload for the flight crew.

ACN: 1570862 (1215 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZAB

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1570862
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were level at 36,000 feet. and deviating for thunderstorms approaching the RNAV arrival and the First Officer noticed dirty sock smell. No change in power setting was noted and both packs were being fed by its respective engine. I called the flight attendants and wanting not to prompt them asked if everything was okay. The A Flight Attendant immediately responded that it smelled like "dank foot." I also began smelling dirty sock at this point. We donned our Oxygen masks and after confirming no smoke or sign of fire was present ran the "Elimination of odors in the cockpit/cabin" procedure in [flight manual]. The flight attendants indicated that they first smelled the dirty sock smell in the aft cabin. Per the procedure we turned off Pack 2 and waited 5 minutes. The smell dissipated both in the cockpit and in the cabin.

We decided to continue to destination. At the top of descent with a power reduction to flight idle a faint dirty sock smell returned. We again donned our oxygen masks. Since we previously confirmed that pack 2 was causing a dirty sock smell we chose not to attempt to cycle Pack 2 on and Pack 1 off. The flight attendants also stated that there was a dirty sock smell in the cabin. Due to our proximity to our destination we continued our descent and landing as planned. I took off my oxygen mask on the approach at approximately 1,000 feet. to facilitate a better sight picture for landing. At that time I again smelled strong dirty sock smell. After landing we turned off Pack 1 and monitored cabin temperatures. At the gate and after engine shutdown I opened the cockpit window. Neither the First Officer nor I experienced any symptoms. One flight attendant reported "tingling" in their throat. No passengers reported symptoms or requested medical attention. A detailed logbook entry was made and the cabin smoke/fumes form was completed.

Synopsis

A320 pilot reported "dirty sock smell" detected by First Officer in the cockpit and Flight Attendants in the cabin.

ACN: 1570377 (1216 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Commercial
Experience.Flight Crew.Type : 1137
ASRS Report Number.Accession Number : 1570377

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Company Policy

Narrative: 1

This report documents an incident of smoke observed and fumes detected in the cabin approximately five minutes prior to closing the cabin door for departure.

I, Captain, was contacted and advised by Flight Attendant, that a customer seated in 23F had reported to her that she had observed smoke rising along the side wall of the cabin interior and that she had detected an odor consistent with something burning. Additionally, customers seated in the same row of the reporting customer in 23F, as well as in 22DEF reported similar observations. FA (Flight Attendant) stated she briefly observed smoke in a formation she described as similar to cigarette smoke. I responded to the incident location to investigate. Upon arrival, I did not observe smoke. However, as I approached 23F, I detected a faint odor consistent with electrical combustion. Based on my observation and the statements of the aforementioned customers and FA, I made the decision to deplane all customers. I then contacted [Maintenance] and briefed the Maintenance Controller of our status and concerns. Upon arrival, a Maintenance Technician initiated a smoke and fume investigation of the aircraft. Upon determining the results of the initial investigation to be inconclusive, the Maintenance Controller suggested contacting and interviewing customers seated in 23F and 24F regarding the location and contents of their carry-on property.

I returned to the boarding area of the terminal and requested Supervisor page customers assigned to 23F and 24F and request that they come to the service counter with their belongings. Since the reporting customer was seated in 23F and we were concerned about property belonging to and stowed under 23F by the customer assigned to 24F, I initially contacted and interviewed the customer assigned to 24F. I introduced myself and explained I was concerned for her safety, as well as that of the entire flight. I observed she was in possession of a backpack. I asked her where her backpack had been stowed and what items she had in her backpack. She stated she had stowed beneath 23F, and, that among other items, it contained an e-cigarette, but stated she had turned it off. I asked if she would voluntarily open her backpack. She answered affirmatively and did so. As I looked inside her backpack, I detected an odor similar to the odor I had detected during initial investigation in the incident location of the cabin. I then introduced myself to the customer assigned to 23F and asked if she could smell the interior of the backpack and inform me if she smelled anything familiar. She did so and immediately, without hesitation, stated she detected the identical burning fume odor she had detected in the cabin.

From the aforementioned statements and observations, I determined the cause of the smoke and fumes was customer 24Fs e-cigarette. In an effort to resolve the matter and ensure the safety of flight, I asked the customer assigned to 24F to take her belongings back through the TSA security check point for re-screening. Additionally, I asked the Supervisor to attempt to persuade 24F to voluntarily relinquish her e-cigarette, which she did without incident.

[Flight] was subsequently re-boarded. After an approximately two [hour] delay [flight] departed ZZZ and arrived in [destination] without further incident.

Synopsis

A320 Captain reported a passenger's e-cigarette in their carry-on caused concern which resulted in deplaning and a smoke and fume investigation.

ACN: 1570010 (1217 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 133
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1570010
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During the arrival I changed the approach from ILS XL to RNAV XXR as indicated by ATIS. The FMS auto sequenced the arrival VIA and linked the approach to the arrival. After [the waypoint] we got an ECAM (Electronic Centralized Aircraft Monitor): L/G LGCIU 1 FAULT. I continued to monitor the autopilot and the FO (First Officer) ran the ECAM. This resulted in an additional ECAM: NAV GPWS FAULT, which was caused by the previous procedure. The FO was asking "OK to clear" and while I was looking and deciding if it was OK to clear the aircraft turned toward the first fix of the approach. I was thinking about the NAV ECAM and looking at the SD (System Display) and missed this turn.

ATC immediately told us to turn right to an easterly heading and informed us that we weren't cleared to the final approach course. We complied with all ATC instructions and was then vectored to the final approach course and cleared for the RNAV XXR approach.

Once on the ground we were given a phone number to. I called and spoke with the controller and supervisor. They were professional, courteous and friendly. They said that due to construction the ILS XXR is not available and they have been using the RNAV instead. Since then, they have had numerous aircraft do the same thing that we did, in fact a wide body did it the previous day. They hadn't witnessed this occurrence with the ILS, only the RNAV. I told them about how we were trouble shooting an aircraft issue and missed the automation mistake. I also said that we hadn't informed ATC about our ECAM because we hadn't gotten there yet. I explained that most ILS approaches in the Airbus 320 don't connect to the arrival but the RNAVs do. I also mentioned that we don't do many RNAV approaches and my briefing emphasis was on the approach and not the arrival. When the first ECAM occurred I should have informed ATC that we needed an extended downwind thus giving us time to clean up the ECAMs.

Synopsis

A320 Captain reported a fault in the aircraft navigational system during the arrival.

ACN: 1569120 (1218 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
Make Model Name : B737 Next Generation Undifferentiated
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1569120
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Ground Event / Encounter : Aircraft
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Evacuated
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After pushing back from [the] gate we received our taxi clearance from outbound Ground Control. With both engines running we started our turn towards [the] taxiway with no wing walkers or guidance from ground personnel. As the FO in the right seat I had my head turned to the right clearing the area where we were about to taxi. Shortly after starting our forward movement I felt the airplane shudder and we immediately applied our brakes and came to a stop. After a short discussion with ground personnel it was determined we struck a parked B737 parked at [the adjacent] gate. The flight attendants reported smelling a fuel smell in the cabin shortly after we came to a stop. We notified Ground Control and requested the fire department investigate the fuel smell and examine the aircraft exterior. It was then determined we should not move the aircraft for fear of causing additional damage. The fire department then provided a set of air stairs and we deplaned the passengers via the 2L door without incident.

Synopsis

A320 First Officer reported striking a parked aircraft at an adjacent gate while taxiing.

ACN: 1568859 (1219 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Descent
Route In Use : Direct
Airspace.Class D : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1568859
Human Factors : Workload
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1568862
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Weather
Primary Problem : MEL

Narrative: 1

On our initial descent into ZZZ the Captain's [side] windshield was not heated and began to freeze. The Captain visibility became impaired and resulted in him transferring the controls to the First Officer. At that point, I acted as Pilot Flying and continued the approach into ZZZ. I was given a poor vector to the outer marker which aided in me being [slightly] high on the approach. At 1,300, [I] began to verbalize go-around and Pilot Monitoring stop me mid-sentence. During the visual approach into ZZZ, I was not stabilized by 1,000 above field elevation and recommended go-around. The Pilot Monitoring stated "hold on, you are trending correctly and we should be okay." We were stabilized by approximately 900 [feet]. Once we landed safely and slowed the aircraft, the Captain asked me to continue taxiing since he had poor visibility on his side and my visibility was fine. We made it to the gate safely without incident. They then proceeded with contacting Maintenance regarding the windshield. The Captain open [his] side window to gain more visibility.

The aircraft had too many MELs and Management could be more supportive versus citing intimidation during operations. We need to feel like a TEAM and not "I." [I should] be more assertive with my go-around decision. Perhaps, I should have not continue to taxi although it was totally safe. In the future, I will not taxi a plane as a First Officer no matter the situation. [I should] be more assertive with the Captain about Company time pressure when preparing for a flight. If I am rushed, I will be more assertive to [the] Captain with stating that I am not prepared and I need more time to ensure I review all items and perform all necessary checks prior to flight.

Narrative: 2

Despite asking for a later report, [I] was told to head to airport for a 3 hour sit. On arrival, the aircraft had [many] MELs. I was concerned with a MEL. I also found an engineering report for a TDI only allowing 10 legs and asked Maintenance to come out to the airplane. I called [Operations Center] because I was being dispatched into a thunderstorm and I was concerned I did not have a functioning windshield heat. Dispatch had been worthless and I would say under some influence. After discussing with personnel from [Operations], I felt confident that only the side windows did not have de-ice feature which resulted in the same issue but it was a valid MEL. The difference being a side window is identified as a window and windshield as a windshield.

On descent, every window on the left side of the aircraft was covered in ice. I could not knock it off with the windshield wiper observing the speed restriction nor could I use a c-fold to rectify the situation. Quite simply, I could not look out the left side of the aircraft. I handed control to the First Officer. We were given a bad vector and slightly unstable on approach but we were trending to stabilized and were fully stabilized by 954 feet. We landed. I elected to open my window so that we could have vision off the left side and have the First Officer taxi the aircraft since he had full vision to the gate. I also thought the air in and out of the window would improve the situation. Otherwise, I would have needed a tug. During taxi in, the aircraft was getting spurious FMAs, apparently retrying to capture the ILS even though we were on the ground. On Shutdown, I contacted [Operations], Dispatch, [and] Maintenance and informed them that I would not take this aircraft out again until I could see out of the window. I also wrote up a high brake temperature.

I then received a call from the Chief Pilot Office. Apparently, [Operations] had contacted them. The mechanic discovered that they had collared and PULLED the circuit breaker for the Window Heat Computer, which was wrong. He pushed the circuit breaker back in and signed off the aircraft. I again stated my position. "If I look out the window and can see, I am more than happy to take the plane." The mechanic got angry and stormed off claiming I should use my fore arm to clean off the windows and somehow use them to get rid of the ice on the windows, both interior and exterior. I again called the duty phone to let them know what was going on. I took pictures as well and sent them. It was apparent to me that this was not getting resolved and we needed leadership to fix this. Up to 3 hours had past, you could not see out of the left side of the aircraft. Quite simply I was getting angry. THIS WAS NOT A SAFE AIRPLANE. I felt pressured by Maintenance and [Operations] to take the plane regardless. 

After a while, window heat defrosted the windshield, but the side window was still frosted over. I had no way of a clear view to my right side during a taxi. I had the mechanic wipe down this window outside, but it would quickly fog back up. I instructed them to board and if need be, I could open the window on the taxi to wipe it off until we got in the air. I sat down to get the airplane ready for flight at which time I received 4 messages from my Dispatcher claiming [Operations] was in a "witch hunt" blaming the current situation on us and claiming that taxiing with the window open was both illegal and not proper procedure. I then refused the aircraft at this point. [I] demanded to talk to a Chief Pilot instead of the Assistant Chief and explained the situation. We cancelled the flight. The window continued to worsen as we sat there. I felt pressured by [Operations], [and] not supported by the Chief Pilot. The end game that I explained to them for over 2 hours was that if I could see out of my windows, I would be happy to fly the aircraft. They cancelled the flight and rebooked us deadheads later. Personnel in [Operations] then claimed over the phone that he never questioned me but then asked the Chief Pilot if I could legally have taxied the aircraft to the gate when I arrived in ZZZ.

Synopsis

A320 flight crew reported cockpit windshield heat was MELed and Captain's side window froze allowing no visibility from left hand side windows.

ACN: 1568425 (1220 of 2908)

Time / Day

Date : 201808

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1568425
Human Factors : Workload

Events

Anomaly.Ground Event / Encounter : Person / Animal / Bird
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Airport
Primary Problem : Airport

Narrative: 1

On the takeoff roll, a bird strike occurred shortly after the 100 kts call. Visibility was good but it was not yet daylight. The bird appeared to be the size of a duck or large crow and struck the FO (First Officer) forward windscreen before bouncing to the right side of the aircraft. The Captain was pilot flying and initiated the abort which was executed as briefed. Fire trucks were requested and they met the aircraft on the taxiway after it was determined that it was safe to clear the runway and that there were no injuries. In concurrence with their assessment and QRH/FM guidance, the aircraft was returned to the gate where the passengers were deplaned. Subsequent inspection did not reveal damage to the aircraft except for the loosening of a seal on the number one engine thrust reverser due to deployment. The decision to abort was supported by both crew members due to the size of the bird and its relative speed at impact. Without confidence that the windscreen had not been damaged and the bird had not been ingested by the number two engine, assurance that the aircraft could become and remain airborne was in question and that criteria was briefed as an acceptable reason for an abort in the high speed regime.

Synopsis

A320 First Officer reported bird strike on takeoff roll and aborting the takeoff.

ACN: 1568410 (1221 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Type : 680
ASRS Report Number.Accession Number : 1568410

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

[During takeoff roll], having accelerated through 100 kts (roughly at 120 kts) but prior to V1 a very large bird (the size of a duck) struck the FO's front window (R1) sounding and feeling like someone had hit the aircraft with a sledge hammer and momentarily covering most of that window before sliding off to the right. Due to the severity of the impact, the size of the bird, and with no way to verify what else the bird may have struck as it moved aft, I had serious doubts about the aircraft continuing to be is a safe condition to fly so I elected to reject the takeoff. The aircraft smoothly decelerated and came to a complete stop still leaving us with about 3,500 feet of runway remaining. When at a complete stop we ran the RTO (Rejected Takeoff) checklist, requested as a precaution that ARFF come out to the aircraft in case the brakes caught fire. We then verified all were still seated and as the brakes were not overtemping we taxied clear of the runway.

We then communicated what happened to the cabin crew and verified all were ok. Then while monitoring the brakes to make sure they just remained hot and not temp critical we advised the passengers of the situation. We contacted Dispatch/Maintenance (who requested we return to the gate) and Operations. Then we verified the aircraft was in a good condition to taxi, that the brakes had cooled, and taxied back to the gate. No passengers or crew were injured so we deplaned the aircraft, sent in an [Maintenance Request] for the bird strike and rejected takeoff and then we notified the [Chief Pilot]. Maintenance came out to inspect the aircraft and verified that the brakes and tires had not been damaged during the RTO and that the bird had not created any secondary damage to the aircraft. During the Maintenance inspection, it was discovered that a gasket/seal had come loose on engine 1 when the reversers were deployed. The technician informed us that this would most likely have happened during a normal landing and reverser deployment was directly due to the RTO.

While the left engine was being repaired I continued to coordinate with all crew members that they were ok and could continue once the Maintenance was completed. All cabin and cockpit crew members repeatedly informed me they were good both physically and mentally to complete the trip. [Once maintenance] was completed we coordinated with Dispatch for a new release and [a Maintenance Release]. We then re-boarded the aircraft and departed without any further incidents.

Synopsis

A320 Captain reported rejecting the takeoff after striking a large bird at approximately 120 knots.

ACN: 1568405 (1222 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class A : ZZZ

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 7214
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6764
ASRS Report Number.Accession Number : 1568405

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Inbound flight had a "Flight control - Spoiler fault."

Maintenance deferred Number 5 spoiler. This deferral required the Number 5 spoiler to be "inoperative, retracted, and deactivated" per the MEL. Enroute, we discovered that the airplane with the autopilot on, was flying with 2.5 degrees left rudder trim and about 2-3 degrees right bank. FO (First Officer) made an inspection of the spoilers from the cabin.

It was discovered that the right spoiler was "floating" i.e. extended during flight when it should have been retracted along with all the other spoilers. Per QRH, this causes an additional 10% fuel burn. As this was a relatively short flight, we determined that the additional fuel burn was not a big concern. As we were beginning descent at this point, the decision to continue to destination was made with no further notification (due to time constraints and relatively minor effect on the flight plan.) We didn't believe this met the criteria for declaring an emergency, and therefore did not.

The flight was continued to destination, landing about 900 lbs short of planned fuel due to the extended spoiler. Maintenance "write-up" was made, and out bound crew advised.

Synopsis

A320 Captain reported Maintenance deferred an inoperative spoiler.

ACN: 1568070 (1223 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Rudder Trim System
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1568070
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

Aircraft X had MEL 27-22-XX Rudder Trim Sys 2 applied. This requires Operations Procedures - Test "2" to be completed. During this test while following directions you hold the rudder trim full L and reset, then hold full R and it instructs you to push in the FAC2 and the test is complete, it omits the reset step after full R trim. I called Maintenance Control and asked them about it and after much confusion on their part she finally understood what was missing and instructed me to push the reset. Test 1 for a Sys 1 does instruct the reset to be pushed after full L and R. Test 2 is missing vital written language the leaves the aircraft in an unsafe condition for pushback. Maintenance manuals need to be accurate and complete for safety of our flight crews and passengers.

Synopsis

A320 Captain reported the procedure to perform a flight control test per the MEL was incomplete.

ACN: 1567654 (1224 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 800
ASRS Report Number.Accession Number : 1567654

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Last 90 Days : 59
Experience.Flight Crew.Type : 59
ASRS Report Number.Accession Number : 1567290

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the First Officer operating Aircraft X. We were in an Airbus A320.

The flight departed ZZZ at XY32 and climbed to FL330. At approximately XY55 we received the ECAM caution messages, HYD Y ENG PUMP LO PR, and HYD Y RSVR LO LVL. A review of the hydraulic page revealed zero pressure in the Yellow hydraulic system and a negligible quantity. We ran the ECAM items and then reviewed the QRH. We concluded that the ECAM checklist was appropriate; disabling the number 2 engine hydraulic pump and the PTU. We also concluded that the appropriate QRH checklist was for HYD Y RSVR LO LVL and reviewed it fully.

The two items of concern on the checklist was the possibility that the spoilers 2 and 4 would float without hydraulic pressure, and that the parking brake accumulator would not have pressure for parking. We had a deadheading pilot check the spoilers, and he stated they appeared to be down - the flight characteristics of the airplane supported this observation. The brake accumulator did not deplete quickly, but on our arrival, there was insufficient pressure remaining to adequately set the parking brake.

We discussed the option of diverting, and then contacted dispatch and maintenance control. We concluded that diverting to ZZZ or ZZZ1 was a proper plan of action. At this point we were closer to ZZZ, but factoring in the winds, ZZZ [over] ZZZ1 were of similar time. 

We also re-considered diverting to a closer airport. With the yellow system failed and the spoilers closed, there was no degradation of the immediate safety of the flight. Diverting would have resulted in an overweight landing, or holding for an extended period of time. We deemed that a higher risk than continuing to ZZZ1.

We descended to a lower altitude to increase fuel burn to assure the landing in ZZZ1 would not be overweight. I calculated the fuel load that would result in maximum landing weight and we referenced this prior to landing.

Approximately 200 miles from ZZZ1, we [notified ATC]. The flight was operating normally and there was no safety loss while en route. Our declaration was to provide us with emergency equipment on the field, runway choice, and greater leeway with ATC considering the slower flap speed, and our inability to fully extend the speed brakes.

I briefed the captain to expect a substantial yaw with only one reverser deployed. We concluded the best course of action would be to land with the flaps in the Full position, auto brakes in Low, and idle reverse with the No. 1 engine.

The approach and landing were uneventful, and we were at approximately 141,000 pounds on touchdown. Fire and Rescue circled the aircraft and reported no apparent problems. Steering and braking were normal, and other than the loss of the Yellow hydraulic system and the accumulator pressure (and thus the parking brake), there were no other system or operational degradations. I made a PA announcement while the airplane was stationary to inform the passengers of the actions of the Fire and Rescue crew, and that we would be proceeding to the gate after inspection.

We taxied to the gate, and when I contacted ramp tower, I asked them to remind the ground crew that the aircraft would have to be chocked prior to engine shutdown as there was no parking brake available - we had contacted the station prior with this information.

Once the aircraft was at the gate and chocked, the engines were shutdown. Brake temperatures peaked around 450 degrees.

Station representatives met us at the gate, and we discussed the matter with maintenance personnel as well. Both the captain and I were debriefed by representatives from the ZZZ1. 

Reviewing our actions, I would conclude that given the information that we had been provided, that the decision making process was sound and that no better option existed. The smooth air en route, the long runways at ZZZ1, and the time to prepare and plan was a superior course of action versus an immediate diversion and an overweight landing, considering the only failure and risk was the loss of the Yellow hydraulic system.

Narrative: 2

We reached our initial cruise altitude of FL330 without incident. I pulled up the synopsis page for all aircraft systems and all appeared normal. After several minutes, we received a HYD Y RSVR LO LVL ecam. The reservoir was completely depleted. I took control of the aircraft and radios and had the FO run the ECAM and QRH. The ECAM called for turning off the Y ENG pump. I then called dispatch and had Maintenance patched into the call as well to go over the mechanical. With the yellow system inop, we were left with partial spoilers and only 1 thrust reverser. With that in mind, I did not want to continue to [destination] with a short runway. After conferring with dispatch, it was decided that ZZZ1 was the best solution for a divert as we had to burn off roughly 6200lbs of fuel to get below landing weight. We continued the flight and landed uneventfully and taxied normally to the gate.

Synopsis

An A320 flight crew reported that ECAM displayed a "HYD Y RSVR LO LVL", and the reservoir was completely empty.

ACN: 1567376 (1225 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Data Transmission and Automatic Calling
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1567376
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft Data Transmissions (bursts) on VHF3 bleeds over and blocks ATC radio transmissions on VHF 1 and 2. We flew over a 2 day period in the same aircraft, Aircraft X and the interference occurred on all frequencies (ramp control, ground, tower, departure, enroute, and approach control) and at all airports. The interference occurs approximately every 5-6 seconds and blocks out all transmissions making ATC instructions and call signs impossible to hear. The only way to stop the interference is to deselect "DATA" on VHF 3. In doing so, you can't receive ACARS messages from dispatch, retrieve ATIS or send messages. While at cruise we would periodically turn VHF 3 back to DATA, check for messages, retrieve weather, send specials, obtain landing performance information then turn it back off. It would remain off once we started our descent until we blocked in at the gate. Once returning to ZZZ, I entered a discrepancy in the aircraft logbook.

The interference from these DATA bursts is becoming more and more frequent and no longer limited to just approach and tower frequencies. Seems to be the newer the aircraft the worse the interference is. This has been an ongoing problem for many years. This aircraft is the worst yet.

Synopsis

Airbus A320 Captain reported that aircraft data transmissions on VHF3 bleed over and block ATC radio transmissions on VHF 1 and 2.

ACN: 1567114 (1226 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Company
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1567114
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Primary Problem : Human Factors

Narrative: 1

Flight was dispatched with ZZZ listed as a destination alternate. After taking a delay for a late inbound aircraft, this delayed the flight out past the time for ZZZ to be a legal alternate for his eta as ALT MINS NA WHEN TOWER IS CLOSED. This was an oversight by the dispatcher who was dealing with a weight and balance issue out of ZZZ1 and ZZZ2, reroutes out of ZZZ3, and holding for ZZZ4. 

I attempted to have other dispatchers on duty to possibly assist me in shedding workload, but they were also saturated with similar issues to my own. At the time, most dispatchers had 10-15 flights airborne each. Adding additional staffing, by either dispatchers or support personnel, would have prevented the saturation. Additionally, a thorough review of the NOTAMs before issuing the release time would have prevented the issue. I did check the weather conditions an additional time but missed the NOTAM.

Synopsis

Air carrier Dispatcher reported that a delayed flight departed with an inappropriate alternate due to a NOTAM not reviewed during planning. Staffing issues were cited as contributing factors.

ACN: 1567053 (1227 of 2908)

Time / Day

Date : 201808

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Ground Personnel : Other / Unknown
ASRS Report Number.Accession Number : 1567053

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After I finished loading the flight I got a message saying that a Hazmat quick pack was added about 50 minutes before departure. I added the Hazmat to pit 2 with a planned 16 bags. I then called the Zone Controller to make sure they call the ramp and tell them to secure the hazmat with 16 bags. I then did my other flight and got busy with a flight. After I finished my flights I looked at all my finalized flight and noticed that Flight X did not have 15 bags minimum with the Hazmat and had just taken off.

Synopsis

Load planner reported miscommunication by Zone Controller resulted in an aircraft departing with incorrectly loaded HazMat cargo.

ACN: 1566796 (1228 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N

Component

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1566796
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During flight plan review, we noted [an] MEL. Reviewing the MEL, we observed that it signifies inoperative Engine Driven Generator, and since 'IDG' was mentioned, we thought that IDG was disconnected. We noted that this MEL comes with several M, O, and DP requirements. One of them being the following maintenance requirement:

Install the placards as follows: 1. On the ELEC Panel (35VU) to read: L (R) ENG IDG INOP.

Therefore, we prepared for and briefed operation with a Single Engine Driven Generator. After engine start, we observed both engine driven generators operating normally, which was unexpected and confusing. We then contacted ZZZ Station Maintenance thinking that perhaps the generator was repaired but the logbook was not updated. We were told that the MEL was still active, and the problem with the generator still existed, it was not repaired due to lack of parts. We asked them why the generator was operational, and not IDG disconnected or otherwise rendered inoperative as the MEL indicated. We were told that the problem was intermittent, so the generator was left useable. Thoroughly confused by then, we contacted Dispatch about a possible MEL discrepancy. We were told the same story as above, and that the aircraft already operated in the above condition the previous day.

Since we were not getting anywhere with this, and not wanting to delay the departure any further, and since safety of operation was not impacted because all generators were operational - we departed for ZZZ1. Nevertheless, after further review of the situation, it is my belief that either Maintenance is using this MEL improperly, or the MEL is not written clearly to address the situation as above.

Based on my aviation career to this date, if a problem is observed and logbook write-up made, and then Maintenance decides to defer repairs using a MEL, the malfunctioning equipment is rendered inoperative and not used in flight until repaired. Our MEL states: "Flight Crews will not operate any deactivated or placarded inoperative item or system." In this situation, dispatched with MEL 24-XX signifies to me operation with inoperative Engine Driven Generator. And yet the generator in question was operative, and we were expected to use it. This, to me, contradicts the letter of our MEL. [The cause was] either incorrect usage of the MEL by Maintenance, or lack of Company guidance that the above described MEL use is indeed approved.

If indeed the company and the FAA approve the usage of the MEL as described above - when a component that faulted - and was written up - is deemed defective, and it is placed on deferral using MEL, but is left operational for as long as it still works - then this should be clearly stated in the MEL, as to preclude flight crew confusion and possible resulting flight delays or cancellations due to flight crew trying to deal with the issue. Otherwise, if faced with similar situation in the future, I will refuse the flight until the MEL is removed, or the component in question [is] deactivated following all the maintenance steps listed in the MEL.

Synopsis

A320 pilot reported that the IDG was improperly deferred.

ACN: 1565854 (1229 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1565854
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

On descent into CLT, got ACARS message trying to assign additional flying after the termination of my trip. I did not acknowledge the message because we were entering a high workload environment, descending via CHESLY 3 RNAV Arrival. It created a big distraction, which is an impediment to safety.

I knew I could not accept additional flying because of: 1. Possible fatigue - had already been up nearly 15 hours. 2. Physiological needs - had not eaten anything since lunch.

Company trying to reschedule while pilots are entering high workload environment. Any schedule change should be while on ground.

Synopsis

A320 Captain reported while entering a high workload environment an ACARS message came requesting additional flying.

ACN: 1565164 (1230 of 2908)

Time / Day

Date : 201808
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : D01.TRACON
State Reference : CO
Altitude.MSL.Single Value : 11000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : STAKR 3
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Taxi
Make Model Name : Citationjet (C525/C526) - CJ I / II / III / IV
Operating Under FAR Part : Part 135
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors
Airspace.Class B : DEN

Aircraft : 3

Reference : Z
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 200 ER/LR (CRJ200)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : DEN

Aircraft : 4

Reference : A
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 170/175 ER/LR
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class B : DEN

Aircraft : 5

Reference : B
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Taxi
Make Model Name : Falcon 900
Operating Under FAR Part : Part 135
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : DEN

Aircraft : 6

Reference : C
ATC / Advisory.TRACON : D01
Make Model Name : Citationjet (C525/C526) - CJ I / II / III / IV
Operating Under FAR Part : Part 91
Airspace.Class B : DEN

Person : 1

Reference : 1
Location Of Person.Facility : D01.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 7
ASRS Report Number.Accession Number : 1565164
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Facility : D01.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2
ASRS Report Number.Accession Number : 1565174
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 726
ASRS Report Number.Accession Number : 1565164
Human Factors : Communication Breakdown
Human Factors : Confusion
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Airspace Structure
Primary Problem : Procedure

Narrative: 1

I was working DR4, a departure gate to the south. Land north at DEN IFR. Land south at APA. I had a flight of F16s arrive from the southeast requesting the Sandbox. There were only a few arrivals to 34R, which could not come in on the approach with the Sandbox hot, so I called final to see if they were willing to accommodate it. Final said they would only have to slightly delay the arrivals for the Sandbox so it was approved. I said point out approved for the first downwind to vector around the Sandbox to the west in my airspace. Coordination was complete and I cleared the fighters to the Sandbox. TM (Traffic Manager) began to question the operation and said I could not do the Sandbox with aircraft on 34R, this got my immediate attention and I began to scan the airspace to see if I'd missed something, double checked that I was ok with final and explained to the TM what we had coordinated. 

In that moment I hear the SR4 Controller saying I'm sorry and Aircraft Y turns yellow on my scope, head on with Aircraft X climbing out of 100. My Aircraft Y was also climbing out of 100, I told him to stop his climb and turn to a 260 heading and issued traffic. He was slow to respond. I tried asking SR4 what he was doing several times, the only response I got was "turning eastbound". I stopped my Aircraft Y from climbing because I knew that SR4 had to get his aircraft above or he'd run into several other aircraft on the departure and arrival. Turning eastbound would mean he was entering the Sandbox, a protected area for the fighters. It was unclear what action he was taking. Turned out, SR4 also stopped the climb on Aircraft X who leveled at 108 and had separation errors with two other aircraft. 

Aircraft Y reported an RA and while I was straightening all that out I realized final had another aircraft on the downwind they were vectoring wide around the sandbox, this aircraft was not yellow on my scope and no coordination was done for a point out. I realize that the aircraft had to go that way to miss the Sandbox but with everything else I had going on, I did not stop my Aircraft Z from climb via in time. When I saw the two aircraft in close proximity, they were on parallel headings, mine was already climbing above with the arrival descending so I did not take any further action. 

Additional assistance by the Supervisor could've aided some of these situations. I would expect my Supervisor to be able to coordinate or offer an extra set of eyes but he did not come over. I have seen that particular Supervisor throw his hands up and imply there was nothing he could do to assist a Departure Controller in trouble. He is I believe uncomfortable and unfamiliar with the policies and procedures of the departure wall. Training on both familiarization and how to assist would be helpful.

I noticed what seemed to be radar lag when I turned Aircraft Y, there seemed to be a significant jump on his track after what felt like a long time after I'd turned him. A calibration check on the radar if that's possible. I would like to see a best practices study on how to accomplish what we call the up and over with efficient coordination. Our SOP does not detail how SR4 can efficiently climb an aircraft to go through DR4's airspace and heading out the north and east gates. It is currently being done by a point out but there is so much other coordination to be done that steps can get missed. Because the up and over is frowned upon by some controllers, there is peer pressure to either not do it or avoid coordination all together. In this situation, I was very busy and SR4 was trying to do me a favor by working the aircraft, as he became more busy the coordination was not completed. Working the aircraft below the final would have added complexity and additional traffic conflict situations, that often lead to RA climbs, not to mention a delay to the aircraft. There are times where the up and over is so much safer and more greatly efficient that it is a great tool and worth further evaluation. There are also times where poor judgment is made and errors occur, usually with a lack of policy and proficiency.

Coordinating the Sandbox could have been a contributing factor but I stand by my decision to do so. Because SR4 was busy with aircraft under the final airspace, and because there were arrivals coming in the SE, workload would have been higher and more complex if I'd decided to go under the final. It also would've caused several more traffic conflict situations. I'd like to see refresher training how to recover from an error. I was extremely frustrated that the SR4 controller was unable to tell me what he was doing with the aircraft. Logic would have told me that he had to continue climbing in order to miss the other aircraft but he didn't. Reinforcing right turns for head on conflicts along with vocalizing altitude would have been a help and a comfort.

The Final Controller told me that the Feeder Controller was still working the downwind Aircraft Z that was on a vector to miss the Sandbox. Final was yelling for the aircraft as he saw the conflict with my departure. Final said that feeder would not switch the aircraft or respond. Final said he wanted to put the aircraft on a better heading to achieve divergence with my departure. Feeder controllers are much less familiar with departure traffic and are not accustomed to missing or even seeing departure aircraft. Refresher training for arrival controllers on departure procedures and traffic could benefit awareness.

Narrative: 2

Working SR4 with multiple VFR flight following, multiple APA departures/arrivals on frequency. A number of VFR callups were trying to check in. There was also a Class B violator that was being tracked and discussed which ended up being one of the VFR callups that I told to standby. Having been on position for over an hour with only moderate traffic, I don't think I was mentally prepared for the wave of traffic and requests that hit me. During the session I had climbed a number of east gate departures off of Runway 17L at APA via a right turn and over DEN pointing them out to DR4 due to there being less traffic in the way than a left turn off the deck and restricted to 080. When Aircraft Y departed in the right turn, I had to get divergence with Aircraft B, an APA inbound, before I could climb him. At that point I was dealing with a lot of traffic and attempted to hand Aircraft Y off to DR1. The tag was in handoff status for a minute or more and at that point I realized the DR1 controller was probably too busy to take the handoff so I climbed the aircraft but didn't realize that I had not done the coordination with DR4. DR4 was busy as well with coordinating the SANDBOX with all affected controllers and also had multiple DEN departures at the time. South gate departures were departing west of DEN probably due to the high volume of east gate departures which took me by surprised when I saw the conflict because with the closure of Runway 7/25, south gate departures have primarily departed off the east side. Aircraft X was climbing via the SID leaving 105 or so when I saw it, while Aircraft Y was climbing out of about 100. I immediately told Aircraft Y to stop the climb thinking that would be the best way to keep separation. I was trying to coordinate with DR4 but she was too busy, meanwhile the CIC was trying to tell me that a Cessna that I was vectoring was entering the SANDBOX. With all this I didn't realize that DR4 had stopped Aircraft X's climb as well. I did issue a turn to Aircraft Y but the closest separation was approx. 1.66 NM and 300ft vertical. Due to the turn issued Aircraft Y and because the SANDBOX was active, forcing arrivals to turn the downwinds to the west, Aircraft Y was closing in on Aircraft A who was level at 110. As soon as I saw this, I climbed Aircraft Y but separation decreased to approximately 1.68 and 900 feet. While this was going on I put Aircraft B on a northbound heading and descended to 080. I also to a handoff on Aircraft C at 090 inbound to APA from DR1. At the time of the handoff, Aircraft C was at 090 and I believed that even though Aircraft C and Aircraft B were converging, I would have separation. DR1, however, descended Aircraft C to 080, which was what they were supposed to do in that configuration, but I didn't realize it at the time due to everything else going on. Aircraft B and Aircraft C passed 2.75 NM apart at the same altitude.

Procedures have been established to take APA departures eastbound at 080 from APA underneath DEN arrival airspace. That procedure certainly has benefits, and would have been better in this case, but on certain days that airspace can be packed with VFR traffic in the practice area as well as typically being used for APA prop arrivals from the east. There are a number of good ways to depart APA with east gate jets, however none of the options seem to be a good idea all the time. I think the APA departures should be looked at and explore other option or procedures. As it is, either way there are multiple frequency changes or coordination involved. There was also no one helping to watch the sectors or to help coordinate even though DR1, DR4, SR4 were all very busy.

Narrative: 3

TCAS RA while on STAKR3 RNAV departure. Crossed BRKEM level at 10000 ft as per "climb via" instructions (BRKEM at or below 10000), then continued climb after BRKEM towards next fix, NMBSS (approximate heading southwest). Climbing through approximately 11,300, Denver Departure controller said "Aircraft X stop climb" followed shortly after with a turn instruction to a heading westbound. At this time, the aural alert of "monitor vertical speed" presented itself, very quickly followed by "adjust vertical speed". The TCAS RA commanded a descent of approximately 2200-2300fpm. I complied with the TCAS RA appropriately (autopilot was off already as I had not turned it on yet). Once we were "clear of conflict" at/around 10000 feet MSL, we were given instructions for a further climb and heading change near on-course from the Denver departure controller. The conflict aircraft was never heard on the radio, or addressed by the controller, however the Captain made visual contact at one point during the RA.

Synopsis

Denver TRACON Controllers and air carrier First Officer reported multiple losses of separation and airspace violations.

ACN: 1565078 (1231 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : Mixed

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1565078
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I am not sure if the original controller actually meant to send us direct to ZZZZZ or to ZZZZZ1 but I believe having the controller spell it and confirm the fix when there are two very similar sounding on a flight plan would help.

Synopsis

A320 Captain reported a communication breakdown with ATC in regard to route clearance.

ACN: 1563413 (1232 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Thunderstorm

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZDC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDC
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 7.0
ASRS Report Number.Accession Number : 1563413
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I was working combined sectors and had a few arrivals that were deviating slightly east of course that I had pointed out to the Patuxent Restricted Area, all in level flight. Aircraft X was at 35000 ft, on his way back on course from his deviation for weather. Patuxent Advisories already said point out approved. Later, they called back for a point-out to me on an aircraft with beacon code XXXX, and the controller said he was at FL300, just east of PXT. I saw the code, and said he's at 30700 ft. Then the controller said "I lost radar." I said point-out approved for the spill-out, and begin to move my traffic out of the restricted area. I tried to move the one at 33000 ft out of the restricted area, since it appeared that the aircraft squawking XXXX was heading south and climbing out of 30700 ft. Next thing I know, Aircraft X north east of the XXXX code said he was responding to an RA. I saw the limited data block in the Patuxent Restricted area at 34000 ft something and merging with Aircraft X's target, as Aircraft X climbed up to 35500 ft or more. It happened so fast I was not able to provide a traffic call or anything. 

We always have done point-outs with the Patuxent Restricted area when we need it. These were minor deviations, and the restricted was active 40000 ft and below. We had called to try to get some of the airspace back, but they would not release it. After reviewing the events, and with the Patuxent controller saying "I lost radar" I am not sure we should be doing point-outs to the Patuxent restricted area if they cannot relay traffic information to the aircraft operating under military control, provide control instructions to them, or even know where aircraft is? I think we need to review these procedures and the Patuxent restricted area Letter of Agreement and revise our procedures, for Area 1 and Area 4. In addition, if the restricted area is active, and there are miles in trail restrictions we need to look at TMU (Traffic Management Unit) restrictions and routes to alleviate this situation when there is weather.

Synopsis

DC Center controller reported an aircraft approved to deviate for weather into a Restricted Area received an RA and took evasive action to avoid traffic in the restricted airspace.

ACN: 1562665 (1233 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y

Component

Aircraft Component : Fuel Distribution System
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1562665
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Routine Inspection
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

Upon showing to the airport and checking the logbook as I normally do, I found that the MEL 28-15-XX and 28-15-XX that were applied to Aircraft X the previous two days had been changed for misapplication of the MEL. The correct MEL should have been 28-15-XX and 28-15-XX which considers the outer tank fuel unusable for flight planning calculations. When the first two MELs were applied the situation we had (transfer valves opening on ground) seemed to be the correct MEL. After subsequent flights on [the next day] another crew saw the transfer valves did not open and suspected the wrong MEL had been applied. This plane has had multiple issues with the fuel system and everything Maintenance does to fix the problem doesn't seem to fix the issue. I am filing this report because possibly flew the airplane with the incorrect MEL applied. When I flew the airplane the MEL seemed to be correct in that the transfer valves were open and that the fuel from the outer tanks drained to the inner tanks before the inner tank reached its low level sensor to command them to open. If the future if I am ever uncertain about an MEL, I will talk to the local mechanic and Maintenance Control to make sure we are all on the same page and that we get the right MEL applied.

Synopsis

A320 First Officer reported MEL may or may not have been applied correctly as associated with the fuel system.

ACN: 1562635 (1234 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : Mixed
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1562635
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

While at level cruise of FL320, we approached a very narrow and rapidly developing isolated thunderstorm. Enroute controllers typically do an exceptional job of calling out severe weather and no such callout was made to alert of the weather. Equally important is the failure of this cell to appear on airborne radar until effectively no time remained to steer clear of it.

I was actually sending an ACARS message to Dispatch to query about enroute weather ahead when I heard the Captain call out an area of weather straight ahead. Initially, he verbalized the weather as yellow, which typically denotes light precipitation, but this was strange primarily because it wasn't showing up on radar prior to such close proximity [and] also the fact that light precipitation doesn't typically exist at such altitudes. Almost like a countdown, the intensity of the weather ahead increased from yellow to red to magenta. From first sign of encountering the weather that led to the upset, was less than 10 NM. The Captain attempted to obtain clearance to deviate, but the frequency was cluttered with other aircraft requiring deviations due to the large number of buildups. By the time the severity of the weather (which led to the event) became apparent, a turn would have been impossible and we concluded that the implications on passengers, staff, and the aircraft would have been far worse if we encountered a severe weather upset while in a bank. For better or worse, we also concluded that the event would be over quite quickly.

From start to finish, we estimate that we gained and lost roughly 1,000 feet of assigned altitude. I do know that we came within 300-400 feet and roughly 3-5 NM of [another air carrier] aircraft, who was given a brisk turn (40 degrees) when we were in the downside of the event. The event began with an aggressive altitude gain, followed by an equally aggressive altitude loss. So, the close proximity to the [other] jet was near the conclusion of the event.

As soon as aircraft control and flight path/assigned altitude were reestablished, the Captain called the flight attendants. Initial reports were of no passenger injuries and very minor injuries of one flight attendant. The second call revealed more extensive injuries [involving] the two aft flight attendants. Up to this point, the Lead Flight Attendant played down the injuries (based on statements made by the affected individuals) and that we could continue to destination. The third communication revealed "...one flight attendant was 'out of commission'," at which point the Captain made the decision to return to [departure airport].

Emergency medical staff awaited the arrival of the flight and the aircraft underwent maintenance inspections following such g-loading forces. The deferral of the No 2 Gen, which was a somewhat complicated deferral procedure for required action items each leg, also restricted operations above FL330. Had we not been constrained by this altitude limit, we would have easily topped this convective buildup. Based on the absence of an alternate, I conclude we could have climbed to at least FL360, if not FL380, based on weight.

Synopsis

A320 First Officer reported returning to departure airport after a Flight Attendant was injured during a thunderstorm encounter.

ACN: 1562629 (1235 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Make Model Name : A320
Flight Phase : Climb
Flight Phase : Cruise
Airspace.Class E : ZZZ

Component

Aircraft Component : Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1562629

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Diverted to [an alternate airport] as the result of fuel valve malfunction (outer to inner tank valves, right wing tank). This [has] been a recurring maintenance issue on this aircraft. It required a repetitive inspection and maintenance action prior to each departure. We had a mechanic assigned to travel with us as contract maintenance was unavailable. Prior to departure our mechanic experienced difficulty accomplishing the prescribed maintenance action per MEL and contacted Maintenance Control. After repeated attempts to secure the fuel valves open, the left valves indicated fully open, but the right valve indicator indicated approximately 1/2 open. Maintenance Control assured us that this was a valid position and met the criteria for the MEL action required as it allowed the fuel in the outer wing tanks to migrate to the inner wing tank, which it did. We strenuously disagreed with this assessment as it was not consistent with the previous 3 legs that we had flown this day. He insisted it was in compliance and we departed. Climbing through 10,000 feet we reviewed the systems pages and discovered that the outer to inner fuel valves had closed and there was now about 200 pounds of fuel in the outer wing tanks. As the flight progressed we noted that the outer tanks were filling at an alarming rate. In about 30 minutes we had 3,000 pounds in the outer wing tanks and felt that if if this continued we would not have enough fuel in the inner tanks to reach [destination] if the valves did not open on schedule as the result of the maintenance action performed before departure. We contacted Dispatch and suggested a diversion to [a nearby alternate]. The Dispatcher concurred and we proceeded to [divert] and landed uneventfully. The maintenance crew that met us upon reviewing the maintenance log and speaking with the Mechanic that was assigned to our trip felt that the procedure that was applied [before departure] was incorrect. [They] produced a printed page from the maintenance manual and applied the procedure with positive results. An entry was made into our logbook reflecting this assessment. We boarded our passengers and continued uneventfully.

Synopsis

A320 Captain reported diverting to an alternate when their fuel system, which was on MEL restrictions, was not performing as expected.

ACN: 1562625 (1236 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Mixed
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Landing
Airspace.Class D : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1562625
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1562607
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Ground Excursion : Runway
Anomaly.Ground Event / Encounter : Ground Strike - Aircraft
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

On final [weather radar] was painting yellow at the end of the runway with red off the end. A "splattering" of green on and in front of and to the side of the runway. We could see centerline on final to the end on the runway. Very near touchdown, below 30 ft, we encountered a very strong gust of wind from the right side. At the same time, visibility dropped to very low. The airplane weather-vaned into the wind [and got] pushed the [to] right of the runway. I tried to keep the airplane on the runway by using rudder and as little ailerons as I could because I did not want to [get a] wing strike. I could no longer see the center line but I could tell we were still on pavement. The airplane floated just a bit due to the sudden added wind speed we touched down with a side load. Just before we touched down, the F/O (First Officer) said "we need to go left." I tried but the aircraft didn't respond right away. The FO said again "left, left" and then because he didn't feel me trying he [input] rudder as well. We got a dual input.

I then said "my controls" and the aircraft started responding and I was able to get it back to centerline. This all happened in a matter of seconds from the gust/ sudden loss of visibility to touchdown right of center line, side-loaded. Just as quickly it passed and visibility returned to what is was before. In the second it happened, I thought of a go-around and very quickly dismissed because of the thunderstorm at the end of the runway and our very low energy. I was worried about wind shear with the low energy and didn't think we could get enough altitude to turn before heavy rain at the departure end of the runway.

Exited the runway and taxied back to the terminals normally. When we reached the gate, local Maintenance was waiting on us. I asked them to the cockpit. I told them that we had a hard landing and a side-loaded landing. I told them I was going to write it up for that. I was in the process of writing it up and calling Dispatch and Maintenance Control when local [Maintenance] came back up and said there was damage and grass on the underbelly. I finished writing up the side-loading and then went out to take a look to see what I needed to write up. The left engine appeared to have ingested FOD (grass) and there was damage to the right engine lower outboard reverser door. I returned to the cockpit and wrote these up as well. Looking back on it, I don't think I should have written up a hard landing. I don't feel like it was a hard landing but it was definitely a side-loaded landing.

I might have been able to glance back inside and continued to use the localizer guidance to be able to tell how far off of center line the aircraft was for the few seconds that the visibility dropped.

Narrative: 2

[We] were on final [and] both noticed that there was some yellow painting on the radar at the very end of the runway, with red painting well off the end of the runway. The only thing that we actually painted on the runway was green.

On short final, at 500 feet AGL, we were able to see the centerline of the runway all the way to the end. The Tower stated the wind was 170 at 18 gust 27. There was rain present also. I never at any time saw any lightning. At approximately 10 feet off the runway, I placed the Captain's windshield wiper on low then followed by high, just to make sure that we had visibility through the rain. As soon as we entered the rain area, there was what appeared to be a strong gust and significant rain, and the plane's nose [weather-vaned] into the wind. At the point where I felt like we were going to touch down, the plane floated a bit, and moved to the right side of the runway. At which point, I stated, "left, left"..... I never heard my Captain's response, so I stated it again, "we need to go left, left" and at which point I did give some input to the left rudder, due to feeling like we might go off the runway. After stating the 2nd time of needing to go left, my Captain acknowledged me and we slowed enough to maneuver to the left and then back to center line.

At no point did I believe a go-around would be a good idea, due to the fact of having little to no energy for a safe go-around. This entire incident took a total of less than 4-8 seconds.

After getting back on centerline, I heard [another aircraft] 3 miles behind us, and advised them to NOT LAND, due to low visibility. They got a heading and broke off the visual approach. There was also [a second aircraft] and I advised the Tower to ask them to also, NOT LAND too due to the same reasons. They too, took a heading and landed later.

We taxied to the ramp without further incident. My Captain talked to Maintenance, wrote it up in the log book and also, did the post flight walk around.

Preventive measures would be to go around/break off the visual approach. However we didn't have anyone to attempt prior to us, which would have been nice, because we did not have any visibility reports prior to landing.

Synopsis

A320 flight crew reported a runway excursion in gusty wind conditions that resulted in damage to the belly and engine nacelle.

ACN: 1562615 (1237 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class C : ZZZ

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1562615
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Distraction

Person : 2

Reference : 2
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1562676
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after takeoff the flight deck was filled with smoke. We immediately put our oxygen mask on and stop the climb at 5000 feet. The lead Flight Attendant also reported smoke in the cabin. We decided to return back to the field. We notified ATC of our situation. We ran the ELIMINATION OF ODOR/FUMES IN FLIGHT DECK/CABIN. We landed back in ZZZ once cleared the runway we had the fire department walk around the aircraft. The fire department did not see anything unusual so afterwards we were given gate to taxi to and deplaned. Some passengers need medical assistance due to the heat and the cabin and the smoke.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 Flight Crew reported the flight deck and cabin filled with smoke shortly after takeoff causing a return to the departure airport.

ACN: 1562205 (1238 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : APU Pneumatic System & Ducting
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1562205
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

Just after rotation an acrid smell was detected followed shortly by unusual colored smoke/fumes mostly on the FO's side of the airplane. The FO immediately put on O2 mask because he didn't feel normal. On downwind, I called the FA's and number 1 reported that she didn't smell anything. We performed the QRC Smoke/Avionics smoke/fumes soon after detecting the acrid smell and this lessened the impact.

After gate arrival, I was informed by flight attendants they were experiencing nausea and headaches plus the FO wasn't feeling well either. Oddly enough, I'm not aware of any passengers requesting medical assistance. After deplaning, FA's and FO's condition got worse so we requested medical assistance from airport EMT's. I know the EMT's administered meds to FO as well a IV bag plus the EMT's evaluated all crew then they released us to go to hospital. Since event occurred on the weekend, there seemed to be some confusion on what local facility could see and treat the crew for their medical condition. Also, local operations couldn't provide transportation for entire crew to the hospital. This caused some delay in getting the crew to a local hospital for treatment. 

The flight [crew] was taken by ambulance to hospital and I'm not sure who took FA's to a different hospital. For some reason the FA's were taken to a different hospital than the flight [crew] even though I requested we all go to same facility. Thanks to local chief pilot for making things happen in regards to getting us to local hospital for immediate care because I eventually started experiencing a headache and facial flushing. I don't believe I was functioning at a high level and this point. I was informed a day later that there was a leak in the APU system allowing fluid to get into aircraft air conditioning system. ZZZ needs to have a protocol in place for medical events that occur when their normal medical facility is closed. It shouldn't be up to the crew to determine what is a appropriate medical facility to go to for treatment.

Synopsis

A320 Captain reported acrid smell and smoke after rotation resulted in a return to the departure airport and the crew being transported to hospitals.

ACN: 1562180 (1239 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1562180

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Immediately after starting both engines and re-configuring the bleed system to supply the PACKs air from their on-side engine bleeds, which itself followed a previous BTB for maintenance, the aft, B, flight attendant called the flight deck to report the aft, D, flight attendant vomiting in the presence of acrid/metallic/oily fumes. The fumes were reported to be noticeable from Row 16 aft. Two passengers also reported noticing the fumes as well as a passenger vomiting as well. It is unknown if [the passenger] perceived the fumes as well. We, the flight deck crew, did not.

Synopsis

A320 Captain reported complaints from the Flight Attendants of noxious fumes shortly after engine start.

ACN: 1561458 (1240 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Drinkable/Waste Water Syst
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1561458

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1561643

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While in cruise the aft flight attendants called the flight deck to report a possible faint electrical smell coming from the aft airplane left lavatory. All cockpit indications were normal. The non-flying pilot reviewed the QRH to look for an applicable procedure. The non-flying pilot reviewed the maintenance logbook for previous related write ups and found none. The non-flying pilot made frequent checks with the aft flight attendants every couple minutes to determine if the smell was getting stronger or dissipating and the flight attendants advised everything was the same, the faint smell was not improving or worsening. During the descent, the flight attendants called to report there was visible smoke coming from under the lavatory sink and they had switched off the water heater. Immediately, the non-flying pilot applied the SMOKE/FUMES/AVNCS Smoke Procedure, leading the pilots to turn off the COMM push button to deactivate the power to the lavatory and galleys, [and] continued to check with the aft flight attendants on the status of the smoke. They advised the smoke was not getting any worse. We continued to landing. At the gate, Maintenance determined the water heater to be faulty and placed the item on MEL. This was a faulty piece of equipment.

Narrative: 2

During cruise the aft flight attendants called to say that they smelled something unusual in the aft left lavatory. They said it smelled more like electrical than anything else. I asked if it was just in the lavatory or if it could be smelled anywhere else. I asked if they saw any smoke and if it appeared to be getting better or worse. They said it was strongest in the lavatory, but could slightly be smelled in the aft galley and they did not see any smoke. I asked them to monitor and to immediately let me know if it got worse. I called every few minutes to ask if it was better or worse. Each time they said that it was about the same and that they did not see any smoke. They reconfirmed that the smell was like electrical.

I sent a message to Dispatch via ACARS to let them know that Maintenance should be notified as we would want to have it checked out [upon arrival]. We referred to the QRH to be prepared to react as things developed. I called again and explained that if there was visible smoke that I would be turning the emergency exit lights on and turning off all power to the galleys, lavatories, etc. I tried to get them to be more descriptive on the smell and how strong it was so that I could determine if I should start removing power.

A few minutes later, during descent to land, they called back to say that they opened the panel under the sink and saw smoke. They turned off the power switch to the water heater and called me. I turned on the emergency exit lights and turned off the commercial button per the QRH. We were about 15 minutes from landing at this point, so the nearest suitable airport was [our destination]. I [advised ATC] and requested crash/fire/rescue to be notified. Throughout the whole process, the First Officer was flying and had control of the airplane. He did a great job and managed the flying and ATC while I was off com 1.

CFR was lined up and down the runway. As we cleared, we asked if they could see any smoke. They did not. We had already coordinated with ATC that if there was no visible smoke on the outside and no more smoke on the inside, that we would plan to taxi to the gate to deplane. The flight attendants reported that there was no more smoke witnessed. CFR followed us to the gate and 2 firefighters went to the lavatory as soon as the main door was opened. They found only residual smoke odor under the lavatory sink and no hot spots.

Synopsis

Air carrier flight crew reported smoke and fumes from the aft lavatory due to a faulty water heater.

ACN: 1560572 (1241 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 16000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1560572
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1560568
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Within moments of turning on the PACKS after takeoff both crew noticed a strong smell I would characterize as musty/moldy/body odor. As I pondered my next move the First Officer asked if I smelled that, followed shortly by I think that is the dirty sock smell. Her next comment of "I'm already feeling nauseated" broke my reverie and both crew members as well as our jump seat occupant donned O2 masks immediately. We attempted to perform the "Elimination of odor in the flight deck/cabin" procedure. We were hindered in our efforts by the fact that the binder of the first QRH employed broke in half when the First Officer attempted to retrieve it leaving her with nearly 200 loose pages of former QRH. I provided my QRH but the procedure is missing from the back, or anywhere, in this particular aircraft's QRH. Volume 2 was referenced for the procedure which was challenging to find given the constraints of the mask.

We leveled the aircraft at 16,000 feet and as the First Officer was already experiencing symptoms I elected to return the aircraft to [the departure airport] without further delay. ATC was notified of our intentions and I terminated the previously mentioned checklist that involved troubleshooting the air packs due to the difficulty in following a procedure through the mask on the Ipad and our need to prepare the aircraft for an air turn back. I never did feel I was able to get a real idea of which pack might be causing the odor. As we descended through 10,000 feet I turned off BOTH packs since I had been unable to determine the source of the odor and opened the ram air valve in an effort to provide as much fresh air to the cabin as possible. The First Officer ran the overweight landing checklist as we were approximately 3000 pounds above max landing weight. ATC vectored us for an ILS Approach which terminated in a successful landing. The aircraft was met by Fire and EMS as well as representatives from flight operations, inflight and local [authorities].

The aircraft was deplaned without further incident. The odor event, crew usage of O2 and overweight landing were documented in the logbook post flight. The flight and cabin crew debriefed with the aforementioned parties prior to departing the scene. Both the Airbus and [company] odor/fume reporting sheets were completed and delivered to maintenance. Both pilots and I believe all flight attendants reported to company provided medical care for evaluation afterwards. On further communication with flight attendants it appears at least some of them showed elevated levels of carbon monoxide in the blood tests their doctors performed. Even though the doctor was familiar with these ongoing odor events in the industry and was provided with [union] and IATA guidance for examination no blood testing of the pilots was completed. Perhaps the company [and other parties involved] can work towards a common strategy for post incident comprehensive physical examination of crew members to ensure all bases are covered.

Ensure that the Elimination of Odor in Flight Deck/Cabin checklist is added to EVERY QRH in the fleet ASAP. I believe our fleet manager has been working very hard to see this happen. Trying to use an Ipad to find critical checklists while wearing a mask is very difficult even without the presence of visibility restricting particulates in the air. Continue working on resolution to this seemingly complex issue as it appears to affect a large portion of the fleet including both Boeing and Airbus aircraft.

Narrative: 2

After takeoff, as we turned on the packs, I smelled an odor. I said, "it smells like device fluid." Then, within seconds, it got very strong and smelled to me like oil. I turned to the Captain and said, "I think this might be that dirty socks smell. Do you smell that?" I started feeling nauseated and told my Captain. We both immediately donned our oxygen masks. Our jumpseater did as well. I went to my QRH to verify the memory item was complete then to start the Elimination of odor in the flight deck checklist. The QRH binder busted open and I had pages everywhere. Then, I used Captain's to find out the procedure, which was not in this QRH. So, we went to Vol. 2 and decided to level off at 16,000 feet and return to [the departure airport].

I advised ATC we had fumes in the aircraft and we needed to return to [the airport]. The Captain gave me control of the aircraft and continued the checklist. We could not determine which pack the fumes were coming from. We turned off both packs through 10,000 feet and opened the Ram Air Valve. I then gave control back to the Captain and contacted company and ran the overweight checklist. We were vectored to land and landed. The fire department and EMS met the aircraft. We successfully deplaned aircraft. All paperwork was filled out and logbook entries made. Maintenance was there and received the odor reporting sheets. The entire flight crew went to get medical treatment evaluations. I personally felt very dizzy and nauseated for the first hour and it lessened as I got fresh air. This fume issue seems to be occurring to often and needs to be resolved. I think Maintenance is having a hard time duplicating the issues we are experiencing.

Synopsis

A320 flight crew reported odors in the cockpit requiring a return to the airport.

ACN: 1560570 (1242 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1560570
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Company Policy

Narrative: 1

At the conclusion of the flight, the Captain made me aware that a passenger sitting in seat 1A boarded the aircraft with a portable oxygen (O2) bottle. The passenger complained that his O2 was not working and the flight attendants administered Oxygen. The gate agent and supervisor noticed he had an oxygen tank with him when he was deplaning. The Captain called the duty manager for guidance. From the flight attendants we were made aware that the gate agent wanted to do a security check, which was completed. We do not know how he made it through security with this tank.

Synopsis

A320 Pilot reported being told, post flight, that a passenger had boarded the flight with an oxygen tank.

ACN: 1559748 (1243 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 988
ASRS Report Number.Accession Number : 1559748
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Type : 1245
ASRS Report Number.Accession Number : 1560148
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Initially, I was the pilot monitoring on this flight and the First Officer executed a normal landing after a stabilized approach. Upon decelerating through approximately 70-80 knots, we received a BRAKE SYSTEM 1 FAULT and NOSEWHEEL STEERING FAULT ECAM. As the Captain, I then took control of the aircraft. At the moment, I was still able to steer the airplane through the rudder pedals and we were decelerating normally. At that moment, the brakes were working. At approximately 30-40 knots and with a few thousand feet of runway still in front of us, we decided to recycle the ANTI SKID/NOSEWHEEL STEERING switch. This was unsuccessful as both ECAM fault messages remained. The tower then asked us if we could expedite clearing the runway. The First Officer replied that we had a brake and steering issue and they then sent the airplane behind us around. My main priority was stopping the airplane safely and not expediting with uncertain braking and steering on a high speed taxiway.

I was able to slow the airplane to approximately 10-15 knots when the tower asked us if we were able to clear the runway. I initially tried to do so and the airplane began turning where I wanted it to go, but then both the brakes and steering completely failed. At that moment, my main concern was not heading off the hard surface and into the grass. I used differential power to execute what was basically a 180 degree turn back towards the runway centerline. Once we had the direction under control and leaving the hard surface was no longer a threat, I called for the Loss of Braking, Immediate Action Items. They were carried out and we brought the airplane to a safe stop. At no point did we depart the hard surface and there was no damage to the aircraft nor to any ground lighting/equipment. We advised the tower of our situation and they closed the runway and sent the trucks out (CFR [Crash Fire Rescue]) to look us over. After the situation was stabilized, I briefed the flight attendants and then our passengers, explaining that we had experienced a steering malfunction and would have to be towed to the gate. We were towed back to the gate and the passengers were safely deplaned. I conducted a debriefing with the crew and then later with the [Chief Pilot] (on speaker phone with the First Officer and myself). Later, upon arrival at the hotel, I called [Maintenance Control] to close the loop. [Maintenance Control] advised that they had found the problem; a canon plug connecting the brake/steer by wire systems with the actual mechanical units had somehow come off, and thus would have prevented us from steering or braking the aircraft by the normal means.

Narrative: 2

[Report narrative contains no additional information.]

Synopsis

A320 flight crew reported a loss of braking and nosewheel steering.

ACN: 1559725 (1244 of 2908)

Time / Day

Date : 201807

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 37000

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZDV

Component

Aircraft Component : Flight Dynamics Navigation and Safety
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Type : 2383
ASRS Report Number.Accession Number : 1559725
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

During our flight to IAH, we discovered our Wi-Fi was inoperative. There were no forecasted areas of convective activity until the DFW-IAH area noted during preflight planning, so no real concern. As we approached the Rocky Mountains (near DEN) we were enveloped in an area of thunderstorms with tops well above our weight-limited ceiling of FL370, estimate tops between 40-45k, some higher. 

Prior [to this], nearing waypoint "YALES" (Northwest of DEN) we were trying to get info from our Dispatcher [regarding] a route to avoid weather. Our Dispatcher began discussing TCC (northeastern New Mexico), ABSOLUTELY USELESS INFORMATION, as we would have to traverse all of the WORST of the weather just to get to the New Mexico border! It seemed he was not paying attention to the explosion of weather near the Rockies (happens most days all summer long), directly ON our route. 

With no WSI [Pilotbrief] for a big picture, no help from [Dispatch] whatsoever, we had to use our Mk-1, grade-A, eyeballs along with our 60-80nm radar to try and find a way through the weather. We got lucky. 

The WSI picture is so invaluable that the Wi-Fi SHOULD be a NOGO item (if INOP, during convective we much like our we radar). With WSI, we can find a routing to avoid thunderstorm penetration (a really bad thing in an all electric jet) and provide a the expected product...fly from point A to point B safely, efficiently, and as close to on time as possible. Without WSI, we are looking out 80nms and praying. Not a good way to conduct our flights.

Synopsis

A320 Captain reported they had difficulty safely navigating through an area of thunderstorms because their Wi-Fi was inoperative, rendering their WSI system unusable.

ACN: 1558145 (1245 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1558145
Human Factors : Situational Awareness
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1558570
Human Factors : Situational Awareness
Human Factors : Time Pressure
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While nearing top of climb, flight attendants called the cockpit to complain of an odor emanating from the rear galley that was causing them concern. We performed the "Elimination Of Odors In The Cockpit/Cabin" checklist, but limited the procedure to the aft cabin/Pack 2 portion of the procedure due to the fact that both the Captain and myself could smell no odor, nor were suffering from any physical/physiological effects.

After completing the checklist items, we again established communication with the Flight Attendants and discovered that both the smell, as well as physical/physiological symptoms that they were experiencing, as well as the "Dirty Sock" smell persisted, we decided to LAND ASAP and ZZZ was selected (with concurrence from Dispatch) as the nearest suitable diversion airport.

After briefing the Flight Attendants and Passengers, we diverted to ZZZ and an overweight landing checklist was performed as we were approximately 8000-9000 LBS. over max landing weight. Tower and ground were advised to have EMS meet us at the gate to assist with Flight Attendant and Passenger medical evaluation as needed. Touchdown was smooth and later determined to be well within limits by maintenance. 

To my knowledge, only one passenger complained of feeling ill, but declined medical treatment; however, all four Flight Attendants complained of feeling nauseous, burning eyes, and dizziness. One flight attendant was observed vomiting in the rear lavatory. After all Flight Attendants were checked at the gate by medical personnel/EMS, they were transported to a nearby medical clinic where it was later learned they had all been diagnosed with Carbon Monoxide Poisoning-with severity ranging from mild to serious. 

All Flight Attendants as well as Captain performed admirably and professionally during this emergency. All crew members worked together to attain the best possible outcome from unfavorable circumstances that we were presented with yesterday and I have nothing but praise and accolades for our crew.

Callback: 1

The reporter stated that the APU was deferred prior to the flight; therefore, the APU was not operating. The reporter stated that a very strong "dirty sock" odor was only evident in the aft galley area and, it's as if there was a wall containing the odor only in the aft galley area. The reporter added that all the flight attendants required medical attention and it was determined that each had carbon monoxide poisoning. Only one passenger complained of the odor, but declined any medical attention added the reporter. 

The reporter stated that they turned off the right hand pack and it didn't seem to make a difference; however, they didn't have time to troubleshoot any further because they were on oxygen and were dealing with landing the aircraft as to not exacerbate the situation. The reporter concluded with stating that that they ferried the aircraft for maintenance, but did not use the packs so there was no odor at that time. Additionally, Maintenance said they did not find any discrepancy, but did a duct burn-out to try and clean the AC ducts.

Narrative: 2

We were greatly delayed because upon arriving at the aircraft, we found that the APU was deferred and no outside air conditioning was connected. The temperature was about 96 degrees on the aircraft and I told the agent we couldn't board until we cooled the aircraft. After connecting ground air and waiting several minutes, we couldn't get the temperature to drop below about 90 degrees on the aircraft. After contacting the chief pilot, we decided to board all the luggage and below wing items and then start an engine at the gate to cool the aircraft. Finally, we boarded and a short time later departed.

When we were within a few thousand feet of top of climb, I received a call from the flight attendants. They stated that they had an odor in the cabin that smelled like dirty socks and that they had been told in recurrent ground school to contact the pilots immediately if they had this issue.

We proceeded to find and run the appropriate fumes removal checklist and contacted the flight attendants after waiting an appropriate time for circulation of the cabin air. They stated that the smell did NOT go away and in fact, it might be getting worse. We decided to contact dispatch and divert to another airport. ZZZ appeared to be the best choice and nearest in point of time and after coordinating with dispatch we identified as an emergency and proceeded with haste to ZZZ. We had EMS meet the aircraft in case any passengers were reporting symptoms. The flight attendants had all been exposed and two were quite nauseous. They later were checked at the hospital and found positive for minor Carbon Monoxide poisoning.

Synopsis

A320 flight crew reported diverting after all four flight attendants noted a dirty sock smell in the aft galley area during climb.

ACN: 1557592 (1246 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Pitot-Static System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1557592
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the day prior to our aborted takeoff, we experience the following faults during takeoff:
AUTO FLIGHT A/THR OFF & AUTO RUDDER TRAVEL LIMITER SYS FAULT. During our subsequent push and after engine start, we again received an AUTO FLIGHT A/THR OFF ECAM (Electronic Centralized Aircraft Monitor). We called Maintenance Control, pulled back to the gate and deferred the Auto thrust system. We also deferred an F/CTL and FUEL Class II MSG. We pushed and taxied to the runway. During our takeoff attempt we did not get the expected FMAs (flight mode annunciators) after setting takeoff thrust. The backup speed system activated, changing the speed tape red and "SPD LMT" flashed red on our PFDs (primary flight displays). Numerous ECAMs were also triggered. We aborted the takeoff and returned to the gate. It was determined that the aircraft's pitot static system was clogged with insect debris.

The next day, we returned to the aircraft and prepared to depart. While waiting for pushback, another AUTO FLIGHT A/THR OFF was triggered. Maintenance Control was called and testing produced no faults even with the ECAM present. The Auto Thrust was again deferred using MEL 22-XX-XX.

We complied with the requirements of MEL 22-XX-XX. We then did a low speed abort, out of an abundance of caution and because of the numerous issues encountered earlier, when we did not receive a MAN FLEX or MAN TOGA FMA. Our thinking was that these would be displayed, as they are manual power settings and not autothrust settings.

While we knew we would not have an autothrust indication on the FMA. We thought that we would still have the MAN TOGA or MAN FLX FMAs. VOL II specifically makes a note that the RWY FMA may be absent, but does not mention the possible absence of MAN FLX or MAN TOGA FMAs.

I called Maintenance Control. Maintenance Control was not sure if the FMAs should be displayed and told us to return to the gate. I called the duty phone. The base Chief Pilot I spoke to did not know either. The mechanics on the aircraft were not sure and asked to speak to the MEL subject matter expert. I then called [another Captain]. [The] Captain informed me that we would not get the FMAs with an inoperative A/THR system. He also took the time, and I am grateful that he did, to explain some other pitfalls that a pilot can expect with an inoperative A/THR system.

The mechanics also included in their corrective action that the MEL was unclear and stated that it needed to be addressed. During climbout we received an ENG GA SOFT FAULT. After landing Maintenance Control did not believe this to be associated with our A/THR deferral and asked me to enter it into the logbook for tracking and crew awareness.

In Summary, our takeoff was aborted after numerous issues were experienced during our previous two flights. This including a previously aborted takeoff due to an ADIRU (Air Data Inertial Reference Unit) 1+2+3 miscompare. This resulted in the loss of all airspeed information and numerous ECAM warnings during takeoff. Two pitot tubes had to be removed and replaced. This abort was also due to unclear guidance contained in MEL 23-XX-XX.

Synopsis

A320 Captain reported that an aborted takeoff.

ACN: 1557586 (1247 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pitot-Static System
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
ASRS Report Number.Accession Number : 1557586
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft X, after numerous write-ups, required a replacement of its NWS (Nose Wheel Steering) actuator. The aircraft sat on the ramp for four days prior to our flight as it underwent repairs and tire changes. During our takeoff, on our first-leg, the aircraft triggered the following ECAMs (Electronic Centralized Aircraft Monitor): AUTO FLT-A/THR OFF and AUTO FLT-RUDDER TRAVEL LIM SYS. The aircraft was above 100 KIAS and accelerating to V1. I elected to continue the takeoff per the guidance provided by both the aircraft manufacturer and contained within Frontier's Volume II manual. Once airborne, we were able to re-engage the AutoThrust and both ECAMs disappeared. We were unable to recall either alert. The aircraft then downgraded itself to CAT III INOP. Noting no deficiencies to the safety of flight, we continued as scheduled and landed uneventfully.

Our scheduled landing time was XA:57. We landed around XA:40 [another airline] is scheduled to occupy our gate until XB:00. The station has reported that they do not have a working radio. The station does not answer at the phone number provided. We are unable to ACARS the station directly due to the aircrafts AOC (Airline Operational Control) menu reversion after landing. Thankfully, we did not have a medical emergency while we sat and waited for [the other aircraft] to push at approximately XB:10 as we have no way to contact the station. 

With the aircraft stopped while waiting for the gate, in an effort to minimize any delays to our airline, we called Maintenance Control and notified them of the above described abnormalities. We were asked to call them back with the exact logbook write-up. I enter the following into the aircrafts logbook and reported this to Maintenance Control as requested:

"During takeoff, above 100 KIAS, ECAM: AutoThrust OFF and Rudder Travel Limiter Faults were observed/triggered. T/Off continued. A/THR engaged airborne with no further abnormalities noted to A/THR or Rudder Travel Limiter. AC status downgraded to CAT III/CAT I ONLY. This too, however, corrected itself after landing."

We also reported the following two Class II messages and entered them into the logbook: F/CTL and FUEL. 

Line Maintenance ran a BITE (Built In Test Equipment) test, deferred our Class II messages and we were cleared to go. We pushed from the gate, but after our engine start, another A/THR ECAM was triggered. We contacted Maintenance Control and were told to return to the gate to defer the autothrust. The autothrust was deferred per MEL 22-XX-XX with no further trouble shooting and/or testing.

After engine start we complied with MEL 22-XX-XX, which required us to press and hold the autothrust's instinctive disconnect until we received a status message noting the A/THR INOP. It should also be noted that, with this MEL and the deactivation of the A/THR, ALPHA FLOOR protections are not available. 

We taxied to the runway. During our brief we had decided that, in the event of further F/CTL ECAMs, we would most likely discontinue the takeoff. Once cleared for takeoff, the thrust levers were moved to the FLEX decent after engine stabilization at 50% N1. We did not receive a corresponding FLEX FMA (Flight mode Annunciator) on our PFD (Primary Flight Display). I increased the power to TOGA (Take-off/Go Around). Again, we did not receive the expected and corresponding MAN TOGA FMA. My attention was momentarily diverted to the speed tape and a flashing red "SPD LMT" in the lower, left corner of the PFD. The speed tape was no longer displayed and was solid red. I believe this was an auto activation of the backup speed scale. At this point multiple ECAMs were triggered. This all happened very quickly, probably during the span of 5 seconds. The takeoff was successful aborted and the aircraft stopped on the runway with no lateral deviation from the centerline. We returned to the gate and offloaded the passengers. The flight was subsequently canceled. A mis-compare of ADIRU 1+2+3 was recorded by the aircraft along with a FAC 1 fault. It was discovered that both the alternate and the Number 2 pitot-static system was completely clogged with insect debris. 

We are fortunate that this occurred during the takeoff phase. Otherwise we could have been faced with the loss of all air data, the use of the backup speed scale, with no auto thrust and no ALPHA floor protections. In hindsight, with our previously reported abnormalities during takeoff, the deferral of two class II messages, including a F/CTL message, the deferral of the autothrust system, some troubleshooting might have been warranted and well worth the impact to the schedule that the company was trying so hard to protect. I believe that we had a total of 7 MELs on the aircraft during the time of departure.

Synopsis

A320 Captain reported that during takeoff roll they received multiple ECAM messages associated with the Air Data Computer.

ACN: 1557458 (1248 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 22500
ASRS Report Number.Accession Number : 1557458
Human Factors : Time Pressure
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1557189
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff, we raised the landing gear and received an ECAM warning for HYD G ENG 1 PUMP LO PR (loss of G system hydraulic pressure).

We were handed off from tower to departure control and as Captain, I [advised ATC] with departure control and requested delaying vectors to remain in the area while we accomplished emergency checklist procedures. We advised departure that we would be returning. As a safety precaution, we requested Fire and Rescue to be standing by for the landing.

Two flight attendants called the flight deck to report unusual noises and a smell. I told them I was aware of the problem and that we would be returning to [our departure airport]. I also advised them we were addressing the issue and that I would get back with them in a few minutes.

We completed all ECAM actions and QRH follow-up procedures. We sent an ACARS divert message to dispatch advising of our return.

I briefed the Flight Attendants of the problem. I told them that we would be returning to [our departure airport] and to prepare the cabin for a normal landing. I told them I would also do a PA to the passengers. I briefed the passengers and told them we would be returning to [our departure airport] and making a normal landing, not to worry and to follow the instructions of our flight attendants.

We advised the controllers that we might not be able to clear the runway due to a loss of nose-wheel steering. I called Operations to advise them of our situation and the fact that we would require being tugged to a gate once we landed due to a loss of the nose-wheel steering.

We flew an uneventful approach and accomplished a normal landing. We were able to exit the runway at the high speed taxi-way exit.

Fire and Rescue examined the aircraft and reported leaking hydraulic fluid, the right main gear door being extended and no other apparent damage to the aircraft. Operations personnel reported to the aircraft, pinned the landing gear and then tugged the aircraft to the gate.

Upon arrival to the gate, I entered an aircraft maintenance logbook write-up for an overweight landing and for the loss of the G Hydraulic System. I spoke briefly with Maintenance personnel to answer any questions they might have. I also contacted Dispatch to insure they were aware of the situation.

Narrative: 2

After takeoff the first indication of the problem was a nose gear slow to retract. After which a series of ECAMs displayed. It was evident pretty quickly that we had lost most, if not all, of our green system hydraulic fluid. [We advised ATC] and ECAM/QRH procedures were followed. Also at this time, the flight attendants informed us of a strong odor/fumes in the cabin. During the process of completing the checklists the Yellow hydraulic system began show an overheat condition. This led us to redirect our priorities more toward a quicker return to [our departure airport]. We landed overweight without nose wheel steering. [Fire and Rescue] inspected the airplane and found a significant amount of hydraulic fluid leaking, but. No other concerns. We were towed to the gate.

Synopsis

A320 flight crew reported that the nose gear was slow to retract and a series of ECAM messages appeared indicating most of the green hydraulic system was lost.

ACN: 1556892 (1249 of 2908)

Time / Day

Date : 201807
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Weather Radar
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1556892

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1556889

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

While descending into [destination] our radar failed for the third time. We were visually deviating to avoid build ups and Center requested for us to turn left for traffic. We were not able because of weather. Center allowed us to stay on our heading and descend to 8,000 feet MSL. He also requested we keep our speed up and slow to 250 knots indicated not at 10,000 feet MSL, but 9,000 feet MSL. We complied with his request so we could continue deviating and avoid the traffic. 

The situation could not be avoided due to the failure of the weather radar system.

Narrative: 2

On the way to [destination] we had a Weather Radar Fail. It came back and went out again. We messaged Dispatch of the issue and informed them Maintenance would be needed on arrival. On descent it came back and about 15,000 feet ATC asked us to expedite down to 10,000 feet. Doing so we increased speed on the way down. ATC asked us to make left turn and we had huge cell. Looking at the radar we had a solid Red Cell with large amount of purple (severe weather). We then got an automated "Weather Ahead" verbal warning before the radar failed again. ATC then told us to go down to 8,000 feet. I told them we are slowing, then down to 8,000 feet. ATC responded saying they need altitude over speed and to go to 9,000 feet and then slow to 250 knots. We complied and slowed before continuing to 8,000 feet. During the descent I noticed we did clear converging traffic on TCAS by 1,000 feet vertically at the 10 mile mark (assuming the reason for the expedited descent).

Based on the failure of the weather radar and ATC situational awareness, nothing could be done on the flight crew part.

Synopsis

A320 flight crew reported their weather radar failed and recovered multiple times enroute.

ACN: 1555234 (1250 of 2908)

Time / Day

Date : 201806

Place

Locale Reference.Airport : SJU.Airport
State Reference : PR
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SJU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Vectors
Route In Use.SID : ACONY4
Airspace.Class C : SJU

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SJU
Make Model Name : Israel Aircraft Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Climb
Route In Use : None
Airspace.Class C : SJU

Person : 1

Reference : 1
Location Of Person.Facility : SJU.Tower
Reporter Organization : Government
Function.Air Traffic Control : Ground
Function.Air Traffic Control : Flight Data / Clearance Delivery
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1555234
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Facility : SJU.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1555551
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Local Controller departed Aircraft X from Runway 08. My positions: FD/CD (Flight Data/Clearance Delivery) combined with Ground Control and CIC (Controller-in-Charge). 

Seconds after, Aircraft Y called SJU Tower and transmitted "Aircraft Y is SQUAWK [code] runway heading". I attempted to Multifunction display squawk code, and did not find any FP (flight plan) in the ASR-8 system, and relayed to Local. It was obvious Aircraft Y had departed from Runway 09 at Isla Grande Airport -SIG, (adjacent airport about 6 miles west of SJU Tower), due to a visible primary target squawking [their code] in the departure end of SIG Runway 09 climbing out of 1,000. After XA:00L SIG Tower class D reverts to Class E, CTAF. 

Local Controller responded "Aircraft Y do not fly runway heading, say intentions, I have a departure." Aircraft Y responded "we request radar vectors please." By this time Aircraft X has already started takeoff roll. Local Controller asked Aircraft Y if he had SJU Tower in sight, and he replied yes. LC (Local Controller) then instructed Aircraft Y to fly direct SJU Tower and continue eastbound between the runways on a heading of 095. LC then asked Aircraft Y to verify call sign. Aircraft Y responded with his squawk code. Aircraft Y did not comply with any instruction and continued to fly eastbound still climbing. LC directed Aircraft Y "proceed over head the control tower now." Aircraft Y continued flying eastbound climbing out of 3,500 feet. 

Aircraft X was then instructed to maintain 3,000, and LC coordinated an altitude of VFR at or above 5,000 for Aircraft Y. Aircraft Y was given a brasher warning/possible pilot deviation and switched to San Juan Approach 119.4 frequency. Aircraft Y is clearly unfamiliar with SJU class C airspace, and additionally does not fully comprehend the English language to the extent necessary to responsibly fulfill his duties as a pilot in command. Additionally, Aircraft Y departed VFR, but checked in with a squawk code which also caused confusion on whether or not he was an IFR or VFR aircraft.

My recommendation would be to validate the qualifications of Aircraft Y and assure the pilot is aware of airspace and regulations. In my opinion, any control instruction given to Aircraft Y would have been unsuccessful due to his lack of the English language and situational awareness. In this case LC reached out to Aircraft X to achieve separation and was effective.

Narrative: 2

At the time of the incident, I had cleared Aircraft X, an A320, for departure on the ACONY1 departure which turns him northwest bound with a clearance to climb to 5,000. When I scanned the radar and outside I noticed an unidentified aircraft and there he called and reported he was on a runway heading and squawking [their code]. I instructed him to not fly runway heading from SIG due to the fact that I had the departure. I asked say intentions, he requested vectors. I asked for call sign and asked if he had the San Juan Tower insight, trying to take him out of the Aircraft X route of flight. Aircraft Y was not complying with any instructions. When asked for his call sign he replied his squawk. When told to fly over head the Control Tower he kept flying eastbound. This pilot action convinced me he was understanding my instructions. I took action fast and stopped Aircraft X at the safe altitude of 3,000, Aircraft Y was at this point at 3,500. Aircraft Y was then observed descending so I coordinated with R5 and instructed him to maintain VFR at or above 5,000 to keep him away of Aircraft X. I climbed Aircraft X to 5,000 feet (his initial climb clearance) once the Aircraft Y was no factor with him. Separation wasn't lost at any point. 

Validate qualifications of Aircraft Y to assure airspace awareness, language and communication skills.

Synopsis

SJU Controllers reported WW4 pilot with language barrier failed to comply with clearances resulting in a traffic conflict.