ACN: 1504254 (1351 of 2908)

Time / Day

Date : 201712
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Microphone
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5742
Experience.Flight Crew.Type : 4660
ASRS Report Number.Accession Number : 1504254
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Enroute, I sent a Maintenance request code stating that the (Captain's) Audio Control Panel 1 was inoperative and that transmissions from the Captain's boom microphone over VHR radios 1 and 2 were being reported as scratchy and unreadable over two separate headsets.

After landing, Local Maintenance met our aircraft and according to the subsequent maintenance release document, the technician removed and replaced the Number 1 Radio Management Panel (RMP). Prior to our scheduled departure, I checked the function and quality of radio transmissions from the Captain's station with Clearance Delivery and Company Hub maintenance and was informed that the transmissions were still scratchy and unreadable. In my radio communication with Hub maintenance, the controller directed me to re-enter the discrepancy via ACARS and said that he would send the technicians back to the cockpit to address the problem.

When the technicians returned to the cockpit, I explained that the original problem we were having with the radios was not corrected and asked for further help. We reviewed all action taken to this point in order to understand where the problem might be and the technician suggested exchanging headsets with the co-pilot station. After doing this, transmissions to Clearance Delivery and Hub Maintenance were described as satisfactory. In the light of the fact that I had earlier tried two headsets with no effect, I requested that our spare headset be replaced. The technician agreed and left the cockpit to obtain a replacement. He subsequently returned with a new, "still in the box" headset from stock. At that point I thanked the technicians and we closed the main cabin door explaining to the agent and purser that we would wait for the new Maintenance Release to be delivered over the cockpit printer before starting our pushback.

After receiving the Maintenance Release over the cockpit printer, I reviewed it and noted that the action addressing my last defect write-up made no mention of the fact that we had been directed by the technicians to exchange headsets between stations or that maintenance had replaced the presumably defective spare unit carried in the cockpit. Instead, the technician stated in the Maintenance Release that the only "Action" taken to resolve the "Defect" was "CLR-ACP OP CHECK NORMAL"

In my experience, anytime a headset is exchanged it is so noted in the corrective action section of the Maintenance Release. In my review of the Maintenance Release, I noted that the sign off for the last discrepancy did not describe the maintenance actions taken to resolve the discrepancy. Considering that the aircraft cabin door was closed and that we were beyond our scheduled departure time, I elected to overlook the disparity rationalizing that we had a valid Maintenance Release and should be legal to dispatch. We released brakes and pushed back for departure.

Enroute we received a message via the ACARS requesting further information concerning our delayed departure. The request stated that the delay had been coded "(FLT OPS CALL TECH OPS)." I responded to the message with a miscellaneous message: "GR - ZZZLM REQD TO RETURN TO ACFT TO FINISH REPAIR OF DISCREPANCY NO. X."

In retrospect, it appears that the technician signing off the final work accomplished concerning Defect No. Y obfuscated the maintenance action taken so as to avoid having the delayed departure assigned to Maintenance. If this is the case then I am concerned it represents a falsification of the aircraft maintenance record by omission. 

All departments concerned with the on-time departure of our flights are under substantial pressure to avoid any delay in those departures. This has been made clear to me in discussions with personnel from various departments at planeside as well as our flight office. That pressure was evidenced in this situation when almost immediately upon entering the cockpit to address our original maintenance discrepancies the two reporting maintenance technicians began talking about what they would and would not take a delay for with respect to the defects noted in our maintenance log. Their preoccupation was avoiding the consequence of a maintenance delay as much or more than the proper repair of the noted discrepancies. Misplaced priorities like these are a direct threat to the continued safe operation of our aircraft.

Synopsis

A320 Captain reported a log book entry was incorrectly signed off causing a delay assigned to the flight crew instead of maintenance.

ACN: 1503082 (1352 of 2908)

Time / Day

Date : 201712
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1503082
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Police / Security Involved
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On the STAR, we were called by the flight attendants about 10 minutes prior to descent regarding a disturbance in the cabin. A female passenger had claimed a male passenger had assaulted her and would not stop touching her. They did not report injuries but requested the police to be at the gate upon arrival. We tried immediately after the call to reach Operations with no answer.

At some point during this, ATC issued descend and maintain clearance to FL220. After initiating the descent, the captain resumed attempting to contact Operations. While on the radio with Operations, ATC issued another descend clearance to 17,000 with an expedite descent. I set open descent and read back the clearance. Just prior to returning from speaking with Operations, ATC issued another clearance, this time descend via the STAR. I input 3000 and continued the descent. 

The captain returned to state he had reached Operations and the police would be present. Shortly after this, ATC instructed us to maintain 13000. I realized we were below the crossing restriction and initiated a climb to 13000. We were in VMC conditions and ATC verified we had returned to descend via clearance and climbed to 13000. No further communication occurred and we thanked ATC for their help.

The combination of flight attendants calling, multiple altitude clearances and a brief expedite clearance caused me to forget to set managed descent from open descent upon receiving the descend via clearance. I knew immediately when queried by ATC what had occurred. 

This was a combination of multiple distractions and varied clearances. Diligence when one pilot is handling a problem outside of normal flight is required. Perhaps once established on an arrival after initial descent expedited descents shouldn't be issued unless for immediate traffic avoidance.

Synopsis

A320 first officer reported that a disturbance in the cabin caused a distraction that resulted in a deviation from the descent profile clearance.

ACN: 1502726 (1353 of 2908)

Time / Day

Date : 201712
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8188
ASRS Report Number.Accession Number : 1502726
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 2061
Experience.Flight Crew.Type : 1044
ASRS Report Number.Accession Number : 1502747
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Terrain Warning
Detector.Automation : Air Traffic Control
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Three legs and long duty day, [and had] not flown into [this] Airport for quite some time. FO (First Officer) and I talked in depth about the night arrival/landing and what approach to expect. We knew that we might not make it in time to land on 29R (closing) due to our arrival time.

As we got closer to [the Airport], it looked like we would land before 29R closed. FO thoroughly briefed 29R, including terrain, engine failure procedure, landing data, etc. We also looked at an alternative runway and discussed that scenario. [We] had some trouble getting landing data from ACARS for that runway and sent a message to Dispatch to see if they could send us the data. We eventually were successful in retrieving landing data for the alternative runway. We checked in with Approach around 13,000 feet and were told to expect the visual for runway 11L, and given a further descent to 11,000. 

The FO briefed the approach for 11L, altitude for the FAF, landing data, engine out, runway turnoff, etc. As we were discussing the approach, we were cleared to 6000. We were being vectored for a left downwind and as we were approaching abeam the field, I mentioned to the FO that I had [both a satellite airport] and [the destination Airport] in sight. Shortly after I said that, ATC gave us a further descent to 6000, pointed out the location of [the destination Airport] and asked if we had it in sight. I said yes, and ATC cleared us for the visual 11L. 

We were still well above and descending to 6000. Thinking that we would start our base turn soon after I called the field in sight, I put the altitude for [the] FAF (4600 feet), in the altitude window and called tower, left downwind for the visual 11L. Tower said, cleared to land. Still at 250 and feeling high, the FO extended the downwind a bit further before turning base towards 8 NM. A few seconds later, we turned base towards 8 NM, but was probably more towards 10 NM. I had extended the FAF, thinking we would still turn base inside 8 NM. As I was looking towards the runway, the FO called out 5600 for 4600 (altitude for the FAF). 

We armed the Approach Mode and continued on the base leg. I believe around 5000 feet and almost simultaneously, we got "Terrain Terrain" and a call from Tower, "Low Altitude alert, minimum vectoring altitudeâ€¦" I immediately moved my hands towards the flight controls; however, the FO was already disconnecting the autopilot, pulling back on the stick and adding power. The warning stopped almost immediately. I told the tower we were correcting and he cleared us to land again as were we are turning final. As we rolled final, we were now high. The FO began to configure with gear and flaps and corrected towards glideslope. Uneventful landing. We thoroughly debrief the event.

Of note... our Jeppesen FliteDeck-Pro were in night mode and dimmed, which makes the terrain display on the 11L Approach Chart, almost invisible. After we parked at the gate, I placed my IPAD in daytime mode and immediately saw the terrain. In night mode, the ILS feather and darkened screen makes it extremely hard to see the terrain.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

An A320 flight crew reported that on final approach they extended the downwind and continued on the base leg when they got a "Low Altitude" alert.

ACN: 1501240 (1354 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Route In Use : Visual Approach
Airspace.Class E : I90

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 1800
ASRS Report Number.Accession Number : 1501240
Human Factors : Situational Awareness
Human Factors : Distraction
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Airspace Structure

Narrative: 1

We were being vectored on a left downwind for a Visual Approach to Runway 27 at IAH. Aircraft were being turned on to about a 25 mile final. Aircraft were also given a descent to 2000 feet and told that they would be outside the Class B airspace. I am concerned because this area has a lot of VFR traffic, many of whom do not talk to Approach Control and there is no way to guarantee that they have an operable transponder that is turned on and accurate. Even when the "heavies" reach the Class B ring they are at the very bottom of protected airspace. Most VFR pilots in this area feel they are safe from airliners if they remain outside (below) Class B airspace. Descending airliners into a busy General Aviation flying area, one that includes a practice area for student pilots, is a very hazardous practice.

Synopsis

A320 Captain reported Approach Control assigned an altitude below the Class B airspace into an area known as a VFR student pilot practice area.

ACN: 1500841 (1355 of 2908)

Time / Day

Date : 201712
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ
Airspace.Class E : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1500841
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1500956
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On let down we got an ECAM: WHEEL - Nosewheel (NW) STRG FAULT. I was the flying pilot. First Officer confirmed that it was not an Exception or an Immediate Action item. Transfer of control of aircraft and radios to First Officer.

I ran the ECAM procedures and follow-up items. We then called operations and spoke with local maintenance. Under direction we cycled the A/SKID & NW STRG switch, with no change in condition. Desiring further time, we asked Approach for vectors while we addressed an ECAM.

Follow-up items cautioned that upon gear selection and another cycle of the A/SKID & NW STRG switch, to look for an additional ECAM of L/G SHOCK ABSORBER FAULT would possibly indicate a nose wheel that was not centered.

We [notified ATC of the situation.] We then called the flight attendants and gave them a briefing with a precautionary landing followed by a tow to the gate.

We asked the station to notify the company and to prepare for a tow-in after exiting the runway. Last I made a PA that gave the nature of the problem and that we would simply need to be towed to the gate.

We transferred control of the aircraft back to me, and the First Officer took control of the radios. We had already briefed the approach and landing. We further briefed the new condition to include the lack of nose wheel steering and further a nose wheel not centered. We also adjusted from a flap 3 to flaps full configuration.

With the aircraft fully configured, we again cycled the A/SKID & NW STRG switch. No change in condition, and more importantly no second ECAM as described above. We ran through a Emergency Landing Checklist.

We accomplished a normal landing with flaps full, LO brakes (transitioning to manual brakes). Vref of 134. Applying manual brakes and full reverse, we were able to slow without trouble. We used differential braking to exit and came to a complete stop on taxiway Papa. 

With the aircraft at a full stop, I made the PA to remain seated. Further explained to the passengers the requirement of emergency vehicles & personnel. Company ground crew and [maintenance] were there to hook up and tow us pretty quickly.

PA's made to the passengers as to the progress to the gate and to thank them for their compliance with the flight attendant instructions.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported that on descent they got a nose wheel steering fault message on ECAM.

ACN: 1500114 (1356 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 800

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Fog
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use : Direct
Route In Use.SID : ZZZ
Airspace.Class E : ZZZ

Component

Aircraft Component : Navigation Database
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6880
Experience.Flight Crew.Type : 5420
ASRS Report Number.Accession Number : 1500114
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 1562
ASRS Report Number.Accession Number : 1500129
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Primary Problem : Aircraft

Narrative: 1

We arrived at the airport [45 minutes before] departure. Weather was 002 OVC, calm winds, with fog. Our departure would be night, IMC. ZZZ is surrounded by significant terrain hazards (see Ops Alert ZZZ Terrain Hazards). For best terrain avoidance, we requested to depart runway XX with the ZZZ SID. FMGC was programed with runway XX, ZZZ, direct ZZZZZ. Both pilots checked the route, and agreed that it was correct. We also built the engine failure procedure in the fix page. Preflight briefing included a detailed discussion of the T procedure and terrain avoidance. The [SID] departure on runway XX requires a runway heading to intercept the 155 radial inbound to ZZZ (VOR). Taxied out and took off on runway XX. At about 800 feet AGL the flight director commanded a left turn. FO was flying and initially turned to the left and then said, "this isn't right, we should be going to ZZZ (VOR)", and rolled out of bank. The FMGC showed ZZZ (VOR) as the "from" point, and ZZZZZ as the "to" point even though we were approximately 5 miles south of ZZZ (VOR). I programmed the FMGC to proceed direct to ZZZ (VOR). Center called and asked if we were on the SID, and I replied that we were direct ZZZ (VOR). Climbing through 7000 feet we were cleared direct ZZZZZ.

The reason for this report is that I am concerned that there may be an anomaly in the FMGC database for this airfield (this aircraft is [from a foreign air carrier]). I am virtually certain that we programmed the FMGC correctly. The FMGC directed us to turn miles prior to the first fix on the departure in mountainous terrain in IMC conditions, and I am not certain that we would have had terrain clearance if we followed the Flight director. Quick recognition from FO kept us from following the erroneous flight director commands. 

I am hoping that someone "a lot smarter than me" could look into the data base, and possibly fly this departure in the simulator. It may save some other crew from making a hard left turn into terrain during night IMC conditions at ZZZ.

Callback: 1

The reporter stated generally the FMC is dead on with very little variation. The reporter stated that in IMC conditions they wanted to avoid terrain so they chose to use the TAKEOFF MINIMUMS, (OBSTACLE) DEPARTURE PROCEDURES. The reporter stated that after takeoff the FMC wanted to take them in the opposite direction from the procedures so they reprogrammed the FMC to take them to a waypoint in the opposite direction to avoid terrain and proceeded without further disruption. The reporter also stated that he feels this may only be an anomaly for this airport and would like to fly this procedure in the simulator to ensure this procedure is programed correctly. The reporter also stated he would not like to see someone else fly this in IMC conditions, not knowing the area, and getting in terrain trouble. The reporter stated that very often if someone is taking off North and heading South, the FMC is programed to take the shortest route without regards to terrain to take you to your destination.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported the FMC directed the aircraft's autopilot to turn in the wrong direction.

ACN: 1499919 (1357 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1499919

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise at FL350 we experienced a faint acrid smell that we both thought smelled like a possible electrical fire. At about the same time the A Flight Attendant called to report a strange smell in the mid cabin. I asked the flight attendant to let me know if it got better or worse. The smell was getting stronger in the cockpit causing the First Officer and I to don our masks, [advise ATC of the situation], and request direct ZZZ (the closest large airport).

About this time we received a BLOWER FAULT. We ran the ECAM, but the fumes continued to stay strong, basically unchanged. We were given direct ZZZ and lower. I asked my First Officer to run the SMOKE/FUMES/AVIONICS checklist which he did while I expedited the decent into ZZZ and sent dispatch a message advising them of our situation. My First Officer finished up the checklist, advised the flight attendants we were diverting to ZZZ and that this was a cabin advisory only and that we did not anticipate an evacuation unless circumstances changed. I asked my First Officer to run the performance numbers into ZZZ which he did. We needed to perform a couple of S turns to lose the last little bit of altitude and we landed in ZZZ without incident.

The fire and rescue reported no visible smoke from the airplane, they followed us to the gate and we deplaned at the gate. I wanted the passengers off the airplane and not exposed to the fumes that we were still smelling very strongly in the cockpit. I asked the flight attendants to open all the doors in order to air out the airplane. We already had the cockpit windows open. The whole thing took about 20 minutes from the first awareness of fumes to landing in ZZZ.

After things calmed down I was going over the fuel load for the next leg and noticed that we still had roughly 18,000 pounds of fuel onboard. I reran the landing numbers and realized we were overweight on landing. I immediately let Maintenance know and put it in the logbook.

I think the default landing fuel should be left blank until the pilot inputs the landing fuel. Often First Officers don't ever change this number due to the fact that you usually land with less fuel than planned. As we all know this is not always the case, but the habit is formed none the less. Leaving it blank would insure an accurate landing weight with regard to fuel load.

Synopsis

A320 Captain reported that while in cruise they experienced a faint acrid smell that smelled like a possible electrical fire.

ACN: 1499211 (1358 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1499211
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1499652
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Captain and FO started and continued takeoff roll without the use of flight directors and auto thrust. FO noticed at approximately 80 KIAS that the FMA had no information in it. FO cross checked the Captain FMA and noted a lack of information as well. The speed bug was set to 100 Knots and approaching 100 KIAS FO stated V1 (where the speed bug was set). The improper V1 call was immediately recognized by the Captain. The proper V1 and Vr information was displayed, called out, and rotation and lift-off happened at the appropriate time. Once safely airborne FO and Captain reselected their respective FD button and guidance returned within 5 to 10 seconds. FO attempted to restore autothrust by selecting the appropriate button and pushing speed to enter the "managed" speed mode. FO and Captain recognized that the aircraft was not accelerating on the appropriate schedule and noted 100 KNOTS was still in the airspeed box with a "dot" next to the airspeed. The Captain then spun the airspeed to 250 and re-selected "managed" mode. At this point the aircraft was in the normal flight director and autothrust.

Captain and FO suspect that the flight directors and auto thrust kicked off at some point prior to 60 KIAS on the takeoff roll. While it is certainly possible that both flight directors and autothrust kicked off, it was not associated with any ECAM. It is possible that an ECAM message was inhibited based upon the critical phase of flight, it is much more likely that the flight directors were not selected on prior to take-off. Once the FO recognized that there was an issue with the FMA it took far too long to communicate the issue to the Captain, trap the error, and correct it. Depending on if/when the flight director turned off could point to poor flow/checklist adherence and poor automation management. The best way to avoid this situation in the future is to adhere to flows and checklists (depending on when the flight directors kicked off). Also, much better communication between FO and Captain. While the airworthiness and regime of flight were never in question FO CRM was poor at best. Adherence to briefed procedure to include anything non-standard seen by the FO should be plainly stated; the reason for this was a combination of poor SA regarding the level of automation being employed until approximately V1 and poor communication with the Captain while attempting to restore the appropriate level of automation for the regime of flight (take-off).

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported that they started and continued takeoff roll without the use of flight directors and autothrust.

ACN: 1497969 (1359 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : OAK.Airport
State Reference : CA
Relative Position.Angle.Radial : 116
Relative Position.Distance.Nautical Miles : 5
Altitude.MSL.Single Value : 3100

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : SFO

Aircraft : 2

ATC / Advisory.TRACON : NCT
Make Model Name : Skylane 182/RG Turbo Skylane/RG
Flight Plan : VFR
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7400
ASRS Report Number.Accession Number : 1497969
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : NCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1498626
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Miss Distance.Vertical : 400
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

NORCAL Departure advised us of traffic at 12 o'clock maintaining 3500 ft. We were on the CNDEL 3 RNAV departure. We crossed LEJAY below 2000 ft as instructed on the SID. Just after passing CNDEL waypoint we had the traffic advisory, we had the traffic in sight, and just before turning to the next waypoint PORTE, we had the resolution advisory "climb, increase climb". NORCAL instructed immediately to turn to a heading of 180. After that the system said "clear of conflict".

Narrative: 2

[A C182] was circling over downtown San Francisco VFR at 4,500 ft. An A320 departed OAK on the CNDEL departure. I issued traffic to [the C182] and he called the A320 in sight with five miles in between them. [The A320] also called [the C182] in sight. I instructed [the C182] to maintain visual separation from [the A320] and that I was going to have the A320 fly straight out until he got above him. [The C182] said he would maintain visual and then flew right into the A320's flight path. I let [the A320] start his SW bound turn on the CNDEL instead of flying straight ahead in order to get away from [the C182]. I then turned [the A320] even more to a 180 heading. [The A320] received a TCAS RA.

Synopsis

A320 Captain and the NCT Controller reported on an airborne conflict that occurred when the A320 departed OAK and a C182 flew into his departure path.

ACN: 1497866 (1360 of 2908)

Time / Day

Date : 201711

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : None
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : External Power
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1497866
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Maintenance
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

While troubleshooting the Aircraft left and Right window anti-ice and ECAM messages, I turned power off from the flight deck, external power button and battery 1 and 2, from inside cockpit. After 5 minutes, I turned power on and batteries on. After the airplane rebooted, my messages had cleared. All system were active, and I waited in flight deck, to see if window anti-ice messages returned. 

After approximately 5 minutes, I saw smoke at 12 o'clock, outside Aircraft. I disconnected external power, and ran outside to nose. I saw a small flame and thick smoke at external power cord to aircraft connection. I pulled power cord off the aircraft, and smoke abated. No fire. I inspected left forward and aft electronics and equipment bay, right electronics and equipment bay, and nose electronics and equipment bay. No further smoke noted. I immediately informed the supervisor and lead. Cause pointed to worn gate power cable, per engineering.

Synopsis

Airbus 320 Maintenance Technician reported that external power cord malfunctioned.

ACN: 1497660 (1361 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZID.ARTCC
State Reference : IN
Altitude.MSL.Single Value : 34300

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZID
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZID

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1497660

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1497773

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Penetrating a line of thunderstorms through a large gap at FL330; painting no radar returns; radar adjusted to -2.0 degrees down, at approximately 10 miles range a small purple dot return appeared on radar. It was representative of a line of cloud that topped no more than FL350. We could see ahead as we were VMC at this time and we turned approx 20 degrees left and avoided the purple return by 3-5 miles best I can guess. Upon entering the smooth looking cloud, we hit severe turbulence which lifted us from FL330 to 34,300 ft within seconds and placed the jet into a 40-degree left bank. As altitude was still increasing and airspeed as well, with the trend arrow going well into the red barber pole for an over speed situation, I took command of the jet, turned the autopilot off and pitched the nose down to arrest the ascent. As I did this airspeed now went to the negative nearly entering an alpha floor situation. I hand flew the jet until aircraft control was regained. I then turned on the autopilot and gave aircraft control back to the FO (First Officer). ATC then queried us on our altitude deviation. No limitations appear to have been exceeded and on checking with the flight attendants there were no issues or injuries in the cabin.

This was a one of a kind event in my [many] years of flying. All normal precautions were taken. Radar was being monitored and nothing unusual was being indicated until the small purple return, the size of a pen dot, appeared at about 10 miles away. We took action and missed the indicated return by 2-3 miles.

Narrative: 2

During our deviation, we clipped a "benign/smooth-looking" cloud which generated severe turbulence. We gained approximately 1300 ft in what we estimate to be 3 seconds and were put into approximately a 40-degree bank to the left.

Synopsis

A320 Flight Crew reported severe turbulence after clipping a benign looking cloud.

ACN: 1493966 (1362 of 2908)

Time / Day

Date : 201711
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Flight Phase : Initial Approach
Route In Use : Visual Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 4100
ASRS Report Number.Accession Number : 1493966
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

This was our 3rd leg together and my 3rd leg since completing Initial Operating Experience. The weather was VMC and the wind light. We flew the arrival to vectors to a visual approach to 25L, backed up by the ILS. The approach was steep and fast and resulted in a go-around at 500 FT AGL. 

At 1000 FT AGL, our vertical speed commanded in the FCU was steeper than -1,000 FPM but was immediately recognized and corrected. At approximately 650 FT AGL, in a descent to intercept the glideslope from above, with both autopilots engaged, as the aircraft intercepted the glideslope, the aircraft pitched aggressively towards the ground. As Pilot Flying, I do not feel that I initiated that pitch input. I immediately disconnected the autopilot and recovered the aircraft. The GPWS announced "terrain, pull up" and the Captain took the aircraft and initiated a go around.

Causation: We [had] accepted clearance for the visual approach while 90 degrees off runway heading, in a position too steep and fast for me to make a stable approach. It was clear to me at 1000 FT AGL that the approach was not stable nor salvageable, but we elected to continue to 500 FT AGL. I don't feel like my initial qualification training prepared me well for this dynamic real world of vectors to a visual approach.

At this time, I have 30 hours in the aircraft and 43 total hours of civil flying experience. After returning to [the Company] after an extended military leave, I have 30 total hours of flying experience in the last 2.5 years. While on visual approach, I still struggle with the automation to make the aircraft descend to glide slope and intercept the ILS glideslope from above - scenarios not covered in simulator training. 

We received a traffic call of a 737 to the parallel runway in the same radio call clearing us for a visual approach. Searching for that traffic further delayed initiation of our descent to glideslope. 

Please consider what caused the alarming pitch down of the aircraft at 650 FT AGL. If it was me, I'd like to know how to not do it again. If it wasn't me, that seems like a problem.

Synopsis

A First Officer piloting an Airbus 320 reported that the aircraft aggressively pitched down when capturing the glideslope from above.

ACN: 1491540 (1363 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb
Flight Phase : Cruise
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 270
ASRS Report Number.Accession Number : 1491540
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

The airplane came from the hangar...and I noticed the Captain's Standby Navigation (STBY NAV) switch was on (showing NAV). I deselected the switch back to normal. The RADNAV page seemed to confirm this. After starting both engines, we noticed we had a Status Message on the Status page saying "Inop System... CAT 2." We couldn't get rid of it by cycling various things, and wrote it up and called Maintenance. After several cycled CB's and switches on the Flight Control Panel to no effect, Maintenance deferred it, which took all Auto-lands out of the picture. Maintenance said they'd never seen just a CAT 2 Inoperative before, didn't really have good guidance for it in their manuals, and weren't exactly sure of the corrective action. We had good weather at destination and figured they'd get to the bottom of it at the next maintenance station. 

Shortly after takeoff, we got a Master Caution and ECAM "NAV LS TUNING DISAGREE." I immediately thought of the STBY NAV switch and its initial setting as well as the fact that part of the Maintenance reset procedure on the ground included manually tuning an ILS frequency on the RADNAV page to see if that would get rid of the CAT 2 message (it didn't). But RADNAV and STBY NAV were showing normal now. At a safe altitude, we looked into dealing with the ECAM. We discovered it's not in the QRH. While I was getting ready to write up a zero code, I thought I'd cycle both STBY NAV switches (Captain and First Officer) to see what would happen. Not only did this get rid of the ECAM but also the CAT 2 Status Message. We sent the write-up and an additional info message to Maintenance via ACARs explaining the event. It appeared at least, part of the airplane thought the Captain's side was still in STBY NAV even after the switch showed normal (and the RADNAV page did as well). I also mentioned in the write-up and communication that no reference was available for this ECAM in the QRH. It was my impression all ECAMs were represented in the QRH, but that obviously wasn't the case here. How many more are there like this? How would we know?

Synopsis

A320 Captain reported that procedures for an ECAM "NAV LS TUNING DISAGREE" warning could not be found in the QRH. Captain questioned what other ECAM warnings are not covered in the QRH.

ACN: 1490411 (1364 of 2908)

Time / Day

Date : 201710
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1490411
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout, both the FO and I noticed a strong rotten egg/sulphur smell permeating the cockpit. I called back to the FAs and they concurred that the smell was in the cabin as well. FO complained of a headache and I noticed that my head was starting to ache as well. Both of us donned our oxygen masks and wore them pretty much for the duration of the flight. I called the flight attendants and explained that I suspected we were having a fume event. I asked if they were dizzy, or if they had headaches and they said no. I told them to watch the passengers closely and call me if any started to experience discomfort as a result of the fumes. During this entire time, we were circumnavigating a line of intense weather. Also during this time, the smell would disappear and then return minutes later.

My symptoms increased from headache, adding light-headedness and mild nausea as we proceeded. I used ACARS to alert Dispatch and crew scheduling of the situation. We landed and parked without incident. I called the Flight Ops phone and spoke with [a representative]. We decided to have EMTs come down to the aircraft after the passengers deplaned. An EMT came down and checked our vitals, with two crewmembers having abnormally high blood pressure. All other vitals were in the normal range, however three of us were still feeling "not right" (Myself, FO and FA). FO and I declined to continue and went home. I called the on the job nurse hotline on the way home and filed a report.

Synopsis

A320 Captain reported several crew memebers experienced headaches and nausea from a rotten egg/sulphurous smell on the aircraft.

ACN: 1489773 (1365 of 2908)

Time / Day

Date : 201710
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
Aircraft Operator.Other 
Make Model Name : Skyhawk 172/Cutlass 172
Crew Size.Number Of Crew : 1
Operating Under FAR Part.Other 
Mission.Other

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14245
Experience.Flight Crew.Type : 11081
ASRS Report Number.Accession Number : 1489773
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Type : 1310
ASRS Report Number.Accession Number : 1489781
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Inbound to [our destination] at 2000 MSL on a 050 heading and 210 KIAS assigned for radar vectors to the visual approach to runway 11. ATC called, "Traffic 10 o`clock at 1800, 2 to 3 miles" I looked for traffic. No Joy. Traffic on TCAS showed as an amber target. ATC called, "Traffic 12 o`clock less than a mile." I Pilot Flying (PF), said to First Officer, "I don`t have him." TCAS issued a Descent RA of 1600 FPM, the autopilot was disconnected, and the RA was followed.

I looked up in time to see a light brown and white C-172 at 12 o`clock pass approximately 300 feet above our airplane. ATC asked, "What are you doing?" Pilot Not Flying (PNF) responded, "Complying with TCAS RA." ATC responded, "Ok, continue." TCAS issued a reduce descent command. ATC said, "When able, turn to 060. I turned to 060. TCAS issued, "Clear of conflict." I leveled off at 1400 MSL.

We asked ATC what altitude he wanted us at, and we were told 1800 MSL. I climbed back to 1800 MSL. We were then turned inbound and cleared for a visual approach to runway 11 and completed a normal visual approach to and landing to that runway. CRM and cockpit communication worked very well in a busy cockpit, which contributed to a safe outcome to this event.

Narrative: 2

We were on a heading of 050 and an altitude of 2000 FT MSL being vectored by ATC for the ILS 11 approach when this incident occurred. ATC informed us about a traffic at 10 o'clock within 2 to 3 miles. I immediately started looking for the traffic and informed the captain that I did not see the traffic and he also stated not in sight. I told ATC traffic not in sight.

After that ATC stated traffic less than a mile 12 o'clock 1800 FT MSL. Without delay, I started looking for traffic, at that very moment the TCAS system instructed us to descend. The captain promptly turn off the automation system and executed the descend. While still looking outside, I saw a Cessna 172 with white and brown colors directly ahead of us and 300 FT above our altitude. The Captain descended to 1400 FT MSL. ATC asked what are you doing? I stated responding to a TCAS RA and the controller said continue. After we cleared the traffic, I queried about our altitude. ATC instructed us to climb to 1800 FT. MSL the minimum vectoring altitude and also stated when able turn to a heading of 060.

Synopsis

Airbus 320 flight crew reported a NMAC during descent, and descending to comply with an TCAS RA.

ACN: 1487592 (1366 of 2908)

Time / Day

Date : 201710
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 3550

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : LAX

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1487592
Human Factors : Troubleshooting
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I have experienced the airplane not recognizing the 3,600 FT crossing restriction at HUNDA the last two arrivals into LAX runway 25L both times the airplanes were with Honeywell MCDUs. The airplane was in DES/NAV mode while pressing the APPR after passing HUNDA so that G/S would not be captured. Each time the airplane ignored the restriction, crossing HUNDA at 3,550 FT the first time and 3,540 FT the second time. I also noticed we were on the glide slope at HUNDA which defeats the purpose.

I don't see anything in the AIP pages except not to arm the approach until after HUNDA. Is this a problem with the airplane or how I'm flying the approach?

Synopsis

A320 Captain reported an issue with the FMS which did not comply with altitude restrictions during an ILS approach.

ACN: 1485636 (1367 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Relative Position.Distance.Nautical Miles : 5
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : A80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : ATL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1485636
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Approach told aircraft to increase speed to 310 knots. The aircraft complied. About 10 minutes later the controller instructed the aircraft to slow to 210 knots and then cleared aircraft direct to ZZZZZ. The aircraft crew complied. Then the controller instructed the aircraft to turn to heading 180 degrees. The crew complied. Then the controller cleared the aircraft direct to the same fix. The crew requested direct to the next fix due to the intercept angle. The Controller approved the request. The Controller then switched the aircraft to the final controller. The crew requested at least a 10 mile final. The Controller then turned the aircraft to a base leg. The crew requested a lower altitude and a slower speed. The controller denied our request. Finally the controller gave the aircrew permission to slow and the crew was able to get aircraft stabilized prior to final approach.

This is happening repeatedly in where the controllers are pushing aircraft and flight crews so the facility can maintain a high arrival rate despite the fact this compromises overall flight safety. The Airline, the Union and ATC know the problems but they fail to address it. I have repeatedly brought this to the attention of supervisory personnel and I continue to be told that there is not a problem. Safety is being compromised and there will be an accident unless this gets addressed.

Synopsis

A320 Captain reported a safety of flight issue due to excessive control instructions by the Final Approach Controller.

ACN: 1482331 (1368 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.MSL.Single Value : 1300

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway XX
Flight Phase : Final Approach
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
Make Model Name : UAV - Unpiloted Aerial Vehicle
Operating Under FAR Part.Other 
Flight Phase : Cruise
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1482331

Events

Anomaly.Conflict : NMAC
Detector.Person : Flight Crew
Miss Distance.Horizontal : 0
Miss Distance.Vertical : 50
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Airspace Structure

Narrative: 1

At about 1300 MSL on the ILS I spotted what I thought was a big black bird and told the FO to avoid it. When we passed under it by 50 ft we saw it was a black with white striped drone. We told the tower and continued uneventfully to land.

Synopsis

A320 Captain reported a NMAC with a UAV on approach to LAX.

ACN: 1481586 (1369 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BUR.Airport
State Reference : CA
Altitude.MSL.Single Value : 4500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class E : SCT

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8350
Experience.Flight Crew.Type : 3066
ASRS Report Number.Accession Number : 1481586
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Airspace Structure

Narrative: 1

Prior to FIL, we were told to fly heading 100, which was different than we expected for the Four Stacks Visual App 15 BUR. The 100 heading split Oat Mountain and San Gabriel Mountain, keeping our altitude higher than the recommended alt on approach (5000 feet abeam Oat). We were told to maintain 6000 feet until clear of traffic. We reported traffic in site and when clear we began a slow descent. However, as we descended we got a TCAS "adjust descent," call which we immediately complied with and slienced the warning. We did not expect our distance and rate to be a factor from the traffic in question. We had that aircraft in sight and continued our scan for other traffic as well as focus on the airport and ground track. While balancing the later two factors above, the TCAS alerted up to our proximity to the first aircraft, which while we adjusted our descent, that aircraft began a hard right turn. I can't be sure if that aircraft had adjusted his path into ours after we began our descent but if they had, that could have been the factor that triggered the TCAS to provide us with the RA which prevented an encroachment into an unsafe zone. After getting "clear of conflict," we continued scanning for other traffic as well as attempting to descend into BUR 15. Due to that first aircraft and another one descending into Whiteman airport, we were unable to descend and stabilize for the approach into 15. Prior to 1000 feet AGL, we went around and continued for vectors for Visual 8. Weather at time was VFR and we had the Four Stacks and airport in site 15-20 miles from the field as well as Whiteman. Winds at BUR were varying 130-180 at 8-10 knots which is why we chose RWY 15.

Finally, our vectors for the approach were a combination of vectors for RNAV15 and Visual. The 100 heading was more in line with the RNAV15 approach which we had line selected and used to back up our SA.

Synopsis

An A320 First Officer reported that multiple traffic targets in the vicinity of Whiteman (WHP) caused TCAS alerts and necessitated maneuvering that prevented the A320 from achieving a stabilized approach by 1,000 feet AGL on the Visual Approach to Runway 15 at BUR. A go-around was performed and the aircraft subsequently landed uneventfully at BUR on runway 08.

ACN: 1481370 (1370 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : Marginal
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1481370
Human Factors : Troubleshooting
Human Factors : Physiological - Other
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent into ZZZ, smelled oil vapor fumes. Both FO and I complied with the Smoke/Fumes memory items and donned our oxygen masks. Per FOM, "If air condition smoke is suspected," (particularly the part about "Faulty Equipâ€¦ Isolate") I turned off pack #1, waited, removed mask, fumes were still present. I put my mask back on, turned pack #1 back on, turned pack #2 off. Waited, then mask off, fumes still present. Mask back on for remainder of flight. Called and asked flight attendants if they smelled any odors in the cabin, but they said they did not.

At gate, tried using APU bleed, no odor detected, but switched off after test just in case. The flight attendants seemed to have mild, if any, symptoms. The FO and I had mild headaches, sore throats, and deep breaths caused me to cough. After a teleconference with the Chief Pilot's office, I called to report what happened and ask for advice, but as the incident/flight was completed, they had little interest and/or advice to provide.

After troubleshooting the packs using APU air, the mechanics felt that the Left Hand/#1 pack caused the oil fumes. The LH pack was deferred, the FO and I ferried the plane from ZZZ to ZZZ1. The flight attendants stayed and flew ZZZ-ZZZ2 with the passengers on a plane brought in. On the way into ZZZ1, we again smelled fumes and again put on our oxygen masks. The fumes open item was already in the logbook, per the ferry permit, so I didn't need to write it again, but informed the mechanics. Also, the station personnel in ZZZ are outstanding. They communicate well, they anticipate what we might need next and ask us about it. Can't say enough good about them, every time I've been there, but particularly with the fumes event.

As the problem was deferred, I don't know the cause. If I'm permitted to speculate, I would say cause was contamination from previous fumes events starting with an oil leak. A fumes event one day, and another later, also on the day of the first fume event led to changing an oil supply line O-ring. After the O-ring replacement, they "ran the packs for four hours." Again, only speculating here, but I'm guessing they didn't do a "pack burn" which involves much higher temperatures and as long as six hours, followed by a filter (ozone?) change. If that is the case, residual oil in the ducts and/or filters is what we smelled. Of course, there could also be another leak, or the residue from the leak in the APU enclosure wasn't completely cleaned up.

There are three steps maintenance must take in a fume event: (1) find the source, (2) fix the leak, (3) clean up. I think we are not terribly good at the last step. Pack burn and filter change would have prevented our fumes event.

Synopsis

A320 captain reported during the descent to the destination airport, oily fumes were detected in the cockpit. After donning oxygen masks and accomplishing all appropriate checklists, the flight continued to the destination airport where the aircraft was removed from service.

ACN: 1480586 (1371 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ

Environment

Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : PHX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : P50
Make Model Name : Commercial Fixed Wing
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1480586
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We had [a Slat Channel] MEL. During clean up and acceleration, the slats were slower than normal to retract. We hit wake turbulence from the aircraft in front of us causing a momentary exceedance of the max flap/slat speed while the slats were still retracting. The event lasted 2 or 3 seconds. 

Normally the slats retract quicker than with this MEL. Company policy is to retract slats and flaps and accelerate to 250 KIAS. There was no caution in the MEL information concerning the risk of potential overspeed. Also the wake turbulence we encountered added airspeed rapidly assisting in the speed exceeding maximum for configuration.

A note of caution in the MEL notes would have been very helpful.

Synopsis

A320 Captain reported a momentary slat overspeed resulted during clean-up after a wake turbulence encounter. A slat channel MEL, which causes the slats to retract slower, was cited as contributing factor.

ACN: 1479740 (1372 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Flight Dynamics
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1479740

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Once level at FL370 I noticed a slight pitch oscillation and the aircraft struggled to hold altitude. After about 10 minutes it got worse and ECAM F/CTL ELAC (Elevator and Aileron Computer) 2 FAULT [was displayed]. We followed procedure and selected "off" then "on" per ECAM. The fault cleared and we continued. The oscillation started again and same ECAM resulted. We followed and reset again per the ECAM. After the second reset I decided that even though the computer was resetting correctly the problem with the pitch oscillation was the ELAC 2 computer. After the 2nd reset much more aggressive pitch up oscillation occurred and I selected ELAC 2 OFF and the problem went away. Problem solved, I considered options. The only operational consideration was loss of CAT III dual. I consulted the QRH and confirmed ELAC 1 had picked up the load. Not concerned about safely continuing to [destination] I consulted the MEL and confirmed that ELAC 2 was not allowed to be deferred. I called Dispatch for consult with Maintenance. Maintenance and Dispatch confirmed that they wanted the aircraft to return to [departure airport]. Having no operational/safety concerns with the aircraft and weather CAVU at ZZZ, a joint decision was made to return without incident.

Synopsis

A320 Captain reported returning to departure airport after experiencing an Elevator and Aileron Computer (ELAC) anomaly.

ACN: 1478714 (1373 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1478714
Human Factors : Time Pressure
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Evacuated
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate
Result.Air Traffic Control : Provided Assistance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Accelerating down runway and after the 80 knot call out, we heard a loud bang and a yaw to the right. It was immediately followed by ECAM messages. The take-off was aborted and we stopped on the runway. ENG 2 FAIL was the top ECAM that was noticed. As we were assessing the situation the F1 called and stated there was smoke coming from ENG 2. There was no indication of fire from the cockpit. An aircraft behind us stated they saw a puff of smoke before the abort and also saw smoke coming from ENG 2. At this point ARFF arrived. We conferred with them and they stated they saw no fire from ENG 2 and that the wheels looked fine. The decision was made not to evacuate. We continued with all the applicable QRC QRH ECAM and normal checklists. Eventually we configured for a one engine taxi and had ARFF follow the aircraft to the gate.

Synopsis

A320 rejected takeoff due to indications [loud bang, right yaw, number engine smoke, right engine fail EICAS] of engine fail. Aircraft Rescue & Firefighting [ARFF] confirm no active fire, escorted to gate.

ACN: 1478466 (1374 of 2908)

Time / Day

Date : 201709
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nose Gear
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1478466
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Pre-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Due to tow-bar disconnection issues experienced during aircraft pushback from gates at multiple airports around the network, we would like clarification on the standard procedures that we should expect. Of particular concern is the position of the tow-bar relative to the aircraft's longitudinal axis after push back, especially when the disconnect procedure occurs with the tow-bar oriented at nearly right angles to the axis, with the nose wheel at the same angle.

Earlier this summer, I experienced a tow-bar shear pin failure during the disconnect procedure after the aircraft was stopped and the aircraft brakes were set. After the shear pin failed, releasing the tow-bar from the nose gear, the pushback crew showed us the tow safety pin and left. I requested Maintenance to inspect the nose gear prior to taxi. No damage was found, and the mechanics retrieved the broken pieces of the shear pin from the tarmac under the nose of the aircraft after the pushback crew departed.

1. Would the ground crew's performance in this case be considered normal procedure for a pushback that experiences a shear-pin failure during tow-bar disconnection?

[Another time] I was pushed back from the gate, tailing east. Clearance was given to start both engines and we did so. The pushback crew stopped the aircraft aligned on the taxi line, but with the nose gear and tow-bar at a severe angle to the aircraft longitudinal axis. They could not get the tow-bar disconnected and asked me to release the brakes to push back further in order to straighten the tow-bar so as to align with the aircraft, thereby relieving the lateral torque. I released the brake and the crew began to move the aircraft. Almost immediately the tow-bar became disconnected from the aircraft nose gear, and with both engines running, the high idle thrust launched the aircraft forward toward the pushback tractor and ramp crew under the nose of the aircraft. I immediately set the parking brake and full braking pressure was applied abruptly stopping the aircraft before crushing the ground crew between aircraft nose and the pushback tractor, also avoiding a collision with the push back tractor. They showed us the tow safety pin and left. I did not suspect any damage to the nose gear. We departed where a walk around inspection also revealed no damage.

2. Does the ground service manual address alignment of the tow-bar prior to stopping the pushback and before setting the aircraft brake?

3. Is there an approved ground service training manual?

4. If a training manual does exist, how does it train to position the tow-bar relative to the aircraft prior to disconnect?

One of our ramp agents [was hurt] after being run over by the aircraft nose wheels during gate arrival. I recently had to bring the aircraft to a stop because a ramp agent was squatting down next to the chalks ready for the nose gear while I was following the marshaller's guidance onto the gate. I motioned for him to clear the area before I continued to the stopping point because I could no longer see him under the aircraft while moving forward.

My concern is that too many of the ramp employees around the network do not have the knowledge base and experience necessary to safely perform their assigned duties.

In addition, their performance on the job is not being supervised properly and that we may suffer another severe injury or worse.

Suggestion: [We] adopt and train a procedure where the tow-bar will be aligned with the longitudinal axis of the aircraft prior to setting the brakes after push back and before disconnecting.

Synopsis

A320 Captain reported a tow bar disconnect during pushback due to shear-pin failure as the push crew aligned the nose wheel. Captain questioned ground personnel training may have contributed to the event.

ACN: 1477627 (1375 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : SNA.Tower
State Reference : CA
Altitude.MSL.Single Value : 800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : SNA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : SNA

Person

Reference : 1
Location Of Person.Facility : SNA.TWR
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 19.8
ASRS Report Number.Accession Number : 1477627
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Our ASR-9 radar is out of service with some sort of temporary fix in place. The radar data tags continue to drop off completely then reappear. They also float off in directions that are not consistent with real time. The aircraft in the pattern are mostly NOT there, but sometimes pop back on the scope. I had [an air carrier] on final, and did not call traffic for the parallel runway because there was not a target to show he was there. I did not see him out the window either because he blended in with the clouds. When I finally did see him, I called the traffic, but it was a late call. [The air carrier] understandably complained that that was a lot closer than he would have liked. 

I complained to my supervisor about the radar again. They have a memo to everyone saying to use our best judgement, but in my opinion, that would be to go to non-radar procedures. How can we continue to call ourselves a Class Charlie airspace, when we are using unreliable equipment to do our job? I can't single-handedly go to non-radar procedures just when I'm working! We need specific guidance, NOT use our best judgement. We need to let the flying public know that they are NOT getting actual Class Charlie service all the time. It's intermittent at best. We need to go to non-radar procedures, or they need to find a much better temporary fix. We have been dealing with this for almost 2 weeks now, and it's unsafe.

Synopsis

SNA Tower Controller reported being unable to provide required Class C airspace services due to a radar outage.

ACN: 1477095 (1376 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : CLT.TRACON
State Reference : NC
Altitude.MSL.Single Value : 14500

Aircraft

Reference : X
ATC / Advisory.TRACON : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : BANKR
Airspace.Class B : CLT

Person

Reference : 1
Location Of Person.Facility : CLT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2.0
ASRS Report Number.Accession Number : 1477095
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Aircraft X checked in descending via the STAR landing north and reported their altitude as 18,500 feet, which matched the Mode C readout. This appeared to be a very high altitude and they had a crossing restriction coming up which was 12000-11000 feet. While the aircraft was over [fix], the mode C showed 14500 feet. I asked Aircraft X to say altitude, and the pilot reported '12,000 feet. At that point the Mode C disappeared off the screen and they became transponder only. The pilot then said 'Uh we're having some issues with our Mode C, we'll try another one'. The aircraft continued to track the arrival with no altitude readouts. There was another crossing restriction of 9,000 feet coming up as well. After they passed that fix, their Mode C became active again and showed 9,000 feet.

I suggested that local management needed to follow up via a brasher warning, to which they declined. We had a verified Mode C readout that was 2,500 feet high on the arrival when the Mode C disappeared, which would be seem to be a pilot deviation.

Synopsis

CLT TRACON Controller reported an aircraft's Mode C quit working after it appeared it would not make a crossing restriction.

ACN: 1475019 (1377 of 2908)

Time / Day

Date : 201708

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Fuselage Skin
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1475019
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

After working full shift, I was dispatched to ZZZ to recover aircraft. I was told it was a lightning strike event and was told to inspect aircraft and map out damage. During [the] drive I was contacted by maintenance control to confirm we were to document any damage and engineering was prepared to give 40 hour limit for each rivet damaged if needed. Contract maintenance found approx. 10 damaged rivets. I inspected aircraft on and off through the night into early morning due to periods of rain and wind. After completion, our duty time was nearing, so I documented that the inspection was completed and also documented additional damaged rivets totaling 50 in the aircraft ELB (Electronic Log Book). I was told by the maintenance controller that he would defer all damage himself and also release the airplane into service. It was my knowledge that the aircraft was to ferry [out] for repair. After rest period, I learned that the aircraft was currently flying subsequent flights around the country. When returning to work from days off, I was approached by my [Union Representative] that the aircraft was down in ZZZ1 for maintenance addressing the damaged rivets.

Callback: 1

The reporter stated that he is an A&P mechanic and was called out to perform an inspection of an aircraft that had a lightning strike. The reporter stated the local FBO was used for a preliminary inspection and he completed the inspection and found numerous damaged rivets. He then confirmed with Maintenance Control the completion of all the required documentation. The reporter also stated the aircraft continued in service for a few more days until it was routed into a maintenance station where the required repairs were made.

Synopsis

Maintenance Technician reported that over 50 rivets were found damaged on an A320 due to lightning strikes. The aircraft continued to fly without proper maintenance action.

ACN: 1474938 (1378 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 16000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZDV

Aircraft : 2

Reference : Y
Aircraft Operator : Personal
Make Model Name : Sail Plane
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Mission : Personal
Flight Phase : Cruise
Airspace.Class E : ZDV

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1474938

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Miss Distance.Vertical : 400
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On climb out of Denver (Captain was PM, FO was PF), we were initially given a clearance to FL230. A minute or so later ATC reissued our clearance to maintain 16,000. We complied and the aircraft leveled at 16,000. ATC said something to the effect of once you are clear of traffic, we will climb you. We were trying to acquire the aircraft (which was said to be a glider), visually, but could not see it. I immediately looked at the TCAS and it displayed no targets. I changed mode from above to all, and it still showed no targets. ATC then said "traffic no factor climb and maintain FL230". As soon as she began to talk we got an RA immediately (the traffic was now showing on the navigation display) saying "Descend, Descend, Descend". The first officer immediately performed the memory items, disconnected the AP and began a descent. I responded to ATC by saying something like "Unable, descending responding to an RA". We descended about 300-400 feet and then we were clear on conflict. The glider flew about 400 feet on top of us slightly to our left, I am unsure of the lateral separation. After we ran the QRH and continued our climb to FL230, I checked with the flight attendants to make sure everything was ok in the cabin, everyone (including FAs) had still been seated in their jump seats with seatbelt fastened during the event. I began looking in the handbook for any further info. I found that this was indeed a NTSB reportable event and that an incident report was required. I followed the guidance and sent an ACARS to dispatch notifying them of the situation and providing the details outlined.

As sent to dispatch:
Prior to COORZ intersection
Responded to an RA while level at 16,000 ft descended about 300 feet, no dangerous articles onboard.

I believe ATC should have vectored us off of course to avoid getting anywhere near the glider. As with gliders, they can rapidly change altitude and headings. While ATC did initially change our clearance from 23,000 to 16,000, perhaps a turn would have avoided this entire situation. I also think that because of rapidly changing altitudes abilities of a glider, that's why we were unable to initially acquire the target on TCAS.

Synopsis

A320 Captain reported a NMAC with a glider west of DEN at 16,000 feet.

ACN: 1473787 (1379 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Trainee
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1473787
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

It was [early] morning as we were approaching on a moonless night into ZZZ (no visual outline of terrain). This was our 3rd flight of the day and our last day of this OE (Operating Experience) trip. We had been cleared for the visual approach so I, the pilot flying, proceeded with the descent. Having less than 20 hours of flight time in the airplane at the time was lacking judgement of descend profile and aircraft performance. As we had been cleared for the visual approach my goal was to descend, level off, configure and intercept. I recall that both pilots had terrain up on the ND (Navigational Display). As we received a GPWS alert, I responded to the alert. When we were clear of the conflict we had enough time to safely maneuver and land the airplane. I believe using open descend was providing an excessive descend rate.

Synopsis

A320 First Officer reported receiving a GPWS alert on a night visual approach.

ACN: 1473709 (1380 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Scheduled Maintenance

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1473709
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1473710
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

It was a normal morning. Inspected the cockpit and log book and found nothing out of the ordinary. I did the walk around and everything was in its place. The logbook had a sign off from the night before of a periodic inspection and the aircraft was signed back into service. So we boarded up and took off. When we were a little over an hour into the flight we got contacted by dispatch saying we were on maintenance and we were not supposed to leave. The dispatcher said he had "blown out" our performance data so it wouldn't come up for Takeoff. Yet I ran the data 8 min prior to departure and it came back fine. We were not informed by dispatch, maintenance, [Operations], the station, or anyone else of any delay. The red "Do not operate" tag was not on the thrust levers and no entry was in the logbook indicating any maintenance was necessary. There was literally no way we as a crew could know anything was going on until after dispatch contacted us. At which point we continued on to [the destination] with no issues. 

Someone from the company or station needs to be aware of the mx event and inform the crew. The red out of service tag should be used and an entry made in the logbook of a maintenance event that is currently happening. That way a crew showing up knows there is maintenance.

Narrative: 2

Aircraft had a "service must" bar on the plot and no [airworthiness alert]. Originally the aircraft was listed out of service until about XAz. When I asked [maintenance] about the aircrafts readiness, I was told aircraft should be good to make the departure time. As stations sometimes are very delayed in getting paperwork to [maintenance] to clear the [airworthiness alert], we usually just ask if we had heard from the station in question. At that time [maintenance] either reviews emails or a phone call is made to the station if the aircraft would be good, at which point it is conveyed to dispatch and paperwork sent to avoid late dispatch release and delays. Dispatch release was prepared and sent at XB:30z with a release time of XC:45z based off that information. 

At around XC:25z [maintenance] informed [operations] and myself aircraft was AOG (Aircraft on Ground) for parts and the flight would need to be delayed until XE:30z. Per our policies, I immediately hit the button to prevent the crew from the ability to get an aerodata solution. I moved my flight down on the plot and even typed a remark for a [release] stating " 1 NEW RELEASE TIME XE:30 AND INCREASED CI... XXXXZ CK" awaiting an actual time for a return to service. At XCz, after the [operations] pass down, the flight was then delayed an additional hour. At which time, I moved the flight down on my monitor again and adjusted my comment to show release time of XF:30z. At XD:00z [operations] sent an email to the station about the out of service aircraft and at XD:37 the station manager responded that the aircraft had already departed. 

At that time, there were no times on the plot and the flight was delayed until XF:30z. [Operations] asked if the flight was airborne and I called [the departure airport's] tower to confirm. Even though I reluctantly tried to tell the controller the aircraft was out of service with an engine issue, he said the flight departed more than 30 minutes prior. Checking on Flight Explorer, the flight was now showing [enroute]. I called ARINC to setup a phone patch and sent an ACARS XD:49z to confirm airborne status and XD:52z that it was imperative to contact dispatch. I inspected the ACARS log and found OUT and OFF times for the flight of XC:38z and XC:45z respectively. These times were entered on the plot shortly after. A phone patch was established and being more than half way, the decision was made to continue on to [the original destination]. 

During the conversation with the crew, they stated there was nothing in the log book or on the copy of the release they had showing that there was a reason not to depart. After talking to [maintenance], this [maintenance action] did actually have 8 hours remaining on it. The decision was then made to complete the round trip and to perform the required actions upon return. 

Fortunately, there was no actual regulation violation as the [maintenance action] still had time remaining to do this round trip. However, there definitely were quite a few company policy violations on multiple fronts. The aircraft was listed out of service and although I was told by [maintenance] aircraft should be ready, release should have been held. After the fact, the takeoff data was run at XC:12z and again at XC:36z, so locking it out around that time would have had no effect seeing as how they already had the numbers in the box.

After [maintenance] advised of the AOG status, dispatch should have not only hit the [disable] button, preventing the crew from getting takeoff data; but should have also called either [the departure airport] tower or Clearance Delivery to remove the strip. Although in this situation, the flight may have unknowingly actually been airborne at that point after reviewing the timeline of event due to the ACARS system not recording OUT or OFF times. Conferring with [maintenance], there should have been some sort of placard on the throttles that should show the aircraft out of service. 

After the aircraft landed, I spoke with [the Captain] about the situation. He says there were no placards stating that the aircraft was out of service on the aircraft, he had a signed off log book all the way back to a heavy check 3 days prior, and the station boarded for an on time departure. So he basically had no idea that the flight was delayed for any reason. 

While I don't know a whole lot about maintenance procedures on how an aircraft or when an aircraft is marked out of service, a greater thoroughness of making sure the correct placards are placed on aircraft when the aircraft is out of service may have at least caused a phone call to dispatch and prevented this situation. The station boarded the flight as though there were no delay although an almost 3 hour delay was published to the plot.

During my conversation with the captain, he stated that in flight a few passengers had asked the flight attendants about a delay to the flight that was showing on Flightaware. They didn't really think anything of it until my communications started.

A better communication between [operations] and stations on delayed flights. For this solution to work however, improvements need to be made on both sides. Our communication to the station of the delay posted on the plot should be enough, but a backup phone call would be welcomed. However, contacting stations via phone sometimes is difficult if not impossible. [Stations at large airports] are almost nearly impossible to contact via phone for any reason. Possibility of having a station contact this is actually present at the station, that would carry a duty cell phone on them at all times, that HAS to be answered would greatly improve communications

Synopsis

A320 Captain and the associated Dispatcher reported that the aircraft was flown on schedule with passengers when it was listed as out of service.

ACN: 1473325 (1381 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BUR.Airport
State Reference : CA
Altitude.MSL.Single Value : 3500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : BUR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class C : BUR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6505
Experience.Flight Crew.Type : 6055
ASRS Report Number.Accession Number : 1473325
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

On arrival to runway 15 BUR (Burbank) we requested runway 15 due to longer runway length and other possible runway had a slight tailwind. During approach and approaching SCRET we received a GPWS "TERRAIN TERRAIN, PULL UP". We complied and performed the escape maneuver. We advised ATC. We then requested and landed on runway 08. This was both the Captain's and First Officer's first time into this airport.

Synopsis

The Captain of an Airbus 320 reported that they received a GPWS alert during final approach.

ACN: 1472481 (1382 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Relative Position.Distance.Nautical Miles : 6
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : A80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ATL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 11000
ASRS Report Number.Accession Number : 1472481
Human Factors : Other / Unknown
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

ATL approach kept aircraft at 4000 feet and 180 knots. As has been the case for the last several years approach control will not give reasonable speed assignments or altitudes on approach. Complaints to the company and the union fall on deaf ears. It is impossible to fly with these restrictions and be stable within 1000 feet of landing. Moreover ATL controllers consistently argue with crews who request vectors around hazardous weather. Continuing this practice will result in an aircraft accident. As a pilot who requests vectors around weather or reasonable speed assignments veiled threats of a violation are waved in front of my face. Interestingly enough Chicago and New York controllers are some of the best in the world and rarely push air crews on approach. Hopefully this report will draw attention to this problem and help prevent an accident.

Synopsis

A320 Captain reported that A80 controllers routinely keep aircraft high and fast during approach and disallow deviations for weather.

ACN: 1471331 (1383 of 2908)

Time / Day

Date : 201708

Place

Locale Reference.Airport : SNA.Airport
State Reference : CA
Altitude.MSL.Single Value : 2200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Route In Use.STAR : DSNEE1
Airspace.Class C : SNA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 132
ASRS Report Number.Accession Number : 1471331
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12080
Experience.Flight Crew.Type : 10870
ASRS Report Number.Accession Number : 1471345
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was the Pilot Flying as FO, and the CA was Pilot Monitoring on our final leg to SNA. Prior to our initial descent into SNA we had conducted a thorough arrival and approach briefing which reviewed the terrain located in the vicinity of the final approach course, and also GPWS cautions and warnings (as well as the appropriate recoveries to me made in the event of a GPWS caution or warning). The CA has also commented that with the new DSNEE routing careful caution should be made to the rate of descent, as a high rate of descent could cause GPWS alert. We were cleared for a visual approach to runway 20R at SNA and I had asked the CA to extend BONKE in the FMGC. I maneuvered the aircraft in heading mode just outside of LEMON intersection. The CA set 2200 as the MSA at LEMON, as we continued our descent, I shifted my heading about 5 degrees (still inside BONKE), just as the ALT displayed, a "terrain, terrain" warning sounded momentarily. We both had visual of the terrain, and also noted the Radar Altimeter reflecting a positive trend as we continued the approach. After a successful approach the CA and I debriefed the event at the gate. We should have set 2800 as our ALT in the FCU as my intercept angle to the final approach had changed by a few degrees vs the lower 2200 at LEMON. This was a positive learning experience, especially as a new FO.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported a terrain warning during a day visual approach to Runway 20R at SNA and continued the approach to landing while the terrain warning quickly silenced.

ACN: 1471320 (1384 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PDX.Airport
State Reference : OR
Altitude.MSL.Single Value : 2300

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : P80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : PDX

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2982
Experience.Flight Crew.Type : 855
ASRS Report Number.Accession Number : 1471320

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10909
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 7490
ASRS Report Number.Accession Number : 1471326

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

On approach into PDX RWY 28R, we were cleared the visual during night/VMC conditions. ATC cleared us to descend and maintain 2,000 feet until 6 mile final or TOLOC. During the descent, approximately 5 miles south of TOLOC, the GPWS sounded "Terrain, Terrain, Pull up, Pull up." Captain initiated the CFIT recovery. As he initiated the escape maneuver, the GPWS warning ceased prior to Captain reaching Takeoff/Go Around (TOGA) thrust and full aft stick. I verbalized, "TOGA thrust, Stow Speed Brakes" and verified speed brakes were stowed. The terrain and landing environment was in clear sight. During the recovery, the altitude horn sounded. Captain began a recovery to assigned altitude, descending to 2,000 feet. Aircraft momentarily descended below 2,000 feet. Almost immediately following the cessation of the GPWS warning, a traffic alert was received. ATC advised us of the traffic and reissued the altitude clearance. Landed uneventfully.

Narrative: 2

On visual approach to runway 28R in PDX coming from the south at 2500 feet VMC radar control with airport and runway in sight, auto-pilot on, ATC told us to descend 2000 until outer marker and cleared for visual app 28R. Captain flying initiated descent on auto-pilot and proceeded direct to outer marker. Approximately 200 knots flaps-1, 5 miles from outer marker and cross-checking glide slope indicator (announced). [At] approximately 2300 feet GPWS system calls, "Terrain, terrain, pull-up, pull-up". Captain initiates CFIT procedure and then quickly terminates procedure as the warning stopped and had ground environment in sight and airport and runway in sight. Then a traffic warning and an ATC call is announced for traffic. The loud altitude warning horn is heard- (initial descent was to 2000 feet but climbing and now greater than 2000 altitude deviation caused the altitude warning horn). Flight Control Unit altitude is increased to stop the horn. Captain descended towards 2000 feet for the approach. Captain momentarily descended below 2000 with traffic in sight, while aligning for runway landing. Aircraft lined up for approach 28R and uneventful landing. During post flight debrief and aircraft ground track analysis, Captain determined the spot elevation 5 miles south of TOLOC outer marker contributed to the potential CFIT trigger.

Synopsis

A320 flight crew experienced a GPWS terrain warning descending to 2000 feet 5 NM south of TOLOC during a night visual approach to PDX.

ACN: 1471141 (1385 of 2908)

Time / Day

Date : 201708
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : MD-80 Series (DC-9-80) Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 23820
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 7900
ASRS Report Number.Accession Number : 1471141
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Runway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were cleared to leave the ramp and taxi to the runway. We were told our sequence was to follow the MD88 ahead and monitor the tower which we did. Approaching the taxiway the MD88 started taxiing to cross the left runway which was being used as a taxiway as there was a tug pulling an aircraft stopped on the runway. So as previously cleared we continued to follow the aircraft ahead.

Approaching the runway Tower called our flight number so I stopped with nose slightly on the runway. My co-pilot then told Tower our clearance was to follow the MD88 and monitor the tower which we were doing. The controller then said he has a phone number to call for possible runway violation. We continued on with no further incident. There was no threat to safety in anyway. The clearance to sequence and follow the MD88 superseded the one given to us on the ramp. We were never told to hold short of a taxiway or the left runway with the second clearance. There was a definite communication failure on both parties, ATC and us. With the tug and aircraft stopped on the left runway the runway was obviously not active. In the future with this type of communication I will clarify the intent.

Synopsis

A320 Captain reported being issued taxi instructions to follow another aircraft and following the aircraft onto a runway to cross, but that was not ATC's intention.

ACN: 1470902 (1386 of 2908)

Time / Day

Date : 201708

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cockpit Furnishing
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1470902
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

After block in I dropped my ink pen, it bounced off my shoe and went through the large hole beneath the footrest. The Airbus used to have black bristles that would prevent objects from entering the holes on each side of the footrest(s), they have been removed over the past few years. The current large opening on each side of the footrests is large enough to fit your iPad through and could interfere with flight control linkages.

Synopsis

A320 Captain reported dropping a pen into a large hole beneath the footrest. The bristle seals have been removed exposing large opening.

ACN: 1469435 (1387 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y

Component

Aircraft Component : Pneumatic Control Valves
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1469435

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

This is an informational report, we did not deviate from any policies or procedures. We flew the aircraft with MEL 30-21 applied for the Number 2 PRSOV valve on the Number 1 engine. During flight we briefly turned on the engine anti-ice and we received the ECAM message "ANTI ICE ENG 1 VALVE CLOSED: Avoid icing conditions" by the time we received the fault we were clear of clouds and we maintained clear of icing conditions for the remainder of the flight.

We entered the discrepancy in the logbook, because under the MEL the engine anti-ice should be operative. Contract maintenance was called and ultimately worked on the plane over the next couple of days. We assisted contract maintenance for several hours during those days with engine runs. We were only there to provide assistance with starting and shutting down the engines during the required testing. They were unable to determine what the issue was and ultimately maintenance secured both valves closed on the Number 1 engine and applied MEL 30-21. We were then able to operate clear of icing conditions on the third day.

Over the course of this situation, a couple areas of concern came up. First, MEL 30-21 requires maintenance to secure the inoperative valve open using AMM TASK 30-21. However, that same task card is used on MEL 30-21 which calls for the valve to be closed. I have not seen the task card, but it seems like having one task card for two opposite tasks may cause confusion when completing the maintenance procedure. The other concern that I have is that there didn't seem to be clear enough direction on which valve is considered "Number 1" and which one is considered "Number 2". The valves were referred to by number and by "forward" or "aft". It seemed to take a while to determine which valve was which and that leads me to believe that the documentation may not be clear enough. If there is confusion on which valve is which, then it would be possible to accidentally secure the wrong valve in the open position when applying the original MEL 30-21, which would cause the failed valve to remain in the closed position and prevent the engine anti-ice from working. I am not saying that this is what happened in our situation, because from the information that I have gathered, the correct valve was secured correctly. I don't know why we received the ECAM message, but I am sure that we shouldn't have received it.

It would be worth reviewing these MELs and Task Cards to see if they need to be revised or if additional guidance needs to be provided. I am curious why we received that ECAM message with the original MEL, because according to the MEL the system should have operated normally.

Synopsis

A First Officer of an A320 reported his concern about the application of an MEL procedure for engine anti-ice.

ACN: 1468009 (1388 of 2908)

Time / Day

Date : 201707

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 581
ASRS Report Number.Accession Number : 1468009
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 790
ASRS Report Number.Accession Number : 1468020
Human Factors : Training / Qualification

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After pushback and during taxi, ground control advised us that we had a 1:10 minute ground delay for our flight due to construction at [destination airport]. Ground control gave us a place to pull over and we shut down both engines and left the APU running. After finishing checklists and discussing the delay and our ample fuel status, the CA asked if I was okay with opening the cockpit door. I replied "absolutely." So, as I was communicating our status with dispatch via ACARS, several passengers wanted to look in the cockpit. The Captain and I agreed that this was a fine idea as we thought it would be great for passengers to have something of interest to enjoy while we were waiting on the ramp for an hour or so. 

Several adults and children came up to the cockpit to take a look around and ask questions with some folks taking pictures, etc. It was really enjoyable handing out wings to the kids and talking to parents about [our company] and aviation in general. About 30 minutes passed and ground control advised us that our delay had been shortened and asked us to get ready for departure. We had all passengers sit down, secured the cockpit door and performed our preflight checklists as per SOP.

We took off and landed without incident. The flight itself was uneventful.

The reason I am reporting this is, during the cruise phase of the flight the Captain and I talked about the recent [industry problems] and wondered if any changes had been made to SOP concerning photos, etc. I looked in the FOM for anything concerning cockpit access on the ground and found the last sentence of a paragraph, which states that you should not allow cockpit access after you have pushed back, even during a ground delay with engines shut down, which was our exact situation. It actually states that you should not conduct cockpit tours, etc. Which is pretty much exactly what we did. So we talked about that and decided we had made a mistake and should report it. We both talked about learning a new lesson in the FOM. I've had people up in the cockpit many times at the gate, so I just deferred to that way of thinking. It was quite enjoyable interacting with our passengers but I realize now that I should have referenced the FOM first.

Narrative: 2

On this flight, as we approached the runway ATC informed us we had a wheels up time of 1 hour 7 minutes later. We were directed where to stop and shut down the engines and we did so.

During my years as a First Officer (FO), many times during similar circumstances, both on and off the gate, Captains have opened the door and invited customers to visit the flight deck while waiting for the wheels up time. It was my understanding that with both engines off, opening the door was authorized. This time I thought it a good gesture for the passengers so I asked the FO if he was ok with doing that and he agreed. At the appropriate time we closed the door, prepared for flight and proceeded without incident.

During cruise the FO and I began to wonder if the FOM contained specific guidance about such activity, and upon inspection we found in FOM that it is not authorized. I don't know if the guidance changed in recent years or if it has always been there, but I acted based on prior experience without first verifying if it was permitted by the FOM.

Synopsis

A320 flight crew reported receiving a ground delay after taxi and elected to shut down and allow passengers to visit the cockpit. Once airborne the crew decided to check the FOM and found that the practice was no longer allowed.

ACN: 1467995 (1389 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZOA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZOA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 366
ASRS Report Number.Accession Number : 1467995
Human Factors : Training / Qualification
Human Factors : Confusion

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

As I was briefing the arrival, approach, missed approach, and engine out procedure it came to our attention that the SFO VOR being NOTAMed out of service could be an issue for our engine out procedure. On the Airbus, to fly the procedure, we put the SFO VOR in the RAD page. But, this is raw data. Neither the FO nor I could come up with a way to comply with the engine out procedure if we had an engine failure. We sent dispatch an ACARS Message and they came back saying the Duty Manager says use the pop up engine failure from the FMGC. I don't believe this is an option on our fleet or least not one we knew we had for SFO.

Since 2 miles was easily seen on the fix page, we put the radial and distance in the fix page. I was told during training this month, that using the fix page is for INFO only and not operational in regards to radial and DME. When I got on the ground, I got a hold of the Line Captain that did my original training on the Airbus. He recommended building a point and coming up with as close as we could to the engine out procedure. Again, this is good info, but I'm not sure if the distance was SFO/2.3 and not 2.0 we would have been able to figure out where we would have been over the ground once we reached the 2.3 mile point. I think this needs to be looked into. I don't feel that on a fleet where we need raw data to compute this information (DME) we should be trying to guess where that point will be when we are in the middle of an emergency. If there is an easy way to use the FMGC to compute DME, it needs to be taught better, because in the short time that the FO and I had to plan for the arrival, we couldn't figure out how to do it.

Synopsis

A320 Captain reported questioning the engine out missed approach procedures specified by his company which requires the SFO VOR which was NOTAMed out of service.

ACN: 1466836 (1390 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Icing
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Engine Vibration Indication
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 622
ASRS Report Number.Accession Number : 1466836
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 797
ASRS Report Number.Accession Number : 1466996

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

We departed ZZZ 2.5 hours late after extensive on gate and taxi delays due to thunderstorms impacting the field and departure corridors. A Crew Critical Offtime (CCO) extension was required on our duty day in order to depart. 

A Take Off Go Around [power] takeoff with wind shear precautions was planned and executed. On climb out we remained IMC to cruise altitude, encountering rain and turbulence throughout the climb. No airframe icing was noted, but engine anti ice was turned on as Total Air Temperature (TAT) dropped to 10C. Engine AI was turned off at -40C TAT. Rate of climb was very low as we passed FL300, and ATC was advised due to less than 500 FPM climb rate. 

As we leveled at FL340, the engine ECAM page appeared with a 6.3 N1 vibration indication flashing. The First Officer (FO) retrieved the vibration checklist in the QRH and we reviewed it. With Auto Throttle off I cycled the thrust from idle to climb thrust. The vibration indicated 3.4 at idle and 8.0 at climb thrust. I sent this information to maintenance. Dispatch immediately sent a suggestion to divert. 

We descended to FL270 (for more airspeed envelope) and I continued to try other power settings with the same result. At high vibration both the FO and I noticed a distinct smell in the aircraft. The smell was also noted by a flight attendant in the aft galley.

I requested a phone patch with maintenance. After discussing the issue with maintenance and dispatch I made the decision to divert. Dispatch sent landing performance and I gave the flight attendants a briefing. I made an announcement to the passengers and the FO coordinated a turn to ZZZ1. 

I decided to land overweight rather than remain airborne with an engine problem. We ran the overweight landing checklist and landed uneventfully.

As we descended to warm air, the vibration indication returned to normal (0.0) so I suspect engine icing as the cause.

Maintenance write-ups were entered for both the engine vibration and the overweight landing.

Narrative: 2

After leveling off at FL340 in IMC conditions (rain) and the engine anti-ice on, the Captain noticed a vibration advisory for the #1 Engine N1 being approximately 5.4 units and slowly climbing. All other engine indications were normal with no apparent airframe vibration or unusual sounds or smells. #2 Engine N1 Vibration (VIB) was under 1.0 units. Asked ATC for FL250 and began a descent. During the descent we ran the Eng - High Vibration, High N1 vibrations in flight, in icing conditions checklist. The Captain performed the ice shedding procedure on #1 engine, multiple times. As the throttle was reduced to idle the VIB indication would decrease slowly below 5.0 and then start to climb again. As the throttle was increased the VIB would rapidly increase, one time reaching 8.4 units, accompanied by a metal friction smell. We broke out of the weather around FL290 so we requested to level off at FL270. Ice shedding was done in and out of clouds. The #1 Engine VIB averaged 6.8 units. Captain was advised by maintenance to not continue the flight and land. Closest divert was ZZZ1. Coordinated divert into ZZZ1 for XYR. Ran the Overweight Landing checklist and requested landing data for XYR from Dispatch. Weather at ZZZ1 was VMC with light winds out of the southeast. During the descent, passing through 10,000 feet we noticed the #1 Engine VIB was under 5 units and then while on extended final at 4000 feet #1 Engine VIB indicated 0.0 units. The Captain made an uneventful overweight landing (147,000 Gross Weight), taxied clear and to the gate with normal VIB indications on both engines.

I would suggest adding something to the Eng - High Vibration checklist providing more information on sounds/smells to expect. Maybe add a False Indication section to the Eng - High Vibration checklist. I wouldn't have done anything different based on all the information/resources available and used.

Synopsis

A320 flight crew reported that they had a very slow climb rate above FL300 and the engine ECAM page appeared with a very high N-1 vibration indication.

ACN: 1466443 (1391 of 2908)

Time / Day

Date : 201707

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Galley
Cabin Activity : Deplaning
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1466443
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Evacuated
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During the deplaning process catering came on board and provisioned the aircraft. After removing the last cart the catering person said our airplane is on fire and pointed to the tail of the aircraft. She hurried to close the door. At that moment Flight Attendant (FA) A called to the aft galley and I informed him that catering said the aircraft is on fire and we need to get everybody off the plane. I started shouting commands to the passengers to leave belongings and get off the airplane and FA C got on the PA from the aft galley and informed people to leave everything and get off the plane due to a fire in the back of the airplane. 

3/4 of the passengers had already deplaned however, we had two wheelchair passengers seated in the back that slowed down the deplaning process. Unfortunately because there were two Flight Attendants in back of the airplane and only one upfront we were unable to assist the wheelchair passengers. Passengers continued to retrieve their bags out of the overhead compartments and we continued to shout commands to leave everything and get off the airplane. FA A made a PA from the front of the plane trying to get passengers to leave their belongings and get off as quickly as possible due to a fire. After the last passenger deplaned the aircraft we spoke to maintenance in the flight deck and he informed us it was safe to get our bags and get off. The three of us checked the cabin to make sure everyone was off and removed all the passengers bags that were left on board and took them up the jetway for the passengers to receive their bags back. We were informed that the back of the airplane was on fire and it had a very strong odor of fuel and smoke.

Synopsis

A Flight Attendant reported being informed that the tail of the aircraft was on fire during deplaning. The Flight Attendants commanded the remaining passengers to evacuate and leave their belongings.

ACN: 1465802 (1392 of 2908)

Time / Day

Date : 201707

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Air Conditioning Distribution Ducting, Clamps, Connectors
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1465802
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

In my case [on two separate incidents] the solution [the company] applied to an oily fumes event was to address the source of the oil leak, then change the cabin ozone filters. From what I can tell, no effort was made in either case to clean the ducts through what Airbus calls a "pack burn."

I attended [an industry safety forum] and the manufacturer of pack filters was there. He said the original filter is not designed to filter out oily fumes. They have a version available that does that, same part number, no STC required, that contains a carbon filter element.

So what [the company] is doing, then, is temporarily covering up the oil fumes until it saturates & overcomes the new filters. I believe this may have, in part, contributed to fume events I had on the ground, at the gate.

I am not completely sure about the oil-saturated filter theory because one of them also involved over-servicing of the APU oil. But in all three cases, heavy rain immediately preceded the fume event. I believe in some cases, oil was already in the packs & filters, but in "nooks and crannies" and was released when the air density became much heavier almost immediately with the advent of the heavy rain.

Anyway, I am concerned that due to lack of cleaning ("pack burns"), we are continuing to subject crews and passengers to continuous low-doses of oil fumes (known carcinogen), and then also large doses during fume events.

Callback: 1

The reporter stated that there is a concern for his health that flying an aircraft after oil smell has been reported and repaired that the ducts still have residual oil and the fumes are still present. The reporter stated he has no way of knowing what kind of repairs Maintenance accomplished to resolve the issue or if a "burnout" of the ducts was accomplished to burn all the oil out. The reporter also stated, too often the source of the oil smell is deferred, ("ie; APU over serviced") but the odor remains. The reporter added he would like more information of any maintenance action that has taken place and would be happy to take a delay in order to ensure all the proper steps have been taken to resolve the issue of the oil smell.

Synopsis

A320 Captain reported that after an oil smell in the cabin report, there was no effort to clean the pneumatic ducts.

ACN: 1464576 (1393 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 11000

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : SCBBY1
Airspace.Class E : ZLA

Person

Reference : 1
Location Of Person.Facility : ZLA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 1
ASRS Report Number.Accession Number : 1464576
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X was an aircraft landing ONT. He was vectored north away from the usual initial fix on the STAR for the SCBBY1 to separate him for other traffic underneath him so he may descend. I APREQd him direct SCBBY for SCBBY1 arrival with Springs sector (combined) and it was approved. Normally from the initial fix, the aircraft descends via with no bottom altitude. I expected this to be ok and didn't coordinate an altitude and was not told to maintain anything specific. I descended him via the arrival and initiated a handful. A few minutes later the sectors were combined for midnight configuration. The relieving controller forgot that Aircraft X was still on the ZLA frequency and removed it from her scope. The aircraft continued to descend via below the MIAs and Springs called and told her to stop his descent which she did. The aircraft was then switched to the appropriate frequency 

An altitude should have been coordinated by either myself or Springs sector to ensure this doesn't happen again.

Synopsis

ZLA Center Controller reported that an aircraft was forgotten during a shift configuration changed and dropped by accident.

ACN: 1464299 (1394 of 2908)

Time / Day

Date : 201707

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13470
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 1464299
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were told to enter a holding pattern due to thunderstorms on the field. While descending we experienced a total green hydraulic failure. We [advised ATC] and diverted. During the divert we had to run many checklists which included manually extending the gear. We came to a complete stop on the runway with emergency equipment standing by which shut down the runway. We were then towed to the gate. At that point we proceed to Ops to fill out safety and incident reports which we could not do because the system was down. We spent several hours in Ops, talked to the Operations Manager several times, and tried to arrange hotel.

Synopsis

A320 First Officer reported the green hydraulic system failed resulting in a successful divert.

ACN: 1462405 (1395 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Aileron Control System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1462405
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Immediately after takeoff, received the following ECAM messages:
F/CTL L AIL FAULT
F/CTL R AIL FAULT
F/CTL ATN LAW
Upon completion of AFTER TAKEOFF CHECKLIST, ATC was advised. Requested to level off at 12,000 feet to comply with sector MEA, ensure terrain clearance, and complete ECAM actions and COM procedures. Due to the nature of the ECAMs, the decision was made to return to the departure airport. In an overabundance of caution, the decision was made to hold within 40NM northeast of the airport and burn fuel until reaching a landing weight below 150,000 lbs. Dispatch, Maintenance, Operations, Cabin Crew and ATC were all involved in decision-making communications. In addition, dead heading Captain occupied the jumpseat and served as an additional crew member. Passengers were briefed of situation and plan to return to the departure airport after decisions and planning/coordination were complete. Once the desired landing weight was ensured, the overweight landing COM procedure was applied and a visual approach, backed up by the ILS, was conducted to [the] Runway. Emergency equipment was requested and standing by.

A normal landing was conducted. Once the aircraft was stopped on the runway, emergency personnel verified that brake temperatures were normal, ensured no other external anomalies were present, and cleared the aircraft for taxi to the gate.

After PARKING CHECKLIST was completed, all discrepancies were entered in the Aircraft Maintenance Log (AML).

Synopsis

A320 pilot reported an ECAM alerted to multiple failures immediately after takeoff.

ACN: 1462037 (1396 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : Mixed

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1462037

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Upon selection of APU Bleed at gate in [departure airport], I noticed a flickering of the APU bleed switch light, then steady illumination. With a couple seconds, the smell of oil became so strong in the cockpit, you would have thought it was poured directly on the flight deck floor. As that subsided, the "wet sock" smell became very prevalent. I notified Line maintenance, and as a result of an investigation, the mechanics deferred the APU, as well as the left air conditioning pack. The flight proceeded without incident until Top of Descent into ZZZ. Once power came to idle, the wet sock smell again filled the flight deck. This time, I felt my eyes wanting to cross on two occasions and I told my First Officer (FO) that we may need to put the oxygen masks on. I could not switch off the right pack as the procedure would suggest as we would become unpressurized and still in the low 30s to high 20s flight levels. The smell did subside as we got closer to the ground, and after landing and clearing all runways, I requested my FO to turn off the right pack. After parking, the flight attendants notified me the "wet sock" smell began again on descent. By that point, I had already made another entry in the aircraft maintenance log. The rest of the evening I did have a headache and felt very lethargic; but as I awoke this morning I feel 100% back to normal.

Not much that I can suggest, but I would like clarification regarding our procedure as it stands to turning off packs. If I had done that, it obviously would have made the night much more interesting.

Synopsis

A320 Captain reported an APU and one pack had been deferred prior to departure but upon descent, fumes filled the flight deck strong enough that he considered putting on the oxygen masks.

ACN: 1461959 (1397 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
Make Model Name : UAV - Unpiloted Aerial Vehicle
Operating Under FAR Part.Other 
Flight Phase : Cruise
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 80
ASRS Report Number.Accession Number : 1461959

Events

Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
Miss Distance.Horizontal : 200
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While on dogleg to final for a visual approach to 24R, I noticed a drone at about 200 feet distance on the right side of the aircraft. The drone appeared to be a quadcopter, black and about two feet in size and was reported to ATC immediately.

Synopsis

A320 First Officer reported a close encounter with a UAV while on a visual approach to LAX.

ACN: 1461941 (1398 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6627
ASRS Report Number.Accession Number : 1461941

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 331
ASRS Report Number.Accession Number : 1461963

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon landing gear extension, we received an ECAM "BRAKES AUTO BRK FAULT" message. No auto brakes had been selected. We also noted that the three gear down-lock indicators (green triangles) were not illuminated. The gear UNLK lights on the panel were also not illuminated. With flaps full selected, I confirmed that the Landing Memo was green, which includes the gear down lock notation. We both felt that the gear was in fact down and locked, but I was contemplating a go-around to give us some time to look at the QRH in order to gain some more knowledge of what might have occurred. (I had yet to check the WHEEL page for green gear confirmation.) Prior to 500 feet AGL, the ECAM message cleared itself, and the three gear down-lock indicators illuminated green. We both agreed that it was safe to continue the approach and landing. My thoughts were that this might just be an Airbus anomaly. After a normal landing on runway XYL, I selected full reverse thrust. At 80 knots, I came out of reverse, and then applied the brakes to exit at [the] taxiway. The brakes felt very soft and abnormal (mushy at best). I could tell that something was definitely wrong. I applied additional pressure to the brakes and realized that I would not make the high-speed taxiway, and focused on making the next turn-off. Once again the brake effectiveness was not going to slow adequately for that exit. I was communicating all of the braking issues with my First Officer. Noting that the normal brakes were not working properly, I reached over and turned the A/SKID; Nose Wheel steering switch off to revert to alternate brakes. I applied several applications, remembering that I was supposed to limit the pressure to 1000 PSI to prevent wheel lock-up. This was not working like the simulator, and the braking effectiveness was nearly nil. I applied considerably more pressure to the brakes in a normal reaction to try to stop the airplane. Realizing that the end of the runway was getting closer, I reverted to using full reverse thrust. My thoughts were to get as slow as I could, and then resort to my final option of using the parking brake to stop the airplane. My First Officer then stated, "What about the parking brake?" I asked him to go ahead and cycle it. He applied several applications, and the airplane came to a complete stop, slightly off the runway center line but still completely on the runway. We were several hundred feet short of the end of the runway. My First Officer instinctively instructed the passengers to "remain seated". We told the Tower of our condition, and they instructed us to hold our position on the runway, and then they began coordinating our tow to the gate. The tower controller asked if we needed emergency equipment do to the potential of hot brakes, but the hottest indication peaked at 240 degrees (a normal green indication). Obviously on a 90 degree day with the amount of braking that had been applied, these were very abnormal brake temperatures. All hydraulic system indications were normal. I discussed the situation with the Purser and made several announcements to our passengers to ensure that we were safely parked on the runway, and that we would be towed to our gate. Maintenance arrived and towed us to our gate, where I briefed them on the event. I submitted several reports to provide Maintenance with a history of what we encountered.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported receiving an ECAM "BRAKES AUTO BRK FAULT" message. Normal brakes did not funtion properly and alternate brakes were also ineffective. The Captain was able to stop the aircraft using applications of the parking brake.

ACN: 1461392 (1399 of 2908)

Time / Day

Date : 201707
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 2800

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : BUR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 556
ASRS Report Number.Accession Number : 1461392
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While on the charted RNAV visual approach into runway 15 at BUR, we were approaching GOBLT on both the lateral and vertical path. We received one "terrain, terrain pull up" just as we were beginning the final turn. The warning happened once and the captain continued the approach due to his familiarity with the terrain and airport. We are aware of SOP to conduct the escape maneuver when at night, but the captain decided the best course of action was to continue because the warning ceased immediately after we began our turn.

Synopsis

A320 First Officer reported receiving a GPWS warning during an RNAV approach. The crew decided to continue the approach instead of following standard operating procedure.

ACN: 1460870 (1400 of 2908)

Time / Day

Date : 201706
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : ORCKA1
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Climb
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1460870
Human Factors : Distraction
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I am a new hire on IOE and was paired with a company check airman. I as the FO was the PF doing the takeoff and departure from LAX. Prior to pushback we briefed, and were demonstrating a flap auto retraction, while climbing out of LAX on the ORCKA1 Departure. We were climbing on heading 251 climbing to DOCKR. We were asked by ATC to begin a southern turn. As the PNF and since I was hand flying, the Captain selected the heading and rolled it in the southern direction 15 degrees. I was head down, hand flying the aircraft and following the command bars as instructed by the Captain, so as to get the full benefit of the flap auto retraction exercise. At that time we experienced moderate bumps that I believe to have been wake turbulence from previously departed B757. This caused the Captain's iPad mount to come loose and fall. The Captain worked to reinstall the iPad, and find the departure procedure in order to identity the correct heading. At about the time the Captain was able to verify the correct heading, ATC advised us we had turned too far south and gave us a new heading. We complied and had no further issues. The FMS was verified and correct prior to takeoff. We discussed and briefed prior to push back the change in heading at DOCKR was 15 degrees. We speculate that ATC may have asked to begin our turn after the FMS was commanding our new heading of 236. So that when a southerly HDG was selected, we essentially were turning in addition to the correct HDG change. I the FO and PF was hand flying the airplane.

I was focused on hand flying directly where the flight directors commanded, NAV had been captured and anticipating our auto flap retraction demonstration. I should have changed several things. First even though the headings were clear on the SID I should have written both initial headings on my paperwork. This is normally my practice with EO procedures and complex departures. I need to do this for all departures. The directions from the Controller were nonspecific. We were told to begin our southerly turn. If we were told to turn to a specific heading or if I would have asked to verify the heading he wanted there would have been no issue. Also in order to reduce my workload in that situation I should have engaged the autopilot as to have reduced my overall workload and widen the situational awareness of my scan.

Synopsis

A320 First Officer reported a wake turbulence encounter with a B757 contributed to a track deviation departing LAX.