ACN: 1435259 (1451 of 2908)

Time / Day

Date : 201703

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 60
ASRS Report Number.Accession Number : 1435259
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The tractor used to push this aircraft was one of the newer ones that picks the nose wheel off the ground and pulls it up a ramp into a carriage, I believe. The problem is the tug operator asks for a brake release in order to pull the nose wheel into the carriage. I asked her if this is the only way to attach this tug type for push. She said she knew of no other way. I specifically verified that she wouldn't move the aircraft in any other way. She said she would not. I then released brakes and stated, "Brakes are released, hold position, not cleared for push or movement." She rogered me.

The flight manual script for pushback doesn't appear to take into account this type of tractor hook up. This hook up procedure can and sometimes does happen before the Before Push Checklist and/or the Cabin Ready. Currently, we don't release brakes until [the] Before Push Checklist complete, Flight Service Coordinator reports cabin ready, AND a push clearance is received. If we wait for all these just to attach this type of tug, I believe it will have unintended consequences. If I mistakenly violated SOP in this case, it was unintentional, and I genuinely attempted to adapt to the situation with extra safety measures in the form of questions and added specific communication.

Synopsis

A320 Captain expressed confusion over brake release procedures when using a new type of tug.

ACN: 1434887 (1452 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZOA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Rain
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Center : ZOA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZOA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11624
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4970
ASRS Report Number.Accession Number : 1434887

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Flight Engineer / Second Officer
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 893
ASRS Report Number.Accession Number : 1434782

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

At FL340, aircraft encountered moderate to severe mountain wave which brought the aircraft unexpectedly to gaining about 12 knots airspeed within seconds of the encounter which brought the aircraft five knots into the red region on the speed tape. Recovered the aircraft by turning off the autopilot, increased altitude 120 feet high and throttles to idle. Brought the aircraft back to FL340 and placed automation of aircraft back on line. Received a call from the back stating that the two flight attendants fell to the floor (not in their seats) and a customer sat down on the floor while not in her seat either. After the flight, the flight attendants said they were alright, but asked them to file a report. Wrote up the aircraft overspeed and notified maintenance.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported moderate to severe mountain wave over the Sierra Nevada mountain range which caused an overspeed.

ACN: 1434606 (1453 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : RSW.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Ground : RSW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ground : RSW
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 6431
ASRS Report Number.Accession Number : 1434606
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

[Policy] states "aircraft taxiing in to the gate area shall give way to aircraft pushing back from the gate area."

During our pushback sequence from RSW's gate, tug driver had to come to a quick stop because [another air carrier] aircraft inbound to its gate--directly opposite [our] gate, failed to yield to our pushback as if we weren't even there. If the Flight Attendants were not seated, this stop may have caused injuries--in this case they were seated at their stations.

Synopsis

A320 Captain reported the tug operator had to come to an abrupt stop during push back when another air carrier aircraft failed to yield.

ACN: 1434279 (1454 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : EUG.Airport
State Reference : OR
Altitude.MSL.Single Value : 4400

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : EUG
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : EUG

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11928
Experience.Flight Crew.Last 90 Days : 145
Experience.Flight Crew.Type : 5065
ASRS Report Number.Accession Number : 1434279
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12890
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 9434
ASRS Report Number.Accession Number : 1434295
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Descending into EUG we contacted Cascade Approach. The ATIS was reporting SCT037 BKN044 with good visibility. The Controller offered us vectors to about an eight mile final and step-downs for a possible visual approach to 16R. The two of us had flown together into EUG and we were well aware and previously briefed the high terrain in the area. Initially we were cleared to 7000 ft and then down to 5000 ft. We noted the MSA of 5300 ft and so 5000 ft seemed reasonable. The FO and I believe that we did hear a "2500 ft" call while at 5000 ft. The FO was flying and had slowed to 210 kts. The Controller asked if we could see the field or if a bit lower would help. We were still solid IMC so I said we probably needed to do the ILS. Right after I said that we broke out and saw the airport and I told the Controller that a little lower might do the trick. He cleared us to 4400 ft with a slight right turn to 280. I believe we were inside of 20 miles at this point and the FO descended on autopilot to 4400 ft still at 210 flaps up. I noted the highest point depicted on the approach chart near us was 3154 ft to the south of us. Just as or after we leveled at 4400 ft we received a terrain warning. We could see city lights below but we were in and out of the broken cloud layer. It was a surprise but I believe the warning was "terrain, terrain, pull up, pull up". The FO did the full book escape maneuver and the warning didn't repeat. We reached maybe 5300 ft then leveled at 5000 ft as we reported the event to the Controller. Neither of us recall seeing any terrain depicted on the ND before the warning occurred.

I made a PA announcement to explain our abrupt maneuver while we were vectored to the ILS. As we intercepted the LOC we broke out of the layer and were cleared for a visual to 16R. After parking we explained the event further to the passengers and offered to answer any questions. The FO and I had a debriefing at the gate and also with the flight attendants in the hotel van. The controller confirmed that 4400 ft was the MVA where we were when he cleared us down. Looking at the VFR sectional on WSI I see there is a ridge in that area just west of MOBIL intersection. The highest points depicted are 2864 ft to the north and 3166 ft to the south.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported receiving a GPWS terrain warning on a night approach to EUG while being vectored for the ILS.

ACN: 1433943 (1455 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC
Altitude.MSL.Single Value : 24000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : CHPTR1
Airspace.Class A : ZTL

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 900 (CRJ900)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : BANKR1
Airspace.Class A : ZTL

Aircraft : 3

Reference : Z
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 700 ER/LR (CRJ700)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : BANKR1
Airspace.Class A : ZTL

Person

Reference : 1
Location Of Person.Facility : ZTL.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 25
ASRS Report Number.Accession Number : 1433943
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

Aircraft X was flashing handoff descending to FL240 on the CHPTR STAR to JQF (a satellite airport of CLT). Aircraft Y and Aircraft Z were flashing handoff (all three from the same sector) on the BANKR Optimum Performance Descent (OPD) STAR to CLT. The CHPTR STAR underlays the BANKR OPD STAR but there is no requirement for these aircraft to be sequenced from the previous sector. As the "descend via" clearance is needed to be given in my airspace to make this procedure work (my airspace is close to both airports). It is imperative that there is no traffic under the OPD arrival (especially from the SAME SECTOR). In a world of PBN and OPD STARs, this should be worked out and required but is not.

Require the preceding sector (that already is required to do Time Based Flow Management (TBFM) and sequencing for CLT) to have all CHPTR STAR arrivals sequenced with the BANKR OPD STAR.

Synopsis

ZTL Controller reported inadequate spacing between two arrivals near CLT and procedures should be revised.

ACN: 1432851 (1456 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZTL.ARTCC
State Reference : GA
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : CHSLY2
Airspace.Class A : ZTL

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1432851
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1432999

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were instructed to descend via the [CLT] CHSLY2 RNAV arrival landing 18L. Prior to the top of descent, we completed an arrival verification per SOPs. It correctly depicted the appropriate restrictions found on the chart. At some point during the managed descent, at an altitude above FL270, 280kts, and with the autopilot on, the MCDU and PFD displayed the incorrect crossing at BURRZ. It had us leveling off below FL240. I manually made BURRZ a hard altitude of FL240. The airplane then leveled off as planned and we both again verified the arrival. We intervened, [employed the CAMI (Confirm, Activate, Monitor, Intervene) procedure] and did not bust any restrictions. This is a known Airbus anomaly. This is a result of the S7A software glitch. I was aware of this because of a previous fleet bulletin and was familiar with what to watch for. However, not all crews are aware of this. We remained alert and CAMI allowed us to intervene. It is my suggestion that [our airline] discontinues use of the CHSLY2 RNAV arrival until a permanent solution is achieved. In my opinion, with regards to safety, it's not worth the risk of a possible deviation.

Narrative: 2

Climb/Descend via clearances are semantically challenging, labor intensive, and often do not work. I think they should be discontinued or at least make them easier to fly. There is a history of repeated failures on every [CLT] STAR arriving from the northeast.

Synopsis

A320 flight crew reported they had to intervene manually to avoid an altitude deviation on the CHSLY2 arrival into CLT. Crew stated this was a known Airbus anomaly.

ACN: 1430480 (1457 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 1201-1800

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1430480
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I smelt odd fumes for about twenty to thirty minutes and also after confirming the smell with the other working flight attendants, after the "B" flight attendant reported the odor to the flight deck, I reported it also. The odd fume went away about thirty minutes after reporting the issue to the captain. In the last hour of the flight I started feeling abnormal. I had a pounding headache, felt dizzy, fatigue and weakness in the legs, my stomach becoming tight and upset, I felt shortness in breath and chest pain, and it felt like day two of a very bad "flu-like" bug in less than an hour. I immediately reported the issue to the captain, and when we landed I called crew service center about the issue, and discussed the injury with my flight service manager. Due to this injury I could not perform my core duties, specifically evacuations or announcements. I confirmed the same symptoms with my flight attendant crew members and we went to the emergency room for immediate treatment at the hospital. 

Based on what happened to me and the test results I feel this is chemical fume injury in carbon fume family, specifically carbon monoxide. Carbon monoxide is a by-product of the incomplete combustion of carbon-containing materials. Aviation fuel contains carbon and is a ready source of carbon monoxide when burned. Carbon monoxide is truly a hidden menace because by itself, it is both a colorless and odorless gas. An individual would not be aware of its presence until symptoms developed, or during treatment it was determined exposure had occurred. The least desirable situation would be incapacitation. The true problem comes when exposure is so gradual that you don't perceive it. We perceived fume, reported the issue, and are now injured.

Synopsis

A320 Flight Attendant reported that cabin fumes became unbearable during the flight, resulting in illness.

ACN: 1430267 (1458 of 2908)

Time / Day

Date : 201607

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1430267
Human Factors : Troubleshooting
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Off Duty
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1430398
Human Factors : Troubleshooting

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1430420
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Evacuated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Prior to boarding, we were under maintenance due to a possible hydraulic leak. The mechanics noticed the fluid on the ground, had the flight deck run pressure checks and then later gave us clearance to operate within 15 minutes. Myself, nor the B & C FAs felt safe operating the flight with knowing that there was a (major) hydraulic leak. The captain said that maintenance gave the ok saying we were good to go. I was worried that the AC wasn't properly evaluated while under maintenance and that if the flight deck spoke up more, this incident could've possibly been prevented. 

Just as the captain announced: Flight Attendant (FA) prepare for take-off, there was a LOUD shrieking noise followed by a very strong fume odor. I notified the flight deck and made an announcement to the passengers. The captain initiated an evacuation of the aircraft. At that point, it was too late to do a full cycle of evacuation commands because the passengers were already in the galley ready to exit the aircraft. I used my hand to hold them back and commenced with come this way, leave everything.

As a suggestion, I would recommend that the commands be amended to incorporate the old commands of stand back until the slides fully deploy.

I also don't know the recommendation for this, but we need to be sure that passengers are willing to stay at the bottom of the slide to help people off and away from the aircraft. The majority of passengers ran away in fear. At least, two times I found myself asking more people to help at the bottom.

Narrative: 2

[Report narrative contained no additional information.]

Narrative: 3

[Report narrative contained no additional information.]

Synopsis

A320 cabin crew reported a failure in the hydraulic system while taxiing resulted in smoke in the cabin and a subsequent call by the Captain to evacuate.

ACN: 1429700 (1459 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1429700
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were ready to push off the gate and the tug driver attempted to contact us via his headset. The headset he was using was intermittent. With lots of static and some squealing. He seemed distressed about the improperly working headset and seemed concerned about pushing as soon as possible. Apparently there was another aircraft in range for the gate. Once communications were established. We completed the before start checklist and called ramp for pushback clearance. Ramp cleared us to push back to spot 14. I instructed the tug driver to push back using standard communication. "Cockpit to ground brake released cleared to push spot 14". He responded with some short statement like roger. A few moments later the aircraft began to move backward from the gate. Everything appeared normal. Then there was some broken static and voice on the headset. Followed by the tug driver screaming into the head set "set brakes!!! Set brakes!!!! Stop!!!!" I immediately applied aggressive braking to stop the aircraft and set the parking brake. The tug driver told me "the tow bar was faulty". I forced him to go get a properly functioning headset which he did. That only took a few minutes. After asking him to clearly explain what happened. I discovered the tow bar was not properly secured to the nose gear. The aircraft had been allowed to roll backwards unattached to the tow bar and tug. I insured no one on the ground had been run over and the First Officer communicated with the ramp controller to insure we did not strike any vehicles.

I contacted the cabin crew to see if everyone was ok. One of the Flight Attendants had been thrown to the floor when I stopped the aircraft. She had hit her head on an armrest and was applying ice to it. Otherwise she was ok and no one else was hurt. The tug driver and operations supervisor told me this had happened once before recently and were trying to talk me into re-attaching the tow bar and pushing back. After a few moments to reflect. We decided to call the Dispatcher and get them involved in the decision process. They conferred with Maintenance Control and we collectively decided to return to the gate. Have the landing gear inspected by maintenance and insure that the tow bar and crew were operational and safe for another pushback attempt. Once these actions were completed we pushed back and continued the flight without further incident. 

There needs to be a shift in focus in our company. It seems clear to me that the overwhelming pressure to push on time has effectively forced the ground personnel and operations leaders into making mistakes that can easily become serious injuries. We need to train our personnel to put safety above all else. Including on time performance.

Synopsis

A320 Captain reported a break away from the tow bar during push back, requiring the aircraft to be stopped abruptly when the tug driver called for the brakes to be set, with the aircraft in motion.

ACN: 1429176 (1460 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 125

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 223
ASRS Report Number.Accession Number : 1429176

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Upon arrival in the terminal area, as expected and briefed, we were in continuous moderate turbulence from 14,000 MSL to the surface. We had briefed the FAs to be seated early and informed the passengers early. We also briefed the severe windshear conditions, other than instrument alerts and warnings requiring us to abandon the approach. The captain elected to use Flaps 3 as suggested in the FM. I briefed the possibility of a high pitch attitude tendency using Flaps 3, as well as, the visual illusion in the 10-7 pages specific to this runway. We both discussed the fact this would be a safe firm landing and would not try to finesse it because of the wind conditions and pitch tendency specific to the configuration and runway. Automation was used to the [maximum] extent possible and we followed the FM guidance for precautions in windshear areas (Minimum ground speed, Flaps 3, additives on approach, etc.). We also made a plan for if a missed approach was required as we had very little fuel if the need became a diversion to our alternate (something we had discussed with Dispatch previously and would put us at 30 min of fuel remaining at the alternate). We determined it was likely more prudent to attempt a second landing if it became necessary and not proceeding to the alternate unless needed for airport operations being suspended at our destination OR the first approach being abandoned for unsafe conditions. The flight was dispatched with full tanks, allowing no added fuel for more contingencies at arrival.

The approach proceeded as expected and we stabilized early using automation, additive speed then Mini-GS. The captain kept the autopilot and autothrust on until approximately 200 ft when he disengaged the autopilot. At approximately 100-125 AGL we received a reactive windshear warning in the final stage of settling the aircraft to the runway. The energy state on the aircraft and proximity to the runway convinced me it was the safer action to complete the landing from this position. The Captain and I agreed at the gate we both felt the same regarding the aircraft position and energy condition and that it was the correct decision. After the flight during our further debrief, we reviewed the Windshear guidance even further and I realized there doesn't exist any relief in the FM for continuing an approach with a reactive windshear warning and the operation thus was in violation of the SOP. The Captain and I discussed that in essence his emergency authority was used in that moment to bring the flight to a safe conclusion considering the position of the aircraft when encountered and the options available (fuel state) if a missed approach was completed.

Synopsis

A320 First Officer reported a windshear alert just prior to touchdown. They elected to land rather than execute the windshear recovery as required by SOP.

ACN: 1429174 (1461 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 19000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18865
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9045
ASRS Report Number.Accession Number : 1429174
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 545
ASRS Report Number.Accession Number : 1429186
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We received a: "Wheel N/W steering fault", while en-route to ZZZ. The QRH page was reviewed and discussed with the First Officer (FO). I sent a message [maintenance control] and asked for additional guidance since cycling the anti-skid was not necessarily ideal in flight. However, [maintenance control] did not respond despite repeated requests. We were trying to wait for [them] to weigh in since the QRH had a caution note, which stated it was possible to lose anti-skid functionality if any braking errors existed. (We did not see or discover any Brake and Steering Control Unit (BSCU) or brake failures). Essentially our options were: 1). Land without Nose Wheel (NW) steering functionality and stop straight ahead on the runway, then reset on the ground or 2). Re-cycle the anti-skid switch in flight with the caution in mind. 

We mutually decided to re-cycle the anti-skid switch per [QRH], just prior to final approach. The reset appeared to be successfully as the NW steering fault disappeared. However, upon landing between 100kts-80kts the NW steering fault re- manifested itself. At this point, I assumed control of the aircraft and pointed the jet toward the high speed turnoff using rudders for steering while slowing the jet. We successfully cleared the runway and I used a combination of engine thrust and differential breaking to ensure the tail section was clear. 

After coordinating with ground control, ATC called company via a land line and arranged for a tug to meet the aircraft. A passenger announcement was made and the tow-in checklist completed. We were then towed to the gate without incident. The tug driver told us during the tow-in, the hydraulic service panel was open. I understood that meant it was open the entire flight. Arguably, maintenance should have closed the access panel and or the push back crew should have seen it during their inspection.

After parking at the gate, our checklist was completed and I followed up with an inspection of the hydraulic service panel. It was open and unsecured as reported. I then placed a call into [maintenance control] and queried them as to why they did not respond to my request for guidance. I was told they were on the phone with other issues and did not have time to respond. Moreover, he stated that [maintenance control] normally does not have the capacity to help in these situations like they did in days past. In my view, items which involve safety of flight should be paramount and receive additional support when requested.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported that while in flight they received a "Wheel N/W steering fault", possibly caused by an open hydraulic service panel.

ACN: 1428341 (1462 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 24000

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Make Model Name : A320
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use.STAR : SEAVU2
Airspace.Class A : ZLA

Person

Reference : 1
Location Of Person.Facility : ZLA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 0.5
ASRS Report Number.Accession Number : 1428341
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

An aircraft cleared via an OAPM (Optimization of Airspace and Procedures in the Metroplex) routing aircraft said there was a mismatch between his database and what the chart says. Aircraft has SEAVU.LOVIN.KRAIN. This just populated when he input SEAVU STAR. Chart does not have LOVIN (may be spelled LUVIN). Update charts.

Synopsis

ZLA Center Controller reported an aircraft was cleared via an OAPM STAR, but the aircraft had an extra fix in their database compared to the chart.

ACN: 1428280 (1463 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 20000

Environment

Flight Conditions : Mixed
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Engine Air Anti-Ice
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 478
ASRS Report Number.Accession Number : 1428280

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Our dispatcher and the Maintenance Coordinator were very soon on line, and after describing our situation maintenance directed me to cycle the engine anti-ice pushbutton several times and to apply pressure to the edges of the switch (assuming possibility that the actual switch was the culprit). The fault remained and the Maintenance Coordinator requested we hold on the line for a moment while he sought further consultation. In the meantime the Dispatcher and I discussed the aforementioned options. ZZZ1 came up briefly as a mid-flight divert for a repair, but with maintenance back on the line we all came to the same conclusion that returning to ZZZ was the safest and best option. Sometime during this discussion I realized that we would be facing an overweight landing in ZZZ. We estimated we would be over our maximum landing weight by approximately 6,000 pounds. With favorable weather conditions and a returned and satisfactory landing distance report, we were all comfortable with continuing without burning fuel down to below maximum landing weight. That decision made, I signed off from ZZZ Radio and together with the First Officer we started the plan to return to ZZZ. We again notified the flight attendants of our plan and I made a very descriptive PA to the passengers of our unpopular, but necessary decision to return.

We informed ATC of our intentions to return to ZZZ with the added bonus and challenge of avoiding potential icing conditions. With a front stretching from lower Mississippi to northern Alabama and Tennessee, this presented a bit of a puzzle: do we continue back towards the east and pick up an east-side arrival to ZZZ or do we attempt to find an adequate path through the front and pick our way through? We decided that we could turn towards the south and start to look for a path from the west to join an arrival. We had plenty of fuel and time, but we were looking for a solution that would provide the safest route that would also have the least time impact on our passengers.

ATC worked very well with our particular requests and eventually vectored us for an arrival. We notified them that we would need to stay at FL310, where we were above the cloud layer, until we could safely make basically a VFR descent and avoiding the necessity of using our engine anti-ice. In the meantime, the First Officer read the overweight landing procedures and considerations from the QRH to prepare us for landing in ZZZ. Once clear of the potential threat of icing in the clouds, we continued the arrival to an uneventful landing.

Maintenance and several customer service personnel were awaiting us at the gate. After a quick repair of locking open the engine anti-ice valve and a maintenance sign-off, thorough discussions with the duty pilot and dispatcher addressing the situation and potential conflicts with flight duty periods, block time limitations and flight attendant duty periods, we were refueled, issued new paperwork and dispatched to continue the flight to [destination].

Synopsis

An A320 Captain reported experiencing a failed engine anti-ice valve and returned to the departure airport for an overweight landing.

ACN: 1428092 (1464 of 2908)

Time / Day

Date : 201703
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1428092
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1428110
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

While on a visual approach at around 2000 feet I called for gear down and it failed to extend. The First Officer followed the ECAM checklist and recycled the gear handle. The gear still indicated not locked. I decided at that point to go-around and work the situation. We received vectors west of the airport and I gave the aircraft and directions to my First Officer and contacted Maintenance. Maintenance coordinated with Maintenance Control and confirmed that we should continue with a manual extension. I completed the extension and had a good down and locked indication. The FO remained with ATC and flying the aircraft using automation and got us on a downwind. I requested towing assistance before landing. I also briefed the flight attendants and made a PA to the passengers. Once complete I took the aircraft and continued to an uneventful landing. We stayed on the runway and got towed in from there.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported the gear failed to extend, so they had to use the manual extension procedure.

ACN: 1427663 (1465 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 23000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : First Aid Equipment, with Medical Kit & Defibrillator
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1427663
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1428264
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were a little more than halfway into our flight when our Flight Attendant (FA) called concerning a semi responsive 57 year old female passenger foaming at the mouth and vomiting. Under the request of the patient, the patient's sister gave her a nitroglycerin tablet. There were two doctors onboard helping the FAs with the patient. I called ARINC and established a phone patch with MedLink and dispatch. We had to wait at least 10 minutes before the Medlink doctor was able to get to her phone. In the meantime, the two onboard Doctors were suggesting that we land soon because of the possibility of the patient having a heart attack. In my mind I was planning on a possible diversion to [nearby airport]. When the Medlink doctor's finally came online, her accent was hard to understand and the squeal on the radio added to the communication issues. Another issue was the fact that neither onboard doctor could get a blood pressure reading. They explained to me that the quality of the blood pressure cuff and stethoscope was in question. The cuff was too tight and they were not able to hear through the scope, therefore they could not get the pertinent vital signs. 

After relaying the patient's current condition to the Medlink Doctor, the doctor said to continue to ZZZ. So acting in accordance with our FOM we continued to ZZZ. I was having trouble understanding one of the requests from the Medlink Doctor. After asking a few times for the doctor to repeat the request, and between myself and my First Officer (FO) we still could not understand her instructions. I ACARS'd my dispatcher to see he was able to understand her instructions. He thought she was saying 2 Benadryl tablets. Which in fact was what Medlink was ordering. The onboard doctors thought this was an odd request and asked to speak personally to me on the FA's phone. Under the request of the onboard doctors, I asked the Medlink doctor why the Benadryl? She said it was for the vomiting. The doctors still thought it was a strange thing to order since the passenger was throwing up. I was making every attempt to make sure the medlink had complete understanding of the condition and symptoms of our passenger. I still thought we had some misunderstandings and I wanted to do everything I could to avoid a mistake. There were a few times I asked my FO to listen along with me. I also gave ATC the heads up that we were busy talking with Medlink in case we missed a call. He understood and said no problem.

During all of the stressful communications, we were cleared direct to ZZZ, then given a crossing restriction; cross 85 miles southwest at FL230. I watched my FO dial in FL230 into the ALT SEL and said "FL230 blue". I watched him then build a fix off the VOR and add the 230 into the altitude box. My oversight was not seeing that he had programmed the wrong distance into his PFD. So basically our fix and our top of descent arrow was incorrect. Also the ATC instructions seemed to have us at a low altitude far from the airport so I asked ATC to confirm the restriction which he did. All of this was happening while I was trying to relay a lot info between Medlink and our doctors. We also [advised ATC of] a medical emergency and notified dispatch. I wanted to keep our speed up and get on the ground. When things calmed down my FO then noticed he built the wrong fix and started down in attempt to make the restriction. I was back on radio duties and told ATC of our mistake. He said not a problem. It was in the early morning hours and hardly any traffic.

In the future, I am going to compartmentalize my actions and treat the situation like we treat our emergency procedures when we work our way to the status page on the ECAM. The flying pilot says "Stop ECAM actions...after takeoff checklist". I will brief my FOs to do the same thing. Get my attention, then and make sure I watch and confirm the fix was built correctly and all restrictions were entered not just the correct altitude. 

Also this is not the first time I have had a medical professional complain about the poor quality of our stethoscope and blood pressure cuff. I will talk to my chief pilot and make him aware of this issue. Things would have gone smoother and there would be fewer distractions if we had vital signs to work with.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported they had a medical emergency onboard, difficulty understanding the Medlink Doctor and the Doctors onboard complained about the poor quality of the stethoscope and blood pressure cuff.

ACN: 1426333 (1466 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZME.ARTCC
State Reference : TN
Altitude.MSL.Single Value : 28000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZME
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZME

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1426333

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We were dealing with continuous light and occasional moderate turbulence for most of the flight. ATC kept telling us other altitudes were better (started at FL340), so we tried FL320 and FL300, and eventually FL280. Had the Flight Attendants (FA) seated, seatbelt sign was on, and told the Passengers multiple times to stay seated with seatbelts. Flew .76M for turbulence penetration speed, and had the engine heat on (VMC, but wanted the Ignition on). Many reports of moderate turbulence across southeast, but nothing worse. Finally somewhere east of the MHZ VOR ATC said it would smooth out around MHZ. Instead, about 30 east of MHZ, experienced rapid onset of severe turbulence. Aircraft was on edge of control, and moving violently. Requested immediate change in altitude, was told FL340 was best bet at that point. Started immediate climb. Turbulence was severe for several minutes, all the way from FL280 to FL300. Airspeed deviations of plus and minus 30 knots between FL280 and FL300. Moderate until almost FL340. Once ride was better, checked in with FA's. No one injured, either FA or Passengers. Never exceeded VMO/MMO either, luckily. Aircraft felt normal, continued with descent and approach. 

By coincidence, both my First Officer and I used to fly [as] Hurricane Hunters. We both felt that this turbulence was the worst (or at least the longest duration of severe) somewhat equivalent to a hurricane penetration.

Synopsis

A320 Captain reported encountering tubulence so severe he compared it to his previous experience in planned hurricane penetration flights.

ACN: 1426030 (1467 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 3100

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 19L
Flight Phase : Final Approach
Route In Use : Vectors
Route In Use : Visual Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24550
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 15790
ASRS Report Number.Accession Number : 1426030
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 12856
ASRS Report Number.Accession Number : 1426046
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

ATC vectors to ILS 19L. On base leg vector at 5000' we were given clearance to descend to 3100' on a heading of 160 degrees to intercept 19L localizer and once established cleared to descend to 2800' until SHAKE. We were a couple miles southwest of BERKS (15 DME ISIA) when given turn to intercept. At about 3100' just prior to LOC intercept we received a GPWS warning for terrain. I disconnected the autopilot and executed the GPWS escape maneuver. We climbed to about 5000' and continued to intercept the localizer and reloaded the approach. We notified ATC and once clear of the terrain warning and acquiring the runway, we continued the approach for a normal landing. We made visual contact with the airport just outside of SHAKE.

Narrative: 2

3100 feet near BERKS is approximately 1000 feet above the terrain but well below the 5000 foot altitude on the approach plate. We were surprised to get the warning but complied as we were night IMC. The whole event lasted just a few seconds.

Synopsis

A320 flight crew reported they received a GPWS terrain warning on a night visual approach to SFO Runway 19L.

ACN: 1425483 (1468 of 2908)

Time / Day

Date : 201702

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : GABRE1
Airspace.Class B : LAX

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1425483
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1192
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 315
ASRS Report Number.Accession Number : 1425356
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Chart Or Publication

Narrative: 1

ATC cleared us to fly a 030 heading to join the GABRE1. I had never flown this SID. I had my IPAD zoomed in on the Runway 6L/R departure side so I wouldn't miss the charted headings. This put SLI out of view on the IPAD. I mistakenly asked the First Officer to sequence the FMGC between GABRE and FOGEX.

Narrative: 2

During departure off runway 6R at LAX, GABRE1 departure, ATC issued "turn left 030 and join the Gabre departure." This was the first time for both pilots performing this SID and the first time departing this runway for the FO. LAX had a rain storm and was departing east. Once instructed to join the departure on the 030 heading, I extended the inbound radial to FOGEX and inserted it into the FMGC. With concurrence from the Captain I executed it. ATC queried our course and advised we were supposed to intercept the Seal Beach VOR 346 radial northbound. Upon review both pilots had the departure zoomed in on their Ipads and did not have the SLI displayed.

Synopsis

A320 flight crew reported deviating from the published procedure due to not having their IPAD/EFB set to display the complete procedure.

ACN: 1425266 (1469 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : PHX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : PHX
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1425266

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Taxiing for takeoff at PHX runway 7L on E taxiway. As we became number one for takeoff we were proceeding past E4 taxiway to turn left on E3 taxiway to hold short for takeoff instructions. As we were crossing over the E3 threshold tower told us to stop to let another aircraft go ahead of us. I figure it was an aircraft behind us because the nose of my aircraft was half way through the E3 taxiway. Then I saw an aircraft on D taxiway making the left turn from D to D1 to E3 right in front of us at a high rate of speed. I told my First Officer (FO) I think he may clip our nose with his wing tip. My FO said his right main wheels were so close to the taxi way lights that he thought he was going to take them out. The wing tip of the aircraft missed us by 5 feet. After they had passed the tower called thanked us for stopping and apologized for the late call for stopping.

They should have stopped and said they could not get through because we were blocking the taxiway. Had they hit us it would have damaged the aircraft.

Synopsis

A320 Captain reported another aircraft passed very close to the front of the aircraft while holding at a taxiway intersection.

ACN: 1425265 (1470 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737-800
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Wingtip
Aircraft Reference : X

Component : 2

Aircraft Component : Wingtip
Aircraft Reference : Y

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1425265
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1425267
Human Factors : Situational Awareness

Person : 3

Reference : 3
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1425600
Human Factors : Situational Awareness

Person : 4

Reference : 4
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 87
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1425592
Human Factors : Situational Awareness

Person : 5

Reference : 5
Location Of Person.Aircraft : Y
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1425751
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Aircraft
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Cockpit preparation and boarding proceeded normally prior to this event. Upon completing the Before Start Checklist (above and below the line), and verifying that both pushback and cabin crew were ready for push, I called ZZZ ground and requested pushback from [the] gate. Ground cleared us to push with "reference Aircraft Y off your right." I looked out my right window (towards the right #2 wing) and spotted a 737 taxiing from West to East on taxiway Charlie. I noted no other aircraft behind [it]. I forwarded the ground clearance instructions to [the] Captain, who then relayed our clearance to the pushback crew. The push commenced and shortly after we were cleared to start engine one. Captain instructed me to start engine one and I began that procedure. Near the end of the engine start cycle, as we were parallel with taxiway Charlie (nose facing East), a very noticeable impact was felt and heard. The ground crew stopped our aircraft immediately, and shortly thereafter I could see a 737 parallel with us on taxiway Charlie stopped at our 10-11 O'clock position. It was immediately evident that the top of the winglet from Aircraft Y was missing, and with confirmation from the pushback crew, we were aware that some portion of our left wing impacted Aircraft Y. We shut down engine one immediately and remained on APU power. Aircraft Y was the first to notify ground control of the impact. Both aircraft remained stationary while airport operations vehicles and the fire department arrived on scene to document the incident. Initially no fuel leaks were reported. After several minutes while waiting for the investigation to complete, we were informed by the pushback crew that there was a fuel leak under the left wing. We deplaned the aircraft shortly thereafter from the 1R door using air stairs and shut the aircraft down. No injuries were reported from passengers or cabin crew at this time. The leak was later identified as a fuel drip from the #1 engine drain mast by a mechanic and was not associated (as far as we could tell and in the opinion of the mechanic) with the impact. Once Aircraft Y was towed away, [the] Captain and I re-powered the aircraft and occupied our duty stations while the aircraft was towed to a remote pad. Under guidance from the mechanic on scene, we started engine #1 for several minutes to stop the fuel leaking from the drain mast. After this successful procedure, [the] Captain made a logbook entry and we left the aircraft powered up and in care of the mechanic.

The exact cause of this incident is unclear to me at this point, therefore it is hard for me to speculate on prevention.

Narrative: 2

[Report narrative contained no additional information.]

Narrative: 3

[Report narrative contained no additional information.]

Narrative: 4

[Report narrative contained no additional information.]

Narrative: 5

[Report narrative contained no additional information.]

Synopsis

The flight crews of an Airbus A320 and a Boeing 737 each reported that their wingtips collided on the ramp during night operations.

ACN: 1422677 (1471 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17862
Experience.Flight Crew.Last 90 Days : 92
Experience.Flight Crew.Type : 9669
ASRS Report Number.Accession Number : 1422677
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

While reviewing the maintenance status of our aircraft I saw there were multiple instances of FCU (Flight Control Unit) failure with multiple resets that would temporarily fix the issue only to have the FCU fail again. After consultation with my dispatcher, Maintenance Control, my flying partner and the FAA inspector riding with us to ZZZ1, I refused the aircraft since we were scheduled to fly to ZZZ1 where the weather was forecast to be at minimums. Maintenance arrived at the aircraft and said they had a replacement FCU and immediately started pulling circuit breakers in front of the FAA inspector riding with us to ZZZ1. At this point I left the cockpit to give the two mechanics room to work on the FCU and went into the cabin to make a PA to the passengers and let the CSRs know we would be delayed for at least an hour while the FCU was replaced. We finally departed for ZZZ1 almost 1.5 hours late after maintenance had replaced the FCU and successfully run their tests. I did notice that during their test, there was a Red Autopilot Failure ECAM present for much of the time.

43 minutes into the flight at cruise in smooth air, Autopilot 1 disconnected. I confirmed that neither one of us had accidently bumped the side stick and neither one of us had our hands or any objects near the stick. I reconnected AP1 and it remained engaged for about 10 more minutes when it again disconnected. I again reconnected AP1 and it remained connected for about 5 minutes when it again disconnected. This time when I tried to reconnect it I received a FCU channel 2 failure and Autopilot 1 failure. I gave the aircraft to my first officer since I had also lost FMGC1 with no map display. Our display showed -FD2 and with my FD on 2FD2. The first officer's FMGC remained normal and AP2 connected successfully. The ECAM had no action items so I referred to the QRH which did have procedures for airborne system resets. Before attempting the resets I established a phone patch with Maintenance Control to discuss the procedure in light of the past history. Maintenance Control agreed with attempting the reset on FCU2 on the 141VU panel. I confirmed the CB location and had my jump seater cycle the CB per Maintenance Control's instructions. The reset was unsuccessful. We then performed a reset on FMGC1 on 49VU to regain FMGC1. This was only partially successful and my FD display returned to 1FD2 but the FMGC remained in independent mode. I conferred with my flying partners on our options. Taking this aircraft on to ZZZ1 with weather at minimums was not feasible. We elected to divert and with the dispatcher's concurrence proceeded to ZZZ2, reviewing the overweight landing checklist and landing data provided by our dispatcher, where we landed about 11000 pounds overweight. Since my FMGC was only partially usable I elected to let the F/O make the landing. I have had previous experience with FMGC failures where the FMGC fails totally at the last minute creating un-needed confusion in a critical phase of flight. Since the F/O's systems were normal and he was a very experienced Airbus pilot, I had no hesitation letting him perform the landing. Landing and rollout were normal. A ZZZ2 mechanic met us at the gate and printed out about 12 inches of maintenance failures from the MCDU and made sure he understood exactly what the failures were. We proceeded to the spare aircraft that was towed from the hangar right behind us as we were taxing in.

When we arrived at the new gate, the CSR had to manually input my file number and the F/O's to check us off. When I pulled up the release information it showed me fit but showed NO next to the F/O. We had to manually input the FIT information and eventually called the dispatcher to get it accomplished. As we were getting ready to push, the release showed a new Release 3 which I had not signed off, and discovered the Dispatcher had made a new release without informing us. I had to reconnect my IPAD to download and sign the new release. About 1.5 hours later, after having to wait for a new Release we taxied out. When the final weights arrived, the weight manifest did not include the jumpseater. We had to call the station with the jump seater's credential information to have him included in the weight manifest. The CSR had failed to do this at the gate. After multiple calls to the station and to dispatch, after almost 30 minutes off the gate, we finally had it corrected.

We proceeded to ZZZ1 where the weather had continued to get worse. We had two alternates. Eventually one of them went below minimums so it was dropped. We discussed our options should the approach into ZZZ1 not be successful and elected, rather than attempt another approach at ZZZ1, we would proceed on to the other where the weather was at CAT1 minimums. We successfully flew a CAT2 autoland into ZZZ1 with weather right at minimums. This aircraft was not autoland limited into ZZZ1 where as the original one was. We arrived at the gate in heavy snow, almost four hours late. Our FAA jumpseater told us we did a great a job handling all of the issues.

To top off the night, we had to take Cabs, including flight attendants from another flight waiting at the curb, to the hotel as the roads were snow blocked and the hotel van was not operating. Our cab barely made it to the hotel dodging sliding cars stuck in the snow.

Synopsis

A320 Captain reported refusing an aircraft with a history of FCU failures. The FCU was replaced by Maintenance, but failed enroute and the crew elected to divert due to low visibility at their destination.

ACN: 1421832 (1472 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Powerplant Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6842.9
Experience.Flight Crew.Type : 5301.2
ASRS Report Number.Accession Number : 1421832

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13016.75
Experience.Flight Crew.Type : 8376.78
ASRS Report Number.Accession Number : 1421577

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise First Officer (FO) noticed that the Number 2 Engine Fire push button light on the overhead panel was illuminated, we had no other indication of a fire, no ECAM, No other lights were illuminated elsewhere which we both thought to be highly unusual. The light was on steady so I called for the QRH engine fire checklist which [the FO] had already gotten ready, then the light went out. It then started to flash randomly off and on. I tested the Number 2 engine fire system and it tested normally.
At that point I was thinking we had a failure within the detection system or warning system and told [the FO] I think we should do the 3 way call with dispatch and [maintenance control]. He agreed and so I gave [the FO] the Pilot Flying (PF) duties and I worked on the phone patch to dispatch. I also sent Maintenance a quick note through ACARS about our situation. It took a few minutes to get the phone patch together, and during that time [the FO] and I both agreed the engine was most likely fine, that we had some kind of detection/warning failure and that a diversion would probably be the best decision and ZZZ was only 50 to 60 miles away.

The ARINC phone patch was established and I informed [maintenance control] and dispatch of our issue. After a few minutes [maintenance control] came back with a response that his only guidance he could give us was to follow the QRH. He had nothing to offer other than that. I was reluctant to follow that idea as there was no ECAM, no other lights or warnings, just an errant flashing fire light. I glanced over at who was monitoring the call and I could tell he was also reluctant with that idea but we both knew it was the most conservative approach to the situation.

I told [the FO] head for ZZZ. I confirmed with [maintenance control] and dispatch about following the QRH engine fire checklist, I'm sure they could hear my reluctance to their guidance. I was about to tell [the FP] that we should consider running the checklist and shut down the Number 2 engine. I happened to glance down at the center pedestal and noticed the red Number 1 engine "fire" light on the engine start and ignition panel was now on and pointed it out. It also flashed on and off a few times. Again, there were no ECAMS, no other lights or warnings. At that point I decided to disregard the [maintenance control] guidance, told [the FO] let's use my emergency authority, disregard both lights and land in ZZZ. I then informed [maintenance control] about the new light, and told them I would use that authority to disregard the [maintenance control] guidance and that we would be leaving both engines running and plan a normal landing. [maintenance control] signed off and dispatch and I finished up with the diversion info such as landing performance, NOTAMS, weather, etc. [my FO] was very proactive and called back to the FAs and informed them of the divert and to close up everything in the back while I finished up with the phone patch. I made an announcement to the passengers informing them of our maintenance issues and our need to divert. I joined back up with [the FO] as we were descending into ZZZ. I set up the [Flight Computer], glanced through the diversion guide, and [the FO] landed. Although we didn't request emergency fire equipment, it was present during the landing.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

The flight crew of an Airbus A320 reported they noticed a Number 2 Eng Fire push button light illuminated on the overhead panel. No other indications of a fire.

ACN: 1421778 (1473 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : S56.TRACON
State Reference : UT
Altitude.MSL.Single Value : 11000

Aircraft

Reference : X
ATC / Advisory.TRACON : S56
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : SLC

Component

Aircraft Component : ILS/VOR
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Facility : S56.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Traffic Management
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 3.0
ASRS Report Number.Accession Number : 1421778
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Training / Qualification

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Airspace Structure

Narrative: 1

Aircraft X was on a vector to intercept the runway 17 localizer. They were issued a 140 heading to join. Aircraft X shot through and was seen descending into the 110 foot minimum vectoring altitude. The instructor took over and issued a heading to reintercept. Aircraft X reported in a 30 degree bank. The aircraft was still allowed to continue descending below the MVA.

Synopsis

S56 TRACON Controller reported an A320 on a vector to intercept a Localizer flew through the Localizer course placing the aircraft below the MVA.

ACN: 1421775 (1474 of 2908)

Time / Day

Date : 201702
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 25000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Direct
Airspace.Class A : ZLA

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Direct
Airspace.Class A : ZLA

Person

Reference : 1
Location Of Person.Facility : ZLA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 26.0
ASRS Report Number.Accession Number : 1421775
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Narrative: 1

Aircraft X was put in holding at JLI for sequencing to LAX. I gave them holding as published. The published holding is left turns and he made right. I was told to give them one turn. So after make half of the holding pattern I gave Aircraft X a right turn direct VISTA and maintain 28000 feet which they read back. I had him stopped them at 28000 feet because Aircraft Y was climbing on the SID out of SAN to 27000 feet. I then assigned Aircraft X 250 knots. They read that back and did say knots and not just 2-5-0. A moment later I saw Aircraft X descending. I told them to stop their descent at 25000 feet. They didn't say anything. I stopped Aircraft Y at 24000 feet which he was at 23800 feet at the time and said they would go back down. I went back and stopped Aircraft X at 25000 feet again. I turned Aircraft Y 10 left and expedite his turn. I turned Aircraft X to a heading of 160 which I had to say twice. I called traffic to Aircraft X. No response. I called traffic to Aircraft Y and he said traffic in sight. I went back and called traffic to Aircraft X again. After they were pulled apart I gave Aircraft X direct VISTA again and down to 24000 feet. I then gave him the brasher warning and had given him the phone number 3 times. Almost every clearance I gave to Aircraft X I had to say twice. 

[Suggest] maybe restate the altitude when I issued the speed. But I did make sure I heard the word knots and not just 2-5-0. So I don't know. Maybe extra separation when obviously the pilot doesn't want to listen.

Synopsis

ZLA Controller reported a pilot deviation descending from an assigned altitude, which resulted in being in conflict with climbing traffic.

ACN: 1421456 (1475 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Ceiling.Single Value : 25000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : ZZZZZ
Airspace.Class B : ZZZ
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15500
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 4100
ASRS Report Number.Accession Number : 1421456
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Our clearance out of ZZZ was via the ZZZZZ SID, ZZZZZ Transition, with an initial altitude assignment of 4000 feet. After takeoff the tower handed us off to departure. I checked in with departure telling the controller that we were climbing out of (our current altitude) for 4000 feet via the ZZZZZ departure. The controller issued us a clearance to climb via the ZZZZZ departure.

From reading memos that our union and company have issued in the past I remembered that a clearance of that sort means that we should climb to the top altitude on the departure procedure while complying with all constraints on the departure.

Since there are different top altitudes on this departure, and since I read these memos months ago, coupled with the fact that I have actually never been issued this exact clearance without the controller also stating an altitude I queried the controller to make sure that we were all on the same train of thought. I asked something like "Just to verify, that means we should climb to 16,000 feet via the departure?"

The controller replied in the affirmative, verifying what I said/asked was correct. The controller had issued the clearance using standard phraseology and answered my inquiry in a professional manner. I was impressed with the departure controller.

My only concern with this departure clearance was the fact that on all of these new departures at every airport I have used them at, not once has a controller used the standard phraseology of "climb via." They have always said something like "climb via the ZZZZZ departure and maintain 16,000 feet."

I understand that the standard phraseology is meant to have less talking on the radio thus helping to relieve radio congestion but as a pilot I can tell you that the way most controllers are issuing the clearance (to include an altitude/top altitude) is much more re-assuring to us when we actually hear the number of the top altitude from the controller.

I am reminded of the "old days" when a ground controller used to issue a taxi clearance to a runway using one phraseology vs another one. One clearance meant that you could cross all runways that were not in use and another way of saying it meant that you could not. After years of runway incursions that phraseology was changed. I think that in the case of these "Climb via the ZZZZZ departure" we could learn that also having the controller issue the number of the top altitude as part of that "climb via" would make for less confusion and less altitude issues over the long run, with only a minimal amount of additional talk on the radio. After all, if pilots ask for clarification, like I did, that will actually tie up the radio more than if the controller just issued the altitude as part of the clearance.

Synopsis

The First Officer of an Airbus A320 reported they were issued a proper clearance, but asked for clarification to ensure the proper altitude.

ACN: 1421181 (1476 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1421181
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While being pushed back from the gate, we received a reroute for weather. I was listening to the clearance and writing it down. I heard the ground crew who were on headset with me tell me to "set brakes". I complied and set the brakes and read back "Brakes set" and requested that they stay hooked up because I was in a NEO and wanted to make sure the engines started before they disconnected. We then proceeded to start the engines and during that process I told the ground crew they could disconnect. My mind went back to watching the first officer input the reroute into the flight plan. While I was distracted with that I heard the ground crew ask if they could disconnect and I immediately reached down and turned the parking brake but turned it the wrong way. I thought I was setting it but actually turned it off, but my feet were on the brake petals and the aircraft didn't move. At the same time the ground crew told me to set the brakes as they saw that I had actually released the brake. I apologized and asked if everything was ok and they said it was. 

Pay more attention to what I am supposed to be watching and think prior to just turning a knob.

Synopsis

A320 Captain reported that when asked to set the brakes he turned the handle the wrong way.

ACN: 1420921 (1477 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PHL.Airport
State Reference : PA
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Ramp : PHL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1420921
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While turning into gate it was required to emergency stop aircraft to avoid colliding with a tug (bag carts attached) traveling on road at a high speed across the front of aircraft path. Tug driver did not give the right of way to the aircraft. Then crossing the nose of the aircraft the tug tried to stop, skidded, seemed to nearly jackknife as the back of the tug 'reared-up', and then to accelerate again as I stopped the aircraft. I stopped the aircraft at the head of gate and awaited a Marshall and wing walkers. After approximately 5 minutes and a call to Operations, the marshal and right wing walker showed and positioned themselves. The left wing walker appeared and very slowly approached from the ramp / terminal building. The left wing walker would not raise his wand or properly observe to signal safe taxi-in nor was he viewing the left wing tip to ensure clearance. He just stood there with his back to the wing tip, facing the marshal waving his wane. A call to Operations describing what was required was made. Then a second call.

The left wing walker stood there and just flipped his wand like a baton. A company aircraft that had just pushed-out from [the adjacent] gate, and while blocked on the taxiway observed the same and relayed this information to the ramp controller. While the left wing-walker stood there a push-out tug approaching from adjacent gate nearly backed into him as it transitioned into the 'clear-zone' and turned quickly away when approximately 3 feet from him. While we were stopped awaiting to complete taxi-in vehicles drove past the aircraft nose transitioning through the gate 'clear-zone'. Vehicles on the roadway and westbound aircraft on the taxiway were blocked. I asked Operations if they thought a 'shut-down tow-in' would help (to alleviate the building of blocked traffic). I did not receive a definitive answer. However our ability to safely park, the ground vehicles ability to continue on the road, and westbound taxiing aircraft on the taxiway were unable to move causing an operational gridlock. After being stopped for approximately 15 minutes a ramp manager counseled the left wing walker. The left wing walker was then properly positioned and followed SOPs. 

The color of the left wing walker's vest indicated to me he was not a probationary employee, had experience, and therefore should have known proper procedures. I would further submit this employee displayed an attitude of recalcitrance as evidenced by his very slow initial appearance from the terminal building. By his very slow positioning on the ramp. By his gestures directed at the cockpit. And by his lack to follow SOPs. Counseling and the mandatory retraining of this employee as well as a documentation of this event in his permanent personnel file.

Synopsis

A320 Captain experienced a ground conflict with a tug pulling carts requiring a sudden stop of the aircraft. Taxi into the gate was delayed further by the late arrival of the left Wing Walker and his seemingly inability to perform his job according to SOPs.

ACN: 1420853 (1478 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : ILS/VOR
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 130
ASRS Report Number.Accession Number : 1420853

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11300
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 802
ASRS Report Number.Accession Number : 1421085

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We first received an ECAM message that the number 1 ILS had faulted. We wrote that up. Then later, we received an ECAM message that FLT CNTL FLAP 1 SYS Fault. The FO ran the QRH for that. The QRH stated that the flaps would move slowly. We sent messages for each of the faults that we received. I decided that I was going to need more time on final to get configured, so I asked the FO to [advise ATC]. He didn't think we needed to, but he did so because I asked. Shortly after we received a third ECAM, GPWS Fault. That QRH called for pulling circuit breakers. I decided that we were too low to the ground to start pulling circuit breakers. It was day VMC. I briefed the FAs. I sent dispatch a message. I made sure I transferred controls to the FO so that someone was flying the plane. I made a PA to the passengers. We were about 15000 feet by the time all that was done. ATC vectored us for a 20 mile final. We were cleared for the visual approach. There was traffic next to us on final, so I asked the FO to turn the transponder to TA. We landed uneventfully. The flaps seemed to work just fine. 

When we got to the gate, we thought we were taking the plane back out, so the FO went out and did a walk around. During that walk around, hydraulic fluid was found dripping from the nosewheel doors. I did not write that up because maintenance had already come on the plane and was troubleshooting. We swapped aircraft and went to the next flight. The only deviation I made from the QRH was not resetting the circuit breakers. And, like I stated above, doing that when below 10,000 AGL just didn't seem prudent.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported receiving a #1 ILS fault then a FLT CNTL FLAP 1 SYS fault and a third GPWS fault. A normal approach and landing ensued.

ACN: 1420220 (1479 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 1801-2400

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1420220
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Received an ACARS from the captain of the flight asking me to call him. I tried and was finally able to get a phone to radio patch on a different frequency via phone patch. Captain had stated he had a Brake Sys 2 fault. The captain requested a patch with Maintenance Control and also advised he had gone over the QRH and thought it was safer for him to not follow the QRH. Once I made the phone patch with Maintenance Control and after the conference, myself, the captain and Maintenance Control also agreed for the captain to not follow the QRH procedure as described because following the reset procedure would make the aircraft's anti-skid system inoperative. Since the flight was going to an airport with short runways, I agreed with the captain that in the interest of safety the best possible thing was to not reset the circuit breaker and to continue on to destination with his antiskid operative for landing. The flight continued on without further incident. 

The wording of the QRH says "resetting the BSCU in flight may cause the brake system to revert to alternate braking without antiskid at touchdown." Perhaps if the wording was different or the QRH was changed to give the option to the captain of doing this, things would be clearer. Two days later after talking with the Maintenance Director, he said he had spoken with the Chief Pilot and they both concurred with the decision myself and the captain made. If this flight had be going to an airport with much longer runways I would have objected to not following the QRH and suggested the captain follow the reset, however with the short runways I agreed this was the best option so the crew still had antiskid as an option.

Synopsis

Air carrier Dispatcher reported he and the Captain agreed to deviate from the QRH given the specifics of the brake problem and the short runways at the destination airport.

ACN: 1418147 (1480 of 2908)

Time / Day

Date : 201701

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component

Aircraft Component : Potable Water Storage, control, indication
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1418147
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was coming off the high side of the nitrogen cart, not focused that it was 100 PSI increments. Thinking it was 10 PSI increments. Thus very over pressurizing the potable water system which resulted [in] aircraft damage.

Stay focused on the task at hand. A pressure relief valve would be nice in system, but still no excuse.

Synopsis

Technician had the potable water system over-pressurized on an A320 causing damage to the aircraft.

ACN: 1417896 (1481 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Aileron Control System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1417896
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1418149
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected.Other 
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was assisting on a aileron servo change for Aircraft X. We were directed by maintenance control to change the servo on the left hand side of the aircraft. During the job setup we pulled all maintenance manual reference's to do the work. We needed an Aileron collar per the maintenance manual which we did not have. We continued to go ahead with the work as per the direction of our lead mechanic. During the course of the night we used a socket to prop up the aileron to keep it from drooping down and to aid me to give more help in removing and installing the left hand aileron servo. We contacted Quality Assurance (QA) to do a pre-installation inspection and was given the ok to install the servo. After we had installed the servo we had to perform a torque on both main bolts at this time another QA inspector came to relieve the night shift inspector. The oncoming QA inspector witnessed the torque of both bolts. At this time we were ready to install 2 cotter pins we ordered earlier from the Illustrated Parts Catalog (IPC). QA stated the cotter pins look to short so he went to the hangar to look for some longer cotter pins and he instructed us to pressurize the hydraulic system and check for leaks as he was looking for cotter pins. [A technician] and I went up to the cockpit to make sure the hydraulic system was fully pressurized on the aircraft. I went down to check for leaks and [the technician] come off the aircraft and asked me if the socket we had used as a prop was removed. At this time I noticed the socket was jammed into the lower part of the aileron and breaking the fiber laminate. QA returned and we told him of our issue. 

We had forgotten the socket was still holding the aileron from drooping. We had also rushed to power up the aircraft without checking the work area for tools. Since the aircraft was on the gate and due to fly in a few hours we felt rushed to get the task finished so we would not add to any delay of the flight.

We had our supervisor and manager document the event and pictures were taken during this time. We were allowed the continue working on the aircraft to finish up the installation. 

Once we had found out we did not have the tool we needed to perform the task we should have stopped. Also to notify our superiors that the task could not be performed due to the missing tool.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

Maintenance technicians reported discovering damage to the left hand aileron of an A320 due to a socket being used as an improvised tool.

ACN: 1417423 (1482 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1417423
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

After the push back we noticed that [the ramp crew] used a baggage cart tug to push the aircraft instead of a normal aircraft tug. The tug they used isn't designed or approved for pushing an aircraft that weight around 160,000 pounds. There are serious safety concerns and potential damage to the aircraft with their action. They never communicated their intention and the reason why they used the baggage cart tug instead of normal aircraft tug.

Our stations staffs are poorly trained. Company has done a poor job maintaining quality control of the contract staffs. These problems can be solved by adequately training contract staffs.

Synopsis

A320 Captain reported that the ground crew used a baggage cart to push out the aircraft instead of a proper tug. He expressed concern about using this equipment and the lack of training for ground crews throughout the system.

ACN: 1416579 (1483 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5577
Experience.Flight Crew.Last 90 Days : 146
Experience.Flight Crew.Type : 1577
ASRS Report Number.Accession Number : 1416579

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20655
Experience.Flight Crew.Last 90 Days : 224
Experience.Flight Crew.Type : 4274
ASRS Report Number.Accession Number : 1416584

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were dispatched with an MEL Autothrust Inoperative and placarded appropriately. Crew briefing included discussion on lack of autothrust, setting 1.31 EPR for takeoff and throttle settings needed for flight. I (First Officer) was the Pilot Flying with the Captain performing Pilot Monitoring duties. Environmentals were: Rwy 15R, winds from the south gusting to 29 kts, VMC. With 1.31 EPR set and a normal takeoff roll and acceleration, an oral "retard, retard" was heard in the high speed regime (100 kts). With good engines and confirmed valid takeoff configuration (takeoff memo green), we continued takeoff with normal rotation and liftoff. With positive rate in climb, I directed gear up. Shortly thereafter, we received a red Master Caution, configuration bell and red arrows on gear light panel. When climb detent was selected (roughly 800 ft) the warning stopped. Flight continued as normal, albeit manual thrust settings were used.

Narrative: 2

After takeoff, gear up, turning to ATC assigned heading, got a Master Caution and bell for configuration. Gear was up with normal flying parameters. Canceled Master Caution. Flight continued normally with no other issues.

Synopsis

A320 flight crew reported an aural "Retard, Retard" message at approximately 100 knots on the takeoff roll. The configuration bell and red gear lights alerted on climbout which ceased after climb detent was selected.

ACN: 1416002 (1484 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19819
Experience.Flight Crew.Type : 8176
ASRS Report Number.Accession Number : 1416002

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We were delayed waiting for maintenance to taxi the aircraft to the gate. They had been working on the aircraft the last 2 days for an Anti-Skid and Nose Wheel Steering loss by a previous crew.

They had replaced a Servo Control and signed the aircraft off. It was now delivered to our gate. We departed with no problems, faults or issues until we were at 2000 feet and lowered the landing gear for landing on runway XY in ZZZ. Several seconds later, we received a Nose Wheel (NW) Steering fault. The First Officer (FO) and I cycled the NW Steering Switch to try a reset of the system. I pulled out the QRH and quickly read through the procedure. We determined that we would have no NW Steering on landing and concurred to continue the approach and landing. The FO landed the aircraft and after we slowed to a slower taxi speed, I took the aircraft from the FO and exited on a high speed taxiway. I stopped the aircraft and we requested a tug to tow us to the gate.

After we stopped, we also received a Brake System 1 Fault. The FO contacted maintenance and we tried a reset of the system. It did not fix the faults, and we then had to wait for the tug to tow us to the gate.

Synopsis

A320 Captain reported accepting an aircraft that had just come out of maintenance for an Anti Skid and Nose Wheel Steering fault. The fault returned when the gear was extended for landing and a reset was unsuccessful. After clearing the runway the aircraft was towed to the gate.

ACN: 1414514 (1485 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
ASRS Report Number.Accession Number : 1414514

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The aircraft was experiencing a vibration shortly after takeoff that neither the cockpit crew nor the cabin crew had experienced before. The vibration was not due to engine vibration and all systems indications were normal. No ECAM warnings or messages. We continued to climb on our route while the First Officer and I talked about possibilities and decide on a course of action. The vibration stopped around FL260. We continued to climb to FL360 and reduced speed to .73 Mach/280 knots. After contacting Dispatch and Maintenance, I made the decision not to continue to [destination] and coordinated with Dispatch to divert [to an alternate]. Approach and landing were uneventful.

After arriving at gate, Maintenance believes the vibration was due to either:
1. A faulty hydraulic service panel door that would not close flush with the fuselage and could possibly have departed the aircraft in flight, or
2. A cracked sealant on a panel beneath the fuselage, or
3. A seal on the right wing root.

None of the possible causes would have been detected by crew walk around. The hydraulic service panel door had to be handled to ascertain that it would not close properly.

Synopsis

A320 Captain reported diverting to an alternate airport after experiencing inflight vibration that was possibly due to a service panel door or a wing root seal.

ACN: 1414216 (1486 of 2908)

Time / Day

Date : 201701
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : DTW.Airport
State Reference : MI
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Navigation Database
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1414216
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Ambiguous

Narrative: 1

I was made aware that our FMS database was incorrect. The date of the database was correct, however, the database number was incorrect and we as a company were using the wrong database. The safety department even put out a pilot read file to warn pilots that we are flying the wrong approach into LAS as the approach that we were flying did not go into effect until [later]...the date of the next database which is the one that we are using even though it's only [an earlier date]. I wrote up my nav data base and the company proceeded to [defer] the data base. 

Days later I was shocked to see that my airplane's database was the same wrong one, but not [deferred]. I called [maintenance] and asked why not, they said this was the first they had heard of it. Well when I wrote my airplane up I told [maintenance] that I felt this was a fleet-wide issue. Local [maintenance] even checked other aircraft on the ground and confirmed that all the airplanes had the wrong data base!

[Maintenance] then told me that they would start to [defer] the aircraft as they landed. 

The next day I flew 2 different aircraft, none of them had the data base [deferred]. So I wrote them all up. When I called my [Chief Pilot] to explain the issue, he told me that Jeppesen told them that LAS was the only affected airport and that we put a read file out about that approach.

So, I don't know if I should accuse the company of ignorance, gross negligence or just not caring one bit about our crews or passengers. 

The FAR's are clear, you need to write up any discrepancies at the time they are found. The wrong data base is wrong, so it needs to be addressed, and the company was WELL AWARE that every single airplane had the wrong data base.

I have never in my life received a Jeppesen revision with just one change, so I doubt this is true, but again, even if it is, the data base is still wrong and needs to be written up.

Safety knew about it, [maintenance] knew about it and flight ops knew about it. Yet not a single person thought about the crews going to South America at night with the potential dangers of not knowing you are using the wrong data base, that's criminal in my mind.

Our crew are taught to look at the date, we have no idea what the [database numbers] should be, so without the company [deferring] the databases, our crews would have no way of knowing they are using the wrong data base. (This data base has the proper date, just the wrong number.)

If we are flying the wrong approach into LAS, who knows what else is wrong in the data base?

I would like someone to explain to me and the FAA how it's okay that our entire fleet is flying around for over 8 days with the wrong data base, and not [deferring] it or addressing the problem in any way. Remind everyone just how important the details of their jobs are. People's lives depend on everyone doing the right thing.

Synopsis

A320 Captain reported that his company seemed unconcerned that the FMS database for an entire fleet had accuracy issues.

ACN: 1413468 (1487 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1413468
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

The primary concern of this report is the flight attendants not disarming their door after arrival. The signal for the cabin to disarm doors is the seatbelt sign going off. We waited about 45 seconds and they still hadn't started disarming doors. I cycled the seatbelt sign in case they hadn't heard it, still no disarming. As the jet way was quickly approaching the door we made a PA for the flight attendants to disarm doors. This is the same crew who didn't check handset the night before until we had done a [return to the gate] for another reason. The same "A" that had no idea what a security check was or why we did it on first flight of day. Same flight attendant that had no idea why we wear O2 masks when one of us leaves the flight deck.

Maybe we should train the new hires in more than 2 weeks so they learn what we used to teach all flight attendants in their class. This in just one example of the many things I see on a daily basis that hardly ever happened a few years ago The training of the new hire flight attendants is only hitting high spots now train them the way we used to so they actually learn something.

Synopsis

An Airbus A320 Captain reported that after arrival the flight attendants did not disarm the doors in a timely manner.

ACN: 1413414 (1488 of 2908)

Time / Day

Date : 201612

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9072
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 6890
ASRS Report Number.Accession Number : 1413414

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

Due to reassignment we were given a 30 minute report with a 10 minute delayed brake release. During pre-flight planning, we notice that the APU is deferred INOP. The APU had been written up for a day or so and the aircraft had come into a maintenance base the day before. The write up stated that "All APU indications showed XXXs." Maintenance response was to defer without any further or added explanations. After contacting the Dispatcher for a better understanding of Maintenance intentions, I, as Captain, using my knowledge and authority given to me by the FAA and [company] FOM, refused the aircraft. The First Officer and I sat down one gate away from our gate while we waited for a decision from the company.

Several minutes later, someone identified himself as the Assistant Chief Pilot for [this base] and, in front of at least one passenger and my First Officer, started questioning me on my decision. Asked me why I wouldn't take the airplane even after they were pouring hot air into it from gate air. I started to explain that it was more than comfort, that it was also operational safety, and he quickly cut me off and threatened with "Now you have to file a [Report]" as if insinuating that would me make reverse my decision and fly the broken airplane.

About 10 minutes later this same Assistant Chief Pilot calls me on my cell and says that they have a new aircraft for me but they can't assign it because I haven't officially refused the original aircraft. I tried to explain to him that I had refused it through Dispatch over an hour ago. Dispatch also assured me that they still had no other aircraft. If the FOM and FAA is clear on the Captain being the "final authority for the SAFE operation" of the aircraft, why does a flight manager need to come to the gate and try to get a Captain to change his mind?

Synopsis

A320 Captain reported being pressured by his company to take an aircraft which he refused due to an inoperative APU.

ACN: 1413301 (1489 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 200

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Ferry / Re-Positioning
Flight Phase : Takeoff / Launch
Flight Phase : Climb

Component : 1

Aircraft Component : Landing Gear
Problem : Malfunctioning

Component : 2

Aircraft Component : Flight Director
Problem : Failed

Component : 3

Aircraft Component : Autopilot
Problem : Failed

Component : 4

Aircraft Component : Autothrottle/Speed Control
Problem : Failed

Component : 5

Aircraft Component : ACARS
Problem : Malfunctioning

Component : 6

Aircraft Component : Fuel Distribution System
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 142
Experience.Flight Crew.Type : 11500
ASRS Report Number.Accession Number : 1413301

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1413429

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The flight scheduled as maintenance ferry from to ZZZ1. The aircraft was fueled for flight from to ZZZ2 so there was plenty of gas on board. Ground operations were delayed due to failure to get MFD (maintenance ferry document) over the ACARS. Eventually the MFD was sent to ZZZ Ops and delivered to the aircraft. Due to improper fueling the crew also had to get a deferral for a FUEL AUTO FEED FAULT. Remainder of ground ops were normal. Taxi and takeoff thru rotation were all normal. The Captain Pilot Flying (PF) called for gear up. First Officer attempted to raise the gear handle and found it locked in the down position. Shortly after that L/G SHOCK ABSORBER FAULT ECAM displayed. Flight director, autopilot and autothrust systems all inop. Crew made the decision to continue to ZZZ1 and completed all checklist. The Dispatcher was advised of the situation and decision to continue. Voice communication with the dispatcher was attempted via the call me ACARS function but failed for unknown reasons. ATC was notified of the situation and informed that the flight would continue to ZZZ1.... There was a strong, gusty crosswind at ZZZ1 and the Captain flew a visual approach and landing to runway at ZZZ1.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew was unable to raise the gear handle after takeoff and subsequently experienced Flight Control Servo, Fuel Auto Feed, Flight Control, Auto Flight, and Auto Thrust faults. Flight was continued to final destination which was a relatively short distance.

ACN: 1413284 (1490 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Landing
Airspace.Class C : ZZZ

Component : 1

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X

Component : 2

Aircraft Component : Autopilot
Aircraft Reference : X

Component : 3

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X

Component : 4

Aircraft Component : Flight Director
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25300
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1413284
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Human Factors : Workload
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff we were unable to raise the landing gear. We received and ECAM stating we had a fault in the sensor. I can't recall the exact ECAM. We ran the ECAM which told us not to retract the landing gear and gave us a limit speed. I contacted dispatch and [maintenance] for further expertise. [Maintenance] said to try and raise the gear. We were unable to raise the gear lever. AP (Auto Pilot), AT (Auto Throttles) and FD (Flight Director) were not available. We climbed to 7000 ft msl and the F/O (First Officer) flew the aircraft manually and worked the primary radios. We decided that this was not an emergency. We had full aircraft control, we were VFR and we had a 10000 ft runway to land on. I requested landing data and from dispatch and we determined the airplane was safe to land based on the data. I requested fire rescue in case of brake overheat. I then advised the flight attendants of our situation. I then notified the passengers. We discussed the soft landing and delayed using the brakes until below 60 kits. The landing was soft and brake temperatures stayed below 200C. In my opinion it was the safest option as opposed to flying the airplane manually for three or four hours with no autopilot and flight directors. I have over ten years as a Captain in this airplane and was confident I could land smoothly and safely.

Narrative: 2

After takeoff [we] discovered the gear handle was locked in the down position. Shortly after takeoff we received an L/G SHOCK ABSORBER FAULT ECAM. ECAM directed to leave the landing gear down and we consulted with [maintenance] who recommended a return to [the departure airport]. After receiving performance data at our current weight decided that an overweight landing would be the safest course of action. Landing was smooth and uneventful and we returned to the gate.

Synopsis

An Airbus flight crew reported being unable to retract the landing gear after takeoff. After troubleshooting the problem and completing all applicable checklists, the gear remained extended, and an uneventful recovery to the departure airport was accomplished.

ACN: 1412402 (1491 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 4R
Flight Phase : Final Approach
Airspace.Class B : NYC

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : Widebody Transport
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : NYC

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 182
ASRS Report Number.Accession Number : 1412402

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

Momentary flap 2 overspeed while on approach [to] 4R EWR. Approximately 3000 ft, 200-205 kts, 2-3 seconds. IMC conditions, hand flown, behind a "heavy" and trying to slow. Turbulence (wake?) drove airspeed to flap limit momentarily. Flaps retracted, slowed and flaps re-extended on schedule and landed normally. Maintenance write up via ACARS.

Synopsis

A320 Captain reported a flap overspeed on approach to EWR due to either turbulence or wake vortices.

ACN: 1412005 (1492 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1900

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Squat Switch
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6790
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 3661
ASRS Report Number.Accession Number : 1412005

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1411908

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

IMC conditions on approach. Aircraft was configured to flaps 3, gear down, on glideslope autopilot and autothrottles on. ATC speed assigned 170. PF managed speed and asked for flaps to full configuration just prior to FAF. A multitude of ECAMs, cautions and warnings occurred rapidly. Not sure of the exact order of ECAMs as these ECAMs cleared upon touchdown/rollout and could not be recalled.

ANTI-ICE ECAM most likely Standby Probes
AP Disconnect Red Master with triple click
CABIN PRESSURE SYS 2 FAULT ECAM
All Performance data gone
All nav data gone. FD reverted to HDG/VS. No ILS info displayed. IRS monitor blank.

Both PF and PM were verbalizing the rapid succession of events and taking corrective action. We broke out to VMC conditions slight left of course and high on the VASI. PF made an early aggressive correction back to the VASI which resulted in two sink rates but corrected rapidly to stabilize the approach slightly high on the VASI. Felt that the approach was safe to continue especially since we were not sure what instrumentations may be available (Weather was IMC at alternates, too). PF felt that the flare mode did not engage as the force to rotate to the flare was not normal. A firm landing was made in the first third of the runway. Upon rollout all previous ECAMs disappeared. A repeat ECAM L/G SYS DISAGREE came back on. Rollout and taxi to gate were normal. PF and PM did a thorough debrief and it was determined that although airborne the airplane had thought itself on the ground. PF and PM also debriefed maintenance.

The L/G SYS DISAGREE ECAM was found on preflight and maintenance had tested the system and could not find any faults. After resetting CBs the ECAM went away and a new MRD was issued. Next morning checked with maintenance and found that the nose gear switch was replaced. Apparently the wiring is routed through a tube which causes corrosion to occur. According to this specific maintenance tech this is now occurring more often on the aging aircraft.

Narrative: 2

Aircraft had ECAM L/G SYS Disagree displayed on arriving at aircraft, MX was able to clear the fault. Flight operated normally with FO as Flying Pilot until established on the ILS at 2000 ft fully configured in Managed Speed with AP2 engaged. Not sure the order in which the following system fault occurred, just my recollection of when we noticed them. All taking place over the span of 30 seconds or so. All faults, except one, cleared upon landing so we were unable to accurately reconstruct exactly which faults presented and the order they occurred in. First, we received an A/I Probe fault approaching the FAF in IMC followed shortly thereafter by the AP2 disconnecting and with a Red ECAM Warning. We verbalized that the FO was to remain flying while I managed the ECAM. We noticed the Speed had come out of Managed and that all of our Performance Data had dumped. We manually selected an appropriate speed for the approach. A CAB PRESS SYS Fault ECAM then presented itself, distracting me from further evaluating and confirming our desired navigation mode. 

We had broken out of the overcast at the FAF and noticed a slightly high and left state. I then saw that we were no longer tracking the ILS (ILS data dumped along with Perf data) and the acft had reverted to Vertical Speed -200 and HDG. FO verbalized a visual maneuver back to reestablish a PAPI visual glide path and I agreed that we continue visually based upon the seemingly unrelated multiple system faults and questionable navigation ability presented. We received 1 "Sink Rate Sink Rate" maneuvering to reestablish a visual glide path and elected to continue as we were otherwise configured, stabilized, and on speed. I don't remember hearing GPWS Altitude callouts of 500, 50, 40, etc. during the landing phase. FO stated during the flare that the pitch wasn't responding as expected to the normal amount of back pressure required to check the descent resulting in a firmer than planned touchdown. 

Taxi to the gate was normal and all ECAM messages disappeared to be replaced by the original L/G SYS Disagree that we first saw at the gate. Maintenance code sent for the presented L/G fault followed by Info describing the other observed faults during approach. Relayed event verbally to both Maintenance Control and Maintenance.

Synopsis

A320 flight crew reported receiving multiple seemingly unconnected ECAM alerts on final that could not be recalled after landing. Maintenance replaced the nose gear air-ground squat switch.

ACN: 1411866 (1493 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : Mixed
Weather Elements / Visibility.Visibility : 5
Light : Night
Ceiling.Single Value : 2000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1411866

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

I inquired [during preflight] about the fuel and the winds, and the Captain informed me of the conversation he had with dispatch, that we would monitor it very closely and if we didn't feel comfortable we would be stopping [enroute] for more fuel. We were all in agreement and off we went. Once airborne our fuel load showed us landing with approximately 4500 lbs of fuel, which was not what we wanted or were comfortable with. We had updated the winds and then put in the step up altitudes throughout the flight. Our initial cruise altitude was FL240 due to the heavy winds at higher altitudes. My Captain also checked on different cost indexes once at cruise, we were at a cost index of 70 because we were already scheduled to arrive almost an hour after scheduled arrival time. 

The Captain was able to bring the landing fuel up to 6500 lbs by changing the cost index to 15. We communicated this with dispatch and again all 3 of us decided this was the best course of action. We monitored the winds and were able to step up to higher altitudes as scheduled, to keep the fuel load at 6500 lbs for landing. The Captain and I decided that we were comfortable continuing to [destination] without stopping for fuel. The weather was reporting I believe 2,000 overcast and 5 miles visibility, with a one degree spread between dew point and temperature. We had talked about what we would do if they gave us delaying vectors and we decided that in the event that happens that we would have to declare min fuel. Once on with Approach the first controller kept us on our route, but the second controller said he had to give us delay vectors. I believe it was a 60 degree turn off course. The Captain and I again discussed our best option and fuel load, and decided to declare min fuel. The controller understood and we descended but stayed on our route. 

That controller transferred us to the next controller, and the new controller explained that min fuel did not mean she had to give us any help and made it clear that she would not help. She told us that the delay vectors would take approximately ten minutes, and if we needed priority we would have to declare an emergency.

Synopsis

A320 First Officer reported declaring minimum fuel in an effort to expedite their arrival, but the Controller indicated they would have to declare an emergency to ensure special handling.

ACN: 1411171 (1494 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1411171
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

The aircraft came in late at [night]. I reviewed the logbook. The last Remain Over Night (RON) check was completed on three days prior. I reviewed the Operating Bulletin (OB) which states "The Airworthiness Release has changed from a validity period of 72 hours to three calendar days, not to exceed 23:59 UTC of the third day." I contacted Maintenance (MX) Control and spoke the supervisor. He stated that this aircraft was good until the following day. I disagreed as that was 4 calendar days after the inspection. He said no, that the aircraft was good as the first day did not count. My First Officer (FO) then referenced the MEL where they do not include the first day when counting the days allowed under the MEL. I then contacted the Chief Pilot on Duty. I explained the situation and he agreed with MX control that the 3 days begin after the day of the inspection. While they may be right, I expressed my frustration that the OB did not correspond to what they were telling me and that the OB should be modified to reflect what I was being told. The Chief Pilot told me that this issue has come up a lot and that all of the other flights had departed based upon his and MX interpretation. I decided to operate the flight that evening. The flight crew who had brought the plane in would also had to deal with this situation. My FO forwarded a union correspondence that he received that supported my initial determination that three calendar days is just that which includes the day of the inspection. Consequently, I may have flown the aircraft out of inspection as did the crew before us. I relied on the statements of MX Control and the Chief Pilot in operating the aircraft. I still do not know what the correct status of the aircraft is in the eyes of the FAA and Pilots should not be put into this position by the company.

Correct the Operating Bulletin to include the words "not including the day of the inspection" or send out a memo that 3 calendar days includes the day of the inspection as indicated.

Synopsis

A320 Captain reported confusion related to the aircraft Airworthiness Release requirements and indicated that the company Operating Bulletin needed updating.

ACN: 1411127 (1495 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1411127
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Four times in the last 6 months, I have had a maintenance issue after leaving the gate and while still the ground, and Maintenance Control has either questioned me as to why I wrote up the discrepancy in the Aircraft Maintenance Logbook (AML), or told me to write it as "info-item." I believe that in all these maintenance issues Maintenance Control has either changed procedures and not conveyed the new procedures to the flight ops department OR Maintenance Control is confused on what is "legal" as far as proper AML documentation is concerned. Received ECAM: F/CTL GND SP:R 1+2 FAULT taxiing out. I contacted dispatch/Maintenance Control. I explained the ECAM message (which had no ECAM procedures or QRH procedures associated with it) to Maintenance Control and stated that I had written it up in the AML. Maintenance Control stated for me to cycle some specific hydraulic switches, at which point I asked Maintenance Control if I followed his instructions would I be able to "balance" the write up in the mechanical discrepancy section of the AML with the notation of per Maintenance Manual such and such action accomplished with his name and employee number? (As we have always done.) Maintenance Control stated as long as I write it up as "info-item." I stated that I would NOT write it up as "info-item" because per MEL introduction and FM 1 the malfunction does not meet the criteria of "info-item." Maintenance Control stated that if I did not want to write it up that way that I would have to return to the gate. I asked him why he is asking me to write it up as "info-item?" Maintenance Control stated that this was the first time anyone told him that it wasn't appropriate. In the other three cases that I had, Maintenance Control was surprised that I wrote it up at all, and Maintenance Control stated that if I didn't write it as "info-item" that it would be an "open write." I explained that it would not be "open" if I entered per Maintenance Manual in the mechanical discrepancy section. Maintenance Control said that pilots can't enter that. I asked since when? I also got on record that I disagreed with Maintenance Control's explanation and asked the dispatcher to research this procedure. Back to the gate in all 4 cases. 6 months later no explanation except line maintenance that stated I was correct, Maintenance Control was wrong, and next time to hang up the phone, call back and hopefully I will get someone different. I would like an immediate answer to this issue please. Thank you.

I think the managing director of ops needs to call me and tell me why Maintenance Control is not on the same page as flight ops. What are the proper procedures now if they have changed, why have they changed, and why has communication failed between departments.

Synopsis

A320 Captain reported observing F/CTL GND SPLR 1 + 2 FAULT during taxi out and was told to enter an Aircraft Maintenance discrepancy as an INFO ITEM. The Captain refused and returned to the gate.

ACN: 1410899 (1496 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 12500

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : ZZZ
Airspace.Class E : ZLC

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 7900
ASRS Report Number.Accession Number : 1410899

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft TA
Miss Distance.Horizontal : 500
Miss Distance.Vertical : 2
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Initial clearance was to 1000 feet which was not the same as the SID. ATC then gave aircraft unrestricted climb to 14000 feet. Around 12500 [feet] Captain queried about traffic 5 miles ahead. ATC said he wouldn't be a factor. Captain queried again and controller stated he could clear aircraft to another fix. Captain accepted at which time crew received a TA. Once in the turn TA disappeared.

This is a recurring problem in Salt Lake. They modify the SID and then we get a traffic conflict. Calls to our company a major carrier reach deaf ears.

Synopsis

A320 Captain reported a Traffic Alert on TCAS while climbing out of SLC.

ACN: 1410856 (1497 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Cloudy
Weather Elements / Visibility : Snow
Weather Elements / Visibility : Icing
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1410856
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1410857
Human Factors : Situational Awareness

Events

Anomaly.Ground Excursion : Taxiway
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Summary: after a safe landing in ZZZ, braking action was nil on taxiway. The left main gear outboard tire departed the paved taxiway surface by about 1 foot. We stopped the aircraft, coordinated with all concerned parties, deplaned the passengers, and had the aircraft towed to the gate.

During preflight, we were already aware of inclement weather and reduced braking action in ZZZ. However, we received the flight paperwork only 5 min prior to ETD. Almost immediately after the paperwork was delivered, we heard the L1 door close and the flight attendants make a PA to arm the slides. I had not handed out the paperwork or authorized this. I saw the slides arm on the ECAM. I stepped into the galley and instructed the flight attendants to disarm the slides and make a PA instructing the aft flight attendants to do the same. As soon as the slides were disarmed, the gate agent opened the door from the outside without receiving a thumbs up, which surprised the lead FA and me.

I instructed the lead FA and the gate agent to enter the cockpit so we could discuss what happened. After hearing each of their descriptions, I determined that the gate agent had felt operational pressure to rush and close the door to (from her perspective) help us depart on time, and had forgotten to receive the paperwork from me first. I reminded the lead FA and the gate agent of the need to check with the captain before closing the door. I also discussed with the gate agent that opening the door from the outside in a surprising manner without a thumbs up could result in relying on the mechanical disarm interconnect as a last resort to prevent slide deployment, if the slides were armed. Finally, I advised the gate agent that due to the late paperwork and the need to fully investigate the weather and braking action in ZZZ, we would be departing late, and I would let her know when we were ready.

We delayed departure to review weather and landing distance. -FZRA was reported earlier, but we received an updated report during preflight that stated all precipitation stopped. The TAF showed no precipitation forecast after XA00Z. Our scheduled arrival time was XA31Z. NOTAMs stated RWY XYR braking action code 2/2/2. The Takeoff/Landing Report (TLR) did not include landing distances for various reduced braking action Runway Condition Assessment Matrix (RCAM) codes. We calculated a landing distance for this RCAM code manually using the COM because it is not possible to obtain a valid result for RCAM code 2 using ACARS.

We determined RCAM code 2 was acceptable with a safe stopping margin even without reverse thrust credit. However, we calculated that if the runway braking action deteriorated to poor, this would not be acceptable. We considered calling dispatch to discuss the situation, but dispatcher remarks on the release stated, "They have sanded and adding chemicals to the runway in ZZZ. I will update you enroute. They are 90 percent sure everything will be good at your ETA." We agreed that our landing distance calculation, this remark, and the improving weather report/ forecast indicated to us that dispatch was fully aware of the situation, and no further coordination was necessary. We departed.

Dispatch did not update us enroute. We regularly re-checked the weather at ZZZ, ZZZ1, and our alternate of ZZZ2 during cruise. All of these reports and forecasts remained acceptable.

ZZZ does not have digital ATIS. We received the ATIS when within radio range. Runway XYR was in use. It stated braking action poor on all runways and taxiways. Based on our previous calculation, this braking action was unacceptable. ATC approved us to hold at present position. We conducted one turn in holding while the First Officer (FO) contacted Tower on the radio. Tower immediately advised that the braking action report on the ATIS was out of date, the runway and taxiways had just been treated, and Tower asked an operations vehicle on the frequency for an updated braking action report. The vehicle replied and we copied runway XYR braking action RCAM code between 2 and 3. We confirmed that our previous manual landing distance calculation for RCAM code 2 was still valid, and we concurred that the runway was acceptable. We inquired about the condition of the taxiways. A vehicle replied reported taxiway braking action also between code 2 and 3, with a remark to "use caution." We asked Tower what to expect for a taxi route. Tower recommended to expect to make a 180 degree turn on the runway, backtrack, vacate at D1, and taxi via D to the ramp, and stated that this was because taxiway D was the taxiway in the best condition. We agreed that this plan was acceptable, discontinued the hold, and conducted a stabilized visual approach in CONF FULL.

I touched down smoothly but positively approximately 1,200 feet past the threshold and applied maximum reverse. Autobrake medium was selected to get the braking started, but per our landing distance calculation, I immediately applied maximum manual braking. Directional control was excellent and the airplane remained exactly on the centerline. Braking action was good and we reported this. Tower cleared us to taxi as previously discussed. I reduced to a very slow speed and used the beginning of taxiway as a turning bay. Directional control and braking action was good throughout the turn. We backtracked, slowed to a very slow speed well prior to the runway exit, and vacated at D1. We kept both engines running with engine anti-ice on. The FO completed his After Landing Flow and started the APU while we were backtracking, well before the turn onto D1. I advised the FO to hold the After Landing Checklist until established on taxiway D so we could both be heads-up.

Directional control during the turn onto D1 was acceptable. The speed was very slow, so I was not applying any brakes at that time. I began the turn from D1 onto D. The airplane began to turn but I realized I was applying more and more tiller input with minimal effect. I attempted to stop the airplane. I applied the brakes moderately, felt no deceleration, and then increased brake pressure to the stop. Braking action was nil. I told the FO we were sliding. Groundspeed at this time was several knots. I applied right differential braking and rudder in addition to the tiller, but it was clear we were not turning right at a rate sufficient to safely make the turn onto taxiway D.

I considered the loss of braking memory items. I decided not to apply reverse thrust because the groundspeed was extremely low, lack of directional control was the greatest threat (and reverse thrust reduces directional control), and there was a high risk of engine damage if reverse thrust was used with the engines over unprepared/contaminated surfaces during a potentially imminent taxiway excursion. I also considered that reverse thrust might make us roll uncontrollably backwards, which would be even worse. I did release the brakes, select the nosewheel steering/anti-skid off and re-apply the brakes. Braking action remained nil. I decided to not apply the parking brake because lack of directional control was still the greatest threat; the lack of braking action was obviously due to external conditions and not a mechanical failure; and the parking brake would eliminate the ability to use differential braking, if it was having some small effect.

I used a very slight amount of forward thrust on engine 1 and idle reverse on engine 2 to complete the turn onto taxiway D. The aircraft was still moving forward on taxiway D at several knots and starting to fishtail. I set both thrust levers to forward idle. The FO suggested I turn the nosewheel steering back on, and I did. I released and re-applied the brakes. Braking action was still nil, but now the nosewheel steering was giving me some control over which way the nose was pointed. I directed the FO to [advise atc], and he did, requesting Airport Rescue and Fire Fighting (ARFF) and reporting nil braking. After a couple of oscillations, I was able to stabilize aircraft somewhat left of the taxiway centerline, but pointed about 20 degrees right of it to correct. However, the aircraft was weathervaning into the wind from the right and sliding to the left due to it. Despite the aircraft pointing to the right, it was still moving slightly to the left while it was moving forward. I released and re-applied the brakes one more time, and the aircraft came to a stop. During this time, the aircraft had moved several hundred feet down taxiway D.

I set the parking brake and made a PA, "Flight Attendant Stations." The FO reminded me that the APU was available, and I shut down both engines. It would have been unsafe to attempt to continue to taxi, and I was concerned that idle thrust could have overcome whatever minimal parking brake force was holding us in place. I directed the FO to ask Tower for their opinion on whether we were still on paved surface and if there was any apparent damage. The Tower said they couldn't quite fully see everything, but based on what they could see, the aircraft was still on the pavement and there were no obvious signs of damage. I called the lead FA and asked her to look out the left windows and give me her opinion on whether we had departed the paved surface, and to tell me if there was any signs of damage. After about a minute, she reported back that she couldn't tell whether we were entirely still on the pavement, but there was no damage that was evident to her. I quickly advised the lead FA what I thought happened, advised that I would cancel flight attendant stations, and stated that I would make a PA. I made a PA, "Remain seated, remain seated, remain seated," and then explained over the PA that due to icy conditions we had to stop our taxi, we would need to have the aircraft checked by maintenance, and then we would likely be towed to the gate when it was safe to do so.

The FO and I discussed, agreed upon, and executed the following plan. The FO contacted Tower to advise that we had stopped taxi and shut down engines due to nil braking action, requested ARFF inspect the aircraft for any damage, advised that we would need to get MX out to confirm whether there was any damage, asked for airport operations to re-treat the taxiway surface between us and the ramp, and advised that we would need to be towed to the gate when ready. The FO contacted the FBO over UNICOM, which relays information to station operations at ZZZ. At my request, he advised them to call dispatch to report the nil braking and suggest that any other inbound flight divert, inform them of our plan, request maintenance, and request a tow crew. The FBO relayed all of this. A bit later, but still early in the sequence of events, the FO and the FBO coordinated to begin assembling busses and air stairs if it became necessary to deplane the passengers at our current location.

I further discussed the situation with the flight attendants. I instructed them that no passenger should be allowed to get out of their seat until ground personnel had given us a better idea of our situation, to ensure that everyone would be safe in the event the aircraft started to slide/be blown by the wind, and to ensure that everyone would be ready in the unlikely event of a gear collapse and potential evacuation. At this point, I was sure the nose and right main gear had never departed paved surface. Based on the information I was given, I believed the left main gear was close to the taxiway edge, but had not departed the pavement.

All of the requested ground personnel began to arrive quickly. ARFF or airport operations (I'm not sure who) reported no obvious signs of damage. We requested the aircraft be chocked, and it was. At some point after the aircraft was chocked, with my concurrence, the FO and lead FA discussed whether they should continue stopping anyone who attempted to use the lavatory. We all agreed we did not want to turn off the seatbelt sign, but it was now OK for passengers to use the lavatory at their own risk. We decided not to make a PA encouraging this.

At some point I believe around this time, we completed the After Landing Checklist.

Ground personnel connected an interphone and now advised me that the left main gear outboard tire only was about 1 foot off the paved surface. Either that tire had not yet started to sink into the ground, or I was not advised of this at the time. It was clear to us that the left main outboard tire had not dragged through the snow/grass for any distance, the slight taxiway excursion had occurred only at the end of the sliding, and this is probably what finally stopped the aircraft. 

I initiated a conference call between dispatch, maintenance control, and the Chief Pilot on Duty. I summarized our situation and requested all parties' concurrence with our plan to be towed to the gate with passengers on board if all 3 of the following conditions could be met:

1. Local Maintenance (MX) inspected the aircraft and determined it was safe to tow.

2. Airport operations finished re-treating the taxiway between us and the ramp and reported an acceptable braking action.

3. The tug driver and I directly coordinated, and in his or her unpressured opinion, it was safe to tow the airplane.

All parties concurred with this plan. Dispatch asked about tarmac delay service. The FO and I discussed and concurred that this would be acceptable as long as no carts were used, in order to keep the aisle clear. I requested the lead FA conduct the tarmac delay service without using any carts and made a PA about this. 

A mechanic arrived, spoke to us on the interphone, gave an initial impression of no visible damage, and stated he would now begin a detailed inspection/coordination with Maintenance Operation Control (MOC), and would report back when done. 

While we waited for the MX inspection and taxiway surface treatment, I made PAs approximately every 15 minutes with updates. I spoke with numerous parties in Operations Control (OCC) several times on the phone. We reported tarmac delay start and completion time, which was within the regulatory limit. On the phone calls, dispatch became increasingly concerned with the tarmac delay rule and deplaning passengers within the 3 hour limit. I emphasized that complying with the tarmac delay rule was *a* concern, but passenger safety was the highest concern. I expressed my position that keeping the passengers on the aircraft was a safe, controlled environment with lighting and heat. The condition of the ground was extremely slippery, and there was the risk of passenger injuries and/or losing control of the situation on the ground if we deplaned in our current position. At this time, a strong right crosswind existed, with the windsock sticking straight out. Visibility had reduced and light snow was falling/blowing sideways right to left.

My position was based on my understanding at the time that it would be safe to tow with passengers on board, and the issue was simply how much time it would take to be ready to safely do this. We came to an agreement that busses and airstairs should be standing by if we decided to deplane on the taxiway, but we had not made the decision to do this at this time. The FO relayed this to the FBO/station operations.

The phone calls became more frequent and reached a level where they distracted from management of the actual situation. I began having to accept calls, but put callers on hold until I had time to talk to them. I spoke to [the Chief Pilot] directly on his company cell phone, and he agreed to become the single point of contact between me and OCC. This tremendously improved communication and stopped the hazardously escalating level of distraction.

During all of this time, the FO was primarily responsible for radio communication with Tower/ARFF and the FBO/station operations. He also queued up flight attendant and ground.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

Airbus A320 flight crew reported that after landing, during taxi, they encountered ice on the taxiway.

ACN: 1409924 (1498 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Windshear
Weather Elements / Visibility.Visibility : 0.25
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : NYC

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : NYC

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 165
Experience.Flight Crew.Type : 4679
ASRS Report Number.Accession Number : 1409924
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 372
ASRS Report Number.Accession Number : 1409939

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

On takeoff from Runway 22R, reported ATIS 1/4 visibility, calm winds reported by tower. Previous departure B757. Normal takeoff roll and rotation. Pitch bars commanded extreme nose up. Wind increased to 20 kts by 100-150 ft, 50 kts by 300 ft, 82 kts by 500 ft. Pitch bars commanding extreme nose up, aircraft attitude was 20-25 degrees nose up and not yet at power reduction altitude (800 ft). Hit wake turbulence somewhere around 500 ft in turn and had a 2-3 kts momentary overspeed. Autothrottles on. Continued nose up rotation but hesitant to over-rotate as aircraft was extremely nose up with positive wind performance. Had second momentary exceedance in turn when encountered wake turbulence from previous aircraft leading to second exceedance (momentary overspeed). Just after power reduction, rapid trend arrow indicating aircraft entering negative effect of relative wind, VLS increased to just under aircraft speed (5 kts below). Thrust applied to accelerate above green dot speed. Aircraft given 290 heading by ATC and turned west and departed in light to moderate turbulence conditions. Exceedance reported to [Maintenance] at cruise. Consulted with [Maintenance who stated] no issues. Proceeded to [destination] and landed uneventfully.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported a flap overspeed shortly after takeoff when they encountered a rapidly increasing headwind and wake turbulence from the preceding B757.

ACN: 1409916 (1499 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1409916
Human Factors : Physiological - Other
Human Factors : Fatigue
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise we noticed an annoying noise in the cockpit that only occurred after the aircraft leveled off in cruise flight and the auto thrust retards the thrust from climb to cruise thrust, the sounds a normal 320 engine [type] aircraft retarding the engine thrust to idle when the fans rumble temporarily.

However on [this aircraft] the rumble of the unsynced fans is continuous and low frequency to the point where you feel it internally. When the [other type] engines are at full power there is no low frequency rumble, [and] when the engines are at idle, there is no low frequency rumble. But when the engines are in cruise thrust, there is a constant low frequency rumble that constantly comes in and out [and can be] heard/felt in the cockpit.

When I stepped outside the cockpit to use the lavatory, both the Captain and I noticed that you cannot hear the fan rumble in the passenger cabin (or at least in the forward galley/forward lav area). Upon returning to the cockpit after a lavatory break, we were subjected to the unsynced fans again.

We didn't notice it during the [first] leg, but it was becoming annoying by the [second] leg and by the time we were flying [the last leg] I had a throbbing headache and I found it difficult to perform my pilot duties and concentrate on the tasks at hand.

I also submitted an accompanying company fatigue report. Investigate the low frequency unsynced fan harmonics on the engines. Limit number of legs crew fly [this] aircraft.

Synopsis

A320 First Officer reported experiencing distracting headaches and fatigue from what he described as a continuous low frequency rumble from the engines in cruise flight.

ACN: 1409471 (1500 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1409471

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1409473

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After departing, climbing through FL360, we got an ENG 2 AIR SYS FAULT message on the ECAM. We followed ECAM guidance, and pulled the #2 thrust lever to idle. At this time we also coordinated with Center to stop our climb at FL360. As airspeed began to deteriorate, I set the No. 1 engine, which did not improve the situation a great deal. The FO quickly found the procedure, which we applied. This procedure leaves the affected thrust lever at idle. We decided that this was not the proper aircraft configuration to continue to [destination]. We continued our coordination with ATC, requesting further descent, and a return to [departure airport]. The weather was VFR, and it seemed to us to be the correct place to take the aircraft, given the situation. 

It should be noted that when the No. 2 thrust lever was moved out of idle, the ENG 2 AIR SYS FAULT ECAM returned, directing us to return the thrust lever to idle. The engine was not shut down, but we were left without the majority of its thrust, basically rendering us engine out. Given this fact, and our pressing need to descend, I [advised ATC], in order to expedite our descent and return to [departure airport]. As we began our descent and return, we coordinated with Dispatch, briefed the Flight Attendants, and made an announcement to the passengers, explaining that we would be returning to [departure airport]. We requested and received clearance for a visual approach. An uneventful landing was made, and an inspection of the aircraft was conducted prior to taxiing to the terminal. Nothing unusual was noted by the Fire/Rescue truck, so we continued to the gate.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported returning to the departure airport after receiving ECAM message "Eng 2 Air Sys Fault" that resulted in operating engine 2 at idle thrust for the rest of the flight.