ACN: 1409225 (1501 of 2908)

Time / Day

Date : 201612

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Rudder Control System
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
ASRS Report Number.Accession Number : 1409225
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Primary Problem : Procedure

Narrative: 1

When I examined the rudder rod ends to replace them with upgraded with grease fitting rod ends per Engineering Authorization (EA); I found that the rod ends that were installed previously on aircraft X were not safety wired. I initiated non routine [check] to make sure that rod ends were safety wired before the aircraft left the hangar. I wanted to ensure that the aircraft would be safe in flight. 

I am concerned that there might be other aircraft in our fleet or in other carrier's fleets that are also flying without the rudder rod ends being safety wired. I highly recommend that an investigation to discover where the work on the rudder was performed is initiated and then that all aircraft that were worked at that location or that person be examined to ensure that the rudder is in a safe condition for flight.

I would also like to ask that the individual that performed the work, signed for the work, and inspected the work be informed that the safety wire was not installed to make sure that they know it should be to prevent this from occurring in the future again.

Suggested Resolution: 'I am concerned that there might be other aircraft in our fleet or in other carrier's fleets that are also flying without the rudder rod ends being safety wired. I highly recommend that an investigation to discover where the work on the rudder was performed is initiated and then that all aircraft that were worked at that location or that person be examined to ensure that the rudder is in a safe condition for flight. I would also like to ask that the individual that performed the work, signed for the work, and inspected the work be informed that the safety wire was not installed to make sure that they know it should be to prevent this from occurring in the future again.'

Synopsis

A Maintenance Technician reported that during an examination of the rudder rod ends on an A320, a discrepancy was found that the rod ends that were previously installed were not safety wired.

ACN: 1406185 (1502 of 2908)

Time / Day

Date : 201612
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Engine Air Indications
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Checklists
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1406185

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1406362

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

Just started our descent into ZZZ. Got ENG 2 AIR SYS Fault. Had put the #2 thrust lever to idle, per the ECAM. Ran com. Message persisted so we declared and landed with 2 engine at idle. 

The engineer that the captain spoke with on the phone after the event, made it sound like we could have used the engine? The Operations Manual (OM) procedure just sorta left us hanging. Didn't say we could try the engine, use the engine or shut down the engine. Or to land As Soon As Possible. I think there needs to be another action step maybe? If message persists or not... If the engine was fine, would have been nice to have. 

They ended up just deferring the issue, but didn't actually do anything physically to the engine to fix. Just a paperwork deferral. So that leads me to believe we just added risk to our approach and landing for no reason? Maybe the OM procedure is fine. But a little more info in the OM, in the moment, would have been helpful to me. Recommend looking at it.

Narrative: 2

At start of descent received an "ENG 2 AIR SYS FAULT" ECAM. ECAM called for thrust lever 2 to idle, did that, ECAM message remained. Followed SOP and referenced the OM procedure. Nowhere in the ECAM or OM procedures does it ever say to bring the engine back out of idle, so we left it at idle, [advised ATC] and landed in ZZZ without incident. After landing, spoke with Maintenance who connected me to [an engine manufacturer] engineer who was in the office since this was a new aircraft. The engineer said we could have brought engine out of idle. However, that would have been against the ECAM and OM. So there is some conflicting info out there about this fault. 

Look into our OM procedure to make sure it is correct. I feel as if the procedure led us to perform an unnecessary single engine approach and landing. But nowhere in OM does it say to take the engine out of idle.

Synopsis

A320 flight crew reported receiving an "ENG 2 AIR SYS FAULT" on ECAM during descent. The published procedure did not provide sufficient information, so the crew landed with the engine at idle thrust.

ACN: 1405345 (1503 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1405345
Human Factors : Physiological - Other

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

Depart hotel at XA:00 for a XB:00 pushback. As usual, no breakfast available at hotel. In airport terminal (XA:20 local) the only food type vendors open [is] a coffee stand that had coffee only. No food, no muffins, no doughnuts, etc. No breakfast, hot or cold, not even the laughable frozen solid crew meal that typically is offered/presented at the time of closing the fwd cabin door and yet the two of us are expected to do a 2+ hour flight, then be at the top of our game for a CAT III 600 RVR at destination! Really? Though I have used this particular flight/pairing, these conditions/events are common/typical on every trip. The company's hypocrisy is demonstrated on within [a Company publication], the human physiology chapters of sleeping and eating [within] a consistent agenda; being 'fit for duty'. Before even pushing back from the gate, we are 'in the red'. 

We are responsible for the lives/safety/wellbeing of 150+ persons, to deliver them and crew safely/alive/intact. Yet we are expected to rise under many different time zones, perform standard/optimum, no breakfast/food. "Avoiding recurrence of this event", well, the construction of pairings, the Company's mindset of crew accommodations/rest/meals availability, all would have to improve significantly.

Synopsis

A320 Captain reported his schedules often do not offer an opportunity to eat prior to departure.

ACN: 1404560 (1504 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 6400

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Flight Phase : Descent
Route In Use.STAR : SBONO1

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 1007.72
ASRS Report Number.Accession Number : 1404560
Human Factors : Situational Awareness
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7750
ASRS Report Number.Accession Number : 1404533
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

SOCAL gave us 7000 feet and cleared for the RNAV VISUAL RWY 31L. With Auto Pilot on the Captain started his decent after SBONO. Between SBONO and WEMIR at about 6400 feet we received a Terrain Warning Terrain, Terrain. Captain applied procedure until clear of conflict. The warning lasted only for about one second. The aircraft was above crossing altitude at all times but one thing I noticed was the RNAV VISUAL approach (Brick) was below the crossing altitudes. The Captain hand flew the approach after the warning.

Narrative: 2

On the RNAV Visual 31L we crossed SBONO at 7000 and started down to 6000 for the crossing altitude at WEMIR. I started to decelerate, working on being on altitude and configured by ULALE as directed. I believe I drifted above the glide path guidance as I slowed and was planning flap extension. Nearing WEMIR we received 1 terrain terrain. I started pulling the nose up, autopilot off, wings level and pushing on the throttles but before I could call TOGA or move the throttles out of the detent, the warning stopped. FM states to fly the maneuver until the warning stops and terrain clearance is assured. As the warning had stopped and there was enough light to see we were clear of terrain, I did not continue to fly the escape, made sure we were at or above route altitudes and resumed the approach guidance.

Synopsis

A320 flight crew experienced a terrain warning at 6,400 feet during the RNAV VISUAL RWY 31L at PSP, with the Captain flying. The Captain took evasive action although the aircraft was above the FMS glideslope (brick) but below the MSA.

ACN: 1404256 (1505 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2600

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : ZZZ
Airspace.Class D : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1404256

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

When climbing through 2600 ft on are way up to 9000 ft we encountered a flock of geese without warning. Upon impact we had a high engine vibration on the number one engine and the flight attendants reported a foul smell after a bird got sucked into the number 2 pack inlet. We [advised ATC and coordinated a return] with an overweight landing. After landing the emergency trucks inspected the aircraft for damage letting us taxi to the gate.

Synopsis

A320 First Officer reported a return to the departure airport after sustaining damage to the #1 engine and #2 pack inlet door.

ACN: 1403921 (1506 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1403921
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1404157
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Pushed off gate with normal indications. Taxi to Runway 36. ECAM brakes and nosewheel steering Inoperative. Reset nose wheel steering, all indications normal. Continued taxi to 36. ECAM brakes and nosewheel steering became inoperative again, unable to brake or steer. Switched nose wheel steering off, applied brakes and reverse to stop aircraft. Stopped on taxiway, set brakes, called Maintenance to have us towed back to gate. Contacted dispatch and were pulled to gate. Aircraft swap and on our way. Maintenance met us at gate, entered information in Aircraft Maintenance Log (AML).

Narrative: 2

After pushback the Captain elected to start both engines to get the five minute warmup for the first flight of the day. As the ground crew cleared they showed us the Nose Wheel Steering (NWS) nulling pin and gave the Captain a salute. ATC cleared us to the spot and told us to contact Ground. After the Captain made the 90 degree left turn and moved about 100 feet a BRAKES SYS 1 FAULT ECAM indicated followed by a BRAKES NWS STEERING FAULT ECAM and the Captain said aloud "I can't steer." He was able to stop the aircraft and we accomplished the ECAM procedures and follow up. Resetting the NWS/ANTI-SKID cleared the fault and we continued taxiing with full brake and steering effectiveness. After crossing the runway on the taxiway we again got a BRAKES SYS 1 fault. I notified ATC that we needed to hold somewhere to talk to Maintenance. ATC instructed us to proceed to the south cargo area to troubleshoot. After making the turn south onto the taxiway the Captain said "there goes the steering. I've got no brakes, nothing!" At this point we were traveling about 10-12 knots ground speed and the aircraft began to track left of centerline and onto a taxiway heading toward the grass and the runway. We initiated the loss of braking from the inflight maneuvers chapter in operations manual and after cycling the parking brake handle and engine reverse we were able to stop the aircraft on the taxiway and an excursion did not occur. We notified ATC, company and maintenance and a super tug came out to tow us back to the gate.

My guess is a failure of Brake and Steering Control (BSC1).

There was no indication on the ECAM that the steering was bypassed and the ground crew showed us the pin. For a period of time the brakes and steering functioned normally. I will say there is a delay after the Anti-skid/NWS switch is turned off for braking to resume. This is a little concerning when you are approaching the grass and an active runway. However, the procedure does work and we were able to stop the airplane using the procedures in operations manual.

Synopsis

An A320 crew reported a Brake and Steering Control Unit (BSCU) failure while taxiing for takeoff. The first brake and steering failure was reset, but the aircraft was towed to the gate after the second occurrence.

ACN: 1403827 (1507 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : OMA.TRACON
State Reference : NE
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : OMA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class E : OMA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 839
ASRS Report Number.Accession Number : 1403827
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16900
Experience.Flight Crew.Last 90 Days : 193
Experience.Flight Crew.Type : 1700
ASRS Report Number.Accession Number : 1403824
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

When we first checked in with OMA approach, they asked us to keep our speed up because we were number one. Two other aircraft checked in behind us. Soon after the check-in we were told to descend to 4000 feet. We were at 250 knots about 20 miles away from the airport. Once abeam the tall antennas that are northwest of the field we were told to descend to 3500 feet. Speed was 250 knots at approximately 13 NM and we were beginning to slow at this point. Also at this point, we stated that we had the airport in sight. We were cleared for the visual approach to runway 14R. We had the ILS 14R in the box as a backup. We leveled at 2400 feet about 3 NM from ZATAR and slowed through 220 knots and flaps 1. At about 2 NM we were still at approximately 200 knots and slowing with speed brakes deployed. Several things happened simultaneously and we descended below glideslope. Flaps were set at the appropriate speeds and the Landing checklist called for. At about 3 NM from the field we noticed the glideslope deviation and began getting the "glideslope" aural warning. I immediately corrected and pulled the nose to level. I recall seeing 1600 feet MSL on the altimeter somewhere between 2 - 3 miles from the runway threshold which was approximately 200 to 300 feet low. The Landing checklist was completed and we were back on glide slope by 500 feet. The landing itself was smooth and we exited the runway.

I should have told [ATC] that we needed to slow sooner. I should have had better awareness of our proximity to the airport. That would have prevented trying to slow down while going down and prevented the excessive descent. A go-around when glideslope warning were sounded should have been initiated. Several lessons have been learned from this.

Narrative: 2

If I had it to over again: I should have been more vocal. I knew we were fast as we passed the antennas and should have said so. ATC is always asking us to keep our speed up but we should just do what we need to do without feeling pressured. I just assumed the Captain felt the same way because he was taking action to slow down but I think I would have been more aggressive. We did have trouble seeing the airport at night so we both probably thought we were farther away than we really were. I should have suggested a go-around at ZATAR. It would have been a "soft" go-around at that point and no big deal. It would have delayed us by 5 minutes at the most. Finally, once I noticed the glide slope deviation I should have just stated "go-around". It would have been a more aggressive maneuver than a soft go-around but again, no big deal. We debriefed all of this after the flight.

Synopsis

A320 flight crew reported descending below the glideslope on approach to OMA, citing ATC requests to keep speed up as contributing.

ACN: 1403720 (1508 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Main Gear Wheel
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1403720
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

[The aircraft] departed, then taxied out towards [the runway] when he pulled off the taxiway and called for maintenance to come look at his #2 wheel due to increasing brake temp, and an earlier report from the pushback crew that the tire was "wobbling". [Three] mechanics responded and drove out to the south cargo ramp, where it was immediately obvious that something was very wrong with #2 tire, it was cocked at an odd angle and on closer inspection there was torn metal on the inside of the wheel, with possible damage to the brake assembly. A decision was made to offload the passengers onto busses, transfer cargo to a new aircraft, and the hangar crew was called to replace the wheel and assess damage to the brake and axle.

My problem with this whole event is that the pilot, told the Hangar Crew Chief that on pushback, the ramp tug driver had called up to him on intercom that they were halting the pushback prematurely to have the #2 wheel checked out because it was wobbling. Then a few minutes later, he informed the pilot that the tire had been checked out and was ok. No call was made to maintenance, so I am guessing that the rampers checked the wheel themselves, or maybe called THEIR lead, and they determined the wheel was serviceable. The tug driver then informed the pilot that the wheel had been checked, and was "ok", which led the pilot to believe that the wheel had been checked by maintenance, which it was not. This is far beyond the scope of their job, and in this case, could have caused a serious, if not catastrophic accident. The wheel was obviously damaged, and if the ramp thought the wheel was "wobbling" maintenance technicians should have been called immediately to check it out.

Passengers had to be offloaded onto busses on the south Cargo Ramp, cargo downloaded and catering accomplished there as well. Hangar crew removed #2 wheel, inspected axle, and replaced #2 wheel and brake assys.

What I would like to see happen is for the ramp pushback crews to be properly trained on when to call maintenance, some of them are quick to call us out for non-events like scuffed paint and dents that have damage file stickers right next to them, but again, I feel this is event was far more serious, and I am thankful the push crews' failure to call maintenance didn't cause any further damage to the aircraft or injury to our passengers.

Synopsis

A Lead Technician reported that when called out to check the Main Landing Gear wheel on an A320, due to high brake temperature, found it unserviceable after the ground crew released the aircraft calling the wheel okay.

ACN: 1402872 (1509 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SLC.TRACON
State Reference : UT
Altitude.MSL.Single Value : 4200

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Airspace.Class B : SLC

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SLC

Aircraft : 3

Reference : Z
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Operating Under FAR Part : Part 121
Flight Plan : IFR
Airspace.Class B : SLC

Aircraft : 4

Reference : A
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Operating Under FAR Part : Part 121
Flight Plan : IFR
Airspace.Class B : SLC

Person

Reference : 1
Location Of Person.Facility : SLC.TWR
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 21.0
ASRS Report Number.Accession Number : 1402872
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Detector.Person : Air Traffic Control
When Detected : Taxi
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

I was working the Local Control West position. Most of the equipment was fully functional, however the Local Control West ASDE-X (Airport Surface Detection Equipment) scope was ONLY operational in one specific configuration. When I asked when it was going to be fixed I was told that the part that allows us to reconfigure the ASDE-X to recall our specific Pref Sets was not operational and that AF (Airway Facilities) didn't have any funds available in order to purchase the parts required to fix it. It has been out of service since last week.

When Aircraft X arrived at the runway for departure I coordinated for a Departure hole. I was informed that my hole was behind Aircraft Y on final. 

I instructed Aircraft Z to Turn Right at Alpha 6 and contact ground. Aircraft Y landed RWY 34L right behind Aircraft Z, and I instructed Aircraft X to Line Up and Wait RWY 34L, and to be ready for an immediate, with traffic 4 mile final. I then instructed Aircraft Y to turn right at Alpha 6 and contact ground. However after Aircraft Z exited RWY 34L he came to a complete stop just past the hold short lines as Aircraft Y was exiting. I then instructed Aircraft Y to expedite to Alpha 7, turn right, and contact ground but it was too late. 

I instructed Aircraft X to exit the runway and cancelled Aircraft A's landing clearance, Aircraft A acknowledged. When Aircraft B checked in I scanned out to the Final and told Aircraft B to continue. As I scanned back to the ASDE-X to ensure Aircraft X was indeed exiting the runway, I had to look twice; (Since my ASDE-X Display was stuck in a configuration that was a full 180 degrees different than what my personal Pref Sets would have been.)

I had to instruct Aircraft X to exit the runway again. Aircraft X acknowledges the instructions that time, but coasts past TWY Alpha 2 on their way to Alpha 3. I then instructed Aircraft A to Go Around to the left side of the runway for traffic. Aircraft A does not acknowledge but the Aircraft begins to climb and sidestep left. I immediately verified the Go Around instructions with Aircraft A and they acknowledged. I then gave climb out instructions to Aircraft A and a speed restriction. He acknowledged. 

Once again I issued instructions for Aircraft X to exit at Alpha 3 and taxi via TWY Alpha to RWY 34L. Aircraft X acknowledged. I then issued a turn to Aircraft A and he acknowledged. Aircraft A was then shipped to Approach Control.

I feel the dysfunctional configuration of the Local Control West ASDE-X display was a contributing factor in not sending Aircraft A around a couple seconds earlier; since I had to look twice to locate Aircraft X on my ASDE-X to verify he was actually exiting the runway. As I stated before, my personal Pref Sets are exactly 180 degrees different than what was currently displayed on the ASDE-X scope. When I work the Local Control West position I usually look at the bottom of my scope to see RWY 34L in my configuration and was having to force myself to look at the top instead. Since EVERY SECOND that goes by is critical at times, I feel the ASDE-X scope was a SAFETY HAZARD for me in its current state.

Furthermore I find it ridiculous that a Government Entity as large as the FAA, couldn't spare a few dollars by repairing a highly critical piece of radar equipment that we depend upon on a daily basis to ensure the safety of the flying public.


If I had not loaded Aircraft X for departure, it might have worked out, but I doubt it since Aircraft Y was not able to completely exit the runway anyway.

The ASDE-X is a highly critical piece of equipment used for separation in the tower and should not be allowed to operate without immediately being repaired when it breaks down. 

My recommendation: Keep highly critical equipment parts to repair the equipment such as the ASDE-X on hand if able or release the funds necessary in order to have the equipment repaired in a timely manner.

Synopsis

A Tower Controller reported late go around instruction due to equipment limitations distracting him.

ACN: 1402858 (1510 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : HCF.Tower
State Reference : HI
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : HCF
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class C : OGG

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : HCF
Aircraft Operator : Air Taxi
Make Model Name : Caravan 208B
Operating Under FAR Part : Part 135
Flight Plan : VFR
Mission : Passenger
Flight Phase : Climb
Route In Use : VFR Route
Airspace.Class C : OGG

Person

Reference : 1
Location Of Person.Facility : HCF.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 17.5
ASRS Report Number.Accession Number : 1402858
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

As I was completing recurring training, I received a page from a supervisor stating I needed to report back to the TRACON Control Room. I was in shock when the supervisor, took me into his office to tell me that I had a loss of separation. I had no such loss of separation. He played back the tapes of, an IFR A320, a VFR C208. I was working "HCF Approach Sector 10." This is Class C airspace. The Supervisor is not certified to work R10, so he must be unfamiliar with the rules, but I performed Class C services per 7-8-2 a.3. of the 7110.65 which states:

7-8-2. CLASS C SERVICES 

Class C services include the following: Separation, traffic advisories, and safety alerts between IFR and VFR aircraft. 

[The A320] had departed OGG, after [the 208], and I issued traffic to [the A320], who stated he had [the 208] in sight. I used section 7-2-1 of the 7110.65 to tell [the A320] to maintain visual separation from [the 208]. [The A320] read back he would maintain visual separation from [the 208] and I told [the 208] that [the A320] had him in sight, and he was maintaining visual separation. All this is in accordance with 7110.65 7-2-1 "Visual Separation."

This is also in compliance with this section of the 7110.65: 

7-8-3. SEPARATION
Separate VFR aircraft from IFR aircraft by any one of the following: Visual separation as specified in para 7-2-1, Visual Separation, para 7-4-2, Vectors for Visual Approach, and para 7-6-7, Sequencing.

I had [the supervisor] measure the closest distance that the A320 and the C208 came to each other, and it was TWO MILES, plus the A320 was MAINTAINING VISUAL SEPARATION from the C208.

I had several different types of separation being applied simultaneously, and I do not understand how the Supervisor thinks it is possible that I had a loss of separation. I believe the Supervisor should read the FAAH 7110.65, especially Section 8, which applies to Class C Service and Section 7-4-1 which applies to visual separation.

Synopsis

HCF TRACON Controller was told by their Supervisor that they had a loss of separation. The reporter stated they did not have a loss of separation.

ACN: 1402730 (1511 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
ASRS Report Number.Accession Number : 1402730

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after takeoff our #2 generator dropped offline followed by an Oil Lvl Low IDG2 ECAM message. We completed the ECAM requirements resulting in the disconnection of the #2 IDG and the disabling of the #2 generator. Prior to pushback, but after the before start "flow"/checklist, one of two center tank fuel pumps failed and the subsequently MEL'd by maintenance. There were other issues with this aircraft including an autopilot aircraft speed set control that cycled between "managed airspeed" and (pilot) selected airspeed without pilot input. While all busses were powered by generator #1 and the APU generator, and with coordination with the dispatcher, operations and maintenance, I elected to return for an overweight landing. It was/is my belief that the safest course of action was to return for maintenance and repair of the IDG/Generator.

Synopsis

A320 Captain reported returning to departure airport after experiencing loss of #2 generator shortly after takeoff.

ACN: 1402528 (1512 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y

Component

Aircraft Component : INS / IRS / IRU
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1402528
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During engine start ADR 1 faulted repeatedly. Contacted Maintenance Control and did a successful com reset followed by an info only write up. Within a minute of taxi ADR 1 faulted again. Contacted Maintenance Control. They wanted to do another reset but I declined. This had been written up previously as well. We returned to the gate where maintenance began another reset. I expressed my concern that we had a history of resets and failures...they said well then we are going to defer it. They swapped number one and three ADR as one is non deferrable. Upon review FO and I became concerned as the MEL states effectivity A320 and subsequent. We were in A320. Contacted Maintenance Control, he was unsure and consulted his Supervisor who said it's good. I asked where I could find a reference to this as FO and I had been digging through the MEL preamble to confirm the validity. We could not. We then contacted Dispatch, asked for both amendment for time and MEL and expressed our concern over the verbiage. I then contacted crew schedule and asked to be transferred to the chief pilot on duty. They asked if I wanted the number and I said yes but also to transfer me. I was connected to a chief pilot not on duty. He asked me (reasonably) to contact the chief pilot who WAS on duty. I called the number provided by schedule and again was greeted by the same voice. He provided me with the number for the chief pilot on duty. I then called him. He referenced the MEL and did some research and ascertained we were good to go. Shortly after liftoff ADR 1 failed resulting in a cascade of additional failures. Gear stuck down, direct law, man trim, no auto thrust and of course, no instrumentation for the Captain (PF). I established a positive flight trajectory, confirmed the FO had reasonable indications from ADR 2 (our newest best friend) and transferred control. We established a box pattern while we worked through the ECAM and com including landing distance procedures. We returned to the departure airport, chose to brace the passengers, and landed uneventfully. Back end crew did an outstanding job managing passengers as well as following SOP and protocol. FO displayed excellent airmanship and CRM. Truly did a great job.

I felt as though we were completely on our own trying to resolve the maint issue prior to flight. During the event dispatch had ATC relay that they wanted us to contact them. Just the teeniest bit too busy for that. Seems like they might have preemptively sent us an acars with landing data as a backup to the charts. That would have been helpful.

Synopsis

During engine start an A320 ADR 1 failed so the crew returned to the gate where Maintenance moved ADR 1 to the ADR 3 position, and ADR 3 to ADR 1 position. ADR 1's history included multiple resets so the crew objected but were released by the Chief Pilot and Maintenance. After takeoff, ADR 1 and ADR 3 both failed, so the flight completed the ECAMs and returned to land.

ACN: 1402508 (1513 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1402508
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

An Automated Delay came across my Screen Printer. I immediately took action and called OPS to gather more information. Once I was informed it was due to late bags, I went to put in the correct delay. Unfortunately they posted an out time right before I hit enter. Now I am unable to change the delay without deleting the out time and manually entering the correct out time.

I have been informed that AUT (Auto) delays have been causing incorrect Minimum OFF Time calculations - if not corrected. So if that is truly occurring; and I have this happen on my desk alone at least TWICE a day if not more. Sometimes I have the time and energy to write a report - most days I do not; and many dispatchers do not seem to care. However, one has to wonder how this is effecting FAR 117. Unnecessary flight cancellations due to discrepancies and flights potentially departing illegally. 

Also, since the introduction of the AUT delay, how is that allowing the company to track the real reason for delays so that supervisors can be alerted to systemic issues that need to be addressed in a timely manner?

And most off all, the AUT delay is impeding on the Dispatcher being able to perform his/her job correctly according to the FAR's. Being informed on why a flight is delayed and then delaying the flight is huge part of our job. Rarely do I have a pilot or a person in ops update me on the status of a flight even though they are right there with the most information on what is occurring with the flight.

The AUT could be a useful tool if it was used to alert the dispatcher of a delayed flight and not actually post uninformative delays. The AUT needs to be an alert message only.

Also the creation of a record history for each flight that allows for the monitoring of delays, crew, maintenance, flight interruptions, notes, etc to be found in one place which can be accessed by dispatcher, gate, ops, unit, bridge, etc. Or provide training on using the Record History in Sabre as a flight monitoring tool for all those involved which may help alleviate unnecessary phone calls and allow for an increased workload to be executed more efficiently.

Synopsis

A Dispatcher reported the Automated Delay alert once activated cannot be easily overridden and after an aircraft out time is established the flight's clock is running even at the gate which can affect FAR 117 crew legality as well as other company and FAA considerations.

ACN: 1402184 (1514 of 2908)

Time / Day

Date : 201611

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 10433
ASRS Report Number.Accession Number : 1402184
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8072
Experience.Flight Crew.Last 90 Days : 217
Experience.Flight Crew.Type : 1112
ASRS Report Number.Accession Number : 1402172

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

While climbing through FL330 our jumpseater directed our attention to my windshield which was sparking on the upper edge. The First Officer was flying and asked if he should level off. I said yes and informed ATC of same. I immediately realized we had an issue so asked to descend to FL180. The sparking continued to grow and in fact started a fire in between the panes then continued on down to the lower right edge. After a few minutes there was a loud popping sound and windshield cracked. We had donned our mask right after noticing the issue. [The] First Officer pointed to [an airport] on the ND which was to the left and only 60-70 miles away. We [communicated) with ATC and asked to divert to [that airport]. FO took the radios from me so I could communicate with the flight attendants, passengers and send a message to Dispatch. After sending a message with our intention to Dispatch, I pulled up the WX [and] set up the FMGC for our approach. We did a quick briefing and FO continued to fly the approach and make an uneventful landing. After clearing the runway, emergency equipment looked over the aircraft and we proceeded to the gate. 

Some items standout from our experience: from beginning to the end the cockpit was very busy. I was completely consumed by organizing the diversion and communicating our plan with all involved. The O2 mask was difficult to work with even though both FO and I are/were very comfortable with donning and familiar with the required radio switches to use. The mask definitely added a dimension to an already challenging environment. The support from ATC, emergency services and the company was outstanding. We did forget to write up the window heat ECAM and windshield cracking while still in the cockpit but the mechanics who met us were already communicating with [Maintenance] and I called Dispatch after talking to the passengers and debriefing the flight attendants and had phone patch to [Maintenance] who confirmed they had the necessary write-ups.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported diverting to a nearby alternate after the Captain's windshield sparked and then shattered.

ACN: 1402178 (1515 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 128
Experience.Flight Crew.Type : 476
ASRS Report Number.Accession Number : 1402178
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise flight, we received a HYD Y RSVR LO LVL ECAM. As Pilot Flying, I kept aircraft controls and ATC communications and directed First Officer (FO) to accomplish the ECAM and then QRH procedure. Yellow hydraulic quantity was confirmed low and ECAM actions expeditiously accomplished. Resulting System Status after procedural completion was Y ENG 2 PUMP LO PRESS, Y SYS LO PR, and BRK Y ACCU PR MONITOR. Crew duties were assigned and FO was given aircraft control and ATC duties while utilizing our Dead Head (DH) FO in jump seat as backup eyes and ears while I initiated a Dispatch Call Me for coordination with dispatch and [maintenance]. Radio communications via ARINC were problematic (spotty reception, dropped calls, bad audio quality) during the entire coordination exercise. While radio patch was ongoing, we received another ECAM. This time it was HYD Y RSVR OVHT. We discussed, as a crew, whether the new flight manual bulletin applied as we were faced with ECAM screen showing Y HYD ENG PUMP LO PR followed by HYD RSVR OVHT (Special). We decided that bulletin did not apply as the LOW PRESS ECAM was a subsequent result of the initial RSVR LO action items and did not occur on opposite systems. We completed the appropriate HYD Y RSVR OVHT ECAM action items and QRH.

We finally received an acceptable radio patch and coordinated with [maintenance] and Dispatch regarding the applicability of the new bulletin procedure. Both dispatch and [maintenance] seemed unfamiliar with the new bulletin, but once they researched the reference provided, agreed that the new FM (special) procedure didn't apply. Power Transfer Unit (PTU), Engine Pump and Electric Pump were confirmed to be off by multiple means and determination was made that the continued overheat indication resulted from delayed heat transfer on the low yellow system reservoir occurring after the PTU was turned off per the initial ECAM procedure. 

In concluding the coordination call, it was determined that [advising ATC of emergency condition] was not necessary. With information from both Dispatch and [maintenance], it was determined that the Dispatcher's calculated landing distance matched that calculated by our DH pilot (in jump seat) and the supplied weather and field conditions posed no significant issues. In conjunction with the remaining hydraulic system redundancies (and 3 member crew), it was determined safe to continue to [destination] for an uneventful, non-emergency, landing.

However, this entire exercise illuminated some Problems of Note: 
1. The placement of Bulletin (special) outside the QRH made it difficult to find.
2. Lack of familiarity with / confusion with the new procedure at crew, [maintenance] and Dispatch levels led to a delay in determining procedural applicability.
3. Lack of adequate clarifying / descriptive notes in two procedures (QRH and FM Procedure (special)).

Recommendations:
1. Expeditiously place FM special procedure in the QRH where it should properly reside.
2. Add clarifying Notes.
i.e. A Note in HYD Y RSVR LO LVL QRH indicating that a Hydraulic Overheat may result after procedure completion and include a reference to the proper OVHT QRH.
i.e. A new Note, or a remark in Condition language of the special procedure, that clarifies/emphasizes/highlights the procedure is only to be run for a condition of Hydraulic Pressure loss followed by Overheat on OPPOSITE systems.

Synopsis

An A320 Captain reported a loss of the "yellow" hydraulic system followed by confusion with a new bulletin that had not been incorporated in the QRH procedure.

ACN: 1400076 (1516 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 15000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : FCU (Flight Control Unit)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1400076
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During climb at approximately 15,000 feet we received an ECAM for FCU (Flight Control Unit) 1 and 2 FAULT. As the Captain and PF I called for ECAM actions and also took communication with ATC. After the ECAM actions were complete I directed my First Officer to the COM (Company Operations Manual). While he was reading the COM procedure I communicated with ATC that we had lost all automation as well as most of our navigation and holding precise altitude would be difficult as we would be using the standby altimeter. I asked for radar vectors to [a possible divert airport] and confirmed VFR conditions in the event we were unable to restore the FCUs. After the FO completed the COM we attempted to coordinate with Maintenance Control for a reset, but had difficulty doing so. We did eventually get the suggestion from them via ACARS to refer to the COM Supplemental. I had the FO brief the lead FA about a possible diversion and that it would be a normal landing. I then switched control of the aircraft to the FO as I had the reset table bookmarked on my iPad and was able to quickly find the applicable procedure. The reset was successful. With the FCU completely restored I [advised ATC] and we determined we had adequate fuel to proceed on to [destination]. We coordinated with dispatch to ensure that we had sufficient fuel. Two legs later the same day we had the exact same failure, however I had left the page with the reset up on my iPad and was able to quickly reset the FCUs without incident after completing the ECAM actions. Both incidents were entered into the logbook as info only write ups and I briefed the First Officer taking our aircraft that evening of the events.

I recommend that we change our training for incidents like this one to first complete the ECAM, then determine if a computer reset is available, and then failing a computer reset, reference the COM. The COM procedure for this ECAM is long and tedious and the PF has his/her hands full. It would also be beneficial if the COM procedure referenced a reset if one is applicable. Teaching these kinds of failures in the simulator would also be a good idea.

Synopsis

A320 Captain reported loss of both flight control units during climb. The reset procedure brought the systems back to normal.

ACN: 1399961 (1517 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 13000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Flap/Slat Control System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
ASRS Report Number.Accession Number : 1399961
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1400049
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

As we were descending on the arrival to cross a fix at 10,000' the A/C was in Managed Decent and Managed Speed. In the decent passing through 13,000' the A/C started to exceed the window of speed the FMCU had for that decent of 290 kts I reached down to start to deploy the speed brakes and as I was watching the speed and did not look down to confirm I had the correct handle started to deploy the speed brakes but in reality had selected the flap handle, as soon as I started to move the flap handle the Flap overspeed sounded and I immediately replaced the handle to the stowed position, the flap handle was not in its transient position for more than approximately 3 seconds and never came close to reaching the Flap 1 detent. once we landed the Captain called MOC and wrote up the overspeed.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

EMB-145 flight crew reported experiencing a flap overspeed due to the inadvertent deployment of flaps in lieu of speedbrakes.

ACN: 1399854 (1518 of 2908)

Time / Day

Date : 201611
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : PHL.Airport
State Reference : PA
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1399854
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During morning pre-flight, side stick gust locks noted installed. Never seen before and both Captain and FO controls had locks. Advised maintenance and the maintenance person had never seen these locks before. A Maintenance Technician was saying that this check is routine and always have the locks in place. No entry in logbook of locks, as would be with gear locks. Oversight from maintenance during evening flight control check.

[Suggest] a maintenance entry in log book! The gust lock is not readily noticeable during late evening, does have flag that hangs down but could be overlooked.

Synopsis

A320 pilot reported both the Captain and First Officer side sticks had gust locks installed with no notation in the logbook. Reporter stated this was unusual and easily overlooked.

ACN: 1399480 (1519 of 2908)

Time / Day

Date : 201611

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1399480

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1399509
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Hydraulic engine 2 yellow pump low pressure. Hydraulic quantity almost completely gone. Contacted dispatch. [Landed at a nearby airport]. Uneventful approach and landing.

Narrative: 2

At XA:52, I received a message from the crew reporting a loss of hydraulic fluid in their yellow hydraulic system. At about the same time, a [maintenance] Controller called my desk to report the event. Flight history shows that the crew also sent a message to [Maintenance] at XA:53, 2 messages to dispatch at XA:57, and another message to dispatch at XA:59. At XA:59, I responded to advise them that [Commercial Radio] would be contacting them to start a phone patch. I sent another message 4 minutes later advising that ZZZ would be preferred for diversion, and received a response confirming that they were diverting. [Commercial Radio] then called me to state that they were having trouble contacting the crew, so I obtained the frequency and sent this to the crew via ACARS. A few minutes later, [Commercial Radio] called back with the crew on a phone patch. But although I could hear the captain, she was apparently unable to hear me and eventually it appeared that she could not hear the [Commercial Radio] operator either. The phone patch was terminated. At this point, the crew was already on descent to divert field.

I later spoke with the captain via phone, and she stated that after she and the First Officer (FO) sent the first message about the hydraulic leak, it took 30 minutes to receive a response. Looking at the message history, I sent a message 7 minutes after receiving her first one, just after I finished conferring with [Maintenance]. I thought that it was odd that so many messages came through in such a short period of time, so my concern is that the messages from the crew did not come through instantaneously, but were delayed for some reason. The lack of response to my later messages also makes me wonder if the crew received them at all, though perhaps this could be attributed to them being busy with other tasks, given the circumstances. I am unsure why the [Commercial Radio] patch was problematic, as I have used them successfully many times in the past.

Whatever the causes, it is obviously a tremendous problem to not be able to quickly establish contact with crews, especially in situations that involve a potential emergency. This occurred while the flight was southbound on the AR's. Could they have been in a sort of "dead one" that prevented their ACARS messages from being delivered in a timely matter? Also, if in fact they did wait an extended amount of time for a response to their ACARS message, why did the crew not initiate a phone patch with [Commercial Radio] themselves?

Synopsis

A320 First Officer reported loss of engine 2 yellow hydraulic system with an uneventful diversion and landing.

ACN: 1398778 (1520 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1398778

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft TA
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Night VMC departure from Runway 01R, assigned ATC clearance SHEAD NINE RNAV departure, which initially calls for a climb heading 010 to 2,681 feet, then left turn direct BESSY intersection.

When clearing us for takeoff, Tower included traffic advisory for helicopter east of runway centerline, no factor. We took off and climbed, and as our flight director commanded our left turn at 2,681 feet, there were multiple TCAS targets 300 feet above us, the closest within 2-3 miles of us, directly in our path if we turn. We cannot see this traffic due to city lights in the background, we are relying solely on TCAS. We continue straight ahead, and notify ATC.

The Tower Controller did not understand the problem, scoldingly saying we are above the traffic, which we were by then. Handed off to Departure Control. We advised the Departure Controller of the now wide turn towards BESSY, he cleared us direct another fix on the SID and we are on our way. If we hadn't had an operative TCAS, we might have turned and hit that helicopter, since we never actually saw it, and was led to believe it was "no factor".

A change of traffic patterns at LAS as implemented by the local ATC facility, this is not a new problem. We were not fond of the tone and attitude from the LAS Tower Controller.

Synopsis

A320 pilot reported deviating from departure procedure track in response to receiving a TCAS traffic advisory of a helicopter in his flight path departing LAS. The Tower Controller seemed unconcerned.

ACN: 1398203 (1521 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Other Documentation

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1398203
Human Factors : Confusion
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1398208
Human Factors : Confusion
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Dispatch
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

After the captain (Pilot Flying) finished loading the MCDU I checked the information. I verified the flight plan route fix by fix and cross checked the total route distance in the MCDU and total flight planned distance on the release and verified that the difference was within approximately 10 NM. I don't believe that the Estimated Fuel On Aircraft (EFOA) number on the MCDU was amber at this point but if it was I somehow missed it.

Required fuel at takeoff was 24,017. Planned ramp fuel was 28,310. Actual Fuel On Board (FOB) upon completion of refueling was 28,400. Actual takeoff fuel was 28,000. The PAX BOOKED section of the release was blank but I thought it was for nothing more than informational purposes. The Estimated Zero Fuel Weight (EZFW) was 94.4 but this didn't raise a red flag for myself or the captain as we have both done multiple repo flights recently.

Our taxi route was A-T-U to rwy XY. Based on the fact that it's not a terribly long taxi as it is, there was taxiway closures and construction, and it was past sunset my workload during the taxi was relatively high. While on taxiway Uniform I switched to the tower frequency and just as I was beginning the Before Takeoff checklist tower stated that there was an aircraft on base and we were cleared for takeoff. In hindsight I clearly should have declined the takeoff clearance and advised tower that we would need another minute or two. Instead I read back the takeoff clearance and allowed myself to get rushed. In doing so I inadvertently failed to reference the dispatch release Estimated Take Off Weight (ETOW) during the gross weight comparison.

During the initial climb we noticed the EFOA prediction on the MCDU was amber. We briefly discussed it and thought that perhaps there was an error in the flight plan or something. We noted that the FOB was still well above our required Take Off Fuel (TOF) so we decided to delay our investigation until an appropriate time above 10,000 feet. 

After passing through 10,000 feet I checked the MCDU flight plan again against the dispatch release. All the fixes, distances, and times were correct but the fuel burn was off. I began recording the predicted time and [estimated] fuel remaining for each fix and continued to update throughout the flight with ACTUAL numbers at each fix. After noting that the difference between the planned vs MCDU predicted Estimated Fuel Remaining (EFREM) was increasing for each successive fix we determined that the actual fuel burn was significantly higher than the planned fuel burn.

At this time we began investigating for possible mechanical issues (such as a control surface deflection, gear door/s open, landing lights extended... ANYTHING) just in case. We immediately ruled out a fuel leak but still continued to monitor fuel burned vs fuel remaining on board. After finding no evidence of mechanical issues we determined that the sole issue must have been the dispatch release and flight planning. 

By this point we slowed to a cost index of 0 and went from showing an EFOA of 1.0 to 1.8. We communicated this to dispatch via ACARS and said that we suspected that the computer may have messed something up on the release and asked if they could run the numbers again on a different computer. (We were told by dispatch to continue to [destination] and that they would keep an eye on it). It was then determined that the flight had been planned with zero passengers and bags. This accounted for a difference of approximately 36,000 LBS between planned TOW and actual TOW. We now knew the reason for the significant difference in fuel burn.

After conferring with dispatch we agreed to divert to ZZZ for fuel. The ZZZ station did an excellent job quick turning us and we continued to our filed destination uneventfully. We blocked in 51 minutes past our scheduled arrival time in our filed destination. 

I've been with this Carrier for about 8 month. For the last (approx) 6 years I've been flying smaller regional jets with shorter stage lengths. In that time I've never had a major problem with a discrepancy between planned and actual loading affecting the fuel burn so significantly. The Airbus is a much larger aircraft with significantly larger loads and longer stage lengths. Unfortunately I was not able to effectively correlate the importance of some aspects of the gross weight comparison during my initial training. To be very honest this was an eye-opening experience for me. I now fully understand the importance of checking the planned weights with the actual weights during the gross weight comparison, how to catch such errors, as well as the repercussions if such errors are not caught. I believe that I have learned a lot from my mistakes during this event and will be adjusting my personal procedures/techniques accordingly so as to ensure that this never happens again.

I also believe people learn best by example and pilots are certainly no exception. I think that sharing a brief narrative of this event along with a reminder of the preventive measures with the pilot group (either as a pilot read file or during recurrent training) would help to prevent future events such as this.

Narrative: 2

Normal preflight/taxi/takeoff. Disclaimer: the PAX BOOKED section of the release was blank. I noticed this when giving the FA brief, but I thought it was merely a glitch. The EZFW was 94.4. This would normally trigger a question for a full A320, but as I have been flying a few repo's lately, it didn't throw any flags for me. This is an accurate EZFW for a repo. The takeoff numbers returned after sending off for them, as opposed to all dashes. On the second page, it said that the ZFW was more than 3% higher than the EZFW. I honestly should have investigated this more, but these messages return all the time, and if there is a large discrepancy, the system will neglect to return takeoff numbers, offering dashes only. Shortly after takeoff, we noticed the fuel on destination prediction had turned to amber. I assumed it was because of a discontinuity/arrival issue, so we continued to climb, and started to trouble shoot after FL180. At this time, we had 2000 LBS more than the minimum required for takeoff, so we did not feel rushed. This also comforted us against the possibility of under-fueling. We inserted current winds to eliminate the possibility of much stronger winds enroute. The First Officer (FO) and I starting comparing planned fuel at waypoints from dispatch vs. airplane FMS predicted fuel at waypoints, and we could plainly see the gap was increasing as the trip went on, obviously showcasing a much larger than planned fuel burn. Additionally, the fuel on destination was about 1.0. We went through all systems to make sure it wasn't something mechanical that we could ascertain, such as a stuck spoiler. The engine parameters were accurate for that flight level and phase of flight. We went to cost index 0 in order to save as much fuel as possible. The FO and I discussed our options, and because of the obvious fuel burn problem, we talked about landing at either of two mid-continent airports, as these were both Company destinations and relatively on our route. We contacted Dispatch over ACARS to tell them of the problem, as well as the mechanics just to make sure they were in the loop. Dispatch said we should continue to our filed destination and we'd keep an eye on it. (The FO and I were already keeping an eye on it, and were NOT going to pass up our last viable divert station showing what was then showing 1.8 fuel on destination). We asked dispatch to run the numbers again using a different system (computer?) and that's when they came back and said that our original flight plan was run using ZFW that neglected PAX or bags, causing our aircraft to be about 31,000 LBS higher than expected. Dispatch then said that we would drop into ZZZ for a fuel stop and continue on to our filed destination. Satisfied that this was indeed the problem, the FO and I both agreed on that plan. We received a dispatch amendment, made the fuel stop in ZZZ, and continued with no incident.

The dispatch release was sent BOOKED PAX as blank. If blank were not an option, i.e. "0" instead of blank, this could have been avoided.

Also, the airplane gave us takeoff numbers when we were showing a 30,000 LB difference, while only saying (on a supplemental page) that there was a >3% difference in zero fuel weights. With that amount of discrepancy, one would assume there would be more attention drawn to the problem by the software.

Synopsis

A320 flight crew reported departing on a transcontinental flight with insufficient fuel due to flight plan not including baggage or passengers.

ACN: 1397893 (1522 of 2908)

Time / Day

Date : 201610

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Emergency Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1397893
Human Factors : Workload
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Detector.Person : Maintenance
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Out of service placard for "ALT R BRAKES RELEASED" ECAM installed in aircraft. After crew inspected logbook it was noted that there were 12 prior resets by maintenance in the last 6 days for the same known issue. When maintenance showed up to the aircraft to address the write up they immediately started doing a random reset. We challenged them on this and said, "There is a problem and the issue needs to be fixed rather than continual resets." Mechanics agreed and started doing more troubleshooting. It was revealed that a brake servo valve was failing when the landing gear was extended. Mechanics asked if we felt comfortable flying with this condition. We said, "Yes, but only if it could be MEL'd." After further review it was determined this fault is not an item that can utilize the MEL. Under ECAM warnings in the A320 MEL book it specifically states that "ALTN L(R) FAULT" is no dispatch.

Maintenance Control was notified by crew stating we were uncomfortable taking this aircraft to a domestic airport. It was suggested that we call the Chief Pilot stating our case. We then called the Chief Pilot for guidance. He said, "As long as maintenance signs it off it is a legal aircraft."

Maintenance Control was then called back with a suggestion from us to swap with another aircraft overnighting. They said, "No, we can't do that because we wouldn't want to send the broken aircraft to ZZZZ. So apparently it was okay for us to take it to a domestic airport, but not ZZZZ?

At this time the 3 mechanics in ZZZ consulted with each other and refused to sign off the aircraft with a computer reset, due to its past and ongoing history.

Maintenance Control was notified and they now decided it was okay to swap aircraft as we had suggested earlier.

Due to the time frame, flight crew was now at the flight duty period limitation. Crew scheduling called both Captain and First Officer asking for an extension. Captain and First Officer agreed to the extension.

To the best of our knowledge there is no limitation for our airline's maintenance that limits the number of times a faulty system can be reset for the same ECAM in a given number of days. Our contract mechanics in ZZZ mentioned that they do work for another carrier also. At the other carrier a fault can only be reset X number of times in X number of days. This seems like a good policy that would apply to this given situation i.e. continuous write ups with a reset to make the problem go away only temporarily when there is obviously an underlying problem causing the ECAM.

Synopsis

A320 Captain reported the ALT R BRAKES RELEASED ECAM alerted on twelve prior flights and was reset by maintenance after each alert. The Captain refused the aircraft, but would fly with the fault deferred. When they determined it could not be deferred, the aircraft was removed from service.

ACN: 1397815 (1523 of 2908)

Time / Day

Date : 201610

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 1546
ASRS Report Number.Accession Number : 1397815

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Security
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

In flight cockpit door security procedure was not followed according to SOP and flight attendant briefly took pilot seat.

Synopsis

A320 First Officer reported that inflight security procedures were not followed when a Flight Attendant was allowed to briefly sit in the pilot's seat.

ACN: 1397639 (1524 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Hydraulic Syst Pressure/Temp Indication
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : INS / IRS / IRU
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1397639
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1397895

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

Flight duty started as normal and waited for aircraft to arrive at the gate. Aircraft arrived on time and after deplaning we made our way onboard for our outbound flight. Upon scrolling through the systems pages I noticed an odd indication on the hydraulic page with the blue system having 2 Amber "X's" below the word blue and the Green and Yellow system both showing the correct Amber pressure reading of 0. I continued through the system pages to the flight control page and noted that the hydraulic system letters representing the blue system, the "B's" were absent from the screen however the indications for the Green and Yellow systems were showing as required for the respective flight control surfaces. I called maintenance to inquire about the indications and no-one had a good idea of what was really happening. We ran through a couple things while Maintenance Control called local maintenance only to get more confused as to what was exactly happening. After running tests maintenance finally found an MEL for the amber X indications and one last test was to completely shut the aircraft down, a "Global Reset", and leave it off for 10 minutes. Maintenance accomplished the task with no fix so the MEL was applied. During the new cockpit preparation we encountered a new issue with OPP FMC IN PROGRESS showing on the number 1 MCDU and the loss of FD 1 on the PFDs. Of course maintenance was still onboard finishing up the hydraulic issue so the trouble shooting began as they tried to figure out the issue.

Long story short, after a couple hours of working this issue, maintenance placed the FMC 1 on MEL and along with it came another 5 MELs. We returned to the aircraft after we were told it was good to go by maintenance and started to go over the multiple pages of logbook entries and new MEL's that we had to deal with and read up on. When we entered the cockpit I had noticed that IRS3 had a solid white "align" light and had started a new 10 minute alignment process on the one IRS. I actually thought that maintenance had restarted the IRS as part of the new list of MELs that were placed upon the aircraft and thought nothing more of it. Upon receiving the final weight and balance paperwork we transferred ground power with a very hard transfer and noticed that IRS 3 had once again fallen offline and automatically restarted itself with a new 10 minute align process. IRS 1 and 2 were both showing no anomalies and remained aligned. Maintenance was present and we waited for it to fully align again and shortly thereafter it fell off again and restarted itself with a new full alignment on that single IRS. It was at this point and now 6 hours into our delay and almost all of it dealing with MEL procedures and adding new MELs to the aircraft that we started to get a feeling that something just wasn't right with the aircraft and began to contemplate denying the aircraft. I began to send messages to dispatch about our problems and concerns and also called the Chief Pilot duty phone to at least let them know where we stood at that point in time. During this time we were trying to convey our concerns to the rest of the crew and the gate agents and that we didn't know if we would feel comfortable with the aircraft as well. We continued to examine the MEL procedures in the MEL and had noticed that when maintenance had placed the MEL on the FMC that it stated that the aircraft was allowed 3 flight legs as a limit however the orange sticker indicated that the aircraft drop dead date was [the next day] (there is actually no drop dead date listed in the MEL, it's a flight leg limit). This essentially meant that the aircraft was now good to get to the destination yet it was not good to return date wise. Upon contacting maintenance they said just go ahead and change it to say 3 flight legs versus a date if that made me feel better. 

This response was another little trigger that had us start to guess what had really been done to the aircraft since the MEL is very specific as to what it states. This issue, IRS and paperwork, was what we believe to have been fixed so maintenance left the aircraft. We started to go over the new MEL for the IRS 3 issue and we noted that the maintenance procedures required some testing of the aircraft that we know didn't happen since they are performed on the FMC. I know this is not our usual procedure to look for maintenance procedures, but it was the only item for maintenance to accomplish and I just happened to catch it as I looked for the operations procedures. We then had to call maintenance back to the aircraft to accomplish this procedure which took longer than expected and after they arrived at the gate we found out that it was shift change and they didn't have anyone there to get to us until the night shift had arrived. During this time the crew timed out for the night and we were released due to duty limits.

Our concerns over the thought to deny the aircraft for the flight were due to the fact that now we had spent 6 hours dealing with MEL's with one that couldn't truly be explained with the hydraulic system since a true issue could not be found, then adding an MEL that takes away FMC 1, AP1 and FD1, and culminating in IRS 3 being MEL'd. Adding in that we were now flying this aircraft with downgraded systems into a short runway to an uncontrolled field at night. True the redundant systems are designed for a reason, but we did not feel comfortable taking an aircraft with known deficiencies into this situation. Upon discussing the possible denial with multiple sources including dispatch and the chief pilot's office we finally came to the conclusion that we would probably take the aircraft and expect the best. Of course during this decision to possibly take the aircraft we were waiting for maintenance to show up for the IRS procedure they had missed. When they finally showed up to complete their procedures we noticed that we now were no longer legal to complete our sequence according to FAR 117 and promptly contacted crew scheduling to let them know we had exceeded our limit. They removed us from the pairing and sent us home.

Additional information:
-One of our flight attendants was distraught over the "issues" we were experiencing and was crying in the aft galley of the aircraft.
-A maintenance employee was there throughout the entire process and actually at one point agreed that he wouldn't feel safe taking that aircraft in its current state either.

In 17 years of flying professionally I have never once denied taking an aircraft due to maintenance or other reason. I don't take it lightly that I have a job to do and I try to do it to the best of my ability. This flight just didn't feel right and things just kept adding up to create a feeling of uncertainty. As the crew who was present for the entire process it was much more detailed and problem filled than if we had just shown up at the airport and had this aircraft come to us this way. We personally witnessed maintenance being unable to find a true issue with our first hydraulic problem versus just seeing an MEL that writes off the Amber X's but states nothing about the lack of other indications on different system pages. This felt like a band aid that tried to cover an issue that was actually more than skin deep. Then the following issues that just happened to occur as we went along did not make us feel very confident in the aircraft.

Narrative: 2

Showed at the aircraft on time after checking the paperwork in the crew room. From that point I touched base with the Captain and completed the exterior inspection and verified with the ramper that we would need him or maintenance to verify that the aft cargo door is closed as per the MEL FR. When I arrived back at the flight deck the CA was on the phone with Maintenance Control about an issue with the blue hydraulic system. There were amber XX's on the blue pump when there should have been zeros as indicated by the G/Y systems. On the FLT CTRL page, the B system pump was not present at all while the other pumps were shown in Amber. I recall asking the CA if he wanted to board and he indicated yes, he was confident that it shouldn't be an issue.

CA and I actually discussed that we had seen this before and reported it to maintenance and I believe we got it go away by doing a couple of resets if memory serves correctly. Maintenance Control had us do a "global reset" for 3 min with passengers boarded up. We accomplished this task to no avail. Same indications as prior. At this point Maintenance Control sent the mechanics to the aircraft. They immediately wanted to do the same thing after turning on the blue pump override, wherein the pump indicated it was in fact functioning correctly but then would indicate XX's again once it spooled down. At this point the maintenance tech's decided to do another global reset for 10 minutes to try to reset it again. This global reset was unsuccessful so maintenance began working on MEL'ing the indication for the blue pump. This entire process of testing, turning off, trouble shooting was easily 2 hours. Once we got the MEL squared away we began our cockpit prep and started everything from scratch to minimize any errors from turning the aircraft off and on. In the process of setting up the cockpit the CA's FMC1 showed OPP FMC IN PROGRESS and FD2-FD2 on both PFD's with a status of AP1 being inop. 

Troubleshooting began yet again which took another hour or so. In the meantime we received word that OPS needed the gate and we deplaned the aircraft so it could be towed off the gate and moved so another flight could be unloaded/reloaded while maintenance worked to square away our MEL of FMC1 which was apparently an extensive MEL. The CA and I coordinated our plans while waiting for the plan with ops removing the aircraft. I happened to run into a co-worker and just started chatting to catch up. Eventually a gate agent saw me and asked where the rest of the crew was and I answered I only knew where the CA was, I accordingly reached out to the CA to let him know that they never removed the aircraft and we should be good to go. The crew showed up to the aircraft to begin the boarding process and depart. While walking down the jetway we ran into the maintenance tech we had been working with who was furiously working on completing the MEL's for the FMC1. We stopped to discuss with him if he needed any help, he indicated that he was good but to double check his work which of course we affirmed. 

We entered the cockpit, began boarding once all the FA's showed up. Immediately as the CA sat down he noticed that IR3 had a SOLID align light and the IRS monitor page indicated that it need PPOS and to enter it every 10 min. We reasoned that maintenance had done this and we would verify when he dropped off the logbook. Once maintenance showed up with the logbook he mentioned no he had not in fact touched the IR's and allowed us to do a full align to start over. We started this process while our first order of business of confirming and verifying the MEL's applied for the FMC and making a list of need accomplishments. The TWO FR's for the cargo door and the blue pump plus whatever was needed for the FMC1. At this point we caught a couple of placarding error's and noticed the drop dead date on the primary FMC MEL had a drop dead date of [current date] effectively rendering the aircraft unairworthy once we landed. Maintenance indicated that this was a "technical glitch" which immediately made me extremely skeptical as we tried to sort through all the MEL's. Were we legal? Are the MEL's applied correctly? Apparently the "computer" generated a drop dead date of [current date] "accidentally" and they changed the primary MEL for the FMC1 to "3 flight legs" with a little pencil whipping on the logbook. 

Once we had this all settled we began to brief and get ready to depart and switched from GPU to APU power and IR3 ALIGN light lit up again prompting us to enter PPOS again. The maintenance tech was still with us in the jetway and he tried a few resets of the IR with no success. At this point we began to discuss whether it was prudent to add yet ANOTHER MEL to the aircraft and downgrade the capabilities of the aircraft further. After a few discussions the CA made with dispatch via ACARS questioning the safety of accepting the aircraft at this point, the CA reached out to discuss the matter with the duty phone CPO. The duty phone CP indicated that he was not going to dissuade us either way and it is ultimately the CA's decision and was extremely professional. We discussed the issue further and decided yes with maintenance we should not accept the aircraft with all the deficiencies of the aircraft. We had floated the idea earlier of possibly swapping aircraft but nothing came of this idea. Further moving along in the discussion another CP called and the entire tone of the plan changed as the CA had the discussion with the CP as I was not hearing the conversation but his body language said all I needed to know. At this point the entire tone of our decision changed as it became apparent that there was pressure sensed (even though not implicitly stated) but tone and words suggested by CP that we should take the aircraft or fear for our jobs "as he can go to bat for us but he strikes out a lot". At this point we seemed to have changed our minds but begrudgingly and hesitantly due to retribution by fearing for being disciplined. This elevated my stress level immensely. I had then realized I hadn't eaten for hours and had a horrible headache and was extremely tired from the stress we have been enduring since showing up for work. 

The CA next realized that the task card for the IR had not been accomplished in the haste of trying to get the airplane moving. Next we realized that we were butting right up on our 12hr duty day. After weighing all the circumstances through my head, dealing with the rolling delay, multiple MEL's that affected the overall capability of the aircraft being downgraded and possible issues we could experience with further failures, night time to uncontrolled field with no services, numerous FR's and continuous duty with the IR being MEL'd then pressure from a CP to take the aircraft regardless of our extreme hesitation. At no point were we refusing to actually fly the scheduled flight, we simply were not comfortable with the aircraft. I decided it was absolutely not safe to extend and I would unquestionably be fatigued at some point during our upcoming flights. I had expected and prepared to be on duty for my scheduled FDP, not double. We then contacted scheduling and told them we would not extend. I personally told scheduling I was not fit for an extension due to fatigue. Ultimately we never refused the aircraft but simply timed out but the issue still remains. 

In the MEL under Deferral Policies it states: 
Multiple items issued against an aircraft can have a negative effect on the interrelationship between those items and can affect aircraft operation and flight crew workload. Therefore, it is left to the Captain's discretion whether or not to operate the aircraft under those conditions. This is a clear statement that backs up exactly why we were hesitant to take the aircraft with ANOTHER MEL added to it. We were not trying to do anything other than break the chain of events leading to a bad situation. Give us a new aircraft, problem solved. Or have the CPO back up his front line employee who has worked here for a decade plus and never once had to call the duty phone until that night and second guesses our decision based on questioning the safety of the operation. This in turn speaks to extreme operational pressure placed on us by management. Yes I get we must make a profit to be successful, but there is no balance. The cultural mindset is get the mission done regardless of safety. We are hired to make decisions on the front line and operate the aircraft in a safe manner dictated by the FAR's and the FOM. I asked myself this: IF this event had occurred and we were being line checked by a FAA inspector, would we be having this discussion? Would an independent FAA inspector have backed us up or pressured us to take the aircraft? I am confident that the answer is the inspector would have had our stance. A true answer to this is the CPO must reestablish trust to me, I no longer trust the CPO with supporting us with operational/safety decisions or to back us up when we make a safety minded decision. I will be hesitant to reach out to them for any assistance until culturally things change. Secondly we must put safety first, not the operation. I would argue that the operation comes first and it is evidenced by the lack of bragging about our safety survey results and everyday operational pressure and firsthand experience on the line. We must work to change the safety culture of this company.

Synopsis

A320 flight crew reported a maintenance issue that started with a blue hydraulic system pressure indication that continued into a rolling six hour delay and multiple MELs. The situation was discussed with a duty manager who indicated that the crew should take the flight or face disciplinary action. When the crew reached their 12 hour duty limit they elected not to extend.

ACN: 1397462 (1525 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Oceanic
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 12321
Experience.Flight Crew.Last 90 Days : 204
Experience.Flight Crew.Type : 7300
ASRS Report Number.Accession Number : 1397462

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was the operating Captain of a [oceanic] flight. I was flying pilot for this leg and my First Officer the pilot monitoring (non-flying). At the beginning of our 4 day pairing, my First Officer complained of not feeling well and mentioned he thought of calling in sick. I mentioned that he should call in if he didn't feel well enough to continue. He agreed he would if he felt worse. He had a cough but didn't say much else about it. His condition seemed better on days 2 and 3 as I inquired how he was feeling on both days. He said "better".

On day four we operated this flight and departed after a delay. Climb out and cruise was normal in VMC conditions. Just prior or about our "coast out" point, he pulled his oxygen mask out and began breathing O2. I thought that maybe he was still not feeling well from before and his use of O2 was to help with that. I asked if he was all right to which he replied he felt "tired, like I didn't get enough sleep". I asked if the O2 helped and he replied yes. Approximately 30 minutes later, he opened the flight deck trash bag and vomited into it. I immediately called inflight and told them to "set up" for a lavatory break. I told the First Officer that if he needed, to use the lavatory to do so. He left the flight deck and returned to his seat a few minutes later. He complained that he felt unwell. He said his heart was racing and that he felt "anxious" and "light headed". I asked if the previous O2 had helped feel better which he said a little but still not well. He mentioned that he felt we may need to divert at some point. I said let me know what you need from me. He began sipping on ginger ale and/or water. I asked if the heart racing/anxious feeling was from having just vomited. He said maybe but didn't know. He told me he'd try to continue. A few minutes later, the First Officer said that something just wasn't right and I asked if he wanted a Medical phone patch. He agreed and we began the process by asking for a discrete radio frequency from ATC. Being overwater, this was via the HF radio. Radio conditions that day were very poor and communication was difficult. We needed a couple of different frequencies before we found one for the patch. Hearing the doctor was difficult but he said we should allow the First Officer to rest and hydrate if possible. During the phone patch, the First Officer said: "lets just go" which I took to mean [the destination] by his gesture forward with his arm. I asked him if he was sure and he agreed. I told the doctor that we'd reestablish the phone patch in 30 minutes if the conditions deteriorated. We terminated the phone patch and the First Officer continued to rest in his seat. 

Approximately 15 minutes after the phone patch, the First Officer said he wanted to divert to ZZZZ and complained again of his heart racing and not feeling well. I immediately asked for a phone patch via HF radio with dispatch and notified them of my decision to divert to ZZZZ due to crewmember illness. They agreed and gave a quick synopsis of weather conditions in ZZZZ. I told ATC of our need to change destinations to ZZZZ and requested a left turn direct to the ZZZZZ waypoint. He indicated that the request was already placed and to standby for clearance. He asked me the nature of the event. I said it was due to crewmember incapacitation. That term was used only to quickly convey that I had a crewmember that was sick enough to not continue to our destination over a radio with poor fidelity. [The First Officer] never lost consciousness. He never was incoherent. He was unable to perform his duties for portions of the flight due to the one vomiting episode and fatigue at points. I recalled inflight and asked our #1 flight attendant to join us in the flight deck. My concern was for the First Officer's safety and health and thought that inflight could help with this. Inflight said we had an EMT and a doctor onboard. I asked the First Officer if he'd like to be evaluated by them. He said yes and left the cockpit for the forward galley. He returned a short time later and said that the onboard equipment couldn't get a reading on him. While the First Officer was being evaluated, ATC gave me clearance to turn left direct to ZZZZZ waypoint then direct ZZZZ which I executed. At the same time I made a traffic call out on the common frequency 123.45 to advise other aircraft of our unusual left hand turn. At this point, I made an announcement to the passengers that we had an ill pilot that needed attention on the ground. I told them we were going to land in ZZZZ and had about 45 minutes to land.

The descent to ZZZZ was made without incident. The First Officer remained in his seat for the remainder of the flight. He said he began to feel better when the aircraft was at a lower altitude. I told ATC that I had the airport in sight and was cleared for a visual approach. I entered a left base for the runway and began slowing the aircraft for landing. Near the normal flap 1 extension point, I looked over at the First Officer and asked if he wanted to extend flaps. He said: "yeah, I can do that". He continued the flight with normal SOP pilot monitoring duties with call outs etc. We landed and taxied to the ramp area as normal. EMTs entered the flight deck after engine shut down. The First Officer told them he felt fine but his heart was still kind of racing. They began to check his vitals and mentioned his blood pressure was elevated and recommended him to be "checked out". He agreed and walked down the air stairs to the gurney to be further evaluated. The EMTs loaded him into the ambulance and I his bags. Attention turned to securing the passengers and aircraft with ZZZZ station personnel. Station Operations offloaded the passengers into a holding area. I called dispatch and crew services to formulate a plan of action and was told a rescue plane was being dispatched with reserve crew to continue to [the original destination] upon arrival.

Synopsis

A320 Captain reported diverting due to the First Officer feeling ill during an oceanic flight. This was the fourth day of the trip and the First Officer had been complaining of not feeling well on each day prior to the diversion.

ACN: 1396603 (1526 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 3933
ASRS Report Number.Accession Number : 1396603

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11483
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 9459
ASRS Report Number.Accession Number : 1396618

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At FL350 we got a Standby Pitot ECAM, followed later by L/G Shock ABS Fault ECAM. Very shortly after, we lost both flight directors, both autopilots, and auto thrust. After consulting with [Maintenance] and Dispatch, we could not do anything to reset our systems. The forecast in [destination] was for possible windshear. I determined with my FO the best possible solution was to divert to [a suitable alternate] with clear VFR and low wind. After lowering the landing gear, systems all reset. Performed a normal landing.

Narrative: 2

Opted to divert because LLWS advisories were in effect at [destination] and we couldn't use proper flight manual precautions upon arrival given the current aircraft state. When we lowered gear [at the alternate airport], the system appears to have reset and caused all fault indications to clear. We landed 800 pounds overweight. Otherwise, uneventful landing.

Synopsis

A320 flight crew reported diverting to an alternate airport after experiencing multiple system faults that reset when the landing gear was extended during approach.

ACN: 1396485 (1527 of 2908)

Time / Day

Date : 201610

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : PMC, Performance/Thrust Management Computer
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1396485
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1396499
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Engine 1; 2 FADEC Ground Power Switches were left on by maintenance. Both are guarded switches on the overhead panel which is not easily visible to either pilot. This was discovered during cruise flight. A message was relayed to [Dispatch] which replied with the following "Maintenance left them on. Doesn't affect anything, as once engines are running, they power themselves. Maintenance will shut off on arrival. Thanks." We also wrote it up when we blocked in.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported discovering in flight, that FADEC Ground Power Switches were left on.

ACN: 1394901 (1528 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class C : ZZZ

Component

Aircraft Component : Cockpit/Cabin Communication
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 448
ASRS Report Number.Accession Number : 1394901
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1394903
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At V1 a loud warning/alarm was emanating from the overhead panel with no associated ECAM. The takeoff was continued aircraft was cleaned up and vectored for departure. We elected to remain with departure at 8,000 feet and 250 kts. We were vectored in a box pattern as we tried to find out what the noise was and why we were getting the alarm. The communication with the cabin crew was lost. A visual inspection of the cabin from the cockpit was conducted and no signs or indications of an emergency were noticed. We could not determine why we were unable to communicate with the cabin crew and why we were getting the alarm/noise. We elected to return to departure airport and ran the overweight landing checklist. The landing and rollout were uneventful, we taxied clear and stopped on the taxiway. The captain made another visual inspection and saw no signs of an emergency in the cabin and we continued to the gate. As we taxied in the purser slid a note under the door giving us his indications and that the cabin was ok but experience some abnormalities with the lights and signs as well as no communication abilities. The cabin crew handled the situation perfectly and Flight Attendant A's quick thinking with the note gave us the reassurance that it was ok to to taxi to the gate and deplane in a normal fashion. At the gate we shut down and after shutdown the power was cycled which stopped the alarm. Maintenance boarded the aircraft and found an issue with CID1.

Narrative: 2

Just prior to lift off on takeoff a loud alarm started sounding in the cockpit. There was no ECAM. We were unable to silence the sound. We suspected the lavatory smoke detector. We were unable to communicate via the interphone with the flight attendants or use the PA system. We returned to [departure airport] for an overweight landing.

Synopsis

A320 flight crew reported hearing a loud, alarm-like noise during takeoff, followed by the complete loss of the communication capability with the cabin crew. They elected to return to the departure airport, where a successful overweight landing was accomplished.

ACN: 1393738 (1529 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY
Relative Position.Distance.Nautical Miles : 10
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 30000
Experience.Flight Crew.Last 90 Days : 30
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1393738
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

First Officer was flying and briefed the approach to JFK for the RNAV RNP 13L. Part of the briefing included the altimeter setting from the ATIS. The captain acknowledged the altimeter setting but did not crosscheck it on the ATIS. First Officer commenced approach and at final approach fix both pilots recognized that aircraft was high even though instruments showed that it was on the glide path for the RNP. Captain advised the first officer to execute a soft go-around and tower asked the reason for the go-around to which captain stated that they were too high to complete the approach. Tower gave a clearance to climb to 2,000 feet and switch back to approach control.

Approach control vectored aircraft back for the same approach. The controller cleared the aircraft for approach by saying "Cleared direct Asalt. Maintain 3,000 or above until passing Asalt, cleared for the RNAV RNP approach to runway 13L." The captain read back the clearance, but questioned the altitude because he said he was at 2,000 feet, not 3,000 feet. The controller said he showed the aircraft at 3,000 feet and stated the altimeter setting which was 30.19. Both pilots had 29.19 set causing the aircraft to appear to be 1,000 feet lower than it actually was. The altimeters were reset and an uneventful approach and landing were accomplished. It should be noted that both pilots had been on duty for nearly 16 hours. There was no conflicting traffic in the area during the either approach.

Synopsis

A320 flight crew reported they did not set altimeters correctly and ended up high on the first approach which necessitated a go-around.

ACN: 1393074 (1530 of 2908)

Time / Day

Date : 201610

Place

Locale Reference.Airport : OKC.Airport
State Reference : OK
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.TRACON : OKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class E : OKC

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 243
ASRS Report Number.Accession Number : 1393074
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Planned south approach into OKC. Two severe thunderstorms well north of airport and one well south along a strong cold front. Upon contact with approach control advised [another air carrier] encountered Windshear on approach, executed go around and returned to departure airport. Approach also advised wind had shifted to north landing gusting 38 knots and Windshear alert of 25 knots. We leveled at 9,000 feet and took delay vectors in clear/smooth ride. Shortly thereafter winds reported 20-25 knots with no windshear alert. We accepted descent and vectors for approach. With configuration flaps 1 speed 210 knots descending on downwind turbulence began to increase to moderate continuous. Airspeed shortly thereafter increased rapidly and I disconnected auto pilot to prevent flap overspeed. It was quickly followed by a rapid severe decreasing performance windshear, severe loss of airspeed and sink. We executed a go around and diverted to AMA. After parking brake set we reported in-time per current company guidance and "stay" on FAA interpretation of flight time. After refuel we returned to OKC and flew 7:46 for the day. However with a ground time of 1:18, it could have been 9:04.

Synopsis

A320 Captain reported executing a go-around and diverting to an alternate after encountering wind shear on approach to OKC.

ACN: 1392575 (1531 of 2908)

Time / Day

Date : 201610
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Flight Phase : Takeoff / Launch
Route In Use.SID : SSTIK 3
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
ATC / Advisory.TRACON : NCT
Make Model Name : B737 Undifferentiated or Other Model
Flight Phase : Takeoff / Launch
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1392575

Events

Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were in position on Runway 01L assigned the SSTCK SID on departure. A B737 was on runway 01R and cleared for takeoff and then we were cleared for takeoff immediately after them. From my position I could not see them as they were lined up behind my field of view. We immediately began our takeoff roll using flaps 3 configuration. I had lined the aircraft just left of centerline, to avoid running over the centerline lights on takeoff roll and to compensate for the strong left crosswind. My flying partner is a very experienced pilot and performed a normal takeoff keeping the aircraft aligned as I had given it to him and he rotated and was airborne just prior to crossing the intersecting runways. After rotation the Flight Director commanded a slight right turn which he did not follow maintaining his initial runway track. The SID directs flying a heading of 014 until 520 feet then turning left to SSTIK. Just after I raised the gear, we heard "Monitor Vertical Speed" and the VVI indicator turned red followed immediately by "Clear of Conflict". This happened so quickly we didn't have time to react to it. By now the Flight Director was commanding a full left turn to SSTIK and tower handed us over to NOCAL Departure. We continued the climb out. 

After completing the After Takeoff Check and checking with NORCAL, I had time to reflect on what had happened. We had received a TCAS Resolution Advisory (RA) but the first part of it was inhibited because of our altitude. When we climbed above the inhibit altitude we heard the tail end of the warning. I suspect the aircraft on 01R drifted into our TCAS threat zone triggering the event. Again, I could not see the aircraft on the other runway, so I am not sure where they were in relation to our position but I know the SIDs on 01R direct a right turn away from the runway track. The problem with the RNAV departures in SFO is they do not account for aircraft in close proximity to each other and direct straight out tracks for longer than they should. I have also witnessed the Flight Directors in strong crosswinds, as it did in this case direct a turn that exacerbates the potential for a close encounter. I did not report this event to ATC because by the time I had figured out what had occurred we were well beyond their frequency.

Synopsis

A320 Captain reported receiving an Resolution Advisory (RA) shortly after takeoff from Runway 1L in SFO, presumably from another aircraft that took off from Runway 1R.

ACN: 1391438 (1532 of 2908)

Time / Day

Date : 201609

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Flight Phase : Landing

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 651
ASRS Report Number.Accession Number : 1391438

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On final approach prior to the FAF, the PF called for the landing gear down. I lowered the landing gear handle and the gear did not come down (none of them). I referenced the checklist and recycled the landing gear handle and the gear still did not come down. The checklist called for the gravity extension procedure. We accomplished the procedure and the gear came down and locked with normal indications. We knew the nose wheel steering would not work. The Captain landed uneventfully and taxied clear of the runway. I coordinated with Ops for a tow vehicle and we were towed to the gate and terminated the emergency uneventfully and accomplished all the normal checklists.

Synopsis

A320 FO reported the landing gear failed to extend using normal procedure, but it did extend using the backup gravity system.

ACN: 1390739 (1533 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class D : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1390739
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1390744

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying and just after V1 during rotation we heard a loud popping sound from the right side of the aircraft. Just after I observed the EPR roll back on engine 2 and ENG 2 EGT overlimit ECAM. I stated "I have control and ATC, ECAM actions", which said to reduce thrust under the limit. So we brought back thrust on # 2 engine. Maybe 20 sec after I spoke with the FA as the FO was continuing to get more ECAMs. The lead FA said that they have a fire and an explosion in the rear of the aircraft. I said I would call back in a few minutes. At that point I switched from CTAF to departure control and [advised them]. I believe I told him we are having engine 2 issues and the FA reported a fire on the right side of aircraft. I told him I want to divert to ZZZ. He ended up giving us vectors to runway XX after he offered YY. Runway XX was longer with an ILS. I believe I started the APU at this point. We also had some autoflight ECAMs. I called the FA back and asked her about the fire. She stated the FAs in the back had said there was fire but not currently sure. I said we would be landing in ZZZ in less the 5 min and we would be stopping on the runway to assess the situation about evacuating with regards to Fire Rescue. We ended up finally getting an ENG 2 shut down ECAM which we accomplished. 

My biggest concern was to get the aircraft on the ground safely because of a possible fire. We landed shortly after on Runway XX and stopped on the runway. Fire rescue took a good amount of time to assess the aircraft. They stated to me that they thought engine 2 to be still running. I looked at the Eng 2 master and it was off so I pushed the ENG 2 fire push button to make sure. During the assessment of the aircraft I spoke to the passengers and FAs and told them at this time there was no visible fire but to please stay seated with seat belts on. A few min later with coordination with ZZZ tower we taxied to a remote area. We were then met with air stairs and busses took the passengers to the terminal. There were no injuries and the FAs did a great job of keeping the passengers calm during the short 10 minute flight.

Narrative: 2

ZZZ is a controlled airport, had great weather and the captain decided to divert. We climbed at around 3000 feet and with the runway in sight we started the approach to runway XX at ZZZ. Captain prepared the airplane to land with flaps 3 and medium brake. Captain touched down smoothly and stopped the airplane without any difficulties. After landing firefighters checked the airplane on the runway to make sure there was no fire around the engine since passengers confirmed seeing fire to the flight attendants. After we received the clear from the fire crew, we exited the runway and parked in a remote area where passenger were disembarked. Nobody was injured.

Synopsis

A320 flight crew reported a #2 engine failure just after rotation and successful diversion to a nearby airport.

ACN: 1390192 (1534 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 33000

Environment

Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : B747-400
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLC

Person

Reference : 1
Location Of Person.Facility : ZLC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Function.Air Traffic Control : Instructor
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 25
ASRS Report Number.Accession Number : 1390192
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was training a developmental on the TRACKING position of R15/16. The developmental was already certified on the position as an R side but we were completing the TRACKER requirement. There was a D Side controller and an R side controller. We were not very busy. The [A320] at FL330 was southeast bound in level flight with about a 40 or 50 knot direct tail wind. No chop or turbulence reports had been received. A B747-400 aircraft was at FL340 southwest bound. The A320 passed approximately 17 miles behind the B747. We did not issue merging target procedures, as they were not even close to merging. The [A320] pilot shortly after passing the path of the B747 asked if they had passed under another aircraft and what type it was. He stated that "We just crossed someone's wake turbulence and got a pretty good jolt there. A heads up would have been nice if you guys are going to do that sort of stuff." We responded with roger. 

I have been regularly filing ATSAP reports on this issue with very little acknowledgement from the FAA that this is a problem. This problem has occurred numerous times, just on my one sector, out of "hundreds throughout the NAS." I suspect it is happening with greater frequency then the controllers are willing to report due to a "don't care" attitude. As I have reported earlier, a pilot complained due to 2000 feet separation from an A380. Another pilot complained because they went 3 miles away from an A380. These pilots all have complained of serious jolts or turbulence each time and they are blaming us as the controller for the incidences. There is a bigger issue the FAA is unable or unwilling to recognize with heavy to super aircraft. This latest issue was not even an A380. A B747-400 can gross weight at takeoff at 900,000 pounds. That makes it nearly a super category. I am not certain that anyone in the FAA decision tree understands this issue. In order to enhance safety with these aircraft it should become mandatory that B747-400 and A380 and the one Antonov 225 in existence be given a greater degree of separation or at least warnings every time. 

I do not believe the FAA or ATSAP is taking this issue as seriously as it should. This has been an issue since the introduction of these extra heavy aircraft.

Synopsis

ZLC ARTCC Controller reported receiving a wake vortex encounter report from an A320 that passed 17 miles behind a B747-400. Controller stated he is concerned about the number of reports he gets from aircraft encountering wake from "heavy" and "super" aircraft. He recommended more separation for these aircraft.

ACN: 1389174 (1535 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.MSL.Single Value : 4000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : BWI

Component

Aircraft Component : Tablet
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1389174
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We received a clearance via PDC and ACARS that stated "climb via SID, except maintain 4000 ft". We departed, and upon checking in with departure were told to climb via the SID. We leveled off at 4000 ft as we both believed this to be our clearance limit altitude. The departure controller queried us as to why we leveled off. We stated it was what we were cleared to. He directed us to climb to upper limit of 17,000 ft. We then continued to climb to the correct altitude.

I believe that we both contributed to the occurrence/event by not seeing or omitting the 17,000 ft altitude limit in the lower right corner of the approach plate. When viewed in landscape form with an iPad, the 17,000 ft fix was not visible on the departure plate page. Scrolling down to read the bottom of the page, (which should move this critical altitude up into view) places the 17,000 ft limit behind the crayon/marker/rotate emblem in the lower right corner, again in landscape view. Also, the font and size of "17000 ft" do not lend themselves to being critical or eye catching, especially when hidden or obscured from view during viewings and briefing. There is a very small window of 1/4-3/8 of an inch in which a pilot might be able to view this critical information.

Move the crayon/marker/rotate function on the iPad to either the side bar or upper bar so as to be out of the field of view where it would not block or obscure critical information. Being just north of the Washington DC Restricted Airspace Fight Free Zone and P56, if not for an alert controller, this event could have ended poorly for all on board in the interest of National Security. The controller did an excellent job of noticing and notifying us of our improper level off altitude and immediately correcting the situation.

Synopsis

A320 First Officer reported they failed to climb to assigned altitude on departure from BWI with usability of their iPad tablet cited as a contributing factor.

ACN: 1388864 (1536 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DFW.Airport
State Reference : TX
Altitude.MSL.Single Value : 15000

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : D10
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Vectors
Route In Use.SID : TRYTN2
Airspace.Class E : D10

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : D10
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Operating Under FAR Part : Part 91
Flight Plan : VFR
Flight Phase : Descent
Route In Use : VFR Route
Airspace.Class E : D10

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1388864
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1388873
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Climbing out of DFW on the TRYTN2, we were initially cleared to 17,000 feet, then re-cleared to 16,000 feet due to VFR NORDO traffic crossing our route ahead. Climbing through about 15,000 feet, we got "TRAFFIC, TRAFFIC" from the TCAS. I immediately leveled off with "Vertical Speed Zero", trying to minimize the conflict while we looked for it. I was a little confused because we were still about 2500 feet below where I thought the traffic was. Very shortly thereafter, we got "DESCEND, DESCEND" and immediately commenced the TCAS procedure although I still couldn't understand why we were getting it so early. By now, the ATC controller had re-entered the scenario, and ordered a 20 degree right turn for traffic separation. Turns out, the VFR NORDO had commenced a descent which had gone unnoticed by ATC. Both our aircraft, and the VFR NORDO, were operating in accordance with their respective IFR/VFR rules. However, in this case, the rules were insufficient to provide standard separation without close scrutiny and input from ATC. The controller had other traffic to deal with, and the momentary diversion of his attention just happened to coincide with the VFR aircraft's descent initiation.

I think a change in ATC protocol regarding mixing VFR NORDO traffic with any traffic under ATC control is warranted. Specifically, I think ATC should immediately stop allowing IFR traffic to simply under fly NORDO traffic. Worked traffic should be vectored behind, or at least 3 miles ahead of, crossing NORDO traffic for the simple reason that you just can't know when the NORDO will change course, either horizontally or vertically. Furthermore, most turboprop aircraft could easily attain a 3500 fpm descent with very little notice. It seems to me, that this could easily overwhelm the TCAS system and make it unable to plot a solution and get the pilot to comply in time to avoid a collision. I think pilots should be given more latitude to initiate separation action before the actual RA is displayed. The RA waits until there is little time remaining to save the aircraft. A pilot could see the situation developing but without ATC clearance, (the controller is busy with another plane) would almost certainly face disciplinary action for flight not in accordance with his clearance. It's like the system is only interested in "close calls", so converting them to "long calls" isn't a priority.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported a TCAS RA event with a VFR aircraft not in communication with ATC.

ACN: 1387941 (1537 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : Airbus Industrie Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 9500
ASRS Report Number.Accession Number : 1387941
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were performing the visual approach to 28L in SFO. At approximately 2,500 feet AGL, we responded to a TCAS climb RA from an Airbus that was landing on 28R. We were left of the 28L centerline and descending on glide path. The [Airbus] was well above us by at least 500 feet, gear down, descending in a right turn. There was no possible way the [Airbus] aircraft had visual contact with us. We complied with the TCAS climb and executed a missed approach with the Tower.

On our second approach to 28L, we were left high by NORCAL while searching for a [second Airbus] below us on the visual to 28R. It is hard to gain visual contact on a grey painted aircraft during the period of twilight just before sunset. We were held at 4,000 feet until we could gain visual on the [second Airbus]. When we did gain visual, we were instructed to slow to 160 KIAS and fly the visual approach. We were too high to comply, as the approach clearance as it was granted very late. We would not have been able to execute a stabilized approach. We executed a second go around to 28L. There was heavy radio traffic on the single frequency and many radio calls were blocked.

Our third attempt was vectors for the ILS 28L. We declared minimum fuel in the base turn. A successful approach and landing was made to 28L. We landed with 45 minutes of fuel remaining.

Synopsis

A320 Captain reported traffic conflicts resulted in two go-arounds on approach to SFO.

ACN: 1387242 (1538 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 11000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use.Other 
Airspace.Class B : DEN

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1387242
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Day before Day 3 we flew 2 legs, with an early east coast wake up and report time. What was supposed to be an easy day tuned into a very long day with multiple delays adding about 4 hours of duty time to our day. Day 3 Departed ZZZ on time. Early AM once again. Visual conditions prevailed at the time of our arrival into Denver approach's airspace. We were told to expect either the ILS 35R or possibly the RNAV RNP Z. We briefed the ILS. I believe somewhere in the descent from 14,000 to 11,000 ft we were given the clearance for the RNAV RNP Z. Being that the prevailing visibility was VFR, I PM/CA gave an abbreviated briefing. I briefed the PF/FO from the EFB checklist. However, I failed to pick up the paper covering the RNAV placard to see that the a/c was not RNAV/RNP approved. It wasn't listed in the remarks section or in the crew briefing of the release. I didn't catch the error until after the approach had been initiated. We continued in VFR conditions and landed with visual conditions uneventfully. Had it been IFR, I wouldn't have done an abbreviated briefing.

Suggestions
1) To be more vigilant.
2) I would like to see some changes with our procedures. Specifically, I would like to see a "monthly" memo of which aircraft are not RNP/AR certified. With the non-standard fleet and aircraft continuously coming and going, it is hard to remember which item applies to which aircraft. Some aircraft in the fleet are not RNP/AR approved, but lack a placard. In speaking with Maintenance, I was told a placard was not a required item. 
3) Have dispatch continue to state in remarks and crew briefing sections of release, that RNAV/RNP not authorized, even though aircraft has been on property awhile; not only when it's new to the line.
4) Move placard to glareshield in direct view of both pilots.
5) Place placard or sticker on logbook stating such (just like overwater aircraft).
6) Make RNAV RNP Not authorized sticker red, not black and white, so it catches your attention.

The Pilot memos only go back so far in my EFB. In searching for information, I could find [only two aircraft] with memos. I feel at times that we are set up for failure. I pride myself on being thorough. It is not my intention to place blame elsewhere, but to draw attention to how easily these simple fixes could help mitigate future error.

Synopsis

A320 Captain reported accepting an approach for which the aircraft was not approved. The approach and landing were without incident due to clear weather.

ACN: 1386642 (1539 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1386642
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1386975
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We shut down the right engine while the engine was in idle reverse and the reverser did not stow correctly. We did not receive an ECAM message untill after both engines were shut down at the gate. 

I had the copilot go down and clear the area and then call me on the phone when the area was clear. Once clear, I started the engine to about 30 percent, the reverser stowed correctly and I shut off the engine.

All conditions normal. 

I think we did everything correctly and ensured the safety of all involved. In the future I would call maintenance first and just ensure they were involved.

Narrative: 2

Following landing and short taxi, the Captain commanded me to shut down the number two engine while approaching the gate area. Due to it being an unfamiliar airport to me, I was especially guarding against unnecessary "heads down" operation during the taxi in.

Upon arrival at the gate I noticed and brought to the attention of the Captain the fact that there was a reverser unlocked ECAM indicated for the number two engine. I asked him what he thought we should do about this shortly thereafter, imagining that I would have to see if the station had company or contract maintenance.

After the Captain pondered the situation for a short period of time, he told me he would like try to "motor" than engine for a few seconds to see if the reverser doors could be closed. I pondered that use of the yellow electric pump might also be employed, but I ultimately yielded to his experience, partly because of my knowledge of his involvement in the company training department. I assumed that he felt this was a safer course of action for some reason(s). Perhaps a few seconds of motoring would result in a slower and safer closing than could be achieved by applying pressure from the yellow system electric pump, was my initial reasoning with myself.

He planned that when I went out to perform the post-flight inspection, I could call him on his mobile phone and indicate that it was clear to engage the air turbine starter for a few seconds of motoring. Once I arrived on the ramp, I performed a complete post flight inspection. I spoke with the ramp staff and indicated that the Captain wanted to "motor" the engine for a very short period of time. Also that I would guard the area during that short time to insure safety. Once I secured the immediate area around the engine, I did as the Captain asked and called him on the phone and said "the area is clear for motoring" and stood a safe distance to the side of the engine and monitored the area.

It became obvious a short time later that the "motoring" was continuing for a longer period than what I was expecting or what had been briefed. I then heard what sounded like the start sequence being initiated. I ran from my position forward and up to the side of the aircraft and began pounding on the fuselage in hopes to prevent this from occurring. It was of no avail, the engine had fuel introduced to some degree.

Having been actively involved in aircraft maintenance for years, I have never known "motoring" to include introduction of fuel to an engine. Unfortunately, at that moment I felt as though the Captain briefed me that X would occur and what actually occurred was X plus Y without a briefing or additional safety assessment.

I cannot explain the actions of the Captain other than to surmise that this may have been an accidental introduction of fuel to the engine. What occurred was not what I was briefed to expect.

In retrospect for myself, the most prudent course of action that I could have taken was to highly suggest, as I initially thought, that this was and should have been a maintenance issue to be disposed of by our maintenance staff. I think that my decision process was impacted by a variety of factors: late night, long duty and block day of flying, the fact that the captain admitted that he caused the situation rather than this being a maintenance malfunction, the Captain's greater experience with the aircraft than mine, and knowledge of his experience on the training committee leading me to assume this was a kind of procedure that he was familiar with to some degree.

In summary, although this situation resulted in no issues, I think it might have had the potential to result in a safety problem. The Captain's intentions were pure and good and forward thinking of the following crew. My intention was only to assist him to resolve his "self-induced" issue by following his directions, as long I thought they could be executed safely. Once the briefed plan was altered, without my knowledge, I felt like the safety assessment that was made was no longer valid. I think that the fact that nothing bad happened could have had worse results had it not been for my presence on the ramp.

My goal and intention going forward is to learn from this experience that many times first instincts are best instincts and issues such as these should simply be referred to maintenance regardless of anyone else's "get it done" attitude or desire for expediency. I will endeavor to insure that an event were safety could be compromised does not occur, to the best of my abilities. 

This event began with an engine shutdown while the Captain inadvertently had the number two engine in idle reverse at the time he commanded for it to be shut down.

Synopsis

A320 flight crew reported they chose, in response to a reverser that failed to stow upon gate arrival, to motor the engine to facilitate stowing the reverser.

ACN: 1385200 (1540 of 2908)

Time / Day

Date : 201609
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : A80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : I-HFW
Flight Phase : Initial Approach
Route In Use.Other 
Airspace.Class B : ATL

Aircraft : 2

ATC / Advisory.TRACON : A80
Aircraft Operator : Air Carrier
Make Model Name : Medium Large Transport
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ATL

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1385200
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1385201
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were cleared for that ILS PRM Approach to Runway 8L and told to maintain 160 knots to SCHEL, the final approach fix. After intercepting the localizer the Captain was concerned about our spacing and asked the Approach Controller how our spacing looked with the aircraft in front of us on the same approach. The Approach Controller said our spacing was fine. The Captain then pointed out to me on our TCAS that we were only around three miles behind the aircraft in front of us. The aircraft in front of us slowed down even more and the Captain started talking about a possible go around. About two miles before the SCHEL intersection I asked the Captain if I should manage my speed to slow down since we both felt like we were gaining on the aircraft in front of us. We both agreed to manage our speed. The Approach Controller was busy with other aircraft, and the Captain was unable to tell him that we were slowing a little early.

Once we arrived over SCHEL, the Approach Controller asked us why we had slowed early. The Captain said because of our spacing and that we were trying to avoid a potential conflict. The controller said our spacing was fine. We were around 400 feet AGL on short final when the aircraft in front of us cleared the runway.

With this event I was trying to avoid a potential conflict where the Captain and I felt we had been vectored in too close to the aircraft in front of us. I should have maintained the assigned airspeed all the way up to SCHEL and chose to go around if there was an issue at that point.

Narrative: 2

Approach Control gave us an intercept heading and clearance for ILS PRM 08L approach; following an aircraft Y (I do not believe we ever called the airport or the traffic in sight). Upon intercepting the localizer course, I questioned our proximity to aircraft Y which appeared to be less than 4 miles ahead on TCAS and asked if our speed/separation was okay. We were told we were fine, and to maintain 160 knots until SCHEL final approach fix intersection.

As the preceding traffic crossed SCHEL intersection, we appeared to be rapidly gaining on the aircraft; separation had decreased to only about 2-2.5 miles on TCAS and was continuing to decrease. At this point, we had visual contact with the preceding aircraft and the field, there was also an aircraft close off our right wing on the parallel approach so "S turns" were not an option. I commented to the First Officer to be ready for a go around and we both agreed we were gaining on the preceding aircraft. The First Officer began to slow our aircraft prior to SCHEL intersection.

Before I could advise ATC of the deviation, we were crossing SCHEL and the Approach Controller questioned why we had slowed prior to SCHEL. When I replied that we were rapidly gaining on the aircraft ahead, the controller responded that he could assure us we had plenty of room. We were instructed to contact the Tower.

The preceding aircraft just cleared the runway as we descended through about 400 feet on final. I believe that if we had not slowed early, we would have had to execute a go around and may have gotten a TCAS RA. After landing and during taxi, we were instructed to call the TRACON.

After parking the aircraft at the gate, I called the TRACON number provided and spoke with a few different supervisors; including one that contacted me back. He explained their separation standards on that approach (about 2 miles between aircraft) and we discussed the incident. He told me that in the future we just needed to inform them if we needed something other than assigned so they could plan or coordinate. We mutually acknowledged each other's responsibilities and concerns in this incident and the supervisor said that they would not be pursuing the matter further.

We should have communicated to ATC that we felt we needed to further slow the aircraft prior to doing so. That said, I had already expressed concern earlier on the approach that we seemed to have been vectored onto the course a little "tight" and even after the controller admonished us for slowing early, he disagreed about the need for us to slow further. We also could have just maintained the 160 knots to SCHEL and executed a go around if/when it became necessary.

The first officer managed the aircraft speed to slow the aircraft; had he instead used selected speed and flown no less than 150 knots at SCHEL we would have been within acceptable +/- 10 knot tolerances. The ATC supervisor mentioned the 10 kt "buffer" to me when we spoke.

Our perception was that we were "closing" on the preceding aircraft; we did what we felt was necessary to avoid getting unsafe following distance from the aircraft ahead, getting a TCAS RA, and/or having to execute a go around. The supervisor acknowledged that a go around would have been challenging for them to deal with at that point too. I was surprised to learn that the allowable separation distance between aircraft was so small on that approach; 4-5 miles is much more standard/common separation (except perhaps LGA).

This airplane/flight had already been delayed several hours as a result of multiple maintenance and weather issues earlier in the day. In addition to the extensive delay, as a result of the ACARS being inoperative, the whole day had required a lot of non-routine procedures to manually obtain, input, crosscheck and relay information; contributing significantly to pilot workload and cumulative fatigue. This may have contributed to a desire to avoid further complications or delays.

Synopsis

A320 flight crew reported slowing early during approach via ILS PRM 08L at ATL due to the perception that they were gaining on the aircraft ahead, without advising ATC. ATC is not happy.

ACN: 1384018 (1541 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.AGL.Single Value : 100

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Landing
Airspace.Class B : NYC

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 215
ASRS Report Number.Accession Number : 1384018
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 435
ASRS Report Number.Accession Number : 1384026

Person : 3

Reference : 3
Location Of Person.Facility : EWR.Tower
Reporter Organization : Government
Function.Air Traffic Control : Trainee
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1384719

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On initial contact, tower stated you are number 3, traffic to follow is on final for 22L and 11, Runway 11 cleared to land. I was PM and I watched traffic 5 miles in front of us land on Rwy 11. I was looking for traffic on final for 22L and did not see any. I concluded the traffic for 22L must have landed before the traffic for 11 in front of us. When we were on very short final, I saw an RJ looking like he was turning left base for 22L. I later learned that this traffic was making a very large S turn on final to generate some spacing. The spacing did not look like it would work. We were definitely lower than the traffic on 22L. I queried tower, "Hey tower, is this guy going to land on 22L?" Pregnant pause. We touched down. I expected tower to send the other traffic around. Right after our nose gear touched down, tower said, "hold short of runway 22L." I did not respond immediately, as I was checking for our stopping devices, spoilers and reversers. Tower came back again, verify hold short of runway 22L." The FO got on the brakes very firmly and I assessed that we would be able to stop in time. I then replied, we will hold short runway 22L."

Runway 11 is not a LAHSO runway in EWR. I did not feel comfortable with the spacing. I did not feel that tower would have enough time to send the other traffic around. I felt the only option was to hold short. I called and spoke with the supervisor in the tower. He stated that this was a legal clearance and it was not a LAHSO clearance. He stated that it is the tower controller's judgment as to when to contact a pilot on the rollout after landing. The nose gear must be down, and the pilot should be at a reasonable speed. We were not at a reasonable speed. He stated that if we replied, "Unable" then the controller would have sent the traffic for 22L around. In my opinion, the spacing was too close. We were not down to a taxi speed that would have allowed for an easy assessment of our ability to hold short of 22L. And because I believed that tower did not have the time to send the other airplane around, I felt our only option was to jam on the brakes and make it work. Bottom line, the spacing for this approach was too tight.

Narrative: 2

I was the pilot flying and cleared to land on runway 11 number three. One of those aircraft was landing ahead of us on 11 and the other was landing 22L. Our clearance was cleared to land. The approach was normal until 30-50 feet when the captain noticed the traffic on short final for 22L and queried tower. Immediately after touching down, tower told us to hold short 22L. I immediately applied brakes and deployed the reversers to slow the aircraft. Tower called back as we slowed and advised us again to hold short and the captain replied. We stopped just past the hold short line for 22R. Tower again told us to hold short 22L and cleared us to exit taxiway P. I feel that given the already short length of runway 11 and the inadequate spacing with the traffic on approach to runway 22L, one of us should have been sent around by tower. It was unsafe. Had we not been able to stop on the runway or had we gone around, it would have presented a traffic conflict.

Narrative: 3

I was training on local control. We were using runway 22L for arrivals, runway 22R at Taxiway W for departures and runway 11 for arrivals. I cleared an A320 to land runway 11 as that was the instrument approach they were set up for. When they landed and were rolling out past taxiway Sierra, I instructed them to turn right on Papa, the pilot just read back the call sign but it was barely readable. I then instructed them to hold short of runway 22L for an arriving aircraft on a 1 1/2 mile final for runway 22L and I received a read back for the hold short for 22L, but the pilot held short of runway 22R.

All pilots should make themselves familiar with the procedures at airports in which they fly and if they cannot or are not willing to comply with a controller's instruction should advise 'unable.'

Synopsis

A320 flight crew and the Controller involved described a landing on Runway 11 at EWR during which the crew was instructed to hold short of Runway 22L for landing traffic. The instruction came during the rollout and the crew believed it left them few options but to brake hard and ultimately stopped before Runway 22R.

ACN: 1383133 (1542 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1059
ASRS Report Number.Accession Number : 1383133

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cruise flight deviations for weather and trying to top weather number one engine rolled back two times briefly. One brief ECAM, engine fail stall, then cleared without pilot action. Requested lower altitude [FL320] and assistance from ATC with weather deviations. We discussed the indications and options. We determined that this was not an indication issue and requested divert to nearest suitable. The flight attendant in the aft cabin called and reported smell of smoke. We [advised ATC]. No ECAMS or smell of smoke present on the flight deck. Clear of the weather we reviewed smoke and engine failure QRH while proceeding to [alternate]. Later, the flight attendants reported they no longer smelled smoke, and that they had smelled smoke on takeoff but also had gone away. Number one engine continued to be stable [and] no other smell of smoke. [After landing] Maintenance found bite stator valve failure.

Synopsis

A320 First Officer reported diverting to an alternate airport after experiencing several engine rollbacks along with a smoke odor in the cabin.

ACN: 1381302 (1543 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14430
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 11458
ASRS Report Number.Accession Number : 1381302
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 189
ASRS Report Number.Accession Number : 1381306
Human Factors : Situational Awareness
Human Factors : Confusion

Person : 3

Reference : 3
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8295
Experience.Flight Crew.Last 90 Days : 150
ASRS Report Number.Accession Number : 1381623
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Object
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Weather
Primary Problem : Company Policy

Narrative: 1

After approximately 40 min of holding we were cleared to land. Landed and taxied to the gate. Ramp was closed due to lightning. We self-parked using the self-park system. Brakes were set and parking checklist completed. I went back to cabin to thank folks for flying with us and then I moved to first class cabin for the same. I noticed that the gate alarm was going off due to jet way sagging approximately 10 low. Agent was there to silence horn or correct. A few minutes later I heard some commotion on the jet way and realized that either jet way was moving or aircraft was. Before I could go back to cockpit, the outbound captain that was waiting to come on board noticed the aircraft movement and rushed to cockpit to verify or set brakes. Aircraft stopped within inches of contacting jet way. There was no damage to aircraft and no passengers were hurt. Once maintenance checked aircraft and determined that is was unaffected, we were towed forward and deplaned the rest of passengers. First officer admitted releasing brake handle to help cool hot brakes. He assumed wheels were chocked even though ramp was closed. He was also unaware of aircraft movement as he was downloading flight plan from i-pad for the next flight and had head buried in his lap. Our procedures are a bit flawed as you don't really know if aircraft is chocked even though they give you the chocks in signal. I find myself more than not looking at aircraft as I walk from gate to gate to make sure it is indeed chocked. I also remember that several years ago we used to never release brakes even if they were hot. On other fleets it is a common procedure to release brakes all the time whether brakes are hot or not.

Narrative: 2

Upon arrival our flight parked utilizing the self-park system without ground crew since the ramp was closed due to thunderstorms. The parking brake was set, the engines were shut down and the parking checklist was performed per SOP. The Captain walked to the galley area to say goodbye to the passengers while I stayed in the cockpit to use my IPad to start flight planning for our next flight scheduled out within approximately 45 minutes. Several minutes later, an ECAM message appeared indicating "Hot Brakes." I instinctively released the parking brake to assist in brake cooling. I resumed flight planning with my head down and soon after the outbound crew's Captain (standing on the jet bridge) rushed into the cockpit and set the brake. I was told that we had rolled backwards but that no damage appeared to have occurred. Maintenance was called to inspect the aircraft and after determining indeed no damage had occurred, the jet bridge was pulled back and the plane was towed forward. Once the plane was again at the proper location, the jet bridge was reattached and normal deplaning was accomplished. 

Contributing factors to this incident are as follows - 99.9% of gate arrivals result in the ground crew inserting chocks within a minute or two but since we utilized the self-park system while the ramp was closed, there were not any chocks inserted. Also, only being on the Airbus for two months, it seems common practice that the brakes get hot on just about every turn resulting in Captains releasing the brakes to assist in cooling and leaving a courtesy note for the next crew that the brake is released. Lastly, the 737 fleet that I flew for three years had the normal procedure to release the brakes as soon as the signal was given that the chocks were inserted. These factors, unfortunately, contributed to the incident that occurred.

Narrative: 3

Aircraft arrived late due to heavy thunder storms including lightning. I met aircraft at gate to help expedite turn around. Jet Bridge settled lower than initially set leaving about a 10 inch drop from aircraft so I started to warn passengers deplaning about step and helping those that needed a hand. Customer service agent also helped to alert passengers and was contemplating resetting jet bridge but passenger were in hurry to make connections and deplaning quickly and we were able to provide a safe assist while standing in front of door. Agent and I noticed what appeared to be Jet Bridge moving forward and agent made a concerned comment as to why it was doing that. I was puzzled also so I looked through the gap between Jet Bridge and aircraft at the ramp and realized it was not the jet bridge moving forward but the aircraft rolling backwards and accelerating. I rushed into aircraft while stopping passengers and set the aircraft brake as fast as possible. Aircraft stopped with approximately 4 inches to spare before it would have struck Jet Bridge. No damage was noted and verified by maintenance. Captain was in first class cabin thanking passenger as they deplaned while First Officer (FO) was in flight deck. Neither one had noticed aircraft was moving as it was very insidious. Apparently FO released brakes because they were hot after parking checklist had been completed and captain left the flight deck, with the good intentions to help aircraft turn around. In all his previous hot brakes he had witnessed captains release brakes after parking checklist complete but did not realized this time the aircraft had not been chocked due to no ramp personnel because of the lightning.

Synopsis

After using the self-parking system to park at the gate due to lightning on the airport, First Officer released the brakes after noticing a Hot Brakes ECAM message. Due to the lightning, no ramp personnel were assisting the aircraft and the wheel chocks had not been inserted. The aircraft began rolling backwards and got to within four inches of the jet bridge before the aircraft brakes were reapplied.

ACN: 1380770 (1544 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14786
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 12552
ASRS Report Number.Accession Number : 1380770
Human Factors : Workload
Human Factors : Troubleshooting
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 397
ASRS Report Number.Accession Number : 1370764
Human Factors : Workload
Human Factors : Troubleshooting
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Total event time approximately 15-18 minutes.

As we climbed to FL320, passing FL270 a loud, but faint noise appeared. Initially I thought the galley chiller had spooled down and then immediately the Lead Flight Attendant called and advised of a loud sound on the floor near Row 7. The First Officer then said he felt a vibration at his feet. We were climbing at approximately 337 knots, Cost Index (CI) of 120, so I slowed the aircraft thinking it might be airframe vibration in the nose gear or wing root and advised both First Officer and flight attendant of that.

Slowing through 280 KIAS the noise cleared. I then began to accelerate the aircraft again to normal speed to confirm the vibration for maintenance. As we began to increase speed the First Officer again stated the vibration at his feet and within seconds of that a flow of "smoke" from behind me and above was moving forward and it began to fill the cockpit. Immediately recognizing the "threat" I directed the First Officer to "put your oxygen mask on," I announced I had the airplane (recognizing he was new on the plane), advised him to [advise ATC of the situation] and to get a direction to the nearest airport to land.

Within seconds of that the Lead Flight Attendant called to advise "smoke" in the cabin and I told her we also had "smoke" in the cockpit and we had [advised ATC] and would be landing in about 15 minutes. I immediately sent an ACARS, in the blind to Dispatch, advising "smoke" and we had [advised ATC of the situation] and are landing at ZZZ1." No reply was made by Dispatch as our ACARS was inoperative. No attempt to communicate via ARINC was made due to being extremely busy trying to land as soon as possible.

I followed ATC altitudes and headings as best as possible and the First Officer began to go through the QRH for smoke/fumes. Communication was quite challenging with smoke, loud noise from aircraft flying speed, small text on the QRH and irritated eyes from the initial smoke exposure and the First Officer and I at several moments had to communicate via hand signals (Ironically the First Officer had complained of his eye contacts bothering him and irritating his eyes a few minutes prior to event and transferring the airplane to me in order to change contacts).

I would like to point that in spite of a lack of ECAM warning and guidance to the initial cause, the First Officer went through the QRH checklist and deciphered as to the best steps to take in an exemplary and decisive manner, although the text should be bigger! I was given a heading towards ZZZ1 which I had requested and ATC gave me the weather as low OVC and visibility. ATC then gave me the weather to ZZZ2, CLR 180/04 Runway 25 and they said it was only a few miles further if we needed it. I looked outside the window and saw a blanket of low clouds/ceilings, I saw the First Officer had run the checklist and I glanced up at the overhead panel switches and noticed the smoke had neutralized, still present, but not filling in the cockpit (I kept mask on due to concern for risk of not knowing contamination could cause us to black out), so I stated we should head for ZZZ2 and I told him I was very familiar with ZZZ2 and he said the same.

I did not think it would be safe to go into ZZZ1 in case navigation equipment may be degrading with low ceilings, thinking maybe it was a fire in the E and E compartment. We were advised by ATC that ZZZ1 runway was 6,000 feet and we acknowledged, both of us feeling more confident that the smoke had been neutralized and that we could safely make it to ZZZ2.

Several communication challenges occurred, once when the First Officer was trying to confirm a part of the checklist and another time when the Lead Flight Attendant called the cockpit three times, each time hanging up when the First Officer answered because she had thought it was the aft flight attendants answering until the First Officer made it clear that she was reaching the cockpit. The First Officer at that point told her we were going to land at ZZZ2 and no evacuation was needed. (Later after our debrief the lead flight attendant had explained how the Captain on the prior flight on the same airplane had asked that anytime the cockpit called the cabin all the flight attendants, forward and aft pick up thus adding to the confusion. Also, the aft flight attendants thought the aircraft might have a belly up landing because the prior flight on the same airplane the Captain briefed them on the wheels being hot).

We were then offered Runway XX to ZZZ2 with light and variable winds and I accepted. The First Officer had already entered the new destination and Runway XX ILS was selected and the approach and landing with medium brakes was made safely. We exited the runway and immediately came to a stop, shut down the number 2 engine and emergency crew and Tower confirmed no smoke anywhere after a 360 degree inspection.

We then taxied into a ramp position and evacuation was kept on hold, but flight attendants were advised that at any time they saw the need to evacuate to do so, engines were off and only APU with electric no PACKS on. It took some effort and time to get an RJ airstair, but it would not safely allow for deplaning, so I requested we be towed to the jet bridge suitable for our airplane. We were towed, jet bridge brought up and all passengers were deplaned safely. We secured the aircraft, I spoke with local Police and emergency personnel, Dispatch, Maintenance (filed a smoke report), Assistant Chief Pilot, discussed and reviewed event with crew.

The crew I had, First Officer and flight attendants, on this particular event was nothing short of professional and executed their safety items in a very effective manner in spite of the many challenges and time constraint, their experience and CRM training was evident (First Officer, although a new hire, certainly used his prior experience in this event). We confirmed that the passengers were safe in the terminal, offering them all available food/drinks from the airplane and once we were told that a rescue jet would be landing shortly to get them safely to [the intended destination] we went to get rest at a hotel.

The following day the entire crew went to an Urgent care for a checkup, I had very irritated eyes and sinus irritation, First Officer woke up with irritated eyes and coughing, flight attendants complained of some irritation to eyes and sinus. There was 33 LBS of dry ice boarded and PACKS had been turned off, in addition to the smoke/chemical exposure. Med link was contacted and approved our return to work.

The event, procedure follow up with all departments and proper care for everyone's safety was very exhausting and taxiing. We finally left ZZZ2 on the flight back to our domicile and everyone felt a great sense of relief once back to normalcy. A delay to return to work for the entire crew was a prudent course of action. A tremendous crew, I was proud to have worked with that day, and an asset to the safe operation for passengers.

Narrative: 2

Going through the QRH, the smoke began to be less intense as the checklist progressed. Unfortunately, during the progression I did not press the galley switch to an off position. The mask and the smoke hindered my vision and communication with the Captain and I was unable to 100% verify that the switch I was intending to press was the correct switch. Moving forward through the checklist, I made the decision that the smoke was not connected to an avionics smoke issue and moved to the lower portion regarding the "air conditioning smoke is suspected" section. At that moment I finished the section and felt like the smoke did not continue to infiltrate the cockpit.

I rejoined the Captain and assisted in changing our destination and set up the ILS approach to Runway XX at ZZZ2. Medium brakes were selected and the approach descent checklist was completed. The Captain made another call to the flight attendants and let them know we did not expect to evacuate the aircraft on landing. The Captain disconnected the autopilot and auto thrust systems. During the approach and descent, it was difficult to hear each other clearly and we reverted to hand signals for the gear and the flap settings.

The Captain landed safely and we exited the runway, started the APU, and came to a complete stop on the ramp. At that moment we received an ECAM message: AIR PACK 1+2 FAULT as well as PACK 1 OVHT. We followed the ECAM procedure. The Fire and Crash crew drove around the aircraft and said they saw a small amount of "smoke" running from our number 2 engine. The Captain directed me to shut down the number 2 engine. We opened our windows to allow the smoke and fumes to exit the cockpit.

Synopsis

A320 flight crew reported during climb the First Officer's eyes became irritated, a floor vibration developed, and then the aircraft filled with sulfur smelling smoke. A diversion was made to the nearest suitable airport. An air conditioning system fault was suspected.

ACN: 1380500 (1545 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10987
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 9257
ASRS Report Number.Accession Number : 1380500
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Security
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After an aircraft change following an air return, passengers were boarded and we pushed off the gate. When we were number 1 for departure, we had not yet received final weights so we sent a MISC LW message to load planning visa ACARS. The reply message said that they were still waiting for customer service to finalize the seat map. I decided for no real reason to ask the Cabin Crew to accomplish a manual head count; this is not a required or even a recommended procedure, since the passenger final count is based on the seat map from customer service. The head count result was 12/117. After waiting for another few minutes, I called the load planning desk on my cell phone to inquire about the delay in final weights, and I gave the supervisor the final count numbers. He took it down, and we hung up. A few minutes later, we received the final weights via ACARS, and the passenger count on it was 9/76, with a final gross weight change of over 10,000 pounds of reduction. I immediately informed load planning via ACARS of the discrepancy, and the reply was that customer service says that their count was accurate. I then replied that we are sure that there are 129 passengers on board, not 85.

After 1.5 hours off the gate, we returned to a gate to refuel, and at that time customer service accomplished a manual "roll call" and checked each passenger against their manifest. During this time, a deadheading pilot informed me that his boarding pass was not scanned when he boarded that second plane, and that customer service just let him through. This indicated an error that probably caused the gross passenger count error. A 10,000 pound discrepancy in the final weight calculations can result in a possible tail strike, or worse, on takeoff.

Synopsis

A320 Captain reported discovering during taxi a gross error in the number of passengers on board which resulted in a 10,000 lb difference between planned and actual weight.

ACN: 1380480 (1546 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway XXR
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : ILS/VOR
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1062
ASRS Report Number.Accession Number : 1380480
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

ZZZ1 has a complex departures procedure which requires extensive briefing. A portion of the departure procedure is predicated around using the localizer frequency which is 110.9. I wasn't aware, but the captain had also hard tuned 110.9 into the Rad/Nav page. We also had to deal with ATC reroutes to our flight plan because of weather in ZZZ.

We briefed the approach and started our descent. In the descent we were very busy deviating around weather. On downwind, my PFD displayed 110.9 which is the ILS frequency for the runway assigned. Once turning base the ILS identified on my PFD was showing the correct four letter identifier and 110.9, and I announced "ZZZZ Localizer 110.9 identified." We were told to maintain 2000 until established and cleared for the ILS. The captain armed the LOC and it displayed as armed on the PFD. However, the autopilot did not capture the localizer. At this time we were in visual conditions. We noticed the error and turned back towards the localizer, and at the same time ATC queried our position. The captain noticed the frequency was wrong, and we were able to correct the mistake by clearing the Rad/Nav page. ZZZZ Localizer loaded in the Rad/Nav page and we were able to rejoin the localizer. However, the distractions caused us to be above the glide slope and fast. The captain attempted to configure and slow the aircraft while on the approach. Above 1000 feet he called for flaps full. I told him I was unable because we were too fast. At 1000 feet he called for flaps full again and I told him we were above the flaps full speed. I also stated we are probably going to need to go around. At 500 feet we caught sight of the runway but were still high, fast, and not fully configured. I called for the Go Around. The captain agreed and executed a Go Around recovering from our errors.

We cleaned up the Aircraft, briefed, and returned back for another ILS. The ILS was executed to a full stop landing.

Synopsis

A320 First Officer described an un-stabilized approach that resulted when the flying Captain had previously hard tuned 110.9 for the ILS of the departure airport in the FMC RAD/NAV page and forgot about it. The arrival airport used the same frequency for the ILS being flown but the autopilot would not capture the LOC until the frequency was retuned. A go-around was initiated.

ACN: 1379953 (1547 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Poor Lighting

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Indicating and Warning - Air Conditioning and Press.
Manufacturer : Boeing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1379953
Human Factors : Time Pressure
Human Factors : Training / Qualification

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I worked Aircraft X at the south end of ZZZ airport. There was little airport lighting in the area at that time. I worked MEL 21-7. Vent avionics sys fault. Changed the skin air outlet valve ended up being no fix. Per MEL procedures I deactivated the valve in the closed position with the internal flap in the open position per Aircraft Maintenance Manual (AMM). But it was hard to tell the difference between when the valve is in the closed position and the internal flap is in the open to 45 degrees position and there was also no amber xxx on cabin pressure page with deactivation switch in the off position during FR EVAC procedure. 

Lack of knowledge of how exactly the valve was supposed to respond after complying with AMM. It was also hard to verify the difference between where the valve is in the closed position and the internal flap is in the open to 45 degrees position. There was absolutely no way to physically verify that the position of the valve was completely closed and locked other than press on it. Which all would do would be seat it in place. Pushing on the valve would only seat it back in place with no slack or movement to help with the verification. Even physically verifying the flap 45 degrees to open was physically impossible. The difference of when the smaller flap open to when the larger flap closes was by turning a lever and back it off. It does not move then backing it off to store the lever in the flush position. Lack of knowledge to document reactivate and deactivation of valve per MEL 21-7 on log page or non-routine which was all just lack of knowledge.

Lack of resources. I was working remotely from the south end of the airport without a computer to verify my work and lack of proper lighting for verification process. 

Stress because of these factors plus pressure management to clear MEL.

Add to the MEL FR process to visual check the skin air valve is in the full close position, flush fuselage skin.

Synopsis

An A320 Mechanic reported being unable to clear an MEL item, and then improperly re-deferring the valve due to time pressure and inexperience.

ACN: 1379697 (1548 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAX
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Route In Use : Visual Approach
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : B747 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6815
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 829
ASRS Report Number.Accession Number : 1379697
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On arrival into LAX we were being vectored off of the VISTA2 behind a 747 for a visual approach to 25L. Approach asked if we had the preceding heavy 747 in sight. We acknowledged that we did and were subsequently cleared for the visual approach to 25L, caution wake turbulence. We were on a base leg and the 747 was on final at 12 o'clock and approximately 4 miles so we continued on the base leg to square off the intercept in order to create sufficient clearance for wake turbulence. However a few moments after clearing us for the visual and us taking responsibility for wake turbulence the controller instructed us to turn left 280 for the intercept. We made the turn but were now closer than we felt comfortable and proceeded with plan b for wake avoidance and flew the approach above the 747's approach path. We both noted the quartering tailwind and in spite of descending slightly above profile had intermittent wake encounters. Nothing violent, but better described as light turbulence or dirty air. As we approached the 500 ft gate we felt that the air was too unstable to continue the approach and initiated the go around. Shortly before our decision to go around the controller issued a takeoff clearance to a heavy on 25R. We were climbing out on runway heading while the heavy was climbing out on an RNAV (wind corrected) track. I pointed out the heavy at 1 o'clock and perhaps a quarter to a half mile to the Captain and we agreed that we did not like how this was developing as the crosswind was drifting us towards the heavy. As we were leveling off at 2,000 ft the tower controller issued a left turn to the proceeding heavy which had him turning directly into our path. As the heavy went wing up in the turn the controller gave us a left turn away from the heavy. The controller then switched us over to Approach Control. The Approach Controller climbed us and vectored us around for another visual where we completed an uneventful landing.

Several aspects of the approach and the missed bothered both of us; and we both agreed that we are seeing more and more of these occurrences. The first is being cleared for a visual approach and then after being cleared for the visual being issued a vector. While most of the time this results in no harm it does create ambiguity. Because by accepting the visual we are responsible for maneuvering ourselves for a stabilized approach and/or for spacing on the preceding aircraft and yet the controller is chipping away at our intercept that we need for a stabilized visual or worse chipping away at our spacing with the preceding aircraft - which we are now responsible for. The second issue is launching aircraft on an RNAV departure with an aircraft on a visual approach to the parallel runway. In the event of a go around the aircraft on the visual is flying a heading while the aircraft on departure is on a wind corrected RNAV track. This can create a dangerous situation on its own. The third issue, and perhaps the most dangerous, is issuing a clearance to an aircraft BEFORE issuing a clearance to the aircraft that could be endangered. For example, Ground Control telling a taxiing aircraft that other traffic will give way before telling those aircraft to give way - I hear this constantly. In this instance the preceding heavy was given a turn toward us before we were given a turn to remain clear. It is out of order and could lead to another Tenerife. All it takes is a stuck mic, a blocked transmission, or missed radio call and the chain of events is in motion.

Synopsis

A320 First Officer reported executing a go-around after encountering wake turbulence in trail of a B747 on approach to LAX.

ACN: 1378721 (1549 of 2908)

Time / Day

Date : 201608
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZMP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Cruise
Airspace.Class A : ZMP

Component : 1

Aircraft Component : Pneumatic Control Valves
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Other Documentation
Problem : Improperly Operated
Problem : Design

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1378721
Human Factors : Workload
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Dispatch
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Flight was operating to the west coast on a manually built route crossing the Rocky Mountains in Colorado. The fight was originally planned and released with a Method 1 (M1) terrain clearance solution. Maintenance deferred an engine anti-ice valve in the open position, so release two was planned and released with MEL 30-21 applied. This new flight plan was planned and released with a Method 2 (M2) solution with DEN and LAX as the listed M2 alternates. 

Once enroute, weather allowed for a more direct routing, so the dispatcher asked the crew to request direct from their present position (over Nebraska a bit west of the SPAWN waypoint) to the BLD VOR (located in southern Nevada). The crew acknowledged, requested, and was cleared direct to BLD, and informed the dispatcher of that fact. Wishing to compute a new flight plan for this new direct route (in order to get new fuel burn and ensure terrain clearance), the dispatcher requested a FMC position report, which was entered in. The dispatcher then put the new route in and computed a flight plan. The error "no point-circle crossing found for ZZZ" (or something to that effect) was displayed. To correct this error, the dispatcher went to the EROPS tab and clicked "default" (the flight had been planned with "prep man" selected and alternate airports of ZZZ, DEN, and LAX). This altered the EROPS solution to alternate airports of LNK, INW, and LAX, and produced a solution with no errors (other than perhaps the usual "insufficient fuel" or similar error that is always generated when calculating an airborne flight). The dispatcher then released the flight with the new direct routing.

Immediately upon inputting release 3, the dispatcher realized that there was no terrain clearance solution generated. The flight was still east of the Rockies at the time and crossing over areas of high terrain, so a terrain clearance solution would have been required. After changing the position report to input the more accurate, lower, zero fuel weight to attempt to force a M1 solution, and "prep manning" the EROPS page to input specific M2 airports, still no solution could be generated that provided terrain clearance. The chief dispatcher on duty was called and could not figure it out at first either. Eventually the chief dispatcher was able to generate a M2 solution when the EROPS "default" button was selected and DEN was removed from the list of available airports (COS was selected as the M2 airport). The dispatcher then put in release 4 with COS as the M2 alternate and provided the crew with the terrain solution and COS weather and NOTAMS. It should be noted that DEN was forecasting gusty variable winds, and thus would be considered unsuitable. However, "ignore suitability" for DEN was attempted and still did not produce a valid terrain clearance solution.

The safety concern here is that the dispatcher was able to plan and release a flight crossing an area that would require [our flight planning software] to generate a M1 or M2 terrain clearance solution with no error generated whatsoever to alert the dispatcher that this had occurred (Additionally, a FAR was unintentionally violated by issuing release 3 with no terrain clearance when it was required). It is conceivable that a M1 or M2 solution would not even be possible in certain circumstances and this would pose a safety risk.

Synopsis

A Dispatcher described his difficulty manually revising an A320's inflight terrain clearance release over the Rocky Mountains. An engine anti-ice valve had been MEL'ed open and special procedures were required because of the available alternate airports.

ACN: 1376415 (1550 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Cabin Lighting : Low

Component : 1

Aircraft Component : Cockpit/Cabin Communication
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Interphone System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Experience.Flight Attendant.Total : 24
Experience.Flight Attendant.Airline Total : 24
Experience.Flight Attendant.Number Of Acft Qualified On : 8
Experience.Flight Attendant.Type : 30
ASRS Report Number.Accession Number : 1376415
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Just before reaching the gate the emergency all call light chimes were flashing and chiming. I picked up the interphone and there was no one there. The chime kept going. I gave the phone to my flying partner asking if he heard anyone on the other line. We tried to call the Captain there was no contact. We tried to call door 1 there was no contact available. There was an odd smell in the cabin and mist. I asked my flying partner to watch my door I was going up to door 1. Walking briskly through the cabin, the smell was strong and constant chime kept ringing. I asked the Purser if she was able to get in touch with the cockpit. She said no. I knocked on the cockpit door. We heard over the pa to pick up the phone. I picked up the phone and could not hear anyone. It took about 30 more seconds and finally heard the pilot over interphone say it was ok to disarm the doors. We did. Chimes were still going. 

I went back to door 2 passengers were asking if that was smoke on the plane. Once at door 2 the flashing light and chime came on to call captain. I tried and there was no contact available. Then the emergency all call lights continued. We were at the gate with jetbridge attached. I called up to the cockpit and asked if the pilots would please stay onboard and debrief with us. The First Officer said the Captain was in the lavatory and he would talk to him when he was out. Meanwhile the chimes are still going. Passengers began to deplane. The First Officer called to the back and ask what it was I wanted. I said we needed to know what the protocol is when something like this happens and how did this happen? He did not know how this happened. He said protocol is just what he did he made an announcement for us to pick up the phone. I was hoping the pilots would be onboard to debrief with the crew. They were gone. Mechanics were onboard after passengers deplaned.

I asked if the pilots explained what had happened? Mechanic had a piece of paper in his hand and was not clear on any details. I explained to him what had occurred. I briefed the new Captain and flight attendants on what happened. Suggestions-pilots stay onboard and debrief with their crew make a pa announcement assuring passengers all was ok (instead I had to do this after finally having connection on interphone to disarm) possibly make an announcement letting us know all was ok considering there was no phone contact for 2-3 minutes but the emergency all call light continued to flash and chime. Possibly stay and brief the mechanics. We were concerned for our safety. In the back of the aircraft we had the strange odor and were getting ready to possibly evacuate. We did not know what was happening up front. The passengers did not know what was happening.

Synopsis

A320 Flight Attendant reported the Emergency ALL CALL lights and chimes were flashing during taxi to the gate along with an odd smell and cabin mist. The pilots did not return repeated intercom calls and did not debrief the crew or Maintenance after deplaning.