ACN: 1376086 (1551 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y

Component

Aircraft Component : Toilet Furnishing
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1376086
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1377117
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Security
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Primary Problem : Human Factors

Narrative: 1

We reported to the aircraft, reviewed the paperwork, the log book and noted MEL[for] Fwd Lav Deactivated. The MEL denotes conditional statements under Remarks or Exceptions. Specifically; one of the exceptions for this MEL follows:

"The Pilot-in-Command will determine if the flight duration is acceptable with a FWD/Upper Deck lavatory unusable..."

I had a discussion with the First Officer (FO) explaining why I believe the "flight duration" for this MEL was not acceptable; giving equal weight to all points.

Leaving one pilot on the flight deck for the time it would take to traverse the entire length of the aircraft presents an unnecessary risk to the safe operation of the flight. Considering our destination and the current threat level (Elevated Condition), leaving one pilot on the flight deck while the other transits the cabin creates increased exposure and an unnecessary risk to flight deck security.

One of the flight attendants also expressed her concern for flight deck security having one pilot in the back of the aircraft passing through roughly 175 passengers to get to the front of the aircraft.

Considering the duration of the flight, it's reasonable to expect that one of the two pilots will need to leave the flight deck at least once to address a physiological need. During flight the passengers will certainly be informed that the Fwd Lav is not usable alerting the attention of any potential "bad actors" that an opportunity may exist. Additionally, passenger perception of flight safety is negatively impacted when a pilot leaves the flight deck to use the Fwd Lav; not to mention the Aft Lav. Simply stated; passengers don't want to see the pilots flying the airplane, in the back of the airplane...for any reason.

And certainly of lesser consideration than safety and security; in the age of Social media everybody has a camera. When a pilot exits the flight deck to use the Aft Lav all the passengers will already know where he's going. I don't believe the Company would appreciate seeing this on the internet, the local news or tabloid TV.

To be sure; there is no shortage of scenarios to contemplate when operating under this MEL provided the Pilot-in-Command takes due care to consider them. I called maintenance control to inform them of my decision. The "flight duration" on this particular flight under this MEL is not acceptable. The Maintenance controller informed me that the Lav will need parts and they will not be able to make repairs.

It brings no joy to my heart to have to make this decision but unfortunately for me... it's my decision alone to make. My report to Maintenance control was followed by a phone call from the Chief Pilot (CP). CP began by asking in a very direct and intimidating tone, "...if [destination] is not acceptable...what is acceptable...". I replied that I could run the airplane up to [a nearby airport]; about a 45 minute flight. That was clearly not the right answer and CP continued with that same line of questioning; "...what is acceptable...".

CP did not seem at all interested in the reasons for my decision and I didn't try to explain my decision to him as he grew more irritated by the minute. As the discussion progressed, it became increasingly obvious to me that the only successful outcome to this conversation was to fly the airplane and accept the MEL, regardless of the circumstances. Needless to say, the conversation ended badly.

CP said (quite emphatically) "...You have an airworthy Aircraft... (I respectfully disagreed)....effective immediately both you and the FO are off the trip.....you're both grounded.....you're not flying again until I see you at my desk for a Section 13 hearing...". He then hung up the phone.

The flight was cancelled about 30 minutes after our conversation ended and myself and the FO were provided dead head transportation back to our crew base.

Over the past several years I've refused an aircraft on two separate occasions under similar circumstances. In those cases; one resulted in a reasonable delay with an Aircraft swap and the other resulted in a cockpit visit from the Assistant Chief Pilot followed by an aircraft swap. In either case, there was no punitive action; apparently that philosophy has changed.

The MEL language has never changed.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

An A320 crew reported refusing to accept a "serviceable aircraft" due to an inoperative forward lavatory for security reasons.

ACN: 1374724 (1552 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class A : ZLA

Component

Aircraft Component : Electronic Flt Bag (EFB)
Manufacturer : Jeppesen
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1374724

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As has happened several times since I have been using our EFB, the Jepp program became unusable in flight. This has occurred when Jepp released a half-baked update that caused failures and required patches to be subsequently released. This failure, however, was unique in that it did not involve any major update, and it involved BOTH of our EFBs at the same time. We had both updated our program in the crew room, and shortly after takeoff, my FO's program started to get real slow. Then, as we prepared to brief the STAR and approach, both of our programs locked up and failed to function after numerous attempts to restart the program and reboot the EFB. It was IMC in LAX, but because my FO had a personal copy of Fore Flight, which is authorized to be used, we did not have to request a written description of the Star and approach (which is a joke and would probably yield a result which resembled not the STAR and approach, but a dog's breakfast) or divert. 

I think, and I'll bet the Department of Transportation would think, it is ridiculous that we do not have company-provided backup for the Jepp software in the form of ForeFlight or some other program. To make pilots have to bear the cost of a ForeFlight subscription if they want to operate a flight with a backup for the Jepp program which has proven to be less than robust, or choose not to and be placed in a less-than-safe scenario such as I found myself, is immoral, unjust, and flies in the face of any assertion that safety is their priority. I can't believe the FAA actually signed off on the practice to only back up the EFBs with a program we have to buy, or a written description sent by dispatch. Can you imagine texting dispatch for another approach because you went missed approach in IMC due to windshear and had to use another approach which the controller is already vectoring you from? Wouldn't happen for real, but it works on paper.

Provide us with Foreflight for free.

Synopsis

A320 Captain reported continuing problems with the reliability of their EFBs and recommended the company provide flight crews with a retail navigation program as a backup.

ACN: 1374645 (1553 of 2908)

Time / Day

Date : 201607

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1374645
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was assigned to do an airstart. It seemed like the crew was not ready and very confused as to when and how to complete the task at hand. I waited for the signal from my co-worker to start the unit. It seemed like the crew was not ready because they wanted to keep the PC (Preconditioned) air on the aircraft as the airstart unit was running. The PC air hose is attached at the bottom of the number two engine, which is very dangerous while the engine is running. PC air got disconnected before the air start unit was started. As I was given the signal to start the air unit I noticed the number one engine was going. That is also very dangerous because I have to disconnect from under the number one engine. The lead on the gate tried to tell the captain two times to start number two only. But he disregarded all direction from him and started the number one which I was under completing the air start. I ask[ed] the Captain to please stop the engine so I can disconnect from the aircraft but he seemed frustrated at the situation and just walked away from me back into his seat. The jet bridge was still up to the plane. I was able to disconnect without getting hurt or any damage to the aircraft.

Synopsis

Ramp Agent reported an airstart at the gate during which the safety of the ground crew was disregarded by the Captain starting both engines.

ACN: 1374378 (1554 of 2908)

Time / Day

Date : 201607

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Pitot-Static System
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
ASRS Report Number.Accession Number : 1374378
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While following A-check job card for cleaning and servicing the pitot probe, you must first stand on an 8 foot ladder to access the Left Hand side E&E compartment, then attach a dry nitrogen cart to the disconnected pitot probe and blow 29 PSI of nitrogen through the pitot for a minimum of 3 minutes!! Then reconnect the pitot probe!

Did I just read this right? I just filled this tiny confined space with nitrogen and now being instructed to climb in there.

If I don't pass out and asphyxiate in the compartment I might just get lucky and pass out and fall of this eight foot ladder on a nice cushy concrete floor. Really? How did this procedure make it to the manual?

Callback: 1

The Reporter stated the procedure in the Aircraft Maintenance Manual as well as the Manufacturer Maintenance Manual calls for using Nitrogen as air pressure to clean debris from the Pitot Probe by disconnecting the plumbing from the probe inside the forward compartment then using a "boot" over the probe on the outside blowing nitrogen into the Pitot Probe which results in nitrogen entering the compartment. The Reporter stated as he understands it, nitrogen is a deadly gas that inhaling it can cause death. The Reporter stated there is no reason why compressed air couldn't be used instead of nitrogen. The Reporter also stated that there is a history of using nitrogen in place of compressed air which caused death.

Synopsis

On an Airbus A320, an A-check job card called out to clean the Pitot Probe using Nitrogen in an enclosed environment.

ACN: 1374356 (1555 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1374356

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1376050

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Evacuated
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Prior to boarding we were told by Captain to hold off on boarding passengers as we may have a major hydraulic leak (as I heard it in back galley). Maintenance was in & out of flight deck. Approximately 20-25 minutes later as I was standing in forward cabin talking to other FAs & the gate agent, the Captain gave a thumbs up to board the aircraft as the mechanic had signed off on the issue. I voiced my concern to the other FAs & gate agent that I could not understand how we could have a possible hydraulic leak & the issue be resolved so quickly. I also understand that a pressurization test was done, but the forward boarding door was never closed. On taxi-out, after Captain gave announcement to prepare for departure & engines were revving up, an odor was smelled and felt by passengers and crew & I heard a loud screeching noise. An FA made announcement that Captain was aware of odor and checking into it. I was trying to call Captain about noise, when the Captain gave the evacuation command. Passengers were already at doors by the time I could access. I had to put up my hand & tell them to stand back until the slide deployed and I knew I had good slide. We had many wheelchair passengers which slowed down evacuation. C FA & I had to switch doors due to elderly gentleman being dragged by his wife w/his cane to my door. C FA, being male said he would grab him under the arms to sit him on slide. I continued evacuation at 2R.

Narrative: 2

I found hydraulic fluid streams and drips on the lower fuselage during the pre-flight walk around, from just aft of landing gear doors to the APU. Additionally, there were two puddles under the aircraft. The captain and I notified Maintenance, who had us write it up in aircraft maintenance logbook. Maintenance lowered the landing gear doors, and I accompanied the contract Maintenance tech while he examined the compartments. No leaks were found. The tech also said he went into a tail compartment, and he said both were dry. The captain suggested pressurizing the three hydraulic systems, which were accomplished, and no leaks were detected by maintenance. Maintenance signed off the discrepancy. We closed the door and pushed back and all was normal. We departed approximately 10 minutes late. We received the load close out; engine 1 start and single engine taxi were normal. Shortly thereafter, we started engine 2. Soon thereafter, we received the first call from the flight attendants came in about noxious fumes. We had completed our Taxi Checklist down to "Line." Almost immediately, the cockpit began filling with fumes. We opened the cockpit windows, and then received two more calls from FAs. This was happening while multiple ECAM messages were popping up and master caution alarms going off at same time. A brief evacuation was implemented. We needed to get people out to fresh air. We grabbed our QRHs and the Evacuation checklist was run. The captain asked, "Are we really doing this?" and I agreed we had to get them outside. An evacuation was commanded over the PA, Tower was notified of the evacuation. The captain and I moved to the cabin to help get the few remaining passengers off, then FAs. The captain coached an elderly lady out on the slide. I found passengers coming back into airplane due to hydraulic fluid on a slide, and so I directed them to another slide. We finished checking the cabin before I departed the aircraft on a slide, with the captain right behind me. When I left, there was a definite rain shower of fluid still pouring out of aircraft. I helped the captain and the #1 FA move the passengers to the port side of the aircraft before then moving to starboard as fire trucks were arriving. Buses were delayed crossing the active runway, but finally arrived and transported passengers back to the terminal. The captain and I stayed with the aircraft until maintenance was able to get air stairs to the aircraft and I could get my non-descript bag from cockpit.

Synopsis

A320 First Officer and a Flight Attendant described a passenger evacuation when a hydraulic leak turned into dense smoke and fumes in the cabin and cockpit during taxi out.

ACN: 1372438 (1556 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1372438
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Before flight was able to push back, they lost all yellow HYD fluid and Maintenance (MX) was called out. After speaking with our contract [mechanic] on the field they found out he was a new A&P and had less than 1 year experience. At that point MX control began to discuss amongst themselves that they hoped a relatively new [mechanic] could find the leak. MX was able to locate the leak at a seal (I believe) and Aircraft On Ground (AOG) located another part. Then everyone realized that because of the inexperience of the [mechanic] on the field the policy would not allow him to inspect the part. Policy states something like, [mechanic] with less than 1 year experience are not allowed to Receive and inspect parts. The more experienced [mechanic] who could have inspected it would not be into the airport for several hours. Crew scheduling jumped in with that the crew would time out, and the pressure went on from SOC and MX supervisor. They started asking if there was an exception to this rule. Could we use an emergency MX provider to receive the part? WHY WAS NO ONE CONSIDERING WHY WE HAVE THE RULE! Was the young [mechanic] on the field exceptional, maybe? Was that the governing decision making process? No, company pressure to not lose a crew or take a longer delay was. Delays should not trump safety! To be clear no one was trying to be unsafe on purpose but the pressure not to take delays/time out crews is now the governing principle valued by management. Considering the amount of MX management and controllers run out of the department because management wasn't getting the performance they wanted, I can understand how this situation could be set up. Here is a couple other examples I have seen in the last couple weeks. In the FOM there [are] 4 MX items that we are not allowed to plan and international flight with inoperative, unless receiving direct authorization from the director of ops. One of which is the TCAS, in the last month an aircraft with a TCAS inoperative was on a line that went internationally multiple times. When I asked if there was a swap for the aircraft, I was told they wouldn't want to take a delay when they can just get permission to fly the aircraft. WHY DO WE HAVE THE RULE? If it is not an issue to even be considered, then why include it as a safety/operational restriction. A new dispatcher had an aircraft with weather radar failure in flight. Some of the first things said to him were that according to the FARs we can continue to destination, will we be able to complete this flight and deal with it at the destination. Despite an area of scattered storms. The flight was able to be sent around the weather safely, but it is a lot of pressure on a new dispatcher to feel as he is dealing with his first mid air issue at the airline. Not intentional pressure to force completion, but again WHY IS THERE A RULE? Captains routinely complain about ops not having headsets while pushing back aircraft into a busy airport. There is an authorized way to do hand communications to safely push back aircraft, I get that. But why is the preferred and standard procedure to use a headset? I get that there are exceptions to all these rules for a reason, there are times when situations don't fit into the governing principle. An airline runs a dangerous operation when it plans to using exceptions instead of the rule. 

[Airline] has to respect safety and the intended way operations are to be conducted constantly making the exceptions work because doing that is the only way to keep the airline running sets us up to have a failure that is a tragedy. Stop lionizing people for making something work. Something that is an exception or slightly in the gray area that because it was used saved a crew, an hour, or whatever. While questioning anyone who doesn't take that route because they are trying to run the operation by the rules it was designed to operate under. IF YOU DO THIS YOU ARE TACITLY ENCOURAGING PEOPLE TO JUST MAKE THE OPERATION WORK BECAUSE THAT IS ALL YOU VALUE. I have been told when I ask questions multiple times that management knows that there are systematic problems, ops not answering their messages or no MX scheduled on the field till hours after the first scheduled departure, but if you just will find a way to keep it moving the management team will sort it all out. They are trying. THAT IS ON ITS WAY TO CREATE A CORPORATE ATTITUDE THAT IS A DISASTER FOR SAFETY. The fix for this is to create a standard operating procedure that you mean, and then follow it. Yes there should be occasional deviations from this but they should be considered significant and viewed in as a possible deviation from the safest course of action. There has to be communication about how we need to operate according to our standard operating procedures (particularly in the SOC), and then support of people who act on that despite the possible delays.

Synopsis

A Dispatcher reported that a flight was unable to push back due to a hydraulic leak and there would not be a qualified maintenance person to inspect a replacement part for several hours.

ACN: 1371387 (1557 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27600

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9250
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 7512
ASRS Report Number.Accession Number : 1371387
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1371416
Human Factors : Situational Awareness

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1371694
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Severe turbulence encounter approximately 100 nm east at FL280. Encountered a 10-15 second period of severe turbulence. Conditions were IMC with light precipitation and light chop. Pax were seated at the time, FAs (Flight Attendants) were not because we were well west of the convective activity (Although aft FAs were self seated). There was no indication of convective activity in front of the aircraft on the radar. All convective activity was behind the aircraft by approximately 30 NM. Altitude loss was approximately 400 ft. ATC queried us about our altitude and we responded with a severe turbulence report and that we were returning to FL280. After exiting the turbulence area, we immediately checked with the FAs for injuries and cabin condition. There were no reported injuries, and since there were no carts out, the cabin condition was good. [Reports] were recorded for severe turbulence and ACARS inoperative via [Commercial Radio] voice with Maintenance Control.

Narrative: 2

Had a flight depart heading to west. Wx had not fully developed at time of occurrence through the mid-west. Remark was added to RLS (Release) to observe convective outlooks/sigmets. There were FPGs (Flight Plan Guidance) present for turbulence watch areas of FL320-410. Flight was planned through area at lower end of that alert FL320. The crew ended up flying a little lower at FL280 where they encountered SVR (Severe) Turb approximately. Ended up calling in on radio because they were having ACARS problems and was advised of the SVR report. The encounter lasted approximately 15 seconds. The crew was expecting choppy rides so everyone was seated and there were no injuries. As a result of the turb the Right Front windscreen developed a crack to the outer pane of the window. Crew followed procedure in QRH and concurred that it was safe to continue with no restrictions.

Narrative: 3

[Report narrative contained no additional information.]

Synopsis

An A320 encountered severe turbulence at FL280 flying through the Midwest and lost 400 ft, however no one suffered injuries. As a result of the turbulence the right front windshield outer pane cracked. The flight continued to its destination.

ACN: 1371136 (1558 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 88
Experience.Flight Crew.Type : 88
ASRS Report Number.Accession Number : 1371136
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5107
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1616
ASRS Report Number.Accession Number : 1371149
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Airspace Structure

Narrative: 1

We were flying the FMS Bridge visual to runway 28R in SFO. When we checked in with approach they told us to join the course only and maintain 6,000 feet. The next communication was confusing. They told us to join the course and comply with the altitudes but we were not cleared for the approach. We didn't understand the clearance. We asked again and they said the same thing; join the course and comply with the altitudes. We read it back that way and stated that it sounded like a weird clearance.

ATC then called out traffic for us at 11 o'clock. We called the B737 in sight but were never told what their projected flight path was. We assumed it was lining up for Runway 28L. The B737 flew through the final for Runway 28L and headed in the direction of our final. I told the First Officer to level off the descent. We got a TA and the B737 crossed 700 feet directly below us. 

We continued the approach and a go around for unrelated reasons (aircraft not clearing the runway). I believe that ATC made a mistake telling us to descend on the FMS Bridge Visual. The phraseology they used is not standard and created confusion. I have never been told to descend via an approach both laterally and vertically but not cleared for the approach. I asked ATC to clarify and they simply restated the confusing clearance.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A pilot received a clearance from ATC using non-standard phraseology and was unsure if they could begin their descent on the approach. The pilot was also issued incomplete crossing traffic information. The aircraft received a TA and stopped its descent for the traffic that the pilot reports passed 700 feet below them.

ACN: 1370617 (1559 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Distribution
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6100
Experience.Flight Crew.Last 90 Days : 244
Experience.Flight Crew.Type : 4470
ASRS Report Number.Accession Number : 1370617
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Issued New Clearance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the last phase of climb and about to reach FL340, the aircraft passes through a zone of light turbulence (without representing any special consideration for it) and at that same time the AP (AUTO PILOT and A/THR (AUTO THRUST) systems disconnected by itself without any apparent reason, following this situation the screens on the cockpit experienced a "BLACK OUT" for about 1 or 2 seconds (A similar situation could be observed with a change of electrical power when using the external power unit when on ground) afterwards ALL navigations capabilities on the aircraft were lost and the EWD (ECAM WARNING DISPLAY) only showed the following ECAMS, AUTO FTL A/THR OFF and ENG THRUST LOCKED (THR LEVERS.....MOVE)

The Capt (PILOT MONITORING) at that time, calls out for a positive exchange of controls as established by the company policies and SOPs "I HAVE CONTROLS"... and then commands the start of "ECAM ACTIONS". We follow the ECAM and made a quick evaluation of the general condition of the aircraft discovering the following:

1) A cross check of the flight instruments was made looking for indications that could lead us to suspect of an ABNORMAL V ALPHA PROT or UNRELIABLE SPEED INDICATION without finding any differences in relation with airspeed or altitudes indications in between the primary and the standby flight instruments.

2) We were not able to re-connect the AP (AUTO PILOT) and A/THR (AUTO THRUST) systems by any means available.

3) Both PFD (PRIMARY FLIGHT DISPLAYS) showed indications that lead to the loss of flight protections (ALT LAW/DIRECT LAW).

4) Both ND (NAVIGATION DISPLAYS) neither showed the map route nor any other navigation references that help us to establish a good geographical position. Both ND remained with the ARC selection.

5) Both MCDU's (MULTIFUNCTION CONTROL DISPLAY UNITS) were frozen and did not accept any inputs.

6) The behavior of the aircraft in flight regarding handling was really sensitive to flight control inputs on hand flying but it remained within positive control for pilots.

The Capt. (now acting as PILOT FLYING) contact ARTCC and informs them about the loss of the automation and navigation systems of the aircraft and made a request to leave RVSM (REDUCED VERTICAL SEPARATION MINIMA) while continuing the flight to destination its evaluated and the possibility of diverting. ARTCC maintain the flight at FL340 for few minutes and ask if we were able to fly to [a nearby airport], our response is that it is not possible due to the lack of navigation systems, we then received a vector to fly and a clearance to descend to FL280. While this is happening we made a new Cross Check on flight instruments looking for differences on indications and we pull down the magnetic compass for a better references in flight without finding variations on indications.

At the same time we continue with the coordination and task sharing for the flight crew so the First Officer (Now acting as PILOT MONITORING) refers, finds and start the procedures stablished by the QRH and Flight Crew Operations Manual regarding the different scenarios that were stablished after the first evaluations made at the start of the failure and considering the following procedures: (presented in this report with no specific order of application)

- IR ALIGNMENT IN ATT MODE - NAV IR DISAGREE - NAV ADR DISAGREE - ADR 1+2+3 FAULT - ABNORMAL V ALPHA PROT - UNRELIABLE SPEED INDICATION - ALTERNATE AND DIRECT LAW - LANDING ASSESSMENT - FUEL CALCULATION

Suddenly the PFD (PRIMARY FLIGHT DISPLAY) and ND (NAVIGATION DISPLAY) of both Captain and First Officer came back to a "NORMAL" state of operation but AP (AUTOPILOT) and A/THR (AUTO THRUST) did not respond again to any activation attempts, seconds after, the failure appeared again and PFD and ND returned to its previous failure state described before.

At this point we were transferred to [adjacent] ARTCC and we transmit our decision to divert, this decision was made after as a CREW, Captain and First Officer determined that the aircraft status in this condition was not reliable for the continuity of the flight, representing an undesirable state, situation that affected the safety of the passengers and flight crew aboard the aircraft. We receive a vector to fly north and minutes after being established on that heading we were transfer back to [original] ARTCC. A moment before contacting ARTCC the aircraft recovered by itself its normal flight conditions. All navigation and automation systems were able to recover for flight.

ARTCC inquired us about the status of the flight and the aircraft condition besides a general description of what was happening; We advised ATC that the navigations and automation systems were not operating normally due to intermittent failures that occurred minutes before and the failures did not make the aircraft reliable for flight, and we stated that this conditions represented a risk for the possible loss of flight protections of the aircraft that could lead to undesirable condition during landing (Thinking about a DIRECT LAW). ARTCC gave us clearance to flight direct to ZZZ, we then informed of our intentions to use RWY 28R at ZZZ witch is the longest runway available at the airport and requested the presence of the Emergency crew and equipment of the airport. 

At that moment we made a new full revision of all procedures described regarding the failures that could have affected us minutes before in order to have a better understanding of what could happen if the failure came back again or by any means what could happened if we had a much worst situation during the remaining of the flight and landing attempt, after that, we ask for vectors to the Northeast in order to burn some of the remaining fuel so we were able to avoid an overweight landing at ZZZ.

We must remark that we made all necessary coordination's and communicate all our requirements at all times with ATC, FLIGHT ATTENDANTS, STATIONS and BRIEFING TO PASSENGERS. The flight Crew CRM performed with an excellent communication level and understanding, always with a full respect and proactivity in order to resolve the situations that we faced during flight until our landing at ZZZ, also, all the decision were made in conjunction and with a full understanding of its consequences. The final approach and landing were made in normal conditions, the failures at those points never came back again, but we were on alert in case any failure appeared once again.

Synopsis

A320 Captain reported experiencing an electrical anomaly at FL340 resulting in loss of autopilot and autothrust along with Navigation Display information and frozen MCDUs. PFDs indicated Alternate Law. ATC is advised and the flight diverted to a suitable alternate airport.

ACN: 1369632 (1560 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : APU Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1369632

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During taxi out (APU was shut down after engine start) prior to takeoff, APU exhaust fire was reported by an aircraft behind us. We then found that APU fuel valve had failed to close after shutdown. We pressed APU Fire and pulled APU fuel circuit breaker per mx control to no avail. Thanks to good work by [the] fire department no evacuation was required as the fire was extinguished. We taxied safely back to gate followed by the fire trucks. We never received any ECAM or cockpit warning of the APU fuel valve disagreement. The only cockpit indication was found when we selected the fuel system page where we were able to see the APU fuel valve stuck in the open (or not closed) position with the APU shut down.

I believe ECAM should have caught this, but am still researching. Should we check the fuel page before takeoff?

Synopsis

A320 Captain reported returning to the gate after shutting down the APU because of an APU exhaust fire.

ACN: 1368899 (1561 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : DC Battery
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1368899
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ELEC BAT 1 FAULT ECAM at cruise approximately. Maintenance Control advised that we could not continue to [the destination] and dispatch recommended diversion. We held for approximately 45 minutes in order to burn enough fuel to reach max landing weight.

Synopsis

A320 Captain reported experiencing an electrical issue in flight followed by a diversion.

ACN: 1368644 (1562 of 2908)

Time / Day

Date : 201607
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35500

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1368644
Human Factors : Distraction

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Primary Problem : Airspace Structure

Narrative: 1

We were given a climb clearance to FL360 from the previous controller who had us on a vector for traffic. He then cleared us back direct to intersection ZZZZZ passing FL320. We received a frequency change passing FL330. After checking in with him, he advised us that we would have to be level in 2 1/2 minutes. At the time we did not see a problem complying with the restriction. About this time we received a company message telling us that our return flight was going to have to land at [a different airport] to clear customs because of the late departure that was expected. At the 2 1/2 minute point we were passing FL355 and the controller asked us to turn 15 degrees left for traffic. We saw the traffic visually and also on TCAS. We did not receive any TA or RA for the traffic. Once level at FL360 he then cleared us back to ZZZZZ. No comment was made about the event from the controller so we decided to file the ASAP to emphasize how easily a distraction can take you out of the green.

This event occurred mainly because of the distraction from the received company message. We were trying to respond to him via ACARS and became distracted from the compliance with the climb instructions which we originally thought would be made easily, but since the climb rate had slowed up more than expected, we ended up being about 500 feet low.

Synopsis

During climb in an A320 in contact with ATC controllers for vectors for traffic and clearance for flight levels, the flight crew was distracted by a company message via ACARS about a return flight causing a slowing of the climb rate.

ACN: 1367853 (1563 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : B787 Dreamliner Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 201
ASRS Report Number.Accession Number : 1367853

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 3050
Experience.Flight Crew.Type : 3050
ASRS Report Number.Accession Number : 1368100

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Airspace Structure

Narrative: 1

On the FMS Bridge Visual to 28R at SFO, I was on the course outside of the San Mateo Bridge when Approach called traffic, a heavy 787 turning short final to 28L. I picked up the visual on the heavy in his base turn just outside of the San Mateo bridge, but lost him above the windscreen shortly thereafter. The Captain had sight at all times and commented that while we were high but correcting, he was extremely high. We were switched to Tower and checked in. At some point, Tower asked the 787 if approach had instructed him to follow behind the Airbus, us, and he replied affirmatively. Tower then told him to start slowing immediately. The Captain made another remark about how close he was and shortly thereafter, we got a "descend" RA. I complied as best I could, but we only had 1,000 feet to work with. The Captain relayed to Tower that we were complying with an RA, but Tower misunderstood and asked to say again. Suspecting a go around, I took one last look out to the left of the aircraft to see if the RA was still valid, and immediately saw the 787 above, slightly ahead and in a left hand turn to align with the runway. I estimate the separation at no more than 500 feet. We executed the go around and landed uneventfully.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew on final approach reported a heavy B787 turning short final above them. The flight crew received a RA to descend, and executed a go-around.

ACN: 1367235 (1564 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1367235
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Dispatch
When Detected : Pre-flight
Result.General : Work Refused
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Flight was originally assigned to aircraft number ABC. This aircraft had two CDLs (Configuration Deviation List) applied on it. CDL 27-13 and CDL 27-14 however the CDL manual shows under limitations "No more than one part of one system may be missing except if otherwise specified. Parts of different systems may be simultaneously missing, unless otherwise specified in this list". Since the CDLs are both under the same system, number 27, I determined the aircraft was not airworthy to Dispatch. I notified Maintenance Control and the Maintenance Manager. They both disagreed with my determination however, since I disagreed I refused to Dispatch Aircraft ABC. A Maintenance Manager, who said he was the boss of the Maintenance Manager [on the phone], came to my Dispatch desk and began to argue and pressure me to Dispatch the aircraft. Maintenance also came to other Dispatcher to try to pressure to Dispatch the aircraft. Aircraft ABC was allowed to continue to fly another 2 flights in an unairworthy condition until it was finally put out of service and one of the CDLs was cleared.

Policies and procedures need to be in place to support the Dispatcher and prevent Maintenance from pressuring them to Dispatch an aircraft they have refused or deem unairworthy.

Synopsis

A320 Dispatcher was coerced into releasing an aircraft with two section 27 CDL's even though the CDL limitation stated "No more than one part of one system may be missing except if otherwise specified. Parts of different systems may be simultaneously missing, unless otherwise specified in this list."

ACN: 1365854 (1565 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZMP.ARTCC
State Reference : MN
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZMP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZMP

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 11430
ASRS Report Number.Accession Number : 1365854

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This is the first report I have written involving an increasing problem with flying into DEN. Today, we were told twice we needed to be rerouted to Denver on another arrival fix and STAR for traffic. They always come back when queried that it comes from the Traffic Management Unit. Every time ATC gives these reroutes well into a flight [they] are 12-20 minutes over original plan and anywhere from 1100 LBS to 2000 LBS more burn for the flight. Every time, the new clearance has put us in a less than 45 minute fuel state at arrival. Every time after accepting the clearance and programming FMS, [we have to] advise ATC the new route puts [us] in a min fuel state where if returned to original flight route puts us landing with more than 45 minutes of fuel. ATC could solve this problem by either giving us a reroute early, like before departure with PDC so evaluation and additional fueling can take place or metering/slotting DEN like the New York airports.

Synopsis

A320 Captain reported an increasing number of reroutes received from ATC enroute to DEN that resulted in fuel concerns.

ACN: 1365225 (1566 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Engine
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1365225
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During takeoff just after rotation received ECAM ENG 2 EGT Overlimit followed by multiple compressor stalls on engine 2. Accomplished non-normal procedures and returned for an overweight landing. Upon return to gate maintenance advised that the N1 Fan section of the engine had seized.

Synopsis

A320 flight crew experienced an engine compressor stall during takeoff. Procedures were followed ending with a return to the departure airport.

ACN: 1363621 (1567 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Cowling
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1363621

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1363624

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Following a normal flaps 1 takeoff and climbing through approximately 5500 feet on departure, we received a call from one of our flight attendants saying that a passenger reported they saw two pieces of something fly off the right side of the aircraft sometime during takeoff. We asked the flight attendant to further investigate by looking out the window at the right engine and then report back to us. During this time I told my FO (First Officer) to request a level off while I also slowed the aircraft as not to aggravate any abnormal condition with the engine or airframe. Also during this time, all engine and other aircraft indications appeared normal in the cockpit. Shortly thereafter, our flight attendant reported back to us that a portion of the inboard right engine cowling was missing from the aircraft. I acknowledged this and shortly thereafter had my FO coordinate a return to the airport with ATC. Also during this unscheduled return, ATC informed us that a portion of our right engine cowling had been found on the departure runway. I briefed the passengers and flight attendants on our situation and informed them that the aircraft was operating normally and that we would be returning to [departure airport] as a safety measure. My FO and I accomplished all required checklist items and I safely landed and taxied the aircraft to the assigned gate without further incident.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported returning to departure airport after losing part of the right engine cowling on takeoff.

ACN: 1361990 (1568 of 2908)

Time / Day

Date : 201606
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SLC.Airport
State Reference : UT
Altitude.MSL.Single Value : 11500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : S56
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : DELTA4
Airspace.Class E : S56

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1361990
Human Factors : Human-Machine Interface
Human Factors : Fatigue

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1361991
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On the Delta 4 arrival into SLC was cleared to cross JAMMN at 17000. Had verified altitude restrictions with the pilot flying. Approaching the fix was cleared to descend via. PF selected 11000 for lowest restriction at MAGNE. We were still in managed descent. Prior to SPIEK we both verified altitude and noticed we were low. The aircraft had reverted to vertical speed. Started the correction and advised ATC asking for a safe altitude. Was given 13000. We had visual on terrain and no TCAS warning. Was then cleared to maintain 11000 and given a runway assignment of 16L. Continued on downwind and cleared for a visual. 

The flight started with a three hour round trip. We had a mechanical upon return and changed planes. Had to wait for a security check on the plane prior to boarding. Then 2 mechanical issues. No catering bins, had to wait on that. Late push. Late arriving into SLC. I feel fatigue was the main factor in catching the event immediately. The aircraft losing automation was the ultimate cause of the event but was preventable had we been more alert.

Narrative: 2

Vertical Speed being activated at some point without us being aware of the change from Managed Descent. High task loading at this phase of flight. Distraction with the FA announcement procedures as the PF and approach chart verification.

Synopsis

A320 flight crew reported missing a crossing restriction at SPIEK on the DELTA 4 Arrival SLC. The FMGC had reverted to vertical speed with 11000 feet set. The flight crew detected the error just before ATC issued a low altitude alert. Fatigue was cited as a contributing factor.

ACN: 1359365 (1569 of 2908)

Time / Day

Date : 201605
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : Mixed
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1359365
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

About 20 minutes into flight while climbing through 32,000 feet to get to 35,000 feet we heard a clicking noise coming from behind the circuit breaker panel and I noticed the Captain PFD and ND flicker a few times. I suspected we might have a generator issue as I had noticed in the logbook that an IDG had recently been changed on one of the engines but I was not sure which one. We were not issued any ECAM messages. The electrical page did not show anything abnormal except a possible high voltage on the left side TR (no amber indications). We were flying to ZZZ which had weather right at minimums so I attempted to start the APU. Starting the APU resulted in an APU auto shutdown. The electrical issues continued and became steadier so I told the First Officer I thought we should divert back. As we began the diversion the problem continued to get worse. I gave the controls to the First Officer because my PFD/ND were going blank so often that they were unusable. We were unsure which checklist to use as I attempted to look through the QRH. No checklist applied to our issue. I did run a diversion checklist.

The problem continued to get worse. The lower ECAM displayed multiple messages and the overhead panel displayed multiple lights flashing on and off in sequence with the clicking noises heard from behind the circuit breaker panel. The Captain's PFD/ND and upper ECAM were mostly unusable. Multiple systems were shutting off and turning back on again. The autopilot and autothrust failed. The First Officer's PFD and ND stayed useable for most of the flight but would flicker on occasion. The First Officer's MCDU was useable but the Captain's MCDU was not. We were in visual conditions all the way back.

When we put down the landing gear it appeared the aircraft went into alternate law however this was intermittent. "Use manual pitch trim" was displayed in amber on the FMA. I observed the trim wheel moving. The flaps and slats worked and the aircraft was controllable for the approach and landing. Communication with ATC was intermittent due to the electrical issues. Some uncommanded movement of the rudder was felt by the First Officer and I noticed some clicking on the rudder pedals during taxi but the aircraft remained controllable at all times.

After parking and shutting down engine #1 all the electrical issues went away. We considered shutting off the left generator in flight but there was not enough data available to support that decision. Also the APU would not start.

Callback: 1

The reporter stated that he believes that Maintenance replaced the generator control panel to return the aircraft to service. He also discovered after the flight that the QRH does have a checklist to cover the situation but it is not located in the electrical section. It is in the section concerning displays.

Synopsis

A320 Captain reported experiencing electrical anomalies during climb to FL350 which resulted in the intermittent blanking of his PFD, ND and the upper ECAM display. With low ceilings at their destination, the crew elected to return to their departure airport. The First Officer's displays remain normal and he becomes the pilot flying for the approach and landing.

ACN: 1357839 (1570 of 2908)

Time / Day

Date : 201605
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cargo Compartment Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 190
ASRS Report Number.Accession Number : 1357839
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During cruise, a momentary "Smoke Fwd (aft) Cargo Smoke" ECAM warning appeared. The ECAM lasted approximately 2-4 seconds then disappeared. The ECAM could not be recalled using the "Recall" key on the switching panel. However, the ECAM status display showed the Deferred Items as depicted in the Flight Manual and the overhead Cargo Fire Warning panel showed a "Disch1" light, as if the initial ECAM warning was still displayed.

After consultation with Dispatch and maintenance, verifying with the cabin crew that there were no indications in the rear of the airplane indicating a problem, and the fact that the Cargo Smoke system tested correctly prior to flight, it was agreed that this was some type of momentary system fault and NOT a threat to the aircraft's safety. No emergency [situation reported to ATC] and we proceeded towards our destination.

Although the ECAM had cleared itself immediately with no action taken by the pilots, we continued to consult the QRH where applicable and coordinated to have all passengers disembarked prior to the cargo doors being opened on the ground. Additionally, Dispatch coordinated to have Fire Fighting personnel available to use infrared scanning to ensure there wasn't any unusual heat signature in the hold.

The potential issue requiring this filing centers around whether it was necessary to land the plane as soon as possible as the Flight Manual calls for if the ECAM warning had persisted. As the ECAM warning had disappeared almost instantly and couldn't be recalled, and I had conferred with dispatch and maintenance, I didn't deem that landing as soon as possible in an overweight airplane at the closest suitable field to be appropriate.

Synopsis

A320 Captain reported continuing to destination after receiving a momentary ECAM for cargo smoke.

ACN: 1356577 (1571 of 2908)

Time / Day

Date : 201605
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 8100

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Cloudy
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use.STAR : EAGUL6
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8700
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 4900
ASRS Report Number.Accession Number : 1356577
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

While commencing the Eagul6 arrival into PHX after experiencing heavy weather, moderate turbulence, multiple deviations, changed altitude crossing restrictions, and a late top of descent clearance, we were cleared to descend via the STAR. As the pilot flying I selected "managed descent" and ensured the FMA agreed with my input. As we broke out of the weather and turbulence in the vicinity of HOMER intersection, I was somewhat relieved and newly complacent about our, now, easier path to the airport. The captain and I had a short discussion about the fact that we had not been handed over to PHX approach yet, which seemed to be late. Our focus was somewhat shifted to that issue and may have distracted us from the fact that the aircraft's vertical model had somehow shifted into "open descent" from "managed descent." I realized this when the floor of the crossing restriction window was not be honored at ESDEE intersection. I then intervened by disconnecting the Autopilot and leveling off at 8,100 feet. The Captain and I noticed at the time that the descent mode no longer agreed with the previous selection. The Captain agreed that he verified "managed descent" was previously verified by him on the FMA at the time the descent via clearance was received.

In retrospect, neither of us are certain that the "At or above" at VNNOM intersection was honored due to the missed mode change during our multiple flight deck distractions (weather, deviations, late handoff, etc.) from the top of descent point. As a result, I can only endeavor to be less complacent, despite flight deck distractions, when arriving at a familiar airport via a commonly used arrival to ensure that all restrictions are actually being met on the profile.

Synopsis

A320 First Officer reported becoming distracted by weather challenges and did not notice the autopilot had switched from managed descent to open descent.

ACN: 1354527 (1572 of 2908)

Time / Day

Date : 201605
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1354527
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : FOD
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While performing the preflight walkaround, I noticed [a contract ramp person] left paperwork and a pen in the intake of the left engine. This shortcut is completely unacceptable. I waited for him to return and said "You can't be doing that.", and he removed the paperwork.

The next time I see this I'm disposing of all paperwork and material in the FOD bin. I'm tired of seeing this happen repeatedly.

Synopsis

A320 pilot reported finding paperwork and a pen in the engine cowling during preflight walk around inspection.

ACN: 1352344 (1573 of 2908)

Time / Day

Date : 201604

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Tablet
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7786
ASRS Report Number.Accession Number : 1352344

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Just as I was briefing for the departure, a new update became available for the iPad Flight Deck Pro app. It started to download automatically without my desire for it to do so. It rendered the App unusable for the flight as the download didn't complete prior to departure. I had to use paper charts for the flight.

Synopsis

A320 Captain reported he had to use paper charts because his iPad started an update on its own at the gate before departure, failed to finish, and was therefore unusable during the flight.

ACN: 1351869 (1574 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : IMC
Weather Elements / Visibility.Visibility : 0.25
Light : Daylight
RVR.Single Value : 2600

Aircraft

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 25R
Flight Phase : Final Approach
Airspace.Class B : DEN

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12660
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 8669
ASRS Report Number.Accession Number : 1351869

Events

Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Set up for Cat 3 dual ILS 35R Denver. ATIS 1/4 SM, RVR 2600. Planning autoland. Vectored to final, instruction to cross DEANE at 8000, cleared for the approach. We complied with the DEANE crossing, PF calls glideslope alive. Shortly thereafter we got a TCAS descent. We had been watching the 35L traffic on the TCAS and thought maybe the 8000 at DEANE instead of the published 9000 was for separation but the traffic continued to descend, however the lateral offset seemed to remain constant. The PF initiated the descent to the green portion of the VSI and he announced "I'm going to the green, 1200 FPM down." I told Tower were TCAS RA descending. Tower responded by telling us our traffic was on the ILS 35, and then asked if we were able to continue the approach. I looked at the PFD and we were 2 dots below glide path descending with the altitude warning sounding. I knew we were not going to intercept GS on our current path but I didn't know our exact altitude. 7000 was in my head as a minimum. I stated we either need to follow the TCAS or go-around (PF was following the TCAS). Shortly thereafter the PF initiated a go-around. I advised tower and requested a heading and altitude. Tower directed, track the localizer and maintain 9000. Clear of conflict announced and we returned for an uneventful approach and landing.

Synopsis

A320 First Officer reported executing a go-around after responding to an RA on the DEN ILS Runway 35R.

ACN: 1351320 (1575 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17401
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1851
ASRS Report Number.Accession Number : 1351320

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

- Dispatch advised :30 minutes out that [destination] might shut down for arrivals due to thunderstorms.
- Approach had continuous light to moderate turbulence.
- While turning to intercept localizer, we observed thunderstorms West and South of [the airport].
- At approximately 1,500 feet AGL we received a "Wind Shear" warning.
- Wind shear was displayed in front of the aircraft in the terminal area.
- Executed a missed approach. Advised ATC. Started climbing to 12,000 feet.
- Observed severe weather south of the airport in the missed approach path on radar.
- Asked ATC for left turn to heading 120 degrees to avoid weather. 
- ATC said unable due to coordination with other traffic.
- ATC asked what are our intentions [were]
- Advised tower we [were] turning to heading 120 degrees to avoid severe weather.
- Passing through 10,000 feet, tower amended our altitude to 9,000 feet. 
- Knowing that these thunderstorms were closing in on [our destination], we decided to go to our alternate.
- We then advised ATC we would like clearance to our designated alternate.
- Clearance was received.

Synopsis

A320 Captain reported diverting to an alternate airport because of weather at the destination.

ACN: 1350803 (1576 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : HYDRR1
Airspace.Class B : PHX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : PHX

Aircraft : 3

Reference : Z
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet CL65, Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : PHX

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1350803
Human Factors : Workload
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1350810
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Received two TAs, reduced range on NAV Display, visually identified first TA off aircraft right side and visually identified aircraft behind and below left side of aircraft. We both noticed that automatic pilot did not honor the 9000 ft restriction at CHAVO intersection, as PF, I intervened at 8700 ft and climbed back to 9000 ft. After landing, we were notified of a possible deviation.

The aircraft did not honor the 9000 ft altitude restriction at CHAVO combined with two TAs resulted in my having to intervene at 8700 ft and climb back to 9000 ft.

Narrative: 2

On arrival to PHX, we were assigned to descend via the HYDRR1 arrival. Prior to CHAVO, ATC instructed us to slow to 210. A few miles prior to CHAVO intersection, we were descending out of 9500ft MSL with an altitude restriction of 9000 at CHAVO, and last I had seen, the aircraft was showing it would make the restriction. Shortly after, I noticed a B737 climbing under us and just to our right and I called out the traffic. At nearly the same time, ATC called out CRJ traffic to our 10-11 o'clock position and I called it in sight as well. At that moment, we received a TCAS Traffic Advisory from the B737 climbing to our right. We reduced our Navigation Display (ND) range per our SOP, and we received another TA from the CRJ climbing at our left. I attempted to reacquire the CRJ and had trouble spotting it again, and was able to make visual contact off our 9 o'clock position. At that point I noted they were 700 feet below us and turning behind us, approximately 3 or so miles passing to our 7-8 o'clock. We called the traffic in sight and PHX approach switched us to the next frequency. As my attention came back into the aircraft, I noted the Captain had engaged the vertical speed mode and was leveling the aircraft, and I saw around 8,640 on the altimeter, and we had not yet crossed CHAVO, which had a 9000 ft altitude restriction. He initiated a gradual climb back up to 9000 ft at that time and we switched over to the other frequency, which then gave us a descent to 7000 ft. After landing, PHX approach notified us that the CRJ received a TCAS Resolution Advisory from us, while we did not receive an RA and were unaware they had received one.

This event occurred primarily due to the autopilot vertical navigation mode not honoring the 9000 foot crossing restriction for some unknown reason. It was exacerbated by the fact that at the critical moment of having to ensure the aircraft stopped its descent, we were distracted by two close-proximity traffic callouts by both PHX approach and TCAS. We spent several minutes trying to identify the TA traffic and keep them in sight right as the aircraft failed to stop its descent at 9000 ft. We were able to immediately identify that we had descended through the 9000 ft restriction after the traffic conflicts were resolved and were no factor.

The best way to avoid reoccurrence of this event is to continually return back "inside" the aircraft to ensure it is doing what it should be doing, particularly if approaching a clearance restriction. However, having two near-simultaneous TCAS TAs on opposite sides of the aircraft made it difficult for both pilots to stay focused inside the aircraft.

Synopsis

A320 flight crew reported receiving a TA after the autopilot overshot a crossing restriction on descent into PHX.

ACN: 1350152 (1577 of 2908)

Time / Day

Date : 201604

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Navigation Database
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Company
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1350152
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Incorrect / Not Installed / Unavailable Part

Narrative: 1

An Aircraft had a job card assigned on a through trip. In the process of performing the assignment which was a Navigation (NAV) Data Load, I found that the reference for the process was indicated as being for a Boeing 737 instead of Airbus. The Maintenance Control Website did not have the NAV Data identification numbers listed and the Disks provided by management had part numbers that were altered manually with a pen and no initials for penned corrections. I questioned the legality of using these disks but management directed me to use them anyway and verify compliance later in the MCDU. Following that directive the aircraft did not accept the download of said disks and I could not accomplish the download. Aircraft was put back to original software data and a significant delay occurred. Quality Control of NAV Data Disks in the Maintenance Department for these events is questionable. Quality Control of NAV Data Disks and reference material has to be improved immediately.

Synopsis

Technician reported he was assigned to update the navigation database of an A320 using a navigation database for a B737.

ACN: 1349554 (1578 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MMSD.Airport
State Reference : FO
Altitude.MSL.Single Value : 3600

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : MMSD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.VOR / VORTAC : SJD
Flight Phase : Initial Approach
Route In Use.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1349554
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Chart Or Publication

Narrative: 1

I was the non flying pilot and had been briefed for the minimum descent altitude on the airway coming into MMSD. Prior to MARUX we were given the cleared for Approach. The Captain flying briefed the 8000 feet until MARUX and then at the next 15.0 dme fix 3600 feet to 11.0 Dme. As we were cleared for the approach the Captain armed the approach and got the Approach Nav and Final annunciation. When final captured we started our descent to 3600 feet. Upon reaching 3600 feet we were yelled at by ATC that we were not cleared to descend below 6000 feet as it was below Minimum Vectoring Altitude of 5100 feet. We both noticed we were on profile and at this point we were both confused due to the on profile indication. We continued on in day VMC with airport in sight and were cleared Visual Approach while tracking the approach profile. Upon landing we debriefed the event and noticed that in fine print that the segment after MARUX and before the 15 Dme fix was 6000 minimum. After 15 Dme the 3600 min until 11 Dme applies.

We both missed the 6000 foot minimum restriction on the chart and it is not coded into the approach in NAV Data for the VOR Approach. Given the VFR conditions, we continued the approach and landed visually. We should probably have climbed back to 5100 foot MVA or 6000 feet for 4 miles and then continued descent to 3600 feet on the approach. The event occurred because we did not properly brief the altitudes on the approach chart.

Synopsis

A320 First Officer reported not noticing the 6000 ft minimum vectoring altitude between MARUX and the IAF on the MMSD VOR DME-2 Runway 16 Approach. The reporter stated the altitude is not coded into their navigation database.

ACN: 1349510 (1579 of 2908)

Time / Day

Date : 201604

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Tower : IAH
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 487
ASRS Report Number.Accession Number : 1349510
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Runway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We landed on 8L in IAH. Exited the runway, taxied down taxiway NP where we proceeded to hold short of 8R behind a 747. As we stopped on NP we were told by tower to expect an expedited crossing clearance. They gave the 747 clearance to cross and we proceeded up to and stopped about 10 feet short of hold line for 8R. We sat there and waited maybe a minute or two for landing traffic. Then the Tower controller came on and gave us an extremely quick and what seemed like to me a very garbled clearance to cross 8R. I immediately asked the Captain if that was for us. He said yes and was in the process of pushing up the thrust levers. I then proceeded to ask the Tower "confirm [flight number] cleared to cross 8R?" I got an immediate affirmative followed by literally in the same breath "hold short of 8R". When the Captain and I heard "hold short" both of us stepped on the brakes at the same time coming to a rather abrupt and immediate stop, but by this time we were roughly 5-10 feet past the hold short line. My response over the radio milliseconds after we stopped was in effect "but we've already crossed the hold short line".

The response by the Tower was "no worries, [flight number] go-around". The plane that went around I'm guessing was around 800-1000ft from touchdown. At no time was there any risk of collision and if we hadn't been told to stop, I feel like we would have expedited across the runway without incident. On the taxi in I asked ground for a Tower number to discuss the situation. In the conversation that the Captain had with the Tower supervisor after we blocked in, it seems that confusion arose from me asking for a verification of the clearance, which I did not hear a flight number for. The Tower sup said we took too long to respond even though we were initially given an affirmative to cross by the controller at the time. The whole situation seemed to have been unnecessarily rushed even though we were expecting the expedited crossing clearance. The whole event happened so fast that it felt like 45 seconds to a minute if that.

Synopsis

A320 First Officer reported they experienced a runway incursion at IAH when they misunderstood an ATC clearance.

ACN: 1348983 (1580 of 2908)

Time / Day

Date : 201604

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1348983
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Upon arriving at the airport 2 hours early I pulled up the paperwork for the flight and saw that the plane had an inoperable APU. I also noticed we were going to continue to DEN with the same aircraft. We were scheduled for a 10:48 hours duty day with all things going smoothly. I texted the Captain with my concerns about flying two legs with an inoperative APU and that I wasn't comfortable with it. I finished my breakfast and walked to ops to meet the Captain. While in ops the Captain received a call and hung up and said he wasn't sure who it was but they asked if he was flying with a new guy and to convince him to take the plane. I asked for the number and called back to see who it was and he said it was the Duty Manager, so I told him I was the first officer and that I would be filing a report, at which point he asked what my safety concerns were and I said I'm filing a report and goodbye. He called the Captain back soon after and said that the first officer doesn't get a vote and to convince him to take the plane. The Captain was later paged in ops and took a call from Operations and was told the plane was loaded and ready to go at which point the Captain said the plane was refused. He called dispatch to confirm the refusal and dispatch said the Duty Manager called and told him we were taking the plane? At some point while sitting in ops someone approached me in a very aggressive manner and said "Are you [the First Officer]? At which point I stood up and held out my hand to professionally introduce myself and he refused to shake my hand. He was extremely agitated. It was the assistant chief pilot, at which point the Chief Pilot (CPO) walked out of his office and quickly intervened by calling him into his office. After he talked to the assistant chief pilot the CPO called the Captain into his office for a conversation of which I was not privy to. I am extremely upset with this entire fiasco and felt pressured to change my position. As a first officer I felt extremely insulted and intimidated by this entire event and sometimes wonder what CRM is all about? If this wasn't pilot pushing I don't know what is. I think the fact that I am a first officer had something to do with it, as though I am not a part of the decision making process.

Synopsis

A320 First Officer arrived early for a two leg trip and discovered that the APU is MEL'd. He informed the Captain of his findings in the hope that the aircraft would be refused, which eventually occurred. This caused a row with the Duty Manager.

ACN: 1348723 (1581 of 2908)

Time / Day

Date : 201604

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAB

Component

Aircraft Component : GPS & Other Satellite Navigation
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 7901
ASRS Report Number.Accession Number : 1348723
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued New Clearance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We were enroute to EAGUL intersection at FL360 and had just briefed the EAGUL STAR and visual approach to PHX runway 8 when we overheard a couple of other flights noting anomalies with their GPS systems. These reports seemed to be connected with NOTAMs and RAIM notices about GPS outages near El Paso and Hector VORs. Then we began receiving ECAM for GPS2 FAULT and notices for loss and regain of GPS navigation. Nav accuracy seemed good, within 1 mile of known VOR locations. However, we noted persistent, random map shifts of up to 7 miles which lagged and was sometimes consistent with the cycling ECAM messages and notepad advisories. Rather than risk being unable to monitor an RNAV arrival, we chose to seek a less rigorous STAR at the suggestion of ATC. We were flying heading only, under ATC direction for about 10 minutes before they cleared us for the BUNTR STAR which we flew uneventfully. We monitored position and track with manual navigation radio and radial tuning. I looked for but could not find a checklist for the cycling malfunctions indicated by the GPS navigation loss notices in the MCDU scratchpad. We did the procedure anyway. The protocol to be directed to a checklist by a set condition fell short of certainty because of the random, sometimes rapid cycling of the notes themselves. We simply applied judgment that told us to confirm position accuracy and then involve ATC with awareness of our less than ideal navigational condition. There was also no time to coordinate more exotic solutions through dispatch because the descent was upon us by the time we received our new clearance.

Synopsis

A320 Captain experienced GPS signal interference and intermittent outages during arrival to PHX. The interference seemed to be connected with NOTAMs and RAIM (Receiver Autonomous Integrity Monitoring) notices about GPS outages near El Paso.

ACN: 1348229 (1582 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Personal
Make Model Name : Small Aircraft, High Wing, 1 Eng, Fixed Gear
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Mission : Personal
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1348229

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1348230

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

When departing on Runway XX it is required to back taxi from Taxiway X. When approaching the runway on Taxiway X there was an aircraft that was not finished with a run up in front of us. The ground controller then instructed the aircraft to cross Runway XX and hold on Taxiway Y to complete the run up. We were then instructed to back taxi full length for Runway XX. When reaching the end I performed a 180 degree turn and we were then cleared for takeoff. When making the turn we cleared the area including having the aircraft in sight sitting on Taxiway Y. Once we were about to rotate, we passed and noticed that the entire aircraft was over the hold short line on the runway side. We continued the departure without incident. The ground and tower controllers should have noticed that the aircraft was on the runway before we were cleared for takeoff.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 crew reported that a general aviation aircraft completing their run up was completely over the hold short line for the runway and they should not have been cleared to takeoff.

ACN: 1347852 (1583 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1347852
Human Factors : Fatigue

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

For some reason, this company is building pairings that violate their own policy on safety and fatigue. This is a relative new practice and is unprecedented in terms of forcing unsafe human factors on the pilots.

I spoke to a scheduler about this and was told, "well, you're legal for it". I tried to explain that being legal was not the point of my phone call. Being safe and fit for duty was.

These red eyes built into the middle of a pairing are unsafe. That's a strong word but 100% accurate.

I just finished one of these and have never been so exhausted in my career. I had every intention of calling off "too fatigued to fly" on the last day of the pairing. But, as it turned out I was in a hotel that was well designed for daytime sleep and I got three hours which allowed me to finish my trip while maintaining an adequate degree of safety. Had that not been the case I positively would have called off fatigued.

Dangerous fatigue reoccurred on the drive home from the airport. I nodded off three time (split second) while driving. I pulled over and napped in a parking lot. This all happened as the direct result of the construction of this pairing. 

But it's not unique. There are many pairings that now have red eye legs in the middle. These pairings are built looking only at legalities and ignore all human factors. Inadequate rest has residual effects and cause mistakes to be [made] in the airplane. Even if they are small ones, like missing a radio call, they are the direct result of fatigue.

Each pilot handles the decision individually. Unfortunately for me, I made the incorrect decision and kept pressing on when I know I should have called off "too fatigued". That's a tough call to make for a professional pilot. 

[On this assignment] the first leg was an evening leg to [our destination]. We arrived at the hotel at XA:30 EST only to learn the hotel restaurant and bar were closed for renovation. We were hungry so my FO (First Officer) and I left the hotel to find a restaurant a small distance away. We returned a little after XC:00 EST [early morning].

The construction noise started at XG:30 AM local and lasted continuously until XS:00 local. After two hours I called the front desk and spoke to the manager on duty. I explained we needed to sleep during the day but he replied there was nothing he could do. The hammering and saws continued non-stop until after our departure time for the red eye leg.

We flew [two more legs] and landed at XE:07 AM EST. When wheels touched down we had been awake for nearly 29 hours without opportunity to sleep. We both discussed calling off too fatigued but decided to try to force some rest. We were expecting street noises to be a problem but the new hotel is actually perfect for daytime sleep and we got through it. 

I don't know why but there are times when being overtired actually interferes with sleeping. I got 2.5 hours of sleep and my FO reports he got 6 hours. We departed that same day at XR:25 local for two legs getting back to [home base] after midnight.

Although this is a specific case, the general point I am making is that these red eyes built into the middle of a pairing are unsafe. They violate company policy as stated in the [company manuals and training courses], and they force the pilot into a tough decision when inserting the Fit for Duty prompts, a federal requirement. And scheduling says you are FAR 117 legal. They have no clue how these impact the body.

Falling asleep at the wheel on the drive home has gelled the safety issue in my mind and caused me to pledge to myself that I will never do this again. It may not be convenient for company if I call off [fatigued] but next time I am faced with these circumstances there will be no decision to be made because the decision has already been made. Safety first.

Synopsis

A320 Captain reported extreme fatigue resulted from his three day assignment that included a "red eye" leg on day two.

ACN: 1347820 (1584 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 5213
ASRS Report Number.Accession Number : 1347820
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

FA's (Flight Attendants) initially advised of unresponsive passenger with the implication that she was not breathing and potential divert. More details to follow. Called back in 5 mins, pax responsive now with paramedic and doctor attending. Sent [request for land-based physician to call me]. FA called back about the same time as [Dispatch] and juggled the two calls. FA provided info from Medical form and indicated neither medical personnel had credentials with them, but wanted to start an IV. [Ground-based medical resource] was very concerned with BP (Blood Pressure) and HR (Heart Rate). Finally was able to provide that info after about 10 more minutes. FA's also advised pax was on O2 and IV. Initially asked for Priority Handling as pax was improving and did not feel [a more urgent flight status] was necessary. ATC wanted to know the nature of the [adverse situation] and seemed reluctant to provide priority handling. Provided the info to them and they seemed less than cooperative. Eventually after discussing with FO (First Officer) and FA's we elected to declare [an urgent medical situation] after Center and approach denied our request to land opposite direction. Our thoughts were, we had previously had a pax who was unresponsive and was now on O2 and had an IV in. Figured it was better to [an urgent situation] and later learn it was not necessary/required than the opposite (and then learn we needed the priority it affords us). Paramedic was very helpful and FA's did a fantastic job as did the Dispatcher. [Ground physician link] also very responsive and helpful. Landed uneventfully and taxied to gate and deplaned pax. Only real issue was that we told station personnel that we would plan a normal deplane as the passenger had improved significantly. Upon arrival at the gate the EMT's wanted to immediately come onboard and get [ill passenger] off the aircraft, so that was a bit confusing. Otherwise it was textbook abnormal event.

Synopsis

A320 Captain reported receiving less than he felt was appropriate cooperation from ATC during a passenger illness event.

ACN: 1347026 (1585 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1347026
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1347033
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

Over an hour after departing a check weights message appeared in the MCDU scratchpad. A check revealed that the computed weight was approximately 20,000 heavier than ECAM displayed weight. We then discovered that the load closeout indicated a Zero Fuel Weight (ZFW) of 103,130 and Total Souls on Board (TSOB) of 29. The actual TSOB was 137.

Prior to gate departure, we discussed the takeoff performance. ACARS indicated a Proposed Takeoff Weight (PTOW) of 155.5, RWY XX, CONF 3, APU ON, and 12 knot tailwind. V speeds based on this message were inserted into the MCDU.

We pushed off the gate and taxied to a holding point to wait for the load closeout. When it arrived, the First Officer inserted the required data into the MCDU. He noticed that the weight was different than planned and pointed this out to me, but it was a performance advantage and the ACARS message was valid. We took off on Runway XX using CONF 3, APU ON, and calculated V speeds.

The event occurred due to an error in finalizing the load closeout. Flight crews no longer verify the actual passenger count against the load closeout as a matter of procedure. The inherent flaw in this policy is that a barrier is removed that in all likelihood would have trapped this error. Since the takeoff was accomplished using this performance data, the potential risk involved was mainly in setting the stab trim from the erroneous data in the load closeout.

The company should consider adopting a policy that specified a limitation on a discrepancy between the passenger count reported by the gate agent and the load closeout.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported responding to a "Check Weights" message in flight and discovered that there was a discrepancy in the weight and balance information. They were approximately 20,000 pounds heavier because of a mistake in the total souls on board.

ACN: 1346872 (1586 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MSP.Airport
State Reference : MN
Altitude.AGL.Single Value : 300

Environment

Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : MSP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : MSP

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : MSP
Aircraft Operator : Air Carrier
Make Model Name : Medium Large Transport
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : MSP

Person

Reference : 1
Location Of Person.Facility : MSP.TOWER
Reporter Organization : Government
Function.Air Traffic Control : Other / Unknown
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 18
ASRS Report Number.Accession Number : 1346872
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

MSP Tower was operating a 30's/35 Converging Runway Operations (CRO)/ Arrival Departure Window (ADW) operation with strong wind and gusts. Aircraft X notified local control west of a loss/gain (a phrase universally used by flight crews when encountering windshear) of airspeed of 15kts on Runway 35. Local Controller West (LCW) notified the Controller in Charge (CIC) with the tower operations manager and a supervisor present. Aircraft Y notified Local Control West of a loss/gain of 15kts at 400 AGL. Sustained winds had been average around 290 at 20 gusting to 26. LCW notified the CC of a second report of loss/gain of airspeed on Runway 35. The Cab Coordinator (CC), CIC nor Operation Manager (OM) or Sup asked LCW if the aircraft would categorize this as "windshear" which is required by MSP SOP. CRO ops were allowed to continue at MSP even though a direct violation of MSP SOP's happened in front of the observing OM, CIC and CC.

I would stop allowing MSP SOP's to require the controllers and flight crews to be over burdened with semantics (16-13(b) during a critical phrase of flight by questioning the pilots about windshear. Second if this requirement remains I would expect the OM, CIC, SUP and or CC to assure it's compliance by controllers during the safety sensitive operations of CRO/ADW] at MSP.

Synopsis

MSP Controller reported of windshear at the airport and management not changing the runway configuration.

ACN: 1346608 (1587 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Taxi
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1346608

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

This report is due to Ramp Service Employees not following Standard Operating Procedure (SOP), I was not assigned to this flight, I was walking to the break room and observed an unsafe operation and noted the Gate number, flight number, time and aircraft number. 

I observed an Airbus 320 at Gate XX, Left engine #1 is running at high idle speed while aircraft marshaller is in the process of lowering aircraft ground power unit cable (GPU) from jet bridge to connect GPU to aircraft. I observed right side of aircraft, aircraft forward and aft cargo doors are open, Ground Support Equipment (GSE) Beltloader is positioned to aircraft cargo doors, GSE tractors, carts and Ramp Service Employees inside Ramp Operation Safety Zone while Engine #1 is running and aircraft beacon light is flashing. Left main landing gear chocks was not in place, right landing gear chocks position is unknown, I did not observe. Ramp personnel at Gate were not following company safety standard operating procedures, and was performing an unsafe duty.

Synopsis

Ramp Agent reported observing a ground crew opening cargo doors and moving belt loaders into position before the number one engine had been shut down and the beacon turned off.

ACN: 1346058 (1588 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1346058

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1347025

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Captain flying. After an extended taxi, with a pass through the deicing pad, no ice detected, continued towards runway. Approximately 10 minutes before TO started engine 2. After engine 2 start and at power change over received a FLCT SPLR FAULT. Following non normal methodology decided to contact dispatched and advised. Connected with maintenance and a reset of SEC 3 was suggested and instructed by maintenance. This cleared the fault and all flight controls and spoiler movement was checked. After takeoff, FLCT SPLR FAULT returned. No high roll rate experienced or autopilot disconnect with fault. Through 10,000 ft followed non normal methodology, and ECAM exception procedures. First officer is now flying pilot. After discussion with dispatch, selected a diversion airport. Enroute [we advised ATC] and managed fuel to an acceptable landing weight. Uneventful landing and taxied to gate.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported a FLCT SPLR FAULT after takeoff. Crew followed recommended procedures. After discussion with Dispatch, the crew diverted for an uneventful landing.

ACN: 1345650 (1589 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 0601-1200

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1345650
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Procedure

Narrative: 1

When we take arrival of A319/A320 aircraft, these aircraft always arrive with the number 1 engine (Captain's side) running. The pilot's procedure is to leave this engine running until the passenger loading bridge is moved into position and the ground power is hooked up and turned on.

This creates a situation where the potential for aircraft hose ingestion is increased. Additionally, this requires that one of the wing walkers remain giving the "Engine Running" signal for the safety of [any] passersby, thus slowing the offload process. The team lead cannot give the "Safe to Approach" signal because this engine is still running, but in spite of this the wing walker on aircraft right tends to take it upon themselves to open the pits and move the belt loaders into position without that signal. Also, we find that the main cabin door is opened and passengers begin to deplane while the engine on that side is still running, exposing them to undue noise.

Airbus aircraft are the only aircraft exercising this procedure.

Synopsis

A Ramp Agent reported A319/A320 aircraft left engines remain running after gate arrival until the jetway and ground power are connected. During this delay passengers are exposed to engine noise deplaning and ramp agents become impatient then approach the aircraft with an engine still running.

ACN: 1345218 (1590 of 2908)

Time / Day

Date : 201604
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 126
Experience.Flight Crew.Type : 108
ASRS Report Number.Accession Number : 1345218
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Well prior to top of descent into EWR we reviewed performance and FM wind shear precautions, profiles, procedures as EWR was landing runway 29 due to high winds and LLWS advisories. We assessed and chose flaps 3 approach and incorporated into brief to include the defined touchdown zone/go around point and verbal review of procedure.

First Officer flew a good approach in very gusty conditions but wind shift and gust at approximately 40 feet in transition towards flare caused auto thrust to over correct resulting in a balloon/float. I directed a Go Around (GA) as we approached the end of our pre briefed touchdown zone.

While directing the GA and monitoring power/position, I reverted to non-standard phraseology (from military career) with the tower "... on the Go" versus "... Going Around". In the subsequent high work load of tower vectors, 2 different altitude assignments and a switch to approach frequency, I failed to recognize and correct the flaps to position 2 from the flaps 3 approach position. We evidently reverted to and called the normal(full flap) go around profile call outs and checked "flaps 3" which resulted in a 5-10 knot flap over speed during the go around.

Neither tower nor approach asked the reason for the GA and it was, frankly, too busy a time to give one. Rebriefed subsequent approach with a selected speed and manual throttles to mitigate engine spool ups/downs, and though a challenging approach, landed uneventfully.

Synopsis

Extremely gusty winds prompted a crew to elect to do a go-around just prior to touchdown. The second approach was uneventful.

ACN: 1343479 (1591 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1343479
Human Factors : Situational Awareness
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1343480
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Our aircraft departed with a serious weight and balance discrepancy that could have resulted in a serious aircraft accident if at another airport or with a shorter runway. During final weight and balance inputs, I failed to properly confirm my First Officer's inputs which resulted in a large error of passenger inputs. Our load manifest matched up with the final weight and balance copy, and as usual I performed the fuel reliability check and started making my welcome aboard PA. At this point, my First Officer incorrectly entered 000/67 in Cabin A, when in fact it should have been 67 in Cabin A, and 89 in Cabin B. The total passengers should have been 156. Instead, we only inputted 67 half weights. This resulted in a weight difference of 26,800 pounds under planned! Unfortunately, I completely failed in my duties to confirm his inputs as it gave me a Weight and Balance (WB) solution with Flaps 1, which we were anticipating. The incorrect solution was auto-uplinked, and we proceeded as normal. This was a grave error, and I fully realize the devastating consequences that could have happened had we not had a 10,000 runway in good weather. On departure, we got a MCDU scratchpad message saying "Check Weight." I then looked at our printed out copy of the first page of our WB solution and discovered the serious error. I then corrected our Zero Fuel Weight (ZFW) to the planned ZFW to achieve proper speeds for cruise and landing.

This event could have easily been prevented by verifying the final inputs on the Weight and Balance. I failed in my duties to verify my First Officer's inputs. Unfortunately, we rushed through the process even though we were 5 minutes early. Complacency was definitely a factor as I have never seen this issue and have been flying the aircraft for a very long time. My previous four day was flown with my First Officer and I found him to be exceptionally bright and competent as he has been here a little under 2 years with an impressive aviation carrier and experience. I was lulled into complacency by getting a weight and balance solution with a Flaps 1 Takeoff (TO)/Flex (which was anticipated). I became distracted with other duties (Fuel reliability, Welcome aboard PA, verifying passenger counts on final WB with passenger load sheet, and confirming the passenger count with the A flight attendant), and failed to confirm the final WB inputs. Unfortunately, I trusted without verifying.

This is why there are two of us, and I failed in my duties. This also happened in day VMC at our home base airport. Complacency was once again a factor. I'm much more cognizant of weight and balance at [other] airports. Needless to say, this is an error that will not be repeated on either of our ends.

This could have been prevented in two ways. Verify the final weight and balance inputs. This procedure is already in place. Do not allow a weight and balance solution with a margin of error so high. We got a solution, albeit a grossly egregious one. We uplinked the solution and got an [anticipated] Flaps 1/Flex TO numbers.

Narrative: 2

We had a prelim of 147 passengers. When loaded we had an exceedance of 3.3 percent of Center of Gravity (CG). I then re-entered the Weight and Balance (WB) on ACARS and I thought I entered with a split cabin to resolve the problem. I had not actually loaded split cabin, but half weights. I had entered 000/67 and /87 passenger. This resulted with a 26,800 lbs difference in the actual weight. The system gave us a solution and I did not catch the error. It allowed me upload the solution and I did not check the additional pages showing the issue. I had checked the engine out procedure and notes when I entered the preliminary WB and didn't need to see that info again. 

On take we noticed an unusual take off roll and got an error message on the scratch pad to check WB. We then realized my error and made adjustments so that the WB would be correct on landing. 

We just got complacent. I have flown with the captain before and he is extremely professional and attentive. We created a self-induced time crunch and I had not caught my error. We were lucky it happened at an airport with a 10K [foot] runway and good weather. One issue is that stations are filling out WB paperwork incorrectly resulting in incorrect prelims. AS a crew, we usually have to split cabins and move weights around so that they are properly set. I have never actually been taught the WB on the ACARS computer but have had a quick once-through during IOE as a new hire. Actually having use of an ACARS for automated WB during sim training would alleviate most errors. Also not making PAs after we get final paperwork would throw attention to checking final numbers prior to pushback. I do not intend to make this mistake again.

Synopsis

An A320 flight crew reported departing with a large (26,800 lbs) self-induced error in weight and balance calculations.

ACN: 1342565 (1592 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1900

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 553
ASRS Report Number.Accession Number : 1342565

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 2839
Experience.Flight Crew.Type : 868
ASRS Report Number.Accession Number : 1342261

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On approach FO lowered landing gear, got "L/G Not Down/Locked" ECAM. Followed ECAM which had us cycle the gear. No help, ECAM then directs to lower gear manually (gravity). Gear then showed down. Landed, had to stop on runway due to no nose wheel steering. Got towed off runway to gate.

Narrative: 2

[I] recycled the landing gear lever per the ECAM and the landing gear failed to extend normally. Continuing with the ECAM, I used the Gravity Extension Handle, turned the handle 3 times, and the landing gear extended, gear doors remained open and nose wheel steering was Amber on the display. F/O opened the QRC, and referenced all the cleanup items. The crew discussed considerations on landing, stopping straight ahead and getting a tug to the gate.

Synopsis

A320 flight crew reported using the alternate landing gear extension procedure when the gear failed to extend on approach.

ACN: 1341460 (1593 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 10700

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use.STAR : DRLLR4
Airspace.Class E : I90

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21100
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5732
ASRS Report Number.Accession Number : 1341460

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 227
ASRS Report Number.Accession Number : 1341465

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cleared to descend via the DRLLR landing 26R. 6000 set correctly in FCU window. DES mode used. Both pilots separately and completely checked each altitude restriction. Two separate times PF had to use full speed brakes for altitude corrections, and waypoints correctly showed orange indicating crossing problems. At our request ATC cleared us runway 27 by descending via the DRLLR4 runway 27 except maintain 6000, and after DOMNO fly heading 085. PF changed runway to 27. As the PM returned from a PA announcement, it was noted the PTROL intersection was crossed about 10,700 ft in the DES mode. The correct altitude is between 11000-13000 ft. The PF disengaged the A/P and leveled off. The occurrence happened in DES mode with no deviation from descent path and no orange waypoint.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported the FMS descended below one of the intersection fix altitudes while in descent mode on the DRLLR4 Arrival to IAH. Crew disengaged autopilot and returned to STAR clearance.

ACN: 1339786 (1594 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class D : ZZZ

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1339786
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We started the day with an auto-brake fault MEL. The manual brakes worked normally, we simply couldn't use the auto-brakes. After takeoff, we received the brakes auto brake fault ECAM, with brake released below it. We were expecting the brakes auto brake fault ECAM, but not the brakes released because in the original MEL, it stated that it cannot be displayed to legally dispatch the plane.

We ran the ECAM procedures. After going thru the QRC, QRH and Aircraft Operating Manual (AOM), we learned that at least one wheel will be released. This was the primary concern because we didn't know if multiple wheels would not be braking.

The Captain did all of the communicating with dispatch and maintenance. Then the confusion came getting landing numbers. The inflight performance for the A320 has performance numbers for the auto brake fault and dispatch was able to give us numbers with one brake released. But they couldn't give us numbers for 2 brakes or more released. They were discussing our current MEL with the new issue and no one was able to explain if more than one brake would be released.

The landing airport was calling for rain and with about 7,600 feet available for landing. With one brake released we needed about 5,600 feet to stop. What we didn't know is if more than one brake was going to be released. This resulted in us [advising ATC] and diverting to another airport to use their long runway.

There is so much that can be learned from this event. We tried to use every available resource to make the best decision we could with what we knew at the time. And of course safety is always number one. I have just learned that for the A320, the auto brake numbers take into account for one wheel to be released. What I also didn't know is that if a second wheel was released there would have been a second ECAM.

We actually talked about this during the flight of how we would know if 2 or more wheels were released and we simply didn't know. According to the AOM, it was entirely possible more than 1 brake could be released. If we just knew that, we would have been so much more comfortable with the situation. I also learned to just stick with the ECAM and don't assume more things can be broken than what the airplane tells you.

Synopsis

Aircraft dispatched with auto-brakes inoperative. After takeoff a brake system fault indicated there would be less than full brakes available. The crew diverted to a long runway airport, advised ATC, and landed normally.

ACN: 1339060 (1595 of 2908)

Time / Day

Date : 201603

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9091
Experience.Flight Crew.Type : 4496
ASRS Report Number.Accession Number : 1339060
Human Factors : Fatigue

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Both the captain and I were completely fatigued on the last leg of the trip because of the pairing schedule. We started with an all-nighter, with 13 hours rest during the daytime, flew a late nighter [east coast to west coast] and arrived near midnight. Then we had a short 11 hour rest and then a very early wake up on the last day for a XA:45 AM van pickup for a three leg, 11 hour day. We both realized that we shouldn't have left [on second leg] that morning because we were already feeling the fatigue. But, due to our devotion to the airline and our professionalism we thought we could handle the turn [out and back]. We were wrong. We both were so fatigued on the third leg we were making mistakes. I personally exceeded the 250 KIAS below 10,000 feet MSL limit by about 15 knots, because of my fatigue and the fact that with the sun angle, and even with the PFD (Primary Flight Display) as bright as it would go, I did not see the LVR CLB flashing on the PFD to cue me to select climb power, because the PFD was too dim. If I had not been fatigued I'm sure I would have noticed that we were not maintaining 250 KIAS based on the noise not because I could see the airspeed well enough. I realized at that point just how fatigued I was and it was a painful flight all the way back.

The captain was making his mistakes too, including clearing the ground crew to disconnect electrical power without verifying the APU was operating. He also nearly made some taxi mistakes that I barely caught in time after finishing my "heads down" taxi out duties, e.g., runway change and runway data acquisition. I won't mention any other mistakes the captain made, but it was obvious our fatigue was serious. Our communication skills suffered severely because of or fatigue.

This pairing is a disaster for crew rest. The next time I feel like I did that morning, and have two flights to go, I will not hesitate to call in fatigued. I will just feel bad that the flight will most likely have to cancel. These pairings that mix two all-nighters with [an early morning] departure are a disaster. Haven't we learned that after an all-nighter that a lot more rest is needed than normal? If that pairing had the last flight be the one leg to base that would have been bad enough, but to add on a turn? Disaster! When I mention this screen intensity problem to Standards Captains, LCAs (Line Check Airmen), and PIs (Pilot Instructors) I fly with, they all indicate there's nothing they or anyone can do. I emailed the [Chief Pilot] about it who said he would look into it.

Synopsis

A320 First Officer described an extremely fatiguing three day pairing that is exacerbated by insufficient brightness of the cockpit display screens.

ACN: 1338539 (1596 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1338539
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Time Pressure

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

ZZZ closed for Thunderstorms (TS) while we were on the arrival. We held for about 30 minutes during which time we were planning our contingencies. We were holding slightly east of ZZZ1 and were planning to divert there and advised dispatch. They replied that ZZZ1 was not usable due to some weight bearing NOTAM. That is information that would have been very useful to know BEFORE we started planning to divert there. Had we not found out when we did it would have led to a fuel critical situation.

Our next closest alternate was ZZZ2. We weren't fuel critical, but we didn't really have fuel to fly to a much further alternate, particularly since a line of weather was between us and all the more suitable northerly alternates. I mistakenly assumed that if flight ops publishes an airport as an alternate, that meant that they had assured that the airport was actually suitable for a diversion. This was not the case. By the time we landed we were getting very close to being timed out on duty time. It was very likely we were going to time out in ZZZ2 since the weather in ZZZ was moving toward ZZZ2. 

Well, it turned out that the airstairs in ZZZ2 were about 4 feet too short for an Airbus. In order for the First Officer to perform his walk around, he had to risk his personal safety and climb down to the VERY wobbly airstairs and then climb up again. Had we timed out in ZZZ2 I have no idea how we would have deplaned. I suppose we would have had to call the fire department to come with ladders. Knowing I had no way to deplane and that we were going to time out put extreme pressure on us to depart before the weather passed. The situation was further complicated by our inability to reach ATC for a clearance. We finally had to call 800-wxbrief to get our clearance. Luckily the weather had dissipated quite a bit during the hour we were on the ground there enabling us to depart sooner than we expected. As it turned out we landed with one minute to spare on our duty time. 

It is incredible to me that an operation of this magnitude has not properly vetted out its alternate airports. [Company] needs to establish policy and practices to ensure that alternates are suitable and have proper servicing equipment. Additionally it would be useful if we had a system to rapidly disseminate changing information concerning alternate airports before we depart. Perhaps we should consider issuing airport info pages for alternates which outline what services are available. Perhaps there is some way we could also graphically rank alternate suitability so it would be more clear during real time operations. 

Duty regs are designed to improve safety, but in this case they served very much to undermine it. I was under extreme pressure to depart soon than I was ready to depart. As a result we made several errors in our predeparture setup, all of which we luckily self-trapped. I was eerily reminded of the Tenerife accident where the captain was pressured to take off in order to remain legal. I'm sorry to see that not much has changed in 40 years.

Synopsis

An A320 flight crew diverted after destination airport closed for thunderstorms and discovered after landing that even though the airport is listed as an alternate its services were not suitable for his aircraft. The air stairs were four feet too short for his aircraft type.

ACN: 1338414 (1597 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 18000

Aircraft

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Direct
Airspace.Class A : ZAU

Person : 1

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Function.Air Traffic Control : Instructor
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 22.1
ASRS Report Number.Accession Number : 1338414
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Instructor
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 8.0
ASRS Report Number.Accession Number : 1338985

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Exited Penetrated Airspace
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

My trainee and I were scheduled to relieve another Controller and trainee at the sector D-side. My trainee and I returned from a break and noticed that the previous Controller that we were to relieve was working another radar position. The sector had no Radar Assist at the time and the sector was very busy and very complex. My trainee and I jumped in with no briefing, as the Radar-Side trainer and trainee were too busy to give a briefing. The military airspace appeared on our radar scope. Aircraft X was in the airspace at the time climbing out of 18,000 feet for 20,000 feet. We coordinated a climb to 28,000 feet to get out of the airspace which was capped at 26,000 feet.

We did then notice two messages which stated the times the airspace would be in use. There was no evidence of any previous messages stating the airspace would be going active. Neither ourselves nor the Radar trainer or trainee had any other prior notification that the airspace was going active. It was apparently listed on the big board, but we did not notice it due to the volume and complexity of the traffic.

There are several possible solutions to the notification of airspace going active that I could recommend:
Having the airspace appear on the radar scope 15 minutes prior to it actually going active. This could be displayed in either yellow or broken coral lines.
Having printed strips distributed to each sector as was previously the practice.
Having mandatory reminders from the Supervisor 1 hour, 30 minutes, and 5 minutes prior to the airspace going active.

There are currently sheets printed which lists all of the airspace that is going active over a long period of time. These sheets are rarely seen, and are very difficult to read as the organization is confusing and the printing is very small.

Narrative: 2

I was training a trainee on the Radar position and we had a Radar Assist trainee as well. We were extremely busy and all 4 people on the position were unaware that the military airspace was going active. Aircraft X departed requesting 20,000 feet. The trainee climbed him to 20,000 feet and the aircraft leveled. The MOA's airspace went active and Aircraft X was in the middle of it. I took over the frequency and asked Aircraft X if he could climb to 28,000 feet. He accepted and the Radar Assist coordinated the point out. I climbed Aircraft X to 28,000 feet to exit the airspace. 

The 1 hour warning message that comes out regarding the airspace is absolutely not enough. We were extremely busy and lost track of time. The supervisor should at least give us a 15 minute warning. We have been asking for a 15 minute pop up warning on the radar scope. We continue to get ignored over this matter and we continue to have airspace violations.

Synopsis

ATC reported allowing an aircraft to be routed through Military Special Use Airspace at the time it went active.

ACN: 1338177 (1598 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZKC.ARTCC
State Reference : KS
Altitude.MSL.Single Value : 34000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZKC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZKC

Person

Reference : 1
Location Of Person.Facility : ZKC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 31.0
ASRS Report Number.Accession Number : 1338177
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Staffing
Contributing Factors / Situations : Weather
Primary Problem : Environment - Non Weather Related

Narrative: 1

I ran to the sector to plug in and assist from the tracker position. After assuming the seated position, I started highlighting call signs of aircraft that were on frequency. About 4 minutes into that, I heard from across the room "watch those 34's" I asked which "34's" and saw [Aircraft X] 2 seconds before the conflict alert activated with [Aircraft Y]. I suggested a 30 degree left turn and descent to 32,000 feet to get [Aircraft Y] away from [Aircraft X], with another aircraft passing off the right of [Aircraft Y] at 33,000 feet. Radar controller turned Aircraft Y right to heading 340 to try to put [Aircraft Y] behind the [Following aircraft]. With wind out of the south, [Aircraft Y] picked up some speed. By the naked eye, I thought [Aircraft Y] never entered the 5 mile polygon fully indicating lost separation. 

The sector was already saturated to the point of unworkable by the time I sat down to track. I was informed that Traffic Management Unit (TMU) for aircraft to be routed around ARTCC airspace as we had moderate to severe turbulence reported at numerous altitudes, and severe thunderstorms. It appeared that no aircraft were routed around, and the aircraft deviating for the weather all ended up in the sector at the same time. I was too busy to see how many aircraft were on frequency at the same time, but with EVERY aircraft asking about the rides and constantly asking for different altitudes, it would have been impossible to work by anybody! Couple this non-routing around us with the fact that the routes that 90% of the aircraft were on were wrong. A lot of fixing was involved and very little preplanning by TMU, System Command Center, and sectors east of us. It is a miracle that no aircraft had RA's during this time. 

Had I been working radar at this sector, before it got busy, I would have reached out to other sectors and have them route traffic north of the sector airspace. This would have alleviated all the choke points and numerous crossing traffic at the same altitudes. If they didn't want to comply, they would have held aircraft until they complied.

When System Command Center is asked to reroute aircraft around the airspace for weather and sever or worse turbulence, they should really do it! Controllers make it work. TMU and System Command Center should suggests helpful solutions instead of turning controllers down. Who has final say at the sector? Controllers. Heed what controllers say.

Synopsis

ZKC ARTCC sector requested an aircraft be routed around its airspace due to weather and traffic saturation deviating for the weather. The request was denied. An aircraft was issued a vector to avoid another aircraft but the vector was not enough to maintain required lateral separation.

ACN: 1337493 (1599 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DTW.Airport
State Reference : MI
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Snow
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1337493
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Airport

Narrative: 1

Upon arriving at the gate there were no personnel to guide us in. The taxi inline was covered in snow. After a few minutes rampers appeared in tugs and on foot. The ramp was slippery as indicated by a ramper falling down after slipping. The tugs were sliding as well. We waited a few more minutes to be marshaled in. Finally the marshals showed up and we proceeded into the gate indicating 1 knot on the ground speed readout on the Navigation Display (ND). I was purposely very cautious on the taxi in. We were given the stop signal as normally presented and set the brakes. The aircraft continued to slide forward even though the brakes were set. The residual thrust at idle was enough to move the aircraft on the ramp under these conditions. The aircraft was not going to hit anything or anyone but I was helpless at this point. I indicated to the marshaller to get the chocks in. He didn't have any!!! I turned on the yellow pump and decided to shut down the engines in the hopes the loss of the residual thrust would help. It did. The aircraft stopped sliding. What a helpless feeling.

Do I go into reverse and blow equipment and people around? We thought the best thing to do was just shut it down. The rampers were quick to open the cargo door creating another obstacle. During the walk around the First Officer (FO) verified that the brakes were set and all the landing gears were plowing the snow in front of them instead of rolling over it. 

The event occurred because the ramp was in no condition to receive this aircraft. We couldn't tell anything of its surface condition. Chocks should have been at the gate instead of waiting for them to show up from another gate. I could have at least gotten the chocks installed quicker at the gate. We were lucky that nothing was touched or damaged. Fortunately the jetway was very far away from its normal position.

In the future I recommend a cursory check of the gate arrival area prior to an arriving aircraft given the fact that the snow was falling during this time. I feel the rampers were not prepared to receive us and we should have been left way off the gate till they were truly ready with chocks in hand. After the event occurred and we were stopped I later wondered if stopping and shutting down way off the gate and being towed in would have been a better idea. After the FO came back up after the post flight and described the ramp conditions (he fell himself on the ramp walking very diligently) even a tow in would not have worked. Could have been worse. In the future I will not follow the marshallers instructions to taxi in when we recognize a similar situation. I would shut it down or go to a gate prepared for our arrival.

Synopsis

An A320 Captain encountered extremely slippery ramp conditions and a lack of preparation by the ground crew during gate arrival at DTW. When the stop signal is received the brakes are set, but the aircraft continues to move forward due to slippery conditions and residual thrust. The engines are quickly shut down and the aircraft stops sliding.

ACN: 1337257 (1600 of 2908)

Time / Day

Date : 201603
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 1300

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : B737-900
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Facility : SFO.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 16
ASRS Report Number.Accession Number : 1337257
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : Weather / Turbulence
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Aircraft X was in-trail of a B737-900 who had just gone around. About a 2.5 mile final, I observed Aircraft X climbing out of 1200 feet and they stated they were going around for a wind shear alert on board the aircraft. I gave standard missed approach procedure and shipped them to departure control. I advised the next three or four arrivals and the single departure I had about the report and all others stated the wind was calm and they had no encounter. 

When Aircraft X came back around, I explained to them that they were behind a B737-900 that had gone around, and I suspected that they had encountered wake turbulence from this aircraft, as we have heard the newer generation of aircraft can have quite a nasty wake turbulence generated from the efficient wing. They acknowledged and landing having encountered no wind at all on final.

I could only recommend that when I advised them the preceding arrival went around, I would have reiterated that it was a B737-900, however, this may have meant not a thing to the pilot upon receipt. I just don't know what information they are being provided regarding aircraft wake anymore.

Synopsis

An aircraft initiated a go-around due to a wind shear alert in the cockpit. The reporter thinks it may have been Wake Turbulence generated by a preceding B737-900.