ACN: 1336441 (1601 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1201-1800

Place

Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1336441

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

A320 entered gate and came to a stop. We started to unload both FWD and AFT cargo compartments. Ramp serviceman noticed the nose gear beginning to rise. We immediately stop all FWD unloading and increased our unloading of the AFT compartment. The nose gear rose a total of 1/2 inch off the ground during this time.

Synopsis

While unloading bags from an A320, ramp crew noticed that the nose of the aircraft started to rise. They stopped unloading from the forward cargo bay until the aft bay was unloaded.

ACN: 1336389 (1602 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 8500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : D01
Make Model Name : Small Aircraft
Operating Under FAR Part : Part 91
Airspace.Class B : DEN

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1336389
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 337
ASRS Report Number.Accession Number : 1335759
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While on a dogleg to final for a visual to 35L in DEN we had a climb RA when descending through 8500 feet to our cleared altitude of 7000 feet. The First Officer (FO) executed the escape maneuver and we notified ATC we were responding to a climbing RA. ATC had previously called VFR traffic climbing to close to our position. TCAS target showed at our 2 o'clock position at a relative altitude of -500 feet and less than a mile when we responded to the RA. Vertical climb required was in excess of 3500 fpm. We climbed to approximately 10000 feet to clear the conflict. We stabilized, queried ATC, and conducted a visual approach to 35L. Jumpseater was the only one of us to acquire the target visually, a [small aircraft]. 

On the ground I contacted the Approach control supervisor. He stated that there had been a departure from Front Range (TFG) that was supposed to climb to an altitude below the Class B and were investigating. The supervisor later called back and related that the aircraft had violated the Class B by over 800 feet and the resulting proximity of his aircraft to ours was quite close.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported responding to an RA on a visual approach into DEN when a small aircraft entered Class B airspace without authorization.

ACN: 1335676 (1603 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Airspace.Class B : BWI

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1335676
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Flight was uneventful until turning final for Runway 33L at BWI. ATC tried to put a 737 in front of us and asked us to slow to final speed. We quickly configured for a configured 3 landing and the automation was actively being used. I noticed the airspeed slowing below the assigned value followed shortly by a "Speed, Speed" aural alert. I elected to perform a soft go-around but failed to follow the SOP after cycling the power to Takeoff Go Around (TOGA) and back to CLIMB. I asked the First Officer (FO) to get a turn back to the approach. During the acceleration, I discovered that the speed brakes were deployed causing the "speed" alert. I stowed the speed brakes and began the assigned turn and a climb to 3,000. There was confusion about the altitude and the direction of the turn between the crew and the controller. We agreed it was a left turn the long way around back to the final approach course and coordinated a new altitude assignment. I was hand flying with the aircraft still in config 3 and with the flight director commanding an opposite turn. We rebuilt the automation and landed without further incident.

ATC should not have attempted to squeeze the 737 inside of us. That created unnecessary pressure and would not likely have worked even without pilot induced error. 

I failed to monitor the spoilers. 

I executed a go-around but did not complete the procedure per the SOP.

I climbed to 3,000 feet in VMC but without proper coordination.

I failed to reestablish the automation in a timely and methodical manner.

ATC prioritized incorrectly by asking questions about why we went around prior to issuing a clear and unambiguous clearance.

The FO and I continued to fly the plane, monitor the path, and coordinate with ATC and limited the event to a minor deviation of SOPs and did not let the error become something worse.

Synopsis

An A320 Captain reported executing a go-around after receiving a "Speed" aural alert related to spoiler extension that was not noticed.

ACN: 1335659 (1604 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class D : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1335659
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Coming in to land [at] ZZZ. I was the PF and called for flaps full, bird off set runway track. I was turning base to final to land at an altitude of approximately 1500 feet. During the checklist reading as I was coming to my inside scan at the same time the Captain read auto thrust I looked down and noticed it was at thrust idle. I then pushed the thrust levers forward to TOGA and lowered the nose as the aural warning said "speed". The airplane slowed to around 125 knots, but quickly recovered to normal speed. Continued the approach without incident. Upon debrief on the ground at the gate the captain said that he accidentally turned his FD off and then back on which causes the airplane to go into thrust idle, when only one FD is on. Preventative measure could have been trying to configure earlier so as to not task-saturate the pilot monitoring.

Synopsis

A320 First Officer reported getting slow on approach when thrust levers were driven to idle by aircraft system logic.

ACN: 1333186 (1605 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 123
Experience.Flight Crew.Type : 6238
ASRS Report Number.Accession Number : 1333186

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

While arriving at [the gate] the distance remaining indicator was not operating properly and showed flashing green line when well within the arrival area. All indicators were correct. Aircraft was stopped and taxi light flashed 3 times at ramp crew. No one approached the aircraft with a head set. I would not use the Accupark as the distance remaining was inaccurate. The ramp crew was arguing amongst themselves and finally we were marshaled into the gate. The Accupark system was reported inoperative, with reason to operations on their dedicated frequency.

About 1 hour later I called operations to see what the outcome of the inoperative system was. No one knew that the system was inoperative!

Problem area....
1. Lack of ramp crew awareness of aircraft to ramp signals.
2. Operations dismissing inputs from aircrew. This will end up causing and injury, damage, or death if inoperative systems are continued to be utilized after being reported as malfunctioning.

I should not be required to check on completion of task when reporting malfunctioning systems. The reporting, and corrective checks and balances used by operations are flawed and dangerous.

Synopsis

A320 Captain reported problems with the Accupark system at the gate.

ACN: 1331217 (1606 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Climb
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1331217
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

I arrived for my 4th of 4 days. The plane had 4 MELs to begin with. I'll focus on two.

After reviewing the MEL, there were several people questioning whether we could even apply this specific MEL. The MOD list needed to be referenced and after that was complete it came down to the word "OR" as the aircraft had one MOD installed but not the other. After 40 minutes of discussion, the [release] was modified. If you also include the passenger medical issue requiring a paramedic sign off, you can see it was a very busy morning already. 

After successful engine start, we performed the first control check (not required by the MEL) and observed only "pair 3" locked out. Per the sign off, the aircraft needed both sets disabled. We needed to leave the ramp for company to push. With all the GA traffic the only place to go was the end of the runway. After a quick review of the MEL, for either a paperwork swap to one pair inop or for two pair inop, we needed a MX. We verified this with Maintenance; :25 later (can't remember exactly) we parked at the gate. Just to take a second here. The system was "MEL'ed" on the 4th and flew. We're the spoilers correctly disconnected? If so, did they trouble shoot overnight and not re-comply? These two items I don't have the info for but I do know for a fact I was given the plane with a non-compliant MEL procedure. 

Company manager met us and inquired about the situation. I gave him a quick airbus systems dissertation on why we were back at the gate. MX quickly met us and took the logbook to examine the work and why we weren't in compliance. Within :10 or so they were back and another :03 the new procedure was apparently completed. "Wow I thought that was quick". Looking at the F/CTL page we now had 3/4 spoilers "X-ed" out. Even though it did not call for the next step, I insisted we do a control check prior to push. They cleared the outside and the "visible cockpit indications" showed correct deflections. We buttoned up and pushed back. Maybe an hour late? This is the SECOND TIME we were apparently MEL compliant. I can't speak for what the other crew saw but as for us, everything we saw on the instruments and paperwork said we were in compliance. 

My FO (First Officer) was flying the first leg. It was windy for our departure. Rotation was normal but the coordination and turning were sloppy, probably due to both turbulence and 1/2 of the roll spoilers gone. 

As we accelerated thru 12,000 ft and the ride calmed down thru the upper teens, we started to notice the LH (Left Hand) wing start to indicate a slight wing down indication on the PFD. As we climbed further and transitioned to Mach, the indication continued. We changed to the #1 DFGC (Digital Flight Guidance Computer); at first there appeared to be some improvement but that later mimicked #2. We disconnected the autopilot and manually trimmed the plane out with the rudder. We couldn't fully right the wings. Once we leveled at FL360, we reengaged #2 and watched the PFD/F/CTL indications. 2-3deg LH turn on the PFD, LH AIL (Aileron) outside the cruise ticks, RH (Right Hand) AIL normal. Rudder showing a 1.7deg R deflection. Hmmm. Why?

We started communication via ACARS with dispatch and MX. Over the next hour the problem continued to progress to a greater offset and more rudder correction. We tried again to disconnect and manually fly but this time there was no correction to be had. We couldn't level the wings. VERY uneasy feeling in a FBW (Fly by Wire) plane that has no aileron trim installed

At our TOD (Top of Descent), the first step down, it actually felt like the plane was in a mild "slip". Not indicated as much but we were definitely not tracking straight. We took a conservative descent approach. The headwinds helped get us down in time. We vectored into the pattern eventually on the downwind. Every time we turned (especially to the left) we experienced a very noticeable uncoordinated slip all the way thru the turn. These factors continued to worsen until we passed flaps 2. Then all indications flopped. Rudder inputs remained but opposite aileron and spoiler indications. 

At 600 ft we have LH AIL deflected 50-60% down, RH AIL deflected 30-40% up. ALL REMAINING spoilers on RH wing extended normal rudder indications but we were in a slip down the GS. We increased speed and flew a shallow approach as we had 11,000' ft + to land on. It was a positive touchdown in light winds and VFR conditions. We used more braking than reverse for this landing; the rollout was normal. I felt VERY uneasy and out of control while operating this airplane that I have trusted now for 2 1/2 years. 

When we arrived at the gate, a mechanic (as I requested) met the plane. As I began to enter as much data into the logbook, the FO sends me a picture of the LH wing. THE LOCKED OUT #4 SPOILER IS DEFLECTED 10-15deg. This is impossible I said to myself! It's locked out electrically via SEC1 and the MX procedure should have had the existing hydraulic pressure holding it in place. We've all read the articles about spoiler float and it can present in different ways; poor rotation or extended TO (Takeoff) run, poor handling, rumbling, etc. of which the handling was assumed, initially, to be the missing spoilers.

After a very thorough search by our "unexperienced airbus on call MX", we discovered the panel WAS NEVER LOCKED OUT. ONLY TWO OF THE THREE STEPS WERE FOLLOWED. This was the 3rd miss by our maintenance team. I talked in great length with [Maintenance Operations], 3 different dispatchers, the duty pilot and my crew about the situation and how we felt the plane may be unsafe to fly. I think what I was looking for was a 100% ". Yes this was a spoiler float event and not electrically influenced." We never received such a statement. For a large portion of the delay I was convinced I was not moving the plane anywhere. We eventually came to an agreement after several control checks, inspecting the spoilers myself and factoring in all the other "pros" we had (long runway, VFR, light winds, only 142k) that we would take the plane. We planned on engaging the autopilot very early in the flight and if the wing dipped, even a bit, we were going to declare an emergency, apply the penalty and land overweight. 

We loaded after an extensive delay. During Eng #2 start, we experienced an IGN (Ignition) B failure that needed to be deferred. I'll just mention here this system has been deferred more than just once. I reluctantly say more than 10 because that may not be factual but if someone was looking at the history, I'd bet I'm not far off. I know they have replaced several parts and continue to troubleshoot but the #2 motors' initial power applications lag, and normal starts have taken 1:40. All significantly different than others but that's another story. At times this aircraft has carried 8+ MELs at one time. This is a great barrier to completing on time, reliable and predictable operations for our customers. 

During this delay, we needed to amend the release for fuel burn AND extend :30 to complete our mission. At no time did the company or safety or the duty pilot or whoever just ask how we were. Look at this chain of events? It's NOT supposed to be like this. I hope I caught everything I needed to because we were definitely doing the job of SEVERAL departments this day. This was easily one of the top 5 most challenging days I've had here. 

Disappointment and major frustration don't begin to explain my feelings surrounding this event. The possibility of an aircraft accident was created by incomplete maintenance procedures, and the lack of possibly parts, tools, or personnel to complete either the deferral or a repair. This is supposed to be a fairly routine procedure. This incident indicates a need for properly trained maintenance technicians and some sort of cross check being implemented for these aircraft.

It highlights the need for readdressing how this company perceives its maintenance function. While maintenance may have failed to accomplish a maintenance procedure, other links in the chain held fast. A properly trained crew was able to safely fly a compromised airframe, without further damage. I hope that I have been able to convey the importance of this incident. This event points out how dangerously thin this company's resources are apparently being pulled. It's also apparent that the schedule pressure is obviously elevated at the expense of a thorough procedure.

Synopsis

A MEL'd spoiler was not locked out and floated open during flight.

ACN: 1331101 (1607 of 2908)

Time / Day

Date : 201601

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Mixed
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1331101

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1331101

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At approximately 90KIAS [on takeoff roll] we received an ECAM Caution for: ENG 1 COMPRESSOR VANE. We did not notice any other abnormalities or unusual indications from ENG 1. We aborted the takeoff and returned to the gate.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported they rejected the takeoff after receiving an ECAM for ENG 1 COMPRESSOR VANE.

ACN: 1330968 (1608 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pitot/Static Ice System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1330968
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At the start of the event, the First Officer (FO) was the flying pilot, and autopilot 1 was engaged due to autopilot 2 being MEL'd. Earlier in the day the aircraft had IR#1 (ADIRU) deferred, along with several associated items, and the aircraft was refused by multiple crews. However, just prior to our flight the Air Data Inertial Reference unit (ADIRU) was signed off cleared from the Maintenance log.

We were level at FL320 and had just received a clearance to climb to FL360 with Center. We were IMC and in light to moderate chop. We went to "thrust climb open climb" and initiated a climb. As the engines went to climb power we started a slow climb. I noticed the airspeed on my PFD begin to slowly increase. As we climbed, it appeared the aircraft began to accelerate past a normal climb speed of about Mach .78. Over the next several seconds I made a few of calls the FO to "watch the airspeed" which were acknowledged and corrections made. According to my airspeed tape the aircraft was now nearing MMO and the speed was still slowly accelerating. At about this moment, the FO and I realized there was a discrepancy in our indicated Mach. Due to [the fact] that we were IMC, in turbulence, low vertical speed (we were heavy and engine anti ice was on), and that the trust was at climb power, it took us several seconds to decipher what speed indications were correct and what action we should take. An ECAM Caution "NAV - IAS Discrepancy" was alerted.

A few seconds later, the high speed CRC warning sounded, Master Warning flashed, and red ECAM appeared. At this time either the high speed protection logic or the First Officer disconnected the autopilot and the First Officer began manually flying the airplane. The First Officer leveled the airplane and we immediately requested a descent on our Center frequency. We got no reply. The request was repeated and again no reply.

The First Officer and I agreed to start a very shallow descent as we began to make sense of the situation. Both a lower altitude (because it widens the margin between high-speed and low-speed regimes) and a lower attitude (because it allows you to fly more "pitch & power") was the best option for us. Several more radio calls were immediately made to Center to advise of our climb clearance deviation and to notify them of our problem - still no reply from ATC.

The First Officer and I agreed that the Captain's airspeed, the CRC over speed warning, and Master Warning were all related, but inaccurate, indications. We followed the ECAM and switched the Captain's Air Data to #3 on the Switching Panel.

Over the next 30 seconds or so, several transmissions on both the Center frequency and 121.5 were made to advise ATC that we had descended back down. We leveled back at FL320 as indications returned to normal and the continuous over-speed chime stopped.

After another minute or two, we were finally able to reestablish communication with ATC and explain why we had descended and leveled back at our previously assigned altitude of FL320. 

[Manual] procedure was followed and ADR 1 was turned off. The flight continued to our intended point of landing.

In debriefing the situation, I believe the First Officer and I did an excellent job assessing the situation. An immediate reaction to a high airspeed indication, Master Warning, and CRC is to reduce thrust to idle and pitch the nose up to bleed excess speed. If the crew would have not recognized other indications by cross checking all flight instruments, it is likely we would have ended up in an Alpha Floor (low speed / stall recovery situation) which would have likely caused larger issues.

It is possible that the Captain's Pitot Tube had also become blocked, essentially making my airspeed an altimeter where it indicating airspeed increased with a gain in altitude. This is how the flight instruments were acting. This same scenario had a fatal outcome for [another air carrier].

Synopsis

A320 Captain reported receiving erroneous airspeed indications climbing to FL360 in IMC, possibly as a result of pitot icing.

ACN: 1330960 (1609 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Aircraft : 3

Reference : Z
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1330960
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

On this event we were call sign Air Carrier XXY, we were cruising at 37000 feet. We were on ARTCC frequency. Due to the late hour we requested to be rerouted. We were told to contact [next] ARTCC.

We checked in with the controller and made our request for the reroute. The controller then cleared us. The controller finished the transmission calling us Air Carrier XYY and stated to maintain 36000 feet. We read back the clearance and emphasized our call sign. We also said "Descending from 37000 feet to maintain 36000 feet." The controller gave no further response and did not make any correction. We entered the reroute into the MCDU and descended to 36000 feet.

A few minutes later we were told to contact [previous] ARTCC which was the previous frequency we were on. Just as we were about to switch, Air Carrier XYY checked in on frequency and the controller began getting confused and asked if they were proceeding to [fix]. They seemed also confused and said they didn't request any changes. At that time we started wondering about our clearance. We asked the controller if they were certain they wanted us to switch to ARTCC and she responded with an affirmative.

When we checked in with ARTCC I asked the controller to verify that we were on the right frequency and that we had indeed received the clearance to [fix] and [fix]. The controller returned a minute or two later saying that we were okay. We followed Air Carrier XYY the rest of the way into the airport and when we tuned into the last TRACON frequency we were also joined by Air Carrier XZY bringing the total of 3 company planes with similar call signs on the same frequency. 

The last controller on ARTCC asked that I inform the company to change the flight numbers and that it is unacceptable to have so many call signs so close together heading to the same place on the same type of plane at the same time. We were on time so unless the other flights were delayed it appears her concern is valid. 

At no prior point was Aircraft Y on the same frequency with us and we never suspected any issues prior to this moment. We also did not receive any warning from ARTCC that there was similar call signs on the frequency. The Center Controller suggest that we have vastly different call signs with something else to distinguish us better. I also suggest we stop filing similar call signs during the same time periods.

Synopsis

On a particular route to an airport aircraft of the same company with similar call sign numbers arrive simultaneously causing confusion.

ACN: 1330690 (1610 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 148
ASRS Report Number.Accession Number : 1330690
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During extended taxi in, I called for right engine shut down just after required cooling time. FO acknowledged and announced generator switch. I confirmed I was off brakes and steering, and FO switched electrics but did NOT immediately shut down the engine. This was due to ground controller calling at that precise time with instructions to taxi to a holding pad. There was unusually heavy ground traffic. I failed to cross check the engine instruments to verify that the engine was shut down as requested, and due to many distractions from the subsequent ground instructions and ground traffic, neither of us noticed the error until I parked at the gate. Since I did not have the APU running for parking, I waited for ground power to be plugged in before calling for the left engine to be shut down. It was only then that we both realized the right engine was still running, as well.

Synopsis

A320 Captain discovered at the gate that the right engine had not been shut down by the First Officer during taxi in due to distraction with ATC communications.

ACN: 1330424 (1611 of 2908)

Time / Day

Date : 201602

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 144.47
Experience.Flight Crew.Type : 4702.18
ASRS Report Number.Accession Number : 1330424
Human Factors : Fatigue

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This is a Fatigue report filed because I called Fatigue on day three of a three day trip and did not complete the last two legs.

I'll mention the first night [Day 0] at the [hotel] (even though it sometimes appears that really not much weight is given to accumulating sleep deficits) only because the first night of this trip I was already set back in my sleep. We arrived late on Friday night at the [hotel] and I was assigned a midlevel room on the east side of the building. I was awake until about XA:30 as the sounds of people and their motorcycles and cars were quite loud from the street as the bars closed and let out. I slept maybe five hours this first night, but was still fit for duty on day two.

On the second night [Day 1] of this trip, we were at [different] Hotel. It pains me to write anything derogatory about this hotel because it is one of the finest in our system. On Saturday night, however, the annual Winter Ball was at the hotel - and the hotel assigned us rooms in the out building, which is an older building that is not part of the main hotel property. The receptionist said, "I'm sorry, we don't have rooms for you here in this building tonight. You'll be across the street." Immediately outside my door was the elevator doors, and I heard people coming and going until the wee hours of the morning. Worse, right outside my window was a frat house - complete with an outdoor terrace. There was a large frat party this night that went until XA:00, complete with young girls screaming, and the sounds of furniture being tossed out the windows, I kid you not. See the pictures. So I had elevator noise through my door and a great frat party making noise outside my window. I got to sleep at around XA:00 and woke at XE:00 for our pickup. I was facing a 12 hours day on four hours of sleep. Not gonna happen. I did the first leg and called fatigued.

Synopsis

A Captain called off a trip fatigued on the third day after two nights of sleep deficit at hotels where late loud noise interfered with rest opportunities.

ACN: 1330416 (1612 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11215
Experience.Flight Crew.Last 90 Days : 152
Experience.Flight Crew.Type : 649
ASRS Report Number.Accession Number : 1330416

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 27975
Experience.Flight Crew.Last 90 Days : 213
Experience.Flight Crew.Type : 11070
ASRS Report Number.Accession Number : 1330429
Human Factors : Situational Awareness

Events

Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Airport
Primary Problem : Airport

Narrative: 1

Landed 27R, exited runway C1 onto Zulu Taxiway. As we made the turn further down the taxiway the aircraft continued to go straight and the nose wheel started to shudder abruptly. Antiskid felt engaged. The aircraft continued to slide towards the left side of the taxiway. Aircraft was sideways with the nose pointed towards the center line sliding sideways. As the Captain regained control and recovered from the slide the aircraft started to slide in the opposite direction towards the right side of the taxiway. As we regained control and were back on centerline I made a PA to the passengers to let them know what happened and that everything was under control and to not be alarmed. I then made a report to ATC (North Ground) to report nil braking action in that turn. We were not Taxiing fast (10kts max 15kts) and had briefed the possibility of slick runways and taxiways. The field condition report did not indicate any issues. The taxiway appeared wet but other than that no significant contamination was noticed. There were several fresh tire makes on the pavement in at least 2-3 of the turns. I reported the braking action as poor in two more turns and Ground seemed not particularly concerned. Pulling into the gate I did a brief walk around and checked the left engine and main gear for damage and found no damage.

Narrative: 2

After exiting 27R, and making the first right turn on Zulu taxi way as I turned the aircraft continued to go straight and the nose wheel started to shudder. The aircraft continued to the left side of the taxiway and got sideways so that the nose was pointed towards the center line. In the deepest part of the skid the left engine crossed over the taxi line boundary. As I regained control I slowed even more and the aircraft skidded again as it returned to the center line. The FO (First Officer) made an announcement, and reported nil braking action in that turn. During the flight I had pulled up 2 apt condition reports and both showed the same report runway dry taxiway wet. I also got performance for 27R. And briefed the turn off of that we would be well slowed and under control leaving the runway. I believe I had the aircraft to below 10 knots. And the taxiway speed to be less than 15 knots. The taxiway appeared wet in the centerpiece area. The FO checked the left engine for damage at the gate and found nothing. I called the tower to see if we had hit lights, they said that they had not heard anything from the city. I also asked if they had had that area sanded and they said that the city would take care of it.

Synopsis

A320 flight crew experienced nil braking and momentary loss of control when turning on to Z Taxiway from C1 after landing Runway 27R at ORD. The runway was dry, but the taxiway was wet with no braking action problems reported.

ACN: 1330358 (1613 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Function.Ground Personnel : Airport Personnel
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1330358
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Aircraft : Automation Overrode Flight Crew

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I arrived for my 4th of 4 days, XA:20 show to operate me scheduled flight. The plane had 4 MELs to begin with. The two I'll focus on are 27-XX-XX-Y. 

After reviewing the MEL, there were several people questioning whether we could even apply this specific MEL. The Modification (MOD) list needed to be referenced and after that was complete it came down to the word "OR" as the aircraft had one MOD installed but not the other. After 40 minutes of discussion, the TLR was modified. If you also include the passenger medical issue requiring a paramedic sign off, you can see it was a very busy morning already. 

After successful engine start, we performed the first control check (not required by the MEL) and observed only "pair 3" locked out. Per the sign off, the aircraft needed both sets disabled. We needed to leave the ramp for company to push. With all the GA traffic the only place to go was the end of the runway. After a quick review of the MEL, for either a paperwork swap to one pair inoperative or for two pair inoperative, we needed a Mechanic (MX). We verified this with Maintenance Control; :25 later (can't remember exactly) we parked at the gate. Just to take a second here ... The system was "MELed" on the 4th and flew. We're the spoilers correctly disconnected? If so, did they trouble shoot overnight and not re-comply? These two items I don't have the information for but I do know for a fact I was given the plane with a non-compliant MEL procedure. 

The flight Ops assistant met us and inquired about the situation. I gave him a quick airbus systems dissertation on why we were back at the gate. MX quickly met us and took the logbook to examine the work and why we weren't in compliance. Within: 10 or so they were back and another: 03 the new procedure was apparently completed. "Wow I thought that was quick". Looking at the Flight Control (F/CTL) page we now had 3/4 spoilers "X-ed" out. Even though it did not call for the next step, I insisted we do a control check prior to push. They cleared the outside and the "visible cockpit indications" showed correct deflections. We buttoned up and pushed back; check Automatic Information System (AIS) for times. Maybe an hour late? This is the SECOND TIME we were apparently MEL compliant. I can't speak for what the other crew saw but as for us, everything we saw on the instruments and paperwork said we were compliant. 

My First Officer (FO) was flying the first leg. It was windy for our departure; rotation was normal but the coordination and turning were sloppy. Probably due to both turbulence and 1/2 of the roll spoilers gone. 

As we accelerated thru 12,000' and the ride calmed down thru the upper teens, we started to notice the Left Hand (LH) wing started to indicate a slight wing down indication on the Primary Flight Display (PFD). As we climbed further and transitioned to Mach, the indication continued. We changed to the #1 DFGC; at first there appeared to be some improvement but that later mimicked #2. We disconnected the autopilot and manually trimmed the plane out with the rudder. We couldn't fully right the wings. Once we leveled at FL360, we reengaged #2 and watched the PFD/F/CTL indications. 2-3deg LH turn on the PFD, LH AIL outside the cruise ticks, RH AIL normal. Rudder showing a 1.7deg R deflection. 
We started communication via ACARS with dispatch and MX. Over the next hour the problem continued to progress to a greater offset and more rudder correction. We tried again to disconnect and manually fly but this time there was no correction to be had. We couldn't level the wings. VERY uneasy feeling in a FBW plane that has no aileron trim installed

At our Time of Departure (TOD), the first step down, it actually felt like the plane was in a mild "slip". Not indicated as much but we were definitely not tracking straight. We took a conservative descent approach. The headwinds helped get us down in time. We vectored into the pattern eventually on the downwind. Every time we turned (especially to the left) we experienced a very noticeable uncoordinated slip all the way thru the turn. These factors continued to worsen until we passed flaps 2. Then all indications flopped. Rudder inputs remained but opposite aileron and spoiler indications. 
At 600' we has LH AIL deflected 50-60% down, RH AIL deflected 30-40% up. ALL REMAINING spoilers on RH wing extended normal rudder indications but we were in a slip down the GS. We increased speed and flew a shallow approach as we had 11,000'+ to land on. It was a positive touchdown in light winds and VFR conditions. We used more braking than reverse for this landing; the rollout was normal. I felt VERY uneasy and out of control while operating this airplane that I have trusted now for 2 1/2 years.

When we arrived at the gate, a mechanic (as I requested) met the plane. As I began to enter as much data into the logbook, the FO sends me a picture of the LH wing ... THE LOCKED OUT #4 SPOILER IS DEFLECTED 10-15deg. This is impossible I said to myself! It's locked out electrically via SEC1 and the MX procedure should have had the existing hydraulic pressure holding it in place. We've all read the articles about spoiler float and it can present in different ways; poor rotation or extended TO run, poor handling, rumbling, etc. of which the handling was assumed, initially, to be the missing spoilers.

After a very thorough search by our "unexperienced airbus on call MX" (who worked at night on the line), we discovered the panel WAS NEVER LOCKED OUT. ONLY TWO OF THE THREE STEPS WERE FOLLOWED. This was the 3rd miss by our maintenance team. I talked in great length with 3 different dispatchers, one in training, the duty pilot and my crew about the situation and how we felt the plane may be unsafe to fly. I think what I was looking for was a 100% "... Yes this was a spoiler float event and not electrically influenced ..." We would never receive such a statement. For a large portion of the delay I was convinced I was not moving the plane anywhere. We eventually came to an agreement after several control checks, inspecting the spoilers myself and factoring in all the other "pros" we had (long runway, VFR, light winds, only 142k) that we would take the plane. We planned on engaging the autopilot very early in the flight and if the wing dipped, even a bit, we were going to declare an emergency, apply the penalty and land overweight. 

We loaded after an extensive delay. During Engine #2 start, we experienced an IGN B failure that needed to be deferred. I'll just mention here this system has been deferred more than just once. I reluctantly say more than 10 because that may not be factual but if someone was looking at the history, I'd bet I'm not far off. I know they have replaced several parts and continue to troubleshoot but the #2 motors' initial power applications lag, and normal starts have taken 1:40. All significantly different than others but that's another story ... At times this aircraft has carried 8+ MELs at one time. This (and apparently sister ships which are both at out departure airport) are a great barrier to completing on time, reliable and predictable operations for our customers. 

During this delay, we needed to amend the release for fuel burn AND extend: 30 to complete our mission. At no time did the company or safety or the duty pilot or whoever just ask how we were. Look at this chain of events? It's NOT supposed to be like this. I hope I caught everything I needed to because we were definitely doing the job of SEVERAL departments this day. This was easily one of the top 5 most challenging days I've had here in all my years. 
The possibility of an aircraft accident was created by incomplete maintenance procedures, and the lack of possibly parts, tools, or personnel to complete either the deferral of a repair. This is supposed to be a fairly routine procedure. This incident indicates a need for properly trained maintenance technicians and some sort of cross check being implemented for these aircraft. It highlights the need for readdressing how this company perceives its maintenance function. While maintenance may have failed to accomplish a maintenance procedure, other links in the chain held fast. A properly trained crew was able to safely fly a compromised airframe, without further damage. I hope that I have been able to convey the importance of this incident. This event points out how dangerously thin this company's resources are apparently being pulled. It's also apparent that the schedule pressured is obviously elevated at the expense of a thorough procedure. Our [company] does not enjoy the luxury of any mistake!
Multiple mechanics doing every MEL procedure. Some system of cross checks MUST be implemented by maintenance.

Synopsis

A320 Captain reported MEL procedures were not complied with completely causing irregular flight characteristics.

ACN: 1330126 (1614 of 2908)

Time / Day

Date : 201602

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 84
Experience.Flight Crew.Type : 4981
ASRS Report Number.Accession Number : 1330126

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At approximately 80 knots "hot brakes" with associated master caution illuminated. I rejected the takeoff at approximately 85 knots. After clearing the runway we accomplished the rejected takeoff checklist and associated hot brakes checklist. We contacted Dispatch and talked to local maintenance on the radio. We described the situation on the radio, told them the peak brake temp and our location, holding in the pad by the runway. They told us no inspection was required unless the temp exceeded 900.

Synopsis

A320 Captain reported rejecting the takeoff at approximately 85 kts when he received a master caution for hot brakes.

ACN: 1329750 (1615 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Manufacturer : International Aero Engines
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1329750

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1329644

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[Our aircraft] had been in maintenance all day. I performed all of my preflight duties, reviewing the logbook and completing my flight deck prep flow. The log book was clear. Soon afterwards the F/O joined me, completed the walk around and we completed our pre departure procedures. All appropriate checklists were completed and we pushed off of the gate. I started both engines to give them sufficient time to warm up (5 minutes minimum-first flight of the day) and all parameters were normal. We planned a Config. 1 departure (flaps 1) with an Assumed Takeoff power setting. We taxied out, completed the taxi checklist and waited to depart. Approximately 8 minutes after engine start we finished the taxi checklist "below the line" and were cleared onto the runway with a "line up and wait."

Tower cleared us for takeoff with an initial heading after departure of 230 degrees. I applied initial power to 1.05 EPR and let the engines stabilize followed by advancement to the FLEX detent. I maintained my right hand on the thrust levers. Very shortly after the application of takeoff/flex power we received an ECAM amber message that stated on the lower ECAM: ENG 2 COMPRESSOR VANE and immediately below it an ECAM action message: ENG 2 THRUST LEVER IDLE. The F/O called my attention to the ECAM. At this point I retarded both thrust levers and stated, "Reject, My Aircraft" as per reject procedures. I selected maximum reverse and the RTO braking occurred and I noted a speed that was very close to 80 kts and rapidly decreasing. The First Officer radioed the tower that we were rejecting the takeoff, again per policy. Almost as soon as the F/O finished his radio transmission the aircraft came to a stop, I noted a burnt rubber smell (I'm sure now it was a result of our rapid deceleration) and I wanted to assess if we had any additional engine issues including fire. We told the passengers to remain seated while I continued to assess the situation. There was nothing else new on the ECAM however, now that the ENG 2 Thrust Lever was in IDLE the ECAM stated in Amber: AVOID RAPID THRUST CHANGES on one line and below that THRUST LEVER IDLE.

I pressed the brake pedals to release the RTO brakes and taxied to the first available taxiway. The burnt rubber smell quickly dissipated. We cleared the runways. We stopped, reviewed, and completed all ECAM and QRH procedures, noted that the brakes were not even hot, spoke with our company to determine a gate, and once all procedures and protocols were completed, we taxied to our original departure gate. Once at the gate a Customer Service Agent met the aircraft and informed the passengers that there was a new aircraft a few gates down. After writing up the engine situation and the RTO, completing all checklists, we left the aircraft and proceeded to the new airplane and gate. 

I can only surmise that the right engine had a problem with the correct configuration of its moveable compressor vanes. In speaking with a mechanic once we had arrived, I was told that the IAE engines have had issues with this problem. It was the first actual RTO for me in my nearly 30 year career and was startling when I decided to reject the takeoff. Nevertheless, training and aircraft systems provided a rapid conclusion to the event. It was a low speed event as the F/O never stated "80 Knots" during the takeoff roll and this ECAM would have been suppressed by Airbus logic above 80 knots.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported rejecting the takeoff below 80 kts when they received an ECAM caution message "ENG 2 COMPRESSOR VANE."

ACN: 1329350 (1616 of 2908)

Time / Day

Date : 201602
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Indicating and Warning - Hydraulics
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 172
Experience.Flight Crew.Type : 6258
ASRS Report Number.Accession Number : 1329350
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

We were at FL320 and got a HYD B LO PR ECAM with no [Blue Advisory] items but got the chime. Since we were stabilized at cruise altitude and I was already the PF I continued to fly and took the radios as the FO began to search the Quick Reference Handbook (QRH) for the procedure. As I followed him through the index we both soon realized that there was no such procedure listed in the QRH. We then checked the Flight Manual (FM) and found the same - no procedure for the Electronic Centralized Aircraft Monitoring (ECAM) we showed. I then had the FO send a Dispatch CALL ME so we could talk to Maintenance Control (MC). The Airbus controller spoke as I monitored at low volume as to maintain ATC watch. The controller was "dumbfounded" as to why there was no direct procedure as he had us check the status of the Electric Blue pump (had no fault light indication), HYD page (amber arrow and amber "BLUE" shown), BLUE system pressure was 2850 PSI, and F/CTL page (amber B indications). He finally decided that it would be prudent for us to run the HYD B ELEC PUMP LO PR (QRH 130.1) as it was the closest procedure - which we accomplished. After fully following 130.1 we signed off with MC and Dispatch as he could not think of any other options to work from. Upon clearing the pages - when the FO arrived at the Status page the first step showed to turn the BLUE ELEC pump back to auto for approach which was NOT part of the 130.1 procedure. We once again contacted MC for clarification and he recommended doing this - just in case the pump "failure" was erroneous and we re-established Blue system pressure for approach and landing. 

After leaving him once again, within two minutes (approx) we received a F/CTL ELEC1 FAULT and followed the QRH procedure which called for a reset of ELEC 1. When we turned it off the aircraft began a slight (less than 5 degree) roll to the right and stayed that way throughout the remainder of the flight. We reset Elevator/Aileron Computer (ELAC) 1 after 5 seconds to no avail - as the message remained. I elected not to call MC for a third time as we believed this to be a related failure (which was in fact MC's opinion as well after we called to debrief them after landing). After finishing all procedures and calls I elected to call the Purser to inform her that the aircraft was flying normally but we did have a hydraulic issue. I asked her to review procedures for evacuation with her partners just in case the situation deteriorated and gave her an estimated time until landing (TEST). As we quickly approached the Top of Descent I began to quickly brief the RNAV arrival to transition to a visual approach. In our haste, the FO and I failed to confirm this in the Multipurpose Control Display Units (MCDU), and had the ILS in instead. I didn't catch it until shortly after the final fix and had him quickly enter the FMS visual procedure - we were slightly SW of course and corrected back to the FMS course expeditiously. I then asked the FO to turn the BLUE ELEC pump back on and the aircraft "jerked" momentarily, which signified to me that in fact there was some pressure provided of course - although the 5 degree right bank continued. I elected NOT to declare an emergency based on the GREEN and YELLOW systems being normal, a "return" of BLUE system, no fault lights or overheats noted on any HYD pumps or systems, and positive aircraft control. By the way, I had been hand flying (auto thrust still on) since around 12,000 feet as to get a feel for aircraft control relatively early. Again, except for the bank the aircraft flew relatively normally - under the circumstances. I also made sure to configure early as we were expecting slow slat movement as per QRH 130.1. We were soon cleared for the approach, configured the aircraft, stabilized the approach early, and landed normally. Upon reaching the Gate and completing all checklists I then called Maintenance Control via Dispatch to debrief him fully. By the time we landed MC had already sent email(s) to various departments documenting the issue(s) and said they would follow up with us if need be. Lastly I would be remiss if I failed to mention the exemplary work displayed by my New Hire FO. All crewmembers and MC displayed true skill and professionalism which ultimately contributed to a safe outcome.

Synopsis

A320 Captain reported a "B" Hydraulic System Low Pressure warning appeared on ECAM. There was no procedure in the QRH to resolve the issue. Pressure indicated normal, but the aircraft tended to roll to the right for the remainder of the flight. They reset MCDU 1, to no avail. Turning the hydraulic pump back on the aircraft "jerked" indicating pressure, but the tendency to roll remained.

ACN: 1328793 (1617 of 2908)

Time / Day

Date : 201601

Environment

Light : Night

Aircraft

Reference : X
Make Model Name : A320
Crew Size.Number Of Crew : 2

Component

Aircraft Component : Strobe Light
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Other / Unknown
ASRS Report Number.Accession Number : 1328793
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Strobes are unusable at night because they flash in the cockpit causing vertigo and disorientation..
Please force Airbus to fix this
Action taken: Turned off strobes

Synopsis

A320 pilot reported he shut off the strobes because the flashing at night causes vertigo and disorientation.

ACN: 1328465 (1618 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SAN.Airport
State Reference : CA
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Cloudy
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : SAN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Flight Phase : Final Approach
Airspace.Class B : SAN

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 165
Experience.Flight Crew.Type : 423
ASRS Report Number.Accession Number : 1328465
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 610
ASRS Report Number.Accession Number : 1328444
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Diverted
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Normal enroute operation. Planned LOC 27 approach. Visibility reported well above minimums. Executed managed/selected method. At DH (Published + 50), no runway lights, no approach environment, no approach lights. Executed normal missed approach. Climbed to 5000 MSL, and was vectored east parallel to Runway 27, on right downwind. Was handed off to SAN Tower. Made positive contact with SAN Tower. Waited for Tower to turn us right base to intercept LOC 27. Call never came. About 25 miles east of SAN, received GPWS Warning "Pull up, terrain, Pull up, Terrain". Executed escape maneuver to 7000 MSL. PM advised SAN of maneuver. Tower issued: "Immediate right turn heading 180. Maintain 7000". 

Crew jointly determined insufficient fuel remained for another LOC 27 approach, followed by a miss and divert to planned alternate. LAX was determined to be best divert airport based on weather, distance and fuel burn, and facilities. Dispatch was advised and concurred. Minimum fuel was declared. Clearance to LAX was obtained, and flight, approach, and landing were normal. Fuel remaining was approximately 40 minutes. Ground turn took 2x than it should due to FA crew desk delaying flight due to their scheduling failure and the need to swap FAs. Return flight to SAN was normal and without incident.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

Flight crew reported a GPWS warning while on the downwind leg at SAN, expecting a turn to base by ATC. During the subsequent ATC vectors for a second approach, the crew determined the fuel state insufficient for a second approach, and diverted to LAX.

ACN: 1328461 (1619 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : RNO.Airport
State Reference : NV
Altitude.MSL.Single Value : 9100

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : WADOL2
Airspace.Class C : RNO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 147
Experience.Flight Crew.Type : 9652
ASRS Report Number.Accession Number : 1328461
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 86
Experience.Flight Crew.Type : 481
ASRS Report Number.Accession Number : 1328706
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were on the WADOL2 STAR arrival into RNO. We were assigned 9,100 by approach control. We were about 5 miles south of HHOPE Intersection when we received a GPWS Terrain warning "Terrain, Terrain, Pull Up". We had no indications of terrain on the ND Display. We complied with the terrain escape maneuver until warning went away, which was almost immediately. We advised ATC of our terrain warning as we leveled off around 10,300. Approach control then told us to descend to 10,000 feet and gave us a right turn (base leg) for ILS 34L. Made normal approach and landing.

Narrative: 2

We were in night IMC lower than some terrain in the area, so the reaction was required.

Synopsis

A320 flight crew reported receiving a terrain warning on a night IMC arrival into RNO.

ACN: 1328458 (1620 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 188
Experience.Flight Crew.Type : 637
ASRS Report Number.Accession Number : 1328458

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 213
Experience.Flight Crew.Type : 1220
ASRS Report Number.Accession Number : 1328490

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through FL290 slight smell of electrical fire fumes. Called Flight Attendants (FAs) and they reported no smell. Fumes intensified very rapidly. Donned O2 masks, [advised] center, coordinated immediate emergency descent/turn to ZZZ. Finished running Smoke/fumes checklist. Noticed Vent Blower and Exhaust ECAMS, ran those checklists. Contacted FAs giving them the TEST briefing and told them to prepare the cabin for evacuation. On descent fumes began to diminish, removed 02 masks, and informed FAs. Continued for normal landing, taxi to gate.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 crew during climb to altitude diverted due to electrical fumes. Avionics vent blower and exhaust ECAM indicated that system was malfunctioning. Checklist run, fumes diminished during descent. Crew continued for normal landing.

ACN: 1328051 (1621 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1328051
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Excursion : Runway
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While landing in EWR LOC 22R the Captain landed on the displaced threshold area prior to the 22R threshold of the runway. At 200 feet I told him he could not land in the displaced threshold area, and he ignored my statement. I repeated myself several times explaining this was prior to the landing area of 22R. He ignored me and continued his approach and landing on 22R. He landed approximately 100 feet before the threshold of 22R, ignoring the displaced threshold markings. He asked what I was saying on short approach and when I explained the displaced threshold, he blew me off stating haven't you ever landed on that before.

The Captain had total disregard for his FO, and the rules about landing in a displaced threshold area.

Synopsis

A320 FO reported the Captain landed on the displaced threshold of Runway 22R at EWR even after the FO warned against that.

ACN: 1328018 (1622 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9023
Experience.Flight Crew.Last 90 Days : 177
Experience.Flight Crew.Type : 5054
ASRS Report Number.Accession Number : 1328018
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Prior to departure as we completed the before start checklist, noticed the fuel value (23K) on the Fuel Service Form (FSF) was 1,000 less than on the most current release 1 (24K) for our planned gate. ACARS sent to Dispatch to confirm we were still on release 1. Response back was yes, and he did not lower our fuel or trim ferry fuel. After calling to speak with Load Planning (LP) supervisor I discovered LP did lower the fuel based on preventing bumping bags or people. Concern is that LP adjusted the amount of ferry fuel without coordinating with Captain or Dispatcher. Although I realize LP has the authority to lower Ferry fuel, I believe they reduced it more than permissible by reducing it to 21,800 + 437 for taxi = 22,237 planned gate. Substantially lower than the 24.0 that the Dispatch and I agreed on. According to FOM, LP cannot reduce Ferry below 75 minutes endurance. They effectively trimmed our ferry by 1,763 LBS which was substantially more than the 12 minutes they are permitted. We did coordinate with Dispatch before takeoff to ensure we had a correct release.

Synopsis

A320 Captain reported discovering during the Before Start Checklist that the fuel load was 1,000 pounds less than their release. Research revealed that Load Planning made the change on their own to accommodate more passengers and cargo.

ACN: 1326783 (1623 of 2908)

Time / Day

Date : 201601

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1326783

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Phase 1 (Sent on landing in ZZZ they sent a supervisor to meet with us. Sprayed lavatory spray to mask smell, said we were good to go. Crew said we didn't think it was good idea to take this plane but they gave us the go ahead).

This plane has been written up multiple times due to flooding in the back AC right lavatory (days ago). They cannot find the leak. The real problem is that it has flooded the carpet and made it a health issue. Everything nasty from the bathroom is being trapped in the carpet. This is a major health issue as it's flowed into the galleys where there's food. Passengers and crew are breathing in this stale air which is a health and safety issue. My sinuses were going crazy on takeoff and finally started to clear up as I was luckily working first class. Mechanics said it was not leaking though the carpet was wet and soiled. (Once again it was written up before today). If it leaves on the next leg it will not be fixed until at least tomorrow. This needs to be addressed before it becomes a bigger health issue. 

Phase 2 (Sent after landing in ZZZZ).

I know they mentioned the sink not shutting off, but the pilots said it had flooded before. Which, I believe as you don't get that kind of a smell from a first time leak. We reported the flight had made the flight attendants (all of us) feel sick on the way to ZZZ. They said they would shut off the water. We explained we didn't think that would help as the smell was coming from the soiled carpet. The solution they came up within ZZZ was just spraying lavatory spray on it. This actually made it worse. We then had to fly the plane to ZZZZ and once again the smell gave the crew headaches, sore throats, stuffy noses, itchy eyes (much worse than on the first leg). We explained this wasn't going to get fixed in ZZZZ. Now we are going to have to work this plane tomorrow. That's three times making us feel ill. Tomorrow will most likely be the worst as it will have been sitting overnight. We left early out of ZZZ because they didn't want to find us another plane and got into ZZZZ 45 minutes early. This needs to be fixed as it's a major health concern not only for the crew but the passengers as well. As we all know health and safety should always come first.

Phase 3 on this Plane. (Supervisors stated they would have cleaners and others in ZZZZ try to get rid of the smell on aircraft before we arrived to take flight back next morning).

They Contacted ZZZZ officials don't know what they did (if anything). Flight was better due ZZZZ crew or the soiled carpet drying overnight. Crew was better off this leg.

Synopsis

A320 Flight Attendant reported feeling ill when the aircraft was operated over several segments with the carpet in the aft lavatory soaked from flooding.

ACN: 1325351 (1624 of 2908)

Time / Day

Date : 201601

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aircraft Cooling System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person : Company
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1325351
Human Factors : Confusion
Human Factors : Communication Breakdown
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

The MEL card in hand indicated for the Avionics Fault deferral "both packs must operate normally". The key word being normally. In the responders text below he indicates a class 2 maintenance message is a "faults detected by the system" that do not affect dispatch. Or as stated in the Flight Manual "failure within a system". I completely agree with the stand alone definitions of the class 2 maintenance message but tie that same definition "failure/fault within a system" with the MEL cards mandate of "both packs must operate normally" and there I have an issue. The definition states "failure/fault" and the MEL cards says operate normally. Again I would have a difficult time explaining to a FAA lawyer how a system definition of "failure/fault" can coexist in the same system mandating "operate normally".

I disagree with the responder's text in that he said the two MEL's are not in conflict with each other yet his own definition of a class 2 message states;

These are faults detected by the systems and which DO NOT have any operational consequences (safety aspects) on the CURRENT flight or on the FOLLOWING flight(s) but which may have consequences if a second fault occurs. These faults are indicated to the cockpit crew on ground by a Maintenance STATUS on the ECAM STATUS page after the engines shut down. 

Original report follows below:

Notice the responder states "if a second fault occurs", thus there had to be an initial fault. Again I have an issue with explaining the initial fault/failure associated with "must operate normally" because there is no way of knowing if that initial fault still existed at time of dispatch, or even what the cause of it was.

Second, pilot pushing. It is my understanding that this person has agreed that during aircraft refusals Captains will be asked the following questions.
1.Are you ok?
2.Is there anything I can do to help?
As this refusal played out there was much more coercion from the company than what was agreed to by this person. That issue was not specifically addressed. I hope that it is being addressed behind the scenes.
The original report and response follow for reference.

We were scheduled to fly this airplane with the following four deferrals. 
1. Aft cargo heat deferred 
2. Center tank fuel pump deferred 
3. Avionics system fault (ASF) MEL 21XXX 
4. Outboard flap rub strip

I arrived at the cockpit to find a mechanic performing the needed tests for the Avionics System Fault (ASF) deferral. I sat in my seat and started my preflight. While performing his tests we got a class 2 maintenance message "pack 2". I assumed the mechanic would fix that issue since it directly related to our ASF deferral. He left without saying anything. After a long delay and it appeared maintenance was not going to work that issue so we wrote it up. 
The same mechanic came back with a deferral sticker in his hand and deferred the class 2 message. I said that was there when you left and I assumed you would fix it. He said "yea I saw it", but I assumed from his actions he chose not to work it. I found it somewhat disturbing that in performing his required test a class 2 message came up, he saw it but did not to work it.

In referencing MEL for the ASF it says under Flight Crew "both packs must operate normally". I asked the mechanic who was deferring the class 2 maintenance message what was wrong with pack 2. He said "I don't know". I asked him how can I take the airplane with a class 2 maintenance message for pack 2 in conjunction with MEL that specifically stated "both packs must operate normally" if no one can tell me what is wrong, if anything with pack 2. He said "it's deferrable". I disagreed and refused the plane. 

I phoned dispatch, told them of the refusal and why I was refusing the aircraft. The dispatcher immediately phoned Maintenance Control (MC) and put him on the line. MC said this was a legal deferral. I asked how it could be if MEL said "both packs must operate normally" and I had a class 2 maintenance message for pack 2 and nobody knew what was wrong with pack 2 causing the message. MC said it doesn't make a difference class 2 messages could be for many things and we don't know what is causing this message but it's not associated with MEL. I again referenced the statement "both packs must operate normally" and said if you can clear the class 2 message OR tell me what is causing the class 2 message I will take the plane if the class 2 message in fact confirms the pack 2 is operating normally. He said he could not again reaffirming his stance it was a legal deferral. I reiterated my aircraft refusal and the conversation ended. 

The next three or four phone calls were from the Flight Operations Manager asking why I refused the plane and again reaffirming that it was a legal refusal. I gave him my opinion as stated to MC above and the Flight Operations Manager still thought it was a legal deferral. I spoke to another mechanic who was on the flight deck working the issue and he said on the original deferral for the ASF MEL maintenance should have changed a "box" which they did not do thus making the deferral incomplete and that they would have to change the "box" to make the deferral complete. I phoned the Flight Operations Manager and told him about that conversation. 

Later the Flight Operations Manager called back saying the mechanic was incorrect and the deferral was correct. I told the Flight Operations Manager I had 3 witnesses to the conversation with the mechanic and I did not misunderstand him. Again the Flight Operations Manager asked if I was willing to fly the plane now, I said no for the stated reason. 

The plane was subsequently taken out of service. I spoke to another mechanic (different than any of the others I had spoken to) and asked him about the "box" issue and he confirmed what the other mechanic had said that the box was not changed and they were taking the plane out of service.

If the deferral was legal, as the company claims, then why did they take the plane out of service? If it was legal then how does one explain MEL "both packs must operate normally" while simultaneously I had a class 2 maintenance message for pack 2 and nobody could tell me what was wrong with pack 2? The Flight Manual states in reference to class 2 maintenance messages "Failure within a system that usually does not affect the flight". That statement from the Flight Manual says "usually", in this instance wherein the MEL says "both packs must operate normally" and the Flight Manual starts its description of class 2 maintenance message with "Failure within a system", how can that be a legal deferral? Nobody was willing or able to tell me what caused the class 2 maintenance message. 

I kept telling the Flight Operations Manager that if someone could tell me what caused the message so a more informed decision could be made or clear the message I would take the aircraft. Neither happened and the plane went to the hanger. 

This is an alarming trend that I see continuing over the last few years. The company has become very persuasive in attempting to get pilots to fly airplanes the company deems safe yet the Captain, for whatever reason, does not. Gone are the days of the company asking if they can help in anyway and here are the days of the company strong arming pilots into taking airplanes deemed unsafe or illegal. This is not my first report on the same subject, I see the alarming trend continuing and worry that illegal and unsafe planes are being flown because of the strong arming of the company only for the on-time and completion rates. 

When Captains refuse airplanes the company should stop handing us over to the Flight Operations Manager and MC and we listen while they try to change our minds. MC's job should be to fix the issue not try and coerce pilots into flying airplanes deemed unsafe. The Flight Operations Manager should be there to help and support the Captain and facilitate getting the plane fixed not spend time trying to get the Captain to fly an airplane he/she has already refused. With over 31 years of airline flying the degradation of Captains authority via pilot pushing has never been as bad as the last two to three years at this company. I hope this report not only brings light to my refusal stated above but to the overall continuing trend of pilot pushing by the management.

Action Item Response
This is the Definition of a class 2 fault according to Airbus and is contained in AMM 31-32-00-0001. (b) Class 2 faults. 

These are faults detected by the systems and which DO NOT have any operational consequences (safety aspects) on the CURRENT flight or on the FOLLOWING flight(s) but which may have consequences if a second fault occurs. 

These faults are indicated to the cockpit crew on ground by a Maintenance STATUS on the ECAM STATUS page after the engines shut down. 

System faults that result only in messages on the CFDS (centralized fault display system) do not affect dispatch and do not require action other than addressed within the company's maintenance program. 

So the 2 MEL's were not in conflict with each other. The original MEL was for an Avionics system fault. The reason the MEL states that both packs must operate normally is due to equipment cooling requirements and at the time of departure BOTH packs were operating normally. 

The 2nd MEL was for the class 2 MAINTENANCE fault, which by the definition has no operational consequences on the current flight and the MEL does not require any maintenance action be taken on the initial deferral. 

The Aircraft (AC) was not taken out of service due to the MELs on the AC. It was taken out of service so that a plane change could be facilitated due to the crew refusal.

Callback: 1

The Reporter stated that he feels when the MEL states that both packs must operate normally if the Skin Inlet Valve is deferred and there is a Class 2 Maintenance associated with a pack, that pack is not operating normal. 
The Reporter also stated that he has discussed this with various other pilots and everyone agrees that it is not Normal Operation if a Class 2 Maintenance is associated with a pack. 
The Reporter stated that there seems to be a trend of pilot pushing by company management that is behind the degradation of Captains authority in these matters.

Synopsis

An A320 pilot reported confusion with an MEL regarding if an Air Conditioning Pack was operational when a Class II Maintenance Message "Pack 1 or 2" was displayed on the Centralized Fault Display System.

ACN: 1325157 (1625 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 741
ASRS Report Number.Accession Number : 1325157
Human Factors : Time Pressure
Human Factors : Fatigue
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We were on the second duty day of a trip. The pick up at our hotel was at XA:50 local time. We flew [the] flight prior to ZZZ. 

On the second leg, we had a HYD YELLOW RSVR LO ECAM right after takeoff. The Yellow system hydraulic fluid was completely lost and empty. We performed the ECAM and the checklist. Although I wanted to return, after consulting the Company and my First Officer (FO) (who had recently experienced the same problem) they all recommended that we continue (3+ hours left in the flight). According to them, that recommendation was made based on: the remaining system was operating normally; QRH does not call for "land(ing) at the nearest suitable"; and QRH is "controlling". Although I felt somewhat uncomfortable continuing the flight with the loss of a major hydraulic system, especially on a dark night, I agreed with the recommendation to continue. Soon into the cruise, I reviewed both the "Yellow & Green HYD System Loss" checklists, just in case we also lost the Green system, which I thought, would put us in a challenging situation for sure. Plus, I was still feeling that we were in a state where "A critical system redundancy is lost." So, I reestablished the radio conference. After a careful review, they now all agreed with me that we should divert instead of continuing. We diverted to ZZZ1. The flight landed overweight and completed the loss of Accumulator pressure on approach. 

We were not even at the gate when the station was calling us on the radio to see if we would agree to continue the flight to ZZZ2. Immediately after completing the flight, both a mechanic and station personnel came on board asking about continuing the flight. I had not even had a chance to talk with Dispatch or Scheduling, when people started bombarding me with questions. I asked them to give me a chance to talk with the company. I next had a talk with Maintenance, the Station Manager, Dispatch, and the Ops Manager. He asked me to waive the 30 min duty extension to continue the flight to ZZZ2. He told me the new plane would arrive in time for an XB:15 departure and I agreed to extend.

Soon after that conversation, I received information that the arrival of the plane would not be until after XB:04. I called back only the Operations Manager had already gone home. Now I was on the phone with another Operations manager who told us that he needed a 2 hour extension. I couldn't tell the passengers that we wouldn't be flying the flight just a few minutes after announcing the flight would continue based on the 30 minute extension. For the sake of our customers, we decided to press on and reluctantly agreed to extend. FO confirmed with me several times that the Crew Critical Off time (CCO) time was now XH:57 even though an ACARS message said XH:00. He "updated" the CCO and we received the revised ACARS message of XH:57. During the climb we received the ACARS message "Extension not available. Must return to the gate." Dispatch stated that we had exceeded the CCO time of XH:05 when we took off at XH:06. This CCO time of XH:05 was never addressed before. I asked what they wanted us to do at that point. Dispatch was consulting with the FAR 117 desk and Ops Manager. It took a while for them to respond and when they did, Dispatch informed us that if we expedited the taxi after landing in ZZZ1, we should be okay. I further asked Dispatch if we had in fact exceeded the CCO time or not. Dispatch said they were reviewing the information to find the answer. After some wait, the Ops Manager sent us an ACARS message saying we had not exceeded the FAA time but the company CCO time. I do not have full confidence in that explanation. If we were okay, then why had they sent us the "must return to the gate" message to begin with? The flight continued and landed normally in the very early hours of the morning.

Synopsis

The A320 lost the yellow hydraulic system and diverted, which put them in a confusing FAR 117 situation.

ACN: 1325107 (1626 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 7000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use.STAR : DYAMD TWO
Airspace.Class E : NCT

Component

Aircraft Component : Aero Charts
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1325107
Human Factors : Distraction
Human Factors : Workload
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Airspace Violation : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

DYAMD TWO ARRIVAL NOTAMed of a change of altitude restriction at ARCHI to 8,000 feet. We missed the change as shown on the paperwork. Paperwork should show NOTAMs in order of importance. Maybe show relevance with BOLD type.

Synopsis

An A320 crew missed the SFO DYAMD TWO Arrival ARCHI NOTAM changing the ARCHI waypoint crossing restriction to 8,000 feet from the published 7,000 feet. Crew descended below SFO Class B.

ACN: 1324556 (1627 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : DEN

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19838
Experience.Flight Crew.Last 90 Days : 142
Experience.Flight Crew.Type : 6402
ASRS Report Number.Accession Number : 1324556
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 125
Experience.Flight Crew.Type : 8502
ASRS Report Number.Accession Number : 1324564
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Automation : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were being vectored for an approach to Rwy 35R in DEN descending to 11,000 ft. We were asked if we had the airport in sight and cleared for the visual. The PNF set the MCP altitude to 7,000 ft. (FAF altitude). About 15 miles from the airport we were asked if we would like 35L. Both pilots saw the airport and we agreed to the Runway change. ATC did not give us an altitude restriction. We were then distracted by ATC giving us another vector to intercept to avoid traffic he cleared for the 35R approach. We discussed the terrain and obstructions before descending. Visibility was unrestricted. At about 7,500 ft. we were given a low altitude warning by ATC. We verified our altitude and continued to descend. A moment later we got a GPWS too low terrain warning and climbed to 8,000 ft. The approach and landing continued without further incident.

Narrative: 2

On approach into Denver, we were given a visual to Runway 17R [35]. I set in the outer marker altitude of 7,000 feet. During the descent, approach asked if we would accept a visual to 17L [35] and of course we accepted. As I was busy entering in the new runway I got distracted and approach called to "check altitude" as we were below the minimum vectoring altitude. It was then that I realize the first officer had descended to 7,000 feet approximately 10 miles early. Just as the First officer was leveling off we received a blurb of a "terrain" warning and we climbed back up to 8,000 ft. The rest of approach was normal. Having been flying into Denver for many years I know better. To rub it in and make me feel even worse was, the next morning as I was flight planning and waiting for dispatch to load flight plan I read the weekly bulletin that on the front page addressed descending too early on a visual approach.

Synopsis

A320 flight crew reported descending early during a visual approach to DEN resulting in a low altitude alert from Approach Control followed quickly by an EGPWS terrain warning. The flying First Officer climbed from 7,000 feet to 8,000 feet.

ACN: 1324271 (1628 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 19L
Flight Phase : Final Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Facility : NCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2.5
ASRS Report Number.Accession Number : 1324271
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12403
Experience.Flight Crew.Last 90 Days : 169
Experience.Flight Crew.Type : 11482
ASRS Report Number.Accession Number : 1324536
Human Factors : Human-Machine Interface
Human Factors : Workload

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 201
Experience.Flight Crew.Type : 10607
ASRS Report Number.Accession Number : 1324569
Human Factors : Human-Machine Interface
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

I was vectoring to final for the approach. Aircraft X was on a vector to intercept the localizer. I issued a crossing instruction for the final approach fix. This protects for the adjacent airport arrivals below them on their approach. Aircraft X read it back and I verified the read back. I was dealing with another aircraft for the approach and when I looked back Aircraft X was at 2000 feet at the Final Approach Fix. About 800 feet low. I then saw conflicting traffic on final (for the adjacent airport) at 1600 feet about a mile west of Aircraft X and eastbound. I asked Aircraft X if he was crossing the fix at 2800 feet and he responded there was a glideslope issue. I instructed them to climb and asked if he had the airport in sight. He said yes so I cleared him for a Visual Approach. The LOSS already happened and there was divergence now.

We had a similar situation last week with a descent prior to the final approach fix where they should have. So the new procedure came out yesterday to change how we clear aircraft. Before I always instructed aircraft to maintain an altitude until established on the localizer. Management instructed us to give a crossing restriction at the Final Approach Fix to fix this problem. I started doing that today which is not what I'm used to. I believe the pilots also were confused which led to this LOSS.

Narrative: 2

I was pilot flying on an ILS Approach. Received very late turn onto final and clearance for approach from Approach Control while descending through approximately 4000 feet. Clearance was Maintain 2800 until SHAKE, Cleared ILS approach. ILS Identifier WAS verified. Approximately 2-3 miles outside of SHAKE, Glideslope captured and descended on glideslope. We then got a gradual but FULL SCALE deflection showing us ABOVE glideslope. While cross checking the approach plate and the altitude of the preceding aircraft 5 miles ahead of us, it became apparent that something didn't make sense. The preceding aircraft altitude was almost the same as ours. Simultaneously, we got a call from Approach Control to State Altitude. We were at 2000 feet over SHAKE. At this time, we also got a FULL SCALE deflection below glideslope.

We were in VMC conditions with the preceding aircraft in sight. The glideslope finally stabilized throughout the remainder of the approach. We reported the issue to ATC. Throughout the Approach the ILS Frequency showed Identified. When we arrived at the gate, we talked to both the Tower and Approach control Supervisors. He explained the sequence of events. They mentioned that a similar event occurred with another aircraft not long after our event.

Narrative: 3

I was looking outside to keep an eye on the preceding aircraft, it was quite hazy. I then looked inside to see that we were at 2000 feet at the same time ATC warned of our altitude. By this time we had a full scale deflection of the glide slope telling us that we were above it which caused us to discuss what was going on with the Glide Slope. Pilot flying immediately stopped the descent and a few seconds later the Glide Slope indicator went full scale in the opposite direction (which was correct). I communicated with ATC what we were seeing with the Glide Slope. ATC indicated that he had not had any problems reported before us.

We maintained our altitude and recaptured the Glide Slope and continued on with no further problems. No doubt we got suckered into following the Glide Slope and not cross checking it with our distance from SHAKE intersection.

Synopsis

An aircraft on approach to SFO received incorrect glideslope information and descended below the glideslope and a crossing restriction into conflict with crossing traffic below it.

ACN: 1323454 (1629 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1323454

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6000
Experience.Flight Crew.Last 90 Days : 67
Experience.Flight Crew.Type : 850
ASRS Report Number.Accession Number : 1323558
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Encountered a seagull strike in left engine. Vibration noticed immediately after 120kts with buzzing sound from engine. Takeoff continued as no other abnormal engine indications. Capt stated "continue" at 140 kts. Rotated 145 kts. Normal climb out but noticeable vibration and noise. After 2000 feet checked engine ECAM and #1 N1 vibration showed 9.1. All other engine indications normal. FO stated he saw a bird on left side and believed we hit it. Captain did not see bird. Leveled off at 5000 feet and vibrations subsided with power reduction. Crew agreed a return to land was prudent. Notified ATC, FAs, and passengers. Arrival uneventful. Landing flawless by FO. MX inspection revealed damage to several fan blades but no apparent core damage. Feather samples taken. Crew re-assigned for next day duty.

Narrative: 2

On takeoff from ZZZZ, I noticed a single light tan bird gliding over the runway in front of the aircraft moving from right to left as we accelerated through approximately 90 kts. Shortly afterward, a loud buzzing began from the left engine. At this point we were already past 100 kts. At approximately 120 kts, the Captain verbalized "Continue." All indications on the engine were normal and I executed a normal T/O. We cleaned up the aircraft and the Captain worked to identify what had happened. Vibration on the #1 Engine was high and flashing green. At a level off, as the power decreased the vibration stopped flashing and the noise from the #1 engine decreased. As climb power was restored, the noise level increased and the vibration indication began to flash again. Captain coordinated with ATC to level off and return to ZZZZ. Power setting on the engines remained below climb power for the remainder of the flight and vibration did not flash again. Other engine indications were normal, we received no ECAMs and the plane flew w/o difficulty. We concluded a normal approach and landing. After shutdown, Mx identified major damage to the blades in the #1 engine.

Synopsis

A320 flight crew reported encountering a bird strike at high speed during the takeoff roll. The takeoff was continued, however a significant vibration in one engine was noted. They reduced power on the affected engine and safely returned to the departure airport, where major damage to the engine was discovered.

ACN: 1323189 (1630 of 2908)

Time / Day

Date : 201512
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZDV

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1323189
Human Factors : Situational Awareness
Human Factors : Other / Unknown
Analyst Callback : Attempted

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After entering and verifying our cleared route, through a strict enter and read back procedure, we departed for destination. Enroute we were given vectors off course for in-trail spacing to destination. Before being returned to our planned route we discovered that instead of KD45U being in the FMS (clearance waypoint), KD54U was entered instead. Also, another KD54U waypoint was used earlier in the flight plan. The waypoint was corrected and we continued to destination uneventfully after a thorough review of the remainder of the FMS flight plan.

This situation occurred despite a full route review and read back by both pilots prior to departure. These two waypoints are also close enough together to appear to be a valid routing when viewed on the display monitor.

I don't know any quick fix for this issue except continuous vigilance. It might be worth reviewing the waypoint numbering system to ensure similar waypoint names are not spaced so closely together. I'll bet this is not the first time this issue has reared its ugly head!

Synopsis

A320 First Officer reports discovering Enroute that NRS waypoint KD45U has been inadvertently entered into the FMC as KD54U. This is detected and corrected before a significant deviation results.

ACN: 1323068 (1631 of 2908)

Time / Day

Date : 201512
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Direct
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1323068
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After [so many] hours in the A320 I thought I had seen it all, but apparently not. This was the first flight of the day for this aircraft. The airplane had an overnight check and also an Auto Land test. When we first got in the cockpit it was not dark and on the init page of the Flight Management Guidance Envelope Computer (FMGC) it showed a flight plan of ZZZ/ZZZ [same airport]. We aligned the Inertial Reference System's (IRS) and put in the flight plan. We were filed to ZZZ1 using RNAV arrival which is a north flow arrival however we knew that ZZZ1 was landing south and put in the ILS XXR and we would get a different arrival when we got closer to ZZZ1. We had a "RW LS MISMATCH" message in the scratch pad at this point which we both assumed was because of the south runway with the north arrival. We flew to ZZZ1 and as expected we were changed to a different fix arrival. It was a beautiful VFR day and we were told to expect the visual XXR. Of course we expected to have the ILS XXR to give us supplemental guidance to the runway as it was tuned in the FMGC's and the Nav displays showed ILS XXR. As usual we were pretty high and tight when approach had us turn to a 140 heading to intercept final for the visual. The FO (who's leg it was) engaged the approach mode but the airplane started to turn left and the localizer display showed deviated to the right and that's when I noticed that the localizer ID on the Primary Flight Display (PFD) instead of the ILS XXR freq and identifier of ZZZ2. I went to the RAD NAV page and saw 109.9 in large font which meant that 109.9 had been hard tuned. Once I cleared it we got the proper signal and we were headed for the runway. Meanwhile in the confusion we had descended below the glide slope but told my FO to just hold altitude and we would head in and reintercept the Glide Slope and continue the landing so we got back on profile around 900'AGL and about 3 miles out and made sure we were configured and we went to flaps full and and completed the landing. My FO suggested we maybe should have gone around but having flown into ZZZ1 for [many] years I was not uncomfortable completing the approach since it was Day VFR and we were looking right at the runway. I am sure the FOQA people will be calling cause we changed a flap setting and got fully configured under 1000' AGL which I normally agree with but I couldn't see going around once we had the situation figured out and under control. Sometimes you just have to look out the window and turn the automation off and just fly the flipping airplane.

After we returned to [home airport] I visited Line Maintenance and asked them if they 'hard tuned' an unused ILS frequency when doing an Autoland check. They said yes but it should not be left hard tuned. Also looking at the [destination airport] area charts I noticed that although 109.9 is an unused frequency in the [departure airport] area it happens to be the ILS frequency for an airport near [the departure airport] so when the plane started to turn left when we thought we were intercepting the XXR localizer it was trying to go to the nearby airport. Had this been at night or poor visibility this could have gotten very interesting very fast. Thank goodness it was a beautiful day. I suppose the first clue we should have picked up on was the ZZZ/ZZZ flight plan and the second clue we missed was our misinterpreting the "RW LS MISMATCH" message. It had nothing to do with our selection of the fix arrival with the ILS XXR approach but rather it is telling you that you have the wrong ILS frequency tuned! A very interesting morning and it just proves that you can't be complacent even in beautiful, weather and thousands of hours.

Synopsis

An A320 pilot reported that when the destination airport ILS was selected, the frequency defaulted to a previously "Hard" tuned frequency. Once the Flight Crew cleared the "Hard" frequency they got the proper signal for the ILS. Later it was uncovered that Maintenance had manually entered a unused frequency in the ILS for an Autoland test and had not cleared it. This frequency turned out to be the ILS for an airport nearby the destination airport.

ACN: 1321695 (1632 of 2908)

Time / Day

Date : 201601
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8962
Experience.Flight Crew.Last 90 Days : 174
Experience.Flight Crew.Type : 4993
ASRS Report Number.Accession Number : 1321695
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Equipment / Tooling

Narrative: 1

Pulling into gate in IAH a truck drove directly thru the safety zone and the "Safe Park" system did not tell us to stop due to an obstruction. Recommendation for improvement- ensure the scanning of the safety zone is accurate and provides instantaneous feedback to the crew via the signage. Useful improvement (do not have to wait on marshallers), but system must be out of beta and ready for prime-time. This example illustrates why it is not yet. Had we followed the signage and continued our approach to the gate, it is possible we would have hit the vehicle. My attention must be singularly focused on the lights guiding me to gate, which means the FO must back up the Safe Park scanning of safety zone.

Synopsis

A320 Captain reported a truck drove into the safe zone while he was parking at a gate in IAH and the "Safe Park" system failed to warn him.

ACN: 1320488 (1633 of 2908)

Time / Day

Date : 201512
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Switch
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1320488
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Received ECAM warning for a yellow system reservoir overheat. Performed ECAM actions which resulted in yellow system deactivation. Because of degradation of a major system I [requested priority handling with ATC]. Proceeded to destination without incident.

Maintenance tech told me the one of the cargo door switches became stuck in the on position resulting in the yellow electric pump running continuously.

Synopsis

An Airbus pilot reported a hydraulic system malfunction which resulted in the deactivation of one of the hydraulic systems. Due to the degradation of some hydraulic system components priority handling was requested, however, the flight continued to destination and landed without incident.

ACN: 1318876 (1634 of 2908)

Time / Day

Date : 201512
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : A90.TRACON
State Reference : NH

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : A90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : BOS

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : A90
Aircraft Operator : Air Carrier
Make Model Name : Cessna 402/402C/B379 Businessliner/Utiliner
Operating Under FAR Part : Part 121
Flight Plan : VFR
Mission : Personal
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : A90

Person

Reference : 1
Location Of Person.Facility : A90.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 2.0
ASRS Report Number.Accession Number : 1318876
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft Y was in front of pack of planes for an ILS approach, Aircraft X was on downwind to base, Aircraft Z was straight in approach. Similar sounding call signs created a bit of confusion at the beginning. I had appropriate separation at the final approach fix with Aircraft Y and Aircraft X. I did however assign 190 knots to Aircraft X and forgot to relinquish the speed control, prior to transferring the aircraft to the tower. Aircraft Y slowed to a slower than normal speed inside the Final Approach Fix. The closest point was 1.81 miles, Tower was providing visual separation. The tower sent the Aircraft X around after ADSB-X alert. 

For an unknown reason the crew of Aircraft X advised the tower of a near mid-air situation. I am not aware of any aircraft in a close proximity to Aircraft X.

Synopsis

A90 TRACON Controller allowed spacing between two aircraft on final approach to decrease to less than required minimums.

ACN: 1318494 (1635 of 2908)

Time / Day

Date : 201512
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1318494
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During take-off roll, noticed a "red" message appear on ECAM. Could not immediately determine its significance, so I rejected the takeoff and returned to the gate. Flight was subsequently cancelled.

This was a mistake on my part. The message I had actually seen was a magenta "T.O. Inhibit" message, which is a normal take-off message which appears when the takeoff inhibit function of the ECAM activates. I guess I thought the message would be in green, and so seeing it in (almost) red, made me think there must be a problem. I have to admit, in the instant before I commenced the rejected takeoff, the A320 rejected takeoff accident in Philly a couple years ago flashed thru my mind. Given that background, I thought it would be better to reject early and get a handle on what I was seeing, rather than continue the takeoff and regret it at a higher speed or in the air.

When we got back to the gate, I called Dispatch and was put through to Maintenance. At this point, the situation went from bad to worse. I thought I would be able to get some more eyes on the problem and maybe get a resolution, or maybe someone would say, "Hey, the T.O. Inhibit is supposed to be magenta". But instead, they kept thinking "configuration warning", and the conversation veered off course a little.

The following morning, we went out and made a high speed taxi run to verify that the ECAM present was in fact, the correct one for the situation.

I think the environment also contributed to my confusion. It was dark, I was looking across the cockpit at the message while steering the aircraft down the runway and accelerating, and my eyes were straining to look thru one of three different lenses on my eyewear to read the message.

I guess my first suggestion, which of course, gets tossed right away, modify the message presentation on ECAM so (nearly) red messages don't flash up during takeoff! Why would they do that?! Airbus supposedly spent so much effort to create a "lights out" environment for takeoff, and then have an almost red message appear halfway through takeoff? That's just dumb.

My second suggestion, and this is just for me, is to be more specifically aware of exactly what will be presented at every phase of flight. I swear, in that moment, I thought the T.O. Inhibit would come up in green along with the other messages which were already present.

A couple months ago, I patted myself on the back for catching EGT indications more than 200 degrees apart, early on a takeoff run. Wasn't an ECAM, but clearly something amiss. I guess engine indications are where I tend to focus most of my inside attention during takeoff. Seeing a magenta ECAM caught me by surprise this time, although it presents on every takeoff.

Synopsis

A320 Captain reported unnecessarily rejecting the takeoff when the standard message "T.O. Inhibit" was displayed on the ECAM.

ACN: 1317945 (1636 of 2908)

Time / Day

Date : 201512
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 2800

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class C : OAK

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1317945
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Cleared direct to PARBB and maintain 4500. I slow to 210 KIAS, go to Flaps 1 and I maintain 4500 to PARBB. After PARBB we initially turn to follow track to ZOLUM and ATC says, direct to MITOE descend 4000. I open descend to 4000 and continue to slow to 200 because we are getting close. There's a stretch of uncomfortable silence from the controller as I feel that we need to further descend because I notice the "profile descent donut" traveling down my PFD, and then I say to the First Officer (FO) "give him a call, because I think he has forgotten about us, we need to descend." At that moment ATC says "descend to 2500." I immediately set 2500 and open descend. Then ATC ask "do you have the airport in sight" and we respond yes. ATC then says "cleared visual approach Runway 30." I set 2000 for MITOE altitude and continue open descent. 

Some time passes then I get "Terrain, Terrain," I initiate a pull up by applying back stick, auto pilot disconnects, then "Pull up, Pull up" is heard, and then silence. So, after a moment, I level the aircraft somewhere above 3000 feet. As I'm leveling and trying to understand what has happened, ATC says be careful of high terrain. FO says yes we noticed. Because this event unsettled me for the moment I tell the FO that I want to discontinue this approach, ask ATC for vectors. He settles me by saying "we are just above 3000' and still on course to MITOE with plenty of room to descend to 2000'. I say "OK," and re-engage the auto-pilot, and proceed to a Flaps 3 visual approach to Runway 30 OAK. Later, I ask the FO, "at what altitude did we get the "Terrain, Terrain." He responded at about 2800.

Possible reasons:
I had anticipated doing a full approach and following the approach and arrival profile. It was late, and it seemed as though we were the last aircraft flying into OAK. There were a few ATC silent moments. Seeing the descent profile donut travel down and out of sight of my PFD caused me to urgently descend. I used "open descent."

I had not fully anticipated the high terrain in the area, even though the ILS approach was briefed. Who would have thought high terrain on an approach to the California coast? I will be more vigilant. Request full approach.

Synopsis

A320 Captain reported receiving a GPWS terrain warning on a night visual approach to OAK when he lost situational and mode awareness descending in "open descent" mode.

ACN: 1313544 (1637 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Final Approach
Airspace.Class B : ORD

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class B : ORD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6403
Experience.Flight Crew.Last 90 Days : 207
Experience.Flight Crew.Type : 3949
ASRS Report Number.Accession Number : 1313544
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While executing a go-around, the Captain failed to reach the TOGA detent resulting in a "Too low, gear" warning. I directed the Captain to push up the throttles. The flaps were retracted to 3 as directed by procedure, then to 1. A climb to 2,000 ft. and turn to 220 degrees was given by ATC. Upon approaching 2,000 the aircraft began to accelerate. I told the Captain to come out of TOGA but he delayed in doing so, causing the flap over speed before I could retract flaps. ATC then issued climb instructions to 4,000 ft. In the confusion of the moment, the turn to 220 degrees was delayed and a traffic conflict arose. We were instructed to call the tower after landing.

Synopsis

A320 First Officer reported several speed and altitude deviations occurred during a go-around, resulting in an airborne conflict.

ACN: 1313222 (1638 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 4900

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use.STAR : TYSSN3
Airspace.Class B : LAS

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1313222
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1313218
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were approaching Las Vegas flying the TYSSN3 arrival. At approximately 30 miles out we were cleared direct to SHAND and descend 8000 ft. A short time later we were cleared for the visual approach to 25L. The auto pilot and auto thrust were engaged and both pilots had Terrain on ND selected ON. Estimating our path to be approaching SHAND on a 300 heading, we discussed flying over the obstacle noted to be 3700 ft on the approach plate and selected 4900 as the altitude for SHAND as published, ensuring we would have more than 1000 ft clearance above the posted obstacle prior to joining the localizer for 25L. We noted how the terrain on ND feature did not display any obstacle in our path. We observed the radar altimeter decreasing and expected that to be our approach to the 3700 ft obstacle. The radar altimeter began decreasing rapidly and we heard a "Terrain" warning quickly followed by a "Pull up". I immediately initiated a climb at TOGA thrust climbing approximately 1000-1500 ft on the same heading. As I recall, the warning sounded once and did not repeat. The FO advised ATC of our climb and subsequent level off and I believe I heard the controller mention the vectoring altitude in that area to be 5000 ft. We quickly reinstated the automation and joined the localizer for 25L. Extending gear and flaps early we were able to rejoin the published glide path and our approach was "stabilized" prior to 1000ft AGL so we continued to an otherwise normal landing. 

To be candid, I understand what triggers the terrain warning but I am uncertain why it triggered in this situation, as I thought we had accounted for the terrain in our approach path by selecting an altitude providing 1000 ft+ clearance. I would be speculating to suggest any other reason as I honestly am not sure. I can say with certainty that recent training on terrain events resulted in what we perceived to be an unquestioned and virtually instantaneous response to the warning once realized.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported receiving a terrain warning on the TYSSN3 Arrival into LAS.

ACN: 1313158 (1639 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 269
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 1313158
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

During initial cockpit setup procedures I noticed a somewhat strong acrid electrical burning smell. Lead FA confirmed the smell and I entered a MRM and issued a follow-up call to MX. Within 5-10 mins the smell had dissipated by the time MX arrived. MX signed off and released the aircraft. Pushback, start, taxi and right seat takeoff were all uneventful and normal.

At approximately 2000 AGL during the departure, we noticed the same strong acrid electrical burning smell, donned O2 Mask and established crew communications. Immediately following we observed the Blower Fault ECAM message. The FO selected AP2 and I accomplished the ECAM procedures. During the ECAM procedure the aft FA called saying "...we have strong fumes in the cabin". I [advised ATC] and requested immediate return to the departure airport. I gave the FO the aircraft and radios while I updated the FMC, coordinated the return with the FAs and made a cabin announcements. We entered a left base 4-5 miles outside the marker at approximately 220 kts. I transferred control from the FO and completed an uneventful stabilized approach and overweight landing. The FO made a cabin announcement for the passengers to remain in their seats, we contacted ground, the fire department, and taxied to the gate.

Synopsis

Flight crew experienced fumes in the cockpit during climbout followed by an equipment cooling alert. Crew elected to return to the departure airport. Landing was uneventful followed by taxi to the gate.

ACN: 1313060 (1640 of 2908)

Time / Day

Date : 201511

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Experience.Flight Crew.Last 90 Days : 137
Experience.Flight Crew.Type : 6097
ASRS Report Number.Accession Number : 1313060
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

I refused the aircraft for an inoperative APU based on enroute terrain considerations as well as forecasted weather conditions in ZZZ1 at the expected time of arrival. The safest course of action was to have an aircraft with an operative APU. At a minimum having either (preferably both) a backup pneumatic and/or electric component provided by the APU is always the safest course of action in any sense.

Synopsis

An A320 pilot refused an aircraft with an inoperative APU because of terrain and weather considerations.

ACN: 1312881 (1641 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 200

Environment

Flight Conditions : Mixed
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use.Other 
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Instructor
ASRS Report Number.Accession Number : 1312881
Human Factors : Training / Qualification

Events

Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

I was the flying pilot on the ILS Runway XXR at ZZZ. The winds had been reported as pretty gusty since we received our weather packet [at the departure airport.]

Last spring, on the same approach to the same runway with similar wind conditions I ran out of throw on the stick while landing and had to do a go-around. On our second attempt we got a wind shear alert and went around again, so this time I was ready for what I anticipated to be a difficult approach and landing.

At approximately 200 feet AGL I encountered a gust from the left (winds were 240 at 13 gusting to 21 KTS). I sharply countered the gust using left stick, however I overcorrected and the plane rolled approximately 10 [degrees] to the left. I countered that roll with right stick, but also over corrected so the plane entered a similar bank angle to the right. I corrected for that with left stick and at that point realized I was encountering a pilot induced oscillation so I then centered the stick to get out of the oscillation. During the oscillation the spoilers had extended apparently causing us to get a bit low on profile.

The Captain said something like "you're getting low." At that point I noticed one white light and three red lights on the PAPI so I decreased my sink and said "correcting." The landing was fine and uneventful. After blocking-in I talked with the Captain about the approach and my oscillation. He suggested that I hand fly the airplane more during gusty weather and disconnect the autopilot earlier during the approach (I disconnected it at minimums.) I plan on following this advice to help get a better feel for the winds effects on the plane.

I think that because of my previous experience on that runway with gusty winds I was "over-primed" to make corrections that in this case were too much.

Synopsis

A320 First Officer landing during gusty crosswind conditions encountered pilot induced oscillations. Captain recommended hand flying the aircraft more during gusty weather and disconnecting the autopilot earlier during approach.

ACN: 1312559 (1642 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Aircraft Auto Temperature System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1312559
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We had just departed on runway 26R. We had cleaned up the airplane and were climbing when when we received an ECAM caution for "VENT SKIN VALVE FAULT". I was pilot flying while the Captain was pilot monitoring. [He] reacted quickly and began the SOP for non normal ops. including immediate action/ECAM exception determinations and designating flying duties. I then called for ECAM actions. [The Captain] performed the correct ECAM actions for the skin valve fault which included verifying which valve was affected. In our case it was the extract valve. Both valves were showing closed but the extract valve was in amber. [He] proceeded to place both the blower and extract to OVRD per the ECAM procedure. While [the Captain] was running the ECAM I was given a higher altitude from [approach] (in the low flt levels I believe...) and after conferring with [the Captain], requested to stop our climb at a lower altitude. I explained that we were working out an issue and needed to evaluate the situation before continuing higher. I believe we were given an altitude of 12,000 ft but do not remember exactly. [The Captain] continued with the ECAM placing both blower and extract to OVRD did not get rid of the caution message or the extract valve in amber. The ECAM then directs manual depressurization of the cabin. I remember thinking that it was odd to depressurize a cabin that seemed to be pressurizing normally. I wanted to suggest holding off on that part of the ECAM until we could fully ascertain what was happening. In the end, I decided against putting in my two cents. [The Captain] was running the ECAM and seemed to have things under control and running things by the books which was a good thing. I had my own duties as flying pilot in busy airspace to try and help troubleshoot the problem. [The Captain] began to manually depressurize the cabin per the ECAM. While he was starting the procedure, I was just beginning to climb through 10,000ft. I voiced to [the Captain] that we can't be going above ten if he is depressurizing the cabin and he agreed and suggested getting clearance to an altitude below 10,000ft. I quickly leveled off somewhere between 10,000 and 11,000 and requested a lower altitude. I believe [ATC] had cleared us to descend to 9,000 but can't remember the exact altitude. By the time the descent was initiated, the cabin had caught up with the aircraft altitude and we received a warning for CAB PR EXCESS CAB ALT. [The Captain] was about to start the SOP for this abnormal but the message went away very quickly as we descended through 10,000ft. We than decided as a crew that it was best to ask [Approach] for holding in the area while we conferred with the company and decided the best course of action. We were given the published hold over ZZZZZ intersection. In the hold, [the Captain] sent ACARS messages to dispatch about our condition and the possibility of a return to field or divert. [The Captain] also called ops at ZZZ and spoke to the ops manager as well as line maintenance about possibly diverting there due to better maintenance handling vs. ZZZ1. We notified ATC that we might be diverting to ZZZ due to a pressurization issue. At no time did we declare an emergency. Before making the decision to divert to ZZZ we decided to troubleshoot our problem one last time. Captain recycled the blower and extract valves back to on from OVRD. Going back to the on position cleared away the VENT SKIN VALVE FAULT ECAM and all valves were than showing closed and green. We than decided to put the cabin back in auto mode. The cabin began to pressurize normally. After another turn in the hold and all systems functioning normal we checked our fuel status and decided that continuing to ZZZ2 was possible and, in fact, the best course of action as we would land with over 8,000 lbs. fuel. [The Captain] sent an ACARS message to dispatch telling them that the problem was cleared and we were proceeding to ZZZ2. We ascertained a new clearance from [approach], exited the hold when cleared and continued the rest of the flight normally. Curiously, we received an ACARS messages from dispatch that they didn't know we were continuing to ZZZ2 and thought we were going to divert to ZZZ. I then sent another ACARS message to dispatch explaining once again that everything was working normally and we were continuing to ZZZ2. Later on in the flight we received one more ACARS from dispatch apologizing for the confusion as apparently our last few messages were going to the wrong desk. We landed at ZZZ2 without further incident. I believe the CAB PR EXCESS CAB ALT warning message was self-inflicted. Better crew communications could have taken place regarding the ECAM actions being executed. Specifically, getting clearance to an altitude below 10,000ft. Before starting to manually depressurize the cabin. This particular ECAM procedure could possibly be written better. Possibly with a better warning about not exceeding 10,000ft. cabin alt when starting manual depressurization procedure. Adding the possibility of cycling the blower and extract valves back to ON from OVRD should be included as well. Either way better communication between crew members should take place when doing something as extreme as manually depressurizing the cabin to prevent exceeding 10,000ft. cabin alt.

Synopsis

Shortly after takeoff, crew received a "VENT SKIN VALVE FAULT" alert message. While performing the QRH procedure and without coordination with the Pilot Flying, the Pilot Monitoring began to depressurize the aircraft while in a climb to 12,000 feet. Crew then received a "CAB PR EXCESS CAB ALT" alert and re-pressurized the aircraft. During that procedure the original alert was cleared. Crew continued to destination.

ACN: 1312537 (1643 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : ORD

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9966
Experience.Flight Crew.Last 90 Days : 148
Experience.Flight Crew.Type : 4145
ASRS Report Number.Accession Number : 1312537
Human Factors : Distraction
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 165
ASRS Report Number.Accession Number : 1312538
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Coming over the marker 27L at ORD on a visual approach I commented on strong x-wind at alt and the large crab we had in. Winds were 190/46 and questioned why we don't use 22L/R anymore. AT 750 feet we received an aural warning with a too low gear. We were at flaps 2 and 170. I then called for gear and flaps [and] landed normal. We should have gone around at the aural warning and even though the landing was normal it was not the SOP. Looking back on it we must have also missed the 1,000 feet call out. If not we would have seen the abnormal configuration.

Narrative: 2

Cleared for visual approach about 20 nm from runway. Wind strong out of south at 42 to 48 knots. Severe clear, excellent visibility. Monitored Pilot Flying (PF), handling crosswind. Reached GPWS RA without realizing gear was not down and flaps at 2. Should have told PF to go around at that point but for unknown reasons, on command put gear down and flaps full. Approach was otherwise stabilized. Landing was normal.

Synopsis

A320 flight crew reported being alerted at 750 feet AGL that gear and flaps were not in landing configuration. Flight crew continued to an uneventful landing after properly positioning flaps and gear.

ACN: 1312210 (1644 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : PCT.TRACON
State Reference : VA
Altitude.MSL.Single Value : 17000

Environment

Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : TERPZ5
Airspace.Class B : PCT

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : HORTO2
Airspace.Class B : PCT

Person

Reference : 1
Location Of Person.Facility : PCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 13.0
ASRS Report Number.Accession Number : 1312210
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was working the morning departure push of aircraft from. At the time of the event I was working between 4 to 6 aircraft. All were on RNAV SIDS. 

Aircraft X, was climbed to 17,000 feet via the SID. Aircraft Y was climbed to 17,000 feet upon initial contact after departing. Aircraft Y was on a SID.

The paths of the SIDs cross. When I climbed Aircraft Y to 17,000 feet Aircraft X was a few miles west of its departure airport out of an altitude of somewhere between 5000 feet and 10,000 feet. 

I was reasonably assured, based on the good climb rate of this morning's group of aircraft, that Aircraft Y would not be in conflict with the other aircraft climbing. I was led to this assumption because aircraft previous to Aircraft X were switched to my frequency well before my airspace boundary and in more than ample time for me to climb the aircraft to 23,000 feet and easily top the aircraft departing DCA which were climbing only to 17,000 feet. 

All of my assumptions and decision making based on how the aircraft were performing was all for naught. Aircraft X was never switched to my frequency by the adjacent sector controller and flew for approximately 6 miles into my airspace at 17,000 feet converging with Aircraft Y which was climbing out of 16,000 feet for 17,000 feet when I recognized the problem. 

I immediately issued a vector to Aircraft X that would have caused it to pass behind Aircraft Y. No answer. I called Aircraft X again, no answer. I instructed Aircraft Y to descend immediately to 14,000 feet and issued the traffic. No answer. Again I instructed Aircraft Y, now level at 17,000 feet, to descend immediately to 14,000. (Why 14,000 feet? It was something to aim for lower the Aircraft X at 17,000 feet.). A pause, then Aircraft Y answered stating they were descending. 2 seconds too late. I then instructed Aircraft Y to turn left to a heading, maybe 280 degrees, I don't remember exactly. Any heading to the left of the course Aircraft Y was on was one that I had hoped would avoid what was looking like an imminent collision, even though I was told the closest proximity was 1.9 miles and 2.1 miles.

After Aircraft Y responded that they were descending to 14,000 feet I called the adjacent sector controller and told her "Aircraft X was not on my frequency." A couple seconds later Aircraft X called me level at 17,000 feet and I immediately issued a climb to 23,000 feet. Again, 2 seconds too late.

I did not issue traffic alerts to either aircraft because in my opinion that was wasting precious time to correct the error and keep a bad situation from ending much worse. 

There has been a great initiative to introduce RNAV SIDs and STARs into the National Airspace System (NAS). Virtually every air carrier that operates within the Potomac TRACON airspace is on a charted RNAV SID or STAR. The result is a known path and altitude crossing restriction for each aircraft with minimal input from the air traffic controller. Sounds like a good idea. And it is except that the air traffic controller now spends more time monitoring an aircraft's compliance with the SID/STAR rather than directly controlling it with vectors and altitude assignments. This, I believe, leads to a sense of complacency or expectation bias that was not as prevalent in air traffic control before the introduction of RNAV SIDS and STARS as they are used now. This does not take away from the fact that I erred in not ensuring separation. This event makes me more aware than ever that with less interaction between air traffic controller and pilot it now requires more vigilance on ensuring separation.

Synopsis

A controller took handoffs on two departures off different airports on converging routes. Although both aircraft were climbing to 17,000 feet, the controller was initially only in communication with one of the aircraft. He could not make contact with the aircraft he intended to keep climbing and waited too long to issue other aircraft instructions to ensure separation.

ACN: 1311326 (1645 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13225.78
Experience.Flight Crew.Last 90 Days : 198.27
Experience.Flight Crew.Type : 10768.05
ASRS Report Number.Accession Number : 1311326
Human Factors : Confusion

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

Requested takeoff data for runway 28 at BWI. ATIS shows departing from intersection U2 with a distance of 8,700 feet. Looking at the takeoff data message for runway 28 at intersection U2 we noticed that it only says U1. ATC says they're designated that intersection from U1 to U2. However, going by that logic the distances should match. Our data shows 9,800 but the ATIS says only 8,700 feet. Checked with Dispatch and the information we got was that engineering has not updated this yet. Just a heads up, 1,100 feet distance available can be significant.

Synopsis

An A320 Captain is informed that Taxiway U2 has been designated as U1 at BWI but the runway available from the takeoff data message does not reflect the new designation.

ACN: 1310877 (1646 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1310877

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On approach the first officer/flying pilot commanded gear down. When the landing [gear] came down with spoilers armed I noticed that the number one and number two spoilers had amber X's meaning inoperative. Shortly after that we received an ECAM for a "nose wheel steering fault". We informed the tower that we were going around and proceeded to do so. On the command gear up, I placed the gear lever to up position. Shortly after that we received an ECAM for "landing gear not up-locked", and we could hear that the gear was still down. We discussed recycling the gear and I placed the gear lever to the down position. I decided to leave the gear down because all indications showed that the gear was down and locked. I also felt that trying to recycle the gear might cause damage. We also noticed that the ECAM for "nose wheel steering fault" had gone away. We decided to proceed back to the airport to land on Runway XY because it was the longest runway on the south side of ZZZ. I ran the performance numbers out of the QRH for two or more spoilers inoperative, and decided that we would be able to land on that runway. The landing was routine and we were able to slow the aircraft with no problems. Somewhere below 70 knots I asked for "control of the aircraft". The first officer gave me control and I went to turn the aircraft with the tiller and the "nose wheel steering fault" came back and we had no nose wheel steering control. I was able to get the aircraft off the runway by using differential braking and thrust. Once stopped we informed ground control of our problem and notified them that we would need to be towed in. Our ground handling personnel were able to pull us into the gate without further incidence.

I don't know of any suggestions, but the aircraft is a tired old bird that has caused me more headaches than need be.

Synopsis

A320 Captain reported several mechanical anomalies were annunciated upon landing gear extension, possibly related to an aging aircraft.

ACN: 1310876 (1647 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MMUN.Airport
State Reference : FO
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : MMUN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class E : MMUN

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1310876
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were cleared for take off 12R MMUN were instructed to follow ROTGI one ALFA departure, contact departure control at the departure end of the runway. Passing through 2,500 we were radar identified and told to turn "left to heading of 360, intercept the airway" The autopilot was not engaged. I read the clearance back turned the heading selector to 360 and asked the flying pilot, the captain, if he would like that set up in the box? The Captain engaged the autopilot told me no and called for Flaps 0 after takeoff checklist. I checked the speed raised the flaps and engaged the right pack and proceeded with the after takeoff checklist. The Prior to intercept of the airway the captain turned the aircraft left [right?], away from course. I told him that I thought the course was to his left. He did not respond but then keyed the mike and informed ATC we are unable to intercept the airway. ATC responded that we should be able to intercept the airway from the 360 heading. I again said the course is to the left of us. ATC then instructed us to contact Merida center. At this point, I thought the captain was flying this aircraft single pilot and he had a lost look on his face. I pulled up the airway on the [tablet], Tuned in CUN VOR, Dialed in the 012 radial and displayed the VOR on my NAV display. The NAV display confirmed we were to the right of course and tracking away from course. The captain still took no action. I then told him "the course is to our left, you see I have the VOR on the NAV display." He then told me he didn't understand. I told him "the course is to the left, it is the 012 radial from Cancun." He then started scrolling through the FMS. I contacted Merida Center. Between transmissions I noticed we were still tracking further from course and the captain still failed to take action. We were VFR and I debated taking control of the aircraft. I then asked Merida Center for Direct RAKAR. Merida directed us to fly direct to MYDIA. 

The captain then tried to blame me for the error, claiming I did not put it in the box which confused him. I told him I asked him if he would like it in the FMS and he responded NO. He then denied it. I told him we weren't that far off and are now on a direct course. He continued to try to blame me for not understanding basic airmanship. He claimed he didn't understand why a course of 012 from Cancun would apply. He then told me he must not have understood what I was asking and he didn't remember telling me not to insert it into the box. I explained that the clearance should have been to turn to a 360 heading and intercept M219, which if he looked on his chart is the 012 radial from Cancun. After several minutes of back and forth, he told me I should have put it in the FMS correctly. 

I have 16 years flying jets 121. Most of my jet time is international heavy experience. This captain is a VERY challenging individual to fly with. He routinely ignores clearances and takes over all of the duties without a single warning, removing the other pilot from involvement. On other legs he would ask for clearances with out asking the flying pilot if it was ok. HE has ZERO CRM skills and to be honest our CRM classes are lacking.

Synopsis

An A320 First Officer reported a departure from MMUN with the Captain flying, during which a track deviation occurred as the Captain did not comprehend the clearance. The Captain believed that the First Officer had not set up the FMC properly and the First Office felt that the Captain had poor CRM skills.

ACN: 1310840 (1648 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : SJC.Tower
State Reference : CA
Altitude.MSL.Single Value : 1020

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : SJC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class C : SJC

Component

Aircraft Component : Traffic Collision Avoidance System (TCAS)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1310840

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were departing SJC on runway 30R. Conditions were clear skies and 10 miles or greater visibility. My duties were the pilot monitoring and the F/O (First Officer) was the pilot flying. Upon reaching thrust reduction altitude, we received a TCAS RA to descend at a rate of at least 2000 fpm. The target aircraft was at our 12 o'clock less than a mile and 500 ft above us. There was no way we were able to comply with the TCAS command [because] as soon as the TCAS alert had happened, it disappeared. Further, our initial climb rate was over 4000 fpm because were very light. We never saw an aircraft and shortly after, SJC Tower instructed us to contact departure control. Tower and NorCal TRACON never reported any aircraft in our vicinity. I don't know if this was a phantom RA or not. Needless to say, this was a very disconcerting event. I truly thought we were going to have a midair collision. We both were very shaken by this event and I even failed to report it to SJC Tower or any controlling ATC authorities. This event only lasted about 3-5 seconds. We didn't even have time to comply with procedures. The FO elected to continue the climb to avoid the RA and we did so by 200 ft.

It is unknown if there was or wasn't an aircraft directly in our departure path, but we never saw one. I just don't know.

After reflecting on this event for several days, I have come to the reasoning, that with the surrounding airspace protections at SJC, the possibility of an aircraft intrusion in the departure corridor is possible, but not likely. My failure to recognize that it was a possible phantom RA, I didn't realize that the TCAS system on this aircraft may be failing as well. I did not write up the TCAS system on this aircraft and it probably should be.

Synopsis

A320 Captain reported receiving an RA on climb out from SJC that was likely a false target generated by a malfunctioning TCAS.

ACN: 1309659 (1649 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MMMX.Airport
State Reference : FO
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : IMWX
Flight Phase : Final Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 178
Experience.Flight Crew.Type : 1781
ASRS Report Number.Accession Number : 1309659
Human Factors : Situational Awareness

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

For nearly a year, the approach fix MAVEK is not auto loaded into the FMGCS when the ILS 05R is selected. It is, however, co-located with SMO. Since most pilots have flown into MMMX enough, we understand how to configure the FMGCS to work for us. There are numerous tips both on 10-7 page and elsewhere to assist the pilot.

However, by now the situation should have been resolved and has not been. It is a matter of time before two inexperienced pilots go in there and cannot find MAVEK in the box or try to manually insert it and end up with a mess at a very critical time. In addition, if one tries to use total automation, the step down altitude from 9,200 feet to 8,800 feet at PLAZA. The airplane will always be a touch too high to intercept the glide slope. As a result, it will revert to V/S and fly right through the glide slope and get high.

Obviously, all of these problems can be solved by going manual and hand flying the approach from MEX 05 all the way down, but can easily catch a new pilot off guard. (Relying on go arounds at Mexico City is not much of a plan)

Synopsis

A320 Captain reported MMMX 05R Approach does not load properly in the FMCGS.

ACN: 1309657 (1650 of 2908)

Time / Day

Date : 201511

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 8412
ASRS Report Number.Accession Number : 1309657

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9506
Experience.Flight Crew.Last 90 Days : 218
Experience.Flight Crew.Type : 4336
ASRS Report Number.Accession Number : 1309653

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At a cruise altitude of 34,000 feet and just prior to our top of descent point, the First Officer (FO) recognized a decreasing oil quantity situation with our number two engine. I used ACARS to send a [message] to Dispatch, turned on seat belt sign and called the Purser to inform her of the impending situation. ACARS returned with a frequency for contact as the FO took over flying duties. [Maintenance] joined the call and we discussed what we should do. We decided to operate engine at idle. The First Officer asked for a lower altitude. As we were approaching the top of descent point, the oil quantity continued to decrease to 3.5 and we decided to shut the engine down to prevent any further damage. I took over the flying duties, as the FO completed the engine shut down and single engine drift down check lists. Again communicated with the Purser. We did not prep the cabin, but asked her to do a quick review as we'd be landing shortly. The rest of the approach and landing was uneventful. Well, as can be with one engine.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

An A320 flight crew reported an inflight shutdown of the number 2 engine following a loss of oil quantity.