ACN: 1309652 (1651 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : DEN
Make Model Name : Small Transport
Flight Phase : Final Approach
Airspace.Class B : DEN

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 1238
ASRS Report Number.Accession Number : 1309652
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8294
Experience.Flight Crew.Last 90 Days : 174
Experience.Flight Crew.Type : 6587
ASRS Report Number.Accession Number : 1309654
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We had briefed the visual for 16R, backed up by the ILS to 16R. We were cleared for the visual approach, and to maintain 210 knots. A [B737] flight ahead of us, also cleared for the visual approach, started to slow rapidly. Denver Tower slowed us to 170 knots, which we did. We were then instructed to slow to 150 knots, which we did. About the time we slowed to 150 knots, approximately 8 miles from the runway, we received a descending RA. I looked out of my window, and saw a turboprop flying just off of our wingtip. The aircraft appeared to be lined up for 16L, and continued to fly parallel to our flight path, not getting any closer or further away. The Descending RA continued. When we were about 1 dot below the glide slope the First Officer started to level off to recapture the glideslope. I commented that we still had an RA, and he returned to the RA escape maneuver. I turned the transponder to TA, but the escape maneuver still displayed that a descent was necessary. We were descending at around 1500 feet per minute, less than 2500 feet above the ground. After some discussion, we decided to ignore the RA, as to continue would result in ground contact somewhat short of the runway. Around 1100 feet above the ground, we flew into the "red descend" zone on the TCAS escape maneuver, and re-intercepted the glideslope. Eventually, the aircraft landing on 16L slowed, and the RA escape maneuver disappeared. We never received a "clear of conflict" message.

Narrative: 2

TCAS RA guidance was still directing a steep descent until approximately 800 feet AGL, but aircrew elected to continue the descent at a shallower descent rate for fear that continuing the steep descent guidance given by the TCAS RA would then trigger an EGWS and/or GPWS escape maneuver, (further [exacerbating] the TCAS RA). Aircraft was stabilized before 500 feet AGL, and landed uneventfully.

Synopsis

A320 flight crew reported receiving a "Descend" RA on approach to DEN that commanded a steep descent until approximately 800 feet AGL.

ACN: 1309110 (1652 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZME.ARTCC
State Reference : TN

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZME
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZME

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1309110

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Loss of Yellow hydraulic system due fluid leak. We had a normal preflight. FO did not notice any hydraulic leaks on walk around and all fluid levels checked on system pages. We had a normal taxi out, take off and climb out. In cruise, I checked system pages and noticed Green system level below green band but well above amber band. Blue system was in green band. Yellow system was below green band and just above amber band. We continued to monitor Yellow system level. About 1 1/2 hours out we got a caution and ECAM for hyd low pressure. We followed ECAM, turning off Yellow engine pump, PTU, and applying landing distance procedure. We followed up in QRH for further guidance. There was no "land ASAP" so I continued. I informed Dispatch of the failure, my intentions, and asked them to coordinate with [Maintenance and Operations]. I also informed them of my intentions and request priority handling to a long runway. We had lost nose wheel steering, number 2 reverser, spoilers 2 and 4, and other goodies, so I felt it prudent to request a long final and emergency equipment to standby. We had slow flaps and slats so I wanted to be configured and on speed early. I landed from the ILS. At touch down I used rudder, differential braking with normal brakes, and idle reverse on engine 1. I was able to get a/c stopped and maintain directional control with about 4000 ft remaining while not making any aggressive control inputs. I held the aircraft with toe brakes even though we had accumulator pressure. Fire and rescue inspected the aircraft and found nothing. I shut down engine 2, left engine 1 running for Green hyd pressure. Tug hooked up and towed us to gate. I shut down when chocks were in place, secured the aircraft, wrote up the discrepancy and notified [Maintenance]. No deviations from FAR or company regs were required.

Synopsis

A320 Captain reported loss of Yellow hydraulic system enroute to destination.

ACN: 1308598 (1653 of 2908)

Time / Day

Date : 201511
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZKC.ARTCC
State Reference : KS
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZKC

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1308598

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were at cruise at FL350. When doing the level off checks, the CA noted that there was lower than normal Yellow Hydraulic Fluid Quantity.

We continued flight and [enroute] we received an ECAM alert "LOW YELLOW HYDRAULIC PRESSURE." We followed the ECAM and turned off the associated pumps, referenced the QRH and [flight manual] and made the decision to continue. [Later] we got "LOW YELLOW HYDRAULIC QUANTITY." We then ran the ECAM and reviewed once again the QRH and [flight manual]. Looking at the associated system failures, we determined that on landing we would be unable to get off the runway without assistance. At that point, we received no land ASAP message and made the [decision] to continue to [our destination]. We then informed ATC of the malfunction, requesting services to help us to vacate the runway and requesting the longest runway due to the malfunctions. We landed uneventfully and were escorted off the runway by operations and a super tug.

Synopsis

A320 FO reported continuing to destination after loss of Yellow Hydraulic System.

ACN: 1306227 (1654 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Flight Phase : Climb
Flight Phase : Cruise
Flight Phase : Landing
Airspace.Class A : ZZZ
Airspace.Class B : ZZZ
Cabin Lighting : Medium
Number Of Seats.Number : 150
Passengers On Board.Number : 138
Crew Size Flight Attendant.Number Of Crew : 5

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
Experience.Flight Attendant.Total : 31
Experience.Flight Attendant.Airline Total : 31
Experience.Flight Attendant.Number Of Acft Qualified On : 3
Experience.Flight Attendant.Type : 100
ASRS Report Number.Accession Number : 1306227
Human Factors : Physiological - Other
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Arrived on aircraft at XA:15 and smelled odor equivalent to oil. As soon as we took off it worsened and Captain called us to tell us we were diverting because of the odor and they had donned their oxygen. We were on the aircraft for over an hour before we were able to get off and smell fresh air. I have since been experiencing painful headaches and dizziness. This happened one other time to me on an A319 but I recovered in 3 days. This time I am not rebounding as quickly. Since we smelled such a strong oil smell before we took off I was real surprised we left our departure airport.

Synopsis

An A320 Flight Attendant detected an oil odor during an early morning preflight. The flight departed but then diverted with the pilots on oxygen after the odor intensified once airborne. The reporter experienced painful headaches and dizziness days after the exposure.

ACN: 1305483 (1655 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9125
Experience.Flight Crew.Last 90 Days : 201
Experience.Flight Crew.Type : 7363
ASRS Report Number.Accession Number : 1305483
Human Factors : Distraction
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The pushback started well with good, standardized communications. Early in the pushback, the captain was told by push crew "cleared to start 1 or 2 or both." Captain acknowledged "clear to start." Captain instructed probationary FO to start 1 and 2. As the aircraft came to a stop, with engine 1 about to reach idle, push crew stated "here comes late bags, can we put them on?" Captain replied with a strong "negative." Captain believed FO was starting engine 2 and did not want to interrupt the start sequence. Again pushback crew asked "can we put on bags?" Captain gave another strong "negative, we are in the middle of our start sequence!" Captain felt forward cargo door open and asked "did you hear me say negative?" Pushback crew said something to the effect "yea, but got the lead talking at me too, it'll be closed in a minute." The very astute FO realized something was wrong hearing me repeat strong "negatives" and did not begin engine 2 start. Captain explained to pushback crew that someone could have easily been killed. This did not seem to bother the tug driver. Throughout this exchange the parking brake had never been set. Captain asked for tug driver's name and file number. Captain called operations and explained what had occurred. Captain passed name to operations and emphasized how dangerous the situation had been and asked that the tug driver not push another aircraft until this had been addressed.

Synopsis

During pushback, the ground crew cleared pilots for engine start then requested to open the forward cargo door for late bags. The request was denied, but the ground crew opened the door anyway. The #2 engine start had fortunately been delayed.

ACN: 1303872 (1656 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MIA.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1303872
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Runway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

This was a runway incursion that occurred while taxiing out to the active runway, no part of the aircraft was beyond the holdbar, but the holdbar used was the incorrect one, designed not for the runway in use but for landing aircraft on adjacent runway. The actual area is designated a 'Hot Spot' and was briefed by the captain at the gate but not obvious to my captain while taxiing out. I was 'inside' (not looking out) while carrying out my First Officer (FO) duties and was not able to assist my captain at that point during taxi. The ground controller was very graceful and pointed out our mistake for "future reference." No immediate safety consequences apply. 
I would like to add that my Captain and I initially felt there was no need for an ASAP report, but I decided to file the report because I'm finding safety issues, hazards and threats starting to spring up while flying the line with even with experienced captains, that if left uncheck will cause aircraft damage, injury, or death. I am no spring chicken: have been with this company for nearly 13 years and have previous experiences at other major airlines that ended due to career advancement or furlough. I hope you take my report seriously and for those that have not been in the right seat for a while, to listen carefully and remember that we have many inexperienced new-hires, and very junior captains. I would like to include the training department and the architects of SOPs that have set up conflicting FO responsibilities to be nearly impossible to comply with in the real world, with the possibility of rendering the FO with an always at fault/lose-lose situation. 
This airport's Hot Spot (HS) 5 is depicted for a good reason. It was briefed and not obvious to the captain. I believe that if my task responsibilities were lighter or less completed I would have been able to follow along with the captain during his taxi. There was never a moment of doubt or curiosity that would have triggered my attention. I believe this particular hot spot will keep on bringing more and more incursions, especially with those unfamiliar with the airport. I've flown to military and civilian airports where controllers issue taxi instructions with hold short remarks such as 'hold short of taxiway XYZ by (a particular landmark, chart depiction (i.e. 'the box') or stationary object)' as emphasis of a particular location. The most obvious characteristic about HS 5 is it's triangular shape. Designating and depicting HS 5 as 'the triangle' at this airport on charts, and ATC asking pilots to 'hold short of the triangle' would bring 'heads up' to pilots unfamiliar with the airport to pay attention and force them to figure where this 'triangle' is, to make sure they hold short where they are supposed to. I really believe this is the easiest and best solution for this airport. 
The following applies to this particular experience and others as well. First, feel free to ask any of our check airmen how many tasks and items are taken care of by the FO from the time the airplane pushes back until takeoff. The workload is intense, and is no secret, as some of our check airmen will proudly tell you to the exact number of how many there are. This of course, comes with great conflict while trying to help a captain find his way around. Either checklists/procedures will be done in error/skipped, or taxiway/runway incursions are going to occur. Where the SOPs are not helping: now combine the previous threat with (especially the latest one) 'no heads down while in a turn,' that brings another checklist implications per SOP on checklists: "If a checklist is interrupted and any uncertainty exists regarding the last item performed, it must be re-accomplish 'Uncertainty' is stated here in a very liberal way, since the captain and FO are leaving it to their short term memory if the task or item had been 'really' performed -truly a Russian roulette. There are also 'flows' where attention and concentration is critical, and the likeliness of a captain holding short of a turn for an FO to finish up is not only impractical but could impact airport traffic flow and perhaps a reprimand from ground controllers, try that at [another busy airport] right? The system in place will always find a way to blame the pilots if they were to fail, especially the FO. 
I flew with an experienced captain not long ago that had complete disregard to my workload. We were in ATL and had a runway change and new ATIS coming up, and we now had an intersection departure assigned and we were #2 for takeoff, with the #1 already lining up on the runway. We had ample time to accomplish everything, but this captain elected to speed through the taxiways. I even asked him, "Hey, slow down a little;" his answer, "I'm doing the best I can." Right, at 30 kts down the taxiway, and he left it unchanged, unbelievable! I verbalized how uncomfortable I was while doing things in extreme pressure while lining up on the runway. I later thought and wondered if we ever did the before-takeoff brief. I was extremely furious. I didn't say anything and waited until cruise altitude to see if he would say anything, but he never did. On a later flight on this same pairing and same captain we are taxiing very early in the morning out of [another airport], I copied the taxiing instruction clearly, understood it, and he had no questions. So I'm going about my flows (inside) and a moment later ground control is yelling at us because we missed a turn on a taxiway. He was speeding along in his taxiing and not paying attention. He didn't say anything! Maybe this is all 'normal' to him! 
I'm sure you are asking yourselves: so why didn't you bring this up? Short answer: (And be ready, I'm going to spill the beans here.) Because I'm done with the careless tribal culture of a few of our captains. I am the one that through the years have spent numerous amount of hours with professional standards 'baby-sitting' some of our most unprofessional captains. I have even written up a problem captain, a captain that professional standards have asked me to write up because they, are done with him. [Chief Pilot] contacts me to tell me that "the truth lies somewhere between what I said and what he said, and that they will not do anything about it." As you can see, I'm done with the captain baby-sitting. On a more personal note, the training department failed me on my initial attempt to upgrade, not because what I didn't know, but because of their own internal issues. My instructor was clueless on what the checkairmen where requiring on certain maneuvers. On my subsequent second attempt the checkairman failed me on a maneuver that was 'passed' on the previous original checkride, while I was doing exactly the same procedures. This lack of standardization and leadership IS affecting and/or WILL affect operations. It will trickle down. I have been a victim of this poor system in many facets; I'm trying to do my best to then be shot down. Very demoralizing frankly. You have to figure how all this fits in the big picture and also figure out what level of safety you want this airline to operate. I'm not holding my breath, and you can see why.

Synopsis

A320 First Officer reports a runway incursion during taxi out to Runway 8R at MIA. The area is designated as hotspot 5 and has its own inset box to explain the confusing hold short lines.

ACN: 1303807 (1657 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MCI.Airport
State Reference : MO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Component

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5600
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 1576
ASRS Report Number.Accession Number : 1303807
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 186
Experience.Flight Crew.Type : 702
ASRS Report Number.Accession Number : 1305204
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

The weather was forecast CAVOK and didn't see a cloud in the sky. Aircraft NO COMM. Made decision to proceed, thinking we would get ACARS back after push. Captain spent the entire time until taxi out with dispatch, verifying and collecting data. Dispatch couldn't (wouldn't?) send performance data to captain, but relayed verbally. After actual push stayed NO COMM (until climbing through 27K). Made decision to continue with flight after push when we realized we were NO COMM based on weather, familiarity with airports, and no other traffic on ground. Took us 27 minutes to verify everything. Made trip uneventfully. 
Several thoughts:
*We had to trust dispatch was passing valid numbers. They seemed reasonable, but I wasn't in the phone loop, so it could have been the winning lottery numbers for all I knew.
*No way to communicate to dispatch without using cell phone. 
*No way to verify any of the info on the takeoff checklist, other than give it a sanity check.
*Dispatch couldn't send performance data to captain's cell phone, but kept sending the flight plan. After getting ACARS back we got lots of takeoff data messages (sending to aircraft printer?)
*There is no checklist for ACARS INOP/NO COMM for the A320, but there should be.
*Searched FOM and FM for NO COMM and ACARS INOP. Nothing useful found. Except update the [off] times, and since it was our first flight of the day, that was the least of our worries. And monitor commercial radio frequency in case dispatch wanted to call us. We did both.
*The performance figures passed were 146, 146, 146. The numbers we were asking for showed up at 27K with ACARS were 142, 143, 146. At least we were on the high side. Not sure if dispatch knows how to read a runway performance message, or if he was reading the correct line...
*There was no way to support the captain, or verify the info in ACARS. 
*Kept getting the MCDU asking us to initialize while flying. Can we? Should we? What if we don't? We just deleted the request message. Worked normally. 
Went through airbus training for the 3rd time 2 months ago. Nothing in the training syllabus prepares you for ACARS INOP. Every time we wanted to talk to dispatch, we sent an ACARS message "call me". We always got performance numbers through the printer. If company insists on sending aircraft out with ACARS INOP or NO COMM, please give us the training and checklists. 
The captain and I both agreed we wouldn't push again hoping ACARS would work, or knowing it was deferred. Next time, we are going to get maintenance involved and fix the aircraft. 
There were too many threats to list. We think we covered everything, but not the right way to fly the aircraft.

Narrative: 2

I had NO COMM at the gate and could not send or receive any data. There have been multiple bulletins out recently regarding ACARS stating new requirements that must be met in order to remain FAA compliant. These added requirements have included no training or additional help to flight crews. The latest bulletin stating that it was permissible to push back the aircraft to try and establish communications is misguided. I opted to do this and regret having done it as communication was not restored until FL270. If I did not have my personal cell phone this flight would have gone nowhere. Once again, we must use our own resources to make things work.
I had to call the dispatcher 4 or 5 times to get all of the information I needed for takeoff. The dispatcher was not very good at reading the performance data and in fact failed to advise me if there was an engine out procedure which I had to call back for. I also had to remember to ask him what weather and winds he used due to the Sabre anomaly. I asked for the performance data to be emailed to me so that I could actually read it instead of relying on it being read correctly by the dispatcher. There is simply too much information to retrieve without a worksheet in front of you to help remember all the requirements. He tried twice but failed to send takeoff data but rather sent the flight plan multiple times. I had to make multiple phone calls after being pushed so we blocked others, but there is no other way to get a [off time] in a reasonable amount of time.
My first officer had not ever been trained on these procedures, so I was basically on my own. I had to instruct the First Officer (FO) where to find the proper frequency for commercial radio and to also call the out/off times back to the station. The workload is extremely high and the entire situation is ripe for omitting required data and/or communications. I will never again accept and airplane with ACARS inop. It puts the safety of flight too much at risk under normal circumstances and would be even worse with bad weather, which would hinder radio reception and frequency availability, or any emergency operation. I simply cannot justify taking the aircraft.
The removal of engine failure on takeoff data from the 10-7 is an additional threat. Yes I can look up landing data during a divert but I have no way to know if I can safely go-around. VHF radio is not always an option as evidenced during some of our past aircraft electrical emergencies. I believe the removal of this important data from the 10-7 pages was an error that should be reversed.

Synopsis

A320 flight crew experiences an ACARS failure at the gate and has difficulty obtaining takeoff data from the Dispatcher. The Captain believes that the ACARS unit is essential to safe operations and the aircraft should not be dispatched with out it.

ACN: 1303541 (1658 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC
Altitude.MSL.Single Value : 24000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : FILPZ1
Airspace.Class A : ZTL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1303541
Human Factors : Other / Unknown

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

ATC giving changes to a new STAR Arrival into CLT, To cross BACKK below 14,000 ft [FILPZ1]. BACKK published restrictions of FL190-12,000 ft. We are 5,000 ft high from what FMS has planned. We have already started our arrival descent and with not enough time to reprogram system. Auto Pilot OFF, Speed Brakes Out, Dive to make restriction. WHY? Two days earlier on IVANE got MAYOS at FL220 with very short notice. A/P OFF and Dive to make restriction. This must stop happening! We did not have an Altitude Deviation but this should not be normal business practice in CLT!

These changes are happening in CLT all too often. Please change STAR to what is needed for an arrival (Isn't this the reason for a STAR?) These last minute changes have to stop or I will stop accepting STARS or refuse to stay on a STAR with these kinds of requests. I hope you can feel my frustration in this matter. I'm too old to keep putting up with this stuff. I would be happy to help someone get this changed.

Synopsis

A320 Captain laments frequent ATC changes to RNAV arrival crossing restrictions requiring the use of speed brakes and defeating the purpose of the OPD.

ACN: 1302748 (1659 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3600

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A319
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 21
ASRS Report Number.Accession Number : 1302748
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Air Traffic Control
When Detected : Routine Inspection
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Aircraft X was an arrival from over [fix] at 7000. Aircraft Y was a straight in arrival on Runway XY. There were multiple layers so we were unable to visual to the capture box. I don't remember the actual coordination but after watching the replay [Sector] was protecting and staying above my arrival. I descended and cleared Aircraft X for and ILS Runway XX approach. When my aircraft was on final Aircraft Y was cleared for an ILS Runway XY and both aircraft were switched to the tower. A RAE event was brought to our attention stating that Aircraft X was not established on the localizer when Aircraft Y was cleared. Looking at our displays on a normal range with multiple aircraft it appeared that Aircraft X was on the localizer. The replay is on a very reduced range showing only the two aircraft involved. What radar data are they using? Is this the same radar data we are using? Are the maps for the replay the same maps we are using on our displays?

This is a systemic problem that both management and controllers are aware of but no changes have been made to correct it. Arrivals over [fix] and straight in traffic on Runway XY require that one aircraft is stopped 1000 feet above the other aircraft. This places one of the aircraft above the glideslope. They are multiple factors into deciding what aircraft goes high and which aircraft descends. Winds, weather, the spacing on final, the aircraft speeds on both finals. We are placed in situations working the finals that make it extremely difficult to comply with the many requirements and provide an efficient flow to the airport. The finals need to be separated until they reach the capture box and are on simultaneous approaches. Unfortunately the redesign team was unable to do this and we are left with basically the same operation we have worked with for the last 20 years but with the added stress of TARP evaluating the finals with a fine tooth comb. The [fix] arrivals need to come to us at 5000 and the straight in arrivals on Runway XY need to have a fix with at or above 6000 feet abeam [fix] intersection. This would be a big improvement on what we have right now.

Synopsis

At this airport aircraft on approach to parallel runways must be separated by 1000 feet vertically until established at a certain point on the approach. The controller believed the aircraft were established on the approach and discontinued vertical separation clearing the aircraft for approach. Upon a later review using more precise displays it was determined one of the aircraft were not established within the confines of the approach. The controllers working the aircraft do not have the detailed precise radar display that is used afterwards to analyze their performance.

ACN: 1302719 (1660 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : MMTJ.Airport
State Reference : FO
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Vectors

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 5863
Experience.Flight Crew.Last 90 Days : 229
Experience.Flight Crew.Type : 3867
ASRS Report Number.Accession Number : 1302719
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We departed from Tijuana Airport with a restriction in our climb of 5000, after the take-off we contacted SoCal departure on 119.6 and received vectors on course, the vector was on a heading of 300 and continue the climb to 15000. Passing 6000 feet we saw on our Navigation Display traffic 3000 feet above us at 5 miles approximately at 1 o'clock [from] our position.

Since English is not our first language, we thought we misunderstood the climb clearance and again contacted SoCal to verify the clearance, and we had the same answer to climb to 15000.

We tried to reduce vertical speed in order to avoid the TCAS warning, but the traffic was too close and we had the TRAFFIC TRAFFIC warning followed by the Resolution Advisory which commanded us to level off at approximately 8500.

When clear of conflict we told the ATC and continued the climb to 15000 with no further information about the traffic.

Synopsis

A320 Captain reported receiving an RA on climbout from MMTJ.

ACN: 1302580 (1661 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1302580
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Arrived at gate and prior to engine shutdown forward cargo door opened. Waved to ground marshaller to come to cockpit, never showed. Called operations for Ramp Manager to discuss danger and procedures violation. No one showed. Called operations 3 times and was advised they passed the word. I advised that I would write the report and was told that is what I need to do.

Synopsis

A320 Captain reported ramp personnel did not follow SOPs on aircraft arrival, putting themselves in danger.

ACN: 1302555 (1662 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8933
Experience.Flight Crew.Last 90 Days : 127
Experience.Flight Crew.Type : 4329
ASRS Report Number.Accession Number : 1302555
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11853
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 9552
ASRS Report Number.Accession Number : 1302547
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

In the early part of takeoff roll all appeared normal. Shortly after the 100 knot call I perceived a thumping sound and vibration. This happened 2 or 3 times in rapid succession. I suspected compressor stall and aborted the takeoff about 130 knots.

Aircraft was quickly slowed to the point I determined we would have no problem stopping on the runway. The FO made a "remain seated" PA announcement. I communicated to the FO to use the switch to deactivate the auto brakes as we were rapidly decelerating to approximate taxi speed and I was concerned with brake temperatures. Tower was notified that we had aborted and their instructions were to exit the runway and turn right on Taxiway R. We asked that fire equipment be dispatched to inspect wheels and tires due to high brake temperatures. After we came to a stop I directed the FO to perform the rejected takeoff QRH. The fire department equipment arrived within about 3 to 4 minutes.

I made an announcement on the PA to inform the passengers and flight attendants that I had stopped the takeoff due to unusual sounds and vibrations and that the Fire Department was coming to inspect the brakes and tires. We were able to establish communications with the Fire Department personnel on a discrete frequency and they said the wheel temperatures were high and the left wheels were smoking.

There was no fire, but the high temperatures had caused the tires to go flat on the left side, and eventually the right side of the airplane. The brake temperatures decreased, and the fire department personnel brought air stairs to the aircraft. During this period I made several PA announcements in an effort to keep the passengers informed of what we were hearing from the Fire Department and that transportation was being arranged for them back to the terminal.

The Fire Department operations told us they were ready to enter the aircraft so after confirming with the purser that the door was disarmed, I opened door 1 left. I spoke with the Fire Department personnel to begin the coordination of the unloading of the passengers.

Shortly after this the Customer service agents and the assistant chief pilot and a flight attendant supervisor arrived and aided in the orderly deplaning of the passengers and their transportation to the terminal. When the passengers were off the aircraft the FO and I ran the parking checklist and the crew was taken to the terminal.

Narrative: 2

It was a normal flaps 1 flex takeoff. Gross weight was approximately 135,000 pounds. I was the pilot flying. After takeoff clearance, I set the power to the Flex detent and called for "Check Thrust" and removed my hand from the thrust levers. The Captain called out "Thrust Set" and I called out the FMAs'. Somewhere after the 100 knot callout by the Captain, the Captain initiated a rejected takeoff. As the pilot flying, my focus was outside the cockpit. The Captain indicated that he heard/felt unusual vibrations and noises. I made a PA announcement to "remain seated, remain seated" and I made a radio call to the Tower that we were aborting our takeoff. I made the 80 knots callout. Once we slowed down, I asked the Captain if he wanted me to turn off the Auto Brakes. He said yes. I made another radio call to the Tower and told him that we were exiting the runway to the right and we would turn right on Taxiway R and to please send the fire trucks to inspect the tires and brakes as we suspect hot brakes.

Synopsis

An A320 Captain rejected the takeoff at about 130 KTS after he heard/felt unusual vibrations and noises. The tires deflated after leaving the runway so crew and passengers were bussed to the terminal.

ACN: 1301650 (1663 of 2908)

Time / Day

Date : 201510

Place

Locale Reference.ATC Facility : PCT.TRACON
State Reference : VA
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : TERPZ5
Airspace.Class B : BWI

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 148
Experience.Flight Crew.Type : 7395
ASRS Report Number.Accession Number : 1301650
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Climbing out of BWI on the TERPZ5 we were given a "climb via" instruction. Then subsequently given a maintain 290 knots or greater out of 10,000. The arrival calls for us to cross TERPZ at or above 11,000 then LITME at or above 14,000 with only a 9.6 mile distance to climb 3,000 feet while accelerating. Both instructions are impossible from a performance standpoint for a fully loaded airliner. I had to get ATC to give us relief from both speed and or altitude to ensure compliance. Changing over to Center we were given instructions to climb to FL230, we had to verify that we were given relief from the LITME restriction. They seemed to not even consider us on the "climb via" anymore?

We were given relief and complied with all instructions as directed. The purpose of this [report] is to bring attention to the procedures and the ATC directions. These procedures are putting flight crews in high probability scenarios of busting airspeeds and or altitudes. ATC changing published instructions and or procedures being published that are at or above the performance of commercial aircraft abilities are a continued hindrance. This puts flight crews in jeopardy and additionally they are not safe. The entire "climb via" and "descend via" procedures are very high risk for flight crews, having to listen for a single word such as "except" to distinguish between what ATC is expecting and clearing a flight crew to do is ludicrous and not safe. ATC shares no responsibility with flight crews once they pass these nonstandard procedures on to the crew leaving us as the final line of making all restrictions. These procedures need to either be issued as published without exception and vetted properly for aircraft performance or they should not be expected to be flown.

Synopsis

A320 Captain reported his concerns with ATC use of "descend via" and "climb via" procedures.

ACN: 1301458 (1664 of 2908)

Time / Day

Date : 201509
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAD.Airport
State Reference : DC
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : IAD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 167
Experience.Flight Crew.Type : 223
ASRS Report Number.Accession Number : 1301458

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We were on final for ILS [Runway] 19C at 3,000 feet and cleared the approach. The weather was moderate rain at night in IMC. The FO was flying. The radar showed green and yellow precipitation ahead and we were in sequence following other aircraft on the approach. The aircraft was gear down and flaps 3 as we were intercepting the glide path in light turbulence with autopilot on. As we approached the glideslope intercept point, a huge radar echo appeared in front of our flightpath and ATC cancelled our approach clearance and told us to maintain 3,000 feet. At this point we encountered severe turbulence and heavy rain and had a very difficult time controlling the aircraft through the autopilot. As the FO cleaned the aircraft configuration up, we encountered a severe downdraft and lost about 500 feet. I commanded the FO to turn the autopilot off and recover the altitude. I requested a climb from ATC and a turn. The radios were very congested and hard to understand and ATC denied our request. We exited the heavy rain and turned north to hold at Martinsburg VOR. In holding we decided to try another approach after the storm front passed and came up with a bingo fuel for our alternate. We headed for another approach to 19C at IAD, but the weather had not improved enough for another approach so we diverted. When we got to [our alternate], I contacted the dispatcher and was told that IAD airport had been shut down for weather while we were on the first approach but IAD never informed the airlines.

Synopsis

A320 Captain reported deviating to an alternate after encountering heavy rain and severe turbulence on approach to IAD.

ACN: 1299616 (1665 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZTL.ARTCC
State Reference : GA
Altitude.MSL.Single Value : 23200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : IVANE5
Airspace.Class A : ZTL

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1299616

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1299617

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While descending via the IVANE5 RNAV arrival with an ATC issued airspeed restriction of 280 KTS, ATC then altered our altitude restriction over MAYOS to descend to FL220. After inserting FL220 into the MCDU both the Captain's and FO's instruments showed us on profile. Subsequently, the FO's profile started showing a 'high' prediction over MAYOS while the Captain's instruments showed 'on profile'. The captain applied speed brakes to increase the descent rate while attempting to maintain the 280 KT restriction. We crossed MAYOS at FL232 and alerted ATC. ATC made no comment and proceeded to clear us to descend via the IVANE5 arrival. The flight proceeded to landing without incident.

The event occurred because of the amended clearance of a hard altitude and airspeed given once on an RNAV arrival. The hard altitude of FL220 at MAYOS followed by a subsequent altitude of FL 220 or above at the next fix appears to confuse the Airbus navigation. Compounding the event was the discrepancy between the Captain's and FO's flight instruments. My profile doughnut showed us right on and his showed us right on until crossing MSTRD.

The company should work with ATC to limit these amended clearances and if they are going to be given then issue them with plenty of time to prepare. Company should put out a read file on this particular situation and if it can't be corrected through software updates than it should be trained. I will be very cognizant in the future when given this clearance and as another barrier have the progress page up.

Narrative: 2

[Report Narrative Contained No Additional Information.]

Synopsis

On RNAV approach, A320 flight crew was issued a speed restriction along with a modified altitude to maintain on the arrival. As a result of maintaining the airspeed, crew failed to meet the fix crossing restriction.

ACN: 1299456 (1666 of 2908)

Time / Day

Date : 201510
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 149
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1299456
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Obvious lack of training on pushback could have caused serious injury or worse. Left both Captain and First Officer (FO) shaken after event.

1. Pushback crew not at aircraft at time of push although buttoned up and ready to go. Possibly at aircraft but confused as to how interphone to cockpit worked.
2. Obvious confusion with push back crew on equipment operation on disconnect of tug.
3. Lack of proper phraseology on communication with push crew.
4. After salute and release, tug crew visibly back in gate area for approximately 5 min or more, after start checklist completed, ATC taxi clearance requested, taxi clearance given, and Captain beginning to taxi, the VIGILANT FO noticed a member of the push crew between nose of right side of aircraft and running engine walking back to the terminal.
5. Captain immediately stopped aircraft, called station operations to explain the dangerous situation. Operations explained it was the individual's first day on the job. I explained this is how people get killed.
6. Once in flight FO called operations to reiterate our concerns and we were told they would conduct a safety investigation.

Synopsis

An A320 flight crew reported ground personnel's failure to observe SOP's during pushback and taxi out which almost resulted in serious injury.

ACN: 1298879 (1667 of 2908)

Time / Day

Date : 201509

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 238
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1298879

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We received a very late gate change. The ACARS started flashing as we were clearing the active runway. Ground was confused as were we. We had just checked 10 mins prior and were going to C-18. Now Ops wanted us at E-14. We had to verify. Finally we taxied to the south ramp area and had to await outbound aircraft. Once clear we started in, approaching E-14, the guidemen were not in position and there were numerous vehicles/personnel between us and the gate. We stopped. Airport maintenance was painting signs on the tarmac directly in our required path to the gate. We sat. Finally, the lead went over to the mx personnel to move, they did. Then had to get other vehicle including a fuel truck to move to give us clearance to taxi in.

After several minutes, two wing walkers arrived and started out to their positions. With them moving, the lead gave me guidance signals to start in. I complied looking mostly ahead and right (fuel truck, ground maintenance truck, numerous personnel were all on the right). All of the sudden a Ford Pickup came into my peripheral view from the left. I hit the brakes quickly not wanting to hit the truck nor rock my passengers. We came within 10 ft of the truck per my estimate, he disappeared under the nose. Ramp Tower was informed but didn't see anything per usual.

Synopsis

A320 Captain reports receiving a gate change after landing then having to wait for outbound traffic before being cleared in. Before reaching the gate, airport personnel painting lines on the ramp along with their vehicles are blocking further progress. Once vehicles and personnel are clear and wing walkers have arrived, the Marshall directs the aircraft into the gate only to have to stop abruptly to avoid a truck dashing through the gap.

ACN: 1297407 (1668 of 2908)

Time / Day

Date : 201509
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PDX.Airport
State Reference : OR
Altitude.MSL.Single Value : 5500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : P80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway 28R
Flight Phase : Descent
Route In Use.STAR : HHOOD3
Airspace.Class E : P80

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Function.Ground Personnel : Vehicle Driver
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 222
Experience.Flight Crew.Type : 4503
ASRS Report Number.Accession Number : 1297407
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

As we were descending via the HHOOD3 arrival into PDX we were given clearance to OMMSI and a cleared alt of 6200 feet. Then after calling the airport in sight, we were cleared direct to TOLOC (OM on the ILS 28R) and told to cross TOLOC at or above 2000 feet. I was hand flying as we approached TOLOC at about a 45 deg intercept to final. There was a peak about 15 SE of PDX which I could see we were clear of, but as we passed over the top of it we got the GPWS terrain alert. I stopped the shallow descent we were in and the alert went away. Looking back at our position I should have been about 1000 feet higher crossing that peak to be on a 3 degree glide path. We continued to a normal landing. I should have looked more closely at the charted terrain on the approach chart and stayed higher longer, but once we got the clearance to cross TOLOC at or above 2000 feet, I continued a shallow descent. It could have been prevented if I had done a quick 3/1 descent calculation while we were still at about 6000 feet.

Synopsis

A320 Captain reported receiving a GPWS "Terrain" warning on a visual approach into PDX airport.

ACN: 1297396 (1669 of 2908)

Time / Day

Date : 201509
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1297396

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1297398

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We started and taxied out normally to [the runway]. At the time [the runway] was being used for both takeoffs and landings. We were cleared to line up and wait just after another A320 had landed. I (the First Officer) was the pilot flying. We were cleared for takeoff. The takeoff roll was normal but as soon as we broke ground we had a very strong rubber burning smell in the cockpit. The initial reaction was that the smell was from the tires of the preceding dispatch via ACARS to have him send us the landing data for [another runway]. However, the smell continued to get stronger and it was obviously not what we had originally thought. I immediately donned my O2 mask and turned the A/P (Autopilot) on. Shortly thereafter we got an ECAM saying that a CB had popped on the overhead panel. The captain contacted dispatch and [maintenance control] while I coordinated with departure for a return to the field. While coordinating with dispatch the captain also ran the fumes elimination checklist while I set up for the downwind and approach. We also contacted Dispatch via ACARS to have them send us the landing data for landing. We never got an answer from him nor did we receive the landing weights data. Even though the CB (Circuit Breaker) had popped the odor continued to be very strong throughout the whole aircraft. We felt it was more important to get the aircraft on the ground as we had a potential electrical fire than it was to wait for landing weights to be sent to us. We were under maximum landing weight. We were given traffic priority and made a short approach to with the fire rescue crews standing by. After landing we cleared the runway and switched over to the fire rescue frequency where we talked to the chief. He reported no visible smoke from the aircraft so we elected to return to the gate and have the fire crews follow us. When we approached the gate a guide man and gate agent were already standing by. The gate agent reported that when she opened door 1L she could smell a very strong odor. We deplaned the passengers without delay. We ran the parking checklist then went outside to talk to the fire crew. Maintenance was already at the plane and had opened the EE bay doors for them. The odor was also very noticeable from outside the aircraft. We debriefed maintenance and left the plane.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 flight crew reported they returned to departure airport after smelling fumes and receiving a warning message advising of a popped circuit breaker.

ACN: 1294384 (1670 of 2908)

Time / Day

Date : 201509
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : IMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737 Next Generation Undifferentiated
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 24
ASRS Report Number.Accession Number : 1294384
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 1
ASRS Report Number.Accession Number : 1294145
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

On a daily occurrence [adjacent ARTCC sector] hands us two or more aircraft that will lose separation within ten to fifteen miles within our airspace. Management is well aware of this and a lot of times when it's brought to their attention they shrug it off and snicker at it. There was a small area of precipitation just west of VOR ZZZ. Aircraft were deviating around it. [Adjacent ARTCC Sector] handed us two aircraft on a collision course at 35,000 feet. The aircraft were to lose separation within 10 miles of our airspace. Both aircraft were deviating for the weather. The 4th line in Aircraft Y's data block read [deviate left, then direct Airport ZZZ] and Aircraft Y's read [deviate left, then direct Airport ZZZ1]. Either aircraft could turn back on course anytime they wanted without notification. Aircraft X could have turned right into Aircraft Y! There was no positive separation insured by [adjacent ARTCC Sector] controller! Noticing this the Radar Assist immediately called [adjacent ARTCC Sector] controller to assign present headings to both aircraft. [Adjacent ARTCC Sector] controller replied that he already shipped the aircraft! This is unbelievable! The transferring controller is to insure that all conflicts are resolved before transferring communication! This was not done! These aircraft could have collided within five miles of the boundary! This is blatant carelessness of the [adjacent ARTCC Sector] controller!

In my opinion [the adjacent ARTCC Sector] takes separating aircraft a low priority. They have become lazy and complacent and is getting worse over the years. They are handing us dangerous situations to deal with on a daily basis! Unfortunately, it's going to take a serious accident or a collision of two airliners before this gets resolved!

Narrative: 2

I was the Radar controller on Sector XX. [Adjacent ARTCC Sector] hands off 2 aircraft both at 35,000 feet and both deviating left for weather. The aircraft were on a merging course heading into my sector airspace. In the 4th line of the data block it showed both aircraft were cleared to deviate left then direct to a fix when done deviating. If either aircraft had gone back on course there would have been a loss of separation on the [ARTCC/ARTCC] boundary. [Adjacent ARTCC Sector] did not ensure separation of the two aircraft. We had to reach out to [the adjacent ARTCC Sector] to get control of the two aircraft which were still in [the adjacent ARTCC Sector's] airspace to fix the situation. Both aircraft were shipped to my sector before the Radar Assist could get a hold of [the adjacent ARTCC Sector] to ensure the separation.

Synopsis

ARTCC Sector handed off two aircraft deviating for weather to adjacent ARTCC Sector. The two aircraft had clearance to return on course at their discretion which would have put them in confliction with each other. The reporter states it is a common occurrence to receive aircraft from this facility which will conflict with each other soon after entering their airspace.

ACN: 1291916 (1671 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 900 (CRJ900)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 0.5
ASRS Report Number.Accession Number : 1291916
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 16.9
ASRS Report Number.Accession Number : 1291065
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1292107
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 4

Reference : 4
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1292106
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 5

Reference : 5
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1291316
Human Factors : Time Pressure
Human Factors : Confusion

Person : 6

Reference : 6
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1291319
Human Factors : Time Pressure
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft Y was handed off to me from the low sector climbing to FL230. I climbed Aircraft Y to his requested altitude of FL360. I left him on his filed route. I initiated the handoff and subsequently transferred radio communications after receipt of the handoff. I accepted the handoff on Aircraft X around the same time. Aircraft X checked in at FL370 deviating right of course. I believe I issued the weather in the vicinity of ZZZ. I noticed that the other sector ha[d] authorized Aircraft Y to deviate for ZZZ1 Airport. I did not yet apply merging target procedures as the aircraft targets did not appear to be merging even though they were deviating. As soon as I noticed Aircraft Y Leave FL360 for FL363 I issued the traffic to Aircraft X. I was informed by Aircraft X that he was already in a left turn do to an RA he had received. After the aircraft passed each other, Aircraft X returned to his normal course and continued to deviate. The pilot of Aircraft X queried me as to what had happened. I told him a CRJ900 (no numbers given) had exceeded his assigned altitude. The pilot then asked me for Aircraft Y numbers. I told him I couldn't give that over the radio. He asked for and was provided a phone number to call the facility. 

I don't [know] why it happened. Aircraft Y read back his altitude assignment of FL360. I was told he checked in with Sector XX climbing to FL360. Either the pilot was responding to a TCAS climb, he just made a mistake and kept climbing on his own or he took someone else's climb. The TCAS climb doesn't make sense though as there was no aircraft below him in his vicinity.

Narrative: 2

Aircraft X was level at FL370 and had been switched to [next] sector. Aircraft Y was climbing to assigned altitude of 360 and was cleared to deviate right of course when able ZZZ1. At XA:54 I observed Aircraft Y mode-C showing FL363 with Aircraft X at 12:30 [position] and about 5.5 miles opposite direction slightly to the right. Because he was right under a B737 (at 390) I thought it was a mode-c swap. I asked Aircraft Y to verify he was level at FL360. There was no response. The next hit showed a mode-C of FL367 and Aircraft X was at 1 o-clock and about 3 miles. I again asked Aircraft Y to verify he was level at FL360. Again there was no response. The next hit was at FL369 with Aircraft X at 3 o-clock and less than 2 miles. When Aircraft Y got back to FL365 I again called the aircraft. This time as the plane was leveling at FL360 the pilot responded that they "were trying to avoid an RA." When questioned the pilot said that they had gotten a descent RA at about FL357 and then got a climb RA and that was why they went above FL360.

Narrative: 3

We were at FL370 on a heading deviating for weather approximately 20-30 miles in front of us at our 11 o'clock position. We were in IMC conditions at the time of the event. We received a TA for an aircraft traveling the opposite climbing at our 1 o'clock position. We lowered the scale on the ND and as we did this the target went to an RA with a climb command. I disconnected the AP and started a turn to the left (away from the traffic) and also started a climb. The conflict traffic was given a clearance to 360 but climbed to 370. We advised ATC that we were responding to an RA and that we were turning and climbing. When we were clear of the conflict we returned to our altitude and continued to deviate around the WX. We passed within approximately 2 miles from the other traffic with minimal vertical separation.

We had just changed frequencies as did the conflict aircraft so there were two different controllers involved. We would have enjoyed listening to the other frequency and the conversation that occurred with the other aircraft.

We requested a phone number for the ATC supervisor. I called upon landing. The supervisor was very professional and very concerned about the event. They had already done a quick look at the radar tape and talked to the other flight crew. The other flight crew claimed they received an RA as they approach their assigned altitude from an aircraft below. The supervisor stated we were the only two planes close to each other. We noticed that the TCAS showed them level at FL370 as they went past. One could make the assumption that the other aircraft set the wrong altitude in their automation. 

The FO and I had a conversation about the short amount of time that this escalated from a TA, which we get on a regular basis even at FL370, to an RA which we don't see that often at cruise. In our debrief we talked about appropriate and proper timely responses to an RA at that altitude. A much more delicate touch and response required at that altitude compared to an event say on approach. 

Another interesting note. As I talked to the supervisor, he asked us during our RA what direction we were given to turn from the plane. I explained that we only get vertical guidance and nothing laterally. Might be an education opportunity for the controllers. Lots of new ones out there.

Narrative: 4

Cruising @ FL370 on a heading for WX avoidance. IMC. Received TCAS TA turning to "climb" RA for traffic at our 1 o'clock position. PF disconnected AP and began turning climb to left for traffic avoidance. It appears that the RA traffic was cleared to FL360 but had climbed to FL370. Advised ATC of our actions. Safely clear of traffic we returned to the desired WX avoidance heading and our assigned altitude of FL370. Deviation from assigned altitude was minimal (<300 feet).

Other aircraft could stay at their assigned altitudes...for starters. A post event brief between CA and FO to discuss pros and cons of reaction technique is helpful for future similar events. ATC should be made aware of the functionality and limitations of the TCAS system, more specifically, that we are given vertical guidance only, not lateral guidance as the air traffic controller in this event assumed.

Narrative: 5

Event occurred while climbing to cruise altitude around 35500 ft for 36000 while in IMC. At this time an RA occurred indicating an I immediate descent to avoid traffic. We reduced power and initiated a descent. After the nose was lowered a RA changed indicating a climb. We changed power and pitch to follow the resolution. The aircraft climbed through FL360 to about 36500 and resolution was cleared. Then correct airspeed and altitude to assigned. ATC was notified that we were responding to an RA and we were leaving 36000. No passengers or crew reported injury. No Aircraft limitation exceeded.

Traffic did not show up on TCAS until the RA occurred and no proper warning was given by ATC. At this we can think of [no] further corrective action.

Narrative: 6

[Report narrative contained no additional information.]

Synopsis

With twelve o'clock traffic an A320 at FL370, a CRJ900 climbing to FL360 responded to a TCAS RA "DESCEND" annunciation which changed to "CLIMB" as the descent began. The A320 responded to the TCAS RA from the climbing CRJ900. The CRJ900 TCAS did not display the A320 until the RA activated.

ACN: 1291277 (1672 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 15000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1291277

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of FL150, we felt a bump followed by erratic engine 1 indications. ECAM showed ENG 1 SYSTEM fault, and ECAM ENG 1 FADEC B fault. We had to bring ENG 1 to idle stabilize the engine. Returned to [departure airport] and made a normal overweight landing.

Synopsis

An A320 Captain reported problems with the #1 engine on climbout, requiring reduced thrust to idle to stabilize engine. The flight returned to departure airport.

ACN: 1290978 (1673 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Landing Gear Indicating System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1290978
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Lead Technician : 8
Experience.Maintenance.Technician : 13
ASRS Report Number.Accession Number : 1291033
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

A320 Aircraft has a recurring landing gear fault. This time the fault occurred in ZZZ1 and was made a Deferred Maintenance Item (DMI) in ZZZ1. Aircraft ferried to ZZZ with the landing gear lock down. Confirmed the aircraft was in the non-MEL configuration before starting work for the L/G not unlocked ECAM Message. The right landing gear retract actuator was ultimately removed and replaced. After all the operation checks were completed and all cockpit indications were normal, cleared DMI for the Landing Gear Indication System. The aircraft took off from ZZZ and returned to the field for FLT L/G Down indication after gear retraction. Aircraft restricted & advised to 235 knots ECAM message per flight crew and log book entry.

After removing and replacing flight controls, landing gear components etc.. after inconvenient chronic or recurring faults, no Test flights are required by our Air Carrier. Inadvertently causing major anxiety and inconveniencing our paying passengers. Prior experience with other operators, a Test Flight would be required before the aircraft was returned to revenue flight.

Narrative: 2

Aircraft had multiple crew reports for the Right Main Landing Gear (MLG) not uplocked ECAM message upon gear retraction. The fault was worked in ZZZ2 and made a few flights, then the fault returned while the aircraft was in ZZZ1. The Landing Gear Indication System was deferred per the MEL by Maintenance team in ZZZ1. Request was made to the company to send the aircraft to our Maintenance facility in ZZZ3 for this repeat problem. They have the proper facility and tooling. Aircraft was Maintenance Ferried to ZZZ with the gear lock down only because we had jacks. No facility and no tools for the job. We work in a Line environment on an un-level ramp and had to jack the aircraft high enough to preform gear swings and trouble shoot the Landing Gear Retract System in an AOG situation on the flight line. 

Company sent a retract actuator, but the paperwork was incomplete. By the Second night we received loaner tooling from another Operator and a replacement actuator from Airbus which had paperwork issues as well. Late into the shift and after working multiple other aircraft, the paperwork issues were resolved and the actuator was released to maintenance to install. The right main landing gear retract actuator was removed and replace per the Trouble Shooting Manual (TSM) and Aircraft Maintenance Manual (AMM). All operational and leak checks and cockpit indications were normal with the aircraft on jacks. The DMI was cleared by the replacement of the retract actuator. The aircraft took off from ZZZ on a revenue flight and the cockpit crew reported an ECAM Advisory Message FLT L/G DOWN indication after retraction and restricted advised to 235 knots. Crew returned to the field. Upon further trouble shooting found the L/G DOWN VMO/MMO selection switch (22FP) on the 188VU panel in the aft E&E bay was inadvertently left in the L/G down position. Reset L/G DOWN VMO/MMO selection switch (22FP) to the L/G normal position. Messages cleared. Due to time constrains and pressure to return the AOG aircraft to service the (22FP) Switch was overlooked when clearing the DMI.

Synopsis

Two Aircraft Maintenance Technicians reported recurring Right Main Landing Gear (MLG) L/G Not UNLOCKED ECAM faults on an A320 aircraft. Aircraft ferried to a Line Maintenance Station with limited resources. Right MLG Retract actuator replaced. No Flight Test required. Aircraft departed but returned to field with ECAM Advisory message FLT L/G DOWN. Found the L/G DOWN VMO (Maximum Operating Speed Limit)/MMO (Maximum Mach Operating Speed) selection switch (22FP) on the 188VU panel inadvertently left in the L/G down position. Reset switch to L/G 'Normal' position.

ACN: 1289655 (1674 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1289655
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1289659
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : Taxi
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

After clearing runway and taxing to the gate. We were told to hold short of Taxiway N and to contact ramp. We started the APU and followed ATC instructions. We contacted ramp and was told to hold short for traffic in the alley. Captain told me to shut down engine 2 and turn on the APU bleed to help with cooling. Right after I turned on the bleed we had a dirty wet sock smell in the cockpit and the APU automatically shut down. Was running the ECAM procedure when we got a call from the aft Flight Attendant (FA). She said there was a smell and haze in the cabin. I told her what happened and said to keep us updated. She asked if I could make an announcement. I made an announcement and told the Captain of the situation. At that point I called back and asked if the haze was dissipating which it was, at the same time we got cleared to the gate. Got to the gate without further problems. Wrote up in maintenance log and waited for maintenance. Maintenance did not show up right away, so left the plane.

When I came the back, crew was taking their bags off. So I got my bags and went up to gate area and found the Captain. Captain said the mechanic told him that the APU blew a seal and what we were smelling was oil. I told him I was feeling nauseous. We decided to go to the ER as a crew. They did a blood test and put us on oxygen. Test came back negative and were released. Have had a headache for last two days.

Narrative: 2

In the cockpit we smelled the (dirty sock) smell and believed it was from a temporary overheat event. After deplaning, and speaking with maintenance, then it came to my attention it was from a very toxic oil vapor issue from the APU.

Train crew members how to recognize oil leaks in the bleed air system.

Note: If this happen inflight from an engine, add affected engines bleed switch off to stop the source, to our smoke and Fume procedure. Why would the company want to keep pumping in the oil vapor into the cabin?

Synopsis

After landing the APU was started, but auto shutdown accompanied by a dirty socks odor and cabin haze. At the gate Maintenance determined an APU oil seal failed allowing oil in APU bleed air. The aircraft was removed from service. The First Officer was examined at an ER and released with a persistent headache.

ACN: 1288134 (1675 of 2908)

Time / Day

Date : 201508

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class A : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1288134

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After departure, following gear retraction, we noticed a louder than normal airflow-like noise in the cockpit, accompanied by a very light rumbling. We looked over the forward panel and upper and lower ECAM . Everything looked normal. We also looked periodically through the ECAM system pages. Nothing out of the ordinary. Contacted dispatch via ACARS text, and wrote that we had a slight rumbling along with some noise. Nothing indicative of a problem. As we neared [an] intersection, we were told by controller to contact [Commercial Radio]. Shortly thereafter, we lost our vhf and ACARS, however we were aware that we could use HF to get info to our dispatcher. Prior to coasting out, I told the dispatcher that there weren't any indications of any problems. We were around 2 1/2 hours late, and felt that the ground crew had left an access panel open. We concurred with dispatch and MOC that if no problem exists, we can fly at .76/FL320 (conflicting traffic at our planned cruise of FL360). We also noted that our projected fuel at [the intersection] was within 300 pounds. We incurred a long sit on the ground, so this slightly excess burn seemed plausible. The last message we received before coasting out said to the effect if you feel comfortable at .76/FL320 that is ok, if any doubts, then we would have to establish a new plan.

At top of climb, I conducted ECAM system checks, paging through the system pages and noted nothing unusual. I kept a fuel log, and noted that we were within about 500 lbs of planned burn during the early part of the flight. This increased to 700 later over [another fix], but I think we were in radar contact I believe by that time.

About halfway through the flight, I asked the Flight Attendants (FA) if they heard anything or felt anything unusual. They said that they did not. I told them that we had a lot of noise and a slight rumbling up in the cockpit, but no other indications. I made my first logbook entry while still out of radio contact: "After Takeoff and at all times during flight, light rumbling and loud noise in cockpit. FWD Galley FA did not hear anything." We still believed the source to be an open panel at this time.

As we neared land, we began to catch up with the ACARS messages that were interrupted after our departure from ZZZZ. Dispatcher recommended I fill out a vibration report. I began to do that. I had conducted at least 2 other ECAM system flows and noted nothing irregular on either one of these. Dispatcher asked if we needed anything, and I said that we were just fine. About 200 miles from [Destination], and after several messages to dispatch, I conducted another ECAM systems check, feeling very confident it was only an open access panel. When I came to the WHEEL page, I noticed 4 amber x's above the nosegear. This was the first time we had seen anything abnormal. We slowed the aircraft down to .67M/280 KIAS, notified ATC. We entered the discrepancy in logbook.

ATC was aware of our situation, but we elected to treat it as a normal procedure unless the gear would not extend/lock. The gear came down with normal indications. We landed, taxied to the gate and everything was normal up to that point. At shutdown, we received the only ECAM message during the entire flight: L/G SYS DISAGREE. We entered that in logbook, and maintenance was waiting for us when the door opened.

Synopsis

A320 Captain reported noticing slight vibration after takeoff, and later in the flight received the ECAM message L/G SYS DISAGREE. Approach and landing were normal.

ACN: 1287698 (1676 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : STL.Airport
State Reference : MO
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : STL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : STL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1287698
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were set up and prepared for a visual approach backed up by the ILS. Everything was briefed prior to descent. The weather was clear and night VMC. The tower asked if we had the airport in sight around 15 miles out. After confirming with the captain that he had the airport in sight we were then cleared for the visual approach. We slowed and configured as normal outside the final approach fix. While working the radio and preparing the checklist I noticed the pilot flying descending below the final approach fix altitude a few miles prior. After a second or two I queried him about his altitude and mention he was low and mentioned the altitude at the final approach fix. We continued the descent at 1,000 feet per minute or so to an altitude of I believe around 1,000 feet AGL, while I was reviewing the approach plate, all the while being queried by the tower about our altitude. I made the call altitude again and said we were too low and we needed to level off and go around. At this point we received the GPWS "terrain" I called for a go around and we climbed to 3,000 feet, still outside the final approach fix. 

After it was confirmed the pilot flying had indeed Misinterpreted something other than the airport as the airport, and confirming with the tower and pilot flying everything was okay and we were still in a position to make a normal landing at this point, we decided to continue the landing. At this point again [we] became too low and slow on the approach and another go around was then called around 800 feet. We performed a complete go around. After reentering the pattern and autopilot re-engaged, we discussed again that the pilot flying was Indeed aware of airport and runway position, we flew the localizer and glideslope back in and a normal landing was made.

The fact the pilot flying Did NOT have the right airport in sight, coupled with his disorientation of position, led to a descent below a safe altitude quickly and suddenly. I was not aware of his disorientation until it was too late. I was taken by surprise as the non flying pilot as I believed the pilot flying was in full contact with the airport and lining up for the runway. I was taken back by how abruptly the airplane was put in an inadvertent, low altitude position. The speed of things happening, led to things becoming unstable quickly. While I called go around the first time a full "TOGA" go around was not completed as should have been.

Synopsis

An A320 First Officer reports being cleared for a night visual approach to Runway 12L at STL with the Captain flying. The Captain begins descending below the FAF altitude well outside of it and is queried by the First Officer. The descent is continued until the Tower asks the flight to check their altitude and a GPWS warning is annunciated. The crew climbs to 3,000 feet and is still outside the FAF so a second visual approach is attempted with the same results. This time the missed approach is completed and vectors are received to the ILS.

ACN: 1286143 (1677 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MMSD.Airport
State Reference : FO
Altitude.MSL.Single Value : 3600

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : MMSD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1286143
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

En route to Cabo, was cleared ENEKO then our filed route direct to Cabo. The First Officer was flying and I was monitoring. On descent I received call from another company flight going to MMGL and said they had a mechanical and couldn't raise company and asked if I could contact them. I sent an ACARS to the company as we continued in. After switching to Cabo Approach was cleared to 5100 ft. We were still proceeding direct Cabo VOR. Then the Approach Controller cleared us for the VOR 2 runway 16 approach. I then received an ACARS from the company requesting [we] tell the other flight to turn back to ZZZ. I tried to call them but no answer. At that point Approach Control asked why we depended below 5100 ft. I ask the FO and he said they had cleared us for the approach and the altitude was 3600.

I checked and he was right and told Approach that he had cleared us for the VOR 2 approach and we had the airport in sight and could take a visual. He responded he did not clear us for the visual and we [had] descended below minimum vectoring altitude. I told him we had misunderstood what he wanted and at the time he cleared us to 2400 ft and then cleared us for the visual approach. After landing I discussed what happened with the FO. He said he had proceeded direct to the 18 mile IF on the VOR 2 approach while I was off the radio. I was distracted and realized we were not on an established segment of the approach and should have not accepted the approach as cleared. They usually clear us to MARUK and then the approach but at the time we were still going direct to Cabo. We were only a few degrees off the inbound and visual so the FO thought we could just intercept the inbound course and descend to that crossing altitude.

The main part on my behalf was the distraction of trying to help the other flight communicate with company so I was out of the loop. The FO should have said we were not on a part on the approach and I should have seen this too and never accepted it as cleared. If I had not been distracted I should have seen this and not accepted the approach as cleared.

As at all times I should have not let the company call distract me, and as at all times in in foreign country I should have verified the clearance he gave us.

Synopsis

A320 Captain reported the flying First Officer descended below cleared altitude on approach to MMSD after misunderstanding an ATC clearance.

ACN: 1286115 (1678 of 2908)

Time / Day

Date : 201508

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Other Documentation
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 6139
Experience.Flight Crew.Last 90 Days : 218
Experience.Flight Crew.Type : 6139
ASRS Report Number.Accession Number : 1286115
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Primary Problem : Human Factors

Narrative: 1

A rushed maintenance deferral resulted in the Maintenance Release (MR) being issued without the actual deferral items being accomplished, as defined in the MEL.

Arriving at the aircraft, the Captain found the APU had a fault light on the Overhead Panel with the switches indicating someone had tried to start the APU. (The aircraft had been at the gate all evening.) Maintenance was contacted and a write-up was completed. After an unsuccessful reset resulted in the Auto-Shut down of the APU, during another attempted start, Maintenance issued deferral of the APU.

Deferral was sent without anyone (Maintenance) coming to the aircraft to accomplish the required MEL items for APU INOPERATIVE. Specifically, 'Cockpit Breakers(s) Pulled and Safetied, (L41 and L42)'. The MR was issued stating both had been accomplished, when neither had been. NO ONE showed up and the MR was issued immediately stating everything was accomplished.

Deferral was initially refused by the Captain, because no one looked at the APU to determine why the APU would not start. Further, the pilot's paperwork did not reveal any APU maintenance history, while there was extensive Auto-Shut down history. 

The rushed maintenance deferral and the release of the flight without the MEL items being accomplished, were not detected by the crew until in flight.

Synopsis

Captain reports about a rushed APU deferral that resulted in the maintenance release of an A320 aircraft without the actual MEL maintenance deferral items being accomplished. MEL had required cockpit Circuit Breakers L41 and L42 to be "Pulled and Safetied." APU had an extensive auto-shutdown history.

ACN: 1285908 (1679 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 17000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use : Direct
Route In Use.SID : KARVR3 RNAV
Airspace.Class E : ZLA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1285908
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Miss Distance.Horizontal : 15000
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Being in contact with LA Center at 17,000 feet we were instructed to fly direct to TIJ (Tijuana) VOR. After abeam DAMUG fix we noticed an aircraft was flying direct to us 600 feet above and descending, we were instructed to climb to FL230 but at the same time we received the traffic aural warning and asked to monitor vertical speed, by the time we saw the plane it was like 2 NM and 200 feet above and the resolution [advisory] was to descend -2,200 feet/min. We followed the resolution [advisory] and almost immediately we were out of the conflict. We informed Center and were cleared to continue the climb.

The frequency was jammed, it wasn't easy for the controller to repeat the instruction immediately.

Synopsis

A320 Captain reported Los Angeles Center frequencies were jammed, making traffic management and awareness difficult.

ACN: 1285600 (1680 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 300

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1285600

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff we experienced what we initially thought was an engine failure but really ended up being a compressor stall. The #1 engine stalled multiple times and also over temp. We [advised Tower] and return for landing. The aircraft was overweight so we computed the required landing distance and made a soft landing. Emergency equipment followed us to the gate. Dispatch was notified and logbook entries were accomplished.

[Cause was] compressor stall of the #1 engine followed by an over temp.

Synopsis

A320 Captain reported returning to departure airport after #1 engine compressor stalled and then over temped.

ACN: 1284590 (1681 of 2908)

Time / Day

Date : 201508
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : BNDTO4
Airspace.Class B : IAH

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1284590
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Flight was filed BNDTO FOUR departure and ATC clearance was PITZZ THREE departure. Departure was changed in MCDU and briefed in MINI BRIEF during taxi, and submitted to ramp control upon initial call up. During taxi out, was given runway change and submitted into MCDU and DISCON cleared. Both PF and PM understood we were cleared and we're going to be flying the PITZZ three, but with short taxi, and no preceding aircraft, a verification of route after runway change was not completed. Upon initial contact with departure control, the controller noted our heading and asked what departure we were given. We replied the PITZZ three, which was what both of us had out in our Jeppesen [charts]. The aircraft was flying the BNDTO FOUR and the fixes in the MCDU were for the BNDTO FOUR. We complied with a heading change, and I, as PM, selected the initial fix in PITZZ and reported to ATC that we have PITZZ on our screen, and reported the approximate heading. We were told to fly direct the subsequent fix MNNKE, which I entered into direct and the aircraft navigated to MNNKE. The remaining two fixes were common between the two departures and we continued without further conversation or incident. Verify departure fixes after all changes to runways and departure procedures.

Synopsis

A320 First Officer reports entering the BNDTO4 in the MCDU departing IAH, then receiving a revised PDC for the PITZZ3, which is entered. During taxi a runway change is issued and entered but the route is not checked prior to takeoff. Once airborne ATC questions the crew about their departure and they find they are flying the BNDTO4.

ACN: 1284543 (1682 of 2908)

Time / Day

Date : 201508

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Attitude Indicator(Gyro/Horizon/ADI)
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Component : 2

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Manufacturer : Airbus
Aircraft Reference : X
Problem : Design

Component : 3

Aircraft Component : Electrical Distribution Busbar
Manufacturer : Airbus
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 162
Experience.Flight Crew.Type : 12688
ASRS Report Number.Accession Number : 1284543
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

On climbout the Standby Attitude Indicator (SAI) started making a grinding noise and became very unstable/shaky. As time went on, it became very loud and shaking worse. It had become very distracting both visually and noise wise. It was VFR to destination so I sent a message to Maintenance Control to see what they thought about pulling the overhead panel Circuit Breaker (CB) labeled NAV Stby Horizon at location F12. They said if I desired to do so, I would have to use my Emergency Authority. The First Officer and I both felt the instrument was a major distraction and useless like it was, so I elected to pull the CB. To our suprise, this action did NOT remove power from the SAI! So I left the breaker out and upon touchdown the flag dropped into the face of the SAI and it subsequently spooled down. We could find nothing in the flight manual that explained this. Upon arrival after accomplishing the checklists, I called and visited with Maintenance Control about this. It appears there is a CB in the E/E compartment labled STBT INST/BATT that controls a relay that latches the SAI to the Hot Batt Bus when the normal power source is removed. This has to be what happened in this case. I am very well aware of how important the SAI is, but I find it hard to believe we cannot remove power in a case like this. It was so bad, that I was wondering if the instrument could get hot enough to be a fire hazard.

Callback: 1

Reporter stated that on touchdown the SAI went dead, apparently due to the squat switch indicating the A320 aircraft was on the ground and now conditions were OK to remove (shut-down) electrical power from the Hot Batt Bus to the SAI. But the inability to remove all power from the SAI under conditions he describes is still a concern; especially since the SAI got so hot. He has not had any response from his Air Carrier about his report.

Synopsis

Pilot reported the Standby Attitude Indicator (SAI) in an A320 making a grinding noise and becoming very hot, unstable/shaky on climbout. Pilot pulled the Overhead panel Circuit Breaker (CB) labeled NAV Stby Horizon at location F12, but that action DID NOT remove electrical power from SAI. Reporter stated another CB in E/E compartment labeled STBT INST/BATT controls a relay that latches the SAI to the Hot Batt bus when the normal power source is removed.

ACN: 1284294 (1683 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Cloudy
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Strobe Light
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1284294
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Physiological - Other
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

As previously reported, the strobes on these aircraft are positioned such that they flash brightly into the cockpit, causing pilot vertigo. As such, the pilot is forced to turn off the strobes for the entire flight to avoid vertigo, thereby creating more of a potential collision risk due to the strobes being turned off. Go fly one of these things at night with the strobes on.

Synopsis

An A320 pilot reported the sharklet equipped aircraft strobe lights are positioned such that in flight at night and in clouds they are blinding in the cockpit and cause vertigo. The reporter turned the strobes off.

ACN: 1283659 (1684 of 2908)

Time / Day

Date : 201508

Place

Locale Reference.Airport : BIL.Airport
State Reference : MT
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : BIL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class C : BIL

Component

Aircraft Component : Speedbrake/Spoiler
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 128
Experience.Flight Crew.Type : 5622
ASRS Report Number.Accession Number : 1283659
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Primary Problem : Aircraft

Narrative: 1

On approach, but prior to landing, the FO requested gear down and flaps full. I selected the requested configuration requests and the aircraft began to shake. I recycled the brakes and brought the flaps back to position 3. I stowed and rearmed the speed brakes prior to selecting gear down and flaps full. Again the aircraft began to shake pretty violently. A firm but safe landing was accomplished. We taxied to the gate and I contacted maintenance and advised them of what just occurred. I also briefed the receiving crew. Upon investigation by local maintenance, it was discovered that we landed with speed brakes 1 and 3 deployed. They were supposed to be locked down prior to leaving the hangar in Denver (they were deferred). I spoke to [maintenance] after learning what happened and was told that the mechanic left the spoilers in Maintenance mode. They were not set for flight. As such, they deployed when they were supposed to be locked down.

Synopsis

A320 Captain reported noticing vibration in flight when flaps were extended that was found to be speed brake panels deployed in flight because they were not locked down in accordance with the MEL.

ACN: 1282541 (1685 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 1801-2400

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1282541
Human Factors : Other / Unknown

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Police / Security Involved

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Received a fire/smoke ECAM for the Lavatories. It went off twice. There was a male passenger in the aft lavatory smoking. I asked for police to meet the flight. Made male passenger get off the airplane upon arrival to talk to police and customer service agent. Because the ECAM warning was causing the flight attendant call bells to ring constantly, I was not able to communicate with the lead flight attendant through the interphone. I had to get up and open the door to ask what was going on and if we had an actual fire.

I have had several smoking incidents over the years and no one seems to care that a passenger has caused an issue on the plane and has created a condition that jeopardizes the safety of the aircraft by creating excessive work load for us in the cockpit, and in the back the flight attendants are distracted not knowing if it is a greater issue of fire or a terrorist trying to do harm. The passenger just stands on the jetway while someone scolds them and then they walk away without any consequences. Why do we not have a set procedure to deal with this ongoing problem? I don't see much guidance in the FOM.

Synopsis

A320 Captain reported passenger was smoking in lavatory and activated the fire/smoke ECAM warning. Reporter stated dissatisfaction with the response to the incident.

ACN: 1281869 (1686 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Cabin Lighting : Medium

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1281869

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Police / Security Involved

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

These two people boarded aircraft and sat in row 29 def side with no one else in the row. Mr. X came to back galley to ask for water to take medication. He was wearing sunglasses and a hat therefore I could not see his eyes. I gave him a cup of water and he went to sit down. We takeoff and start beverage service. They are the first to order as they are in the very back. He orders two wines each for them, but he orders two red and two white. We did not think this was unreasonable as this is a four hour flight. We give him the wines, then he says oh could we just have all white. Fine we say and I have to make a note for inventory purposes. At this point I think they are both a little off but maybe they are just weird. As I have to walk back to galley to get something for another passenger I noticed him take something out of his backpack and pop it into his mouth. On our way back to galley F/A (Flight Attendant) B notices a can on their tray table and asks what it is. He tells her it's beer and then she informs him that is not allowed per regulations.

Flight continues and he has his face in her lap with a blanket over him. Both F/A A and B tell him to sit up. Ms. Y, the female whose lap he is in says we are just resting. But then takes a few minutes to get adjusted with her clothing. At this point we all agree they will get no more to drinks from us. Then while I am passing out immigration forms he says can I ask a question? Sure I say, he says can I get another wine I wanted to propose to my girlfriend and put the ring in it. I looked straight at him and said no. They continue to act very odd as though they are either extremely drunk of on some medication or both. Very antsy, dropping things that type of behavior. Then she goes to the lavatory in the back AC left. She locks herself in and I wait about five minutes while she keeps trying to open the door. When she does I am right there and the smell of smoke is unmistakable. I immediately ask for her name and call her to me. I then get gloves to prepare to go through the trash. I ask her where she put the cigarette. I had to ask her 3 times before she says in the toilet. I then tell her it's a federal law that you cannot smoke in an airplane and instruct her to get her passport and sit down. 

He then approaches me to ask if I couldn't just forget it happened. I again had to tell him No. He continued to ask if I had a heart at all. My response was I do have a heart for all the passengers on board this AC that her ignorant actions could have killed by causing a fire on board, mine included. I took her passport and green card and had the lead hold them. So at this point authorities were to be called as their behavior was clearly escalating and who knew what was going to happen next. For the remainder of the flight Mr. X took video and pictures of us without consent. I repeatedly requested that he delete them and he actively refused. 

A Sheriff and CBP (Customs and Border Protection) came on board and put them in the back galley to question them. They were both very slow to move and non-compliant at first. But they soon changed their tunes and started to just lie about everything. Of course the authorities couldn't really do anything unless they found something else on them. They did however make them delete what photos or videos but with the iCloud who knows if they really are deleted.

Synopsis

A320 Flight Attendant describes passenger misconduct on an international flight.

ACN: 1281177 (1687 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6255
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4226
ASRS Report Number.Accession Number : 1281177
Human Factors : Confusion
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Airport

Narrative: 1

ORD hold short: was given a clearance to hold short of taxiway G on T followed by a clearance to hold short of the approach path for Runway 9. As we approached G we were asked by ground to hold short of G and that we'd missed the Runway 9 approach point. We never saw any markings on the taxiway or the sides. ORD needs to improve their signage for that point.

Synopsis

Flight crew Captain, southbound on Tango at ORD is told to hold short of Golf which is missed, then to hold short of the approach path for Runway 9R which is also missed. Lack of signage is cited as a factor.

ACN: 1281161 (1688 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 15000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : DRLLR4
Airspace.Class E : I90

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : B777 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Descent
Flight Phase : Initial Approach
Route In Use.STAR : DRLLR4

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 192
Experience.Flight Crew.Type : 6566
ASRS Report Number.Accession Number : 1281161

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22000
Experience.Flight Crew.Last 90 Days : 162
Experience.Flight Crew.Type : 4597
ASRS Report Number.Accession Number : 1281154

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I was the Pilot Monitoring to IAH. During our descent to IAH, we were vectored east off the MSCOT4 arrival into IAH for traffic separation. Houston Approach advised us to expect the DRLLR4 arrival now and also advised we would be following a heavy B777 which was following a Super A380. We were given direct to MPORT and cleared to descend via DRLLR4 after MPORT. We were assigned 280 KTS and were told to expect 250 KTS after MPORT. TCAS display had target at about 8 NM precisely on arrival ND green line in front of us. I looked at our wind and noticed calm winds at FL180. I then stated something to the effect that this wasn't going to be good for wake turbulence.

Somewhere between MPORT and DRLLR around 15000 FT MSL and prepared for an encounter, we hit the worst wake turbulence I have ever experienced! With autopilot on in NAV and DES mode the aircraft rolled toward 35 degrees left very quickly. At 30 degrees AOB (Angle of Bank), without thinking, I immediately turned off the autopilot and thought I said I have the controls (not sure) and rolled the aircraft back towards wings level. We then got a "DUAL INPUT" call and Captain stated he had the controls. We leveled off and called Houston Approach with PIREP. Approach immediately assisted us with vectoring us off course and had us level off at 14000 FT MSL. The aircraft behind us also quickly complained about wake turbulence very soon after our encounter. After additional separation from the Heavys, Approach cleared us to descend and we received a different runway assignment to avoid additional encounters.

I called Houston Approach and asked that they review the tapes and see if there were and separation/spacing issues and suggested they save the tape for data collection on significant wake turbulence encounters. During our conversation I learned that we trailed the Heavy 777 by 6 miles when we entered its wake. The precision of the GPS and RNAV arrivals/departures combined with standard 3 degree descent profiles are now creating a greater risk of wake turbulence encounters. Add to that a light wind summer day aloft and we have the formula for a serious incident.

I would also like for the [company] to review our data for collection and training. Our flight profile could be used for a [training] event and I believe my reaction could have made the situation worse if I hadn't had a very similar wake turbulence episode with "Dual Input" as the pilot flying on takeoff. During that event I had proper full stick right during the recovery and the IOE Captain reacted like I just did but didn't stop his input when "Dual Input" was heard and we rolled all the way over to 35 degrees to the other side before we recovered. Having flown the A320 for over 13 years, not knowing what the other pilot is doing with the stick input can be a serious deficiency when attempting unusual attitude recovery. We need to re-emphasize unusual attitude recovery with auto-pilot on and wake encounters.

Narrative: 2

On the arrival into IAH (DRILLR RNAV ARR) we were advised by ATC that we were following an A-380 heavy. Descending thru 15,000 feet A/P engaged, the aircraft entered into slow progressive 35 degree left bank. I disengaged the A/P and flew the airplane manually. Advised ATC what had happen and we were given a different altitude and heading, re engaged the A/P continued on the arrival. Rest of flight was uneventful.

Synopsis

A320 flight crew reported encountering severe wake turbulence 6 miles in trail of a B777 on arrival into IAH.

ACN: 1281144 (1689 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach

Component

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 319
ASRS Report Number.Accession Number : 1281144
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 81
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1281182

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Approaching the final fix, when the gear were extended at 160 KIAS and flaps 3 there was a red UNLOCK indication above the green triangle for the left main landing gear, the ECAM L/G LGCIU (Landing Gear Control Interface Unit) 1 FAULT illuminated, and there were flashing XXs for the left main gear on the SD (System Display) wheel page. Indications for the nose gear and right main landing gear were normal. Went around and wanted to do a left turn to avoid terrain but were continually directed to do a right turn and climb so [advised] tower. WX was VMC so turned right and received vectors back around for the ILS 05R due to our low fuel state (6K pounds). Accomplished the checklist for the ECAM. Received word from Tower that aircraft on the ground confirmed we had 3 landing gear down. Landing was uneventful, taxied clear and to the gate.

Narrative: 2

Approaching FAF, 160 kts, flaps 3, put gear down for landing. Nose and Right main were normal but Left main showed both a green down arrow and a red UNLK. Also the wheels showed a flashing Left main. LGCIU1 (Landing Gear Control Interface Unit) fault ECAM also a landing gear disagree ECAM.

Aborted Landing/went around. Controller wanted us to do a right 340 degree turn (mountains right and behind us). Had to comply with less than desirable turn due to language barrier. Worked on checklist for LGCIU1 fault and unsafe landing gear. Fuel was short and based on Tower's assessment of all of our gear being down, we cut off checklists and landed uneventfully.

Synopsis

On approach an A320 left main landing gear indication was a green down, red UNLK and an ECAM LGCIU (Landing Gear Control Interface Unit) 1 FAULT so the Captain executed a go-around. The QRH was followed and safe landing completed.

ACN: 1279787 (1690 of 2908)

Time / Day

Date : 201507

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1279787
Human Factors : Fatigue
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected.Other

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

After a two day trip, I received a call from the crew desk advising me I had a one day trip the next morning with a 10 hour 45 minutes block to block separation. I asked if I was the only reserve available and they said yes. I received just enough rest to fly the trip. While walking to my car after the flight, I received a call from the crew desk giving me a 13 hour callout [for the next morning], field standby. I informed them I was tired and got just enough rest to fly the Mexico turn after a 10:45 block to block callout. They said the assignment was legal and I informed them if I didn't get a good night sleep I would be calling in fatigued. After driving home my body was fatigued and out of sync. It took me a while to fall asleep and when I did, the sleep shallow and uneven. I have been a reserve captain for over eleven years and I've written several reports on the legality of changing a 2 am short call to an 'all-nighter,' and I'm equally concerned about flying 10:45 block to block trips and subsequently being assigned a minimum rest trip after your return to home base. I was assigned a 10:45 block to block trip last week and it was equally tough but it was also the last trip of the week. The Bottom Line--assigning a minimum rest trip, after a 10:45 block to block trip is exhausting. A 10:45 block to block assignment will work if it's the last trip of the week, but if an additional assignment is added on the body can't recuperate fast enough. 

I sincerely hope NASA and any appropriate agencies will test fly a trip with west coast pilots acclimated to central or eastern time zones, returning back to the west coast and receiving a 10:45 block to block assignment. Then subsequently assigning the pilot a minimum rest trip the next day. The minimum rest after a 10:45 block to block assignment should be extended.

Synopsis

A Reserve Captain discussed his company's pilot scheduling policy which includes minimum rest periods at home between duty periods after transcontinental time changes and alternating on duty times varying from night to day.

ACN: 1279501 (1691 of 2908)

Time / Day

Date : 201507

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : ORD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 990
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 146
ASRS Report Number.Accession Number : 1279501
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Received a reroute from clearance delivery after pushback from the gate. Controller read the full route phonetically, starting with PUMKN as the first fix. We inserted it into the [Flight Computer] as was read to us. We did not notice that the [flight route] started with the first fix being PMPKN. After departure were cleared direct PMPKN, and when the aircraft started the turn, the departure controller immediately gave us a vector to heading 060 and queried us about the spelling of the fix.

Synopsis

A320 Captain reported a track deviation when they navigated to PUMKN instead of PMPKN after takeoff from ORD.

ACN: 1278046 (1692 of 2908)

Time / Day

Date : 201507
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : MMZT.ARTCC
State Reference : FO
Relative Position.Angle.Radial : 358
Relative Position.Distance.Nautical Miles : 20
Altitude.MSL.Single Value : 6500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 25800
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 1278046
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were enroute to MMGL airport from the north and were given a descent from FL340 to 11,000 feet by ATC enroute controller (Mexico Center). As we approached 11,000 feet we were switched to MMGL approach control which gave us a further descent to 6,500 feet and vectors for the ILS approach to runway 10. The weather was VFR except for a thin layer of clouds above the terrain. As we descended below the clouds on an ATC vector we noticed that the altitude of 6,500 feet given by ATC was too low in order to maintain a safe margin from the terrain. We then leveled off at a safe altitude above the terrain, informed ATC of our level off, and also that the airport runway was in sight. ATC cleared us then for a visual approach to runway 10. We then maintained our altitude till intercepting the glide slope for runway 10 and made a normal landing. The ATC approach controller may have been new and did not consider the terrain or minimun vectoring altitude for the terrain area we were flying in.

Synopsis

A320 Captain reported the MMGL Approach Controller issued an inappropriate and unsafe altitude on approach to Runway 10. Flight crew stayed at a safe altitude and flew a visual approach.

ACN: 1274081 (1693 of 2908)

Time / Day

Date : 201506

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pitot/Static Ice System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1274081
Human Factors : Troubleshooting
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

At the start of the event, the first officer was the flying pilot, and autopilot 1 was engaged due to autopilot 2 being MEL'd. Earlier in the day the aircraft had Inertial Reference (IR) # 1 Air Data Inertial Reference Unit (ADIRU) deferred, along with several associated items, and the aircraft was refused by multiple crews ("Flight Crew refused legal aircraft"). However, just prior to our flight the ADIRU was signed off cleared from the MX log.

We were level at FL320 and had just received a clearance to climb to FL360 from Center. We were IMC and in light to moderate chop. We went to "thrust climb open climb" and initiated a climb. As the engines went to climb power we started a slow climb. I noticed the airspeed on my PFD begin to slowly increase. As we climbed, it appeared the aircraft began to accelerate past a normal climb speed of about Mach .78. Over the next several seconds I made a few calls to the First Officer (FO) to "watch the airspeed" which were acknowledged and corrections made. According to my airspeed tape the aircraft was now nearing MMO and the speed was still slowly accelerating. At about this moment, the FO and I realized there was a discrepancy in our indicated Mach. Due to the fact we were IMC, in turbulence, low vertical speed (we were heavy and engine anti ice was on), and that the trust was at climb power, It took us several seconds to decipher what speed indications were correct and what action we should take. An ECAM Caution "NAV - IAS Discrepancy" was alerted.

A few seconds later, the high speed Continuous Repetitive Chime (CRC) warning sounded, Master Warning flashed, and red ECAM appeared. At this time either the high speed protection logic or the first officer disconnected the autopilot and the first officer began manually flying the airplane. The first officer leveled the airplane and we immediately requested a decent on our Center frequency. We got no reply. The request was repeated and again no reply.

The first officer and I agreed to start a very shallow descent as we began to make sense of the situation. Both a lower altitude (because it widens the margin between high-speed and low-speed regimes) and a lower attitude (because it allows you to fly more "pitch & power") was the best option for us. Several more radio calls were immediately made to Center to advise of our climb clearance deviation and to notify them of our problem - still no reply from ATC.

The first officer and I agreed that the Captain's airspeed, the CRC over speed warning, and Master Warning were all related, but inaccurate, indications. We followed the ECAM and switched the Captain's Air Data to #3 on the Switching Panel.

Over the next 30 seconds or so, several transmissions on both the Center frequency and 121.5 were made to advise ATC that we had descended back down. We leveled back at FL320 as indications returned to normal and the continuous over-speed chime stopped. After another minute or two, we were finally able to reestablish communication with ATC and explain why we had descended and leveled back at our previously assigned altitude of FL320. Aircraft Operating Manual (AOM) procedure was followed and ADR 1 was turned off. The flight continued to our intended point of landing.

In debriefing the situation, I believe the first officer and I did an excellent job assessing the situation. An immediate reaction to a high airspeed indication, Master Warning, and CRC is to reduce thrust to idle and pitch the nose up to bleed excess speed. If the crew would have not recognized other indications by cross checking all flight instruments, it is likely we would have ended up in an Alpha Floor (low speed / stall recovery situation) which would have likely caused larger issues.

It is possible that the captain's Pitot Tube had also become blocked, essentially making my airspeed an altimeter where it indicating airspeed increased with a gain in altitude. This is how the flight instruments were acting. This same scenario had a fatal outcome for an A330 over the Atlantic.

Synopsis

A320 Captain reports erroneous airspeed indication during climb from FL320 to FL360 resulting in exceeding Vmo on the Captain's airspeed indicator, while the First Officer's airspeed indicated normal airspeed. The aircraft is returned to FL320 and ATC is advised.

ACN: 1273925 (1694 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 19000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1273925

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out at FL190 ECAM alerted us to the fact that our green hydraulic system had low volume, followed by low pressure. We followed SOP. Completed QRH and ECAM procedures. Contacted dispatch with issue who contacted [maintenance]. All agreed to proceed to [destination]. All degraded systems involved landing and taxiing the aircraft. Took vector to 20 mile final. Completed all QRH, ECAM, and normal ops check lists configured and landed. Cleared runway on high speed taxiway, with differential braking stopped aircraft and was towed to [gate].

Leak in green hydraulic system.

Synopsis

An A320 Captain reported loss of green hydraulic system during the climbout. The Flight continued to the destination.

ACN: 1273301 (1695 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : PDX.Tower
State Reference : OR
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : PDX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : PDX

Component

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 67
ASRS Report Number.Accession Number : 1273301

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After cleared for visual to Runway 28R at 2,500 feet, we began visual turn to final, and descended. We received a GPWS alert and although it seemed unlikely, the Captain correctly climbed per SOP. The alert stopped and we resumed the visual approach uneventfully. We reviewed the terrain after landing and saw no threats that should have set off the alert. It is worth noting that [another] arrival behind us experienced the same alert, which we heard them report on the tower frequency.

Synopsis

An A320 First Officer reported an apparent anomaly in the GPWS system when they got an invalid terrain alert on visual approach to PDX.

ACN: 1271446 (1696 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : DFW.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : DFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13197
Experience.Flight Crew.Last 90 Days : 174
ASRS Report Number.Accession Number : 1271446

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8445
Experience.Flight Crew.Last 90 Days : 214
ASRS Report Number.Accession Number : 1271448
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Taxi out, change of runway and ATC reroute... sent reroute and takeoff data to dispatch. Received takeoff data promptly. Inadvertently took off before new release came across ACARS printer. Minimum fuel declared to ATC landed with 5800 lbs at [destination].

Narrative: 2

[Report Narrative Contained No Additional Information.]

Synopsis

A320 flight crew failed to obtain a new/amended release and had to declare minimum fuel for landing.

ACN: 1271226 (1697 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Flight Phase : Initial Approach
Airspace.Class E : ZNY

Aircraft : 2

Reference : Y
Make Model Name : Small Aircraft
Flight Phase : Climb
Route In Use : None
Airspace.Class E : ZNY

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Air Traffic Control : Approach
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 1271226

Events

Anomaly.Conflict : NMAC
Detector.Person : Flight Crew
Miss Distance.Horizontal : 0
Miss Distance.Vertical : 300
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Narrative: 1

While on arrival, cleared to descend from 8000 FT to 7000 FT and slow to 210 KTS. Observed on TCAS the aircraft 10 miles ahead of us encounter traffic climbing to their altitude. A resolution advisory was declared by Aircraft Z with subsequent change in TCAS readouts by both aircraft. Transmissions by Aircraft Z asking clarification of positive control of the intruding Aircraft Y led me to believe it was a VFR aircraft attempting to fly very near the base of the clouds which we were flying in. Subsequently the intruder Aircraft Y (which had descended to 6000 FT after its near encounter with Aircraft Z), was seen approaching our flight on TCAS. At approximately 5 miles and closing, the intruder Aircraft Y display showed a climb from 6000 FT which would place Aircraft Y in the clouds which we were then flying through. FO queried ATC and requested a turn which ATC granted, a right turn to 090. I turned the heading knob to 090 and pulled, starting the right turn and immediately recognized a rapidly devolving situation. Pressing the autopilot instinctive disconnect button I steepened the bank to 45 degrees and began a climb while commanding "flight directors off." At about the same time a RA was alerted by TCAS, followed by the command "Monitor vertical speed." We climbed at approximately 2000-3000 FT per min for about 15-20 seconds before the TCAS reported "Clear of conflict.", at which time we leveled off (about 8000 FT), and notified ATC of our ability to resume the arrival. No further incidents occurred. The closest distance I saw displayed was 0 miles and 300 ft. Recognizing the possibility that the intruder Aircraft Y did not have TCAS or would not respond properly to a TCAS event, I chose to continue the turn while climbing, in order to gain "angles off" the intruder's heading. I believe this to have been the proper course of action. FO made telephone contact with the ATC controller on duty at the time of the incident and was told the Aircraft Y was flying VFR and the proper authorities had been notified to meet him upon arrival. A VFR pilot was attempting to penetrate IMC conditions in order to gain "VFR on top", without proper communications and subsequent authorization. In doing so he unwittingly flew his aircraft opposite direction to a major arrival route into one of the busiest airports in the nation. All VFR pilots flying in the vicinity of major airports and their arrival/departure corridors need to be made aware of the risks involved regarding separation of general aviation and commercial/business aircraft. All too often, a VFR pilot wanders into the path of a large aircraft and the events mentioned here happen all over again.

Synopsis

A320 crew experienced an NMAC while on initial descent for approach and landing. Crew experienced an RA after receiving evasive heading change from ATC. After clear of conflict crew continued the approach and landing.

ACN: 1271194 (1698 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZHU

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2300
ASRS Report Number.Accession Number : 1271194

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after reaching cruise at FL340 we had an ADR1 fault ECAM. We ran the checklist. Within a minute, the autopilot disengaged, the autothrottles disengaged, all FMA's disappeared, and the FD's disappeared. We lost all autoflight capability. I hand flew the airplane while the F/O ran the ECAMs. We coordinated with ATC and dispatch to divert into KIAH. I performed an uneventful overweight landing at IAH.

Synopsis

At cruise altitude, A320 flight crew experienced ADR1 fault with the loss of FMA's and Autopilot. The flight crew diverted.

ACN: 1270950 (1699 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAS
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Phase : Landing
Flight Phase : Final Approach
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1270950
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Tower had us line up and wait on runway 25R. After short wait, we were cleared for takeoff. I increased thrust for takeoff and began roll. Tower than issued this instruction "cancel take off clearance, inbound traffic to intersecting runway". I retarted thrust to idle and stopped. Approximately 1-2 minutes later we were cleared for takeoff. Take off was made uneventfully.

Synopsis

After receiving takeoff clearance, A320 crew advanced thrust levers and commenced takeoff roll. Tower then cancelled their takeoff clearance. Crew stopped aircraft and 1-2 minutes later received a second takeoff clearance.

ACN: 1270764 (1700 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Relative Position.Distance.Nautical Miles : 6
Altitude.AGL.Single Value : 1800

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Route In Use.STAR : PEEKK3
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Airspace.Class B : DEN

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 7485
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 3720
ASRS Report Number.Accession Number : 1270764

Events

Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Miss Distance.Vertical : 400
Result.Flight Crew : Took Evasive Action

Narrative: 1

In this opportunity let me submit the following report to inform about the event TCAS/RA from the Flight into DEN. On our approach to Denver's airport we were given radar vectors from the right to land at runway 16R, everything was going as usual, on the last Vectors to intercept final approach the controller asked us if we had the field insight to complete visual (we were completely on a visual condition) to runway 36R so we answer YES could make a VMC approach to runway 36R, she also told us that simultaneously approaches where in progress to runway 16L and she ask us if we had a Boeing 737 in sight (to our left) with the traffic being 100% insight we gave the controller the traffic insight. We intercept final and suddenly the TRAFFIC alert system detected the other airplane so IT gave the Advisory TRAFFIC, TRAFFIC like 3 or 4 seconds later gave the Resolution Advisory DESCEND, DESCEND I performed the maneuver by disconnecting the AP and descending to successfully perform the maneuver. I calculate approximately we were 1800 ft AGL and the maneuver lead us to lose about 400 ft until we heard the respective "CLEAR OF CONFLICT "of the airplane system, after that I just level the airplane to recover the flight path and landed with normal procedures. I would like to add to the report that all the time we has visual contact with the other traffic, what my logic takes me to, was that both systems sensed we were going to make the impact because of the proximity, time and speed of both aircraft but we were VMC all the time with the separation to the parallel runway. In conclusion they are still machines that's why they sense such events.

Synopsis

A320 crew received an RA while on a parallel approach. Uneventful landing completed after "Clear of Conflict."