ACN: 1270636 (1701 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1270636
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through 25,000 FT observed blue arcing in lower left side of CA FWD window followed by a loud bang. The window had fractured over entire surface. Followed QRH procedures and determined inner layer was intact.

Immediately leveled at 26,000 FT and advised ATC of problem. Contacted dispatch and decision was made to divert to Airport ZZZ. Descended to lower altitude to reduce differential pressure and proceeded to ZZZ for normal landing with first officer flying. No further difficulties encountered. Windshield heat shorted, but didn't have time to pull CB as per QRH.

Synopsis

An A320 Captain noted a blue arcing in the low left of his forward window prior to a loud bang and the entire window fracturing. After completing the QRH and determining an outer pane anomaly, the flight descended and diverted to a nearby airport.

ACN: 1270612 (1702 of 2908)

Time / Day

Date : 201506

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 5025
Experience.Flight Crew.Last 90 Days : 205
Experience.Flight Crew.Type : 4067
ASRS Report Number.Accession Number : 1270612
Human Factors : Situational Awareness
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Shortly before reaching our initial cruise altitude of FL340, we received a Y HYD Res OVHT ECAM message. As I was the PF, the First Officer worked the ECAM and followed on with QRH procedures. The procedure allowed to us to reengage the right engine driven hydraulic pump after the reservoir overheat condition subsided. When we did that, the system overheated again right away.

With the loss of a main hydraulic system, we made the decision to divert to ZZZ as that was the closest practical airport considering all options. We notified dispatch of our intention and reason for diverting and we had degraded flight control surfaces and sluggish response as per the flight manual. We then worked the overweight landing checklist and reviewed the flight manual procedure. I also communicated a cabin advisory to the FAs and informed the passengers of the situation.

While inbound, the field was surrounded by thunderstorms and winds gusting to 25 knots out of the west. After a normal landing and rollout, as we exited the runway, we noted ARFF was available to inspect the aircraft. They could see no anomalies, so we taxied to the gate. Of note: The flight manual approach procedure allowed us to reengage the Y ENG hydraulic pump for the approach as the overheat condition had subsided. We did just that and had normal hydraulics for the landing and rollout.

Synopsis

A320 Captain experiences a "Y HYD Res OVHT" ECAM message just before reaching a cruise altitude of FL340. The pump is turned off and the overheat subsides. The pump is turned on and the overheat returns. The crew elects to divert to the nearest suitable airport. The pump is turned on during the approach and full hydraulics are available throughout the approach and landing.

ACN: 1270011 (1703 of 2908)

Time / Day

Date : 201506

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Navigation Light
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning
Problem : Design

Component : 2

Aircraft Component : Wingtip
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7977
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 7490
ASRS Report Number.Accession Number : 1270011
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

This aircraft still has the left wing-tip Navigation lights operating opposite the right wing-tip [NAV light] and opposite every other aircraft I've seen during routine walk-around inspections. I was contacted by a representative and was told by him that after discussions with maintenance in ZZZ1 that they believed that the wing-tip light on this aircraft was not behaving correctly for a given cockpit switch position. I wrote-up this issue again today as it still seems to be operating incorrectly. Can I please get some definitive information on whether this aircraft is wired correctly and every other aircraft is wrong. Or, if it doesn't matter which bulb illuminates with the switch position; i.e. Position-1 and Position-2 have no meaning.

Synopsis

First Officer files another report about the same A320 aircraft with the left wing-tip navigation lights operating opposite the right wing-tip NAV lights. Pilot requests some definitive information on whether this aircraft is wired correctly and every other aircraft is wrong or if it matter which bulb illuminates with the NAV switch in Position-1 or Position-2.

ACN: 1269967 (1704 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1269967
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1269673

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After departure and out of sterile, the A Flight Attendant called to inquire if we had an armed individual onboard as her manifest showed that. I informed her no one had advised me or presented me with any LEO paper work or credentials. We began discussing other manifest anomalies including a CEO/Must Ride sitting where one of our reciprocating jump seater was sitting. I then asked her to come to the cockpit and bring her paperwork to discuss it further. Once it was determined that there were other errors on the manifest I asked her to go back and do a passenger count. A discrepancy of 12 passengers was determined. It was then noted that the Gate Agents had given us a manifest dated [incorrectly]. We counted all extra passengers as adults and calculated we were 2300 Lbs. heavier than expected at Take Off and adjusted our Landing Weight accordingly. I then rechecked our T.O. numbers to see if we had exceeded any performance limitation weights, which we hadn't, but the V speeds were approximately 2-3 knots slower than desired.

More thorough training and supervision of station personnel.
More auditing from management.

Narrative: 2

[Report Narrative Contained No Additional Information.]

Synopsis

A320 flight crew reported discovering several anomalies in the weight and balance data after takeoff. Apparently station personnel used numbers from a flight on a different date.

ACN: 1268493 (1705 of 2908)

Time / Day

Date : 201506
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Cabin Lighting : Medium

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1268493

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We were proceeding down the runway and the Captain (CA) applied the brakes hard instead of lifting off and we came to an abrupt stop. I immediately yelled to remain seated as I looked out my door to see if I saw any fire/smoke. At that time the CA came on and said Flight Attendant (FA) stations and then immediately said Remain seated, Remain seated, Remain seated. FA-D was assessing her door. There was no immediate danger and at that time the CA called me. He told me the takeoff was aborted due to a wind shear alert. He asked if everything was ok, I told him yes and he said he would make an announcement. I called the AFT to inform the crew what happened and they informed me that they assessed the situation from their doors. When the CA was made aware of the tires deflating, he called and said stairs were being bought to the aircraft with busses. We deplaned all passengers with no incidents.

Synopsis

An A320 Flight Attendant reported the Captain executed a rejected takeoff because of a windshear alert. Following the rejected takeoff at least some of the main gear tires deflated.

ACN: 1267305 (1706 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Component

Aircraft Component : Cabin Furnishing
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1267305
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : In-flight
Result.General : Maintenance Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

I was the captain and the pilot flying. On our descent passing through 18,000 ft, we made our initial PA announcement to have all of the passengers seated for our arrival and turned the seatbelt sign on. There was a layer of scattered white puffy clouds with tops at 13,500 and bases at 11,000 feet. There was not any convective activity in the area and nothing in the ATIS about severe turbulence. When we reached the cloud layer, we encountered severe turbulence sending one of the aft flight attendants into the ceiling. Two of the passengers also hit the plastic panels above their seats because apparently they were not wearing their seat belts contrary to the illuminated seatbelt sign being on and the announcement from the flight crew members that was previously made. We then called and talked to the Lead Flight Attendant to check everyone out and she then had called us back to tell us that one of the aft flight attendants was hurt. We then called ops and requested an ambulance. We then told ATC about the severe turbulence we had just encountered and also asked for an ambulance and then asked them for a short base turn to get the aircraft on the ground. We then landed the aircraft and taxied to gate. The paramedics met the aircraft to check out the injured flight attendant and determined that she needed to go to the hospital. The two passengers walked off the aircraft and never asked for any medical attention from us.
With that being said, I made four entries in the aircraft log book:

1.)The aircraft encountering severe turbulence and speed between 280-340 and lasted 6-8 sec straight and level flight. 

2.)O2 panel was damaged in row XX ABC.

3.)Side panel damage in XX ABC 

4.)The tray table was damaged in Row XY seat E.

Synopsis

A320 Captain experiences severe turbulence during descent, upon entering scattered white puffy clouds with tops at 13,500 feet, at a speed of 340 to 280 knots. Two passengers and a Flight Attendant are tossed into the overhead, with the Flight Attendant requiring medical attention.

ACN: 1266489 (1707 of 2908)

Time / Day

Date : 201505

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 2565
Experience.Flight Crew.Last 90 Days : 120
ASRS Report Number.Accession Number : 1266489
Human Factors : Fatigue

Events

Anomaly.No Specific Anomaly Occurred : All Types

Assessments

Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

In my company's fatigue program it states that there will be PROACTIVE risk assessments of flight duty and rest requirements utilized by the company to avoid pilot fatigue. It also states that predictive assessments will use fatigue modeling software to verify alertness levels during trips and scheduling construction. 

Under the program the pilot's schedule leading up to the event, rest periods, work schedule, sleep opportunities and city pairs are a few of the factors to be considered to help avoid fatigue. In the above pairing I find it hard to believe that a trip that reports for duty [early] on day one would follow smart fatigue avoiding practices if no more than 42 hours later you report for duty [early] pilot domicile time. Stated another way in home domicile times. I wake up at 0430 to get to work on Wednesday, the next day I arrive on the West coast at 1300 I'm expected to sleep DURING THE DAY when I'm normally awake, and then report for duty at 0200 right in the middle of my normal sleep cycle. How can this pass any reasonable scrutiny, either human or software based?

Per the Memorandum of Understanding (MOU) with the company [Union] is responsible for ongoing discussions of fatigue events, or possible events. What is being done about these onerous two duty periods in one day trips? It's one thing to have a double duty day when the "all nighters" is [Airport 1-Airport 2], approximately two hour flight. But when the trip is a true all night flight it makes no sense. These trips should be eliminated in the line building stage and never make it into our bidding. 

It is very difficult for pilots, being highly motivated goal oriented people, to call in fatigued at the departure time. We have managed our naps during the day and generally feel fine at departure time. But the real question is whether we will still be alert at top of descent through landing. This is something we can only guess at when we are still 4 to 5 hours from landing. But you can be certain that with this type of trip we will definitely not be at the top of our game during a demanding time.

Synopsis

A-320 captain points out potential fatigue issues with certain flight pairings.

ACN: 1266314 (1708 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZBW.ARTCC
State Reference : NH
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use.SID : HYLND3
Airspace.Class A : ZBW

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7743.63
Experience.Flight Crew.Last 90 Days : 219.15
Experience.Flight Crew.Type : 5857.65
ASRS Report Number.Accession Number : 1266314
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13829
Experience.Flight Crew.Last 90 Days : 163
Experience.Flight Crew.Type : 10249
ASRS Report Number.Accession Number : 1266317

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Just past the waypoint GONZZ, ATC asked where we were going. Replying our next fix as HANKK ATC stated that wasn't the routing they had. We read off our routing which was nothing the controller had us filed with. At this point ATC gave us a new routing which we flew and stated they had no issue, no problem. We checked our ACARS Release Verify which stated we were still RLS1. After speaking with Dispatch, he stated that there was a re-route right around our departure, that he re-ran the flight plan, released it and Sabre re-filed but DIDN'T re-release. At this point I figured the issue was either with BOS clearance or our computer system. Then I remembered something had caught my attention about the PDC during preflight. Looking back at the PDC it listed our filed route as the HYLND3.HYLND CAM ..., which RLS1 stated but the Clearance Revision stated HYLND3.HYLND SYR .... 

What I did immediately after noting that I cannot remember, either last night trying to figure out what happened or today filling out this report. Perhaps I was interrupted... or the fact that many Clearance Revisions state the filed route through the flights first sector with no actual changes and basically adding noise to the preflight. I don't know.

We were lucky this time but obviously an aircraft flying a route that's different from ATC is a giant issue. I realize as one of the links in the chain, had I queried about what do I do after SYR on the Revision, then I would have realize I didn't have the routing I was filed with.

Narrative: 2

As we were navigating on Q29 to HANKK intersection, the controller asked us where we were going, to which I responded, HANKK. He advised that HANKK was not on our route and asked us to verify our filed routing. He then cleared us to HOCKE, which was on the route ATC apparently had us filed on. We checked our papers and confirmed that we were on Release 1 and that the proper route had been loaded into the FMGCs. All looked good to us at that point so we confirmed this routing to the controller. The controller advised us otherwise, and proceeded to issue the remainder of the route we should have been on according to his information. 

I contacted Dispatch to see what was up and was advised that somewhere near our take off time that he had received a new ATC routing, had run a new plan, refilled us and cancelled the original release. Apparently Sabre did everything he had requested except for re-releasing our flight, which apparently it should have. He advised that he would follow up with a report.

After getting on the same page as ATC, I proceeded to review all the documents that we had received and discovered that we had overlooked a small change in the Revised Segment of the PDC. Although the revised segment included our filed SID, (HYLND3.HYLND), I noted that direct to SYR from HYLND had been included. I initially thought the revised segment was a repeat of our cleared routing, which I believed included SYR. I did so based on prior experience with revised segments that were identical to our filed route but never actually contained anything different. Upon closer inspection, I realized I had missed an opportunity to identify a threat had I only inquired about SYR and discovered its significance to our flight. Although we should have investigated why SYR appeared in the revised segment it should be noted that none of the reroute ATC had given to our Dispatch other than SYR appeared there.

Synopsis

A320 flight crew experiences a track deviation when a revised segment on their PDC is overlooked, believing that no change actually exists. Complicating matters the Dispatch computer did not generate a release number two with the new routing and the PDC only contained one fix on the new routing.

ACN: 1265462 (1709 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MSP.Airport
State Reference : MN
Altitude.MSL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : MSP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : MSP

Aircraft : 2

ATC / Advisory.Tower : MSP
Make Model Name : B757 Undifferentiated or Other Model
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1265462

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1265707

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I was the Captain/ pilot flying. Air Traffic Control requested us to maintain at least 170 knots to the marker. After setting managed speed the Tower advised us that a Boeing 757 would be departing with minimum separation before our landing and cleared us to land. I advised the First Officer to be ready for a go around in case the 757 did not continue its take off roll. The 757 was airborne by the time we were at 500 feet so we continued the approach. At 50 feet we encountered a sink which caused a firm bounce on the main gear and subsequent second touchdown. There were no further problems with the rollout and turn off to the taxiway. During taxi in an ECAM message was generated and an action report printed out. I called dispatch and maintenence control to confer with them prior to writing the discrepancies in the logbook. 

Keeping our speed up to the marker and then having to make sure we kept our spacing on the slow moving departing aircraft contributed to encountering that aircraft's jet blast and turbulance during our flare and was a factor in the bounced landing. 

In the future I will have to ask ahead of time if a certain distance can be extended for a departing 757.

Narrative: 2

Landing behind a 757 which previously took off on 30L MSP. Had about a 15 knot left crosswind. Crossing the numbers the aircraft pitched over and I immediately stated to "watch it". I believe he had already began his input to correct the pitch. The aircraft contacted the runway in what seemed to be a flat configuration. The aircraft made a bounce and landed and the rollout seem as normal as could be for the circumstance. I really can't explain what caused the nose down attitude over the numbers. It just all happened so quickly. 

I believe that we met all SOP's during this approach. The Captain was still on low time. He was aware of the 757 and showed concerned for the potential jet blast. He had added 3 knots extra to his approach speed for the winds and potential jet blast. From my perspective it all happened too fast for any action that could be taken on my part to mitigate the hard landing.

Synopsis

A320 flight crew reported a hard landing following a B757's takeoff from the same runway. Captain believes jet blast could have been a factor.

ACN: 1264030 (1710 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Turbine Engine
Aircraft Reference : X

Component : 2

Aircraft Component : Fan Blade
Aircraft Reference : X

Component : 3

Aircraft Component : Checklists
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1264030
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

First Officer during pre flight noticed possible bird strike indications on #2 engine cowl and a large mark on one of the fan blades. I entered this into the logbook and notified Maintenance as well as Operations. The cowl had streaks of material and the fan blade showed a large white mark about four inches in diameter near the tip. Maintenance took the log book with about 10 minutes to go before scheduled departure. They returned the log book to me 12 minutes later with a signoff saying no damage noted ok for flight. We were about to close the door when maintenance returned to get the log book back from me. They took it from the flight deck and returned with another notation saying in the Discrepancy block "landing gear safety devices installed" and in the Corrective block "landing gear safety devices removed". At no time were these devices removed from the flight deck. I also understand the only safety devices for landing gear must be used from each individual airplane. In other words a Mechanic cannot have a set they keep in their tool box. This appeared to be an afterthought just to make the previous entry for "ok for flight" legal. How would I know, since pilots have no idea what is required to be completed.

With the removal of the requirement of an Airworthiness Release after a bird strike inspection it leaves open any manner of inspections. We, as pilots, can only accept the log book from maintenance in hope that the entire procedure was accomplished by maintenance personnel. This obviously was a "quick" inspection due to our close departure time and the fact that maintenance had missed this on their post flight inspection after the planes arrival from ZZZ1 into ZZZ2. Who knows what was ingested into that engine. I know for a fact there is no bug in this world that can leave that size mark on a cowl and fan blade. Currently any Mechanic can sign this [Bird Strike Inspection] off, but it [company] should return to the requirement of an Airworthiness Release and a more thorough inspection be accomplished, no matter how long it takes. I just hope any subtle damage will not later manifest itself in a failure at a later date.

Callback: 1

The reporter stated the A320 flight was on a 'quick turn' and everyone was rushing to get the aircraft pushed for an 'on-time' departure. His Air Carrier has removed the requirement to have an Airworthiness Release signed after a Bird Strike Inspection and even states that on their Maintenance Release. Given the quickness that Maintenance had signed off the Bird Strike Inspection in their Logbook without any reference to any document of items to check for when performing that type of inspection, made him wonder just what was inspected. He's not sure if Maintenance requires a certain qualification for technicians to be allowed to do a Bird Strike Inspection. He still believes a more detailed inspection should have been done. He still doesn't know why the "Landing Gear safety devices" were even entered in the Logbook, because they should not have any bearing on the issue of inspecting the engine for damage.

Synopsis

A Captain expresses his concerns about the 'quickness' that Maintenance had performed on a Bird Strike Inspection on # 2 Engine for an A320 aircraft, without any reference procedure noted in the Logbook sign-off. Adding to his concerns, was his Air Carrier's removal of a previous requirement that an Airworthiness Release also be issued after a Bird Strike Inspection.

ACN: 1264006 (1711 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : MCI.Airport
State Reference : MO
Altitude.MSL.Single Value : 2000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : MCI
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : MCI

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 1264006
Human Factors : Situational Awareness
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1264314
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

At 3,000 FT after being cleared for a Visual approach at MCI I decided to extend the downwind leg to intercept course outside the FAF. I mis read the altitude at FAF and began to descend. upon starting our base turn atc said we appeared low. I started a climb to get back on profile. We intercepted the G/S just inside the FAF configuered for a stabilized approach and landed normally.

Although I had reviewed the approach plate I inadvertantly thought it was a different altitude at the FAF. Lack of visual cues and night time ops, also situational awareness contributed.

This was an eye opening lesson. how one can easily make a simple mistake of mis remembering an altitude. Even on the clearest of days under the best conditions errors can be made and missed. Also reminded me that night time ops can be a challenge even to the most seasoned of us due to the lack of all the visual cues available during daylight ops. Check and recheck altitudes especially on visuals when atc is no longer providing alt guidance. Lesson learned!

Narrative: 2

Descending into MCI we were being vectored by ATC to a right downwind for 19R right at 3,000ft. ATC asked if we had the traffic on short final in sight we acknowledged that we did and received clearance for a visual approach to follow the proceeding aircraft. We began a descent and a turn to base leg when we simultaneously with ATC recognized that we were low and made immediate corrections to our profile, continued our turn to final and intercepted the localizer and glide slope and were stabilized for a normal landing. Proper altitude for segment was misinterpreted and mistakenly verified.

I strive and take great pride in always trying to complete the "perfect flight" in reference to Company Policies, Procedures and SOPs. If I may make one suggestion that might alleviate as many opportunities for this to occur, is to consider requesting vectors to the final rather than accepting a visual approach on a downwind leg, especially at night.

Synopsis

A320 flight crew reports getting low during a night visual approach from the downwind position. ATC is the first to detect that the crew has descended out of 3,000 feet prematurely.

ACN: 1263641 (1712 of 2908)

Time / Day

Date : 201505

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Other Documentation
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1263641
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During our preflight, I determined the assigned aircraft as "unable to operate"." A320 was replaced with another A320 aircraft. After opening the CCS Sabre Flight Plan Manager, I selected the briefing tab, checked all applicable selections, and the selected "Print All." The Sabre Briefing Package presented was labeled Flight XXX, Release 03. The Briefing Package contained an OFP now listing the newly assigned aircraft. However, the Sabre Briefing Package contained the DIL and log for Aircraft XXX (the previously assigned aircraft). Additionally, Release 03 for Flight XXX now included an alternate when before there was none. The Sabre Briefing Package, Release 03 did not include the weather and NOTAM information for this alternate airport. 

This is a reoccurring problem which has been reported several times before. I myself have submitted several reports identifying this very problem with the Sabre Flight Planning Program. Please ensure that: 
1) Pilots are notified that their Sabre Flight Plan Briefing Packages may be omitting regulatory required documentation.
2) Please correct this reoccurring problem with Sabre not updating the flight planning package according to changes in the flight planning process.

Synopsis

An A320 Captain noted that when his flight release is revised, his air carrier's revised SABRE flight plan briefing packages do not always include the alternate's weather and NOTAM information.

ACN: 1263640 (1713 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1263640
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Airport

Narrative: 1

During departure from DEN Runway 08, night VMC, I noticed that the runway centerline lights are only visual for 2000 to 3000 feet ahead of the aircraft. The centerline lights to the end of the runway were not visible including the alternating red and white lights of the last 3000 feet. This would prove difficult during an engine-out situation in which directional control is enhanced by using an aiming point of the centerline lights at the farthest end of the runway. I have spoken with other pilots who have noticed the same condition on other runways at DEN.

Synopsis

An A320 Captain noted that departing DEN Runway 08 at night, only 2,000 to 3,000 feet of centerline lights preceding the aircraft are visible. The runway end alternating red and white lights are not visible until approaching them.

ACN: 1262735 (1714 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 20000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Power
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15720
Experience.Flight Crew.Last 90 Days : 162
Experience.Flight Crew.Type : 7719
ASRS Report Number.Accession Number : 1262735
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1262738
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

On descent at approximately FL200, I received an interphone call from Purser telling me that there was white smoke coming from a ceiling panel near the forward galley in the area of Circuit Breaker Panel 2000VU. She also told me there was a burning smell that accompanied the smoke. I requested an immediate vector to our destination airport. We had a company A320 First Officer deadheading in the cockpit who I sent to the cabin to assist the purser and give me an evaluation of the situation. He reported back that the smoke was dissipating, but the smell was still noticeable. ATC vectored us to a visual approach where we landed uneventfully. On the ground we stopped on the taxiway where we were met by air rescue and firefighting (ARFF) who checked the aircraft with their FLIR and reported there was nothing unusual seen. Since both the smoke and smell had dissipated we made the decision to taxi to our gate with ARFF following. 

We were not able complete the smoke/fumes checklist in the A320 QRH because of the time available to landing (12 minutes) and the coordination with our jumpseater and Purser. We did the immediate action from memory donning our O2 masks and establishing communication. They had thunderstorms in the area and were on a non-standard routing for the flight. Upon landing, an evacuation was considered, but since the smoke had dissipated and the smell was diminishing along with the fact that ARFF saw no indication of a fire, we decided to continue to the gate.

Narrative: 2

On the arrival at FL200 received a call from cabin that a passenger had witnessed smoke "puff" from the ceiling near the forward galley and also smelled smoke. Flight attendant said she also could smell smoke but could not see any. Captain instructed her to put on her PBE, evaluate the situation and to keep him informed. The flight crew donned O2 masks at this time and the Captain informed ATC of our condition. The cockpit jumpseater was sent back to the cabin to verify condition and to also render aid and report to cockpit. No further smoke was seen but smell was reported as electrical/ozone and getting stronger. Odor of electrical smoke was now present in the cockpit at which point the Captain advised ATC. First Officer continued duties as pilot flying and initiated a rapid descent. Captain requested and received clearance for visual approach with progressive vectors and altitude recommendations by ATC. Landing was uneventful, on glide path and on speed. 

We exited at Taxiway and to turn left onto another taxiway where we immediately came to a stop to do a final evaluation of the inside of the aircraft by the jumpseater and flight attendants and the outside of the aircraft with infrared equipment by the fire department. Flight crew removed masks at this point. The Captain and First Officer came to the decision that an evacuation was not necessary and proceeded to the gate for a normal deplane. We requested that the fire department escort us all the way to the gate while maintaining fire watch and communications with the cockpit. The flight attendants were informed of the plan for a normal gate deplane but were told to be prepared in case the problem escalated. The aircraft arrived at the gate where a normal deplane commenced and a final fire inspection was initiated and completed and an all clear given.

Synopsis

An A320 Flight Attendant reported smoke from the ceiling near the forward galley during descent to their destination so ATC expedited the arrival. At the gate, the aircraft was deplaned normally because the smoke had dissipated.

ACN: 1262477 (1715 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 35500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZHU

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7500
Experience.Flight Crew.Last 90 Days : 220
ASRS Report Number.Accession Number : 1262477

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure climb from KMSY, we were cleared from FL340 to FL360. At FL355, Captain (flying) pointed out a TCAS target slightly behind (less than 1 mile) and exactly at our cleared altitude of FL360 on both NDs. No TCAS RA was received, so he asked me to confirm with Center if there was an aircraft at our cleared altitude. ATC didn't hear my initial transmission, as I believe the hand mike cord came out of the socket as I grabbed the hand-held mike. Meanwhile, the Captain stopped the climb momentarily at FL355 as ATC asked if we were level yet at FL360. I attempted confirmation again that there was no aircraft at FL360, while the Captain disconnected the autopilot and auto-thrust to keep the aircraft level at about FL355. At this time, ATC directed us to turn right immediately 30 degrees and had another aircraft that wasn't initially on our TCAS, but was climbing to FL350, then descend to FL340. I don't know if that aircraft received a TCAS RA, but I re-established communication with my headset and confirmed we had turned 30 degrees right and were at FL355. ATC asked if we were maneuvering for a TCAS RA, which we weren't. ATC didn't mention any loss of separation, but subsequently cleared us the block FL350-360 momentarily as we assessed the situation. At that time, we both noticed "TA ONLY" on our NDs, even though traffic mode was selected. We also knew this aircraft had been written up the previous day for this same malfunction (without the phantom targets). Captain then re-selected auto-flight and leveled at FL360.

Synopsis

A320 First Officer reported receiving a TCAS warning for what was apparently a phantom target.

ACN: 1261588 (1716 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cockpit Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 27000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4700
ASRS Report Number.Accession Number : 1261588

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This aircraft had a history of keypad cockpit door chime failures. The keypad had been replaced earlier in the day. Upon arrival to the airplane I tested the cockpit door chime and it failed to ring. Maintenance was called where the Technician advised he was going to MEL the door chime which required the removal of power to the entire keypad. So, with power removed from the keypad emergency access to the cockpit is inhibited. There would be no way to enter the cockpit should there be an incapacitation of both crew members. There is no key access in this design on the Airbus. This MEL on the Airbus should be eliminated. With it, a flight deck crew is sealed off from the cabin without an option for emergency access should the need arise. I would have refused this aircraft had the Technician not been able to fix the chime, resulting in this MEL being applied.

Synopsis

A320 Captain reported concern about an MEL on the fleet that allows aircraft to be dispatched with the cockpit access keypad inoperative, since that would preclude any emergency access to the cockpit.

ACN: 1261551 (1717 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use : GPS
Flight Phase : Cruise
Airspace.Class A : ZLC

Component

Aircraft Component : GPS & Other Satellite Navigation
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 70
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1261551
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8430
ASRS Report Number.Accession Number : 1261604
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

With autopilot 2 engaged, the aircraft initiated a turn to the left to correct back to what it perceived as its FMGC generated course. The turn was fairly aggressive, 30 - 40 degree intercept, and was questioned by the pilots. A message then appeared on MCDU screen FMGC 1/2 disagree. We noted that, although both boxes indicated that they were both in high accuracy with estimated position deviations of only 0.08 mile, they did, in fact, disagree by almost four miles. We did not know which, if either was correct and were about to query ATC when they contacted us, asking why we were deviating to the north. Aircraft was already correcting to course when we received the call. Turns out, FMGC 2's course was possibly corrupt and that was what the aircraft was following. Within approximately one minute, the two FMGC's updated each other, generating a single accurate course. We informed ATC of the transient downgrade. In addition, GPS was being jammed across the Utah area, with GPS lost message on CRT, but had recovered to GPS primary at time of event.

Narrative: 2

While in cruise with autopilot 2 on, aircraft made an unexpected 20-30 degree turn left. Analysis showed FMGC 2 showing 3 nm Right of course, but FMCG 1 showing 1.3 nm left. We had been in an area of GPS jamming and the GPS primary had come and gone several times. Data page showed FMCG 1 most closely in occurrence with GPS position, but GPS wasn't reliable. Discussion ensued about which FMGC was most correct and why radio updating would show 2 different positions between the FMGCs to determine correct navigation course. Then FMGC 2 position seemed to update and move toward FMGC 1. Both positions moved to agree and airplane nav moved to correct back on that course. Salt Lake Center queried about turn and we informed of our navigation uncertainty but that problem seemed to be fixed and we showed on course.

Synopsis

A320 flight crew experiences GPS jamming in ZLC airspace resulting in a momentary track deviation. The jamming was NOTAMed but the aircraft's reaction to the jamming was unexpected.

ACN: 1261521 (1718 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 225
ASRS Report Number.Accession Number : 1261521
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Diverted
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cruise altitude 33000 feet just prior to descent the aircraft yawed along with ECAM (Electronic Centralized Aircraft Monitoring) alerts. The clear indication from engine instruments and ECAM alerts was the number 1 engine had failed. During this failure, we were given a descent by ATC. As the pilot monitoring, I read back the clearance. I assumed control of the aircraft, started the descent and directed the copilot to run the ECAM. During this time, along with following along with the ECAM procedure, I was attempting to find the nearest suitable airport. The ECAM procedure was engine 1 fail. The engine did not relight and he continued with the engine 1 shutdown. During this time, using input from the First Officer, ATC, ACARS weather and weather radar the decision was made to divert to a nearby airport. This was the closest suitable airport. Somewhere in this timeframe, I received a call from the flight attendants in regards to odd noises in the cabin. I informed her we had an engine failure and I would get right back to her. Further coordination with ATC included nature of the emergency, souls on board, fuel remaining and ground emergency equipment. The First Officer finished the ECAM and QRH lists and then coordinated with the flight attendants for a cabin advisory. I made an announcement to the passengers stating the emergency condition and tried to reassure them this was an event we were highly trained to handle. I also provided the time and location of touchdown. It was at this time I sent a "call me" to dispatch attempting contact. A short time later we received a frequency and the First Officer informed him of the situation, location of divert ensuring no issues, and requested landing data. He then ensured the FMGC was programmed correctly and we briefed for a single engine ILS to Runway 5R. Now with most issues completed, we attempted an engine inflight start. This was unsuccessful and we focused our attention to the approach and landing. We were vectored to final and completed an ILS to Runway 5R in VMC conditions. The approach and landing were uneventful and the aircraft was taxied to the gate and shut down. The passengers were deplaned using air stairs and were taken inside the terminal. There were no injuries or aircraft damage during this event.

Synopsis

An A320 engine failed at cruise prior to descent, so the QRH was completed and the flight diverted to the nearest suitable airport.

ACN: 1260742 (1719 of 2908)

Time / Day

Date : 201505

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Visual Approach
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : FCC (Flight Control Computer)
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 13000
ASRS Report Number.Accession Number : 1260742
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 120
ASRS Report Number.Accession Number : 1261605
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

At cruise- ECAM GPWS fault..went to FOM...allowance of Circuit Breaker (CB) pull in flight..28 VDC and 115 VAC...searched on panel 121VU ..located breakers below adiru. N6/N7. Both had corresponding 28 vdc/115 ac...never noticed right above same breakers p6/p7...upon pulling n6/n7...aircraft entered alt mode/ direct law after gear extension..contacted disp/ Maintenance Control tried to reinstate systems to original status..all breakers reset but not normal law...went through landing dist/ vref proc/ thrust issues for landing preparation ..destination not acceptable due to landing distance...diverted.

Narrative: 2

Prior to top of descent I, the First Officer, was flying pilot with autopilot engaged. Ecam "Nav GPWS fault" displayed. Followed QRH which has an air reset procedure. I remained flying pilot while the Captain searched the Circuit Breaker (COB) panel behind my head for the appropriate CB's. The Captain located the CB's after some time spent searching. He then asked me to verify the CB's before he did the reset. I turned in my seat as much as possible and craned my neck to see behind me and saw the GPWS label on the CB panel near where the Captains hand was and told him it was verified. The Captain then pulled the two CB's just below the correct breakers. Immediately, the First Officers PFD and MFD screens blanked out, the autopilot, autothrust, and flight directors disengaged and the aircraft reverted to alternate law.

At that time my focus reverted to hand flying the aircraft via the Captains instruments which were functional except for the flight director. I flew the aircraft for the next few minutes until the Captain returned to his seat at which point he took the controls and I became the pilot monitoring. 

We now had additional ECAM messages which I tried to sort through while monitoring ATC. The Captain was communicating on comm 2 with the company to try and rectify the situation. At this point ATC started our descent and the workload increased significantly. I was giving hand signals to the Captain for speed and altitude assignments while the Captain was communicating with Maintenance Control and the company on comm 2 while also hand flying the aircraft.

The decision was made to divert for the longer runways and the reduced flap setting that the checklist called for. We made the decision to [notify ATC of our situation] in order to reduce the workload on us that was increasing by trying to meet all the restrictions on the arrival while hand flying and working the problem. ATC gave us a heading and altitude for our diversion.

The Captain briefed the flight attendants and passengers and we shot an uneventful visual approach. Captain asked me to control the thrust levers and speed on the final approach while he focused on smoothly hand flying in alternate and later direct law when the flight controls can be awkward and overly sensitive. The Captain resumed control of the thrust levers on short final. Uneventful landing.

Synopsis

A320 flight crew experiences an EGPWS fault prior to descent and checks manual for a circuit breaker pull and reset, which is allowed. Upon pulling and resetting N6/N7 on the 121VU panel the aircraft goes into alternate law and the crew diverts to a longer runway. Directly above N6/N7 are circuit breakers P6/P7 for the ADIRU's.

ACN: 1260716 (1720 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 105
ASRS Report Number.Accession Number : 1260716

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 489
ASRS Report Number.Accession Number : 1261624

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition

Narrative: 1

Extract Fan failure in cruise at 36,000 feet. Audible fan rotation and wind-down as the bearing seized followed immediately by perceptible smoke odor, Extract Fan ECAM and Extract Fan Switch 'Fault' illuminated. Donned Masks and accomplished Emergency Descent. Blue action step directed Extract Switch--OVRD. Accomplished Avionics Smoke QRH for perceptible smoke. Smoke dissipated quickly. Flew normal approach and landing (not over-weight) at nearest suitable airport. Aircraft deemed safe by Fire Department and taxied to the gate.

Narrative: 2

[Report narrative contained no additional information].

Synopsis

An A-320 experienced an Extract Fan failure in cruise. The crew completed the Avionics Smoke QRH checklist and landed at the nearest suitable airport.

ACN: 1259509 (1721 of 2908)

Time / Day

Date : 201505
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Taxi

Component

Aircraft Component : Winglet
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1259509
Human Factors : Time Pressure
Human Factors : Fatigue
Human Factors : Distraction

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Airport
Primary Problem : Airport

Narrative: 1

Pushing back, hooked up to tug, off gate, approximately 1/3 of the way through pushback the aircraft made a shudder and came to an unusual stop. I immediately asked the tug driver if everything was ok. He replied his wing walker let him hit a B757 tail. We set the parking brake and secured the aircraft. [This Gate] is extremely tight to operate in. Extreme caution needs to be used in and out.

The flight was an empty ferry flight. The aircraft had less than normal fuel, no bags or passengers and new Sharklet wings. This makes for a very high wing tip off the ground. Also, the Red eye flight landing that morning, departing that same night, only allows for 4-5 hours of circadian sleep. While able to operate per the 117 rules, lack of sleep does not allow for 110% clear focus. This type of pairing should be reviewed.

Synopsis

An A320 was being pushed at dusk in a confined area when one of its Sharklet wingtips hit a B757 tail because the wing walker did not warn of the collision and also the new wingtips are very high off the ground.

ACN: 1258883 (1722 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1258883

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1258884

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

In cruise flight the Flight Attendant (FA) called and said she smelled smoke. She said she didn't see smoke but smelled some sort of electical fire/smoke odor. We advised ATC that we were [had a situation] and would like to start an imediate descent and head towards [the nearest suitable airport]. We told the FA to keep us posted if the smell got worse or she saw smoke or found the source of the problem. ATC gave us a decent initiallty to FL190 and a heading followed by direct the airport. It was about a little over 100 miles away at this point. Just as we were about to level at FL190 the FA called back and said that she had detected the source of the smell. That the smell was coming from one of the FA's purses that had been in the front galley. She told us that the smell was coming for a car remote that had gotten wet and that they had searched thouroughly and were 100% confident that the smell had come from the purse and that there was nothing wrong with the aircraft. We verified this with her at least 3 or 4 times. We were at FL190 at the time and decided since the source of the smell had been detected and we were confident that the safety of flight was not impacted or would be impacted we decided to continue the flight. We advised ATC what had happened and they reissued a clearance to destination. 

As we proceeded back on course we decided to contact dispatch by radio rather than try to explain it over ACARS. Dispatch concluded with us that as long as the safety of flight was not impacted and the source of the smell had been determined that we would be safe to continue the flight. We verified with dispatch our Fuel on Board (FOB) and had them double check our new arrival fuel. We were tankering so there was plenty of fuel to continue the flight safely. Our arrival FOB was still scheudled to be over 11,000 lbs. We also verified with the FA's that the passengers were calm and that she 100% certain again that the odor was coming from her purse and that there was nothing wrong with the aircraft. We continued to [our original destination] and landed safely. 

Upon landing the FA's came to the cockpit to debrief the situation. Only then did she tell us that there had been an electronic cigarette in her purse and she briefly saw what looked like smoke come from her purse. We then concluded that the electonic cigarette had most likely been the source of the odor.

Narrative: 2

In cruise flight, Flight Attendant (FA) called the flight deck and reported an electrical fire smell coming from the galley area with no smoke. I was pilot flying and took the radios, while the first officer pulled out the manual. I [notified ATC of the situation] and requested to ATC that we change our destination [to the nearest airport] and to have equipment standing by. I called back to the cabin crew to get an update and they said they still said they smell the odor and cannot find the source. After a couple minutes passed and passing through FL190, the lead FA calls back to inform us that they located the electrical smell and that the smell is coming from a flight attendant's purse that was sitting up in the forward galley. The FA assumed that the smell was coming from a car key remote that was possibly shorting out in her purse. I asked her if she was 100% sure that this was the source of the smell and she confirmed with a "yes" answer. So I proceeded to level off the aircraft and notify ATC that [the situation] no longer exists and explained the situation. I asked to climb back up to our crusing altitude and continue to destination. I called dispatch to explain the situation and to go over fuel numbers to proceed safely with the flight. Thankfully we were a tankering flight and had plenty of fuel on board to continue safely. After talking to dispatch, I checked with the FA's another time to make sure that the smell was once again the purse and to make sure the passengers are calm. They informed me once more that the purse was 100% the cause of the smell and that the passengers were calm. We continued to [our destination] with no further events and the flight was completed safely. Upon parking at the gate, I asked the two forward flight attendants to come up to the flight deck to explain the situation again to us. What she failed to inform us inflight was that there was visible smoke that temporarily came from the purse and that there was also an electronic cigarette in the purse as well. I saw her car key remote and it looked like there was no damage to the remote at all. I can only assume that the electronic smell and smoke came from the electronic cigarette that was in her purse. 

I have heard more and more issues with electronic cigarettes in flight. From passengers using them in the lavatories and setting off smoke detectors, to this event. I really think we need to consider more strict regulations in regards to electronic cigarettes onboard our aircraft.

Synopsis

A320 flight crew is informed by a Flight Attendant that electrical fumes are present in the cabin. ATC is advised and a diversion is planned. At FL190 the Flight Attendant advises that the source of the smoke has been traced to a purse and later confirmed to have been caused by an E cigarette. Flight continues to destination.

ACN: 1258213 (1723 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DCA.Airport
State Reference : DC
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 01
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : DCA

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : PCT
Make Model Name : Helicopter
Flight Phase : Cruise
Airspace.Class B : DCA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 3600
ASRS Report Number.Accession Number : 1258213

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1187
Experience.Flight Crew.Last 90 Days : 174
Experience.Flight Crew.Type : 961
ASRS Report Number.Accession Number : 1258227

Events

Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

On the Mount Vernon Visual on 11 mile final and level at 2500 ft, Potomac advised us of pop-up helicopter traffic 500 feet below our altitude. Due to the potential conflict Potomac cancelled our approach clearance and said to maintain 2500 feet. We had no traffic displayed on the TCAS at that time. A few seconds later at about 10 mile final the traffic displayed in red and we received a TCAS resolution advisory (RA) to climb. We climbed and told ATC that we were climbing due to the RA. The "climb" warning stopped after about 300 feet, we deviated about 500 (3000 MSL) in total. After the "clear of conflict" alert we returned to 2500 feet. We then asked for a new clearance to get back in sequence for arrival. I informed Potomac that I would file a report. I saw 400 feet on the TCAS as the closest distance between the two aircraft.

Narrative: 2

[Report narrative contained no additional information]

Synopsis

A320 Captain reported experiencing an NMAC with a helicopter on the Mount Vernon Approach to DCA.

ACN: 1258125 (1724 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 18000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : TWSTD3
Airspace.Class A : ZHU

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 585
ASRS Report Number.Accession Number : 1258125
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were flying to IAH. I am an A320 Captain and was receiving right seat training (to become a LCA) from a Line Check Captain. I was the pilot monitoring. We were originally filed on the WHACK2 arrival (which is for a west operation in IAH). Since IAH was landing to the east I asked Houston Center if we could still expect this arrival. Houston re-cleared us via the TWSTD3 arrival.

At some point we were cleared direct to WHACK and to cross WHACK at 13,000. As I recall we were then at FL220. The Pilot flying (PF) (Captain) properly selected 13,000 in the FCU and entered the crossing restriction altitude in the FMS. I verified that his entries were correct. Several miles prior to the TOD arrow I made a remark that it was almost time to start down. A couple of miles prior to the TOD arrow the PF initiated a managed descent to 13,000. We were below the path (later directly on the vertical path). Everything looked as we expected it to look.

The published crossing altitudes for the WHACK fix are between FL200 & 17,000 @ 280 KTS. Again, we had been cleared to cross WHACK at 13000. The fix after WHACK (TWSTD) has a published crossing altitude of between 16,000 & 13,000 @ 280 KTS. 

Approximately 10 miles prior to WHACK both the PF and myself realized that the aircraft was not descending the way it should. The PF clicked off the autopilot, went into open descent, and extended the speed brakes (only 1/2 speed brakes are available with the autopilot connected). 

Several miles prior to WHACK ATC asked us to slow to 250 KTS. I informed him that we would not be able to make WHACK at 13000. He told us "no problem" then gave us a revised clearance to descend via the TWSTD3. I also asked whether he wanted us to maintain 250 or comply with the published speeds on the STAR. He told us to maintain 250 KTS.

The LCA I was flying with said that he had programmed the FMS correctly (I agree) and that this was a known anomaly with the Thales FMS. He said that when we were cleared to an altitude below what the published altitude for WHACK, with the corresponding other crossing altitudes at TWSTD, that the FMS gets confused as to what altitude to make. He told me he would be following up on this with the Fleet Captain and technical pilots for corrective action. 

Obviously we both could've done a better job of monitoring the ongoing descent. I was lulled into believing that since everything was loaded correctly and was descending on profile that it would make the restriction as entered.

Callback: 1

The reporter states that his company has issued a bulletin covering this anomaly, descending to a hard altitude with window altitude to follow, will cause the hard altitude to be missed under certain circumstances. The window altitude must be changed to a hard altitude at or below the earlier altitude to use managed descent.

Synopsis

A320 First Officer on the TWSTD3 RNAV arrival to IAH reports being cleared by ATC to cross WHACK at 13,000 feet and the Captain sets 13,000 in the FCU and in the FMGC WHACK. The waypoint after WHACK is TWSTD with a crossing restriction between 16,000 and 13,000 which remains displayed and the reporter states confuses the FMGC into starting a late descent for the WHACK restriction. The flying Captain states that this was a known anomaly with the Thales FMS.

ACN: 1257187 (1725 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class C : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1257187
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16100
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1050
ASRS Report Number.Accession Number : 1257184
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Human Factors

Narrative: 1

Controller 1 gave us direct [to a fix], then he gave us Pilots Discretion to FL240. We then started our descent. When we reached FL240 we were then cleared again Pilot's Discretion to FL190. We were never given a descend via clearance - we were only given direct. We were then switched to another controller. 

This controller then cleared us to descend via [the] arrival into ZZZ. I set the lowest altitude 5,000 feet for transition. Before I moved my finger I verified that the Pilot flying (PF) put it in Managed Descent and Managed Speed of which he did. Aircraft started to slow and we were on profile. Shortly thereafter the Controller was very curt in his question of our speed of which I professionally replied 278 knots. His agitated response was you should've been at 270 knots. This controller took it further by now telling us to call when we land. We were surprised by this tone, that we actually needed to have him repeat it. 

If there were separation issues, I don't think giving us a phone number twice fixes the separation issue - (of which we were not aware of any other aircraft - he did not point any traffic out). We are on a descent into mountainous terrain at night and becoming confused by ATC's handling of our flight. After that he told us to fly an off course heading. I heard turn 30 degrees left, the captain initiated a 30 degree right turn to what he thought was to be a 030 heading. When I saw him turn right I responded turning 30 degrees right thinking I misunderstood him of which the controller did not correct us. 

The next transmission came from the controller and he said I thought I told you to turn to the left. The controller told us stay on your course to the right to I believe a 030 heading. I queried the Captain and said that's where the high terrain is, we need to go back left. When I was about to ask ATC to go left he cleared us direct [to a fix] descend and maintain 7,000. 

There was a chain of events that started because we never were initially cleared to Descend Via the Arrival until inside the arrival. The manner in which this was handled, in my opinion raised safety issues by creating communication barriers and confusion. We landed without incident and then called the phone number at the gate. The Captain called and spoke with the ATC Supervisor. I did not hear the conversation the Captain had but I was told a synopsis from the Captain. He gave me a thumbs up that it's no problem and not to worry. Apparently this controller had a very heated discussion regarding this arrival with the Chief Pilot over this exact thing regarding speed on the arrival. In my opinion, from what I was briefed by the Captain, it almost seems we were almost setup to fall into what I believe was a "trap"; so as to possibly validate his discussion with the Chief Pilot, just my opinion.

Narrative: 2

...At this point as a crew, we were confused and losing situational awareness. The controllers tone and agitation was only adding to our stress and creating a communication barrier. We continued to receive vectors and landed uneventfully. I contacted approach at the given number after parking at the gate. Approach control supervisor took the call for the controller we were working with during the approach. [He] said he had review the situation and we discussed the arrival and communication errors. [He] explained the controller on duty recently had a heated discussion with the Chief Pilot over his approach control clearances and the controller was still on edge with pilots over the discussion. We ended the conversation on good terms and exchanged names and numbers.

Synopsis

An A320 flight crew received a 'Descend Via" clearance and were in Managed Descent and Managed Speed. ATC queried the crew about their speed and they replied "278 knots"; they were slowing to the required 270 knots as required by the profile. The Controller gave them a number to call upon landing. The Captain called and spoke to a supervisor who relayed that the controller on duty had recently had a heated discussion with the Chief Pilot about his dissatisfaction with pilot's arrival and communications errors.

ACN: 1257074 (1726 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1257074
Human Factors : Workload
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1257074
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At this time I was Pilot Monitoring; about an hour into the flight Captains (CA)s Primary flight Display (PFD), Navigation Display (ND), and either the upper Electronic Centralized Aircraft Monitor (ECAM) or lower System Display (SD) flickered. We looked in QRH and made a note of it. Approximately 30 minutes later his screens flickered again as well as ELEC GEN 1 FAULT ECAM caution message. ECAM action and QRH procedures were completed and GEN 1 was reset and operating normally. 10 minutes later ELEC GEN 1 FAULT ECAM reappeared and CA assigned me Pilot Flying with radios while he completed ECAM Action and QRH procedures which included starting the APU. 

Once complete, I sent ACARS message to dispatch and maintenance regarding the ECAM and requesting fuel burn with APU in continuous operation. CA called A Flight Attendant (FA) up and told her what was going on. She said passengers were complaining that wifi was not working and galley and cabin lights were flickering. 

10-15 minutes later ELEC IDG 2 OIL LO PR, ECAM Caution message appeared. CA assigned me pilot flying from that point until landing. ECAM action required resetting or disconnecting IDG 2, with GEN 1 offline this would give us only our APU GEN and RAT and Batteries as back up. We briefly discussed the ECAM action when ELEC GEN 2 FAULT ECAM caution message appeared. CA completed ECAM action as we confirmed IDG 2 DISCONNECT. It was discussed and agreed on that ZZZ would be our nearest suitable diversion airport. We tried contacting maintenance and Dispatch but ACARS was not working.

We asked center to relay message regarding our diversion to dispatch. ZZZ ops was contacted. After numerous ECAM and System displays were reviewed and cleared, we reviewed overweight landing QRH and runway distance required. CA briefed FA/passengers and descent/approach checklist and flows completed. We flew flaps full approach to overweight landing at approx 148,000 lbs touching down at less than 200 FPM to manual breaking.

Narrative: 2

About an hour into flight Caps Navigation Display (ND) and Primary flight Display (PFD) flickered momentarily. Several minutes later the GEN 1 fault light started to flicker and reports from cabin lights flickering and internet inoperative. 

Electronic Centralized Aircraft Monitor (ECAM) GEN 1 fault. Reset Gen 1 and returned to normal for a few minutes. ECAM Gen 1 fault and Gen 1 would not reset. Started APU and advised maintenance and dispatch. Intended to continue to ZZZ1 using APU as backup Generator. Requested new fuel burns from dispatch and assessed possible fuel stop. ECAM ELEC IDG 2 low oil pressure. First Officer (FO) now flying pilot. GEN 2 still operating normally. ECAM directed IDG 2 disconnect. All electrical parameters were normal with no overloads or overtemps and all busses powered. We decided not to disconnect IDG 2 to prevent single generator operation if possible. 

GEN 2 fault light began to flicker followed shortly by GEN 2 failure. Autopilot disconnected and autothrust failed followed by multiple ECAMS and system malfunctions. Autopilot re-engaged but autothrust remained inoperative for remainder of flight. FO assumed all flying and ATC duties while I worked through ECAM and system failures. Requested clearance to ZZZ2 160 miles to the Northwest.

GEN 2 would not reset. Tried again to reset GEN1 to try to get a second Gen online. Both GEN 1 and GEN 2 completely inoperative. With GEN 2 inoperative and no reason to leave the Integrated Drive Generator (IDG) connected - confirmed guarded IDG2 disconnect with FO and disconnected IDG 2 as directed by ECAM.

After completing all remaining ECAMS we had 2 pages of system failures and inoperative components including:
complete Blue system hydraulic failure, N1 degraded mode on both engines, #1 reverser inoperative, Slats slow, and multiple overhead and panel switch lights inoperative making it difficult to know switch positions and status. Also at some point we lost AC bus 1 and TR 1and that bus and TR remained unpowered at gate even with external power connected. ZZZ2 maintenance was not able to reset AC bus 1 or TR1.

We also lost ACARS and cockpit cell/internet phone so were not able to contact dispatch in flight. Asked ATC to relay our status to dispatch via telephone and called ZZZ2 ops on VHF radio to advise we were landing MSP. Briefed Flight Attendants (FA)s and Passengers to expect normal landing. Discussed approach and landing with FO including overweight landing requirements aircraft and go-around capabilities and contingencies such as possible emergency electrical configuration. ZZZ2 weather was VFR and FO approach and landing were normal.

Synopsis

An A-320 aircrew lost both engine driven electrical generators. They were unable to reset either one and elected to divert. They landed uneventfully at the diversion airport.

ACN: 1256059 (1727 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 11500
ASRS Report Number.Accession Number : 1256059
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1256078
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After landing on Runway 28C, ATC ground controller instructed us to hold short of taxiway D on EE, then follow an aircraft under tow on taxiway Alpha-18, then Alpha to the ramp. As the aircraft cleared in front of us, the Captain started rolling and then we received a revised clearance from ground controller to continue holding on EE for an RJ approaching from our left on D, then follow aircraft under tow to the ramp. Captain stopped our aircraft on taxiway EE, well short of taxiway D but with the aircraft nose blocking one lane of the ground vehicle service road. Ground service vehicles to our left and right had stopped at this time, so we assumed that they would not move until we completely cleared the service road. As the RJ cleared the intersection of EE on D, captain released the brakes, and just as we started rolling, the first officer saw a truck approaching and immediately alerted the captain to stop the aircraft. The captain immediately stopped the aircraft, and we both watched in amazement as a truck passed on the open service road lane, from our right to left. As he passed, I would estimate that the left side roof of the truck passed within 3 feet of the nose cone of our aircraft. This was the closest that I have ever come to a metal bending ground incident.

Narrative: 2

After landing Chicago O'Hare 28C and clear of 28R were given taxi N, EE, hold short D to follow an aircraft under tow via A18, A to ramp. Ground then instructed to give way to an RJ approaching left to right on B which caused me to stop partially on service road. All service vehicles were stopped at that time. We were cleared to continue to follow the aircraft under tow upon which we began rolling but to our amazement a truck began passing right to left in front of our aircraft. The First Officer recognized and called stop. I promptly stopped the aircraft and the truck made no effort to stop or swerve missing the plane by less than 10 feet. This was too close for comfort! The taxi light remained on throughout the taxi.

Synopsis

An A320 flight crew describes a near collision with a catering truck at ORD after holding short of a service road for two aircraft to pass. The catering truck began to cross the taxiway and did not stop, at the same moment the Captain begin to taxi. A quick stop by the Captain was required.

ACN: 1255858 (1728 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 12300

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : TRTLL 2
Airspace.Class E : C90

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1255858
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1256150
Human Factors : Workload
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Dispatch
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

Had incorrect barometer (30.27 vs 29.55) setting which resulted in leveling off approximately 700 [FT] below assigned altitude of 13,000 FT while descending into Chicago airspace. Received a message from dispatch prior to descend stating "Attn crews: ATIS is reporting old info. Please check with dispatch for updated WX. The WX request page is still accurate." We read the message but did not realize this was 1- specifically for us and 2 was referring to Chicago ATIS. We sent for and received Chicago ATIS just prior to descent which gave the barometer setting as 30.27. We did not notice the time on the arrival information was invalid. Shortly after this we received descent instructions including changes to crossing altitudes and speeds which created additional activities while descending. While inputting the arrival restrictions we passed transition level, set 30.27, completed the checklist and then leveled off at what we thought was 13,000 FT. Approach ATC contacted us, asked to confirm our altitude and informed us the correct altimeter was 29.55. We reset our altimeters and climbed back to 13,000 FT. We requested another ATIS which was completely different from the first one and also incorrect. We listened to the audio ATIS which gave the correct info and said at the end that the ATIS was "unreliable."

The message from dispatch could have been clearer in describing the problem we were about to encounter which I have never in my entire aviation career encountered before (display of completely incorrect ATIS information). For instance: "Attn flight X: Chicago digital ATIS is invalid or inaccurate. Here is the current correct weather at KORD: Do not rely on the ATIS from KORD!" Plain language would have alerted us to the gravity of the situation. Chicago should also not send known incorrect WX information via Digital ATIS. They should shut the system down until it works correctly as expectation bias of always receiving the most up to date ATIS crept into this situation compounding it.

Narrative: 2

Digital ATIS from ORD should have been taken offline or caused to send info that would have lead us to listen to the actual broadcast rather than relying on the ACARS printout.

Synopsis

A crew was on the ORD TRTLL 2 RNAV leveled at 13,000 FT and was queried about their altitude then notified the digital ATIS was wrong and their aircraft was 700 FT low. Digital ATIS nationwide was apparently in error.

ACN: 1255419 (1729 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Other

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1255419
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1255422
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We planned on departing 15R and programmed the FMGCs accordingly. We also obtained performance for 15L in case we got switched to that runway for departure. The we later ordered performance for 15L, expecting to receive an updated Zero Fuel Weight (ZFW) and takeoff weight, in case tower launched us from there. Mid taxi we did get switched to 15L and changed runways and entered the flaps and speeds from the performance printout for that runway and intersection, and then we completed the checklists.

The takeoff was normal with the nose raised at VR and smoothly transitioned to flight, with a substantial amount of runway remaining. After liftoff I notice V2 had dropped below VLS, lowest selectable speed, and managed speed had moved to the upper end of the speed tape. When leveled off at cruise I took another look at our performance printouts and discovered the last performance numbers we received from Sabre, were for 138.3 and not our final weight of 156.2. Minimum Takeoff Gross Weight (MTOG) for that intersection was 169.7. It appears we took off with V-speed for well below our actual weight, which explains the unusual speed bug behavior. What remains a question is how we ended up with Sabre performance for a weight so far below our actual take-off weight.

Narrative: 2

During the preflight the First Officer (FO) had pulled up runway data for full length 15R and 15L. Since we were going west on departure we anticipated a 15R full length takeoff. 15L full length was our backup plan and I thought it would be a good idea to get 15L data from intersection wv as a backup to the backup. 

I requested data from ACARS for runway 15L at [an] intersection using the ACARS runway code. Runway 15R was entered into MCDU with applicable V-speeds, flap setting, and Flex temp. I never looked at the 15LA numbers and did not print them since I thought there was a small chance of using them. I did mention to the FO that those numbers were available on acars.

After a delay for bags we departed roughly 30 minutes late. We performed a single engine taxi across the RA bridge to spot 2 near the taxiway. The second engine was started and we were issued a clearance to taxi for takeoff on 15L at [an] intersection. The runway was changed and the ACARS numbers for 15L were entered in the MCDU. On takeoff roll, upon hearing "rotate"; I executed a nose up, aft stick movement and immediately sensed something wasn't right. The airplane responded sluggishly and I immediately stopped the rotation with main gear wheels still on the runway. I knew we had 12,000 feet of runway and decided to just let the aircraft fly off the runway without increasing AOA.

I did notice Vls approach the speed I was flying after rotation and it got to approximately 10-15 knots below actual speed. Once airborne the gear and flaps were retracted and we flew the departure. After level off we looked at the numbers. The numbers had a gross weight of 138.0 and the 15L numbers had a gross weight 156.2. It was then I realized I must've entered the numbers incorrectly, much to my horror.

Synopsis

An A320 crew, relying on incorrect V speeds and weights, rotated and did not take off. The Captain maintained the takeoff attitude and let the aircraft fly itself off the runway. Once established in cruise, the crew compared the performance printouts and discovered the last performance numbers received from Sabre, were for 138.3 and not the final weight of 156.2.

ACN: 1254921 (1730 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Oceanic

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1254921
Human Factors : Confusion
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Physiological - Other
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

After departing and climbing through approximately FL240 the pilot flying and I noted an unusual vibration that lasted about 10 seconds. We both commented but the vibration ended quickly. About an hour later at FL340 we both noted a steady and unusual humming sound very slowly getting louder. We thought it was located forward of the flight deck. As we discussed it (vent blower fault, nose gear door open, PACK failure etc...) and checked systems it grew into a slight vibration. 

We sent a message describing the unusual loud hum and slight vibration to Maintenance Control via ACARS. 

Shortly thereafter it grew to an intensity that the lead Flight Attendant (FA) called to report an unusual hum and vibration by the flight deck door. I told him we are aware of the situation, all systems are indicating normally, let us know if the noise changes, and check with the aft crew for any unusual noises or vibration. 

In retrospect, having had a Mom that was a public school music teacher for 40 years, I believe the loud hum was pretty darn close to a low E Major chord on a 1965 Wurlitzer organ. I kept that information to myself. 

We detected a distinct smell of burning oil from the air outlets which quickly became light fumes. The First Officer (FO) as PF executed an emergency descent as we began an immediate diversion to a nearby airport. As this began the lead FA reported the smell of burning fumes in the cabin and I quickly told him yes we are diverting now and I'll call him back with his TEST information as soon as I can but start preparing. 

We suspected a PACK fault from prior experience and as we descended through about 13,000 FT I grabbed the COM and as I selected PACK 1 off it was immediately followed by a VENT BLOWER OVRD command. That took all of three seconds and the loud humming immediately stopped and the fumes dissipated as well.

I gave the lead FA the TEST information, made an announcement that of course we had an abnormal condition but we believe we had isolated the problem and we'd be making a precautionary landing. Landed without incident, stopped on the high speed taxiway, and gave the remain seated command. ARFF noted no smoke or visible heat signatures. As the loud humming and fumes had immediately stopped with ECAM actions we taxied to the gate. 

Turns out it wasn't PACK 1 although when I talked to Maintenance Control he said he was getting "all kinds of weird information on PACK 1 on his side." It was the Blower Fan that gradually came apart. I guess it took the strain of turning off one pack to finally and immediately throw the ECAM message and Fault light. 

It would have been easier to use the QRH COM procedure if "air conditioning smoke suspected" was a separate procedure with its own tab.

Synopsis

Early in the flight a hum and vibration were detected, which the A320 crew suspected to be a pack malfunction. Later when fumes developed the crew began to divert to a suitable airport. Once the COM QRH procedure was commenced, a VENT BLOWER OVRD ECAM confirmed a VENT Fan failure.

ACN: 1253887 (1731 of 2908)

Time / Day

Date : 201504

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1253887
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

I arrived at my aircraft that was deplaning. I stepped into the aircraft to put my bag in the cockpit. Walked out and the Lead Flight Attendant said do you know the aircraft was hit by lightning twice going into the previous station? I turned and asked the first officer that was still there and he confirmed that it had been hit twice. The captain of the flight had already left and maintenance was doing a full inspection of the aircraft. I asked the first officer if they had it inspected after the strike and he said no. He then told me that the other captain did talk to maintenance. The other captain came back to the aircraft only to get his water bottle that he had left behind. He heard us talking about the lightning strike and said he didn't want to delay the flight five hours by getting inspection in an out station. He then left without offering to write the aircraft up or make sure the aircraft was ok for service. I told the first officer that I would not accept the aircraft unless it was written up and signed off by maintenance. He agreed to do so and tried calling the other captain but got voicemail. As he was writing the aircraft up for possible lightning strike the other captain appeared in the cockpit. The other captain questioned why the aircraft needed to be written up and I told him the aircraft should have been written up at the station after it was hit. His next comment was what if I had never told you? I looked at him and said that you didn't tell me the Lead Flight Attendant told me. He started to argue with me and I told him to leave my aircraft and that I was calling the assistant chief pilot. I called him and told him what was going on and he completely agreed with me. 

What is really bothersome is that this other captain complete disregard for safety and fellow crew members. There were signs of a possible lightning strike on the radome on captain side of the nose and a very large round area that was just above that. Maintenance told me that was just paint peeling away but I really don't believe them. I do believe that the other captain called maintenance in the previous station and they told him to bring the aircraft back. Proper procedures are to call your dispatcher and then they connect you with maintenance control. The other crew did say they did a walk around of the aircraft there which is not for crew members to decide if the aircraft is safe for flight. There are station notes in that captain delayed boarding for possible lightning strike.

Synopsis

An A320 was hit by lightning prior to landing at the previous station but the Captain failed to have an inspection. On arrival at the next airport, the Lead Flight Attendant informed the oncoming Captain about the strike and no inspection. The previous Captain returned and had words.

ACN: 1253214 (1732 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Commercial
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1253214

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise flight from Airport ZZZ to Airport ZZZ1, we experienced a green HYD overheat that went away after about 30 seconds. A few minutes later, we experienced a green HYD low pressure that did not go away. While running ECAM actions and QRH follow up, we noted the green system reservoir showed 100% overfill. However, the PTU (Power Transfer Unit) was doing its job and supporting the system. 

At this point we contacted Maintenance Control and Dispatch via ACARS with the problem to see if they wanted us to continue to Airport ZZZ1. After several minutes delay and a fuel inquiry, Dispatch advised us they wanted us to return to Airport ZZZ. We advised ATC and were turning back to Airport ZZZ when we had a total failure of the green HYD system. Reservoir content dropped to 0% and ECAM told us to shut the PTU off. At his point the Captain [notified ATC of urgency] and requested vectors to Airport ZZZ2, which was 80 miles north of us and the closest airport. I advised Dispatch we were diverting.

Enroute to Airport ZZZ2 we requested lower and delay vectors to run QRH checklists for overweight landing, Landing Gear Gravity Extension, Emergency Landing Checklist and Non-Normal Landing Distance Procedures. We reviewed the Status page extensively as many systems were effected. 

On the downwind, we configured early, and I landed overweight on [RWY XY] at 145,400 LBS, at less than 300 FT per min. sink rate. Fire trucks were standing by. With no nose wheel steering we were towed to the gate. We got another airplane and continued to Airport ZZZ1. 

A total failure of the Green HYD system. I believe all green system fluid was lost. 

Both the Captain and I were surprised at the lack of advice in the QRH for this particular system failure. However, the STATUS page was extremely helpful in letting us know how we were affected. 

We were both surprised at how long it took to hear back from Dispatch and Maintenance Control concerning our problem. In this situation, someone should at least let us know they have received our messages if they are "working on it." 

I found ATC and dispatch inquiry to be highly distracting while trying to run QRH checklists, but I understand the necessity. I thought my Captain did a superb job as pilot not flying while working with the ECAM and QRH. Extremely professional and we worked well together as a team.

Synopsis

A320 First Officer reported loss of Green system hydraulic fluid in cruise flight. The flight diverted to a suitable airport and landed in emergency condition.

ACN: 1252940 (1733 of 2908)

Time / Day

Date : 201504
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : B767 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7500
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 4800
ASRS Report Number.Accession Number : 1252940
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1252959
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

SFO ground control assigned RWY 28L to us on taxi-out. While #1 holding short of 28L for departure, tower clears us to taxi across 28L and hold short of 28R. First Officer (FO) verbally confirms with tower 28R is our new departure runway. Crossing 28L approach end, taxing North on taxiway C and at the hold short line of runway 28R, FO visually observes an aircraft on short final which appears lined up on runway 28R. We immediately get tower instructions to line up and wait, runway 28R. Captain taxi's our aircraft across hold short line. FO asks tower to confirm the aircraft on short final (B767) is lined up for and cleared to land on runway 28L. Tower confirms. FO, observing the B767, believes this aircraft is lined up on 28R and transmits, "Really?!" Tower immediately instructs the B767 to go around. Captain announces to me intention to continue taxing North, ACROSS 28R and clear of the runway. He never stopped, aircraft was always moving north. We clear 28R as the B767 roars behind us on go-around. The B767 was indeed lined up for and would have landed on 28R. The B767 approximate altitude when go-around initiated-200 ft. AGL. 

In my opinion, contributing factors were:
The airport winds were approximately 170/20G24 which resulted in large crab-angles of approaching aircraft. These large crab angles made it difficult to tell which runway approaching aircraft were lined up on.
SFO was using only runways 28L and 28R for both departures and arrivals. This made communications potentially confusing as tower was switching assigned runways for departures and arrivals with little notice.

Narrative: 2

Flight cleared to taxi to Runway 28L for takeoff. Approaching number one for departure, Tower instructed us to cross 28L and hold short 28R. FO confirmed that 28R was now our departure runway with Tower and he then proceeded to install the new Runway and Takeoff Data. I confirmed the data and we verified 28R. Tower instructed us to line up and wait on 28R. As we taxied forward onto 28R full length it became apparent that the aircraft that we thought was landing on 28L was lined up on our Runway. FO queried tower as to which runway he was landing on and was told 28L. Since he appeared to be on short final for 28R we decided to continue north straight across 28R. He was told to go around and after a short delay we departed 28R normally.

Synopsis

After initially being cleared to taxi to Runway 28L at SFO, an A320 crew is instructed to cross Runway 28L then to line up and wait on Runway 28R. A B767 on final appears to the First Officer to be lined up for 28R, so the Tower is queried. Tower responds that the B767 is lined up for 28L. The A320 crew elects to continue across 28R as the Tower issues a go-around to the B767.

ACN: 1249278 (1734 of 2908)

Time / Day

Date : 201503

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 6500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Nav In Use : FMS Or FMC
Nav In Use : GPS
Nav In Use.Localizer/Glideslope/ILS : Runway 19R
Flight Phase : Final Approach
Route In Use : Direct
Route In Use : Visual Approach
Route In Use.STAR : GRNPA1
Airspace.Class B : LAS

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 70
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1249278
Human Factors : Confusion
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1249886
Human Factors : Training / Qualification
Human Factors : Confusion

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

ATIS reported visual approaches (not CHARTED VISUALS) to 25L and 19 L/R. Flights ahead of ours were getting 25L and we were on the GRNPA arrival. We briefed a visual to 25L. Upon our change over to Vegas approach we received a change to 19R. We were short on time to make the change. In the rush to change to 19R neither of us found an RNAV visual approach due to the placement in the iPad Jeppsen [FD PRO]. We found an RNAV to 19R but it was GPS. Since we were cleared for the visual we loaded that into the FMC. We were both aware of terrain and selected a safe altitude and made a slight turn to be sure to avoid the terrain and started a decent. ATC noticed our course change and questioned which approach we were flying. Advised we were on the incorrect approach. It was not until we parked that we were able to find the RNAV VISUAL to 19R.

Narrative: 2

We were on the GRNPA RNAV arrival to LAS and assigned direct TRROP and 250 KTS. Previous aircraft had been sent to RWY 25L and we were briefed and programed for the same. The ATIS reported visual approaches to 25L and 19L/R. Prior to TRROP about 15 nm from LAS we were cleared for RNAV Visual 19R. We opened the APP button in Jeppesen FD PRO and scrolled down to RWY 19R and only found one approach, and that was the RNAV (GPS) 19R. Thinking that was what ATC meant FO replied RNAV GPS 19R and ATC countered with RNAV Visual 19R.

Not seeing any other options we felt that ATC was combining a visual approach and the RNAV. So with terrain and airport reported in sight we inserted that approach and direct to JOGMU and turned right and started descent to 5300 feet. Around 6000 feet ATC complained that we were North of course and to climb to 6500 feet, which we did immediately. We reported the field in sight again and was cleared for a Visual Approach 19R and to slow to final speed. We were high and fast, but a slight purposeful overshoot and level S turn to final got us quickly configured and back on profile for a normal visual approach and landing.

Once at the gate we found the RNAV VISUAL 19 R/L approach under the General category not under RWY 19R. I assume it is placed there because of the 19 R and L designation, but that is not intuitive. It should be listed under each runway it serves and given a distinct name such as Newark's Turnpike Visual 4L/R, or the River Visual to DCA. Then the ATIS could list the desired Visual approach, which we carefully read at cruise altitude and carefully program with no other distractions. Also, I'm sure ATC could have told us sooner their plan for our runway assignment than to wait until inside of 20 nm out. In hindsight, it's dangerous to make assumptions and I could have refused the last minute assignment and declared we were only set up for the 25L approach or take radar vectors to 19R visual.

Synopsis

A320 flight crew reports being cleared for the RNAV Visual to Runway 19R at LAS but can't locate the procedure on their iPads. The RNAV (GPS) 19R is loaded believing this is what the controller meant. It is not, and vectors are issued for a standard visual approach.

ACN: 1248202 (1735 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : EAGUL6

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1650
ASRS Report Number.Accession Number : 1248202
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

The guidance system was programed for a west landing in PHX which is usually how PHX is organized in the afternoon. On initial contact with approach, we were assigned a descend via, landing east.

At that point the cockpit was very busy trying to comply with new company procedures which require a lot of search and typing on the computer. The correct runway was neglected. The aircraft began a turn toward the wrong arrival path, when detected, we immediately entered the correct fix and went direct resulting in a 1.5 nm deviation from cleared track. At the same time, the aircraft was descending for the wrong fix, and caused us to be below the published altitude for the correct fix. There was no loss of separation.

The crew was task loaded with new procedures and software without sufficient training. The "training" being watered down "handouts" that read like stereo instructions from days gone by. It feels like companies are really skimping on training to save money. With the advancement in the aviation system rapidly accelerating, classroom and hands-on training is a must!

Synopsis

An A320 First Officer experiences a runway change while descending on the EAGUL6 which is not correctly executed in the FMGC, resulting in descent below the next crossing restriction. New procedures and software without sufficient training were cited as contributing factors.

ACN: 1246705 (1736 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : TNCM.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : TNCM
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1246705
Human Factors : Workload
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1247150
Human Factors : Time Pressure
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Dispatch
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We got on the plane in TNCM prior to schedule departure, at that time we requested the release and TPS, we were told it was not available but will be coming shortly, passenger boarding began, as we were setting up we noticed that the ACARS were not available and with no paperwork we were unable to set up the FMGC. As we waited we observed all the planes including company airplanes taking off and landing on Runway 10 into a 14 knot headwind, it was a busy day in TNCM this being the weekend. We tried to reset the ACARS with no success, and departure time went by with the passengers onboard and still no release and no tps, without cellphone service we were unable to contact dispatch all operations was telling us was that the paperwork was coming. 20 past departure and the passenger were starting to be agitated as they have been on the plane for an hour. 25 minutes past departure the paperwork showed up and the main door closed and they were ready to push us back, however we were not ready. After loading the MCDU and the performance page prior to push back (in accordance with procedures that became effective the day before) we are also using a hard copy of the load close out as we could not use the ACARS. After route verification, briefing and checklist we are ready to push. Ground control informed us that we did not have a flight plan on file, again we go thru ops and spend the next 30 trying to get a flight plan refilled, and a clearance. 1h15 past departure we takeoff, takeoff was uneventful and we get enroute. Once we got the ACARS back dispatch told us that the performance numbers provided to us were for taking off Runway 28 with the strong tail wind and against the flow of traffic and that by taking off on 10 we were above the maximum takeoff weight for that Runway.

- Tps was only showing numbers for one Runway, I must have assumed it was the one in use and overlooked that it was in fact for 28.
- Under the numbers for Runway 28 there was a note regarding Runway 10, regarding special departure,reinforcing my mistake that this was for Runway 10.
- Before [the procedure change] using uplink you would have been unable to insert number for a Runway that was different from your take of Runway on the flight plan page.
- Getting the paper work including TPS on our second flight under the new procedures 20 past departure.
- Being unable to contact dispatch or have dispatch contact us.
- Being unfamiliar with TNCM.
- Task loading and multiple distractions from ATC, ops, Flight Attendats wanting to know what was going on.

Narrative: 2

Crew arrived at the airport 1+15 prior to departure [and] at aircraft 1+05 prior to departure. Agent said that paperwork was not ready and would be brought out later. Did pre-flight checks but ACARS said no comm and we were unable to initialize the IRS's and the MCDU Init page was frozen and would not allow any entry. 
We pondered the problem for 10-15 minutes discussing our options and the First Officer (FO) tried deselecting and reselecting the database and that allowed us to get the IRS's aligned and to enter some information but the flight plan did not load so we were left waiting on the flight plan. We reviewed the runway 10 departure because of the unique terrain features it was noted that tailwind takeoff's were prohibited. We were located on the remote pad facing South so we just waited and watched 15-20 and takeoffs and landings on runway 10 which was right in front of us.. 

At departure time one of the agents came up and said that they were having problems getting a final fuel load but they would have one shortly. The agent also told us that since the changeover to the new system that they had been taking delays. 

We still did not have our paperwork at departure time +20. Approximately 10 minutes later the fuel truck showed up and started fueling. The agent wanted to know our fuel load to tell the fueler but I told him that that information should come from ops to be sure the load was correct. By now we are well past departure time. I did hear the FO tell tower that we were 20 minutes from engine start. A few minutes later another agent showed up with the paperwork. The FO entered all of the data and we then verified our flight plan. A takeoff on runway 10 was briefed by the FO who was the pilot flying. On the TNCM field report I noticed that runway 10 was listed first and that 28 was open for departure but only if the winds or direction change but the winds were predominantly out of the southeast. I noticed that the weather at destination was poor. I believe it was 400 OVC and 1/2 mile. I then shifted my focus to the flight plan to see what our fuel status was. The FO looked at the paperwork and noticed that we had a re-release so a discussion of that took place because I had never done a re-release before. 

We were ready to go and informed tower of such and was advised that we could push. Tower then had us stop because he did not see our flight plan. There was no clearance on file. I went off frequency and spent about 5 minutes on that issue. After another 10 minutes we still did not have a flight plan so I called ops again and the clearance was re-sent. It went through this time but we still waited about 10-15 more minutes for clearance from San Juan. We were now an hour late having shown up on time and boarded on time. We taxied out and was about 200# from our takeoff min and we were given a re-route. We finally departed with the FO as the pilot flying off of runway 10 and our climb to cruise was uneventful. 

After level off, we got a message from dispatch wanting to know our winds. We sent back 140/14. He then sent us a message wanting our max tailwind. I sent back that we took off on runway 10 with a headwind. 

We were puzzled by this exchange so we started double checking everything and discovered that we had been flight planned for a tailwind takeoff on runway 28 against traffic. This is when we realized that our data entry was applicable to runway 28 and not runway 10. Upon landing I called the dispatcher and he informed me that we had been overweight for runway 10 for second segment climb performance. He also stated that he had contacted ground personnel in St. Maarten and had told them to tell us to come back to the ramp to offload passengers and bags. We were never at any time told to return to the gate by anyone. 

Our workload and stress level increased dramatically when we had no comm with the company and could not load any data or align the IRSs. No provision for an error to be caught on flight closeout with new procedures. We were extremely time compressed receiving the paperwork at departure time. It seemed like we were working 4-5 issues at once. Our focus was on our legalities and safe operations off of runway 10. It did not enter our thought process that we would be given a tailwind takeoff against traffic on runway 28 given the field report. We sat for over an hour and never saw a takeoff on runway 28. 

There was no reason to get the paperwork at departure time. Due to extreme task loading out CRM broke down allowing an error to go un-noticed. A method of communication with our dispatchers when operating out of a foreign country with no Comm. Give a complete closeout including the runway in use. You may be legal but not safe. A crosswind takeoff out of TNCM with a 7,500 foot runway may not be entirely safe. Get the paperwork upon arrival at the aircraft.

Synopsis

A320 flight crew describes the problems departing TNCM due to late paperwork, procedure changes and no ACARS. The flight departs over an hour late and does not use the runway planned by Dispatch and shown on the weight and balance.

ACN: 1246531 (1737 of 2908)

Time / Day

Date : 201503

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1246531
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Not one part of this pushback from this station's personnel followed SOP - not the verbiage, not the hand signals, nothing. The most egregious was that I never got the arms at 45 degree signal (aircraft is clear/ready for next) from the marshaller. All he did was give a salute-like gesture then he walked away. I tried to call Ops to get someone out there who actually knew what they were doing, but, or course, no one would answer the radio. This seems to be the case more and more as we outsource our stations. I'm not sure if is a lack of training, lack of ability, or lack of caring, but these outsourced stations rarely follow SOP and are a huge safety risk.

Synopsis

An A320 Captain noted that as station operations are outsourced, adherence to ramp SOP is lacking. At this Station, the pushback verbiage and hand signal were not employed.

ACN: 1245629 (1738 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MDPC.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1245629
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This incident started about 5-10 minutes after the last passenger deplaned. Doors 1L and 2L were both open, ostensibly for deplaning and boarding. There were many cleaners on aircraft from the front to the back. The situation was busy at best, frantic at worst. I was standing on the front air stairs when a Flight Attendant asked me if I could supervise her opening door 2R. The cleaners were beating on the door to have it opened so that the trash could be emptied. I agreed to supervise as no other crew members were available. I followed her to the back of the airplane where various ramp personnel were in aft galley conducting their work. She proceeded to arm the door as it had been disarmed from deplaning. It was at this point that I became confused. Before I could intervene, she pulled up on the handle, the door opened and the slide blew. She looked back at me in shock and said, "Oh my god, I am so fired!" She was visibly shaken and in shock as was I.

I should have done a better job confirming what was actually going on and trying to slow the process down. This included finding another flight attendant, pulling out manuals or rechecking door disarm indications in the cockpit. The overall issue for me was being distracted, rushed and uncertain of my supervision objective. I also believe she was trying to do the right thing, especially as a new employee. She was rushed and getting pressure from the cleaners. In retrospect, I think she was operating on muscle memory. Since she had already disarmed the door, the next event was to arm it. Additionally, she has a great attitude and was just trying to help others do their job. Procedurally, this might be avoided if aircraft crew deplanes immediately and gets out of the way of outstation cleaners. Or a procedure needs to be implemented as it relates to who is authorized to open and close doors during cleaning.

Synopsis

A320 Captain reports being asked by a new Flight Attendant to monitor her actions while she disarms and opens an aft galley door. The door is already disarmed but is rearmed and opened before the reporter can react, blowing the slide.

ACN: 1245596 (1739 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : System Monitor: Indicating and Warning
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : EICAS/EAD/ECAM
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Cargo Compartment Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1245596
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

In cruise mid-way through the flight, we received a SMOKE FWD CRG DET FAULT message. We completed the ECAM, completed the Cockpit Operating Manual (COM), and then looked for a COM reset. The reset in COM 3.224.15 says "On Ground" only, so we did not perform it. As soon as I finished looking up the reset (within a minute or two of the first ECAM), we then received a SMOKE AFT CRG DET FAULT message in addition to the SMOKE FWD CRG DET FAULT message continuing to be displayed. The COM procedure was the same for both messages, and the reset was not applicable to either message.

We both considered it unusual to receive one of these messages, and then the other a couple of minutes later. It seemed more likely that a Cabin Intercommunication Data System (CIDS) fault would cause both of these messages to be displayed simultaneously. We considered the situation to be additionally unusual because the Airbus Flight Warning Computer (FWC) software did not have a combined message for this situation (i.e. FWD + AFT), and therefore it seemed to be a situation that is considered not likely to occur.

The COM "condition" for each of these messages is "Smoke detection fault." The ECAM Status page displayed, under INOP SYS, "FWD CRG DET" and "AFT CRG DET." Based on both the COM "condition" and the ECAM Status, we both believed that all cargo smoke detection capability was lost (i.e. both CIDS cargo smoke detection channels for FWD cavity 1 and both channels for AFT cavities 1+2). Now, I wonder if this is ambiguous, and if we would have received the SMOKE DET FAULT message if that were the case.

After receiving the second ECAM, we contacted the flight attendants, explained the situation, and asked them to walk up and down the cabin and report back if there was any smoke, hot spots, odors, etc. They called back within a couple of minutes and reported none. We asked them to walk the cabin approximately every 10 minutes until we gave them the "flight attendants prepare for landing" PA.

We advised Maintenance Control that we had received the two ECAMs and that there was no evidence of smoke. Maintenance Control replied, asking us to try the reset on COM 3.224.15. We advised Maintenance Control that the reset said it was applicable "On Ground" only. Control replied, again asking us to do the reset. We replied, explaining that we were not comfortable with that without them explaining their reasons for requesting this. Maintenance Control replied without mentioning the reset any further, and advised that local Maintenance had been notified of the faults.

We discussed the situation, again reviewing that it was unusual to us that we would receive the FWD message, and then a couple of minutes later, the AFT message. We discussed our belief at the time that all cargo smoke detection capability was lost. We considered asking the ground crew, on arrival, to not open the cargo doors until cleared to do so by local Maintenance. However, we discussed that Maintenance would not be able to help us in the extremely unlikely event there actually was fire/smoke. We agreed that, out of an abundance of caution, and to ensure the highest level of safety for our passengers and ground crew we would:

1) Request ARFF escort us to the gate.
2) Taxi to the gate and deplane normally after landing, without stopping for inspection.
3) Instruct the ground crew to not open the cargo doors until cleared to do so by ARFF.

We informed the flight attendants of our plan, but stated we would not make a PA to the passengers until after landing, in order to prevent panic/hysteria/uncommanded evacuation. The Captain reviewed with the flight attendants procedures in the highly unlikely event of actual fire/smoke/need for evacuation at the gate. All communication with the flight attendants ensured clear understanding that we only had a detection fault--not an actual smoke indication--and this was merely contingency planning in the extremely unlikely event evacuation became necessary.

We messaged Maintenance Control our plan, who forwarded it to Dispatch. We requested they coordinate with both ZZZ ATC to arrange ARFF and with ZZZ Ops to inform the ground crew. Dispatch replied that this would be done.
I explained our plan to ZZZ Operations (Ops) when calling in-range. When we checked in with ZZZ Approach, they had not been informed of our situation, so I fully explained our plan to them. We were extremely clear that we were not declaring an emergency. I reiterated our plan to ZZZ Tower, who relayed it to ARFF. After landing, I re-confirmed the plan with ZZZ Ops while the Captain communicated directly with ARFF. I made a PA explaining that airport safety vehicles would follow us to the gate due to a maintenance issue, that there was no cause for concern, and that we would be deplaning normally once we reached the gate.

After parking, ARFF contacted us, stating that there was no evidence of fire or smoke, and no hot spots on thermal imaging. They cleared the ground crew to open the cargo doors. I confirmed with ATC that they did not need a written report from the company. The Captain spoke with the ARFF Incident Commander in person, and then called the Chief pilot.

Maintenance came on board and began troubleshooting. After engine shutdown, the ECAM Status displayed several items that were not displayed in flight. If I recall correctly, I believe they were INOP SYS "CIDS" and Class II Maintenance Status messages "CIDS 2" and "SMOKE." After maintenance troubleshooting, the faults were fixed, and we continued on our next leg with no further issue.

The COM "condition" for SMOKE FWD (AFT) CRG DET FAULT should more clearly indicate whether this is a single-channel or dual-channel "Smoke detection fault." I would appreciate it if the company could contact Airbus and request clarification of what is the difference between "SMOKE DET FAULT" and the combination of the two messages "SMOKE FWD CRG DET FAULT" and "SMOKE AFT CRG DET FAULT".

The COM procedure for SMOKE FWD (AFT) CRG DET FAULT applicable to our aircraft does match Flight Crew Operating Manual (FCOM) PRO-ABN-26 P 8/70 dated JAN 2015, but neither paper procedure reflects that the ECAM Status page will display FWD (AFT) CRG DET under INOP SYS. This should be investigated.

AOM 1.26.50.8 dated Oct 2012 should be updated IAW FCOM DSC-26-50-40 P 1/2. The vast majority of AOM-1 has not been updated IAW [Airbus] FCOM DSC in years. Our outdated AOM-1 refers to a SMOKE LAV + CRG DET FAULT message that can no longer be generated (it is replaced by SMOKE DET FAULT).

Maintenance Control should not request that we perform "On Ground" only COM Supplementary resets in flight without an extremely clear explanation of why they are recommending such action. IAW COM 3.224.3, "In flight, as a general rule, the crew must restrict computer resets to those listed in the following tables, or to those in applicable OEBs. Before taking any action on other computers the flight crew must consider and fully understand the consequences."

The company should have contacted ZZZ ATC to arrange ARFF, as we requested and Dispatch replied would be done. We should not have relied on the company to arrange this, and should have asked Center to relay this to ZZZ Approach ahead of time.

Synopsis

A First Officer (FO) describes the procedural steps that he and the Captain followed that led them to believe their A320 aircraft had lost all cargo smoke detection capability after receiving two ECAM messages for SMOKE FWD CRG DET FAULT and SMOKE AFT CRG DET FAULT in-flight. The ECAM Status page also displayed, under INOP SYS: "FWD CRG DET" and "AFT CRG DET." After engine shut-down, ECAM Status displayed INOP SYS "CIDS" and Class II maintenance status messages "CIDS 2" and "SMOKE."

ACN: 1245578 (1740 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aero Charts
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1245578
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

I reported for an all-nighter to ZZZ. I was in the back computer area checking in and trying to connect my iPad to the Wi-Fi provided. I knew I had one update that was needed in my Jeppro app. I tried numerous times to get the update to load but it would not finish the process. 

At that time I decided I need to get to the gate for our departure time. I figured I could update at the hotel when I arrived for the overnight. Upon checking into the room I went to work and had the same result of not being able to finish the download. I have read in [Company] messages hotel have trouble sometimes with bandwidth, I believe I experienced that issue at the hotel and in the computer area of the lounge in ZZZ1. At this point I went to bed and was going to try in the morning before our departure for the airport. To no avail the update would not upload. We proceed to ZZZ and as we our running our preflight duties a federal inspector came on board to preform and line check. I immediately notified him of my situation of lacking an update in my Jeppro app. He told me I need to ASAP this situation, I have read the [company] messages about keeping your iPad up to date. All I can say is that I tried to comply to the best of my ability in this situation. 

The federal inspector did provide good insight on how to avoid this in the future as in, check the airport Wi-Fi for example ZZZ is free, mobile hotspot off your phone. That is what the inspector did to get the update on my iPad. I can say I have gone this route and now have mobile hotspot on my phone to prevent this from ever coming up again. I do believe with my mobile hotspot and how the inspector went through the steps on using one and now having my own this will never happen to me again.

Synopsis

An A320 pilot had been unable to update his IPad at several different locations. An FAA inspector, during the course of a line check, showed him how to use the airport Wi-Fi to update his IPad.

ACN: 1244954 (1741 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 1244954

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Pre-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon boarding the aircraft to do preflight duties we immediately encountered a rancid heavy pungent odor permeating the entire aircraft. It smelled of rotten eggs, wet socks or dirty laundry smell. Even with heavy deodorizers the smell continued. We began to feel very nauseated. We reported it to the Captain who said it was fine and we were taking it as is.

We then taxied out onto the runway and mid demo the engine roared to make a turn. Then the air from the bleed air in the aft galley vent blew air onto my face while reading the demo. I lost my breath, became dizzy, started coughing and wheezing and had difficulty breathing. I felt disoriented with a severe migraine, nausea and tremor on the left side of my body. My heart began racing and I felt very ill. I tried to finish the demo as quickly as possible. We made an Emergency call to Captain stating that myself and several passengers in the aft cabin were feeling very ill from the noxious fumes. He decided that a gate return was in order. I felt as though I was losing consciousness so with the assistance of [another] Flight Attendant I obtained a POB and took oxygen which helped the racing sensation in my heart and the dizzy disorientation.

Maintenance did not replace or address the Wet Seals where the toxins were coming from. Instead they simply ran the engines for 20 minutes and changed the filters. After this I was told that there was no Flight Attendant available and if I went home sick then the flight would cancel. So I felt pressured to continue the trip. I arrived very sick from the chemical exposure. I will work my flight home and then will go to a [facility] for chemical exposure testing upon arrival [at home].

We need a Cabin Air Quality program at the airline and a mandated mandatory Wet Seal change out program for our aircraft bleed air systems in order to prevent future Fume Events and Aerotoxic Syndrome exposures to our Cabin Crews and Passengers.

Synopsis

Flight Attendant reported becoming ill from noxious fumes (that smelled of rotten eggs or dirty socks) in the cabin.

ACN: 1244859 (1742 of 2908)

Time / Day

Date : 201503
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Route In Use.STAR : PHLBO
Airspace.Class B : EWR

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 7490
Experience.Flight Crew.Last 90 Days : 49
Experience.Flight Crew.Type : 49
ASRS Report Number.Accession Number : 1244859
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15653
Experience.Flight Crew.Last 90 Days : 211
Experience.Flight Crew.Type : 7818
ASRS Report Number.Accession Number : 1245231
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were arriving into Newark from the south on the PHLBO arrival, and Newark was landing on 22L. We were still south of the airport and at 3,000 feet when approach control vectored us off of the arrival to give us a right downwind heading for a 22L landing. At this point the approach controller asked us if we had the airport in sight. We confirmed with one another that the field was indeed in sight, and the captain told the controller that we had the field in sight. We were then cleared for a visual approach to runway 22L. At this point we were on a wide right downwind for the runway. The captain said he would set in 1,500 feet as that was the final approach fix altitude for runway 22L. I agreed and confirmed the selected altitude, and began a descent down. Both the captain and myself had terrain selected on our NDs, and nothing indicated that we would have any issues. As we were descending to 1,500 feet, somewhere around 1,700 feet or so we got a GPWS "Terrain, pull up". At that same time the approach controller came on and told us that the minimum vectoring altitude in that area was 2,000 feet, of which we had no idea, and that we should climb back up to 2,000 feet. I had already begun climbing as the captain was resetting 2,000 feet. As soon as I began initiating a climb the GPWS terrain warning went away. I leveled the aircraft off at 2,000 feet, for a total climb of approximately 300 feet. After we returned to 2,000 feet we continued with the visual approach to a normal landing on runway 22L without incident.

Narrative: 2

We were descending via the PHLBO arrival into to EWR when we were vectored off the arrival for a visual approach to runway 22L and given clearance to descend to 3000 ft. Approximately 10 miles SW of the airport NY Approach asked if we had the airport in sight. After conferring with the First Officer (FO) that we both were seeing the airport, I replied that we had the field in sight. At that time we were cleared for the visual approach to 22L. I set and confirmed an altitude of 1500 in the MCP for the planned FAF crossing at BUZZD. The FO began an Open Descent to 1500 and briefed that he would descend first and then slow the aircraft as we approached BUZZD on the base turn. As we descended through 2500 ft., we were handed off to EWR Tower. Before checking in I observed some green terrain depictions on my ND and could see what appeared to be a dark area separating steady ground lighting out the window in front of us. Before I could contact EWR tower, they contacted us and advised us that we had descended below their MVA for that area and cleared us to climb to 2000 ft. for terrain. At this time we were at about 1700 ft. As we were receiving this advisory and clearance, the GPWS warning activated with "terrain, terrain." I was already setting 2000 ft. into the MCP window as the first officer initiated a climb, and as soon as we started the climb, the GPWS warning ceased. After leveling, the tower informed us that there was a ridge below us in that area, but that it was no longer a factor and we were re-cleared for the visual approach to 22L. The remainder of the approach and landing proceeded normally. 

The Threat in the case was obviously the terrain southwest of EWR. Our error was not being aware of that area of terrain, which resulted in an ATC low altitude warning and a GPWS activation which was a surprise to us both. While SOP would dictate performing the escape maneuver, we elected to not perform that maneuver in light of the fact that we had the conflicting terrain in sight and we had a corrective clearance from ATC for a climb to 2000 ft., which was only 300 ft. above our current altitude. In my judgment, performing the full escape maneuver would have been far too aggressive for the correction needed and would have potentially placed us in conflict with our clearance and any overlying traffic. Instead, the FO disengaged the autopilot and auto thrust and performed an immediate, but normal climb to 2000 ft. at which time the GPWS warning ceased. It should be noted that the warning we received was a GPWS, not an EGPWS warning.

Factors that contributed to our error would include accepting a night VMC visual approach clearance while 8 to 10 miles from the field, an assumption by me that terrain is not a threat in this area, so I was not concerned about the FO's descent and approach planning, and not giving more attention to the green EGPWS depiction on the NAV display. I have seen this before and I interpreted it to mean the terrain was well below us. While we did not receive any EGPWS warnings, clearly our proximity to the terrain was close enough to activate the radar altimeter based GPWS, at least momentarily.

In the future I will not accept visual approaches at night unless we are established on or near the final approach course with a clear visual indication that terrain is not a factor for us. I will give more consideration to remaining at or above the airport's MSA until clearly in a position to establish a visual descent path to the runway on a visual approach. I will give more consideration to any EGPWS depictions of terrain in our path and not assume that a green depiction is a "safely above" indication. While we did experience an unwanted aircraft state on this approach, the warnings received from both ATC and the GPWS system alerted us to our error and allowed us to correct out flight path in a timely manner.

Synopsis

A320 flight crew reported triggering a terrain warning from the GPWS when they descended below MVA on a night visual approach to RWY 22L at EWR.

ACN: 1243999 (1743 of 2908)

Time / Day

Date : 201503

Place

Locale Reference.Airport : PDX.Airport
State Reference : OR
Altitude.MSL.Single Value : 5200

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : P80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : HHOOD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1243999
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

First Officer (FO) flying HHOOD2 arrival, VMC until about 7500 feet. ATC gave us descent to 6200 feet and we were in clouds at that point. Received further clearance to descend to 5000 feet. During descent at about 5300 feet [GPWS] "terrain terrain" announcement sounded. FO reacted with power and pitched up to stop announcement. We told ATC and broke out of clouds. I assume GPWS was a closure rate warning.

Synopsis

An A320 Captain reported receiving a GPWS terrain alert on arrival into PDX in IMC. The pilot flying executed an escape maneuver. The reporter feels it was likely a closure rate alert.

ACN: 1242776 (1744 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 22000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : IVANE 5
Airspace.Class A : ZDC

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1242776
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1242778
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Initial cruise altitude FL180 with Washington Center. We later climbed to FL240 due to turbulence. We were given the clearance descend via the IVANE5. We set the bottom altitude of 6,000 feet and armed the descent mode 10 miles prior to top of descent. Aircraft started down and crossed MAYOS intersection at 280 knots and FL220, MAYOS has a constraint of 280 knots and cross between FL260 & FL240. 

Because of the initial cruise altitude of FL180, the higher altitude constraints were deleted by the FMS. After receiving a climb clearance to FL240, we manually re-entered the higher constraints. For MAYOS we entered 280KTS and a hard altitude of FL240 (our current cruise altitude). All constraints were then re-checked by both pilots and were properly displayed on the navigational display. Approximately 10 miles prior to top of descent I armed the descent mode. At some point the FMS deleted the MAYOS altitude constraint and descended to FL220 which is the next constraint for KIYEN intersection. This resulted in a low crossing over MAYOS. For many years prior to the IVAN arrival, MAYOS had a FL220 crossing altitude. I believe this helped contribute to our missing the FMS deleting the FL240 constraint over MAYOS. 

While flying any RNAV arrival in the Airbus, I would suggest to continually cross check the RNAV chart to what you see on the navigational display throughout the descent. This would enable the pilots to catch any FMS constraint deletions that may occur unexpectedly after the initial verification (by both pilots) as required by SOP's.

Narrative: 2

[Report narrative contained no additional information].

Synopsis

A320 flight crew reported they overshot a charted restriction on an RNAV arrival into CLT when the FMS dropped an altitude constraint and the crew did not catch it.

ACN: 1242468 (1745 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Navigation Light
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Wingtip
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1242468
Human Factors : Confusion
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I did the walk-around prior to our flight. The navigation light switch was in the # 2 position which I've always known to illuminate the aft (outboard) Nav lights on the wing tips. The right wing tip was illuminated correctly but the left wing tip light was illuminated in the # 1 position (forward and inboard). When we switched it to the # 1 position then it illuminated the aft and outboard bulb. This seemed backwards from normal so we wrote up the discrepancy and after an hour of research maintenance said it was normal. We both explained that we had never seen it before but they said that was how it was supposed to work according to their wiring diagram. So either we are being taught incorrectly and our FM is vague or the maintenance document is incorrect. But I have never seen two different bulbs illuminated on opposite wing tips with a given switch position.

Callback: 1

Reporter stated he saw the same A320 a week later and the same NAV light discrepancy still existed. He still suspects that having two different bulbs illuminated on opposite wing tips with a given switch position is not correct. Because if that opposite light bulb configuration is actually correct, that means all the other A320 aircraft are incorrect; which doesn't seem likely.

Synopsis

During a pre-departure walk-around, a First Officer (FO) notices two different light bulbs would illuminate on opposite wing tips when the A320 Navigation light switch was moved to either the # 1 or # 2 position. When in position #2, the right wing tip outboard aft Nav light was illuminated correctly, but on the left wing the inboard forward bulb was illuminated.

ACN: 1242462 (1746 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : IAD.Airport
State Reference : VA
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class B : IAD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 3560
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 3117
ASRS Report Number.Accession Number : 1242462
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Washington approach was vectoring long down winds for runway 1R. 1L was unprepared and 1R was reported braking action fair. We were at 4000 feet and approach continued us on downwind after the last fix on the HYPER5.01R. ATC told us that they were delaying our turn a bit at 18 DME AML. At 20 DME AML, I told the Captain (CA) that we were exiting Class B airspace and ATC should have told us according to JO 7110.65V. ATC turned us to 280 and descended us to 3000 MSL (floor of Class B from 20-15 DME AML). Just inside 20 DME, ATC told us to descend and maintain 2000 which would put us below Class B floor without giving us specific authorization to operate below Class B. This is not legal. I told Captain I was going to delay descent in an effort to stay above the floor. ATC then repeated the descent clearance at about 17.5 DME, and we told him that we were endeavoring to stay above Class B floor which is required. We were then switched to tower without incident or comment. We heard approach tell traffic following us that he was descending them below class B airspace, which is what he should have told us in conjunction with our clearance to 2000 MSL.

Synopsis

A320 First Officer reported being cleared below class B airspace without notice from ATC.

ACN: 1242084 (1747 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Deplaning
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1242084

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While I attempted to disarm the boarding door (L1) to deplane I was unable to lift the arming lever to disarm without using excessive force. I asked Flight Attendant D to try as well, but she couldn't. The cabin pressure light was never illuminated in the door window. I then called the Captain and informed him of the situation. He attempted to disarm the door as well. We had the gate agent wait before opening the door and the captain contacted maintenance. We continued trying to disarm the door and I would say after 5 minutes and maybe ten times attempting to disarm, the arming lever finally budged and the gate agent opened the door and we disarmed all the other doors in normal fashion.

Synopsis

Flight Attendant reported considerable difficulty disarming door L1 after gate arrival. Several other crewmembers tried without success also, and after about five minutes they were able to disarm the door.

ACN: 1241285 (1748 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Pneumatic Control Valves
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1241285

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climb-out Engine 1 NAC (Nacelle) 300 degrees flashed. Also noticed Bleed Air High Pressure valve open on Engine 1. Output of Intermediate Pressure valve showed matched temp and PSI comparing Engine 1 and Engine 2. Leveled at FL250 per ATC. Engine 1 NAC message became constant. Reduced thrust on Engine 1 and NAC temp message went away with almost idle thrust. Suspected bleed air leak since low power was only way to keep NAC temp message off. Made decision to return to Airport ZZZ. Accomplished overweight landing checklist with estimated landing weight of 160,200 lbs. Uneventful landing with minimal sink rate. Had fire and rescue inspect Engine 1 for possible fire damage. No damage noted and taxied to gate. Maintenance personnel met plane and I wrote up Engine 1 NAC. Maintenance personnel told us not to write up overweight landing due to low sink rate on landing. I should have written up overweight landing according to checklist procedures. 

First officer did a great job flying the airplane and coordinating with ATC to expeditiously get plane to ZZZ while I coordinated with company through ACARS. All flight attendants did an excellent job of getting cabin ready for precautionary landing.

Engine 1 High Pressure valve was stuck open causing NAC message and possible bleed leak.

Synopsis

A320 Captain reported returning to destination airport when they received a nacelle high temperature ECAM warning that was apparently related to a High Pressure bleed valve stuck open.

ACN: 1241271 (1749 of 2908)

Time / Day

Date : 201502

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1241271
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Observer
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

The previous hours report showed rain and snow earlier in the AM hours. It was VFR upon our arrival at the airplane. The FO performed the walk around and returned with the assessment that all moisture visible on the wing surfaces was water and nothing was adhering to the plane.

We taxied toward the runway and the aft FA called on the interphone and said a deadheading pilot saw ice on the wings. We altered our taxi course to go through the deicing bays for type 1 fluid application and departed without incident.

After takeoff, I discussed with the FO where he viewed the wings from during the walk around and he said from the jetway stairs and in front of the airplane nose.

We reviewed the interior aisle overhead markings as the best view of the top of the wings as another resource for this evaluation.

Synopsis

A320 Captain reported they departed the gate with undetected ice on the wings that was brought to their attention by a deadheading crew member.

ACN: 1241003 (1750 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Snow
Weather Elements / Visibility : Rain
Work Environment Factor : Poor Lighting

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1241003
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Weather
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were two and a half hours late due to a maintenance issue which allowed the next major snow storm to arrive before we were able to leave BWI. So we had to deice.

There were two issues concerning the deice procedures. One, the pad is not well lit. After being instructed which pad to use, I found it difficult to see any markings and determine safe clearance from the other aircraft. I stopped and one of the deicing crew marshalled us in. Under snowy conditions, the marshaller should be in place to guide the plane in before clearance is given to proceed.

Two, after deicing was complete, we had to ask a couple of times for the start times of the applications and to verify that both Type 1 and 4 fluids were used. (With the completion radio call, they said the equipment was clear, but both my First Officer and I thought they were in our taxi path; they were well clear when we were ready to move.) When we first tuned into the ICEMAN frequency, the preceding aircraft had the same issues. 

We did a wing inspection (although not required) and were satisfied that the job was properly done. The deice crews should be trained not only in procedures for deicing the airplane, but also in what information has to be relayed to us before calling the procedure complete. Also, equipment should really be out of the way, before calling all clear.

Synopsis

An A320 flight crew experienced problems with deicing. The pad is not well lit and marshallers were not in place to provide guidance. Additionally, the de-ice crews had to be called a couple of times for the start times of the applications and to verify that both Type 1 and 4 fluids were used. Also, there was confusion as to whether or not the equipment was actually clear.