ACN: 1240834 (1751 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None

Component : 1

Aircraft Component : Pax Seat
Aircraft Reference : X

Component : 2

Aircraft Component : Toilet Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
ASRS Report Number.Accession Number : 1240834
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

Upon taking over the aircraft and doing preflight. I noticed there was biohazard in row 28 and on seat carpet area of seat 28B, as well as both aft lavatories. The cleaners that boarded came and did only trash. They came on board with no cleaning supplies and only cleaned the area with a safety card, dry paper towels. The Captain and I both insisted on having the correct biohazard cleaning done. 

However, we were both overruled and forced to accept the aircraft in this unsanitary and unsafe condition for the next group of passengers. Once again agents stated no training and no supplies provided to meet biohazard and OSHA cleaning standards. We remain non-complaint at the airline with unsanitary, unsafe and unhealthy cleaning standards where we reamain in violation daily at the airline.

The flight continued and another group of passengers also fell ill in the exact same group of seats on this aircraft. The areas were warm and there were issues with unmaintained and nonfunctining cool air packs in this area of the aircraft. Also passengers once again gave signs of Aerotoxic Syndrome poisoning with violent regurgitation with migraines.

This is a classic sign that our wet seals in the bleed air chambers have become compromised and are leaking organic phosphate toxins into the cabin from the skydrol hydraulic fluid creating tcp. I have requested many times over for a Cabin Air Quality program. A proactive change out out policy on wet seals in the chambers. Yet, it continues to fall on deaf ears here.

AFA Safety Report filed

Airline Safety Report filed

Synopsis

An A320 Captain and Flight attendant discovered biohazards in the aft of the aircraft cabin. The cleaners who came on board only took out trash and were unable to properly clean the aircraft. On the next leg, passengers in the same area became ill.

ACN: 1240192 (1752 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : D01.TRACON
State Reference : CO

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1240192

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the descent into DEN while on the WAHUU arrival with all crossing altitude restrictions verified. We had the aircraft in Speed/ Destination/ Nav mode; at 17,230 feet we crossed SAYGE intersection and the aircraft auto switched into V/S -1,100 FPM. We got the "degraded" aural warning and then switched the aircraft back into managed mode. It stayed in this mode the rest of the way down making all altitude crossings. This is the second time this has happened to me in the last week. I had the same thing happen in February while descending into BWI. This is why i'm filing it this time because it seems to be a pattern. 

Our crews should be made aware that this is happening and to use greater vigilance while in managed descent mode and to report it when it happens so that we can get Airbus to do something about it!

Synopsis

While in a managed descent on the WAHUU arrival to DEN, an A320 Captain reports crossing SAYGE at 17,230 feet when a chirp is heard and the autopilot reverts to V/S at -1100 FPM. VNAV is reengaged and performs normally for the remainder of the arrival.

ACN: 1239724 (1753 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MMGL.Airport
State Reference : FO
Altitude.MSL.Single Value : 8500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : MMGL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : GPS
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway 10
Flight Phase : Final Approach

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 500
ASRS Report Number.Accession Number : 1239724
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 6000
Experience.Flight Crew.Last 90 Days : 65
Experience.Flight Crew.Type : 460
ASRS Report Number.Accession Number : 1239746

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

After GDL approach control changed our approach 3 times we finally flew the VOR to runway 10 to intercept the ILS to runway 10. This was a very confusing clearance seeing as the controller could have just cleared us to the VOR to start the ILS procedure turn. Our original clearance was vectors to the ILS 10. Also, my iPad did not have a single VOR approach in it. Only the FO's iPad had the VOR approach. We confirmed that the radials and altitudes on the approach were the same, so we continued the approach. After crossing the 12 DME fix outbound on the procedure turn we could descend to 8400 feet until crossing the 12 DME fix inbound on the ILS. During the turn inbound to intercept the ILS at approximately 8500 feet we got a Caution Terrain on the GPWS. We were both somewhat confused about the warning as we could see the top of the hill and per procedure cleared to 8400 feet on the turn inbound. I turned off the autopilot and stopped the descent rate and climbed a few hundred feet. As soon as I stopped the descent the warning went away. We could only assume that it was the descent rate with open descent selected on the mode control panel that caused the warning. In any case, that certainly was a very confusing warning as per procedure you are cleared to 8400 feet on the turn inbound to the ILS. We continued to land without incident and discussed the warning after the parking checklist was finished. We both concluded that we flew the procedure correctly and that the only issue that could have caused the GPWS warning was our descent rate closure to the hills below us.

Narrative: 2

The weather was clear calm winds. Descending into MMGL the controller instructed us to descend 10,000 for vectors ILS 10 MMGL. While in the descent, the controller informed us we were number 5 for the approach, and reassigned the ILS DME 1 full procedure at MMGL. We asked him if we could still get vectors for ILS 10 and he denied our request. The captain briefed up the approach and the safety warning on the 10-7 page and we both referenced the spot elevation on the arc of 7,840 feet. We were cleared to the VOR and descent to 9,000. We intercepted the outbound 269 radial and set in the next altitude (8.400 feet) for the next segment of the approach. The autopilot and autothrust were both engaged and at 12 DME we started a right turn onto the arc. We had flaps 2 and 180 kts selected and the captain pulled for open descent the aircraft started to descend as we approached 8,400 feet we received a "CAUTION, TERRAIN, TERRAIN," the captain immediately disconnected the autopilot and arrested our descent and started a climb leveling at 8,700 feet and the caution ceased. We continued on the approach, intercepted the ILS and landing uneventfully.

Synopsis

As an A320 approached a known 7,840 foot obstacle on the MMGL ILS 10 approach, the EGPWS alerted CAUTION, TERRAIN nearing the 8,400 foot descent constraint.

ACN: 1239414 (1754 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch
Airspace.Class E : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 4500
Experience.Flight Crew.Last 90 Days : 50
Experience.Flight Crew.Type : 175
ASRS Report Number.Accession Number : 1239414
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 328
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1240445
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On takeoff I was the pilot flying. After taking off, I called for the gear up. The Captain was distracted by other aircraft and helicopters operating in the vicinity and did not raise the gear. This was my first flight in over 30 days and I was distracted flying the aircraft and making a rapid turn out at 400 feet to heading 020 for the departure. I called for the flaps up and the after takeoff checklist on climb out. The Captain raised the flaps and stowed the spoilers but failed to run the checklist or raise the gear. The cockpit was very loud on departure. I checked the pressurization (which was fine) and assumed that it had been so long since I'd flown that my memory of the noise was off. As we passed 10,000 feet, the barber pole speed moved to show a limit of 280 knots. This prompted the Captain to identify that the landing gear was down. I slowed the plane above 10,000 feet and the Captain raised the gear. The landing gear doors were oversped as the landing gear was raised.

Narrative: 2

[Report narrative contained no additional information].

Synopsis

A320 flight crew failed to raise the landing gear after takeoff. Gear status was recognized when barber pole speed limit of 280 knots was noted passing 10,000 feet.

ACN: 1239399 (1755 of 2908)

Time / Day

Date : 201502

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Taxi

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1239399

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19500
Experience.Flight Crew.Last 90 Days : 45
Experience.Flight Crew.Type : 700
ASRS Report Number.Accession Number : 1239406
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

While in line (#18) for takeoff with the Before Takeoff Checklist completed down to final items, we started to hear reports of wind shear on the tower frequency. There may have also been a wind shear advisory on the ATIS. The Captain decided to change the takeoff thrust setting and use TOGA. I asked if he would also like to use Flaps 2. He said yes and I requested Flaps 2 takeoff data from the ACARS. While I was doing this the Captain began reprogramming the Perf page by entering 2 in the Flap setting. When we received the Takeoff data, we had a long discussion about what speeds he wanted for takeoff. He decided to use the Max thrust V-speeds for V1 and Vr and a speed of 142 for rotate. This speed was well below the 154 Vr maximum listed on the data, but he chose it because it matched the 10 knot tailwind Vr for a reduced takeoff (I guess this is his technique). I programmed the speeds into the Perf page and shortly thereafter we departed 4L, rotating at 142 knots. At 800 feet AFE, upon power reduction, the Captain noticed and stated that the flaps were still at position 1 and that we had never set Flaps 2. We discussed what had occurred and compared the speeds we had used with the Flaps 1 data. It appeared that we rotated 4 knots below the rotate speed for Flaps 1 reduced, but I didn't think to compare it to the Vr speed on the TOGA thrust line for the Flaps 1 data.

The error was that we did not comply with the flight manual and repeat the Before Takeoff Checklist after repositioning a flight control. Other contributing factors were my flow for setting the new Flap position on the Perf page followed by moving the flap lever to the proper position was interrupted by the Captain selecting position 2 on the Perf page; the extensive discussion on V-speeds; and the first time flying together with this out of domicile Captain.

Narrative: 2

This was first flight as a crew

Wondering why the backup [for takeoff], we were told there were reported 15 knot wind shears on approach and takeoff. I stated we should swap to TOGA power and the First Officer (FO) asked should we consider flaps 2. I agreed. While he got the performance data I set 2 in the performance page of the FMC. We briefed the flaps two data and also that we would delay rotation slightly using V1 VR and "rotate" at a higher speed using the flaps 2 data. After the briefing we should have run the takeoff checklist again, but we both forgot. This led us to not resetting the flap lever to 2 even though all plans now were for flaps 2 and TOGA.

Synopsis

An A320 flight crew, number 18 for takeoff due to reported windshear, reports on takeoff and landing, wisely opted to change their takeoff configuration to flaps position two and TOGA power as a precaution. They programmed new performance numbers based on the new configuration but failed to repeat the before takeoff checklist after doing so. During climbout they realized they had not reset the flaps and they had remained in their original flaps one position.

ACN: 1238811 (1756 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Rain
Weather Elements / Visibility : Icing
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : JFK
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1238811

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

The airplane was on an overnight in the New York area in which a winter storm was moving into the area. The forecast called for freezing rain and falling temperatures in the early morning. The forecast was accurate. 

When the captain and I were walking to the gate, I noticed from the walkway, between terminals, the airplane had type IV de-ice fluid on the wings. I mentioned this to the captain but I don't think he heard me. When we reach the gate, the captain said he would do the outside preflight and I thanked him. I checked the top of the wings, from the emergency row exit, and was unable to see the right wing due to the ice that covered the windows on the right side of the airplane. I then checked the left wing and noticed type IV de-ice was sprayed on the wing. The captain came in, from the outside preflight, and I told him what I was able to see and not see. He told me to contact operations about the de-icing that was done and wanted to know what time it was done, we needed deiced, and will it be done at the gate or somewhere else. I contacted operations and they said de-icing will be done behind the gate in the alley and to contact the deicer about when the fluid was sprayed onto the airplane. I contacted the deicer and he mentioned it was done early in the morning to help with removal of contaminants.

We boarded the airplane, closed the doors, and called for pushback. Ramp mentioned that it will be some time because a B767 was behind us and getting deiced. Ramp finally cleared us to pushback and mentioned that we will de-ice in the alley. As we came to the end of our pushback, we noticed the de-ice trucks approaching us. Then ramp asked if the ground crew can tow us to the top of the alley near TA taxiway. The conditions were not conducive for a tow. We asked ramp what they want us to do because there was a great amount of confusion going on the frequency. One minute we would get de-iced in the alley, then to TA taxiway, then in the alley. Also, there was additional confusion due to the monitoring of two frequencies, ramp and the deicer. Ramp finally said to go to TA taxiway and we notified the deicer to meet us at TA. We disconnected and started both engines to taxi to the de-ice spot.

We configured the airplane and notified deicer we were ready for de-ice and anti-ice. There was a great deal of ice on the airplane especially on the right side. The process took about 17 minutes. I was unable to get the employee number who said we were free and clear. I was able to copy 336***. As we started the engines, a flight attendant contacted the flight deck and the captain answered. I did not hear the conversation but I did hear the captain mention, at least two times, are you sure it is ice and not foam from the de-ice/anti-ice fluid. The captain mentioned to me, the flight attendant said she could not see out of ANY of the windows on the right side of the airplane. 

The captain called the deicer and told him that there is ice on the right side of the airplane and it needs to be checked. The deicer came back over to the airplane and said they are going to de-ice the airplane again, especially the right side. We shut down both engines and waited for the de-ice. The flight attendant was keeping the captain up to date about the progress that was being made on the ice removal on the right side of the airplane. 

The captain made the comment to the deicer about bringing in the varsity team because of how bad the job was done the first time. The deicer made the comment he was putting his better team on the job. After about 18 minutes, the job was complete and the deicer said we were free and clear. The captain was in contact with the flight attendant and she said she was able to see out of all windows on the right side of the airplane.

We started the engines and proceeded without further incident until climbout when I have never experienced this in my airline career. Starting around 4000 feet (MSL) the captain and I looked at each other in bewilderment. It sounded like we were flying through heavy rain but we were between cloud layers. I looked at the top of the nose of the airplane and noticed chunks of ice, measuring about 3 inches by 4 inches, flying past. They were coming up from the bottom of the nose cone and following the slipstream. The shedding of ice stopped around 8000 feet (MSL). 

If it was dark outside, I am not so sure that the flight attendant would have been able to see the ice.

What an excellent lesson in CRM.

Synopsis

A320 First Officer experiences a poor deice job after the aircraft had spent the night in freezing rain. A Flight Attendant was responsible for informing the Captain that ice was still adhering to the aircraft after the first deicing. The deice crew is called back for a second deicing before departure. Climbing through 4,000 feet the aircraft begin shedding ice from the nose radome and continued through 8,000 feet.

ACN: 1238520 (1757 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : D01.TRACON
State Reference : CO
Altitude.MSL.Single Value : 9200

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : DEN

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 14000
ASRS Report Number.Accession Number : 1238520
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1238533

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We were on a visual approach just about to intercept the localizer. I was hand flying the airplane with auto-throttles OFF. Just as we were approaching CHOLA, I observed a pop up TCAS target at 1130 position 500 feet below us in the climb. I notified the First Officer of the target and he reported it to ATC. Simultaneously we got an aggressive RA to Climb, Climb NOW and ATC gave us a traffic alert climb immediately! I went to TOGA (Take Off/Go-Around) thrust from flight idle and began to follow the RA guidance on the VVI. To my and the first officer's surprise, the thrust took 4 to 5 seconds to go to TOGA!!! Recognizing the delay in thrust response, I started a right turn away from the TCAS target to evade the target laterally. The FO reported to ATC we were in a TCAS climb and called for me to keep climbing and confirmed I had TOGA selected. As thrust finally went to TOGA and with the turn away from the target, we began to aggressively climb and the RA conflict resolved. We simply could not believe how long it took for the thrust to go to TOGA! We wrote up the airplane thrust system upon arrival at the gate in DEN. The aircraft was taken out of service to test the thrust system. I called approach and they concurred what data that had at the time indicated we had 200 feet and 1 mile separation.

Narrative: 2

We were on a vector to intercept the ILS, and had just been cleared for the visual approach to that runway. The captain was hand flying the aircraft, with the autothrust off, the flaps were up, and we were leaving 10,000 feet and descending to 7,000 feet. As we approached CHOLA intersection we saw a TCAS target appear on the ND, at about the 1130 position and about 300 feet below and climbing. The captain remarked about the traffic and I called ATC and advised them of the traffic. We then received a climb command from TCAS. The captain began the TCAS climb, pushing the throttles to TOGA. ATC then directed us to make an immediate climb. I responded that we were executing a TCAS RA climb. With the throttles at TOGA from idle thrust, the power took about 4 to 5 seconds to respond. Because of the lag, the captain also began a turn away from the target. We never acquired the target visually, and it passed below us and continued away from us. We talked with the controller and proceeded to re-intercept the visual approach and landed uneventfully.

Synopsis

A320 flight crew reports receiving a TCAS RA while descending to 7,000 feet with the engines at flight idle. After selecting Takeoff/Go-Around (TOGA) mode to evade, a 4-5 second delay is experienced before TOGA thrust is achieved.

ACN: 1238261 (1758 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Hydraulic Syst Pressure/Temp Indication
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : System Monitor: Indicating and Warning
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 1238261
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Primary Problem : Manuals

Narrative: 1

During preflight, the first officer noticed on the HYD ECAM page that the line extending up from the blue pump was green instead of amber as it should be with no Blue System pressure. The System was otherwise indicating no pressure. We entered the discrepancy in the log book and called maintenance. The maintenance technicians were instructed by Maintenance Control to apply MEL 29-30-03F, ECAM HYD Page Indications, System Pressure. I was uncomfortable with the validity of this MEL as our Flight manual indicates that system pressure is depicted by the three numerical values at the top of the ECAM page only. I brought this up to the maintenance technicians who were applying the MEL and they said they were doing what Maintenance Control had instructed them to do and assured me that that green line was, indeed, a pressure indication. I objected and asked them to provide some form of documentation to show that this fell under the MEL they were applying. Maintenance Control was not willing to provide any documentation, but the technicians provided some documentation that this was a pressure indication.... Still not comfortable, I tried to call scheduling to get a number to talk to a chief pilot but sat on hold for quite some time. I also tried to look up some reference which would give me an idea of whether this MEL was appropriate. As we no longer have any kind of a Systems book other than the A320 training manual, which is too vague for this purpose, I was unsuccessful in that endeavor. At this point, I had nothing other than a vague recollection from my original systems training in 1996 that this was different from a system pressure, and the people whose job and responsibility it is to determine what is an appropriate MEL all assured me that it was correct. At this point we looked again at the ECAM HYD page and noticed the line had turned amber (normal). At that point, since the problem was clearly documented the fault no longer existed and we couldn't find anything excluding this fault from the MEL, we decided to depart.

After we departed, I was still uncomfortable, so I looked again at the MEL for more definitive information. The one thing that I noticed is that for this MEL, it states that there are three installed and one may be inoperative. Because there are three numerical pressure indications at the top of the ECAM page, I feel this casts some doubt on the appropriateness of this MEL for this particular failure.

The main reason the event occurred is that the MEL and Flight manuals are not specific enough, in my humble opinion to determine if this is an appropriate MEL for the system failure being addressed. Also, I have been told things by local maintenance in the past that turned out to be not correct. I can't name a case where Maintenance Control was incorrect, but after a few experiences, I don't trust maintenance to tell me what I can and cannot fly with without clear documentation. In order to be more sure of our actions, and not delay flights with maintenance issues, we need someone we can easily and quickly contact during all operating hours, whom we trust, to discuss such issues. This is a complicated airplane and history has shown me that maintenance is not that someone. When writing an MEL, it must be impossible to think of all the scenarios which may occur like this. That is, cases where it is unclear if an MEL is appropriate. It would be helpful in this case if a note were added to the MEL including or excluding the green line in question. It would also be helpful if the Flight manual labeled this green line. I was unable to find a reference anywhere describing exactly what it is....

Please understand, that like most pilots at this airline, I want to operate a safe aircraft and I want to operate on time. I find ambiguity in the operation to be my large impediment to attaining this goal.

Synopsis

A Captain reports their A320 MEL and Flight Manuals are not specific enough to determine if an MEL 29-30-03F of an ECAM Hydraulics Page Indications, System Pressure is appropriate for the system failure being addressed. First Officer (FO) had noticed on the ECAM HYD Page that a line extending up from the blue pump was green instead of amber as it should be with no Blue System pressure.

ACN: 1238259 (1759 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MMMX.Airport
State Reference : FO
Altitude.MSL.Single Value : 9200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : MMMX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway 05R
Nav In Use.VOR / VORTAC : SMO
Flight Phase : Initial Approach
Route In Use.STAR : DATUL2A

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1238259
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Workload
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On the DATUL2A Arrival landing 05R we programmed the FMS with STAR DATU2A, ILS 05R. The database did not allow or show any transitions or vias. I briefed the ILS DME 5R, plate 11-2. This approach tracks outbound from SMO then joins the localizer. The FMS database did not show this track outbound and showed a discontinuity between SMO [and] CI05R. We deleted the discontinuity to connect SMO to CI05R (Incorrectly). We verified the track in the FMS agreed with the Jeppesen Chart. When we passed SLM, Approach Control gave us direct to MAVEK and cleared for the RNAV ILS 05R. I had never heard of an RNAV ILS so I thought it just language/accent issue and continued with the ILS plan we had made. I confirmed with the Captain (CA) that we were cleared for the approach. At this point MAVEK is not programmed in the FMS because we were expecting to go over SMO. The CA said he saw MAVEK on the other Jeppesen Chart for the GPS approach and that SMO [and] MAVEK are the same spot. He programmed direct to MAVEK, which put in a discontinuity, so I then connected it to CI05R. It was still looking good to both of us at this point. We started to slow to 160 KIAS and get configured per the 10-7 pages but ATC requested 200 KIAS until MAVEK which we accepted. Just before MAVEK I reselected 160 KIAS. For an unknown reason, reaching MAVEK, the aircraft reverted to heading mode flying straight ahead on our current heading and did not turn left as we expected. The CA quickly said that it was in heading mode and we need to turn left. At this time ATC also called us and asked if we were going to turn left. We replied yes. I intervened by pressing the heading button to arm the NAV which it captured right away with a hard left turn to re-intercept the course. Before MAVEK we were at 12,000 FT and had set 9,200 FT in the ALT Select for the next step descent at CI05R (Incorrectly). ATC must have seen our sharp turn to re-intercept our course and gave us a vector for the 05R localizer. I think it was around a 160 degree heading to join. We took the heading and continued our descent to 9,200 FT. Reaching 9,200 FT ATC called us and said we were supposed to be at 9,700 FT. We replied and did an open climb back up to 9,700 FT. We received one more vector, about 10 degrees right of the previous heading to join. We joined the localizer and intercepted the glideslope. We were handed off to Tower, cleared to land, and landed without further incident.

We knew that MMMX was considered an enhanced threat airport and I prepared by completing a MMMX PowerPoint presentation in Learning on Line. We had also thoroughly reviewed all the Company Charts in the Jeppesen app. The first contributing factor was the fact that although MMMX is shown to have D-ATIS we were unable to retrieve ATIS through our ACARS. All we could receive was TERM WX which was helpful but did not print approaches in use and landing runways. The CA had to listen to the ATIS over the normal frequency and write it down. It was broadcast in Spanish and English. The parts I heard were hard to understand but ILS 05R was clear enough.

The next factor was the FMS database and Jeppesen Charts. The only FMS choices are for the ILS05R, VOR05R, or the basic 05R. There is no RNAV/GPS 05R or ILS DME 05R. Additionally, once we selected ILS 05R with the DATU2A, there were no options to select a Transition or Via. With this combination of DATU2A [and] ILS05R it displays a discontinuity between SMO [and] CI05R and omits outer waypoints. Now I know that to display the entire ILS 05R you have to select NO STAR but this is cumbersome because you are flying the STAR up until you get cleared for the approach. 

As to the Jeppesen Charts, plate 11-1 RNAV (GPS) OR ILS Rwy 05R [and] plate 11-2 ILS DME 05R, are too similar and easily confused. Both plates have the word ILS in them. Also the first plate 11-1, the one we actually should've used, starts with RNAV/GPS so I think we instantly overlooked it and didn't further read the "OR ILS 05R" part of it. I would suggest that this plate should split into each of its respective functions instead of a combo plate. I would also suggest that the FMS database be able to accept DATU2A, ILS 05R via MAVEK. 

Another problem with the DATU2A is that it has you fly to SLM then track SLM R-219 to SMO and expect approach authorization from there (it's even labeled IAF on the STAR Plate). SMO happens to be the IAF for plate 11-2 ILS DME 05R. I feel like it leads you to that approach. 

1. Split Jeppesen Chart 11-1 into two separate procedures. 
2. Update FMS Database to allow approach vias. Update FMS to include ILS DME 05R. 
3. Include info in the MMMX training PowerPoint about putting in NO STAR to activate all fixes. Include a 
warning not to confuse the ILS 05R with the ILS DME 05R.

Synopsis

A320 First Officer experiences confusion after Approach Control gave him direct to MAVEK and cleared for the RNAV ILS 05R at MMMX. The plain ILS 05R is selected from the FMGC database but no transition from SMO is available and is created manually. Heading and altitude deviations occur and ATC eventually takes over with vectors and altitude assignments.

ACN: 1237717 (1760 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 2100

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 19L
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1237717
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1237722
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Being vectored for the ILS 19L. Just coming up on BERKS, instructed by Norcal to descend and maintain 2100. Selected 2100 in MCP and, per SOP, altitude was acknowledged by First Officer (FO). I read back altitude clearance to Norcal and we began descent to 2100 feet. Concurrent with this, as we turned onto the localizer, we receive a FUEL L WING TANK LO LVL ECAM. I complete ECAM while FO continues approach. We both acknowledge that 2100 is below the published 2800 for the segment between BERKS and SHAKE and resolve we are clear of terrain and communication from Norcal was clear and unambiguous, as was our readback and that 2100 must be the minimum vectoring altitude for that segment of the approach. Reaching 2100, Norcal asks us to confirm our altitude. I respond "2100". Norcal says "cancel approach clearance and climb to 2900". I acknowledge clearance and we climb to 2900. Norcal then reclears us for the ILS19L shortly prior to SHAKE. Landed uneventfully.

Narrative: 2

We were being vectored on to ILS 19L from right downwind at 5000 FT. From base we were given a heading to intercept LOC inside BERKS intersection and told to descend to 2100 FT. After reading back the intercept clearance and the clearance to descend to 2100 the LOC was intercepted as we started down. Soon after, we got an ECAM indicating a low fuel level in the left wing tank with the appropriate to-do list. It was determined that the ECAM was simple enough to complete and continue. Upon completion of the procedure and leveling at 2100 FT; ATC asked us to "say altitude." We were then instructed to climb to 2800 FT and continue inbound. Clearance for the ILS 19L was received and the rest of the approach and landing was normal.

Synopsis

A320 flight crew reports being cleared by NCT to descend and maintain 2,100 feet approaching BERKS during an ILS 19L at SFO. This is below the approach segment altitude of 2,800 feet but was clear and unambiguous and the readback was not challenged. This altitude is not correct and NCT issues a climb to 2,900 feet.

ACN: 1237573 (1761 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : RNO.Airport
State Reference : NV
Altitude.AGL.Single Value : 3000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Tower : RNO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Landing
Airspace.Class C : RNO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1237573
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1237705

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

On approach to Reno [Runway] 16R, encountered moderate turbulence, LLWS, shifting high winds with severe gusts and blowing dust. Airspeed fluctuations on final with a shift to a tailwind necessitated a missed approach go around. While accomplishing the missed approach encountered strong moderate turbulence at flaps 3 and the gear up, atmospheric conditions created an airspeed gain in excess of 30 to 45 knots thus overspeeding the flaps for configuration. The pitch was immediately adjusted but the overspeed occurred with a massive airspeed gain. In reconfiguring to flaps one, a similar airspeed gain caused a second overspeed in the flaps one configuration. The pitch attitude was adjusted and no further over speeds occurred. Upon taxi to gate, overspeed condition was written up and reported to maintenance for inspection.

Narrative: 2

The winds were changing constantly gusting to 35/50 knots. The First Officer configured early to get the plane stabilized. At approx. 1,000 feet we were losing airspeed and altitude and showing a 15 knot tailwind while tower was still calling a 30 knot headwind. I could also see a dust line coming down the runway towards the landing zone. I directed a go around and while climbing we got the 30-40 knot headwind gusts. While getting the flaps from 3 to 1, the airspeed hit the bottom of the red line twice. The moderate turbulence and gusts made it difficult to read the speed tape and fly the plane. After landing on the second attempt, I called Maintenance Control and wrote up the flap overspeed.

Synopsis

A320 flight crew experiences turbulence and strong gusty winds during approach to Runway 16R, resulting in a decision to go-around. During the go-around, flap overspeed occurs, and again while configuring for the subsequent approach.

ACN: 1237337 (1762 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZTL.ARTCC
State Reference : GA
Altitude.MSL.Single Value : 23500

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : IVAN FIVE

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1237337
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1237600

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Weather
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Human Factors

Narrative: 1

Was in contact with Washington Center at FL260 just passed LYH VOR in light to moderate chop, asked for lower altitude and descended to FL240, had a hard altitude crossing restriction of FL240 at MAYOS in the MCDU on the IVANE5 RNAV STAR into Charlotte. Next was a frequency change to Atlanta Center and was cleared to descend VIA the IVANE5. We set altitude alerter to 6000 feet for the bottom restriction on the IVAN5 STAR. The FMS decided to drop the flight restriction of FL240 at MAYOS fix all by itself. The aircraft began to descend to FL220 looking at the next restriction of KIYEN. Next call was from Atlanta Center asking what altitude are you descending to? We were just passing 23,500 feet. It was too late to intervene and make the FL240 restriction at MAYOS. We were instructed to descend to FL220 by Atlanta Center and missed the restriction of FL240. We asked ATC if there was a conflict with traffic and none was noted at this time. There is a corridor that has passing traffic below FL240 at MAYOS. 

We were relying on automation of the FMC and high workload due to turbulence on the descent phase of flight.

Double check FMS and intervene at all fixes with a hard altitude at the beginning of the STAR. Never trust the FMS 100%

Narrative: 2

We did not notice that the Mayos restriction of FL 240 had been deleted from the computer due to the early descent.

Synopsis

After descending to FL240 due to turbulence the flight crew of an A320 was cleared to descend via the IVANE RNAV STAR to CLT. Although they had set FL240 as a firm crossing on their preplanned programming of the arrival they failed to realize that so doing satisfied the crossing restriction at MAYOS (Between FL260 & FL240) and, thus, the restriction was removed from consideration by the FMGC. When they selected a VNAV/Managed Descent the FMGC factored the next crossing at KIYEN (At or above FL220) and began a descent...thus triggering the deviation. The Primary reporter cited reliance on automation as a contributing factor.

ACN: 1237034 (1763 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1237034
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We took off with one generator on MEL. (I believe it was 24-20-01) Number 1 generator inoperative. 
About 60 miles east of destination we got a low oil pressure Indicated Drive Director (IDG) 2 Electronic Centralized Aircraft Monitor (ECAM) which calls for IDG disconnect and subsequent 1 generator operation. I handed aircraft and radios to first officer after he checked to see if it was an immediate action item or ECAM exception. I did ECAM and disconnected IDG. We were now down to single generator Auxiliary Power Unit (APU) and I had the First Officer (FO) declare an emergency. The QRH had a follow-up but didn't add anything new.
We thought that although the weather wasn't great at destination, there was really no better alternative and we were already set up to go in there. I contacted the Dispatcher and he agreed that was best.
We requested runway 28R which was longer and quicker and it was approved. I also requested the equipment just in case. We were given priority handling and landed uneventfully. The flight attendants were told to prepare for a normal landing. The equipment followed us to the gate uneventfully.

We picked this aircraft up just as maintenance was MELing it for the inoperative IDG. The write-up in the logbook was IDG low oil pressure and I thought it very strange that here was the identical failure on the next flight for the other IDG. I just wonder if maybe the IDG's had not be serviced properly. Other than that I am not sure how to prevent this except maybe make generators unable to be on MEL.

Synopsis

A320 Captain reports being dispatched with an inoperative generator (IDG low oil) with the APU generator operating. During descent the remaining engine driven generator Indicated Drive Director (IDG) indicates low oil and is disconnected. A normal landing ensues at destination.

ACN: 1236956 (1764 of 2908)

Time / Day

Date : 201502
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 22000

Aircraft

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : IVANE5
Airspace.Class A : ZDC

Person : 1

Reference : 1
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Experience.Air Traffic Control.Time Certified In Pos 1 (yrs) : 23
ASRS Report Number.Accession Number : 1236956

Person : 2

Reference : 2
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
ASRS Report Number.Accession Number : 1236965

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

an A320 was cleared to descend via the IVANE 5 into CLT. This gives the aircraft a window crossing the MAYOS intersection between FL240 and FL260. The aircraft crossed MAYOS at FL220 and descending. ZTL had to stop the aircraft from descending further. I was the field level manager (FLM) of the area at the time of the event and the CPC working the position brought this to my attention. This is a known issue that has been reported numerous times before. The descend via the IVANE 5 needs to be terminated to prevent this from happening in the future.

Narrative: 2

The aircraft was level at FL240 and I issued a descend via clearance. The aircraft is supposed to hit a window at mayos of FL24-FL260 then continue down via the arrival. I switched the airplane and then the next controller called me and said the aircraft checked in descending to six thousand, missing all of the windows. he stopped the aircraft at FL220 and then questioned the pilot. Train the pilots who fly the airbus to manually hit the windows and override the computer.

Synopsis

Two ZDC Controllers report another instance of an Airbus flight crew missing the crossing restriction at MAYOS on the IVANE5 arrival to CLT.

ACN: 1235981 (1765 of 2908)

Time / Day

Date : 201501

Place

Locale Reference.ATC Facility : D10.TRACON
State Reference : TX
Altitude.MSL.Single Value : 23000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : D10
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : SEEVR
Airspace.Class A : D10

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 45
Experience.Flight Crew.Type : 90
ASRS Report Number.Accession Number : 1235981
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1235984

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

We were filed into DFW via the BRDJE1 RNAV arrival. Approaching SEEVR with the BRDJE1 arrival in the FMGC, Fort Worth Approach told us were cleared to SEEVR; descend via the SEEVR arrival. We checked the STAR, and were somewhat confused over the clearance, because the title page of 60-2B looks like this:BRDJE ONE RNAV ARRIVAL (SEEVR.BRDJE1) Note how it says (SEEVR) in parentheses. We thought we had understood the clearance, and were descending to cross SEEVR at FL200-230 as published. 

What the controller meant was for us to go to SEEVR, and join the SEEVR ONE RNAV ARRIVAL [Jeppesen title page 60-2Q]. The controller told us we were looking high at SEEVR. That got our attention, and we finally figured out that we were heading to the same fix [SEEVR], but cleared for a different arrival, with a different crossing restriction [of between] 13000-16000. Seeing that we would be high at SEEVR, the controller gave us a 270 heading and a clearance to descend to 6000, then cleared us direct to INWOD, which is a fix on the approach to runway 17L.

The annotation on page 60-2B certainly confused us. Getting a clearance for a different [STAR] close into a duplicate arrival fix certainly confused and time-compressed us as well. We should have been more proactive in resolving the ambiguity.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

Although they were originally cleared "as filed" and, thus, via the BRDJE RNAV STAR to DFW the flight crew of an A320 was cleared "...direct to SEEVR, descend via" the SEEVR arrival. Aware that SEEVR was a fix on their filed route AND on the BRDJE STAR they mistakenly assumed the FMGC was properly programmed and planned to cross SEEVR between FL230 and FL200. As they approached SEEVR ATC advised they looked to be too high and, only then, did they realize that--although sub-titled (SEEVR.BRDJE1)--their latest clearance was for the SEEVR RNAV STAR--sub-titled (SEEVR.SEEVR1)--and they were to cross SEEVR between 16000 FT and 13000 FT.

ACN: 1235592 (1766 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Brake System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1235592
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Staffing
Primary Problem : Manuals

Narrative: 1

January 2015, I received a call from ZZZ Line Maintenance Technician X. An A320 Aircraft had a popped Circuit Breaker (C/B) L32, which was for Left gear brake fan. Technician X stated when C/B was pushed in, an [electrical] arc was seen on [Left] gear. ZZZ inspected the left gear conduit. I asked if the conduit was secure. He said it was secure, but with a burn mark. I said we need to contact engineering. We had a 3-way conference with company Engineer X. I had concerns from last year, many Airbus's had problems with landing gear conduits. I ask if we could dispatch aircraft in this condition. Engineer X checked his records and did not find any documents that we had to stop the aircraft. A Brake Steering Control Unit (BSCU) Bite check showed no faults. A review of MEL 32-48-01a, has no Maintenance (M) or Operations (O) procedures for this condition. C/B's L32-L35 were pulled and collared per Typical Repair (TR) 32-00-14, Brake Fan MEL 32-48-01a.

On Takeoff out of ZZZ, A320 aircraft had an Auto Brake Fault. [Airbus] Web Airman showed Tachometer-1 fault. We had the aircraft divert to ZZZ1 for repairs, due to the fact it was going to a Non-maintenance station. Mechanics have pressure to move aircraft on Time. The aircraft was fully loaded and ready to go. The popped breaker was pulled and collared, but no thought was giving to other system wires damaged. Engineering was contacted. ZZZ did do a Bite check for other faults, but none found. We believed at the time the only damage was to the Brake Fan system. ZZZ1 maintenance found that the wire harness was damaged, which made Tachometer-1 fail. ZZZ1 [Maintenance] replaced the bad harness. The Airbus desk is very understaffed. Operations put great pressure on maintenance to move the aircraft. I have been told many times by dispatch, we do not change the Aircraft for maintenance convenience. The Typical Repair or the MEL should give more guidance [for] Landing Gear conduits. Maintenance Operations Center (MOC). Technician. Maintenance Control.

Synopsis

A Maintenance Controller reports how their understaffed Airbus desk and pressure by Operations to move aircraft 'On Time', contributed to deferring and collaring a popped Brake Fan Circuit Breaker (C/B) for a Left Main Gear harness that had previously arced, left burn marks and a Tachometer-1 fault. Airbus A320 diverted after takeoff due to an Autobrake Fault.

ACN: 1235010 (1767 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1235010
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Had problem at gate when turning on Brake Fans tripped CB. We were parked at the gate with the side of the aircraft fairly close to the terminal building. While maintenance was troubleshooting, they pushed in the CB and pushed the Brake Fan button. Simultaneously there was a large flash that looked like a flash of lightning on the terminal as it was illuminated by the flash. Maintenance evaluated it for about 1/2 hour. We were quite reluctant to take the aircraft, but after a lengthy discussion the mechanics, we were cautiously optimistic that they were correct and that by pulling both Brake Fan CBs that no power was going to that area. I was concerned that other systems could have been affected by the arcing of the short. They ran some byte tests and they said things looked normal. 

After gear retraction we got an Auto Brakes ECAM and a BRAKE RELEASED ECAM message. We did the ECAM and wrote to dispatch to advise them and evaluate what the message meant exactly. He spoke to maintenance and wrote back to us and said the Number 1 brake was now inop and that 2, 3 & 4 were normal. Dispatch asked us to divert as our destination wouldn't have the necessary parts. The FO and I decided that we should declare an emergency as we were uncertain as to whether or not there might be some unforeseen braking problems as there was obviously more damage than anyone realized. We advised ATC, Flight Attendants and passengers of our diversion and that although we anticipated a normal landing that they would see Emergency Vehicles after we landed. We landed uneventfully and the Vehicles followed us to the gate. Once again we declared the emergency because we felt that the braking system was partially compromised and for the slight possibility of additional problems.

Maintenance should have dealt with the problem initially or done an aircraft swap. I should have stuck with my gut instinct that there was more to the problem than what they thought.

Synopsis

A319 Captain describes a short circuit in the number one brake fan that causes the circuit breaker to trip and a large arc when Maintenance attempts to troubleshoot. The fan is deferred but when the gear is retracted after takeoff an AutoBrakes and a BRAKE RELEASED ECAM message is displayed. The crew is directed to divert to a suitable airport for repairs.

ACN: 1234755 (1768 of 2908)

Time / Day

Date : 201501

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Flight Phase : Taxi
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Autoflight System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autoland
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Minimum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1234755
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

An A320 aircraft arrived to Airport ZZZ with a log item for FMGC-1 INOP. An attempt to reset the computer per Trouble Shooting Manual (TSM) 24-00-00-810 was not successful. At this point MEL 22-83-Y was complied with, for the FMGC-1 INOP. The CB was pulled and Autoland Status downgrade placard installed. Per MEL instruction MEL 34-48-E was also applied for the Terrain Avoidance and Warning System (TAWS) INOP. The aircraft pushed off the blocks for its next flight leg, but returned to the blocks when Dispatch advised that they could not dispatch the aircraft due to weather minimums. At this time, another CB cycle was attempted and was successful. The applicable Aircraft Maintenance Manual (AMM) test was completed Auto Flight System (AFS) Test 22-96-00 and all systems passed. The MEL 22-83-Y and 34-48-E procedures were reversed and the aircraft returned to normal configuration. The Autoland Status downgrade placard was also removed. A new Maintenance Release was completed and sent to the flight deck. 

The question has been raised as to the validity of the Autoland Status and if the Land Verify Test needed to be completed. At the time of the incident I did not feel the Land Verify Test was applicable for the following reasons:
1) General Maintenance Manual (GMM) 05-35-40 states if a CAT II/III component of system is reported to be malfunctioning, it will require that the system test specified in the applicable AMM-FIM Fault Isolation Manual be accomplished. If a CAT II/III component, listed in the CAT component tables is replaced, reseated, swapped or associated wiring, plug or receptacle is disconnected, the Ground Recertification procedure must also be accomplished by a CAT II/III Qualified Technician before retaining or upgrading the CAT Status. NOTE: Cycling of circuit breakers does not require Ground recertification. 

2) MEL General - Definitions: Takeoff: ...For purposes of MEL relief, this translates to the point at which the pilot physically begins to apply power to initiate the takeoff...All actions that were taken were within the applicable AMM, TSM and MEL requirements before the aircraft reached the point of dispatch. No system components were disturbed. The flight was never at the point of dispatch - movement of the throttles for takeoff. I do understand that if the aircraft indeed proceeded through the point of dispatch or actually flew, then a CAT recertification would have been necessary. As an additional note, there are systems that affect the CAT status of the aircraft, requiring CAT Downgrade placards being installed, that when repaired do not require a CAT recertification, only that the placard be removed. Ref MEL 24-03-0 for Engine Driven Generator Channel.

If my interpretations of this are incorrect, then I believe a more clearly defined policy should be implemented such as: Before removing any Autoland Downgrade Placard, a Ground recertification must be accomplished.

Synopsis

A Maintenance Controller explains why he believes an A320 'Land Verify Test' was not required for an FMGC-1 that had been MEL'd under Chapter 22-83-00, but later reversed prior to dispatch, when another Circuit Breaker (C/B) cycle attempt was successful.

ACN: 1234709 (1769 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1234709
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Gate Agent / CSR
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Our aircraft came from the hangar not serviced, cleaned or catered. As the cleaning staff deplaned they mentioned the strong smell of fumes onboard. As I approached door 1L the smell of toxic aircraft fumes was prominent. All the crew noticed the smell, the cockpit and the flight attendant's (FA). The captain walked to the back of the aircraft, said he would turn on the air to clear out the smell of fumes. One FA went to the back to store her bags and do her safety checks. She came back to 1L for air as the odor was overwhelming and uncomfortable. I informed the FS, Captain and Agent that I was uncomfortable inhaling the fumes, it is a health hazard, and might not be able to work the flight, depending on how severe the odor remained and its effect on my body. I was previously exposed to aircraft fumes and consequently developed severe health issues. The agent phoned zone to inform them of our situation. She was told to go ahead and board, (regardless of cabin air quality and potential health risk to passengers and crew). I took the greeter position, when I was able to step in and out of the cockpit for air, the cockpit did not smell of fumes and the air supply was on high flow. During the boarding, inflight supervisor came onboard. He also acknowledged the strong odor of jet fumes. The odor had not completely dissipated within the 35 minutes of boarding. Due to the very light load, exits were briefed and cabin was ready well before departure time.

I strongly feel that my Airline's safety standards were not adhered to. Crew and passenger health was put at risk and jeopardized. I felt compelled to board, impair my health rather than deservicing the passengers and the Company's schedule.

Synopsis

An A320 Flight Attendant reported even though the aircraft smelled strongly with a fuel odor, it was boarded and departed. Adherence to Company safety standards were questioned.

ACN: 1234623 (1770 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.MSL.Single Value : 1600

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : A90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 33L
Flight Phase : Descent
Route In Use.STAR : QUABN3
Airspace.Class B : BOS

Component

Aircraft Component : Altimeter
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3800
ASRS Report Number.Accession Number : 1234623
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1234622

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Descending to BOS, First Officer (FO) pilot flying, cleared to descend via QUABN 3 RNAV. WX indicated low CAT 2 mins, so a CAT 3 autoland was briefed by the Captain and transfer of control from the FO was decided to occur once the next ATIS was updated. Through transition altitude, Approach descent checklist was completed and altimeters were set to 29.96 (in error but read back by both pilots. The altimeter seemed correct at the time).

Final ATIS indicated VFR at the field and the FO briefed the normal CAT 1 approach as a backup to the visual and never turned control over to the Capt. Level offs were made via the descent at 11000, 7000 and 3000 without any reports of deviation from ATC. A further descent to 2000 was issued and after descending (out of 3000), we were contacted by ATC with a low altitude alert and the correct altimeter setting of 29.28. An immediate climb was made to return to 2000 once the proper altimeter indicated we had actually descended to approximately 1600 feet. Clear night visual was maintained with good visibility of shore lights and no GPWS warnings occurred.

A long duty day preceded the event and after debrief of the incident, we were surprised that there was no previous indication of a deviation at our level off altitudes even though the incorrect altimeter was set at those times. We also commented during the initial descent out of 3000 that the altitude seems a little low but after checking that the glide slope intercept was 1500 feet, it seemed that the 2000 was not out of order.

Further approach to landing was normal.

Narrative: 2

The GPWS and Enhanced GPWS did not signal a terrain conflict. After reaching the assigned altitude of 2000 ft., we resumed the approach. The weather conditions at the time were VFR night. The surface was a well lit suburban/city environment making the terrain visible and distinguishable.

During our Threat/Error debrief, both the First Officer and Captain noted how easily the chain of events lead to the low altitude alert issued by the controller. We were also surprised at how easily we bypassed several checks and observation which could have quickly lead to our discovering the incorrect altimeter setting. Also a contributing factor was the length of our duty day. At the time of this event, we had been on duty for approximately 10 hours and some 11 hours 30 minutes awake for the Captain.

Synopsis

An A320 flight crew descending for a planned CATIII autoland at BOS incorrectly set their altimeters to 29.96 when the actual pressure was 29.28. A low altitude alert was received from ATC as they descended to 2000 MSL (or 1320 MSL with the correct pressure setting) at which point their altimeters were corrected, they climbed back to 2000 and completed a visual approach consistent with the now improved weather.

ACN: 1233858 (1771 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZFW.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 17000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : BRDJE
Airspace.Class E : ZFW

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1233858
Human Factors : Confusion
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1234055

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Cleared to descend on the BRDJE ONE arrival. ATC issued an instruction to maintain FL240 and a speed of 290 KIAS. (I am not positive of these numbers but I am sure there was a modification to the arrival). Subsequent clearance was to descend via the arrival and normal speed. The next fix we were coming up on was SEEVR to cross below 23000 but above 20000. We set 8000 in the altitude select window and managed the speed and the altitude. This I am sure of. At this point we checked the FMS against the chart and both pilots verified the restrictions would be met. 

I then got busy with talking to the passengers to update arrival time and thank them for flying with us and such. Also called operations to get a gate assignment. When I finished with that I got back in the loop and noticed the airplane altitude was 17000 feet. I noticed we were not yet at SEEVR but the airplane had descended to the next fix after SEEVR altitude (BRDJE between 19000 and 17000). The altitude was constrained at 17000 at this point. We selected a level of at 17000 with a push of the VSI selector until we could figure out what happened. I immediately contacted center and told him we were 3000 feet too low at 17000 feet. He asked me if we were on the BRDJE arrival and if I was familiar with it to which I replied "yes". He said maintain 17000 and descend via the BRDJE arrival. There was no mention of a deviation and no traffic conflict of any kind. The remainder of the trip went without incident.

I am not sure of what I can add to help resolve the problems associated with these arrivals other than to beat the same horse about monitor, monitor, monitor. I do not want it to be an excuse for my failure to fly the airplane at the proper altitude, but these arrivals have problems with our airplanes as well as other operators. I have a 30 year record with no altitude deviations other than the last month or so, and now I have two. Both of them on these new arrivals. I have lost confidence in the automation and how it works on these arrivals. I think that I will start flying them all in selected mode and just meet all the lower altitudes. I have observed this on several [other] Airline flights that I have jump seated to work on. Also, these arrivals may (probably would) work just fine if the controllers would not make modifications to the altitudes or the speeds. It defeats the purpose and intent of the arrival when it has to be adjusted manually to meet the requirements of ATC.

Narrative: 2

While enroute starting our initial descent into DFW we were given vectors off course for spacing. I believe it was 20 to the right of course. At the time we had just finished a restroom break. I was informed of the vector when I got back into the cockpit. ATC issued an instruction to maintain FL 240 and a speed of 290 KIAS. We were then instructed to proceed direct SEEVR. 

I was reviewing the arrival procedure and the approach plates for both 35C and 36L, and the airport diagram chart in case we got a late switch in runways. We were both then putting on our shoulder harnesses, putting our headsets on and getting ready for the approach checklist procedures. Before I realized it we were descending through 17200 feet still several miles from SEEVR. I immediately pressed the VSI selector to level the airplane off as I knew the altitude was not correct for our location. 

I am still fairly new to the airbus and the managed concept with the automation from the MCDU. I had put my confidence in this concept and it makes sense when you can see the constraints on the MCDU and the airplane is managed that it will follow the programming. I think that I will start flying them all in selected mode to meet all speeds and altitudes. I am not sure if being vectored off course for spacing and then direct to a fix on the arrival made the automation fail to capture the altitudes or not. The speed seems to not change, however on other arrivals I have noticed with the large speed window tolerance the aircraft allows it is easy to get slow on speed for some arrivals. I have been selecting speed when this occurs. It defeats the purpose and intent of the arrival when it has to be adjusted manually to meet the requirements of ATC.

Synopsis

After having been vectored off the BRDJE RNAV STAR to DFW and descend to FL240 an A320 flight crew was cleared direct to intercept the STAR at SEEVR, and cleared to descend via the arrival. Believing they had assured the arrival was properly programmed both pilots undertook other required preparations for the arrival. Shortly, they noticed that they had descended well beyond the minimum SEEVR crossing of FL200 although they had not yet reached that fix. They advised ATC who re-cleared them and the balance of the flight was uneventful. Both airmen expressed skepticism about the ability of their aircraft to successfully navigate the complex procedures, especially when any ATC alteration to the line selectable procedure is required by ATC.

ACN: 1233195 (1772 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 1233195
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was acting as pilot monitoring (PM) while First Officer was the pilot flying (PF).

At our cruise altitude of FL380 a HYD Y RSVR LO LVL, ECAM appeared on our screen.

After completing the actions required by the ECAM and checking in the QRH, I contacted dispatch via ACARS.

Two attempts were made on two different frequencies suggested by dispatch, to talk to dispatch. Both attempts were unsuccessful. We completed communication via ACARS.

Dispatch decided that the best thing to do would be to return to [our departure airport and] I concurred.

Center was advised that we wanted to return and a new clearance was received.

At this time I called, via interphone the Purser and requested her presence in the cockpit. I explained the situation, advised her that an evacuation or cabin preparation was not necessary, that we had approximately 40 minutes flight time until landing and that at this point we had no idea what was going to happen in regard to the passengers, gate information, clearing customs or the continuation of the flight. 

An attempt was made to send [an equipment code report] as to the loss of yellow hydraulic system fluid to maintenance but we received an ACARS message stating ''' INIT ERROR, ACARS INIT ISSUE, PLEASE CHECK "FLIGHT NUMBER" DEPT STATION "DEST STATION" FLIGHT DATE. SUGGEST AUTO INITALIZATION TO CORRECT."

A message was sent directly to MAINT via ACARS as to the write up, no response was received for approximately 20 minutes. A message was sent to dispatch asking if they had notified [the airport] of our return and no response was received. A message to airport operations via ACARS was sent directly and no message was received.

There is an issue here, we as pilots are doing our best with a damaged aircraft and expecting support that has been promised to us, to provide us some information, the passengers some information and nothing comes back to us.

This is the second time in two years that I have been the pilot in command of [one of our] aircraft that experienced a problem in flight and was unable to contact or be contacted by dispatch in a timely manner. I realize that dispatch is busy, understaffed and over worked but a little help would go a long way.

Synopsis

An A320 Captain, after suffering the loss of yellow system hydraulic quantity, opted to return to their departure airport but found himself unable to communicate reliably with either Dispatch or Maintenance. The reporter expressed in strong terms his conviction that the company was not providing the communications capabilities required by their flight crews and required by regulation.

ACN: 1232667 (1773 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Gear Extend/Retract Mechanism
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1232667
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy

Narrative: 1

Lead Mechanic assigned myself, Aircraft Maintenance Technician (AMT) X and two other AMTs, Y and Z, to Task Card XXX002 with attached Engineering Change/ Repair Authorization (EC/RA) to secure and torque the R/H main landing gear side stay bushing. Lead Mechanic went to Hangar-B to secure a torque wrench, I cleaned and prepped the side stay and the attach points on the main landing gear. AMT Y ordered the sealant needed for the job. We briefly reviewed the paperwork with [Lead Mechanic] and he told us the torque was 500 foot pounds. We started with 440 foot pounds but could not get any of the tabs to line up. I think we advance the torque setting to 480 foot pounds one time and could not get any tabs to line up. We then advanced the torque setting to the maximum of 500 foot pounds and still could not get any tabs to line up. We backed off the nut and re-torqued it at least 3-4 more times with an inspector present and were never able to align any of the tabs. We discussed our dilemma with the midnight shift supervisor and he turned over the job to midnight shift Mechanic X. Midnight AMT X reviewed the paperwork and pointed out that the paper read 500 lbf. in. It was obvious to us at that point that we had over torqued the side stay bushing. I believe that lbf. in (pounds force inches) is very confusing and led to our mistake in applying the proper torque for the job. Perhaps lb.in. (pound inches) or spelling out foot pounds or inch pounds would be clearer.

Synopsis

An Aircraft Maintenance Technician (AMT) reports how misinterpretation of an A320 aircraft Job Card reference contributed to the right-hand Main Landing Gear (MLG) side stay bushing retaining nut having a final torque twelve times greater than specified. Issue involves Airbus abbreviations for Pounds Force Inches (lbf) (in.) versus more common reference of lb. in. for Foot Pounds inches.

ACN: 1232107 (1774 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 31000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Aircraft Heating System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1232107
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Aircraft
Primary Problem : Procedure

Narrative: 1

Departed with an MEL'd Pack Flow Sensor. All MEL procedures were complied with. This included leaving the Hot Air p.b. switch in the OFF position. About 10 minutes after takeoff, Flight Attendant (F/A) called to say that cabin was cold. I adjusted the cabin temp selectors to a warmer position. As I monitored the cabin temp getting colder instead of warmer, I selected increasing amounts of warm temperature with no effect on the cabin temperature. A F/A called again to say the cabin was REALLY cold. By now we were level at FL310. The observed temperatures were 35 degrees F air going into the cabin with the cabin temp at 60 F and decreasing with all cockpit and cabin temp selectors selected to full hot. 

I initiated a call to dispatch and was patched through to maintenance to see if they had any ideas on how to fix our problem. I was told that they had nothing and that this particular MEL had been a problem before. In fact he added that this MEL should instead be an aircraft refusal. Armed with this knowledge I advised dispatcher that we would not be able to continue to [destination.] He suggested (two airports ahead of us). By now cabin temp was 54 F. [and his closest option] was still 450 miles away. I advised dispatch that my preference was for ZZZ, 120 miles directly south. Dispatch said [it was] my call and, at that time, the phone patch was lost. 

My First Officer (F/O) got the weather for ZZZ1 and ZZZ. ZZZ weather was good but ZZZ1 visibility at a mile and a half. With increasing cold and discomfort on the aircraft (by now the cockpit was uncomfortably cold), I decided safest course of action was diversion to ZZZ with overweight landing. Diversion was coordinated with dispatch over ACARS and then requested with ATC. Cabin was prepped for normal arrival. 

As we approached ZZZ, I received message from dispatch that gate would not be available for some time. I decided to enter holding to take a little extra time to review the overweight landing procedure in the QRH. I also called ops via VHF and found out that we had both an open gate and a mechanic standing by. With good weather, light winds, and a [very long] runway, I was confident that the overweight landing was the safest course of action. Out of extreme caution I had the F/O [advised ATC] and asked to have the equipment standing by. We made a very uneventful landing at 149400 pounds, touching down at less than 200 FPM. The taxi to the gate was normal. 

I will not take an airplane again with this MEL unless the flight is under an hour and then only in the summer.

Synopsis

An A320 dispatched with a deferred AC Pack Flow Sensor was unable to heat the aircraft interior and diverted to an enroute airport for maintenance.

ACN: 1231963 (1775 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Repair

Component : 1

Aircraft Component : Fuselage Main Frame
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Fuselage Attachment
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1231963
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This report is being submitted in order to bring attention to the third recorded Airbus 320 keel beam re-work damage. This Aircraft is currently undergoing a maintenance visit at our Maintenance Base. During the inspection of the Keel beam area, it was found to have numerous mis-drilled/mis-located holes into the structure. It is believed that this damage occurred during a previous maintenance visit by an Outside vendor. The reason this area was worked previously, was to comply with an Engineering Authorization (E/A) to strengthen the Keel beam areas that were manufactured prior to Airbus using a stronger Keel beam structure currently being used today. The original Keel beam modification was covered on numerous Aircraft during maintenance visits at Airport ZZZ. Aircraft / Cabin potential damage caused by other damage. Contracted Maintenance. [Recommend] to have a fleet directive assigned to all Airbus Aircraft that had this modification performed at the Outside Vendor (OSV).

Callback: 1

Reporter stated that so far, three out of four of their A320 aircraft with serious keel beam damage occurred during a keel beam modification intended to beef-up, strengthen the earlier A320 aircraft Keel beams. Instead, the Airbus Service Bulletin (S/B) the outside vendor had signed-off as accomplished at the repair station during a previous maintenance has now caused extensive maintenance delays and an engineering nightmare for obtaining approved repairs to the keel beam mis-drills and mis-located holes for the main landing gear door fittings. His air carrier has decided to issue a fleet directive to try and identify just how many of their A320s are affected. He and other mechanics at his air carrier are not encouraged by what they have found so far.

Synopsis

An Aircraft Maintenance Technician (AMT) reports another company A320 aircraft was found with numerous mis-drilled and mis-located holes in the aircraft's keel beam. Damage was believed to have occurred during a previous maintenance visit by an outside vendor who had signed-off for accomplishing an Airbus Service Bulletin to strengthen the earlier model A320 keel beams.

ACN: 1231459 (1776 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Elevator ControlSystem
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1231459
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

We were cleared for takeoff on Runway 22L, as we initiated our takeoff at about 20 or 30 knots we received an ECAM message "F/CTL ELAC 1 PITCH FAULT". We rejected the takeoff and notified Tower that we aborted our takeoff and needed to clear the runway to perform a procedure for an alert we had received. After clearing the runway we applied QRH procedures and informed Maintenance Control. We were advised to do an ELAC1 reset which solved the problem. Upon completion of the procedure we proceeded to Runway 22L for another takeoff and received the same fault again with another rejected takeoff. After that we were advised by Maintenance Control to return to the gate. Maintenance deferred ELAC1 which required us to perform a fairly lengthy flight crew action associated with the MEL. After they completed the deferral there were no further faults were noted and the flight continued without any other faults. Maintenance advised us that our aircraft was not the only one with this problem that morning, and it seemed that due to the extreme frigid cold temperatures over the USA, that it was playing havoc and the "ELAC1 Pitch Fault" alert had happened to 6 other company aircraft.

Synopsis

A320 First Officer experiences an ELAC1 fault during takeoff and the takeoff is rejected. The ELAC is reset and the second takeoff attempt produces the same result. The crew returns to the gate and Maintenance defers ELAC1, allowing the fight to depart. The reported states that he was informed by Maintenance that six other company flights that morning had the same fault, which was attributed to extreme cold weather.

ACN: 1231417 (1777 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Gear Extend/Retract Mechanism
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 17
ASRS Report Number.Accession Number : 1231417
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was assigned to work on securing the RH main landing gear side stay bushing Task Card with an Engineering Change/Repair Authorization (EC/RA). I, Mechanic X, was directed by my Lead Mechanic to work with Mechanics Y and Z. Lead Mechanic handed me the task card with EC/RA attached just before our afternoon break. I placed the paperwork in my toolbox and locked it, then went to break. Immediately after break, Lead Mechanic met with myself and Mechanics Y and Z to discuss the job. Lead Mechanic told us that the new retaining nut and washer were on order and should be delivered within minutes, and to go ahead and set up for installation. At that time, we briefly went over the paperwork for this phase, and the Lead Mechanic told us the torque was 500 foot pounds. While I ordered the sealant as required, Mechanic Y went to the hangar to obtain the torque wrench for this operation. Upon his return, I set the tooling in place, put the nut and locking tab washer in place, spun it down by hand, and then engaged the tooling to begin the final torquing of the retaining nut. Mechanic Y read off to Mechanic Z that the final torque setting was 500 foot pounds and that the initial torque setting was 440 foot pounds. Mechanic Z then set the torque wrench to 440 foot pounds, showed the setting to our inspector, and then attached the wrench to the tooling. I held the tooling in place while Mechanic Y and Mechanic Z applied the torque to the retaining nut. 

Once the initial torque was reached, we checked the tab lock positions (myself and our inspector), and it was necessary to advance the position of the retaining nut by close to 1/4 inch to align the lock tab. Once we reached 500 foot pounds, the tab lock was still not aligned. The inspector instructed us to back the collar off, and then reapply the minimum torque of 440 foot pounds, and recheck the tab lock position. We continued this through four break/reset sequences with no better luck. Mechanics Y and Z and myself went to see the incoming midnight supervisor, and explained the dilemma. He took the paperwork and briefly perused it, then said that we should turn the issue over to the incoming crew. We turned the paperwork over to Mechanic A, an Aircraft Maintenance Technician (AMT) on midnight shift, and explained what problem we were having. Mechanic A left our presence with the paperwork, and then returned to me approximately 15 minutes later to show me that he read (after struggling with the interpretation) that the applied torque was to be no more than 500 INCH pounds. The paperwork as written (exactly) showed "500 lbf. in", in the text. By this misinterpretation, the applied torque was 12 times greater than was intended in the operation.

Callback: 1

Reporter stated there is a difference between Airbus and Boeing in the way maintenance information is presented to mechanics. References noting foot pounds and inch pounds are simpler and more clear-cut with Boeing Manuals. Boeing uses LB/FT for Foot pounds and LB/In for Inch pounds; whereas Airbus will reference Foot pounds Force with LBF-Ft and Inch pounds with LBF-In. Their Job Cards have the same information as the Airbus Aircraft Maintenance Manuals (AMMs), but in their situation, no diagrams/pictorials were included due to Airbus having sent the special tooling directly to Maintenance. Most mechanics take about a year to learn how to use Airbus AMMs and not get themselves into too much trouble due to misinterpretation of the information they are reading. But, as his report shows, even experienced technicians still mis-read Job Cards.

Synopsis

An Aircraft Maintenance Technician reports how misinterpretation of an A320 Job Card reference note contributed to a right-hand main landing gear side stay bushing retaining nut final torque being twelve times greater than specified. Technician also noted the differences in how information is presented to AMTs using Boeing or Airbus Aircraft Maintenance Manuals.

ACN: 1231361 (1778 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1231361

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1231414

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[On] short final the nosewheel steering failed. Landed and taxied clear of runway. Towed to the gate by a super tug.

Narrative: 2

While performing visual [approach] we received an ECAM/wheel nose gear steering fault upon gear extension. The wind was gusting to 20 kts from the west. We were lined up on final with the aircraft stabilized at final approach speed and in the landing configuration. We opted to continue the approach and land into the wind realizing and informing tower that we may have to stop the aircraft on the centerline and may not be able to exit the runway. After landing and slowing the aircraft to taxi speed we were able to use rudder authority to exit the high speed taxiway and stop the aircraft clear of the runway. We attempted a reset per the QRH and it was unsuccessful. We contacted maintenance and operations and requested a tow in. Maintenance later informed us that one of the hydraulic solenoids had failed and would be replaced. We both agreed that since we were in a stabilized approach configuration and landing directly into the wind our greatest threat would be not be landing but taxiing. The decision was made to continue the approach and stop straight ahead and evaluate our ability to taxi once the aircraft was safely on the ground and stopped.

Synopsis

A320 flight crew on a visual approach reported receiving a nosewheel steering ECAM message on short final. Crew elected to continue landing.

ACN: 1231201 (1779 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Icing
Work Environment Factor : Temperature - Extreme

Aircraft

Reference : X
ATC / Advisory.Ground : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1231201
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23600
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8600
ASRS Report Number.Accession Number : 1231343

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Taxi
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

We arrived at the de-ice pad, set brakes, and configured for de-icing as there was at least 1/4 inch of hard clear ice on the wings and fuselage. We were de-iced at approximately XA:55 with a clear aircraft, start of last step at XA:47. We de-configured, contacted ground and proceeded to taxi. As soon as we did both the front and rear flight-attendants called us to say that they still could not see out their jumpseat windows on the left side of the aircraft due to the ice, and that passengers were taking pictures of their iced up windows with their cell phones. We elected to return to the de-ice pad, and were assigned a different bay this time. A de-ice coordinator, checked the fuselage and said the aircraft was clear of all ice and that de-icing fluid may appear like ice. The captain elected to do a visual check himself and concluded all windows WERE covered in ice and to have the airplane properly de-iced. The amount of time this deicing took seemed proper and we were told last step started at XB:08. We visually checked and the windows were clear this time.

Narrative: 2

We were the first flight of the day. The aircraft was coated with ice from freezing drizzle over night. There was no precipitation from when we arrived at the aircraft until takeoff. The APU and packs were running when we arrived at the aircraft. The cabin temp was increased to nearly full hot prior to passenger boarding to try to assist with fuselage ice removal. OAT was about -3 C. Extra fuel was requested and granted in the event that deicing were to take longer than dispatch had estimated. There was no delay getting into the deice pad. We configured and were told we would get a full body hot type I. I asked how long it was estimated to take and was told about 15 to 20 min. In less than 10 min we were told that deicing was complete and given the deicing data. After beginning the taxi out of the pad both front and rear FA called to say that there was still ice on the windows. We told the "iceman"; we wanted to come back for a proper deicing. We pulled into a different pad with a different deice crew and initially were told that the windows looked clean. I walked through the cabin and observed that most of the windows especially on the left side were still iced over. Another round of deicing followed and took about 15 to 20 minutes. I walked the length of cabin again and observed that the ice had been removed. We received our deicing information and had an uneventful departure and flight.

Synopsis

A320 flight crew reports having to return to the deice pad after being told deicing was complete, due to ice adhering to the fuselage and passenger windows.

ACN: 1230642 (1780 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : PHX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1230642
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Human Factors

Narrative: 1

On departure I noticed that the airplane was climbing slowly, and realized that the landing gear was down, I then requested that the gear be brought up. The gear was retracted at 218 KIAS and in the retraction process the aircraft exceeded the gear retraction speed. Although the over speed was not known at the time. It was not until I was notified by my company that I became aware. 

I am not entirely sure what distracted us, although I have a strong feeling that it was related to wake turbulence avoidance.

Synopsis

A320 First Officer reported flight crew forgot to retract the landing gear, probably because of wake turbulence distraction.

ACN: 1230607 (1781 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Fog

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1230607

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18427
Experience.Flight Crew.Last 90 Days : 207
Experience.Flight Crew.Type : 4007
ASRS Report Number.Accession Number : 1230615

Events

Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Un-forecast fog quickly developed during our approach into Airport ZZZ. The RVR at ZZZ was approximately 1,000 feet and holding steady. This Airbus is not authorized for autoland at ZZZ1 so our minimum visibility was 1800 RVR. Airport ZZZ1 was near minimums and just out of fuel range. Airport ZZZ2 was below minimums. The only airport within fuel range that had weather at or above minimums was Airport ZZZ3. ZZZ3 was long enough, wide enough, in the database, and we had charts. We notified dispatch we were going to divert to ZZZ3. Dispatch notified us that ZZZ3 is not approved for the Airbus and we would need to declare an emergency. We declared an emergency and landed at ZZZ3.

Narrative: 2

On final approach the weather at Airport ZZZ went below CAT 1 minimums and because of The Airbus limits we were not able to Autoland at ZZZ. Because of the un-forecast low visibility we needed to divert to a safe suitable airport. Airport ZZZ2 was also below minimums and we did not have adequate fuel to safely proceed to any other airports that were not in the area. We discussed our options with Dispatch and he suggested that we declare an emergency and divert to Airport ZZZ3 field. At the time of diversion ZZZ3 had weather that was above our landing minimums so we proceeded to ZZZ3 without incident.

Synopsis

A320 flight crew reports arriving at their destination in their CAT I restricted aircraft, to find the weather below minimums along with all nearby alternates. A suitable field is identified by Dispatcher and advises it is not approved for the A320, but could be used if an emergency was declared. Flight crew declares an emergency and lands.

ACN: 1229729 (1782 of 2908)

Time / Day

Date : 201501
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Icing
Weather Elements / Visibility : Snow
Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1229729
Human Factors : Situational Awareness

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

We taxied into gate during cold weather ops. All airport surfaces were covered in snow with braking action good to fair. After receiving the chock signal from the guide man, I released the parking brake due to hot brakes and a quick turn. I went heads down to complete the parking checklist and noticed the aircraft had started moving backwards. I reset the brakes and looked for the guide man who was looking at the nose wheel but gave me no signal to reset the brakes. Another ramp person emerged from under the nose and gave me the set parking brake signal. I spoke with a ramp supervisor. He stated that the chocks were not strong enough to hold the aircraft due to ice under the snow. From the tire and shock drag marks in the snow I estimated about two feet of rearward movement from the original stop mark. I'm not sure how much movement occurred after I reset the parking brake. Luckily the belt loaders had not yet pulled up to the aircraft. This was a new and unexpected experience for me and I could not find any reference or warnings for this type of occurrence in the cold weather ops section of the flight manual.

Synopsis

A320 Captain experiences aircraft movement at the gate after being choked and releasing the brakes on an icy ramp.

ACN: 1228849 (1783 of 2908)

Time / Day

Date : 201412

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1228849
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23300
Experience.Flight Crew.Last 90 Days : 300
Experience.Flight Crew.Type : 12000
ASRS Report Number.Accession Number : 1226541

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 3700
Experience.Flight Crew.Last 90 Days : 73
Experience.Flight Crew.Type : 500
ASRS Report Number.Accession Number : 1226544

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Dispatch was advised the involved flight taxied out and was tailheavy. load planning asked dispatch to advise crew of situation and ramp was going to rectify situation by removing approximately 1300 lbs. of weight from rear. Dispatch sent crew message about tailheavy situation and that ramp was going to resolve before departure. crew acknowledged. later after flight reported off ground, loadplanning advised dispatch that ramp never removed weight and flight took off tail heavy. dispatch advised crew once airborne of situation, crew reported that final weights sent to ac before departure did not reflect tailheavy situation, and that crew did not experience any parameters on takeoff that would indicate a tailheavy situation. flight continued on and landed without incident. crew was asked by [a duty flight manager] to file [a safety report].

Narrative: 2

Number one for takeoff without final weights. Sent msg to load planning after receiving the [ACARS] message to standby, sent msg to dispatch. Dispatch stated we have extra weight in the aft pit that they are rectifying. Some time passed and the final weights came thru. The numbers were within normal range and we departed.

Narrative: 3

Final weights were not provided as we approached runway for takeoff. Contacted load planners for final weights and then dispatch to get assistance with obtaining final weights. Dispatch replied that a tail heavy situation was being rectified. Soon thereafter, final weights were sent to the airplane with normal (in limit) CG. Trim was set, takeoff performance data checked and uneventful takeoff/flight/landing followed.

Synopsis

As the outbound A320 approached their departure runway the flight crew was advised by load planning they had a tail heavy weight situation for which 1300 pounds would have to be removed. Shortly thereafter a final weight message reporting an acceptable Center of Gravity/stab trim setting was received and the flight departed and completed the segment without incident.

ACN: 1227800 (1784 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZHU

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 26000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1227800
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

At some point less than an hour after takeoff we were confronted by a huge line of thunderstorms directly ahead and to the east/north east of our position. I am not sure why the Captain and Dispatcher did not seek an alternate route prior to our departure, perhaps the line of thunderstorms moved in an unanticipated manner. Approaching the line, the only safe way to continue, other than doing a 180 and returning to our departure airport, was to take a southerly deviation which took us more than 50 miles offshore on an aircraft that was not over-water equipped. We had a lot of fuel so that was not an issue at all. I don't approximately know exactly how far offshore we got because I was too busy making sure we did not get too close to these buildups. 

We were approximately 220 miles or so from MSY when we started the deviation. The plan was to skirt the south west side of the thunderstorms and this we did in safety and in smooth air. We reached MSY as planned, in fact we were able to stay east of MSY, and thereafter continued north to our previously filed route in an uneventful manner. The deviation added about 30 minutes to our scheduled flight plan time, with which the extra fuel was not an issue whatsoever. In lieu of the unexpected size, height, and extent of the line of thunderstorms, I believe this was by far the safest course of action in spite of the assumptive violation of the 50 mile rule.

Synopsis

An A320 flight crew facing unplanned thunderstorms, flew beyond 50NM offshore while circumnavigating the weather.

ACN: 1227165 (1785 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Component

Aircraft Component : Lubrication Oil
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1227165

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17234
Experience.Flight Crew.Last 90 Days : 246
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1227195

Person : 3

Reference : 3
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1227208

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise roughly 1 hour out, while doing a system check, noticed oil quantity low and decreasing. A message was sent to Maintenance (MX) describing the condition and the possibility of an erroneous reading. Oil continued to decrease and at zero, the oil pressure started falling. This verified our quantity issue. We reduced the #2 engine to idle and assessed our diversion options. Considerations included: mountainous terrain ahead, heavy weight landing, weather, and the upcoming single engine situation. 

We received clearance from Center for a pilot's discretion descent to FL240. Our best choice at the moment was to divert to ZZZ at our left 7:30 position and a distance of over 120 miles, with a nice [long] runway. We received this clearance and relayed our plan to dispatch via ACARS. At about the same time the Oil Pressure ECAM came on dictating the engine shutdown. Stable inbound to ZZZ we declared an emergency with ATC and followed it up with dispatch. 

During this entire process [the First Officer] flew the airplane and I handled the ECAM first and then the QRH engine shutdown checklist. By now we are level at FL240, unable to maintain any higher altitude comfortably. Dispatch forwarded the landing data for our heavy weight condition, weather was checked, and the visual backed up by the ILS 21 was input and briefed. Flight Attendants were notified of our situation and to clean up the cabin. A short time later the seat belt sign was turned on and an announcement made to the passengers. Heavy Weight landing checklist was the next one reviewed and with that and normal checklists done, we were ready for the landing. With favorable conditions, and his leg, [the First Officer] flew the excellent approach and landing. With our declared emergency. Taxiing in, hydraulics and steering remained normal and the fire crew followed us to the gate.

Narrative: 2

The autopilot was on and we set up for an overweight landing.

Narrative: 3

...crew landed about 15000 lbs overweight.

Synopsis

When the right engine oil quantity fell to zero and the oil pressure dropped as they approached a mountain range, the flight crew of an A320 shut down the engine, requested a lower altitude and diverted to an airport behind them.

ACN: 1226839 (1786 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ground : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16500
Experience.Flight Crew.Last 90 Days : 31
Experience.Flight Crew.Type : 1880
ASRS Report Number.Accession Number : 1226839
Human Factors : Situational Awareness
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1226843
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Aircraft taxiing south on taxiway R cannot see westbound vehicular traffic on the access road approaching taxiway R until they are immediately adjacent to taxiway R near the southwest corner of the hanger. This blind spot is due to the close proximity of the hanger to taxiway R. After the First Officer (FO) contacted South Ground near check point 2 on taxiway R and told them that we were parking at gate XX, the controller responded that the gate was occupied and instructed us to turn right onto taxiway B to hold short of taxiway U (pointed west). The South Ground frequency was congested but the FO was able acknowledge the clearance back to ground. FO told me that he would contact company/ramp to determine when the gate and ally would clear. I would continue to monitor ground control frequency.

We were approaching the southwest corner of the hanger (on my left) and access road on taxiway R, when out of the corner of my eye I picked up a westbound black pickup vehicle moving at high speed (estimated 40 -50 mph) headed for the access road. I uttered an expletive and immediately jammed on the brakes to try to stop but by this time we were on top of the access road. Hearing my expletive, the FO then also saw the vehicle. The driver didn't appear to notice us until he was on the taxiway directly in front of us and only then appeared to swerve before disappearing under the nose of the aircraft. Both FO and I momentarily braced for collision until we saw the vehicle re-emerge under the right side of the nose and continue west to the southern hardstand area. Due to the aggressive braking, the aircraft came to an abrupt stop on the access road but once we saw the vehicle head for the hardstand we continued taxi as instructed. The FO who had switched his audio panel to make the gate query call inadvertently reported the "near-miss" to ramp before switching back to ground to report the incident to the ground controller. Ground responded that they would report the vehicle to Airport Ops, but he was very busy working other aircraft and we knew that this report was a low priority for him.

Narrative: 2

[Report narrative contained no additional information].

Synopsis

An air carrier crew took evasive action to avoid a collision with a speeding vehicle on taxiway R near the maintenance hangar.

ACN: 1226023 (1787 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SJC.Airport
State Reference : CA
Altitude.MSL.Single Value : 4700

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 12R
Flight Phase : Descent
Airspace.Class C : SJC

Component

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 6059
Experience.Flight Crew.Last 90 Days : 93
Experience.Flight Crew.Type : 46
ASRS Report Number.Accession Number : 1226023
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 1500
Experience.Flight Crew.Last 90 Days : 85
Experience.Flight Crew.Type : 1900
ASRS Report Number.Accession Number : 1226034

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

We were being vectored for an ILS approach to 12R in SJC. The conditions where IMC with moderate turbulence. The NorCal controller cleared us to descend to 5100 and cleared us direct to JESEN intersection. While level we received a GPWS terrain/terrain. I was the pilot flying so I performed the terrain escape maneuver per [company procuedures] and climbed approximately 900 feet. ATC was notified and she responded that the Minimum Vectoring Altitude for the area we were [at] was 5100 feet, she then told us to descend to 4700 feet so we descended back down to 4700 feet and were given a heading of 310 and continued vectoring for the approach and landed without incident.

Narrative: 2

At 5100 while being vectored for an approach to RWY 12R we had a GPWS warning. Night IMC, and the approach plate showed MSA of 5600. We performed terrain escape maneuver to silence the warning then advised ATC. Maximum altitude was about 6000 feet (climb from 5100). Moderate turbulence at the time of the occurrence with updrafts and downdrafts. ATC responded that minimum vectoring altitude in that sector was 5100 feet and vectored us to final. I suggested the occurrence might have been caused by the moderate turbulence. Total time of the event was approximately 2 minutes. Location of the event was approximately N37.07 W121.82, having been cleared direct JESEN prior to JAWWS.

Synopsis

A320 flight crew experiences a terrain warning at 5100 feet south of SJC in night IMC, while being vectored by NCT. Evasive action is taken by the crew, climbing to 6000 feet although NCT states that 5100 feet is the correct MVA for that area. The Captain believes the turbulence may have been a factor in the false terrain warning.

ACN: 1225515 (1788 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 4500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : PHX

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1225515

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1225118

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On approach to final to PHX Runway 25L, aircraft was targeted by a high intensity ground based laser while descending through 4500 feet. Laser emanated from a residential area approximately 10 miles south of the 25L approach corridor. Laser was pointed directly at cockpit using the aircraft landing lights to take aim. At initial detection, Captain (PF) rapidly ducked head into cockpit while First Officer tried to pinpoint exact location. Unfortunately, FO suffered corneal burns to his right eye in doing so.

Narrative: 2

Approaching DERVL intersection saw a flash at 12 o'clock. Looked up and realized that there was a green laser tracking and hitting us from the front-center. I covered my eyes and looked down and commanded "laser - look down" to the FO. He had already been hit in the eyes.

Synopsis

A320 flight crew reported being targeted by a laser on approach to PHX Runway 25L that resulted in corneal burns to the First Officer's right eye.

ACN: 1225165 (1789 of 2908)

Time / Day

Date : 201412

Place

Locale Reference.Airport : ORD.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Ramp : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : B767 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 40
Experience.Flight Crew.Type : 4770
ASRS Report Number.Accession Number : 1225165
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected.Other 
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During push back from gate, the push back driver made an abrupt stop and stated a B767 cut him off. I asked him if everything looked ok and if there was any visible damage to the tow bar and wheel assembly. He stated the tow bar pin had popped out and was calling a supervisor. After he cleared me to set the parking brake I advised him we were going to write up the nose gear assembly for inspection and call maintenance. He acknowledged and left to get his supervisor. I asked the First Officer to check with Ramp Control if we were in fault in any way, and Ramp Control confirmed we were cleared to push with no restrictions and were not at fault. After Maintenance completed its inspection we took off uneventfully.

Synopsis

A320 Captain reported his ORD push crew had to stop abruptly to avoid a collision with a B767 that taxied behind them.

ACN: 1225139 (1790 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : RNO.Airport
State Reference : NV
Altitude.MSL.Single Value : 7700

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : RNO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Final Approach
Airspace.Class C : RNO

Component

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1225139

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1225182

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On final approach to ILS runway 16R inside Tackle final approach on glide path and on locaizer at approximately 7,700 feet- received a GPWS warning with no other EGPWS indications of terrain on scope or radar altimeter warning. "Terrain terrain pull up" narrated then ceased. Began to initiate recovery, but stopped as warning stopped abruptly and runway in view and glidepath normal. Landed.

Narrative: 2

[Report narrative contained no additional information].

Synopsis

A320 flight crew reported receiving a momentary GPWS "Terrain - Pull Up" warning on approach to RNO when they were on the glideslope with the runway in view.

ACN: 1224424 (1791 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PHL.Airport
State Reference : PA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : PHL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 1224424
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We had received a clearance from PHL ground to taxi to hold short of rwy 27r, via taxiway kilo3. We were not told to give way to any aircraft on our way to hold short of 27r. I noticed that there was a Regional Jet taxiing on taxiway kilo, east bound towards taxiway kilo 3. I slowed and stopped to ensure there would be no conflict with the Regional Jet. I saw the Regional Jet stop just prior to the intersection of taxiway kilo 3 and kilo. I intended to ask my co pilot to confirm with phl ground that we had right of way to taxi, when I saw the Regional Jet start taxiing again. My aircraft was past taxiway Juliet, on kilo 3, holding short of taxiway kilo. The Regional Jet started turning left, in to kilo 3. I turned on my taxi/landing light and yelled "STOP!" on the ground frequency as the Regional Jet continued his turn in to me. The Regional Jet moved to the far right of taxiway Kilo 3, far to the right of the yellow taxi line. The Regional Jet's left wingtip passed within feet of the radome on my aircraft. 
The incident occurred because [the other aircraft] was incredibly reckless and dangerous in the operation of his aircraft. There was no obstruction by my aircraft on taxiway kilo. He could have continued his taxi straight ahead, or after turning in to kilo 3 could have stopped and had tugs clear us apart.

Synopsis

A320 Captain taxiing at PHL reported a Regional Jet deviated from his clearance and taxied dangerously close to his aircraft.

ACN: 1224131 (1792 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SJC.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ground : SJC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8500
ASRS Report Number.Accession Number : 1224131

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1224152

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

During pushback the push crew said the tow bar had disconnected and to stop the aircraft, I applied light brake pressure and came to a stop. He said the tow bar was defective and they confirmed that there was no damage to the nose gear. We proceeded and the flight was uneventful. After rereading the breakaway procedure in the Normal Pushback section it was noted that we should have had maintenance inspect the nose gear. I called Maintenance Control and they had the inspection done. No damage was observed.

Narrative: 2

When looking at the Breakaway procedures section, there is no warning of contacting maintenance in the actual breakaway procedure section. It was only after the flight that the Captain found the warning on a previous page at the beginning of the normal section.

Due to using a tablet vice a physical manual, it was not readily apparent that maintenance should be contacted during any issues with the tow bar. I would recommend an additional warning be added to the breakaway section.

Synopsis

When the towbar broke during pushback, the ground crew alerted the Captain of the A320 who brought the aircraft to a gentle stop. The ground crew advised there was no visible damage and the flight crew continued the flight. They later discovered a normal procedure AOM reference required a specific inspection after a breakaway from a tow. Maintenance was advised and the procedure completed at the next stop. Both pilots advised the location of the requirement, on tablet pages preceding the breakaway procedure, made it questionable that it would be seen by flight crews accessing only the procedure itself.

ACN: 1224129 (1793 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Rudder Trim System
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1224129
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

ACARS printed Maintenance Flight Release danger. Dispatched with a number two rudder trim system inoperative. There is a flight crew system test procedure required before each flight with numerous steps. There are different procedures for the A319 than the A320 aircraft with different values required as trim limits. The safety issue is with the way the ACARS prints the Maintenance Flight Release. Our Maintenance Flight Release was four pages long but printed in a one, two, four, three order thus mixing the procedural steps and we inadvertently continued the procedure for our A320 on the page that was intended for the A319. So the rudder limit of 20 left/right showing on the test (and correct for the A320) read as a failure for the 319 page (which requires 25 left/right). I noticed as maintenance was running their test that their paperwork showed only requiring 20 left/right. I relooked at the Maintenance Flight Release and was able to deduce that the pages had been printed out of order leading us to look for the wrong limits during the test. A reprinted Maintenance Flight Release had the exact same page order one, two, four, three. This is an issue that could absolutely affect the safety of flight and procedures. This needs to be fixed.

Synopsis

An A320 crew was provided a maintenance document that had been printed out of sequence. The document as printed would have led them to apply A319 rudder trim test limits to their A320.

ACN: 1223865 (1794 of 2908)

Time / Day

Date : 201412
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Other 
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A321
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1223865
Human Factors : Situational Awareness
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We had briefed a night time VMC Tiptoe approach to SFO 28L off of the Big Sur STAR. Over MENLO at 210 knots heading 330 and 5,000 feet, NORCAL called traffic for 28 R at 3 o'clock (believe it was A321). Captain (CA) called traffic and SFO insight without First Officer (FO) concurrence which led to NORCAL to instruct us to maintain visual separation. CA had a hard time hearing my calls for flaps and speed settings due to ATC frequency congestion and possible hearing impairment. Once cleared to maintain visual separation and cleared for tiptoe visual approach, due to no other information, I began to slow dramatically in order to keep traffic in front of me. Additionally we had a 15 knot tailwind as we approached final course. NORCAL informed us that the traffic had us in sight and was maintaining visual separation. Never did I hear from NORCAL who was to lead and who was to trail on approach. The A321 began to slow likewise and both aircraft were within 1 mile of each other racing to final approach speed attempting to both be the trailing Aircraft. This distraction initially put us very high on profile and which then diverted my attention to getting down instead of assuming the lead in formation with the A321! I lost sight of A321 because the geometry of the rendezvous merge never gave me a chance to trail the other traffic before we had to turn toward runway. I elected to remain a little left off course until 1,000 feet. I believe we got the TCAS traffic advisory around 1,500 feet. I had briefed the CA of my desire to keep the TCAS in TARA for safety before approach but he elected to turn it off once we got the traffic advisory. I am convinced the traffic advisory would have gone to a RA had we kept it on. Based on visual sighting of an A321 at 4 o'clock at 1,000 feet, I estimate we were within 2,000 feet laterally and 50 feet vertically with a 10 knot left to right crosswind (night time SWAG!). As a side note: because we slowed so early, I believe SFO tower squeezed out another departure on RWY 1 which led to a departing airplane crossing our runway below 100 feet AGL. Other than that, the rest of the flight was a normal SFO 28 visual! I contacted NORCAL and spoke with the desk manager who reviewed the tapes of our approach and confirmed everything stated above including our wing tip to wing tip distance. Controller stated that once they issue visual separation they can merge Aircraft within 3 miles but that they didn't understand the premise of our obligation of keeping adjacent Aircraft in front of us is the only way we can assure visual separation. Additionally, the NORCAL controller has written guidance that Aircraft should be paired about 1/4 mile in trail to ensure departure capacity isn't diminished. This seems in direct conflict with NMAC definition and wake turbulence separation criteria. "A near midair collision is defined as an incident associated with the operation of an aircraft in which a possibility of collision occurs as a result of proximity of less than 500 feet to another aircraft, or a report is received from a pilot or a flight crew member stating that a collision hazard existed between two or more aircraft." If wing tip to wing tip isn't the definition of a potential collision hazard I don't know what is! This serves as a pilot's statement of our Near Mid Air Collision.

Synopsis

A319 First Officer does not approve of simultaneous visual approaches to Runways 28 Left and Right at SFO. FO believes that one aircraft must be ahead of the other to maintain visual separation. NCT and SFO Tower do not see it that way.

ACN: 1222577 (1795 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Poor Lighting
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Cabin Lighting
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1222577
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This flight departed at night so it was very dark. The 2014 FA Policies and Procedures book states on page 18 that the ceiling and sidewall lights for the Airbus should be both be "Off" for night time departures. FAs have been taught to set the lights inside the airplane match the brightness or darkness of the time of day so passengers' eyes are able to see as the exit the airplane if there is an emergency evacuation. Page 4.43 of the FAOM states that the cabin view panels must be open to allow greater cabin visibility during critical phases of flight. FAs are taught that we should monitor the cabin for passenger safety and security (passengers standing up during taxi, possible medical emergencies, possible terrorist activity, etc.). These 2 policies are in direct conflict with each other on this airplane at night. When the lights were turned off, the cabin was so dark that it was impossible to see the passengers sitting in the first row of the economy cabin from my jumpseat at 1L, much-less anyone in the cabin. In fact, I could barely see the passengers sitting in row 3. I was extremely uncomfortable that I could not see the main cabin or the passengers. I was concern from a passenger safety standpoint, if anyone got up, I could not see them. If anyone had a medical emergency, I could not see them. And most importantly, if anyone was doing anything to endanger the security of the flight, I could not see them. I weighed the 2 policies and decided to follow the intent of the cabin view panel policy and I left the lights on dim 2 setting for taxi and takeoff. I had tried to turn off the sidewall lights, but leave the ceiling lights on. When I pushed the WDO button to turn off the sidewall lights, the sidewall lights did not go off. When I tried again before the beverage service, I was able to get the sidewall lights to turn off. The service was done with only the ceiling lights on Dim 2 and then all the lights were turned off for the duration of the flight. I was still concerned for the safety of any passengers (trip hazards, unobserved medical emergencies, etc.) but all of the passengers clearly wanted to sleep on this red-eye flight.

I've brought this issue to my supervisor before and expressed my concern for the safety and security aspects mentioned earlier.

Synopsis

An A320 Flight Attendant commented that her Company Policy requiring dark cabin lighting during night takeoff and landings does not permit her to monitor the cabin or passenger conditions.

ACN: 1220624 (1796 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : ZZZ
Airspace.Class A : ZZZ

Component

Aircraft Component : Navigation Database
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1220624
Human Factors : Situational Awareness
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
ASRS Report Number.Accession Number : 1220639

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We were cleared to descend via the STAR. We were linked to runway 26L we had both reviewed the charts and missed the constraint of ZZZZZ intersection at or above FL240. The database in FMC didn't have this constraint. At about 19,000 feet the center controller queried us whether we had the constraint and we told him it wasn't in the database. His response was that we were not the only crew missing the constraint and he recommended reporting it through company and he would report it through ATC ATSAP. We had reviewed the charts and database but we just didn't catch this.

Now let's add in the environmental aspects - while we were mostly flying in VMC the better part of our flight was in light to moderate chop, at FL260 or 240 because we simply could not find and maintain any ride that did not have turbulence in it. We had the flight attendants down and the seat belt sign on for substantial portions of the flight. By the time we got to ZZZ I felt physically beaten up like a "frog in a blender" and I am not certain what else I might have missed. 

It had turned into a long day. I had risen early to get to work, and while cleaning off the car, I slipped on black ice and fell. I had a minor abrasion on my right hand and I struck my right knee. I monitored my progress closely as I proceeded to work, got on the bus, signed fit for duty and flew the first leg. We sat at the intermediate stop for nearly 4 hours before flying to ZZZ1. Physically I was fine but apparently, the event stuck with me and became a focal distraction, further aggravated by being in continuous turbulence enroute. But with an early morning report and a short ride from my icy driveway, I knew there would be opportunities to further assess fitness, continue or not continue and report to the duty manager if it was not possible to continue. But the fatigue issue is only one element - why wasn't the constraint in the database? It just wasn't there, and when we use these procedures, we come to rely on them being accurate --It wasn't. We got caught by this and back checking I see now that we were in error and responsible for confirming and double checking the chart against the database. 

ATC was very accommodating but that doesn't correct the problem. The problem is in the Database publication and it needs to be addressed.

Narrative: 2

[Report Narrative Contained No Additional Information]

Synopsis

When cleared to descend via a STAR, the flight crew of an A320 failed to cross an intersection at or above FL240 as charted. Both pilots advised the restriction was not in the aircraft's NAV database.

ACN: 1219257 (1797 of 2908)

Time / Day

Date : 201411

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Main Gear Door
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Fuselage Main Frame
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
ASRS Report Number.Accession Number : 1219257

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I am writing this statement in regards to an Airbus 320 aircraft. The Aircraft is currently undergoing a C-Check at ZZZ Hangar. During the preliminary Inspection, unusual wear was found on both Left and Right Main Landing Gear Door Fittings. Both doors were removed in order to effect repairs. While removing both doors, they were both found to be missing the Shim material that is installed on/around the hinge pin/bolt area. This is done so that during the final 'rig' on installation, the doors will fair correctly with the Fuselage along with reducing/eliminating any fore and aft play about the hinge centerline. It is believed that the lack of having shims installed contributed to the wear that occurred. Multiple structural parts were damaged beyond limits and had to be replaced with new parts. While the Doors were off the Aircraft, it was also discovered that one of the Hinge fitting fastener holes in the Keel beam was not within specifications, 'oblong'; these fasteners secure the door hinge fitting to the Keel beam. Currently our Air Carrier Engineers are in talks with Airbus EG in order to adequately address the mis-drilled hole.

Synopsis

An Aircraft Maintenance Technician (AMT) reports that both left and right Main Landing Gear (MLG) door hinge fittings were found with unusual wear on an A320 aircraft. Multiple structural parts were damaged beyond limits due to missing shim material on/around the MLG door hinge pin/bolt areas.

ACN: 1219223 (1798 of 2908)

Time / Day

Date : 201411

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Component

Aircraft Component : Speedbrake/Spoiler
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 8185
ASRS Report Number.Accession Number : 1219223
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Our aircraft came in with a spoiler fault on the #4 spoiler. The mechanics attempted to fix, then decided to defer the spoiler inoperative. My First Officer (FO) and I watched the mechanics (as many as 4 with a lead mechanic) work on this problem for more than an hour. They finished the work and told us that the #4 spoiler was now deferred inop. We re-boarded the aircraft, received a new maintenance release and [a dispatch] release from dispatch. 

After pushback we started both engines, accomplished all checklists including the crew actions from the MEL card. We departed IAH without any problem and climbed to a cruise altitude of FL350. Almost an hour into flight we got a flt/cntrl spoiler fault ECAM with spoilers inop 2+4. We went to the spoiler fault procedure in the QRH and ran through the procedure and found that the only concern was runway length since the spoilers showed no deflection and we had no other faults associated with our condition. 

We called dispatch and maintenance control on the radio. We determined that the ZZZ mechanics had indeed inop'd the wrong spoiler set and that spoiler set #4 was not inop'd. We now had spoiler set #2 and #4 inop. We worked with dispatch and calculated using two spoiler fault landing distance that we had enough runway at LGA based on weight and weather conditions to continue to destination. 

While continuing our flight the FO and I determined that somehow we [had] misidentified the spoiler set with the amber xx'S displayed on the flt/cntrl page while accomplishing the crew actions of the MEL card. How we missed this I'm not sure but a couple things come to mind. As English readers and writers we read and write left to right. I also have a lot of time flying the B-727. That aircraft counts spoilers left to right. The spoilers don't show numbers on the flt/cntrl ECAM page in this situation. We trusted that the mechanics had done the work correctly after more than an hour working on the plane, and that what was displayed on the flight control page must be right. For all these reasons we forgot for a moment that A-320 spoilers count 1-5 inboard to outboard. Both pilots in this case have many hours flying the A-320. My thinking is that if this can happen to us it certainly can happen to others, especially a newly experienced A-320 crew.

Synopsis

Despite a prior spoiler set fault having been diagnosed, inop'd and deferred by Maintenance prior to departure the flight crew of an A320 subsequently received the same fault message after departure. Maintenance later determined the incorrect spoiler set had been inop'd.

ACN: 1218726 (1799 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC
Altitude.MSL.Single Value : 11600

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : IVANE 4
Airspace.Class E : ZTL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1218726
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Assigned IVANE4 Arrival South Transition. Several speed changes and heading changes after level-off were issued including a temporary Cleared Direct to MAJIC. After the MAJIC clearance, we were assigned a SW heading and speed reduction to 250 knots. Then clearance to 13,000 FT was issued. Shortly thereafter, we were assigned "300 knots or greater", cleared "direct to HIKRR, cleared IVANE4 arrival-South Transition, resume published speeds at HIKRR." We were anticipating landing on runway 23 therefore the bottom end of the constraint windows are preferred in order to remain on profile. At this point we initiated a slow descent to cross HIKRR at 11,000 FT in order to maintain the 23 arrival profile. Passing 11,600 FT Atlanta Center gave us a frequency change to approach control on 126.15 and stated that we were supposed to be at 13,000 FT. We responded [that we] were cleared direct HIKRR not MAJIC and that our chart indicated HIKRR at or above 11,000 FT. He then told us to switch to Charlotte approach on 126.15. We then contacted approach and asked them if they had an issue with our altitude. They said they did not. 

Initially, I thought the Atlanta Center controller assumed we were going direct to MAJIC which restricts descent to 13,000 FT. After running the situation through my mind after landing, I wondered if the controller expected us to maintain 13,000 FT until we were actually on the arrival (we were approaching HIKRR from the ENE) which would have put us high on a South Transition to RW23. In retrospect, we could have maintained 13,000 FT until reaching HIKRR and then conducted a more aggressive descent for the 23 approach. The fact that I am even writing this indicates the clearance instructions were vague. 

I believe a clearance stating "cleared direct HIKRR, maintain 13,000 FT until HIKRR, at HIKRR cleared IVANE4 arrival South Transition, resume published speeds at HIKRR" would have alleviated any question concerning the arrival.

Synopsis

An A320 flight crew on vectors enroute to CLT was cleared to "...maintain 13,000 FT, maintain 300K or greater cleared direct to HIKRR, cleared the IVANE arrival, south transition, resume published speeds at HIKRR." Then, in preparation for their planned landing on Runway 23, they initiated a slow descent to cross HIKRR "At or above 11,000 FT" charted restriction. When given a frequency change to Approach Center remonstrated them for failing to maintain 13,000 FT.

ACN: 1218357 (1800 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1218357
Human Factors : Physiological - Other

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1218841
Human Factors : Physiological - Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At FL270, started to become aware of a strong noxious odor in the flight deck. Symptoms included headache and stinging eyes. Flight attendants reported the same in the cabin. Landed in ZZZ without incident. Grounded the aircraft and crew was met by paramedics. Cleared by paramedics with all showing good vitals. Released as per Dispatch.

Narrative: 2

During the cruise portion of the Flight, the airplane had a foul, noxious odor. I had noticed my eyes were beginning to burn and I was starting to get a sinus type headache. The captain called me and asked if the plane had a foul smell and I said it did. He asked if the FA's were having any symptoms. I talked with the C and the B and relayed to the captain that all three of us had burning eyes, dull headaches, sinus pain, and ear pain. The captain and first officer were experiencing similar symptoms. As the flight progressed, the symptoms began to worsen and we also began feeling nauseous and dizzy. Before landing, the B fa began having blurred vision. The passengers were also complaining of a foul odor on the plane. One passenger was feeling ill but he declined needing any medical assistance. The flight was moderately turbulent and he was feeling nauseous. It should also be noted, we were only able to serve for approximately 15 minutes before the captain had us discontinue our service and sit down due to turbulence. After landing and deplaning, the paramedics were called to assess the flight crew. Our next flight on that plane was canceled and the crew went to the hotel for the night.

Synopsis

A320 Captain becomes aware of a strong noxious odor on the flight deck at FL270. The flight attendants are called and report the same. The crew elects to continue the short flight to the destination airport.