ACN: 1218299 (1801 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Spoiler System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Minimum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14300
Experience.Flight Crew.Last 90 Days : 211
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 1218299
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were scheduled for a 1015L push from ZZZ for a flight to ZZZ1. When we arrived at the aircraft, Maintenance, three mechanics were already on the aircraft in the cockpit and one up top who they were communicating with a walkie talkie. They informed us that the incoming crew had a spoiler inoperative ECAM and that they were in the process of deactivating the written up spoiler. After a few minutes, two of the three mechanics left the cockpit and just one mechanic stayed behind in the Captain's seat. I asked the mechanic if it was OK if I sat in the co-pilot seat so I can load the box and prep for the flight. As I sat there and loaded the box, I noticed that the mechanics were having a difficult time trying to figure out how to get the spoilers out so they can check and verify that the spoilers they deactivated were actually deactivated. 

They had the electric hydraulic pump switch on, along with the Blue override system on. With both systems on, they lowered the flaps so the flaps were down in the Full position. In this configuration, they were unable to get the spoilers to come out [up] so they can verify that the spoilers that were written up were in fact deactivated. As I sat there and watched them, I did not ask which spoilers they were deactivating and assumed they were deactivating the ones written up. At this point, another two mechanics showed up in the cockpit and the three of them still could not figure out how to get the spoilers out to verify. After several more minutes, they said they had to make a call to a technical expert. After they spoke to their technical expert, they were able to get the spoilers out to verify the spoilers were out [up] and that the deactivated ones indeed showed an Amber X; and were in the locked position. I saw the FLT CNTL page on the lower [ECAM] screen and saw the two Amber X's over a spoiler on each side. They said we were good to go and an Maintenance Release would be on the way. They left the cockpit, the Captain got in his seat and we proceeded with our pre-flight duties. 

After several more minutes, the new Maintenance Release came across the printer. The Maintenance Release said that the previous crew had reported that the # 4 spoiler had an ECAM fault. It also showed that Maintenance deactivated the # 4 spoiler along with the MEL and associated crew actions. Up until this point, I assumed and trusted that Maintenance had deactivated the correct spoiler since there were Amber X's over a spoiler on each side. After engine start, the MEL stated for the crew to confirm that the # 4 spoilers were deactivated, which we thought we did. We pulled up the FLT CNTL page and we saw the two Amber Xs over the two spoilers that Maintenance had deactivated. We took off from ZZZ with no problems. After an hour into the flight, we got an ECAM that said we had a flight control spoiler fault. We pulled up the Flight Control Page and noticed another two Amber X's over a spoiler on each side. After a few minutes of trying to figure out what was going on and kind of odd that another two spoilers would fault, the Captain had Dispatch put us through with Maintenance Control. Maintenance Control informed us that they spoke to ZZZ Maintenance and that Maintenance had locked-out the wrong spoilers. We pulled out the A320 flight manual to look at the spoiler schematic and at this point we realized that Maintenance deactivated the # 2 spoilers on each side and not the # 4 spoilers. Unfortunately the spoilers are not labeled, and when we looked at the spoilers to verify which ones were deactivated as per the MEL after engine start, we thought that the ones we were looking at were indeed the # 4 when they were actually the # 2 spoilers. We checked landing performance as per the charts and landed in ZZZ1 without incident.

Synopsis

First Officer (F/O) reports about the confusion and difficulty Maintenance was having trying to figure out how to get the spoilers up on an A320 aircraft, so they could check and verify that the spoilers they deactivated for an MEL deferral, were actually deactivated. Flight crew realizes in flight that # 2 spoilers were deactivated instead of # 4 spoilers that had a previous ECAM fault.

ACN: 1217932 (1802 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MMSD.Airport
State Reference : FO
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : MMSD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.VOR / VORTAC : SJD
Flight Phase : Initial Approach
Route In Use.Airway : V4

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1217932
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Clearance was ENEKO MARUX V4 SJD. While descending into MMSD, we were on V4 between ENEKO and MARUX, then after MARUX, given clearance to descend to 6000. Approximately 20 miles from the SJD VOR, we were cleared the VOR DME-2 Rwy 16 approach. The FMS was programmed from MARUX to the D11.0 fix, instead of including the the IAF (which is the DME 15.0 fix). The approach plate is confusing, and led us to believe we were cleared to descend to 3600 when we had to wait until the IAF, which was not programmed correctly into the FMS. ATC informed us that we were descending early and below his MVA. He cleared us to level off, and continue on a visual approach. We were in VMC conditions the entire time. The two aircraft arriving before us were given headings to intercept the approach, and the controller was very difficult to understand, and was unintelligible at times. I was the pilot flying and this was also my first time flying into Cabo, as well as my only international flight since I completed training. Those factors combined with unclear approach charts (especially depicting the transition from the encounter structure to approach), as well as the controller stating "cleared for VOR DME-2 Rwy 2 Approach" without saying "VIA MARUX", led us to omit the IAF during our briefing and approach. 

I believe this could have been avoided by having better awareness about confusing which waypoint is the initial approach fix. Something as simple as giving the IAF, D25, and the D11 fixes names would make the charts and radio communications less confusing. Also, communicating to the crews that this approach clearance can be easily misunderstood.

Synopsis

A320 First Officer reported descending early on the VOR-DME-2 Runway 16 approach to MMSD, citing chart confusion and poor ATC communications as contributing.

ACN: 1217879 (1803 of 2908)

Time / Day

Date : 201411
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14300
Experience.Flight Crew.Last 90 Days : 211
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 1217879
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1217785

Person : 3

Reference : 3
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1219409

Person : 4

Reference : 4
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Type : 8185
ASRS Report Number.Accession Number : 1218616
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

When we arrived at the aircraft, 3 mechanics were already on the aircraft in the cockpit and one up top who they were communicating with a walkie-talkie. They informed us that the incoming crew had a spoiler inoperative ECAM and that they were in the process of deactivating the written up spoiler. After a few minutes, 2 of the 3 mechanics left the cockpit and just 1 mechanic stayed behind in the Captains seat. I asked the mechanic if it was ok if I sat in the co-pilot seat so I can load the box and prep for the flight. As I sat there and loaded the box, I noticed that the mechanics were having a difficult time trying to figure out how to get the spoilers out so they can check and verify that the spoilers they deactivated were actually deactivated. They had the electric hydraulic pump switch on along with the blue override system on. With both systems on, they lowered the flaps so the flaps were down in the full position. In this configuration, they were unable to get the spoilers to come out so they can verify that the spoilers that were written up were in fact deactivated. As I sat there and watched them, I did not ask which spoilers they were deactivating and assumed they were deactivating the ones written up. At this point, another 2 mechanics showed up in the cockpit and the 3 of them still could not figure out how to get the spoilers out to verify. After several more minutes, they said they had to make a call to a technical expert. After they spoke to their technical expert, they were able to get the spoilers out to verify the spoilers were out and that the deactivated ones indeed showed an amber X and were in the locked position. I saw the FLT CNTL page on the lower screen and saw the 2 amber Xs over a spoiler on each side. They said we were good to go and an maintenance release would be on the way. They left the cockpit, the Captain got in his seat and we proceeded with our pre-flight duties. After several more minutes, the new maintenance release came across the printer. The maintenance release said that the previous crew had reported that the #4 spoiler had an ECAM fault. It also showed that maintenance deactivated the #4 spoiler along with the MEL and associated crew actions. Up until this point, I assumed and trusted that maintenance had deactivated the correct spoiler since there were amber Xs over a spoiler on each side. After engine start, the MEL stated for the crew to confirm that the #4 spoilers were deactivated, which we thought we did. We pulled up the FLT CNTL page and we saw the 2 amber Xs over the 2 spoilers that maintenance had deactivated. We took off with no problems. After an hour into the flight, we got an ECAM that said we had a flight control spoiler fault. We pulled up the flight control page and noticed another 2 amber X's over a spoiler on each side. After a few minutes of trying to figure out what was going on and kind of odd that another 2 spoilers would fault, the Captain had dispatch put us through with [maintenance control]. [Maintenance control] informed us that they spoke to maintenance and that maintenance had locked out the wrong spoilers. We pulled out the A320 flight manual to look at the spoiler schematic and at this point we realized that maintenance deactivated the #2 spoilers on each side and not the #4 spoilers. Unfortunately the spoilers are not labeled, and when we looked at the spoilers to verify which ones were deactivated as per the MEL after engine start, we thought that the ones we were looking at were indeed the #4 when they were actually the #2. We checked landing performance as per the charts and landed without incident.

Narrative: 2

While accepting turnover from dayshift dispatcher, I was briefed on the conclusion of a phone patch. It was determined a mechanic deactivated the wrong spoiler pair. Pair two and four were the actual inop spoilers. Dayshift Dispatcher advised both Captain and Dispatcher concurred, via Commercial Radio, applicable required landing distances using 320 QRH pages 90.17 - 90.18 yielding required distance result with spoilers inop, well below [arrival airport] active landing runway distance. Flight continued and landed without incident and with NO emergency declared.

Narrative: 3

During preplanning maintenance control called me to defer the 4th spoiler pair. After referencing the MEL I concurred and agreed to the MEL. I did notice it was taking a long time to get the MEL entered so that I could issue a new release. I even called [maintenance control] twice during the process. Finally the MEL was entered and I sent the new release. Everything appeared to be normal until after takeoff. I was then contacted by the Captain to talk with maintenance control. He reported conflicting status messages with the deferral. Spoiler pair 2 status messages rather than Spoiler 4 messages were illuminating. It was then that [maintenance control] realized the mechanic had deactivated the wrong spoiler pair. Crew, maintenance control and dispatch consulted the QRC and confirmed the flight could continue for a normal landing. Field conditions were good and weather was VFR. Because I was at the end of my shift, I thoroughly briefed my relief and departed for 3 days off.

Narrative: 4

Our aircraft came in with a spoiler fault on the #4 spoiler. The mechanics attempted to fix, then decided to defer the spoiler inop. My First Officer and I watched the mechanics (as many as 4 with a lead mechanic) work on this problem for more than an hour. They finished the work and told us that the #4 spoiler was now deferred inop. We re-boarded the aircraft, received a new release from dispatch. After pushback we started both engines, accomplished all checklists including the crew actions from the MEL card. We departed without any problem and climbed to a cruise altitude of FL350. Almost an hour into flight we got a FLT/CNTRL spoiler fault ECAM with spoilers inop 2+4. We went to the spoiler fault procedure in the QRH and ran through the procedure and found that the only concern was runway length since the spoilers showed no deflection and we had no other faults associated with our condition. We called dispatch and got maintenance control and the dispatcher on the radio. We determined that the mechanics had indeed inoped the wrong spoiler set and that spoiler set #4 was not inoped. We now had spoiler set #2 and #4 inop. We worked with dispatch and calculated using two spoiler fault landing distance that we had enough runway based on weight and weather conditions to continue to destination. 

While continuing our flight the FO and I determined that somehow we misidentified the spoiler set with the amber Xs displayed on the FLT/CNTRL page while accomplishing the crew actions of the MEL card. How we missed this I'm not sure but a couple things come to mind. As English readers and writers we read and write left to right. I also have a lot of time flying the b-727. That aircraft counts spoilers left to right. The spoilers don't show numbers on the FLT/CNTRL ECAM page in this situation. We trusted that the mechanics had done the work correctly after more than an hour working on the plane, and that what was displayed on the flight control page must be right. For all these reasons we forgot for a moment that A320 spoilers count 1-5 inboard to outboard. Both pilots in this case have many hours flying the A320. My thinking is that if this can happen to us it certainly can happen to others, especially a newly experienced A320 crew.

Synopsis

A320 flight crew reports being dispatched with the number four spoiler set deactivated by Maintenance to comply with an MEL procedure. With two symmetrical X's appearing over two spoilers on the flight control page the crew assumes the procedure has been done correctly. In cruise another ECAM spoiler fault is annunciated and two additional X's are noted on the flight control page. It is determined that Maintenance deactivated the wrong spoiler pair. Spoilers are not numbered on the flight control page.

ACN: 1216422 (1804 of 2908)

Time / Day

Date : 201409
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Relative Position.Distance.Nautical Miles : 10
Altitude.AGL.Single Value : 3500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 25L
Flight Phase : Initial Approach
Route In Use : Visual Approach
Route In Use.STAR : SEAVU2
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 300
ASRS Report Number.Accession Number : 1216422

Events

Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Ambiguous

Narrative: 1

I was the pilot flying on approach to RWY 25L into LAX. We had been cleared to descend via the SEAVU2, cleared ILS 25L (standard operations for LAX). The weather was VMC. Winds aloft were approximately 12-15 mph with headwind directly down the inbound approach course. At approximately 15 nm on Final Approach Course, ATC advised a B757 would join the approach ahead of us, entering from left base. I recall the B757 was asked to maintain 250 KTS as long as possible and we were instructed to slow down. We reported the B757 in sight. They rolled wings level on final approach approximately 6 nm ahead of us, speed and spacing looked good. Still approximately 6 nm behind the B757, approximately 10 nm from the airport, approximately 3,500 FT AGL, on the LOC and glideslope (GS), we encountered the wake turbulence of the B757. There were no indications that alerted us of the rough air ahead. Our aircraft rocked left and right, then rolled left approximately 45-60 degrees. I recovered the aircraft manually, leveled the wings, and momentarily maintained altitude to fly above the B757's flight path. I briefed the Captain that we would continue the visual approach "One Dot High" on the GS. Other than the momentary roll to the left, the aircraft was stable. We continued without further event flying slightly high on the GS (One Dot) and on the LOC centerline with the Autopilot, Auto-Thrust and Flight Directors all off. Apart from that brief encounter with wake turbulence, the flight and landing were otherwise smooth and uneventful. During deplaning, there were passengers inquiring about the turbulence on final approach. The Captain, while standing in the galley/cockpit area bidding farewell to the passengers, explained the turbulence to the passengers asking about it [as] they exited the aircraft. Opinion: The headwinds on Final Approach were approximately 12-15 mph, and despite our distance behind the B757 being more than adequate (approximately 6nm), I believe the winds were the reason we encountered the wake turbulence at a distance greater than that recommended in the AIM, and despite no other FAA literature supporting that theory. At the average approach speed of an A320 approximately 135kts, at approximately 6nm we were approximately 2m40s behind the B757. More DETAILED Wake Turbulence Study Required.

Synopsis

A320 First Officer reported encountering wake vortex in 6 miles trail of a B757 on approach to LAX that rolled the aircraft up to 60 degrees. Reporter cited wind conditions as probably contributory.

ACN: 1214366 (1805 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.MSL.Single Value : 10200

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : F11
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class E : F11

Aircraft : 2

Reference : Y
Aircraft Operator : Personal
Make Model Name : UAV - Unpiloted Aerial Vehicle
Crew Size.Number Of Crew : 1
Airspace.Class E : F11

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1214366
Human Factors : Training / Qualification
Human Factors : Distraction

Events

Anomaly.Conflict : NMAC
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After takeoff, and just north of the ORL VOR, we were given a turn to 270 degrees and a climb to and maintain 8000 feet. We leveled off at 8000 feet and stayed there for a minute or so. We had to wait until we were clear of [another] aircraft to climb higher.

After we were clear, we were given a clearance to climb to 16000 feet. After passing 10000 feet, I performed my flow. I monitored to FMA, and saw that the climb speed increased from 250 knots to something around 290 knots. As we were accelerating, we hit a patch of rough air. I elected to select 250 knots for a temporary climb speed to climb a little quicker into smoother air.

Just as I pulled the speed selector to command the flight guidance to maintain 250 knots, something caught my eye. Orlando approach requested we switch to Jacksonville Center frequency at the same time. As the First Officer began to read back the instructions, the same thing caught his eye, also. He stopped mid sentence.

What we saw was a small remote controlled red aircraft with white and blue markings. I was fortunate to have pulled for open climb at 250 knots, because if I had let the aircraft accelerate, we may have been on an intersecting flight path. As it was, we only had 2-3 seconds to respond, but did not have to respond aggressively. We passed above the remote aircraft by 100-200 feet.

All of this happened 17-18 northwest of MCO at approximately 10,200-10,500 feet MSL.

We immediately informed Orlando Departure Control of what happened. He took the report, and then re instructed us to contact Jacksonville Center. We did, and the rest of the flight was uneventful.

[I believe] all remote control aircraft owners need to register their aircraft so that a regulation book can be given to the owner of the craft.

The remote controlled vehicle should be subject to the same rules as any other aircraft. If it flies into a B, C, or above 10000 feet in E airspace, it must have a transponder. There should also be a maximum distance and altitude a remote controlled vehicle can fly from its operator. Two miles high is too far for an operator to safely maneuver his craft safely.

And finally, a written exam should be given to any operator of a class of remote controlled aircraft that can fly high enough, or fast enough to come into conflict with a traditional aircraft. If the test is not taken, or if it failed, the operator should be prohibited from flying his craft until the test is taken and passed.

Synopsis

An A320 flight crew had a close encounter with a UAV they identified as a red, white and blue model aircraft operating above 10,000 FT, within the confines of and at or above the MCO Class B.

ACN: 1213919 (1806 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.MSL.Single Value : 5000

Aircraft

Reference : X
ATC / Advisory.TRACON : MCO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : JAGUAR4
Airspace.Class B : MCO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1213919
Human Factors : Fatigue
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I am filling this to be added to the database for this SID if there is one. If I am the only one to make this mistake, well, so be it. I fortunately learned this lesson the easy way and hope no one learns it the hard way. Both the FO and I looked over the JAGUAR 4 SID for an altitude. We both came to the conclusion that 18000' was the altitude. We both missed the small print stating there is a second page to the JAGUAR 4 SID. The second page contains the "MAINTAIN" altitude and the second lesson learned. Two suggestions. Put the two pages on one page. An example is DEN 11-11 chart and with the iPad it is very easy to expand the page. The other is to remove the small print and replace it with standard size print stating "SEE PAGE 2."

Synopsis

A320 Captain reports not noting the fact that the JAGUAR4 Departure from MCO has a second page containing the initial climb and routing information.

ACN: 1213645 (1807 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 31000

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Cruise
Airspace.Class A : ZDC

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 1213645
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Flight was in cruise at FL310. ATC informed us of a rocket launch scheduled to occur at our 2:00 and 15 miles. When the launch attempt occurred (the rocket exploded shortly after liftoff), the aircraft was within approximately 5 miles. It seems that the aircraft was routed much closer to the launch site than safety dictates. ATC apparently has more faith in the rocket launch people than is warranted.

Synopsis

Air Carrier First Officer at FL310 reports being routed a little too close to a rocket launch that explodes just after lift off.

ACN: 1213496 (1808 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Landing

Component : 1

Aircraft Component : Main Gear Tire
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1213496

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1213646
Human Factors : Confusion

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1213643
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Air Traffic Control
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control
Result.Air Traffic Control : Issued Advisory / Alert
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Primary Problem : Ambiguous

Narrative: 1

Rough landing. Plane felt as if we were off runway. Prepared mentally for evacuation. Captain announced for passengers to remain seated. Waited for emergency personnel. Deplaned passengers via air stairs and bussed to terminal. Mechanical failure. Training aided in remaining calm and keeping passengers calm.

Narrative: 2

Conditions at the airport were VFR, the runway was not contaminated, and light winds were present. The captain performed a stable approach and a very smooth touchdown on the runway. A few seconds after touchdown (around when nose wheel was lowered to the ground) the captain said, "We lost brakes, I got nothing." The rollout felt 'rough', almost like centerline lights x 2. The flight attendant more clearly described it as 'driving on a gravel road'. I don't remember if this intensified or dissipated as we slowed. The aircraft started to move to the left of centerline. (Reverse thrust was being used, not sure how much but I believe the Captain was using max reverse). The captain was able to bring the airplane back on centerline. The airplane was brought to a stop as we were exiting onto the taxiway. At that time Tower advised us that they observed sparks on the right side of the aircraft, we communicated to have them roll the trucks. The captain made a PA to have all remain seated. The QRH was reviewed, we discussed and made a plan. 

Highlights of what we thought was important and wanted to place emphasis on while waiting to offload the pax and cargo: Deplane normally. Reassure pax the situation is completely stable. Pin the gear. Communicate with pax, every few minutes on the status. Communicate with flight attendants. Communicate with the dispatcher via acars and keep him in the loop. Keep ops on com 2 the entire time

No fire was observed after the aircraft was brought to a stop by either the tower, or the fire trucks. The brake temps on the left side were in the low to mid 300's, and the brake temps on the right had amber x's, although the fire command did use thermal equipment to observe the temp on the right main. All passengers deplaned out 1L via air stairs and bussed to the terminal.

Two flights prior to the flight mentioned above on my exterior preflight inspection I noticed that the right outboard (#4) tire was extremely low and appeared to almost be deflated. I mentioned this to the captain we both inspected the tire, he then called Maintenance. The mechanics felt it best to change both of the tires on the right side (3 & 4). It took about 45-50 minutes to change the tires. 

At this point, the Captain and I both were ready to go as we had been radioing ops, and they had mentioned everything is done they are having issues with the computer though. One of the mechanics came to the flight deck to reset a circuit breaker that he had pulled before changing the tires, he said "There's a problem with the serial number on one of the tires, the computer is saying its not a new tire, but the tag that is on the tire means its good. We need to sort it out." The number 3, and 4 tires are the ones that burst.

Narrative: 3

Approach was normal and upon a smooth landing and full thrust reverse selected I started to apply manual braking (auto brake was not selected). Aircraft decelerated normally for the first 4 seconds or so and then deceleration decreased while veering towards the left. The aircraft felt different and I could not figure out why. While maintaining directional control and limited braking, I commented to the First Officer that I'm having trouble stopping the airplane and that I thought we lost the brakes. 

The tower stated there were sparks from the right side of the aircraft. I directed the First Officer to apply the loss of braking procedure but that had no effect. The aircraft continued to decelerate at a less than desirable rate and as we approached the end of runway I decided to veer towards a taxiway because I did not want to go off the end of runway. As we approached the taxiway we were now slowing at an acceptable rate and aircraft was veering to the right and also I noticed the aircraft leaning to the right and that is when I realized we had lost both right main gear tires. Finally I brought the aircraft to a complete stop, set the parking brake and made a PA saying to remain seated. First Officer performed the QRH tire failure procedure while I asked the Tower if they saw any fire. Tower said negative, so I made a PA informing the passengers that the airplane is in good shape and that there is no fire (also confirmed by Fire Trucks) and that my plan was to have an airstair attached to the aircraft and have a normal exit through the forward entry door. Also I communicated the plan to the flight attendants. As we waited for busses we were in communication with the Fire trucks and Operations. Mechanics pinned the landing gear and after that was accomplished deplaning commenced. I was the last one off the plane and all crew and passengers were safe.

On the walk around on the previous flight the First Officer noticed the right outer main tire deflated and invited me to take a look. I thought the tire was excessively deflated and as a result we contacted maintenance. Maintenance informed us that this would replace both right main tires while putting one hour delay on the flight. After the tires were installed I thought it would take another 10 minutes to complete the paperwork. After approximately half an hour we inquired with operations why it is taking so long. Maintenance called us on the radio and informed us that they were having computer problems and explained that the serial number on one of the installed tires indicates it was a bad tire but the tag on the tire indicated it was a good tire. Maintenance said they needed time to resolve the conflict. About 15 minutes later maintenance brought us a signed off logbook, and we proceeded to depart. Uneventful flight. The walk around was normal, takeoff and on our way to. There is a possibility that one or both tires were not up to standards or damaged in some way. They looked fairly new to us. Another possibility is that we hit debris on the runway and punctured one or both tires. If indeed a bad tire was installed, then we need a better system of resolving conflicts between tire serial numbers and tags and the way tires are labeled.

Synopsis

A Flight Attendant and the two pilots involved describe a rough landing caused by the failure of both tires on the right main gear, also causing brake failure. The two tires in question had been replaced by maintenance two flights previous to the incident flight (second landing).

ACN: 1212913 (1809 of 2908)

Time / Day

Date : 201410

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1212913

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Staffing
Primary Problem : Staffing

Narrative: 1

While working sector got a message to all dispatchers stating needed to move some Airbus flights to different desks due not having enough qualified A320 dispatchers. Assumed meant maybe would get a few more releases to work up for later in shift. About :15 later an already enroute flight was added to DV for me to flight follow. No briefing was given on its status, just thrown onto my desk. Flight was in air 2 hours without a qualified A320 dispatcher exercising operational control. So to sum up---No operational control for 2 hours---no briefing to myself as assuming operational control of flight just tossed at me---and the biggest issue: Why is [company] not being more diligent about what dispatchers are able to assume what desks with what flights?

Synopsis

Air carrier Dispatcher reported operational control issues related to insufficient staffing.

ACN: 1212782 (1810 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1212782

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On takeoff at approximately 80kt Tower said "Takeoff clearance canceled, vehicle on runway."

I immediately rejected the takeoff. Accomplished rejected takeoff procedure. When the autobrake engaged in RTO the aircraft pulled to the left. I compensated and brought aircraft to a stop. ATC directed us to exit to the right. ATC informed us that "there was a lot of blue smoke" from the aircraft when we rejected. 

Clearing the runway we asked for a spot to stop and made an announcement to the passengers. I also asked if airport operation vehicles that were checking the runway could inspect our wheels. This was done and they said there was no damage. Due to the smoke and the pull to the left by the autobrake system I decided to return to the gate.

Maintenance and dispatch were advised and we taxied to gate.

Synopsis

A320 Captain reported rejecting the takeoff at 80 knots when ATC reported a truck on the runway. During the stop the aircraft pulled to the left and smoke was observed, so they returned to the gate for maintenance attention.

ACN: 1212230 (1811 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 26700
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1212230
Human Factors : Training / Qualification

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Once again Houston is the problem and outsource ramp people not properly trained is an accident waiting to happen. Upon arrival at IAH the copilot saw the ramp person was signaling him to release the brakes. I said no and asked why? After several more requests to release the brakes the copilot opened his side window and asked what the problem was. The ramp people said he parked [us] past the stop point and wanted the "slope of the ramp [to] coast the plane backwards" into the proper spot. I refused. With no guidance, no headset, people in the aisle. He wanted me to possibly pop a wheely. I refused. The ramp guy got a headset. The tow bar was hooked to the plane, the tug was then hooked up to the towbar and we then moved the 6 inches to the proper spot. I also made a PA announcement to the passengers warning of this moment The jetway came up, and no injuries. I then [tried] to find someone to express my displeasure. I would [hate] to have a new captain fall for this. Cause it has been done many times before the ramp guy said!!!!

Synopsis

A320 Captain described ramp personnel encouraging what he felt were dangerous deviations from SOPs at IAH.

ACN: 1209636 (1812 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1209636

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1209824

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight enroute experienced a rapid depressurization event while cruising at FL340. Aircraft was dispatched with System 1 Cabin Pressure Control System on MEL. Cabin climbed rapidly as System 2 failed. We immediately began Emergency Descent Procedure, declared emergency with ARTCC, and descended to 10000 ft MSL. Cabin climbed above 14,000 as we descended, triggering automatic deployment of passenger oxygen masks. We eventually controlled cabin with Manual Pressurization System, received priority handling into ZZZ1, and made normal landing. Arrived at gate with no injuries or reported concerns by any passengers to flight crew upon deplaning.

Normally reliable Automatic Pressure Control System on the Airbus allows one of two systems to be dispatched inoperative. The remaining System 2 in this case failed while cruising at FL340. We immediately began descent, hoping to avoid cabin altitude exceeding 14,000 ft, which would automatically deploy passenger oxygen masks. We were unable to manually close outflow valve in time to avoid this, but eventually controlled cabin manually to make an uneventful, normal landing in ZZZ1.

Narrative: 2

We were doing a ZZZ round trip out of ZZZ1. Picking up the aircraft in ZZZ1, it had a deferred #1 pressure controller. We looked up the MEL and complied with the crew procedure for both legs. On the leg from ZZZ to ZZZ1, we were cruising at FL340. The ride was not good so we requested and were cleared to FL320. After leveling, I (PF) was reviewing the arrival and the Captain (PM) was getting a new ATIS. Just as I notice something different, the captain told me to put on my O2 mask. I notice the cabin alt on the lower ECAM turning red. I put on the mask and the Captain said we needed lower. I rolled the altitude selector down on the FCU and pulled for an open descent. The Captain then took control of the aircraft. I called ATC and said we need lower. ATC initially gave us FL280, then asked how low we needed. I responded 10,000. ATC gave up 12,000, follow shortly by 10,000.
During this time the captain continued the descent. He also selected manual mode on the pressure controller and was closing the outflow valve. I check my QRH for ECAM exceptions and immediate action items for a Press Control 1&2 fail. There were none, so I procedure to do the ECAM. I declared an emergency about half way through the descent. Leveling off at 10,000, we took our masks off. The Captain called the FA's and made an announcement to the passengers. I completed the ECAM, there were no follow up procedures in the QRH. We were able to control the cabin via manual control. After a discussion to make sure we were on the same page, I referenced the expanded ECAM book. We assessed the condition of the passengers through the lead FA. ACARS dispatch, and checked fuel. We elected to continue to destination at 10,000. We landed in ZZZ1 uneventfully. We were met by the FA supervisor and line Maintenance at the gate.

The event occurred because of the failure of the #2 pressure controller with the #1 pressure controller deferred.

While not a suggestion about preventing the event, actions for an emergency descent should be in the QRH like an evacuation. I just completed training on the AB and we only did one in initial. Also, the emergency descent isn't a physical action, like other maneuvers, but a list of items that must be completed. I believe I could have done a better job if I could have flipped over my QRH and followed a step by step checklist. Things were very hectic; understanding the situation, reacting, talking with oxygen masks on. Having a checklist would help calm the situation.

Synopsis

A320 flight crew reported cabin depressurized when #2 cabin pressure controller failed after being dispatched with #1 cabin pressure controller inoperative.

ACN: 1208959 (1813 of 2908)

Time / Day

Date : 201410
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1208959
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Upon arriving to gate area I observed a passenger approx age 65 face down in the boarding area on the floor with her face in a plastic bag regurgitating violently. She was unable to stand and kept falling over. She was sweating profusely, clammy, feverish, disoriented and seemed to be extremely ill. I brought this to the attention to the agents, the cabin crew and flight deck crew. We decided to interview the passenger and her husband traveling with her before boarding. They were evaluated by the agents, myself and a traveling police officer paramedic. I also have a mini paramedic certification from the Ft Lauderdale Fire Dept. It was decided as a collective group that we would also contact Medlink for a 2nd opinion. The Flight Attendants, Agents and Flight Deck were all in agreement. I contacted the Nurse on Duty. I gave him the scenario and symptoms. The age of the passenger in question. It seemed unlikely to be typical alcohol poisoning as often in ZZZ. The husband and wife both stated they had drank and ate the same items. Yet, he was just fine. However, she seemed severely ill due to some unknown reason. She claimed food from a buffet with hopes of just getting on and being allowed to travel so we would not think it was something worse or contagious. She was also holding her chest and looked to have breathing difficulties at one point as well. After the evaluation myself, the police officer paramedic and the Medlink Nurse as well as the entire Flight Crew were a vote of NO on boarding in support of the Medlink Denied Boarding requested. Authorization code # XXXXXX. Yet, the [Customer Service Representative] came onboard and brow beat the Capt and our crew into taking this passenger anyway even with three trained medical opinions requesting a denied boarding to protect the safety of passengers and crew. As well as ensure the integrity of the flight to prevent a potential divert if the passenger became too ill to complete the flight. We agreed to allow the passenger to board only due to being pressured to do so by the ground staff. I feel this sets a very dangerous precedent during a time of Ebola rising in the abroad and now in the US. We are placing revenue above safety and risk to our crew and passengers. If this was in ZZZ1 with not ZZZ routing in the schedule then I would have held my ground on these three medical opinions. Yet, since it was ZZZ I just accepted the ground managers poor judgment decision and hoped for the best. 

We need better and more advanced training for our ground staff. I was not going to make much of an issue of this since luckily for us the flight landed without event. The passenger in question passed out and became unconscious for most of the flight. She only woke up 30 mins before landing seeming completely disoriented and staggering around and then made it back to her seat with no further event. I plan to copy this to OSHA now since I feel that our company is purposely placing our crew and passengers at risk over these unsafe and poor judgment decisions. Due to setting a precedent that it is okay to pressure Flight Attendants to override Medlink and board passengers anyway. Even Though, a Denied Boarding was requested since the medical risk seemed too high to allow a passenger to continue. All simply because the company does not want to lose the revenue and the agent doesn't want to deal with the inconvenience of a denied boarding passenger. This is simply unacceptable and this behavior needs to be addressed by our airline as well as OSHA. This is particularly most disturbing during the beginning of an Ebola Pandemic in our country.

Synopsis

Upon arrival in the boarding area for departure, a Flight Attendant notes one of the passengers being violently ill. The passenger is interviewed by a paramedic on the flight and Medlink is contacted. All agree that the passenger should not be boarded but the Customer Service Representative overrules everyone and brow beats the crew into accepting the ill passenger.

ACN: 1208539 (1814 of 2908)

Time / Day

Date : 201410

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 11000
ASRS Report Number.Accession Number : 1208539
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Arriving at the gate, the marshaller accepted us and directed us to taxi straight, beyond the lead in line. He turned away to talk to someone else while directing us toward the aircraft at the next gate over. Once he realized the aircraft was not where he thought after reestablishing eye contact, he was unable to properly signal a right turn to return us to the line. His signaling included a pre marshaling alignment signal that was prior SOP, and a handoff signal, which presumably could have been a right turn. I signaled him to connect headset to the aircraft, and he said he knew how to marshal an aircraft to the right. I let him know he had put the aircraft in a precarious position and marshaling was required for continued movement, so using an alignment signal wasn't appropriate. He left the aircraft, and either continued to Marshall our plane into the nearby aircraft, or give the signal to turn over marshaling to the next marshaller. Eventually, a ramp manager showed up and informed me the aircraft would be towed in. The actual tow in was not performed in accordance with SOP, and the crew had to be instructed to establish communications with the cockpit. The crew did not follow any tow in procedure, and disconnected communication prior to completion of tow in, resulting in the aircraft cargo doors being opened prior to brake set. The marshaller was a lead, as described on his safety vest, but obviously requires retraining of marshaling and tow in procedures. He placed the aircraft in precarious situations on 2 occasions, and had no ability to correct his actions. I can't safely be marshalled on my perception of 'what I think he wants.'

Synopsis

A320 Captain reports being confused by a Marshall's taxi directions, resulting in the aircraft being towed into the gate. The Captain believes that the Marshall has not been properly trained to perform the new SOP.

ACN: 1207231 (1815 of 2908)

Time / Day

Date : 201409

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Tablet
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1207231

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While at cruise altitude, I opened the company issued iPad and selected the [normal] icon. The iPad displayed the usual [company] logo but did not continue to the menu. After several tries, I shut down the iPad and restarted it. After the iPad re-booted, I again selected the [normal] icon. [The menu] did open but I found that all material was deleted. Under all menu selections, "No Content" was displayed. We reverted to single iPad procedures as per the FOM. All other information on the iPad and other applications are operating normally. I notified the [Duty Manager] via ACARS of the inflight failure of my iPad and that I would need a replacement iPad for my next flight, as per FOM. The Dispatcher responded via ACARS with an authorization to operate inflight with a single iPad.

Synopsis

A320 Captain discovers his iPad database containing charts and other necessary in flight information to be deleted. The flight continues to destination using the remaining iPad, where the faulty iPad is replaced.

ACN: 1206957 (1816 of 2908)

Time / Day

Date : 201409
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9500
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1206957

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1206978

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Passing approx 5000 [feet] we broke out of the clouds. I turned off the Eng Anti-ice. About 1000 feet later we felt the aircraft yaw and got an ECAM for Eng Stall. It cleared itself and then happened again about 10 seconds later. We performed the checklist, declared an emergency, spoke to Dispatch and [maintenance] and the flight attendants. Diverted back to [departure airport] uneventfully.

Narrative: 2

[Report narrative contained no additional information]

Synopsis

A320 flight crew reported they encountered an engine stall and returned to departure airport.

ACN: 1204865 (1817 of 2908)

Time / Day

Date : 201409
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 16000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : GUMBY1
Airspace.Class E : I90

Component

Aircraft Component : FMS/FMC
Problem : Improperly Operated
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1204865
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Completed

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On the GUMBY1 RNAV SID from IAH, Runway 15L, we were assigned a top altitude of 4,000 FT on the PDC. When we contacted Departure they [cleared us to] climb via the SID except maintain 16,000 FT. I read back climb to 16,000 FT and continue via the SID. Departure stated again to climb via the SID except maintain 16,000 FT. By the time we correctly interpreted the clearance we had already removed the restriction to cross DWSUN Intersection at or below 5,000 FT from the FMS. Subsequently, we crossed DWSUN above the charted altitude. Departure handed us off to the center and the flight continued uneventfully. 

During high task loading and during a RNAV departure we should have queried ATC about any doubt or confusion of a clearance before entering a change into the FMS or selecting a new altitude.

Callback: 1

Reporter advised that, being based on the East Coast, this had been his first departure from IAH utilizing the newly formatted SIDs and that he wasn't, at the time, completely conversant with the specific implications of the "climb via" phraseology. His Captain on the flight had only recently returned from flying primarily international operations and was in a similar situation; thus the delay in proper programming of the climb via clearance.

He responded affirmatively to the analyst's question as to the presence of the phrase "...top altitude 4,000..." in the PDC. In response to a follow up question he stated that the phrase "climb via (the assigned SID)" was also included although he couldn't remember the exact sequence or how the two restrictive phrases had been juxtaposed when transmitted. 

Reporter followed up these responses by addressing his displeasure for the lack of a common format for PDC clearances and the fact that they often included information not germane to their actual cleared route. He also commented that their intent is often difficult to derive due to "coding" and/or the inclusion of filed routes that are eclipsed by actual cleared routes given in a separate section of the clearance.

Synopsis

When the flight crew of an A320 misinterpreted ATC's "climb via except maintain 16,000 FT" clearance they had already exceeded 5,000 FT prior to passing DWSUN which must be crossed at or below 5,000 FT.

ACN: 1204337 (1818 of 2908)

Time / Day

Date : 201409
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZFW.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : BRDJE1
Airspace.Class E : ZFW

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23600
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1204337
Human Factors : Fatigue
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Our flight started out with a medical issue before we even reached our cruise altitude. A commuting flight attendant was experiencing severe stomach pain. We had a doctor and a paramedic on board. We also contacted dispatch and Medlink. To make a very long story short this became a very, very busy flight. It was decided to continue on to DFW, but there was much drama including our commuting flight attendant accusing one of our flight attendants of not following protocol which was totally incorrect and unfounded. We had numerous maintenance log items because of all the medical equipment used in the back. During descent we had a couple additional calls on the status of things in the back and a request for additional information from dispatch. These contributed to the issue I am about to describe but even without them I think we probably would have flown the arrival we flew. Our PDC and flight plan had us on the BRDJE1 arrival in DFW which is the SEEVR.BRDJE1 arrival on the chart. 

During descent we were told to maintain 310 knots. Later we were vectored off of the arrival and were then given direct SEEVR and if I remember correctly cleared the SEEVR arrival. I looked at our arrival and saw SEEVR so we continued on the arrival (SEEVR.BRDJE1). I am not quite sure exactly where on the arrival (I am guessing it was not long after SEEVR) but approach control asked us what arrival we were on. We said the SEEVR/BRDJE1. They told us that was the wrong arrival and to turn right to heading 270 (I believe). We asked what arrival should we have been on and the controller said the SEEVR. At this point I was quite confused because that is what I thought we were on. The controller said, "no big deal, that we were not in trouble, and this was a learning curve for everyone." I still was confused. 

After we landed we then tried to figure out where we went wrong. I had read the NOTAM's and the only NOTAM for this arrival was about approach frequencies. I started digging through all of the arrival plates (of which there are very many) and finally to my amazement discovered that there was a SEEVR.SEEVR1 arrival. When I had looked at the BRDJE arrival during the flight I had noticed that there wasn't a designation as to what runways the arrival was attached (just says "RUNWAY TRANSITION not to scale"). As I now look at the plate again (after a good night sleep) there is a note I missed which would have indicated we were on the wrong arrival. Note number 5 does state the arrival is for north flow and to expect the SEEVR arrival for south flow. With everything going on and being tired from the long day I missed note number 5. Somehow I still think the way these two arrivals are set up the potential for this happening again is great. There really should be a complete separation of the arrival names. Two arrivals should not have SEEVR (SEEVR.BRDJE1) designated on them. It is fine for someone sitting at a desk and well rested to figure this out, but in the heat of battle with many distractions and being tired the likelihood of an error goes up significantly.

Synopsis

A320 Captain describes a track deviation that ensues when ATC clears the flight direct to SEEVR and changes the arrival from the BRDJE 1 RNAV to the SEEVR 1 RNAV approaching DFW. Without being aware of the SEEVR 1 the crew believes that the BRDJE 1 is correct since SEEVR is part of it.

ACN: 1201665 (1819 of 2908)

Time / Day

Date : 201409
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : IAH
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 16202
Experience.Flight Crew.Last 90 Days : 224
Experience.Flight Crew.Type : 3422
ASRS Report Number.Accession Number : 1201665

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1201668

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Ramp had us stop on the RA bridge. The ramp controller did not understand the safety hazard of a potential evacuation on the bridge.

Narrative: 2

We were given clearance to taxi RA spot 1, monitor West Ramp. As we were taxiing across bridge we noticed another aircraft stopped just over the end of the bridge. We did not want to stop on bridge in case we would have to emergency evacuate the aircraft. We told the West Ramp controller, we would like to move as the aircraft in front of us moved and he would not give us clearance. We advised him we were moving just over the bridge and we would then stop. Our aircraft was never in any danger or close proximity of another aircraft while taxiing, but I wanted to write this to see if there is some other procedure we need to be following while in IAH. We were taught to never stop on the bridges in case of evacuation. This does not seem to be the case in IAH, I just want some clarification of this as for our SOP.

Synopsis

Air Carrier flight crew believes that stopping on a taxiway bridge is not safe due to the inability to evacuate should the need arise. The Ramp Controller does not seem to understand this logic.

ACN: 1199064 (1820 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : IAH
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : ACARS Printer
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 1199064
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Flight Attendant
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1199324

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Pre-flight

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Our ACARS was deferred [with the] clock and printer inoperative but able to use any functions that worked. During set up ACARS and the printer operated normally. We briefed the departure (including the maintenance release status) approximately 20 minutes before schedule departure. A few minutes later we received an ACARS message that a new maintenance release was required as well as a status message on the printer advising the aircraft was ready for release. 

Before having an opportunity to address the maintenance status the lead flight attendant advised she was concerned about the location of an armed passenger and the status of a dog on board and advised another attendant had had a verbal conflict with a gate agent regarding overhead bin space and was asking for our intervention. 

Finally after dealing with these issues we pushed back and immediately realized we hadn't called maintenance about the unusual maintenance release status. Maintenance attempted to send the release to the printer but were unable. They then sent a truck out with a hard copy passed in through a cockpit window. We, then, started engines and continued the flight uneventfully.

Narrative: 2

[Report narrative contained no pertinent additional information.]

Synopsis

Broken and deferred communications functions of ACARS combined with conflicts between flight attendants and passenger agents created a chaotic pre-departure scene which culminated with the flight crew's failure to obtain a hard copy maintenance release prior to gate departure.

ACN: 1198821 (1821 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Thrust Reverser Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1198821
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Aircraft : Automation Overrode Flight Crew

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

During cruise flight had system status display inoperative systems REVERSER 1 AND ENG 1 THR. WITH A DESCRIPTION STATING WHEN SLATS out, engine 1 at idle, on ground Engine 1 maximum power: idle. After discussing options and original destination [we] declared [an] emergency and diverted.

Upon slat selection engine did not reduce to idle thrust as expected uneventful landing. We suspect an unknown possible autothrust lever TLA fault.

Synopsis

An A320 ECAM status alerted REVERSER 1 AND ENG 1 THR along with a note that with slat extension engine 1 thrust would idle and on ground max thrust available, idle. An emergency was declared, the flight diverted and crew suspected a thrust lever angle (TLA) fault.

ACN: 1197804 (1822 of 2908)

Time / Day

Date : 201408

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Hydraulic Lines, Connectors, Fittings
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Aircraft Documentation
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1197804
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were number three for takeoff on RWY XXL when we received an urgent ACARS message and call from the Tower to return to the gate. We received clearance from Ground and headed back to Gate-XX for what we confirmed to be a Maintenance landing gear issue. After blocking in, I called Station Maintenance to get clarification and they had very little information. I then called the Dispatcher to get a connection with Maintenance Control. The Maintenance Controller was relatively brief in his explanation but it turned out to be a Mechanic that was concerned he had not fully completed work on a Main Landing Gear (MLG) rebuild that had been done a week and a half earlier in ZZZ2. He was concerned if "all the parts were in place," according to Maintenance Control. I then coordinated with Ramp Operations, ZZZ Maintenance, and Customer Service (C/S) on the issue and waited for the mechanics to complete their inspection (estimated time 15-30 minutes). They completed the work in approximately 25-30 minutes from blocking back in and the Lead Mechanic briefed us personally in the flight deck that all looked good, everything was in place, and the aircraft was good to go. He found that all the parts in question were in place and he would sign it off and send a new Maintenance Release. 

I then called to inform Ramp Operations whereby she informed me they had already swapped the aircraft for an A319 (from an A320) at Gate XY, and that the passengers were to be deplaned and moved. It took me no less than ten more minutes to debate with her and talk to her Supervisor explaining the absolute outrageousness of that decision since now our aircraft was signed-off and we were ready to close the door! After talking to the Supervisor, he agreed and we quickly closed the doors, received a new Maintenance Release and pushed back to head to ZZZ1. We departed with Flight Plan XYZ04 (after several aircraft changes, finally back to the original A320 aircraft) and Maintenance Release ZZZ August 2014. This entire incident is beyond baffling, and while I can certainly see that things can be missed, what appalls me more than anything is the fact that this aircraft flew for at LEAST one and a half weeks before this potential error was addressed. Thankfully the mechanics in ZZZ found everything in order, but what if they didn't? What about all the subsequent flights since the gear change in ZZZ2? This kind of thing cannot happen anywhere; least of all at our air carrier. This occurrence was absolutely unacceptable and should be researched to the fullest extent.

Synopsis

While in line for takeoff, an air carrier flight crew received an urgent message to return to gate. They were informed that a Mechanic was concerned he had not fully completed rebuild work approximately a week and a half earlier on their A320 Main Landing Gear (MLG). Captain very concerned about the number of flights flown and length of time before the maintenance error was addressed.

ACN: 1197138 (1823 of 2908)

Time / Day

Date : 201408

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 20000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
ATC / Advisory.Tower : ZZZ
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Visual Approach

Component

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1197138

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent the left engine oil quantity showed six quarts and decreasing as we progressed toward landing. A deadheading pilot confirmed smoke was trailing from the engine with the throttle at idle. We requested a straight in approach. We briefed the flight attendants and flew a visual approach with the left engine at idle. Upon exiting the runway we shut down the engine, CFR confirmed the engine was secure, and we taxied to the gate.

Synopsis

An A320 flight crew requested assistance and landed straight in at their destination when they determined the left engine was losing oil quantity.

ACN: 1196698 (1824 of 2908)

Time / Day

Date : 201408

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Strobe Light
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1196698
Human Factors : Human-Machine Interface
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The strobes on this aircraft are still flashing inside the cockpit when it gets dark out. In addition they are so bright that when the pilot looks out to scan for traffic they blind you, forcing you to turn them off. Just when I'm sure that the Airbus Corporation and their product couldn't set the bar lower, they do with their apathetic attitude to this clear safety issue. I have personally submitted a couple of safety reports on this and nothing has happened. This situation causes potential pilot vertigo. It destroys night vision. It forces me to turn off a lighting system, thereby making my aircraft less visible. And Airbus, with their arrogance, apparently could not care or less. I absolutely cannot believe that they are not being required to fix this. Unbelievable. 

Airbus [should] man-up, drop the arrogance, and fix their awful designs.

Synopsis

An A320 pilot reported the aircraft's strobe light system is poorly designed and located and constitutes a safety hazard by compromising flight crew vision.

ACN: 1195963 (1825 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 255
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1195963
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

We got high on the slide slope and started a descent to recapture it and had a high sink rate which gave us TERRAIN WARNING. At that point we did a go-around and got vectors to another approach to a landing.

Synopsis

An A320 Captain executed an IMC go-around in response to an EGPWS TERRAIN warning after he developed a high sink rate to capture the ILS glideslope from above.

ACN: 1195507 (1826 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : APU Electrical
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1195507
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

ECAM Gen Fault Number 2 after start. [We] followed ECAM procedure for fault. Problem was not solved, contacted Maintenance Control. [We] returned to the gate per Maintenance Control for MEL, since not deferrable by crew. MEL 24-20-01B required running the APU for duration of the flight. Departure was normal. During cruise FL320, ECAM APU Fault appeared. [We] ran the ECAM per company procedures. Tried to restart the APU, started fine, APU generator would not accept generator load, then ECAM APU Fault illuminated. [We] started a descent for turbulence, cool starter motor and possible diversion. Level at FL300 repeat the previous steps with no success. Per APU starter limitation, tired one last start of the APU at FL280, no success. Communicated a plan with ATC, First Officer, A Flight Attendant and Dispatch for a diversion since aircraft was down to one generator. [We] advised the passengers of diversion and precautionary landing at a diversion airport. Performed the Overweight Landing QRH procedures, configured the aircraft early to burn fuel. [We] landed at 151,800 LBS at less than 300 FPM, with no issues. [We] made a logbook entry for overweight landing.

MEL issued to aircraft, APU failure in flight, aircraft had overnight check before this event.

Synopsis

An A320 engine generator faulted after start and was MEL'ed. At cruise the APU faulted and would not restart so crew diverted to a nearby airport.

ACN: 1194822 (1827 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 2300

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : ORD

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14150
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 2870
ASRS Report Number.Accession Number : 1194822

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1194822

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Airport
Primary Problem : Ambiguous

Narrative: 1

On vectors to intercept final on visual approach to Runway 10C ORD. Approach gave descent to 4,000 FT and assigned heading 360. Called field in sight, were assigned heading 070 and given clearance to 3,000 FT and cleared for visual approach Runway 10C. Heading had us intercept final inside of RAYYY. Upon contacting Tower, [we were] given clearance to land behind E170. Inside RAYYY, showing 1 to 1/2 dot high on glideslope, Tower called issuing low altitude alert. Confirmed we were above glideslope (1 to .5 dot high), within intercept criteria. Continued approach to uneventful landing.

Narrative: 2

On arrival into Chicago received a low altitude alert from ATC (ORD Tower) on arrival to Runway 10C. We were given a heading to join final approach course and cleared for the visual approach. We were above glideslope profile slightly and descending while intercepting the final approach course. We received a low altitude warning from ATC. Our cross check verified we were on profile and we continued to normal landing.

Synopsis

A320 flight crew reports receiving a low altitude alert from the Tower while on a visual approach to Runway 10C at ORD. The crew was above the glideslope intercepting final inside RAYYY when the alert was issued and continued to a normal landing.

ACN: 1194296 (1828 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : L30.TRACON
State Reference : NV
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Route In Use.STAR : GRNPA 1
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1194296
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

LAS GRNPA 1 STAR, groundtrack cannot safely transition to RNAV 19L. We asked for direct to two fixes HAMIG or JOGNU on RNAV 19L but ATC said unable, follow GRNPA 1 groundtrack, cleared visual 19L. MVA is 5200 feet and STAR shows 6100 feet MEA to LEMNZ. Airport elevation is 2100 feet but you cannot lose 4000 feet in 3 miles, unsafe. My third visual with exact same problem resulting in potentially unstable approaches.

Synopsis

A320 Captain reports being cleared for the visual approach to Runway 19L via the GRNPA 1 to LEMNZ which is less than 3 NM from the runway end. STAR shows a 6100 foot MEA to LEMNZ, resulting in an unexpected close in turn to final with potential for an unstabilized approach.

ACN: 1194286 (1829 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZSE.ARTCC
State Reference : WA
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZSE
ATC / Advisory.TRACON : P80
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : HHOOD 2
Airspace.Class A : ZSE
Airspace.Class E : P80

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1194286
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

The Airbus FMGC, had it been followed during the HHOOD 2 RNAV arrival into PDX, would have descended the aircraft below the published MEA. The FMGC was checked prior to descent. With the proper 280 knot descent programmed, the FMGC would have descended the aircraft below the published MEA of 7200 feet between HHOOD and BLRUN. It is a continuing problem with the programming of the Airbus FMGC, it is trying to set up to handle a "TOO STEEP PATH" alert after SHAFR, and thinks the aircraft needs to be at 7090 feet at BLRUN in order to meet the later requirements. This has not been a formal ATC issue because PDX approach normally starts vectoring us off the arrival and assigning a Minimum Radar Vector Altitude (which is different than an MEA) prior to reaching BLRUN. However...if we had descended via the RNAV arrival all the way to SHAFR and beyond, we would have been below the MEA. This is a continuing Airbus problem that needs to be addressed by the fleet and the FAA. It is a potential violation or accident waiting to happen.

Callback: 1

Reporter states that a restriction must be manually entered at BLRUN to prevent the aircraft from descending below the MEA. He believes the MVA at BLRUN is 7000 feet.

Synopsis

A320 First Officer reports that when the HHOOD 2 RNAV arrival to PDX landing Runways 28L and R is entered in the FMGC, the MEA of 7200 feet between HHOOD and BLRUN is not honored. Manual entries to the FMGC are required to keep the aircraft above 7200 feet until after BLRUN.

ACN: 1194149 (1830 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : Flunk Three
Airspace.Class E : NCT

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : NCT
Make Model Name : Cessna 425/441 Conquest I/Conquest II
Crew Size.Number Of Crew : 2
Flight Plan : VFR
Flight Phase : Climb
Airspace.Class E : NCT

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 125
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1194149

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1194171

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Miss Distance.Vertical : 200
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

[We were] descending to 10,000 FT approaching TENCO. NORCAL called out climbing traffic to our north--then gave us two vector headings in quick succession. We then got a TA at approximately 10,200 FT followed by a descend RA--followed immediately by a climb RA. I disconnected the autopilot and flew the escape maneuver. At about 10,400 FT we got an "adjust vertical speed" RA followed quickly at about 10,500 FT by the "clear of conflict" advisory. I never saw the traffic but believe I saw an altitude difference of -200 FT on the ND.

Narrative: 2

[We were] filed on the Flunk Three Arrival into SMF, cleared to descend to 10,000 and direct to TENCO....Upon landing I talked with a supervisor at NorCal who told me the traffic was a VFR Cessna Conquest climbing out of Grass Valley who happened to request flight following. I told her my concern that we had so little warning and separation. She said the Conquest was operating within its rights flying through an area where IFR traffic was descending into SMF and it could happen again anytime.

Synopsis

An A320 flight crew descending on the FLUNK Arrival to SMF suffered a TA\RA conflict with a VFR Cessna Conquest in the process of requesting flight following from ATC during its climbout.

ACN: 1193656 (1831 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1193656
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On takeoff at rotation speed (after decision speed V1) it took a very extreme stick movement to get the aircraft to rotate. This also consumed approximately 1,000 extra feet of runway. After lift off, the aircraft trimmed a large amount from the original setting. I assumed that this was due to misloading since we set the proper trim. After 25 minutes from pushback (in flight) we received a message from Operations that the Weight and Balance (W/B) was off by 40 bags in the front bin. Had this happened at a short runway airport this might have resulted in an accident! Why did it take that long for Operations to communicate the problem? Do these out stations have any idea how to get hold of the aircraft during taxi (call the Tower)? Does the training for the outsourced stations express how dangerous a W/B mistake can be? Why did it take so long for them to admit the mistake?

Synopsis

A320 Captain used an abnormal amount of aft stick to get the aircraft to rotate during takeoff. Once airborne it is determined that the aircraft has auto trimmed to a substantially different setting from the takeoff setting. A message is received from the company indicating that 40 bags were loaded in the forward bin but not accounted for in the weight and balance.

ACN: 1193652 (1832 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Dusk

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Flap/Slat Control System
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1193652
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Aircraft

Narrative: 1

While on approach to Runway XX at ZZZ, flap handle broke and stuck at [Position]-2. Initiated go-around to reference paper procedure and select longer runway. We [advised ATC] at this time, and requested fire trucks. Normal landing at Flaps-2 without excessive brake temperatures. Airline Transport Pilot.

Callback: 1

Reporter stated this was a situation where the aircraft doesn't know it was broken. A non-ECAM Abnormal situation and no QRH reference procedure. All systems were functioning on the A320, except that he could not move the flap handle from Position-2 (14-degrees Flaps). A spring loaded Release Collar under the Flap Handle knob must be pulled out (pulled-up) out of detent, to allow the Flap lever to move past the balks (gates) on the Center Pedestal. But, the Release Collar was flopping around and could not be pulled up, preventing flap lever movement. Flaps were stuck at Position-2. Alternate Electric Flaps are not available on the A320.

Synopsis

A Captain reports about a Release Collar under the Flap Handle knob of an A320 aircraft that broke while on approach. The spring loaded Release Collar was flopping around and would not lift out of the flap detent to allow the flap lever to be moved. Flaps were stuck at Position-2 (14-degrees Flaps). Go-around initiated and aircraft landed in emergency condition.

ACN: 1193107 (1833 of 2908)

Time / Day

Date : 201408
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Door
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
ASRS Report Number.Accession Number : 1193107
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1193654

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

[Shortly before top of descent] the First Officer called the Flight Attendant [to come forward so he could] go back and use the restroom. The A Flight Attendant came up and relieved the First Officer. When the First Officer was ready to return he knocked on the flight deck door. The A Flight Attendant looked through the peephole and attempted to open the cockpit door but was unable as it had become jammed somehow. [While] she attempted multiple times to pull the door open, the First Officer called and I advised him of the situation. 

While this was going on Center gave us a clearance to cross the intersection at FL330. We were currently at FL370 and the intersection was approximately 15 miles ahead of us. The First Officer and I remained in communication trying to resolve the stuck door and I was advising the Flight Attendant on what to do to attempt to free the door. I moved the cockpit door switch from normal to unlock numerous times although no faults were showing but this didn't help. Around this time Center called us. I had missed the descent point with the door jam distraction and they advised us to maintain FL370. 

I advised the Flight Attendant through coordination with the First Officer to unlatch the cockpit door escape panel and push it out to allow the First Officer to climb thru and back into the cockpit. The Flight Attendant was able to do so and the First Officer returned to the cockpit thru the panel. He then was able to forcibly open the cockpit door. The Flight Attendant exited the cockpit and the First Officer returned the panel to its normal position and returned the pins. He then resumed pilot not flying duties. 

I informed Center of what had happened which led to the distraction and missed crossing restriction. Nothing further was said from ATC. The flight was completed uneventfully, I entered a write up in the logbook and Maintenance was called out to fix the cockpit door. Maintenance found that all but one screw were missing from the door frame causing it to jam the door. Repairs were made and the logbook signed off, returning the aircraft to service. 

This was a rare occurrence and a significant distraction in the flight deck. In [retrospect], I should have included ATC in what was happening on the flight deck and requested to remain at altitude.

Narrative: 2

After a several minutes of effort by the Flight Attendant, the cockpit escape hatch popped out of the door and onto the galley floor. I immediately crawled through the hatch, stood up on the flight deck, grabbed the flight deck door handle, turned it, and yanked on it with substantial force! It flexed and opened with these efforts. With the door now open, the Flight Attendant immediately exited the flight deck and I closed the door behind her. I then knelt down and called to the Flight Attendant in the galley to replace the escape hatch and apply pressure to it while I re-secured the pins. I secured the escape hatch and returned to my duty station.

In my opinion, the crew handled this event appropriately and with professionalism. This very unusual situation created unusually intense risks and distractions. Any malfunction of the cockpit door is a distraction because such malfunction creates a grave threat to security of the flight deck. In this situation, the threat to security of the flight deck was compounded by an actual, current risk to flight safety in the form of a pilot locked out of the flight deck.

Synopsis

When the First Officer of an A320 returned to the flight deck after a break the Flight Attendant was unable to open the door to allow him to return. During the extended time required to find a way for the pilot to gain admittance the Captain failed to comply with a descent clearance due to distractions directing potential solution suggestions to the Flight Attendant.

ACN: 1191381 (1834 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PHL.Airport
State Reference : PA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1191381
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After pushback from gate and after engine start and released for taxi-out by ground crew I started the taxi-out. As I approached the outer roadway a pickup truck rapidly approaching did not stop for the aircraft or the road STOP SIGN, but rather sped-up trying to outrun the aircraft. I promptly stopped the aircraft. The pick up truck, I guess realizing that a collision was imminent if the driver continued, abruptly stopped and approximately 20 FT from the aircraft. Then backed up on the roadway to the STOP sign area. Not yielding to the aircraft. Driving in an unsafe manner. Promote safe driving techniques. Reinforce the rule that ground vehicles yield to aircraft, that aircraft always have the right of way.

Synopsis

A320 Captain reports a ground conflict with a pickup truck that fails to yield as the aircraft is taxied out for departure.

ACN: 1190561 (1835 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Fan Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1190561
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Descending thru FL180, checking Status Page on Descent flow we observed:
"ENG2 at Idle on GRD"
"ENG2 Max Pwr Idle"

INOP SYS:
"REVERSER 2"
"ENG2 THR"

[There was, however, no ECAM message; No Immediate Action; No ECAM Exception; No QRH Procedure or guidance in the QRH

I asked a jumpseating Captain to see if there was something in the ECAM Supplemental Guide for supportive information. [Although I understand the] ECAM Supplemental is for actual ECAM and Crew Awareness [and that] we never received an ECAM [however it appeared] we had a possible situation with the Number 2 Reverser. The guide did provide some insight into what COULD be wrong. We discerned that a probable reason is there could be a disagreement relating to the FADEC and Thrust Lever Position. Nevertheless, we figured this had to with Reverser Door position with the STS/Inop SYS messages.

The ECAM Supplemental Guide under "ENG REVERSER PRESSURIZED" explains...
"Reverse Thrust System Pressurized Without Reverse Deployment order"

ACTION: "THR LVR 2..... Idle" 
"If flight conditions permit, reduce thrust lever of affected engine to IDLE as a precautionary measure".

What we didn't know was if the reverser was pressurized or not. 
What we DID know [is] if the reverser sleeve/door opened it would not be good.

Landing distance [decrement was] quickly checked and verified with 25% increase. 

We were in a position to land [while] on base leg when I made the decision to bring the Thrust Lever to IDLE as a PRECAUTIONARY measure. 

We briefed and used BOTH reversers on touchdown. Landing and rollout were normal with BOTH thrust reversers operating. STS messages and INOP SYS cleared.

After shutdown, a Maintenance Message: ENG2 FADEC [was displayed]
[An MEL:] "Reverser Inop" was added for next flight.

Synopsis

Odd Status page messages--observed during completion of the Descent Checklist--regarding the right engine reverser but which had not triggered an ECAM message/resolution presented a quandary to the flight crew of an A320.

ACN: 1189498 (1836 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1189498
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Deplaning
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1189892
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : Taxi
Result.General : Evacuated
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We pushed back, started both engines. We were approximately Number 5, first flight of the day, shut down APU after engine start, taxied to Spot 2. We got a call from "A" Flight Attendant reporting fumes/odor (dirty socks) in the cabin. With parking brake set, called Dispatch/Maintenance. Maintenance advised to go back to the gate. Asked Ground to return to gate, was approximately Number 4 for takeoff, turned south on Taxiway Y. Got another call from the "A" Flight Attendant expressing increase odor and fumes, asked Ground to expedite return to gate, they did. We returned to the gate, customers deplaned, no injuries during deplaning, customers left all belongings on aircraft, medics met the aircraft, took crew's vitals, went to the hospital for further evaluation, tested for TCP's and carbon monoxide. 

Possible pyrolization of aircraft oil or hydraulic fluid.

Narrative: 2

Our flight departed the gate and began taxi-out for departure. After completing the safety demo, and conducting necessary cabin checks, I strapped into my jumpseat located at the 1L door. Upon taxing out for departure, I began smelling a very strong odor that smelled similar to dirty, sweaty feet or socks. The smell continued to grow stronger, at which point I looked over to the "C" Flight Attendant and asked her if she could smell anything, she could. Neither one of us could determine where the odor was coming from. 

I immediately picked up the interphone and notified the Captain, that we detected a strong odor in the galley/cabin. The Captain asked me to keep him informed and give him an update every minute. I called the "B" Flight Attendant, and asked him if he and the "D" Flight Attendant could smell any strong odors. Both flight attendants confirmed they smelled something unusual. I called the Captain again to give him an update, and he confirmed he heard the conversation I had with the "B" Flight Attendant over the interphone. The Captain advised me that we were going to return to the gate. 

After giving the Captain the update, I left my jumpseat to walk through the cabin to determine if the strong odor was also in the entire aircraft. While walking through the main cabin area, I determined that the odor was also present throughout the entire passenger cabin. Upon reaching the emergency exit row, I met a non-rev Flight Attendant and knelt down next to him to quietly advise him that due to an odor in the cabin we would be returning to the gate. After speaking with him, I walked to the aft galley to advise the "B" and "D" Flight Attendants that we would be returning to the gate. It should be noted that the odor was most noticeable and stronger in the AFT and FWD most areas of the aircraft. While walking to the front of the aircraft to return to my jumpseat, I noticed that numerous passengers were covering their nose and mouth, and that the odor began to grow stronger. 

As I returned to my jumpseat, I began to feel very light headed and dizzy. As a result, I began to breathe through my uniform sleeve in an effort to reduce the amount of fumes/odor being inhaled. I also noticed that some of the first class passengers were beginning to look around and ask what the smell was, and what was going on. As the odor grew stronger, I picked up the interphone to give the Captain another update. The Captain asked me to hold on the interphone, which I did for approximately 30 seconds. When the Captain returned to speak with me, I advised him that we needed to expedite to the gate as the odor was becoming stronger. The Captain advised me that we were quickly returning to the gate and would be evacuating the passengers off the airplane via the front of the aircraft, and instructed me to have all passengers leave their belongings, taking nothing with them.

After speaking with the Captain, I made a PA announcement to passengers to provide information about our return to the gate. My PA announcement included that we were returning to the gate due to the odor that was accumulating in the aircraft. I advised the passengers that we would be doing a controlled evacuation through the front of the aircraft and advised them to leave personal items, keep all overhead bins closed, and to remain seated, with seatbelts fastened, until we arrived at the gate, and the Captain turns off the seatbelt sign. The announcement was immediately followed up by the "D" Flight Attendant, in Spanish. 

Upon arriving at the gate, I got on the PA and reminded the flight attendants to "Monitor and Challenge" and to "verify and disarm doors." The 1L and 1R doors were properly disarmed via team monitor and challenge. Once the 1L door was opened, the "C" Flight Attendant and I directed passengers out of the aircraft via the 1L door. Each passenger was closely monitored and reminded to watch their step as they exited the aircraft. It was noticed that passengers were covering their nose and mouths as they left the aircraft. After a few passengers left the aircraft, the Captain exited the flight deck and utilized the FWD PA to provide a follow up announcement ordering passengers to exit the aircraft and not to take any personal belongings.

Due to growing concerns of inhalation complications with passengers, and the light headedness I was experiencing, I asked the First Officer, who was still seated in the cockpit, to have medics respond to the aircraft. He confirmed he was making the request. Note: I did not observe any passengers running, pushing, or acting in a panic state. The evacuation was orderly. 

Following the Captain's announcement, I got on the PA and reminded the flight crew that there were three (3) unaccompanied minors (UM) on board. After nearly all passengers exited the aircraft, I met one of the UMs and asked him to remain with me. I then reminded the flight crew via the PA to check all bathrooms and to exit the aircraft. After confirming with the crew the aircraft was clear of all passengers, I deplaned and walked the UM to the top of the jetway. At the top of the jetway, I met one of the gate agents (name unknown), and handed over the UM that I escorted up the jetway. I advised her that there were two other UMs onboard, but I was not sure where they went during the evacuation. I began looking around the boarding area, where most passengers went after exiting the aircraft, and located the other two UMs who was seated with other adults. All three UM's were then accounted for and in the custody of the customer service gate agents. 

After accounting for the three UM's, I returned to the top of the jetway, in the glass enclosure outside of the boarding area, and met with medics to request having my vitals checked. I advised the medics that I was feeling light-headed and was checked accordingly. One of the representatives, possibly a Customer Service Supervisor, asked us if we had our "green card" or "odor event" card on our person to provide direction on how to handle the event. This representative said she had twenty-years previous experience as a Flight Attendant with the Company and attempted to give us direction; however, I was not aware of any such card, nor were any of the other flight attendants at the scene. 

While having my vitals checked with the medics, I observed an influx of several company representatives, none of whom were introduced to me personally, with the exception of [the] In-flight Supervisor. She asked if I was continuing on the trip, to which replied which I wanted to be removed. I explained that I was experiencing concerning symptoms (which included elevated and abnormally high blood pressure as told to me by medics), which I felt it unsafe for me to continue on to another flight. 

Note: The company representatives handling this event, particularly those assigned to the flight (i.e. gate agents, customer service supervisors), appeared to lack full understanding of the seriousness of the situation and were quick to state that another aircraft was available for us to continue onto the next flight. The management team at the scene, excluding the Supervisor, were unknown to me, including their capacity and their involvement in the situation; this was frustrating and caused great confusion. 

In the interest of learning how best to handle the event and more accurately understand the significance of the odor event/fume exposure, I contacted the union emergency number and left a message requesting to speak with someone who could provide direction and knowledge on odor/fume events. Shortly thereafter, I was contacted by a representative who put me in contact with a Safety Chair. 

Due to symptoms I was experiencing, I requested to be transported to the hospital for further evaluation. It should be noted that medics provided resistance to transporting me and other crewmembers to the hospital and stated to me, "the hospital can't do much for you," and "anything you inhaled is likely out of your system already." However, all crewmembers, including non-rev Flight Attendant, requested treatment and were transported to the ER via ambulance for medical evaluation. 

After being evaluated and released from the hospital, the crew was transported back to the airport via cab to meet with the Supervisor, who secured our crew luggage at the airport. We retrieved our luggage from the In-flight Services crew room shortly thereafter. The crew was provided hotel transportation and accommodations at the local hotel in for the night, and the crew responded to their respective duties the following day.

Synopsis

After start and beginning to taxi, the Captain shut down the APU. Shortly thereafter, the "A" Flight Attendant notified the Captain of a dirty socks odor throughout the cabin so the aircraft was returned to the gate and deplaned. The crew was evaluated at an emergency room.

ACN: 1188783 (1837 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : AC Generation
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Distribution
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : AC Generation Indicating and Warning System
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 7480
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 2785
ASRS Report Number.Accession Number : 1188783
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At FL270, while at cruise altitude inbound to our destination, we had an electrical power transient that disconnected the Autopilot-1 (AP1) and flickered the Captain's PFD, ND, and upper ECAM. The system reset by itself with no apparent lasting effect. Within about 10-minutes, the same incident reoccurred with additional results of GEN-1 FAULT ECAM and a loss of ability to transmit on any VHF radio despite using all microphones and checking all connections. We attempted to ACARS Dispatch, but they didn't understand our shorthand message. Due to the close proximity to [our destination] and the complications of complying with clearances without transmission capability, our time was focused on continuing to navigate and get the aircraft to [our destination] safely.

We complied with all clearances and managed to acknowledge all transmissions by ATC using our Transponder IDENT function. We elected not to complete the GEN-1 ECAM as we believed the automatic process of the GEN-1 trying to reconnect to the electrical bus was creating our communications problems. We elected to put the GEN-1 switch into the 'OFF' position and leave it there to prevent a reconnect of the Generator to the Bus System. Our transmission ability was restored with GEN-1 offline. We verified that GEN-2 was fully powering all busses and systems. The APU was started, but the APU GEN was not connected to the bus for the same reasoning as GEN-1. The VMC conditions facilitated our descent and approach into our destination without incident. The GEN-1 Generator Control Unit (GCU) was found faulty by Maintenance and replaced. Aircraft System Malfunction. Electrical Power. Single Generator Failure.

Synopsis

A320 First Officer describes an inflight event caused by an electrical power transient that disconnected the Autopilot-1 (AP1) and flickered the Captain's PFD, ND, upper ECAM, a GEN-1 FAULT ECAM and loss of VHF radio. GEN-1 was manually positioned to 'OFF" to prevent GEN Auto-reconnect. VHF restored. Generator Control Unit (GCU) for Generator-1 found faulty by Maintenance and replaced.

ACN: 1188203 (1838 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : TPA.Airport
State Reference : FL
Altitude.AGL.Single Value : 800

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : TPA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : TPA

Component

Aircraft Component : MCP
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1188203
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were several hours delayed departing. As we taxied, we had a maintenance issue and had to work the problem. We were under the pressure of an EDCT time. After working the problem through Maintenance Control, we rushed through a before takeoff checklist and were cleared for takeoff. As we passed through 400 FT, the Captain called for heading select, I selected heading and saw that the heading was set at 021 which is way off of the RNAV SID heading. We then received an ECAM (master caution) that distracted me from the turn that seemed wrong. I fixed the problem quickly and then asked him why we were turning. Before we could ask for clarification, ATC began vectoring us back on course. It is unknown why that particular heading was set in the window. I had been so busy that I assumed that we had received the heading and I missed and the Captain indicated that he thought I had set it and that he had missed it. The flight continued without incident.

Synopsis

An A320 departed TPA with an incorrect initial heading selected, but neither pilot remembered setting it. ATC and crew both quickly identified the track deviation.

ACN: 1188199 (1839 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 1188199

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through FL240, we experienced a thrust loss on Number 1 Engine with associated yaw. Coordinated to stop climb and descended back to FL240. We ran checklists which directed us to turn on engine and wing anti-ice and then check to see if engine thrust could be regained. Engine anti-ice came on normally but wing anti-ice would not come on with amber caret on system page. Thrust would not recover. We left the engine at idle with normal parameters, consulted with Dispatch and coordinated a divert. On short final, we got an Engine Fail ECAM. First Officer ran the ECAM and shut down the Number 1 Engine. Landed uneventfully and taxied to gate under our own power after consulting with Fire Department. Approximately 10,000 LBS overweight for landing which we entered into logbook.

Synopsis

A320 Captain experiences a loss of thrust on the Number One Engine climbing through FL240 and the aircraft is leveled there to asses. QRH procedures are complied with, without success and the flight diverts to a suitable airport with the engine at idle. An Engine Fail ECAM is annunciated on short final and the engine is shut down.

ACN: 1187948 (1840 of 2908)

Time / Day

Date : 201407

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1187948
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

It appears that the vast majority of the PDC's now includes the clearance to "climb via SID", yet so many SID's have that altitude clearance written in small font buried someplace on the page. In other cases the altitude clearance on the SID is somewhat confusing as in the LAS SHEAD 8. This is just an incident waiting to happen. If ATC is going to put this on our back then the altitude on the SID should be more conspicuous.

Synopsis

A320 Captain laments the altitude restrictions burried in the routing section, in small font, in many of the new RNAV SID's.

ACN: 1187437 (1841 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : LAS
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1187437
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Ambiguous

Narrative: 1

Captain was pilot flying, First Officer pilot monitoring. ATIS info approach and landing RNAV VISUAL RWY 19L. From heading 170 degrees and east of TRROP and upon receipt of visual contact confirmation with LAS, Approach cleared [us] direct TRROP, "Cleared for RNAV VISUAL RWY 19L approach." Approaching TRROP they then handed [us] off to Final Controller who then cleared us direct LEMNZ, "Cross LEMNZ above 3,500 feet for helicopter traffic, cleared for the visual 19L." This would have required placing LEMNZ into the FMGC flight plan page while inside of TRROP and there was insufficient time to do so while maintaining situational and traffic awareness. LEMNZ in not noted on the [special, company tailored] approach chart and manual construction is not authorized. 

Conflicting requests by ATC should be avoided in high-density airports even in VMC.

Synopsis

An A320 flight crew, cleared to, "Cross LEMNZ above 3,500 for traffic, cleared for the visual runway 19L" at LAS was at a loss to do so as LEMNZ was not part of their FMGC flight plan route and they are not authorized to manually construct it.

ACN: 1187424 (1842 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1187424

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft had a deferred item: "SFCS 2 Flap Channel."

On climbout, at approximately 25,000 FT, the following warnings and cautions were observed: Warning--Auto Flt AP Off; Caution--F/CTL Flaps Locked-Wing Tip Brk On; Slat Sys 1 Fault; Auto FLT A/THR Off. The following systems were inoperative: Autopilot 1, 2; Autothrust; Flight Director; Speed Warnings, indications and flaps. ECAM and QRH actions accomplished. We declared emergency with ATC and coordinated return to our departure airport with ATC. 

After troubleshooting, we configured for landing with flaps selected 3, actual flap position was 0, actual slat position was 2. QRH landing speed and distance additives were applied. Landed safely on Runway X at our departure airport. 

Flap system failure was likely related to MEL.

I suggest we should re-evaluate whether maintenance procedures for releasing aircraft with MEL 27-51-01A are adequate. Secondly, we should train for 0-Flap landings without use of autopilot, Flight Director, or autothrust.

Synopsis

Dispatched with a deferred flap channel fault the flight crew of an A320 received multiple system fail warnings including locked flaps during the climb. They declared an emergency and returned to their departure airport landing with zero flaps and slats at position 2.

ACN: 1187061 (1843 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : L30.TRACON
State Reference : NV
Altitude.MSL.Single Value : 3500

Aircraft

Reference : X
ATC / Advisory.Tower : LAS
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Route In Use.STAR : GRNPA
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1187061
Human Factors : Confusion

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While approaching LAS on the GRNPA Arrival, we expected the RNAV VISUAL RWY 19L approach. The ATIS stated that visual approaches were in use for 19L/R and 7L/R. I thoroughly briefed the RNAV Visual 19L, as this approach plate states to expect it when 19L/R is in use. As we crossed LUXOR, we were given a 170 heading and told to expect direct TRROP and the RNAV VISUAL 19L. We were informed to slow to 210 knots, and then descended to 9,000 FT. We were cleared direct TRROP, descend to 6,000 FT, and I believe, cleared for the RNAV Visual 19L. Then, that Controller handed us off to the next Approach Controller, and he told us to slow to 170 KTS and asked if we had the airport in sight. We reported the airport in sight as we approached TRROP, and he cleared us, "After TRROP, go direct LEMNZ, cross it at or above 3,500, cleared for the visual 19L." We had not been expecting to go back on the GRNPA Arrival (where LEMNZ is located) so it took several seconds to get the fix back in the box. I flew a heading of about 270 for the direction of LEMNZ. As it turns out, LEMNZ is 8.5 miles from TRROP (on the arrival), while RIZZN is 8.6 miles from TRROP (on the RNAV Visual) so the fixes were overlapping each other on the ND, making it impossible to read either. As we neared the final approach course, ATC asked us if we were on a heading or track. I replied that we were having trouble with LEMNZ, but we had it now and would cross at 3,500 FT. He said to disregard the altitude restriction; spacing was good with the aircraft ahead, and contact Tower. We landed without further incident.

Synopsis

A320 Captain experiences confusion transitioning from the GRNPA arrival at LAS to the RNAV visual approach to Runway 19L. The last fix on the GRNPA, LEMNZ, is very close to the first fix RIZZN, on the RNAV visual and overlap on the ND and are not readable.

ACN: 1186006 (1844 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DFW.Airport
State Reference : TX

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : D10
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Airspace.Class B : DFW

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : D10
Aircraft Operator : Air Carrier
Make Model Name : MD-11
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Mission : Traffic Watch
Flight Phase : Initial Approach
Airspace.Class B : DFW

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1186006

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

[While] descending (autopilot on) [and] proceeding directly to PENNY fix on the final approach for Runway 17C at DFW international airport, we experienced a momentary aircraft upset, probably from wake turbulence that I believe [was] generated by the leading aircraft ahead. We were approximately seven miles from the leading aircraft that, unknown to us at the time, was [an] MD-11. As I noticed the rate of closure was increasing - aircraft separation was decreasing. I started slowing down the aircraft and almost at the same time our aircraft experienced an uncommanded 30 degree roll to the left in conjunction with 8-12 degree nose-down immediately after the aircraft rolled through wings level to 45 degree right bank angle. 

The aircraft also experienced an altitude loss of 300-400 FT in seconds causing high sink rates of up to 4,500 FPM. I immediately disconnected the autopilot and gracefully gained control of the aircraft without allowing the event to exceed any aircraft limitations or causing deviations. During this unexpected event, I tried my best to reduce the loading on the wings. 

[This was] an event that I was well prepared for, given that I just came out of my bi-annual simulator training and practiced a perfectly replicate of this event, which in turn laid the foundation to a successful outcome by giving me the tools to [recover] from this unwelcome risk. After all, training is the bone on which pilots sharpen their teeth. This event served as a constant reminder of how much gravity [is] resting on pilot knowledge and training. It demonstrated that an investment in pilot training and knowledge pays the best interest.

Synopsis

A320 Captain reported encountering wake turbulence in trail of an MD-11 on approach to DFW resulting in a 45 degree bank and 8-12 degrees nose down attitude.

ACN: 1185977 (1845 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1185977
Human Factors : Fatigue
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

[We] diverted due to thunderstorms closing [our destination]. Since this was an international flight, no Customs could be done. I informed Dispatch to make sure the on time was annotated but not to put an in time to insure the crew computer reflected flight time since all crew were to remain on flight duty. The computer did reflect an in time. We were on the ground for 51 minutes and not reflected as FAR 117 flight time. (I need clarification on this issue and guidance as to divert ground time on an international flight counting toward flight time or just duty time). Subsequently departed for [our destination], landed and cleared Customs. The next leg was cancelled. We immediately attempted to contact Crew Scheduling, were placed on hold and subsequently dropped. Scheduling finally called me at 1 hour 20 minutes after block-in and I was informed a message was sent to hotel accommodations. We watched for a change on our trip and tried to call Hotel Desk only to be placed on hold and subsequently dropped. I called the Duty Manager, left a message. He called me back and connected me directly with the Hotel Desk Supervisor. The Supervisor said "we are working on it" and he will call me back. An hour later I got a call from Hotel Desk sending us to Hotel X with voucher XY. When we got to the hotel, we were informed all "extra" crew were voucher XY and our inbound flight number had to be faxed or emailed and specifically tagged to rooms. No fax was sent. The Hotel Desk Supervisor called Hotel Desk and remained on hold. We, also, called and were on hold and after another hour of waiting I decided to find a hotel with rooms (the Hotel X was full except for voucher XY rooms specifically tagged to other inbound flights). I expensed a room at a nearby hotel almost 5 hours after block in time. I was still unable to contact Crew Scheduling to have them change our duty time in the crew computer for proper FAR 117 calculations (based on the fact we were waiting for a hotel assignment) until the next morning. Crew Scheduling informed me they could not change it and I had to call the Duty Manager to get it changed. I did this and had to leave a message. Duty Manager never returned my call so crew computer never properly reflected correct duty time and therefore all subsequent FAR 117 calculations were in error. 

Based on my personal calculations, I don't believe there were any FAR 117 violations, but am not 100% sure due to what happens next. [The next day] the plan was deadhead to pick up the balance of our trip. After flight planning, briefing the crew, loading the passengers, closing the doors, pulling the jetway, completing the Preflight Checklist, arming the slides and waiting for the last cargo door to close, the Gate Agent reconnected the jetway, knocked on the door and opened 1L. After seeing the door opened at 1L open, I asked the First Officer to determine what was happening. He returned and informed me the crew was supposed to call the Crew Scheduling. I told the Gate Agent to close the door we were sterile cockpit. My thinking was no sane operation would interrupt a flight that far along with a major change and I would deal with what I thought to be a minor problem when I got on the ground in [our destination]. I was wrong! When we called for push Ramp informed us we could not push and we needed to call Crew Scheduling. I did, breaking sterile cockpit, to be informed we were being reassigned to fly to [another location]. I told the Scheduler it made no sense based on where the flight was in terms of nearing pushback but he persisted. I told him to connect me with the Duty Manager. I spoke to the Duty Manager explaining how ridiculous it was from a safety point of view to take a crew at pushback and move them to another flight. He said he understood but it was contractually legal and therefore it had to be done. I continued to stress it might be legal but not smart. I also emphasized Crew Scheduling may not [be] working with correct flight and duty time calculations. This fell on somewhat sympathetic but deaf ears. I asked for an "ordered to fly" annotation in the record. I then told the First Officer to run the Parking Checklist while I informed the passengers and then slink away. I sent the First Officer to the next aircraft while I went to the planning room [and] took care of paperwork. When I pulled up our new trip I noticed we were being flown into days off; legal, yes but absolutely infuriating. During Pre-Departure Checks we discussed the need to let go of the emotion and focus on the task at hand to prevent bent metal and/or violation. It was not easy. I remain baffled why we as a company would take such a safety risk so last minute. I believe this problem could have been addressed at least 2 hours prior. To deal with this at pushback is not intelligent business or a safe practice.

Synopsis

A320 Captain describes a trip paring disrupted by weather diversions and the inability to communicate with the company due to the number of crews out of position.

ACN: 1185844 (1846 of 2908)

Time / Day

Date : 201407
Local Time Of Day : 1801-2400

Environment

Weather Elements / Visibility : Turbulence
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1185844
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Security
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Throughout the duration of this trip the Captain had poor communication skills and no perceivable thoughts as to the safety of his cabin crew. On the first night all three flight attendants were placed on separate floors, causing unsafe hotel conditions. The problem was only fixed the next night after I, as A Flight Attendant, asked that it not happen again. Although the Captain did state during his briefing that safety was most important, he failed to communicate when there was turbulence. I understand that there are times when it comes on unexpectedly but when it did come on he would not switch the seat belt sign on until we called up to have it done, and did not call back to alert or inform us. 

He also opened the flight deck door to use the restroom without informing the flight attendants so that we could pull out a cart to block off the aisle. There was a pilot jumpseating in the flight deck, but blocking protocol should still have been followed. I quickly pulled a cart out when I saw him and blocked off the aisle as per procedure. 

During the last flight of the night we were experiencing extremely moderate turbulence, due to the storms on the East Coast, and as A Flight Attendant I determined that it was unsafe for the B and C flight attendants to be out in the aisles at all, let alone with a cart, so the C Flight Attendant called up to the Captain to ask if he could make an announcement that the last water and coffee service of the night would be canceled. When he did he told the passengers to ring their call button if they needed anything. 

While the flight attendants in the back were still experiencing turbulence the Captain asked for coffee, which is a hot beverage. As the water was shut off in the front galley the only coffee was in the back where they were experiencing the brunt of the turbulence that was already communicated to the pilots. 

Several landings were more rough than necessary; one caused several passengers in the cabin to scream, while another got aborted by ATC for coming in at the wrong angle. We had to re-ascend and do it over. We also had an on ground delay during the trip, and the Captain failed to update the passengers every 15 minutes as per protocol. When I asked him if there were any updates he was very short with me.

It would seem to me that most of these problems could be solved by this pilot communicating better. The conditions of this trip were such that the flight attendants felt as though lines of communication were lacking between the Captain and cabin crew. This creates an unsafe work environment when the cabin crew does not feel comfortable communicating their conditions to the flight deck. On several occasions throughout the trip the flight attendants expressed to me their feelings of un-safeness. If an emergency were to happen we weren't sure that we would be given all the information we would need to properly keep passengers and ourselves safe.

Synopsis

An A320 Lead Flight Attendant reported a Captain's poor communication skills made the crew feel unsafe after he refused to turn the seat belt sign ON during turbulence, disregarded the cabin crew's other requests and then requested hot coffee during turbulence. Poor landings and a go-around also contributed to the sense.

ACN: 1185838 (1847 of 2908)

Time / Day

Date : 201406
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Cabin Activity : Safety Related Duties
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1185838
Human Factors : Physiological - Other
Human Factors : Workload
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Approximately 15-20 minutes after takeoff, the Captain turned off the fasten seatbelt sign. I was working in Economy. We had just finished setting up the beverage cart to start our service. The turbulence then became moderate. Coffee was coming out of the pots, water bottles were rocking back and forth as were we, in the back of the aircraft. The other Aft Flight Attendant called the flight deck and spoke to the Captain and asked him to turn on the seatbelt sign, as we were unstable in the rear of the aircraft. The Captain said "no" and refused to turn on the seatbelt sign. Both aft flight attendants were shocked at his response. She hung up the phone, not knowing what to say at this point. 

It was at this time a passenger was exiting the most rear lavatory. She had to hang on to the assist handle as she could not make it back to her seat. I then called the Captain, introduced myself and said, "Captain this is the back. I don't think you are aware of how bumpy it is back here. Could you please turn on the seatbelt sign?" He then answered, "I am in control of the seatbelt sign and I am not turning it on." I then said, "Are you kidding?" I thought he was being sarcastic and could not believe that he was questioning our judgment. Both aft flight attendants could not understand or believe his response. We then notified the Purser of what had taken place. 

I then talked with a deadheading pilot who was seated in the last seat position "F." I asked him if I was being unreasonable and how best to communicate with this pilot. He was also shocked at this pilot's response to our request. We considered our own safety needs and did not start the service until we felt comfortable. During the service, while it was perfectly smooth, the Captain came on the PA and asked the flight attendants to be seated. Both aft flight attendants looked at each other as we could not believe what he was asking. When we got to the rear galley, we stowed the carts and sat in our jumpseats. 

We called the Purser to find out what was going on. The Captain had not communicated anything to her prior and it was only after she called him, he mentioned there might be turbulence up ahead. We sat for approximately 15 minutes with NO turbulence. Both aft flight attendants felt that this pilot was not concerned with our well being and/or safety, or the passengers. This is a safety issue and our respect for this Captain was lost when he gave the response he did. We also then did not trust him or his judgment throughout the flight and was actually contemplating that he was purposely being erratic and irresponsible. His behavior conveyed a complete lack of concern for our safety and our passenger's safety. I would feel extremely uncomfortable ever flying with this Captain again.

Synopsis

A A320 Aft Flight Attendant asked the Captain during turbulance to turn the seat belt sign ON, but he refused. When asked a second time by the Lead Flight Attendant he responded "I am in control and not turning it on." The crew was in disbelief and distrusting.

ACN: 1185234 (1848 of 2908)

Time / Day

Date : 201407

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
ASRS Report Number.Accession Number : 1185234
Human Factors : Other / Unknown

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

My First Officer and I were reassigned to cover a flight that had diverted due to ground stop [at our destination]. They blocked in at the maintenance ramp hard stand a XA43. At approximately XC25 we relieved the crew due to their crew day legalities. We were briefed on the ground delay by the crew. We were unable to get flight planning information from Dispatch since Crew Scheduling had not officially assigned the flight to us. The plane was fueled at 10,000 LBS. XD20: Lightning begins on the field XD23: Crew Scheduling calls and assigns us to the flight. This allows Dispatch to flight plan with us as the crew. XD26: Passengers advised of their right to deplane; all remained. XD30: Still no flight plan in computer with ATC. XD43: Ground stop extended to approximately XD15. XE00: Ground stop lifted. We were unable to get takeoff data. Cleared by Clearance Delivery to go, we were told we were number 3 in line for takeoff and to call Ground when ready. We still could not obtain takeoff data. We were also given a re-route and a new flight plan. XE25: We called Ground and began our taxi. ATC gave a take off time of XE41. Ramp was also called at this time and informed we were taxiing for takeoff but had concerns with fuel and may have to return to the gate. XE35: I was uncomfortable with the fuel situation and made the decision to return to the gate. Our best scenario was to arrive with 51 minutes or 4,300 LBS of fuel remaining. 

Flight attendants informed us passengers were getting very anxious and many now wanted off the plane. We taxied onto the ramp and asked for a gate to deplane passengers and to refuel. Ramp would not allow us to go to a gate because opening the door would cause the flight attendants to go illegal and possibly cause the flight to cancel. We were told since the passengers were given the chance to deplane at the 3-hour mark that they were reset for another 3 hours. Dispatch was contacted and they confirmed this policy. Passengers were expressing a strong desire to get off. We expressed our concern that no water or food had been boarded and we were out. Potable water was available and served to the passengers. XF10: Flight attendants advised us the situation with the passengers was getting bad and they wanted to return to the gate. Flight attendants were now saying the situation was becoming a safety issue. This information was passed on to Operations and we insisted that we needed to park at the gate now. I informed them that this was my decision. They reminded us that the door could not be opened or the flight may not continue. I emphasized we had a safety consideration now and we needed a gate to deplane passengers. XF12: Dispatch advises the Tarmac Desk wants us to return to the gate due to lack of water and food. We continued to press Operations for a gate and now Dispatch is advising the same, little concern was shown by Operations. XF13: Flight attendants told us a women was getting ill and medical persons were attending to her and she was on oxygen. They confirmed we had a medical emergency. Operations was informed and we emphasized we had a medical emergency and needed a gate immediately with paramedics meeting the plane. Their answer was they were calling the Director. 

XF15: Flight attendants tell us another woman was ill and medical persons were attending to her. Passed to Operation that we had a second medical emergency and needed additional paramedics at the gate and a gate was needed immediately. Their response was, so you have a medical emergency? We confirmed that we had a medical emergency and needed a gate now. XF23: We were assigned and proceeded to a gate arriving at XF25. Ramp personnel were unloading other aircraft and we were without a marshaller or a jetway driver. XF29: Blocked in. XF32: Door opened with paramedics boarding. About XG00 the flight cancelled. To my knowledge the ill passengers declined further medical care from the paramedics. 

During the time we began requesting a gate and the second emergency call, two scheduled flights were parked at gates next to our position. I find the lack of response to our safety issues and declared medical emergency to be alarming. During our debrief, one of our flight attendants advised me that the pediatrician attending the first medical emergency told her that the ill passenger "was doing this to show what they had driven me to do." insinuating that she was faking the emergency. This underscored the flight attendants belief that we were having a safety issue due to passengers getting desperate and losing it. The second medical emergency appeared to be quite real, as a Flight Attendant told me the woman was a diabetic, looking poorly and going into shock due to a lack of food. Most of the passengers were on the aircraft over 10 hours.

Synopsis

Air carrier Captain describes an extensive delay caused by weather and poor handling by the company.

ACN: 1185224 (1849 of 2908)

Time / Day

Date : 201406
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1185224
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Work Refused
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

30 minutes prior to departure, I notified the crew desk that no First Officer had shown up for this trip. At departure time, crew desk informed me that they were still looking for a First Officer. I was asked if I was willing to fly with another Captain flying as my co-pilot, and I agreed to this request. A Captain was found who was willing to fly the red eye. Upon arriving at the aircraft, the Captain told me he wasn't aware that the assignment was for him to fly as a First Officer, and he told me it had been years since he had last been a co-pilot, and that he was not a Check Airman or LCA. I notified the duty Manager and Crew Scheduling that I didn't think this was a legal assignment, and I wasn't willing to fly with this Captain who was not qualified in the right seat. 4 hours after brief time, the flight canceled.

Synopsis

A320 Captain discovers no First Officer has been assigned to his flight and none is apparently available. A Captain is found and assigned the flight, but apparently not informed of the seat assignment. Upon arriving at the aircraft and learning the details he declines the assignment believing he is not qualified, having not flown as a First Officer in many years.

ACN: 1184594 (1850 of 2908)

Time / Day

Date : 201407

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Ground Personnel : Ramp
ASRS Report Number.Accession Number : 1184594
Human Factors : Training / Qualification

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I punched in and went to the west break room. My name came up and I was assigned to Gate X for a departure, it also listed two part timers that were a later start time. I then went to my locker and got my company issued radio and scanner. I walked over to Gate X stopping at a nearby gate to pull the trip up and see who my crew was. As I was signing in I noticed the aircraft was being marshalled in. I closed down the computer to assist in the download and arrival of the aircraft. When I arrived to Gate X there were two guys from training and a bag runner who was behind the engine and looked shaken up. I inquired what had happened and was told he walked behind the engine while it was still running and was knocked over form the jet blast. I proceeded to assist in the downloading and scanning of inbound bags.

Synopsis

Ramper reports arriving at his assigned gate just after the aircraft arrives to be informed that a new ramp employee has walked behind a running engine and was knocked down.