ACN: 1144862 (1901 of 2908)

Time / Day

Date : 201401
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20300
Experience.Flight Crew.Last 90 Days : 65
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1144862
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Ground Personnel
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.General : Evacuated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Upon my arrival at the gate, plane was completely dark and shut down. External power was not connected due to a write up that had deferred its use, this would effect this flights situation later. I proceeded to start the APU and power up the aircraft. During preflight setup I noticed the plane had 13,000 pounds more fuel than required for the short flight. This predicament would have us landing overweight, thus it had to be removed or switch aircraft. I notified Dispatch and Station Ops. Ops said they would have the fuel removed and we would have a short delay. My experience with de-fueling large aircraft is that it's never a quick exercise! The aircraft was booked full and was completely boarded before the fuel truck arrived to de-fuel. I advised the passengers of our predicament and that we would be delayed. The de-fueling was nearly complete when I was advised by the ground crew that the de-fueling had ceased due to a small fuel spill, and that the fire department had been called and the station advised! I left the aircraft to investigate, upon seeing the ramp flooded with fuel, which the Fire Chief later estimated to be 250-300 gallons of flammable Jet A. This fuel had migrated all over the ramp and was under and around the aircraft and pooling under the tail of the plane near the APU exhaust which was running due to our external power deferral! I went back upstairs to the plane and using the PA handset requiring all the passengers to evacuate the plane and leave all their stuff behind. The first half of the plane did a fairly good job of understanding the severity of the situation given the plane was beginning to smell like jet fuel, the back half started getting their things from the bins when a Flight Attendant keyed on the PA telling them to drop their stuff and get out! When the plane was empty I shut down the APU. I called Ramp and told them we still hadn't seen any emergency vehicles they said they would call, and then I powered down the aircraft total shut down (Go Dark)! Before leaving aircraft I referenced A320 QRH. Over half a dozen fire vehicles showed up and took on oversight of the fuel spill cleanup. 

During the cleanup which lasted over 3 hours I conferred with the local Fire Captain, Dispatch, Maintenance Control, gate CSR's, and later on the Duty Manager. Talking with Ramp personnel about the delayed response time of emergency vehicles I was led to believe our Ops center did not call fire trucks as requested since this was not a company responsibility but the fuel vendor's problem. The Fire Captain told me that his units were not advised of the situation by the Company even though we have a direct line with his station. He received a 911 about a fuel spill with no idea of the severity until his units arrived on site. At no time did I witness any company supervisory personnel attempt to involve themselves with this near disaster in anyway. The Fire Captain advised me nobody from the company other than me had made themselves available to him during this situation. This is our largest hub I believe, not some out station on the other side of the planet. This lack of support so close to the mother-ship gives me cause for concern! Before re-boarding the aircraft we utilized a tug to reposition the aircraft. I and The Fire Captain felt this would be a safer scenario given the state of the fuel soaked ramp. Fortunately I can say 4 1/2 hours later we left with most of our original passengers.

Synopsis

A320 Captain reports a fuel spill during defueling with the passengers on board. The passengers are asked to evacuate the aircraft via the main cabin door. CFR does not appear quickly and the Captain learns that they were not called. The company believed it to be the fuelers' responsibility to call.

ACN: 1143283 (1902 of 2908)

Time / Day

Date : 201401
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1143283

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

During the pre flight I became aware that the aircraft was downgraded to Cat 1 only and the ILS was inoperative on my side. The forecast for our arrival time was 400/1 and well within our landing minimums for the RNAV approach. When we were approximately within 600 miles of the airport the weather had deteriorated to VV 100 and 1/16 of a mile. I notified Dispatch that this made any approach impossible and that I felt it was time to divert to our alternate airport. The new forecast did not have the weather improving to RNAV landing minimums until XA:00 CST. I was informed that he would like me to continue in hopes of improvement. We were within 200 miles of the alternate and moving further away as we flew. After another ACARS message to company I was informed that I was to continue on and hold for an hour and then divert. As we got closer, the weather was still not improving, so I decided to contact Dispatch. At this time I made it clear that there was no way to land in this weather with the downgraded conditions of our aircraft. He felt that by that time we would be able to shoot the RNAV if there were no simultaneous approaches being conducted. I was told to continue on and hold and hope for better weather. When we arrived in the terminal area we were placed in holding and I once again contacted Dispatch via ACARS. I was then told to refer to the chart for the RNAV. I was then told that my landing minimums were RVR 2400 with no simultaneous approaches. I explained that the minimums for us were RVR 4000. We were not category A or B aircraft and that the best visibility on the field was only RVR 1200. He then wanted me to try the approach and if I went missed to continue on to my alternate. At this point I felt there was a lack of understanding of how poor the weather was and decided to depart for the alternate before something bad happened. 


Upon arrival at the alternate, maintenance met the aircraft. The aircraft was restored to CAT 3 status and we departed for our original destination. We shot a CAT 3 autoland and the weather was VV100 and RVR 1100. We did not see the runway until 50 feet and would never have gotten in on the RNAV. When I got to the gate I was told MX had been waiting all night for us because our aircraft had all the MX parts for RON aircraft. I suddenly got the feeling that this may have been the reason for all the pressure to get in at all cost. I don't feel Dispatch had any malice in pushing me, I just feel he was pressured to get us in. I had notified them that the aircraft was downgraded and my First Officer was high minimums on his first trip out of OE. I had a MX Supervisor on the aircraft the whole time and he couldn't believe the aircraft was not signed off before being dispatched into that kind of weather. I don't really have any suggestions on how to fix this type of problem. I just hope the next guy doesn't shoot an approach that is illegal because of being pressured. I would hate for someone to lose their license or worse.

Synopsis

A320 Captain reports being dispatched in an aircraft downgraded to CAT I with weather forecast to be above approach minimums. Enroute the weather goes below minimums and a diversion is requested from Dispatch but denied. After holding with no improvement, Dispatch suggests that the Captain attempt an approach with the weather below applicable minimums. The Captain diverts to his alternate.

ACN: 1142384 (1903 of 2908)

Time / Day

Date : 201401

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase.Other 
Maintenance Status.Released For Service : Y

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1142384
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

This incident was reported to me as a Manager by a Service provider that is unable to file a safety report [using our Air Carrier's Safety Form]. The details are quoted as follows. Leaking Dangerous Goods (DG):

Please check this Bulletin that originated in ZZZZ and going to ZZZZ1 via ZZZ. It came to my knowledge only a few hours ago that this is a leaking DG and was set up and sent on a flight to ZZZZ1. I do not know when and where the leak started, but it was in ZZZ for several days and the Customer was following it up at ZZZ. The warehouse Manager gave the 'go' signal to send it to its final destination by just wrapping the freight in Quick Packaging (QPK) plastic bag. I am a DG Specialist in ZZZ and I believe that the procedure for handling a damaged DG was not followed for this freight. If I knew earlier that this was a leaking DG, I would have voiced out my objection in sending this freight to its final destination and have the Shipper's Representative in ZZZ to come and pick up the freight in our station.

Synopsis

A Dangerous Goods (DG) Specialist reports he was informed about a damaged freight that had been improperly shipped from their Station, leaking hazardous fluid on an A320 flight.

ACN: 1142376 (1904 of 2908)

Time / Day

Date : 201401

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1142376
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

[We] aborted takeoff at approximately 80 KTS due to ECAM for aft cargo door open indication. [We] returned to the gate. Maintenance found the door handle out of the stowed position. No maintenance action required. Topped off fuel and departed uneventfully.

Synopsis

An AFT CARGO DOOR OPEN ECAM alerted during takeoff, so the takeoff was rejected. At the gate the door handle was found out of place, closed properly, fuel topped off and the flight departed uneventfully.

ACN: 1141670 (1905 of 2908)

Time / Day

Date : 201401

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Component

Aircraft Component : Rudder
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
ASRS Report Number.Accession Number : 1141670
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After pushback and engine start we performed the flight control check; all normal except for the rudder. With full rudder pedal deflection by the First Officer or me, the ECAM rudder position indication was of only half the required travel. We reported the issue to Maintenance and the decision was made to MEL this as an indication problem. I queried Maintenance Control on how the MEL's required external visual full rudder travel inspection would be accomplished. Initially Maintenance Control stated there were full travel markings on the tailcone but then rescinded that and told us there was only a rudder neutral marking. I asked Maintenance Control how they could determine full rudder travel versus 50% travel and I was told there was no way to guarantee that. 

We decided to refuse the airplane. I am very confused by this MEL item. The MEL states that full rudder travel must be verified visually. If the airplane has no markings and apparently there's no other way to measure full rudder travel from the ground. How can this be a legal and more importantly, SAFE, MEL?

Synopsis

An A320 Captain refused an aircraft after the full rudder throw flight control check yielded only half the required movement on the ECAM Rudder Position Indicator. The MEL allowed dispatch after a visual check but no rudder position indicators are installed on the empennage.

ACN: 1141579 (1906 of 2908)

Time / Day

Date : 201401
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Switch
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Technician : 27
ASRS Report Number.Accession Number : 1141579
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

[I] replaced right hand #20GA Proximity Switch. I have just been informed by Supervision that the aircraft needed [to be] jacked to check clearance of [Proximity switch] Target. The Aircraft Maintenance Manual (AMM) hyperlink to this Task was inadvertently missed due to the Airbus Aircraft Maintenance Manual (AMM) format that is hard to navigate. I was made aware of this after another aircraft had the same switch replaced and the jacking procedure was discovered.

Maintenance Control stopped the aircraft in ZZZ1 to comply with jacking procedures the same day. Airbus AMM hyperlink format is not user friendly. Job specific hyperlinks are not located in the specific task being performed.

Callback: 1

Reporter stated he replaced the Proximity "Squat" switch on the right Main Landing Gear (MLG) of an A320 aircraft at the Line (Terminal). They had to rob (cannibalized) the switch, due to a lack of available parts at their Station, from another Airbus that was already 'Out of Service'. He used the Airbus Trouble Shooting Manual (TSM) and the Centralized Fault Display System (CFDS) to identify the cause of a 'Hard Fault' that appeared on the A320's ECAM five minutes before departure. The Proximity 'Squat' switch had failed to the
'Open' position, showing 'no continuity'. 

Reporter stated he is more familiar with the Boeing Maintenance Manuals than the Airbus Manuals which references required Checks and Tests differently than Boeing does. As a result, one of the maintenance pages for the Airbus Task to Remove and Replace (R/R) the proximity 'squat' switch that he had changed, also lists (includes) all the proximity switches for the right MLG. What he did not do was to scroll down past the entire list where a Maintenance "Note" would appear, instructing him to jack the aircraft and perform a "gear extension"; not a gear swing, to verify the Proximity switch and 'Target' had the proper gap, or needed adjustment. The Proximity switch (sensor) is attached with two bolts and is not adjustable.

Reporter stated he felt confident at the time about having corrected the discrepancy, even though he was unaware of the required MLG jacking and extension requirement check, because the BITE Tests showed the ECAM fault had cleared. When the A320 arrived at the next Maintenance Station, no jack was available to perform the Gear Extension Check. But since the aircraft did not have any inflight issues with a recurring proximity fault, Engineering allowed continued operation of the aircraft for twelve more days before a Gear Extension Check was actually accomplished.

Synopsis

A Line Aircraft Maintenance Technician (AMT) describes how a chain of events, starting with how the Airbus Aircraft Maintenance Manual (AMM) is formatted, contributed to his inadvertently missing a required Main Landing Gear (MLG) Extension Check; after replacing the right MLG Proximity 'Squat' switch on an A320 aircraft.

ACN: 1141205 (1907 of 2908)

Time / Day

Date : 201401
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 8900
ASRS Report Number.Accession Number : 1141205
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Contract Maintenance was in cockpit [neither] unsupervised nor observed by any crew member. We were both in operations flight planning return leg. Disregarding normal safety practices, Contract Maintenance did not ensure flight controls were safe to move prior to actuating. Consequently, aircraft was seriously damaged and refueling personnel nearly injured when contract maintenance moved flap lever with fuel truck connected.

Synopsis

A320 First Officer reports flap damage caused by Contract Maintenance when the flaps are extended at the gate with a fuel truck parked under the wing.

ACN: 1140946 (1908 of 2908)

Time / Day

Date : 201401
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 1140946

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1140952

Events

Anomaly.Ground Excursion : Taxiway
Anomaly.Ground Event / Encounter : Other / Unknown
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Weather
Primary Problem : Airport

Narrative: 1

We pushed back from the gate and getting underway was a tedious process due to the severe cold, ineptitude of the ramp personnel, and the usual slow ACARS performance with takeoff data. Due to the horrendous [winter] conditions of the night before we planned to start both engines, let everything warm up slowly, and carefully taxi out due to expected poor braking action on the apron and taxiways. We were cleared to Runway XX via Taxiway XX and X. 

As we slowly transitioned from the apron to Taxiway XX the aircraft began to skid toward Taxiway X. I tried to arrest the skid by reapplying the brakes and tried to steer the aircraft onto Taxiway X but braking action was NIL and the airplane continued to slide down the slightest incline on XX from the apron to X. The airplane came to a stop at an angle with the nose gear and left main gear in the snow on the left side of Taxiway X. We did not know at the time if we were off the hard surface so we started the APU, shut down the engines, and notified the flight attendants and passengers as to what had just happened. 

We sent Dispatch a message through ACARS that we slid off the taxiway and he started coordinating with Flight Operations managers and Maintenance with regard to moving the airplane. We also notified Ground and they sent an OPS vehicle to meet us and gave us a discrete frequency to communicate on. The Operations people told us that our nose gear was about two feet into a snow bank with snow up to the axle. He said the left main gear was on the edge of the snow bank and that the aircraft was still on the taxiway surface. He called for a front end loader to remove the snow in front of the aircraft so the tug could pull us back to the gate. The tug pulled us to the gate without further incident.

Narrative: 2

ATIS reported slippery conditions all taxiways. It was late evening and the temperature was approximately -18C. The Polar Vortex weather conditions that we experienced on arrival the night prior were still present, but there was no precipitation and visibility was good. The pavement appeared patchy ice and dry.

Synopsis

An A320, taxiing for takeoff during extreme winter field conditions lost traction, slid into a snow bank with the nose gear off the hard surface.

ACN: 1140217 (1909 of 2908)

Time / Day

Date : 201401
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Fan Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1140217
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Ambiguous

Narrative: 1

At the start of our descent we had a Number 2 Reverser fault caution light. We ran QRH, Resets, Normal Checklists and consulted the AOM. The only guidance we found was in the AOM. It dealt with the number 1 or 2 reverser fault. The action was to reduce thrust in the right engine. We called Dispatch and Maintenance and consulted as to the correct course of action. We involved a phone patch with our Fleet Manager. With their concurrence we confirmed and reduced thrust on the Number 2 Engine to idle. As a precaution and to ensure priority handling we declared an emergency with ATC. The AOM guidance included language to land at the nearest suitable airport. By the time we had the situation under control we were only about fifty mile from the destination airport and still about 25,000 FT MSL. We felt the best course of action was to continue our descent and prepare for a straight in landing. It was already programmed and the runway is over 11,000 in length and 200 FT wide. We felt confident that this was the correct course of action. We used the single engine guidance from the AOM and did a flap three landing. The emergency was really a non event with a positive outcome.

Synopsis

A320 First Officer experiences a reverser fault during descent. After consulting the QRH, AOM and Maintenance Control the crew elects to reduce the engine to idle and continue to destination after declaring an emergency.

ACN: 1137333 (1910 of 2908)

Time / Day

Date : 201309

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Relative Position.Angle.Radial : 075
Relative Position.Distance.Nautical Miles : 18

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Weather Elements / Visibility.Other

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 8800
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5800
ASRS Report Number.Accession Number : 1137333
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
Detector.Person : Ground Personnel
Miss Distance.Vertical : 400
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Ambiguous

Narrative: 1

There was traffic with no communication, we saw them so close. The evasive action was smooth and we continued with our approach. It is important to tell you that ATC advised us about traffic without altitude information in the area. I think that this event occurred because of the distraction of the other aircraft in the communications and in the airspace class that they were flying.

Synopsis

An A320 on approach took evasive action to a near miss TCAS RA east of LAS from ATC identified traffic which did not report altitude information.

ACN: 1136882 (1911 of 2908)

Time / Day

Date : 201312
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PDX.Airport
State Reference : OR
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Fog
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1136882
Human Factors : Human-Machine Interface

Events

Anomaly.Other 
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

Performed normal preflight duties and procedures, ATIS advertised Runway 10L [departures] IMC conditions. 300 overcast plus 1/2 mile and fog. During our briefing we noted company provided single engine failure procedure requires PDX VOR to identify 8.4 DME fix. We checked radio aid and found it to be inoperative. Checked chart NOTAMs as well as flight release NOTAMs. PDX VOR should have been in service. We contacted ATC (PDX Ground) and they stated that the PDX VOR was decommissioned over 1 month ago. 

Our engine failure procedure requires us to identify the PDX 8.4 DME fix so that we avoid rising terrain in the Columbia Gorge. Since we cannot determine how fast we will climb on a single engine waiting until we reach 1,031 FT could cause us to enter this area of rising terrain. If we reach 1,031 FT before 8.4 DME we are allowed to turn and head out the Columbia River towards the Pacific Ocean.

[There appears to have been] inadequate review by the company when providing engine out procedures at our scheduled airports. An alternate procedure should have been provided without our inquiring.

Synopsis

An A320 flight crew discovered their company's special engine failure procedure for departures from Runway 10L at PDX required DME information from the OTS PDX VOR.

ACN: 1135611 (1912 of 2908)

Time / Day

Date : 201312
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : BIL.TRACON
State Reference : MT
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : BIL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 155
Experience.Flight Crew.Type : 1250
ASRS Report Number.Accession Number : 1135611

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1135924

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Approaching BIL level at 7,000 FT in VMC. We were about 5 south of airport awaiting clearance from ATC for an approach. I noticed traffic at one o'clock departing the pattern at BIL climbing towards us on TCAS. As we proceeded the traffic continued towards us but ATC said nothing. We eventually got a TCAS climb and climbed approximately 4,500 per TCAS direction. The other traffic continued to climb into us on the TCAS. As I told ATC of the TCAS climb the Controller pointed us out as traffic to the aircraft causing our TCAS climb. I inquired if the other traffic was VFR or IFR and was told it was VFR. The Controller was controlling us and in radio contact with the other aircraft, but never attempted to prevent the TCAS event. As I watched the map display the other aircraft came within 400 FT of us on the TCAS. It's disconcerting that ATC did nothing to prevent this avoidable TCAS event.

Narrative: 2

[Report narrative contained no additional information.]

Synopsis

A320 Captain reports a TCAS RA approaching BIL at 7,000 FT with a VFR departure, both talking to ATC. ATC makes no attempt to prevent the conflict and the A320 climbs 4,500 FT at TCAS direction.

ACN: 1133954 (1913 of 2908)

Time / Day

Date : 201312
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Snow
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DEN
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway 35L
Flight Phase : Final Approach
Airspace.Class B : DEN

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : D01
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class B : DEN

Component

Aircraft Component : Traffic Collision Avoidance System (TCAS)
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1133954
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Training / Qualification
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 12579
Experience.Flight Crew.Last 90 Days : 207
Experience.Flight Crew.Type : 3479
ASRS Report Number.Accession Number : 1133935
Human Factors : Human-Machine Interface
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

Denver was operating parallel approaches to 35L and 35R. We were on a straight in approach to Runway 35L in IFR conditions with reported heavy snow and parallel operations being conducted to Runway 35R. Approximately 23 miles from the field we were instructed to contact Tower. The Captain was flying the aircraft with both autopilots engaged for the briefed Cat III approach to autoland. At an approximate altitude of 10,000 MSL the TCAS gave a commanded descending RA because of an aircraft on approach to the parallel Runway 35R. All procedures and radio calls were made appropriately with the Captain disengaging the autopilot and immediately initiating a vertical descent while remaining on the lateral profile and I made a radio call to Tower of TCAS descent. While the Captain was primarily engaged with the maneuver I focused on the intruder position and mentioned he was to the right and below. We both agreed that a descent into the intruder did not make sense, but the TCAS command was followed as we do not know what the intruder is being commanded or if they were receiving any TCAS commands. I would approximate 10 seconds later the TCAS gave an abrupt reversal command for a maximum climb which the Captain immediately performed and I gave another call to Tower of a TCAS climb and told the Captain they were close to the right and 200 FT below.

Shortly after initiating the TCAS climb the event was over. I asked the Captain if he would like to do a go-around which he instructed yes since we would not be able to get back on a stabilized vertical profile. We were instructed by Tower to maintain heading and climb to 10,000 FT and eventually we were switched over to Approach Control. At all times we remained on the proper lateral profile for the approach to Runway 35L. At no time did I hear any response from Tower to the two TCAS callouts made.

Our TCAS was obviously operating in the TA/RA mode. Subsequent discussion with the Captain about the event was the Company FOM mentioning of an allowed procedure to switch the TCAS mode to TA only when intentionally operating in close environments to other aircraft to prevent RA's. In contrast to a PRM approach where there is a monitoring Controller, in this case we have two aircraft more than 20 miles from the airport on different Tower frequencies, on different approaches, to different runways, approximately 5,000 FT apart. While I understand the intention of the procedure of switching the TCAS mode to TA only and its benefits while in VFR conditions it should be noted this is not a required procedure and I personally do not agree with the procedure when in IFR conditions operating outside Tower's normal area of responsibility, not knowing who is responsible for monitoring the positions of close aircraft on non-PRM approaches, on different frequencies, coupled with the fact that this procedure will never prevent an overshoot or blunder into the path of an adjacent aircraft.

Further discussions with the Captain was that I am sure even with parallel runways approximately 5,000 FT apart that Approach Control was attempting to stagger (offset) the aircraft to adjacent runways but for some reason it did not work out that way for our approach. Our second approach had the adjacent aircraft between 2-3 miles ahead of us on their profile and no TCAS events occurred. If possible I would like to receive notification of how close our two aircraft came to each other.

Narrative: 2

I was pilot flying on ILS 35L at DEN between CHOLA and CELBI descending between 10,000 FT and 9,000 FT on glideslope when we got a "traffic, traffic" advisory followed by a Resolution Advisory (RA) commanding descent. Intruder was to our right and approximately below us. It did not make sense, but I followed the RA as required. A few seconds later TCAS reversed the command calling for a maximum TCAS climb which I immediately followed. The First Officer made the 2 appropriate "TCAS descent" and "TCAS climb" calls on Tower frequency. During the reversal the First Officer called out to me "traffic close on right 200 FT below." Within seconds after establishing the TCAS climb the event was over. 

Soon (seconds) thereafter I initiated a go-around because there was no way we were going to be able to get back to glideslope in a stabilized condition. We leveled at 10,000 FT, informed Tower and accepted vectors for another ILS 35L Approach which was normal. All proper TCAS maneuvers and Tower calls were made thus avoiding what really looked to us like a near midair situation. Tower never acknowledged our calls during the event. After leveling at 10,000 FT following the go-around I asked Tower about the position of the intruder aircraft. Tower said that the other aircraft was established on the Localizer for ILS 35R. I do not believe this was true. We had been established on and identified on the Localizer for ILS 35L since at least CPPER. It should be noted that the altitudes and positions noted in this report were reconstructed by myself and the First Officer following the event and were therefore approximate notes.

Synopsis

An A320 crew on the DEN 35L ILS took evasive action in response to a TCAS DESCEND command with lower abeam traffic on the 35R ILS, but the command reversed to the correct CLIMB command after about 10 seconds. A go-around was initiated at that point.

ACN: 1132584 (1914 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 7500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2400
ASRS Report Number.Accession Number : 1132584
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1132590
Human Factors : Distraction

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This was our first flight of the day and the first leg of the trip. Preflight, pushback and start were completely normal and to SOP. Both engines were started and the Pre-taxi checklist was completed prior to starting to taxi. The weather was clear and all ramp areas were dry. There was a company 737 pushing at about the same time from an adjacent gate. When given taxi clearance from Ramp Tower, we were instructed to transition to the Charlie line for a west taxi to avoid the 737 taxing east. We began our taxi and I visually cleared the right side of our aircraft and verbalized clear "right" to the Captain at least two times while we were moving toward the Charlie line. Once we were established west bound on the Charlie line I looked forward. The Captain was looking left to ensure we were going to be clear of the 737 taxiing east. In my peripheral vision, I detected movement to our right. I saw a baggage tug with three or four carts in tow traveling toward us from right to left perpendicular to our path at a high rate of speed. I yelled "watch out" to the Captain. The Captain immediately applied full brakes and abruptly stopped the aircraft preventing a collision with the tug. The tug driver appeared to have been looking at a white sheet of paper while he was driving and the tug driver swerved right toward the west at the last second when he finally saw us. The tug driver continued south without stopping. Ramp Tower witnessed the incident and called us to ask if we got an identification number off the tug. I commented to Ramp Tower that we just had a close call with the tug and we thought the tug number was. About the same time, the Captain asked me to check on the flight attendants. We received a call from the Purser that a flight attendant who had been standing while doing her Cabin safety checks had been hurt. We asked if medical attention was needed and the Purser said yes. We told Ramp Tower we needed to return to the gate and to have the EMTs meet us at the gate. We coordinated with Operations to return to the Gate. It took several minutes to get ground personnel to park us and the jetway back to the aircraft. We blocked in approximately five to seven minutes after the incident. The EMTs arrived shortly thereafter. The Captain stepped back to the Cabin upon completion of the Parking checklist. He asked everyone to remain seated to allow the EMTs to attend to the [injured] Flight Attendant. The EMTs removed [the injured] Flight Attendant from the aircraft.

Narrative: 2

Pushback from gate was uneventful for a west taxi. Started both engines and completed all checklists prior to calling for our taxi clearance. One of our 737s had pushed from an adjacent gate and was set up to taxi east. When we called Ramp Tower for our taxi clearance, we were given "taxi around the 737, taxi west on the Charlie line, contact Ground". We were complying with those instructions and as I was looking left to ensure proper wing tip clearance from the 737, my First Officer yelled out to watch out for the fast moving baggage tractor towing four carts, coming from right to left. I looked up in time to see him zipping by while reading a paper. I had no option but to apply maximum braking on the aircraft to avoid the impact. He swerved away and continued on his merry way. Ramp Control saw it all happen and immediately asked us if we saw the vehicle number. [We thought we saw the vehicle number] but the paint was streaked and scraped quite a bit so we could not be certain of it. Ramp Control told us they were trying to track down this ramp serviceman right away and asked us if we needed any assistance.

When I executed the rapid stop, our flight attendants were walking in the aisle performing their safety checks. We received a call from our Purser, letting us know that our 3rd Flight Attendant was down in the aisle and needed medical attention. We immediately let Ramp Control know that we would need to return to the gate and that we would need paramedics. Ramp Control was very helpful in getting us back to gate and sending medical help.

Upon return to the gate, I asked for all of our passengers to remain seated while the paramedics tended to the [injured] Flight Attendant. After completing our Parking checklist I opened the cockpit door in time to see one of our passengers tending to the Flight Attendant. I assume she was a medical professional that was helping out. A Flight Attendant Supervisor came on board and fulfilled the requirement to have three flight attendants on board while FA was taken off by the paramedics. He also took her bags back to the domicile office after our replacement arrived. I also spoke with our other two flight attendants. One injured her back but chose to continue. The other reported that he was not injured and also chose to continue. I also called our Duty Manager and filled him in on what had happened.

Synopsis

A320 flight crew reports a near collision with a tug pulling carts during taxi from the gate. Hard braking is used and the tug driver swerves at the last second to avoid collision. A Flight Attendant is injured and the flight returns to the gate for medical assistance.

ACN: 1131579 (1915 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Air/Ground Communication
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : VHF
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 105
Experience.Flight Crew.Type : 2600
ASRS Report Number.Accession Number : 1131579
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1131582
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

All three Radio Management Panels (RMPs) [radio] frequency windows went blank and we were unable to change radio frequencies. However, we were still able to communicate with ATC on last assigned radio frequency before windows went blank, then [did communicate] on Guard frequency, and also through relay with other aircraft. We were able to send and receive ACARS messages with Dispatch and Maintenance. Per Maintenance, we attempted a reset of all three RMPs and were able to use VHF-3 for radio frequency changes and communication. We were able to continue and land at ZZZ1.

Callback: 1

Reporter stated the reset procedure for the Radio Management Unit RMPs suggested by Maintenance was to cycle the 'On-Off' power switch and wait five seconds between position selection, before returning to the 'On' position. Only # 3 RMP was functional for frequency and communications after that on the A320. The "on guard" reference was the phrase they use when they monitor 121.5 on Comm # 2 and not meant as a reference to the 'guarded' NAV selection on the RMPs. Maintenance later replaced the Audio Management Unit (AMU) and # 1 RMP.

Narrative: 2

We lost the ability to change frequencies on our radios. The [radio frequency] windows were blank. We were able to speak with ATC on Guard [frequency]. Through our ACARS data link we could communicate with Dispatch. After nearly exhausting all of our options to recover the ability to change frequencies, VHF-3 came back to life and the Captain was able to communicate normally. System malfunction.

Synopsis

Two pilots report all three Radio Management Panel (RMP) frequency display windows went blank in flight and they were unable to change radio frequencies on their A320 aircraft.

ACN: 1131394 (1916 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2100

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1131394
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Distraction
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1132270
Human Factors : Troubleshooting
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Just after setting climb thrust on takeoff I noticed that the cockpit smelled like dirty socks. The First Officer confirmed the smell as well. I then transferred control of the airplane to the First Officer and contacted the flight attendants. They also confirmed the smell as well. We told them we would be retuning to departure airport and to prepare the cabin for landing. We then notified ATC that we would be returning. I contacted Maintenance and they notified Company that we would be returning. The smell started getting worse. Since we were above landing weight we completed the Overweight Landing Checklist. We landed and then returned to the gate a half hour after takeoff. All the crew was experiencing effects of chemical inhalation so we had paramedics come to the gate. Many passengers were also experiencing symptoms as well according to the flight attendants. A few of the passengers approached me and were very thankful we landed and confirmed that they were also worried about the "smell" as it was very strong. The paramedics recommended that we (the crew) all be transported to the hospital for further treatment and observation.

Narrative: 2

Captain called Flight Attendant A, asked if we smelled anything, we did not. Asked Flight Attendant C to due a walk through cabin. I was C, so I did a walk through cabin. Asked B if he smelled anything, he did not. As I walking back up to my jumpseat and reached overwing, I did smell a bad snuffy odor. When I got to my jumpseat, I told Flight Attendant A I smelled the odor at overwing. She called Captain back, and apparently the odor was really bad in the flight deck, and the Captain then made an announcement, we were going to return to the gate, we were probably only 10-15 minutes out. 

Passengers all deplaned, [and] then all crew stayed in the jet bridge. Captain called paramedics and they met up us. Paramedics checked us all out upstairs, and took us all to the hospital were we all spent approximately 5 hours in ER.

Synopsis

An A320 developed a strong odor after takeoff, so the Captain asked the flight attendants to confirm cabin odor then returned immediately to the departure airport where paramedics suggested medical treatment for the crew.

ACN: 1131276 (1917 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZMP.ARTCC
State Reference : MN
Altitude.MSL.Single Value : 35000

Aircraft

Reference : X
ATC / Advisory.Center : ZMP
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZMP

Person

Reference : 1
Location Of Person.Facility : ZMP
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1131276
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working Sector 30 and was advised by the adjacent Controller that Air Carrier X was heading towards my sector and that he was having radio problems and could only transmit and receive on 134.57 (and 121.5). Air Carrier X departed ZZZ enroute to ZZZ1. The 134.57 frequency is not a frequency we can transmit or receive on. That meant we could only communicate indirectly with Air Carrier X, using other aircraft to relay to him on 134.57 or 121.5. Apparently the Command Center was aware of this problem, and the Supervisor was coordinating with adjacent facilities. As we weren't busy at the time we objected in principle to the situation but had no real pressing need to talk to the aircraft as it transited ZMP airspace. However, not being in direct communications with an aircraft for extended periods of time, heading to a busy destination such as ZZZ1, creates an unsafe situation, as well as additional workload for the controllers. The aircraft should have been ordered to divert to another airport to fix the equipment malfunction before being allowed to proceed to a busy east coast airport such as ZZZ1.

Synopsis

ZMP Controller voiced concern regarding the decision to permit an air carrier, limited to only one frequency, to transit multiple airspace environments and land at a busy east coast airport.

ACN: 1129362 (1918 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing

Component : 1

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1129362

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

About 30 minutes before departure we got Brakes System 1 Fault ECAM and notified Maintenance. They deferred Brake System 1 and we blocked out 4 minutes late after having to wait for the maintenance release. After start and before taxi we got N/W STRG Fault ECAM and I contacted Maintenance Control. He had me cycle the A/Skid and N/W STRG switch which cleared the ECAM. During the short taxi to the runway, I had to hold full left rudder or about 10 degrees left with the tiller to track the taxiway centerline. The First Officer and I both agreed that this was not acceptable and we returned to the gate. Maintenance replaced the BSCU black box which cleared the Brake System 1 deferred item and completed the autoland test to return the aircraft to CIII status. Maintenance finished the repairs and after a discussion with Dispatch about fuel for the flight we were OK with the same planned fuel and called for the fuel truck while they boarded the flight. After about 40 minutes and six calls to Operations and no fueler, but were told each time that the fueler was on the way, I called Dispatch again and raised the fuel 700 LBS to generate a new fuel request and advised him of our problems getting fuel. He said that he would advise Load Planning as they had a direct line to the fuelers. After a two hour maintenance delay this additional delay was NOT acceptable. The taxi and takeoff were normal. After landing we got Brake System 1 Fault and Steering ECAMS with some erroneous steering inputs that I corrected with differential braking and we exited the runway onto taxiway. During the rollout the Tower cleared us to cross the parallel runway but without steering I stopped short of the runway and we advised Tower of our mechanical problem and would need a tug to be towed to the gate. The First Officer contacted Maintenance and they wanted us to reset several circuit breakers which I declined after having just experienced the steering failure. I did not want to risk blocking the runway as just past the hold short line is a sharp turn that I did not want to attempt with differential braking between landing aircraft if we had another failure.

Synopsis

A320 Captain experiences brake and steering faults prior to departure and Maintenance replaces the BSCU. After landing at destnation the same faults return and the aircraft is towed to the gate.

ACN: 1129070 (1919 of 2908)

Time / Day

Date : 201311

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18700
Experience.Flight Crew.Last 90 Days : 185
Experience.Flight Crew.Type : 205
ASRS Report Number.Accession Number : 1129070
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : Taxi
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

New Integration procedures and changes to SOP. I was Captain on this Airbus 320 Equipment. This fall 2013, the Company instituted new Phase SOP and Checklist changes dramatically changing the Preflight to takeoff SOP in the Airbus 320 Fleet. I have now flown about 8 legs under the new system and feel compelled to report my observations. I went into the changes with an open mind completing the computer based training early and reviewing the documents describing the changes. What I have found upon flying the new procedures for about 2 weeks on the line is a set of changes that I feel degrade the safety of the operation and load up the pilots in critical pushback and taxi time periods. The new procedures take much of the regular planning and briefing items and shift them to taxi out taking the first officer to a heads down workload at some of the busiest times on taxi out. I have tried several different methods to try to make the new system work for me, but whether it is on single engine taxi out, or two engines, whether we get the weights before taxi or later in the timeline, it still loads us up. It seems by the time we takeoff we have been doing checklist after checklist, and discussing items that could have clearly been taken care of at the gate. In my opinion the new system is prone to cause a mistake more likely and even more so to cause violations, or misapplied taxi instructions just due to the chaotic flow of the new procedure. 

Starting with the preflight setup it begins pushing workload by preventing loading of V Speed Data, and performance during the setup. I see no reason why we should wait to discuss planned flaps and speeds at the preflight level. Professional pilots are adaptable to change items later if needed, but why not alleviate the delay when a high percentage of the time the pilots know generally what to expect for departure? This is the first point that I feel needs to be addressed. I do not see any problem with the Before Start Checklist and its format works EXCEPT for the glaring change away from checking the speeds, checking the FLEX temp, Altitudes in the FMC at this lower workload, quieter period. To make us do all that on taxi out is a mistake. I've seen it over and over now on the 8 legs that we are loaded up going heads down during taxi out while we should be focusing on the taxi and FAA instructions. We have to modify this new procedure before someone bends metal or gets violated for missing turns. 

The next spot on the procedure I feel was not well thought out is the After Start checklist. We push the final weights loading, and setting of the flaps to a time period when we are usually in a crowded alley or taxiway with other aircraft waiting on us, but we are sitting there with the RADIOs off doing this new SOP. It doesn't work well. We should already have the data loaded, and all that discussion taken care of. It is just as bad when we go on one engine as we still are waiting and loading up all that work to the taxi out portion. It is a terrible flow and should be reconsidered. 

The next point I'd like to address is all these checklists. After Start, Delayed Engine start, Before Takeoff checklists, newly loaded up with many more points. If I am told it is streamlined then I have to disagree, I have done 8 legs now and I feel loaded up all the way to the runway. It's not a good system. I am very senior in my equipment and although newly trained in this fleet most of the first officers I fly with are very high time in the fleet. I do not look forward to the day when the new hires start showing up trying to integrate all this work into the taxi out. 

I'd like to add another point here. I have to wait on the flaps setting in order to do my control check. I do not like to be taxiing around getting FAA clearances and moving while doing that check. The new procedure almost forces me to do that. It is not smart and I do not feel comfortable with the change making us wait on the flaps until we are taxiing, forcing the control check at that time which pushes me to do that critical check while trying to navigate at the airport. I haven't decided how I'm going to address this but needless to say the new SOP is not good. I could stop on the taxiway, but is this really necessary? This is much safer to be completed at the gate area as we set the flaps to our planned setting and do the control check there. Just a few seconds and its done, and we don't have to divert attention on taxi out. I'm not resistant to change and I went through the entire gambit of changes on the 757 before moving to the Airbus but I can say the new changes in this Phase are mostly change for the sake of saying we did it. It is not a well thought out SOP. Please revisit this new Phase and fix it (UNLOAD us on TAXI OUT) before someone bends metal.

Synopsis

An A320 Captain described his company's new Training Phase implementation and its negative safety impact on taxi out workload because of increased checklists, distractions with tasks better completed at the gate, heads down operation and single pilot taxi at busy airports.

ACN: 1128825 (1920 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 11000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : BULLZ
Airspace.Class E : ZAU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 145 ER/LR
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZAU

Person

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
ASRS Report Number.Accession Number : 1128825
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

RE the BULLZ RNAV STAR into ORD.

If an aircraft is cleared to descend via the BULLZ arrival that aircraft will descend and reduce speed at specified points of the arrival. The BULLZ has aircraft cross TEDDY intersection at 11,000 feet at 250 knots.

Both aircraft in this report were assigned 300 knots. Both aircraft were cleared direct TEDDY intersection. Both aircraft were issued a descent clearance to 15,000 feet. Both aircraft were then cleared to cross TEDDY at 11,000 feet. Both aircraft reduced speed to 250 knots at TEDDY without clearance to do so.

I recommend speed restrictions be removed from the published BULLZ arrival. 

70% of the time ORD is on a westerly flow and Approach Control requires the aircraft on the BULLZ arrival to cross TEDDY at 300 knots. This often causes confusion for the pilots and sometimes results in an aircraft not performing as directed or expected. Constantly having to restate or clarify clearances causes an increase in complexity and a decrease in safety and efficiency.

Synopsis

An ARTCC Controller expressed consternation at the failures of some flight crews to fail to maintain their assigned airspeed of 300 knots at TEDDY waypoint on the BULLZ RNAV STAR, instead slowing to 250 knots as published on the arrival.

ACN: 1128333 (1921 of 2908)

Time / Day

Date : 201311
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Distribution
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1128333
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cruise flight at FL350 about 180 NM west of [a major airport], got "ELEC AC BUS 1 FAULT" on ECAM. Lost approximately 24 systems associated with AC Bus 1. First Officer took over controls as pilot flying maintained stable flight as we lost both autopilots and autothrust. I carried out our Non-Normal Procedures as per Flight Operations Manual, A320 Pilot Hand Book, and QRH, conducting ECAM actions. ECAM actions guided us to a point where we were to reset Number 1 Generator to restore AC BUS 1; "one attempt only, if no reset then select GEN 1 off"; which was the case. After completing ECAM, and ECAM follow-up in QRH, it was apparent that we were not going to restore AC BUS 1.

After reviewing the failures which included Thrust Reverser 1, Spoiler 3, and Brake System 1 associated with loss of Blue Electric Pump and Blue Hydraulic System, I decided that as a precaution we should not attempt a landing but should, instead, divert to an airport with a longer runway. I contacted Dispatch and he agreed that we divert to a nearby airport as it was closer to our "Present Position", which we had been considering. 

We then advised ATC that we were diverting due to aircraft systems lost to an electrical failure that could affect our required landing distance and I wanted a longer runway than at [the nearest airport]. I then advised the flight attendants of the situation and, as we had lost the cabin PA function from the flight deck, asked them to inform the passengers. 

We were given expeditious handling by ATC and I decided it unnecessary to declare an emergency as we had full control of the airplane and all crucial systems were operating normally except the automation. The left Thrust Reverser was inoperative, but had the right; spoiler three was inoperative as well but we had considered these failures when we selected our diversion airport and runway. Weather in the area was clear VMC. 

Upon contacting Approach I decided it would be prudent to have the fire trucks called out. The Controller said that we would have to "declare an emergency" to have the crash trucks out. After a few seconds of contemplation on my part the Controller said "we declared an emergency for you" and they deployed the fire trucks along our landing runway. I did not at any time declare an emergency. 

The only event that frustrated our course of action was the loss of communications with operations due to loss of ACARS. I could have gone over to commercially provided radio but decided I needed to get back with the First Officer who was flying the airplane manually and handing ATC communications. I wanted evaluate our fuel situation and make the decision to start heading to a suitable diversion airport. I decided that letting ATC know our intentions, would get back to Operations and we could deal with our primary duty of getting the airplane to a safe landing. In range of our diversion station I called operations and informed them of our ETA.

The landing was uneventful. Maintenance met the aircraft at the gate.

Synopsis

An A320 flight crew enroute suffered the loss of AC BUS 1 and elected to divert to an airport with longer runways due to the resulting partial loss of spoilers and reverse thrust. ATC declared an emergency for them when they requested CFR as a precaution.

ACN: 1127024 (1922 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Data Processing
Aircraft Reference : X

Component : 2

Aircraft Component : Flight Data Recording
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1127024
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Were Passengers Involved In Event : N

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

On an A320 aircraft, I installed a Data Loading Routing Box (DLRB) Reference AMM 31-38-34. It has been brought to my attention that the correct software was not installed when unit was installed. I don't remember reading that software was to be installed at that time. [Suggest] double check paperwork and check revision dates and check IPC to see if software is to be loaded in units.

Synopsis

An Aircraft Maintenance Technician (AMT) was informed correct software had not installed when he replaced a Data Loading Routing Unit (DLRB) on an A320 aircraft.

ACN: 1126294 (1923 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 39000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1300
ASRS Report Number.Accession Number : 1126294

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4300
ASRS Report Number.Accession Number : 1126299

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout we got an ENG 1 Compressor Vane ECAM. We followed all ECAM steps and referred to the QRH. All engine parameters were normal or within limits. Engine 1 FF, EGT, and N2 were a little higher than Engine 2 but well within limits. At top of descent, the First Officer retarded the throttles and the number 1 engine stalled, the EGT went up to approximately 835 degrees, and we got an ENG 1 Fail ECAM. Followed all ECAM steps, referred to the QRH and shutdown Engine 1. Declared an emergency with ATC, Dispatch, and briefed the flight attendants. We prepped the cabin for an emergency landing but did not issue a "Brace" signal. First Officer had just completed his PC so I had him continue flying and accomplish the landing. Landed uneventfully, had the fire department inspect the engine, then taxied to the gate.

Narrative: 2

[Narrative 2 had no additional information]

Synopsis

A320 flight crew reported ENG #1 compressor vane ECAM after takeoff, with no other issues they continued to destination. At top of descent the #1 engine rolled back and showed a high EGT, so the engine was shut down and non-eventful single engine landing ensued.

ACN: 1125547 (1924 of 2908)

Time / Day

Date : 201310

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1125547
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13300
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 7521
ASRS Report Number.Accession Number : 1125550
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Setting the lowest published altitude on an arrival/approach makes for a very nice descent when all is going well. ATC provided many speed assignments to "keep our speed up." While transitioning from a managed descent to the approach mode, we briefly failed to observe the subtle changes to the FMA's that indicated a mode reversion to our VNAV profile. It would be helpful to have a more noticeable and audible alert when a reversion occurs.

Narrative: 2

Approaching FUELR on approach into LAX having transitioned to 25L approach from River 2 arrival, crew did not recognize in a timely fashion a reversion from managed descent mode. Pilot Monitoring alerted Pilot Flying descending through restriction but with rate of descent final level off occurred 500 feet below 7,000+ restriction. Managed descent was re-engaged and next restriction, 6,000+ was made.

Synopsis

A320 flight crew reports descending below the FUELR crossing restriction on the ILS 25L approach to LAX when the aircraft transitions to approach mode from LNAV/VNAV without the crew noticing. The crew detects the error and the remainder of the restrictions are met.

ACN: 1125361 (1925 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cargo Pit Compartment
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1125361
Human Factors : Troubleshooting
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Gate Agent / CSR
When Detected : Pre-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The Mechanic at our departure airport delivered a paper copy of the MEL along with a diagram of which section of the cargo compartment that was inop. He said we could use CP4 for cargo but the damaged section could not be used. The First Officer and I along with the Mechanic discussed the MEL 25.50-03 and we continued on our way. We arrived at our destination and the confusion began regarding the MEL because cargo was in CP4 but not in the inop section. We verified this with a Station Ramp Agent. I called and spoke to Dispatch and Maintenance Control as we began to load the aircraft for our next flight. Maintenance Control said the MEL restricts us from using CP4 (rear cargo compartment) all together. The bags for our next flight were loaded into CP1 and we continued on our way.

I believe the MEL wording should be changed to read: The rear or front cargo compartment must not be used for any luggage. Our load sheet is broken down into zones and the MEL refers to cargo compartments. We were under the assumption that cargo could be in section CP4 but not CP3. Our cargo load sheet was accurate and listed all cargo in CP4 and CP3 (inop section) was empty.

Synopsis

An A320 had a MEL referencing CP3 cargo compartment damage, but there was confusion over whether cargo could be loaded in CP4 and only the damaged area excluded. It was determined the MEL restricted any cargo carriage in that compartment.

ACN: 1125317 (1926 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 198
Experience.Flight Crew.Type : 10200
ASRS Report Number.Accession Number : 1125317
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 25
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1125322
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Detector.Person : Passenger
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On the short leg climbing with the autopilot engaged, as Pilot Flying, I had just completed briefing the arrival and ILS XX approach and my Captain, Pilot Monitoring, began the Approach Descent Checklist. Coincidently, the aircraft began leveling at FL290 and the autothrottles began retarding from climb to cruise thrust. Two loud pops were heard with momentary ECAM displays of ENG1 stall and ignition-on. The Pilot Monitoring announced ECAM and split cockpit duties as the warnings extinguished. He checked engine parameters and found that they indicated normal. 

The Lead Flight Attendant called via interphone to ask about the source of the noise. The Pilot Monitoring explained the situation to the Lead and followed up with a PA to explain the source of the noise to concerned passengers. Following the PA the Lead called again and announced that the left engine was on fire. The Pilot Monitoring asked the Lead to confirm the fire visually as there were no indications in the cockpit and the condition had evidently been reported by passengers. The Pilot Monitoring completed the Approach Descent Checklist and pushed the ECAM recall button which returned a NORMAL. The Lead called back reporting that there was no fire, just flashes of fire associated with the loud pops. The Pilot Monitoring directed the Lead to station a crew member in position to monitor engine 1 visually and provide reliable information. 

As, Pilot Flying, I continued with radio duties as the Pilot Monitoring looked up and sent the appropriate Maintenance code. Duties were then returned to normal and shortly after Center directed a descent to FL200. I pushed the altitude knob to begin a managed descent and as the autothrottles reduced thrust another loud pop was heard with ENG1 Compressor Stall ECAM. The Pilot Monitoring split the duties in the cockpit and proceeded with the ECAM checklist. Per the Pilot Monitoring's direction, as Pilot Flying, I matched throttle position to EPR indication, disengaged the autothrottles and retarded no. 1 throttle to flight idle. The ECAM disappeared and the engine returned to normal indications. Satisfied the condition of the no. 1 engine was stable, the Pilot Monitoring, declared an emergency reporting, souls on board, endurance and asked for CFR support at our destination. The Pilot Monitoring made a PA followed by another call from the Lead reporting an engine fire. When asked to verify the report the Lead indicated that the stationed crewmember confirmed no current visible evidence of fire. The Captain, Pilot Monitoring, briefed the Lead to expect a normal landing using a TEST briefing. 

As Pilot Flying, I communicated our desire for an ILS guided visual approach and complied with vectors and descent guidance as the Pilot Monitoring communicated our situation via ACARS to Dispatch. Passing approximately 16,000 feet, the Pilot Monitoring returned cockpit duties to normal, was told by Approach to expect ILS 25R and immediately began reprogramming the MCDU for the new runway. As Pilot Flying, I continued to fly the airplane and the Pilot Monitoring briefed the new approach. Approach, landing and rollout were uneventful. CFR followed us to the gate to assure no further support was necessary. The Captain called the Maintenance Engine Controller to report the incident and indications. Cursory post-flight inspection of visible engine components revealed no damage. After leaving the aircraft, the Captain also received a phone call from the FODM to recap the evening's events.

Narrative: 2

I gave the aircraft and radios to the Pilot Flying and was preparing to do the ECAM procedures when the engine stabilized, the ECAM went away, autoignition went off and all engine parameters were normal. We kept the cockpit split and I took the first call from the Purser and he told me the engine was "on fire" and I asked him, "Still?" because we had no indications of fire in the cockpit. I asked him to go back and make sure. During this time I asked for the Maintenance Code book and started to write up the malfunction and what we had observed when the Purser called back to tell me that the passengers on the left side of the plane and behind the wing had seen flames shoot out of the engine at the time of the loud bangs but there was no fire at this time. The Pilot Flying suggested having a flight attendant keep watch for the duration of the flight instead of relying on a passenger and I made this request which was acknowledged. 

I completed entering the maintenance write up but noted on the ACARS response that the code I used indicated that we had shut the engine down which was not the case. I finished the Approach Descent Checklist and was about to correct the mistake when we received a clearance to descend to FL200. At that time I felt we should focus on flying and we went back to Pilot Flying, Pilot Monitoring SOP and put the cockpit back to "normal" figuring I'd correct the Maintenance reporting gaff in descent. I set FL200 in the altitude window, we verified and the Pilot Flying pushed the altitude select button to begin a managed descent. When the autothrottle pulled the thrust back we immediately got another single loud bang, large EPR swings on engine 1, autoignition and the ECAM. 

Again, we went into standard ECAM procedures and split the cockpit duties and just as I was having a look at the ECAM procedure it all went away at the stabilized reduced trust for the managed descent. One of the things I had noticed on the procedure was to turn off the autothrust and reduce thrust. The Pilot Flying turned off the autothrust in an air manlike manner and gently pulled the engine 1 thrust lever to idle with no further compressor stalls. While he was doing that I fielded another volley of calls with the Purser exactly like the first series only this time I gave him TEST and told him we were declaring an emergency, proceeding to our destination and to expect a normal landing with equipment rolling next to us. We discussed the decision to continue as the best course of action and to leave channel nine on with a PA com to the passengers. 

I declared an emergency with Center, souls and 1:20 fuel (6200+ lbs) with a request for equipment as precautionary and that we expected a normal landing, rollout and to exit the runway on our own. We were cleared direct the airport and I made a PA to the passengers that covered all of the above and when we would be landing. At this time, we stayed split cockpit and I notified Dispatch via ACARS of all of the above and corrected the MRM mistake by telling him that we had a DEGRADED engine and that it WAS NOT shut down which was acknowledged by "COPY ALL." We also made sure our QRH's were handy and contemplated the possibility of an evacuation. We put the cockpit back together and continued our descent with engine 1 at idle and all parameters normal. When we transitioned through FL180 we were switched to Approach and were given the ILS and we asked to be vectored onto the approach as opposed to taking a visual for terrain considerations. We were downwind and south of the field. We split the cockpit one more time and I installed the approach and briefed the Pilot Flying from my plate and handed it to him for review and briefly took the airplane. When we were both satisfied with our situational awareness we went back to Pilot Flying, Pilot Monitoring SOP. 

The remaining approach and landing with engine 1 at idle thrust were completely normal and we asked for a fire truck to follow us to the gate which he did. We post flighted the number 1 engine and saw no outward signs of damage and I called the Engine Controller to give him our perspective. The Maintenance Manager called to close the loop and we proceeded to the layover hotel.

Synopsis

An A320 left engine experienced two compressor stalls at cruise: first when thrust was reduced to set cruise power and when reduced again to begin the descent. The QRH was completed, an emergency declared and the flight continued to its filed destination with both engines operating.

ACN: 1124089 (1927 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Compressor Bleed Valve
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pneumatic System Control
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1124089
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Training / Qualification
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

I was assigned to work an MEL on an A320 aircraft and I was supposed to change the High Pressure (HP) 10th stage bleed valve closure solenoid (4029ks). Instead I changed the HP 10th stage solenoid (4023ks), not realizing I changed the wrong solenoid because the terminology confused me. See Aircraft Maintenance Manual (AMM) figures [drawings] in Chapters 75-23-53 and Troubleshooting Manual (TSM) 36-11-00. I was working at night, outside on Pad-5 on my third night. At the same time, I was assisting a Mechanic working a 2.5 bleed actuator on the opposite motor and may have been distracted. I had changed the wrong part and re-deferred the MEL. Two nights later, another Mechanic was assigned the same job to fix the MEL, but could not locate the part in stock that I had installed incorrectly. After researching, that Mechanic found the part installed in the incorrect position. Distraction and confusion between AMM figures in 75-32-53 and TSM 36-11-00, the MEL was cleared during the second visit.

Synopsis

An Aircraft Maintenance Technician (AMT) reports he mistakenly changed the High Pressure 10th stage solenoid (4013ks) instead of the High Pressure (HP) 10th stage bleed valve closure solenoid (4029ks) on an A320 aircraft engine. Distraction with helping another Mechanic and confusion with the drawings in the Aircraft Maintenance Manual (AMM) and Troubleshooting Manual (TSM) were noted as contributors.

ACN: 1123959 (1928 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 21800
Experience.Flight Crew.Last 90 Days : 125
Experience.Flight Crew.Type : 4200
ASRS Report Number.Accession Number : 1123959
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

While cruising at FL360, [we had the] Number 1 FMGC failed. Approximately 15 minutes later, the Number 2 FMGC failed while we were running through the QRH in accordance with Maintenance's directions. Number 1 FMGC failed to reset after performing the Circuit Breaker reset procedures and it made the no FMGC condition worst while following the QRH. Number 2 was still having NAV engaged and Number 2 autopilot still engaged. We advised Maintenance and elected not to continue with the reset till we are closer to our destination airport for fear of losing Number 2 System completely with no Auto Flight capabilities. (We did reset Number 2 FMGC as we were less than a 100 NM from our filed destination in accordance with the QRH and Number 2 failed to reset also.) 

[We] notified ATC of Unable RVSM as well as Unable RNAV. [We] descended to FL280 per ATC instructions. [We] reverted to Standby NAV remainder of the flight. [We] advised Dispatch of our problems. Uneventful approach and landing Runway 25L.

Synopsis

Both FMGCs on an A320 failed in flight and would not completely reset so the flight descended out of RVSM and continued to their destination in Standby NAV.

ACN: 1123368 (1929 of 2908)

Time / Day

Date : 201310

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 211
ASRS Report Number.Accession Number : 1123368

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1124192
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

About three minutes into the taxi out phase, the lighting, instruments, door lock, First Officer's CRT's generator transfer and number 2 bus all started to "go nuts." The lights were blinking on and off; the taxi light was going on and off. The door was locking and unlocking. My CRT's were flashing on and off and ECAM's were blinking on and immediately going off. The generator was clicking on and off, transferring back and forth to the APU [#1] generator. Lighting in the cabin was flashing with each transfer and they were transferring every couple of seconds. My (com 2) radio was unreadable and at times had a constant tone like a carrier only with no voice capability on the frequency. It all seemed to be associated with the #2 generator and its transferring issues. It would connect, and then trip off while the transfer appeared to disrupt momentarily all the items associated with the number two generator.

The Captain taxied to the de-ice area and set the brakes to work the issue and call Maintenance Control. I got out the QRH but no ECAM stayed on the screen long enough to try to run either an ECAM or a checklist. We decided to disconnect [deselect] the generator and run on one generator and then started the APU to run on that. By then my instrument CRT's and the bottom screen all had slashes across them and did not come back until we were almost at the gate after taxi back. NOTE: the Maintenance person advised after a reset like ours it takes quite a while to run the self-tests and get back on line.

We were able to stabilize the situation with the #2 generator off and the APU on (although I believe just turning off the #2 generator would have eventually cleared it up). I feared that we had permanently damaged some electrical items with the rapid and incomplete power transfers.

Narrative: 2

We started both engines and with taxi clearance began to taxi. The First Officer, as part of his flows, turned the APU off. Approximately 1-2 minutes later, we started getting multiple ECAM messages to include SEC 2, ELAC 2 faults along with what sounded like generator relay switching. (The ECAM messages would constantly cycle through the faults). The taxi light, cabin and cockpit lights would intermittently go on and off. Both the First Officer's ND and PFD along with the lower ECAM displays turned to diagonal lines. We stopped the aircraft and eventually turned the #2 GEN off. This stabilized the flashing lights and ECAM. We started the APU and put the APU GEN on line. This was verified by selecting the ELEC page on the ECAM. The First Officer's ND and PFD along with the lower ECAM still displayed a diagonal line. Once stopped the lead Flight Attendant called to ask what was going on due to the flashing cabin lights. I made a PA announcement informing the passengers that we had an electrical problem and have stabilized the problem. We contacted Maintenance Control via phone and coordinated a return to the gate (this took about 20 minutes). The taxi back to the gate was uneventful. We parked the aircraft and shut down both engines. External power was applied and the APU was shut down. After the APU was shut down and on external power, the First Officer's ND and PFD along with the lower ECAM returned to their normal display.

Synopsis

A320 flight crew describes an electrical anomaly during taxi out for takeoff, apparently caused by the number two generator connecting and disconnecting from AC Bus 2 rapidly. Eventually the number two generator is turned off and the APU started stabilizing the electrical system before returning to the gate.

ACN: 1123250 (1930 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : EWR
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : EWR

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : EWR
Make Model Name : Helicopter
Crew Size.Number Of Crew : 1
Airspace.Class B : EWR

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11400
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1123250

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 11500
ASRS Report Number.Accession Number : 1123230

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were informed that there was a helicopter that had us in sight and was maintaining separation. Shortly there after, I got a TCAS RA to adjust vertical speed at 1,000 FT MSL on approach to [Runway] 22L at EWR in darkness. I saw 200 FT separation on the TCAS altitude. After telling Tower about TCAS Climb, I got "clear of conflict" and was in a safe position to continue to land. Landing was uneventful.

Narrative: 2

We NEVER saw the helicopter. VERY unsafe. When we informed Tower of our conflict he was not concerned. After landing Tower had NO comment on the conflict that had transpired. We expressed our concerns over the conflict again with Tower once we were on the ground. Tower had no comment. Very unsafe practice.

Synopsis

A320 flight crew reported a NMAC with a news helicopter on a night visual approach to EWR. Crew mentioned they were unhappy with the Tower's handling of the event.

ACN: 1122807 (1931 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2200
ASRS Report Number.Accession Number : 1122807
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 2800
ASRS Report Number.Accession Number : 1122818
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were cruising at FL320 at night, IFR, with the First Officer flying. We started getting multiple ECAMs and the First Officer told me to take the aircraft as he had lost all flight instruments and displays. I took control and at about the same time the autopilot disengaged with a Master Warning, auto-thrust disengaged leaving THR LK on my FMA, #2 EPR gage in yellow with X's. The cockpit lights were flashing on and off and there was a loud clicking towards the back of the cockpit which sounded like the solenoid on the door lock. We immediately started the APU. We had ECAMs for #2 EPR mode inop, ELAC 2 inop, SEC 2 inop, SEC 3 inop, FAC 2 inop, Cab Sys 1 inop, cargo smoke detect fault inop, SDCU detect inop, the ECAM lower SD system display was blank except for a diagonal line, and overhead on the ADR panel the "ON BAT" light was illuminated. Additionally we lost communications with the cabin crew. They were calling us with their call button and pounding on the cockpit door but we couldn't respond immediately as the interphone was inop. I flew the aircraft while the First Officer diligently worked the ECAMs in sequence but we kept getting repeat ECAMs and it became apparent the FWC was taking us in endless circles and there must be a bigger problem the FWC is not picking up. 

At this point it seemed obvious we had major problems and should land as soon as possible. I declared an emergency with Center and started a descent. Next we pulled up the system pages and noticed the #2 Gen was showing red with volts about 80 and freqs about 120. Evidently the GCU for the #2 gen was not forcing a protection trip and we never got a gen fault light or ECAM for such. With the APU running we elected to deselect the #2 gen allowing the APU gen to pick up A/C bus 2 rather than using gen #1 possibly contaminating A/C bus 1 and our only 2 remaining flight control computers. This seemed to slowly clear up most of our problems. Still, the First Officer had no ND, the lower SD system display was still inop and we still had 4 flight control computers faulted. We were concerned about the 4 inop flight control computers so we decided to slowly and carefully reset each one at a time. We got all flight control computers back on line. With the aircraft under control we briefed the flight attendants (TEST) and passengers on our divert and then contacted Dispatch connecting with Maintenance Control as well. Maintenance advised he had been receiving a large volume of downloaded faults from our aircraft and we should land as soon as possible. We accomplished the divert items listed in the FOM and Dispatch sent us weights for landing. ATC was very accommodating and we made an uneventful overweight landing per checklist. Emergency equipment was standing by but not needed. After landing we debriefed with everyone and elected to take another Airbus on to destination 2 hours later. The next leg was uneventful.

Narrative: 2

In cruise at FL320, multiple ECAMs starting appearing, cockpit lighting was flashing, door lock clicking, AP and AT disconnected. First Officer PFD and ND as well as lower eng display blanked out. I handed Pilot Flying duties to the Captain as I had no instrumentation. Main ECAMs were ELAC2/SEC2+3/FAC2, cargo smoke fault, cab sys 1 fault, IRU on bat light was on. ECAMs were cycling through with the aforementioned ELAC/SEC/FAC fault lights flashing. Cabin interphone was inop. Both engine EPR gauges showed XX. Attempted to run ECAM however it was cycling fast so was hard to do. Followed ECAM which had me reset ELAC/SEC/FAC, no help. Both engines were put, per checklist into N1 mode. #2 engine did not seem to respond well to throttle inputs. Captain declared the emergency. We started APU but did not help the problem. I looked at the ELEC page and saw that GEN2 showed red and HZ/V seemed abnormal.
I showed the Captain and I suggested that we disconnect the gen, he agreed. As soon as the gen was disconnected the light show and multiple ECAMs stopped. I contacted Dispatch and Maintenance Control, we agreed that diverting was the safest course of action and diverted. Accomplished the overweight landing checklist as we were at 148,000 lbs, uneventful landing. Fire trucks met us, no further action was required.

Synopsis

A320 flight crew experiences loss of the First Officer's flight displays along with autopilot and autothrust disconnects in night IMC at FL320. ECAMS were displayed for #2 EPR mode inop, ELAC 2 inop, SEC 2 inop, SEC 3 inop, FAC 2 inop, Cab Sys 1 inop, cargo smoke detect fault inop, SDCU detect inop, the ECAM lower SD system display was blank. The APU is started and the number two generator is deselected allowing most systems to return to normal. Flight diverts to a suitable airport.

ACN: 1122806 (1932 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : L30.TRACON
State Reference : NV
Altitude.MSL.Single Value : 4800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : LAS

Component

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16620
Experience.Flight Crew.Last 90 Days : 208
Experience.Flight Crew.Type : 6653
ASRS Report Number.Accession Number : 1122806

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1122817

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were offered a visual to Runway 1R instead of 25L as winds were from the north at 15 knots. We accepted vectors for the 1R visual. The First Officer disconnected the autopilot somewhere south of the airport in a dogleg turn about 25-30 south. We reported the airport in sight just prior to that point and were cleared for a visual to 1R without other airspeed or altitude clearance. We were at 7,000 feet and 210 knots. I then put 3,500 feet in the altitude window as the First Officer had a reference point in the FMGC to the runway giving us a 3 degree glideslope for reference. I had the 1R RNAV approach plate in front of me and saw that we should have been at 5,000 feet at 10 NM. We were on that profile inside of 15 NM and I spoke with the Tower during this time and noticed we were going to cross the 10 nm point just below 5,000 feet. Before I started to mention it, we received an EGPWS warning "TERRAIN, TERRAIN, PULL UP, PULL UP". It lasted about 2-3 seconds. I believe we reached 4,800 feet or slightly above at this time at about 10.5 nm. 

The First Officer began to start a climb and as we were at level and he put his hands on the levers, the warning stopped after one cycle. There was some sparse green terrain on our displays the entire time and we could visually see our position to it. We stayed at that altitude a few seconds and then continued the approach without further incident to a normal landing. We could not understand the sudden warning as our training always demonstrates this system to work by first advising of a potential conflict and then strengthening in warning until finally commanding a pull up. We got no advisories or other preliminary warnings-just the very sudden pull up, and just as sudden ending of it. There was no corresponding red display as we have seen in the simulator with the aural command. At no time did ATC mention anything about our altitude.

Narrative: 2

Both the Captain and I operate into LAS frequently and are very familiar with terrain restrictions. We both had "Terrain" selected on our ND's per SOP. At no time did we lose sight of the airport or surrounding terrain.

Synopsis

A320 flight crew reported receiving a "Terrain, pull up" warning from the EGPWS on a visual approach to LAS RW 1R that appeared to be inaccurate.

ACN: 1122804 (1933 of 2908)

Time / Day

Date : 201310

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 1200

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 16600
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 11000
ASRS Report Number.Accession Number : 1122804

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We contacted NorCal Approach descending into SFO. We were assigned the Quiet Bridge Visual to 28L. Since this approach is not in the database we asked for the FMS RNAV approach. NorCal said we could fly this but it would have to be to 28L since 28R was closed. We were high at the start of the approach but quickly got closer to the lowest altitudes on the RNAV approach. After descending to the lowest altitude on the approach, about 1,200 FT over the bridge, the Captain switched our approach to the 28L ILS to give me backup navigation since it was night time VFR. At that point the FD kept showing a descent as I was trying to level off at 1,200 FT until the last point on the RNAV (approximately over the bridge). The autothrottles were not adding power even though I was pulling back on the stick trying to level off. This resulted in us descending slightly below the last altitude of 1180 as I was trying to move over to the 28L. At that point I got a "glideslope" alarm from the aircraft (GPWS). The FD was trying to get me to go lower even though the VASI was showing that I was below the glideslope already. I leveled off and caught both the localizer and glideslope. We landed with no further incident.

Synopsis

A320 First Officer reported several autoflight anomalies on a night visual approach to SFO that, if uncorrected, would have taken the flight into a CFIT situation.

ACN: 1122731 (1934 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZID.ARTCC
State Reference : IN
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZID
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZID

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZID
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZID

Person : 1

Reference : 1
Location Of Person.Facility : ZID.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1122731

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1123252

Person : 3

Reference : 3
Location Of Person.Facility : ZID.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1122993
Human Factors : Situational Awareness

Person : 4

Reference : 4
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1123247

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working a heavy amount of traffic, but the complexity was low. I handed Air Carrier X off to Sector 76 level at FL340, Air Carrier X had no issues. As Air Carrier X was entering Sector 76 I took a handoff on Air Carrier Y level at FL350. Air Carrier Y had no request and I left him on his route. At the point that Air Carrier X was 10 miles into Sector 76 and Air Carrier Y was around 20 to 15 miles still within Sector 76 I noticed the conflict alert activate. Air Carrier X showed climbing through FL343 while I was talking to Air Carrier Y at FL350 only 7 miles apart head to head. Air Carrier Y immediately reported that he may have to respond to a TCAS from the aircraft climbing in front of him. I responded "Air Carrier Y Roger, turn 20 degrees left vectors for traffic." I prevented the targets from merging; it was to late prevent the loss of separation. Air Carrier Y never responded to an RA and remained level at FL350. I was told that the aircraft came within 3.5 miles and 400 FT. I think this was a simple case of missed traffic. I do not have any procedural recommendations to prevent this situation.

Narrative: 2

We were Air Carrier X in cruise at FL340 south of CVG. The Captain was pilot flying and I was the pilot not flying. The Controller issued a clearance to turn left 15 degrees. We assumed this was for traffic, so the Captain asked me to ask the Controller if we could climb to a different altitude. So, on my reply to the Controller I requested a different altitude for which she issued a clearance to climb to FL360. Shortly after the Captain initiated the climb we received a TCAS TA (traffic, traffic). At that time I decreased the range on my ND to more easily see where the traffic was. The traffic was above and to our right, and soon it became apparent that a conflict was imminent. The Captain asked me to query the Controller. When I began to do so, we received a TCAS RA (descend, descend) so I reported a TCAS descent to the Controller. She then issued a 30 degree left turn. The traffic passed approximately 400 FT above and to our right. I don't know the lateral distance as I was busy setting the new heading and verifying our vertical speed remained outside the avoid area. After the traffic passed we were cleared back on course.

Narrative: 3

I was working radar at Sector 76. [This] was combined with Sector 66 and the northern portion of Sector 70 which owned FL240 and above. There was no D-side or Supervisor in the area as it is a time of evening in which staffing is minimal. The workload had been pretty steady and constant. Prior to the event with Air Carrier X, Air Carrier X westbound was in a potential conflict with another westbound aircraft at FL340. I had told Air Carrier X to turn 15 degrees right, however the pilot responded with asking whether an altitude change would work. I am not even sure whether or not the aircraft turned. I asked the aircraft what altitude they preferred and they said FL360. I did not observe any potential traffic for them in my scan and issued the climb. During Air Carrier X's climb from FL340 to FL360, Air Carrier X alerted me that they were responding to an RA and descending. I had overlooked Air Carrier Y who was eastbound at FL350 and Air Carrier Y had already been shipped to Sector 93. I am not sure whether Air Carrier Y responded to an RA as well. In addition to Air Carrier X descending I turned them 30 degrees left upon realization of the situation. I was unable to call the OMIC and alert them to the separation loss as work stayed steady and I was shaken up. It was quite a few minutes before the night Supervisor came down from the OMIC desk and asked about a potential RA and I informed him that I had lost separation.

There are a few things I would recommend to prevent this occurrence from happening again. For myself I would recommend working on my scan. I still am not sure how I overlooked the most important and closest piece traffic. I would also recommend that better judgment is used as to the type of workloads we often work with the shift from evening personal to mid shift operations. I feel that during the hours that we have a supervisor or CIC physically in the area, that the type of complexity on the sector would normally have had a D-side. And although I overlooked traffic in my scan, having a D-side may have been able to alert me to my action may have helped minimize the loss, or prevented it.

Narrative: 4

While on course, straight and level flight, autopilot on, at FL340, 30 miles south of CVG, the ATC Controller instructed our flight to turn left 15 degrees off our present course, which we presumed was for traffic. We acknowledged the clearance and asked the ATC Controller if we could climb to FL360 instead. The ATC Controller concurred and instructed our flight to climb to FL360. As our flight started the climb to FL360, we received a TCAS TA indicating that our climb clearance was going to cause a traffic conflict with an aircraft above and to our right so the Captain asked the First Officer to query the Controller. Before the First Officer could query the Controller on the conflict we received TCAS RA descent guidance. The Captain performed the TCAS RA descent maneuver while the First Officer informed ATC of our TCAS RA descent. The ATC Controller then instructed the flight to turn left while we were following the TCAS RA descent guidance. We complied with both the ATC Controller turn instructions and the TCAS RA descent guidance until we were clear of the traffic conflict. The Captain then queried the ATC Controller concerning her climb clearance, which brought our flight within 400 FT vertically of the other aircraft. The ATC Controller's response indicated that she had missed the potential traffic conflict. The ATC Controller then cleared the flight back on course and we proceeded without further incident.

Synopsis

Two controllers and a flight crew involved describe a TCAS RA event at FL345 in ZID airspace.

ACN: 1122145 (1935 of 2908)

Time / Day

Date : 201310

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Emergency Floor Lighting
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1122145
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

A320 aircraft Service Check, Emergency Light Check failed. Emergency Power Supply Unit (EPSU) unit aft left. Emergency lights at [seat] rows 22C-24C, 28-30, 35-37 not working. Found wires and lights 'J' light and 'B' swapped at connector 13WL ESPU (Emergency Supply Power Unit). Returned wires to their correct location per Wiring Diagram Manual (WDM) 33-51-08. Also found pinched wire at splice, at seat Row 29-ABC causing Emergency Exit Lighting (EEL) light to go dim, removed seat and rerouted wire. Operational Check of EEL seat lighting OK. This is the third Emergency Light Wiring defect on an Airbus that I have come across in the last six weeks. Aircraft Out of Service. The checks are not being followed.

Synopsis

An Aircraft Maintenance Technician (AMT) reports finding a third A320 aircraft in a six week period that failed their Emergency Light Checks during a Service Check. Wires were found swapped at the Emergency Supply Power Unit (ESPU).

ACN: 1121510 (1936 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 1700

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Taxi
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Route In Use : Vectors

Aircraft : 2

Reference : Z
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 900 (CRJ900)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Other / Unknown
ASRS Report Number.Accession Number : 1121510

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working another sector when I saw this happen and I talked to the RADAR controller involved. I thought it was very very dangerous and I was shocked that it happened so that's why I'm filing a report. LAX Tower cleared an A320 routed south of the north complex runways for a departure into the Manhattan RADAR (MANR) Sector. MANR has control to turn this aircraft and climb them. About the same time maybe a 1/2 mile offset, a CRJ-900 routed north was coming off the south complex. These aircraft have to swap out. MANR was never advised about the CRJ-900 and even though the CRJ-900 was instructed to maintain visual separation, MANR had no idea who that aircraft was. Ninety Five percent of the time a departure off the north complex is immediately issued a south bound heading. In this case the controller waited wondering who this unknown target was (CRJ-900). Had MANR done the norm, and turned the A320 south, there not only would have been an ugly loss of separation but a potential MID-AIR collision. The CRJ-900 would have freaked out if the A320 had turned. Dangerous and Scary! Recommendation, training to LAX Tower on the proper use of Visual Separation, requirements to inform Departure Sector of aircraft using visual and stopped at lower altitude.

Synopsis

SCT Controller observed a conflict event involving two air carrier aircraft departing LAX, the north complex aircraft routed south and a south complex aircraft routed north.

ACN: 1121484 (1937 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 16000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZAU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZAU
Aircraft Operator : Fractional
Make Model Name : Embraer Phenom 100
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZAU

Person

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1121484

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Everybody up the line from me failed to do their jobs and created an unsafe environment for ORD traffic. Out of the 4 arrival sectors into C90, I was the only sector with weather on my approach. I made my Supervisor aware that the small holes in the weather I was depicting were quickly closing, and inquired as to why I still had a long stream of ORD traffic coming towards my sector. I commented that the traffic should be rerouted to another arrival sector, any other arrival sector, as none of them had weather as I did. The Supervisor did absolutely nothing. My D-Side and I did the best we could to coordinate with C90, as the pilots were deviating further and further off of the arrival, getting close to departure sectors. In one instance, we coordinated two aircraft deviating east through a departure sector, and the coordination was successful with C90, as they had no departing traffic at the time. While I was coordinating with C90, my D-Side, and the pilots, the Supervisor was in my ear telling me how to run my traffic, and had apparently decided that I had been correct all along and we should give the pilots last minute reroutes to another sector. I advised him that coordination had already been done with C90 for those two airplanes, and transferred communication to C90. Moments later, C90 called and put us in the hold until "we could figure out the best plan of action for our aircraft." It became quickly apparent that there was no plan. The TMU did nothing to reroute this traffic, and neither did the supervisor despite my concerns that things were going to go south very quickly. At that time, I had several ORD arrivals in my sector, all deviating around weather so I had to re-clear all the aircraft back to the arrival to hold. An A320 at the time was approximately 6 miles ahead of a Phenom 100, both aircraft going to the same fix to hold. The Phenom was level at 160, I believe. The A320, ahead of the Phenom, same direction was leaving 170 for 120. Conflict Alert was activated and at that time and I realized that the A320 had significantly reduced his speed in anticipation of entering the holding pattern, so the Phenom was closing the gap between them. I told the A320 to expedite through 150 and report leaving 150, which he did, but there may have been one hit inside the bubble, if at all. To be quite honest, I was far too busy to be overly concerned about 4.95 mile separation instead of 5 mile separation, especially since, while working all the traffic that should have not been inside my airspace in the first place, getting slammed in the hold. Vectoring and amending altitudes of every single airplane to try to get them back to the arrival holding fix, having my Supervisor give me contradictory instructions to what had already been coordinated; I also had a formation flight breakup in my airspace. Recommendation, I understand it is a weekend day, and people may be in an end of the week "lull" but we have jobs to do. The TMU should have been the first line of defense and had the foresight to reroute these airplanes further out so that they were not being vectored around the only weather in the arrival sectors and flying through departure sectors, they failed. The second line of defense should have been my Supervisor when I told him plain and simple that sending the airplanes through my airspace was not the best of ideas. It became apparent that nobody cared. I'm not sure exactly what I can recommend to rectify this type of event. You can't teach common sense.

Synopsis

ZAU Controller experienced a loss of separation event when handling heavy ORD arrival traffic was complicated due to weather deviations, the reporter claiming the Supervisor and TMU failed in their responsibilities.

ACN: 1121290 (1938 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1121290
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10350
Experience.Flight Crew.Last 90 Days : 155
Experience.Flight Crew.Type : 7073
ASRS Report Number.Accession Number : 1121575

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

The #2 Engine vibration indicated 9.9 in cruise at FL340, M.80. Vibration went away for 10-15 minutes after slowing to M.77. While discussing the event with Maintenance Control and Dispatch the #2 Engine Compressor stalled and significant aircraft yaw/vibration occurred. We reduced #2 thrust lever to idle, declared an emergency and told Dispatch we were heading to a nearby suitable airport. The First Officer took over flying and ATC duties, set up the approach, got ATIS and expeditiously descended to a 10 mile abeam position for landing on [Runway] 14L at the diversion airport. After signing off with Dispatch and Maintenance Control I ran the Engine Stall Checklist, talked to the flight attendants, and passengers, requested landing data, ran the Overweight Landing Checklist and took the controls for a hand-flown visual single engine approach and landing. 

The #2 Engine had normal indications at idle so we left it running at idle. At around 1,200 AGL, the #2 Engine temperature spiked to hot/max red (didn't see an actual temperature) but we elected to continue the landing rather than run the Overheat Checklist. Fire trucks met us at mid-field; we immediately shut down #2, and were inspected by the fire crew. No indications of fire existed so we taxied to the gate and deplaned passengers. 

The First Officer did an outstanding job of coordinating the emergency with ATC, descending the aircraft and setting us up for a timely approach and landing, and backing me up throughout the landing. Somewhere in the busyness of it all we got a "FIRE LOOP B DET FAULT" that flashed for a few moments then went away, and I believe but can't be certain that we also got a momentary Engine #2 fire detection inoperative ECAM that also went away after just a few seconds on the screen. I am curious if we had fire detection during the overheat while landing since the #2 Engine temperature got so hot with no fire indications.

Narrative: 2

[No additional information was included in the secondary narrative.]

Synopsis

The flight crew of an A320 diverted to the nearest suitable airport following receipt of high vibration indications and compressor stalls from the right engine.

ACN: 1120649 (1939 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1120649

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1121093
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We were doing a visual approach and the weather was clear and there were light winds. I was the Pilot Flying. We were landing straight in and we were a little fast so I opted to extend the gear before the flaps. After the gear was deployed I called for flaps in normal sequence. We decelerated and the approach was normal. By my estimation we were going to be stable by 1,000 feet. When I called for flaps full the autothrust added a lot of power -- it felt like it was going all the way to climb power but I didn't have time to notice the actual N1. Our airspeed was increasing rapidly towards flap overspeed. Other than the autothrust behaving abnormally, all other indications were showing that we were flying a normal stable approach path so I pulled the thrust levers to idle to disengage the autothrust. With me manually controlling the thrust, the plane began to decelerate again and we were stable by 500 feet. However, in my haste to pull back the thrust levers I accidentally pulled the thrust reverse triggers and put the thrust levers into the reverse position. I noticed this almost immediately and put the thrust levers back into the idle position. The reversers did not deploy and we did not receive an ECAM message. During all this I noticed that my First Officer seemed to be spending more time than normal getting the flap handle into the right position for flaps full. After we landed I asked my First Officer if he saw anything that would've caused the unexpected thrust increase. He said that when he reached down to select flaps full, he noticed that the flap indicator showed the flaps at only 1+F rather than 3 (the flap selector was at 3). When he put the flaps to full, the indicator didn't move so he moved the flap lever back to 1+F and then quickly re-selected each setting; 1, 2, 3, and full. He said that the flaps then moved to full. I didn't see any of this because I was hand flying the plane. The plane was behaving normally as we went through the configuration changes for landing. We didn't get any ECAM warnings. The only abnormality I saw was the increase in thrust. My First Officer said the thrust increased at the exact second he picked up the flap lever to move it from 3 to full, it happened before he re-selected Flaps 1. We contacted Maintenance after the flight and had the flaps checked. They found no defects.

It is my understanding that in managed speed and approach mode the autothrust maintains S-speed when flaps are set to 1. I am pretty sure that the power increased when the flaps went from 3 back to 1. As Pilot Flying I had no idea that anything was out of the ordinary with the flaps. When my First Officer reset the flap handle he didn't tell me. If I would've known that there was an issue with the flaps I would've executed a go around. I thought we were just seeing an autothrust malfunction or possibly a mode reversion. Once I had the thrust in manual the aircraft decelerated and we were able to continue the approach normally. I didn't know there was anything abnormal about the flaps until after we landed. When we did the Before Landing Checklist I noticed the flaps were in full. This is a normal part of my scan before landing. As for pulling the thrust levers over the reverse gate -- I did that by instinct. I rarely fly with manual thrust and when I do I use the match and mash technique (as trained in my last sim session). This was the first time in the aircraft that I disengaged the autothrust by pulling the levers to idle. After 5 years of flying the Airbus it's hard to fight the habit of cracking the reverse when pulling the thrust levers from the climb detent back to idle. 

All flap lever position changes need to be communicated. Also, malfunctions on final approach need to be called out. Final approach is not the place to try to fix a malfunction through a non-standard method. Also, we need to practice flying with the autothrust off more often.

Narrative: 2

I was the Pilot Monitoring, flaps where at 3 and I verified they where at 3. Captain called for, "flaps full Landing checklist." I responded, "Speed checked flaps full Landing checklist." I lifted the flap selector handle and verified flap position, to my surprise they had self retracted to 1+F automatically. The aircraft engine spooled up to meet S speed. The Captain elected to disconnect autothrottles and fly manual due not knowing what was going on. I matched flap position 1+F and reselected each flap to full. Flaps deployed to full ok. On short final I heard a double click and realized that the Captain had turned off autothrottles and had clicked in reverse in flight. It was quickly fixed by him pushing the throttles forward of the gates and we landed soon after. Upon touchdown we received 2 ECAM messages.

The events surrounding this are not entirely known. Maintenance met the aircraft and cycled the flaps and slats to verify correct position. Ops checked ok, and ECAM messages where cleared. This aircraft had just come out of C-check and this was the 1st revenue flight for the aircraft. The Captain realized he had clicked into the reverse gates out of habit of doing that on landing, and quickly fixed the issue. I didn't catch his error because I was working on the flaps self retracting to 1+F and the power quickly coming up to increase airspeed to S speed, in addition to me trying to communicate to him the problems that where unfolding. Upon landing, on the return flight, we received the same ECAM messages we had gotten [on the earlier flight], Maintenance was notified.

Synopsis

A320 flight crew reports an unstabilized approach due to a flap anomaly. With the flaps selected to 3 and verified, the flying Captain calls for Flaps Full and the First Officer notes the flap indication to be 1+F. Flaps 1 through Full are reselected by the First Officer with the flaps responding normally. During this time autothrust responds with a large thrust increase and the Captain responds by disconnecting, with thrust levers to idle, then inadvertently selects reverse thrust. This is quickly corrected and the reversers do not deploy.

ACN: 1120118 (1940 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : CYYZ.Airport
State Reference : ON
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 7100
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1120118

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 260
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1120608

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Maintenance
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The aircraft we flew from ORD to YYZ had just come from heavy maintenance following a seat reconfiguration. We flew the leg by SOP and in accordance with the FARs and company policy. However, when we were taxiing out for our return leg, Load Planning said that we had to return to the gate as they had determined that the engineering tables in the computer didn't match the new configuration of the airplane. The Captain and I then determined that we had flown the previous leg most likely with improper weight and balance information since the engineering tables had not been updated at that time. After a 90 minute delay, the Captain, Maintenance Control, Dispatch, and Load Planning coordinated to have the proper engineering tables inserted into the computer for the new configuration. We then received the correct weights for our flight back.

Narrative: 2

THE AIRCRAFT HAD COME TO US FROM HEAVY MAINTENANCE AND SEATING CONFIGURATION CHANGE. ALL WENT AS PLANNED ON THE FIRST LEG, SO WE THOUGHT. IT WAS ON THE TAXI OUT ON THE RETURN FLIGHT THAT MY FIRST OFFICER AND I WERE TOLD THAT WE HAD TO RETURN TO THE GATE DUE TO THE FACT THAT LOAD PLANNING COULD NOT SEND US OUR FINAL WEIGHT AND BALANCE FIGURES. THEY DID NOT HAVE THE CURRENT SEATING CONFIGURATION AND ENGINEERING GRAPHS FOR OUR NEWLY CONFIGURED AIRCRAFT. WE HAD TO WAIT APPROXIMATELY 1 HOUR BEFORE WE RECEIVED THE OK FROM MAINTENANCE, A NEW DISPATCH RELEASE, AND AN ACCURATE FINAL WEIGHT MANIFEST AND WERE LEGAL FOR DISPATCH. DURING THIS PROCESS, I REALIZED THAT WE HAD DEPARTED IN THIS SAME AIRCRAFT. THERE WAS NO MENTION OF ANY ISSUES OR DISCREPANCIES ON THAT LEG BUT IT QUITE EVIDENT TO ME THAT IF WE DIDN'T HAVE THE PROPER ENGINEERING GRAPHS FOR THE SECOND LEG OUT AFTER THE AIRCRAFT'S RETURN TO SERVICE THEN WE DIDN'T HAVE THEM FOR THE FIRST LEG EITHER. MY FIRST OFFICER AND I WERE OF COURSE COMPLETELY UNAWARE OF ANY POSSIBLE ERROR, DISCREPANCY, ILLEGALITY UNTIL WE WERE BROUGHT INTO THE LOOP ON THE TAXI OUT. HAD I ANY CLUE THAT THE PROPER FINAL WEIGHTS WERE IN QUESTION, I WOULD HAVE NOT TAKEN OFF. TO THIS POINT, I'M STILL NOT SURE WHAT THE SOURCE OF THE ERROR WAS AND HOW DEEP DOES THIS "RABBIT HOLE" GO. THE LOAD PLANNER SAID THAT WE WERE LEGAL BUT HAD THE WRONG NUMBERS. HOW COULD WE BE LEGAL THEN? HOW WRONG WERE THE NUMBERS? THE AIRCRAFT HANDLED FINE ON THE ENTIRE FLIGHT. ARICRAFT TRIMMING WAS IN A NORMAL RANGE THROUGH OUT THE FLIGHT SO I DON'T SUSPECT THAT WE WERE OUTSIDE OF THE NORMAL CG ENVELOPE BUT UNTIL THE NUMBERS ARE CRUNCHED, ONE CAN'T BE SURE. SINCE WE HAD NOT PASSED OUR POINT OF DISPATCH ON THE RETURN FLIGHT, THERE IS NO REPORT FOR THAT LEG.

Synopsis

A320 flight crew reports accepting an aircraft out of heavy maintenance after a seating configuration change and flying their first leg. Prior to the next leg it is discovered that the weight and balance data has not been change to reflect the new configuration.

ACN: 1119742 (1941 of 2908)

Time / Day

Date : 201310
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Emergency Light
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Toilet Furnishing
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1119742
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise over ZZZ, Flight Attendant called to say lights in both aft lavs were flickering, [and] then went out. Emergency Exit light in aft lav was only light on, and too hot to touch. I passed information to Maintenance Controller and he suggested pulling Circuit Breaker (C/B) H7 on Captain's overhead panel that controls the three Emergency lights in three lavatories. I pulled C/B and had Flight Attendant check; hot light went out. All other operations normal, continued to destination. Maintenance Control suspected light batting [insulation].

Callback: 1

Reporter stated the aft Flight Attendant called him after she started noticing an odor that smelled like overheated, melting plastic coming from one of the aft lavatory light fixtures on an A320 aircraft. Concerns about potential cabin smoke or fire developing could have been a greater issue if pulling the Circuit Breakers (C/B) for the aft lavatories Emergency Lights had not shut the lights off and stopped the odor.

Synopsis

Captain on an A320 aircraft reports the aft Flight Attendant informed him of an odor that smelled like overheated, melting plastic coming from one of the Emergency light fixtures in the aft lavatory.

ACN: 1119380 (1942 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Scheduled Maintenance

Component : 1

Aircraft Component : Exterior Pax/Crew Door
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Nose Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1119380
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Object
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

201309 at approximately XA:00am, my partner and I were tasked to move an A320 aircraft from a Terminal Gate to Spot XX. I was upstairs riding brakes; my partner was driving the [Tow] tractor. Upon parking at Spot-17, he moved an Entry Stand up to Door-1 Left (1L). I opened the door, departed the airplane, leaving the door open. My partner then lowered the [tow] cradle holding the nosegear, at which time I heard the horrible sound of something breaking. I rushed to the cab of the tractor, stopped my partner from further lowering the cradle. He then lifted it back up to the raised position. I ascended the Entry Stand, saw the damage, and showed it to my partner. We then found our Lead Mechanic and informed him of the situation. [Suggest to] always lower the nosegear cradle before positioning any equipment up to an airplane.

Synopsis

An Aircraft Maintenance Technician (AMT) describes how Door-1 Left (1L) on an A320 aircraft was damaged by an Entry Stand as another Technician lowered a towbar cradle holding the nosegear.

ACN: 1117919 (1943 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 15500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1117919
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

As the aircraft was leveling at 370 with the autopilot and autothrottle engaged, the Flight Attendant brought our breakfast to the cockpit. The Captain was monitoring the aircraft while I got out of my seat to open the door and retrieve the meals. After sitting back down in my seat, I glanced down at the airspeed and saw it rapidly decaying to green dot. I then looked at the airspeed window on the display management panel and noted that a speed of 205 KTS was displayed in the speed window. I made a callout to the Captain of "airspeed." We did not have enough thrust to stop the airspeed from deteriorating, so the Captain disconnected the autopilot and lowered the nose to accelerate. As he lowered the nose, he began a gradual descent. He then instructed me to call ATC and ask for a block altitude. As he was asking me to call ATC, Center called us and asked us our altitude and we requested a block altitude of 330-370. The request for the block altitude was granted. I was quite busy monitoring the airspeed and radio, but I believe we descended 500-600 FT to get the airspeed back and start climbing. We climbed back up to 370 without any further incident.

Callback: 1

The First Officer reported that the Captain entered the performance data during preflight and when scrolling through the Performance Preflight pages somehow skipped over the Climb Performance page and in the following page, Cruise Performance, entered 205 KTS. As a result, when the aircraft leveled at FL370, just as the reporter was retrieving crew meals, the aircraft slowed toward 205 KTS and the Captain did not immediately see the decrease. As a result, the heavy A320 could not accelerate by thrust alone and so an altitude decrease was necessary.

Synopsis

An A320 Captain entered the preflight performance data and apparently skipped the Climb Performance page and entered 205 KTS as the Cruise Performance airspeed parameter which resulted in a speed loss during transition from climb to cruise.

ACN: 1117043 (1944 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1117043
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1117702

Events

Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Equipment / Tooling

Narrative: 1

First Officer got push clearance from Ramp to push to the T and repeated to me cleared to push to the T. I told tug driver, "Brakes released cleared to push to the T." The aircraft started moving backwards. We had not pushed very far, just a few feet when a panicked tug driver started saying I have lost the brakes, I have lost the brakes. I started asking him do you want me to set the brakes? After asking three times, and not getting a reply, I applied brakes to stop the aircraft and set the brakes. I told the tug driver, the brakes are set. He did not talk to us for what seemed like a minute or two. He finally said set the brakes and I repeated brakes set. At this point I could see part of the tug out of my window. I told him I would need to talk to Maintenance. Maintenance then took over communication from the tug. Maintenance disconnected the tow bar and tug. With a new tug and tow bar we were pulled back to the gate.

Narrative: 2

After receiving cabin ready notification, I requested a push clearance from Ramp Control; which they cleared us to the "T". Captain asked via interphone if crew was ready for pushback, tug driver responded that they were. Captain then stated "Brakes released, cleared to push to the 'T'." we started rearward motion, then within a second or two, the Captain and the tug driver were conversing; the tug driver was stating that his brakes weren't working. Captain then asked if he wanted brakes set, which he had to repeat multiple times before he got an okay from the tug driver. It was at that time, we got a violent jolt because the tow bar broke. The Captain then set the brake as the airplane came to a rest.

Synopsis

A320 flight crew reports a brake failure on the tug pushing them back from gate as indicated by the tug driver. Aircraft brakes are applied but not before a violent jolt is felt and the tow bar breaks. The aircraft is towed back to the gate by a different tug for maintenance.

ACN: 1116751 (1945 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZDC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDC
Make Model Name : PA-46 Malibu/Malibu Mirage/Malibu Matrix
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Plan : VFR
Flight Phase : Cruise
Airspace.Class E : ZDC

Person

Reference : 1
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1116751
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Automation : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

I was working Aircraft Y, a VFR PA46 Southbound at 12,500 FT. I had given Aircraft X, A320, the descend via clearance for LGA. The aircraft crossed over each other just South of DAVYS intersection. I had never issued a traffic call, I was coordinating off landlines with controllers in my area about a flight of 2 F16 I was working, and needed a point/handoff to adjacent sectors. The Aircraft X was at 13,400 FT and keyed up that they had they traffic in sight. The Aircraft Y then said they had the traffic in sight. The Aircraft X restriction on the descend via was to cross DAVYS at or above 13,000 FT, and the confliction point was South of DAVYS. I should have still called traffic and didn't. "Conflict alert" goes off all the time at various sectors. It is almost a second job just to divert your attention to the "conflict alert" and determine if it's real or not. In this case, however, I do not believe "conflict alert" went off until they were already passed each other. See if there are any improvements on the criteria for "conflict alert", as well as to have real time information about actual aircraft (VFR) that will be entering our space. In this regard, ERAM should help a lot. "Conflict alert" goes off for aircraft that appear to be climbing, but aren't actually going into our airspace, and the descend via protects for the airspace. Some controllers become complacent to the fact that "conflict alert" goes off all the time, and it is a distraction-either to analyze continuously, or the pure distraction it is and then to ignore it, even it's real.

Synopsis

ZDC Controller described a VFR/IFR conflict acknowledging traffic information could have been more precise, adding that the "conflict alert" activates all too often when separation has been established.

ACN: 1116723 (1946 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 200 ER/LR (CRJ200)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1116723
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working the RADAR at Sector X. Traffic was picking up and I got a D-Side. A CRJ2 departed a Class B and was filed up on jetroute. I initially climbed him to FL280. After being level for several minutes he requested FL340. I told him it would be about 3 minutes to clear eastbound traffic at FL290. I physically pointed out the plane climbing to FL340. As the CRJ2 got closer to Area 4's airspace, the A320 was either PVD'ed or the handoff was initiated. The aircraft [was] leaving FL287 climbing to FL320. My D-Side coordinated with Sector Y while I issued clearances to other aircraft. My D-Side told me to climb the CRJ2 to FL310 with respect to the A320. I had already begun climbing the CRJ2 to FL340 so I amended the altitude to FL310 and then once more to FL290 realizing the A320 was climbing slowly through FL290. FL310 was in the datablock as it was coordinated that I would climb him to the altitude and Sector Y accepted the handoff that way. I had the CRJ2 highlighted so I knew that he wasn't climbing to FL310 but rather I was to climb him underneath. I issued several other clearances to other aircraft and Sector Z's D called me to say that the A320 had descended back to FL290. At this time Sector W's D was also on the line and I told the CRJ2 to climb and maintain FL300 and expedite. The pilot questioned the clearance so I told him to turn to a 360 heading and climb to FL310 and to report out of FL300. At some point in the transmission to the CRJ2 I managed to tell both Sector Z and Sector W that I was climbing the CRJ2 since I realized they didn't know what was happening. I feel like my not yet 3 years of experience solo on the RADAR doesn't quite qualify me to figure out a solution but what I realized is this was a very unsafe situation that ended up with nearly 8 controllers involved between 4 sectors and 3 areas. Multiple things were being coordinated with not enough time between to figure out what the most current clearance should be and I ended up reacting to a situation rather than controlling it.

Synopsis

Enroute Developmental described an unsafe event when multiple sectors were involved in coordination and/or lack there of resulting in the occurrence.

ACN: 1116348 (1947 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase.Other

Person

Reference : 1
Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22700
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1116348
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

Our arrival fuel was planned for 4,793 lbs, including 1,731 lbs extra for altitude flexibility. I called the Dispatcher and asked if, in expecting to possibly use this extra 1,731 lbs, he was OK with us actually landing with 3,062 lbs? He said, "Yes." I informed him that in all my years as an A320 Captain I had never, ever seen a landing fuel anywhere near this number and he informed me that he was a probationary dispatcher and had to follow policy. It was simply policy.... 
Accident[s] usually occur through a series of steps, any one of which, if avoided, breaks the chain and prevents the accident from happening. 3,062 lbs of landing fuel gives the crew no time to say, fix a gear problem discovered at the outer marker. It allows for no possibility of a minor reroute or landing runway change; should there be even the most minor mechanical after taxi out, a return to the gate, for refueling would be required. All in all, the idea that we would plan for a flight landing with the FAR minimum is ludicrous.... 
My point is this: I am an experienced Captain with a natural distrust of the potentially hazardous environment within which I operate. I will not let a decision like this back me into a corner by rubber stamping the Dispatcher's decision because it is "policy." A different, less discerning mind, may well simply agree. We now have two steps towards disaster.... So let's not allow the first step of the error chain to exist. I added 3,000 lbs and burned 400 lbs under flight plan with all the safety margins intact. 6,000 lbs of landing fuel allows for a normal FAR landing fuel of 45 minutes flight time, contingencies that are common and normal, and/or an abnormality at the end of the flight.

Synopsis

A320 Captain questioned the safety margin allowed by the Company's fuel reserve policy.

ACN: 1115041 (1948 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SFO.Tower
State Reference : CA

Environment

Flight Conditions : Marginal
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : SFO

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1115041
Human Factors : Workload
Human Factors : Distraction
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1113693
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

On final approach to 28L, while configuring the for landing, the "GPWS" master caution came on while the gear was in transit. The delay in configuration was cause by: crew high work load, last minute runway change, non-standard ATC instructions, out of sequence configuration due to slow traffic ahead and low fuel condition (extended hold/eminent diversion).

While enroute to SFO ATC started issuing extended holding patterns due to a marine layer moving in over the Bay Area. Even though we had opted to add extra 1,000 LBS of fuel and flew very efficient profiles, we advised ATC of a possible diversion to SJC 12 minutes from our agreed "bingo fuel." Approximately 11 minutes later, we were given vectors for approach to Runway 28R at SFO. ATC issued us a fair amount of delay vectors putting us on a very long final for Runway 28R. With our fuel now being 4,800 LBS, we verbally agreed on changing our alternate from SJC to OAK. ATC had also vectored a slower moving traffic in front of us, and began issuing us very low approach speeds on long final, so considering our distance from the airport, and fuel situation, I elected to extend "Flaps 3" (out of sequence) and wait on the gear. 

As we approached the FAF, I called for: "Gear Down, Flaps Full, and Landing Checklist." As the Captain reached for the gear handle, ATC issued a "go-around", which they recanted immediately and called us in sight (we had broken out of the layer seconds prior to ATC calling for the "go-around"). With the slower traffic ahead and airport now insight, we requested the visual to Runway 28L. ATC cleared us for both the visual, and for landing Runway 28L. As I maneuvered for Runway 28L, I called for: "flaps full." Captain was entering the new runway in the FMS, after selecting flaps full, he reached for the gear lever while looking at me, and I nodded in agreement. He selected gear down and started to read the "Landing Checklist". Seconds later, with the gear still in transit, the "GPWS" master activated. We both signaled to each other that we were comfortable continuing with the landing given we still had plenty of altitude, were stable, and now fully configured as the gear was now indicating down and locked. We proceeded to land uneventfully on Runway 28L 

I suggest better dispatch fuel planning on evening flights to SFO. Marine layers seem to happen frequently even when not forecasted. More leeway on Captain discretion to add fuel: (1,000 LBS) does not go very far on an A320. A go-around is at very best 2,000-2,500 LBS of burn. ATC avoiding to vector slower traffic in front of larger planes when holding has been issued and/or ATC has been made aware of light fuel load.

Narrative: 2

First Officer then elected to extend "Flaps 3" (out of sequence) before deploying landing gear, due to position, ATC spacing assessment and low fuel observation. As spacing improved, First Officer called for "gear down." As I reached for the gear handle to select landing gear down, ATC immediately issued a 'go-around' instruction to us--so I did not put the gear down. I held my hand up to the First Officer to signal that gear must now wait (because of the new go-around clearance). This action may have confused him, as he paused before initiating a 'go-around.' Just before he initiated the assigned 'go-around,' ATC then cancelled the 'go-around' clearance, saying, "Tower has you in sight," and where we were now told to continue approach (even though we had just over 2 miles separation from the preceding slowing aircraft). I was certain that another 'go-around' was going to be issued behind the slowing aircraft ahead, and very cognizant of our low fuel situation, so I initiated a request for Runway 28L while above 1,000 feet. ATC cleared us to land on Runway 28L and I began to activate the new runway in the FMS for accurate glide slope and an unexpected go-around into clouds. First Officer quickly called for "flaps full" while he aligned the aircraft, where I was already looking down. I selected flaps full, verified the runway was clear, and then looked at the First Officer while reaching to select landing gear down. He nodded in agreement and said "gear down" while I selected the gear down, and I attempted to read the landing checklist. While the gear was in transit, the GPWS master warning activated. 

Perhaps an earlier call to ATC for advising of our TIME UNTIL DIVERSION (i.e., at 15 mins. vs. at 12 mins.) would have alleviated the stresses, and prompted ATC to sequence us before the slower aircraft--essentially eliminating out of sequence flap selection and erroneous 'go-around' directives. Following SOP "to the T" despite major distractions. Another suggestion would be to allow for more holding fuel, above the Captain's 1,000 lb. limit, to alleviate fuel concerns and accommodate overwhelmed ATC facilities.

Synopsis

ATC put slower traffic ahead of an A320 on a Runway 28L ILS and while slowing for that traffic, Flaps 3 were selected causing an EGPWS warning because the gear was still up as ATC issued, then canceled a go-around.

ACN: 1115004 (1949 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Parked

Component

Aircraft Component : Electronic Flt Bag (EFB)
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1115004
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Other 
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

A checklist design issue arose on a ferry flight I flew tonight. When looking at the A320 Ferry Checklist on the iPad the First Officer initially thought no doors were supposed to be armed. The first bullet on that page is "All cabin...doors...Arm." He thought that this was bullet #6 under the header: "If cabin will be occupied" which is on the previous page. Since our cabin was not occupied he thought this bullet did not apply. When you look at this on the iPad, it's not hard to see how this can be misinterpreted. 

I pulled out the QRH and on that it was readily apparent that the "All cabin...doors...Arm" bullet had a different indentation than the proceeding five bullets under the "If cabin will be occupied" header. This indentation is not readily apparent on the iPad though. 

I'm not trying to open a can of worms here, but just point out this issue. If the combination of only viewing one page at a time on the iPad, along with the indentation formatting issue lead to confusion with one person tonight (with a mundane checklist being read with brakes set), I'm sure it could happen again with other checklists.

Synopsis

The formatting of a ferry flight preflight checklist on their iPad EFB led to confusion regarding the arming of cabin doors.

ACN: 1114617 (1950 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class D : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1114617
Human Factors : Workload
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1114618
Human Factors : Workload
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Diverted
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was acting as PIC and Flying Pilot on this flight. Right after takeoff, Engine #2 Failed with loud bang and jolt. I remained Flying Pilot. Emergency was declared and Captain Emergency Authority exercised. We executed Aircraft Emergency procedures relating to the engine failure. Emergency Equipment was requested. Flight attendants were advised of the situation and landing expectations. Aircraft landed safely at a nearby major airport. After stopping on the runway and setting the brake, the aircraft was met on the runway by emergency equipment to inspect and assess the aircraft, and then they escorted us to the gate. Logbook entries made for Engine Failure and Overweight Landing. 

The First Officer provided careful, comprehensive, and overall excellent support of working through procedures, checklists, etc. throughout the entire event. The flight attendant crew assured proper cabin procedures, followed instructions, and ensured a calm, orderly and safe arrival at the gate with happy passengers. 

Air Traffic Control provided immediate, accurate, and continued expeditious handling and coordination throughout the entire event. Airfield Rescue and Fire Fighters were in position and ready at the time of landing to inspect the aircraft and assure a safe taxi to the gate. 

In my opinion, the entire crew, ATC, and Emergency system worked well together for an uneventful outcome other than the engine needing repair.

Narrative: 2

The Captain stabilized the flight path and called for ECAM actions. Tower called to ask intentions. The Captain told me to declare an emergency and we made a decision to fly to a nearby major airport. The decision was based on several factors including the overweight aircraft, runway length, precision approach availability, crash/fire rescue resources and time required to complete appropriate checklist. We believe this decision was consistent with the Land ASAP of the ENG 2 FAIL abnormal procedure. We resumed the ECAM for ENG 2 FAIL (in flight) and after "no relight", the ECAM was completed assuming engine damage based on aural cues and observation of the ENG 2 gauges. 

While the Captain, using the autopilot, flew to a nearby large airport, I completed the ENG 2 SHUT DOWN procedure, Overweight Landing procedure and set up the approach in the MCDU. The Captain took over responsibility for most of the radio communications and provided a "cabin advisory" to the flight attendants. The Captain conducted a stabilized ILS approach visual conditions. 

After an uneventful, overweight landing, the aircraft was brought to a stop on the runway for inspection by crash fire rescue. The Captain set the parking brake and told the passengers to remain seated. After determining that it was safe to continue, the Captain taxied to the gate while I completed the After Landing checklist. The aircraft parked at the gate and the parking checklist was completed. The Captain and I reviewed Volume 1 to ensure compliance with appropriate logbook entries and required reports. Additionally, the Captain communicated and coordinated with Dispatch, Maintenance, and the Assistant Chief Pilot. 

Simulator training and the available procedures allowed us to handle the emergency safely and adequately. Unless the committee has additional recommendations, I believe we handled this to the best of our ability, consistent with Volumes 1 and 2.

Synopsis

An A320's right engine failed after gear retraction so an emergency was declared, the QRH completed and the flight diverted to a nearby major airport for an uneventful single engine landing.