ACN: 1114303 (1951 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 25

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17375
Experience.Flight Crew.Last 90 Days : 215
Experience.Flight Crew.Type : 3800
ASRS Report Number.Accession Number : 1114303

Person : 2

Reference : 2
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1114312

Events

Anomaly.Aircraft Equipment Problem : Critical
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Normal operation regarding pushback and taxi for takeoff. No abnormal indications and checklists completed without interruption. On gear retraction, felt strong vibration in the rudder pedals and knew the gear doors remained open. ECAM showed the loss of the green hydraulics followed by an overheat of the yellow hydraulic system. Captain performed the ECAM procedures while I flew the aircraft and dealt with ATC. We declared an emergency and returned to [the departure airport]. We landed without any trouble. We were then towed to the gate after emergency personnel checked the aircraft

Narrative: 2

On climbout, lost green hydraulic system followed minutes later with yellow hydraulic system overheat. First Officer flew plane and talked to ATC while I ran ecams checklists. Declared emergency with ATC and returned to [the departure airport]. We landed without problem. I had already advised ATC that we would be staying on runway after landing as we had no nose wheel steering. Emergency personnel checked airplane and allowed us to be towed. Tow team towed us back to the gate.

Synopsis

The Pilots of a A320 described a green hydraulic system fluid loss accompanied by a yellow hydraulic system overheat. An emergency was declared and a successful landing was completed at the departure airport.

ACN: 1113960 (1952 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1113960
Human Factors : Situational Awareness
Human Factors : Fatigue

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were on a vector to intercept the localizer. I armed the localizer, which would have allowed everything to work out just fine. But, since we were 25 NM out, I thought it would have been better to capture the course in NAV mode. After I did that, the FMGC showed no NAV intercept. Then, I attempted to correct the problem by selecting Direct to a fix on the approach. Finally, I just rearmed the localizer and turned in with the heading bug to intercept the course. 

By then we were flying through the localizer. I clicked off the autopilot and turned in manually toward the course. The Captain grabbed his control side-stick to turn in more aggressively to re-intercept the localizer because we had gone far enough through that we approached the center line for [a parallel runway]. The whole time this was going on, I never looked outside to get a better situational awareness. We finally got back on the localizer for a stabilized approach. 

I have not been sleeping well. That is why I was tired prior to the start of the duty period. Furthermore, my marriage has not been going well. That is why I froze up for a couple of seconds and did not take the right action when it needed to be taken. If I had only armed the localized and left it alone and used heading on the FCU to turn inward a mile before intercept, then none of this would have happened. This was due to a combination of over-thinking something and then being to slow to react when it went wrong.

Synopsis

A320 First Officer reported a course deviation on ILS arrival that was caused at least in part by fatigue and personal problems.

ACN: 1113424 (1953 of 2908)

Time / Day

Date : 201309
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 8400
ASRS Report Number.Accession Number : 1113424
Human Factors : Situational Awareness
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

On pushback we were cleared to start both engines. As second engine was starting, we suddenly got a "fwd cargo door open" warning. Captain queried the tug driver and was told, "They just need to throw on another bag." Captain told them the Number 2 engine was turning. By the time the engine stabilized, the door was closing. This could easily have resulted in either serious personal injury or damage to the aircraft engine. Truly a first time experience for me.

Synopsis

A320 First Officer reports the forward cargo door opening while the engine on that side is being started. The ground crew is queried and the crew informed that a late bag is being loaded.

ACN: 1112533 (1954 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZNY.ARTCC
State Reference : NY
Altitude.MSL.Single Value : 13000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZNY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19900
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 4300
ASRS Report Number.Accession Number : 1112533

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had just leveled at 13,000 FT when both MCDU's went "blank" (just menu prompts, with no FMGC prompt for a few seconds) and both Map displays went blank on the ND's. We were able to re-enter the initialization information and restore Nav/map capability after a few minutes. Just after the FMGC blanked, ATC assigned 7,000 FT. We advised ATC of the loss of Navigation and requested to remain at 13,000 FT, as well as assigned heading. ATC re-assigned us as requested, with a slight change in heading assigned. We were able to re-establish the FMGC within a minute or two and resume the arrival. No emergency was declared and I am not aware of any Priority Handling for Traffic Priority received. The FMGC and IRU drift rate were written up to Maintenance on arrival.

Synopsis

A320 Captain experiences a dual MCDU failure during arrival. Vectors are requested while the reset procedure is performed successfully.

ACN: 1112499 (1955 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SEA.Airport
State Reference : WA
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.TRACON : S46
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : SEA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1112499
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1112500
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On approach we were level at 6,000 FT cleared for the ILS 16L. With the LOC captured the airplane also gave us GS* [glideslope acquire]. As the PF was setting Go Around Altitude, we both noticed the Glide Slope jump 1 dot above. The plane began to add power to climb to the Glide Slope. The PF selected a vertical descent of 300 to stop the Climb. The plane's peak altitude was 6,260 FT. We rearmed the glide path and proceeded on the approach. Initially we attributed this to a distance of 28 miles. Proceeding further down glide path we noticed descent rates of 200 to 1,400 FPM. We asked the Controller about any spurious glide slope reports for 16L. He initially responded that there were no reports of spurious glide path. The Controller came back a minute or so later to respond that Tower was crossing airplanes across the ILS critical area from 16L to 16C on the C taxiway. Makes sense. Suggest Tower write a NOTAM when they are doing simultaneous approaches with a scattered layer obscuring the airport advising us of a spurious glide path.

Narrative: 2

Although we had the instantaneous "Dog looking at the TV stare." Call outs were made by me the PM and action was taken by the PF the Captain. We were paying total attention to the situation and monitoring the approach on the final phase like we were supposed to do. Suggest Tower write a NOTAM when they are doing simultaneous approaches with a scattered layer obscuring the airport advising us of a spurious glide path.

Synopsis

A320 flight crew experiences an unstable glideslope during approach to Runway 16L at SEA. It is ultimately determined that the Tower is crossing aircraft to depart Runway 16C which is causing the instability.

ACN: 1111436 (1956 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3500

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Corporate
Make Model Name : Gulfstream V / G500 / G550
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class C : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1111436
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 departed on SID X. A GLF5 departed on the SID Y and was instructed to maintain visual separation with the A320 departing before him. The GLF5 failed to maintain visual separation with the departing aircraft because the GLF5 had a greater climb rate than the A320. The GLF5 was leaving 041 as he reported he lost sight of the departing traffic which was at 035. I called traffic and the GLF5 still did not have the A320 in sight. The next transmission was a vector for separation. When the GLF5 reported he lost sight of departing traffic, I should have vectored for separation first and then call traffic. Unfortunately I called traffic first, then vectored for separation.

Synopsis

TRACON Controller experienced a loss of separation when attempting to establish visual separation between two successive departures, the second aircraft climbed faster than the first.

ACN: 1111307 (1957 of 2908)

Time / Day

Date : 201308

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1111307
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Prior to takeoff, while looking for something I dropped, I shone a flashlight beyond the rudder pedals (First Officer side) and discovered a 3/4 full water bottle beyond the rudder pedals. 

Someone must have dropped a water bottle on the floor and it rolled past the rudder pedals toward the aircraft nose. I have no idea how long it had been there. (It was not my bottle.)

This event causes me alarm. It is not the first time I have found objects beyond the rudder pedals, but most of the time they are small items, like pens, paper, etc. What might have happened had the water bottle begun rolling around, potentially lodging in the rudder pedals? It could be especially catastrophic on takeoff (or landing). And in the event of a catastrophic event, would crash investigators ever discover the cause? I suggest some barrier be mounted on the floorboard to prevent recurrence.

Synopsis

An A320 First Officer reported finding a partially full water bottle behind the rudder pedals. The bottle could have interfered with rudder movement and had apparently been in that location for a period of time.

ACN: 1110323 (1958 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 30000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 2700
ASRS Report Number.Accession Number : 1110323

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 7900
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2108
ASRS Report Number.Accession Number : 1111299

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Avionics smoke ECAM Came on as we climbed toward FL250. We handled the ECAM and determined that there were no other indications of smoke, fire or instrument degradation. ECAM directed LAND ASAP. We declared an emergency with Center and requested a return to our departure airport (approximately 30 miles away). We notified company Dispatch through ACARS and then landed safely. We were inspected by CFR who determined that there were no other indications of fire or smoke. They then escorted us to the gate.

Narrative: 2

No additional information was provided in the secondary narrative.

Synopsis

Following receipt of an Avionics smoke ECAM warning an A320 flight crew performed the associated checklist procedures, declared an emergency and returned to their departure airport.

ACN: 1109870 (1959 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZTL.ARTCC
State Reference : GA
Altitude.MSL.Single Value : 19000

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.Center : ZTL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : IVANE
Airspace.Class A : ZTL

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1109870
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Work Refused
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On the IVANE RNAV STAR, approaching MAYOS...FMS message [repeatedly advising] "too steep path." ATC repeatedly re-assigning speed. Aircraft was attempting to exceed depicted speeds in order to make altitudes. I did not feel safe. Using [my best] judgment and PIC discretion, I requested a different arrival.

Why? The Airbus cannot reconcile [the complexities of] the IVANE [STAR]. The STAR is poor, no better than KELLS. Add in tailwinds and ATC continuously vectoring us off the arrival and continuously re-assigning speeds. We reduced speeds also because of turbulence. [Finally I] requested ATC re-clear us on the MAJIC STAR. ATC did so. Problem solved.

Lots of task loading, turbulent conditions, argumentative ATC. And then later, as I was trying to get home afterwards, my Chief Pilot calling my standby/spare/children's cell phone and leaving me messages to call him on Sunday night when I am OFF! ALL of these things put me "in the red."

Synopsis

An A320 Captain expressed his disgust for the IVANE RNAV STAR into CLT advising its complexities exceeded the capabilities of his autoflight system, compelling it to exceed required airspeeds in order to attempt to comply with the associated altitude restrictions. Further difficulties were provided by the demands of ATC to comply with the published restrictions and/or to reassign airspeeds, vector or otherwise interfere with the flight crew's efforts to comply with the published constraints of the arrival. They ultimately refused the IVANE and requested and were assigned the MAJIC, a less complex, non RNAV STAR.

ACN: 1109044 (1960 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : I90.TRACON
State Reference : TX
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : I90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Route In Use.SID : LFK6

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12600
Experience.Flight Crew.Last 90 Days : 239
Experience.Flight Crew.Type : 2920
ASRS Report Number.Accession Number : 1109044

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 8756
Experience.Flight Crew.Last 90 Days : 132
Experience.Flight Crew.Type : 355
ASRS Report Number.Accession Number : 1109170
Human Factors : Human-Machine Interface
Human Factors : Other / Unknown

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On climbout with an assigned altitude of 4000 FT the flying pilot (Captain) did not follow his command bars and allowed the aircraft to accelerate to 270 KTS. This appeared to be an automation and workload management issue. As I announced this deviation he was slow to correct which allowed the airspeed to achieve a maximum of 270 KTS betweent 3500-3800 FT. Shortly after this correction we were given a climb to 5000 FT. All normal altitude SOPS were followed but as we approached 4500 FT I could see that the flying pilot was again deviating now above the pitch bar and was not appearing to level. I made one verbal attempt to correct the condition with no results. I then, as pilot monitoring, pitched the aircraft over in order to minimize the altitude deviation. We achieved a maximum deviation of 180 FT. As I leveled I the aircraft at 5000 FT I gave full control back to the flying pilot. There were no conflicts or comments from ATC. At cruise altitude we performed a full debrief to include possible causes and remedies.

Narrative: 2

After dept on 360 heading, [we] entered clouds. As I approached my cleared altitude of 4000 FT', I accelerated to 250 KTS. I was hand flying, attempting to slow the climb rate, and with the pilot monitoring on radio I ignored the flight director and got 20 KTS fast. Pilot monitoring pointed out the excess speed and we were given a climb to 5000 FT with a turn and a traffic report at 6000 FT. During the correction to airspeed I overshot my altitude by 180 FT leveling at 5180 FT. I was dealing with speed correction, turn, and level off without flight director guidance because of earlier deviation and thinking of the 6000 FT (as my clearded altitude vs. the traffic's altitude) when Pilot monitoring helped spot/correct Altitude deviation.

Synopsis

A320 Flight Crew reports altitude and speed deviations during departure with the Captain hand flying and not precisely following the flight director commands.

ACN: 1108683 (1961 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : None

Aircraft : 2

ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : B767-200
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Route In Use : Visual Approach
Airspace.Class B : JFK

Person

Reference : 1
Location Of Person.Facility : JFK.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1108683
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working Local 1 at JFK working ILS to Runway 31R and visuals to Runway 31L. An A320 was told to line up and wait on Runway 31L with a B767-200 on 5 mile final. The A320 was cleared for takeoff and notified of traffic on a 3 mile final. The A320 did not commence his takeoff roll. With the B767 on about 1.5 mile final I told the A320 he needed to go. The A320 slowly rolled down runway at less then taxi speed. I sensed he had a problem and told the A320 to exit the runway at Taxiway Y to allow the B767 to land. At this point the A320 advised he was rolling. The B767 was too close for this to work so I told the A320 to cancel takeoff and exit now at the next turn, which was Runway 4L. I tried to send the B767 around, but he had keyed the frequency asking about what he should do causing me not to be able to speak to the B767. Finally the B767 unkeyed and I sent him around as the A320 exited the runway. If an aircraft is cleared for takeoff but is not ready, they should notify ATC as soon as possible to prevent this type of situation.

Synopsis

JFK Controller described a go-around event when traffic cleared for takeoff delayed longer than expected and separation collapsed; the reporter noted the departing aircraft should have informed ATC of any expected delays.

ACN: 1107452 (1962 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9924
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7769
ASRS Report Number.Accession Number : 1107452
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

It was the second flight of our four flight sequence today. The flight was off a quickturn and plane change in ZZZ. The flight through taxi in and the engine shutdown sequence was uneventful. I was the Pilot Flying on this flight. We parked the airplane while the ramp was shutdown due to lightning in the area. The Captain parked the airplane, and then we ran the engine shutdown checklist, and left the IRUs on, as the airplane was quickturning. I sent the in-report, and then packed up my equipment and got up to say goodbye to the passengers like I do on every flight. When the aircraft was deplaned approximately two-thirds of the way, all of sudden the jetway started to move from right to left and I immediately called for the Captain to ensure that the parking brake was set. I had no idea that he had released the parking brake, as it was set when I got out of the seat, and he never told me that he was thinking of releasing it. He told me after the fact that he thought the chocks had been put in place, but that was impossible since the ramp had been shutdown. The only thing I could have done, since the brakes were about 380 C when I got out of the seat would have been to remind the Captain not to release the brakes under any circumstances since there were no rampers working under the weather conditions. That is a crosscheck I will put in place in the future.

Synopsis

A320 First Officer standing at the entry door reports the aircraft rolling forward during passenger deplaning and immediately notifies the Captain who is still in his seat. No damage is reported. The aircraft had been parked without the aid of ramp workers due to lightning in the area and no chocks had been installed.

ACN: 1107307 (1963 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZBW.ARTCC
State Reference : NH
Altitude.MSL.Single Value : 12000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors

Aircraft : 3

Reference : Z
ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : B737-900
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Aircraft : 4

ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Aircraft : 5

Reference : V
ATC / Advisory.Center : ZBW
Aircraft Operator : Air Carrier
Make Model Name : A310
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person

Reference : 1
Location Of Person.Facility : ZBW.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1107307
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

I was working the RADAR position at the Parso Sector when I was advised of a no notice hold for BOS arrivals from my Supervisor. I advised the aircraft, an A320, that there would be holding over SCUPP and the pilot proceeded to SCUPP only to advise he was unable to hold over SCUPP. I then gave him holding instructions over OOSHN, one fix further on the OOSHN2 arrival into BOS. This hold overlapped approach airspace and required a lot of coordination because the Approach Controller was maneuvering aircraft through this area of airspace as well. I advised the Supervisor that we should not take any extra aircraft landing BOS through our sector because of the extensive weather situation. My D-Side then coordinated with the Nantucket Sector to send a BOS arrival to the SUFFI fix as on RADAR it looked to be an opening in the weather. After issuing holding instructions at SUFFI for that aircraft, three more BOS arrivals were flashed through the Augusta Sector from the Cambridge Sector to me. They were all deviating North of the weather that covered the entire Eastern half of my sector. The aircraft were a B737-900, a second A320, and a third A320. Upon check in the pilot of the B737-900 asked why they were rerouted through my sector where weather covered most of their descent into BOS when the previous route they were on had no weather. Following this, Boston Approach called to say the A310 was RADAR contact and to vector it in on a westbound heading. I tried to issue multiple vectors but the A310 pilot advised they were unable any vectors. I finally had to issue a blanket deviate as necessary, when able direct to BOS. The aircraft was stuck in heavy rain and moderate turbulence with nowhere to turn. Fortunately it eventually broke through the weather and was able to continue on to BOS. At this point I decided to shut off the high sector who was flashing another BOS lander. I vectored the three aircraft back to the West side of the storm that had been moving east for several hours and several hundred miles. My D-Side then APREQed routing into BOS from the Northwest. It is standard procedure at ZBW for BOS arrivals to be sent around the East side of a storm that is moving East, only to have to hold in that storm the entire time it moves through. When I spoke with my Supervisor after, he informed me that the aircraft were routed through my sector because the sector that was supposed to work them, despite having no weather, was saturated and holding BOS arrivals. These BOS arrivals should not be sent around and in front a storm just to hold at slow speeds and low altitude while the storm then rolls through the exact airspace in which they are holding. They should be held in areas prior to the weather so that they don't have to be put in the extremely dangerous situation. This is further proven by the fact that the pilots of these aircraft are then questioning why they were rerouted into the weather when their previous route kept them away from it.

Synopsis

ZBW Controller described an unsafe condition when aircraft were re-routed through his/her sector with extreme weather conditions to avoid another sector that was holding aircraft for the same destination, the reporter suggested holding should have been accomplished prior to the weather areas.

ACN: 1107162 (1964 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ONT.Airport
State Reference : CA
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : PRADO 7
Airspace.Class C : ONT

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1107162
Human Factors : Human-Machine Interface
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

During preflight briefing, we noticed the ONT PRADO7 departure SID for Runways 26, did not include the PDZ 6 DME "at or below 4,000" constraint in the database.

Callback: 1

Reporter states that he is certain that Runway 26 was programed as the departure runway and that the six DME crossing restriction was not part of the PRADO 7 departure routing in the FMGC.

Synopsis

A320 Captain reports discovering during preflight that the FMGC does not show the six DME crossing restriction on the PRADO 7 SID departing Runway's 26 at ONT.

ACN: 1106186 (1965 of 2908)

Time / Day

Date : 201308
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1106186
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying; cruise altitude was FL320, clear visibility and light turbulence. We also had a jumpseater. ATC assigns our flight to descend and maintain FL240. While descending we felt a 'thud' and heard a 'popping' noise originating from the right side of the aircraft. Shortly after we received the following ECAM - ENG 2 STALL. We were at approximately FL250. The Captain, stated, "You have the airplane, ATC, I will perform ECAM Actions." The engine continued to stall. Before initiating ECAM, the Captain called for me to reduce power on the right engine. After confirming the 'right Thrust Lever,' I reduced power on the right engine and the engine stalls discontinued at thrust idle. When the engine stalls discontinued, ECAM - ENG 2 STALL cleared.

ATC instructed us to descend and maintain one, one thousand feet (11,000 FT). We accepted the descent and the Captain asked me to increase engine two thrust in an effort to recheck the operation of the engine. A total of 3 attempts to increase thrust would all result in the engine stalling with power settings just above idle. (At this time we received a 'cabin call' from a flight attendant. We ignored the 'cabin call' yet called and spoke to the Lead Flight Attendant several minutes later explaining the situation.) Thrust was then set to idle and the Captain performed COM - ENG 1 (2) STALL resulting in an engine shut down. COM ENG 1 (2) SHUT DOWN was referenced and completed. The APU was started and used. 

I declared an emergency with ATC while continuing the descent to our destination airport. Captain notified Dispatch via ACARS that we had declared an emergency. Captain also called the cabin and spoke with the Lead Flight Attendant explaining the type of emergency, evacuation would not be necessary, estimated time before landing and that we would be bringing the aircraft to a stop on the runway for emergency vehicle aircraft. Approximately 15 NM from our destination the Captain requested control of the aircraft before landing and I notified the passengers. Airport Tower ATC asked if we needed emergency vehicles. We stated affirmative. ATIS weather conditions were calm winds, 10 SM visibility and clear skies, temperature 24 due point 22, altimeter 29.97. The landing was a visual approach backed up with the RNAV Runway 18 made with Flaps 3 configuration, Engine 1 autothrust MCT, hand flown inside the FAF. The Captain brought the aircraft to a stop using medium autobrakes and idle reverse on the operating engine, set the parking brake and made a PA, "Flight Attendants Stations." As a precaution we referenced the Emergency Evacuation Checklist. Airport Fire and Rescue approached the airplane to assess the exterior condition of the aircraft. Fire and Rescue stated the aircraft appeared to be good condition. The Captain made another PA announcement, "Remain Seated, Remain Seated." We then taxied the aircraft, exiting the runway, performed the After Landing Checklist and proceeded to the gate without further incident. 

Learned that the low bleed valve on the IAE engines have issues, hopefully a fix will be forthcoming soon. Also perhaps a read file to other pilots that this issue is happening and that they should review appropriate checklist for engine compressor stalls.

Synopsis

An A320 ECAM alerted ENG 2 STALL during descent, so an emergency was declared, the Captain completed the ENG 1 (2) STALL ECAM, the engine was shutdown and the flight continued to the filed destination.

ACN: 1105364 (1966 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.TRACON
State Reference : FO
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : XR
Route In Use : Visual Approach

Component : 1

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1105364
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff at approximately 8,000 MSL we had an ECAM, EGT Limit Exceeded No. 1 Engine. Captain flew the aircraft while I ran the ECAM. A red tick mark was present on the No. 1 EGT gauge. Upon reducing thrust between MCT and CLIMB detent the EGT was back in limits and thrust lever remained in CLM detent until completion of ECAM and emergency declared and emergency return to the field coordinated. Captain left thrust levers in CL detent with auto thrust engaged until hand flying the visual approach. Limits were never again exceeded.

During the return to field the FMGC would not take the foreign airport identifier as a new destination. Four attempts were made by First Officer and verified by Captain. We even tried the alternate three letter identifier but that did not work either. We elected to not get fixated on automation and loaded the XR ILS manually and flew a visual approach to the runway. Could not get landing performance data and contacting Dispatch at the critical time was considered too distracting during the landing phase. The overweight landing checklist was accomplished with the noted exception of land performance data. Landing was uneventful. Write ups for over temp and overweight landing were accomplished by Maintenance.

Synopsis

An A320 flight crew declared an emergency and returned to their departure airport for an overweight landing following a left engine over temperature event. During the return some FMS and performance data functions failed to function.

ACN: 1105143 (1967 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : JAX.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : JAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1105143
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

We were taxiing out from gate for takeoff when a severe storm began at the airport. We had only taxied a short distance from the gate and were stopped along the grass line of the ramp to await our EDCT, on what used to be called Taxiway J. The storm was so severe the Control Tower could not see us from the Tower cab and since we were going to wait until the storm passed before continuing, Ground Control asked us our position. I replied that we were on Taxiway J. 

However, the airport diagram (10-9 page) no longer accurately depicts the taxiway configuration around the ramp area, as a result of additional taxiway concrete being laid outside the existing ramp in preparation for the construction of a replacement Terminal B in the future. As a result, the taxiway identifications no longer match the printed guidance available in the flight deck. This caused temporary confusion as Ground Control thought we had entered the new taxiway instead of staying on the ramp. They asked us for clarification, and we told them we had remained on the ramp, on what used to be called Taxiway J, the portion of the main ramp along its western edge between where Taxiway R hits the main ramp and Taxiway J appears to hit the main ramp on the 10-9 page (dated 4 Jan 13).

For some reason, [commercial charting company] has not issued an update to its taxi diagram for JAX since the new taxiways have been opened (at least four months ago). In addition, JAX does not have access to modern taxiway monitoring tools such as ASDE-X. As a result, tower controllers must rely on the ability to see aircraft positions on the airport, or ask the aircraft for position reports in the event of fog or heavy rain. Aircraft have difficulty describing their location due to the airport diagram not matching the current actual taxiway configuration.

I suggest updating the airport diagram to reflect the current actual taxiway configuration. In addition the airport might consider installing ASDE-X in the Control Tower.

Synopsis

An A320 First Officer advised the commercially provided airport page for JAX does not illustrate the revised taxiways not in use. Subsequent to this report a new page was published with the revised taxiways illustrated.

ACN: 1104949 (1968 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1104949

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 1104947

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The right engine compressor stalled at FL350. Reduced thrust to idle per immediate action and continued with QRH procedures. Engine recovered to normal operation at flight idle but would stall again when power advanced so we left it at idle, declared an emergency, completed ECAM/QRH and landed at the nearest suitable airport. No visual indication of engine damage or ingestion was found upon inspection on the ground.

Narrative: 2

The flight attendants called and I told them we had heard it and would get back to them. We set up for a drift down and declared an emergency with Center. They said ZZZ was off the nose at 110 miles if we wanted to go there. I said yes and we were cleared direct. We sent a message to Dispatch letting them know we were diverting and to coordinate things for us. We discussed the single engine profiles, diversion guide, and approach and landing data. We let the flight attendants know the problem and plan, [and] then made a PA explaining the situation to the passengers. 

We executed a normal single engine visual approach and landing without incident at max landing weight. Good CRM and management along with no rush from the Captain made this a non-event from the pilot's perspective.

Synopsis

An A320 flight crew declared an emergency and diverted to a nearby airport when the right engine compressor stalled at thrust settings above idle.

ACN: 1104311 (1969 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1104311
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21100
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 9800
ASRS Report Number.Accession Number : 1104317
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

In preparation for our flight, I noticed our planned takeoff weight was roughly 164,500 lbs. Knowing that planned weight was near our maximum takeoff weight, I requested performance numbers for Runway 28L and Runway 1R at an assumed weight of 166,000 lbs. The performance numbers came back for Flex takeoff setting of Flaps 1 for 28L and a Flex takeoff setting of Flaps 3 for Runway 1R. The Captain and I discussed the benefits of both runways and the decision was made to disregard the Runway 28L numbers. With that, a Flex takeoff with Flap 3 for 1R was entered into the FMGC. While taxing and knowing we were to fly the Porte departure from the 1's, the Captain asked me to retrieve the performance numbers for Runway 1L. While waiting for the Takeoff data, the Captain and I both shared our skepticism about the aircraft performance abilities for 1L. We were both surprised to see that Flaps 3 performance numbers came up for 1L with the Final Weight data of roughly 164,000 lbs. We were even more surprised to see that TOGA thrust was not necessary. 

So I proceeded to the box and made the appropriate changes to the runway. Switched from 1R to 1L, entered the V speeds for Flaps 3 at 1L, entered the Flex temperature for 1L and somewhere in there I mistakenly changed the current flap setting of 3 to the erroneous setting of 1. No adjustment to the actual flaps was necessary due to Flaps 3 was planned for Runway 1R. Shortly there after we commenced with the Before Takeoff Checklist. With his hand on the Flap handle Captain questioned the flap setting. I glanced down behind the flap handle where the Takeoff data was located and inadvertently confirmed Flaps 1. I can only assume I failed to look at the appropriate runway data. We proceeded to Line Up and Wait on runway 1L. The Captain transferred the controls over to me. Once cleared for takeoff, I applied the power appropriately considering the weight of the aircraft and the length of the runway. Acceleration felt normal. The "V1" and "Rotate" call out came prior to crossing Runway 28L which, on reflection, did stand out as I began a slower than normal rotation. In the Second Segment Climb the aircraft accelerated as well as I expected for a heavy A320. Crossing the acceleration altitude I lowered the nose, saw a positive green trend arrow, brought throttles back to Climb and called for flaps 1. That is the moment I realized I failed to keep situation awareness on the ground and that led to an Undesired Aircraft State. We were very fortunate that we did not experience an engine failure after V1.

Narrative: 2

Why did we takeoff in an undesired aircraft state? Not because the First Officer made a mistake, but because at no time did I double check the takeoff data for 1L with my own eyes nor did I confirm that the data was properly loaded into the FMC. I had simply watched the data being loaded thinking that I was verifying it at the same time. Complacency on my part certainly played a major role in my actions. In retrospect, I recognize that although virtually every single pilot on the Airbus has many years of experience on it, mistakes will still occur. We need to be cognizant of complacency, remember our SOPs, and be always vigilant in order to back each other up thus ensuring the double checking and crosschecking required for the safe operation of our aircraft.

Synopsis

An A320 departed SFO Runway 1L with Flaps 1, Flex power near maximum gross takeoff weight and they realized after takeoff that the slow climb and acceleration were the result of not having selected Flaps 3.

ACN: 1103499 (1970 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Engine Driven Pump
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Minimum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15400
Experience.Flight Crew.Last 90 Days : 276
Experience.Flight Crew.Type : 6565
ASRS Report Number.Accession Number : 1103499
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

This A320 aircraft had a Deferral (DF) with the narrative of the Green System Hydraulic Pump cockpit switch [was] inoperative (inop), in that it could not turn off the Green System Hydraulic pump. It had been DF'd a few days prior in ZZZ1. Since the Green System pump is a major aircraft system and the ability to turn it off, directly affects the ability to carry numerous Green Sys fault checklists; I was concerned with the DF. A call to Maintenance Control to question the Deferral (DF) led to my call to the Dispatch Desk to refuse the Aircraft. If the [Green] System lost [hydraulic] fluid the pump was uncontrollable and I felt this could lead to catastrophic results in relation to Engine-1. Further questioning of Maintenance Control led me to believe that the MEL used for the DF had been fairly loosely translated to apply to this malfunction. The gate Mechanic said the switch was not the problem, but the actual solenoid on the [Engine Driven Hydraulic] pump itself. From start to end of this event, I could not fathom why this Aircraft had been DF'd at all. As Maintenance Control said later, glad we [flight crew] caught it before an airborne incident happened. I couldn't agree more.

Synopsis

A Captain questions Maintenance Control about their deferral of a cockpit control switch for Number 1 Engine's Green Hydraulic System Engine Driven Pump (EDP) on an A320 aircraft. Captain noted that if the Green System lost hydraulic fluid, the pump would be uncontrollable and the inability to turn off the pump affects the ability to carry numerous Green System fault checklists.

ACN: 1102761 (1971 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Temperature - Extreme

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1102761

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Ground Event / Encounter : Other / Unknown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

This was day four of five scheduled days in a row. I had a back to back three day and a two day trip, this being day one of the two day. The first two days of the three-day trip were late nights with the second day resulting in minimum rest (9 hours at the hotel). I spent the entire day prior to my report time literally laying around the house resting for XC18 LCL departure. I was attempting to recover from the three-day trip I had just flown. The fatigue call was a result of TWO issues: the creeping delay which was going to result in landing at approximately XI:30 AM body time and heat exhaustion due to the pack issue while taxiing. I arrived at the airport employee lot at approximately XA:35 PM and waited about eight mins for a bus. I was drenched with sweat when I got on the bus. I took the people mover over to the D concourse and upon entering the terminal, the first thing one noticed was the hot temperature in the terminal. It was approximately 80 in the terminal and the same in ops (numerous pilots were complaining). Aircraft was about 50 minutes late on the arrival. I spoke with the Captain and asked him if the aircraft was "good." He said it was, but that the APU was, "a little anemic on the ground." We boarded the aircraft and as you can imagine, it was chaotic with all the activity. There were several minor maintenance issues, and cabin issues that added to the mix. The aircraft door was closed at about XC35 LCL and we pushed at XC55 LCL due to the delay with the maintenance documentation. Scheduled departure was XC18 LCL. The aircraft temperaTURES were hard to gauge because the aft cabin doors were opened multiple times do to servicing issues. The aft cabin was approximately 82 and the forward was 84, but the jetway was well over a 100 and the hot air was pouring in. The APU was just keeping up. My First Officer and I agreed that once the doors were closed and the engines running (based on the previous crew comments) that the temps would come down. Please note, at this point, it was not unsafe. In that type of heat and fully loaded airplanes, many APUs can't keep up. We started up and taxied out to 1R. During the taxi it became apparent that the packs were not keeping up with the demand even when on bleed air from the engines. We were in a long line of aircraft and concurrent with the pack issue, we were given a wheels up time of approximately 1+20 and assigned a spot in the runup block. Once in the blocks, we contacted maintenance via phone patch with Dispatch. We tried every combination of bleed air, hot air valve combinations but we were unable to get the temps below 88 degrees. I asked the Lead Flight Attendant if the temps were acceptable, meaning not unsafe. She said they were bearable at the moment but a prolonged delay was not acceptable. Dispatch was in the loop and was trying to get us priority to get in the air. A short time later, the Lead Flight Attendant advised us that all the ice melted (apparently they no longer put dry ice on the ice) and that we were out of bottled water. Well, with the cabin temps of 88, no ice, and the prospect of spending 4+ hours with no bottled water and warm drinks, we decided to go back to the gate. ATC still had a ground stop in place so the decision was now very easy. I could not leave these people on the ramp with those temps and unacceptable liquids. Also, I was starting to feel the effects of the heat. My undershirt was soaked and I was very uncomfortable. We returned to the gate and the total block time was approximately 1 hour 10 mins. The people were deplaned and Maintenance began to work on the issue. I left and went in the terminal in search of some cool air, which was very hard to find. There was one spot by the train entrance, which was cool. At one point I went to operations to use the restroom and splash cold water on my face. ALL THREE SINKS put out only HOT water no matter where the faucet was selected. It was HOT water, not warm. I could not get myself cool. I was sweating profusely and could not recover. At about XD:00 I returned to the gate and was told by the Mechanic that the problem was solved. He said it was the pack controllers. The airplane was cool, but I noticed that the airflow was much higher than normal. I asked him if he had the ground air on and he did. I asked him to put the APU on the aircraft and he was a little cranky that I wanted to check it out for myself. He placed the bleed in the ON position. I then asked him to remove the ground air. He did and guess what? The air output was warm. Now it's XD:15PM, our original flight attendants went illegal, new flight attendants are brought in and at about XD:30, THREE mechanics come up to the fight deck and ask, "you guys have some sort of pack problem?" I am sweating profusely, so much so one of the replacement flight attendants becomes concerned. She makes me sit down and gives me more water. Now the search for another airplane begins. And it becomes apparent to me that I will not be able to physically fly the flight, which at BEST, would leave at XE:45 PM. The cumulative affect of late nights, the creeping delay and the heat exhaustion with the pack issue made it unsafe to fly across the country at that time of the night/morning. I called the Chief Pilot on duty and informed him of the situation. I went home at about XE:15 PM. I am going to pick up the balance of my trip Friday evening.

Synopsis

A320 Captain describes calling in fatigued after numerous delays at the gate and a ground stop. While on the taxiway to wait out the ground stop heat inside aircraft becomes excessive and they return to gate to remove passengers. Mechanics are brought in but unable to fix the issue with the PACKS and ultimately the flight cancels.

ACN: 1101530 (1972 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 1100

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1101530

Events

Anomaly.ATC Issue : All Types
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

We were on final approach for Runway 28R. We had selected the FMS Bridge Visual 28R and had been cleared for the approach. As usual we were slightly high on the profile due to numerous ATC speed assignments during the approach. At 1,100 FT we were on profile with the PAPI showing two white and two red lights. We were 3 miles from touchdown and descending at about 700 FT per minute. At this point the Tower Controller transmitted, "low altitude alert, check your altitude immediately." This caused us to delay our 1,000 FT call and was very distracting. We replied that we were on profile and that the PAPI confirmed that. A short time later the Tower Controller transmitted "no comment".

Synopsis

A320 Captain reports being issued a low altitude alert by SFO Tower while passing 1,000 FT on the PAPI for Runway 28R.

ACN: 1100656 (1973 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 22
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : LGA

Component

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1100656
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance
Result.Aircraft : Automation Overrode Flight Crew

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

While being vectored for the ILS 22 through moderate weather we turned base, then final, armed the approach and began the descent. We came through the bases of the clouds and it became obvious through multiple lightning strikes, the clouds we were in were very convective which we did not previously know. There were multiple lightning strikes around us and the airspeed began to stagnate. I announced the stagnation and decrease in airspeed to the Captain and he responded that he was seeing it to. This was still before the FAF between 3,000 and 2,000 FT. I was busy switching from Approach to Tower at this time and [making] commanded configuration changes and running the Landing Checklist but the stagnation continued. I was waiting for the autothrust to give him additional power and it never came. Researching it later, below 3,200 radio altitude the autothrust should be more sensitive and should never accept the speeds that the aircraft was slowing too, well below the magenta bug. This makes me think there is a problem with the autothrust on approach with this aircraft. I then noticed he was taking an action, not necessarily what I would have done. He took the thrust and placed it in MCT or slightly between CLB and MCT and moved it around a little seeming to be trying to get the thrust up. After seeing multiple mode changes and continued stagnation I called for a go-around. We never received a Windshear Warning but it seems as if it was windshear. I notified Tower of the go-around and called positive rate. Captain responded with gear up and the speed increased quickly as if on the backside of the windshear. I mentally calculated it to be about a 30 KT shift and let ATC know. At this time the flap speeds were exceeded. I made a callout to pitch up since we were well below the command bars, more pitch was needed to get into the command bars or less power due to the intermediate go-around, the speed was increasing so fast that I treated it like a windshear and reconfigured the flaps on speed just announcing it and not waiting for the command. ATC assigned runway heading and 2,000 after we had already passed that altitude on our way to 2,700, I set 2,000 and selected and then ATC assigned 3,000, I set that altitude and selected. 

First and foremost I think the ATC facility should not have been using this runway. They continued to use it after we went around and announced the windshear. After landing we sat on the parallel taxiway watching the other aircraft land on [Runway] 22 and seeing the lightning strikes that surrounded them and knowing they were just realizing it when the were coming through the base layer left me with a feeling of disappointment in ATC. I know they do not like changing runways around but when the other 3 directions did not have any Convective activity and the winds were actually favoring landing on [runways] 4 or 31 according to the windsock, the runway change should happen faster. As far a piloting the aircraft goes. I feel more time training intermediate go-arounds and proper ways to turn off, when to turn off, and what to do when autothrust is not doing what is desired is needed. Trying to get the thrust to come up due to slow speed by bumping it up above CLB is poor technique. The entire situation would have been prevented by simply turning off the autothrust and setting about 60% N1 for configuration full to fix the slow speed. This should be previously trained and frequently practiced on the line. Additional training should also be done on intermediate go-arounds and what to do with pitch and power and how long TOGA is needed. ATC increased the confusion of the situation by having a published altitude, followed by an altitude we were already passing (2,000 FT) followed by a new altitude (3,000 FT) in an already complex situation.

Synopsis

A320 First Officer describes an ILS approach to Runway 22 at LGA with lightning, turbulence and possible windshear. The autothrust system is not maintaining the desired reference speed and the Captain's "technique" to override it is not successful. A go-around is announced by the First Officer, above 1,000 FT, resulting in flap speed and altitude exceedances.

ACN: 1099875 (1974 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737-800
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class C : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1099875
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.General : Declared Emergency
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I was working an A320 a departure off of Runway XX and RADAR identified the aircraft and climbed to 15,000 MSL. Pilot reached 3,000 MSL and advised that he wanted to level off. Asked if he needed assistance and he said no that they were checking a landing gear problem. I then asked if he could accept a climb to 5,000 MSL for MVA and airspace integrity. He said that he could climb. After a few seconds he advised that he wanted to return to [the airport]. My Coordinator was listening to the frequency and started to coordinate his return to the airport. We were in a southeast configuration so the best path to return to the airport was not clear but I turned aircraft back to the airport on a heading of 310. Then the A320 advised that he needed to return to the airport immediately and get down. I asked the A320 if he had the airport in sight, and he confirmed that he did. My Coordinator had already advised the surrounding sectors of the emergency and they cancelled approaches and emptied the final because we were going to be opposite direction. I cleared the A320 for the visual approach to Runway 29. He acknowledged the clearance and advised that he needed to make a maneuver to stabilize the aircraft. I then noticed a target coming off of the airport. The Tower was unable to stop the departure of a B737. I needed communications with that aircraft to move him out of the way. My Coordinator was on the line with the Tower trying to get communications and also issue the heading that I wanted him on of 090 which would have pulled the B737 away from the emergency aircraft. I was delayed in receiving the B737 but I radar identified him and immediately turned him to heading 070 with an expedited climb to 15,000 MSL and called the traffic. The B737 reported the A320 in sight, but standard separation was already lost. This was an emergency situation that escalated quickly. I do not know how the situation could have been prevented or handled differently.

Synopsis

TRACON Controller described an emergency event when a departure requested an immediate return to the airport opposite direct to the flow of traffic, the Tower then released an opposite direction aircraft.

ACN: 1099641 (1975 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : MMUN.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : ELB / ELT
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1099641
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Dispatch
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I dispatched a flight from MMUN. This aircraft had MEL 25-61-01A which is a portable ELT. This particular MEL states "May be inoperative provided no Extended Overwater operations are conducted." Under Operation Procedures for the MEL, it further states "Aircraft may NOT conduct extended overwater operations; however it may be eligible for limited overwater operations. In order to comply with this MEL, the flight must be routed in a way so it does not exceed 30 minutes flying time in still air with one engine inoperative, or 162 NM from the nearest shoreline, whichever is less. I dispatched the flight. This route did not comply with the MEL and I, nor the flight crew, caught this error. I made the assumption that the aircraft was allowed to fly the standard route based on the fact that it had already flown with that MEL, and after I had spoken with the Safety Officer, confirming that it only restricted ETOPS operations. I did reference the MEL manual, but failed to read the Operations Procedures which outlines the restrictions.

If it is a possibility with dispatch software, a safeguard that would prevent a flight plan from calculating if it was in violation of an MEL restriction would prevent this from happening in the future (much like how a flight plan will not calculate if you attempt to plan a non-overwater A/C on an overwater route). I have reason to believe that numerous people missed this restriction due to the fact that we are not ETOPS qualified. As a result, assumptions are made that there are no Operations Procedures, that it does not pertain to us, and they then fail to continue reading past the MEL to see if any action needs to be taken. Perhaps a notification can be incorporated into dispatch monitor when running a flight plan with this or similar MELs. For example, a pack MEL will notify you that you must remain at a certain flight level and that there is a fuel penalty. These notifications appear on the flight summary page before taking you to the flight plan. The note could say something along the lines of, "Flight is restricted to limited overwater operations due MEL 25-61-01A." Also for the Dispatch group, reiterate the importance of reading an MEL in it's entirety via training or an Ops Memo.

Synopsis

A Dispatcher realized after the fact that an A320 with an MEL restricting extended overwater flights, was routed with a flight segment over the Gulf of Mexico.

ACN: 1099596 (1976 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10670
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3027
ASRS Report Number.Accession Number : 1099596
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21500
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 320
ASRS Report Number.Accession Number : 1099580
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Soon after reaching cruise altitude of 37,000 FT, we received an ECAM: HYD B SYS LO PR. We complied with the ECAM and turned off the Blue Electric Hydraulic Pump. At this point, we ran the checklist per SOP for a Non-normal Laning and completed the landing distance procedure. We called Dispatch and Maintenance for a conference. While talking with Maintenance, we received a HYD B RSVR LO LVL. The only difference from the previous ECAM was not to turn the Blue Electric Pump on during approach. Otherwise, all protocol remained the same. Maintenance entered the failure into the maintenance history and informed arrival station Maintenance. We discussed the issue with the flight attendants. We declared an emergency with ATC and asked for 34L and direct our destination VOR. Once we checked on with ATC, they assured us that airport authorities had been notified and emergency equipment was ready. We continued and landed on 16R without further incident.

Narrative: 2

[No additional information from Narrative #2]

Synopsis

An A320 ECAM alerted HYD B SYS LO PR followed shortly thereafter by HYD B RSVR LO LVL so an emergency was declared, the QRH completed and an uneventful landing followed at the destination airport.

ACN: 1099338 (1977 of 2908)

Time / Day

Date : 201306

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1099338

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After an uneventful cross country flight and landing, exited on high speed and turned north on taxiway. Noticed the plane veer left on taxi and was unable to control airplane with nose wheel steering. Had ECAM Green 
Hydraulic Low Pressure and loss of nose wheel steering. Stopped airplane with brakes before exiting the taxiway. Coordinated with Ground and Maintenance to have a tug pull us to the gate. Handled the ECAM and shutdown the engines. Coordinated with the flight attendants that we were ok and would be towed in. Airport personnel said they saw hydraulic fluid leaking. Uneventful tow in to gate.

Synopsis

A320 Captain experiences a Green Hydraulic system failure during taxi in after landing resulting in loss of nose wheel steering. After stopping on the taxiway the aircraft is towed to the gate.

ACN: 1099291 (1978 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : DC Regulator
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1099291
Human Factors : Human-Machine Interface
Human Factors : Confusion
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1099304
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[In cruise] got ECAM alert: BCL1, immediately followed by BCL2, losing ess DC power. Also had multiple ECAM warning/advisories including ELECT: DC EMERG CONFIG. Followed ECAM through many pages, leading to deployment of the RAT. The RAT cleared most of the ECAM's and re-powered the ess DC bus, so we were out of the emergency elect configuration. Had some ECAM warning/advisories left, including BCL1/2 fail; Engine 1 A-C FADEC supply; APU no battery start; APU no fire detect; Number 1 RSVR inoperative; Number 1 Engine no start. Elect Page showed DC battery bus unpowered. After ECAM complete and malfunctions stabilized, we contacted Maintenance Control thru a patch. All agreed with crew assessment that we were no longer in an emergency situation. All agreed with crew decision to continue to destination. Following discussion with Maintenance Control, we called a deadheading A320 pilot up to the cockpit to put fresh eyes on the problem. He also concurred with crew actions and decisions so we proceeded to destination. During approach brief, all ramifications of the thrust reverser inoperative and possible radio problems were discussed. We used the longest runway, and before the RAT died, briefed Tower/Ground that we might have radio problems and received clearance to our gate. Approach and touchdown went normally, but after touchdown things got complicated and weird. I believe we had additional electrical problems that were not apparent to us, specifically with LGCIU's and/or ground proximity switches such that the aircraft thought it was still in the air. After touchdown and engines to idle, it appeared to go to flight idle (600-700pph), the ground spoilers did not deploy; after the RAT died, we lost comms, and the airplane went through multiple rapid BIT checks, including fire/detect cycles including lights and alarms, etc, etc. It is my belief that the only braking we had was accumulator pressure. I say this because at the gate with the parking brake set, we had zero normal brake pressure and the accumulator was near zero too--indicating that was what we were using for braking. At the gate, the engines would not shut down normally. With master switches to off, the FF would go to zero, then back to idle as the engines went into an auto-start. Engines had to be shut down with the fire buttons. The airplane thought it was still flying, and it was not something we expected. There were no FAA violations, and all is well that ends well. But something very unusual happened to this airplane.

Narrative: 2

Master caution and multiple ECAMs came on and autopilot disengaged. Captain flew and First Officer did ECAMS. Failure drove to deploy RAT, after RAT deployed most ECAMS disappeared. Both engine AC generators working but essential busses powered by RAT. Electrical page showed Amber Bat Bus. Engine Reverser 1 inoperative, Eng 1 start inoperative, APU no bat start, no APU fire detected and min RAT speed 140 on status page. Autopilot was working and engaged. The Captain contacted Dispatch and Maintenance Control who brought a Duty Manager into the loop. All agreed to continue to our destination. [We] landed [on the runway] using selected speed 140. Contacted ground and told them we had elec issues. Taxiing to gate lost radios, ACP and RMP's blinking. At gate had to use QRH: Engine fails to shut down checklist on both engines (fire light switches to shut down).

Callback: 2

The First Officer states that the first thing that the maintenance technician did upon entering the cockpit was to cycle the battery switches off then on then proceed to start the APU. The ECAM BCL faults disappeared and the APU started normally. During approach it was noted on the electrical page that all buses were powered except the battery bus which was amber. Although this maintenance action seemed to clear the faults entirely, it was discovered in maintenance history that ultimately maintenance elected to replace both BCL relays and both batteries.

Synopsis

A320 flight crew experiences an ECAM alert for BCL1, immediately followed by BCL2, losing essential DC power while in cruise. Many pages of ECAM's are complied with until finally the RAT is deployed, which cleared most of the ECAM's and repowered the ess DC bus. Flight continues to destination but after touchdown the ground spoilers would not deploy, after the RAT died, communications were lost, and the airplane went through multiple rapid BIT checks, including fire/detect cycles including lights and alarms. The engines would not shut down normally at the gate and the fire buttons had to be used.

ACN: 1098661 (1979 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : IAD.Airport
State Reference : VA

Aircraft

Reference : X
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : IAD

Person

Reference : 1
Location Of Person.Facility : IAD.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1098661
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Landing Without Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working all the locals combined up at Local Control 3 due to a closure on 19L. It was the arrival bank. I had a Local Assist. PCT was landing 19C and 19R. Because of construction on the high speed exits of 19C/1C PCT can't run reduced separation on 19C. So PCT has been told they need to give IAD at least 3 miles between arrivals to 19C. The arrival bank was just starting to get busy. PCT had just started landing 19R. I noticed that the PCT Final Controller had about 8 arrivals they were working and the Final was not split. I had an arrival (RJ) to 19C with an A320 close behind and overtaking the RJ. I was talking to the RJ, but not the A320. Both aircraft were in my airspace. The FLM heard me comment to the Local Control Assist that PCT needed to quickly slow and switch the A320 or that "this is not going to work." The RJ needed to go down to the 3rd high speed to exit due to the construction. The FLM said he would call PCT and advise them they needed to comply with the restriction on 19C final. I advised the FLM that I was still not talking to the A320. At that time the RJ was 2 mile final and the A320 was maybe 4 mile final. I noticed that the A320 was starting to "S turn" on final. I then noticed that I had 2 arrivals to Runway 19R. I was talking to the 1st 19R arrival but not the 2nd. I kept trying to reach the A320 on the frequency to no avail. The Local Assist Controller called the Final Controller and told them to switch the A320. The RJ landed 19C and I advised them to hurry down the runway as I was not talking to the A320. The Local Assist told Final to clear the A320 to land 19C. At that time I believe that the RJ was still on the runway. The RJ then cleared 19C just prior to the A320 landing 19C. I never cleared the A320 to land 19C. The A320 cleared 19C and called me on Local. I switched the A320 to Ground. I advised the FLM that the A320 landed without me giving him a clearance. I told the FLM he needed to call PCT and make sure they cleared the A320 to land 19C. The FLM called PCT and PCT advised that they did not clear the A320 to land 19C. As all of this was going on, I had the same thing happen on 19R 2 minutes later with another air carrier not talking to the Tower until 1 mile final with a preceding arrival still on the runway. If the above 2 incidents were not enough, later I was working Local Control 1 during a departure bank. We were north departing 1C and 30, landing 1C/1L, 1/R was closed. An A380 landed 1L and was holding short of 1C at W3. I had numerous aircraft awaiting departure 1C and a 1C with the A380 and depart 1C prior to the next 1C arrival. I knew I would slow up the 1C arrival or "S" turn him if I had to. I reached out to the aircraft on 4 mile final 1C to advise them to slow. They were not on the frequency. I advised the FLM that I was not talking to the aircraft on final 1C. I advised the A380 to cross 1C with no delay. I told the A380 that traffic on the runway was waiting for them and that there was traffic on final I was not talking to. The view out the window was getting worse. I started to worry about if it was going to work and what I was going to do if the arrival never checked in. The A380 cleared 1C and now I am panicking. I am still not talking to the aircraft 1 1/2 mile final 1C. I have an aircraft LUAW on 1C and if they are not cleared for an immediate takeoff I will lose separation. But if I clear him for takeoff and the 1C arrival goes around 1C because he does not have a landing clearance, what is the go-around going to do? Fly runway heading and my departure will climb into him from underneath him? I cleared the 1C LUAW aircraft for immediate takeoff and advised him I was not talking to the 1C arrival. Finally the 1C arrival checked in short final. I cleared him to land and just had legal separation. Why does PCT put me in that situation on a DAILY basis? I have to base what I am doing with aircraft on the assumption they are going to switch people to the Tower. This issue of PCT not switching aircraft to the Tower is beyond unacceptable. If it is because they are to busy with the Final all combined up then they need to split the position sooner. PCT needs to comply with the restriction they are given on Final. PCT needs to split the Final position sooner. A new LOA needs to be developed that requires arrivals to contact the Tower prior to entering the Tower airspace.

Synopsis

IAD Controller described an on going unsafe condition chronicling several late frequency changes from PCT making landing/takeoff decision very difficult.

ACN: 1098149 (1980 of 2908)

Time / Day

Date : 201307
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737-300
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 3

Reference : Z
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737-700
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1098149
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Air Carrier X level at FL360 had multiple crossing traffic at his altitude. I asked him if he could accept FL380 for traffic. His response was unintelligible to me. I told him that I missed his response and asked him if he said affirmative to being able to accept FL380. He said, "Copy." I told him, "Roger, I will have that for you in a couple minutes, standby." Approximately one minute later, Conflict Alert was activated and showed Air Carrier X at FL365. I told him to maintain FL360; I had never issued him a climb clearance. I issued a traffic alert and told him to descend immediately. Air Carrier X lost separation with Air Carrier Y at FL370 and Air Carrier Z at FL370. I never issued a climb clearance to Air Carrier X, nor did he ever read back an altitude change. When I told him I would have an altitude change for him in a couple minutes and to standby, he never responded at all. Air Carrier Z did get a TCAS RA. I issued the traffic to him and told him that Air Carrier X was descending out of FL370 which is what the RADAR indicated. Air Carrier Z never left FL370. What led to this event was me asking the pilot if he could accept a higher altitude for traffic, questioning his response when it was unreadable, I verified he could accept FL380, [and] then told him I would have it for him in a couple minutes and to standby. This event happened because the pilot was never issued an altitude change, never read back an altitude assignment, but left FL360 and climbed through two other aircraft at FL370. I told him I would have the altitude change in a couple of minutes because I was fully aware of the two aircraft passing him at FL370. During further discussion with the pilot on the frequency he claims he heard "expedite" instead of "expect" when I never in fact used the word "expect" at all in the initial transmissions to him, I told him I would have it for him in a couple minutes. This is very blatant pilot deviation which put three aircraft in very close proximity to each other. I am deeply concerned about why a pilot would leave their altitude without a clearance to do so, and why he would climb to a new altitude without ever reading the clearance back to me. Why was this not caught or questioned by either the pilot or co-pilot? My recommendation would be for a pilot to always read back an altitude change, and for the pilot and co-pilot to check each other when issued an altitude change, which I believe is standard procedure for most airlines. If the pilot was alone in the cockpit, he should have asked me to verify the altitude again, which I believe is airline procedure also. If the pilot would have looked at his TCAS display prior to leaving FL360 he would have seen two aircraft in his immediate vicinity at FL370, and should have questioned what he thought was a clearance to FL380. Any of these actions by the pilot would have prevented this incident from occurring.

Synopsis

Enroute Controller described a loss of separation event involving several aircraft. The reporter indicated the pilot started an unauthorized climb after being advised of the Controllers' plans.

ACN: 1097797 (1981 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 3000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use : Vectors
Airspace.Class E : SCT

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Aircraft Operator : Personal
Make Model Name : Skyhawk 172/Cutlass 172
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Plan : VFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class E : SCT

Person

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1097797
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Had a C172 enroute to SMO maybe 4 miles east of LGB at 4,000. LBG Tower flashed a release for an A320, which I accepted. My plan was to keep the A320 at 3,000 until in front of the slow moving Cessna. LGB Tower called to say the A320 was going to be a couple minutes late on his release because they were getting jammed on final. Swiss cheese hole 1: Now my plan was to turn the Cessna up along the boundary to get out of the way of the A320 when he departed. But, DOWNE Sector called to use the racetrack and descend an aircraft to 5,000 (swiss cheese hole 2). This tied my hands a little on running the Cessna tight to the line. Now my plan was to turn the A320 out to a 250 heading to get divergence from the Cessna. However, when I turned the A320 out there was a VFR aircraft opposite direction at 2,500, which I assumed was descending for LGB, swiss cheese hole 3, but upon rechecking the situation, I noticed the VFR at 2,800. I turned the A320 to a 270 heading and he got an RA from the VFR aircraft. I then climbed the A320 to 6,000 as per the handoff. I think there should be a bit of a redesign on the Bravo Airspace, or a Class Charlie Airspace to cover LGB for this departure as well as the LGB...VTU departure. On my side, once I notice the situation I should have cancelled the release with LGB, or told DOWNE approved with the racetrack and shown my traffic (C172) and requested a point out at 4,000. Also never assume what a VFR aircraft is going to do.

Synopsis

SCT Controller described a conflict event when an IFR release from LGB was delayed and plans to handle a VFR aircraft failed to materialize. The reporter suggested airspace changes that might prevent future occurrences.

ACN: 1096939 (1982 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6282
ASRS Report Number.Accession Number : 1096939
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1099367
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After uneventful flight, landed on 22L and exited on high speed and turned north on Taxiway P. Noticed the plane veer left on taxi and was unable to control airplane with nose wheel steering. Had ECAM HYD G LO PRESS and loss of nose wheel steering. Stopped airplane with brakes before exiting the taxiway. Coordinated with Ground and Maintenance to have a tug tug us to the gate. Handled the ECAM and shutdown the engines. Coordinated with the flight attendants that we were OK and would be towed in. Airport personnel said they saw hydraulic fluid leaking. Uneventful tow in to gate.

Narrative: 2

[Reports adds no additional information]

Synopsis

An A320 ECAM alerted HYD G LO PR on landing and with the resultant loss of nose wheel steering the aircraft veered left and was uncontrollable.

ACN: 1095755 (1983 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Fuel Line, Fittings, & Connectors
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1095755
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Climbing out of 10,000 FT the aircraft began to vibrate quite a bit. As we climbed out we tried varying the pitch and speed and recycling the landing lights. We found the vibration was speed sensitive. I contacted the A Flight Attendant who stated the forward galley was vibrating, but the most extensive vibration was overwing. We request of ATC to level at 310; we found we had to maintain a speed of .72 or less to keep the vibration under control. We had a current and qualified, A320 Captain, commuting in the cabin. I ask the A Flight Attendant to put him on the interphone. I had the pilot walk the cabin and evaluate the situation. After doing so he came into the flight deck and discussed the problem with us. He had found that the greatest vibration was from overwing to four rows in front of overwing. There was also vibration in the forward galley and flight deck. He felt the problem was either landing lights failed to the down position or an open panel of some type. The First Officer and I had already discussed that the vibration felt like extended lights. After our discussion with the other pilot we also suspected that the fueling bay door might have come open. We attempted to call Dispatch three times using the flight deck in-flight phone. We were the lucky winners of three dropped calls using the flight deck in-flight phone. We ended up talking to dispatch using Commercial Radio. We explained to Dispatch that due to our weight we would be close to an overweight landing if we returned to the departure airport. But we would only be able to maintain .72 if we continued to our destination. The decision was made to return. We got the aircraft turned around and then consulted the Flight Operations Manual as to if we needed to declare an emergency. We felt the situation did not meet the emergency criteria. We elected to go with precautionary landing. I called the A Flight Attendant and told her to prepare for a precautionary landing. I also briefed the passengers about our return to the departure airport. 

We used approved techniques, lower altitude, gear; speed brakes to increase fuel burn to get the aircraft below maximum landing weight. We landed without further incident. Upon arrival at the gate we had to wait several minutes for a ground crew to marshal us in. After shut down we were told that the aircraft was found to have the fueling bay door open. There was quite a bit of damage to that area of the aircraft. The door had started to separate from the aircraft. I believe the only reason it had not done so was because we had slowed our speed. Part of the door hinge was missing and the electronic wiring inside the fuel bay had been damaged. We deplaned the passengers and had a post incident brief with the flight attendants. I showed them the pictures of the aircraft damage. We change to a different aircraft and continued to the planned destination.

Synopsis

A320 flight crew returned to their departure for a precautionary landing due to an inflight vibration during departure climb. Post landing aircraft inspection revealed the fueling bay door was open.

ACN: 1095485 (1984 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DCA.Airport
State Reference : DC
Altitude.AGL.Single Value : 900

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DCA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : DCA

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : DCA
Aircraft Operator : Military
Make Model Name : Helicopter
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class B : DCA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 19500
Experience.Flight Crew.Type : 9500
ASRS Report Number.Accession Number : 1095485
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Detector.Automation : Air Traffic Control
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

It was the end of a long day and the events were happening very quickly during a high workload portion of the flight. As all pilots know, the River Visual is a very demanding approach during the best of circumstances. It is very labor intensive when done at night during periods of high traffic volume. The advertised approach in use at DCA was the LDA 19 with VASI being out of service. During the initial approach, Potomac asked if we would accept the charted River Visual approach (VASI back working) and we did. At approximately the 7 DME point we were in the end process of configuring the airplane for landing and the First Officer was accomplishing the Landing Checklist. The radio chatter was considerable and the Tower at some point advised us of helicopter traffic ahead and to the left (I believe he said 10 to 11 o'clock) and he was going to pass from left to right, below us and he "HAS US IN SIGHT." It has to be noted that these traffic calls and helicopters have unfortunately become background noise. There are always numerous military and government helicopters running up and down that river at all times of the day and night. Because of this, what would normally be alarming at any other airport in the country has become commonplace at DCA. The FAA allows these aircraft to operate in this environment and we have no choice, but to accept it and deal with it. Therefore, the helicopters being very close are not out of the norm. As you can imagine, I was concentrating on the approach and doing the required gyrations to stay over the river and not violate the White House prohibited airspace, and to keep the aircraft on the proper vertical profile. The Runway 19 visual was loaded in the FMGC and I was using it as a backup to my eyes. I was 'dead on' the donut and everything looked normal. I was at or very slightly above the recommended altitudes at each DME point. As I was approaching 4 DME, I caught a glimpse of the helicopter out of my left window and he did look higher than I expected to see him. I asked the First Officer to confirm if he was going to cross from north to south. He confirmed, however I directed him to ask the Tower and confirm it with them because it did not look right. The First Officer could not get a word in with the Tower. It was at that point, we got a TCAS "TRAFFIC" call. Again, this is not out of the 'norm' for DCA. I was getting ready to take possible evasive action because it was getting uncomfortable; all this while trying to fly the demanding profile. Within seconds, we got a TCAS alert, "MONITOR VERTICAL SPEED." I complied with the TCAS alert and was thinking of going around but it all happened so very quickly that the point was moot. The helicopter passed 200 FT (MAX) below us. We were at 900 FT, slightly above the path. The Tower never called out the traffic beyond the first notification. The landing was uneventful. Due to the workload, we asked the Ground Controller for the Tower number and I called when I got to my vehicle. I called the Tower and spoke with the Controller in charge. At first it was it was obvious he was somewhat on the defensive and I had to press the point and ask very specific questions about the standard helicopter operations in the area. After some prodding, he admitted that it was a military helicopter flying from/to Ft. Meade and that he was at 700 FT and was "a little higher than normal." I asked what altitude was he supposed to be at and he replied, "three to four hundred feet." In addition, he did admit that the Tower version of collision avoidance did go to alarm but before they had a chance to react, the event was over. I cannot imagine what business is so pressing that these helicopters are allowed to cross the path of airliners carrying hundreds of people! I do not understand why they are not crossed IN-BETWEEN arrivals. They have at least 4 miles between these arriving aircraft and it seems that would be more prudent.

Synopsis

A320 Captain experiences an airborne conflict with a military helicopter at 900 FT during a River Visual to Runway 19 at DCA. TCAS RA states to monitor vertical speed with the helicopter 200 FT below the A320.

ACN: 1095230 (1985 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 38000

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAB

Person

Reference : 1
Location Of Person.Facility : ZAB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1095230
Human Factors : Other / Unknown
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working Sectors 91/65 combined. We were 15 miles in trail to LAX; we had MIT to LAX all day shift. I had an A320 on a vector for traffic, and then turned back to PKE for flow. I took a handoff on a CRJ off TUS to LAX. I turned them to the right to go behind the A320 since the CRJ was never going to stay in front. Right about the time I started nudging them back to the west to stay clear of GLADDEN, Sector 42 yelled across the room and said he had another guy off PHX to LAX, right under both of the other two [aircraft] I had. It was another CRJ, can't recall the call sign, I forgot to print the strips up because I was fuming. The FLM called TMU to see if they could buy off on 10 MIT, there was no one on the north flow to compete with, they said yes. I APREQed the A320 direct TNP to stay out in front with ZLA39, which they approved. I slowed the first CRJ to M71, and the second to M70 once I got them from Sector 42 who had initiated a vector. At the same time, I had a C550 landing HII (Lake Havasu), who was going to need to get down and was stacked on top of this LAX mess. I vector them out to the left to get them down because I had other traffic that needed to descend for LOA restrictions. At the same time, I had the B737 direct IPL that I nearly missed a point out to ZLA60 all because I was so busy with the LAX spacing. And the releases been properly timed, I never would have missed the point out on the B737. So, we were 15 MIT in LAX, but I had three of them with three miles front to back, with limited space and only about 60 miles to get the extra 27 miles. First, the ZAB TMU uses ACM (which isn't pushed to our scopes) as a means of scheduling releases off the various airports for departures to LAX. TUS and PHX are fairly close to the ZLA boundary, so the ACM function to the controllers doesn't work because of the freeze horizon. The problem with scheduling these times is that it doesn't really take into account the miles in trail since it is time based metering and miles in trail is a mutually exclusive function (you can't do in trail and hit times, doesn't make sense). The TMU should be giving times based on the miles in trail rather than based on the times into ZLA's airspace. We are constantly having to vector airplanes because of the TMU problems here, especially with LAX spacing. Second, a concerted effort should be started to figure out how to overcome the bad design of the freeze horizon with hub airports. It doesn't make sense to impose miles in trail when time based metering might mitigate some of the vectors.

Synopsis

ZAB Controller described an unsafe condition during extensive flow control limitations for LAX, the reporter noted the variances of several flow measures and the pitfalls of each effort.

ACN: 1094921 (1986 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Central Computer
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Flight Data Recording
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Function.Maintenance : Inspector
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Inspector : 10
Experience.Maintenance.Technician : 12
ASRS Report Number.Accession Number : 1094921
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

This statement is in response to an inquiry/notification as to how/why the wrong Part Number (P/N) for a Centralized Fault Display Interface Unit (CFDIU) was installed on an A320 aircraft. 

When I was assigned to work on the A320, there was a chronic non-routine load to change the CFDIU as a precautionary step to help troubleshoot an intermittent fault message. The part number was listed, and the part was shown as ordered and ready for pickup. When I removed the existing unit and unboxed the new unit it was apparent that the new part was not the same as the one being replaced. I then went to the Illustrated Parts Catalog (IPC) for the aircraft type, (A320), to verify that the part number for the new unit was in fact correct for that aircraft. The applicable section for the IPC was anything but clear, because of the numerous notes re: revisions, parts shown as being forward and backward interchangeable, numbers superseded, and being pre-modification (pre-mod.), post-mod., etc. After reviewing the available information, I determined that the part in question was acceptable to use on that aircraft. The part was installed per the Aircraft Maintenance Manual (AMM) and the required Operational (Ops) Check was performed and passed. 

The company has since informed me that I was incorrect in my determination that the part in question was applicable to that particular aircraft. I have also been informed that the IPC which was in use at the time was in fact, incorrect, but has now been corrected.

In retrospect, it seems that a combination of factors including: unfamiliarity with fleet type, unfamiliarity with the merged air carrier's manuals, maintenance records and procedures, as well as errors in the manual system itself, contributed to the inadvertent error. Although I realize and understand that as the installing Technician, I am ultimately responsible for installing the correct part.

The incorrect part has already been replaced during subsequent maintenance. [I] have spoken with company officials regarding the events and contributing factors.

Callback: 1

Reporter stated the way the merged air carrier's Illustrated Parts Catalog (IPC) references for the CFDIU was worded, almost made no part acceptable as a replacement. Although the IPC has been corrected, he was given time-off for a discrepancy involving multiple factors that he did not have control over.

Synopsis

A Line Aircraft Maintenance Technician (AMT) reports he installed an incorrect Centralized Fault Display Interface Unit (CFDIU) in an A320 that had chronic, intermittent fault messages. The CFDIU had passed all required Operational Tests. The merged air carrier's Illustrated Parts Catalog (IPC) was found to be confusing with incorrect reference data.

ACN: 1094709 (1987 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : MIA.TRACON
State Reference : FL

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : MIA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1094709
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We followed pre-departure procedures per FOM and PH, including route verification and departure briefing. After takeoff, when handed over to Departure Control, we were told that we were not supposed to turn on the RNAV departure and instead we were supposed to hold a heading. We explained that we had not received that amended clearance via PDC NOR had the Tower told us about it. ATC's reply was "It was on the ATIS." Although I have heard ATIS information say things like "expect to fly the RNAV departure" or "expect to fly runway heading," I have never gotten an amended release to a PDC clearance via ATIS. Out of MIA, in fact, we are sometimes assigned a heading by the Tower before takeoff. NOTE: I am new on the Airbus, and ATIS arrives to us via a piece of paper. I noted the ATIS we got after this event and it was Arrival ATIS. Apparently, this is always the case when requesting ATIS via the MCDU. The "amended clearance" was on the DEPARTURE ATIS which, in hindsight, is something that we should have gotten over the radio. 

1) There should never be an amended clearance delivered over the ATIS, especially if it is not reflected on the PDC. 
2) We got arrival ATIS instead of departure ATIS. 
For # 1: Tell ATC to stop that confusing procedure. PDCs should never be amended via ATIS. 
For # 2: Emphasize that sometimes airports have different ATIS; one for arrival and one for departures...and put out a bulletin emphasizing the importance of getting the correct information for the next phase of flight.

Synopsis

A320 Captain reports departing MIA without a revision to their PDC issued via Departure ATIS. ATIS requested via ACARS is Arrival ATIS and did not contain the revision.

ACN: 1094452 (1988 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY
Altitude.MSL.Single Value : 1100

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : JFK

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1094452
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Fatigue

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

This was leg two of a continuous duty period pairing. Flows, checklist, and SOP were followed while parked at the gate. These included and were not limited to: the route verification, departure procedure including the briefing pages. I even went as far as placing a radius disk at the JFK 4 NM. We discussed in length the Breezy Point climb on the JFK 1 departure. When we were departing the ramp via V for Runway 31L, taxi instructions were A, hold short of H. I read it back and Captain repeated but instead of turning right, he heard turned left on A. I tried to catch it by stating that H was in the other direction but we were committed to the turn. Within seconds, Ground corrected us and gave us new taxi instructions which included a turn which we missed and told us to contact clearance on a different radio for a re-route. Ground gave us new taxi instructions which were join A, short of H. Before switching frequencies I verbalized "Short of H." The Captain responded "Short of H, I got one" [radio]. I left Ground and contacted Clearance. I was writing down the new clearance but was distracted because I was trying to keep tabs on the taxi route. I finally captured the new route and returned to Ground frequency as we were coming to H. Ground asked how much longer because they were about to turn the airport around. I stated 3 minutes, in which time I reprogrammed the FMS. Verified it with the Captain who had noted the new route was in the coded departures. He sent a message to Dispatch, while I took down the continuation of the taxi route to the Runway 31L. New taxi instructions were cross 22R, right on Z and hold short of 31L. I read it back. We crossed 22R and Captain called for flight control check and Taxi checklist when ready. We completed the Taxi checklist to the line when I switched to Tower. Tower called and asked if we were ready. Capt said "yes" and below the line items on the taxi checklist. I completed the flow, finished the checklist and gave it one quick look at the JFK 1 departure which states, "Via vectors to assigned route/fix. Expect cln to filed alt/fl 10 min after departure." Wavey departure expect vectors to JFK R-156. I stated out loud what I read, while Tower cleared us for takeoff. Then we were airborne, going straight into LGA airspace because we forgot all about the Breezy Point Climb. NY Departure turned us to a heading of 180, then on a heading of 130. Informed us of our error. We apologized. NY Departure said no worries and handed us over to NY Center. 

Contributing factor to this incident was the fact that the crew had been on a continuous duty period since [9 PM] EDT the night before. Although we got our scheduled break of 6 hrs block to block. We flew a fairly full aircraft to JFK. After deboarding, van waiting, ride and signing in, an hour of our rest had been utilized. Leaving us with no more than 3:20 at the hotel since we were scheduled for a 5 AM van. I myself had only 90 min of sleep prior to this incident. In my opinion, these continuous duty period schedules are dangerous. It is almost impossible to adapt to the change in the sleep pattern. In addition, distractions due to previous errors made on the taxi compounded by continuous taxi instructions and having to change frequencies to pick up new route clearances caused this incident. Once the new route was in the FMS we realized it was going to be a longer flight and not wanting to delay the flight any further we rushed to complete the tasks at hand. I was trying to do too much with too little. Although, it seemed that everything got done and we did not short cut checklist. By rushing we did forget our original Breeze Point Climb. Eliminate continuous overnight schedules which do not allow for a full FAR crew rest.

Synopsis

A320 First Officer describes errors made during taxi out and takeoff from JFK after a continuous duty overnight, during which minimal rest was obtained.

ACN: 1094134 (1989 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 20500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZAB

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1094134
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

While descending via the Eagul5 Arrival into PHX, I noted a discrepancy in comparing the chart with the aircraft Navigation Display. Specifically, the chart directs crossing EAGUL intersection at or above FL180 (based on QNE; that is, aircraft altimeters set to standard 29.92 in Hg.) The aircraft navigation database, in contrast, directs crossing EAGUL intersection at or above 18,000 FT (based on QNH; that is, aircraft altimeters set to a local altimeter setting.) Please note that the upper clearance limit of FL230 at EAGUL intersection is not at issue here. I queried the Air Traffic Controller as to which lower limit was correct at EAGUL intersection: FL180 or 18,000 FT. He replied that the lowest clearance limit to cross EAGUL intersection is FL180, not 18,000 FT (that is, the chart is correct and the aircraft navigation database is incorrect.) I asked him if he was certain of this and he replied that the lowest clearance limit to cross EAGUL intersection is FL180 and has always been so. I believe, but I am not certain, that this aircraft navigation database discrepancy is present in all A320 series aircraft. It is important that this discrepancy is promptly corrected because a sufficiently low local altimeter setting could result in a clearance deviation/violation/aircraft accident. For example, a local altimeter setting of 29.62 in Hg could cause an A320 series aircraft to cross EAGUL intersection at FL177 - which is 300 FT below the clearance limit altitude of FL180. The performance standards for Airline Transport Pilots delineate a maximum tolerance of 100 FT.

Callback: 1

The reporter states that his company reported to him that this is just the way the A320 is and that keeping QNE until past the fix will prevent an altitude deviation.

Synopsis

A320 Captain reports his FMGC database depicts crossing EAGUL between FL230 and 18,000 FT, on the EAGUL 5 arrival to PHX. The chart, which is correct, indicates the lower limit should be FL180.

ACN: 1092865 (1990 of 2908)

Time / Day

Date : 201306
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : Marginal
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZLC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLC
Aircraft Operator : Military
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Phase : Descent
Airspace.Class A : ZLC

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1092865

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1093018

Events

Anomaly.Conflict : NMAC
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
Miss Distance.Horizontal : 300
Miss Distance.Vertical : 300
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were approximately 25 NM from BAM VOR at 33,000 FT. The ATC Controller told us of traffic ahead, 12 o'clock descending to 34,000 FT and after passing you he will continue to descend. At about 7-8 NM (approximately 20 seconds from the event) we noticed on our TCAS that the aircraft directly ahead of us was at 33,800 and still descending, arrow still next to it indicating more than 500 FPM, I inquired to the ATC Controller, is he going to level off, to which I'm not sure what he responded since things happened very quickly at this moment due to our closure rate. Next thing we see is 500 FT on the TCAS, at that moment the other aircraft breaks out of the clouds the TCAS goes to TA for a fraction of a second, then immediately to RA, descend, and the Controller is saying turn left now. The First Officer disconnected the autopilot, started turning and descending at that moment the traffic was over and behind us at approximately 300 FT according to the TCAS. From the time the TCAS sounded to the event being over a maximum time of 6-8 seconds elapsed! The other aircraft was a B-737 (military) according to ATC and we never heard any communication between the ATC controller and the aircraft in question.

Having military aircraft transmit on MHZ frequencies when operating in commercial RVSM airspace could have provided us with some situational awareness since we would have heard what was being said between them and ATC. Also, checking into why the TCAS didn't provide the normal sequence from TA to RA at the proper time would be worth investigating!

Narrative: 2

I disconnected the autopilot, turned left, and began a descent. I noted the target symbol showed +300 FT and still descending as it passed over us. The other aircraft was a military 737.

A contributing factor was the cloud layer that prevented us from seeing the 737 until they were directly in front of us. We also never heard any communication between ATC and the 737.

Synopsis

TCAS TA and RA warnings were instrumental in separating the reporter's A320 and a military B737 which descended below its cleared altitude. Situational awareness was cited as a result of ATC and the B737 not communicating on the same frequency as civil operators.

ACN: 1091266 (1991 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Flight Dynamics Navigation and Safety
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1091266
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were prepared for a departure from SFO on Runway 28L. As we were holding short of 28L we were given clearance to cross 28L and line up on Runway 28R. The First Officer put in the changes to the box and briefed me on the new V-speeds. They were roughly the same as the numbers for 28L and I specifically remember that V2 was 140 KTS. As we turned onto 28R I looked down at my PFD and saw V-speeds. I also asked him if he was ready and he said yes. When we were cleared for takeoff, I advanced the throttles to the MAN/FLEX detent. As we began rolling, we got a Master Caution with a "ding." I assumed that the flex number had dropped out of the FMC so I advanced the throttles to TOGA to continue the takeoff. We were light and there was quite a bit of wind so the airspeed jumped up and we accelerated very quickly. I was waiting for the V1 call when the First Officer announced that we had no V-speeds displayed on the PFD. At that point we were approaching 160 KTS which I knew was well above V2. I felt it was safe to continue the takeoff and that it was a better decision than to abort at a speed well above V1. I believe that the First Officer changed the runway correctly prior to entering 28L but either hit the wrong button or didn't push hard enough for the runway insertion to take. As we started the takeoff I believe he inserted the change and that's when the speeds dropped off the airspeed tape. By that time I was completely outside and the "ding" distracted me from my FMA check. I just went to TOGA and thought everything was normal except for the lack of a flex temp.

Synopsis

When an A320's throttles were set to MAN/FLEX, a Master Caution and "DING" alerted but the Captain continued the takeoff after the FMC dropped the V speeds because the First Officer did not fully finish the runway change input from SFO 28L to 28R.

ACN: 1091163 (1992 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1091163
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On approach in after putting gear and flaps down at about 1,500 feet AGL ECAM on status page normal brakes INOP and ALT Brakes INOP. We decided to go around and look at ECAM. After go around ECAM changed to brakes AUTO BRK FAULT. Went in for normal landing. Called Ops and Maintenance on ground, wrote up problem in logbook.

Synopsis

A320 Captain experiences an ECAM Status message during approach indicating both normal and alternate brakes are inoperative. A go around is initiated. After go around the ECAM message has changed to AUTO BRK FAULT and a normal landing ensues.

ACN: 1091099 (1993 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : OKC.Airport
State Reference : OK
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1091099
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
When Detected : Pre-flight

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After diverting into OKC contacted Dispatch for new Flight Plan and Release. Flight Plan RLS2 ZFW = 90,135 pounds. Initial RLS01 ZFW = 118,995 pounds. We had 6 passengers deplane in OKC, fuel upload approximately equal to initial RLS01 but I saw that TOGW was 28,000 pounds less in OKC than for initial takeoff. We phone patched with Load Planning and they stated new ZFW was not 91,035 pounds but 118,468 pounds; re-dispatched flight on RLS03 with correct ZFW. Flight uneventful.

Synopsis

A320 Captain reports receiving a release after diverting with a substantial change in ZFW while almost no actual change occurred. Load planning is contacted and the correct ZFW, almost 29,000 pounds heavier, is supplied.

ACN: 1091002 (1994 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZSU.ARTCC
State Reference : PR
Altitude.MSL.Single Value : 1600

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use.Other

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : Twin Otter DHC-6
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : VFR
Flight Phase : Descent
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Facility : ZSU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1091002
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working the R1 Sector and I had Aircraft Y [a] VFR DHC6 turning right base in front of Aircraft X an IFR A320 that was 4 miles in trail for Runway 10. Aircraft Y reduced speed to 80 knots so I told Aircraft X to reduce to slowest practical speed. When they were 2.5 miles apart the Tower gave me the handoff for Aircraft X because they had sent Aircraft X around. Nobody called me for coordination and when I called and asked I told them to let Aircraft X do the approach and send Aircraft Y around but they switch me Aircraft X for re-sequence. No separation was lost on this event. There have been plenty of issues with SJUT and the biggest problem that we have in approach is the attitude and lack of professionalism in handling this type of situation ending up in compromising safety. This has to change, to be a better system and to provide a better service. They give a poor service to pilots and they don't want to cooperate with us starting with their management. This has to stop; many coworkers are frustrated because no action has been taken. Safety is being compromised!

Synopsis

ZSU Controller described a go around event when the Tower sent an A320 around that was following a DHC6, the TRACON Controller indicating the Tower ignored the TRACON's direction.

ACN: 1089975 (1995 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuel Quantity-Pressure Indication
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1089975
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1090118

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

[After fueling was complete] we showed 32.2K on the total fuel quantity gauge and adding the individual fuel quantity gauges together agreed. During the Before Start Checklist, however, the left inboard fuel gauge lost 2,000 LBS and the total fuel quantity gauge showed the 2,000 LBS lost as well.

Maintenance was called and drained 2.5 gallons of water from the left main tank and a company Mechanic stated that fuel sample checks were not being performed as required. Ultimately, mechanics could not fix problem and we refused the aircraft due to class 2 nav.

Narrative: 2

Maintenance reset circuit breakers, waved chicken bones, prayed and changed the fuel processor. Still 10,200. Changed fuel processor, 10,200. Maintenance Control suggested checking for water in tank. The first Mechanic said they drained five gallons of water out of left main, second Mechanic said 2.5 gallons and said every second check, fuel sample should have been taken and, obviously, wasn't.

Synopsis

An A320 flight crew refused their aircraft and canceled the flight when fuel gauge discrepancies occurred followed by the discovery of significant amounts of water in the fuel.

ACN: 1089863 (1996 of 2908)

Time / Day

Date : 201305

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class A : ZZZ

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1089863

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 22500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 11000
ASRS Report Number.Accession Number : 1089867

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout passing about FL260 received ECAM for COMPRESSOR VANE ENGINE 2. First Officer continued to fly aircraft and handle radios while Captain handled ECAM and pulled out QRH. The First Officer notified ATC of request to maintain FL290 while coordinating future plans with possible diversion to ZZZ. ECAM and QRH instructed to disconnect Autothrottles and minimize throttle movements. Captain called Dispatch and Maintenance Control to discuss problem. Aircraft was still north of ZZZ about 120 NM when event occurred. Captain's discussion with Dispatch and Maintenance Control included previous maintenance plans to change the engine later that night in a maintenance base. Maintenance Control also informed crew of only 1 Maintenance Technician at the destination. Crew discussion was whether to continue to destination or divert to ZZZ. After considering all information crew, elected to divert. First Officer notified ATC and aircraft was given clearance direct to a fix for the arrival to ZZZ. A turn was begun direct to the fix and a descent began to FL270. Further clearance was given to FL100 and 250 knots. On descent with thrust lever 2 approach Idle, Compressor Stall occurred and subsequently Engine 2 was shut down and ECAM completed. Emergency was declared and approach was continued followed by an uneventful landing. Aircraft exited the runway and stopped while fire trucks and firemen approached aircraft for inspection. Inspection showed some fluid leaking from number 2 Engine, with no signs of fire and no excessive temperatures. Aircraft was cleared to taxi to gate.

Narrative: 2

On climbout passing FL260, ECAM ENG 2 COMPRESSOR VANE. Followed ECAM, leveled at FL290. Engine parameters normal. Coordination with both Dispatch and Maintenance Control. Initial thought was to divert to ZZZ. After initial discussion with Maintenance Control, we looked at continuing to destination. When told of my aircraft refusal out of destination airport, coordination to divert to ZZZ accomplished and executed. On arrival when thrust was reduced toward idle, Engine 2 COMPRESSOR stalled. ECAM commanded Engine 2 shutdown. Declared emergency with ATC. Continued with arrival and approach. No issues through approach, landing, rollout, and taxi. Met and inspected by emergency personnel. Cleared to taxi to gate. Normal shutdown and deplaning.

Synopsis

A320 flight crew on climbout passing FL260 reports receiving ECAM for COMPRESSOR VANE ENGINE 2. The company wants the crew to continue to destination, then on to maintenance. The Captain declined and the crew diverted to a suitable alternate. During descent the engine compressor stalls and the engine is shut down. A single engine landing ensues.

ACN: 1089672 (1997 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9500
ASRS Report Number.Accession Number : 1089672

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1089682

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climb out we experienced a complete loss of green hydraulic fluid passing through FL180. We completed the ECAM for Green Hydraulic System low level and then referenced the QRH. We then reviewed the alternate gear ext procedure. We discussed the loss of nose wheel steering, reverser one and no green brakes. We contacted Dispatch and coordinated a divert. We then reviewed the landing distance procedure and decided it was safe to land. Although I was the pilot flying at the time of the hydraulic loss, the Captain decided that he should land the aircraft. I then became the pilot not flying and reviewed the Overweight Landing Checklist since we weighed about 149,000 LBS. After we declared an emergency with ATC, we were given vectors direct to the field. We ran all appropriated checklists and alt extended the gear about 7 miles out from the FAF. The Captain made a very smooth touchdown and decided to exit the runway via the high speed. We then stopped the aircraft on the high speed, set the brakes and ran the tow in checklist. We were then towed in.

Narrative: 2

[Narrative #2 contained no additional information.]

Synopsis

A320 flight crew experiences a Green Hydraulic System failure due to fluid loss climbing through FL180. Crew elects to divert to a suitable airport and lands normally after gravity extending the landing gear.

ACN: 1088881 (1998 of 2908)

Time / Day

Date : 201301

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Powerplant Installation
Aircraft Reference : X

Component : 2

Aircraft Component : Powerplant Mounting
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Contracted Service
Qualification.Maintenance : Inspection Authority
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Inspector : 2
Experience.Maintenance.Technician : 33
ASRS Report Number.Accession Number : 1088881
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

As a Quality Control (QC) Inspector at Contract Maintenance Repair Station X, I inspected the installation of an [IAE] V2500 series engine on Customer X's Airbus A320-200 aircraft. I failed to verify the proper installation of a Retainer Assembly for the Forward Engine Mount. Reference Customer X's A320 Task Card 71-00-20, Item-19, Step-3. The step by step sequence of Item-19 is broken because the Engine Hoist Assembly has to be removed to get the final torque of the mount bolts. With proper tooling, the sequence of this task would not have been broken, and this critical step would not have been missed. As an Inspector with RII authority, I should have been aware to the reason for this deviation in the Task Card. After investigation, it was found that this is a standard deviation for Repair Station X. [This] Repair Station is in the process of correcting this process. I don't recall this event that happened in January, 2013. At times there is only one Inspector on the floor with three aircraft in maintenance. This may have also been a factor. In the future I will be more diligent in my duties as an Aircraft Maintenance Inspector.

Callback: 1

Reporter stated he was informed a channel type retainer clip was found missing at the Forward mount on a Customer's V2500 engine. The channel clip is used to secure the engine mount bolts that are installed with bolt heads upside down; to prevent the mount bolts from falling into the engine's diffuser case if the mount bolts sheared or if anyone of the nuts securing the bolts came off. The missing retainer clip was noticed during an A-Check by one of the Customer's Line Maintenance Technician's several months after he had signed-off the A320 aircraft's inspection for the 2nd part of the Engine Change paperwork.

Synopsis

An Aircraft Inspector at a Contract Maintenance Repair Station reports that a sequence of Task Card steps had been broken due to a lack of proper tooling during installation of a V2500 Engine on a Customer's A320 aircraft. He missed a required step to verify the proper installation of a retainer assembly at the Forward Engine Mount.

ACN: 1088120 (1999 of 2908)

Time / Day

Date : 201305
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Autoflight Yaw Damper
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : FAC (Flight Augmentation Computer)
Aircraft Reference : X
Problem : Improperly Operated

Component : 3

Aircraft Component : Minimum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1088120
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Arrived at the gate at ZZZ with the flight delayed and four mechanics in the cockpit. Much debate as to what was going on, but what I got from the discussion, the plan was to reset a malfunctioning #1 Yaw Dampening System and then run an engine at the gate to test the Yaw Dampening System. This required resetting both Flight Augmentation Computers (FACs), as I knew from 10+ years A320 experience, having flown the A320 in both seats as well as having been an Instructor on the A320/319 for 3.5 years. The Mechanic running the test failed to reset the FACs correctly, mentioning to me "I can't get #2 FAC to disengage." He believed this because the FAC-2 'Off' light on the Overhead Panel was not coming on with the switch, I could see the switch was working properly as the ECAM was showing the #2 FAC OFF and the switch was working correctly. 

The Mechanic was being distracted because the 'OFF' light in the switch [was] not coming on, which is a common anomaly on the A320 Overhead Panel, usually fixed with a series of light finger taps on the switch. I could tell my presence in the cockpit was not making anyone comfortable, so I departed and let the mechanics do what they had planned. Being unable to reset the FACs, the Yaw Dampener Fault never cleared and the engine run was cancelled. Before the mechanics left I offered to reset the FACs and doing so properly, the Yaw Dampener problem was cleared and the ECAM Status page was 'NORMAL'. At this point the Mechanic said he had only had a three week course on the A320 and did not know the airplane well. Now with a 'NORMAL' status page I asked why we could not test the Yaw Dampener system now, the Mechanic said "OK, let me talk to my Supervisor." He left for about 20 minutes and when he returned, he said "OK, we are all set, I am going to defer #1 Yaw Dampener." I was a bit miffed as the original plan was to clear the fault and then test the system, but since they did not know how to clear the fault the test was aborted, now the fault was gone, why not test? Why did we have to [make] 'Inoperative' (Inop) a system that was currently not broken without even testing it? The Mechanic could not verbalize to me why we were doing this, only that the decision to defer had been made to avoid more problems with the #1 Yaw Dampener. 

Given the progressive delay and having flown A320s in the past with a Yaw Dampener Systems deferred, and in the interest of getting the passengers to destination, I agreed to fly the aircraft with the #1 Yaw Dampener System deferred. I was on a phone call with Dispatch and Maintenance Control while the Mechanic was carrying out the procedures to defer the #1 Yaw Dampener. When the Mechanic left, I noticed he had shut off the #1 FAC, put collars around the #1 FAC Circuit Breakers (C/B) and put colored stickers on the #1 FAC Switch. Looking at the new Maintenance Release coming over the printer, it indicted that the #1 Yaw Dampener was deferred but made no mention of the #1 FAC being differed or it's circuit breakers being pulled and collared. I realized that the Mechanic had clearly not performed the appropriate procedure to defer the #1 Yaw Dampener and had instead inoped a perfectly good FAC, while leaving the suspected #1 Yaw Dampener active and then issuing me a Maintenance Release for the aircraft that stated the #1 Yaw Dampener was deferred and had been deactivated. 

In summary: 
1). System that was to be deactivated was not deactivated (#1 Yaw Dampener). 
2). Perfectly good system (#1 FAC) was incorrectly deactivated. 
3). Maintenance Release issued for the flight stated that the #1 Yaw Dampener had been deferred.
4). Mechanic tasked with 'fixing' the problem admitted to me he had only had a three week training program on the A320 and In My Opinion (IMO) clearly demonstrated a lack of systems knowledge to be performing maintenance procedures on the A320/319 series aircraft. 

After a series of phone calls with Maintenance Control, Dispatch and the Duty Manager the #1 Yaw Dampener was eventually deferred correctly and a correct Maintenance Release was issued. We departed ZZZ after an approximate three hour delay and had an uneventful flight to ZZZ1. I will say in my 18+ years at Air Carrier X, I have never experienced anything remotely this disorganized or troubling in terms of getting appropriate maintenance performed and correct maintenance documented. The only thing that stopped an aircraft from flying with 100+ people on board, after having incorrect maintenance actions being performed with an illegal Maintenance Release issued was me. In this case, a combination of luck and A320 experience allowed me to catch this significant error before it impacted the safety of my passengers and crew or triggered certificate action by the FAA. But 'luck' should not come into play in a business as serious and potentially deadly as transporting people in airplanes.

Callback: 1

Reporter stated the lack of Mechanic training on Airbus A320/319 aircraft is really noticeable at certain Stations. Many of his fellow pilots have also expressed concerns about how the lack of Maintenance experience on A319/320 aircraft have contributed to troubleshooting difficulties, an increase in improper maintenance procedures, lack of proper documentation of work accomplished; all of which are raising concerns about the safety of the aircraft for dispatch. Over time, the effects of their merger have really started to become apparent.

Synopsis

A pilot reports that in all the years he has flown for his air carrier, he has never experienced anything remotely as disorganized or troubling in terms of getting appropriate maintenance performed and correct maintenance documented on their merged air carrier's A319/320 aircraft. Lack of maintenance experience and training was apparent at certain Stations.

ACN: 1087836 (2000 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 1500

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : LOOP6

Person

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1087836
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

An A320 departed Runway 25 from LAX on the LOOP SIX departure (non-RNAV SID). A B737-700 departed Runway 24 simultaneously with the A320. The LOOP SIX departure has Runway 25 departures, "Climb heading 250 to cross SMO VOR/DME R-160 at or below 3,000 [feet], then turn left heading 235 expect RADAR vectors." Having received this information long after the event, I don't recall having any issue with this departure. However, upon reviewing the replay, I noticed the following: On the replay display, the centerlines of both runways are displayed, which are not displayed on my scope and it appears that the A320 did turn north of the 250 degree heading for approximately a mile prior to turning to his 235 degree heading. Having worked in the area for 15 years, I am used to occasionally seeing an erroneous RADAR hit that makes it appear the aircraft are off course having the next hit show that the aircraft is indeed in the correct position. Also, the display I reviewed was at a 10 mile range while I look at a 33 mile range on my scope so to see the slight turn north is very difficult. If anything, this was a pilot deviation as indicated by the turn north of the aircraft. The best way to prevent this from happening again is to convene a work group of controllers from LAX Tower and SCT to review and rewrite the departure procedures off of the LAX airport. Possibly preventing simultaneous departures from the 2 complexes, or having the Tower ensure the aircraft on non-RNAV SID's make the turn prior to switching the aircraft to SCT.

Synopsis

SCT Controller was notified of a loss of separation that had been discovered during a post event review long after the fact, the reporter claiming the tools available to the Controller are not as precise as those used during the post event reviews.