ACN: 1085023 (2001 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Normal Brake System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Indicating and Warning - Hydraulics
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Minimum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1085023
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Primary Problem : MEL

Narrative: 1

The aircraft arrived with MEL 32-60-04-XX: Fault(s) Indicated by BRAKES N/WS MINOR FAULT Caution on ECAM E/WD, Not Confirmed by a Brake Pressure Transducer Fault??? which was added a couple days earlier. This MEL requires Alternate Braking System is checked 'operative' before the first flight of each day. Although it wasn't the first flight of the day for the aircraft, it was our first flight of the day on that aircraft, and for peace of mind, we decided to perform the Alternate Braking System Check described in the MEL. The test failed; the Alternate Braking System did not work correctly. This caused a five hour delay. It is very likely the aircraft flew several legs without an operational alternate braking system. It was subsequently discovered that the Left Normal (Green) Brake Pressure Transducer was faulty (it constantly sensed 2,000 PSI pressure regardless of the actual pressure). So it's unclear to me if it should have instead been dispatched under MEL 32-60-04-XY; 'Fault(s) Indicated by BRAKES N/WS MINOR FAULT Caution on ECAM E/WD', following a Confirmed Fault on a Brake Pressure Transducer'. 

I believe several brake related MEL items are unsafe because they allow aircraft to be dispatched without an operational ALTERNATE Braking System. The MEL procedures related to faulty brake pressure transducers seem to be correct only when a faulty transducer does not sense pressure and indicates zero pressure. But the procedures don't make sense when a faulty transducer erroneously indicates a positive pressure. Even though MEL 32-60-04-XY was probably the correct MEL and 32-60-04-XX was applied instead, the situation would have been less safe and would not have been recognized if the correct MEL was applied, since 32-60-04-XY doesn't require an Alternate Brake Check at all. MEL 32-60-04-XX says the 'BRAKES N/WS MINOR FAULT' ECAM message may be displayed provided: 
A). BRAKES pressure indicators operate normally. 
B). Alternate Braking System is checked 'operative' before the first flight of each day, and 
C). It is checked prior to each departure that the following Centralized Fault Display System (CFDS) failure message related to BRAKES N/WS MINOR FAULT is not displayed: 
A fault on Alternate Pressure Transducer. 
Condition A and C were met on all flights. 
Condition B was met only on the first flight of each day. 

With a faulty 'Normal' brake pressure transducer erroneously sensing a positive pressure, the Alternate Braking System would test OK on the first flight of the day (if the aircraft electrical power was cycled off overnight), and all the conditions to dispatched under that MEL would be met. However, the Alternate Braking System would subsequently not be operational. That's because after powering up the aircraft, the braking system is defaulting to ALTERNATE and will pass the test described in the MEL. But once the 'Normal' Braking System is used (the Green Hydraulic System is pressurized and the brake pedals are pressed), the BSCU's won't automatically revert to ALTERNATE braking in case of loss of Green hydraulic pressure if they sense pressure from a 'Normal' brake pressure transducer (the ECAM WHEEL page will indicate the Braking System is in Alternate, but there will be no brakes available unless the A/SKID and N/W STRG switch is turned 'Off' or the parking brake is used). Similarly, MEL: 'BRAKES Pressure Indicators' allows a flight to be dispatched with a faulty alternate pressure transducer. But I'm not certain it should [be] allowed to do so with a pressure transducer erroneously sensing a positive pressure. Was the BSCU logic taken into account for this case? (Specifically, after landing gear extension, when the BSCU tests the braking systems.) 

I think MEL 32-60-04 and probably MEL 32-44-03 should not apply when a brake transducer erroneously senses a positive pressure. I think the 'Alternate Brakes Check procedure in MEL 32-60-04-XX should begin with steps to pressurize the Green Hydraulic System and apply 'Normal' brake pressure, then test the automatic reversion to Alternate Braking System after losing Green hydraulic pressure. Also, I think MEL 32-60-04-XY should require an Alternate Brakes Check. Flight delay. Procedures inadequate.

Callback: 1

Reporter stated the initial trigger was the earlier deferral of Faults Indicated by BRAKES N/WS MINOR FAULT Caution on ECAM E/WD applying MEL 32-60-04-XX. He decided to perform the Alternate Braking System Test at the gate on their A320 aircraft by activating the Yellow Hyd System Electric pump and pressurizing the Green Hyd System using the Power Transfer Unit (PTU). Then positioning the PTU switch to 'Off' and viewing the ECAM Hyd System Page showing the loss of the Green Hyd System pressure. He then applied the brakes and should have seen Yellow Hydraulic System pressure to the Alternate brakes on the Triple Indicator, but no pressure was indicating. At that point the Alternate Braking System was not functional as required per MEL 32-60-04 and the aircraft took a five hour delay. 

Reporter stated further troubleshooting by Maintenance during that time noted a Brake Pressure Transducer error code that corresponded to the Normal Green Hydraulic System. The pressure transducer had failed to a 'positive' hydraulic pressure, always showed 2000 psi of pressure, preventing the automatic reversion from Normal Green hydraulic pressure to Yellow System pressure for the Alternate Braking System. Maintenance swapped the faulty Green Hyd System transducer with the good Yellow System pressure transducer to verify the Green pressure transducer was showing a constant pressure on the Yellow System Brake Pressure Triple Indicator even if with Yellow System pressure off; which it did. Maintenance then performed another test of the Alternate Braking System that was successful because the 'good' pressure transducer from the Yellow System that was installed in the Green System, allowed the automatic reversion of the Alternate Braking System to occur. The Green and Yellow Braking Systems seem to function properly then. That's when MEL 32-44-03 was applied showing a pressure indication deferral in the Yellow System.

Reporter stated the issue of deferring a Green Hyd System pressure transducer that has faulted to a constant 'positive 'pressure reading under MEL 32-60-04 should not be allowed because automatic reversion to the Alternate Braking System will not occur, even if the Green System fails, because the Normal Green pressure transducer will continue to show a 'positive' pressure in the Green System.

Synopsis

A Captain reports that several brake related MEL items are unsafe because they allow A320 aircraft to be dispatched without an operational ALTERNATE Braking System. Deferring a faulty Brake Pressure Transducer that erroneously indicates a 'positive' hydraulic pressure in the 'Normal' Green Hydraulic System under MEL 32-60-04 would prevent automatic reversion to the Alternate Braking System even after losing Green hydraulic pressure.

ACN: 1084307 (2002 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ONT.Airport
State Reference : CA
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ONT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : ONT

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9500
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 6600
ASRS Report Number.Accession Number : 1084307
Human Factors : Distraction
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1083727
Human Factors : Training / Qualification
Human Factors : Distraction

Events

Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Miss Distance.Horizontal : 0
Miss Distance.Vertical : 450
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were cleared for a visual approach to Runway 26R at Ontario, CA. We were configured and descending on the glide slope. At about 5 miles from the runway Tower called traffic at 11 o'clock. They stated they were not in contact with them and intentions were unknown. We began looking to see the traffic and saw it on TCAS. It was about 3 miles away. The terrain and buildings on the ground made it impossible for us to visually acquire the traffic. Then the TCAS target went to a TA. We were still trying to find the traffic and then the TCAS RA occurred. I started a shallow climb to comply with the vertical guidance. The traffic passed 450 FT below us, and we were never able to see the other aircraft. After clear of conflict received, we reported the TCAS climb to the Tower. Then I started to descend again and recaptured the glide path. Then we made a normal landing.

Narrative: 2

[Report contains no additional information.]

Synopsis

An A320 crew on approach to ONT Runway 26R took evasive action to a TCAS RA as an aircraft passed about 450 FT beneath their aircraft.

ACN: 1084306 (2003 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 8500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 5508
Experience.Flight Crew.Last 90 Days : 152
Experience.Flight Crew.Type : 4545
ASRS Report Number.Accession Number : 1084306

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 255
Experience.Flight Crew.Type : 6700
ASRS Report Number.Accession Number : 1084320

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climbout we had an LGCIU #2 failure and completed the QRH procedure and reference actions for the failure. Enroute we requested and received landing data and determined no issues existed with our planned landing. On descent through approximately 8,500 feet MSL for landing at the Captains (PF's) direction I placed the landing gear handle in the down position with no resulting gear extension. We completed the ensuing ECAM and QRH procedures for the L/G Gear Not Down Locked event without success and accomplished a go around. The go-around was normal considering the aircraft configuration. Established on a downwind we successfully accomplished the L/G - Gravity Extension QRH procedure, declared an emergency due to the loss of nose wheel steering capability, and completed a normal approach and landing with approximately 4,000 FOB.

Narrative: 2

At cruise the LGCIU #2 ECAM appeared. On final approach when landing gear was selected down nothing happened. Following ECAM procedure the landing gear handle was recycled, with no effect. We went around. On downwind we manually extended the gear and got three green indications. Landed normally and cleared the runway at the high speed turnoff and stopped (No nose gear steering). Tug came and towed us to the gate.

Synopsis

A320 flight crew reports failure of LGCIU Number 2 in cruise. During approach the landing gear fails to extend when selected, and the gear handle is cycled to no avail. A go-around is initiated and the landing gear gravity extension procedure is followed successfully extending the gear.

ACN: 1084128 (2004 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1084128
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The Captain was the PF and I, the First Officer was the PM. Just as we started to configure the aircraft, we got an ECAM message stating that the flaps were locked due to the actuation of the wingtip brakes, with a note not to exceed 200 knots. We were at approximately 2,500 feet at the time and executed a missed approach and notified ATC. We were given vectors and an altitude. The Captain kept control of the aircraft staying well below 200 knots and took over ATC communications. I pulled out the Manual and searched for the appropriate procedure. Per the slat/flap indicator on the upper ECAM, our flaps appeared to be 0, while the slats were working just fine. Since we had recently switched over to the "new" Manual, we both were determined to ensure that we were accomplishing the right procedure, along with the right landing distance calculations and corrections to our landing/approach speed. Not only did we maintain a continued dialogue throughout the whole event, but I went thru the procedure several times, and we even handed over controls so that he (the Captain) could review the Manual procedures and make sure that we got it right. We notified company and had a message relayed to Maintenance, hoping to get some helpful guidance. The message back was just to follow the Manual. In the meantime, we were vectored around for another visual. There was very little traffic in the area so we were not really concerned with sequencing behind other aircraft. We discussed our landing distance calculation (5,000 feet or so from what I can recall) and we discussed our landing speed, which would be flown 25 knots faster per the procedure, and decided as a precaution, to have emergency equipment standing by. We briefed the flight attendants, got new weather (VFR still) and completed appropriate checklists. We just treated it as a slightly faster than normal landing using max reverse, low brakes, with everything else being fairly normal (other than the flaps.) It was an uneventful approach to an uneventful landing. The emergency vehicles followed us to the gate and we were met there by our Maintenance personnel. 

One issue concerning this event was that while trying to involve Maintenance in the process, we received word from them that we should just follow the procedure. Fair enough, that is just what we did, but we would have liked to get them more involved in the process, as we value their opinion on all matters mechanical. They may not have had anything to tell us that would have made any difference in the actions we took, but it's always nice to have them in the loop during these kinds of situations and to receive some kind of helpful feedback. Secondly, I do not want to speak for the Captain, but for me, trying to follow an unfamiliar "new" Manual procedure for a no-flap landing with partial slats was a bit unnerving. The procedure itself is really fairly simple now that we have seen it and ran through it in real time, but it IS different than the old procedure that I ran in SIM training. We spent a lot of time during this event running and re- running the procedure, when it could and should have "flowed" better. I think we knocked out all the appropriate checklists, but I just can't be sure. Overall, I think we did a good job, but I personally would have felt more confident about it with a recurrent SIM (with the new Manual) under my belt.

Synopsis

A320 First Officer reports a flap fault with wing tip brakes locked during approach. A go-around was initiated in order to comply with new procedures. Eventually a zero flap with slats 3 approach and landing ensues, flown at Vref+25 to an uneventful landing.

ACN: 1083201 (2005 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZOA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZOA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZOA

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13585
Experience.Flight Crew.Last 90 Days : 205
Experience.Flight Crew.Type : 5805
ASRS Report Number.Accession Number : 1083201

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 1083208

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

About 30 minutes into the flight we experienced an emergency descent ECAM at FL370. We accomplished the immediate action items, donned oxygen masks and declared an emergency. We initiated a descent to 10,000 FT. As we continued with the ECAM the pressurization stabilized at a normal position. We quickly assessed that the cabin had never exceeded approximately 8,000 FT. Since the aircraft was stable we agreed that there was no need to continue down to 10,000 FT. We coordinated with ATC to level at FL250. We called the flight attendants and they said everything in the cabin was fine and the passengers hardly noticed anything. We then continued with SOP. We referenced the flight manual because the ECAM was complete and we were left with a pressurization system 2 failure. The Flight Manual states that if CPC 2 fails the number 1 system will pick up the load. That appeared to be what happened. After a lengthy discussion and a systems review we agreed that there must have been a system delay in the transfer of CPC control from system 2 to system 1 thereby leading to a partial loss of pressurization resulting in the ECAM. We then discussed the situation with Dispatch and Maintenance. After a lengthy discussion we all agreed that continuation to destination at FL250 was the best course of action. We terminated the emergency with ATC. Finally we made a PA giving a brief description of events to the passengers.

Narrative: 2

Shortly after level off in cruse an ECAM was annunciated depicting loss of cabin pressure and to initiate emergency descent. Both pilots noticed a small change in cabin pressure in the inner ear but not dramatic. The ECAM warning was silenced and the Captain began the emergency descent profile while the Copilot informed ATC and the cabin. Noting that the Cabin Pressure was still less than 10,000 FT and the situation was not a rapid decompression, a controlled descent was performed and not at maximum IAS. The ECAM steps were completed during the descent at about FL270 and automatic cabin pressure control was regained. Screens indicated a loss of cabin pressure number 2 controller and control was taken over by the number 1 controller. Cabin pressure was noted by the crew to be stable at about 7,000 to 8,000 FT. After checking all cabin pressure systems the crew requested a level off at FL250. A conference call was placed to Dispatch and Maintenance and the decision to continue was made in light of the only item lost was Cabin Press Controller 2. It was assessed that there was a temporary loss of pressure while cabin pressure control was automatically transferred to the number 1 system. Considering the aircraft maintenance status, weather at destination and all en route alternates, and fuel, it was decided to proceed to destination but remain at FL250 as a precaution.

Synopsis

A320 flight crew experiences a cabin pressurization anomaly at FL370 and initiates a descent. With the automated controller switching to CPC 1 and the cabin pressurization remaining below 10,000 FT, the crew elects to continue to destination and land uneventfully.

ACN: 1083199 (2006 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.MSL.Single Value : 11000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : LGA4
Airspace.Class E : N90

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1083199
Human Factors : Other / Unknown

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were assigned the LGA SID with Whitestone Climb. At the gate, I asked for PDC early to review the departure but failed to catch a small font note at top of page that is listed as 3 in a list of at least [6] notes concerning the departure. The note stated "aircraft cleared over a certain list of fixes had to hold 250 KTS until 11,000." We were very light and climbed through 11,000 FT quickly and neither of us were aware of a speed deviation. However, when we were at 15,000, ATC requested we slow to 250 KTS and then the First Officer (new back in the seat after 8 years off) read the note. I had studied the departure page and missed this completely!

Synopsis

A320 Captain reports exceeding 250 KTS prior to 11,000 FT on the LGA 4 departure, as specified by note 3.

ACN: 1082018 (2007 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1082018
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Operating a through flight with an aircraft and terminal change in the hub. Dispatch release received with tankering fuel. I did not thoroughly review the release (down to the last pound of fuel/minute of flight time), so I missed that the flight plan was short both time and fuel. If you look at the last segment, it shows from the VOR to the airport, a distance of 21 NM, would take 1 (ONE!!!) minute. It actually took 16 minutes. Our burn was 1,000 pounds over release fuel. Dispatch is trying to tighten up on fuel and at the same time the dispatchers are not adjusting the poor flight plans. The result is the pilots having their necks hanging out. 

Here is the list: dispatcher training, new flight planning system, get Management off the dispatchers' backs regarding fuel loads (putting a dispatcher in for an above and beyond award because he made his restricted fuel quota for the month sends a very bad message.)

Synopsis

A320 Captain reports being shorted on fuel for the planned flight which resulted in using fuel that was being tankered. New dispatchers and new dispatch planning software were cited as factors.

ACN: 1082012 (2008 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 15000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11600
Experience.Flight Crew.Last 90 Days : 95
Experience.Flight Crew.Type : 8500
ASRS Report Number.Accession Number : 1082012
Human Factors : Fatigue
Human Factors : Workload
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1081990
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately 60 NM west if our destination at approximately 15,000 FT, we lost all data from both FMGCs. We tried unsuccessfully to restore any or all data, leaving us with no traditional electronic navigation. We had good ADI information, complete with speed, VSI and altitude information. Because of the late arrival and long duty day, coupled with the potential for complications, I elected to declare an emergency with Approach Control. We received a heading direct to the airport and flew a visual approach. We were able to use stand-by navigation for ILS information and good PAPI information. The approach and landing was otherwise uneventful. Company Dispatch, Maintenance and Flight Operations Duty Manager were notified.

Narrative: 2

Only ECAM was CABIN PRESS-set manual landing altitude. Lost all navigation ability. Lost all non-precision and non-precision ability. Declared emergency. Approach vectored us to a visual approach and normal landing. STBY NAV selected on Captain's RMP, course and frequency set. Glideslope appeared unreliable. Unable to reload any info back into FMGC using the MCDU.

Synopsis

While descending on an arrival, both A320 FMGCs failed causing lost precision and non-navigation data, so an emergency was declared followed by a visual approach. The RMP frequency and course were set, but the glideslope appeared unreliable and the only ECAM displayed was CABIN PRESS.

ACN: 1082001 (2009 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1082001
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

I was the First Officer. During the preflight inspection I discovered hydraulic fluid leaking from the left hand main landing gear area. Fluid was running down the landing gear on to the tire and puddled underneath the brake assembly area. I returned to the flight deck and found the green system reservoir quantity to be below limits. I reported the condition to Captain, made a write up, and verbally reported the log item to Maintenance. Maintenance quickly confirmed the hydraulic leak and started working the problem. A mechanic with a radio came up and turned on the yellow hydraulic system to pressurize the yellow and green hydraulic systems while mechanics inspected the wheel well area for leaks. After about 30 minutes, a Maintenance Supervisor came up and informed us that despite all the fluid found and the area found to be "saturated" with Skydrol, they had not found anything wrong or leaking. He was going to leave the system pressurized for another 20 minutes and sign of the log item as lines tightened - no leaks found. I tried to be diplomatic and did not want to over step my bounds, but finally, in the interest of safety, I - and the Captain - had to say we were not taking the airplane unless they performed a procedure that would pressure the landing gear extension/retraction portion of the hydraulic system. Due to the location and nature of the fluid leak, we highly suspected this part of the system to be the source of our problem. The procedure was to pin the gear, place the aircraft in the air mode, and pressurize the gear actuator system by placing the gear handle in the up position. From my previous maintenance experience as an airline Mechanic and Supervisor, this seemed to be a logical and obvious troubleshooting step and, personally, a requirement before I would certify the integrity of the system in question. The method they were using only pressurizes and tests a small part of the hydraulic system. Not one of the five mechanics or Maintenance Supervisor even mentioned this procedure and were prepared to sign the aircraft off and send us on our way: very troubling to me. After performing this procedure, the Mechanic reported the hydraulic system was "leaking like a sieve" and the aircraft would be going to the hangar. The flight was canceled and we were deadheaded to our next assignment.

Synopsis

An A320 was removed from service after a flight crew refused the aircraft because of a hydraulic leak which inadequate troubleshooting could not resolve until the full landing gear system was pressurized.

ACN: 1081857 (2010 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZLA

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : B777 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Airspace.Class A : ZLA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1081857

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1081868

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

In climb out of FL330 encountered significant wake turbulence from B777 6.5 miles ahead of us. Aircraft rolled about 45 degrees left then right and left at less bank during recovery. [We] notified ATC, placed seat belt sign on and seated flight attendants during recovery. No injury or aircraft problem noted at time. After flight one flight attendant notified us she banged her shoulder while taking seat and may report if still bothering her tomorrow for medical advice.

Narrative: 2

Notified LAX Center immediately and was told we had legal separation. It was fast and violent, Captain as pilot flying did a superb job. We briefed the passengers what happened and answered all questions as they deplaned.

Synopsis

A320 flight crew reported wake vortex encounter in trail of a B777 at FL330 in the LAX area that resulted in sharp roll and a minor flight attendant injury.

ACN: 1081794 (2011 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1081794
Human Factors : Workload
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1083410

Person : 3

Reference : 3
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1082661

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Primary Problem : Human Factors

Narrative: 1

I was training on R6 position first and we were told to combine with R4 to continue training. Traffic was getting slow. So after awhile it started picking up, trainee started to fall behind and it went up to a point that I had to take him out. In the middle of this busy session, it was brought to my attention that Air Carrier X needed lower. From FL360 to FL340. I made several calls with no answer. I kept on working. I tried again on a different occasion. Nothing. Kept working. I think I tried him again, but can't recall. It was so busy that I believe there were more than 30 aircraft on different frequencies stepping on each other. It happens that the Air Carrier X was on a frequency other than mine. ZZZ1 Center was advised of the situation and it was coordinated that he was going [to] be turned on a 360 degree turn back to ZZZZZ, and it was going [to] be descended on RADAR to FL340 to comply with what was coordinated. The aircraft descended to FL340 and switched. By this time I had a CIC, an FLM and the trainee all watching over me to help. Proper training for CIC duties needs to be established at ZZZ Center. CIC could have done a better job if properly trained. OMIC and FLM's, if any are on the operations room, need to be more engaged in the operation. ZZZ Center is under staffed, flow has to be implemented every day, specially on weekends. It does not look like anybody is taking the time to do this. If D-Side requests an Aside and it can't be provided, CIC should help until someone can be assigned. Sectors are being worked combined when they should be split mostly because of staffing and it has become the norm every day. Even when there is severe weather or heavy traffic. Staff should be scheduled to cover for this, especially on weekends. On the same note, FPL's are being used almost 24/7 for CIC duties. FLM's should be called in on overtime to cover so FPL's could be used on the floor to work traffic and train.

Narrative: 2

I was working sectors combined with no Coordinator position. My area had a CIC, and there was an FLM on the floor that had just arrived for his shift. We had major weather deviations and bad frequencies that required switching back and forth between different receivers. I ask both the FLM and CIC to assist the adjacent sector. The sector adjacent to me was becoming heavily saturated with traffic and had minimal weather deviation; however a new RADAR trainee was receiving OJT. There were airplanes in my sector from the adjacent sector not on my frequency and vice versa. The adjacent sector began to get busy before me, and I heard the OJTI take his trainee out of position, an obvious sign that there was too much to handle. Before this happened, I had already recognized that the trainee and OJTI was in over his head and asked for an FLM to assist. Moments later it was getting worse, and I started to become saturated with major weather deviations. The FLM was not assisting nor was the CIC. The FLM sat down on the empty scope next to me to watch both airshows, and I told him again we need help. He said, "I'm watching." No coordination, no splitting sectors. For the record, it was too late to split sectors. I have guys calling and stepping on each other, because with the sectors combined I was operating 9 frequencies. In the midst of this, I had Air Carrier X call me and ask if he was still supposed to be on my frequency. I did not know where he was so I ask. He replied, "ZZZZZ". He had been out of my sector for about 30 minutes and just crossed the boundary with ZZZ1 Center. I ask the other sector to see if he had been coordinated at FL340 and he was at FL360. I was told by the FLM that had finally walked over to assist, to tell him to make a right hand 360 degree turn back to ZZZZZ and descend to FL340 and I complied. I then terminated Air Carrier X. Turns out that there was non-RADAR traffic in the area at FL350. I do not know if separation was lost. Refresher training for all CIC's. We have not had it in years. FLM's need to be more involved in the operation and work more proactively, always ready to assist the sector.

Narrative: 3

I'm an FLM and needed to perform a skill check on a developmental. When I reported to the control room, I coordinated with the CIC in order to provide the skill check. While I waited for the trainee, I noticed that R4 was very busy and there was training. The trainee was not keeping up and the OJTI was making most of the transmissions. Although I was not in charge, I decided to remove the trainee from the position and allow the OJTI to take over the sector completely. I told the CIC that R4 needed a tracker to help. I did provided assistance to the R-Side for a few minutes until the tracker arrived. My developmental was ready and we assumed the position. About a minute into it, R4 noticed that Air Carrier X had not descended from FL360 to FL340 as "cleared." At that same time R2 was yelling that Air Carrier X was on their frequency. At that moment I saw how close Air Carrier X was from the boundary and told R2 to spin him because he was coordinated at FL340 and there was not enough separation with traffic in front at FL360. My immediate concern was to not let the aircraft depart the airspace at FL360. I called ZZZ1Center and advised them about the situation. I told them I would keep the traffic, spin it and descend it on RADAR inside my airspace and that I would re-coordinate once at FL340. I told ZZZ1 Center that the aircraft was going to enter their airspace. I told R2 Controller that once the aircraft was back on our airspace he could descend it on RADAR to FL340. We re-coordinated the traffic at FL340 and realized that there was another aircraft already coordinated with ZZZ1 Center at FL340. This aircraft was not a factor with me but it was going to be a factor with ZZZ1 Center and we negotiated an altitude with them. Later Air Carrier X was terminated. Later, I was presented with RADAR data which showed Air Carrier X descended non-RADAR with another aircraft inbound to ZZZ Center from ZZZ1 Center over ZZZZZ. I was shocked to see this. I do not remember at any time having an aircraft on the bay at FL350. I remember looking at the bay for any aircraft at FL350, and I don't remember seeing one. I think, although not sure, I asked the developmental if there was anyone at FL350 as part of the training. I do not know if the strip was accidentally pull down from the bay or at that moment the RADAR Controller and/or the tracker blocked the view while marking the strips. ZZZ1 Center did not mention anything. I did not see any ARINC messages claiming an RA or a close call. Neither the R4 Controller, R2 Controller or tracker mentioned anything to me or any of the FLMs on duty that day. I talked to the developmental today to see if he remembered an aircraft at FL350 at that moment. He stated to me that he did not remember anyone at FL350. He said that he looked at the scope at one moment and saw Air Carrier X leveled at FL340 and no one at FL350. There were many factors with this situation. The R2 Controller did not ensure that Air Carrier X received the frequency change and R4 Controller did not ensure the read back from the clearance to descend to FL340 (apparently he tried a few times). In addition it appears that the R2 Controller issued a clearance to the Air Carrier X inside ZZZ1's airspace, not as I advised him "in our airspace descend it to FL340," but I take full responsibility for the loss in separation because ultimately it was a non-RADAR issue and I was a non-RADAR Controller. If I missed the traffic at FL350 I should have seen it. 

This is a difficult situation to evaluate. This all happened immediately after I assumed responsibility for the sector. The main problem that precipitated the main event was a failure from the RADAR Controllers to ensure a frequency change and subsequent hear back/read back when the altitude clearance was issued. I should have advised R2 to turn the aircraft only and switch it to our sector for subsequent clearances. It is obvious that if the strip was posted I should have seen the traffic and issue appropriate actions to the RADAR Controller. In a future occasion I will correlate all traffic that is departing the airspace with RADAR data to ensure any discrepancies are resolved before I assume responsibilities.

Synopsis

Enroute Controller described a very busy and confused traffic event that apparently resulted in a loss of separation event, three varied viewpoints were provided for this occurrence.

ACN: 1081781 (2012 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ABQ.Airport
State Reference : NM
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ABQ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : GRIZZ1 RNAV
Airspace.Class B : ABQ

Person

Reference : 1
Location Of Person.Facility : ABQ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1081781
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Air Carrier X was departing Runway 8 with the route ABQ2...FTI. Assuming that the air carrier would be on the GRIZZ1 RNAV departure, I cleared Air Carrier X for take off without giving a vector. Air Carrier X departed runway heading, towards rapidly increasing MVAs. I think the "RNAV to (point)" phraseology with the take off clearance would be a good idea for all airports, not just ones with parallel runways. I would suspect that a pilot would question a clearance like "RNAV to TYILR" if they're expecting a RADAR vector, providing a last minute sanity check. The first fix that shows in the aircraft's tag is different depending on whether the aircraft is on a RNAV or non-RNAV departure. Double checking the field before switching the aircraft to departure would be another last-second chance to fix the problem. Changing local procedures to highlight non-RNAV air carrier or air taxi departures might also help, but it's not necessarily Ground Control's or Clearance Delivery's responsibility check every strip for Local Control.

Synopsis

ABQ Controller described a confused departure route event when unsure if the departing aircraft was issued a RNAV or conventional SID. The reporter suggested a number of procedural changes to reduce future occurrences.

ACN: 1081094 (2013 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1081094
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As we taxied into position on the runway, two following flights reported that significant flames and smoke were coming from our APU compartment. We verified the APU had shut down, exited the runway, and requested CFR support. 

The QRH provided no guidance given the lack of ECAM indications, and I elected to deploy the APU fire extinguishing system as a precaution. Once we were able to communicate with the CFR crews on a discreet emergency frequency, we determined that the event was over. 

We returned to the gate and debriefed with the CFR and cabin crews.

Synopsis

The flight crew of an A320 was alerted by other aircraft of significant smoke and flames emanating from their APU while taking the departure runway. The Captain activated the APU Fire Extinguishing system and contacted the CFR personnel, determined the event was over, and returned the aircraft to the gate followed by a crew briefing.

ACN: 1081025 (2014 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Flight Data / Clearance Delivery
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1081025
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Air Carrier X's original clearance was removed from the FDIO system because the pilot's company was planning to file another clearance for the aircraft. Air Carrier X's company then filed a new clearance after the old clearance was removed and disposed. Air Carrier X then accessed the old clearance via PDC a significant amount of time after it was originally removed. The pilot then taxied out, believing this out-dated PDC'd clearance was his appropriate clearance. Ground Control asked Air Carrier X if he had his clearance because the flight strip was not located at Ground Control. Air Carrier X apparently responded that he had his clearance. The problem was he wouldn't have had any new changes from the new flight plan his company filed, but Ground, Clearance Delivery, and the pilot were all blissfully unaware because apparently the company had not told the pilot that they were going to file a new clearance. Ground asked Clearance Delivery (myself) if I had a strip for Air Carrier X. I did and delivered the strip to ground. The strip was unmarked at flight data, but it is fairly common for a PDC's strip to be unmarked at clearance delivery because of the volume of traffic ZZZ handles and the amount of strip marking the Tower requires. While issuing PDCs, or verbally issuing clearances to large groups of aircraft, one may go unnoticed and fall into the unissued column at Clearance Delivery. Ground will then have to ask Clearance for the strip and it will be delivered at that time. The clearance became available while I was on break and could have been issued at that time so I gave the strip to Ground. Little did I know at the time, the route after the PDR on the strip could have been different from what the pilot had as a result of the pilot pulling up the old clearance via PDC. There was no full route required for the new clearance per what was on the flight strip and the pilot was not informed by the company of a new clearance. Recommendation, there are several correctable issues presented by this event. Primarily, old clearances should not continue to be available via PDC after the clearance has been removed from the system. This is a simple, obvious thing in my opinion and I can not believe that it has not yet been implemented. Second, the company should have a better checklist when changing flight plans, it's absurd that a pilot can be given multiple flight plans for the same route of flight or destination and it always creates more work and a safety hazard for everyone involved, even when clearance delivery and the pilot are aware of the situation. As far as I know, the outcome of this situation was relatively harmless, but it could definitely pose a serious safety risk and even in this situation the pilot could have flown a different routing from what was filed after I had left.

Synopsis

Tower Controller described a very confused clearance event pointing out the potential for errors when companies change a requested route but do not notify the flight crew.

ACN: 1080052 (2015 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 2700

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : LAS

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Facility : LAS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1080052
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

We were using the backup RADAR, as the primary was released to maintenance. An A320 was on final for Runway 19R, and a B737 was LUAW on Runway 25R. I cleared the B737 for take off when the A320 was 3 miles out, which is a normal operation. The A320 was at 190 KTS indicated on the display, and appeared to be OK for landing. The B737 started rolling, and then I noticed that the A320 disappeared from the RADAR, both primary and beacon slash. The next update had a beacon slash a mile and a half closer than the previous hit, only 10 seconds earlier. I looked at the B737, who was nose wheel up, and then at the A320, who announced that he was going around. I issued traffic to the A320, who did not see him, then turned the A320 30 degrees left to avoid B737. I then went to the B737 to issue traffic, and he reported him in sight, while saying "That was nice". I was not looking at the arrival when he actually initiated the go around, so am not sure at what exact point he started. I am wondering, however, if the back up RADAR is not as accurate, and the three mile line that I used to judge a safe distance was not good enough. Maybe the A320 was closed than the RADAR indicated, and that is why, on the go around, the A320 ended up much closer than I would have liked to the Runway 25 departure. Give refresher training on the use of the backup RADARs. This is probably the first time I have used it in over a decade, and did not realize that the normal spacing used for judgments may not be sufficient, due to a less accurate display.

Synopsis

LAS Controller described a go around event while utilizing a back up RADAR system, claiming that the lack of familiarity and training deficiencies were both casual factors.

ACN: 1079581 (2016 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1079581
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Procedure
Primary Problem : Equipment / Tooling

Narrative: 1

During pre-planning phase of release Dispatch system recommended NO ALTERNATE for flight. This recommendation was in spite of the weather forecast of winds gusting to 33 KTS. The runway configuration Runway XX L/R and YY puts a 29 KT crosswind for Runway XX. [The arrival airport] is in ground delay program for this reason and has a high potential of staying in a ground delay program for a while. Dispatcher caught weather and added destination of ZZZ1. This crosswind configuration also was the limit of the company prescribed operational limitation for this fleet of aircraft (29G35 KTS) A319/320/321 PH 1.3.1 operational limits. Cause, Dispatch system does not factor in considerations for alternate and does not assist in computing fuel as required by FAR 121.647. It does not factor in the operational limits of company fleets and the overall weather picture. System provides multiple messages in a condensed time period to provide distractions to the Dispatcher's primary duties of operational control and safe monitoring of company flights. System provides false information and has a high potential of causing at minimum departure delays for adding alternates and fuel if caught by flight crew and Dispatcher. System provides high potential of causing illegal flights if not caught and could be a major contributing factor of illegal or delayed flights. This automation is distracting and goes against the Safety Management System (SMS) and Threat Error Management (TEM) models of the SMS and provides additional safety barriers at a time when the company is trying to reduce them. Suggestions, remove this system: This is a hindrance to the professionalism, experience and judgment of the Dispatch group. This system also provides many barriers to safe operation including multiple and conflicting messages over a condensed time period. This system is also activated at a time when the company is closing dispatch desks providing already increased workload. System also provides multiple NOT RECOMMENDED messages AFTER Dispatch release is calculated increasing message traffic and possibility of errors/conflicting information. Dispatch system uses inaccurate and potentially illegal information to base the recommendation.

Synopsis

Dispatcher voiced concern regarding the functionalities of the current decision making tool providing a number of examples of the limits encountered.

ACN: 1079554 (2017 of 2908)

Time / Day

Date : 201304

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : TELLR ONE
Airspace.Class A : ZDV

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1079554
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

Our routing into DEN was filed on the PEEKK RNAV STAR via OURAY transition. Upon switching to ZDV (DEN Center), we were cleared onto the LDORA1 arrival. With some low level convective activity ahead on the arrival, we were expecting to ask for some slight deviations for weather. 

We switched to the next ZDV sector, and we were given a left turn heading 340 degrees for en route spacing. Several minutes later we were cleared to intercept the TELLR1 arrival. Shortly thereafter ZDV tells us: "...cleared to deviate up to 15 degrees either side of course, and descend via the TELLR1 arrival." By this time we were both heads down for most of the descent reprogramming and verifying all the new route changes, as well as the new constraints. 

ZDV handed us off to DEN TRACON at FL250, and we were then issued to proceed direct POWDR direct DVV, and to descend and maintain 17,000. We were then cleared to deviate at our discretion for weather avoidance to DVV. Normal radar vectors and altitudes from the mid-teens to landing were then provided by TRACON. 

Also, we twice experienced airspeed anomalies during the managed descent. The FMGC and charted speeds were not being followed correctly by the airspeed tape. It was commanding 210 KTS instead of the required 250 and 280 KT speeds. We as the pilots had to manually select the correct speeds, and also use full speed brakes to meet required altitudes. This required the jet to be hand flown several times on the RNAV STAR just to be at the correct altitude and airspeed at several fixes. THIS IS NOT WORKING!

Synopsis

An A320 First Officer reported specific and generalized problems with respect to conducting RNAV STAR operations into DEN. His summation states: "THIS IS NOT WORKING!"

ACN: 1079477 (2018 of 2908)

Time / Day

Date : 201304
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 18750
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 5600
ASRS Report Number.Accession Number : 1079477

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
Miss Distance.Horizontal : 6000
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

[Received a TCAS RA at 5,000 feet to climb while receiving vectors from ZLA.]

Synopsis

A320 Captain experiences a TCAS RA at 5,000 feet approaching SNA.

ACN: 1078600 (2019 of 2908)

Time / Day

Date : 201304

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Component

Aircraft Component : Central Warning/Master Caution
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1685
ASRS Report Number.Accession Number : 1078600
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approaching cruise altitude, I noticed an intermittent flashing near the flap position indicator on the E/WD. It flashed in amber Flap - F above and F-Locked below the symbol. It then alternated to flash Flap - S and S-Locked. There was no associated ECAM to confirm a failure or fault of the system. No C/B's had popped. I checked the F/CTL system display page. All was normal. I disconnected the autopilot to confirm that there were no flight control abnormalities. There were none. The flap position indicator showed everything stowed, and I confirmed that. The flashing persisted. We then got an ECAM with aural chime for COND AFT CRG ISOL VALVE. It, too, was intermittent. The system display page showed both valves continuously closing and opening. We were unable to clear the intermittent ECAM normally or silence the chime using the emergency cancel button. The two problems, although completely separate systems, were both randomly flashing with the one ECAM chiming. This made us wonder if there was an underlying related electrical issue even though all other system pages showed normal operation. I had earlier sent a message briefly describing the problem to Maintenance Control through ACARS, but eventually I determined the need to speak directly. I requested a phone patch to Maintenance Control. I had been PF, so I gave control of the aircraft and ATC radios to the First Officer. 

I talked through the problems we were experiencing so far with Maintenance Control. He took several moments to reference the systems. There was a discussion to determine whether the flaps/slats had locked up, but without a corresponding ECAM failure or fault and nothing highlighted as a fault on his end. It was agreed that it was more likely an indication problem. For now, the COND AFT CRG ISOL VALVE was considered to be a separate issue with the only action of crew awareness. We referenced the flight control system flap/slat chapter of the Flight Manual, but there was no information regarding this scenario of flashing lights. Maintenance Control guided me to cycle flap/slat control CB's to clear the flashing indication. I was somewhat distracted by the continuous chime and flashing, but at the time, this seemed a reasonable course of action as I had confirmed the flaps were up. I proceeded to cycle the C/Bs using Maintenance Control's instructions and my Captain's Emergency Authority in attempt to silence the warnings as I felt a sense of urgency with the two issues occurring simultaneously. It did not correct the flashing. Maintenance Control continued to reference the Flight Manual. Meanwhile, I became even more concerned that there could be some connection between the two issues and some larger underlying problem. The first officer agreed. She requested a lower altitude with ATC while I considered diversion airports to give us options in light of the situation and in the event the situation deteriorated. During this time, we had to transfer us to another frequency due to a medical emergency for another aircraft. After several minutes, I reconnected with Dispatch/Maintenance Control. 

Maintenance Control came back on the radio suggesting that it might be one of the Flight Warning Computers. While this discussion was going on, other systems then started to intermittently flash and chime. The ones that I remember included: nose wheel steering disconnect message, a BRAKES HOT, and an AIR PACK 1 OVHT ECAM. There was no supporting information that corroborated these ECAMS or messages on the system pages. They were intermittent except for the AIR PACK 1 OVHT. It stayed on, so I completed the ECAM and shut off Pack 1 per procedure. I relayed this new information to Maintenance Control. The suspicion that we had issues with the FWC's seemed more plausible. My sense of urgency to understand the problem before deciding with the F/O to divert due to electrical issues continued, but by now we were well into the arrival, making our destination the most suitable airport. I wanted a long runway to land with zero flaps/slats if that scenario were to present itself. We continued to work to understand the problem. Maintenance Control gave me reference to the FWS FWC 1(2) FAULT procedure. Per this procedure and Maintenance Control guidance, I cycled the FWC C/Bs separately. This resolved the additional random issues, but the original flashing of the flaps/slats and the AFT CARGO intermittent ECAM remained. It was at this point I returned our attention to the flap/slat issue. Even though we felt it an indication problem, we discussed the possibility of a no flap/slat landing. I had acknowledged and discussed this possibility with Maintenance Control. The First Officer pulled out the expected non-normal procedure, F/CTL FLAPS FAULT/LOCKED, to review in case that ECAM should occur. There was nothing left to address further with Maintenance Control, so we both signed off. 

I continued the conversation with Dispatch. Dispatch was able, with our ATC coordinator, to forward our request for the long runway. If we had issues with flaps/slats on approach, I would elect to declare an emergency then and request appropriate assistance. Upon switching to Approach Control, I requested to slow early to extend flaps/slats. Flaps/Slats extended normally, and we landed without incident. After the flight I further reviewed the Flight Manual and found that the flap/slat control CBs were not to be cycled in flight. I was unaware of that restriction at the time Maintenance Control advised me to cycle them. 

There was never a defined ECAM or unannunciated procedure to follow. There was a sense of urgency to silence the warnings yet mindful consideration to understand the cause of the random indications. In hindsight, it was an unintentional failure on my part to not question the guidance/direction to cycle flap/slat control C/Bs. A flag went up in my mind at the time, but it was not immediately apparent to me that these were of prohibited status in regards to cycling while in flight. At the time, I relied on and trusted my resources with that information to make a judgment call when I exercised my Captain's Emergency Authority to cycle those C/Bs. Maybe those C/Bs should be highlighted in some way on the CB panel and/or within Maintenance Control procedures to alert us to their prohibited status in flight. I talked with Maintenance Control directly on the phone after the flight to explore further his thought process for this guidance. He mentioned that he thought we were on the ground when he suggested we cycle those C/Bs. This concerned me as I had assumed he knew that we were airborne based upon my initial contact and phone patch through Dispatch and lengthy discussions. It's clear there was a misunderstanding and communication error here.

Synopsis

An A320 had an intermittent Flap/Slat Locked indication in cruise, followed shortly by an intermittent COND AFT CRG ISOL VALVE warning. Maintenance Control suggested cycling the flap/slat control circuit breakers, to no avail. New ECAM's appeared, including nose wheel steering disconnect message, a BRAKES HOT, and an AIR PACK 1 OVHT. The AIR PACK 1 OVHT ECAM was complied with and the others were cleared by cycling the FWC (Flight Warning Computer) circuit breakers. A landing at destination with normal flap operation was made.

ACN: 1078360 (2020 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : BOS

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : BOS
Make Model Name : SR22
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Mission : Personal
Flight Phase : Cruise
Route In Use : None
Airspace.Class B : BOS

Person

Reference : 1
Location Of Person.Facility : BOS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1078360
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

While working Local Control West (LCW) a SR22, was conducting a city tour with Local Control East (LCE). LCE advised me that the SR22 had the Airbus in sight. Due to his proximity, no traffic call had to be made while clearing the Airbus for takeoff. Once the airbus was at the departure end of Runway 27, I realized the SR22 had moved closer to the airport, and issued a traffic advisory to the Airbus and informed the pilots the SR22 had them in sight and would maintain visual separation from them. No separation was lost. Maybe have more restrictions on the aircraft that are conducting city tour when departing Runways 27 or 33L.

Synopsis

Tower Controller reports a possible conflict between an A320 departing Runway 27 at BOS and an SR22 flying over the downtown area.

ACN: 1077881 (2021 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Reverser Position Indication
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11135
Experience.Flight Crew.Last 90 Days : 145
Experience.Flight Crew.Type : 5142
ASRS Report Number.Accession Number : 1077881

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While at cruise flight at FL380 we received ENG 2 REVERSER FAULT ECAM. I was the pilot flying with autopilot 2 engaged. The Captain instructed me to continue flying and to take the radio while he handled the ECAM. The Captain briefed me on the ECAM and we agreed to contact Dispatch via ARINC patch. A decision to divert to our nearest and most suitable airport was made since the ECAM directs to land ASAP, which I supported after considering weather, mechanical clues (no secondary indications), facilities, terrain and overall passenger and crew safety. 

We coordinated with Dispatch and ATC about the diversion. The Captain reviewed the diversion guide, briefed the flight attendants and passengers, while I followed ATC instructions to proceed. The Captain elected not to declare an emergency after he considered all factors in front of us aided by the inputs that he received from Maintenance, Dispatch, and our threat and systems analysis. We briefed the visual approach and landed without incident.

Synopsis

An A320 flight crew received a reverser fault indication at FL380, they did not declare an emergency but decided to divert to the nearest suitable airport.

ACN: 1077770 (2022 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Landing

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1077770
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We experienced the toxic fumes (associated with dirty socks or laundry) after the gear came down on approach. The odor permeated the cabin from the exit rows to the rear of the aircraft. "B" Flight Attendant immediately came forward to notify me and the "C" to cover our noses and mouths. Except for having to speak and breathe for the arrival announcement, I kept my nose and mouth covered. Passengers toward the rear, Rows 23-25, were complaining of a foul musty smell. Passengers deplaned without incident. During our short turn, the Captain had another airline's Mechanic inspect the aircraft. My complaint after the incident entailed a bad taste in my mouth (metallic) that originally felt like I had hair on my tongue. Neither liquids nor gum would take it away. I also developed a dull headache. These symptoms stayed with me until I was almost back in my domicile. The other Flight Attendant and I stayed out on the stairs - in the fresh air - during our short turn. After the aircraft was checked out, we did fly the flight back to our domicile, no more incidents were reported upon landing. 

A Mechanic checked the aircraft out after we landed. He discussed with the Captain what could have caused the fumes/odor to permeate the cabin exit rows to the rear of the aircraft after the gear came down, on approach for landing. 

This is not the first time I've been involved in a toxic fume incident but it is the first time I have felt the effects. From what I could understand from listening to the Captain and the Mechanic, it is still an oil problem that leaks into the system that bleeds into the cabin air. The problem is still occurring.

Synopsis

An A320 Flight Attendant reported a dirty socks odor when the landing gear was extended after which she developed a headache, and a metallic taste in her mouth.

ACN: 1077396 (2023 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 1077396

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1077420

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were flying an airplane that had just been serviced on the ground with 8 quarts of oil. It was serviced to 22 quarts of oil on the ground due to low oil quantity. En route oil quantity steadily decreased to 10 quarts. We were in contact with Maintenance Control and Dispatch, and ATC and decided to divert. We declared an emergency and landed in [the divert airport] without further incident.

Narrative: 2

Number 1 Engine oil quantity steadily decreased from 14 quarts to 10 quarts at FL350. After coordination with Maintenance Control, Dispatch, and ATC, we diverted.

Synopsis

The pilots of a A320 observed the Number 1 Engine oil quantity decreasing so they elected to divert where a successful landing was performed.

ACN: 1076831 (2024 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34700

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7300
ASRS Report Number.Accession Number : 1076831

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 1076834

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight suffered a Number 2 Engine compressor stall and a subsequent engine failure. The seat belt sign was on. The flight was descending to FL340 due to chop at FL350. We had initially started a climb to FL370 but had reversed and started a descent after ATC informed us that the next sector had mod chop at that altitude. In the descent, in smooth air, open descent, Mach .78, managed speed. We heard a thump. Not the loud banging we had in training, just enough to wonder "what was that". Our attention went to the engine instruments and everything was normal, then the Purser called to say he had heard a loud bang from the right side of the aircraft. I told him we were aware of the odd noise and I would get back to him. 

We scanned the engine instruments and saw a momentary flash of amber stall on the upper screen, but it lasted only a second or so. Then the Engine 2 oil filter clog amber annunciation came on. I immediately grabbed the Flight Manual to start looking for the correct procedure. The Purser called us again to say he heard another loud bang. I told the First Officer (pilot not flying) to fly and handle the communications and I would work the problem. We requested a lower altitude from ATC, and ATC asked us if we could stay at FL340, for another minute. We said yes. I continued to try and find the Oil Clog procedure when the pilot flying said the Number 2 Engine had failed; I declared an emergency and requested lower FL250, from the engine out data on the FMC. 

That was issued by ATC and I began to complete the engine failure ECAM that had just popped up on the screen. I sent Dispatch a message to CALL ME and continued the running and completing the ECAM. Dispatch responded with a frequency, which I put into the VHF #2 and passed along our current condition, and divert information to [nearest suitable airport]. We had continued our descent to 12,000 FT and subsequently got a descent to 6,000 FT navigating with ATC vectors to the airfield. I called the Purser to inform him of the time remaining, destination, weather and landing conditions.

Fortunately all the back up systems were working perfectly and the weather was better than CAT 1 minimums, with the runway clear and dry. I decided not to prep as the aircraft was flying well with all systems backed up. It was about 15 minutes before touchdown. We completed all the checklists and continued down to 4,000 FT at 15 NM south of the airfield. Immediately after checklist completion we notified ATC and they gave us a vector to the localizer for the ILS to the field. Approach and landing were normal; we rolled out straight ahead and stopped on the runway to allow the emergency vehicles and crew to checkout our engine. 

Everything was reported to look normal from the chief of the firefighters so we taxied clear of the runway and to the gate. The flight taxied to the gate and shut down normally. Once at the gate my phone was ringing constantly, even before we were out of the seats. I forgot to place a write-up into the ACARS, partly due to the phone calls from Operations and others who I relayed our flight problems to. I subsequently called [Maintenance] later that night to check if I had indeed forgot to enter it. The technician I talked to said yes but that all the information was inputted from transmitted data and some others I had talked to.

Narrative: 2

[Narrative 2 had no additional information.]

Synopsis

A320 flight crew reported compressor stall and subsequent failure of #2 engine at FL340. Flight declared an emergency and diverted to nearest suitable airport.

ACN: 1076234 (2025 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1076234
Human Factors : Troubleshooting
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

I [performed] the noise abatement Flaps 3 Improved takeoff and upon climbout the aircraft failed to pressurize. We ran the ECAM and flight manual procedures for: Pressurization Failure SYS 1, 2; overweight landing non-normal and diverted to a nearby Maintenance Station using Captains Authority to autoland at 148 knots. ATC, Dispatch and Station Operations were all advised; flight attendants and passengers were informed.

Synopsis

After departure an A320 ECAM alerted CABIN PR SYS 1 + 2 FAULT so the flight diverted to a nearby airport for maintenance.

ACN: 1075953 (2026 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 5050

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Route In Use : Visual Approach
Route In Use : Vectors
Airspace.Class B : LAS

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class D : HND

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1075953
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Analyst Callback : Attempted

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The First Officer was flying. We were turned right, off the GRNPA arrival, to a heading of around 280 and cleared the visual approach to Runway 01R in LAS (behind traffic). The vector took us directly over the top of Henderson airport. When descending through 5,100 the Controller said "do not descend below the Class B (floor is 5,000 there)."

Right after that we got a monitor vertical speed TCAS RA for an aircraft right below us over the Henderson airport. Within five seconds the Controller advised we were below her minimum vectoring altitude and "correct your altitude immediately" or some words close to that. I may have the order of events wrong. All three events happened within about 20 seconds. 

We leveled off at 5,050 FT until turning inbound which put us a little high over GALNE the FAF for the RNAV 01R approach. Altitude over GALNE is depicted as 4,000. We were able to make a stabilized approach with flaps full. The fix outside GALNE is KIBSE and its altitude is 5,100 FT. Altitude wise we were right where we needed to be. Crews are not given Minimum Vector Altitude charts. Only the Controller has those. At no time was there any yellow or red showing on my terrain mapping which was on at the smallest possible scale. Nor was there ever a terrain call out from the GPWS. 

This night visual approach from the southeast is a bad idea when vectored over the Henderson airport; too much chance of an RA. No idea why the Controller felt we were too low for the terrain. I looked the next day on departure to the south and it looks flat to the west of that airport for at least a few miles. It is true that any terrain and obstacles are mostly unlighted but they are primarily southwest of Henderson when on a base leg. The terrain is irregular however.

I think we need a charted RNAV visual approach built similar to the one built for Runways 19R/L. It should have suggested altitudes and speeds and configurations for each point (see the RNAV visual for 19R/L). In this way we remain clear of all terrain and do not conflict with their MVA chart. The charted base leg should be south of the Henderson airport traffic area. The aircraft will be on glideslope and on speed and everyone knows what to expect. I will no longer accept visuals approaches at night to those runways until a charted procedure is published. I will only accept the ILS 01L or the RNAV 01R. It is way too stressful to have 2 Controller warnings and a RA on a visual approach. Lets get an RNAV visual built to Runways 01L/R.

Synopsis

An A320 crew on a visual approach to Runway 1R at LAS was instructed not to descend below the floor of the Class B, encountered a TCAS RA for traffic at HND, and then were advised that they were below the minimum vectoring altitude and told to correct their altitude immediately. The reporter suggests night visuals from the southeast over HND are a bad idea.

ACN: 1075949 (2027 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : RAVNN THREE
Airspace.Class B : BWI

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Check Pilot
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1075949
Human Factors : Fatigue
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was giving the first leg of Operating Experience (OE) to a new hire First Officer. This was the 6th straight day of work, which had included early morning wake-ups and the first two legs had been cancelled from our trip, however we had already checked in early because the First Officer was a new hire and needed an introductory briefing. 

Our aircraft had an MEL for an inoperative IDG, which required repetitive maintenance action. It was a short leg, so I flew so the new First Officer would have time to settle in. At first ATC check in, Approach cleared us to cross SABBI at 15,000 on the RAVNN 3 arrival. Shortly after crossing SABBI, we were given a 070 heading with descent to 8,000. After leaving the arrival for a period of time, we were given direct to RAVNN and cross RAVNN at 6,000. Because RAVNN was noted "for Runway 33, expect radar vectors", it was my assumption that we were simply going direct to a fix with a crossing restriction (all intervening fixes were deleted) and no longer on the STAR. 

The ATIS stated that Runway 33L visual approaches were in use, and I cleared the additional legs beyond that point from the FMS so it would be easier for the First Officer to clean up the FMS when we were on vectors. Approaching RAVNN, we were given a frequency change and the new Controller cleared us to 5,000. Remembering that Maintenance had not acted to comply with the MEL to check APU oil on each departure, I asked the First Officer to make sure that Operations had notified Contract Maintenance to accomplish the MEL follow-up action. Shortly after crossing RAVNN and now on a 050 heading, the Controller asked why we did not proceed to NAVEY (the fix beyond RAVNN's "Runway 33 expect radar vectors" notation). I simply replied that we had been expecting radar vectors as noted. The Controller then cleared us direct to the FAF and subsequently for a visual approach.

Although the assumption that we would be given radar vectors (as noted on the approach chart) was the primary cause, contributing was my personal fatigue at the end of a 6 day stretch of early mornings, as well as the task loading associated with OE for a highly inexperienced First Officer and the additional task-loading while making sure that our MEL follow-up action was going to be accomplished.

Synopsis

A320 Check Airman conducting IOE, neglected to proceed to the next approach fix on an arrival procedure.

ACN: 1075512 (2028 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Normal Brake System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic Lines, Connectors, Fittings
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1075512
Human Factors : Situational Awareness
Human Factors : Other / Unknown
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Following Troubleshooting Manual (TSM) and Aircraft Maintenance Manual (AMM) procedures to determine the cause for brake overheat, I determined that possibly the Anti-Skid System was not managing the brake, which was causing the overheat and not just a temperature reporting error. Maintenance Control agreed with my explanation, allowing the deactivation MEL and subsequent procedure. After retrieving the exact AMM reference for deactivation, I entered it into the maintenance computer, which displayed several subtasks. One of these was a specific procedure for deactivating only the Green [Hydraulic System] NORMAL side. I chose this based on the previous TSM Task for complying with the NORMAL Brake System Tachometer Functional Test. The TSM was referencing only the Green-Normal System with a possible fault. I mistakenly misidentified Green from Yellow [hydraulics], causing only the Yellow hydraulic line to be stowed for the MEL. Had I removed the correct one [hydraulic line], the brake would not have activated, even though one side was disconnected and stowed. 

Since the brake was still active, the high temperature condition recurred upon landing. Since I [had been] working on the NORMAL Braking System, I [had] decided to only deactivate that side, but confused Yellow from Green hydraulic hose couplings. [After landing], the Green hydraulic line was disconnected and stowed as part of the complete MEL procedure. Please update the MEL reference for the deactivation procedure to read the EXACT subtask [in the maintenance computer] that will link directly to deactivating the entire brake. Currently when this task is entered, several other DEACTIVATION choices appear. Flight delay.

Synopsis

A Line Aircraft Maintenance Technician (AMT) reports he mistakenly mis-identified a Yellow System hydraulic line hose coupling for a Green NORMAL Hydraulic System hose coupling, causing only the Yellow hydraulic line to be stowed for an MEL deferral of an A320 Brake Temperature Indication. High temperature condition recurred due to brake was still active on landing.

ACN: 1075103 (2029 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Weather Elements / Visibility : Turbulence

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Make Model Name : PC-12
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1075103
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While working [two sectors] combined. An A320 asked for a lower altitude because of choppy ride conditions. The A320 was at FL310 which is the lower limit of my stratum in that area. I put an interim altitude of FL310 in the data block, and entered an assigned altitude of FL290 and initiated the hand off to the sector below me. The hand off was accepted and the interim altitude was taken out by the lower controller and I gave the A320 the descent. Shortly after the A320 started their descent and were passing through FL304, the pilot of the A320 advised that "he had traffic at FL290 right in front of him at his 11 o'clock. I didn't see a limited data block because FL290 was below my altitude limits so I tried to quick look [the lower sector] twice, not realizing they were combined with [another sector]. Just as the A320 showed FL302, The CA started to flash and I saw a PC12 level at FL290 right below the A320. I immediately advised the A320 to maintain FL300 and called [the lower sector] but they were in the middle of a transmission, I then received a call from what I think was the D-Side at [the lower sector] and I advised that I had stopped the A320 at FL300. I never saw the A320 go below FL300 and I do not believe there was a loss of separation, however, if the A320 had been descending faster and/or had not been paying attention to the traffic either through his TCAS or visually this would have been a loss of separation with a very low MOC. The RADAR targets were very close to being right on top of each other. The A320 pilot didn't say he received an RA so I believe he was just watching his TCAS or had visually scene the aircraft. This is my second facility. If there was one thing that bothered me most about the way traffic is worked at [my first Center] it would be the use of what we call the "flash through" or interim altitude procedure. This procedure in my mind is dangerous and should not be used the way it is used here. Basically, if you have an aircraft that wants higher or lower, the procedure here is to put in higher or lower the data block and flash to the next sector. The next sector then accepts the hand off and takes out the interim or assigns another interim they can accept the traffic at and the transferring controller assigns the aircraft that altitude. To take this one step further, if the aircraft is close to the boundary of another sector, the initial receiving controller can elect to "flash the aircraft on to the next sector". There have been plenty of times when I have seen this procedure create confusing situations and also more work for controllers through the resulting coordination. It is extremely easy to create an unsafe situation with this procedure as seen with the above example with the PC12 and the A320. I have tried voicing my concern about this procedure since I started training on the floor and it has taken me a long time to even begin to become comfortable with it. This procedure is so ingrained that it is almost impossible not to participate in it and still work traffic. Obviously, because it is the accepted practice here, it is difficult to voice my concern and have it met with open ears. I think it is a misconstrued use of AIT procedures and I would like to see this procedure stopped or at least used with more restriction.

Synopsis

Enroute Controller described a near separation loss, claiming the facility "flash through" procedures are dangerous and should be terminated.

ACN: 1074091 (2030 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Communication Systems
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1074091

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1074098

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After pushback, when the ramp agent disconnected the headset, there was a very loud static squeal making the intercom and radios unreadable for several minutes. The Ramp Agent was called back to aircraft and asked to plug in again. After disconnecting, the radios and intercom returned to normal. Taxiout was okay, but once we were cleared for takeoff, as power was applied, the squeal returned making the radios unreadable. We exited the runway. After further troubleshooting we attempted another takeoff, but the squeal returned. After exiting the runway we returned to the gate for Maintenance to investigate. Takeoff power never got set so a rejected takeoff was not an issue. After further troubleshooting aircraft was dispatched under MEL 23-43-XXXX aircraft flown to ZZZ with no further problem. In ZZZ, Maintenance replaced the headset jack assembly. As soon as ramp agent plugged headset in the squeal returned at the same volume as before. The discrepancy was re-entered in to the logbook for further trouble shooting. I was pressured to take the aircraft. It was not until I explained to the maintenance supervisor that I was not happy to take the aircraft until further troubleshooting had been performed did they change the Audio Management Unit in the Electric and Electronic compartment. The aircraft was returned to service with no further problems noted. Total delay [was] 3 hours and 30 minutes.

Narrative: 2

The aircraft developed a whining noise in the ground interphone at pushback. The problem was thought to be remedied but reoccurred upon spool up on the runway. The aircraft taxied back off of the runway and attempted remedial action and contacted Maintenance personnel. The aircraft again took the runway and the condition reoccurred. The crew then returned to the gate for Maintenance. As the aircraft never reached takeoff power and the brake was never released it was not thought that either would be considered a rejected takeoff. Apparently the Dispatcher on the afternoon rotation who relieved me was contacted and asked why the rejected takeoffs were not reported. He submitted a report at that time regarding the occurrences and e-mailed me to advise me of the situation. I am submitting this as clarification on the matter. I have always regarded takeoff to begin at brake release when the aircraft began to move down the runway under takeoff thrust. As this condition was not met I did not see a requirement for a rejected takeoff report to be submitted and consequently did not submit one for either of the events.

Synopsis

The Captain and Dispatcher describe the radio and intercom issues on an A321 after the pushback crew disconnected the headset. The problem returned when the throttles were advanced for takeoff, so the crew performed a slow speed reject. After troubleshooting, the Flight Deck to Ground Interphone system was deferred as inoperative. However, when the next station replaced the headset jack and the system reactivated, the problem returned. The system operated normally after the Audio Management Unit was replaced.

ACN: 1072285 (2031 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 6500

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway XX
Flight Phase : Initial Approach
Route In Use : Vectors
Route In Use : Visual Approach

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1072285

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Was getting vectored for ILS approach and had been slowed to 210 knots by ATC. Was descending to 5,000 feet and upon passing 7,000 feet I called for Flaps 1. At approximately 6,500 feet got a F/CTL Slat Locked-Wingtip Brk ECAM. We told ATC that we had an issue to resolve and asked for holding or vectors. They said that they would give us vectors and asked us to maintain 6,000 feet. We complied. We followed all ECAM/QRH standard procedures in preparing the aircraft for landing.

The First Officer was the Pilot Flying and I worked on the slat issue, conferring with him as necessary. I declared an emergency with ATC at ZZZ, advised the flight attendants that we had declared the emergency and to prepare the cabin but it would be a normal landing. I spoke to the passengers and advised them that the landing would be normal however they might see some equipment standing by for precautionary purposes, by our request. I also sent off a quick message to Dispatch to inform them of our situation. I never had time to check their response. 

At this point our fuel was already down to 7,600 pounds in spite of the fact that I had added 1,500 pounds to our fuel load [before departure] because I was unhappy with Dispatch's fuel load which had us landing with 7,300 pounds. We stopped the rest of what we were doing and requested the approach. We made an uneventful ILS to a visual approach in good VFR weather. We taxied to the gate and left the flaps in their landing configuration. 

My only comments are that I feel that the QRH procedure for this problem should be simplified or made a little more user friendly. There are points that I think could be a little more clearly presented by a simple change in verbiage. Also, the fuel issue here is an excellent example of a continuing problem with regards to insufficient fuel loads, especially on longer transcontinental flights. I was extremely happy that I had added the fuel but actually wish that I had added more.

Synopsis

An A320 flight crew complied with all appropriate checklists, declared an emergency, and landed safely after encountering a malfunctioning flap/slat extension system.

ACN: 1072026 (2032 of 2908)

Time / Day

Date : 201303

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Tablet
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1072026
Human Factors : Troubleshooting
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Training / Qualification

Events

Anomaly.Other 
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

During my 48 years of flying, 36 as an airline pilot, I have witnessed major improvements in equipment and technology. Most were fully tested and vetted prior to their introduction onto the flight decks of our passenger aircraft. The latest such technology is the I-pad. 

My comments need to be prefaced by stating that computers are a revolutionary technology with which I am very comfortable and in the use of which I am extremely well versed. That being said, the rush to put the I-pad into full use aboard our airline's aircraft--WITHOUT BACKUP PAPER MANUALS BEING ON BOARD--is an extremely poor decision and could lead to unintended outcomes. There must be more testing, training and evaluation prior to their full implementation without paper backups being on board. 

There needs to be several training and checking cycles during which the electronic pad is used as the primary information and reference source. Data from these events should be collected, evaluated and codified to determine the practicality and desirability of full implementation without such redundancy. The collected data needs to be compared against a base line established from training and checking sessions using the existing paper manuals. This is the only means to determine the feasibility and safety of full implementation. 

The comparative data collected must be the evaluations of line pilots only, and not skewed by the inclusion of data related to line check airmen, pilot instructors or standards Captains. These individuals are highly trained in the use of the I-pad and their jobs require daily usage. This is not the case with the average line airman. I strongly urge our managers to abstain from their "RUSH TO JUDGEMENT AND IMPLEMENTATION" of the I-pad without far more training and evaluation. An aircraft accident or incident related to the early implementation without proper training and evaluation would be difficult to justify.

Synopsis

An A320 Captain discussed his concerns with respect to his airline's, in his opinion, rush to implement the availability of aircraft operational content solely by "pad type" hand held electronic devices. He believes their use as the sole on board procedural resource should be proceeded by extensive testing and evaluation of the average line pilots' facility in its use.

ACN: 1071941 (2033 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : JFK

Person

Reference : 1
Location Of Person.Facility : N90.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1071941
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

JFK was on a 4 L/R operation with southwest traffic departing 31L, along with doing the WHITE/WAVEY route swap. There was a large volume of traffic off Runway 31L going to the next sector. Due to volume, they stopped traffic going to that sector. I called JFK Local Controller and verbally stopped all departures off Runway 31L. Unknown to me, JFK Tower and New York TRACON Traffic Management Unit (TMU) were discussing southwest departures off JFK and the volume/in-trail. Someone either in New York TRACON TMU or JFK Tower believed that meant that southwest bound traffic was released and started to depart Runway 31L without my approval on JFK departure RADAR position or without coordination/approval from the JFK Sector sequencer. This caused an airspace deviation into my sector. Traffic was stopped and should have NOT resumed. JFK Tower should be mandated to get release on all departure stops from the JFK area/Departure Controller. Even if there is a fix stop by TMU. The reason is that if there is either a stop from the Departure Controller or the TMU, the sector may use that airspace for other reasons that the Tower or TMU are not aware of. The Tower or TMU just releasing traffic into a sector without the sectors knowledge is very dangerous.

Synopsis

N90 Controller expressed concern regarding the release procedures used for JFK Tower and the interaction of the TMU vs. the departure sectors.

ACN: 1071852 (2034 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Temperature - Extreme
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Galley Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1071852
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Primary Problem : Incorrect / Not Installed / Unavailable Part

Narrative: 1

Inbound flight from ZZZ1 to ZZZ for Routine Overnight Maintenance (RON). During RON and (interior) Seat Inspection (Seat Program), I noticed what appeared to be a possible leak from the Aft Galley on an A320 aircraft. A large puddle extending to the lavatory and the Aft Right Entry Door (which had began icing over) was found and later determined after inspection of the galley, to be solely from a galley trolley ice cart. Inspection of the ice container revealed that it was installed into the aircraft as an unserviceable unit, as it was wrapped into a black garbage bag to attempt to prevent leakage. After removing remaining ice and the trash bag, the container was found to be marked in black marker SEVEN times, as being in an unserviceable condition and having an old Company Material (COMAT) label dated September 2012, ZZZ1 to ZZZ. Markings were as follows: Leaks," "Leeks," "Broken," "Do Not Use," "Leaks," Leaks," "Broken." The container was also determined to be missing the access doors and hinges and appears severely damaged with sharp edges, where the hinges would be mounted.

Clearly an unserviceable part was allowed to be placed into service. As the container previously was COMAT to ZZZ1 per the attached label, it can be assumed it was taken out of service by ZZZ Maintenance and routed for proper disposition and repair to ZZZ1 company Stores. I believe an investigation should be conducted to determine via COMAT and ZZZ/ZZZ1 Stores records, where the failure to quarantine and properly handle the unserviceable part is exhibited. Water ingress from galley ice containers has been an issue repeatedly addressed, and the resolve of utilizing better performing containers has closed many issues relating to the matter. Maintaining these containers in Serviceable condition is necessary to ensure compliance for safe operations and with Company policies and FAR's, but can easily be overlooked as Catering Vendors may choose to not abide to such. Due to the extreme nature of the markings on the container and the manner of "preventing" continued leakage with a trash bag, it is my believe a poor decision was made by a Vendor allowing this container to be placed back into service, possibly on multiple occasions. A review of Policies and Procedures for our Catering vendors maybe in order to ensure no further violation reoccurs.

Callback: 1

Reporter stated the Galley Service Carts have an insulated container to store large amounts of ice that should not be leaking when in-service on the aircraft. Even though the Aircraft Manufacturer installs Mylar sheeting material under the galley moon mats, to reduce water ingress under the galley flooring, lots of corrosion damage still occurs on the galley floor beams from leaking ice carts. Heavy Maintenance is required to repair the damaged floor beams. The ice containers are approximately three feet long and one foot high and are placed inside the Service Carts. No one appears to be taking responsibility for addressing the issues of unserviceable galley ice carts and Contract Vendor serviceability requirements.

Synopsis

A Line Mechanic suggests that an investigation should be conducted to determine where a failure to quarantine and properly handle unserviceable galley ice cart containers had occurred, after noticing that a large puddle, extending from an aft galley ice cart to the lavatory and the aft right entry door on an A320, had started to ice over.

ACN: 1071679 (2035 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZMA.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 38000

Aircraft

Reference : X
ATC / Advisory.Center : ZMA
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZMA

Person

Reference : 1
Location Of Person.Facility : ZMA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Other / Unknown
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1071679
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

[The A320] was a handoff from ZJX and by LOA should have had BAHAA HIBAC TRV. The A320 was BAHAA TRV and got within a mile of Sealord before he was pointed out. When I'm by myself never assume ZJX will give you every aircraft per LOA. Check every handoff flight plan. I [informed] ZJX of their error.

Synopsis

ZMA Controller reports a botched handoff from ZJX resulting in a late point out to military controllers at Sealord.

ACN: 1070948 (2036 of 2908)

Time / Day

Date : 201303
Local Time Of Day : 0001-0600

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Electronic Flt Bag (EFB)
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Sea
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Rotorcraft
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1070948
Human Factors : Human-Machine Interface

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Primary Problem : Company Policy

Narrative: 1

During my 48 years of flying, 36 as an airline pilot, we have all witnessed major improvements in equipment and technology. Most were fully tested and vetted prior to the introduction into the flight decks of our passenger aircraft. 

The latest such technology is the iPad. My comments need to be prefaced by stating that computers are a revolutionary technology with which I am very comfortable and in the use of which I am extremely well versed. That being said, the rush to put the iPad into full use aboard our fleets, WITHOUT BACKUP PAPER MANUALS ON BOARD, is an extremely poor decision and could lead to unintended, very bad conclusions. 

There needs to be several more years of testing, training and evaluation prior to their full stand alone implementation. There needs to be several training and checking cycles during which the electronic pad is used as the primary information and reference source. Data should be collected, evaluated and codified to determine the practicality and desirability of full implementation. The collected data needs to be compared against a base line established from training and checking session using the existing paper manuals. This is the only means to determine the feasibility and safety of full implementation. 

The data collected must be the evaluations of LINE PILOTS ONLY, and must not be allowed to be skewed by the inclusion of data related to line check airmen, pilot instructors or standards Captains. These individuals are highly trained in the use of the iPad and their jobs require daily usage. This is not the case with the average line airman.

You are strongly urged to prevent our airline from "RUSHING TO JUDGEMENT AND IMPLEMENTATION" of the iPad without far more training and evaluation.

The potential for an aircraft accident or incident related to premature implementation is not acceptable.

Synopsis

An A320 Captain expressed concern about the proposed implementation of iPads as the sole flight deck source of information and data currently provided through paper documents and publications. The reporter believes an extended period of vetting, with the paper documents retained as backups, should be required to insure reliability and provide flight crew with adequate opportunity to gain facility and confidence in their use.

ACN: 1070621 (2037 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 19200
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1070621
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 16500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1070628
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During cruise got an Engine 2 Reverser Fault ECAM with no other messages. Having had this happen [to me] numerous times during my many years on this equipment I was not concerned and knew what to expect when referencing the flight manual. Upon referencing the Flight Manual I was dismayed to see that the first action item was "Land as soon as possible" followed by the next steps which read "if pressurized engine to idle" with no further direction. I knew from previous experience that this seemed either poorly written or mis-written. A reverser fault never required a diversion unless there were other qualifiers such as pressurized system or unlocked situation. Contact was made with Maintenance, Dispatch, Duty Manager and the A320 Fleet Technical Manager who all agreed that the procedure was improperly written. They concurred that a diversion was not required and a continuation to destination was appropriate.

Narrative: 2

[Narrative #2 contained no additional information.]

Synopsis

An A320 Engine 2 Reverser Fault ECAM alerted and even though the Flight Manual directed a diversion the crew knew is was not necessary. Dispatch and the Duty Manager confirmed the procedural error so the crew continued.

ACN: 1070615 (2038 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 17000

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1070615
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1071261
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying. While climbing through 14,000 FT we encountered chop. I advanced the thrust levers to MCT so as to expedite our climb through the chop. When passing through 17,000 FT we got a single chime that drew our attention to the engine gauges. The Number 1 EGT was in the red band. This was accompanied by a fuel flow that was 600 LB/hr higher than the Number 2 fuel flow. I immediately reduced thrust to the climb detent and engaged the autopilot. The EGT stabilized below the red band. The fuel flow remained higher than the other engine. The Captain began coordinating with Dispatch and advising the flight attendants of the situation. I advised ATC of our need to return to [our departure airport]. We landed uneventfully. We came to a complete stop after exiting the runway. We asked the fire trucks to observe the Number 1 Engine for anything out of the ordinary. They stated that nothing seemed abnormal so we proceeded to the gate. During taxi the Number 1 Engine still had higher EGT and fuel flow than Number 2. We deplaned the passengers, spoke to some mechanics, obtained another aircraft, and departed on an uneventful flight.

Narrative: 2

Further examination of engine parameters after the event showed that Number 1 Engine fuel flow was 6,000 LB/hr and Number 2 engine fuel flow was 5,400 LB/hr, Number 1 Engine EGT was 40 degrees higher than Number 2 Engine EGT and remained 40 degrees higher to include ground taxi.

Synopsis

An A320 encountered turbulence during the climb at 17,000 FT so maximum continuous thrust was set to expedite the climb which caused an EGT overtemp so after reducing thrust the flight returned to the departure airport for a precautionary landing.

ACN: 1069980 (2039 of 2908)

Time / Day

Date : 201302

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : KOOLY FOUR

Component

Aircraft Component : Altimeter
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 1069980
Human Factors : Confusion
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 12
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1069980
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

We were nearing the top of descent on our filed and cleared route that included the KOOLY RNAV STAR to PHX. We expected and briefed the arrival for Runway 25L. We then received a clearance to fly direct to SQUEZ and descend via the KOOLY4 for landing Runway 25L. We set the MCP [altitude alert] to 4,000 FT for a managed descent. In the descent I realized that it was time for a new ATIS so I requested one through ACARS. 

Upon passing FL180 we set the altimeters to the just received ATIS altimeter setting (30.37). I am not certain of the exact sequence of the following events, but we received a clearance to delete the speeds on the arrival and change to Runway 25R. This runway does not have an ILS, so we needed to spend some time determining what approach page matches the identifier in the FMGC, in this case I believe it was RNAV Y 25R. 

About this time we were crossing SQUEZ and the Controller informed us ATIS was now Tango and the new altimeter setting. We reset our altimeters, and I requested the new ATIS since we had Sierra. After resetting to the new setting (30.03) we realized that this was a large difference. Soon the Captain discovered that I had requested, received, and told him the altimeter for PDX and not PHX. At this point we realized that we had likely crossed SQUEZ low.

The rest of the flight was normal. The approach planning and briefing was conducted in a non hurried manner which included review of the NOTAMs and runway performance. The Captain is conscientious and disciplined, and he creates an atmosphere in which I should be able to perform my best. In the future I will read the ATIS line by line to try to avoid making the same mistake.

Narrative: 2

It is certainly possible the Controller issued the altimeter setting during the descent, but we either misheard or were possibly distracted with a route change shortly before SQUEZ.

I will certainly give extra scrutiny to ATIS requests, particularly where airport identifiers could be easily entered as a similar airport, such as PDX and PHX or SNA and SAN.

Synopsis

When the First Officer aboard the A320 inadvertently picked up the ATIS for PDX vice their destination, PHX, the altimeter thus obtained may have caused the flight to cross their active waypoint, SQUEZ below the required 15,000/12,000 FT window before they discovered their error.

ACN: 1069807 (2040 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1069807

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Accomplished steps on ECAM. Verified G HYD SYS level low. Referenced the FM for additional information. Declared an emergency with ATC. Flew aircraft to our filed destination. Gravity extended the gear per FM. Accomplished normal landing. Stopped the aircraft on runway. Shut down engines. Got inspected by crash, fire and rescue. Maintenance towed us to gate.

Synopsis

An A320 ECAM alerted HYD G RSVR LO LVL so the crew complied with the QRH, declared an emergency, and continued to their destination where the stopped on the runway, then were towed to the gate.

ACN: 1069796 (2041 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14825
Experience.Flight Crew.Last 90 Days : 45
Experience.Flight Crew.Type : 5405
ASRS Report Number.Accession Number : 1069796
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight declared an In-flight Emergency and landed uneventful at destination airport after experiencing a HYD G ENG 1 PUMP LO PR ECAM while stable at cruise altitude. Pre-dispatch of our flight was normal. Maintenance had cleared 2 inbound write-ups on our aircraft, both were cosmetic cabin items and irrelevant to our event. Due to late inbound arrival of this aircraft, our flight pushed back from the gate nearly 45 minutes late. Our aircraft was deiced with type II and Type IV fluid before initial taxi. Taxi out and takeoff was uneventful. At 21 minutes into our flight (and only a few minutes into level cruise at FL360) crew received a HYD G ENG 1 PUMP LO PR ECAM message. Captain remained as Pilot Flying (PF) while First Officer remained as Pilot Monitoring (PM). Flight crew contacted Dispatch and Maintenance, and all agreed to continue to destination airport. Flight crew informed Center Controller of situation, and declared an emergency. While on downwind leg, crew performed the Landing Gear-Gravity Extension checklist. Approach and landing was normal on runway. Crew used idle reverse thrust on the Number 2 engine as briefed, as the Number 1 engine reverse thrust was inoperative due to the hydraulic malfunction. Aircraft stopped approximately 3,500 feet from the end of the runway on the centerline. CFR personnel approached the aircraft after the engines were shut down, and determined some residual hydraulic fluid was still leaking on the runway. Approximately 25 minutes after landing, the CFR personnel allowed the aircraft to be towed by company tug to planned arrival gate. Passengers deplaned normally through Jetbridge upon gate arrival.

Synopsis

An A320 crew declared an emergency and landed uneventfully at the destination airport after the ECAM alerted HYD G ENG 1 PUMP LO PR.

ACN: 1069544 (2042 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PIT.Airport
State Reference : PA
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : PIT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : PIT

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1069544
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 6000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1069554

Events

Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While on right base leg to Runway 28L at 3,000 FT. we were given a visual and accepted as airport was in sight. Airplane was at 170 KIAS and at flaps 3 with autopilot on. The vector put us on a heading to intercept final at 90 degree angle to final 28L and inside the FAF PITTZ which has a glide slope intercept altitude of 3,000 FT. The glide slope for 28L indicated we were high so I asked for gear down, managed speed, flaps full and final check as I recall. Then we set 2,000 FT in the altitude window and I selected open descent to start down and attempt to get on vertical profile while still correcting simultaneously to a 60 degree angle to final.

We intercepted the glide slope and within 5-6 seconds of starting down we got a "caution terrain,terrain" warning and immediately went to TOGA and disconnected autopilot and corrected situation and climbed back up to 3,000 MSL. We were 5 miles approximately north of the airport and I estimate we were at 2,500 FT MSL when we got the warning. We were not aligned with Runway 28L when intercepting glide slope but well north of the runway so we had no vertical protection. I then hand flew the airplane towards 28L but we were too high to make a safe landing so we proceeded to line up up with 28L and executed a normal go around and received vectors from ATC to return for a ILS 28L and made a uneventful approach and landing.

Narrative: 2

In my estimation, we were approximately 2,500 FT MSL (1,500 AGL) when the GPWS was triggered, on a vector of about 210 to pick up Runway 28L for the visual. It appeared we were taken inside the FAF (approximately 2-3 miles) during this vector, and with the field in sight we were cleared for the visual approach. It could have been a Tower or the ridge on the south side of the river that triggered the caution terrain. The weather was VMC so we could see the ground, although we appeared to be high on approach by virtue of our tight vectoring, wich put us on an aggressive intercept to 28L northeast of the field.

Synopsis

A320 Captain flying visual approach to PIT got a GPWS terrain warning, and after adjusting altitude was too high to continue approach.

ACN: 1069276 (2043 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : N90.TRACON
State Reference : NY
Altitude.MSL.Single Value : 13000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : KORRY 3
Airspace.Class E : N90

Component

Aircraft Component : Altitude Hold/Capture
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1069276
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Captain was pilot flying and was cleared to "descend via the Korry 3 arrival" into LGA. All crossing restrictions were programmed in the MCDU and verified. Captain had planned for a high speed descent as there was no speed restrictions assigned. During descent, ATC adjusted our clearance to include "270 KTS at BRAND." Speed brakes were extended to help meet this assignment and aircraft was in managed speed and managed descent. The aircraft was programmed to cross DAVYS at or above 13,000 FT and the MCDU indicated that this restriction would be met. At approximately 13,300 FT I noticed that with the current descent rate, it did not appear that the level off would occur by 13,000 FT at DAVYS. The vertical speed level-off button was pushed and even then, the descent rate was such that the aircraft was going to descend below 13,000 FT 2-3 miles prior to DAVYS. Just below 13,000 FT, the Captain disengaged the autopilot and leveled off around 12,200 FT followed by a return to 13,000 FT and then the KORRY 3 arrival without further complications. There was no alert or indication by ATC that a crossing restriction had not been met. 

It appears that on the A320 aircraft specifically, using speed brakes at higher speeds on descent will induce descent rates that will not be able to be overcome to meet some crossing restrictions while using the autopilot. In this circumstance, the A319 aircraft probably would not have had a problem, and this event serves to remind that the A320 aircraft are slower to respond while on autopilot to changes in pitch. In any regard, when ATC changes an airspeed clearance on an arrival, in the descent, to something less manageable, it may be prudent to say "unable" next time.

Synopsis

During a high speed descent on the LGA KORRY 3 arrival, ATC issued a 270 KTS BRAND constraint which caused an A320 to go below the DAVYS 13,000 FT constraint by 750 FT.

ACN: 1069174 (2044 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Fan Blade
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1069174

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1069176

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Returned To Gate
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Weather
Primary Problem : Airport

Narrative: 1

We got de-iced and anti-iced in the alley with the engines off. Once cleared by the de-icers we started both engines, put the engine anti-ice on, and proceeded to taxi out of the alley. The alley and ramp area were a mixture of loose and packed snow. Once we were on [Taxiway] D, the taxiways were clear of snow to the pavement, but were still a bit icy. 

As we taxied we noticed a slight engine vibration. At an intersection short of the runway was a good, non-icy portion of pavement where we ran up the engines. We noticed a significant vibration in the Number 1 engine and no vibration on the Number 2 engine. We told ATC that we needed to return to the gate and they cleared us back to the ramp. A post flight inspection revealed several damaged fan blades on the Number 1 engine. Maintenance was called and a log book entry was made.

Narrative: 2

Airport Operations needed to have the ramp and taxiways plowed better.

Synopsis

The flight crew of an A320 noted excessive vibration in the left engine during taxi out after being de and anti-iced at the gate. They returned to the gate for an inspection which revealed several damaged fan blades.

ACN: 1069034 (2045 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DCA.Airport
State Reference : DC
Altitude.MSL.Single Value : 8000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Route In Use.STAR : TRUPS
Airspace.Class B : DCA

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1069034
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : FLC Overrode Automation
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Arrival [waypoint] restrictions [on the TRUPS RNAV STAR] as well as the approach procedure were briefed and verified before top of descent. 

ATC cleared us to cross RANJR at FL280 per the arrival. The Captain started descent a little late plus we had a strong tailwind. He elected to use open descent and selected airspeed instead of Managed Descent Mode. I calculated we were going to be high and informed the Captain who said we were going to be OK. He may have trusted the blue bottom of descent arrow on the ND which showed we would make the restriction; however, this indication is totally inaccurate in open descent mode. He then extended speed brakes but we were still going to be high. 

I informed ATC that we were not able to make the restriction. ATC said it was OK but to cross the next one at or below FL180 as charted. We were also high for that one and again informed ATC in advance. He again said it was OK and to delete the next three speed restrictions and descend via the arrival. We barely made the 18,000 FT restriction by using a lot of speed brake. 

The bottom of the arrival was at 6,000 FT and the Captain set it and we both verified. The last two waypoints on the arrival are RLLLL, to be crossed at 210 KTS/6,000 FT and LACKE also at 6,000 FT. The Captain was using selected speed in the descent and I saw him make the reduction to 210 KTS. I was looking out for traffic but quickly looked at the ND when crossing RLLLL. I noticed we were crossing it at 8,000 FT (the crossing altitude for the previous waypoint, LETZ) and told the Captain. At the same time ATC asked why we were still at 8,000 FT. I told them we simply missed it. The Captain the started the descent to 6,000 FT but ATC told us to descend to 4,000 FT then gave us vectors for the Mt Vernon visual to Runway 1. Everything was normal after that.

I don't know why the Captain changed the altitude to 8,000 FT. Maybe it was to manually make the speed reduction (the way this arrival is coded, the activation and confirmation automatically takes place before RLLLL so the PF has to manually set the 210 KTS.) I did not see him set 8,000 FT nor did he verbalize it or ask for verification. I became partially distracted looking outside for traffic and did not notice the change until it was too late.

I think the arrival would have gone well if it had been flown in Managed Mode instead of selected speeds. The Captain did not communicate an intermediate level off at 8,000 FT. The coding issue with activation and confirmation automatically taking place before a charted speed restriction is an unnecessary distraction that may have contributed to the event.

Synopsis

An A320 First Officer reported that their flight was unable to comply with multiple crossing constraints on the TRUPS RNAV STAR to DCA. The reporter cited tardy initiation of the descent, a strong tail wind and the Captain's decision to not use the Managed Descent VNAV mode as contributing factors.

ACN: 1068620 (2046 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PDX.Airport
State Reference : OR
Altitude.MSL.Single Value : 1300

Aircraft : 1

Reference : X
ATC / Advisory.Tower : PDX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 10L
Flight Phase : Final Approach
Airspace.Class C : PDX

Aircraft : 2

Reference : Y
ATC / Advisory.CTAF : VUO
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Plan : VFR
Flight Phase : Initial Approach
Route In Use : None
Airspace.Class C : PDX

Person : 1

Reference : 1
Location Of Person.Facility : PDX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1068620

Person : 2

Reference : 2
Location Of Person.Facility : PDX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Other / Unknown
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1068633

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 was on Runway 10L ILS approach to PDX. When he checked on my frequency, I immediately issued traffic on two targets that appeared to be in the pattern for Runway 8 at VUO. The pilot stated that he was just starting to break out at about 2,000 FT, but did not have the traffic in sight due to rain and poor visibility. One of the targets appeared to be turning base in front of the A320, so issued traffic again. He never saw the traffic. After passing VUO, the pilot stated that he had been unable to stay on the glide slope due to the traffic. He was not happy and obviously felt it was an unsafe operation. He was still able to continue visually and land. Open a Tower at VUO and control the traffic. Current VUO procedures are a joke. The VUO advisory position monitors a CTAF frequency but can't control anything. It's also confusing to the pilots.

Narrative: 2

I was on the Pearson advisory position, monitoring the VUO CTAF frequency. Several aircraft had reported in the VUO Runway 08 traffic pattern, including a C172, a SR22, and another C172. An A320 was on an ILS approach to Runway 10L at PDX. I issued traffic information and a wake turbulence advisory on CTAF when the A320 was about 3 miles west of VUO descending through 020, this is about 6 mile final to Runway 10L. At or about that time the A320 pilot leveled off and left the glide slope due to the VUO traffic. I issued information again as the A320 passed over VUO at 1,300 FT descending. The 10L glide slope goes directly over the 08 threshold and penetrates the airspace. Two of the VUO aircraft reported light chop on short final below 100 FT. The ceiling was about 018. Put a Control Tower back at VUO to control the traffic.

Synopsis

PDX Controllers voiced concern regarding the current procedures between PDX arrival traffic and aircraft operating at the VUO airport. The reporters recommend an ATC Tower be established at VUO.

ACN: 1068347 (2047 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Emergency Exit
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Hangar / Base
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1068347
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

A320 aircraft right wing overwing slide deployment, February 2013, late morning. Aircraft was in Heavy Maintenance Bay Hangar. While performing Lighting Strike Inspection, was preparing to perform Aircraft Crown Inspection, [but] noted aircraft crown was still wet from snow melting. At that time decided to perform top of Wing Lighting Strike Inspection. [I] entered cabin to gain access to wing top [and fuselage] side surface. At that time I was thinking about completing the right Wing Lighting Strike Inspection before lunch break [approximately thirty minutes away]. I did not consider our safety program that has [another mechanic verify certain procedures are performed by the working mechanic before activating critical equipment or systems, such as overwing slides]. Removed cover from [Overwing] Emergency Exit access handle and pulled handle. Stopped handle movement at sound of overwing slide inflating. After slide deployment, realized did not disarm right overwing emergency slide. [I] called Lead Mechanic and Maintenance Supervisor. [Recommend] having two each placards installed on both sides of "PULL" handle labeled "SLIDE" and one placard labeled "SLIDE" on the interior cover, to alert the mechanic to a caution: Note: Exit is "SLIDE" equipped.

Synopsis

Mechanic reports he inadvertently deployed the right wing overwing slide on an A320 aircraft when he pulled the cabin overwing exit handle without disarming the door. He had attempted to gain access to the right wing upper surface for a Lightning Strike Inspection.

ACN: 1068187 (2048 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : D01.TRACON
State Reference : CO
Altitude.MSL.Single Value : 23000

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Facility : D01.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1068187
Human Factors : Situational Awareness

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was working both north arrival gates, with light traffic. LANDR/SAYGE had just handed me a string of arrivals over the SAYGE metering fix, all approximately 5 miles in trail. The last in the line of airplanes was Air Carrier X. I observed he was between the metering fixes of LANDR and SAYGE and attempted to contact ZDV sector to find out what he was doing. I called 3 times with no response and on the fourth attempt, they finally answered. When I asked them what Air Carrier X was doing (already in my airspace and NOT on my frequency at this point), they advised he was "direct EEZEE" (which is over 5 miles inside my airspace and 15 miles from where transfer of communications is supposed to be affected). I told him that wouldn't work for me, as Air Carrier X was 150 KTS faster than the aircraft he was following, 5 miles ahead and asked him to turn Air Carrier X westbound. The ZDV Controller advised me he had already sent him to my frequency. I immediately reached out to Air Carrier X to turn him westbound and keep him separated from the traffic he was rapidly overtaking, and slowed him to 250 KTS. I then called ZDV back to ask him if he realized he gave me Air Carrier X 5 MIT of the preceding aircraft with a 140 KTS overtake, the controllers response was "he was descending via and should have had the restriction." I don't understand how he could have had this "descend via restriction" when Air Carrier X was instructed by ZDV to join the arrival inside my airspace? Also, even if he was given the restriction, he was way too close and RAPIDLY overtaking the preceding aircraft. Response to this report was that the issue has been resolved by the implementation of RNAV. This could not be further from the truth. ZDV is not even remotely using RNAV as it was designed, and the same problems continue to arise on an hourly (more like minutely) basis. The FAA and airlines have spent millions of dollars on RNAV (NexGen), only to have ZDV race the airplanes to the metering fixes and then give them the "descend via" often inside D01 airspace, for a very brief few miles, wherein we at D01 usually have to take them off the RNAV route, just to get separation to the runway. The purpose of RNAV was to allow the airlines to float down, saving in fuel costs. How is this being accomplished by forcing aircraft max speed to my airspace boundary, then descend via at the last possible second? My solution is to force ZDV to use the RNAV routes, as designed, and if they need more in-trail spacing to accomplish this, place restrictions on the second tier Centers. We are the NAS, National Airspace System, and we should be working together to accomplish more effective, safer traffic flows. If this is an unacceptable solution, my second recommendation would be to cease the use of 8 STARS into DEN. It is almost completely ineffective during times of moderate or greater traffic, especially the way ZDV feeds us, as is evident by the lack of use of 8 STARS in the rest of the NAS. At best we could feed 4 STARS 5-7 MIT and use the other 4 as offload STARS 25 MIT. Although DEN has 6 runways DEN Tower rarely lets D01 use more than 2 at a time. The 8 STAR arrival system DOES NOT WORK to 2 runways, but we continue to work harder and not smarter.

Synopsis

D01 Controller described an over take and potential loss of separation event between DEN arrival aircraft, claiming that ZDV Controllers fail to utilize the RNAV STAR procedure effectively.

ACN: 1067822 (2049 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 19000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : INS / IRS / IRU
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20250
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 2200
ASRS Report Number.Accession Number : 1067822

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1067827

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise flight we had a fault light on IR Number 2. We complied with the ECAM and referred to the FM and completed the additional steps. We did switch the IR to ATT mode and entered our heading per the FM but the IR seemed to remain completely inoperative. The MRM code for the malfunction was entered into ACARS. Approximately 55 miles out during descent IR Number 3 faulted. This resulted in the loss of the A/T along with both autopilots and flight directors and the First Officer's flight displays and put us into alternate law. I became the PF and the First Officer completed the ECAM and referred to the FM. The ECAM kept repeating and was difficult to clear and step through. By this time we were with Approach Control and cleared direct to a fix on the ILS to intercept. Approach control advised us LLWAS was in effect with winds out the N-NW at 10-15 KTS and that there had been recent reports of W/S. We were less than 30 NM out and there were still more of the checklist to read through and also the Approach/Landing Corrections for Failures procedure to complete. Considering these factors and since I was hand flying on raw data and the First Officer was very busy we decided it was prudent to declare an emergency and request delay vectors. The First Officer completed the checklists, the landing procedure and landing distance calculations, and notified Dispatch. The flight attendants and the passengers were briefed. I did not ask for cabin prep or request ARFF equipment. We completed an extended 270 degree right turn and landed uneventfully on the slightly longer runway. The airplane was in direct law for landing and flaps 3 per the FM. The wind was reported I think 350/11 with no recent W/S. The ARFF trucks were out and followed us to the gate. We briefed Maintenance and the outbound flight crew. Had we been further out when the second IR failed we may not have declared an emergency. In that case we would have had plenty of time to do everything and this would have been more of a non-event. However this situation presented like it would have made a good traditional simulator training scenario happening so close to arrival. Over the years I have been advised that declaring an emergency helps buy time and reduce workload with the service ATC provides. We did not especially want to hold with raw data or be rushed in for landing. I also thought the risk of a pilot deviation was greater due to no autoflight at all and the workload as it was.

Narrative: 2

About 2 hours into the flight we lost our No. 2 IR. I was just returning from the bathroom so the Captain had complied with the ECAM. We reviewed the Flight Manual (FM) and sent in the maintenance code. About an hour and a half later, in VFR conditions, while on the arrival descending below 20,000 feet the autopilot kicked off and I, Pilot Flying, (PF), lost most of my Primary Flight Display, (PFD) and Navigation Display, (ND). We had lost the No. 3 IR and got the ECAM Nav IR 2 + 3 Fault. The Captain quickly finished the Approach Descent Checklist and then he became the PF. I went through the ECAM checklist and then to the FM. The checklists were somewhat long and I could not get them done as ATC was vectoring us now for landing. The Captain opted to declare an emergency at this time to give us more time to complete the checklists associated with the loss of IRs 2 + 3. ATC was great giving us vectors around while we completed the checklists and informed dispatch. After calculating the landing distances and going through the F/CTL Direct Law non-normal procedure we reviewed all we had lost and when the aircraft would go from ALTN LAW to Direct Law and Manual trimming of the aircraft. When we were ready we notified ATC and told them we would take the longer runway. We received vectors to final for an uneventful landing flown by the Captain and taxi-in was normal. We did leave the ADIR's on at the end of the shutdown checklist to facilitate Maintenance troubleshooting and talked to Maintenance and the Fire Rescue who came on board to check to make sure everything was alright.

Synopsis

A320 flight crew experiences the loss of two IRU's. The first in cruise and the second an hour and a half later during descent. This results in Alternate Law flight controls and Direct Law when the gear is extended for landing. The time required to accomplish all checklists is long and the crew elects to declare an emergency to accomplish them.

ACN: 1067712 (2050 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1067712
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Fluid was discovered on the bottom of the Number two engine cowl during the exterior inspection. There was also fluid that had accumulated on the ground as the aircraft sat overnight. The engine drain mast showed no signs of fluid leakage. A log item was entered and Maintenance was contacted. I was waiting outside when Maintenance arrived at the aircraft. As the Mechanic approached the engine, he immediately stated that the fluid on the ground was old and not significant in quantity. I showed him the fluid that was on the bottom of the cowl and stated that I believed the fluid on the ground was fresh from this aircraft's engine. His tone was extremely condescending and dismissive as he stated that this was completely normal for these engines. I explained to him that I was required to inform Maintenance of the defect and that in my experience this was not normal for the Airbus. As I began to return to the cockpit, he stated (without any inspection) that the fluid was due to an over service of the engine. The Mechanic followed me to the cockpit where the discussion continued in the presence of the Captain. The Mechanic was clearly irritated that I had entered the log item and that I disagreed with his assessment of the source of the leaking fluid. He then abruptly left the cockpit and returned to the Maintenance area. Shortly before pushback we received the MRD. The log item was signed off as "Inspected Number 2 Engine, no leaks noted". It is important to note that the Mechanic never wiped the fluid from engine cowl, never opened the engine cowl, and never did any kind of inspection. He signed it off as "no leaks noted" even though he admitted to both me and the Captain that there was fluid on the ground and on the bottom of the engine cowl. After pushback during the engine start, the Number two generator failed to come on line automatically. I was able to bring the generator on manually, but was concerned that these two items may be related. Upon arrival in ZZZZ I inspected the engine and found the underside completely soaked in fluid as well as fluid already beginning to accumulate on the ground. The outbound crew was briefed.

Synopsis

A320 First Officer discovers fluid leaking from the Number two engine during preflight and contacts Maintenance. The Maintenance Technician states that the small leak is normal and signs off the logbook with "Inspected Number 2 Engine, no leaks noted" without opening the engine cowl or wiping down the fluid on the bottom of the cowl.