ACN: 1067128 (2051 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : D01.TRACON
State Reference : CO
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : KOHOE ONE
Airspace.Class A : ZDV

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1067128
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1067127
Analyst Callback : Completed

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were cleared to "descend via" Kohoe One Arrival EXCEPT to maintain 270 KTS till SLMON. Captain typed 270 KTS in PERF page (DES) of MCDU. We crossed HERNG at FL320 and 270 KTS in managed descent. The aircraft started a slow descent after HERNG and barely made KOHOE at FL280 (270 KTS). To compound the situation (and we believe this to be the determining factor to the issue), ATC slowed us to 230 KTS. The aircraft performance was unable to lose 6,000 FT between KOHOE and SAYGE (only 15 DME) with max drag (spoilers) and a change in automation to OPEN DES (because managed descent had us leveled off trying to slow to 230 KTS). We advised ATC unable to cross SAYGE below FL220; then advised unable to cross MOKME below 17,000 FT. ATC then gave clearance to maintain 250 KTS and descend to 14,000 FT.

We believe trying to slow the aircraft started a chain of events that would not allow the aircraft performance to cross at or below the crossing restrictions. The Captain was using good CRM the entire time and was aware immediately that we would not be able to cross SAYGE below FL220 and MOKME below 17,000 FT. 

With the experience, I would notify ATC (on future flights) that we could either give them the speed or altitudes, but not both.

Narrative: 2

Three things. ATC should not expect to give additional speed constraints without possible difficulties in flying profile descents. The software on this approach needs to be re-evaluated for possible errors. Finally, ATC should acknowledge that the deviation in altitude was not a concern. 

...we checked and crosschecked each point on this arrival and flew the arrival as trained. We did not change any constraints and if not for intervening and notifying ATC we would have zipped right through one of the programmed constraints built into this arrival. I suggest taking a close look at the programming of this arrival.

Callback: 2

The reporter verified what was implied in his reassurance about flying the arrival "...as trained" and the fact they "...did not change any constraints." He advised SOP at his carrier does not allow flight crews to physically alter constraints in the FMC/Autoflight system. He agreed, however, that alterations to those constraints required by ATC can quickly become a quagmire requiring much flight path button pushing in attempts to satisfy both the FMS and ATC and will, at times, result in an inability to comply with constraints as happened in this case.

He also restressed his belief that stated relief from responsibility for any deviations that result from ATC intervention should accompany any such clearance.

Synopsis

When cleared to "descend via" the KOHOE RNAV STAR to DEN "except maintain 270 KTS" the flight crew of an A320 was unable to comply with subsequent altitude crossing restrictions at SAYGE. The reporters cited the initial 270 KTS and subsequent 230 KTS ATC restrictions as primary contributing factors.

ACN: 1067125 (2052 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 19000

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZAB

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1067125
Human Factors : Confusion
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were on arrival on the GEELA RNAV STAR for runway 25L. We were taken off the approach 3 or 4 times for traffic spacing. Additionally, there were several speed changes during the arrival. Our last clearance vectored us on a North heading and to descend to FL 190. Then we were cleared direct to GEELA and to join the arrival we began our descent to cross GEELA below 16,000 feet. I noted that The First Officer set the altitude to 7,000 feet on the MCP and the FMA indicated we were in managed descent. Passing 18,000 feet we began our descent checks, made the arrival PA and read the Descent Approach checklist. MNSTR is the arrival fix after GEELA and requires an altitude between 15,000 feet and 16,000 feet. I looked up and saw we were passing below that crossing altitude. The First Officer noticed it at the same time and made corrective action.

I am not sure why the computer failed to capture the crossing altitude except that perhaps since these crossings were so close together and being so high on profile due to ATC spacing.

I have noticed this trend and while it's not the prescribed way to descend on a managed descent I feel that selecting the lowest altitude for the arrival is part of the problem during these kinds of situations. I think it would be better to set the lowest crossing altitude for the next fix in the MCP when high workloads are expected in this case if 16,000 feet was set the MNSTR crossing would have been honored. Additionally, continued resetting of the MCP ALT window will keep the crew in the loop even in periods of low workloads on these arrivals.

Synopsis

An A320 crew failed to arrest their descent as required while utilizing Managed Descent to vertically navigate the GEELA RNAV SID to PHX. The reporter suggested setting each waypoint's lowest crossing restraint to ensure compliance rather than their current SOP of setting the ultimate lowest altitude and depending on Managed Descent to comply with intervening altitude and speed constraints.

ACN: 1067084 (2053 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 26400

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZDV

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 999
Experience.Flight Crew.Last 90 Days : 99
Experience.Flight Crew.Type : 9
ASRS Report Number.Accession Number : 1067084
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15500
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 4800
ASRS Report Number.Accession Number : 1067096

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

The airplane was unable to slow from 280 to 270 and descend at the same time to cross WOOKY at FL260 and 270 KTS. It's not an airplane issue; it is a physics issue with the crappy TSHNR1 RNAV STAR.

Narrative: 2

Prior to descent we discussed and verified proper altitudes and speeds programmed in the FMGC. In the descent, however, the green donut several times showed us high when in reality we were low on the profile. We tried to "recruise" to fix the issue but were still not confident in the information calculated by FMGC. Due to the confusion we did get high and I mentioned to First Officer he needed to take action.

Synopsis

An A320 flight crew was unable to comply with the crossing restriction at WOOKY on the TSHNR RNAV STAR to DEN.

ACN: 1066945 (2054 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 11000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : MOLTN1
Airspace.Class E : D01

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1066945
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1066946
Human Factors : Workload
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I failed to cross HIMOM on the arrival at the prescribed 11,000 FT. I was flying the aircraft on autopilot in the select speed and V/S modes. As I approached HIMOM I pushed the "PUSH TO LEVEL OFF" button and then dialed my speed down to 210 KTS. As I immediately went from the V/S knob to the speed knob, I failed to check the PFD to ensure that I had entered "V/S zero" mode. My speed was not slowing as I thought it should so I employed the speedbrakes, further focusing on making my speed restriction. At this point the First Officer, who had been heads down inserting a runway assignment change, caught the deviation in my altitude and brought it to my attention. Before I could take corrective action Approach Control (who I learned later had noticed the deviation at this time) issued me a lower altitude. I continued to landing with no further incident.

I have had a personal problem with these new RNAV approach procedures from the beginning. With the Airbus' frequent +/- 20 KIAS and 200 FT airspeed and altitude tolerances, I have yet to fly one of these arrivals fully managed where I have not had to intercede (or should have) to make an acceptable crossing restriction. Even during the portions of the arrival that were flown fully managed, it was like riding a roller coaster as the airplane pitched up and down to meet the seemingly arbitrary altitudes it had chosen to cross each waypoint. The engines spool up and down between 0% and 60% and that is just to stay within the [system's wide open] 40 KTS window. It makes for a very poor ride for the passengers and should be an affront to any professional pilot. 

On previous arrivals I had tried using "selected speed" to stabilize the engines while allowing the autopilot to descend in the managed mode. The pitching was even worse as now I'd taken away its 40 KTS window. This leg I tried for the first time to fly the arrival with V/S and Selected Speed selected. The descent was very pleasant and only required occasional tweaking of the V/S to stay on profile. The engines stayed steady to hold speed. If I had managed to actually activate the push-to-level-off function at HIMOM the arrival would have been perfect and would be my preferred way of doing the arrivals from now on. Since I didn't, all I really managed to do was substitute the imperfections of the autopilot with my own. 

I have only flown a few of these arrivals where Approach Control has not vectored me off for 'spacing'. I have never flown one where the throttles go to idle at the top of descent only to spool up at the final approach fix, as we were told would be the case during training. Before [the implementation of OPD RNAV STARS], I only had to sweat making one crossing restriction at either TOMSN or DANDD. Now I have to sweat a dozen on each arrival into DEN. And I can now blow the restriction in either selected or managed mode! 

That's called progress I suppose. 

I suggest that we give the guy who thought of these arrivals a pat on the back, a promotion with a big fat bonus check and a corner office as far away as possible from anything important that may be going on in aviation. Then [trash] can these arrivals and let's get back to flying airplanes instead of generating deviation reports. 

I want to add the First Officer was doing his job and there was nothing procedurally he could have done to prevent this, given the duties he was performing at the time. This incident was entirely my fault.

Narrative: 2

The Captain was flying the aircraft on autopilot using 'vertical speed mode' as we were "descending via" the MOLTN1 RNAV arrival into Denver. 

The use of "Managed Descent" mode of the autopilot could have prevented the deviation. Better monitoring of the approach during periods of high workload/task saturation on my part should have caught the error in time to level off.

Synopsis

The Captain of an A320, "descending via" the MOLTN RNAV STAR to DEN, utilizing Vertical Speed and Selected Speed modes vice Managed Descent, failed to comply with the "intermediate" crossing restriction at HIMOM when he failed to ensure the jet was leveling at the prescribed 11,000 MSL. The First Officer alerted the deviation at 10,700 and ATC issued a timely further descent.

ACN: 1066874 (2055 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ
Altitude.MSL.Single Value : 13000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Airspace.Class E : P50

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1066874
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Procedure

Narrative: 1

We were taken off the GEELA RNAV STAR, MOHAK transition, given numerous heading and airspeed clearances and then cleared direct GEELA and cleared to "descend via" the arrival. We verified and executed the clearance, then selected Managed Descent.

At this point we transitioned FL180 and I called for the descent Approach Checklist. Upon reaching MNSTR I noticed that we were at 13,000 although the procedure required crossing between 16,000 FT and 15,000 FT and at 250 KIAS. I advised the Captain, verified the situation, ensured we were in Managed Descent and complied with all remaining restrictions.

I'm not sure what happened! I just do not know how much more I can do. ATC is always issuing new clearances and restrictions which create a lot of confusion when running checklists and managing the vertical path and ensuring compliance with restrictions. 

We get way too many clearance modifications from ATC on all of the arrivals, all of the time and on a regular basis.

Synopsis

An A320 flight crew, as well as the Managed Descent mode of the FMGC, were flummoxed by multiple ATC directed modifications to the GEELA RNAV STAR and descended to cross MNSTR at 13,000 vice between 16,000 and 15,000 as required. The reporter expressed his dismay for the complexity of the STAR procedure, the inability of his autoflight system to comply with that complexity and especially for modifications to it given by ATC which may render the flight crew unable to comply due to excessive workload.

ACN: 1066849 (2056 of 2908)

Time / Day

Date : 201302

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Icing

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1066849
Human Factors : Situational Awareness
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Evacuated
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

My First Officer and I arrived one hour before departure at gate. I immediately went to operations and my First Officer stayed on the plane to prepare the plane for departure. I returned back to the plane 15 minutes later after flight planning. I started doing my cockpit checks as my First Officer stepped out to do the walk around. Gate agents started boarding the passengers. Purser notified me that cabin temperature was cold and she needed water pressure. I started the APU after the fire test and I turned on the air condition packs for heat. Shortly after, Purser notified me of smoke and haze filling the cabin. My First Officer walked in and said that there was excessive deicing fluid dripping from the airplane. I also smelled a strong glycol smell. I immediately turned off the packs and APU bleed. I also ordered the Lead Flight Attendant to deplane the passengers. The smell, smoke and haze started burning eyes and throats of the passengers and crew. I notified Operations of the situation and called the mechanics to the plane. I also asked the deicing to be stopped immediately. Everybody quickly departed the plane using the Jetway. One older female wheelchair passenger was helped to leave the plane. The Lead Flight Attendant noticed that passenger was having difficulty breathing. She brought the portable oxygen bottle from the plane and started giving oxygen to the passenger. Paramedics were called and she received medical assistance inside the terminal. Eventually she was taken to a hospital. Customer Service (CSR) Supervisor and Gate Agents were all notified and passengers were moved back to the terminal. Mechanics showed up and started working on the plane. They ventilated the plane by using external air and air conditioning packs. I also called both Duty Manager and the Maintenance Control to report the situation. Maintenance Control made a write-up for us since we could not access the airplane. I also had a chance to talk to deicing Truck Driver. He told me that they thought that we were not in the cockpit before they started deicing. So they did not attempt to contact us to configure the airplane for deicing. He said that they thought the APU was off and Boom Operator was experienced. He said that Boom Operator must have accidentally sprayed the APU intake. It took a long time to glycol smell to disappear. Cabin crew and passengers did not want to board till airplane completely cleared of smell and haze. I noticed that Lead Flight Attendant and one other flight attendant's eyes were red and irritated from the glycol. The Lead Flight Attendant also started losing her voice. Finally after one hour, aircraft cabin air returned to normal and we started boarding. One couple with a young baby refused to board and returned to terminal. We departed 1 hour 5 minutes late due to this incident. I think similar incidents can be easily avoided in the future by communicating with flight crews and configuring the aircraft for deicing. Unfortunately I have been noticing a growing trend of lack of communication between the departments or employees. Necessary communication or information exchange has been seen as a burden by maintenance or ops personnel. It is a big treat to our safety. It can cause a serious injury or accident.

Synopsis

A De-ice Crew did not know an A320's APU was operating during de-ice at the gate so after the APU injected glycol the aircraft had to be evacuated because it filled with fumes and smoke which made some passengers ill.

ACN: 1066825 (2057 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Indicating and Warning - Air Conditioning and Press.
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1066825
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14200
Experience.Flight Crew.Last 90 Days : 117
Experience.Flight Crew.Type : 1800
ASRS Report Number.Accession Number : 1066848

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While descending to FL300 ATC issued a clearance to FL240. Nearing FL300, we noticed that the pressurization page had popped up on the systems display. The systems page showed the cabin altitude pulsating green and indicating a cabin altitude of 8,900 feet. The cabin vertical speed was showing a climb of about 500 fpm. There were no other system faults indicated. We decided to expedite our descent in case the situation deteriorated further. We asked for and received an immediate clearance to 12,000 feet initially and then 10,000 feet shortly thereafter from Center. As we descended, the cabin altitude continued to drift higher at 700-800 fpm. When the indicated cabin altitude reached 10,000 feet we put our oxygen masks on and established communications. This was followed shortly thereafter by the master warning and ECAM CAB PR EXCESS CAB ALT around FL250. We completed the ECAM and also did the Emergency Descent QRH. Since we already had our clearance to 10,000 feet and were quite busy at the time, we did not declare an emergency or squawk 7700. 

We still had no indications of any other systems anomalies or faults at this point. The pressurization page showed the outflow valve to be in a normal position and CPC SYS1 showed green. I checked the bleed system display page and everything indicated normal outputs on it. Nevertheless, the indicated cabin altitude continued to drift higher as we descended. Passing below FL200, I selected the pressurization control to manual and tried moving the outflow valve towards closed. The outflow valve was near the first tick mark on the displayed range of movement. As I moved the outflow valve towards the closed position, the cabin vertical speed spiked downward to a 2,000 fpm descent rate and the display turned amber. Also, the safety valve indication became amber and the safety valve opened. I then moved the outflow valve back towards its original position near the first tick mark and the safety valve closed. At this point, I selected the auto mode again which brought CPC SYS2 on line with a green indication. CPC SYS2 did not appear to react any differently than CPC SYS1 did. The indicated cabin altitude peaked at about 14,500 feet which caused the passenger oxygen masks in the cabin to automatically drop. Upon reaching 10,000 feet we were able to brief the flight attendants on our status as well as make a PA to the passengers. We were then able to continue with a normal approach and landing.

Narrative: 2

On our arrival, approximately 30 minutes from landing, after an uneventful flight, Center had cleared us to descend from FL300 to FL 240. The pressurization page appeared on the lower center screen soon after that, and the cabin altitude was flashing and green, indicating approximately 8,900 feet, and the cabin rate of climb was indicating a 500-600 fpm ROC, with no associated ECAM's, or fault light on the auto pressurization switch. We donned our O2 masks, established communications, and I began an expedited descent, after accessing that there was no apparent damage to the aircraft indicated, and asked the FO to inform center we had a pressurization problem, and to request a descent to 10,000 feet, which they approved. The cabin pressure continued this climb at a rate of 500-700 fpm. The FO switched the pressurization control to manual to arrest the climbing cabin altitude, and at one point saw the cabin overpressure safety valve symbol flash amber to indicate it opened but then quickly went away when he manually opened the outflow valve in response. This was confusing, since the cabin altitude was indicating a climb. (He then selected sys. 2; no fault lights illuminated, but went back to manual in order to evacuate the odor in the cabin produced from the hot O2 generators. We were never convinced that there was anything actually wrong with the automatic systems, but still had to respond to the indicated cabin altitude climbing uncontrollably). The cabin O2 masks deployed at approximately 14,500 feet cabin altitude with the associated ECAM for excessive cabin altitude. The FO accomplished the ECAM items, and reviewed the QRH for compliance. I called the Lead Flight Attendant, and gave her a brief explanation of what was happening. She indicated no issues in the cabin with the aircraft or passengers, besides the O2 mask deployment. At no time did anyone indicate they were experiencing any ear discomfort normally associated with such a depressurization, and the pressurization system indicated no problem other than the climbing cabin altitude. Neither of us recalls looking to see what the cabin pressure differential was indicating. Upon reaching 10,000 feet, we removed our O2 masks, and I made a PA to the cabin concerning our situation, and that the O2 masks were no longer needed, reassured them that the aircraft was safe, and that we were proceeding as planned, and our ETA. We had made the decision to not declare an emergency, and proceeded to destination for an uneventful VFR approach and landing. The actions of my FO as well as the cabin crew in response to this incident were very commendable. Everyone did an outstanding job.

Synopsis

A320 flight crew experiences an apparent pressurization anomaly after commencing descent and receives clearance for an expedited descent to 10,000 feet. The cabin pressure indication eventually rises to 14,500 feet where the passenger oxygen masks are automatically deployed. Attempts to control the cabin pressure manually are not successful with the safety valve opening at one point, indicating a cabin over pressure.

ACN: 1066790 (2058 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11200
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1066790

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 5357
ASRS Report Number.Accession Number : 1067078

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

While climbing out on the departure, BRAKE: SYS 2 FAULT appeared. As we continued to climb and began to comply with procedures, BRAKE: AUTO BRAKE FAULT appeared. The captain elected to remain in US air space and hold as we began discussing problem with Dispatch and Maintenance Control. The Captain kept flight attendants informed. Of particular interest was the status page which indicated the following 4 inoperative items: CAT 3 DUAL, Normal Brakes, Auto Brakes and Brake Sys 2. We were puzzled by Normal Brakes being inoperative given that only Brake Sys Fault 2 was on the upper ECAM. Maintenance Control could not decipher why we had that indication. If normal brakes were inoperative, shouldn't we also have a BRAKE SYS 1 FAULT? We elected to return, declare an emergency and operate under the assumption that we would decelerate with ALTERNATE brakes. We landed without incident using ALTERNATE brakes. I monitored and called out pressures from the triple brake indicator as the Captain decelerated. NWS worked so we taxied to the gate.

Narrative: 2

After printing paperwork for flight, reviewed maintenance history and Open Inbound Defect Reports. Received call from Dispatcher stating that he turned down aircraft for brake history. Waited in operations until notified that Maintenance replaced BSCU and cleared aircraft for departure. Completed normal preflight and pre-departure checks. On climb passing 7,000 feet BRK SYS 2 FAULT and BRK AUTO BRK Failure Ecams annunciated. Completed ECAM procedures and reviewed Status page indicating Inop Systems: CAT 3 DUAL, NORM BRK, AUTO BRK, BRK SYS 2. After brief discussion and review of maintenance history, contacted Dispatch and Maintenance Control through ACARS and voice. It was determined and agreed that continuing to destination with current aircraft status was not feasible. Requested delaying vectors and holding from Center. Maintenance Control stated that the ECAM/Status information did not make sense and with current fault information, could not determine the exact aircraft failures/condition. Declared an emergency, briefed the flight attendants on aircraft status and divert plans, and announced to passengers the plan for a return. After consultation with Dispatch and Maintenance Control, it was determined that the worst case scenario for landing would be alternate brakes, no anti skid, and no nose wheel steering. Landing performance and procedures for landing with alternate brakes and possible failure of nose wheel steering was reviewed and briefed. A normal ILS approach was flown by the Captain in visual conditions. At touchdown it was determined that braking was alternate yellow system braking with fluctuating pressure indicated on Brake Pressure Gauge. First Officer called pressures during rollout and remained below 500 psi during rollout. After determining nose wheel steering availability exited runway and stopped on taxiway for systems status check. Normal taxi and shutdown.

Synopsis

A320 flight crew accepts an aircraft after a BSCU change and during climb passing 7,000 feet BRK SYS 2 FAULT and BRK AUTO BRK Failure ECAM's annunciated. Status Page indicates NORM BRK, AUTO BRK, BRK SYS 2 have faults and the crew elects to return to the departure airport. After landing braking is via Alternate Brakes and the nose wheel steering is functional.

ACN: 1066161 (2059 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Emergency Exit
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1066161
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1066258
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Ground Personnel
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I was advised by the "gate keeper" after the fact that we had flown an Airbus aircraft from a Maintenance facility with the overwing exits not armed. I do not recall anything abnormal on that flight. There was no indication of an unarmed door. There was no ECAM alerts. It was a busy flight with last minute calls for catering and two jump seaters. We were concerned that the tug, without chains, would be able to push us on a solid sheet of ice. Planned, briefed, and executed De/Anti Ice. Since Maintenance had worked on the overwing exits, it would be nice to know that procedure had been done. Sort of like gear pin write ups. You don't rely on the writeup to check for gear pins, but you do double check that the gear pins have been removed! 

Maintenance did some work on the overwing exits but failed to re-arm the exits. As pilots we have learned to look primarily for ECAMs and pulsating amber indications. It is easy to overlook something that is just not where it usually is, that is part of human nature. Similar to a pilot doing a gear check and thinking he is seeing three green lights, when in fact he only sees two greens. He has seen and said, "Down and three green" for so many times he says, "Down and three green" again. We did not see anything to draw our attention to the lack of an armed slide, and we proceeded to fly the flight. 

I suggest three things, all are barrier builders: First, if an over wing exit is going to be disarmed and then rearmed by maintenance, then, before the fact, a log book entry is made, similar to Gear pins installed, that an overwing exit is being disarmed. Followed by a log book entry, that the overwing exit is confirmed rearmed. The second thing that can be done, if Maintenance does not want to do the above, Maintenance should confirm on the Doors Page that the overwing exit is showing armed after they worked on it. The third thing, suggest to Airbus that an amber indication shows on the doors page that an over wing exit is NOT ARMED when it should be. Now, we only get an amber indication if the over wing is actually opened.

Narrative: 2

[Narrative 2 had no additional information]

Synopsis

A320 flight crew reported departing on a ferry flight with the overwing emergency exits not armed by Maintenance personnel prior to flight.

ACN: 1066068 (2060 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1066068

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After liftoff I tried to raise the landing gear. The gear handle would not move and ECAM message L/G SHOCK ABSORBER FAULT was displayed. Other faults such as WINDSHEAR DET FAULT, ANTI-ICE FO FAULT, ATO FLT A/THR OFF, BRAKES AUTO BRK FAULT, BRAKES A/SKID N/WS FAULT were also displayed. 

We advised ATC, Company Maintenance, and Airport Operations of our intended return to our departure airport. We declared an emergency with ATC and were vectored while we accomplished the related abnormal procedures. We were able to get nose wheel steering and normal brakes back through ECAM follow-up using the QRH. We briefed the flight attendants for a precautionary landing and made a PA to the passengers to explain our return. Finally, we advised ATC we would be able to land with normal braking, clear the runway and taxi to the gate.

Synopsis

When the A320's landing gear lever failed to move after takeoff and numerous ECAM messages were presented the flight crew followed the appropriate QRH and ECAM procedures returning many of the lost functions, diverted back to their departure airport and landed without further incident.

ACN: 1066012 (2061 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : RSW.TRACON
State Reference : FL
Altitude.MSL.Single Value : 10000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : RSW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : SHFTY2
Airspace.Class E : RSW

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : RSW
Make Model Name : Citation II S2/Bravo (C550)
Operating Under FAR Part : Part 91
Flight Plan : IFR
Nav In Use.VOR / VORTAC : LBV
Flight Phase : Cruise
Airspace.Class E : RSW

Person : 1

Reference : 1
Location Of Person.Facility : RSW.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1066012
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Facility : RSW.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1066017

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Air Traffic Control

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working SOU RADAR and saw the EST Controller not take a hand off over LBV on a jet arrival from LBV Sector at 100 FT. He did not take the hand off due to an A320 aircraft also at 100 FT over MOEMO. This has been an on-going issue with no resolution. LBV Sector does not have an out for these aircraft other than APPREQ 5 miles from the boundary. Separation was not lost. We need to stop feeding two jets over the same fix at the same altitude from 2 sectors. This procedure is unsafe. We are currently trying to work this issue with ZMA. Although there was no loss of separation, this clearly shows that the procedure is flawed and requires coordination. We have repeatedly asked ZMA for a face-to-face meeting and have been denied. We strongly encourage corrective action on this item.

Narrative: 2

I was working the EST RADAR position. ZMA LAL (C67) handed off an A320 15 miles northwest of the LBV VORTAC descending to 100 FT on the SHFTY STAR. ZMA LBV (C47) was attempting to handoff a C560,[that] was level at 10,000 FT direct LBV VORTAC. I did not take the handoff on the C560 because I was not sure if the A320 and the C560 would have standard separation over LBV. ZMA LBV (C47) called with an APPREQ and asked if I wanted them to descend the C560 to 8,000 FT. I replied yes to ensure we would have separation over LBV VORTAC. While I believe that the C560 would have passed behind the A320, this procedure is not working and has not worked for quite some time. We have been trying to work on this issue with ZMA for quite some time with little to no progress and what appears to be little concern from ZMA. We are still trying to have a meeting with ZMA to fix this procedure; however, it doesn't appear as though they are able to make the time. The procedure, while it doesn't happen everyday, needs to be fixed.

Synopsis

RSW Controller expressed concern regarding the dual arrival procedures over the same fix from two different Center sectors.

ACN: 1065805 (2062 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 3
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance

Component : 1

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Emergency Exit
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Component : 4

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1065805
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1065806
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Conducted a routine safe flight from ZZZ to ZZZZ on. I was contacted by Gatekeeper Representative regarding this flight. We completed our duties/checklist in accordance with company policies and procedures outlined in the FOM and had no indication of a faulty or inoperative Over-Wing Slide. The aircraft was found to have had multiple faulty/inoperative Over-Wing Emergency slides, which was discovered by Company Maintenance personnel. Again, we had no cockpit indication of these faults. The flight was completed safely with no issues.

Narrative: 2

Prior to push back, all normal duties were performed and all checklists were completed. I, as Captain checked the DOOR page on ECAM to review all doors 'Closed and Armed.' I noticed forward and rear Cabin Doors, closed and armed, but the Over-Wing window [Emergency Exits] did not show 'Armed.' I questioned this reading and asked the First Officer (F/O) and IRO to verify and confirm the indication, which is a no indication of "SLIDE". I then called the flight attendants to confirm that the Over-Wing Exits looked normal, which they confirmed looked normal.

Prior to pushback, the DOOR page on ECAM was checked and indicated that the Forward and Aft cabin doors were 'Closed and Armed,' but the over-wing windows had no indication. I questioned the F/O and IRO about the indication then called the flight attendants to check the exits, which they confirmed looked fine. After a discussion about the indication, the three pilots mistakenly surmised that since this was a new airplane maybe this is the new indication. I briefly considered calling Maintenance, but noticed the loading bridge was parked, the agent was gone and we had gotten clearance to push back. In our minds, everything was normal so I dismissed the calling of Maintenance for assistance.

Even though we had been taught that the Over-Wing windows are always armed on the Airbus A320, the absence of both lights on the ECAM confused all three pilots and we mistakenly fell into a false sense of normalcy, believing this must be normal, since this is a new airplane. In all the years I have flown the airplane, we are taught that "the Over-Wing Exits are always armed" and seeing a "NO lights" indication on the ECAM did not sufficiently alert any of the three pilots on the flight deck.

Synopsis

Two pilots were informed they had flown their A320 aircraft over water with multiple Over-Wing Emergency Exit slides disarmed. Pilots noted there were no slide fault indications on their DOOR ECAM page.

ACN: 1065803 (2063 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 3
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Emergency Exit
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1065803
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1065808
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Conducted a routine safe flight from ZZZZ to ZZZ. I was contacted by Gatekeeper Representative Office regarding this flight. We completed our duties/checklist in accordance with company policies and procedures outlined in the FOM and had no indication of a faulty or inoperative Over-Wing slide. The aircraft was found to have had multiple faulty/inoperative Over-Wing Emergency slides, which was discovered by Company Maintenance personnel. Again, we had no cockpit indication of these faults. The flight was completed safely with no issues.

Callback: 1

Reporter stated the A320 aircraft had been in the hangar the night before and Maintenance had left the Over-Wing Emergency Exit Door release pin in the 'Safe' mode. No deferred items were involved with the Over-Wing Exits. The Captain was returning from reserve status. Airbus had not changed any of the door indications on the newer A320 they had flown. He and the other two crew members were just not alert to the missing word SLIDE on the ECAM page.

Narrative: 2

On the previous flight, ZZZ-ZZZZ, an issue with no indication on the ECAM of the Over-Wing Window Exits being armed was discussed and verified by the three pilots in the flight deck and checked and verified by the flight attendants, prior to pushback. All physical checks looked normal and we all mistakenly misinterpreted the indications of the Exits being armed. The rationale being that this was a new airplane and this must be the correct indication. After a discussion we all felt that all was normal so we proceeded to ZZZZ, then returned to ZZZ the same afternoon, never even considering that there was, in fact, a problem with the Over-Wing Exits.

Prior to push back in ZZZZ, all checks and checklists were performed and the Door Page on the ECAM was again checked to verify all doors were 'Closed and Armed.' Since we were taught that the over-wing exits were "always armed," the three pilots in the flight deck mistakenly thought a no indication must be the new 'normal' for this new airplane. We all falsely believed that all was normal and I, as Captain, never even considered calling Maintenance for assistance. 

I am alarmed to find out that I indeed, unintentionally, flew an airplane with an issue with the Over-Wing Exits. We are taught that "the over-wing exits are always armed," but all three pilots on the flight deck misinterpreted the indication on the ECAM, which is, no indication. The no indication, or blank space where there should have been [the word] "SLIDE", did not sufficiently arouse our attention. Complacency.

Synopsis

Two pilots were informed by a Gatekeeper Representative that the A320 aircraft they had flown two trips over water on, had multiple inoperative Over-Wing Emergency slides and no slide fault indications on their DOOR ECAM page.

ACN: 1065774 (2064 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 28000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Stabilizer Attachment
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 10
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1065774

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 19300
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 3300
ASRS Report Number.Accession Number : 1065780

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departure was uneventful until passing approximately 28,000. Airspeed during the climb was 320 KIAS due to overtaking traffic. Moderate airframe vibration of about 90-120 hertz was felt throughout the aircraft. Autopilot and auto-throttles were manually disconnected during the event. Decreasing both engine power and airspeed had little effect on the severity of the vibration. (All engine parameters were normal during the event and flight.) The event ceased about 45 seconds after it began. The flight attendants called the cockpit shortly after the event to report same. We leveled at FL330 and continued to troubleshoot the event. ECAM reports viewed via the FMGC maintenance reports, showed malfunctions on the THS and the right blue elevator servo. Dispatch was contacted through ACARS and then a phone patch. Maintenance Control input was also requested. The connection to Maintenance Control was never established. The First Officer and I decided with the unknown vibration and the flight control abnormality a prudent course of action would be a return to the nearest airport. (A divert airport was chosen due to known good weather [VMC and light winds], long runway configuration, maintainer support and passenger support.) We informed the flight attendants of the divert and declared an emergency with ATC and received expedited vectors to the airport. At 10,000 FT and 25 NM from the airport we configured and performed a modified controllability check. Landing was uneventful. CFR inspected the aircraft prior to taxi to the gate. At arrival aircraft Maintenance was debriefed on the event. Areas of concern: While the connection to Dispatch went well, the follow on connection to Maintenance Control was unsuccessful. As such we, the flight crew, were on our own in making the decision to divert. This lack of communication with Maintenance Control needs to be corrected as soon as possible.

Narrative: 2

While climbing out, we were told to keep our speed up. We were climbing out of 25,000 FT at 320 KTS when the aircraft started a very strong, fast rhythm vibration, shaking the entire airplane and lasting for about 20-30 seconds. It was unlike anything I had ever encountered. We immediately slowed and after a few seconds the vibration stopped. We had no ECAM's and all engine parameters were normal. We initially thought high speed buffet, or gear door, but it was a much different and stronger vibration than that. The flight attendants immediately called with concern and we told them we felt it too and were working on the problem. The Captain said he encountered something like this before and it was elevator buffet. The Captain then accessed the ECAM reports via the FMGC maintenance reports. He noted there were two elevator related faults. The Captain informed Dispatch that we had a flight control problem with two recorded errors and that without further information, we were considering returning to the nearest airport. We were unable to connect with Maintenance Control for further evaluation. With an unknown problem with a primary flight control, we elected to declare an emergency and return to the airport.

Synopsis

A320 flight crew climbing out of FL250 at 320 KTS experiences a very strong, fast rhythmic vibration, shaking the entire airplane and lasting for about 20-30 seconds. The vibration stops after the aircraft is slowed and the crew elects to divert to a suitable airport.

ACN: 1065268 (2065 of 2908)

Time / Day

Date : 201302
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZMA.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 19000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZMA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZMA

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5005
ASRS Report Number.Accession Number : 1065268

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In open descent, the aircraft pitched over too far resulting in rapid build up of airspeed. About a 5 knot overspeed with bells and alarms. The First Officer recognized the problem but turned off the autopilot too late to prevent the overspeed.

Synopsis

An A320 in Open Descent mode pitched over too aggressively and an approximate five knot overspeed resulted.

ACN: 1064670 (2066 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1064670
Human Factors : Workload
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On descent into our destination, cabin notified flight crew of smell/odor in the cabin. Flight crew did not notice at the time any odor. Captain asked the cabin crew for specific details concerning the odor so he could notify Maintenance on arrival. Captain notified Maintenance Control (MOC) through early alert, and we landed without incident. On arrival at the gate, we did notice an odor and the Captain then proceeded to write it up. I then left the aircraft as the Captain was finishing the paperwork, and told the oncoming First Officer about the incident. While waiting for the van and Captain curbside, I was called from Dispatch (OCC) to ask the Captain to call MOC, and told them he wasn't there yet, but I would. I notified him to call MOC, he said he would. Then during the van ride I called Crew Scheduling to see about a later DH, and once again I was asked to have the Captain call MOC. I told the individual that we were in the van with civilians and it might be better to talk to the Captain at the hotel instead of in front of the flying public. I then passed on the second request to the Captain and he said he would call them at the hotel.

Synopsis

A320 flight attendants reported an odor during descent for landing which the flight crew also detected after parking at the gate, resulting in a maintenance log entry.

ACN: 1064535 (2067 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1064535
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff at approximately 1,000 FT, flight deck fumes of unknown origin. Flight attendants reported fumes in cabin as well. Declared emergency and returned to the departure airport. Taxied to gate and off loaded all passengers. Flight crew and flight attendants, all reported eye irritation, sore throats, [and] headaches. Flight crew used oxygen 100% per Smoke/Fume QRH procedure. QRH procedure accomplished and emergency landing at the departure airport.

Synopsis

An A320 crew detected fumes of unknown origin after takeoff, so the pilots donned oxygen masks, declared an emergency, completed the QRH and returned to the departure airport where the crew reported sore throats and burning eyes.

ACN: 1063009 (2068 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZNY.ARTCC
State Reference : NY
Altitude.MSL.Single Value : 35000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZNY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZNY

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZNY
Aircraft Operator : Fractional
Make Model Name : Falcon 900
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZNY

Person

Reference : 1
Location Of Person.Facility : ZNY.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1063009
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

A A320 level FL350 southbound to BOVIC L461 non-RADAR estimated BOVIC [within 2 minutes]. Falcon 900 Bermuda departure RADAR direct BOVIC climbing to FL360. Took over the sector with this situation in place. The Falcon 900 did not reach FL360 15 minutes prior to passing the A320. Based on estimates the aircraft passed. Did not realize this situation until after loss of separation and the A320 had been RADAR identified. Separation rules that pertain to the airspace in question and updated to 7110.65. Present rules were recently instituted by New York Center Oceanic and Airspace Procedures Office which require more separation between a RADAR and non-RADAR aircraft than is needed between 2 non-RADAR aircraft. This is counter intuitive as the separation should be less.

Synopsis

ZNY Controller described a loss of separation event, noting current facility separation requirement exceed those listed in national FAA Orders.

ACN: 1060779 (2069 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : N90.TRACON
State Reference : NY
Altitude.MSL.Single Value : 2500

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Facility : N90.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1060779

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working combined position on the late night shift. The weather at the airport was low visibility with visibility down to 1/4 of a mile. During an arrival push my RVR equipment failed for an unknown reason. Due to the unavailability of other runways, I was forced to switch to an approach which is an uncommon approach for our sector. Over a year ago construction had changed the MVA in the area of the approach but I was not sure if the MVA map was updated. I could not remember what the new MVA was or if the map I was looking at was updated so I used the map which I thought was the most up to date and descended the arrival aircraft to 2,500 FT MSL. I inquired about the new MVA and was informed it had changed to 2,800 FT MSL. I immediately corrected it for all of the other aircraft I was working and no other planes were descended below the MVA. The map needs to be changed immediately or some other map change needs to be developed to advise other controllers of the new MVA.

Synopsis

N90 Controller described a below MVA event when he/she failed to note a change in the MVA requirements and issued an assigned altitude below the altitude limits.

ACN: 1060226 (2070 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 20000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZDV

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDV
Make Model Name : Citationjet (C525/C526) - CJ I / II / III / IV
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZDV

Person

Reference : 1
Location Of Person.Facility : ZDV.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1060226
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was working Air Carrier X into Denver Approach Control and he had been issued a descend via clearance on the Kipper STAR I believe. The descend via clearance had Air Carrier X crossing Landr in the block FL200-220 and continuing his descent as per the STAR after Landr. Approach then called us up and APREQed Citation Jet entering our airspace from approach at FL210, which was landing at SNY an airport inside of my airspace. My D-Side then "unabled" them because of the traffic and said that FL190 was approved and that we would have Air Carrier X cross Landr at and maintain FL200 at 250 KTS. Everything appeared to be completed. Air Carrier X was shipped to approach and I think that Citation Jet was on my frequency. Both aircraft were inside of approaches airspace. Next thing I know the two aircraft are flashing and Air Carrier X was losing altitude and is directly above Citation Jet. I noticed about a 500 feet vertical separation at its worst point. Not sure what Approach was thinking! This is a rare situation but this is going to be a problem with the new RNAV STAR's and there needs to be some kind of procedure in place. Either Approach needs to take this aircraft out of the departure gate next time or they need to hand the SNY arrival off to Denver Center Sector 15 at or below the altitude restriction for the first fix inside of Approaches airspace.

Synopsis

ZDV Controller described a conflict involving traffic handed off to Denver TRACON and traffic entering his/her sector, noting the separation instructions were forwarded but apparently not followed.

ACN: 1060088 (2071 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 20
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 1060088
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We got to the airport about 55 minutes before departure. Flight planning and release went normally. I went up to the jet, briefed the flight attendants, and then went into the flight deck. When I sat down, I commented to my First Officer that the window looked opaque. Forward visibility was severely restricted. The sun was shining straight through the window and it appeared that there was a film on both forward flight deck windows. I asked the First Officer to to call for window cleaning, which he did. 

About 10 minutes later a Mechanic showed up to clean the window. Using glass cleaner and a rag made no difference on the transparency of the window. He said that was the best he could do at that point. I was somewhat sure that visibility would be most impaired with sunlight going straight through the window. Since we would be flying away from the sun I thought that we might be able to take the aircraft if the defect could be deferred. I then called Maintenance Control to see if there was a way to write up the window as having the visibility affected but still deferable at the Captain's discretion. They replied that [when I wrote it up] they would have to deal with it then. It could not be deferred. I ended the call and entered an AML discrepancy "Visibility through the forward cockpit windows impaired due to hazy coating, cleaning window no help" and advised operations we had done so.

Mechanics then returned and tried different methods to clean the window. They tried soap, hot water, alcohol, acetone, wiping with paper towels and even newspaper but the haze on the window was still evident. Even the mechanics commented on how hazy the window was from inside the cockpit. 

While this was happening, I was keeping the flight attendants, passengers, and Customer Service informed as to what was going on. At one point early in the event, I overheard the passenger agent comment to her Supervisor "Well, I called Flight Operations Management and his Chief Pilot, maybe they can get him to take it." A few minutes later an Assistant Chief Pilot [whom I knew personally] called me on the phone. 

While he didn't push me to take the aircraft, he did say that I should get names of all involved and take pictures of the defect, implying that I would have to defend myself in the future. I have a working relationship with this manager and I personally did not feel pushed by this exchange. However, if I were not familiar with him, the nature of this conversation could most definitely have felt like coercion to take an aircraft that I felt was not airworthy. 

After I got off the phone the Passenger Service Supervisor came up to me and asked if I was going to take the aircraft now. I said that the defect had to be taken care of before we could go. She said that I was doing a real disservice to the customers. I disagreed. 

While this was going on the mechanics were still trying to clear the window. It became evident the film I initially thought was there was actually an abrasion to the windows' exterior surfaces. There was also evidence of this abrasion on the leading edge of both forward side windows. The mechanics exhausted their attempt to clean the window. I asked the mechanics if they could polish the window and they said they were not allowed to do that anymore. One of them said that they would have to replace the windows. He also said that this would be impossible as they had no Airbus windows in stock. This is funny because the aircraft was out flying a day later with new windows. 

At this point the mechanics left and said they were going to conference with Maintenance Control. Several minutes later a new maintenance release came out of the ACARS printer with the log item cleared, stating "ACTION:CLEANED PER AMM #." This was obviously not right as the abrasion still existed on the window. 

Dispatch sent us a message at about the same time asking if the ["fix" was satisfactory to which I replied "negative"] and subsequently refused the aircraft for the uncorrected window defect. The aircraft was removed from service for the window change and we were assigned a new aircraft several hours later.

Synopsis

The Captain of an A320 was forced to turn down the aircraft for his flight due to the abraded condition of both forward windshields which rendered forward visibility unacceptable.

ACN: 1059714 (2072 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : L30.TRACON
State Reference : NV
Altitude.MSL.Single Value : 8500

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Dusk
Ceiling.Single Value : 18000

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : SUNST3
Airspace.Class B : LAS

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Design
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1059714
Human Factors : Workload
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1059715

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This event occurred during descent on the SUNST RNAV arrival to LAS. I was pilot flying and the Captain the pilot not flying. We had been cleared and descended to cross FUZZY at 16,000 FT and 250 KTS when I noticed, several miles outside of IPUMY, that we hadn't heard from ATC in a while and were rapidly becoming too high to comply with the 11,000 FT and 230 KTS crossing at IPUMY (we still had not yet been given the 'descend via' clearance). 

When we received no response to our inquiry to ATC we went back to the last assigned frequency and were told to switch to another frequency. When the Captain checked in and advised of our altitude we were cleared to descend to 10,000 FT and to delete the restriction at IPUMY. I selected open descent but, before we leveled off at 10,000 FT, ATC cleared us to "descend via" the SUNST RNAV arrival. I selected 8,000 FT in the altitude but forgot to push "altitude" for a managed descent. 

I announced the next crossing, which was NIPZO at 9,000 FT, but neither of us realized we were still in Open descent. Shortly after passing 9,000 FT we received traffic advisory and both of us focused on this. The traffic was 2,500 FT below us, but this also contributed to our not noticing the altitude deviation. We realized we had gone approximately 500 FT below the NIPZO crossing when ATC advised us. I deselected the autopilot and climbed back up to 9,000 FT and no other disruptions occurred. 

Several factors led to this event: the confusion and distraction from the lost communications, receiving a non-standard/out-of-the-ordinary clearance to descend to 10,000 FT rather than to "descend via the SUNST RNAV arrival," and being distracted with a traffic advisory, all which took place during a high workload environment.

To prevent this from happening again, I will be more alert and prepared to confirm automation when a change in descent clearance is given other than what we are used to receive in a multiple crossing RNAV arrival. I would also be quicker to query ATC when I notice approaching our next descent point rather than waiting to query after well crossing it and being further behind and being put in a catch up mode.

Narrative: 2

[No substantive additional information was provided by the secondary narrative.]

Synopsis

A temporary loss of communication with ATC and non standard clearances while flying the SUNST RNAV STAR to LAS resulted in two deviations from charted crossing restrictions for the reporting A320 flight crew.

ACN: 1059497 (2073 of 2908)

Time / Day

Date : 201301
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : EAGUL FIVE
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1059497
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Approximately 20 minutes prior to top of descent, as the pilot monitoring, I received the most current ATIS information for Phoenix Sky Harbor International Airport via ACARS which indicated that runways 07 and 08 were in use for arriving aircraft and verified that the FMGC and ND displayed the correct information for the EAGUL FIVE RNAV ARRIVAL RUNWAY 08 TRANSITION, in accordance with the "Arrival Verification and Briefing" procedure in the A320 Pilot Handbook Bulletin. When cleared by Albuquerque Center to "Descend via the EAGUL FIVE RNAV arrival, Runway 08 transition, Phoenix Sky Harbor altimeter three zero two six," I verified that the pilot flying had set 7,000 in the FCU after which I received an arrival briefing. I monitored the aircraft's compliance with the arrival routing, altitude constraints and airspeed restrictions while acknowledging comments from the pilot flying. I had preset each anticipated radio frequency and responded promptly to each radio call addressed to us; however, my attention was diverted from monitoring tasks within the flight deck due to traffic vigilance. 

Unbeknownst to me, perhaps between EAGUL and HOMRR, the FMGC had been modified from Runway 08 to Runway 26, perhaps while I was communicating with Albuquerque Center and Phoenix Approach Control during the handoff. The first indication that we were off course occurred when the approach controller asked if we were on the Runway 08 transition. I acknowledged in the affirmative, however, when I saw that Runway 26 was, in fact, displayed in the FMGC and determined that we had passed the HOMRR Intersection, I endeavored to communicate to the Controller that we were now aware of the error. The aircraft was descending through 8,800 FT in the managed mode as verified by the FMA. Immediately prior to keying the side stick "push-to-talk" comm switch in order to convey my urgent message, the Controller instructed us to turn right to a heading of two one zero and maintain nine thousand. I acknowledged the clearance while the PF set the heading on the FCU to 210, however, when the pilot flying did not also promptly reset the FCU altitude from 7,000 to 9,000, I immediately alerted the pilot flying. When the pilot flying did not respond to my second alert, I ensured that the FCU altitude hold selector had commanded the aircraft to maintain the present altitude and advised the pilot flying that we must immediately climb to 9,000. I then advised the Controller that we could proceed directly to GEENO and resume the arrival. He immediately and very politely responded by clearing us to "proceed direct GEENO, maintain 9,000 and resume the EAGUL FIVE arrival." I thanked him and reminded myself to remain "in the green." I informed the pilot flying to reset the FCU altitude to 7,000 and, now fully aware of the adrenaline rushing through my circulatory system, easily maintained an elevated level of situational awareness.

I believe the cause of the event to have been poor CRM.

Synopsis

An A320 flight crew descending on the EAGUL RNAV STAR to PHX discovered the wrong runway transition inside of HOMRR was programmed in their active route.

ACN: 1058545 (2074 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1058545
Human Factors : Confusion
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Adjustment locking mechanism failed on First Officer's rudder pedals. Waiting on the runway for takeoff clearance, Captain set the parking brake and transferred control of aircraft. Takeoff clearance was given, parking brake released, power set to 50 percent N1. As the aircraft started to roll and needed ground directional control, both First Officer rudder pedals pushed all the way to their full travel together with no directional or braking control. Checked that the adjustment handle was in the stowed/locked position. Pedals still failed. Transferred aircraft control to Captain, he rejected the takeoff, slowed the aircraft to exit at the first taxiway. Speed at occurrence was approximately 30 knots. Stopped on the taxiway, confirmed that the locking mechanism had failed and returned to gate. 

Mechanical failure of the First Officer's rudder pedals. Other than a personal setting adjustment at the gate, there is no procedure to check First Officer's rudder pedals for proper movement and use. Failure was not found until actual use for ground directional/flight is required. I would suggest adding a procedure to check First Officer rudder pedals for proper movement and use.

Synopsis

An A320 series First Officer rudder pedals traveled full forward together with the handle in the stowed/locked position so the Captain rejected the takeoff and returned to the gate.

ACN: 1058057 (2075 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Pressurization Control System
Aircraft Reference : X

Component : 2

Aircraft Component : Aircraft Auto Temperature System
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Lead Technician : 2
Experience.Maintenance.Technician : 15
ASRS Report Number.Accession Number : 1058057
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

Before the aircraft arrived, Maintenance Control advised us that Aircraft X will arrive with pressurization issues. I was the Lead [Mechanic] in charge during the overnight shift. I assigned two technicians to remove the controller from Aircraft Y, also an A320, which was Aircraft Out of Service (AOS) in ZZZ in preparation for the arrival of Aircraft X. The Aircraft Maintenance Manual (AMM) reference [Chapter] (21-31-34) for the Cabin Pressure Controller (CPC) was printed from Airnav [Airbus Manual]. The CPC and Pack Temperature (Temp) Controller (PTC) were located next to each other in the Forward Electronic Compartment. The placard identifier next to each controller had the Number Two [#2] written and it was taken for granted that the part was the correct part that was removed without realizing that it was the incorrect part. I told the Technician to remove the CPC and the PTC was removed instead, without realizing. The Technicians may have misunderstood exactly what part was to be removed. I went to the flight deck and debriefed the flight crew and told the technicians that I will pull the Circuit Breakers (C/B) and let them know when it would be safe to replace the controller on Aircraft X. They installed the part and I proceeded to perform the Operational (Ops) Check per the AMM. The Ops Check passed and no CTC faults were present. Since the controller had an intermittent fault, it was not currently faulting. I also ran both engines per Maintenance Control's request to verify that no faults returned after [electrical] power swap. All checked OK. I verified the effectivity of the CPC before Aircraft X arrived and it was effective to both Aircraft X and Y. However, the incorrect part was removed from Aircraft Y, that was located next to the CPC and that part was not effective for Aircraft X. I was made aware of the configuration issue after aircraft departed.

Callback: 1

Reporter stated that Aircraft X did not actually have a cabin pressurization issue, but a fault issue that appeared after engine starts at the previous departure station. Even though the Cabin Pressure Controller (CPC) was not removed and replaced, the original fault did not recur after he performed the engine runs. He decided to sign-off the discrepancy because there is only one sign-off block in the logbook and multiple technicians had been working the aircraft. The aircraft later diverted to another airport for continuing cabin pressurization faults. That's when their downline Maintenance technicians noticed the PTC was mistakenly replaced with a similar PTC part that was not effective for that A320. They also realized the CPC was never actually replaced. The PTC and CPC are physically different in size, but located next to each other with 'Control' labeled on the electronics rack for both controllers.

Synopsis

A Line Lead Mechanic explains how a Pack Temperature Controller (PTC) was removed and replaced (R/R) instead of a Cabin Pressure Controller (CPC) for cabin faults appearing after engine starts on an A320 aircraft. Aircraft later diverted after incorrect part replaced.

ACN: 1057978 (2076 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Main Gear
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1057978
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

Employee Mechanic Y and I were conducting maintenance on the right-hand main landing gear of an A320 aircraft while the main landing gear was mounted on the Omni-arm. We were articulating the landing gear to a near horizontal position to facilitate seal replacement. While rotating it to a horizontal position the Omni-Arm suddenly released from its rotating position and suddenly dropped without command 10"-12" inches. Employees Mechanic Y, Z and Supervisor observed this mishap event.

Synopsis

A Line Mechanic reports about an A320 right Main Landing Gear (MLG) suddenly releasing and dropping 10" to 12" inches, uncommanded, from an Omni Arm landing gear support equipment. They were rotating the MLG to a horizontal position using the Omni Arm device to facilitate seal replacement.

ACN: 1057977 (2077 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Aircraft Reference : X

Component : 2

Aircraft Component : Electrical/Electronic Panel & Parts
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1057977
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

A borrowed part from Aircraft X, an A320 aircraft, [with] Part Number (P/N) -E01 for Base Contactor and a Three-Pole Contactor P/N -Y1 was installed on Aircraft Y, also an A320 that had Base Contactor P/N -00 and Three Pole Contactor P/N -Y00 removed. The installed parts have a different aircraft 'Effectivity'. The Base Contactor installed on Aircraft Y, from Aircraft X, did not have an index pin [but was able to be] installed on Aircraft Y with no problem. 

The [Base Contactor] P/N -Y00 removed from Aircraft Y, has an index pin and is not installable on Aircraft X. The Illustrated Parts Catalog (IPC) 29-24-08 Item # 105, shows that the [Base Contactor] P/N -Y00 may be used as a spare part on Aircraft X, but [Base Contactor] P/N -E01 does not show that the part is a spare on Aircraft Y. Both aircraft have the same [Airbus] Functional Item Number (FIN) number XXX-5GX for the parts. When trying to install P/N -Y00 Base Contactor on Aircraft X, the part was not installable because an index hole is not present in the mounting box 107vu126 [panel] for the Yellow Hydraulic Power Contactor. [Non] ETOPS.

Synopsis

An Aircraft Maintenance Technician (AMT) reports about the different aircraft 'Effectivity' of two Base Contactors used for the Yellow Hydraulic System Power contactor on A320 aircraft. The Illustrated Parts Catalog (IPC) did not identify that one contactor had an index pin and required an index hole in order to be installed.

ACN: 1057624 (2078 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Night
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1057624
Human Factors : Fatigue
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Ambiguous

Narrative: 1

We were on the TYSSN3 arrival into LAS, ATC kept giving us directs, taking us off the arrival. They asked if we had the field in sight and we confirmed that we did have it in sight and they cleared us direct SHAND and the visual 25L. The First Officer asked for 5,000 FT, the published altitude for SHAND. He continued his gradual descent to 5,000 FT and maintained 250 KTS, intending to slow down at 5,000 FT to help expedite our late arrival into LAS. It was a clear night with a half moon, I believe. 

At no point did I consider terrain separation an issue. However, on our dog leg to final, I noticed after we landed that on the [Commercial chart] page it does show a rising terrain with a top elevation of 3,738 FT just south of LARRE. I believe this is what we flew over and my guess is that the rapidly rising terrain is what caused the EGPWS to activate the "Too Low, Terrain" call. He disconnected the autopilot and autothrottles and climbed to approximately 5,600 FT, again with everything in sight (airport, city lights, terrain). I do believe the long day we had, being 2 hours late and it being after XA00am also played a factor.

Synopsis

A320 Captain reported receiving a EGPWS Terrain alert on night visual approach into LAS while being vectored off arrival routing. Fatigue was mentioned as a factor.

ACN: 1056929 (2079 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8500
Experience.Flight Crew.Last 90 Days : 125
Experience.Flight Crew.Type : 4333
ASRS Report Number.Accession Number : 1056929

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 18500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 12000
ASRS Report Number.Accession Number : 1056916

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise during a normal systems check the Captain noted a decreasing Green Hydraulic System fluid level. Shortly after we became aware of the decrease the situation resulted in a HYD Green System Reservoir Low Quantity ECAM. We completed the ECAM steps, referenced the Flight Manual, conferred with Dispatch and Maintenance Control and evaluated our landing performance. With their inputs included we determined the appropriate course of action was to continue to destination. Flight attendants and passengers were advised. We used the gravity gear extension procedure as per the Flight Manual and executed a normal landing. We taxied clear of runway and were towed to the gate.

Narrative: 2

In cruise flight, Green Hydraulic System began to lose fluid. We first noticed this during routine periodic system review and got to watch it drain before our eyes. Fluid depleted and ECAM triggered. ECAM procedure was followed, Flight Handbook procedure was reviewed, Dispatch and Maintenance Control were conferenced, and we discussed our options, deciding that proceeding to destination was the best available choice. Remainder of flight was uneventful and routine. Flight attendants were briefed, ATC and destination airport were advised, and passengers were informed. Approach and landing were normal, with consideration of unique requirements of the procedure. We landed, taxied clear of the runway, shut down the engines, and were towed to the gate by company Maintenance personnel. Flight concluded without any further occurrences.

Synopsis

A320 flight crew reports loss of Green Hydraulic System pressure at FL340 due to loss of Green System fluid. Crew elects to continue to destination and lands normally after using alternate means of landing gear extension.

ACN: 1056878 (2080 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Brasilia EMB-120 All Series
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Coordinator
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1056878
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1056878

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Air Carrier X was sent around on a 120 heading due to Air Carrier Y slow roll still in the intersection with Air Carrier X very short final. I immediately coordinated with Approach Controller of the go around and was told to give the Air Carrier Y a 090 Heading due to the go-around. Told the Local Controller to issue Air Carrier Y a 090 heading but the Controller said they had already shipped the pilot to Approach. I immediately went back to Approach and they said they had him and had issued the 090 heading. During this time the Supervisor was shouting instructions to the Local Controller and to my amazement the Air Carrier X was now on a 070 heading back toward Air Carrier Y. I then coordinated this to Approach. In my opinion the Supervisor should not have been shouting instructions to Local as he did not know what was being coordinated. Had the Supervisor stayed out of it, all would have been fine. To my understanding he had told Local to "give him a 070 heading" without specifying a call sign. His intervention without knowing the details caused a loss of separation in my opinion.

Narrative: 2

Cleared Air Carrier Y for take off from Runway 10R with Air Carrier X on a 4 mile final for Runway 19L. Told him no delay. The aircraft was slow to roll. So I sent Air Carrier X around on short final. Told him to fly heading 120. Local Assist called Approach, and they told us to have Air Carrier Y fly heading 090. He was already switched to Approach, so Local Assist went back and told Approach. The Approach Controller was putting Air Carrier Y on heading 090. I then heard my Supervisor tell me to put "him"on a 070 heading. The only aircraft I was talking to was Air Carrier X. Air Carrier Y had been given heading 090 by Approach, so I assumed, wrongly, that he meant Air Carrier X. The way that Local Assist coordinated was, Air Carrier X heading 120 Air Carrier Y heading 090 separated both aircraft. Having me put Air Carrier X on a 070 was not a good heading at all. My Supervisor meant Air Carrier Y, but he did not specify and I was no longer talking to that aircraft. I should have questioned heading 070 before issuing it. It is difficult when multiple people are giving you different headings and different instructions. In weather like this I should have been more conservative. Even though there was more than enough spacing, the take off roll should have been cancelled earlier. The Air Carrier Y is normally a fast roller. Question before issuing headings, even if it is from a Supervisor. Do not run tight traffic in SE with bad weather. Slow down the operation, with bad visibility.

Synopsis

Tower Controller described a loss of separation event during a go around, the ATC instructions confused when a Supervisor countermanded previously coordinated headings between the Tower and TRACON.

ACN: 1056623 (2081 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 1056623
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1056631

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure, a G HYD LO PR ECAM annunciated. That was followed by G HYD RSV LO. At the time we were operating in a high workload environment; on vectors and communicating with Departure about an altitude change. When the ECAM annunciated, we requested a level-off and vectors while we worked the problem. The Captain declared an emergency, and we prepared the aircraft for a return. We completed the checklists and requested vectors for a visual approach. The landing was uneventful. The Captain stopped on the runway. Airport fire services were on hand to inspect the exterior. Maintenance towed us in to the gate.

Narrative: 2

Passing 10,000 feet the Green Low Pressure and Green Reservoir ECAMS activated. Leveled off at 11,000 feet and asked for spacing vectors. After determining the Green System was lost and ECAM actions were complete, we called Dispatch and Maintenance to review the problem. The crew elected to return and Maintenance and Dispatch concurred. An emergency was declared and ATC was advised of the situation and the need to stop on the runway for a tow-in due to loss of nose wheel steering. Landing was uneventful and the aircraft was brought to a stop on the centerline. Fire trucks reviewed the aircraft and advised that hydraulic fluid was leaking onto the runway. The tug was connected and the aircraft towed to gate.

Synopsis

A320 flight crew experiences a Green Hydraulic system low pressure followed by a reservoir low level ECAM climbing out of 10,000 feet. An emergency is declared and the flight returns to the departure airport.

ACN: 1056212 (2082 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : PHL.TRACON
State Reference : PA
Altitude.MSL.Single Value : 1500

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : PHL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : PHL

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : PHL
Aircraft Operator : Air Carrier
Make Model Name : A330
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : PHL

Person

Reference : 1
Location Of Person.Facility : PHL.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1056212
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Was windy on final approach and had good visibility. The first aircraft (an A320 or similar) was given a visual approach. While the aircraft could have turned toward the field, the pilot turned back up to join the localizer on his own. This created about a loss of about a mile, but not a loss of separation with the heavy aircraft that was about 4 behind and higher which turned into 3. I did not expect this so I maintained 1,000 feet and turned the trailing aircraft northbound to gain additional mileage, looking for around 4 miles on final. Additional traffic was to the north eastbound at 4,000 feet, but the path of the trailing aircraft I turned was behind that traffic so not a factor. When an additional mile was gained, I cleared the heavy for a visual approach. The company traffic was well ahead and there was not a separation problem but a slight over take on the speed due to an increase in compression caused by strong headwinds. I asked the trailing aircraft to advise when he saw the traffic ahead so he could maintain visual separation. He advised he had traffic in sight and instructed him to "maintain visual separation" with traffic ahead. It took numerous attempts to get a correct read back from the pilot. He said he had responded three times but had only responded with his call sign, nothing else. I advised him that I needed more than just a call sign as a response. I could have called the tower to have them maintain visual separation if able with the traffic instead of continually trying to get a proper read back. Many times the pilot will say they have the airport in sight on their own accord and I will clear them for the visual approach. Instead of this, I believe I will run more ILS approaches even with good VFR weather to control the aircraft.

Synopsis

PHL Controller reported a possible loss of separation when traffic on final failed to respond appropriately to a visual approach clearance.

ACN: 1056108 (2083 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : KOHOE ONE RNAV
Airspace.Class A : ZDV

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1056108
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

This report is submitted to document the gross errors of the automated FMS in the managed descent mode of the KOHOE ONE RNAV STAR into Denver. All altitude/speed constraints were verified to be correct in the FMS prior to the Descend Via clearance, and all constraints were met by the PF using selected modes because we noticed ahead of time that the FMS was in error. 

We were at FL360 (ALT CRZ) and were given Descend Via the KOHOE ONE. The pilot flying put 9,000 FT in the Altitude Selector. We also increased the Cost Index to 35 to make sure the descent speed was at least 280 KTS. At this time, the FMS showed a proper Top of Descent (TOD) to meet HMRHD at FL340 and the FMS indicated this. However, the FMS also showed HERNG with a planned altitude of FL340 (with an orange asterisk). KOHOE, SAYGE, and MOKME all also had incorrect high planned altitudes with orange asterisks.

We began a managed descent (DES) at the TOD and it went into ALT CST mode at HMRHD at FL340. When this happened, a new TOD appeared but it was past HERNG (which was supposed to be at or below FL320). At this point the PF selected V/S [Vertical Speed mode] to meet the restriction at HERNG. When we crossed HERNG at FL320, the PROG page showed us 2,000 FT below the managed/planned glide path. Throughout the arrival we continued to meet the constraints in V/S mode even though the managed/planned glide path and altitudes still indicated high (with orange asterisks). 

We remained in managed speed at 280 KTS, until approximately 2 NM prior to KOHOE when the FMS speed changed to 250 KTS, so the pilot flying went into selected speed at 280 KTS (per the arrival). Nearly every constraint on this arrival was calculated incorrectly in Managed Descent mode, except the "hard" constraint at DOGGG. Inbound from SAYGE it looked like managed DES (with selected speed) would work, so the PF managed DES. However, the FMS still planned FL173 at MOKME (even after we pushed the correct altimeter setting). The pilot flying selected V/S again to meet the restriction yet when we crossed MOKME at exactly 17,000 the PROG page showed that we were 350 FT below the managed/planned glide path. 

Lastly, several miles prior to DOGGG when the FMS should have reduced managed speed to 210 KTS (per the arrival), it actually went into Approach mode and reduced to green dot. This arrival would have caused violations of altitude and/or speed at almost every waypoint had we not been closely watching its every move.

The FMS needs to be tested and corrected on this (and all) of the arrivals to ensure it is working properly. I have had the same problems (to a lesser extent) on the EAGUL FIVE RNAV ARRIVAL (landing east) into PHX, where the FMS will do an ALT CST at TINIZ (at FL330 even though the restriction is "At or below FL330"), and then begin a descent too late to meet the restriction at PAYSO. Also, on the JAGGR arrival (DEN), the FMS "dives" to FL270 at LYNKN even though this constraint is "At or above FL270", then shallows into a very shallow descent to CLPTN. This is not needed and not efficient. When the constraint at LYNKN is deleted it calculates a much better crossing altitude of FL302 with a THR IDLE descent all the way down. 

The FMS seems to work correctly when there is a hard altitude/speed constraint. But something is not working properly in the FMS when it calculates these descent profiles when there is an "At or above/below" or a "Between" constraint.

Synopsis

A-320 fails to comply with the complex altitude and airspeed restrictions on the KOHOE RNAV STAR to DEN. The reporter believes the problems to be systemic to the A-320 autoflight systems and backs that opinion up by citing previous similar problems when attempting to comply with similar RNAV STAR arrivals at PHX and DEN.

ACN: 1055550 (2084 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : FMS/FMC
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 275
Experience.Flight Crew.Type : 2200
ASRS Report Number.Accession Number : 1055550
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.ATC Issue : All Types
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Denver has posted new RNAV STARs which are unsafe and full of distractions. First of all they are on multiple pages displayed in half [sections]. Today we started out on the TSHNR which states in its notes that its ["corresponding STAR"] is the MOLTN if DEN is landing north. Denver was landing north today and also west on 26. So what are we really going to fly? 

We installed our filed TSHNR arrival, and then we [were] changed to the MOLTN STAR on descent. Then we were given 080 heading out of BAACK. [This caused] a lot of confusion and the changes resulted in distractions during the descent. 

The next safety issue was that we had an A320 with the newest, most sophisticated FMGC aboard which was unable to fly the speed and/or altitude restrictions in these new arrivals. We had to kick off the autopilot twice and the auto throttles once and hand fly to keep up with all the altitude and speed changes. 

This is the best technology and it was not even close to being able to comply with the multiple altitude windows and speed changes in these new arrivals. An example was for the aircraft to cross an altitude window between FL250-FL220 at 280 KTS. Instead of complying with the speed it dove for FL220 and was going to be 40 KTS fast. 

These new arrivals are beyond the aircraft's capability and will distract pilots into mistakes. Please help.

Synopsis

An A320 First Officer expressed concerns about the newly initiated RNAV STAR procedures at DEN, believing them to be excessively complicated and demand capabilities the state of the art FMGC installed on his aircraft was unable to provide. In the instant case it was necessary to disconnect the autopilot twice and the autothrottles once to avoid non-compliance with crossing restrictions.

ACN: 1055547 (2085 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 1055547
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was operating as the pilot not flying. Received a call from the Lead [Flight Attendant] telling me there was a strong burning plastic smell from the aft galley. He briefed me on the actions he and the other two flight attendants took to locate the source and mitigate it. They were unable to locate the source and asked if one of us could come back and take a look. My initial concern was someone might be trying to breach the cockpit. I don't like the cart being moved or a pilot going into the cabin during flight. We established security was normal and decided the First Officer would go to the aft galley. I got the weather for an airport (we were less than 100 NM from it) and other airports along our route. I fully expected the First Officer to return and tell me he found the problem and it was nothing of note. Upon his return, the First Officer expressed concern about the burning smell. I told the Lead that at that time we might be diverting. I'd let him know in a few minutes. At the same time, I sent an ACARS to Dispatch to call me with Maintenance Control. After the Lead left the flight deck, the First Officer further briefed me as to his opinion. He then told me "Captain, in my best crew resource management, I strongly recommend divert." Dispatch was on the radio at this time. Dispatch was breaking up and hard to understand, Maintenance was impossible to understand. I gave the radios and aircraft to the First Officer, told him to declare an emergency. We donned our oxygen masks, established communications. I told the flight attendants we were diverting, made an announcement that we were making a precautionary landing, sent an ACARS to Dispatch in case he didn't copy while on the radio, got the ATIS, loaded the box for the arrival to divert city. I turned off the Galley/CAB switch then inquired the cabin crew about the burning smell. I have to admit, Swissair accident was going through my mind, the speed of that fire. I was informed that after I turned off the power, the smell went away. We landed on the runway. Almost the entire event went smooth, no additional threats, just as trained at the training center. Great CRM between pilots and cabin, the only real problem I saw was our new Flight Manual format. It took too long to find information and checklists. We need tabs in it. I wanted to review the Smoke/Fumes checklist in case it was needed later, took too long to find. I would have liked a little systems review, took too long to find. Since my iPad was off, I didn't try it. It would take too long to boot up and too many sign-ins.

Synopsis

An A320 Lead Flight Attendant notified the Captain of an aft galley burning plastic smell. After the First Officer investigated, an emergency was declared and the flight diverted to an enroute airport.

ACN: 1054480 (2086 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 1054480

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 1054485

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Cruising at FL380 and start to step down for our approach. Step down to FL360 then FL320. While at FL320 notice on ECAM that cabin altitude is reading >9,000 feet and pulsing green then I believe steady amber. As cabin altitude continues to climb cabin altitude displays red followed by CAB PR EXCESS CAB ALT ECAM (my recollection). Donned masks and requested lower from ATC. First Officer flying and Captain runs through ECAM checklist. Recall 3 or 4 steps. CREW OXY MASK - ON, ATC - NOTIFY, PAX OXY MASKS - MAN ON. Followed ECAM. ATC asks if we are declaring an emergency and at that point we state affirmative. They give us direct to destination as we decide on alternates. First Officer continues to step down. Recall at one point Safety Valve Position drives open then closes. Feel a cool refreshing air flow. At approximately 16,000 feet enter into FM and this suggests trying CABIN PRESS MODE SEL to MAN. Doesn't appear to respond. Continue our descent and request a phone patch to Dispatch. Also ACARS Dispatch stating that we have declared an emergency and request possible alternate airports if necessary. Response from Dispatch is that destination looks good. Once through 10,000 feet, remove oxygen masks and then level off at 8,000 feet. Notice cabin altitude is equal to aircraft altitude. Establish communications with Purser, passengers and Dispatch. Decide not to Maintenance as we are closing in on destination and everything is under control. First Officer briefs approach and lands uneventfully.

Narrative: 2

We were getting step down descents, FL380, FL360, and was now level at FL320. ECAM for Cabin Pressure. Saw cabin pressure rising over 8,000 feet, both pilots donned O2 masks, Captain worked ECAM, [and] I continued to fly. Cabin pressure continued to rise, we declared emergency and requested lower. Aircraft continued to slowly depressurize. Captain continued with ECAM (including passenger mask deployment), QRC, and into the FM. Captain commented that Manual Pressurization did not seem to be working well either. I was working with ATC to get lower as [soon as] possible, we were over the mountains so step downs came in intervals, first FL200, then 16,000 feet, then 14,000 feet, and eventually down to 8,000 feet. Captain was working with Purser and Dispatch, we agreed on continuing to destination as the aircraft was now able to be flown de-pressurized. At one point during descent, aircraft began to over pressurize and safety valve opened. At 8,000 feet we removed O2 masks, Captain and I got back on the same page, reviewed approach, finished checklists, conferred and confirmed everything that needed to be done had been done, and concluded with an uneventful ILS in VMC conditions. Autopilot was engaged until the outer marker. On taxi in, ECAM calling to make sure cabin was depressurized, I was able to manually open outflow valve at this point.

Synopsis

A320 flight crew experiences a gradual loss of cabin pressurization during initial descent for landing with the First Officer the pilot flying. ECAM actions are complied with when cabin altitude exceeds 10,000 feet and manual pressurization is attempted. At one point during descent, aircraft began to over pressurize and safety valve opens. Flight continues to destination for normal landing and during taxi in ECAM alerts to cabin being pressurized on the ground.

ACN: 1054476 (2087 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Nose Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1054476

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff Captain raised landing gear and nose gear failed to retract resulting in an ECAM. Captain accomplished ECAM which resulted in normal gear indication but ECAM read "Landing gear gravity extend." All indications other than left over ECAM were normal with gear up. Captain contacted Dispatch, Maintenance Control, and the Duty Manager to discuss further action. All three parties as well as the crew felt that the aircraft was 100% and ok to continue. Flight was normal and uneventful. Dispatch declared an emergency upon our arrival on our behalf. Gear was extended normally and a normal landing and taxi to gate was accomplished.

Synopsis

A320 First Officer reports nose landing gear failure to retract after takeoff. After ECAM actions nose gear is retracted with ECAM reading "Landing gear gravity extend". Landing gear extends normally during approach at destination.

ACN: 1054371 (2088 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Cabin Activity : Deplaning
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1054371
Human Factors : Communication Breakdown
Human Factors : Fatigue
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Other 
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Primary Problem : Company Policy

Narrative: 1

As detailed below, all passenger's, as well as the cabin crew's, safety was jeopardized for the above listed flight. Due to the increasing delays and constant phone call disturbances, we were not able to get adequate rest (actually none) before working the modified pairing, which you will find at the end of this email. I called the Captain in the hotel, who was unaware of the delays. He was never notified. Actually, I initiated the first call to Crew Scheduling to have our show time adjusted. After this time, the Captain made all calls to the Scheduling Department. Due to the constant changes in departure, calls from the Captain with updates, changing van times with the local transportation company, my contacting the others with the Captain's updates, none of us were able to get any sleep from the time we got up that morning, until after our arrival in our base at sunrise the following day, at which point I had been awake for over 24 hours. The Crew Scheduling Department called me the evening before and I was told to call In-flight about the Captain's and our concern about not feeling safe to do our second flight due to fatigue, not being alert/aware, having the responsibility for the SAFETY of our passengers. The Captain called the In-flight Operations Department and spoke to a supervisor, who told him she understood/agreed with the situation, however she could do nothing about it. The Captain was adamant each time he spoke to Crew Scheduling about the flight attendants not being fit to continue our trip. He continued to call about the matter while on the way to the airport, by ACARS during flight, to which the Company's response was, "flight attendants are expected to continue as scheduled, to contact In-flight upon arrival." Upon arrival he received a message, no In-flight supervisor on duty at airport. I tried to call In-flight- no response. I spoke with a supervisor in Crew Scheduling, who told me he could find no replacements for the cabin crew, implying we were to continue to our crew base even though I voiced our concern about fatigue. The Captain was released at the intermediate station since he was not safe to continue, so why were the flight attendants not released? What made us safe to fly if he and we were all lacking the same rest? I realize on paper that we were well within the duty limitations of our union and FAA regulations, however, the situation of not being able to rest before working the flights was not taken into consideration when we and the Captain repeatedly voiced our concerns about the large responsibility we have while operating a flight. In conclusion, why does the PIC have the ultimate decision about the safety and security of his/ her passengers and crew, if this decision is totally disregarded?

Synopsis

A Flight Attendant describes hotel sleep interruptions, flight delays and a duty time combination preventing sleep for nearly 24 hours during which time the pilots were released due to fatigue but not the flight attendants.

ACN: 1053924 (2089 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance

Component : 1

Aircraft Component : Fan Disc
Manufacturer : Aerospatiale
Aircraft Reference : X

Component : 2

Aircraft Component : Fan Blade
Manufacturer : Aerospatiale
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1053924
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1054579
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Midnight shift. Myself as Mechanic X and Mechanic Z were attempting to remove the Retaining Ring for the Number 1 Engine. We were unaware that the retaining ring had been installed incorrectly. Therefore, when we tightened the jackscrews to remove it we encountered resistance. Ultimately, the jackscrew at the twelve o'clock position sheared off. At that point we thought that the jackscrews were too short so, we tried longer jackscrews to no avail. We were not able to remove the retaining ring so we notified the Maintenance Supervisor. The problem was not resolved before the end of shift. When I came into work for my next shift, I was made aware that the Retaining Ring had been installed incorrectly. As a result of that incorrect installation, our attempt to remove it ended up causing slight damage to the flange; aircraft damage.

Narrative: 2

During the installation of the Front [Fan] Blade Retaining Ring there is a (...CW...) item [signature block item on Job Card] and it is not clearly understood as to whether this is a 'Clearance' item, or a 'Check' item before 'Continuing' [on with the work] item. [Mechanic Y].

Synopsis

Two mechanics report about their efforts to remove a Retaining Ring at the Forward Fan Blade assembly on a V2500 engine. The A320 aircraft's Number 1 Engine flange was slightly damaged.

ACN: 1053870 (2090 of 2908)

Time / Day

Date : 201212
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 20000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Powerplant Fuel Control Unit
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1053870
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The flight crew reported for duty at about sunrise and performed their appropriate duties to prepare for passenger boarding. Neither the pilots nor flight attendants reported any abnormalities. I called Maintenance Operational Control (MOC) to clarify a date on an MEL item listed on the Flight Release to confirm it was correct, and was advised it was. Upon completion of the cockpit setup and preflight walk around by the First Officer, we completed the Preflight Checklist; I briefed the flight attendants, and then waited for the completion of boarding. Completion of boarding, pushback, engine start and taxi to the assigned departure Runway 28 was uneventful. Our position in the takeoff line up was number 3 behind two air carrier aircraft. After both departed, Tower cleared us for takeoff; I transferred control to the First Officer, who was the Pilot Flying (PF), he powered up as appropriate with a pause at 1.05 to allow the engines to spool, then proceeded to apply power to the Flex Thrust setting. No abnormalities were noted during this phase until, in retrospect, the aircraft reached the speed of 80 KIAS and the "80 Knots - Power Set" callout. I was the Pilot Monitoring (PM) and made the callout but noticed that the right engine (Engine 2) was slightly hunting for the correct power setting of approximately 1.35 EPR while the left engine had locked onto the appropriate setting. The fluctuation in the right engine was minimal, around 0.005 EPR and seemed to stabilize by 90-95 KIAS. This appeared insignificant at the time. The rest of the takeoff throughout the initial climb to 10,000 MSL was uneventful. Upon climbing past 10,000 MSL to an assigned altitude of Flight Level 230 (FL230) the aircraft was about to enter possible icing conditions therefore Engine Anti-Ice was turned ON in preparation. No abnormalities were noted and climb-out continued uneventfully until approximately 20,000 MSL where icing conditions no longer existed and the Engine Anti-Ice was turned OFF. Shortly thereafter, we were alerted by a single chime to the ECAM display indicating the following message: "ENG 2 FUEL CTL FAULT" and subsequent action items below the title line. ATC directed us to climb to FL300, to which I responded our need to maintain this altitude because we were working a problem and would advise when able. The First Officer remained as PF, maintaining control of the aircraft and assuming communications. I began executing the ECAM actions the first of which, if I remember correctly, commanded the Thrust Lever on the affected engine be reduced to idle for 30 seconds. As we confirmed and slowly reduced the ENG 2 thrust lever, I noticed the trend arrows on the EPR gauge displayed the commanded power reduction to the new EPR TLA position (idle). I further noticed the engine initially had not spooled down completely on its own but was at a reduced power setting as compared to ENG 1. The engine then proceeded to match the idle thrust lever setting, after the 30 seconds had expired, the next ECAM Action Line commanded to return the thrust lever to the Climb Detent to presumably check the ability to control the engine. This action had no effect and ENG 2 remained at IDLE thrust. Subsequent ECAM Action command lines included placing the Engine Mode Selector to IGN/START and then shutting down the affected engine. I continued performing subsequent additional ECAM Actions of "ENG 2 FAIL" thereby securing the engine, and confirming failed systems on the ECAM display. It was at this time the APU was started. A plan of action was devised to return to the departure airport since it was both the closest airport and closest Company airport. I re-assumed control of communications and declared an emergency to ATC and requested vectors to return to the departure airport. ATC immediately complied as did we with a turn back to the departure airport. I sent an ACARS message to Dispatch stating our condition and returning. I notified the Lead Flight Attendant of the aircraft status, time remaining to landing, my intent not to evacuate unless other indications led to an evacuation decision and finally that I would be making an announcement to the passengers. I then calmly made the PA announcement to the passengers informing them of what had occurred, emphasizing that the aircraft was still very safe to continue flight, and that we would be landing in 15 minutes. I finally added that the flight attendants would be giving them further instructions. After completion of all the immediate action items, the First Officer and I reviewed and completed the appropriate COM items while continuing to the airport. Descending through 10,000 MSL, I informed the flight attendants over the PA then shortly thereafter called the Lead Flight Attendant via the intercom to confirm they were indeed ready for landing and obtained an update as to the status of the passenger cabin. The lead informed me they were ready, the cabin was prepared and the passengers were calm. We finished our duties and prepared for the landing, selected Flaps 3 on the MCDU and GPWS Mode Panel on the overhead, then performed a quick review of what we had accomplished to this point. The remainder of the approach and landing occurred without incident. The landing was "on target/on speed" and under MGLW. Fire trucks followed us down the runway as we decelerated toward the exit. We queried Ground Control regarding any other signs of damage, smoke or fire to which the reply was negative; this was also confirmed by the Fire Marshall. I announced "Flight Attendants Stations" to keep the crew informed of my decision not to evacuate the aircraft. We continued to the gate uneventfully. Once parked and all checklist were completed.

Synopsis

An A320 ECAM alerted ENG 2 FUEL CTL FAULT, so as the ECAM checklist was accomplished, the engine shutdown as directed, an emergency declared and the flight returned to the departure airport.

ACN: 1051121 (2091 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 260
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 1051121
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Incursion : Runway
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

The First Officer landed on Runway 28R at SFO. We proceeded to Taxiway T turnoff where we exchanged control of the aircraft. As the exchange was happening, we were instructed by SFO Tower, "cross Runway 28L expedite crossing aircraft on final." My First Officer acknowledged the crossing and I aligned my aircraft with Tango, added power to hurry across, I did my last check of 28L before crossing when I saw an Airbus going rapidly down the runway. I immediately brought my aircraft to a stop past the hold short line but before the runway edge lights. The Airbus on 28L stopped his aircraft short of Delta and called the Tower. SFO Tower sent the aircraft around, the Airbus on 28L cleared the runway at Delta and when the way was clear I taxied across 28L and on to my gate. I remembered later a partial clearance to another aircraft to expedite which I believe was the other Airbus on 28L. I am certain that if I had not stopped my aircraft we would have at least collided wingtips. There is not enough room to have an aircraft clear 28L at Delta and simultaneously cross an aircraft of Taxiway Tango.

Synopsis

A320 Captain reported a ground conflict in SFO when he was cleared to cross 28L at Tango at the same time another Airbus was clearing the runway at Delta. Reporter feels that is a dangerous clearance.

ACN: 1051111 (2092 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2200

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1051111

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1051264

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[We were] established on final [when] I called for the gear down, landing checklist. The Captain lowered the landing gear handle and we both noticed the left main landing gear light did not illuminate. After a very brief discussion we both decided not to attempt a landing. At approximately 1,000 feet we executed a normal go around and were cleared to 4,000 feet. At 4,000 feet we were vectored by ATC allowing time to declare an emergency, execute the Gear Disagree [checklist] and review the Partial Gear Landing checklist. After executing the appropriate checklists we were vectored to final, executed an uneventful landing and stopped straight ahead on runway.

Narrative: 2

[On final, after placing gear handle down] left main gear light did not illuminate. Pressed the light fixture and light did illuminate while being held in. Test switch on overhead panel would NOT illuminate the light. Between 1,000 feet and 500 feet we elected to go missed approach. Did a normal go around and accepted 4,000 feet and vectors. Advised flight attendants what was happening. Prior to downwind gear lever placed down with same results. Pressed light fixture numerous times with same results. Accomplished the Gear Disagree checklist including alternate gear extension with no improvement in the condition. Declared an emergency, asked company be notified and equipment standing by. Said we would stop straight ahead on the runway. Flight attendants told to prep for emergency landing with possible left gear collapse. Inside 500 feet I advised BRACE over the PA. Landed without incident. Advised [passengers] over PA to remain seated. Stopped on the runway. Met by crash equipment and awaited for Maintenance to arrive and pin all three gear. Taxied to gate without incident.

Synopsis

A320 flight crew reported left main gear light remained out after gear extension. They ran appropriate checklists, declared an emergency, and landed normally.

ACN: 1051085 (2093 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 16000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : BAZBL
Airspace.Class E : ZHU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : B737-900
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Airspace.Class E : ZHU

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1051085

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

During descent encountered moderate wake turbulence following a B737-900. Fortunately seat belt sign was on and no injury to the passengers, however it did throw flight attendant into her aft jump seat area. We immediately contacted the all flight attendants after the event and learned she had injury to her shoulder and neck area, but not critical and no need for medical priority handling. Within several minutes we encountered a second event of light wake turbulence behind the same airplane. At this point we elected to slow our airspeed to 210, creating greater spacing. Made comments to ATC for reference, based on TCAS we already had approximately 5 miles or greater of separation when this took place. Landed on Runway 26L without incident and the other aircraft arrived on Runway 26R.

Synopsis

A320 Captain reported a wake vortex encounter following a B737-900 on arrival to IAH that resulted in a slight injury to a flight attendant.

ACN: 1050976 (2094 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight
Ceiling.Single Value : 20000

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 190/195 ER/LR
Operating Under FAR Part : Part 121
Flight Plan : None
Flight Phase : Parked

Component

Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Experience.Flight Crew.Total : 9794
Experience.Flight Crew.Last 90 Days : 214
Experience.Flight Crew.Type : 5721
ASRS Report Number.Accession Number : 1050976
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Aircraft
Detector.Person : Ground Personnel
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

While taxiing into gate at approximately 2-3 KTS, both my First Officer and I picked up our initial lead in marshaled. The taxiway was clear with an EMB-190 to our left that was parked in front of a maintenance hangar. While following this Marshaller's lead he directed me to pick up the Marshaller on the lead in line near the jetway. I made visual contact with this Marshaller and began following his guidance. The initial Marshaller then moved up and now became our left wing walker. I now had a total of three marshallers, lead in, left Wing Walker, and right Wing Walker. I was following the lead in Marshaller who was lining me up on the lead in line, and my left Wing Walker was giving me an all clear. We then felt a thud and both of us realized that our left wingtip fence had made contact with the tail cone of the EMB-190. I brought the aircraft to a complete stop, shut down the left engine and requested a tug to tow us to the gate. There were no injuries to either crew or customers.

Synopsis

A320 Captain reported their wingtip fence made contact with a parked aircraft during gate arrival.

ACN: 1050205 (2095 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ONT.Airport
State Reference : CA
Altitude.MSL.Single Value : 3400

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 26L
Flight Phase : Final Approach
Airspace.Class C : ONT

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : RAL
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Route In Use : None
Airspace.Class E : SCT

Person

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1050205
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Airspace Structure

Narrative: 1

An A320 was on ILS 26L into ONT just past PETIS NDB descending out of 3,400. I observed a VFR target without transponder from north of Highway 10 southbound going to cross the ONT final and issued traffic to the A320. I offered the A320 a vector around the traffic and he declined. The traffic did not merge, but only just barely. This is a known VFR transition area just below the Class C at ONT. The traffic was clear of the A320 and I transferred communication to ONT. Subsequently I called RAL Tower and asked if they were talking to the aircraft without transponder, they were and I asked the altitude of the aircraft. They advised he said he was at 2,200. This is an all too common problem at ONT, what I would suggest is that we protect the aircraft on the ONT final better and have the east extension of ONT's Class C be down to the surface out to PETIS NDB. This problem needs to be addressed before something does happen on the final into ONT.

Synopsis

SCT Controller described a conflict event involving Air Carrier traffic inbound to ONT on the ILS 26L and VFR traffic crossing the ILS path, the reporter recommending ONT's Class C be expanded to reduce same type events.

ACN: 1049267 (2096 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35500

Environment

Weather Elements / Visibility : Rain

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Aircraft : 2

ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : DC-10 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1049267
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

As FLM, I was quick-looking the sector from another sector due to weather, deviating and moderate-high traffic volume. Sector was split and, although it was not "red", I put a D-Side in. When Air Carrier X and Air Carrier Y were approximately 60 miles apart and head-on, I noticed Air Carrier X had been given climb from FL350 to FL370. With head-on traffic at FL360 and weather/precipitation in the area, I measured the distance of the head-on traffic on the scope that I was using to quick-look. The climb clearance was a good clearance and did not originally look risky, although it beared watching. Air Carrier X was climbing at a normal rate which was working well to top the traffic but, as the traffic continued, Air Carrier X started to slow the climb at FL366 and stayed at FL366 for a few 'hits'. At this point, I walked over to the sector. The aircraft reached FL367 for 1 'hit'. The head-on traffic, Air Carrier Y, was 20 NM away at that point. The altitude read out then descended and read FL366 again. I heard the Controller inquire about the climb. I told her to turn the aircraft. She issued a "turn 20 degrees right...correction left." There was very little time as the closure rate was high and I told her again to turn the aircraft. I told her to give him a heading. She again issued a degree turn. I told her to turn the other aircraft too. She issued a degree heading to Air Carrier Y also at that time. Air Carrier Y had to be turned left/South, which was toward the weather he was deviating around. The sector was split and there was a D-Side working with the Radar Controller, assigned by me. It was not a 'red' sector. When the R-Side Controller and I discussed the event, I recommended the use of an assigned heading rather than a degree turn. That way, the pilot has less to think about and can just enter a heading into their equipment. Also, we discussed how weather/precipitation in the vicinity can affect traffic in unexpected ways. With precipitation and deviations, do not issue a climb with head-on traffic. I also recommended the R-Side pay more attention to the tone of voice. Listening to the tapes, the Controller realized how "NOT" urgent the control actions seemed. The Controller originally thought that they were using a stern tone of voice. We talked about how, in actuality, the tone of voice was quite pleasant, and did not convey urgency. We discussed the balance of not panicking but letting the pilots know more of the story so they respond how we need them to.

Synopsis

Enroute Supervisor described a developing head on event involving two Air Carriers and weather deviations, the reporter eventually directing the Controller to turn both aircraft saving separation.

ACN: 1049030 (2097 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 29000

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZLA

Person

Reference : 1
Location Of Person.Facility : ZLA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1049030
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Flight ABC showed not auto hand off eligible even though established on the LOA arrival for PHX. We reported several of these instances to our ERAM team today and have been advised that there were some errors in our facility adaptation for the new Chart Date. We have had several instances of aircraft 4th line data not displaying due to aircraft type classification. Several aircraft have shown to need preferential routing via classic procedures even though filed RNP/PBM with appropriate qualifiers and established on the appropriate routes. Several instances where ZAB and ZLA each had track control on same aircraft and we were unable to hand off to our adjacent facility ZAB. After the ERAM team evaluated these issues, one of the team members returned to advise that the issues had been identified and would continue until the next charting date. There were errors in the adaptation for the chart date system that were identified but will endure for the next two weeks. Since the errors have been clearly identified, they should be corrected within the next day or two, not waiting two weeks.

Synopsis

ZLA Supervisor voiced concern regarding current procedures on ERAM software updates, noting after anomalies are documented updates should be completed in a timely fashion and not delayed until official charting schedules.

ACN: 1048660 (2098 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1048660

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Detector.Person : Flight Attendant
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After landing checked with Operations, they advised ground air and electric available. We arrived at gate with engine one running. Ground crew parked the aircraft and attempted to start ground power unit. After waiting 4 minutes, we started the APU. The ground power unit was never connected prior to the crew leaving the aircraft. After the cockpit door was opened a flight attendant stated that ground personnel were walking within 10 feet of the running number one engine. There are no engine inlet safety markings on the ramp. The company procedure does not account for the lack of these markings or proper training of ground personnel. The only other airline that does not [use] APU taxi in procedure has engine inlet safety marking at all of their gates and use only company ground crews. We violate the Airbus 19 foot inlet safety guideline in many of our locations. I recommend we suspend the "No APU taxi in" procedure until a comprehensive review by the Safety Department is completed. No action will result in a catastrophic event.

Synopsis

Flight Attendant reported to the flight crew that ground personnel were walking within 10 feet of a running engine at the gate.

ACN: 1046680 (2099 of 2908)

Time / Day

Date : 201211

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1046680
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Enroute to our destination we had an ECAM ENG1 REVERSER FAULT followed by another ECAM ANTI ICE FO PITOT. After completing the ECAM and referencing the Flight Manual, I contacted Dispatch and Maintenance Control to discuss the situation. While talking to Maintenance Control he advised that he had downloaded more information from the aircraft and informed me that the LGCIU 1 [Landing Gear Control Interface Unit] was also inoperable and probably causing the REVERSER INOP ECAM msg. While talking to Maintenance Control, I began scrolling through the MCDU menu to the CFDS menu to view the LEG REPORT and the LEG ECAM REPORT status pages. I noticed there were no indications of LGCIU 1 in-operation or failure on the ECAM or LEG message pages, but Maintenance Control said the DATA LINK indicated so. I was not convinced of this (nothing to verify this fault on board), but we discussed the fact that LGCIU 2 was available for Gear Extension, normal braking (Anti-skid also available) and we still had Reverser 2. The weather at our destination was VMC and no operation issues of concern, so Dispatch, Maintenance Control, and the flight crew agreed to continue to our destination that was 2 hours away. With the time remaining to our destination, I discussed with my First Officer what we could expect with any performance concerns, the ENG 1 Reverser issue and our landing, roll-out expectations and concerns. We also reviewed the Flight Manual about the gear operation, WHEELS page display for LGCIU 1 INOP and the NON-NORMALS for potential other gear issues/warnings that might develop with Gear Extension. We also discussed lowering the gear sooner in the approach to see if we would have anything else to deal with before landing. As Pilot Flying (conducting an First Officer IOE), we began to configure about 15 miles from the airport. At about 12 miles from [Runway] XXR, we lowered the gear and experienced an L/G ECAM. The First Officer began doing the ECAM while I continued to slow and keep configuring the aircraft. As the First Officer performed the 1st ECAM another ECAM started and over the next 6-8 seconds (as I recall) we received 3 maybe 4 rolling ECAMs and the screens were cycling between them. This was a bit unnerving as the MASTER WARNING alarm was almost continuous as the screens were cycling and the First Officer was trying to comply with the ECAM steps. As this was going on, I advised ATC we were having a Gear Extension issue and would get back to him. He replied "Roger, keep me advised." In the rolling of the ECAMs, the First Officer said that an ECAM (not sure which ECAM) directed to cycle the gear up. He continued with the ECAM procedure and about this time the First Officer advised that one of the ECAM screens (we're not sure which one) called for the Gravity Gear Extension procedure. At this point I advised him to crank the gear handle and he began the Gravity Gear Extension. I also advised ATC that we were doing the Alternate Gear Extension (my words) and the Gear Doors will remain open as this a result of the procedure. He advised that he already call the [emergency] trucks as part of his procedures. 

My attention was diverted back to the First Officer who was managing the manual Gear Extension and we noticed that the 3 GEAR Green lights on the CENTER PANEL were green and the WHEELS page showed both LGCIUs confirmed the gear was down and locked. I made a quick call to the flight attendants to let them know the NORMAL Gear extension didn't work but the ALTERNATE method did and the gear was fine and the landing would be normal but might not have steering capabilities to exit the runway and to stay seated until we advise otherwise. The aircraft was fully configured and we both agreed to continue the landing and all checklists were completed. Knowing both LGCIUs were indicating, I mentioned that we would have both reversers (we did) and touch-down and braking and would try and clear the runway if possible. As I recall, he did advise the Tower we might be stopping on the runway. Touchdown and rollout were uneventful. I was able to clear Runway XXR and exit on a taxiway and not block the runway. CFR vehicles met us and we advised the passengers to stay seated, secured the aircraft, and coordinated with CFR and our company for a tow to the gate.

Synopsis

A320 Captain experiences an ECAM ENG1 REVERSER FAULT followed by another ECAM ANTI ICE FO PITOT during cruise flight. Maintenance Control believes LGCIU 1 is causing the problem and the flight should continue to destination. During approach the landing gear does not extend normally and the ECAM screen begins cycling rapidly with audible warnings making it almost unusable. After gravity extension of the landing gear all is quiet and a normal landing ensues.

ACN: 1046589 (2100 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 100

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1046589

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During landing gear retraction the Green Hydraulic System failed. The Pilot Monitoring secured the PTU and the engine pump as per ECAM, but the System Display indicated a total loss of fluid. We continued climb out to cruise. We discussed the problem with Dispatch and Maintenance Control, during which we decided to divert to a nearby airport. An emergency was declared. We selected the diversion airport based on its long runway, good weather, pilot familiarity, excellent facilities, and available Company Maintenance. 

We debated whether an overweight landing should be accomplished, but considered doing so prudent and the safest course of action. We configured early and had to use the landing gear gravity extension procedure. The overweight landing at 154,000 pounds was uneventful. We stopped straight ahead on the runway and were towed to the gate. 

A deadheading pilot, who was invited to the flight deck for the remainder of the flight for possible additional input, reported loud squealing sounds on takeoff during gear and flap retraction. The passengers and cabin crew were briefed several times en route to our diversion airport.

Synopsis

After suffering a Green Hydraulic System failure after gear and flap retraction the flight crew of an Airbus, after consultation with Dispatch and Maintenance, elected to divert to a nearby airport where an emergency was declared and an uneventful overweight landing resulted.