ACN: 1046558 (2101 of 2908)

Time / Day

Date : 201211
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZSE.ARTCC
State Reference : WA

Aircraft

Reference : X
ATC / Advisory.Center : ZSE
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise

Person

Reference : 1
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1046558
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Signed on and had my data blocks go full bright. The relieved Controller's method of tracking NORAC's is to highlight the aircraft he is talking to. My method is the complete opposite (I highlight the ones I am NOT talking to.) It is very distracting and consumes valuable time right after assuming control of the sector to have to go to each and every aircraft to unwell the data block. We need one simple standardized way to track NORAC's. While changing over to my system of tracking NORACs I missed a hand off flashing at me and Sector 13 had to call. Later on Sector 46 asked if I could try flight ABC to Sector 13 frequency because they were NORAC. When I checked to see if the aircraft was on my frequency they were still there so I must have converted the dwell but thought I didn't. I thought the aircraft had already been shipped to Sector 13 by the Controller had being relieved because of the difference in tracking methods and the conversion process. Implement a standard method for tracking NORAC's so that there is no need for switching methods immediately after taking over a sector. In the meanwhile create a function to allow an all aircraft dwell toggle so that I can toggle all data blocks immediately to my dwell method of tracking NORACs. If I could toggle all data blocks to the opposite of what they are then that would be great.

Synopsis

ZSE Controller described a confused relief effort when the relieving controller utilized the exact opposite procedure regarding data block illuminations, the reporter suggesting a common facility procedure be adopted.

ACN: 1046217 (2102 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5700

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : VHF
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1046217
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Just after takeoff when attempting to contact Departure Control, we received the ECAM "VHF1 EMITTING." We continued with the departure procedures, watched for traffic and continued to contact Departure Control. At level off we tried both our Radio Management Panel (RMP) and Audio Control Panel (ACP) without success. We then received another ECAM for "VHF2 EMITTING." Having one of our captains in the jump seat I had him try the third radio while I accomplished the QRH procedures and squawked 7600. He was able to reach a Controller on 121.5, however when instructed to contact ATC on another frequency, we received nothing. After continuous effort for several minutes to reach ATC, our ECAM messages cleared and our radios were normal for the duration of the flight. The QRH assumes there will only be one radio inoperative. My suggestion would be to make a note in the QRH regarding the transponder procedure if there happens to be a total loss of communication. It would also be nice to have a reference page for the lost comm procedure that is found in the Flight Operations Manual.

Synopsis

A320 Captain experiences lost communication on departure due to a stuck mic that is annunciated by ECAM as "VHF1 EMITTING". The problem transfers to VHF2 when that radio is selected and eventually clears on its' own.

ACN: 1046149 (2103 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 4000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb

Person

Reference : 1
Reporter Organization : Government
Function.Air Traffic Control : Departure
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1046149
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Air Carrier X off opposite direction Runway 8 ONT, climbing to 4,000 FT. Cherokee off AJO climbing to 4,000 FT, stopped at 3,000 FT when communications were not transferred in a timely manner. I called to verify runway traffic configuration. Air Carrier X 1 1/2 South of ONT, I reached out to see if he was on frequency (not there) ONT Tower called and informed me that he was issued the wrong clearance and that they would re-clear and transfer communication after clearance was issued. I instructed the Tower to transfer communication and I would give him alternate instructions. Communication was not transferred in a timely manner after coordination with the Tower. Air Carrier X checked on frequency abeam PDZ. I originally gave him a right turn then realized he was going to fast and had already passed PDZ to make the turn and remain clean of the MVA's so I immediately re-issued instructions for a left turn and an immediate climb above the MVA's. Air Carrier X entered the edge of a 5,300 FT MVA as he was leaving 4,000 FT.

Synopsis

SCT Controller described a below MVA event when ONT Tower failed to transfer radio communication on a departure as requested.

ACN: 1045304 (2104 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZOB.ARTCC
State Reference : OH
Altitude.MSL.Single Value : 33000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZOB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZOB

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZOB
Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 170/175 ER/LR
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZOB

Person

Reference : 1
Location Of Person.Facility : ZOB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1045304
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

I was working an increasingly busier session on RADAR at the Sector 68. The E170 was northwest and level at FL340, requesting FL320 for moderate chop. I descended the E170, switched communications to Sector 67, and lost the Data Block for an A320 in a mess of Data Blocks, who was eastbound level at FL330. When I realized separation was going to be lost, I attempted to turn the A320 away from the E170, but the instructions were not received until the third transmission, for some apparent reason. As the A320 was making the turn I instructed, Sector 67 was turning the E170 away from the A320 at the same time, and separation was regained, but the A320 responded to a RA which instructed them to climb, and separation was lost on the next RADAR hit. Work on my scan, but why would TCAS climb an aircraft into one at a higher altitude?

Synopsis

ZOB Controller described a loss of separation event when failing to note conflicting traffic prior to issuing a descent. The reporter stated the TCAS RA resolution provided compounded the conflict.

ACN: 1044215 (2105 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 22000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : INS
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Problem : Malfunctioning
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1044215
Human Factors : Human-Machine Interface
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1044217
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were inbound to ZZZ at a cruise altitude of FL220 and were cleared to descend via the ZZZZZ TWO, south transition. We had already passed ZZZZZ1, which is the first fix on the ZZZZZ TWO and has a crossing restriction of FL230. The next fix is ZZZZZ2 which has a crossing restriction of FL220. We were between ZZZZZ1 and ZZZZZ2 at FL220 and selected 6,000 for the south transition and started a managed descent.

I was anticipating the FMS would have a constraint of 220 until ZZZZZ2 but the FMS started descending before ZZZZZ2. We were both discussing the ZZZZZ charting and we noticed the descent at about FL212 at which point we climbed back up to 220 just about the time that we crossed ZZZZZ2. Center reported no traffic conflicts but did provide a landline number to call which the Captain did after landing. He also spoke with Approach.

Both the Captain and I have been on the Airbus for many years so we are well familiar with the aircraft's systems and behaviors. Neither of us can figure out why there wasn't a constraint at ZZZZZ2 for FL220. Perhaps it was because we "cleared via" the ZZZZZ TWO while at a cruise altitude "below" the initial [restricted] fix (ZZZZZ1)? Neither of us could recall starting a "Descend Via" STAR at such a low altitude before so perhaps that is an issue with the FMS. 

If it can be determined that the FMS cannot accept starting a "Descend Via" STAR at an altitude below the initial one then there needs to be a software modification to allow it to do so or Airbus pilots will have to manually perform the descent.

Narrative: 2

I selected 6,000 FT for the lowest altitude on the arrival and selected managed descent past ZZZZZ1 and prior to ZZZZZ2 Intersections.

Upon reverifying the altitude constraints it was noted that that ZZZZZ2 constraint of FL220 had been deleted.

Synopsis

The flight crew of an A320 was cleared via the ZZZZZ RNAV STAR while in cruise flight at a Flight Level (FL220) below that of the initial altitude restriction on the STAR (FL230). When Managed Descent was engaged the FMGC initiated a descent from FL220 despite not yet crossing the next waypoint, which requires crossing at FL220. When advised by ATC the crew noted that the FL220 restriction had not been processed by the FMGC.

ACN: 1043723 (2106 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1043723
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Equipment / Tooling

Narrative: 1

Fuel truck leaking fuel onto ramp while fueling our flight, we called the company and the ramp and requested the fire equipment to be called out. The company overrode the Captains authority and canceled the emergency call for the fire equipment. After the First Officer completed the walk around he advised me that the fuel truck was leaking fuel into a five gallon bucket overflowing onto the ramp behind our right wing. He brought this to the fueler's attention and the fueler left to get someone. I advised the First Officer to call the Fire Department out... He called the company and ramp to advise them that we had a large fuel spill and to call out the Fire Department. I then proceeded up the jet way to have the boarding stopped. I told the agent to stop boarding because of the fuel spill. I then went down to the ramp and saw that the fuel truck was leaking. The puddle was around 30-40 FT in diameter. I told the maintenance man that we needed the fire company called out. He said that he had it under control. I went back up to the a/c and had the First Officer call the Company and ramp to find out where the fire equipment was. They replied that they are on their way. The agent resumed boarding without my consent. The agent came up and asked me if we were ready to close. I said we had a fuel spill. She said that it was taken care of. My First Officer again called the Company and requested the Fire Department and was told that they had cancelled the call to the Fire Department. He then asked them about the fuel spill, they said that only one gallon had spilled on the ramp, the First Officer replied "No, a five gallon bucket was under the truck spilling over for ten minutes." They then said five gallons had spilled. It was obvious that the company had no idea how much fuel had spilled. The fuel truck was moved from behind our aircraft out of view and it was still leaking onto the ramp. 

Prior to pushing after the doors were closed I called the Dispatcher and advised what had occurred to have this on record. I also advised the push crew we had a fuel spill behind our aircraft and he said we have it under control and that they are ready to push. We pushed off the gate right through a large puddle of fuel about 30-40 FT in diameter leaving fuel tire marks all the way back until we stopped. It was then we saw the same fuel truck fueling the aircraft on the next gate leaking fuel into a five gallon bucket all over the ramp. We advised Ground that the same truck that caused our fuel spill was fueling another aircraft and that they needed to call out the Fire Department. We were told by Ground that no one advised them about our fuel spill and that we had called company to have the fire equipment sent out. I feel that SOPs were not followed in this case. Also, the Captain's judgment and authority was over ridden by the company to cancel the call to the Fire Department. We are trained that safety is our number one priority. The company should have never canceled our call for the Fire Department this put all of us at risk.

Synopsis

Air Carrier Captain reports a leaking fuel truck to his company operations and to the Ramp Controller. No action is taken and the flight is loaded and pushed back through the puddle.

ACN: 1042924 (2107 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 19000

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class A : ZLA

Person

Reference : 1
Location Of Person.Facility : L30.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1042924
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This report is regarding a procedure that is used routinely, that causes conflicts in the air, and potential safety hazards in order to expedite and reduce taxi time. At LAS there are several RNAV departures, and several non RNAV departures, that are designed for direction of departure from TRACON to Center airspace. Runway 25 is generally used by Air Carrier aircraft, and Runway 19 is generally used by private and business aircraft. This causes the problem described in this report. The STAAV Departure was designed to allow departure traffic to be balanced between the two departure sectors. The STAAV Departure is only used with departures off of Runway 25. Here in lies the problem, the Runway 19 departures, who naturally would rather not taxi to Runway 25, but instead depart Runway 19, are assigned a departure SID that is non-RNAV, and converges in the air to the Northeast of LAS causing an imbalance of traffic volume in the two sectors, as well as additional workload to both sectors due to the SID being non RNAV. Possible solution, either allow the Runway 19 departures to fly the STAAV departure SID, or discontinue allowing the Runway 19 departures, instead requiring those aircraft to taxi to Runway 25 to be flowed in with all departures thereby providing the built in balance of volume between the sectors. Either allow the Runway 19 departures to fly the STAAV SID, or discontinue allowing the Runway 19 departures, instead requiring those aircraft to taxi to Runway 25 to be flowed in with all departures thereby providing the built in balance of volume between the sectors.

Synopsis

L30 Controller voiced concern regarding the current departure runway assignments noting alternate departure runway assignments and/or changing the SID would elevate the potential conflicts now being experienced.

ACN: 1042872 (2108 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLA

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 7500
ASRS Report Number.Accession Number : 1042872
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Prior to takeoff Clearance Delivery provided a full route reroute. The First Officer loaded new route and I verified the new route was loaded correctly. After departure, flight was given direct LAS. The First Officer inserted direct LAS, which I verified, and engaged with "abeam points". The First Officer loaded en route winds into flight plan. During loading, the First Officer remembers seeing the route with J100 from LAS to SNY while loading BCE winds. 

Shortly after passing LAS, Center asked if we were on J100. We responded we were supposed to be and checked the FMGC route. The route now showed LAS direct to SNY with no "abeam points". We corrected it and got back on course. We do not know how the route got changed. 

Less than two minutes later, the same thing happened to another air carrier flight behind us. The most likely explanation would be that one of us mistakenly inputted a "direct SNY". However, the First Officer's technique would be to program such a change including "abeam pts". They were not there. The First Officer does not remember making that input and I did not verify any such input. This makes that mistaken input somewhat less likely. I did not make any inputs. 

We are at a loss to explain what actually happened. Since the same thing happened to the next flight, at the same point, on the same route, on the same day, at almost the same time, there is a small possibility that there is a database or other FMGC glitch. This is worthy of further investigation.

Synopsis

An A320 flight crew received a reroute clearance prior to takeoff and programmed the FMGC accordingly. Inexplicably, after takeoff when ATC inquired as to their deviation from that route they found the programmed "airways" route between LAS and SNY had vanished and was now showing a "direct" route. Strangely enough, another air carrier A320 flight crew flying the same route shortly behind them reported what appeared to be an identical anomaly.

ACN: 1042617 (2109 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 1800

Environment

Flight Conditions : IMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : B747 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Landing
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1042617
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We [were] flying the LDA PRM 28R in SFO. We were put 2.5 miles behind a 747-400 who was on the ILS 28L. The winds below 4,000 FT were 180 at 15 KTS and surface winds were 250 at 10 KTS. I asked to slow due to the 747's close proximity on the approach given the southerly winds that would blow the 747's wake into our path. The Controller seemed upset that we were concerned about the possible wake encounter and said that we could go missed if we had a problem. We broke out at 1,700 FT and stayed high and flew over the wake that we could clearly see due to the moisture in the air! My issue is with the Controller not having any situational awareness in regards to the 747 and the winds aloft considering the close proximity of the aircraft and these dangerous approaches. Smaller aircraft should be still given 5 miles behind a heavy on this approach considering how close the 2 runways are (750 FT)! This is an accident waiting to happen!

Synopsis

A320 pilot reported he was upset with SFO Controller's technique in not providing enough margin to a preceding B747 wake turbulence.

ACN: 1042521 (2110 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : BWI
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : BWI

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1042521
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The confusion began when we were given instructions to "line up and wait" on Runway 33L. A heavy had just landed and was rolling out long and we were taking our position on the runway as cleared. Additionally, there was an inbound aircraft on an approximately 4 mile final at this time, and it was obvious the Controller was attempting to speed our departure along in an effort to prevent a go-around by the inbound aircraft. Next, the Controller issued departure instructions to us while we were in position, which we believe included a heading and something regarding contacting Departure Control. Unfortunately, the aforementioned instructions were very hurried, mumbled and quite ambiguous. Both the Captain and I were unsure of what the Tower Controller had just said; although we were certain no takeoff clearance had been issued. The landing aircraft was still on the runway. The Captain, who was the pilot not flying, queried the Controller on the aforementioned instructions. In fact, the instructions were so vague and confusing that we asked the Tower Controller if he wanted us to contact Departure at that time while we were in position. No reply was received, instead several seconds later we were cleared for takeoff as the heavy aircraft was exiting the runway. During the initial climb out, the Captain again asked the Controller to verify the heading at 250 degrees and the response we received was "150" with no turn direction included. As a result, he turned the heading indicator to 150 and we initiated a turn to the right, absent any directional instructions. We turned right since this was the shortest direction to that heading (runway heading 335). We were approximately 30 degrees into the turn and approaching acceleration altitude when the Captain asked the Tower Controller to switch to Departure frequency. The response from the Tower was to inform us the 150 degrees heading was to be a left turn. As a result, we immediately initiated a turn direction back to the left. Shortly thereafter we contacted Departure Control. We received no traffic alerts and nothing extraordinary was said on the Departure frequency regarding our initial turn to the right. The flight continued to our destination without incident. 

ATC Controller should issue clear, appropriate departure instructions at the appropriate time. Flight crew should not begin takeoff roll without being 100% certain of departure instructions, regardless of proximity of inbound aircraft.

Synopsis

An Air Carrier issued a hurried takeoff clearance from BWI was not given a clear departure vector heading and after a takeoff track deviation were given a new heading by the TRACON.

ACN: 1042351 (2111 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : Y

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1042351
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1042353

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While working on an Flight Control Data Concentrator (FCDC) #2 write-up, my partner notified me that, while in the main avionics compartment, that he noticed the Engine Interface Unit (EIU) #2 hold down knobs were loose but that the EIU was securely seated in its rack. No further info was given to me at that time and I was under the belief that he was taking responsibility to secure the loose EIU hold down knobs. After this information was given to me I continued and then completed my repair of the FCDC #2 problem. Shortly after the aircraft pushed off the gate, my co-worker asked me if I had tightened the EIU #2 hold down knobs. I told him that I did not because I was under the impression that he had taken responsibility to correct that discrepancy. He immediately informed a Maintenance Lead of the unsecure EIU #2 and requested that the aircraft return to the gate. The aircraft returned and the discrepancy was corrected and documented. The aircraft then departed with no further incident.

Narrative: 2

Crew refusal for FCDC 2 fault, previously placed on MEL 2795A, my partner and I then worked other maintenance items on 2 gates. We were given a new FCDC to install. I opened main equipment avionics door for access to FCDC 2, while my partner, and setup the cockpit for replacement. I noticed the EIU 2 was seated in the rack near the door with the hold down knobs loose. I went upstairs to let my partner know door was open and that I found the knobs loose on EIU 2. A few minutes later, I was called on the radio that 3 each seat covers were needed on another aircraft. I left to take care of seat covers. When I was finished, I met my fellow Mechanic who said Aircraft FCDC 2 was replaced and ready. As soon as I realized the aircraft was taxiing out and the holddown knobs were not tightened, I immediately contacted Company Bravo South Lead, so Aircraft X could be returned to the gate for recheck of holddown knobs. Aircraft gate returned approximately an hour later. Both EIU 2 knobs were tightened. Log item was addressed thru maintenance data and released.

Synopsis

Two mechanics report an aircraft nearly taxied out without having the Engine Interface Unit (EIU) 2 hold down knobs tightened.

ACN: 1041166 (2112 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1041166

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately 45 minutes into flight, ECAM appeared for Green Hydraulic system low pressure. We had lost all hydraulic fluid and performed the ECAM then the FM. The Captain gave me the airplane and ATC while he talked to Dispatch and Maintenance Control. The decision was made to declare an emergency and divert. The flight attendants were briefed and we reviewed the manual gear extension and performance problems for loss of Green system. Captain took control of the airplane for a normal descent and approach. [The Captain] configured the aircraft early for a visual approach. Smooth touchdown as we were at max landing weight, with a roll off the runway at the high speed. We were met by Crash Fire Rescue equipment and Maintenance with a tug. Gear pins were installed and we were towed to the gate under escort.

Synopsis

A320 First Officer experiences a Green Hydraulic system failure in cruise due to loss of fluid. Maintenance is contacted and the decision is made to divert to a suitable alternate. Uneventful landing and tow to the gate.

ACN: 1041057 (2113 of 2908)

Time / Day

Date : 201210
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 39000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1041057

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While in cruise flight at FL390, we heard a loud pop/bang and immediately saw that the right forward windscreen had completely shattered. The cracking appeared to emanate from the lower left corner of the windscreen and we could also see what appeared to be some splattering of something in that same area. It sure seemed like a bird strike, but at FL390? We felt the inside of the window and checked the pressurization and determined that it appeared that it was only the outer pane that was shattered. Within about 30 seconds of the initial sound, an ECAM message came up stating that the window heat in that window was INOP. Out of an abundance of caution, and to avoid some potential icing conditions that we could see between our position and the airport, we elected to declare an emergency and start an early descent. This also allowed us to both reduce the pressure differential in the cabin as well as slow the aircraft speed. We then proceeded to destination without further incident. Upon arrival at the gate, and after making a couple of requested calls to the Tower and the company, I was able to look at the damaged area from the outside. There appeared to be no other damage to the nose of the aircraft or any other sign of a bird strike, so I now doubt that was the cause. What the splattered material is at the base of the windscreen, and what caused it to shatter, I do not know.

Synopsis

A320 Captain experiences a shattered right forward windshield at FL390. Crew begins an early descent to reduce cabin differential pressure and declares an emergency. A normal landing at destination ensues.

ACN: 1040272 (2114 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 3400
ASRS Report Number.Accession Number : 1040272

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were on a visual approach following an uneventful flight. At approximately 2,000 FT, the Captain, who was flying the leg, asked for the landing gear extension. I lowered the landing gear and almost immediately, we starting receiving numerous ECAM messages. The Captain continued flying while I began running the ECAM sequence. We quickly deduced that we had lost all quantity in the Green Hydraulic System. After a brief analysis and discussion, we informed Tower that we would be going around. After beginning the go around sequence, we were handed off to Approach Control. The Captain instructed me to declare an emergency and inform ATC that we would like vectors to sort out the issue and accomplish appropriate checklists. We also requested an eventual return to [the long runway]. After completing all procedures, communicating with company and cabin crew, we agreed that we were ready to return for landing and were vectored for a visual approach. We informed Tower that there was a good possibility that we would have to stop on the runway due to no steering capabilities. The Captain flew the approach and landing and we touched down without incident. The Captain was able to steer the aircraft clear of the runway on a high speed taxiway. After stopping the aircraft, we shut down the engines allowing inspection by the emergency/fire personnel. They informed us everything appeared normal, and we waited for the arrival of our tow-in vehicle. We were subsequently towed to the gate without incident.

Synopsis

A320 crew experienced a Green Hydraulic System failure during approach when the landing gear was extended. A go around initiated, an emergency declared, and ECAM actions complied with before returning to land on the longest runway.

ACN: 1038967 (2115 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Descent
Route In Use.STAR : FRDMM RNAV

Component

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19500
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1038967
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Workload
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Leaving FL300 we were cleared to descend via the FRDMM STAR into DCA. The First Officer engaged the DES mode and managed speed after I set the bottom altitude in the MCP. We verified the restrictions on the MCDU and on the ND. After making the first several crossings with no problem, the First Officer and I briefed the approach. 

At this time the autopilot seemed to "go dumb" and continued descending below the next crossing altitude. The FMA still indicated DES mode engaged, but the autopilot was descending below profile. The First Officer disengaged the autopilot and climbed the jet to meet the restriction. The doughnut and the PROG page displayed correct information, but the autopilot was not maintaining the profile. 

We then reverted to the old way of setting the most restrictive altitude and the cockpit workload went through the roof as there are 15 altitude and 5 airspeed restrictions on this arrival. We attempted DES mode again and it made the altitude crossings, but would not make the airspeed restrictions even though we were in managed speed. We used selected speed to comply with the restrictions.

Upon landing, we entered an item for the malfunction and called Maintenance Control. A Mechanic reset the FMGCs and we flew the airplane again, but did not have the opportunity to use DES mode on the the following flight. I have seen this problem several times on the Airbus; the autopilot disregards FMGC profiles.

Callback: 1

The reporter advised there were no adverse wind or weather effects to complicate the profile. He stated the initial crossing restrictions were complied with in harmony with the FMGC profile. He did believe the errors began when they were programming a runway change and speculated that doing so may have overloaded the processing capabilities of the FMGs and suggested the commands to the autoflight system (vertical navigation and authothrottles) may have suffered as a result although they received no ECAM warnings to that effect. He stressed that the FMGs continued to display appropriate "descent doughnut" and PROGRESS PAGE predictive altitude displays. The only computer generated alert was an amber circle around the next waypoint displayed when their actual altitude was below the minimum crossing at that fix.

He further speculated on the processing demands of multiple crossing airspeed and altitude windows often spaced only a few miles apart which require the calculations for each approaching window to include computations of what has to be considered for each of the subsequent windows far down stream. Finally, he stressed his belief that it was the autoflight system, not the FMGs, that erred...with the caveat that the demands on the FMGs may have been such that commands to the autoflight system were compromised.

Synopsis

An A319/20 Captain reported the failure of the aircraft's FMGC and autoflight systems to comply with the altitude and airspeed restrictions while conducting the FRDMM RNAV STAR to DCA.

ACN: 1038044 (2116 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway XXL
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 1300
ASRS Report Number.Accession Number : 1038044
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Distraction
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

On downwind at least 18 miles out was being flashed by a very bright green laser aimed directly at the cockpit window. On approach with the laser blaring in our faces put the gear down to have a 2 line ECAM. AUTO BRKS on line 1 and NORM BRKS on line 2. Went missed approach at 10,000' and asked for delay vector while trying to relay the position of the laser to the tower and running the ECAM's. Went to the book and found the AUTO BRKS page which said to go to the adjusted landing distance table. Started looking for the NORM BRKS and never could find it in the book. Now running short on time and fuel with the laser still flashing us. Did not have time to call Dispatch or Maintenance and the landing distance says to add multiple failures together to get the correct required landing distance but nothing in the book for the NORM BRKS ECAM. We were WITHOUT guidance on an ECAM from our books. The only book reference was to adjust the landing distance as we have no auto brakes and no normal brakes and it had no guidance for the 2 failures. We are now test pilots. Decided to reset the Antiskid Switch even though there was no procedure calling for it! It worked and the ECAMs cleared off. After landing and talking with Maintenance it took another 20 minutes for them to figure out it was a failure of one of the BSCU computers. We had switched from the bad computer to the good computer by turning OFF and ON the antiskid switch with the gear up on the missed approach. The ECAM was misleading and totally worthless in helping to fix this issue as it said nothing about a BSCU failure. Lately however I have heard of numerous BSCU failures. The Captain had a picture of the number 1 and number 3 tires that he blew on landing with a failed BSCU. I had a total loss of nosewheel steering due to a failed BSCU that was not resettable in flight and had to have a tug at the runway to tow us to the gate just a couple weeks ago. The Mechanic said another 320 just blew some tires in landing a month ago on landing with a failed BSCU. The next day I called the Tower and delivered a street map with the exact streets where the laser was coming from so they stop the laser flasher. The Tower admitted that this was a common problem at this airport and even the Tower was lasered on a different occasion. Never before had anyone given the Tower a map with the street address so they were very happy and surprised because they were thinking it was in a different location.

Callback: 1

The Reporter stated that his Company began a series of Airbus upgrades about a year ago which has lead to numerous aircraft systems malfunctions. The BSCU is one component which appears to be malfunctioning with some regularity and the checklist does not consistently correct the problems. One recent BSCU fault which the Reporter experienced at cruise altitude was not corrected after cycling the ANTI SKID AND N/W STRG switch but after landing with no nose wheel steering, setting the parking brake and cycling the switch, the fault was corrected. During the current event cycling the ANTI SKID AND N/W STRG switch with the gear up did work. So even though a BSCU fault was indicated no BSCU FAULT ECAM was given. The Reporter also adamantly discussed the eye discomfort he experienced after the laser illumination. He was amazed at the power and accuracy of the laser beam during the twenty minutes or so as the crew was trouble shooting the anti skid problem. His eye discomfort lasted about two weeks.

Synopsis

An A320 ECAM alerted AUTO BRKS and NORM BRKS when the gear was extended for landing but no corresponding NORM BRKS checklist was found so during the go around the crew cycled the ANTISKID switch as per the AUTO BRKS ECAM which switched to the alternate BSCU and removed the ECAM alerts. During this event the cockpit was illuminated by a green laser which appeared to have a lock on and which caused eye discomfort.

ACN: 1038011 (2117 of 2908)

Time / Day

Date : 201209

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1038011
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

The left pack was selected off prior to flight to comply with the MEL deferring it as inoperative. Later, while in flight, we received an ECAM alert for a pack overheat on the inoperative pack. We were surprised to say the least. We complied with the ECAM procedures and monitored the inoperative pack and the overheat condition. 

As we were scheduled to continue on with this aircraft we sent a message to Dispatch refusing the aircraft for the next leg. On arrival to our destination, while at approximately 5,000 FT the Duty Flight Manager sent an ACARS message asking me to rescind my refusal and take the broken aircraft [notwithstanding the] inoperative yet overheating pack. As we were in sterile cockpit at the time we disregarded the message until we we're on the ground with the engines off and all checklists completed. 

I contacted the Dispatcher to see if he had authorized the ACARS message and he said "no." I then contacted the Duty Flight Manager to find out why he was pushing me to fly the broken aircraft. Then, as soon as I got off the phone with him, two local flight managers were at the aircraft to tell me to fly the aircraft. I politely spoke to them about the aircraft status and they left. 

This pilot pushing is unacceptable, especially violating sterile cockpit regulations and having two flight managers come to the aircraft.

Synopsis

An A320 Captain expressed his displeasure at being pushed by three flight managers to accept an aircraft which, on their previous leg, had received an ECAM warning for an overheat condition in the left air conditioning pack which had been previously deferred inoperative. As required by the MEL, the pack was selected off prior to and during the flight on which the overheat warning was received.

ACN: 1037005 (2118 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1037005
Human Factors : Workload
Human Factors : Communication Breakdown
Human Factors : Distraction
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

At the Captain's request I initiated a three way phone patch with Maintenance Control. My initial radio contact with the flight was very weak but readable so I completed the patch with Maintenance. After Maintenance Control came on line, however, the whole patch failed. I contacted the flight again with the same very weak but readable radio contact, brought Maintenance Control on the line but then couldn't re-raise the flight. 

The Captain then contacted me via our commercial radio link system and that connection was loud and clear. I requested that they make the patch to Maintenance Control as my phone system appeared not to be working well. They completed the initial patch successfully. However, when Engine Maintenance Control said the flight needed more input from Airframe Maintenance Control and tried to transfer the patch, there was a long silence. I finally asked the Captain if he was still there and he was. However Maintenance Control appeared to have dropped off the patch.

At this point the Captain decided he would continue to destination and the patch was terminated. Airframe Maintenance Control then advised he could hear me and the flight on the line but could not join in. While this was all going on I had to ask two other Dispatchers to assist me with my workload. The airport weather forecast had changed to require an alternate so that flights en route had to be advised and I had to add an alternate to flights not yet departed.

Callback: 1

Although the reporter provided no more "incident specific" information she did expand on the generic problems associated with their new communications system, advising that similar breakdowns in "conference" calling are regular occurrences and that pilot's frequently lose patience with the system and elect to "go it alone" rather than face the continued distractions and interruptions that fail to resolve themselves. Dispatchers find themselves frequently having to multi-task and cover for one another in terms of completing "routine" tasks while one or another is involved trying to communicate with a flight in need of assistance.

Synopsis

An air carrier Dispatcher was unable to establish a flight crew/Dispatch/Maintenance Control communication link request from a domestic flight. Ultimately, the Captain gave up the attempt due to the distraction being more of a problem than was the reason for which the patch was requested.

ACN: 1036263 (2119 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : TNCA.Airport
State Reference : FO
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : TNCA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 11
Flight Phase : Initial Approach

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : TNCA
Aircraft Operator : Air Carrier
Make Model Name : MD-80 Series (DC-9-80) Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use.Localizer/Glideslope/ILS : Runway 11
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1036263
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : NMAC
Detector.Automation : Aircraft TA
Detector.Person : Flight Crew
Miss Distance.Horizontal : 0
Miss Distance.Vertical : 200
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

It was a beautiful VFR day with sky clear. We were cruising at FL370 on as planned heading towards DUSAN Intersection. As we exited the Dominican FIR, the Captain contacted Curacao Control, with jurisdiction of Aruba, our destination. Instead of proceeding directly to DUSAN, we were cleared to fly direct to the 10 mile final fix for the ILS 11 and to descend at pilot's discretion to 9,000 FT. That's what we were expecting.

As we approached 9,000 FT and in the vicinity of 37 NM from BEA, we were switched to Tower frequency. We knew we were entering a non-radar environment. Tower was overseeing clearance, Ground, Tower and Regional Approach all on one frequency. The Tower frequency was very congested and you could tell the Controller was very busy yet was doing a good job. When he acknowledged our frequency check-in, he informed us that an MD80 had just reported inbound on the ILS and 15 miles out from BEA and that we were number two behind him.

We overheard conversation between the Controller and the MD80 but couldn't understand because they were speaking in Spanish. The Controller asked us our groundspeed and DME from BEA VOR and the Captain replied about 35 DME at 330 KTS (I can't remember exactly). At no time was the MD-80 on our Nav Display with the transponder traffic knob in the "look down" position. The Controller asked us to slow down for separation and I slowed the aircraft to 250 KTS. At this point we were just within 30 miles out from the field. In the meantime, several ATC exchanges with other aircraft occurred. 

At this point we were getting high so I slowed the aircraft to 150 KIAS with flaps at two. We flew in this configuration for two to three minutes and were approaching the 10 mile final fix on the ILS approach. The Captain repeatedly asked for lower and we were given 6,000 FT. I commanded the gear down and flaps 3. As briefed earlier, I turned all automation off including autothrust and started the descent manually to get a more prompt response and so as not to be too overwhelmed with automation. We were now turning inbound 10 miles from the BEA VOR. Visibility was excellent, there were no clouds in the vicinity of the airport and the airport was in sight the entire time. Although we were high, there was no doubt that we would be stabilized well before 1,000 FT AGL.

As we approached 6,000 FT we were cleared to descend first to 4,000 FT, then eventually to 2,000 FT MSL, but were not cleared for the approach. At no time did I have to level off. It was a gradual and stabilized descent intended to capture the glideslope. Between 2,000 and 1,500 FT the Captain--leaning towards his left window as if he was looking down at a boat--suddenly shouted "break right - break right, now!" while pointing his finger down. 

I knew right away he meant there was traffic directly below us so I banked aggressively to the right, added significant power, pitched up and leveled off. Immediately, we heard the caution "TRAFFIC - TRAFFIC." I was not looking inside so I don't know what the lowest separation was but after my maneuver the TCAS showed -800 FT. At no point did I see the traffic. 

When I was established in the turn, the Captain advised ATC we just had a near mid-air collision within 200 FT of the MD80 and we were abandoning the approach. ATC cleared us to continue the turn and rejoin the approach after which we were cleared for and landed uneventfully.

It was clear to us that the MD80 flight crew clearly lied about their position from the VOR so as to be number one for the approach. There is no way we would have been able to catch them since we were around 30 miles out slowing to 150 KTS indicated and still heading directly to the 10 mile final fix on the ILS. Furthermore, I strongly believe that the pilots of the MD80 intentionally turned off their transponder which would explain why we never had an advisory but then suddenly received a "Traffic" caution at low altitude (I think the RA was inhibited because of our altitude) so as to disguise their real distance from the VOR and to only turn it back on at the last minute. I am obviously outraged about what happened and believe that it was not merely unsafe but criminal to act like this and jeopardize the lives of more than 300 people in order to shave off three to four minutes of flight time.

Synopsis

An A-320 flight crew experienced an NMAC with an MD-80 during initial approach to TNCA. The reporter alleges the MD-80 flight crew purposely mis-stated their position and shut off their transponder to gain an arrival advantage.

ACN: 1035981 (2120 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1035981
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Dispatch
Detector.Person : Other Person
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Environment - Non Weather Related

Narrative: 1

Approximately 30 minutes after departure, the flight experienced multiple failures associated with loss of FMGC 1. The Captain requested a phone patch with Maintenance. The SELCAL frequency was 128.9. The Captain had difficulty hearing Maintenance so Dispatch requested ARINC to patch the flight to Dispatch. During the 2nd phone patch, the Captain was unable to speak to Maintenance numerous times. ARINC said the background noise from our office was blocking out the crew's ability to transmit. After asking a group of people sitting close to Dispatch to quiet down, the Captain was able to communicate better, but the room noise was still so loud that constant communication could not be maintained throughout the patch.

Synopsis

An A320 Dispatcher reported that the human generated noise in the dispatch office was so high that he had difficulty on a phone patch with a flight discussing an FMGC issue and so he demanded people sitting close by be quiet.

ACN: 1035964 (2121 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1035964
Human Factors : Situational Awareness
Human Factors : Fatigue
Human Factors : Physiological - Other

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

[I] called in fatigued for a field standby. [I] had just flown a 2 day trip; early departure both days. The layover on the West Coast resulted in very limited sleep. I slept possibly 3 hours. Two major factors affected my sleep that night. One was the air conditioning system; it would not keep the fan running as programmed. Every time it turned on it "clunked." Also had neighbors who routinely entered and exited their room several times until just before my wake up time. They were also extremely noisy. I tried to compensate by wearing ear plugs for both issues, but the end result was still no sleep. The next morning I compensated for fatigue with coffee. I had a 3 hour 28 minute layover at the first destination. But due to the coffee and substandard rest facilities in our operations area, I was unable to get any rest with a nap. I completed the trip and went home. Shortly after arriving home and right before bed I was contacted for a late morning field standby on what would be my day of duty. After the assignment I made the preparations for driving back to the airport in the morning. That night I was still wound up from the coffee I had during the day (I don't normally drink coffee) and had trouble unwinding. After about 3 total hours of fitful sleeping I made the decision to call in fatigued just before morning twilight.

Synopsis

An A320 First Officer called in fatigued for airport standby on his third duty day because on the previous two nights he was not able to get fitful rest.

ACN: 1035754 (2122 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 8000

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1035754
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We experienced multiple failures that most likely stemmed from a primary failure of FMGC Number 1. Below is the chain of events. Based on the time stamp in the "last leg maintenance report." We first experienced a data hit: "Invalid Disk/MDDU." I noticed that the MDDU panel light came on/lit up. I thought it said, "Ready," then, "Invalid Disk." After obtaining an updated ATIS a little less then a hour before arrival, Captain changed the runway on the Number 1 FMGC, from Runway 1 to Runway 19. After the insert key was pushed, the Captain's map was lost and FMGC Number 1 froze up. The message in scratch pad said to wait. FMGC Number 2 displayed "Independent Operation" in the scratch pad. The FMA's displayed 1FD2, and remained so for the duration of the flight. We referred to the Flight Manual and the system reset procedures. We then attempted to reset the Number 1 MCDU CB on the overhead Panel 49. This procedure restored the ACARS functions but not the FMGC. It appeared that we could only get the status page for the FMGC. Thus, no other buttons such as RAD/NAV etc would function. Note: We subsequently received a "GPWS TERR" ECAM and secured it per the ECAM procedure. Later we attempted a second attempt to reset the Number 1 MCDU which also failed. Maintenance was notified via ACARS of the situation. We also briefed the NAV/ILS tuning to ensure we would have the ILS information displayed on both sides. Note: Prior to LOC intercept the First Officer was flying with the autopilot on. When we started to intercept the final approach course the autopilot began to turn right vs. left on the intercept. In other words, our raw data was showing that we needed to turn opposite of the FD course information. We were getting ambiguous information at a critical time on both sides. With the runway visually acquired, the First Officer turned his FD off and followed raw data. On final about 1,500 FT AGL we received a "landing elevation fault." The system status panel showed the elevation was zero, Captain elected to wait till after rollout to deal with CAB PR ECAM and focused on the final checklist and SOP call outs. When managed speed selected on final, we did not have a "speed bug" on the tape. We flew the remainder of the approach without approach speed bug reference and we used the adjusted VLS plus 5 KT speed that was displayed in the MCDU, about 140 KTS. On landing rollout, the pressurization outflow valve failed closed and a CAB PR ECAM was displayed and directed us to manually open the outflow valve, which we did. Once fully open, the ECAM cleared and we taxied to the gate. Upon landing, Maintenance was briefed in detail. Maintenance told me they have received reports of bugs since the last FMGC software update was installed. I was informed that this should be corrected on the next update cycle. Maintenance also said that based on the data he downloaded from our inbound flight; it appeared we actually experienced a dual FMGC failure or full failure of Number 1 and a partial failure of Number 2 FMGC. The multiple failures of cabin pressure, etc compounded our workload at a very critical time and not to mention at the end of a red eye flight. Good review of standby navigation procedures and anomalies.

Synopsis

An A320 crew was notified after flight that the FMGC failure they had experienced in flight is believed caused by a "bug" in a recent software update.

ACN: 1035219 (2123 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 9200
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 1035219

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 2900
ASRS Report Number.Accession Number : 1035223

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through FL235 Captain noticed high oil temperature on Engine 2 followed quickly by ECAM to shutdown Engine 2. Led to overweight landing on return to the airport (approximately 153,000 LBS). Engine 2 showed 10 gallons [quarts] at shutdown. Uneventful approach and landing made by Captain.

Narrative: 2

Engine 2 High Oil Temperature ECAM annunciated at FL240 climbing to FL320 necessitating engine shutdown, declaring emergency, overweight landing at 15,4000 LBS. Uneventful landing on runway.

Synopsis

A320 flight crew experiences high engine oil temperature climbing through FL235 which results in an engine shut down and return to the departure airport after declaring an emergency.

ACN: 1035032 (2124 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : Marginal
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1035032
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after passing 10,000 FT in our climb, the Lead Flight Attendant called to inform us of a strong electrical smell in the forward galley area. Our conversation included old food in the ovens or coffee that had spilled onto the hot pad. In the cockpit, we only had a faint smell of cheap perfume or a weak solvent. The consensus was that the odor was dissipating after a few minutes, but 5-10 minutes later the Lead Flight Attendant called to say that she and 2 other flight attendants were experiencing respiratory distress (burning throats, irritated nasal passages) as well as irritated eyes. The First Officer and I didn't notice any irritation, but elected to go on oxygen as a precaution and declared an emergency and diverted back to the departure airport. The Lead Flight Attendant was cared for by a paramedic/passenger and then was cared for by paramedics that had met the aircraft at the gate. I don't know what caused the event, but the number one engine had been replaced and this was the first revenue flight for that engine. The IDG oil cooler on that same engine was also replaced during the same maintenance session. When cabin odor reports are submitted, an extensive checklist is followed to try to isolate the source of the odor. Why not use the same checklist to inspect the aircraft any time a piece is replaced that has a direct contact with the pneumatic system or air recirculation. One step further would be a requirement for a test flight.

Synopsis

An A320 Lead Flight Attendant became ill after a strong electrical smell developed in the forward galley during climb, an emergency was declared and the flight returned to the departure airport.

ACN: 1034997 (2125 of 2908)

Time / Day

Date : 201208

Place

Locale Reference.ATC Facility : ZNY.ARTCC
State Reference : NY
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZNY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : BOJID ONE
Airspace.Class A : ZNY

Component

Aircraft Component : FMS/FMC
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7644
ASRS Report Number.Accession Number : 1034997
Human Factors : Distraction
Human Factors : Human-Machine Interface

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On the BOJID ONE arrival we were cleared out of FL330 to cross COFAX at FL240. We set the altitude in the FCU and the restriction in the FMGC. I pushed to initiate the descent and failed to verify the FMA's [Flight Mode Annunciator] that we were indeed in the descent, the First Officer noticed as we checked in with a frequency change and I took the autopilot off, autothrust off, and immediately attempted the crossing restriction but failed to do so as the First Officer simultaneously got relief from the Controller and vectors for the descent. We were inadvertently distracted by our conversation in the cockpit.

Synopsis

A320 Captain reports failing to verify that the FMA shows that descent has been initiated after being assigned a crossing restriction on the BOJID ONE Arrival to PHL.

ACN: 1034953 (2126 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1034953
Human Factors : Physiological - Other
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was working Local West when the City vehicle called and asked for a runway inspection of Runway 15L, I gave the City vehicle permission to proceed onto Runway 15L. I had an arrival to Runway 15R and verified with the pilot that he was indeed lining up for Runway 15R. The pilot verified that he would be landing Runway 15R. I also had tagged the City vehicle on the ASDE-X. I picked up the visual aid to place on the strip podium but the visual aid fell out of its protective sleeve. I started to try to fix the visual aid while someone tried engaging me in a conversation that I tried to stay out of but did answer specific questions. I dropped the visual aid again while trying to fix it. It fell on the table below and then my attention was directed elsewhere at taxing traffic. Looking ahead to the taxing traffic the visual aid that I was trying to repair was now in the wrong position, as I was also looking for tape that might be lying around to fix it. I then again felt just fine scanned the runway and did not notice anything as an aircraft was ready for take off. I think the City vehicle tag had dropped off of the primary target, something that I have heard of it doing before. I then cleared the aircraft for take off. Shortly after the aircraft began take off role the City vehicle called clear of Runway 15L at [Taxiway] WL. After the aircraft had gotten airborne he reported that the vehicle might not have been clear of the runway when he was rolling but didn't sound sure. I verified the City vehicle was clear and proceeded with the operation until I was relived shortly after. I am beside myself and have a hard time believing I allowed myself to get distracted with the visual aid repair and then forget to get a report from the City vehicle that he was off the runway. I should not have tried to fix the visual aid on my own and should have used a different reminder in its place. I am not sure how relevant the brief period of nausea was but it may have caused loss of focus. I wanted all circumstances to be known as to why this mistake happened.

Synopsis

Tower Controller described a ground conflict event when clearing an aircraft for departure without making sure that an airport vehicle on the runway had cleared.

ACN: 1034627 (2127 of 2908)

Time / Day

Date : 201209
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DFW.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : DFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1034627
Human Factors : Fatigue
Human Factors : Time Pressure
Analyst Callback : Attempted

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Taxiway
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

As I began to check NOTAMs, I was interrupted by a passenger with a weapon carry form (LEO). There were over 20 pages of NOTAMs for DFW alone. I missed the NOTAMs referring to the closure of exit points for Ramp E. There is a note on the ramp diagram that exit 45 is closed at that time of morning. We exited via point 48. Enroute, I re-reviewed the NOTAMs as I normally do. I then noticed the DFW ramp exit closure NOTAM exit 48 was closed. (No cones or barricades) ATC didn't mention our exit point. There were so many pages of NOTAMs, I just missed it. The previous day was long (12:55 duty) with an early report. There was no good time to get food during the day. I ended the day hungry and tired. The pairing was built with a reduced rest and an early report time (XA:45am van). I had a granola bar for breakfast, which was the only thing available (provided by the hotel) and insufficient. With the interruption during my review of the paperwork, I missed the NOTAM.

Synopsis

Air Carrier Captain expresses concern regarding the number of NOTAMs for DFW combined with long duty day which results in missing a Notam pertinent to his flight.

ACN: 1034156 (2128 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1034156
Human Factors : Situational Awareness

Person : 2

Reference : 2
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 1034157

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : MEL

Narrative: 1

While troubleshooting [an Airbus 320] Yellow [Hydraulic] System accumulator [deferral], it was found to be not accomplished properly. I then began to test the system per AMM 29-13-42 and at approximately 1,820 PSI, as I was looking at the pressure gauge, something in the system let loose and sprayed me in the face and side. I was able to get out of the area without any damage but could have been much worse. What we ended up finding out, not only was the accumulator damaged due to not doing the MEL procedure properly, it caused damage to the yellow system reservoir, yellow system pump, and check valve.

Narrative: 2

I was assigned to troubleshoot an aircraft's Yellow Hydraulic System deferral to determine if we needed an accumulator. Since this was MEL item we checked to see if it was complied with. It was not complied with. We proceeded to troubleshoot yellow system and charged yellow accumulator to see if it would hold pressure, while charging to target 1,850 PSI at 1,820 PSI the safety valve let loose approximately 2 gallons skydrol while fellow Mechanic was in the charging bay, he was doused with skydrol as a result. It was clear that the accumulator internal bladder had been previously ruptured resulting in 0 pressure, further troubleshooting resulted in finding the yellow system reservoir had been over serviced, we attempted to relieve head pressure air and approximately 0.5 gallons of skydrol was blowing out of reservoir air pressure vent valve. At this point it was clear that the bladder on the yellow reservoir was also ruptured due to skydrol in the reservoir air system. We informed Management of our findings and removed the yellow system reservoir and that concluded our shift.

Synopsis

The reference A320 Minimum Equipment List procedure called for the high pressure pneumatic line to be disconnected from the reservoir to prevent pressure from acting upon the system. The technicians reported that this line was found to be connected, which may have caused additional damage.

ACN: 1034005 (2129 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X

Component : 2

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1034005
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Other Person
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

While waiting to board the arriving A320 aircraft in ZZZ to operate flight to ZZZ1, FAA Inspector indicated that he rode the jumpseat from ZZZ1 and that an excerpt from the Aircraft Maintenance Manual (AMM) was in the back of the logbook regarding Task 32-42-27-210. This appears to be a job card to "Check Heat Pack Wear indicator" (Parking Break Applied) and has a hand written highlighted notation on the top indicating, "Aircraft missing # 4 Brake Wear Pin, see Page-3 for note saying it's OK". The note on page three says, "NOTE: the brake units usually have two wear pins. If you find a brake that does not have the two wear pins: -the brake is serviceable if there is one wear pin and the wear pin is in the limits. You must deactivate the brake if there is no wear pin. (reference AMM TASK 32-42-00-040). 

My impression is that this Note on Page-3 indicates that it's still 'OK' to perform the Inspection Task per the approved AMM and has no bearing on accepting an aircraft as airworthy prior to flight with a Brake Wear Indicator pin missing. Normally the CDL would be consulted for continued operation with a missing part but no listing is indicated there, nor the MEL, nor the NEF, and the AMM excerpt in the back of the Logbook does not constitute an Engineering Authorization (E/A) as per FOM Volume-1. FAA Inspector indicated that in his conversation with the PMI, every Captain should be entering a Maintenance discrepancy in the logbook since no relief is given per our Air Carrier approved procedures for continued operation with this missing item. I contacted Maintenance Control to indicate I would be entering the discrepancy before operating flight to ZZZ1. The Maintenance Controller seemed a little annoyed but said that he would arrange for a mechanic to address the write-up prior to departure. 

I cannot tell from the prior logbook pages when this became an issue, but the prior Captain said he thought it has been more than a week. I would concur, since no indication was found in the logbook pages present in the logbook indicating a discrepancy regarding the missing brake wear indicator pin. Did the Maintenance Department find this missing part or did a pilot find it on the exterior inspection? And how does the aircraft continue to be dispatched in this manner? And does the AMM have any bearing on the continued operation of the aircraft? 

[Recommend] properly applying Maintenance and Flight operations procedures for the continued airworthiness of the aircraft and answer the following questions: When did the Maintenance Department find this missing part or did a pilot find it on the exterior inspection? And how does the aircraft continue to be dispatched in this manner? And does the AMM have any bearing on the continued operation of the aircraft in this manner?

Synopsis

An FAA Inspector informs an A320 Captain he should be entering a Maintenance discrepancy in their logbook since no relief is given per their air carrier approved procedures for continued operation with a missing Brake Wear Indicator pin.

ACN: 1033973 (2130 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZDV

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : B747 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZDV

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1033973
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft TA
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

En route at FL350, we requested a climb to FL370. The Controller never responded for about 30 seconds, and then assigned us FL370 and to be there in 2 minutes. This didn't give us time any time to decide. We started climbing. I saw an aircraft on TCAS 1,000 FT higher than us, but I couldn't determine if it would conflict with our path and climb. After climbing 1,600 FT in about 1 minute 50 seconds we got a TA on the traffic, a B747 at FL360. He queried the ATC Controller about us. I then reported that we had the traffic in sight. We continued climbing to FL370 and after the 2 minutes had expired, we were 200-300 FT low with no extra energy to increase the climb rate. The ATC Controller never made any comments, but I felt a less than friendly attitude from him. 
I know that normally, it takes 3 minutes at high altitudes, to climb 2,000 FT typically, when requesting an en route climb; the ATC Controller will ask us how long it will take, if there is conflicting traffic. This Controller didn't ask. If he had, we would have told him that we needed 3 minutes. However, he never gave us any time to discuss this. We should have denied the climb, but we thought that we could make it in time, if we traded speed for altitude. The Controller never should have put us or the other aircraft, in this position. Jet aircraft at high altitude, reach a point that speed decreases to the point that climb is not possible, until energy increases again. I don't think that the Controller, forcing the issue in this case, knows this. I did, and should have refused the climb until clear of any conflict.
Always have at least 3 minutes to climb 2,000 FT at cruise altitudes. Don't let an ATC Controller force you into a "coffin corner" box.

Synopsis

A320 flight crew receceived a TCAS TA while attempting to comply with ATC instruction to climb from FL350 to FL370 in two minutes.

ACN: 1033456 (2131 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuselage Panel

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1033456
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

This is the fourth time in six weeks (an unacceptably high rate) that I have found speed tape covering a bad belly fastener on an Airbus [320]. Someone in Maintenance found a bad fastener, could not fix it for some reason, and then did not document. More disturbingly, in three of the four cases the fastener could NOT have been deferred per the AMM [Aircraft Maintenance Manual]. The fasteners were at the corner or [adjacent to the] corner position of large belly panels.

Synopsis

An A320 First Officer reports finding several incidents of undocumented maintenance regarding speed tape covering a bad fastener.

ACN: 1032852 (2132 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Cockpit/Cabin Communication
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Cabin Address System
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Interphone System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1032852
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During a normal push back and engine start the flight attendants did the passenger safety briefing. Shortly after the briefing the Flight Attendant Panel (FAP) began to act up. The flight attendants could not communicate via the cabin inter-phones or the PA. The cabin lights and emergency exit lights began to blink off and on. Unknown to the flight deck crew, we could not communicate with the cabin either. The D Flight Attendant came forward in the cabin to communicate with the forward flight attendants and the cockpit by alternate means. When she got to the forward galley we began the takeoff roll. There were no passenger seats available so the D Flight Attendant sat on the floor against the bulkhead in the forward galley until we were airborne. We had a short taxi to the runway. The First Officer (pilot not flying) made his PA to the flight attendants to prepare for departure and we were cleared for takeoff. We lined up on the runway, transferred control to the First Officer (now pilot flying) and the takeoff roll went normally. 

After takeoff, the flight crew got an ECAM for "SMOKE Lav Det Fault." We ran the ECAM and checked FOM. This is a crew alert ECAM and refers the crew to the reset procedures in the FOM. Although there is a reset procedure and it says it is permitted in flight, the reset procedure specifies "on the ground" only. As we finished the ECAM procedures, we were interrupted by a knock on the cabin door. A pilot was on the jump seat and asked if I wanted to let them in. I recognized that the lav smoke detectors may be some sort of distraction so I said, "No" and tried to call the cabin via the interphone. There were no "dings" and no answer. I tried again, still no answer. I was concerned about a possible fire that was undetected so we let the B Flight Attendant in. She explained what had happened with the FAP and the cabin/exit lights. I explained that we had a smoke detector fault (not an indication of a fire) and asked her to check the lavatories for smoke, heat or any indication of fire. I told her while she did that we would try to reset the communication system (CIDS reset). We did the CIDS reset and the A Flight Attendant came back to the flight deck after the lav check and reported that there was no indication of a fire. I told him the reset procedure may take up to 4 minutes and to report back to us whether it was successful or not. We discussed alternate means to contact each other in the event that the reset did not work and he left the flight deck. We got a call from the cabin about three minutes later that everything was working normally. The reset also cleared the SMOKE Lav Det Fault. I entered the reset into the aircraft logbook. This event went well, all the crew members doing the best they could under the circumstances. My only concern was having the flight attendant on the floor for the takeoff. Immediate action by the flight attendants MAY have helped avoid taking off with the D Flight Attendant away from her station but they acted quickly as they identified the problem and would have had time to contact the flight deck before takeoff if the taxi was not so short.

Synopsis

A320 Captain describes CIDS (Cabin Information Display System) fault that renders all intra aircraft communications inoperative along with emergency exit lights blinking. This is discovered by the flight attendants during taxi out but they are unable to relay the information to the flight crew prior to takeoff. Once airborne the Captain is made aware and a CIDS reset cures all the problems.

ACN: 1032786 (2133 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BOI.Airport
State Reference : ID
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BOI
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : BOI
Make Model Name : Cessna Stationair/Turbo Stationair 6
Crew Size.Number Of Crew : 1
Flight Phase : Landing
Airspace.Class C : BOI

Component

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1032786
Human Factors : Situational Awareness
Human Factors : Fatigue
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Facility : BOI.Tower
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1032171

Person : 3

Reference : 3
Location Of Person.Facility : BOI.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1032178

Person : 4

Reference : 4
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 1900
Experience.Flight Crew.Last 90 Days : 280
Experience.Flight Crew.Type : 2800
ASRS Report Number.Accession Number : 1032785

Events

Anomaly.Ground Incursion : Runway
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After pushback we were given clearance by BOI Ground Control to taxi to Runway 10L via Taxiway A and then Taxiway W. As we taxied out we did not receive the final weight manifest from Load Planning in the normal time frame so we performed the before takeoff checklist "down to" the Manifest Changes line. As we continued to wait for the final weights from load planning we began approaching the intersection of Taxiway A and Taxiway W. 

As we approached the above intersection I went "heads down" to switch the aircraft radio to Tower frequency and to enter the "address" for Load Planning into ACARS in regards to not receiving the final weights. After performing the two above tasks, I looked up and noticed that we were passing over the hold short line to Runway 10L at Taxiway A and Taxiway J. We traveled a short distance past the line and then the Captain stopped the airplane. We then advised BOI Tower that we had passed the hold short line for Runway 10L.

Narrative: 2

The A320 stopped well past the hold line. As a result a Cessna on approach was sent around to avoid landing over the top of the Airbus. I was CIC and noticed the event simultaneously with the Controller who was being relieved at the time and prompted the assuming Controller at Local Control to send the Cessna around.

Narrative: 3

This event is directly attributable to the taxiway layout and design of that area of the BOI airport. Also, pilots from this airline are consistently waiting for their numbers while taxiing. Perhaps if they would obtain their numbers prior to taxiing, there would be less of a distraction in the cockpit.

Narrative: 4

Third late night in a row contributed to fatigue factor.

Synopsis

When an A320 flight crew was distracted by receiving their final weight and balance numbers from their company they inadvertently crossed the hold short line of the runway to which they had been cleared. ATC directed a Cessna on final approach to go around due to the incursion.

ACN: 1032599 (2134 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Relative Position.Angle.Radial : 090
Relative Position.Distance.Nautical Miles : 2
Altitude.MSL.Single Value : 500

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 6500
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 200
ASRS Report Number.Accession Number : 1032599
Human Factors : Distraction
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 95
Experience.Flight Crew.Type : 400
ASRS Report Number.Accession Number : 1032600
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

Descending toward the Bay Area, we picked up the latest ATIS. As per the previous ATIS, I had selected the ILS 28R approach in the database, but the current ATIS advised to expect visual approaches to Runway 28L and 28R. I was not familiar with the San Francisco approaches and began reviewing the visual approaches available. I was just asking the Captain which approach he thought we would get when we got a descent assignment as well as an approach assignment. I put the approach into the database and began looking at the approach plate. As we descended through 18,000 FT we set our altimeters to the current altimeter setting and I mentioned the descent checklist, but didn't call for it as I knew it required an approach briefing. I read through the approach plate and began to brief it but was interrupted and distracted several times by step down altitude assignments, speed assignments, and traffic call outs for the parallel runway. My briefing for the approach came late and in a hurried manner. Aircraft configuration for the approach as well as the descent and approach checklists also were called for and completed late. There was a 777 on a parallel approach off to our left, and a B737 aircraft being cleared for takeoff on an intersecting runway. Our landing runway was in sight, but with all the distractions and contributing factors above I descended below 1,000 FT before being fully configured and below the glideslope. Descending through 2,200 FT, I was still above profile, flaps set at two or three, and had the speed brakes extended. The altitude set in the FCU was 1,800 FT, the lowest altitude at the end of the approach. As I didn't want the aircraft to level off at 1,800 FT, I selected 100 FT (field elevation) in the FCU altitude selector for the visual approach. My Captain was just questioning my altitude when we received a low altitude alert from the Controller. This was approximately 2 miles from the threshold and approximately 500 FT AGL. I immediately disconnected the autopilot and arrested the descent. I then re-intercepted the glide slope and continued down it to an uneventful landing. We considered going around but had parallel traffic on the left, departing traffic off to the right, terrain straight ahead, and our long landing runway in sight straight ahead. The best solution would have been to ask for an expected approach long before we finally received it. This would've helped provide the time I needed to review and then brief the approach. Because I am much more familiar with them, if we would've been assigned an ILS approach, probably none of this would ever have happened. I take responsibility and should have been better prepared for an unfamiliar airport and its approaches. There always are and always will be step down altitude assignments and traffic callouts that can be distracting and we must contend with. Thereafter, when I realized the descent and later the approach checklists were going to be delayed I should've asked for vectors around for another approach. Realizing this and requesting it early would have probably minimized any errors. The biggest mistake I actively made was setting the FCU altitude to field elevation instead of 1,000 FT above field elevation. If I would've done this, a 1,000 FT floor would have been set. It would have alerted us that I needed to get fully configured before continuing down, as well as precluded us from getting a low altitude alert. Earlier, more proactive, and more thorough preparation would have prevented the above situation.

Narrative: 2

[Narrative #2 contained no additional information.]

Synopsis

An A320 First Officer, unfamiliar with the SFO FMS Bridge Visual, was not expecting the level of work required to stay on profile and descended to 500 FT on a two mile final before a Low Altitude alert reminded the crew to return to the profile.

ACN: 1031510 (2135 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 400

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 8500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3621
ASRS Report Number.Accession Number : 1031510
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was flying pilot. On initial climb out at about 400 AGL, the Captain announced that he had lost his attitude display (on the PFD). At the same time I lost my flight director and the autothrust failed. The Captain told me to continue flying while he did the ECAM. The ECAMs that displayed were: NAV ADR 1 fault, NAV GPWS fault, and Autothrust Failure. I tried cycling my flight director on and off but it did not return. I tried to engage the autopilot two and autothrust but neither one worked. The Captain was confused by the ADR 1 ECAM and kept switching AIR Data Switch to First Officer on 3 instead of Captain on 3. Once we got the switch in the correct position, he regained his display. We advised Departure and later Center that we were not RVSM capable, did not have RNAV capability, and may have to return. I advocated a return, but because we were over landing weight by about 7,000 LBS at that point, the Captain elected to continue for the time being. We noticed that ADIRU 1 [Air Data IRU GPS] was in the off position. This was very strange since neither one of us had touched it. The Captain contacted Dispatch and asked to phone conference with Maintenance Control. Meanwhile, I flew and handled VHF 1. The call got cut off, and he had to reconnect. Dispatch ran the fuel burn for FL270 and found that we had adequate fuel for the flight to continue to destination. The decision was made to continue. The Captain looked up the ECAM in the Airbus Flight Manual. It took him about ten minutes (and later me five minutes) to find the procedure in the book. We could not see any reason why the flight directors, autopilots, and autothrust did not work. He called the flight attendants and advised them that we were at a lower altitude and that they may get a few questions from passengers. 

After he returned, he cycled his flight director off. When his flight director was cycled to the on position, the flight directors, the autopilot, and autothrust systems were restored. Nowhere in the flight manual (that we could find) does it say to do this, and nowhere in the NAV ADR 1 fault does it say you could lose flight directors, autopilots, or autothrust. I had been hand flying for about 40 minutes when the autopilot was finally engaged. We discussed the sequence of events during the initial climb out. The Captain mentioned that just after rotation he repositioned his sun visor. I remember working with another Captain a few months back that told me that he accidentally shut off ADIRU 1 by bumping it while adjusting his sun visor. Since the system was working fine on taxi out and the timing matched, that seemed the only logical explanation for what had happened. We continued without incident. On a side note, I was an Airbus pilot instructor for nine years. There have been a few occasions where I have had to reference the new flight manual for ECAMs and other non-normal procedures. Although the new flight manual format for non-normal and emergencies probably works very well for the Boeing fleets (especially the 737), for the Airbus this new format is not as intuitive. It would have taken me 30 seconds to find what I needed in the old manual (without even looking in the index). With the new format it took me five minutes! Had this happened to a less experience crew, there is no doubt in my mind that given more extenuating circumstance that this could have resulted in a major incident!

Synopsis

A320 First Officer experiences autothrust and flight director failure shortly after takeoff while the Captain's attitude display disappears and the autopilot will not engage. It is discovered later that the rotary switch that controls ADIRU 1 is in the off position. Once the Captain's ADR switching is placed on ADR 3 and the flight directors are cycled, all systems become operational minus ADIRU 1.

ACN: 1031425 (2136 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 400

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3621
ASRS Report Number.Accession Number : 1031425
Human Factors : Confusion
Human Factors : Distraction
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I was the flying pilot. At about 400 AGL on initial climb out from Runway 25 the Captain announced that he had lost his attitude information on his Primary Flight Display. At the same time I lost my flight director and the autothrust system. He told me to continue to fly while he worked the check list on the ECAM. After some initial fumbling with the transfer switches, he regained his PFD by switching to the stand by system. We advised departure (and later Center) that we were not RNAV or RVSM capable due to a temporary navigation problem. At that point air traffic control gave us vectors.

We noticed that the number 1 Air Data Inertial Reference System (ADIRS) was in the off position. The ADIRS controls barometric altitude, flight path vector, airspeed, Mach, angle of attack, temperature, overspeed warnings, heading, track, speed trends, VSI, groundspeed, and airplane attitude. This explained why we lost all of the Captain's information, but it did not explain why the number 2 flight director, both autopilots, and the autothrust systems did not work. ADIRS 1 should have been in the NAV position, and neither one of us had touched it since the initial cockpit set up at the gate. I tried repeatedly to restore my flight director, the autopilot, and the autothrust by cycling the switches on and off, but none of the systems returned at that time. The Captain then contacted Dispatch and Maintenance via radio number 2 as I continued to fly and handled ATC on radio 1. 

After some discussion and evaluation, we determined that we had adequate fuel and navigation capability to continue to our destination at FL270. The Captain then advised the flight attendants of our situation. None of us could understand why the flight directors, autopilots, and autothrust were not working. Our company had no answer. The flight manual made no mention of this happening with an ADIRS failure.

I have been on the Airbus for thirteen and a half years. Nine of those years I was a simulator instructor in addition to being a First Officer. In the simulator we often times cycled both flight directors off and that would reset them. We tried it and it worked! Both flight directors, both autopilots, and the autothrust system were restored. I had been hand flying for about 40 minutes. Autopilot 2 and the autothrust system were engaged. 

The Captain and I then went through a sequence of events on initial climb out to try to understand what had happened. He said that after rotation the sun was in his eyes, so he repositioned his sun visor. The sun visor in the Airbus is about 18x10 inch tinted hard plastic and is hinged via a ball socket above the pilots head. I remembered talking to another captain a few months earlier who told me that he accidentally turned off ADIRS 1 by bumping it with his sun visor while repositioning it. The ADIRS system is supposed to have a detent to prevent this from happening, but clearly it did not work. The timing of our ADIRS 1 failure matched the Captain repositioning of his sun visor, and it was the only explanation that we could find for the ADIRS 1 to go from the NAV to the Off position. We continued the flight uneventfully to destination.

Synopsis

An A320 flight crew suffered the loss of multiple systems associated with the Air Data System shortly after takeoff. A comprehensive review of switch positions discovered the Captain's ADIRS was in the off position despite having been on during flight preparation and the initial takeoff. They suspect the Captain's adjustment of his sun screen immediately prior to the event may have struck the switch and turned the ADIRS off. All systems were eventually restored and the flight continued to destination.

ACN: 1031251 (2137 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 23000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZAU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : McDonnell Douglas Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZAU

Person : 1

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1031251
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1031257

Person : 3

Reference : 3
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1031254

Person : 4

Reference : 4
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 1031255

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was working another position when my Supervisor instructed me to go to the Badger D-Side. Upon plugging in my headset and glancing at the RADAR scope, I noticed that the sector was fairly busy. I decided to forgo the briefing, and simply ask if anything needed immediate action. A few point outs and coordination items were completed by me. Eventually a few minutes passed, and we received an aircraft from Sector 27 that was climbing westbound, roughly out of FL240. At the same time, we had an A320 routed Petty and points eastbound, climbing to FL230. At the time the aircraft were approximately 20 miles apart and my RADAR Controller decided not to climb the A320. The A320 was still out of FL180, so I estimated that there would be no need to call the lower sectors to inform them that we were not climbing the A320. A few moments passed, and now the A320 was approaching FL230, roughly out of FL220. At that time, the Conflict Alert started going off with an aircraft at Badger which we had no knowledge about, a McDonnell Douglas also climbing to FL230, roughly 5 miles ahead of the A320. I then told the RADAR to climb the A320; the Controller did, to FL240. In my opinion, there was very little to no delay of the A320 being level at FL230. After my personal review of the Falcon replay, there was already a loss of separation with the A320 at FL229 and the McDonnell Douglas at FL223. Squib had "flashed through" the A320 with an AIT function. This is a sign to me that they had accepted responsibility for separation for the two aircraft that they had both received from the Harly Sector. The Harly and Squib Sectors need to learn how to separate their own traffic. Harly needed to use speed control between the A320 and the McDonnell Douglas, their own traffic. Badger had no knowledge whatsoever in the DTW arrival, the McDonnell Douglas. Also, Squib should not have turned the McDonnell Douglas who was on a vector towards the SUDDS Intersection however drastically they did to make the A320 and the McDonnell Douglas traffic for each other. Since the loss of separation occurred before Badger could do anything to resolve the situation and separate traffic, I do not feel that calling the lower sectors (Harly and Squib) would have achieved anything.

Narrative: 2

I was working BAE R-Side and an A320 checked on from low Sector 62 climbing to FL230. This aircraft was a flash through procedure with low altitude Sectors 62 flashing to Sector 27 and then to high altitude Sector 60. Conflict Alert went off shortly after as the A320 aircraft was over taking a McDonnell Douglas also climbing to FL230. Separation was lost between Sectors 62 and 27 airspace in low altitude stratum below my sector. I expedited a climb to FL240 for the A320. FL240 was only available due to traffic opposite direction. Stop the flash through procedure. Separation was lost due to confusion as to whose responsibility it was to separate the aircraft involved. Was it 62 or 27? Sector 27 took action by giving a control instruction to the McDonnell Douglas while being aware of the A320 aircraft because he executed a flash through. However, said Controller believes it was 62 Sector's responsibility to separate the two aircraft by using altitude because in the flash through procedure Sector 27 has control for turns on aircraft therefore Sector 62 should ensure or protect for such turns. Take away the flash through procedure, Sector 27 takes RADAR on all aircraft climbing through their sector. Or perhaps take away the ability for Sector 27's control for turns.

Narrative: 3

A McDonnell Douglas was in level flight proceeding direct MKG via POLAR3. Squib had received the aircraft from Harley Sector and has control to turn to MKG per SOP. An A320 had departed ORD via PETTY ECK. Harley Sector had climbed the A320 to FL230 and flashed at Squib. Per AIT (Automated Information Transfer) and per SOP, Squib flashed the A320 onto BAE Sector. BAE received transfer of communication from Harley per SOP without any further coordination from Harley. The A320 turned toward ECK and BAE did not climb the aircraft. The A320 was faster than the McDonnell Douglas and caught up to the McDonnell Douglas a few miles later. Harley Sector needed to ensure separation and made necessary coordination with BAE prior to transfer of communication. The AIT gives Squib control on the McDonnell Douglas, and Harley based separation on Squib stopping McDonnell Douglas at a lower altitude. BAE should have coordinated that they would level the A320 for some time in Squib's airspace.

Narrative: 4

The hand off was taken on a McDonnell Douglas. He was then put direct MKG. An A320 was a flash through climbing above the McDonnell Douglas and then put on course direct ECK. This is a procedure that is done several times a day. It seems as if the Controller working the A320 was unaware or not very familiar with the procedure and did not watch the speed of the A320 following the McDonnell Douglas. I recommend that controllers become more aware of flash through procedures and not forget about aircraft route separation.

Synopsis

ZAU Controller described a loss of separation event involving a number of sectors, the reporter claiming the "flash through" procedure was a contributing factor.

ACN: 1030597 (2138 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : C90.TRACON
State Reference : IL
Altitude.MSL.Single Value : 4000

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach

Person

Reference : 1
Location Of Person.Facility : C90.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1030597
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

[I was] working [Runway] 22R approach at C90. Triple runways were in use and traffic from southwest the Plano Sector was rerouted to the right downwind for 22R. Runway 22L was opened and being used as a trip runway and visual approaches to that runway were in use because the localizer was OTS. Thunderstorms had just passed thru and high winds aloft from the west were a factor. We were using the new fusion RADAR and it was taking some getting used to. The timing of turns is different as is the movement of the targets. I turned base leg from the right downwind behind a heavy Airbus with what I thought was an A320 and was a little early from the west side. Turned in behind the heavy and was being pushed by the wind. Turned in to final and was gaining on the heavy. Tried to obtain visual on the heavy but a scud layer had moved in. [I] vectored the A320 off to the right to avoid loss of separation. Obtained visual on the airport with the A320 and vectored him back in towards the airport. The timing with the new fusion RADAR and the wind played a roll in the use of the maneuvering to maintain separation behind the heavy. More experience with the new fusion RADAR will help with the timing when vectoring to the finals. Many controllers have been late on turns because of the pace at which this RADAR moves and what we've been used to with the old ASR 9.

Synopsis

C90 Controller experienced a loss of separation event claiming the new "fusion" RADAR.

ACN: 1029640 (2139 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 350
ASRS Report Number.Accession Number : 1029640

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately 250 miles west of ZZZ, the number 1 engine N1 vibration indicator started flashing, it was above 5.0. The Captain was in the restroom at this time. When he got back to the cockpit just a few minutes later the vibration indicator was above 8.0, it got up to a high of about 8.5. This was also accompanied by a noticeable airframe rumble or vibration, similar in nature to the vibration or rumble we hear in the Airbus when the landing lights are extended at the higher air speeds, like just before retracting the landing light passing 10,000 FT. I took the aircraft and radios and the Captain handled the checklist and radio/ACARS communication with Maintenance Control and Dispatch, etc. I heard some but not all of that interchange since I was flying and talking with ATC. Other than the vibration indications, all other engine indications were normal throughout the flight. As per the checklist, the throttle in question was pulled back to a setting where the vibration indication dropped below 5.0. On the initial reduction the vibration indication was reduced to about 1.5 or so and the airframe vibration/rumble stopped completely. We descended to about 26,000 FT; maintaining altitude and airspeed was not a problem, and the engine seemed to be operating normally. All indications were normal. While all this was in progress, I checked weather for several possible alternates. I also notified ATC of our possible intent to divert. By the time we finished reviewing the checklist and by the time the Captain had terminated discussions with the aforementioned agencies, we were now much closer to ZZZ and actually in a position to make a near normal profile descent into the ZZZ airport. At this time at the lower altitude the engine was operating fine, within vibration limits, even at higher thrust settings. As a precaution we kept the engine at a lower thrust setting. Also, at this point in time the diverting to one of the possible alternates would have necessitated a course reversal. ZZZ seemed like the most reasonable and closest alternate at this point in time. It also had one of the longer runways. I think this was a comforting thought for us, especially after a very long duty day, over 13 hours I think, and a 2am landing. 

On the previous leg we had experienced a flap malfunction and we landed in ZZZ1 at high speed with no flaps, slats only. We used quite a bit of runway in that scenario. One note to add here. On the walk around in ZZZ1 I noticed a fluid leak from the number 1 engine drain mast. We wrote it up and notified Maintenance. Maintenance decided to do an engine run up. The passengers were deplaned. Then Maintenance changed its mind and decided a run up was not required. The passengers were re boarded. They told us that they had found the problem. It was, I think, one of those "acceptable parameter" leaks, possibly around the engine oil cooler, but I am not sure about that; I can't remember exactly. Anyway, the item was signed off. I expressed my doubts to the Captain about the efficacy of this fix since no corrective action was actually taken. I do know that some leakage is acceptable in certain cases. When we experienced the vibration problem in flight I also told the Captain that the drain mast leak and the vibration problem were probably related. I have to state here that this is pure conjecture on my part, with no actual proof, but it was my gut feeling as a pilot. 

I would like to relate what happened on a previous trip a few weeks prior because it may shed some light or have some bearing on the event now in question. Just a couple of weeks prior, I found a similar drain mast leak. They had only one mechanic there and they wanted us (the pilots) to do an engine run up. We refused to do the run up. The flight cancelled and we stayed over night. The next morning two mechanics were flown in and they found the problem. It was an internal engine O ring, in a critical part of the hot section that was "fried and hard like plastic", their words not mine. A part was flown in later that morning and the engine repaired. I asked the mechanics if this problem might have caused an in flight engine failure/loss of oil/possible engine shutdown due to fluid loss, etc. The answer was yes. I also asked them if, doing a run up, the leak would have dissipated, or "gone away", at least to the naked eye, because a static leak could be blown out the back of a running engine. The answer again was yes. I believe we may have likely averted an in flight problem with this engine by refusing the run up and by essentially mandating that the proper number of qualified mechanics properly address the problem.

Synopsis

A320 First Officer experiences high N1 vibration (5 units) at FL350, which increases to 8 units in the next few minutes. As thrust is reduced for descent and landing at destination the vibration is reduced to 1.5 units and does not return.

ACN: 1029415 (2140 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Wheels/Tires/Brakes
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1029415
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1029417
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 3

Reference : 3
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1029416

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Removal and Replacement (R/R) of # 2 Brake was difficult to remove from the [left Main] Landing Gear (MLG) axle. I, Mechanic X, inspected the axle sleeve, but found no obvious damage. We installed a Serviceable Brake Assembly and Wheel Assembly. One of the Mechanic's working on it went upstairs, signed the Logbook. After a few hours in, I received a phone call from one of the Mechanic's working on the brake to let me know that he and a Supervisior looked over the Unserviceable brake and noticed four axle bushings for the Brake Alignment studs were stuck to the [removed] brake. I talked with the ZZZ1 Supervisor and let him know what had happend. He told me that he was already aware of it, and it would be addressed. Alignment pin bushings seperated from axle flange. Recommend Inspection of Axle Protective Sleeve and Axle Hub and Bushings for proper installation. Thru flight operation.

Callback: 1

Reporter stated he doesn't recall if the bushings on the Brake Axle Flange have a collar, or, are the flush face type. They are part of the Main Landing Gear (MLG) Assembly. His Air Carrier issued a Maintenance Alert for mechanics to look for flange bushings when they remove brakes. But their Engineering group has not indicated whether Airbus would make a revision to the Aircraft Maintenance Manual (AMM) to check for bushings on the Brake Alignment pins. There are three bolts with nuts securing the brake to the brake flange (not counting the alignment pins).

Narrative: 2

I, Mechanic Y, was assisting in replacing the Brake Assembly on an A320 aircraft due to both of the Wear Indicator pins missing. We determined it [brake] need full replacement. During the removal process, please note that it was very difficult and hard to remove, taking longer than usual to remove the brake assembly. When the brake assembly was removed, I was installing the new brake with Mechanic A. After the brake assembly and wheel were installed, I cleaned up the area and got the tooling put away. We then returned to the Shop, where we spoke with Supervisor X, who opened up the [Brake] container and noted that four of the Axle Flange bushings were still stuck to the brake. However, the A320 aircraft had already departed to ZZZ1. Supervisor X contacted Maintenance Control and ZZZ1 Supervisor. Was unaware bushings had separated from Axle Flange and the Aircraft Maintenance Manual (AMM) does not specify to inspect axle flange bushings for proper installation. Recommend updating Maintenance Manual (M/M) to reflect an inspection [procedure] of protective sleeve and Axle Flange and bushings for proper installation.

Narrative: 3

Found both Wear Indicator pins on # 2 Brake missing; determined that A320 aircraft needed a new Brake Assembly. While removing old brake, it was noted that brake assembly was very hard to remove; it took longer than usual to removed assembly. When brake assembly was loose and removed, I grabbed Serviceable Card and verified Serial Numbers 'On' and 'Off'. After the brake assembly and wheel assembly were installed, I, Mechanic Z, went upstairs to sign-off Logbook entry. When we went back to the Line Shop we talked to Supervisor X, and he opened up the Brake container and noted that four of the Axle Flange bushings were stuck to the Brake (Alignment studs). By that time, the A320 aircraft had departed to ZZZ1. Supervisor X contacted Maintenance Control and ZZZ1 Supervisor. Did not realized bushings had separated from the Axle flange, and AMM does not specify to inspect axle flange bushings for proper installation. Through flight operation.

Synopsis

Three Aircraft Maintenance Technicians (AMTs) report that four bushings had separated from the left Main Landing Gear (MLG) Axle Flange when the # 2 Inboard Brake was removed. The bushings had remained stuck to the removed Brake Assembly alignment pins and were not noticed until the A320 aircraft had departed.

ACN: 1029058 (2141 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1029058
Human Factors : Time Pressure
Human Factors : Physiological - Other
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Analyst Callback : Attempted

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Company Policy

Narrative: 1

Inbound aircraft to make up our flight had a companion dog that vomited and defecated multiple times. Stench quickly traveled throughout aircraft. Dog defecated in jetway as it was deplaning. Cleaners attempted to disinfect and cleanup the problem. When my crew boarded the aircraft, the fumes from the cleaning fluid were very intense. Flight Attendant thought she was getting a headache from the odors. My evaluation of the situation was that the fumes were way too strong to allow passenger operations. The cleaners had used a cleaning product called "Aero Clean X-200." The label on the bottle says in part, "Vapors may cause central nervous system depression, headache, and dizziness..." I had to refuse the airplane for passenger operations. While all along the company kept lying to me saying there were no other aircraft available in a maintenance base and major hub, at night with multiple overnight airplanes. When I refused the airplane, amazingly there was another airplane available. Unnecessary 2 hour delay.

Synopsis

A Companion Dog on an A320 became ill and loose boweled, requiring a potentially toxic cleaning agent to clean up. The crew refused aircraft for flight because of fumes.

ACN: 1029008 (2142 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1029008
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Landing north during simultaneous approach procedures. The weather was marginal VFR with a scattered layer between 1,000-3,000. I was working arrivals to Runway 1R. The aircraft checked in on a visual approach. I cleared him to land. When he was 2-3 miles from the airport, and around 1,500-2,000 FT, he said to that he lost the airport, but was comfortable continuing the approach if I was. Having never faced this situation, nor having been trained what to do in this situation, I allowed him to continue inbound, telling him to inform me when he got the airport again. After flying about a mile he had the airport in sight and he landed without issue. After speaking with several colleagues who were former RADAR controllers, they told me that I should have cancelled his approach clearance, and delivered him back to the Approach Control facility. I don't think that the final controllers should vector aircraft for visual approaches when we have coordinated and are running simultaneous operations. Additionally, the Monitor should have spoken up and provided instructions to the aircraft when the aircraft said he had lost the airport.

Synopsis

Local Controller failed to issue missed approach instructions after an aircraft cleared via a visual approach lost sight of the airport, this all occurred when simultaneous approach procedures were suppose to be in place.

ACN: 1027458 (2143 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Aileron
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12900
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 1630
ASRS Report Number.Accession Number : 1027458

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

With the exception of a security ordered return to the gate to remove a bag, everything was normal and the First Officer was flying. We were climbing out of [departure airport] and were accelerating through 250 KTS and 10,000 FT when the aircraft began a consistent, rhythmic shudder. It stopped around 12,500 FT for a short time and began again. We had no ECAM alerts or messages, just the airframe vibration. We decided to level at FL200 and slowed to 250 KTS and began trouble shooting by going through the system pages. We noticed that the right aileron indicator was vibrating at the same rate as the aircraft. We contacted the flight attendants for their input and were told that it could be felt throughout the airplane and passengers were showing signs of concern. 

We initiated contact with Dispatch and Maintenance through ACARS. The aircraft was always in control, but the vibration was quite uncomfortable. Because the shudder was nauseating, we made the decision to divert [to a nearby airport]. We did not declare an emergency as we were receiving priority handling and the aircraft was responding normally. We did a few normal turns in the descent to see if we could gather additional information for Maintenance, however, the vibration continued to the ground. I exercised emergency authority to execute an overweight landing. The landing, roll out, and taxi in were all normal. Upon deplaning, a couple of passengers mentioned that they could see the aileron "fluttering in the wind", but didn't want to alarm other passengers by saying anything. We were able to pass this information on to Maintenance.

Synopsis

A320 Captain reported abnormal aileron vibration severe enough they decided to divert to nearest suitable airport.

ACN: 1027385 (2144 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 32000

Environment

Weather Elements / Visibility : Turbulence

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAB

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Cruise
Airspace.Class A : ZAB

Person

Reference : 1
Location Of Person.Facility : ZAB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1027385
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 was at FL320 requesting FL360 because they were getting a choppy ride. I told him that I had his request, there was and aircraft at FL340 flying right over the top of him. Once he cleared the FL340 traffic I climbed him to FL360. Before I climbed the A320 I did a quick traffic scan thinking in the back of my head he was at FL340. Thinking that he was at FL340 rather than FL320 I did not see any traffic for him and climbed him. There was an Air Carrier southbound at FL330 missed the traffic because I thought the A320 was at FL340. I turned both aircraft 30 right, one second later, both were responding to a RA. Double check the altitude of the aircraft requesting higher and note the Data Block.

Synopsis

ZAB Controller described a loss of separation event when neglecting to properly scan the Data Block altitude information before assigning another altitude.

ACN: 1027133 (2145 of 2908)

Time / Day

Date : 201208
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1027133
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
Were Passengers Involved In Event : N
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Equipment / Tooling
Primary Problem : Equipment / Tooling

Narrative: 1

After departure [one of my] flights experienced a nose wheel steering ECAM alert. While trying to work this situation which was a declared emergency by the crew, I experienced numerous delays/lock ups by [our new communications system]. If this situation had been time critical, these breakdowns would have presented a significant hindrance in my ability to transmit and receive critical information to/from the flight crew in a timely manner. This tool represents a significant safety issue. It needs to be fixed or replaced before someone gets hurt.

Synopsis

A Dispatcher's ability to communicate with a flight experiencing problems was compromised by dysfunctional new communications hardware.

ACN: 1027112 (2146 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : IAH.Airport
State Reference : TX
Altitude.MSL.Single Value : 800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : IAH
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : IAH

Component

Aircraft Component : Aircraft Cooling System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1027112

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When flight planning for flight, log history showed two previous write-ups for sudden excessive heat surging into flight during descent below 10,000 FT. When aircraft arrived at gate, Inbound Captain debriefed us that on both the flight to and the return, they suffered the same problem and tried to control temperature by turning off hot air. This did not work and they went to APU for air conditioning and it seemed to work. Also had REV deployed ECAM in flight on the leg to IAH. The local mechanics had never performed the required reverser test and had difficulty getting through the procedure. Eventually, they found someone who had a little experience with that procedure and he came to assist. While mechanics were going through the test required for the reverser, I talked with Maintenance Control about the plan to defer the hot air to address the excessive cockpit temperature problem. Maintenance Controller said the previous crew did not give the hot air off a chance to correct problem. Since this was different than my understanding from my conversation with the crew, I gave the [Maintenance] Controller the benefit of the doubt and accepted the aircraft with the deferral. I asked what we could do if it recurred and he brought the Environmental Control System (ECS) report to my attention and provided the Flight Manual (FM) reference for this additional procedure. He explained that they had difficulty troubleshooting this kind of problem and having a print out of exactly where all the valves were positioned during an event would help. We departed and when we were only a few hundred feet in the air, a rush of hot air flooded the cockpit that was as severe a change as walking into a sauna. Unfortunately, the cabin temperatures also ran very hot this time - that had not been reported as occurring during the previous events. If we were not able to control the dramatic and severe rise of temperature, we would be forced to return to the field. I attempted to control the swing by turning down all three temp zone controllers and grabbed for the FM for the ECS procedure. I believe I was able to capture the event with this first report. With much attention and changes, the cabin temperatures stayed about 74 degrees for a while at cruise. However, after a short period of time, we watched the zone duct temperatures in the fwd and aft cabin shoot up to 90 plus degrees. I attempted small changes to catch the up swing with little effect. Only large knob turns seemed to affect the zone temps. A Flight Attendant called and alerted that a passenger in row 10 was put on O2 and believed it was a result of the stuffiness of the cabin. Also, she said the gaspers were not putting out cold air (later said it was just row 10). Throughout the flight, I provided engineering reports and corresponded with Maintenance Control about the problems. Also, we used the ECS report two more times and noted the circumstances on each print out. During descent into our destination, the aircraft got suddenly very humid and the zone and cabin temperatures again began to rise without any input to the controllers. I moved the forward and aft cabin temperature controllers full cold with little effect. Then, when I selected full cold on the cockpit temperature controller, the forward and aft cabin zone temperatures dropped dramatically. I discussed all our experiences with the outbound crew and the local mechanics upon arrival. I am now concerned about my decision to accept the airplane with a deferral that I felt was wrong because the result was a very work intensive flight that dis-serviced our passengers with a very uncomfortable flight.

Synopsis

A320 Captain believes that he erred in accepting an aircraft with ECS (Environmental Control System) anomalies. The cabin temperature cannot be controlled and much of the time is excessively hot.

ACN: 1025394 (2147 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.MSL.Single Value : 1100

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Flight Phase : Initial Climb
Airspace.Class B : BOS

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Taxi
Make Model Name : Cessna 402/402C/B379 Businessliner/Utiliner
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 135
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : BOS

Person

Reference : 1
Location Of Person.Facility : BOS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1025394
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

I cleared an A320 for takeoff on Runway 4R. Wind was strong out of the northwest. Once airborne, I instructed the A320 to contact the Departure Controller. Almost immediately, I noticed the A320 was climbing toward a C402 that had departed Runway 4L. The C402 was climbing on the 4R extended center line area instead of tracking to the northwest as expected. The C402 had been worked by the Local Control West (LCW) Controller and had been assigned a 350 heading which was inadequate, during strong northwest wind to ensure separation between the two runways. I immediately notified the Departure Controller of the impending conflict between the two aircraft. I should have talked more with the LCW Controller about the wind and the Runway 4L departures. I should have also recognized the conflict before I had the A320 change frequencies so that I could have issued instructions to resolve the conflict.

Synopsis

BOS Controller described a conflict event between two IFR departures from different runways, listed coordination failures and lack of intervention as causal factors.

ACN: 1022925 (2148 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1022925
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I have had two events recently where pilots have operated an aircraft system with the 'CAUTION MAINTENCE BEING PERFORMED' sign posted in the cockpit on the [Center] Pedestal. When ask what they thought they were doing, they both said that they were not operating flight controls, not understanding they are not to touch any systems. This is a very dangerous condition that the pilots [are] putting all of us in, by either ignoring or not understanding what this sign means. This situation needs to be addressed quickly before something is damaged or someone is hurt or killed. 

After one pilot was notified, he was very apologetic, the other [pilot] still thought he did nothing wrong, or put anything or anybody in danger because he had not operated any flight controls. Both pilots have over 30-years seniority. [Recommend] more safety training for pilots to have them understand that it is everybody's job to put safety first.

Synopsis

A Line Mechanic reports about two events he experienced where pilots had operated an aircraft system when the "CAUTION MAINTENANCE BEING PERFORMED" sign was posted on the cockpit Center Pedestal. Mechanic noted the situation needs to be addressed quickly, before equipment is damaged, or someone is hurt or killed.

ACN: 1022311 (2149 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1022311

Events

Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On descent, received ACARS message from Dispatch stating VIP movement at airport and there may be a 10-minute delay. After checking weather, no delays were noted; weather was clear. After contacting Approach was advised to hold at KADDY Intersection, expect 25 minute in holding. I sent ACARS message to Dispatch notifying of hold. We had 35 minutes of hold fuel in release. I also notified Dispatch that I would burn down to arrive [with] 4,000 LBS fuel and if not released to [destination] by then would declare minimum fuel.

After reaching our EFC, asked Approach how much longer. They stated at least 25 minutes. My First Officer and I discussed this and decided to declare minimum fuel. Approach stated "sorry we can't help you", and that we should proceed to our alternate. I again sent a message to Dispatch asked about alternate airports. Dispatch stated that [a nearby airport] was approved for 757/767 only and that we may have to declare an emergency to get to [destination]. After several minutes my First Officer and I decided to declare emergency and proceed to [destination]. We notified Approach, [and] were given direct airport. Landed [and] taxied to gate. No further problems. 

VIP movement was supposed to be completed by an hour before, but this did not occur. Dispatch had hold fuel for delay; however additional holding was not expected. Approach did not honor minimum fuel request for priority handling. This forced us to declare an emergency.

Synopsis

A320 Captain reported minimum fuel to Approach while holding for VIP movement on destination airport. When that did not produce the desired result, he declared an emergency and landed at destination.

ACN: 1022088 (2150 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1022088

Events

Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

VIP movement [resulted in] ATC hold, EFC 20 minutes, held 20 minutes, given another 20 minutes airborne hold, unable to hold, declared emergency fuel and landed [with] 4,500 LBS, reserve fuel 3,199 LBS.

Synopsis

Dispatcher reported an aircraft was holding because of VIP operations, but finally had to declare a fuel emergency when the delay dragged on.