ACN: 1021949 (2151 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Trailing Edge Flap
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Other Documentation
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1021949
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Primary Problem : Human Factors

Narrative: 1

We [my Air Carrier] received an A320 left-hand (l/h) Inboard (I/B) Flap from a Contract Maintenance Repair Station with an FAA 8130-3 Tracking Tag. The Status/Work [section on the Tag] says: Repaired. The FAA 8130-3 Tag indicates all other discrepancies left as is and Customer will complete all other items listed in Tear Down [Disassembly] Report. This flap was delivered with several parts not installed and just put inside a box loosely. This flap is not a complete unit. Several bearings [that were] not written-up are bad and have paint over-spray; both wiring harnesses on unit are chafed and worn thru, only one was written-up in Tear Down Report. There are several discrepancies with this flap not written-up and [this flap] should not have an FAA 8130-3 Tag attached to it. Nowhere on the FAA 8130-3 Tag does it indicate [the flap] unit is not assembled per the Illustrated Parts Catalog (IPC); or [indicate] what parts are not installed, [also] some parts missing and not documented.

Synopsis

An A320 Left Inboard (I/B) Flap unit received from a Contract Repair Station with incomplete repairs, parts not installed, numerous discrepancies not written-up, worn bearings, chafed wiring harnesses and missing parts not documented had an FAA 8130-3 Tag that stated: Repaired.

ACN: 1021635 (2152 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Aileron Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1100
ASRS Report Number.Accession Number : 1021635
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out with autopilot engaged. We notice airplane was flying in a slight left bank. We analyzed the problem, fuel balanced, engine parameters all normal. Flight control page showed right aileron full down. Rudder trim indicated 1.5 to 3.5 right. Stopped climb at FL270 and slowed to 280 knots. Contacted Dispatch and Maintenance Control. We concluded the best course of action was to return. On descent we experienced a jolt as right aileron returned to normal deflection, followed by right aileron returning to full down deflection. Thinking flight controls were in deteriorating situation, we declared emergency and landed safely.

Callback: 1

The reporter states that the handling qualities did not seem to be compromised in any way and the aircraft landed normally with full flaps. He has not heard what actions by maintenance were necessary to return the aircraft to service, but did notice that it was still present 24 hours later.

Synopsis

A320 Captain reports noticing the aircraft is flying with the left wing slightly down during climb and the rudder trim is to the right. The ECAM flight control page indicates the right aileron is deflected full down. An emergency is declared and the flight returns safely to the departure airport. During descent a jolt is felt as the aileron returns to its' normal position and quickly returns to the fully deflected position.

ACN: 1021114 (2153 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 19000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 85
Experience.Flight Crew.Type : 6877
ASRS Report Number.Accession Number : 1021114
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Narrative: 1

While level at FL190, an Auto Thrust ECAM appeared, with associated and intermittent FMA warnings of Thrust Lock and move throttle. Subsequently, the number 1 engine retarded uncommandedly to idle. Unsure, of the exact problem, and not sure if we could hold altitude while we troubleshot the problem, we declared an emergency with Center and advised them to standby. As the pilot flying, I disconnected the autopilot, retarded both throttles to idle, and we assessed both engine indications. We ensured that the Auto Thrust system was disengaged and slowly advanced each engine throttle. No other abnormalities were encountered and both engines appeared to be operating normally. We felt the situation was safe so I rescinded the emergency declaration. During the event, there were no altitude or route deviations. Subsequently, we made a safe, hand flown approach under VMC conditions.

Synopsis

An A320 flight crew experienced failed autothrust capability and, after analysing their capabilities and the weather, continued utilizing continued to their destination using manual flight and thrust controls.

ACN: 1020890 (2154 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 19000

Environment

Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1020890
Human Factors : Workload
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were on the arrival into our filed destination and had just finished deviating around a few thunderstorms. All of the sudden we got an ECAM that the autothrottles turned OFF and we got an ECAM that the autothrottles were in thrust lock. Then the Number 1 engine suddenly rolled back to idle thrust while the Number 2 engine remained at normal thrust. We declared an emergency letting ATC know we were experiencing and engine failure and we would get back to them. Within one minute later, we were able to regain normal control of the Number 1 engine, but not the autothrust. We rescinded the emergency with ATC and continued the flight to ZZZ1 without the autothrust. The rest of the flight was uneventful.

Synopsis

An A320's autothrottle transitioned to THRUST LOCK followed by the number one engine spooling back. The crew declared an emergency, but the engine returned to normal operation with no autothrottles.

ACN: 1020820 (2155 of 2908)

Time / Day

Date : 201207
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : LAX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Ground
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1020820
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X was holding on Taxiway 1 for an outbound Airbus in taxi lane XX. Aircraft X taxied to his gate without waiting for the outbound aircraft as instructed. I made numerous attempts to get Aircraft X to stop, but he had left the frequency to talk with the Company Ramp Tower. I could have had aircraft pushing off gates YY or ZZ. I could have had any number of aircraft taxiing on Taxiway 2. Though this did not happen, it does not diminish the potential for an accident in the future if these types of actions occur again. After the Tower spoke with the Company Ramp they said they did not tell Aircraft X to taxi, but they did answer affirmative when the pilot of Aircraft X asked if the Airbus in XX was waiting for them. Recommendation, these scenarios play out almost daily at LAX. The ramp towers at LAX lost their usefulness after 9/11. Soon LAX will be burdened with another position, Ground Control 3, at our critically, short-staffed facility. To provide a modicum of work for this new position, all LAX ramp towers need to close and service must be returned to the tower before someone gets hurt.

Synopsis

LAX Controller voiced concern regarding a ground conflict resulting from the handling provided by "Ramp Control," The reporter suggested Ramp Controller operations be terminated for safety concerns.

ACN: 1020307 (2156 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1020307

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

No airspeed on Captain's side during takeoff. Aborted takeoff and went back to gate.

Synopsis

A320 Captain lost his airspeed indicator on takeoff roll. Rejected takeoff executed and aircraft was returned to the gate.

ACN: 1019850 (2157 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Thunderstorm
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1019850

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

No airspeed indication on the Captain's PFD on takeoff roll. Aborted takeoff at 80 KTS. Failure of onboard aircraft systems to provide airspeed info.

Synopsis

A320 Captain experiences a loss of airspeed indication and rejects the takeoff at 80 knots.

ACN: 1019846 (2158 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 1019846
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After clearing 25L at H-6, we joined H and were instructed to "face west and hold short of P." The intersection of H and P is not a sharp 90 degree turn, but rather a gradual easement (i.e., a high-speed turnoff). Taking care not to encroach onto any portion of P, we stopped short of the point where the pavement begins to diverge from H onto P. After stopping, ATC insisted that we "move up and hold short of P." This was perplexing as, in our best assessment, we were doing just that, and moving forward would have constituted an incursion onto P. 

Before we could ask for clarification, ATC then told us, with some urgency, to move forward as we were "blocking the runway." Presumably, ATC meant that we were blocking the exits for 25L, which are in close proximity to P. Since we were well established on H, no part of the aircraft was over any runway hold line. We then moved forward, passing P, and were instructed to hold further west on H, behind another aircraft which was holding short of T. 

There a few possible explanations for this confusion. First, it may me possible that ATC intended us to hold short of T, behind the other aircraft, rather than hold in the area near P, which is in the vicinity of the several 25L turnoffs. ATC may have mistakenly said P, meaning T, as we all sometimes do. The other is that ATC may have indeed wanted us to hold short of P but we did not move far enough forward. If so, there are inadequate pavement markings with which to determine the proper holding position.

Synopsis

An A320 flight crew and ATC at LAX had a conflict over where to hold short of Taxiway P when Westbound on H. The "quasi" high speed entrance to P at H combined with the lack of any hold short markings were contributing factors.

ACN: 1019714 (2159 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : CLE.Airport
State Reference : OH
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : CLE
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : CLE

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 2475
ASRS Report Number.Accession Number : 1019714
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

For the second time in as many attempts, I experienced a failure of both FMS-1 and FMS-2 while "cleaning up the box" on approach into CLE. Please see my previous ASRS report for details, as this was an exact duplicate of my previous experience. Please note in my previous report, that both my First Officer, along with both the outbound Captain and First Officer had all experienced similar failures recently. Additionally, the mechanic who met our flight in CLE told me that he has seen a sudden spike in these failures. There is obviously a problem here. From my limited experience, it appears that the problem is associated with only certain airports.

Synopsis

A320 Captain experiences a dual FMGC lock up while attempting to "clean up the box" during vectors for approach. This is the second occurrence the reporter has experienced recently.

ACN: 1019472 (2160 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 5000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Aircraft Logbook(s)
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1019472
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

When we attempted to lower the landing gear on final nothing happened. After we unsuccessfully cycled the landing gear lever three times we elected to do a go around and trouble shoot. After checking in the Flight Manual, checking for popped circuit breakers, and consulting with Maintenance we prepared for our return and successfully performed the Gravity Extension checklist on a long final. 

During the conversation with Maintenance Control we were told that the same thing had happened eleven days previously. In our maintenance history the last entry went back thirteen days but there was no entry for the previous gear malfunction. This knowledge could have been quite beneficial. Our initial fuel load had us planned to land with 4.8 thousand pounds of fuel. Luckily we had increased this by 1,000 pounds prior to departure. Had we not increased the fuel load we would have had very little fuel time left to further troubleshoot our problem if the gravity extension was not successful.

Synopsis

An A320 flight crew was unable to lower their landing gear, initiated a go around, successfully conducted a gravity extension procedure and returned for a safe landing. Their fuel remaining could have compromised more extensive trouble shooting had the gravity extension not proven successful. A previous identical failure some 11 days before was not annotated in the logbook or in the flight crew's maintenance briefing message provided for flight planning.

ACN: 1019333 (2161 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Gear Extend/Retract Mechanism
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1019333
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

The [landing gear change] crew had been briefed that if they needed to remove and use a part from the 'Off' [removed] gear, that they were to ensure that the damaged or Unserviceable part was shipped with the 'Off' unit. This was briefed based on the information provided by [our] Air Carrier Vendor Representative and the Contract Maintenance Landing Gear Repair Station Vendor Representative, during a Station visit during the first Main Landing Gear (MLG) change. They explained that the contract was different from the previous contract in that, the gear was Overhauled as a whole [Assembly]. If parts were missing, our Air Carrier is charged, but if the part is Unserviceable it is covered under the contract. This is how the decision came to swap the MLG Upper Side Stay Spring attach pin with the 'Off' unit. All of the parties involved, including myself, were aware that the MLG was a Time-Limited item, but not the individual pin. The paperwork does not state that these are Time-Limited parts.

Synopsis

A Line Mechanic working on an A320 aircraft in a Hangar, reports that an overhauled Main Landing Gear (MLG) Upper Side Stay Spring attach pin was removed and replaced with the pin from the 'Off' (removed) MLG. Their MLG paperwork did not state the old attach pin was also a Time-Limited part.

ACN: 1019201 (2162 of 2908)

Time / Day

Date : 201206

Place

Altitude.MSL.Single Value : 31000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11800
Experience.Flight Crew.Last 90 Days : 165
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 1019201
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Fatigue isn't an appropriate word to describe my state during the last hour of cruise. Totally exhausted would be more appropriate. I could not remember frequencies I had read back 10 seconds earlier. Although our departure had been delayed an hour due to late arriving aircraft, I felt ok to fly and was fine through takeoff, climb, and first few hours of cruise. But the ridiculous schedule I've been given caught up to me in the late stages of the flight. I have had 1 of the last 11 days off, and in that period have flown 3 all nighters, and one early departure. This type of scheduling is unnecessary, thoughtless, unhealthy, and dangerous. Going forward I have 2 days off, then 3 on, 1 off, and 5 on. This type of flying might sound fine to someone who is in an office 8 hours a day, 5 days a week, but in the actual operation with almost every flight being delayed (note not ONE due to pilot issues); it is proving to be unsustainable. I will be evaluating my level of fatigue carefully before each leg.

Synopsis

A320 Captain describes extreme fatigue after a series of late night flights.

ACN: 1019118 (2163 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Cabin Lighting : Off
Number Of Seats.Number : 150
Passengers On Board.Number : 150
Crew Size Flight Attendant.Number Of Crew : 3

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Cabin Activity : Safety Related Duties
Cabin Activity : Deplaning
Cabin Activity : Boarding
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Experience.Flight Attendant.Total : 10
Experience.Flight Attendant.Airline Total : 10
Experience.Flight Attendant.Number Of Acft Qualified On : 2
Experience.Flight Attendant.Type : 100
ASRS Report Number.Accession Number : 1019118
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

Please do not allow my company to make their flight attendants work over 17 hours, all day and night with no rest allowed. On this date we reported for our usual show time for our flight East Coast to West Coast and return, arriving back home 12 hours later. We were not informed by the company that we were delayed three and a half hours. Knowing this specific trip/pairing is a redeye trans-continental turn. Our original release is an early morning arrival back in base. Yet on this trip we were on duty from early evening UNTIL the early afternoon with NO REST. We HAVE to fly 17 hours and 59 minutes with NO REST or we will be fired or put on extreme disciplinary action. We could NOT have evacuated that aircraft if we tried as we were all beyond exhausted. We are not allowed to close our eyes once or we will be fired. We do not have crew rest on board and the company does not allow us to take a break. All other jobs get a 15 minute break every 8 hours. At this company we are not allowed a break as in flight crew members until we land and are off duty. Our original pairing is 12 hours, with no breaks, no crew rest, no closing our eyes once. Then add on another 6 hours like today and we all felt physically ill trying to perform our jobs. Please help us find a solution to this practice.

Synopsis

Flight Attendant describes a east coast to west coast round trip flight sequence that is scheduled to depart in the evening and return at dawn. The trip ends up being delayed six hours after the crew has checked in resulting in an 18 hour duty day.

ACN: 1019094 (2164 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 1019094
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Incursion : Runway
Detector.Person : Flight Crew
Miss Distance.Horizontal : 1500
Miss Distance.Vertical : 0
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While operating flight from ATL Tower cleared us to position and wait on 9L while an RJ flight took off in front. Tower then cleared us to take off as normal. All calls were normal through 80 KTS. At about 120 KTS the Captain initiated an abort and I looked up from my scan of the instruments and saw an aircraft taxiing across the runway on Taxiway DIXIE. The abort went as per normal and I made a call to the Tower and a PA to the passengers as the Captain brought the ship to a stop off the runway at M14. We stopped at M/DIXIE Taxiways and ran through the procedures for post abort and then made calls to ground ramp and company for further guidance. After consulting with operations we were instructed to return to the gate.

Synopsis

A320 First Officer departing Runway 9L at ATL describes a rejected takeoff at 120 KTS when the Captain notices an aircraft crossing the runway ahead.

ACN: 1018948 (2165 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 3100

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Facility : SFO.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1018948
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Speed : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

An A320 departed Runway 1 left PORTE3 SID. In the next departure hole - after landing traffic on Runway 28L/R, a B757, departed Runway 1L on the PORTE3 SID. Upwind four miles, a B757 was about 1-2 miles east of the A320 who was incredibly slow and turned on a dime. I do not know whether or not a loss occurred in NCT airspace. Is it the aircraft -or- the pilot? Either way this is an example of a dangerous situation where Airbus aircraft are neither performing to their capability, and nor meeting the needs of the NAS.

Synopsis

SFO Controller voiced concern regarding the flight characteristics and/or pilot techniques of the A320 with regard to SID compliance.

ACN: 1018285 (2166 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 4100
ASRS Report Number.Accession Number : 1018285
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Work Refused
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I reported for duty this evening for a scheduled departure an hour later. The aircraft was at the gate being worked on by mechanics for an inbound writeup. It took three hours for us to push and subsequently return to the gate a half hour later with the same failure being displayed. The aircraft was repaired an additional hour later but during this time another unrelated failure took place. The failure required a new flight plan and added fuel for operation at lower altitudes. 

It was at this time I calculated my awake time from the previous day and concluded that if all went well, as it hadn't up to this point, I would have been up well over 24 hours while landing. Because of the ongoing maintenance items and the uncertainty of our departure time I felt it prudent to relent to my physical condition of certain fatigue if I were to continue. I called the Crew Desk and advised them of my current condition of fatigue.

Synopsis

Following multiple maintenance delays and false starts for a "red eye" flight, the First Officer of an A320 felt he would be physiologically impaired due to fatigue at their likely arrival time at destination and refused to fly due to fatigue.

ACN: 1018008 (2167 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1018008
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1018499
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 3

Reference : 3
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1018009
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

While performing [Landing] Gear change on an A320 aircraft per Maintenance Task Card, Bolt Item #240, in Illustrated Parts Catalog (IPC) 32-12-14, was torqued and in doing so, failed its Torque Limits and threads were stripped. The bolt from the old [removed Landing] Gear Side Stay was taken and put in the place of the new bolt. I, being the Lead of the crew, directed my mechanics to do so. I got this direction from my direct Supervisor. The outcome is that we cannot substitute a used part from the old [removed] gear and install it on the new [Landing] Gear.

Narrative: 2

During Main Landing Gear (MLG) change, the Left-Hand Lock Stay, Lock Spring bolt was stripped on the replacement gear from Vendor. Installed bolt from removed MLG. Was notified later that bolt was a Time Limited Part.

Narrative: 3

A320 aircraft, replacing Main Landing Gear (MLG) Side Stay, Job Task Card Item #24, to install Lock Spring. Installed pin (bolt), torqued nut to 30-45 inch pounds, the threads stripped. Removed pin and installed pin from the removed MLG. Installed pin and torqued nut. The outcome is not to use any parts from the removed gear.

Synopsis

A Lead and two Mechanics were informed the Lock Spring bolt they had robbed from a removed A320 left-hand Main Landing Gear (MLG) and used to secure a Lock Spring on the Side Stay assembly on a new MLG was a Time Limited Part and should not have been reused. The Lead Mechanic was directed by his Supervisor to reuse old bolt.

ACN: 1017906 (2168 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 1017906

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise, we got a Green Hyd Engine 1 low presure ECAM. All other systems were normal. We accomplished the procedure and then contacted dispatch and Maintenance Control. There appeared to be a discrepancy between the ECAM checklist and the AOM checklist that we referenced after accomplishing the ECAM. As I recall, the ECAM directed us to restore the PTU [Power Transfer Unit] in order to lower the landing gear. The AOM gave no such direction. 
Since our Green system quantity was almost zero, turning the PTU back on didn't seem to make sense. Maintenance Control agreed, and we left the PTU off for the rest of the flight. I briefed the Flight Attendents, declared an emergency and we landed in ZZZ. Because we had to gravity extend the landing gear, we had no nose wheel steering or normal brakes but backup systems worked as advertised. We did, however, have to wait for approximately 20 minutes to get towed off the runway.

Synopsis

When the flight crew of an A320 attempted to resolve a Green Hyd System 1 low pressure ECAM, they discovered a discrepancy between the ECAM Checklist and the more complete procedure in their Flight Manual as far as utilizing the PTU to extend the landing gear. They declared an emergency, landed safely and was towed to a gate after a 30 minute delay.

ACN: 1017089 (2169 of 2908)

Time / Day

Date : 201206

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA

Aircraft

Reference : X
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Route In Use.SID : FIXIT1
Airspace.Class B : LAX

Person : 1

Reference : 1
Location Of Person.Facility : LAX.Tower
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1017089
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : LAX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1017626
Human Factors : Situational Awareness

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Air Carrier X was a FIXIT1 departure. This requires LC1 to issue a heading of 250 even though this is an RNAV departure and the 250 heading will take him off his RNAV route. I forgot to issue the 250 heading and Air Carrier X flew the RNAV departure and eventually turned heading 219 into DR1's airspace vs. DR2's. The FIXIT1 needs to be suspended immediately. The previous Controller also forgot to issue a 250 heading to a FIXIT and switched the aircraft to departure. Fortunately, the pilot had acknowledged the frequency switch he had not, and the LC1 Controller realizing their mistake reached out to the aircraft and was able to correctly issue a 250 heading. I am aware of innumerable other times where an OD has occurred, because the LC1 Controller did not issue a 250 heading. The FIXIT is set up for controllers to fail and nothing about the FIXIT promotes safety.

Narrative: 2

The aircraft departed on a FIXIT1 departure. Due to the shortcomings of the system this RNAV departure is to be issued a 250 heading. I missed issuing that heading. The FIXIT1 needs to be suspended.

Synopsis

LAX Controller described an airspace infraction when an aircraft departing on the FIXIT1 RNAV departure was not issued the required heading. The reporter noted that the RNAV SID sets controllers up for errors.

ACN: 1016822 (2170 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Poor Lighting
Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Interphone System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1016822
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Declared Emergency
Result.General : Evacuated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Primary Problem : Ambiguous

Narrative: 1

Our jet was parked in remote parking. APU air was working for approximately five minutes but then quit. We requested the air start cart for a #1 engine start. During the #1 engine start ground personnel started yelling through the interphone that we were moving. With no lights in the area, we could not detect any aircraft movement, but we shut down the #1 engine when we saw smoke coming from in front of the right wing and #2 engine. CFR was notified and we shut down and left the aircraft through the air stairs.

During the event, the ground person was very difficult to hear. Later, in Operations, one of the ramp personnel commented that the headset had been giving them trouble in the past.

Synopsis

A ground crew headset, previously noted as "giving them trouble" may have contributed to a breakdown in communication between the flight and ground crews when an engine start at a remote location at night ran amok due to uncommanded movement of the aircraft.

ACN: 1016606 (2171 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SEA.Airport
State Reference : WA
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : S46
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : SEA

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 1016606
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While approaching CLE, the Airbus I was flying experienced a simultaneous dual FMS failure but at the time, thought it was a spurious fluke. Last night while approaching SEA I had another simultaneous dual FMS failure. After landing the FO told me he had recently seen the exact same thing. When briefing the Captain who was taking the aircraft out of SEA, he told me that he too had experienced a recent dual FMS failure, and that he had just flown with another FO who had experienced two of them. From these discussions, I learned that all of these happened at the exact same point in the flight; when? Cleaning up the box? After being vectored off of the arrival STAR. For at least a year I have frequently used the technique of selecting "No Star" under "Arrivals" after being vectored off of the STAR. This would result in the box automatically entering all of the fixes on the loaded approach, along with the crossing altitudes. This saves a lot of time in the approach environment, and is useful at time such as when ORD TRACON vectors you off one of the southeast arrivals with a turn to intercept the 27L ILS, which has multiple step-downs which must be complied with. In the case of my two FMS failures, I had just entered "No Star." I noticed that the fixes on the ILS did not automatically load as I have seen in my previous experience. I then attempted to clear out the remaining waypoints from the STAR we had just been vectored off of. That is when both boxes failed simultaneously. "No Map" was displayed on the ND's, initially the FMS froze and would not respond to any input, then everything was dumped and the orange FMS-1 (and FMS-2 respectively) came on each box. This also causes the pressurization system to lose its landing field elevation value requiring manual over-ride. Also, the landing ref airspeed dumps too. The outbound Captain I spoke with last night said that he does not use the "No Star" technique I described above, but that his FMS failure did occur when he went to "clean up the box" after being vectored off of a STAR. I would suggest that someone look into whether there is some systemic issue with a recent data-base update or software change to our FMS's. I know I'm preaching to the choir, but this type of failure occurs at a very inopportune time in the flight. I was fortunate in that both of the failures I experienced occurred in VMC. Had we been IMC we definitely would have had to perform a go-around as it takes a bit of time to get an ILS hard-tuned with an inbound course in Rad-Nav and selected on Rose-ILS, and the pressurization issue has to be dealt with too. It's not a good situation.

Callback: 1

The reporter's research indicates that other methods of cleaning up the approach can cause the dual FMGC failures and that the failures seem limited to arrivals to SEA and CLE only. His company is currently working on the issue.

Synopsis

Another A320 Captain experiences the loss of both FMGC's during approach when the "no star" function is used to clean up the approach after being vectored off an arrival.

ACN: 1016603 (2172 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1016603

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We aborted our takeoff at 70 KTS because the FMC V speeds dropped out of our ASIs. We went back to the gate to let Maintenance resolve the issue.

Synopsis

An A320 flight crew rejected their takeoff due to the loss of FMC generated V speeds on their ASIs.

ACN: 1016598 (2173 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Distance.Nautical Miles : 50
Altitude.MSL.Single Value : 13000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ
Maintenance Status.Released For Service : Y

Component

Aircraft Component : FCC (Flight Control Computer)
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 50
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1016598

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Climb going through 13,000 FT, we felt a moderate vertical jolt similar to wake turbulence. It was followed by ELAC 1 fault which had a long history in the past 7 days of faulting. Contacted Dispatch and Maintenance Control. All agreed to returning and an overweight landing. Performed needed checklist and ECAM. Returned to field and uneventful landing.

Synopsis

A320 Captain reports returning to the departure airport after an ELAC1 fault, which had occurred several times previously to this aircraft.

ACN: 1016323 (2174 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 1016323
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We departed the gate at ORD cleared to taxi to Runway 32L, Intersection T10 via A, A7, T. While taxiing on A approaching the intersection of A7 we noted a ground vehicle approaching the same intersection. As we continued east toward taxiway A7 the ground vehicle (an employee shuttle bus) suddenly surged out onto the taxiway in an attempt to cross in front of us. I quickly realized that a collision with this vehicle was imminent and very abruptly stopped the aircraft. The bus very abruptly stopped also about one third of the way across the taxiway. Fortunately no cabin crew were injured during the stop.

Synopsis

While taxiing for takeoff at ORD an A320 Captain had to brake abruptly to avoid a collision with an employee shuttle bus crossing the taxiway ahead of them.

ACN: 1016055 (2175 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : JFK.Airport
State Reference : NY
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility.Visibility : 6
Light : Daylight
Ceiling.Single Value : 700

Aircraft

Reference : X
ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use : None

Person : 1

Reference : 1
Location Of Person.Facility : JFK.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1016055
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8027
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1017071

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Experience.Flight Crew.Total : 5000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1017072

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 was in position on Runway 4L; a B757 was in position on Runway 31L KE Intersection. I issued the B757 takeoff clearance; the A320 took the B757 clearance and did read it back with his call sign. The B757 immediately asked for a confirmation of the take off clearance. I then restated the B757's clearance to take off. He then stated that he someone else took his take off clearance. At this point I noticed the A320 had begun his departure roll on Runway 4L. He may have gone a hundred feet down the runway when he stopped and asked if he was cleared for take off. I stated that I did not clear him for take off but he was at this time now cleared for take off. There was no loss of separation or possibility of it. The A320 and the B757 both departed with no further issues. Pilots and controllers must be more aware of hear back read back errors. I did not realize that the A320 read back the B757 clearance. I heard what I thought was the B757 acknowledging the clearance. The call signs were similar but not similar enough that I pointed them out to both pilots. The A320 heard what he was waiting to hear even though the clearance was not for him.

Narrative: 2

Aircraft cleared position and hold 4L at JFK. Second aircraft cleared position and hold 31L, KE Intersection. ATC issued takeoff clearance, both aircraft responded, at which point we delayed takeoff until clarification was given by ATC; normal departure without incident.

Narrative: 3

Aircraft was cleared into position 4L at JFK. Another aircraft was cleared into position and hold on 31L, KE Intersection. ATC issued takeoff clearance. Both aircraft responded to call. Event created uncertainty as to who was actually cleared to takeoff. At which point, we delayed takeoff until clarification was given by ATC; normal departure without incident.

Synopsis

JFK Controller described a confused take off event when an Air Carrier initiated a take off without being cleared, confused by another another Air Carrier's clearance departing an intersecting runway.

ACN: 1015985 (2176 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Fire Extinguishing
Aircraft Reference : X

Component : 2

Aircraft Component : Cargo Compartment Sidewall Panel
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Inspector
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Inspector : 20
Experience.Maintenance.Technician : 10
ASRS Report Number.Accession Number : 1015985
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

During C-Check Visit it was found that the Cargo Compartment Firex Bottle had a sense line not connected, as indicated on Task Card, Page five, Item J. I verified Safety and Security of components.

Callback: 1

Reporter stated the Firex Bottle is located in the Forward Cargo Compartment sidewall just aft of the cargo door. The braided, flexible sense line is on the backside of the Firex Bottle. The Firex Bottle could still discharge even though the sense line was not reconnected. The A320 had flown for almost two years before returning for another C-Check Maintenance Visit. That's when a Mechanic noticed after removing the cargo compartment sidewall, the line was disconnected. His company sent out an alert to check 80-aircraft for the same condition; fortunately no similar discrepancies were found. As an Inspector, he does that particular inspection all the time and believes the line was connected at the time. The Job Card Illustrations were not a problem.

Synopsis

An Aircraft Inspector reported that during a C-Check, a Mechanic noticed a Firex Bottle sense line was not connected in the Forward Cargo Compartment right-hand aft sidewall. Inspector had signed-off for the Firex Bottle's Safety and Security on the A320 aircraft approximately two years earlier.

ACN: 1015972 (2177 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1015972
Human Factors : Situational Awareness
Human Factors : Fatigue

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

We had an aborted takeoff and had to return to the gate. Our duty day on this all nighter trip was extended by almost two hours. I arrived home after sunrise. I was now awake for over 14 hours. I almost crashed my car on the way home. I pulled into a 7/11 and gathered myself and continued home. I hope the Carrier will reconsider its scheduling tactics. All nighters on the back end should have ONE leg home not 2 or 3. It is dangerous!!!

Synopsis

A Flight Attendant reported fatigue because of multiple legs following an all night flight and as a result he felt unable to drive home safely.

ACN: 1015868 (2178 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cowling
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 260
Experience.Flight Crew.Type : 2900
ASRS Report Number.Accession Number : 1015868
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

My First Officer did a walk around on the aircraft and found landing gear door seal missing a two inch chunk and the right engine had a six inch chunk of honeycomb missing just outside the fan blades. The Captain went out to look at the engine saw it was part of a repair to the acoustical material just in front of the fan blades and at the bottom inside of the cowl. The repair was approximately six inches wide and two inches deep. I called Maintenance after approximately 1.5 hours to find out the status of the aircraft and was told the aircraft was fixed. I asked about the engine and was told everything was OK and it was on a twenty cycle deferral. We flew the aircraft and on the next walk around the First Officer noticed the repair patch was disintegrating. I inspected the repair and saw a piece was about to come off and go into the engine. I talked with Maintenance Control and they told me the missing pieces of acoustic lining are within tolerance, but they failed to address my concern of the loose pieces of the patch coming off and going into the engine. I refused to fly the aircraft back in its current state. I do not understand why the integrity of the repair patch was not inspected when a small part of it was missing before the previous departure.

Synopsis

An A320 Captain refused an aircraft when a section of the inlet cowl acoustical material was found loose after the same section was temporarily repaired prior to the previous flight.

ACN: 1015790 (2179 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : Marginal

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class C : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : Dash 8 Series Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class C : ZZZ

Aircraft : 3

Reference : Y
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Military
Make Model Name : Eagle (F-15)
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 91
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1015790
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was training someone on Final and it was very busy and just barely VFR weather. A flight of two F-15's were cleared for ILS approach Runway 28L. My trainee was behind the power curve and it looked like he was working to make his plan work and then the lead F15 asked for a vector to rejoin his wing man. This was the start to a very distracted session on final with aircraft being fed at a higher rate of speed from the feeders than they should have been. The Lead F15 had trouble explaining what was going on and I suspected he had an emergency so I broke him out and in the process of hearing UHF and VHF frequencies overlapping it sounded like he was having some sort of emergency. In the mean time I had to put the final back together and took planes that were too high and too fast across the final approach course. An A320 was taken north across final for descent and then back south for a clearance to Runway 28R. A Dash 8 was being vectored behind the A320 for Runway 28L. The Dash 8 was level at 040 and the A320 was descending to 030. It looked like I had divergence but in all the chaos I didn't have time to measure it. The military needs to train pilots to be clearer on their needs and what they need when they encounter in flight problems. The military should use VHF radios when operating in a terminal environment and should make sure their aircraft are not constantly breaking down and causing in flight emergencies. Feeds to final should be at 210 KTS or less when final is busy. It is difficult to assess when a trainee is not going to be able to save a bad situation. Sometimes things just happen.

Synopsis

A TRACON Controller providing OJT had a probable loss of separation involving several Air Carrier aircraft and a flight of military fighters.

ACN: 1015315 (2180 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1015315
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately 1 hour and 15 minutes into flight received ECAM regarding low level hydraulic fluid in the green system. Complied with ECAM procedures, consulted QRH and follow up. Consulted all available flight deck information. Notified Maintenance Control, Dispatch, ATC and departure station Maintenance. Briefed Flight Attendants. Emergency gear extension was required, nose wheel steering, thrust reverser inop. Landed safely, had to be towed off the runway. After landing there was a cracked hydraulic line found in the green system.

Synopsis

An A320 ECAM alerted HYD G RSVR LO LVL at cruise and the QRH complied with. The flight continued to its destination where an emergency landing gear extension was completed and a landing followed without nose wheel steering or thrust reversers.

ACN: 1015245 (2181 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Cruise

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 1015245

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were at cruise and large single crack or melting was noted on the windshield. Then got an Ecam for an overheat. Completed ECAM actions. About 30 seconds later we heard a load pop, looked up and the fo windshield had cracked. Followed non-ECAM abnormal procedure (windshield crack) The decision was made to divert. Descended to lower altitude to reduce landing weight but encountered turbulence. Completed windshield crack procedure prior to landing and followed overweight landing procedures as a precaution (FMGS predicted the landing weight within limits but may have been inaccurate due to SID). Maintenance did an overweight landing inspection.

Synopsis

A320 pilot experiences a cracked front window in cruise and elects to divert after completing QRH procedures.

ACN: 1015156 (2182 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Mission : Ferry / Re-Positioning
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 1015156
Human Factors : Fatigue
Human Factors : Situational Awareness

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Work Refused
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Long call-out reserve, First Officer and I received call out notification at 0728 for a 2145 departure on a ferry flight for maintenance. On arrival at Flight Planning Area, weather was deteriorating rapidly with severe thunderstorm and tornado watch in the vicinity. Aircraft we were receiving diverted resulting in a 2.5-hour delay to a 0318 (0118 body clock) scheduled arrival. After push back from the gate, a thunderstorm was near overhead. We chose to delay start and listen to Tower. Tower was issuing takeoff clearances with "loss of 20 knots." I called Dispatch to get a better idea of the present cell and other cells heading our way. After consulting with Dispatch I determined present cell was slow moving and the next cell was close behind and also slow moving. If anything I felt I had a very small window to safely takeoff. I chose to cancel the flight for weather and related fatigue issues. I was asked by Scheduling if I was willing to be reassigned to a flight a Captain had just fatigued out on. I respectfully told the Scheduler I was not. The reason was the takeoff threat had not changed except now I would have 132 passengers versus an empty airplane. In a separate but related issue, I was concerned about fatigue from the start. Both the First Officer and myself were unable to get any sleep from callout to pushback. With a scheduled body clock arrival of 0118, I was vigilant but not concerned. As the time continued to creep, my vigilance became concern. In a February 2002 NASA study (NASA/TM?2002-211394), a Dawson and Reid study of fatigue versus performance impairment is referenced which equates fatigue to blood alcohol equivalence. At 17 hours of straight wakefulness performance levels are at the equivalence of blood alcohol (BAE) .05%. At 21 hours of straight wakefulness, BAE is .09%. The decision for takeoff is now being determined at 17-hour and the landing will occur at the 21-hour point. Had either one of us been able to get rest before flight, my decision MIGHT have been different (weather remained my #1 concern). I felt threats were piling up too quickly. I like 12-hour callout and believe it to be a sound way of assigning reserves. Unfortunately, in this case, rest before flight could not be achieved. Again, with a scheduled 0118 body clock arrival, vigilance was warranted but not a game changer. Increased threat of weather and delay became a problem I chose to act on.

Synopsis

Air Carrier Captain describes the circumstances leading up to a flight cancellation due to fatigue and weather.

ACN: 1015111 (2183 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1015111
Human Factors : Distraction
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Fatigue

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

During cruise ECAM: R WINDOW HEAT INOP. Window heat controller malfunctioned developing large single crack in outer layer of pane. Completed ECAM actions. Approximately 2 minutes later right window cracked completely. Followed non-ECAM abnormal procedure (windshield crack). Conferred with Dispatch and the decision was made to divert to a nearby airport due to enroute forecast icing conditions. Descended to lower altitude to reduce landing weight, but encountered turbulence. Completed windshield crack procedure prior to landing and followed overweight landing procedures as a precaution (FMGC predicted the landing weight within limits, but may have been inaccurate due to SID). Complied with international divert procedures upon arrival at gate. Repositioned aircraft to domestic gate after clearing customs. Determined crew needed to enter rest after confirming with Maintenance Control that no immediate fix was possible and Dispatch had no options for replacement aircraft. Calculated supplemental rest requirements and coordinated with crew planning for hotel assignment. I would suggest more accurate FMGC fuel prediction would aid crews in determining forecast landing weight quickly to avoid the potential for landing overweight.

Synopsis

An A320 ECAM alerted R WINDOW HEAT INOP accompanied by a large single crack which minutes later developed into a completely cracked window, so the flight diverted to a nearby airport to avoid enroute weather.

ACN: 1014920 (2184 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class E : ZLC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLC
Make Model Name : Gulfstream V / G500 / G550
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class E : ZLC

Person

Reference : 1
Location Of Person.Facility : ZLC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1014920

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 inbound to GPI, 20 miles out was cleared to maintain 100 until ITONE cleared for 30 approach. The weather was VFR and prior to the fix requested to go to the FCA VOR that services the airport instead. At the same time, Glacier Park Tower called for a release on a departing aircraft, a G5 ready for immediate departure. I released the G5 to 090. The A320 was at 100 and now cleared to the FCA VOR. It took at least four or five attempts to insure that the A320 had the clearance. As far as I can remember the clearance was to maintain 100, but with the confusion and the situation I can't be certain. At the same time the G5 departed and with the several attempts to assure the A320 had his clearance I think the two aircraft were blocking each others communication. I noticed now the G5 had departed and was on the wrong code, Data Block not tracking. I noticed how fast he was climbing and heading towards the A320 which at the same time I noticed now was at 090 the same altitude the G5 was at, but the Data Block wasn't tracking on the G5 and at that same moment was when I noticed the A320 assigned altitude of 100 but Mode C showed at 090. At that time the A320 had the airport and terrain in sight. As far as I remember the A320 clearance was 100, with the head on situation and not talking with the G5 I turned the A320 to a 350 heading to protect for the route of flight for the G5. Made communication attempt and got a hold of G5 and asked if he could accept a 190 heading and RADAR identified him, and climbed it to get him above the A320. I turned the A320 to a 020 heading to maintain separation. The A320 was in what I thought was a 090 MEA. The A320 questioned the heading because it turned him away from the airport and towards the mountains. After the turn the A320 lost sight of the airport but had the terrain in sight and was asking for a 360 back to the airport. I knew there was higher terrain East that I was paying attention to and knew I had to turn him back to the west. At the time I hade seven miles separation with the G5 and also had vertical separation so I turned the A320 to a 280 heading back to the airport. The A320 got the airport in sight and was cleared for a visual approach. After all was over, I still had his comment in mind about getting closer to the mountains, which at the time I was 100 % sure he was in a 090 MEA. I also knew the safe altitude on the approach for his quadrant was 088. I thought the assigned 100 and when I noticed the unsafe situation of both aircraft being at the same altitude, I took immediate action to resolve it with out having time to check the overhead maps. I knew he had the terrain in sight and I knew he was at a safe alt. and I thought he was above the MEA. As I looked up after the fact, I realized I had taken the aircraft into a 097 MEA thinking that was the 090 MEA. I was protecting him from the higher MEA East that was 118, but didn't realize at the time he was in a 097, but was still under the impression he was assigned 100 and at that point. I don't know what to recommend other then not trying to do too much at the same time. There is a lot of focus on airport delays, and as controllers we try and keep things moving as fast as we can. We don't like to delay aircraft. It was a situation were it was right at that moment if we could get the departure out safe with the communication problem with the pilot and the confusion it caused and the G5 not on the right code made it into a bigger issue then it should have been.

Synopsis

ZLC Controller described a below MVA event after issuing an altitude believed to be in MVA compliance but that was in fact below required standards. The Controller listed a number of distractions as causal factors.

ACN: 1014442 (2185 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Ferry / Re-Positioning
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Airframe
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Fuselage Floor Beam
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Location In Aircraft : General Seating Area
Reporter Organization : Contracted Service
Function.Maintenance : Lead Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Lead Technician : 5
Experience.Maintenance.Technician : 15
ASRS Report Number.Accession Number : 1014442
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

During a shift change, a floor beam was picked up on to continue installation. While working the installation it was found that the floor beam had been forced down over a Hi-Lok, causing a gouge in the beam in an area where no limits for damage are allowed; the beam was removed. When we came back to work, the same beam had been re-installed. I feel this is a safety issue. This happened at our Contract Maintenance Repair Station. This was on Air Carrier X's A320 aircraft. Two people were heavily involved with this. It was also brought to the attention of the Director of Maintenance (DOM) with no response.

Synopsis

A Lead Mechanic working for a Contract Maintenance Repair Station reports about a new floor beam that had been gouged when forced down over a Hi-Lok during installation on a Customer A320 aircraft. The beam was removed due to no damage allowed, but later found reinstalled.

ACN: 1014158 (2186 of 2908)

Time / Day

Date : 201205

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 1014158
Human Factors : Fatigue

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Attendant
When Detected : In-flight
When Detected : Routine Inspection
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On ready reserve received four hour call out at 5:30 pm from crew Scheduler who told me I had a ZZZ and a ZZZ1 layover. I assumed it was a three day. Only to find when I pulled it up it was a two day with a continuous duty over night on the first night. I called to inquire how this was legal after being available all day, getting into ZZZ after midnight with a 5 hour layover? We of course were provided a hotel room, but by the time we deplaned, waited for in our rooms to basically try to lay down for a quick nap in our uniforms. We were all so fatigued starting out that late at night knowing that we would not be able to sleep until we got to ZZZ the next day landing 10 hours later. In the event of an emergency it is completely unsafe to function with no rest. It is unrealistic to expect a reserve to perform such an trip safely with 4 hours notice.

Synopsis

Reserve Flight Attendant reports receiving a four hour call out at 5:30 pm after being on standby all day. It is soon discovered that the first duty period is a continuous duty over night with minimal time in the hotel and an early morning departure.

ACN: 1014015 (2187 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : DEN.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1014015
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

A320 taxied out with an associated Data Block on the ASDE-X. When I asked CD for a flight progress strip, I was told that the old flight plan had been removed from the system and a new flight plan had been filed. I checked the ASDE-X and the A320 was on the old beacon code and not the new code. Had CD not been aware of the situation I could have taxied the A320 out and the flight would have departed on the old code and clearance in this case, the filed route was the same. I have seen this before, when a proposed flight plan is removed from the NAS, the data is not removed from the ASDE-X. I believe this is a serious flaw. Last summer I was working LC when a flight departed on an old clearance (different route) and did not discover the problem until the flight was airborne and had to ask the Departure Controller to fix the problem. When a proposed FP is removed from the NAS, the data should be removed from the ASDE-X. I can see no reason that this is not the case.

Synopsis

DEN Controller voiced concern regarding the ASDE-X equipment noting that beacon changes are not updated to reflect the correct aircraft/flight, adding that changes in the NAS equipment should be reflected in the ASDE-X logic.

ACN: 1013670 (2188 of 2908)

Time / Day

Date : 201206
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 39000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 1013670

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cabin pressure landing elevator fault, GW XX LBS on lower panel, First Officer NAV Display no map. First Officer FMGC froze. First Officer RAD NAV page froze. Told ATC no RVSM requested lower altitude. Contacted Dispatch and Maintenance Control reset FMGC circuit breaker per Flight Manual procedure, no luck. Captain FMGC worked fine. Captain switched to pilot flying and landed uneventfully.

Synopsis

A320 Captain reports FMGC2 failure at FL390 along with related system faults and failures. An attempted reset of the FMGC is unsuccessful and the flight continues to destination below RVSM airspace.

ACN: 1013031 (2189 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.MSL.Single Value : 3800

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : LGA

Person

Reference : 1
Location Of Person.Facility : N90.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1013031
Human Factors : Other / Unknown

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Primary Problem : Airspace Structure

Narrative: 1

I was working LGA Departure. LGA was departing R31 and the first A320 departed on an assigned SID heading of 340 degrees. On initial contact from the Tower, the first A320 was climbed to, I believe 15,000. As the first A320 was about four miles northwest of LGA I noticed a EWR R22 arrival, a second A320, southeast bound at 4,000 heading toward the first A320. The EWR arrival would normally be at 3,000 at this point on the EWR final but for some reason was not. I instructed the first A320 to turn right to a 360 degree heading to keep these aircraft further apart and instructed the first A320 to maintain 8,000 so as not to interfere with the previous LGA departure off his right front that was climbing out of 8,000. I called the second A320 traffic to my first A320 aircraft but the first A320 did not report having the second A320 in sight. The EWR Controller turned the second A320 aircraft onto their R22 final, without a point out and the two aircraft passed with slightly less than three miles within less than a 1,000 FT vertical separation. I at no point left my airspace nor violated EWR's airspace during this incident.

Synopsis

N90 Controller described a likely loss of separation event when traffic inbound to EWR was higher than required and conflicted with LGA traffic.

ACN: 1012898 (2190 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 30000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZLA

Component : 1

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1012898
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On descent into LAX out of FL300 on the SEAVU2 arrival the FMGC in our aircraft would not hold the programmed airspeed entered in the perf descent page (ATC assigned 300+ knots, then later assigned 280 KIAS). I tried several times to select managed speed and was forced by the aircraft (it was pitching up trying incorrectly to achieve 250 KIAS) to return to selected speed and after three attempts found myself almost 1,000 FT above path. (I wrote up the anomalies upon arrival in LAX).

In order to make my restriction of 17,000 FT/280 KTS at KONZL, I selected full speed brakes and disconnected the autopilot (in order to get full spoiler deflection). We monitored our speed and vertical path and managed to meet the constraint airspeed and altitude at KONZL. As we approached ENGLI we noticed that we were starting to descend below the constraint altitude approximately two and a half miles prior to the fix. 

I pushed V/S to stop our descent and was surprised that we continued a gentle descent. At the same time we noticed that the NAV display was indicating that we were 0.3 miles right of course centerline. At this time the First Officer noticed that we were not following the Flight Director. I realized that after stowing the speed brake we had never reengaged the autopilot and the airplane had been tracking pitch and roll attitude in Control Wheel Steering since the speed brakes had been stowed.

We crossed close abeam ENGLI at approximately 15,600 FT (almost 400 FT below constant altitude). A quick visual scan and scan of the TCAS indicated that we were not in danger of a midair collision. A gentle correction to our descent rate reestablished us on path and the autopilot was reengaged. We advised ATC that we were going to miss our altitude constraint at ENGLI. The rest of the approach was flown within parameters and without incident.

The event was caused by our distraction with equipment malfunction and failure to remain aware of the autopilot disconnect. Because Control Wheel Steering is the automatic default in the A320 it can easily be mistaken for an autopilot engaged mode.

We needed better delineation of pilot flying and pilot not flying duties. Only close attention to the path allowed us to catch it (the autopilot disconnect) as early as we did.

Synopsis

An A320 Captain discussed the effects of flight crew confusion and distraction associated with a malfunctioning vertical navigation programming on flight path control and situational awareness.

ACN: 1012788 (2191 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZMA.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 28000

Aircraft

Reference : X
ATC / Advisory.Center : ZMA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZMA

Person

Reference : 1
Location Of Person.Facility : ZMA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1012788
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 was climbing out of MIA heading south towards Havana Airspace, direct UCA. As per our LOA with Havana, we must call on the land line and hand off each individual aircraft. The A320 was the first of three aircraft I had to hand off, so I let it get up to 20 miles North of the boundary so the aircraft behind would be close enough that Havana could take all three at the same time. As the A320 neared about 20 north of the boundary, I attempted to call Havana to affect the hand off. Havana did not pick up. I tried numerous times between 20 miles north of the boundary and five miles north of the boundary. Still, Havana would not pick up the line. Finally, I said over the line that I would have to turn the A320 if they did not pick up and take the hand off. Havana still did not pick up. Because of Havana's refusal to pick up, I turned the A320 to a 090 heading and stopped their climb at FL290 since the third aircraft going South was going over URSUS (to the East of where the A320 would have crossed the boundary) and was already leaving FL290. The A320 delayed and did not start their turn until they had already violated Havana's airspace without a hand off, point out, or any acknowledgement from Havana whatsoever. When our automation with Havana went into effect we had to change our LOA with them. One of the changes Havana pushed for and got was the removal of a procedure we could use if the receiving controller was unable to be coordinated with for a hand off. So long as an estimate was received and non-RADAR separation existed between the aircraft you were attempting to hand off and all other aircraft transitioning the boundary, we were able to terminate RADAR service and switch the aircraft to the receiving facility's frequency if we were unable to manually hand off the aircraft. With this provision removed we must manually hand off each aircraft transitioning the boundary. Yet, despite this necessity, Havana still remains very difficult to get on the line to affect a hand off. In fact, they only seem to have gotten worse with getting on the line when we need to give them a hand off. Only when they have an aircraft to hand off to us at the same time can they reasonably be relied on to answer the line in a timely manner. Many times they don't even call to hand off their own aircraft to us, leaving us in the quandary of having to abide by an LOA they insisted on, yet many times don't even follow themselves. This situation is absurd. Fortunately, the event I write about in this report happened at a slow time of the day when I could devote my entire attention to this issue, and still the A320 violated Havana's airspace. The A320 was only traffic for one other aircraft I had and the conflict was easily resolved. Later on during the day this could easily not be the case. If we must turn aircraft at the last moment to avoid entering Havana's airspace because they refuse to get on the line and take a hand off a very unsafe situation arises when numerous aircraft are transitioning near URSUS Intersection and Sector 40 is busy with other aircraft and weather. Being able to terminate and ship aircraft when Havana will not communicate with us to do hand offs is absolutely necessary. We need this provision back, especially when many Havana controllers do not even abide by it themselves.

Synopsis

MIA Center traffic incurred into Havana airspace when Havana failed to respond to numerous land line calls regarding hand offs. The reporter noted Havana Center does not comply with the LOA provisions.

ACN: 1012605 (2192 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Weather Elements / Visibility : Fog
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Gear Down Lock
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Hydraulic Lines, Connectors, Fittings
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning
Problem : Design
Problem : Failed

Component : 3

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1012605
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Manuals

Narrative: 1

Aircraft X, an A320, on Final into ZZZ, selected 'Gear Down' and lost Green System hydraulics. Aircraft was towed from runway to Gate. A [hydraulic] leak was noted at the left Main Landing Gear (MLG) Down Lock actuator flex hose, inboard of the two flex lines. A replacement line was shipped in from ZZZ1 and installed per Aircraft Maintenance Manual 32-11-22 'Installation of Lock Stay' actuator hose. After installation, hydraulics was serviced to required level and ran engines and operated flight controls to move hydraulics though Green System. A Functional Test was required AMM 32-31-00, but thought brace tool was gear safety collars and the Functional Test was completed. Green System hydraulics was again serviced to proper level. Aircraft released to service. [I] was called by our ZZZ Maintenance Manager that Aircraft X had lost Green System hydraulics's and was diverting to ZZZ2. [Realized] I misinterpreted a step in the Functional Test. Then looked into what this brace tool was. 
[Recommend]: 
1) Confer with more Aviation Maintenance Technicians AMTs on understanding [AMM] terminology. 
2) This job, I have seen been accomplished and no problems occurred as this. 
3) When I selected the AMM reference, AMM 32-31-00-710, this reference was right below the one I should have selected, AMM 32-31-00-720.

Callback: 1

Reporter stated he has 29-years of Mechanic experience with two airlines. Many of the misunderstandings in reading Airbus Aircraft Maintenance Manuals (AMM) involve Airbus terminology compared to Boeing AMMs. The purpose of the lock stay brace tool was not explained in the Airbus AMM. Only that a procedure had to be performed involving the Lock Stay Down Lock actuator because the hydraulic hose to the down lock actuator had broken away from the actuator in flight where the flex line attach fitting connects to the hose. The separated hose caused an excessive loss of Green System hydraulic fluid.

Reporter stated their Line Station does not have any facilities; no Hangar to do gear swings and did not have a brace tool to accomplish cycling of the lock stay down lock actuator after they had replaced the broken flex hose. He selected one of two alternate procedures in the Airbus AMM that allows for installing gear pins, 'slugging' the main gear and moving the gear handle up and down to check for hydraulic pressure and any leaks at the actuator hose fittings on the down lock actuator; without having to do an actual gear swing. Unfortunately he selected the wrong alternate procedure. 

Reporter stated that since the broken flex line caused the loss of hydraulic Green System hydraulic fluid, Airbus requires a "priming" of the Green System by actuating the down lock actuator numerous times with the brace tool installed, which was not done since they did not have a brace tool at their Station. The same hose seems to break or leak fairly often, probably due to the constant flexing of the hose at the actuator connection and the fact that the hose is exposed when the gear is extended. Although they did operate the gear doors, flight controls, run the engines and service the Green System to required levels, the A320 Green System quantity dropped and the flight diverted halfway to their destination. The diversion was not due to actual hydraulic fluid loss. The Green System just required more fluid to replenish the System. They now have a brace tool at their Line Station.

Synopsis

A Line Mechanic reports that misunderstandings of terminology used in Airbus Aircraft Maintenance Manuals (AMM) compared to Boeing AMMs and the lack of an Airbus Brace Tool led to an A320 diversion due to low Green System hydraulic fluid. A hydraulic flex hose to the Lock Stay, Down Lock actuator on the left Main Gear had broken on a previous flight.

ACN: 1012507 (2193 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1012507

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

When we checked in for our morning departure, we were advised the aircraft was being towed into the gate due to a maintenance issue. When the aircraft finally arrived at the gate, Maintenance advised us a line on the Green Hydraulic System had failed and it would be over five hours before the aircraft would be repaired. We finally pushed, and had a normal departure. After reaching cruise and upon performing a HEFOE (Hydraulic Electrical Fuel Oxygen Engine) check, we noticed the green system quantity was somewhat lower than normal. As we proceeded westbound, it was apparent the quantity indicator was indeed decreasing. We notified Maintenance of the condition and asked if we could turn off the green system pump thinking that maybe the suspected leak would decrease in intensity, but was told to abide by the QRH procedures. 

By this time we had already reviewed the QRH, and had not found any procedure for our situation. When the quantity indicator was within 1/8 inch of the amber low level line, we asked Dispatch if maybe a diversion would be appropriate. They concurred, so we told Center we were going to divert for a precautionary landing. Apparently someone declared an emergency for us as the equipment met us upon landing and followed us to the gate. Maintenance was still troubleshooting the situation when we left the airport.

Synopsis

A320 Captain reports departing late due to a maintenance required repair of the Green hydraulic system. Once in cruise the Green system quantity is found to be decreasing and the crew elects to divert.

ACN: 1012233 (2194 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12700
Experience.Flight Crew.Last 90 Days : 105
Experience.Flight Crew.Type : 4778
ASRS Report Number.Accession Number : 1012233

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23500
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1012237

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At 100 KIAS the First Officer airspeed tape went red, lost airspeed indication. First Officer, who was pilot flying, announced "airspeed failure, take controls". Captain immediately took controls and executed a normal takeoff profile. First Officer reverted to pilot not flying. Captain asked me to complete after takeoff checklist, coordinate for a return with Departure Control and Dispatch. With autopilot 1 engaged, FMGC programmed, all checklists complete and on vectors, First Officer's air data switch was selected to AD3 with no restoration of speed tape. Switch was returned to normal and back to AD3 with a successful restoration of airspeed indication. [We] executed an uneventful landing.

Narrative: 2

Just prior to 100 KTS, First Officer (pilot flying) announced "Airspeed failure, Take Priority"; I took control and decided the safest course of action was to continue the take off vs. a high speed rejected take off. All normal procedures and checklists were followed. After the Takeoff checklist was completed the First Officer selected air data first officer on 3. The airspeed still showed failed. We decided the prudent and safest course of action was to land at the nearest airport--returning [to departure airport]. A non-emergency request to return was made, we notified Dispatch, completed all associated checklists and a normal uneventful landing was accomplished. It should be noted that a second attempt selecting First Officer on 3 did restore his airspeed indications.

Synopsis

A320 flight crew reports First Officer's airspeed failure at 100 KTS during takeoff. The Captain assumes control and continues the takeoff. First attempt at switching the First Officer to AD3 is unsuccessful and the crew elects to return to the departure airport, although the second attempt is successful.

ACN: 1011947 (2195 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 12500
ASRS Report Number.Accession Number : 1011947
Human Factors : Other / Unknown
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I had finished an all night flight [redeye] back to base early this morning with poor, at best, rest at the west coast departure city. [I] drove home 2.5 hours after the all night flying. Tried to rest with very broken sleep then was disturbed 4.5 hours later by crew scheduling to be assigned a pre-dawn short call. [I] could not get back to a rest state until six hours after to no real avail then received a call from scheduling assigning me an early morning flight. There would be no SAFE way to be rested to perform the duties asked of me without placing myself, my crew, the aircraft and the flying public in harms way. It is truly tragic that the company fails to take the initiative to thwart this kind of fatigue problem! A suggestion would be to prohibit any assignment before a minimum of a 24 hour adjustment/rest period is concluded after an all nighter for the safety of the entire operation and traveling public.

Synopsis

An Air Carrier Captain reports being unable to get proper rest after returning from an all night flight due to calls from Crew Scheduling.

ACN: 1011826 (2196 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 300
ASRS Report Number.Accession Number : 1011826
Human Factors : Fatigue
Human Factors : Situational Awareness

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Current air carrier scheduling practices allow for trip sequences (pairings) to be constructed that may not be consistent with the Fatigue Risk Mitigation Plan (FRMP). A demanding pairing was competed. Fatigue was declared for a subsequent pairing. Through the generation of required air carrier internal reports for that declaration, I became aware of an inconsistency between the flight time/duty time regulations governing this pairing and our newly implemented Fatigue Risk Mitigation Plan (FRMP). This inconsistency has been reported to the airline, the Federal Aviation Administration, and the department of transportation.

Synopsis

An A320 Captain refused a trip because after flying a trip the subsequent trip was not constructed in accordance with the FAA Fatigue Risk Mitigation Plan (FRMP).

ACN: 1011446 (2197 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Dawn
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Recirculation Fan
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11700
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 2148
ASRS Report Number.Accession Number : 1011446
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 1600
ASRS Report Number.Accession Number : 1011461

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During flight, pilots noticed unusual noise from what they thought was the air conditioning system. Noise sounded like a loud whine and continued to get louder until stopping suddenly. VENT EXTRACT FAULT ECAM message appeared concurrently with switch fault light and strong electrical odor/smoke in cockpit. Pilots donned oxygen masks and established communication with each other and cabin crew. QRC was accomplished as well as reference actions and checklist for VENT EXTRACT FAULT ECAM. First Officer flew plane to the divert airport while Captain ran checklists and managed situation. Captain directed First Officer to continue descending at 300 KTS below 10,000 FT to expedite arrival at the divert airport. After visual scan by Crash/Fire rescue personnel, aircraft taxied to gate. Cabin crew was briefed during descent and two PA announcements were made to passengers to keep them updated. Captain's overall impression is that everyone did a great job. The First Officer performed admirably, the Lead Flight Attendant stated during the debrief his complete satisfaction and thoroughness in the Cockpit/Cabin communications. ATC provided expedited handling and the ARFF crews were by the runway during our landing.

Narrative: 2

During cruise flight, noticed a noise followed by a VENT FAN ECAM. Cockpit immediately filled with an electric, burning type smell. Did not see any smoke. O2 mask donned, crew communication established. Ran ECAM, no effect. Started to divert, I stayed on controls and handled the radio. Captain ran Smoke/Fumes Checklist. Smell did not dissipate. Descended through 10,000 FT and briefly kept speed up above 250 to expedite in case smell turned into actual smoke or fire. Cleared for visual 1L, uneventful landing with masks still on. Crash, Fire, Rescue inspected aircraft, no signs of smoke.

Synopsis

An A320 developed an air conditioning system noise followed by a VENT EXTRACT FAULT ECAM and a strong electrical odor in flight. An emergency was declared and the QRH complied with including a diversion to a nearby airport.

ACN: 1010855 (2198 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 28000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization Outflow Valve
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1010855

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 143
Experience.Flight Crew.Type : 6871
ASRS Report Number.Accession Number : 1010865

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While in cruise at FL280 we lost Auto Cabin Pressurization. As the cabin altitude began to climb through 9,500 feet, we began an emergency descent to 10,000 feet where we complied with ECAM and Emergency Procedures. We continued to our destination where we completed a normal approach and landing.

Narrative: 2

We advised ATC and started an emergency descent and then descended to 8,000 feet MSL. Consulted with Maintenance Control and Dispatch. Selected Pressurization System 2 and regained control of pressurization.

Synopsis

An A320 Flight Crew performed an emergency descent after noting the cabin altitude approaching 10,000 feet. When level, they performed appropriate ECAMs and checklists, switched to an alternate system and continued to their destination.

ACN: 1010761 (2199 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : INS / IRS / IRU
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 1010761
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 25
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1010770
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff roll at approximately 100 KTS, the First Officer lost airspeed indications. Numerous dings/ECAMs and autothrottles went off. Captain called to abort takeoff at approximately 125 KTS. Fire trucks called as precautionary. Brake temperatures reached 725 degrees. Taxied back to gate.

Narrative: 2

Previous crew refused aircraft. Aircraft had deferrals for ADRU 2 bad and other issues. Flew to current airport with no issues. On takeoff roll at estimate 100 KTS, First Officer lost airspeed indication and numerous dings and ECAMS.

Synopsis

An A320 First Officer's airspeed indication failed with numerous ECAM and aural alerts so the Captain rejected the takeoff and returned to the gate. The Aircraft previously had an ADRU 2 deferral.

ACN: 1010565 (2200 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : ZZZ
Maintenance Status.Released For Service : Y

Component

Aircraft Component : Speedbrake/Spoiler
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 1010565
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Right hand spoiler 2, 3, and 4 failed to retract during approach, with no associated ECAM memo. Noticeable aircraft yawing, the Beta Target "still green" was 1-1/2 units out to the right. The yawing became noticeable passing 300 feet. Identified on the lower ECAM the RH side spoiler panels 2, 3, 4 were extended. This was accompanied by a continuous noise from aircraft right. Captain was PF, autopilot was off, and rudder trim is normal "0.2R". A normal landing was made and upon touchdown ground spoilers deployed on schedule. After landing all spoilers retracted normally. I followed up with Captain, and asked the question about a low altitude go around with a single side spoilers deployed in regards to Airbus flight characteristics, if the spoilers did not retract with spoiler deactivation.

Callback: 1

The reporter could not add any information except to state that the company is looking into it. The subject of disarming the spoilers was briefly considered during approach and rejected since the aircraft was stable and the possible negative consequences were unknown.

Synopsis

A320 First Officer reports noticing right hand roll spoilers 2, 3, and 4 are extended during approach with noticeable aircraft yawing and a continuous noise. There were no ECAM messages and the Beta target was well displaced out to the right but remained green. Captain elects to continue the approach to landing where all spoilers extend normally on touch down and retract normally when disarmed.