ACN: 1010235 (2201 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : B737-500
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Facility : SFO.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1010235
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

An A320 was cleared for take off with traffic about 2 3/4 miles on final. The aircraft took forever to start their roll. With a 15 KT headwind, I used 1 1/2 miles as a cutoff with some movement and clearance acknowledged. The A320 still was not accelerating, when I cancelled their take off clearance. As such, it was at this time, the aircraft was powering up and then down as they acknowledged the aborted departure. Of note, the aircraft was bound for one place, and the next departure in sequence was same company, same model, going to another. The performance of the next departure roll was incredibly different. Both rotated at about the same take off distance. This is mind boggling when observed, and definitely a safety hazard for an airport with intersecting runways. These pilots need to be performing more similar in this situation, as it is undoubtedly an inconsistency in flying technique. It is unacceptable that this occurs, as this has involved unsafe aborted departures, and unfair go around's for arrivals.

Synopsis

SFO Controller voiced concern regarding the very different same company pilot techniques during takeoff's, the reporter noting performance within the same airline should be more consistent.

ACN: 1009425 (2202 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1009425
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1009428
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the Captain and flying pilot on takeoff roll on Runway 19R. At the 80 KTS call, I checked my airspeed indicator and it was normal and indicating approximately 80 KTS. Then a few seconds later at an airspeed somewhere between 80 KTS and V1, I glanced back down at my airspeed indicator and it was showing below 80 KTS. As Captain, I exercised my command decision to reject the takeoff. The rejected takeoff maneuver was completed, and we taxied clear of Runway 19R. My First Officer informed Tower, consulted the QRH and turned on the brake fans. I made a PA announcement to the passengers and stated the reason for the rejected takeoff. I also informed them that we would be returning to the gate after our brakes had a chance to cool. We then informed Operations and Maintenance and returned to the gate. After consulting with Maintenance, we deplaned the passengers without incidence. I then called Dispatch and informed the Dispatcher of the event. Systems Scheduling then decided to cancel the flight. The event occurred due to a faulty and inaccurate airspeed indication.

Narrative: 2

At approximately, V1 the Captain called "reject" and said something about his speed and rejected the takeoff. I advised the Tower we were aborting the takeoff. The Captain let the autobrakes decelerate the aircraft until a safe speed and finished the braking manually. The Tower asked if we needed any assistance, we declined and they told us to contact Ground clearing the runway. We cleared on Taxiway W1. At this time I advised we needed to go back to the Gate but needed to stop and let the brakes cool. We pulled off on the taxiway to let the brakes cool and I advised Operations we needed to return and have Maintenance meet us. I got out the QRH and reviewed the hot brake section with Captain. After about 10 minutes with the brakes cooling down we proceeded to the Gate and met with maintenance. On the taxiway while we waited for the brakes to cool the Captain said his airspeed indicator was only showing 80 KTS when he glanced down at it. Up to the time of the reject my airspeed indicator was showing normal acceleration. I believe the Captain rejected the takeoff due to the fact he didn't think there was sufficient airspeed to continue.

Synopsis

An A320 Captain rejected the takeoff near V1 because his PFD airspeed indicated around 80 KTS while the First Officer's indicated near V1.

ACN: 1009085 (2203 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 9000

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Fuel Booster Pump
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 1009085

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While on arrival ECAM appeared indicating possible loss of green hydraulic system. We declared an emergency and continued the arrival. After Captain handled the ECAM, we got another ECAM indicating loss of fuel pump pressure on right main fuel tank. Captain handled the second failure, the rest of the arrival and landing was uneventful. Aircraft was met by crash crew, had to be towed to the gate.

Synopsis

A320 First Officer reported losing green hydraulic system and a fuel boost pump on arrival. They declared an emergency and were towed to the gate.

ACN: 1008893 (2204 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 13000

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZZZ

Component

Aircraft Component : Air Data Computer
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Lighter-Than-Air
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 1008893
Human Factors : Workload
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Last 90 Days : 200
ASRS Report Number.Accession Number : 1008895
Human Factors : Workload
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Became Reoriented
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

We were cleared to descend to cross an en route intersection at 10,000. During the descent and at 280 KTS I noticed my airspeed decreasing. I assumed the autothrottles were not working properly and switched off autothrottles. When speed continued Towers minimum selectable I selected autopilot off and pushed the nose over. The First Officer was busy as this was our third or fourth route change and two runway changes and he was busy. Then he noticed on the descent checklist that we had degraded to CAT 3 single and was investigating that anomaly. In the meantime I was struggling to control the aircraft because my airspeed was wrong and I was getting close to overspeeding the aircraft by pushing the nose over and advancing the throttles. The First Officer called airspeed. He was referring to the impending overspeed on his side - I thought he was referring to the impending stall that I was sensing on my side. In the heat of this struggle we crossed the intersection at 11,700 vice 10,000 assigned and told ATC that we could not make it as we had a problem. After we discovered whose instruments were correct via reference to the standby instruments I switched to Captain on Air Data 3 and switched to Autopilot 2 and autothrottles back on. It was very confusing and now we were behind. I was not confident that the speeds were correct and the First Officer was very busy, but doing a great job as pilot not flying. He was reading the EMERGENCY QRC MACH AIRSPEED UNRELIABLE, setting up the FMGC for the new runway and communication with ATC. After confirming the aircraft was under control we proceeded hurriedly, but uneventfully to touchdown on 27L.

Callback: 1

The reporter stated that this event lasted about 60 seconds and what ran through his mind was the A330 accident. The short period of this event was very confusing because there were no other indications of an airspeed malfunction nor any clue as to what the crew should do other than begin the AIRSPEED UNRELIABLE procedure. He was so shaken by this event that he removed himself from the remainder of the trip.

Narrative: 2

We were given a descent to cross the intersection at 10,000. As we started our descent I began the approach/descent checklist as the Captain was the pilot flying. Shortly thereafter we received a runway change from 27R to 27L. I entered the new runway into the FMGC and continued the checklist. Upon reaching the "Status check" part of the checklist I noticed that CAT three single was displayed on the ECAM status page. Neither I nor the Captain knew why. I had just entered the new runway and I was trying to figure out if there were any correlations to the new approach. While I was trying to figure out why the Captain disengaged the autopilot and soon after disconnected the autothrottles, we were passing through 13,000 FT. The Captain then advanced thrust and then went to climb thrust and engaged the autothrust stating the autothrust was not working. I didn't understand the statement because it was commanding idle thrust and we were in open descent mode. Again the autothrust began reducing trust toward idle. We were at around 280 KTS and 11,000 FT when the Captain disconnected autothrust and went to climb thrust. As the airspeed climbed I asked the Captain to watch the speed and he said "I know." We climbed to 11,300 FT and airspeed was still increasing. I told the Captain that we were going to overspeed. That is the time we figured out that his airspeed indicator was incorrect with cross checking all three indicators. Captain switched to Captain on AIRDATA 3; the airspeed overspeed by about 5 KTS. The throttles were brought to idle and speed brakes deployed. By this time we were too high to make the intersection crossing and we notified ATC. We were given a descent to 9,000 FT and we were given a heading. We were later slowed and given a heading to intercept 27L. With all the confusion we passed slightly through the LOC intercept and we were given a heading of 310 to join. Total 1 1/2 dot south of centerline; we then centered LOC and the remainder of approach was normal.

Synopsis

An A320 Captain's airspeed decreased toward V1s during descent, so he disconnected the autopilot and autothrottles to manually increase airspeed when the First Officer discovered the aircraft was overspeeding. They determined the Captain's airspeed was in error and the air data computer was at fault.

ACN: 1008223 (2205 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 1008223
Human Factors : Fatigue
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

This is a fatigue issue. Our flight landed at after delays due to thunderstorms in the terminal area. Total duty day at that point, went to hotel layover for minimum 9 hours rest at hotel. Got to hotel [and] got to rooms shortly thereafter. Call to Crew Desk explained min rest and would attempt [in 9 hours later] van next morning back to airport. Scheduled departure was [later] and would be delayed pending our arrival and required flight planning and aircraft preparation. Had alarm set for maximize rest. Had 7 hours of flight time to complete the next duty period for day 3 of a 3 day. [I] was awakened by Crew Desk calling hotel room phone [2 hours before my alarm] to notify me of scheduled departure slipped to [an hour later]. [I] was in a deep sleep and not expecting an interruption of minimum crew rest. Was unable to go back to sleep. Reported for duty and notified Crew Desk of fatigue shortly thereafter. The relevant points to me are as follows: Minimum Rest at a hotel is usually after a longer than originally scheduled duty day due to weather or maintenance delays. The crew is, in most cases, exhausted by the time they get to the hotel. Rest is always crucial, but particularly on minimum rest layovers, with a late arrival, after a long duty day, with a lot of flying the subsequent day. When that rest is interrupted, it's a recipe for fatigue. 

The Crew Desk states that they must notify crews of delays to their departures to keep the duty clock from starting too soon and as a courtesy to the crews. There is no place in the contract that clearly spells this out about notification, or best time to notify. At a minimum, on min rest layovers, the Crew Desk needs to be made aware of this with a note in the computer, giving them a heads up to not call until toward the end of the 9 hours to notify the crew of any delays. Waking a crew up to essentially notify them they have more layover, on a min rest when sleep is the most important asset they have right then, is counterproductive for all involved, for obvious reasons. I feel after experiencing this scenario that there needs to be clear guidance to the Crew Desk on notification of crews on min layovers. All involved need to understand the importance of uninterrupted rest for as long as possible in these situations. Crews are responsible to be fit to fly. Interrupting a minimum layover rest greatly raises the chances that they won't be, and may leave them with only one responsible option left, to declare themselves fatigued. This doesn't help out the Crew Desk or the airline's operations. It's a lose/lose. Guidance is needed.

Synopsis

A pilot reported that during a minimum rest period after a 13+44 hour duty day, the Crew Scheduling Desk called to notify him about extending his rest which interrupted and prevented further rest so he called in fatigued.

ACN: 1008159 (2206 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1008159
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff received ECAM message Auto Brake Fault Brakes Released. Phone patch to Dispatch, conferred with Maintenance, declared emergency, burned off fuel to landing weight [and then] landed.

Synopsis

An A320 ECAM alerted AUTO BRAKE FAULT BRAKES RELEASE so an emergency was declared and the flight returned to the departure airport.

ACN: 1007895 (2207 of 2908)

Time / Day

Date : 201205
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : OKC.Airport
State Reference : OK
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : OKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class D : PWA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1007895
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 248
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 1007899
Human Factors : Other / Unknown
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Brief, checklist, start, and all phases leading to the incident were in accordance with SOP and no issues. I was the Pilot Flying. Approach had us at 4,000 FT and on a vector to intercept final for Runway 17R at OKC. We reported the field in sight and were cleared for the approach and to contact Tower. I was completely visual at this point and locked onto the PAPI at what turned out to be Runway 17 at Wiley Post. We were in a gentle descent to 2,800 FT (IAF altitude at OKC 17R) when OKC Tower came on frequency and said it looked like we were lined up on Wiley Post, and to climb to 3,000 FT and gave us an appropriate heading. We complied with the instructions and picked up OKC 17R and landed without incident. There were no TCAS RAs/TAs, the aircraft was always in a stable configuration, and there were no additional instructions from the Tower.

Narrative: 2

Upon reaching 2,800 FT the pilot not flying commented that the ILS DME for Runway 17R still showed 12 NM, but that the runway seemed very close. We then both commented that the ILS glide slope showed low while the VASI showed high.

Our company procedure of backing up all approaches with available navaids helped the crew realize something was not right during the approach, but it was the Tower that clarified our visual error.

Synopsis

Cleared for a visual approach to OKC, the flight crew of an A320 mistook PWA for OKC and began a descent to land there. Approach Control noted their track and altitude and brought the error to their attention.

ACN: 1006947 (2208 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Place

Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 1006947
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Other

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Dispatch
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Staffing
Primary Problem : Company Policy

Narrative: 1

I'm currently a line holder on the afternoon shift. Today due to the Company's inability to appropriately staff the office I was scheduled to work the first hour on Desk X and then move to the ATC Coordinator Position when another afternoon shift Dispatcher arrives an hour after me. When I accept the shift on Desk X I have approximately 6 releases due in the first hour of the shift. At this time I am also signing into the applications on the desk in order to maintain situational awareness and begin my shift briefing. Due to the Company not being able to adequately staff the office and thus having me move to the ATC Coordinator position after the first hour this does not give me sufficient time to comply with FAR 121.599 prior to having to accomplish these releases before I am expected to complete a shift turnover to the oncoming Dispatcher. This situation also sets both dispatchers up to fail as I don't feel I was adequately self-briefed to provide a comprehensive shift turnover to the incoming Dispatcher. At the time there was significant weather impact at a Mid West airport with thunderstorms. When the afternoon shift Dispatcher arrived I had not been able to complete the releases on the worksheet in the one hour which now puts this Dispatcher behind on the workload when they assume operational control of the shift.

Synopsis

A Dispatcher reported that Company policy has their Dispatch Center under staffed and unable to complete the FAR dispatch duties particularly at shift change and brief times.

ACN: 1006907 (2209 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZSU.ARTCC
State Reference : PR
Altitude.MSL.Single Value : 31000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class A : ZSU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : A330
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 129
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class A : ZSU

Aircraft : 3

Reference : Z
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : A319
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZSU

Person : 1

Reference : 1
Location Of Person.Facility : ZSU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1006907
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1007121

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Similar sounding call signs are going to cause a serious accident if action is not taken. I estimate 20 plus aircraft in my sector during the time of incident. The sector should have been de-combined but the FLM was not engaged. I issued a descent clearance to Air Carrier ZXX from FL330 to FL310 to get them under the A330 who was descending to FL320 landing MDPC. Opposite direction to Air Carrier ZXX was Air Carrier XXX at FL340 who took the descent clearance for Air Carrier ZXX, FL340 descending to FL310. So Air Carrier ZXX is still at FL330 with the A330 converging and descending through FL340 for FL320. It was too late to stop the descent of the A330. I screamed at Air Carrier ZXX to begin the descent thinking the pilot thought it was a PD. Air Carrier XXX heard this and realized what was going on meanwhile he was opposite to Air Carrier ZXX. Now these two were both descending to FL310. Air Carrier ZXX barely got under the A330 in time and now Air Carrier XXX started climbing back up to FL340. Air Carrier XXX said I had told them initially to descend to FL310 and they had read it back. Regardless who did what, that was very close and unsafe and was a direct result of this similar sounding call sign madness that plagues our industry. Please make this situation a top priority to mitigate. ZSU has the same call signs everyday and it is reported dozens of times and nothing happens.

Narrative: 2

During flight we were advised by San Juan Center that Company Flight ZXX was on the same frequency. We acknowledged, as did Flight ZXX. Approximately 5 minutes after level off at FL340, ATC issued a descent to FL310 for traffic to Flight XXX (our callsign). We acknowledged and used our callsign and begun a descent to FL310. About three minutes later, as we were leveling at FL310, we heard ATC question Flight ZXX about why they had not started their descent. They said they had not been given a clearance and were immediately directed to descend and turn for traffic. I realized that the Controller had given the descent to us instead of Flight ZXX. I questioned ATC as to the altitude he wanted us at and pointed out that he had given Flight XXX the descent not Flight ZXX. He immediately told us to return to FL340 and expedite through FL320. The Controller got confused by the similarity in the call signs. Maybe we could investigate our flight numbers and routes to try to reduce the number of similar call signs in close proximity to each other.

Synopsis

ZSU Controller voiced concern regarding the continued use of similar sounding call signs by aircraft operating within their airspace citing a recent conflict resulting from similar sounding call signs.

ACN: 1006841 (2210 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 300

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : LAS

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAS
Make Model Name : Learjet 60
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : LAS

Person : 1

Reference : 1
Location Of Person.Facility : LAS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1006841

Person : 2

Reference : 2
Location Of Person.Facility : LAS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1006844
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Unplanned go-around [we were] on Runway 25 by an A320 who was under my control at the Local Control 1 position. Traffic departing 19 complex under the control of Local Control 2. The A320 advised going around, I immediately looked for traffic off 19's. Traffic was rotating midfield, the departure, P180, was to the left of the A320 on a 175 degree heading. I issued traffic on Lear converging and unkeyed to try to hear what Local Control 2 was instructing his traffic to do. My assistant instructed Local Control 2 to have his aircraft stay low and pass below the A320. I keyed up to tell the A320 what the Lear was going to do but visually, saw the Lear climbing and banking right. I instructed the A320 to turn left and established Visual Separation with Lear Jet. Local Assistant was coordinating with L30 and he told me to turn the A320 to 240 degrees, climbed the A320 to 6,000 confirming still visual separation with Lear Jet. Told the A320 to contact L30; the A320 came back to my frequency asking what runway he can expect. I advised Runway 25L and told to contact departure again. During the time of this situation a Traffic Management Coordinator was the CIC. TMU usually monitors Ground Control to help them anticipate flow times. I don't know if the Traffic Management was monitoring a position. I feel if a regular controller was staffing the CIC position they might have caught the go around earlier. When I advised Local Control 2 the A320 was going around I didn't get an immediate response from him that he heard me. Perhaps a Supervisor/Controller CIC would have yelled louder at him. Also a Supervisor/Controller CIC might have taken more immediate action to have Local Control 2 cancel the take off clearance of another A320 who was departing behind the Lear with visual separation.

Narrative: 2

I was working Local Control 2 during a slow session. I was issuing takeoff clearances to aircraft departing Runway 19L. I issued a take off clearance to a Lear 60 Runway 19L. I gave a subsequent takeoff clearance to an A320 using visual separation from the LR60 ahead of him. After giving both clearances, I heard the Assistant Local Control 1 Controller say that they had a go-around off Runway 25L. I saw the go-around traffic about midfield Runway 25L. I immediately called the traffic to the LR60 who was about 3/4's of the way down Runway 19L and airborne and requested that he "stay low" or "go low." The pilot of the Lear immediately reported the traffic in sight and started to climb at a remarkable rate. I told the Lear to maintain Visual Separation on the traffic when I noticed the Airbus start to begin a left turn. I told the Lear that the airbus was in a left turn now below him and to his left. I then instructed the Lear to contact Departure. With the LR60 now approximately 1.5 miles south of the airport with the go-around approximately 0.5 miles in trail and lower, I issued traffic to the subsequent departure off Runway 19L. The departure reported the traffic in sight, I told him to maintain visual separation with the traffic and to let the Departure Controller know that he had him in sight. I then told the A320 to contact Departure. I don't really know if there is a solution to this issue. The late go-around from the other A320 certainly didn't help the situation.

Synopsis

LAS Controllers described a developing conflict involving a go-around on Runway 25L and departure from the Runway 19 complex. The reporter indicated that a more experienced CIC may have completed the need internal coordination more quickly.

ACN: 1006682 (2211 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MSY.Airport
State Reference : LA
Altitude.MSL.Single Value : 11000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : MSY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : MSY

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 1006682
Human Factors : Other / Unknown

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

On arrival over AWDAD Intersection level at 11,000 with autopilot on, advised of VFR traffic at 10,500 at 1 o'clock position. About 5 seconds later traffic popped up on TCAS as a TA. Aircraft began left turn over AWDAD and then we received a climb RA. I turned off autopilot and began climb while leaving airplane in the turn. Captain reported RA climb. We climbed to about 11,400 and got clear of conflict. I never saw him visually and last I saw on TCAS was traffic -400. We returned to 11,000 and resumed the arrival.

Synopsis

A320 First Officer experiences a TCAS RA at 11,000 FT with VFR traffic 500 FT below. The traffic was reported by ATC and the crew was clear of conflict at 11,400 FT.

ACN: 1006597 (2212 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component : 1

Aircraft Component : Fuel Line, Fittings, & Connectors
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Wing Fueling Panel
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1006597
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Primary Problem : Human Factors

Narrative: 1

This morning I, a Maintence Manager, was waiting to board a company flight in the gate area when I observed an Aircraft Contract Fueler removing the bonding cable between the aircraft and the fueling hydrant cart BEFORE disconnecting the fueling hose from the aircraft. This is a safety related incident with a fairly high risk factor. It is also opposite to the required procedure of removing the bonding cable AFTER the fuel hose has been disconnected to avoid static electricity sparking at the fuel connection when the hose is removed. Reference company Air Carrier Fueling Manual and FUEL form.

Synopsis

A Maintenace Manager observed Contract Fuelers not in compliance with company procedures during the fueling operations of a company A320.

ACN: 1006489 (2213 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Distance.Nautical Miles : 10
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 22L
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 1006489
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9800
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 6800
ASRS Report Number.Accession Number : 1006269
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On a vector to intercept localizer, both FMGC's dropped all data and we lost NAV radios, maps, cabin pressure controller and managed speed. Declared emergency, took vectors to work the problem. First Officer emailed Dispatch looking for VFR. I manually inserted the ILS into the RMP, got a good ident, depressurized the aircraft, and flew an ILS with selected speed to a normal landing.

Narrative: 2

Approach assigned a turn off the arrival, vectoring for the ILS at 4,000 FT in IFR conditions. With turn to heading, I saw that my display now said map not available. I checked and saw that Captain's display also said map not available. Neither FMGC displayed navigation information. Then ECAM alarmed Cabin Landing Elev not set - use manual pressurization. We asked for vectors off approach to try to investigate situation. ACARS to Dispatch for closest VFR weather; Captain asked ATC for weather at nearby airports and declared Emergency Aircraft. Manually tuned ILS available on Captain's radio and set inbound course. [We] received good ident and DME asked for vectors to ILS. Manually depressurized aircraft to landing elevation; used selected speeds for approach and landing.

Synopsis

A320 flight crew reports dual FMGC failure during approach in IMC. The ILS frequency is successfully tuned using the RMP and the flight lands at destination after manually depressurizing the aircraft.

ACN: 1004745 (2214 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1004745
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Facility was mid-way through a 48 hour ERAM test. Received position briefing and relieved RADAR position. Went to invoke "pref-set" (Preferential Settings) for this sector and there were none. I had apparently overwritten Sector X "pref-set" the night before when using an atypical sector combination. I was using the previous Controller's "pref-set" until I could update my own with 21 plus aircraft under my control. Air Carrier Y was in the middle of a 2 gate double arrival stream. Air Carrier X was an overflight "swimming upstream," requested a descent clearance to FL310 from FL350. Several of the south stream aircraft had been descended to FL320 to clear traffic; now given FL300 for Air Carrier X new requested altitude. The plan was to wait for descent on Air Carrier Y until past Air Carrier X at FL350, then Air Carrier X requested lower. Air Carrier Y needed descent due to proximity to arrival gate and Air Carrier X's lower altitude. Air Carrier X was given a left turn 15 degrees for his descent and cleared to FL330 interim. Air Carrier Y was descended to FL300 then given a gate crossing clearance of FL190. When Air Carrier X was established north of traffic, cleared to FL310; the RADAR Associate updated Air Carrier X route. The route was close to Air Carrier Y, but appeared to clear, so Air Carrier X was cleared on course to preclude a further conflict with aircraft on either arrival stream. Air Carrier X passed Air Carrier Y, both aircraft descending and appeared to clear with minimum separation of 5 NM. Recommendation, take more time to establish that correct ERAM "pref-set" is available. Using another Controller's "pref-set" is definitely a distraction and disadvantage. It would have been more prudent to clear Air Carrier x on course "reaching FL310", giving more time and altitude margin to ensure separation.

Synopsis

Enroute Controller described a probable loss of separation event during a ERAM testing period, the Controller listed the lack of Preferential Settings ("pref-set") availability as one causal factor.

ACN: 1004175 (2215 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Antiskid System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Component : 3

Aircraft Component : Main Gear
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1004175
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Primary Problem : Human Factors

Narrative: 1

Contract Repair Station Vendor overhauls all Landing Gear assemblies for 'Time' gear change [for our Air Carrier]. Receiving Inspection required of all components. One task requires confirmation of Main Landing Gear (MLG) wiring harness routing in the axles. Found the Left-hand (L-H) MLG wiring incorrect. Outboard (O/B) harness routed to Inboard (I/B) and I/B routed to O/B position. This wiring concerns the Anti-Skid Transducers. See maintenance write-up for Corrective Action. Vendor Repaired Part.

Callback: 1

Reporter stated his Station has started doing Landing Gear changes and Receiving Inspections of the Landing Gears for their A320/319 aircraft when they arrive from the Contract Repair Station Vendor. He works as an Inspector and does the wiring inspection prior to the landing gear being installed. After he found the Main Landing Gear (MLG) Anti-Skid electrical wiring harness cross-connected, mechanics were able to snake and reroute the harness wires without having to separate any components. The same incorrect wire routing on MLGs received from overhaul Vendors has happened before.

Synopsis

An Air Carrier Inspector, reports finding a left-hand Main Landing Gear (MLG) Anti-Skid System electrical wiring harness to the Wheel Spin Transducers cross-wired in the axles. Outboard (O/B) harness routed to Inboard (I/B) and I/B to O/B. He was performing a Receiving Inspection of the A320 MLG from a Contract Repair Station Overhaul Vendor.

ACN: 1003987 (2216 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Landing
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Reverser Lockout
Manufacturer : Aerospatiale
Aircraft Reference : X

Component : 2

Aircraft Component : Fan Reverser
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Hydraulic Syst Valve
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1003987
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1004620
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 3

Reference : 3
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1003988
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Primary Problem : Human Factors

Narrative: 1

[I, Mechanic X, was] assigned Job Card XXX-010 on an A320 aircraft in our Hangar [in ZZZ]. [I] accomplished Step-1 of Job Card on a V2500 engine. In Part Five of Step-1, [Job Card states] make Thrust Reverser (T/R) unserviceable for Maintenance (Deactivate). Not being familiar with the V2500 Engine Thrust Reverser System, we asked a fellow Mechanic how to deactivate the T/R on the V2500. We used the HYDRAULIC CONTROL UNIT (HCU) Deactivation pin. After pinning HCU, verified [reverser] deactivation per Aircraft Maintenance Manual (AMM) 71-13-00. Thrust Reversers did not deploy upon landing. Step-4 on Job Card [is a problem]. Suggest separate sign-offs and graphics for Deactivation and Activation in Heavy Maintenance Visit (HMV). Manager notified.

Narrative: 2

While accomplishing Job Card XXX-004, the proper Thrust Reverser (T/R) lockout pins [with streamers] were not installed due to the fact that there are none in ZZZ. I, [Mechanic Y], was tasked with closing the T/Rs, closing the pylon panels and closing cowlings. Due to the fact that improper pins were installed, there were no streamers [attached to the pins]. The removal of the [Reverser] pins was inadvertently missed and there was no Operational Check of the T/R after Job Card completion. The A320 aircraft left and landed at its next destination where the Thrust Reversers did not deploy on landing.

I did not see lockout pins and closed cowlings. Event was identified in the arrival station. I have no information on what happened at that station. Suggest using proper pins with long streamers, change paperwork to have second Mechanic verify pins removed because there is no indication of T/Rs deactivated. Also add to paperwork, Thrust Reverser Operational Check per Aircraft Maintenance Manual (AMM) 78-31-00.

Narrative: 3

I, [Mechanic Z], was assigned Job Card XXX-004 on an A320 aircraft. [I] accomplished Step-1 of Job Card; on Part Five of Step-1, [Job Card states] make Thrust Reversers (T/R) unserviceable for maintenance [deactivate]. This maintenance was accomplished on a V2500 engine. Not being familiar with the V2500 motor T/R System, we used the de-activation pin located in the Hydraulic Control Unit (HCU).

Synopsis

Three Mechanics, unfamiliar with the V2500 engine on A320 aircraft, report about their involvement with installing improper Thrust Reverser lockout pins in the Hydraulic Control Units (HCU). Reversers did not deploy on landing after aircraft was released from Heavy Maintenance Visit (HMV).

ACN: 1003890 (2217 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1003890

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff, set thrust to flex, and below 80 KTS Engine Number 2 failed. Performed rejected takeoff procedure [and] returned to gate. Attending Mechanic said turbine failed.

Synopsis

A321 Captain reported losing Number 2 Engine on takeoff roll. Takeoff was rejected and the flight returned to the gate, where a Mechanic reported turbine failure.

ACN: 1003777 (2218 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Pneumatic Control Valves
Aircraft Reference : X

Component : 2

Aircraft Component : Engine Air Anti-Ice
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 1003777
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I troubleshot a #2 engine anti-ice valve discrepancy on an A320 aircraft. I reactivated the valve for troubleshooting and determined that there was good voltage to the valve. I manually actuated the valve and noticed a sticking in the 'Closed' position and determined the valve was bad. We did not have a valve in stock so I was told to defer the discrepancy. I mistakenly deactivated the valve according to MEL 30-27-01A instead of MEL 30-27-01D. 

At this point in time I do not know how I made this error. My only thought is that due to poor lighting in the cockpit I mistook the 'D' for an 'A'. Aircrew noticed improper anti-ice valve indication on engine start. I did not thoroughly review which MEL had been applied to the aircraft. Proper MEL was applied [after a] gate turnback. Suggest [I] thoroughly review and make sure I understand which MEL has been applied to the aircraft.

Synopsis

A Line Mechanic reports he mistakenly deactivated #2 engine anti-ice valve according to MEL 30-27-01A, instead of MEL 30-27-01A. Flight crew returned the A320 aircraft to gate after noticing improper anti-ice valve indication during engine start.

ACN: 1003503 (2219 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 5700
ASRS Report Number.Accession Number : 1003503

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During taxi out for departure all systems and checks were normal and the Tower cleared us for takeoff. As we took the runway, I completed a positive transfer of aircraft control to the co-pilot. Takeoff power was selected and at about 70 KIAS there was a Reverser Unlocked ECAM that both pilots observed. At about 80 KIAS, I took positive control of the aircraft back from the co-pilot and initiated an abort. We brought the aircraft to a safe stop and exited the runway. There were no reported problems from the cabin crew or passengers. After contacting Dispatch and Maintenance Control we taxied the aircraft back to the gate without further problems.

Synopsis

An A320 flight crew initiated a low speed rejected takeoff upon receipt of a Reverser Unlocked ECAM at about 70 KIAS.

ACN: 1003319 (2220 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Fuel Tank
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Fuel Line, Fittings, & Connectors
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Technician : 23
ASRS Report Number.Accession Number : 1003319
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Primary Problem : Human Factors

Narrative: 1

At approximately XA:30pm, I went to the Hangar, an A320 aircraft, to make sure that the Fuel Tank Venting equipment was setup and operating. When I got there the blower for the right-hand side Main Fuel Tank was not operating and the blower on the left-hand main was operating at a low level. I made sure that both sides were venting and informed the Shift Supervisor in the Hangar that they had been unhooked. 

After approximately 20-minutes, I returned to start sumping the main tanks with other members of the fuel crew to find the right hand blower was unplugged from the air source and not operating. I asked several mechanics on the Hangar floor if they had knowledge of why the blowers air source had been unplugged and they did not. At that time, I told the Supervisor to please brief his Maintenance crew and inform them that the fuel crew needs to purge the tanks for entry. I again made sure that the blowers were operating properly and we left until after break. After break we returned again and now the blower for the left main tank was not operating. This is an unsafe practice and can not be tolerated.

Callback: 1

Reporter stated the air blowers that are used for venting the wing fuel tanks are old and noisy. When the air blowers are in use along with additional air supply hoses attached to venturi type horns positioned in the fuel tanks to increase air movement through the wing fuel tanks during fuel tank maintenance work, the hangar environment is very loud and noisy. The blowers and venturi horns are left running to purge the wing tanks even though the fuel crew is not actually in the tanks. 

Reporter stated Hangar floor mechanics working on the same airplane get frustrated with the continuous loud noises and when they don't see anyone working in the fuel tanks, they have been known to disconnect the air blowers and even the air supplied to the venturi horns to have a quieter work environment. But that is an unsafe practice, especially if someone is still in a tank and also requires the air blowers to run even longer until the fuel tanks are fully vented. His Air Carrier is slowly buying new and quieter air blowers that should make a big difference in the Hangar noise level in the future.

Synopsis

A Fuel Tank Technician reports about finding their left and right fuel tank, wing venting air blowers had been unplugged from an A320 aircraft in their company Hangar, creating an unsafe work environment.

ACN: 1003063 (2221 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : MSP.Tower
State Reference : MN
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 1003063

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 1003068

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

After pushback from gate with the brakes set and starting engines I noticed a tug pushing another air carrier aircraft from a nearby terminal. As the other jet continued the push, its right wing WENT UNDER our left wing by four to five feet. We contacted Operations and reported it and they informed us they witnessed it from their office and would contact the other company. It is a very tight ramp area and I was surprised that Ground Control would OK a push with us parked and starting engines from those gates.

Narrative: 2

[No substantive additional information was provided in the secondary narrative.]

Synopsis

An air carrier flight crew reported another aircraft pushed back from an adjacent gate after they were already pushed back, with their brakes set and were starting engines. The other aircraft's right wing passed well under their left wing while doing so.

ACN: 1002832 (2222 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4500

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1002832
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On approach, I called for flaps 2 and we then received an ECAM message FLAPS LOCKED - DUE TO ALIGNMENT. The flaps stayed at almost 2 and the leading edge devices operated normally. We discontinued the approach, declared an emergency, and asked for vectors to deal with the ECAM. We ran the ECAM (which had no actions), performed the status page items and then followed up with the QRH. This all took less than five minutes and we were vectored back around for a normal landing. The landing was uneventful and we taxied to the gate. Everything was normal except the flaps stayed at the position they failed (locked) at when we selected flaps 0 after landing.

Synopsis

During approach, an A320 ECAM alerted FLAPS LOCKED - DUE TO ALIGNMENT, so an emergency was declared, the QRH procedure completed, and a normal landing ensused.

ACN: 1002434 (2223 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb

Component : 1

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1002434
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1002433
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Shortly after takeoff we experienced and ECAM master caution for an ENG 1 BLEED AIR FAULT. The Captain (pilot not flying) performed the ECAM actions and applicable OEB from the QRH and cleared the caution. Immediately after, another ECAM master caution appeared for a HYD YELLOW SYS RSV OVERHEAT. The Captain again performed the ECAM actions as directed by the flying pilot and cleared the caution. The aircraft status resulting from both ECAMS showed several items inoperative for the flight, including some spoilers, yaw damper, nose wheel steering, and engine bleed 1. Considering the inoperative systems, flight duration remaining, and possible further degradation of the aircraft systems, the Captain contacted Dispatch and Maintenance to discuss a possible air return. After discussing the aircraft status via air cell, the Captain, with cockpit crew agreement, made the decision to return and land overweight with concurrence from Dispatch and Maintenance. The flight crew notified ATC of their intentions to return and declared an emergency as a precaution due to our degraded status. The in-flight crew was briefed and the passengers were informed of our return. Flight was cleared to a holding point and began preparing for an overweight landing (157,000 LBS) on with a tow off by ground personnel. The First Officer (pilot flying) continued flying the aircraft and talking to ATC while the Captain reviewed landing performance data with the inoperative spoilers and overweight procedures in the QRH. With mutual agreement on sufficient landing distance by the First Officer and Dispatch, flight began approach preparations for a Captain flown ILS. The prevailing winds were from the south at 190/23 gust 33 with approaches being conducted to crossing runways. All applicable checklists were performed and the aircraft was cleared for approach and landed without incident. The aircraft came to a stop on the Runway and was met by a company tug crew and emergency equipment. The aircraft was towed clear of the runway and into the gate. Upon securing the aircraft, logbook entries were made for the engine bleed, HYD RSV overheat, and overweight landing. The aircraft was deplaned and passengers/crew reassigned to a standby aircraft.

Narrative: 2

[Narrative #2 contained no additional information]

Synopsis

Immediately after takeoff an A320 ECAM alerted ENG 1 BLEED AIR FAULT, then YELLOW SYS RSV OVERHEAT so an emergency was declared and the aircraft returned to the departure airport.

ACN: 1002409 (2224 of 2908)

Time / Day

Date : 201204
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZFW.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZFW

Component

Aircraft Component : ACARS
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 450
Experience.Flight Crew.Type : 4600
ASRS Report Number.Accession Number : 1002409
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

This incident started while in cruise at FL360 southeast of DFW. We checked in to Fort Worth Center and were direct to LBL. We first noticed that the ACARS had stopped functioning. No ATSU menu information was available, printer function stopped etc. What little troubleshooting we could do didn't help. We waited a little while to see if it would begin working again and after about ten minutes selected the appropriate Commercial Radio frequency on the number 2 radio to monitor (131.65). Another carrier was using the frequency to deal with a lengthy medical emergency. Unknown to us at this time ATC had failed to pass us off to the next sector frequency. Several distractions from the cabin occurred; a bathroom break, etc. We passed into the next Commercial Radio sector and switched to 131.30 which was quiet. Up to this point radio chatter on ATC was always present, and we had no real reason to suspect we had missed any radio calls. Approaching LBL it was apparent something was amiss and several calls to Fort Worth by us weren't answered. Another aircraft relayed a frequency to us and we contacted Kansas City on the correct frequency over LBL. Within a minute Dispatch was heard on Commercial Radio calling us. They requested we Confirm OPS Normal gave us the appropriate sector frequency and asked about ACARS messages that weren't answered. We told them the ACARS was inoperative and that we had reestablished ATC communications. The ACARS was reported inoperative to Maintenance upon landing. I called Dispatch and Fort Worth ATC after landing. They informed me we had flown through multiple sectors 'NORDO' over a period of about 25-28 minutes. Fort Worth said they had failed to properly hand us off but wanted to know why we couldn't be reached by company, etc. Other than listening better to subtle indications on the radio that we had flown past the hand off, the time elapsed and realizing over DFW sectors would be changing quickly I don't see much else we could have done differently. Dispatch tried to send us ACARS messages, but none got through. I didn't see an immediate need to call Dispatch via Commercial Radio to tell them our ACARS was inoperative, especially when the frequency was being used for an emergency. Sector frequency changes can occur in a little as a minute or more than 20 minutes. Sector lines and frequencies are not indicated on HI Charts as is the case on Latin American charts. Perhaps a sector change indicator could be incorporated into the NAV data base somehow. We have become very reliant on ACARS, yet will quickly try to dispatch aircraft with it inoperative. The requirement to monitor Commercial Radio with an inoperative ACARS is unknown to some and I could only find one small reference to it in the FOM. Monitoring guard is recommended, not required by the FOM.

Synopsis

A320 Captain reports an ACARS failure en route at FL360. At about the same time the aircraft flies out of the current ZFW sector without a frequency change. Over twenty minutes pass before the crew detects the lost comm situation and contacts Center.

ACN: 1001517 (2225 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 1000

Environment

Weather Elements / Visibility : Windshear

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : SFO

Component

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 1001517
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On takeoff roll I set the power levers in the flex detent, confirmed the position and raised my hand off the levers. In placing his hand on the levers the Captain managed to advance the levers to TO/GA. I was not aware of this. Rotation was normal; I began the immediate left turn using the flight director to follow the PORTE departure off runway 19R. At the annunciation for power reduction I moved the levers aft one click and advanced them one click thinking I had selected MCT. In actuality I had selected TO/GA. The flight directors then indicated a steep climb and a turn back to runway heading which would have taken us into terrain. While in IMC trying to understand the indications, we hit a 30+ knot increase shear. The speed tape blurred and jumped to 250. I called for flap retraction and managed speed. Because we were in TOGA managed speed could not be selected. I called for speed intervene 250. I maintained a left turn to continue the PORTE departure ignoring the command bars right turn. Within seconds the sheer was over and the speed tape jumped back to 220 KTS. There was never a sheer warning. Tower gave us a heading and directions to proceed to a fix further along the departure. Tower then asked what departure we were flying. The Captain stated the PORTE and that we encountered turbulence and sheer and therefore had trouble navigating. Tower stated "No problem". Obviously I was not precisely on track because the aircraft was in the go around mode and indicating direction to fly a programmed climb and track.

Synopsis

A320 First Officer reports inadvertent selection of TOGA thrust during a planned flex thrust takeoff and the problems encountered at thrust reduction/clean up altitude. Thrust levers are retarded one detent to get into the auto flight climb mode then advanced one detent to maintain MCT (actually TOGA), putting auto flight into go around mode. A flap overspeed occurs due to windshear and the high thrust setting.

ACN: 1001400 (2226 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Nose Gear Door
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 1001400
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Incorrect / Not Installed / Unavailable Part

Narrative: 1

Nose Landing Gear (NLG) AFT Door. AFT Hinge Repair Bushings Part Number (P/N) XXX-1. These flanged bushings are part of a repair for worn hinges (Oversized). The problem is this P/N XXX-1, I believe, are not Manufactured (MFG'd) correctly, compared to the others we have had in the past. These bushings are approximately half the length of the others. If you look at Page 4 of 4, of the Airbus Supplement to the Aircraft Maintenance Manual (AMM) 52-003, it does not show what the length should be for the Flanged bushing, but the non-flanged bushing shows a typical length of .155" to .160" of an inch. In my opinion the flanged bushing should be .155" to .160" of an inch, plus the flanged dimension of .078" to .080" of an inch. Most of the bushings are found in [Stores] Kit XXX [Parts Bin] for the Maintenance Base, since this is a common repair at every [C-Check] visit.

Callback: 1

Reporter stated the Flanged/Hat bushings were driven by a Service Bulletin from Airbus and made in their Air Carrier's Machine Shop. But the Airbus Maintenance Supplement to their Aircraft Maintenance Manual (AMM) never gave any dimension length. So the drawings from company Engineering to the Machine Shop never included the Flange/Hat thickness dimension as an additional length to the original Aft Door hinge fitting bushings. The Machine Shop had assumed the 'Hat' thickness was inclusive of the total bushing length, due to the Engineering drawing did not specify otherwise. There are two bushings involved; one bushing floats and one is the 'Flange/Hat' bushing.

Reporter stated the Airbus Service Bulletin was issued in response to pilot reports about high noise vibration in the aircraft's nose section during Takeoff rotation. The Flange/Hat bushings were designed to protect the Aft Door rod assemblies from being crushed during re-assembly. The bushings were only part of the Service Bulletin to address the nose vibrations.

Synopsis

A Technician reports the Nose Landing Gear (NLG) Aft Door, Aft Hinge Repair Bushings found in their Stores Kits are approximately half the length of the replacement bushings they have been using in the past on their A320 aircraft during Maintenance Visits. The bushings were part of an Airbus Service Bulletin to address nose vibrations on takeoff rotations.

ACN: 1001328 (2227 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Fluid
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1001328

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.General : Police / Security Involved
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Passing through FL330, we received an ECAM message that said, "GREEN HYDRAULIC SYSTEM RESERVE LOW." I was the flying pilot and assumed control of the aircraft and radios and assigned the First Officer to consult the QRH for immediate action and exception items there were none. I then asked for "ECAM ACTIONS". The procedure was followed which called for the isolation and shutting down of the Green Hydraulic System. After completion of the initial ECAM procedure it was clear that it led to many more ECAM items and follow up items that must be accomplished. It was truly a very busy day in the cockpit. At this point I notified ATC of our situation and declared an emergency and to standby for more information of our plan on diversion or return. I also turned to our FAA Line Inspector, who was on our jump seat observing and told him that he was no longer evaluating us but was now a crew member and any and all input from him was appreciated. The flight attendants were notified using T.E.S.T. as a communication guide and the passengers were also notified from the cockpit. The three of us returned to the business of managing all the follow up items associated with the Green Hydraulic System now inoperative. We communicated with Maintenance Control and Dispatch our situation. We advised them that we were holding at FL330 present position and working through the procedures. We elected to return to departure airport. We planned on holding at a lower altitude with flaps 2 to burn down below max landing weight. Dispatch did give us the go ahead to land overweight but we thought having only one reverser, no autobrakes, slow flaps, only some spoilers, no nose wheel steering, manually lowering the landing gear, and an FAA guy helping us in the jump seat, that having lots of time, we elected to burn to less than 142 KTS for landing. At this point we received a call from the flight attendants that there was a passenger disturbance in back. An individual claiming to be a reporter would not follow crew member instruction during a declared emergency. This man was attempting to film and interview crew and passengers as the event unfolded and ignored all requests from crew and fellow passengers to sit down and behave. We finally assigned two armed police officers who were traveling with us to baby sit him while the adults handled the in flight emergency. After holding for more than an hour, our weight was below 142 KTS and we made an approach and landing. The landing gear was lowered manually with no event; we had briefed all the items that would pop up when we did this and planned for the extra time. The landing was uneventful and we stopped at the far end of the runway remaining on the runway until we were pulled to the gate. There was much rejoicing and lots of police to meet our, non compliant, passenger.

Synopsis

A320 crew experienced a GREEN HYD SYS RES LOW passing FL330 and elects to return to departure airport after declaring an emergency and burning down to landing weight.

ACN: 1001325 (2228 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Cabin Address System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1001325
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Events

Anomaly.No Specific Anomaly Occurred : All Types
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight had a scheduled departure. I arrived at the airport, checked in and preceded to the aircraft ahead of schedule. The airplane was cold with everything shut down. I proceeded to do my check and noticed the maintenance logbook with a blue sticker on the cover with MEL 23-31, indicating that the aircraft PA system was inoperative. I read the MEL, to see what procedures were required. I then followed up with reading the FOM. Though the MEL allowed for departure, the FOM required its use during Latin America operations. The FOM has 'highlighted' specific language I am required to say over the PA when departing a non-US airport to the United States. I am to read the italicized statement, "verbatim", to the passengers. I called the Dispatcher to discuss whether the aircraft was legal. The Dispatcher came to the same conclusion as I and would coordinate with Maintenance to get a different aircraft. I had provided my cell phone number to the Dispatcher, so he could call me directly when he had another aircraft for us. Other than now realizing we may be running late, I felt comfortable knowing that everyone that was involved with our flight knew of the situation, the aircraft swap and that we were now only awaiting a call from Dispatch with the new aircraft number and gate. I chose to use this time to brief the Flight Attendant's in order to help expedite our now late departure. My phone rang, thinking it was Dispatch, I answered only to find out it was an Assistant Chief Pilot. The Assistant Chief Pilot seemed to know what was going on in respect to our flight, the aircraft, the aircraft MEL and the procedure for MEL 23-31. He made the comment that the aircraft was legal, but if I did not think it was safe he was okay with my decision. I then explained to the Assistant Chief Pilot what was in the FOM and he then fully understood the problem we had with the aircraft with that particular MEL. After hanging up, I resumed my briefing with the Flight Attendants. Again my phone rang, and again it was the Assistant Chief Pilot. This time, rather than question the decision to switch airplanes, he began explaining that now we would have many misconnects if I didn't hurry the departure, do a quick turn and hurry back! I wasn't sure how to respond to this request, since it sure sounded as if he wanted me to violate the FOM standard operating procedures, other than to say, we would do our best.

Unfortunately, the Assistant Chief Pilot's politeness appeared to only disguise the message I heard, the first time he called, which was that I was making the wrong decision about the safety of the aircraft. Since the decision to switch aircraft was a joint decision between the Dispatcher, myself and possibly Maintenance Control, I felt that the Assistant Chief Pilot's objective for calling me (without knowing all the facts), was to intimidate me and harassing in nature. Additionally, with my, 28 years of service for this airline, I feel the Assistant Chief Pilot's first call to me was not typical or normal; his second call to me crossed the line in every professional and ethical way. If what his true objective was to help me, he failed. He did however; succeed in pushing me into the "Yellow" even before we left the ground. I do not know how it was that the Assistant Chief Pilot came to know that we had a delay and why, since he never said. He also did not start the conversation asking what the problem was, rather presumably knowing everything about our flight (which is how he initiated the conversation), it was more condescending.

Synopsis

A320 Captain explains the reasoning behind his rejection of an aircraft for a flight to a Caribbean Island with a non-functional PA system.

ACN: 1001319 (2229 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0001-0600

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Component

Aircraft Component : Electronic Library (other than Nav database)
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 1001319
Human Factors : Human-Machine Interface
Human Factors : Other / Unknown

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

I must report that in my opinion the iPAD is not ready for prime time. As a very experienced computer user (and iPAD owner from day one), I feel I have a pretty good grasp of the intended concept and can clearly use an iPAD to its fullest extent. Within minutes of testing the company iPAD I quickly determined the SEARCH feature to be useless and limiting when searching for key elements such as "stabilized approach". This only sent me to a chapter reference not to the page I needed to review. Searching is one of the most important features of any electronic document reader and when it is limited in capability as the current feature is, it becomes useless. In a time crunch one might find during an irregular procedure, this thing is useless IMO. The "bookmark" feature does not allow any title to be assigned and therefore more than one bookmark simply appears as an unidentifiable mini icon that cannot be discerned from other bookmarks (icons) that may have been archived. You therefore loose the purpose of quickly referencing a bookmark for quick resource. I also lost my position several times, as the iPAD software would randomly go to some other chapter when I touched the left arrow on the screen designed to hide the menu and open a full page view. I can reproduce this anytime for those who doubt me. It appears to me that the Company has again chosen the CHEAPEST proprietary version of some textual software that only mimics the capability of Adobe.pdf presentations. Simply putting the downloadable company Adobe files (FOM or FM) on my PERSONAL iPAD provides a quicker, better and SAFER reference than the pathetic software used on the Company iPAD application. I would not even hand these things out until they get a little more serious about the use of this product in flight. As far as I am concerned it sits on the shelf at home until they want it back or fix the issues. There is also no functional training on how to use the iPAD document reader. I can only imagine someone trying to find the support information in flight and then losing the page or not knowing how to restore the page while distracted.

Synopsis

A320 Captain laments the search ability of his Company issued iPAD flight manual.

ACN: 1001134 (2230 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Oil Contents Indication
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 1001134
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Training / Qualification
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departing Engine # 1 thrust reverser inoperative and on MEL. Captain is flying pilot. On the takeoff roll, after making the 80 KTS call and looking back to the engine instruments I noticed the oil quantity was at 7 QTS, I called the abnormal oil level out to the Captain. Immediately after bringing this to the Captain's attention the oil quantity was now showing 2.5 QTS and pulsing. I said to the Captain "oil is now at 2.5 QTS." The Captain then chose to reject the takeoff at approximately 125 KTS, V1 was 135 KTS. The aircraft came to a full stop, on the center line of Runway 36. I made the call to the Tower and notified them of the reject while the aircraft was coming to a stop. After the aircraft came to a full stop, the Captain made his PA announcement to the cabin. We assessed the situation, turned on the brake fans and taxied clear of the runway and into the ramp. While waiting for our gate and communicating with maintenance and the company the oil level was now showing normal. We returned to the gate and wrote the malfunction up in the FDML. The brake temps rose to approximately 760 degrees C and then cooled with the brake fans on. Maintenance then informed us that this aircraft had a history of this oil quantity indicator problem and the flight was canceled. After seeing the oil quantity drop rapidly to 2.5 QTS and pulsing, we believed that we had lost the oil from the number 1 engine and that an engine failure was imminent. We were below V1, so the decision was made to reject the take off. A thorough knowledge and understanding of the reasons to reject a take off above 80 KTS would have led to a different decision about rejecting at a high speed. The previous statement is a reference to myself.

Synopsis

An A320-200 Captain rejected a takeoff at 125 KTS after the oil quantity decreased to 2.5 QTS during the takeoff roll and after arrival at the gate the crew was told the aircraft history indicated a quantity indicating problem so the flight was canceled.

ACN: 1000476 (2231 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 33000

Aircraft

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZLC

Person

Reference : 1
Location Of Person.Facility : ZLC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 1000476
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 was enroute on course to KU45Q when I, the ZLC R46 Controller took the hand off from ZLA R33. The A320 made an unexpected turn right of course. I called ZDV R23 to make a point out prior to the aircraft violating ZDV's airspace. ZDV Sector 23 Controller called RADAR Contact. I then asked the A320 what fix he was navigating to when I think the pilot responded with KU45S (KU54S was the next fix in the Airbus' flight plan). I called ZDV Sector 23 and coordinated that the Airbus was direct KU54S. After talking to several controllers I discovered this has happened several times before. The two fixes KU45S and KU54S are only 100 miles apart, but are too easily mis-entered and misheard on a read back/coordination call. When KU54S is the fix in the NAS flight plan after KU45Q, and the aircraft has KU45S entered into autopilot, the flight plan will not auto flash to ZDV Sector 23 and the controller only has 60 seconds to realize that the aircraft is off course (and that the issue is not related to "vector wobble" ERAM discrepancies) before violating ZDV's airspace. The "K" fix naming convention needs to be changed. It is too dangerous having two fixes with such a similar naming convention in such close proximity.

Synopsis

ZLC Controller described an unexpected course change by an air carrier, the reporter expressing concern regarding the similar "K" fix naming convention as problematic.

ACN: 1000318 (2232 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 180
ASRS Report Number.Accession Number : 1000318
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Person : Flight Crew
Detector.Person : Maintenance
When Detected : Pre-flight
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Primary Problem : Human Factors

Narrative: 1

Upon arriving at aircraft I noticed the First Officer's PFD (PRIMARY FLIGHT DISPLAY) was too bright for night-flying and, since the captain was not yet present, in the interest of preventing further delay, I sent an MRM code on ACARS and called Ramp Maintenance to let them know. After my walk-around, a Mechanic was in the cockpit who told me it was the same brightness as the Captain's PFD. I told him it wasn't and that it was too bright and I needed it changed. He said he'd see if they had one. When the Captain came in the cockpit I told him that I wrote up the PFD and they said they'd see if they had one. In the meantime, my overhead map light quit working and with the Captain's concurrence I sent a maintenance release and told Ramp Maintenance verbally that we needed that light fixed as well. They told us they would defer it to prevent a delay. I told the Captain that I relied very much on that light and we both knew they could change the bulb in 5 minutes or less. The Captain told them we wanted it changed and Maintenance replied that "it is deferrable." The Captain told him that yes, while it's deferrable, it must be adequate for the pilots to accept the deferral. They told us the Captain would have to refuse the airplane. While this conversation was occurring, we got the maintenance release over the ACARS and saw that the PFD was signed off, not even deferred. They also deferred the map light saying they had no time. So, the Captain stepped out of the cockpit and called the Dispatcher to refuse the airplane. Then when he came back he told me they were going to fix the map light but we would take the PFD as is, because they didn't have one to replace the (too bright) one in this airplane. I told the Captain I wasn't comfortable with that and he essentially told me I could walk off the airplane if that wasn't alright with me, because I was "putting him in a box" with my insistence that they fix the PFD. I was shocked that the Captain wouldn't back me up on this and that we would take the airplane with a maintenance discrepancy write-up that the Mechanic ignored and signed off. They didn't even defer it, which I would have been OK with. But since they just signed it off and ignored my "safety of flight" issue, I was not going to accept it because I thought it was illegal as well as wrong. The Captain pressured me to take it as is, trying to convince me that it wasn't that bright, even having me turn the PFD full bright and then full dim to "convince" me it wasn't that bright. The Captain then accused me of being "a big guy and you got your way" which appears he was mocking me because I didn't back down on my principles and professionalism. After some heated words of disagreement, the Captain seemed visibly upset, told me I could get off the airplane and that's when I no longer felt comfortable in the cockpit with him and the maintenance situation, and I chose to leave. After communicating with the Crew Desk and the Flight Operations Duty Manager (FODM), (approximately 30 minutes later) I discovered that maintenance somehow found another PFD they would install and they also would fix the map light. I told the FODM that due to the hostile cockpit environment I just left; I did not feel the comfortable, at all, completing that flight with that Captain.

Synopsis

An A320 First Officer removed himself from a trip after a disagreement with Maintenance and the Captain over the aircraft's maintenance status made working together unsafe.

ACN: 999258 (2233 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : ZZZ
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24607
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 9700
ASRS Report Number.Accession Number : 999258
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Preflight and takeoff were all normal. Since we were heavy, about 160,000 LBS. on a short runway, we needed TOGA power and I kept the flaps out a little longer to ensure we could meet the altitude restrictions in the SID. After a couple more minutes we received an ECAM message HYD Pump Low Pressure Green. I told the First Officer to take care of the ECAM; I had the aircraft and radios and put on autopilot 1. The First Officer turned off the Green Pump and it appeared that the PTU was taking care of Green Pressure. After a bit of discussion, we sent Dispatch a message to call us. Dispatcher called and talked with the First Officer. We went into a holding pattern at FL190. At this point we started getting some occasional ECAM's and it appeared as though the PTU was not keeping the Green System above 2,700 PSI. The Yellow System was also indicating an overheat. We determined that the PTU was cavitating because the Green quantity was gone. This was confirmed when a PTU low pressure ECAM illuminated and we turned off the PTU. The Yellow System overheat went away and the hydraulic system seemed stable. The First Officer had been trying to talk with Maintenance but never could get a clear frequency, and ended up just dropping the phone patch. Dispatch agreed that we should return. We declared an emergency called Flight Attendants, made PAs, and the First Officer started on the ECAM. We felt like we were not in a hurry at this point and it would help to go slow and burn off some gas. We worked our way down to 10,000 FT in the holding pattern while the First Officer ran through the ECAM. We told Company and ATC we would need to stop straight ahead as we had no nose wheel steering. We completed the ECAM. We made one PA to the passengers, reviewed all the failed systems and told Approach we were ready. Our weight was 154,000. 

The First Officer cranked the gear down [Landing Gear Gravity Extension] with the checklist and we slowed to 210 about 10 miles out. We started putting flaps out early and were fully configured prior to the marker. All checklists were completed and we landed with a sink rate of 300-400 FPM. Landing was uneventful. We stopped the aircraft just short of Runway 09 and Tower told us to make a right. We reminded them that we had no nose wheel steering and could not turn the aircraft. We started the APU and shutdown the engines. A tug did come out to tow us to the Terminal, but the tug driver thought the gear doors might scrap and we asked a mechanic to come out and check. After a while the mechanic made it out to the aircraft and felt it was OK to tow in and we went slowly to the gate. There were no human factors. Dispatch, the First Officer, and all the ATC members were helpful and supportive.

Synopsis

A320 Captain experiences an ECAM message HYD Pump Low Pressure Green. The pump is turned off and initially the PTU appears to be maintaining Green system pressure. While holding at FL190 a PTU low pressure ECAM is annunciated along with a Yellow system overheat. The PTU is turned of and the crew elects to return to the departure airport after the Yellow system cools.

ACN: 998791 (2234 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 12500

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : GEELA5
Airspace.Class E : ZAB

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 1425
ASRS Report Number.Accession Number : 998791
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 15500
Experience.Flight Crew.Last 90 Days : 21
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 998813
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were cleared to descend via the GEELA5 arrival and had verified the constraints. We were on speed and thought on altitude when ATC queried us as we were descending out of 12,200 FT outside of PUNNT (which had a constraint of 12,000-9,000 FT). ATC informed us that the GEELA arrival had a NOTAM for PUNNT at 10,000 FT. We were given a heading of 040 to fly. No further problems were encountered. Upon debriefing with the First Officer, we verified that we did have all the papers and that the NOTAMs were reviewed. Factors that most likely contributed to this NOTAM being missed were the short turn time with an airplane change, an excessive number of NOTAMs for PHX which gave us several pages of them, the lack of order to the NOTAMs, and the fact that this NOTAM printed with two others separated by space on our papers. We did find it, but we had to search.

Narrative: 2

We were cleared to PHX on the GEELA FIVE RNAV ARRIVAL 7R, and received a clearance to descend via the arrival. We were doing a managed descent and set 5,000 FT in the ALT window. Approximately one mile from PUNNT descending through 12,200 FT at 250 KTS ATC asked our speed and altitude. ATC then informed us that the restriction for PUNNT was 230 KTS at 10,000 FT. ATC vectored us off the arrival and gave us a 040 heading to intercept the Runway 08 localizer. Subsequent communications with ATC revealed that a NOTAM had been issued for the GEELA FIVE ARRIVAL changing the crossing restriction. The flight continued to landing without incident. The Captain and I did the preflight planning for the PHX flight while airborne on the previous leg as we were running about 25 minutes late and knew we would have less than thirty minutes to change planes and depart for Phoenix. The flight papers printed contained an "INCOMPLETE MESSAGE" and large blocks of largely unformatted text. Prior to descent into PHX the Captain and I checked to make sure we had the current Commercial Chart page and we each checked the crossing restrictions in the FMGC to make sure they matched the charted restrictions. The sheer number of NOTAMs combined with the difficulty in reading them as printed by the aircraft ACARS printer contributed to us missing the change to our arrival procedure.

Synopsis

A320 flight crew reports missing a NOTAM changing the crossing restriction at PUNNT on the GEELA 5 RNAV to PHX. ATC advises and issues a vector. The NOTAM in question was buried in a very large list.

ACN: 997408 (2235 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FADEC / TCC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 997408
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Aircraft

Narrative: 1

During cruise at FL350 we received an ECAM: Engine 2-FADEC B Fault, Sensor Fault, avoid rapid Thrust Changes. I contacted Dispatch and Maintenance and after a conference, decided to return to our departure airport for the following reasons: If we lost the other channel, the next step required that we land at the nearest suitable airport. Another concern was the potential loss of the left engine, which would leave us with only a degraded number 2 engine with the thrust change caution. On a single engine, we would very possibly require rapid thrust changes during certain phases of flight (approach, go-around). Continuing to our destination would have found us flying over higher, more mountainous terrain with fewer suitable alternate airports. I felt the safest option was to return to our departure airport with great VFR weather, and light winds.

After an uneventful landing, a Maintenance Technician informed me that neither A or B FADEC channel was testing properly for engine 2, and both channels were testing normally for engine 1. He also informed me the FADEC had a history and had been worked on the previous evening. Our Flight Papers had no record of this maintenance in our Log History, despite the importance of the critical engine component. An additional concern is that our Flight Manual does not include the caution to Avoid Rapid Thrust Changes. In the event of an incomplete ECAM message it would be important for the flight crew to be aware of this operational precaution.

Synopsis

An A320 Captain elected to return to his departure airport following receipt of an ENG 2 FADEC Sensor Fault ECAM message the resolution for which included the caution to "avoid rapid THR (thrust) changes." He then enumerated his reasons for opting to return to his VFR departure airport.

ACN: 997232 (2236 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ramp : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 997232
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
When Detected : Taxi

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Received final weights then received message saying "final weights not valid; new weights to follow." We received updated weights and input the new numbers as per SOP. We were informed the next day that the [passenger] count may have been incorrect. [We] flew flight without incident.

Synopsis

Prior to takeoff an A320 flight crew received a second final weight check after a notification their first manifest was in error.

ACN: 997228 (2237 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 997228
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Work Refused
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

I was on my 8th day in a row away from home; two 4-day trips back-to-back, each with 30+ hour layovers that arrived late on day 1 and departed early on day 3, which puts my body clock out of whack. Predawn hotel pickup; the Captain and I both missed the correct altimeter setting on the before start checklist and the revised route on the PDC, even though I picked up the PDC and read it, which is my standard technique. We discovered the altimeter error on taxi-out and the PDC revision on departure, when Departure cleared us to a fix that was not programmed in the FMGC. We both removed ourselves from the remainder of the trip due to fatigue.

Synopsis

An A320 First Officer reported that he and the Captain removed themselves from a trip fatigued because the four day trip upset their circadian rhythm and they were making too many errors.

ACN: 996915 (2238 of 2908)

Time / Day

Date : 201203
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 996915
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 997231
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

As we taxied out we received a final weight manifest. The weight and balance was adjusted in the FMS to reflect the changes published on the manifest. Five minutes after takeoff we received a second "final weight manifest" time stamped 26 minutes later than the first. The two manifests were 3,324 LBS different. Contacted Dispatch imputed the updated weights and continued to our destination.

Narrative: 2

The Captain sent a message to Dispatch and asked what was going on. Dispatch contacted Load Planning and sent us the following reply: LOADPLANNING SAYS CUSTOMER SERVICE CHANGED PASSENGER COUNT AFTER YOU WERE AIRBORNE.

Synopsis

An A320 flight crew received a second "final" weight manifest message while already airborne.

ACN: 996286 (2239 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Powerplant Mounting
Manufacturer : Airbus

Component : 2

Aircraft Component : Powerplant Installation
Manufacturer : Airbus A320
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 996286
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Procedure

Narrative: 1

Number 2 Engine change was being accomplished. This was an assigned engine change on the Work Package. As part of this process, four forward engine mount bolts were installed to attach the engine to the A320 aircraft. These four bolts (copies of the Part Tags are in the attachment) were entered into the [Records] System under Repair Orders XXXX and YYYY. There was no entry in the [company's Part Tag] remarks section, Block-9, showing what actions were taken to make these bolts serviceable. Maintenance Procedures Manual (MPM) XX-00-12, 9-A, [referencing] Block 9, Item 10, NOTE 1, states: "Record data as needed in place provided. In the space provided for REMARKS, enter an appropriate 14 CFR, FAR 43.9 entry. EXAMPLE: 'Repaired / Inspected, in accordance with Component Maintenance Manual (CMM) / Aircraft Maintenance Manual (AMM) / Job Planning Card (JPC) / Engineering Order (EO) Section and Document Number." MPM XX-00-12,9A, Block 9, Item 10, NOTE 2 states that "The term used to identify the work performed by the Vendor on the Vendor's Certification Tag is to be transposed onto Block 9 of the company's Parts Tag." MPM XY-00-10, 8.B, states "Component Job Cards produced in the Engine Shops will remain with the Serviceable Tag until installation on the next higher assembly. Upon installation, they will be routed to Aircraft Records with the company Part Tags. 

Engine Shop Job Cards that are part of a Work Package will be routed to Aircraft Records with the completed Engine Pack. These company Part Tags had no documentation with them of any kind. There was just the serviceable copy that was turned in with the Work Pack. The other company Part Tags that came from our Engine Shop had the company Part Tags and the Engine Job Cards that were referenced in the remarks section of the company Parts Tag. 

I was auditing the Aircraft Work Pack and found the parts tags with the missing documentation in the remarks section of the company Parts Tags. I feel that there is a lack of information of what the requirements are to complete paperwork based on the MPM. I feel this is true at the Mechanic and Management level. I feel this is a perfect example of complacency, lack of knowledge, and norms that are three of the "Dirty Dozen" that are alive and well in this situation. I feel this is true of the individuals that made the parts serviceable, and the individuals that installed the parts on the aircraft. 

I talked to our Flight Safety Representative and he suggested that I make the Quality Control Manager aware of the situation in an attempt to correct the situation. I notified the Quality Control Manager and he called me back. He told me there was no problem, since there was an individual that signed Block 11 [of the company's Parts Tag] with their identifying information in Block 13 that was signing that, the statement in Block 10 had been complied with.

Mechanics need to have ongoing classroom and other training about what is in the MPM, with the opportunity to ask questions about what and why. Computer Based Training (CBT) training does not allow the opportunity to ask questions, and hear the responses that will often trigger additional questions. Mechanics need to know when, and why; to question if a parts tag, procedure, job card, or Engineering Order is actually in agreement with the MPM and other documentation.

Synopsis

A Mechanic reports four forward engine mount bolts were installed to attach an engine to an A320 aircraft without any entry in the Remarks section of their company's Part Tags indicating what actions were taken to make the bolts serviceable. He was auditing the Aircraft Work Package and found the Part Tags with the missing documentation.

ACN: 995865 (2240 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Reverser Lockout
Manufacturer : International Aero Engines IAE
Aircraft Reference : X

Component : 2

Aircraft Component : Fan Reverser
Manufacturer : International Aero Engines IAE
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 995865
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

One night, February 2012, I, Mechanic X, and Mechanic Y continued to remove the Bleed Valve off of an A320 aircraft for an engine boroscope from where swing shift had left off. After the Engine Shop had finished with the boroscope, we both proceeded to reinstall the bleed valve per the Maintenance Manual Supplement 72-037. After installation, I ran the engine for a leak check. Then we towed the plane down to the gate. 

Apparently when the plane was approaching the runway in ZZZ1, the reverser would not deploy upon landing. The mechanics in ZZZ1 had found that the reverser was still locked-out. That's when I got a call from Supervisor X on my Days-off and he had told me what they had found in ZZZ1. I told Supervisor X that there was no write-up that the reverser had been locked-out per the General Maintenance Manual (GMM). So then I called up my partner, Mechanic Y, and he told me that Swing Shift [Maintenance] had left a note up in the cockpit taped to FADEC, saying the reverser had been locked-out, but [he] did not bring it to my attention at the time. He had told me this three days later, also on his Days-off.

Synopsis

A Line Mechanic reports an engine Reverser on an A320 aircraft failed to deploy upon landing after he had completed a bleed valve installation. Mechanic also noted that a lack of adequate shift turnover information and failure by the previous shift to make a write-up for the Locked-out reverser, contributed to the event.

ACN: 995570 (2241 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15690
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 7731
ASRS Report Number.Accession Number : 995570
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 7600
ASRS Report Number.Accession Number : 995583
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the flare with a crosswind, First Officer found the rudder pedals to be jammed. It took an inordinate amount of force to free them. The First Officer landed safely. I taxied to the gate uneventfully. We wrote it up for maintenance in ACARS and followed up with a call to Maintenance.

Narrative: 2

During the flare with a crosswind I found the rudder pedals to be jammed. An inordinate amount of force was required to free them. Once they were free they operated normally. I landed safely and Captain taxied to the gate uneventfully. I recall during flight whenever I put my weight on the rudder adjustment housing, it seemed to move and make a popping noise. This may be what jammed the pedals temporarily.

Synopsis

An A320 First Officer had to physically override the aircraft rudder pedals which he found jammed during the landing flare. They freed up and operated normally after great deal of force was applied.

ACN: 995513 (2242 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 995513
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Air Carrier X (A320) was level at 350 going into one airport; Air Carrier Y, also an A320, was at 360, landing at another. My trainee issued traffic to the Air Carrier Y, "11 o'clock one zero miles southbound Airbus 320 one thousand feet below you, expect lower passing traffic." Air Carrier Y read back, "roger, maintaining FL360." Traffic was issued to Air Carrier X. About 20 seconds later, Air Carrier Y said "traffic in sight, going direct to the fix descending to 120." My trainee said, negative maintain FL360, issued traffic again, and told the pilot to report level at 360. But Air Carrier Y had already descended to FL357 before going back up. It's hard to say what to recommend in this situation. I believe my trainee used the right traffic call using "one thousand feet below you" instead of "traffic at FL350." I was told that the charts show the STAR arrival as a profile descent, but I personally haven't seen the charts. Maybe the charts need to be updated if that is the case.

Synopsis

Enroute Controller described a loss of separation event that may have involved the issuance of altitude phraseology and misunderstanding of same.

ACN: 995435 (2243 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Main Gear Tire
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 995435
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Equipment / Tooling

Narrative: 1

I am filing this report due to an ongoing safety concern with regards to unsafe tooling for changing the tires on the A320/319 aircraft. The tooling in question is the poorly designed socket in the Tire Change Kits at the Terminal and Hangar. Last year, 2011, I strained my back due to this socket slipping off the [axle] nut while torquing a tire on an A320 and I was off work for over a month and went through several sessions of physical therapy. Tonight, I was involved with another A320 tire change and nearly a year later, I see nothing has been done to replace or fix this unsafe socket. After I came back from the hospital last year just after injuring my back and before going home, I had sat down with several Mangers, Supervisors and a company Safety Representative and discussed ideas to fix and redesign this socket. I am upset that nothing has been done to fix this. The potential for injury to me and my co-workers still exists; not to mention damage to the axle nut and possibly improper torque of this nut, from the sloppy fit of this tool. My most important concern is to keep co-workers from getting hurt like I was last year.

Synopsis

A Line Mechanic raises safety concerns about an unsafe and poorly designed axle nut socket tool used to remove and install Main Landing Gear (MLG) tires on A319/320 aircraft. The potential for possible injury to himself and others still exists including improper nut torque and damage to the axle nut from the sloppy fit of the tool.

ACN: 995139 (2244 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PHL.Airport
State Reference : PA
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : PHL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Landing

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : PHL
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Landing

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14100
Experience.Flight Crew.Last 90 Days : 125
Experience.Flight Crew.Type : 7800
ASRS Report Number.Accession Number : 995139
Human Factors : Distraction
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 995148
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Incursion : Taxiway
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On landing in PHL on [Runway] 27R we followed a B757 and experienced wake turbulence in the flare. This extended our touchdown in the touchdown zone. We were in reverse and slowing when ATC told us to exit and turn right onto Kilo to the gate. I advised them that we missed the right 90 turnoff and ATC advised us to hurry down to the next right turnoff which turned out to be K5. We kept our speed up on the runway and were slowing to try to hurry and exit at K5 for traffic on final. At this time we were given taxi instructions again. The clearance was right on Kilo and hold short of Tango. I answered as we were distracted by slowing in a hurry to turn off. I answered to turn right on Kilo like our first taxi instructions, but I am not sure if I acknowledged the hold short. As we taxied up to the parallel Kilo we saw [the other aircraft] landing and exiting on the high speed. We were short of the high speed and just slowing for [the other aircraft] to exit when ATC told us we were supposed to hold short of Tango. I looked to the left and saw the K4 taxi sign coming up. I knew we had to stop for [the other aircraft] to exit and I guess I thought that the high speed was Tango, not Kilo 4. [The other aircraft] was told to taxi to parking in front of us and then ATC advised us to taxi to parking. We taxied to the gate and we noticed I had forgotten the flaps in my after landing flow due to the hurried exit and answering ATC. The combination of the hurry to exit and answering ATC at the same time probably led to the confusion.

Narrative: 2

We encountered some wake turbulence (air was completely smooth until the last 50 FT). This did have [some] effect on the landing performance. We did land in the touchdown zone but further rather than closer to the approach end of the runway.

Synopsis

An A320 flight crew reported encountering wake turbulence just before touchdown at PHL behind a B757 that extended their touchdown point and caused some issues with ATC.

ACN: 994968 (2245 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZNY.ARTCC
State Reference : NY

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 70
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 994968
Human Factors : Fatigue
Human Factors : Time Pressure

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

I was scheduled for a Class II line check featuring a [northeast airport] to Saint Maarten (TNCM) dead head flight with an hour plus on the ground and then me flying the aircraft back as the Captain where I would also receive training and a check ride. I had an early report time, which for me is a cross town assignment making for a long drive and a very early get up to deal with traffic associated with [the area]. Looking at the trip a week or so prior to departure, I cringed at how long of a day it was going to be especially having to fly and receive training and checking after a dead head in the cabin of a full aircraft. While legal, I know of no pilot who considers dead heading in the cabin of full aircraft restful in any respect. I got to TNCM already tired. I felt like I had been put in a box by scheduling and had expressed my concerns with my Chief pilot about this trip a few days prior; asking him, "Why are we building training like this?" This was not an operational necessity, it was a choice made by the line check desk to have me trained under these conditions. I contemplated declaring myself fatigued in TNCM but I felt tremendous pressure to fly the flight. I knew if I did not operate the flight we would have 150 stranded passengers and the flight would cancel. I also knew that I would have to deal with a ration of phone calls and perhaps even threats of discipline if I canceled the flight. I told my Check Airman that I was dead tired and he, coming off of a 25 hour TNCM layover, agreed with me; stating that this was a poor way to schedule training and that in his opinion a flying leg down with a layover and a flying leg out was the correct way for a pilot to see both the entry and exit into TNCM (which can be challenging) and to receive the Class II Training. We were planned at max gross, every seat full, taking off from TNCM, with a planned landing fuel of 5.9 LBS. While I was very tired, we had an uneventful flight up to the East Coast, flying the planned profile altitudes and cruise speeds. ATC had us descend nearly 200 miles from destination and with the over burn due to heavier than forecast head winds, fuel started to become an issue. New York ATC vectoring into sequence compounded the over burn issue and we declared "Minimum Fuel." We were eventually vectored onto the final just as a rain storm was rolling onto the field. We picked up heavy precipitation at approximately 500 FT AGL and with the fuel at 3.8; I thought to myself, this is going to be a very bad situation if we have to go around for wind shear now. We made an uneventful landing but I recall my heart beating in my chest rather rapidly for the last 15 minutes of the flight due to the fuel situation, weather and the countless cups of coffee I was forced to drink throughout the day to overcome the fatiguing nature of this training assignment. I recall thinking to myself while waiting in line to clear customs that this assignment had red flags all over it prior to it beginning. With 25+ years of military and civilian aviation experience, I saw the potential difficulties prior to beginning the assignment and even voiced my concerns to my Chief pilot with the complaints being answered with "nothing I can really do for you, it's all legal." Every mishap happens with a chain of events that proceeds it and while this situation ended normally, I walked off that aircraft absolutely spent and had difficulty sleeping again that night thinking about the "what ifs." By the time I left the airport I had been on the property for over 13 hours, had over 8.5 hours of combined dead head and flight time and had not slept for 17+ hours. It was one of the longest and most stressful days I have had flying in a long time and again this was not part of some irregular operation; this was scheduled training. Note: The Check Airman who conducted the training was both fair and competent, and while the day was a completely negative experience by the nature of the schedule, the Check Airman did his duties in a professional manner considering the circumstances.

Synopsis

A320 Captain describes a fatiguing training/line check scenario beginning with a long drive for an early check in to deadhead to TNCM. Training/checking occurs on the return leg ending with a low fuel declaration and landing in a down pour.

ACN: 994930 (2246 of 2908)

Time / Day

Date : 201202

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 9500

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 145 ER/LR
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 994930
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

OJT was in progress and we were working a busy final session. Our attention was primarily focused 20 miles north of the airport because that is where it was busiest at the time and we didn't have any down winds. I did not see the departure at all until the A320 said he was getting a TCAS/RA. At that time I looked at him to see what his traffic was, and it was only then that I saw the E145's limited Data Block. I immediately looked at the altitudes to see if there was anything I could do, but by then the E145 was already 6 to 800 FT above the A320 and the interval was increasing. The A320 continued his approach and landed. I couldn't tell any action that he took to the RA. We should always keep up our scan. One thing I think that contributed to me not seeing the departure personally was that I was watching from slightly behind and off to the side of my trainee. My trainee sets his scope up quite a bit differently than I would. He doesn't use as much contrast as I do between full Data Blocks and limited Data Blocks. He also has his digital map quite a bit brighter. All these items make me have to work harder to see what I need to see. He would adjust it more to my liking if I asked him to, but I try to work how he is comfortable, if I am able. We are told to always use the same scope as the trainee so we always see the same thing; however, I move to my own scope while training, whenever I have the opportunity. Being able to set up my own scope my own way is very advantageous to me.

Synopsis

TRACON Controller described a loss of separation event when failing to note the conflict aircraft in time to prevent the occurrence. The reporter indicated that the Developmental's RADAR display set-up contributed to the error.

ACN: 994693 (2247 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 994693
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13900
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 3800
ASRS Report Number.Accession Number : 994695

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Inbound flight logged 2 items- SYS 1 FAULT DISPLAYED ON UPPER ECAM-BRAKES and AUTOBRAKE FAULT DISPLAYED ON UPPER ECAM SCREEN-BRAKES. Maintenance deferred the first ECAM with MEL 32-42-03 BSCU 1 inoperative. That deferral makes a reference to the autobrake panel mode lights being inoperative and has a note FOR BSCU CHANNEL/SYSTEM 1 INOPERATIVE REFER TO MEL FOR AUTO/BRK PANEL MODE LIGHTS DEFERRAL. Maintenance took that reference and used it to defer the second ECAM listed above (MEL 32-42-04). In my review of the MEL, once on board, I did not question the linking of the second ECAM to the first ECAM MEL, guidance that was made by Maintenance. I ASSUMED that Maintenance had specific guidance that the second ECAM was a result of the loss of BSCU 1 and the autobrake panel lights, nothing more complex that had to be evaluated for compound failures. We complied with all MEL restrictions prior to departure to include verifying that the autobrakes were armed on the wheels SD page. Checking recall prior to departure only brought up the first ECAM. No autobrake ECAM or status messages were present. This furthered my belief that the maintenance issues had been addressed correctly. Taxi, takeoff and initial climb were normal. Nearing FL350 we received the same autobrake fault ECAM received by the first crew. We complied with the Irregular Procedure. We determined that we had lost normal brakes but did have antiskid. Further review of the Flight Manual to include the system resets section made it apparent to me that the autobrakes ECAM, while obviously related to the brake system fault ECAM, was not linked to it as a secondary failure due to the lights failure. We had two separate failures, as did the inbound flight that should have been treated as such. The maintenance dispatch was improper/illegal. Trust but VERIFY everything! Careful verification of maintenance actions could have prevented this.

Narrative: 2

Illegal dispatch; the aircraft was dispatched with a BSCU channel 1 fault, along with the annunciator lights for the autobrakes inoperative. In fact the autobrakes and the BSCU 1 were written up on the inbound flight and Maintenance interpreted the autobrake fault as just an annunciator light issue. The plane should have been dispatched with an autobrake fault and BSCU channel 1 fault. Enroute we ran the autobrake fault ECAM, contacted Dispatch and Maintenance and continued to destination.

Synopsis

A320 flight crew experiences an autobrake fault in cruise after the fault had been previously written up and diagnosed as an autobrake panel light fault and MELed as such. BSCU 1 was also MELed separately. Flight continues to destination.

ACN: 994436 (2248 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 19675
Experience.Flight Crew.Last 90 Days : 244
Experience.Flight Crew.Type : 2300
ASRS Report Number.Accession Number : 994436
Human Factors : Physiological - Other

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Security
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Upon setting the parking brake and shutting down the right engine, the Captain announced he had to go back and use the restroom. Upon returning to his seat, he said that his understanding was that since the aircraft was effectively disabled with an engine shutdown, he could do so without calling the Flight Attendant and setting up the barrier. We then taxied to the gate without further incident.

Synopsis

Air carrier First Officer reports the Captain using the restroom facilities while waiting for a gate with one engine shut down and the parking brake set, without summoning a Flight Attendant.

ACN: 994141 (2249 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Aircraft Auto Temperature System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 994141
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected.Other 
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

SKIN VALVE FAULT ECAM alerted on takeoff roll. Rejected takeoff at 75 KTS.

Synopsis

An A320 Captain reject the takeoff at 75 KTS after the ECAM alerted SKIN VALVE FAULT.

ACN: 993708 (2250 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Reverser Lockout
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Hydraulic Main System
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 18
ASRS Report Number.Accession Number : 993708
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Human Factors : Workload
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

Basically since the new Director of Maintenance (DOM) spot was assigned, he has pushed maintenance personnel to their breaking point in the form of rushing the work on aircraft and threatening that if the aircraft doesn't make it out in his time frame "heads will roll", meaning whoever is involved will be fired. Case in point we have had two instances where the Thrust Reverser Hydraulic Control Unit (HCU) lockout pin was left in the aircraft causing the thrust reverser not to deploy on landing. Mr. X came in during the first instance, (I was there), and rushed Aircraft Maintenance Technician X, (AMT) to hurry and finish the work he was performing on a V2500 Engine #5 Oil Bearing Job Card; and in hurrying AMT X forgot the lockout pin. Honestly, I cannot remember the last time a HCU Lockout pin was left on the aircraft BEFORE this DOM arrived. 

All the maintenance personal were and are fearful for their jobs, making a less than desirable place to work as well as asking for an accident or mistake to be made. I know there would be more of these safety reports filed if the mechanics were made aware that our Air Carrier is interested in hearing of any possible safety problems they encounter as well as assuring the mechanics there will be no negative actions taken against the mechanics for filing a safety report.

We are constantly being rushed to HURRY and work on aircraft; problem is sometimes we are given unrealistic time frames. I understand the need for aircraft to fly as quickly as possible, but not to the point of rushing people beyond their means. 

I suggest a meeting from upper management with ALL Company Maintenance Bases and advise them of the importance of notifying the Safety Department when they feel something is being performed in an unsafe manner. Also need to let personnel know there will be no negative actions taken against personnel for making such a notification as long as the guidelines set forth in the Safety Manual are met. Finally take a look at the time frame we are given and ensure we have adequate time to perform the job at hand without sacrificing safety.

Callback: 1

Reporter stated he has eighteen years Maintenance experience, twelve with the Air Carrier he currently works for. His company closed the only hangar they had about a year ago. All their maintenance work is accomplished outside on the Line, including engine changes in the rain. His Air Carrier has even divided up their C-Checks into segments, so they would not be required to have a hangar to perform a full Maintenance C-Check inside. Their Heavy Checks are worked at a Foreign Repair Station.

Reporter stated they are losing many mechanics; the turnover rate is very high. The experience level of most of their remaining mechanics is really low. According to the reporter, the current Director of Maintenance (DOM) has little experience and is the fourth Director in twelve months; mostly due to unrealistic maintenance goals and no upper Management support of the DOM.

Reporter stated at the time he filed his NASA ASRS report he was their Midnight shift Maintenance Supervisor. Since then, he has resigned as Supervisor due to the same unrealistic company goals and attitude towards the Maintenance group.

Synopsis

A Maintenance Supervisor describes Air Carrier Line Maintenance environment that includes low experience mechanics, high mechanic turnovers, a Director of Maintenance (DOM) who pushes mechanics to "rush" their work, no maintenance hangars for their aircraft resulting in engine changes that are accomplished in the rain.