ACN: 992185 (2251 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : Mixed
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 125
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 992185
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5200
ASRS Report Number.Accession Number : 992188
Human Factors : Training / Qualification
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon extending landing gear, we received an ECAM WHEEL N/W STRG FAULT. We performed a go-around, referred to flight manual for ECAM, completed appropriate checklists, and determined that since the ECAM was NOT combined with a shock absorber fault that the nose wheel was in a normal position aligned with the runway. We declared an emergency, notified Dispatch via ACARS. I advised the flight attendants of the situation and that we were not planning an evacuation. I initially informed passengers via PA that we had gone around for an indication that necessitated the go around. Subsequently I briefed them that there would be emergency equipment approaching the aircraft after landing and that we had an indication of a problem that may inhibit our ability to taxi after landing (something to that effect, I don't recall the specific wording). We landed uneventfully and cleared the runway on the high speed taxiway. We stopped the aircraft and made contact with fire and rescue. 

We then performed the Flight Manual prescribed system reset for the fault. Nose wheel steering returned to normal indications and we proceeded to the gate once fire and rescue were notified and clear. After landing I contacted both Dispatch and the Flight Manager via telephone to update them. A couple of items of note: 1) The Flight Manager indicated to me that operations was under the impression we had performed a "fly-by." We did NOT do a fly by; we only did a go-around in order to have time to analyze the situation and appropriately deal with the problem. Tower did re-assign us from 1C to 1R for landing. 2) Upon parking at the gate, as soon as the door opened there was a Customer Service Supervisor, CS Agent, Flight Attendant Supervisor, 2 Mechanics and an Assistant Chief Pilot that all entered the plane before the people had even exited. While we appreciated the show of support it was distracting and disruptive. The Flight Attendant supervisor immediately wanted to know if the flight attendants were OK to continue. I had not even had time to de-brief the crew and would have appreciated everyone waiting until we had deplaned. We had also had a minor medical issue on the same flight. I wanted to thank the Doctor onboard that had assisted and speak to the woman he (and med-link) had assisted. We did all debrief as a crew once we had deplaned.

Narrative: 2

[Narrative #2 has no additional information.]

Synopsis

An A320 ECAM alerted WHEEL N/W STRG FAULT when the landing gear were extended, so a go around was executed to comply with the QRH, an emergency declared and a normal landing completed followed by a steering fault reset which allowed a normal taxi to the gate.

ACN: 992183 (2252 of 2908)

Time / Day

Date : 201202
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Snow
Weather Elements / Visibility.Visibility : 0.75
Light : Daylight
Ceiling.Single Value : 300

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 35L
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Nosewheel Steering
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 992183
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Weather: snow 3/4, visibility 300 overcast. ILS 35, Captain pilot flying. Aircraft configured on speed just out side the outer marker. Right before marker and glide slope intercept, ECAM alerted WHEEL N/W STRG FAULT. [We] briefly discussed options and meaning of ECAM while continuing approach. Told Tower we needed to break off approach and work issue. [We] climbed to 7,000 FT then soon after turned east and climbed to 9,000 FT. Ran WHEEL N/W STRG FAULT check list. No actions available in flight. Discussed issue "no nose wheel steering." [We] discussed techniques, outcomes, and reaction to outcomes due to no nose wheel steering on ground in snow and low visibility conditions upon landing. Briefed flight attendants and deadheading crewmembers. [We] attempted to contact Dispatch and Maintenance via ACARS. No reply in time we had decided to get on ground due to deteriorating weather conditions and the possibility of lower CAT approach weather than the aircraft potentially had capability of. [We] briefed new approach [and] ran normal checklist. 

DECLARED AN EMERGENCY and had the trucks roll with Approach Controller. No nose wheel steering on FAIR braking conditions on runway. [We] informed ATC of potential needs; TOWING. ILS 35 approach normal. Touch down and rollout normal with normal procedure; differential braking until 25 KTS. At approximately 25 KTS tested to steer aircraft with nose wheel steering. NO nose wheel steering available. [We] came to a full stop on centerline using differential braking. Contacted Tower and emergency vehicles, all was fine on and with the aircraft except we could not steer aircraft and needed tow. Shut down engines, coordinated with Ground Operations for tow in, briefed flight attendants and passengers. [We were] towed to gate, discussed event with Maintenance. Maintenance confirmed fault.

Synopsis

An A320 ECAM alerted WHEEL N/W STRG FAULT on approach so a go around was executed to comply with the QRH, an emergency was declared, and a normal landing completed on a snowy runway after which the aircraft was towed to the gate.

ACN: 991887 (2253 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : BOI.Airport
State Reference : ID
Altitude.MSL.Single Value : 6300

Environment

Flight Conditions : VMC
Light : Night
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : BOI
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class C : BOI

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1900
ASRS Report Number.Accession Number : 991887
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

On the visual approach to 28R at night we got a "caution terrain" warning. The warning occurred at 6,300 FT with a radar altitude of 1,850 FT. We climbed to 7,300 FT and in the process oversped the flaps by 10 knots. The Tower recleared us for a right downwind visual to 10L. After landing the overspeed was written up and Maintenance notified.

Synopsis

An A320 crew executed the escape maneuver after an EGPWS CAUTION, TERRAIN warning at 6,300 FT on a night visual to BOI Runway 28R ten miles from the airport near a 4,180 FT obstacle.

ACN: 991779 (2254 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Cockpit/Cabin Communication
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Cabin Address System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 991779
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Avionics
ASRS Report Number.Accession Number : 991781
Human Factors : Troubleshooting
Human Factors : Confusion

Person : 3

Reference : 3
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 991780
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

Aircraft X, an A320 aircraft, arrived at gate with a log gripe concerning the Cabin Intercommunication DATA System (CIDS). The flight crew verbally informed us that they had a CIDS Director-1 and -2 ECAM fault and an Evacuate Warning that would not silence. Due to previous history of CIDS and Evacuate Warning problems, my Aircraft Electronics Technician (AET) coworker, Technician Y, and myself determined that we would replace CIDS Directors-1 and -2. The new CIDS Directors were ordered referencing Illustrated Parts Catalog (IPC) 23-73-34 for this aircraft effectivity. The last update of this document was May 2011. During the installation of the CIDS units, it was noted that the Company Part Numbers (CPN) of the old and new [CIDS Director] units were different. 

The IPC calls for CPN XXX729 for this aircraft effectivity, however, CPN XXX732 CIDS units were found installed. We contacted Maintenance Control for clarification and Maintenance Controller X agreed that this aircraft effectivity should have the CPN XXX729 CIDS units installed per IPC 23-73-34, Page-1f. Per IPC 23-73-34, Page-1, the CPN XXX731 unit is effective only for Aircraft Y, [a different A320 aircraft]. Installation and Test of both CIDS Directors was accomplished per Aircraft Maintenance Manuals (AMM) 23-73-34, with all tests passing and aircraft was then dispatched. 

A few minutes after the aircraft left ZZZ, my Supervisor, found an Airbus Maintenance Tip (MT) in the AMM. The Maintenance Tip was found to have a Part Number effectivity table that is in conflict with the current IPC. We immediately contacted Maintenance Control, this time speaking with Controller Y, and informed him of the conflicting part number information on these documents. Controller Y concluded that proper procedures were followed and that he would contact the Engineering Group to correct the documents in question. In addition I submitted a report to Engineering about the matter and hope to get a response soon.

Narrative: 2

Aircraft X, an A320 aircraft, arrived with a logbook write-up of Cabin Intercommunication DATA System (CIDS). The crew reported verbally and the log was updated to (Evacuation Warning sounding and could not reset). The crew also reported and the post flight report showed the CIDS Directors-1 and -2 had failed. The fault could not be duplicated on the ground. It was determined that it would be best to replace both CIDS Directors reference Maintenance Manual (M/M) 23-73-34. The parts were ordered reference Illustrated Parts Catalog (IPC) 23-73-34 Page-1F, dated (May 2011). The parts arrived and [it] was found that the parts [previously] installed were different.

Narrative: 3

Received a call from ZZZ Maintenance. I was asked to verify a Company Part Number (CPN) for the Cabin Intercommunication DATA System (CIDS) Director on Aircraft X, an A320 aircraft. I received the CPN and the Illustrated Parts Catalog (IPC) reference IPC 23-73-34 from the Mechanic. I reviewed the IPC and determined that CPN XXX729 was the correct effectivity for Aircraft X. 

I was then told about an Airbus Maintenance Tip (MT) showing CPN XXX731 was the effective CIDS Director for Aircraft X. The revision date on IPC 23-73-34 is May-2011. The revision date for MT XXX is February 2004. With the information I had at the time, I concluded the most up to date document was the IPC, and that the correct CPN was installed in the aircraft. I gave all of this information to the Maintenance Control Fleet Manager for the Airbus fleet. He informed me he would get the discrepancy resolved.

Synopsis

A Line Technician, Avionics Technician, and a Maintenance Controller report about conflicting information between their Illustrated Parts Catalog (IPC) and the Airbus Maintenance Manual regarding the correct Cabin Intercommunication DATA System (CIDS) Directors they could use on an A320 aircraft with a CIDS Director ECAM fault and Evacuation Warning that would not silence.

ACN: 991726 (2255 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLC.ARTCC
State Reference : UT
Altitude.MSL.Single Value : 37000

Environment

Ceiling : CLR

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLC
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Phase : Cruise
Airspace.Class A : ZLC

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15800
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 7200
ASRS Report Number.Accession Number : 991726

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 1100
ASRS Report Number.Accession Number : 991881

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

At approximately 15 NM west MLF [at] FL370 (75 south southwest DTA) we experienced one moderate bump of turbulence in clear air. Bump was suggestive of wake turbulence (perpendicular track) versus a weather phenomenon. Smooth air was experienced for a few minutes prior and a few minutes after. The greater area was described as containing occasional light chop (by other aircraft relayed by ATC), but seat belt signs were on mostly due to recent light chop experienced combined with earlier reports of moderate chop west of DTA. All passengers were seated. Our flight attendants were aware of earlier ride reports in region, but had been updated by me of more recent and local light chop reports. Approximately [an hour and a half later] I was informed by our First Flight Attendant that both flight attendants in the rear of the airplane had been injured during that event. She had been informed immediately and I believe that all thought that the other party had informed the cockpit.

[One of the flight attendants reportedly] descended to her knees. [She] described a strained or stiff back and a bruised knee. [Another flight attendant] was in the aft cabin and describes bruising her elbow as she caught her balance. I became aware of injuries as meals were passed to the cockpit and during a lavatory break. We had not been called previously, nor had it been discussed during a call for cabin temperature or for meals to be prepared. It was obvious that [one of the flight attendants] was in pain, but she did walk to the forward cabin to speak to me during my break. Our Dispatcher was informed immediately after we collected the necessary information. An in-flight Supervisor was requested and we were met at the gate on arrival. The cockpit was informed within ten minutes of the top-of-descent that [the injured Flight Attendant] would occupy a passenger seat for landing due to injury. A Law Enforcement Officer was identified at that time as an able bodied helper if one was needed for an evacuation.

Narrative: 2

We encountered momentary moderate turbulence that lasted approximately 2 seconds. It felt like classic wake turbulence because it stopped as fast as it started. Their was no aircraft that [we] suspected of causing the wake although we were in trail of another aircraft prior to encounter that was above our altitude and 15 NM ahead.

Synopsis

An A320 flight crew reported a wake turbulence encounter that caused minor injuries to two flight attendants.

ACN: 991717 (2256 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Cabin Address System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 991717
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Primary Problem : Human Factors

Narrative: 1

Company A320 aircraft arrived with some maintenance write-ups. Some cabin incidentals, but the big thing was an inoperative (red light) on the Flight Attendant's Cabin Intercommunication Data System (CIDS) panel. On walk-around, First Officer observed fresh fluid stream on tail and brought to attention of Captain who inspected it as well. We concluded it was necessary to write-up, which we did. A different Mechanic arrived shortly thereafter to deal with this and the first words out of his mouth were that he was not familiar with the Airbus and not been trained on it. 

This became obvious the more he talked. We were not comfortable with his assessments...first deice fluid, then hydraulic fluid, then stating that all Boeing [aircraft] have a lot of fluid originating in the main wheel well area...note this is an Airbus! He also stated that this was old hydraulic fluid because it was tacky. Yes on the belly it was tacky, but on the tail where the big stream was it was flowing fluid and actually streaming to the ground now that there was no slipstream. Captain finally convinced Mechanic to drop main gear doors to inspect the belly bay. Note...this Mechanic who knows nothing about Airbus, has yet to contact Maintenance Control! 

Upon dropping [main gear] doors, right door had fresh hydraulic fluid all over it, and left door is completely dry. This Mechanic didn't seem too concerned, stating that all the Boeing's look like this. At this point, Captain had enough of this Mechanic and called Maintenance Control himself. Maintenance Control became involved in giving this Mechanic guidance and solution to properly inspect and ascertain whether or not there were issues to be addressed. Tests (all hydraulic systems pressurized, various components selected "On") and checks (fluid levels, etc) were subsequently accomplished and it was determined that this fluid was the result of an over-servicing. In the process of running some of these tests, the flight crew had to show Mechanic where various switches and displays were located...basic stuff (Hydraulic System Page on Lower [ECAM] Display, Yellow [Hydraulic System] Electric switch, Blue [Hydraulic electric override on overhead Maintenance panel, etc). After observing tests and checks and speaking with Maintenance Control, Captain was satisfied that all maintenance issues on aircraft had been properly addressed and a Maintenance Release was issued by Maintenance Control for departure of next flight. 

Bottom line...this incident could have been resolved much quicker and timely had the Mechanic who met the aircraft been properly trained and consulted with Maintenance Control for guidance and instructions from the beginning of his trouble-shooting process.

Synopsis

A First Officer reports about a Captain's frustration with a Line Mechanic who was sent out to their gate to determine the cause of a fresh fluid stream on the belly and lower tail surface of their A320 aircraft. Company Mechanic did not have any training or experience on Airbus aircraft.

ACN: 991569 (2257 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component : 1

Aircraft Component : Escape Slide
Aircraft Reference : X

Component : 2

Aircraft Component : Emergency Exit
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 991569
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Upon preflight, found aircraft left [overwing] emergency slide panel release pins painted over (forward 2 of 3 pins). This was confirmed by the First Officer, then by Mechanic X. Also, via ACARS, asked Maintenance Control to please reference company Alert Bulletin XXX. Per Alert Bulletin XXX and Engineering Order 25-YYY, paint should be removed from the overwing slide enclosure ball locks. Maintenance removed the paint from the effected areas, and then reported to Captain and First Officer the ball locks were never painted over. Logbook sign-off read, "ATA Chapter 05-35: Please comply with Engineering Order (E.O.) 25-YYY, inspection of both sides, left/right paint removal from the overwing slide enclosure ball locks." Suggest Maintenance, as well as the pilot group, could have this information disseminated, to avoid a potentially dangerous situation and to be in compliance with the 2008 Engineering Order and the 2011 Alert Bulletin. The Paint Vendors could be more proactive in not painting over the ball lock release pins.

Synopsis

A Captain reports he found the left overwing emergency exit slide panel release pins painted over during a preflight of an A320 aircraft.

ACN: 991492 (2258 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 15000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 991492
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Departure strip printed and I retrieved for an A320. Pilot checked in climbing to 3,000 FT on runway heading. I observed the primary target off the departure end, established RADAR contact with the aircraft, and instructed it to climb to 15,000 FT and turn left heading 360. I meant to ask the pilot to make sure transponder was on, but did not. I got busy with other departures and arrivals and forgot about the primary target aircraft. Pilot called back about ten minutes later to verify he was supposed to still be on my frequency. I expanded the RADAR scope range and asked the pilot to turn the transponder on, the A320 tagged up about 30 miles beyond Approach airspace boundary, level at 15,000 FT. I called Center to perform a late hand off, Center accepted the hand off and I switched the aircraft. Recommendation, be more alert.

Synopsis

TRACON Controller described an airspace incursion event into an adjacent Center sector when departure traffic, without a transponder, went unnoticed by the reporter after initially being RADAR identified as a primary target off the runway.

ACN: 991380 (2259 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 991380
Human Factors : Distraction
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

[We] arrived at aircraft mid morning. Overwhelming exhaust smell permeated entire aircraft. Crew could not even stay on aircraft. Aircraft was towed over from remote location to gate before we arrived. There was no external air hooked up at the time of our arrival and the APU was off. We believed that the exhaust smell was from an external air cart hooked up while the plane was at the remote parking site. We turned on the APU to get air flowing and get heavy exhaust smell out. We did not allow passenger boarding until it was less prevalent. Also, we opened the cockpit windows to try to get fresh air in. The exhaust smell was much heavier in the cockpit. After boarding we taxied to deicing. Pack flow on high to try and get smell out. Exhaust smell still present. We took off and 20 minutes into the flight exhaust smell still present. Pack flow still on high. I elected to put on oxygen mask at 100%. The Captain put on his mask too. At this point I am sorry I did not put on mask earlier. The smell [was] still present. We rang back to flight attendants. They smelled it, but not strong. The Captain sent messages to Maintenance/Dispatch about the exhaust fumes smell. I left my mask on until 3 mile final. After turning off of runway I opened my cockpit window for fresh air. Upon arrival at gate, Maintenance, Chief Pilot and other supervisors met the aircraft. I believe that having our maintenance done in Central America and hiring contracted vendors elsewhere has degraded the quality of maintenance performed and is sub-standard. This is a recurring problem at this airline. The problem of fumes persists and continues. This is the second fumes event for me. I am very concerned about the affect of these events on my health. The passengers are also being put at risk. Everyone is aware of this problem; however, it appears profits are put ahead of health and safety.

Callback: 1

The Reporter stated that the odor she experienced during this event was pure oil. On a previous similar event the smell was that of dirty socks and after that event the lingering effect she noted was abnormally high blood pressure for a period of days. She did not experience prolonged effects after this event because she donned her oxygen mask early. After landing four mechanics and the Chief Pilot all acknowledged the oil smell. One person noted that when the main cabin door was opened the odor was detected in the jetway prior to entering the aircraft.

Synopsis

An A320 crew detected an oil odor during preflight which dissipated but at cruise flight the odor again became very strong so that the crew donned oxygen until landing.

ACN: 991347 (2260 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : GPS & Other Satellite Navigation
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16300
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 5060
ASRS Report Number.Accession Number : 991347
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

Was approached in flight operations by a fellow Captain who wanted to provide me information about the aircraft I was to fly. It seems there are several aircraft in the Airbus fleet that have GPS installed, BUT IT IS NOT CONNECTED. She discovered this one day by pushing the Data button on the FMC and looking at the GPS data on an aircraft in flight. The data displayed was not there, indicating no GPS. That was this aircraft. According to her, she spent considerable time talking with Dispatch and Maintenance Control. She filed a report with her concerns and discoveries. She was told a bulletin would perhaps be issued. After speaking with this Captain, I called Dispatch and Maintenance Control. I had a lengthy discussion with them about the issue. He said there was a prior Captain a couple weeks ago who brought up the same issue (probably the one I talked with). After much discussion and digging he also found that THE PRIOR NIGHT the GPS had been connected on my aircraft, so all the navigation systems were fully operational for my flight into Mexico. He indicated that the Airbus fleet put out the GPS guidance prematurely, with no definite plans to connect the GPS on several aircraft. I thanked him for his time, went to the aircraft in the International terminal, and pushed back late. 

Enroute I asked Maintenance Control to ACARS me the aircraft that do not have GPS connected. There are currently eight aircraft in this category. According to the Flight Manual and the Airbus fleet, all aircraft now have GPS. Accordingly, pilot procedures have changed in the Flight Manual to reflect this. For example, we no longer have to load a runway shift for an intersection takeoff. Also, we no longer are required to perform a Nav Accuracy Check prior to the approach and arrival: GPS is installed on all the aircraft. This situation raises some questions in my mind: It appears that the Company has been dispatching aircraft without GPS (installed but not connected) on many revenue flights, both domestically and into Mexico. For these flights, nowhere on the release, flight plan, maintenance history, etc., is it documented that GPS is inoperative. The Captain is not informed of this inoperative equipment, yet signs the release. Is this legal per the FAR's? Pilots on these flights are conducting Normal Procedures per the FM that assume GPS is working on all Airbus aircraft. These flight crews have no idea that GPS is inoperative. Is this legal? --Maintenance Control responded via ACARS that the inoperative GPS wouldn't be on the flight plan, but if you were dispatched to an airport with only a GPS approach, dispatch would get you a different aircraft. However, we fly RNAV approaches on the line, and legally can do so. According to the FM, GPS is required for RNAV approaches. Have these aircraft flown RNAV approaches without an operative GPS? Maybe I am not informed or missing some information, but it appears that some questionable operations are being performed in the Airbus fleet. It would seem to me that when an aircraft is missing a key piece of navigational equipment, it should be properly documented and accounted for (CDL/MEL), with the crew being informed and following any "Flight Crew" procedures per the MEL. This doesn't seem to be the case.

Synopsis

An A320 Captain discovered that although all of his Company's airbus aircraft have GPS installed, not all of the systems have been connected. The Flight Manual does not reflect this and appropriate MEL actions are not being accomplished.

ACN: 990998 (2261 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 800

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 990998
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 990999

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We were cleared for the FMS Visual 28R SFO. (We saw the airport 30 miles out) after loading the FMS Visual I hard tuned the ILS frequency for 28R in my MCDU. While descending on the FMS Visual we armed the approach in error prior to reaching the final approach alt. realizing this I elected to hand fly the arrival and meet all crossing restrictions. At the FAF we were gear down flaps 3 and on speed. Inside SAMUL SFO Tower instructed us to sidestep to 28L, we complied. At 1,000 AGL we were flaps full on speed on vertical and lateral profile. At this point I was on the VASI for 28L, target speed plus 5 KTS. The pilot not flying informed me the ILS 28L was installed so I turned my Flight Director (FD) back on. However my FD told me to fly to 28R. I asked the pilot not flying to clear the hard tuned ILS to 28R so I might regain 28L FD info. At about 800 AGL we got a GPWS warning whoop, whoop pull up terrain pull up. I had complete confidence of our position, we were over SFO bay, short final on 28L on VASI (two red, two white) meeting all stabilized criteria, 30 miles visibility, there is no terrain 2 miles prior to 28L threshold, just bay. I knew where we were the FMGC did not. I elected to safely continue the approach and landing versus going around with a confused FMGC; landing was normal. When we parked at the gate we noticed both altimeters read -450 MSL. We have no idea how that happened both FCU altimeters were set accurately. The GPWS sounded twice when the pilot not flying removed the hard tuned 28R ILS. I had flew this aircraft on the prior leg and the FMGC failed enroute and I wrote it up, it returned to normal 5 minutes later so I canceled the write up. At first I thought we had an FMGC map shift when we got the warning until we parked and saw the altimeter error. From now on I'm not going to hard tune the ILS and correctly execute the managed approach.

Narrative: 2

[Narrative #2 had no additional information.]

Synopsis

A320 Captain experiences an GPWS terrain warning during visual approach to Runway 28L at SFO and continues the approach to landing. The ILS 28R had been hard tuned previously and had to be changed when cleared for Runway 28L. Both altimeters read 450 feet low at the gate.

ACN: 990897 (2262 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 23000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAB

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25700
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6900
ASRS Report Number.Accession Number : 990897
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Due to moderate to severe turbulence at higher altitudes, this flight was flown at FL240. We were cleared to descend via the EAGUL5 arrival into PHX, at which time the final altitude of the arrival was dialed into the window of the autopilot. Perhaps because we were starting at a lower altitude than usual for this arrival, the FMGC had eliminated the FL240 restriction at PAYSO. It was initially confusing, because the speed restriction (270) was not eliminated. I have recently had many issues with altitude restrictions not displaying properly in the new FMGC software (no ring displayed, particularly the red ring), and have attributed it to the software upgrade, though there has been no consistency from aircraft to aircraft with this anomaly. At any rate, and prior to PAYSO, the aircraft began a descent out of FL240 when it should not have. Rather than climb back up, I called ATC and got a clearance to FL220 so we could let the FMGC continue (I wanted to see what direction it would follow). Though below the "dot", it continued down, passing FL230 when the next fix constraint was FL180-230. The descent dot showed the box to know that it was below profile the entire time. I did not let the box descend below FL220 until passing PAYSO, at which point it did continue to descend below the dot, but remained within the follow on ALT constraint at the next fix. There were no other aircraft in the area, and separation was not a factor. I am going to play with lower altitude STAR intercepts to determine what the FMGC does to handle the intercept constraints which are not below the initial STAR intercept altitude.

Callback: 1

The reporter believes, after thinking back on the event, that a managed descent was initiated prior to reaching PAYSO.

Synopsis

A320 Captain cruising at FL240 and cleared to descend via the EAGLE5 RNAV arrival to PHX reports his aircraft descending of its' own volition prior to PAYSO with a minimum altitude of FL240.

ACN: 990657 (2263 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 24000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Airspace.Class A : ZAB

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 990657
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 990908
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

[We were] cruising at FL240 given the EAGLE5 Runway 25L transition. Set final altitude of 4000 MSL in altitude selector and verified by First Officer. At indicated top-of-descent point selected managed descent on autopilot and managed the speed. All crossing restrictions had been verified prior to descent. At PAYSO Intersection I noticed that we were at FL230 at 270 KIAS instead of the published FL240-FL280. Apparently the crossing restriction in the FMC had been deleted by our new cruise altitude (FL240). I had checked the crossing restrictions on the arrival prior to descending to FL240. When we descended to FL240, that altitude was added as a cruise altitude. I did not notice the restriction at PAYSO Intersection had been deleted (apparently automatically). I will be checking each crossing restriction as it comes up on the descent in addition to the normal pre-descent check of all restrictions.

Narrative: 2

Level at FL240 received clearance to descend via EAGUL 5 to 25L. Captain set 4,500 FT in FCU alt window but then pushed it. The airplane was still in cruise phase and this reset cruise alt to 24,000 thereby deleting the 24,000 minimum altitudes at PAYSO and the airplane started down. I had checked all alts in box prior to this but we did not catch the newly deleted restriction until about FL230. I then noticed problem and the Captain told Center and we were cleared to our present altitude. This has been a problem with the FMGC for years and has not been fixed.

Synopsis

A320 flight crew reports inadvertently deleting a crossing restriction on the EAGLE5 RNAV arrival to PHX, resulting in an early descent below the fix altitude.

ACN: 990086 (2264 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Snow
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZDV

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 1900
ASRS Report Number.Accession Number : 990086
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 990093
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Training / Qualification

Events

Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Air Traffic Control
Detector.Person : Dispatch
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

Two hour delayed departure to EGE for weather below minimums due to snow storm. [We] checked weather with Dispatch via ACARS just prior to launch; weather above minimums. Arriving at Top of Descent (TOD) for EGE, Denver Center advised the last aircraft (business jet) to land an hour earlier reported braking action poor. That's less than our landing data would allow. Entered holding at TOD and asked Center to get an update from EGE. Advised Dispatch and asked for current field conditions. EGE Tower advised the MU readings were 49, 57, 72. Dispatch relayed same. MU equivalents were recently removed from our manuals. Asked Dispatch if they had a correlation but they didn't. [We] eventually landed uneventfully based on field reports from EGE concerning runway condition and treatment. However, the only landing report prior to my arrival was poor and the only runway friction report was based on MU values that we no longer have available. This created an airborne situation and question concerning the viability of landing at EGE. Given that we operate into the difficult environment of EGE and the airfield uses MU readings, it would seem that safe operations would require MU based evaluation data for our aircraft at EGE to reliably evaluate a safe condition for landing.

Narrative: 2

Flight was enroute to EGE when ATC advised them that braking action CFM (Continuous Friction Measurement) values were .47/.59/.71. Crew advised me this via ACARS, stated that they were in the hold, and asked if I had "an equivalent to Good and Poor." I had previously gotten and advised them of a field condition report, based upon a statement by the Operations Agent at EGE, of a wet runway, with a MU value of 41; I do not know where that came from (it was probably CFM information from the airport). The FOM does not reference what CFM is. I called the Tower to confirm the information, and the Controller said that she didn't have a recent reliable braking action report from an airplane. I passed this on to the flight. Then I asked Tower to have the Airport Operations Manager contact me. I talked to the gentleman, who told me that those CFM values (I later found out, via the INTERNET, that CFM means Continuous Friction Measuring) are all that they are allowed to give out. He could not state whether those were equivalent to MU values. The flight landed shortly thereafter without responding to my ACARS message. If CFM is equivalent to MU values, or ICAO standard decimal values, can this information be put in the FOM?

Synopsis

EGE Airport issues Runway Braking Action Reports in a CFM format but because the flight crew and Dispatch had only MU numbers they could not accurately determine the braking action until the aircraft landed.

ACN: 989866 (2265 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 10000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class E : ZLA
Maintenance Status.Released For Service : Y

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 989866
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Shortly after takeoff, while retracting flaps, we got an amber ECAM alert that indicated the left side slats had not retracted. ECAM action was to recycle flap lever. We leveled at 10,000 FT and slowed to 210-220 KTS then recycled the flap lever several times, but failed to fix the problem. During that time we also tried to contact company on ACARS and the SAT phone, but had no luck. Was later told power was out at operations at that time. After about 10 minutes of trying to troubleshoot on our own with no company communication available, we decided to return to our departure airport for an overweight landing. We did not declare an emergency.

Synopsis

Upon receipt of an ECAM slat message during flap retraction, the flight crew of an A320 attempted to contact their company for assistance, but were unsuccessful. They then returned to their departure airport.

ACN: 989784 (2266 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 989784
Human Factors : Situational Awareness
Human Factors : Physiological - Other
Human Factors : Distraction
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Aircraft had one MEL 49-10 inoperative APU. Normal operations with assisted ground air start after starting engine #1 while still at gate, configured aircraft for taxi. Turning both packs on and opening the cross bleed, there was a very noticeable smell and odor. It was a dirty, sharp, acrid, smell in the cockpit. I called the First Flight Attendant; she smelled an odor and said the passengers were complaining about the smell too. I switched off the bleeds and packs and called the gate back to the aircraft. We deplaned all the passengers and I entered a discrepancy in the aircraft log for smell and odor filling out the additional maintenance work sheet. While waiting with the crew for operational decisions to be made. The flight attendants all had complained of headaches and dizziness. The First Officer said he had a headache, I had no symptoms. The flight was canceled and my crew all deadheaded to the crew base. The flight attendants went to the company clinic. Pilots were released and went home.

Callback: 1

The Reporter stated that the crew assembled in the airport lounge but that EMS was not called at that time. The flight attendants did go the Company clinic after the deadhead flight which was about three hours after the actual event. The Reporter did not know the outcome of the flight attendants' clinic visit. He did smell the odor but had no ill effects. The odor was difficult to describe but the term "dirty socks" would be appropriate. This aircraft had the APU MEL'ed because the same smell was detected by the previous crew resulting in the flight cancellation. An APU malfunction was suspected.

Synopsis

The crew of an A320 with the APU MEL'ed started engines at the gate, turned the bleeds and packs ON and immediately the aircraft filled with a dirty, acrid smell which made some crew members ill and caused the flight to be canceled.

ACN: 989688 (2267 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SEA.Airport
State Reference : WA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Icing
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : SEA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 6000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 989688
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 8200
ASRS Report Number.Accession Number : 989773
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

We were in Seattle and needed to get de-iced. The ramp personnel were having difficulty with the headset and were unable to communicate with us. We used the Operations frequency to coordinate the push for de-ice and were pushed just slightly off the gate with clearance from Seattle Ramp. The Captain was using hand signals and when the de-icing was complete the ground crew requested us to release brakes and we did. They began to push us further back and we had not gotten further clearance from Seattle Ramp. As we were pushing, I called Ramp and told them our ground crew had begun to push us back and we weren't able to communicate with them. Ramp said it was okay but were not happy that we started pushing without further clearance. In addition we never got a clear to start engines and had to go through Operations to get the ground crew back out to clear us to start engines.

Narrative: 2

The pushback crew was having trouble communicating with us on their headset. We were able to communicate this to the pushback crew and that the brakes were released. Pushback commenced. When we stopped we couldn't communicate anymore. Concerned they may move the tug more, we did not set the brakes. We called Operations and told them we couldn't communicate with the pushback crew. Operations had no way of communicating with them either. We communicated with the deice crew (on a deice frequency) that we were configured for deicing before the push. Deicing began with the brakes off while we were hooked up to the tug. When finished, the deice crew called and gave us the deice information. At the same time, the pushback crew appeared and gave us the "Set Brakes" signal - we did. Within a few seconds he came back and gave us the "Release Brakes" signal - we did. We had no idea what was happening. The airplane started to push back further on the ramp without clearance from SEA Ramp. Another aircraft was starting to push next door at the same time but stopped. 

We told Ramp we could not communicate with the pushback crew. When the airplane stopped we got the "Brakes Set" signal - we did. They disconnected and gave us the "Aircraft Clear" signal. We never got the "Cleared to Start Engines" signal so we didn't flash the taxi light. They just walked away anyway while the tug was backing into the gate. We were flashing our taxi light but they never looked back. We called Operations and told them we would not start engines unless we got a verbal from the pushback crew via Operations. Eventually someone came back out and gave us the "Cleared to Start Engines signal". This whole time we were blocking the aircraft next door from pushing. SEA Ramp called Operations on the phone to let them know what happened. Numerous pushback SOP violations due to intermittent headset that became inoperative.

Synopsis

An A320 Ground Crew headset became inoperative during pushback at SEA but the Ground Crew continued pushback after deicing without Ramp or flight crew clearance and blocked another aircraft.

ACN: 989284 (2268 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 989284

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 989290

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff we received an ECAM that the autobrakes had failed. Upon further investigation we had also lost normal brakes on top of a deferred item BSCU1. Due to deteriorating weather conditions in [destination] and the multiple system failures we conferred with Dispatch that in the event of any additional failures that the best course of action would be to declare an emergency and continue to [destination]. We landed without incident and canceled the emergency.

Narrative: 2

[Narrative 2 had no additional information.]

Synopsis

An A320 flight crew reported being dispatched with BSCU1 deferred, then losing autobrakes after takeoff. They continued to destination after declaring an emergency.

ACN: 989059 (2269 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 20000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use.STAR : TYSSN THREE
Airspace.Class A : ZLA

Component

Aircraft Component : Data Processing

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 989059
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This was a flight to LAS. Enroute to PGS ATC cleared us to FL260 and assigned us 280 KIAS. After we leveled off I re-cruised us at FL260. Our planned Cost Index was 36 which calculated a managed descent speed of 282 KIAS. However I had selected 280 on the FCU. East of PGS ATC cleared us to descend via the TYSSN THREE RNAV arrival for a visual approach to Runway 25L. I dialed in 8,000 to meet the restriction at PRINO. Both the First Officer and I had triple checked all the altitude constraints on the arrival, and we both had the constraints pb pushed. PGS has an "at or above FL200" constraint and CEJAY has an "at or below FL190" constraint. With a top of descent arrow displayed about halfway between our position and PGS, I engaged managed descent. I calculated we needed 21 miles to get below FL190 over CEJAY from FL260. PGS to CEJAY is 19 miles so I felt safe. I engaged managed descent, with the airplane starting a nominal 900 FPM descent, still in selected speed of 280. I selected the PROG (Progress) page on the MCDU and it showed I was approximately 2,400 FT low on the path but slowly decreasing deviation. The Descent Path Indicator (donut) on the Primary Flight Display (PFD) was at the top of the scale but moving down slowly. 

I felt very confident (not complacent) that we could meet all constraints without any problem. I was expecting a slower than normal descent since I had started down prior to the computed Top of Descent and we had a roughly direct headwind of 60 plus KTS. Prior to reaching PGS my very sharp First Officer remarked that we had an amber missed constraint circle around CEJAY on the Nav Display (ND). I acknowledged his concern, noted the PROG page showed we were still slightly low on the path with the donut slightly above center. In my mind we were intercepting the path from below. I did briefly check the VERT REV page for CEJAY (on the MCDU) and made sure it showed ALT CSTR -190. But for the life of me I did not think to cross check for an ALT ERROR. My mental math told me I had 19 miles to lose a bit over 5,000 FT. The First Officer again questioned me about the amber missed constraint circle. We were now past PGS still descending at approximately 1,000 FPM, but the donut was centered and the PROG page showed we were on the path. Not once did I think to check the FLT PLAN page and read the altitude over CEJAY. I was trying to come up with an answer as to why I was not getting a MORE DRAG message in the Flight Management Annunciator (FMA) since we had crossed one way point with a constraint. Or why I did not have a vertical discontinuity with a TOO STEEP PATH AHEAD message in the MCDU scratch pad. 

I became preoccupied with trying to diagnose the conflicting information between my Primary Flight Display (PFD) and my Nav Display (ND). I failed to recheck my mental math or aggressively correct our flight path. I erred in not advising ATC of my mistake. ATC did not mention my mistake. We crossed CEJAY in excess of 1,500 FT high. The First Officer reported he saw FL207 as we crossed. The MCDU showed FL209. At this point I assessed that the airplane would not meet the next restriction 22 miles ahead at KADDY. I disconnected the autopilot and selected full speedbrakes in order to cross KADDY at 250 KIAS and 12,000 FT. About 5 miles prior to KADDY at 12,000 FT and 250 KIAS I re-engaged the autopilot and managed descent. The rest of the profile from KADDY to PRINO was uneventful. PREVENTIVE MEASURES My First Officer deserves a great deal of credit for voicing his concerns about the amber missed constraints circle on the ND. We were anything but complacent. I was mentally scrambling trying to sort out conflicting information and determine a possible input error while thinking any moment the airplane is going to lower the nose and pick up the descent rate. For the life of me I cannot figure out what I did incorrectly.

Synopsis

An A320 Captain on the LAS TYSSN THREE RNAV calculated his descent constraint compliance incorrectly and failed to meet one of the constraints before realizing his error even though his First Officer was cautioning about the error.

ACN: 988900 (2270 of 2908)

Time / Day

Date : 201201

Place

Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Maintenance Status.Released For Service : Y

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 800
ASRS Report Number.Accession Number : 988900
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Our push back crew failed to secure the tow bar to the pushback vehicle, resulting in the aircraft being pushed back without control. I applied brakes gently after recognizing the problem, but the brakes did not respond immediately due to the generator coming on line after engine start. Shortly thereafter, the brakes did respond resulting in an abrupt stop. There were no injuries or damage to airplane.

Synopsis

When pushback of the A320 was initiated without the tow bar securely attached to the tug, the aircraft rolled backwards without restraint and had to be braked abruptly to a halt by the flight crew.

ACN: 988896 (2271 of 2908)

Time / Day

Date : 201201

Place

Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Position Computing System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 988896
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Lost nav capability on downwind ILS 34. IFR ceiling 500 FT visibility 1.5 miles. Went to STBY NAV Mode, manually tuned ILS's while on base. Got intercept vector. LOC did not engage, was still 15 miles out. Got re-vectored, did manual hand flown ILS. LOC was squirrelly. Uneventful landing. Got L/G SYSTEM DISAGREE ECAM during taxi in.

Callback: 1

The Reporter stated that the Map display failed on both ND's even though the ND's appeared to continue normal operation. The event occurred in very busy Class B and he momentarily considered executing a missed approach but because of fuel considerations and a very knowledgeable efficient crew they were able to make the required manual navigation radio selections. The lack of automatic loc and glideslope lock-on surprised him. He felt that the Company training for this type event was inadequate and that he was ill prepared for all that transpired. A checklist of some sort for manual radio setup and flight procedures would help but following the checklist would certainly require a go around to get properly set up. The basic fact he will remember is there is no NAVAID auto tuning and the aircraft is flown raw data. After arrival, the aircraft was powered down and after re-powering, all systems returned to normal with no ECAM or other annunciations indicating what had happened.

Synopsis

While on approach vectors, an A320 Map function failed with operating ND's so the STBY NAV Mode was selected, the ILS tuned, and the approach flown raw data to a successful landing. The systems reset after the aircraft was powered down at the gate.

ACN: 988463 (2272 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Released For Service : Y

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 988463
Human Factors : Physiological - Other
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While in cruise flight, approximately 20-25 minutes into flight, the Chief Flight Attendant called the cockpit and said that all the flight attendants had noticed a strange smell of "dirty rancid socks" at about row 6 and further back. All the flight attendants wanted to tell me what they smelled and were clearly concerned. I asked if there actually dirty socks or anything to account for the smell. They reported in the negative. I asked if it smelled like deicing fluid and they said no. (We had deiced prior to takeoff due to moderate accretion of mixed ice on arrival on the previous flight.) I told them that an airport was straight ahead and that we may divert and to keep me updated frequently. I asked the First Officer to contact Operation Control through the Operation Control frequency. Operation Control did not come up under repeated calls. (We later heard that it hadn't been monitored). He then made contact by relaying through Airport Operations frequency. I typed messages to Dispatch informing that we may need to divert and asked if there was a history of fume events on the aircraft. I called back to the flight attendants and asked if the fumes had dissipated. They said it "may" be less strong, but it was still there. I asked if the passengers had smelled it and they said yes, but had not reported it. They reported that a couple felt ill. Remembering the Union Safety Committee admonition and the Director of Flight's update on the fume events ("We are continuing to address each and every one of these air quality issues by immediately removing an affected aircraft from service.") to treat these events [as a] QRH Fume event and land at the nearest suitable airport; I decided to divert. I was concerned that we, the passengers and crew, were being exposed to TCP neurotoxins. I ordered O2 masks on, transferred control of the airplane to the First Officer to perform the flying duties, declared an emergency, and descended into the divert station. I informed the flight attendants and the passengers that we would be landing as a precaution in 15 minutes due to an unexplained odor in the cabin. There was little time to continue with the QRH Fume procedures or contact our contract medical service. We were vectored by ATC for a visual approach. I saw the runway first and resumed flying duties, landed, taxied to the gate and was met by paramedics. On taxi in I ordered the bleeds and packs to be shutdown. At deplaning, the flight attendants clearly seemed worried about TCP exposure. You could see it in their faces and eyes. We were checked out by paramedics and most of the crew was found to have elevated pulse and BP, and low O2. We declined transport to the hospital, being advised that we should call 911 if we felt ill at the hotel. They strongly recommended that we not continue flying at that time. The flight attendants were surprised after landing because a least two of the passengers, from row 6 and 11, hadn't sought medical treatment from the EMT's. They had reported feeling ill in flight. I faxed a cabin air quality report from the hotel.

Callback: 1

The Reporter stated that because of the heightened "dirty socks" awareness he did not hesitate to respond to the perceived neurotoxin threat although he First Officer attempted to talk him out of responding. His personal physical condition as determined by the paramedics was slightly elevated blood pressure and pulse, and oxygen level at about 90. The Reporter has been in contact with three other crew members who reported no lingering effects although all were to told to seek medical attention if they felt the need. The Reporter's Company did not report to him what was discovered and he personally wonders if maintenance outsourcing is a component in this problem. He was told that of the "dirty socks" events, maintenance reported the causes as follows: 58% APU, 30% engine, the remainder unknown.

Synopsis

An A320 Captain declared an emergency and diverted after flight attendants and passengers reported a dirty rancid sock smell from row 6 and beyond. Paramedics discovered elevated BP, heart rate, and low O2 levels.

ACN: 988453 (2273 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 988453

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4900
ASRS Report Number.Accession Number : 988456
Human Factors : Situational Awareness

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 988947

Person : 4

Reference : 4
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 989195
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

During pushback, the aircraft came to an abrupt halt. I asked the Captain and he said there was a breakaway from the tow bar and he applied the brakes to avoid rolling into an active taxiway. (As First Officer I do not monitor cockpit to ground communication during pushback.) The aircraft rolled about 15 FT from the tow bar. I queried the Purser and she said the 2 flight attendants in the back were mildly injured (possible bruised back, stiff neck, injured hand). We returned to the gate for a maintenance inspection of the nose gear and no damage was found. The Captain called the Duty Manager and the decision was made that the 2 injured flight attendants would not continue as working crew members.

Narrative: 2

During pushback, the pushback crew yelled, "Breakaway, breakaway, breakaway." The tow bar became disconnected from the aircraft. Not knowing how much clearance I had behind me in the alley I applied the brakes and stopped the aircraft. After the brakes were set, I called the flight attendants to check on their condition. The Lead Flight Attendant said that two flight attendants in the rear had been hurt. We returned to the gate to have both the aircraft and the flight attendants checked over. I called Maintenance, the Duty Manager, and Dispatch.

Narrative: 3

I was advised I was on a ferry flight. Did not sit in my jumpseat for takeoff. All required safety duties were preformed.

Narrative: 4

Plane was being ferried. Aircraft had become disengaged from tug and was rolling back into the alleyway on initial pushback, leaving the Captain to slam on the brakes. He was sent back to the gate for a check of the nose gear before continuing on. Once at the gate he was informed that two flight attendants had been injured. He went down to Operations and advised them of the two injured flight attendants and was told that they could not work the trip home. There was another aircraft at the next gate which was leaving earlier than the amount of time that it would have taken to re-crew our flight so passengers were deplaned and re planed on the other aircraft. Once everything was cleared, in terms of the aircraft being airworthy, the extent of the flight attendants' injuries, and the other administrative matters, the crew was permitted to ride back on our original aircraft. We armed the doors when the jet bridge was pushed back and we then all took a seat in first class and strapped in. Upon arrival once at the gate and seatbelt sign was turned off we disarmed all doors. We did not sit in our jump seats because there were no passengers on board and we were not aware that jump seats had to be occupied when ferrying the aircraft.

Synopsis

A320 flight and cabin crew describe a break away from the tug during pushback that results in Flight Attendant injuries and a ferry flight back to base. During the ferry flight the flight attendants do not sit in their jump seats.

ACN: 988450 (2274 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ramp : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Cabin Furnishing
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20462
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 7110
ASRS Report Number.Accession Number : 988450
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During pushback the Flight Attendant called the Flight Deck on the interphone and advised us that one of the video screens was inoperative. I directed the First Officer to enter the appropriate maintenance Code into ACARS. Due to the maintenance problem I decided to delay starting the engines. After pushback, believing that the video screen would be deferred by Maintenance Control, I elected to have the pushback crew disconnect form the aircraft and asked the First Officer to advise Ramp Control that we had a maintenance issue on pushback and would be waiting for Maintenance Control to contact us for deferral and a new Maintenance Release. After a few minutes the Ramp called and asked us to return to the gate so the Maintenance could look at the problem. Because we hadn't started an engine yet, I directed the First Officer to start the left engine. 

Shortly after the left engine was started and at idle RPM we both noticed a guide man and two wing walkers standing in front of the aircraft having a conference. Additionally, two Mechanics drove up with an elevated maintenance cart. The First Officer and I were a confused as to why the guide men were apparently not prepared to marshal us back to the gate with the two mechanics next to them on a maintenance cart. I asked the First Officer to call Ramp Control on the radio and ask them what was going on, no answer. Before we got any reply from Ramp Control, the two Mechanics on the cart approached the nose of the aircraft, below on the right side. The First Officer and I could no longer see the Mechanics below the nose. I thought that one of them was going to plug into the interphone to speak to us directly. As our confusion grew, the First Officer suddenly noticed a Mechanic on the elevated cart just outside his right side window by Door 1R. I quickly called the Flight Attendant on the interphone and ordered her to disarm Door 1R as it looked like a Mechanic was attempting to come onboard. I then ordered the First Officer to shut down the left engine. 

WITH NO ADVANCE COMMUNICATION OR COORDINATION WITH THE CAPTAIN, the mechanics boarded the aircraft and put a deferred sticker on the inoperative video screen. They did, however, have the courtesy to call us on the cabin interphone to say that the video screen was deferred and that we would receive a new Maintenance Release. At a loss for words, and in shock and disbelief, I asked the mechanic with whom I spoke who was responsible for making the decision to perform maintenance in such an unexpected and non-standard manner. He replied that Maintenance Control directed them to perform the maintenance off the blocks. Our Operations Manual states that maintenance will be performed by Station Maintenance when discrepancies are discovered prior to block departure and that maintenance discrepancies discovered AFTER block departure will be managed by Maintenance Control. 

In the end, what we experienced was a half baked shortcut, led by a comedy of errors that could have had caused an accident. When is it permissible for Station Maintenance to perform maintenance to an aircraft while it is "off the blocks?" If maintenance is required to be performed on an aircraft prior to take off, is the aircraft required to return to the blocks? I would like the safety department to research this incident to determine if any FAR's were violated.

Synopsis

An A320 flight crew was nonplussed when maintenance, without prior coordination, approached and boarded their aircraft after pushback to defer a passenger video screen which had been reported inoperative during pushback. This non-standard operation was apparently conducted to avoid a return to the blocks and the loss of an on-time departure.

ACN: 988376 (2275 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Aircraft Cooling System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 988376
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 988377

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Primary Problem : MEL

Narrative: 1

I was coming back from a canceled tow, when Dayshift Lead told me to stop by [another Gate] and help out Mechanic Y on an [aircraft] return to gate call. I printed out MEL 21- 26-04 and we went thru it together. When I went thru the process of opening skin air outlet valve per MEL 21-26-04 I got confused with the instructions provided. MEL Figure-A does not differentiate the [outlet valve] main door from the auxiliary door and internal valve doors. This picture only shows the handle, the On/Off switch and auxiliary flap (which appears to be pointing at whole door). Also instructions are a little confusing. So I made the mistake of partially opening the main flap instead of the auxiliary flap. Then I proceeded to finish the rest of the maintenance MEL procedure. I asked Mechanic Y to take a look at the MEL procedure and told him what I had done. He agreed that the MEL was confusing and agreed with what I had done. Then Mechanic Y did an Avionics Equipment Ventilation Computer (AEVC) Test and finished paperwork. We heard on the radio that the airplane was coming back to the gate. Air turn back [due to] avionics outlet skin air valve main door was not fully closed, [causing] an avionics vent fault.

Then Mechanic Y and I did more research involving Aircraft Maintenance Manual (AMM) 21-26-00 and Task 21-26-00-040-002 that made it very clear on position required of main/auxiliary [outlet valve] flaps and Deactivation Procedure. We re-accomplished Deactivation Procedure in accordance with MEL and AMM 21-26-00 and Task 21-26-00-040-002, re-accomplished the Avionics Equipment Ventilation Computer (AEVC) Test and paperwork. Suggest better labeling of figures/drawings and procedures in MEL 21-26-04 and more classroom/hands-on training.

Narrative: 2

An A320 aircraft returned to gate after pushback due to avionics vent fault. I went to determine what was wrong with it and found avionics vent skin air outlet valve fault. Contacted Maintenance Control and we agreed to defer the vent skin air outlet valve using MEL 21-26-04. Mechanic X came to help me. He accomplished the MEL maintenance procedure and I did the Avionics Equipment Ventilation Computer (AEVC) Test and signed the paperwork.

Synopsis

Two Technicians report about misinterpreting an MEL 21-26-04 Maintenance procedure for deferring an A320 aircraft's skin air outlet valve in the closed position. Inadequate MEL Maintenance illustrations lead them to believe the skin valve's main door, not the smaller auxiliary door, should be left partially opened.

ACN: 988141 (2276 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : PSP.Tower
State Reference : CA

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : PSP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 988141
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Out of 12,000 FT in descent into PSP we were queried by ATC if we had the airport in sight. We were just coming up upon abeam the airport and eventually were cleared for the visual to Runway 31L. I told the approach controller that we would be making a longer downwind to lose altitude plus both myself and the Captain had never been to this airport before so we wanted to play it cautiously. We planned on flying all the fixes on the approach but once we were comfortable with our altitude and space we turned towards the airport for the approach. We were probably 15 to 20 miles out with the airport environment in sight. We had the beacon, as well as some airport lighting in sight and were following the published approach course for the VOR-B approach to 31L. At least 10 miles out we had what we believed to be the runway and the VASI in sight. We configured prior to the final approach fix and descended along the VOR-B course. Never once did we see a second set of runway lights or anything to make us believe this was the wrong runway. I did get the feeling that the runway looked small but thought it was just an optical illusion born of the terrain. We descended below MDA (2,350 FT AGL with airport elev. of 477 FT) on the approach course to fly visually to the runway. At somewhere below 1,000 FT AGL I still had the feeling that something didn't look right. The runway lights were very bright and we were right on the VASI. Somewhere in this area I realized that we were lined up on the wrong runway as I started noticing the lights for the other runway. The lights for RWY 31L were much farther spaced out and definitely seemed to be at a much lower intensity. I immediately stated to the Captain that we were lined up for 31R and we needed to go around. We performed the go-around and returned to land runway 31L without further incident. No other traffic was evident and no special handling was needed. Neither the Captain nor myself had ever previously been to PSP. The key for our error was the fact that 10 to 15 or so miles out we had the runway lights and the VASI in sight for one runway on the airport. We made the incorrect assumption that that was for our runway. Unfortunately they were for Runway 31R. We knew there were 2 runways that were close together parallel from our briefing but assumed since the lighting was so bright it must have been the correct runway. Upon our return to the airport after the go-around we guessed that the runway lights for 31R were probably at Medium to High and the runway lights for 31L were probably at low. The lights for 31R were also closer together and appeared to be newer (white) while 31L appeared to older (yellow). We did find it rather odd after the fact that 31R was so well lit while 31L was definitely not. What we should have done is found both runways upon getting closer to the airport to guarantee that we were lined up with the correct runway. As there is no ILS or LOC for lateral guidance, we discussed possibly extending the center line on the runway in the MCDU to better orientate the offset center line of the correct runway. Also, the First Officer side MCDU had gone blank in the descent and was inop causing another distraction. As we discussed, extending the center line on the runway on the MCDU was an option we could have taken advantage of. The reality is, we just need to make sure we can verify that we are lined up with the correct runway since there is truly no lateral guidance available here. In this case we should have made sure we had both runways in sight so we could verify that we were headed to the correct runway. I do feel we made the correct go around call rather then forcing a possible unstable approach.

Synopsis

A320 First Officer determines late in a visual approach to Runway 31L at PSP that they are lined up with Runway 31R and calls for a go around. The lights for Runway 31R were far brighter and whiter than those for Runway 31L.

ACN: 986836 (2277 of 2908)

Time / Day

Date : 201201
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 35000

Environment

Weather Elements / Visibility : Turbulence
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZDV

Component : 1

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : MCP
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14100
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2572
ASRS Report Number.Accession Number : 986836
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Early in the Cruise Phase, First Officer was pilot flying. It was brought to my attention by the First Officer that his ND was displaying "Map Not Available" in red color; no map was displayed on the number 2 ND. Seconds later a level 2 ECAM (single chime) with the message of CAB PR LDG ELEV FAULT, which was then accompanied by another fault, the lower ECAM GW display was amber XX LBS. As the pilot not flying I began to troubleshoot what seemed to be the most critical aspect of the multiple failures. At the time it seemed that the Cabin Pressurization was the most critical, not to mention, it was that problem that triggered the Level 2 caution. Once I determined that the integrity of the pressurization system was not compromised, I moved on to the other issue, the lack of the map display on the First Officer side. It seemed that the number 2 FMGC had failed, however, the PFD indications were consistent with no failures. FMA's displayed MACH ALT CRZ NAV all in green, no mode reversions had occurred. Even the FD mode was 1FD2. Yet, as I scanned over to my display, I realized the aircraft failed to make a turn upon passing the previous fix so I immediately went to HEADING Mode turning the aircraft back to re-intercept the course and engage AP1. The deviation was 6.1 NM left of course. At that moment, ATC had queried our position and asked what our next FIX was; I told ATC what our next fix was. I then requested DIRECT, and was cleared direct. This lateral deviation would have been avoided had the aircraft been equipped with a lateral deviation alert similar to altitude deviation alerts that most aircraft are equipped with. Although flying the airplane is the priority at all times, it was very deceiving to the pilots when the aircraft indicated that it is flying based on what the PFD FMA's were displaying. As a former instructor on this airplane, I always used to tell my students to believe what the FMA displays. In this case, it lied about its NAV capability, even though it displayed a normal green NAV display, it wasn't. There must be a communication link between the ND and the PFD.

Callback: 1

The Reporter stated that the disparity between the green FMA status indications and the FMGC failure to correctly guide the autopilot could not be reconciled. The FMGC faults should have triggered a transition to the default heading mode along with an appropriate FMA change. The only indication the crew saw, other than the ECAM, was the ND2's MAP display fault and on further investigation both FMGC's displayed amber INDEPENDENT OPERATION in small letters at the bottom of the screen. The autopilot was selected to number one and functioned normally to landing. All systems were operating normally even with the FMGC anomalies. After landing the systems reset and operated normally.

Synopsis

An A320 number 2 ND MAP failed and the autopilot did not track correctly causing a track deviation although the FMA indications remained green and the FMGCs entered independent operations.

ACN: 986695 (2278 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : FCC (Flight Control Computer)
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 986695

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Preflight was normal. On the Takeoff roll at about 60 KTS we got the F/CTL SPOILER FAULT ECAM. The Takeoff was aborted and we cleared the runway. We did the reset in the FM and the ECAM was cleared. We coordinated with Dispatch and Maintenance Control, got a new release, topped off the fuel and departed. The same crew flew this aircraft the previous day. We had the same ECAM on engine start, the FM reset didn't help. We were delayed for 4 or 5 hours while Maintenance got a SEC and an actuator transducer flown in from another station. The SEC that arrived was a bad part and couldn't be used. The transducer was put in the cargo hold. Maintenance re-racked the first SEC, the fault was cleared and the aircraft released. The aircraft then spent that night, apparently with no further work done.

Synopsis

A320 Captain experiences F/CTL SPOILER FAULT ECAM during takeoff and rejects. The same fault had occurred the previous day and was "repaired" by re-racking the faulty SEC [Spoiler Elevator Computer]. The replacement SEC was faulty and no further attempt was made to obtain a serviceable part.

ACN: 986611 (2279 of 2908)

Time / Day

Date : 201112

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Elevator Trim System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 986611

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 275
Experience.Flight Crew.Type : 8100
ASRS Report Number.Accession Number : 986620

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After normal engine start and taxi out the First Officer attempted to set trim for takeoff. The trim wheel had an abnormal amount of resistance and binding. Parked in pad and consulted with Maintenance Control then returned to the gate for a maintenance inspection.

Narrative: 2

Engine start, taxi out normal. Upon receiving final weights and adjusting final trim setting, I encountered very strong resistance to moving the trim wheel. We pulled over, stopped the aircraft and the Captain confirmed my findings. In both directions, the trim wheel was extremely hard to move. After conferring with maintenance, we returned to the gate for further investigation. Flight was canceled.

Synopsis

An A320 flight crew returned to the gate for maintenance when attempts to set the takeoff trim resulted in abnormal resistance and binding.

ACN: 986395 (2280 of 2908)

Time / Day

Date : 201112

Place

Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 986395

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Airport

Narrative: 1

We had been cleared to leave our spot and to taxi to our gate. Had I not moved off of our spot well south of the lead-in line an air carrier B737 taxiing out would have hit our right wing. This ramp at night is an accident waiting to happen. I don't know if the B737 was on the wrong line or whether we should have been given a more definitive clearance, but it was uncomfortably close. I recommend that no aircraft be cleared into our gates at night with another airplane taxiing out for take off.

Synopsis

An A320 Captain described a ground conflict with a B737 exiting the ramp area as he was beginning his taxi into the gate area from a holding location.

ACN: 986158 (2281 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZMP.ARTCC
State Reference : MN
Altitude.MSL.Single Value : 34000

Aircraft

Reference : X
ATC / Advisory.Center : ZMP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZMP

Person

Reference : 1
Location Of Person.Facility : ZMP.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 986158
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Aircraft
Primary Problem : Ambiguous

Narrative: 1

Aircraft X was on a K fix routing and I issued an alternate K fix for traffic. It took about 45 seconds of trying to understand and read back the clearance to get it right and then 30 plus seconds later Aircraft X advised me that that fix was not in his database. I then issued a turn and proceeded to get separation. The fixes should be thrown out, they are unsafe and confusing.

Synopsis

ZMP Enroute Controller reported difficulty in issuing a revised "K" fix. When coordination was completed, the aircraft did not have the fix in the database.

ACN: 986079 (2282 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : CLT.Tower
State Reference : NC
Altitude.MSL.Single Value : 160

Environment

Flight Conditions : VMC
Light : Night
Ceiling : CLR

Aircraft : 1

Reference : X
ATC / Advisory.Tower : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Initial Approach
Airspace.Class B : CLT

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Landing
Airspace.Class B : CLT

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 986079

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

We were extensively vectored [on our early morning arrival to CLT] by Atlanta Center (up to 90 degrees off course) due to a 30 mile in-trail restriction, even though it was a severe clear morning. ATIS indicated only Runway 18C was open. I had briefed for that approach. I also stated in my briefing that due to the time of the morning (circadian rhythm), our weight and high landing speed (Vref of 148 or 149 KIAS for company preferred flaps three arrivals), if they opened up Runway 23 and assigned it to us, we would NOT accept that runway for our arrival. Note: normally, this runway is listed in the ATIS for large four engine and turbojet aircraft for noise abatement before 0630 or 0700 local. However, this runway is by far the shortest of the four runways with a severe downhill gradient. But that morning it was not being advertised on the ATIS. As soon as we were handed off to Charlotte Approach, we were told to expect a visual to Runway 23. The First Officer politely declined the runway assignment. We were then told to expect Runway 18R. We ended up being vectored behind a company flight, another Airbus. I found out later in the hotel van that they were vectored behind a Dash 8. We were told to maintain 4,000 FT and vectored behind company flight on final for 18R. Our vector would put us right over the FAVA point. But we had slowed to 180 KIAS so at that point things were good. 

We were told we would be following a company Airbus three miles ahead. By this time we were fully configured for a flaps three landing. At the 1,000 FT auto call I noted we were only 2 1/2 miles behind company according to my TCAS readout on my ND. And we were just inside 2 1/2 miles at the time [company] apparently touched down on 18R. But he made the first turn-off so the runway was clear well before we got there. I had loaded the 200 FT decision altitude in the MDA field of the PERF page so we got the "hundred above" and "minimums" auto callouts. A second or two after the "minimums" call-out the aircraft rolled to the left about 15-20 degrees. I countered with about half right stick and the aircraft started to recover when, all of a sudden, the aircraft rolled fast to a steeper bank to the right. I countered with full left stick yet the aircraft continued top roll right. I had the stick hard against the stop yet it continued to roll right. I was pretty sure we were in a wake turbulence encounter and being so close to the ground I simultaneously pulled the nose up to fifteen degrees, pushed the thrust levers to TOGA and called out "Go-around, TOGA..... NAV." The airplane responded with a good climb, the aircraft stopped rolling as we pitched up and it followed my stick inputs to level the wings and we completed the initial phase of the go-around procedure. 

As we finished that at about 500 FT AGL, and before the First Officer had a chance to call the go-around to the Tower, we got a quick call from the Tower saying to turn right to heading 270 degrees for traffic departing Runway 23. We simultaneously received a TCAS Traffic Advisory. I was head inside starting the turn and saw the TA target was not far; However, I cannot recall the altitude difference. I took a quick look outside after getting the heading set at 270, but never saw the other aircraft. We completed the normal go-around procedure and cleaned up to flaps one as we were given an immediate turn to the downwind for Runway 18R. After we landed, we were given a phone number to call by the Ground Controller. I made the call once inside the terminal. I didn't understand the name or title of the person who answered, but it sounded like a supervisor in the TRACON and not the Tower. He basically wanted to know if they had done anything to cause the go-around. At that time, being a bit tired, I said "no." But reflecting back on the event, I'm not so sure. Perhaps we were vectored in too close behind company. We were definitely overtaking him for two reasons: 1) his choice of flaps full instead of flaps three as we had done and, 2) his need to slow down behind the Dash 8. 

I did tell him about our TA with the aircraft departing Runway 23, as I wasn't sure how close we were to him. The man on the other end of the call said we weren't close at all as the Controller on 23 saw us initiate the go around. That Controller immediately called it out to the [departing aircraft] crew and instructed them to turn to a heading of 180, which he said they did right away. He said we weren't close at all.

Synopsis

An A320 Captain reported encountering wake vortex from preceding A320 on approach to CLT, resulting in a strong right roll. A go around was executed and a safe landing followed.

ACN: 986077 (2283 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 986077
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 986078

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During pushback/engine start number 2 engine started normally. Around the same time that the number 1 generator power came online we received an ECAM ENG 2 FAILURE. I noted that engine 2 N2 was dropping through 25%, so we did ECAM, called Company and Maintenance and returned to the gate. I was recently made aware of an industry incident causing electrical power failure of all the flight instruments due to power transfer failure from the number 2 engine. I wanted to verify that number 2 generator would transfer, so I elected to use the supplemental normal for starting number 2 then number 1 generator prior to taxi. I do not see any connection between starting number 2 generator first then number 1 and an engine 2 failure. I do not know why the engine shutdown. I recognize the importance of SOP's and Supplemental Normals and following the procedures. Starting engine number 2 first may not have been the best thing to do, but I do not see how it can cause an uncommanded engine 2 shutdown. I will not use this supplemental/normal again unless absolutely necessary for an external start.

Narrative: 2

Captain said another aircraft experienced total EFIS NAV SYS DISPLAY FAILURE when electrical relays failed to pickup a load when another system failed. He wanted to make sure our relays worked. As described in Supplemental Normals with the engine starting sequence (starting number 2 first, then number 1) was accomplished. Engine 2 started without incident. Engine 1 started without incident, however it appeared that when IDG number 1 connected to the system, FADAC was not happy and performed an AUTO SHUT DOWN on Engine number 2, ECAM was annunciated ENG FAIL RIGHT and the N1/N2 began to spool down. The engine was shutting down. ECAM Action included moving Engine Master 2 to the off position. We were only a few feet from the gate and coordinated with Operations, Ramp, and Maintenance on bringing the plane back to the gate.

I'm not sure what the bus logic is here, but it's clear that something failed and that the FADAC sent a shutdown signal to the engine. It should not matter which engine is started first; engine 2 failed after adding engine 1 on line. Check all Airbus to see if this is a common problem. In the "old days" we alternated engine starting sequence to check the relays. Maybe we should do this on the first flight of the day since we have to start both engines after push for a 5 minute warm up anyway.

Synopsis

A320 flight crew starts engine number two during push back then starts engine number one. When generator number one comes on the line engine number two shuts down with the ECAM indicating ENG 2 FAILURE. Crew returns to the gate.

ACN: 985930 (2284 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Pax Seat
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Repairman
ASRS Report Number.Accession Number : 985930
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Primary Problem : Human Factors

Narrative: 1

At the shift beginning, I was assigned to an A320 aircraft to repair a broken spreader bar at seat 24 D/E. After waiting for gate calls, I proceeded to pick up my pre-drawn parts, then on to aircraft. Everything proceeded as planed until I went to check the security of the opposite side of the seat back pivot, [where] there were parts missing and the pivot bolts were broken and not secured to the pivot. I ran into trouble in gaining access to the area to do the repair. I called my Lead Mechanic, Mr.X, to get some assistance at about XA:30 am in the morning. We were able to remove the armrest shroud and pivot bolt at about XA:45 am. At that point parts needed to be ordered. They took 45-minutes to be delivered and turned out to be the wrong parts. At that point my Supervisor arrived at the aircraft and attempted to find the proper knuckle. 

We proceeded with a repair of the bad knuckle (nutplate). This brought us to about XB:45 am. I put everything together in about 10 minutes and proceeded in a hurry to clean up and on to get punched out by XC:30 am. As I was walking out the door of the hanger, it hit me that I had not installed two mounting nuts and bolts that secure the knuckle to the isle spreader bar. I immediately called my Supervisor and informed him of my mistake. I kept in touch with him until the repair was done. 

My best assessment was that I was in a hurried situation and overlooked the securing bolts. I knew that I had not installed the bolt at the time and became overwhelmed with trying to tie up all of the loose ends. I was not in the area, however my Supervisor and the dayshift Lead, took charge of the repair. I need to learn to work situations with a little more calmness rather than trying to beat the clock.

Synopsis

Mechanic reports walking out of company Hangar and realizing he had not installed two mounting nuts and bolts that secure a nutplate to a cabin seat row aisle spreader bar on an A320 aircraft at a terminal gate.

ACN: 985747 (2285 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZID.ARTCC
State Reference : IN
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.Center : ZID
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZID

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 985747
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

At cruise the Captain's MCDU failed and the First Officer's MCDU was intermittent. The decision was made after a conversation with Maintenance Control and Dispatch to return to departure airport. During the conversation with Maintenance and Dispatch both parties kept telling the crew that we could reset Circuit Breakers (C/Bs) in flight. We felt this was a clear change to our new joint operations policy. After telling Maintenance we could not reset a C/B in flight they proceeded to tell us we could and that we just did not want to. After a long discussion we found that there was bold guidance in the flight manual that states specifically pilots are NOT allowed to reset a C/B in flight. Dispatch then suggested the Captain use emergency authority to reset the C/B at which time an emergency did not exist. The Captain and I declined the suggestion. The flight returned to IAD uneventfully.

Synopsis

An A320 flight crew refused to reset a popped circuit breaker in flight as directed by Maintenance Control; citing a specific bold print prohibition against doing so in the newly developed joint FOM for the two recently merged air carriers.

ACN: 985649 (2286 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0601-1200

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 985649
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

Almost hit by city snow plows due to out of control driving. We were holding short of Gate due to a high pile of snow that blocked our entry. Two city snow plows arrived at high speed to remove the snow. The first one barreled in from our left (the east), and jack-knifed after hitting the snow sliding out of control for about 30 FT. It appeared that he was close to hitting the wing-walker/marshaller on our forward left side. The second snowplow barreled in from our right side (the west) and hit the same pile of snow after the other plow backed away. He also jack-knifed his plow, spinning about 270 degrees within 10 FT of the aircraft. I thought he was going to hit the aircraft. This was unprofessional, dangerous and completely unacceptable. I called operations to capture the event on the Gate camera and a picture is worth a thousand words. These drivers should not be working on the flight line; careless disregard for the safety of others.

Synopsis

A320 Captain describes apparently reckless snow plow driving by two drivers attempting to move a pile of snow blocking access to the gate.

ACN: 985326 (2287 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZMA.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZMA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZMA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 985326
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

This was a red eye to FLL. It was dark outside and the radio was pretty quiet with very sporadic transmissions. As we approached FLL we checked in with Miami Center and they acknowledged us checking in at FL350. Approximately 230 miles from FLL while at FL350 we got a call from Miami Center asking us to descend to FL200. I answered the call with "Descend and maintain Flight level Two Zero Zero, Air Carrier XXX" The First Officer, who was the flying pilot, set 20,000 in the ALT window and started a gradual descent. At the time I thought we were still a little too far from FLL to start our descent but because of traffic going into Miami I have had them issue this early descent in the past so I thought nothing of it. As we descended through FL260 Miami Center issues a clearance for us to descend at pilot's discretion to FL240. I tried to figure out what was going on and after a few seconds the Miami Center Controller re-issued the clearance for us to descend at pilot's discretion to FL240. I answered the Controller by saying that I was trying to figure out the last clearance that had already been issued us a previous clearance for us to descend and maintain FL200. The Controller then informed me that they were manning two frequencies and that there was another aircraft with a similar callsign and same XXX flight number on the Controller's other frequency. This was the first time we had heard anything regarding the other aircraft with similar callsign and same flight number. The Controller then said for us to be aware of the other aircraft on the other frequency with a similar callsign and same XXX flight number and re-issued the clearance for us to descend at pilots discretion to FL240. I read back the clearance and continued descending from FL260 to FL240. The Controller then advised the other aircraft of our similar callsign and same XXX flight number. The radio was very quiet but we never heard the other aircraft check in so I have to assume that they had checked in with Miami Center before we did. There were no TCAS alerts or other indication that there had been a conflict with another aircraft but the fact remains that we descended 9,000 before the Controller issued the clearance for us to descend at pilot's discretion. Nothing else was said by the Controller. Cause, two aircraft with similar callsigns and exact same flight number in the same sector being handled by the same Controller in two different frequencies and neither one of the aircraft was aware of the other. Neither aircraft had the chance to catch the error by saying "I think somebody else took our clearance". The Controller should have also heard two read backs as he/she was manning both frequencies. Suggestions, by simply making sure each aircraft were aware of each other and for the Controller to issue very clear instructions of which aircraft needs to do what.

Synopsis

Air Carrier on frequency with ZMA described callsign confusion when the Controller was working two frequencies with two aircraft with identical flight numbers, no advisory was issued by ATC.

ACN: 985303 (2288 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23200
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 985303
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Other

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After clearing runway we observed an avionics smoke ECAM. Although no smoke was detected in the cockpit the Purser was called and he said that he thought there might be a sent of smoke in the forward galley. I immediately taxied to gate while the First Officer called Maintenance to have them come to the aircraft to check the avionics compartment (a message was sent as well). The marshaller parked us just past the stop line (within a couple of inches). The CSR refused to bring the jetway up to the aircraft although I opened my window and told him to bring the jetway up anyway as we might have smoke (possibly fire) and we needed to deplane the passengers and allow myself or the First Officer to immediately inspect the avionics compartment as Maintenance was no where in sight. Although we were past the stop line the jetway could have been brought up and the door opened. Meanwhile the marshaller had disconnected and I could not talk to him. At this point I opened door 1L myself and had to jump the 2 and a half feet to the jetway so that I could run down to the ramp and open the avionics compartment door myself to check for smoke as again Maintenance was not there. No smoke was observed. I nearly had to call for the Fire Department to get any response from anyone. It seems that there is some sort of turf war going on between Customer Service and Ramp as to how the aircraft are parked. The agents are unable or unwilling to use any discretion or even common sense when it comes to parking an aircraft as this is the third time in as many trips to this airport that we can't seem to park the aircraft without stopping, hooking up, and backing up as little as 3 inches before the CSR will bring the jetway up. The problem here is that the passengers are already out of their seats and we have to back up with them standing, thus exposing them to injury. This airport is the only place where I have had this problem.

Synopsis

A320 Captain experiences an avionics smoke ECAM after landing, although no smoke is detected by sight or smell, and taxis to the gate expeditiously. When the aircraft is parked at the gate the nose tire is several inches off the mark and the jetway operator will not connect to the aircraft until it is corrected. The Captain jumps the gap to physically inspect the avionics compartment. No smoke.

ACN: 985294 (2289 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 9000

Environment

Weather Elements / Visibility.Other 
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 2100
ASRS Report Number.Accession Number : 985294

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 400
Experience.Flight Crew.Type : 5861
ASRS Report Number.Accession Number : 985289

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On approach when flaps extended to position 2 we got an ECAM F/CTL Flap fault. The ECAM called for cycling the flap handle in an attempt to reset the system so we asked for and received increased assigned speed from ATC. It became apparent that we would need to break off the approach and work the problem. We did this. I assigned the First Officer to fly and work the radios while I worked the problem. 

I followed the ECAM and retracted the flaps. On re-extension the problem recurred. I used the irregular section of the Flight Manual which directed a Flap 3 handle position for landing and review of Maneuvers section Abnormal Slat/Flap Approach. Additionally we were to apply landing distance calculation procedure. I determined that we had more than adequate available landing distance with 16R, weather was not a factor and we had about one hour of fuel remaining. I spoke to the purser and told her what the problem was and that we would need another 15 minutes or so to complete checklists. I also said we did not need to prepare for an evacuation. I made a PA announcement to inform the passengers of our problem and that they could expect a normal landing after our checklists were complete. 

We reviewed our problem with Dispatch and Maintenance Control and determined there was no fix we could attempt. During this time the First Officer suggested declaring an emergency. I agreed, and we did so. The First Officer and I briefed each other on what each had been doing and we then got clearance back to the airport. I took back the airplane on final approach and an uneventful landing followed.

Narrative: 2

I was flying the approach at 9,000 FT MSL and 210 KIAS with flaps 1 selected. ATC cleared us for the visual approach. ATC said to slow to 170 KIAS. I selected 170 KIAS, disconnected the autopilot and at 190 KIAS requested flaps 2. Flaps 2 was selected which resulted in the F/CTL Flaps Fault ECAM. At this point we were in a slight descent as we were capturing the glide slope. I continued to fly the airplane as the Captain began to execute the ECAM. As we needed to cycle the flaps we told Tower that we needed to break off the approach. We climbed back to 9,000 FT MSL and were given a turn towards the north. The Captain selected 200 KIAS and cycled the flaps with the same result - the ECAM returned. At this point my attention was solely on flying the airplane, talking with ATC and keeping the Captain apprised of our fuel state while he worked the problem. While working through the problem the flaps were cycled twice to no avail. About this time I told ATC that we were declaring an emergency and provided them with fuel and souls on board. I also requested the longest runway and explained our problem and that we would be landing at a much higher speed then normal. I also said that we would probably have no problem clearing the runway after landing. 

After approximately 15 minutes the Captain briefed me on the approach and landing that we would need to perform as he had reviewed the abnormal checklist for Flap Fault, the Maneuvers section of the Flight Manual for the approach profile and flap and speed settings and had calculated the runway required for the speed and configuration flown. As we had the fuel for a little more flight time the Captain contacted [Maintenance] to see if they had any additional help to provide. I pointed out that we were at 4,200 LBS of fuel and 45 miles from the airport at which time we decided that at 4,000 LBS we would fly the approach. I told ATC that we were ready for the approach, they gave us a heading of 140 to intercept the LOC and I turned control over to the Captain who flew the approach and landing. Auto brakes Medium were selected flaps 3 selected on the Performance page and a reference speed of 157 entered. The approach looked fairly normal with the touchdown about the 2,000 FT marker with a normal roll out. As we taxied to gate we left the flap handle in the flap 3 detent for maintenance. I also made a PA reminding everyone to remain seated until at the gate.

Synopsis

Following receipt of a F/CTL flap fault ECAM, an A320 flight crew executed a go around, performed appropriate checklists and notifications, declared an emergency and landed without incident.

ACN: 985111 (2290 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZMA.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 10500

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZMA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class E : ZMA

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class E : ZMA

Person : 1

Reference : 1
Location Of Person.Facility : ZMA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 985111
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Facility : ZMA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 985611
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

[There is a] recurring safety problem for Miami Center controllers, especially in the Bimini/Nassau area, [in which targets are not displayed] in the HOST system. However, when you switch to EBUS, the target is displayed perfectly. In this situation an A320 was making his descent into FLL, he advised that he is responding to a TCAS RA. The R42 Controller had no display of any target; however, when switched to EBUS as the Controller had awareness of this HOST display problem, the target was observed perfectly, but now being the A320 aircraft. Pilot called in later and advised it was very, very close! This [is] a known, very serious safety issue that has been previously reported through various venues. It is a very serious problem in a highly congested air traffic area that could be rectified with the installment of Freeport RADAR and the upgrade of Nassau RADAR, as it is very old system with questionable maintenance problems. Recommendation: installment of Freeport RADAR, upgrade of Nassau RADAR, and adaptation upgrade so that HOST can display the same targets as EBUS with the same back-ups and redundancy. In other words, the RADAR data is available, but HOST is not displaying it as efficiently as EBUS.

Synopsis

ZMA Controllers voiced concern regarding two RADAR systems that fail to provide needed RADAR data to controllers.

ACN: 984546 (2291 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5920
ASRS Report Number.Accession Number : 984546

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 17500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5800
ASRS Report Number.Accession Number : 984547

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Just prior to top of descent, we received an ECAM message HYD G Res Low Level. First Officer completed the ECAM items and we reviewed the Flight Manual for any additional considerations. While the First Officer monitored the airplane and ATC, I established a phone patch with Dispatch. We informed them of the issue and advised that we would need a tow-in from the runway. I then briefed the flight attendants on the interphone of the situation and emphasized that cabin prep would not be required. After informing the passengers of the issue and assuring them of a normal landing, the First Officer and I had time to discuss all the aspects of the approach and landing. When we switched to Approach, we declared an emergency and advised that we would need the tow-in and would most likely be stopping on the runway. The weather was VFR and we completed the visual approach with no complications.

The landing gear was extended using the Manual Gear Extension procedure with no problems although the gear doors remained open and increased the noise levels in both the cockpit and First Class. As we approached taxi speed on the rollout, I felt I had sufficient control to safely exit on the high speed turnoff. We stopped between the runways and shut down to await the tow-in. The airport authority safety personnel arrived with our Maintenance crew and would not allow us to cross the runway until the hydraulic leak was stopped. That took about 10 minutes after the reservoir was depressurized. The tow-in was completed without incident. First Officer was extremely helpful and efficient in providing input and suggestions and proved to be an invaluable resource through out the incident; his calm professionalism and ability to anticipate what was needed, made handling this event easy. He is one of the finest First Officers I have flown with in my 12 years as Captain.

Narrative: 2

[Narrative 2 had no additional information.]

Synopsis

An A320 flight crew received an ECAM message HYD G RES LOW LEVEL. They ran the procedure, declared an emergency, and landed safely.

ACN: 984117 (2292 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 14500

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Airspace.Class E : LAX

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 984117
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 984118
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Crossed KONZL at 17,000 FT. ATC cleared us to descend via the BASET 3 Arrival and expect ILS 7R, we had briefed and programmed ILS 6L in FMS. I was pilot flying and dialed in 10,000 FT in the altitude window and pushed altitude, and then changed the runway to 7R in FMS. We were briefing the new runway when ATC said to maintain 14,000 FT as we missed our crossing restriction at TRTLE and he said he didn't want us to go below his airspace. We were at 14,500 FT at that time. No other aircraft were within 15 miles of us. We continued our descent to 14,000 FT and that was the end of the altitude deviation. I think the crossing restriction dropped out when I changed runways or went into open descent when the new runway was inserted and I didn't notice it. Then we were briefing the new approach we had just been assigned when we flew through the altitude. Task saturation and unfamiliarity with landing on the 6's and 7's in LAX as we almost always land on 24's and 25's contributed to this deviation.

Narrative: 2

While descending into LAX airport we were switched to the Baset 3 arrival for 07R instead the Seaview 2 arrival as per our flight plan, a clearance to cross KONZL at FL170 was also issued which we complied with uneventfully. After passing KONZL at FL170 we were cleared to descend via the Baset 3 arrival but another runway change was issued for 06L at the same time. We switched the arrival in the MCDU and commenced our descent as per the arrival. After switching the runways we reviewed the altitude/speed restrictions and briefed the new approach, but the TRTLE restriction to cross at 16,000 got deleted and there are only 6 miles from KONZL to TRTLE and the descent already started, we were busy re-briefing and rechecking the restrictions and the Aircraft descended to 14,700 FT, the ATC Controller told us to level off at 14,000 to remain in his airspace, that we missed the restriction. The ND and MCDU did not show the restriction, it was deleted during the change. There was no loss of separation with other traffic and we proceeded to LAX without any further comment from ATC.

The event occurred due to several factors: 
1) Several arrival and runway assignment changes during the busy phase of descent. 
2) Software feature on MCDU sometimes drops altitude and speed restrictions form certain fixes when runway or arrival is changed in the MCDU. Sometimes the arrival is selected and all the altitude/speed restrictions are missing and have to be manually entered. 
3) We were distracted by the change of arrivals and runways and all the tasks that came along with that, like reprogramming and re-briefing the new procedure. 
4) Time constraints were also an issue since we wanted to get the change done properly and promptly to review the restrictions since we were already on the descent phase, and TRTLE was very close to KONZL (only 6 miles away)

1) Correct the MCDU software to avoid dropping altitude and speed restrictions when the present procedure is changed. Maybe develop a warning message to make pilots aware. 
2) Make ATC aware on how these many runway and arrival changes impact our operational safety, especially when issued during high workload phases like descent. These changes are very common in LAX but they are usually given earlier with more time to prepare, at a phase were pilots are not that busy. 
3) Pilots should delay all briefings and supplemental duties until all the restrictions are verified are correct and the aircraft is a position to meet them safely.

Synopsis

A320 flight crew reports missing the crossing restriction at TRTLE during descent on the BASET 3 to LAX. The aircraft went into open descent while entering an ATC assigned runway change into the FMGC and was not detected by the flight crew.

ACN: 983849 (2293 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 983849
Human Factors : Physiological - Other
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Detector.Person : Flight Attendant
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

The APU was on the MEL. We started engine #1 at the gate with an air-start machine. We pushed back and cross-bleed started the 2nd engine. As we began the taxi, while still on the ramp, the 'A' Flight Attendant called us and said there was a bad odor in the cabin and many passengers were complaining. I told her it was probably due to the air start and or cross-bleed start and it would probably clear up. I told here to call me back in about 5 minutes and tell me if it was any better. She called back about 5 minutes later and said it was no better, perhaps worse. We called Ground Control and asked them if we could pull into the cargo ramp, and we did. I turned the airplane into the wind, so as to eliminate any engine exhaust from blowing back into the airplane. I bumped the power up slightly (25% N1) above idle to help with the air flow. I selected "HI" flow for our packs and we sat there for a few more minutes to see if it got better. I sent the First Officer back to see what he thought. He came back a few minutes later and said he certainly smelled it and it smelled like a faint electrical smell to him. Passengers and flight attendants reported it as a burning plastic or musty smell. I too, thought it smelled musty. After a few minutes, we all decided it wouldn't be a good idea to fly this airplane. We returned to the gate and I radioed for a Customer Service and Maintenance Supervisor to meet the airplane. They did and all agreed there was a bad, unidentifiable odor in the airplane. One female passenger complained of burning eyes. We transferred to another airplane and completed our flight.

How about an air quality monitor in the cabin? This is costing the airlines millions and, in some cases, causing harm to crew members and passengers. Most of these odor problems are probably nothing, but how do we know? We have to keep safety in mind. If there's any doubt, we don't go! There are lots of odors on an airplane. Most of which are probably harmless. We have carbon monoxide detectors in our homes. Can't we get something to monitor the air in our cabins? It would probably benefit the airlines to do this; we would probably have a lot less "Toxic Fume" incidents if the crew knew that what they smelled wasn't going to harm them!

Callback: 1

The reporter stated that he included the various smell descriptions as his crew reported them to him, but when he actually went to the cabin himself after returning to the gate he would describe the smell as "dirty socks." Thirteen passengers plus the entire crew acknowledged the smell. Several years ago he experienced the same smell and the entire crew had headaches after flying two short legs with the same aircraft before finally refusing to fly it further. The next day he was assigned that same aircraft at a different station and nothing had changed, so the aircraft "dirty socks" odor was not being treated seriously at that time. After this event, the reporter was told that mechanics are told to service the APU one half quart below full to preclude oil migration into the air conditioning system.

Synopsis

An A320 with an inoperative APU developed a dirty socks odor after a cross bleed engine start, so the aircraft returned to the gate where it was removed from service. Captain suggested that adding air quality monitor devices would help.

ACN: 983667 (2294 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 18000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : SCT

Component

Aircraft Component : Altimeter
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12280
Experience.Flight Crew.Last 90 Days : 157
Experience.Flight Crew.Type : 8200
ASRS Report Number.Accession Number : 983667
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9700
ASRS Report Number.Accession Number : 983843
Human Factors : Other / Unknown

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Incorrect altimeter setting entered into FCU (30.32) versus the correct setting (29.92) during descent on the LYNDI2 arrival into SAN descending through FL180. The Captain (pilot not flying) missed/provided incorrect setting during the descent checklist; and the pilot flying also did not double check to ensure the correct altimeter was set. In addition, during altitude changes assigned by ATC, the Captain set the altitude and pulled or pushed to initiate the descent without the concurrence or verification (finger pointing to the correct altitude) of the pilot flying. As a result of the incorrect altimeter setting in the FCU, several altitudes on the arrival were crossed below the minimum altitude restrictions. Probably the most notable human factor was the constant and continuous talking from the Captain during the descent phase which may have been a contributing factor to missing the correct altimeter setting.

Narrative: 2

During the arrival, the altimeters were set incorrectly. They were supposed to be set to 29.92, but were incorrectly set to 30.45. We missed the arrival altitudes by the corresponding amounts. We were asked by SOCAL Approach to say our altitude at the bottom of our descent to 5,000 FT, but weren't given a corrected altimeter by SOCAL. We were given a visual approach with a crossing restriction, and visually noticed we were low on the Localizer 27 approach. The error was discovered and addressed on visual final approach. A debrief of the event was conducted, and during it, we discovered a few of the errors that were made. ATIS was pulled during the enroute letdown, and I noticed it was 29.92, and made a mental note that there would be no actual change in altimeter as STD is set above 18,000. During the checklist, we were given a change in clearance to descend via the LYNDI arrival, just as the descent checklist was being completed, so we had new ATIS, new descent instructions being accomplished during checklist. The key error was the crosscheck, where I picked up the ATIS, thinking it was the one just off the printer, and read it, and set it, and reminded myself there was no altimeter change. Unfortunately, it was the departure ATIS from the trash pile. The checklist was completed, and probably the long day prevented the nagging feeling of a mistake, or the ignoring of the clues from SOCAL TRACON. Fortunately, we were in visual conditions, and were able to notice the abnormalities in the approach and correct them. I believe the 2,500 foot call, was the beginning of catching the error, although it was actually addressed a bit lower. Unfortunately, two tired pilots made some sloppy errors, and broke some habit patterns, then failed in the first few of the feedback loops. A final crosscheck, and good situational awareness, when it counted, kept this problem within the constraints of this report.

Synopsis

A320 flight crew reports missetting the altimeters descending through FL180, resulting in the aircraft being at least 400 FT low during all phases of the arrival and initial approach. The error is detected and corrected once established on the LOC.

ACN: 983634 (2295 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ
Altitude.MSL.Single Value : 3500

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Facility : P50.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 983634
Human Factors : Situational Awareness

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Was training a CPC-IT, weather had just moved in (again) and they decided to make this aircraft #1 to the airport. With the traffic coming behind, it was the correct decision. They issued an expedited descent to 4,000 (to keep him in the Bravo) but when turned to the base leg and issued 3,000, the aircraft continued descending expeditiously and descended momentarily out of the Bravo. By the time we caught that; we needed to turn the aircraft in to the final and were unable to issue the 'leaving/re-entering Bravo' phraseology.

Synopsis

P50 Controller reported ACR descending faster than expected resulting in an Bravo Airspace excursion.

ACN: 983505 (2296 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 983505
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Hangar / Base
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 983507
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party2 : Maintenance

Person : 3

Reference : 3
Location Of Person : Hangar / Base
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 983508
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

While accomplishing Door-1L (Left) and Door-1R (Right) slide RII Inspections, Door-1L slide was deployed. we completed Door-1L installation up to the bustle installation. We were still having Emergency Test issues, so the mechanics started the installation of Door-1R. We had problems with the Door-1R arming, so we were comparing the feel of the arming handle on both 1-Right and 1-Left. We resolved the issues with Door-1R and when the Mechanic was installing the bustle on Door 1-Left I heard the slide deploy. It was the end of shift and we were trying to complete the installation. We were under the assumption the slide was not the emergency light problem, so we wanted to complete the installation of the slides.

Narrative: 2

While installing Door-1Left (1L) door slide bustle, after slide reinstallation, bustle did not fit correctly. I lifted the door handle to open door to see if there were any obstructions and check for misfitting bustle, door slide blew, as it was left in the "armed" position.

Narrative: 3

After assisting in Door-1Left (1-L) slide installation, I started Door-1Right (1-R) slide installation, when I heard Door-1L slide deploy.

Synopsis

A Line Inspector and two mechanics describe events surrounding the inadvertent deployment of cabin Door-1Left escape slide on an A320 aircraft during an RII Inspection.

ACN: 983402 (2297 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDV.ARTCC
State Reference : CO
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZDV
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZDV

Component

Aircraft Component : Traffic Collision Avoidance System (TCAS)
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 983402

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12600
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 983454

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

RA with flight instruments indicating [TCAS commanding] over a 3,000 FPM descent. Clear of conflict at FL345; returned to FL360. ATC stated there was no aircraft within 100 NM of our flight. RA appeared again in a few minutes, confirmed again with ATC that no aircraft were near us. Sent a MRM code to Maintenance stating TCAS was unreliable.

Narrative: 2

This TCAS failure mode could have turned into a worse outcome if any actual aircraft were in our immediate area while executing or RA avoidance manuever.

Synopsis

An A320 flight crew reported a false alert from TCAS that commanded a 3,500 FPM descent from their cruise altitude of FL360. ATC reported no traffic within 100 NM.

ACN: 983330 (2298 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : LGA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 983330

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
ASRS Report Number.Accession Number : 983338

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

Controller crowded us with a 3 mile separation instead of 5 miles. [We experienced] moderate turbulence encounter with a 150 degree wind shift. Instructed to go around, over sped the flaps and hard to keep altitude and airspeed because of turbulence.

Narrative: 2

I could tell the Captain was having some trouble with the shear/turbulence as the aircraft began climbing again and our heading was drifting right instead of left. We then reached an altitude of 4,000 MSL. I tried to offer as much assistance I could without distracting the Captain further. ATC inquired about our altitude and I explained our situation and told him we were trying to descend and turn but had hit turbulence. During this time we over-sped the flaps twice by 10 KTS. We returned to our assigned heading and altitude and landed uneventfully. The flaps were written up and cleared by Maintenance.

Synopsis

An A320 reported overspeeding the flaps on a go-around event at LGA triggered by turbulence that may have been a result of close spacing.

ACN: 983272 (2299 of 2908)

Time / Day

Date : 201112
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 18000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZDC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : B737-300
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZDC

Person

Reference : 1
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 983272
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

An A320 was at FL200 with a B737 underneath at FL180. The B737 was on a vector to gain spacing. The B737 was then cleared to 17,000, and a few second later the A320 was cleared to FL180 with anticipation of the B737 aircraft leaving FL180 on that update. The next update showed the B737 leaving FL180 but at his altimeter was adjusted; it showed the aircraft back at FL180 on the next update. The A320 had a rapid descent rate, and I thought there was a Mode C error in the update so I didn't react quickly enough to stop the A320 before entering the altitude stratum of the B737. No recommendation, just a better understanding of the effect of an altimeter change when pressure is high, and a strict adherence to the rules with aircraft leaving and altitude before assigning it to another aircraft.

Synopsis

ZDC Controller described a loss of separation event when stepping down aircraft in close proximity, assigning an altitude immediately upon received a vacating notification.

ACN: 983136 (2300 of 2908)

Time / Day

Date : 201112

Place

Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 983136
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During the push back with the parking brake set, the push crew gave me the [salute] with lit wands prior to the tug pulling out from under the plane. This was a late evening departure after flying a transcon followed by a two hour sit. My First Officer and I were tired but alert and motivated to complete the flight. We had three wing walkers with lit wands including the Lead Marshaller. After the push was complete, I set the parking brake as instructed by the tug driver. The Marshaller gave the crossed wand signal. The tug driver disconnected his headset and said good-bye. I looked up and immediately got the lowered wands at a 45 degree angle from the Marshaller. I asked the copilot "Do you see the tug"; he said "No." We waited and then saw the tug pull out from under the plane all the while being given the cleared to taxi signal. The Marshaller gave a quick wave of the wand and ran to the tug to catch a ride. I never had a chance to flash the taxi light. We could easily have taxied right over the tug had we followed the Marshaller's instructions.

Synopsis

Despite receiving the "safe to taxi" sign from one Ground Crewman, an alert A320 flight crew refused to taxi until they personally viewed the pushback tug back out from under their nose and depart.