ACN: 976324 (2351 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Angle.Radial : 232
Relative Position.Distance.Nautical Miles : 12
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Night
Ceiling.Single Value : 10000
RVR.Single Value : 6000

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 976324

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were conducting the approach under visual conditions. Captain was flying pilot and [I] was pilot not flying. Current weather was light winds out of the southwest with greater than 10 miles visibility. While descending on the approach Captain stated he was not feeling well and gave me command of the aircraft. At that time he got up from his seat and was standing behind me in the cockpit. I assumed he had a very upset stomach. He told me to land the aircraft and that he had to go into the forward lav. At this time we were descending through approximately 1,500 FT on the approach. Since we were configured for the landing and were on profile I elected to continue rather than declare an emergency and complicate matters any more. The approach to landing was uneventful. Upon clearing the runway I opted to continue taxiing to the gate not knowing the extent of Captain's situation. I wanted to get the aircraft to the gate as soon as possible.

Synopsis

A320 First Officer reported the flying Captain became ill on visual approach and left the cockpit to go to the lav. First Officer landed the aircraft and taxied to the gate.

ACN: 975686 (2352 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : PHX.Tower
State Reference : AZ
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Tower : PHX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Initial Approach
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 975686

Events

Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

The First Officer was the pilot flying the visual approach for Runway 26. At approximately 1,100 FT AGL, having just finished scanning the airport and runway environment, I noticed in the corner of my eye a bright green light being emitted just over the nose of the aircraft from the ground. Since it was peculiar that a green light would be flashing in the darkness below, I looked at the green light directly and it filled my eyes with intensity. I immediately felt some pain in my left eye, followed by seeing dark spots in my vision. From the intensity of the light beam and the pain in my left eye, I knew immediately that my eye had been hit by a laser. I reported this to my First Officer and asked him if he had seen a green laser light. He answered that he did not. He asked me if my vision was impaired. I answered yes, but only in my left eye. I instructed him to continue the approach and landing. After landing, we taxied safely to the gate. We reported the incident to the Control Tower. They mentioned that they had two aircraft hit by a green laser the night before. The next day, I visited a doctor [and] he reported that I had two abrasions on my left eye. He prescribed an antibiotic and that my eye should heal up in about 2-3 days.

Synopsis

An A320 Captain reported he sustained abrasions on his eye after being targeted by a laser on approach to PHX.

ACN: 975516 (2353 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 975516
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At top of descent we got an ECAM message indicating a Yellow Hydraulic Reservoir overheat. We followed the ECAM/QRH procedures and then declared an emergency with ATC as it involved a hydraulic system that could compromise flight controls and reverser. Airport Rescue and Fire Fighters was asked to standby as a precaution. Company was contacted and apprised of the situation. The flight attendants were briefed. We continued to our destination and landed and taxied to our gate in a normal fashion with no other incidents. We were met at the gate by Maintenance and after briefing him I contacted the Dispatcher by phone and discussed what had occurred.

Synopsis

An A320 ECAM alerted HYD Y RSVR OVHT at the top of descent so the ECAM was complied with, an emergency declared and a normal landing at the destination followed.

ACN: 975375 (2354 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Flight Phase : Cruise
Flight Phase : Descent
Airspace.Class A : ZZZ
Airspace.Class B : ZZZ

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 975375
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 975373
Human Factors : Distraction
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Flight pushed six minutes late from gate. Flight was climbing to FL320. One of the Flight Attendants called to the flight deck and notified us of two issues. Issue number one CABIN PRESS annunciation on Door 2R was flashing. This indicates that 2R door handle is not in the full down and locked position. Issue number two, Door 2R would not remain armed. Door 2R would only show remained armed as long as the slide arming handle was held in armed position. As pilots, both of us were concerned that the door handle was no longer in the full closed position. It was our understanding that if the cabin door is not closed then the cabin door will not arm. We questioned the aircraft integrity if we had a problem where the door could come open in flight. The Flight Attendant confirmed that the handle was down, but we had two separate indications that the Door 2R security is now in question. If the door opened at any speed it would be devastating. Having the Flight Attendant adjust/move handles on the door would not be a safe course of action in case of malfunction with two separate indications showing Door 2R not closed and secure. Dispatch contacted us regarding a joint Dispatch/Maintenance decision to return and land. They wanted us to return to the departure airport. In the following five minutes we received three ACARS messages to return. About ten minutes later we received an ECAM for Brake SYS 1 Fault. We notified Dispatch and Maintenance Control. After that Maintenance Control still wanted us to land over weight. At this time with a Brake SYS fault and a Door security issue known, Dispatch/Maintenance still wanted us to land overweight. My concern as Captain was the "meta message" or what wasn't being said. Dispatch sent ACARS messages to us indicating that he was checking the QRH and penalties. A short time later we received the go ahead to land overweight after confirming with Maintenance Control. Again, my concern was that Maintenance must be concerned about the aircraft if they still wanted us to land overweight with a Brake fault. As pilots we entered a very high workload period. We received a clearance to return from ATC. Now we had a little more than 45 minutes to enter a new destination, runway, RNAV arrival, complete the BRK SYS 1 FAULT QRH, verify all charts in QRH and Pilot handbook, communicating with Operations about a gate, notifying passengers about a return to the departure airport and keeping them updated, and keeping flight attendants and passengers informed about precautionary landing. On downwind we finally completed all QRH, Checklist and PH section (as a precaution) about Loss of All Brakes, as noted in the QRH procedure. We then declared an emergency with ATC. We then re-verified that all charts and QRH Checklist had been completed. Before touchdown we had Door 2R issue and Brakes SYS 1 fault. We landed with a touchdown of 100 FPM and monitoring brake temp did not exceed 800C. A max brake temperature of 545C was noted. After landing I was made aware that this aircraft had a history of door issues previously. This served to validate my concern at the time, that Maintenance and Dispatch had good reason to request that we get the aircraft on the ground ASAP. I believe this event illustrates the relationship between Dispatch, Maintenance and pilots at this Air Carrier. As a pilot I try to get all of the information I can regarding a problem and then use my experience to find the safest solution to that problem. I don't have all of the answers and I rely on the expertise provided by the Crews, Dispatch, Maintenance, Operations, ATC, and my crew in the cabin. I relied heavily on the perception that Maintenance and Dispatch both wanted the aircraft back on the ground. Although the aircraft remained pressurized we felt that it was not a given that it would continue to remain pressurized. The decision (right or wrong) in our minds, after deciphering all of the other information presented, was the intent was to get the aircraft on the ground ASAP. There has been speculation that we should have burned down to landing weight before we landed. The First Officer and I discussed this at length and attempted to sway Dispatch/Maintenance toward that end, to validate our concerns, at least twice. Instead our course of action was validated by the continued insistence that we continue and land overweight. Our reliance, especially on Maintenance, to give us the information we need can be tainted by goals or objectives that don't match ours, from a pilot's perspective. For those that have the luxury of looking at these events after the fact and with the benefit of knowing the outcome, (myself included) it can appear pretty clear cut. This discussion would be completely different had the door come open while we were burning down to landing weight.

Synopsis

An A320 had a Main Cabin Door CABIN PRESS annunciation. The door would not remain armed. Returning to the departure station the ECAM alerted BRK SYS 1 FAULT. An emergency was declared and an over weight landing was made.

ACN: 975344 (2355 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 28000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 280
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 975344

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1128
ASRS Report Number.Accession Number : 975352

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight

Narrative: 1

Captain and I heard a bang, I looked down and saw smoke coming from the Captain's computer bag. I went on oxygen and switched the packs to high flow. The Captain checked his bag and discovered his flashlight's lithium battery had exploded. Rest of the flight uneventful.

Narrative: 2

While eating my breakfast in cruise at FL280 my First Officer and I heard a loud popping noise from the rear of the cockpit. Looking back he noticed some smoke rising from my computer bag located behind my seat. We also detected some strange-smelling fumes. He promptly put on his oxygen mask and I told him to continue to fly the airplane and talk to ATC as necessary while I attempted to resolve the situation. Getting up out of my seat, I observed that the smoke was coming from my lithium battery powered flashlight. I promptly took the flashlight out of the bag and placed it in an open space on the floor close to the cockpit door. While the flashlight smoke started to subside (very quickly) we turned off the cabin fan switch and placed the pack flow switch to high to increase the fresh air flow. The Purser called the cockpit and informed me that there was the smell of fumes in the forward lavatory. After explaining what had happened I told her that we appeared to have solved the problem and that we would continue on to destination. I referred to the FM and FOM to make sure we had "covered all the bases." With the smoke and the fumes gone we proceeded on to our destination and the First Officer made an uneventful (and smooth) landing.

Synopsis

A320 flight crew reports a lithium battery failure in a flashlight stored in the Captain's computer bag. The failure is announced by popping noises and smoke rising from the bag. Flashlight is placed on the floor for observation and flight continues to destination.

ACN: 975335 (2356 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 26000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 975335
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise at FL340, [we] had a sudden low hum and vibration. There was a windshield pressure leak reported and fixed. First thought was leak was back. Started decent and contacted Dispatch and got Maintenance on line. Sound and vibration was below my feet. I felt it could have been a loose nose gear door; Maintenance felt could be loose radar dome, or leaking window. No emergency declared, but decided ZZZ was best place to get looked at. Approximately 80 miles east of our divert airport at FL240 noise and vibration increased. A few minutes later it stopped and we got an Avionics exhaust fan ECAM. [It] had small electric smell. Declared an emergency and picked up speed and landed at the airport. Had equipment standing by, but briefed Tower and flight attendants that it was only precautionary. [We had a] normal landing and taxi to gate.

Synopsis

A320 Captain experienced a low frequency vibration below his feet at FL340 and elects to divert for maintenance. During descent the vibration increases then stops abruptly, illuminating an avionics exhaust fan ECAM.

ACN: 975205 (2357 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 975205

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

An A320 Flight Crew reported Door 2R was not armed in flight and the cabin pressure light illuminated in climb. I asked if there was any loss of pressure or did the crew hear any squealing noise. No pressure loss and no noises from door. It appeared the door arm handle would not stay in the armed position. After conferring with maintenance I decided to bring the flight back to its departure airport. I also decided that an overweight landing was preferable to burning 15000 pounds of fuel and so advised the crew. 

The Flight Crew had also received a Brake System 1 fault on ECAM. The affected BCSU was reset per the QRH but the ECAM message remained. I calculated the landing distance with the brake fault and we were legal to land. The Flight Crew declared an emergency and landed safely at 160000 pounds at a descent rate 100 FPM and airspeed of 155 knots using full flaps and medium autobrakes. Following landing the number two brake temperature reached 580 degrees.

Synopsis

An A320 flight returned to its departure airport following receipt of a door not armed ECAM message and an illuminated cabin pressure light. An over weight landing was made which resulted in one brake temperature reaching 580 degrees.

ACN: 974818 (2358 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Aileron Control System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 974818

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 975353
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Ops normal in cruise at FL350. Began to pick up what felt like light chop. We noticed within a few minutes that the characteristics of the chop were unusual. 
1.The aircraft only moved in pitch-absolutely no role or yaw. 
2.The chop was a very consistent "bump" that occurred on a cycle of 10-15 seconds. 

We pulled up the F/CTRL display on the SD. Immediately we noticed that both the left and right aileron indices were moving in sync with the bump. Both indices would move up and down rapidly 1/16 to 1/8 of an inch of total travel on the synoptic as each bump was felt. Applying systems knowledge, we knew that ELAC1 normally controlled the ailerons. During preflight planning, we had noted that this particular aircraft had just had ELAC1 replaced after multiple write-ups for ELAC1 Fault and Ail Servo ECAMS in flight. Accordingly, I pulled out the FM and reviewed the F/CTL ELAC 1/2 Fault Irregular. It reads: One computer failed: CAUTION: Do not reset ELAC if uncommanded maneuvers occurred during flight. ELAC (affected)...OFF THEN ON 
Based on the uncommanded pitch motion, the maintenance history of the aircraft, my systems knowledge and the caution above, I decided that the safest course of action was to turn off ELAC1. The First Officer disconnected autopilot and was hand flying as I turned off ELAC1. Aileron movement and pitch bump stopped IMMEDIATELY and did not recur for the remainder of the flight. We contacted Dispatch via ACARS and added Maintenance to the patch. I described situation and agreed to make maintenance write up as we continued to destination. I also reminded Dispatch that we were now downgraded to Cat 2 status with the ELAC1 turned off. The rest of the flight was routine to landing.

Narrative: 2

At FL350 began to experience a rhythmic bump which at first was thought to be light chop, however after a few minutes the chop was occurring on a 10-15 second interval and with a slight vertical bounce to the aircraft. FCTL display showed ailerons moving 1/16 to 1/8 inch in sync with bump. ELAC1 had just been replaced which according to systems knowledge controls the ailerons. After reading the FM for ELAC failure we turned off ELAC1 and the bump immediately ceased and did not return. We contacted Dispatch and Maintenance Control and briefed them on what had happened. We coordinated a write-up to describe the event and our actions, remainder of flight was uneventful.

Synopsis

A320 Flight Crew experiences what is described as bump or rhythmic light chop at FL350. While checking the Flight Control page it is discovered that the bump is in sync with aileron movement. Due to the maintenance history of the aircraft, the Captain elects to turn of ELAC1 and the bump immediately disappears.

ACN: 974777 (2359 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 974777

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climbout, we received ECAM, RH Window Heat INOP. Upon inspection, we noticed that the First Officer Right rear window had a severe crack that was progressing. Aircraft was leveled off at 13,000 to prevent any possible pressurization problems. Upon consultation with Dispatch and Maintenance, the decision was made to return and execute an overweight landing. Landing made per Flt manual guidance. Less than 100 FPM descent at touchdown. Weight approximately 153,000 pounds. No other issues. Captain Authority exercised to allow overweight landing.

Synopsis

A320 Captain reports an ECAM for RH Window Heat INOP during initial climb. Upon inspection the right rear window is found to have a severe crack. After consultation with Maintenance Control the flight returns for an overweight landing.

ACN: 974210 (2360 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Wing Access Panel
Manufacturer : Airbus
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12950
Experience.Flight Crew.Last 90 Days : 249
Experience.Flight Crew.Type : 5006
ASRS Report Number.Accession Number : 974210
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 974232
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

The Captain performed the preflight and noticed that the left outboard wing vent panel which had drainage holes was installed backwards as compared to the right side wing. The panel number was 534AB. The Captain wrote the panel number down and went back to the cockpit. When airborne, the Captain thought through the reason for the drainage holes and their position and came to the conclusion that the drainage holes should be on the low side of the wing versus the high side and submitted the writeup via ACARS. Although this is not a required preflight item, the Captain decided to write the issue up rather than remain silent and the Captain explained to the First Officer what the Captain saw and why the Captain wrote it up. The flight conditions were CAVU with no known icing conditions and were forecast as such throughout the flight. 

Between flights, the Captain received a phone message from Flight Operations, and was told that Mechanic X was writing me [the Captain] up and filing a report. The Captain attempted to get in touch with Mechanic X through local Maintenance, through local Maintenance frequencies, phone numbers and even tried to find someone in an office near the gate, but was not able to. Once the Captain reached the cockpit for the next flight, the First Officer said that after the Captain left the cockpit, a person entered the cockpit in non-maintenance attire and spoke with her about the writeup. Then the First Officer, of all things, told the Captain that she found the very same issue on her preflight for this aircraft. 

The Captain went outside and visually checked both wings with the First Officer and confirmed that the left wing vent panel seemed to have been installed backwards. We both decided to contact Maintenance before writing it up because now we weren't sure if this was systemic or just a non-issue. Moreover, we thought it best to get a Mechanic's opinion before proceeding. Back in the cockpit, the Captain contacted Maintenance Control and explained the issue to them and that we wanted someone from Maintenance to come out to the aircraft and look at the vent panels and make a determination before we wrote the mismatch up. A Mechanic entered the cockpit and we explained the issue and the First Officer accompanied the Mechanic outside and showed the Mechanic the discrepancy. The First Officer returned to the cockpit and informed the Captain that the Mechanic advised us to write it up and that they would research it and either fix it or defer it. The Captain submitted the Maintenance Code in ACARS. After a few minutes the Gate Agent asked if we were ready and that she was informed by Operations that everything was good to go. The Captain elected to visually inspect the panel. The panel had been repositioned and we received a new maintenance release. 

The flight departed from the gate two minutes late because we had yet to perform the Before Start Checklist. The new maintenance release ACTION for the log item stated "Panel 534AB was installed backwards removed panel and reinstalled in correct position". Further notes: on the ground, the Captain was fully aware that the flight would be flying into no known icing conditions and was aware that the forecast for the route of flight was clear. The Captain wrote the discrepancy up while airborne after taking some time to think about the issue, and fully aware that it was a pre-flight discrepancy; the Captain believes that after the issue was thought through it would be best to submit a Maintenance Report enroute versus after block-in. 

The Captain did not write this up to increase the workload of company Maintenance. The Captain did not write this up because of Union-Management contract issues. The Captain's intent for the write-up, once the low side/high side vent hole discrepancy was thought through, was solely for safety. Additionally, upon airborne reflection regarding the discrepancy and the subsequent decision to input the write-up, the Captain felt that it was the right thing to do. The Captain confirmed this feeling when the subsequent aircraft had the exact same discrepancy and the panel was shown to be installed backwards. The Captain can attest that all of the above facts are stated truthfully and accurately.

Narrative: 2

I was Pilot Flying on this flight. My Captain performed the preflight walk around for our flight as he normally does for his First Officer when his First Officer is the pilot flying. Upon completion of his walkaround he made no comments on his inspection and we completed our checklists and made an uneventful departure from the airport. During cruise phase I began preparing my approach plates and runway diagrams for our arrival, and my Captain began requesting the next leg paperwork through the ACARS. When I was finished with my preparations I noticed my Captain finishing entering a maintenance write-up into the ACARS. He entered a [numerical] designation and sent it. I queried him about what he had written up. 

He said there was a small panel under each wing tip that had drain holes for drainage of moisture should any enter the panel during deicing. One of our panels was installed backwards. Each wing tip had an identical symmetrical panel and the panel should be installed with the drain holes on the lower edge of the opening due to wing dihedral. I mentioned that I was unaware of the particular panel other than it is one of many panels that are visible during walk around and should be secure with all fasteners installed and no leaking fluids. And that to my knowledge I have never been instructed on the possibility of this panel's drain holes being positioned improperly. He said he became aware of this situation during a pilot class or pilot training at the training center and it isn't one of those things that are put in the Flight Manual. I was notified after the flight by my Captain that Maintenance is issuing a report due to our entering the write-up inflight rather than prior to the flight.

Synopsis

A Captain and First Officer describe why a left outboard wing vent panel #534AB on their A320 aircraft had been installed backwards. It was written-up up in-flight, instead of prior to departure when Captain noticed the discrepancy during a pre-flight walkaround.

ACN: 974204 (2361 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Ferry / Re-Positioning
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 974204
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were to fly a plane down to be painted and take home one that was done. I already had two nights of very poor sleep with one in ZZZZ and the other at home not being able to get to sleep. Once I finally got to sleep at home, the Crew Desk called around XA30 to tell me of the trip for this evening. I never got back to sleep and was unable to nap during the day. Had this trip gone as planned, I felt I could have safely completed it. However, nothing was done in preparation for this flight. The plane sat on the ground for five hours with numerous open write ups and it had not been fueled. Nothing was done to the aircraft until we finally got to it at the hangar and started the ball rolling ourselves. Delays rolled on and on. The more we were delayed the more fatigued I got, until finally I determined I was no longer able to safely complete the mission. I called in fatigued.

Synopsis

A320 Captain reports calling in fatigued when poor preflight rest and maintenance delays combine to produce unacceptable fatigue.

ACN: 973373 (2362 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Other 
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Oxygen System/Pax
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 973373
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Crew O2 was serviced prior to departure (2nd time in a week) as it was 1170 PSI. During cruise we noticed that crew O2 pressure was decreasing at what appeared to be faster than cold-soaking would account for. We contacted dispatch. During radio contact with dispatch and Flight Duty Officer the rate of O2 pressure decrease seemed to accelerate. I immediately decided to begin descent at that point in time. After coordination with the above parties and considering ferry fuel we fortunately had on board, we determined that the safe course of action would be to continue to descend so as to arrive at 10,000 feet altitude with much greater than Flight Manual required oxygen quantity and continue to destination at 10,000 feet while closely monitoring fuel reserves and available alternate airports. This we did and we landed at destination with approximately 9,000 lbs or 1.5 hrs of fuel on board on a VMC day with no alternate required. We blocked in with greater than FM required O2 quantity.

Synopsis

An A320 Captain descended to 10,000 FT after the oxygen level unexpectedly decreased and was able to continue to the destination with sufficient fuel reserves.

ACN: 972990 (2363 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 972990

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

Aircraft dispatched with BSCU1 inoperative. BSCU2 failed in flight resulting in loss of normal brakes, antiskid and nose wheel steering. Decision was made to divert to [an airport with a longer runway] where an uneventful landing and rollout ensued.

Synopsis

A320 pilot reports diverting when BSCU2 (Brake Steering Control Unit) fails in flight after the aircraft has been dispatched with BSCU1 deferred.

ACN: 972743 (2364 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 972743

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We were dispatched with BSCU #2 deferred. About 1:45 into the flight we got BRAKES A/Skid NWS FAULT ECAM with Cat 3 Dual, A/Skid, N/W steering, BSCU 1 and BSCU 2 INOP on the status page. Having departed with BSCU #2 INOP and the status page now showing BSCU #1 INOP also, I tried the FM BSCU reset with no effect. Established a phone patch with Maintenance Control and Dispatch. Maintenance Control had no further suggestions and it was determined we would be landing with alternate brakes, no antiskid or nose wheel steering. A tow in from the runway would be required. Given better facilities in ZZZ then ZZZ2 I suggested diverting to ZZZ, Maintenance Control and Dispatch agreed. (ZZZ wx was good with light north winds) We discussed communications, Dispatch would advise ZZZ ops and Tower or our situation, we would arrange the destination change with ATC. We were about 250 miles from ZZZ. The First Officer and I discussed landing and, due to the lack of steering and possibility of tire damage decided a cabin prep would be prudent. I had the Purser come to the cockpit and we briefed all items for the for the prep with about 30 minutes to go. She went back to brief her crew and after a few minutes I made a PA telling the passengers what was going on and what was going to happen. We proceeded toward ZZZ and reviewed landing distance requirements, FM procedures and system descriptions. About 20 or 25 minutes later the purser called back advising that they were ready in back. As we approached ZZZ we verified that ATC knew we would be an emergency for the landing and that equipment was in place. It was. ZZZ had several wind changes and we discussed which runway would be best for minimum cross wind and no tailwind. We ended up using XXR with SW winds at about 12 knots. Landing was uneventful and we were able to stay on the centerline and stop without incident. Airport OPS personnel and emergency equipment checked our main gear and found no abnormalities. Maintenance towed us to the gate.

Synopsis

A320 Captain reports being dispatched with BSCU #2 deferred. Enroute BSCU #1 fails which results in the loss of normal brakes and nose wheel steering. The crew elects to diverts to an airport with a longer runway and lands safely.

ACN: 972525 (2365 of 2908)

Time / Day

Date : 201110
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 972525
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Captain performed the walk-around and noted what appeared to be a hydraulic fluid leak coming from the left horizontal stabilizer. Maintenance release was sent, Operations was notified, and a Maintenance release receipt came up on printer with a time stamp of XA:19Z. After about 15 minutes, First Officer called Operations to ask if Maintenance had been to aircraft yet, because we hadn't seen or spoken to any Contract Maintenance personnel as of that time. At XA:38Z, Contract Maintenance Technician arrived at cockpit (I know this was when he was there because a new Maintenance release had started rolling out of the printer while he was there) and told Captain and First Officer that "everything was ok." When we asked [the] Mechanic about the source of the leak and the number of drips per minute, he reported that he had not seen any leaks. He then pulled up the HYD page on the ECAM display to check the hydraulic fluid quantities (it must be noted that the Maintenance release was already printing at the time that the Mechanic was checking the hydraulic quantity for the first time). The Mechanic then left cockpit and was followed by the Captain out the jetway door. First Officer stayed behind to read the Maintenance release that had finished printing and saw that the hydraulic leak had been signed off and a new validated Maintenance release had been issued before the Mechanic had ever seen the hydraulic leak or seen the HYD page on the ECAM display. First Officer then followed the Captain and Maintenance Technician outside to look at the spot were the Captain had seen fluid on aircraft. First Officer immediately spotted hydraulic fluid leak coming from junction of horizontal stabilizer and fuselage, fluid streaks leading to lower section of fuselage, a fluid drop hanging from fuselage at the termination point of the fluid streaks, and at least 8 fluid spots from drips on the ground directly under the hanging fluid drop. First Officer then returned to cockpit to get a camera and took pictures. Later, the Captain told First Officer that [the] Mechanic said that he had been looking at the leading edge of the horizontal stabilizer for a fluid leak. Captain then called Maintenance Control and reported that leak was not properly inspected prior to Maintenance release being sent. Maintenance Control issued new log item (time stamped XA:48Z) and hydraulic leak was properly checked by Maintenance prior to our departure. It seems as if this entire issue would have been averted if the Mechanic had simply asked the Captain and/or First Officer to show him were the fluid leak was prior to signing it off and sending a new Maintenance release.

Synopsis

An A320 crew discovered a hydraulic leak during preflight at an out station and after reporting it to Maintenance they received a Maintenance release, clearing the aircraft. However, the Mechanic had not looked at the leak before clearing it.

ACN: 972388 (2366 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Component

Aircraft Component : Hydraulic Syst Reservoir Tank
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 972388
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Notified Maintenance that the green hydraulic system appeared over-serviced. Maintenance came out, after a delay, and confirmed over-service. Their procedure was to station one mechanic in the cockpit to monitor ECAM quantity, operate Yellow pump and PTU, communicating via walkie-talkie with mechanic at left main landing gear who bled hydraulic fluid from the "appropriate" system. A fluid spill inevitably occurred, twice, and it is likely the wrong system was first serviced, due to inconsistencies concerning how the mechanic in the cockpit had to come down to the main gear to talk with the mechanic servicing the systems, despite both having an operating radio and talking with each other. 

Company management appears more concerned with an "on-time" departure than encouraging front-line employees to do procedures in a correct manner. This behavior is reinforced from the executive level. The attitude expressed by one of the mechanics (a supervisor) who stated to me that it was okay to go with the quantity indicating all the way up to the shut-off valve indication on the ECAM. I told him that our procedure was only to notify maintenance and not to diagnose the problem. The fact that maintenance went ahead and lowered the hydraulic quantity tells me that he did not fully understand why maintenance was performed, much less knowing the correct procedure. I observed the correct procedure on the same issue accomplished the previous day. Lower gear doors and correct over-service at the reservoir, minimizing risk of bleeding wrong system and spillage. Time to do it right: ten minutes, Time to do it wrong: twenty-five minutes, minimum.

Synopsis

A320 Captain reports a hydraulic fluid spill after informing Maintenance that the Green Hydraulic system is over serviced, maintenance attempts to "bleed" the system.

ACN: 972387 (2367 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : JAX.Tower
State Reference : FL
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : JAX
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
Make Model Name : Citation I (C500)

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 972387
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We had just completed engine start after pushback from the gate at JAX. I started to release the parking brakes as the First Officer called JAX ground control for taxi instructions when a Cessna Citation taxied across our nose from right to left between us and the terminal building. The Citation had less than 10 feet wingtip clearance from the nose of our aircraft and a similar spacing from his right wingtip and a ground vehicle. If we had moved forward at all, we probably would have had a collision. Ground control had said nothing to us about the Citation taxiing in front of us. The airport information page says specifically that taxiing aircraft are to remain on taxi centerlines on the air carrier ramp.

Ground control was more interested in moving aircraft around than safety. If I had released the brakes a second or 2 sooner, there would have been a collision. Ground control needs to follow its own procedures and make aircraft hold position until it is safe to taxi. The Citation should never have accepted taxiing so close to our aircraft not knowing when we were going to start moving.

Synopsis

A320 Captain reports a citation jet taxiing between his aircraft and the terminal building after pushback and engine start at JAX.

ACN: 971783 (2368 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 2100
ASRS Report Number.Accession Number : 971783

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 971769

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During preflight I was concerned about having enough fuel based on somewhat vague writeup with discrepancy between left and right inner wing tanks at end of flight (left tank higher). Called Maintenance to discuss fuel issue. He thought we were okay to go based on his conversation with contract maintenance and fueler. Had tank drip stick performed and added fuel as a precaution against a fuel quantity indicating problem. Told First Officer to keep a very detailed fuel log. Discovered at first check that although fuel used for each engine was the same that the right wing fuel tank is showing less quantity. First Officer looked up fuel leak checklist in FM. Sent jumpseater back to look at right side for secondary indications of a leak from engine or wing. No evidence found. First Officer performed 30 minute fuel discrepancy procedure IAW Flight Manual fuel leak procedure. After 30 minutes found 500 lbs missing, less than the 660 lbs required by the Flight Manual to call it a leak. After much discussion about no secondary indications of leak and not meeting the Flight Manual definition of having a fuel leak, decided to continue and monitor with First Officer performing another fuel discrepancy check. No change to the calculations. Later noticed fuel temperature differential between left and right outer fuel tanks. Wrote up all the above data to facilitate maintenance troubleshooting. Accidentally reported a loss of 21000 lbs/hr vs actual of 1000 lbs/hr. Maintenance queried and we corrected with a message reply on ACARS. Also, noted right engine fuel flow is fairly consistently 40-80 lbs/hr higher than left engine. Sent ACARS message to station and maintenance to have mechanic meet us at the gate so we could thoroughly debrief them on what we saw. At fuel check required at end of flight, did not meet requirement to write up (fuel used plus FOB compared to FOB at start of flight) difference of greater than 1000 lbs. Concerned about the way the Flight Manual fuel leak procedure is written and why the magic number to call it a valid fuel leak is 660 lbs in 30 minutes with no secondary indications of a leak.

Synopsis

An A320 flight crew described some confusion dealing with a possible fuel leak in flight.

ACN: 971362 (2369 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3850
ASRS Report Number.Accession Number : 971362
Human Factors : Fatigue
Human Factors : Time Pressure

Events

Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

We were supposed to do a procedure that involves opening the fuel transfer valves on the ground before departure according to an MEL. We were 12 hours into our 13 hour duty day, and due to fatigue we both forgot it. I have had this MEL many times before without incident. I didn't realize we had forgotten it until the next morning when we had the same plane. We didn't forget it this time. This could have caused a trapped fuel condition that has serious consequences. Fortunately, in this case it did not.

Synopsis

A320 Captain forgot to do a procedure that involves opening the fuel transfer valves on the ground before departure according to an MEL. Fatigue was cited as a factor.

ACN: 971245 (2370 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 971245
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

First Officer noted on exterior inspection abnormal accumulation of apparent hydraulic fluid on aircraft belly from gear area to tail. Maintenance writeup generated Maintenance response. Gear doors were lowered during Maintenance inspection where more fluid that had accumulated was evident. Following actuation of hydraulic systems and flaps, no source of leak was noted and plane returned to service with "no active leak". Throughout the delay I updated PAX on the hydraulic leak issue. When plane was signed off I could not accept aircraft without determining the source. I advised Dispatch that we would be "unable to operate" the flight with the present aircraft without the leak source identified. We were assigned another plane.

Synopsis

A320 Captain describes the discovery of apparent hydraulic fluid on aircraft belly from gear area to tail during preflight. When the aircraft is signed off by maintenance as "no active leak discovered", the aircraft is refused.

ACN: 971244 (2371 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Other 
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 4700
ASRS Report Number.Accession Number : 971244
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Arrived at aircraft and maintenance was installing a temporary TR unit in last seat of aircraft to power a passenger nebulizer. Using an extension cord, maintenance ran the cord from aft power outlet by door 2L to the TR unit. Maintenance could not get 120v from outlet though aircraft fully ground powered. Both pilots and flight attendants knew that these outlets do not provide adequate power. A Flight Attendant remarked that they are instructed in training NEVER to use them. The Flight Attendant also remarked that in the Flight Attendant Operating Manual, Nebulizers are NOT an approved device. The Maintenance tech said he was told by his Super that they are approved. The Captain said he only knows of battery-powered Nebulizers being observed previously. My concern was of in-flight fire hazard because of the inconsistent voltage known to come from many of these outlets, and the 400hz cycles that I'm fairly certain harm sensitive electronics that are used to 60hz household power. Problem solved itself because maintenance could not come up with needed voltage on our aircraft after testing numerous outlets onboard aircraft. They removed item and wiring from aircraft. We were told that this nebulizer was for an outbound passenger the following morning. This report being filed in the spirit of safety and inter-departmental issues that were presented.

Synopsis

Against the flight crew's wishes an A320 Mechanic attempted to install an extension cord from an aircraft AC power outlet to power a passenger nebulizer but failed because reliable power could not be obtained.

ACN: 971065 (2372 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 9000

Environment

Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 95
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 971065
Human Factors : Other / Unknown
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 50
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 971601

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

After takeoff from DEN Runway 34R, we were given a right turn to 095 with a climb to 13,000 FT and handed off to Departure Control. The heading pointed us directly toward a cumulus build-up approximately 10 to 15 NM ahead with dense virga coming out of the bottom that nearly reached the ground. We began to experience moderate chop and I immediately requested a 15 degree right turn to avoid the weather. Departure told me unable right turn, so I requested a 20 degree left turn and again Departure told me unable because we were not in his airspace. I then informed him that we were going to make a turn left or right, his choice, but that we had to turn for the weather. We showed nothing on the TCAS on either side. The Controller got very huffy and demanded a PIREP and then gave us the right turn we originally asked for. This unnecessarily long conversation with Departure drove us [very] close to the weather. The deviation required us to track approximately 5 to 10 NM right of course and 15 to 20 NM down range before we could return to the original flightplan. I requested the initial deviation for a reason and I was clear about that reason. It is completely unsatisfactory and distracting for the Controller to second guess or be indignant about my stated requirement to maneuver in order to ensure the safety of my passengers, crew, and ship.

Synopsis

Air carrier departure from DEN expressed concern regarding ATC handling during requested deviations because of weather, characterizing the Controller's performance as "completely unsatisfactory."

ACN: 970267 (2373 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 4
Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 970267
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were repositioning the aircraft after having maintenance performed at an off line station. During the takeoff roll, we received an ECAM message: Brakes-Released. The takeoff was aborted. After running through ECAM actions, we determined that the aircraft was safe to move and we returned to the maintenance hangar.

After inspection, the mechanic determined that a set of wires had not been reconnected properly which most likely caused the initial ECAM message.

Synopsis

On its first flight after maintenance had been performed an A320 flight crew rejected their takeoff upon receipt of a Brakes Released ECAM message.

ACN: 970177 (2374 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 970177
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

A B737 stopped at FL350 for traffic handed off to Sector X. I called to point out the B737 to Sector X; Sector X started flashing an A320 to me at FL350. Sector X said the A320 was 20 left to go behind the B737 and approved point out. I climbed the B737 to FL380, aircraft took a while to start climb. Sector X switched the A320 to me in my airspace, I then realized the 20 degrees left turn was not working and turned the A320 additional 15 degrees left, then told the B737 again to climb to FL380 and turn 20 left and report out of FL360. It appears separation was lost at about 4 miles and 500 FT. Don't trust the controller will put an aircraft on a heading that will work.

Synopsis

Enroute Controller described a loss of separation event when relying on another controllers' vectors to achieve diverging courses which did not provide adequate spacing.

ACN: 969828 (2375 of 2908)

Time / Day

Date : 201109

Place

Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10100
Experience.Flight Crew.Last 90 Days : 215
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 969828

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

While established at cruise, FL340, with smooth ride and ATC reporting the same, we encountered a momentary jolt, three quick bumps with no greater intensity than moderate chop. We suspected wake turbulence from a passing aircraft above. It was a surprise to both crew members as the passing aircraft was shown on TCAS as 5,000 FT above our cruising altitude. The fasten seatbelt sign was OFF and no notification of crossing traffic was given by ATC. The aircraft did not suffer any loss of altitude or upset attitude, and at no time was any strain felt against my lap belt. The ride was smooth and there were no adverse ride reports. Approximately 1 hour later the Lead Flight Attendant notified the cockpit that a passenger claimed to have suffered a broken toe. Later, I was learned that the passenger was resting his foot on part of either the sidewall or aft end of the armrest from the seat in front of him. Upon sensing the quick jolts of the aircraft, he quickly withdrew his foot striking the structure. The Lead Flight Attendant reported to me that the passenger mentioned it might have been his (the passenger's) fault for the injury. The passenger claimed he injured his right foot, first toe, and the top of his right foot. The passenger was given an aspirin. A volunteer medical professional was summoned and a doctor responded. The doctor advised that all that could be done was to elevate the foot, apply ice as necessary and administer ibuprofen. After consultation with Dispatch and [medical personnel], it was determine safe to continue. Paramedics were summoned to the aircraft and subsequently met the passenger.

Synopsis

An A320 Captain reported encountering wake vortex that resulted in moderate chop. A passenger complained of a broken toe as a result of the event.

ACN: 969796 (2376 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 145 ER/LR
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 969796
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

RADAR training was in progress. An A320 was at FL330, southeast bound. LOA requires arrivals at FL290. An E145 was enroute at FL320. Trainee initiated a hand off on the E145 to an incorrect sector. The controller of that sector called trainee to advise of the error. The instructor looked down at note pad and was recording the discrepancy. During that time, trainee cleared the A320 to FL290. Data Block overlap was a problem, as the Data Block of the E145 was obscured by other Data Blocks. The on-duty FLM was standing behind the trainee, observing the sector operations. He caught the descent clearance and the conflicting traffic. He alerted the trainee and instructed the trainee to void the descent clearance. Trainee failed to respond. Instructor reacted to supervisor and attempted to stop the descent but the aircraft had already left FL330 and come into conflict with the E145. Recommendation, it is an unfortunate reality of OJT that occasionally, an error cannot be recalled. What is fortunate in this case is that the two aircraft in question were not in danger of an actual collision. I do not believe there is a better or more effective way to learn how to do this job. A computer simulation cannot equal live traffic. As an instructor, I do my best to ensure safety while still allowing my trainees to experience learning opportunities. Also, as an instructor, I do my best to accurately record anything notable ASAP, in order to ensure descriptive and meaningful training reports. When this error occurred, my eyes were averted from the scope and I missed the clearance that was issued.

Synopsis

Enroute Controller providing OJT described a loss of separation event when failing to note a conflicting descent clearance issued by the developmental, noting the documentation of performance as a causal factor.

ACN: 969619 (2377 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 9500
ASRS Report Number.Accession Number : 969619
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

Captains Authority. After a very hectic last minute boarding with transferred passengers from a mechanical delay on another flight, a Company pilot arrived as door was getting closed. The cabin was full and so I asked for his Flight Deck Authority Access Card. He did not have one. So I tried to quickly get the CSR's attention as he was pulling the jetway away to come back. The concern was that the CSR did not comply with my clear hand signal to come back and instead only stopped the jetway and made a phone call. This is by no means a critique towards the CSR but a critique on the misguided policy of Company gate departure procedures that place a heavy burden on CSR's to get the flights out on time and not get charged a delay. The serious concern here is that my Captain's authority was completely ignored and deviated from, when the need for the jetway to return was requested, as this could have been for a myriad of possible emergencies to include, unathorized/unkown person on board, medical emergency, terror threat, fire, smoke, fuel threat, etc. The responsibility lies with the Captain and thus his or hers decision to return the jetway should not be questioned when it is clear they are signaling for it to return and thus the CSR should not be in any way charged with any delay whatsoever. The delayed action to return the jetway presents a liable and dangerous scenario!

Synopsis

An A320 Captain reported that an unauthorized jumpseater was on board but the Gate Agent refused his command to return the jetway thus ignoring Captain's authority.

ACN: 969428 (2378 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Relative Position.Distance.Nautical Miles : 20
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ORD

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 969428
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Just below 10,000 FT on descent Captain's ND went to map not available, all nav data was lost. Tried to manually tune the ORD VOR in MCDU 1... Wait for response was displayed..VOR on ND was useless. All speeds dropped from the FMGC. There was no ECAM or other fault message. Later there was a pressurization ECAM regarding lost landing elevation data. The approach mode would not arm on either side on final approach. The weather was clear, we made a visual approach, manually set the landing elevation and landed without incident. In bad weather it could have been serious. The weather was clear.

Synopsis

While on initial arrival the flight crew of an A320 suffered the loss of multiple FMGC related functions including the Captain's ND, all nav data, landing performance data, and landing elevation reference. Fortunately, clear skies prevailed and an uneventful landing ensued.

ACN: 968408 (2379 of 2908)

Time / Day

Date : 201109

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 968408

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 968770
Human Factors : Human-Machine Interface

Events

Anomaly.Ground Event / Encounter : Ground Strike - Aircraft
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Physical Injury / Incapacitation
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Ambiguous

Narrative: 1

With the Captain flying, [this] night landing was at 6.5 degrees of pitch. Immediately after landing, aircraft did pitch up a little, but did not seem at all unusual. There was no unusual sensation or noise during landing touchdown or rollout. However, shortly after touch-down, there was a very quick amber ECAM indication that I noticed out of my peripheral vision. During my scan, I was looking at airspeed at time of amber indication and was unable to identify since it was only illuminated about two seconds. Captain did not see the actual indication readout either, only seeing the amber light. Captain chose to look aircraft over after arriving at the gate. He stated that there had been a slight amount of damage to tail section, underneath APU, due to what he thought might have been a tail strike. Captain said he entered the occurrence in aircraft logbook.

In retrospect, it appears that Captain touched down within pitch limitation, but may have pulled back on side stick at touchdown, causing a slight grazing of tail section against runway surface. This is a difficult one, because it is very difficult at times to know where the pitch is in relationship to the limitation without staring at the attitude indicator continuously. The pitch was well below the 10 degree limitation at touchdown, but evidently it changed just enough to cause a problem just after landing. I felt equally responsible for not seeing the amber indication reading, so I could report this to the Captain. I had just made the "speed brake" callout and had shifted focus on the airspeed, anticipating the high speed turnoff ahead, when the amber light flashed on and then off. In the future, I will put more of an emphasis on monitoring pitch, not only during landing, but also just after touchdown. My scan is going to be more intense at this critical time during the flight. I also will be determined to improve my scan overall, as monitoring pilot, making sure not to fixate, but take in the entire picture before me.

Narrative: 2

During the landing, the flare may have been slightly early resulting in a slight rise after touchdown (slight bounce). While correcting for second touchdown I heard a ding (possible ECAM) with no ECAM displayed. Touchdown was smooth. During post flight inspection, discovered APU drain mast was scraped. I called Maintenance Control and made a log book entry.

It is possible the ding we heard was an ECAM. [The] pitch resulted in a possible APU drain mast scrape. Winds were reported calm. However, during the approach we had a slight tail wind. In addition, the aircraft felt heavy on takeoff, climb, and approach; sluggish or slow to respond. [The aircraft was] possibly overweight. With a bounced aircraft, I normally hold attitude while [it] settles in order to prevent exceeding pitch attitude. It is a balance of attitude/airspeed to prevent floating down the runway. Airspeed bleeds off and second touch down occurs usually close to Vls. I suspect an overweight aircraft due to the close proximity of second touchdown with aircraft pitching nose up (stall) so rapidly resulting in a possible APU drain mast being dragged on the runway.

One technique a First Officer showed me was to pull up the aircraft calculated weight in the FMGC (via FAC). He said he was told to accomplish this by a Check Airman as a matter of technique. The FAC uses data from elevator/thrust/airspeed and will give a weight display according to data obtained. That weight is highly accurate and often times we are about 5,000 LBS heavier than weight and balance data. As a matter of technique, if 5,000 LBS overweight add 5 KTS to the Vls as protection against a stalled landing. I have started to use his technique. However, on this flight due to the short duration I neglected to check the FAC produced weight data. I would like to see more published on this topic and training should include this cross check of available data in normal operations.

Synopsis

A320 flight crew experiences a tail strike during a night landing. Damage to the APU drain mast is noted during post flight.

ACN: 968218 (2380 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 968218
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After takeoff, when I went to retract landing gear, we received MASTER CAUTION and MASTER WARNING lights and audible alert. Upper ECAM and Lower ECAM indicated complete loss of the GREEN Hydraulic System. Landing gear would not retract. There was loss of all items associated with loss of GREEN Hydraulics, including Nosewheel Steering, Normal Brakes, #1 Thrust Reverser, etc. I ran QRH and ECAM checklists. We declared EMERGENCY with Departure Control, and received vectors for landing Runway 18 ILS Approach. I notified flight attendants, Operations, and briefed the passengers before landing. I told Tower we would need [to be]towed off Runway 18 due to loss of nosewheel steering. We flew a normal ILS approach, landing, and rollout. Fire trucks were awaiting our landing. We came to full stop on runway centerline, started the APU, and shutdown engines after the 3 minute cooling period. A Tug towed us to the gate.

Synopsis

An A320's GREEN Hydraulic system failed after takeoff. An emergency was declared, the flight returned to land and after stopping on the runway was towed to the gate.

ACN: 967858 (2381 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZNY.ARTCC
State Reference : NY
Altitude.MSL.Single Value : 28000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZNY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZNY

Component

Aircraft Component : Oxygen System/Crew
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 967858
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were climbing to FL360 when I checked the ECAM and noticed that the oxygen level was down to 450 PSI. The First Officer checked masks on his side and found that the observer's mask was releasing oxygen. We leveled off at FL280 to assess the situation. The First Officer checked manuals to look for guidance while I contacted the Dispatcher via ACARS. The First Officer found no guidance in the manuals. The Dispatcher and I considered the weather at our destination, deleted our alternate, and determined the fuel burn to destination at 10,000 FT MSL. We declared an emergency, advised the Flight Attendants, Dispatch, and the passengers and proceeded without incident.

Synopsis

An A320 flight crew descended to 10,000 FT MSL and continued to their destination after discovering the oxygen system pressure had dropped to 450 PSI due to a leaking observer mask.

ACN: 967786 (2382 of 2908)

Time / Day

Date : 201109
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 2200
ASRS Report Number.Accession Number : 967786
Human Factors : Training / Qualification
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 967795
Human Factors : Training / Qualification
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At about 1,500 FT after takeoff, ECAM [for] HYD YELLOW RSVR OVHT illuminated. After safely airborne and the after takeoff checklist completed, I completed the ECAM and then the appropriate flight manual procedure for the overheat [condition]. During cruise and after sending Maintenance our MRM code and other information for the ECAM fault, Maintenance informed us that the fault was in the switch for the Yellow system engine driven pump. The Flight Manual directs that, after the overheat indication clear and during approach, turn the Yellow system engine driven pump and PTU back on. With this additional information from Maintenance, the Captain and I elected not to, but rather leave the Yellow system depressurized. We had already determined the landing distance with the Yellow system unpressurized and verified with Dispatch that this would not be an issue. The approach and landing was completed uneventfully by the Captain.

Synopsis

An A320 ECAM alerted HYD YELLOW RSVR OVHT at 1,500 FT after takeoff. The ECAM actions were completed and the flight continued to its planned destination with the pumps and PTU off.

ACN: 967574 (2383 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 967574

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Passenger Misconduct
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise, the Purser contacted the flightdeck and advised us of the presence of electrical fumes around rows 19 and 20 of the aircraft. The fumes were not present in any other section of the cabin. He stated the fumes had been present for a while and were not diminishing. I contacted Maintenance via phone and asked for any input they could offer. The Maintenance Controller suggested to shut off the IFE (in flight entertainment) using the emergency shutoff switch. This was performed by the cabin crew. After approximately 10 minutes, I phoned the Purser and was told the fumes had not dissipated and were now moving aft of the original location. Other sources of possible fumes by operating electronic devices were investigated but could not be located. During this time, we were in contact with Dispatch via ACARS. We were advised of the weather along our route of flight at possible diversion airports. Based on this information, the First Officer and I completed the Smoke/Fumes/Avionics Smoke checklist which advised us to LAND ASAP. Dispatch was contacted and the decision was made to divert. The flight attendants were advised and the cabin was prepared for landing. An emergency was declared and we began an immediate descent. We landed and stopped on the runway. The Fire Department inspected the airplane from the outside using infrared cameras and did not find anything wrong. We then taxied to the gate and were met by ground service personnel.

During the descent into the divert airport, a guest had become boisterous and began to say things that caused concern and began to frighten our other guests. It was reported to me by our flight attendants and other guests that the guest in question made verbal comments referring to terrorists and that he believed we were all going to die as a result of the issues with the aircraft that caused us to make a diversion.

Synopsis

A320 Captain describes a diversion after electrical fumes are detected by the Flight Attendants. Inappropiate passenger behavior is also reported during the diversion.

ACN: 967236 (2384 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : MAIER 4
Airspace.Class E : P50

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 967236
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During the Maier 4 arrival, we deviated around a thunderstorm. Re-cleared the Maier 4 after direct BRUSR. First Officer (pilot flying) noticed several altitude restrictions dropped off the flightplan. We both missed putting 9,000 FT MSL back in at EDDNA. ATC queried us as we stopped the descent at 8,500 FT MSL and climbed back to 9,000 FT prior to EDDNA. Confusion over why some altitudes dropped out and did not recognize one was still missing. Less time spent on the why and more on what is missing.

Synopsis

A320 Captain reports an altitude deviation during the MAIER 4 RNAV arrival to PHX. After deviating around weather and returning to the arrival several crossing restrictions were noted to have dropped out. All but one were reinserted.

ACN: 967205 (2385 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility.Other 
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Pitot-Static System
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Cowling/Nacelle Fasteners, Latches
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14323
Experience.Flight Crew.Last 90 Days : 257
Experience.Flight Crew.Type : 4232
ASRS Report Number.Accession Number : 967205
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft was taxied to the gate at XA:40am local time by mechanics for an XB:29am departure. Crew boarded and began the normal set-up. Captain performed walk-around. On walk-around Captain noticed both forward static ports had streaked and running red paint from the circular border as well as the lettering. From aircraft knowledge, as well as the label on the [Static Port] area, Captain knew areas were supposed to be clean and smooth. The running paint seemed to violate this protocol. A rivet head was also noticed missing from the number one engine cowl. 

Per Aircraft Flight Manual procedures, both items were entered into ACARS and sent to Maintenance Control for action. Local maintenance was notified via radio. Local company Mechanic was observed to pull a stand to the left side of aircraft and clean the static port area. He then returned the stand to it's area. Since both sides [of the aircraft] were affected, the Captain went to the ramp [office] to inform the Mechanic that perhaps he had misread the write-up. The Mechanic then stated the write-ups were nonsense anyhow; (he used the longer form of nonsense, but I understand some mechanics are very sensitive to swear words so I won't repeat--although I hate to leave out facts). 

I politely informed him that our procedures require a write-up of discrepancies (which of course he knows) but he said any write-up this close to departure makes them look bad. Since they brought the plane to the gate, any earlier write-ups were of course impossible. I am not sure if they were embarrassed they missed them, but all the mechanics in the office heard the conversation and no one corrected him. I assume the Mechanic had the option to sign-off the write-ups as fraudulent, or unable to find, but instead he cleaned the areas and deferred the missing rivet, thus validating my concerns. I am once again amazed at the attitude of a professional company Mechanic. His opinion was that I should not enter very valid write-ups as to make the station look good. In the future they will find me just as uncooperative in this regard, as I prefer safety over style points.

Synopsis

An A320 Captain reported the poor attitude of a company Line Mechanic after writing up paint on the forward static ports and a missing rivet on the #1 engine cowl, having to insist proper corrective action be taken.

ACN: 967055 (2386 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : PHL.TRACON
State Reference : PA

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.TRACON : PHL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Airspace.Class B : PHL

Component

Aircraft Component : Pitot-Static System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 967055
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On short final approach we received an ECAM "Capt TAT" message during flight through bands of heavy rain. We continued and completed the landing with normal indications. Once on the ground the ECAM disappeared. I contacted Maintenance Control and wrote it up in the logbook. 

One theory of a recent overwater high altitude accident is that the TAT probe filled with water while flying through a thunderstorm and iced up, negatively impacting pitot/static systems. The heavy rain we encountered prior to landing may have done the same since we had a strong 30 KTS cross wind coming from the Captain's side, potentially filling the TAT probe with water. We were not at altitude so there was no icing potential. We experienced no obvious difference in air speed or altitude indication. 

I believe the aircraft manufacturer, the NTSB, the FAA, Maintenance and our company need to address this issue.

Synopsis

An A320 flight crew received an ECAM Capt TAT warning on short final in heavy rains. They made a safe landing after which the message disappeared. Maintenance was advised.

ACN: 966807 (2387 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 966807
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Distraction
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flying aircraft with Thales FMS Release 1A, changed runway in FMC, MCDU froze upon entry of new runway, and did not switch ILS frequencies. The other MCDU was similarly frozen. About 1 1/2 minutes elapsed until entries were accepted and actions began to occur in the MCDU (unfroze). The Thales box is non-airworthy and Thales/Airbus should be notified that this is so, and that they are responsible for the ramifications thereof.

Callback: 1

The Reporter was extremely knowledgeable about the FMGC, the various revisions, the "misbehaviors" that came with the various revisions and some work arounds for same. The anomaly which his report described still does not have an explanation, but other similar dual FMGC freezes do have work arounds such as the NO STAR freeze which results when an aircraft transitions from an arrival route to an arrival heading for a vector. The proper procedure is to select the required heading, delete the manual heading LEG, then select NO STAR after which the pilot can begin building the assigned runway approach. If a turn is made, NO STAR selected before the manual heading is deleted then a dual FMGC lockup occurs. There is a known lockup "misbehavior" on the SEA CHINS arrival when transitioning from IMB toward RADDY. Also if alternate routes are programmed into the primary route, then the secondary route, if an anomalous behavior is entered in the secondary route, a dual FMGC lockup occurs. If an aircraft flies within 5 miles of an inbound fix while being vectored outbound the FMGC locks up. A newer issue has come up in a feature called "enhanced localizer" which made a recent arrival unstable when the aircraft pitched over, descending rapidly below profile. The Reporter believes that the FMGC digital processor and bus have been overloaded resulting in the anomalies which they are experiencing now.

Synopsis

Both A320 FMGC's on an A320 with the Thales FMS Release 1A installed locked up after a landing runway change entry, but subsequently reset after about a minute and a half.

ACN: 966765 (2388 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 19000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 966765
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8500
Experience.Flight Crew.Last 90 Days : 158
Experience.Flight Crew.Type : 1600
ASRS Report Number.Accession Number : 966769

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Leg one on day one of a three day trip. Pre-departure, taxi, and takeoff [were] normal. The First Officer (FO) was hand flying the aircraft. As we passed through FL190, the Purser called and informed me the aft Flight Attendant reported smelling a strong electrical smell with no visible smoke present. I asked him to keep me informed. I advised the FO and told Center we might have a problem and needed to level off at FL210. The Purser called again at which time I gave the radios to the FO and answered. He said the smell was very strong and they were very concerned. They removed all power from the cabin. I asked him if he believes it's serious enough to divert, which [he] answered yes. I informed the FO and told him that I think we should land at ZZZ since we were directly abeam the airport; he concurred. We declared an emergency, informed Center of our issue and turned toward ZZZ. The Purser called back and said the smell is very strong and he sounded very worried. I told him we are diverting to ZZZ and will be on the ground in 5 minutes. The FO was flying the aircraft and talking to the controllers. I sent a short ACARS message to Dispatch and started focusing on where the aircraft was. 

The FO did an excellent job and had already put a new destination in the MCDU, pulled up weather, and had his charts out. I thought it would be a good idea for him to continue flying so I could catch up, which he concurred. He briefed the approach and just about then the Purser called and said the aft flight attendants were putting their protective breathing equipment on (later turned out he had said they were pulling them out and I may have miss understood). This made the situation very serious to me. I asked for the aircraft and pushed the speed up and received direct clearance for [the runway] at ZZZ. We landed uneventfully and made the turn off where the emergency vehicles were waiting. On short final, I asked the FO to contact the Purser for an update. We were informed the smell had gone away so we told the flight attendants we would not evacuate and also told the Fire Marshal of the status in the back and that we would stop by them and not evacuate unless they see or tell us other wise. All seemed normal to the Fire Marshal and no sign of heat or smoke so we taxied to gate. I asked the passengers to deplane in an orderly fashion and leave their carry on items. Every one - the flight attendants, First Officer, passengers, and ground support - were excellent.

Synopsis

A320 flight crew is informed by the Purser during climb of electrical fumes in the aft galley. Flight Attendants turn off all galley equipment and the flight crew elects to divert to the nearest suitable airport after declaring an emergency. Smoke dissipates prior to landing and aircraft is taxied to the gate.

ACN: 966722 (2389 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 966722
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction
Analyst Callback : Attempted

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Ground Event / Encounter : FOD
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

I thought I had taken out my one foot by one inch metal ruler from my flight bag that I use to cut up the flight release into manageable sections and put it on my tray table with the release on top of it. When I went to review my release my ruler was not under it on my table. When I stowed my tray table I heard a slight "clank" from behind it as it stowed and thought that maybe my ruler had slid behind my tray table. I could not see anything behind the rudder pedals, it was not in the tray table tracks and the tray table stowed without any problems. As a precaution I had station maintenance come out to check behind the rudder pedal area and the forward E & E to see if by chance it had worked its way back there. After a 40 minute thorough search by two mechanics done multiple times behind the rudder pedal area and E & E the only FOD found was a metal butter knife, ball point pen and a plastic spoon all located out of reach from the cockpit on the floor behind my rudder pedal adjustment cover. We also did a hydraulic power-on rudder pedal movement check while the mechanics checked the area under the floor and under my rudder pedal position adjustment cover verifying that the rudder pedal linkages were not fouled. It is very possible that the "clank" I heard was the metal butter knife falling to the floor after being trapped behind the tray table as it made the same sound when I dropped it into my side cockpit storage bin to get it out of the way. 

After a search of the rest of the cockpit (including looking under my seat cover just in case I had put it on the seat and it had slipped to the back of the seat under the cover) I never did find my ruler but it is possible that I it had been lost earlier because when I checked my flight bag the storage area where I kept it was open and unzipped. I also don't believe that the tray table would have stowed OK with a foot long ruler in the tracks. At any rate, after hearing the "clank" I wanted to be sure that the ruler wasn't where I didn't want it to be. After the thorough search and finding the metal butter knife I felt confident that we had found the source of the sound I had heard and that we had no other FOD issues. It was surprising the amount of FOD that was found behind the rudder pedals. It may be a good idea to periodically check that area for FOD during routine scheduled maintenance checks.

Synopsis

An A320 Captain thought he dropped a ruler as his tray table was stowed but a maintenance inspection found other FOD behind the rudder pedal covers, including a metal butter knife, a plastic spoon, and a ball point pen.

ACN: 966697 (2390 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.AGL.Single Value : 100

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 4R
Flight Phase : Final Approach
Airspace.Class B : BOS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 1800
ASRS Report Number.Accession Number : 966697
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

On approach to 4R noticed that directly below us in the harbor was a "bait/buy" boat. In a former life I worked on a crab boat. These bait or buy boats tend to attract a LOT of seagulls. I am sure that I spotted at least 20-30 gulls feeding and flying around the boat, which was directly below the approach to 4R (approx 1/8-1/4 mile from end of Runway at approximately 200-400 FT). I do not know if the Mass Port Authority or Airport Authority has any control or influence over this, but it would seem like a very dangerous situation to be intentionally attracting this many birds directly below a very busy approach path.

Synopsis

An A320 Captain reported seagulls 200 FT to 400 FT above a bait or buy boat about one-eighth to one-quarter of a mile from BOS Runway 4R. The boat and birds could pose a threat to arriving aircraft.

ACN: 966574 (2391 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Aircraft Cooling System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Mininum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Indicating and Warning - Air Conditioning and Press.
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Avionics
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 25
ASRS Report Number.Accession Number : 966574
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : MEL

Narrative: 1

AVNCS SYS FAULT [message] displayed on UPPER ECAM. Chronic Issue. Tried to implement MEL 21-26E, Deactivate Isolation (Iso) Valve. The Iso Valve (24HQ) needs to be placed in the "Open" position. But the MEL 21-26E does not tell the Aircraft Maintenance Technician (AMT) to "Remove" the cannon plug from the Iso Valve (24HQ). The Avionics Equipment Ventilation Computer (AEVC) Test must be accomplished before each flight and the override switch in the flight deck selected; this would move the Iso Valve to "Open". Per Maintenance Manual (MM) 21-26-00, Deactivation of Iso Valve (24HQ) does not match MEL 21-26E.

Callback: 1

Reporter stated if they have a short turnaround time with a AVNCS SYS FAULT displayed on the Upper ECAM and no Duct Temp Sensor Fault displayed, then Maintenance Control and the Mechanic will most likely defer the Skin Isolation (Iso) Valve (24HQ) in the "Open" position. His concern was that MEL 21-26E Maintenance Procedure only directs the Mechanic to select "Override" on the cockpit overhead panel and visually check the 24HQ Iso Valve is in the "Open' position in order to maintain avionic equipment cooling temperatures in flight. He believes the cannon plug to the 24HQ Iso Valve should be removed prior to departure to prevent any possible change to the valve position during flight.

Reporter stated many of their chronic Airbus Avionic System and Ventilation (Vent) Fault ECAM messages involving the Blower and Extract Fan lights illuminating amber, have been resolved by replacing the large and always found clogged Demister Filter, located just downstream of Skin Air Inlet Valve (15HQ). The chronic faults did not recur after the filter change. The clogged Demister Filter seems to reduce air flow in the Ventilation System supply ducts that trigger an Avionics System or Blower, or Extract Fault light and ECAM message. He suspects the same problem existed with the Demister Filter that may have contributed to the recent MEL 21-26E deferral.

Synopsis

A Line Mechanic reports a Deactivate Isolation (ISO) Valve (15HQ) Maintenance Procedure in MEL 21-26E for the A320 should include disconnecting the cannon plug to the same valve in order to assure the ISO Valve remains in the "Open" position in flight.

ACN: 966353 (2392 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 35L
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 966353
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Prior to the FAF we lowered the landing gear. We had no green lights or ECAM indicating that the gear was down and locked, however it "sounded" as though the gear was down. We executed a go around at approximately 1,500 FT AGL. Earlier in the approach we flew through moderate rain-producing clouds which generated excessive static electrical discharges. We were clear of clouds and rain approximately 5 miles outside of FAF in VMC. We flew north of the airport to work the problem. We first consulted the aircraft FM and found no procedure addressing the problem of no gear down indication. We then, through ACARS, asked Dispatch to call us to discuss our situation with them along with Maintenance Control. However, after numerous attempts we were unable to make contact with them. After debating all options we decided to declare an emergency, and return lowering landing gear normally and make a normal approach and landing. When we lowered the gear outside the FAF, all indications were normal. Landing was uneventful. Maintenance believes that we probably had a lightning strike and that we had momentarily lost our 1 LGCIU.

Synopsis

A320 pilot reports no green gear indications or ECAM indications after gear extension during approach. After trouble shooting a second approach is accomplished and the gear indicates normally. Flight flew through clouds which generated excessive static electrical discharges during the first approach.

ACN: 966340 (2393 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 4200
ASRS Report Number.Accession Number : 966340
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We received a late plane change. I was told this aircraft just came over from the hangar, where it had the #2 engine changed. This was its first flight. We were prepared for a flex takeoff and did all the necessary procedures, including loading the flex temp in the FMGC. When cleared for takeoff, I advanced the throttles to 1.05 EPR and both engines stabilized normally. Upon advancing the throttles to the flex detent, I read out MAN FLEX on the FMA. Simultaneously, I felt the aircraft pull right of centerline. Something felt wrong. I was correcting with left rudder when we got an ECAM. I aborted at approximately 60 KTS. I believe the ECAM was, "ENG - THR Levers Not Set". However, it disappeared when I pulled the throttles back to idle. It would not recall. We coordinated a return to the gate after working with Maintenance. The #2 engine did not make takeoff thrust or was slow to spool up. The FMGC, throttle position and FMAs were correct for a flex takeoff.

Synopsis

On the first takeoff after an engine change the new engine on an A320 failed to accelerate to takeoff thrust, the takeoff was rejected and the aircraft returned to the gate for maintenance.

ACN: 966335 (2394 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 29000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 966335

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 966352

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was flying with the autopilot engaged on our way to FL340. Passing FL290 the autopilot kicked off. I tried to reengage the autopilot with no success. At that time we got an ECAM NAV ADR 1 Fault, Autopilot Off and GPWS Fault. The Captain instructed me to fly the airplane and talk to ATC while he ran the ECAM. After finishing the ECAM and running through the appropriate procedures, we had lost numerous systems necessary for auto flight. I now was hand flying the aircraft. The Captain discussed the situation with Dispatch and Maintenance and after working through the appropriate procedures decided to divert. He exercised Captain's Emergency Authority to recycle FMGC 1 and 2 Circuit Breaker. Despite running all appropriate procedures we were unable to return the aircraft to normal flight. We did not declare an emergency with ATC. We descended [below RVSM] and flew block altitudes to the divert airport. We also burned down our fuel in order to land under max landing weight. The landing was uneventful. After arriving at the gate, we debriefed with Operations and Maintenance.

Narrative: 2

We determined we had lost auto pilots, auto throttles, flight directors and autoflight guidance as well as airspeed indications.

Synopsis

After suffering multiple autoflight and instrument failures the flight crew of an A320 descended out of RVSM airspace and diverted uneventfully for maintenance.

ACN: 966143 (2395 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Weather Elements / Visibility.Other 
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 966143

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent from FL380 to FL360 as engine came back at start of descent heard loud thump thump thump followed by ECAM engine 1 stall. EPR went to zero, ignition came on and engine recovered on its own. Followed up with engine 1 stall procedures and continued to destination with no other indications. Maintenance met aircraft and gave them engine stall report. No additional help asked for from ATC nor given.

Synopsis

A320 Captain reports compressor stall at start of descent with automatic relight and recovery. No further problems encountered.

ACN: 966015 (2396 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Door
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 966015
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

All aircraft slides were confirmed "Disarmed" per Standard Operation Procedures (SOP) at post flight, after block-in and engine shutdown in ZZZ. Approximately 20-minutes later, the First Officer performed a post-flight walk around with no discrepancies noted. A few minutes later I noticed several of the Ground crew were in the aft galley and aft few rows of the aircraft, instead of spread out, cleaning and doing their security search. 

The Shift Manager approached me in the cockpit seeming very upset. She said something about the slide and was surprised I had not been told the Door-4R (Right) slide had been deployed. During this whole time my entire flight crew was in the front of the aircraft (first class or cockpit) so none of us witnessed the event. When I went back to Door-4R, I found the door secured in the open position, the slide completely deployed, and the Arm handle in the "Unarmed" position. The slide was removed by Maintenance and moved to the aft cargo compartment. 

The flight was canceled, we received a new Release and Ferry Permit and ferried the aircraft to ZZZ1 with only the crew on board. It is unclear to me, since none of my crew witnessed the event, why it occurred. It could have been human error or some type of mechanical failure with the door and/or slide mechanism. I will probably never be told.

Synopsis

An A320 Captain reports after all aircraft slides were confirmed "Dis-Armed" after block-in and engine shutdown, a Shift Manager informed him the Door-4Right slide had deployed. Captain notes slide was fully deployed and the Arm handle in the "Unarmed" position. Cause unknown. Flight was canceled and aircraft ferried for slide replacement.

ACN: 965973 (2397 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : EICAS/EAD/ECAM
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Powerplant Fuel Control Unit
Manufacturer : Aerospatiale
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : RPM/N1/N2/Etc Indication
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 965973
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

Extended delay due to number 2 Engine Overspeed ECAM [message]. We have a company bulletin for this issue. I am concerned that it was assumed, this is an Airbus glitch, and not a real problem. It appears that one of the solutions, (six start attempts), is beyond the scope of the Flight Manual (FM) limits without proper cooling time[for the engine starter]. In the Maintenance history, the aircraft had displayed the fault and was cleared. Then on the inbound flight the crew reported the same fault. Contract Maintenance cleared the fault and the flight attempted to operate. As soon as number 1 Engine Start Switch was placed to "run," we got the number 2 engine Overspeed message. After number 1 engine stabilized and we contacted Maintenance Control, the message cleared.

Maintenance Control noted the problem, which broke [voided] the Maintenance Release (M/R) and then they issued a new M/R. As soon as we started number 2 engine, the number 2 Engine Overspeed ECAM message reappeared. Called Maintenance Control again and they advised to return to gate. Mechanics and parts were flown in on the XA:30 pm flight and apparently they first tried to clear the software issue, then changed the engine Fuel Management Unit (FMU). The problem did not clear. Line Maintenance indicates you need to start the engine three times for the message to unlock. Word of mouth in the mechanic group indicates it requires as many as six starts. On start number 5 the problem cleared. Apparently there has been a component change/upgrade and there is a conflict in the software that Airbus has not been able to solve.

Callback: 1

Reporter stated he has experienced numerous delays and some cancellations due to the Engine Overspeed ECAM messages reappearing. Multiple reset of Circuit Breakers (C/Bs) does not clear the messages. When attempting to clear the Overspeed messages after an FMU change, Mechanics have been shutting off the Inertial Reference Units (IRUs) after each engine start to clear the Air Data Computer (ADC), as if the aircraft had just completed a flight. 

Reporter stated there seems to be an interface problem with the engine Fuel Management Unit (FMU) and aircraft systems. He has noticed the starter cool down time required between multiple engine starts is not being followed. The ECAM Engine Overspeed message does not seem to reappear after the FMUs are changed and the ECAM message is "Unlocked."

Synopsis

An A320 crew has an extended departure delay due to a recurring number 2 engine overspeed ECAM message. Efforts to "clear" the software issue were not successful, resulting in a number 2 Fuel Management Unit (FMU) change.

ACN: 965688 (2398 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : DEN.Tower
State Reference : CO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 965688
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9500
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 3400
ASRS Report Number.Accession Number : 965692

Events

Anomaly.ATC Issue : All Types
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Weather
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

On departure roll on 34L, at about 110 KIAS, DEN Tower canceled our take off clearance after they detected a vehicle on the runway. They detected this through their ground RADAR, and it indicated a vehicle entering the runway at, I believe, Taxiway WC. I aborted the takeoff and cleared the runway joining the Eastbound parallel and held short of D1 Taxiway. We told the passengers to remain seated, and shutdown our engines. Airport operations first showed, then the fire department. The fire department chocked our nose gear, so we could release our brakes. They did their inspection and everything checked out okay. While all this was happening I also communicated with flight Dispatch, informed them of the incident and got Maintenance Control in the loop. They dispatched a Team to inspect my tires, and eventually towed me in. Hottest brake temp was about 710 degrees. My First Officer talked to Tower while I was communicating with others and found out the vehicle incident was a false alarm. I'm sure there were things I could have done better, but overall the incident worked out well with the help I got from all parties involved.

Narrative: 2

I was the First Officer on a flight from Denver. The weather was VFR but deteriorating due to an incoming gust front and rain. We were cleared to take off from Tower on Runway 34L. At 110 KTS Denver Tower said "Aircraft X take off clearance canceled." The Captain initiated an abort per SOP. I told Denver Tower, "Aircraft X aborting on Runway 34L." When below 60 KTS and everything seemed under control, I told the crew and passengers on the PA to remain seated as we were beginning to taxi off the runway and we did not want the crew prematurely opening the doors and slides as we came to a stop. The Captain taxied off the runway and came to a stop. As he spoke to the crew and passengers I coordinated initially with Ground and Maintenance for overheating brakes. Max temperature of brakes reached 660/630 left and 680/705 right brakes. We were 148,000 LBS and the outside temperature was 29C. An emergency was never declared as it was deemed unnecessary. Denver Tower was professional and somewhat apologetic because they canceled the take off clearance due to an alarm indicating vehicles on the runway. This was a false alarm and they indicated they could not verify vehicles on the runway because of the gust front and approaching rain reducing their visibility. We told them better safe than sorry, basically they did their job and we did ours.

Synopsis

Air Carrier on departure role from DEN was issued a "cancel take off clearance" by ATC at about 110 KTS. After the abort ATC indicated that a "false alarm" on their equipment dictated the action taken.

ACN: 965490 (2399 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Takeoff / Launch
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 965490
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Automation : Air Traffic Control
Result.Flight Crew : Rejected Takeoff
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Aircraft X, A320, departing Runway XXL, very light rain, ASDE-X in normal operations mode. As Aircraft X approached V1, a safety logic alert was generated indicating that the runway was occupied. ASDE-X showed an uncorrelated primary target on the West side of Runway XXL at intersection WC as Aircraft X was approaching intersection WB accelerating towards the target...both Aircraft X and the uncorrelated primary target at WC were encircled in red indicating the approaching conflict and an aural runway occupied warning was generated. Initial visual scans could find no vehicle or other obstruction corresponding to the target at WC. Local Control canceled the take off clearance of Aircraft X and advised of indicated traffic and Aircraft X aborted using what appeared to be max brakes to stop. CFR (crash fire rescue) response was required due to extreme brake temps and aircraft required tow back to gate. Brake temps were reported by pilot 5 minutes after stopping as exceeding 700 degrees and pilot indicated the wheel pressure plugs would have blown at 900. For approximately 10 minutes after the initial alert, ASDE-X continued to paint multiple false primary targets on the West airport runways/taxiways with the worst conditions being on/near the departure end of Runway XXR. At one time 4 simultaneous false primaries were displayed, on the runway and connecting taxiways, at the departure end of Runway XXR. ASDE-X was placed in both rain mode then limited mode with no effect on mitigating the false targets. Airport operations personnel checked both Runways (XXL and XXR) and found nothing that would/should generate primary targets on either runway. A subsequent channel change seemed to resolve the display of false primaries...but someone needs to ensure a thorough investigation gets done to account for what happened and what can be done to ensure it doesn't happen again. This is not the first incidence of false targets being generated by ASDE-X, on the airport runways/taxiways but certainly the most serious. Locally, hundreds of ASDE-X trouble reports have filed by controllers documenting false targets on the airport's taxiways/runways since the system was installed/certified. An in-depth analysis of the system factors that caused this false alert needs to take place. After hundreds of trouble reports by controllers pertaining to false targets, most controllers have quit filling them out out of frustration that nothing ever comes of them. The reduced incidence of reports over the past months is more an indication of controller frustration with lack of action than an indication that incidence of false targets have diminished. Early belief by facility staff personnel was that the false targets were being generated by the differing heights of the 2 ASDE antennas. Additionally, concern was also expressed over the choice to use existing legacy ASDE antennas rather than replacement with more compatible ASDE antennas. We were told that both of these issues can and have been the cause of false targets at other airports. We were told that addressing the problem via adjusting the 2 antenna's heights to comparable levels or replacement with antennas was cost prohibitive. Regardless of the cause, the problem still exists and resulted in potential damage to an aircraft this evening and the inability to use another needed arrival/departure runway due to continued false targets.

Synopsis

Tower Controller described a high speed ATC issued abort because of an ASDE-X false alert, noting multiple false alert reports have been filed with no action on the part of management.

ACN: 964938 (2400 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SFO.Tower
State Reference : CA
Altitude.MSL.Single Value : 1600

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Route In Use.SID : SFO 8
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 200 ER/LR (CRJ200)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Route In Use.SID : PORTE 3
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Facility : SFO.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 964938
Human Factors : Confusion

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

An A320 departed Runway 1R on the SFO8 SID-heading 030, a CRJ2 departed Runway 1L on the Porte3 SID-intercept SFO350R; when upwind, Local Control transferred their communications to NorCal Departure. Both aircraft appeared to be turning appropriately while this was taking place. A conversation was happening on the other side of Local Control, when I looked back the A320 aircraft appeared to be flying the Cuit2 SID-SFO011R to 4 DME then would be a left turn into the CRJ2. I called the CI1 position and advised them, they in turn advised the Sector Controller to turn the A320 right. At this time, the A320 was at about 3 DME. The flight plan had a preferential departure route of: +SFO8 SFO.LIN JSICAâ€¦, while the company had filed: KSFO CUIT2 LIN Q128... I think it is entirely possible that the crew misread their clearance. Recommendation, find a way to eliminate the possibility of misreading the Pre-Departure Clearances, although they are a very valuable tool for reducing frequency congestion.

Synopsis

SFO Controller described a near conflict event when traffic departing Runway 1R started a wrong turn toward a Runway 1L aircraft, allegedly due to mis-reading the PDC clearance.