ACN: 964609 (2401 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Landing Light
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 964609
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

I was the Lead Mechanic [working] outside on an A320 aircraft which needed a left inboard landing light assembly that was found on a walk-around by the Flight crew. Another Lead Mechanic ordered the part and a Mechanic changed the part. To help with a on-time departure, I accomplished the [Maintenance] paperwork and entered the information into the Logbook. After the aircraft had departed, we found the aircraft "Effectivity" was for a different series A320 aircraft.

Prior to entering the information into the computer I was checking Serial Number (S/N) information and found that the Part Number (P/N) installed did not match the removed Part Number that had been installed. The wrong part was ordered and I did not double-check and verify that the part was correct. Time was also a factor, trying to make sure that the aircraft left on time. I also relied on my co-workers to order and install the correct part. 

Notified Maintenance Control, the Supervisor On Duty and spoke with the other Lead Mechanic and Mechanic that were involved. Maintenance Control reported that they would put [light assembly] on another flight. 

Prior to the aircraft leaving, all paper work should be completed. Double-check the aircraft effectivity, slow down and don't be in such a rush to release the plane.

Synopsis

A Line Lead Mechanic realizes after their A320 aircraft had departed, he had given a Mechanic a left inboard Landing Light assembly to install that was not "Effective" for the aircraft. Trying to make sure the flight left on time, he failed to notice the Landing Light Part Numbers (P/N) did not match.

ACN: 964519 (2402 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Fuel Booster Pump
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : DC Regulator
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Check Pilot
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 964519
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10900
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 4975
ASRS Report Number.Accession Number : 964526
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We departed with 30,000 LBS total fuel on board and 2,700 LBS in the center tank. We were dispatched in accordance with MEL XXXX (center #2 boost pump inoperative). After takeoff, we noticed that the center #1 boost pump failed to start feeding the engines. We monitored the fuel system and referenced the flight manual. The fuel panel configuration from the MEL compliance was XFEED-ON after takeoff. This created a fuel imbalance and subsequent fuel auto feed fault ECAM (even though the center tank was not feeding). Accomplishing the auto feed fault ECAM, we selected fuel mode select to manual. This started the center #1 pump feeding to the engines. At this point we were going to call Dispatch and Maintenance to conference, but we then got a BCL 2 (Battery Control Limiter) fault ECAM. At this point we were at FL360 and elected to divert. We declared an emergency and landed overweight (146,400 LBS). The diversion was uneventful. Upon conferencing with Maintenance it was determined that the underlying problem was a fuel auto feed system malfunction. Since we didn't need center tank fuel to continue to our destination we accepted the additional deferral for fuel auto feed fault and continued. The divert station folks, supervisors, customer service representatives, mechanics, and fueler could not have provided more superior service for our diversion. They are all to be commended.

Synopsis

An A320's #2 center fuel tank boost pump was MEL'd and after takeoff the crew had difficulty starting the #1 Center Fuel Boost pump, so they declared an emergency and diverted. While troubleshooting the fuel problem the BCL2 ECAM alerted.

ACN: 964399 (2403 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 215
Experience.Flight Crew.Type : 1580
ASRS Report Number.Accession Number : 964399
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17400
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 1050
ASRS Report Number.Accession Number : 964388
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Departed gate on time. Normal taxi with no abnormal brake indications noted. The Captain had briefed an abort in the low speed regime for anything prior to departure. We were cleared for takeoff and the controls were transferred to me as we lined up and I applied FLEX Take Off power. At approximately 75 KTS, an ECAM and Master Caution light came on. I looked down briefly and saw "BRAKES HOT" ECAM. At the same time, the Captain announced, "aborting" and retarded the power levers to idle. RTO braking was engaged and the aircraft quickly came to a stop. The Captain was unable to disengage the autobrakes with the brake pedals and had to de-select the autobrake switch. I advised ATC when the abort began and the Captain announced "Remain Seated" on the PA as the aircraft came to a stop. We exited the runway and requested the emergency equipment to come to inspect our brakes. We were given taxi instructions to cross Runway XX and enter the XX hold pad. The Captain held the brakes and we shut down the engines. The Fire Department inspected the brakes and their readings concurred with our indicated brake temperatures, which peaked at 605 C on the #1 brake and was above 500 C on the others. We talked to Dispatch on and patched through to Maintenance. After the brakes had cooled we arranged for a gate to return to and taxied back to a gate with the fire department following. At the gate, we had the aircraft chocked and turned the aircraft over to Maintenance for inspection. We were given another aircraft and continued the flight uneventfully. I don't believe significant human factors were involved.

Narrative: 2

Spoke to flight attendants and found out they have different evacuation terminology and a bit of a gray area with what they are to observe and if they need to stay seated or stand up and assess the situation. In this case, it was better for them to be seated due to movement of the aircraft off the active runway.

Synopsis

An A320 ECAM alerted "BRAKES HOT" at 75 KTS on the takeoff roll so the Captain rejected the takeoff and requested ARFF inspection before taxiing to the gate. Flight Attendants stated that cabin crew procedures are different from flight crew's.

ACN: 964187 (2404 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 2500

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase.Other

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Malfunctioning
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 964187
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

While flying the intermediate approach segment of the ILS, ATC discontinued our approach and issued non standard missed approach instructions. At 2500 ft and prior to the final approach fix, ATC issued instructions to "turn 30 deg right and climb/maintain 4000". The First Officer elected to use "open climb" to 4000 and use heading select to turn the 30 deg right. Unlike the expected "open climb" response, the airplane pitched up to approx 15 deg without adding power. The airspeed rapidly decayed and Alpha Floor/Toga Lock engaged based on the extended trend arrow causing the autopilot to increase pitch further to approx 17.5 up. Airspeed rapidly decayed further to approx 120 knots as the engines increased to full power. To emphasize the quickness of this occurrence , the First Officer was just making his "go around flaps" callout to which I responded "no, not at 120 knots" and advised him of Autothrust being in TOGA lock. He reached up, placed the thrust levers in the TOGA DETENT and began to disengage the auto thrust. Uncomfortable with the high pitch and low airspeed condition and the time involved with issuing instructions I called "My Aircraft,", and lowered the pitch to increase airspeed. With airspeed increasing through approx 180 knots I called out flaps 1, up etc. While turning to the assigned heading and climbing to the assigned altitude. The First Officer indicated he was ready to resume flying duties. I transferred control to the First Officer who at that point when attempting to adjust his map display received the "set offside range" message indicating an FMGC conflict. Autopilot 2 had remained on throughout the previous events. We "synced" maps, switched to autopilot 1 and continued to a normal landing. Checking at the gate indicated status "normal" and both FMGC's appeared to be operating normally.

The aircraft ahead (a B737) slowed to a speed below our final approach speed creating the need for a go around. I can only guess that perhaps the actual speed may have been a few knots above the bug when open climb was pulled. Thus delaying the Autothrust response until it was too late or perhaps the event occurred at the moment of the FMGC hiccup. I'm unsure what to add here. With varying go around procedures for ILS, ILS PRM escape, and two additional RNAV Go Arounds, I could not fault the First Officer for choosing open climb from an ILS while still outside the marker for small turn and climb. I agree It would not have resequenced the waypoints as desired but I never expected a near stall event.

Synopsis

A320 Captain reports autopilot pitch up, airspeed loss, and Toga Lock during attempted go around at ATC direction. Pilot flying is slow to notice the airspeed loss and Captain assumes control for the recovery. One FMGC locks up during the recovery but all systems appear normal at the gate after landing.

ACN: 964013 (2405 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 964013

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 964419

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were planning a flex takeoff with an assumed temperature of 38 degrees Celsius that was supposed to produce 1.36 EPR. Flaps 1+F. Once aligned with the runway I handed control of the airplane to the First Officer. He spooled the engines to 1.05 EPR and then advanced the thrust levers to the flex detent. I then put my hand on the thrust levers and made sure they were firmly in the proper detent. #2 engine came up to speed properly. I noticed #1 engine wouldn't accelerate past 1.25 or 1.26 EPR. I assumed control of the airplane and rejected the takeoff. We were in the low speed regime and the autobrakes never activated and we had to disarm them manually so the entire maneuver was completed below 72 KTS. We cleared the runway and returned to the gate.

I have no idea why the engine did not produce takeoff thrust. Having said that, I wasn't going to wait and see what would happen, risk accelerating into the high speed regime, and then rejecting the takeoff. In my opinion we did the safest thing we could. Under the same circumstances, I would do the same thing.

Narrative: 2

Once aligned with runway I received controls from Captain. I spooled the engines to 1.05 EPR and then advanced the thrust levers to the flex detent. I removed my hand from the thrust levers noted the power was coming up and moved my eyes outside the aircraft to continue with the takeoff. The Captain then rejected the takeoff and I handed controls of the aircraft to him. We exited the runway after the low speed reject and he explained that #1 engine was slow spooling up and never reached takeoff power (1.26 EPR). Auto brake never actuated. Contacted Maintenance and returned to gate. Brake temperatures were less than 300 C.

Synopsis

A320 flight crew experiences low EPR on the number one engine during attempted takeoff and rejects at low speed.

ACN: 963763 (2406 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZSU.ARTCC
State Reference : PR
Altitude.MSL.Single Value : 3000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Vectors
Airspace.Class E : ZSU

Aircraft : 2

Reference : Y
Make Model Name : Citation II S2/Bravo (C550)
Crew Size.Number Of Crew : 2
Flight Plan : VFR
Flight Phase : Initial Climb
Airspace.Class E : ZSU

Person

Reference : 1
Location Of Person.Facility : ZSU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 963763
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While [I was] working the R-1/R-5 position. An A320 was inbound to TJSJ on a 150 heading, leveled at 030 FT, expecting the clearance for the ILS8. Mean while VFR traffic, a Citation from the TJIG airport, departed westbound requesting an IFR clearance into MDJB. I was busy with the multiple departures and inbounds, the position should have been split, but as it has been usual for the past months to be short on controllers, and so it was combined for about 30 minutes. I heard the Citation called me on the frequency, instantly I replied to maintain VFR at or below 025, I glanced at the TJIG area and saw a VFR target moving pretty fast, so I knew it was him. I was still busy and kept working. A few seconds later I noticed the VFR traffic too close to the A320 and at 030. I gave a traffic call to the A320, which should have been a traffic alert but it took me a second to think about the correct phraseology and noticed that it was too late. The A320 didn't answer; I noticed he climbed to 035. 

I kept working, when the A320 told me he responded to a RA, I told him that I gave him the traffic call but he didn't answer, he was probably already responding to the RA. There where multiple contributing factors to this situation. First the two positions where combined and it was busy. Second, the TJIG Contract Tower was closed a few days ago, therefore those pilots depart VFR and pick up IFR in the air, which is a risky thing to do considering how close the final approach courses for both TJSJ runways are from the TJIG airport. Even though is risky, they run out of options since our frequency cannot hear them when they are on the ground so we can issue and IFR clearance prior departure. And third, I was constantly coordinating with the TJSJ Tower, which made the situation a little more complex. It is imperative that TJIG is opened as soon as possible. There are NOTAMs restricting VFR aircraft at or below 020 when heading westbound out of the TJIG airport, but not all of the pilots comply.

Synopsis

ZSU Controller described a TCAS RA event involving VFR traffic departing TJIG and and IFR traffic inbound to TJSJ, the reporter noting combined positions, coordination requirements and VFR departures as causal factors.

ACN: 963634 (2407 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR

Component

Aircraft Component : Emergency Exit
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 963634

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Routine Inspection

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During our preflight inspection we found the EMERGENCY OVERWING ESCAPE SLIDE PINS PAINTED OVER. We assume this occurred when the aircraft was painted in the new livery. [Because this is a fleet wide project we should] have the entire fleet checked for compliance with the relevant and previously issued bulletins and engineering orders.

Synopsis

An A320 Flight Crew discovered overwing emergency exit pins had been painted over in non-compliance with previously issued bulletins and engineering orders. Reporter cautioned the fault may be widespread as the entire fleet is undergoing repainting.

ACN: 963430 (2408 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Cockpit Furnishing
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 963430
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

A320 aircraft had deferred item assigned. Item stated plastic trim panel on First Officer's forward instrument panel was incorrect. Panel had been previously replaced due to damage on edge. New panel was removed. A replacement panel had been researched and ordered through Operations. When I received the panel, I verified through Illustrated Parts Catalog (IPC) that this new panel was correct for this aircraft. There were no references in IPC Chapter 25-13-01 as to this panel being a Pre or Post Service Bulletin panel. Panel was installed. Flight Crew on subsequent flight reported that this was an incorrect configuration for this particular aircraft. Panel was modified, and original deferral reopened. Awaiting Engineering assist to locate correct panel for this aircraft.

Synopsis

A Line Mechanic reports difficulty determining the correct plastic trim panel replacement for a First Officer's Forward Instrument panel on an A320 aircraft. No references in their Illustrated Parts Catalog (IPC) Chapter 25-13-01 indicated whether the panel was a Pre or Post Service Bulletin panel.

ACN: 963273 (2409 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 19500

Environment

Flight Conditions : VMC
Light : Night
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 963273
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 963278
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the normal climbout, we experienced an ECAM involving the loss of the Green engine driven hydraulic pump with no apparent fluid loss. The Yellow system through the PTU kept the Green system pressurized normally. The First Officer was the pilot flying through the event as I handled the ECAM and coordinated contact events with Dispatch and flight attendants. I declared an emergency and coordinated a return to the departure airport. The First Officer landed uneventfully. Taxied to the gate was uneventful.

Synopsis

An A320 ECAM alerted G HYD Press Low after the engine driven pump failed during climb so an emergency was declared and the aircraft returned to the departure airport for a normal approach and landing.

ACN: 963265 (2410 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12400
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 1776
ASRS Report Number.Accession Number : 963265
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 963267

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

A normal departure and climb out to FL370 was performed. At FL370 an AIR ENG 1 BLEED FAULT ECAM occurred. The A320 ECAM and then FM Irregular procedure was followed. One minute later ENG 2 FIRE LOOP A FAULT ECAM occurred which was noted as part of a maintenance deferral and the ECAM was cleared. Two minutes after that an AIR ENG 2 BLEED FAULT ECAM occurred resulting in both engine bleeds faulted/closed and the cabin pressurization compromised. An immediate descent was requested from Center requiring a 30 degree right turn for traffic separation and an initial descent clearance to FL320. The cabin altitude passed 9,500 FT and Engine 1 Bleed was cycled to attempt to restore cabin pressure and was successful. Flight leveled at FL320 and continued to destination without further issues other than further ENG 2 FIRE LOOP A FAULT ECAM occurrences per the deferral item. Log items were sent to Maintenance and a phone call to describe the fault sequence was made following arrival.

Synopsis

A320 flight crew experiences an AIR ENG 1 BLEED FAULT ECAM at FL370. Three minutes later an AIR ENG 2 BLEED FAULT ECAM occurred resulting in both engine bleeds faulted/closed and the cabin pressurization compromised. During descent cabin altitude reaches 9,500 FT and the number one engine bleed is restored successfully, restoring pressurization and the flight continues to destination.

ACN: 962784 (2411 of 2908)

Time / Day

Date : 201108
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 962784
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Physiological - Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Detector.Person : Flight Attendant
Detector.Person : Passenger
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

While at FL350 cruise, flight attendants notified flight deck of a suspicious acrid/burning sulphuric type fumes/odor in cabin emanating from row 10 to aft of the aircraft. Passengers were also aware of odor and were voicing complaints and concerns to cabin crew. We responded to cabin crew to keep us informed if it got worse. We first attempted to mitigate/dissipate smell by switching to hi on packs selector with no noticeable results. We then attempted to isolate the source by alternately operating left and right air conditioning packs with no results. Smell continued to get worse according to cabin/passengers (although we had no detection or smell in cockpit). We then elected to accomplish fumes/smoke QRH. Since passengers were complaining and concerned (as well as flight attendants) we elected to divert to a nearby airport.

We did not declare emergency to ATC, however, we requested priority handling with a description of cabin fumes/odor issue. We accomplished the "test" issues with cabin crew and accomplished a normal (but over weight) landing and proceeded to assigned gate. Station personnel were waiting for us. I had the passengers remain on board until conferring with dispatch and maintenance control, then arranged with station personnel to allow passengers into gate area for possible rebooking. I also arranged for medical personnel to be at gate and notified passengers that if anyone was feeling ill that they should inform EMT personnel. All 3 flight attendants were feeling ill and after conferring with the EMT personnel, elected to go to hospital to be checked out, as well as some of the passengers. Cabin crew was admitted and subsequently discharged. I arranged to have them RON. Captain and First Officer suffered no ill effects except for a continuous low-grade head ache for me (Captain) that has lasted through the next day. All information documented in the maintenance log as well as cabin air safety report was faxed to Maintenance Control. 

There is indeed something that cause the fumes/odor, as they somewhat dissipated when thrust levers/power retarded for descent. I suspect either a seal/o-ring in engines or air cycle machines. At no time was smell/odor detectable on flight deck.

Callback: 1

The Reporter stated that the flight attendants in the aft cabin as well as some passengers complained of eye irritation, nasal/airway irritation and breathing difficulty. The smell was described a sulfuric in nature, and acrid as when a match head is struck during lighting. After the flight attendants third call the Reporter realized that the flight crew could not control the irritants entry and so decided to divert. 

At no point did the smell enter the cockpit but the Reporter did experience a low grade headache for two days after this event and he attributed it to whatever the chemical was because he virtually never gets headaches. He is aware of similar events on the B757 and the Airbus aircraft but his air carrier attributed the events to job actions and safety slow downs not mechanical malfunctions. The Reporter is now a firm believer that this event was real and caused by something originating in the aircraft. He has an A&P license and believes that whatever is causing this type event is a systemic issue which must be addressed before someone dies. All of the flight attendants are back at work with no apparent long term effects but Reporter states that is not the case with another crew which experienced a similar event and has been off work for months.

Synopsis

An A320 diverted because a sulfuric type odor in the cabin made flight attendants and passengers ill. Some required medical attention.

ACN: 962757 (2412 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10900
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7200
ASRS Report Number.Accession Number : 962757
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While in cruise, Purser called to report an odd noise coming from the overhead panel around the bulkhead between 1st class and economy. A few minutes later the Purser called and reported that one of the aft flight attendants reported smelling "sort of an electrical burning odor". Through the course of several calls from the Purser it was established that 2 flight attendants and a space available ground service employee all smelled the odor, and that it was moving further aft throughout aircraft. The Captain determined, and I concurred, the best course was to declare an emergency and land [at the nearest suitable airport]. The Captain was still flying, and I called and declared emergency and asked direct to [the nearest suitable airport]. ATC immediately gave us a left turn direct to the airport, which the Captain via the autopilot started. In the turn the Captain transferred control of the aircraft to me, and he proceeded to coordinate with other parties. ATC gave us pilot discretion to FL200 then FL100 then controlled our descent after that. During the descent, the Captain and I briefed each other where we were at with the cabin, dispatch, and Center and the Captain re-assumed control of the aircraft as pilot flying. We continued descent into the airport receiving clearance direct to the 8 mile fix off the ILS flew the ILS and landed uneventfully. After landing, as there was no visible smoke or sign of immediate danger, we proceeded to a parking area and had passengers deplaned via an air stair.

Synopsis

A320 First Officer is informed by the Purser of a strange noise in the overhead and an electrical odor. After some discussion it is decided to divert to the nearest suitable airport. An uneventful landing ensues.

ACN: 962540 (2413 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 962540
Human Factors : Physiological - Other
Human Factors : Distraction
Human Factors : Time Pressure
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I arrived at the airport with my First Officer one hour prior to departure time. Flight was progressing on ground normally. There was a small maintenance item on the arming of over-wing exit slide which was tended to. Flight left 17 minutes late out of gate. Climb out and cruise was normal. At approximately 2 hours into flight we both had our crew meals. The First Officer had the Ravioli, and I had the Teriyaki Steak dinner. Within 15 minutes of eating the meal, I started experiencing the onset of an allergic reaction. My eyes began to itch, my nose started to run, and my hands began to itch. The rate at which these symptoms appeared was familiar to me as I am severely allergic to certain food items (peanuts, pine nuts, sesame seeds, pumpkin seeds, etc.). Within 30 minutes of eating the crew meal, my tongue had swollen to the point where I was not able to speak understandably; my eyes had swollen to the point where my vision was significantly restricted; my breathing was labored. I am very familiar with my own reaction to these allergies and knew at that point I would be unable to perform my duties as the pilot flying and PIC. Fifteen minutes later I transferred flying duties and PIC duties to my First Officer. Fortunately, we had a jumpseater in the observer seat to aid the First Officer with altitude verifications and radio calls. We plugged him in with his headset from the observer's seat. At this point, I had no choice but to take 75 mgs of Benydryl to help lessen the effects of the allergic reaction. I also retrieved my Epi-Pen from my suitcase. I instructed the jump-seater on how to administer the Epi-Pen "just in case." During descent into our scheduled destination, I sent an ACARS message to Dispatch instructing them [to] contact Scheduling for replacement. They contacted my wife to pick me up from the airport. Upon arrival, I contacted the Duty Officer to relay the incident. I then proceeded immediately to the Emergency Room. There I received additional medication to counteract the allergic reaction and was released 3 hours later. Follow up treatment is steroids for 5 days with Benydryl. I ingested food that I am allergic too. Post a memo alerting flight crews about the possibility of being served crew meals that may cause allergic reactions.

Synopsis

An Air Carrier Captain described an incapacitating allergic (anaphylactic) reaction to his crew meal during flight which required the First Officer and Jumpseater to coordinate the remainder of the flight.

ACN: 962499 (2414 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 962499
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

When tug driver called for pushback he told me he was ready to go. [I] asked him if predeparture check was complete, if tow bar was secure, if bypass pin was installed, then proceeded with pushback. Need more training, supervision, checklist cards in tugs for pushback crews in this station; very infrequently, is it done properly.

Synopsis

A320 Captain experiences improper phraseology by the tug driver when announcing that he is ready for pushback.

ACN: 962388 (2415 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Spoiler System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Ground Spoiler
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Avionics
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 962388
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
Detector.Person : Maintenance
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

Mechanic Y and I were asked to go on a Field Service to ZZZ1 for an A320 aircraft due to a [Flight] Crew refusal for deferred inboard spoilers. After arrival, I asked Mechanic Y to reactivate the spoilers to [perform] ops check on them. He seemed to be confused on the procedure even though he had the proper Maintenance Manual. I am the Avionics (Electrician) and he is the Mechanic. After a while he seemed confident the spoilers were reactivated. I swapped in a new Spoiler Computer to see if that would fix it, it did not. I advised Mechanic X the Flight Fault was the right #1 spoiler actuator. He proceeded to change it. 

The indication in the cockpit got better, but was not in accordance with Maintenance Manual. Mechanic Y at this point wanted to get back to ZZZ on the next flight. With the aid of the FBO Mechanic, they proceeded to deactivate the left and right #1 Spoilers and re-defer the spoilers. I was packing up tools, parts and starting to clean the area up. With the aircraft released, we flew the same aircraft with the spoiler problem back to ZZZ. On approach into ZZZ, I first noticed a vibration in the First Class area. The vibration got worse and the aircraft was shaking. I was seated by the galley and could hear the equipment shake also. One Flight Attendant described it like a landing roll out, when the whole aircraft shakes with reversers deployed, but we were airborne. 

After landing, we were deplaning and the Captain wanted to talk to me in the cockpit. He asked me if I felt the shaking and I said yes. He stated this happened when the flaps were lowered. We agreed that the spoilers were floating. He also stated this aircraft was unsafe to fly and un-airworthy. I thought the plane was going to come unglued during the shaking from where I was seated. If I knew this was going to happen, I would not fly this aircraft or release it to the crew. The Lead Mechanic verified on arrival at the gate that the #1 Spoilers were deflected up to two inches when they were supposed to be deactivated per MEL. Mechanic Y also asked the Flight Attendants if they felt the shaking on the previous flight. The answer was no. There was no report of vibration on the previous flight. This all happened after the spoiler actuator was replaced.

Callback: 1

Reporter stated the vibration and shaking began as soon as the flaps were lowered. The A320 spoilers are numbered one through five on both wings, with #1 spoiler being the most inboard spoiler near the fuselage and a ground spoiler only. 

Reporter stated that part of the procedure to deactivate the inboard ground spoilers was to try and lift the trailing edge of each deactivated spoiler to verify the spoiler actuator is properly locked-out and to prevent the type of spoiler float that occurred as soon as the flaps were lowered. He's not sure if Mechanic Y actually performed that procedure. Mechanic Y was quite concerned about getting back to ZZZ. 

Reporter stated the MEL Spoiler Deactivation procedures were revised to provide better clarity for mechanics.

Synopsis

A Line Avionics Mechanic reports he worked with another Mechanic to address a Flight Crew refusal of an A320 aircraft with deferred inboard ground spoilers. Efforts to correct problem were not successful. Inboard spoilers were deactivated and aircraft released. Aircraft experienced severe vibration on approach when flaps lowered.

ACN: 962381 (2416 of 2908)

Time / Day

Date : 201107

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 962381
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

During my exit row briefing the Purser called for flight attendants to arm doors for departure. I was unable to approach my door and arm it because passengers were still up in the isle getting settled in. While waiting to get to my door the "All Called" and I was again unable to attend the call because passengers were in the aisle getting settled in. During the safety video I stood in my designated position unable to see video screens to know if they were on and working or not.

Synopsis

A320 Flight Attendant reports hearing the "arm doors for departure" PA while she is still giving the emergency exit row briefing with passengers standing in the aisles.

ACN: 962276 (2417 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aircraft Cooling System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 33500
ASRS Report Number.Accession Number : 962276

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 300
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 962294

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Evacuated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At FL360 an ECAM Vent Blower and Extract Fan warning appeared. We followed ECAM procedure, referenced flight manual procedures and advised Maintenance. We experienced no further problems during flight.

After landing we got an ECAM Avionics Smoke message. We followed ECAM procedures and since we did not have any smoke we taxied to the gate, which was close by. After engine shutdown and checklist, the warning horn in the nose wheel sounded and the First Officer smelled smoke, so he notified ground of a possible fire. All passengers deplaned through door 1L. When CFR arrived all passengers were off. No fire was detected by the Fire Department, so the Fire Chief ended emergency.

Synopsis

An A320 flight crew received ECAM Vent Blower and Extract Fan warning in flight. After following the appropriate procedures they continued to destination. After landing they received and ECAM Avionics Smoke warning, declared an emergency at the gate, and evacuated the aircraft. CFR found no evidence of fire.

ACN: 961957 (2418 of 2908)

Time / Day

Date : 201107

Place

Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7850
ASRS Report Number.Accession Number : 961957

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 961959

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Got Master Caution Light and audible ding after throttles were pushed up and aircraft began accelerating for takeoff. [We] aborted takeoff per SOP at 75 KTS. Uneventfully exited runway, and taxied back to gate. ECAM stated HOT BRAKES even thought no brake temperature exceeded 270 degrees (300 or above triggers fault and master caution). Maintenance could not find the problem, and tried to push aircraft back on us. [We] refused to fly for safety concerns.

Narrative: 2

On takeoff roll at about 75 KTS, the ECAM BRAKES HOT resulted in a low speed abort. Auto brakes did not engage. Follow SOP for abort below 80 KTS, cleared runway completed checklist and returned to the gate. Multiple write-ups for brakes without correcting the root cause resulted in an abort. After a three plus hour delay back at the gate and only after a Captain refusal of the aircraft via three way phone call with Dispatch and Maintenance was the correct maintenance performed resulting in a safe aircraft for dispatch.

Synopsis

An A320 flight crew reported a rejected takeoff due to a hot brakes ECAM. All of the brake temperatures were bellow 300 degrees but this was a repeat anomaly so the crew refused the aircraft until it was properly repaired.

ACN: 961896 (2419 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 961896
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Other Person
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Upon initial entry onto aircraft Captain and First Officer greeted by FAA Safety Inspector who informed us that upon his inspection he found that the escape slide ball locks for the right overwing emergency slide were painted over and this was in non compliance of an Engineering Order. Maintenance was notified to correct the discrepancy and if I would enter it into the logbook. I did and maintenance cleaned the paint, signed off the logbook and we departed. The Inspector also commented that painting over the pins could cause the emergency over wing slide to not deploy properly. It is unknown how the pins were painted over. I would suggest more comprehensive monitoring by our Maintenance Department.

Synopsis

An FAA Inspector discovered that an A320's right overwing emergency escape slide ball locks were painted over and would not operate if needed. Maintenance cleaned the paint.

ACN: 961807 (2420 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 961807

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 961813
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Track / Heading : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Being vectored to final for 28R, received instruction from NORCAL, "fly heading 210 maintain 3000." The controller was very busy with VFR traffic and we were never told to intercept 28R. There was traffic, 737, intercepting 28L at our altitude, approaching centerline, we were unable to get in with ATC, I turned away from the traffic and the First Officer was able to report to ATC we had turned away from traffic at that time we were cleared for the visual for 28R, at the gate I called ATC and they had already played the tapes and confirmed that we had only been given a heading and altitude clearance and not cleared to intercept or cleared for any kind of approach.

Synopsis

Air Carrier being vectored by NCT was not given instructions to turn and intercept the final for Runway 28R causing a conflict with traffic intercepting the Runway 28L final, ATC acknowledged an error on their part.

ACN: 961806 (2421 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility.Other 
Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Air Data Computer
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : INS / IRS / IRU
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11100
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 961806
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

We submitted a report detailing that the nose light "ADIRU on BATT" indicator was inoperative. Maintenance came out and claimed that this indicator was operating normally and released the A320 with a new maintenance release. We departed uneventfully. Upon arrival, we found that the nose light "ADIRU on BATT" indicator was in fact still inoperative which would mean that the maintenance release was not a valid release. 

I have seen that Line Maintenance is being pushed to release and or defer items in order to get on-time departures and reduce delays blamed on maintenance; similar to the problem we have on the Operations side with Flight Managers pushing pilots to take unacceptable and or unsafe aircraft. We trusted that the write-up was cleared properly. I don't have an FAA Airframe and Powerplant (A/P) license, nor am I trained to know for sure that a write-up was properly cleared. It is unacceptable for [our] air carrier management to place a pilot, passenger and or equipment in jeopardy just for on-time departures. This is a small incident; however it must be noted for those concerned, that larger and higher safety related items are [also] being treated the same way, to pacify managements' desire for [financial incentives] associated with the on-time departures and arrivals.

Callback: 1

Reporter stated the same issue with the A320 ADIRUs happened twice before to his First Officer and now to him where the ADIRU "On Batt" indicator light was inoperative. The ADIRU was operating from the battery bus. When the ADIRU operates on battery only, an aural warning horn is supposed to sound, but that was not happening either. When maintenance did a "Press to Test", the system tested "OK". But the aural warning does not occur during that test, so there was no indication the aural warning had also faulted. Only when maintenance performs the full ADIRU test will the failure of the aural warning be noticed.

Reporter stated that one particular station is notorious for pushing pilots to accept aircraft and pushing mechanics to sign-off maintenance items due to airport departure curfews. He has seen the same pressure becoming more noticeable at other stations because there are no spare parts or spare airplanes available.

Synopsis

A Line Captain reports inadequate maintenance trouble shooting of an A320 "ADIRU on Battery" indicator light being inoperative and subsequent sign-off and the improper maintenance release that was issued. Adding to his concerns was the noticeable "pilot pushing" to take unacceptable aircraft and "mechanic pushing" to release aircraft or defer items.

ACN: 961669 (2422 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Relative Position.Distance.Nautical Miles : 5
Altitude.MSL.Single Value : 1400

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Cloudy
Weather Elements / Visibility.Visibility : 6
Light : Night
Ceiling.Single Value : 2500

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : LGA

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 961669
Human Factors : Confusion
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11800
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4800
ASRS Report Number.Accession Number : 961294

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Human Factors

Narrative: 1

As pilot flying I briefed and prepared for the LDA-A at LGA. ILS 22 was also briefed and loaded in the secondary route. New York Approach said to expect a visual approach to Runway 22. On base leg with field at in sight at 2,000 FT we were cleared for a visual approach to 22. At this time I called for the secondary approach to be activated. Outer Marker COHOP on LDA-A is 1,600 FT, Outer Marker GREKO on ILS 22 is 1,900 FT. In the transition to the visual (backed up by the ILS 22) I thought I needed to cross GREKO at 1,600 FT, placing the aircraft high on profile. I selected -1,800 FT/minutes Vertical Speed to intercept glide slope from above. As the Captain cross-checked he realized the aircraft was low on profile. At that time (approximately 1,600 FT) I disconnected the auto pilot, arrested the descent, and maintained level flight until re-intercepting the glide slope (at approximately 1,400 FT), and landed normally. Approach Control and LGA Tower informed us they received a low altitude alert.

Spend more time flying the aircraft and less time managing the automation. Had I tracked the LDA-A course and flown a Visual Approach it would have eliminated a high work load in a time compressed situation.

Narrative: 2

I (pilot not flying) went to the MCDU and activated the secondary route. I then received a message that there was an ILS/RWY mismatch. This was due to the fact that, per the airport briefing page, we were to hard tune the LDA frequency in the RAD/NAV page for the LDA-A approach. I selected the RAD/NAV page in the MCDU and deleted the LDA IDENT/FREQ and then the ILS information automatically displayed.

I [then] noticed that we were at approximately 1,600 MSL and descending. I alerted the First Officer that he was low and then noticed that the speed was approaching flap overspeed. I advised "watch your speed, you're fast". At this point the First Officer disconnected the autopilot, leveled and slowed the aircraft, and we intercepted the glide slope from below.

I didn't remember if approach had handed us over to the Tower so I queried approach if he wanted us to to go the Tower? His reply, after a short moment, was to ask if we were still with him. He said he had a low altitude alert and to contact the Tower. After I switched to Tower frequency, the Controller said that he'd been trying to contact us and that he had had a low altitude alert and that we had been too low.

Synopsis

An A320 flight crew, caught up in autoflight manipulation and company mandated airport procedural requirements, descended below the glide slope towards terrain triggering low altitude alerts from both Approach Control and the Tower.

ACN: 961392 (2423 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Rudder Control System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2900
ASRS Report Number.Accession Number : 961392
Human Factors : Human-Machine Interface
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The aircraft passed all preflight checks on the ground and flew the entire leg without incident or any mechanical problems. The weather was clear and the winds were 5-7 KTS out of the west. We conducted a visual approach but once we touched down, the nose of the aircraft started to drift left of the centerline. I immediately started to apply right rudder but to no avail. It felt like the rudder was frozen. I then started to apply a great deal of pressure to the right rudder (I would estimate 100-200 LBS of pressure). The rudder then started to move a little bit and I was able to straighten the nose out and parallel the center line. I asked the First Officer if he was on the rudder and he said no. Then the rudder started to fluctuate left and right and the nose started to respond by moving left and right. At this time I asked the First Officer to come on the rudders with me and help me keep them centered. He did and we were able to keep the aircraft going down the runway straight until I got the aircraft slow enough to introduce the tiller. When we cleared the runway, we could move the rudder pedals full deflection in either direction with no resistance or feel at all. The rudder pedals felt as if they were not connected to anything. However, when we moved the rudder pedals from left to right they did command the nose wheel steering to steer left or right. I then used the nose wheel steering disconnect button on the tiller and accomplished a flight control rudder check. When I pushed the rudder pedals full left and then full right, the rudder indication on the flight control display page did not move. The rudder on the flight control display page stayed in the center position. We taxied to the gate without incident and wrote the discrepancy up. We then waited for a maintenance technician and we briefed him.

Callback: 1

Reporter states that maintenance found that a nut had come off a bolt in the tail of the aircraft and caused the problems reported. After clearing the runway the rudder pedals moved freely without resistance which confused the issue even more.

Synopsis

A320 Captain experiences "frozen" rudder pedals upon landing and subsequent difficulty maintaining directional control. With help from the First Officer control is maintained and the Captain is able to steer using the tiller. Once clear of the runway a flight control check is accomplished and the rudder shows no movement for a full scale rudder pedal deflection.

ACN: 961129 (2424 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 9500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Service/Access Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 961129

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Just after takeoff and passing through 9,500 MSL we received the ECAM "Door - Avionics" with associated ECAM SD showing the right-side fuselage Avionics door open. ECAM procedure was accomplished which only consisted of a restriction against flying above 10,000 MSL. FM Procedure expanded this and advised that the Avionics door is a plug type door and that, if pressurization was normal, there was no further action required. The differential pressure was approximately 2.0 PSI with cabin pressure indications all normal.

However, we had just departed westbound and would be over high terrain (above 14,000 MSL) within minutes without a deviation. We were still at/below 10,000 and would be able to make a normal approach/landing back in the airport without ever exceeding 10,000 MSL. For these reasons I decided that the safest, obvious decision was to remain at low altitude and stay in the traffic pattern for a return to the airport, accomplishing this in VMC conditions with minimal delay.

Synopsis

An A320 departing a high altitude airport displayed an Avionics Door Open ECAM message. Although pressurization appeared normal the flight crew opted to return rather than fly over higher elevations with the potential for greater problems should the event prove less than benign.

ACN: 961092 (2425 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SFO.Tower
State Reference : CA

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 200 ER/LR (CRJ200)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Facility : SFO.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 961092
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I plugged in to Local Control and advised her that I had the status. Local Control gave me the briefing. The Controller indicated a Rebas departure rolling on Runway 01L, a SF8 departure rolling on Runway 01R and indicated the arrivals on final. The Controller stated that there were no helicopters or transitions and I told the Controller I had the position. The Controller moved back away from the position and I and moved into the position. I scanned the departing traffic and I noticed that the Runway 01L departure was in fact a CRJ2 and it was still going straight out. I thought that the Controller had said that it was a Rebas, which is a "prop" departure. I thought that it should be an E120 and it should be turning left by now. I looked at the strips in front of me and I found a strip for an E120, Rebas. I looked for a strip for a CRJ2, but I didn't see one. Then the CRJ2 asked "Tower, should we do a runway heading? We are on a SF8 and ..." I looked at the upwind traffic and saw the CRJ2 going straight out at about 200 FT altitude off the departure end of Runway 01L, and next to him was an A320 at about the same altitude going straight out off Runway 01R. I told the CRJ2 to "Fly runway heading. The coordinator pointed to my strips and said something like "That's the wrong strip!" 

I heard the coordinator begin to coordinate with CI-1 (NCT TRACON) as I saw the A320 begin their right turn away from the CRJ2 which was still going straight out. The CRJ2 acknowledged the runway heading. I told the A320 to contact Departure. Someone brought over the strip for the CRJ2 and I told him to contact Departure. Recommendation, this event must have begun at Ground Control when they confused the strips. Neither Ground Control nor Local Control noticed. The pilots and the controllers didn't hear the different call sign/s. I can easily see that this might happen with such similar sounding call signs. This has happened at least two, maybe three times as the aircraft was taxied to the runway, told to line up and wait, and then cleared for take off. Ground Control and Local Control as well as perhaps the coordinator had opportunity to look at the strip as well as scan out the window to see the difference between the E120 on the strip and the actual aircraft, the CRJ2. I failed to notice that the "Rebas" departing Runway 01L was in fact a CRJ until I had physically moved into the position and taken a good look at the strips. By then the pilot realized that something was wrong, since he was supposed to make a right turn, but there was an A320 directly to his right. He did the right thing and asked a question. Local Control definitely needs to match the strip with the aircraft they see at the runway for positive identification. I was complacent in accepting the position before I had actually read the strips of at least the rolling aircraft. I just assumed that they were what I was told they were.

Synopsis

SFO Controller described a confused departure event involving two Air Carrier aircraft on parallel tracks of Runways 1L/R, the reporter noting wrong strips were present at the position prompting the confusion.

ACN: 960901 (2426 of 2908)

Time / Day

Date : 201107

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Generator Drive
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 960901

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 960892

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aborted takeoff at 40 KTS due to number 1 IDG failure. Abort uneventful. Returned to gate.

Synopsis

A320 flight crew reports rejecting a takeoff at 40 KTS for an IDG failure.

ACN: 960612 (2427 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.AGL.Single Value : 1500

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component : 1

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 5200
ASRS Report Number.Accession Number : 960612
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

My First Officer and I were assigned an all-night ID that had us deadhead to LCQ and then pick up a plane and fly it to ZZZ. The airplane had had landing gear maintenance performed on it while at LCQ. We initially had to divert due to weather where we landed without incident. After a short delay we proceeded once more to ZZZ. On approach to ZZZ after the landing gear was lowered we had LGCIU 1 indicate the right main landing gear was not down and locked. LGCIU 2 indicated the right main gear was in fact down and locked. 

We executed a go around and contacted Maintenance Control to confirm LGCIU 2 was providing a good down and locked indication. During our second landing attempt we received a landing gear not down and locked ECAM Warning at 750 feet AGL. Uncomfortable with this we executed another go around and confirmed with maintenance that this was, in fact, expected for our LGCIU 1 and LGCIU 2 indications. We then landed safely at ZZZ. During taxi in we lost nose wheel steering and had to be towed to the gate.

Synopsis

An A320 suffered multiple LGCIU related gear unsafe warnings and executed two missed approaches before landing safely. During taxi-in they suffered the loss of nose wheel steering.

ACN: 960420 (2428 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Spoiler System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 960420
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

After pushback and starting engine #1 we had ECAM "flight control spoiler fault" and called company and returned to the gate for maintenance. Contract maintenance showed up tried to troubleshoot the problem with help via phone with Maintenance Control. The contract mechanic was unfamiliar with the Airbus type. After 4 and 1/2 hrs the conclusion was a number 1 spoiler fault and secured the spoilers and apparently not having the proper tool to secure the number 1 spoilers, we departed with the number 1 spoilers on MEL. Upon arrival and after configuring the aircraft with gear down and going from flaps 3 to full a very pronounced noise and vibration occurred. Lateral control was unstable and extremely sensitive. After quick brief we went back to flaps 3 configuration and having enough control to safely land we continued and landed. Contacted maintenance and advised them of problem. Contract maintenance [was] unfamiliar with Airbus and apparently [did] not have a proper tool to stow the number 1 spoilers properly.

Synopsis

A320 Captain experiences a spoiler fault after pushback and returns to the gate. Contract Maintenance is unable to correct the fault and the aircraft is dispatched with the number one spoilers on MEL. During approach, when the flaps are positioned from 3 to full, a very pronounced noise and vibration occurs and lateral control becomes unstable and extremely sensitive. Flaps are returned to 3 for landing.

ACN: 960222 (2429 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SFO.Tower
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SFO
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 960222
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Received takeoff clearance Runway 1R. At about same time aircraft on Runway 1L was also cleared for takeoff. Aircraft on 1L rotated about 1-2 seconds before we did. Both aircraft airborne there was a point before each made turn away where in any other situation would be considered a near miss. Tower should have staggered the takeoffs by delaying one of us an appropriate number of seconds.

Synopsis

Air Carrier departure from SFO voiced concern regarding the simultaneous departures from Runway 1L/R, suggesting Tower momentarily delay one of the two departing aircraft.

ACN: 960209 (2430 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 138
Experience.Flight Crew.Type : 4906
ASRS Report Number.Accession Number : 960209

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the pilot flying when we got a Y HYD LO (Yellow Hydraulic System Low) warning per ECAM. We followed the procedure, advised Maintenance Control, spoke with Dispatch, declared an emergency and diverted where the Captain landed uneventfully. We pulled off the runway and stopped while CFR inspected the aircraft and advised they saw fluid under the #2 engine without any signs of fire. We were then towed into the gate.

Synopsis

An A320 declared an emergency and diverted to a nearby airport after experiencing a Yellow Hydraulic System low fluid level warning.

ACN: 959302 (2431 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 959302
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Weather
Contributing Factors / Situations : Procedure
Primary Problem : Weather

Narrative: 1

After a thunderstorm with hail passed over the airport, and during the following 90 minute or so ramp delay due to lightning within 5 miles, the First Officer and I each performed separate walk-arounds to see if there had been any hail damage to our plane. We also sought higher vantage points (top of the jet bridge stairs) and from no location were we able to find any evidence of hail damage. As we were then awaiting our turn to speak with Maintenance, we overheard several different crews asking questions regarding any additional inspections required. In all cases those crews were instructed that per Maintenance Control, if the pilot performing the walk around could not detect any damage, the aircraft was ok to continue. During the delay, I also heard this information come straight from an on-duty mechanic speaking to another crew on a different gate. The following day, I received a call after we blocked in at the end of our trip from a Chief Pilot informing us that around 18 planes, including ours, had been taken out of service due to hail damage that "was not visible from a normal walk around/from the ground." As far as the "cause" section of this form is concerned... the First Officer and I were confident we had done everything necessary for the safe operation of flight.

Although this is the first time in my 17-year aviation career that I've ever experienced hail damage to an aircraft, I think a procedure for maintenance to inspect aircraft from above to detect any possible "undetectable" damage would be beneficial to us all.

Synopsis

After a strong thunderstorm with large hail. A A319 Captain performs a walk around, detecting no damage. Maintenance advises several crews that if there was no visible damage on a walk around/preflight inspection, the flight was good to go without a maintenance inspection. Damage is discovered the next day.

ACN: 959244 (2432 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 959244
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On descent, we armed auto-brakes to low. Then we got a brakes BSCU CH1 FAULT ECAM. We accomplished the ECAM and continued the descent. Since a failure of only one channel of the BSCU does not affect the braking system at all I did not see a need to try a system reset. In fact, the Airbus is routinely dispatched with one channel of the BSCU inoperative. On initial contact with Approach we were given the right runway and I deselected the auto-brakes since I was going to have a longer roll to desired high speed. On landing, full reverse was applied and I noticed out of the peripheral vision that the wheel page was displayed and something was written in amber. I applied brakes pressure and the aircraft responded but in an unusual manner. We exited the runway and I reapplied the brakes, but needed much more than normal pedal depression to get any braking. I then looked closer at the wheel page and noticed a green "ALT BRAKES" and amber "AUTO BRKS" indication and saw that the brake pressure indications on the accumulator gauge were in fact reacting to my brake inputs, which indicated a failure of the entire normal brake system. I slowed the aircraft and using the 'A' SKID/NWS switch, reset the system and normal braking returned. My fundamental question is, how did the failure of just one BSCU channel cause the system to revert to alternate brakes? Why did we get no other ECAMs or warnings that we had lost normal braking? If this had occurred on a short runway with slippery conditions the outcome could have been different.

Synopsis

An A320 Captain experienced a BSCU CH1 fault when autobrakes are armed during approach. During landing it was discovered that only alternate brakes were available. The A SKID/NWS switch was used to reset the system.

ACN: 959217 (2433 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Hail
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 959217
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 959219
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

After arriving at gate, a Mechanic came on the aircraft and lowered the flaps (non-standard action for a post flight). He radioed down to a Mechanic on the ground who said that there was hail damage to the flaps (which could only be seen with the flaps extended). Maintenance took the airplane out of service. This hail damage was not detectable from a normal FOM approved walk around. The First Officer did his normal approved walk around before departure and did not notice or mention to me any damage to the aircraft. If the company has knowledge of some abnormal event like this (the hail storm the previous day), which the crew in out stations may not know about, then Maintenance, Operations, Dispatch or someone in the company should have called all the crew who were flying airplanes that had come from the airport where the damage could have occurred during the hail storm so that Maintenance could have performed this inspection before the crew flew the airplanes.

Narrative: 2

Hail damage was found on the upper portion of the flaps (after the flaps were lowered by Maintenance). No damage was noted during my preflight exterior inspection. The flight crew that experienced the hail storm (if they were on the plane) [should] have left some information with the Operations, so that Operations could have informed us.

Synopsis

Maintenance personnel entered an A320 after arrival, lowered the flaps and discovered flap damage from a hail storm the previous day. The flight crew could not see the damage during a normal walk around with the flaps up prior to their flight.

ACN: 959130 (2434 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Make Model Name : A321
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 959130
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The airport was on midnight status, meaning opposite direction arrivals and departures. In this runway status, the Departure Controller has to release and issue a heading to departing traffic. The Departure Controller must not give a release to the same complex as an aircraft with an approach clearance, i.e. if an aircraft is given a visual approach to the west complex [Runway] XXR/XXL they must not release a departing aircraft off that complex. Air Carrier X, [an] A321, was released from the Departure Controller [and] told to take southwest bound. Air Carrier X was given opposite direction traffic, [an] A320 on a 12 mile base to final for that same runway. Air Carrier X was then instructed to fly heading 205, Runway YYL cleared for takeoff. Once airborne, Air Carrier X was given a further turn away from the inbound aircraft to a heading of 220. Air Carrier Y checked in shortly thereafter, was given traffic departing opposite direction, [an] A321, southwest bound, issued the winds, and then cleared to land. Visual separation was applied between the two aircraft. One aircraft was climbing out and turning; the other aircraft was straight in and descending towards the runway. No TCAS event was reported nor was a collision alert issued by the STARS. 

The TRACON Supervisor later called the Tower twice to ask what type and how was separation being applied. The Tower Supervisor told him we were using visual separation. This is a safety related event because it is clearly not safe to aim jets at each other in an effort to mitigate noise. Although visual separation was applied, the procedure calling for opposite direction operations in an effort to be a "good neighbor" is not safe. There is no formal noise abatement program [at this airport]. Rather, we risk the lives of hundreds to silence the voices of a few. We conduct these procedures of aiming air carrier jets at each other in an effort to mitigate noise, yet there is no formal outside procedure stating we shall do this rather it is an internal procedure to document how good of a neighbor we are. I suggest that [the airport] either be in a South operation or a North operation all the times. Having a midnight operation of aiming air carriers at each other is too risky and far from safe. The biggest noise complaint is that the arriving aircraft make too much noise at night. If the TRACON were to issue a speed restriction to arriving aircraft after hours they would be less noisy and create less of a nuisance to those whom live just north of the airport; this is currently not done. People around the airport, specifically one person, complain. Hence, we aim jets at each other to make him happy and be a "good neighbor."

Synopsis

Tower Controller described a potential conflict involving opposite direction takeoff/landing aircraft required by late night noise abatement procedures, claiming the procedures are unsafe.

ACN: 958947 (2435 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 958947
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Cruising at FL370, engine screen came up with flashing 4.0 on oil quantity on #1 engine. Maintenance and Dispatch contacted. Maintenance stated if oil pressure and temperature were normal that we could continue. They were, but we instead diverted. Emergency was declared and safe landing and taxi to gate was accomplished. [We] diverted because we could not justify why we would continue with low oil quantity showing even though Maintenance suggested otherwise.

Synopsis

After an A320 ECAM alerted engine one OIL QUANTITY 4 at cruise Maintenance recommended continued flight because pressure and temperature were normal but the crew diverted because of the unusual engine operating condition.

ACN: 958722 (2436 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LGA.Airport
State Reference : NY
Altitude.AGL.Single Value : 200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : LGA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : LGA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 500
ASRS Report Number.Accession Number : 958722
Human Factors : Situational Awareness
Human Factors : Distraction
Analyst Callback : Attempted

Events

Anomaly.Other 
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

On short final, saw [a] flagged crane to the north and along side of the final approach to Runway 31. [The] crane was at or above the low visual (VASI red-red-white) glidepath to Runway 31. [The] crane appeared dangerously close to final approach course. We were finishing the Expressway Visual to Runway 31. The left turn from over the expressway to final approach keeps the airplane in a position such that the crane is not visible until rolling wings-level on very short final. When I did see the crane it was apparent to me that any overshoot of the final approach path to Runway 31 could put an airplane in jeopardy. Cranes affecting approaches to KLGA and specifically Runway 31 are referenced in the current NOTAM list for KLGA. However, all references to these obstructions are suffixes to NOTAM describing changes to approach minimums for approaches to that runway used in IMC conditions. There is no separate NOTAM for the Expressway Visual 31 (our approach today) or a separate NOTAM describing the crane as a simple obstruction. Given the fact that the crane is so close to the final approach and intrudes up into the glidepath, I believe there should be a stand-alone NOTAM not associated with a terminal procedure highlighting the danger. Also, I think it would be advisable while using this runway to have the Local Controller remind approaching aircraft of its proximity to the final when clearing the aircraft to land.

Synopsis

An A320 Captain reported a crane adjacent to the final approach to Runway 31 at LGA is a hazard to aircraft which overshoot the turn to final when flying the Expressway Visual approach. Although the crane is referenced within FDC departure and approach NOTAMs the reporter believed a separate obstacle NOTAM is necessary.

ACN: 958714 (2437 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 958714
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We received our 4th version of our final weight manifest message after takeoff. We originally taxied out, but called back to the gate due to an out of limits CG. Ramp opened the cargo doors and changed the CG. During this time we received 2 more final weight messages, both received after the Ramp was done loading, and we assumed these were our final weights. We finally received the last "final" weights message in the climb eight minutes after takeoff. Our weight dropped 507 pounds and the CG stayed the same.

Synopsis

An A320 flight crew received four different sets of "final" weight manifest data; one before returning to the gate to shift cargo containers, two while at the gate undergoing cargo shifting, and the last takeoff weight data message received eight minutes after takeoff while in the climb.

ACN: 958530 (2438 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 958530

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 958511

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

WE were enroute at FL360, deviating around thunderstorms and in light turbulence at about .76 mach when number two engine exceeded normal limits in N1 vibration. Our limits are 5.0 and the engine lower display ECAM was flashing at 6.0. Over the next five minutes the vibration number increased to a high of 8.3 and noticable vibration was felt throughout the airplane and controls. No other adverse indications from any other engine indications. 

We referred to the Flight Manual and contacted maintenance through Dispatch. As the Captain tried to communicate problem to Maintenance we reduced the bad engine power to idle. I coordinated a slow descent with ATC since we could not hold altitude with one engine at idle. The N1 vibration at idle initially indicated 3.5 which was below the 5.0 limit. We decided it best to keep the engine at idle. We did try one increase and the vibration increased again. We did not try again. After discussions with Dispatch and Maintenance we made the decision to divert to the closest suitable airport. 

We made an uneventful descent and landing using the good engine and keeping the vibrating one in idle. I did note the N1 Vibration was almost zero once we were with approach and at about 10,000 feet. After landing we shut the engine down as soon as we cleared the runway. Our checklist in the Flight Manual provided very little guidance. It did direct us to land at the nearest suitable airport and that's the only statement that really provided any useful guidance in this situation. It should be also noted that our maintenance did not have access to engine data while we were airborne as is provided by other modern aircraft. This resulted in us having to spend time to accurately report the condition of the engine to Maintenance. I think ZZZ logistically would have been a better divert airfield but would not have met our requirement to land at the nearest suitable airfield.

Narrative: 2

At FL360, in and out of clouds, SAT -54, lower EWD [Engine Warning Display] rolled to ENGINES. The right engine indicated N1 VIB (vibrations) of over 5.0, peaking at 8.3. [We] descended to FL280 with the thrust lever retarded, unable to bring VIB below 5.0. Descended to FL200 with the thrust level at idle, we were able to bring VIB to below 5.0. We were in contact with Dispatch and Maintenance Control. With thrust lever at idle and per the Irregular checklist we decided to divert.

Synopsis

An A320 flight crew diverted to the nearest suitable airport when the vibration level of the right engine suddenly increased beyond limits.

ACN: 958516 (2439 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 958516

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

This trip was the third leg of a re-qualifying check ride for me after returning to the cockpit from a one year medical absence. Prior to departure the Check Pilot and I jointly decided to accept the aircraft with an MEL, Yellow Electric Hydraulic Pump Inoperative. Situational conditions made this a prudent decision in this case. During climb out, ECAM, Hyd Y Rsvr Lo Air PR, displayed. All ECAM procedures and Flight Manual actions were completed. The ECAM contained "conditional" item that dealt with fluctuations in pressure of the affected system. Initially, this condition was not met but after a few minutes, pressure fluctuations did occur and the conditional item on the ECAM was accomplished.

We then decided to return to the departure airport. Consideration was given to a close monitoring of a now isolated Hydraulic system and continue toward our destination, but after a review of current and forecast weather at our destination, the decision to return to our departure airport was made. [It was an] uneventful return to departure airport. All coordination required with Dispatch and others accomplished. ECAM and Flight Manual actions associated with approach and landing accomplished.

Synopsis

After departing with the Yellow system electric pump deferred, an A320 flight crew returned to departure airport when reservoir pressure to the yellow system was also lost.

ACN: 958316 (2440 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Fuel Tank
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Mininum Equipment List (MEL)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 958316
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Logbook Entry
Primary Problem : MEL

Narrative: 1

Assigned an A320 aircraft with a deferral for Fuel Tank Wing Transfer valves on both wings locked "Open." In such condition, MEL has a multi-step procedure after fueling, including four Circuit Breaker cycles assigned to pilots. This is clearly a Maintenance function and should be removed from pilot "To Do" list for dispatch of flight. 

Additionally, with fuel valves in deferred "Open" position, fuel is not available for Integrated Drive Generator (IDG) cooling via the designed recirculation system, setting up potential fuel tank temperature limit exceedence. MEL states that should that condition occur, to deselect [engine electrical] Generators. It seems contrary to safe operations to enable a fuel problem to evolve into an electrical problem. This should not be a deferred item. Aircraft was refused and maintenance was begun, as we were assigned another aircraft.

Callback: 1

Reporter stated the MEL deferral of the Wing Tank Fuel Transfer Valves in the "Open" locked position should not be allowed if the wing tank fuel temperatures are susceptible to rising to a level requiring a Pilot to "De-select" (disengage) the engine IDG Generators, due to a lack of adequate cooling fuel from the wing tanks to cool the IDG Generator oils. Normally the APU IDG should be available, but that would be assuming the APU IDG Generator was not also previously deferred. Extended wait times on hot ramps,long taxi prior to take-off and hot weather environments can elevate fuel tank temperatures. 

Reporter stated the response he received from his company's flight technical group regarding his refusal of the aircraft and his concerns the MEL deferral was inappropriate.

Synopsis

A Captain explains why he believes an MEL deferral of the Wing Tank Fuel Transfer Valves in the "Open" locked position on the A320 aircraft should not be allowed if the wing tank fuel temperatures are susceptible to rising to a level requiring a Pilot to "De-select" (disengage) the engine IDG Generators. Pilot refused aircraft.

ACN: 958132 (2441 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Weather Radar
Aircraft Reference : X

Component : 2

Aircraft Component : Filter
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 958132
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Primary Problem : Procedure

Narrative: 1

We are finding severely clogged Equipment Cooling air filters for the Weather Radar Transceivers on the A320 and B757 fleets. Some of these filters are so clogged you are unable to see direct sunlight through them. These small, washable filters are located on the Weather Radar Transceiver trays and are the last catch to clean the cooling air before it enters the computer. While a clogged filter may not cause an immediate failure of the Transceiver (TR), the elevated temperatures inside this component must drastically affect its service life. 

The airline effect is this: Weather-Radar (W-R) systems are rarely deferred, refused by either Dispatch or flight crew on a consistent basis. At non-maintenance stations, [this] can be a cancellation. Maintenance requirements: repairs can add hours to an aircraft refusal, especially on the B757 (B757s require a pushback from the gate and 300 FT clearances for radar operational tests).

Synopsis

A Line Mechanic reports finding severely clogged Equipment Cooling air filters for the Weather Radar Transceivers on A320 and B757 fleets, causing elevated temperatures inside the Transceivers.

ACN: 958101 (2442 of 2908)

Time / Day

Date : 201107

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 958101
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

During the service the pilots requested a break. The Purser came to me to ask that I help fulfill cockpit security procedures. Unfortunately I was working the cart (by myself) with another Flight Attendant and cart in front of me. There was no place to move and/or stow my cart. I advised that I was unable to leave my cart unattended and could not assist in fulfilling cockpit security duties and asked that she wait for a few minutes. The Purser seemed surprised and in discussing the FAR later she mentioned that she hadn't considered the FAR as she was "so used to what other flight attendants did." This was an excellent and very knowledgeable Purser so her response surprised me. It concerns me that other flight attendants may be inadvertently violating FAR's as they try to accomplish all their required duties. It also concerns me that there are not enough flight attendants to fulfill all of our required duties. The company reduced staffing to reflect the reduced "meal service offerings" but has not considered the staffing reduction's impact on safety and security procedures.

Synopsis

A Flight Attendant reports being asked by the Purser to assist with security duties while the pilots take a restroom break. She is alone on her cart with no place to stow it and is surprised the Purser had asked due to regulations concerning unattended carts in the asile.

ACN: 958062 (2443 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Taxi
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway 1L
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 958062
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

While on the arrival I had setup and briefed for an ILS to visual to Runway 19R. But then between two arrival intersections, the airport turned around and I re-briefed for Runway 1L. The First Officer changed the runway and the PERF APPR Page in the MCDU. Shortly after, we turned downwind and were given a heading by Approach Control. I then asked the First Officer to extend the runway center line back the final approach fix. He did this by selecting NO STAR while cleaning up the approach. While descending out of approximately 4,000 to 5,000 FT we experienced what I believe was a dual FMGC failure. Both PFDs froze. Both ND's went blank. Both MCDU's froze and no data could be entered. While still descending I attempted to switch over to the ILS via raw date by transferring to the NAV ILS mode on the EFIS Control Panel and by engaging the NAV ILS buttons on the RMP Standby NAV. I was successful in entering the ILS frequency, but was not able to enter the course. The ND displayed the compass rose, but no guidance needle came to view. We told ATC we were experiencing navigations problems and asked them for a vector/heading toward the end of the runway. There was a cloud layer at with ragged bases between 1,500 and 1,000 FT, so using ATC headings and the DME from the PROG. Page, I started a 700 FT per minute descent from where I estimated the final approach fix to be and broke out below the clouds approximately 3 miles from the end of the runway. We landed on Runway 1L and taxied to the gate. I have no idea why we had this failure. Maintenance preformed a full systems check and found no failures.

Synopsis

An A320 Captain reported that as the First Officer was reprogramming the FMGC following a runway change he selected NO STAR and both FMGC's and PFD's froze and the ND's blanked. A known NO STAR program bug caused both FMGC's to crash.

ACN: 958046 (2444 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10900
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1490
ASRS Report Number.Accession Number : 958046

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 958499

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise at FL350 approximately 2 hours 45 minutes into flight, I heard a loud bang and felt the aircraft shudder. Looking down at the E/WD, I saw the ECAM "Eng 2 Stall" and saw engine indications that back this up. I immediately retarded the power to idle on engine 2 and asked for the QRH. Retarding the power to idle stopped the stall and the ECAM disappeared. We declared an emergency and began the driftdown. ATC advised that ZZZ (about 80 NM southeast) was available and the weather was good. I continued to fly and took over communications with ATC. The Captain communicated with Dispatch, the flight attendants, and the passengers. We were vectored for a visual approach to ZZZ. We discussed leaving the #2 engine running at idle and did so, discussed overweight landing (we were almost at maximum landing weight - maybe 500 LBS over), etc. and completed the necessary checklists. I briefed single engine approach and go-around procedures. I continued to fly and made an uneventful landing with #2 at idle and #1 operating normally. The touchdown was smooth and I avoided braking until below 80 KTS to prevent any brake issues. The emergency equipment visually inspected our brakes and engine 2 and we taxied to the gate normally. I believe we handled this emergency by the book and with great CRM and airmanship. No human factors adversely affected this event.

Synopsis

A320 flight crew experiences an engine stall at FL350 and retards the thrust lever to idle. An emergency is declared and the flight diverts to the nearest suitable airport with right engine at idle.

ACN: 957698 (2445 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : L30.TRACON
State Reference : NV
Altitude.MSL.Single Value : 11000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : SHEAD7
Airspace.Class E : L30

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 208
Experience.Flight Crew.Type : 6800
ASRS Report Number.Accession Number : 957698
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 957704

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were assigned Shead 7 departure SID out of LAS. My First Officer and I reviewed and noted the NOTAMs for the Shead 7 departure. NOTAM No 1/242: Shead 7 departure; cross MDDOG at 9000'; ATC/at or above 9000';. Cross TARRK at 11000'; ATC/at or above 11000';. Our PDC cleared us to FL190 with SID restrictions. When I checked in with LAS departure, I heard the controller assigning unrestricted climb to previous traffic. Climbing through 5000';, I asked the Departure Controller about the NOTAM altitudes and which set applied to us regarding ATC vs. at or above altitudes. He replied with yes, comply with the NOTAM at or above altitudes and then stated climb unrestricted through 11000';. I read that back and understood that we were to climb unrestricted to 11000'; and then continue to climb to FL190 while complying with the rest of the restrictions. My First Officer had the same understanding and he was the pilot flying. Controller never said "climb and maintain 11000'". ATC gave us a traffic alert at 12000' and instructed us to maintain 12000'. ATC notified us of a B737. We had said traffic in sight. There was not any TCAS conflict. 

We were given a phone number and advised by the controller to call it upon landing, for a possible deviation. I contacted the TRACON supervisor upon arrival. After hearing my explanation, he agreed to investigate further as well as listen to the tapes. He called me back to inform me that he concurred with our explanation. He acknowledged that the controller did not use standard phraseology. He also said that altitude busts unfortunately happen quite often in Las Vegas due to the complex altitude restrictions. I agree with that assessment. I think the communication issues coupled with multiple complex crossing restrictions are causing deviations and confusion. Standard, clear, and timely communication between the controller and pilot is crucial.In this case, my question about the NOTAM to the controller aggravated the problem. Next time, I will try to clarify the questionable NOTAMs on the ground to avoid confusion. I will also query the controllers about incomplete and suspect clearances.

Synopsis

Air Carrier departure from LAS described confusion regarding the Shead 7 SID, the crossing altitudes ATC vs. conventional, the NOTAM instructions and ATC's phraseology.

ACN: 957494 (2446 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SEA.Airport
State Reference : WA
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : SEA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : Runway 16C
Flight Phase : Final Approach
Airspace.Class B : SEA

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 957494
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 957465
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While setting up for an approach to 16C at SEA, passing thru about 5,000 FT, the Captain asked me to sequence the box. There were about 3-4 points to delete. While I was deleting the points the MCDU froze up and would not allow any input. Seconds later we lost all data in the MCDU, lost both map displays, lost ILS signal. We still showed 1FD2 on the PFD so I guess the FMGC's were working. We both tried to do a lateral revision from a fix to get the "New Destination" prompt but the box would not accept this. We figured out how to manually enter the ILS frequency with the standby nav technique (this is something we never do and stumbled to remember how to do it). This all happened within minutes of landing at a very inconvenient time. We broke out of the clouds and could make a visual approach thankfully. The box was trying to reinitialize itself but we landed before it fixed itself. We wanted to make a record of this incase there was a trend with the new box. Once again, it all happened when we were sequencing the box for final intercept.

Narrative: 2

We were on a heading for vectors to Runway 16C at SEA. First Officer was sequencing the box and both NDs showed map unavailable and both FMGCs went blank. We put SEA back into the FMGC when it booted back up.

Callback: 2

The Reporter stated that this air carrier installed new FMGC's some time ago and has had several software updates each with unique anomalies. The QRH has procedures to deal with this specific type of fault but since the aircraft were on approach and became VMC shortly after the MCDU froze it was not necessary to complete the procedure.

Synopsis

Both A320 new series FMGCs blanked and both ND's showed MAP UNAVAILABLE as the First Officer was sequencing waypoints on final approach to SEA. The ILS was manually turned to complete the approach.

ACN: 957491 (2447 of 2908)

Time / Day

Date : 201107

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 957491
Human Factors : Physiological - Other

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I met the aircraft as it parked. Some maintenance personnel went into the cockpit as passengers deplaned. While that was occurring, I did my walk around. After getting into the cockpit, I immediately found the weather Radar in the "on" position with a ground returns showing on the Nav Display. I was irradiated. I do not know the short term or the long term health effects of this irradiation. I do not think that company procedures allow radar technicians to stand in front of an emitting radar for good reasons. It is bad for your health. The ground crew was also subject to this radiation. The company apparently has no idea what to do if their personnel have been subjected to the powerful, full strength, weather radar radiation. There is no medical support for this occurrence. I want to be clear that I do not know if I will be subject to any short term or long term health effects. As of now, 15 minutes after the incident, I do not feel dizzy or nauseous. Without any medical support available and feeling asymptomatic, I feel I must fly tonight's trip or be subject to administrative punishment procedures for using Sick List if I refuse to fly due to my fear of how I may be affected by this exposure. This is a serious lapse on many levels.

Synopsis

A320 First Officer reports finding the weather Radar in the "ON" position upon entering the cockpit after completing the walk around preflight, and believes he has been irradiated.

ACN: 957388 (2448 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 275
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 957388
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I believe that an incorrect Mel was issued by ZZZ maintenance. Mel 34-61-01 was issued. Preflight was difficult because there was no way to load the route or other entries into the MCDU, but at the time we believed that this was the issue that was covered by the MEL. Later in flight the flight crew became aware that the condition was much worse and was possibly issued the wrong Mel. Both FMGCs were found to be inoperative. Associated systems that were affected beyond just the NAV DATA being out of date included autopilot inoperative, Autothrust inoperative, GPWS degraded, pressurization degraded, along with a total loss of all automation afforded from the MCDU. ACARS was accessible. The aircraft was operated without incident, and a safe landing was performed at the intended destination.I believe that the discrepancy should have involved both failed FMGC's and under Mel 22-83-01 this does not afford the airplane to be dispatched with both FMGC inoperative. I believe that the maintenance department withheld information from the flight crew and instead of grounding the aircraft affixed a placard that allowed dispatch.

Callback: 1

The Reporter stated that incompatible NAV DATABASE and FMGC software were used to update that aircraft's FMGC after the NAV DATABASE out of date alert appeared the previous day. As a result the flight crew could not load all of the needed flight data and after takeoff when they realized the magnitude of the automation loss had to remain below RVSM and fly VOR airways to the destination. The correct software was loaded after arrival and the FMGCs worked normally. The Reporter still believes that maintenance knew what the problem was and did not want to deal with it. Another issue was passenger service pushing the crew to get off the gate because they were hours late. The flight crew had not finished the preflight when boarding was complete and the passenger service agents were pushing the crew to depart. The pressure did not allow the crew to research the FMGC problem as much as they normally would have and so departed not understanding the complexity of aircraft's systems losses having relied on maintenance to have researched the issue. Instead maintenance was not truthful about the FMGC problems.

Synopsis

An A320 was released under MEL 34-61 after maintenance stated the NAV DATABASE was out of date. However, incompatible FMGC system software was loaded following the NAV DATABASE OUT OF DATE alert which rendered the FMGC's inoperative for flight.

ACN: 957336 (2449 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Aircraft Cooling System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 957336
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb we experienced a hot air valve ECAM. The ECAM procedure was to turn off the hot air valve to isolate the duct overheat in the system. The switch had no effect on the hot air valve, leaving the system with no protection from the overheat. Maintenance was contacted for further information and input. We talked with Dispatch and coordinated a return to the departure airport. Upon leveling off and descending, the valve finally closed on its own. With the overheat isolated, we decided to hold and burn off fuel to the maximum landing weight. During our holding, the hot air valve started opening and closing. With the system uncontrollable and unstable, we decided to descend into the airport. The decision was made to land overweight due to the unknown, uncontrollable, and overheated system. The risk of landing overweight was minimal when compared to the risk of an overheated system in this state. An emergency was declared, and the landing was completed without incident.

Synopsis

An A320 ECAM alerted HOT AIR VALVE but the ECAM procedure did not close the HOT AIR VALVE so the crew declared an emergency and returned to the departure airport for an overweight landing.

ACN: 957189 (2450 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Leading Edge Slat
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Commercial
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 957189
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Primary Problem : Ambiguous

Narrative: 1

We had a slat seal that was partially missing at the intersection of the right wing and aircraft fuselage. This was discovered by my copilot during the walk-around. We wrote it up and contacted Maintenance. Maintenance applied the correct CDL procedure and reviewed it with me using a copy of the CDL from their Maintenance Manual. Unfortunately, they only made a copy of the first of a two-sided page and neither I nor they noticed the "continued" note at the bottom. Because of this we failed to note that this CDL was not allowed to be applied (and deferred) when the slat seal at the meeting of the wing and the fuselage was involved; which was the specific seal we had written-up. My copilot referenced our own onboard Manual while in flight and noted the procedural error. As the aircraft was operating normally and without any problem(s) we notified our Dispatch and Maintenance and continued on to our planned destination (now only less than thirty minutes away). We made a new logbook entry and the slat seal was replaced and signed off before the aircraft was again flown/used.

Synopsis

An A320 slat seal at the intersection of the wing and fuselage was discovered missing but because the CDL was not completely read, Maintenance and the Flight Crew did not know this item was required for flight.