ACN: 956976 (2451 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 956976
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[Received] ECAM: Hydraulic Y Reservoir LO. Following ECAM procedure gave us a yellow system failure. Maintenance Control has been removed from in-flight assistance. At least let Maintenance Control answer questions.

Synopsis

An A320 ECAM alerted HYD Y RSVR LO during climb so the crew diverted to a nearby airport for maintenance.

ACN: 956955 (2452 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : VHF
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 956955
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

This aircraft had continuous static interference noise on both radios when flying in clouds at high altitude while using the radar to avoid thunderstorms. The static was present whether the radar was on or off. The static was loud enough to drown out almost all ATC communications. Dispatch was notified and an early maintenance alert was made. The airplane was grounded for repairs on arrival. I understand that some static wicks and flap track canoe fairing bonding straps were replaced. The VHF antenna were also checked. I was assigned to reposition the aircraft empty back to a maintenance base later in the day. On the reposition flight we had the same static issue as before while flying in cloud avoiding thunderstorms with radar, except that the static happened only when a radio transition was made either by ATC or another aircraft, where before the static was continuous when flying in clouds. As before, radar on or off made no difference. On both days ATC reported that our radio transmissions were OK. Another maintenance early alert was made for this static issue. The static also appeared to be kicking the ACARS in and out of standby. An additional early alert maintenance write-up was made for the ACARS as well. This is not the first time I have encountered this type of static problem on the Airbus A320. It is very disconcerting situation as it will usually only make itself apparent during thunderstorm season when we encounter clouds with higher static electricity. Going around thunderstorms at night in the clouds is not the time to find out that you can't communicate with ATC due to radio static. I would like to know how often the static wicks and binding straps are checked by maintenance. The pilots do a visual check that they are in position on the walk around but I understand that there are additional checks for proper grounding and connections that maintenance performs? I would like to know how often the static wicks and binding straps are checked by maintenance. The pilots do a visual check that they are in position on the walk around but I understand that there are additional checks for proper grounding and connections that maintenance performs.

Synopsis

An A320 had VHF radio static so loud it blocked ATC communications when the aircraft was flying in clouds and avoiding thunderstorms.

ACN: 956940 (2453 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 6000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17800
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 956940
Human Factors : Human-Machine Interface
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 165
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 957022
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We experienced a double Elevator and Aileron Computer (ELAC) failure on the arrival phase. Approximately 8 miles west of the airport descending to 6,000 FT, the autopilot kicked itself off. As the flying pilot, I took control of the aircraft and began to manually fly. We then realized that the failure was due to the ELAC ECAM. The First Officer read the messages off the ECAM. We were in "Alternate Law" without an autopilot We had an ELAC fault and an ELAC pitch trim failure in BOTH ELAC's. Note: The ECAM did not give us the "Blue to do items" it just gave us the problem, VRef + 10, Direct Law with gear down info. The First Officer requested an extended downwind and declared an emergency. The irregular procedure in the flight manual was then referenced and read by the First Officer. When the ELAC switches were cycled we regained the functionality of both ELAC's. We then finished up the action items per the manual and took a turn back toward the airport. Landing and taxi in was uneventful.

Narrative: 2

During arrival just prior to turning downwind, auto pilot pilot kicked off accompanied by slight roll to the left. Captain hand flew. Numerous ECAM's popped up: alternate law, ELAC faults 1 and 2, ELAC pitch faults 1 and 2. We declared emergency. I ran the ECAM and numerous checklists while Captain flew and talked with ATC. ELAC's were recovered just prior to the Outer Marker. Captain landed uneventfully.

Synopsis

A320 flight crew experiences a dual ELAC failure during descent which is annunciated by the ECAM but without any ECAM actions. Flight Manual procedures indicated a reset of the ELAC's, which was successful, and a normal landing ensued.

ACN: 956865 (2454 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : EWR.Tower
State Reference : NJ
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 956865
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We LNAV'd the EWR 8 departure and noticed that the FMGC didn't turn at the 2.3 DME fix off of the localizer ILSQ. The FMGC didn't make a turn until 3.5 or so. I haven't flown out of EWR in awhile, but recall this happening before. I've called around to other pilots and they've noticed this as a trend too. A notice needs to come out to have us not LNAV the departure until the software gets corrected.

Synopsis

A320 Captain reports his aircraft turns late on the EWR 8 departure off of Runway 22R using LNAV. Turning at approximately 3.5 DME instead of 2.3 to a 220 heading.

ACN: 956851 (2455 of 2908)

Time / Day

Date : 201107
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use.SID : OFFSHORE FIVE
Airspace.Class B : SFO

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3700
ASRS Report Number.Accession Number : 956851
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Taken off SID by Controller then assigned heading 190 to rejoin the Offshore Departure SID. Subsequently cleared Direct to SEGUL intersection. I commenced left turn while clearing visually left for traffic. First Officer went heads-down to enter the direct routing into the FMGC via the MCDU. As he initiated the direct, I scanned my ND display to get an approximation of my roll out heading. In the new FMGC software in the Airbus, the new fix is not displayed in the upper right corner until after the command is executed. As the First Officer executed the direct command, and I continued my left turn to align my heading with the ND displayed track, I glanced in the upper left corner to see that the FMGC was actually giving us guidance directly to the MCKEY intersection. I announced our error and immediately started a turn back to the right to correct our course back to SEGUL, as the First Officer reentered a new direct routing command to SEGUL in the MCDU. It was about that time that the Controller queried our heading and we verified that we were cleared to SEGUL. In our post-error discussion, the First Officer said that he thought the execution error happened as he inserted the direct routing as the MCDU was "auto-sequencing" and rearranging pseudo waypoints, etc. The Controller made no further comments over the frequency regarding our brief overshoot turn beyond the direct SEGUL routing. The overshoot was approximately 30 degrees beyond the heading we should have initially rolled out on for the direct routing to SEGUL. We were on the erred heading 15 seconds or less. We corrected our error promptly. To our knowledge, there were no other aircraft in our immediate vicinity according to information displayed by TCAS, and no loss of separation occurred. However, this minor deviation did uncover and highlight for me, the potential with the new Airbus FMGC software, to be led toward a navigation waypoint before you can actually verify it on the ND, as is built into Airbus pilots' previous habit patterns.

Synopsis

An A320 was taken off a SID but when re-cleared to a fix on that SID, the FMGC transitioned to a SID point downstream just as the DIRECT button was pushed causing a track deviation. The crew did not expect the FMGC transition which was different behavior after installation of new FMGC software.

ACN: 956802 (2456 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 2000

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Navigation Database
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 956802
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Workload
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

As I lifted off on Takeoff, my flight director blinked then disappeared. Auto Thrust did not engage. I manually flew the aircraft to the initial level off altitude of approximately 3000 ft. The captain attempted unsuccessfully to engage autopilot two then one, and reengage the auto thrust system. We determined that even with these systems INOP we could continue the flight to destination provided we stayed below RVSM airspace. We leveled off at 28000 feet and notified Dispatch. They sent new projected fuel burn numbers confirming we had enough fuel for the trip. We then navigated using manually tuned NAV radios, airways and radar vectors. Enroute we discovered that we were unable to access any portion of either FMCG. The captain wrote the discrepancy up and alerted Maintenance. We completed the flight with an uneventful landing.

[Prior to the flight] While awaiting transportation to the hotel at the airport, scheduling called indicating that there was an airplane on mechanical that we would be taking instead of our originally scheduled flight and plane. The reason given was that the original crew was running into a crew duty day issue and would operate our flight while we would wait for the broken airplane to get fixed. Scheduling asked me to notify the Captain, stay at the hotel and call back for an update. About thirty minutes later scheduling called indicating that maintenance said the airplane was ready and to come to the airport. Arriving at the airport, passengers were applauding our arrival and the gate agents started making preparation for passenger boarding. I briefly looked at the log book which indicated that the NAV data base was out of date, and on MEL. I then completed my walk around. Upon my return to the cockpit I observed that the MCDU was displaying a message to the effect that there was an incomplete NAV data upload and that the FMGC was in separate operations. I was unable to align the INS and called maintenance. Passenger boarding was almost complete. The maintenance tech aligned the INS using the overhead panel. I queried the tech as to what if any other malfunctions would be impacting our flight and specifically would the autopilot and autothrust work. He indicated that they should work. The Captain and I reread the MEL, discussed manually tuning NAV aids, got out the enroute charts and high lighted the route and discussed the flight. When we took the runway for take off, I felt that we were in compliance with the MEL and would not have any difficulty in completing the flight. At no time had anyone, scheduling, maintenance or station supervisor (who came down to check on us) indicate that the previous crew had refused the aircraft. Upon reflection of the events, I believe that the MEL was misapplied. We were working way too hard to safely accomplish what should have been a simple flight. It now appears that not only was the data base was out of date but that there was no NAV data base at all and that we had we had a more serious problem. While I had received some clues that there was something else wrong, (nav data upload incomplete, unable to align INS normally, FMGC separate operations, autopilot and auto thrust "should work") at that time I was unable to determine what the bigger problem might be until after level off at cruise altitude. In my eleven plus years with this company, I have never encountered a situation where maintenance or operations has tried to hide the actual status of the aircraft from me. My honeymoon now appears to be over. On a better day I may have had the clarity of thought to ask the right questions and get a truer picture of what was wrong with the airplane before taking off. [Suggest Company] not shop for another crew to take a broken airplane. Don't hide actual aircraft status from the crews. As an aircrew member maintain a healthy skepticism of operations and maintenance actions.

Synopsis

A320 First Officer reported events leading up to being assigned an aircraft with one FMGC on MEL. On takeoff autothrust and autopilot cannot be engaged. The flight was continued to destination using manually tuned NAV radios and Radar vectors.

ACN: 956643 (2457 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cabin Entertainment
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 956643

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I made a passing comment to the First Officer that there was a slight cigarette smoke smell in the air. He smelled nothing. Since I had just come off sick list, I figured it was just my nose acting up. Thirty minutes later the Lead Flight Attendant notified us that an armrest had been smoking and the smoke stopped when the Passenger Entertainment System was shut off. The First Officer and I briefly discussed the situation and decided to divert. Overweight landing and we flew above 250 KTS below 10,000 MSL. No further smoke was observed.

Synopsis

An A320 diverted under emergency conditions to a nearby airport when the passenger entertainment system at one passenger seat was smoking. The smoke ceased when power to the Passenger Entertainment System was shut off.

ACN: 956462 (2458 of 2908)

Time / Day

Date : 201106

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 956462
Analyst Callback : Attempted

Events

Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Attendant
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

As Flight Attendant number 2, I was responsible for briefing the exit row passengers. When I went to brief the passenger in 20A he asked me to speak in Spanish. He apparently did not speak any English. I asked the purser to help me out since he speaks Spanish. According to the Company we cannot move passengers from the exit row because they do not speak English. However as Flight Attendant number 2 I am responsible for window exits during an evacuation. While the passenger in 20A may be capable of opening the window, since he does not speak English he would not understand my commands to either evacuate or to remain seated. I cannot understand how this can be acceptable for safety purposes.

Synopsis

While attempting to brief the exit row passengers an A320 Flight Attendant discovers that the passenger in the "A" seat does not speak English and the Company allegedly does not allow passengers to be moved for this reason.

ACN: 956395 (2459 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cabin Entertainment
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 956395

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 956561

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

A Flight Attendant called and advised a passenger had reported a "strong electric smell and some smoke" at one row. She agreed there was a trace of smoke smell. She further indicated they were requesting one of our deadheading pilots to help investigate and they would call me with additional information. First Officer located the Smoke/Fire/Fumes checklist and both pilots noted multiple suitable airports near the route of flight. Weather was not a factor (clear, VFR, daytime). Captain sent an advisory note to Dispatch.

Passengers were asked to move out of the affected area and the deadheading pilot checked the area in question thoroughly. Passenger seated across from the area told the pilot checking that he had also smelled smoke, and that the audio system had static then the video screen failed. Dispatch had set up a communication link and we discussed the need to consider a diversion. Maintenance Control needed the specific location of problem and asked if the audio or video was affected. They advised we have had multiple failures of the entertainment system. From the cabin reports we knew that some or all of the video screens had failed. Maintenance did not see a need to pull system breakers but, as a precaution, the suspect entertainment system was to remain off for the duration of the flight.

The deadheading pilot found no popped circuit breakers. All electric systems appeared normal and lightly loaded. On arrival Maintenance met the aircraft and the fault was quickly discovered in the suspect location and it appears the fault was completely contained in the entertainment unit.

Narrative: 2

FA called cockpit and reported smelling smoke near row 22. Captain asked for deadheading crewmember to investigate. DH crew reported PES [passenger entertainment system] not working and residual electrical smell near row 22. Directed lead FA to shut down PES after which smoke smell went away. Captain contacted DISP and SAM. SAM indicated similar problem with 4 other aircraft in last week isolated to PES control box under seat. All concurred that threat was over and continue flight to destination. Flight landed uneventfully. Maintenance had box pulled within 20 minutes after landing.

Synopsis

An A320 flight crew isolated the source of a burning, smoky odor to the Passenger Entertainment System. The system was turned off and the flight completed.

ACN: 956369 (2460 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : MKE.Tower
State Reference : WI
Altitude.MSL.Single Value : 2000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : MKE
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : MKE

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : MKE
Make Model Name : Helicopter
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Flight Phase : Cruise
Route In Use : None
Airspace.Class C : MKE

Person

Reference : 1
Location Of Person.Facility : MKE.Tower
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 956369

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We had numerous VFR helicopters and banner tow aircraft in the vicinity of the final to 19R. An A320 came in from the west and I assigned a heading for sequencing to avoid the VFRs near the Final. I turned them to the localizer 5 miles outside the fix and cleared them for the Visual Approach. I called traffic on the banner tows and the helicopters. He had the banner tows in sight but not the helicopters. I made additional traffic calls and when I felt he was past the traffic, shipped him to Tower. At that same time the aircraft reported an RA. The pilot said he was going around so I gave him go-around instructions. Coordination between Tower and TRACON was questionable as far as who should be talking to the VFR aircraft. Tower felt TRACON should be talking to them and TRACON felt Tower should be talking to them. Airspace and/or procedures in this case are very questionable. Should VFR lingering be allowed that close to the final of an active runway? Recommendation, who should talk to VFRs working close to the Final inside the marker?

Synopsis

TRACON Controller voiced concern regarding helicopter and banner tow operations near the final approach course that caused a TCAS RA event, claiming clarity should be established as to who, Tower and/or TRACON, should handle these types of operations.

ACN: 956070 (2461 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ
Maintenance Status.Released For Service : Y

Component : 1

Aircraft Component : Air/Ground Communication
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : VHF
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 956070
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

[We were] unable to receive clear ATC communications on all three radios and all frequencies. [We] tried for approximately twenty minute period while flying through a stratus layer of clouds associated with thunderstorms in the area. Also [we were] unable to establish any communications with company Dispatch during this period. [There was a] constant loud squelch/interference noise disrupting ability to hear any clear ATC reception. It appears we were able to still transmit, because on occasion we could hear ATC responding to our attempts to call them. 

We broadcast our intentions and route in the blind on both ATC assigned frequencies and 121.5 while attempting other ATC frequencies noted on Enroute Chart, as well as trying to reach any other aircraft that might have been in area. Finally were able to re-establish communications with ATC once we were clear of all clouds. We were able to stay clear of all clouds for the rest of flight and landed without further incident. Dispatch/Maintenance notified once in the clear and Contract Maintenance met aircraft upon arrival.

Suspect faulty RF5, according to Mechanic on arrival....flying in stratus cloud associated with Thunderstorms. We all know that the Airbus definitely has a problem with radio reception while flying in any precipitation...This by far was almost a total loss of communications due to equipment and associated weather.

Synopsis

An A320 Captain reported being unable to receive clear ATC communications on all three radios and all frequencies during a twenty minute period while flying through a stratus layer of clouds associated with thunderstorms. Constant loud squelch/interference noise disrupted ability to hear any clear ATC reception.

ACN: 956017 (2462 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 956017

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 3900
ASRS Report Number.Accession Number : 956232

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The First Flight Attendant called the cockpit to inform that there were sparks coming from the floor below a seat and was using a fire extinguisher and that she would get back to us. I discussed diversion options with the First Officer. Shortly Flight Attendant advised me that the extinguisher did not put out the sparks and were deploying the second extinguisher. I declared an emergency with ATC and advised them that we needed to divert. I ran the smoke fumes fire QRH while the First Officer flew the aircraft. I spoke with Flight Attendant several times as she advised that the sparks did not subside. I told her the time to land and to prep for evacuation but would check in again with her. I notified Dispatch via ACARS that we were diverting and to have fire and rescue ready. Flight Attendant advised that there were still sparks but not as bad. We landed and taxied clear of the runway. Flight Attendant informed me that the sparks had stopped so I elected to taxi to gate and deplane via jetbridge. Passengers were deplaned quickly without there bags. Fire and rescue found a bare wire under the seat that was wet and appeared to be the cause. We had to totally power down the aircraft to get the sparks to completely stop.

Narrative: 2

Flight Attendant called back with more details: electric sparks flying in emergency exit row. Captain made decision to divert; by then we were 64 NM west of the divert airport. Captain declared emergency with ATC. ATC initially slow to react then gave us direct to the airport and lower. I was flying aircraft, started descent, input direct to the airport, and then changed destination, descended at 320 KTS with full speed brakes. Captain was busy coordinating with ATC, Dispatch, and cabin, ran "smoke/fire/fumes" QRH, and got ATIS. I told ATC we would land XXR, input landing data, activate and confirm, kept speed at 320 KTS with full speedbrakes to 3,000 FT, then slowed and configured aircraft. Approach did excellent job vectoring us for altitude loss and put us in a perfect position for landing XXR. [It was] VMC at the airport; [our] landing [was] normal. Cleared runway and taxied to gate followed by emergency vehicles. Deplaned passengers via jetway. [We] had to completely power down aircraft to stop sparking. 

I still have no idea how much time passed between getting first call and landing. Probably the usual 10-15 minutes; in my mind, it feels like a blur of focused, coordinated activity. Aircraft was under maximum landing weight. Captain and I were debriefed and answered a series of questions. I thought the questions were well formulated and the answers offered good options. However, I feel they were asked too soon after the stress event: I answered truthfully and the way I felt at that time but I still had a fairly large amount of adrenaline in my system. Had I been asked the same questions 2 hours later, I think some of my answers would have been different. Both Captain and I flew the continuation of our flight. I realized after takeoff that I was in my "coming down from the adrenaline rush" process; I felt shaky and unfocused during the flight. In hindsight, I feel that I should not have been flying that leg AT THAT TIME.

Synopsis

A320 flight crew is informed of electrical sparks beneath passenger seats in the cabin. Crew diverts to the nearest suitable airport while the Captain preforms the QRH procedure for smoke and fumes and flight attendants use fire extinguishers on the sparks. Flight lands safely and passengers are deplaned at the gate. All power is removed from the aircraft before the electrical sparking stops. The crew accepts assignment to continue to destination in another aircraft.

ACN: 955760 (2463 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 4000

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 955760

Person : 2

Reference : 2
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 9300
ASRS Report Number.Accession Number : 956022
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Captain was flying the right base Visual Approach to 28R in San Francisco. NORCAL gave us a heading of 100 degrees down the bay, and then a slight turn towards the airport of 120 degrees just past the San Mateo Bridge. The weather was essentially clear and a million. We were descending through 6000 FT MSL, with a clearance for 4000 FT at the time. NORCAL then called our traffic for 28L, a MD-80, which was descending and located right around the Dumbarton Bridge, and they said the MD80 had been cleared for the Tipp Toe Visual Approach to 28L. I identified the aircraft visually, pointed it out to the Captain, and we agreed that was the aircraft, and called it in sight. 

NORCAL then turned us further towards 28R, approximately 200 degrees, cleared us for the Visual Approach to 28R, and told us to maintain visual separation with the MD80, all of which I read back. As the Captain maneuvered towards 28R, the MD80 continued to get closer, and we got a traffic advisory from our TCAS, we never got a RA. I considered asking the Captain if he would like to go TA Only at that point like we often do on visuals at SFO, but reconsidered without saying anything, because the MD80's vector towards us looked a little too aggressive for my liking. NORCAL asked the MD80 if they had us insight, and they said no. The MD80 continued to get closer, and eventually flew right over the top of us about 1000-1200 feet above us according to our TCAS--we were between 3000-3500 FT MSL and roughly 3-5 miles from the San Mateo Bridge at that point, even though we were well East of the 28R center line at that time. I told the Captain at that point that maybe they were going to land at Oakland, because it looked like they had no intention of turning towards SFO. Finally, they made a turn towards SFO and us, acquired us visually, and ended up about a 1/2 mile behind us, roughly 800 feet above us, and well right of us, even though they were supposed to be landing on 28L. I had them in sight at that point again after losing them after they flew over the top of us. At that point, NORCAL asked them again if they had us in sight, and what they wanted to do. They replied that they did have us insight now, but were uncertain how to proceed. 

NORCAL initially tried to hand us over to tower next, and then rethought that plan, and then I responded by asking him if we could land on 28L since that was the safest way to proceed from that point, and let the MD80 land on 28R, since they were already on that side. NORCAL agreed, cleared us for the Visual Approach to 28L, handed us to tower, and the rest of the approach and landing was uneventful. After the flight the Captain learned from an ATC Supervisor that only one NORCAL controller was on duty at the time, controlling both runways, instead of the normal two, one for 28L, and one for 28R. We felt that maybe that contributed to his inability to see that the MD80 was flying into our approach corridor until after the fact.

Narrative: 2

We were on a right downwind to Runway 28R for SFO. NORCAL asked if we had a MD80 for the parallel in sight, to which we said yes. They gave a base turn and cleared us for the Visual to 28R, also after the MD80 had us in sight cleared them for 28L. Flying the radial for the Quiet Bridge we noticed the MD80 converging but not descending. As I slowed to put flaps out and descend, the MD80 was getting rather close, this was just outside San Mateo Bridge. The MD80 then went through the final for the left, wings level and passed overhead by about 1000ft. We got a TA but no RA. They were still wings level, but now we were concerned if they would descend rapidly. As they flew past 28R I had to ask my First Officer if they were supposed to be going to SFO. He wondered aloud if they were headed for OAK. NORCAL was now querying him and he was 800ft above at our 5 o'clock. My First Officer astutely asked them for us to sidestep to the left as we went over San Mateo to get more lateral separation. They approved and we switched to tower and continued for an uneventful landing. Upon arriving at the gate and shutdown,checklists, I called NORCAL and ended up talking to the supervisor on duty. who was not aware of the incident but did call back. After listening to the tapes, the supervisor concluded that the MD80 was indeed at fault and sounded a bit confused.

Synopsis

Air Carrier landing SFO cleared for a Visual Approach to Runway 28R observed an MD80 assigned Runway 28L overfly. The reporter suggested to ATC that the runway assignments be switched. ATC agreed.

ACN: 955672 (2464 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 12000

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Airspace.Class E : ZHU

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 184
Experience.Flight Crew.Type : 2100
ASRS Report Number.Accession Number : 955672
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Training / Qualification

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

With aircraft cruising at FL310 on arrival into IAH, ATC gave clearance to cross RIICE at 12,000. Pilot flying put 12,000 at RIICE in FMC and pushed for managed descent. Pilot not flying confirmed entry and pilot flying also checked Prog page, which showed below required path. The pilot flying thought the FMC was handling the descent properly and briefed the approach to Runway 27. When 12 NM from RIICE, pilot flying realized there was still 10,000 FT to descend to cross RIICE at 12,000. Pilot flying directed pilot not flying to tell ATC that we would not make the crossing altitude at RIICE. ATC responded to do the best possible. Soon thereafter, ATC gave a frequency change and new controller gave a descent to 7,000. The aircraft crossed RIICE at approximately 14,000 and continued descending to 7,000. The remainder of the approach and landing was uneventful. Neither pilot flying nor pilot not flying could figure out how the altitude constraint was not met by the FMC when originally all indicators looked good. There was a distraction by being given an unexpected runway while descending. The briefing, although fairly short, was enough to divert attention long enough to loose situational awareness concerning the crossing restriction.

Synopsis

A320 Captain reports missing a crossing restriction descending into IAH. The crossing restriction had been entered into the FMGC and a managed descent initiated. A runway change was entered during the descent.

ACN: 955183 (2465 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : APU Controls
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Mininum Equipment List (MEL)
Manufacturer : Airbus

Component : 3

Aircraft Component : Hydraulic Fluid
Manufacturer : Airbus

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 955183
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
ASRS Report Number.Accession Number : 955898

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Human Factors

Narrative: 1

A320 Logbook page write-up for hydraulic leak/fluid streaming from under aircraft left stabilizer and elevator. Visible shine of fluid seen on surfaces; no hydraulic fluid found on ramp. Large area of paint lifting, crinkling and peeling off elevator, I believe, due to corrosive Skydrol hydraulic fluid. I was advised that no hydraulic leak history was documented.

APU [had been] MEL'd in Logbook. APU Auto Shut-down at gate five days earlier. MEL maintenance placarding procedure done by Maintenance. Under MEL Placarding Procedure, item # 4 was not complied with. Downgrade aircraft status as appropriate for each restriction under ''Operational Restrictions'' and install placard(s). Operational Restriction [was] ''ETOPS Operations Prohibited''. Placard sticker not performed.

Aircraft Logbook page read ECAM STATUS: ONE PACK ONLY IF Wing Anti-Ice (WAI) ON. Write-up for First Officer seat electrical motor, full aft cutout sensor inoperative. Motor continues to run resulting in clutch jumping. The First Officer's seat was previously written-up a few days earlier. Standard maintenance action [was] ''unable to duplicate''.

I was distracted with hydraulic fluid situation, felt pressure to depart as soon as possible (ASAP) and did not thoroughly read the placarding procedure for the APU MEL. I will try not to let others pressure me.

Narrative: 2

Our flight release had us tanker fuel to our destination due to cost. During taxi out, we received our Weight and Balance via ACARS which showed our Maximum Takeoff Weight (MTW) was restricted by landing weight and contained a requirement to burn additional taxi fuel. The additional fuel was used during taxi. Our destination's ATIS reported Runway 35R in use and the FMS estimated fuel remaining landing Runway 35R at our destination of 13,200 LBS and landing weight of 142,100 LBS (Maximum Landing Weight 142,198). When we contacted our destination's Approach, they changed our arrival runway to 18R. After the FMS changes were made we did not notice that with the reduced distance and flight time to our destination, the estimated fuel remaining of 13,600 LBS resulted in an overweight landing of 142,500 LBS. The Captain decided not write-up the Overweight Landing since it was minimal.

Synopsis

A Captain and First Officer reported distractions caused by numerous maintenance discrepancies and MEL deferral procedures not complied with, on-time departure pressures, having to "tanker" additional fuel on their A320 as well as arrival runway changes resulted in a 300 LB overweight landing.

ACN: 954965 (2466 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cabin Lighting
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 2300
ASRS Report Number.Accession Number : 954965
Human Factors : Other / Unknown
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

At approximately 100 miles out of ZZZ, the 'A' Flight Attendant (FA) came up to the flight deck as the First Officer (FO) had requested a bathroom break. When he left, she informed me that the lights on her FA panel and the FA aft panel had "been flickering on and off like crazy." She asked if this was serious because this airplane (the Airbus) has a lot of electrical problems. I told her that it is something we need to look at so I need to know exactly what is going on. As we were talking I pulled up the electrical page to look for any abnormalities in the electrical system. All indications were normal. I told her when I step back to use the restroom I will look at the lights she is talking about to assess them. I put in a call to Dispatch thru the ACARS so that I could speak to both Dispatch and Maintenance Control about what the FA was reporting.

I told her I would watch for the cycle she was talking about. When I used the facility, I cycled the lock a few times to see if there was any issue with the lights in the lav. They functioned normally. When I stepped out of the lav, I said the 'A' FA had reported the panel lights in the forward and aft galley had been flickering on and off and asked if that is what he was seeing in the aft galley area as well. He said yes, they were doing the same thing back there. I turned to the FA panel (near door 1L) to look for any failure lights, put my hand on the panel and asked him to confirm that they were getting the flickering lights on their FA panels. He said yes again.

I got a hold of Dispatch and we got patched through to Maintenance Control. I told Maintenance Control what the FA had been reporting and relayed to him the length of time she had said the lights had been flickering. [He] did some research in his manual and referenced the page on generator shorting. I also had the manual out to that page. He said we could have that situation. I told him that all our electrical indications were normal and we had no display blanking issues or ECAMS. Maintenance Control said that we could have normal indications, but one of the symptoms is cabin lighting blinking issues. He referenced his manual and the bulletin that had come out about that. He said that they had done some maintenance on the #2 generator in May and it could be a shorting on that connector. 

I asked him what he suggested as I was showing all normal indications. He said, using Captain's emergency authority, he suggested disconnecting the #2 CSD. Let me point out that Maintenance Control was very through in his explanation and we had a good discussion about the situation. With Dispatch on the radio, I told him that if we were talking about taking a generator off line that we were not continuing onto destination and we needed to look at coming into ZZZ1 as we were about 100 miles out of ZZZ1. They both concurred since the weather in the destination area was bad and if there was an issue with the generator, they wanted to have the jet looked at. He asked if there were any failure lights on the FA panel or any ECAMS. I said, "No ECAMS, but let me verify with the FA one more time." I called the 'A' FA and asked her. She confirmed they had no failure lights. Maintenance Control suggested it was best to get the jet looked at given the similar issues on the fleet with generator problems. I concurred. I briefed FO on the conversation and said we were going to ZZZ1. He told ATC while I briefed the flight attendants and passengers. I asked FO to get lower so that I could start the APU so we could set up to take the #2 generator off line. We were going to set up to do so and I had the check list out. The FO and I discussed the taking the #2 generator off line. Neither of us were certain there was a short going on, but we did not want to disregard Maintenance's advice. We determined the most prudent course of action was to set up to take the generator off line, checklist out, but since we were so close to ZZZ1 (now below 21,000 FT with the APU running and less than 60 miles to ZZZ1) we would keep the configuration as is and land. We had all checklists complete, routing in the box, announcements made to the passengers, and briefed the FA's. She asked if I had reset the generator as the lights had stopped flashing. I said no, we had not reset anything. She then said, "Well it [was] only the reading lights above her galley area." I stopped and asked her to confirm this change. I told her she had stated to me several times the lights on the FA panels in the forward and aft galley had been flickering on and off like crazy. She said she used the wrong terminology and it was the overhead reading lights not the panel. I asked her if that was the same in the aft galley. She said yes.

I called Dispatch right away and conveyed that the FA's were now reporting that she used the incorrect terminology; it was not any of the panels but the overhead reading lights in the forward and aft FA area. I told him we wanted to continue to destination if he was good with that. He said he was, but that destination was now in a hold due to severe weather that built just to the west of the airport and ZZZ2 was ground stopped and asked about our fuel situation. I asked him what the burn was to destination. He said 8,000 [LBS]. We currently had less than 16,000 [LBS] on board and we were just west of ZZZ1. We determined with the burn, landing fuel, fuel for alt, [that] we had no hold fuel. We discussed the fuel and concurred we needed to land in ZZZ1 and get more fuel.

Before we took off again, I spoke with the 'A' FA. She said that the lights had been flickering like crazy, but she had mistakenly said it was the lights on the FA panels. She said she had spoken to the aft FA and asked him if he had said anything that lead to the miscommunication. She said he told her that when I touched the FA panel (to verify what was happening and make sure I understood what areas they were speaking of) he had said those were the lights on the panel, but did not mean to indicate those were the lights that were flashing.

Synopsis

A miscommunication between an A320 Captain and the 'A' Flight Attendant lead the Captain to believe that a generator arcing event may be occurring and a diversion for maintenance is planned. Before landing the miscommunication is cleared up but destination weather and fuel remaining make the diversion necessary.

ACN: 953612 (2467 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 38000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLA

Component

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 2900
ASRS Report Number.Accession Number : 953612
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Airborne we got a LGCIU 2 ECAM. No reference action associated. The Flight Manual was referenced and indicated no number 2 thrust reverser would be available (checklist was a little confusing). We could not find a weight penalty associated. Called Dispatch and they informed me that there was a weight penalty and that the required landing distance was only two hundred feet short of the entire runway length available at SNA. We have a system fault reset in our book that is applicable on the ground but not in flight. Maintenance said that we could cycle the Circuit Breaker (CB) in the air, but obviously, as we already were aware, this constitutes using Captain's emergency authority. I did not deem this an emergency from the information I had as it appeared to be only an issue with the runway length in SNA (checklist again very lacking in complete systems lost). Dispatch, Maintenance and I conferred and I decided that with the information I had at this point going to LAX (our designated alternate) would be a suitable safe option and make the stopping distance a non issue. I realized we still had 2.5 hours or so to consider our options. I still wanted the MEL information. I wanted all possible information, so I asked Dispatch to have the MEL for the issue to be sent to the aircraft. Upon receiving the MEL it was quite lengthy, involved numerous items lost once on the ground (reference MEL for LGCIU2 inoperative). With this further information, I determined that no matter where we landed that the situation would have negative consequences on safety, even just taxiing in at our destination. With this in mind, and after getting assurance from Maintenance that there were no additional consequences that would be generated by cycling the LGCIU 2 CB I opted to use my Captain's emergency authority and did so. The CB was uneventfully cycled, the system was fully recovered, and we opted to continue to our original destination of SNA. As a caveat I can only assume that the cycling of CBs in flight being deemed a use of Captains Emergency Authority was generated with some specific instances in mind. The criteria is entirely too broad. This was a simple reset procedure that should not have required me to judge whether to use this authority or not. I feel the emergency authority to recycle CBs in flight designation should be reviewed. As pilots it appears to be a simple, obvious procedure in this instance that generated a gray area for us to operate in. I understand that there are CBs that influence numerous systems. This CB just dealt with LGCIU 2 which was inoperative already with the commensurate repercussions. The entire legality exercise was wholly unnecessary.

Synopsis

A320 Captain experiences an LGCIU 2 ECAM at FL380 and learns that this failure will render the number 2 thrust reverser inoperative. The destination runway is short and Maintenance offers a circuit breaker reset procedure if the Captain elects to exercise his emergency authority. This is successfully accomplished.

ACN: 953531 (2468 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : PCT.TRACON
State Reference : VA
Altitude.MSL.Single Value : 9000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class B : IAD

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : PCT
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class B : IAD

Person

Reference : 1
Location Of Person.Facility : PCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 953531
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Giving Instruction during the main arrival bank of the day. ZDC was late on getting aircraft down and switching to my position and was the initial reason our aircraft were being vectored abnormally high into our sector for sequence on the arrival. The complexity of the sector was increasing so I took over from the developmental, and gave an altitude of 9000 to a south bound ACR Y, when I meant to restate 8000, because of traffic opposite direction at 7000. ACR X was opposite direction to ACR Y as well and some 10 miles away. I thought I had lateral and vertical, but when I noticed the situation, I turned ACR Y right to a 270 heading. No response from ACR Y. I turned ACR X East to 090 with no response. I again did this to each aircraft with no response from either one. Finally after issuing a Traffic Alert to ACR X, who acknowledged turning East to 090. It was a case of meaning one thing and saying another. Recommendation, should have put ZDC into a hold earlier than I did. Should have re-scanned my altitude assignments again, especially during high complex situations. Focusing more on all prospective situations. Make sure I say what I mean and mean what I say.

Synopsis

PCT Controller described a loss of separation event resulting from issuing one altitude but thinking another, the reporter adding that ZDC should have been placed into holding much sooner.

ACN: 953377 (2469 of 2908)

Time / Day

Date : 201106

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 953377
Human Factors : Physiological - Other
Analyst Callback : Attempted

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
When Detected : Taxi
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

[We had a] foul chemical like smell throughout cabin starting while we were on a tarmac delay [which] increased on take off, decreased some inflight and increased again on descent. The smell caused the crew to become nauseated with headaches. Crew reported to cockpit and cockpit already reported to Operations. Maintenance investigated on landing and the next flight was canceled.

Synopsis

An A320 Flight Attendant reported strong chemical smells beginning during a taxi delay and prevailing at different intensities throughout the flight. Maintenance action at their destination caused the cancellation of the aircraft's subsequent flight.

ACN: 953058 (2470 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Records Complete : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Nose Gear Tire
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 953058
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Primary Problem : Manuals

Narrative: 1

I was assigned to work on the aircraft tow crew. We were also assigned to work a #3 service check. During the check, we wrote up both Nose Landing Gear (NLG) tires as "worn." During the replacement of the NLG tires, I noticed that something was missing. According to the Maintenance Manual (MM), the part missing was called a "casing" (reference MM-32-41-12, page 403 Item #9). At this point we completed the RH NLG tire installation and generated a Maintenance write-up item for the missing [casing].

Callback: 1

Reporter stated there is still some confusion about whether or not all the Airbus A320s require the "casing" part he found missing, or whether there are different types of "casings" depending on whether the A320 is an older (classic) or a newer model A320. The Aircraft Maintenance Manual (AMM) refers to the part as a "casing," but their Illustrated Parts Catalog (IPC) calls the same part a different name; not casing.

Reporter stated the "casing" is an Airbus required item for the nose tire installations. When he made the write-up for the missing "casing," Maintenance Control decided to rob (cannibalized) one from another A320 and later had another Mechanic install and sign-off the write-up reporter had made. But his A320 was later grounded because the casing that was robbed was not effective on his A320. Some mechanics believe the "casing" was originally designed to support a cannon plug and connector for an onboard nose tire pressure sensor system on Airbus aircraft, which was not an option in use by his air carrier.

Synopsis

A Line Mechanic reports noticing a part missing at the left nose gear axle during replacement of the left and right Nose Landing Gear (NLG) tires on an A320 aircraft.

ACN: 952803 (2471 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 1100
ASRS Report Number.Accession Number : 952803
Human Factors : Situational Awareness
Human Factors : Time Pressure
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Takeoff aborted on Runway XX due to ECAM for Engine Thrust Levers not set for takeoff. After lining up the aircraft on the runway centerline, I transferred control to the First Officer who was the pilot flying for this leg. The First Officer set flex thrust for takeoff and the aircraft veered left of centerline. The First Officer corrected with almost full right rudder. We then received an ECAM and I took control of the aircraft and aborted the takeoff. We were approximately 65 KTS when I began the abort. The auto brakes did engage as the aircraft continued a slight acceleration during the abort and the aircraft was stopped using max auto brakes. I taxied clear of the runway, shut down the number two engine and called Dispatch. After consulting with Maintenance about the brake temperatures, I taxied back to the gate where the aircraft was eventually taken out of service.

Synopsis

An A320 swerved left as takeoff thrust was set and the ECAM alerted ENG THR LEVER NOT SET FOR TAKEOFF so the Captain rejected the takeoff, shut the number two engine down and returned to the gate where the aircraft was removed from service.

ACN: 952617 (2472 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SEA.Airport
State Reference : WA

Environment

Flight Conditions : Marginal

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : S46
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class B : SEA

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : S46
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Airspace.Class B : SEA

Component

Aircraft Component : PFD
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 952617
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Environment - Non Weather Related

Narrative: 1

[We] received a TCAS RA to adjust vertical speed. Problem, crews with white shirts, morning sun reflecting on MFD displays. TCAS RA information on PFD was so impacted that it was unreadable without focusing on the PFD versus looking out the window at the threat. It occurs to Captain [reporter] that 100% of the RA events he has experienced in daylight condition have had this issue. Our training materials complain that we over respond to the RA and should only respond to the extent necessary. On this event Captain had not engaged the autopilot yet. The TCAS RA always comes as a surprise. ATC had just pointed out the other aircraft and indicated they were stopping 1,000 FT above our assigned altitude. Captain responded by reducing vertical speed, tried to determine if he was in the proper response range (in the green), discovered the TCAS information was so dim compared to the remainder of the PFD that it was unreadable, correctly leveled the flight at the assigned altitude and LOOKED out the window at the conflicting aircraft.

Synopsis

An A320 Captain reported that TCAS RA displays during bright daylight condition are too dim and interpreting them accurately distracts the flight crew from looking outside for visual confirmation of the threat.

ACN: 952156 (2473 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : PHL.Airport
State Reference : PA
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft : 1

Reference : X
ATC / Advisory.Ramp : PHL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Ramp : PHL
Aircraft Operator : Air Carrier
Make Model Name : EMB ERJ 170/175 ER/LR
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 952156
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Flight contacted Ramp for taxi after pushback. Flight was cleared left turn on J to Spot 11. Flight followed an EMB170 out of the alley. At the top of the alley, another EMB170 approached from the right on J. It appeared that they were not going to stop, so I emergency stopped the aircraft. The EMB170 turned right on K3. We queried Ramp as to our clearance and they confirmed: left turn J to Spot 11. We asked about the aircraft on our right, Ramp said something about "You guys have your own agenda", as if to imply that we taxi at different rates and it is not his problem. He took no responsibility for aircraft separation. I said "We are trying to prevent a collision." We were on our own, at that point, so I waited until the coast was clear then turned onto J and went to Spot 11. 

We were distracted and incredulous at this blatantly political response to our request for traffic separation on Taxiway J, one of the busiest in the system. Ramp Control is controlling Taxiway J and not providing instructions to aircraft on who is supposed to follow who when there is an incursion/near miss. Taxiway J should be under control of ATC Ground Control, or Ramp should provide positive control to ensure adequate separation between aircraft. Safety culture is flawed, and we need upper management to set the tone that this type of behavior is unacceptable.

Synopsis

A320 Captain reports a ground conflict with an EMB170 on Taxiway J at PHL after being cleared by Ramp Control to Spot 11 via J. Reporter suggests that Ramp Control is not properly controlling aircraft on Taxiway J.

ACN: 952155 (2474 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 952155
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 952152

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : Taxi
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We taxied out 2 hours 20 minutes late because of maintenance. On taxi out a Flight Attendant called up and said a passenger reported hearing a strange noise that he insisted was not normal. We then had him ask a deadheading Airbus Captain what he thought of the noise who said (I believe) that it sounded like the PTU might be coming on and off. However, indications in the cockpit were normal so we continued. After taking off and climbing to cruise altitude, the Flight Attendant called again and said the deadheading Captain would like to come up to the cockpit. Once in cockpit he said he could still hear the noise in the cabin. We checked hydraulic page and saw that PTU was running and green and yellow quantities were both below normal indication and the indicator was "bouncing". After watching for a while the yellow quantity seemed to be getting lower. The Captain contacted Dispatch and Maintenance through ACARS and mad the decision to divert. 

During the descent the master caution with ECAM for yellow system low quantity and later low pressure was indicated. At that time the Captain declared an emergency. At 4,000 FT we asked approach for delaying vectors for approximately 15 minutes to burn off fuel to avoid an overweight landing. Landing was uneventful.

In retrospect, we should have investigated further on the ground or returned to gate.

Narrative: 2

Upon preflight, hydraulic system green and blue quantity were over serviced. Upon cargo door closure, yellow hydraulic system quantity was found to be over serviced. Maintenance corrected the conditions.

[Maintenance determined] the PTU valve "O" ring failed.

Synopsis

Despite advisories of unusual sounds on taxi from both a passenger and a deadheading, qualified in type, Captain the flight crew of an A320 elected to continue the flight and later encountered multiple hydraulic system anomalies eventually determined to have been the result of a faulty PTU "O" ring.

ACN: 952147 (2475 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MSY.Airport
State Reference : LA
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 300
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 952147
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After push back and release from guidance with both engines running, the push crew approached the aircraft on the left side and gave the stop signal and then reconnected to the headset. I was informed by the tug driver that they had seen a latch possibly unlatched on the aft cargo door. I informed her not to approach the aircraft until we had shut down the number 2 engine. As the First Officer was setting up to do this, a ramp employee walked behind the running number 2 engine and re-checked the latch. As soon as the tow operator told me what had happened, we immediately shut down the number 2 engine; however, he was already behind it. Both the tug operator and the operations agent who could see what was going on from operations were shocked at how dangerous his action was. I strongly feel that the MSY station chief should instruct MSY personnel in the safety fan of a jet engine, as someone could have been seriously injured in this event.

Synopsis

An A320 Captain reported a ground crewman walked behind the operating right engine of the jet to check the closure status of the aft cargo door.

ACN: 951996 (2476 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Flight Phase : Initial Approach
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Component : 3

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 951996
Human Factors : Confusion
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At the gate the Captain's altimeter failed, MCP was replaced. In cruise FMGC displayed independent operation, #1 FMGC was locked, this was followed by ECAM NAV GPWS fault, amber, GPWS...off, blue. GPWS system switch was displaying a fault, it was selected off, however the ECAM GPWS...off remained blue, the status page was selected and reviewed, inop CAT II, GPWS, as expected. Maintenance Control was contacted looking for direction with the #1 FMGC locked up. Maintenance Control directed us to reset circuit breakers b-1, b-2, both green monitored circuit breakers, they wanted them out until ECAM showed them pulled. After several minutes the ECAM never recognized the circuit breakers pulled. Maintenance Control directed to push back in, left FMGC still locked, Captain's map locked and could not select any mode or display, First Officer's side in same mode and range. On descent slowing for a visual when flaps 1 selected, status page displayed inop CAT II, GPWS, as expected, and inop CAB PR 1 and 2, with aircraft flight level and cabin altitude target reference settings displayed. I pushed recall and got NAV GPWS fault amber, and blue GPWS...off, I pushed clear then for the first time I saw CAB PR SYS 1 and 2 fault displayed directing mode sel in blue to be selected to manual, it was. The ECAM did not recognize the switch was selected to manual, we were on final and I was moving the outflow valve to full up at this time. The First Officer was flying in managed speed, it failed and he selected approach speed from the FMGC 2, the Captain's airspeed was lost displaying 4 red flags, spd sel, spd, spd limit, V1 inop..it appeared the ECAM was locked and did not recognize switch or circuit breaker position, however the clear, status, recall buttons did work.

Callback: 1

The reporter states that the aircraft was returned to service after two global resets, the first reset was not successful. A global reset entails completely powering down the aircraft, including disconnecting the battery. The fact that the ECAM system could fail in this manner was very troubling to the reporter. Significant faults and warnings could go unreported by the system.

Synopsis

A320 Captain experiences multiple system malfunctions in cruise, including FMGC #1 locked, GPWS fault, and ECAM malfunctions.

ACN: 951992 (2477 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 24000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18900
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 951992
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 951997
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While on the arrival we got a call from flight attendants that they smelled something burning in the AFT cabin. We asked them to verify the kind of smell and they said it was not like electrical smell, but it smelled like burnt popcorn. The First Officer was communicating with them, so I asked him to have the jumpseating commuter pilot to verify the smell. The jumpseater verified that there was a burning smell, and all of them verified there was no smoke associated with the smell. I asked ATC for expeditious handling since both the First Officer and I did not perceive this as an emergency situation. We just did not want a long downwind to final. ATC apparently declared an emergency for us and assigned us Runway 7. After acquiring the field ATC cleared us for a visual to Runway 7. The Approach, landing and taxi in were uneventful. Emergency response vehicles met the aircraft and followed us to the gate.

Narrative: 2

Passing FL250 got a call from a Flight Attendant that there was an odor near row 25, smelled like burned popcorn. After landing I called Flight Attendant who told me odor had dissipated, no longer smelled. We taxied to the gate

Synopsis

An A320 Flight Attendant reported a burnt popcorn smell around row 25 as the aircraft began its descent for landing. The flight was expedited to the airport, ATC declared an emergency and after a normal landing the smell dissipated.

ACN: 951986 (2478 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZFW.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZFW

Component

Aircraft Component : Air/Ground Communication
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 9100
ASRS Report Number.Accession Number : 951986
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Enroute we deviated east of two Level 4-5 thunderstorms downwind by 45 NM. As we entered some remaining moisture from a dying downwind cell (looked like a thumb print on radar, not fully colored in and only green spotty), we began to see St. Elmo's fire and the radios got loud with static. In about a minute the radios were completely unreadable. We advised ATC of static and that they were very unreadable. We could faintly hear their reply. We were on a heading of about 130 and advised in the blind. We also broadcast on 121.5 and sent a text to Dispatch. The loss of radios lasted about 5 minutes or so where the radios were severely loud and unreadable! There was another aircraft, I believe an A340 that was 1,000 FT below us within a few miles that ATC had alerted us on. The concern is that radios should not fail so badly in moisture and conditions we were in as I have been in similar conditions hundreds of times without any failures or such static. I have also been in light moisture (green return no cells) conditions and experienced static and wrote the radios up to maintenance. They advise the antenna casing gets cracks and allows moisture to get in thus static is created. Note both radios (Captain/First Officer) had the same problem on different frequencies. This was a very disconcerting and unsafe event, especially with multiple airplanes deviating, vectoring, and switching altitudes for thunderstorm avoidance.

Synopsis

A320 Captain describes loud static in the radios while deviating around thunderstorms, making communication impossible.

ACN: 951901 (2479 of 2908)

Time / Day

Date : 201106
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 37000

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZHU

Person

Reference : 1
Location Of Person.Facility : ZHU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 951901
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 checked on ZMA Miami Controller's frequency without a hand off, the Miami Controller called the D-Side thinking we had an Operational Deviation, the D-Side told me to flash Miami the hand off. I advised the D-Side that I had not shipped the A320 nor had I even RADAR identified the aircraft yet. The A320 advised that the Commercial Charts instruct them to contact Miami 10 minutes prior to entering their airspace. We have aircraft checking on our frequency that Merida is working and tying up our frequencies when we should not be working them, they are out of communication range and we do not have updated control information on them coordinated by Merida. We cannot control these aircraft because they are not in our airspace, we are blocked from communicating to the aircraft we are working and attempting to separate in a NON-RADAR environment, and the frequency coverage is extremely weak because the aircraft are out of frequency range. Controllers are being interrupted from the aircraft they are working and searching for aircraft they are not working, their radios congested.

Synopsis

ZHU Controller voiced concern regarding Commercial Chart instructions for aircraft, still under Merida Control, to contact ZMA ten minutes from the boundary, the reporter noting this practice is a safety issue.

ACN: 951406 (2480 of 2908)

Time / Day

Date : 201105

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 951406
Human Factors : Physiological - Other
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Shortly after takeoff I noticed doors 2L and 2R were not armed. Why? 

Our inbound flight was delayed more than 2 hours so the turn time was very quick. The Agent was very hurried to get the plane cleaned and get it out ASAP. I was greeting passengers. When they were all boarded, the number three Flight Attendant came to the front to say [that the] exits were briefed. At that point we were all near the front door.

The Agent closed the door and pulled the jetway and the Purser said to arm doors. Number three and I then walked to the back of the plane, planning to arm the doors. As I left the front of the plane I realized I had a personal problem. I walked back to get something out of my bag and told my partner I needed to take care of something and went to the lavatory. She, at that time, was also getting something out of the other lavatory for a passenger. When I came out she was in the jumpseat, so I put my bag away and sat down, forgetting to arm my door.

Shortly after takeoff, I noticed her door wasn't armed, then looked at mine and realized both doors did not get armed. We then both armed our doors.

We later tried to go over the reasons why this happened. I believe the number one reason was trying to rush our departure combined with distractions that happened right as we needed to arm our doors.

Synopsis

Caught up in a maelstrom of activity and the need to address a personal problem as their departure was rushed due to a late arrival, two flight attendants aboard an A320 realized only after takeoff that neither had armed their door for departure.

ACN: 950592 (2481 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZFW.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : VFR Route
Airspace.Class A : ZFW

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 950592
Human Factors : Confusion
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Physiological - Other

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

On the arrival into DFW we chose to divert to a more westerly airport for fuel due the the weather over the field. We received vectors of a heading and altitude. After leveling off at FL270, we experienced severe turbulence for approximately 20 seconds. We lost about 400 FT, airspeed fluctuated +/- 30 KTS, but also I was trying to slow us down at the same time. The autopilot never kicked off, although after the worst of the turbulence I turned it off myself to level the aircraft back up to the assigned altitude. We received no warnings or ECAM's; we were in VFR conditions; the radar never indicated any disturbance. We checked with the cabin crew and crew and passengers were OK, just shaken up. We informed ATC and also Dispatch when we landed.

Synopsis

An A320 experienced severe turbulence over western Texas during night VMC without warning and with no injuries.

ACN: 949790 (2482 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 949790
Human Factors : Other / Unknown

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I had turned on to the lead in line and the lead was marshaling me into the gate. While well into the process a white truck with yellow doors drove between the aircraft and the Marshaller. While I did not have to slam on the brakes to avoid the truck it was an unsafe condition. Driver [was] not paying attention to ramp conditions. We need to instill a greater sense of safety awareness to the ramp workers.

Synopsis

A320 Captain reports a ramp vehicle driving between the Marshal and the aircraft during the final stages of parking at the gate.

ACN: 949742 (2483 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Pitot/Static Ice System
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Air Data Computer
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 949742
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We arrived at the aircraft and there were two MELs on the Maintenance Release: Chapter 22-63-01B Yaw Damper 2 Inop and Chapter 30-31-03C Anti-Ice First Officer Static Probe Inop. The logbook was not on the aircraft. I contacted Maintenance and they stated "Maintenance probably has the [log] book and they are working on an issue on the airplane." 

Several minutes later two mechanics arrived and replaced the Captain's Primary Flight Display (PFD). The Mechanic returned several minutes later and asked for his Maintenance paperwork. I replied that it was on the glare shield next to the trim panel that they had removed. The Mechanic took the paperwork and left the panel on the glare shield. Several minutes later he brought the logbook back, handed it to me in the galley and immediately left before I could say anything or review the logbook. I checked all of the pastel pages of the logbook for compliance with the Static Probe MEL, but it was not addressed in the current [log] book, however there were no longer any write-ups indicating the release of the aircraft for service. 

The trim piece for the Captain's instruments was left on the glare shield and a piece of plastic was left obstructing the throttles. I wrote up the new discrepancies and called Maintenance to address them. The same Mechanic responded and I asked him about the Static Probe Heat MEL that addresses two different Air Data Reference (ADRs). The shaded logbook placard had "Honeywell" scratched out and the words "aircraft is not equipped with Litton Inertial Reference System (IRS)" in different writing below the initial placard lettering with the word "not" specifically circled. The differences in IRSs determine the RVSM qualifications of the aircraft per the MEL in icing conditions. 

The Mechanic told me they should have checked them when they put the MEL on the aircraft and told me to call Maintenance Control. I checked the pastel pages of the logbook again to see if I missed the write-up and documents addressing the MEL. I handed the logbook to the Mechanic and he began to clear the write-up on the trim and obstruction [at the throttles]. I called Maintenance Control who stated he did not think the aircraft had Litton IRSs, but suggested I have the Mechanic physically check the IRSs. He stated it should only take five minutes. I then asked the Mechanic to physically check the [type of] IRSs for compliance. He left the aircraft again with the logbook. No one opened the E & E compartments on the plane. 

About ten minutes later the Mechanic showed up with a Maintenance Supervisor who asked me what he needed to do to satisfy my concerns. I stated that I wanted the IRSs "physically verified" that they were not Litton IRSs and explained the RVSM seriousness of the verification. He stated that he personally verified that they were not Litton IRSs. I confirmed with him a second time that he personally physically verified that the IRSs were not Litton's and he stated again that he had. We discussed the placard briefly and I told him if they were verified, than I was OK with the aircraft. I sent dispatch an ACARS that the IRSs were verified not Litton's, so we were good for RVSM on our route. 

Upon arrival in ZZZ1, I requested a Mechanic to "physically check the IRSs" to see if they were Littons. A ZZZ1 Mechanic physically went into the E/E compartment and verified that all three IRSs were in fact Litton IRSs by reading the Manufactures label on the [IRS] boxes. I wrote the discrepancy up in the logbook, and the logbook placard was changed to reflect the installation of Litton IRSs and the added RVSM prohibited placard with ADR-2 "Off." ADR-2 is required to be "Off" in any icing conditions. The failures and actions of these men in ZZZ Maintenance, when I voiced my safety concerns, displayed a total disrespect for my mandated responsibility and blatant reckless disregard for the seriousness of their own responsibilities. Maintenance [in ZZZ] did not physically verify the Manufacturer of the IRSs and were deceptive in their answers and actions.

Callback: 1

Reporter stated their A320s that are equipped with Honeywell IRSs are allowed to fly in RVSM icing with the number 2 pitot static probe heat and the number 2 Air Data Reference (ADR) portion of the Air Data Inertial Reference Unit (ADIRU), deferred inoperative (inop). But A320s with Litton type IRSs are not. He believes the Maintenance Supervisor's behavior was reckless. He has not heard from his Chief Pilot regarding any follow-up company action regarding the Supervisor's comments.

Synopsis

A Line Captain describes his efforts to have Maintenance verify their A320 aircraft was not equipped with Litton type Inertial Reference System (IRS) units in order to satisfy an MEL deferral of the number 2 Air Data Reference (ADR) in RVSM icing conditions. A Maintenance Supervisor verified the IRSs were not Littons. Upon arrival at the next station, a Line Mechanic noted all the IRSs were Littons, not Honeywells.

ACN: 949338 (2484 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Parking Brake
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : CVR
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Electrical Wiring & Connectors
Manufacturer : Airbus
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Avionics
Experience.Maintenance.Technician : 25
ASRS Report Number.Accession Number : 949338
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Object
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Working Deferred Item (DF), found wire to Parking Brake Pressure Switch broken from cannon plug. Placed ladder near right wheel well, repaired wire, reconnected cannon plug, went up to flight deck to accomplish Cockpit Voice Recorder (CVR) Test. Came back down to remove tools and ladder. A320 aircraft was resting on ladder. Removed ladder, found damage to lip of right wheel well after opening that showed cracks in the fairing.

Callback: 1

After a quick trouble shooting check, Reporter found one of the two wires for the CVR's electrical ground circuit shredded and pulled from the wire pin that remained inside the cannon plug at the parking brake pressure switch, in the right wheel well, near the aft bulkhead fuselage body fairing.

Reporter stated he always finds so much hydraulic fluid on and around the cannon plug connector at the parking brake pressure switch that the hydraulic fluid causes the rubber isolator in the cannon plug to swell, distort or become mushy. The rubber isolator holds the connector pins that are swedged to the small electrical wires, one of which was the one he found broken.

Synopsis

A Line Avionics Mechanic reports finding an electrical ground wire for the Cockpit Voice Recorder (CVR) shredded and broken on a cannon plug at the parking brake pressure switch in the right wheel well of an A320 aircraft.

ACN: 949018 (2485 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : TUL.Airport
State Reference : OK
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : TUL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Facility : TUL.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 949018
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

A Company X A320 taxied out to Runway 36R. No Company X aircraft was observed. There was a Company Y jet moving off the ramp. It was determined that the Company Y was in fact the Company X in disguise. This is a very unsafe situation for Ground Control, taxiing, Local Control and Approach/Departure Control regarding traffic calls and/or sequence information. This has been an on going problem with Regional Jets, but I do not recall major Air Carriers being allowed to do this. This really needs to be corrected. We don't care what they call themselves to the public, advertising etc. We don't care who is signing their paychecks. But in the ATC system, a plane calling itself Company X should be painted Company X.

Synopsis

An air carrier using a call sign that was not reflected in the paint scheme on the aircraft caused confusion for a TUL Tower Controller. Potential ATC identification problems are listed by the reporter.

ACN: 948911 (2486 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.AGL.Single Value : 1500

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : BWI

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12800
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6902
ASRS Report Number.Accession Number : 948911
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I, the First Officer, was the non-flying pilot. Our PDC had a departure frequency listed of 128.7. After takeoff from Runway 15R, BWI Tower switched us to Departure on a new frequency of 119.7. I called Departure and noticed that this frequency of 119.7 was a higher altitude frequency with other communications of aircraft descending out of the flight levels. I switched to our original frequency of 128.7 and Potomac Departure responded with our clearance. I immediately informed him that BWI Tower had given us 119.7 vs. 128.7 to which the Controller responded that they were "switching frequencies." The reason for this report is due to a similar problem which I had last month out of a different Washington Metro Area airport. Each problem was associated with "Potomac Departure Control." There is a problem in Potomac Approach/Departure. This is my second report in 20 days with communication/frequency issues here. I know for a fact during this incident we did what we were told to do. I am expressing true concern of the ATC system here in DCA.

Synopsis

Air Carrier departure from BWI voiced concern regarding assigned departure frequencies as related to BWI Tower and PCT Departure Control, alleging multiple events of wrong frequency assignments.

ACN: 948764 (2487 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 3000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Night
Ceiling.Single Value : 3000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Direct
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 420
ASRS Report Number.Accession Number : 948764
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Other / Unknown

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were cleared direct to the outer marker expecting a visual approach. We were at 6,000 FT 17 miles from the airport, so I slowed to 210 KTS. The Captain told me not to slow any further, so I maintained 210 during the subsequent descent to 3,000. Needless to say, I wound up high and fast. On final, we got one GPWS "sink rate" callout, but the Captain said "you're good, keep going" (or something similar). I had thought that I was too experienced to allow this to happen, but the Captain had been overriding my decisions the entire trip, and apparently, I just got tired of it. If I had said, "let's go-around" would he have approved it? Clearly, he didn't want to go-around. So, what's a First Officer to do? When the Captain says maintain speed, at what point can I slow? When he says continue, may I go-around? Old questions; no answers. We should have gone around, but in these circumstances, is the flying pilot really flying the aircraft?

Synopsis

A320 First Officer reports slowing to 210 KTS during a visual approach and being told not to slow any further by the Captain. An unstabilized approach results and is continued to landing.

ACN: 948486 (2488 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ATL

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 948486

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

[We] received CADIT RNAV departure on our PDC. Informed Ground Control that we were assigned and expecting that departure. We carefully reviewed the NOTAMS, ATIS, departure page, 10-7 page, and the 10-3 page that describes the RNAV procedures at ATL. AFTER we were in position on the runway, the Tower cleared us for take off and added: "Fly heading 295 at the marker." We asked him to clarify and he just repeated the same thing. I asked him did he mean one of the markers for 8R. He said "yes, the middle marker." I informed him that we were assigned the CADIT RNAV. His answer was that they were doing triple parallel departures. We turned up our marker beacon audio (there are NO marker beacon lights on the Airbus) and departed. After takeoff I again reviewed all our paperwork and there is NO MENTION ANYWHERE OF THIS MARKER BEACON THING! One note on the 10-3 page says that if triple parallel departures are being conducted to expect a HEADING; NOTHING about a marker. If this is a standard departure, why isn't it in the 10-7 page or somewhere? It should be on the ATIS at least. Why didn't Ground Control warn us that we could expect it? We don't use the marker beacon audio on the Airbus so how do we know if it's going to work? Had we had some warning we could have set something up to identify the point.

Synopsis

An A320 Captain took umbrage at being cleared for the CADIT RNAV SID from Runway 26L at ATL and, when cleared for takeoff, was cleared to fly a 295 heading after the marker.

ACN: 948100 (2489 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 1
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 948100

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The First Officer completed the preflight and did not note any discrepancies with the aircraft. We departed a few minutes early. Forty- six minutes into the flight we received an ECAM HGD G RSVR LO LVL. We completed the appropriate check list. The check list had us shutdown the Green System. At that point the green reservoir showed just above empty. We notified Maintenance Control and Dispatch of the condition of the aircraft and asked them where they would like us to land as the aircraft was fine in flight; however performance of the aircraft was not sufficient to land at our destination. 

While we were awaiting the decision from Dispatch, as to our alternate, I informed the A Flight Attendant that we had a mechanical condition which would require a precautionary landing. I asked her to review her procedures and told her I would get back to her when we had more information. Dispatch and Maintenance suggested ZZZ2 as the alternate. I was against this as they had ninety degree cross winds gusting above twenty knots. With possibly no brakes and steering and loss of some flight controls I asked for a different airport. After going back and forth with Dispatch and Maintenance Control, it was decided that we would make a precautionary landing in ZZZ. 

The First Officer and I reviewed all of the procedures and decided, for performance, in case of a go around a flaps three landing was called for. Also, in reviewing Flight Operations Manual (F.O.M.), we decided that the aircraft situation was "Urgent" and did not meet the definition of an Emergency. Therefore, an emergency was not declared. Approaching the top of descent point I called the A Flight Attendant and once again briefed her that a precautionary landing was being made and told her I would be making the announcement to the passengers in five minutes. I waited five minutes and read the precautionary landing announcement from the F.O.M. to the passengers. 

The ECAM procedure for the aircraft condition allowed for us to try and restore the Green System to lower the gear. We successfully attempted this procedure and lowered the gear. However, due to the small amount of hydraulic fluid remaining it was determined to not be safe to try and taxi to the gate. We landed on the runway and came to a stop. The fire crews inspected the aircraft and found a puddle of fluid under the left engine. The Maintenance crew came to the aircraft they requested and we delivered through the First Officer's window the gear pins. The Maintenance crew secured the gear and we were towed to the gate. It was found that the hydraulic pressure valve had blown an "O" ring and was leaking fluid when under pressure.

Synopsis

After suffering the loss of the Green Hydraulic System an A320 flight crew opted to divert to an airport more suitable for landing with possibly compromised braking and or steering.

ACN: 948057 (2490 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14323
Experience.Flight Crew.Last 90 Days : 265
Experience.Flight Crew.Type : 4234
ASRS Report Number.Accession Number : 948057
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Captain met inbound First Officer on jet bridge and First Officer mentioned the brake temps were a bit strange but it was probably due to the deferral of the BSCU1. Crew boarded aircraft and indeed noticed brake temps were "strange" in that the number 1 and 4 wheels were hot and the 2 and 3 wheels were at least 220 degrees C cooler. This is odd in that the two wheels per truck are usually warm but neither crew member recalled seeing the outside ones hot and the inside ones cool. Winds at the airport were straight down the runway. The temperatures were within parameters outlined in the Flight Manual, but the configuration of the temperatures caused us to enter a maintenance item for further investigation by Maintenance. The mechanic physically felt the brakes and confirmed the brake temps were accurate (hot/cold). The mechanic observed the brakes operating and signed off the item. Checks were done per the deferral item and the door was closed for departure. A discussion in the cockpit still revealed some question as to the brake operation so maintenance was contacted for further info. Maintenance pointed crew toward the AIDS system page where the pressure values for brakes 2 and 3 were blank. Maintenance told us they should be equal across all the brakes. Maintenance contacted Station Maintenance to return for further troubleshooting. After much testing it was determined the best course of action would be to clear the deferred item for dispatch as all parties agreed there was an unidentifiable fault with the brakes. The CIDS prior legs pages ID'd some brake valve faults from previous legs. It is the belief of the crew that the aircraft flew at least one leg with the inner brakes of both main wheels operating at diminished or zero operating capacity. This could have been problematic if faced with a rejected takeoff. Parts could not be secured so the flight was canceled by maintenance. The outbound crew believes the inbound crew should have reported their concerns of the brake system to Maintenance. There is no mention of the fault they observed in the deferral or the MEL. If the outbound crew had arrived at the aircraft after a period of time that had allowed the brakes to cool, this mechanical issue might not have been discovered until it was too late. It is quickly becoming an unwritten SOP to report discrepancies to the "outbound" crew so that they can report them inbound to a mechanic station. This procedure is not allowable by Flight Manual and should be strongly discouraged. The question of "did you write it up" should not have to be asked as it is clearly directed by Air Carrier policy to do so.

Synopsis

An A320's BSCU1 was previously deferred because of strange brake temperature indications but the flight crew's investigation with maintenance determined that brakes 1 and 4 were operating normally while brakes 2 and 3 may have been symmetrically not activated.

ACN: 947963 (2491 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance

Component

Aircraft Component : Parking Brake
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 947963
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 947964
Human Factors : Communication Breakdown
Human Factors : Confusion
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was assigned to move an A320 aircraft from the gate to the hangar. Mechanic Y unplugged the power cord and went in the aircraft and closed the door and started the APU. I was in the tractor [tug]. I got out of the tractor and pulled the [wheel] chocks. I got back in the tractor and was preparing to approach the aircraft. I then saw the aircraft rolling back away from the jetway. I called on the radio for Mechanic Y to set the brakes; he did and the aircraft stopped. There were no injuries or aircraft damage. The brakes on the aircraft were not set upon entry to the aircraft and we received no notification that the brakes were released for any reason.

Narrative: 2

Mechanic X and I were assigned to bring an A320 aircraft from the gate to the hangar. I went into the aircraft and started the APU, then went back and closed the [Main] entry door, then returned to the cockpit. As I sat down, Mechanic X called to set the brakes, as the plane was moving backwards. I stopped the aircraft with the pedals and then set the brakes. There was no damage to the aircraft or jetbridge.

Synopsis

Two line mechanics preparing to move an A320 aircraft from a terminal gate to their hangar, report the aircraft had rolled back from the jetway after they had removed the wheel chocks.

ACN: 947844 (2492 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZZZ
Aircraft Operator : Military
Make Model Name : Globemaster (C-17)
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 947844
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

I had an operational error between Air Carrier X and Aircraft Y. Both aircraft were at FL370 on a slowly converging course. I recognized the confliction early and, using URET, tried to correct the issue using route separation. However, this proved to not be acceptable. I checked URET for the winds in the area and, according to URET, the winds favored a vector to the north for Air Carrier X. I vectored Air Carrier X 30 degrees left to the north thinking that it would take the aircraft out of the wind and slow the aircraft down. This along with the vector should have worked sufficiently. However, after approximately one minute, I checked the progress of the two aircraft and noticed Air Carrier X had gained 45 KTS, not lost the 20 KTS as I was expecting. I vectored Air Carrier X an additional 15 degrees left and asked the Controller east of me to descend Aircraft Y to FL360. I continued to monitor the situation. I felt we had five miles, but the OEDP gave a reading of 4.8 miles. This really is a cheap error for our facility. If there is such a thing as a controlled error, this is it! The problem, I guess, I should not have trusted the winds in URET enough to attend to non-separation task, and monitored the situation more closely until it proved to be true, the winds actually being fairly correct. The pilot of Air Carrier X kind of argued with me on the second vector, which if he would not have it may have made a difference. [Recommend] monitoring the winds more closely or get a wind report from the aircraft prior to the vector. This error is really a controlled error; embarrassing, really.

Synopsis

Enroute Controller described a loss of separation event when permitting crossing traffic to fly within 4.8 miles, the reporter relying on URET wind information for spacing judgement.

ACN: 947419 (2493 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class C : ZZZ

Component

Aircraft Component : Pressurization Outflow Valve
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17230
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 873
ASRS Report Number.Accession Number : 947419
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 947421

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On liftoff, Captain noticed a lot of ear clearing and then a loss of CAB SYS 2 ECAM in amber. One minute later, loss of CAB SYS 1 on ECAM (we were climbing through 8,200 MSL). [We] elected to shallow climb to get into CAB PR (1+2) irregular procedure. [We] did all items in checklist. When switching to MAN cabin mode selector, MAN illuminated + FAULT on the switch. Placed MAN V/S control down and outflow valve moved minimally and closed to the 3/4 closed position and no other movement. Called Dispatch and phone patch to Maintenance. Dispatch and Captain figured FOB (Fuel On Board) landing at destination at 4.9 LBS traveling at 10,000 FT MSL. Maintenance Control asked Captain to use Emergency Authority to pull and reset Y23 circuit breaker for "CTRL MAN." Captain elected not to pull and reset Y23 circuit breaker due to not knowing if that would bring the cabin altitude down too quickly. Captain decided to press on to our destination at 10,000 FT MSL with concurrence of Dispatch. No oxygen masks deployed or oxygen used on board in the cabin. Dispatch was to alert all ATC agencies we were to travel to our destination at 10,000 FT MSL. Captain notified Flight Attendants to monitor passenger for breathing difficulties and to monitor passenger from going near the doors due to being unpressurized and possibly being able to open doors in flight. Captain and Dispatch did not declare emergency with ATC. Reached our destination and landed with 5.3 LBS of FOB and no other incidence. First Officer did a great job flying the aircraft and coordinating with ATC during the irregularity. Captain decided since there was no hull damage, minimal mechanical problems and no passenger abnormalities to continue to our destination.

Narrative: 2

I was the flying pilot and climbing out of 5,000 FT we got an ECAM - CAB PR 2 fault. The Captain handled the ECAM and I kept flying the airplane and handled the radios. After about 45 seconds we got a second ECAM, CAB PR (1+2) Fault. At this time the Captain stated that she was unable to control the cabin altitude with the manual pressurization controller and that we were climbing unpressurized. We were now climbing out of about 7,500 FT and I asked ATC for a slow climb, and to level off at 10,000 FT, which was given. We did a quick check of the cabin alt and it was climbing as we were climbing. We leveled at 10,000 FT and looked at our fuel. We had a discussion; determined that it would be safe and that we had enough fuel to continue on to our destination.

Synopsis

A320 flight crew experiences cabin pressure anomalies during initial climb and levels off at 10,000 FT with the cabin unpressurized and the outflow valve stuck at 3/4 open. The crew elects to continue to destination at 10,000 FT and lands with 5,300 LBS of fuel.

ACN: 947277 (2494 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Altitude.MSL.Single Value : 1500

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Nav In Use : FMS Or FMC
Flight Phase : Climb
Route In Use : Direct
Airspace.Class B : ATL

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : A330
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class B : ATL

Person

Reference : 1
Location Of Person.Facility : ATL.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 947277
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

An A320 was given a LUAW clearance on Runway 27R. When separation was assured with the preceding departure, I scanned the runway and cleared the A320 for takeoff, with instructions to "RNAV TO FUTBL". The A320 responded quickly, yet completely, with "CLEARED TO FUTBL" and his call sign. After clearing him, I resumed my normal operations and LUAW the next aircraft, informing that aircraft I would be crossing down field prior to their departure. As the A320 passed the intersections, I started crossing at Taxiway N5, T, and P with a total of 4 aircraft. Right as I finished crossing the last aircraft, I observed the A320 in a hard right turn. I transmitted "Flight XXX TURN LEFT IMMEDIATELY". The A320 acknowledged and the aircraft rolled left into the turn. I asked the A320 to verify if he was going to "RNAV TO FUTBL" and he replied in the affirmative with his call sign. At this time, I observed an A330 climbing off of Runway 26L. The A320 was still sliding north, but had banked left so I did not call traffic and provided visual separation from the Tower; the two aircraft started to separate with my closest estimate of 2 miles during visual separation. The A320 was about 3 miles off the runway end and I switched him to DR-S. I continued to observe the target on the RACD and saw he had not continued South towards FUTBL, but upon looking out the window, saw he had leveled his wings after switching to departure and was tracking straight out again. I called DR-S to inform them of his non-compliance. Recommendation, stop using RNAV until a fix can be completed for closely spaced parallel runways. After observing the incorrect turn, I should have given the A320 a 240 heading to ensure no issues with flying towards FUTBL, as I later found out that the A320 said he was having FMS problems.

Synopsis

ATL Controller witnessed an air carrier initiate a wrong after departing Runway 27R that was assigned a "RNAV To FUTBL" clearance, with a correct read back, the reporter listing FMS problems as a possible causal factor.

ACN: 947238 (2495 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZME.ARTCC
State Reference : TN
Altitude.MSL.Single Value : 28000

Aircraft

Reference : X
ATC / Advisory.Center : ZME
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZME

Person

Reference : 1
Location Of Person.Facility : ZME.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 947238
Human Factors : Other / Unknown

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

I was working an aircraft that was on an acquired track, climbing from ZME airspace to ZTL airspace and appeared to have handed off to ZTL, but was not the case and the aircraft entered ZTL airspace. The aircraft automation failed from ZME to ZTL with no notification or failed track to let me know that no hand off occurred. ZTL called to ask about the aircraft and said that they had been watching. Recommendation, evaluate the automation between ZME and ZTL.

Synopsis

ZME Controller described a failed automated hand off event between ZME and ZTL.

ACN: 947133 (2496 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ
Altitude.MSL.Single Value : 9000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 947133
Human Factors : Distraction

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

First Officer was flying EAGUL arrival Runway 8 into PHX. We got wake turbulence from aircraft ahead. Correcting for the wake distracted us and we descended below the 9,000 FT restriction. Corrected back to 9,000 FT. Asked ATC if altitude was a problem and he said no.

Synopsis

An A320 Captain reported encountering wake turbulence on a PHX arrival resulting in an altitude deviation.

ACN: 947082 (2497 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Fire Protection System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 3650
ASRS Report Number.Accession Number : 947082
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 947086
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During a break at cruise alt the First Officer noticed that four circuit breakers on the 121VU panel were out. The breakers, all in a row, were Q41,Q42,Q43,Q44. These were the squibs for both engine fire systems. We contacted Maintenance via ACARS and after his research he determined the breakers had not tripped but had probably been left open after maintenance that occurred earlier in the morning. He asked that I use my authority to close the breakers and thus regain fire fighting ability for the engines. We did so. The breaker location is right near the edge of the panel. More or less right under your nose. I suppose several flights flew with these open.

Narrative: 2

I came back from using the lavatory. I happened to notice that four circuit breakers on the back panel were popped out. I was sure that I had checked the breakers before we left and that they all had been in. I guess it is possible that I just missed these.

Synopsis

An A320 crew found four fire extinguishing squib circuit breakers open in flight. The crew determined Maintenance had opened them earlier in the day and so using the Captain's emergency authority reset the breakers.

ACN: 946933 (2498 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : EWR.Airport
State Reference : NJ
Altitude.MSL.Single Value : 3000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : N90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Route In Use.SID : NEWARK 8
Airspace.Class B : EWR

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 946933
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 946937

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

Before departure, the Captain gave a full briefing to include the departure SID. The flight [plan] showed, "KEWR DCT WHITE". The PDC filed route showed "KEWR WHITE". The PDC remarks showed "Newark Eight Departure".

We took off from Runway 22R and climbed out as called for by the SID expecting radar vectors for WHITE. Upon switching to Departure Control, we were cleared direct ELVAE. I read back the clearance, found the point on the departure procedure, typed it in the FMS and proceeded direct to the point. I then asked what the Controller would like us to do after ELVAE. He stated that it was on the SID plate and we needed to follow the guidance to COL then to WHITE.

After much confusion, distraction (during a critical phase of flight) and numerous radio calls, we eventually flew to COL then onto WHITE. We also followed up with a phone call to the approach facility. The SID chart is confusing and the clearances are misleading. If we are cleared by ATC to ELVAE, they should follow that up with "and fly the published routing to WHITE". Everything we had indicated radar vectors to WHITE. The verbiage on the SID is unsafe and needs to be made crystal clear. The SID should also utilize transitions as they have in DEN and other airports.

Synopsis

When cleared KEWR WHITE after departing EWR, an A320 flight crew was confused by the "ROUTING" format of the NEWARK 8 DEP chart. Attempts to seek clarification from ATC were rebuffed.

ACN: 946702 (2499 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Make Model Name : Small Transport, Low Wing, 2 Turbojet Eng
Crew Size.Number Of Crew : 2
Flight Phase : Landing

Person : 1

Reference : 1
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
ASRS Report Number.Accession Number : 946702
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : ZZZ.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 946710

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were in the middle of a moderate arrival and departure push. The wind situation made this a more complex operation than normal. The wind was variable 190 to 230. Wind speed was as low as 10 to as high as 17 KTS. Aircraft started to refuse take off clearance due to the tailwind component. The Local Controller was keeping the aircraft advised of the current wind conditions. I discussed the pilots concerns with both the OSIC and the TMU FLM. After they talked it over I was advised we would remain on our current flow. The TMU FLM requested the call signs of aircraft that were refusing to depart due to the current wind. Local Controller was using his personal "memory jogger" of turning the departing aircraft flight progress strip over until he was able to clear the 9L aircraft for takeoff. I completed a call to TMU when I noticed an A320 on takeoff roll Runway 9R with a bizjet rolling out on Runway 17. I asked the Local Controller if we were OK with the A320 and the bizjet, he replied "No". I immediately had the Local Controller relived and advised the OSIC of the situation. Recommendation, less land line conversations between the Tower FLM and RADAR room/Traffic Management.

Narrative: 2

While working multiple runways, there was a tailwind that varied from 230 to 190 at 12 to 16 KTS. Several aircraft informed me that they would be unable to depart. An A320 was holding in position on Runway 9L for departure. During the time of going back and forth with the pilots about who could go and who couldn't, a bizjet checked in on a short approach for landing on Runway 17. I cleared the bizjet to land. Using my memory jogger, I turned the flight progress strip of the A320 upside down. While waiting for the bizjet to land, I cleared several aircraft to cross Runway 9L into the ramp. The ramp was very congested, so I had to get several gate assignments from the aircraft that needed to cross the runway. When the aircraft crossing the runway cleared, I turned the flight progress strip of the A320 right side up. I checked with the A320 to see if they could still depart with the tailwind. They replied that they could, I then cleared the A320 for departure forgetting about the bizjet landing on Runway 17. When I realized my mistake, I felt it was safer to allow the A320 to depart and the bizjet to land knowing that the bizjet would be able to hold short of Runway 9L. Recommendation, [I need the] use of different memory joggers.

Synopsis

Tower Supervisor and Controller described a conflict event between a departure and arrival when the Local Controller failed to remember the landing aircraft, memory jogger techniques and coordination requirements listed as casual factors.

ACN: 946597 (2500 of 2908)

Time / Day

Date : 201105
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Ground : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 946597
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Landed uneventfully [Runway] 28R, crossed 28L, and cleared at [Taxiway] D. Ground instructed us to taxi "B-F-A to the gate." We taxied eastward on B and in the depicted hotspot east of D we transitioned to A. Ground informed us that we had just turned on E, not on F as instructed. We obtained a new clearance and taxied via A uneventfully to the gate.

Looking at the airport diagram, taxiing east on B, it is confusing to interpret the instruction "taxi B-F-A to the gate." In particular, looking at the close-up of the associated hotspot, the portion of "F" where we were instructed to turn is not labeled "F"; in fact it is not labeled at all. The portion of "F" where we should have turned is as follows: looking westward from where "F" intersects Runway 1L, F turns directly south, crosses B, and joins A in approximately the same location that F1 joins A. This is the portion of F we should have turned on. The only indication of this is signage on the ground. In other words, it's impossible to determine what exactly constitutes "Taxiway F" beyond the portion immediately west of 1L by looking at the chart. If anything, my interpretation of the HS1 depiction would be that "F" continues straight west from where it intersects 1L, crosses B, and terminates in the same vicinity of where A, E, and J are joined. To conclude, the clearance "taxi B-F-A to the gate" is confusing and I honestly can't tell you by looking at the 10-9 chart exactly where ATC expects us to taxi. The chart needs to be updated immediately in my opinion to reflect clearly and accurately all of the taxiways in the hotspot southwest of the Runway 1L/28L intersection. In particular, Taxiway F needs to be labeled clearly and in its entirety.

Synopsis

A320 Captain reports deviation from the taxi clearance due to confusion over the location of Taxiway F as depicted inside HS1 at SFO.