ACN: 946217 (2501 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25800
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1600
ASRS Report Number.Accession Number : 946217

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 946228

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During landing roll out lost green hydraulic system. Stopped aircraft on runway and accomplished ECAM for green hydraulic system low pressure. Green quantity was completely lost. Advised Tower and they had fire rescue check aircraft. They found fluid was leaking from the belly area. Notified operations and they sent a tug to tow aircraft to gate. [We] deplaned passengers normally at gate. Notified Maintenance and completed maintenance log item.

Narrative: 2

After landing we had a Green System Hydraulic malfunction resulting in loss of green system hydraulic fluid. After transferring controls upon slowing to taxi speed Captain was not able to taxi clear of the runway due to loss of nose wheel steering (on green system). We were towed into the gate the airport emergency crew noted a large puddle of hydraulic fluid under aircraft and on runway.

Synopsis

A320 flight crew reports loss of Green Hydraulic system pressure during landing roll. After stopping, the aircraft is towed to the gate.

ACN: 945937 (2502 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Emergency Exit
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 945937
Human Factors : Confusion
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

[I was] working an A320 aircraft, hurrying up to get a cabin check completed to get aircraft to the gate. During completion of paperwork, the step called to "verify" on ECAM that overwing emergency doors [hatches] were armed, [with] indication showing on ECAM. I glanced at the step on the card, failing to read it completely. [I] visually checked that the doors were installed and closed; not checking that the word "SLIDE" was showing on the ECAM. At the gate, the flight crew on their checks noticed the slides were not armed, visual indication on ECAM of overwing door slides not armed, and [they] alerted Maintenance. The plungers were [then] armed.

During the cabin check, the overwing emergency exit doors were not armed. Per log page XXX [report], [Maintenance] found plungers extended [not armed] at both overwing exits. Pushed plungers back to 'ARMED' position. Operational check [was] normal. Verified overwing slide bottles were not pinned in aft cargo bin as well. I need to take the time to completely read the task card steps, even when trying to make an estimated time of release (ETR).

Synopsis

Mechanic reports that in his rushing to complete a cabin check in order to get an A320 aircraft to the gate, he failed to verify that both overwing exit doors (hatches) were indicating "SLIDE" on the cockpit ECAM display. The exit door slides were not "armed" as required per his jobcard.

ACN: 945629 (2503 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZJX.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 34900

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZJX

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : B737 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZJX

Person

Reference : 1
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 945629
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

A B737 was handed off to me at FL340 requesting FL400; there were two northbound tracks at FL350. After the B737 passed the first aircraft at FL350, I did a quick traffic search and climbed the B737 up to FL380. After reviewing the incident on Falcon, it appears that the target jumped slightly east and possibly the track had a "bad hit." The vector lines looked good when I initially climbed the B737. Conflict Alert when off when the B737 was abeam a A320 at that point I put a J-Ring up and the target was on the edge of the J-Ring. I didn't do a traffic call at that point because the aircraft were already passed and assumed initially the separation was exactly 5 miles based on the J-Ring. If there is a way to prevent the track jumping due to the RADAR sort box that would be helpful. In the future, I'll be more cautious especially in that particular area.

Synopsis

ZJX Controller described a conflict event when climbing an aircraft through the altitude of another. A track jump due to a RADAR sort box anomaly may have contributed to the event.

ACN: 944969 (2504 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Fan
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5600
ASRS Report Number.Accession Number : 944969

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 450
ASRS Report Number.Accession Number : 944968
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

Enroute at FL350, experienced noticeable aircraft vibration with associated flashing N1 indicator, reading of 6.1, on engine #2. Other parameters [were] normal. [We] referred to Flight Manual, High Engine Vibration irregular. Requested lower altitude and turned on engine anti-ice thinking ice build up may have caused condition. Able to stabilize indication/vibration less than 5.0 at FL310, .75 MACH. Captain communicated with Dispatch, Maintenance, flight attendants. [We] decided to declare emergency with Center intending to proceed direct to destination. Vibration indication appeared normal by approach phase below 10,000 FT. Normal approach, landing and rollout.

Narrative: 2

During cruise at FL350, engine 2 displayed 6.1 on the N1 vibration scale, and a pronounced aircraft vibration was detected through the flight control side-stick, tray table, and the cockpit floor. Engine 2 thrust was reduced to maintain a vibration level at or below 5.0 units. The high vibration irregular checklist was consulted for guidance. Decent to FL330 was requested to allow for an idle thrust application of engine heat. After determining the irregular procedure was not an adequate remedy, additional guidance was sought from Dispatch and Maintenance. The Maintenance Engine Controller downloaded our aircraft's engine parameter data, but no evidence of our vibration appeared from this effort. A diversion was considered prior to and during consultation with Dispatch and Maintenance, but ultimately deferred until our collaboration was complete. At the conclusion of our "out-reach", however, we advised Dispatch that (due to the degradation) we planned to declare an emergency with ATC and request priority handling, since we were nearing the top of decent and this was our closest option in point of time.

Synopsis

A320 flight crew experienced high N1 vibration at FL350. Altering thrust and altitude along with engine anti-ice have little effect. An emergency was declared during descent but the vibration completely disappeared below 10,000 FT.

ACN: 944856 (2505 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 944856

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Landed normally after uneventful flight. During taxi-in, we held behind an aircraft while the taxiway was blocked by another aircraft pushing back. We were stopped, with the parking brake set, both engines running. We then received a Class I ECAM "HYDR G RES LO LVL," accompanied by a rapid drop in G hydraulic fluid level distinctly visible on the ECAM HYD page. We promptly accomplished ECAM actions and turned off the PTU and G hydraulic pump. We then informed Ground Control (ATC) that we were without steering and had to be towed in. ATC asked if we could move straight ahead to unblock the intersection (our location at the time), and we moved forward a few feet. I was not comfortable with moving the aircraft, however, and stopped. We were towed in from that point. Airport personnel responding to the potential spill informed us that there was a considerable amount of hydraulic fluid spilled on the taxiway. They later gathered information from both pilots at the gate. Tow-in was accomplished quickly and efficiently by ground personnel. Their response was excellent. Passengers were simply told (by me) that we would be towed to the gate from that point and reminded to remain seated until the sign was off. All I can say is that timing is everything. A few minutes earlier and we would have been in the air. Apart from never, this is just about the best time to lose a major system, on the ground, stopped, and close to our intended gate!

Synopsis

A320 crew lost green hydraulic system fluid while holding for a gate with the parking brake set. The aircraft was towed to the gate due to loss of nose wheel steering.

ACN: 944332 (2506 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Weather Elements / Visibility : Thunderstorm
Work Environment Factor : Poor Lighting
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Elevator
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Static Wick
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Technician : 2
ASRS Report Number.Accession Number : 944332
Human Factors : Confusion
Human Factors : Time Pressure
Human Factors : Troubleshooting
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

An A320 aircraft arrived with a "possible lightning strike" report from the flight crew. The Captain was met by me on arrival and I was briefed by him upon completion of his post flight inspection. In his brief we discussed the "possibility of a lightning strike" on the lower fuselage aft of the cockpit during flight. We discussed that on his walk-around he did not see any damage in the suspected area or on any area below wing. After our brief I went to the flight deck to obtain the logbook, preceded back to the office for the inspection TASK 05-51-18, (inspection after lightning strike on condition), suited-up in preparation for the heavy rain that was falling and grabbed my flashlight for the approaching nightfall. 

Please note that on the same day, our ZZZ airport and the surrounding area experienced a major weather event which involved heavy precipitation fall, severe lightning and associated turbulence which lasted in the area for approximately 36-hours.

My inspection where I began with the step by step set up of the aircraft for the visual below and above wing inspections. Upon completion of the aircraft set up, I began a Detailed Inspection that started at the fuselage radome exterior surface and lightning diverters. I went through the task step by step for the inspection until I was complete. I returned the aircraft to its original configuration per the task and completed the inspection with the logbook and maintenance computer entries. 

The strike hole was discovered three days later, and was corrected with an Engineering Order and was released for service. I was notified three days later and resulted is this form being submitted. Gaps in days due to days off of Lead Mechanic and Aviation Maintenance Technician. Lead Mechanic was also not able to notify me on the day of discovery. The only AMT action that could be mentioned that might have avoided this event was to ground the A320 aircraft until the weather environment was suitable for a safe inspection.

Callback: 1

Reporter stated a passenger had told a Flight Attendant that he thought the tail area had been hit by lightning on approach; the Flight Attendant informed the pilot. The rain was very heavy and there was lightning at the time he was performing the detailed inspection of the aircraft. He realizes now he probably should not have been working from the hi-lift platform truck given all the lightning going on, but Maintenance Control and Customer Service were pushing hard to turn the aircraft around for departure. About an hour into his detailed inspection the outbound flight was canceled due to the severe weather. The inspection took about five hours to complete. He did not find any lightning strike damage, so he signed-off the inspection tasks under the Air Transport Association ATA Chapter 05-51-18 for Unscheduled Inspections.

Reporter stated the A320 did fly in and out of ZZZ for approximately three days, even with the bad weather, before a First Officer noticed a static wick was missing from the outboard trailing edge of the right elevator. Prior to anyone noticing the missing static wick, multiple walk arounds by pilots and mechanics had been accomplished and no one noticed any discrepancies with the elevator. He feels he did a very thorough inspection of the elevator, despite the weather conditions, and believes the static wick could easily separated from the elevator after his inspection. 

Reporter stated a temporary repair was issued by their Engineering Department, item deferred and aircraft released for service. The missing static wick was replaced during overnight Maintenance. His company has since issued an alert memo to their flight crews and maintenance groups to give additional attention to certain areas of the aircraft during walk arounds.

Synopsis

A Line Mechanic performed a detailed inspection of an A320 fuselage and tail section during heavy rain and lightning for a possible lightning strike that was reported by a passenger.

ACN: 944224 (2507 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US
Altitude.MSL.Single Value : 2000

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach

Component : 1

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 944224

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On our climb out we got an ECAM indicating auto brakes inoperative. I informed Maintenance via ACARS, computed the required landing distance and continued to our destination. On approach about 7 miles on final the First Officer called for flaps 3. We got an ECAM "F/CTL Flaps Locked" message. With flaps at 2, the gear down and stabilized the First Officer continued flying while I checked the QRH which was already opened from my earlier use. I consulted the chart for flaps/slats performance and established from the chart that the runway was more than twice as long as required and that the safest thing to do was to land. Our fuel was about 6.0 at the time. I finished the ECAM action and we landed easily making the normal turnoff intersection. We taxied to the gate, contacting Maintenance on the way in. I made the proper write up in the logbook. Maintenance said the airplane had just come out of C check. My First Officer said everything looked normal on his preflight. When we took off the temperature was 3c and for our landing it was 28c.

Synopsis

Following receipt of ECAM Autobrake and Flap Control warnings, the Flight Crew of an A320 performed all required QRH requirements and landed without incident.

ACN: 943390 (2508 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 17000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use.SID : LOOP 6
Airspace.Class E : ZLA

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLA
Make Model Name : Bonanza 36
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Flight Phase : Cruise
Airspace.Class E : ZLA

Person

Reference : 1
Location Of Person.Facility : ZLA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 943390
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 was climbing on the Loop6 departure off LAX to FL230 and a BE36 was southbound at VFR/175 direct DANAH Intersection direct CRQ. I realized the aircraft were going to get closer than I originally thought so I stopped the A320 at 17,000 FT. I called traffic to both aircraft. The BE36 said he would start his VFR descent soon and I asked that he not descend until he had the A320 in sight; the BE36 "rogered" me. The A320 said he was responding to an RA but I did not see the A320 descend or turn or anything. The A320 then said the BE36 was only 300 FT above him and he wasn't sure what that was all about. I told the A320 my RADAR showed the BE36 500 FT above. The A320 told me they were going to file a report on the incident and I "rogered" his transmission. I told the BE36 the traffic was no factor and they could start their VFR descent. The A320 asked if I had a new clearance for them so I climbed them to FL230. Recommendation, I would use 1,500 FT of separation between VFR and IFR aircraft. However in this case the A320 was already going to go through 16,000 FT. I could have descended the A320 back down to 16,000 FT. I could have turned both aircraft as well.

Synopsis

ZLA Controller described a TCAS RA event between a VFR overflight and a LAX Loop departure, the reporter noting more than minimal IFR/VFR separation may have prevented the occurrence.

ACN: 943262 (2509 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Engine Pressure Ratio Indicat
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 943262
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 943465
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Dispatch
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

I arrived at the gate approximately 45 minutes prior to departure. There was a mechanic in the jetway. He asked if I called Maintenance. I said no. He told me that somebody called him about ENG #1 EPR probe. He told me Maintenance had worked on the EPR probe and there was some excess material on the probe. He told me the aircraft could operate with the EPR probe in this condition for 14 days. He also told me that the probe had already been written up. He also informed me that the EPR probe was on maintenance watch. I meet the First Officer in the cockpit. I asked him if he called Maintenance. He said he did call because there was some kind of material around the EPR probe. I checked the log book and discovered that engine #1 EPR probe had been written up twice. The write ups were both the same: On climb out EPR #1 fault. Departed [and] at about FL 190 received ECAM message: Engine #1 EPR fault. We completed the ECAM, the QRH and then consulted ECAM Non-Normal Supplemental Manual. The manual recommended we try to recover EPR mode by switching off both ENG N1 Mode PB. The EPR recovery failed. I sent Dispatcher/Maintenance Control a message about the EPR fault. Dispatch informed me that our destination was VFR and to proceed. I agreed and continued. The following contains some of the ACAR messages between Dispatch and me. Dispatch: This is a chronic problem. They recommended a return to departure airport. 

I didn't print my messages to Dispatch but the following is close to what I actually sent. Captain: Are you sure you want me to return to departure airport. I'm 20,000 LBS overweight. Dispatch: Technicians and parts to destination please return to departure airport at this time. Please advise landing weight. Captain: I need to burn down to 142.2 I now weight 160.0K. Dispatch: We are doing this under strong recommendation from Maintenance Control. Captain: In an emergency I will land overweight but this is a convenience not an emergency. Dispatch: Landings can be accomplished safely if as a crew you think it is not prudent in the area of safety to continue holding in your current ATC environment. etc. I personally do not see a need to declare an emergency. Dispatch: Maintenance advises no problem with overweight landing as long as you record rate of descent an actual weight. Captain: I will do an overweight landing. Dispatch: Maintenance is ok with overweight landing; your Dispatcher in full agreement with your decision to do and overweight landing. I had the First Officer fly the approach and landing. I informed the Flight Attendants and passengers that we were returning. We landed at 151,500 LBS at 200 FPM on the longest runway. Touchdown was normal with normal brake temperatures once we parked the aircraft.

I agreed to land overweight but I had a gut feeling that this was the wrong decision. I decided to try and burn as much fuel as possible enroute. We descended to 12,000 FT, extended the speed brake and started the APU. I then consulted the FOM and the QRH to see if I could find some information that would support an overweight landing. There is no information in the FOM under Overweight landing that supports or denies such an event. There is one note about overweight landing at a non-maintenance station. The note indicates that a Captain with Maintenance Control's help can sign off an overweight landing and return the aircraft to service. I felt this was still a weak argument for an overweight landing. I then checked the QRH and found nothing to support or deny the landing. In the end I landed overweight. I let Dispatch and Maintenance talk me into a landing that should have never happened. 

I knew I could land and stop the aircraft on a 12,000 FT runway. I also knew I would land with less than a 200 FPM sink rate. I also knew the landing would be safe but a limitation is a limitation is a limitation.

Narrative: 2

The crew then advises they would be overweight for landing. Thus began the discussion between crew and myself on the issue of an overweight landing. They held for some time and our landing weight was 150,000 LBS. A review of our ACARS history will clearly state that upon discussion with Maintenance and with the crew I did not see a safety issue with an overweight landing. Any reference to crew and passenger convenience and petrol is secondary on my list of concerns. The airport was VFR with no gusty winds, good runway length and runway conditions. When asked whether an overweight landing could be accomplished - my reply was yes and that I saw no problem with it other than an overweight landing inspection being required. 

I would like to see absolute and clear guidance in both the flight and dispatch manuals for the decision making process regarding overweight landings. If there is currently a company bulletin or company guidance in existence then I clearly need recurrent training regarding this. Past practices with our airline are not in sync on this issue. As I stated to our Chief Pilot in an e-mail; "for future action on my part as a Dispatcher - in no way will I be in agreement with a flight crew wanting to perform an overweight landing unless the crew deems it an operational necessity to conduct a safe operation regarding a particular flight." my thanks to the crew for completing a safe operation during an overweight landing.

Synopsis

An A320 returned to their departure airport for an overweight landing when a chronic EPR Probe ECAM warning recurred during the climb.

ACN: 942602 (2510 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 5900
ASRS Report Number.Accession Number : 942602

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 1050
ASRS Report Number.Accession Number : 942835

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Leaving holding fix on radar vectors to join [the] arrival, First Officer saw crack develop in his forward windshield. The crack was not of the usual variety in that it did not spider web. This caused concern regarding structural integrity. We were in descent from FL250 to FL210 at the time of the incident. I advised ATC that we would be slowing and needing lower altitude due to the crack we had in the windshield. I also sent a quick ACARS message to Dispatch. The Dispatcher declared us an emergency aircraft and notified ATC accordingly.

[Flight Management] guidance was to reduce differential pressure to 5 PSI or below. When I pulled up pressure page we were at 5.8 decreasing, so we attained the needed differential pressure quickly. I briefed the Purser on our situation stressing we were not in immediate danger and for the crew to prepare for landing with touch down in approximately 20 minutes. I then briefed our passengers on the situation and that they may notice the fire trucks following us after landing. We continued with SOP descent approach and landing. After securing aircraft at gate we made Maintenance write up and were then met by Flight Management. We went to conference room and conferred with Flight Operations Duty Manager and ZZZ Flight Management. We decided that it would be best if we discontinued our ID and return home.

Narrative: 2

I had my sun visor in front of me and moved it to the side. Upon moving it I noticed a crack in the windshield that ran from the top right hand corner of the windshield to about 3/4 of the way across and about 1/3 of the way down the windshield. From there it spider [webbed] out into two directions for about 3 inches and the other about 7 inches.

Synopsis

An A320 flight crew suffered a cracked windshield during arrival sequencing.

ACN: 942481 (2511 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Fuel Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 942481
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During flight [we] had "CENTER TANK 1 LO PRESS" on ECAM. [We] accomplished ECAM procedure. When center tank [was] no longer feeding, [I] was somewhat surprised to see on fuel page a very large imbalance. The fact that right tank was full suggested to us both engines were being fed from left tank and right tank was not feeding right engine. Even though cross-feed was open per ECAM and we were in process of closing, we concluded left pumps could not completely override right pumps, therefore a feed issue. Because of our uncertainty as to how much if any fuel was available from right tank and our flight path was approaching terrain and not so many good diversion airports, we felt the safest course was to divert to an airport less than 100 miles away. Had the right tank quantity burned down even a small amount, I would have assessed an imbalance vs. a feed problem. The fact the right tank was full along with past aircraft history of fuel problems led us to conclude we had a fuel feed problem, plus the uncertainty of our total fuel quantity, influenced our decision to divert. Normal landing, conferred with Maintenance, and were away in an hour with new airplane.

Synopsis

A320 Captain experiences center tank fuel pump failure in cruise and opens crossfeed valve in compliance with ECAM. After center tank fuel was depleted a large fuel imbalance between the left and right wing tanks was discovered. Due to aircraft maintenance history and confusion over actual fuel available, the crew elected to divert.

ACN: 942443 (2512 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZDC

Person

Reference : 1
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 942443
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Air Carrier X came over from ZCC with data block saying he was at FL380 when he was suppose to be at FL360. I questioned the Controller at ZCC and he/she confirmed aircraft was at FL360. Air Carrier X checked on level at FL360 with his Data Block still showing FL380. I put him in at a reported altitude of FL360. When I did this another aircraft flashed Air Carrier ZZZ that was right on top of him showing FL380. Now during this time an aircraft at FL360 began to flash and he was traffic with the Air Carrier X. I asked the Air Carrier X if he observed an aircraft on TCAS converging with him and he said yes it was at FL340. This really set off alarms because I was showing them both at FL360. At around this time, I became aware of an intruder at FL360 already well into my airspace. I asked are there any aircraft on my frequency squawking XXXX the Air Carrier X said yes. That is when it finally became clear what was happening. The Air Carrier X at FL360 was sharing codes with Air Carrier Z at FL380 and the actual track for Air Carrier X was actually 20 miles ahead of his data block. This created much confusion because of the data block not associated with the actual track this put other aircraft in the vicinity of the actual track in danger. Recommendation, something needs to be done through automation to prevent a similar situation. This could have been tragic if I had descended the aircraft for his supposed traffic into another aircraft not realizing he wasn't where he indicated.

Synopsis

ZDC Controller described a potential conflict event when two enroute aircraft were sharing the same transponder code resulting in incorrect altitude and associated target identification information.

ACN: 942130 (2513 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : APU Fuel System
Manufacturer : Auxiliary Power International Corporation
Aircraft Reference : X

Component : 2

Aircraft Component : Engine Oil Seals
Manufacturer : Auxiliary Power International Corporation
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : APU Controls
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 942130
Human Factors : Situational Awareness
Human Factors : Training / Qualification
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Maintenance
Were Passengers Involved In Event : N
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Troubleshot auto-shutdown on APU to replace Fuel Control Unit (FCU). Installed FCU, but had trouble engaging and alignment, which had to pull out [FCU] a few times. Oil ring [was] damaged, causing oil leak. No known factors.

Synopsis

A Line Mechanic reports damaging an oil ring while changing an APU FCU on an A320 aircraft after trouble shooting an auto-shutdown of the APU.

ACN: 941448 (2514 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Rudder Control System
Manufacturer : Boeing
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic Main System
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Rudder
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 941448
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Logbook Entry
Primary Problem : Company Policy

Narrative: 1

In our morning scan of inbound work, it was noticed that an A320 aircraft inbound to ZZZ2 from ZZZ1, after a flight leg on [same day] from ZZZ, where a log gripe of rudder flutter that went away when right-rudder was applied, was being deferred to us in ZZZ2 under the reference of Airbus Maintenance Tip 53-XXX. That particular Maintenance Tip appears to be merely an information-gathering tool, rather than a section designed to address a problem that is actually known, as was the case here as enumerated by the flight crew of the A320.

Callback: 1

Reporter stated when the A320 arrived at his station (ZZZ2), one of their Maintenance Supervisor's signed-off the previous flight's rudder vibration deferral because the inbound flight to ZZZ2 did not have a vibration problem and Maintenance could not find any loose wing to fuselage body fairings that were loose. Another Mechanic, himself (reporter) and a different Maintenance Supervisor tried to get Maintenance Control to reconsider and not release the aircraft because no one was doing a rudder actuator end play check. Maintenance Control was not very concerned, aircraft was released.

Reporter stated the aircraft flew two more weeks and had two more separate rudder vibration incidences in-flight. But the sign-offs were still being done using the Airbus Maintenance Tip Chapter 53. Still, no one was checking the rudder. Finally, a Lead Mechanic at a downline station took the A320 out of service and had his mechanics and an Inspector performed a rudder actuator rod end play check. All three rudder actuators were found worn beyond limits for continued service. All three actuators were replaced with no subsequent vibration reports.

Synopsis

A Line Mechanic reports an A320 aircraft with a Logbook gripe of in-flight rudder flutter that went away when right-rudder was applied. Mechanic notes the reference document and sign-offs did not address checking the rudder itself. After two more flutter incidences, a Lead Mechanic removes aircraft from service and finds all three rudder actuators were out of limits.

ACN: 941378 (2515 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 941378
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

During the preflight the green hydraulic system quantity was above maximum refill. Maintenance Control was contacted through Dispatch and, in turn, called out Contract Maintenance. Contract Maintenance understood the write up and was in agreement with flight crew as to the necessary maintenance action to take. Maintenance Control, however, asked the Captain to change the discrepancy write-up to "entered in error." The Captain preferred not to change the original write-up as, in his opinion, it would have been inaccurate.

While the right main landing gear door was still open, the Contract Mechanic, standing in the forward galley, was on the phone with Maintenance Control and asked the Captain to pressurize the hydraulics. The Captain preferred not to comply as it was unsafe without clearing the area first. The Contract Mechanic then pressurized the hydraulic system without clearing the area. The flight crew remained clear of the flight deck while maintenance was being performed.

Maintenance and Contract Maintenance personnel did not follow SOP, they must be retrained to not ask flight crew to change maintenance discrepancy write-ups to "entered in error" when, in fact, a discrepancy exists and, also, to require contract mechanics to perform their duties with strict adherence to established safety protocols.

Synopsis

When an A320 flight crew wrote up the green system hydraulic quantity as above the max refill level during their walkaround they were asked by Maintenance Control to modify the report to "entered in error." The flight crew refused to do so. In addition, a Contract Mechanic was allowed to circumvent established safety practices regarding operation of the hydraulic system without an external monitor to insure no injury resulted when doing so.

ACN: 941364 (2516 of 2908)

Time / Day

Date : 201104
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Galley Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 941364

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon leveling off at FL340, the aft Flight Attendant called the cockpit to say he smelled wires burning in the aft galley. The Captain immediately turned of the galley/cab switch in the cockpit. He told the Flight Attendant to call back in one minute if he still smelled wires burning. After one minute the Flight Attendant called and said he still smelled the wire burning smell. He also said the Circuit Breaker (CB) panel in the aft galley was hot. At this time the Captain told me to ACARS the company and tell them we were diverting. The Captain declared an emergency with center. Flight attendants were told to prep the cabin but to expect a normal landing. Normal approach and landing were made. Once on the ground Maintenance was called to inspect the smell. Upon removing one of the aft coffee makers, It was evident it was the source of the smell.

Synopsis

An A320 flight crew declared and emergency and diverted to the nearest suitable airport following an advisory of burning electrical odors in the aft galley from a Flight Attendant.

ACN: 941302 (2517 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 14500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Cargo Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 941302

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through 14,500 FT we got an ECAM forward cargo door warning light and at the same time noticed an increase in the cabin rate of climb. We called Center and requested a descent to 9,000 FT while we completed the ECAM for the problem. When we leveled at 9,000 we had enough differential pressure from the bleed system that the cabin remained below 7,000 FT. At that point we sent a message to operations that we were going to return and have Maintenance look at our problem. We also advised the flight attendants. We sent a 2nd message to Operations advising them that we would be landing overweight. We went through the QRH and FOM to cover all events of the landing. The First Officer did a great job in helping with this.

After landing we taxied to the gate where Maintenance advised us the forward cargo door latch mechanism was broken. He said a new latch was coming over from the hanger and that it would take about an hour and 1/2 to install. We made all log book entries and I called operations for a new release. All operations normal after latch mechanic fixed.

Synopsis

An A320 flight crew returned to their departure airport when the latch mechanism on the forward cargo door failed and the cabin altitude started to climb.

ACN: 941123 (2518 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.MSL.Single Value : 6000

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : B767 Undifferentiated or Other Model
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : SEAVU 2
Airspace.Class B : LAX

Aircraft : 3

Reference : Z
ATC / Advisory.TRACON : SCT
Make Model Name : PA-23-250 Aztec
Operating Under FAR Part : Part 91
Flight Plan : VFR
Mission : Photo Shoot / Video
Airspace.Class B : LAX

Person : 1

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 941123
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 941832

Person : 3

Reference : 3
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 941847

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Had just taken the position with a trainee. Mitigating factors were a VFR PA27 on a picture taking mission westbound on the downwind at 5,500 FT and Air Carrier X was a foreign aircraft with limited English. Air Carrier X, an A320 on base leg heading 330 at 6,000 FT [was] to stay above the PA27 [and] to precede Air Carrier Y, a B767, straight in. [The] trainee turned Air Carrier X to 300 heading. Air Carrier X was 3 miles ahead of Air Carrier Y who was descending out of 6,500 FT. It didn't appear Air Carrier X turned as instructed and couldn't be descended due to the PA27 in the way. I instructed Air Carrier Y to climb to 7,000 FT for traffic and had Air Carrier Y maintain visual separation from Air Carrier X. Air Carrier Y responded, but advised they were descending for an RA. I subsequently turned Air Carrier X left to downwind for re-sequence. We routinely have problems with this airline not responding in a timely manner. When we first assumed responsibility for the position, I new it was going to be difficult to make this sequence work, mainly due to Air Carrier X not being able to descend with VFR traffic in the way. I tried to let the trainee work it out, but waited too long to take over myself. Recommendation, VFR picture missions should not be approved on downwind during known times of aircraft inbound from SLI. These missions are routinely approved by someone who obviously isn't familiar with our traffic. They should be approved by the FLM after the FLM has checked traffic for that day.

Narrative: 2

Socal Approach cleared us for the SEAVU Two Arrival and ILS 25L approach to KLAX with CAVOK weather. We were on the localizer approaching GAATE at 6,500 FT when Approach Control informed us of traffic. A few seconds later, we received a TCAS Traffic Advisory quickly followed by a descend Resolution Advisory. We saw the traffic slightly above our altitude at our 10 o'clock position and approximately two miles. The First Officer did an outstanding job and immediately notified ATC that we were descending due to an RA. The traffic passed 100 FT above and 1.69 miles in front of us. ATC gave us a turn to 220 degrees. There was an unnecessary distraction in the middle of the RA maneuver while ATC and our conflict aircraft discussed a missed turn. ATC then instructed us to climb, but we continued to follow the commanded descend RA. After another vector we intercepted the 25L ILS and landed uneventfully. After parking, I phoned Socal Approach and was informed of the following: Our conflict aircraft was an Airbus on an LAX arrival from Seal Beach. Our Approach Controller was a trainee who planned on putting the Airbus ahead of us for landing on 25L. The Airbus either turned too slowly or failed to turn when instructed. Our closest point of approach was 100 FT and 1.69 miles. Two miles separated us when we were at the same altitude. The ATC Safety Coordinator told me it was good that we did not follow the ATC climb instruction, because the Controller had misinterpreted the data and thought we were above the Airbus.

Synopsis

SCT Controller providing OJT described a TCAS RA event caused when a foreign aircraft failed to turn as directed and the subsequent aircraft received a TCAS RA.

ACN: 941014 (2519 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC

Environment

Flight Conditions : VMC
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Tower : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Landing
Airspace.Class B : CLT

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : CLT
Aircraft Operator : Corporate
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Operating Under FAR Part : Part 91
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 941014

Events

Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During landing flare on Runway 23 the aircraft suddenly rolled right and shifted right of centerline. The winds were light and there had been no turbulence on the approach so this was totally unexpected. I briefly considered a rejected landing/go around but was able to roll the airplane level and get back to centerline. After clearing the runway, I queried Ground Control about the situation and learned that a corporate jet had been cleared for takeoff on Runway 18L at Intersection A at the same time we were landing on Runway 23. Due to the very close proximity of Runway 18L intersection A to the touchdown zone of Runway 23, Charlotte Tower should not be clearing aircraft for takeoff from this point while aircraft are landing on Runway 23. Runway 23 already poses its own set of challenges; a relatively short runway with a downhill slope. In addition Charlotte Tower very frequently uses Runway 23 when a tailwind component exists. Encountering jet blast while landing can place the aircraft in an unsafe position very quickly.

Synopsis

An A320 First Officer reported encountering jet blast on approach to Runway 23 at CLT from a corporate jet taking off on Runway 18L. Reporter suggested CLT Tower refrain from using Runway 23 for landing when Runway 18 is used for departures.

ACN: 940969 (2520 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC
Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Fuel Crossfeed
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 940969
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after departure we got an ECAM for center tank pump one low pressure. Because this had occurred on the last three flights, it wasn't totally unexpected. The Captain (non flying pilot) accomplished the ECAM procedure and everything worked as advertised. Approximately two hours into the flight the First Officer (flying pilot) noticed the fuel page was displayed on the lower ECAM screen. The right inner and outer wing tanks were full of fuel and flashing green. The right wing did not appear to be feeding. The left wing tanks appeared to be feeding normally except that the fuel on that side was significantly lower than expected. The center tank was empty. The Captain referred to the Flight Manual (FM) but didn't find a procedure to cover this situation. During this time the First Officer did some basic fuel calculations. The calculations considered trapped fuel in the right wing, inaccurate gauge indications and fuel being transferred from the left wing tanks to the right wing tanks. According to these calculations, there would potentially not be enough fuel to continue to the destination. Both pilots discussed the situation and concluded that both engines were being fed from the left wing tanks. Both were unclear as to whether the fuel in the right wing tanks was trapped or whether the full indications were inaccurate. In either case, the amount of fuel onboard was uncertain. The Captain decided to call Dispatch and Maintenance to get their thoughts and to help remedy the situation. He described our situation and indications to Maintenance who suggested resetting circuit breakers which had no effect on the problem. Both the Captain and the First Officer became increasingly uncomfortable with the developing situation and agreed that it would not be prudent to continue the trip. The Captain, with the concurrence of the First Officer made the decision to divert. Dispatch suggested diverting to a nearby airport. A normal descent and landing was accomplished and the flight continued to the destination on an alternate airplane.

Synopsis

An A320's fuel crossfeed system malfunction prevented the right wing fuel tanks from feeding an engine and possibly pumping the left wing tank fuel to fill the right wing tanks (inner and outer). The flight diverted to a nearby airport.

ACN: 940790 (2521 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 940790
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Inbound crew noted Captain's window leaking. My flight was canceled and Maintenance replaced the window overnight. The next morning, we were to ferry the aircraft. On departure passing through 10,000 FT, we heard a loud bang and air rushing through the Captain's front window. This occurred at about 4.2 psi differential and the cabin altitude started climbing. We leveled off at 10,000 FT; decided oxygen masks were not required and asked for vectors back. We declared an emergency, ran all checklists and flew an ILS to landing and parked at the gate uneventfully.

Callback: 1

The reporter states that there were no obvious defects in the windshield before or after the bang but a lot of rushing air made the decision to return very easy.

Synopsis

A320 First Officer reports a loud bang and air escaping from the Captain's forward windshield during climb. The windshield had been recently replaced by maintenance and the crew elected to return to departure airport.

ACN: 940787 (2522 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X
Problem : Malfunctioning
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 940787
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

At gate with APU powering aircraft at second engine shutdown (no GPU available or attached) ECAM started issuing faults and warnings. Captain felt faults were generated due to power shift. First Officer wanted to immediately start running Flight Manual and/or ECAM directed procedures. First Officer very technically orientated. In between steps he was selecting and displaying alternate FMGC modes and pages as well as reviewing system status and circuit breaker status. Captain was trying to get GPU power since the APU source change seemed to generate the issues. First Officer was moving and changing things so rapidly that system stability between steps would be impossible. Captain stopped the First Officer from additional changes and established a slower pace. Captain did not trust that the displayed faults were in fact real. We were on the ground and parked which seems a low threat environment. First Officer was rushing through procedural steps and trying to confirm/verify faults via other pages and system displays. Both crew members displayed a lack of trust in the aircraft fault and warning systems. Too often we see the sole repair step performed by maintenance is to cycle power to a system to clear a fault. That re-enforces to the crews that the aircraft is subject to system glitches and sensitive to power changes.

Synopsis

A320 Captain experienced multiple ECAM faults and warnings as the second engine was shut down at the gate and power shifted to the APU generator. First Officer complied with ECAM actions while the Captain believes a computer glitch is causing false warnings.

ACN: 940632 (2523 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Electrical/Electronic Panel & Parts
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 940632

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Evacuated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Captain and First Officer were on flight deck completing preflight duties prior to passenger boarding. AVIONICS SMOKE ECAM displayed accompanied by Strong Electrical/Burning smell - No Smoke observed. Accomplished QRH, Immediate Action/ECAM Procedure. Strong Electrical/Burning smell continued - Captain requested Fire Equipment from Ground Control. No passengers were on board. Had flight crew and station personnel leave aircraft and proceed to terminal. Captain powered down all electric systems and left aircraft.

Synopsis

An A320 Captain reported an AVIONICS SMOKE ECAM with strong electrical burning smell while parked at the gate before boarding. The aircraft is powered down and ARFF was summoned.

ACN: 939997 (2524 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Main Gear Door
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 939997

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I did the walk around. Upon arrival at left main gear, I noticed a part of the well seal hanging down. I wrote it up in the Log and called it in via radio. Had to do other stuff and left area. Upon return, First Officer advised me that mechanic had yelled at him that the problem was not "MEL-able or CDL-able" and that we would be taking a delay. After an hour and a half or so, mechanic comes up and says (sarcastically) "have a safe flight, Captain". As we prepared to depart, that little voice we have said...take a look. So I went down to the wheel well area and observed the piece of seal completely missing. The mechanic had obviously torn it off and signed the book as being repaired. I returned to the aircraft and wrote it up again. The same mechanic returned, as frustrated as ever and I informed him that the seal was missing and I would not accept the aircraft until repair was effected. We took another long delay and the aircraft was finally repaired and signed off correctly. We then departed. I have sent pictures of the wheel well area and the logbook pages to the Air Safety Chairman.

Lazy, incompetent, complacent management willing to overlook just about anything to keep the schedule going, the metal moving and the bonus coming. Perhaps if we had a more responsive Maintenance Department as well as more (and better) FAA oversight, stunts like this would not be acceptable. This is classic pencil-whipping from decades ago and I will not stand for it. Neither should the FAA.

Synopsis

An A321 Captain noticed a main gear door seal hanging down and entered the discrepancy in the aircraft Log and informed Maintenance. The maintenance Technician involved removed the dangling seal and signed off the Log book, which is not a proper repair. The flight is further delayed while a new seal is installed.

ACN: 939972 (2525 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Distance.Nautical Miles : 20
Altitude.MSL.Single Value : 5000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Component : 1

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Antiskid System
Aircraft Reference : X
Problem : Failed

Component : 3

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8500
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 4800
ASRS Report Number.Accession Number : 939972
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1900
ASRS Report Number.Accession Number : 940056
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Just prior to ILS approach, we had a malfunction of the primary brakes, nose wheel steering, and antiskid. After running checklists and talking with company, we elected to declare an emergency just to be on the safe side although we never suspected it would be other than a normal landing, but then would need to be towed to the gate. The landing was uneventful and plane was safely stopped and shutdown on the runway prior to towing.

Narrative: 2

On an angling downwind getting vectors, in intermittent snow, we got a brakes A/SKID N/WS (Anti Skid Nose Wheel Steering) fault ECAM. There were no "to dos" with the ECAM so I went to the book. The procedure had no actions either. I called Dispatch and asked him to get Maintenance on also. Maintenance had nothing to add so I told Dispatch what the situation was and told him I was going to declare an emergency and would need a tow off the runway. By the time I was done talking to everyone we were being vectored to final, I declared the emergency and reiterated that we would be stopping on the runway and would need to be towed off. The approach and landing were normal, we came to a full stop on the runway and switched to a discrete frequency and waited for the tow.

Synopsis

An A320 ECAM alerted BSCU during the initial approach phase so the crew completed the QRH procedures, flight crew declared an emergency and the aircraft was towed to the gate after landing.

ACN: 939943 (2526 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SLC.Tower
State Reference : UT
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Takeoff / Launch

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 200 ER/LR (CRJ200)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Landing
Airspace.Class B : SLC

Person

Reference : 1
Location Of Person.Facility : SLC.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 939943
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

I was providing training on the Local Center position. The trainee put an A320 into position and informed them of the traffic on final (a CRJ2). When the runway was clear from the previous arrival, the trainee cleared the A320 for takeoff. The A320 started takeoff roll and the trainee began to issue an IFR release to the Local East Controller. I instructed the trainee to not give the release, but to prepare to use the next arrival hole for his own IFR aircraft that had been waiting longer. From what I observed, the A320 was rolling a bit slowly, but still appeared to be departing, I couldn't see the arrival on about a one mile final because a RADAR scope, the ASDE screen and two people were in the way. From the STARS display, things still appeared normal. The A320 said they were aborting, and as I stooped down to see the CRJ2 out the window, the trainee keyed his microphone and told the A320 to exit the runway at H6. I expected the trainee to send the CRJ2 around, and couldn't override his instructions until it was too late. The A320 was already reading back the instructions and it would do no good to transmit over the reply. I observed the CRJ2 already climbing about 1/2 mile from the runway and it was obvious he was going around. I also thought the A320 had a chance to exit the runway before the CRJ2 crossed the threshold. I issued go-around instructions as soon as the frequency was available, but I am not sure if it was before the CRJ2 was over the runway. Again, I saw the arrival climbing and could tell that the aircraft was not going to attempt to land. Recommendation, move some of the equipment hanging from the ceiling that blocks the view of final for Local Center.

Synopsis

SLC Controller described a pilot initiated go around event after a departure aborted, the reporter listing tower equipment as obstructing the view of the aircraft on final and a late reaction to the developing event.

ACN: 939759 (2527 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23790
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 3478
ASRS Report Number.Accession Number : 939759
Human Factors : Human-Machine Interface
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

The aircraft had BSCU (Break Steering Control Unit) two and aft cargo heat inoperative. The brake issue required a check of the alternate brake system which required brakes chocked. We checked, yellow system worked and continued. At brakes set, accumulator pressure dropped to the 6 o'clock position which is almost a 1,000 PSI drop. Since BSCU 2 was MEL'ed, I had to review what would happen if we lost BSCU 1 also. The brake system would revert to yellow system pressure only with manual applications of brakes, maximum 1,000 PSI. It would be very easy to over step and blow tires in this condition. What concerned me more was the loss of yellow system pressure would also force us into emergency braking on the accumulator which had just shown a major drop of 1,000 PSI on its first brake application. I notified Maintenance, and asked for guidance, making sure he was aware of this situation. We took a two minute delay performing the brake check at the gate after brake release. We were number ten for takeoff and had at least another twelve minutes plus on taxi in addition to the four minutes to start both motors. We waited another eight minutes for Maintenance to confirm their acceptance of this situation. Then we entered the takeoff queue again. My concern is the levels of safety are starting to erode. We should fix these aircraft at maintenance stations with braking type problems. Granted a yellow system and BSCU 1 failure is remote but, given the potential outcome, I feel the risk is not worth it. I also called the duty manager to explain this issue and found he was over worked and could not help in a timely manner.

Synopsis

An A320 Captain reported concern over being dispatched with BSCU 2 (Break Steering Control Unit) inoperative.

ACN: 939580 (2528 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway XXL
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 939580
Human Factors : Workload
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On glide slope cleared for approach when Flaps 2 selected FLT CTL FLAPS FAULT on ECAM. Recycled Flap handle per ECAM. Fault remained [and we] discontinued approach. Requested vectors [and] climbed to 7,000 FT. Switched pilot flying/pilot not flying. QRH Landing with Slats or Flaps Jammed completed. Placed Flap Handle to 3 per QRH for full slats; applied Airspeed additive and Landing Length Correction for flaps one (or less) full slat approach and landing. [We] declared Emergency with ATC; contacted destination Operations, Maintenance and Dispatch. Passed TEST Info to Flight Attendants and had them prepare cabin for normal landing. Spoke to Passengers Switched pilot flying/pilot not flying. Requested vectors long Straight In ILS Final XX. [We made an] uneventful approach and landing [then] taxied to gate in flap/slat configuration.

Synopsis

An A320 ECAM alerted FLT CTL FLAPS FAULT when flaps 2 were selected for landing. An emergency was declared, the approach abandoned, the QRH procedure complete and a normal landing followed.

ACN: 939556 (2529 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Leading Edge Slat
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 939556
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 939557

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We had to return to the field with a slat problem...Upon reaching V1 the refueling light came on, I pointed it out to Captain but we both said press on due to our high speed into the airport. When I announced rotate the refueling light went out so we pressed on as normal...when we retracted the flaps we got a FLT CNTL slat fault on the ECAM and the flap indicator showed slats were not fully retracted. Captain flew aircraft and I proceeded with ECAM which had me recycle flap handle. This did not fix the problem so I pulled out the book and finished the irregular procedure and called Maintenance. The refueling light never came back on at all after airborne by the way. We also had to stop climb at FL200 and stayed at 220 KTS for the rest of the flight. I went back to observe the slats from the cabin and saw that the slats were indeed not fully retracted and something was hanging from the fuel door area. With the concurrence of Maintenance and Dispatch we returned to the airport uneventfully. Flaps and slats all extended properly.

Narrative: 2

During high speed regime First Officer noticed green "refueling" note come on then go off; when retracting slats from 1 to 0, received "FLT/C SLAT FAULT". Ran the ECAM, consulted the manual, contacted Maintenance Control, and inspected the slats [but] was unable to resolve problem. [We] elected to return to our departure airport [and] executed uneventful arrival. Upon post flight inspection noticed refueling panel, with 1 latch open, and 2 closed, bent in a "U" shape.

Synopsis

A320 flight crew experienced a refueling door note during takeoff which quickly extinguished. During slat retraction a FLT CNTL SLAT fault is indicated on ECAM and slats will not completely retract. Flight returns to departure airport where refueling door was found open and damaged.

ACN: 939516 (2530 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC

Aircraft

Reference : X
ATC / Advisory.TRACON : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Airspace.Class B : CLT

Person

Reference : 1
Location Of Person.Facility : CLT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 939516
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

The winds on the field were a constant crosswind component of 25 KTS, and gusting regularly to 30 KTS. The FLM instructed us to land Runway 18R, and depart Runway 23 at the same time. This was a very unsafe operation due to the fact that if a Runway 18R lander had to go around, which was highly likely due to the crosswind component, they would have been placed in a very unsafe situation with the departure off Runway 23. This operation was in effect for the greater part of an hour, putting numerous aircraft, and the flying public in great danger. The recommendation that I have is to determine that the flight paths of Runway 23 and Runway 18R intersect. If that is determined, then this operation will no longer be authorized.

Synopsis

CLT Controller described a landing Runway 18R and departing Runway 23 simultaneous operation contending the operation was unsafe especially given the potential of a Runway 18R go around.

ACN: 938528 (2531 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZOB.ARTCC
State Reference : OH
Altitude.MSL.Single Value : 34000

Aircraft

Reference : X
ATC / Advisory.Center : ZOB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : None
Airspace.Class A : ZOB

Person : 1

Reference : 1
Location Of Person.Facility : ZOB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 938528
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Facility : ZOB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 938551

Person : 3

Reference : 3
Location Of Person.Facility : ZOB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Handoff / Assist
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 938727

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Associate RADAR "slant OK'd" a Data Block from the ONT Sector thinking he was starting track on a departure off of YYZ southbound, when he realized that he has picked the wrong call sign, he canceled the flight plan on the aircraft instead of looking to see who the aircraft belonged to. The RADAR Controller then looked over and saw what was going on, and the process began with the CIC giving orders to start a track with the call sign then put in a VP with the code for quick flight plan processing, then hand offs and coordination was taken care of. Recommendation, if you have to "slant OK" a data block you need to check all your information before doing so.

Narrative: 3

I was working the D-Side at Warren and Ontario Sectors when the following event occurred. An A320 checked on frequency and the RADAR Controller acknowledged. The aircraft was approximately 10 miles inside our airspace boundary when it first became apparent that the Data Block for the A320 had disappeared, and all we had was a Limited Data Block on the RADAR screen. When we tried to restart a track, we noticed that the Flight Plan information had been deleted. We then started a track with the call sign and created a new flight plan. I then pointed out the new track to the high sector to the north of us, Sector 36, and we handed off the aircraft to the DKK sector. Recommendation, be absolutely sure of the aircraft identity before starting tracks, dropping track on an aircraft, etc.

Synopsis

ZOB Controllers described a confused Data Block event when needed information was dropped when an Associate Controller inadvertently manipulated the Data Block information.

ACN: 938133 (2532 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 938133

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 4000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 938140

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Due to a #1 thrust reverser ECAM during cruise flight, a diversion was initiated as per Flight Manual Irregular procedure and after consulting with Dispatch and Maintenance Control. Flight proceeded normally to the diversion airport where an overweight landing was accomplished. Landing was normal with vertical descent less then 300 FPM. Rollout was normal except for the inoperative #1 reverser. Normal taxi to the gate. Aircraft was about 6,000 LBS over maximum landing weight.

Synopsis

An A320 flight crew diverted to the nearest suitable airport following an ENG 1 THRUST REVERSER ECAM message at cruise altitude.

ACN: 937988 (2533 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine Thrust Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1455
ASRS Report Number.Accession Number : 937988

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 4095
ASRS Report Number.Accession Number : 937986

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ENG 1 REVERSER FAULT - ECAM illuminated at FL370. No other indications noted. Airplane in smooth air continued to fly well. Two jump seating Captains, one typed in the airplane, visually inspected the engine from the cabin and determined it looked normal. The AOM said to LAND ASAP. After discussing our options with Dispatch, Maintenance Control, and the expanded crew and with reverser pressurization and deployment a possibility, ZZZ was determined to be the best option for its longer runway in case of directional control issues. At the point of declaring an emergency, the distance to ZZZ and ZZZ2 made descent into ZZZ more manageable. The longer runway was requested as was CFR equipment as a precaution in the event the reverser situation deteriorated. The flight attendants were advised and took appropriate actions to prepare themselves and the cabin. Landing was normal.

Narrative: 2

The checklist action item is to land ASAP. It further notes that if a second ECAM occurs that indicates the reverser is pressurized, the effected engine should be brought to idle.

Synopsis

An A320 flight crew declared an emergency and landed with priority handling at their destination when they received an ENG1 REVERSER FAULT ECAM message.

ACN: 937938 (2534 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZJX.ARTCC
State Reference : FL
Altitude.MSL.Single Value : 36000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZJX

Aircraft : 2

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : B737-300
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZJX

Aircraft : 3

Reference : Z
ATC / Advisory.Center : ZJX
Aircraft Operator : Fractional
Make Model Name : P180 Avanti
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Phase : Cruise
Airspace.Class A : ZJX

Person : 1

Reference : 1
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 937938
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 937940

Person : 3

Reference : 3
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 937960

Person : 4

Reference : 4
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Last 90 Days : 121
ASRS Report Number.Accession Number : 938371

Person : 5

Reference : 5
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 169
ASRS Report Number.Accession Number : 939112

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working an A320 level at FL360 flying northbound at R86. I took a hand off from R17/85 on a B737 who was at FL370 and showing a descent to FL350. According to the SOP, the B737 must cross the boundary of R17/85 and R16/86 at FL350. When I took the hand off on the B737 I figured with 80 miles between the two aircraft the B737 should easily get underneath my A320, so I flashed through my B737 to R16 which is allowed according to our SOPs it is an approved Automated Information Transfer (AIT) procedure. R17/85 took the hand off on my A320 and did not notice the situation developing. I noticed soon after that the B737 was not starting down to FL350 and immediately assumed he was issued PLYER at FL350 which is a common practice in the area but not a requirement. The requirement is to cross the boundary at FL350 and PLYER is outside the boundary. As soon as I noticed I called D17/85 and asked if the B737 was issued PLYER at FL350 and he said he was and that we should stop him at FL370. At that point I assumed he was still talking to the B737 and that he was told to maintain FL370. The aircraft had 15 or so miles between them at that point. I yelled across the room that I had assumed RADAR contact on the B737 and took track control of the aircraft. 

The aircraft was a few miles from PLYER so I still assumed he was level at FL370 because he had not started down. I was still questioning who was talking to the aircraft so I yelled over to R17/85 asking them if they were talking to them and they said no, so I asked R16 to make sure the aircraft was stopped at FL370. R16 then asked the B737 if he was "issued anything lower than that" and I believe the pilot at that point forgot he was issued the restriction and started down. R16 said no maintain FL370 and the pilot reported he was level at FL370 after showing a 500 foot drop. But it was too late to prevent an RA for the A320 and he started descended in response to a RA and notified me. I then noticed that there was P180 southbound at FL350 and told the A320 to continue descent to FL340 and turn 40 degrees right. I turned the P180 just 10 degrees right because the warning areas where hot off to his right. The A320 did not descend all the way to FL340; I believe the RA was telling him to climb. If he would have descended when I told him there would be no loss of separation but I believe the RA advisory confused the pilot. I recommend a standard crossing restriction for the RSW arrivals so that everyone is on the same page about altitude assignments.

Narrative: 5

Approximately 60 miles from PLYER we were cruising at FL370 and ZJX ATC issued a clearance to cross that fix at FL350. As pilot flying I entered the restriction in the FMC and the pilot not flying confirmed it was correct, I then entered 350 in the MCP and the pilot not flying once again confirmed. We changed frequencies and the pilot not flying checked in with the issued crossing restriction. At the initiation of the descent we received a TA to monitor vertical speed; I decreased our descent rate, and was asking the pilot not flying to advise ATC when we received an RA to climb. As we had already started a gradual climb I continued back to FL370 and was quickly clear of conflict. The Controller immediately asked us if we were at FL370 and we confirmed that we were climbing back up and that we had been following instructions to make that restriction. I asked the Controller if we should not have been given that clearance and he acknowledged that that was a fact. The pilot not flying did confirm that he saw the other aircraft pass by.

Synopsis

ZJX Controller described a loss of separation event when the altitude of crossing traffic was uncertain because of unclear crossing restrictions, compounded by Automated Information Transfer (AIT) procedures.

ACN: 936773 (2535 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 2300

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 936773

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On initial climb out just after thrust reduction and prior to flap retraction, number one engine loud bang and aircraft yaw followed by second bang. Captain retarded number one thrust lever and engine continued to cause high airframe vibration and banging sound. Number one engine ceased to run and high EGT noted immediately followed by engine shutdown ECAM. Captain remained the flying pilot and I proceeded with declaring an emergency, pulled out QRH to check for immediate action items then continued with ECAM actions. After ECAM was complete the overweight landing check list was completed. Captain made a very stable approach followed by a smooth landing. Emergency vehicles circled the aircraft on the ramp before we proceeded to the gate; internal mechanical failure of number one engine.

Synopsis

An A320 engine failed with a loud bang during takeoff after the thrust reduction. An emergency was declared and the crew returned to the departure airport for landing.

ACN: 936697 (2536 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1800

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Hail
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Rudder Pedal
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 936697
Human Factors : Confusion
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

Maintenance write up in logbook was Captain's rubber pad on left rudder pedal needs reattaching, (I can't remember exact wording). Captain's left rudder pedal rubber pad that covers metal apparently for traction on pedal to avoid slippage of foot under pressure. The result of maintenance action, fix, or work completed by maintenance personal. Maintenance signed off logbook as reattached, OK for flight operations. On Taxi out all procedures and checklists were completed In particular the flight control check to confirm proper rudder control. Once cleared for takeoff the take off roll was normal, wind was gusting and attention was given to that. Still normal conditions I felt. However where it went wrong was right after liftoff. What I consider more pressure was going to be needed for control because of the current wind conditions WAS apply to the rudder PEDALS to control the aircraft was given. At that point the maintenance fix to the left rudder rubber pad came loose and caused my left foot slip a considerable amount to cause a moment of a control issue. At some point I was able to get my foot in a position as to regain any control that was lost, BACK IN CONTROL. We are talking about SPLIT seconds during this time. I did not even have time to communicate with my Co-Captain what was happening even though a second after it all took place I believe he did comment. Once safely airborne my Co-captain and myself had a discussion about how to continue rest of flight. We decided rather than return to the departure airport and put the passengers and aircraft in a situation of a heavy overweight landing (which would possibly require extreme rudder pressures due to a heavy landing weight) that we would continue to our destination, discuss and evaluate situation and communicate with dispatch on the issues. On landing we decided even though it appeared that my the Captain's rudder pedal was usable it was not up to our Air Carrier's SAFETY STANDARDS OF MEETING AND EXCEEDING ALL SAFETY STANDARDS. So out of MY (the CAPTAIN) concern, I made the decision to have the First Officer (Co-Captain ) land the aircraft just in case of any issues, engine failure etc. Its this Captain's opinion that strongly feels that SAFETY conditions at our Air Carrier are being affected by the huge push from the top personal to GET THE AIRCRAFT OUT ON TIME for financial incentives. I know they would care first about the Safety of our passengers and aircraft. However I don't think they are possibly aware of what really is happening out on the line. I find myself having to watch and double check everything that may be a flight safety issue. However, the the culture of our Air Carrier's workplace STILL FEELS TERRIBLY RUSHED to GET the aircraft off the gate!!! So in conclusion, in order to make it all SAFER when ANY issue comes up at the gate, no repercussions or discipline to employees will happen, for a simple late gate Departure.

Synopsis

An A320 Captain's rudder pedal rubber friction pad came off on takeoff in a cross wind with a rudder correction applied causing a momentary loss of control until the Captain's foot reestablished rudder contact.

ACN: 936694 (2537 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 29000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 936694
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise altitude FL290, ECAM alerted LGCIU #1 FAULT. [We] complied with ECAM, QRH, [and] supplemental procedures. Notified Dispatch via ACARS [and was] instructed to return to the departure airport, [with the] understanding [that an] over weight landing will be performed. Verified, again, that Maintenance was instructing our flight for an air return [and the] air return [was] confirmed. Notified ATC, requesting an air turn-back at [the] request of Operations and this not an emergency. Notified and briefed [the] cabin of [the] air return and approximate ETA, and that this is not an emergency. Then the passengers were briefed of return and that this was not an emergency, but a request by Operations for Maintenance to perform repairs necessary for flight to continue. [The] flying pilot requested lower altitude to burn off as much fuel as possible. Referred to QRH for overweight landing procedures and complied. Prepared [the] aircraft for arrival [and the] flying pilot briefed [the] arrival and inoperative systems that could impact flight. As an extra precaution, [we] notified Center and Approach to have equipment standing by. Briefed passengers that fire equipment may be visible upon landing and this is only for a precaution. Landing [was] uneventful. Landing weight [was] 149,000 LBS and sink rate [was] less than 150-100 FPM. Released fire equipment and proceeded to [the] gate.

Synopsis

After an A320 ECAM alerted LGCIU 1, Maintenance and Dispatch directed the crew to return to the departure airport for maintenance. An emergency was not declared.

ACN: 936581 (2538 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 28
Flight Phase : Final Approach
Airspace.Class B : ORD

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 450
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 936581
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17500
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 936582
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Air Traffic Control
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Ambiguous

Narrative: 1

On about 8 mile final the ILS Glideslope became [unreliable]. As we continued to fly the glideslope we were actually about 500 to 800 FT low in altitude. At the outer marker, WILLT, the weather became VMC and noticed we were low. We also noticed that a jumbo jet was making a 90 degree turn to line up on Runway 28. This aircraft interfered with the localizer signal. The new holding area on the approach end of Runway 28 needs to be looked at. The aircraft that landed ahead of us also complained about a very similar phenomenom on final.

Narrative: 2

While still in IFR conditions; approaching the final approach fix for Runway 28, the glideslope slowly moved to almost 2 dots high. I slowly began correcting and just as we broke out I picked up the PAPI and noted we were low. Just then, Tower called us low. The First Officer and I both noted a B747-400 holding short of Runway 28 which was disrupting the ILS signal. I corrected to glide path and continued the approach. ORD Tower was contacted and they indicated this has happened several times since the new south side access was created to hold short of 28 for departure.

Synopsis

A320 flight crew experiences an unreliable glideslope during an ILS approach to Runway 28 at ORD. At WILLT the glideslope returns to normal and a B747 is noted holding short for Runway 28 on the south side. The Tower indicates that it has happened several times since the new south side access was constructed.

ACN: 936143 (2539 of 2908)

Time / Day

Date : 201103
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Hydraulic Main System - Regulator
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 936143
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

This is the fourth and fifth [Hydraulic] Accumulator in ZZZ from [Contract Repair] Vendor in the last two weeks. Both [accumulators] will not hold nitrogen at all on air side; just blows through [diaphragm] to hydraulic side of accumulator. Both accumulators came from the [same Vendor Repair] Shop again, and were put on the refused list. Something needs to get done soon because this is making our job very difficult to do when parts need to be tested before being installed on aircraft.

Synopsis

A Mechanic working an A320 A-Check in a hangar, reports about a fourth and fifth Hydraulic System Accumulator from a Contract Repair Station Vendor that will not hold nitrogen at all on the air side of the accumulator diaphragm; air just blows through to the hydraulic side of both accumulators.

ACN: 935469 (2540 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12900
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 4380
ASRS Report Number.Accession Number : 935469
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

For almost six years on the Airbus, the only time I have used alternate brakes has been in the simulator. Now I have used them twice this month! The first was caused by a No. 1 tachometer and BSCU (2) failure and [that] caused wheels one and three to lock upon touchdown. Autobrakes were not selected but shortly after landing we received ECAMS for an autobrake failure then for a BSCU failure. On this second occurrence, my First Officer selected autobrakes to MED for strong crosswinds. Shortly after landing he advised we were not getting braking and manually applied the brakes. There was no ECAM but the gear page showed AUTO BRAKES in amber and Alt Brakes in green. Although alternate brakes did have a different feel, there was no problem controlling or stopping the aircraft. (Although it would be more exciting in SNA!) When I called in the gripe to Maintenance, he said he was working another similar Airbus brake problem.

Callback: 1

The lack of auto brakes on touch down was the first indication that something was amiss, then the ECAM gear page appeared showing auto brakes in amber and alternate brakes in green. Automatic switching to the alternate brake system is normal with failure of the BSCU although none was indicated. Three weeks prior two tires failed on touch down due to a BSCU failure along with a number one tachometer failure.

Synopsis

An A320 Captain experienced auto brake failure on touch down. The system subsequently switched automatically to the alternate brake system.

ACN: 935339 (2541 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Flap Fairing
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Wing Trailing Edge
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 935339
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

The damaged seal was found on walk around. I was told to get the damaged piece trimmed and make a long term deferral item to cover it. We were also working a vibration issue on the aircraft. I made the long term deferral item using company Maintenance procedures XX-XX for an item that requires attention, but is otherwise ok for service. It turns out that I should have used the aircraft's configuration deviation list (CDL) chapter-27 aircraft, as the controlling reference.

Callback: 1

Reporter stated he was working as a Lead Mechanic that day and was actually deferring the seal for another Mechanic doing the trim repair. One of the left wing flap fairing seals had partially separated from a flap fairing where the fairing mates up to the wing flap lower surface after the flaps are retracted.

Reporter stated the MELs/CDLs are not always easy to use and can be confusing.

Synopsis

A Lead Mechanic reported making an improper deferral of a wing flap fairing seal that had partially separated on an A320 aircraft. He deferred the seal under a company Maintenance procedures manual instead of the aircraft's configuration deviation list (CDL) manual.

ACN: 935236 (2542 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.MSL.Single Value : 14000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class E : ZLA

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZLA
Make Model Name : PA-46 Malibu/Malibu Mirage/Malibu Matrix
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Flight Phase : Cruise
Airspace.Class E : ZLA

Person

Reference : 1
Location Of Person.Facility : ZLA.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 935236
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 was climbing out of SCT's airspace, 130 and below, into ZLA's Sector 21/22's airspace (140-230) when an RA went off with a PA46 who was flying VFR through the departure corridor and had been advised of such. Traffic had been called to the A320 and who had been vectored off of the departure procedure to avoid the PA46, as well as have been given a speed to increase its climb rate through 150, but the RA still went off. The A320's rate of climb seemed to ensure that it would top the PA46, which it did, but the RA happened regardless. Recommendation, SCT Approach climbs all departures into ZLA's sky, which is an unusual procedure and very unusual at any center. SCT Approach could have been made aware of the VFR potential traffic, even though it was outside of their sky so they could give traffic advisories as we did. The VFR traffic could also have been vectored away from the departure corridor, which is unpopular amongst VFR pilots but which would provide a safer situation.

Synopsis

ZLA Controller described a TCAS RA event after VFR traffic conflicted with IFR traffic in a busy departure corridor, the reporter claiming SCT should not climb VFR into ZLA's airspace without coordination.

ACN: 934840 (2543 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 4000

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Person : 1

Reference : 1
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1300
ASRS Report Number.Accession Number : 934840
Human Factors : Other / Unknown

Person : 2

Reference : 2
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 934841

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Aircraft
Primary Problem : Environment - Non Weather Related

Narrative: 1

Controller vectored us right over a mountain top, 3 miles NE of Okain Intersection on the approach at 3800 ft. We got a terrain pull up event night IMC. The weather was marginal VFR as we were in the snow storm from SFO now over SAN. It was the end of a long duty day with icing, thunderstorms, and moderate turbulence. These schedules are onerous and now we have to deal with this event. We called ATC and they said that it was a legal altitude for them and that they will not raise it to avoid this in the future due to pressure from the airlines not to have people to high. This event will happen again as no resolution came from the call with ATC. Suggest just 200 ft more to 4000 ft and this could be avoided possibly in the future.

Narrative: 2

Arrival weather was marginal VFR with moderate chop and moderate rime ice. Downwind vector was at 5000 ft. Cleared to 4000ft and base turn, saw the towers and hill being vectored over, then cleared to 3800 ft. over hill got an RA for terrain. Performed escape maneuver and climbed to 4500ft and now localizer alive and VFR. Told approach and also OK to continue. On profile at REEBO (FAF) and landing normal. Had a conversation with TRACON supervisor and not sure why GPWS was triggered as at legal vectoring altitude.

Synopsis

Air Carrier arrival to SAN received a GPWS during a descent to 3800, one reporter suggesting an increase in the 3800 altitude procedure to prevent future events.

ACN: 934839 (2544 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : LAS

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAS
Make Model Name : Light Transport, Low Wing, 2 Turbojet Eng
Crew Size.Number Of Crew : 2
Flight Phase : Initial Climb
Airspace.Class B : LAS

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 934839
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 934843

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Air Traffic Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Upon receipt of takeoff clearance on Runway 1R in LAS, tower advised that another aircraft would be cleared for takeoff from 1L but that it would be no factor. Shortly after takeoff at approximately 300 feet we received a resolution advisory, directing us to "maintain vertical speed". This was shortly after our power reduction, so to maintain the vertical speed we decreased our airspeed. I noticed that the aircraft to our left was co-altitude and very close on TCAS. Later, the flight attendant remarked that the aircraft on our left during takeoff was very close. The two runways are very close together at LAS, and it seems that two aircraft should not be allowed to take off at exactly the same time. There's very little margin for error. Also, most aircraft taking off on the left runway are smaller civilian aircraft. These pilots may not have very much experience. Any deviation from center line could be disastrous if both aircraft are cleared for takeoff at the same time. Also, our aircraft flight director takes off in runway track or NAV if on an RNAV SID, where others may take off initially with runway heading if not RNAV capable. If there's a left crosswind this would cause the aircraft taking off in heading from the left runway to veer into the path of the aircraft on the right runway. I've seen this problem before at LAS when taking off to the north, so this is an issue that needs to be addressed.

Narrative: 2

On take off at about 1000 AGL we got an RA to monitor vertical speed. We complied with the RA. We were cleared for take off on 1R and the business jet was cleared to take off Runway 1L. I believe the RA was with that traffic. I believe the tower told us and biz jet about our respective traffic.

Synopsis

LAS Air Carrier departure from Runway 1R received a TCAS RA from traffic departing Runway 1L, traffic information was provided to both involved aircraft.

ACN: 934664 (2545 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Hydraulic Main System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic Fluid
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 934664
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

An A320 aircraft took three accumulators from same repair vendor to fix [a failed accumulator]. Two accumulators where bench tested before installation and leaked. Air came out fluid side [of accumulator] instantly. And Vendor said they were bench tested [at repair station]. There is no way possible from the leaks we had, that the vendor checked these after rebuild. 

The first accumulator that was put on aircraft on midnight shift filled the hydraulic side of the aircraft causing three hours of bleeding [time] to clear out nitrogen. Took pictures of accumulator blowing glove off the fluid ends of accumulators for proof that this is really a vendor problem of pencil whipping parts.

Callback: 1

Reporter stated midnight shift mechanics were doing an A-Check on the A320 and performing a nitrogen pressure check of the green, yellow and blue hydraulic systems accumulators. One accumulator for each hydraulic system bolts into the each hydraulic system's high pressure manifold. Each accumulator has a rubber bladder that separates the air pressure side of the accumulator from the hydraulic fluid. Mechanics had applied nitrogen pressure to the air side of one of the accumulator's already on the aircraft to check the bladder integrity and realized the bladder was allowing pressurized nitrogen to contaminate and aerate the system hydraulic fluid. 

Reporter stated midnight shift and day shift mechanics changed four accumulators before a serviceable accumulator was found. After recognizing there was a serious problem with the vendor repaired accumulators, he and other mechanics placed a surgical type latex glove over the hydraulic fluid side of each accumulator from the vendor. Nitrogen air was than applied and the Vendor accumulators with the failed internal rubber bladders blew the latex glove off of the fluid side of the accumulator, indicating the accumulator was leaking internally. The rubber bladder in each accumulator is supposed to be able to handle up to 3,000 pounds PSI of pressure.

Reporter stated each of the failed Vendor repaired accumulators had a signed-off FAA-8130 Serviceable Tag.

Synopsis

A Line Mechanic reports a hydraulic system accumulator bladder test performed during during an A-Check revealed internal leaking of an system accumulator. Three replacement hydraulic accumulators from the same vendor repair station were rejected, even though they were signed-off FAA-8130 serviceable tags, before a serviceable accumulator was actually found.

ACN: 933736 (2546 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : Antiskid System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 25
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 933736
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We departed with the BSCU 1 and the #1 fuel pump in the right tank deferred inoperative. All systems functioned normally for the flight. At about 500 FT AGL on an approach to Runway XX circle YY, we had multiple ECAM's indicating BSCU 1, BSCU 2, Nose Wheel Steering, and Anti Skid were inoperative. We executed a go around and took delaying vectors in order to secure the systems and properly prepare for landing. We determined we were going to have to land in that configuration and declared an emergency. After landing we planned to taxi clear of the runway on one of the high speed taxiways, shut down and get towed to the gate. We experienced moderate chop the entire time we were in the pattern through final landing. I felt there was no need to prepare the cabin so we informed the flight attendants and passengers that we would be making a normal landing and that emergency equipment would be following the plane as a precaution. The landing and roll out was uneventful. I noted the airspeed was 110 KIAS at 55 KTS ground speed during the roll out so I didn't need to apply brakes until about 30 KTS ground speed. As a result there was virtually no chance of skidding the tires. We cleared the runway, the Fire Chief cleared the underside of the plane and we were towed to the gate after 30 minute wait. The First Officer's assistance and professional work was invaluable in ensuring this was an uneventful emergency. It felt like a simple training simulator session.

Synopsis

An A320 departed with BSCU 1 MELed and on final approach at the destination the ECAM alerted BSCU2, Nose Wheel Steering and Antiskid INOP. A go around was executed, the ECAM actions completed and a normal landing completed followed by a tow to the gate.

ACN: 933734 (2547 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 15000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14700
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 933734
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 6550
ASRS Report Number.Accession Number : 934042
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After having the number 2 air conditioning pack deferred, we had a normal taxi-out and takeoff. Passing 8,000 FT the Captain and I noticed a Land ASAP (Amber) Memo. There had been no ECAM warning or caution associated with this ECAM Memo. The Captain selected the RCL (recall) button to see if there were any items displayed. Sure enough, there was an Avionics Smoke procedure displayed. There was no perceptible smoke. We asked the Flight Attendants if any indication of smoke in the cabin, and there was none. Both the Captain and I were suspect of this problem, because there didn't appear to be any direct confirmation of the smoke. In addition, the lack of any other ECAM warning or caution made us somewhat suspect of the Land ASAP Memo. The Captain sent a four page ACARS message describing the problem to Maintenance. We also referenced the Flight Manual for further guidance with this particular issue. After further thought, the Captain decided for a return to our departure airport, the closet airport. We informed ATC of our intent to return to our departure airport, and were cleared direct to the airport. We notified Dispatch and the Flight Attendants of our situation. We referenced the overweight landing checklist. We told the flight attendants this would be a normal landing, but did inform them of our indications in the cockpit. After a normal landing, ATC informed us there was no visible smoke. We taxied, parked, and accomplished all required check-list. The Avionics Smoke ECAM message was not displayed after shut-down.

Narrative: 2

During the climb out I noticed a Message that stated Land ASAP. I pointed it out to the First Officer. We looked at everything to see if we could find a problem. We could see no other indications. No ECAM or other warning lights. I thought it must just be incorrect message. With further investigation, after selecting ECAM recall there was an ECAM for Avionics Smoke. We checked the Flight Manual and it said to follow the ECAM if there was perceptible smoke. There was no smoke, flight attendants perceived no smoke and there were no warning lights. I was sure there was no real problem. In the past I had had this message before and an ECAM engine failure turned up. Well, it was easy to figure out the engine was fine. Not so easy to know if there is smoke in the avionics compartment. I sent some ACARS messages to Maintenance telling them what was happening to see if they had any ideas. We didn't get a response. Originally I thought we would just continue, but then I started feeling more uncomfortable with the situation and told the First Officer that we should return to our departure airport. I asked for a clearance back to our departure airport and notified Dispatch of our plan. Dispatch said our departure airport Operations was notified and we said that as of now the only thing we needed was a mechanic to meet us. We returned to our departure airport with an uneventful landing, except the landing was overweight. We wrote up the overweight landing, but since the rate of decent at touchdown was 100 FT per minute there was no inspection required. There were never any more warning lights and never any smoke. We parked, and our departure airport Operations did a great job of helping us. The mechanic did some computer resets; we refueled, got a new release from Maintenance and Dispatch and left again with no further problems.

Synopsis

A320 flight crew discovers a land ASAP ECAM Memo during climb out. No previous ECAM warnings had been received by the crew but when the recall button was pushed, an Avionics Smoke message is displayed. After some discussion the crew elected to return to the departure airport.

ACN: 933085 (2548 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 933085
Human Factors : Other / Unknown

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While turning in to park at ORD, a fuel truck that didn't stop required me to abruptly stop the airplane. The fuel truck driver then stopped, backed up, and allowed us to continue to the gate. The lead-in lights were on and the light was green indicating it was safe to proceed to the gate. The light turned red when we stopped for the fuel truck. The truck appeared to be in a hurry to its' next fuel stop, and wasn't adequately aware of the aircraft in its' vicinity.

Synopsis

A320 Captain reports stopping abruptly to avoid a fuel truck that fails to yield during taxi to the gate at ORD.

ACN: 932590 (2549 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 932590
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Before taking the runway we noted the hottest brake temperature was 255. FLEX takeoff power was applied and at 80 KTS the ECAM displayed "hot brakes delay take off." Rejected Take Off (RTO) procedures were applied and the aircraft was brought to a stop on the runway. The Captain notified the Flight Attendants to remain seated and the First Officer notified the Tower we were still on the runway. We exited runway and requested a gate return.

We brought the aircraft to a stop abeam the gate suspecting tire damage. We then contacted Operations for airstairs to deplane the passengers. The hottest brake temperatures observed were 885 on the left side and 885 on the right side.

Synopsis

An A320-200 flight crew rejected their takeoff at 80 KTS upon receipt of an ECAM "HOT BRAKES DELAY TAKEOFF" message.

ACN: 932555 (2550 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class C : ZZZ

Component

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 932555
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure off of the runway landing gear failed to retract. Landing gear handle would not release. Max speed prior to retraction was 160 KIAS. Advised Tower of intent to return, positioned emergency vehicles on the runway, contacted Operations/Station and Dispatch via ACARS, and directed Flight Attendants to perform cabin advisory. Reviewed all pertinent procedures, uneventful approach and landing. Landing was conducted within normal weight limits.

Synopsis

A320 Captain discovers landing gear will not retract after takeoff. Flight returns to departure airport uneventfully.