ACN: 932117 (2551 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 8000

Environment

Light : Dusk

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Aircraft : 2

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 932117
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 932119
Human Factors : Troubleshooting
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During arrival Brakes BSCU 2 fault occurred, followed ECAM procedures. Declared emergency due to possible single brake system failure. Brakes operated normally, but no nose wheel steering after stopping, so we were towed to the gate.

Synopsis

An A320's ECAM alerted BSCU during the arrival descent. An emergency was declared and after landing the brakes operated normally but the steering was inoperative so the aircraft was towed to the gate.

ACN: 931878 (2552 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2200
ASRS Report Number.Accession Number : 931878
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Fatigue
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We had a short layover (11:59) scheduled for the second night of our 3 day trip. We checked into the hotel XA:45 (+1 hour Home Base Time) and I got to bed at approximately XB:45. Five hours and fifteen minutes later I was awoken by loud noises and beeping sounds outside of my window. Apparently my room over looked the rear deliver docks for the hotel and trucks were arriving for their weekday morning deliveries. I was unable to go back to sleep so with about 5 hours of rest I started my day. Check in was at XL:00 and our day was scheduled for 7:05 block time, 10:25 duty. Our first segment was uneventful. While planning for our next segment we noticed that the destination weather had deteriorated significantly (SN, FZFG, 1,400 RVR) and we flight planned accordingly. Just prior to push we were advised of an EDCT time for destination and remained on the gate with our passengers on board for 30 minutes. We then pushed, taxied out and shut down on a taxiway near the end of the runway. There was a discrepancy in our wheels up time so we spent some time going back and forth with ATC and Dispatch to clarify things- our destination then went to a ground stop. We remained on the taxiway for 2 hours updating the passengers, talking with Dispatch, the flight attendants and eventually Company Operations as time started to become a factor and extended delay kits were issued. The flight attendants were calling with passenger questions as they were hearing about local airports and destination power outages over their cell phones. After a busy 2 hours sitting on the taxiway we were cleared for takeoff. Approximately 20 minutes after takeoff, while in the climb, in IMC with engine anti-ice on we received an ENG BLEED LEAK ECAM. We ran checklists, talked to Dispatch, talked to Maintenance and were unable to fix the problem. Unable to continue into known icing conditions we coordinated with ATC, declared an emergency, turned around and headed back to the departure airport. The passengers were briefed and the flight attendants were given a cabin advisory. Aside from the 15 minutes of (very) moderate turbulence on the descent the return was fast paced but uneventful. Almost 4 hours after boarding and 3 hours and 8 minutes after push back we were back where we started. It felt like an action packed PC had just successfully ended. The Captain and I debriefed and then headed down to operations. Upon arrival in operations at XT:45 we were advised that all of our passengers had been accommodated on another flight that was just leaving for our planned destination. We were then told by the station that there was another Airbus inbound, arriving at XU:15 that they planned to turn and we would ferry it to our planned destination at XV:00. We called the crew desk and were told that was indeed the plan and that we would also operate back to this airport for the overnight. When the Captain advised that would put us at 9:56 minutes of flight time and 14:00 hours of duty for the day as well as get us back into our over night fifteen hours after check in (if everything went as planned) the response was "it's legal." We did not receive any calls to discuss the emergency on our last flight or to check on our status. We were also never asked if we were OK to go with the revised plan, it was legal!! Feeling the adrenaline rush start to decline I headed upstairs to the terminal for a cup of coffee and to get myself in the mindset for another 5 hours of flying. I was sitting at an empty gate waiting for our airplane to arrive when I hit the proverbial "brick wall." I attempted to contact the Flight Duty Manager several times over the next 20-30 minutes but was unable to reach him. After doing a self assessment on my fitness to fly I determined that I was approaching my limits. I would not be able to do a round trip, arriving back at the destination after being awake 18 hours, with 10 hours of flight time and 14 hours of duty, safely. I called the Crew Desk and advised them I was fatigued.

Synopsis

An A320 First Officer called in fatigued after being reassigned flying that would extend his awake time to nineteen hours with 9+56 flight hours and fourteen hours on duty.

ACN: 931871 (2553 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 196
Experience.Flight Crew.Type : 4357
ASRS Report Number.Accession Number : 931871
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Detector.Person : Ground Personnel
When Detected : In-flight
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had an auto brakes fault upon landing but auto brakes were not selected. After a normal approach and very smooth touchdown by my First Officer, I thought he got on the brakes very quickly. I was about to say something when we got an ECAM BRAKES AUTO BRK FAULT, shortly followed by another ECAM BRAKES SYS 2 FAULT; the rest of the rollout was normal, albeit with some vibration that disappeared as we slowed down. I took control of the aircraft, further slowed and felt that braking and turning seemed normal so I carefully made the second high speed turn-off and proceeded slowly to the ramp. While taxiing, I continued to evaluate the brakes and steering; steering was completely normal and the brakes seemed spongy, but had no problem slowing and stopping the aircraft. Brake temperatures were normal. Because of the ECAM's and sponginess, I had my First Officer request a tow-in to the gate. (As we entered the ramp, I noticed that the brake temps on two and four were increasing normally, but one and three were cold.) The tug driver advised we had two blown tires, number one and three mains. Maintenance met us and I advised the Mechanic of the sequence of events. The maintenance post flight report showed the ECAM's and a series of system failures; apparently it was the number one wheel tachometer that failed as that was replaced overnight (along with the tires) and the airplane was returned to service.

Callback: 1

This event was the first of two brake failure events the reporter experienced in one month. The second event did not result in any blown tires.

Synopsis

An A320 Captain experienced an auto brake fault on landing although auto brakes were not selected. Upon arrival at the gate two flat tires were discovered.

ACN: 931793 (2554 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Icing
Weather Elements / Visibility : Rain
Work Environment Factor : Temperature - Extreme
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Liferaft
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Exterior Pax/Crew Door
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 931793
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Function.Maintenance : Inspector
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 932685
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Person : 3

Reference : 3
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 932686
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Human Factors

Narrative: 1

Aircraft Maintenance Technician (AMT) was performing a 3-Service Check on an A320 aircraft and while performing the overwater check per maintenance computer, found both the X colored and Y colored lanyards not installed correctly at Door 2-L. Reference Maintenance Manual (MM) 25-62-44 page 486, paragraph two and Figure 410: 1st) the X colored lanyard was routed through the trigger and 2nd), the Y colored lanyard was not routed through the trigger.

Narrative: 2

ZZZ Maintenance found survival kit strap and lanyard misrouted while completing the overwater portion of a service check. Inspection of the routing is to be completed referencing Aircraft Maintenance Manual (AMM) 25-62-44 page 401, as specified by the service check.

Callback: 2

Reporter stated there is confusion at times about which colored lanyard "X" or "Y," is supposed to have the end loop attached to the girt bar trigger for release of the slide/raft once the raft deploys. Because the slide/raft contracted vendor does not replace the old, dirty, worn out lanyards during overhaul, the lanyards (straps) become difficult to distinguish. Each lanyard has a loop and both loops are at the forward end of the girt bar. Only the "Y" loop should be attached to the girt bar trigger. The "X" loop should remain free and visible for the cabin crew to see during their pre-flight safety checks, because that is the loop the survival kit is attached to.

Reporter stated there are two survival kits: one in the forward cabin and one aft cabin, but all main doors are suppose to have the "X" colored lanyard loop visible for the overwater slides. The door bustle (cover over the slide/raft) only allows about an inch or less of the loop visible at the forward end and since the bustle also extends down to the cabin floor level, trying to see if the lanyard strap is properly secured by the little piece of velcro on the slide/raft Girt bar skirt is very difficult. Visibility and trying to determine the color of the loops of the two different lanyards and the security of the straps at the velcro strip is worse at the cabin doors where the food service caterers load and unload an Airbus.

Narrative: 3

I have been notified that an A320 aircraft Door 2-L slide/raft was found with the girt bar release strap and or the survival kit lanyard misrouted. I installed that slide in January 2011. I am very familiar with this slide installation and also viewed [company] training video prior to that day's installation. 

This video clearly illustrates routing of these components. The Maintenance Manual (MM) is also quite clear, although the illustrations could be better. The training posters listed are also helpful. I am quite sure that all was routed correctly after installation. My Inspector Mr. X, my Supervisor Mr. Y and Maintenance Trainer Mr. Z, all checked my routing, girt tie and mooring line prior to closing the girt flap. I can only think that something occurred in the weeks following my installation and reporting of this defect. Please note also that this aircraft had at least two number 3-service checks accomplished in this time. This check clearly states to inspect the lanyards.

Callback: 3

Reporter stated most lanyards (straps) on their slide/raft assemblies are old and rotten. The two different colored lanyards become bleached out over time, but are not replaced by the slide vendor during the slide overhaul. There is only a small piece of velcro stitched to the skirt, which eventually wears away and loses the ability to hold the straps to the girt bar skirt. He suspects that's what happened with the straps on the slide/raft assembly they had changed and between the time they had installed the replacement slide/raft and the 2nd service check was accomplished, someone had tried to re-secure the straps with the velcro but misrouted the "X" colored lanyard with the "Y" colored lanyard to the girt bar release trigger fitting at the forward end of the girt bar.

Synopsis

A Line Inspector and a Line Mechanic report about their involvement with two different colored lanyards found not positioned correctly on a slide/raft assembly they had installed on an A320 at cabin door-2L. A Down-line Mechanic also reports he noticed the lanyard discrepancies for the girt bar release hook and the survival kit during an overwater maintenance check.

ACN: 931756 (2555 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : PFD
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8600
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1120
ASRS Report Number.Accession Number : 931756
Human Factors : Physiological - Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

At some point during preflight setup I noticed that my (First Officer) PFD was too bright at its dimmest setting. I turned it off and on and tried to dim it. It wouldn't get any dimmer and I knew it would be a safety hazard in flight, like a flashlight in my face. I conferred with the Captain and we called Maintenance and wrote it up. A mechanic came in and looked at it and said, incredibly, "is that a problem?" I said yes it's too bright. Another mechanic came and said we will be delayed or they could defer it. We said we could defer it because the weather was good enough. After about 10 minutes he came back and after several iterations of different solutions he replaced the PFD scope and ran the tests. It was still pretty bright but workable. The mechanic said these scopes come from the vendor that way and they have no way to dim them. He said it is a problem, because the first person to check the brightness on them once they come from the vendor is the pilot using it. These bright scopes are a hazard and it looks like the only way to fix the problem is to replace all those that are too bright and make the vendor adjust them correctly. Apparently, Quality Control at the vendor is very poor according to the mechanic. It might be wise to have someone in maintenance check the brightness when we get them from the vendor and send the bright ones right back, instead of taking delays on the ramp.

Synopsis

A320 First Officer discovers a PFD that is too bright on its dimmest setting. A replacement PFD is also too bright and the Mechanic explains that this is the way they are coming from the vendor.

ACN: 931751 (2556 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : PFD
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 8600
Experience.Flight Crew.Last 90 Days : 260
Experience.Flight Crew.Type : 1330
ASRS Report Number.Accession Number : 931751
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

During preflight setup in the cockpit, I noticed my Primary Flight Display (PFD) was a bit bright in the ramp area at it's dimmest setting. I knew from experience that airborne against the night sky it would be like a flashlight shining in my face the whole flight. I tried turning it off, dimming it, etc., to no avail. I called Maintenance and told them we were writing it up as too bright. A Mechanic came up and asked what was up, and agreed it was too bright, approximately 5-7 times brighter than the Captain's PFD at it's dimmest setting. He said we'd have to take a delay or refuse the airplane because he didn't think they had time to change it, but he would find out. 

In the mean time a Maintenance Supervisor came up and looked at it, and personally didn't think it was too bright based on his experience in cockpits and simulators. I advised him that of course it isn't too bright here in the ramp with all the bright lights, but at altitude against the night sky it was unsafe. He also said we'd take a big delay and he was swamped with broken airplanes that night. The Captain and I decided that they needed to fix it, and I will add that the Captain deferred the decision to me if I could fly with it like that. After some discussion, the mechanics agreed that they had a part to replace it (PFD scope) and they would, but it was going to be a big delay. They replaced the PFD, which had correct brightness qualities and tested it. In the discussion, this Mechanic said they come from the vendor that way and the only way to get them fixed was to replace the bright ones and send them back to the vendor to get adjusted correctly. 

This bright PFD is an ongoing problem on the Airbus. I have been on this airplane for about 20 months and have had to fly too many times with bright PFDs and decided it is unsafe to do so. I wish someone, somewhere, with some pull, would have Maintenance, or Pilot Training check out the PFDs in the aircraft, before a delay is taken. Also, the mechanics have told me that the first person to see if the PFD works after it comes out of the parts warehouse, after shipped from the vendor, is the pilot. No one else is quality checking these PFDs before they are put in the airplane. 

Mechanics should block out all the windows on the ground and dim the cockpit lights to a night setting to see if all the scopes (displays) are at the correct intensity. This would save delays when a pilot encounters a scope that is too bright. Many pilots, both captains and first officers complain about the intensity of these scopes as too dim in the day time and too bright at night. Recently a company pilot, who jumpseated on another air carrier, noticed their scopes were great, intensity-wise and asked about them. The Captain of that air carrier informed him that they are not being adjusted correctly and you have to insist on it from the vendor, or you keep getting junk. [The] mechanics of this air carrier have figured out how to keep the bright or dim ones out of the cockpits and send them back to the vendor.

Callback: 1

Reporter stated he has not seen any effort by his air carrier to address the continuing problem of the A320 and A319s PFDs being too dim during the day or too bright at night. The issue is a safety problem and he feels like he is constantly being put in a position to have to "refuse" an aircraft before Maintenance will address the issue. He has not seen any consistency in the way Maintenance determines whether the light intensity is within limits.

Synopsis

A First Officer reports about an ongoing problem with cockpit Primary Flight Displays (PFDs) on A319 and A320s being too bright for night flights, or too dim for daylight flights. First Officer notes the issue is a safety problem and he feels pilots are constantly being put in a position to have to "refuse" an aircraft.

ACN: 931355 (2557 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 28000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 931355

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 931356

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We received an HYD B LO LVL ECAM message--intermittent at first but then came on steady. Followed AOM procedure, including shutting off Blue System pump which resulted in the need to perform a landing distance calculation for loss of Blue System spoilers. Also had slower slat extension. We declared an emergency with Center because of flight control involvement. No assistance was needed. Notified Dispatch and Maintenance Control, briefed the flight attendants and landed uneventfully.

Synopsis

An A320 flight crew declared an emergency and landed uneventfully after suffering loss of the Blue Hydraulic System.

ACN: 931051 (2558 of 2908)

Time / Day

Date : 201102
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : MMEX.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 33000

Environment

Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : MMEX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Cruise
Route In Use.STAR : KOBEK 1B

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 931051
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 931059
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were at FL330 approaching Mexico City and the QET VOR expecting the Kobek 1B arrival. We were in contact with Mexico Center and were told to maintain FL330 and contact Mexico Approach. We checked in with Mexico Approach at FL330 and were told by approach to maintain FL330 and expect the Kobek 1B arrival. After a few minutes the Captain and I began to feel like we were too high for our proximity to the airport and position on the approach. We were now inside QET and outside of Kobek. We were about to query Mexico Approach about our altitude when Mexico Approach called us and asked us what our altitude was. We told them FL330 assigned. The Controller responded by saying, "Negative. You should be descending to 13,000 FT. Turn to heading 090, descend to 13,000 and intercept the SLM VOR 360 radial inbound to SLM." We followed these new instructions, but asked ATC to verify if we had missed a previous clearance to 13,000. It took 3 attempts to finally understand what the Controller was trying to say which was that he had cleared us to descend via the Kobek 1B arrival when we first checked in. Neither the Captain nor I recall receiving this clearance. We intercepted the SLM VOR 360 radial as instructed and resumed the arrival and ILS DME 2 for Runway 23L and landed without incident. In addition to the communication barrier, the Captain and I were dealing with possible windshear and moderate turbulence reports on the arrival and approach into Mexico City. We are both aware of the challenges of flying into MMMX and had fully briefed many things to expect. As I write this report it is my recollection that no descent to 13,000 was given. I will take from this experience the need to be even more cognizant of potential communication miscues especially as we get closer to the airport and arrival fixes and on initial check-in with approach.

Narrative: 2

It was a very high workload environment due to language barriers, turbulence, and Runway 23 in use.

Synopsis

An A320 on approach MMMX was told to expect the KOBEL 1B arrival but was never cleared to descend from FL330 until after QET. A descent clearance was issued off track when the Controller realized what happened.

ACN: 930817 (2559 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : APU Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 175
Experience.Flight Crew.Type : 8600
ASRS Report Number.Accession Number : 930817
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Fatigue
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21500
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 932041
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Evacuated
Result.General : Declared Emergency

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

We had a false APU fire warning after APU shutdown. We had already completed the parking checklist, and I was up out of my seat greeting departing passengers. I heard the warning buzzer in the nose wheel well sound, and turned to see a red fire light illuminated in the APU emergency shutoff switch. The APU was already shut down, and the Captain had selected ground power. I was surprised that there was no repetitive warning bell for fire, but in the press of events I thought it was due to the plane being on battery power (which it was not). The ECAM screens were already off for the night. The Captain activated the emergency shutoff switch, and I advocated discharging the fire bottle, since the light was still illuminated. He called ground and I called station operations to summon the fire trucks. I went to the cockpit door and determined that most of the passengers had deplaned or were forward and on their way out. With the Captain's concurrence, I ran outside to get the baggage handlers out of the rear cargo pit and away from the airplane while we awaited the fire trucks. I saw no evidence of a fire, and initial analysis by the mechanics pointed to a failed warning relay that had triggered a false fire warning. I was initially confused by the indications because the ECAM screens were secured, but if they had been on I'm not sure I'd have done anything differently. Even if I suspected a false warning, I think the prudent thing to do would be to honor it and discharge the bottle. I was also somewhat tired at this later hour in my domicile time, and didn't immediately realize that we were not getting all the correct indications of an APU fire appropriate to our current electrical configuration. I am familiar with the Airbus-specific issue of false cargo pit fire ECAMS on the ground with cargo doors open, and of course would not have responded to that situation with a fire bottle discharge.

Narrative: 2

False Fire Warning: After being towed in to gate per usual procedure there, we completed parking SOP's including shutting down APU. Less than a minute after the APU appeared to shut down normally and while waiting for the flap to close before shutting off the aircraft batteries, we got an APU fire warning light and squib light plus the outside APU fire horn. The ECAM screens were already shut down (won't do that again). Followed SOP and pressed fire switch and also discharged extinguisher bottle, which had not discharged by itself. The red APU fire light did not extinguish. Called fire department. First Officer went outside to clear ground personnel from the nearby area and to check for evidence of fire. None found. During examination of the Centralized Fault Display System by maintenance, a relay in the APU fire warning system was noted as faulting. Talked to Dispatch and Flight Duty Manager and CFR and police responders and station personnel. At time of fire light, the cabin was empty of all but the flight attendants who were on their way out and maybe a couple of passengers preceding them. Secured airplane.

Synopsis

An A320 had an APU fire warning while parked at the gate with the APU OFF and the aft cargo door open so fire fighting procedures were followed but fire fighters found no fire.

ACN: 930501 (2560 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 300
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 930501
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During our cruise portion of the flight the aft Flight Attendant called to tell us she smelled a burning plastic smell and the aft electrical panel in the galley next to the coffee makers was hot. I told her to turn off all electrical equipment in the galley and let me know if the smell went away. She called back up and informed me that the coffee maker Circuit Breaker (CB) had popped and the smell was going away.

I called Dispatch for a phone patch to Maintenance Control to let them know what was going on. We discussed the possibility of a divert if the situation changed at all for the worse. The Flight Attendant monitored the area for about 5 minutes and then called back and said that the smell was coming back and the panel was again getting very hot. Shortly after that she told me that a T/R CB had just popped also. At that time we felt that it would be prudent to get the aircraft on the ground to get it inspected for possible damage to the electrical system. 

We told Dispatch and ATC we were going to divert to ZZZ as a precaution. The galley never had any smoke and we never had any unusual ECAMs in the cockpit. We landed and taxied to the gate.

Synopsis

An A320 diverted to a nearby airport when attempts to overcome heat and a burning odor from an aft coffee maker proved unsuccessful.

ACN: 930317 (2561 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Aircraft Reference : X
Problem : Failed
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 65
Experience.Flight Crew.Type : 500
ASRS Report Number.Accession Number : 930317
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 930322

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During preflight I noticed circuit breakers AA 7 and 8, "EXT PWR EGIU 1" and "APU GEN EGIU 2" popped open without collars. Since the aircraft had been written up for APU problems I told the Captain we should talk to the station operations to see if the mechanics who worked the previous issue had forgotten to collar them as part of the previous troubleshooting, or had simply left them pulled unintentionally. When the Mechanic arrived, he noted the circuit breaker for "APU GEN EGIU 2" (AA 9) was physically broken.

He called Maintenance Control to advise the circuit breaker must be repaired prior to flight. After the replacement circuit breaker arrived the power was removed from the aircraft by selecting the "EXT PWR SW" off and de-selecting the aircraft batteries. The ground power unit was not disconnected and the Mechanic began opening the circuit breaker panel and removing the bad circuit breaker. Immediately the circuit breaker panel arced loudly as an electrical short occurred and smoke filled the cockpit. It was not a small amount; it was smelled all the way in the first class cabin. Visibly upset about the error, the Mechanic immediately physically unplugged the ground power.

My primary concern follows the arching and smoke. While talking to Maintenance Control, the Controller asked the Contract Mechanic to take cell phone pictures of the damage so they could determine if the aircraft was still "good to go." I voiced my concern to the Captain and the Mechanic that a full check and test of the entire electrical panel should be accomplished, not simply having Maintenance Control look at cell phone pictures. The amount of smoke generated by the arcing event indicated that much greater damage to the electrical panel could have occurred.

Based on the overheard conversations I was very concerned the aircraft would be returned to revenue service without a full inspection of the panel being accomplished. Unless the degree of the arcing event and resultant smoking was made clear it could simply be written off as a non event. I refused to fly the aircraft without a complete inspection. The flight was canceled and the aircraft parked on the service ramp.

Narrative: 2

Maintenance Control had a replacement circuit breaker shipped to us on the next flight.

Eventually the aircraft was pulled out of service and the flight was cancelled.

Synopsis

An A320 flight crew refused an aircraft for service when a maintenance operation resulted in significant electrical arcing and smoke from the involved circuit breaker panel.

ACN: 930004 (2562 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 3000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Airspace.Class B : ORD

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 930004

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

At flaps 2 and 180 KTS assigned with autopilot on, the aircraft pitched over to chase erroneous glideslope and accelerated past the 200 KTS limit for flaps 2 by 5 KTS before I could disengage the autopilot. There was no warning from previous aircraft on approach or from Controller. I called Tower upon landing and the problem had occurred previously but not to the extent our aircraft experienced.

I was told that the close proximity of a taxiway to the glideslope antennae with passage of a wide body could cause the problem and a B747 was maneuvering in described area at time of event.

Synopsis

An A320 captured a false and/or corrupt glideslope signal and descended at an excessive rate causing a flap overspeed. Tower advised a B747 was maneuvering in the vicinity of the glideslope transmitter at the time of the event.

ACN: 929995 (2563 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 929995
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11400
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 6300
ASRS Report Number.Accession Number : 929979
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff (First Officer flying) the landing gear failed to retract as a result of [Landing Gear Control Interface Unit] LGCIU 2 failure, at which point we cycled to gear to get a successful retraction. During the climb-out we then had multiple failures to include engine 2 reverser fault, anti-ice Captain AOA fault ECAMS, as well as indications on the First Officer PFD that the aircraft air/ground sensor believed the aircraft was still on the ground. At this point we continued to work the failures and coordinate for an emergency return to the departure airport. After declaring an emergency with ATC we returned without incident for a normal landing, the Captain now flying. After landing we then had a landing gear system disagree ECAM fault, and were able to taxi to the gate without incident.

Narrative: 2

On the ground, Maintenance found that the wires and connection to one of the [Landing Gear Control Interface Unit] LGCIU ground proximity sensors had been damaged.

Synopsis

An A320 had damage to wires and connectors of one LGCIU ground proximity sensor which presented numerous ECAM annunciations and a First Officer PFD symbol indicating that the aircraft was on the ground.

ACN: 929888 (2564 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 25000

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : GEELA4
Airspace.Class A : ZAB

Person

Reference : 1
Location Of Person.Facility : ZAB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 929888
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The A320 came over from ZLA on the GEELA4 arrival at FL250. I had a BE90 in front of the A320 that wasn't going to allow me to give a complete descend via clearance for the A320; PHX was on the 25L transition. We still, even when not descending via, have to issue the runway transition. Therefore, I gave the A320 the route clearance for the transition when they came over so that I wouldn't forget to issue it. I said "the A320, after GEELA cleared the Runway 25L transition, cross SCOLE at FL250." I was going to have to give GEELA at 150/250k. We are having discussions in my area about the runway transition clearance in isolation and reiterating the altitude assigned at the time in the same transition. We are doing this because we believe it might be, at tacitly, a .65 requirement and it should help reiterate the fact that the pilot is cleared for the transition only and not to descend via. We have had a lot of instances where the pilots construe this clearance as a descend via clearance. In this case, I told them to cross SCOLE at FL250, which is the previous clearance ZLA gave, rather than say "maintain FL250" and have the pilot possibly cross SCOLE high because of the difference in verbiage. The pilot read back my clearance as follows "the A320, cross SCOLE at 250, after GEELA cleared for the 25L transition." Soon after crossing the border, the A320 started down on their own. I asked the A320 if they were descending, they responded "affirmative, we are descending via the GEELA4 transition, GEELA4." I told them the transition clearance was a route clearance only, and then amended their presumed clearance to GEELA at 150/250. Recommendation, First, I think someone needs to discuss with the companies indicating that unless you get an actual descend via clearance, the transition clearance is not a VNAV clearance, only LNAV. We are having a lot of problems with the aircraft assuming the transition clearance is the descent clearance. Second, I think the RNAV Arrival work group needs to be convened to discuss this and other issues involving these procedural problems. Also, we are having some difficulty and the pilots as well, with phraseology, since these sorts of clearances are brand new in the NAS.

Synopsis

ZAB Controller provided a detailed discussion regarding the new RNAV arrival procedures into PHX and the confusing "descend via" phraseolgy.

ACN: 929769 (2565 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 20
ASRS Report Number.Accession Number : 929769
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Pushback crew had problems communicating with me so they decided to pushback using hand signals. The communication that I received from the person relaying signals to the tug driver was very poor. He gave me the brake release signal then sat in his tug and slowly followed the airplane while typing into his bag scanner device. He was about 50 yards away from the airplane most of the time. He was not available for me to communicate with him and I had no way of communicating with the tug driver. After the plane reached the spot to stop, he then drove up in his tug and gave me the brake set signal. He then gave me the engine start signal. In my 21 year career including 13 years as Captain, this was the most unsafe pushback I have ever encountered.

Synopsis

A320 Captain describes a pushback without headsets in which the ground crewman in charge remains in his vehicle and does not monitor the progress of the pushback until it is time to set brakes and start engines.

ACN: 929622 (2566 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 12000

Aircraft

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18800
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 929622

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 929625
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

The clearance was to descend to 13000 and expect 12000 at PRISE. We leveled at 13000. There was a controller change, and the next clearance was to cross PRISE at 12000, just as we expected. The clearance came close to WASTN and the aircraft automation was slow to push the nose over. I was unable to get relief from the clearance due to congestion on the frequency. We crossed the fix 600 feet high.

Narrative: 2

During decent on the WATSN arrival into ORD we were given a decent to 13000 and advised to expect to cross PRISE intersection at 12000. 13000 was set in the MCP altitude window by the captain (PM) while I (PF) started the aircraft down with FMA's of THR IDLE, OPN DES and the selected speed set at our assigned speed of 310 KIAS. Everything appeared normal and the aircraft was on profile to make the expected crossing at 12000. Shortly after beginning decent our clearance was amended to cross PRISE at 12000 as expected. The MCP altitude was reset and verified. The FMA's remained unchanged and the aircraft was still on profile. Nearing the fix the captain and I were reviewing the final details of the planned approach from our earlier briefing. When my full attention went back to the aircraft status, the airspeed was 15 KTS below the selected speed with the aircraft pitching up with the FMA's still reading THR IDLE and OPN DES but still well above the MCP altitude of 12000. I reached up and spun the selected airspeed higher approximately 15 KTS (~325KIAS) in an attempt to force the auto flight to pitch down to re-capture the selected speed. The aircraft was slow to respond and we subsequently crossed the fix at approximately 12,900 MSL. 

Frequency congestion and a controller change made it impossible to advise the controller and obtain an amended clearance. In hindsight I should have just disengaged the auto flight, disregarded the flight director guidance and continued the decent manually. I've flown this aircraft for seven years and have seen it attempt to fly smoothly, while completely disregarding the programmed crossing restrictions while in a managed decent. I've also seen the aircraft enter the ALT capture mode early and attempt to miss the crossing restriction. This is the first time I've seen the auto flight be so far off in the OPN DES mode and actually pitch the wrong direction needed to regain the selected parameters.

Synopsis

A320 flight crew missed a crossing restriction during descent on the WATSN 1 RNAV arrival to ORD due to the vagaries of the A320 auto flight system.

ACN: 929566 (2567 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : PHX

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : PHX

Person : 1

Reference : 1
Location Of Person.Facility : P50.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Trainee
Qualification.Air Traffic Control : Developmental
ASRS Report Number.Accession Number : 929566
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Facility : P50.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Instructor
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 929567
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Separated Traffic
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

I was training on Finals on the Freeway sector. I took the radar handoff on Air Carrier X who was descending via the EAGUL4 arrival Runway 26 transition. When I took the radar hand off on Air Carrier Y, I saw that they were about 4 miles in-trail. Air Carrier Y was also descending via the EAGUL4 arrival. I watched to make sure that Air Carrier Y was slowing to a similar speed as Air Carrier X and noticed that Air Carrier Y was indeed slowing down. I cleared Air Carrier X for a visual approach and noticed that Air Carrier Y was still showing a faster speed than Air Carrier X (I assumed that the speed difference was from the altitude difference and that the airplane was slowing more than it actually was). After I cleared Air Carrier Y for a visual approach I went back to slow him down further. The speed still was in transition and I felt it was not going to be enough to hold minimal separation. (A square turn to final approach course was not an option since there was an aircraft on opposite base and I felt Air Carrier Y would overshoot the localizer). Then I gave Air Carrier Y a ten degree turn to push his turn on final further behind Air Carrier X. My OJTI asked Air Carrier Y if he had the Air Carrier X in-sight and Air Carrier Y acknowledged that he did. We used visual separation to clear Air Carrier Y and still reduced his speed to 150 KTS since Air Carrier Y's speed was still showing faster. Then I shipped Air Carrier Y to the Tower. Air Carrier Y thereafter became a go-around and subsequent investigation. I was notified that we had lost minimal separation outside 10 miles of PHX. I am unsure exactly what point the two aircraft lost separation, if it was before or after I tried to maintain the minimal separation.

The EAGUL4 STAR is very fast and the airplanes are very high when they come to the Finals sector. I would recommend that the aircraft be taken off the STAR for more stabilized altitude and speeds.

Narrative: 2

I was conducting OJT on the north final position. It was early in the session when this event occurred and it was the trainee's first day on this airspace configuration since the arrival and procedural changes at P50. Air Carrier X was descending via the EAGUL 4 arrival, Runway 26 transition. Air Carrier Y was closely behind Air Carrier X also descending via the EAGUL 4 arrival, Runway 26 transition. Air Carrier X reported the airport in sight. The trainee issued a heading to join final and cleared the aircraft for a visual approach to Runway 26. Shortly after, Air Carrier Y reported the airport in sight. The trainee issued a heading to join final and cleared the aircraft for the visual to Runway 26. 

Sometime during these clearances the trainee questioned the Arrival Controller on the spacing she was receiving. She was not comfortable there would be enough room for all aircraft she was handed off. I discussed this briefly with her and gave her a couple of tips to make it work. I also sensed she was uncomfortable with the new RNAV routes the aircraft were flying. We again discussed this briefly during the session. She was willing to continue working with the RNAV arrival routes.

Between working the remaining aircraft and our discussion of the RNAV routes I noticed Air Carrier Y looked a little too close to Air Carrier X on final. I used the override and issued traffic to Air Carrier Y on Air Carrier X in front of him. Air Carrier Y reported traffic in sight. I instructed Air Carrier Y to follow Air Carrier X. Both aircraft were executing the visual approach to Runway 26. The trainee issued a 150 KTS speed restriction to Air Carrier Y and switched the aircraft to the Tower.

Air Carrier Y executed a go-around. When that happened the FLM started a QAR investigation. It was though this investigation where it appeared a loss of separation may have occurred between these two aircraft. As I write this report the final outcome of the investigation was still pending.

After Air Carrier Y went around and my further sense that the trainee was uncomfortable with the RNAV routes, I instructed the trainee to issue a heading and altitude assignment to all future inbound aircraft. This seemed to help her a great deal. I sensed and she even admitted later that she felt more in control. We continued training with no further issues.

The new arrival routes and procedures have been a challenge to all of us at P50. My trainee is no exception to these challenges. One of my beefs with the new RNAV arrivals is, in my opinion, that there are several built in traps. There used to be safe guards built into our arrivals and facility procedures that would procedurally keep airplanes separated if something went wrong. Now those redundancies don't seem to exist and if I was to do nothing as a controller, airplanes can easily come together. Several examples of this can be seen with the EAGUL 4 and MAIER 4 arrivals, Runway 26 transitions. The EAGUL 4 arrival descends all the way to the runway at and angle towards the airport. This descent runs though the arrival routing of the MAIER 4 arrival. This seems dangerous to me.

Another example of this is the EAGUL 4 routing itself. We used to be required on the feeder positions to vector aircraft on a base leg, perpendicular to the final. This created an angle that could be used to help separate and enhance spacing of aircraft. Since this no longer exists the feeder can leave aircraft on the arrival and it is up to the final controller to execute all vectors for spacing. Again, this seems dangerous and wrong to me.

Though I feel we could have handled our event better I feel these new procedures had an impact on this situation and should definitely be looked at. It is my opinion that the feeder positions should be vectoring aircraft off the EAGUL RNAV arrival and assigned a heading perpendicular to the final in this airspace configuration. Further, the aircraft on this base leg should be capped at 8,000 FT which is 1,000 FT above the aircraft on the MAIER 4 arrival.

Synopsis

Training Controller and his student report a loss of separation incident resulting in a go around during visual approach to Runway 26 at PHX. Both controllers lament the loss of control that resulted when the EAGUL 4 RNAV procedure was instituted.

ACN: 929460 (2568 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 929460
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise flight we received an ECAM message and felt generator #1 disconnect (drop offline). Auto pilot and auto thrust disengaged. We received a subsequent ECAM message for AC BUS 1 FAULT. I was the flying pilot and after auto pilot was re engaged and navigation was checked, I transferred control of the aircraft to the pilot not flying and completed the ECAM, followed up with the QRH. There is not a specific checklist for AC BUS 1 FAULT so I consulted the handbook and we discussed our plan with the faults that we had. Contacted Dispatch with the radio since ACARS was inoperative. We lost control of the cabin temperature and one point had -20 degree air coming into the aircraft. Made the decision to descend and divert to the nearest suitable airport. Informed Dispatch but never talked to Maintenance Control. Lost radio contact once we started our decent. I declared and emergency with ATC, then informed the Flight Attendants. [I] informed them to get the cabin ready for a precautionary landing. Noise Wheel Steering was one of the faults we had so I informed them that we would have to be towed off the runway. Instructed them to make an initial cabin announcement about our diversion and that I would brief the passengers when I had time. First Officer made a excellent normal approach and landing. We stopped on the runway. Shut down the engines and at that point, the emergency crew checked out the aircraft. They gave us the OK to be towed to the gate.

Callback: 1

ECAM procedures directed that the number 1 generator be reset and if unsuccessful that it be turned off, which was accomplished. Examination of the ECAM electrical page showed that the ESS AC bus had automatically transfered to AC Bus 2. AC Bus 1 remained unpowered and the APU was started with the hope that its' generator would pick up AC Bus 1, but that too was unsuccessful. The mechanic who was assigned to fix the aircraft at the divert airport was familiar with the issue and the aircraft was ferried without any further electrical problems.

Synopsis

A320 Captain experiences an AC BUS 1 FAULT at FL340 which cannot be corrected by QRH or ECAM procedures. The crew elects to divert to the nearest suitable airport. Nose wheel steering was among the faults annunciated and the aircraft had to be towed to the gate after landing.

ACN: 929429 (2569 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 20000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Gear Up Lock
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9100
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1035
ASRS Report Number.Accession Number : 929429
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 2780
ASRS Report Number.Accession Number : 929440
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through FL200, speed 290 IAS, First Officer was pilot flying with auto pilot on. ECAM LG (Landing Gear) not uplocked occurred. We also felt the nose gear strut settle onto the gear doors. We discussed fuel burn (4,000 LBS/HR) and determined we could continue to destination at this cruise state and arrive with approximately 5,000 LBS. Discussed possible "nose gear cocked" scenario and reviewed landing technique and flare for nose gear problems. Decided to extend gear on base leg to final to assess gear situation. First Officer flew visual approach and landing. Safe gear indication, safe landing.

Narrative: 2

Climbing through approximately FL200 we heard and felt a thump from beneath our legs and then saw on the center instrument panel a red UNLK light illuminate. A few moments later we had an ECAM and overspeed warning horn sound at which time ECAM instructed us to slow below 220 KTS. At the time we were doing about 280 KTS in the climb. We slowed and leveled of at FL250 after requesting with ATC. ATC asked us if we needed assistance at which time we were still trying to figure out our situation. We looked at the resulting status page and saw that we had "LG Gear Not Uplocked" displayed. The Flight Manual told us to recycle the gear. We chose not to since we were concerned that if we brought the gear back up we may make things worse and possibly not be able to extend the gear when needed. We also knew that if we extended the gear at that time we wouldn't have enough fuel to make our destination. The weather was excellent all the way to our destination and we had many airports along our path so we new at anytime we could land quickly if needed. Fortunately the Captain had added fuel for our trip and after discussing our situation with Dispatch and Maintenance it was agreed that it was safe to continue to our destination. The Flight Attendants were advised of the situation and given a cabin advisory. Dispatch informed us that Tower and emergency services would be advised of our situation. Also we explained our situation to enroute ATC and told them that we did not need any assistance and requested a speed of 220 KTS to destination. After that it was a slow quiet ride. On base we lowered the gear and with out any incident proceeded to land. After landing we taxied off the active runway and the Fire Department inspected the nose gear and gave us the go ahead to continue to the gate.

Synopsis

A320 flight crew experiences a nose gear uplock failure climbing through FL200. Nose gear doors remain closed but aircraft is slowed to 220 KTS to comply with overspeed warning and ECAM logic. After consultation with Maintenance the crew elects to continue to destination at reduced speed. Nose gear extends normally.

ACN: 929424 (2570 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 929424
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 2400
ASRS Report Number.Accession Number : 929445
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During our cruise portion of the flight about an hour after [departure], the aft Flight Attendant called the Purser to tell her that she smelled a burning plastic smell and the aft electrical panel in the galley next to the coffee makers was very hot. After contacting me I directed her to turn off all electric equipment in the galley and let me know if the smell went away. She called back and informed me that the #2 coffee maker circuit breaker had popped out and the smell had subsided. I called the Dispatcher and Maintenance to let them know what was going on. We discussed the possibility of a divert if the situation changed at all for the worse. The Flight Attendant monitored the area for about 5 minutes before calling me again to inform me that now the smell had returned, the panel was getting hot again and the T/R circuit breaker had just tripped out also. After a few more minutes she said the smell was going away and the temperature was cooling down. At that time we felt that it would be prudent to get the aircraft on the ground to get the electric system checked out before going any further. We were about 150 miles west of [a suitable airport] when we decided to divert as a precaution. We informed Dispatch and ATC. We had a Flight Attendant remain in the galley to inform us of any changes. There was no smoke or flames or any unusual ECAMs or indications in the cockpit that we noticed. We landed about 15-20 minutes after the decision was made to divert.

Synopsis

An A320 Flight Attendant notified the pilots of a burning plastic smell in the aft galley which dissipated when the coffee makers were turned off. Later a T/R circuit breaker popped with more electrical smell so the crew diverted to an enroute airport.

ACN: 929402 (2571 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZKC.ARTCC
State Reference : KS
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class A : ZKC

Person

Reference : 1
Location Of Person.Facility : ZKC.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 929402
Human Factors : Situational Awareness

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 checked on frequency at FL360 requesting ride reports as he had been in continuous light chop for the last 50 miles. After checking with the next sector west of mine, and the sector above for reports at FL380, the A320 requested FL340. I entered the altitude in the computer, trial planned it on URET, pointed out the aircraft to ZMP sector P39 whose airspace the aircraft was above and coordinated descending to FL340. I issued the clearance and after getting a smooth report out of FL355, I relayed this to the ZMP sector working FL350-FL380. Only after the A320 had leveled at FL340 did it occur to me that I did not coordinate with the sector working FL350-FL380 for the portion of the descent between FL360 and FL350. No further incident. [Contributing to the problem is the fact] 4 of the 5 surrounding sectors have different sector altitude splits.

Synopsis

ZKC Controller described an airspace incursion event when descending an aircraft due to light chop, noting airspace altitude layering at adjoining sectors contributed to this event.

ACN: 929334 (2572 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Ice Detection System
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Fuselage
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 929334
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

When our air carrier paint job was accomplished by [contract] vendor, the Ice Detector was not taped over and was inadvertently painted, rendering it inoperative.

Callback: 1

Reporter stated the Ice Detector on the A320 is basically a metal peg chamfered bolt, approximately two and a half inches in height, that is installed in one of the bolt holes for the Captain's forward windshield at the lower inboard position near the aircraft's centerline. The Ice Detector surface has a brush type metal finish. Early model A320s had an unlit detector, while later model A320s had a lighted detector. His air carrier uses both types of detectors, but stocks only the lighted ones. 

Reporter stated the Ice Detector he found was the unlighted type and painted with a glossy white finish, making ice detection almost impossible. He believes the aircraft had been out of the Contract Vendor's paint shop for two or three weeks. Their aircraft configuration did not allow for the use of a lighted Detector as a replacement for the unlit type. And since his carrier did not stock the unlit type of Detector, they took a lengthy delay before they could find a replacement for the painted one.

Reporter also stated the Ice Detector does not send any signal for an ECAM message of icing conditions; the Detector is only a visual indicator.

Synopsis

An air carrier Line Mechanic reports finding an Ice Detector painted over with a glossy white finish on one of their A320 aircraft, rendering the Ice Detector inoperative. The A320 had been flying for approximately three weeks since being released from a Contract Vendor paint shop.

ACN: 929235 (2573 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Reverser
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 929235
Human Factors : Situational Awareness
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 8700
ASRS Report Number.Accession Number : 929237
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff on Runway 33 had an ECAM alert THRUST REVERSE NOT IN AGREEMENT/FLT CONTROLS. Did a normal abort. High speed abort 100- 110 KTS. Used approximately 2,000 FT. Exited runway called Ground, Maintenance, Dispatch. Returned to the gate.

Narrative: 2

High energy stop and we had hot brakes.

Synopsis

An A320 ECAM alerted THRUST REVERSE NOT IN AGREEMENT/FLT CONTROLS at about 100-110 KTS on takeoff, the takeoff was rejected.

ACN: 929225 (2574 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SAN.Airport
State Reference : CA
Altitude.MSL.Single Value : 6000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use.STAR : LYNDI2
Airspace.Class B : SAN

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 14098
Experience.Flight Crew.Last 90 Days : 215
Experience.Flight Crew.Type : 7489
ASRS Report Number.Accession Number : 929225
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 929433

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were on the LYNDI2 arrival into SAN and were cleared to descend and cross TRIXI at 12,000 FT. While in the descent we were given a speed restriction of 280 KTS then 270 KTS, which we complied with. The Controller appeared to forget about us and by BARET Intersection we were still at 12,000 FT and had to request a lower altitude. The Controller was obviously overloaded and with the great visibility that we had we called the field in sight so as to be cleared for the visual approach. The Controller gave us a clearance direct to VYDDA, descend to 6,000 FT and cleared for the visual approach to Runway 27. Prior to reaching VYDDA, the Controller canceled our clearance and gave us a turn to 180 degrees. [The Controller] then gave us a turn to 240 degrees while still level at 6,000 FT south of OKAIN. While in the turn to 240 degrees, another Controller came on and gave us a further turn to 350 degrees with the following exchange: Controller: Air Carrier X turn right to 350 degrees. Air Carrier X: We are currently in the turn to 350 degrees. Controller: Air Carrier X I need you to turn right immediately to 350 deg. Air Carrier X: We are in the turn to 350 degrees. Controller: Air Carrier X you have parachute jumping activity at 12 o'clock, ? mile at 10,500 FT. Air Carrier X: We are looking for the traffic. We never saw the traffic that was over 4,000 FT above us and no TCAS warning was received. The Controller did not say anything else after that other than to once again clear us for the visual approach. No explanation, no apology, no nothing. The Controller seemed upset that we were in the airspace that another Controller had vectored us into. I made a comment on the radio that this was the worst controlling I had ever seen and received no response whatsoever. This was obviously a close call with traffic that we knew nothing about. No warning about parachute jumping in the vicinity of our location, no traffic call out other than as a last minute evasive maneuver, no explanation, no nothing. The impression we got was that Controller training was in progress and the instructor took over at the last minute. The airspace we entered was exactly where the Controller vectored us into and the close call was certainly not of our doing.

Synopsis

Air Carrier operating with SCT, reportedly was vectored into a potential conflict with a parachute operation with no prior notification, the reporter suspecting controller training was in progress.

ACN: 928891 (2575 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Light : Dawn
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 8500
ASRS Report Number.Accession Number : 928891

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Autopilot [and] auto-throttles kicked off four times with subsequent loss of navigation maps (we still had VOR/DME/heading capability). Each time we were able to restore all units for a limited time (only minutes between failures). After the fourth time, [the] Captain called Center, explained that we had lost our navigation capability and wanted vectors to [a nearby airport] (we were only approximately 45 minutes out). Did not declare [an] emergency. [Aircraft was] hand flown [to an] uneventful landing.

Synopsis

An A320 Captain reported losing FMC navigation capability, autopilot and autothrottles. He elected to divert to a nearby airport.

ACN: 928604 (2576 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : PHX.Airport
State Reference : AZ
Altitude.MSL.Single Value : 12000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class E : ZAB

Person

Reference : 1
Location Of Person.Facility : ZAB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 928604
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

It was a busy arrival push into PHX this morning. With the new GEELA 4 RNAV arrival there is quite a bit more transmissions that need to be made to ensure the pilot will do what we need him to do. If the pilot is to descend via the arrival, well that's a straight forward clearance. When we have to vector the aircraft for sequencing the phraseology to put the aircraft back on the arrival is very confusing. An A320 was issued a clearance to cross GEELA at and maintain 12,000 FT and 250 KTS. The pilot read this back correctly. The pilot was then issued "Cleared the GEELA 4 arrival, Runway 7R transition". The pilot read back "Cleared for the GEELA 4 arrival, 7R transition." At no point was a descend via clearance given or read back. The aircraft then entered PHX Approach and said he was descending "VIA" the arrival. PHX Approach then called us and let us know. 

We need to come up with some type of phraseology that will allow us to clear the aircraft for the arrival and transition without the pilot thinking he is cleared to "DESCEND VIA" the arrival. It seems the pilots are associating the runway transition with a descend via clearance. Maybe give the runway transition on initial check on if that is legal.
.

Synopsis

ZAB Controller described a confused clearance event involving the GEELA 4 RNAV STAR and the absence of clear phraseology that captures route information but not any "descend via" inference.

ACN: 928261 (2577 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 928261
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21260
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 928264
Human Factors : Situational Awareness

Person : 3

Reference : 3
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 928348
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : FOD
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Captain on preflight walk around found a wrench lodged between the rotors and stators in engine number one. He alerted me [First Officer] and maintenance the wrench was pinned between two door panels. Maintenance opened the engine doors and removed wrench.

Narrative: 2

On walk-around, Captain found a 10-inch wrench wedged inside the #1 engine, aft of the fan section. Maintenance Control notified. Line Maintenance [Mechanics] came to aircraft, opened all engine nacelles [cowling], and removed wrench. No damage noted other than a 3/4-inch long x 1/8-inch deep impression of the handle butt of said wrench where it was wedged into the nacelle door. After Line Maintenance inspection, all doors closed and we departed 48 minutes late.

Narrative: 3

Upon arrival at work for night shift, I opened my tool box and realized I misplaced a Ford wrench and realizing I used it the night before on the #1 engine of an A320 aircraft for an A-Check to change an IDG oil filter. I realized it might have been left in the bypass area of the engine where I sometimes place it in the process of changing the filter. I was multi-tasking at the moment because I had another repair with epoxy that needed to dry so I started it early while doing these other checks on the engine. During close-up I obviously didn't do a good enough check of the bypass area. Upon checking the history on this aircraft, I was relieved that no significant damage occurred and realized how it could have been much worse for the passengers, the airplane and for me.

Synopsis

Two pilots and a Mechanic report a 10-inch Ford wrench was found wedged inside the #1 engine, aft of the fan section on an A320 aircraft.

ACN: 928190 (2578 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Relative Position.Angle.Radial : 090
Relative Position.Distance.Nautical Miles : 80
Altitude.MSL.Single Value : 29000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning and Pressurization Pack
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Experience.Flight Crew.Total : 4000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 200
ASRS Report Number.Accession Number : 928190
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Dispatched with pack 1 INOP. Climbing through FL290 notice cabin alt was climbing through 9k at a rate of approx 1k/min. Coordinate an immediate descent with ATC to 15K (we were over the mountains, scattered deck below). As we were descending, cabin continued to climb. With cabin alt passing approx 9.7k, capt decided to turn pack 1 ON. That worked and the cabin stopped climbing and started descending. We leveled at 25k and cabin alt stabilized in normal range. Capt called Dispatch/Maintenance and coordinated Return to base. We declared an emergency and Returned to Base at 25K with both packs ON. Return to base was uneventful.

Synopsis

A320 Captain is dispatched with one pack on MEL and discovers the second pack is not able to maintain pressurization passing through FL290. At 15000 feet over mountainous terrain the cabin is still climbing and the Captain elects to turn on the deferred pack. This act is successful but the crew elects to return to departure airport after declaring an emergency.

ACN: 928092 (2579 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Wiring & Connectors
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 500
ASRS Report Number.Accession Number : 928092
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We diverted one hour into the fight with multiple failures. At cruise at FL340 we received numerous ECAM's in quick succession, in addition to a hydraulic blue system low pressure ECAM. With the First Officer flying I declared an emergency with ATC and told them we were having electrical issues with the aircraft. The First Officer continued to fly and he worked the radio as well, so I could run all the ECAM's/checklists/work with the cabin crew/Dispatch, etc. We were close to ZZZ (we were both familiar with the airport, we were VFR and could see the ZZZ area) the First Officer and I decided to divert to ZZZ based on the numerous failures that we currently had and additional failures that we were receiving, to include the Blue System Hydraulic failure. I sent an ACARS message to Dispatch to say we declared an emergency, we were having electrical and hydraulic issues and we were diverting to ZZZ. I then continued to run the ECAM's. While running the presented ECAM's, both my screens (Captain side) failed several times. The screens went blank for a few seconds, both came back and followed this pattern several times. In addition, the upper ECAM E/W screen failed several times with a white diagonal line across the screen. In reference to the bulletin regarding A319/320 electrical malfunctions, I immediately pulled up the electric page to both assess the electrical failures and determine what was causing the malfunctions. All indications on the electric page were normal, with no failures. In addition, during this time both the First Officer and I could hear a clicking sound (like a generator coming on and off line) on my side of the aircraft for several minutes. We also had the following failures: Air Pack 1 regulator fault, Anti-ice Left windshield fault, Anti-ice Left window fault, Nav GPWS fault, Nav GPWS Terr. DET fault, Hyd B System Lo Pressure ECAM, Pack 1 fault, Hyd B elec pump LO PR, Vent blower fault, Captain Pitot fault. The Captain side airspeed tape was erratic after the screens had come back on the Captain side. In addition, the Flight Attendant reported that the emergency exit lights came on above the exit doors and the lights in the cabin (overhead bin area) came on. I also lost electric power to my seat for a period of time. We were also unable to activate on the performance page for the arrival. The push button worked, but the airspeed bug would not move. This lasted for about 5 minutes. We were eventually able to get control of the airspeed indicator. We did find out after landing that the vent blower Circuit Breaker had popped. In addition to monitoring the electric ECAM page, we did monitor the HYD page for the blue system failure. We never received any ECAM's indicating a loss of a generator bus and/or any other electrical bus failures. All indications on the electric page remained within normal parameters. There was no galley power shed that would have occurred with a generator failure. We did begin the start for the APU for backup generator power if there was a loss of generator. We ran all the ECAM's, and additional ECAM's as presented. I gave the First Officer a status brief on the failures while he flew, after all ECAM's were complete. I briefed the flight attendants, telling them we were diverting, we would be landing in about 15 minutes, and at the time I did not need them to prep the cabin. With all the failures, the aircraft was handling normally.

If we had received or we started to receive any flight control malfunctions or handling issues a cabin advisory and/or prep would have been issued. I told them I would give an announcement to the passengers. I completed the announcement to the passengers letting them know we were having some minor electrical issues (as they could see the lights in the cabin) and a minor hydraulic issue and we would be landing as a precaution. I ran the over weight check list, the approach descent checklist and did the landing performance based on the failures. We did ask Dispatch to run the numbers as well, unfortunately we were unable to get the data back as a check on our numbers. As the runway was almost 8,000 FT and dry, we were well within the landing performance data. These pieces of information would have been very helpful in reducing our workload, or any other information that could have been given to us to help with the diversion. Dispatch did ask our ETA, and while I understand that there is coordination going on, sending ACARS messages asking our ETA, when flight following is available to the Dispatcher and asking us below 10,000 FT what landing numbers we came up with is difficult for the crew at best. I understand that there was not much information for the Dispatcher to go with under this situation due to the multiple failures, and this is not for criticism, but pointed out as a learning tool as to the workload of the crew and what is reasonable for us to expect.

During the descent, the ECAM's stopped and both screens on the Captain side as well as the upper ECAM screen stabilized and were operational and a normal approach (with all the remaining failures presented) and landing were accomplished. Since the electrical malfunctions seemed to stabilize, we did not elect to deselect either generator, possibly causing further failures as verification of generator loss as opposed to arcing was not certain at the time with the information presented. The First Officer did an excellent job flying while I coordinated all the checklists, communications, etc. He had the aircraft slowed and stabilized for the arrival. Due to the suspected arching issue, the situation stabilized, all the ECAM's and check lists completed, performance completed and cabin secured, we elected to continue the approach and not delay landing. In order to follow up and contact Maintenance Control we would have to delay landing and hold. Under the circumstances, we felt that could cause further risk of failure and continued arcing issues. We had an overweight landing, and touched down at 200 FPM (350 FPM max per the checklist) within the touchdown zone.

In reference to the Bulletin: Airbus has alerted all operators of A320/A319 aircraft of two abnormal electrical conditions. The ECAM system may not annunciate the fault. Pilots are required to analyze and correct the malfunctions. The first malfunction is intermittent electrical power supply interruptions from Gen 1 or Gen 2. The cause can be IDG connector feeder arcing or a malfunctioning generator. The cockpit indications include flashing or blank cockpit displays on the one side of the cockpit, a flashing MCDU screen, intermittent flight control law reversions and uncommanded rudder trim inputs. Pilot actions should be to identify and deselect the malfunctioning generator. These failures seem to follow the above description in reference to the intermittent electrical power supply interruptions from Gen 1 or 2. We did have the blanking of both the Captain screens and the upper ECAM screen. We did try to analyze the situation on the electrical ECAM page, but all indications were presenting normally, we also used all other information presented to include the FOM. Without any indications on the electrical page of a malfunctioning generator and/or other electrical bus failures directing the pilot to identify and deselect the malfunction generator is lacking in information and or direction for the crew, while dealing with a large number of failures.

Having just gone through recurrent training on the Airbus, the demonstration of AC 1 bus failure as presented is significantly different then the failure we experienced. Thus, there is seemingly no training for this type of failure. In addition, below is the supplemental information. We suspected arcing. The failures were consistent with Gen 1, but not all the failures were present that a crew would expect to see or are listed on the Gen 1 Failure Checklist. We did not get MCDU1 failure or ADR 1 failure. While it can be suspected to be Gen 1, the check list calls for positive identification. Either there are other issues (as some of the failures we experienced can also be found on the AC essential bus) or the check list should read under identifying the affected generator: any or all of these failures can occur, allowing for positive identification of the generator. In addition, what is sustained flickering? We experienced the flickering for several minutes, enough to get through many of the ECAM's, but the system then stabilized. At this point, as referenced in the explanation above, we determined the most conservative action consistent with our diversion was to keep the configuration as is, in a stable operation. What is the guidance when the flickering is not sustained, but momentary or lasting several minutes?

Electrical power supply interruption, electrical arcing in the IDG feeder cable at the pylon/nacelle connector can occur, causing power supply interruption that affects one or more generator electrical phases. Maintenance has determined that corrosion leads to the breakdown of the gold plating on the connector contacts, which in turn leads to pin arcing. Due to these type failures and number of other reported electrical failures in the Airbus fleet, will there be a required inspection on the IDG connectors for the feeder, so these types of failures can be mitigated before they occur? We did have more than 50% loss of displays, [so it became an] NTSB reportable issue.

Callback: 1

Maintenance found that the electrical connections to the left generator had in fact been arcing and were burned. Airbus has issued a bulletin concerning this phenomena. Unfortunately this malfunction did not present exactly as described in the bulletin, making corrective action decisions difficult.

Synopsis

A320 Captain experiences electrical anomalies at FL340 and elects to divert. Anomalies included flickering flight displays on the Captain's side, along with the upper ECAM display. Numerous ECAM faults were displayed when the ECAM screen could be viewed.

ACN: 927797 (2580 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Aerofoil Ice System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 927797
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Fatigue
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 178
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 927519
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Fatigue
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

This flight suffered three separate system faults as indicated on the ECAM beginning shortly after takeoff. Upon turning off the wing anti-ice system the right wing anti-ice valve failed to close resulting in an ECAM message. No action was required and we determined that wing anti-ice was available and would need to be turned on for the approach, where icing conditions were reported, in order to prevent asymmetric deicing of the wings. This fault was a recurring write-up on this aircraft. The fault was reported to maintenance via ACARS and dispatch was contacted to clarify the "thrust limit penalty" indicated on the ECAM. Shortly after resolving the above matter the ECAM indicated a BSCU 1 fault. Using the System Fault Reset section of the flight manual this fault was cleared and no further action was required. At gear extension on the approach to Runway X the BSCU 1 fault reoccurred along with an LGCIU 2 fault. A go-around was executed due to the loss of the #2 thrust reverser associated with the LGCIU 2 fault. Runway X was reported to have poor braking and to be contaminated with several inches of dry snow and we did not feel it appropriate to attempt a landing with the reverser inop. 

We also needed time to trouble shoot the ECAM faults and to assess our options. The BSCU 1 fault and LGCIU 2 fault could not be resolved using either the System Fault Reset or Irregular procedure sections of the FM and were reported to maintenance via ACARS. Out of concern for possible deteriorating runway conditions, understanding the implications of any further failures and recognizing the onset of fatigue after an extraordinary work load while on duty more than 12 hours I declared an emergency with ATC. Dispatch was informed of this via ACARS message. The passengers and flight attendants were advised of our situation and the flight attendants were given a Cabin Advisory. We determined that a safe landing was possible on a runway of appropriate length with good braking action. Approach advised that Runway YR was open and met our criteria and we set up and briefed for the ILS to Y. Shortly after being vectored onto the localizer by ATC the localizer signal went off the air and we were forced to abandon this second approach. The localizer signal was restored a short time later and we were vectored back for another approach attempt. The approach was normal and we landed using medium auto-brakes and idle reverse on the left engine.

Synopsis

An A320 had a wing anti-ice valve fail open after takeoff then the BSCU and LGCIU ECAM alerted during approach. The BSCU reset but the LGCIU remained on and after two go arounds the fatigued crew declared an emergency and landed safely.

ACN: 927717 (2581 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 14000

Environment

Flight Conditions : IMC
Weather Elements / Visibility.Visibility : .75
Ceiling.Single Value : 700

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 10000
ASRS Report Number.Accession Number : 927717

Events

Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We encountered severe icing while holding at ZZZZZ intersection. We requested a change in altitude or vectors to an area away from icing. Due to high traffic volume, ATC delayed our request. While holding, the #2 engine began to vibrate and reached a level of .9. This was followed by what seemed to be a compressor stall which was probably the result of ice being ingested into the engine. Shortly after this, the #1 engine began vibrating and reached a level of .7. Before the situation could worsen, I declared an emergency to get out of holding and severe icing and to land ASAP. The company and flight attendants were notified of the situation. I told the Flight Attendants to prepare the cabin for a normal landing, but if the situation changes, I would advise them. A normal landing was made and the emergency was terminated. After parking at the gate, a visual inspection of the engine showed large build-ups of ice on the engines.

Synopsis

A320 Captain experiences severe icing while holding at 14000 feet. Increasing engine vibration is indicated and felt as ice accumulates on the fan and possible compressor stalls occur as the ice is shed. An emergency is declared to get out of icing and land quickly.

ACN: 927580 (2582 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Galley Furnishing
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Galley
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 927580
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

An A320 aircraft had a deferred aft galley chiller inoperative. Two Aviation Maintenance Technicians (AMT) were assigned to deferred [chiller]. They performed an operational test per maintenance manual (MM) 25-34-51 and found no air flow at [connecting] ducts. They removed the chiller and discovered that the duct, item #150, reference Illustrated Parts Catalog (IPC) Chapter 25-33-03a was not installed, thus preventing airflow to duct. They were able to access the back of the galley through the trash compartment access panel in the aft lavatory. This is where they found the duct, lying on water and electrical lines. The aircraft had overnighted [this month] and an AMT had replaced the chiller, but in his defense an air flow check is not required per the maintenance manual, only an electrical check. But the aircraft just came from a #3 heavy check in December, [where] the galley was removed and replaced per MM 25-33-41 and does require an airflow check of chiller system. We installed the duct assembly and performed an operational check.

Synopsis

A Lead Mechanic reports an A320 aircraft arrived with an aft galley chiller deferred. Mechanics found an airflow duct not installed, lying in water and on electrical wires.

ACN: 926406 (2583 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZID.ARTCC
State Reference : IN
Altitude.MSL.Single Value : 30000

Aircraft

Reference : X
ATC / Advisory.Center : ZID
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Descent
Route In Use : Vectors
Route In Use.STAR : GEMNI2
Airspace.Class A : ZID

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Facility : ZID.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
ASRS Report Number.Accession Number : 926406
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification
Human Factors : Other / Unknown
Analyst Callback : Attempted

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Possible FMS issue with Airbus aircraft. When the aircraft are vectored off of an arrival and then given a fix down the way with a crossing off of that fix, the FMS is reverting to the original route and turning the aircraft to rejoin the original route. This has been seen in Airbus A300 series aircraft and the Company involved has been notified through our QA department. QA at Indy Center has spoken with the Company about the issue and we thought it was a Company specific issue. This is the first time we have seen it happen with another carrier. The safety issue in this scenario is that aircraft have been seen to turn anywhere from 15 degrees to 40 degrees to rejoin the original route and sometimes the pilots are correcting and sometimes they are not. You can imagine the safety issue of having aircraft turning those amounts when pilots and controllers are not expecting this to happen. I do not know if the pilots all are aware that the aircraft are off course or not. If they are, why are they allowing the A/C to turn off course and if they don't know then all pilots need to be aware of this issue so they can correct. This is beyond me but it appears that it could be a software or programming issue with these FMS in the Airbus. Are these FMS in any other A/C as well? I believe the A300 operator has already run these scenarios in the simulator. They forwarded the reason why this is happening to our QA department. This issue needs to be brought up industry wide and find out if the FMS needs to be reprogrammed or if this can be solved through information dissemination through the pilot programs.

Synopsis

ZID Controller describes what he believes may be a glitch in A300 and A320 FMC's causing track deviations during arrivals to DTW.

ACN: 926295 (2584 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Parked

Component

Aircraft Component : Fuel Quantity-Pressure Indication
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 926295
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We arrived to find our next flight to ORD was on a ground delay due to low ceilings and deteriorating weather in Chicago. I called the Dispatcher to confirm the delay from the gate. He advised me of wheels up time. I discussed with him the fact that we had a long day already and would time out about 46 minutes after the proposed wheels up. This is where the outrageous pressure came from the Gate Supervisor; I informed her that we would be close to "timing out" as a crew. I then returned to the aircraft and received the ACARS message from Dispatch showing the wheels up time; I also printed her [Gate Supervisor] a copy. At about 2.5 hours from wheels up time I called scheduling and confirmed the information with him regarding the delay. I wanted to give him the "heads up" that the possibility would exist that we may "time out". He said the "powers that be" were talking it over. He suggested that I call back in a few minutes. He also mentioned someone from the station had called him. The Supervisor had apparently called him to reconfirm the information I had already given her. I had noticed that there were several mechanics in and out of the jetway. I went in to the aircraft to find three mechanics in the flight deck. I approached one and pleasantly asked if my aircraft was broke. He responded by saying, "define broke". I observed the mechanic who signed off the MEL on the cell phone in the cockpit. I returned to the aircraft about fifteen minutes later and noticed that he was still on the phone. I had grown suspicious after I had witnessed at least seven or eight mechanics come and go over a period of thirty to forty minutes. The mechanic in the cockpit had now been on the phone for a very long time. I assumed he was talking to Maintenance Control. Meanwhile, I thought we could begin boarding at XA55 and push at XB25. This would give us plenty of time to taxi and takeoff. Five minutes before boarding the Gate Supervisor grew very impatient and began interrupting me while I was explaining the situation to the Assistant Chief Pilot. The mechanics in the cockpit and I told her we would let her know when it was reasonable for her to begin the boarding process. I phoned scheduling and he informed me they were going "to roll the dice" and hope for a XC07 or earlier departure. The Gate Supervisor continued to interrupt us even after she was told there was no "new information" regarding the amount of time the mechanics would require to resolve the situation. This was very annoying and disruptive. After a long period of time the mechanic got off the cell phone and informed me he was putting MEL 28-40-06 condition L and N inner and outer fuel gage inoperative. The problem was that the MEL stated "2 installed and 1 required". Both channels in the right wing outer and right wing inner were inoperative also causing the FOB (Fuel On Board) to be inoperative. I looked at the MEL. This is when it crossed my mind that the lengthy conversation was how to GET AROUND all the malfunctions in the fuel system. Now the gate agent was back while I was working to understand what the mechanics have tried to do. I tried to explain to the mechanic that this MEL requires one of the systems to be working and they were not. With a long day behind us, and a three hour flight ahead of us with an alternate, I needed to know how much fuel was on board. The mechanic who added the MEL suggested I call my Chief Pilot. I called the chief pilot office at XB05 and asked if I could speak to someone regarding an airbus issue. I was on hold when, of course, comes my favorite Gate Supervisor to get her five minute update. I tried to explain that I had called Dispatch and tried to get another aircraft at XA45 but, was informed there weren't any. She did not accept that information gently and demanded that she "had to know". I spoke with the Assistant Chief Pilot and he said he would be right over. I showed him the MEL. We agreed this is a bigger problem than one channel out. The entire fuel system was down. He called the Airbus Fleet Manager. He agreed this was not a valid MEL. I called my Dispatcher to advise him of the situation. We called Scheduling and they released us.

The MEL requires one of the systems to be working. If a mechanic cannot explain the MEL or I am not comfortable with the MEL and the way it reads, being the Captain in command of the aircraft, I should not be suggested to call the Chief Pilot. There was no safe way to get around this serious malfunction. 

The problem I have noticed that has occurred in 2010 is that too much pressure is put on Maintenance to get the aircraft off the gate. It appears this group will do anything to get an MEL added even if it is not correct. In regard to safety, this culture is extremely dangerous. If the Captain of the aircraft says "no"....that means "no". The repetitive interruption from gate agents and Gate Supervisors only adds to the length of a delay.

Synopsis

A320 Captain described maintenance and weather delays including continuous interruptions by the gate agent requesting updates. The Captain believed the the MEL number assigned by Maintenance is not valid and the Assistant Chief Pilot agreed. The crew was eventually released for duty time limitations.

ACN: 926275 (2585 of 2908)

Time / Day

Date : 201101
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Powerplant Lubrication System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1483
ASRS Report Number.Accession Number : 926275
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1280
ASRS Report Number.Accession Number : 926273
Human Factors : Situational Awareness

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During pre-flight, our review of the log revealed that this aircraft had a recent history of: CARGO SMOKE ECAM warning in flight, smoke/fumes in the cabin, overweight landing, excessive oil consumption on #2 engine. While at the gate we initiated a conference call with Dispatch and Maintenance to obtain further details of what had actually occurred, and what Maintenance had discovered and accomplished on the aircraft. Maintenance advised us that repairs had been made and that the aircraft was being returned to service. Although engine runs had been performed on the ground, no test flight had occurred. We advised Dispatch that if any problems resembling the previous history occurred on our flight after departure that our intent was to return. I conducted a more thorough briefing than normal with the Flight Attendants, specifically advising them that if they detected any cabin smoke or fumes, not to hesitate in notifying the cockpit crew. Hazmat was loaded in the aft cargo pit (radio active). 

Soon after takeoff, we detected a very slight oily odor in the cockpit, similar to the smell you get in flight after being deiced. The First Officer and I were both comfortable continuing. About 12 minutes after takeoff, the Purser advised me that the Flight Attendants in the aft cabin were detecting a slight odor. I asked him if he thought it was bad enough to return. He went to the aft cabin himself to assess and then advised me "not at this time". Several minutes later I was advised by the Flight Attendants that it was getting "really bad", especially near the lavatories in the aft cabin. I decided immediately to return. An emergency was declared and we returned expeditiously. We advised Tower and the rescue personnel that we were transporting hazmat in the aft cargo compartment. I commend my entire crew for an exemplary job during a stressful situation. A normal landing occurred and no injuries occurred. In hind-sight, I think this aircraft should have had a maintenance test flight prior to being returned to revenue service.

Synopsis

A320 flight crew is dispatched in an aircraft with a maintenance history of smoke and fumes from a right engine oil leak. Approaching FL340 the problem returns and the crew elects to return to the departure airport.

ACN: 925798 (2586 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : Marginal
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Aux Engine Turbine
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Design

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 925798
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 925876
Human Factors : Troubleshooting

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17108
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 3844
ASRS Report Number.Accession Number : 925938
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Flight was dispatched with an inoperative APU due to an auto-shutdown of the APU during taxi-in on the previous flight. We preformed an engine start at the gate using external air followed by a cross- bleed start. During the walk around, the First Officer noted minor ice accumulation on the wings. We elected to have the aircraft de-iced. We taxied to the de-ice pad for deicing. After de-icing was complete we taxied to our runway for takeoff. The "ECAM Status Check", last item before taking the runway, showed [we had] "Aft Cargo Heat" and "Aft Cargo Vent" inoperative. We advised Ground and taxied north of the runway entrance and contacted Maintenance via ACARS. The Maintenance Controller responded with a recommendation to pull and reset two circuit breakers which cleared the ECAM status page. Once the Status page was cleared and all procedures and checklist were complete we taxied to the runway and were cleared for takeoff. The takeoff was normal. I reduced power to climb thrust and was calling for "FLAPS UP." We were handed off to Departure frequency when we received a Master Warning light and an ECAM warning of smoke in the aft cargo compartment. I flew the aircraft while the First Officer ran the ECAM. At this time, the Purser called to advise that we had smoke in the cabin and that all three lavatories fire detectors were flashing indicating fire. Though the First Officer and I were aware of the previous APU write up, we elected to treat this as an actual in flight fire and declare an emergency. We notified ATC, donned our O2 masks, and he continued to run AFT CARGO FIRE ECAM. ATC gave us vectors to the nearest runway. We continued to receive Master Warnings and Smoke ECAMS. We specifically chose not to Emergency Cancel the message, since it gave us an indication that the situation was continuing. The Purser called to inform us that it appeared that the smoke was heavier in the rear section of the aircraft. This added to the urgency of the situation. We informed the Purser to prepare the cabin with four minutes remaining to touchdown. I was on a base to the runway configuring the aircraft for landing. On final I observed we had full emergency vehicle support on the ground. The First Officer and I discussed the need for an evacuation. The outside air temperature was 18 F with about ten inches of snow on the ground. We both suspected that the smoke was caused by the APU oil leak. On short final the First Officer called the Purser for an update. He informed us that the smoke had diminished considerably. We instructed him to only perform an evacuation if smoke increased or the situation deteriorated. We preformed a smooth overweight landing. The First Officer made a "remain seated" PA announcement. We exited on the high speed and Tower handed us off to the Fire Control frequency. The emergency personal did a full inspection of the aft cargo area that also included thermal imaging. They informed us that the imaging showed only slightly warmer temperatures surrounding that area. We requested an escort to the gate and proceeded. We deplaned the passengers normally.

Narrative: 2

Aircraft came into the gate. We had gotten a call that APU auto shut down on taxi in. [I] then got a call that they had low oil level. Found oil level below sight gauge. Added three quarts serviced to full. Started APU to check for leaks in compartment. None found. Loaded up APU electrical and air at which time we got smoke in cabin. Shut APU down and rechecked oil level was empty again. 
Defer APU reference MEL 4910. Ran #1 engine for approximately thirty minutes to burn packs out. Cleared cabin of smoke but still had a slight oil smell. I had forgotten to enter oil in oil service display but oil quantity was stated in deferral text.

Narrative: 3

Flight was dispatched with an Airbus A320. It had been scheduled for a long flight and had 27,400 LBS of fuel, almost double the cleared fuel load of 16,000 for our planned destination. It was dispatched with an inoperative APU due to an auto-shutdown of the APU during taxi-in on the previous flight. The extra fuel produced an overweight landing condition at the original dispatch loads and Captain initiated contact with Dispatch to resolve the issue. Boarding had started but was stopped. We believed it was to resolve the weight problem but were later told the boarding was stopped due to an airport security breach. This is unsubstantiated. The overweight landing concern turned out not to be an issue due to a large number of misconnecting passengers, but it was a significant distraction in the cockpit preparation phase. Distractions included an unidentified CSR Supervisor pushing us to close the door, assuring us "we were OK on the weights and legal to go," even though we still did not have a legal dispatch release. Captain did not allow door closure until we had a legal dispatch release and the passenger count was verified. The lack of an APU required an engine start at the gate with external air followed by pushback and a cross-bleed start. We taxied to the de-ice pad for deicing of minor snow and ice accumulations from the ground time. The aircraft was deiced with Type I. We completed the off-gate deicing reconfiguration, configured the aircraft, conducted the control check, and contacted Ground Control for taxi. We conducted the Before Take-off Checklist while taxiing to the runway. The "ECAM status check", last item before taking the runway, showed "Aft Cargo Heat" and "Aft Cargo Vent" inoperative. We advised ground and taxied north of the runway entrance and contacted Maintenance via ACARS. The Controller responded with a recommendation to pull and reset two circuit breakers which cleared the ECAM status page so we advised Ground, received our sequence and taxied for departure. The take-off was normal. As we were handed off to Departure and were approaching 10,000 MSL we received a Master Warning light and an ECAM warning of smoke in the aft cargo compartment. This took place as we were cleared for a west-bound turn and cleared to FL230. As pilot not flying I responded, set the altitude alerter, and went back to the ECAM. At this time the Purser called to advise that we had smoke in the cabin and that the lavatory fire detectors were flashing. I acknowledged this and advised Captain. She directed me to complete the ECAM action item while she declared an emergency and asked for an expedited return to the field. The A320 aft cargo compartment fire extinguisher panel is different from the other cockpit switch and light displays so I asked her for confirmation that I was discharging the correct extinguishing agent into the correct cargo compartment. She confirmed the action, I discharged the bottle, and we put on our oxygen masks. We continued to receive Master Warnings and Smoke ECAMS. I specifically chose not to Emergency Cancel the message, since it gave us an indication that the situation was continuing, although it proved to be extremely distracting. I asked Captain about a cabin prep. I told the Purser to do a cabin pep with landing in about four minutes. We were on a base and Captain was configuring us for landing when I got back to her. I attempted to set up the FMC for a return but did not select a runway at that time, which left Captain navigating visually to the runway. I finally selected the runway on short final. We extended the gear and I called the Purser for an update. He said the smoke seemed to be improving. I told Captain and we discussed an evacuation on the runway. We decided not to unless we had any further indication, since the outside temperature was about 18 F. On landing I used the PA to direct the passengers to remain seated. I called Operations, asked for a gate, and told them we had discharged the aft cargo fire extinguisher bottle. This was not passed on to the fire crews. Our first contact with a company employee was when a Maintenance Supervisor entered the cockpit. He aggressively demanded to know why we fired an extinguisher bottle, and then asked if we had done it "because the Fire Department wanted us to." I replied we had done so at the direction of the ECAM procedure when we had an airborne smoke event. He stated he thought they had one bottle in stock and could replace it and send us out, with no regard for the actual cause of the smoke in the cabin. Discussions with maintenance personnel indicated that the smoke may have resulted from oil contamination of the pressurization ducting caused by the APU failure. An individual stated that the crew that deferred the APU should have done a high-power run to "burn out the ducts," but that this apparently had not been done.

Synopsis

An A320 APU auto shut down on taxi in due to an oil leak. During maintenance troubleshooting leaking oil got into the air conditioning system and caused smoke. The APU was MEL'ed but after takeoff the cabin filled with smoke causing the crew to declare an emergency and return to land.

ACN: 925782 (2587 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SNA.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility.Other 
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 925782
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4200
ASRS Report Number.Accession Number : 925741
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We pushed a few minutes late due to ATC and no maintenance release. We planned for a takeoff on Runway 19R according to the ATIS. After conversations between the Captain and the push crew about brake release without clearance, we pushed. With the initiation of the second engine start the maintenance release started printing on ACARS. We also noticed at that time that runway 1L was in use. 

We got the latest ATIS and the data for Runway 1L. Captain made the appropriate changes on the flight plan and performance page and I called for taxi. ATC instructions involved back-taxiing on Runway 1R-19L with a couple of taxiway turns before and after. We started taxiing and I selected flaps and was backing the Captain up with the clearance. While on Runway 1R ground told us to contact clearance for a reroute. As I copied the clearance, the Captain inserted the changes. I verified and continued the checklist from the manifest changes. 

When we went back to ground control they sent us back to tower for an immediate takeoff. The Captain applied full power on the brakes and we started down the runway. As I always do when checking the proper power settings, I glanced at the flaps and performance data. I noticed it called for flaps 3 and we had flaps 1. I told the Captain we should abort the takeoff because we had an improper flap setting. He quickly looked trying to assess. While maintaining directional control he decided to continue because we were accelerating very quickly. He delayed the rotation about 15K and we got airborne.

Narrative: 2

[I] accomplished first flight-of-day checks and normal cockpit preparation. I conducted initial trip briefing and the briefing for this flight as I would be pilot flying. I expected and briefed departure from 19R and engine failure profile for first flight of day. No maintenance item due to a maintenance walk needed. Operations were notified and they contacted maintenance to remedy. Before Start and Pushback Checklists [were] completed. Shortly before departure time, Operations informed us the maintenance walk around issue had been resolved and to expect new maintenance release. About this time, ground crew called saying checklist complete and release brakes if you want. [We were] told by ATC to hold on gate. I told pushback crew we were instructed to hold. Pushback crew again asked for brake release. I responded that I can't release brakes until I receive pushback clearance. I mention this because it was an additional distraction combined with the following distractions. We noticed that departures were using 1L and First Officer started ACARS request for 1L performance. At this time, we received ATC pushback clearance. Normal pushback and both engines started. The maintenance release came across the printer as we finished engine start. ATC instructed us to taxi 1L Alpha hold short Juliet. As we proceeded, we discussed that we would need a new departure procedure or a heading and vectors departing 1L. This uncertainty was a contributing distraction. Before reaching Juliet, instructed taxi Juliet back taxi 1R Hotel Alpha hold short Delta. As we turned off 1R, we began the before takeoff checklist. This was completed down to the manifest changes line. Flaps had been set to flaps 1. I responded to the checklist item with 1 planned 1+F indicated detent. 1L takeoff performance data called for flaps 3. I am very familiar with SNA departures and should have noticed the mistaken flap setting at this time. I am also very familiar with the 19R pre-curfew taxi routing. I believe the unusual taxi routing combined with the late recognition of 1L being the active runway offered further distraction. As we turned onto Taxiway Alpha, ATC instructed us to contact clearance for new routing. After inserting the new route and altitude, we finished the checklist but were again distracted by the new changes to the manifest changes portion of the before takeoff checklist. We were given line up and wait clearance. The pacing was quick but not hurried. We received takeoff clearance and I held the brakes to 1.10 EPR and selected toga thrust and released brakes. This is the 19R noise abatement procedure but I elected to use it for better performance. At 130,000 LBS, we accelerated quickly. Accelerating through 80 KTS, I heard the First Officer say something regarding the flap setting. By the time I processed what the First Officer said about the flaps, considering our relatively light weight and rapid acceleration, I felt the safest course of action was to continue the takeoff and verbalized this to the First Officer. I delayed rotation to a speed approximately 10-15 KTS beyond Vr. In retrospect, I needed to slow down the operation as our tasks increased. I allowed several distractions and work flow interruptions to affect the safety of our flight.

Synopsis

Following a plethora of runway and routing changes during pushback and taxi, the flight crew of an A320 failed to note the need for flap setting three vice one on takeoff from SNA. When alerted by the First Officer well into the takeoff roll the Captain elected to increase the Vr speed and continue the takeoff.

ACN: 925703 (2588 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 925703

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 925745

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Corporate
Function.Other

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

We arrived at the aircraft in the early morning, noticing light frost on wings and tail. I immediately walked through the cabin and examined wing surface. The First Officer was doing aircraft acceptance. We agreed to reposition aircraft into sun at scheduled departure time due to no deicing equipment on field. Due to expected extended taxi we added 1,000 LBS of fuel. 

Due to unexpected boarding issues and fueling delays, we pushed back 1:40 late, negating the need to park in the sun. By this time frost on wings had become a non issue, because the sun had been up for 30 minutes. During the push we observed other aircraft that were parked next to the terminal facing the same direction and their tails and wings looked good. We performed yet another contamination check and were in agreement that the aircraft looked safe. We slowly taxied out giving plenty of time for engine warm-up, performed yet another contamination check utilizing all company detection procedures. After several inspections of the wings, three from the jet way looking down on top of the wing and at tail surfaces, and one from the cabin we safely took off 42 minutes after sunrise with what appeared to be a clean aircraft. 

In summary, the First Officer and I were in total agreement that the aircraft was safe, and neither cabin crew nor passengers expressed any concerns. After takeoff, the flight was uneventful. Upon arrival we were notified by ACARS to contact Scheduling. They informed us we were to deadhead to ZZZ on the next flight. After departing the aircraft we received a voicemail to contact the ZZZ Chief Pilot who then ordered us to return to ZZZ and come to his office directly. We asked why and he stated that a passenger, who is also a corporate pilot, submitted incriminating photographs that we flew an aircraft with possible contaminated wings on our just completed flight. 

Why didn't this pilot ring the call button to notify the crew of his concerns? This professional pilot instead allowed the flight to continue despite his concerns. He then used his electronic device to photograph the take off. We have been informed by the Chief Pilot these photos show evidence of moisture flying off of the top of the wing surfaces. We have not been allowed to view these photos. With the current facts we still believe we followed all company procedures and we departed with an aircraft that was free of any contamination.

Synopsis

An A320 flight crew was accused by a licensed pilot passenger of taking off with contaminated wings which he had documented with photos taken during the takeoff.

ACN: 925693 (2589 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : AC Generation
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14700
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 6700
ASRS Report Number.Accession Number : 925693
Human Factors : Troubleshooting
Human Factors : Confusion
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 926457
Human Factors : Troubleshooting
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of 10,000 FT autopilot on, number 1 generator failed. Got ECAM, autopilot and autothrust went off. Captain told me to fly the airplane and he accomplished ECAM procedures. Generator 1 came back online for about 20 seconds than failed again. This particular generator had a history of malfunctions and Contract Maintenance had cleared a write up just before we left. Captain started the APU and we decided with the maintenance history we would return since we were only about 35 miles away. We did not declare an emergency since we had two generators working one being the APU. We returned for an uneventful landing.

Narrative: 2

During climb through 13,000 FT ECAM for number 1 generator loss. Verified on ELEC page. ECAM has reset of generator. It stayed on line for 30 seconds and kicked off again. Flying First Officer stopped at 15,000 FT and took radios. I ran ECAM and started APU for second generator. We elected to return since we were only 45 miles from departure airport and the aircraft had a history on the same generator that day and two the day before including a mention of arcing.

Synopsis

A320 flight crew experiences a generator failure which, when reset, will not remain reset. After starting the APU, the crew elects to return to the departure airport due to a history of failures for this aircraft.

ACN: 925311 (2590 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 925311
Human Factors : Communication Breakdown
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Primary Problem : Company Policy

Narrative: 1

[I] rescheduled onto a late flight and an airplane change. Entered cockpit and mechanic handed me logbook and told me I was good to go. Maintenance Control had applied MEL 27-93-01B ELAC #1 inoperative [a] very complicated MEL with operational procedures. [I] called Dispatch and Maintenance Control. Write-up was a flight control issue on previous flight - I was very uncomfortable from the start with this. Dispatch had sent an amended release with new MEL. During the discussion with Maintenance and Dispatch, I learned that this was a revised/temporary MEL and the paperwork should have shown a "*T" to inform me of a revision. Dispatch sent a new amended release by ACARS, and I informed Maintenance Control to send a mechanic with the new revised MEL. The current MEL book in the aircraft did not have a condition B. When mechanic showed up with revised paperwork, we could not determine if the revised procedures were used when the original mechanic applied the MEL - the original mechanic had gone home. I told the mechanic that we needed to start over. We needed to ensure that the new revised procedures had been properly completed and complied with. From start to finish, it was over an hour before all procedures and paperwork was completed and I could accept the aircraft. The aircraft had a history - this was the third ELAC write-up in 16 days. This came about because:
1. New MEL procedures are way too complicated - too easy to miss something or make honest mistake. 
2. Not advised properly of the revised MEL (condition B) *T.
3. 4 pages of MEL procedures - way too much and too complicated - took over an hour to accomplish.
4. Little to no info given to flight crew on the maintenance procedure. This is a flight control issue, and crew needs more information to determine safety of flight.
5. 3rd write-up in 16 days - company should have been more proactive in troubleshooting issue.
6. A complicated operations procedure for flight crew to accomplish - procedure had little explanation of the purpose of the procedure (what exactly are we checking?) Condition B states to complete condition A procedures - too easy to miss. Condition B should state the procedures to follow on its own.
7. took too long to accomplish all items - over 1 hour to apply and evaluate MEL.
8. Too little information given to the flight crew. New "culture" in training to minimize pilot's handbook, makes research and ability to find information easily difficult in real world line ops. 
9. Almost no information given to flight crew on maintenance procedure - had to review Maintenance Control procedure with mechanic with info from the maintenance manual.
10. Complicated operations procedure to be complied with prior to each departure did not require an "orange" placard.

The new MEL system is almost non-workable in complicated instances like this. It is too hard to follow, and allows the opportunity for too many mistakes to be made - honest mistakes that may affect safety of flight. The fact that this took over 1 hour reflects the complexity of this MEL and its application, in addition to the understanding of it by the flight crew and mechanic. Procedures failed as flight crew was not properly notified of the temporary revision - this could have been a huge factor in this case. Because the system is so complicated - it was not immediately found by the crew. In fact, there is no way for the crew to know, unless the procedures are followed and Maintenance and the Dispatcher properly advise the crew. Over and over in this case, I had to search for information from multiple sources - complicated. Needed to devote all attention to this complicated MEL - but since flight were now delayed - constant interruptions - which opened the door to possibility of mistakes. As a line pilot, I do not like this MEL system. It is difficult to use, hard to understand, and many times, requires several readings of the procedure for even a minimal understanding of what is required. Note: since this new MEL system was implemented - have found numerous mistakes in MEL applications when picking up new aircraft or having MEL's applied. Incorrect MEL conditions, incorrect placards, etc. It is not people not doing their job - it is because the system is so complicated and hard to understand, that it is hard to apply correctly, and hard to ensure that MEL's have been applied correctly. My suggestion-return to the old system - or completely rewrite this system.

Synopsis

An A320 ELAC was MEL'ed before the crew's arrival in accordance with a complicated new procedure published as an addendum and not in the current MEL. Uncertain about the MEL's proper application the Captain required a mechanic to reapply the MEL which required an hour's work.

ACN: 925114 (2591 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 23000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Icing

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 925114
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

ATC informed us that there was reported light to moderate icing from FL185 to FL225. We were in moderate icing from 14,500 to approximately FL270. We had engine and wing anti-ice systems on and operating. We were both very concerned about the amount of ice building on the airplane as evidenced by the significant amount building on the probe between the windscreens and all of the way around the two front windscreens. I selected Maximum Continuous Thrust (MCT) at about FL180 to facilitate climbing through the icing more quickly. Our clearance was to FL230 which was set in the FCU window with the autopilot flying. We were both significantly distracted talking about and observing the icing and also coming up with a plan of action, that our brains were focused on climbing out of the icing. So when the autopilot leveled off at FL230, the airplane kept accelerating since we were still in MCT. Thinking we were getting unreliable airspeed due to icing, and expecting to keep climbing instead of being level, I disconnected the autopilot and pulled up. The altitude alert went off at 23,300 FT and I realized what happened and started back down to FL230 after reaching 23,400 FT. Also contributing was that during the climbout, the VVI (Vertical Velocity Indicator) fluctuated between -300 and +1,100, so seeing the VVI at 0 was somewhat expected in the back of my mind. Our concern was the unknown amount and weight of the ice on the unheated parts of the airplane since the ice did not sublimate enroute.

Synopsis

A320 flight crew becomes concerned with ice build up during climb and selects MCT to expedite the climb. When the autopilot levels off at FL230 the aircraft rapidly accelerates. The pilot flying disengages the autopilot and begins to climb before the altitude chime alerts him to the altitude clearance and that the MCT setting is causing the airspeed increase.

ACN: 924809 (2592 of 2908)

Time / Day

Date : 201012

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 924809
Human Factors : Other / Unknown
Analyst Callback : Attempted

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Physical Injury / Incapacitation

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Received ECAM SMOKE LAVATORY message. Rang Flight Attendants to find source of smoke/fire. Response was that a male passenger was using aerosol hairspray in aft left lavatory. Once lavatory door was opened, alarm went off (ceased). Two Flight Attendants were injured in rush to determine cause of fire alarm. Inform passengers through safety demo that detector's will alarm if hairspray is used.

Synopsis

A320 Captain reports lavatory smoke alarm activated by aerosol hairspray used by a passenger. Two Flight Attendants are injured in the rush to determine the cause.

ACN: 923949 (2593 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class E : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16300
Experience.Flight Crew.Last 90 Days : 70
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 923949

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 923960

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Air Traffic Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Vectors over Mt. Diablo leveling off at 5,000 FT. MSA is 5,100 FT in this area. Knew where the terrain was, but still received a "Terrain, Terrain, Pull Up" warning from EPGWS. [We] executed the terrain avoidance maneuver. Climbed up to 5,600 FT and was in VMC at that point. Leveled off and told Controller of our actions. Controller cleared us back to 5,000 FT clear of terrain. We descended and landed uneventfully.

Narrative: 2

While on RADAR vectors with Norcal Approach in IMC conditions we were given clearance to descend and maintain 5,000 FT. As the aircraft started to level off at 5,000, we received an automated terrain alert "Terrain Terrain Pull UP" from the EGPWS. We immediately initiated the terrain escape maneuver climbing to approximately 5,600 FT. The Captain contacted ATC to advise them of our climb. We quickly flew into VMC conditions and were able to visually verify our flight path and stopped the climb. ATC re-cleared us to 5,000 FT and we continued the approached uneventfully.

Synopsis

The Flight Crew of an Air Carrier inbound to OAK reported an EPGWS warning near Mt. Diablo when complying with an ATC issued a descend clearance to 5,000.

ACN: 923915 (2594 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : CLT.Airport
State Reference : NC
Altitude.MSL.Single Value : 3000

Aircraft

Reference : X
ATC / Advisory.TRACON : CLT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach

Person

Reference : 1
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
ASRS Report Number.Accession Number : 923915
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Detector.Person : Flight Crew
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

CLT was doing ILS approaches to Runway's 18L/C/R. Runway 23 was closed and being used for deicing procedures. I was cleared to fly, and had intercepted, the ILS 18C CLT. I was cleared to land 18C. ATC requested 170KIAS until OZEJI. Flying a dual-coupled approach, I was instructed to turn off the localizer approach for re-sequencing due to proximity to an aircraft on approach to 18R, not previously mentioned by approach control. We had seen the aircraft on TCAS, though we had not received a TCAS TA. When we had started the approach he had been to our right lined up on 18R, approximately 2.5 miles in front of us. Flying the requested speed, I saw our distance to the aircraft on the parallel approach decrease to less than a mile when approach vectored us off our approach. Upon re-sequencing, we were very alert to aircraft approaching the parallel runway. Again we were cleared and were flying a coupled ILS 18C approach. We were cleared to land. Approach requested we fly 170 KIAS since we were 6.5 miles behind the aircraft in front of us to 18C. We were only 2.5 miles behind and to the east of an aircraft flying the ILS to 18R. As this distance diminished, we queried approach control about the speed but he insisted we stay at 170. We complied and saw the distance continue to decrease. We received a TA highlighting the aircraft on approach to 18R we had been watching. We questioned approach and they broke off our approach again vectoring us around for re-sequencing. On the third approach we successfully landed. The Approach Controller and monitor or supervisor, we heard two distinct voices, were only worried with the horizontal spacing between aircraft assigned to the same runway. They were so intent on keeping a particular distance; they appeared to give no consideration to position relative to a parallel runway. In two instances the speed they requested of us caused closure to a near parallel position to an aircraft on the parallel approach. The aircraft on the parallel approach drifted off course enough to pose a potential threat to our flight, necessitating breaking off our approach and being re-sequenced in the pattern. On the first approach, the vectoring occurred before a TCAS TA occurred. On the second approach a TCAS TA occurred. 

Suggestions: 
1. In a non-normal situation using Runway 5/23 for deicing procedures consider using the east and west runway for approaches and the center runway for departures. 
2. Suspend controller training during adverse weather/non-normal operations. Let them observe, but not control. 
3. Institute requirements that controllers keep nose/tail, for lack of a better term, separation distances not only for aircraft on approach to a runway but for those on near parallel runways. I am not suggesting the same distance but simply not having a parallel approach pass one another. There are too many things that can go wrong since the thresholds are approximately parallel, hence parallel glide paths.

Synopsis

An Air Carrier experienced multiple go arounds at CLT during simultaneous approach procedures.

ACN: 923691 (2595 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 2400

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flight Dynamics Navigation and Safety
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 923691
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were cleared for the Localizer 27 approach. First Officer was pilot flying. ATC issued usual speed restrictions and we briefed about that and the Aircraft ahead of us. Specifically [we were] to maintain 210 KTS to the first intersection then 170 to the second intersection. Descending from 3,600 FT from the third to the fourth intersection I noticed an unusually high sink rate. Looking at the flight plan page I noticed the constraint for the fourth intersection i.e., 2,400 FT, had dropped out of the box. The First Officer had a managed descent selected and the altitude selected in the altitude box was 1,800 for the restriction at the second intersection. I pointed out the conflicting info and the First Officer had already begun to make corrections. Due to a higher descent rate the aircraft did not level off immediately and simultaneously I believe due to the sink rate a terrain GPWS was triggered. I called missed approach and the First Officer performed a climbing maneuver commensurate with the GPWS warning. Another factor with this scenario is that we had a FWC failure during the flight that was giving other warnings that were not correct involving flap positions and speeds. In other words a great deal was happening at once during our go around. Priority was given to GPWS warning which was "Terrain". The procedures were followed correctly and we notified ATC we were going around. The following approach and landing was uneventful. After arriving at the gate I discussed at length with the Maintenance personnel what indications we were getting and they could not find any evidence of speed exceedances or control surface position. Although we had warnings generated perhaps by the faulty Flight Warning Computer (FWC)? I believe we generated a Terrain GPWS warning due to a sink rate issue. Regardless of the cause the necessary steps and actions were taken to maintain the safety of the aircraft. In this case a go around with priority for a terrain warning. Nothing occurred out of the ordinary other than a constraint issue on a non-precision approach. The deviation was identified and the proper action initiated. The additional factors with a failed FWC were a surprise. But flying the airplane was the priority. Perhaps a little more info related to such a system failure may be useful. Information provided by the QRH and subsequent equipment manuals are adequate for basic knowledge. I would have like to have known that there may be spurious warnings triggered for flight configurations. If in fact they do? Or maybe [it was] just a coincidence.

Callback: 1

The Reporter stated that he talked with Maintenance personnel for at least a half hour after this event giving them as much information as he could and also trying to understand the indications the crew had in flight. The Flight Warning Computer (FWC) should not alert failure unless both computers have failed. The other odd thing related to the event was not a single equipment failure or flap exceedance was recorded in the automated fault logs.

Synopsis

An A320 FMGC dropped an arrival intersection altitude constraint and so the aircraft continued its descent to the point where the EGPWS issued a TERRAIN warning. During the go around a FWC failure caused incorrect flap position and speed alerts but neither the FWC failure nor flap exceedences were recorded in the automated post flight maintenance log.

ACN: 923508 (2596 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : Marginal
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 923508
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Time Pressure
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 7720
ASRS Report Number.Accession Number : 923510
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[We were] on vectors for ILS 28R at 6,000 FT. We were on downwind and when I went to select no star to clean up the box. At that moment the MCDU stopped responding and I got a red map not available message on my display. That was followed by a cabin press LDG ELEV fault ECAM. Shortly after the First Officer commented he had also lost his map. At this point we were given a turn to base and were cleared to 4,000 FT. More failures followed by loss of INIT weights, auto tuned NAVAIDs and all flight plan data. While the First Officer continued to fly I quickly manually tuned the ILS on NAV panel. At that moment we were given an intercept vector and told to maintain 3,000 FT until established on ILS. There were broken clouds in the area at 1,600 FT. We continued on the ILS and also had the bridge in sight. [We] landed uneventfully. This was similar to a dual FMGC failure without the loss of autopilot 2 and auto throttles.

Callback: 1

The Reporter determined that the FMGCs were the Thales updated units and that a post flight maintenance examination did not find any faults recorded for either FMGC. The restarts were normal and the system operated normally for 10 days at which time the #1 FMGC failed in flight. The #2 FMGC continued normal operation in that event. The Reporter still finds it odd that even though the #2 FMGC indicated failure the auto throttles and Autopilot continued functioning. A FMGC failure should have caused the automation systems to loose performance inputs which would automatically disconnect auto throttle computations and multiple autopilot inputs.

Narrative: 2

While descending and receiving vectors for downwind for Runway 28 , the Captain was cleaning up the flight plan page when his ND stop showing his map display and displayed the message "map not available." We then noticed the same thing on my ND with the loss of navigation data and the same message. We then got a CAB PRES LDG ELEV FAULT ECAM. I then noticed all my navigational data was lost including the destination VOR which was hard tuned and even my ILS scales on my PFD. The DDRMI had orange flags as well. At that point we lost all navigation capability and reference. The autopilot and auto throttles remained on during the event until on final when I turned them off. I told the Captain I lost all navigational info and he switched to standby navigation on his side. He manually tuned the ILS to 28 and got the ILS info on his PFD but I still did not have any navigational data. I selected ROSE ILS on my ND mode and picked up a good ILS signal with the IDENT but without DME. I just flew the final approach course referencing the rose ILS display. All the while we continued to take vectors to final and descent clearances. When cleared to intercept final we tried to arm the LOC mode but it would not arm, so I just intercepted manually using the heading mode. The Captain was looking in the FMC pages on the MCDU but everything was gone. All the flight plan data had dumped. While on final I called for managed speed but that was not available so we just used a selected speed referencing Vls. We broke out of the clouds about 1,500 FT and I got a little low on the glideslope but corrected. We were told to sidestep to Runway 28L which we did and landed and taxied to the gate without further complications. From the time we first lost our navigational data until the time we landed was about five minutes. After debriefing and taking time to sort out what happened, what we did and look in the flight manual, we suspect we did not have a total dual FMGC failure since the autopilot remained on but somehow lost all FMC flight plan data since all the capabilities which rely on that data were lost. I believe we handled the situation well. If we had more time we could have referenced the dual FMGC failure checklist, but we were concerned with getting some navigational reference and our situational awareness back while being vectored. The dual FMGC failure checklist would have had me also select standby navigation which may have given me ILS info on my PFD but I don't think we would have gained anything else. The Captain and I and flown together many times previously and I feel we handled the situation very well.

Synopsis

An A320 crew reported a Dual FMGC failure while on vectors for an ILS but the Autothrottles and Autopilot continued functioning on #2. They were unable to reset the FMGC's but manually tuned the ILS and completed an ILS below broken clouds.

ACN: 923466 (2597 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : JFK.Tower
State Reference : NY
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : JFK
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Route In Use : None
Airspace.Class B : JFK

Person

Reference : 1
Location Of Person.Facility : JFK.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 923466
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 advised that flight deck observed an Emergency vehicle drive across the landing runway while aircraft was on landing roll. No alerts from ADSE/X, and aircraft exited the runway in a normal manner. Recommendation, fix ADSE/X. Train vehicles to be more observant of active runway/s.

Synopsis

JFK Controller described an unauthorized runway crossing by an emergency vehicle, noting the ADSE-X did not detect the infraction.

ACN: 922795 (2598 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 922795
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 922871
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected.Other 
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was pilot flying. Smooth flight. [We had a] normal descent, approach and touchdown. Selected reverse thrust. Engine 2 went into reverse but Engine 1 did not. [We got a] initial ECAM of number 1 engine reverser inoperative. [We] stopped aircraft on the runway and set parking break. Reverser ECAM went away and Green Hydraulic System Press Low ECAM displayed. Completed ECAM and referenced FM for additional information. [Our] nose wheel steering [was] inoperative due to loss of Green Hydraulic System. Tug arrived and we were towed to gate.

Narrative: 2

On landing rollout as pilot flying First Officer applied reverse thrust I noticed that #1 engine reverse was not deployed and aircraft was yawing slightly to right. First Officer kept us on centerline with rudder. As I mentioned that #1 thrust reverser was not deployed we got the ECAM #1 reverser fail. I called out the ECAM and commanded the First Officer to bring #2 engine to idle reverse, which he was already doing. At the same time we got multiple ECAMs related to the green hydraulic system. We had lost all green hydraulic fluid. I took control of the aircraft at 80 KTS and brought us to a stop on centerline. First Officer advised Tower that we had had a hydraulic failure on landing rollout and would not be able to clear the runway. At that point the First Officer ran the remaining ECAMs. I then made an announcement to the passengers to remain seated with seatbelts fastened, then spoke to the Flight Attendant on interphone, explained the situation and emphatically told her that no emergency existed and to make sure all passengers stayed in their seats. I wanted to ensure that no passengers initiated an unnecessary evacuation. We then contacted Dispatch informing them of our situation. We received a tow to the gate, where after the aircraft was secured I called Dispatch and the Chief Pilot. I am in the habit of checking both the fuel and hydraulic synoptic at Top of Descent and coming through 10,000 FT, so I can say with certainty that all hydraulic indications were normal at that time.

Synopsis

An A320's Green Hydraulic System line was damaged on landing causing the system to loose all fluid and pressure so that without nose wheel steering it had to be towed off the runway to the gate.

ACN: 922569 (2599 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 119
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Main Gear Tire
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 50
Experience.Flight Crew.Type : 6600
ASRS Report Number.Accession Number : 922569
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 925349
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance

Person : 3

Reference : 3
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 927693
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Primary Problem : Ambiguous

Narrative: 1

Number 1 Main Landing Gear (MLG) tire previously deferred. Preflight revealed an additional deep puncture in same tire. Weather was 10 KTS gusting to 25 KTS crosswind and cold. Crew requested a tire change. Maintenance refused, stating that it was deferrable and that it was cold and crew should "go fly their airplane." Mechanic repeatedly barged into cockpit, interrupting Captain's PA where we were actually commending the superior quality of our Maintenance Personnel. Mechanic made disparaging comments about the announcement and told Captain that they would not address requested maintenance with out a refusal meter from Dispatch. All actions up to including the stated demand for a "refusal meter" in order to accomplishing the requested work can only be classed as pilot pushing. There is no FAR or FOM requirement for a "refusal" to obtain flight crew deemed necessary maintenance. Furthermore, the abusive tone and interruptions in front of the passengers and in my personal workspace was highly inappropriate. A follow on call was placed to FOD, with his assistance, a plane change was accomplished following an unnecessary 1 hour delay.

Narrative: 2

A log was entered about 10 minutes prior to dispatch. The log stated as follows: "No. 1 main tire needs to be replaced." At this time I figured the crew had not read the Maintenance Release, as the tire had been previously deferred. The station Maintenance Controller called and informed me the crew was refusing the aircraft due the log write up. I went to the gate, inspected the tire and found it to be serviceable per Maintenance Manual.

Synopsis

An A320 Captain reported a tire cut which he deemed required a tire change. A Mechanic said the cut was MEL'ed and acceptable. The Captain refused the aircraft and so required an aircraft change.

ACN: 922318 (2600 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : EICAS/EAD/ECAM
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Ignition System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 3

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 25
ASRS Report Number.Accession Number : 922318
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
When Detected : In-flight
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

An A320 aircraft already had ignition system 'B' deferred, on #1 engine. Crew reported not getting an ECAM warning that: CIRCUIT BREAKER TRIPPED REAR PNL (PANEL). I deferred this discrepancy per MEL 74-31-03, per our Maintenance Control. This MEL states that an ECAM caution ignition fault may be inoperative. After reading the MEL again, I am still not sure whether it could be used to cover the tripped circuit breaker warning on the ECAM, since the circuit breaker is part of the ignition system. I suppose it is only for the ignition fault ECAM warning. However, not getting the CIRCUIT BREAKER TRIPPED REAR PNL ECAM warning for that specific ignition system circuit breaker, is also a fault of that part of the ignition system; meaning the circuit breaker of the ignition system [that was] already deferred, was faulty. 

I pulled the other ignition system circuit breakers, one at a time, to verify that they gave the ECAM warning: CIRCUIT BREAKER TRIPPED REAR PNL and then reset them. Perhaps an MEL that addresses the ECAM warning: CIRCUIT BREAKER TRIPPED REAR PNL being inoperative, for a system that is already deferred and inoperative with the circuit breaker pulled and collared, would alleviate this issue. I cannot see any reason that the circuit breaker tripped warning needs to work for a specific system that is deactivated and the circuit breaker pulled and collared. In the future, I will do my best to make my own decisions regarding applicability of an MEL and not let a Maintenance Controller misguide me. I am sure though that he did not see a problem with using the MEL as applied.

Synopsis

A Line Mechanic reports deferring a flight crew write-up about not getting an ECAM display: CIRCUIT BREAKER TRIPPED REAR PNL warning on an A320. Maintenance Controller advised him to use MEL 74-31-03, even though that MEL only states that an ECAM caution ignition fault may be inoperative.