ACN: 921699 (2601 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DEN.Airport
State Reference : CO
Altitude.MSL.Single Value : 8500

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : DEN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : DEN

Component

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7500
ASRS Report Number.Accession Number : 921699
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7500
ASRS Report Number.Accession Number : 921689

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was pilot flying, hand flying a visual to 16L at DEN. Approximately on an eight mile final I lost my PFD (Primary Flight Display) and ND (Navigation Display) for about 5 seconds. Glanced at engine instruments to ensure both engines still running. Glanced up and noticed Gen 2 fault light on and stated "we lost generator two". The Captain concurred and said he would run the ECAM and I would keep flying. The generator would not reset. The Captain started APU. Landing and taxi in were normal.

Narrative: 2

Deselected generator #2 and powered AC Bus2 via APU. Approach and landing were uneventful. Maintenance was notified and debriefed on the event.

Synopsis

An A320 Flight Crew suffered the loss of generator #2 on an eight mile final in VMC. Re-powered the bus with the APU and landed uneventfully.

ACN: 921648 (2602 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Facility : LAX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 921648
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

False safety logic alert. An A320 target was generated on Runway 25R at B6 and triggered an alert (warning Runway 25R occupied) with another Air Carrier 3,000 FT into his departure roll. It was too late to cancel and miss the false target at B6, I looked for a vehicle or possibly a taxiing aircraft that may have wondered onto the runway but did not see anything. Not that it would have mattered because the Air Carrier could never have become airborne nor could he have stopped before B6. The target reappeared for a few more swipes and then disappeared. It turned out that the A320 pushed back off their gate about 10-15 minutes later. Their gate is not far from B6.

Synopsis

LAX Controller described an equipment safety alert event received after an aircraft started the takeoff roll, noting the aircraft call sign listed in the alert was that of an aircraft parked in a near by gate.

ACN: 921425 (2603 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Component : 2

Aircraft Component : Emergency Exit
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 921425
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was working aircraft to inspect a crack in panel (412 DL) left ENG PYLON; tried to gain access on several ladders but could not get a good look at the panel. With that, I was thinking that I could get a better view and access to the area to be inspected by going over the left wing. Here my lack of awareness led me to think it was O.K. to remove the left Emergency Exit door. I removed the handle cover and did not notice the "SLIDE ARMED" light. Then proceeded to open the door and inadvertently deployed the slide. [I] reported to my Supervisor the slide deployment. [One must] think think think about what you're doing!!!!

Synopsis

A Line Mechanic reports trying to inspect a crack in a left engine pylon panel using ladders. He decided to access the A320 aircraft upper pylon by going over the left wing through the emergency exit door. He did not notice the "SLIDE ARMED" light when he removed the exit handle cover, opened exit door and slide deployed.

ACN: 921313 (2604 of 2908)

Time / Day

Date : 201012
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZJX.ARTCC
State Reference : FL

Aircraft

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent

Person : 1

Reference : 1
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Function.Air Traffic Control : Trainee
ASRS Report Number.Accession Number : 921313
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Instructor
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 921318

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Issued direct routing (DIINO) to aircraft. Used URET to input routing. URET decided to change STAR from FISEL to GISSH. I didn't notice the change. Aircraft proceed to DIINO then previous routing. Recommendation, why do we need a machine to determine a route change is needed when the one the aircraft already has is acceptable? This is a known problem at ZJX and the "powers that be" have decided to let it happen over and over. I've been told "don't put direct DIINO in the machine." This solves the problem of unnoticed route changes, however, it causes another problem. If the URET system doesn't have the actual track/route as in this case direct DIINO, then the ALERT portion of the URET system is inaccurate.

Narrative: 2

Aircraft was enroute to FLL filed via the GISSH arrival. The RADAR trainee cleared the aircraft direct DIINO intersection and made an amendment in URET. The aircraft's route was changed to DIINO.FISEL2.FLL. The Trainee nor the Trainer noticed the change and the aircraft was handed off and switched. The aircraft went over to ZMA's frequency cleared via the GISSH [arrival] and the flight plan showed the aircraft on the FISEL arrival. Recommendation, Fix URET.

Synopsis

ZJX Controller described a confused routing event allegedly caused by a unnecessary URET re-route, unnoticed by the Controller's involved. The ALERT functionality was inaccurate.

ACN: 921198 (2605 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Last 90 Days : 50
ASRS Report Number.Accession Number : 921198
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Upon arriving we were told they would be taking our aircraft and using it for a different flight and we would be getting a new aircraft. This aircraft had been sitting overnight with an electrical issue with generator #2 and multiple failures associated with it. We were told by Flight Operations the odds of it being fixed were pretty low as they were being told Maintenance was still unable to ascertain what the issue with the aircraft was and that our now delayed flight would most likely cancel. After more than an hour, our flight was canceled. We were later told that the generator control cables were replaced on the aircraft due to possible chaffing (see current operations bulletin for airbus electrical issues that might apply to this), however this was relayed to us and not verified. We dead headed back to base and did our flight planning on arrival for our next leg. The maintenance history, in a 6 day period, of the aircraft assigned showed 4 FWC (Flight Warning Computer faults, to include one that day). The fix for 3 of the write-ups were to reset Circuit Breakers, which obviously did not fix the maintenance issue, simply reset the system. The system repeatedly failed. I asked for the system to get fixed before I took the aircraft (especially since we were presently in a Maintenance Base). Maintenance and the Dispatcher agreed and Maintenance Control suggested I refuse the aircraft as this was the fastest way to get maintenance preformed. I do understand that it is policy that we do not have to refuse a aircraft to get maintenance performed, however in the interest of an on time departure and to negate any further delay in trying to get maintenance accomplished I agreed and refused the aircraft.

The mechanics came in and out of the cockpit a few times (we were standing out side as they needed to sit in our seats) and left saying they could not find a part. At about the same time we were given a gate change and new aircraft. The log history had multiple write ups regarding class 2 flight control status; multiple write ups for IR3 faults and NAV AIR 3 faults. In addition, it had a flight warning condenser fault write up. I again called Dispatch and Maintenance Control. I asked him about the aircraft and all the write-ups to make sure we were not overlooking a bigger picture issue with all these faults and questioned if they were all related in a way we needed to look at. Status write ups was nothing he was concerned about, but it did indicated that there was an issue going on that they would need to look into. I asked him if the aircraft was coming inbound clean, and he said yes. I asked the First Officer if he was good with everything and the explanations, he said yes. I told Dispatch and Maintenance Control that it sounded good to us and that we would take the aircraft. We were at the gate when the aircraft came in and met the crew. IR3 and ADR 3 faulted again, inbound. They also received a triple chime for NAV downgrade from CAT3 to CAT1 on the approach and then back up to CAT3 status. I called Dispatch and let him know the aircraft just took another hit on IR3. He asked if it was deferrable (and he was not even sure they would defer it) would we take it. I said no due to the fact it just failed yet again and the extensive history with the failure of the system. I also had concern that this had been switched back and forth from IR2 to IR3 with both having failures in the history. The same 3 mechanics came to the aircraft and worked for over an hour. They informed us they could not fix the aircraft and they stated they were not sure why the replacement part was failing. We ran out of duty time while they were still working on the aircraft.

Turning down aircraft is not something I want to do, nor should I have to refuse aircraft with histories of system failures to get maintenance accomplished. Shouldn't this be a function of Fight Operations and Maintenance? It would seem to me that part of the morning briefings in which there is a overview of the aircraft routings and the current deferrals would include looking at the maintenance history of the aircraft to see a trend in the failures, and what is likely to continue failing. Due to how these failures are written up and what maintenance chapters they are recorded in, a truly chronic issue may not been seen as chronic due to the recording of the failures. However, reading the maintenance history can clearly illuminate these failures/issues.

Synopsis

A320 Captain describes refusing two aircraft due to their extensive log histories of repeat failures. Maintenance is unable to repair the aircraft prior to the crew running out of duty time.

ACN: 920948 (2606 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 12000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : AC Generation
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 920948
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Passing through approximately 12,000 FT we began to get a series of ECAM warnings. The warnings were coming so fast we couldn't catch all of them. The first one we saw was AC 2. At this time the First Officer lost his PFD and ND. We also lost the lower ECAM. Two other ECAM's we had were associated with the rudder trim. One was Rudder Trim Limiter and I think the other one was Rudder Travel. We reset the generator as per the ECAM and then FAC 2, both reset. The only thing we had at this time was fuel used indications were both XX. These never returned to normal indications. After we finished this and talked to the Flight Attendant I mentioned to the First Officer that this sounded just like the new operations bulletin. I printed out a copy and read through the items. We had all of the indications on the bulletin with the exception of the unwanted rudder trim inputs, although we did have the rudder trim ECAM's. The bulletin referred us to FOM (Flight Operations Manual) and we got that out and read it. The First Officer noticed that a the bottom of the page it stated disconnect the generator. So we started the APU and once it was online disconnected GEN 2. We also noticed that there was a specific maintenance code, so we sent this in but it references the IDG portion of the bulletin and not the GEN. This should be noted on the page and maybe a separate code for the GEN portion of this situation. The other issue that we encountered was during taxi in. We were pulling up the gate and probably 20-30 FT from the parking spot when we lost NWS (Nose Wheel Steering). I set the parking brake and got the agent to plug in. I told him about the steering and that he would have to hook up and pull us the rest of way in. While the mechanics were trouble shooting he mentioned that they also had a memo from Airbus on this situation. He said that the NWS loss is all part of the same issue and he was surprised we made it as far as we did without losing the steering. If this is true we should be adding that note to the bulletin so that pilots are aware that they may lose steering.

Synopsis

A320 Captain experiences a generator 2 electrical malfunction that allows AC bus 2 to become unpowered. The generator is successfully reset, however a company bulletin advises that the APU should be started and the generator disconnected which is accomplished. Upon arriving at the gate NWS fails.

ACN: 920790 (2607 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Exterior Pax/Crew Door
Manufacturer : Airbus
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 920790
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 920791
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

While working an A320 aircraft for logbook [write-up], a Flight Attendant had reported door 2L difficult to arm. Flight Attendant cycled door 2L five times in order to arm it. I and another Mechanic cycled the arming lever several times and could not find any problems with arming and disarming door. We did find the door slide bustle had a missing clip and missing velcro. After reinstalling the bustle we verified the left and right aft main entrance doors were 'Disarmed'. I noticed the trim on the bottom of the bustle was folded under. I raised the door 2L handle slightly to fix the trim when the slide deployed.

Narrative: 2

Cycled arm and disarm [handle] several times in reference to logbook [write-up], found no defects. I verbally called for Door 2L and 2R doors "Disarm". Supervisor came at that moment and ask if the plane was ready. [Other] Mechanic operated the door and the slide deployed.

Synopsis

Two Line Mechanics report they were trouble shooting an A320 main cabin door 2-Left for a logbook write-up of "difficult to Arm" when the door slide deployed.

ACN: 920751 (2608 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 5000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 920751
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC
Analyst Callback : Completed

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

We were cleared the approach for Runway 19L just after GRNPA Intersection. This immediately caught our attention as it was a clear night and the terrain ahead (rising to 4,080 FT) was clearly visible. LAS Approach usually clears you for the approach later on the arrival and advises you to to turn final prior to the Stratosphere (about 3.5 mile final or so thus avoiding the terrain to the northeast). Being cleared the approach from where we were put the terrain directly between us and a five mile final.

We decided to maintain 5,000 FT until passing over the terrain, as we started the descent out of 5,000 FT though, we got GPWS terrain warning. Since we were south of the crest of the terrain we thought we'd avoid the GPWS advisory, but once we started the descent, we got the advisory. We immediately began a shallow climb and the GPWS ceased. We continued to an uneventful landing. Although we were aware of the terrain, our descent was initiated too early resulting in the GPWS advisory and momentary warning. We could have avoided any potential conflict by continuing initially toward the final further south and turning final at an appropriate altitude and distance from the runway.

Callback: 1

The reporter clarified the time of the event as being local time rather than UTC as originally provided. He confirmed that the visual approach clearance included no provisos or restrictions.

Synopsis

An A320 inbound to LAS was cleared for a visual approach to Runway 19L from the vicinity of GRNPA intersection. As they began their descent from 5,000 MSL they received an EGPWS warning.

ACN: 920653 (2609 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.TRACON : A90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Flight Phase : Takeoff / Launch
Airspace.Class B : BOS

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13305
Experience.Flight Crew.Last 90 Days : 174
Experience.Flight Crew.Type : 5472
ASRS Report Number.Accession Number : 920653
Human Factors : Fatigue
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We briefed a Flaps 2, TOGA, BLEEDS OFF take off from Runway 27. After takeoff, we noticed that we had forgotten to select the engine bleeds off. I believe two items contributed to our omitting to select the engine bleeds off as we had briefed. 1. As BOS was using Runway 27 for takeoff, I elected to do a single engine taxi to conserve fuel. Taxiing from our gate to Runway 27 involved crossing 3 runways and a taxi route that I was not very familiar with. While we remained focused on the taxi route and coordinating clearance to cross the 3 runways enroute to Runway 27, we also became distracted from our bleeds off SOP set up. As an additional distraction, the First Officer was also tasked with starting the number 2 engine while backing me up with the taxi route. 2. I also believe that fatigue was a contributing factor in our SOP omission. This was day three of a trip with a wake pattern of early, early, and earliest. Day one involved a XD:00 domicile time wake up. Day two was XD:15 and day three was a XA:45 domicile time wake up. While I felt fit to fly, I also felt tired from the very early wake up on day three. I will try to be more careful and deliberate with SOP compliance and give more consideration to minimizing distractions, especially when fatigued in the future.

Synopsis

An A320 Captain reported that the crew failed to select the engine bleeds off for a BLEEDS OFF takeoff because the First Officer was starting an engine, they were dealing with complex BOS taxi requirements, and experiencing fatigue.

ACN: 920545 (2610 of 2908)

Time / Day

Date : 201011

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Distance.Nautical Miles : 10
Altitude.AGL.Single Value : 2000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : ZZZ
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Throttle/Power Lever
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Engineer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 131
ASRS Report Number.Accession Number : 920545
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 112
Experience.Flight Crew.Type : 2357
ASRS Report Number.Accession Number : 920561

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Inflight Shutdown
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We were dispatched with MEL 22-30-01 Auto Thrust System Inoperative. On descent we noticed #2 Eng Thrust Lever Disagree ECAM message flickering on and off. We also noticed the #2 engine N1 indication fluctuate from 30-51% N1. We initially thought this was associated with the Auto Thrust inoperative MEL item. As we approached 10000ft MSL the N1 stayed about 32% and N2 about 65%. As the F/O brought thrust levers up on both engines we noticed some aircraft yaw. Engine #1 was operating normally. We moved the #2 thrust lever from idle to full climb power with no response on the engine indications and no thrust or yawing of the aircraft as we moved the #2 thrust lever. The engine seemed to have failed and was wind milling. Oil pressure was normal, yet we had NO control of the engine with the thrust lever, there were no other ECAM indications. There were no vibrations or fire or overheat indications. To avoid thrust lever problems, and since it appeared we had lost all thrust and throttle control of the #2 engine, I ran the engine #2 failure checklist in the QRH. This was about 2000ft AGL. We completed the Engine Failure and Landing checklists and landed without incident. We taxied to the gate and completed our after landing and shutdown checklists and called Maintenance to report the malfunction. There was little time (last 3-4 min of flight) to do much coordination as this happened shortly before landing.

Callback: 1

Reporter states that Maintenance discovered that the cannon plug providing electrical signals to the FADEC (J9) was severely corroded and needed to be replaced. It was also deemed likely that this plug was the source of the auto thrust anomalies. The reporter believed at the time that the engine had failed but understands now that it was at idle with no cockpit control of thrust.

Narrative: 2

My primary concern while the Captain was consulting the checklists was to not become distracted at such as close distance to the airport and a critical phase of the flight and to safely land the aircraft. The flight landed without further incident and we taxiied to the gate and notified maintenance control of the incident.

Synopsis

After being dispatched with autothrust deferred, an A320 crew discovered during descent that the number two engine was not responding to thrust lever movement. Crew continued and landed without further incident.

ACN: 920373 (2611 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aerofoil Ice System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8100
ASRS Report Number.Accession Number : 920373
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 920390
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were dispatched with a wing anti-ice valve inoperative. Part of the MEL required the crew to verify that the wing anti-ice valve was visible on the system screen prior to takeoff. We performed the check and verification. I now find out that the valve was not secured open, which means we must have confused the valve indications on our verification. Perhaps this check should be made by Maintenance prior to each departure.

Narrative: 2

When the Captain and I arrived at the gate and began looking at the papers, we saw an MEL for a system that was inoperative (wing anti-ice valve) that was ridiculously long and complicated and confusing. We discussed turning down the airplane but because of pressure from the company to fly airplanes with MEL'ed items (increased "line checks" for Captains who turn down "too many" airplanes), we decided to take it. Part of the MEL required the crew to verify that the wing anti-ice valve was visible on the system screen prior to takeoff. We performed the check and verification. I now find out that the valve was not secured open, which means we must have confused the valve indications on our verification. In my opinion the MEL required an A and P rating with an Airbus specialty. Our flight manual is not specific enough to cover these kinds of systems in detail. The wording was confusing about the indications we were supposed to see and we mistook the "arrow" the MEL was speaking about as the correct indication for the system working as it was supposed to. It required a bleeds off takeoff, us remembering after a 5 hour flight to do a bleeds off landing on top of negotiating weather during the descent and landing. I will never accept another airplane in which I am expected to "play mechanic" again.

Synopsis

An A320 crew reported that their aircraft had wing anti-ice valve MEL'ed inoperative and required to be secured in the open position. The crew determined later that the valve was not in the open position but they identified an incorrect valve as open on the system screen.

ACN: 920310 (2612 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 10300

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class E : ZZZ

Person

Reference : 1
Location Of Person.Facility : ZZZ.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Flight Data / Clearance Delivery
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 920310
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During a late night shift, an A320 was inbound via a STAR arrival. A customs aircraft had been operating in the vicinity at various altitudes during the time when four or five successive aircraft were inbound. I had to call traffic to each of the arrivals as the customs aircraft was in close proximity to the STAR. The customs aircraft was not talking to ATC. It was squawking a discrete code and I was advised it was operating with no lights. I called traffic multiple times to the A320, the last being three o'clock and three miles at one zero thousand six hundred unverified. The A320 never saw the customs aircraft which seemed to alter course and go behind the Airbus. With no Mode C verification and no communications this situation seems to be very unsafe. Customs aircraft should be RADAR identified and handed off to the facility affected even if only to monitor the frequency for traffic calls and to provide a verified Mode C. In this case, when I called the AMIC, he advised he didn't even know the type aircraft. Operating at night without aircraft lights may be a necessary mission for national security, but doing it with a lack of procedures is dangerous.

Synopsis

An Enroute Controller voiced concern regarding a Customs surveillance aircraft operating without lights and no verification regarding altitude and/or routing information.

ACN: 920161 (2613 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 20
ASRS Report Number.Accession Number : 920161
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During preflight pulled up history of the aircraft. Write up of issue with the aircraft in 3 degree left bank and excessive rudder trim requirement to keep aircraft in level flight. The aircraft [also] had 3 additional write ups regarding same issue with flight controls. I called Dispatch to refuse the aircraft based on flight control issues and inadequate maintenance answers regarding issue, and/or discovery of cause. The flight control issue clearly was not corrected. We then conferenced in with maintenance who explained that they would like us to take the aircraft from a maintenance base to another maintenance base and do some procedures in flight to give them additional information regarding the aircraft. I again refused the aircraft. We are not a [flight test] crew. As we were in operations awaiting a new aircraft, we spoke to the crew that flew the aircraft inbound. They explained that on takeoff the aircraft rolled right, right after takeoff and what they did to keep wings level. They also explained the previous maintenance regarding the write up they had in checking the fuel tanks for balance. We were then told by both the crew and the flight managers that spoke to us regarding the aircraft, that when they landed there was talk of a [test] crew flying the aircraft. It was then discovered the aircraft was assigned to a flight prior to being assigned to our flight and the Captain refused the aircraft. The aircraft was then assigned to our flight after the previous Captain refused the aircraft. We were assigned another aircraft within the hour. I do not have any further information on aircraft. Flight Managers spoke to me directly and stated that they also refused the aircraft to fly based on the maintenance history and that they felt the issue [would have to be] corrected before it flew again.

Synopsis

A320 Captain reports refusing an aircraft which has been written up several times for flight control issues with no corrective action by Maintenance. The aircraft had been refused just prior to being assigned to the reporter.

ACN: 920128 (2614 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA
Altitude.MSL.Single Value : 24400

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Airspace.Class A : ZDC

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZDC
Aircraft Operator : Fractional
Make Model Name : Cessna Citation Sovereign (C680)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class A : ZDC

Person : 1

Reference : 1
Location Of Person.Facility : ZDC.ARTCC
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 920128
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 4100
ASRS Report Number.Accession Number : 920460

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 920464

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Human Factors

Narrative: 1

I climbed an A320 to FL230. During read back there was a loud horn sound from the flight deck. There was D-Side training in progress, all 3 of us missed the bad read back to climb to FL260. Sector 18 Controller stopped a C680 descent to FL260, TEB arrival. When conflict alert activated, I began a transmission to instruct the A320 to maintain FL230, but changed to say altitude as I was trying to get situational awareness on the C680 altitude. Pilot responded we are in a RA descent, we were read back FL260. Recommendation, new rule for me, if there is excessive noise (alerters, horns etc.) I am going to verify instructions again. We also at the time had a report of unauthorized laser activity and the broadcasts for that were really increasing my workload. Must remain vigilant.

Narrative: 2

At FL210, we were given an airspeed, heading and altitude change. The First Officer read back an altitude change from FL210 to FL260, the OMC, another airline pilot, heard him read back FL260 to ATC and we started our climb. Passing FL240 we got a "DESCEND"; RA which we followed and advised ATC. Washington asked what we were doing and that our assigned altitude was FL230. We replied that we were issued FL260. He replied to descend and maintain FL230. No further reply from Washington Center.

Narrative: 3

Climb out of IAD. Level at 10,000 FT, given climb to FL210, expect light to moderate turbulence on climb. Captain elected MCT thrust, 250 KTS and open climb. At FL210 the aircraft began to level off, but thrust still at MCT, so airplane accelerated to 280 KTS. At this time the Captain selected V/S +1,600, and because the thrust was at MCT, the plane started to climb. I mentioned that FL210 was our assigned altitude, at which time the Captain kicked off the autopilot and began to correct back to FL210. At this time, of course, ATC gave what I heard as "A320, turn 20 degrees right, climb to FL260, speed 290 KTS". I read this back and adjusted the MCP for these instructions. The radios were very busy at this time, constant chatter. Somewhere around FL230 we received a TCAS TA followed shortly by an RA to descend. The Captain complied and started a descent. I told ATC we were in a TCAS descent and he replied that we were assigned FL230. I stated that I read back FL260, which was confirmed by the Captain and our jump seater. It was very busy on the radios and the distraction of our FL210 altitude mess up may have caused me to hear the wrong altitude. Only the tapes can tell you that. The flight continued from that point normally and ATC made no further mention of the incident.

Synopsis

ZDC Controller described a TCAS RA event when one Air Carrier was issued a climb to FL230, read back FL260, but was missed, the reporter alleging cockpit background noise on the frequency contributed to the read back error.

ACN: 919857 (2615 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Weather Radar
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : GPWS
Manufacturer : Honeywell
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Qualification.Maintenance : Airframe
Qualification.Maintenance : Avionics
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 919857
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Other
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : MEL

Narrative: 1

MEL 34-41a, MEL 34-41b request Weather Radar circuit breaker to be pulled before flight. However, when the C/B is tripped and collared, about five seconds later the Terrain fault light illuminates in the overhead Terrain switch. The MEL does not direct Maintenance to address this "induced" light fault. Neither does the MEL inform the flight crew in their operations placard on the Maintenance Release that there will be a Terrain fault displayed. The Terrain function is truly faulted at this time because there is no response to any preflight requests by the crew or (Centralized Fault Display System) CFDS test by Maintenance. This same flight deck effect happened on another A320 and was simulated on two other A320s.

Callback: 1

Reporter stated the TERR (Terrain) test pattern would not appear on the Navigation Display (ND) after the Weather Radar C/B was pulled and collared. When the TERR (Terrain) switch is pushed in (On position), the TERR fault light and the TERR white "Off" light would normally would be out. But after they pull the Weather Radar (W/R) Circuit Breaker, the TERR fault light comes on. That MEL procedure causes an induced fault without any information noted on the Maintenance Release for the flight crew to be aware of. 

Reporter stated if Maintenance doesn't get a test pattern for the TERR display on the ground, he doubts the flight crew will get one in flight. At this point, no one is clear what effects, if any, pulling the W/R C/B has on the "Below Glide Slope" fault or Reactive Windshear Warning part of the EGPWS; or whether the TERR fault light only indicates a system the EGPWS uses for information has failed, but the EGPWS is still fully functional.

Synopsis

A Line Mechanic reports about an MEL Chapter 34-41 deferral procedure that requires pulling and collaring the Weather Radar circuit breaker on an A320 aircraft. Approximately five seconds later the TERR (Terrain) fault light illuminates in the overhead TERR switch. No reference in the MEL about the fault light or potential effects on the EGPWS system.

ACN: 919760 (2616 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Main Gear Tire
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 919760
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

During preflight the left outboard tire looked like it had low air pressure. I called it in and a mechanic confirmed it had 70 PSI of air. This tire did not have a gage. I am certain it had flown several days in this condition as there was a write up nearly a month ago regarding low tire pressure. It looks as though the wrong tire was checked and signed off. This is the second tire I have found with pressure less than 110 PSI and no gages installed that has flown at least one leg in this condition.

Synopsis

An A320 pilot reported a tire with a low air pressure. Maintenance found the pressure to be 70 PSI. The reporter suspects this aircraft had flown some time in this condition.

ACN: 919714 (2617 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 7500
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 175
ASRS Report Number.Accession Number : 919714
Human Factors : Fatigue

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

This is a fatigue report. I had awakened at very early [hour] and commuted down to my base to be in position for my morning short call assignment. I had been at the airport all day long and had been fighting a cold. In the evening, I received a phone call from the crew desk saying they had a late night departure that I was legal for. I was already exhausted from being at the airport all day and did not expect to be called that late into my duty period, that anything after that should have gone to the afternoon short call pilot. I told him I was not fit to fly that late after being on duty for that long and the fatigue I was experiencing. I would have been departing 12 hours and 41 minutes after I started on duty.

Synopsis

An A320 pilot on reserve duty in the morning was called for a trip departing late in the evening and because of illness and fatigue refused the trip.

ACN: 919596 (2618 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Parking Brake
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 919596
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Other
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Amber alert. Loss of Yellow Hydraulic System pressure and quantity, thirty miles out, inbound to ZZZ. Found O-ring eroded on the union attached to the parking brake valve. (Illustrated Parts Catalog) IPC 32-45-03-3b, Item #230. I have the damaged packing, it looks like the material the O-ring was made of is not compatible with Skydrol; O-ring just disintegrated; Part Number NAS-1612-8.

Synopsis

A Line Mechanic reports finding a deteriorated O-ring at the union attached to the Parking Brake valve after the inbound A320 aircraft loss Yellow Hydraulic System pressure and quantity. O-ring appeared to be made of material not compatible with Skydrol type hydraulic fluid.

ACN: 919305 (2619 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : N90.TRACON
State Reference : NY
Altitude.MSL.Single Value : 5000

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : EWR

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 275
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 919305
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

After hand off to New York TRACON we were at 5000' and taking weather avoidance vectors at the "Broadway" fix area for approach to 22L at EWR. We were given a heading of 90 degrees, which was into the center of a cell. I stated "unable", due to weather and that we would need 100 degrees for a heading. The controller then stated that "I know it is your job to fly the airplane", and that "airplanes have been coming through there all night and I have 500 airplanes in the area." He then said he needed a heading of 090 and asked what I needed, I said, "100", and he then said, "turn right 150", and from there he vectored us out to a position 40 miles West of EWR while he sequenced a series of what he called, "slow movers", in front of us. We were sequenced in for approach about 30 minutes later while pounding through rain showers at 5000 feet the whole time. We thought he would re sequence us within a few minutes, but after 20 minutes we realized that we must have been given a low priority. During this time he sent a Air Carrier X and a Air Carrier Y out of the pattern for the same reason. When Air Carrier Y asked how long it would be, he said, "about 10 minutes", Air Carrier Y replied that he was tight on fuel, and NY TRACON replied that he really "Didn't have time to give him the attention he needed right now. " Frankly, I was surprised at the attitude of the controller about cells we were painting. I was also surprised to be placed in a "holding area" with no idea of an EFC, while other aircraft appeared to be continuing the approach. If aircraft are to be placed in a holding area because they can't take a vector through active weather, they should be given some idea of the amount of time they will be there, for planning purposes.

Synopsis

Air Carrier Captain expressed concern regarding the ATC handling provided by N90 noting the controller's attitude and absence of an EFC when weather deviations were necessary and requested.

ACN: 919299 (2620 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 850
ASRS Report Number.Accession Number : 919299
Human Factors : Situational Awareness
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 919309
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Ground Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Landed on the right runway, exited Tango to hold short of the left runway. Shut down engine #2 and held 3 minutes between runways. When cleared to cross the left runway released parking brake, brake pressure 0, and added power on #1 engine. Airplane veered left and skidded. Felt like left brake was grabbing. I stopped airplane, set parking brake. Airplane was over the hold short line for the left runway. Tower informed of the problem. Aircraft on short final was sent around. Recleared to cross the left runway, released parking brake, added power and operations [were] normal. Taxied to gate with no further problems. Maintenance notified.

Synopsis

After landing and holding between runways waiting taxi clearance to the gate, the Captain released the A320's brakes and the aircraft veered and skidded sharply left because of a severely grabbing brake. The brake released after the parking brake was set and released again.

ACN: 919253 (2621 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.AGL.Single Value : 700

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Route In Use.Other 
Airspace.Class B : LAX

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : LAX
Aircraft Operator : Air Carrier
Make Model Name : B737-800
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Route In Use.Other 
Airspace.Class B : LAX

Person : 1

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 919253
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : LAX.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 919462
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

IFR, tail wind, we were running staggers. These were the first aircraft I worked. I was warned by the previous controller that there was a tailwind. I slowed both an A320 and a B737 to 220 KTS about 30 to 37 miles from the airport. The A320 was going to follow a heavy so I slowed the A320 to 180 KTS at about 10 miles in trail to the heavy. The A320 seemed to slow normally, as if the tailwind was not that bad, so I was going to let the B737 close up on the A320. I got distracted, and let the B737 get too close, so I slowed the B737 to 160 KTS. I then told the B737 that he was gaining on the A320 ahead and to slow to final. It looked like it was going to work so I switched the B737 to the Tower but then the A320 slowed to about 130 KTS inside the marker. I was using the bat. The Tower called and said they could not provide visual but at that time it still looked like 2.5 to me. I then gave the Tower instructions to break the B737 out for re-sequencing. Recommendation, I should of stopped the B737 approach and had him go around.

Narrative: 2

While training on the LC1 position, a situation developed between an A320 and a trailing aircraft (B737). Separation on final was collapsing between these two arrival aircraft. The Parallel Monitor had just been opened. The Parallel Monitor contacted the Local Assist (JA) and asked the Controller if "visual separation" could be applied between these to aircraft. The Local Assist repeated the question, and the Parallel Monitor Controller assumed that the Local Assist had accepted separation responsibility. The Local Assist called the Parallel Monitor back and advised that visual separation could not be utilized. The Parallel Monitor instructed the Local Assist to cancel the approach of the B737, climb them to 3,500 FT, and break them out "now". Since the Parallel Monitor airspace continues until the runway threshold, I believed that the Parallel Monitor was responsible for all separation of the B737, including MVA requirements. Had the Parallel Monitor not stated, "Break them out now," I would have waited to turn them at a later point. The normal protocol is for the Parallel Monitor to take over the frequency and broadcast control instructions directly to the aircraft. Why this Controller broke the protocol in unknown, but is the major cause in the Operation Deviation/Error. Recommendation, ensure that the Parallel Monitor confirm their responsibility of control to arrival aircraft during simultaneous approaches by being the only one to broadcast control instructions directly to aircraft to prevent or resolve loss of separation, as contained in the Letter of Agreement between LAX Tower and SCT.

Synopsis

SCT Controller described a go around event caused by an overtake situation on final reportedly due to tail winds and flawed spacing judgement/s.

ACN: 919238 (2622 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ
Altitude.MSL.Single Value : 8000

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Facility : P50.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 919238
Human Factors : Situational Awareness

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An A320 was vectored off of the KOOLY2 RNAV arrival for spacing and issued 8,000. I handed him off to finals (VERDE) very early and accidentally turned my Data Block to the same color as my primaries and limited Data Blocks. In doing so, I overlooked the target until prompted by the pilot. I then issued a point out to PIMA and SANTAN but had already violated PIMA's airspace. There were no other aircraft within several miles (IFR or VFR). As I could not turn the aircraft back to the original color white, stars not capable, I should have put a J-Ring or a Leader Line to remind myself but did not and disregarded the aircraft. I will personally pay better attention.

Synopsis

P50 Controller described a airspace incursion resulting from an early hand off to the next sector and an inadvertent Data Block color adjustment, overlooking the developing airspace problem.

ACN: 919126 (2623 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 11000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : ZZZ

Component

Aircraft Component : Leading Edge Slat
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 919126
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 919128

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : FLC complied w / Automation / Advisory
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During vectors to final approach pilot flying called for Flaps 1. The slats tracked almost to position 1 and we received an ECAM F/CTL SLATS LOCK..WING TIP BRAKE ON. The ECAM was followed, Flight Manual procedures and maneuvers reviewed, conference call to Maintenance and Dispatch made, flight attendants were briefed to perform a cabin advisory, and passengers were informed of our minor malfunction. An emergency was declared and an uneventful landing with flaps 3 made. We selected the longest runway. Taxi to gate was made without any problems. All agencies provided excellent and professional assistance.

Synopsis

An A320 ECAM alerted F/CTL SLATS LOCK..WING TIP BRAKE ON after flap position 1 selected. An emergency was declared, the QRH procedure completed and discussions held with Maintenance and Dispatch. An uneventful landing followed.

ACN: 919112 (2624 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11900
Experience.Flight Crew.Last 90 Days : 195
Experience.Flight Crew.Type : 3675
ASRS Report Number.Accession Number : 919112
Human Factors : Situational Awareness
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 6900
ASRS Report Number.Accession Number : 919127
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Requested ATC Assistance / Clarification

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

As pilot flying the landing was normal until time to exit runway. An ECAM "nose wheel steering fault" was noticed prior to exiting runway. Used differential braking to exit runway on high speed taxiway. Asked pilot not flying to try steering tiller and [it was] no good. Held short of the next runway until cleared by Tower. After exiting clear of that runway used differential braking to make a right turn on to taxiway. Very hard getting the aircraft to turn. After straightening out, used small turns with brakes to get to a holding area. Stopped aircraft in a holding area and shutdown engines to wait for tow to gate. Brake temp were 385 C on the right and 150 C on the left.

Narrative: 2

Loss of nose wheel steering (NWS) during landing rollout. First Officer was pilot not flying on first flight of day. Aircraft was on first flight after "C" check. Flight proceeded as normal during preflight, ground operations, departure, enroute, decent and initial approach to landing. Upon landing rollout on the runway and shortly after spoiler deployment and reverser application and just prior to turning off to a high speed taxiway, received a NWS STEERING FAIL ECAM. Captain maintained aircraft control using differential braking and brought aircraft to stop in a normal manner prior to the next runway in accordance with Tower instructions. Received clearance to cross that runway, contacted Ground and informed Ground of the issue. Received clearance to taxi as able to holding pad. This was accomplished using differential braking. Both tillers were used to see if control of the NWS could be obtained to no affect. Aircraft stopped and NWS switch cycled to no affect. Aircraft parked and parking brake operated normal. Airport Operations informed and a tow in was coordinated with Company Operations and Ground Control. Aircraft tow crew arrived, hooked up tug in accordance with tow in checklist, and aircraft towed to gate with no other issues. Aircraft turned over to Maintenance upon arrival at gate.

Synopsis

An A320's nose wheel steering failed after landing but the Captain was able steer the aircraft to a holding area using differential braking, and subsequently the aircraft was towed to the gate.

ACN: 918827 (2625 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Relative Position.Angle.Radial : 270
Relative Position.Distance.Nautical Miles : 150
Altitude.MSL.Single Value : 33000

Environment

Weather Elements / Visibility.Visibility : 100
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 918827

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Total loss of yellow hydraulic system, leaving the green system and the emergency blue system. We declared an emergency and diverted to the nearest airport and landed uneventfully. The loss was due to a seal leaking in the wheel well. ATC was very helpful and got us on the ground without any delay. Everyone was professional, this was just one of those things that happen.

Synopsis

A320 First Officer experiences Yellow Hydraulic system failure at FL330. The crew declares an emergency and elects to divert for maintenance.

ACN: 918766 (2626 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Fan Reverser
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 918766
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After completing a #3 Service Check on an A320 aircraft, by doing the fuselage, tail and wheel well walk- around; signed for completion of #3 Service. Performed pre-taxi walk-around prior to aircraft delivery to gate for outbound flight. During walk-around, I did visually inspect #1 engine inlet and exhaust prior to boarding aircraft for delivery taxi to gate as a right seat taxi Mechanic.

After delivery to gate, [we] de-boarded and while my left seat driver performed the post taxi walk-around at gate, I signed-off the pre-departure and aircraft Logbook Release. We departed the gate and returned back to company Maintenance Base. Shortly [thereafter we] received a call from the gate to return to assist in retrieving a tool (flat tip screwdriver) found by the outbound crew copilot during his pre-departure walk-around. The tool was located at the 6:00 o'clock position, right-hand side of the fan exit strut, at the intersection between the fan case and right-hand reverser. As viewed from front of engine, [screw driver was] behind fan blades and the fan exit guide vanes. Mechanics on gate opened right-hand reverser cowl, retrieved tool, inspected area, found minor crack and scratches on rubber seal at 6:00 o'clock position on the fan exit strut. Deferred the item and aircraft released for departure.

Synopsis

A Line Mechanic reports he taxied an A320 to the gate and signed-off a post taxi walk-around accomplished by another Mechanic. He was later informed that an outbound First Officer had found a flat tip screw driver at the 6:00 o'clock position on the #1 engine right-hand fan exit strut, during his walk-around.

ACN: 918600 (2627 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 918600

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Bird strike on initial climbout - saw approximately 6-8 geese in just an instant before they appeared to go below the right side of the aircraft nose - I thought we would pass above them. A "pop" followed by vibration and a high whine followed. N1 vibration indication on number 2 engine went to 9.9. There was no time to react from the time I saw the birds to the time they passed the aircraft. It was very quick. Declared an emergency with ATC and requested return to field. [We were] unable to determine if any other damage occurred - other than the vibration in the number 2 engine. Leveled off and turned back to field - once power reduced - vibration reduced to a "non-advisory" level (not flashing). QRH for high vibration stated engine could be left running with vibration below advisory limit (non-flashing below 6.0). [We] did not shutdown engine. Advised Operations - asked them to call Dispatch. Advised flight attendants and made announcement to passengers. [We had a] normal landing with emergency equipment standing by as a precaution. [We] taxied to the gate. At gate arrival [we] determined engine damage [had occurred] to first stage fan blades. No other damage was visible, but not sure if there was another bird strike to any other part of the aircraft. Maintenance said they would inspect - logbook entry made.

Synopsis

An A320 experienced a bird strike on climb that resulted in high right engine vibration. The flight crew reduced right engine thrust to idle, declared an emergency, and returned to the departure airport.

ACN: 918265 (2628 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 23000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 5800
ASRS Report Number.Accession Number : 918265
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : ATC

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 2900
ASRS Report Number.Accession Number : 918272
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We lost the #1 Engine Hydraulic Pump (Green system) during climbout. The system was secured per the ECAM and the PTU pressurized the green hydraulic system.

We advised, Dispatch, Maintenance Control and the flight attendants of the situation. Told Dispatch we were not declaring an emergency and would do a normal landing and taxi in. After switching to Center, they asked for FOB (fuel on board) and SOB (souls on board). Center said that someone at company declared the emergency for us.

The Captain made a PA explaining what our situation was and how it would be a normal landing. The landing and taxi in was uneventful except we were met by the CFR. We tried to have the Tower and the CFR cancel the emergency, but nobody would. The Captain talked to the CFR and they left.

As a pilot, I really thought it was ridiculous how after telling Dispatch we were not declaring an emergency, someone on the ground could do that for us. This totally blindsides the crew while they are flying. My flight's events highlight the fact that only the crew flying the aircraft should be able to declare an emergency. We advised Dispatch clearly that we were fine and not declaring an emergency, but our informed and educated PROFESSIONAL opinion and decision was disregarded by someone sitting on the ground who is not the A320 expert. This policy is not smart. It needs to be changed.

Narrative: 2

After switching to Center, was asked for our fuel on board and souls on board. I asked who prompted that, they said the company. I told them that we were NOT an emergency aircraft.

Synopsis

Following the loss of the left engine driven hydraulic pump during climb out, the flight crew of an A320 was distressed to learn that an emergency had been declared for them upon arrival at their destination.

ACN: 917933 (2629 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZJX.ARTCC
State Reference : FL

Aircraft

Reference : X
ATC / Advisory.Center : ZJX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : GISSH

Person : 1

Reference : 1
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Function.Air Traffic Control : Trainee
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 917933
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Facility : ZJX.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Instructor
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 917938

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Training on sector; issued direct routing to DINNO on FISEL and GISSH STAR to FLL, URET changed route from GISSH to FISEL. Automation is the problem here, URET takes over and changes the route automatically. Controllers know the prefered routes. Allow Controller to decide which route the aircraft needs.

Narrative: 2

Aircraft filed GISSH arrival to FLL. We cleared him direct DIINO and put it into URET. URET changed the arrival to FISEL and reflected that in his flight plan without my knowledge. Aircraft took a wrong turn somewhere downstream. This is a known automation issue and needs to be fixed immediately. URET should not be "preloading" routes for the Controllers. It is a tool not an assistant. URET should keep whatever route the aircraft is on still preloaded. Let the Controller decide if it should be changed.

Synopsis

ZJX Controllers voiced concern regarding the functionality of URET, claiming route changes should only be made by controllers not automation.

ACN: 917489 (2630 of 2908)

Time / Day

Date : 201010

Place

Altitude.MSL.Single Value : 4000

Environment

Weather Elements / Visibility : Fog

Aircraft : 1

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach

Aircraft : 2

Reference : Y
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Crew Size.Number Of Crew : 1

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 917489
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Conflict : NMAC
Detector.Automation : Aircraft RA
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Miss Distance.Horizontal : 0
Miss Distance.Vertical : 300
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Shortly after being cleared for a visual approach on a left base leg and out of 5,500 FT the Controller came back with an instruction to maintain 4,000 FT. I immediately stopped my decent and leveled off at 4,000 FT. Then I got a TCAS RA to CLIMB which I followed as the First Officer pointed out the traffic at our 10 o'clock and slightly beneath us. Then I saw a small white T-Tail private aircraft pass directly underneath us by about 300 FT. Then discovered he was at 4,000 FT by the Controller reprimanding him for not being at his assigned 3,500 FT. The TCAS saved our lives because we were able to climb to about 4,500 FT before CPA. Had we of just stopped at 4,000 FT we would have been on collision course.

Synopsis

An A320 Captain reported a 300 FT NMAC with a light aircraft on a visual approach when the other aircraft did not maintain its assigned altitude and ATC detected that aircraft's deviation until just prior to the conflict.

ACN: 917479 (2631 of 2908)

Time / Day

Date : 201011
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : L30
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : LAS

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 917479
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Overshoot
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

We departed LAS Runway 25R on the SHEAD 6 RNAV departure. At approximately 4,000 FT MSL, the Departure Controller cleared us to "climb to FL190, comply with restrictions." We selected 19,000 in the altitude window and CLB for vertical navigation. After passing ROPPR (at or below 7,000 FT), I noticed that that the SHEAD 6 had a few new points, and new terminology when compared to the SHEAD 5 departure. I asked the departure controller to clarify the new altitude restrictions, showing both "at or above" and "(ATC)" at the next several points. (These changes in the depiction and charting terminology were new to me.) The Controller replied "We don't really know why they changed the charts, but it appears that the new altitude restrictions are something for you?" During this discussion, the First Officer requested open climb. (This was a typical request on the SHEAD 5, before the new restrictions were added). I then asked the Controller to clarify what altitude restrictions he expected us to comply with. He confirmed that we should comply with the altitudes which were depicted as (ATC) altitudes. At this time we were approaching MDDOG passing 9,000 FT and eventually crossed MDDOG at approximately 9,500 FT (500 FT high). The remainder of the SID and flight went as published.

Synopsis

An A320 Captain flying the LAS SHEAD 6 RNAV Departure queried ATC about the new waypoints and ATC altitudes shown. ATC did not know the reason for them but they are the altitudes the crew was expected to comply with.

ACN: 916888 (2632 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Gear Down Lock
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 916888

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were on about a 4 mile final and when I selected gear down we got a L/G GEAR NOT DOWN LOCKED ECAM message. We went around and began our procedures. We complied with the ECAM and did a landing gear gravity extension which was successful. We called ahead to Maintenance to have a tug tow us to our gate and after landing we exited a high speed taxiway and stopped. The tug was there right away and towed us to the gate. There was an aircraft swap and we continued on with our original pairing.

Synopsis

A320 Captain experienced a GEAR NOT DOWN LOCKED ECAM message when gear is extended during approach. Gravity gear extension procedures are employed successfully in compliance with ECAM actions and a normal landing ensued.

ACN: 916742 (2633 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3200

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 916742

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

During approach to [runway] XXR the flaps would not extend to position two. We advised ATC, discontinued the approach, ran the ECAM checklist and explored the situation. We then told ATC we could accept an approach to the longest runway and as a precaution asked for equipment to stand by. We declared an emergency and advised we would anticipate a normal landing and rollout. We advised the Flight Attendants and landed without incident.

Synopsis

An A320 Flight Crew followed ECAM procedures and landed safely after declaring an emergency due to the loss of flap control at less than a landing flap setting.

ACN: 916707 (2634 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 8900
ASRS Report Number.Accession Number : 916707
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

During the pushback, the push crew gave us the command, "cleared to start 1 only" and later "cleared on 2". This is the SOP at this airport. Every time I query about the lack of standardization, they simply tell me that's what they were taught. When the push crew asked us to set brakes I did so but the indications on the brake indicators were not normal. Pressure was low and the accumulator appeared flat (which was found to be the case). Flight deck said to the push crew, "Brakes set, pressure NOT normal. Stay connected to the plane".

At that point, we had to make a determination of whether the brakes would operate and determine the nature of the maintenance issue. To determine the nature of the issue, we wanted to release and reset the brakes again to verify the problem. We called down to the ground crew, and asked if he was still connected, to which he said, "yes". We told him we had a brake issue and wanted to test the brakes with him connected. He said he was ready, and we released the brakes, pumped them and the plane started to roll. The tug was not connected to the plane! We stopped and reset the brakes and did not strike anything. As we reset the brakes there was almost no accumulator pressure, and the brake pressure was low.

The tug was re-connected, and we were towed back into the gate. The push crew was instructed to chock the aircraft and leave the tug hooked up. As we went downstairs during the maintenance cycle, we found the aircraft neither chocked or hooked up to the tug with maintenance working in the main wheel well with the door down. The aircraft was chocked shortly after.

This should not be dismissed as a simple miscommunication. It was discussed with the push crew over the interphone and he stated he misinterpreted the words. Words such as: "BRAKE PRESSURE NOT NORMAL", and "STAY CONNECTED, WE HAVE A BRAKE PROBLEM". There are times all of us don't really listen, but this time there were just too many cues. And certainly a responsibility to resolve ambiguity. I believe I could have told the push driver, "Brakes set, I just kidnapped your children and sold them to Gypsies" and he would have disconnected and saluted me away normally. Clearly, someone did not hear, after multiple cues, and should not be pushing back aircraft. The Supervisor was spoken with in regard to this incident, and could not determine who exactly pushed the aircraft. He understood the concerns and seriousness of the event, and said there would be some follow up. Literally, someone was asleep at the wheel today, and we are fortunate nothing bad happened, because many errors were made.

Synopsis

The A320's push back crew failed to listen critically to the flight crew's transmissions regarding abnormal brake pressure and disconnected the tug despite being directed not to do so. A subsequent test of the brakes allowed the aircraft to move forward because of the lack of restraint.

ACN: 916420 (2635 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Toilet Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 916420

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

About 1 hour 20 minutes into the flight the C Flight Attendant reported a burning smell coming from the water heater in the aft right side lavatory, NO SMOKE OR FIRE associated with the odor. I directed the pilot jump seater to check it out. He confirmed it was the water heater and turned it off. The odor then stopped. After following the QRH we contacted Maintenance Control via Dispatch and verified the water heater was turned off correctly. With 15 minutes remaining until top of descent I continued to destination. At NO TIME DURING THIS EVENT WAS THERE ANY SMOKE OR FUMES IN THE FLIGHT DECK OR CABIN AND NO ECAM OR LAV SMOKE DETECTOR WARNINGS. After landing we MEL'ed the water heater and flew the next leg.

Synopsis

A320 Captain is informed by the C Flight Attendant that a burning smell is coming from the water heater in an aft lavatory. A jump seating pilot confirmed it was the water heater and turned it off. After conferring with Maintenance the flight continues to destination.

ACN: 916325 (2636 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 34000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAU

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 916325
Human Factors : Situational Awareness
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Shortly after takeoff from ORD we were assigned a heading of 275. After two frequency changes and multiple climb clearances we reached our cruise altitude of FL340. I wanted to make sure our current Controller knew we were still on an assigned heading of 275. I asked if she still wanted us on the heading. She said yes but you can have direct if you want a short cut. I said yes, whatever will work. I'm just checking because I'm not used to being kept on a heading this long. I was then assigned a reroute. At this point a male voice asked if I had time for a question. I said go ahead. He asked "Have you flown out here before?" I stated I had. He then very sarcastically replied "Well then you have been kept on a heading this long before." My response was an exaggerated, "Well excuse me!" The fact is I have been left on a heading on that departure and forgotten about numerous times. I didn't like his condescending and confrontational demeanor. I asked for a phone number to contact him. He gave me a number. I then asked for his initials twice. He kept silent. Several hours later I called the number. I spoke to a Supervisor and he explained to me that he thought he knew who it was and that training was going on. The male voice was the instructor. I am reporting this because I do not think he displayed a positive role model for his new student in training. I attempted to enhance communications. He attempted to disparage communications. It is lack of communication between pilots and ATC that lead to disaster.

Synopsis

A320 pilot reports being left on a heading after departing ORD up to cruise altitude. When ATC is queried about the heading, the Training Controller provides a sarcastic answer.

ACN: 916313 (2637 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 11000

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Type : 1170
ASRS Report Number.Accession Number : 916313

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 440
ASRS Report Number.Accession Number : 916324

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Ambiguous

Narrative: 1

During preflight, noticed air conditioning cart and fuel truck operating near aircarft while performing walk-around. Minutes before push back Avionics Smoke ECAM illuminated and smoke light on overhead panel as well. After fuel truck and air conditioning cart removed, ECAM warnings cleared out and showed normal. Takeoff was normal and performed by First Officer. After passing about 10,000 FT Avionics Smoke ECAM and Land ASAP illuminated. Notified ATC and First Officer took over radio communications and flew never getting any further than about 30 NM from the airfield. Captain ran ECAM, talked to Flight Attendants about possible smoke and had them run Cabin Advisory checklist. He then ran normal checklists, sent Dispatch a quick ACARS message and set up the box for an opposite direction ILS. First Officer landed the aircraft normally with no irregularities. Aircraft was stopped short of the gate and had Fire Department check for any sign of smoke - there was none. Returned to gate and contacted Maintenance. Flight Attendants commented that they noticed peculiar odor on take-off in the back of the aircraft. Aircraft was inspected, signed-off, re-boarded and flew to destination without incident. Neither Captain, First Officer, or Flight Attendants had any issue with continuing on.

Narrative: 2

After landing, aircraft was stopped short of gate while Fire Rescue checked aircraft for smoke. After confirmation of no smoke, aircraft was taxied to gate and passengers deplaned. Maintenance notified and Avionics E and E compartments and cargo holds were opened with Fire Rescue standing by. No evidence of any smoke. Flight Attendants reported strange odor in aft cabin after landing. Maintenance checked aircraft and eventually cleared for flight. Uneventful flight followed after all parties briefed on maintenance actions.

Synopsis

A320 flight crew experiences Avionics Smoke ECAM at the gate from ground equipment exhaust fumes. After takeoff the warning reappears passing 10,000 FT and crew returns to departure airport after declaring an emergency.

ACN: 916097 (2638 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Weather Elements / Visibility : Turbulence
Weather Elements / Visibility : Windshear
Ceiling.Single Value : 3500

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Check Pilot
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9600
ASRS Report Number.Accession Number : 916097
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 7000
Experience.Flight Crew.Last 90 Days : 21
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 916714
Human Factors : Workload
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

Forecasts winds [at destination] were 25038G44 all day. Ceilings at 3,500 AGL with good visibility.

Moderate to severe turbulence started around 3,000 MSL about 20 miles out on final to Runway 22. When we contacted Tower for landing clearance he advised "previous aircraft reported plus and minus 30 KTS on final, windshear reports at 1,000 FT, several aircraft have gone around, you are cleared to land Runway 22". Had I been aware that these conditions had been in effect for hours and were forecast for hours more I would never have departed. Dispatch never said a word.

Our alternate had similar conditions. We discussed windshear preventive measures and a flaps 3 approach and landing was flown. Airspeed was all over the place. We experienced a loss of 40 KTS at one time but the aircraft recovered back to mini GS. Most of the final approach was plus or minus 30 but never less then 150 KIAS. At 1,000 FT we did get the "windshear" warning four times but it stopped and again IAS never got below 150. the turbulence was mod [with] a few severe jolts. We also had the 20 KTS gain below 1,000 FT. Just prior to touchdown tower reported winds at 26038G47. We touched down about 1,500 FT down the runway and slowed very quickly in about 3,000 FT to taxi speed.

Needless to say this was an unexpected end to an unplanned reassignment with no fore-warning. Until the Tower issued our clearance we had no idea these were the conditions at our destination. We had several high probability and a few medium probability windshear indications and yet ATC Controllers were allowing flights to land and takeoff. We do not train for planned windshear approaches and landings. However, at no time did we allow our airspeed to reach an unsafe state nor did we feel the aircraft was in any dangerous or unrecoverable position. This is one of those real world events that we had never encountered. It seems that Dispatch needs to notify crews in advance of current field conditions like these and that perhaps we need to train our pilots for extreme wind events like these.

Narrative: 2

Did normal descent planning and briefed for a LOC (Managed/Selected) approach. Did not brief wind shear due to ATIS not reporting it. Airport landing Runway 22 due to winds reported at approximately 240/34G47. Vectored by Approach for a LOC to Runway 22 due to a ragged ceiling. Winds on the descent were 15 to 20 KTS. Configured as per brief and used Flaps 3. Configured for the approach about 1.5 miles prior to the FAF. Tower reported that airplanes prior were reporting +25 to -5 KTS on final. Winds became stronger the lower the descent, but the steady state winds and gusts stayed fairly constant. Airplanes were landing at an interval to accommodate takeoffs and landings for single runway operation. Airplanes before us were not reporting any wind shear, but were reporting airspeed gains and some losses after landing.

The approach became a visual below the ragged ceiling and before the FAF. Captain was descending on the PAPI and localizer course. The VVI fluctuated between 700 and 800 FPM down as we were using a briefed 3 degree glideslope. The airspeed was fluctuating with the mini GS, but not more than -5 to +10 KTS. A quick 5 KT drop in airspeed coupled with a momentary increase in VVI set off the FAC windshear warning. The warning came as a surprise because the airplane was fairly steady given the winds and no wind shear had been reported by the prior landing airplane.

Long discussion on the ground about the weather conditions and how the weather was affecting the approach, especially that the weather was strong enough to cause a single runway operation. The weather and runway configuration situation were not given the significance that it deserved in the flight papers. I was wrongly under the impression that the weather had moved on and we were delayed because of the volume of traffic, not by single runways operations and continuing winds.

The FOM states that in every case it is the responsibility of the pilot flying to assess the situation and use sound judgment in determining the safest course of action. The safest course of action seemed to be, in this situation, to continue the approach and land.

Synopsis

Surprised to learn there was significant windshear at their destination, an A320 Flight Crew chose to land regardless because their alternate was no better.

ACN: 915803 (2639 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Compressor Bleed Valve
Manufacturer : International Aero Engines IAE
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pneumatic Ducting
Manufacturer : International Aero Engines IAE
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 915803
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During completion of an EA (Engineering Authorized) driven 7th stage bleed duct installation on graveyard shift, the duct coupling clamps were tightened by hand only, to approximately the required 85-in lbs. The bleed system leak test was not the one used per the AMM (Aircraft Maintenance Manual) reference printed in the EA. An alternate test was performed using back-driven ground bleed air, which would have excluded the leak integrity of only one of the four clamps during the test.

Mechanic did verify that ALL clamps were properly in position, with no duress caused by duct misalignment. The IP (Intermediate Pressure Stage) check valve lower clamp was found separated (intact with the check valve) from the 7th stage bleed valve duct. This resulted in hot bleed air contacting and damaging the surrounding thrust reverser inner structure. ECAM reported a #2 eng BLEED fault.

Mechanic did not utilize the correct bleed system operational test, opting for a simpler test to be performed. Maintenance personnel were dispatched to ZZZ to remove and replace the left thrust reverser. Suggest that the correct bleed system test be followed.

Synopsis

Mechanic reports that during completion of an Engineering Authorized (EA) 7th-Stage bleed duct installation on #2 engine of an A320, an alternate test was performed instead of using the bleed leak Test Maintenance Manual chapter referenced in the EA document. The intermediate pressure stage (IP) valve lower clamp separated, allowing hot bleed air to damage thrust reverser.

ACN: 915770 (2640 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Make Model Name : A320
Flight Phase : Parked

Component

Aircraft Component : Escape Slide
Aircraft Reference : X

Person : 1

Reference : 1
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 915770
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 144
Experience.Flight Crew.Type : 6100
ASRS Report Number.Accession Number : 915771
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were tasked with a Part 91 reposition flight and the flight was routine. After a second reposition flight on a different aircraft and while in a taxi to our overnight hotel we received a call from the crew taking the original reposition aircraft. They asked us if we had noticed any discrepancy with the overwing emergency exit door "Slide Armed" indication. I advised them that I did not recall any malfunction and the slides themselves do not generate malfunctions to the FWC unless a cover is removed from the overwing exits. As we were a reposition flight only we did not arm the slides due to concern over inadvertent opening of an armed door during a non-normal operation. If the overwing slides were not armed it escaped our notice during the flight. 

We were only advised of a possible situation by the crew subsequently flying the aircraft we had repositioned. Failure of the maintenance facility to rearm the overwing emergency exit slides may have been a factor, however as I did not notice any slide malfunctions and we did not arm the slides prior to departure I cannot say that this may be the case. It may be helpful to add the overwing slides to the post maintenance inspection procedures and possibly add a slides armed item to the before start or after start checklist.

Narrative: 2

Before running the after start checklist prior to taxi, I ran my First Officer flow including verification the doors were not armed. The Captain and I made a conscious decision not to arm the slides to prevent inadvertent slide deployment upon arrival. All slides were in agreement of being disarmed, so I continued with the checklist.

Synopsis

A320 flight crew is informed after the fact, that they may have flown an aircraft on a Part 91 ferry flight with the over wing exit slides disarmed. The cabin doors were intentionally left disarmed but the crew did not notice that the over wing exit slides had been left disarmed by Maintenance.

ACN: 915768 (2641 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 915768

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 915769

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

Over wing slides were not armed for the first leg. It was detected before the second flight and we returned to the gate. [We usually are only] checking the slide arming on the 4 doors regularly checked, as the overwing slides are always armed (supposedly...). The fact that the overwing slides were not armed escaped the scan in our [original departure station]. Suggestions: Include in the after start checklist (or before start) a check like, "6 slides- armed."

Narrative: 2

During the second leg of the our day, after the aircraft was pushed back and each pilot completed the same flows as previously done (to include the after start flow), I happened to glance at the lower ECAM, as the First Officer selected and de-selected the Door ECAM Page, that it didn't appear to look correct. I reached down and re-selected the Door Page. I noticed that the overwing exits were not showing armed. I coordinated a return to the gate with the cabin crew, ramp control, and station personnel. Once at the gate, I worked with Maintenance Control, on-call maintenance, station personnel, Dispatch, cabin crew, First Officer and others to ensure that the aircraft overwing exits were properly armed. It became apparent during this time that the overwing exits were disarmed since the heavy check and the aircraft was returned to service improperly.

Synopsis

An A320 flight crew reported operating the aircraft with the over wing exits not armed. The aircraft apparently was released from a heavy maintenance check and returned to service with the exits not armed.

ACN: 915747 (2642 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 915747

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 915753

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Personal
Function.Other 
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While taxiing for takeoff we received call from a Flight Attendant stating that a passenger was reporting that the wing was on fire. The Flight Attendant said she didn't see anything, but that she was not in a good position to see it without moving passengers. I asked her to move the passengers and look. We notified the Tower and asked for the fire trucks. We had no abnormal indications in the flight deck. The Flight Attendant called back and said she saw nothing, but that at least two passengers had witnessed flames under the right wing.

I made a PA to the passengers to alert them that we had the fire trucks approaching the aircraft to inspect a possible problem. The Fire Department reported no fire and took a heat signature of the aircraft. They reported that all looked normal with some smoke near the number 2 engine. We decided it would be best to go back to the gate and have Maintenance inspect the aircraft.

When we arrived at the gate one of the passengers came forward to explain what she saw. Her description was consistent with engine torching. Maintenance came and inspected the engine and wing area, ran the engine and found no anomalies. The logbook was signed off and we continued with no further incident.

Synopsis

Following the report of a fire beneath the right wing by a passenger and an inconclusive inspection by CFR, an A320 returned to the gate for a maintenance inspection. No evidence of fire was found.

ACN: 915727 (2643 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.MSL.Single Value : 2600

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : LAS

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 1548
ASRS Report Number.Accession Number : 915727
Human Factors : Confusion
Human Factors : Human-Machine Interface

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 915729
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During preflight while programming the box, we originally set 25R. Later, with an updated ATIS, we changed the runway to 1R. We verified the departure for the Before Start checklist, then again during the Before TO checklist. As First Officer and pilot not flying, I verified during the FMGC check by double clutching 1L on the FMGC and read "Runway 1R, Shead, OAL transition". During the takeoff roll, I visually verified 1R on the Navigation Dispaly. Shortly after takeoff the lateral command bar turned left, instead of waiting until the 2,681 FT bubble, which was displayed, which would have taken us into the strip. The Captain ignored the command bars until 2,681, then started his left turn. Then, the next fix was RBELL, instead of BESSY. The box thought we were taking off from 25R. We told ATC that we were having navigation problems and flew an assigned heading of 210. A few moments later, after I manually inserted the correct fixes, we were given a turn direct SHEAD. At no time were we more than 3 miles off course and passed between the two fixes, RBELL and BESSY.

Synopsis

After programming their LAS departure via the SHEAD RNAV SID from Runway 25R, the A320 flight crew instead departed via Runway 1R. When 1R was entered into the FMGC the departure procedure failed to transition to the revised track.

ACN: 915661 (2644 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : S56.TRACON
State Reference : UT
Altitude.MSL.Single Value : 9000

Aircraft

Reference : X
ATC / Advisory.TRACON : S56
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Route In Use.SID : NSIGN2
Airspace.Class B : SLC

Person

Reference : 1
Location Of Person.Facility : S56.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 915661
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

An A320 assigned NSIGN2 RNAV Departure via the PDC to climb to 23,000. The A320 checked [in] on Departure frequency stating assigned altitude of 9,000. I questioned his assigned altitude and he responded again 9,000 as he leveled at 9,000. I climbed him to 23,000 and later told him of the mistake. I had two similar instances of aircraft on the LEETZ2 departure last week. Both were climbing to an altitude of other than 10,000 (assigned by Tower via the PDC) and I corrected both. SLC Tower should assign 10,000 to all SLC departure regardless of route and the TRACON could then climb them on contact or later if there is traffic. The Non-RNAV SIDS and the RNAV SIDS are too confusing for the pilot to always know the correct assigned altitude. They are required to figure out by reading the chart that they are assigned 10,000 on the Non-RNAV departures. They are required to figure out that they are climbing to the top altitude on the NSIGN RNAV, but all the other RNAV SIDS they are assigned either 10,000 or 14,000. This becomes extremely confusing and unsafe when we change runways.

Synopsis

S56 Controller described a wrong altitude level off event, noting RNAV departure procedures are causing confusion regarding initial altitude assignments, adding the Tower should issue initial altitude with the takeoff clearance.

ACN: 915538 (2645 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 915538
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While taxiing out for takeoff, aircraft rolled approximately 10 FT after setting the parking brake. I noticed immediately that the aircraft was moving, and pointed it out to the First Officer. The aircraft was only rolling for a few seconds, and by the time I was going to intercede, the parking brake engaged, bringing the airplane to a complete stop. I tried several times, but could not duplicate the anomaly. I wrote the preceding event up enroute. Upon landing, the parking brake worked normally. I called both maintenance and the Flight Manager to debrief them as to what happened.

Synopsis

An A320's parking brake did not initially engage with the brake handle set until after it rolled about 10 FT.

ACN: 915333 (2646 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component : 1

Aircraft Component : Speedbrake/Spoiler
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 915333
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Human Factors

Narrative: 1

Spoilers were deactivated improperly per MEL 27-65-00. Both #5 spoiler servo controls [Left/Right wing] only had cannon plugs disconnected. ZZZ failed to rotate servo valve to the locked position. Hex head against 'O' causing unwanted movement of the spoilers during operation of the aircraft. Aircraft is now in the normal configuration and no longer deferred.

Synopsis

An A320 arrived to maintenance with the left and right wing flight spoiler servo controls (Actuators) at the #5 positions, improperly deactivated per MEL 27-65-00. Only the cannon plugs were disconnected, allowing unwanted movement of spoilers in-flight. Up-line maintenance failed to rotate servo valve to 'locked' position.

ACN: 915188 (2647 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 21000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 915188

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Passing FL200, we noticed the Captain front windshield had shattered. We immediately leveled off at FL210 and Captain ran the Cockpit Windshield Cracked checklist in the abnormal procedures section of the QRH while First Officer slowed to 250 KTS to lessen impact on windshield. Captain addressed the associated ECAM action. I believe it was "Captain windshield heat inoperative." Pressurization and all other cockpit indications were normal. Captain determined that we were to descend immediately to 10,000 return to departure airport due to the fact that FL230 was the maximum altitude and we didn't have the fuel to make it to our destination, First Officer agreed. First Officer was pilot flying and arranged to return to our departure airport with ATC with the possibility of needing to burn off fuel to land below max landing weight limit based on Captain's and Dispatcher's determination. Captain contacted Dispatch and Maintenance Control and determined that landing overweight was the best course of action. First Officer agreed. Captain ran the overweight landing checklist while ATC informed us they would be rolling CFR. We agreed with ATC and gave them souls and fuel on board. Captain contacted the Flight Attendants and made a PA to the passengers. Airport Operations sent an in-range ACARS with return gate information. Captain then became the pilot flying. First Officer re-ran the cracked windshield, overweight landing checklist, ECAM action/status, and all normal checklists. Captain landed with at approximately 158,000 LBS less than 100 FT per/min. After landing, we stopped the aircraft so that CFR could inspect brakes. All cockpit indications were normal. We proceeded to the gate with one CFR escort.

Synopsis

A320 Captain's windshield shattered passing FL200, the crew leveled off at FL210. First Officer reports that after consultation with the Company the crew elected to return to departure airport for an overweight landing.

ACN: 915153 (2648 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BWI.Airport
State Reference : MD
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : BWI
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1700
ASRS Report Number.Accession Number : 915153
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Returned To Clearance

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

At BWI airport given taxi instructions (pushed back from the gate) to Runway 28 "VIA PAPA, BRAVO, CROSS RUNWAY 22". As I looked up from starting the second engine the Captain had already turned onto Runway 22 and was taxing to Runway 28 via Charlie. I asked the Captain where he was going and he then completed his left turn and proceeded north east on Runway 4/22. The Captain then turned right onto Bravo and picked up the rest of the taxi instructions. In my departure brief, (I was the pilot flying) I asked the Captain if I could start both engines prior to taxiing first flight of the day, heavy for an Airbus, and the busy nature of BWI in the morning). The Captain stated "as soon as I start to move forward you can start the second engine". During the push I started #1 engine and tactfully asked "just start one"? The Captain again stated "wait until I start moving". After we were released from guidance, the Captain directed me to call for taxi. We received our taxi clearance and began to move. I then began to start the number 2 engine. The Company has pushed the single engine issue to the point where Captains are not exercising good judgment when and where not to employ single engine operations. As a retired Air Force Fighter Pilot I am not going to say that I can't do something/anything. If the Captain wants me to start the second engine and not assist during the taxi (during the start) then that's what I'll do. Bottom line we taxied a plane full of people down a runway without clearance. When I noticed it there was nothing I could do about it. I am not confident the Captain would have listened anyway. I strongly recommend more guidance be given to Captains about the COMMON SENSE issues associated with these unsafe single engine practices. I have to work too hard with too many Captains which could be mitigated if Company procedures were more smartly implemented.

Synopsis

A320 First Officer reports his Captain not taxiing via the cleared route while the First Officer is starting the second engine. Runway 4/22 was used as a taxiway instead of crossing it as cleared.

ACN: 915149 (2649 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17800
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 7500
ASRS Report Number.Accession Number : 915149
Human Factors : Human-Machine Interface
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We were ready to go for an on-time. I realized we had an FMS 1 Legacy FMGC installed and asked Dispatch to provide guidance on dispatch legality. FOM revision summary stated that all Airbuses had been converted to FMS2 and we found ourselves with a now obsolete piece of equipment. The Duty Chief Pilot responded quickly stating "aircraft is legal for dispatch." We were assigned an on-gate delay. I confirmed that it was a crew-based delay. I contacted Dispatch because I was concerned about a possible violation for accepting an aircraft that wasn't legal. We were somewhat rushed because the aircraft was late in arriving and we were full. It was a "quick-turn" and I wanted to be certain we did not miss anything before dispatching. Many thanks to the Duty Chief Pilot and Fleet Technical Manager, for their quick response and clarification of this publication discrepancy. Details like this are probably "no big deal" but since our FOM is now basically an electronic-based document online and the only contact we have with the paper-based volume is on the flight deck, we are reliant on the FOM summary. We are required to review the most current revision summary when it comes out, and I called the Duty Chief Pilot a week prior to let the crew desk know about the publication discrepancy. A bigger concern is that in the operations area, casual conversation suggests that some pilots are not in compliance with the requirement to review the document online. This will become one of our biggest challenges in the future. It is imperative that everyone at a minimum log on and document their compliance, since the online system captures log-ons and website navigation. If we want to remain paperless and "virtual flight bag" then we have to maintain our currency. If we don't provide proof of compliance we may be asked to drag those flight bags around again. I'm certainly glad we are carrying less paper. I would hope everyone is logging on and checking their revision summaries like we are supposed to.

Synopsis

An A320 Captain discovered that his aircraft's FMS had not been updated to the latest FMS as indicated by Company manuals only to be told the aircraft was legal. The issue was pilot accountability in an all electronic and online technical distribution system.

ACN: 914888 (2650 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : Nose Gear
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autoflight System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 6500
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 1335
ASRS Report Number.Accession Number : 914888
Human Factors : Troubleshooting
Human Factors : Confusion
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 167
Experience.Flight Crew.Type : 3317
ASRS Report Number.Accession Number : 914891
Human Factors : Troubleshooting
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise at FL370 approximately 300nm from destination and four hours into the flight we received a ECAM for L/G -Shock Absorber Fault. We then received ECAM for A/P and A/thr. We then noticed the engines had rolled back to idle. The Captain (PF) then backed the throttles out of climb detent to regain control. We also noticed that we lost both Flight Directors and FMA's. Initiated a call to Dispatch to help establish contact with Maintenance. As we were talking to Maintenance (About 10 minutes later) we noticed that the Flight Directors reappeared on both of our PFD's. We then attempted to engage the autopilot and autothrottle. This worked for less then a minute then we received the L/G shock absorber ECAM again followed by the A/P and A/thr disconnect ECAM's. At this point we decided we would not use the Auto Flight systems for the remainder of the flight. Since there is nothing in the Flight Manual about this situation we consulted Maintenance for any other information we would need to know to conduct a safe approach and landing. They informed us that to be alert for an ECAM for the N/W which we had already noted in the flight manual. In coordination with Dispatch we declared an emergency and the Captain flew an uneventful ILS approach to landing.

Callback: 1

The reporter believes that a sensor failure caused the reported anomalies although no definitive cause was reported by Maintenance. He also states that Autothrust did not reduce thrust during the second failure, indicating Thrust Lock and maintaining the current thrust setting. He believes that the first instance was the same except that Autothrust had just made a large thrust reduction at the moment of failure, giving the appearance of a failure caused thrust rollback.

Narrative: 2

Uneventful until about 2 hrs into flight... Received Anti Ice Stby Pitot ECAM. Worked to clear and ECAMS complete, screens normal. About 4 hrs into flight... At FL370 received L/G Shock Absorber Fault, AP and ATHR kicked off, throttles went to idle thrust. Immediately followed by Auto FLT AP OFF, Auto FLT A/Thr off ECAM. Lost all FMA's and both F/D. Attempted to restore Autoflight, but would not engage. Discussion with Dispatch and Maintenance were inconclusive about actual system fault determination. Upon reviewing FM, further notes, warnings and cautions revealed possible nosewheel offset upon landing. Dispatch was coming to same consideration upon review of FM, and had already notified Tower to standby with equipment upon landing. Flew manual and raw data approach to an uneventful landing.

Synopsis

A320 Flight Crew experiences ECAM for L/G Shock Absorber Fault at FL370, followed by Autopilot, Autothrottle, and FMA failures. Crew continues to destination for uneventful landing using manual flight and throttle controls.