ACN: 914805 (2651 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight
Ceiling.Single Value : 25000

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Aircraft : 3

ATC / Advisory.Tower : ZZZ
Make Model Name : Embraer Jet Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 914805
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Deviation / Discrepancy - Procedural : Landing Without Clearance
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were cleared for a visual approach to the runway behind an Airbus A320. Because we were vectored in tight behind the A320, we requested S-turns to give us some extra spacing. We were also advised that there would be an Embraer jet departing another runway prior to our arrival. On short final, we encountered wake turbulence from the preceding A320 and I was busy stabilizing the aircraft while, at the same time, paying attention to our separation from the departing Embraer jet. As my comfort level with regard to spacing from the departing Embraer jet was beginning to diminish, I began to prepare myself for a go-around. Shortly thereafter, it became apparent to me that the Embraer jet would be airborne and beyond the intersection of our runway and his prior to us touching down. I then decided to continue the approach and land the aircraft. After arriving at the gate, I questioned my First Officer if he remembered receiving a landing clearance because I did not remember receiving one. He did not remember receiving a landing clearance either. 

I believe that I landed without a clearance, unintentionally, because of a number of factors. First of all, I was task-saturated at a critical point of the flight - on short final. Because of the tight spacing behind the A320, the wake turbulence encounter and the need for ensuring adequate separation from the departing Embraer jet, I was distracted and failed to remember to obtain a landing clearance (at least I think I failed to obtain the landing clearance). Although I am dissatisfied with myself and the fact that I unintentionally landed without a clearance (or at least I think I did), I do feel that safety was at the top of my priority list in that my attention was focused on stabilizing the aircraft and avoiding a potential separation conflict with the departing Embraer jet.

Synopsis

An A320 Captain reported possibly landing without a clearance after he was distracted by a wake vortex encounter and concern with the tight spacing on the preceding aircraft.

ACN: 914141 (2652 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Fuel Line, Fittings, & Connectors
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Fuel Tank
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 914141
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Fatigue
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 914145
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Fatigue
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Person : 3

Reference : 3
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 914644
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Fatigue
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

An Engineering item was called out to replace left wing Fuel Pressure Holding Valve and I mistakenly replaced the same valve on the right wing by mistake. We were on feild trip to accomplish a tank entry to complete the job. It was our intent to just open the tank and start venting and return after rest to go in tank. But when we opened the access door to reach valve, discovered that it could be changed easily within five minutes, by reaching in approximately ten inches; no tank entry required so we completed the job. I can't explain why we changed the valve on the wrong side except to say that I asked which side and was told it was the right side, so there was some miscommunication.

Narrative: 2

I was called for field trip to replace a pressure holding valve on an A320 per Maintenance Engineering. After arriving we went to the aircraft. There was some discussion about the valve being replaced and other work on the aircraft. The Engineering callout required replacement of the left tank valve and I replaced the right tank valve. I am not sure how this happened except that there was some miscommunication. We did have some trouble bringing up the item in the Scheduled Maintenance computer, because we were at ZZZ and by this time it was [before sunrise].

Narrative: 3

Only after it flew, it was discovered that the wrong valve had been changed.

Synopsis

Three Line Mechanics report about a Maintenance Engineering call out to replace an A320 left wing fuel pressure holding valve on a field trip. Mechanics mistakenly replaced the right wing holding valve instead, with miscommunication cited as a contributor.

ACN: 913988 (2653 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Compressor Stator/Vane
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 913988

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 24000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 5400
ASRS Report Number.Accession Number : 913998

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

During preflight planning we noticed much logbook history on left engine compressor vane problems. During takeoff roll, I had just announced "80 KTS, thrust set" when we received an ECAM for left engine compressor vane. Captain aborted takeoff (his takeoff) and I advised ATC of our abort. We exited the runway and the Captain instructed me to tell the cabin to remain seated via PA announcement (which I did) and then began to run the ECAM while the Captain was clearing the runway, now at a safe taxi speed. ECAM required left engine shutdown which was accomplished as we were joining the parallel taxiway. We set the parking brake and completed Flight Manual checklist and review of problem. Captain contacted Dispatch and advised them of our abort and coordinated a return to gate.

Narrative: 2

During preflight briefing the First Officer and I discussed possibility of left engine compressor vane occurring on take off because of the extensive history noted in flight papers. As we accelerated through 80 KTS the left engine compressor vane ECAM came up. As pilot flying I immediately called "aborting, call ATC" and executed the abort at approximately 85 KTS. Autobrakes engaged and as we slowed through 60-70 KTS I disarmed the autobrakes and directed First Officer to make PA for passengers to remain seated as we exited the runway. Brake temperatures did not exceed 500 degrees.

Synopsis

A320 flight crew noted during preflight that the aircraft had a history of stator vane problems. During takeoff ECAM announced left engine compressor vane fault and the takeoff successfully rejected.

ACN: 913580 (2654 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 10500

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Route In Use : Vectors
Airspace.Class E : SFO

Person

Reference : 1
Location Of Person.Facility : NCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 913580
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was monitoring the Quake Sector. The Controller is newly rated as I understand either today or yesterday. The Controller issued direct Avenal VOR leaving 9,000. Outside air temperature was 31 Celsius and aircraft were climbing out very slowly. This control instruction turned the aircraft into the Boulder Sector. Boulder goes up to 12,000 and Quake owns 13,000 and above. I observed that the aircraft was not going to top the sector. I went over to the Controller and told him that a point-out was required. He initiated an auto-point out to the Boulder Controller. Boulder did not respond. The Quake Controller allowed the aircraft to enter Boulder's airspace while the point-out was still flashing. Boulder acknowledged the point-out as the aircraft was climbing through 12,500 well inside Boulder airspace. 

What causes me concern about this seemingly innocuous incident is the conversation I had with the Controller after he/she was relieved from the sector. I asked the Controller if he/she knew where Boulder's airspace was. The Controller replied "yes". I asked why a point had not been initiated to Boulder if he/she was aware of the airspace boundary. The response was "I don't know." I asked why he allowed the aircraft to enter Boulder airspace without approval even after I asked him to get the required point-out. The response was "I don't know." I informed the Controller that the actual "flash" is not approval to enter. If the Receiving Controller has not responded, verbal coordination needs to be made. The Controller did not know this. The Controller didn't know that entering another Controller's airspace without prior approval is a loss of separation and constitutes an operational deviation. How this individual was certified not understanding what a point-out is, I can't answer except to say the training program is failing. This individual is not the only newly rated controller having these problems. Street hires at this facility do not appreciate the seriousness of operational deviations and errors. Self identifying reporting programs exacerbates this because there are no consequences to their poor performance. Operational Supervisors look the other way when performance issues arise, at least in my area. Fifty percent of them are so operationally incompetent, they don't even recognize the deviation or error is occurring. The Union and peers come down all over individuals who raise the safety flag on controllers who consistently perform not just poorly, but dangerously.

Synopsis

NCT Controller witnessed an unauthorized airspace entry event where the Controller in question had a poor understanding of point outs and airspace restrictions.

ACN: 913459 (2655 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 913459
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

In cruise at FL350 we got an ECAM for yellow system hydraulic fluid quantity low. We performed the ECAM checklist, QRH and follow up. Initially we fixated on the yellow electric hydraulic pump switch on/auto positions not the PTU on/off switch. The PTU was subsequently intermittent and then failed. We performed the loss of yellow system pressure ECAM/QRH procedures. We declared an emergency in perfect VMC conditions, descended and turned back toward the airport. The green system intermittently indicated overheat during the turn. The Flight Attendants and cabin were notified of the upcoming precautionary landing and time. Distance and speed adjustments from the QRH were checked for the approach and landing. When flaps were selected, only slats 3/flaps 0 were available. We made a left turn in the pattern as green/blue pressures were still good to recheck new distance requirements and reference speed adjustments. When turning final again the gear did not release from the uplocks when selected down, so we manually released it and it quickly extended and locked. The aircraft was over the fence at adjusted Va with a good touchdown and near normal deceleration. We were rolling slowly by 5,000 FT, and continued slowly to near a far end turnoff and set brakes. We made a P/A to the passengers and Flight Attendants that we were fine but that we would be towed into the gate. We instructed the emergency crews to stay away from the landing gear until the tires cooled. The left pair (with reverser and upwind) was less than 300 degrees C. The right pair (no reverser and downwind) was in the mid 500 degrees C. The fans were turned on and the right pair came down to the mid 300s C prior to tug arrival. The aircraft was towed to the gate. The passengers were upbeat during deplaning and there was no immediate apparent injury or mental trauma. During our post-flight debrief we both looked up at the hydraulic panel at the PTU on/off switch where it had been the whole time and realized our procedural error.

Enroute an o-ring in the yellow hydraulic system failed rapidly dumping all yellow system fluid overboard. Needless to say, I will probably live the rest of my life with heightened hydraulic system awareness. Two thoughts come to mind, however: 
1. If desired, a following note could be placed in the QRH after the loss of yellow system fluid checklist directing the crew to perform the loss of yellow system pressure ECAM, indicating that if this ECAM checklist was not present that the loss of fluid checklist was not complete. 
2. A spot event could be flown during recurrent training, not involving the whole hydraulic system loss scenario, but merely the initial step of selecting the PTU off, so the crews have a look at that never-used switch. The electric yellow hydraulic pump switch is used once or twice per leg during normal operations to inhibit PTU operation. This might be of some value to other crews.

Synopsis

An A320 experienced Yellow Hydraulic Reservoir low level ECAM at FL350. ECAM procedures were complied with except the PTU switch was not turned off. The flight diverted. During approach only slats extended and the gear was extended manually. A no flap landing ensued with a tow to the gate.

ACN: 912963 (2656 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 35000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLA

Component

Aircraft Component : AHRS/ND
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 450
ASRS Report Number.Accession Number : 912963
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Shortly after reaching cruise just east of LAS my (Captain's) ND showed "map not available" and would remain [so] until landing. For the next 3 hours we would receive conflicting information on our level of failure modes. My FMA displayed 1FD2, AP2, and my MCDU illuminated the FM1 light along the top row indicating per the FMS2 Pilot Guide that FMGC1 had failed. I would have anticipated my FMA to have shown 2FD2 but it did not. A little later my FMA showed "-FD2" and then again later show 1FD2 while both FD's remained on. On the MCDU scratch pad I would occasionally show "independent operation" or "opp FMGC in Progress". When we were done with the weather radar we tried to select TERR but were unable to select further indicating failure of FMGC1. My MCDU could operate the ACARS but not the FMGC on either side. I had to look cross cockpit to use the First Officer's ND and use his MCDU to input information. On the data page FMGC1 showed blanks for lats and longs. All other indications of location were close and agreed with raw data off VOR's. We reviewed system resets and per the procedure we did not touch any circuit breakers for FMGC's since we were airborne. We did however locate all applicable circuit breakers in the event we would need to reset them later. We studied the FMGC dual failure irregular checklist and the additional procedures for standby navigation in the event FMGC2 would fail. I tested my RNP's ability to manually tune VOR's and ILS's on the ND which it did. I never did get any ILS indications on my PFD which I thought I would have at some point. Additionally, we turned off both FD's for 5 min to see if we could get FMGC1 to re-synch with no luck. We then contacted Dispatch to advise and got a phone patch with Maintenance Control to discuss. He confirmed that we had done all that was available and with the good weather decided to continue on to destination. Things were pretty much routine until turning an 8 mile final for 27L ORD. The First Officer turned off the autopilot and autothrust. At the time we were in HDG and VS -1200. We were set up for the ILS with the ILS 27L in the flight plan, both FD's on, LS buttons selected on with the correct data showing on the RAD page. He asked for the approach mode which would not arm. I tried the LOC mode and that would not arm as well. We remained in HDG and VS -1200 down final. I tried de-selecting my FD and LS and using just raw data on the ND while in the ILS mode and still could not get approach to arm. At around 700-800 FT got an ECAM for CAB PR- LDG Elev Fault. I set the landing Elev manually per the ECAM. Later at about 400 FT I got a MCDU scratch pad note indicating I had lost my speed target on my speed tape and to reference the VLS indication. We were indicating about 135-140 KTS. and that sounded reasonable. The First Officer later reported his speed target appeared normal. After landing normally we confirmed that the outflow valve had indeed opened properly before pulling up to the gate. Reflecting back if I were to see an FMGC failure like this in the future I would not continue on into adverse weather as we were limited to a raw data approach at the destination. In a similar scenario I think it would have benefited us to pull the circuit breaker for FMGC1 forcing a complete shutdown of FMGC1 instead of what appeared to be partial and inconsistent operation of it.

Synopsis

An A320 Flight Crew reported multiple and varied autoflight and FMS system anomalies which they were unable to correct. Despite this they continued to their destination. Fair weather enroute and at their destination contributed to the decision to continue.

ACN: 912960 (2657 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Relative Position.Distance.Nautical Miles : 10
Altitude.MSL.Single Value : 4000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 912960

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10540
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 5812
ASRS Report Number.Accession Number : 912968

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

When selecting flaps 2, received ECAM "flaps locked". Ran checklist, landed uneventfully.

Narrative: 2

Upon configuring aircraft going from flaps 1 to 2, we received an ECAM stating the trailing edge flaps were locked. Declared emergency. Flew published missed approach and ran checklist. Uneventful landing after Visual approach. Taxied to gate.

Synopsis

A320 Flight Crew reports "flaps locked" ECAM when selecting flaps 2 during approach. ECAM actions are complied with after a go around and crew lands uneventfully with partial flaps.

ACN: 912857 (2658 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : IAD.Airport
State Reference : DC
Altitude.MSL.Single Value : 2200

Environment

Ceiling : CLR

Aircraft : 1

Reference : X
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : IAD

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : B757 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent
Route In Use.STAR : SHONN
Airspace.Class B : IAD

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 5900
ASRS Report Number.Accession Number : 912857

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 1400
ASRS Report Number.Accession Number : 912862

Person : 3

Reference : 3
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 4900
ASRS Report Number.Accession Number : 913277

Person : 4

Reference : 4
Location Of Person.Aircraft : Y
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3900
ASRS Report Number.Accession Number : 913287
Human Factors : Situational Awareness

Events

Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Automation : Aircraft RA
Detector.Person : Flight Crew
Miss Distance.Horizontal : 900
Miss Distance.Vertical : 400
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

On approach to Runway 1C at IAD at 2,200 MSL on an approximate 6-7 NM visual final at 180 KIAS with flaps 2 and gear up, we noticed an aircraft coming at us from the left slightly below our altitude. As we were contemplating either taking evasive action or querying Tower, we received a TCAS RA to climb, which we complied with. The other aircraft turned towards the correct Runway 1L, and the RA went away. We were able to continue the approach and land with no other issues. The approximate range was 300-500 FT vertically and 800-1,000 laterally.

Narrative: 2

We were cleared for visual, as we were descending through about 2,200 FT, aircraft lining up for 1L overshot their turn and came into our flight path. We got a TCAS climb. First Officer initiated climb, I advised Tower about TCAS climb. Once clear of conflict, we continued approach without further incident.

Narrative: 3

We were on the southwest heading which is the last part of the SHONN arrival for Runway 1L. The weather was VFR but with strong gusty winds from the northwest. The heading was approximately 210 and the First Officer was pilot flying, flight directors were on but he was hand flying. Approach Controller called out traffic for our runway and it was a B777. I looked at First Officer and he seemed to acknowledge traffic so I called it in sight. A short time later the Controller vectored us to a southeast heading and cleared us for the visual approach at which time he also gave an easterly heading. As the winds were strong First Officer started a turn to final and it seemed that separation with the B777 was closing in so he started to slow as well, on the turn Controller called out traffic for 1R and mentioned if we were turning final to which I mentioned yes, we were already in the turn. At that time a TCAS alert announced to descend and First Officer was already descending and turning toward the Runway 1L which when I looked down at the ND display showed that we were in compliance with TCAS directions. TCAS warning was brief. We were positioned just east of 1L centerline and correcting fast to centerline. The TCAS announced clear of conflict and we continued the approach and landing. It was a busy time for us and the Controller, but at no time did I feel that First Officer was not in control and in a position to continue the approach, he did a great job with all the distractions and the winds that were present.

Narrative: 4

The Controller turned us to an east heading and gave us a descent to 3,000 so I kept the descent going and called for flaps 1. We were at approximately 215 KTS at that point with the speed brakes still extended and I called for flaps 5. As we rolled out on the east heading, I looked back down at my Navigation Display and saw that the winds were at our tail (due west) at 20 KTS and that the extended centerline (magenta line) was only approximately 1 mile away. Recognizing that the compression of time and speed combined with radio saturation had prevented us getting a turn to final, I immediately went into a left turn toward final. Just as I started to make the turn, I looked up and saw an A320 on final for 1C. Simultaneously, ATC called out the A320 at 1:00 and we got a TCAS warning and descend command. We were already in the descent and left turn so I only had to lower the nose 1 degree and the TCAS funnel immediately was above the ADI aircraft symbol. All this occurred while I was in a 30 degree and temporarily a 35 degree bank to the left to join final. As the TCAS declared "clear of conflict" we continued slowing and configuring for landing. We had overshot final to the east, placing us between 1C and 1L. I corrected to final and continued to slow, configure, and perform procedural and checklist items. The rest of the approach and landing were uneventful except we still had concerns with being so close to the B777 ahead. 

The compression of our approach was such that we landed only 3 miles behind. Considering the strong surface winds we were comfortable continuing the approach. If the Controller ever cleared us for a visual approach, I never heard it. Therefore, my mindset was one of following his directions, not doing what I would normally do to maneuver the aircraft for landing behind a preceding airplane. He did not give us a slower speed until we were on base but as stated above I had begun to slow due to the sensed compression of the approach. The radios were completely saturated as they often are at that time of the day and the cockpit tasks, commands, and responses were also fast and furious from the time I put the speed brakes out on downwind until short final. The scenario is fairly common but from my perspective, there were three factors that created problems leading to the TCAS incident. First, the vectors on downwind that caused our distance to final to decrease at the same time the winds were strong for overshooting final was a factor. I would have expected vectors west of 190 degrees with those winds, not the opposite. If the Controller ever cleared us for the visual approach, my missing it was an obvious problem because I remained on the east heading when in reality, from our position, it needed to be a continuous 150 degree turn to final, eliminating a "base leg" altogether. Lastly, even if the Controller had given us a clearance for a visual approach, the issuance of two heading changes from downwind implies that he still has control of our flight path leaving me with the impression he will also be giving a "dogleg" heading to final. This condition was misleading and confusing at best. To illustrate the point, had I been aware we had received a visual approach clearance, I would have flown the approach referencing the B777 and would not have turned base at all until they were at least abeam us on final. Once again, the lesson I have learned is that the visual approach into this airport is probably the most demanding environment for us in that it can create a dynamic set of circumstances that greatly enhance our chance for errors, and supreme preparation, attention, and vigilance are required.

Synopsis

A320 flight crew on the ILS to Runway 1C at IAD describe a TCAS event with a B777 intercepting the Runway 1L centerline for the visual. A B757 crew following the B777 described the event from their perspective.

ACN: 912159 (2659 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Check Pilot
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 350
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 912159
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 912312

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Environment - Non Weather Related

Narrative: 1

The aircraft was ready for pushback and all checklists complete. We were waiting for the push crew to call ready for a push clearance. Push crew informed me that the static ports did not look right covered with a plastic film. I said I would call Maintenance to come and take a look, since the doors were closed and jetway not connected to the aircraft. My First Officer called Maintenance and asked [for a] mechanic to come out and check the static ports. Maintenance said no because, "[they did] not want to take a delay for this flight." In my opinion, this is a one time event, however, being the last chain in the safety net, I hope my fellow employees are looking out for us. Our push crew stated, "[our] fellow employees always give [us] grief when [we] point out irregularities because they do not want the blame for loosing the 'on time'." What kind of company have we become when 'on time' is the sole metric we are judged by?

Narrative: 2

I very firmly informed [maintenance] that he would be sending a mechanic to take a look or we weren't going. While we waited, the Ramp Agent informed us that his supervisors give him a hard time anytime he reports that something looks wrong with an airplane. We thanked him for being observant and vowed to report this. The mechanic showed up and it only took 3 minutes to determine that we were OK. We left a few minutes late but arrived early. This ridiculous obsession with 'on-time' has over-ridden safety first and I truly believe that this culture is setting our airline up for an accident. You should know that I am extremely easy going and ALWAYS work hard to stay positive. I always go the extra mile for the company, so when someone like me is concerned, it better raise a big red flag in flight operations.

Synopsis

A320 flight crew was informed by the pushback crew that the static ports appear covered by a plastic film. When the crew called for maintenance they initially refuse, stating they do not want to take the delay. No discrepancy was noted and the flight departed three minutes late.

ACN: 911916 (2660 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Repair

Component : 1

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Cabin Furnishing
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 911916
Human Factors : Communication Breakdown
Human Factors : Confusion
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Equipment / Tooling

Narrative: 1

I was given a write-up to remove [slide] girt bar fittings. After removing sealant covering bolt heads, it showed that three bolts had holes drilled through (all the way into bolt shaft) and one bolt had a drill mark.

Callback: 1

Reporter stated the A320 was in for a Corrosion Visit. He noticed the drilled bolt heads at cabin Door 2-Right. The bolts secure the door slide girt bar fittings to the cabin galley floor area, which also has corrosion problems that extend to the door sill plate and threshold entry areas.

Reporter stated the design of the girt bar floor fittings doesn't allow the use of a standard socket, only a very thin-wall socket will fit. Reporter stated most mechanics have to modify their own socket to fit the cutout area where the bolt is installed in the fitting. He believes the aircraft had been at a Contract Maintenance Repair facility for a Heavy Check and mechanics there had drilled into the bolt heads and used an "E-Z Out", to back-out three of the four bolts from girt bar fitting and used the same bolts again to reinstall the fitting.

Reporter stated his concern is the girt bar fitting could tear from the floor during an emergency use of the slide because the drilled bolts would be weakened and shear at the girt fitting. Using a twelve-point bolt head instead of the six-point bolt head would allow better access to the bolts.

Synopsis

Mechanic reports he was given a write-up to remove cabin door escape slide girt bar fittings on an A320 aircraft during a corrosion visit. After removing sealant, he found three bolts had holes drilled all the way through into the bolt shaft and one bolt with a drill mark.

ACN: 911728 (2661 of 2908)

Time / Day

Date : 201010
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ATL.Airport
State Reference : GA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ATL
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Facility : ATL.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 911728

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : Taxi
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working the Local 3 position in a west configuration. An A320 was holding in position on Runway 27R, I told the A320 to RNAV to FUTBL, Runway 27R cleared for takeoff. The A320 read back RNAV to MPASS cleared for takeoff. I restated to the A320, verify RNAV to FUTBL, they responded we have MPASS as our first way point. I told the A320 to hold in position, they had the wrong runway programed in the FMS. If the A320 had RNAV'ed to MPASS off of Runway 27R, he would have crossed over both departure courses on the north side of the runway, creating a potential for disaster. This is not the first time pilots have attempted to turn the wrong way while doing RNAV departures here at ATL. 

I think ATL should use headings off the ground. We have so many different configurations, weather it be east, or west configuration, and constantly changing departure split, some runways RNAV, some don't, some do with certain departures, then change back at a moments notice.

Synopsis

ATL Controller described a RNAV departure runway programming error, had the error not been caught the aircraft would have crossed both departure courses.

ACN: 911586 (2662 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.Tower
State Reference : US

Environment

Flight Conditions : IMC
Weather Elements / Visibility.Visibility : 8
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Landing
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 911586
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Conflict : Ground Conflict, Critical
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Prior to departure, we called Dispatch to discuss the weather and planned fuel upon arrival at our destination; in particular the tornado watch in the northeast. Dispatch advised us the weather shouldn't be a factor but we have an alternate due to a low ceiling and we can't pick up additional fuel because we were at our maximum takeoff weight. We were dispatched with 40,100 LBS of fuel. Departure, climb, cruise and initial descent were normal.

During the initial descent we received holding instructions and an EFC (Expect Further Clearence) of XA45Z. We sent an ACARS to Dispatch with the holding information and an FOB of 10,000 LBS. We briefed the need to exit the hold and proceed to our alternate by time XA57Z or 6,700 LBS fuel remaining. Destination ATIS indicated: wind 190/17G24, vis 9 SM, SCT 007, BKN 031, BKN 140, BKN 250, 22/20, altimeter 29.71. ATIS at our alternate indicated: wind 220/12G17, vis 10 SM, SKC, 25/14, altimeter 29.65.

A short time later we were cleared to leave the hold and continue our STAR and we sent an ACARS to Dispatch advising them. During the descent and arrival the instrument approach was changed three times, from the left runway to the right and then back again to the left. ATC vectored for the approach behind a heavy foreign carrier ahead of us on the approach. We contacted Tower when established on the approach and were cleared to land. Passing 1,000 FT on the approach we heard the Tower multiple times giving taxi instruction for the foreign carrier to exit the active runway. Each time the carrier replied inaccurately. At 500 FT the Tower instructed us to Go-Around due to traffic conflict on the runway. ATC vectored us around for another ILS approach. We were positioned behind several arrivals on downwind at 4,000 FT where we advised the passengers about the Go-Around, the conflicting traffic on the runway and advised them we should be landing shortly.

Reviewing the fuel situation on the downwind leg the fuel was around 5,700 LBS, we briefed the need to declare Minimum Fuel if ATC delayed our vectors. We were still on downwind behind several arrivals with no sign of vectors towards final approach. We declared Minimum Fuel with around 5,200 LBS fuel remaining and briefed the need to declare Emergency Fuel if ATC further delayed our vectors. After we called ATC to expedite our approach without any resolve we declared Emergency Fuel with around 4,500 LBS fuel remaining. Immediately ATC started vectoring us towards the final approach, handed us over to Tower, cleared us for an approach and cleared us to land. The approach and landing were normal. Upon touchdown we had around 3,700 LBS fuel remaining and taxied to the gate on aircraft own power with no further assistance needed.

Synopsis

An A320 Flight Crew departed on a transcontinental flight already concerned about their planned arrival fuel. Weather, traffic congestion and an ATC directed go around at their destination due to traffic on their runway precipitated an Emergency Fuel declaration and expedited arrival handling.

ACN: 911552 (2663 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 911552
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Prior to departure, Maintenance reset a SYSTEM 2 PRESS FAULT. During cruise at FL380 we once again encountered and advised Maintenance of a SYSTEM 2 PRESS FAULT. Approximately 5-10 minutes later we experienced a SYSTEM 1 PRESS FAULT, followed by an ECAM directed emergency descent. We followed the ECAM guidance and requested an immediate descent to 10,000 FT with Center. Due to terrain, the Controller could only clear us to 12,000 FT, which required both pilots to remain on oxygen while coordinating with each other, Flight Attendants, Dispatch and turning on the seatbelt sign. With the weather being completely VFR and unlimited visibility, we requested a descent to 10,000 FT and would maintain our own terrain separation. ATC cleared us to descend and track off course if necessary. At this point communications became much easier after removing the oxygen mask, which had fogged up. The First Officer flew the aircraft and talked to ATC, while I discussed the situation with the Flight Attendants, confirming the cabin oxygen masks did not deploy and no injuries to crew or customers. After briefing the passengers on what had transpired my focus then turned to coordinating a plan with Dispatch and Maintenance Control. Based on terrain analysis and fuel planning, we were able to continue to our diversion airport. Everyone on board was told of our intent and the station was well prepared for our arrival. The First Officer and I created an arrival strategy using all of our available resources, including the FOM Diversion Guide. Maintenance Control indicated he would log the items in the maintenance computer for us, knowing it was a busy time. We landed without further incident.

Synopsis

An A320 flight crew received ECAM pressurization system pressure warnings for systems one and two. The ECAM directed an emergency descent so the crew declared an emergency and diverted for maintenance.

ACN: 911304 (2664 of 2908)

Time / Day

Date : 201009

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 911304
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Other

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Company Policy

Narrative: 1

When landing today we had another issue with the new 10,000 foot double chime and turbulence. We are having more turbulence on descent and are preparing for landing while trying to protect ourselves. We couldn't ensure that things were properly stowed and turned off and be ready to sit down when the Captain says "flight attendants prepare for landing" at 10,000 with the amount of turbulence that we are experiencing on approach at many of our airports. Is this another change driven by customer complaints of not having electronics on below 18,000 feet? We have done electronics at 18,000 feet for the 10 years that I have been flying and never had an issue with it. At a few airports the "seatbacks upright" at 18,000 feet can be a long time, but was the safest thing for our flight attendants and also allowed me to contact the cockpit between 18,000 and 10,000 with any last minute cabin write ups.

Synopsis

A Flight Attendant reported the company's decision to delay restricting passengers to their seats to 10,000 FT during descent vice the previous 18,000 subjects flight attendants to hazards associated with low level turbulence.

ACN: 911266 (2665 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Fog

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway XXL
Flight Phase : Initial Approach
Route In Use : Vectors

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 911266
Human Factors : Troubleshooting
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 911267
Human Factors : Troubleshooting
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

On vector for final ILS, I proceeded to "clean up flight plan", [but] in that operation lost both FMGCs and got a "set field elevation manually press fail" ECAM. Took vectors to work through problems. Set field elevation manually and restored FMGCs. Aircraft showed "CAT 3 dual" with a good id, but never coupled [and] strayed off course, no glideslope. Performed go around and from QRH surmised unable to shoot CAT 1 to minimums required for weather conditions with the available equipment. In communication with Dispatch, ZZZ had better than 1000/3 and we diverted to ZZZ.

Narrative: 2

Flight was proceeding normally until approaching destination. When being vectored to final approach the First Officer asked if he could clean up the flight plan. I agreed and as he started to clean up the screen, the FMGC was lost resulting in the loss of map info both left and right. The ECAM displayed something to the effect "set field elevation manually." We asked approach for a delay vector as we worked to restore map/ILS guidance. After approximately one minute the FMGCs returned, minus all flight plan/performance information. The First Officer worked to put pertinent info back into the "box" as I flew the aircraft. With no clearcut direction we assumed that we had proper info back. When approaching the final again, the flight director/autopilot did not capture the course/glide slope. I turned on the standby navigation and the ILS in an attempt to establish a working ILS that we could fly. The frequency was correct, the course was not. The information was unacceptable for a stabilized approach. The First Officer and the Company Jumpseat rider called for a go around. We missed and climbed to 5000 FT where we declared an emergency and started looking for suitable alternates. The company Dispatcher was contacted and ZZZ was selected. After a initial course correction at ZZZ on the stby ILS, we received an ASR approach. The First Officer preformed a uneventful landing. CRM worked extremely well. The First Officer and the Jumpseat rider helped bring a successful conclusion to this event. None of the standard ECAM direction was available for a quick resolution to this problem. FOB was a concern as we were working the problem, as I have a desire to land with more rather than less FOB. 

Cause. Maintenance problem, followed by lack of clear cut direction. Not using the QRH.

Synopsis

A320 flight crew experiences dual FMGC failure during preparation for an ILS. The FMGCs are restored and reloaded, but the crew cannot get the autopilot to track the LOC or the glideslope. Crew diverts and attempt to use standby NAV ILS is unsuccessful. ASR approach to landing ensues.

ACN: 911262 (2666 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1100

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Component : 2

Aircraft Component : Leading Edge Flap
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 911262
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

After takeoff out of 1,100 FT MSL, I called for climb thrust. First Officer said "flaps up" while simultaneously moving the flap handle. I tried to correct the First Officer, but it was too late. She had already selected flaps up, realized the mistake, then reselected flaps to position 1. At that time we received a flaps locked message and the FD, AP and auto thrust disconnected. When our airspeed was at flap retraction speed we tried unsuccessfully to get the flaps up. We also tried unsuccessfully to put the flaps into position two. We informed ATC that we would like to level at 5,000 FT and maintain our heading until we could assess the situation. I made the decision to keep flying the airplane. There were no immediate action items, so the First Officer began the ECAM action items list. The Flight Attendants, passengers, Dispatch and Maintenance were briefed on our situation. Maintenance had no solution the the problem while we were airborne. We used the pilot jumpseater who had a seat in the back to verify the flaps were stuck in position 1. We calculated landing distance and decided to return. We utilized the fire department as a precautionary step then returned and made an uneventful overweight, flaps one landing and taxied to the gate normally.

Synopsis

A320 Captain reports "flaps locked" ECAM message after takeoff when First Officer mistakenly selects flaps zero and quickly re-selects Flaps 1. Flight returns to departure airport for an overweight landing.

ACN: 911243 (2667 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pressurization Control System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 911243

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

On ground we had a CAB PR 2 fault during setup. Maintenance reset the fault and we were on our way. While at cruise FL380 the CAB PR 2 faulted again. A few minutes later CAB PR 1 faulted and we followed the ECAM to descend immediately. While putting on the oxygen mask there were a pair of reading glasses stuck inside my mask, which I didn't figure out until about 5 minutes into the process. Then the O2 mask fogged up and I had to clear the mask. These two events definitely made the process a little more involved. The cabin never went above 8000 FT and the passenger masks never dropped. We took off our masks at 10k FT. Captain coordinated with Dispatch, Flight Attendant's, and Operations. I worked with ATC mostly. I felt as if it was just like training (calm, not rushed-just do the right thing) and we were both in the loop as to what the other pilot was doing and it became uneventful because of that.

Synopsis

A320 First Officer reports a CAB PR 2 fault during preflight, which is reset by Maintenance. In cruise at FL380 CAB PR 2 faults again, followed shortly by CAB PR 1. The crew initiated an emergency descent and diverted.

ACN: 911077 (2668 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 7500
ASRS Report Number.Accession Number : 911077
Human Factors : Troubleshooting
Human Factors : Confusion
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 911073
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While at cruise Captain's PFD, ND, lower and upper ECAM screens went blank for 3-5 seconds. Ess A/C feed switch showed fault briefly but extinguished after a few seconds. Then screens recovered. Note this this sequence occurred 4-5 times within the next 30 minutes before landing. ECAM, once screens recovered, showed HYD Blue SYS Lo Pressure. The blue pump switch showed a fault. ECAM directed to PNF to turn off Blue pump. ECAM also directed, among other things, to turn off right no 1 fuel pump, Avionics cooling blower fan, and directed DMC to be selected to Captain. ECAM also directed to apply landing distance penalties and procedures. The status page told us the following were inoperative: NWS, Blue HYD Sys, Spoiler 3, Yaw Damper, Cat 3, RT Fuel pump no 1, Flaps/Slats slow to extend, Apply landing procedures, etc. While performing ECAM and analysis, we turned direct destination and declared an emergency.

Narrative: 2

While enroute at FL370 approximately 30 minutes from destination, we had the following malfunctions: Capt PFD, ND, and upper ECAM screen began blanking. Multiple ECAMS appeared with the top ECAM being HYD B sys lo Pressure. This was due to the failure of the HYD B ELEC pump. As the Captain flew the aircraft, I began to work the ECAM procedures. After completing the ECAM's, we decided it was prudent to declare and emergency and proceed direct to destination. With one hydraulic system failed, and the electrical system continuing to blank screens and switching sources, we wanted to land as soon as possible. Center gave us clearance direct to the airport and we landed approximately 20-25 minutes after the first ECAM. On the status page we had a NWS inop message, so we coordinated with Dispatch to have a tow available after landing. We cleared the runway and were towed to the gate. A full description of the events was given to the 5 mechanics on the jetway upon blocking in.

Callback: 2

The reporter states that the electrical failure indications in the cockpit were only of an AC bus #1 failure. He does not recall an indication of a generator #1 failure. He is not aware of what maintenance action was required to return the aircraft to service.

Synopsis

A320 flight crew reported electrical anomalies approaching top of descent. Malfunctions included Captain's PFD, ND, upper ECAM screen blinking, and HYD B lo press among others. An emergency was declared and the flight proceeded direct to destination for a normal landing.

ACN: 910803 (2669 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Main Gear Tire
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 910803
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On takeoff during rotation we a heard a "thud" and then a slight airframe vibration. It lasted 2 or 3 seconds and stopped at gear retraction. No further vibrations or noises were heard. About one hour into flight the 1st Flight Attendant told us a deadheading Flight Attendant sitting in Row 5 said she saw a black object the size of a hockey puck fly off the aircraft toward the side of runway. At that time we suspected we may have tire damage and may have been the "noise" [we heard]. We contacted Dispatch and Maintenance and advised both of our situation. We all agreed that we should continue to destination and land. Dispatch also contacted the Tower and arranged for us to get the longest runway. Dispatch also notified crash and rescue that we may have a damaged tire and our ETA. Upon first contact with Approach we declared an emergency, but they were already aware of our possible problem. We landed normally and taxied clear of runway. We asked CFR to inspect our tires and they informed us the right side inboard tire was blown. At the same time we noticed the brake temps on right gear of 45 degrees on inboard and 520 on outboard tire. We advised CFR of the high temperatures and to use caution around the gear. We then called Maintenance and they advised it would be safe to taxi to gate. We turned on brake fans and taxied slowly to the gate. It appeared the number 3 tire had blown on rotation during takeoff.

Synopsis

Airbus Captain reports being informed by a deadheading Flight Attendant, one hour into the flight, that a tire may have failed during takeoff. Precautions are taken during approach and landing at destination. A right main gear tire is found to be flat.

ACN: 910759 (2670 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 910759
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We declared an emergency with Approach due to a Nose Wheel Steering Fault. We had been working this issue on and off since shortly after rotation. The First Officer was pilot flying and Captain was pilot not flying. Captain did a great job of communicating with all parties concerned. The flight attendants also did a great job of taking care of the passengers' concerns. The emergency occurred because a steering servo broke. Our experience with Maintenance Control during this event was less-than-stellar. We had noticed in both the QRH and in the Non-Normal Supplemental Handbook that with this fault we might also encounter a Shock Absorber Fault upon gear extension. When Maintenance Control was queried about what to expect, they had no idea that this was a possibility and, in fact, told us that the fault would be a non-event. They were quite surprised when we informed them otherwise. We were quite disappointed in the fact that Maintenance Control did not appear to have either reference book lying open in front of them and that they would speak off-the-cuff when advising a crew.

Synopsis

An A320's nose wheel steering servo failed and caused a WHEEL N/W STEERING FAULT ECAM. When the landing gear was lowered the SHOCK ABSORBER FAULT ECAM alerted so an emergency was declared prior to landing. In flight maintenance technical support was minimal.

ACN: 910638 (2671 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SLC.Airport
State Reference : UT
Altitude.MSL.Single Value : 6000

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SLC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SLC

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : SLC
Make Model Name : DA40 Diamond Star
Crew Size.Number Of Crew : 1
Flight Plan : VFR
Flight Phase : Cruise
Airspace.Class B : SLC

Person

Reference : 1
Location Of Person.Facility : SLC.Tower
Reporter Organization : Government
Function.Air Traffic Control : Flight Data / Clearance Delivery
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 910638
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

A DA40 called inbound on the VFR Barn Transition from the west, a normal routing given to VFR inbound aircraft. SLC was in a south flow. I told the DA40 to maintain 6,000 on initial contact. He was the only airplane inbound to SLC anywhere close to Towers airspace and on Tower's RADAR display. I got distracted working ground traffic and discussing things with the Controller in Charge. The DA40 didn't follow the prescribed routing direct to the approach ends of the runways and flew eastbound towards about a 5 mile final north of the airport. Almost simultaneously three events took place. I noticed him off course, the Conflict Alert Warning started going off and approach was calling on the line to see what the DA40 was doing. An A320 was on approximately 6-7 mile final for Runway 16L, still talking to the Approach Controller. I immediately issued an immediate right turn to the DA40 to stay west of the A320 and west of Runway 16L final. I had visual separation with both aircraft and also told the DA40 to maintain visual separation which he acknowledged he would do. He turned and stayed west of final for Runway 16L. The A320 continued inbound, climbed a little, then Approach broke him out, brought him around again and he landed. After talking to the Approach Controller they issued traffic to the A320, didn't know what the DA40 was doing exactly, and issued a traffic alert with "suggest you climb". The A320 climbed per the Approach Controller's instruction. The A320 also advised he had the other aircraft insight. When the A320 was asked if they wanted to continue the initial approach and land, they requested a 360 to come back around. Instead of the Approach Controller approving the 360, they just took him out and brought him back around. The DA40 continued his flight south, then over the airport eastbound and transitioned the airspace exiting Towers airspace to the north. Recommendation, the VFR transitions that SLC [handles] from the west have, for years, caused many aircraft to be in an unsafe proximity to other aircraft on approach to SLC. It is not uncommon for a VFR aircraft from the west on the Barn Transition to end up in the wrong place north of the airport. It probably happens at the very least once every other day. I would eliminate the current VFR transitions SLC has for routing VFR aircraft inbound and outbound from/to the west. There have been so many close events with many different circumstances over the years with VFR aircraft on the current transitions. In my opinion they are a dangerous or a precarious operation at times involving IFR aircraft on approach. Usually Tower is talking to arrivals farther out on final which would have alerted me to the situation developing sooner. Approach is not required to transfer communications until the final approach fix.

Synopsis

SLC Controller described a conflict event between a VFR inbound via the "Barn Transition" and an Air Carrier on final to Runway 16L. The Controller mentioned the Barn VFR arrival procedure frequently causes conflicts.

ACN: 910420 (2672 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 910420
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Other 
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

The PDC clearance assigned us the SEBBY FIVE DEPARTURE (SEBBY5.DAG). In reviewing the SID we were confused by the altitude restrictions presented. There were three fixes at which two restrictions were displayed in the following format- (Over SLI) "At or above 10,000 (ATC)" and "At or above 4,900". There were no notes on the SID page explaining which restriction applies. We checked the legend pages for an explanation and this SID Altitude Information Box was not displayed. We called the Clearance Controller to clarify and he didn't know. He had to ask someone else. When he came back he said it was the lower restriction unless ATC assigns the higher one. The FMC database displays the higher restriction. Additional confusing factors- the SID begins with an assigned heading after takeoff with no information about how long the flight will be on that heading. That makes it difficult to determine if the aircraft performance will be adequate to make the assigned restrictions. Note 1 on the top of the SID page states- "This SID to be used only if unable to use LOOP Departure." We were never asked about the LOOP Departure. The SEBBY FIVE was just assigned. A short explanation on the SID and in the legend pages would be helpful.

Synopsis

An A320 flight crew was confused by the presence of multiple crossing restrictions at the same fix on the SEBBY SID from LAX.

ACN: 909931 (2673 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZKC.ARTCC
State Reference : KS
Altitude.MSL.Single Value : 37000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZKC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZKC

Aircraft : 2

Reference : Y
Aircraft Operator : Military
Make Model Name : Military
Operating Under FAR Part : Part 91
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZKC

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 909931

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 909624

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Automation : Aircraft RA
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Ambiguous

Narrative: 1

While at FL370 Kansas City Center gave us a traffic advisory of oncoming traffic at FL380, heading the opposite way. We acknowledged to ATC we saw him and then we received a TCAS RA to descend. The First Officer smoothly clicked off the auto pilot, pitched down to match the RA VSI bar and reduced the throttles to ensure we did not overspeed. Simultaneously I told ATC on the radio we were doing an emergency descent and following a TCAS RA command. The opposing aircraft flew right over us and we estimated it was less than 800 FT. We had descended to FL365 when the TCAS announced clear of conflict. We climbed back to FL370, re-set the auto pilot and auto throttles and told ATC what happened and that we were back on track. The seat belt sign was on because we had just transited some turbulence. No passengers or Flight Attendants were injured, although after the flight the Flight Attendants inquired about the pitch down.

Narrative: 2

While cruising at FL370 and deviating around a line of thunder storms (HDG 100) we were given traffic information at 12 o'clock FL380.

I believe the other aircraft was military since they were not on the same frequency and had a non-airline call sign.

Synopsis

While deviating around weather an A320 received a TCAS RA with respect to an apparent military aircraft traveling the opposite direction.

ACN: 909863 (2674 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZSU.ARTCC
State Reference : PR
Altitude.MSL.Single Value : 2000

Aircraft

Reference : X
ATC / Advisory.Center : ZSU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class E : ZSU

Person : 1

Reference : 1
Location Of Person.Facility : ZSU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
ASRS Report Number.Accession Number : 909863
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : ZSU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 909875

Person : 3

Reference : 3
Location Of Person.Facility : ZSU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 909092

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

I was in the Area, monitoring and walking by each sector checking Strip Bays and [performing] other FLM responsibilities. I stopped in between the R1 and R5 Sectors and noticed that they did not look busy. I then particularly supervised the R5 Sector and noticed an A320 over the ocean on downwind, 23 miles in the weather. I asked regarding his intentions and the Controller said deviating. I then realized and said "at 2,000 FT", then said, "that's unacceptable". The R5 Controller realized it and proceeded to vector the A320 back to SJU over the ocean, still in the 3,000 FT, MVA at 2,000 FT for about 7 miles. The A320 then entered the 1,700 FT MVA. I reported this to the OMIC. He then proceeded with the preliminary investigation concluding it to be an operational error. It is still unknown if R1 caused the event by handing off the aircraft in a descent to or level at 2,000 FT to R5 while deviating for weather into the 3,000 FT MVA. Recommendation, increase position awareness and no chatting between the sectors. One plane is all it takes. Attention to rules, regulations and procedures. This was an obvious mistake that had it been over land or mountains could have been fatal. These sectors are close in proximity and could have been prevented. However, the MVA over the ocean approximately 20 miles north of SJU airport is 3,000 FT when we "know" that it is the ocean. I believe a revision of MVA's at ZSU CERAP Area is in need.

Narrative: 2

While receiving a hand off from adjacent sector on an A320, the aircraft was descending to 2,000 in a 1,500 MVA. While deviating around weather they got into a MVA area of 3,000 FT. This was an unusual situation, because we were working Runway 26/28 operation, which is worked around 2 to 3 times a year only. Recommendation, since we work this operation only very few times a year, and get a refresher only once a year in unusual situations, it would be helpful to do a refresher 2 to 3 times a year.

Narrative: 3

I instructed an A320 to fly heading 100 and to descend. I am not sure if I descended the aircraft all the way to 2,000 FT. I switched the A320 to the next sector while he was deviating to the north. He entered a 3,000 FT MVA. Neither the R-5 Controller or me noticed the event until it was questioned by the FLM. Recommendation, maintain aircraft high when deviating due to weather.

Synopsis

ZSU Controller described a MVA infractions when controllers failed to note the error during weather deviation maneuvers, the reporter noting the MVA over the ocean area needs to be revised.

ACN: 909100 (2675 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuel Quantity-Pressure Indication
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 909100
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Primary Problem : Company Policy

Narrative: 1

Upon arriving at operations the first officer and I printed and reviewed the paperwork for the flight. The maintenance history for showed several MELs for the fuel quantity indicating system. There are four fuel tanks on an A320. Two of the four tanks had Fuel Quantity Indicators (FQI) that was in a degraded condition and a third FQI was inoperative. The flight crew actions section of that MEL included a restriction that all other FQIs must operate normally. 

I called Dispatch and Maintenance Control. The Controller informed me that the aircraft had been in all night, but that he needed me to refuse the airplane in order for local maintenance to fix the discrepancies. He was polite and understanding, but seemingly helpless to effect repairs. He even agreed with the Dispatcher and me that the airplane was illegal for dispatch. I told him that I recently viewed an e-note from Flight Operations Management that said I wasn't supposed to hear this sort of thing from maintenance, and that our pilot managers consider it a "myth" that a Captain must refuse an airplane in order for it to be fixed. He just chuckled. The Dispatcher said it was understandable to refuse the aircraft.

This sort of conversation is common and I personally encounter this situation once a month on average. The requirement for a Captain to refuse an aircraft in order to affect repairs is commonplace and hardly a myth. Our company consistently tells employees and customers that safety is our number one priority. This program is simply a form a pilot-pushing. The law of averages is such that Management will inevitably convince a weak-willed Captain to take an unairworthy airplane into an unfortunate circumstance. The pressure put on pilots in this regard is an unnecessary intrusion into our safety margin. Will Pilot Management and Maintenance Management wait for a hull loss to react? 

Does a mechanic really need an A&P license to know that pilots need to know how much fuel is on the aircraft? My 11-year old, upon over-hearing my conversation on this matter asked, Dad don't you need to know how much fuel you have? Has common sense been sacrificed for the sake of expediency? The passengers expect a certain level of safety from an airline. 

Perhaps I've grown cynical, but I suspect this is about making the pilots look bad. Customer Service made an announcement at the gate that the pilots were refusing to fly the airplane. The passengers blamed us for their missed connections and their inconvenience. Or perhaps, this is simply manufactured evidence in a lawsuit against the pilots for engaging in a "job action". If so, the company is sacrificing passenger safety for legal advantage. In conclusion, Maintenance Management and Senior Pilot Management have become far removed from the customer experience and more importantly, are not upholding their legal and moral obligations to aviation safety. This program of [requiring an] aircraft refusal to generate a maintenance action must be stopped immediately.

Synopsis

An A320 Captain reported the aircraft provided for his flight had three of four fuel quantity indicators that were either failed or in degraded condition. These items had been written up and MEL'd. The Captain was told that he must refuse the aircraft for service before maintenance would be allowed to rectify the anomalies.

ACN: 908965 (2676 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4200

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Horizontal Stabilizer Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 908965
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

First Officer [was] pilot flying. After selecting gear up we received multiple master cautions and ECAM messages, aircraft goes into Alternate Law while pitch becomes erratic. ECAM displays flight control stabilizer jam. After completing takeoff check list we proceeded with QRH and ECAM procedures, while continuing with the departure I advised ATC that we have a flight control problem and we will be leveling off at 10,000 FT and a 060 heading for an extended downwind with a return to airport. Initially trim wheel had become frozen; from 8 to 10,000 FT we were able to move trim wheel and level aircraft off at 10,000 FT. After level off we reviewed QRH/landing performance. We notified Flight Attendants of a slight problem with the flight controls and [that we would] be returning to the airport, a normal approach and landing anticipated, passengers were also notified in this manner. After an uneventful landing we were instructed to taxi on Whiskey hold short of Taxiway Zulu. Aircraft was determined safe and taxied to the terminal without further incident.

Callback: 1

The Captain reports that initially the stabilizer trim wheel could not be moved but approaching 10,000 FT they decided to force it with positive results. The aircraft remained in Alternate Law all the way to landing with trim being adjusted via the trim wheel. On post flight it was noted that the stabilizer was in the full nose down position. The Captain states that the cause of the jammed stabilizer has not been revealed to him.

Synopsis

A320 Captain reports a jammed stabilizer shortly after takeoff with numerous ECAM messages and the aircraft reverting to Alternate Law. Control of the stabilizer is eventually regained using the trim wheel and and the crew returns to the departure airport in Alternate Law using manual pitch trim.

ACN: 908584 (2677 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 20000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Aileron Control System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 908584
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 850
ASRS Report Number.Accession Number : 908841
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

ECAM indication of flight control Aileron Servo during climb. Conferred with Dispatch and Maintenance Control via radio. In collaboration with Dispatch, since it was a flight control issue we elected to return to ZZZ. Also, as a precaution should the problem escalate, an emergency was declared (Souls on Board), the cabin advised to prepare the passengers and that we were approximately 10-15 minutes from landing. 

I do not believe we received any priority handling, however the fire/rescue equipment did follow us upon landing. Landing was otherwise uneventful. My understanding from the post flight maintenance report is that there was an autoflight ELAC2 fault, as well as an ELAC2 Aileron "order disagree". We changed airplanes and continued uneventfully to our original destination.

Narrative: 2

We could have gone into the closest field; however, doing so would have been a rush and I would have had to use flight control drag devices at high speed which I didn't think was prudent. Returning to our departure station was a normal descent profile.

Synopsis

An A320 flight crew returned to their departure station upon receipt of and ECAM Aileron Servo warning.

ACN: 908465 (2678 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Door Warning System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11760
Experience.Flight Crew.Last 90 Days : 6
Experience.Flight Crew.Type : 4860
ASRS Report Number.Accession Number : 908465

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We had just started the takeoff roll when we had 3 intermittent forward cargo door ECAM warnings. Aborted below 70 KTS and rolled to the highspeed exit. Pilot not flying notified ATC of the abort and that we didn't need any assistance at this time. I cleared the runway and continued to taxi and the First Officer advised the Flight Attendants to remain seated. At the gate Maintenance was able to defer the door warning on the ECAM per the MEL and confirmed the cargo door was secured.

Synopsis

An A320 flight crew rejected their takeoff at a low speed when they received a forward cargo compartment door warning ECAM message.

ACN: 908091 (2679 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 908091
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

The airport was taking off Runway 27 and landing Runway 33L. Receiving the departure clearance we went over what we thought may be our taxi route which involved crossing three runways to get to Runway 27, namely Runway 04L, Runway 04R, Runway 33L via Taxiways Charlie and Delta. We pushed back and were positioned behind a A320 waiting to taxi. Ground Control issued a clearance to the A320 as follows "Runway 27, CHARLIE, DELTA, CROSS Runway 04L, HOLD SHORT OF 33L". As this was the route both myself and my First Officer were expecting, we discussed the fact that the Controller hadn't instructed the A320 to hold short of Runway 04R. After the A320 had crossed Runway 04L they asked for and received clearance to cross Runway 04R. When the First Officer called for our taxi clearance we received the same instructions. The First Officer asked if we should hold short of Runway 04R and the Controller aknowledged that we should and stated that ATC was only allowed to clear us across one runway at a time stating that he/she thought we had been briefed on this. I expected a taxi clearance that stated 'Runway 27 CHARLIE, DELTA, CROSS Runway 04L, HOLD SHORT OF Runway 04R". After that I would expect to hear "CROSS Runway 04R, HOLD SHORT OF Runway 33L" You can't get to Delta without crossing Runway 04R. This is a runway incursion waiting to happen because in low visibility, high workload, your mindset will be on Charlie, looking for Delta and a 33L hold bar. This isn't safe. 

ATC didn't issue a clearance that should have instructed us to hold short of Runway 04R. 

ATC in BOS should be educated on the correct way to issue taxi instructions in the future.

Synopsis

Air Carrier departure from BOS described ATC taxi clearance phraseology that was in error with regard to runway crossing authority.

ACN: 907961 (2680 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 907961
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On route to a foreign destination at FL350 the ECAM announced GREEN ENGINE HYDRAULIC PUMP LOW PRESSURE. We followed procedures. Hydraulic PTU was providing pressure to green system. We sent a message to Dispatch and Maintenance. We were advised to return to our departure airport. All hydraulic systems were supplied with pressure and all hydraulic quantities were good. No emergency was declared.

Synopsis

An A320 ECAM alerted GREEN ENGINE HYDRAULIC PUMP LOW PRESSURE in flight. The Company directed the crew to return to its domestic departure airport and the crew did not declare an emergency.

ACN: 907220 (2681 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Fan Blade
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 907220

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

During take off roll there were numerous birds over the departure end of the runway. At Vr, I slowed my rotation rate in the hopes of staying below the birds. Immediately after lifting off, I saw a large bird off my left side. We felt the bird strike the aircraft and immediately felt a surge and a strong vibration coming from the number one engine. Raising the gear, we continued straight ahead. At 400 FT we advised Tower of a bird strike and the need to return to the airport. All engine parameters on the number one engine were normal. We leveled at 2,000 FT performing standard post departure clean up and power reductions. Setting a speed of 210 KTS, we entered downwind with Tower clearing us to land. My First Officer briefed the flight attendants and the passengers. We landed at approximately 151,000 LBS. The landing was smooth and without further incident. I chose not to use reverse on the number one engine and shut down the engine immediately after touchdown. Airport authorities said the bird appeared to be a Huron. Markings suggest the bird struck the number one engine nacelle at the five o'clock position. There was no damage to the nacelle, but significant damage to one fan blade and damage to two additional blades. After saying goodbye to the passengers, I debriefed the flight attendants. My First Officer and I then debriefed, reviewing our own performance deciding it would not have hurt to declare an emergency and have equipment standing by.

Synopsis

A320 Captain reports hitting a large bird just after lift off causing initial engine vibration. The crew elects to return to the departure airport.

ACN: 907210 (2682 of 2908)

Time / Day

Date : 201009
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : VFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cargo Compartment Fire/Overheat Warning
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 907210
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 164
Experience.Flight Crew.Type : 3330
ASRS Report Number.Accession Number : 907211
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Maintenance
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

While preparing the aircraft for departure, my First Officer found that a wind scan light was inoperative and also the ACARS was not functioning properly. Some ACARS messages were being sent and received but the system was slow and often displayed "NO COMM". Maintenance was notified via the radio of the two write ups. As I went through my preflight checks, although not required since it was not the first flight of the day, I performed the fire tests on the engines, APU system, and Cargo Smoke Detector system. I routinely perform this test on every new aircraft I get into as I have observed an occasional unsatisfactory test on the Cargo Smoke Detection system. On this particular day, the Cargo Smoke Detection system did not test properly as outlined in the flight manual when after pressing the Cargo smoke test button for 3 seconds, the associated lights and warning bells only cycled though once versus the proper two test cycles. I called maintenance on the radio approximately 30 minutes prior to departure and notified of the test failure. When the mechanic arrived, I again ran through the test sequence with him and we together verified that it was only testing once versus the required two test cycles. I told him that it appeared that one of the channels on the detector systems was inoperative. He said that he would look into it and would return with the required paperwork since ACARS was now inoperative. We started to receive some ACARS messages and one of them was from dispatch advising us to amend our release to show that the ACARS and the CHNL 2 smoke detector was inoperative. I was expecting maintenance to defer the Channel 2 and that we would be OK to continue as is. Shortly before our scheduled departure time, he returned with the maintenance release and said that he had difficulty getting all the paperwork done because the computers in the maintenance kiosks were also inoperative. I asked him if he needed to do a test of the system through the MCDU unit and he said that it was not necessary since the system had been deferred. He asked us to place the required deferral stickers in place and then left. As I looked at the maintenance release I saw that the write up said Gripe: "Smoke Det Channel 2 will not test" Action: "Complied with MEL." We then pushed back, taxied out and departed normally. Approximately 30 minutes after departure we received a message from Dispatch advising us that we should return to [departure airport] since there was a concern that the Cargo Smoke detector system had been improperly deferred and that we may not have smoke detection capability. I contacted Dispatch via ARINC to verify that he wanted us to return and to make certain that the proper system had been deferred. He advised us to return to [departure airport] to make certain everything was in proper working order. We then advised ATC that we would need to return to [departure airport] due to a potential system malfunction and that we would like to have emergency equipment standing by since we would be landing overweight and I was concerned about potential brakes overheating. We returned to [departure airport] and landed at a weight of 147,500 with a normal descent rate and at a speed of approximately 140 KTS. Minimal braking was used. The emergency equipment met our flight as we taxied off the runway and after they verified that all was normal, we taxied to the gate.

Narrative: 2

While preparing for departure, the first item that happened was the ACARs showed NO COMM shortly after initialization. While waiting for it to come back I decided to go outside and do my exterior inspection. While outside I noticed the Left Wing Scan light was inoperative. When I got back to the cockpit, I noticed the ACARs was still inoperative, so I notified MX that I had two Write-ups but could not send an MRM. I gave them both write ups verbally, and proceeded to continue my pre-flight duties. When the Captain came in he discovered that the Cargo Fire Suppression System was not testing properly. It was only cycling through one channel. The ACARs had been intermittent throughout our preflight, so we verbally notified MX about the Cargo Fire Suppression System also. After a few more minutes the mechanic came on to the flight deck. He told us the Wing Scan Light and the ACARs were deferred. After pushback the ACARs started working, so I checked the release verification. It showed Release 2 as valid and the MRD as being current, I did not actually read the new MRD. Push back, taxi, take off and climb were normal. At cruise altitude we got a message from Dispatch that we had left with an improper deferral. The entire Cargo Fire Protection System was deferred. There should have been no cargo loaded in either pit, the lavatories should have been closed off and unusable. The combination of factors definitely contributed to the mistake. The intermittent ACARs made communication with Dispatch, and Maintenance more difficult. The mechanic also told us he was having problems with his computer. I think he was having to find a computer he could use, and still trying to get things finished in order to have an on time departure.

Synopsis

An A320 departed with a Cargo Smoke Detector System MEL deferral but was recalled to base while at cruise because the system may not have been properly deferred and in fact rendered completely inoperable.

ACN: 907093 (2683 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Fuel Control Computer
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 907093

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Cleared for takeoff and set flex thrust. Engines surged twice in a very short period of time at approximate 90 KTS and we also experienced ECAM Warning. At that point the Captain took command of the aircraft and rejected the takeoff at approximately 100-110 KTS. Advised Tower of a rejected takeoff and when the aircraft came to a complete stop the Captain advised passengers that we had discontinued our takeoff and please remain seated. Next we communicated with Tower and they asked if we needed assistance. First Officer told Tower we do not need assistance, but we would like to have our landing gear brake assembly looked at by the fire department to make sure there was no smoke or fire. We reviewed the immediate action items and determined hot brakes was the only one that applied. Ran the procedure and then proceeded with the ECAM actions, which were advisory in nature only. Followed up with the ECAM handbook. After the fire department gave us the okay, we continued to the gate and we advised maintenance and operations and told them our brakes were hot and to keep ramp people away from that area until they cooled. The cause had something to do with the right engine fuel control and possibly the FADEC.

Synopsis

A320 First Officer reports engine surge during the takeoff roll. Captain assumes control and rejects the takeoff at 110 KTS.

ACN: 906750 (2684 of 2908)

Time / Day

Date : 201008

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 450
ASRS Report Number.Accession Number : 906750

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During the preflight First Officer found a couple of oil leaks. A Mechanic came out to fix the leaks and ask us to shut down the right pack while he worked on the source. During the work the Gate Agent asked if all was well and could she close the door. Because the Mechanic had used the maintenance recall functions on the computer and I did not know if he would need to look at them again, I said no. The Gate Agent reported the situation to company. After sitting for a while the Gate Agent came back and said Company said the leaked was fixed and to close the door. I said OK, and she did close the door. We received a release stating the Mechanic had tightened a fitting and the aircraft was cleared to go. The jet bridge pulled back and I turned the pack back on and was waiting for my push crew check in. Shortly thereafter the Mechanic came on the headset and said he was still working on the leak and he didn't tell me to turn the pack on. I informed him that we were told the work was done and we also had a current release so stating. There was no answer to that fact but I turned the pack back off and asked the Mechanic to personally inform me when he was done. While I waited for the Mechanic to finish I call maintenance on the radio to inform them as to what just happened. The Dispatcher said the Mechanic's lead must have signed off the work early to expedite the departure. First I thought, with the exception of work that required supervisory inspection, the person who did the repairs signed off his own work and work is never signed off before complete for any reason. The potential for a serious injury to occur during this event is obvious.

Synopsis

A320 flight crew received an updated release at the gate stating all work was complete and they were ready to go after a Mechanic had worked on a leak found during preflight. The crew prepared to depart, which required turning on the air conditioning pack that had been turned off for maintenance, little did they know the Mechanic was still working and the release was prematurely issued from higher ups.

ACN: 906547 (2685 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Engine Control
Manufacturer : International Aero Engines IAE
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Compressor Bleed Valve
Manufacturer : International Aero Engines IAE
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 906547
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Inflight Shutdown

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

I and co-worker Mechanic "X" were assigned a job task for an A320 aircraft to replace the "dummy bracket" for #2 engine's BSV (Booster Stage Valve). The job task was an MEL relating to a Class-2 message that was associated with the FADEC. I printed out AMM reference 73-11-70, primarily for location and torque values of the "dummy bracket" being installed. Because this "dummy bracket" was installed and not the BSV, we performed a dry motor run with # 2 FADEC "On", to ensure no faults were generated. No faults were recorded, so the presumption was that everything else in AMM reference 73-11-70 was ideally for the BSV installation. And because no fuel lines were disconnected, the idea of an idle leake check didn't seem necessary.

Aircraft engine auto-shutdown. A fuel leak was identified. No specific information on how to remove and install "dummy bracket". I suggest a job card providing information on removing and installing "dummy bracket".

Synopsis

A Mechanic reports being assigned a job task to replace the "dummy bracket" for a #2 engine BSV (Booster Stage Valve) relating to an MEL Class-2 message associated with the FADEC on an A320 aircraft. Mechanic performed a dry motor run believing no fuel lines had been disconnected at the Bleed Valve Actuator. Engine later auto-shutdown inflight due to fuel leak.

ACN: 906486 (2686 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 300
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 906486
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 906482

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

This event involved a loss of primary navigation mode (VOR, ILS) for both the Captain and First Officer. On the ground in ZZZ2 the power cord was removed prior to the APU running. On the Airbus this causes numerous electrical problems ECAMs, warnings and cautions. Once the electrical power was restored and the screens were normal (this took several minutes) we did a check of all the systems and all appeared normal. What we later learned was that apparently the NAV system defaulted to the standby navigation mode with only minimal indication and some abnormal indication it had done so. Once airborne and on climbout we received a NAV accuracy downgrade, followed shortly thereafter by an IRS NAV only message. We were unable to enter anything into the RAD/NAV page. After protracted troubleshooting with Dispatch and Maintenance Control we made the decision to divert to ZZZ in order to do a ground reset. ATC did declare us an emergency aircraft although we did not request it. ZZZ was chosen as the divert airport as the weather was clear there while it was IFR with CBs at our destination.

Synopsis

An A320 flight crew diverted to an enroute airport when their navigation system reverted to IRS NAV only and they were unable to program their route. A power interruption at the gate prior to departure may have initiated the anomaly.

ACN: 906422 (2687 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 31000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : AC Generation
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 6700
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 960
ASRS Report Number.Accession Number : 906422
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 906910
Human Factors : Troubleshooting
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb out, First Officers' PFD, ND, and some cockpit lights began flickering while several ECAM's cycled on then off. ECAM's stayed on after First Officers' PFD, ND, and some cockpit lights failed. ECAM's included AUTO FLT A/THR OFF and ELEC GEN 2 FAULT, to name only a few. ECAM's were accomplished and APU was started. Descent to lower altitude and uneventful landing at destination coordinated with ATC.

Narrative: 2

Reporting due to a loss of the autothrust system. Primary fault was loss of Generator Bus 2 failure (out of phase). We tried to restore the generator by cycling (per ECAM) and lost it again, then used APU generator. Could not restore autothrust. After the trip found that this problem was in the Flight Manual (supplemental section). That procedure contradicts the ECAM. Maybe this needs to be highlighted.

Callback: 2

This reporter states that this anomaly is covered in the supplemental section of the Flight Manual and that an attempt to restore the generator should not be made. This procedure is in opposition to the ECAM actions. Once the APU was up and running all electrical buses were powered and the flight continued to destination.

Synopsis

A320 flight crew experiences Generator 2 failure in flight and attempts to restore operation are ultimately unsuccessful. Generator 1 is unable to power AC Bus 2 which remains unpowered until the APU is started. Flight continues to destination.

ACN: 906393 (2688 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 906393

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 906394

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After take off and clean up passing 5,500 feet, and approximately 230 kts, the number two engine rolled back and shut down. The Captain was flying so we changed aircraft control and the Captain ran the engine failure checklist while I flew the aircraft. We leveled off at 6,000 ft and declared an emergency, and asked to return to our departure airport. We tried to restart the engine per the checklist but it failed to restart. When all checklists were completed we landed. From engine failure to landing was about 15 minutes. We landed normally and taxied to the gate.

Maintenance advised the fuel control unit failed.

Synopsis

An A320 Flight Crew declared an emergency and returned to their departure airport following the failure of the right engine shortly after takeoff.

ACN: 906151 (2689 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 100

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Final Approach

Component

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 906151
Human Factors : Confusion
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While conducting a night, VMC approach utilizing the VOR-DME approach, we received a Low Energy Warning ("Speed-Speed-Speed") at approximately 100 FT AGL which necessitated the use of TOGA thrust to limit further decay in airspeed and prevent a hard landing. The initial approach was normal and we were vectored to intercept the final while given a descent clearance to 1,600 FT and being asked to maintain our speed (185 KTS) until 5 miles out. Upon receiving the approach clearance, I armed the approach and immediately noted FINAL APP annunciated on the FMA. At that point the aircraft began to climb to 2,000 FT to capture the descent path. As pilot flying, it seemed to me that the aircraft didn't start the final descent at the top of descent point so I disengaged the autopilot and autothrust concurrently and started descending toward the PAPI approach path while configuring for landing. I attempted to reach stabilized approach criteria as quickly as possible but at 1000 FT AGL I was target +20 KTS and sinking 1,200 FPM with thrust at idle. At the 500 FT gate I was stable on speed and sink rate (target + 8 KTS, sink 900 FPM), but with the thrust still at idle (I had NOT re-engaged autothrust). I was maintaining the PAPI glide path to 100 FT when simultaneously the First Officer (pilot not flying) and I became aware of our below target airspeed while receiving almost immediately afterwards the LOW ENERGY WARNING. I responded by placing the thrust levers at TOGA to limit any further decay in airspeed and arrest the rate of descent. The resulting touchdown was firm and in the landing zone but otherwise uneventful. Taxi-in and parking was normal.

I have endeavored to closely follow SOP's throughout my career and take my responsibilities very seriously. I know the aircraft Pilot Handbook and FOM well and follow their guidelines rigorously and yet failed to conduct the approach described herein properly by neglecting to adhere to some of our most basic principles. In conducting the approach setup for the VOR-DME, I failed to utilize the QRH as an aid believing that I was proficient from having flown numerous VOR approaches in Mexico recently. I may have inadvertently failed to clear present position while flying the approach which may have caused the aircraft to climb to 2,000 FT instead of continuing the descent to the FAF altitude in open descent. The aircraft response certainly increased my task loading, but my response in disconnecting the automation removed a barrier that would have prevented the unstabilized approach that resulted. When the aircraft failed to descend at the expected point in the approach, my reaction in disconnecting autothrust was instinctive but ill-advised since as stated, it removed a barrier but also was not pre-briefed and is not authorized at night. Our descent below 1,000 FT AGL while unstable should have resulted in a go around with plenty of time and fuel to conduct another approach. Finally, inadequate monitoring of the flight instruments by the pilot not flying removed the final barrier that would have prevented the decay in airspeed. Very simply, my event should cause all of us to rededicate ourselves to following SOP's and guidelines as published in our manuals. My appreciation for the barriers which serve to prevent bad consequences and outcomes has increased immeasurably. Our job can become so routine that we fail to adequately recognize when an abnormal or non-routine situation has put us "into the yellow", and even two well-trained, conscientious and diligent professional pilots can make mistakes. The Pilot Monitoring has to monitor, even on beautiful clear nights and, most importantly, has to know what to monitor.

Synopsis

A320 Captain reports receiving a Low Energy Warning at 100 FT AGL during a night VMC approach. Autothrust was disengaged when the aircraft did not responded as expected when capturing the VOR DME approach path. TOGA thrust was used to prevent a very hard landing.

ACN: 906019 (2690 of 2908)

Time / Day

Date : 201008

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 906019
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were preparing cabin for landing when I noticed that the landing gear was being recycled 3 or 4 times. We were given the "prepare for landing" command and executed a go around. I discussed with the jumpseater that I thought there must be a problem with the landing gear and that I thought we'd be hearing from the cockpit soon. The Captain did call us and said that we were experiencing problems with the gear and they were in touch with Maintenance to determine plan of action. She then said to prep the cabin. I asked her if she meant for us to prep the cabin for an emergency landing and prepare our passengers she said no that she had just wanted to give us A Cabin Advisory (whew). She told us they were going to fly by the Tower to determine landing gear situation and then asked if there were any questions. I asked specifically where the problem was with the gear and she said it was the left main gear. I briefed my jumpseater that in that situation we would probably not use the left exits at least as primary and before opening any doors make sure to look for smoke fire metal etc. The attendants in the back briefed our 3 dead headers via notes who were sitting at the overwing exits so all exits were covered. After we landed brakes were very discreetly used and seemed to just kind of roll to a stop. Then you could tell there was some problem with steering the airplane. Captain called and told us that now they were experiencing problems with the nose gear and had lost the ability to make turns with the airplane. Mechanics came out to the aircraft and put pins in the gear and tugged us in. We were met by emergency vehicles which weren't needed and also met by Supervisor who was very supportive and understanding of the stress and anxiety we had just been through. The Captain did ask to meet with all of us and asked about communication via cockpit and crew and we agreed we worked through the event quite well, calmly and professional.

Synopsis

An A320 Flight Attendant described their cabin preparation procedure for landing with unsafe gear even though the flight crew had not commanded an emergency preparation. Very good communications within the cabin crew.

ACN: 905771 (2691 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ramp : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cargo Compartment Fire/Overheat Warning
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2600
ASRS Report Number.Accession Number : 905771
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 905775
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Set parking brake and completed parking checklist. Smoke warning aft cargo activated and First Officer opened his side window to confirm no fire/smoke with ground personnel. This was confirmed with cargo door already open, called Maintenance.

Narrative: 2

After block in, shut down, and with both cargo doors open, received an aft cargo smoke ECAM. First Officer opened window and confirmed with ground crew that all was OK. They confirmed that it was and emergency cancel button was depressed. First Officer then went down to cargo bay to reconfirm with ground crew. Maintenance alerted.

Synopsis

An A320 AFT CARGO SMOKE ECAM alerted after taxiing into the gate and setting the parking brake. The First Officer confirmed visually and verbally with the ground crew that no smoke was present.

ACN: 905659 (2692 of 2908)

Time / Day

Date : 201008

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 905659
Human Factors : Other / Unknown
Human Factors : Fatigue

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Attendant
Were Passengers Involved In Event : Y
When Detected : Pre-flight
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

I was called for the ZZZ turn in the afternoon with an early evening check in. I woke up that day early morning and had no idea the ZZZ turn was a possibility. I had no time to prepare for this trip...to take a nap/rest/eat quality food etc. This turn is a danger to reserve flight attendants who get assigned this trip at the last minute. If reserves are going to get this trip we must be allowed a break. If reserves are given this trip we not only need the break but augmented staffing and have an additional working flight attendant. On this flight safety was compromised because I was tired and unprepared to have a duty day near 13 hours beginning in the evening. Reserve flight attendants who are given this trip are set up to fail because we cannot be at our best. We cannot be the best safety professionals we know we can be when we are trying to stay alert. Safety cannot be jeopardized like this on an airplane. Flight attendants are truly the eyes ears and nose for the airplane and pilot and we cannot provide the best security of the passengers, crew, and aircraft when we have been up for 25+ hours. We have to have proper rest and advanced notice for a trip like this. I have been assigned this trip before on reserve and nothing has been done. I seriously hope the company and FAA will take these reports seriously because it is truly a huge danger when the safety professionals are exhausted.

Synopsis

Reserve Flight Attendant reports being assigned to an all night two leg flight with no prior warning, after sleeping normally the night prior and waking up in the morning.

ACN: 905620 (2693 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 34000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 905620
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 905591

Events

Anomaly.Other 
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight was progressing normally until, as we leveled off at FL340 and accelerated, we heard and felt the aircraft start to vibrate and rumble. In the cockpit, this rumbling came on rather fast. The Captain and I were discussing something interesting and both of us froze and fell silent as the rumble started. I first thought it was mach buffet, but I've never experienced mach buffet in the Airbus. I looked at the mach speed and it was only around .75. The Captain then commented that he thought it sounded like mach buffet and he selected a lower mach of around .71.

As the throttles came back, the aft Flight Attendant called and reported that she too was experiencing a very unusual buffet in the back of the aircraft. We pulled up the flight control page on the ECAM and observed no unusual indications. As we slowed, the buffet dissipated significantly. We informed ATC that we wanted to stop our climb. We had the Flight Attendant set up the cart, and I went back to inspect the rear of the cabin. The Captain and I both agreed that it was in the best interest of safety to try to find out if the vibration was from the rudder or the elevator. Before I left the cockpit, the Captain asked how I felt the flight should proceed. He finished the sentence with, "I'm leaning toward going back." I replied that I agreed, but I was really thinking oh thank God. I'm not going to have to convince this guy to do the right thing.

As I reached the aft cabin I did observe a vibration in the floor of the aft galley. It felt normal to me, and I asked the Flight Attendant if this was normal vibration on most flights. She replied that it was not. I then called the cockpit, and the Captain slowly increased the mach speed. As the speed increased, there was a point where the vibration increased significantly. The Captain maintained that speed for approximately 20 seconds as I physically felt around the galley walls and ceiling to try and locate the source of the vibration. With all the plastic absorbing the vibration, I was unable to definitively determine the source. The galley floor was definitely affected, but that did not help my investigation. On the interphone I informed the Captain of my lack of result, and he slowed the aircraft as I returned to the cockpit.

As I reached the cockpit, the Captain had already informed ATC of our return to departure airport and we were in a left 180. Before the turn was complete we had discussed how to proceed back to the airport with regard to speed and flap configuration and emergency status. The Captain commanded me to declare the emergency and I did so, giving ATC SOB's (souls on board) and fuel in hours. We agreed that the landing should be a normal flaps full landing but that it should be VERY stabilized and we should require the longest runway. ATC was informed of this too, while we simultaneously informed Dispatch of our return and emergency state via ACARS. During decent, we discussed Cabin Advisory vs. Cabin Prep, and decided that a Cabin Advisory would be sufficient for the situation. The call was made to the Purser, and she was informed of this and the time remaining.

Emergency crews were requested upon arrival with both ATC via radio, and Dispatch via ACARS. Overweight landing was discussed, but the gross weight (140.0) was already below the limit. I checked the landing distance on the card and determined that runway was more than sufficient for the landing. The quick reference diversion guide was checked with nothing missed or new. The flight proceeded very normally from FL180 on to touch down with the exception that we were set up very early and were slower. On 5 mile final, we informed Tower of our planned runway exit so that the emergency crew could be aware of our plans. From 1,000 AGL the flight was a normal, everyday flight. As we cleared the runway, we canceled the emergency and proceeded to our gate with no further complications.

Callback: 1

The reporter advised that he understood through a hearsay conversation that one or more rudder PCUs were replaced and the aircraft is back in revenue service. He stressed that this was not first hand knowledge although he did know the aircraft was flying.

Synopsis

A rapid onset of airframe vibration at cruise Mach caused an A320 flight crew to return to their departure airport for maintenance.

ACN: 905612 (2694 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 905612

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During cruise, First Officer discovered the Green Hydraulic System quantity indication decreasing. It appeared to be a slow leak, but also thought it might be due to cold soaking. While monitoring the situation, I sent an ACARS message to Maintenance explaining the quantity indication was approaching the low level amber box, but no response. Sent another message asking if anyone was home and still no response. Contacted Dispatcher and he established a VHF conference call. All parties agreed we would be prepared in case the Green System failed. Instead of waiting for an ECAM to appear, we decided to be proactive and review the irregular procedure for Green Hydraulic Low Level, accomplish the performance computations and brief the Flight Attendants on the potential of a cabin advisory in advance. Upon initiating our descent received an ECAM for Green Hydraulic Low Level and accomplished the procedure. I then notified ATC of our issue and after a brief discussion with the First Officer, elected to declare an emergency as a precautionary measure. We then sent an ACARS to Dispatch and Maintenance confirming our mechanical problem and briefed the Flight Attendants on a cabin advisory, followed by a PA to the passengers informing them of situation and what they could expect. Fortunately, when we turned the Green Hydraulic Pump back on, per the ECAM, we regained enough hydraulic quantity to put the landing gear down, land without incident and taxi to the gate. CFR followed us to the gate and the flight was complete at this point. 

NOTE: I felt the entire crew, Dispatcher, ATC and CFR all performed extremely well, but with respect to Maintenance I have some concern. Throughout the majority of my career, Maintenance was considered to be a valuable resource and the technical experts we could rely on. I do not find this to be the case anymore, especially with the individual that was on duty during this event. Unfortunately, Maintenance Control has become extremely inconsistent, depending on which controller we draw for the day. This is a non-emotional comment and I strongly believe this issue needs to be addressed. Also prior to our landing, ATC departed a heavy aircraft, which left behind some wake turbulence we had to confront during flare. Had we been dealing with a controllability issue, this would not have been a good situation. I called the Tower from my hotel that evening and briefed them on what had transpired.

Synopsis

An A320 Captain reported losing Green hydraulic system fluid in cruise flight. They ran the procedure, and after coordinating with Dispatch and Maintenance decided to declare an emergency. A normal landing was made although they encountered some wake turbulence on short final.

ACN: 905607 (2695 of 2908)

Time / Day

Date : 201008

Place

Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 905607
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Primary Problem : Ambiguous

Narrative: 1

Taxiing into the gate everything was behind the line. There was a belt loader parked on the Captain's side of the plane with someone sitting in it. As I was taxiing in and about abeam him he moved the entire belt loader out of the area from behind the line and into my protected boxed area. He was looking in the opposite direction. Not only was it very distracting it was also unsafe.

Synopsis

An A320 Captain reported inappropriate vehicular activity within the protected area while parking her aircraft at the gate.

ACN: 905263 (2696 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Turbine Assembly
Manufacturer : International Aero Engines IAE
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Engine Oil Seals
Manufacturer : International Aero Engines IAE
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Function.Maintenance : Technician
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13878
Experience.Flight Crew.Last 90 Days : 256
Experience.Flight Crew.Type : 3254
ASRS Report Number.Accession Number : 905263
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Aircraft had a history of high oil consumption since August 2010 (or earlier). Special care was given to engine #1 on walk-around and when observed shortly after shut-down (approximately ten minutes) engine #1 had a pool of fresh oil near the rear section of the engine with fresh oil observed dripping from the bottom of the rear cowling. 

Maintenance Control contacted and Mechanic did inspection of leak area. It was determined by Maintenance Control an engine change was required. Report filed because of multiple Inspections of engine prior to this walkaround failed to notice any obvious leaking engine section. While the engines do not normally leak from the rear areas....an inspection by company maintenance should have identified leak prior to multiple dispatches. Obviously the continued servicing of the engine with large quantities of oil was another indicator of a problem and should have flagged the issue long before this event.

Synopsis

A Captain reports an A320 with a history of high oil consumption on the #1 engine. During a post flight walk-around shortly after shut-down, he observed pool of fresh oil near the rear section of the engine and oil dripping from the bottom of the rear cowling. Engine change was required. Pilot concerned about multiple dispatches by maintenance of an aircraft with known oil leak.

ACN: 905258 (2697 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZOA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZOA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZOA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12228
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 2710
ASRS Report Number.Accession Number : 905258
Human Factors : Training / Qualification
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Prior to descent I asked the First Officer to inform me when he was ready for a briefing for the ILS, [but] when he informed me he was "ready", he had no approach plate out and didn't care. I asked him again and he informed me the frequency and identifier were located on the PFD. I asked him for a third time and he pulled out his publications binder and "half" followed me through the approach and then put away his binder with an "attitude". Last month I had a similar incident on arrival when an First Officer informed me he didn't need to pull out any publications during my brief. After explaining company SOP he conformed and I had no further problems during the four day trip. In June, I had a similar experience prior to approach with an First Officer who would not pull out any publications during the entire flight. Is this a trend and are other Captains having these problems? I'm not back in the Air Force flying a single seat aircraft. This is a dangerous trend of First Officers deliberately being cocky, complacent, and arrogant. I strongly believe flight standards should issue a bulletin to Airbus pilots explaining the seriousness of this matter especially at airports they're familiar with.

Synopsis

A320 Captain believes that some First Officers at his company are becoming complacent and are not complying with SOP when it comes to approach briefings.

ACN: 904988 (2698 of 2908)

Time / Day

Date : 201008

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Exterior Pax/Crew Door
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 904988
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Primary Problem : Ambiguous

Narrative: 1

My flying partner and I advised the Captain that door 2R's door pressure gauge was reading 1,350 and according to numbers posted on the door should be closer to 1,800 PSI. The Captain came back to door 2R and confirmed the door pressure gauge read 1,350. The Captain contacted mechanics to look at door 2R. A Maintenance Supervisor who declined to identify himself wanted to talk with me and questioned why I was checking the door pressure gauge on the aircraft? He asked me if it was in my Manual as a safety check. I told him at one time it was but it was no longer. I told the Maintenance Supervisor that we noticed the discrepancy and felt it was important to tell the Captain and have it checked. In the presence of the Purser the Maintenance Supervisor made the following statement: "Checking the door pressure gauge is not your responsibility and is a waste of my mechanics time because the door pressure gauges are checked periodically by my department." To my knowledge the door assist pressure was not filled and we left the gate on time. Every door on the Airbus has posted on each door numbers for door assist pressure gauges according to outside temperatures. The outside temperature for 1,350 PSI is in the neighborhood of -40 C. The temperature this day was approximately 26 C (80 F). Are the posted door assist gauge numbers a "suggested" minimum level or are they the minimum pressure for safe usage of an Airbus door assist during an evacuation? If they are the minimum and no additional pressure was added did we leave approximately 400-500 PSI lower than the minimum? If I see or encounter anything that appears safety related should I ignore it because it is "not my responsibility" to check it? I do not believe that pointing out an apparent discrepancy in any safety related situation is a waste of time.

Synopsis

A320 Flight Attendant discovered a low pressure reading on a door assist bottle during preflight and informed the Captain. The Flight Attendant was subsequently told by a Maintenance Supervisor, that it is not her duty to check the bottle pressures, no maintenance was done prior to departure.

ACN: 904970 (2699 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 904970
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Since our airline has been charging for checked baggage I have noticed that very few bags are being checked and that customers are being allowed much more than their allowed carry on bag allowance. With 168 passengers we had only 58 bags including 18 that were forced to be gate checked because the over head bins were completely full. The bins were full prior to the boarding of the final 30 passengers. I questioned the gate agent about the low bag count and he assured me that it was correct. Upon reaching cruise altitude I began the process of comparing our dispatch release planned weight and the weight that we calculated at take off. There was more than a 2,000 LB weight difference. I then checked the weight that the aircraft uses for all of its calculated speeds and configurations. At top of climb after burning 5,500 LBS of fuel using the weight and balance calculation we should have weighed 148,400 LBS. The aircraft FAC [Flight Augmentation Computer] calculated weight (both FAC 1 and 2) however was substantially higher showing a calculated weight of 152,680 LBS; a difference of 4,280 LBS.

The discrepancy was determined at top of climb using the aircraft AIDS FAC [Aircraft Integrated Data System; Flight Augmentation Computer] calculations. This situation occurred because our airline began charging for checked bags but does not enforce the passenger carry on bag policy; allowing the passengers to carry on much more than their personal limit. The flight was en route and there is no way to identify this problem while on the ground.

Our staff must be trained correctly on carry on baggage allowance, size and restrictions. They need to be trained that NO baggage that does not meet these requirements shall be brought onto the aircraft. The company cabin crew members would require the same training.

Synopsis

An A320 Captain noted the gross weight of the aircraft based on company supplied weight and balance figures is routinely substantially less than that calculated by the aircraft's on board data computer system. Reporter believes the discrepancy is the result of the airline's policy of charging for checked bags which has resulted in very few being checked and most being brought on board as "carry on" luggage by the passengers. Such bags are often much heavier than the standard allowance for a passenger and his/her carry on luggage as established by the FAA.

ACN: 904969 (2700 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 904969
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Analyst Callback : Attempted

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Upon arrival I was informed that 5 cases of alcohol "COMAT" had been removed from cargo bin. These cases had not been listed on the weight and balance form. We were not aware that this had been put on aircraft. Aside from the fact that carriage of alcohol in this manner is questionable under current company and HAZMAT policies, it is highly irregular to put undocumented cargo on board. The ramifications of this are dangerous and unsafe. I had no idea what I was carrying, was it dangerous, hazardous, illegal? Who did it belong to? The need to document correctly and notify flight crew of every item being carried aboard other than regular passenger bags is paramount to the safe operation of the aircraft.

Synopsis

A320 Captain is informed after gate arrival that the cargo bin contained some unlisted COMAT that consisted of five cases of alcohol.