ACN: 904764 (2701 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Interphone System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 600
ASRS Report Number.Accession Number : 904764
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 904960
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Person / Animal / Bird
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

When ground person hooked up to the interphone, it was very weak and I had to ask several times during push back process to "say again". Push back was normal until after the clear to start engines clearance was given. The First Officer and I were discussing T-procedure and terrain issues, during which time I heard the tug driver talk. Thinking (incorrectly) it was at the point during push back that the response was to "disconnect headset" I said that to the tug driver. When I saw tug driver back away from airplane I noticed the plane start to roll forward. The plane rolled several inches to about 1 FT as near as I could tell when I immediately applied brakes and stopped the airplane. At that point the normal "salute and release from guidance" procedure was accomplished and we taxied out. Thank God no one was injured and no equipment was damaged. Fatigue was a factor due to previous long duty days on this ID. I think the low volume during the push back was also a factor; otherwise it would have more obvious what point in the push back that we were in.

Narrative: 2

During push back the Captain and I were clarifying some T Procedure elements. In the future I, as a First Officer, will be certain that there is no distraction through conversation until we have received a release from guidance.

Synopsis

An A320 flight crew failed to insure the parking brake was set prior to clearing the ground crew to disconnect the tow bar. A weak interphone system and the flight crew's preoccupation with briefing special takeoff procedures during the pushback contributed.

ACN: 904125 (2702 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Main Gear Door
Aircraft Reference : X

Component : 2

Aircraft Component : Hydraulic Lines, Connectors, Fittings
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 904125
Human Factors : Confusion
Human Factors : Situational Awareness
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
Experience.Maintenance.Technician : 21
ASRS Report Number.Accession Number : 904218
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

It has been six weeks, to best of my memory I removed valve that was called out, was sent wrong valve, had to wait for new, correct valve. Changed valve, did not safety attach bolts where the two valves meet, it did not come in safetied and there is no access to safety. The attach bolts on the forward side were safetied as there was access. Operational check with fellow Mechanic who pressurized hydraulics, I operated manual controls for gear doors from ground, leak check was good, no leaks, message went away, signed off item.

Callback: 1

Reporter stated there are two valves that come as one unit; one valve for each Main Landing Gear (MLG) door. They are interlocked together and he doesn't know how anyone could safety wire the inner bolts, because of no way to access them. There are six bolts total. He believes the valves are the Sequence Valves for the MLG doors.

Reporter stated the valve was not leaking, but was found with the inner bolts not safetied when the A320 arrived at a Contract Maintenance facility for a Heavy Check.

Narrative: 2

Changed a landing gear valve. Hydraulic circled out. Message went away. Signed off item. Six weeks later was called as [valve] leaking and not safetied.

Synopsis

A Line Mechanic reports he did not safety wire the inner attach bolts for the Main Landing Gear (MLG) door sequence valves on an A320.

ACN: 904120 (2703 of 2908)

Time / Day

Date : 201008

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 904120
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Working crew (flight attendants and cockpit) were not aware that we had armed FAMs on board in first class. Flight attendants were not aware that the Captain was armed (due to lack of briefing by the Captain and lack of CLR). The FAMs did not know the Captain was armed (due to lack of CLR and proper briefing with Captain) and in turn the Captain was unaware of the presence of FAMs on the flight. In addition I as Purser was completely distracted from my boarding and Pre-Departure duties. I was not able to provide any safety and security vigilance that is expected of me during the boarding process. 

This was all due to the fact that our Captain became upset over me serving him his coffee with cream on the side instead of inside the coffee. As the result of this a confrontation ensued between myself and the Captain that lasted throughout the boarding process. 

The action taken by the Captain and the inflight supervisor was to remove me from the Purser position. My flying partner took over being Purser to appease the Captain. However that did not change the fact that the working crew (both flight attendants and cockpit) was unaware of the armed FAMs onboard and as well as the FAMs were unaware that the Captain was armed.

Synopsis

An air carrier Flight Attendant reported flight attendants and pilots were both unaware that FAMs were on board, and the flight attendants and FAMs were unaware the Captain was an FFDO. Problem reportedly stemmed from lack of communication resulting from friction between the purser and the Captain before departure.

ACN: 903940 (2704 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : DCA.Airport
State Reference : DC
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : Marginal
Weather Elements / Visibility : Haze / Smoke
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : PCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Nav In Use.Localizer/Glideslope/ILS : LDA 19
Flight Phase : Final Approach
Flight Phase : Initial Approach
Airspace.Class B : DCA

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 903940
Human Factors : Training / Qualification
Human Factors : Fatigue
Human Factors : Situational Awareness
Analyst Callback : Attempted

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Late night arrival, we were first cleared for the visual to Runway 19. It was too hazy and dark for us to pick up the river in time. We were vectored around to the LDA approach. We quickly reviewed and briefed the approach and commenced the approach. The visibility was poor, though legal, for the approach. It was very difficult to see the visual landmarks and do the circling part of the approach. We were very aware of avoiding the prohibited areas and may have inadvertently strayed too far west. It was VERY difficult to see our exact position as it was after midnight on a very dark night with haze. I had never done this approach in my 30+ years of flying and we were both very fatigued. We made every effort to fly the approach correctly but this was, by far, the most difficult approach I have ever flown in my 5 years on the Airbus. I do not feel that the Commercial Chart pages and depictions are adequate for a night approach by someone unfamiliar with the airport and the extremely difficult approaches, especially at night and in reduced visibility. This approach should be a simulator training approach instead of a very simple NDB!

Synopsis

A320 Captain describes difficulties encountered during the LDA DME 19 approach to DCA. The approach occurred late at night in reduced visibility and was the first time for this reporter. Reporter believes that simulator training should be required prior to flying the actual approach.

ACN: 903932 (2705 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Landing
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 75
Experience.Flight Crew.Type : 4500
ASRS Report Number.Accession Number : 903932
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 903948
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Up until this point, normal approach and landing, short final at 1,000 FT AGL when the ECAM's flickered on and off indicating a potential issue. Discuss if this was an anomaly or not and saw other indications that we had lost hydraulic fluid in the blue system. I turned off the blue hydraulic pump and status page showed loss of both number 3 spoilers. Quickly discussed implications, went through pages and reported ECAM complete. First Officer on top of situation and concurred with me when I made decision and stated we should continue to land rather than go-around. Runway was very long with a 19 knot head wind, so landing distance was not a factor. Maintenance met aircraft and discussed situation. Maintenance notified us blue system had a severe leak in left wheel well. System still had fluid in blue system after engine shutdown. On preflight, both Captain and First Officer verified all systems and fluid level were normal include the blue system. First Officer did not note any issues on walk around prior to departure.

Narrative: 2

Normal VFR approach and landing. At approximately 1,000 feet AFE ECAM sounds and reports a blue hydraulic system low level. Captain turns off the blue hydraulic pump and system page goes to hydraulics. We note that the level is nearly empty. Status page notes we've lost both number 3 spoilers and the blue elevator flight control. We note the increased landing distance is not a factor on the 16,000 foot runway plus a 19 knot headwing component. Captain clears the ECAM and reports ECAM complete, screens normal. Continue approach to a normal landing. Turn off the runway and taxi to the ramp. Notified Maintenance and they found a severe leak in the left wheel well. Blue hydraulic system checked normal on preflight with the level on the slightly high side but within limits. No leaks noted on the walk around.

Synopsis

A320 flight crew experience blue hydraulic system fluid loss at 1,000 FT on final approach. ECAM actions are accomplished and flight lands normally.

ACN: 903677 (2706 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Other Fire Extinguishing System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13150
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5605
ASRS Report Number.Accession Number : 903677

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 903678
Human Factors : Other / Unknown
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Approaching destination the First Officer left his seat to stretch and await the Flight Attendant to enter the cockpit for a restroom break. As he stood up, his head struck one of the agent toggle switches on the cargo smoke overhead panel. Approximately 15-30 seconds later, the "smoke fwd cargo smoke" ECAM appeared momentarily. After further observation we noticed that the "DISCH1" light was illuminated on the cargo smoke panel, indicating that the 1st bottle of smoke retardant had been fired into the forward cargo pit. After reviewing the Flight Manual and discussing the situation, we determined that the ECAM was the result of retardant being inadvertently discharged into the forward cargo pit. Upon taxi-in the second bottle automatically discharged itself into the forward cargo pit. Once parked at the gate we were met by a Maintenance Supervisor. After our debriefing with the Maintenance Supervisor, I spoke to Flight Operations and was told sufficient action had been taken on our part, but we should still file a report.

Narrative: 2

With approximately 45 minutes remaining I made arrangements for personal lavatory needs. Anticipating the Flight Attendant, I slid back my First Officer crew seat and stood up. As I did the top of my head made inadvertent/unintentional contact with the overhead panel. I briefly looked up to see if any switches or knobs were mispositioned. Not noticing anything out of the ordinary I continued to the back of the cockpit near the door. (It should be noted there was no injury to my head and no remaining pain because the bump was light and brief.) Just as I reached the back of the cockpit the ECAM warning sounded. When I turned to see what it was I saw a red ECAM message but it quickly extinguished. Recall attempts by the Captain at the controls were unsuccessful. He scanned various ECAM pages and all appeared normal while I let in the Flight Attendant then returned to my crew position to help assess the situation. As I returned I noticed an illuminated light on the overhead panel near where I bumped my head. It was DISCH1. We pulled out the Airbus Flight Manual and looked up the exact meaning of the light: Cargo 1 agent had been discharged. We then reviewed and discussed the situation: I must have inadvertently bumped the guarded agent discharge switch with my head; agent was discharged into the cargo compartment; as the agent discharged the cargo smoke detectors detected the agent and sounded the warning ECAM; when the agent stopped discharging the air cleared and the ECAM no longer reported an anomaly. The Captain and I were 100% certain there was nothing requiring an immediate landing and we elected to continue to our destination. While enroute we contacted Maintenance Control via ACARS and told them of the situation. We requested ramp be notified to NOT open the pits, a mechanic meet us at the gate and only with the mechanic's instruction should the pits be opened. Furthermore, the Captain and I discussed the likelihood the second bottle of agent would be released upon landing. During taxi in, I contacted Ramp AND Maintenance to once again ensure the pits would NOT be opened without specific instructions from a qualified mechanic. Both sections ensured me all parties were notified. Upon arrival at the gate we were met by a Maintenance Supervisor. We explained to him the above and asked what was needed from us as follow-up. He said as far as he was concerned we had complied with all requirements.

Callback: 2

The reporter states that this particular A320 had an older switch design that was actually a switch and not a recessed push-button. The guard was also quite different from the current design, which allowed this incident to occur. The reporter was advised by maintenance that accidental discharge was quite common, thus the new design.

Synopsis

A320 First Officer reports inadvertent contact with the guarded cargo agent discharge switch while standing up to leave his seat at cruise altitude. Bottle one is discharged causing a momentary ECAM warning. Bottle two automatically activates during taxi in.

ACN: 903455 (2707 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 903455
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During climb through FL260, we heard a snap and immediately noticed First Officer's windshield had large spider-like cracks. Immediately leveled off at FL260; communicated with Center describing situation, worked the QRH, notified Center that we needed to return to our departure airport. Communicated with Flight Attendants, passengers, departure airport operations and Maintenance Control. Performed overweight landing procedures in QRH; landed at about 151,500 LBS. First Officer performed duties of pilot flying and pilot not flying flawlessly. The Flight Attendants did an exceptional job keeping the passengers calm.

Synopsis

An A320 First Officer's windshield developed cracks climbing through FL260. The crew complied with the QRH and returned to the departure airport.

ACN: 903452 (2708 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 16000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Powerplant Fire Extinguishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 30000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 700
ASRS Report Number.Accession Number : 903452
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climb out all fire extinguisher squib circuit breakers opened. Called Maintenance and Dispatch and we decided to return to our departure airport. Uneventful overweight landing.

Synopsis

All of an A320's fire extinguisher squib circuit breakers opened shortly after takeoff. The crew returned to land at the destination airport.

ACN: 903390 (2709 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : CVG.TRACON
State Reference : KY
Altitude.MSL.Single Value : 4000

Aircraft

Reference : X
ATC / Advisory.Tower : CVG
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb
Route In Use.SID : BENGL RNAV
Airspace.Class B : CVG

Person

Reference : 1
Location Of Person.Facility : CVG.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 903390
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Aircraft X departed Runway 8L CVG on the BENGL RNAV SID. This SID is flawed in that aircraft departing the east runway on the RNAV SID, if the SID is flown, turn right, across the other 2 departure runways, to get to the programmed fixes on the departure. CVG has instituted a series of workarounds and memory joggers to keep the pilots from flying the SID, instead of NOTAMing out this and 2 other SIDs with similar problems. In this case, the aircraft departure strip that was received in the TRACON indicated that the aircraft had been assigned instructions to fly the published noise abatement noise track, which turns to a 165 degree heading to provide separation from Runway 18C departures. The aircraft did not turn, and their track instead appeared to follow the runway heading. No loss of separation resulted. This departure, and the HAGOL 18L departure, and the KENLN 36R departure, should be NOTAMed out of service until the SIDs are fixed, which is supposed to be in the works.

Synopsis

CVG Controller expressed concern regarding the BENGL RNAV SID procedure suggesting the implemented workaround efforts are not successful and that the SID should be NOTAMed out of service.

ACN: 903113 (2710 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component : 1

Aircraft Component : Normal Brake System
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 130
ASRS Report Number.Accession Number : 903113
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8800
Experience.Flight Crew.Last 90 Days : 25
Experience.Flight Crew.Type : 480
ASRS Report Number.Accession Number : 903269
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

When crew arrived, brakes were not set and "HOT BRAKES DELAY TO" ECAM was indicated. Brake temps were slightly below 400 approximately 45 minutes prior to departure. While pre-flighting the aircraft, ramp crew also disconnected external power without notifying crew and while the APU was off. Crew immediately started APU. When aircraft powered up, we had numerous spurious warnings including Master Warnings and Master Cautions that lasted several minutes. Ramp personnel came to the cockpit and informed us that they had proper disconnect indications at the external panel. Ground Supervisor came to cockpit with Shop Steward and informed me that he was going to reprimand the ground crew. We quickly did another preflight to ensure proper departure configuration. We flew the short flight and landed with auto-brakes off, on runway turnoff at the last high speed the brake temperatures were even on all brakes and approximately 80. We taxied to gate and on arrival brake temperature had risen to approximately 280. We had a short turn and requested the wheels to be chocked and fans put on the brakes. Maintenance chocked the wheels but told us they had no fans. We were then assigned to fly another leg in the same aircraft. Prior to departure, ramp personnel called several times to have us set brakes so they could remove the chocks, but we delayed until close to departure and the temperatures had subsided to "Green" indications at approximately 150. We set brakes and pushed on time. During taxi out all checklists were completed and indications appeared normal. At power application we received ECAM FLEX TEMP NOT SET. The proper Flex temp from the RWY DATA message was set, but we immediately increased thrust to reject takeoff per SOP (Flight Manual Normals 10.40.2) and continued takeoff. At 80 KTS, when we checked THRUST SET we noticed we had a red SPD SEL indication (selected speed data failure) and no speeds on both PFD's (although speeds were set in the FMGC with no amber boxes or CHECK TAKEOFF DATA MSG), and being that we were in the low speed regime decided that it was prudent to abort the takeoff. We aborted takeoff and did not achieve max braking; we cleared the runway and coordinated with Ground Control. They immediately asked why we had aborted the takeoff and we informed them that we had an indication malfunction. They asked if we needed assistance. We informed them that we did not need emergency vehicles at that time, but were coordinating with maintenance to have our wheels chocked. We did not set parking brake and immediately referenced the QRH. We determined that our minimum ground waiting time was 46 minutes based on an overly conservative speed of 120 KTS and 150,000 LBS (we were a lighter weight and aborted at a slower speed). We taxied slowly and shut down one engine. We reiterated our need to have the brakes chocked. Maximum temperature was 735 ten minutes after the abort, and was indicated on left brake #2. We continued to slowly move position to facilitate a rapid response from Maintenance. We were told to go to a Spot at the maintenance area to await chocking. We continued to ask for a rapid response from Maintenance, but were unable to coordinate through the Operations frequency. I was holding the brakes manually during this time and slowly rolling when we needed to move. After waiting approximately 15 minutes near the maintenance area, Station Operations informed us that we needed to go to a terminal gate. At this time brakes had already cooled to 600 so we obtained clearance and taxied toward the gate. While turning into the alley we stopped to confirm the gate location and heard a loud bang indicating a tire explosion. We immediately asked Ground Control if they could see anything other than a flat tire and both ground vehicles and passing aircraft all reported no indication other than flat tires. We decided to not evacuate the aircraft and requested emergency vehicles. We notified the Flight Attendants and passengers via the PA they were to remain seated and NOT to evacuate the aircraft when they saw the emergency vehicles. At this time we were simultaneously talking to Ramp, Ground, waiting for Maintenance on the interphone, and talking to the emergency vehicles (who informed us that their infrared indicated Hot Brakes). We coordinated with Ground Control and had a people mover and air-stairs brought to the airplane so the passengers could be deplaned. When Maintenance came on board they informed us that the aircraft had previous brake issues and BSCU faults. The brakes had recently been bled to correct a hot brake issue. They informed us they were concerned the brakes were binding. We made following log write-ups: 

1) Crew Report Hot Brakes. 
2) Aborted Takeoff due to Speed Select failure on Captain's and First Officer's PFD. Said SPD SEL in red. Flex Temp was imputed and displayed but on initial power application warning of FLEX TEMP NOT SET appeared. TOGA then selected per SOP max speed approx 110 KTS taxied off runway parking brake not set brakes 620 on all 4 brakes. 
3) 10 minutes after abort max temp #2 brake was 735 all remaining brakes within 30 degrees. Aircraft weight was 148,000. 
4) After applying TOGA thrust due to FLEX TEMP NOT SET experienced Selected Speed failure on both PFD's at approximately 80 kts. Aircraft experienced in an external power cord removed on previous preflight and various electronic indications appeared then all back to normal after 10 minutes. 

When Maintenance boarded, they informed us that the two left tires had blown, and the two right tires had deflated. The flight manual states that that blowout plugs prevent over pressurization due to heat buildup, and "fusible plugs" melt in the event of an overheat to protect the tires from bursting. Why did they not work as advertised in the burst tires? A more timely response and better coordination by Station Operations and Maintenance would have allowed us to avoid this situation by having the wheels properly chocked and allow them to cool instead of having us taxi to the maintenance facility then back to a gate.

Callback: 1

The reporter stated that when maintenance personnel queried the computer after the flight a BSCU fault was seen which would explain the right wheel overheat and tire failure. He is puzzled why the brake fuses did not deflate the tires before tire failure occurred. The other issue of the FLEX TEMP NOT SET ECAM is still puzzling. Both pilot saw the speeds in the FMGC and on the PFD before taking the runway. When power was being set the speeds were no longer present and the ECAM alert appeared. The speeds remained in the FMGC but were not displayed on the PFD. The mechanics the Captain has spoken with since then do not want to talk about the issue of the disappearing speeds. The reporter has spoken with other pilots about it and apparently this has happened before. His carrier is replacing the PFD's with a non OEM type and the reporter wonders if there is an interface issue. In one instance the reporter heard of an aircraft overspeed after takeoff because the autothrottle system did not go to climb and the crew did not realize that because the speeds dropped out and climb power would not be selected.

Synopsis

An A320 with a brake overheat history had tire failures while returning to the gate after a rejected takeoff for a FLEX TEMP NOT SET ECAM on takeoff.

ACN: 902881 (2711 of 2908)

Time / Day

Date : 201008

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 902881
Human Factors : Fatigue
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

This flight left two and half hours later than planned due to a mechanical. It arrives in Anchorage and flight crews come back. It is already scheduled as a 13 hour duty day. With a two hour delay we were at a 15 hour duty day. Flying all night with no ability to rest can be very dangerous. This flight arrived in Anchorage [in the middle of the night home time] and it was turned in 40 minutes. There was absolutely no time to close one's eyes for even a minute. The flight is five hours each way. There were several times on the way back that I was not fully alert and was beginning to nod off. I became very drowsy and tripped a couple of times as I was walking up and down the aisle to check on seat belts. It would have been very helpful to have a place to shut my eyes for a brief power nap to enable me to be more alert during the flight. I would suggest a crew rest seat be made available for this particular flight for flight attendants to be able to rotate a brief rest during this long flight in the middle of the night. After we landed [at home] and we were told to disarm doors I found myself having to concentrate very hard on the door to ensure that I disarm correctly. Additionally during my drive home I found myself nodding off.

Synopsis

A Flight Attendant reported extreme fatigue on an all night two-leg flight to ANC and back.

ACN: 902737 (2712 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component : 1

Aircraft Component : Rudder Control System
Aircraft Reference : X

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 7900
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 500
ASRS Report Number.Accession Number : 902737
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

We pushed back performed all our checks to include the flight control check. We were given takeoff clearance I added power and set the thrust levers to Flex EPR. As I began to blend in rudder steering with nose wheel tiller steering I felt that the rudder was not performing with enough authority. I relaxed the tiller to natural thinking I was cross controlling the two. I was still not able to track centerline with rudder inputs alone and came on with tiller. I tried releasing the tiller when I was on center line and took over with rudder pedals alone and began tracking to the left of centerline. At this point I did an aborted takeoff, the winds were straight down the runway. As I taxied clear I tested the rudders and the right rudder felt soft and did not provide the correct amount of authority. I asked the First Officer to try his and he later commented that the right rudder on his side felt soft.

Callback: 1

The reporter did not have any information to add to his narrative except to state that he felt the issue was rudder pedal control of nose wheel steering and not the rudder itself that was malfunctioning.

Synopsis

A320 Captain reports lack of rudder authority during takeoff with winds down the runway and rejected the takeoff.

ACN: 902734 (2713 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Ground : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 3400
ASRS Report Number.Accession Number : 902734
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 902742
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Detector.Person : Passenger
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the initial cockpit check, the APU FUEL LP VALVE ECAM came on. Wrote maintenance write up and maintenance came up to try to fix it. Couldn't, so they deferred it which in opted the APU. Had to do an engine start at the gate (left engine). Pushed back normally. Since we had a wheels up of 44 minutes past the hour, we taxied single engine. About 10 minutes prior to wheels up we started the right engine. (all parameters normal). Had about 8 minutes on the right engine when we were cleared into position on to runway 22R. Still everything normal. First officer was his leg. We were cleared for take off. Again all was normal, about 100 mph heard a noise and (thought the right tire had blown) felt the airplane lurch slightly. By then 110 kts. I told the First Officer I had the airplane and did a high speed abort. Everything went like clockwork. The First Officer was sure it was the nose gear; I felt it was the right main. Ground control said saw no smoke so we taxied to the gate (16). They had us wait with one engine (left) running waiting for a Customer Service Representative at gate 16. After 5 minutes with left engine running our purser called and said a passenger at 21F had seen sparks and fire from the right engine. Now all this time we thought it was tire problems. I got on the operations frequency said we needed a Customer Service Representative to get the jetway now! Finally after a couple of minutes they came. I ordered the passengers off which I'm sure the Customer Service Representative's would have done too. Our abort went like a perfect abort. First Officer and I worked very well together. Only fault I had with myself, we did not declare an emergency. I had thought it but again we thought it was only a tire issue especially when Airport Operations said everything looked fine outside. My thought it was tire debris that made the airplane swerve slightly. We had no loud sounds in cockpit; No ECAMs came on. Everything looked normal. When maintenance came on board, found on CIDS that the 2 engine stall occurred. We were surprised then I spoke with the passenger in 21F, and it was a textbook scenario of a compressor stall. Sparks then none when power brought up then on takeoff he heard a boom and a ball of fire coming out of the right engine. Again, my surprise was how quiet it was in the cockpit. No indications etc. either. My complaints would be as follows: 1. Lack of communications and follow through on the part of Company Operations. 2. Shows on the CIDS we had an engine stall but nothing on ECAM. 3. I should have regardless what happened declared an emergency. Can always retract it for future reference for myself.

Narrative: 2

The Captain thought what ever it was it was on the right side of the aircraft. As we exited the runway a Airport Operations vehicle was right there and reported no smoke from anywhere on the aircraft and we taxied back to Gate with Airport Operations following us. (Note: No aircraft in the long line of departures offered any inputs to our queries on the Tower Frequency about our status). The Captain suggested I make a PA and call the flight attendants. I made a PA to the passengers and told them we suspected we blew a tire and would have to go back to the gate and have the aircraft inspected. I called the flight attendants to ensure they were OK and see if they had any inputs. We arrived back at Gate and asked for the wheels to be chocked and for the Pax to be removed as we suspected this was going to take a while to come to a logical conclusion maintenance found all the wheels were fine and were getting ready to look at the engine 2 when I pulled up the ECAM report in CIDS and it showed an ENG 2 STALL. (Note: Why was it in CIDS and we got no ECAM on the ECAM Display at the time of occurrence?) We gave the CIDS report to the mechanics. The Flight Attendant told us when we opened the cockpit door a passenger had seen sparks and fire from the number 2 engine during the takeoff roll. The Captain debriefed the passenger on what he saw. We debriefed with the Flight Attendants and an ALPA Safety Representative who happened to be onboard the aircraft.

Synopsis

An A320 crew rejected a takeoff at 110 knots thinking that a tire had blown. At the gate it was discovered that a passenger saw flames from the right engine which was confirmed on the CIDS by maintenance as a compressor stall.

ACN: 902730 (2714 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 27000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise

Component

Aircraft Component : Hydraulic Fluid
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 27000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 9000
ASRS Report Number.Accession Number : 902730
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

At level off, FL270, ECAM for Yellow system hyd quantity low. Accomplished ECAM steps, verified low Yellow system quantity and notified dispatch and maintenance via dispatch of problem. Had dispatch notify Tower of election not to declare emergency and request for Runway XXL to minimize taxi distance. Also had dispatch notify ramp of need to chock all wheels due to no parking brake. Notified A flight attendant of situation and that they could expect a normal landing with no need for a cabin advisory or need to inform passengers. Increased landing distance was calculated and found sufficient, as was no right engine reverser and no alternate brakes for backup. On approach aircraft was configured early for possible slow flaps/slats. Landing, runway clearing, taxi to gate and parking were all normal with the exception that the ramp hadn't notified anyone about the need to have the wheels chocked before the left engine could be shut down due to no parking brake. This was despite having been notified by dispatch, having been called by the copilot in the air prior to landing and again told by the copilot after landing. Finally, ramp had been told to tell ground personnel not to open the cargo doors as there was no Yellow fluid or low fluid that may pump out on the ground. For some reason this also wasn't passed on.

Synopsis

A320 Captain receives ECAM for Yellow Hydraulic System low at FL270 during climb and elects to return. Attempts to inform the ground crew to be prepared to chock the aircraft upon arrival are unsuccessful.

ACN: 902663 (2715 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : CVG.Tower
State Reference : KY
Altitude.MSL.Single Value : 7500

Aircraft : 1

Reference : X
ATC / Advisory.Tower : CVG
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Vectors
Airspace.Class B : CVG

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : CVG
Make Model Name : Skylane 182/RG Turbo Skylane/RG
Crew Size.Number Of Crew : 1
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class B : CVG

Person : 1

Reference : 1
Location Of Person.Facility : CVG.Tower
Reporter Organization : Government
Function.Air Traffic Control : Handoff / Assist
Function.Air Traffic Control : Departure
Function.Air Traffic Control : Coordinator
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Flight Data / Clearance Delivery
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 902663

Person : 2

Reference : 2
Location Of Person.Facility : CVG.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 902674
Human Factors : Distraction
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working all RADAR positions combined to the Tower. The C182 was at 7,000 FT level and passing through my airspace. The A320 checked on my frequency with intentions of landing at CVG. I issued a heading of 310 and altitude of 8,000 FT. I was also performing CIC duties and Dayton FSS had called with an INREQ for an aircraft overdue to an airport in my airspace. I turned around to the TMU station trying to figure out how to work some of their equipment to see if I could locate that aircraft. The A320 asked for lower, I turned around, walked to my scope looked right at the situation and issued the A320 a decent to 3,000. I looked right at the scope and thought the two aircraft had passed. I was back at the TMU station when the Conflict Alert sounded. By the time I looked at the situation again I determined that the aircraft were going to miss but with a loss of separation. We have a piece of equipment here nicknamed the "fish finder" which is what I was trying to find at the TMU station. I have not been trained on the use of this equipment so maybe if I had had that training then I would have been able to quickly help the FSS with their INREQ and been paying closed attention to my traffic.

Synopsis

CVG Controller experienced a loss of separation event when issuing a descent clearance through an occupied altitude, the reporter listing the distraction of assisting with an INREQ as a causal factor.

ACN: 902500 (2716 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 902500
Human Factors : Troubleshooting
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed As Precaution

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

During climb out on SID, deviating for thunderstorms we received an ECAM warning: Eng 1 EPR mode fault. [We] followed ECAM and company procedures. [We] sent ACARS message to Maintenance Control that went unanswered. Continued climb to FL330. Twenty minutes later received additional ACARS messages: Eng 1 reverse unlocked; and Eng 1 hot fuel. [We] continued to ACARS Maintenance Control without answer or confirmation. Finally called Dispatch for link to Maintenance Control. Maintenance Control and Dispatch advised us to return. Poor maintenance [this] aircraft had been written up previously and cleared for flight. Need a way to call/contact Maintenance Control without delay for in flight problems.

Synopsis

A320 Captain reports receiving obscure ECAM warnings for the number one engine but was unable to timely contact Maintenance Control. Once communications were established the crew was instructed to return.

ACN: 902474 (2717 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 179
Experience.Flight Crew.Type : 300
ASRS Report Number.Accession Number : 902474
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Maintenance
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

On arrival in operations, while completing my preflight duties, I noted 4 log history items concerning possibly interrelated flight control issues and elected to call Dispatch and Maintenance Control to gain further insight. The Maintenance Controller stated that he needed to get up to speed (perhaps he had just started his shift?) and that I "should just refuse the aircraft!" I protested that course of action and advised him that I was just seeking clarification of the write ups in the log, because they were vague and I could not determine if the problem had been properly corrected. The Maintenance Controller stated that he did not see any additional information in "his system" and that he was not sure if the problem had been handled correctly by maintenance, so it was better to keep the aircraft on the ground rather than send it out of a maintenance station with a potential unresolved problem. I agreed and asked if another aircraft was available for our flight. He stated he did not know, and at that time, the Dispatcher stated that based on the conversation that he had just heard between myself and Maintenance Control, that he was going to refuse the aircraft and either have maintenance performed or find us another ship. I advised the dispatcher on how to contact me and waited for further information. My first officer suggested that we go visit the Maintenance Controllers to see if they could assist. I briefed the controller on my concerns and what had transpired in my conversation with Dispatch and Maintenance Control. After looking in his computer system, he stated that there were no related deferrals or open items on the aircraft and that "I could NOT refuse the aircraft." At that point I reminded him that I had not refused it , the Dispatcher had, but regardless, I had concerns about the log history and wanted them resolved prior to signing a release for the flight. At that point, he became indignant and told me that maintenance had run a ground test on the system and the aircraft had told them it was OK. I again reminded him of the potentially chronic nature of the issue and asked at what point will good judgment take a precedent over continually returning an aircraft to the line after a "ground checks normal". His remarks were that the "aircraft was smarter than we are", and if it ground tests ok, then as far as he is concerned it's not broken and I should just go fly. At this point, I went to the flight office and briefed the duty manager on what had transpired and we proceeded to conference call with Maintenance Control. Dispatch, Maintenance Control and I all expressed concern over a potential malfunction with the aircraft, but base maintenance continued to protest further troubleshooting, stating the aircraft had flown two legs since the write-up with no gripes so it must be ok now, and it would take two mechanics 30 minutes to perform the testing necessary to rule out any problems. The Maintenance Controller ordered the work performed despite the protests of base maintenance and it was discovered that a Flap/Slat control computer had failed and needed to be replaced. The repair was made and the flight departed approximately 1:30 late. It is obvious to me that the attitudes displayed by some parties to this discussion, display far less than our stated focus of a safety mindset. Furthermore, seeking to intimidate those who put forth a legitimate concern about the safety of our flight operations should never be tolerated.

Synopsis

A320 Captain questioned maintenance actions taken to address flap/slat problems on an aircraft he was assigned and is criticized by the maintenance personnel involved. A defective FSEU (Flap Slat Electronic Unit) was eventually detected and replaced.

ACN: 902472 (2718 of 2908)

Time / Day

Date : 201008
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAS.Airport
State Reference : NV
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 902472
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

Altitude restrictions on the SHEAD 6 SID out of LAS are confusing. Some fixes have two altitudes listed with one of the altitudes having (ATC) marked near it. Please clarify the altitudes themselves or put an explanatory note on the SID page.

Callback: 1

The Reporter stated that when the Tower was queried about the (ATC) restrictions on the SHEAD 6, he was told to climb unrestricted by the (ATC) restrictions but to heed the normal SID altitude restrictions.

Synopsis

A320 pilot reports that on the SHEAD 6 RNAV departure from LAS many of the fixes have two crossing restrictions, one with '(ATC)' next to it.

ACN: 901897 (2719 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : INS / IRS / IRU
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 145
Experience.Flight Crew.Type : 3960
ASRS Report Number.Accession Number : 901897
Human Factors : Other / Unknown
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This report concerns the loss of IR1 and both autopilots. We also had communication difficulties initially with Dispatch and Maintenance. During our climbout, shortly before 18000 feet, I lost all information on my PFD (Primary Flight Display) (attitude, airspeed and altitude). Shortly thereafter, we got an ECAM for an IR1 fault. I looked up and the switch was in the off position! This was then followed by multiple ECAM's for A/THR OFF, THRUST LOCK, YAW DAMP 1 (I think) and GPWS TERR. My First Officer was hand flying at the time and continued to fly. I worked through the ECAM's and switched my PFD to the No. 3 IR and displayed good information. I selected IR1 to ATT. When I reached the status page, I saw that both autopilots (AP) were inoperative (AP 1+2) as well as the yaw damper, GPWS TERR (A/THR?) and Cat 2. Both FMGC's seemed to be working with normal map displays that worked independently. We could select direct to a station but NAV would not engage (we had the dashed line on the map display). [We did a] position cross-check with the VOR's which we could tune with the RAD/NAV page. 

When we realized that neither autopilot would engage, I sent an ACARS to Dispatch to call me. I soon saw ACARS call on the upper ECAM, but could not retrieve the frequency from either box. Since I do not have an FOM in my ship set, I took the aircraft and radios while my First Officer looked up the frequency. We established a phone patch with Dispatch, although at first he could not hear us. We finally established the connection and described what was going on. The symptoms really didn't match anything. With one IR inoperative, we should have had the right autopilot, but we had none. Nor did we have either FD. There were No FMA's on either PFD. There was an FD flag on my side, and the FD flag was intermittent on the First Officer's side. We were hoping that Maintenance would have some ideas for a reset, but he was stumped also. He asked if we were ok to continue to destination; we conferred with Dispatch about any changes in en route and destination weather and decided we were ok to continue. We terminated the phone patch and were about to let ATC know our autopilot situation to see if we could continue at our altitude when the FMA's came back; we selected autopilot 2 and it worked, as did the auto thrust. 

We had no further problems during the flight. Before writing this, I saw that Maintenance deferred the DMC 3 and changed it. How would this have kept the autopilots from working? After reviewing the FM, IR faults, one can get loss of autopilots with one IR failure if the ELAC's (Elevator Aileron Computer) lack confidence in the other two IR's, but if this was the case, I think we should have also degraded to direct law. The crew coordination throughout this event was very good and once we established contact with Dispatch and Maintenance that went well also. In hindsight, probably should have let ATC know what was going on sooner. I knew that when I sent a call me to Dispatch and we didn't get back to him quickly, he would be concerned. I called once we were on the ground and he said was about to send a confirm ops normal. Since we were not able to retrieve ACARS messages for several minutes, we would not have gotten that one either.

Callback: 1

The reporter does not have any idea how the switch came to be off but believes that it must have been on until just before the anomalies began to appear. He is certain that the switch was not touched by either crew member just prior to the event.

Synopsis

A320 Captain experiences loss of PFD information climbing through FL180, followed quickly by ECAMs for IR1 fault and multiple computer failures. IR1 switch is discovered to be in the off position. After switching Captain's PDF to the #3 IR and placing the #1 IR switch in ATT position systems return to normal and flight continues to destination.

ACN: 901550 (2720 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class E : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 400
ASRS Report Number.Accession Number : 901550

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing through 10,000 received ECAM for low Green system hydraulic pressure followed by low quantity. ECAM procedure followed. All checklist completed. Performance data reviewed. Dispatch and Maintenance Control notified. Emergency declared. Cabin advisory completed. Returned to our departure airport. Landing was without incident. Towed back to the gate.

Synopsis

An A320 returned to the departure airport following the loss of Green Hydraulic System pressure and quantity.

ACN: 901512 (2721 of 2908)

Time / Day

Date : 201007

Place

Altitude.MSL.Single Value : 32000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Aeroplane Flight Control
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 901512
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Aircraft

Narrative: 1

While in cruise with the autopilot engaged, we experienced a continuous heavy tail vibration. Our speed at the time was .77 Mach, at an altitude of 32,000 feet. We contacted the Flight Attendant in the aft end of the aircraft to ascertain what the cabin crew was experiencing. The Flight Attendant said that there was a continuous heavy vibration and shaking in the aft galley, extending all the way up to mid-wing area. The First Officer and I discussed and evaluated the situation, and determined the safest course of action was to descend into more dense air and reduce airspeed, suspecting a flight control problem. We contacted ATC, requesting FL 300 and reducing speed to .75 Mach. The vibration continued in frequency and intensity. We continued descent to FL 260, speed 270 kts, at which point the vibration decreased to continuous light vibration from the tail section. We had dispatched with yaw damper #2 inoperative, and we suspected at this point we may be experiencing a problem with yaw damper #1.

In cruise, upon realizing a flight control problem, we used the QRH for any possible assistance. The only info provided there was to fill out the [standardized company] vibration form, found in the FOM. Maintenance Control also requested that I disengage the autopilot and put in control inputs to further assess the area of concern. I denied that request in the best interest of my crew and passengers. Thus far there is no known cause for the vibration. [I am] awaiting word from the Fleet Manager concerning maintenance's findings. Company radio was very unreliable when we needed it most. Due to the seriousness of the situation, typing our problem via ACARS would have been time consuming and would take one pilot out of the loop regarding the safe management of the aircraft.

Maintenance's request that I turn off the autopilot and enter "some control inputs" at cruise altitude was discomforting, to say the least. I refused their request, in the interest of safety for my crew and passengers. Please advise Maintenance Control that we don't "test fly" an aircraft with passengers and crew on board. We were hoping for some further direction on alleviating our problem from the QRH. We found the information provided inadequate and of no assistance. We suggest adding to the QRH suggested airspeeds, Machs, and altitudes that may possibly reduce flight control vibration.

We kept ATC informed of our flight control situation and they provided assistance as required. We informed Dispatch and Maintenance Control of our situation via company radio. Dispatch asked us if we were diverting. We informed them that we would continue to our destination unless the situation deteriorated. ATC, along with Dispatch, coordinated priority handling and we proceeded with no delays. Descent, approach, and landing were normal.

Synopsis

An A320 Captain reported unexplained aircraft vibration believed to have been related to the flight control systems.

ACN: 900876 (2722 of 2908)

Time / Day

Date : 201007

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 900876
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Fatigued. Too long of a day without crew rest is a safety issue. And that it can go to 16 hour duty day if irregular operations is unfathomable. We had a 11 hour 48 minute flight time, 14 hour 11 minute duty stop to refuel holding with passengers all this without any rest. Only one meal and no crew water (we ran out of bottled water).

Synopsis

A Flight Attendant reports excessive duty times and lack of food and water during an irregular operation.

ACN: 900647 (2723 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5700

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 900647

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

First Officer [pilot flying] takeoff [was] normal until gear retraction. [We] heard unusual grinding sound from nose gear area, presumably, followed by multiple ECAM messages to include both LGCIU (Landing Gear Control Interface Unit) failures and BSCU 1 failure followed by numerous related failures. Gear did not initially indicate up and locked. Gear handle was left in the retract position and within 15 seconds indicated up and locked and the LGCIU failure ECAM went away. BSCU 1 fault remained until flight termination. Left the plane in the flaps 2 takeoff position and reported to Tower that we had a problem. We turned west per ATC and then turned north to a downwind for the runway. Contacted Dispatch and communicated our situation to Maintenance Control. We kept them on line and extended the gear which extended normally. We did not declare an emergency but the equipment was rolled for our landing. We requested Tower to observe our landing gear as well as possible with binoculars and they did not see anything unusual so we made a visual approach and uneventful landing. Am unaware at this time what the problem was but our decision to return appears to have been the correct choice.

Synopsis

An A320 returned to the departure airport after grinding noises were heard upon gear retraction, accompanied by ECAM LGCIU and BSCU failure warnings.

ACN: 900355 (2724 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Fuselage
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : APU Pneumatic System & Ducting
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 900355
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Myself and another AMT were assigned the job of replacing the APU on an A320 for cause, using AMM 49-11-98. The APU was removed using the above AMM. The new APU was prepared and placed on a Cargo King to lift the APU into it's compartment. A ladder was also placed on the Cargo King to gain the necessary height needed to install the APU mount nuts.

I was positioned on the left, aft side of the APU compartment to observe the left side and exhaust duct for clearance. I was also the Mechanic operating the Cargo King lift controls. The other AMT was standing on the ladder and positioned on right forward side of the APU compartment. His upper torso was above the APU, so that he could observe the right and forward side of the APU and at the same time align the APU mounts and install the mounting nuts.

We needed another inch of lift to be able to install the nuts. The AMT on the ladder said "up." Neither of us noted any clearance issues. I noticed that the Cargo King would no longer raise. No unusual sound was heard. It was at that time we noted the damage to the APU inlet diverter from the ladder making contact. We lowered the Cargo King a few inches, enough to reposition the ladder. The Cargo King was then again raised to finish installing the APU mount nuts. At that time we notified our Supervisor of the incident.

Synopsis

A Mechanic reports an APU inlet diverter was damaged during their installation of an APU into an A320. He and another Mechanic failed to see the ladder on the lift platform contact the inlet diverter.

ACN: 900354 (2725 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Fuel Booster Pump
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 900354
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 900356
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Equipment / Tooling
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

Mechanic "X" and I where assigned to perform an A-Check Job Card: the removal of fuel pumps 21QA - 25QA -22QA -and 26QA to check for detached labels within the pump and canister. As per the AMM Task 28-21-51, we [were] in the process of positioning the extractor-fuel pump tool hooks in the pump holes, as instructed on Step 15 of the Task, when the pump and slide valve fell out of the plane into my hands. Attempts to replace the pump past the flow of fuel were not successful and approximately 58 gallons of fuel were spilled before the spill response team could stop the spill.

Upon investigation of the pump, slide valve, and pump canister, we found a bent stop pin, as well as the two screws and retaining plate that holds the retaining pin and spring assembly were loose and caused the pin to wobble. With the retaining pin and spring assembly loose, [one cannot]properly retain the slide valve within the canister, while the pump is removed. With the slide valve removed, as well as the pump, fuel is free to exit the aircraft. There were no injuries or aircraft damage due to this situation.

Narrative: 2

While Mechanic "Y" and I were performing Steps 15 and 16 of (Maintenance Manual) MM 28-21-51, the slide valve and fuel pump came out together. This caused a small fuel spill. Found retaining pin spring assembly faulty and stop pin bent.

Synopsis

Two Line Mechanics reported a wing fuel pump change that resulted in a fuel spill when the pump and slide valve fell out of the A320. Examination of the pump found a bent stop pin, retaining pin and spring assembly loose, that prevented the slide valve from being retained by the pump canister, allowing fuel to freely exit the aircraft.

ACN: 900225 (2726 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 12000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9345
Experience.Flight Crew.Last 90 Days : 135
Experience.Flight Crew.Type : 2173
ASRS Report Number.Accession Number : 900225
Human Factors : Physiological - Other

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Condition night VMC about 5 to 10 NM northeast of CEDES Intersection on a heading of roughly 300 degrees, descending from 12,000 to 11,000 FT. The flight crew experienced a green laser illumination sighted at the aircraft for approximately 3 to 4 seconds. The laser broadcast four or five sweeps with two direct sightings in the cockpit. The Captain received direct exposure to the left eye. The event was reported to NCT Approach as a PIREP. Also, the laser illumination was reported to the Company after landing. The source of the laser appeared to be ground based from the vicinity of the northern San Jose or Milpitas area at a distance of 5 SM due north of the San Jose International Airport. After noting the area and street locations, a local road map revealed the source area to be Berryessa Creek Park. The aircraft continued the approach and landing without further incident.

Synopsis

A320 Captain reports his cockpit being targeted by a green laser at 11,000 FT northeast of SJC approaching SFO.

ACN: 900217 (2727 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Airspeed Indicator
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 900217

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 900221

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Evacuated
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

It was my leg to fly. On take off roll, the Captain announced "80 KTS thrust set". I scanned my instruments and noticed that I had no airspeed indication. I announced "I have no air speed". The Captain elected to abort the take off. While decelerating I notified Tower that we were aborting and then notified the Flight Attendants and passengers to remain seated over the PA. We cleared the runway and decided to return to the gate. While we were on taxi back to the gate, we coordinated with ramp control to have the wheels chocked due to the hot brakes. We returned to the gate and waited for ramp personnel to chock the wheels. A few minutes later the ramp personnel notified us that the fuse plugs had blown and the left main tires were flat. Shortly after that, maintenance personnel boarded the aircraft and notified the Purser that the left main gear was smoking, and that she should get the passengers off the aircraft as soon as possible. She initiated an expeditious deplaning of the passengers through the jetway. No further event occurred. After maintenance determined that there was no further fire hazard, passengers were allowed to return to retrieve their belongings.

Synopsis

An A320 flight crew reported the First Officer's airspeed indicator failed on the takeoff roll. The Captain rejected the takeoff, and after returning to the gate the left main gear tire fuse plugs melted and the passengers were evacuated through the jetway.

ACN: 900213 (2728 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ORD

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 22500
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 900213

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
ASRS Report Number.Accession Number : 900224

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On approach First Officer pilot flying. While configuring to land from flaps 1 to flaps 2 received ECAM F/CTL FLAPS LOCKED - Alignment Fault. I maintained altitude and initiated a missed approach. Speed was flown at 180 KTS and we maintained the existing flap configuration of F2. Approach Control vectored us out for an extended downwind while the Captain contacted Dispatch and Maintenance. We completed the Flight Manual procedures for the abnormal configuration landing, including receiving adjusted landing distance information from Dispatch. We switched to the longer runway for landing. The approach and landing were flown at Configuration 2. The Captain, having a vastly superior amount of Airbus experience landed the aircraft. Approach and landing were nominal.

Narrative: 2

I ask the First Officer to maintain altitude and configuration while I informed Tower of our situation. Performed a go-around retracted gear and left flaps in position 2. [We] requested to be vectored away from airport area while we attempted the ECAM procedure. First Officer remained the flying pilot while I ran the ECAM. [We] consulted Dispatch and manual for landing distance info. After completing the ECAM, we switched duties and I became the flying pilot. I asked the First Officer to review what I had done, and to verify we had done everything we could do. Since the flaps were locked we really could do nothing with them. We requested the longest runway for landing, completed all appropriate check lists and landed without any other issues. After landing, [we] taxied to the gate without further incident.

Synopsis

A320 flight crew experienced a flap failure on approach, executed a go-around to run the ECAM and accomplished an uneventful landing.

ACN: 900176 (2729 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 400
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 900176

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[I] aborted takeoff at approx 40 KTS due to an ECAM caution. We cleared the runway and discovered that the yellow hydraulic fluid was well below normal range. Taxied back to the gate, serviced yellow system, checked for leaks, and continued to our destination.

The flight we had just flown in with the same aircraft was a start of day departure. Maintenance was rushing to complete the required scheduled maintenance [for that departure]. A Maintenance Supervisor showed up 10 minutes prior to departure and watched over the mechanics shoulder obviously concerned about the on-time departure. Maintenance had done some work with the yellow hydraulic system and needed to pressurize the system. A LOW AIR PRES ECAM was present. They increased the APU output and got the message to clear. The yellow system appeared to be over-serviced and some of the fluid was drained to get within limits. The blue and green systems were also topped off. We pushed back late and were going to taxi on one engine. The LOW AIR PRES ECAM came back on and we decided to start the right engine to see if we could get the ECAM to go away which it did.

We then took off on our initial flight. I was flying and was a bit concerned with the brake temperature indicator being deferred going into a short runway with a quick turn. When we landed I checked the charts for the required cooling time; which was 30 minutes. I went out to do the walk-around to observe the brakes. They seemed warm but not hot and I was happy with the 30 minutes required to cool the brakes. We briefed the takeoff procedure and were cleared for takeoff and had just gotten takeoff power when we received the ECAM for the low air pres, and that's when I aborted the takeoff.

Synopsis

An A320 flight crew aborted their takeoff and returned to the gate for maintenance due to a LOW AIR PRESS ECAM message.

ACN: 899937 (2730 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class C : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 129
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 899937

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7500
ASRS Report Number.Accession Number : 899947

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Takeoff and initial climb normal. At approximately 1,500 AGL [we experienced] total electrical failure. All screens dark, radios inoperative. Power came back on after about 10 seconds then dropped off again. Power came back on and remained on. We flew a visual pattern and landed [at departure airport]. Power remained on until we arrived at the gate and shut the aircraft down. Tower declared an emergency for [us]. Flight Attendants were advised no cabin prep required.

Synopsis

An A320 flight crew reported total loss of all electrical power shortly after takeoff. They returned to departure airport as ATC declared an emergency for them.

ACN: 899739 (2731 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8800
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2800
ASRS Report Number.Accession Number : 899739
Human Factors : Other / Unknown

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

For an aircraft refusal I was asked repeatedly by the Station Supervisor if I would do an "airstart" in front of the entire crew. After I told him that I needed the deferral cleared, he then made the comment, "so that's it, you're refusing the airplane because you won't do an airstart." Subsequently, as I walked down the jetway stairs the supervisor then asked me, "is there a pilot action going on?" I found this question very shocking and his entire attitude antagonistic. He did not introduce himself until I asked him his identity while walking down the jetway stairs (of which he was following me and then asked the inappropriate job action question). This kind of intense pressure to reverse my decision about the aircraft refusal I found to be a major saftey issue! The boss of this Supervisor should counsel him on the policy of aircraft refusal written in the FOM.

Synopsis

A320 Captain describes pressure from a Supervisor when the Captain refuses an aircraft for lack of an APU.

ACN: 899596 (2732 of 2908)

Time / Day

Date : 201007

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 899596
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Safety Related Duties
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 900003
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We were on tarmac taxiing out. There apparently was a delay in takeoff due to unknown reasons and this was not communicated by the cockpit to the flight attendant's. Because of the delay, safety checks were completed another time to ensure customers were in compliance before takeoff. While safety checks were being completed the pilots had initiated takeoff without making the announcement "flight attendants prepare for takeoff". One flight attendant was not close to their assigned jumpseat and had to take closest available jumpseat for safety.

Narrative: 2

During taxi out we prepared cabin for take off. After an extremely long taxi and wait I felt that it was necessary to prepare cabin again for take off. I was at door 1 left and out of nowhere the engines roared up and we began our take off roll. After being stunned I immediately strapped into jumpseat position #4 and we took off. When it was safe I returned to my position. This could have been avoided if the pilots would have given us a prepare for take off. Now apparently the reason we didn't get it was that ATC had lost our flight plan and we taxied around for a long time. It's my understanding this is what distracted the pilots from giving us the take off warning.

Synopsis

Several flight attendants reported their flight began the takeoff roll following an extensive ground delay with no PA from the flight deck announcing the takeoff was imminent.

ACN: 899483 (2733 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : S56.TRACON
State Reference : UT
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.TRACON : S56
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Climb
Route In Use.SID : ENSIGN RNAV
Airspace.Class E : SLC

Person

Reference : 1
Location Of Person.Facility : S56.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 899483
Human Factors : Other / Unknown

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Air Carrier X was departing north out of SLC on the ENSIGN RNAV Departure. This departure has the aircraft depart at 230 KTS. The speed assignment is two fold: one is for in trail spacing on successive departures and the other is so when the aircraft reaches 10,000 FT they won't push the nose down and try to gain speed. If they do try to gain speed, and slow or stop their rate of climb, then terrain becomes a factor. They need to maintain a positive rate of climb to clear the mountains prior to turning. So, with that said, I observed Air Carrier X's ground speed picking up at about 100. I asked him his speed and he replied 250. I reminded him that his assigned speed was 230 and now to maintain 250 KTS. I did not need him to slow but more importantly I did not want him to speed up and clear the terrain. I then shipped him to the Center. We have had these RNAV departures for three years or so. We are constantly asked by departing aircraft about the climbs and speeds. When we say "Climb and maintain..." Many of the pilots ask if that is an unrestricted climb, or if they have to comply with the crossing restrictions etc. We still spend a lot of time answering these types of questions. The union was not brought in on the design phase of these departures and it shows. These were drawn up and implemented by others not active controllers who work the airspace and know the ins and outs of the sectors. These really need to be fixed. We have pilots climbing out to the wrong altitudes often as well. They need cleaned up and standardized. Safety is the issue. Recommendation, we need to cleanup the RNAV departures at SLC. The departures have too many crossings, instructions, etc. We, the FAA, are just confusing the flying public and have created a safety issue.

Synopsis

S56 Controller expressed concern regarding the new RNAV SID procedures claiming the design was flawed and prompting frequent questions from flight crews as well as speed violations as noted in this report.

ACN: 899376 (2734 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 500

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16628
Experience.Flight Crew.Last 90 Days : 214
Experience.Flight Crew.Type : 6096
ASRS Report Number.Accession Number : 899376
Human Factors : Fatigue
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Approach Controller kept us high at 7,000 feet on downwind leg. Followed by a descend clearance to 4,000 feet and turn for a base leg just past abeam the marker. Our speed was 210K. ATC reported fairly calm winds on the ground but winds were quartering tail winds about 16 knots at the altitude. My First Officer tried to slow down and go down but the tailwinds made the situation worse. He configured the airplane and started doing S turns but approach was fast and high. We crossed the outer marker 1,300 feet high. Sink rate was more than 1,200 feet per minute. Weather at the time was VMC. I can tell the aircraft was approaching the correct flight path; however airspeed was excessive and more than Vref+15k. First Officer started reducing the airspeed after catching the glide slope. I came very close to calling the go around but I didn't since he got the situation under control and was correcting rapidly. The runway was fairly long. Landing was normal. Threshold height was okay. However airspeed was more than the target. I think we didn't meet the stabilized approach criteria. I failed to call go-around. My First Officer displayed very good flying skills and situational awareness on the previous flights. I think that made me complacent. This was also last day of 4 day trip. Every day was very early get ups with long duty days. I think sleep deprivation and fatigue were catching up. I debriefed the approach and the landing with my First Officer. We reviewed the stabilized approach criteria and we decided it is okay to go around.

Synopsis

An A320 Captain reported his First Officer flew a high, fast, unstabilized approach to a landing when they should have gone around. The reporter cited fatigue as a factor.

ACN: 898812 (2735 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 36000

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Route In Use : Vectors
Airspace.Class A : ZAB

Person

Reference : 1
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 898812
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

I was working Sector 46. Not a whole lot going on, although there was a lot of weather along the border from OLS, East and West. I heard MZT call Sector 91 for coordination. Thinking there was arrival traffic coming from there and knowing they were deviating, I expanded out to see who it was. The route from MZT's airspace is always VYLLA.J92.TUS, then points North. Because of this, all flight plans over VYLLA are routed through Sector 46 for coordination through the A-Side into and out of Mexico. I happened to see a limited target (Mode C intruder) West of the DUG VOR at FL360 and said something to Sector 90. Sector's 90,80, and 47 were combined Sector 90 and were extremely busy with weather deviations and hadn't seen the limited data block. MZT was in the process of, presumably, coordinating the deviating traffic, but with the wrong sector. Eventually we figured out it was Aircraft X who called them up after MZT coordinated with 91 to have them shipped to 90. Recommendation, the problem with deviating traffic over VYLLA is we don't do hand offs. It's all Non-RADAR. The flight plan goes through Sector 46, although 91 or 65 will have the appropriate URET information on the flight plan. If the aircraft is deviating East of VYLLA, it puts them in 90 or 80's airspace depending on the stratum. Without any coordination, they can't protect for the aircraft because they have no knowledge of the aircraft. I think the simplest thing to do is to do hand offs between us and MZT. Also, both facilities need highlighted information on altitude stratum's and sectorization. MZT isn't aware of any sectors other than 46, 91, and 65. Most of our controllers aren't aware there are two sectors at MZT along the border. Non-RADAR separation and protection only works along the route. There are obvious issues with deviating traffic, especially in a sector who doesn't know the aircraft is coming.

Synopsis

ZAB Controller described a confused hand off/coordination event between ZAB and MZT Center, suggesting that improved procedures must be develop between the two facilities to accommodate aircraft transitioning the boarder.

ACN: 898665 (2736 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Wing Access Panel
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 898665
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During climb, cabin crew [Flight Attendants] advised that a passenger seated over left wing saw an object depart the wing during takeoff. Flight Attendant advised that there was an opening visible on top of left wing. With appropriate precautions, First Officer went back and determined that a small circular panel was missing [from] top of left inboard wing (approx 3 inches in diameter). Contacted Dispatch/ Maintenance Control and with their concurrence returned to departure airport.

Aircraft had just come out of C-Check. Missing panel was access to gear hydraulic actuator. Don't know why it came loose, but may not have been re-secured during C-Check.

Synopsis

During climb, a Flight Attendant advised Captain that a passenger seated over the left wing saw an object depart the wing during takeoff and an opening was visible on top of the A320 wing. Dispatch and Maintenance Control were contacted; aircraft returned to departure airport.

ACN: 898653 (2737 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 898653
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Maintenance Action
Result.Flight Crew : Took Evasive Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We arrived at gate as assigned but no guide man was available. We shut down #2 engine while we waited. The guide man came out and marshaled us in. I cleared on the left and verbalized it and my First Officer cleared on the right stating everything was behind the line. Just prior to stopping in the park position my First Officer yelled out for me to stop and I did aggressively. He stated he thought we may have made contact with a belt loader. We then remained in position and the jetway was connected as we were almost on the parking stop spot. We shut down, completed checklists, and went down to the ramp. The #2 engine had brushed a belt loader and the bottom outside right nacelle was scratched. A Ramp Supervisor told us the guide man should not have brought us in without a wing walker; also that the belt loader was improperly parked even though behind the red line. The aircraft was perfectly centered on the parking line. The Ramp Supervisor stated engineering was on the way as there was no significant damage other than some scratches.

Synopsis

An A320 engine cowling struck a belt loader as the aircraft was being guided into the assigned gate. The loader was improperly parked and no wing walkers were available to assist.

ACN: 898507 (2738 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Maintenance : Technician
ASRS Report Number.Accession Number : 898507

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

I have been keeping track of this aircraft. The aft galley oven R41 Circuit Breaker trips repeatedly. Originally made a deferral in June 2010 for galley oven R41 popping. It was signed off. Accomplishing deeper research I found in the past year that this oven has had 45 hits where the R41 oven has popped its circuit breaker. The oven was just replaced again in July for R41 Circuit Breaker popping. It is not the oven.

Synopsis

Maintenance Technician reports that an A320 he was assigned to repair has had 45 prior instances where the R41 aft galley oven circuit breaker trips for no apparent reason.

ACN: 898506 (2739 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component : 1

Aircraft Component : Circuit Breaker / Fuse / Thermocouple
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Galley Furnishing
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 898506
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

I have been keeping track of an A320. The aft galley oven R41 Circuit Breaker (C/B) pops. Originally made a deferred [item] in June 2010 for galley oven R41 popping. It was signed-off. Accomplishing deeper research I found in the past year that this oven has 45 hits where the R41 oven has popped its C/B. The oven was just replaced again July 2010 for R41 C/B popping. It is not the oven.

Synopsis

A Line Mechanic reports he has been tracking a company A320 that has a chronic history of the aft galley oven R41 Circuit Breaker popping. Ovens have been replaced and deferred items signed-off. In the past year, the same R41 C/B has had 45 write-ups for popping. Mechanic notes the oven is not the problem.

ACN: 898432 (2740 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 898432
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

On the visual to 28R, we were told that a Brasilia was turning left base to final 28L. We reported him in sight and were cleared to continue the visual. Approach again pointed him out at our 8 o'clock as we were on final and ahead of him. ATC told us, "if he passes you, don't pass him". I've never heard of such a clearance. I thought he was going to be staggered slightly behind us, but no, the Brasilia caught us and then remained even with us till landing. I think this pilot group has been very understanding and has tried to work with ATC on a lot of issues here in SFO, but it has gotten to the point of being ridiculous. These side by side approaches are unacceptable. From now on, I will be liberal with my own speed adjustments from 8 miles on in and will do what I need to do to get spacing acceptable to me from the guy on the parallel.

Synopsis

Air carrier landing SFO during parallel approaches to Runway 28L/R questioned ATC's procedures with regard to passing and visual separation, and considered the operation unacceptable.

ACN: 897812 (2741 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 897812
Human Factors : Other / Unknown
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 897813
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : Weight And Balance
Detector.Person : Maintenance
When Detected.Other 
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I had the privilege of flying our troops in July. The flight was uneventful. I was made aware by the Chief Pilot two days later that they had discovered 40 bags in bin 5. This bin is normally not used and was only supposed to have contained the Flyaway Kit. During our pre-flight preparations we got the figures on the weights for that day. We ran a preliminary calculation with cargo weights distributed by a 3rd into the main cargo bins. Specifically bin 1, 3, and 4 and got a good weight and balance. Toward the end of our prep we got the load form from the Agent. The OP-10 did not indicate anything other than the flyaway kit in bin 5 as was expected, however when we ran the ops program it showed us slightly out of Aft CG limits, as they had placed more bags in the back. The quickest resolution was to move a few passengers forward in order to bring the CG within limits. I was surprised to find out that I had failed to note the move on the load form as is required. There was weather moving in the area and in haste I either forgot or I wrote it on the yellow copy, as the Agent had gone to fax the paperwork. The take off and flight were normal and we did not experience anything abnormal during all phases of flight. I have a habit of including the trim wheel in my scan and it was in the green arc during all phases of flight. This would include the return flight which the First Officer flew and he also did not note anything out of the ordinary.

Upon returning the Mechanic went to retrieve the Flyaway kit and discovered the extra bags bin 5 is normally not used combined with the fact that we went to an airport that the ground crews may have been unfamiliar with the A320. I don't know how the bags got into bin 5 I can only guess that access to bin 5 through bin 4 may have confused the ground crew into thinking that was part of bin 4. For my part I have learned that particularly on these charter flights where ground crew may be unfamiliar with the A320, more attention has to be paid by designating a crew member to observe and or direct first hand the loading of cargo. I will be much more diligent about notating all changes on the load form.

Narrative: 2

Aircraft was loaded incorrectly by placing 40 bags in bin 5 and not noting this in the weight and balance form. The only item in bin 5 should have been the fly away kit (145 LBS) which was computed into the operations program for weight and balance. After receiving the weights and computing the weight and balance the CG for the ZFW was slightly aft. This was remedied by moving a few passengers forward. Also note that in the cockpit was all crew bags properly secured due to requests by the military to segregate the crew bags from the soldiers carry ons (m16s). The aircraft flew normally on both flights.

Synopsis

A320 flight crew is informed after the fact that 40 bags were loaded in bin 5 and not noted on the weight and balance form. The bags were in an unusual location and not offloaded at destination, remaining on board for the ferry flight back to base. No adverse flying qualities were noted.

ACN: 897799 (2742 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5500

Environment

Flight Conditions : IMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Landing Gear Indicating System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 897799

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 897672

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After rotation and positive climb, I commanded gear up. Captain was pilot monitoring and stated the gear handle would not move. We then received a landing gear strut failure ECAM, and lost use of the flight directors, autopilots and auto thrust. I continued to fly raw data while Captain reviewed/ran ECAM actions and QRH for the failure and upcoming overweight landing. After completing, ran remaining checklists and was vectored in for an ILS approach, but had visual approach conditions 10+ miles from field. [We] touched down at 146,500 LBS with a 20 ft/min descent rate. Upon returning to gate, mechanic reviewed aircraft printout of a ground proximity failure.

Narrative: 2

...landing gear handle jammed in down position, ECAM display [read] "L/G SHOCK ABSORBER FAULT".

Synopsis

An A320 returned to the departure airport when the landing gear failed to retract due to a landing gear strut failure.

ACN: 897798 (2743 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.MSL.Single Value : 2500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : MCO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : MCO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 897798
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Unstabilized Approach
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

ATC was late giving us lower, then hold 210 KTS. We were very close-in using speed brake on descent, set up for 17L. ATC gave us a heading and advised we would get 18R. We re-set up everything for 18R (we had briefed this, but did not expect it this close in) cleared direct to ORL VOR at 3,000 FT. We called the airport in sight to slow and go to 2,500 FT (our last assigned speed was 210). ATC said he would clear us for a visual in a bit. I slowed to 190 anticipating the clearance soon. We were just north of ORL VOR when he cleared us for the visual to 18R, maintain 2,500 until south of the VOR. I set 2,500 in and started slowing, gear down, slow down, flaps 3 landing check. At this point we were about 400 FT high, so auto pilot off, flight directors off, flaps full. We were stable at 500 FT and landed.

ATC late runway change, we just did the same approach two days earlier and requested the right side but did not get it (our book says don't request it, they know) so my mind set was we would get 17L. I should have slowed down and told ATC I was slowing instead of holding the assigned speed of 210 KTS. I should have not accepted the runway change. Accepting the late change would have worked out had ATC cleared me for the visual early as I could have slowed and descended to 2,500 and got the gear down early. It worked out, however induced a very high work load and stress, and a lot of heads down to reprogram the new runway/approach.

ATC should make up their minds which runway they are going to use at least 30 miles out, and if the other comes up at least ask if you want it not [just] assign it to you. Clear you for the visual or at least give you a normal speed not assign 210 and forget they did. I should have told them I was slowing and just slowed. It seems to me they keep you at 12,000 FT too long when landing to the south.

Synopsis

A320 Captain experiences a late runway change and a late clearance for a visual after previously being assigned 210 knots. An unstabilized approach fortunately results in a normal landing.

ACN: 897394 (2744 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Climb
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 897394

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Anomaly.Deviation - Altitude : Undershoot
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : Became Reoriented
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

PDC printout appeared to read maintain 3,000 FT when it should have shown 9,000 FT. On contact with Departure we stated that we were climbing to 3,000 FT and Departure did not question it. After being level at 3,000 FT for approx one to two minutes, Departure asked if we were cleared to 9,000. We told him we were cleared to 3,000. He asked if that was assigned by Tower. We told him "No" and stated that it was from our PDC. He then cleared us to 9,000. After reviewing ACARS we saw that it was 9,000 and printed out another copy which clearly showed 9,000. We let Departure know that we found where the error was and asked if there was any conflict. He stated that there was no problem and sent us off to the next frequency. There is an "At or Above 4,000" constraint departure that we passed at 3,000 due to this error. Poor printing on the PDC printout making cleared altitude to appear to be 3,000 instead of 9,000. Second copy printed better.

Synopsis

An A320 First Officer reported he and his Captain misread a poorly printed PDC and leveled off at a lower than cleared altitude on departure.

ACN: 897384 (2745 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Distance.Nautical Miles : 15
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 897384
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Human-Machine Interface
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Flight [was] uneventful up to top of descent. A Flight Attendant advised forward lavatory overflowing onto carpet in First Class and back into middle coach. We wrote it up and advised maintenance via ACARS. Later, on 15 mile final, an ECAM for F/O AOA vane was received. On taxi in an ECAM for AIR PACK Regulator was received and, while waiting for a gate, an ECAM for FAC1 appeared. These ECAMs cleared, but eventually came back to some degree during the next 10 minutes. Several more (I forget which ones) ECAMs were received while we were briefing maintenance at the gate. I think the lavatory overflow may have leaked into the E & E bay during descent and taxi, causing multiple, unrelated ECAMs to be displayed. I feel very fortunate to have been on the ground, and not had to deal with the majority of these events while airborne. After thinking the situation through, this seems to be an anomaly that may require landing ASAP if a forward lavatory overflows.

Synopsis

An A320 Captain reported multiple seemingly unrelated ECAM messages were displayed on approach and after landing following a report of an overflowing forward lavatory shortly before beginning the descent.

ACN: 897075 (2746 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : NCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 7000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : SFO

Component

Aircraft Component : Navigation Database
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 897075
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Altitude : Crossing Restriction Not Met
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Chart Or Publication

Narrative: 1

FMS database shows ARCHII at 7,000. Cleared to descend on the FMS bridge visual. Too many FDC Notams for SFO with irrelevant info. Missed info on Notams. Chart was overlooked.

Synopsis

An A320 flight crew failed to make a crossing restriction on arrival to SFO. Reporter felt that excessive numbers of NOTAMS contributed to their workload and confusion.

ACN: 896793 (2747 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 9000

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 22R
Flight Phase : Final Approach
Route In Use.STAR : WYNDE 2

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1600
ASRS Report Number.Accession Number : 896793
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

After checking in with Approach Control, we were told to plan for the ILS 22R approach. While still on the WYNDE 2 arrival inside of FIYER we were given a clearance that read "cross FNUCH at 7,000 cleared ILS 22R maintain 170 KTS to RIDGE." While this clearance in and of itself is fine, at no time did Approach Control ever give us instruction or clearance on how to transition from the arrival to approach phase. This led to some confusion and I then asked if we should maintain our present heading to join the localizer 22R. The Controller agreed and recleared us for the approach. I am beginning to see a trend in this type of clearance which lacks some crucial part. Chicago is a very complex environment in which mistakes can be made. The controllers need to be clear and concise and not give 3-4 clearances in one transmission that can lead to confusion or omission on the part of the flight crew.

Synopsis

An A320 flight crew received an approach clearance to ORD while still on the WYNDE 2 RNAV STAR. The clearance provided no method of "transition" from the arrival to the IAP.

ACN: 896637 (2748 of 2908)

Time / Day

Date : 201007
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component : 1

Aircraft Component : AC Generator/Alternator
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : APU
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 22000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 896637

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 1100
ASRS Report Number.Accession Number : 896715

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On departure climbing through FL250 generator number two faulted. Attempted reset per ECAM - no effect. Start APU and put APU generator on line. Coordinated with dispatch via ACARS on aircraft status/configuration. Approximately 120 west of [departure airport] the APU auto shut down. Set up radio patch with Dispatch and Maintenance. Attempted reset of generator control unit number two per Maintenance and Capt emergency authority - no help. Coordinated return to [departure airport]. Landed uneventfully. No traffic priority was required.

Synopsis

An A320 flight crew reported the loss of a generator followed later by an auto shut down of the APU. After consulting with Dispatch and Maintenance, and with only one AC power source, they returned to departure airport.

ACN: 896575 (2749 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZZ.ARTCC
State Reference : FO
Altitude.MSL.Single Value : 36000

Environment

Weather Elements / Visibility : Thunderstorm
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Climb
Route In Use : Direct

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 11500
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 896575
Human Factors : Training / Qualification
Human Factors : Troubleshooting
Human Factors : Human-Machine Interface
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

First Officer, pilot not flying, on A320. Flight plan had initial altitude of FL280, but Center cleared us directly to FL360. I thought the altitude was unusually high for our high gross weight, but the (new) FMS display a REC MAX of FL363, so we accepted the clearance and maintained OPEN CLIMB to FL360. The instant we entered "ALT CAP" at FL360, the managed speed index moved from approx .78M to approx .72M (Green Dot/VLS). When I saw this, I selected the PROG PAGE and noticed that the REC MAX altitude changed from FL363 to 334. Since the Captain was new on the airplane and was unfamiliar with the idiosyncrasies and consequences of low mach at the upper limit of altitude capability, I manually selected a speed of .77M and suggested we request a lower altitude. We requested a lower altitude from Center and received a clearance down to FL340, at which time the REC MAX display changed from FL334 to FL343. We captured FL340 and the PERF page at that new altitude showed a new optimum speed of .77M, whereupon we re-selected managed speed. 

It appears that, in the new FMS, the performance algorithm for calculating REC MAX altitude in the climb is different than that used for cruise. Thus, the displayed REC MAX altitude changed from FL363 to FL334 the instant we changed from OPEN CLIMB to ALT CAP. This may be an item for the fleet technical group to look at. On our flight, the displayed REC MAX altitude was unrealistically high during the climb and could have resulted in a high-altitude unacceptably low-speed condition had we not intervened in overriding the automatic features of the FMS/AutoPilot.

Synopsis

An A320 FMS computed a higher recommended maximum cruise altitude during OPEN CLIMB than was computed when the FMS transitioned to ALTITUDE CAPTURE at cruise. The crew descended down to the REC MAX computed for cruise.

ACN: 896328 (2750 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 896328

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During descent while slowing to 250 KIAS above 10,000 feet, ECAM alert alerted "L WINDOW HEAT". Shortly [thereafter] the First Officer heard a slight "crack" and noticed that the fixed side window on the Captain's side was cracked in a spider web pattern. There were no other indications. Descent was continued to a normal landing. Dispatch and Maintenance were advised. A Left Window Heat system fault may have contributed to thermal shock, causing the window to fracture.

Synopsis

An A321 Captain reported his cockpit window cracked shortly after receiving a "L WINDOW HEAT" ECAM. A normal landing followed.