ACN: 895990 (2751 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Person

Reference : 1
Location Of Person : Hangar / Base
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 895990
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Other
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

During aircraft corrosion visit, it was discovered that on prior repair incorrect (non-structural) fasteners were used and in some instances fasteners were missing in floor support structure, which is primary structure. Write-ups were generated.

Callback: 1

Reporter stated the galley floorboards were removed for access to the cabin fuselage floor primary structures at the right aft galley door (2R) area. The A320 was in for a corrosion check visit. Inspection found previous corrosion removal and repair work had been poorly done by a contract maintenance provider on the cabin floor frames and door threshold sill plate area. The lower portion of the cabin galley door pressure seal rests against (on) the floor sill plate when the door is closed in order for the cabin to pressurize. 

Reporter stated inspection also found the Sill plate that attaches to the outer fuselage skin and inner primary frame floor supports in the galley area, had missing fasteners and sub-standard (non-standard) fasteners installed, instead of the required structural fasteners. The threshold sill plate is normally installed with Hi-Loks and rivets to the fuselage skin and primary frame supports. The sub-standard fasteners that were installed to attach the Sill plate to the aircraft were 5/32" of an inch or smaller, using non-structural screws and low profile nuts that were not tightened.

Reporter stated his company told him they do not know who, or where, the door sill plate was previously installed at.

Synopsis

A Mechanic reports about an A320 that was in for a corrosion check where inspection found the threshold sill plate had been improperly installed at the aft right galley door (2R) floor area. The Sill plate had missing fasteners and sub-standard (non-standard) screws and low profile nuts that were not tightened.

ACN: 895930 (2752 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZLA.ARTCC
State Reference : CA
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZLA
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZLA

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 400
ASRS Report Number.Accession Number : 895930
Human Factors : Distraction
Human Factors : Human-Machine Interface

Events

Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation
Result.Air Traffic Control : Issued New Clearance
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

In cruise ATC told us to overfly DOBNE Intersection. During the call with ATC the Flight Attendant's called with a maintenance writeup that was distracting, especially the overly loud chime that the Airbus has. We were told to overfly DOBNE and given a new arrival into SFO. While I was inputting new route the First Officer noticed the aircraft had started an early turn and we were not overflying DOBNE. The First Officer used heading mode to turn towards the waypoint so that we would overfly it. ATC then called and gave us a heading to overfly the waypoint which we did. A few minutes later ATC said we were now clear of Joshua Airspace and we were cleared direct. I believe we did fly over the waypoint with the First Officers intervention. The Airbus does have a tendency to cut corners over waypoints. In retrospect, I should have just used the overfly button at DOBNE to avoid any confusion. It would have been helpful if ATC would have told us of Joshua's Airspace and the need for an overfly rather than the usual route the FMGC would follow.

Synopsis

A320 Captain reports being chimed by the Flight Attendant's at the same time a routing change was being issued by ATC.

ACN: 895775 (2753 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 25000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 895775

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out at approximately FL250 we got a blue hydraulic system overheat ECAM. I was the pilot not flying, so I complied with the ECAM and followed up with the book. The system cooled and I turned the blue pump back on as per the ECAM and Flight Manual. After that was complete, I called Dispatch and got a patch with Maintenance. The reception was bad, but I was able to convey the issue. Maintenance told me if the ECAM returned I could reset the pump again if it cooled. On descent, the ECAM returned. Again I complied with the ECAM, but this time the First Officer and I felt it would be better to not turn the blue pump back on. We calculated the landing distance as per the manual and landed without incident.

Synopsis

An A320 Captain reported the blue hydraulic system overheated in climb. Book procedure was followed, pump turned back on when system cooled, but overheat later returned on descent.

ACN: 895769 (2754 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 4700
ASRS Report Number.Accession Number : 895769
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 90
Experience.Flight Crew.Type : 4700
ASRS Report Number.Accession Number : 895772

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

A few seconds after takeoff, with flaps still extended, we got a "Vent Skin Fault" ECAM. (Approximately 1,000 FT AGL) I was flying and I directed the First Officer to do the ECAM while I flew and took over the radios. The ECAM was completed. Then the First Officer got the book out and looked at the additional items in the checklist. That is when he noticed that the ECAM now said "Vent Blower Fault". He ran that checklist as well as looking at the additional items for the "Vent Skin Fault". The "Vent Blower Fault" requires us to limit flight time to 2 hours. The First Officer wrote to Maintenance via the ACARS. We were waiting for a response. I asked the First Officer to take over the flying and the radios and then wrote an ACARS to Dispatch. I informed Dispatch of the situation, and asked them to get in touch with Maintenance and discuss the situation. I received a frequency from Dispatch and got on the radio with both Dispatch and Maintenance. After discussing the situation with Maintenance and Dispatch it was decided that we would divert. I also mentioned to Maintenance that we would be doing an overweight landing. I then called and spoke to the flight attendants and then after a few minutes made an announcement to the passengers. In the meantime the First Officer was changing our destination with ATC. I ran the overweight landing checklist, and looked at the performance and runways available lengths etc and decided to start the APU once we were below 15,000 FT. (Starting the APU is required on the overweight landing checklist). We requested the longest runway. We did NOT declare an emergency but I was willing to do so if ATC was not willing to give us the runway or if any additional problems were to have occurred? After completing the checklists, briefing etc I took over flying the airplane again and the First Officer took over the radios. The APU bleed was opened on downwind. I put the flaps out early and the gear down early to help burn off some fuel. We landed uneventfully and touched down with a vertical speed of about 100 FPM and a gross weight of 148,900 LBS gross weight.

Synopsis

An A320's "VENT SKIN FAULT" ECAM alert at 1,000 FT after takeoff. Later the "VENT BLOWER FAULT" ECAM alerted requiring a flight no longer than two hours. The Captain diverted to an enroute airport.

ACN: 895755 (2755 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 14000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 8800
ASRS Report Number.Accession Number : 895755

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[We had a] normal preflight and boarding. Previous gripe, engine 1 high pressure (HP) bleed valve fault was fixed overnight so no MEL items on release. On taxi out we got an ECAM for the engine 1 HP bleed valve fault. Stayed on ramp, contacted Maintenance but could not clear it. As I had added fuel and the weather was VFR, we agreed to a deferral and did not have to return to the gate. Upon receiving the new release and complying with its restrictions we got a new flight plan for FL300 with the MEL applied. Climbing through 14,000 FT we got engine 2 bleed fault ECAM. Two resets were unsuccessful and as engine 1 bleed was closed per the deferral, I declared an emergency to return to departure airport. Using emergency authority I opened engine 1 bleed as the low pressure valve was working fine, this controlled the cabin pressure. My First Officer was the pilot flying and he handled ATC and flew the aircraft while I told the flight attendants we would be back on the ground in 7 minutes. Message was sent to Dispatch via ACARS followed by a PA to the passengers. All ECAMS were done and no overweight checklist to contend with. Started the APU and resumed ATC communications. Oddly enough, ATC was vectoring us for sequence. Confirmed we were an emergency aircraft and requested vectors for landing airport in sight. They seemed perturbed by this, not sure why, but oh well. On clearing the runway I advised Tower that we would taxi to the gate and no further assistance was needed from the emergency vehicles. Taxi to the gate and parking was uneventful. Upon shutdown and checklists complete, made a PA to passengers with more info as to why the return.

Synopsis

An A320 Captain reported losing #2 engine bleed after departing with #1 engine bleed deferred. They declared an emergency and returned to departure airport. Captain reported that for some reason ATC was vectoring them for sequence even though they were an emergency aircraft.

ACN: 895451 (2756 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cargo Compartment Fire/Overheat Warning
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 215
Experience.Flight Crew.Type : 6200
ASRS Report Number.Accession Number : 895451

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Declared Emergency
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After shutting down both engines, I observed an ECAM for Smoke Aft Cargo. I alerted Captain to ECAM as we were just beginning shut down flows. Captain directed me to complete QRC actions and informed me he had witnessed a "puff" of smoke on his side of aircraft. Ground crew had not established communications yet. Along with ECAM, there was an overhead warning light illuminated indicated smoke in aft cargo area. I completed QRC actions including discharge of one fire bottle. After a short period of time, the overhead red warning light for indication of smoke/fire extinguished. Captain was able to establish communication with aft Flight Attendants who also reported smelling an odor similar to smoke. Normal disembarkation by passengers continued as there were no other indications of fire or a problem from either ground personnel or aft cabin Flight Attendants. I contacted ramp personnel to request Fire Department response as a precaution. I was told to "Stand By". I then contacted Ground who directed me to contact Tower who switched me to a discreet tower frequency. Fire Department responded very quickly and did a thorough inspection of the aft cargo area before coming up to cockpit to report to us that there appeared to be nothing abnormal in the cargo area. Eventually, several other [company] individuals arrived to be debriefed on the situation. I thought the Captain and I handled the situation calmly, professionally, and in a timely manner. I never anticipated this type of ground emergency happening after the engines were shut down and the passengers had started their deplaning. I think a ground evacuation would have resulted in injuries, so allowing the normal deplaning to [continue] was a good decision.

Synopsis

An A320 First Officer reported noting a "Smoke Aft Cargo" ECAM after gate arrival. The crew ran the QRC, shot the fire bottle, and decided that normal passenger disembarkation was safe to continue.

ACN: 895229 (2757 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 14000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Airspace.Class E : ZAU

Aircraft : 2

Reference : X
ATC / Advisory.Center : ZAU
Make Model Name : Cessna 425/441 Conquest I/Conquest II
Crew Size.Number Of Crew : 1
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class E : ZAU

Person

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 895229
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Cessna 425 was descending to 11,000 with a good rate down and I stopped the aircraft at 13,000. I wanted to get the A320 under the Cessna 425 because the A320 was overtaking the Cessna 425 by a large difference in speeds. I descended the A320 to 14,000 and called traffic to each airplane. Cessna 425 was the first plane to call the other in sight, so I asked the aircraft if he could maintain visual separation with the A320 when the A320 descends. The Cessna 425 stated he was able to maintain visual separation with the A320, so I cleared the A320 to cross FISSK waypoint at 11,000 feet. I advised the A320 aircraft that Cessna 425 had him in sight and would maintain visual separation from him. Although I thought I gave the Cessna 425 an instruction to "maintain visual separation from the A320," upon review of the tapes afterward I forgot to give that instruction. I was trying to abide by my LOA crossings with SBN ATCT and thought I gave that instruction, but apparently never did. On the same token, it appears the rules written for visual separation were written to avoid us using this tool. Perhaps we can take another look at why we need to give so many transmissions to an aircraft to maintain visual separation. In a busy sector like LOOTH, it takes away a valuable tool to move air traffic, and it actually creates more workload for me. Either I leave A320 high for MDW and have to APPREQ the altitude with SBN as well as point out to BEARZ, or I vector the Cessna 425 well out of the way thereby providing a disservice to the pilot.

Synopsis

ZAU Controller experienced loss of separation event involving two descending aircraft when failing to comply with the required verbiage during the application of visual separation.

ACN: 895186 (2758 of 2908)

Time / Day

Date : 201006

Place

Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Cabin Lighting : Low
Number Of Seats.Number : 150
Crew Size Flight Attendant.Number Of Crew : 3

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Cabin Activity : Service
Cabin Activity : Safety Related Duties
Cabin Activity : Deplaning
Cabin Activity : Boarding
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Attendant : Current
Experience.Flight Attendant.Total : 13
Experience.Flight Attendant.Airline Total : 13
Experience.Flight Attendant.Number Of Acft Qualified On : 10
Experience.Flight Attendant.Type : 100
ASRS Report Number.Accession Number : 895186
Human Factors : Fatigue
Analyst Callback : Attempted

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Attendant
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

I am writing to address a very serious concern regarding crew member fatigue, which I feel demands your full and immediate attention. As a Flight Attendant for over 13 years, it is no secret that ours is a demanding job of long days and short nights. However, never since I started my airline career in 1997, have I seen perfectly healthy flight attendants suddenly collapse while on the job. I personally have been affected by 2 incidents of collapsing Flight Attendants within the last four months the most recent being one of the reasons behind the delay we took on this trip, while awaiting the late arrival of our pilot crew, whose previous flight attendant crew had one that collapsed while on the jumpseat. I am including in this letter a copy of the itinerary of my last trip, which I found to be very taxing on my health and performance. Never as a 13 year veteran have I ever worked 4 legs on a redeye trip with a 5 plus hours of a layover in the middle of it, then to continue on working another leg. I speak for the entire crew when I claim that none of us were alert enough to have been able to efficiently respond to an aircraft emergency as our job requires, under such condition. I bring this to your attention and ask that you review it and let me know if this is in fact what is expected on me as flight attendant. This is the summary of a 5 day trip where both flight attendants came back extremely drained and ill, where no crew meals were provided, and where the schedule made it so a decent meal was inaccessible. Please note that I am a West Coast based flight attendant, and ask that you make an allowance for the time zone changes. This schedule also fails to note the 1-hour report time that the crew is obligated to meet, prior to departure, nor does it reflect the time it takes to deplane, to wait for, and take the hotel shuttle, check in for our rooms, to finally wind down.

Day 1: depart ~0200, en route 4+30, layover 10+30, depart ~1930, en route 4+50, depart ~2300 4+00 en route layover 12+30. 
Day 2: depart ~2030 en route 1+30 depart ~2245 en route 1+00 layover 5+30.
Day 3: depart ~0545 en route 1+00 layover 12+15 depart 2030 en route 5+00, layover 18+30
Day 4: depart ~1815 en route 1+45 depart 2145 mechanical return to gate 1+00 depart 0100 en route +45 depart ~0230, en route 3+30, layover 12+00. 
Day 5: depart ~2230 en route 4+30 END!

Granted, that this trip was on my monthly schedule, thus permitting a generous number of days off after its completion to recuperate from fatigue and the cough and cold that I've picked up from a weakened immune system. However, my reserve schedule in July 2010 could very well award me something similar, which would then only give me 12 hours to recuperate or worse, assign me an international flight on the 13th hour of being back in my base. Even the different pilot crews we have encountered while on this trip couldn't fathom our long days. I fully understand the demands of my job, and try to do my best at it each day. However, I find that my performance, judgment, and alertness are severely impaired by the level of fatigue in my system, no less than alcohol would cause. Much as we comply with FAA alcohol consumption regulations in order to ensure our best performance on the job, it does no one any good when we come to work under such fatigued conditions.

Synopsis

Flight Attendant describes a very fatiguing five day trip.

ACN: 895152 (2759 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 895152
Human Factors : Situational Awareness
Human Factors : Confusion

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Detector.Person : Maintenance
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was assigned an aircraft. In the work pack Job Card XXX001 was assigned to be accomplished. I do not recall if I printed the job card or it was previously printed. After arriving at the gate, I assisted in opening #2 engine for a bore scope inspection, completed 2 other work release items and started an overnight check. I accomplished Job Card XXX004. This was the job card that was in the work pack. I signed the card off on the work release according to the job card number. I was notified two days later of completing the wrong job card. Due to the error, an overfly did occur for Job Card XXX001. Management was notified on and the aircraft was stopped in ZZZ and the Job Card XXX001 was accomplished.

Paperwork auditor found the error. At the beginning of the shift several job cards are printed. The wrong job card may have been placed in the wrong work pack. Awareness of the correct job card sign off.

Synopsis

Maintenance Technician was informed after the fact that the job card he signed after completing the assigned work did not match the work pack number (004 instead of 001).

ACN: 895042 (2760 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : HPN.Tower
State Reference : NY
Altitude.MSL.Single Value : 1500

Aircraft : 1

Reference : X
ATC / Advisory.Tower : HPN
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : HPN
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet 700 ER/LR (CRJ700)
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Facility : HPN.Tower
Reporter Organization : Government
Function.Air Traffic Control : Coordinator
Function.Air Traffic Control : Flight Data / Clearance Delivery
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 895042
Human Factors : Confusion

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Runway
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Staffing
Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

I kept the Tower open with all the positions combined. Several itinerant aircraft landed, but they did not have any parking at the main terminal. I staged these aircraft at several locations on the field. In addition, I was clearing aircraft to reposition for overnight parking, clearing GA aircraft to their FBOs and issuing IFR clearances to aircraft looking to depart HPN. I was coordinating with Westchester County Operations about the Tower visibility, gate availability, and other aspects of the operation and coordinating with N90/LGA regarding PIREPS and Tower visibility. Rapidly decreasing visibility conditions, down to 1/4 SM, occurred. At the time of the event, Aircraft Z was holding short of Taxiway Foxtrot on Taxiway Lima. Aircraft X was holding short of Runway 11 on Taxiway Lima. Aircraft Y was on a 7 mile final and cleared to land when Aircraft X was on the opposite side of the field from the main terminal, cleared to turn right onto Runway 11, cross Runway 16, then turn left onto Taxiway Alpha, and taxi to the main terminal. Aircraft X was advised of the rapidly decreasing Tower visibility and instructed to advise clear of Runway 16. Aircraft Y's landing clearance was cancelled. Aircraft Y was told that they would be advised when Runway 6 was clear. I waited until Aircraft Y was on a 3 mile final but had not received verification of Aircraft X's position. At that moment, Aircraft X asked me where they were supposed to clear the runway. I asked Aircraft X to confirm their current position, and they replied "on Runway 16." Aircraft Y was on a 2 mile final. I immediately cancelled Aircraft Y's approach clearance and issued "Go Around" procedures. Aircraft X advised they were clearing Runway 16 at Taxiway Bravo. Recommendations, hold at least two CPCs on overtime and pay more attention to pilot read backs for their accuracy.

Synopsis

HPN Controller described a go around event when a taxiing aircraft failed to report clear of the landing runway, the situation complicated by very low surface visibility.

ACN: 894882 (2761 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Ground : LAX
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 5400
ASRS Report Number.Accession Number : 894882
Human Factors : Confusion
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 186
Experience.Flight Crew.Type : 600
ASRS Report Number.Accession Number : 894885
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Ground Incursion : Taxiway
Detector.Person : Flight Crew
When Detected : Taxi
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

First Officer was pilot flying and briefed for visual approach backed up by the ILS approach to Runway 25L at LAX. Briefing included expected runway exit plan to taxi to gate from a landing on 25L. On approximately 20 mile final, our clearance got changed to Runway 24R. After landing, the Captain assumed control to taxi to the gate. We were cleared to cross 24L, and we were cleared by ground to taxi via E and R and to hold short of C and contact south ground. I had not taxied via this routing from the north side since LAX had re-configured the taxiways, and was expecting the old "south route." The Captain and I both referenced our airport diagram, but nevertheless we turned right on Taxiway E-13 which is just west of R. We saw the Taxiway R lighted signs ahead of us, and I thought that we were turning at the correct spot until we finished the turn and realized we were on E-13. Darkness and unfamiliarity with that routing led to the error. We stopped, and Ground Control coordinated with LAX Airport Operations who cleared us to make a 180 turn on E-13, so that we could exit that area and resume our taxi to the gate.

Narrative: 2

Late runway change from 25L to 24R due to another aircraft with emergency needing priority. Changed FMC and quickie brief on approach and exit plan. Tower directed to clear on forward high speed (denied reverse high speed) and then cleared us across 24L and to ground control frequency. Ground Control cleared us to taxi east on E and south on R, hold short of C. Neither First Officer or I had ever heard of R taxiway and referenced our airport diagram, which we both had displayed and available. Copilot called "next right" and I agreed. Upon taking the turn, we realized I had turned on E-13 and into a maintenance ramp. We asked Ground Control if we could do a 180 and rejoin E, he said we needed to wait for city operations to clear area. We did, he did, and we executed a 180 and continued with taxi in. 

Lessons: 1. Construction at LAX has significantly altered landscape of LAX taxiways. Neither First Officer or I had been on the north side of LAX in months. I had not flown in 21 days before this trip. 2. Quickie brief needs to include taxi in plan, if possible. 3. When we both had to reference the airport diagram, I should have stopped the aircraft and made sure where we were and where we were going. Local familiarity only goes so far with so much construction. 4. Strong co-pilot adds to complacency, just a strong student can make the instructor pilot complacent. 5. Stop, ask if questions if not sure. Same old lesson again.

Synopsis

A320 crew familiar at LAX discovered recent construction has changed various taxiways resulting in a taxiway incursion.

ACN: 894742 (2762 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 31000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 894742

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise flight, with no warning, a very loud "crash" sounded behind me to my left. Upon turning around, I and the First Officer observed the rear window completely shattered. I and he immediately went on O2 and rechecked sear-belt integrity. I subsequently got an immediate decent to FL190 at which time we complied with QRH procedures and lowered delta-P to 4.8 PSI. The inner pane of glass was found to be completely intact. An anti-ice ECAM had appeared after the fact which we complied with as well. Seat belt sign was immediately put on and I made a PA to have Flight Attendant's cease service and buckle-up. Once established at FL190, I allowed Flight Attendant's to unbuckle but not serve. Seat belt sign was left on to landing as a precaution. We were later given an altitude restriction which we were going to be very tight on as we started down late. We were given a heading and a new altitude with a change of Centers. [There] appeared to be no conflicts. Attention to the window and current operations led to the late departure from FL190 to 12,000 FT. We probably would have made the restriction, but I informed Center as a precaution. [There was an] anti-ice malfunction on that window.

Synopsis

A320 flight crew experienced left aft cockpit window cracking at FL310. After complying with QRH procedures they continue to their destination.

ACN: 894731 (2763 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Compressor Blade
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 3800
ASRS Report Number.Accession Number : 894731
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 4900
ASRS Report Number.Accession Number : 894740
Human Factors : Situational Awareness
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Normal start, taxi, no issues noted prior to takeoff roll. Planning flaps 2 reduced power takeoff, 67 degrees assumed temperature. 1.29 predicted EPR power. Once cleared for takeoff, the First Officer advanced power and waited for engines to stabilize at low power. Throttles advanced with all indications normal. Initially [aircraft] required right rudder which quickly became full deflection, plane not accelerating normally. The First Officer retarded throttles at approximately 40 KTS and called abort. First Officer initially slowed the aircraft and positively passed control to Captain. Aircraft taxied clear of runway.

Narrative: 2

During normal takeoff from runway Engine 1 experienced an ECAM ENG 1 COMPRESSOR VANE FAULT at about 30 to 40 KTS. [We] aborted on runway at 30 to 40 KTS; talked with maintenance after clearing the runway. Maintenance had us return to gate. Engine indications at idle and at lower power settings were normal. No unusual noise or compressor stalls could be heard. Engine 1 would not accelerate past 40% N1 during takeoff.

Synopsis

An A320 crew reported rejecting a takeoff after an engine would not accelerated above 40% N1 and the ECAM annunciated ECAM ENG 1 COMPRESSOR VANE FAULT at about 30 to 40 KTS.

ACN: 894704 (2764 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : BOS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi
Route In Use : None

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : BOS
Make Model Name : Cessna Stationair/Turbo Stationair 6
Crew Size.Number Of Crew : 1
Flight Plan : IFR
Flight Phase : Climb
Route In Use : None

Person

Reference : 1
Location Of Person.Facility : BOS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 894704
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

I was monitoring a developmental on Local West. Aircraft Y, a C206 was cleared for takeoff on Runway 4L. Air Carrier Z was holding short of the Runway 4L approach awaiting further instruction to Runway 9. Air Carrier X was holding short of Runway 4L on Runway 33R. Air Carrier W was holding short of Runway 4L on Taxiway Echo awaiting instructions to Runway 9. The developmental instructed Air Carrier Z to taxi to Runway 9 via Bravo and Mike. The pilot acknowledged and at the same time another aircraft acknowledged. Aircraft Y was on departure roll. Due to 2 other aircraft holding short of Runway 4L with similar call signs, the trainee attempted to talk to Air Carrier W due to the closeness in call signs and the ASDE-X went off due to Air Carrier X crossing Runway 4L down field. The C206 was airborne and turning northbound at the time of the alert. The developmental never used the words "Cross Runway 4L", during his transmission to either aircraft, only taxi was used. Recommendation, company call signs may need to be addressed so that we don't have similar flight call signs in and out at same time. As controllers we are required to complete computer based training on hear back read back so that we don't hear what we want instead of what is actually said. A refresher on the same lesson might be helpful for the pilots as well. We often get use to a sequence of transmissions. In this case, I believe Air Carrier X heard what he expected to hear because the taxi instructions for his outbound company aircraft (Air Carrier Z) did not correlate to where he was on the airfield. The word cross was not used in the transmissions involving both pilots. The only words used were taxi to.

Synopsis

BOS Local controller providing OJT described a ground conflict event when an air carrier entered a runway with traffic departing, similar flight numbers were listed as a casual factor.

ACN: 894324 (2765 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Main Gear Wheel
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 894324
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

A broken tie bolt was found on an A320 #3 main tire. Tire and wheel assembly were replaced, and airplane departed without incident. My concern is that in the past (approximately) one month, I have seen this five times. (Three Airbus and two B757s). One of the B757 tire's had four broken [tie] bolts in a row. Before this string of events, I have not seen this on any airplane in (approximately) two years. [This is] a 'part failure.'

Synopsis

A Line Mechanic reports finding a broken tie-bolt on an A320 #3 main tire wheel-half prior to departure. Mechanic notes that in the past month he has seen sheared tie-bolts on three Airbus and two B757's, including one B757 with four bolts in a row. He has not seen that many broken bolts in two years.

ACN: 894175 (2766 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 8500
ASRS Report Number.Accession Number : 894175
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Ground Personnel
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This report concerns parking and marshalling issues. We taxied in from the east and pulled parallel to an aircraft parking simultaneously at the adjacent gate. While getting established on the lead-in line, we watched a catering and fuel truck in the red area. The Marshaller accepted the aircraft and marshalled us forward as the catering truck pulled away, and while the fuel truck was still not clear of the area. He stopped us as the fuel truck pulled in front of us. After the fuel truck cleared, he held up his wands and did not move them. Then, he would move them for a few seconds and stop. He did this a few times, at which point, I flashed the lights at him to clarify his instructions. At the time, he was looking away. I repeated this later. There were 3-4 attempts at non-standard taxi instructions, at which point we called Ramp to get another Marshaller. Another Marshaller showed up, eventually, and through a combination of both non-standard and standard taxi instructions, was able to marshall the aircraft successfully. I taxied the aircraft based on the intent of the Marshaller, and with visual clearance, after a short discussion with the First Officer. We figured this was the best we were going to do for Marshaller's; if non-standard training was conducted in a non-standard manner, the bad training was passed on to most Marshaller's. After parking the aircraft, an unidentified employee, assumed but not known to be the Ramp Supervisor, inquired as to the problem; after a short, terse discussion which included pointing out the safety issues, he walked away saying he would review the video. If this is in fact a Ramp Supervisor, one could speculate that this appears to be a systemic disregard for the process of marshalling, and the role and position of the pilot in that regard. Employee contact of a safety or constructive nature should not be done in a disrespectful or judging manner, but to seek information to improve the safety and efficiency of the operation. The exchange was unprofessional, and did nothing to support the operation or respect fellow employees. A poorly trained Marshaller who gave instructions to taxi toward a fuel truck, and an unprofessional, unknown, Supervisor looking employee compromised safe practices today, and greatly impacted future cross-employee participation during safety debriefs.

Synopsis

A320 Captain laments the lack of safety awareness and training that the Marshalls are receiving at his Air Carrier.

ACN: 894173 (2767 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 894173
Human Factors : Fatigue
Human Factors : Other / Unknown

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

While on the final day of a 3-day trip, I took myself off the trip due to fatigue. This was a demanding 3-day trip with 20 hours of flight time, multiple legs and 2 late-night layovers. While flying the first leg of day 3, both the Captain and I remarked several times that we were very tired. Following our arrival, we were scheduled for a turn to finish the trip. While briefing in Operations, Dispatch informed us that we would be changing planes for the return flight and departing late (approximately 45 minutes). While walking to the gate, Dispatch called the Captain with an updated plan which would put us back in base after midnight, nearly 3 hours late. When the Captain informed me of this, I became extremely angry. We discussed our fatigue and the possibility of taking ourselves off of the turn. The Captain decided to continue and told me he supported whatever decision I made regarding the trip. He departed for the plane and told me to do what I needed to do. I pulled out my cell phone and began to make a call to scheduling, but instead of completing the call, I hurled my phone against the wall. Since there was no activity at the gate, I don't think anyone witnessed my anger, but as I was picking up the pieces of my shattered phone, I realized I was in no condition to fly. I returned to Operations, called scheduling to inform them that I was fatigued and would not be flying the turn. This incident was not the result of a one-time late-night flight, but the cumulative effects of several months of fatigue, stress and frustration. I realize that most every pilot is dealing with these problems. The past 6 months for me have been particularly difficult due to a series of never-ending colds, EXTREME CHRONIC FATIGUE, and a rash of late-night arrivals, deferred maintenance, trip changes, musical Captains, merger rumors and other stress at work. When [I am] at home, I must assume parental duties first thing in the morning to accommodate my wife's work schedule. (You can't support a family in this area on A320 First Officer pay.) Because of this, I am often unable to recover from one trip before beginning another. This situation has been exacerbated recently because I have flown several trips which have arrived back at home base 3 or 4 hours later than scheduled. I am in a state of EXTREME FATIGUE all the time, and as a result, I have become unreasonably irritable. I have reacted to minor problems and inconveniences with anger in some cases and apathy in others.

Synopsis

A320 First Officer describes a demanding trip and home life that results in a fatigue call to crew scheduling.

ACN: 894160 (2768 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.MSL.Single Value : 4000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : C90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 10
Flight Phase : Final Approach
Airspace.Class B : ORD

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 894160
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Human-Machine Interface
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

Prior to this flight we returned to the gate to have Maintenance check a bang the Flight Attendant's heard. During approach when cleared to intercept Runway 10 LOC, the LOC would not arm. I switched runways and reentered 10, but it still would not arm. I tried arming the approach and that didn't work either. The mechanic who came to address the issue said it was a known problem and occurred because of the return to blocks. This could have been extremely dangerous if this occurred in IMC and fuel or mechanical problems were also being dealt with. Why doesn't my Air Carrier: 1. train us properly on the new FMGEC's (Flight Management Guidance Envelope Computer) and 2. not inform the pilots of KNOWN problems that we may encounter?

Callback: 1

The reporter believes that the problems that he has encountered can be traced to the new FMGEC's that the company is installing in their Airbus fleet. The upgrade is primarily concerned with increased memory capability but also has some quirks that the company is aware of but has not conveyed to the Crew's. This is the second instance that the reporter has encountered where the flight guidance did not perform as expected during an ILS approach.

Synopsis

A320 Captain reports problems with the new FMGEC's that are being retrofitted into the fleet. On two separate occasions the FMGEC would not give proper flight director guidance to fly ILS approaches.

ACN: 894046 (2769 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Direct
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 894046
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Approximately an hour out of our destination, the cabin reported a 'hot' odor in the back of the cabin. We shut down the TV system and my First Officer went back to investigate. He noted that the TV box at the suspect row was warmer than the rest. The odor faded quickly. So we continued. Approximately an hour and a half later the cabin reported the smell had returned. I exercised my emergency authority for a precautionary diversion to a nearby airport. I did not declare an emergency however. The approach and landing where hurried but uneventful. Maintenance inspected the aircraft and could not find any problems. As a precaution the TV system was place on placard. I suspect that the smell was associated with a personnel electronic device of some sort. It was also thought that we may have been over weight on landing. That was entered in the book as well. The aircraft was signed off and we continued to our scheduled without further drama.

Synopsis

An A320 Captain exercised his emergency authority and diverted to an enroute airport after flight attendants reported a hot odor in the aft cabin but could not identify a specific source.

ACN: 893991 (2770 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL

Aircraft

Reference : X
ATC / Advisory.TRACON : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Flight Phase : Initial Approach

Component

Aircraft Component : Autothrottle/Speed Control
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 893991
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 894190
Human Factors : Human-Machine Interface
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Regained Aircraft Control
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were inbound and cleared for approach to 27L at ORD with AP and AT engaged on the ILS. ATC had assigned a speed of 250 KTS since we were still a good distance out. ATC offered us sidestep to Runway 28. We said yes and they assigned us a heading of 10 degrees left to join the approach for 28. While on the new heading, the new approach was entered into the MCDU. At this point, the Auto Thrust (AT) advanced so the plane started to accelerate. The pilot flying quickly reset the the FCU speed select window to a speed lower than 250 (actually 180). The AT did not pull back the engines. The pilot flying then clicked off all automation and hand flew. By this point the plane had accelerated to about 275 KTS. Plane slowed with commendable pilot technique, Continued in for normal landing on Runway 28. Event written up in logbook. We spent a lot of time deconstructing the event during a post flight debrief. Also chatted with the mechanic and 2 other pilots. Various ideas put forth, but neither of us can specifically identify why the automation behaved this way.

Narrative: 2

Cleared to intercept 27L LOC about 30 miles out 10000 FT with assigned speed 300 KTS. Captured LOC 27L and cleared to descend to 8000. Selected speed 250 then descended to 8000. ATC switched us to Runway 28 with an initial 10 degree heading left/south, cleared to intercept LOC. Pulled heading and switched approach to Runway 28 in the MCDU. Armed approach to intercept 28 LOC. Intercepted 28 LOC. Cleared visual 28 and pull VS to descend into glideslope for 28. Thrust remained more than 50%. ATC assigned 180 speed. Selected speed 180 and deployed speed brakes. FMA had LOC green and MCDU was in approach mode. Pushed managed speed to get the thrust to come back to reduce speed. Thrust instead increased. Pulled speed button and reduced it to 180 but thrust didn't come back. With speed increasing to over 270 KTS, manually pulled throttles back to about 40% and pushed AT disconnect button. Thrust decreased and pulled back more and then to idle. Shut off FD and continued with a visual approach and normal landing to Runway 28.

Synopsis

A320 flight crew experienced Auto Thrust malfunction during approach to Runway 28 at ORD. Auto Thrust added thrust while the flying pilot was attempting to slow. Control was regained by disconnecting Auto Thrust and retarding the thrust levers.

ACN: 893582 (2771 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US
Altitude.MSL.Single Value : 7000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 8000
ASRS Report Number.Accession Number : 893582

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Returned To Departure Airport
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Aircraft had number 1 bleed deferred for eng 1 HP valve fault...on climbout passing 7,000 FT number 2 bleed faulted and ECAM message displayed...reset faulted number 2 bleed and malfunction cleared...later the fault returned and temp on pre-cooler displayed amber with a temp of 265 C...with no viable bleed source I coordinated a level off with ATC at 13,000 with a return to our departure airport...once clear of the mountains coordinated a descent to 9,000 FT...cabin altitude never exceeded 4,500...uneventful landing and taxi to gate.

Synopsis

Dispatched with one pack inoperative due to a failed high pressure bleed valve, an A320 flight crew returned to the departure airport when the other pack bleed sources malfunctioned.

ACN: 893570 (2772 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 30000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 893570
Analyst Callback : Attempted

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 893735
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : FLC Overrode Automation

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

We were at FL370 (approx), autopilot was on, weather was clear, visibility unlimited, smooth air. Suddenly, for no apparent reason, the aircraft banked approx 25-30 degrees right, then went about 20 degrees left. The autopilot did not disengage, but I disengaged it manually and maintained level flight with aileron input only. After both pilots confirmed that all flight computers were functioning normally, I re-engaged the autopilot. All flight phases were uneventful after I re-engaged the autopilot. Everything was functioning normally. It was at this time that the First Officer (pilot not flying) noticed a spiraling shaped vortex at our flight level. To me, it looked like wing tip vortices. It was there momentarily, and, after a brief discussion, we determined the vortex to possibly be some wake turbulence (like one would see on final approach or while observing landing aircraft) from another aircraft. I then made an announcement to the passengers about our circumstance, and then I talked with the Chief Purser on their experiences in the passenger compartment. Though everyone was startled, all Flight Attendants were OK as well as all of the passengers. There were no reports of any injuries, though I was told one woman in the back was crying. As turbulence is a "part of the job" and because no passenger or Flight Attendant was injured, both pilots decided that a report was unnecessary for this incident. In retrospect, I believe I should have informed ATC and asked if there were any large/heavy aircraft near our plane that may have caused our temporary problem with their wake, but we did not elect to do this. Finally, I made it a point to go out in the cabin during deplaning to address any passenger concerns or questions. I received no negative comments about our flight, only praise.

Narrative: 2

At cruise we encountered an uncommanded roll to the left and then a roll to the right of approximately 30 degrees of bank. It was the Captain's leg so he grabbed the side-stick and disconnected the autopilot. The airplane corrected back to level attitude. At this time I noticed the remnants of a contrail just above our altitude. It was parallel to our flight path. We did have traffic pass over us in opposite direction minutes before the incident. I also noted a direct tailwind of 50 KTS. The roll was abrupt enough to cause a coffee pot to fall of a serving cart in the back. No injuries were reported. We figured it must have been wake turbulence but could not be certain.

Synopsis

An A320 Flight Crew reported a probable wake turbulence encounter at FL370 that resulted in significant uncommanded roll in both directions. The crew reported what appeared to be spiral-shaped wing tip votices at their flight level.

ACN: 893533 (2773 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ORD.Tower
State Reference : IL
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use.Other 
Airspace.Class B : ORD

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : ORD
Make Model Name : Any Unknown or Unlisted Aircraft Manufacturer
Flight Phase : Takeoff / Launch
Airspace.Class B : ORD

Person

Reference : 1
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 893533
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

Weather went down rapidly, an air carrier went around on short final, I have no airspace so I said I was going around and asked for a heading. South Local said to go to 270, I immediately did that and then South Local said something about a 28 departure, I replied you're not rolling are you? South Local said yes, I then found the aircraft off the departure end directly underneath my go around and I told my aircraft to climb to 5000 as fast as he could. I'm not sure how close they came but it sure was unnerving. Recommendation, if your going to roll 28 IFR and land 22R they need to be gaped.

Synopsis

ORD North Local Controller described a conflict event when an arrival to Runway 22R executed a weather prompted go-around and conflicted with south local traffic departing Runway 28, the reporter suggested gaping procedures are necessary for this runway configuration.

ACN: 892774 (2774 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 892774
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

On arrival we parked at gate and noticed on ACARs gate information that we were to take this same aircraft out in the morning. The next morning the Gate Agent and I discussed how to get as many passengers on as possible. Gate Agent handed me release with letter stapled to the top of it. Letter discussed APU usage and ground air at the airport and how to avoid unwarranted APU usage. I then proceeded to aircraft. Flight Attendant said cabin was hot. I check cabin temp and they were at 88 degrees. I elected to start the APU for passenger comfort. First Officer performed walk around and returned to the flight deck. He stated that Gate Agent asked him why we were using the APU and handed him same letter I received. First Officer stated that on arrival to aircraft it was hot in cabin. Further Ramp Agent stated to First Officer he should go to ramp to discuss hooking up ground air, however my First Officer had other duties to attend to. Gate agent was anxious to depart early. Flight Attendant's were also ready and asked if we could close. We closed and departed. En route at approximately two hours airborne I looked for logbook and could not find it. I remember seeing it when we deplaned. I also thought I saw it when I first walked on aircraft in the morning for our flight. I sent an ACARS message to my Dispatcher and confirmed logbook was at our departure airport. He stated a new logbook would be at our arrival airport for our flight.

Effort by station to depart early in addition the issue of APU usage by the station agent to both myself and First Officer caused distractions. In addition this four day trip had other factors. On day one we had an engine issue that required a diversion and landing single engine. On one leg our first Flight Attendant was written up for serving pre departure drinks to first class and the gate said she held up boarding. Additionally we left an hour late which caused a shorter layover with a very early wake up body clock time. On the before start check where it has 'logbook---c---onboard' if it also included ---v--- for verify by the First Officer this would add another barrier to prevent leaving without the log book.

Synopsis

A320 Captain departed without the aircraft logbook on board. He detailed multiple distractions as the cause.

ACN: 892573 (2775 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cockpit Door
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 892573
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 2200
ASRS Report Number.Accession Number : 892582
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Ambiguous

Narrative: 1

Upon arrival at plane, we were confronted with the cockpit door aural warning not working and the door access panel being disabled. The Flight Attendant stated that she was not comfortable flying in the plane with no way to get into the cockpit if we were incapacitated. The system was deferred. Upon discussion between the Captain and myself I do not believe it is a safe idea to defer such a problem. No one can get in the cockpit if both pilots are incapacitated and in the event of a loss of control on landing/taking off, no one can get into the cockpit either from the inside or outside of the plane if we are unable to get out ourselves. The A320/319 does not have on exterior handle by the First Officer's window to get into the plane. The door will unlock if no power is on the plane, but if the engines are still running after the incident the door cannot be opened and also noone can get in the cockpit to shut down the engines if we are dead/incapacitated. I don't believe a plane would be dispatched if both escape lines were missing (one way of getting out of the plane) and I don't think it is a good idea to dispatch a plane with the above condition.

Narrative: 2

[We] refused to fly the aircraft with a deferral on the door entry system. The oral warning was inoperative so maintenance deferred it and deactivated the outside entry pad to the cockpit door. If we had flown the aircraft we would now put ourselves in a situation so that if we were incapacitated either in the air or on the ground that no one would be able to enter the cockpit from the outside. Also, there is no way to get into the cockpit from the outside on the aircraft as with Boeing [aircraft] I have flown in the past. I don't believe that this should be a deferrable item on the Airbus. The MEL should be changed to reflect this fact.

Synopsis

An A320 Flight Crew reports refused an aircraft due to the deferral of the cockpit door entry system.

ACN: 892412 (2776 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 892412
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Time Pressure
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Procedure

Narrative: 1

On approach to SFO Runway 28L we were expecting to land with 5500lbs of fuel. On initial approach ATC gave us a "box" pattern vector which put us at 4400lbs of fuel. We asked to not delay us any more due to fuel constraints and they replied that no more delays were anticipated. We began the approach with 3300lbs of fuel. On a ten mile final we were given go around instructions due to the aircraft in front of us going too slow. We began an immediate turn to SJC, but they told us to keep the turn going and would get us in immediately after we declared minimum fuel. We turned around and landed on Runway 28L. As we were in the turn back to the Runway the low fuel ECAM came on. This event did occur, as with most incidents, because of a multiple chain of events. The winds were higher than planned, and then we had to contend with weather that was heavy to extreme precipitation. We went around it to the south which took more time and fuel. As a crew we decided to continue and talked about our plans along each step of the way. The more we got into the SFO approach area; we told the controllers that we could not accept any delays, never saying MIN FUEL. They told us not to worry and would not have any issues. As we got vectored around we discussed that we would only attempt one approach and then on to SJC. We declared minimum fuel after the aircraft was under control as we were pulled off the approach. They gave us a 270 back to final. At that time the low fuel light came on and land ASAP annunciated. We discussed quickly that we would land this time since we did not have any other options left. After being able to sit back and replay our situation in our minds the Captain and I came to an agreement. In the future we would not hesitate to declare an emergency to get us to airport and on the ground. We were keeping up with our fuel burn, winds, and had asked Dispatch to run the new numbers for us with the route change etc. We asked Approach if we would have any issues on final and that we could not take any delays. I have made a set of guidelines for me in the future... For example: less than forty five minutes of fuel remaining must declare "min fuel" and less than thirty minutes you must declare an emergency as per our Operating Manual. We were aware of our situation along the entire route of the flight and the information we were given led us to believe we would not have any fuel issues, but as it played out and changes popped up at us, we dealt with it as a team.

Synopsis

An A320 First Officer reported that after arriving in the SFO area after an excessive transcontinental fuel burn they were given delaying vectors and a go around. As they entered the final approach the LOW FUEL LAND ASAP ECAM annunciated. They did not declare an emergency.

ACN: 892381 (2777 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 892381
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Time Pressure

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 892597
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Time Pressure

Events

Anomaly.Conflict : Ground Conflict, Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : Physical Injury / Incapacitation
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

[On] taxi out a catering truck was traveling at a high rate of speed. [I] did not think he would stop creating impending crash. [I] Yelled stop. [The] Captain immediately applied brakes. At the same time the truck saw us and applied his brakes. It backed up so we could clear. [We] reported truck number to Ramp Control [and] called in back to find out if anyone [was] hurt. A Flight Attendant said she fell and hurt her back but felt she was OK to continue.

Narrative: 2

I was looking out my left side window, watching an aircraft coming off the gate. The First Officer commanded stop in a serious voice. I stopped the aircraft firmly. A Flight Attendant told me during the stop she fell backwards onto one of the aircraft's bulk heads and may have bruised her back but felt fine to work the trip back.

Synopsis

An A320 Crew taxiing at ORD reported having to abruptly stop in order to avoid a high speed catering truck which also had to take evasive action. A Flight Attendant hurt her back during the stop.

ACN: 892379 (2778 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.CTAF : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 892379
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Taxi
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

I am writing about two concerns. How and who flies maintenance ferry flights and continued concern about a published system fault reset procedure outlined in the Flight Manual that cannot be accomplished due to the permanent disabling of a switch by heavy safety wire. I was assigned a "reposition" flight. This turned out to be a maintenance ferry flight being conducted under FAR Part 91. The flight was for corrosion in excess of limits in the galley floor structure. The flight was to arrive after the Tower closed. Upon taxi-in the airplane annunciated several faults, including APU Bleed Fault, Hydraulic Reservoir Overheat, Avionics Cooling Fault and uncommanded Evacuation Horn. The Flight Operations Manual (FOM) stipulates line pilots may fly maintenance ferry flights provided it can be operated within the Flight Manual, MEL, and CDL. It then stipulates that a special flight permit (Part 91) flight will be issued for flights that 'MAY' not meet MEL/CDL requirements. I was assured by the Operations Manager and Dispatch that I was authorized to conduct this flight, however I believe these two statements are in contradiction to each other... and that line pilots should not be conducting Special Flight Permit flights. These flights are operated under FAR Part 91, and whereas our FOM provides guidance for no-tower operations for charter flights, it provides no guidance for ferry flights into non-tower airports. In particular, this flight was for a corrosion issue that appeared non-critical, but upon arrival the airplane exhibited several faults that may have been related to the reason for the ferry flight (corrosion due to spills in galley). Had these faults occurred in flight it would have been a much more serious problem. On arrival, unlike a charter flight with a charter kit -- we were provided very limited information as to where to park the aircraft or how to get to our hotel. The ground crew did not provide a "follow me" vehicle as we were told via ACARS. The crew used non-standard hand signals with no wands. They approached the aircraft without communicating to us and placed a stairway to the aircraft and then did not open the door. As we were parking, several faults began appearing, including uncommanded EVAC horn. Per a previous note the procedure cannot be performed as the switch is permanently hard wired in the Capt/Purser position. I continue to feel strongly this tool should be available to the Captain in the event of a fault to silence the horn to allow the CIDS fault reset to be performed in a quieter and less stressful environment. I believe in particular this flight should have been an Engineering crew, and all Part 91 maintenance flights in general should be non line pilots. Absent that, a "charter" kit should be provided with all the applicable information.

Synopsis

An A320 Captain reported conducting a maintenance ferry flight to a non Tower airport late at night. After landing he found MEL/CDL discrepancies which may have required an engineering crew to fly the flight according to the Company's policy.

ACN: 892237 (2779 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 892237

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 650
ASRS Report Number.Accession Number : 892246

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

In cruise, we received a HYD G RSVR LOV LVL ECAM message. Declared an emergency. Completed ECAM and Aircraft Operating Manual procedures. Had a cabin prep accomplished, then diverted and landed below max landing weight limit at ZZZ. Stopped aircraft on runway and was towed to the gate.

Synopsis

An A320 Flight Crew responded to the loss of Green System hydraulic fluid.

ACN: 891983 (2780 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Fan Reverser
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Other Documentation
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 22
ASRS Report Number.Accession Number : 891983
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Ground Strike - Aircraft
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

An A320 aircraft landed with left landing gear up, and thrust reverser was severely damaged. After investigation thrust reverser (T/R) was scrapped. Swing shift Lead in Repair Shop instructed mechanics to remove parts from scrapped unit; mechanics also washed-off red paint indicating unit was scrapped. Parts are sitting in the shop without any tag indicating serviceability of parts.

Callback: 1

Reporter stated that although the damaged A320 Fan Reverser had "SCRAPPED" painted all over both reverser halves, the unit was not quarantined in a secure area as required. Numerous parts, including reverser extend/retract hydraulic lines, actuators on the reverser sections, reverser blocker doors and lower closing latches are in high demand for repair of other A320/319 reverser assemblies.

Reporter stated their Shop Lead Mechanic directed other mechanics in their Component Repair Shop to strip parts from the "scrapped" reverser and use acetone to remove the "SCRAPPED" red lettering from both reverser halves. 

Reporter stated he told the mechanics in the shop that the parts they were removing were no longer considered serviceable parts and had already been removed from their stores parts inventory control system. He also told his Lead that what he was directing the mechanics in the shop to do was illegal. The Lead told him to go see the Supervisor. The Supervisor told him there was nothing he could do and he (Reporter) should file a report to their safety department which would force him (Supervisor) to answer the report. The Supervisor did not put a stop to the parts robbing from the "scrapped" reverser.

Reporter stated shortly after he filed a company report he was brought into a room and harassed by two other Supervisors who harshly questioned him about his role and another Mechanic's role in alerting the shop mechanics about the improper maintenance. 

Reporter stated the whole experience was very negative and left him very concerned about his Management's attitude towards those who bring forward any safety issue. Since then, the reverser halves were moved to a more secure area.

Synopsis

A Shop Mechanic attempted to stop the robbing of parts from damaged A320 reverser halves that were painted "SCRAPPED" in red lettering. The reverser halves were not quarantined, the Lead would not stop and the Supervisor said there was nothing he could do. Mechanic was later harshly questioned by two supervisors about his role in alerting shop mechanics about the illegal practice.

ACN: 891978 (2781 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Manufacturer : A
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 891978
Human Factors : Confusion
Human Factors : Distraction
Human Factors : Situational Awareness
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Primary Problem : Human Factors

Narrative: 1

Inadvertently looked at procedures for MEL 36-11-07-a when MEL 36-11-07-b is what I was clearing condition for. "A" does not lock HP (High Pressure) valve in "Closed" position. Condition "B" does lock HP valve "Closed," which led to me allowing an Airbus A320 to fly with HP valve locked out (Closed).

Contract Maintenance noticed valve in closed position. I was the last one to do Maintenance on system I believe this was the aircraft's first flight since maintenance. I did not look close enough at MEL. Contract Maintenance unlocked (opened) valve. My Supervisor informed me of my mistake and issued me a performance report. My suggestions [would be] to take my time.

Synopsis

A Mechanic reports he inadvertently referenced the Maintenance procedures for MEL 36-11-07a when MEL 36-11-07b was the correct MEL. An A320 departed with the HP (High Pressure) valve in the locked out ("Closed") position.

ACN: 891918 (2782 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Pneumatic System - Indicating and Warning
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 891918
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While flight planning the First Officer and I reviewed the aircraft maintenance history. We observed two recent write-ups involving AIR WING BLEED LEAK warning. Maintenance action both times stated the warning was reset, and the warning went away. The First Officer and I were both concerned that troubleshooting was not attempted for either an actual bleed air leak, or a faulty warning system for a wing bleed air leak. Bleed air leaks in the wing can be a serious defect. I telephoned Dispatch to submit a Captain's refusal for the aircraft. Dispatch arranged a phone consultation between me and a Maintenance Representative, to further explain to me that there was currently no defect or deferral on the aircraft, so no maintenance was warranted. I stated our concerns about the unexplained air wing bleed leak indications. A Flight Operations Representative telephoned me to inquire as to my concerns and ask what support he might be able to provide. I requested either a leak check on the aircraft, or to assign a different aircraft for the flight. The Flight Operations Representative telephoned me again after gathering more information, explaining that no further maintenance would be performed, and asked me to reconsider my decision to refuse the aircraft. I declined. The Assistant Chief Pilot then called my First Officer to gather information. The Assistant Chief Pilot then called me. He explained to me that an off-line Mechanic reset the fault, and ran the #2 engine at idle, and no warning occurred. The Assistant Chief Pilot attempted to persuade me to fly the aircraft. I respectfully declined. Over the course of a couple of hours, the flight was canceled, and then reinstated with a different aircraft, and the flight was completed successfully. In reviewing this incident, I feel that all company procedures were followed regarding the aircraft refusal policy in our FOM, communication was good, CRM applied, and a safe decision was made.

I do however have three concerns: 
1) A degradation of aircraft maintenance occurring at our airline.
2) Pressure was applied to me by Flight Operations to fly an aircraft that I felt was not safe. 
3) Priority is given to getting flights out rather than accomplishing proper maintenance.

Synopsis

A320 Captain reports refusing an aircraft for repeated AIR WING BLEED LEAK warnings. Corrective action by Maintenance was to reset the warning system which was not acceptablee to the reporter.

ACN: 891860 (2783 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Relative Position.Distance.Nautical Miles : 30
Altitude.MSL.Single Value : 9000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb

Component

Aircraft Component : Fire Protection System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 400
ASRS Report Number.Accession Number : 891860
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 891872

Person : 3

Reference : 3
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 9500
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 3800
ASRS Report Number.Accession Number : 891868
Human Factors : Training / Qualification
Human Factors : Human-Machine Interface
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

At gate while conducting preflight duties we received an Avionics Smoke ECAM. Visible condensation was present coming from all vents onboard the aircraft. I stopped the boarding process. The ECAM subsequently disappeared. No action was taken as all conditions were normal. Takeoff and departure were normal. At approximately 9000 FT we received a LAND ASAP message with no accompanying ECAM. The First Officer continued to fly as I researched the problem. When RCL (recall) was selected the Avionics Smoke ECAM was visible. No smoke was present in the cockpit or cabin (Flight Attendant's were contacted). We leveled off and began to return to MMUN. After a brief discussion we decided the ECAM was left over from the ground visible condensation condition. I contacted Dispatch and Maintenance to confirm the false LAND ASAP message referenced in the Flight Manual. With concurrence from Maintenance and Dispatch I used Captain's emergency authority to cycle the following circuit breakers as per Flight Manual guidance: FWC 1 SUPLY, FWC 2 SUPLY. The reset was successful and all indications returned to normal. I requested a new flight plan from VITAR Intersection with 25.0 FOB (fuel on board) to ensure FAR fuel was still on board. The flight proceeded without further incident. I have been told by our Dispatcher that this is a recurring situation in humid, tropical destinations. The procedure is in our Flight Manual. I am not sure why Captain's emergency authority must be used to solve a known system fault. Perhaps this needs to be addressed in a future Flight Manual revision.

Narrative: 2

On departure crew got a "Land ASAP" message. A phone patch was conducted with Maintenance, Dispatch and the flight. Crew followed the guidance in the Flight Manual for a "false LAND ASAP message" with Dispatch and Maintenance concurrence. Captains Emergency Authority was required to be used per the Flight Manual guidance. Message was cleared and flight continued on to destination.

Narrative: 3

During ground Ops/Preflight with high humidity/heat, we received an "Avionics Smoke" ECAM, which was only transient and which was cleared quickly. After takeoff during climbout, we received a LAND ASAP message with NO accompanying ECAM. I continued to fly the airplane while coordinating with ATC for return to the departure airport, while the Captain worked on the problem with Maintenance and Dispatch while referring to the False Land ASAP message referenced in the Flight Manual. Captain reset circuit breakers in coordination with Maintenance instructions. The problem was corrected and new flight plan obtained for the flight to our destination.

Synopsis

A320 Flight Crew and Dispatcher comment on circumstances surrounding a false LAND ASAP message with no accompanying ECAM.

ACN: 891802 (2784 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Light : Night

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 891802
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Entering weather data on performance page and FMGC's froze further inputs. Both FMGC's were unresponsive for the remainder of the flight. Captain could only access performance page with the winds remaining in the scratch pad. The First Officer could only access Flight Plan page. Neither pilot could make any changes on FMGC page displayed and could not select any other page. ACARS access was available. Contacted Maintenance Control and they suggested a procedure which only allows a reset on the ground. Neither pilot was willing to attempt a FMGC reset while airborne with potential random results. Advised ATC we had a frozen display and could follow the arrival route in heading. We were able to select VOR's and ILS' if necessary in standby mode. Asked ATC to follow our progress and if any deviations were noted to advise us. ATC was not busy and did not indicate they had any concerns with our request. Captain offered that if any deviations or problems arose that he would declare an emergency; also noted that if a go-around became necessary that declaring an emergency would be immediate. ATC provided vectors to final intercept. Weather was night VFR. Airport was acquired early. FMGC's auto tuned the field VOR and correctly tuned and ID'd the ILS as the anticipated arrival runway was the one we used on arrival. We wrote up both failed FMGC's and briefed maintenance on arrival. Aircraft had experienced a recent failure and replacement of FMGC #2.

Synopsis

An A320 Flight Crew experienced dual frozen FMGC displays while at cruise altitude. They continued to their destination utilizing standby navigation and helpful vectors from ATC.

ACN: 891355 (2785 of 2908)

Time / Day

Date : 201006
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : IMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Turbine Engine
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Check Pilot
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 891355

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 891541

Events

Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Gate
Result.Flight Crew : Rejected Takeoff
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

On takeoff roll at approximately 130 KTS bird ingested into left motor resulting in a loud bang and yaw similar to a tire burst. The aircraft was approximately 13 KTS below V1. Rejected take off, auto brakes decelerated properly initially then at between 80 to 100 KTS deceleration green light on braking system no longer showed aircraft decelerating. Advised IOE Captain to go to manual braking, [but] still no appreciable deceleration felt by Flight Crew. Advised Captain to stop braking and turned off the nose wheel anti skid, [and] had him reapply the brakes and called out brake pressures. Aircraft seemed to now be stopping. Brake temperatures were in the 860 to 915 range, brake fans immediately applied, cleared runway, [and] when brake temperatures showed below 550 started taxiing back towards the gate.

Narrative: 2

During take-off roll I saw birds. I [saw] them fly in front of the airplane and knew we would hit at least one but did not expect it to be a problem.

Synopsis

An A320 Flight Crew reported ingesting a bird on takeoff roll resulting in engine damage. They executed a rejected takeoff during which they experienced some difficulty with braking, and taxied to the gate.

ACN: 891200 (2786 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 32000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 750
ASRS Report Number.Accession Number : 891200

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 6400
ASRS Report Number.Accession Number : 891345

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise we experienced a Yellow system hydraulic failure. We executed the ECAM checklist uneventfully. We contacted our Dispatch and Maintenance Control to notify them of the failure and get any recommendations they could offer. We made a normal landing [at our destination].

Synopsis

An A320 Flight Crew reported losing their Yellow hydraulic system. After running the procedure and coordinating with Dispatch and Maintenance, they continued to destination for a normal landing.

ACN: 891187 (2787 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Hydraulic System Lines, Connectors, Fittings
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10350
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 2170
ASRS Report Number.Accession Number : 891187

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 7737
Experience.Flight Crew.Last 90 Days : 151
Experience.Flight Crew.Type : 255
ASRS Report Number.Accession Number : 891349

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During takeoff, upon gear retraction, we received an L/G Gear Not Uplocked ECAM. The cockpit duties were then divided with First Officer continuing to fly the aircraft and Captain handling the ECAM. The ECAM directed a recycling of the gear, which was done. Shortly thereafter, we received another gear not uplocked ECAM, followed by HYD G Engine Pump Lo PR. Captain selected the green pump off per the ECAM and noticed a decreasing quantity on the green hydraulic quantity indicator on the hydraulic schematic. At this time, we were talking to Departure and received clearance to climb to 7000 feet. As we were climbing out of 3000 feet, we received another ECAM - HYD PTU Fault. Per the ECAM, Captain turned off the PTU. We realized that continuing to destination was not a possibility with the gear still down and the loss of green hydraulics and informed Departure that we would be returning. First Officer put on the autopilot at this point to accommodate the division of duties. We received a southerly vector and leveled off at 5000 feet. We did not declare an emergency at this time. The Flight Attendants called us to let us know they still heard the gear down. Captain then informed our Flight Attendant that we had hydraulic issues and issued a cabin advisory. Captain also informed the passengers about a hydraulic malfunction and the return. Then we had another ECAM - HYD Y ELEC PUMP OVHT, followed by the autopilot disconnecting. First Officer continued to hand-fly the aircraft while Captain declared an emergency with Approach and addressed the Y OVHT ECAM. The yellow engine pump was then selected off per the ECAM. The Flight Attendants were then instructed to prepare the cabin for evacuation. Approach set us up with the longest runway. Emergency equipment was requested and a tug to clear us from the runway (with Dispatch) due to the loss of nose wheel steering. Since we were down to one hydraulic system and overweight (by 13,000 pounds), we consulted with Dispatch and Maintenance about the recovery of the yellow hydraulic system (as indicated in the irregular procedure). About 15 miles form the field the yellow hydraulic system was reselected on and was successfully restored. Captain coordinated with Dispatch for a tug and also requested landing performance data from Dispatch with the loss of green hydraulics. Captain also reviewed the landing distance abnormal or irregular configuration, whereby it was determined that with the yellow system back and normal landing configuration, a mostly normal landing could be made (except for the overweight status and alternate brakes). The prep for evacuation was canceled, but the cabin advisory remained. Checklists were referred to and accomplished (all of the above ECAMS and the overweight landing checklist) as well as normal checklists. Captain took control of the aircraft at about 1500 feet and landed the aircraft. The aircraft was brought to a stop straight ahead on the runway. It was not convenient to clear the runway without any nose wheel steering. Fire vehicles met us on the runway, and airport operations communicated with us from there. We shut down the engines, and indicated to the firemen that the brakes were set. They inspected the landing gear and informed us that there was a hydraulic leak in the left gear well area. The left brakes were then heating up, and we informed the Flight Attendants of a possibility of evacuation due to the leak with brake temperatures increasing and to stay alert. Passengers were informed to remain seated. The firemen were also requested to chalk the aircraft so we could release the brake to increase cooling, and they also got fans. The brakes finally peaked around 580 degrees then started cooling. Eventually a tug came out to tow us to the gate.

Synopsis

An A320 flight crew experienced the failure of landing gear to retract completely after takeoff followed shortly by a HYD G Engine Pump LO PR ECAM. The flight returned safely to the departure airport and was towed to the gate due to lack of steering.

ACN: 890890 (2788 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aircraft Auto Temperature System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12819
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 2847
ASRS Report Number.Accession Number : 890890
Human Factors : Communication Breakdown
Human Factors : Troubleshooting
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 16700
Experience.Flight Crew.Last 90 Days : 186
Experience.Flight Crew.Type : 5250
ASRS Report Number.Accession Number : 890893
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

A new maintenance release was sent to the aircraft. After the aircraft doors were secured and the jetway pulled away, the Captain looked at the maintenance release and noted that the maintenance action of replacing the zone temp controller was not on it. The Captain had the jetway returned to the aircraft and called for the pilot on special assignment. Once he arrived in the cockpit, the Captain asked him why the refusal with the replacement fix was not on the log. He made a phone call to confirm that the maintenance was documented. He gave the Captain this information which documented the fix to the refusal. ACFT DATA DISPLAY: DUE TO FWD CABIN DIFFICULT TO CONTROL. RPL ZONE TEMP CONTROLER. REPLACED ZONE TEMP CONTROL LER. OPS NORMAL () AMM REF 21. PARTS: NONE NEEDED INSP.

Narrative: 2

Captain determined due to ship's log history regarding temperature control in the cabin and the maintenance procedure accomplished that it would not be suitable for the long scheduled segment. Captain contacted Dispatch and requested a 3 way conference call with maintenance to discuss the aforementioned item. He conveyed in the call that we will be in operations standing by for the developed plan. A gentleman with a fluorescent green vest and radio device arrived and entered Flight Ops Management Offices. He walked out shortly after and accessed one of the computer terminals and began typing away. My first impression was that he was with maintenance. He never approached us sitting in the table by the back. He stated out loud that our aircraft was ready. Captain asked what had been done. The gentleman stated that the temperature zone controller was replaced. We expedited to the aircraft. Upon arrival all the passengers had been boarded. Upon entering the aircraft we were constantly asked if the door can be closed, distracting the Captain and me from our preflight duties. After our preflight checks the Captain gave the OK to close the door. When the Captain reviewed the new and current maintenance release, he could not find any record of the maintenance performed as mentioned by the gentleman with the fluorescent green vest and radio in flight operations. Captain requested a Flight Operations Representative to come to the aircraft. The jet bridge was reattached and the gentleman with the fluorescent green vest and radio walked up from the ramp and into the flight deck. He stated to the Captain that it is a "glitch" in computer but that the aircraft was released and the log history does reflect the replacement of the temperature zone controller. He offered to bring a new printout of the log history. Captain said "I trust you." Captain had access to updated log history through ACARS. We departed. En route we had difficulty controlling temperature in the both First Class and Coach cabins. No faults were displayed. Approximately 3 hours into the flight we had a Pack 1 overheat. ECAM actions accomplished. Pack returned to normal operations. Shortly after that we had the Pack 1 Air Regulator Fault. 2 (two) ECS 19 reports were printed, 1 before pack overheat due to temperature control difficulties and 1 after the pack fault and placed in the green book.

Synopsis

An A320 crew refused an aircraft with a history air conditioning problems. Maintenance claimed to have repaired the system but while enroute numerous air conditioning control issues developed indicating that the maintenance had perhaps not addressed the system.

ACN: 890846 (2789 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : IAD.Airport
State Reference : DC
Relative Position.Distance.Nautical Miles : 5
Altitude.MSL.Single Value : 1500

Aircraft

Reference : X
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : IAD

Component

Aircraft Component : Fuel System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20005
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 4005
ASRS Report Number.Accession Number : 890846
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Received a L+R WING TK LO LVL ECAM, with land ASAP message. Fuel quantity was normal, and we had done several fuel quantity checks enroute. Suspect the opening of the outer wing tank valves caused this momentary ECAM, which did go away.

Synopsis

A320 Captain experienced a L+R wing tank low level ECAM message during approach. Fuel quantity was normal and ECAM quickly disappeared.

ACN: 890768 (2790 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger

Component

Aircraft Component : Hydraulic System Pump
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 150
ASRS Report Number.Accession Number : 890768

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On descent we lost our Blue Hydraulic system electric pump. Among the ECAM messages was something like "blue system press low". As the flying pilot I continued to fly the aircraft and obtained delay vectors while the Captain worked the problem. With dispatch we determined it was best to divert due to landing distance requirements with the blue hydraulic system out. Divert was uneventful.

Synopsis

An A320 First Officer reported losing blue hydraulic system electric pump. The decision was made to divert to an airport with a longer runway.

ACN: 890755 (2791 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Airspace.Class A : ZZZ

Component

Aircraft Component : Air Conditioning Distribution System
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17800
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 3600
ASRS Report Number.Accession Number : 890755
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
ASRS Report Number.Accession Number : 892372

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Just after takeoff we noticed there was no airflow in the cockpit, the packs were on and the aircraft was pressurizing. The trim air/hot air system was deferred. I turned on the trim air/hot air system and the air flow resumed. Temp and pressure were normal. Phone patched with Maintenance Control and Dispatch and all agreed to continue on to our destination and to monitor the situation. The rest of the flight was uneventful and all systems operated normally.

Narrative: 2

At this point I referenced the A320 Flight Manual "Emergency"and "Irregular" chapters and found there were NO reference to ANY Checklists or Procedures for "NO/LOW AIR FLOW" in the Cockpit/Cabin, only "TEMPERATURE, Cockpit or Cabin Too Hot/Cold".

The First Officer and I agreed that: "WE COULDN'T CONTINUE THE FLIGHT MUCH LONGER" under these circumstances. I again scanned the instruments and systems and surmised that we were approaching an emergency situation that would require an Emergency return to our takeoff airport, and could possibly require a depressurization in an attempt to cool the aircraft, the occupants and its systems. Determining that all systems were normal EXECPT for the "HOT AIR SYSTEM" I suggested to the First Officer that before we have to possibly "Dump the Cabin" (open the outflow valve), declare an emergency, and put the avionics cooling systems in jeopardy, we try switching the "HOT AIR SWITCH" to ON. I stated: "if that does not work we would still have to do all the above"and that the switch would be turned off again and that it would be a moot point if we needed to de-pressurize I asked my Copilot if he agreed, and upon his affirmation: My switching ON the "HOT AIR SYSTEM" immediately restored NORMAL airflow to the ENTIRE Aircraft. 

I used my emergency authority as Pilot In Command to propose and execute a plan of action I believed was both safe and prudent, and took into consideration the well being of my Passengers/Co-workers and maintained aircraft systems integrity.

Synopsis

An A320 flight crew faced with a loss of air conditioning airflow utilized the Captain's authority to operate the deferred hot air/trim air system. Doing so returned system operation to normal.

ACN: 890703 (2792 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : D01.TRACON
State Reference : CO

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Fog
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : D01
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Airspace.Class E : ZDV

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 890703

Events

Anomaly.Inflight Event / Encounter : Fuel Issue
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

About 80 NM from DEN we were advised that severe thunderstorms were in progress and they had shut off arrivals. We entered holding over HGO. Our alternate was ABQ. We were advised by ATC that the severe storms over the DEN airport were moving little and they expected the airport to remain closed for at least an hour. We only had 25 minutes of holding fuel. After extensive discussion with our Dispatcher we decided to divert to COS. Approximately 35 other aircraft diverted to COS. 22 passengers wanted to deplane at COS and arrangements were made for them to do so.

After about 90 minutes we were refueled and got a release for the flight to DEN. We started engines and taxied for takeoff. Once again DEN Approach Control shut off arrivals to DEN. After about a one hour delay we departed for DEN. Flight to DEN was normal.

Synopsis

An A320 diverted to COS when their destination airport was closed by unforecast severe thunderstorms.

ACN: 890564 (2793 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : IAD.Airport
State Reference : DC
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use.Other 
Airspace.Class B : IAD

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : IAD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Flight Phase : Landing
Airspace.Class B : IAD

Person : 1

Reference : 1
Location Of Person.Facility : IAD.Tower
Reporter Organization : Government
Function.Air Traffic Control : Flight Data / Clearance Delivery
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 890564
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : IAD.Tower
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
ASRS Report Number.Accession Number : 890594

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

An Airbus was in position on Runway 19L with another Airbus was on final for Runway 19L. The Airbus on final was fast due to a tailwind and was S-turning. The departing Airbus was cleared for takeoff and rolling. The squeeze play was tight and as the situation progressed it became fairly apparent that it wasn't going to work out. As the arrival crossed the threshold LC1 failed to send him around. At this point the departure aircraft was approximately 4,000 feet down the runway and had not rotated. When the arrival was around the 1,000 ft marker and flaring out LC1 issued go-around instructions which was too late for the aircraft to comply due to the phase of flight. At this point the departing Airbus was still on departure roll and not airborne. I recommend that if an airplane needs go-around instructions, they need to be issued the go around prior to the arrival aircraft crossing the runway threshold. Better control judgment should have been applied. Perhaps maintaining a higher level of situational awareness should have been applied. I think if aircraft limitations were understood better and controllers can think more about the high levels of concentration required by flight crews upon landing then this situation could have been averted. The aircraft should have been issued go-around instructions prior to crossing the runway threshold.

Synopsis

IAD controller described a very close arrival vs. departure event involving two Airbus aircraft, the departure was approximately 4000 down the runway when an arrival crossed the runway threshold, the arrival unable to comply with go around instructions issued during flare out.

ACN: 890187 (2794 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Electronic Flt Bag (EFB)
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 890187
Human Factors : Workload
Human Factors : Situational Awareness
Human Factors : Distraction

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 889870

Events

Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Other Person
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was advised by the Flight Manager that we had operated a flight with an expired EFB containing old airway and performance data. Although I believe that I checked the dates against the dispatch release, it is possible that I did not recognize the expired condition. To my best recollection the release and EFB dates corresponded, but that may have been different than the actual date. I did not recognize the error/series of errors that lead to the occurrence. I suggest that a higher degree of vigilance in an otherwise simple task as well as crosschecking could prevent a recurrence of this event.

Narrative: 2

I was informed two days after the flight that I operated with an outdated EFB revision. During the preflight phase of this flight I did in fact check that the effectivity dates did match that which was on my flight release. Though the dates did match, I failed to realize that the dates on both, though matching, were in fact expired. The flight was operated with no further incident.

Synopsis

An A320 flight crew reported conducting a flight with the EFB out of date. They preflighted the aircraft but somehow missed the expiration date.

ACN: 890177 (2795 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 500

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : VFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 890177
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Training / Qualification

Events

Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Detector.Person : Observer
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted
Result.Flight Crew : Returned To Departure Airport
Result.Flight Crew : Landed in Emergency Condition
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

Fight departs VFR. Conditions dry runway. After gear retraction about 500 FT #2 engine ingested two large birds. We experienced two "thuds" with compressor stalls. N1 and N2 on #2 started to roll back (decrease) then recovered. Vibrations indication showed .5 N1 and .7 N2. Observer on the ground reported two flashes of red and two plumes of black smoke from the #2 engine. Take off weight 162,321 LBS. APU bleed on for increase performance. Flight time 10 minutes total. No odor of bird inside cabin due to APU bleed used for takeoff versus engine bleed. Visual inspection on ground of #2 engine revealed bird strike residue. [We] immediately returned for precautionary landing. Landing Weight 160,400 LBS. VRF 150 KTS max vertical decent at touch down with minimal reverse and low auto brake. [We had an] uneventful taxi in and shut down; contacted Dispatch/Maintenance Control; deplaned passengers [and] canceled flight.

Synopsis

An A320 crew reported ingesting two large birds in the #2 engine. After compressor stalls the engine continued to run as the crew declared an emergency and returned land. The aircraft was removed from service.

ACN: 889526 (2796 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap/Slat Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 889526

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Downwind [with] flaps one. Cleared visual, turned base, put flaps to 2. Got ECAM "FLAPS LOCKED ASYM". Stopped approach and worked problem. Notified ATC, Dispatch, Flight Attendant's. Landed partial flap.

Synopsis

An A320 Captain reported ECAM "FLAPS LOCKED ASYM" message on downwind. They ran the procedure, coordinated with Dispatch and ATC, and landed with partial flaps.

ACN: 889105 (2797 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 18000

Environment

Weather Elements / Visibility : Thunderstorm
Light : Dusk

Aircraft

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Route In Use.SID : MAXXO 1
Airspace.Class A : ZAB

Component

Aircraft Component : Air/Ground Communication
Aircraft Reference : X
Problem : Improperly Operated

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 889105
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Took Evasive Action
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

While climbing above FL180 the departure, the pilot not flying and I noticed thunderclouds nearby. We saw cloud visually, as well as on radar, painting some red in the cloud. The pilot not flying and I discussed and agreed to deviate left (north) of course, for weather avoidance. Three or four times the pilot not flying tried to contact ABQ Center for permission to deviate, with no reply from ABQ. Meanwhile, the frequency was a bit busy with other aircraft checking in, and ABQ Center issuing weather advisories. Faced with the choice of penetrating a cloud returning red echoes, and deviating without clearance from ATC, we opted to choose what we felt was the safest option and deviated left for a short time. We assumed the pilot not flying's radio was dead, so the pilot flying tried to contact ATC with no answer. [We] finally got a hand off from ATC to new frequency which the pilot flying answered. Upon checking in with new ATC frequency we advised ATC of our deviation, and that we were returning on course. At about this time, we realized the pilot not flying's microphone jack had come disconnected, and that's why ATC could not hear us.

Synopsis

A320 First Officer reports lost communications with ABQ Center and deviates without clearance to avoid weather. It is later discovered that the Captain's microphone jack had come unplugged causing the outage.

ACN: 888745 (2798 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : PTK.Airport
State Reference : MI

Aircraft

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.STAR : CRUXX 4
Airspace.Class A : ZAU

Person

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 888745
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

This aircraft evidently did not have the CRUXX 4 arrival after LFD. When I talked to the aircraft, his route showed he had been issued the CRUXX 4 arrival. I had no reason to doubt or believe he was not going to fly arrival so I descended the aircraft to FL350 and shipped to Joliet Sector. The real issue here is the automation over the LFD fix. Aircraft going to DTW and PTK over LFD, if you give aircraft direct LFD and enter it on the R-Side it will remove the arrival from the route of flight. This is a known issue that has been around as long as I have been at ZAU. I would like to know why automation has not updated/fixed this known problem. If you enter direct LFD from D-Side it leaves the arrival in place but many times you are working without a D-Side and do all entries from the R-Side. This is an issue that needs to be fixed in a "safety first" culture. Recommend automation FIX adaptation over LFD fix for DTW and PTK arrivals. This issue has been known for years.

Synopsis

ZAU controller described a long standing automation anomaly that alters flight plans depending upon the data entering position.

ACN: 888530 (2799 of 2908)

Time / Day

Date : 201005

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 888530
Human Factors : Fatigue

Events

Anomaly.Other 
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Work Refused

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I called fatigued for [a trip assignment]. Five days of reserve. My first 4 days were filled with [an assignment]. I had just finished this 4 day trip, with 24 actual hours of flying, which included early morning wake ups every day, and for some reason I was struggling to feel rested for the entire 4 days. I was physically and mentally exhausted after this flying, dealing with challenging weather and ATC delays every day. *On day 3, we exceeded 8/24*. At the end of this, the crew desk attempted to assign me [another trip] for the following day. I called fatigued for this trip. It was a one day turn, with 8:36 flight time, and an additional 10+ hour duty day. To make it legal, it was built with a deadhead on the last leg to keep me from exceeding and 8/24 and 30/7. To me, this was too onerous and I called in fatigued.

Synopsis

An A320 Captain reported calling in fatigued after four difficult days on reserve.

ACN: 888445 (2800 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : SFO

Aircraft : 2

Reference : Y
Aircraft Operator : Air Carrier
Make Model Name : B747-400
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 888445
Analyst Callback : Attempted

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We taxied to our assigned Runway 1L during night operations at SFO. We were then instructed to taxi into position and hold on Runway 1L. Aircraft were taking off on 1L and 1R and were landing and taking off on the 28 runways. We were cleared for takeoff and started our takeoff roll. Approaching V1 we noticed a 747-400 on taxiway F between runway 1L and 1R waiting to cross 1R. We reached VR approximately right behind the 400 and started our rotation. At liftoff our aircraft started to violently buffet left and right several times. I had to use full flight control deflection to maintain control. Once we reached about 50 feet the aircraft became controllable and everything smoothed out. My F/O and I surmised that the 400 was on ground control frequency and had been cleared to cross 1R shortly after we started our takeoff roll. Since we were on different frequencies we were not aware of his status or intentions. That aircraft obviously had to use considerable power to get rolling again and probably wanted to expeditiously cross. We just happened to rotate right into his thrust and barely recovered. Since 1L and 1R are so close together you can imagine how close we were to the rear of that aircraft. This is an extremely dangerous scenario and came close to ending tragically. This definitely needs to be addressed with SFO ground/tower operations.

Synopsis

Just after rotation on Runway 1L at SFO an air carrier A320 encountered jet blast from a B747-400 on an intersecting taxiway. Full control deflection was required to maintain control.