ACN: 888305 (2801 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Facility : SFO.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 888305
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Air Carrier XXX was #1 for departure off Runway 1R, the strip I was provided was Air Carrier XYY. The Air Carrier XYY departure strip was at Ground Control. During a extremely busy time, I taxied Air Carrier XYY into position and hold. Once traffic permitted, I cleared Air Carrier XYY to depart Runway 1R, and at about 1/2 mile upwind, I advised Air Carrier XYY to contact NCT Departure. After switching to NCT, the aircraft tagged up as Air Carrier XXX and NCT called to ask us to scan Air Carrier XXX again. The RADAR Coordinator did some coordination with NCT and we discovered that the aircraft I called Air Carrier XYY on every transmission was actually Air Carrier XXX. I have not listened to any recordings, but if Air Carrier XXX used his correct call sign I did not catch the incorrect identification. In addition Air Carrier XXX took every one of my clearances for Air Carrier XYY. Note: no loss of separation occurred as Air Carrier XYY and Air Carrier XXX were fortunately on identical departure procedures. Pay closer attention to aircraft callsign during readbacks.

Synopsis

SFO Controller described confused similar same company call sign event when the wrong strip was utilized and the wrong callsign used and read backs missed during all tower operations.

ACN: 888195 (2802 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Hydraulic Main System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Commercial
ASRS Report Number.Accession Number : 888195
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Rejected Takeoff
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Shortly after pushing the power to flex we got a ECAM that the Yellow HYD Reservoir was low quantity. I aborted the takeoff at about 60 KTS. We returned to the gate and on post flight inspection found a large leak in the wheel well. The aircraft was repaired the next day and we returned our base the next day.

Synopsis

An A320 Captain reported rejecting a takeoff at 60 knots after the Yellow Hydraulic Reservoir alerted low quantity on the ECAM.

ACN: 888179 (2803 of 2908)

Time / Day

Date : 201005

Place

Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 7800
ASRS Report Number.Accession Number : 888179
Human Factors : Fatigue
Human Factors : Workload
Human Factors : Situational Awareness

Events

Anomaly.No Specific Anomaly Occurred : All Types
Detector.Person : Flight Crew
When Detected : Pre-flight
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

This is report three of a series of three for the day's flying. We operated this flight after flying two legs that had event sets. We got into this airport about 40 minutes behind schedule. The day's flying was built for efficiency, no prolonged breaks, no long sits, arriving at or about the time most food facilities are closing down, getting into our RON station at about the beginning of the window of circadian low (WOCL). The delays from the previous leg that were due to a passenger issue and weather pushed the day even longer. Our aircraft for last flight was delayed by the same weather we had just experienced and that we were going to go back through and came in from a Midwest airport. Scheduled departure time was XA:17. We blocked out at XB:00, approximately 2300 for our body clocks. I was aware at arrival in at this intermediate airport that the day's events were going to take a toll. I went for a walk to get rid of some of the residual chemistry of fatigue and perk myself up. I found a food vendor that I could purchase a muffin, a bag of potato chips and an orange from. There were other opportunities to eat during the day so it wasn't a case of nourishment. I needed some quick energy release food to help maintain alertness for this 1 plus 29 flight. I used the same techniques that athletes use when training for an event, walking/pacing out, breathing exercises, not overeating, drinking enough water in small amounts throughout the day. I used some caffeine but in very small quantity. I had a cup of coffee at about noon, and I had one full can of Diet Coke about three hours prior to this flight. Arrival weather at this airport was a significant factor. We had found moderate turbulence on our descent. The same turbulence was waiting for us as we departed. We avoided the intense part of the storms by weaving across the outbound course. When we finally were able to go direct we found winds at our altitude in excess of 30 KTS above forecast for any altitude (FL230 wind on ND was 210@105). These winds were associated with the weather. I noticed that I was becoming shorter and more abrupt in my direction to first officer. As one becomes fatigued, communication skills erode and this was an early indicator. The problem is that both he and I had been awake since early in the day. I woke up about mid morning and had a "lazy, lounging" morning. I rested, relaxed, sat at my computer for a while, paid a few bills, but I did not even dress for work until just before going to work. My First Officer, however, has two small children that are awake early before sun up. He did not get a nap or any lounge time before beginning this day's adventures. The real challenge of this job is trying to make it all work out even with limited or declining resources. The most important resource is pilot alertness; once that is lost, displaced, slips away due to intruding thoughts, activities or other businesses the burden on the remaining pilot increases ten-fold at least. I'm sure I've been on the donor side of the coin in the past. But this night I was the person carrying the load. I might have been snappy with my First Officer in my remarks and more directive than exhortative, nice way of saying I ordered, I didn't ask. But I was aware that we were both declining in our alertness. Another thing that happens is selective hearing. Stuff gets missed, or you think you heard what you didn't hear or you hear what you want to hear. We were very careful to double and triple check what was said by controllers. It isn't easy when you are too tired to think. The flight attendants made only a couple of very simple requests. Unfortunately we had been dealing all day with a call chime that had bad news at the other end and problems to solve. I asked my First Officer to handle all interactions with the in-flight crew because I was very likely to snap at them. Every time the call chime went off I was "jumpy" and irritated by it. The last flight operation was without incident. We performed a flaps 2 TOGA takeoff and made a turn to heading 260 as part of weather avoidance techniques. We delayed rotation until 151 KTS, the Vr for our runway limit, only an additional 10 KTS. We kept the flight attendants down until about FL260 and well clear of the storms. We did all of this but it felt disconnected to me. I am writing this report the day after and have an awareness that if not for the training, if not for the years of experience, if not for a long list of excellent mentors and conditioning for this type of event over the years, it wouldn't have come off as smoothly as it did because the part of my brain that has self-awareness, was not fully operating at the time. I was overly dependent on automation, and so tired that I wasn't able to accurately assess my own condition. Arrival at RON station was also uneventful but having that disconnected quality about it. On taxi in to gate I noticed my visual acuity was "bottoming out." I couldn't make out the large lettering for the gate, the large white sign on the top of the jetbridge, until I was relatively close. When visual acuity begins to deteriorate, that is a bad thing. The timing was pretty much perfect for completion of a flight without incident. But fatigue had fully set in. We arrived at the hotel in a timely fashion, and the First Officer's phone rang. It was obvious that his day was far from over, so we said goodnight and I headed for bed. I woke up the next morning at XA:30 local, XB:30 local and then at XD:00. I wrote the first two reports and went down to breakfast at XG:00 am. I went for a short walk and then took a one hour nap. I wrote this report at about XM:39, just prior to the original departure time. As I was hastily preparing to load up and check out, the phone rang. Today will not be a good day either, we are delayed one hour plus forty already. Back through the weather; pickup time will also be delayed. I am almost back to full capacity, but I can tell that it will be about two more days before the fogginess is fully gone. I live locally at my domicile. I do so because I commuted in the past and I understand a commuter's pain. It is a far easier life being very close to the airport. It makes pairings like this one much easier to handle. The reality of fatigue mitigation is it must be planned for by the individual. No amount of rule or policy making will make fatigue go away. 

That's my fatigue report. I hope I've provided you with useful data. Besides having some sort of objective way to determine fatigue levels, we need fatigue mitigation training that is more than a page in our recurrent training. It needs to be a presentation that is visual and holds our attention. We need further conditioning to manage fatigue if we are going to continue to fly our current ambitious schedule.

Synopsis

An A320 Captain reported feeling fatigued by a combination of preflight delays, weather, and passenger issues. The reporter suggested fatigue mitigation training be developed to help pilots maintain alertness when emotional and circadian factors begin wearing them down.

ACN: 887895 (2804 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZMP.ARTCC
State Reference : MN

Aircraft

Reference : X
ATC / Advisory.Center : ZMP
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Initial Approach
Airspace.Class A : ZMP

Person

Reference : 1
Location Of Person.Facility : ZMP.ARTCC
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 887895
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

The local PM push. TMU (Traffic Management Unit) had us metering and delaying aircraft for no reason. The final at M98 never got out past 20 miles. We were delaying aircraft 15-20 minutes. At one time all of M98 only had 14-16 aircraft total including departures. There were 10-20 mile gaps; sometimes 30-40 mile gaps; in the earlier push, the same thing. I called TMU and asked how much longer are we metering, and they said M98 has a glitch in the metering program so we are delaying aircraft for glitches. I said how about looking at the final and seeing that it is drying up and terminate metering so we can provide a service to the public. Recommendation, get rid of the metering program. Most people in TMU haven't spaced aircraft. We delay aircraft way too often for no reason or just because the metering program says so. Controllers use judgment, TMU's should have to as well. We delay aircraft way to often unduly, almost daily. Allow the controllers more say. They have a better idea of traffic situations.

Synopsis

ZMP Controller expressed concern regarding the ZMP TMU's delaying actions involving M98 arrivals.

ACN: 887851 (2805 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Fuel System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 887851
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Primary Problem : Manuals

Narrative: 1

Refuel valve actuator was installed in right (RT) position which I thought in accordance with the Aircraft Maintenance Manual Illustrated Parts Catalog (IPC) reference figure was the center position. Maintenance Records found the error.

The Aircraft Maintenance Manual Illustrated Parts Catalog reference figure showed right wheel-well position. Maintenance Control had the aircraft stopped and the discrepant part changed. I have suggested the IPC reference figure be updated.

Synopsis

A mechanic installed an A320 refuel valve actuator in the incorrect position based on an incorrect figure in the Illustrated Parts.

ACN: 887343 (2806 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.AGL.Single Value : 200

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : SFO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : SFO

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 887343

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 887348

Events

Anomaly.Inflight Event / Encounter : Unstabilized Approach
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

At 200 FT on short final we encountered turbulence which rolled the aircraft 35 degrees to the right. The First Officer was flying and applied opposite control and the aircraft recovered. I decided to go-around in the interest of safety as we were no longer lined up and were now slightly high. This took place on Runway 28R and a 767 had just landed before us on Runway 28L. The wind was 220/20G26 and I believe we encountered some wake turbulence from that 767. We came around and landed uneventfully on Runway 28L.

Synopsis

An A320 flight crew reported encountering wake turbulence on short final behind a B767 landing on a parallel runway at SFO, a go-around was executed.

ACN: 887243 (2807 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 10000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 887243
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Descending through 10,000 ft, I pulled up the RAD NAV page and discovered that ALL the data normally there was missing. About the same time ATC issued a turn and instructed us to intercept the localizer. I turned to the assigned heading and selected the LOC PB, and confirmed LOC on the FMA. I also noticed that the lateral displacement from the localizer course was approximately 6 miles. Just about the time I realized that the aircraft had not intercepted the LOC course, ATC indicated we had gone through the localizer and he issued a turn to re-intercept the localizer. We indicated we had the airport in sight and were subsequently cleared for the visual approach. As far as we could tell, there was no conflict with any other traffic and ATC made no indication that there was any problems associated with this incident. Indications were that all systems were normal with the NAV system on the aircraft except for the lack of data on the RAD NAV page. I discovered this fact when I selected RAD NAV to hard tune the VOR. I was "scratching my head" trying to figure out what was going on when I discovered that we had flown through the LOC course. This was an inadvertent error on my part and it illustrates how a simple problem can distract the crew from the task at hand. Having had this happen to me I would be aware of the situation if it were to occur again, but given the circumstances and the chain of events that occurred, I could envision other crews making the same inadvertent error as we did.

Synopsis

An A320 Captain reported flying through the localizer on the approach when he was momentarily distracted with an FMC problem.

ACN: 887240 (2808 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZDC.ARTCC
State Reference : VA

Aircraft

Reference : X
ATC / Advisory.Center : ZDC
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 887240
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Company Policy
Primary Problem : Company Policy

Narrative: 1

Inbound to PHL, while on Washington Center frequency, we observed numerous similar call signs, all of which were the same company flights. Our flight number was ABC. There were a minimum of 7 similar call signs on the same frequency, also apparently inbound to PHL. ATC began issuing holding instructions to the aircraft inbound to PHL. Along with holding instructions, ATC was issuing numerous altitude change clearances as aircraft were entering and exiting holding. ATC frequently repeated clearances, due to the number of similar call signs, further congesting an already highly-congested frequency. Cause, the department responsible for assigning flight numbers did not adequately consider the safety implications of numerous similar call signs. Suggestions, the assignment of flight numbers is a function presumably performed within marketing or another department other than flight operations. I strongly believe that flight operations should provide input and coordinate to preclude assignment of similar call signs. The hazard identification and risk assessment process under SMS would be extremely useful in addressing this issue.

Synopsis

Air carrier Captain on arrival into PHL voiced concerns about clearance complications that resulted from multiple flights with similar callsigns.

ACN: 887156 (2809 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZID.ARTCC
State Reference : IN

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Center : ZID
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Cruise
Airspace.Class A : ZID

Person

Reference : 1
Location Of Person.Facility : ZID
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 887156
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working APE HI RADAR. I had a D-Side working at my sector with me. A huge area of thunderstorms were north of my sector surrounding DTW area airports. I was working the DTW arrivals northbound. At the time the DTW arrivals were required to be only on the GEMNI arrival 10 miles in trail. The super high sector, LCK SH, and high sector south of me, UNV HI, were feeding me DTW arrivals that still needed to be tweaked to meet the spacing requirements. During the spacing and working of traffic in the sector, ZOB RAV HI called and said they needed all the DTW arrivals now on the WEEDA arrival. My D-side and I started moving the aircraft to this arrival west of where the aircraft were. We told LCK and UNV that the aircraft needed to be swapped routes. ZOB RAV HI called again and said that they needed all the aircraft on 360 degree headings and all slowed to 250 knots. We once again started working to provide RAV with their requirements. We told LCK and UNV that we needed to slow the back aircraft first so we can keep the spacing. During this time I frequency switched Air Carrier X to RAV HI without a hand off. RAV HI called our sector and my D-Side answered them. They told him to flash the hand off on Air Carrier X to them. I looked and saw Air Carrier X still in my airspace slant zeroed (indicating I frequency switched him) without a hand off to RAV HI. Air Carrier X's data block did have a speed and heading entered in the fourth line of the data block. I then flashed the data block to RAV HI and they took the hand off. Recommendation, I take great pride in my work and am dismayed that this happened. It is my own fault that I did not hand off the aircraft to the next sector.

Synopsis

ZID Controller initiated a frequency change to the next sector without completing a hand off, noting weather caused re-routes and DTW in-trail requirements complicated the situation.

ACN: 886800 (2810 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 37000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Navigational Equipment and Processing
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 20005
Experience.Flight Crew.Last 90 Days : 270
Experience.Flight Crew.Type : 4001
ASRS Report Number.Accession Number : 886800
Human Factors : Distraction
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During cruise, the First Officer noticed that neither MCDU was responding to any typed input, and were stuck on one page. [We] notified ATC that we were using standby navigation, and needed VOR frequencies and headings. I asked the First Officer to try the flight manual MCDU reset procedure which was unsuccessful. He then called Dispatch and Maintenance Control to advise of condition. Maintenance Control offered no fix, so we proceeded to our VMC destination and landed uneventfully. After local maintenance reset the system on the ground, all systems worked normally, and we returned to Chicago with no further problems.

Synopsis

An A320 flight crew encountered frozen MCDUs and continued to their destination utilizing standby navigation.

ACN: 886789 (2811 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Thunderstorm

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 700
ASRS Report Number.Accession Number : 886789
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

For starters, this may not be the appropriate avenue since this episode is primarily an ATC event and nothing wrong happened, other than the way we were hassled by ORD tower, and earlier in the day in pretty much in the exact same fashion by IAD tower. Typical summertime weather day with boomers all over creating the usual havoc with the ATC system. We were issued a PDC with an Estimated Departure Clarence Time for wheels up. Prior to push we confirmed our estimated departure clarence time. Upon initial taxi we again confirmed the time with ground. Prior to shutting the engines down in the block we again confirmed the wheels up time, this time with tower. ATC tower advised us of a new time. At XA:00 I started the left engine. At about XA:00 and 30 seconds tower wanted to know if we were ready to go. "No" was my reply. This began the "well why not" line of questing as I started Engine No 2. Next comes the instruction to taxi up and hold short and to be ready. "Unable, we need 2 minutes" was my reply. The second engine was still starting, the control checks hadn't been done, people were still up in the back, the checklist hadn't been started etc. "Well if you don't go NOW you'll mess up the flow and you'll lose your slot time and we need you to go now." He was agitated and and it showed. We had confirmed our time on three occasions to prevent this exact scenario, ATC had pushed our time up by nearly 16 minutes without a word. Never mind the fact that we told him that our engines were shut down. The controller then wanted to know when we would be ready and I said that we haven't even begun the checklist yet, as number two was still starting. "We still need two minutes". With this we were greeted with an invitation to lose our slot and get back in line. Had I been the Captain, I would have gladly taken him up on it. I'm sure that other planes in the block who had no idea how we got to this juncture thought we were inept. Fortunately, my Captain was a good sport and didn't try to rush either. He had rushed a 757 crew right before us. Not intentionally, but by revising their wheels up time and or sequence as well. This exact same scenario played out just 3 hours earlier in IAD with almost the same frenetic response to our "unable" after our wheels up time had been pushed up without our knowledge and with minimal notification. In both cases ATC was aware that our engines were shut down. In both cases, they changed our sequence at the last minute and didn't allow us time to get our fertilizer in a singular parcel. I HATE being rushed and will not tolerate it during a critical phase of flight. It's a bit of a tightrope walk as a First Officer because not all Captains share my attitude. I'm not a sloth, I can get this stuff done fairly quickly but I will not allow anyone to rush me into a stupid and/or unsafe situation again. Those days are over. I am paid to be methodical and deliberate. I suppose that this is more venting than anything, but if we had something go wrong because of a Helter Skelter approach to getting that mother in the air I'd be filling out a job application at Wal Mart instead of this FSAP. Have a swell day and thanks for reading this.

Synopsis

Air carrier pilot voiced displeasure with ATC handling with regard to expected release times, engine start changes and the general uncertainty of ATC release procedures, listing ORD and IAD as example airports.

ACN: 886639 (2812 of 2908)

Time / Day

Date : 201005
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15800
Experience.Flight Crew.Last 90 Days : 135
Experience.Flight Crew.Type : 5600
ASRS Report Number.Accession Number : 886639

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

3 hours 30 minutes into the flight, while checking the system status via the lower ECAM, we noticed that the Yellow Hydraulic quantity had fallen to just above the low level mark. We contacted Dispatch and Maintenance Control to discuss the problem. During the call the ECAM HYD Y RSVR LO LVL illuminated. The decision was made to divert. An uneventful approach and landing was made and on postflight inspection a copious amount of hydraulic fluid was found to be leaking from the main landing gear doors.

Synopsis

An A320 flight crew diverted to an enroute airport when the Yellow System Hydraulic fluid low level ECAM message was displayed. Post flight inspection provided visual evidence of the loss of fluid.

ACN: 886041 (2813 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Flight Phase : Descent
Airspace.Class A : ZZZ

Component

Aircraft Component : Toilet Furnishing
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 886041
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Illness / Injury
Anomaly.Inflight Event / Encounter : Fuel Issue
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Before top of descent, the Flight Attendant's called and informed me that the aft lavatory was flushing continuously. I was concerned about the ramifications of that, and called Maintenance and Dispatch. My main concern was that when the vacuum system ceased to function as we descended through FL180 or so that the flush motor would then be running continuously. I asked Maintenance if there was a danger of the motor overheating and causing a fire, and he said he hadn't heard of that happening, but couldn't be sure. I asked if there was anything I could do, and was told that I could pull the Flush system CB [Circuit Breaker] in the aft galley, but doing so would be using my emergency authority. I was concerned enough about the motor overheating and causing a fire during the approach that I elected to do that, and preceded to the aft galley to pull the CB. I was informed by the Flight Attendants that as we descended below FL180 the toilet stopped flushing, which is what I was hoping would happen. I felt that in order to operate the flight at the highest level of safety that exercising my emergency authority to pull the CB in flight was the correct action.

Descending to our destination, we were given holding. The field was VFR, and holding was not forecast or expected. We were immediately talking to Dispatch about our options, because fuel was going to be critical very quickly. As we were about to divert to another airport, we got a call from the purser stating that a female passenger in her 20s, was unconscious and that they would get back to me when they could. Knowing that fuel was getting to a level where a decision had to be made immediately, I had my First Officer declare a medical emergency and requested priority handling. I coordinated with Dispatch to have the paramedics meet the flight. We were very busy but, as we turned about a 15 mile final, we were able to ascertain from the Purser that the passenger was on O2, had started to regain consciousness, and was traveling with her parents. The parents at this point said that they thought that she would be fine, and didn't want to bother with the paramedics anymore. I told the Flight Attendant to inform them that we had already declared an emergency, and would be landing shortly, and that the paramedics would be there. I also requested that they keep everyone seated until the paramedics had reached the passenger who was in distress. The approach and landing were normal, and the paramedics were waiting for our arrival and boarded the airplane.

Synopsis

An A320 Flight Crew declared a medical emergency due to an unconscious passenger and landed at their destination. The event was preceded by an anomaly regarding a toilet flush motor that caused the Captain to exert his command authority to pull the flush motor circuit breaker.

ACN: 885830 (2814 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 170
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 885830
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Hazardous Material Violation
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

We were faced with an airplane change, maintenance items found on preflight, pressure for on-time departure PLUS a HAZMAT conflict. Departed gate with the HAZMAT question unresolved. Ultimately returned to gate and discovered HAZMAT had been loaded without paperwork to crew. The First Officer returned from his walkaround with two HAZMAT forms. The ACARS printout and Planned Weight Manifest both indicated HAZMAT -no quantities noted on those forms. ACARS printed out PLANNED DANGEROUS GOODS SUMMARY with three (3) dangerous goods items noted which conflicted with the two (2) forms in our possession. We advised operations of the problems. We were assured that there were three items planned but only two items arrived in time to be boarded and that the final information would reflect two HAZMAT items.

After departing the blocks the FINAL DANGEROUS GOODS SUMMARY printed via ACARS showing three Dangerous Goods items. Called Zone and they assured us it was a mix up and that a corrected version was coming. We soon received another FINAL Dangerous Goods form, still showing three items. Upon return to gate discovered that three items had been loaded and that the notification form was still attached to the third package that contained 40lbs of dry ice. That package matched the information on the ACARS FINAL DANGEROUS GOODS SUMMARY. [It] appears that this HAZMAT was not handled or loaded properly. The flight crew was not notified properly. The station did all they could to encourage the crew to depart the station and potentially fly coast to coast with unknown HAZMAT on the aircraft.

Pressure to depart on-time with new minimum fuel boarded policy pushed the Flight Crew to taxi towards the runway with HAZMAT still in question. If operations was correct and we did not have the HAZMAT in question on board then we needed to get to the runway and takeoff before we consumed the allotted taxi fuel. If we delayed another 200 pounds of fuel burn we would have had to return to the gate for fuel anyway. The station could not determine the HAZMAT status so we returned to the gate and discovered HAZMAT onboard. Ultimately, the flight ran late and arrived late 31 minutes. The Flight Crew was reassigned to a different schedule due to the late flight, over 8 hour projection and minimum time layover.

Synopsis

An A320 flight crew felt pressured to depart with conflicting Hazardous Materials data.

ACN: 885613 (2815 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : APU Electrical
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : AC Generation Indicating and Warning System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13765
Experience.Flight Crew.Last 90 Days : 265
Experience.Flight Crew.Type : 3875
ASRS Report Number.Accession Number : 885613
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Release Refused / Aircraft Not Accepted
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Company Policy

Narrative: 1

While flight planning, flight crew noticed APU Generator was deferred inoperative. The original Write-up said the generator was operating normally, just the volts indication was incorrect. The sign-off said "No-Time"; so the Generator was deferred "inoperative." As we were flying at night, the crew decided the generator was a necessary piece of equipment rather than depart with it inoperative just because of a perceived lack of time to fix it, or defer correctly (which was not done in the first place as no deferral sticker was noted in cockpit). Since the Captain has been told numerous times; "if you don't refuse it, Maintenance will not address it;" a refusal was given to Dispatch.

Maintenance met in cockpit and they tested APU (it was operating normally except for the bad [voltage] indication), the deferral was amended and we departed with this major aircraft system "Operative." Maintenance had to be called back to cockpit to complete the MEL item and install a deferral sticker. It seems most maintenance stations have completely deleted this required step from their job description. 

To conclude; the "No-Time" deferral seems to be gaining popularity with air carrier maintenance personnel. This is my second occurrence in about 30 days where the "No-Time" deferral was used incorrectly (in this case taking away a major aircraft system for no reason). If a deferral was done, why not the most correct option over the quickest? It seems to me both would have taken the same amount of time, although one required a bit of thought.

Synopsis

A320 Captain noted the entire APU electrical generator was deferred instead of taking the time to just defer the APU electrical indication, reporting his air carrier's increased use of 'no-time' as a reason for deferrals of major systems. Captain refused aircraft until the correct deferral was applied.

ACN: 885452 (2816 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : DFW.Airport
State Reference : TX
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft : 1

Reference : X
ATC / Advisory.Tower : DFW
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : DFW

Aircraft : 2

Reference : Y
ATC / Advisory.Tower : DFW
Aircraft Operator : Air Carrier
Make Model Name : B767 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 50
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 885452
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were on a one mile finaL, LAHSO operation for 35C was in effect to facilitate crossing down field, with 9,000 feet of runway available. There was a B767 waiting to cross, and the Controller cleared him to cross. The Captain refused, saying he'd wait for us to land. Tower then ordered him across, so we landed with him on the same runway. While I'm fully capable of landing in the required distance, landing on the same runway where another aircraft is operating is really dumb. The risk reward here is tilted so far in the favor of risk that I can't believe I'm even filling out this report. What will you guys agree to next to save a few minutes of taxi time?

Synopsis

An 320 pilot took exception to the Local Controller coercing a B767 Captain to cross his runway while on short final during LAHSO operations.

ACN: 885394 (2817 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Multiengine
Qualification.Flight Crew : Instrument
Qualification.Flight Crew : Flight Instructor
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 23240
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 10400
ASRS Report Number.Accession Number : 885394
Human Factors : Training / Qualification
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Pre-flight

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

Due to stress caused at the beginning of the flight due to First Officer being angry and insubordinate during preflight concerning management of cabin temperature, communication and attention to detail suffered on the ensuing flight. We were right at maximum landing weight on landing and could have had a better safety margin since the flight time was 3:52 and the First Officer identified that landing weight issue after about 1:30 into the flight. I do not believe that we landed overweight but due to our Quality Assurance System data possibly showing it because of fuel quantity being incorrect at a second in time at touchdown because of being so close to the limit, I wanted to make a report. Captains need to be given more power to handle misbehaving First Officers. If First Officers that like to challenge Captain's thought they would be in real jeopardy of consequences for bad behavior which is insubordination, they would be much less likely to do it. Then the overall chain of command would be intact and function correctly thus enhancing safety margins and regulatory compliance.

Synopsis

An A320 Captain described being unable to resolve a CRM conflict with his First Officer regarding seemingly minor issues such as cabin temperature and aircraft landing weight.

ACN: 885359 (2818 of 2908)

Time / Day

Date : 201004

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 35000

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight
Ceiling.Single Value : 3000

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Qualification.Flight Crew : Flight Engineer
Qualification.Flight Crew : Flight Instructor
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 2500
ASRS Report Number.Accession Number : 885359

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

I was the flying pilot cruising at FL350 when we received a Master Caution Yellow Hydraulic quantity low. The Captain asked me to handle the radios and fly the airplane. He proceeded with the ECAM and then followed the QRH items list. We then called Dispatch and Maintenance Control. We returned to our departure airport and landed at 140,000 lbs.

Synopsis

An A320 followed ECAM procedures and returned to their departure airport following receipt of a Yellow Hydraulic System quantity low Master Caution warning.

ACN: 885031 (2819 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Fan Reverser
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 885031
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

A320 reverser sleeve was received in the shop for overhaul due to internal damage. A pre-inspection was done and a write-up documented the internal damage. Repair procedures [for excessive delaminations] are not covered in shop document or structural repair manual (SRM), so I requested an engineering order (EO) calling out specific repair procedures. Engineering came down to the shop and entered the following statement on the jobcard; Engineering recommends repair per maintenance manual (MM) 6-0-2. 

I emailed Engineering and stated that a written engineering order was also needed because of the limitations stated in Paragraph 5 of MM 6-0-2. While waiting for engineering to provide the required engineering document; the off-shift signed- off the repair and had inspection buyback. This unit now has been repaired without fully complying with the intent of MM 6-0-2 or the shop document and is soon to be released to service.

Callback: 1

Reporter stated one of their A320 engine reverser sleeve's delamination area had exceeded the approved repair limitations in their component repair manual. So they needed Engineering to write out an engineering order (EO) or variance to allow mechanics to repair the excessive skin delamination on the reverser sleeve with the same component manual repair procedures. Reporter stated that even though the reverser was already signed-off by a Mechanic and Inspector from another shift, he pulled the reverser from the serviceable parts area and had the serviceable parts tag rejected until the proper documentation was completed by engineering.

Reporter stated their engineering department has since issued an advance revision to their component manual procedures to allow repairs to reverser sleeves with excessive delaminations.

Synopsis

A Shop Mechanic reports an A320 engine reverser was received in the component shop for overhaul due to internal delamination damage. A written engineering order to approve repair of the excessive delaminated area was not issued prior to an off-shift Mechanic and Inspector signing for the serviceability of the reverser.

ACN: 884680 (2820 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Aircraft Cooling System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Company Operations Manual
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 884680
Human Factors : Time Pressure
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

Upon accessing aircraft cockpit discovered aircraft in faulted status. Had deferral on the A320 aircraft that required maintenance to check avionics cooling and issue a new maintenance release prior to every flight. Station was unaware of the problem and had powered the aircraft and left it in an avionics fault state. Maintenance Control had not coordinated for maintenance locally. ZZZ station had not coordinated for local maintenance. Local maintenance was notified and they worked with Maintenance Control to clear the issue and allow a new maintenance release to be issued.

The avionics cooling system FAILED the first maintenance reset and had to be reset again. During that reset another system failed and maintenance had to reset it as well. Finally allowed complete access to our workplace and departed, I believe, one minute late. Checked with Dispatch after one hour of flight and NO delay code had been assigned. When I reported for my next days flight I checked and discovered that a cockpit checklist DELAY had been assigned to our flight. Checked with an assistant Chief Pilot and made four phone calls on my own and discovered that Maintenance Control is TEFLON. They have an AUTOMATIC software program that REJECTS DELAY CODES assigned to Maintenance. The delay code is placed on the station in this case. ZZZ Station does not have a Company domicile oversight. ZZZ Station decided to assign the delay to the [flight] crew. This problem happened over a weekend so the delay code remained and was not changeable.

Pilots are under scrutiny for delays. My flight was AUTOMATICALLY charged with a delay. This situation is unacceptable. How can we be assigned a DELAY BEHIND our backs - especially when this delay was clearly a FAILURE on maintenance and Maintenance Control's part? The pilot group needs the same automatic rejection software that Maintenance Control has implemented to level the playing field. I am just thankful that while these games were being played that the avionics did not suffer heat damage and have to be replaced.

Callback: 1

Reporter stated that in the past his Air Carrier's practice of assigning unearned delays to an outbound flight crew was quite common. Most pilots would not challenge the unearned delays. Today however, with the increase emphasis on departure times, pilots are tracked for refusing an aircraft or questioning an MEL deferral that results in a delay.

Reporter stated that in his report he was charged with a cockpit checklist delay that was clearly a maintenance issue. Reporter further stated, pilots efforts to correct the delay assignments have not been very successful.

Synopsis

An A320 Captain reports on his company's practice of assigning departure delays to flight crews when many delays are clearly a maintenance issue.

ACN: 884493 (2821 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0001-0600

Place

Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 884493
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Other

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Detector.Person : Maintenance
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Primary Problem : Aircraft

Narrative: 1

En route I got papers for next leg. Captain's rudder pedal disconnect inoperative. I refused the plane and ACARS'ed Dispatch. After landing I got call from a Flight Manager who met me in Operations and asked multiple questions about my system knowledge and why I thought the plane would be unsafe. He said his opinion was the rudder pedal disconnect only sends signal to disconnect and nothing else. I said stray signals could affect the tiller and other systems. I called Maintenance Control and talked to a Mechanic. He looked in 320 manual and said all these signals go to the BSCU which is the brains. I asked if its possible that a problem (i.e. signal) with pedal disconnect could affect tiller or other systems on the BSCU. He said YES!!! End of story.

Synopsis

An A320 Captain refused an aircraft with a rudder pedal disconnect discrepancy because other components which run through the BSCU could also be affected such as nose wheel steering.

ACN: 884294 (2822 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC
Light : Dawn

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Flight Phase : Climb
Airspace.Class B : ZZZ

Component

Aircraft Component : Gear Extend/Retract Mechanism
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 884294
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Workload
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Landing gear failed to retract after take-off. ECAM message for "gear not uplocked" and "landing gear doors open", also "eng 2 reverser fault". I asked ATC for a level off at 3000 ft and for an extended right down wind and told ATC that we will need to return to the airport. The First Officer placed the gear handle in the down position. We then went through the QRH and non-normal supplemental book. Following the non-normal supplemental book for the ECAM messages we recycled the landing and this time the gear retracted. The gear indications were back to normal. I then decided not to return to the departure airport but to continue on to our destination since the gear was up and no fault messages were present. I felt it was safer to continue on because I would be landing 9000 lbs over weight if we landed at our departure airport. After reaching cruise I talked to Dispatch and Maintenance Control, they agreed that continuing was the best course of action to take. Landed with no problems. Right main landing gear sensor failed momentarily.

Synopsis

An A320 Captain reported that after takeoff the landing gear did not retract and a reverser unlock ECAM alerted. After completing the QRH and Supplemental procedures the landing gear retracted and crew continued to its destination.

ACN: 884241 (2823 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Mission : Ferry / Re-Positioning
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : N
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Fan Blade
Manufacturer : International Aero Engines IAE
Aircraft Reference : X

Component : 2

Aircraft Component : Aircraft Logbook(s)
Manufacturer : Airbus
Aircraft Reference : X

Component : 3

Aircraft Component : Aircraft Documentation
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Other / Unknown
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Avionics
Qualification.Maintenance : Airframe
Experience.Maintenance.Inspector : 2
Experience.Maintenance.Lead Technician : 2
Experience.Maintenance.Technician : 12
ASRS Report Number.Accession Number : 884241
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Other Person
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

An A320 aircraft experienced a bird strike to the number 1 Engine in ZZZ, resulting in two blades having minor bends. On-call [Contract] Maintenance accomplished an engine inspection jobcard. The flight crew reported no engine parameter shifts. Per the jobcard, an engine boroscope was not required at this time. Per the procedures manual, a maintenance ferry is a procedure where I fill out a permit to let the aircraft fly non-revenue to a base for repair. I spoke to the Captain, the Dispatcher, Maintenance Control, the Airline Operations Manager, and Engineering.

I discussed with all parties the intent to ferry the aircraft to ZZZ1 for blade replacement and engine boroscope. The situation is under review due to the fact the damage was not documented properly in the logbook and the jobcard was not filled out correctly. Also, the engine run was not documented, although Maintenance Control had the On-call Mechanic run the engine. Also, the Crew encountered the bird on approach, the engine did not suffer parameter shifts. I spoke to the Captain directly. He had no issues with the maintenance ferry.

In the future, I will ensure more documentation in the logbook and follow the maintenance procedures more closely. We also have a new procedure to put a copy of Engineering's "OK to Ferry" in our maintenance ferry logbook. I forgot to copy the e-mail out that I received from Engine Engineering, but I did call and received a verbal "OK." I have since maintenance ferried aircraft with Engineering concurrence and have saved the e-mails as required.

Callback: 1

Reporter stated the changes made in their procedures to better document the maintenance work has helped.

Synopsis

A Maintenance Control Manager who signed for an A320 aircraft maintenance ferry due to a bird strike, reports about the blade damage not properly documented in the logbook and a fan blade engine inspection jobcard not filled out correctly.

ACN: 884158 (2824 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch

Component

Aircraft Component : Indicating and Warning - Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 19000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 700
ASRS Report Number.Accession Number : 884158
Human Factors : Time Pressure
Human Factors : Human-Machine Interface
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 182
Experience.Flight Crew.Type : 670
ASRS Report Number.Accession Number : 884687

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the cockpit setup I observed the right brake temps over 450 degrees with the left wheels in the mid 300s. By the time we pushed back, the right wheels were in the mid 200s and the left wheels were in the mid-100s. Taxi clearance was received for Runway 25, a generally downhill route from the gate. Because of the temperature split between the right and left trucks, I elected to use minimal braking with the left gear only, allowing the right truck to cool further on the way to the runway. We had a short delay in receiving final weights necessitating a period of time with the parking brake set just prior to beginning the takeoff roll. During takeoff at approximately 70 KIAS the Hot Brakes ECAM and Caution sounded indicating that one of the left wheels was over 300 degrees. This did not make sense, as just prior to taking the runway both brake temps on the left side were well below 200 degrees. The First Officer was pilot flying and I advised him to continue the takeoff. The power was never reduced, nor were the brakes applied. As a precaution, once safely airborne we completed the "Hot Brakes" ECAM procedure. Point one: Prior to my flight day, I read through the maintenance records of this aircraft and one other. When seeing the above ECAM during takeoff, I incorrectly remembered the maintenance history indicating the exact same brake temperature event during a takeoff by another crew. The mistake: This event occurred in the other aircraft, not ours. Point two: While the rapid rise in left wheel temps during the start of the takeoff roll did not seem to be likely, or even possible, I feel that I should have relied upon the takeoff safety briefing (as given by me) and aborted the takeoff in the low-speed regime. Point three: Contract Maintenance was called out to check the left wheel assembly and a loose connection was found, and the brake temperature indicating system was deferred.

Synopsis

An A320 Captain continued the takeoff from a high altitude airport despite an ECAM HOT BRAKES warning received below 80K.

ACN: 883827 (2825 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 3874
ASRS Report Number.Accession Number : 883827
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 9500
ASRS Report Number.Accession Number : 883833
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We where at cruse when the Flight Attendant called and said she could hear a noise. We could also hear it and had her check the ovens and cooler. About the same time we started to smell a burning plastic odor. The Flight Attendant in the back called and said they were smelling an odor also. The Captain got a call off to Dispatch and Maintenance. The odor was getting worse so I asked the Captain if he wanted me to run the QRH. We put on our O2 mask, and ran it. When I put the two fan switches in OVRD, the noise went away. I lifted my mask to see if the smell was going a way and it was getting better but still there. We then started to divert and declare an emergency. Landed with out any further incident and the odor was gone.

Narrative: 2

Thinking it was the chiller, I confirmed it was off as well as the ovens. Shortly after. Sent the First Officer back to investigate. When the First Officer returned we also started smelling smoke in the cockpit, also got a call from the aft flight attendants that they also smelled smoke in the back. After donning our oxygen masks, the First Officer did the smoke in the cockpit QRH and as I still had Dispatch on the frequency I coordinated our diversion airport as it was approximately 60 miles behind us. Told First Officer to declare an emergency and get us headed to the divert airport as I called the flight attendants and advised them of our intentions and would be landing in about 10 minutes and to expect a normal landing. Made a PA announcement. After landing and exiting the runway the emergency vehicles saw no sign of smoke and as we no longer smelled anything we taxied to the gate. Shut down checklist accomplished and passengers disembarked.

Synopsis

An A320 crew reported a burning plastic odor in the aircraft at cruise. An emergency was declared, the QRH checklist completed and the aircraft diverted to a nearby airport. The odor and noise decreased when OVRD was selected for the two fan switches.

ACN: 883825 (2826 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10164
Experience.Flight Crew.Last 90 Days : 115
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 883825
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Workload

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Attendant
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise, a Flight Attendant reported a loud pop and then smelled electrical smoke. She appeared very nevous. A passenger in first class also smelled the acrid odor. We declared an emergency, ran the smoke QRH followed by the cabin smoke checklist. We contacted Dispatch and coordinated a nearby airport as a divert airport. We diverted without further event.

Synopsis

An A320 First Class Flight Attendant reported a loud pop and an acrid electrical smell at cruise. The Captain declared an emergency and diverted to a nearby airport.

ACN: 883819 (2827 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : Marginal
Light : Dawn

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Design

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 700
ASRS Report Number.Accession Number : 883819
Human Factors : Situational Awareness
Human Factors : Time Pressure
Human Factors : Training / Qualification
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Company Policy

Narrative: 1

The First Officer, non flying pilot, I arrived at the airplane 45 minutes prior to departure. Completed the Exterior Preflight and started the Flight Deck preflight. The FMS had been changed to the New FMS. The day prior, really, I had reviewed the FMS "Briefing pages". I had read the first few pages thoroughly then started to get confused because the pictures shown in the briefing stated that they did not match the box installed. I assumed it would be intuitive. Having flown the A320 a year, I am not a "wiz" with the box, however, certainly comfortable. This is when the preflight goes from typical to very, uncomfortable. The Captain arrives with the paperwork and we get to work together on the FMS. Numerous interruptions; Customer Service, Maintenance, Fuel....typical, but the Captain and I are working on what has become a very involved, confusing and frustrating preflight preparation, because I certainly and I assume the Captain as well, felt very unprepared to program this FMS. With the clock running, interruptions, and the importance of having it done correctly the first time, was very unnerving. I have never felt less prepared for a flight at this Airline in my nearly 13 years. Not confidence inspiring for anyone. At the very least a software program for people like me to work with prior to going live would have been helpful. I am sure with a little practice it will go fine next time. But being in the airplane this time, the first time, and being that uncomfortable is simply intolerable.

Synopsis

An A320 First Officer reporter that his Company changed the FMGC system and gave crews a "Briefing Paper" to prepare them for the change. The Reporter and his Captain had a great difficulty programming the new system for the first time.

ACN: 883330 (2828 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Initial Climb
Flight Phase : Climb

Component

Aircraft Component : FADEC / TCC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 883330

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

[On] initial climb noticed aircraft vibration followed by an ECAM message ENG1 FADEC B fault followed by ENG1 sensor fault. ECAM action and follow up with supplemental manual was followed and departure was advised of a possible return to airport. We consulted with maintenance control and decided a return to land was warranted. A message was sent to dispatch concerning our situation and intent. Since this was not an emergency situation, we asked for vectors to burn fuel so that an overweight landing could be avoided. Flight Attendants and passengers were kept advised of our situation and a normal and uneventful landing was made. The aircraft was release to maintenance.

Synopsis

An A320 Captain reported noticing vibration on climb, followed by ECAM ENG1 FADEC B. After consulting with maintenance, they returned to departure airport.

ACN: 883146 (2829 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap/Slat Control System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 102
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 883146
Human Factors : Distraction
Human Factors : Confusion
Human Factors : Time Pressure

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was the non flying pilot, the weather was VFR upon our arrival. We were cleared for a visual approach. The Captain who was the flying pilot was flying a very fast approach. He was maintaining 250 KTS until approx 2.5 miles from the outer marker. He began configuring by calling for gear just below 250 KTS. At 210 KTS he called for flaps 1 shortly thereafter we received a Flight Control Slats Locked ECAM. I was starting to analyze the ECAM message, and first noted a maximum speed message which I read aloud. During this process the Captain continued to call for flaps as he was trying to configure the airplane for landing and trying to maintain stabilized approach criteria. I also was trying to finish the landing checklist to comply with stabilized approach criteria. I double checked we had the correct speed for the next flap setting and complied with his command to set flaps. I was momentarily distracted by his approach speed and wondering if we were going to be stabilized. When I returned to the ECAM, I realized we had a slat issue and was beginning to brief the Captain on what we had so far. The Captain continued the approach and was still calling for flaps. Once again my attention was diverted from the ECAM. By the time I set the final flaps, and returned to ECAM we were on short final. I looked at the airspeed indicator and realized VLS was indicating higher than normal, I believe I said something to the extent that something seems not correct and that we have a slat problem. The Captain said he agreed, but decided to continue the approach. Our airspeed at this time was still considerably higher than normal. The Captain maintained this higher airspeed until touchdown. The touchdown and rollout were normal. I did not feel it was prudent to have my head buried in the ECAM on short final, and since the airplane appeared to be flying normally I focused my attention on landing and abandoned the ECAM. I also felt the extra airspeed was prudent at the time given the fact we did have a slat malfunction. After landing and clearing the runway, I finally had a chance to perform the rest of the ECAM procedure. It was then clear we had a slat malfunction. I realized immediately we should have abandoned the approach and gone around in order to troubleshoot the problem. I attribute the failure to properly carry out ECAM procedures to two major issues. The first was the Captain's unconventional high speed approach. Second, was my distraction in trying to comply with the Captain's commands to configure the airplane. I was also distracted while trying to ensure the Captain was flying a stabilized approach. Additionally, I only was able to see the first page of ECAM which listed a maximum speed, each time I set flaps the ECAM page changed which meant I had to reinterpret the new page and subsequently was unable to move to additional ECAM pages. I disagree with flying high speed approaches in this manner. Even if the approach is stabilized it creates a huge distraction for the non flying pilot. Things were happening fast enough during the approach, that I never really had time to complete ECAM and to analyze what the extent of the problem was. I feel quite comfortable in the airplane, but even so I simply did not have enough time to complete the procedure. I also did not have enough information on what the failure was at the time to call for the Captain to abandon the approach. Obviously there are several rather important considerations associated with this particular ECAM. Once the Captain realized as well we had some type of malfunction, he used prudent judgment and increased approach speed to a margin well above VLS.

Synopsis

An A320 First Officer reported noticing F/CTRL Slats Locked ECAM on approach, but the workload imposed by the high speed approach led the crew to ignore the message and continue to a landing.

ACN: 883088 (2830 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Pressurization Control System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 13
ASRS Report Number.Accession Number : 883088
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party1 : Other
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Chart Or Publication
Primary Problem : Chart Or Publication

Narrative: 1

An A320 aircraft lost cabin pressure en route from ZZZ to ZZZ1. The aircraft returned to ZZZ where it was discovered that the #2 cabin pressure controller had shipping cap installed on the pressure port. I was assigned to work on cabin pressure controller #2 box two days earlier. Upon installation I thought that [cap] was a dust cover; I left cover on the Port. 

All work done per Maintenance Manual (MM) 21-31-00-710. It was honest mistake, never thought that cover has to be removed because it passed the Ground Operations Check. There was not any Caution tag on the box; but it was in AMM. I thought the AMM note was referring to the electrical pins cover on the back of the box. It was clearly my mistake that I did not read the whole paragraph. I learned my lesson from this mistake.

Callback: 1

Reporter stated the #2 Cabin Pressure Controller he replaced was a borrowed part from a different airline. He had originally removed the dust cover on the pressure port but decided to re-install the cap because he wasn't sure if the cap was suppose to stay on or be removed. He figured if the operational test failed than he would remove the cap and perform the test again. He later noted after the air turnback, their in-house Controllers had a Caution tag attached to the pressure port dust cover; the borrowed controller did not have a Caution tag. 

Reporter stated the Airbus procedures use the the A320's on-board Multipurpose Control and Display Unit (MCDU) to perform the operational test of both #1 and #2 pressure controllers. The static ports are not plugged on the A320s like they are on a Boeing 757 aircraft when verifying a cabin pressure controller function. Both of the A320's controllers passed the operational check, so he decided the dust cover cap he was concerned about must be OK to stay on the port. All of his work was verified by another Mechanic who signed-off on his work as an Inspector.

Reporter stated the A320 flew one leg from his station to ZZZ1 without any incident using the #1 pressure controller. On for the return leg back to his station, the Airbus had automatically switched over to the #2 pressure controller and during climbout all the passenger oxygen masks had dropped due to cabin pressurization failure. He and three other Mechanics had to replace all of the passenger oxygen generators and re-install all of the oxygen masks.

Reporter stated that after the air turnback, both he and the other Mechanic who signed as an Inspector and his Supervisor went over to another A320, performed the MCDU Cabin Pressure Controller Check without the dust cap on the pressure port and then with the dust cap installed; the results were the same. Both operational tests passed without any faults noted.

Reporter stated the job card is not being revised, even though the procedure will not indicate the pressure port is not functional and a fault does not appear, even with the dust cap on the port.

Synopsis

A Line Mechanic reports about an A320 air turnback due to a #2 cabin pressure controller failure. Mechanic had previously replaced the #2 controller with a borrowed part from another carrier. Both of the A320's pressure controllers then passed the A320's on-board MCDU operational test without any faults noted even though the dust cover was still on the #2 controller pressure port.

ACN: 882999 (2831 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : MWCR.Airport
State Reference : FO
Altitude.MSL.Single Value : 3000

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Personal
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Initial Approach

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 882999
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Primary Problem : Aircraft

Narrative: 1

On approach to MWCR we selected flaps 2. We then got an ECAM, F/CTL (flight controls) flaps locked. We broke off the approach to hold south of the airport to run the ECAM and the QRH. The flaps locked somewhere between up and 1. We could not get them up or down. After looking at landing performance with the First Officer we initially came up with a distance of about 7000 feet. We were just under max landing weight. The runway at MWCR is 7008 feet. With the flaps locked at about 1 we knew that we would not be able to divert back to the US. After conferring with the First Officer and the Dispatcher, I felt the runway that provided the largest safety margin possible was MKJS, where the runway is 8735 feet. Fuel was not a problem as we were tankering for the flight back to the US. The landing was made at MKJS without any further problems.

Synopsis

A320 Captain reports flaps locked during approach to MWCR. Attempts to remedy the situation are unsuccessful and with the flaps less than 1 the crew elected to divert to an airport with a longer runway.

ACN: 882992 (2832 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 30000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : EICAS/EAD/ECAM
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13875
Experience.Flight Crew.Last 90 Days : 265
Experience.Flight Crew.Type : 3845
ASRS Report Number.Accession Number : 882992
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While climbing through FL300 ECAM displayed a message and associated chime. By the time both pilots attention was brought to the ECAM screen the message had disappeared. Something in regard to ENG2 had been observed but the rest of the script disappeared too quickly to be read. A "RCL" [recall] was attempted and the message could not be brought up. The Captain brought up CFDS (centralized fault display system) function in his MCDU (multi-function control and display unit) and a "CURRENT LEG ECAM REPORT" was requested from button 2L. At this time the ENG2 Reverse Unlocked was noted in the history as the ECAM we had seen. Although brief, this ECAM caused concern to the crew as it is a fairly significant flight event to have a reverser unlock or un-stow. An emergency phone patch was requested from dispatch and Maintenance Control. Maintenance Control could not definitively give us a cause or an in-flight reset of this fault but it caused us enough concern to return to our departure airport. As the fault did not re-appear, we did not run the IRREG Checklist but we briefed it and had it ready to execute if needed. We also decided the LAND ASAP section of the IRREG checklist could be belayed as long as the fault did not come back. We did have multiple diversion airports available. 

We declared an emergency with ATC to get priority routing back to the airport. We landed without further incident. AMTs (Aircraft Maintenance Technicians) there informed us the ECAM fault had a history over the past few days but since it was such a brief fault we were the first to report it. An initial verbal report from the AMTs indicated "probably" a wiring fault. The aircraft was removed from service for repair. 

I am very troubled we took off in an aircraft with a potentially catastrophic fault. As the AMTs noted, this fault had been occurring for a few days. The jet knew it was faulting; it appears the company has no way to cull this information prior to an actual fault event. I would strongly recommend we utilize the technology we have available and have a qualified AMT take a reading from the CFDS (Centralized Fault Display System) computer between every leg. I believe this can be done remotely. ALSO, systems knowledge allowed the crew to access the CFDS system and extract much needed safety information for this flight (i.e; what was that ECAM we couldn't recall?). The AOM is fairly limited with information on this valuable informational tool and the crew recommends a more comprehensive description of the CFDS system and its capabilities be included in our training and the AOM. Without the information we could have continued with a very insidious but serious fault that could have manifested into a serious incident.

Synopsis

A briefly illuminated EICAS message was eventually determined to have been an ENG2 Reverse Unlock advisory after a review of the CURRENT LEG ECAM REPORT on the CFDS page. The A320 Flight Crew then opted to declare an emergency and return to their departure airport for maintenance.

ACN: 882975 (2833 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cockpit Window
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 882975
Human Factors : Distraction
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 7500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 2720
ASRS Report Number.Accession Number : 882993
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

While at a cruise altitude of 36000", and approximately half way through our flight, my First Officer noticed a crack in his forward window. As we examined it, it was continuing to increase in length. It then started to create an air tunnel in which there seemed to be fluid. About this time we noticed that it started to develop a small (maybe an inch) electrical arch, and then we heard a loud bang, and the outer pane of the window shattered. This all happened very quickly, and had it not been for the keen eyes of the First Officer, we would have had no idea why this happened. We contacted Dispatch and had them conference in Maintenance Control. After discussion with our Maintenance Control and referencing our flight manual, we determined that it was indeed the outer pane that had shattered, and that there was only one flight restriction which would not be prohibitive to continuing the flight to our destination. Upon further consultation with Dispatch and Maintenance, it was agreed that all parties involved were comfortable with the flight still being able to be conducted in a safe manner, and that no need to deviate was necessary. All SOPs and flight manual procedures followed. The rest of the flight was uneventful and a normal landing was made at our destination.

Narrative: 2

After referring to the Flight Manual we descended to FL220, slowed down, brought the differential pressure to less than 5, and made sure the windshield heat was off on the First Officer side. After consulting with Maintenance and Dispatch we continued to our destination. Landing was uneventful and we debriefed Maintenance.

Synopsis

An A320 First Officer's front windshield outer pane cracked while cruising at FL360. The outer layer was non structural so the flight slowed and descended, as per the QRH, then continued to its destination.

ACN: 882717 (2834 of 2908)

Time / Day

Date : 201004

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Flight Phase : Takeoff / Launch

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
Qualification.Flight Attendant : Current
ASRS Report Number.Accession Number : 882717
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Workload
Communication Breakdown.Party1 : Flight Attendant
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Flight Attendant
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

We were taxing out to the runway and the cockpit said to us "flight attendants prepare for takeoff". The safety video had not ended and we had not gone out to do our cabin safety checks. I called the Purser and told him to call the cockpit and tell them that we had not done our cabin safety checks. I went out into the aisle to do my checks. As I worked my way back there was a man with a large briefcase on his lap. I repeatedly told him to stow his briefcase. Just then I hear the engines rev and we started to roll down the runway. I was still standing in the aisle. I ran back to my jumpseat and barely got to sit down and put my seatbelt and shoulder harness on before we lifted off. I waited till 18,000 FT and then called the Purser. I asked him "Did you call the cockpit and tell them that we were in the aisle doing our cabin safety checks". He said "Yes and they said we got to go, we got to go". I then waited until the seatbelt sign went off and the cockpit finished their announcement to the passengers and called them. The First Officer answered. I asked him: Did the Purser call you and tell you that we were in the aisle doing our cabin safety checks because I was still in the aisle when you took off". He said to me "The Purser said give me two seconds. We were on the runway and if we would have pulled off then we would have had to wait 30 minutes". I again told him "Well I was still in the aisle when you guys took off. Thank you I just needed to check what had happened."

Synopsis

An A320 crew took off with a flight attendant in the aisle performing the cabin safety check.

ACN: 882581 (2835 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 26000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Turbulence

Aircraft

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : BULLZ ONE
Airspace.Class A : ZAU

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 8800
ASRS Report Number.Accession Number : 882581

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 30
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 882579

Events

Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Weather
Primary Problem : Weather

Narrative: 1

This report addresses an aircraft encounter with severe turbulence. The event occurred in clear air during descent into ORD. Recent convective activity and a strong frontal gradient were in the area during the descent. There were no reported injuries as a result of the incident. The crew was briefed on the weather and ride situation into ORD. We were familiar with each other as we had flown together for 3 legs over two days, and had left ORD the previous evening during convective activity. The crew was prepared and anticipated turbulence, but severe turbulence was not forecast or reported recently. We had a lengthy discussion about spring weather and preparedness, especially since we had a rough ride leaving ORD the night before. I believe the briefing, preparation and awareness of the crew was instrumental in avoiding injury during this incident. The aircraft was flying to JVL VOR to intercept the BULLZ1 arrival and beginning the initial descent. There was no recent report of turbulence in the area, though we were wondering when it would begin. We saw clouds in front of us while in the descent through FL260. This is when we anticipated the turbulence would begin. As [we] continued our descent, the airspeed began to increase inside the limits of the managed descent bracket, approaching Mmo/Vmo. The autopilot was disengaged to prevent an overspeed, and the descent rate was arrested from the 6000 FPM range. The aircraft began to slow, and we quickly discussed getting the flight attendants down when the first wave of turbulence hit. The plane encountered moderate turbulence for about 15 seconds, and the seatbelt sign was turned on and the flight attendants were told to be seated. After a break of about 30 seconds, we began to encounter severe turbulence passing FL220 and continuing down to 17,000 FT. The autopilot, which had since been selected back on, was turned off again, as the slow response seemed to make things worse. The aircraft was marginally controllable at times with VVI variations between +2500 to -6000 FPM, +/- 20 to 30 KTS airspeed variation, and at one point had idle thrust, full speed brakes and a +1000 FPM VVI. We had been attempting to descend through the turbulence and slow to the TAPS (Turbulence Auto-PIREP System) airspeed during that event. There was un-commanded roll and pitch which was violent, but within normal aircraft pitch and roll limits. The aircraft left the turbulence at about 16,000 FT. We noticed that the wind vector had shifted from 250/115 at FL260 to 170/65 at 17,000. At the end of the incident, the Lead Flight Attendant was called to see if anyone was injured. She had told us no one was hurt and that the two of the flight attendants were in their jumpseats and one stayed on the floor. The flight attendants then prepared the cabin for landing, and after a short lull, the remainder of the flight concluded in light to moderate chop. 

During the incident, Chicago Center Controllers seemed to be in disbelief as to the severity of the turbulence. They continued to give instructions and crossing restrictions even after receiving information that our control of the aircraft was diminished due to the severe turbulence. They were reluctant to inform other aircraft of the severe turbulence. I was disappointed with the performance of the typically elite controllers in the Chicago Center. With two jumpseating pilots in the cockpit, everyone agreed this was unquestionably severe turbulence and for the majority, the worse clear air turbulence encountered. Chicago Center seemed unresponsive in this situation, and though we can all get reluctant to use the phrase "severe turbulence", this doesn't appear to be a mis-characterization of the situation. We listened and it seemed that other aircraft followed us through the area. After landing, we taxied to the gate, informed Maintenance, and conducted a crew debrief. Upset passengers had to wait 20 minutes for a jetway so they could de-plane, and people were again asked if anyone was injured. No one indicated any injuries. Though this was an unfortunate and unexpected clear air turbulence encounter, the preparation and training of the crew played an important role in avoiding serious injury. We had expected some turbulence in the descent, but associated with the front and convective activity, not the clear air shearing that was encountered. Continued contact and updates from Dispatch and especially Chicago Center could have helped to mitigate the circumstances even further. Nonetheless, for our situation, we are fortunate that the outcome was positive.

Synopsis

An A320 flight crew reported encountering severe turbulence on descent into ORD.

ACN: 882459 (2836 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Cabin Entertainment
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Oxygen System/Pax
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Location In Aircraft : General Seating Area
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Avionics
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 882459
Human Factors : Confusion
Human Factors : Situational Awareness
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Passenger
Were Passengers Involved In Event : Y
When Detected : In-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

In reference to a A320 aircraft. I installed the Video Control Unit (VSDU) per Maintenance Manual (MM) 23-36-97-401. The unit had previously been replaced twice during the visit. I performed the Operational Test of the Passenger Entertainment (Video) per MM 23-36-00-710. I did not pull the Oxygen 02 Circuit Breakers (CB)'s or "Arm" any O2 mask container since I did not believe this to be applicable to the Task. 

In addition, I did not see any mention in Maintenance computer records that documented that the O2 masks had been previously "Armed." This aircraft visit was using the hangar paper package and maintenance computer for documenting write-ups. This caused confusion between the Maintenance Ramp Crew and the Hangar Crew. This method of mixing methods has caused confusion in the past and is a ongoing problem.

Callback: 1

Reporter stated the A320 had been in the hangar for a landing gear and aft lavatory corrosion maintenance visit. The Video System Control Unit (VSDU) had been robbed and replaced three times during the aircraft's visit. The VSDU he replaced was a newer version that uses a solid state hard drive for storing movies instead of the older model video entertainment unit that used a tape for showing movies.

Reporter stated that although the VSDU unit he was going to do an operational test on was not the same unit he had previously replaced, but actually the third VSDU to be installed, he was following a hangar paper write-up that stated an operational test of the Video System Control Unit was required. When he pulled-up the Airbus Maintenance Manual Task he noticed there were approximately eighty pages to review, including numerous Engineering Orders that needed to be verified. 

Reporter stated about thirty-five to forty pages into the VSDU Operational Test Task procedure, he noticed a reference to a Passenger Oxygen (O2) System Sub-Task that was also called out. He thought that Oxygen Test Task could not possibly be part of the Video Operational Test. So he skipped over the lines that directed Mechanics to pull Circuit Breakers (C/B)'s for the Passenger Oxygen System and to "Arm" each Passenger Service Unit (PSU) to allow the passenger oxygen masks panel to deploy slightly. The arming procedure requires either a manual extension of a small red tab, or the turning of a screw that twists like a dime with part of it red. Either method is designed to "catch" the O2 mask panel and allows simulation of a mask drop without actually dropping the masks completely out of the passenger service units. The aircraft he worked had the newer screw type drop catch. 

Reporter stated he works the Ramp as an Avionics Technician and what he did not know at the time was the previous shift hangar Mechanic had already pulled the Passenger Oxygen CB and "Armed" each passenger row's PSU unit. The hangar Mechanic had informed his Supervisor and Lead of the situation but did not reference anything on the Write-up about the CB's being pulled or the "Armed" PSU's.

Reporter stated he completed the VSDU Operational Test and the aircraft was released. Three revenue flights later, the A320 was pulled out of service because someone in the cabin questioned why all the Passenger Service Units were hanging down slightly and a little RED indicator could be seen when looking up from a passenger seat. In that condition, in an Emergency situation, the passenger oxygen masks could not fully deploy and passengers would not have use of their oxygen masks.

Reporter also stated he later learned the Video System Control Unit (VSDU) Check does include and require the passenger Oxygen masks Drop Check. That's because if the Video screens are deployed and a Cabin emergency occurs such as a sudden de-pressurization, the video screens are suppose to retract when the Oxygen masks are dropped. His Carrier has since revised the Maintenance procedure at the beginning of the VSDU Operational Test that specifically notes the Passenger Oxygen mask drop Test must be accomplished whenever the VSDU's are removed and replaced. But they have not addressed the continuing problem with using two different Maintenance recording systems, paper or computer, for their different Maintenance groups.

Synopsis

A Ramp Avionics Technician reports about the events that contributed to an A320 being flown on three revenue flights with the Passenger Service Units (PSU) Oxygen mask panels partially opened in the maintenance test position; preventing full deployment and use of the oxygen masks in a cabin emergency situation.

ACN: 882404 (2837 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Relative Position.Distance.Nautical Miles : 80
Altitude.MSL.Single Value : 31000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 882404
Analyst Callback : Attempted

Events

Anomaly.Inflight Event / Encounter : Other / Unknown
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On climbout during a ferry flight out of ZZZ where heavy maintenance had been performed, at FL310, 240kts, Mach 0.70 the aircraft began a significant shudder not associated with anything obvious. We coordinated for an immediate level off and a lower altitude. Shuddering continued until approximately FL290 whereat we elected to slowly accelerate to almost 290K when the shuddering began again. We continued to lower altitudes, slowed down and eventually ended up at FL230 and approximately 260K. Shuddering was intermittent and less intense, but had no logic between airspeeds and altitudes. We discussed continuing to our destination until we determined this was periodic shuddering and could not be avoided. We coordinated with Dispatch and Maintenance Control and elected the best course of action was a return to the maintenance base at our departure station. We performed exterior visual inspections, analyzed all systems and found no anomalies. We coordinated for a controllability check in accordance with the manual on our return. Although shuddering was still intermittent and lasted less than a minute and was spaced more than 3-5 minutes apart, no issues were found with controllability. We landed uneventfully although the Captain noticed minor strange "looseness" and nosewheel shimmy. Aircraft did yaw once to the left when we lowered gear, but at no time did we feel any problems with total control of aircraft. A thorough debrief with maintenance was conducted.

Synopsis

An unexplained recurring shudder in the airframe caused an A320 Flight Crew to return to their departure airport where the aircraft had just undergone heavy maintenance.

ACN: 882009 (2838 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : MSP.Airport
State Reference : MN

Environment

Weather Elements / Visibility : Fog

Aircraft : 1

Reference : X
ATC / Advisory.TRACON : M98
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Final Approach
Route In Use.Other 
Airspace.Class B : MSP

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : M98
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet CL65, Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use.Other 
Airspace.Class B : MSP

Person : 1

Reference : 1
Location Of Person.Facility : M98.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Trainee
ASRS Report Number.Accession Number : 882009
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : M98.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Function.Air Traffic Control : Instructor
ASRS Report Number.Accession Number : 882530

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

MSP weather: 32007 KTS 2 1/2nm -DZBR OVC 006; strong northeast winds at altitudes above 7,000. I was on OJT (on the job training) at North Approach (N) doing ILS 30 L/R approaches to MSP. Air Carrier X was cleared for the ILS 30R approach with a speed of 170 KTS to JACKO (FAF) followed by Air Carrier Y descending to 4,000 from 7,000 with a speed restriction of 170 KTS indicating 233 KTS. Air Carrier Y was given a 280 heading to intercept the 30R localizer, instructed to cross SAMMZ at 4,000, cleared ILS Runway 30R approach maintain 170 KTS to JACKO (FAF). Upon getting established on the localizer, Air Carrier Y's speed was still 100 KTS greater than Air Carrier X ahead and was issued a speed reduction to 150 KTS to JACKO. Just prior to shipping the aircraft to MSP, I instructed Air Carrier Y to reduce to final approach speed because the aircraft's speed was still 100 KTS faster than Air Carrier X ahead and contact MSP. Per RADAR replay, separation was lost as Air Carrier Y began crossing JACKO with a 90 KT overtake behind Air Carrier X. The MSP Controller on initial contact reduced Air Carrier Y to final approach speed and informed Air Carrier Y that they were 90 KTS faster than Air Carrier X ahead. The MSP Controller made a different transmission then returned to Air Carrier Y and canceled his approach clearance upon noticing that Air Carrier Y was not slowing. Air Carrier Y did not respond. The MSP Controller had to make three transmissions to Air Carrier Y to cancel their approach clearance and issue missed approach instructions. Recommendation, data blocks that turn, say yellow, similar to TARP when separation may be lost if actions are not taken and then data blocks that turn red when separation is lost.

Narrative: 2

I was conducting OJT on the north arrival position at M98. The Developmental issued Air Carrier X approach clearance and 170 KTS to FAF, Behind Air Carrier X was Air Carrier Y the aircraft was slowed initially to 210 KTS, when the aircraft was approximately 6 miles in trail the Developmental cleared and slowed the aircraft to 170 KTS to FAF. The aircraft didn't slow as expected and separation was lost as Tower cancelled the aircraft approach clearance. I was providing instruction to the Developmental and did not catch the overtake in a timely manner. After reviewing the tape, it appeared that Air Carrier Y did not slow as quickly as all the other aircraft, and it was not caught quick enough. Recommendation, I believe I need to pay closer attention while performing OJT. And to my Developmental, and to not be afraid to take quick and decisive action when an overtake situation is occurring.

Synopsis

An M98 Controller receiving training experienced an over take loss of separation event with two arrival aircraft on final.

ACN: 881646 (2839 of 2908)

Time / Day

Date : 201004

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 18000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb
Airspace.Class A : ZZZ

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 2000
ASRS Report Number.Accession Number : 881646

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 400
ASRS Report Number.Accession Number : 881647

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

After numerous maintenance resets on the ground for inoperative FMGC2, flight departed. On the climb, passing FL180, dual FMGC failure. No navigation information from FMGC caused request for vectors to downline fix. Procedure followed for dual FMGC failure and subsequent reset. Able to restore FMGC 1 with no success for recovery of FMGC 2. Maintenance contacted along with Dispatch to advise of failure. Use of Captain's emergency authority to reset circuit breakers in flight. After recovery of FMGC 1, flight proceeded to destination.

Narrative: 2

Climbing out lost navigation and map on both PFD's. Autopilot on. Several resets of FMGC's prior day and that morning before departure. As pilot not flying, Captain gave me the aircraft and radios as he went to Dispatch and Maintenance. I reverted to back up navigation on heading with autopilot on the once receiving a suitable VOR navigating there. No alternative or navigation deviation noted but FMGC never came back. Captain resets CB's, maintenance reports filed, continue to destination with no further incident.

Synopsis

An A320 flight crew reported a dual FMGC failure in flight after several preflight resets by maintenance for similar failures. FMGC 1 reset successfully.

ACN: 881593 (2840 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Windshear
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 881593
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Troubleshooting
Human Factors : Training / Qualification

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

On downwind for Runway XX, aircraft was slowed to flap speed for visual approach. Flaps 1 was selected with normal leading edge device indications. When flaps 2 was selected, no trailing edge flaps extended. Followed ECAM and Non-Normal Procedures and recycled flap handle position several times, from flaps up to flaps 3 positions and in between. [We] discussed diverting to a domestic airport as an alternate. Tower then had windshear report and decision was made to proceed to a domestic airport and land. Company, ATC and flight attendants/passengers were also advised. Prepared for full Slats/No Flap landing at our diversion airport and declared emergency and asked for equipment to stand by as precaution. If no slats/no flaps on final approach Runway XXR, decision was made to divert to another nearby airport if necessary, for longer runway lengths. On final approach, both slats and flaps operated normally and landing was made and taxied to the gate.

Synopsis

On initial approach to the scheduled destination an A320's trailing edge flaps did not extend with flaps 2 selected. The crew declared an emergency and diverted. During the second approach the flaps operated normally.

ACN: 881515 (2841 of 2908)

Time / Day

Date : 201004
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : LAX.Airport
State Reference : CA
Altitude.MSL.Single Value : 2000

Environment

Flight Conditions : Marginal

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Initial Approach
Route In Use.STAR : RIIVER 2
Airspace.Class B : LAX

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 17000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 881515
Human Factors : Other / Unknown

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Planned, reviewed, confirmed and briefed the RIIVR 2 and ILS 25L arrival and approach. SOCAL cleared us for the RIIVR 2 and ILS 24R. I think the controller heard hesitation on the read back and she offered that when we got closer in they may be able to change us to the ILS 25L. Captain responded that we just had to re-enter and re-check it all. Closer in the controller had a conflict developing and he canceled our approach clearance for ILS 24R and gave us a heading and clearance for the ILS 25L. He gave us a clearance from 7000 to 2500 and asked us to expedite through 4000. He noted he was trying to get us down and would have a plane over the top of us. Controller noted that the speed assignment and low altitude would take us below the BRAVO airspace and then we would be back in the Bravo airspace. I know that LAX used to pursue certificate action for inadvertent deviations below the Bravo airspace. Since we were under SOCAL control vectors and assigned speeds I assume this is not an issue.

Synopsis

Air carrier inbound to LAX described an excursion from Class B because of an ATC assigned altitude.

ACN: 881325 (2842 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Nose Gear Tire
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 881325
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 140
Experience.Flight Crew.Type : 800
ASRS Report Number.Accession Number : 881529
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Ground Personnel

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Vehicle
Detector.Person : Ground Personnel
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

On pushback, after setting the brakes, the Tug Driver indicated they were having some difficulty disconnecting the tow bar. He asked me to release the brakes so he could pull us forward to straighten us out. After setting the brakes again, he indicated they were still unable to disconnect the tow bar. By this time, both engines were running at idle thrust. He asked if I would again release the brakes so he could push us back a few feet and try again. Prior to releasing the brakes, I confirmed with the Tug Driver that he would make sure no one attempted to remove the tow bar prior to verifying with me that the brakes were once again set. I specifically asked the question because of the multiple failed attempts already made to disconnect. He acknowledged my request and I released the brakes. He pushed us back approximately 3 feet and stopped. Almost immediately after the push stopped, the aircraft started to move forward. As I was unsure whether or not he was pulling us forward (he never indicated he was going to) or if the tow bar had come loose, I immediately stopped the aircraft and set the brakes. I believe we moved forward approximately one foot. As I was setting the brakes the Tug Driver instructed me to "set brakes" and informed me the tow bar was stuck in between the tires. I asked him to explain exactly where the tow bar was stuck and he said it was actually jammed under the nose tires and he was unable to remove it. I phoned Maintenance Control who entered the item into the maintenance system and said he would call a Local Mechanic. Contract maintenance arrived at the aircraft approximately 15 minutes later and disassembled the tow bar in order to remove it from under the nose tires. Dis-assembly took approximately 30 minutes. After further discussion with Maintenance Control, it was determined that we needed to return to the gate for further inspection. It was later decided to ferry the aircraft without passengers for further inspection. Ferry flight was accomplished normally with no discrepancies noted.

Narrative: 2

I was starting both engines on the pushback. I didn't hear the communications between the Captain and the push crew until the engines were started. I turned on the flight interphone when the push crew asked if we could release brakes so he could push us back a few feet and try to release the tow bar again. The Captain confirmed with the Tug Driver that he would make sure no one attempted to remove the tow bar prior to verifying that the brakes were set. He acknowledged the Captain's request and the Captain released the brakes. He pushed us back about 3 feet and stopped. Almost immediately after the push stopped, the aircraft started to move forward.

Synopsis

An A320 Crew reported that following a difficult tow bar removal, the tug driver failed to ask the Captain to set the parking brake and the aircraft with engines running rolled over the tow bar. The aircraft was damaged.

ACN: 881140 (2843 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 33000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic Fluid
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 260
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 881140

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 186
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 881141

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Diverted
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Climbing out of FL330 on our way to FL370 got ECAM, "HYD Y sys lo lvl." Captain assigned me flying and talking duties and ran ECAM, turned off PTU and engine 2 pump. Yellow electrical pump stayed in auto. We elected to level at FL350. Captain contacted Dispatch and Maintenance. Our gross weight was approx. 146,000. We elected to divert where our weight was projected to be 139,500, below our max landing weight. We told ATC our intentions and got a route for ZZZ. The Captain than briefed the Purser of the situation, and gave them a time until landing and nature of problem. We had an uneventful approach and landing. The fire trucks were waiting and we cleared the runway and had them do a visual on the gear and brakes. After a thumbs up from them we continued the gate. The braking system was normal brakes and the accumulator was fully charged, but we had the ramp chock all wheels.

Synopsis

A320 flight crew experiences yellow hydraulic system low level approaching cruise level. ECAM actions are complied with and Dispatch consulted before diverting to a suitable alternate.

ACN: 881139 (2844 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Turbine Engine
Manufacturer : International Aero Engines IAE
Aircraft Reference : X

Component : 2

Aircraft Component : Cowling/Nacelle Fasteners, Latches
Manufacturer : International Aero Engines IAE
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13765
Experience.Flight Crew.Last 90 Days : 245
Experience.Flight Crew.Type : 3674
ASRS Report Number.Accession Number : 881139
Human Factors : Workload
Human Factors : Troubleshooting
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Other

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 882314
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

During the walk-around Captain discovered several missing and loose rivets from the tail cone section of engine number 1. The Maintenance office was close by, so the Captain verbally reported the defect to the Mechanic on duty in the office. The Mechanic instantly retorted that he would defer it. Captain showed area of concern to the Mechanic and the Mechanic said he would investigate. The Captain was told the area would be deferred under an [Airbus] Maintenance Tip (MT). Captain said if legal to do so, fine. 

Item entered into ACARS by Flight Crew. The Maintenance Release was received and the deferral verbiage caused some concern for the Crew. The action item for our logbook [write-up] simply said continued deferred - no time. As it was being deferred under the Maintenance Tip, I thought it necessary to mention it in the Maintenance Release as well as the Maintenance Tip requirements of maximum flight hours on the deferral. We asked for this amendment over the radio to ZZZ Maintenance as the door was closed. 

Maintenance seemed to have a hard time understanding our concerns and they did re-issue the Maintenance Release, but only added a line stating 15 rivets loose or missing...still no mention of the Maintenance Tip I was told was being used as the Deferral authority for the flight. Maintenance asked if this was good enough and would we go with that. Said "no". Brought Dispatch and Maintenance Control into the mix over ACARS and explained our concerns. Maintenance Control was on the phone with ZZZ Maintenance and it was determined the Maintenance Tip was not valid and we were not legal to go. 

After a great deal of time, we were told the flight could depart with an Engineering Authority for the missing/loose rivets. This took quite some time to generate and when it was reported as done we conferenced with Dispatch and Maintenance Control since the Engineering Authority is barely referenced in the FOM. We decided we could safely proceed. A Maintenance Release was generated with the required references (after 2 more requests) and the flight was then legal to depart. 

During our delay, ZZZ had a heavy hail event and we did need to de-ice. The Operations personnel continuously fed the [Flight] Crew false information as to time requirements for de-ice and the door was slammed a full 25 minutes before de-ice showed up. This was on top of our almost two hour maintenance delay. 

To conclude; I believe the myopic focus on deferring every item has the Mechanics distracted and sometime missing obvious items that can't be deferred. The [Flight] Crew should not have the job title of Maintenance Quality Control thrust upon them as well as all the other normal duties we have. The ZZZ Station folks all, at some time during our delay, only half jokingly asked the Crew what we broke. As I am carrying out my duties as a professional airline pilot, the detraction of non-certificated personnel opinions as to an aircraft's fly-ability are not required or welcome.

Narrative: 2

Flight Crew found rivets missing and "working" on Engine Common Nozzle Assembly. I called the Engine Desk for guidance. He referred me to Maintenance Tip (MT) 78-001. I deferred the item per this MT and released the A320 aircraft. The Captain was not happy. There was not enough information displayed on the Maintenance Release for him. 

While I reviewed the MT, I found that it would not cover the damage. It does not allow adjacent, missing rivets, which we had. I contacted the Engine Desk again. He said an Engineering Authority would be required. I contacted Engineering and posted photos to the Visual Inspection web site. It took quite some time to get the Engineering Authority. I changed the deferral reference and Released the aircraft again. The Captain was not happy. He wanted more details. He spoke to Maintenance Control himself.

Synopsis

A Captain and a Mechanic report about discrepancies in the deferral process of an A320 aircraft. The Captain had found numerous rivets missing and "working" on the # 1 engine Common Nozzle Assembly. Captain believes his Carrier's focus on deferring every item has the Mechanics distracted and sometimes missing obvious items that can't be deferred.

ACN: 880632 (2845 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation
Maintenance Status.Maintenance Items Involved : Work Cards

Component

Aircraft Component : Pitot-Static System
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Airframe
Qualification.Maintenance : Powerplant
ASRS Report Number.Accession Number : 880632
Human Factors : Time Pressure
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Primary Problem : Company Policy

Narrative: 1

Overnight Maintenance was started on an A320 in ZZZ. I was assigned to work an Engineering Order (EO) for replacement of all three Pitot probes. Complications began due to a Static leak on the Alternate (ALT) Static System. Pressure Switch (14wn) was changed and later fixed this problem. Larger then normal amounts of Black Electrical tape was used to hold the Pitot/Static hoses due to gusts upwards of 35-mph pulling the hoses from the aircraft. Finding no leaks in all three Systems, I cleaned up and Pitot/Static hoses were removed. Work was completed on March XX at XA:30 am and paperwork was started. Paperwork was competed at XY:30 am. 

Two days later, XZ:40 pm, I received a call from my Shop Steward informing me that when the A320 was at the gate in ZZZ1 after its first flight, tape was found on the aircraft. 

Due to out of the ordinary problems that occurred during the Engineering Order (EO), high winds removing the Pitot/Static hoses from the aircraft, large amounts of tape were used to keep the Pitot/Static hoses from moving, to complete a full Leak Check of the Pitot/Static system. Was told that the tape was removed from the aircraft. 

I suggest Upgrading our Work Card or EO to include an Inspector or another Mechanic to verify that no tape has been left or damage has occurred during the completion of this EO.

Synopsis

A Line Mechanic reports he was informed that tape was later found on an A320 fuselage at the gate of a downline Station. Mechanic had used a greater than normal amount of Black Electrical tape to secure the Pitot/Static hoses during the Pitot Leak test during heavy winds, after three Pitot probes had been replaced.

ACN: 880628 (2846 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Items Involved : Testing

Component

Aircraft Component : Indicating and Warning - Fuel System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Lead Technician : 15
Experience.Maintenance.Technician : 25
ASRS Report Number.Accession Number : 880628
Human Factors : Time Pressure
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

In reference to a Log Write-up, an A320 aircraft departed ZZZ and experienced a Right Wing Outer Tank Low Fuel Temp upper ECAM Display, which self-erased two minutes later. This aircraft has multiple hits with this same type of occurrence (referenced in deferral process as INF-information). I called Maintenance Control to confer if this recurring situation should not be addressed with a Fuel Tank Entry [procedure], with a Probe change and Sensor change, along with a possible Wiring Check at some time in the near future. 

We agreed that the existing situation with No Message Displayed was a deferrable condition, but something with this much history should be addressed soon. We agreed to make the deferral as was written by me, after the A320 arrival in ZZZ1, and requesting further Crew comments should the condition occur prior to FIX.

Callback: 1

Reporter, who was also the Lead Mechanic on-duty, stated the A320 that had the multiple hits of Right Outer Tank Low Fuel Temp upper ECAM Display which self-erased two minutes later, is a common problem on A318's, A319's and A320's. Mechanics from at least three other carriers at his Station, have called him asking about a "FIX" for the same problems on their Airbus aircraft. 

Reporter stated these are spurious, intermittent ECAM Messages that are displayed, generally only once during a flight segment, then disappear with no other indications. The "FIX" is to replace the Fuel Temp Sensor in the wing outer fuel tank, which takes about 35-minutes to remove and replace. But to get access to the Temperature Sensor, the fuel tank must be drained in order to remove at least one lower wing Fuel Tank Access Plate. Although the process for accessing the Sensor is called a Fuel Tank Entry Procedure, full body entry is not required; only one's hands, arms and head. Anywhere from six to twelve hours is the average time needed to get the tank opened, perform the repair, check the tank wiring, test the new Temp Sensor, close the tank, refuel the tank and check for fuel leaks; just to correct the spurious ECAM Messages. 

Reporter stated there is a Temp Sensor for each wing and they are interchangeable. He believes the caliber of the Temp Sensors being replaced are the problem. He also noted Maintenance Control had canceled the outbound trip for the A320 he was working with the Temp Sensor issue, even though six out of the seven previous Airbus with similar spurious ECAM Messages had been deferred, by the same Maintenance Controller.

Synopsis

A Line Lead Mechanic reports about a common problem of spurious, intermittent RT OUTER TANK LOW FUEL TEMP upper ECAM Display messages which self-erase two minutes later on the Airbus A318's, A319's and A320's. Replacement of the wing Fuel Temp Sensor generally resolves the problem, but requires a wing Fuel Tank Entry Open-up procedure.

ACN: 879344 (2847 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Required / Correct Doc On Board : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Indicating and Warning - Air Conditioning and Press.
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Pressurization Control System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 879344
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

An A320 aircraft had a deferred Write-up on the air conditioning Extract valve. The Write-up indicated that the Extract valve System/Synoptic Page showed amber where the System should show the valve position. The deferral procedure was to manually open the Extract valve and leave the valve "Open." Our constraint was that we needed deicing leaving ZZZ and the valve had to be "Closed" for deicing. The airplane was enroute to ZZZ after flying, ZZZ, ZZZ1, ZZZ. The airplane had been deiced leaving ZZZ. A Mechanic went to the deicing pad and closed the valve for deicing, then opened the valve after deicing so the aircraft was correctly dispatched. We were to take the airplane from ZZZ to ZZZ2 to ZZZ and then to ZZZ1. 

The airplane deferral required that the Extract valve operation be checked for valve position before each flight and then issued a Release. When the Mechanic confirmed the valve position during our preflight, I arranged for him to follow us to the deice pad in ZZZ. During our preflight we saw the Extract valve still had amber on the System/Synoptic Page and the deferral indicated that we would see amber indications and valve position on the Synoptic Page. We discussed the conflict but the original write up was exactly what was on the Page, a Mechanic had just checked the same Page and cleared the Write-up and the airplane had just completed two flights with the same deferral. I assumed that the airplane was correctly deferred. 

We departed the gate, went to the deice pad, had the Extract valve "Closed", had the valve "Opened", ran the rest of the preflight checklist and departed ZZZ for ZZZ2. After takeoff we got a VENT SKIN VALVE FAULT message. We knew the valve was closed so I directed the First Officer (F/O) to clear the message and we continued to ZZZ2. In ZZZ2 I watched the Mechanic test the valves and clear the deferral. The test was correct, so we continued the trip back to ZZZ. The same fault message appeared leaving ZZZ2. After our landing in ZZZ and during our preflight in ZZZ, we specifically asked the Mechanic if he knew if the valve Synoptic page was not indicating valve position because of the original Write-up, or if there was something else wrong. All he could tell us was what we already knew, the valves tested correctly using his procedure. 

We continued the trip and left for ZZZ1. Same process same result, we got the same fault message [VENT SKIN VALVE FAULT] leaving ZZZ. I contacted Maintenance Control to see if what we were experiencing was normal. The answer appears to be there was probably something else wrong. We were probably Dispatched on the wrong deferral. I do not know if the previous flights were dispatched in the same manner as ours, or if there was another failure after those flights and before our flight. 

The system operated as expected and we knew the Extract valve was closed. Yet, my experience has been that when a fault message is generated by a deferral then the Maintenance Release usually mentions the expected fault message. This situation just felt wrong. I needed to insure that the Crew that got the airplane from me was not getting an unknown problem, so, we just kept asking questions. If wishes were horses then beggars would ride; I wish I had asked the correct question to the correct person the first time I asked.

Synopsis

A Captain reports about an A320 that had a deferred Write-up for the air conditioning Avionics ventilation Extract valve with an expected indication on the ECAM System/Synoptic Page showing an amber VENT and Extract valve position. The valve position would not display; instead a VENT SKIN VALVE FAULT message kept recurring after Takeoff.

ACN: 879299 (2848 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Function.Flight Attendant : Flight Attendant In Charge
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 18550
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7400
ASRS Report Number.Accession Number : 879299
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Flight Attendant
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Attendant
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Chart Or Publication

Narrative: 1

During the pre-flight, the Lead Flight Attendant advised me that they could not locate the lanyard at each door as described in their Flight Attendant Manual. Since this was an overwater equipped aircraft, we sent in a maintenance report and asked for local Maintenance to come out. In order to locate the lanyards, the Mechanic had to open each of the doors and literally hang out of the aircraft to search underneath the slide/raft package. Upon locating the lanyards, he cleared the defect and subsequently sent a new Maintenance Release. 

However, the Pre-flight Checklist in the Flight Attendant Manual indicates that the lanyards should be visible, which was not the case, even after maintenance action. Although we chose to accept local Maintenance's actions, in light of the fact that we were not flying over long distances of water, I feel that this aircraft was not actually in compliance with the expectations of the FAA. The purpose of the lanyard is to attach the survival kit to the slide/raft in preparation for a ditching. This would require that the lanyard be accessible from inside the airplane with the doors closed to ensure the attachment of the survival kit. Since the only way to access the lanyard on this airplane was to open the door, there is no way that the survival kit could be attached for a ditching. 

I bring this situation up because I believe that if this aircraft were to be operated in an actual overwater operation, we would not be in compliance with the FARs, but more importantly, we would be operating in an unsafe manner. I further believe that our Air Carrier should audit our entire fleet of overwater A320s for similar instances of lanyard condition in the event that this is not an isolated occurrence. We might save ourselves a lot of grief by doing so and spare us an FAA action, or worse, the possibility of operating in an unsafe manner.

Callback: 1

Reporter stated his Air Carrier has not responded to his suggestion to have all of their A320's inspected for the survival strap being visible.

Synopsis

A Captain reports their Lead Flight Attendant advised him they could not locate the survival loop lanyards at each cabin door slide/raft position as described in their Flight Attendant Manual Pre-flight Checklist. Although Mechanic verified the lanyards were present, they were still not visible when the doors were closed. Pilot raises concern about possibly not being in compliance.

ACN: 879290 (2849 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Cockpit Door
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 130
Experience.Flight Crew.Type : 4130
ASRS Report Number.Accession Number : 879290
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : FAR
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : Release Refused / Aircraft Not Accepted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Procedure
Primary Problem : Company Policy

Narrative: 1

This aircraft had a MEL item. The cockpit door chime was inoperative. The fix was to defer the aircraft with the door keypad disconnected with the caveat that the door cockpit would not be accessible from the outside. I talked to the Dispatcher and was told that it would not be fixed prior to the flight (two hour sit) but would be deferred to the overnight station for repair. I was asked if I would be willing to fly it the one leg to repair (it had multiple legs on it already). I refused the aircraft. I feel that we plan for the worst case scenario and in this situation if a situation arose that the crew was unable to help themselves, then they were doomed due to the inability to access the cockpit. After explaining this thought process to the Dispatcher, who relayed it to Maintenance, the aircraft was shopped to a flight to a different destination. I don't understand how an aircraft could be deferred with ANY door access unavailable.

Synopsis

An A320 Captain reported refusing an aircraft after the cockpit door was MEL'ed inoperative and unable to be opened from the outside. Maintenance attempted to get other flight crews to accept the aircraft.

ACN: 878811 (2850 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : MCO.Airport
State Reference : FL
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Tower : MCO
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Airspace.Class B : MCO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
ASRS Report Number.Accession Number : 878811
Human Factors : Situational Awareness

Events

Anomaly.Inflight Event / Encounter : Bird / Animal
Detector.Person : Flight Crew
Detector.Person : Maintenance
Result.Aircraft : Aircraft Damaged

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

During landing, (flare, touch down), several small birds attacked aircraft. Due to lack of river water for ditching and the fact touchdown was imminent, landing was continued without incident. Tower was notified promptly after clearing runway. During post-flight, First Officer noted at least two suicides had been achieved: One by use of the "head-butt" method into the left side of aircraft. The second (apparently) by fire as Maintenance discovered charred remains and feathery material in right engine. All other important information was relayed to Dispatcher and can be found in his report.

Synopsis

An A320 Series Captain reported several bird strikes while landing at MCO.