ACN: 878796 (2851 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 600
ASRS Report Number.Accession Number : 878796

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

On landing rollout we had an electrical failure that caused us to lose communication and nose wheel steering along with multiple other failures. We were unable to clear the runway until we got the airplane stabilized and we used differential braking and power to clear the runway. We also were able to reestablish communication as we worked our way clear of the runway after we got a few systems reset.

Synopsis

An A320 flight crew suffered multiple system losses on the landing roll when electrical power to them was lost.

ACN: 878770 (2852 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0001-0600

Place

Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Landing

Component

Aircraft Component : Electrical Power
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6000
Experience.Flight Crew.Last 90 Days : 80
Experience.Flight Crew.Type : 150
ASRS Report Number.Accession Number : 878770
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Upon rollout lost electrical power (no radio contact with tower, limited directional control). Cleared runway, troubleshot, and reestablished electrical power. With clearance, taxied to gate uneventfully.

Synopsis

An A320 Captain reported losing all electrical power on landing roll, power was reset successfully and the aircraft was taxied to the gate.

ACN: 878581 (2853 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Takeoff / Launch
Airspace.Class B : ZZZ

Component

Aircraft Component : Nose Gear
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 878581

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.Flight Crew : Landed As Precaution
Result.Flight Crew : Returned To Departure Airport

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Normal takeoff by the First Officer. During gear retraction, the gear handle came up, and immediately the ECAM sounded with multiple messages appearing! It was obvious with all of the noise and excess drag that there was a gear malfunction, confirmed by the nose gear indicating red with the gear handle in the up position. We placed the gear handle in the down position, gear came down, but we were left with the L/G Not Uplocked ECAM. We advised ATC of our situation and proceeded to hold. We contacted Dispatch and Maintenance to advise of our condition. It appeared the L/G doors were closed. We coordinated with Tower to do a low approach and confirm gear down and locked and a controllability check. During the approach we selected full flaps and got excessive vibration. We retracted the flaps back to position 3, where the vibration ceased and the low approach confirmed 3 gear extended. We flew an uneventful flaps 3 landing with minimum sink rate on landing, as requested by Maintenance.

Synopsis

A320 Captain reports the nose gear fails to retract after takeoff but appears to extend normally. Flight returns to departure airport for uneventful landing.

ACN: 878439 (2854 of 2908)

Time / Day

Date : 201003

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 878439
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Ground Event / Encounter : Other / Unknown
Detector.Person : Ground Personnel
When Detected : Aircraft In Service At Gate
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We had an inoperative APU, and were starting an engine at the gate. Received clearance to start engine one, and I called for the First Officer to "start 1" and we did. The First Officer thought were cleared to start both, and then started number 2. Ground personnel on the headset told me to shut it down, and we immediately shut it down. It only rotated for about 3-5 seconds without any ignition, etc.

Synopsis

A320 Captain is cleared to start the number one engine at the gate due to an inoperative APU. The First Officer misunderstands and attempts to start the second engine before being cleared to do so.

ACN: 877945 (2855 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 300
Experience.Flight Crew.Type : 3712
ASRS Report Number.Accession Number : 877945
Human Factors : Troubleshooting
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Primary Problem : Human Factors

Narrative: 1

After completing an autoland and clearing the runway on the P exit, the aircraft swerved right, then left and almost went off the left side of P. Although I did not believe we had left the pavement or hit anything (specifically taxi edge lights), we notified Ground to have the taxiway inspected to make sure. The aircraft was easily turned back to centerline and I felt it was safe to taxi to the gate. I called Dispatch after landing and let him know what happened and got the phone number of Tower. The Tower Supervisor informed me that the Operation's vehicle observed skid marks but there was no damage and nothing had left the pavement. Maintenance saw no damage to the aircraft or any evidence that we left the pavement. We did the autoland here because the autoland on the previous leg was marginal. The plane touched down right of centerline and "hunted" to get back (the inflight portion of the approach was fine). I disconnected the autopilot and we taxied to the gate without a problem. My First Officer and I discussed whether to rate the autoland as SAT or UNSAT and decided that since we were taking the plane back to a maintenance station, we would attempt another autoland and see how the plane performed. At our destination, although we touched down right of centerline, it was very close and the rollout was smooth. In both cases, I had the brakes set to low and there was a slight right crosswind. I left the autopilot on a little longer than I have done in the past, but after coming out of reverse thrust and applying brake pressure to release the autobrakes, I thought I had disconnected the autopilot, using the control stick disconnect button (Later, I realized I didn't hear the disconnect caution chime). The turn to exit the runway at P seemed normal and I didn't seem to be fighting the airplane. After clearing the Runway, I configured the exterior lights for taxi and about this time, the plane swerved right, then left. The left turn might have been my reaction on the brakes, rudder and tiller to stop the swerve. During this time, my First Officer said something like, "Hey, the autopilot is still on." I think we both tried to disconnect because I heard "priority left." In any case, the autopilot disconnected and we stopped the plane. Although I don't think we were going very fast, both my First Officer and I were hard on the brakes to stop. In retrospect, while the Airbus will do some weird things, I don't think the autopilots came back on, so I didn't get a disconnect even though I thought I did. What I do not understand is how I was able, so easily, to turn off the Runway centerline to the taxiway without the autopilot fighting to stay on course. When I first checked out on the plane, I remember trying to steer before turning off the autopilot; it was not easy and very evident what was wrong. There was none of that feedback this time until we were on the taxiway. In the future, I will add the disconnect time to my approach briefing to both bring the First Officer more fully into the rollout loop (no pun intended) and also raise my own awareness of the last part of this procedure. I will also brief to listen for the disconnect chime.

Synopsis

An A320 Pilot completed an autoland but failed to turn the autopilot off after landing. While exiting the runway the aircraft attempted to turn back toward the runway with a jerking motion.

ACN: 877852 (2856 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZAU.ARTCC
State Reference : IL
Altitude.MSL.Single Value : 26000

Environment

Weather Elements / Visibility : Turbulence
Light : Night

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use.STAR : WATSN
Airspace.Class A : ZAU

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZAU
Aircraft Operator : Air Carrier
Make Model Name : Regional Jet CL65, Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Descent
Route In Use : Vectors
Route In Use.STAR : WATSN
Airspace.Class A : ZAU

Person

Reference : 1
Location Of Person.Facility : ZAU.ARTCC
Reporter Organization : Government
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 877852
Human Factors : Workload

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Weather
Primary Problem : Human Factors

Narrative: 1

The southeast area was impacted by severe weather and I had been working several ORD inbounds on the WATSN STAR. I cleared Air Carrier Y into the hold at WATSN at FL260, and climbed Air Carrier X to FL270 to ensure separation with the Air Carrier Y flight. I then cleared Air Carrier X to hold at WATSN. Another Carrier (I'm not sure which one) was landing DAY and I had descended that aircraft to FL280 opposite direction of the first two aircraft I had holding. My intention was to clear Air Carrier X and then expedite the DAY inbound down to FL240 to get the aircraft out of my airspace. Once the Air Carrier Y aircraft began its outbound turn in the holding pattern I noticed Air Carrier X's altitude readout was FL264 and descending. I issued an immediate right turn to the Air Carrier Y flight ensuring separation with the deviating flight. After that, I asked Air Carrier X to verify he was at FL270. He said, "We are trying to maintain that altitude, but our autopilot kicked off and we're getting back up to it." My DAY arrival was about to leave FL290 for FL280 when I noticed Air Carrier X's altitude readout to be climbing back up to FL270, then FL274, then FL278. The DAY inbound and Air Carrier X were 10 miles apart, head on, and my DAY arrival was leaving FL289. I issued a traffic alert to the DAY arrival, and an immediate left hand turn to a 050 heading. This would not have ensured separation, but at this point I wanted to avoid a mid-air collision. I then told the DAY arrival to stop his descent and level off wherever he was at. The aircraft did so, and I then noticed Air Carrier X's altitude readout in a descent back to FL270. The Pilot then said he thought he had control of the aircraft, but that holding at WATSN was almost impossible due to weather there. Other pilots (I was clearing four aircraft to hold at WATSN) were telling me similar reports. After vectoring the DAY arrival around my holding pattern and getting him out of the way, I started vectoring all my aircraft to the east of the WATSN holding pattern, knowing the weather there is a little clearer. Then our BEARZ sector took me out of the hold, and I was able to bring those aircraft back in and hand them off to BEARZ. I have no idea how close Air Carrier X came to that DAY arrival. It was close enough to scare me, and the fact that the Pilot did not appear to be in control of his aircraft worried me even more. This situation spiraled out of control due to my having to work too many aircraft in an area of known weather, and I had no where else to go. Recommendation, first of all, our Supervisor had allowed a dangerous situation to escalate. The controllers in the southeast area knew there was severe weather impacting arrivals, and that nearly all ORD and MDW arrivals through the area were unable to make it through the weather. Throughout the night I heard and voiced to myself that the situation was going to escalate out of control. That being said, we were still forced to work several arrivals that had little chance of making it to their destination. Had our Supervisor had intimate knowledge of how the traffic flow in the southeast area worked, we could have avoided the situation. In the past, years ago, Operational Supervisors had to be certified on all positions in their area they are charged with. Now, Supervisors are allowed to be certified on only one position, usually the position with the least amount of traffic or the least complex traffic. Couple this with a Supervisor who may not have the fortitude to stand up to a demanding TMU Supervisor who wishes nothing more than to push the maximum amount of aircraft into a sector, and you invite a dangerous situation. I had no choice but to hold aircraft near weather, because that was what was handed to me, and my Supervisor and Flow Control stated that this plan would work. This is obvious proof that safety of the operation was not on the minds of these parties. My situation was only one (though a very dangerous one) of several situations that occurred that night creating unneeded safety hazards and an unprecedented amount of work by myself and other controllers in an attempt to ensure safety. Air Carrier X stated he could barely control his aircraft. I placed him in a precarious situation by clearing him into holding at the WATSN intersection, in close proximity to weather. The sad part is I had little other choice. I was working a normal traffic load in abnormal conditions where severe weather created numerous weather deviations and a huge increase in workload. I had no other airspace to hold my ORD arrivals - they were taken by enroute aircraft attempting to deviate around weather. Had I not been taken out of the hold in a timely manner, I really do not know what I would have done with the four aircraft I had to hold. I had no place to go with them. Had safety been a priority, these aircraft bound to ORD and MDW would have been routed around the weather, and clear of our airspace. Our Supervisor would have the fortitude to protect his area from having a dangerous situation present itself. Instead, in an attempt to get as many ORD arrivals into the airport as possible, ZAU's Management and Flow Control jeopardized the lives of countless people aboard those aircraft.

Synopsis

ZAU Controller described a heavy complex traffic period when holding was required for ORD arrivals and aircraft were unable to maintain altitude because of weather, reporter claiming supervisory/traffic management oversight and planning was lacking and safety was compromised.

ACN: 877778 (2857 of 2908)

Time / Day

Date : 201003

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Descent
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Cabin Jumpseat
Reporter Organization : Air Carrier
Function.Flight Attendant : Flight Attendant (On Duty)
ASRS Report Number.Accession Number : 877778
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Attendant

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Aircraft

Narrative: 1

We Flight Attendants were strapped in for landing when it felt as if the cockpit had decided against a landing and decided to go around again. Then the airplane went down and up down and up (short little dips) about 4 or 5 times in a row. (The cockpit may have been troubleshooting at this time.) We looked at each other and wondered aloud what are they doing? Next we heard an announcement over the PA from the Captain. It wasn't clear but what we got out of it was that there were problems with the flaps and they were going to try to do something about it. We circled a few times before we heard the Lead Flight Attendant's phone ring in the front. A couple minutes later our phone rang in the back. It was the Lead telling us that the cockpit said that there were problems with the flaps. Due to these problems we were expected to come in too fast and possibly go off the runway. We had 15 minutes to do cabin prep. The signal to brace would "Brace" and the signal to remain seated would be "Remain seated" There weren't any special instructions. The Lead made the jumpseat prep announcements. In the back we double-checked the galley to make sure everything was closed, latched and secured. Then we sat down, strapped in, and reviewed our evacuation duties. We got an "All Call" from the First Officer. This was helpful because we got the same information at the same time. He didn't explain why (probably due to time constraints) but he said that the emergency may be cancelled once on the ground. This made us feel a little better in the back. When we heard the cockpit say "Brace" over the PA we yelled "Brace brace brace" until landing. As we landed we could tell that we were coming in much faster than normal. Almost immediately the cockpit announced "Remain seated". We had not gone off the runway but there were many fire trucks and ambulances along the taxiway just in case. After all passengers deplaned Onboard Supervisor met the crew to debrief and remind us of the EAP resources available to us if needed. The Captain offered to debrief with us as well. A different crew performed a cabin preparation on this flight four days later on also due to flaps problems also on this same plane! Apparently the problem has not been fixed!? I am VERY concerned about this. The lives of our passengers and crew are at stake. Maintenance needs to know about this ASAP.

Synopsis

An A320 Flight Attendant described the flight attendants' cabin preparation following notification by the flight crew about an impending emergency landing as the result of a flap malfunction.

ACN: 877713 (2858 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Taxi

Component

Aircraft Component : Autopilot
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 15000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 877713
Human Factors : Confusion

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Human Factors : Situational Awareness
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
When Detected : Taxi
Result.Flight Crew : Regained Aircraft Control

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

After auto landing, the Captain was some what concerned about the performance of the auto pilots with respect to the landing. We both considered it to be satisfactory but he elected to do another one on our return flight. The landing was again satisfactory. The aircraft decelerated through 60 knots and Tower told us to turn left and contact ground. As we made the turn, I noticed that both auto pilots were still engaged. I immediately brought this to the Captain's attention. It was at this time that the aircraft veered extremely hard left toward toward the grass. We both applied maximum braking and came within inches of leaving the pavement. Both auto pilots were then turned off. We taxied to the gate without further incident. The Captain was absolutely certain that he had disconnected the auto pilots while still on the runway before turning off. We were concerned that we may have hit blue taxi lights and reported it to the Tower. After further inspection, no lights were damaged.

Synopsis

A320 First Officer reports the Captain attempting to turn off the runway after an auto land without turning off the auto pilots. A hard left turn results which is only arrested by quickly stopping the aircraft.

ACN: 877263 (2859 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZNY.ARTCC
State Reference : NY

Environment

Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZNY
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Cruise
Route In Use.Airway : J230
Airspace.Class A : ZNY

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 877263
Human Factors : Human-Machine Interface
Human Factors : Confusion
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

JFK to ZZZ J230 LARRI to GRAHM. Software inserts [waypoint] THS04. Navigation display shows a left turn without connecting to anything and no discontinuity. At THS04 the aircraft starts a left turn. I don't know to where or how far it would have turned.

Synopsis

An A320 Captain reported his FMC deviated from J230 between LARRI and GRAHM, starting a left turn with no downline waypoint and no discontinuity.

ACN: 877247 (2860 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Landing
Route In Use : Visual Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 5950
ASRS Report Number.Accession Number : 877247
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Experience.Flight Crew.Total : 20000
Experience.Flight Crew.Last 90 Days : 42
Experience.Flight Crew.Type : 42
ASRS Report Number.Accession Number : 877136
Human Factors : Situational Awareness
Human Factors : Troubleshooting

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : FLC complied w / Automation / Advisory

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Captain flying, cleared for a visual approach. We were on a left base at flaps 1. While turning final outside of the final approach fix, Captain is slowing and calls for gear down and flaps 2. After I move the flap lever to the flaps 2 position, we get a F/CTL Flaps Locked-Alignment Fault ECAM on the upper ECAM. Following the ECAM, I tried to raise the flaps back to flaps 1. The ECAM remained and the flaps were locked. The Captain and I agreed that we needed to break off the approach. I told the Tower Controller that we had a flight control problem and needed to break off the approach. He gave us a heading to fly and gave us an altitude of 6,000. I told him we needed to go hold while we worked through the problem. Captain remained in control of the airplane and took ATC while I got the Flight Manual out and I work through the procedures of the ECAM. After determining our landing configuration, approach speed, and landing distance I got a patch to Maintenance Control to get any additional input. Maintenance Control said follow the flight manual procedures. At this point the Captain reviewed the landing chart in the maneuvers section with abnormal flaps. We were now ready to return to land. This took about 10-15 minutes. I advised ATC that we are declaring an Emergency and would like to return to Runaway XXR and we would like the emergency equipment standing by. The Captain made a very nice approach and landing on Runway XXR. The Captain made several PA's to the passengers throughout this event and the Flight Attendants did a cabin advisory. After landing on Runway XXR, we exited and set the brakes. The emergency folks inspected the airplane and we were cleared to taxi to gate.

Synopsis

An A320 crew reported an ECAM alerting F/CTL Flaps Locked-Alignment Fault. After following ECAM and Flight Manual procedures, an emergency was declare and the Captain landed on the longest available runway.

ACN: 877212 (2861 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : P50.TRACON
State Reference : AZ
Altitude.MSL.Single Value : 5000

Aircraft

Reference : X
ATC / Advisory.TRACON : P50
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Descent
Route In Use : Visual Approach
Airspace.Class B : PHX

Person

Reference : 1
Location Of Person.Facility : P50.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 877212
Human Factors : Other / Unknown

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Inflight Event / Encounter : Loss Of Aircraft Control
Detector.Person : Flight Crew
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Separated Traffic
Result.Aircraft : Automation Overrode Flight Crew

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

While working the Biltmore Sector Air Carrier X went around and climbed through the satellite corridor airspace. After talking to the Final Controller it became evident that Air Carrier X was having software difficulties and sped up to 320 knots on a 5 mile final, unsure and with no time to coordinate the Final Controller scanned ahead in the corridor and saw that it was clear and determined that the best course of action would be to climb Air Carrier X to an altitude above Tower departures and above the MVA. So the Final Controller climbed the go around to 6,000. He thought about starting a turn, but it is actually best he didn't as this would have been more of a conflict with the traffic he didn't see 15 north southbound in my sky at 5,000. In hindsight and after talking with the Final Controller I stated that I thought that the best thing would have been to just switch the arrival to the Tower so that they, who are trained in go arounds, could coordinate with the Departure Controller. However, a straight shot over the top of PHX at Air Carrier X's high speed was in actuality the best course of action short of shipping the arrival to the Tower as it was clear and any turns would have put Air Carrier X in the Biltmore transition corridor for a longer time. The Arrival Coordinator came over and coordinated the go around with me and I was watching the traffic, and while the words point out approved never came out of my mouth it was in effect very similar as I was watching the traffic and providing separation from it with what little traffic I had (a VFR Citation 13NM north of the airport southbound at 5,000 who had the A320 in sight when they were still 6 miles apart and 600 feet vertically.) Recommendation, some sort of simulation training or SOP for final controllers to know what to do in such events. I think the Final Controller would agree.

Synopsis

P50 controller described airspace incursion event when aircraft on final, experiencing equipment difficulties, went around with airspeed and altitude deviations that resulted in an uncoordinated airspace entry.

ACN: 876797 (2862 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 3500

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 876797

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 60
Experience.Flight Crew.Type : 6800
ASRS Report Number.Accession Number : 876793

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At about 3000 feet on final to XXL we put the flap handle to flaps 2 and got an ECAM for F/CTL FLAP FAULT- ALIGNMENT. At this point we tried recycling the flaps from two to one and back to two to try and correct the problem. The leading edge devices moved normally but the trailing flaps remained up, with an amber indication. At this point we discontinued the approach and asked for vectors to work the problem. The copilot flew and worked the radio while I contacted Dispatch and Maintenance Control via ACARS. I made an announcement to the passengers. With Dispatch and Maintenance on the radio we went over the problem and tried cycling the flaps again with no fix. Maintenance Control said there was no way to correct the problem and we went over correct approach procedures. At this point I had the First Officer declare an emergency. The First Officer and I decided to have the cabin prepped for an evacuation due to the unusual approach, high speeds, possible tail strike, blown tires, hot brakes, etc. The Flight Attendants were notified, and I made another announcement to the passengers. We briefed the approach, with all the additional items and proper VREF which we calculated to be 158kts. On downwind to base I became pilot flying. The approach and landing to XXR, was uneventful. Brace was called for at 500 feet, and we told everybody to remain seated as we slowed to taxi speed. The landing roll and braking effort was a little more than normal and we saw no problem to continue to the gate.

Narrative: 2

The initial response was for the First Officer to continue as pilot flying, and talk to ATC, and the Captain would handle the ECAM. The approach was continued, gear was lowered passing the FAF and one attempt was made to recycle the flaps. At approximately 1600' a decision was made to go-around in the current configuration, and a climb to 2000' and then to 5000' was coordinated with ATC.

Synopsis

An A320 flight crew declared an emergency and landed safely after the flaps failed to extend normally.

ACN: 876265 (2863 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : A90.TRACON
State Reference : NH
Altitude.MSL.Single Value : 9000

Aircraft

Reference : X
ATC / Advisory.TRACON : A90
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Climb
Route In Use.SID : SSOXS1
Airspace.Class B : BOS

Person

Reference : 1
Location Of Person.Facility : A90.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Departure
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 876265
Human Factors : Other / Unknown

Events

Anomaly.ATC Issue : All Types
Anomaly.Deviation - Track / Heading : All Types
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Procedure
Primary Problem : Procedure

Narrative: 1

Air Carrier X departed Runway 09. Aircraft was flight planned over NELIE intersection and the FDIO strip indicated the PATSS1 departure appropriately in box 10, per BOS SOP and BOS/A90 LOA. Upon reaching the vicinity of GLLET the aircraft failed to turn westbound, as per the procedure, and began to continue southbound. The trainee on position, realizing that this was incorrect, immediately issued a vector to the west. This submitter then took over the position to verify which SID the pilots were flying. The pilot stated that they were flying the SSOXS1 departure. The flight received an IFR clearance via PDC so we asked them to verify the procedure on their clearance printout. They stated the PDC indicated SSOXS1. If Air Carrier X had been NORDO, the aircraft would have gone to SSOXS and then turned westbound, potentially conflicting with arrivals to BOS from over PVD. Boston now has 8 SIDs published. Six of the SIDs are RNAV procedures that were developed to meet noise abatement wishes of the surrounding communities. All six of these procedures are dependent on the direction (routing) the aircraft will exit A90 airspace, not which runway the aircraft departs. These procedures were published in the Fall of 2009, however were not put to use until February 1, 2010. The only indication a controller has of which procedure was issued to the pilot is a single letter written in box 10 of the FDIO strip. There is no requirement for BOS ATCT to verify that a pilot has received the correct SID (routing). After one month of these procedures being used, and numerous and various errors surrounding the use of them, there continues to be no requirement for BOS ATCT to verify with the pilot that the correct SID (routing) was issued and received. A90 controllers have no confidence in these procedures. Anticipated separation can no longer be used and this submitter has resorted to applying vertical separation primarily because lateral separation cannot be guaranteed. Recommendation: 1. BOS ATCT must be required to verify with pilots the receipt of proper SIDs. 2. Required JO 7110.65 takeoff clearance phraseology should be amended to include RNAV SIDs. ex., Air Carrier X, after departure fly the PATSS1, runway niner cleared for takeoff. 3. Pilot phraseology on departure should include call sign, SID name, and altitude verification information. 4. Currently, each of 6 RNAV SIDs at BOS are designed to serve 5 different runways to an exit fix. RNAV SIDs at BOS should be redesigned to be runway dependent with transitions to the proper exit fixes.

Synopsis

A90 Controller described a confused SID assignment event when traffic had to vectored to ensure separation, the Reporter commenting that new RNAV SIDS are causing confusion and that new verification procedures should be implemented.

ACN: 876162 (2864 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Thunderstorm
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : Landing Gear
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Nosewheel Steering
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 14800
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 520
ASRS Report Number.Accession Number : 876162

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 240
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 876298
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Separated Traffic

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

At approx 700 feet we got ECAM message "L/G DOORS NOT CLOSED". The Captain completed the ECAM procedure. The only things that needed to be noted were performance items due to the gear doors down. We had 3 green lights on the the ECAM display and on the Main Gear Panel. We elected to land. Upon landing and slowing below 60 knots the Captain assumed control of the aircraft. The exit, M3, was to the left but the aircraft was veering to the right. We stopped just short of the runway edge. We notified Tower that we were disabled. After coming to a complete stop and setting the parking park the Captain recycled the Nose Wheel/Anti Skid switch. The steering system came back on and we were able to turn left and exit the runway. We taxied to our gate, and notified Maintenance of the problem. At no time did the aircraft ever leave the runway or paved surface.

Narrative: 2

I was Pilot Not Flying during this ILS in visual conditions. After gear extension we received ECAM's for "L/G DOORS NOT CLOSED" and also for "L/G UNSAFE". I instructed the First Officer, who was Pilot Flying, to continue the approach while I analyzed the situation. We had three green gear down indications on the landing gear panel from the LGCIU 1 [Landing Gear Control Interface Unit], and the corresponding down indications on lower ECAM. However the background gear indications showed an unsafe nose landing gear. Additionally, the gear doors appeared to be hanging for both the mains and the nose, and this was corroborated by the First Officer, who was having to carry more power than is usual for the approach, which was now well below 1000 feet. We engaged in a brief discussion of the situation, and elected to continue the approach to a landing. This was done with the assurance from the LGCIU 1 that the gear were, in fact, down and locked, and also with the recent Newark incident in mind and doubts as to gear door sequencing, had we elected to cycle the gear. I also elected to "emergency cancel" a subsequent Level 3 ECAM for gear not down at 750 feet. The landing was smooth and uneventful until the First Officer offered up the airplane to me while adding, "I don't think we have steering". Not only did we not have steering but the aircraft pulled moderately to the right after it decelerated below 70 knots, and it was all that I could do to keep it on the paved surface of the runway abeam taxiway Mike 3. I brought the aircraft to a stop and set the parking brake while instructing the Tower to tell following aircraft to go around. I was acutely aware that the Tower had already cleared us to exit Mike 3 and contact Ground Control, had also cleared an aircraft behind us to land, and I was extremely worried about our position on the runway and our lack of visibility in this night operation. I then cycled the anti skid and nose gear steering switch and everything returned to normal, and we were able to taxi to the gate uneventfully, considering our proximity to the right runway edge. In retrospect, I think it is clear that the better option would have been to go around and sort out the situation airborne with the assistance of the checklists and possibly of Maintenance Control, but it should be pointed out that the maintenance personnel who met the airplane characterized the situation as "really messed up". I have had LGCIU failures before, and this just did not seem like the garden variety failure.

Synopsis

A320 flight crew reports ECAM message L/G DOORS NOT CLOSED and L/G UNSAFE after gear extension during approach. Two other indications are that the gear is safe so approach is continued to landing. During roll out the nose wheel steering is discovered to be inoperative causing the following aircraft to go around.

ACN: 876138 (2865 of 2908)

Time / Day

Date : 201003
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZAB.ARTCC
State Reference : NM
Altitude.MSL.Single Value : 29000

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Airspace.Class A : ZAB

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZAB
Aircraft Operator : Air Carrier
Make Model Name : B757-200
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZAB

Person

Reference : 1
Location Of Person.Facility : ZAB.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 876138
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working Sector R91/R65 combined. I took the hand off from Sector R42 for Aircraft X and put in an interim altitude of FL280 due to the Aircraft Y directly overhead at FL300. I also put a J-Ring around the Aircraft Y so as to let me know when I could climb Aircraft X to his requested altitude. Aircraft X checked on from Sector R42 climbing to FL250. I acknowledged Aircraft X's check on and proceeded to climb him to FL380 and he read back FL380. I thought I had climbed Aircraft X to FL280 since I had already put the interim altitude in the data block prior to him checking on and there was a J-Ring over the aircraft that I had to miss when I was ready to give Aircraft X his requested altitude. The next controller plugged into the sector and I began to give him a normal relief briefing. I unplugged after the briefing was done I proceeded to leave the area. About 2 minutes later, the relieving controller called me over and told me that Aircraft X had climbed through FL290 and that the pilot had said that they were assigned FL380. I reported the incident to my FLM (Front Line Manager) and we proceeded to the OM to take a look at what happened. Recommendation, I am not sure what I would recommend to prevent this kind of situation. In my mind I had climbed Aircraft X to FL280 because it was in the data block and there was a J-Ring around the Aircraft Y that I was supposed to miss. It was just an instance of my mind seeing and believing one thing to be true and then my mouth saying something that was completely different. If there was anything I could have done different is just to re-verify the assigned altitude of Aircraft X and do that same procedure with any similar situations.

Synopsis

ZAB controller experienced conflict event when failing to restrict climbing traffic below occupied altitude and failing to properly brief his/her relief.

ACN: 875827 (2866 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Work Cards
Maintenance Status.Maintenance Items Involved : Installation

Component : 1

Aircraft Component : Electrical Distribution
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Longeron/Stringer
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 875827
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Other / Unknown
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Routine Inspection
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Jobcard requires removal of aft Electronic Bay (E-bay) floor boards and displacing insulation for corrosion inspection under the water separators. The aircraft arrived without insulation around frame 27. The problem is this aircraft did not have any insulation installed. I did the Inspection and signed off the card that I did [in fact] remove and reinstalled the insulation. After work I started to think about avionics operating parameters operating outside normal ranges and the adverse effects on these systems [that it] could have. Really think the insulation needs to be installed immediately.

After serious consideration, I feel that this insulation needs to be in place. I am submitting this report to correct this situation.

Synopsis

A Mechanic reports he signed off a Jobcard for the removal and reinstallation of insulation under the aft electronic bay floorboards around frame 27 of an A320. Aircraft however, had arrived without any insulation installed. Concerned about the lack of insulation causing adverse temperatures in Avionics compartment, Mechanic files report to have insulation installed.

ACN: 875747 (2867 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : BOS.Airport
State Reference : MA
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 13564
Experience.Flight Crew.Last 90 Days : 225
Experience.Flight Crew.Type : 3451
ASRS Report Number.Accession Number : 875747
Human Factors : Other / Unknown

Events

Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Procedure
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

Upon arriving at the aircraft I noticed the weather had turned to snow and it developed into a fairly strong snow fall with a restriction to visibility being observed. We checked ATIS several times and noticed it never reported the snow. As timely and accurate information is critical to safe flight operations, we queried ATC as to the lack of an accurate ATIS report. He said that they can't control it, it is automated. I asked if it is obviously wrong, as it was now, can they over-ride it? He said no and really did not seem terribly interested in doing anything to correct this gross discrepancy. I am noticing that ATIS reports at a lot of airports do not reflect current conditions in any form of late.

Synopsis

Air carrier preparing for departure from BOS noted a discrepancy between the ATIS weather information and the conditions being experienced, i.e. snow vs. rain, ATC indicating any lack of over-ride capability to revise/update the automated ATIS information.

ACN: 875642 (2868 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 38000

Environment

Weather Elements / Visibility : Rain

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Airspace.Class A : ZZZ

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 2900
ASRS Report Number.Accession Number : 875642

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 7000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 700
ASRS Report Number.Accession Number : 875635

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Requested ATC Assistance / Clarification
Result.Flight Crew : Overcame Equipment Problem
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Apparent dual FMGC failure prior to top of descent. Lost all map displays and MCDU displayed please wait for remainder of flight. Reported the nav failure to Center and received vectors/altitudes in conjunction with activating standby navigation. Flew a visual approach.

Synopsis

An A320 Flight Crew experienced dual FMGC failure on arrival. Backup navigation was used to facilitate the visual approach.

ACN: 874865 (2869 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Scheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection

Component : 1

Aircraft Component : Liferaft
Manufacturer : Airbus
Aircraft Reference : X

Component : 2

Aircraft Component : Escape Slide
Manufacturer : Airbus
Aircraft Reference : X

Person : 1

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Inspector
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 874865
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Person : 2

Reference : 2
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Inspector
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
ASRS Report Number.Accession Number : 874866
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Deviation / Discrepancy - Procedural : FAR
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action
Result.General : Flight Cancelled / Delayed

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Procedure
Primary Problem : Manuals

Narrative: 1

It was called to my attention that the survival strap installations on the cabin door 2-LH and 2-RH slide installations were not done properly. This was a result of a inbound logbook item on the door 1-LH. All of the slide installation on the aircraft were done wrong, all four slides. I had checked these on an "A" Check, the items were inspected and flagged as Service Difficulty Report (SDR) item. Why were they now incorrectly done? This is an obvious issue and needs to be looked in to.

Callback: 1

Reporter stated these are extended overwater slide rafts used on their A320's, not the standard domestic cabin door slides. The "A" Check calls out to specifically check the overwater slide rafts for a visible white survival strap. These slide rafts have three straps: a mooring strap, a white survival strap and a yellow strap. All four cabin door slides were missing the required white survival strap.

Reporter stated that from his perspective as a mechanic and inspector, the Airbus Maintenance Manual is a mess, has conflicting data, unclear reference pictures and difficult to follow. Plus the "A" Check Job Card was written incorrectly. Since he became involved, his carrier has re-written their "A" Check procedures and Airbus has corrected the reference pictures that involve the slide raft installations. As a result of their write-ups, numerous A320's in their fleet had to be re-inspected for missing survival straps.

Narrative: 2

A320 aircraft had a log write-up for the survival strap lanyard, not visible at cabin Door-1R. A month before, I had Inspected DOORs 1-R, 2-L and 2-R on this aircraft. I have a complete understanding on how the survival straps on the A319/320 overwater slide rafts are routed, as I have inspected numerous installations of these slides. I noticed the information in our maintenance computer that the aircraft had very recently had a "C" Check accomplished. I suspect these slides were removed during the "C" Check.

Synopsis

Two inspectors report about the same A320 aircraft they had previously inspected on different dates and had made similar write-ups about, concerning the missing white survival straps on all four cabin door slide rafts.

ACN: 874824 (2870 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 4000

Environment

Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Flight Plan : IFR
Mission : Ferry / Re-Positioning
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Trailing Edge Flap
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 874824
Human Factors : Troubleshooting
Human Factors : Training / Qualification
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

Repositioning flight with no flight attendants or passengers. On approach to Runway XX when flaps 2 selected, ECAM alerted CAUTION FLAPS LOCKED. I performed a go around, completed ECAM actions and QRH follow up. I declared an emergency out of abundance of caution, requested the longer runway and landed without further problem. This aircraft apparently has recent history of flap problems.

Synopsis

An A320 pilot reported a CAUTION FLAPS LOCKED ECAM message as flaps 2 was selected. A go around was executed, an emergency declared and the aircraft returned for an uneventful landing.

ACN: 874742 (2871 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : DFW.Airport
State Reference : TX

Aircraft : 1

Reference : X
ATC / Advisory.Center : ZFW
ATC / Advisory.Tower : DFW
ATC / Advisory.TRACON : D10
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR

Aircraft : 2

Reference : Y
ATC / Advisory.Center : ZFW
ATC / Advisory.Tower : DFW
ATC / Advisory.TRACON : D10
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR

Person : 1

Reference : 1
Location Of Person.Facility : D10.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 874742
Human Factors : Situational Awareness

Person : 2

Reference : 2
Location Of Person.Facility : DFW.Tower
Reporter Organization : Government
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 874746

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

I was conducting OJT (on job training) on AR1/AR2 combined. OJT was also being conducted on AR3. DFW was running simultaneous dependent ILS approaches to Runways 36L/35R using procedures defined in our local wavier. AR3 had coordinated to turn their arrival in early and Aircraft X was to be cleared for approach dependent to AR3's aircraft. The AR3 Controller was late turning their traffic in. We turned Aircraft X to final and maintained altitude separation until AR3's aircraft was established on final. We then cleared Aircraft X for ILS Runway 35R approach but that aircraft was above glide slope. They descended to glide slope but never slowed the aircraft. We transferred communications to DFW Tower. DFW Tower advised us they were not talking to Aircraft X. We attempted communications transfer again. Aircraft X advised on our frequency they were going around. We switched Aircraft X to DFW Tower again for instructions. We verified Aircraft X was on Tower frequency. Aircraft X called us again over the departure end of the runway again. I observed traffic to their left that had departed Runway 35C. I issued traffic and a turn to Aircraft X. I instructed developmental to hand off and switch Aircraft X to the appropriate controller.

Narrative: 2

I was working LE 1 north flow. I cleared Aircraft Y for take off NOBLY (RNAV) Runway 35L. I cleared Aircraft X (in the blind) to land Runway 35R. I heard LE3 YELL "Aircraft X is going around." I observed Aircraft X climbing out of 2,300 FT, Aircraft Y was climbing out of 1,700 FT, I asked if he could stop at 2,000 FT. Aircraft Y said he would try and did a good job and stopped at 2,000 FT. I observed Aircraft X descending back to 2,000 FT. I then instructed Aircraft X to climb and maintain 10,000 FT. I observed a heavy departing the west side of the field. I observed Aircraft X climbing again. I observed Aircraft X turn right to the northeast I shipped Aircraft Y to departure about 4 north. Aircraft X was never on my frequency.

Synopsis

D10 Controller described go around event when traffic on parallel approach to Runway 35R did not respond to repeated calls from ATC, elected to initiate a go around that resulted in considerable confusion between the tower and the approach controllers.

ACN: 874349 (2872 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 39000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Pneumatic System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 23000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 3000
ASRS Report Number.Accession Number : 874349
Human Factors : Troubleshooting
Human Factors : Communication Breakdown
Human Factors : Training / Qualification
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Diverted
Result.Flight Crew : Overcame Equipment Problem
Result.Flight Crew : Landed As Precaution
Result.Air Traffic Control : Issued New Clearance
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At cruise I was working with Dispatch and Maintenance Control on an ACARS issue when the Captain and I noticed our ears popping. The Captain looked at the ECAM and noticed one of the bleeds had gone away and pulled up the bleed page on the System Display. I put down the books and turned my attention to the issue and noticed both engine bleeds had closed, both packs had shut down and the pneumatic cross bleed was closing. The Captain called for Masks On and Emergency Descent Checklist. We had difficulty in communicating with ATC and so began our descent, turned the seat belt sign on, and finally got clearance to a lower altitude. The Checklist was finished, we double checked the ECAM items, and called the Flight Attendants. During the descent between FL250 and about FL200, I reset the bleeds, the packs, and shut the pneumatic cross feed based on system knowledge. The cabin started to stabilize somewhere around FL200-15000 ft, and was stable before reaching 10000 ft. The Captain was talking to Dispatch about a divert field and two options were offered. He and I discussed both fields and we concluded that ZZZ would be better due to crew familiarity with the weather there and more recent operations. The flight continued with no further issues to ZZZ.

Synopsis

A320 First Officer at FL390 detects cabin pressure change and discovers both engine bleed valves have closed. The Captain initiates an emergency descent and the First Officer is able to reestablish bleeds and pack operation passing FL200. Flight crew elects to divert to a suitable alternate.

ACN: 873365 (2873 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 9000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 1050
ASRS Report Number.Accession Number : 873365

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition
Result.Air Traffic Control : Provided Assistance

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

The First Officer disengaged the autopilot and made the turn toward final and used a little speed brake to aid the descent. At that time we got an ECAM for Green System low hydraulic quantity. I accomplished the ECAM by turning off the PTU and the green (#1) system engine pump. This gives you a second ECAM Green Sys Low Press. I decided it best for me to fly the aircraft and coordinate with ATC while the First Officer worked the ECAM. He also referenced the flight manual for any notes/warnings/cautions. I declared an emergency with ATC and notified the company. I also asked for some delay vectors to give us a little more time to work the problem. We had about 4600 lbs of fuel at this point so we didn't have a lot of extra time to work with. I gave a cabin advisory to the Lead Flight Attendant and since we had a deadheading Airbus Captain on board I had him come to the cockpit to back us up on anything we might have overlooked or anything else we might consider. Satisfied, we told ATC we were ready for the approach. The landing gear free fell as advertised and we landed uneventfully. I was able to make the turn off the runway, but with no nosewheel steering stopped clear of the runway. We started the APU, shut down the engines, let the fire department check the aircraft and, given the okay, were towed to the gate.

Synopsis

An A320 suffered the loss of the green hydraulic system. The flight crew complied with the appropriate checklists, declared an emergency and landed at their destination.

ACN: 873356 (2874 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class B : ZZZ

Component

Aircraft Component : Flap Control (Trailing & Leading Edge)
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 21800
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 3045
ASRS Report Number.Accession Number : 873356

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 6500
Experience.Flight Crew.Last 90 Days : 148
Experience.Flight Crew.Type : 2660
ASRS Report Number.Accession Number : 873363

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Diverted
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During approach to our destination, a "Flight Control Flap Lock" ECAM annunciated during flap handle movement from position one to two. Delay vectors were provided as the issue was clarified via ECAM and irregular procedure. Ultimately no further flap extension was possible. Verbal contact was initiated with Dispatch and a divert was coordinated, accounting for proximity and runway length. An emergency was declared with ATC, the Flight Attendants were advised, and passengers were briefed. An uneventful partial-flap landing was made at a suitable nearby airport with emergency equipment standing by.

Synopsis

An A320 flight crew diverted to a nearby airport with longer runways when the flaps failed to extend beyond position one during approach to their destination.

ACN: 872381 (2875 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Inspection
Maintenance Status.Maintenance Items Involved : Installation

Person

Reference : 1
Location Of Person : Hangar / Base
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Airframe
Experience.Maintenance.Technician : 17
ASRS Report Number.Accession Number : 872381
Human Factors : Troubleshooting
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Deviation / Discrepancy - Procedural : Maintenance
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

In the middle of my night shift, the Maintenance Control Manager told the Maintenance Controllers working the Airbus desk that we had an incident in ZZZ. We were told that another airline's winglet struck our A320 wing tip fence and a Mechanic would call for a Configuration Deviation List (CDL). I received the call early morning for the CDL. I asked the Mechanic if they had accomplished speed taping the wing tip per the Maintenance procedures. He said they accomplished the Maintenance procedures so I issued the CDL. 

I did not ask the Mechanic directly if they accomplished a Wing Tip Impact Inspection. The Maintenance Control Manager had a wing tip robbed from an aircraft in Heavy Maintenance and shipped to the next Maintenance station, to be installed on the aircraft when it arrived from ZZZ. Two Managers called me at home two days later and informed me to fill out a report in regards to the CDL that I issued to the Contract Maintenance Mechanic.

Callback: 1

Reporter stated the Wing Tip Impact Inspection is required to inspect for structural damage to the wing tip area where the Wing Tip Fence is attached. After the Fence is removed, Speed tape is applied to the exposed wing tip to basically prevent moisture from entering the wing cavity. He never thought to specifically ask the Contract Mechanic to inspect the area, because he thought the area would be looked over by the Contract Mechanic after removing the damaged Fence and prior to installing the Speed tape over the area. 

Reporter stated he has seventeen years as a Mechanic and recently transferred into the Maintenance Controller's position.

Synopsis

A Maintenance Controller reports he did not specifically asked a Contract Maintenance Mechanic if he had accomplished a Wing Tip Impact Inspection on an A320 for a wing tip Fence damage prior to issuing a CDL deferral.

ACN: 872322 (2876 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Aerofoil Ice System
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 16000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 1200
ASRS Report Number.Accession Number : 872322
Human Factors : Confusion

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : Pre-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : MEL
Primary Problem : Ambiguous

Narrative: 1

Aircraft had come in from a maintenance base and remained overnight. Aircraft showed Right Wing Anti Ice valve stuck in the open position. The problem with this deferral is that it requires three ECAM procedures to operate the aircraft safely. Procedures give two options for takeoff. The first option is very poorly written and does not specify switch position for the bleed and pack valve. 

This whole deferral really needs to be evaluated for safety reasons. Why did this aircraft depart a maintenance station with this problem? Is it in the interest of safety to have the flight crew do a bleeds off takeoff and run an ECAM procedure at low altitude with mountainous terrain? The Captain put several calls in for a Fleet Captain. None were returned as the person was out of the office until Monday. It is clear our maintenance deferral procedures and our Flight Operations feel it is okay to allow crews to fly around with inoperative equipment with poorly written deferral procedures. We were required to receive runway data from the Dispatcher for specific runways. The data was sent, but nowhere on the data message showed the penalty or deferral reference. It wasn't a factor, but I'm not confident that the performance data from the Dispatcher was correct.

Synopsis

A320 Captain questioned the MEL procedures for dispatching with the Right Wing Anti-ice Valve stuck in the open position.

ACN: 872273 (2877 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 1500

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Climb
Airspace.Class B : ZZZ

Component : 1

Aircraft Component : AC Generation
Aircraft Reference : X
Problem : Failed

Component : 2

Aircraft Component : FAC (Flight Augmentation Computer)
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 25000
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 5000
ASRS Report Number.Accession Number : 872273
Human Factors : Other / Unknown
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

Early morning takeoff, dark, overcast. Captain was flying the aircraft in a right climbing turn after takeoff. Generator 1 and FAC 1 failed, causing the Captain's instruments to go blank. First Officer immediately put hand back on the control stick to take control but Captain immediately stated he was flying and First Officer to do the ECAM, and then he states we have a generator 1 failure. Instead of transferring control of the aircraft to the pilot who has the only good instruments in front of him, the Captain wants to maintain a right climbing turn by looking six feet across the cockpit at an attitude indicator the size of an orange, and then he takes his eyes off the instruments to look at the ECAM screen! Given the circumstances of low altitude with this guy wanting to perform a maneuver that is mind boggling the First officer kept his hand on the control stick ready to push the takeover command button and waited until wings level before accomplishing the ECAM.

Callback: 1

After the wings were level and the aircraft established in a climb, the ECAM actions were accomplished. The generator and FAC were reset and the flight continued to destination with no further problems.

Synopsis

An A320 Captain elected to continue flying after a generator and instrument failure. The First Officer questioned this choice and prepared himself to take over flight control until stable flight was established before he performed the required ECAM actions.

ACN: 872220 (2878 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Icing

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Climb
Flight Phase : Parked
Flight Phase : Cruise
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Installation

Component

Aircraft Component : Ice/Rain Protection System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 100
Experience.Flight Crew.Type : 6000
ASRS Report Number.Accession Number : 872220
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 872216
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : MEL

Narrative: 1

Aircraft was dispatched from ZZZ1 in moderate icing conditions with the Right Wing Anti-ice valve deferred. Maintenance in ZZZ deferred the valve by locking it open. Following MEL and ECAM procedures still left the aircraft with hot bleed air being sent into the right wing leading edge through the wired "Open" wing anti-ice valve. My Copilot and I were not comfortable flying for an extended period of time with a known bleed air leak on the right wing. Based on our experience we decided the safest course of action was to treat this maintenance condition as a bleed air leak. 

After takeoff and climbout from ZZZ1, in moderate icing conditions, where we used wing anti-ice ["On" for] both bleed valves, we isolated the right wing. Number 2 bleed - Off, Cross bleed valve -Closed, and Pack 2 -Off (although it would be off anyway with no bleed air on the right wing). We flew to ZZZ2 at FL310 (single pack altitude) and we added fuel in ZZZ1 to account for the lower altitude. 

I feel that the MEL should be readdressed to determine the danger of flying around with a manufactured bleed air leak and just allowing the Bleed Monitoring Computer (BMCs) to monitor and detect an overheat after the damage has already been done. A bleed air leak is one of the most dangerous emergencies that can occur and could possibly lead to a wing fire. I feel that we operated the aircraft in the safest way possible with the Right Wing Anti-Ice valve wired open.

Narrative: 2

Aircraft was dispatched from ZZZ to ZZZ1 with Wing Anti-ice valve stuck open. Aircraft arrived with significant ice on leading edges of all lifting surfaces as well as significant icing under wing area between engine and hull. Our leg-ZZZ1 to ZZZ2. The MEL did not address anything other than engine start, taxi, and takeoff. MEL should have addressed cruise configurations as well. 

We decided the only way we'd take aircraft was to isolate the problem and prevent bleed air from entering the wing while flying. During climbout, above icing conditions, we shut the crossbleed, turned off engine bleed 2, turned off Pack 2, cruised to ZZZ2 at FL 10 single pack, with no bleed air on the wing. We left the APU on for added safety precaution. There was a SERIOUS judgment error by the Routers, as well as Dispatch, to send this aircraft into known moderate icing. MEL should address specific configuration at all phases of flight. Simply because something CAN be done, does not mean it SHOULD be done.

Synopsis

A Captain and Copilot report about their A320 being dispatched with the Right Wing Anti-ice valve deferred in the wired "Open" position. The MEL allegedly did not address anything other than Engine Start, Taxi and Takeoff.

ACN: 872018 (2879 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : SAT.Airport
State Reference : TX

Environment

Flight Conditions : IMC
Weather Elements / Visibility : Rain
Light : Night

Aircraft

Reference : X
ATC / Advisory.TRACON : SAT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach

Component

Aircraft Component : FMS/FMC
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 1000
ASRS Report Number.Accession Number : 872018

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach
Result.Flight Crew : Overcame Equipment Problem

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

FMGC was auto tuning and auto tuned the wrong localizer frequency (109.9 vs 110.90). Runway 12R was correct and the box was current. Did a go around and had to manually tune the localizer on the RAD NAV page. Weather was moderate rain cells and night IFR. We needed an approach and yet the automation failed us.

Synopsis

A320 pilot reports the FMGC auto tuning the wrong ILS frequency for the ILS Runway 12R at SAT. The correct frequency is then manually tuned on the RAD NAV page and a normal landing ensues.

ACN: 871896 (2880 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 1000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.Tower : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use.Localizer/Glideslope/ILS : Runway 28
Flight Phase : Final Approach
Airspace.Class B : ORD

Component

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Design
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 150
Experience.Flight Crew.Type : 6420
ASRS Report Number.Accession Number : 871896

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 200
Experience.Flight Crew.Type : 6500
ASRS Report Number.Accession Number : 871883

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.ATC Issue : All Types
Anomaly.Inflight Event / Encounter : CFTT / CFIT
Detector.Automation : Aircraft Terrain Warning
Detector.Automation : Air Traffic Control
Were Passengers Involved In Event : N
When Detected : In-flight
Result.Aircraft : Equipment Problem Dissipated

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Environment - Non Weather Related

Narrative: 1

We were established normally, on profile, and cleared for the ILS Runway 28 approach at ORD. Inside the final approach fix the glide-slope appeared to become intermittently erratic. We subsequently received a low altitude alert from the Control Tower, along with a glide-slope alert at approximately 1,000 FT AGL. At this point we had the runway environment in sight and continued to an uneventful landing. The Control Tower indicated that a heavy aircraft had been taxiing past the ILS transmitters.

Narrative: 2

This happened at 1,000 AGL and we got the ground in view at the same time. The weather was 1,000 FT overcast and visibility 5 NM.

Synopsis

Just as they broke out of the overcast, the flight crew of an A320 received glide slope warning and an ATC generated low altitude alert due to a heavy jet taxiing through the ILS glideslope transmitter signal. The flight continued to an uneventful landing.

ACN: 871837 (2881 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Weather Elements / Visibility : Rain
Light : Daylight

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : N
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair

Component

Aircraft Component : Aircraft Logbook(s)
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person : Gate / Ramp / Line
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Maintenance : Lead Technician
Qualification.Maintenance : Powerplant
Qualification.Maintenance : Inspection Authority
Qualification.Maintenance : Airframe
Experience.Maintenance.Lead Technician : 5
Experience.Maintenance.Technician : 20
ASRS Report Number.Accession Number : 871837
Human Factors : Situational Awareness
Human Factors : Communication Breakdown
Human Factors : Distraction
Human Factors : Time Pressure
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Maintenance
Were Passengers Involved In Event : N
When Detected.Other 
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Logbook Entry
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Weather
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Human Factors

Narrative: 1

February 2010, Mechanic informed me that a gate call from one of our A320 aircraft was in progress. Captain logbook discrepancy, 'Green Hydraulic System Over Serviced.' When in the cockpit, the ECAM screen on the hydraulic page indicated the arrow at the top, but not above the spart of the square. I do not know how long the aircraft was on the ground, but the ground baggage crew was in process of loading the baggage and utilizing the yellow system. 

I recall from my weekly and overnight Check work that the Airbus should sit on the ground with (Hydraulic) pumps "Off" for 1 hour before an accurate reading can be achieved. As this was not the situation, I signed-off the log page discrepancy 'Green Arrow in Range with Hydraulic Pumps On.' This was a technical and administrative error on my part. 

The weather was raining cats and dogs that day. We had a down aircraft and the AOG (aircraft on ground) parts had been misrouted twice. Numerous other calls, escort, etc. There were only two Mechanics on duty; me and another guy. I entered the above data on the logbook white copy, ATA, station corrected and date, but on the copy that stays in the logbook, I omitted the station, ATA code and corrective action.

Callback: 1

Reporter stated he had already pulled the white top page from the logbook and had returned to the maintenance office to enter the logbook data into their maintenance computer system. While doing the data entry he noticed he had not filled out the ATA chapter code and the "Y" for "YES" in the corrective action section of the top page he had removed from the logbook. He added the missed data to the top sheet and completed the entry in their computer. He completely forgot to go back to the gate and add the same information to the page that remained with the logbook. Apparently the Captain didn't notice the sign-off discrepancy either.

Reporter stated when he turned on the Electric Hydraulic pumps to check the "Green" hydraulic system fluid level, the Captain noted the pump procedure was not in their Pilot's Handbook when a question of fluid level comes up.

Synopsis

A lead line mechanic forgot to return to the gate to enter additional information on the remaining logbook page of an A320 aircraft with a "Green" Hydraulic system fluid level discrepancy after he had already pulled the top page.

ACN: 871542 (2882 of 2908)

Time / Day

Date : 201002
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : LAS.Tower
State Reference : NV
Altitude.AGL.Single Value : 0

Aircraft : 1

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi
Route In Use : None

Aircraft : 2

Reference : X
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : Commercial Fixed Wing
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Taxi
Route In Use : None

Aircraft : 3

Reference : Z
ATC / Advisory.Tower : LAS
Aircraft Operator : Air Carrier
Make Model Name : B767 Undifferentiated or Other Model
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Taxi

Person

Reference : 1
Location Of Person.Facility : LAS.Tower
Reporter Organization : Government
Function.Air Traffic Control : Flight Data / Clearance Delivery
Function.Air Traffic Control : Local
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 871542
Human Factors : Situational Awareness
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Ground Conflict, Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Human Factors

Narrative: 1

Air Carrier XXX1 was #1 for departure, I thought. I cleared him for take off, and he replied that he was number 5 in line. The #1 aircraft was actually Air Carrier XXX2. I had gotten the call sign wrong on initial taxi out, and the pilot of Air Carrier XXX2 did not correct me. I canceled take off clearance for Air Carrier XXX1, and issued it to Air Carrier XXX2. I then asked Air Carrier XXX1 his position, and he said "Spot 4". I looked at Spot 4, and saw a Air Carrier X aircraft that was near the gate, still quite a ways from Spot 4, and a B767 parked in the holding pad. I had Air Carrier YYY1 on Taxiway C, approaching right to left but he was no factor for the aircraft I saw. I told Air Carrier XXX2 to taxi via C to 25R, and went on to the next item. A few seconds later, a Air Carrier X aircraft asked who this guy was off his right, and that he was close. I looked back at Spot 4, and saw both aircraft that I had seen before, neither having moved. I also saw a 3rd aircraft, that had apparently been on the other side of the B767, out of my sight. He was much closer to the exit point than the aircraft I had seen. Had I seen him when I issued the clearance, I would have told the Air Carrier X aircraft to give way to the Air Carrier Y aircraft. The Air Carrier X aircraft did give way, but the aircraft should not have been that close. Recommendation, there are 2 lights on the holding pad in front of the D concourse, and lights on the western portion of the ramp by this concourse. The ramp on the other side of the holding pad is unlit. With the heavy parked in the pad, I was unable to see the ramp on the other side of the parked plane. Lighting this area would be of help. Even with no planes parked there, at night it is sometimes impossible to tell what aircraft is the one calling, unless they are in close proximity to the holding pad and its lights, or they have their own lights on. I did not see any lights on the Air Carrier X aircraft outbound, so am assuming it had none on when he called for taxi.

Synopsis

LAS Local Controller described a confused aircraft identity event when several aircraft awaiting departure were confused by the controller, an incorrect take off clearance issued and a related ground conflict developed.

ACN: 871228 (2883 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0001-0600

Place

Locale Reference.Airport : ZZZZ.Airport
State Reference : FO
Altitude.AGL.Single Value : 0

Environment

Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Oil Pressure Indication
Aircraft Reference : X
Problem : Failed

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 21000
Experience.Flight Crew.Type : 7000
ASRS Report Number.Accession Number : 871228

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Person : Flight Crew
When Detected : In-flight
When Detected : Taxi
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I reported to the plane for my flight to SFO and read all the paperwork and noted that the #1 eng reverser was inoperative and secured for this segment. Read and was aware of all the penalties associated with this deferment. We were a little late to push due to the security lines at the airport. Upon push back we started the #1 eng--normal start with the exception of "no oil pressure gauge" all xxx's in the gauge. Apparently I overlooked this circuit breaker (n40) being popped on my cockpit setup. After airplane movement stopped and brakes set, I pushed in this CB. The gauge came back, started #2, taxied and took off. Several minutes after take off, this gauge went to xxx's again. I looked back and this same CB (n40) had popped. We sent a message to Maintenance relaying this information and also our dispatcher and asked if we should drop down and have this fixed. This dispatcher notified us that he was on a conference call with maintenance and the flight ops duty manager and will let us know. Several minutes passed and then the dispatcher established radio contact with us. All involved parties agreed that the primary method of determining if there was no oil pressure was through the ECAM system which was a separate system than the gauge itself. We were all comfortable with the decision to continue the flt to SFO. Uneventful landing and taxi to gate. I did paperwork for my next assignment. As this plane was still at the gate, I went aboard and spoke to the new cockpit crew and told them the what happened. I then went outside and spoke with the mechanics working on the engine. It turns out that a wire was chafing that caused our problem. Wire was replaced and flight left.

Synopsis

A320 Captain discovers inoperative oil pressure gauge after first engine start and a tripped circuit breaker. Breaker is reset and flight departs. Enroute breaker trips again and is left out. After discussion with maintenance flight continues to destination where it is discovered that a chaffed wire was causing the problem.

ACN: 871201 (2884 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1201-1800

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 39000

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Route In Use : Oceanic
Route In Use.Airway : A509
Airspace.Class A : ZHU

Person

Reference : 1
Location Of Person.Facility : ZHU.ARTCC
Reporter Organization : Government
Function.Air Traffic Control : Handoff / Assist
Function.Air Traffic Control : Oceanic
Function.Air Traffic Control : Enroute
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 871201
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Human Factors : Situational Awareness
Human Factors : Workload
Communication Breakdown.Party1 : ATC
Communication Breakdown.Party2 : ATC
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Completed

Events

Anomaly.Airspace Violation : All Types
Anomaly.ATC Issue : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
When Detected : In-flight
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : ATC Equipment / Nav Facility / Buildings
Contributing Factors / Situations : Human Factors
Primary Problem : ATC Equipment / Nav Facility / Buildings

Narrative: 1

I began splitting off Sector 72 at about XA30Z. I received separate briefings from the radar and radar associate. The radar gave me briefings about the aircraft that were displayed on the scope, as well as weather, military, etc. The radar associate then brought his strips over to my sector. He gave me about four strips to lay flat in the board, saying they were already taken care of which meant they were separated and coordinated. The rest of the strips maybe about eight to ten of them he said he hadn't had a chance to look at because they were some time out from the sector. 

I took the sector at that time and immediately began taking care of the highest priority items: talking to airplanes and coordinating new altitudes and speeds with Merida(Mexico). Then I went through my bay of traffic and started sequencing new strips. When I got a few more strips into the board I started to check all of the strips against the radar display. I immediately saw that there were no position reports or estimates on Aircraft 1's strip only a possible altitude check. I looked on the scope and there was no data block either. I did a route display on the scope but no route showed up (meaning he was either still in Merida's airspace or already in Miami's). Then I looked at the MINOW time (the fix where I would begin to get radar on the aircraft). The time on the strip showed XA34Z and it was already about 10 minutes past that. 

I thought it was possible the aircraft must have already been handed off to Miami Center and shipped and that the previous controllers had simply failed to mark the strip. I then started looking for the limited data block to ensure what I thought was true. I ranged out on the scope but didn't see the limited in Miami's airspace before I had to range back in so I could talk to additional aircraft entering my sector. At this point I really thought Aircraft 1 had left my airspace because it was well past the MINOW time and I would have seen the limited had he been any closer to my sector. I left the strip in the bay, intending to pursue the matter, but I had other work I needed to do first. Then the Miami Center controller called over the shout line and told me to ship the Aircraft 1. I think I immediately swapped into Enhanced Backup Surveillance System (EBUS) and there was the limited as clear as day but in Host, nothing. Aircraft 1 was already in Miami Center's airspace. I issued the frequency change to Aircraft 1 and he responded. But he never reported the two mandatory reporting points (MINOW and EPSON). 

Fort Green Long Range Radar was out of service. It appears that the ADS-B radar site (VRE: Virtual Radar East) prevented the display of the limited target in my airspace from MINOW eastward along A509. VRE had replaced Fort Green as the primary site in a few radar sort boxes. Aircraft 1 wasn't ADSB equipped, hence no display. The data blocks we start in OCEAN EAST AND WEST that are non radar are put immediately into coast track so that the data block will 'fly' along the aircraft route. 

Recommendations: Where to begin?

1. Utilize proper strip marking procedures. Carry through time changes or cross out the inaccurate times period. The time Merida coordinated with the receiving ZHU controller was three minutes later than the time on the strip. The coordinated time was written on the first fix but the rest of the fix times (three total on one strip) were not corrected. Instead of XA34z at MINOW, the corrected time would have been XA37. I didn't notice that at first.

2. Enter the adjusted time in Host. I might have gotten a route display showing the aircraft should be at or past MINOW.

3. Obtain and write down position reports on the strip.

4. Start a data block and put it in coast track while aircraft is non-radar.

5. After receiving the individual position relief briefings from radar and radar associate, correlate the information given from each position to make sure everything coincides.

6. Have pilots comply with the requirement to report mandatory fixes while non-radar.

7. Implement a safe procedure to deal with the impact of ADS-B testing on live traffic. (This isn't the first incident with IFR traffic being 'hidden from view' because of ADSB being primary for BEACON targets in radar sort boxes.

Callback: 1

Reporter stated that ADS-B integration into HOST still has some issues. Reporter states that the long range radar outage caused ADS-B to be the primary display system in the vicinity of Aircraft 1, and that this has occurred with other non ADS-B equipped aircraft on previous occasions. Reporter states that determining the correct altitude selections and target sources in the HOST Mosaic sort boxes is an issue in these occurrences.

Synopsis

A combination of pilot failure to position report, air traffic controller coordination error, and technology issues result in an ZHU airspace deviation.

ACN: 870870 (2885 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 36000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Hydraulic System
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 9000
Experience.Flight Crew.Last 90 Days : 110
Experience.Flight Crew.Type : 360
ASRS Report Number.Accession Number : 870870

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 17
Experience.Flight Crew.Type : 17
ASRS Report Number.Accession Number : 870872

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Automation : Aircraft Other Automation
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Declared Emergency
Result.General : Maintenance Action
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

We were at FL360 when we received an ECAM G HYD SYS LOW PR. At that time I took control of the aircraft while the First Officer ran the appropriate check lists. It was determined through ECAMs and in talking with Maintenance Control and Dispatch that we had lost our Green Hydraulic System. We then declared an Emergency and ran the Appropriate Checklists for approach and landing: Landing distances required, Gravity Gear Extension, Tow in. I also had the purser do a Cabin Advisory. After briefing the Purser and passengers we prepared for landing with a brief about what to expect and when checklists would need to be run. I landed the aircraft and stopped on the runway. We were towed to the gate after the aircraft was secured by ground personnel and CFR.

Narrative: 2

At cruise, FL360, ECAM message "HYD G SYS LO PR" appeared, followed by "HYD G RSVR LO LVL". At that time, the Captain took control of the aircraft while I ran the ECAM and checklist. At the completion of the ECAM, a subsequent ECAM "G ENG 1 PUMP LO PR" illuminated. It was a result of the previous ECAM actions. As a result of the loss of Green System hydraulic fluid and no chance to recover the Green system, the checklist advised that we had lost Nose Wheel Steering and that the we would have to do a gravity extend of the landing gear. The Captain and I discussed the situation. The Captain contacted Dispatch and Maintenance Control to alert them to our condition and discuss options. At that time, considering our condition, a landing at ZZZ was our best option. The Captain declared an emergency with ATC and informed them of our decision to continue to ZZZ. The Captain contacted the #1 Flight Attendant to inform her of the situation and directed a cabin advisory. At that time I completed the Landing Distances Required performance considerations. Runway 7 was more than adequate. Other considerations for the landing were a loss of normal brakes, the number 1 reverser and some spoilers. Approach to the terminal area was normal. The emergency equipment was standing by for our arrival. On downwind, I ran the Landing Gear Gravity Extend Checklist and extended the landing gear. With the loss of the Nosewheel Steering, the decision was made to stop on the runway. Landing was normal. The Captain brought the aircraft to a stop straight ahead and set the parking brakes. We completed the after landing flows. The APU was started and the engines were shut down. I completed the Tow In checklist. Emergency personnel inspected the aircraft and cleared it for towing. Ground personnel pinned the landing gear and connected the tug. Ground cleared us to the gate.

Synopsis

An A320 flight crew declared an emergency, performed the appropriate checklists and landed safely after suffering a Green Hydraulic System failure.

ACN: 870486 (2886 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ORD.Airport
State Reference : IL
Altitude.AGL.Single Value : 0

Environment

Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Ramp : ORD
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 91
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Escape Slide
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 18000
Experience.Flight Crew.Last 90 Days : 120
Experience.Flight Crew.Type : 1500
ASRS Report Number.Accession Number : 870486
Human Factors : Other / Unknown

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Door Area
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 1000
Experience.Flight Crew.Last 90 Days : 220
Experience.Flight Crew.Type : 900
ASRS Report Number.Accession Number : 870489
Human Factors : Training / Qualification
Human Factors : Fatigue
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Flight Deck / Cabin / Aircraft Event : Other / Unknown
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Aircraft In Service At Gate
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

We had just pulled into the gate after a ferry flight. The F/O and I were the only crew on board. I had armed 1L and 1R on the ground, and had asked the F/O to disarm after we parked the aircraft. He got up from his seat and proceeded to the cabin to disarm the doors. Shortly thereafter, I heard the slide inflate, and realized we had inadvertently blown the emergency exit. I assume that he simply got confused and raised the wrong handle. I then went back and disarmed the other door. I think a factor here may have been that I armed the doors prior to departure, but asked the F/O to disarm them upon arrival. It would probably make more sense for the same person to both arm and disarm on ferry flights, since these are unfamiliar duties to us.

Narrative: 2

I blew the slide door on 1L. I reached for the wrong handle. I had early am short call. Was asleep at XA00pm night before and crew desk call to assign me an XI00am ferry flight out of IAD to ORD. I woke up at XF00am for my XH00am check in. Not a great night sleep. Normal flight and landing. Upon parking I went back to disarm door and pulled the wrong handle.

Synopsis

A320 flight crew experiences an inadvertent slide deployment when the First Officer attempts to disarm the 1L slide after a ferry flight.

ACN: 870139 (2887 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 17000

Environment

Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZZZ

Component

Aircraft Component : Cooling Fan, any cooling fan
Aircraft Reference : X
Problem : Failed
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 870139
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 11000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 5500
ASRS Report Number.Accession Number : 870368

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Flight Deck / Cabin / Aircraft Event : Smoke / Fire / Fumes / Odor
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Declared Emergency
Result.Flight Crew : Landed in Emergency Condition

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

At a cruise altitude of FL380 I noticed a slight humming sound coming from behind the First Officer seat. I called the A Flight Attendant located in the forward galley to ask if she felt anything, her comments were, "I hear the sound and there is a vibration on the floor when I stand in front of the forward galley". I notified Dispatch via ACARS of the humming noise,and sent Maintenance a log item via ACARS. On descent the humming noise increased and an electrical smell was noticed for the first time in the cockpit. Called the Flight Attendant again to ask if she smelled anything unusual, her reply was: "I don't smell anything back here". There was never any smoke present associated with the noise or electrical smell. After switching to Approach Control I declared an emergency with ATC, requested priority handling and fire rescue equipment standing, we got cleared for the visual approach. In the mean time, I called the A Flight Attendant again to inform her of the emergency and directed her to conduct a Cabin Advisory. Upon landing, we requested an outside visual inspection of the airplane for possible smoke or heat signature, none was noted by the fire personnel. We taxied to the gate followed closely by firefighting equipment. Enroute to the gate, the noise increased as did the electrical smell until a CB tripped and the noise stopped. Upon arrival at the gate, we where met by firefighting personnel who found no heat signatures in the cockpit. After that the passengers deplaned normally.

Synopsis

A320 flight crew reports slight vibration in the cockpit floor and electrical fumes during descent. An emergency is declared and normal landing ensues. During taxi-in the vibration increases then stops at the sound of a circuit breaker tripping.

ACN: 870137 (2888 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.ARTCC
State Reference : US
Altitude.MSL.Single Value : 17000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.Center : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Initial Approach
Airspace.Class E : ZZZ

Component

Aircraft Component : Flap/Slat Control System
Aircraft Reference : X
Problem : Improperly Operated

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13800
Experience.Flight Crew.Last 90 Days : 210
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 870137
Human Factors : Troubleshooting

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Flying
Experience.Flight Crew.Total : 10500
Experience.Flight Crew.Last 90 Days : 115
Experience.Flight Crew.Type : 115
ASRS Report Number.Accession Number : 870143
Human Factors : Human-Machine Interface
Human Factors : Confusion
Human Factors : Situational Awareness

Events

Anomaly.Deviation - Speed : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft Other Automation
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Became Reoriented

Assessments

Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

At approximately 17,000 feet and 280kts I heard the flap overspeed warning horn going off. The First Officer inadvertently moved the flap handle thinking it was the speed brake handle. I immediately moved the flap handle back to zero. The warning horn went off for no more than 5 seconds so I know the slats didn't fully extend. After regrouping, we continued the approach and landing uneventfully. I sent an overspeed write up in to Maintenance Control. After the flight I also called Dispatch to inform them what had happened. Dispatch and I also talked to Maintenance Control on the phone to make sure if any other action was required after the flight. Maintenance Control said no. The First Officer is new to the airplane and mistook the flap handle for the speed brake handle.

Narrative: 2

As First Officer, I was the Flying Pilot. We had been taken off the STAR and given direct to the outer marker prior to our descent. Then, we were assigned to cross 30NM west of the outer marker at 11,000 feet. As we were approaching near the top of descent, I was heads down in the MCDU trying to create a fix for situational reference, however, this became a major distraction. The waypoint I had created was not accurate and I started down in the descent early for our crossing restriction. Believing that I might be high, I pulled out what I thought were the speedbrakes when in actuality it was the flap lever. The speed at the time was 280kts at at altitude of 17,000 feet. Instantly, ECAM alert and master warning ocurred. The Captain immediately diagnosed the problem and had me stow the flaps back to zero. The flap overspeed warning sounded for 5 seconds or less. Fatigue was not a factor. You can have a quick mind but in aviation but need hands slow as molasses. Subsequent landing was uneventful.

Synopsis

A320 First Officer inadvertently grabs flap handle to extend speed brakes at 17000 feet and 280 knots. Over speed warning sounds and flaps are quickly retracted.

ACN: 870136 (2889 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Light : Dusk
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Parking Brake
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 13000
Experience.Flight Crew.Last 90 Days : 250
Experience.Flight Crew.Type : 6200
ASRS Report Number.Accession Number : 870136
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Workload

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : Aircraft In Service At Gate
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Procedure
Primary Problem : Ambiguous

Narrative: 1

We arrived at gate. The Captain set the parking brake and announced brakes set. While turning the APU generator on and the number 2 generator back on I noticed the aircraft appeared to be moving backwards. At the same moment I was announcing the aircraft was moving the Captain stopped the movement with the toe brakes. We rolled back approximately 4 FT. We both looked at the parking brake switch and it was on. The Captain reset the parking brake, checked the pressure and the brakes reset. No incident occurred from the aircraft rolling backwards. This was the first time I have seen this happen in my 6,000 plus hours on the Airbus. Maintenance was advised of the problem.

Synopsis

An A320 rolled backwards at the gate after the parking brake apparently failed to set properly.

ACN: 869702 (2890 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : OAK.Airport
State Reference : CA
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : IMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Route In Use : Vectors
Airspace.Class E : NCT

Component

Aircraft Component : GPWS
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Experience.Flight Crew.Total : 12500
Experience.Flight Crew.Last 90 Days : 180
Experience.Flight Crew.Type : 3200
ASRS Report Number.Accession Number : 869702
Human Factors : Confusion
Human Factors : Training / Qualification

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Captain
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 10000
Experience.Flight Crew.Last 90 Days : 160
Experience.Flight Crew.Type : 4000
ASRS Report Number.Accession Number : 869571

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Detector.Automation : Aircraft Terrain Warning
When Detected : In-flight
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Environment - Non Weather Related
Primary Problem : Aircraft

Narrative: 1

I was the Flying Pilot. Shortly after crossing the ECA VOR, we were given radar vectors off of the arrival, and a descent clearance to 8000 feet. The vectors took us directly over the Byron (identifier C83) airport, and generally toward Mt. Diablo. We then entered IMC. As briefed in the approach briefing, the MSA for our sector was 5100 feet, with Mt. Diablo at an elevation of +-3900 feet. We were cleared and descended to 5000 ft, and began to slow from 250 KIAS to 200 KIAS as we were approaching the SFO Class B airspace boundary. Both the Captain and I had our NAV Displays set to Weather Radar as we entered into light rain showers. I temporarily set my NAV Display back to Terrain to see if it would "paint" Mt. Diablo, (which it did not), and then switched back to Weather. Shortly after switching back to Weather, we received a GPWS "Terrain, Terrain, Pull-up, Pull-up." By the second "Pull-up", I had initiated the GPWS Terrain escape maneuver (as outlined in the Flight Manual), and concurrently the Captain advised NorCal Approach that we were climbing due to the GPWS Terrain warning. The warning ceased almost immediately after the initiation of the climb. NorCal Approach re-cleared us, when able, to descend to 4000 feet. Recovery from the climb started at about 6000 feet, and (to my recollection) never exceeded 7000 feet, at which point we began a slow descent back down to 4000 feet. During the descent back to 4000 feet, I re-engaged the autopilot and autothrust. At this point we were approximately on the OAK VOR 015 at 15DME. We broke out into VMC shortly thereafter, continued to get vectors onto the localizer for the OAK ILS 11, and made an uneventful visual approach and landing. The hardest part of the whole event, was trying not to keep thinking about what just happened (GPWS Warning), but to focus on what still needed to be done (approach and landing). The Captain did a great job of communicating with ATC, verifying our clearances and status of aircraft configuration with regard to autofight, position etc. with me. There was no other mention of the event by ATC.

Narrative: 2

Clean configuration. Without warning, we received a "Too Low, Terrain. Terrain, Terrain, Pull Up, Pull Up". There was no pop-up feature on the ND. The First Officer immediately performed the escape maneuver, and I told ATC that we were climbing for a terrain warning. The terrain warning ceased climbing through around 6500 feet, and we leveled close to 7000 feet. ATC cleared us back down to 4000 feet when able, and we began a gradual descent back down to that altitude, with the First Officer's terrain function on.

Synopsis

A320 Flight Crew experiences a GPWS terrain warning at 5000 feet during vectors to OAK. The Crew takes evasive action and informs ATC.

ACN: 869244 (2891 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.ATC Facility : ZHU.ARTCC
State Reference : TX
Altitude.MSL.Single Value : 39000

Environment

Flight Conditions : VMC
Weather Elements / Visibility : Icing
Light : Daylight

Aircraft

Reference : X
ATC / Advisory.Center : ZHU
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Cruise
Airspace.Class A : ZHU

Component

Aircraft Component : Pneumatic Valve/Bleed Valve
Aircraft Reference : X
Problem : Failed

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 869244

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 869599
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Ground Personnel
Communication Breakdown.Party2 : Dispatch

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : Other / Unknown
Anomaly.Inflight Event / Encounter : Weather / Turbulence
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Diverted

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Manuals
Contributing Factors / Situations : Weather
Primary Problem : Aircraft

Narrative: 1

Aircraft generated ECAM AIR ENG 2 Bleed Fault. Crew performed ECAM actions. Next notified Company Dispatch (via ACARS) and reviewed AOM for further guidance. After review of AOM procedures to see if affected item required any special procedures. There was one which would affect our flight. Do not operate into known or forecast icing conditions. In addition, the procedures required that we descend to FL310 and avoid icing conditions. We contacted ATC and requested descent to FL 310 and contacted company to advise about the MEL. This is were the crew and company saw things differently. The Captain established a phone patch to speed up communications and express our concerns regarding the inop ENG 2 Bleed Air. Our concerns regarding MEL requirements of known icing and FL 310 required diverting to an airport with no known/forecast icing, however, the company's repeated view of the no MEL situation on the aircraft and no restrictions from the AOM allowed us to continue to our ice impacted destination at FL 390. 

This discussion lasted while we continued northbound toward our original destination. I, being the Flying Pilot, handled the aircraft and communications with ATC, while the Captain communicated with the company. He provided operating manual references which state: If any instrument or item of equipment required for the particular operation becomes inoperative en route, the Captain shall comply with the approved procedures for such occurrence as specified elsewhere in this manual, the AOM, applicable MEL's/CDL's, and any pertinent FAR's. The Captain and I discussed the issue at length, and made the decision to divert to an airport with no known icing due to inop AIR ENG 2 Bleed Fault and MEL restrictions, which we did. This maintenance diversion caused the crew to exceed eight hours within 24 hours which required 18 hours rest and removal from the next day's trip.

Narrative: 2

It is the opinion of this Captain and crew that these events were precipitated by management's decision to continue the flight without regards to the FAR's, safety of operation, and/or adherence to company issued manuals. This crew was PUSHED repeatedly to continue to our destination without regards for safety. When Dispatch and or management disagree with a Captain in regards to the safe operation of an aircraft, the safest course of action should always be taken without the extreme pressure that this crew encountered.

Synopsis

After suffering the loss of an engine bleed an A320 Flight Crew encountered resistance from Maintenance and Management regarding their decision to divert to a station with no known icing as required by the company's operating manuals.

ACN: 868954 (2892 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Flight Conditions : VMC
Weather Elements / Visibility.Visibility : 10
Light : Daylight

Aircraft

ATC / Advisory.Ground : ZZZ
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Taxi
Maintenance Status.Released For Service : N
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Repair
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Aileron
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Hydraulic System
Manufacturer : Airbus
Aircraft Reference : X

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 6622
Experience.Flight Crew.Last 90 Days : 89
Experience.Flight Crew.Type : 1058
ASRS Report Number.Accession Number : 868954
Human Factors : Confusion
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Taxi
Result.General : Flight Cancelled / Delayed
Result.General : Maintenance Action
Result.Flight Crew : Returned To Gate

Assessments

Contributing Factors / Situations : Aircraft
Primary Problem : Aircraft

Narrative: 1

During the taxi-out to the runway, after second engine was started, we performed the flight control check. I noticed when either sidestick was deflected full right, the left aileron deflection was only 1/2 down. There were no other indications of a flight control malfunction. I advised ZZZ Ground that we needed to run a checklist and required more time prior to departure. The Captain then rechecked the flight controls using his sidestick and observed the same results. The Captain set the parking brake, assigned control to me, and called Maintenance on his cell phone. Maintenance instructed us to return to the gate. I told ZZZ Ground we were safe, but needed to return to the gate. Several hours later we swapped aircraft and continued without incident. The left aileron outboard actuator was malfunctioning.

Callback: 1

Reporter stated the left aileron indicator on the ECAM Flight Controls page would show full up deflection, but would not indicate full down. When they returned to the gate, their mechanics started trouble shooting by isolating each hydraulic system to see if there would be any change in deflection of the left aileron. None were noted so they opened a lower wing panel and noticed the outboard actuator rod inner section had separated, including a large ring that was part of the connection.

Reporter stated the ECAM never displayed a message indicating any problem existed with the outboard actuator. The only indication of any aileron travel discrepancy was the ECAM page showing that the left aileron did not indicate full down deflection.

Reporter also stated that some Airbus Pilots have developed a rule where the sidestick is deflected and held for 3-seconds before deflecting it to another position. The belief is that if a fault exists, that a 3-second hold will be enough time for a fault to be noted on the ECAM.

Synopsis

An Airbus A320 First Officer reports that during taxi-out, when either sidestick was deflected full right, the left aileron deflection was only half down as noted on the ECAM Flight Controls page. No other indication of a flight control malfunction, it was later discovered that the left aileron outboard actuator rod had separated.

ACN: 868857 (2893 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0001-0600

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA

Aircraft

Reference : X
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Flight Phase : Climb
Route In Use : Vectors

Person

Reference : 1
Location Of Person.Facility : SCT.TRACON
Reporter Organization : Government
Function.Air Traffic Control : Supervisor / CIC
ASRS Report Number.Accession Number : 868857
Human Factors : Situational Awareness

Events

Anomaly.ATC Issue : All Types
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Tower launched departure same route less than 3 miles behind previous departure. Recommendation, use diverging courses or visual seperation until separation is insured and maintain two way communication with trailing aircraft or coordinate a lower altitude with the trailing aircraft.

Synopsis

SCT Controller voiced concern regarding the minimal successive departure separation provided by a Tower Controller, suggesting visual separation and/or diverging headings should be used by tower controllers.

ACN: 868541 (2894 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : SFO.Airport
State Reference : CA
Altitude.MSL.Single Value : 6000

Environment

Flight Conditions : VMC

Aircraft

Reference : X
ATC / Advisory.TRACON : NCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Nav In Use : FMS Or FMC
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class B : SFO

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 8000
Experience.Flight Crew.Last 90 Days : 25
Experience.Flight Crew.Type : 120
ASRS Report Number.Accession Number : 868541
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : ATC

Events

Anomaly.ATC Issue : All Types
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Executed Go Around / Missed Approach

Assessments

Contributing Factors / Situations : Airport
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Refused visual approach clearance when issued clearance to fly Visual 28R after being vectored to 1 mile in trail of a 767 for 28L. Uneventful go around and subsequent landing executed. Following day I was advised that NORCAL wished to talk to me. Spoke to Quality Control and realized that we do not view this in the same light. I do not see the need for me to fly in trail of a heavy when operating to parallel rwys within 2500 feet of each other. I see no reason for me not to have been vectored abeam, ahead or a little in trail (1/4 to 1/2 mile) as opposed to 1 mile in trail when the turbulence reaches max levels. In addition, approach speed difference would mean I would fall further behind and more into the danger zone. I thought ATC was supposed to maintain responsibility for my heavy turbulence clearance until it issues me the visual clearance. If this is true then how could I have gotten to 1 mile in trail if they did not violate this requirement. My main reason for this report is the disagreement (cordial) between NORCAL and I as to; #1 the importance of NORCAL maintaining heavy separation for me until I accept a visual clearance; #2 NORCAL stated policy to me that putting me ahead, abeam or inside 1/2 mile in trail is not as important to them as closure rates or the fear that the 767 might pass me and consequently need to go around; and #3 their complete befuddlement at why a pilot would refuse such a clearance; quote "no other pilots have done that".

Synopsis

Unhappy with spacing behind a B767 on approach to parallel runway, an A320 Flight Crew executes go around at SFO.

ACN: 868393 (2895 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Parked

Component

Aircraft Component : Fuel Booster Pump
Aircraft Reference : X
Problem : Improperly Operated
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 868393

Person : 2

Reference : 2
Location Of Person : Gate / Ramp / Line
Reporter Organization : Air Carrier
Function.Maintenance : Technician
Human Factors : Troubleshooting
Human Factors : Confusion
Human Factors : Human-Machine Interface

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
When Detected : In-flight
Result.General : None Reported / Taken

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Human Factors
Primary Problem : Ambiguous

Narrative: 1

Flight was dispatched with #2 center tank pump inoperative, with 6500 LBS in center tank at departure. Approximately 30 minutes after takeoff, Captain requested a voice patch with Maintenance and told them that he believed the wrong center pump was deferred after not getting proper fuel flow from the #1 center pump while in "Auto Fuel Feed." Captain came to this conclusion by turning the center fuel feed to "manual" and turning on the deferred #2 pump. In this state, fuel flow from the center tank was restored normal. Maintenance agreed that the MEL was probably wrong, but made it clear that Captain could not continue to operate the #2 pump as it was officially deferred. Captain agreed to shut off #2 and keep #1 running in manual mode for duration of flight until the MEL discrepancy was corrected. I advised Captain I was filing a report as I believed the MEL to be incorrect based on what he was telling me, but nevertheless, he should not have turned on that #2 pump without exercising his emergency authority.

Synopsis

A320 Dispatcher is informed by Captain enroute that the MEL applied to the #2 center tank pump should have been applied to the #1 pump. Flight continues to destination.

ACN: 868384 (2896 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1801-2400

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Cruise
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Records Complete : N
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing
Maintenance Status.Maintenance Items Involved : Work Cards

Component : 1

Aircraft Component : Drinkable/Waste Water Syst
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Galley Furnishing
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Function.Maintenance : Technician
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 13456
Experience.Flight Crew.Last 90 Days : 275
Experience.Flight Crew.Type : 3542
ASRS Report Number.Accession Number : 868384
Human Factors : Communication Breakdown
Human Factors : Confusion
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Communication Breakdown.Party2 : Flight Crew
Analyst Callback : Attempted

Events

Anomaly.Aircraft Equipment Problem : Less Severe
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Chart Or Publication
Contributing Factors / Situations : Human Factors
Contributing Factors / Situations : MEL
Contributing Factors / Situations : Procedure
Primary Problem : Human Factors

Narrative: 1

Aircraft was presented to Crew in ZZZ with a deferred item open, stating the forward coffee maker and spigot were inop'ed at a previous Station and the deferred MEL included draining the A320 of potable water and deferring the entire water system. While reviewing the Maintenance Release, the First Officer reported the forward lavatory toilet flushed normally. Water (H20) quantity was checked and it was found to be full, in direct contrast to the MEL instructions for the deferral on the Maintenance Release. Maintenance Control was contacted and a Maintenance Report item sent. 

Contract Maintenance called to aircraft in ZZZ and he reinstated the water system as no faults could be located. A few other write-ups were handled by ZZZ Maintenance Technician and he left the aircraft, with the Deferral placard still located on the forward Cabin Intercommunication Data System (CIDS) panel. This item was not discovered until en-route to ZZZ1. Aircraft was not serviced with potable water in ZZZ, so it operated at least one leg in violation of the MEL. The ZZZ Maintenance Technician stated the aircraft appeared to be not configured correctly for the "No" potable water operation as all the valves had been left open. 

If the aircraft had a frozen water system as originally expected in the first write-up, how can one drain a water system that is frozen? Rhetorical question but that was the procedure listed under the MEL.

Synopsis

An A320 Pilot reports about a deferral of the potable water system that did not comply with the MEL procedures for their A320. Captain also asks "if the aircraft had a frozen water system as originally expected in the first write-up, how can one drain a water system that is frozen"?

ACN: 868119 (2897 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : ZZZ.TRACON
State Reference : US

Environment

Light : Night

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Phase : Descent

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Not Flying
Function.Flight Crew : First Officer
Function.Flight Crew : Check Pilot
Qualification.Flight Crew : Air Transport Pilot (ATP)
ASRS Report Number.Accession Number : 868119
Human Factors : Situational Awareness
Human Factors : Distraction
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Deviation - Altitude : Excursion From Assigned Altitude
Anomaly.Deviation / Discrepancy - Procedural : Clearance
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Anomaly.Inflight Event / Encounter : Wake Vortex Encounter
Detector.Person : Air Traffic Control
Detector.Person : Flight Crew
When Detected : In-flight
Result.Flight Crew : Returned To Clearance
Result.Air Traffic Control : Issued Advisory / Alert

Assessments

Contributing Factors / Situations : Environment - Non Weather Related
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

During descent on the RNAV arrival we encountered wake turbulence from the preceding aircraft. The Captain, without verbalizing it, decided to fly off to one side of the center of the arrival route ostensibly to avoid the wake turbulence. I noticed what he was doing and asked if he would prefer to offset to one side of the track within the RNP limits. He said no, he would manually do it to maintain his mental acuity. As soon as I turned back, I noticed and notified the Captain that we were 500 FT below the minimum altitude for the next waypoint. He immediately climbed back to the correct altitude. As we approached the correct altitude, ATC asked us for verification of the altitude. No further comments were made by ATC concerning altitude. The Captain chose to fly the aircraft off the RNAV course and refuse my recommendation to use the offset function that would have continued to abide by altitude constraints. If the Captain had notified ATC of the wake turbulence, they may have allowed an alternate descent profile which would have required less monitoring. If he had verbalized his plan, I could have spoken up and made an alternate recommendation. There are some pilots whose complacency has become so ingrained that they become at odds with SOP's and choose to rationalize making up their own. Simply line checking on short hops doesn't catch the pilot who exhibits this behavior when they are tired or trying to expedite their last leg home. Line checks need to be done much more often and on a mix of long and short legs if they are to achieve any usefulness beyond checking a regulatory box.

Synopsis

A321 First Officer reports altitude deviation by the Captain flying when attempting to hand fly off the RNAV arrival to avoid wake turbulence.

ACN: 868108 (2898 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1201-1800

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.AGL.Single Value : 0

Environment

Work Environment Factor : Temperature - Extreme
Ceiling : CLR

Aircraft

Reference : X
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Mission : Passenger
Flight Phase : Parked
Maintenance Status.Maintenance Deferred : Y
Maintenance Status.Released For Service : Y
Maintenance Status.Maintenance Type : Unscheduled Maintenance
Maintenance Status.Maintenance Items Involved : Testing

Component : 1

Aircraft Component : Air Conditioning Distribution System
Manufacturer : Airbus
Aircraft Reference : X
Problem : Malfunctioning

Component : 2

Aircraft Component : Indicating and Warning - Air Conditioning and Press.
Manufacturer : Airbus

Person

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : First Officer
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 14000
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 3800
ASRS Report Number.Accession Number : 868108
Human Factors : Training / Qualification
Human Factors : Confusion
Human Factors : Communication Breakdown
Human Factors : Situational Awareness
Communication Breakdown.Party1 : Maintenance
Communication Breakdown.Party2 : Maintenance
Analyst Callback : Completed

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Deviation / Discrepancy - Procedural : MEL / CDL
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : Pre-flight
Result.General : Maintenance Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Incorrect / Not Installed / Unavailable Part
Contributing Factors / Situations : Procedure
Contributing Factors / Situations : Staffing
Primary Problem : Company Policy

Narrative: 1

I have noticed an alarming trend with Contract Maintenance at some non-hub airports. This trend was perfectly illustrated this morning on flight from ZZZ-ZZZ1. When we arrived at the aircraft this morning the temperature was -8C (approximately 17 degrees Fahrenheit). The aircraft had been parked in a remote parking area overnight with no heat. The Operations agent reported that the APU had been started approximately 1.5 hours prior to our arrival. As I started the Cockpit setup, I noticed an ECAM: Vent Avncs Sys Fault. The Captain and I reviewed and complied with the Flight Manual Fault Reset procedure. The ECAM message remained. We sent an Maintenance Report and advised Maintenance Control who responded with three Circuit Breakers to reset. This didn't correct the problem either. Maintenance Control then dispatched Contract Maintenance.

Contract Maintenance arrived and asked what the problem was. After explaining the problem and our attempts to reset the system, the Contract Mechanic called Maintenance Control on his cell phone and we remained in the Cockpit while Maintenance Control ran the Mechanic through several system checks. Then came the MEL card (procedures) over the ACARS printer. We then watched as the Mechanic went through the deferral procedure step by step. The Mechanic had Maintenance Control stay on the phone and coach him through the entire procedure. 

The Mechanic asked us several times where switches were and whether indications were normal or not. During the 25 to 35 minutes that the Mechanic worked through the deferral steps, the horn in the Nose gear area that indicates "No" air circulation through the Avionics bay, was sounding. We brought this to the attention of the Mechanic several times. The deferral directed the Ventilation Extract fan to be selected to Override. The Mechanic, without direction from the MEL card or Maintenance Control, decided to also place the Blower switch to Override. After reading the MEL card and noticing the discrepancy, I mentioned it to the Mechanic. He responded that he was trying to get airflow over the Avionics. Returning the Blower switch to Auto eventually silenced the horn. 

The complete lack of Airbus systems knowledge demonstrated by the Contract Mechanic was disturbing to the Flight Crew. The Mechanic couldn't answer general questions about the system, let alone the specific questions that need to be answered so that we could understand how the system would function in its deferred condition. 

This begs the question, who is legally responsible for the deferral? If the Mechanic who physically completes the repair, doesn't know why he is doing what he is doing, how can he sign for the repair? If it is Maintenance Control who is signing for the repair, how is that legal when Maintenance Control is relying on an untrained (in Airbus) Mechanic's word that the steps set out in the MEL have been correctly accomplished?

Callback: 1

Reporter stated he was very upset with his carrier's use of a Contract Maintenance Mechanic who did not have adequate training or experience on the A320 he was going to fly. Even though the Contract Mechanic was suppose to be following Maintenance Control's direction, as part of the MEL deferral for the ECAM Avionic's Vent Fault, the Mechanic was also opening and closing Circuit Breakers that were not part of what Maintenance Control was directing him to do. Also, if Mechanic did not understand what he was doing, what makes Maintenance Control so sure the Contract Mechanic secured the correct fan or valve for the deferral. 

Reporter stated Maintenance Control had deferred either the Extract or Blower fan and a Circuit Breaker, to get the aircraft Released for departure; even though neither the Extract or Blower fan had an ECAM Fan fault message.

Synopsis

A First Officer questions who is legally responsible for a deferral when Maintenance Control directs a Contract Mechanic who physically completes and signs for a repair on an A320. Said Contract mechanic is reportedly untrained on the aircraft.

ACN: 867864 (2899 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 0601-1200

Place

Locale Reference.Airport : ZZZ.Airport
State Reference : US
Altitude.MSL.Single Value : 5000

Environment

Flight Conditions : VMC
Light : Daylight
Ceiling : CLR

Aircraft

Reference : X
ATC / Advisory.Tower : ZZZ1
ATC / Advisory.TRACON : ZZZ2
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Initial Approach
Airspace.Class C : ZZZ

Component

Aircraft Component : Indicating and Warning - Hydraulics
Aircraft Reference : X
Problem : Malfunctioning

Person : 1

Reference : 1
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : Pilot Flying
Function.Flight Crew : Captain
Qualification.Flight Crew : Air Transport Pilot (ATP)
Experience.Flight Crew.Total : 12000
Experience.Flight Crew.Last 90 Days : 190
Experience.Flight Crew.Type : 3500
ASRS Report Number.Accession Number : 867864
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 2

Reference : 2
Location Of Person.Aircraft : X
Location In Aircraft : Flight Deck
Reporter Organization : Air Carrier
Function.Flight Crew : First Officer
Function.Flight Crew : Pilot Not Flying
Experience.Flight Crew.Total : 14500
Experience.Flight Crew.Last 90 Days : 230
Experience.Flight Crew.Type : 8200
ASRS Report Number.Accession Number : 867865
Human Factors : Workload
Human Factors : Time Pressure
Human Factors : Distraction
Human Factors : Communication Breakdown
Human Factors : Human-Machine Interface
Communication Breakdown.Party1 : Flight Crew
Communication Breakdown.Party2 : Dispatch

Person : 3

Reference : 3
Location Of Person : Company
Reporter Organization : Air Carrier
Function.Dispatch : Dispatcher
Qualification.Dispatch : Dispatcher
ASRS Report Number.Accession Number : 867869
Human Factors : Confusion
Human Factors : Communication Breakdown
Communication Breakdown.Party1 : Dispatch
Communication Breakdown.Party2 : Flight Crew

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Conflict : Airborne Conflict
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Automation : Aircraft RA
Detector.Automation : Aircraft Other Automation
Detector.Person : Flight Crew
Were Passengers Involved In Event : N
When Detected : In-flight
Result.General : Maintenance Action
Result.Flight Crew : Diverted
Result.Flight Crew : Took Evasive Action

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airport
Contributing Factors / Situations : Company Policy
Contributing Factors / Situations : Human Factors
Primary Problem : Aircraft

Narrative: 1

I was pilot flying. We were at flaps 2 speed 180 as we rolled onto final at 15 miles and 3000 FT. In the turn we got an ECAM "HYD G RSVR LO LVL." I told the First Officer to do the ECAM checklist, I have the plane and radios. I tried to slow us down to give him time to work the problem and called for "Gear Down" and we got the "L/G GEAR NOT DOWNLOCKED" ECAM. At this point we needed some time to figure out what was going on and I told Tower we were unable to land and needed to overfly the airport to work on a hydraulic problem. We received instructions to turn to 300 degrees maintain 3000 FT, contact SOCAL. 

The First Officer and I had a quick discussion of what systems were inoperative and agreed our destination was out of the question and we should go to ZZZ as he continued working the ECAM checklist. I contacted Approach and told them we were having a hydraulic problem and need to go to ZZZ. He asked if we were an emergency aircraft, I stated not at this time but we will let him know if conditions change. I called the flight attendants and told them the nature of the problem and this was a Cabin Advisory get out your checklist and review the next step in case we run into more problems. We then got a TCAS "Traffic" alert followed by a "Climb" RA. I performed the climb maneuver until clear of conflict. 

We reviewed the checklist of inoperative items and I determined we should declare an emergency due to the fact we had to hand crank the gear down and ground steering was inoperative upon landing. As we reviewed the items we noticed we had hydraulic quantity (almost to the normal range) in the Green system that was not there when this whole ECAM process started. We turned the pump back on and had 3000 PSI on the Green system and all the ECAMs cleared from the screen. Not knowing how long it would stay pressurized I called for gear down and flaps to full to get configured for landing. We advised LAX Tower we would like the fire trucks in place as a precaution and we may not be able to turn off the runway. The landing was made in the touchdown zone and all systems operated normally. Fire crews followed us to the gate where the flight was terminated. 

Due to the close proximity of our destination to ZZZ we were able to make a quick decision to divert. We were very limited in time due to the fact it was a nice day with no weather forecast and we did not have a lot of extra fuel. The cockpit environment was very busy with radio calls, checklist, passenger and cabin crew communications. We were unable to contact Dispatch and advise them of the diversion due to the fuel/time limitations, reprogramming of boxes and checklists. We did not declare an emergency because we were able to get the airplane fully configured and made a stabilized approach. We landed at ZZZ with 3,600 pounds of fuel remaining.

Narrative: 2

Our destination was not a good option due to short runway and the potential for loss of steering and normal braking. 

It was difficult to insert the new destination due to the overflying of the original destination--FMGC box sequence, etc. Eventually completed, though took longer than normal due to lack of a "new destination" prompt. I have been on the Bus for a long time and know how to do this--but the FMGC was sequencing as we overflew our original destination and the "new destination" prompt was not offered despite several attempts.

Extensive radio communications with atc in the busy terminal area was very distracting and made for difficult division of duties between flying and configuring the aircraft, reviewing systems and status, etc.

Narrative: 3

I received a phone call from ZZZ operations asking if one of my flights was diverting there. I called ZZZ Tower and was told the flight was on short final with a hydraulics issue and emergency equipment was standing by the runway. The flight landed with no prior contact with Dispatch. Crew debriefed with Dispatch after parking.

Synopsis

An A320 diverted to an airport with a longer runway when they got a Green System Hydraulic low quantity ECAM message on final to the single short runway at their destination. Busy terminal traffic, heavy ATC communications load and low fuel quantity for the diversion trumped their ability to contact Dispatch until at the gate at the diversion airport.

ACN: 867833 (2900 of 2908)

Time / Day

Date : 201001
Local Time Of Day : 1801-2400

Place

Locale Reference.ATC Facility : SCT.TRACON
State Reference : CA
Altitude.MSL.Single Value : 3300

Environment

Flight Conditions : VMC

Aircraft : 1

Reference : X
ATC / Advisory.Tower : SNA
ATC / Advisory.TRACON : SCT
Aircraft Operator : Air Carrier
Make Model Name : A320
Crew Size.Number Of Crew : 2
Operating Under FAR Part : Part 121
Flight Plan : IFR
Mission : Passenger
Flight Phase : Final Approach
Route In Use : Visual Approach
Airspace.Class C : SNA

Aircraft : 2

Reference : Y
ATC / Advisory.TRACON : SCT
Make Model Name : Cessna Citation Undifferentiated or Other Model
Flight Phase : Initial Approach
Flight Phase : Descent
Airspace.Class E : SCT

Person

Reference : 1
Reporter Organization : Government
Function.Air Traffic Control : Approach
Qualification.Air Traffic Control : Fully Certified
ASRS Report Number.Accession Number : 867833
Human Factors : Situational Awareness

Events

Anomaly.Aircraft Equipment Problem : Critical
Anomaly.Airspace Violation : All Types
Anomaly.Deviation / Discrepancy - Procedural : Published Material / Policy
Detector.Person : Air Traffic Control
Result.Air Traffic Control : Issued New Clearance

Assessments

Contributing Factors / Situations : Aircraft
Contributing Factors / Situations : Airspace Structure
Contributing Factors / Situations : Human Factors
Primary Problem : Human Factors

Narrative: 1

I was working the Tustin Radar position unassisted. I was conducting visual approaches to SNA Runway 19R with numerous aircraft. In the area just North of the airport, I own 3000 and below and just to the North of that I own 5000 and below. My aircraft, on a visual approach, descend from 5000 and must be at 3000 or below before that airspace boundary line. The Shore/Maverick sector typically has airliner traffic passing Southwest bound just north of the SNA airport descending to 4000 landing at LGB. An A320 had been cleared for a routine visual approach and communication switched to the tower. I had previously approved a point-out on a slow-moving Cessna at 3000 from an adjacent sector (Shore/Maverick) to transition through my airspace East to West across my final just north of the airport (SNA). Simultaneously, I received a hand off from the Shore/Maverick sector on a VFR Citation from the Northwest also landing SNA that was very high and fast. At the time of the hand off, the Citation was about 15 miles from the airport and it's altitude was approximately 11,500, descending to 5,500, and going approximately 230 knots ground speed and heading Southeast bound. I had also accepted a couple off handoffs on an IFR SNA airliner arrivals from the East from the same sector descending to 6000. I was concentrating on getting the Citation sequenced with the other traffic when I noticed that the A320 had significantly slowed it's descent and was unusually high, at 3300 about 4 miles from the airport. This placed the aircraft in Shore's/Mavericks's airspace and also in proximity to a LGB arrival that was descending to 4000. This required the shore controller to slow the descent of the LGB jet arrival and turn the Cessna away. SNA tower then called and advised me that the A320 had problems and needed to go around. I assigned a right turn, heading 320, and altitude 2500. The A320 eventually diverted.

Synopsis

SCT controller experienced a loss of separation event when failing to note the slow descent of a SNA arrival that conflicted with traffic in an adjacent sector, aircraft mechanical problems later reported as a causal factor of the slow descent.